3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303919

E-meeting, 17 – 26 April, 2023

Agenda Item:	9.4.1.2

Source:	Moderator (Huawei)

Title:	FL summary#2 for AI 9.4.1.2 SL-U physical channel design framework

Document for:	Discussion and decision

# Introduction

The latest R18 WID on sidelink evolution (RP-230077) includes the following objective regarding support of sidelink on unlicensed spectrum (SL-U):

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

This contribution provides discussions related to SL-U physical channel design framework (AI 9.4.1.2), including summary of contributions, FL’s proposals, discussions, outcome of this meeting, etc. The related email thread is as below:

[112bis-e-R18-SL-02] Email discussion on physical channel design framework for unlicensed spectrum by April 26 – Mixiang (Huawei)

- Check points: April 21, April 26

# Issues

## [Closed] Issue#1: SL bandwidth part and resource pool

### Background

- Some issues are mentioned, e.g., whether to use guardband PRBs for PSFCH/S-SSB transmission, whether to consider Uu TDD configuration, whether to set bitmap to all “1”s, whether to support one SL resource pool includes sub-set of PRBs of one RB set, resource pool/sub-channel configuration details, etc.

- FL assumes such issues are not very urgent at this stage, and some of them can be discussed after the details are clearer (e.g., PSFCH/S-SSB transmission scheme, sub-channel mapping, etc.). Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

### [Closed] 1st round discussions

So far, no proposals are given. If you have any comments, please provide in box below.

| Company | Comments |
| --- | --- |

FL’s comment: no comments in 1st round, no more discussions needed for this meeting.

## Issue#2: Slot structure

### [Closed] 1st round: Proposals and background

#### [H] Proposal 2-1

Background: TBS determination

- Summary

  - Ok with previous WA, no need to introduce a different number (10): OPPO, Docomo, vivo, ZTE, Apple, [Intel], Sharp, [Panasonic], Transsion Holdings , ITL

  - Other designs (13): CATT, Spreadtrum, CMCC, Huawei/HiSilicon, Qualcomm, Samsung, Nokia, MediaTek, Xiaomi, Lenovo, Docomo, LGE, [ZTE]

    - Different solutions are given. A similar idea is that if all the initial and retransmissions of a TB are within a COT (e.g., MCSt case), TBS should be determined based on 1st starting symbol to improve spectrum efficiency.

- FL’s view

  - A proposal is given to reflect the above.

[H] Proposal 2-1

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- If all the initial and retransmissions of a TB are within a COT, TBS is determined based on a reference number of symbols as follows

  - Option 2: The reference symbol length is determined based on 1st starting symbol

- FFS details, e.g., how Rx UE can differentiate the above two cases, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | Even if TX UE assumes all (re)transmissions of a TB are performed within a COT, it may be the case actually. For example, due to LBT failure, due to NACK feedback, etc. Therefore, Option 2 with condition is not reasonable. Option 4 should be used for all situations. Besides, as we commented in our contribution, usage of ‘PSFCH overhead indication’ field in SCI-1 when Option 4 is applied should be discussed. |
| vivo | No | We can understand the motivation to improve the efficiency and would be fine if it can be achieved with reasonable effort and complexity. But the current proposal is difficult to us. Firstly, the current proposal is not clear to us. According to the 1st sub-bullet, the TBS of the initial transmission should be based on a ref number of symbols, but the 2nd sub-bullet seems conflicts with the 1st bullet, saying that the TBS of the initial transmission is based on 1st starting symbol. Secondly, the proposal strives to agree first the 2nd sub-bullet first, and then try to find out a solution. However, this actually is our concern – without the specific solution, we can hardly estimate the impact, efforts, etc. In our view the candidate solutions on the table requires the UE to prepare two hypotheses of TBs (1st starting symbol or ref symbols), as well as complicated MAC layer work to manage the data buffer (e.g., based on which hypothesis take places). Thirdly, it may inevitably increase the SCI overhead to indicate the option for TBS determination, as it seems no solid solution of implicit indication can avoid misunderstanding between Tx and Rx UEs. Moreover, this again implies that two hypotheses of SCIs should be prepared and used according to the channel access result. |
| LGE | No | Considering that the RX UE can miss the SCI, the proposal will cause misalignment on the TB size between TX UE and RX UE. To be specific, when the RX UE misses some SCI, the understanding on the initial transmission is different between TX UE and RX UE. Regarding inside/outside COT, when the RX UE may not know whether the TX UE uses COT or not. We think that Option 4 without consideration of COT needs to be supported. For simplicity, no specification change is needed. |
| Nokia/Nsb | No | The dynamic Tx UE indication could flexibly solve the issues no matter if the transmission is within the COT or not. |
| OPPO | No | For COT sharing case, there is not necessary to introduce a separate reference number of symbols for TBS determination. When COT sharing UE has LBT failure at the 1st starting symbol, it is possible that this UE tries LBT again at the 2nd starting symbol. Therefore, one (pre-)configured reference number for TBS determination is enough. If SL feedback is enabled, it is hardly for the TX UE to know whether all the initial tx and re-tx are within the COT since whether there is re-tx depends on HARQ feedback. Furthermore, if two separate reference numbers of symbols are supported, “How Rx UE can differentiate the above two cases” is a relative issue that needs to be resolved. So for COT initiation and COT sharing case, one reference number of symbols by (pre-)configuration is enough. |
| Apple | No | The solution of this proposal (at least the second bullet) seems incomplete. For example, do we consider the case where initial transmission is in a COT (but not initiating a COT), while retransmission is out of a COT? Also, the Rx UE does not know whether both initial transmission and retransmission are in a COT. Overall, we prefer Option 4 is applied to both COT initiation and COT sharing. |
| NEC | No | We share above companies' view. Beside, in the case initiating UE access to the channel from the 2nd starting symbol in slot and retransmission starting from the 1st starting symbol within a same COT, we don't think TBS determination based on 1st starting symbol is beneficial. |
| Ericsson |  | Does the second bullet refer to all transmissions except the one initiating the COT? For all those, full slot can be used. |
| Qualcomm |  | We support the TBS to be determined by a fixed reference symbol length (i.e., 1st starting symbol to maximize the spectral efficiency) for transmission starting at 1st/2nd starting symbol, inside/outside a COT and initial Tx or reTx. Suggest removing the wording before the option 2 to address companies’ concerns. For option 4, we are worried about the spec impact and the progress if the preconfigured symbol number is different from the 1st starting symbol unless we want to spare a bit in SCI to indicate the TBS calculation is based on preconfigured reference symbol length or actual number of PSSCH symbols for Tx. More specifically, to keep the same TBS for initial transmission (e.g. in the COT beginning slot with two starting symbols and preconfigured reference number of symbols being half slot ) and retransmission (e.g. in the slot in the middle of a COT or with only one starting symbol, i.e., PSFCH slot), the retransmission scheduling SCI may need to indicate the preconfigured reference number of symbol being used for TBS determination since the Rx UE may not detect the 1st transmission and may think that the actual number of received PSSCH symbols is to be used for TBS calculation. |
| Samsung | No | We cannot distinguish two cases with different UE behavior without knowing how the UE can distinguish the two cases. We are ok with Option 4 only. |
| Sharp | No | We do not think a different reference symbol length is needed for transmissions within a COT. Although second bullet imposes the limitation on all initial and retransmissions of a TB to be within a COT, there still be a case that a UE may initiate another COT to retransmit the TB upon detection of NACK feedback. Moreover, there is also the probability of the fact that a UE may have already generated a TB before it acquires a shared COT information. We prefer a unified solution for TBS determination and not complicate it. |
| Intel | No | We also share same view with other companies, and we believe that pre-configuration should apply to all cases and there is no need to fragment the design and complicate unnecessarily the design to aim for small optimizations. |
| Lenovo | No | It is not clear how Rx UE can know “all the initial and retransmissions of a TB are within a COT” when the Rx UE decodes the initial transmission of the TB. For simplicity, there is no need to differentiate the two cases for TBS determination. A unified rule is necessary, e.g. either based on pre-configuration or 1st starting symbol. |
| Spreadtrum | No | We are fine with option 4 for all cases to reduce the complexity of UE implementation. |
| Sony | No | We prefer a unified procedure for the TBS determination. We support Option 4 for both cases. |
| CMCC | Yes | For how Rx UE can differentiate the above two cases, we think that at least for COT sharing case, since destination of the transmission from responding UE must include the initiating UE, and the COT is shared by initiating UE, there will be no ambiguity in that case. |
| xiaomi | No | For the first bullet, the reference number of symbols should be determined based on 1st starting symbol for transmission in a shared COT shall also be supported. Because when a UE shares a COT, the UE has the larger possibility to access the channel from the first starting symbol in the actual LBT procedure, so we make the following revision: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - The reference number of symbols is determined based on 1st starting symbol for transmission in a shared COT - If all the initial and retransmissions of a TB are within a COT, TBS is determined based on a reference number of symbols as follows - Option 2: The reference symbol length is determined based on 1st starting symbol - FFS details, e.g., how Rx UE can differentiate the above two cases, etc. |
| CATT/GOHIGH | Comment | To improve the spectrum efficiency, the 1st starting symbol can be used for TBS determination for some predictable cases: - All the transmissions of one TB are within a COT, such as blind transmissions - All the slots related to the transmissions of one TB are configured with PSFCH resources, where only one starting symbols is supported, there is no need to use the (pre-)configured parameter for TBS determination. Regarding how to distinguish option 2 and option 4 from Rx UE side, one bit can be included in SCI to indicate which option is finally used, or other ways which can be further discussed with the FFS part. |
| Panasonic | No | We think Option 4 for both initiating COT and within COT needs to be supported. |
| Transsion | Yes | The time domain information of the sidelink transmission is not indicated in the SCI, in that case, it is appropriate to determine the reference number of symbols according to the (pre)configuration. |
| MediaTek | NO | A unified reference symbol length for TBS determination for all cases is preferred from the perspective of spec effort and complexity. From this point of view, the 1st starting symbol can be used as a fixed symbol length for TBS determination as some other companies also proposed. |
| ZTE | No | Same as the above companies' view, the current proposal is incomplete and means a dynamic indication in SCI-1 is needed to differentiate different cases including e.g. an initial transmission is within a COT and a retransmission is outside a COT. Meanwhile, the assumption that only the first starting symbol is used for transmissions within a COT is not always true. Considering UE cannot predict whether the 1st or 2nd starting symbol will be used for its further transmissions, we think only one (pre)configured reference number of symbols is needed regardless initiating COT or within a COT. As pointed in our contribution, for PSFCH slot, as the second starting symbol cannot be used in these slot, TBS determination should also take this into account, and the updated proposal is shown below: [H] Proposal 2-1 If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined with based on a reference number of symbols as follows: - Min(Lref - 2, sl-LengthSymbols - 2 - ) - The reference number of symbols Lref is determined by (pre-)configuration - sl-LengthSymbols and is determined as legacy |
| Huawei/HiSilicon | Yes | We support this proposal to improve efficiency since most transmissions on remaining slots within a COT can start at 1st starting symbol taking the advantage of COT sharing. Since TX UE can know when it starts its initial transmission and whether its (re-)transmissions are within a COT or not, the TX UE can use one bit in SCI to indicate whether to use the reference number or legacy R16 SL way for TBS determination. |
| WILUS | No | We think that there is no need to differentiate the two cases for TBS determination. It is enough to support Option 4 only. |

#### [M] Proposal 2-2

Background: AGC symbols

- Summary

  - Regarding Tx UE behaviour:

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

      - Support (9): CATT, Huawei/HiSilicon, OPPO, CATT, CMCC, Spreadtrum, NEC,  Panasonic , Sony

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

      - Support (8): Qualcomm, Ericsson, Intel, ZTE, Futurewei, [Docomo], MediaTek , Transsion Holdings

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

      - Support (8): Samsung, Apple, vivo, Lenovo, InterDigital, ITL, Nokia, Fraunhofer

  - Regarding Rx UE behavior:

    - Option A: The Rx UE always monitors two AGC symbols in such slot

      - Support (3): CATT, Huawei/HiSilicon, NEC

    - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

      - Support (4): Qualcomm, Lenovo, InterDigital, [Samsung]

    - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

      - Support (2): OPPO, CMCC

    - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

      - Support (10): [Ericsson], Intel, ZTE, Nokia, MediaTek, Futurewei, [Docomo], Spreadtrum, Sony, Transsion Holdings

- FL’s view

  - Regarding Tx UE behaviour:

    - RAN1#110b-e WA has a bullet saying “The candidate starting symbol(s) are intended for AGC purpose”, thus Option 1 is more aligned with this.

    - Option 2 may increase resource usage, but may cause decoding failure when AGC is inaccurate.

    - In Option 3, some companies give optimized designs, e.g., the conditions are based on Tx UE’s explicit indication, or based on the transmission bandwidth, etc. But it may cause decoding failure when AGC is inaccurate. Considering the limited TU, FL suggests not spending too much time on such optimization.

    - FL suggests to quickly close this issue by supporting both Option 1 and 2.

  - Regarding Rx UE behavior:

    - Some companies give solutions that Rx UE could drop monitoring 2nd AGC symbol based on conditions, e.g., Tx UE’s explicit indication, detection of PSCCH or RSRP from 1st starting symbol, etc.

    - Similar as above, considering the limited TU, FL suggests not spending too much time on such optimization and suggest to adopt the simple and effective way, i.e., Option D.

  - The following proposal is given to reflect above.

[M] Proposal 2-2

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol,

    - The PSCCH/PSSCH transmission has 1 or 2 symbols for AGC purpose subject to (pre-)configuration

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | From TX UE perspective, there is no need to have two signal mapping mechanisms. Besides, meaning of ‘for AGC purpose’ ‘monitor 1 or 2 AGC symbol(s)’ is quite unclear for us. These terminologies are not spec-wise. Although the options were agreed at the last meeting, wording should be changed. |
| vivo | No | Regarding the Tx behavior for AGC symbol, we have evaluated the performance of option 3 in our contribution (R1-2302487). It is observed that 9.9% UPT performance gain can be achieved by option 3 compared with option 1, which is a significant performance improvement. Given that this is only the Tx behaviour, the spec impact is low (i.e., basically the conditions to select 1 or 2 symbols for AGC). The simplest (and probably useful) condition of using one AGC symbol can be that when the UE allocates all the sub-channels for a PSSCH transmission (or more flexibly, when the number of allocated sub-channels is larger than a threshold). |
| LGE | No for TX UE behavior | AGC itself is RX UE’s behavior. In our understanding, if necessary, the TX UE can select proper DMRS pattern (especially when the number of CB is large) so that the one of DMRS symbol is overlapping with the 2nd staring symbol. Then the PSSCH DMRS can be used as AGC training symbol. |
| Nokia/Nsb | Agree |  |
| OPPO | No | We still think option 1 and option C should be supported. From Tx UE’s perspective, different UEs may start in 1st or 2nd starting symbol whenever 2 candidate starting symbols are configured, and there should be 2 symbols for AGC purpose. From Rx UE’s view, monitoring 2 candidate starting symbols can help to avoid missing any transmission, and it is also a simple way to left UE’s implementation whether to detect the 2nd candidate if the 1st one is already detected. |
| Interdigital | No | We support Option 3. In our view, Option 1 has spectrum efficiency problem and Option 2 may cause TB decoding problem due to inaccurate AGC. Option 3 can solve the two problems of Option 1 and 2. |
| Apple | See comments | We support Option 3, and think the explicit (pre)configuration is unnecessary, or can be replaced by “implicit” (pre)configuration. Option 1 has spectrum efficiency problem as an additional dedicated symbol is used for AGC purpose which may not always needed. Option 2 may have AGC issue, not only restricted in hidden node case. For Option 3, our proposal does not rely on Tx UE’s explicit indication or transmission bandwidth. Actually, it simply depends on how many RB sets are in the resource pool. If a resource pool is composed of 1 RB set, then the AGC issue does not occur except hidden node issue. In this case, a single AGC symbol is enough. If a resource pool is composed of more than 1 RB set, then the AGC issue may occur since a UE using RB set #2 (starting from the second candidate symbol) may lead to AGC issue for a UE receiving PSSCH on RB set #1 (starting from the first candidate symbol). It is easy to determine Rx UE behavior after Tx UE behavior is determined. |
| NEC |  | As compromise, we also support combination of Option 3+option B to allow only one AGC symbol under conditions mentioned by proponent. |
| Ericsson | No for TX behavior; Yes for RX behavior | We do not see why the TX would need to have 2 AGC symbols. LBT ensures that this is not necessary. |
| Fraunhofer | No | We still support option 3 and see benefits, as also pointed out by other companies. |
| Qualcomm | No on Tx Ok on Rx | We don’t see the need of introducing 2nd AGC symbol for transmission staring from 1st starting symbol. Firstly, it increases overhead and the Rx UE implementation most likely cannot make use of it since retuning AGC hurts the phase continuity of channel estimation. Secondly, additional AGC symbols also require redesigning DMRS patterns to avoid getting punctured/replaced by AGC symbol (which happens at least for some combinations of second starting symbol location and DMRS pattern, e.g., #7 and 4-symbols DMRS). If companies are really worried about the AGC issue for the full slot receiver, the Tx UE could (by implementation) choose lower MCS and the Rx UE could (by implementation) use the PSSCH SCH REs in 2nd starting symbol for AGC retuning and puncture that symbol in PSSCH decoding. |
| Samsung | No | We don’t think a (pre-)configuration of 1 or 2 symbols for AGC can work. It should be by SCI to be compatible with the dynamic behavior of channel access. |
| Intel | No on Tx Ok on Rx | We do not thing that 2nd AGC symbol is needed and we share in toto the comments from Qualcomm, as a second AGC would incredibly decrease spectral efficiency and require a more complex implementation while the issue highlighted is already deemed by the LBT procedure and it rarely occurs. Also as highlighted by Qualcomm, if companies believes that there is an AGC issue, this could still be handled via implementation. |
| Futurewei | OK on Rx, No on Tx | We do not think that the second AGC is necessary. |
| Lenovo |  | It seems contradicted between the two bullets: if 1 or 2 AGC symbols are based on pre-configuration, why is it up to Rx UE implementation? |
| ETRI | No | We support option 3 because options 1 and 2 have some drawbacks as pointed out by other companies. |
| Spreadtrum | No | We don’t prefer it subject to (pre-)configuration. As a compromise, we are fine with option 3. |
| CMCC | No | We still prefer Option 1 since if Tx UE1 and Tx UE2 are hidden nodes, there will be AGC issue on the Rx UE in-between the two Tx UEs if there is only 1 AGC symbol. |
| xiaomi | No | Regarding Tx UE behavior, we prefer option 2 that the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose, because PSCCH/PSSCH transmission starts from the 1st starting symbol, the second preconfigured AGC symbol is used to PSSCH transmission, otherwise, the PSSCH transmission will be interrupted. |
| CATT/GOHIGH |  | Regarding Tx UE behavior, we think the configuration of two AGC symbols is really needed, whether from the perspective of hidden-node or FDM-based transmissions between different RB sets. Regarding Rx UE behavior, our preference is option A, which is consistent with the motivation of the configuration of two AGC symbols, but we can flexibly compromise to option D to reduce the specification effort. |
| Panasonic | No | For PSSCH with 1st starting symbol, 2nd AGC symbol is not required to be a copy of next symbol, it could be no difference from the PSSCH transmission without 2nd starting symbol. Whether Rx UE uses this symbol for AGC or reception of PSSCH is implementation matter. For Tx UE, for the MCS determination, the possibility that this symbol may not be received should be taken into account as UE implementation. |
| MediaTek | Comment | Regarding Tx UE behaviour: No We understand the main motivation to support a 2nd symbol for AGC purpose within a slot is for the case that a transmission on the adjacent one or multiple LBT channel(s) may starting from the 2nd starting symbol, which may introduce additional interference for the transmission from the 1st starting symbol within a slot. But as analyzed in our contribution, we don’t think the second AGC is necessary in this case with the following observations: - The spectrum efficiency will be degraded if a 2nd AGC symbol is introduced. - The power impact from the adjacent channel(s) is limit (e.g., max to 6dB for a max of 80MHz BW assumption), thus there is no need to adjust the AGC on the 2nd starting symbol. - Considering WiFi is an asynchronization system, the AGC issue may always exist on unlicensed spectrum. - Phase jump issue maybe occurred if AGC is readjusted on the 2nd starting symbol. Regarding Rx UE behaviour: Yes |
| ZTE | Not Ok for the Tx | 2 symbols for AGC purpose for Tx are not essential, and Rx UE can still monitor 2 AGC symbol(s) in such slot when 1 symbol is set for AGC purpose for Tx |
| Huawei/HiSilicon | Yes | Due to different capability on PSCCH decoding(Tproc,0 is needed for PSCCH decoding), a RX UE may not know whether a TX UE starts from the 1st starting symbol or 2nd starting symbol quickly, therefore 2 starting symbols for AGC purpose is necessary. As for spectrum efficiency mentioned by above companies, such issue can be solved by proposal 2-3 below, i.e., only the first slot of a COT has 2 candidate starting symbols, remaining slots of the COT only has one staring symbol. As a compromise on TX behavior, we support to minimize specification workload via (pre-)configuration. |
| WILUS | No on Tx Ok on Rx | We don’t see the necessity to introduce 2nd AGC symbol for transmission staring from 1st starting symbol. |

#### [M] Proposal 2-3

Background: Tx UE behavior on using 1st or 2nd starting symbol

- Summary

  - The following companies mentioned Tx UE behavior on using 1st or 2nd starting symbol should be clarified. Some companies propose that within a COT, Type 2 LBT failure probability is very low, so that Tx UE should only choose the 1st starting symbol for PSCCH/PSSCH transmission, which can reduce Rx UE blind decoding.

    - Qualcomm, Huawei/HiSilicon, Lenovo, InterDigital, Docomo, etc.

- FL’s view

  - A proposal is given to reflect the above.

[M] Proposal 2-3

Regarding Tx UE behavior:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

- For the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - Note: Rx UE only needs to decode PSCCH/PSSCH once in such slots within the COT.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM |  | For the first bullet, does it mean that TX UE shall try to perform the 1st starting symbol before trying the 2nd starting symbol? We believe that this is a key point and thus suggest the following update. - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: TX UE can use the 2nd starting symbol for the TX only if the TX from the 1st starting symbol at the same slot is failed due to LBT failure. |
| vivo | comment | For the 2nd bullet, it is not clear whether it is applicable to the COT initiator, or also the responding UE sharing the COT. The former case is fine. Regarding the later case, whether the responding UE can start transmitting from the 1st starting symbol may depend on the decoding time of COT sharing information (in SCI or MAC CE?). Moreover, the ‘note’ seems confusing as even if there is only one starting symbol, a UE may decode more than one PSCCH/PSSCH. |
| LGE | No | In case of the shared COT, the TX UE can start transmitting in the middle of the shared COT. When the TX UE uses Type 2A or 2B channel access procedure, it will happen the TX UE fail to access the channel. In this case, to minimize the case when the shared COT is interrupted by other transmission, it would be better to use the 2nd starting symbol as well. |
| Nokia/Nsb | Agree |  |
| OPPO | No | Firstly, this is not essential issue and should be down-prioritized. Secondly, the motivation to introduce multiple starting symbols is to increase the channel access opportunities for SL-U UEs. It is also possible for UEs to perform type 2A failure within a COT, in that case, starting from 2nd starting symbol is also benefit for SL-U to access the channel. |
| Interdigital | Yes | We support the proposal. The UE should always strive to use slot-based transmission after the first slot of a COT. |
| Apple | No | We do not think the restriction of the second bullet is necessary. Even in COT, type 2A or 2B channel access procedure may lead to LBT failure. In this case, starting from the second candidate symbol is allowed. |
| NEC | No | Agree with OPPO and Apple. |
| Ericsson | Yes, but remove the note | What is the intention with the note? It is unclear. |
| Qualcomm | Yes | We support the proposal |
| Samsung | No | The proposal didn’t include the behavior in a shared COT (we think LG’s comment makes sense). For the behavior within an initiated COT, we agree with DCM’s comment for the first slot. For the remaining slots in the COT, we don’t see the reason to prevent using the second starting location (e.g., the UE’s transmission is not contiguous slots, or the UE fails LBT at the first starting location in the slot) |
| Sharp | No | For the second bullet, a UE is required to perform Type 2 SL channel access procedure in a COT if there is gap symbol. It is not guaranteed that channel would be always idle when performing Type 2A/2B SL channel access procedure. |
| Intel | Yes | OK with the proposal. |
| Futurewei | OK |  |
| Lenovo | comment | For the 1st bullet, we share the same view as DCM. For the 2nd bullet, the 1st starting symbol should be symbol 0 to avoid the risk of losing the channel. This should be clear in the 2nd bullet. |
| ETRI | No | As pointed by some companies, performance can be improved by using the 2nd starting symbol for LBT failure cases that may occur when using Type 2A or 2B channel access procedures within a COT. |
| Spreadtrum | No | For the first bullet, there is no need to specify TX UE’s behavior. For the second bullet, as mentioned by other companies, TX UE may fail to access the channel at the 1st starting symbol. The TX UE still has another transmission chance at 2nd starting symbol. Moreover, although the transmission is from 1st starting symbol, RX UE also need to monitor the 2nd starting symbol for other transmissions. It cannot reduce the RX UE complexity. And RX UE may not distinguish whether the transmission is at the 1st slot of a COT or the remaining slots of a COT. |
| Sony | OK |  |
| CMCC | No | If UE fails to access the channel in both of the 1st and 2nd starting symbol of the first slot, it can try to access the channel in both of the 1st and 2nd starting symbol of the second slot, so we agree with the modification from DCM. |
| xiaomi | Yes |  |
| CATT/GOHIGH | Yes |  |
| Panasonic | Yes | We support the proposal. |
| MediaTek | Yes | An appropriate solution to reduce the complexity of PSCCH blind detection can be considered. |
| ZTE | No | Agree with OPPO and Apple. |
| Huawei/HiSilicon | Yes | We support the proposal. The probability that type 2A or 2B LBT fails is relatively low, and the benefit of always enabling 2 candidate starting symbols within a slot is also limited but with large cost on resource utilization. |
| WILUS | Yes | We support the proposal |

#### Others

- On usage of gap symbol in case of MCSt:

  - Some companies discuss this issue, e.g., Huawei/HiSilicon, Qualcomm, Ericsson, Lenovo, Xiaomi, ITL, etc.

  - After coordination between FLs, this issue is intended to be treated in Channel Access AI. Companies can comment there.

- Some other issues are mentioned:

  - Some companies (e.g., Qualcomm, ZTE, Samsung, vivo, etc.) mentioned some PSCCH blind decoding reduction designs, e.g., limit PSCCH search locations, having only 1 common PSCCH occasion in such slots, introducing preamble, etc.

  - Some companies (e.g., Apple, Docomo, etc.) mentioned the PSFCH location may be 1 slot further compared with legacy design due to processing time.

  - Some companies (e.g., OPPO, Sharp, etc.) mentioned some signaling for 1st and 2nd starting symbol should be different, e.g., DMRS pattern, etc.

  - Some companies (e.g., OPPO, Sharp, etc.) mentioned the OFDM symbols used for SL RSSI measurement should start from the next symbol of the 2nd candidate starting symbol.

  - Some companies (e.g., Qualcomm, etc.) mentioned using 2nd starting symbol is subject to UE capability.

- FL assumes such issues are not very urgent at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |
| Qualcomm | In RAN1#112 agreement, it is agreed that the 2nd starting symbol needs to be later than the 1st starting symbol. However, the PSCCH/PSSCH waveform starting from the 2nd starting symbol may be different from the 1st starting symbol, e.g., the DMRS scrambling sequence depends on symbol index. For some per symbol Tx UE waveform generation implementation, a few symbols after the 1st starting symbol may be needed to generate the waveform for the 2nd starting symbol if Tx UE fails the LBT at the 1st starting symbol. Hence, it may be worthwhile to study how many symbols are needed for Tx UE to prepare the Tx waveform for the 2nd starting symbol when the LBT fails at the 1st starting symbol. Proposal: Companies to study the gap needed between 1st and 2nd starting symbol for UE to prepare the waveform at the 2nd starting symbol if the LBT fails at the 1st starting symbol |

### [Closed] 2nd round: Proposals and summary of previous round

#### [H] Proposal 2-1

Background: TBS determination

- Summary of previous round

  - Companies mentioned the previous proposal is unclear.

- Updates on proposal (marked in red) and justifications

  - Proposal is updated to be clearer.

    - E.g., in Alt 2 below, if all the initial and retransmissions of a TB are within a COT, Tx UE can indicate to use Option 2 via SCI. Rx UE can know which Option to apply via SCI and there is no blind detection.

  - Add an FFS based on comment from Docomo and ZTE. FL also copied related spec below for reference.

(copied from TS 38.214) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList.

[H] Proposal 2-1b

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Down-select one of the followings

  - Alt 1: Support Option 4 only

  - Alt 2: Support Option 4 and Option 2, and Tx UE uses 1 bit in SCI to indicate which of them is currently used

- Note: the options are as below

  - Option 2: The reference number of symbols is determined based on 1st starting symbol

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

FL’s comment: please also indicate which Alt you support.

| Company | Agree? Alt 1 or 2? | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Alt 1 | A single reference symbol length for TBS determination should be configured to cover all possible cases, as UE cannot predict whether the 1st or 2nd starting symbol will be used for its further transmissions, e.g., up to 32 resources may be selected for a TB transmissions at MAC layer, it is difficult to assume all its (re-)transmissions are within a COT and the 1st starting symbol is always used for a TB transmission. |
| Nokia/NSB2 | Alt 2 |  |
| LGE2 | Not support | Why don’t we just confirm the WA. Why do we discuss the other options ruled out in the WA? The WA is breaking the system? I do not think so. The remaining issue should be how the reference number of symbols is (pre)configured. Which is the existing parameter or will we introduce new parameter. That’s all. |
| CMCC |  | We think only when the (pre-)configuration value is NOT the first starting symbol, Tx UE needs to indicate to use Option 2 via SCI. Otherwise, the 1bit in SCI is useless. - Alt 2: Support Option 4 and Option 2, and Tx UE uses 1 bit in SCI to indicate which of them is currently used only when the (pre-)configuration value is NOT the first starting symbol |
| DCM | Alt 1 | Same view with LGE. Unless critical issue is found, we should follow the working assumption. Otherwise, what is the motivation to agree the working assumption at the last meeting? We do not understand why Option 2 is back now. Option 4 can work, then just specifying Option 4 is sufficient. No further discussion is necessary. |
| vivo | Alt 1 | As commented before, Alt 2 requires the UE, based on the assumption of either 1st starting symbol or ref symbols, to prepare two hypotheses of PSCCH and PSSCH transmissions, i.e., one version of SCI (e.g., indicating option 2) and TB (with TBS based on option 2), and another version of SCI (e.g., indicating option 4) and TB (with TBS based on option 4), for every single slot transmission because the UE cannot predict the LBT result. It inevitably increases the complexity, cost and power consumption of the UE. Moreover, it complicates the MAC layer work to manage the data buffer. Traditionally MAC may generate multiple TBs for multiple HARQ processes parallelly, as the TB size of each HARQ process is fixed when triggering the resource selection procedure. However, if the TB size may be vary depending on the LBT result (which is after the resource selection procedure), it is very difficult, if not impossible, for MAC to generate multiple parallel HARQ processes. |
| xiaomi | Alt 2 | Alt 2 has benefit on the flexibility. |
| MediaTek | Comment | A unified reference number of symbols should be applied for all scenarios of outside and inside a COT. From our perspective, only outside a COT, it is worthy discussing the transmission is started from the 1st starting symbol or 2nd starting symbol. For inside a COT, the majority transmissions should be started from the 1st starting symbol. Therefore, the 1st symbol should be (pre-)configured or fixed to determine the reference number of symbols regarding spectrum efficiency. |
| NEC | Alt.1 | For the L_ref we don't see any confusion, which should = sl-LengthSymbols (i.e., the number of symbols used for PSCCH/PSSCH transmission includes the AGC and GP symbol) based on following agreement, but we'are OK to have this FFS if other companies want. Agreement @RAN1#112 For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 |
| Lenovo | Alt. 2 | Alt.2 is beneficial in providing flexibility for different coding rates. |
| Intel | Alt.1 | We share same view as LG, and agree that to begin with we should confirm prior WA. |
| Apple | Alt 1 | We think the working assumption can be confirmed first, i.e., Option 4 is applied at least for COT initiation. Then we can discuss the COT sharing case. In our view, we do not need to have a separate design for COT initiation and COT sharing, based on the arguments in our first round comments. |
| Transsion | Alt 2 | When transmission is within the COT, using symbol 0 as the starting symbol and using the entire slot for transmission avoids channel loss and improves spectral efficiency. In that case the reference number of symbols should be 14. However, if only a single reference number of symobls is introduced, then the values of the 2nd starting symbol will be limited considering that the 1st starting symbol takes the value 0. Otherwise, if the 2nd starting symbol takes a larger value (e.g. #7), the number of symbols used for SL transmission will be much smaller than the reference number of symbols, which will lead to a high code rate and the receiving UE will not be able to decode the SL transmission correctly. As analyzed above, two reference number of symbols are needed, one for the case outside the COT and the other for the case inside the COT. |
| OPPO | Alt 1 | We support Option 4. No matter there is COT initiation or not, in a resource pool, only one reference number (pre-)configured is enough to determine the TBS. |
| Panasonic | Alt 1 | We also think working assumption should be confirmed if there is no significant issue. |
| Sharp | Alt.1 | We agree with other companies about confirmation of working assumption. |
| Huawei/HiSilicon | Alt2 | Alt 2 is beneficial to provide flexibility and improve spectrum efficiency compared with Alt 1. Assume the 1st and 2nd starting symbol is at sym#0 and sym#7 respectively, and assume L_ref = 10 and L_ref replaces sl-LengthSymbols. A COT may include up to 20 slots under 30kHz SCS, transmissions during the COT(except the 1st slot) will probably start at 1st starting symbol due to Type 2 LBT. If we still use L_ref =10 for TBS determination for TBs whose initial and retransmissions are within a COT, it causes (14-10) =4 difference over 14 symbols, the spectrum efficiency is largely decreased. In addition, in proposal 2-2, some companies mentioned one more AGC symbol would decrease spectral efficiency. Then, those companies need to explain why the low efficiency in Alt 1 is not a problem. |

#### [M] Proposal 2-2

Background: AGC symbols

- Summary of previous round

  - Regarding Tx UE behaviour:

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

      - Support (4): CATT, Huawei/HiSilicon, OPPO, CMCC

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

      - Support (9): Qualcomm, Ericsson, Intel, ZTE, Futurewei, Docomo, MediaTek, Xiaomi, WILUS

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

      - Support (8): Samsung, Apple, vivo, InterDigital, ETRI, NEC, Fraunhofer, Spreadtrum

  - Regarding Rx UE behavior:

    - Most companies are fine with proposal.

- Updates on proposal (marked in red) and justifications

  - For Option 1: companies showed concern on resource efficiency.

  - For Option 2: companies showed concern on AGC inaccuracy, e.g., hidden node issue, there could be FDMed transmissions in other RB sets or resource pools, etc.

    - It seems if the AGC from 2nd starting symbol is inaccurate, it impacts the decoding of all the subsequent symbols from 2nd starting symbol till the end of the slot, not only 1 symbol.

  - For Option 3: different solutions are given:

    - Based on explicit indication (1 bit in SCI): Samsung

    - Based on transmission bandwidth (e.g., larger than a threshold or not): Vivo

    - Based on the number of RB sets included in a resource pool (e.g., larger than 1 RB set or not): Apple

  - FL suggests not to go with Option 3 due to the limited TU and progress.

  - Considering the situation, FL feels current proposal is still the compromise.

  - Clarified meaning of AGC symbol as per Docomo’s comment. The intention is to reuse legacy NR SL.

[M] Proposal 2-2b

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol,

    - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols for AGC purpose subject to (pre-)configuration

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

| Company | Agree? | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Only yes for the Rx | Not convinced to support two AGC symbols, this new slot structure than legacy may require more normative work, e.g., TBS determination may be adjust to support 2 AGC symbols. And the 2nd may not that useful as anyway it is up to RX UE implementation whether to monitor the 2nd AGC symbol. |
| Nokia/NSB2 | Agree |  |
| LGE2 | Only yes for RX. | Additional duplication is not preferred since it will reduce the peak data rate. |
| CMCC | Only yes for the Rx | Regarding Tx UE behavior, if only one AGC symbol is supported, the Rx UE’s AGC performance will be affected, the reasons are summarized by FL, such as hidden node issue, there could be FDMed transmissions in other RB sets or resource pools, etc. |
| DCM | Only yes for RX | Same view with other companies above. From TX UE perspective, the duplication is only the initial symbol of TX as in legacy SL. This is enough. No need to change signal generation in spec. |
| vivo | No | As commented before, we observed significant performance gain of option 3 (9.9% UPT performance gain compared with option 1), and minor spec impacts (as least compared with the current proposal 2-2b). As a compromise, we suggest the following updates to have a unified solution: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols for AGC purpose subject to a (pre-)configured threshold (pre-)configuration - Regarding Rx UE behaviour: - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot - Note: AGC symbol is duplication of next symbol Specifically, the threshold (X%) can be (pre-)configured, and the Tx UE determines 1 or 2 AGC symbols by checking if the ratio of assigned/reserved PRBs of the TB is larger than the Threshold. If the ratio of the PRBs > X%, 1 AGC symbol is used, otherwise, 2 AGC symbols are used. Noted that by this way, if X is configured as 100%, 2 AGC symbols are always used (as the ratio of a TB is always not larger than 100%), while if X is configured as 0%, 1 AGC symbol is always used (as the ratio is always larger than 0). |
| xiaomi | Yes |  |
| MediaTek | Tx: No Rx: Yes | Only one AGC symbol for Tx UE is supported. Considering anyway WiFI is an async system, so it is not convinced that introducing a 2nd AGC symbol for Tx UE can resolve the issue of FDMed transmissions (e.g., WiFi) in other RB sets or resource pools. Besides, for the potential decoding failure of Option 2 (i.e., one ACG symbol for Tx UE), it can be addressed/relaxed by a proper AGC margin. |
| NEC | OK |  |
| Lenovo | No | We have below comments: (1) Since Tx UE can’t predict whether it can transmit from 1st starting symbol before performing LBT, how can Tx UE generate the PSSCH? Does the UE generate the PSSCH with one or two AGC symbols? If UE generates the PSSCH with 2 AGC symbols based on the pre-configuration while it starts transmission from the 2nd starting symbol, how can the Tx UE transmit the PSSCH? Does it drop the 2nd AGC symbol because it starts from 2nd starting symbol? This UE behavior is quite unclear to us. In addition, the TBS determination is also one problem. (2) As mentioned in 1st round, it seems contradicted between the two bullets: if 1 or 2 AGC symbols are based on pre-configuration for Tx UE, why is it up to Rx UE implementation to monitor 1 or 2 AGC symbols? |
| Intel | No on Tx Ok on Rx | We do not thing that 2nd AGC symbol is needed as a second AGC would incredibly decrease spectral efficiency and require a more complex implementation while the issue highlighted is already deemed by the LBT procedure and it rarely occurs, and could be always handled by implementation. |
| Apple | OK in direction | We agree that whether PSCCH/PSSCH transmission has 1 or 2 AGC symbols can be based on (pre)configuration. But we think explicit (pre)configuration is not necessary, instead an implicit (pre)configuration is enough. If a resource pool is composed of 1 RB set, then autonomously 1 AGC symbol. If a resource pool is composed of more than 1 RB set, then autonomously 2 AGC symbols. |
| Futurewei | OK |  |
| Transsion | Only yes for Rx | For the behavior of Tx UE, the 2 AGC symbols overhead for all sidelink slots is too large. And the Rx UE implementation will most likely not take advantage of it, because the AGC adjustment of the second starting symbol can impair the phase continuity of the channel estimation within the whole sidelink slot. |
| OPPO | No | The AGC symbol is mainly depending on Rx UEs’ behavior. Whenever 2 candidate starting symbols are configured for PSCCH/PSSCH Tx, different Tx UEs may start from different starting point. As long as the Rx UE perform reception from either 1st or 2nd starting symbol, the 2nd starting symbol should always taken as an AGC symbol. So for Tx side, there should have 2 AGC symbol if the transmission is from the 1st starting symbol (Option 1). From Rx UE side, if it is totally depending on UE’s implementation to monitor 1 or 2 AGC symbol, missing detection will happen, which degrades the detection performance. We think Option C should be considered to make sure about the detection. |
| Panasonic | Only yes for RX | Additional duplication symbol for 2nd AGC is not necessary. For the MCS determination, the possibility that this symbol may not be received could be taken into account as TX UE implementation. |
| Huawei/HiSilicon | Yes | 2 AGC symbol is necessary to ensure successful decoding. As a compromise, we are ok to support both via (pre-)configuration to minimize specification workload. |

#### [M] Proposal 2-3

Background: Tx UE behavior on using 1st or 2nd starting symbol

- Summary of previous round

  - Support (13): Nokia, InterDigital, Ericsson, Qualcomm, Intel, Futurewei, Sony, Xiaomi, CATT, Panasonic, MediaTek, Huawei/HiSilicon, WILUS

  - Not Support (10): LGE, OPPO, Apple, NEC, Samsung, Sharp, ETRI, Spreadtrum, CMCC, ZTE,

- Updates on proposal (marked in red) and justifications

  - Add the following as per Docomo’s comment.

    - “Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol”

  - Made the following change as per comment from vivo.

    - “Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information”

    - “Note: Rx UE only needs to decode PSCCH/PSSCH once transmitted from 1st starting symbol in such slots within the COT.”

  - Some companies do not see the benefits of 2nd bullet. Proponents of 2nd bullet can further clarify and respond to this. From FL’s perspective, at least one benefit is reducing Rx UE blind decoding within a COT.

- FL’s response to some companies’ comments:

  - @Ericsson: the intention of the Note is just for clarification. If companies are aligned on the understanding, we can also remove the Note and only focus on main bullets.

  - @Leonovo: based on previous agreement, the starting symbol is (pre-)configured, maybe not #0.

[M] Proposal 2-3b

Regarding Tx UE behavior:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- For the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information

  - Note: Rx UE only needs to decode PSCCH/PSSCH once transmitted from 1st starting symbol in such slots within the COT.

| Company | Agree? | Comments |
| --- | --- | --- |
| ZTE, Sanechips | No for the second bullet | The second starting symbol can also be used within a COT, it increases UEs’ channel access opportunities and is beneficial for maintaining a COT. |
| Nokia/NSB2 | With comments based on observations from last round discussions | Regarding the 2nd bullet point, if the transmissions within the COT is with MCSt, then the current proposal makes sense, and Type-2A/2B could be avoided, e.g by CPE or other means, in order to make sure the contiguous transmissions within the COT. And starting from the very 1st symbol makes a lot of sense. Otherwise, as commented by companies in the last round, the Type-2A/2B can be also failed within the COT, which may also require the starting from the 2nd. So as compromise, to make both cases working, the Tx UE choose the 1st starting symbol for PSCCH/PSSCH MCSt transmission within the COT, otherwise the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. |
| LGE2 | No | As mentioned before, even for the remaining slots of a COT, the UE can use the 2nd starting symbol depending on the LBT results. |
| CMCC |  | Maybe it needs to clarify the definition of “the 1st slot of a COT” firstly. If a UE fails to access the channel in both of the 1st and 2nd starting symbol of the first slot of the selected multiple consecutive slots, whether the second slot in the selected resource become the 1st slot in a COT and UE will try to access the channel in this slot’s 1st starting symbol then 2nd starting symbol? If so, we can accept this proposal. |
| DCM | OK except for the last sub-bullet | In our understanding, even when there is a COT initiated by UE-A, UE-B may not notice it due to hidden-node issue and thus UE-B can start TX from the 2nd starting symbol with another COT initiation. That is, UE-A’s COT and UE-B’s COT can be partially overlapped. Considering this case, RX should always decode TX from the 2nd starting symbol. UE-A’s COT initiation does not guarantee that all SL-UEs follow it. |
| vivo | OK | In our view, the 2nd bullet is at least reasonable if the TX UE is the COT initiating UE, e.g., to maintain the COT. Is there any use case that the COT initiating UE chooses the 2nd starting symbol for transmission during its COT? If not, is it acceptable to change the Tx UE to COT initiating UE in the 2nd sub-bullet and putting FFS for the responding UE, as below? Regarding Tx UE behavior: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - For the remaining slots of a COT, the Tx COT initiating UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS for the responding UE sharing the COT - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information - Note: Rx UE only needs to decode PSCCH/PSSCH once transmitted from 1st starting symbol in such slots within the COT. |
| xiaomi | Yes | With this proposal, the complexity of PSCCH blind detection can be reduced. |
| MediaTek | Yes | Per CPE operation, within an initiated COT, it is a low risk of Type 2 failure before the 1st starting symbol due to the interference for both COT initiating UE and COT sharing UE. In our understanding, the only case is that for 60kHz, where the length of GP is less that the duration of Type 2A (16.67us vs. 25us). But from our point of view, a proper Type 2 channel access (e.g., Type 2A/2B) can be applied in this case to guarantee the Type 2 can be finished within the GP and the 1st starting symbol can still be used. Therefore, the current proposal should be supported. |
| NEC | OK |  |
| Lenovo | See comments | We have below comments: (1) The note “Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information” is not clear to us. If the Tx UE intends to share the COT, it should start transmission subject to LBT type 2 outcome instead of transmission directly from 1st symbol. (2) after initiating a COT, SL transmission from the 2nd slot of the COT should start from symbol 0 so that the spectrum efficiency can not only be improved but also avoid the risk of losing channel if the RRC configured 1st starting symbol is not symbol 0. Hence, we propose SL transmission from 2nd slot of the COT is always started from symbol 0. For reference, some suggestions are listed below: [M] Proposal 2-3b Regarding Tx UE behavior: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can start transmission from the 2nd starting symbol only if LBT fails at the 1st starting symbol - For the remaining slots of a COT, the Tx UE starts from symbol 0 for PSCCH/PSSCH transmission. - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information - Note: Rx UE only needs to decode PSCCH/PSSCH once transmitted from symbol 0 in such slots within the COT. |
| Intel | OK |  |
| Apple | No (for the second bullet) | The new version does not address our comment in the first round: Even in a COT, type 2A or 2B channel access procedure may lead to LBT failure. In this case, starting from the second candidate symbol should be allowed. |
| Futurewei | OK |  |
| Transsion | OK |  |
| OPPO | No | As we commented in the 1st round, we think any slot in a COT (initiating slot or remaining slots) can starting and use the 2nd starting symbol due to LBT failure at the 1st starting symbol. We would like to suggest de-prioritize this proposal while it seems not quite essential. |
| Panasonic | OK |  |
| Sharp |  | We are not OK with second bullet. As long as a TX UE is required to perform Type 2 SL channel access procedure for transmission in a COT, 1st or 2nd starting symbol selected for the transmission should depend on whether the TX UE clears the LBT. |
| Huawei/HiSilicon | Yes | This proposal is beneficial to largely reduce PSCCH decoding complexity within a COT. Type 2 LBT failure within a COT is very low probability. Because if another UE tries to access channel via type 1 LBT, such Type 1 LBT will probably fail since at least Td=Tf+mp*Tsl is needed for type 1 LBT. |

### [Closed] Round 3: Proposals and summary of previous round

#### [H] Proposal 2-1

Background: TBS determination

- Summary of previous round

  - Support Alt 1(9): ZTE, DCM, vivo, NEC, Intel, Apple, OPPO, Pana, Sharp,

  - Support Alt 2 (5): Nokia, xiaomi, Lenovo, Transsion, HW

- Updates on proposal (marked in red) and justifications

  - Based on the situation, FL suggests to go with Alt 1.

  - Add the following based on comment from NEC. FL assumes this should not be very controversial, there is no strong need to FFS on this point. Let’s save some time.

    - “L_ref replaces sl-LengthSymbols in TS 38.214 Clause 8.1.3.2”

- FL’s response to some companies’ comments:

  - @LGE, DCM, Intel, Apple: the previous WA is not complete since it says “At least for COT initiation …” and thus having the FFS. RAN1 needs to have a complete design. Alt 1 is a complete design.

(copied from TS 38.214) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList.

[H] Proposal 2-1c

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Down-select one of Support the followings

  - Alt 1: Support Option 4 only

  - Alt 2: Support Option 4 and Option 2, and Tx UE uses 1 bit in SCI to indicate which of them is currently used

- Note: the options are as below

  - Option 2: The reference number of symbols is determined based on 1st starting symbol

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- L_ref replaces sl-LengthSymbols in TS 38.214 Clause 8.1.3.2

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE | OK for progress | Our original view is that L_ref is not needed and sl-LenghtSymbols is sufficient, but we can accept it for progress. |
| Futurewei | OK |  |
| Apple | OK |  |
| DCM | OK |  |
| ZTE, Sanechips | No for the 4th sub-bullet | At first, we think the 4th bullet is relative to the FFS sub-bullet, which should be discussed together. And if we replace sl-LengthSymbols with L_ref directly, there may cause some problem and low efficiency. Take L_ref=10 as an example, then the number of symbols for TBS is determined as 10-2-NsymbPSFCH, which is either 8 or 5(which may less than 6), but for PSFCH slot, as agreed before, the second starting symbol cannot be used, it is not necessary to further minus 3 symbols from the L_ref. As proposed by us in previous round discussion, the following should be used to determine the number of symbols, it doesn’t require any dynamic indication in SCI, and achieves higher efficiency: min(L_ref - 2, sl-LengthSymbols - 2 -NsymbPSFCH) |
| vivo | OK |  |
| OPPO | Yes |  |
| NEC | Yes |  |
| CATT/GOHIGH | Comment | We share the similar concern as ZTE. The case that all the slots related to one TB are configured with PSFCH resources should be additionally considered, where only the 1st starting symbol is supported. |
| Transsion | No | When SL transmission is within the COT, the configuration of only a single reference number of symbols reduces the spectral efficiency, considering that the 1st starting symbol is used for SL transmission. |
| Spreadtrum | OK |  |
| WILUS | OK |  |
| Sharp |  | Regarding the 4th bullet, we think there is a valid point in ZTE’s comments, i.e. there are three cases for number of symbols for TBS to consider (two cases in a non-PSFCH slot and one case in a PSFCH slot). However, based on their reasoning our observation is that the complexity is introduced by -NsymbPSFCH. Therefore, we propose to simply let L_ref replace Nsymbsh (rather than sl-LengthSymbols). L_ref can be (pre-)configured to reflect an average case. |
| Xiaomi |  | We support the Alt 2, we think Alt 2 has the benefit on the flexibility and resource efficiency. If the reference number of symbols is preconfigured to be determine based on the 2nd starting symbol (e.g., symbol 7), but the PSSCH transmission occurs in a shared COT, and the PSSCH transmission uses the type 2C LBT, then this PSSCH is transmitted from the first starting symbol (e.g., symbol 0). In this case, if the TB size is determined based on pre-configuration, 7 symbols can’t be used to determine the TB size, the resource efficiency is low. Considering above case, we add a note under the alt2 to make a compromise, so we make the following revision to Alt2: - Alt 2: Support Option 4 and Option 2, and Tx UE uses 1 bit in SCI to indicate which of them is currently used - NOTE: When PSSCH transmission uses type 2C LBT in a shared COT, the option 2 is supported. |
| CMCC | OK for progress |  |
| Panasonic | OK |  |
| MediaTek | Comment | The majority transmissions will start from the 1st starting symbol within a slot especially for the transmissions within an initiated COT. Then Alt 1 will suffer a low spectrum efficiency if the reference number of symbols is not (pre-)configured as the 1st starting symbol. |
| Huawei/HiSilicon | See comment | Alt 2 is beneficial to increase spectrum efficiency with limited workload and complexity. We are unclear why some companies do not care about spectrum efficiency in this issue, but care about spectrum efficiency in 2 AGC symbol issue. |
| Nokia/NSB | OK | Same view as LG, that L_ref is not needed and sl-LenghtSymbols is sufficient, |
| Qualcomm | No on the 4th bullet | For the 4th bullet, we think it could have a huge impact on spectrum efficiency if the L_ref is allowed to be configured based on other symbols than the 1st starting symbol. If L_ref is set to 7 symbols, then the spectrum efficiency is only 50%. Not that we do agree to have slots with only one starting symbols, e.g., PSFCH slot. It would be the best to support L_ref for the slots with 2 starting symbols and sl-LenghtSymbols for the slots with 1 starting symbol. |
| Intel | OK | We also share same view with LGE and Nokia regarding L_ref. |
| Samsung | OK in general | We didn’t see the need for the last two bullets, which is for spec writing phase. |

#### [M] Proposal 2-2

Background: AGC symbols

- Summary of previous round

  - Regarding Tx UE behaviour:

    - Views are still divergent.

  - Regarding Rx UE behavior:

    - Most companies are fine with proposal.

- Updates on proposal (marked in red) and justifications

  - Add an FFS based on comment from VIVO and Apple. RAN1 can FFS whether this (pre-)configuration is explicit or implicit.

    - Explicit (pre-)configuration: this is very straightforward, e.g., 1 bit (pre-)configuration to indicate having 1 or 2 AGC symbols.

    - Implicit (pre-)configuration: please see example in Vivo’s and Apple’s reply above. E.g., this may be implicitly determined by the number of occupied PRBs or RB sets.

[M] Proposal 2-2c

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol,

    - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols subject to (pre-)configuration

      - FFS: whether this (pre-)configuration is explicit or implicit

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE |  | At this moment, it seem that we can make agreement only for RX part. |
| Apple | Agree |  |
| DCM | NO | Comment is the same as that in the 2nd round. Besides, we feel that more companies commented in the 2nd round do not prefer 2 AGC symbol from TX UE perspective. We should go with the direction rather than ‘1 or 2’. If ‘1 or 2’ is adopted, UE implementation of both TX UE perspective and RX UE perspective becomes more complicated. |
| ZTE, Sanechips |  | Sharing similar view with LGE, only Rx part is ready for an agreement. |
| vivo | OK | OK for the progress. Let’s see if the following minor modification can make it more acceptable to companies concerning on the Tx behavior. For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, - The PSCCH/PSSCH transmission can has 1 or 2 AGC symbols subject to (pre-)configuration - FFS: whether this (pre-)configuration is explicit or implicit - Regarding Rx UE behaviour: - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot - Note: AGC symbol is duplication of next symbol |
| OPPO |  | For Tx side, we do not support it. For Rx side, we can live with Option D for progress. |
| NEC | OK with comment | Clarification on the added FFS, if it's implicitly determined by the number of occupied PRBs or RB sets, does it imply that the (pre-) configuration is per UE? (Because it's based on each UE's resource allocation.) |
| CATT/GOHIGH |  | Only fine for the Rx UE behavior. |
| Transsion | OK only for Rx side |  |
| Spreadtrum |  | We only support Rx UE behavior in this proposal. |
| Lenovo |  | If Tx UE determines 1 or 2 AGC symbols based on its (pre)configuration for a certain resource pool, why Rx UE cannot reuse the (pre)configuration for reception in the same resource pool. Is there some case Tx UE and Rx UE have different (pre)configuration on AGC symbol number in the same resource pool? The above issues need a clarification. |
| WILUS | OK only for RX side |  |
| CMCC |  | Same view with LGE, only the Rx side part should be agreed. |
| Panasonic |  | We only agree with Rx side. |
| MediaTek | Tx: No Rx: Yes | For Tx behavior, it is not convinced that introducing a 2nd AGC symbol for Tx UE can resolve the issue of FDMed transmissions in other RB sets or resource pools considering anyway WiFi is an async system. |
| Huawei/HiSilicon | Yes for progress | Regarding Tx UE behavior, 2 AGC symbol is necessary to ensure successful decoding. As a compromise, we are ok to support both via (pre-)configuration to minimize specification workload. |
| Nokia/NSB | OK |  |
| Qualcomm | No on Tx Ok on Rx | We share the same view with MediaTek. I don’t see the need of over optimizing the AGC design in the presence of asynchronous Wifi nodes. If companies are really worried about the AGC issue for the full slot receiver, the Tx UE could (by implementation) choose lower MCS and the Rx UE could (by implementation) use the PSSCH SCH REs in 2nd starting symbol for AGC retuning and puncture that symbol in PSSCH decoding. |
| Intel | No on Tx Ok on Rx | We still are not convinced of the need of two AGC symbols, and we consider this an over optimization of an issue which could be simply solved by implementation. |
| Samsung | OK with the RX side |  |

#### [M] Proposal 2-3

Background: Tx UE behavior on using 1st or 2nd starting symbol

- Summary of previous round

  - Support (13): vivo, xiaomi, MTK, NEC, Intel, Futurewei, Transsion, Pana, HW, ZTE (only 1st bullet), DCM (only 1st bullet), Apple (only 1st bullet), Sharp (only 1st bullet)

  - Not Support (2): LGE, OPPO

- Updates on proposal (marked in red) and justifications

  - Made the following update based on comment from Nokia.

    - “For the remaining slots of a COT, at least for MCSt, …”

- FL’s response to some companies’ comments:

  - @LGE, Apple, OPPO, Sharp, etc.: there is a tradeoff between more channel access chances and Rx UE blind decoding complexity. If RAN1 allows two starting symbol in all slots within a COT, the Rx UE blind decoding would be high.

  - @CMCC: your understanding is correct.

  - @DCM: I think the proposal is aligned with your example due to the use of “…within the COT” in the last Note, i.e., we do not use “within any COT” or “within all COTs” or other wording.

    - If I misunderstand, could you please elaborate a bit, and how you want to modify the proposal (remove the last Note?).

  - @vivo: with the 2nd Note on responding UE, I’m not sure we need to differentiate the behaviour of COT initiating UE and responding UE. Can you elaborate a bit on why responding UE case needs to be FFS?

  - @Lenovo: I can understand your motivation of starting from symbol#0 from 2nd slot of a COT. But previously RAN1 already agreed both 1st and 2nd location are (pre-)configurable.

    - E.g., if the location of 1st starting symbol is (pre)configured as #3, then all SL UE can only transmit from symbol#3, no matter it’s the 1st or remaining slots of a COT.

    - Do you have any suggestion to support your preferred way while obeying previous agreement?

[M] Proposal 2-3c

Regarding Tx UE behavior:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- For the remaining slots of a COT, at least for MCSt, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS the case of non-MCSt

  - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information

  - Note: Rx UE only needs to decode PSCCH/PSSCH transmitted from 1st starting symbol in such slots within the COT.

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE |  | For progress, if we add “with no greater than 16us gap except the first transmission of the MCSt” after “at least for MCSt” and, we can accept it. MCSt can start in the middle of the shared COT, in this case, allowing 2nd starting position is beneficial to avoid the COT is interrupted by other node or RAT. |
| Futurewei | OK |  |
| DCM | NO | For the last bullet: from RX UE point of view, RX UE does not consider which TX is included in which COT. Any TX UE can start COT at any timing. That is, RX UE should always try to decode TX from both the 1st starting symbol and the 2nd starting symbol. For example, in this case below, UE-A initiates a COT from slot n and UE-C initiates a COT from slot n+2. Then if UE-B decode only from the 1st starting symbol at slot n+2 due to the existence of UE-A’s COT, UE-C’s TX from the 2nd starting symbol will be missed. To avoid such a situation, whole the last note should be removed. Besides, I do not understand Nokia’s comment. We do not think ‘MCSt’ means no TX gap or always type 2C. If LBT failure due to LBT type 2A/2B should be considered, then we believe that the following simple proposal is enough. [M] Proposal 2-3c Regarding Tx UE behavior: - For the 1st slot of a COT, tThe Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - For the remaining slots of a COT, at least for MCSt, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS the case of non-MCSt - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information - Note: Rx UE only needs to decode PSCCH/PSSCH transmitted from 1st starting symbol in such slots within the COT. |
| ZTE,Sanechips |  | Similar view as DCM |
| vivo | OK | @FL It seems to us some companies’ concern is on the responding UE, not on the COT initiating UE, thus we try the note to see if it can be a comprise. |
| OPPO |  | Even we still think this proposal should de-prioritized for discussion, we can live with it for progress with current version. Regarding MCSt, how to use the GP symbol is not decided, and how long is the gap between 2 adjacent transmissions is not decided yet. We can accept the suggestion from LGE to add a restriction for the gap length. Regarding the update by DCM, it seems like a general procedure for Tx UE after LBT to select 1st or 2nd starting symbol. |
| NEC |  | Within a COT, because of Type 2 LBT failure probability is very low, COT responding UE has high possibility to access to the channel at the 1st starting symbol. Thus, PSSCH starting from the 1st starting symbol is a good result of TX UE's channel access. However, the 2nd bullet in the proposal try to change the result to a condition for TX UE's channel access. We think though UE has high possibility to start PSSCH from the 1st starting symbol, we should not prevent it from starting PSSCH from the 2nd starting symbol if the COT is still maintained, e.g., 2nd starting symbol is still within the COT Based on this, we think the 2nd bullet is not needed. |
| CATT/GOHIGH | Yes |  |
| Transsion | OK | Allowing the Tx UE to transmit PSCCH/PSSCH from the 2nd starting symbol within the COT will increase the blind decoding complexity of the Rx UE. We see that whether or not to support multiple starting symbols is a tradeoff between more channel access opportunities and the blind decoding complexity of the Rx UE. We believe that it is worthwhile to set more starting symbols to obtain high channel access probability when Tx UE does not get COT. However, when the Tx UE gets a COT, there is no need to start transmission on a second symbol |
| Spreadtrum |  | We have the similar concern with DCM on the RX UE behavior. RX UE needs to detect the PSCCH/PSSCH in both 1st and 2nd starting symbols to avoid missing other PSCCH/PSSCH reception from 2nd starting symbol. We prefer to remove the last note. |
| Lenovo |  | @FL: We agree to add “for MCSt” in the 2nd bullet and replace “1st starting symbol with” with “symbol 0” as there are consecutive transmissions in MCSt. If 1st starting symbol is configured as symbol 3, and all transmissions are restricted to start from symbol 3, then MCSt can’t happen. According to previous agreement, for slots with 2 candidate starting symbols, the location of 1st starting symbol can be configured. For remaining slots of MCSt, there is a single starting symbol per slot, it does not revert agreement if SL transmission in remaining slots of MCSt is always started from symbol 0. Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 With addition of MCSt in 2nd bullet, we can support it with below minor change: Regarding Tx UE behavior: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - For the remaining slots of a COT, at least for MCSt, the Tx UE only chooses the 1st starting symbol 0 for PSCCH/PSSCH transmission. - FFS the case of non-MCSt - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information - Note: Rx UE only needs to decode PSCCH/PSSCH transmitted from 1st starting symbol in such slots within the COT. |
| WILUS | OK as TX UE behavior | We suggest to remove 2nd note regarding Rx UE behavior or put on FFS for Rx UE behaviors |
| Sharp |  | We are fine with LGE’s addition to limit it to cases that UE does not sense channel. |
| xiaomi | Yes |  |
| CMCC | OK | By the way, this is Tx UE’s behavior but not Rx UE’s. |
| Panasonic | OK |  |
| MediaTek | OK | For the last FFS, from our perspective, within an initiated COT, if there is a transmission from the 1st starting symbol within a slot, there should be no other transmission from the 2nd starting symbol within the same slot of the same initiated COT. Thus, there will be no PSCCH/PSSCH missing issue in such a case. If the stated case is for hidden node problem, we think it is more preferred to resolve/relax by IUC rather than mandating Rx UE monitor multiple PSCCH occasions. |
| Huawei/HiSilicon | Ok for progress | We think there is no need to differentiate whether the transmission belongs to MCSt or not. If the majority think the newly added part (i.e., MCSt) is necessary, we can accept it for progress at this stage, and come back to it in next meeting. |
| Nokia/NSB | OK |  |
| Qualcomm | OK |  |
| Intel | OK |  |
| Samsung | OK with the first bullet | For the second bullet, we need to finalize the issue of how to deal with the gap symbol between consecutive transmissions. If there is guard symbol, LBT is needed and there is no guarantee that the UE can transmit from the first starting location. We wonder what’s the necessity to have the second bullet to restrict UE TX behavior? We understand it’s a reasonable implementation, but why we need to specify it? |

### Round 4: Proposals and summary of previous round

#### [H] Proposal 2-1

Background: TBS determination

- Summary of previous round

  - Support (14): LGE, FTW, Apple, DCM, vivo, OPPO, NEC, SPRD, WILUS, CMCC, Pana, Nokia, Intel, SS

  - Not support (3): ZTE, Transsion, Xiaomi,

- Updates on proposal (marked in red) and justifications

  - Based on comment from ZTE, CATT, Sharp, etc., FL removed 4th bullet and keep it as FFS. Companies can bring more details next meeting.

- FL’s response to some companies’ comments:

  - @Xiaomi: thanks for the compromise, maybe you can try to have more offline with Alt 1 camp to see whether they can accept the compromise.

  - @LGE, MediaTek, QC, SS: 4th bullet if FFS now, let’s discuss it in next meeting.

(copied from TS 38.214) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList.

[H] Proposal 2-1e (TBS, L_ref)

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Support the followings

  - Alt 1: Support Option 4 only

- Note: the options are as below

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- L_ref replaces sl-LengthSymbols in TS 38.214 Clause 8.1.3.2

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

FL’s comment: this proposal is near stable, let’s discuss by email directly.

#### [M] Proposal 2-2

Background: AGC symbols

- Summary of previous round

  - Regarding Tx UE behaviour:

    - Views are still divergent. Suggestions are welcome on how to move forward.

  - Regarding Rx UE behavior:

    - Most companies are fine with proposal.

- Updates on proposal (marked in red) and justifications

  - FL splits the Tx and Rx proposal, hope the Rx proposal can be endorsed by email directly so that we can have “some small progress”.

- FL’s response to some companies’ comments:

  - @Vivo: I feel “has 1 or 2” is more accurate than “can have 1 or 2”.

  - @NEC: it’s open, up to the companies’ detailed design.

[M] Proposal 2-2-1e (AGC, Tx)

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol,

    - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols subject to (pre-)configuration

      - FFS: whether this (pre-)configuration is explicit or implicit

- Note: AGC symbol is duplication of next symbol

[M] Proposal 2-2-2e (AGC, Rx)

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

FL’s comment: on Proposal 2-2-1e (AGC, Tx), please provide your view below. Suggestions are welcome on how to move forward.

FL’s comment: on Proposal 2-2-2e (AGC, Rx), this proposal is near stable, let’s discuss by email directly.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm |  | We still don’t see the need of introducing 2nd AGC symbol in the presence of async Wifi. The 2nd AGC symbols also have issues of overhead, phase discontinuity and UE complexity, and the gain is not clear in unlicensed deployment. |
| LGE |  | RX UE may or may not perform AGC procedure twice. We think that the any further change would be needed for 2 AGC. For compromise, if RAN1 does not pursue the specific enhancement for 2 AGC symbol for TX UE perspective, we can accept it without (pre)configuration, that is “The PSCCH/PSSCH transmission has 1 or 2 AGC symbols”. |
| Ericsson |  | We are still not convinced by having 2 AGC symbols for TX UE perspective. LBT should prevent if there is sudden change in the power level of 2nd symbol. We believe that 1 AGC symbol is fine. |
| ZTE, Sanechips |  | Share similar view with Qualcomm, the benefit has not been justified but with large overhead and UE complexity. |
| NEC | OK |  |
| Lenovo |  | We think it is better to discuss Tx UE and Rx UE behaviors together. |
| OPPO |  | Proposal 2-2-1e (AGC, Tx) Different Tx UE may start from 1st or 2nd starting symbols, and it is up to RX UE’s implementation to monitor 1 or 2 AGC symbol(s) if it is agreed. Rx UE’s behavior determines Tx UE’s behavior, and considering there are UEs always monitor 2 AGC symbols by implementation, Tx UE should have 2 symbols for AGC. |
| Intel |  | We share same view with other companies regarding the cons of introducing two AGC symbols while the benefits may be doubtful. |
| Spreadtrum |  | We support Rx UE behavior in Proposal 2-2-2e. There may exist the case that RX UE always monitor 2 AGC symbols mentioned by OPPO. TX UE should support 2 AGC symbols to avoid the impact on the PSCCH/PSSCH reception of other TX UE. As a compromise, we can agree that the PSCCH/PSSCH transmission can have has 1 or 2 AGC symbols firstly, then can further discuss whether it is (pre-)configured, under some condition(s), or up to UE implementation in the next meeting. [M] Proposal 2-2-1e (AGC, Tx) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, - The PSCCH/PSSCH transmission can have has 1 or 2 AGC symbols subject to (pre-)configuration - FFS: whether this (pre-)configuration is explicit or implicit - FFS: whether it is (pre-)configured, under some condition(s), or up to UE implementation. - Note: AGC symbol is duplication of next symbol |
| MediaTek |  | We share similar views as QC. For the supporter of 2 AGC symbols regarding Tx UE, we just wondering how to address the impact of WiFi on SL-U considering anyway WiFi is an async system. Additionally, we also think if there is only 1 AGC symbol in a slot for Tx UE, it is then unnecessary for Rx UE to monitor two AGC symbols in such slot. |
| vivo | OK | Also OK with spreadtrum’s revision. |
| Nokia/NSB |  | We think the 2nd AGC symbol can be needed in some scenarios as explained in our Tdoc, where practically the transmission range of SL-U UE can be larger than the LBT detection range, which may result in the AGC issues as discussed in above that could not be solved by LBT, meaning that for some scenarios, the AGC issues simply could not be ignored. To avoid the overhead issue brought by the 2nd AGC symbol, as a tradeoff, the needs of the 2nd AGC symbol can be based on Tx UE indication. Regarding Tx UE behavior, we have the following proposal: [M] Proposal 2-2-1e (AGC, Tx) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols subject to (pre-)configuration - FFS: whether this (pre-)configuration is explicit or implicit - FFS: Operation based on Tx UE indication on 1 or 2 symbol for AGC purpose. - Note: AGC symbol is duplication of next symbol Similarly, for Rx UE behavior, we have the following proposal: [M] Proposal 2-2-2e (AGC, Rx) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Rx UE behaviour: - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot - FFS: Operation based on Tx UE indication on 1 or 2 symbol for AGC purpose. - Note: AGC symbol is duplication of next symbol |

#### [M] Proposal 2-3

Background: Tx UE behavior on using 1st or 2nd starting symbol

- Summary of previous round

  - Support (13): FTW, vivo, OPPO, CATT, Transsion, Xiaomi, CMCC, Pana, MTK, HW, Nokia, QC, Intel,

  - Not Support (2): DCM, ZTE,

- Updates on proposal (marked in red) and justifications

  - Made the following updates based on comment from LGE, OPPO, etc.

    - “…at least for MCSt with no greater than 16us gap…”

    - “FFS the other cases of non-MCSt”

  - Removed the last Note on Rx UE behavior and make it FFS, based on comments from Docomo, ZTE, etc.

- FL’s response to some companies’ comments:

  - @Lenovo: I feel you need to have more offline to convince the group that your proposal is aligned with previous agreement.

[M] Proposal 2-3e (COT, Tx UE behaviour)

Regarding Tx UE behavior:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- For the remaining slots of a COT, at least for MCSt with no greater than 16us gap, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS the other cases of non-MCSt

  - Note: if the Tx UE is a responding UE who shares the COT, the Tx UE behaves as above after decoding COT sharing information

  - Note: Rx UE only needs to decode PSCCH/PSSCH transmitted from 1st starting symbol in such slots within the COT.

  - FFS: Rx UE behavior

FL’s comment: this proposal is near stable, let’s discuss by email directly.

## Issue#3: PSCCH/PSSCH

### [Closed] 1st round: Proposals and background

#### [H] Proposal 3-1

Background: IRB, frequency domain resource indication

- Summary

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

    - Support (14): OPPO, Huawei/HiSilicon, Intel, Fraunhofer, MediaTek, CATT, CMCC,  Lenovo, ITL, NEC, Apple, Sharp, Panasonic , ITL, WILUS

  - Option B: Support that the used interlace index(s) in different RB sets can be different

    - Support (2): [Nokia], InterDigital

- Summary

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

    - Support (17): Qualcomm, OPPO, Huawei/HiSilicon, LGE, MediaTek, CATT, CMCC, Lenovo, NEC, WILUS, [Docomo], ZTE, Apple, Intel, Sharp, Spreadtrum , Panasonic

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - Support (2): Nokia, Fraunhofer

- FL’s view

  - Thanks to the down-section in RAN1#112 that “1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool”, most companies share similar view on the frequency domain resource indication design as shown above.

  - Many companies also gave detailed designs:

    - ZTE, CATT propose that for a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s).

    - LGE, ZTE, OPPO, CATT, Qualcomm, etc: signaling design can be based on legacy NR SL FRIV.

    - Sharp, CMCC, ZTE, etc: considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource and update resource selection, etc.

  - FL gave a proposal to reflect the above.

    - Note: NR-U specified the “intersection” behavior in TS 38.214 as below. FL adopts similar wording for SL-U.

      - From TS 38.214: “… For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. …”

[H] Proposal 3-1

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that the used interlace index(s) in different RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s)

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource and update resource selection, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | Although our preference is to allow different interlace index(s) in different RB sets (Option B with some restriction), we can live with Option A in consideration of the remaining time. However, we are not sure why the initial transmission and retransmissions shall use the same number of RB set(s). For example, 1st TX uses 2 sub-channels in a single RB set and then 2nd TX (reTX) uses 1 sub-channel in RB-set A and 1 sub-channel in RB-set B. Why is this flexibility precluded? |
| vivo | OK |  |
| LGE |  | We do not need to have the 2nd bullet of the Option 1. Even in Rel-16/17 NR SL, we don’t have such a restriction. In other words, in rel-16/17 NR SL, different number of subchannels between initial TX and reTX are already allowed. |
| Nokia/Nsb | Agree |  |
| OPPO | Yes | OK with the proposal. |
| Interdigital | Yes | Even though we support Option B we can accept the majority’s preference. |
| Apple | Yes with comment | We are generally fine with the direction of this proposal. For Option A, it should be clarified that if a transmission uses interlaces over several RB sets, then the used interlaces only in these RB sets are the same. Consider an example that a resource pool has 2 RB sets, and a UE uses the first interlace in the first RB sets but does not use any interlace in the last RB set. In this case, the used interlace index in the first RB set is different from the used interlace index in the second RB set, i.e., it is not always the same. |
| NEC | No | Could you elaborate more on the intention of 2nd bullet in option 2? The other parts are OK for us. |
| Ericsson |  | Same interlace in different RB sets with an offset to ensure equal spacing between RBs. |
| Fraunhofer | OK |  |
| Qualcomm | OK |  |
| Samsung | Agree |  |
| Sharp |  | On the 3rd bullet of Option 1, we prefer to use R16 NR-U RIV/bitmap for indicating sub-channel index(es) such that SL-U and NR-U can be well co-existed in terms of resource allocation. For this meeting we think the bullet “R16 NR SL FRIV is reused as baseline” can be removed, and details can be decided in the next meeting. |
| Intel | OK |  |
| Futurewei | OK |  |
| Lenovo |  | One concern on “R16 NR SL FRIV is reused as baseline”. Since R16 SL FRIV uses contiguous subchannels in frequency domain, it causes contiguous interlaces in frequency domain based on R16 SL FRIV. In that sense, R16 NRU non-contiguous interlace combinations will not be supported for R18 SL-U, which leads to PSD limitations |
| Spreadtrum | OK |  |
| Sony | OK |  |
| CMCC | OK |  |
| Xiaomi | Yes | We are generally fine with the proposal. For the FFS part, we prefer to reuse single-slot candidate resource and the legacy sidelink resource selection as much as possible. |
| CATT/GOHIGH | Generally fine | We are generally fine with the direction of this proposal. Regarding the 1st bullet, some clarification is needed. From our understanding, option A is mainly defined from the perspective of one PSSCH transmission, i.e., the used interlace index(s) for one PSSCH transmission in different RB sets are always the same, but the used interlace index(s) for different PSSCH transmissions of one TB are not necessarily the same. Regarding the 2nd sub-bullet of option 1, we think it is really needed and technically justified. Firstly, if we want to use FRIV to indicate RB set information, the RB set number for different transmissions of one TB should be the same, otherwise, FRIV cannot be directly used. Besides, the used sub-channels in different RB set for one PSSCH transmission should also be the same considering the following two aspects: - If the number of used sub-channels per RB set is not the same, there is a potential power imbalance in different RB sets due to PSD limit restriction. - From the perspective of resource indication, if the number of used sub-channel per RB set is not exactly the same, then multiple FRIVs are needed for resource indication which will cause larger signaling overhead. However, if the direction of the 2nd sub-bullet is adopted, then only one FRIV is need to indicate the used sub-channel(s) of the reference RB set, and then the used sub-channel(s) of other RB set(s) can be identified implicitly. Regarding the final FFS part, we think the signaling design is also needed, such as whether RB set related information is needed to be indicated from higher layer [H] Proposal 3-1 Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Option A: Support that the used interlace index(s) for one PSSCH transmission in different RB sets are always the same - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s) - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, etc. - FFS others - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource and update resource selection, higher layer parameter indication, etc. |
| Panasonic | Yes |  |
| Transsion | Yes | We support this proposal. |
| MediaTek | Comment | For the last sub-bullet of Option 1, from our point of view, two interlace indication methods are defined for NR-U, i.e., RIV from 15kHz SCS and bitmap for 30kHz SCS as described below (copied from TS 38.214, Clause 6.1.2.2.3) TS 38.214 Clause 6.1.2.2.3 For µ=0, the X=6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by: For µ=1, the X=5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE. Therefore, we think for the indication of sub-channel, two options of RIV and bitmap should not be included based on the SCS. The other proposals are OK from our side. |
| ZTE | OK |  |
| Huawei/HiSilicon | Yes |  |
| WILUS | Agree |  |

#### [M] Proposal 3-2

Background: IRB, remaining details on mapping between sub-channel and interlace

- Summary

  - RAN1#112 supported that “1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool”.

  - As shown in Figure 1 below, some companies (Ericsson, Docomo, vivo, CATT, MediaTek, LGE, etc.) point out that depending on the (pre-)configuration, in a resource pool, the interlace with lowest frequency location may not be interlace#0 (assume interlace is indexed as per NR-U), so RAN1 needs to clarify the detailed mapping.

    - E.g., in Figure 1 below, in RP1, the interlace with lowest frequency location is #2, not #0.

  - In addition, the above companies propose that “sub-channel with the same index is mapped to interlace with the same index in different RB sets”, to ensure regular waveform.

    - E,g., in Figure 1 below, in RP0, sub-channel 0 in RB set 0 and RB set 1 are both mapped to interlace 0.

- FL’s view

  - A proposal is given to reflect the above.

Figure 1 Illustration of mapping between sub-channel and interlace (Based on Figure from R1-2303368)

[M] Proposal 3-2

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to interlace with the same index in different RB sets.

- In a resource pool, down-select one of the following

  - Option 1: sub-channel#0 is mapped to interlace with lowest frequency location in the resource pool

  - Option 2: sub-channel#0 is mapped to interlace#0

    - Interlace is indexed as per NR-U

FL’s comment: please also indicate which Option you support.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | YES | It seems that both options can work without any difference, then just Option 2 as per NR-U can be agreed. Or if there is performance difference, we are open to consider Option 1. |
| vivo | comment | - The first sub-bullet (and likely the second) seems to miss the case of 1 sub-channel mapped to 2 interlaces (i.e., K=2). - The last sub-bullet “Interlace is indexed as per NR-U” seems to be applicable to all the sub-bullets (i.e., not only the option 2), right? - We slightly prefer option 1. |
| LGE | Yes | Option 2 is supported as in NR-U. Similarly, we also need to clarify that the lowest sub-channel implies the sub-channel with the lowest index for PSCCH mapping. Otherwise, for PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest location, since different PSSCH allocations can have the same PSCCH location, there could be ambiguity on the PSSCH allocation even though the RX UE knows the location of PSCCH and the number of sub-channels. |
| Nokia/Nsb | Agree | Option 2 is preferred |
| OPPO | Yes |  |
| Interdigital | Yes |  |
| Apple | Yes |  |
| NEC | Yes |  |
| Ericsson |  | It is not clear to use that the proposal ensures that the waveform is regular. The intracell guard band breaks the regularity. Proposal: Mapping to between sub-channels and interlaces ensures that the waveform is regular |
| Fraunhofer | Yes |  |
| Qualcomm | Yes | Option 2 as in NR-U is preferred since it guarantees equal spacing among interlaced RBs across different RB-sets and reduces the PAPR of Tx waveform. |
| Samsung | Agree |  |
| Sharp | Yes | Option 2 is preferred. To reflect that 1 sub-channel equals to K=2 interlaces for 15kHz SCS, a minor modification is proposed. For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(es) in different RB sets. - In a resource pool, down-select one of the following - Option 1: sub-channel#0 is mapped to K interlace(s) starting from interlace with lowest frequency location in the resource pool - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace interlace#0 - Interlace is indexed as per NR-U |
| Intel | Yes | As both options can work without any drawbacks, we prefer option 2 as this follows NR-U. |
| Futurewei | OK |  |
| Lenovo |  | OK with the 1st bullet. For 2nd bullet, it is straightforward to map subchannel 0 to interlace 0 as NR-U. |
| Spreadtrum | Yes | We prefer option 2 as in NR-U. |
| Sony | OK |  |
| CMCC | Yes | We are fine with both options. |
| xiaomi | Yes | We prefer option 2. |
| CATT/GOHIGH | Yes | We are fine with the first bullet. Regarding the second bullet, option 1 is the preference. |
| Panasonic | Yes | Option 2 as in NR-U is preferred. |
| MediaTek | Yes with comment | Option 2 is supported per NR-U, which can solve the sub-channel to interlace mapping relation across different RB sets within one RP. In additionally, we are not clear whether the current proposals also capture the sub-channel to interlace mapping relation across different RPs (the current agreement on sub-channel definition is restrained within one RP as pointed by FL). From our understanding, even across different RPs, the current proposal of the 1st bullet and Option 2 in the 2nd bullet are suitable as well. Therefore, we may want to add a note to reflect the above issue. For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to interlace with the same index in different RB sets. - In a resource pool, down-select one of the following - Option 1: sub-channel#0 is mapped to interlace with lowest frequency location in the resource pool - Option 2: sub-channel#0 is mapped to interlace#0 - Interlace is indexed as per NR-U Note: Across different resource pool, the sub-channel to interlace mapping relation defined above is also applicable. |
| ZTE | Yes | Option 2 is preferred |
| Huawei/HiSilicon | Yes |  |
| WILUS | Yes | Option 2 is supported as in NR-U. |

#### [M] Proposal 3-3

Background: IRB, TBS determination

- Summary

  - The following companies point out that since one interlace in one RB set may include 10 or 11 PRBs subject to (pre-)configuration, and initial and retransmission may occupy different interlace, RAN1 needs to clarify how TBS is determined.

    - Qualcomm, Huawei/HiSilicon, LGE, Apple, OPPO, CATT, Sharp, Spreadtrum, etc.

    - E.g., assume one PSSCH transmission occupies 1 sub-channels in 1 RB set, and assume 1 sub-channel equals 1 interlace

      - Assume initial transmission occupies interlace#0 which has 10 PRBs in a RB set

      - Assume retransmission occupies interlace#1 which has 11 PRBs in a RB set

      - Then, how to calculate TBS. Different solutions are given by companies.

- FL’s view

  - A proposal is given to reflect the above.

[M] Proposal 3-3

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s):

- TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings:

  - Option 1: N_ref is a fixed value, e.g., 10, 11

  - Option 2: N_ref is (pre-)defined

  - Option 3: N_ref is (pre-)configured

  - Option 4: N_ref is dynamically indicated by Tx UE

FL’s comment: please also indicate which Option you support.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM |  | We are not sure when the number of PRBs within 1 RB set is different. The situation should be clarified first. |
| vivo | No | We are not sure this is really an issue. If P3-1 is agreed, it is probably that the number of REs are same between initial Tx and re-transmissions. Even if there is slight difference between them, thanks to the quantization step of TBS determination, the slight difference between the number of PRBs of allocated subchannels and the number of actual PRBs may not have actual impact on TBS determination. |
| LGE | Yes | We think that N_ref=10 is enough. As we know, in Rel-16/17 NR SL, the subchannel size of 10 PRB is already supported, but not 11 PRBs. It would be useful to reuse the same set of TBS compared to the Rel-16/17 NR SL even for the interlaced structure. |
| Nokia/Nsb | Agree | Option-2/3/4 |
| OPPO | Yes with comment | We are OK with the direction, and for the solutions options, we would like to add one more option: [M] Proposal 3-3 For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s): - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings: - Option 1: N_ref is a fixed value, e.g., 10, 11 - Option 2: N_ref is (pre-)defined - Option 3: N_ref is (pre-)configured - Option 4: N_ref is dynamically indicated by Tx UE - Option 5: N_reft is the average number of PRBs per sub-channel, which is determined by total number of PRBs of the RP divided by the number of sub-channels. |
| Apple | Yes | We prefer Option 3 for flexibility. It is unclear to us the difference between Option 1 and Option 2. |
| NEC | Yes | The number of PRBs in different interlace may be different (10/11 PRBs). Thus, option 2/4 is preferred to solve TBS determination issue. |
| Ericsson | Yes | We prefer Option 1. |
| Qualcomm | Yes | We prefer N_ref being a fixed value (option 1) and the value can be chosen to be the minimum number of interlaced RBs in different interlaced subchannels from all the RB-set(s) |
| Samsung |  | Agree with the comment that the potential issue without the proposal should be identified first. |
| Sharp | Yes |  |
| Intel | Yes | We prefer option 1 and we do not think that pre-configuration is need as we believe this would be an optimization. As commented by vivo, even if number of allocated PRBs may be different between interlaces, the difference is actually very slight and may not have actual impact on TBS determination. |
| Futurewei | OK |  |
| Lenovo |  | It is not clear enough to us. E.g., for 15kHz SCS, the number of PRBs is 11 for interlace 0 to 5, for PSSCH on one or more interlaces of interlace 0 to 5, N_ref is not needed; the number of PRBs is 10 for interlace 6 to 9, for PSSCH on one or more interlaces of interlace 6 to 9, N_ref is not needed. Even the initial transmission on interlace 0 and retransmission on interlace 6, there is a single RB difference. Considering TBS quantization, the difference is trivial. |
| Spreadtrum | Yes | We prefer option 3. |
| Sony | Yes |  |
| CMCC | Yes | Maybe option 3 is better, the condition is similar to 2 starting symbol cases. |
| xiaomi | Yes | - We prefer option3, in the legacy sidelink, the subchannel size is preconfigured, and the similar design can be reused to SL-U. |
| CATT/GOHIGH | Yes | We prefer option 1/2 |
| Panasonic | Yes | We prefer Option 1 with N_ref=10. |
| MediaTek | Yes |  |
| ZTE | Yes | For PSCCH/PSSCH transmission across multiple RB sets , PRBs within guard band between the multiple RB sets can be used according to the previous agreement. Therefore, TBS is also related to guard band between adjacent RB sets for PSCCH/PSSCH transmission across multiple RB sets. We would like to add one bullet as following: [M] Proposal 3-3 For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s): - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings: - Option 1: N_ref is a fixed value, e.g., 10, 11 - Option 2: N_ref is (pre-)defined - Option 3: N_ref is (pre-)configured - Option 4: N_ref is dynamically indicated by Tx UE - The determination of TBS also depends on the additional available of PRB(s) in guard band(s) for PSCCH/PSSCH transmission across multiple RB sets - the additional available of PRB(s) can be dynamically indicated by Tx UE - FFS: the number of additional available of PRB(s) |
| Huawei/HiSilicon | Yes | We prefer Option 4. Due to the existence of intra-cell guard band PRB, when a transmission occupies multiple RB sets, the available PRBs in different sub-channels may be different, resulting in the number of total PRBs of different transmissions are in relative big difference. |
| WILUS | Yes | We prefer Option 1 with N_ref=10. |

#### [M] Proposal 3-4

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary

  - For contiguous PRB case, there was an “FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs”, i.e., sub-channel 3 and 4 as shown in Figure 2 below.

  - The following companies propose to use such sub-channels to improve resource efficiency.

    - ZTE, Apple, Nokia, Qualcomm, Huawei/HiSilicon, Intel, etc.

  - Different solutions are given, with similar idea that such sub-channel(s) are used for “PSCCH and PSSCH transmission” or “PSSCH only transmission” depends on condition, e.g., may depend on the number of remaining PRBs excluding intra-cell guardband PRBs, etc.

  - CATT, OPPO, CMCC, etc: this may have impact on resource selection.

- FL’s view

  - A proposal is given to reflect the above.

Figure 2 Illustration of sub-channel(s) including intra-cell guardband PRBs (Based on Figure from R1-2302550)

[M] Proposal 3-4

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs:

- Such sub-channel(s) can be used for “PSCCH and PSSCH transmission” or “PSSCH only transmission” depending on conditions

  - FFS detailed condition

    - E.g., whether depending on the number of remaining PRBs excluding intra-cell guardband PRBs, etc.

  - FFS impacts on resource selection

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM |  | We think contiguous RB-based structure should be deprioritized in consideration of the remaining time. |
| vivo | No | We understand the motivation to improve the spectrum efficiency. But it seems this issue can be avoided/mitigated by proper configuration. |
| LGE |  | When we see this issue, it would be much better to go back to previous Option 2. At this moment, we are fine whit this direction according to the current agreement set. Regarding the 1st FFS, the example needs to be removed or changed into “whether depending on the PSCCH PRBs are overlapping with inter-cell guardband PRBs”. |
| Nokia/Nsb | Not agree | Do not understand the intention why there is the need to support PSSCH only with remaining PRBs? The basic question for this issue to discuss is whether the remaining PRBs can be treated as an independent subchannel for PSCCH&PSSCH transmission |
| OPPO | Agree |  |
| Interdigital | No | We prefer R16’s solution, in which such sub-channel can be considered unused. Proper network configurations can limit the unused PRBs. |
| Apple | Agree | Regarding Nokia’s question, in our understanding, the following agreement prevents the PSCCH transmission on guard band. If the number of PRBs in a sub-channel not in guard band is less than the number (N) of configured PRBs for PSCCH, then this sub-channel cannot used for PSCCH transmission. Furthermore, PSCCH generally occupies the lowest PRBs in a sub-channel. If the first N PRBs of a sub-channel have overlap with guard band, we may need to specify how to avoid the overlap between PSCCH and guard band. Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission |
| NEC | Yes | We should also FFS the impacts on resource indication to RX UE because RX UE may not know whether this remaining PRBs are used or not by the TX UE. |
| Ericsson | OK |  |
| Qualcomm |  | We share the same view as LGE. Suggest to remove the e.g. in 1st FFS or include the following options provided by companies. For example Option 1: Do not transmit PSCCH in the subchannel overlapping with intra-cell guardband PRBs Option 2: Depend on whether depending on the PSCCH PRBs are overlapping with inter-cell guardband PRBs Option 3: Depend on whether depending on the number of remaining PRBs excluding intra-cell guardband PRB Option 1 or 2 avoid irregular PSCCH rate matching pattern in time/frequency and are preferred |
| Samsung | OK in general |  |
| Sharp |  | We are fine with the direction of Qualcomm’s proposed changes. Furthermore, if such sub-channel is not used for PSCCH transmission, updating definition of “candidate resource” is necessary. “definition of candidate resource” is added in the last FFS. We think the following changes in red can be useful: For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select to one of the following options: - Option 1: such sub-channel(s) cannot be used for PSCCH transmission. - Option 2: use of such sub-channel(s) for PSCCH transmission is conditional. - FFS detailed condition - E.g., whether depending on the number of remaining PRBs excluding intra-cell guardband PRBs, etc. - FFS impacts on definition of candidate resource and resource selection |
| Intel | Yes |  |
| Lenovo |  | We prefer to deprioritize this issue. |
| ETRI | Yes |  |
| Spreadtrun | No | It should be deprioritized, or such sub-channel can be considered unused as in R16 SL. |
| Sony | Yes |  |
| CMCC |  | Maybe the intention of this proposal should be clarified first. The following two understanding may cause some confusion: - This proposal is discussing whether/when the sub-channel including the PRBs in intra-cell guard band can be used for PSCCH/PSSCH; - This proposal is discussing whether/when PSCCH can be transmitted using the PRBs in intra-cell guard band (with the consideration of previous agreement raised by Apple). |
| xiaomi | Yes |  |
| CATT/GOHIGH | Comment | Such sub-channel only with part guard band PRBs cannot be used for PSCCH/PSSCH transmission, such as single sub-channel 3 or single sub-channel 4 as in the figure proposed by FL, but the combination of sub-channels with guard band PRB can be used for PSCCH/PSSCH transmission, such as sub-channel 3+sub-channel 4 which is consistent with the previous agreement. |
| Panasonic | Yes |  |
| MediaTek | Comment | We share similar opinion as LGE on the 1st FFS and Option 1 from Qualcomm’s comment is OK from our side. |
| ZTE | OK with comments | For contiguous RB-based PSCCH/PSSCH transmission in SL-U,it should be clarified that whether OCB requirement also needs to be considered. Reflecting change as below For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs: - Such sub-channel(s) can be used for “PSCCH and PSSCH transmission” or “PSSCH only transmission” depending on conditions - FFS detailed condition - E.g., whether depending on the number of remaining PRBs excluding intra-cell guardband PRBs, etc. - FFS impacts on resource selection FFS whether/how OCB is considered for contiguous RB based PSSCH/PSCCH transmission. |
| Huawei/HiSilicon | Yes | As per R16 SL design, the minimum size of a sub-channel is 10 PRBs while the default number of PRBs within guard band is less than 10 PRBs as per TS 38.101, thus, it’s hard to avoid sub-channel overlapping with guard band PRBs by (pre-)configuration. Such sub-channels should be used to increase spectrum efficiency. |
| WILUS | Yes |  |

#### [L] Proposal 3-5

Background: IRB or CRB

- Summary

  - Qualcomm, Samsung, CMCC propose that a resource pool is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

- FL’s view

  - A proposal is given to reflect the above.

[L] Proposal 3-5

A resource pool is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM |  | We think contiguous RB-based structure should be deprioritized in consideration of the remaining time. |
| vivo | OK |  |
| LGE |  | We think that as in NR-U SL BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both. |
| Nokia/Nsb | Agree |  |
| Interdigital | Yes |  |
| Apple | Yes |  |
| NEC | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Agree |  |
| Sharp | Yes |  |
| Intel | Yes |  |
| Futurewei | OK |  |
| Lenovo | Yes |  |
| ETRI | Yes |  |
| Spreadtrum | Yes |  |
| Sony | Yes |  |
| CMCC | Yes |  |
| xiaomi | Yes |  |
| CATT/GOHIGH | Yes |  |
| Panasonic | Yes |  |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| Huawei/HiSilicon | Yes |  |
| WILUS | Yes |  |

#### Others

- Some other issues are mentioned:

  - Some companies propose to add more values for K (1 sub-channel = K interlace). While some other companies are against this.

  - Some companies propose other enhancements, e.g., supporting PSCCH locates in every RB set of corresponding PSSCH, enhancing TRIV, etc. While some other companies are against this.

- FL assumes such issues are not very urgent at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |
| LGE | For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest location, since different PSSCH allocations can have the same PSCCH location, there could be ambiguity on the PSSCH allocation even though the RX UE knows the location of PSCCH and the number of sub-channels. In this case, there are two approaches. Option 1: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest index. Option 2: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest frequency location. - For the case when different PSSCH allocations have the same PSCCH location, some PSSCH allocation is not used to avoid the ambiguity on the PSSCH allocation between TX UE and RX UE. |

### [Closed] 2nd round: Proposals and summary of previous round

#### [H] Proposal 3-1

Background: IRB, frequency domain resource indication

- Summary of previous round

  - Support (20): vivo, Nokia, OPPO, InterDigital, Apple, Fraunhofer, Qualcomm, Samsung, Intel, Futurewei, Spreadtrum, Sony, CMCC, Xiaomi, CATT, Panasonic, Transsion, ZTE, Huawei/HiSilicon, WILUS

  - Not Support (2): DCM, NEC

- Updates on proposal (marked in red) and justifications

  - Add “Option A: Support that for one PSSCH transmission, the used interlace index(s) in different RB sets are always the same” as per CATT’s, Apple’s comment.

  - Add “FFS: whether additionally support different number of sub-channel(s) and different number of RB set(s) in such case” as per Docomo’s comment, companies can further check.

  - Add “FFS details, e.g., signaling design, bit size, whether to consider bitmap design, etc.” since Sharp, Lenovo may want to further consider bitmap.

- FL’s response to some companies’ comments:

  - @Docomo, LGE, NEC: Regarding “For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s)”:

    - The intention is to follow legacy NR SL design and reduce signaling overhead.

    - E.g., in legacy NR SL, all the initial transmission and retransmissions will use the same number of sub-channels LsubCH, as provided by higher layer to PHY during resource selection. Hope this clarifies.

  - @CATT: regarding your last comment, FL assumes it is some further details and can be discussed later.

[H] Proposal 3-1b

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of sub-channel(s) and different number of RB set(s) in such case

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource and update resource selection, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes with comments | For the FFS part which is marked as red, we think different number of sub-channel(s) and different number of RB set(s) should not be considered because it will increase the resource indication overhead; In R16, a same number of sub-channel(s) is kept for initial transmission and retransmission, then only additional starting position is needed to be indicated for the retransmission (starting position of the initial transmission can be derived by PSCCH), therefore, same principle should be followed in our view; We are open to further consider the bitmap design which is also marked as red. |
| Apple | Yes with comment | We think the definition of Option A should be further updated to address our comment in the first round: Option A: Support that for one PSSCH transmission, the used interlace index(s) in different RB sets (if they are used) are always the same |
| InterDigital | Yes |  |
| Intel | Yes |  |
| Fraunhofer | Yes |  |
| vivo | Yes |  |
| DCM |  | We can accept this proposal with the newly added FFS, but our view is that the number of sub-channel should be the same. To reflect this, we suggest the following small update. - FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case |
| LGE | OK in principle | On the 2nd bullet of the Option 1, now I understand what is the intension. However, precisely, when the chain is broken or not maintained, even in Rel-16/17 NR SL, such restriction does not exist. So, it would be better to change into following wording: - For PSSCH allocation in a slot where the corresponding SCI is transmitted and PSSCH allocation indicated by the SCI, the same number of sub-channel(s) and same number of RB set(s) are used (i.e., a single SCI indicate a single value for the number of sub-channel(s) and a single value for the number of RB set(s) for PSSCH allocation(s)). With the above wording, the FFS is no longer needed. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| NEC | Yes | Thank you for the clarification, seems we misunderstood the first round proposal. |
| Samsung | Ok in general | We prefer to remove the following FFS - FFS: whether additionally support different number of sub-channel(s) and different number of RB set(s) in such case |
| Panasonic | Yes |  |
| Spreadtrum | Yes | We are fine with the updated proposal by DCM. |
| MediaTek | Yes with comment | We share similar concern on the new added FFS. From our side, the sub-channel(s) number for initiation transmission and retransmission should be same while the RB set(s) number can be different. The current modification from DCM still remains the case of different sub-channel(s) number. Therefore, we suggest the following modification: - FFS: whether additionally support same number of sub-channel(s) and different number of RB set(s) in such case We support bitmap design as the resource indication. |
| Lenovo | Yes | Generally, we are OK with this proposal. Additional comments are listed below: (1) Regarding allocated RB sets, it is worth noting that only contiguous RB sets can be supported. This should be captured in the proposal since it impacts on signaling design details. (2) In LTE eLAA and NR-U, spec supports predefined non-contiguous interlace combinations, e.g., interlace 0+5, interlace 1+6, …, when more than one interlaces are assigned so as to avoid PSD limitation of contiguous interlace allocation. We think this principle should be reused for SL-U; otherwise, PSD limitation happens for contiguous interlace allocation. |
| Sharp |  | For the third bullet of Option 1, we think it is fine to reuse Rel-16 FRIV for RB set indication. However, for sub-channel (interlace) indication, we are concerned about reuse of Rel-16 SL FRIV which would make it difficult to co-exist with NR-U resource allocation and would not be future proof. Since FL had added “consider bitmap design” (which is clearly not using Rel-16 SL FRIV) in the 2nd sub-sub-bullet, we think 1st sub-sub-bullet (“R16 NR SL FRIV is reused as baseline”) should be removed to avoid confusion and can be put in FFS details for further study. - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, R16 NR SL FRIV design, or whether to consider R16 NR-U bitmap design, etc. |
| WILUS | Yes |  |
| xiaomi | Yes |  |
| ZTE, Sanechips | Agree in principle | - The first FFS may not be needed if we agree to reuse R16 NR SL FRIV for subchannel index(s) indication and RB set index(s) indication. - For Option A, according to option 1 in the second bullet, the used subchannel index(s) in different RB sets should be always the same for a PSSCH transmission, and if Proposal 3-2b is agreed, then option A is always there, so we proposed to discuss Proposal 3-2b first. |
| Transsion | Yes |  |
| CATT/GOHIGH | Yes with comments | We are generally fine with the updated proposal. Regarding the newly added FFS, we think it is not needed. If we want to reuse the FRIV defined in R16/R17 NR SL, the RB set number and sub-channel number should be consistent between different transmissions of one TB, otherwise it cannot be reused directly or will cause larger signaling overhead. Regarding the higher layer parameter indication we proposed in the first round discussion, from our understanding, it is parallel with candidate single-slot resource definition and resource selection procedure. All of them will be captured in TS 38.214 Section 8.1.4 if needed. Anyway, it is just a proposed example with justification, we can have further discussion. So it is preferred that if such FFS issue (higher layer parameter indication) can be added. |
| Huawei/HiSilicon | Yes | On the FFS, our view is that the number of sub-channel should be the same. Firstly, the impact of difference number of sub-channels used among different (re)transmissions on TBS determination is very large and it can hardly ensure the same TBS, which had been discussed in R16 SL. In addition, the number of used sub-channels, i.e. Lsub-CH is indicated by MAC layer after estimating the amount of data size, it’s unreasonable to re-indicate a different number of sub-channel for the same TB from MAC layer. |
| Nokia/NSB2 | Agree |  |

#### [M] Proposal 3-2

Background: IRB, remaining details on mapping between sub-channel and interlace

- Summary of previous round

  - Support (23): DCM, LGE, Nokia, OPPO, InterDigital, Apple, NEC, Fraunhofer, Qualcomm, Samsung, Sharp, Intel, Futurewei, Lenovo, Spreadtrum, Sony, CMCC, Xiaomi, CATT, Panasonic, MediaTek, ZTE, Huawei/HiSilicon,

    - Support Option 1 (4 3): DCM, vivo, CMCC, CATT,

    - Support Option 2 (13 14): LGE, Nokia, Qualcomm, Sharp, Intel, Lenovo, Spreadtrum, CMCC, Xiaomi, Panasonic, MediaTek, ZTE, WILUS, DCM

- Updates on proposal (marked in red) and justifications

  - Based on the situation, FL suggests to go with Option 2.

  - FL made the following updates based on comments from vivo, Sharp.

    - “sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.”

    - “Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0”

- FL’s response to some companies’ comments:

  - @LGE: thanks for the comment. I may organize discussions on that point later (give me some time…)

  - @MediaTek: FL assumes the current proposal is clear as shown in the Figure and no need for the Note. Each RP knows how to do mapping as per the proposal.

Figure 1 Illustration of mapping between sub-channel and interlace (Based on Figure from R1-2303368)

[M] Proposal 3-2b

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, down-select one of support the following

  - Option 1: sub-channel#0 is mapped to K interlace(s) starting from interlace with lowest frequency location in the resource pool

  - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

- Interlace is indexed as per NR-U

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes |  |
| Apple | Yes | We are also fine with Option 1, since the two resource pools could be independently operated. |
| InterDigital | Yes |  |
| Intel | Yes |  |
| Fraunhofer | Yes |  |
| vivo | OK | We slightly prefer Option 1 but are OK to go with Option 2 following the majority. |
| DCM | Yes | (We modified our company’s position in the above summary of the 1st round) |
| LGE | OK |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| NEC | Yes |  |
| Samsung | Yes |  |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| MediaTek | Yes |  |
| Lenovo | Yes |  |
| Sharp | Yes |  |
| WILUS | Yes |  |
| xiaomi | Yes |  |
| ZTE, Sanechips | Agree in principle | For Option 2, as it is still under discussion whether a subset of interlaces can be configured (e..g interlace 3 ~9 instead of interlace 0~9) for a resource pool, we propose to change it more general: - Option 2: sub-channel#0 is mapped to K interlace(s) starting from the lowest interlace#0 index (pre)configured for the resource pool |
| Ericsson | Yes |  |
| Transsion | Yes |  |
| CATT/GOHIGH | Yes | We can be flexible to option 2. |
| Huawei/HiSilicon | Yes |  |
| Nokia/NSB2 | Agree |  |
| Futurewei | OK |  |
| OPPO2 | Yes with comment | Sorry for updating this comment, but we just realize the wording in option 2 is not quite clear. As ZTE mentioned above, it is still open about FDMed resource pools in one RB set, which is to say that partial RB set is configured to a resource pool. With the current wording in option 2, if the sub-channel#0 is always from interlace#0, another FDMed resource pool starting from interlace rather than #0 (e.g. interlace#3) cannot be mapped correctly. We think the proposed update by ZTE is acceptable which is more general. |

#### [M] Proposal 3-3

Background: IRB, TBS determination

- Summary of previous round

  - Support (20): LGE, Nokia, OPPO, Apple, NEC, Ericsson, Qualcomm, Sharp, Intel, Futurewei, Spreadtrum, Sony, CMCC, Xiaomi, CATT, Panasonic, MediaTek, ZTE, Huawei/HiSiliconm, WILUS

    - Option 1: N_ref is a fixed value, e.g., 10, 11

      - Support (7): LGE, Ericsson, Qualcomm, Intel, CATT, Panasonic, WILUS

    - Option 2: N_ref is (pre-)defined

      - Support (3): Nokia, NEC, CATT,

    - Option 3: N_ref is (pre-)configured

      - Support (5): Nokia, Apple, Spreadtrum, CMCC, Xiaomi,

    - Option 4: N_ref is dynamically indicated by Tx UE

      - Support (3): Nokia, NEC, Huawei/HiSilicon,

  - Not Support (1): vivo,

- Updates on proposal (marked in red) and justifications

  - FL adds Option A and Option B below.

    - FL assumes Option B reflects vivo’s and Lenovo’s thinking.

    - @vivo, Lenovo, please check whether Option B is your thinking. If not, please suggests some wording changes, thanks.

  - Add an example in Option A2 as per OPPO’s comment.

  - Add an FFS under Option A based on ZTE’s comment.

[M] Proposal 3-3b

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), down-select one of the followings:

- Option A: TBS of initial transmission and retransmission(s) are guaranteed to be the same

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - FFS: whether additional solution is needed when the PSSCH transmission occupies multiple RB sets

- Option B: TBS of initial transmission and retransmission(s) are not guaranteed to be the same

  - TBS for each initial and retransmission(s) is individually determined based on the actual number of occupied PRBs for that transmission

  - Note: the determined TBS might be different so that HARQ combining is not possible

FL’s comment: please also indicate which Option you support.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes | We prefer Option A and Option A3; the reason is different TBS for initial transmission and retransmission may cause more complexity for implementation. Option A3 can provide more flexibility than other options, the case is similar to TBS determination when we introduce the 2nd starting symbol. |
| Apple | Yes | We prefer Option A and Option A3. It has the flexibility to cover Option A1 and A2. |
| Interdigital | Yes | We support Option A and A1. |
| Intel | Yes | We support Option A and A1. |
| vivo | Yes | We prefer Option B as it don’t have to introduce new behavior and spec changes. Comments on Option A: even for option A, the TBS is not always guaranteed to be the same. Noted that even in Rel-16 TBS is not always guaranteed to be the same. My understanding is that Opt Ax can only ensure the same number of allocated REs for TBS determination. |
| DCM |  | As commented in the last round, we are not sure whether such a situation occurs. If not, using the number of actual PRBs is sufficient. This point should be clarified first. To clarify this, we suggest adding one another option under option A. - Option A5: N_ref is the actual number of occupied PRBs for that transmission except for PRBs within intra-cell guard band, if any. - Note: it is assumed that the number of PRBs within each sub-channel is the same. If this note is infeasible, we support Option A1. |
| LGE | OK for progress | We think that Option A is definitely basic assumption in NR. For the value of N_ref, we prefer to have integer number, and the number need to be aligned with the subchannel size of Rel-16/17 NR SL to align the set of supportive TB size considering UE implementation or complexity. |
| Qualcomm | Yes | We prefer N_ref being a fixed value (option A1) and the value can be chosen to be the minimum number of interlaced RBs in different interlaced subchannels from all the RB-set(s) |
| OPPO | Yes | We prefer Option A and Option A2. |
| NEC | Yes Option A | We think only option A goes with the direction of R16 NR SL principle, as per following agreement we made for NR SL: Agreement@RAN1#100 - It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signaled for this TB. - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s). |
| Samsung | Yes | Prefer Option A and A3. In addition, how to configure option A3 needs to be clarified, e.g. whether it’s RP level configuration or UE specific configuration. |
| Panasonic | Yes | We support Option A and A1. |
| Spreadtrum | Yes | We prefer Option A and Option A3. |
| MTK | OK | Both Option A1 and Option A3 are OK from our side. |
| Lenovo | Yes | Option B is preferred. |
| Sharp | Yes | We prefer Option A and A2. |
| WILUS | Yes | We support Option A and A1. |
| xiaomi | Yes |  |
| ZTE, Sanechips | Agree | The main bullet of Option A is aligned with our understanding, to make the FFS more clear, we proposed to updated as: - FFS: for TBS determination, whether/how to handle the impact of the additional available of PRB(s) in guard band(s) for PSCCH/PSSCH transmission across multiple RB sets |
| Ericsson |  | We are confused by this proposal. What does it mean that the TBS of an initial transmission and retransmission (of the same TB, we presume) are not the same. |
| Transsion | Yes | We prefer Option A and A1. |
| CATT/GOHIGH | Yes | We prefer Option A, either Option A1 or Option A2 will be OK. |
| Huawei/HiSilicon | OK for progress | Support Option A. Option A follows legacy NR SL (see R16 NR V2X agreement below). Option B is not reusing legacy NR SL. In option B, the TBS of initial transmission and retransmission(s) may be different, thus UE cannot do HARQ combination. This is a serious problem. == Agreements (RAN1#100): For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations. - … - It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB. - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s). |
| Nokia/NSB2 | Agree | To our view, the Option A1, A2, A3 may result with the same with minor differences. We could down-select among Option A1, A2, A3. Not sure if we should have the newly added Option B. It is not very clear for us. |

#### [M] Proposal 3-4

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary of previous round

  - Support (13): OPPO, Apple, NEC, Ericsson, Samsung, Intel, ETRI, Sony, Xiaomi, Panasonic, ZTE, Huawei/HiSilicon, WILUS

  - Not Support (4): vivo, Nokia, Interdigital, Spreadtrum,

- Updates on proposal (marked in red) and justifications

  - FL updated the proposal to be clearer.

Figure 2 Illustration of sub-channel(s) including intra-cell guardband PRBs (Based on Figure from R1-2302550)

[M] Proposal 3-4b

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings:

- Option 1: Such sub-channel(s) cannot be used for PSCCH/PSSCH transmission

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

FL’s comment: please also indicate which Option you support.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes | Option 2 For example, in the above figure, when LsubCH=2, Sub-channel 3 + Sub-channel 4 can be a candidate resource, if this candidate resource is selected, multi-channel access procedure is required. For PSCCH transmission, if it only occupied partial sub-channel, e.g., the yellow part of sub-channel 3, it can be transmitted. |
| Apple | Yes | If Option 1 is used, then we do not see the motivation of defining such sub-channels. Regarding Option 2 and Option 3, it may depend on the conditions. For example, if sub-channel #3 in the figure has yellow part larger than or equal to the configured number of PRBs for PSCCH, then it can be used for PSCCH/PSSCH transmission. Otherwise, it cannot be used for PSCCH transmission, but can be used for PSSCH transmission. To allow sub-channel #4 in the figure for PSCCH/PSSCH transmission, we need to check whether PSCCH is always located at the lowest PRBs of a sub-channel, or it can be adjusted to “the lowest PRBs of a sub-channel not belonging to guard band”. Overall, we support both Option 2 and Option 3, and think the selection between Option 2 and Option 3 depends on conditions/configurations. If only Option 2 is supported, it is likely violating the previous agreement mentioned in our first-round comment. |
| Interdigital | Yes | We support Option 1 similar to R16. |
| Intel | Yes | We support Option option 2. |
| vivo | comment | Isn’t only option 3 complying to the following agreement? Maybe option 2 may still be possible but then some restrictions are needed (e.g., excluding the GB PRB for PSCCH). Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission |
| LGE | Yes | In our understanding, once we agree to use Rel-16/17 NR SL contiguous RB-based structure, it would be necessary to maximize the usable sub-channels as much as possible. Moreover, we should avoid non-contiguous transmission for PSSCH. My another question is whether we will support standalone transmission with a single sub-channel including guard PRBs even if the PSCCH resources within in the sub-channel is not overlapping with the guard PRBs. |
| Qualcomm | Yes | Option 3 is preferred. As Vivo pointed out that the previous agreement excludes PSCCH from the guard PRB, option 3 is the simplest solution to avoid PSCCH irregularity. |
| OPPO | Comment | We share similar view as vivo. According to the agreement (cited by vivo), at least option 1 conflict with the agreement and should be removed. Furthermore, the 3 options in current proposal does not help to provide clear direction for further discussion or down-selection. We can focus on the conditions on which the subchannels who include PRBs of GB cannot be used. Therefore, we would like to suggest to remove the 3 options. [M] Proposal 3-4b For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings: - Option 1: Such sub-channel(s) cannot be used for PSCCH/PSSCH transmission - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission - FFS details, e.g., conditions to apply the above Option(s) or not apply these sub-channels - FFS impacts on definition of candidate resource, and resource selection |
| NEC | Yes | Option 3. |
| Samsung | Yes | We prefer Option 3 for better resource efficiency. |
| Panasonic | Yes | We support option 3. |
| Spreadtrum | Yes | Prefer option 1. |
| MediaTek | Yes | Both Option 1 and Option 3 are OK. |
| Lenovo | Comment | We prefer to deprioritize this issue. |
| Sharp | Comment | In addition to the RAN1 agreement cited by vivo, we think the following RAN1 agreement should also be taken into account, Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set. Therefore, for option 2, it depends on whether such a sub-channel is “the lowest sub-channel of lowest RB set” of the PSCCH/PSSCH transmission. We are fine with option 2 with some restrictions, or option 3 for simplicity. |
| WILUS | Yes | We prefer Option 3 |
| xiaomi | Yes | We perfer option2, option2 has benefit on the resource utilization. |
| ZTE, Sanechips | Agree | We think all these options are possible. |
| Ericsson | Comment | We could be fine with Option 2 or Option 3 (for simplicity). But Option 1 is the same as saying that the intra-cell guardbands PRBs cannot be used at all. We do not prefer option 1 as it not effective. |
| Transsion | Yes | We prefer Option 3 |
| CATT/GOHIGH | Comment | Firstly, one clarification is needed, “such sub-channel(s) can be used….” refers to all the PRBs of such sub-channel can be used or all the remaining PRBs excluding guard band PRBs can be used? For example, Sub-channel 2 + Sub-channel 3 can be used for PSCCH/PSSCH transmission or Sub-channel 2 + Remaining PRB (yellow part) of Sub-channel 3 can be used for PSCCH/PSSCH transmission? We support option 1 and option 2 depend on the conditions. - If only one sub-channel is needed for PSCCH/PSSCH transmission, both sub-channel 3 and sub-channel 4 cannot be used. - If more than one sub-channel is needed for PSCCH/PSSCH transmission, the sub-channel combination including both sub-channel 3 and sub-channel 4 can be used for PSCCH/PSSCH transmission. |
| Huawei/HiSilicon | Yes | We support Option 2, 3. We share similar view with Apple. Option 2 and 3 can still satisfy previous agreement, i.e., guardband PRB are not used for PSCCH transmission. |
| ETRI | Yes (comment) | We support option 2. But, according to the previous agreement mentioned by vivo, PRBs belonging to GBs in such subchannels cannot be used for PSCCH transmission and can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission. |
| Nokia/NSB2 | Not agree | About the remaining PRBs, there can be “remaining PRBs” after the mapping either due to the total number of PRBs in the RP that cannot be divided by the configured subchannel size or due to the PRBs of subchannel are overlapped with the configured Guard Band PRBs between RB sets. The figure quoted by the FL from R1-2302550 only show part of the picture. As shown below figure in our Tdoc, the Subchannel#7 can be also as remaining PRBs. The applied solution should be applied for both “remaining PRBs” issues. If the remaining PRBs is 1 or 2 PRBs less than the number of PRBs of subchannel size, then it is worthwhile can be treated as a subchannel for better spectrum efficiency and utilized for PSCCH/PSSCH transmission. |

#### [L] Proposal 3-5

Background: IRB or CRB

- Summary of previous round

  - Support (24): vivo, Nokia/Nsb, Interdigital, Apple, NEC, Ericsson, Fraunhofer, Qualcomm, Samsung, Sharp, Intel, Futurewei, Lenovo, ETRI, Spreadtrum, Sony, CMCC, xiaomi, CATT/GOHIGH, Panasonic, MediaTek, ZTE, Huawei/HiSilicon, WILUS

- Updates on proposal (marked in red) and justifications

  - FL assumes this proposal is rather stable, hope we can endorse it by email.

- FL’s response to some companies’ comments:

  - @Domoco: RAN1 already agreed to support contiguous RB, hope you can be flexible.

  - @LGE: FL assumes resource pool level granularity is flexible enough to achieve what you mentioned. FL suggests not spending too much time on this proposal.

[L] Proposal 3-5

A resource pool is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |
| Intel | Yes |  |
| Fraunhofer | OK |  |
| vivo | Yes |  |
| LGE |  | In our understanding, regulation itself would be carrier-specific, but not resource pool-specific. When we agree this proposal, will we allow the case when a resource pool enables contiguous RB-based PSCCH/PSSCH while another resource pool enables interlace RB-based PSCCH/PSSCH for the same carrier? For compromise, we can accept this if the following sentence is added. In a SL carrier/SL BWP, all the resource pools is commonly (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| NEC | YES |  |
| Samsung | Yes |  |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| MediaTek | Yes |  |
| Lenovo | Yes |  |
| Sharp | Yes |  |
| WILUS | Yes |  |
| xiaomi | Yes |  |
| ZTE, Sanechips |  | We tend to agree with LG that such (pre-)configuration should be SL BWP level. |
| Ericsson | Yes |  |
| Transsion | Yes |  |
| CATT/GOHIGH | Yes |  |
| Huawei/HiSilicon | Yes |  |
| Nokia/NSB2 | Agree |  |

### [Closed] Proposals for Thursday GTW (Week 1)

#### [H] Proposal 3-1

Background: IRB, frequency domain resource indication

- Summary of previous round

  - Support (23): vivo, Nokia, OPPO, InterDigital, Apple, Fraunhofer, Qualcomm, Samsung, Intel,  Spreadtrum, CMCC, Xiaomi, CATT, Panasonic, Transsion, ZTE, Huawei/HiSilicon, WILUS, Docomo, LGE, NEC, MediaTek, Lenovo,

  - Not Support ():

- Updates on proposal (marked in red) and justifications

  - Add bracket on the 1st FFS point since multiple companies prefer to remove it.

  - Made updates “…and/or…” based on Docomo’s comment.

  - Add the following based on comment from CATT.

    - “and update resource selection and/or signaling from MAC to PHY”

- FL’s response to some companies’ comments:

  - @Apple: I do not get your point, not sure what’s the issue with current Option A.

  - @LGE: in legacy NR SL, even the chain is broken, the resources of retransmission are still from S_A (i.e., a single S_A as in resource selection procedure), so all the resources have the same sub-channel size.

[H] Proposal 3-1c

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s)

    - [ FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case ]

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

#### [M] Proposal 3-2

Background: IRB, remaining details on mapping between sub-channel and interlace

- Summary of previous round

  - Almost all companies are ok with it.

- Updates on proposal (marked in red) and justifications

  - No updates are made.

- FL’s response to some companies’ comments:

  - @ZTE: so far, we have no agreement to support that case, and I feel probably that will not be supported based on my Tdoc review.

Figure 1 Illustration of mapping between sub-channel and interlace (Based on Figure from R1-2303368)

[M] Proposal 3-2b

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

- Interlace is indexed as per NR-U

#### [M] Proposal 3-3

Background: IRB, TBS determination

- Summary of previous round

  - Support Option A (18): LGE, Nokia, OPPO, Apple, NEC,  Qualcomm, Sharp, Intel,  Spreadtrum, CMCC, CATT, Panasonic, MediaTek, ZTE, Huawei/HiSiliconm, WILUS, InterDigital, Samsung,

  - Support Option B (2): vivo, Lenovo,

- Updates on proposal (marked in red) and justifications

  - As pointed out by NEC and Huawei/HiSilicon, Option B is not aligned with legacy NR SL design (see RAN1#100 agreement below).

  - Based on the situation, FL suggests to go with Option A. We can do the down-selection of A1~A4 later.

  - Add Option A5 with bracket based on comment from Docomo. Companies can further check whether this is ok.

  - Replaced the FFS based on ZTE’s comment.

Agreements (RAN1#100):

For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations.

- …

- It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

  - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s).

[M] Proposal 3-3c

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), down-select one of support the followings:

- Option A: TBS of initial transmission and retransmission(s) are guaranteed to be the same

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - [ Option A5: N_ref is the actual number of occupied PRBs for that transmission except for PRBs within intra-cell guard band, if any. ]

      - Note: it is assumed that the number of PRBs within each sub-channel is the same.

    - FFS: whether additional solution is needed when the PSSCH transmission occupies multiple RB sets

    - FFS: for TBS determination, whether/how to handle the impact of additional available of PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

- Option B: TBS of initial transmission and retransmission(s) are not guaranteed to be the same

  - TBS for each initial and retransmission(s) is individually determined based on the actual number of occupied PRBs for that transmission

  - Note: the determined TBS might be different so that HARQ combining is not possible

#### [M] Proposal 3-4

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary of previous round

  - Support (18): CMCC, Apple, IDC, Intel, LGE, QC, NEC, Samsung, Pana, Spredtrum, MTK, WILUS, Xiaomi, ZTE, Ericsson, Transsion, HW, ETRI,

    - Support Option 1 (5): IDC, SPRD, MTK, ZTE, CATT,

    - Support Option 2 (9): CMCC, Apple, Intel, Sharp, Xiaomi, ZTE, CATT, HW, ETRI

    - Support Option 3 (10): Apple, QC, NEC, SS, Pana, MTK, Sharp, WILUS, ZTE, HW

  - Not Support (2): Nokia, [Lenovo]

- Updates on proposal (marked in red) and justifications

  - So far, no changes are made.

- FL’s response to some companies’ comments:

  - @all: previous agreement will be obeyed for sure, companies should give detailed designs for Option 2 later while satisfying previous agreement.

  - @CATT: this is subject our previous agreement on usage of intra-cell guardband (see below).

  - @Nokia: the issue you mentioned is another issue, i.e., “FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size”. We can organize related discussions later if needed. Let’s not couple issues together.

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Figure 2 Illustration of sub-channel(s) including intra-cell guardband PRBs (Based on Figure from R1-2302550)

[M] Proposal 3-4b

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings:

- Option 1: Such sub-channel(s) cannot be used for PSCCH/PSSCH transmission

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

#### [L] Proposal 3-5

Background: IRB or CRB

- Summary of previous round

  - Almost all companies are fine with it.

- Updates on proposal (marked in red) and justifications

  - Changed to “SL-BWP” level as per LGE and ZTE comment. This might be simpler.

[L] Proposal 3-5b

A resource pool SL-BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

### [Closed] Round 3: Proposals and summary of previous round

#### [H] Proposal 3-1

Background: IRB, frequency domain resource indication

- Summary of previous round

  - Support (23): vivo, Nokia, OPPO, InterDigital, Apple, Fraunhofer, Qualcomm, Samsung, Intel,  Spreadtrum, CMCC, Xiaomi, CATT, Panasonic, Transsion, ZTE, Huawei/HiSilicon, WILUS, Docomo, LGE, NEC, MediaTek, Lenovo,

  - Not Support ():

- Updates on proposal (marked in red) and justifications

  - Made the following update as per Apple’s comment. @Apple, please confirm whether this is your suggestion.

    - “Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same”

  - Add bracket on the 1st FFS point since multiple companies prefer to remove it. Companies can indicate whether you want to remove the FFS.

  - Made updates “…and/or…” based on Docomo’s comment.

  - Add the following based on comment from CATT.

    - “and update resource selection and/or signaling from MAC to PHY”

- FL’s response to some companies’ comments:

  - @LGE: in legacy NR SL, even the chain is broken, the resources of retransmission are still from S_A (i.e., a single S_A as in resource selection procedure), so all the resources have the same sub-channel size.

    - @All: please also share your view on this part. Let’s try to save online time. We really need progress.

[H] Proposal 3-1d

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s)

    - [ FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case ]

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

FL’s comment: please indicate whether you want to keep the 1st FFS or not.

FL’s comment: please share your view on LGE’s comment (see above).

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We would suggest removing the 1st FFS to minimize the spec work since the legacy FRIV cannot be reused for the future reservation if the initial and retransmission choose different number of RB-sets. To address LGE’s concern, we may put the same subchannel(s)/RB-set(s) restriction on FRIV signaling only. Based on Tx UE implementation, if the retransmission is not exploiting the previously SCI reserved resources for retransmission, the Tx UE may freely choose different number of RB-set but the number of interlaced subchannel needs to be adjusted so that the TBS size is the same. |
| Apple | Yes | We are fine with the addition on Option A. Thanks. Also, we think the first FFS could be removed since otherwise, the signaling scheme will be redesigned and signaling overhead will be large. Regarding LGE’s concern on chain broken, a Tx UE may re-select resources with the same total number of sub-channels across different RB sets. For simplicity, we are fine to restrict the re-selection such that the same number of sub-channels and same number of RB sets are used. |
| Intel | Yes | Also agree with other companies that the FFS could be removed. |
| LGE | OK with some modification | First of all, thanks for the feedback. On 2nd bullet of Option 1, I tried to check the existing spec again. First of all, according to TS38.321, at least for Mode 1, I cannot find such a restriction. In my understanding, it is up to gNB decision whether or not to schedule the same or different number of sub-channel across different DCIs. (For a single DCI, all the same number of subchannels are allocated to up to 3 SL resources) Moreover, even for Mode 2, I found that the following text in TS 38.321: 1> if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or NOTE 1: If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection. 1> if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU: NOTE 2: If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection. NOTE 3: It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CE triggered according to clause 5.22.1.7. 2> clear the selected sidelink grant associated to the Sidelink process, if available; 2> trigger the TX resource (re-)selection.. When the TX resource reselection is triggered, depending on CBR, it seems that it is allowed to change the number of subchannels according to TS 38.321. 3> select an amount of frequency resources within the range, if configured by RRC, between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;. In short, at least for Mode 1 and resource reselection in Mode 2, it seems that companies may have different understanding on the 2nd bullet of the option 1 is the existing restriction or not. To resolve this issue, I just suggest to add “indicated by the SCI” or “associated with the same SCI” after “initial transmission and retransmission” for clarification. We are also OK to remover FFS of the 2nd subbullet of Option 1. I have some clarification question on some FFS parts. On bitmap design, how to indicate the reserved resources? As we know, the main difference of FRIV from RIV is that it can indicate current allocation (with PSCCH location) and reserved resource in the future. In our understanding, indicating reserved resource via bitmap would be not easy or inefficient in terms of payload size. On whether or not to redefine single-slot candidate resource, what happened if we agree that the subchannel consists of interlaces with contiguous indexes. To maximize the TX power under the PSD limit, re-define single-slot candidate resources may not sufficient. Moreover, even for this case, it is necessary to clarify how to indicate reserved resources. |
| Futurewei | OK |  |
| DCM | Yes | If the first FFS brings concerns, we suggest removing a part of the FFS. At least the number of sub-channels should be the same as in legacy SL. Meanwhile, whether only the same number of RB-sets is allowed or not is completely new issue. Further discussion is necessary. [H] Proposal 3-1d Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any - For a TB, the initial transmission and retransmissions use the same number of sub-channel(s) and same number of RB set(s) - [ FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case ] - Use X bits for indicating sub-channel index(s), and use Y bits for indicating RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, etc. - FFS others - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc. |
| ZTE, Sanechips | Yes | For LG’s comments, we think it is possible that a different RB sets number can be used for TB’s re-transmissions if resource reselection is triggered, e.g., due to LBT failure. To reuse FRIV indication, we agree with Qualcomm that the same subchannel(s) number and the same RB-set(s) number restriction on FRIV signaling is needed. |
| vivo | Yes | We prefer to remove the 1st FFS, but can live with it if it helps to make progress. Our understanding is that in R16 the at least the resources reserved by the same SCI have the same number of sub-channels. If MAC layer for some reason trigger resource reselection, it seems no guarantee that same number of sub-channels are used. Reselecting the resources with a lower MCS for retransmission is discussed in R16 and is not precluded. |
| OPPO | Yes with comment | We share the similar view with Qualcomm that the whole [FFS: …] should be removed if legacy SL FRIV indication is expected to be reused. Furthermore, if different number of sub-channels and/or different number of RB sets are used, it leads to a more complicated indication method which has a higher indication overhead. |
| CATT/GOHIGH | Yes | The 1st FFS should be removed considering the feasibility of reusing legacy FRIV. And for simplify, we prefer to use the same number of RB set(s) and sub-channel(s) for one TB. |
| Transsion | Yes | Share the same view as Qualcomm that if we intent to reuse the function of SL FRIV, the first FFS should be removed. |
| Spreadtrum | Yes | We are fine with the proposal. |
| Lenovo | Yes with comments | We agree with other companies to remove the whole bullet of the 1st FFS. Regarding allocated RB sets, it is worth noting that only contiguous RB sets can be supported. This should be captured in the proposal since it impacts on signaling design details. In LTE eLAA and NR-U, spec supports predefined non-contiguous interlace combinations, e.g., interlace 0+5, interlace 1+6, …, when more than one interlaces are assigned so as to avoid PSD limitation of contiguous interlace allocation. We think this principle should be reused for SL-U; otherwise, PSD limitation happens for contiguous interlace allocation. Suggestions from our side: … - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, predefined non-contiguous interlace combinations, etc. … |
| WILUS | Yes | We support this proposal. We are also fine to remove the 1st FFS bullet. |
| Sharp |  | On the 3rd bullet of Option 1, we see no response from FL on our comments. As indicated by other companies, it is unclear how a bitmap design works with FRIV. We propose to add “Rel-16 NR-U interlace indication” to the first sub-bullet, i.e., “R16 NR-U interlace indication and R16 NR SL FRIV are reused as baseline”. |
| xiaomi | Yes with comment | we think the the 1st FFS shall not be supported, we share the similar view with LG. If the initial transmission and retransmissions use the different number of sub-channel(s) and different number of RB set, the TB size might be different for the initial transmission and retransmissions ,this will has larger specification impact. |
| CMCC | Yes | We also think the FFS should be removed. |
| Panasonic | Yes | We are OK for first FFS modified by DCM. When the total number of subchannels in multiple RB sets is same, different number of RB set(s) can be acceptable for retransmission. |
| MediaTek | Yes | For the 1st FFS, our preference is to remove it, or at least the sub-channel number should be same for the initiation transmission and retransmission of a TB. |
| Huawei/HiSilicon | Yes | We think the first FFS should be removed since it’s not aligned with legacy FRIV design. Otherwise, the signaling scheme will be re-designed and spec work will be large. |
| Nokia/NSB | OK | Also OK to remove the 1st FFS |

#### [M] Proposal 3-2

Background: IRB, remaining details on mapping between sub-channel and interlace

- Summary of previous round

  - Almost all companies are ok with it.

- Updates on proposal (marked in red) and justifications

  - Made some updates based on comment from ZTE and OPPO2 (see their reply above).

    - ZTE and OPPO2 are mainly talking about the FFS in previous agreement (see below).

    - If the FFS is supported, then a resource pool might be (pre-)configured with interlace#1 only (i.e., sub-set PRBs of one RB set), then sub-channel#0 in this resource pool should be mapped to interlace#1.

    - If the FFS is not supported, i.e., a resource pool always include integer number of RB sets as agreed, then sub-channel#0 is mapped to interlace#0.

    - So the updated Option 2 is safer, i.e., if the FFS is supported (unknown now), we do not need to revise the agreement or defining a new mapping.

  - Initially, FL does not want to do such updates since this proposal is almost stable after two rounds. But if such updates can resolve ZTE’s and OPPO2’s concern and help endorsing it by email, FL would like to give it a try.

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - …

Figure 1 Illustration of mapping between sub-channel and interlace (Based on Figure from R1-2303368)

[M] Proposal 3-2c

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0 with smallest index in the resource pool

    - Note: for the agreed case that one SL resource pool can be (pre-)configured to include integer number of RB sets, the interlace with smallest index in the resource pool is interlace#0

- Interlace is indexed as per NR-U

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| LGE | OK | Can we clarify that the lowest subchannel is sub-channel#0 as note? (the lowest sub-channel is not sub-channel located in the lowest frequency) |
| Futurewei | OK |  |
| DCM | OK | Maybe, additional text is necessary like: sub-channel#1 is mapped to …, and so on. |
| ZTE, Sanechips | Yes |  |
| vivo | comment | - Firstly, the proposal is not complete. For the sub-channel mapping of Option 2, it actually only defines the mapping for sub-channel #0, but leaves empty for the others. The intention seems to be something like this: - sub-channel#0 is mapped to K interlace(s) starting from interlace#0, sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on - Secondly, the updated text, seems to intend to open the door for the case that the smallest interlace index in a pool is not zero, or in other words, a resource pool only covering partial RB set. Although I don’t see good motivation for this right now, I am open to this given that it has not been discussed before. But to serve this purpose, the meaning of ‘smallest interlace index in a resource pool’ is not clear and should be clarified for better understanding. - According to the existing procedure, the resource pool is defined by the lowest sub-channel. Thus, the sub-channel#0 should be determined first. The original option-2 is clear because interlace#0 is clearly defined. The option-1 is also clear because the lowest frequency location is clear (by RRC configuration). However, the updated option-2 is not clear because the ‘smallest interlace index in a resource pool’ to determine the sub-channel#0 depends on the resource pool itself, which can only be determined after the sub-channel#0 is defined! This creases a chicken-egg problem. |
| OPPO | Yes with comment | As we commented during round 2, whether “resource pool can include subset of PRBs of one RB set” is FFS, we would like to suggest that the mapping should consider this case, at least leave it open. Furthermore, during email discussion, some more detailed wording was proposed to determine the following sub-channels’ mapping procedure after sub-channel#0. Regarding the comment from vivo about the ambiguous part, we think the “in the resource pool” can be removed to make it clearer (yellow highlighted deleted). We would like to suggest the following update: [M] Proposal 3-2c For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets. - In a resource pool, support the following - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace #a, sub-channel #1 is mapped to K interlace(s) starting from interlace #(a+K), where a (a>=0) is the #0 with smallest interlace index in the resource pool. - Note: for the agreed case that one SL resource pool can be (pre-)configured to include integer number of RB sets, a=0. the interlace with smallest index in the resource pool is interlace#0 - Interlace is indexed as per NR-U |
| CATT/GOHIGH | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Lenovo | Yes |  |
| WILUS | Yes | We support this proposal. |
| Sharp |  | OK with OPPO’s comments. |
| xiaomi | Yes with comment | As the email discussion, we share the similar view with OPPO, and we support the updated proposal by OPPO. |
| CMCC | OK |  |
| Panasonic | Yes |  |
| MediaTek |  | Our preference is not to support the case that one SL resource pool can include sub-set of PRBs of one RB set. At current stage, we prefer to go back to the previous proposal 3-2b and just add an FFS to state the above case. [M] Proposal 3-2b For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets. - In a resource pool, support the following - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0 - FFS the mapping rule when one SL resource pool can include sub-set of PRBs of one RB set. Interlace is indexed as per NR-U |
| Huawei/HiSilicon | Yes |  |
| Nokia/NSB | OK |  |

#### [M] Proposal 3-3

Background: IRB, TBS determination

- Summary of previous round

  - Support Option A (18): LGE, Nokia, OPPO, Apple, NEC,  Qualcomm, Sharp, Intel,  Spreadtrum, CMCC, CATT, Panasonic, MediaTek, ZTE, Huawei/HiSiliconm, WILUS, InterDigital, Samsung,

  - Support Option B (2): vivo, Lenovo,

- Updates on proposal (marked in red) and justifications

  - As pointed out by NEC and Huawei/HiSilicon, Option B is not aligned with legacy NR SL design (see RAN1#100 agreement below).

  - Based on the situation, FL suggests to go with Option A. We can do the down-selection of A1~A4 in next meeting.

  - Add Option A5 with bracket based on comment from Docomo. Companies can further check whether this is ok.

  - Replaced the FFS based on ZTE’s comment.

Agreements (RAN1#100):

For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations.

- …

- It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

  - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s).

[M] Proposal 3-3d

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), down-select one of support the followings:

- Option A: TBS of initial transmission and retransmission(s) are guaranteed to be the same

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - [ Option A5: N_ref is the actual number of occupied PRBs for that transmission except for PRBs within intra-cell guard band, if any. ]

      - Note: it is assumed that the number of PRBs within each sub-channel is the same.

    - FFS: whether additional solution is needed when the PSSCH transmission occupies multiple RB sets

    - FFS: for TBS determination, whether/how to handle the impact of additional available of PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

- Option B: TBS of initial transmission and retransmission(s) are not guaranteed to be the same

  - TBS for each initial and retransmission(s) is individually determined based on the actual number of occupied PRBs for that transmission

  - Note: the determined TBS might be different so that HARQ combining is not possible

FL’s comment: please share your view on Option A5, e.g., whether it is reasonable to be kept.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm |  | For option A5, it is not clear how we can guarantee same number of RBs for all subchannels due to the presence of guardband. Are we dropping one RBs of the other 10-RB interlaces if one interlace has only 9 RBs? If so, we are worried about the overhead. |
| Apple |  | We do not think Option A5 can solve the TBS calculation issue without the assumption in the Note. It is not a complete solution. We still think Option A3 is more flexible and covers Option A1 and A2. Also, Option A3 does not increase SCI payload, comparing with Option A4. |
| Intel | Yes | Option A1 is preferred. |
| LGE | OK | Considering UE complexity, it is preferred to use TBS set used in Rel-16/17 NR SL as much as possible. The simple way for this is N_ref=10 since the Rel-16/17 NR SL supports subchannel size of 10 PRBs. Regarding Option A5, the existing quantization procedure in TBS determination may or may not work properly. At this moment, we do not need to spend more time for removing it. |
| Futurewei | OK | We also think that Option 5 is not clear as QC pointed out |
| ZTE, Sanechips |  | Option A5 is not clear to us either, for example, if the initial transmission occupies two interlaces with 11+11PRBs, and a re-transmission with two interlaces with 10+10 PRBs, how to guarantee TBS of initial transmission and retransmission(s) to be the same if the actual number of occupied PRBs is used for TBS determination? |
| vivo | OK | Although we believe no additional spec change is needed (i.e., option B) as this is not an issue, we can accept the proposal for progress. Regarding the A5, we are OK to keep it. If A5 is applied, ensuring the same number PRB at least can be achieved by proper resource allocation. |
| OPPO |  | We also share the same view with Qualcomm on how to guarantee the PRB number in each sub-channel same. Option A5 may require additional designs/restrictions, which may introduce extra complexity. |
| CATT/GOHIGH |  | Regarding option A5, some ambiguity is existed. One example is given as follows: The initial transmission occupies sub-channel(s) with guard band PRBs (such as 8 PRBs) and re-transmission occupies sub-channel(s) without guard band PRB (such as 11 PRBs), both of them (8 PRBs and 11 PRBs) are the actual number of occupied PRBs for transmissions, then which one will be used for TBS determination? |
| Transsion | OK | Option A1 is preferred. |
| Spreadtrum |  | We also have concern on option A5. How to guarantee the PRB number in each sub-channel same is unclear. Some clarification is needed. |
| DCM | Yes | Our intention of Alt 5 is, at first it should be clarified when the number of PRBs for each sub-channel is different. RB-set design is defined in table 5.3.3-2 of 38.101. Here, 100 MHz bandwidth with 30 kHz SCS may lead to such a case. Is it correct understanding? If correct, and if it is clarified that R18 SL-U supports different number of RBs per sub-channel, we are fine with the direction. [M] Proposal 3-3d For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), down-select one of support the followings: - Option A: TBS of initial transmission and retransmission(s) are guaranteed to be the same - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113: - Option A1: N_ref is a fixed value, e.g., 10, 11 - Option A2: N_ref is (pre-)defined - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces. - Option A3: N_ref is (pre-)configured - Option A4: N_ref is dynamically indicated by Tx UE - [ Option A5: N_ref is the actual number of occupied PRBs for that transmission except for PRBs within intra-cell guard band, if any. ] - Note: it is assumed that the number of PRBs within each sub-channel is the same. - FFS: whether additional solution is needed when the PSSCH transmission occupies multiple RB sets - FFS: for TBS determination, whether/how to handle the impact of additional available of PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool. - Option B: TBS of initial transmission and retransmission(s) are not guaranteed to be the same - TBS for each initial and retransmission(s) is individually determined based on the actual number of occupied PRBs for that transmission - Note: the determined TBS might be different so that HARQ combining is not possible |
| WILUS | OK | We also don’t think Option A5 can solve the TBS calculation without additional designs and restriction. |
| Sharp | Yes | We prefer Option A2. For Alt.5, we share same views with other companies about how to guarantee the same TB size between initial and retransmission when GB RBs are involved. For last FFS bullet, a typo “of ” needs to be removed from “additional available of PRB(s) in intra-cell guard band(s)”. |
| xiaomi | Yes | We prefer option A3 with the flexibility. |
| CMCC | OK | Option A3 is preferred. |
| Panasonic | OK | We support option A1. |
| MediaTek |  | As commented by other companies, Alt 5 may introduce additional issues to guarantee the number of PRBs within each sub-channel is the same, which is not preferred. |
| Huawei/HiSilicon | OK for progress | For option A5, it’s unclear how to guarantee the same TBS between initial transmission and retransmission if used interlace(s) are not the same. Therefore, we would like to remove option A5. |
| Nokia/NSB |  | Do not agree with the new adding of A5. Also the A1 and A2 can be merged together under pre-defined option |

#### [M] Proposal 3-4

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary of previous round

  - Support (18): CMCC, Apple, IDC, Intel, LGE, QC, NEC, Samsung, Pana, Spredtrum, MTK, WILUS, Xiaomi, ZTE, Ericsson, Transsion, HW, ETRI,

    - Support Option 1 (5): IDC, SPRD, MTK, ZTE, CATT,

    - Support Option 2 (9): CMCC, Apple, Intel, Sharp, Xiaomi, ZTE, CATT, HW, ETRI

    - Support Option 3 (10): Apple, QC, NEC, SS, Pana, MTK, Sharp, WILUS, ZTE, HW

  - Not Support (2): Nokia, [Lenovo]

- Updates on proposal (marked in red) and justifications

  - Add RAN1#113 as the ddl for down-selection.

- FL’s response to some companies’ comments:

  - @all: previous agreement will be obeyed for sure, companies should give detailed designs for Option 2 later while satisfying previous agreement.

  - @CATT: this is subject our previous agreement on usage of intra-cell guardband (see below).

  - @Nokia: the issue you mentioned is another issue, i.e., “FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size”. We can organize related discussions later if needed. Let’s not couple issues together.

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Figure 2 Illustration of sub-channel(s) including intra-cell guardband PRBs (Based on Figure from R1-2302550)

[M] Proposal 3-4b

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 1: Such sub-channel(s) cannot be used for PSCCH/PSSCH transmission

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Option 3 is preferred to reduce the spec work on PSCCH. Option 1 has a large overhead with big subchannel size. Option 2 may require special handling of PSCCH ratematching in the GB overlapping subchannel. |
| Apple |  | Option 1 may lead to large resource waste. If Option 1 is used, the benefit of “sub-channel aligns with resource pool boundary” over “sub-channel aligns with RB set boundary” does not exist. We do not think Option 1 is a proper option. |
| Intel | Yes | Option 2 is preferred. |
| LGE | OK | For simplicity, we are also fine with Option 3. |
| Futurewei | OK | Option 3 is preferred |
| ZTE, Sanechips | Yes | Different options can be applied for different conditions, and we think special handling of PSCCH may not needed if a proper condition is defined for applying option 2, e.g., the condition may be there’s no overlapping b/w legacy PSCCH PRBs and intra-cell guardband PRBs for these subchannels. |
| vivo | Comment | As commented before, option 1 (and probably option 2) does not comply with the previous agreement. If this is the intention (to reverse/override the previous agreement), we would have some concern. If not, we think option 1 should be removed, and option 2 should be updated to ensure the guard band PRB are not used for PSCCH. Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission |
| OPPO | Not with option 1 | Option 1 does not obey the previous agreement. We would like to suggest to remove it from the proposal. |
| CATT/GOHIGH | Yes | Firstly, one calcification is needed. Regarding the previous agreement, which only says such PRBs are not used for PSCCH transmission, from our understanding, it doesn’t directly mean such sub-channel(s) cannot be used for PSCCH transmission. So option 3 only should not be supported. Anyway sub-channel 3+sub-channel 4 can be used for PSCCH/PSSCH transmission and such transmission entirely obeys the previous agreement, if the remaining PRBs within sub-channel 3 excluding guard band PRBs is sufficient for PSCCH transmission. Regarding option 1 and option 2, we can go with the direction that under which conditions option 1 can be applied, or under which conditions option 2 can be applied, either one is ok |
| Transsion | Yes | Option 3 is preferred. |
| Spreadtrum | OK |  |
| Lenovo | Yes | Option 3 is preferred. |
| WILUS | OK | Option 3 is preferred |
| Sharp |  | Agree with other companies to remove Option 1. |
| Xiaomi | Yes with comment | In the above agreement，there just means such PRBs within intra-cell guard band of two adjacent RB sets are not used for PSCCH transmission, but this proposal is discussing whether the sub-channle which includes intra-cell guardband PRBs can be used for PSCCH/PSSCH transmission or not, we think these two issues are the separate issue. And in current agreement, there still has a FFS whether/how to use intra-cell guardband PRBs, so the mapping rule between the guard PRBs and the subchannel is not clear, it is not reasonable to use the above agreement to exclude the option 2. |
| CMCC | OK | Option 2 is supported. |
| Panasonic | OK | We support option 3. |
| MediaTek | Yes |  |
| Huawei/HiSilicon | Yes | We think option 2 and option 3 can be supported under different conditions, i.e., whether the available PRBs of such sub-channel is larger than or equal to the configured number of PRBs for PSCCH. Option 2, 3 can still satisfy previous agreement with the condition above, i.e., guardband PRB are not used for PSCCH transmission. |
| Nokia/NSB | OK | Option 2 is preferred |

#### [L] Proposal 3-5

Background: IRB or CRB

- Summary of previous round

  - Almost all companies are fine with it.

- Updates on proposal (marked in red) and justifications

  - Changed to “SL-BWP” level as per LGE and ZTE comment. This might be simpler.

[L] Proposal 3-5b

A resource pool SL-BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

FL’s comment: Proposal 3-5b is endorsed by email directly.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| LGE | OK |  |
| Futurewei | OK |  |
| DCM | OK |  |
| CATT/GOHIGH | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | OK |  |
| Lenovo | OK |  |
| WILUS | OK |  |
| Sharp | OK |  |
| xiaomi | Yes |  |
| CMCC | OK |  |
| Panasonic | OK |  |
| MediaTek | Yes |  |
| Nokia/NSB | OK |  |
| Huawei/Hisilicon | Yes |  |

### Round 4: Proposals and summary of previous round

#### [H] Proposal 3-1

Background: IRB, frequency domain resource indication

- Summary of previous round

  - Support (20): QC, Apple, Intel, LGE, FTW, DCM, ZTE, vivo, OPPO, CATT, Transsion, SPRD, Lenovo, WILUS, Xiaomi, CMCC, Pana, MTK, HW, NOK

    - Remove 1st FFS (15): QC, Apple, Intel, LGE, vivo (prefer), OPPO, CATT, Transsion, Lenovo, WILUS, LGE, CMCC, MTK (prefer), HW, NOK

    - Keep 1st FFS (4): DCM/Pana (keep RB set part), vivo/MTK (can live with)

  - Not Support (/): /

- Updates on proposal (marked in red) and justifications

  - Made the following updates as per comment from LGE.

    - “For a TB, the initial transmission and retransmissions indicated by a SCI use the same number of sub-channel(s) and same number of RB set(s)”

  - Made the following updates as per comment from Docomo. Let’s see whether companies can live with this version. I hope companies can be flexible somehow, if some companies think the design in the FFS is not necessary, they can bring more analysis next meeting.

    - “FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case”

  - Made the following updates based on comment from Lenovo.

    - “Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)”

    - “FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether the used interlace(s) can be non-contiguous, etc.”

- FL’s response to some companies’ comments:

  - @LGE: FL suggests not spending too much time on refining the wording of FFS, especially the examples in FFS. We use “whether to consider bitmap design”. Companies can bring more detailed analysis next meeting.

  - @Sharp: whether to consider bitmap design is unsure now, so let’s FFS it.

[H] Proposal 3-1e (IRB, RA indication)

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and retransmissions indicated by a SCI use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of sub-channel(s) and/or different number of RB set(s) in such case

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether the used interlace(s) can be non-contiguous, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

FL’s comment: this proposal is near stable, let’s discuss by email directly.

#### [M] Proposal 3-2

Background: IRB, remaining details on mapping between sub-channel and interlace

- Summary of previous round

  - Almost all companies are ok with it.

- Updates on proposal (marked in red) and justifications

  - There were quite some active discussions on the mapping if RAN1 supports that one SL resource pool can include sub-set of PRBs of one RB set. However, views are still divergent.

  - From FL’s perspective, considering the limited time in this meeting, it does not make too much sense to spend more time discussing a case that RAN1 has not supported yet. If RAN1 support that case later, we can come back to the mapping then.

  - FL updated the proposal accordingly.

- FL’s response to some companies’ comments:

  - @LGE: I organized a separate proposal for that point later, see Proposal 3-6.

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - …

Figure 1 Illustration of mapping between sub-channel and interlace (Based on Figure from R1-2303368)

[M] Proposal 3-2e (IRB, mapping)

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

    - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on

    - Note: if RAN1 agrees to support that one SL resource pool can include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace will be further discussed

- Interlace is indexed as per NR-U

FL’s comment: this proposal is near stable, let’s discuss by email directly.

#### [M] Proposal 3-3

Background: IRB, TBS determination

- Summary of previous round

  - Have concern on keeping Option A5 (12): QC, Apple, FTW, ZTE, OPPO, CATT, SPRD, WILUS, Sharp, MTK, HW, Nokia

  - Ok to keep Option A5 (3): LGE, vivo, DCM,

- Updates on proposal (marked in red) and justifications

  - Based on Docomo’s comment, remove Option A5 and add a Note.

  - Corrected a typo “…available of PRB(s)…”.

Agreements (RAN1#100):

For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations.

- …

- It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

  - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s).

[M] Proposal 3-3e (IRB, TBS N_ref)

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings:

- Option A: TBS of initial transmission and retransmission(s) are guaranteed to be the same

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - [ Option A5: N_ref is the actual number of occupied PRBs for that transmission except for PRBs within intra-cell guard band, if any. ]

      - Note: it is assumed that the number of PRBs within each sub-channel is the same.

    - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration.

    - FFS: for TBS determination, whether/how to handle the impact of additional available of PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

FL’s comment: this proposal is near stable, let’s discuss by email directly.

#### [M] Proposal 3-4

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary of previous round

  - Support Option 1 (1?): [CATT]

  - Support Option 2 (5): Intel, CMCC, HW, Nokia, [CATT]

  - Support Option 3 (8): QC, LGE, FTW, Transsion, Lenovo, WILUS, Pana, HW,

- Updates on proposal (marked in red) and justifications

  - Remove Option 1 based on comment from vivo, OPPO, Shapr, etc., since it violates previous agreement (see red part below).

  - Add the following to Option 2 to highlight there is no intention to violate previous agreement.

    - “PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement”

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Figure 2 Illustration of sub-channel(s) including intra-cell guardband PRBs (Based on Figure from R1-2302550)

[M] Proposal 3-4e (CRB, guardband PRB)

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 1: Such sub-channel(s) cannot be used for PSCCH/PSSCH transmission

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

  - PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

FL’s comment: this proposal is near stable, let’s discuss by email directly.

#### [L] Proposal 3-6

Background: IRB, clarification on lowest sub-channel

- In previous agreement, there is a sentence that “Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH” (see below).

- LGE mentioned RAN1 needs to clarify the meaning of “lowest sub-channel” (see below).

- FL organized a proposal to check views.

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

(LGE’s comment in 1st round)

| LGE | For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest location, since different PSSCH allocations can have the same PSCCH location, there could be ambiguity on the PSSCH allocation even though the RX UE knows the location of PSCCH and the number of sub-channels. In this case, there are two approaches. Option 1: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest index. Option 2: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest frequency location. - For the case when different PSSCH allocations have the same PSCCH location, some PSSCH allocation is not used to avoid the ambiguity on the PSSCH allocation between TX UE and RX UE. |
| --- | --- |

[M] Proposal 3-6e (IRB, lowest sub-channel)

For interlace RB-based PSCCH/PSSCH transmission in SL-U, down-select one of the following:

- Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

  - i.e., in a resource pool, lowest sub-channel is sub-channel#0

- Option 2: lowest sub-channel is the sub-channel with lowest frequency location

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | Legacy FRIV assumes that PSCCH is located at the 1st subchannel of the PSSCH allocation, and the allocation/reservation is relative to the subchannel where PSCCH is detected. If legacy FRIV for interlaced subchannel indication is reused, option 1 would have less spec impact while option 2 FRIV interpretation is more complicated. |
| LGE | Option 1 | We have the same understanding with QC. |
| ZTE, Sanechips | Option 1 | But the ‘i.e.,’ part is not needed, as we are talking about lowest sub-channel of a PSCCH/PSSCH transmission Not lowest sub-channel of a resource pool. |
| NEC | Option 1 | Seems option 1 is a simper one |
| Lenovo | Option 1 | We share the same views as QC. |
| OPPO | Option 1 |  |
| Spreadtrum | Option 1 | Same understanding with QC. |
| MediaTek | Option1 |  |
| vivo | Option 1 |  |

## Issue#4: PSFCH and SL-HARQ

### [Closed] 1st round: Proposals and background

#### [H] Proposal 4-1

Background: PSFCH transmission for 15/30 kHz, OCB requirement

- Summary

  - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Support (9): Futurewei, Nokia, NEC, InterDigital, ITL, [Qualcomm] (merge Alt 1-1a/2-4a), Docomo (merge Alt 1-1a/2-2a), Huawei/HiSilicon, National Spectrum Consortium

  - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

    - Support (2): CATT, Docomo (merge Alt 1-1a/2-2a),

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

    - Support (3): Vivo, Sharp, Xiaomi

  - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

    - Support (7): LGE, Samsung, Intel, CATT, Sharp, WILUS, [Qualcomm] (merge Alt 1-1a/2-4a)

  - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - Support (5): OPPO, ZTE, Apple, Lenovo, Panasonic

- FL’s view

  - Alt 1-1a:

    - Quite some companies support it since it’s simple and effective.

    - Some companies point out the power sharing issue if common PRB and dedicated PRB locate within 1 MHz. A simple way to handle this is to drop common PRB in this case. This is reflected in the updated proposal.

  - Alt 2-2a:

    - Some companies point out the interference issue is serious since there are multiple transmissions on the same PRB, e.g., Qualcomm, Docomo, Huawei/HiSilicon, etc.

  - Alt 2-3a/2-4a:

    - Some companies support this and think PSFCH capacity may not be an issue, mainly because the actual number of PSFCH transmissions might be limited due to limited number of FDMed transmissions, e.g., Vivo, Intel, etc.

    - However, as pointed out by some companies (Docomo, Panasonic, Huawei/HiSilicon, etc.), how to do PSSCH-PSFCH mapping is unclear for Alt 2-3a/2-4a (see Example#1 below). It seems Alt 2-3a/2-4a cannot reuse legacy PSSCH-PSFCH mapping and need much larger specification changes than Alt 1-1a/3-2a.

    - In addition, Alt 2-3a/2-4a may have following issues: #1 Very small group size for groupcast option 2; #2 High interference since the number of CS pairs cannot be small; #3 Seems unable to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”.

    - @Proponent of Alt 2-3a/2-4a, please try to clarify how to do PSSCH-PSFCH mapping in Example#1 below and how to update Alt 2-3a/2-4a accordingly, to help companies know whether/how it works. For now, FL add a bracket for Alt 2-3a in the updated proposal.

  - Alt 2-4a:

    - As mentioned by some companies (Docomo, Nokia, etc), PAPR reduction is a separate issue and can be further considered regardless which Alt is selected.

    - To avoid coupling different issues and to make RAN1 discussions more focused, FL remove Alt 2-4a and add a separate bullet to reflect this.

  - Alt 3-2a:

    - FL add more details for K2 and K4 based on the input.

  - In summary, FL suggests to down-select among updated Alt 1-1b, [2-3a] and updated 3-2b in this meeting (hopefully) or next meeting. The following proposal is given to reflect above.

Example#1 (PSSCH-PSFCH mapping): As shown in Figure 3 below, assume 30 kHz SCS, a resource pool includes 1 RB set and 5 sub-channels (1 sub-channel = 1 interlace), PSFCH periodicity is 4 slots.

- There are 5*4=20 candidate resources within 1 PSFCH period (i.e., each sub-channel on each slot).

- As per legacy NR SL PSSCH-PSFCH mapping (TS 38.213 Clause 16.3.0), the number of PRBs for PSFCH transmission is at least 20 by (pre-)configuration, so that each candidate resource is mapped to orthogonal PRB for PSFCH.

- Alt 1-1a and 3-2a can easily reuse legacy PSSCH-PSFCH mapping by (pre-)configuring more than 20 dedicated PRBs for PSFCH.

- However, for Alt 2-3a/2-4a, since there are only 5 dedicated interlace in this case, it’s impossible to allocate orthogonal interlace to each candidate resource. So how to do PSSCH-PSFCH mapping is unclear for Alt 2-3a/2-4a.

  - Some companies discussed this case and mentioned a potential way is using multiple CS pairs, e.g., Docomo, Huawei/HiSilicon, Panasonic, LGE, Samsung, Ericsson, Qualcomm, etc.

  - For example, assume 6 CS pairs are (pre-)configured. Then, the total number of orthogonal PSFCH resources in one PSFCH period is 30 (5 orthogonal interlace * 6 CS pairs), and a corresponding PSSCH-PSFCH mapping might be:

    - PSSCH on R#0 is associated with PSFCH resource on {interlace#0, CS pair#0}

    - PSSCH on R#1 is associated with PSFCH resource on {interlace#0, CS pair#1}

    - PSSCH on R#2 is associated with PSFCH resource on {interlace#0, CS pair#2}

    - …

    - PSSCH on R#6 is associated with PSFCH resource on {interlace#1, CS pair#0}

    - …

  - However, the above PSSCH-PSFCH mapping have following issues:

    - #1 Very small group size for groupcast option 2: in the above example, the group size for groupcast option 2 is only one!

    - #2 High interference: It seems the number of CS pairs must be 6, i.e., cannot be 1,2,3, thus having high interference.

    - #3 Seems unable to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”: RAN1 already agreed to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”, some companies propose that multiple different PSFCH resources sets should be used (e.g., Xiaomi, Qualcomm, etc.), then how to do PSSCH-PSFCH mapping in this case for Alt 2-3a/2-4a is unclear.

Figure 3 Illustration of PSSCH-PSFCH mapping (Based on Figure from R1-2302354)

[H] Proposal 4-1

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

  - [ Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace ]

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

- FFS: Regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

FL’s comment: please also indicate which Alt you support.

FL’s comment: Proponent of Alt 2-3a, please try to clarify how to do PSSCH-PSFCH mapping in Example#1 above and how to update Alt 2-3a accordingly, to help companies know whether/how it works.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | Comment | For Alt 1-1b, the 2nd sub-bullet should be updated. It seems that the 2nd sub-bullet reflects PSD limitation. However, more power allocation for dedicated PRB(s) is another important aspect. Therefore, the condition of ‘within the same 1 MHz bandwidth’ is not reasonable. Besides, it should be clarified that CDM is also applicable among UEs. - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured, FFS value range - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB - PRBs within the common interlace that are the closest to each of K3 dedicated PRB(s) are not used - At the K3 dedicated PRB(s), user-multiplexing by using different cyclic shifts can be applied With this update, we support Alt 1-1b. For Alt 3-2b, how to meet PSD limitation always is quite unclear. |
| vivo | OK | We think Alt 2-3a is the most simple and workable solution. The PSSCH-PSFCH mapping can be done based on the starting sub-channel and the interlace of PSFCH (as well as the CS as defined from Rel-16). The issues mentioned in the example#1 above may actually not an issue in unlicensed band (as least for #1 and #2), due to the target deployment scenario. Regarding the issue#3, our understanding is that this issue is common to all the options, and can be handled by shifting the index (interlace index, or PRB index, or CS index, etc.). Due to various regulation rules (LBT, OCB, etc.), it is anyway not expected that there is a large number of FDMed UE or a groupcast with a large number of receivers in unlicensed band. |
| LGE |  | First of all, feasibility of each alternative needs to be checked. As we know, in RAN4 perspective, Alt 2-2a/2-3a/2-4a are feasible since it comes from NR-U PUCCH format 0. Our understanding is Alt 2-4a is interlace PUCCH format 0, why this option is removed even though it is supported by a number of companies. Alt 3-2a would be feasible as well since it can be seen as the simultaneous transmission of multiple single-PRB PSFCH transmission. However, Alt 1-1a seems brand new format. It can be seen as simultaneous transmission of a single interlace and some non-contiguous PRBs. Considering that the maximum number of multiple single-PRB PSFCH transmission in a time is limited, it is unclear whether the Alt 1-1 is feasible or when it can be feasible. It would be necessary to confirm with RAN4 if we go to Alt 1-1a. Considering that the time is limited, it would be more safe direction to go with Alt 2-2a/2-3a/2-4a or 3-2a which are feasible. Regarding the GC option 2, if the group size is big, then we already have a solution which is to use GC option 1 as a fallback mode. Moreover, it is also possible that the PSFCH resources associated with the allocated subchannels are used instead of the starting subchannel. It can increase the number of PSFCH resources for GC option 2. In other words, the system is not broken even though interlaced structure is used. |
| Nokia/Nsb | Alt 1-1b is preferred |  |
| OPPO | Yes | OK with the proposal and we prefer Alt 3-2b. |
| Interdigital | Yes | We prefer Alt. 1-1b for simplicity. |
| Apple | Yes | We prefer Alt 3-2b for its simplicity. In Alt 1-1a, K3 dedicated PRBs are used for PSFCH transmission, and in Alt 3-2b, K4 dedicated PRBs are used for PSFCH transmission. Hence, in nature, there is no performance difference between these two options. For Alt 3-2b, we think common PRBs and dedicated PRBs could be within 1 interlace to address PSD limit. |
| NEC | Yes | Alt. 1-1b. We think this alt1-1b is the simplest way. By using the common interlace, the remaining PRBs (excluding this common interlace only) could be totally reuse the R16 PSSCH-PSFCH mapping procedure. For Alt 3-2b, we don't know whether PRBs in two edge side would cause RAN4 impacts. |
| Fraunhofer | Yes | We are ok with Alt 1-1b. |
| Qualcomm | Yes | Ok with the proposal. In our view, Alt 2-3/4a is from NR-U PUCCH format 0 and spec support could be simple. Alt 1-1 may also be needed if GC option 2 is to be supported for large number of GC group. |
| Samsung |  | With the understanding that the deleted Alt 2-4a is a special case of Alt 2-3a, we are ok with the proposal by removing the bracket around Alt 2-3a. Also, we want to note that the most important metric for comparing the alternatives is the detection performance, while by seeing many parameters unknown in Alt 1-1b and Alt 3-2b. We want a consensus of such parameters for evaluation purpose. |
| Intel | No | We actually we do not agree with the proposal and to remove 2-4a, as this option is already supported in the spec and already optimized for PAPR issue, so from spec perspective all except 2-4a are new designs and would require spec impact. As for the assessment and question on how PSSCH-PSFCH mapping would be done for option 2-3/4a, our understanding is that a few directions could be taken: (1) not to configure multiple SL sub-channels, with the motivation that high multiplexing may be problematic in SL-U due to propagation delays and sync-error which may lead to blocking each other on PSSCH transmissions in different sub-channels, i.e., better to keep as small number as possible. Thus PSFCH capacity may only be limited by PSFCH periodicity of 1,2,4 slots. (2) To use CS to improve capacity, and CS may be extended beyond only 6 values, because PSFCH occupies multiple RBs, thus multiple CS values may be added vs. 6 CS available by default for 1 PRB. For what concerns the feasibility of option 2-3/4a, as mentioned by other companies, option 2-4a is supported in NR-U for PUCCH format 0, and would require the really minimum spec impact. As option 1-1b and 3-2b are meant to improve capacity, we would like to ask companies to explain to us how in a practical deployment FDM for SL may work and how alignment across more than two UEs may be feasible if UEs may additionally have different source of synchronization, may be subject to mobility and may not know their reciprocal position. |
| Futurewei | Alt 1-1b is preferred |  |
| Lenovo | Yes | Alt 3-2b is preferred. |
| Spreadtrum | Yes | We are fine with the proposal. We prefer Alt 3-2b. And we can further discuss how to meet PSD limitation. |
| Sony | Yes | We prefer Alt. 1-1b. |
| CMCC | Yes | OK with Alt.1-1b. |
| xiaomi | Yes |  |
| CATT/GOHIGH | Comment | We can accept alt 1-1b and alt 2-4a which are aligned with the previous agreement that only IRB-based structure can be used for PSFCH transmission. Besides, for alt 1-1b, one FFS part on the coverage issue due the lower power in dedicated PRB is needed. |
| Panasonic | Yes | OK with the proposal and we support Alt 3-2b. |
| Transsion | Yes | We prefer Alt. 1-1b |
| MediaTek | Yes | Alt 1-1b is preferred. |
| ZTE | OK with comments | For alt 1-1b, if common PRB at the edge is directly dropped, then OCB requirement may not be met, so it should be clarified that the common PRB dropping satisfies the OCB requirement. Moreover, considering the power on common PRB may lead to IBE interfere to adjacent dedicated PRB, guard band PRB/RE between common PRB and dedicated PRB(s) need to be considered. For alt 3-2a, in our opinion, K2 common PRBs located at the two edges of a RB set does not exclude the case that the K2 common PRBs are within a interlace. So the proposal is suggested to be updated: Regarding PSFCH transmission with 15 kHz and 30 kHz SCS: - RAN1 down-select one of followings: - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured, FFS value range - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB under OCB requirement - [ Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace ] - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - K2=2 - K4 is (pre-)configured, FFS value range Note: K2 Common PRB within a interlace is not excluded - FFS: Regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR FFS: whether IBE issue exists and whether/how to address it |
| ZTE | comments | We think the second sub bullet is better to be for FFS. |
| Huawei/HiSilicon | Yes | We prefer Alt 1-1b. As per R16 SL design, PSFCH resource allocation for PSCCH/PSSCH within one PSFCH period is designed to support orthogonal resources among orthogonal PSCCH/PSSCH transmission. As shown in example#1, Alt 2-3a cannot support GC option 2 due to only 1 RX UE is supported. In addition, since more than 1 PSFCH occasion per PSCCH/PSSCH transmission is supported, how Alt 2-3a works is unclear even for unicast type. |
| WILUS | No | We don’t agree with removal 2-4a which is already supported in NR-U. As mentioned by Samsung, if Alt 2-4a is a special case of Alt 2-3a, we can accept the proposal by removing the bracket around Alt 2-3a. |

#### [M] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Summary

  - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Support (15): Qualcomm, Samsung, OPPO, Docomo, vivo, ZTE, [LGE], Nokia, Apple, Intel, CATT, Xiaomi, Spreadtrum, Panasonic, CMCC

  - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - Support (14): vivo, Nokia, MediaTek, [Lenovo], Futurewei, National Spectrum Consortium, Huawei/HiSilicon, Spreadtrum, Sony, InterDigital, Transsion Holdings, Fraunhofer, NEC, WILUS

- FL’s view

  - Some companies supported Option 1 and gave some details, e.g., locations of such PSFCH occasion(s) (e.g., TDMed, FDMed, etc.), impact on HARQ RTT, UE behavior on transmitting PSFCH (e.g., a UE attempts transmit PSFCH only if it has not transmitted on any precedent PSFCH occasion corresponding to the same TB), etc.

  - Some companies supported Option 2 and point out that in case of COT sharing and no HARQ-ACK feedback is needed (e.g., blind retransmission, broadcast, MCSt enabled, etc.), Option 1 will cause COT interruption and waste resource.

  - A compromised proposal is given below.

[M] Proposal 4-2

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- By default, one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. Other PSFCH occasion(s) within its COT are used for PSSCH transmissions.

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | We do not support dynamic indication. How to avoid PSFCH conflict among UEs is unclear. This issue was solved by semi-static PSFCH-PSSCH mapping in R16/17 SL. It seems that this proposal does not reflect our concern. |
| vivo | OK |  |
| LGE | No | RB set outside the shared COT still have the PSFCH occasion, and they will be FDMed with the RB set disabling PSFCH occasion in the same time. It definitely cause the AGC problem. Moreover, it is unclear whether the RX UE always knows whether the TX UE utilize the COT for its PSCCH/PSSCH transmission. If not, TX UE and RX UE would have different understanding on the PSSCH resources. |
| Nokia/Nsb |  | We can only agree with the first bullet point |
| OPPO | No | As it is analyzed in our contribution, dynamically indication on PSFCH occasions has some potential issues that need to be considered, such as PSFCH collision, PSFCH resources misalignment due to FDMed resource pools. It also has impact on resource selection procedures and significant large specification work to be done. Therefore, Option 1 should be supported. |
| Interdigital | OK |  |
| Apple | No | If the first few slots in a shared COT have PSFCH resources, the corresponding PSCCH/PSSCH is likely transmitted before the shared COT. The dynamic indication of using these PSFCH resources in impossible. It seems the gap of PSFCH transmission in a COT is not avoidable, hence, we do not need to additionally design the dynamic indication of PSFCH occasion usage. |
| NEC | Yes | Regarding PSFCH collision issue, for the UEs within a shared COT, UE could avoids PSFCH collision from the received COT sharing information, e.g., how many slot, sub-channels, etc. are within this COT and mapped to the dynamically indicated PSFCH occasion |
| Ericsson | Yes |  |
| Fraunhofer | OK |  |
| Qualcomm |  | We support the first bullet point only. The 2nd bullet could be for FFS. |
| Samsung |  | We agree with the first bullet for now, and don’t agree with the second bullet. We didn’t see the reasoning on the relationship between PSFCH occasion selection and COT sharing. |
| Sharp |  | To have a common understanding, we would like to ask whether cross-COT PSFCH transmission/reception is allowed or not. That is, within a sharing COT, is it allowed for a UE to transmit PSFCH on the (pre-)configured PSFCH occasions associated to PSSCH resource preceding the COT. |
| Intel | No | We do not support dynamic indication as we believe this is an optimization and not an essential component of the design. Dynamic indication may also require a large spec impact while leading to even more collisions and uncertain behavior as the TX UE does not have any assurance or certainty that in those dynamic resources a RX UE may be able to transmit since some gaps may be unavoidable. Therefore, option 1 is preferred. |
| Futurewei | OK |  |
| Lenovo | YES |  |
| Spreadtrum | No | We support the 1st bullet, and the second bullet can be FFS. For the dynamical indication, if other PSFCH occasion(s) within the COT are used for PSSCH transmissions, other UE (not RX UE) cannot use the pre-configured FDM PSFCH transmission resources. Or, the PSSCH transmissions may be collided with other UE’s PSFCH transmissions. So, it is not clear that how to avoid the collision between PSFCH transmissions or PSFCH and PSSCH transmissions. |
| Sony | Yes |  |
| CMCC | No | We share the same views as DCM, LGE, OPPO and Apple, where the dynamically indicated PSFCH may cause some further issues. |
| xiaomi | Yes | We support the PSFCH occasion(s) are (pre-)configured, and we have concern about dynamic indication. With the dynamic indication, the mapping relationship between PSCCH/PSSCH and the corresponding PSFCH will be broken, thus PSFCH transmissions collision problem needs to be investigated and solved. |
| CATT/GOHIGH | No | The COT sharing mechanism is still unclear and is under discussion in AI 9.4.1.1, so we can only accept the first bullet. |
| Panasonic |  | We support only first bullet. |
| MediaTek | Yes |  |
| xiaomi | Yes | We support the PSFCH occasion(s) are (pre-)configured, and we have concern about dynamic indication. With the dynamic indication, the mapping relationship between PSCCH/PSSCH and the corresponding PSFCH will be broken, thus PSFCH transmissions collision problem needs to be investigated and solved. |
| Huawei/HiSilicon | Yes | As for 1st bullet, due to the advantages of COT sharing, PSFCH TX UEs within a COT can successfully feedback via type 2 LBT, therefore, additional PSFCH occasions within a COT are unnecessary. In addition, there may be cases that no PSFCH transmission is needed, resulting in COT loss and reduced COT duration for PSCCH/PSSCH transmission. We support 2nd bullet to solve issues above. |

#### Others

- Some other issues are mentioned:

  - Some companies propose to introduce new PSFCH format, etc.

- FL assumes such issues are not very urgent at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

### [Closed] 2nd round: Proposals and summary of previous round

#### [H] Proposal 4-1

Background: PSFCH transmission for 15/30 kHz, OCB requirement

- Summary of previous round

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Support (13): [DCM], Nokia, Interdigital, NEC, Fraunhofer, [Qualcomm], Futurewei, Sony, CMCC, CATT, Transsion, MediaTek, Huawei/HiSilicon

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

    - Support (6): vivo, [LGE], Qualcomm, Samsung, CATT, [WILUS]

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - Support (6): [LGE], OPPO, Apple, Lenovo, Spreadtrum, Panasonic

- Updates on proposal (marked in red) and justifications

  - For Alt 1-1b:

    - Add “On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH” as per Docomo’s comment.

    - Add “…UE only transmits on the dedicated PRB subject to OCB requirement” as per ZTE’s comment.

  - For Alt 2-3a:

    - Vivo gave some PSSCH-PSFCH mapping designs. It seems different from legacy NR SL PSSCH-PSFCH mapping, and maybe PSFCH collision may happen.

    - Intel mentioned RAN1 can consider introducing more than 6 CS pairs if needed.

    - Qualcomm mentioned Alt 1-1 may also be needed if GC option 2 is to be supported for large number of GC group.

    - FL reflected the above as examples in the proposal. Proponents of each Alt need to give more details to help other companies understand whether/how it works, thus helping RAN1 down-selection.

  - For Alt 3-2b:

    - Add “FFS: how to meet PSD limitation” as per Docomo’s and Spreadtrum’s comment. E.g., when UE transmits on 2 common PRBs and 1 dedicated PRB, and use 23 dBm power, then the PSD limit may not be satisfied.

    - Add “FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s), e.g., whether they are on the same interlace with common PRB” as per Apple’s comment.

    - @all: as mentioned by CATT, Alt 3-2b may not be aligned with previous agreement as below. Companies, especially proponents of Alt 3-2b, please share your views on this aspect in this round.

      - Agreement: To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

  - For the FFS on IBE:

    - Add “E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB” as per ZTE’s comment.

- FL’s response to some companies’ comments:

  - @LGE, Samsung, Intel, WILUS: on Alt 2-4a

    - As FL explained in the 1st round background, the intention to remove Alt 2-4a and have a separate sub-bullet about PAPR reduction is to decouple different issues and help RAN1 discussions being more focused.

    - Whether or not to support PAPR reduction as in NR-U can still be discussed, and FL used “regardless of which Alt above is selected, …” to highlight this.

  - @Docomo: within one interlace, the distance between two IRBs is 10/5 PRBs for 15/30kHz SCS, i.e., 1.8 MHz. Considering the regulation has PSD limit (e.g., 10dBm/MHz), to avoid power sharing between dedicated PRB and common PRB within 1 MHz, UE may need to drop 1 or 2 common PRB depending on the location of dedicated PRB in Alt 1-1b. FL assumes this is aligned as your suggestions, and hope it clarifies.

  - @CATT: as for the coverage issue you mentioned, FL assumes this can be addressed by configuring larger value of K3 if needed. Since the value range is FFS, companies can take this into account when they propose detailed value range.

  - @ZTE: in Alt 3-2b, the main bullet says “K2 common PRBs locate at the two edges of a RB set”. So their locations are already determined.

[H] Proposal 4-1b

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB subject to OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s), e.g., whether they are on the same interlace with common PRB

- R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details

  - Note: companies are encouraged to give more details and analyze the specification impact

    - E.g., in Alt 2-3a, whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc.

    - E.g., in Alt 2-3a, whether introducing more than 6 CS pairs is needed

    - E.g., for group cast option 2, what’s the maximum group size that can be supported

    - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

  - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes |  |
| Apple |  | We do not think the newly added second bullet is needed at this stage. But for progress, we can keep the main second bullet with the following changes: 1. Remove all the examples under this bullet. Maybe some examples could be added under the proper Alternatives in the first bullet. 2. Change “baseline” to “starting point”. This is because in SL-U, a resource pool can have multiple RB sets and the indexing of PSFCH resources over different RB sets needs to be explored. For Alt 2-3a, we still think the PSFCH capacity is an issue. |
| InterDigital | Yes |  |
| Intel | Comments | After FL’s explanation, we are generally OK with the proposal and to compromise to make a step forward. However, we are still not OK to keep option 3-2b since we have serious concerns with it: - It does not comply with the agreements so far. We have already agreed that PSFCH would be mapped into an RB-based interlace and clearly option 3-2b won’t follow this agreement; - As in this option only few PRBs are used for transmission, while regulatory requirements mandate a PSD of 10 dBm/MHz, this solution would pose serious issues in terms of coverage. |
| Fraunhofer | Yes |  |
| vivo | Yes |  |
| DCM | OK with update | For Alt 1-1b, as commented before, drop of some common interlace PRBs is not only for OCB/PSD requirement but also for boosting power for dedicated PRB. From this point, regardless of regulation, this dropping behavior should be applied always. We suggest removing the added part. - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB subject to OCB requirement |
| LGE |  | To move forward, Alt 1-1b should be deprioritized. To complete this WI for all the WG, we also need to consider the impact to another WG. To be specific, in the current RAN4 spec, there are three type of allocations: contiguous resources, almost contiguous resources (the gap PRB portion over the occupied PRBs <= 0.25), interlaced type resource allocation. If we go to Alt 1-1b, RAN4’s work load could be excessively large since they need to analyze MPR or performance requirement for a variety of the locations of a single interlace and the locations of dedicated PRBs. On the other hand, if we go to Alt 2-3a, it is possible to reuse MPR or performance requirement for interlaced PUCCH format 0, and it will simplify RAN4’s work. Moreover, in RAN1 perspective, for Alt 1-1b and Alt 3-2b, we may need to discuss further how to design sequence mapped on the common PRB/interlace. For compromise, we can merge all the alternatives. First of all, for a single UE perspective, a single interlace within a RB set is used for a single PSFCH transmission. Next, the UE map its own sequence for control information feedback on the interlace within the RB set. Among multiple UEs or different PSFCH TX, some PRBs belonging to the interlace within a RB set is not overlapping each other (like dedicated PRB). Among multiple UEs or different PSFCH TX, the other PRBs belonging to the interlace within a RB set can be overlapping each other. |
| Qualcomm | Yes | Alt 2-3/4a is from NR-U PUCCH format 0 and spec support could be simple. Alt 1-1 may also be needed if GC option 2 is to be supported for large number of GC group. |
| OPPO | Yes with comment | Generally we are fine with the proposal. For the first FFS in Alt 3-2b, to address the concern about the PSD limitation, one potential solution can be as follows (we are open to any other candidate solutions). - The gap between each of K2 common PRBs and each of K4 dedicated PRBs is >1 MHz so that common PRB and dedicate PRB will not be within 1MHz; - If K4 =1, only this PRB is limited by PSD limitation. if K4>1, all K4 PRBs equally share the transmit power which is determined by PSD limitation. For the second FFS in Alt 3-2b, we are OK to keep it open now. |
| NEC | Yes | A wording comment as following: - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE may only transmits on the dedicated PRB and this behavior should be subject to OCB requirement Add "may" because based on possible outcome of Proposal 6-1, UE may adjust the PSFCH power to solve the power sharing problem and thus no drop is needed. Add " and this behavior should be " because the original wording may imply that UE only transmits on the dedicated PRB to meet OCB requirement which is misunderstanding. |
| Samsung |  | We don’t think the second bullet is needed. Anyway, either proposal needs to consider enhancement to the PSSCH-PSFCH mapping. Also, is it possible to try to narrow down the value of parameters K3 and K4? In our understanding, every proposal has potential tradeoff between detection performance and capacity, and the parameter values are the key for evaluating the performance. |
| Panasonic | Yes | For Alt 3-2b, we are OK the restriction that K3 and K4 are located on a same interlace. It is similar as previous alt 3-1a in RAN1#111. The restriction is aligned with previous agreement and can meet the OCB and PSD requirement. |
| Spreadtrum | Yes |  |
| MediaTek | Yes |  |
| Lenovo | Yes with comments | Alt 3-2b is preferred although we are OK with this proposal. Further comments are listed below: 1. On Alt 1-1b, It is not aligned with RB-based interlace structure with the additional UE behaviors of dropping PRB of the common interlace if the PRB is near to the dedicated PRB. E.g., interlace 0 is used as common interlace, if the dedicated PRB is PRB 4 within 1MHz BW of PRB 0 and PRB 0 is dropped, can we still deem the UE transmits PSFCH on a common interlace? The intention of Alt 1-1b is to provide abundant PSFCH resources compared with using dedicated interlace. why is it necessary to use multiple CS pairs on dedicated PRBs? If companies think Alt 1-1b can’t provide abundant PFSCH resource, then we need go with Alt 3-2b which can provide more resources. 2. On Alt 2-3a We do think CS pair should be considered for dedicated interlace structure; otherwise, the PSFCH capacity is quite limited, e.g., only 5 interlaces in case of 30kHz SCS. 3. on Alt 3-2b Alt 3-2b isn’t against existing agreement since there is “at least” put before RB-based interlace. RAN1 has not achieved any conclusion to exclude the structure of Alt 3-2b. In fact, Alt 1-1b can be deemed as a variant of RB-based interlace which is not aligned with RB-based interlace. Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Additionally, to address PSD issue, one way is to consider guard band with 1MHz separation between common PRB and dedicated PRB; another way is to restrict common PRB and dedicated PRB are on same interlace. Both ways can easily solve the PSD issue. 4. on the newly added bullet We think PSSCH-PSFCH mapping is separate issue from PSFCH structure design because only a PSFCH resource index is needed after PSSCH-PSFCH mapping and whether this PSFCH resource index is pointing to a PSFCH resource based on Alt 1-1b, Alt 2-3a or Alt 3-2b is not relevant to PSFCH structure. |
| Sharp | Yes |  |
| WILUS |  | We also don’t think the second bullet is necessary. If it needs to further consider enhancement of PSSCH-PSFCH mapping, we suggest to put on the FFS for enhancement of PSSCH-PSFCH mapping in SL-U without mentioning which one is baseline. |
| xiaomi | Yes | We support 2-3a, and we support use the cyclic shift for 2-3a to improve the capacity. |
| ZTE, Sanechips | Agree in principle | For Alt 3-2b, there may also some location restriction on the K2 common PRBs, we proposed to updated the FFS as: - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and K2 common PRBs, e.g., whether they are on the same interlace with common PRB |
| Transsion | Yes |  |
| CATT/GOHIGH | Yes with comment | We are generally ok with the proposal. Alt 3-2b is not aligned with previous agreement that IRB-based structure should be used for PSFCH transmission. Thus, alt 3-2b should be removed |
| Huawei/HiSilicon | Ok for progress | We support Alt 1-1b. For Alt 2-3a, PSSCH-PSFCH mapping is still unclear, we assume quite large specification changes are needed. If Alt 2-3a cause PSFCH resource collision, then the reliability of PSFCH transmission itself can not be guaranteed. It’s unreasonable to re-design a new mapping rule whilst such rule still can not ensure the same HARQ performance as existing R16 design under the cost of large specification workload. |
| Nokia/NSB2 | Agree | The Alt 1-1b has the major support. And we should down-select the other two alternatives |

#### [M] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Summary of previous round

  - Support (10): vivo, Interdigital, NEC, Ericsson, Fraunhofer, Futurewei, Lenovo, Sony, MediaTek, Huawei/HiSilicon

  - Not support (13): Docomo, LGE, Nokia, OPPO, Apple, Qualcomm, Samsung, Intel, Spreadtrum, CMCC, Xiaomi, CATT, Panasonic,

- Updates on proposal (marked in red) and justifications

  - FL adds a FFS to the 2nd bullet for further study.

- FL’s response to some companies’ comments:

  - @Sharp: From my point of view, cross-COT PSFCH transmission/reception as in your example is allowed, whether the UE use Type 1 LBT and Type 2 LBT to transmit the PSFCH is subject to Channel Access AI agreements.

[M] Proposal 4-2b

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- By default, one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. Other PSFCH occasion(s) within its COT are used for PSSCH transmissions.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | No | We still think the dynamically indicated PSFCH is not feasible. |
| Apple | No | We may keep the second bullet as FFS and agree on the first bullet at this stage. |
| InterDigital | Yes |  |
| Intel | No | We still have same concerns as indicated in first round regarding the feasibility and actual benefits of the dynamic indication of PSFCH, considering also the specification impact that this may have. We would suggest to leave the whole second bullet as an FFS as a compromise. |
| Fraunhofer | Yes | We are ok to compromise on the 2nd bullet, e.g., as FFS. |
| vivo | Yes |  |
| DCM | No | If whole the 2nd bullet is FFS, we are fine with it, and further discussion on collision issue can be done in future meeting. |
| LGE | No | The AGC issue definitely exist. The only way to avoid the AGC is the COT always occupies the all the RB sets in SL BWP or SL carrier (“in RP” is not sufficient since RPs can be FDMed). |
| Qualcomm |  | We support the first bullet point only. As stated in the previous round, the 2nd bullet may have more spec impact, and PSSCH and PSFCH collision needs further study. Hence, suggest to have the entire 2nd bullet for FFS. |
| OPPO | No | We still think dynamic indication on PSFCH occasions are not feasible due to the following-up issues and large specification impact it introduced. We support the first bullet only. |
| NEC | Yes |  |
| Samsung | OK in general | We want to clarify the intention of “By default,” in the first bullet. Can it be removed? For the second bullet, the wording “When COT sharing is applicable,” is confusing, and should be deleted. From the COT initiating point of view, it only needs to provide the indication, and no need to consider whether COT sharing is applicable or not. Also, is there typo in “Other PSFCH occasion(s) within its COT are used for PSSCH transmissions.”? It conflicts with previous wording. |
| Panasonic |  | We support the first bullet, and second bullet should be FFS. |
| Spreadtrum | No | For dynamical indication, many companies have some concerns on AGC issue and PSFCH/PSSCH collision issue. We prefer to keep 2nd bullet as FFS. |
| MediaTek | Yes | For 2nd bullet, i.e., dynamically indicated, we think it is the spirit of the previous agreement made in COT sharing of AI 9.4.1.1 Agreement For UE-to-UE COT sharing, - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator Additionally, we suggest the following modification to avoid ambiguity: When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. Other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions. |
| Lenovo | Yes | We think periodic PSFCH occasions have below drawbacks: When a UE initiates a COT by performing Type-1 channel access procedure, the COT may be frequently interrupted by the periodic PFSCH occasion. This is especially true since the PSFCH period is relatively small, i.e., 1, 2 or 4 slots. When the COT has a duration of more than 4 slots, like COT 3, it will be interrupted twice. This interruption brings below drawbacks: - The Tx UE’s sidelink transmissions can’t be contiguously transmitted in time domain because the Tx UE needs to monitor the channel continuously and perform channel access type 2 before it resumes its sidelink transmission. - Tx UE is quite likely to lose the channel. When the gap is larger than 16us, then there is risk of losing the channel. When the COT is interrupted by one PSFCH occasion and there is no any PSFCH transmitted during the occasion due to NACK-only based feedback or disabled HARQ-ACK feedback, considering one PSFCH occasion occupies 4 consecutive symbols about 280us, the probability of losing the channel is quite high. Figure 6 Periodic PSFCH configuration Hence, we support dynamic indication of PSFCH occasions so as to avoid the ongoing COT frequently interrupted by periodic PSFCH occasions. |
| Sharp |  | For second bullet, if cross-COT PSFCH transmission/reception is allowed, some (pre-)configured PSFCH occasions located in the beginning of COT may be associated to PSSCH outside the COT and may be used for feedback transmission by other UEs. Therefore, we are concerned on the PSFCH occasion handling, i.e., “Other PSFCH occasion(s) within its COT are used for PSSCH transmissions.”. This kind of PSFCH occasion handling can only be performed on some PSFCH occasions by UE only if the UE can make sure there are no PSFCH transmission from other UEs on these PSFCH occasions. We propose the following modification. - When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. Other PSFCH occasion(s) within its COT are used for PSSCH transmissions. - FFS: handling of other PSFCH occasion(s) within its COT. - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them |
| xiaomi | No | We don’t support dynamic indication on PSFCH occasions. All (pre-)configured PSFCH occasions can used to transmit the PSFCH, which has more opportunities than only using the (pre-)configured PSFCH occasions within its COT. |
| ZTE, Sanechips | Only the 1st bullet | Seems no much concern on the first bullet, we should agree this first, FFS whether dynamic indication is needed. |
| Ericsson | OK |  |
| Transsion | Yes |  |
| CATT/GOHIGH | No | Only the first bullet can be accepted. |
| Huawei/HiSilicon | Yes with comments | The motivation to introduce more PSFCH occasions for one PSCCH/PSSCH transmission is to address LBT failure in SL-U. However, within a COT, the probability of LBT failure is largely reduced when PSFCH TX UEs are granted to use type 2 LBT. Introducing more than one PSFCH occasions per PSCCH/PSSCH within a COT is not justified and has a lot of issues, e.g., causing COT interruption in case no HARQ-ACK is needed, reduce resources for PSCCH/PSSCH, etc. How to resolve such issues are unclear so far. Thus, we have concerns on supporting only 1st bullet. |
| Nokia/NSB2 |  | Again, the following part with the first bullet point can be easily agreed: [M] Proposal 4-2b Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings: - By default, one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration - FFS value range of N - FFS detailed design of such N associated candidate PSFCH occasion(s) - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc. - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s) |

### [Closed] Proposals for Thursday GTW (Week 1)

#### [H] Proposal 4-1

Background: PSFCH transmission for 15/30 kHz, OCB requirement

- Summary of previous round

  - Support (19): CMCC, InterDigital, Fraunhofer, vivo, DCM, Qualcomm, OPPO, NEC, Panasonic, Spreadtrum, MediaTek, Lenovo, Sharp, Xiaomi, ZTE, Transsion, CATT, Huawei/HiSilicon, Nokia

  - Not support ():

- Updates on proposal (marked in red) and justifications

  - FL suggests to keep “R16 NR SL PSSCH-PSFCH mapping is reused as baseline”.

    - WID objective clearly says “The existing NR sidelink and NR-U channel structure shall be reused as the baseline.”. So if some design does not reuse legacy NR SL as baseline, it is out of WID scope.

    - In addition, usually FL does not want to have too many examples in the proposal. But for this proposal, such examples are important to be kept to help down-selection.

      - E.g., so far, how Alt 2-3a will do PSSCH-PSFCH mapping is unclear, and it may not reuse legacy NR SL PSSCH-PSFCH mapping and may have the issues mentioned in the examples. The specification impact might be quite large.

      - In next meeting, RAN1 probably “must” down-select one Alt and move forward.

      - So FL encourages proponents of each Alt to give more details on how to resolve the issues in the examples, to help companies clearly know how it works and what’s the specification impact. Otherwise, it’s very hard to do down-selection in next meeting.

      - We all know TU is very limited, and we need to minimize specification impact.

  - For Alt 1-1b:

    - Add the followings based on comment from Docomo and NEC. Let’s FFS this part and companies can bring more details next meeting.

      - “FFS: whether any impact on meeting OCB requirement”

  - For Alt 3-2b:

    - Made the following updates as per ZTE’s comment. The example is removed otherwise the sentence reads strange (i.e., “…common PRBs, … with common PRB”)

      - “FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether they are on the same interlace with common PRB”

- FL’s response to some companies’ comments:

  - @LGE: if there is any RAN4 work, RAN4 will work on it as usual business. FL does not see any critical issue here. RAN1 can also inform RAN4 via LS about our design for PSFCH/S-SSB/etc.

  - @Samsung: so far, not many companies give detailed number for K3 and K4. FL feels it’s hard to agree on detailed value range for K3/K4 in this meeting. But I agree with you that such detailed value range is important to help better understand each Alt. So FL would encourage companies to give more details on this aspect in next meeting.

  - @Lenovo: for Alt 1-1b, the bullet of “multiple CS pairs” is just to reuse legacy NR SL design.

[H] Proposal 4-1c

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB subject to OCB requirement

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether they are on the same interlace with common PRB

- R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details

  - Note: companies are encouraged to give more details and analyze the specification impact

    - E.g., in Alt 2-3a, whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc.

    - E.g., in Alt 2-3a, whether introducing more than 6 CS pairs is needed

    - E.g., for group cast option 2, what’s the maximum group size that can be supported

    - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

  - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

#### [Suspend] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

FL’s comment: this part will be updated later.

### [Closed] Round 3: Proposals and summary of previous round

#### [M] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Summary of previous round

  - Support (11): IDC, Fraunhofer, vivo, NEC, [SS], MTK, Lenovo, [Sharp], Ericsson, HW, Transsion

  - Not support (13): CMCC, Apple, Intel, DCM, LGE, QC, OPPO, Pana, SPRD, Xiaomi, ZTE, CATT, Nokia

- Updates on proposal (marked in red) and justifications

  - Add FFS to the 2nd bullet.

  - Made the following updates based on comment from Samsung.

    - “By default, One PSCCH/PSSCH transmission …”

    - “When COT sharing is applicable, COT initiating UE …”

  - Made the following update based on comment from MediaTek and Sharp.

    - “…FFS: whether other associated candidate PSFCH occasion(s)…”

- FL’s response to some companies’ comments:

  - @Samsung: there is no typo. Previous wording says “…which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions”. So other PSFCH occasion(s) are not available for PSFCH transmission, and are used for PSSCH transmission.

[M] Proposal 4-2c

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- By default, One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- FFS: whether to support that When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

  - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE | OK for progress |  |
| Futurewei | OK |  |
| Apple | OK for progress |  |
| DCM | OK |  |
| ZTE, Sanechips | Yes |  |
| vivo | OK |  |
| OPPO | OK for progress |  |
| NEC | Ok | To clarify, "COT initiating UE can dynamically indicate which …" the dynamically indicate here is still open for explicitly indication or implicitly indication, right? |
| CATT/GOHIGH | Yes |  |
| Transsion | OK for progress |  |
| Spreadtrum | OK |  |
| Lenovo |  | We have strong concern on the COT interruption and COT losing for configured PSFCH occasions. So we support dynamic PSFCH indication. Having said that, we would like to accept this proposal with below added FFS points. - FFS other details - E.g., how to meet the HARQ RTT restriction - E.g., UE behavior on transmitting PSFCH - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions |
| WILUS | OK for progress |  |
| Sharp | Yes |  |
| xiaomi | Yes with comment | We don’t support dynamically indicate the PSFCH occasions. All (pre-)configured PSFCH occasions can be used to transmit the PSFCH, which has more opportunities than only using the dynamically indicated PSFCH occasions within a COT. Moreover, we has spent several meetings to discuss whether to support dynamically indicate the PSFCH occasions. Considering the limited meeting time，we needs make a down selection rather than achieve a FFS. So we suggest to remove the following FFS part: - FFS: whether to support that When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions. - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them |
| CMCC | OK for progress |  |
| Panasonic | OK |  |
| MediaTek | OK for progress | We are OK for the current Proposal. Just a clarification for the 1st FFS of the 2nd bullet. From our understanding, this FFS is only available in the case that the HAQR transmission on dynamically indicated subset of the (pre-)configured PSFCH occasions within its COT is successful. If so, we may want to add a limit over the current statement to avoid ambiguity. - FFS: whether to support that When COT sharing is applicable, COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions if the HARQ transmission on the dynamically indicated subset of the (pre-)configured PSFCH occasions within its COT is successful. |
| Huawei/HiSilicon | See comments | As indicated in last round, the motivation to introduce more PSFCH occasions for one PSCCH/PSSCH transmission is to address LBT failure in SL-U. However, within a COT, the probability of LBT failure is largely reduced when PSFCH TX UEs are granted to use type 2 LBT. Introducing more than one PSFCH occasions per PSCCH/PSSCH within a COT is not justified and has a lot of issues, e.g., causing COT interruption in case no HARQ-ACK is needed, reduce resources for PSCCH/PSSCH, etc. We do not see the necessity to introduce more than one PSFCH occasions for one PSCCH/PSSCH transmission within a COT. Also, how to resolve such issues are unclear so far. Thus, we have concerns on supporting only 1st bullet. |
| Nokia/NSB | OK |  |
| Qualcomm | OK |  |
| Intel | OK | We can accept this proposal as a compromise. |
| Samsung | OK |  |

### Round 4: Proposals and summary of previous round

#### [M] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Summary of previous round

  - Almost all companies are fine with it.

- Updates on proposal (marked in red) and justifications

  - Add the following based on comment from Lenovo.

    - “E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions”

  - Add the following based on comment from MediaTek. @MediaTek: FL feels no need to add too many details now. Your mentioned case is open for discussion.

    - “FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios.”

- FL’s response to some companies’ comments:

  - @NEC: since the whole 2nd bullet is FFS, your mentioned case is also open for discussions.

  - @Xiaomi: the 2nd bullet is FFS, FL assumes this is already a compromise. Otherwise, FL assumes this proposal cannot be agreed in this meeting.

[M] Proposal 4-2e (PSFCH, location)

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

  - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

  - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

FL’s comment: this proposal is near stable, let’s discuss by email directly.

## Issue#5: S-SSB and synchronization

### [Closed] 1st round: Proposals and background

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement or OCB exemption

- Summary

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - Support (10): Ericsson, LGE, Samsung, ZTE, Futurewei, Spreadtrum, Sony, InterDigital, Transsion Holdings, Lenovo

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - Support (12): Samsung, CATT, MediaTek, OPPO, vivo, Panasonic, Sharp, Huawei/HiSilicon, Qualcomm, Lenovo, Xiaomi, National Spectrum Consortium, WILUS

  - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

    - Support (8): Qualcomm, Intel, Apple, Futurewei, OPPO, Huawei/HiSiliconm, Nokia, [Sharp] , WILUS

- FL’s view

  - There is no clear majority view. FL gave a proposal below to reflect the possible down-selection alternatives.

  - There are some good evaluation results from companies. E.g., Nokia/MediaTek/etc. points out that Option 1-1 may have worse correlation performance compared with legacy S-SSB through evaluation. Companies are encouraged to check.

  - FL would like to remind the group that UE implementation complexity will be high (i.e., UE needs to implement multiple S-SSB detection algorithms), if RAN1 supports multiple S-SSB transmission schemes. So a unified design to support all scenarios may be preferred.

    - E.g., to resolve the FFS under Option 1-1, or to meet the minimum 2 MHz requirements under 15 kHz SCS for Option A, the S-SSB transmission scheme might be different, thus increasing UE implementation complexity.

  - Companies also gave detailed design for each Option

    - E.g., how to resolve the FFS under Option 1-1

    - E.g., how to reduce PAPR under Option 3-1, value of N and gap

    - E.g., how to meet the minimum 2 MHz requirements under 15 kHz SCS for Option A

    - FL suggests to come back to these details later after down-selection.

[H] Proposal 5-1

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 3: Support Option A only

  - Alt 4: Support both Option 1-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

| Company | Which of Alt 1~5 do you support? | Comments |
| --- | --- | --- |
| DCM | Alt 1 or Alt 2 | Single structure should be supported. |
| vivo | Alt2 | In our view, the number of S-SSB repetition in frequency domain is configurable in option 3-1. Thus, if N=1 is supported by option 3-1, Option A is supported. By this way we are also fine with Alt 5. However, in this case sub-bullet of option A should be removed (i.e., no need of further enhancement to meet the minimum 2 MHz requirements under 15 kHz SCS). |
| LGE | Alt 1 or Alt 4(with comment) | If the Option A is replaced with Rel-16/17 NR S-SSB structure, then we are Ok with Alt 4 as well. In this case, when the OCB requirement is not present, Option A can be used. In this case, it does not need to consider 2MHz requirement. In case of Option 3-1, the feasibility of this non-contiguous transmission needs to be checked based on UE capability or RAN4 perspective. If we go with Option 3-1, since we concern about the PSD requirement, it should have large number of S-SSB repetition rather than two S-SSB at the edge of the RB set. For instance, for 15kHz SCS, if the two S-SSB repetition is used, the maximum transmit power will be limited to 16dBm for the case of 10dBm/1MHz. |
| Nokia/NSB | Alt 3 | As discussed in our Tdoc option 1-1 is not a good choice to meet OCB requirement. Option 3-1 or some other modifications to legacy S-SSB can be considered. |
| OPPO | Comment | We think Option 3-1 and Option A can be supported, and Option A can be a standalone method regardless option 3-1 while Option A is only applicable to SCS 15KHz. So, we support Option 3-1 and Option A, and they can be enable independently. |
| Interdigital | Alt. 1 | One structure (e.g., interlace) can be supported for simplicity. |
| Apple | Alt 3 | Option A is the simplest solution. |
| NEC | Alt.1 | Option 3-1 (Alt.2) has higher PAPR/cubic metric issue |
| Ericsson | Alt 1 |  |
| Qualcomm | Alt 5 | Alt A does not necessarily require OCB exemption. In some region/band, there is no OCB requirement. Option 1-1 and option A share the same legacy 11-RB S-SSB part, one can reuse the legacy Rx UE searcher and the performance/complexity is well understood. |
| Samsung |  | We object Alt 3 and open to other alternatives (with the following clarification to Option A). To further clarify, Option A shall be described as “legacy S-SSB structure” and whether it’s due to OCB exemption or not doesn’t need to be part of the proposal (i.e., doesn’t need to be subject to configurability). For example, legacy S-SSB structure can be used for the case without OCB requirement, and it’s not related to OCB exemption at all. The following is the suggested wording change for Option A: - Option A: legacy S-SSB structure - Note: for the case with OCB requirement, if only Option A is supported, OCB exemption is applied, and continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. |
| Sharp | Alt 2 or Alt 5 |  |
| Intel | Alt-3 | Option A may be sufficient, but we are open to option 5 as a compromise to help moving forward. |
| Futurewei | OK | Alt 1/Alt 3 |
| Lenovo | Alt 1 or Alt 2 |  |
| Spreadtrum | Alt 1 or Alt 4 |  |
| CMCC | Alt 1 |  |
| xiaomi | Alt 2 | Alt2 has benefit on the simplicity. |
| CATT/GOHIGH | Alt 5 | We share the same view as QC |
| Panasonic | Alt 2 or Alt 5 |  |
| Transsion | Alt 1 |  |
| MediaTek | Alt 2 | A unified S-SSB design for all SCS of 15/30/60 kHz is preferred with consideration of spec effort and complexity. Therefore, Alt 1 is not supported due to the uncertainty of interlace utilization, unavailable for 60kHz SCS. In additional, the degraded correlation performance of Alt 1 (as shown in our contribution) can not be accepted from our side. Additionally, we support Vivo’s opinion that Alt 3 can be achieved by configuring N = 1 in Alt 2. |
| ZTE | Alt 1 | Single structure should be supported for simplicity. |
| Huawei/HiSilicon | Alt 2 | For option 1-1, the receiving SNR will be worse for the higher 20MHz noise bandwidth. Furthermore, option 1-1 requires 20MHz bandwidth to do timing detection at least during the sync source searching stage, resulting in higher detection power assumption. While option 3-1, the legacy S-SSB receiver can be reused on the S-SSB synchronization raster, the similar performance as Rel-16 S-SSB can be expected. Hence, we think option 3-1 is enough. After the option 3-1 is applied, S-SSB OCB issue can be solved for all cases, no need to further discuss the other solutions. A single option 3-1 applied to all cases can further simplify the implementation both for S-SSB transmitter and receiver. |
| WILUS | Alt 5 or Alt 2 |  |

#### [H] Proposal 5-2

Background: additional S-SSB occasions, belong to resource pool or not

- Summary

  - Additional candidate S-SSB occasions are excluded from resource pool

    - Support (15): Ericsson, OPPO, Docomo, Samsung, LGE, Intel, Apple, NEC, CATT, WILUS, MediaTek, Xiaomi, Futurewei, National Spectrum Consortium, Spreadtrum

  - Additional candidate S-SSB occasions belong to resource pool

    - Support (11): Qualcomm, Nokia, Intel, vivo, ZTE, Lenovo, Huawei/HiSilicon, InterDigital, Panasonic, Futurewei, Transsion Holdings

- FL’s view

  - Companies who support “exclude from resource pool” mainly want to follow legacy NR SL design and keep the design simple.

  - Companies who support “belong to resource pool” mainly want to reduce resource overhead.

    - Qualcomm has simulation results to show “exclude from resource pool” will cause 10~20% UPT loss. Vivo has very detailed overhead analysis. Companies are encouraged to check.

    - Some companies (Qualcomm, Vivo, Nokia, etc) point out S-SSB and PSCCH/PSSCH overlapping can be avoided with proper CPE design, thus this is not an issue.

  - FL suggests to use (pre-)configuration to enable one of them.

  - The following proposal is given to reflect above.

[H] Proposal 5-2

Resource pool level (pre-)configuration enables one of the followings:

- Alt 1: Additional candidate S-SSB occasions are excluded from resource pool

- Alt 2: Additional candidate S-SSB occasions belong to resource pool

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | Motivation of Alt 1, i.e., simple design and small workload, disappears by (pre-)configurability. |
| vivo | OK |  |
| LGE | No | Only Alt 1 is supported. It is difficult to understand the relation with Proposal 5-1. As we know, S-SSB will be transmitted in SFN manner, then if S-SSB repetition is supported, how to avoid the collision between S-SSB repetition by a UE and interlaced PSCCH/PSSCH transmission by another UE? If the UE still can transmit S-SSB to maintain the COT, why the addition S-SSB occasion need to be included in the resource pool? |
| Nokia/NSB | No | Additional S-SSB occasions should belong to RP. Overhead is the problem but collisions between S-SSB and PSSCH are not because they can be avoided with proper CPE design. |
| OPPO | No | All of S-SSB occasions should be excluded from RP, because the half-duplex issue cannot be ignored. Tx overlapping between S-SSB and PSCCH/PSSCH dramatically degrade the sync. performance of SL system. Considering about the S-SSB overhead, we don’t think it is a critical issue that needs to be addressed, while the periodicity is 160ms and only a tiny partial of the slots are used for S-SSB which is to guarantee the synchronization procedure. |
| Interdigital | Yes | Our preference is Alt. 2 but we can compromise to support configuration. |
| Apple | No | We prefer Alt. 1 to have a clean design. The S-SSB overhead can be controlled by configuring the number of additional slots for S-SSB transmissions. |
| NEC | No | Alt.1 only. S-SSB within a resource pool will have larger impacts on many legacy procedures, we don’t think a configuration between this two options is a good WF. |
| Qualcomm | Yes | Ok with the proposal |
| Samsung | No | We don’t support Alt 2. Alt 2 may have significant specification impact since it changes the fundamental design of resource pool that never includes S-SSB transmission. The amount of extra needs to be clarified. Also, as mentioned several times already, it’s not reasonable to fully up to the receiver UE to blind detect the transmission type, which is an unnecessary burden to support Alt 2. Regarding the issue of “waste of resource” in Alt 1, the number of slots for S-SSB transmission is already configurable, so if one is worried about the overhead. |
| Sharp | No | We prefer Alt 1. |
| Intel | Yes | Ok with the compromised solution. |
| Futurewei | OK |  |
| Lenovo | Yes | Alt 2 is preferred due to performance considerations. But we can support configuration as a compromise to get us forward. |
| Spreadtrum | No | Only prefer Alt 1. |
| CMCC | No | Alt 1 only. Because we should further consider the half-duplex issue and the Tx limitation when S-SSB and PSCCH/PSSCH are transmitted in a same slot. |
| xiaomi | Yes | We prefer alt1, because alt 1 reuses the Rel-16/17 sidelink design to avoid other sidelink signals interfering the S-SSB’s transmission. |
| CATT/GOHIGH | No | Only alt 1 is supported. Since the synchronization is the most important thing for sidelink communication, hence we prefer that the additional S-SSB occasions should be excluded from resource pool. |
| Panasonic | Yes | Our preference is alt 2. We can compromise to support configuration. |
| Transsion | Yes | We can accept this proposal as a compromise. |
| MediaTek | No | Only Alt 1 is supported consideration the spec effort and complexity. Additionally, if additional candidate S-SSB occasion(s) belong to resource pool, blind detection issue for S-SSB will be additionally introduced on the basis of the blind detection of control signal, which may impose additional blind detection complexity on UE. As for the spectrum efficiency, which can be solved/relaxed by a proper configuration on the number of additional candidate S-SSB occasion(s) (e.g., each R16/R17 S-SSB has one additional candidate S-SSB occasion). |
| ZTE | Yes with comments | (pre-)configuration better to be carrier/BWP level rather than RP level. |
| Huawei/HiSilicon | Yes | We prefer Alt 2 to reduce the S-SSB system overhead. This issues will impact the configuration of the S-SSB resources and available resources for data transmission. To make agreement on this issue as soon as possible, we can compromise to the above FL proposal. |
| WILUS | No | We prefer Alt.1 only. S-SSB within a resource pool will have larger impacts on many legacy procedures |

#### [M] Proposal 5-3

Background: Tx UE behaviour on additional S-SSB occasions

- Summary

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

    - Support (7): Huawei/HiSilicon, Nokia, [Apple], OPPO, NEC, InterDigital, Lenovo

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

    - Support (6): Qualcomm, Ericsson, LGE, Spreadtrum, Intel (if not belong to RP), MediaTek

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

    - Support (5): Qualcomm, Samsung, Docomo, National Spectrum Consortium, [ZTE]

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions

    - Support (2): Intel (if belong to RP), WILUS

- FL’s view

  - Considering the situation, FL suggests to support both Alt 1 and Alt 2, and enable one of them by (pre-)configuration.

  - The following proposal is given to reflect above.

[M] Proposal 5-3

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, (pre-)configuration enables one of the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | If both Alt 1 and Alt 2 need to be supported, Alt 3 = Alt 1 or Alt 2 by UE implementation is better than semi-static determination. |
| vivo | comment | We are basically OK to accept this way forward as compromise, but would like to know the relation with proposal 5-2. In our view, if P5-2 is agreed, then Alt-1 of P5-3 is used if Alt-2 of P5-2 is enabled, while similarly Alt-2 of P5-3 is used if Alt-1 of P5-2 is enabled. In other words, the number of combinations to be implemented should be limited. |
| LGE |  | We support Alt 2, but for progress we are also fine with Alt 3 as well. I’d like to understand the relation with Proposal 5-5. In Proposal 5-5, it try to transmit S-SSB to maintain the COT. Then, Alt 1 seems like to go opposite direction since it will not transmit S-SSB when it success to transmit even though the additional S-SSB occasion is in the middle of COT. |
| Nokia/NSB | Comment | RAN1 should first decide if additional S-SSB occasions belong to resource pool or not or if (pre-)configuration is used to determine if S-SSBs belong to resource pool. Our preference is Alt 1. |
| OPPO | Yes | We prefer Alt 1. |
| Interdigital | OK |  |
| Apple |  | Our preference is Alt. 4, but we can accept Alt 1 to avoid redundant S-SSB transmissions. |
| NEC | OK | Actually we don't see much benefit differences between options, thus this is a good compromise. |
| Qualcomm | Yes | Alt 2 is preferred |
| Samsung | No | We need to finalize Proposal 5-2 first and see whether there is a need to distinguish S-SSB occasion in legacy Rel-16/17 or extra S-SSB occasion. The UE behavior in Alt 1 and Alt 2 is too restricted. Why the UE has to transmit S-SSB in the earlier occasions? To us, it’s fully up to implementation, which is Alt 3. |
| Intel | No | If proposal 5-2 is agreed, then a different behavior is needed depending on whether the additional S-SSB occasions are pre-configured to be excluded or included in the resource pool. - When the additional candidate S-SSB occasions belong to the resource pool, Alt 4 could be used as this mimics the principles of Rel.16 NR-U and allows to indeed optimize the spectrum utilization, which is the main advantage of configuring the additional candidate S-SSB occasions within a resource pool. In fact, in this case it seems unnecessary to actually transmit on multiple occasions even if LBT has succeeded in a prior one, and transmission was successfully done as these resources could be used by other UEs. When the additional candidate S-SSB occasions are excluded from the resource pool, Alt 2 could be used as is this case the whole resources would not be utilized for any other purposes. |
| Futurewei | OK | OK with both alternatives. Slightly prefer Alt 1. |
| Lenovo | No | Only Alt 1 is supported. |
| Spreadtrum | OK | We can accept this proposal. |
| CMCC | OK | Alt 1 |
| xiaomi | Yes | We perfer alt2. we think it is not necessary to distinguish the legacy SSB occasion and the additional SSB occasion. |
| CATT/GOHIGH | No | Alt 3 shares similar supported companies as alt1/2, so alt 3 can also be (pre-)configured as a candidate option. Besides, from the perspective of synchronization performance, the total number of successful S-SSB transmissions doesn’t need to exceed the number of S-SSB occasions defined in R16/R17 NR SL |
| Panasonic |  | Proposal 5-2 should be concluded first. When the additional candidate S-SSB occasions are included in the resource pool, Alt 1 could be used. When the additional candidate S-SSB occasions are excluded from the resource pool, Alt 2 should be used. |
| MediaTek | Yes | Alt 2 is preferred and as commented by some other companies, postpone Proposal 5-2 after there is an agreement on Proposal 5-2 is also acceptable from our side. |
| ZTE | Yes | We prefer Alt 1. |
| Huawei/HiSilicon | Yes | We prefer Alt 1. |
| WILUS | No | Only Alt 1 is supported. |

#### [M] Proposal 5-4

Background: number/location of additional candidate S-SSB occasions

- Summary

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

    - Support (6): Intel, LGE, Docomo, ZTE, OPPO, Spreadtrum

  - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

    - Support (6): Huawei/HiSilicon, CATT, Apple, vivo, NEC, InterDigital

  - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

    - Support (4): Intel, Nokia, vivo, Lenovo

  - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

    - Support (3): Qualcomm, Xiaomi, NEC

  - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

    - Support (3): Samsung, Xiaomi, WILUS

- FL’s view

  - Option 1, 2 are straightforward.

  - Option 3 has more signaling than Option 1, 2.

  - Some companies (Huawei/HiSilicon, CATT, etc.) point out Option 4 may face continuous LBT failure since WiFi may occupy the COT for several milliseconds.

  - Some companies (CATT, etc.) point out the details of Option 5 is unclear.

  - FL suggests to down-select between Option 1 and 2.

    - FL added “…in different time slot(s)…” in Option 2 to avoid misunderstanding as mentioned by Intel (i.e., multiple S-SSBs in the same slot).

  - The following proposal is given to reflect above.

[M] Proposal 5-4

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#112b-e:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | OK | Support Option 1 |
| vivo | Comment | Regarding option 3, additional RRC signaling seems not to be an issue. On the other hand, we would like to understand better on option 1. Does option 1 mean changing the existing S-SSB configuration signaling? If yes, we have concern that it affects the legacy signaling as well as licensed band S-SSB transmission. If no (e.g., a new R18 parameter is introduced dedicated for unlicensed band), it also introduces new signaling (similar as option 3), but with less flexibility. Then, what is the benefit of option 1 over option 3? It seems more reasonable to down-select between option 2 and option 3. |
| LGE | Yes | We support Option 1 for simplicity. |
| Nokia/NSB | comment | We think that option 3 will be selected if no optimization for signaling can be agreed. Regarding these two options we prefer option 2. |
| OPPO | Yes | We support Option 1. |
| Interdigital | OK | We support Option 2. |
| Apple | Yes | We support Option 2. |
| NEC | Yes | Option 2 |
| Qualcomm |  | Ok with option 2 if the gap can be preconfigured to be zero |
| Samsung | No | In our understanding, we need to distinguish two RRC parameters: number of S-SSB candidate and number of transmitted S-SSBs. The legacy RRC parameter sl-NumSSB-WithinPeriod is for which one in Option 1? If there is no distinguish, Option 1 seems bounded with Alt 2 in Proposal 5-3? |
| Sharp | Yes | We think down-selection between option 1 and option 2 depends on Tx UE behavior on additional S-SSB occasions. If additional S-SSB is transmitted regardless of whether R16/R17 S-SSB is transmitted, option 1 is a simple solution. |
| Intel | Yes | We prefer option 1. |
| Futurewei | OK | Either option is OK for us. |
| Lenovo | No | Previous option 3 should be included in the down-select list. |
| Spreadtrum | Yes | We prefer option 1. |
| CMCC | Yes | Option 2. |
| xiaomi | No | We suggest to add a sub-bullet under the option1, “The interval between two adjacent S-SSB can be preconfigured to value 0”. Because the contiguous S-SSB configuration has larger channel access opportunity, so we make the following revision: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - The interval between two adjacent S-SSB shall be fixed to value 0 - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. |
| CATT/GOHIGH | Yes | Support option 2. |
| Panasonic | Yes | We support option 2. |
| MediaTek | Comment | While our preference is Option 4, which seems not captured in the FLS. Considering the progress, we can accept Option 2 with an additional FFS. - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - FFS gap length of 0 |
| ZTE | OK | Support Option 1 |
| Huawei/HiSilicon | Yes | We support Option 2. |
| WILUS | Yes | We support option 2. |

#### [L] Proposal 5-5

Background: more than one S-SSB on multiple RB sets

- Summary

  - When the COT contains multiple RB sets and includes S-SSB slot(s), UE transmits S-SSB in each RB set

    - Support (9): Ericsson, Apple, CATT, Futurewei, Spreadtrum, InterDigital, Panasonic, Huawei/HiSilicon, OPPO

    - Not support (5): ZTE, LGE, Intel, MediaTek, Sharp

- FL’s view

  - FL lists two Alt to reflect the situation.

[L] Proposal 5-5

When the COT contains multiple RB sets and includes S-SSB slot(s), down-select one of the followings:

- Alt 1: UE transmits S-SSB repetition in each RB set of the COT

- Alt 2: UE only transmits S-SSB in one RB set as per legacy ARFCN configuration

FL’s comment: please also indicate which Alt you support.

| Company | Agree? | Comments |
| --- | --- | --- |
| vivo | OK | We support Alt 1. Moreover, we don’t think the prerequisite is needed (i.e., When the COT contains multiple RB sets). We think it should be revised as: When the COT BWP contains multiple RB sets and includes S-SSB slot(s) |
| LGE |  | I’d like to understand the relation with Proposal 5-2. In Proposal 5-2, in Alt 1, the UE will not transmit S-SSB on the additional occasion if it success to transmit before. It seems that they are contradicted each other. Some resolving would be necessary. Moreover, if it targets non-contiguous S-SSB transmissions, its feasibility needs to be checked first. It would be up to RAN4. |
| Nokia/NSB | Alt 2 |  |
| OPPO | OK | In order to maintain the COT which contains multiple RB sets, S-SSB should be transmitted by occupying all the RB sets of the COT. |
| Interdigital | Yes | We support Alt. 1. |
| Apple | Yes | We support Alt. 1. |
| NEC | Yes | Alt.1 |
| Qualcomm | OK | Alt 1 is preferred and the S-SSB repetition is meant for COT padding but not for synchronization |
| Samsung | OK | For Alt 1, how does the UE know the multiple frequency location of the S-SSB transmissions? |
| Intel | OK | We prefer Alt.2. We really do not understand why “COT padding is needed”: per our understanding the ETSI BRAN does not provide any restrictions in frequency domain on where a responding device should transmit to keep a shared COT. For instance, if this is true then wouldn’t this imply that the COT sharing in FDM over multiple RB sets is not allowed as each UE should be required to transmit over all RB sets over which the initiating device has acquired the COT? |
| Futurewei | OK | We prefer that in Alt 1 to be “UE may transmit S-SSB repetition in each RB set of the COT”, such formulation is complementary to Alt 2, and allow UE to judge the need of transmitting S-SSB in all RB sets. For instance, if the transmission is at the end of COT there is no necessary to be transmitted in each RB set to maintain the COT. |
| Lenovo | Yes | Alt. 1 is supported. |
| Spreadtrum | Yes | We support option 1. Similar issues may also exist for PSSCH and PSFCH transmission within the COT containing multiple RB sets. It should be also discussed. |
| CMCC | OK | Alt 1 |
| xiaomi | Yes | We prefer alt1, because alt1 can increase the performance of SSB detection. |
| CATT/GOHIGH | Comment | We respect the motivation of proposal 5-5 that is used to maintain COT, so additional transmission can be performed. But from our understanding, such additional transmission can be dummy signal transmission or repetitive S-SSB transmission. |
| Panasonic | OK | If the transmission of S-PSS/S-SSS/PSBCH N times by repetition in frequency domain is supported in an RB set, multiple transmission of S-SSB repetitions in more than one RB set could be also supported. |
| MediaTek | Yes | Alt 2 is preferred. For Alt 1, we have some concerns on the power splitting issue if S-SSB is repeated in more than one RB set considering the max Tx power of UE is limited. For example, max of 23 dBm for one RB set S-SSB transmission. While for S-SSB repeated in multi-RB sets (e.g.,4 RB sets), the total power will be split into 4 parts, each of 17 dBm. |
| ZTE | OK | Support Alt 2 |
| Huawei, HiSilicon | ok | We support FL’s proposal. We support Alt1 to avoid the COT lost, especially for the RB sets which does not include the S-SSB ARFCN. |
| WILUS | Yes | We support Alt. 1. |

#### Others

- Some other issues are mentioned:

  - Some companies (e.g., ZTE, etc.) mentioned RAN1 needs to discuss how to determine the frequency location of S-SSB.

- FL assumes such issues are not very urgent at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

### [Closed] 2nd round: Proposals and summary of previous round

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement or OCB exemption

- Summary of previous round

  - Alt 1: Support Option 1-1 only

    - Support (11): DCM, LGE, InterDigital, NEC, Ericsson, Futurewei, Lenovo, Spreadtrum, CMCC, Transsion, ZTE

  - Alt 2: Support Option 3-1 only

    - Support (9): DCM, vivo, Sharp, Lenovo, Xiaomi, Panasonic, MediaTek, Huawei/HiSilicon, WILUS

  - Alt 3: Support Option A only

    - Support (4): Nokia, Apple, Intel, Futurewei

  - Alt 4: Support both Option 1-1 and Option A, and enable one of them by (pre-)configuration

    - Support (2): LGE, Spreadtrum

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

    - Support (6): OPPO, Qualcomm, Sharp, CATT, Panasonic, WILUS

- Updates on proposal (marked in red) and justifications

  - Based on the situation, FL suggests to drop Alt 3 and 4.

  - Vivo, LGE mentioned for Option A, no need to further consider how to meet 2 MHz requirement.

  - FL added Option B and Alt 6 below. In Option B, since only 30 kHz is supported, RAN1 can reuse legacy S-SSB and no additional work is needed.

  - FL would like to remind the group again that UE implementation complexity will be high (i.e., UE needs to implement multiple S-SSB detection algorithms), if RAN1 supports multiple S-SSB transmission schemes. So a unified design to support all scenarios may be preferred.

    - E.g., to resolve the FFS under Option 1-1, or to meet the minimum 2 MHz requirements under 15 kHz SCS for Option A, the S-SSB transmission scheme might be different, thus increasing UE implementation complexity.

[H] Proposal 5-1b

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 3: Support Option A only

  - Alt 4: Support both Option 1-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

  - Option B: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH for 30 kHz

    - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS.

| Company | Which of Alt 1,2,5,6 do you support? | Comments |
| --- | --- | --- |
| CMCC | Alt 1 |  |
| Apple | Alt 5 |  |
| InterDigital | Alt 1 |  |
| Intel | Alt 5 |  |
| vivo | Alt 2/Alt 6 | Just for clarification on Option B, our understanding is that it allows by configuration to enable OCB exemption, but whether such exemption can be applicable is up to the regulation. |
| DCM | 1/2/6 | We are fine with Option B if no additional work is necessary. |
| LGE | Alt 1 | If we go with Alt 2, we should focus on contiguous resource or almost contiguous resource to minimize RAN4 specification work. We think that for SL carrier without OCB requirement, Rel-16/17 NR S-SSB structure can be directly reused. |
| Qualcomm | Alt 5 | Alt A does not necessarily require OCB exemption and should be clarified. In some region/band, there is no OCB requirement. Suggest to update option A as following Option A: Apply OCB exemption to all of Legacy S-PSS/S-SSS/PSBCH Note: Applicable in the region/band with no OCB requirement, or with OCB exemption Option 1-1 and option A share the same legacy 11-RB S-SSB part, one can reuse the legacy Rx UE searcher and the performance/complexity is well understood. |
| OPPO | Alt 5 |  |
| NEC | Alt.1 |  |
| Samsung | Alt 1, Alt 2, or Alt 5 (with change to Option A) | We believe our comment in the previous round is not addressed. “Apply OCB exemption” doesn’t need to be in the wording of the option if it’s configurable. There are cases the legacy S-SSB can be used without applying OCB requirement. - Option A: legacy S-SSB structure Note: for the case with OCB requirement, if only Option A is supported, OCB exemption is applied, and continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. |
| Panasonic | Alt 2 and Alt 5 |  |
| Spreadtrum | Alt 1 |  |
| MediaTek | Alt 2/5/6 | For OCB exemption, we share similar view as Vivo even it can be (pre-)configured, whether it is applicable depends on the regulation. |
| Lenovo | Alt 1 or Alt2 |  |
| Sharp | Alt 2 or Alt 5 |  |
| WILUS | Alt 5 (with modification of Option A) | Option A: Apply OCB exemption to all of Legacy S-PSS/S-SSS/PSBCH |
| xiaomi | Alt 2 |  |
| ZTE, Sanechips | Alt 1 |  |
| Ericsson | Alt 1 |  |
| Transsion | Alt 1 |  |
| CATT/GOHIGH | Alt 2/6 |  |
| Huawei, HiSilicon | Alt 2 | We prefer a single solution, to reduce UE implementation complexity. |
| Nokia/NSB2 | Alt5 |  |

#### [H] Proposal 5-2

Background: additional S-SSB occasions, belong to resource pool or not

- Summary of previous round

  - Support (11): vivo, InterDigital, Qualcomm, Intel, Futurewei, Lenovo, Xiaomi, Panasonic, Transsion, ZTE, Huawei/HiSilicon

    - Some companies prefer Alt 2 (i.e., belong to RP), but can accept the proposal as a compromise.

  - Not Support (13): DCM, LGE, Nokia, OPPO, Apple, NEC, Samsung, Sharp, Spreadtrum, CMCC, CATT, MediaTek, WILUS

    - Most of these companies support Alt 1 only (i.e., excluded from RP)

    - Nokia support Alt 2 only (i.e., belong to RP).

- Updates on proposal (marked in red) and justifications

  - So far, no updates are made.

  - Companies are encouraged to suggest other way forwards.

  - ZTE suggests to consider carrier/BWP level (pre-)configuration, rather than RP level. Companies can express their view in this round.

- FL’s response to some companies’ comments:

  - @LGE: On PSSCH/S-SSB collision, some companies mentioned this can be avoided by CPE design, e.g., Nokia.

[H] Proposal 5-2

Resource pool level (pre-)configuration enables one of the followings:

- Alt 1: Additional candidate S-SSB occasions are excluded from resource pool

- Alt 2: Additional candidate S-SSB occasions belong to resource pool

FL’s comment: please also indicate which granularity is supported and why, e.g., resource pool/SL-BWP/carrier-level.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes | At least resource pool level (pre)configuration can configure additional candidate S-SSB occasions are excluded from resource pool is a little bit strange, we prefer BWP/carrier-level configuration. |
| Apple | No | We still support Alt 1 only, and no need to discuss the configuration granularity. |
| Interdigital | Yes | The configuration should be applied to SL BWP to have unified UE procedure in all resource pools of the BWP. |
| Intel | Yes | We are OK with the proposal as a compromise, while our first preference is for option 1. |
| vivo | Yes | We are OK with either per-BWP/carrier or per-RP. We understand the point that having the same behavior to all the pools is easier, but on the other hand, including/excluding the S-SSB occasions is done during logical slot determination step for each RP, then it seems natural to have this configuration per RP. |
| DCM | No | This is not a compromise for Alt 1 supporters. If it is clarified that no additional work is pursed for Alt 2 in this proposal (while how to handle S-SSB vs PSCCH/PSSCH/PSFCH conflict is quite unclear), we accept it. |
| LGE | No | According to the previous discussion, when we do not support FDM between S-SSB and PSCCH/PSSCH by using different CPE, why Alt 2 is needed? Moreover, since the RX UE does not know whether S-SSB is dropped or not, it will increase false alarm. In addition, in Alt 2, how to avoid the AGC issue for the case when S-SSB is FDMed with PSCCH/PSSCH/PSFCH? |
| Qualcomm | Yes | Ok with the proposal |
| OPPO | No | Only Alt 1 is feasible by excluding all S-SSB occasions from the resource pool. A resource pool containing S-SSB slots changes the legacy SL principle much, which leads to performance degradation on both synchronization and communication. |
| NEC | No | Only ALT.1. The design of SL-U should firstly consider existing sidelink baseline except when serious problem occurs. We don’t see the problem to exclude S-SSB from RP as SL is always doing so |
| Samsung | No | Without a clear understanding on what further specification impact is needed and how to avoid blind detection at the receiver UE side for Alt 1, we cannot accept the proposal. |
| Panasonic | Yes |  |
| Spreadtrum | No | We still perfer Alt 1 only. Alt 2 needs large workload to discuss how to handle S-SSB and PSCCH/PSSCH/PSFCH transmission conflict. |
| MTK | No | Only Alt 1 is supported and the configuration granularity of either resource pool or carrier/BWP is OK from our side. For Alt 2, in addition to the issues proposed by other companies, the following issue is also unclear for us. Considering we are going to agree that a resource pool is (pre-)configured with either CRB-based or IRB-based PSCCH/PSSCH transmission. If Alt 2 is supported, does that mean there will exist two mechanisms (IRB and CRB) for PSCCH/PSSCH and S-SSB transmissions within one resource pool? For example, IRB for PSCCH/PSSCH while CRB for S-SSB within one resource pool. Or does that mean the mechanism (IRB and CRB) will be bound together for PSCCH/PSSCH and S-SSB transmissions? Both cases seem unreasonable from our side. |
| Lenovo | Yes | Either per-BWP/carrier or per-RP can be supported. Per-BWP/carrier can align UE behaviors across all RPs, while per-RP provides more flexibility. |
| Sharp | No | We share same views with LGE and we support Alt.1 only. |
| WILUS | Yes | We are OK with the proposal as a compromise, while we prefer to have Alt 1. |
| xiaomi | Yes |  |
| ZTE, Sanechips | Alt 2 | we support SL BWP level for such configuration as legacy S-SSB. Moreover, the Alt 2 wording needs to be changed to align with Alt 1 as below because even if the S-SSB occasions are not excluded as per 38.214, the bitmap can be set as 0 as well. - Alt 2: Additional candidate S-SSB occasions are not excluded from resource pool |
| Ericsson | No | This departs significantly form previous releases. They cannot be part of the pool. |
| Transsion | Yes |  |
| CATT/GOHIGH | No | Only alt 1 is supported |
| Huawei, HiSilicon | Yes | We prefer Alt 2 to reduce the S-SSB system overhead. |
| Nokia/NSB2 | No | Our preference is Alt 2. In Alt 1 overhead is the problem. Potential collisions between S-SSB and PSSCH in Alt 2 can be avoided with proper CPE design. Because of this we think that only Alt 2 is needed. |

#### [Suspend] Proposal 5-3

Background: Tx UE behaviour on additional S-SSB occasions

FL’s comment: Since the decision of Proposal 5-2 may impact Proposal 5-3 and 5-4, let’s come back to this later.

#### [Suspend] Proposal 5-4

Background: number/location of additional candidate S-SSB occasions

FL’s comment: Since the decision of Proposal 5-2 may impact Proposal 5-3 and 5-4, let’s come back to this later.

#### [L] Proposal 5-5

Background: more than one S-SSB on multiple RB sets

- Summary of previous round

  - Support Alt 1 (16): vivo, OPPO, InterDigital, Apple, NEC, Qualcomm, Samsung, Futurewei, Lenovo, Spreadtrum, CMCC, Xiaomi, [CATT], Panasonic, Huawei/HiSilicon, WILUS

  - Support Alt 2 (4): Nokia, Intel, MediaTek, ZTE,

- Updates on proposal (marked in red) and justifications

  - Based on the situation, FL suggests to go with Alt1.

  - Add “FFS details, e.g., location of such S-SSB repetition(s)” as per Samsung’s comment.

- FL’s response to some companies’ comments:

  - @VIVO: FL feels not much difference between the proposal and your version. Since other companies seem fine with the current version, FL made no changes.

  - @LGE, Samsung, ZTE: the following example shows the motivation of Alt 1, hope it clarifies

    - Assume the resource pool includes RB set 0,1,2,3

    - Assume S-SSB is in RB set 0 as per legacy ARFCN configuration

    - Assume UE’s current COT includes RB set 1,2 and include S-SSB slot(s)

    - If Alt 2 is taken, there will be no transmission on RB set 1, 2 on those S-SSB slot(s), thus UE’s COT will probably be lost (e.g., WiFi UE may get in).

  - @Futurewei: FL feels “UE transmits …” lead to a determined UE behavior, maybe better.

  - @Spreadtrum: for PSSCH and PSFCH, we can organize such discussions later depending on companies’ input.

  - @CATT: technically, dummy signal is also ok to maintain the COT. But repetition maybe simpler and avoid further discussions on designing of a dummy signal.

[L] Proposal 5-5

When the COT contains multiple RB sets and includes S-SSB slot(s), down-select one of support the followings:

- Alt 1: UE transmits S-SSB repetition in each RB set of the COT

  - FFS details, e.g., location of such S-SSB repetition(s)

- Alt 2: UE only transmits S-SSB in one RB set as per legacy ARFCN configuration

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |
| Intel | Yes | After further considerations, we are OK with Alt.1. |
| vivo | OK |  |
| Qualcomm | OK | Alt 1 is OK. However, the S-SSB repetition is meant for COT padding but not for synchronization |
| OPPO | Yes |  |
| NEC | OK |  |
| Samsung | Yes | Some further comment: If we are supporting transmitting multiple repetition of S-SSB anyway in the frequency domain, is this solution can also be used for multiple S-SSB in one RB-set (e.g., Option 3-1 in Proposal 5-1)? |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| MediaTek | Comment | As we commented in last round, we share some concerns of power splitting issue for Alt1. For example, max of 23 dBm for one RB set S-SSB transmission. While for S-SSB repeated in multi-RB sets (e.g.,4 RB sets), the total power will be split into 4 parts, each of 17 dBm. If it can be clarified, we can accept Alt 1 for progress. |
| Lenovo | Yes |  |
| WILUS | Yes |  |
| xiaomi | Yes |  |
| ZTE,Sanechips | No | We cannot see the benefit, even with such repetition, the COT may still lost in some RB sets as we doesn’t ensure the initiating UE and responding UEs will always use all RB sets of the COT for all their transmissions. |
| Ericsson | OK |  |
| Transsion | Yes |  |
| CATT/GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia/NSB2 | No | The wording agreed in the offline session in the last meeting was: “When a UE’s COT contains multiple RB sets, UE may transmit S-SSB in each RB set, FFS details”. In our opinion, the specification should not mandate such S-SSB repetition. |
| LGE2 | Postpone | Before deciding it, we should check the S-SSB structure in SL-U. Moreover, we also need to consider RAN4 impact. At least, if we support this S-SSB repetition, it should be one of the resource forms which RAN4 currently considers (e.g., contiguous resources, almost contiguous resources, interlaced type resources). Otherwise, it will cause a huge impact on RAN4 or it may or may not be feasible. |

### [Closed] Proposals for Thursday GTW (Week 1)

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement or OCB exemption

- Summary of previous round

  - Alt 1: Support Option 1-1 only

    - Support (11): LGE, InterDigital, Ericsson, Lenovo, Spreadtrum, CMCC, Transsion, ZTE, NEC, Samsung, Docomo,

  - Alt 2: Support Option 3-1 only

    - Support (10): vivo, Sharp, Lenovo, Xiaomi, Panasonic, MediaTek, Huawei/HiSilicon, Samsung, CATT, Docomo,

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

    - Support (10): Apple, Intel, OPPO, Qualcomm, Samsung, MediaTek, Sharp, Panasonic, WILUS, Nokia

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

    - Support (4): vivo, CATT, MediaTek, Docomo,

- Updates on proposal (marked in red) and justifications

  - Made some updates based on comment from Qualcomm, Samsung, WILUS, vivo, MediaTek.

  - The difference between Option A and B is clarified in the sub-bullet. Option B has no additional work, but Option A has.

- FL’s response to some companies’ comments:

  - @vivo: yes, whether such exemption can be applicable is up to the regulation.

[H] Proposal 5-1c

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH Legacy S-SSB

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

  - Option B: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH Legacy S-SSB

    - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

#### [H] Proposal 5-2

Background: additional S-SSB occasions, belong to resource pool or not

- Summary of previous round

  - Support (12): vivo, InterDigital, Qualcomm, Intel, Lenovo, Xiaomi, Panasonic, Transsion, [ZTE], Huawei/HiSilicon, CMCC, WILUS

  - Not Support (12): DCM, LGE, Nokia, OPPO, Apple, NEC, Samsung, Sharp, Spreadtrum,  CATT, MediaTek, Ericsson

- Updates on proposal (marked in red) and justifications

  - Made the following change to help having same behavior among resource pools.

    - “Resource pool SL-BWP level…”

  - Made the following change as per ZTE’s comment, since it’s subject to bitmap configuration.

    - “S-SSB occasions belong to are not excluded from resource pool”

  - We have to make decision on this proposal during Thursday’s GTW, and FL assumes the discussions will be quite difficult.

    - FL added two sub-bullets under Alt 2 trying to address the concerns.

    - Companies are encouraged to further think how to address the concerns and how to find a way forward, suggestions are welcome during GTW.

[H] Proposal 5-2b

Resource pool SL-BWP level (pre-)configuration enables one of the followings:

- Alt 1: Additional candidate S-SSB occasions are excluded from resource pool

- Alt 2: Additional candidate S-SSB occasions belong to are not excluded from resource pool

  - On such occasions, a UE transmits either S-SSB or PSCCH/PSSCH/PSFCH, i.e., FDMed transmission from a single UE is not allowed.

  - Among different UEs, S-SSB conflicting with PSCCH/PSSCH/PSFCH is avoided by CPE design

#### [Suspend] Proposal 5-3

Background: Tx UE behaviour on additional S-SSB occasions

FL’s comment: Since the decision of Proposal 5-2 may impact Proposal 5-3 and 5-4, let’s come back to this later.

#### [Suspend] Proposal 5-4

Background: number/location of additional candidate S-SSB occasions

FL’s comment: Since the decision of Proposal 5-2 may impact Proposal 5-3 and 5-4, let’s come back to this later.

#### [L] Proposal 5-5

Background: more than one S-SSB on multiple RB sets

- Summary of previous round

  - Support (18): vivo, OPPO, InterDigital, Apple, NEC, Qualcomm, Samsung, Lenovo, Spreadtrum, CMCC, Intel, Xiaomi, CATT, Panasonic, Huawei/HiSilicon, WILUS, Ericsson, Transsion

  - Not Support (2): Nokia, ZTE,

- Updates on proposal (marked in red) and justifications

  - No updates are made.

- FL’s response to some companies’ comments:

  - @Qualcomm: I think RAN1 does not need to specify the purpose of such S-SSB repetition, so there should be no problem.

  - @Samsung: that will be separately discussed.

  - @LGE: FL suggests not to further postpone this issue. Because Channel Access AI is waiting for this to determine multi-channel access procedure for S-SSB.

  - @all: please check MediaTek’s comment on power splitting and clarify.

[L] Proposal 5-5

When the COT contains multiple RB sets and includes S-SSB slot(s), support the followings:

- Alt 1: UE transmits S-SSB repetition in each RB set of the COT

  - FFS details, e.g., location of such S-SSB repetition(s)

### [Closed] Round 3: Proposals and summary of previous round

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement or OCB exemption

- Summary of previous round

  - Alt 1: Support Option 1-1 only

    - Support (11): LGE, InterDigital, Ericsson, Lenovo, Spreadtrum, CMCC, Transsion, ZTE, NEC, Samsung, Docomo,

  - Alt 2: Support Option 3-1 only

    - Support (10): vivo, Sharp, Lenovo, Xiaomi, Panasonic, MediaTek, Huawei/HiSilicon, Samsung, CATT, Docomo,

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

    - Support (10): Apple, Intel, OPPO, Qualcomm, Samsung, MediaTek, Sharp, Panasonic, WILUS, Nokia

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

    - Support (4): vivo, CATT, MediaTek, Docomo,

- Updates on proposal (marked in red) and justifications

  - Made some updates based on comment from Qualcomm, Samsung, WILUS, vivo, MediaTek.

  - The difference between Option A and B is clarified in the sub-bullet. Option B has no additional work, but Option A has.

- FL’s response to some companies’ comments:

  - @vivo: yes, whether such exemption can be applicable is up to the regulation.

[H] Proposal 5-1c

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH Legacy S-SSB

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

  - Option B: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH Legacy S-SSB

    - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

| Company | Which of Alt 1,2,5,6 do you support? | Comments |
| --- | --- | --- |
| Qualcomm | Alt 5/6 | Our point is to reuse legacy S-SSB waveform whenever regulation allows it. If we don’t have time to work out 15KHz solution, we can live with option B. |
| Apple | Alt 5 | Option B (or Alt 6) may violate the OCB exemption rule, which is not preferred. |
| Intel | Alt 5 | We also share same view with Apple regarding Option B. |
| LGE | Alt 1 | For compromise, we can accept Alt 2 and Alt 6 if the only gap of 0 is supported. If we support non-zero gap, we should check the feasibility and spec impact with RAN4. In RAN1 perspective, it is useful to overcome the PSD requirement. Please remember that one of the motivations of having the same interlace across different RB sets for PSSCH in another proposal is to minimize the RAN4 spec impact. On checking the feasibility with RAN4, I checked NR SL discussion history. When we designed new physical structure including multiplexing PSCCH and PSSCH, simultaneous PSFCH transmissions, S-SSB design with respect to transient period, we’ve checked RAN4’ views on them by sending LS. It should be also applied to the new PSFCH structure as well. If not, we should focus on the physical structure minimizing the impact on RAN4. |
| Futurewei | Alt 1 | OK with Alt 5 also |
| DCM | 1/2/6 | We do not support Alt 5 since it leads to workload increase. Other Alts can work well. |
| ZTE, Sanechips | Alt 1 | Does NOT see benefit to support more than one S-SSB frequency-domain structure as Alt 1 works. |
| vivo | Alt 2/6 | @Apple @Intel We don’t know why Option B violate the OCB exemption rule. It means whenever the OCB exemption rule cannot be met by a single S-SSB (e.g., in the case of 15kHz), Option B itself is not applicable to this case – thus RAN1 does not spend time to work out a solution in this case. In our view that is why Alt 6 makes sense – in this case, more than one S-SSB should be transmitted to meet the OCB requirement. |
| OPPO | Alt 5/6 | For progress, we tend to support Alt 6, that Option B has no spec impact by reusing legacy S-SSB for OCB exemption with best effort. |
| NEC | Alt.1 |  |
| CATT/GOHIGH | Alt 2/6 |  |
| Transsion | Alt 1 |  |
| Spreadtrum | Alt 1 |  |
| Lenovo | Alt 1/2 | To meet OCB requirements, applying interlaced RB transmission to S-SSB is straightforward. Regarding the transmission by repetition in the frequency domain, contiguous RBs are beneficial for reliability. |
| WILUS | Alt 5 | We also share same view with Apple and Intel regarding Option B. |
| Sharp | Alt 2 or 5. |  |
| xiaomi | Alt 2 | Technically, we prefer alt2 with the simplicity. |
| CMCC | Alt 1 |  |
| Panasonic | Alt 2/6 |  |
| MediaTek | Alt 2/6 | Both Alt 2 and Alt 6 can minimize the spec impact. For Alt 6, our understanding for the current proposal is that - For 15 kHz SCS, Option 3-1 (i.e., repetition) is (pre-)configured - For 30 kHz and 60 kHz (if supported) SCS - For region with no OCB requirement, or with OCB exemption, one of Option 3-1(i.e., repetition) and OCB exemption (i.e., legacy S-SSB) is enabled by (pre-)configuration - Otherwise, Option 3-1 (i.e., repetition) is (pre-)configured |
| Huawei, HiSilicon | Alt 2 | We prefer a single solution, to reduce UE implementation complexity. |
| Nokia/NSB | Alt 5 | Legacy S-SSB should be used whenever possible. We can also consider Alt 6 if there is no time to further study 15kHz SCS . |

#### [M] Proposal 5-3

Background: Tx UE behaviour on additional S-SSB occasions

- Updates on proposal (marked in red) and justifications

  - RAN1 already made a WA that “Additional candidate S-SSB occasions are excluded from resource pool”.

  - On one hand, it seems straightforward to go with Alt 2. Because other Alts may cause no transmission on additional S-SSB occasions in some cases and cause COT loss.

  - On the other hand, Alt 2 may increase the duty cycle of S-SSB transmission, which may impact whether S-SSB can be qualified as SCSt (short control signalling transmission, see agreement below).

  - So far, FL made no change. Companies can express their views.

Agreement

- Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met:

  - Time duration is at most 1ms per transmission

  - The duty cycle of the S-SSB transmissions is at most 1/20

  - FFS: details of EDT

  - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined

- …

[M] Proposal 5-3c

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, (pre-)configuration enables one of support the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE | No | According to the working assumption, additional S-SSB is now excluded from a resource pool. In this point of view, Alt 1 is not necessary. |
| Apple | Yes | We support Alt 1. Even the additional S-SSB is excluded from resource pool, a UE does not necessarily transmit S-SSB in additional candidate S-SSB occasion(s) if the S-SSB transmission is already successful. |
| DCM | NO | If both Alt 1 and Alt 2 are considered based on each pros/cons, then why cannot we go with Alt 3? Can someone clarify this? If we receive a reasonable comment on Alt 3, then we may consider this proposal. |
| ZTE, Sanechips |  | Down-selection is preferred or leave it to UE implementation. |
| vivo | OK for progress | Actually, we prefer Alt 1, at least due to the duty cycle limitation. Moreover, Alt 1 is already good enough from coverage perspective, we don’t know why Alt 2 is needed, especially considering the UE power saving perspective. |
| OPPO |  | We support Alt 1. The total number of transmitted S-SSB can be kept as the same as that in legacy R16/17 SL, which can guarantee the sync. performance, therefore, it is not necessary to transmission S-SSB on all the additional candidate S-SSB occasions unless UE fails to transmit on legacy S-SSB occasion(s). Duty cycle is another reason that Alt 1 should be supported. |
| NEC | OK | For progress, we can accept the proposal |
| CATT/GOHIGH | No | Once additional S-SSB occasions are excluded from resource pool, we don’t see clear motivation to distinguish legacy S-SSB occasions or additional S-S-SSB occasions, the critical issue is how to guarantee the synchronization performance and avoid raising additional issues, such as duty cycle limitation. Thus, alt 3 (UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)) is our preference. |
| Transsion | Comments | We prefer Alt 1. From Rx UE perspective, we do not see the difference between Alt 1 and Alt 2. The Rx UE will attempt to receive the S-SSB on the S-SSB occasion or additional S-SSB occasion, and stop receiving after it successfully receive a S-SSB. In this sense it is possible to choose alt 1 or alt 2 by the UE implementation. |
| Spreadtrum | OK for progress |  |
| Lenovo | No | We support Alt 1, which achieves a good balance between coverage and power consumption. Down-selection is needed between the two alternatives. |
| WILUS | OK for progress | We prefer Alt 1. |
| xiaomi | Yes |  |
| CMCC | Yes | Alt 1 |
| Panasonic | OK | Additional candidate S-SSB occasions are excluded from resource pool is WA. Then, we think Alt 2 is straightforward. |
| MediaTek |  | For Alt 2, whether it violates the rule of duty cycle may depend on the definition of “observation period” in SCSt of SL-U, and also the detailed design on additional candidate S-SSB occasion(s) (e.g., the number of additional candidate S-SSB occasion(s)). Currently, Alt 1 is a way seems can better respect the rule of duty cycle in SCSt but if there is a majority of supporters on Alt 2, we can live with the current proposal or just leaving it to UE implementation. |
| Huawei/Hisilicon | Yes | We prefer Alt 1. Even additional S-SSB is excluded from resource pool, a UE does not necessarily transmit S-SSB in additional candidate S-SSB occasion(s) if the synchronization precision is met via previous S-SSB transmissions. In addition, in NR Uu, the number of SSB slots occasions per SSB period for 15 kHz and 30 kHz is 10 and 20 respectively. If such number is reused in SL-U and Alt 2 is supported, the power consumption is largely increased even if synchronization precision is met. |
| Nokia/NSB |  | We prefer to just select Alt 1. |
| Qualcomm |  | We share the same view with LGE. With additional S-SSB slot being excluded from the RP, only Alt 2 is acceptable for COT continuation. |
| Intel | No | For this proposal, we would rather prefer to postpone it until we conclude or confirm the working assumption related to excluding the additional candidate S-SSB occasions from the resource pool. |
| Samsung | No | We didn’t see a need to limit the TX UE behavior. We want to ask what’s the benefit of the proposal brings to RX UE behavior? If not, we don’t need to specify any TX UE behavior and leave it as UE implementation. |

#### [M] Proposal 5-4

Background: number/location of additional candidate S-SSB occasions

- Summary of previous round

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

    - Support (8): Intel, LGE, Docomo, OPPO, Spreadtrum, Sharp, FTW, ZTE

  - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

    - Support (12): CATT, Apple, NEC, InterDigital, QC, FTW, CMCC, Pana, MTK, Huawei/HiSilicon, WILUS, Nokia

- Updates on proposal (marked in red) and justifications

  - RAN1 already made a WA that “Additional candidate S-SSB occasions are excluded from resource pool”. Companies can further express their views on which Option should be supported and why.

  - For Option 1: Add some sub-bullets to clarify, to avoid confusion mentioned by Vivo.

  - For Option 2: Made the following updates based on comments from QC and MTK.

    - “…details on gap length (including possibility of being 0)…”

- FL’s response to some companies’ comments:

  - @Lenovo, vivo: For Option 3 below, RAN1 may need to further discuss the detailed design of location, offset, etc. It causes additional workload. FL suggests not to go with Option 3.

    - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

  - @Xiaomi: I feel probably this will be left to implementation. Note that RAN1 even agreed the 1st starting symbol of a slot is (pre-)configured and can be non-zero.

(below is copied from TS 38.331 h40)

SL-SSB-TimeAllocation-r16 ::=          SEQUENCE {

sl-NumSSB-WithinPeriod-r16             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                          OPTIONAL,    -- Need R

sl-TimeOffsetSSB-r16                   INTEGER (0..1279)                                                   OPTIONAL,    -- Need R

sl-TimeInterval-r16                    INTEGER (0..639)                                                    OPTIONAL     -- Need R

}

[M] Proposal 5-4c

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#112b-e:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

  - This is a new R18 parameter (denoted as sl-NumSSB-WithinPeriod-r18)

  - For UEs operating on unlicensed spectrum, UE shall use sl-NumSSB-WithinPeriod-r18 regardless sl-NumSSB-WithinPeriod-r16 is provided or not

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE | OK | Option 1 is much simpler. |
| Futurewei | OK |  |
| Apple | Agree | Option 2 is more flexible than Option 1, and we prefer it. |
| DCM | Yes | Support Option 1. It seems both can work without any large difference, then reusing the legacy mechanism is better to conclude this topic. If we go with Option 2 instead, what is the benefit compared to Option 1? |
| ZTE, Sanechips | Yes | Option 1 is preferred. |
| vivo | OK | With the clarification we would be OK for this proposal. Between them our preference is option 2. |
| OPPO | Yes | We support Option 1 which is a simple and efficient way. |
| NEC | OK | We prefer option 2. For option 2, the R16/R17 S-SSB occasion(s) and R18 additional occasions are distinguishable and could be with more flexibility of usage, e.g., proposal 5-3c. |
| CATT/GOHIGH | Yes |  |
| Transsion | Yes | Option 1 is preferred. |
| Spreadtrum | OK | We prefer option 1. |
| Lenovo | Ok | Option 1 is preferred. |
| WILUS | Yes | We prefer option 2 with more flexibility. |
| Sharp | Yes | We prefer option 1. |
| xiaomi | Yes |  |
| CMCC | Yes | We support Option 1 |
| Panasonic | OK | Option 1 is simpler. |
| MediaTek | Yes | Option 2 is supported considering it is a more specific way for detailed implementation. |
| Huawei/Hisilicon | Option 2 | We prefer option 2. As per legacy design, the S-SSB occasions should be equally distributed within one S-SSB period to cope synchronization misalignment. If option 1 is supported, it may not meet the intention to provide another chance for S-SSB transmission as quickly as expected, while option 2 is possible via the (pre-)configuration of gap length. In addition, take proposal 5-3 into considerations, option 2 is more suitable since the legacy and additional occasion(s) are differentiated. For example, if the TX UE have already transmitted on legacy S-SSB occasion, the TX UE does not need to transmit S-SSB on additional S-SSB occasion(s). |
| Nokia/NSB | OK | We support option 2. |
| Qualcomm | OK | We support option 2 |
| Intel | OK | We support option 1 |
| Samsung | OK | We support Option 1 |

#### [L] Proposal 5-5

Background: more than one S-SSB on multiple RB sets

- Summary of previous round

  - Support (18): vivo, OPPO, InterDigital, Apple, NEC, Qualcomm, Samsung, Lenovo, Spreadtrum, CMCC, Intel, Xiaomi, CATT, Panasonic, Huawei/HiSilicon, WILUS, Ericsson, Transsion

  - Not Support (2): Nokia, ZTE,

- Updates on proposal (marked in red) and justifications

  - Made the following updates as per Nokia’s comment. Hope this is a compromise so that we can endorse it by email to save online time.

    - “UE may transmits…”

  - Add the following based on MediaTek’s comment. Companies are encouraged to check and provide more analysis next meeting.

    - “FFS details, e.g., location of such S-SSB repetition(s), whether/how to address potential power reduction”

- FL’s response to some companies’ comments:

  - @Qualcomm: I think RAN1 does not need to specify the purpose of such S-SSB repetition, so there should be no problem.

  - @Samsung: that will be separately discussed.

  - @LGE: FL suggests not to further postpone this issue. Because Channel Access AI is waiting for this to determine multi-channel access procedure for S-SSB (see FFS in agreement below). Otherwise, we may block the progress of Channel Access AI.

Agreement

For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel

- FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set

- FFS: whether type A or type B or both will be supported for this case for PSFCH

- FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

[L] Proposal 5-5b

When the COT contains multiple RB sets and includes S-SSB slot(s), support the followings:

- Alt 1: UE may transmits S-SSB repetition in each RB set of the COT

  - FFS details, e.g., location of such S-SSB repetition(s), whether/how to address potential power reduction

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| Intel | Yes |  |
| LGE |  | For compromise, if the guardband PRBs between S-SSB repetitions are used, then we can accept it. As in a PSSCH transmission with multiple RB sets, we should consider the RAN4 impact. In that point of view, we need to focus on interlaced type resource or contiguous resource or almost contiguous resource as defined in RAN4 spec. |
| Futurewei | OK |  |
| DCM | OK |  |
| ZTE, Sanechips | Comments | From Tx UE’s perspective, a UE can only transmit S-SSB on the (pre)configured synchronization resources. From Rx UE’s perspective, it only monitors potential S-SSB on the (pre)configured synchronization resources. I.e., Alt 1 can be supported only when the resources for such ‘repetition’ are also (pre) configured. |
| vivo | Yes |  |
| OPPO | Yes with comment | This proposal is related to another issue that SL BWP contains multiple RB set, and this issue can have the following aspects: - SL BWP contains multiple RB sets, whether S-SSB should be transmitted in each RB set of the BWP. - A COT contains multiple RB sets, whether S-SSB should be transmitted in each RB set of the COT. For issue 1) above, if the default principle is to transmitted S-SSB in each RB set of SL BWP, whenever there is a COT that contains multiple RB sets, S-SSB should be transmitted in each RB set of the COT/BWP. However, for issue 1), if S-SSB is not transmitted in each RB set of SL BWP, whether there is a default/(pre-)configured RB set which is used for S-SSB transmission? If yes, and when the COT does not contain such default/(pre-)configured RB set, UE needs to transmit multiple S-SSBs in non-adjacent RB sets, i.e. 1 S-SSB in the default RB set, and n S-SSBs in the RB sets of the COT. Whether this kind of transmission of S-SSBs can be supported should be confirmed by RAN4. Similar issue for PSFCH transmission in multiple RB set, a draft LS is planned to be sent to RAN4, we think similar situation is applied to S-SSB transmission. |
| NEC | YES |  |
| CATT/GOHIGH | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Lenovo | Yes |  |
| WILUS | Yes |  |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |
| MediaTek | Yes | We are fine on the added FFS of potential power reduction issue. Thanks. The potential power reduction may result in a variance of S-SSB Tx power (e.g., the S-SSB Tx power is variable with the change of RB set number contained within different COTs), which may further impact the RSRP based SyncRef UE selection in legacy SL. |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes with comment | We don’t want to block the progress, but the proposal right now is more about an intention to resolve a specific issue, without clear specification impact. Our interpretation of this proposal is more like an intermediate step, and we still need to discuss the actual spec support for the proposal. |

### Round 4: Proposals and summary of previous round

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement or OCB exemption

- Summary of previous round

  - Alt 1: Support Option 1-1 only

    - Support (9): LGE, FTW, DCM, ZTE, NEC, Transsion, SPRD, Lenovo, CMCC,

  - Alt 2: Support Option 3-1 only

    - Support (9): DCM, vivo, CATT, Lenovo, Sharp, Xiaomi, Pana, MTK, HW,

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

    - Support (7): QC, Apple, Intel, OPPO, WILUS, Sharp, Nokia

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

    - Support (7): QC, DCM, vivo, OPPO, CATT, Pana, MTK,

- Updates on proposal (marked in red) and justifications

  - I made no updates.

  - There is no majority view.

  - During the down-selection, two factors need to be considered:

    - RAN1 workload: since we only have 2 meetings left.

    - UE implementation complexity: if finally there are N S-SSB structures, UE needs to implement corresponding S-SSB detecting algorithms for such N S-SSB structures and do the testing. So smaller number of N is preferred.

  - FL’s view on the above two factors are summarized in the table below. Based on the analysis, FL would suggest to go with Alt 2.

- FL’s response to some companies’ comments:

  - @Apple, Intel, WILUS: On your point “Option B (or Alt 6) may violate the OCB exemption rule” --> FL shares similar view with vivo/MediaTek that this is not a valid argument. As per the last Note, if Option B is applicable, then (pre-)configuration can use Option B. If Option B is not applicable, then (pre-)configuration can use Option 3-1. Nothing is violated.

FL’s view on the alternatives:

| Alternatives | RAN1 workload | How many S-SSB structures (this will directly impact UE implementation complexity and testing efforts) |
| --- | --- | --- |
| Alt 1 | High: RAN1 needs to additionally design a S-SSB structure when each interlace only has 10 PRBs, i.e., FFS of Option 1-1 | 2: due to the FFS, there will be two different S-SSB structures |
| Alt 2 | Low: one unified design is possible | 1: one unified design is possible |
| Alt 5 | High: in addition to Alt 2, RAN1 needs to additionally design a S-SSB structure to meet 2 MHz for 15kHz, i.e., sub-bullet of Option A | 3: due to the 2MHz issue, there are 3 S-SSB structures |
| Alt 6 | Low: same as Alt 2 | 2: in addition to Alt 2, legacy S-SSB is also used |

[H] Proposal 5-1e (S-SSB, OCB)

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Legacy S-SSB

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

  - Option B: Legacy S-SSB

    - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

FL’s comment: FL may suggest to go with Alt 2 during GTW based on the analysis above. What’s your view and suggestions?

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE |  | Even though the LS for the non-contiguous S-SSB targets different RB sets. We can consider whether the S-SSB repetition will be contiguous or non-contiguous for future design. Even for the single RB set transmission, we should consider PSD requirement. For compromise, if we focus on the only contiguous resources with gap 0, we can accept it. Depending on the feedback from RAN4, we may or may not further consider non-contiguous S-SSB repetition in a RB set via (pre)configuration or something else. |
| Samsung |  | We believe Alt 6 with configurability on one or multiple instances of legacy SSB can cover both Alt 2 and Alt 6, which could be the simplest solution with the almost same amount of spec impact as Alt 2. This solution can also be generalized to multiple S-SSB transmission in multiple RB-sets. In this sense, we prefer Alt 6. |
| Lenovo |  | We are not sure if the workload for Alt 2 is lower than Alt 1, considering the details for Alt 2 need further study. In that case, we suggest keeping only Alt 1 and Alt 2 for further consideration. |
| OPPO |  | We prefer Alt 6. |
| Intel |  | We generally OK with the FL’s assessment. However, this won’t include the 60 kHz SCS case. In this case both Alt1 and Alt-2 would require additional work, while option A won’t require any. While we still prefer Option A alone, we think supporting option 3-1+Option A may be already a good compromise, and therefore we would be OK with Alt.5. |
| Qualcomm |  | Alt 6 is preferred. |
| MediaTek |  | Alt 2 first, then Alt 6. For Alt 1, additional to the FFS, we have strong concern on the degraded correlation performance of S-PSS/S-SSS as shown in the papers from us and some other companies. For the comment from @Intel on 60kHz SCS, it is obviously Alt1 would require additional work considering IRB is not supported by 60kHz. But we think there is no additional work for Alt2 because the repetition manner can still be used for 60kHz. Anyway, the current proposal is focus on 15 and 30 kHz SCS, 60kHz (if supported) can be further discussed. |
| vivo |  | In my view, essentially Alt 2 and Alt 6 are the same if value of N can be 1 in option 3-1 (which is currently not precluded in Alt 2), can the companies support only one of the Alt 2 and Alt 6 clarify why you can only support one of them, and what the different between them in your view? Regarding the workload argument, Alt 2 (option 3-1) was proposed and discussed by several companies during R16, the spec impact should be clear. While for Alt 1, interlace based S-SSB is something never been discussed (even in Uu). It is obvious that Alt 1 has higher work load and spec impact than Alt 2. |
| Nokia/NSB |  | We propose to select Alt 6 if complexity/workload of Alt 5 is considered too high. We still think that legacy S-SSB should be used whenever possible. |

#### [M] Proposal 5-3

Background: Tx UE behaviour on additional S-SSB occasions

- Summary

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

    - Support (9): Apple, vivo, OPPO, Transsion, Lenovo, WILUS, CMCC, HW, Nokia,

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

    - Support (3): Pana, [LGE], QC,

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

    - Support (3): DCM, CATT, Samsung

- Updates on proposal (marked in red) and justifications

  - Views are still divergent.

  - FL adds Alt 3 (which is equivalent to leave it to UE implementation). RAN1 can down-select one of them.

Agreement

- Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met:

  - Time duration is at most 1ms per transmission

  - The duty cycle of the S-SSB transmissions is at most 1/20

  - FFS: details of EDT

  - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined

- …

[M] Proposal 5-3e (S-SSB, Tx UE behaviouir)

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, (pre-)configuration enables down-select one of the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

FL’s comment: FL may suggest to down-select to one during GTW. Any suggestions on how to move forward?

| Company | Agree? | Comments |
| --- | --- | --- |
| LGE |  | For alt 1, when UE determines not to use additional S-SSB occasion due to its success of S-SSB transmission on R16/17 resources, but the S-SSB occasion is located in the middle of a COT, what is the UE’s behavior? The wording itself seems that these two proposals are contradict each other. Some note or wording change or clarification is needed first. |
| NEC |  | Maybe Alt.2 (transmit on all of xxx) as a default behaviors if Alt.1 (transmit on some of xxx) is not configured. |
| Samsung |  | Again, we want to emphasize that when a UE behavior is specified, we should have clear understanding on its necessity – either to avoid confusing or with benefit for the UE. We didn’t see any benefit from restricting UE TX behavior to Alt 1 or Alt 2. No matter Alt 1 or Alt 2 is selected, the RX UE behavior is the same – checking each potential location from earliest to the latest until receiving one S-SSB. Then why we need to restrict the TX UE behavior at this point? We still prefer Alt 3 – how to transmit is up to the UE’s implementation, which covers the UE behavior in both Alt 1 and Alt 2. |
| Lenovo |  | Alt 1 is supported. |
| OPPO |  | We support Alt 1. |
| Intel |  | We still believe the working assumption should be first confirmed before we can make this agreement. However, if companies prefer to move forward we prefer Alt. 2. In this case, it is unclear what the benefit of Alt1 and Alt2 would be given that the resources for additional S-SSB occasions are excluded from the resource pool. |
| Spreadtrum |  | If there is no S-SSB transmission on the additional S-SSB occasions, these resources are wasted and it may cause COT to be interrupted when the S-SSB occasion is located in one COT. So, we prefer Alt2. |
| Nokia/NSB |  | We prefer Alt 1. Transmitting additional S-SSBs just to be safe should be avoided. |

#### [M] Proposal 5-4

Background: number/location of additional candidate S-SSB occasions

- Summary of previous round

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

    - Support (12): LGE, DCM, ZTE, OPPO, Transsion, SPRD, Lenovo, Sharp, CMCC, Pana, Intel, SS

  - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

    - Support (8): Apple, vivo, NEC, WILUS, MTK, HW, Nokia, QC,

- Updates on proposal (marked in red) and justifications

  - Option 1 has more supporters.

  - One thing unclear is whether RAN1 needs to differentiate Tx UE behavior on legacy and additional S-SSB occasions (related to decision of Proposal 5-3).

    - Example#1: if UE always attempt to transmit on legacy S-SSB occasions, but choose Alt 1 or Alt3 on additional S-SSB occasions, then the Tx UE behavior on legacy and additional S-SSB occasions are different. In this case, Rx UE needs to know which are legacy S-SSB occasions and which are additional S-SSB occasions. It seems Option 1 cannot achieve this?

(below is copied from TS 38.331 h40)

SL-SSB-TimeAllocation-r16 ::=          SEQUENCE {

sl-NumSSB-WithinPeriod-r16             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                          OPTIONAL,    -- Need R

sl-TimeOffsetSSB-r16                   INTEGER (0..1279)                                                   OPTIONAL,    -- Need R

sl-TimeInterval-r16                    INTEGER (0..639)                                                    OPTIONAL     -- Need R

}

[M] Proposal 5-4e (S-SSB, number/location)

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#112b-e:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

  - This is a new R18 parameter (denoted as sl-NumSSB-WithinPeriod-r18)

  - For UEs operating on unlicensed spectrum, UE shall use sl-NumSSB-WithinPeriod-r18 regardless sl-NumSSB-WithinPeriod-r16 is provided or not

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

FL’s comment: please check Example#1 above, and share your view on whether RAN1 needs to differentiate Tx UE behavior on legacy and additional S-SSB occasions, and whether Rx UE needs to know which are legacy S-SSB occasions and which are additional S-SSB occasions?

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| LGE | Option 1 | Even though Alt 1 or Alt 3 is adopted, the RX UE may need to continue monitoring S-SSB since the RX UE does not know whether TX UE success or not on the R16/17 resources. |
| NEC | Option 2 | FL made a valid point, we also shared our view in last round that For option 2, the R16/R17 S-SSB occasion(s) and R18 additional occasions are distinguishable and could be with more flexibility of usage, e.g., based on out come of proposal 5-3c. |
| Samsung | Option 1 | Our understanding is, there is no need to distinguish, same as NR-U. |
| Lenovo | Option 1 |  |
| OPPO | Option 1 |  |
| Intel | Option 1 |  |
| Spreadtrum | Option 1 |  |
| MediaTek | Option 2 | At least for Alt1 and Alt3 of Proposal 5-3e, there is a different behavior for Tx UE regarding legacy S-SSB and additional candidate S-SSB occasion(s). |
| vivo | OK to down select | As commented before we prefer Alt 1, at least due to the duty cycle limitation, and considering the benefit from UE power saving perspective. Regarding LGE’s comment, I would like to ask a question on what CPE of S-SSB is considered in this case. If a short CPE or no CPE (=0) is used, still the COT can be lost because the gap between the previous Tx and the S-SSB is large enough for others. If a long CPE is used (e.g., ensuring a 16us or smaller gap), generally the S-SSB cannot be transmitted by type-1 or type-2A LBT, which means mostly the UE cannot transmit S-SSB unless it has transmission just before the S-SSB and the COT is large enough. Having saying that, we would OK to have an FFS point in Alt.1 on this issue if it helps for progress. |
| Nokia/NSB | Option 2 | We think that RX UE should know which are legacy S-SSB occasions and which are additional S-SSB occasions. If decoding of S-SSB is successful in the legacy occasion, the UE does not need to decode S-SSB in additional occasions. |

#### [L] Proposal 5-5

Background: more than one S-SSB on multiple RB sets

- Summary of previous round

  - Support (18): QC, Apple, Intel, FTW, DCM, vivo, OPPO, NEC, CATT, Transsion, SPRD, Lenovo, WILUS, Xiaomi, Pana, MTK, HW, SS

  - Not Support (2): [LGE], [ZTE]

- Updates on proposal (marked in red) and justifications

  - FL updated the proposal to be more generic. Companies can bring more details next meeting.

  - Let me use the Figure from OPPO (see below) to clarify the situation

    - Assume SL-BWP has 5 RB sets (RB set 0,1,2,3,4,)

    - Assume as per legacy ARFCN, S-SSB is in RB set 2.

    - If no new agreements are made, then, on all S-SSB slots, all sidelink UEs will go to RB set 2 to transmit/receive S-SSB.

      - It means the COT on other RB sets (RB set 0,1,3,4) may easily get lost and SL-U’s performance will be pretty bad

    - So it’s reasonable that a UE transmits S-SSB repetition on more than one RB set to maintain the COT.

      - Most companies share similar view here. The updated proposal reflects this.

    - Then, other details are FFS.

      - E.g., as shown in Figure below, if UE’s COT includes RB set 0, 1, and default S-SSB is in RB set 2, what should the UE do? Transmit S-SSB repetition in all RB set 0,1,2? Or transmit S-SSB only on RB set 2 so that its own COT is lost? Or else? --> Anyway, such details are FFS.

      - RAN1 also sends LS to RAN4 about whether S-SSB transmissions over non-contiguous RB sets is feasible, this may impact RAN1 design. --> Again, this can also be FFS and is reflected in the Note.

        - FYI: the LS to RAN4 is approved in R1-2304218 (“LS on PSFCH and S-SSB transmissions over non-contiguous RB sets”).

  - The intention to agree this proposal in this meeting is mainly about progress.

    - Because Channel Access AI is waiting for this to determine multi-channel access procedure for S-SSB (see FFS in agreement below). Otherwise, we may block the progress of Channel Access AI.

    - E.g., if we further postpone the issue, I imagine the progress will be: RAN4 reply to RAN1 at the end of May meeting. Then, in August meeting, SL-U PHY AI discuss this and try to decide. Then, SL-U Channel Access AI has no time to discuss the multi-channel access procedure for S-SSB and the WI cannot be completed by August!

- FL’s response to some companies’ comments:

  - @LGE: FL suggests not to further postpone this issue, see reasons above. We’ll take RAN4’s reply into account, this is reflected in the last Note.

  - @LGE: Proposal 5-5 is about frequency domain. Proposal 5-3 is about time domain. FL think there should be no confusion.

  - @Futurewei: thanks for the comment. The proposal is updated, so there is no confusion now.

(Figure from OPPO’s reply)

Agreement

For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel

- FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set

- FFS: whether type A or type B or both will be supported for this case for PSFCH

- FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

[L] Proposal 5-5e (S-SSB, multiple RB sets)

When the COT SL-BWP contains multiple RB sets and includes S-SSB slot(s), support the followings:

- Alt 1: UE may transmit S-SSB repetition in each more than one RB set of the COT

  - FFS details, e.g., location of such S-SSB repetition(s), whether/how to address potential power reduction

  - FFS: the relationship with UE’s COT

  - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

FL’s comment: this proposal is near stable, let’s discuss by email directly.

## [Closed] Issue#6: Others

### [Closed] 1st round: Proposals and background

#### [L] Proposal 6-1

Background: power control updates due to PSD limit

- Summary

  - Updates on power control are needed (3): vivo, Apple, MediaTek

  - Updates on power control are not needed (1): Intel

  - Should be RAN4 discussions (1): ZTE

- FL’s view

  - So far, the level of interest is low. Companies are encouraged to further check, e.g., the cases mentioned by Vivo (e.g., multiple PSFCH transmissions in 1 MHz).

[L] Proposal 6-1

Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

| Company | Agree? | Comments |
| --- | --- | --- |
| vivo | OK | Considering that this issue may have impact on PSFCH prioritization, it should be handled by RAN1, not RAN4. |
| LGE | Yes | In addition, we may also need to consider power restriction due to UE-to-UE COT sharing. In NR-U, when gNB uses the COT initiated by a UE, then the power is limited so that the resulting EDT is smaller than the EDT used to initiate the COT. |
| OPPO | Yes |  |
| Apple | Yes |  |
| NEC | Yes | One comment is that if this is studied, then the PSFCH transmission drop in common PRBs due to PSD limitation is still necessary? |
| Qualcomm | OK |  |
| Futurewei | OK |  |
| Spreadtrum | OK |  |
| CMCC | OK |  |
| xiaomi | Yes |  |
| CATT/GOHIGH | Yes |  |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |

#### Others

- On 60kHz SCS:

  - Positive on supporting 60kHz SCS (9): OPPO, Huawei/HiSilicon, Apple, Lenovo, CATT,  Futurewei, Lenovo, InterDigital, Intel

  - Deprioritize 60kHz SCS (3): Samsung, Nokia , ITL

  - FL’s view: Some companies support 60 kHz SCS and give related design (e.g., for PSFCH). However, as discussed in last meeting, let’s first focus on 15/30kHz SCS, and come back to 60 kHz SCS later if time allows. Companies are encouraged to check and provide simple/unified design for 60 kHz SCS.

- Others

  - Some other issues are mentioned, e.g., congestion control, CBR measurement, etc.

  - FL assumes such issues are not very urgent at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

### [Closed] 2nd round: Proposals and summary of previous round

#### [L] Proposal 6-1

Background: power control updates due to PSD limit

- Summary of previous round

  - Support (13): vivo, LGE, OPPO, Apple, NEC, Qualcomm, Futurewei, Spreadtrum, CMCC, xiaomi, CATT/GOHIGH, MediaTek, ZTE

- Updates on proposal (marked in red) and justifications

  - Companies who replied to this proposal are all ok with it. No companies showed concerns.

  - So FL suggests to endorse it by email directly.

- FL’s response to some companies’ comments:

  - @NEC: such details can be further studied, it’s up to companies’ analysis.

[L] Proposal 6-1

Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

| Company | Agree? | Comments |
| --- | --- | --- |
| CMCC | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| LGE | Yes | We can further discuss the change needed for UE-to-UE COT sharing condition. Is it common understanding, touching the PSFCH power control also includes touching the determination on the number of simultaneous PSFCH transmission? |
| Qualcomm | OK |  |
| OPPO | Yes |  |
| NEC | OK | OK and we suggest to add "may" in proposal 4-1 |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| ZTE, Sanechips |  | OK to further study this, but this should be also RAN4 involved. |
| CATT/GOHIGH | Yes |  |
| Nokia/NSB2 | Yes |  |
| Lenovo | Yes |  |

FL’s comment: Proposal 6-1 is endorsed by email directly.

# Proposals for Wednesday online

[H] Proposal 3-1g (IRB, RA indication)

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

Cited Agreements from RAN1#98b:

- Support at least an initial transmission and reservation of the resource(s) for retransmission(s) to have the same number of sub-channels

==

[M] Proposal 3-4f (CRB, guardband PRB)

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

  - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

[H] Proposal 2-1f (TBS, L_ref)

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Support the followings

  - Alt 1: Support Option 4 only

- Note: the options are as below

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

[M] Proposal 3-2g (IRB, mapping)

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets

    - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

      - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on

      - At least support that the above K interlace(s) are contiguous

        - FFS: whether/how to support the above K interlace(s) are non-contiguous

  - Note: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace will be further discussed

- Interlace is indexed as per NR-U

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - …

==

[L] Proposal 5-5i (S-SSB, multiple RB sets) (as Working Assumption)

When the SL-BWP contains multiple RB sets, support the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - Alt 1: UE may transmit S-SSB repetition in more than one RB set

    - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP

    - FFS: the relationship with UE’s COT

    - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

[H] Proposal 5-1e (S-SSB, OCB)

For S-SSB transmission within 1 RB set:

- Down-select one of the followings for 15 kHz and 30 kHz SCS in RAN1#112b-e:

  - Alt 1: Support Option 1-1 only

  - Alt 2: Support Option 3-1 only

  - Alt 5: Support both Option 3-1 and Option A, and enable one of them by (pre-)configuration

  - Alt 6: Support both Option 3-1 and Option B, and enable one of them by (pre-)configuration

- Note: the Options are as below

  - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

    - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

  - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

    - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

    - FFS gap of 0

  - Option A: Legacy S-SSB

    - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

  - Option B: Legacy S-SSB

    - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

FL’s view on the alternatives:

|  | RAN1 workload | How many S-SSB structures? (this will directly impact UE implementation complexity and testing efforts) |
| --- | --- | --- |
| Alt 1 | High: RAN1 needs to additionally design a S-SSB structure when each interlace only has 10 PRBs, i.e., FFS of Option 1-1 | 2: due to the FFS, there will be two different S-SSB structures |
| Alt 2 | Low: one unified design is possible | 1: one unified design is possible |
| Alt 5 | High: in addition to Alt 2, RAN1 needs to additionally design a S-SSB structure to meet 2 MHz for 15kHz, i.e., sub-bullet of Option A | 3: due to the 2MHz issue, there are 3 S-SSB structures |
| Alt 6 | Low: same as Alt 2 | 2: in addition to Alt 2, legacy S-SSB is also used |

==

[M] Proposal 5-3e (S-SSB, Tx UE behaviouir)

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, (pre-)configuration enables down-select one of the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

[M] Proposal 2-3g (COT, Tx UE behaviour)

Regarding Tx UE behavior, at least when it initiates a COT:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- FFS: whether/how to support that for the remaining slots of a COT, at least for MCSt with no greater than 16us gap and the 1st staring symbol for PSCCH/PSSCH transmission is (pre-)configured to be #0, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS other cases

  - FFS applicable scenarios

    - e.g., at least for MCSt with no greater than 16us gap

    - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE

  - FFS: Rx UE behavior

FFS: COT sharing case

[M] Proposal 3-6f (IRB, lowest sub-channel)

For interlace RB-based PSCCH/PSSCH transmission in SL-U, down-select one of support the following:

- Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

  - i.e., in a resource pool, lowest sub-channel is sub-channel#0

- Option 2: lowest sub-channel is the sub-channel with lowest frequency location

==

[M] Proposal 2-2-1f (AGC, Tx)

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol,

    - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols subject to (pre-)configuration

      - FFS: whether this is subject to explicit or implicit (pre-)configuration is explicit or implicit, or under some conditions, etc.

- Note: AGC symbol is duplication of next symbol

==

[M] Proposal 2-2-2e (AGC, Rx)

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

FL’s comment: it seems quite some companies prefer to discuss Tx side first, then let’s postpone the discussion on Rx side (i.e., Proposal 2-2-2).

[M] Proposal 5-4e (S-SSB, number/location)

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#112b-e:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

  - This is a new R18 parameter (denoted as sl-NumSSB-WithinPeriod-r18)

  - For UEs operating on unlicensed spectrum, UE shall use sl-NumSSB-WithinPeriod-r18 regardless sl-NumSSB-WithinPeriod-r16 is provided or not

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

(below is copied from TS 38.331 h40)

SL-SSB-TimeAllocation-r16 ::=          SEQUENCE {

sl-NumSSB-WithinPeriod-r16             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                          OPTIONAL,    -- Need R

sl-TimeOffsetSSB-r16                   INTEGER (0..1279)                                                   OPTIONAL,    -- Need R

sl-TimeInterval-r16                    INTEGER (0..639)                                                    OPTIONAL     -- Need R

}

# Conclusions

TBD

# References

- R1-2302290	On Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

- R1-2302325	Discussion on physical channel design for sidelink on unlicensed spectrum	FUTUREWEI

- R1-2302354	Physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2302487	Physical channel design framework for sidelink on unlicensed spectrum	vivo

- R1-2302520	Sidelink Physical Channel Design Considerations	National Spectrum Consortium

- R1-2302550	On PHY channel designs and procedures for SL-U	OPPO

- R1-2302602	Discussion on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2302705	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2302798	On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2302848	Discussion on physical channel design framework for SL-unlicensed	Sony

- R1-2302923	Discussion on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

- R1-2302952	SL U physical layer design framework	InterDigital, Inc.

- R1-2302985	Discussion on physical channel design for sidelink-unlicensed	xiaomi

- R1-2303130	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2303169	Physical channel design for sidelink on unlicensed spectrum	Panasonic

- R1-2303199	Discussion on physical channel design framework for SL-U	ETRI

- R1-2303236	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CMCC

- R1-2303314	Physical layer design framework for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2303324	PHY channel design framework for SL-U	Ericsson

- R1-2303368	Discussion on physical channel design framework	MediaTek Inc.

- R1-2303375	Discussion of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2303401	Discussion on physical layer structures and procedures for SL-U	ZTE, Sanechips

- R1-2303485	On Sidelink Physical Channel Design Framework for Unlicensed Spectrum	Apple

- R1-2303539	NR Sidelink Unlicensed Physical Channel Design	Fraunhofer HHI, Fraunhofer IIS

- R1-2303592	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2303670	Discussion on physical channel design framework	NEC

- R1-2303714	Discussion on channel design framework in SL-U	NTT DOCOMO, INC.

- R1-2303769	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

- R1-2303820	Physical Channel Design framework for SL-U	ITL

- R1-2303833	Discussion on PHY channel design framework for SL-U	WILUS Inc.

# Annex A: Contact Information

Please provide/update your contact information here so that delegates can better communicate with each other if needed.

| Company | Name | Email |
| --- | --- | --- |
| Huawei | Xiang Mi | Shawn.mixiang@huawei.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| LGE | Daesung Hwang Seungmin Lee | daesung.hwang@lge.com edison.lee@lge.com |
| CATT, GOHIGH | Rui Zhao Shupeng Li Xiaoran Wen | zhaorui@goghigh.com.cn lsp@catt.cn wenxiaoran@gohigh.com.cn |
| OPPO | Zhenshan ZHAO Kevin Lin | zhaozhenshan@oppo.com kevin.lin@oppo.com |
| vivo | Zichao Ji | jizichao@vivo.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| Intel | Salvatore Talarico | salvatore.talarico@intel.com |
| CMCC | Pengyu Ji Jingwen Zhang | jipengyu@chinamobile.com zhangjingwen@chinamobile.com |
| Panasonic | Ayako Iwata | iwata.ayako@jp.panasonic.com |
| xiaomi | Wensu Zhao Qun Zhao | zhaowensu@xiaomi.com zhaoqun1@xiaomi.com |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| Nokia/NSB | Naizheng Zheng | naizheng.zheng@nokia-sbell.com |
| Ericsson | Ricardo Blasco Jose Leon Calvo | name.surname at company . com jose.leon.calvo@ericsson.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| InterDigital | Aata El Hamss | Aata.elhamss@interdigital.com |
| Lenovo | Xiaodong Yu Karthikeyan Ganesan Alexander Golitschek | Yuxd1@lenovo.com kganesan@lenovo.com aelbwart@lenovo.com |
| Samsung | Hongbo Si | hongbo.si@samsung.com |
| Futurewei | George Calcev | gcalcev@futurewei.com |
| Qualcomm | Giovanni Chisci Chih-Hao Liu Stelios Stefanatos | gchisci@qti.qualcomm.com chihliul@qti.qualcomm.com sstefana@qti.qualcomm.com |
| NEC | Zhaobang MIAO Jin YANG | miao_zhaobang@nec.cn yangjin@labs.nec.cn |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| MediaTek | Tao Chen | tao.chen@mediatek.com |
| Fraunhofer | Tom Wirth | thomas.wirth@hhi.fraunhofer.de |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |
| Sony | Kazuyuki Shimezawa | kazuyuki.shimezawa@sony.com |
| Johns Hopkins University APL | Vijitha Weerackody | vijitha.weerackody@jhuapl.edu |
| National Spectrum Consortium | Amitav Mukherjee | Amitav.Mukherjee@nationalspectrumconsortium.org |

# Annex B: Outcomes of RAN1 meetings

## RAN1#109-e (May 9 – 20, 2022)

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported

- PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets

  - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc.

- FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool

- FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc.

- FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point

  - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions

  - FFS: whether/how to address IBE (In Band Emission) impact

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission (if supported), at least the following candidates can be discussed:

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

    - FFS: Other resource allocation granularity, e.g., RB-level

  - 1 sub-channel equals K interlaces if sub-channel is supported

    - FFS details

  - Other candidates are not precluded

  - FFS: mapping of PSCCH to frequency resources

  - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Agreement

For slot structure in SL-U:

- At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported

  - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

  - FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

Agreement

For S-SSB and synchronization in SL-U:

- FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc.

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

- FFS: whether to support 4 symbols S-SSB

  - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above

- FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission

- FFS whether any changes to R16/R17 NR SL synchronization procedure

## RAN1#110 (August 22 – 26, 2022)

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

## RAN1#110b-e (October 10 – 19, 2022)

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding 1 sub-channel equals K interlace(s)

  - At least K=1 and K=2 is supported for 15 kHz SCS

  - At least K=1 is supported for 30 kHz SCS

  - FFS: details related to multiple RB sets

Working assumption:

Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

- RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol

- The candidate starting symbol(s) are intended for AGC purpose

  - FFS: other potential uses of the candidate starting symbol(s)

- FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc.

- FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Agreement

To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS:

- Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH

- Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported

FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3)

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- When more than one RB set is used for transmissions, down-select one of the followings

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

  - Option B: Support that the used interlace index(s) in different RB sets can be different

- FFS details

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- Down-select one of the followings

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - At least RB set index(s) is not explicitly indicated

- FFS details

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Agreement

At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool.

- Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Agreement

At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details

Agreement

To address PSFCH transmission dropping due to LBT failure, the followings are to be studied:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Alt 2: PSFCH resources are dynamically indicated

- Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE

- Alt 4: drop PSFCH transmission

- Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

Regarding additional candidate S-SSB occasions:

- Their number and time domain locations are (pre-)configured or pre-defined

## RAN1#111 (November 14 – 18, 2022)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding the location of 1st starting symbol, down-select one of the followings:

  - Option 1: it is fixed as symbol#0

  - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL

- Regarding the location of 2nd starting symbol, down-select one of the followings:

  - Option A: it is a fixed location

    - FFS the location, e.g., symbol#4, #7, etc.

  - Option B: it is a (pre-)configured location per resource pool

    - FFS the details of candidate locations

- Note: assume symbol index in a slot starts from #0

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, down-select one of the followings:

  - Option 1: The reference symbol length is dynamically indicated by Tx UE

  - Option 2: The reference symbol length is determined based on 1st starting symbol

  - Option 3: The reference symbol length is determined based on 2nd starting symbol

  - Option 4: The reference symbol length is (pre-)configured

Agreement

Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC

  - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc.

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 interlace

- Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE uses different cyclic shifts on different PRBs in the interlace

- Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - The above dedicated PRB and common PRBs are within 1 interlace

  - FFS: value of K2

- Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth

- FFS: whether IBE issue exists and whether/how to address it

- Note: in the above descriptions

  - The dedicated PRB/cyclic shift conveys ACK/NACK information

- Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS.

Agreement

Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, RAN1 further study the followings:

  - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool

  - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set

  - FFS: whether/how to use intra-cell guardband PRBs

Agreement

For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

- Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

- Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K

  - FFS gap of 0

  - FFS PSBCH resource

- Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements

  - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

- For Option 1-1 and 1-2 above

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set

- FFS: whether transient period issue exists and whether/how to address it

Agreement

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

  - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

- Regarding Rx UE behaviour, down-select one of the followings:

  - Option A: The Rx UE always monitors two AGC symbols in such slot

  - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

    - FFS details

  - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Agreement

To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - FFS other details, e.g., HARQ-ACK timeline

- Alt 2: PSFCH occasions are dynamically indicated

  - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT

  - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc.

- FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH

- FFS: Whether/how to address PSFCH collision if any

- FFS: Whether/how to handle the linearly decreased PSFCH capacity

Agreement

Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

- Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

- Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

- Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

Agreement

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, RAN1 further study the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions

- FFS details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, further study the following options:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size

  - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size

    - FFS: whether/how to use intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size

  - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size

    - FFS: how to use intra-cell guardband PRBs

    - FFS: how to use the subchannel including PRBs in guardband

Agreement

Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives:

- Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs

  - FFS details

- Alt 2: Each PSFCH transmission occupies some dedicated PRBs

  - FFS details

Agreement

Regarding S-SSB, RAN1 further study the following:

- How to transmit S-SSB when a SL BWP contains multiple RB sets

## RAN1#112 (February 27 – March 3, 2023)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.