3GPP TSG RAN WG1 Meeting #112bis-e		R1-2303938

e-Meeting, 17th – 26th April 2023

Agenda Item:	9.6

Source:	Rapporteur (Ericsson)

Title:	RAN1 agreements for Rel-18 NR RedCap

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made so far for the Rel-18 WI on ‘Enhanced support of reduced capability NR devices’ (WI code NR_redcap_enh-Core, WID in [1], sometimes colloquially referred to as ‘eRedCap’). This document also indicates estimated spec impacts in red color – these are just suggestions from the WI rapporteur. Useful references can be found in the end of this document.

Introduction	1

1	UE BB bandwidth reduction	1

2	UE peak data rate reduction	7

References	8

- UE BB bandwidth reduction

RAN1#110bis-e:

R1-2210248 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210249 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210250 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210251 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Initial BWP Agreement: [38.213, 38.331] For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs (replaced by later agreement) Number of PRBs Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (at least for unicast), down-select between the following options for the maximum number of PRBs that the UE can [receive/process]: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH (at least for unicast) and PUSCH Agreement: (replaced by later agreement) Replace the agreement on the maximum number of PRBs supported by UE with the following: For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (at least for unicast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH (at least for unicast) and PUSCH. SIB1 bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for SIB1 (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of SIB1 to be within 5 MHz - Option 2: Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: (replaced by later agreement) Replace the agreement on SIB1(PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: UE post-FFT buffering “assumption” (replaced by later agreement) OSI bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for broadcast OSI (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of OSI PDSCH to be within 5 MHz - Option 2: Allow the scheduling of OSI PDSCH to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: (replaced by later agreement) Replace the agreement on broadcast OSI (PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation) Paging bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of paging channel to be within 5 MHz - Option 2: Allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous RAR bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of RAR PDSCH to be within 5 MHz - Option 2: Allow the scheduling of RAR PDSCH to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous PUSCH bandwidth Agreement: [38.213] For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: [38.213] - For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: (replaced by later agreement) - For UE BB bandwidth reduction, it is FFS whether a UE can be expected to receive an UL grant in a RAR with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

RAN1#111:

R1-2212533 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212534 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212535 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212536 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Number of PRBs Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH and PUSCH. UE post-FFT buffer size Conclusion: (no spec impact) For UE BB complexity reduction, for broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz. Unicast PDSCH bandwidth Agreement: [38.213] - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e SIB1/OSI Conclusion: (no spec impact) For UE BB complexity reduction, broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. Paging bandwidth Agreement: (replaced by later agreement) From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). RAR bandwidth and Msg3 transmission time Agreement: [38.213] For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Msg3 bandwidth Agreement: [38.213] For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

RAN1#112:

R1-2301886 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301887 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301888 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301889 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Initial BWP Conclusion: (no spec impact) There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Number of PRBs Agreement: [38.213] For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Paging bandwidth Agreement: [38.213] Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. RAR bandwidth and Msg3 transmission time Agreement: [38.213] For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs MsgA bandwidth Agreement: [38.213] For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Msg4 bandwidth Working assumption: (replaced by later agreement) - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Simultaneous reception Conclusion: (no spec impact) For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

RAN1#112bis-e:

R1-2303933 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2303934 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2303935 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2303936 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2303937 FL summary #5 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2304261 FL summary #6 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) SIB1/OSI bandwidth Agreement: [38.213] Update the agreements for SI PDSCH with the clarification as follows: - For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation). The scheduling of SIB1 PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. - For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation). The scheduling of OSI PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Random access timeline and early indication Agreement: [38.213, 38.321, 38.331] Down-select one among the following options in RAN1#113: - Option 1: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 2: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 3: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - FFS: Whether legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 4: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). Agreement: [38.213] The potential timeline relaxations for the following cases are FFS: - For 2-step RACH: - Case 2a: Between reception of fallbackRAR and transmission of Msg3 - Case 2b: Between reception of successRAR and transmission of corresponding HARQ-ACK - For 4-step RACH: - Case 4a: Between reception of RAR PDSCH in which UE does not correctly receive the transport block and upcoming transmission of PRACH - Case 4b: Between reception of RAR with RAPID which is not associated with the corresponding PRACH transmission and upcoming transmission of PRACH Msg4 bandwidth Agreement: [38.213] Confirm the following working assumption by assuming that Msg3 indication is available: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Agreement: (LS) [Draft] LS R1-2304258 is endorsed in principle with changing “to specify” to “to consider”. Final LS R1-2304262 is endorsed Simultaneous reception Conclusion: (no spec impact) For UE BB bandwidth reduction, for autonomous SI acquisition, the following paragraph in TS 38.214 clause 5.1 still applies: - “The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.” - FFS: Msg4 PDSCH scheduled by TC-RNTI case

- UE peak data rate reduction

RAN1#110bis-e:

R1-2210248 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210249 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210250 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2210251 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, (replaced by later agreement) - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, [38.306] - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary.

RAN1#111:

R1-2212533 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212534 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212535 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2212536 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Agreement: (replaced by later agreement) - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL

RAN1#112:

R1-2301886 FL summary #1 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301887 FL summary #2 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301888 FL summary #3 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) R1-2301889 FL summary #4 on Rel-18 RedCap UE complexity reduction Moderator (Ericsson) Agreement: Revise the earlier agreement by removing the square brackets like this: [38.306] - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: [38.306] For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

# References

- RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”

- R1-2300177, “WI work plan for Rel-18 RedCap”

- TR 38.865 V18.0.0, “Study on further NR RedCap UE complexity reduction”

- R1-2301881, “RAN1 agreements for Rel-17 NR RedCap”

- R1-2304262, “LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs” (from RAN1 to RAN2)