3GPP TSG RAN WG1 #112bis-e	R1-2303952

e-Meeting, April 17th – 26th, 2023

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary#3 on 9.9.1 Coverage enhancement for NR NTN

Agenda Item:	9.9.1

Document for: 	Discussion and Decision

- Introduction

This document is made for discussion on coverage enhancement for NR NTN. Schedule for discussion is below in UTC time. FL requests companies to consider the schedule.

Week 1

- Meeting start: Monday UTC 9:00

- 1st round input: – Tuesday UTC 9:00

- 1st GTW session: Tuesday UTC 20:30~23:30

- 2nd round input: – Thursday UTC 9:00

- 2nd GTW session: Thursday UTC 20:30~23:30

- Discussion via reflector

- 1st check point: Friday UTC 16:59

- Quiet period: UTC 23:59 April 21st – UTC 23:59 April 23rd (48 hours)

Week 2

- 3rd round input / Discussion via reflector: – Monday UTC 6:00

- Discussion via reflector

- [3rd GTW session: Wednesday UTC 12:00~15:00]

- 2nd check point: Wednesday UTC 16:59

This topic is mentioned in Rel-18 NR NTN WID as captured in Appendix-1. As discussed/concluded at the previous RAN plenary meeting, we focus on coverage enhancement of PUCCH for Msg4 HARQ-ACK and discussion of DMRS-bundling for PUSCH. Although FL found that several companies propose other mechanisms in their contributions, they will not be summarized/handled since not aligned with the WID description.

In this meeting, FL’s plan is to agree at least the following aspects.

- For PUCCH of Msg4 HARQ-ACK

  - Down-selection of:

    - Signaling for repetition request or capability report

    - Dynamic indication

    - RSRP threshold

- For PUSCH DMRS-bundling

  - Conclude discussion on phase difference limit

  - Agree high-level concept of TDW determination

FL assumes that at least the following should be discussed in the next meeting while plan may be changed after further consideration.

- For PUCCH of Msg4 HARQ-ACK

  - xxx

- For PUSCH DMRS bundling

  - xxx

In addition, ‘contact information’ in the last section is copied from the summary at the last meeting. Anyone can use/add/update/remove some of the list if necessary.

- Collections of agreements/conclusions in RAN1#112bis-e

Observation

For NTN-specific PUSCH DMRS bundling,

- In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

  - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed.

  - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier.

Working assumption

For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - FFS: how to determine the actual TDW

- FFS: specification impact

- Send an LS to RAN4

Agreement

Final LS is endorsed in R1-2304094 with the following revision to the action:

ACTION: RAN1 respectfully asks RAN4 to take the above RAN1 observations and agreement working assumption into account.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE.

- Option B: Higher layer signaling in Msg3 PUSCH

Send an LS to RAN2 to ask the feasibility of Option B, and if feasible, to specify the details of Option B.

Comeback for LS – Shohei (Docomo)

Agreement

For NTN-specific PUSCH DMRS bundling, support Alt 2 for TDW determination.

- Alt 2: gNB-centric TDW determination

  - Nominal TDW is determined based on gNB configuration.

  - Actual TDW is determined based on gNB configuration/indication.

  - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN.

  - FFS: details, including UE capability and assistance information reporting

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, support Alt 1-1 for dynamic indication of repetition factor from gNB. Further discuss which field(s) to be used.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field(s)

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, apply frequency hopping mechanism in R15/16/17 defined for PUCCH transmission for Msg4 HARQ-ACK, in every slot.

- Proposals for agreements/conclusions

Proposal 1-1_v3

PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for:

- PUCCH transmission when dedicated PUCCH resource configuration is not provided.

  - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource, i.e.,

    - The same configuration of PUCCH repetition provided via SIB is applied to any PUCCH transmission by using common PUCCH.

    - The same signaling of repetition request or capability report from UE is used for any PUCCH transmission by using common PUCCH.

  - FFS: how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_0 and with CRC scrambled by C-RNTI.

  - FFS: whether/how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_1 and with CRC scrambled by C-RNTI.

Proposal 1-2a

R1-230xxxx              [Draft] LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK   Moderator (NTT DOCOMO, INC.)

Final LS is endorsed in R1-230xxxx.

Proposal 1-4_v3

Update the working assumption at RAN1#112 with down-selection of Alt B for RSRP threshold to determine whether UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request or not.

Decide at RAN1#113 whether or not RAN1 confirms the working assumption at RAN1#112 including Alt A and Alt B for RSRP threshold to determine whether UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request or not.

- If the working assumption is confirmed as an agreement, Alt B is down-selected.

  - Alt B: New RSRP threshold is introduced.

    - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

    - FFS: how to define new RSRP threshold (e.g., absolute value, relative value to the RSRP threshold for R17 Msg3 repetition)

  - Note: This update does not imply the working assumption is confirmed as an agreement.

Proposal 1-4_v2

Update the working assumption at RAN1#112 with down-selection of Alt B for RSRP threshold to determine whether UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request or not.

- Alt B: New RSRP threshold is introduced.

  - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

  - FFS: how to define new RSRP threshold (e.g., absolute value, relative value to the RSRP threshold for R17 Msg3 repetition)

Note: This update does not imply the working assumption is confirmed as an agreement.

Proposal 1-6c_v3

For PUCCH repetition for Msg4 HARQ-ACK, the following RRC signaling-related aspect is up to RAN2 decision.

- Whether the factor can be ‘1’ when only one repetition factor is configured via SIB

Note: If ‘1’ is configured, RAN1 understands that the UE behavior is the same as in R17 specification.

Proposal 1-6c_v2

For PUCCH repetition for Msg4 HARQ-ACK, the following RRC signaling-related aspect is up to RAN2 decision.

- Whether the factor can be ‘1’ when only one repetition factor is configured via SIB

Note: If ‘1’ is configured, RAN1 understands that UE behavior is the same as in R17 specification.

- Discussion

As in the previous meeting, FL recommends companies to use the following values for discussion in this meeting if any and for future evaluations. Still FL found that some companies are using different values to make observations/proposals. Observations/proposals may be changed dependent on the applied CNR value; thus different values should not be chosen among companies.

| Case | Satellite orbit | Satellite parameter set | Elevation angle [degree] | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP [dBm] | RX: G/T [dB/T] | No. of PRBs | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | LEO-1200 | 1 | 30 | 2.0 | -5.5 | 18.0 | 1.1 | 1 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -8.1 |
|  |  |  |  |  |  |  |  | 2 | 0.36 |  |  |  |  |  |  | -11.1 |
|  |  |  |  |  |  |  |  | 3 | 0.54 |  |  |  |  |  |  | -12.9 |
|  |  |  |  |  |  |  |  | 4 | 0.72 |  |  |  |  |  |  | -14.1 |
|  |  |  |  |  |  |  |  | 6 | 1.08 |  |  |  |  |  |  | -15.9 |

  - PUCCH enhancements for Msg4 HARQ-ACK

    - [Email/High] Design target / terminology

RAN#99: NTN_enh offline discussion on Tuesday morning coffee break (draft discussion uutcomes)

Dear Nicolas and all, Thank you very much for moderating the discussion, and sorry for sending email just before the deadline. Regarding PUCCH enhancements for Msg4 HARQ-ACK, we had some offline discussion with companies. A number of companies is getting the intention of following Option 1 that it tried to address the coverage hole between 1) Option 2 and 2) PUCCH with dedicated PUCCH resource configuration. As discussed in our contribution RP-230358, if the configuration is not provided via Msg4 PDSCH, feedback of one or more subsequent PDSCH is conveyed on a common PUCCH. The coverage issue should be addressed together. Option 1: PUCCH transmission when dedicated PUCCH resource configuration is not provided Option 2: PUCCH transmission when PUCCH resource is indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI. However, it seems some companies need more time to consider this issue. Also, some other companies have concern whether the corresponding solution would be applied to CONNECTED mode as well if Option 1 is adopted and dedicated PUCCH resource configuration is not provided at all. Considering current situation, we would like to propose to set a checkpoint in RAN#100 whether there are missing cases in which “PUCCH enhancements for Msg4 HARQ-ACK” should address the coverage issue, and if identified, task WGs to address the issue. In 2Q’23, WGs can proceed their work according to the current WID. Proposal: Checkpoint in RAN#100 whether there are missing cases in which “PUCCH enhancements for Msg4 HARQ-ACK” should address the coverage issue, and if identified, task WGs to address the issue Best regards, Shinya NTT DOCOMO, INC.

Dear Shinya, all, Thank you for your response. May be best not to do anything at this meeting (no WID revision, no chair’s note). However, let us discuss how to revise the WID if needed at RAN#100, based on a comprehensive proposal. Best regards Nicolas PS: This doesn’t prevent companies to socialize further the issue at next RAN1 WG meetings

At RAN#99 meeting, it was discussed whether WID should be revised to cover the case where PUCCH transmission when dedicated PUCCH resource configuration is not provided, but the conclusion was pending. (Probably) correspondingly, FL found that there are more inputs on this issue in this RAN1 meeting, and also that discussion in subsequent sections may be related to this issue; thus, this section is prepared.

At least 4 companies believe that PUCCH rep for Msg4 HACK-ACK should also be applied to subsequent PUCCH transmissions before dedicated PUCCH resource configuration is provided. In [12/Ericsson]’s phrase as an example:

- [12/Ericsson] RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation. During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later.

This situation can be illustrated as follows.

FL would like to ask whether this issue can be solved in RAN1.

      - 1st round

Q: Do you think the above illustrated situation is valid? If NO, please share the reason.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Sharp | Yes | The network is allowed to put RRC configuration of dedicated PUCCH resources (e.g. PUCCH resources with R15 PUCCH repetitions) in Msg4 PUSCH. If R15 PUCCH repetition is the pre-requisite feature of Rel-18 Msg4 HARQ-ACK repetition and when a UE requested Rel-18 Msg4 HARQ-ACK repetition, then the network is aware of that the UE is also capable of R15 PUCCH repetition. Therefore, that network may put R15 PUCCH repetition configuration in Msg4 PUSCH. |
| LG | YES | We think that PUCCH repetition for Msg4 HACK-ACK should also be applied to another PUCCH transmissions before dedicated PUCCH resource configuration is provided. |
| OPPO | Yes | The Msg4 PUCCH repetition should also be applied to the PUCCH transmission when the dedicated PUCCH resource configuration is not provided |
| MediaTek | YES | Enhancements for MSG4 HARQ Ack repetitions can be discussed and agreed first, and used as baseline for the subsequent PUCCH transmissions before dedicated PUCCH resource configuration. |
| Ericsson | YES | The solutions to support PUCCH repetition for Msg4 HARQ-ACK should apply for any PUCCH when dedicated PUCCH is not configured. |
| QC | Yes | Solutions to support PUCCH repetition for Msg4 HARQ-ACK should apply for other PUCCH before dedicated PUCCH is configured. |
| Apple | Yes | The Msg4 PUCCH repetition scheme could be extended before dedicated PUCCH resource is configured. We prefer to prioritize the design for Msg4 PUCCH repetition first. |
| Lenovo | Yes | Agree that the solutions for the support of PUCCH repetition for Msg4 HARQ-ACK should apply for any PUCCH, in case dedicated PUCCH is not configured. |
| Nokia, Nokia Shanghai Bell | No | If the coverage need is detected by the gNB (and the UE supports this), the gNB should configure dedicated PUCCH resources with repetitions after the Msg4. There is nothing preventing gNB to do so, so if it does not configure dedicated PUCCH resources, it means gNB believes that UE is able to transmit HARQ-ACK PUCCH even without repetitions. For this reason, we do not see a need to extend the enhancements to PUCCH transmissions when PUCCH dedicated resources are not configured. |
| DCM | Yes | R15 discussion assumed that this situation is feasible, and thus spec uses corresponding terminology. |
| Samsung | Yes | It should address both cases instead of only Msg.4 PUCCH. It is general issue. |
| Xiaomi | Yes | Support to reuse the Msg4 HARQ-ACK repetition solution for other common PUCCH. |
| China Telecom | Yes | The solutions to support PUCCH repetition for Msg4 HARQ-ACK should also be applied to subsequent PUCCH transmissions before dedicated PUCCH resource configuration is provided. |
| Panasonic | YES | PUCCH repetition should be applicable to another PUCCH transmissions before dedicated PUCCH resource is configured. Otherwise, HARQ-ACK for PDSCH containing RRC reconfiguration message after RACH procedure can not be reached to gNB. |
| Spreadtrum | Yes | PUCCH repetition for Msg4 HACK-ACK should also be used to other PUCCH transmissions before dedicated PUCCH resource is configured. |
| Huawei, HiSilicon | Yes | We agree it can happen. |
| Intel | Yes, but | We share the same view as Nokia. The illustrated case may be valid, but there is solution how to avoid it. |
| CMCC | Yes | We think that PUCCH repetition for Msg4 HACK-ACK can be applied to subsequent PUCCH transmissions before the dedicated PUCCH resource configuration is provided. |
| Thales | Yes | The Msg4 PUCCH repetition should be also considered for other PUCCH transmissions before dedicated PUCCH is configured. |
| Baicells | Yes | We believe the illustration is valid. Solutions for Msg4 HARQ-ACK repetition should apply for other PUCCH before dedicated PUCCH is configured. |
| ZTE | Yes | The msg4 HARQ-ACK should be focused in the mechanism design. The applicability of the PUCCH repetition can be extended to some common PUCCH, e.g., before dedicated PUCCH is not configured. |

FL’s observation

It seems that now most companies are aligned with the assumption that the above illustrated situation occurs. In addition, although a few companies feel that RAN1 should focus on Msg4 HARQ-ACK repetition and then consider the additional situation, it seems that more companies think that RAN1 should consider this situation from now. Given the majority’s preference, FL suggests the following proposal for the 2nd round. For the 1st online session, the proposal will not be treated since the proposal below is now prepared.

      - 2nd round

Proposal 1-1_v0

PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for:

- PUCCH transmission when dedicated PUCCH resource configuration is not provided.

  - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to this PUCCH transmission.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| FGI | YES |  |
| LG | YES |  |
| OPPO | YES |  |
| Sharp | YES |  |
| Ericsson | YES |  |
| Huawei, HiSilicon |  | In our understanding, the reason that we can extend the common PUCCH is considering after Msg4, there could be some short duration in which gNB may not know exactly the UE’s capability considering the UE may report capability after RRC connection setup. I copied the figure below from FL’s summary in the 1st round: - [12/Ericsson] RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation. During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later. This situation can be illustrated as follows.. Therefore, we consider this duration should be a temporary stage, and gNB can configure dedicated PUCCH resource when gNB knows the capability of the UE. Therefore, we prefer not to impact too much on our RAN1 work due to this extension considering we have many details need to be fixed in following meetings. Therefore, we prefer to add a note to focus on DCI format 1_0 based indication of repetitions for common PUCCH. Proposal 1-1_v0 revised PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for: - PUCCH transmission when dedicated PUCCH resource configuration is not provided. - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to this PUCCH transmission. - Note: it is assumed that DCI format 1_1 and DCI format 0_1 shall not support the dynamic indication of common PUCCH repetition. |
| ZTE |  | OK for the main bullet. But for the note, clarification is preferred on the meaning of “applied to this PUCCH transmission”. There seems to be different understanding on applying the repetition for PUCCH for msg4 HARQ-ACK to other PUCCHs: - The first understanding is to revise all the previous agreement and working assumptions for this PUCCH. Separate capability or repetition indication may be designed for different PUCCH. - Another understanding is that the repetition configuration for PUCCH for msg4 HARQ-ACK can be applied for subsequent PUCCH before dedicated PUCCH configuration. No need to consider new design. Since previous evaluation and discussion are made for PUCCH for msg4 HARQ-ACK, we prefer the second understanding which is simple and low workload. |
| Apple | Yes | About ZTE’s comment on the note, our understanding is the second (i.e., no need to consider new design). |
| Qualcomm | Yes | OK for the main bullet. Suggest to modify the subbullet - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to PUCCH transmissions without additional signaling. |
| Intel |  | In our view we should clarify the following points in the proposal: - PUCCH indicated by DCI format 1_0 shall be only considered - The same UE capability and/or UE request signaling is used for PUCCH in response to Msg4 PDSCH and other PUCCH when dedicated PUCCH resource configuration is not provided |
| Samsung |  | We would like to suggest as follows. PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for: - DCI format 1_0 with TC-RNTI/C-RNTI. There is no need to mention about dedicated RRC signaling. Regarding UE capability, it should be common for both cases. |
| Spreadtrum | YES |  |
| vivo | YES |  |
| Panasonic | YES |  |
| Nokia, Nokia Shanghai Bell | No | The WID is quite specific on this aspect, and it was also discussed extensively at RAN plenary. Further, expanding the scope of the PUCCH repetitions would de-facto force the gNB to provide dedicated PUCCH resources for all UE with support of this capability in case the gNB wants to “disable” UE’s automatically enabling this feature. And having a mixture of UEs expecting PUCCH repetition at all time instants makes the book-keeping for UE capabilities significantly more complex and potentially pushes the gNB to poor resource utilization. So we are not supportive of this proposal. |
| Baicells | YES |  |

FL’s observation

YES: 11

No: 1

Comment: 4

It seems that most companies do not have concern on this direction. Then FL suggests agreeing this proposal. For the details, some refinements would be necessary. Probably, one more round via summary is necessary. FL replies for each as follows:

To HW, FL is not sure what is the exact intention of HW’s update. If HW’s intention is not to introduce dynamic indication of repetition factor in DCI format 1_1 for common PUCCH case, at least there is no such restriction in the current spec (see 9.2.1 of 213). This point should be FFS in this stage.

To ZTE, the intention is the 2nd bullet in ZTE’s comment basically. The proposal is updated to clarify the exact intention.

To Intel, for (1), please see the above comment to HW. For (2), it is the intention. The update proposal reflects that point.

To Samsung, for C-RNTI, the current spec already covers the case when dedicated PUCCH config is provided. FL does not see any reason to tough this existing part. If your wording = DCI format 1_0 with TC-RNTI/C-RNTI is used, two schemes for 1_0 with C-RNTI are defined. Why is this required? Regarding the wording, the existing spec is already using this wording (see 9.2.1 of 213). FL does not understand the reason why the same wording should not be used.

Regarding Nokia’s comment, FL does not understand the concern. Regardless of this expansion, gNB can configure dedicated PUCCH resource as gNB wants. If resource utilization is concern, gNB can configure dedicated PUCCH as soon as possible, or gNB can indicate repetition factor = ‘1’ for UE. In short, gNB can do anything based on its preference as in conventional 3GPP NW.

      - 3rd round

Proposal 1-1_v2

PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for:

- PUCCH transmission when dedicated PUCCH resource configuration is not provided.

  - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to this PUCCH transmission any PUCCH transmission by using common PUCCH resource, i.e.,

    - The same configuration of PUCCH repetition provided via SIB is applied to any PUCCH transmission by using common PUCCH.

    - The same signaling of repetition request or capability report from UE is used for any PUCCH transmission by using common PUCCH.

  - FFS: how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_0 and with CRC scrambled by C-RNTI.

  - FFS: whether/how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_1 and with CRC scrambled by C-RNTI.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE |  | Thanks FL for the clarification. Actually, we think the simple and straightforward way is to directly apply the indicated repetition factor for PUCCH for msg4 HARQ-ACK to other common PUCCH. Then no need to consider how to indicate repetition factor for other PUCCHs as shown in current FFS. With this consideration, we suggest the following revision: Proposal 1-1_v2 PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for: - PUCCH transmission when dedicated PUCCH resource configuration is not provided. - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to this PUCCH transmission any PUCCH transmission by using common PUCCH resource, i.e., - The same repetition factor of PUCCH for Msg4 HARQ-ACK is applied to any PUCCH transmission by using common PUCCH - The same configuration of PUCCH repetition provided via SIB is applied to any PUCCH transmission by using common PUCCH. - The same signaling of repetition request or capability report from UE is used for any PUCCH transmission by using common PUCCH. - FFS: how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_0 and with CRC scrambled by C-RNTI. - FFS: whether/how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_1 and with CRC scrambled by C-RNTI. |
| LG |  | We think the last FFS bullet may not be aligned with the intention of main bullet. To be specific, in the main bullet, the PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for any PUCCH transmission using common PUCCH resource. However, in the last FFS bullet, it is left open to further discussion whether PUCCH repetition is supported in certain cases. Therefore, we prefer to remove the “whether” in the last FFS bullet. |
| FGI |  | Agree with LG. We think there is no need to preclude any enhancements for DCI format 1_1 with CRC scrambled by C-RNTI, because for a UE that has not yet been configured dedicated PUCCH resources, the coverage gaps of PUCCH are the same no matter the PUCCH including HARQ-ACK information is in response to a PDSCH reception scheduled by DCI format 1_0 or DCI format 1_1. |
| Xiaomi | Yes | We support this proposal in general. For LG’s comments, our understanding is whether directly reuse the same repetition number with Msg4 HARQ-ACK repetition for other common PUCCH or a new repetition number is indicated by DCI scrambled by C-RNTI. Therefore, we suggest to update the FFS part as following: FFS: whether the same repetition factor of Msg4 HARQ-ACK is reused for other common PUCCH transmission. If different repetition factor can be indicated, how to indicate the repetition factor for PUCCH transmission scheduled by DCI format 1_1 and DCI format 1_0 scrambled by C-RNTI. |
| Intel |  | We propose to limit the discussion to DCI format 1_0 and with CRC scrambled by C-RNTI. We don’t understand what is the use case of DCI format 1_1 before dedicated PUCCH configuration which cannot be supported with DCI format 1_0. |
| Baicells |  | The existing mechanism for PUCCH for Msg4 HARQ-ACK can be applied to any other ‘common’ PUCCH for HARQ-ACK. FFS other cases. |
| Samsung |  | Last FFS should be removed. Motivation is not clear to us. Only applying DCI format 1_0 is enough. In general, we think this agreement should be simple and only about the rewording of “PUCCH for Msg4 HARQ-ACK” to “PUCCH transmission when dedicated PUCCH resource configuration is not provided”. It can be added that past agreements apply but ongoing work/design should not be affected by this agreement. Proposal 1-1_v2 PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for: - PUCCH transmission when dedicated PUCCH resource configuration is not provided. Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK apply. |
| Ericsson | Yes |  |
| Apple | Yes | We are fine with the proposal in principle. We do not think it is proper to apply the same repetition factor as for Msg4 PUCCH for other following PUCCH transmissions, considering channel condition changes. Hence, the proposed solution by ZTE is not preferred. We are fine with LG proposed change by removing “whether” in the last bullet. In this way, we could combine the last two FFS into one. |
| OPPO |  | We share the similar view as Intel and Samsung that the use case of DCI format 1_1 scheduling PUCCH repetition is not clear, and suggest to focus on DCI format 1_0. |
| Panasonic | Yes | We are generally fine with the proposal, but we prefer to remove the last FFS on DCI 1_1 as suggested by several companies. We share the view with Apple on the repetition factor. Restricting to the same repetition as for Msg4 PUCCH would be unnecessary. |
| Huawei, HiSilicon |  | We don’t prefer to extend this common PUCCH repetition too much. The motivation of the extension of the scope is: - [12/Ericsson] RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation. During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later. This situation can be illustrated as follows.. Let’s just focus to resolve issues in the above mentioned case and do not extend it too much considering we have very limited TU. We support the view from Intel, OPPO, Samsung that only DCI format 1_0 scrambled by TC-RNTI/C-RNTI considers the indication of common PUCCH repetition number. |
| Lenovo | Yes | We support the proposal; however, we would also prefer to remove last FFS. |

FL’s observation

It seems that companies have different understanding/preference. Let FL respond to each below:

- To ZTE: your update is one way to solve the FFS parts. We can consider it, but at least agreeing the way now would be difficult. Let’s move forward little by little.

- To LGE/FGI/Intel/Samsung/Apple/OPPO/Panasonic/HW/Lenovo: Regarding whether 1_1 case is considered or not, although FL understands that use case and/or workload is the concern, actually the current 38.213 does not differentiate 1_0 case and 1_1 case for common PUCCH. FL is not sure the restriction is necessary. Considering more companies’ position, FL removes the bullet for the next version of the proposal, but please continue discussion.

9.2.1 PUCCH Resource Sets If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of N_BWP^size PRBs. …

- To Xiaomi, please see reply to ZTE. Whether the same factor is used commonly or not is the next step.

- To Baicells, when dedicated PUCCH config is not received, there is no SR/CSI configuration. Thus it is clear that only HARQ-ACK is the target.

- To Samsung, although we already tried such a simple note, there were a few clarification comments on the note. Given that, it is difficult to agree without sufficient explanation of the intention.

Further discussion is continued via reflector directly.

Proposal 1-1_v3

PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for:

- PUCCH transmission when dedicated PUCCH resource configuration is not provided.

  - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource, i.e.,

    - The same configuration of PUCCH repetition provided via SIB is applied to any PUCCH transmission by using common PUCCH.

    - The same signaling of repetition request or capability report from UE is used for any PUCCH transmission by using common PUCCH.

  - FFS: how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_0 and with CRC scrambled by C-RNTI.

  - FFS: whether/how to indicate repetition factor for PUCCH transmission scheduled by DCI format 1_1 and with CRC scrambled by C-RNTI.

      - 4th round

Discussed via reflector.

    - [Email/High] Signaling for repetition request or capability report

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports

At the last meeting, the above working assumption was reached. FL observed that all companies are discussing these three options in their contributions and there is no observation/proposal to change this working assumption. Therefore, FL recommends focusing on down-selection of one option from these three.

- For Option A, 11 companies support this option while at least 11 companies raise their concerns explicitly.

  - The main motivation is to reuse Msg3 rep mechanism.

  - The main concern is further PRACH resource fragmentation.

  - As the counterargument, FL can find suggestion of introducing some mechanism to use the same preamble/occasion for e.g., both Msg3 repetition request and Msg4 PUCCH repetition-related information.

  - However, FL is not sure whether this way can solve their concern since anyhow at least both dedicated resources for Msg3 rep and the shared resources are necessary to accommodate R17 UEs and R18 UEs.

- For Option B, 15 companies support this option while at least 5 companies raise their concerns explicitly. Besides, several companies comment for Option B that decision should be up to RAN2.

  - There are two main concerns for this option.

  - The first one is whether this option is feasible or not is unclear from RAN1 perspective; this issue can be solved by sending an LS to RAN2.

  - The second concern, which is pointed out by [2/Nokia, NSB] [4/OPPO], is an ambiguity issue due to UE collision. Two UEs may use the same PRACH resource/occasion and then only either corresponding Msg3 PUSCH may be received at gNB. The resulting Msg4 PUCCH indication may be intended for UE capable of the PUCCH rep, or may be intended for UE incapable of the PUCCH rep. As a result, DCI misinterpretation may occur. However, FL’s understanding is that this issue can be solved by adopting appropriate option for the dynamic indication (e.g., option 1-1d) and/or by appropriate gNB implementation.

  - Therefore, FL does not find any technical issue from RAN1 perspective.

- For Option C, 3 companies support this option while at least 12 companies do not prefer this option.

  - There seems to be so many concerns mainly from gNB implementation complexity. FL suggest deprioritizing this option.

Based on the above analysis, FL’s suggestion is to take Option B from RAN1 perspective and send an LS to RAN2 to ask the feasibility. If infeasible, possible RAN1 choice is only Option A, and thus Option A is agreed instead of Option B.

      - 1st round

Proposal 1-2_v0

For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE.

- Option B: Higher layer signaling in Msg3 PUSCH

  - E.g., LCID codepoints, R bits in the MAC subheader

- Send an LS to RAN2 to ask the feasibility of Option B.

  - Note: if feasible, which signaling of higher layer is used is up to RAN2 decision.

- If Option B is infeasible from RAN2 perspective, Option A is supported instead.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Sharp | YES | We support the proposal. |
| LG | NO | We prefer to support Option A. This is because, the Rel-17 Feature combination framework can be used for this issue. Also, we can also handle PRACH resource fragmentation by combining the features of Msg.4 HARQ-ACK PUCCH repetition with those of Msg.3 PUSCH repetition. |
| OPPO | NO | We prefer Option A and its feasibility has been verified by R17 Msg3 PUSCH repetition. However, for Option B, the UEs with and without Msg4 PUCCH repetition capability cannot be differentiated by PRACH resource, so the ambiguity issue may happen when UEs incapable of Msg4 PUCCH repetition(UE1) and capable of Msg4 PUCCH repetition(UE2) select the same PRACH resource to initiate a random access procedure. As illustrated in our contribution, when the Msg3 PUSCH of UE1 is successfully received by the gNB, UE2 cannot realize it and will assume its Msg3 PUSCH was received. As a result, UE2 may mistakenly interpret the Msg4 PUCCH repetition factor indicated in the DCI field and perform Msg4 PUCCH repetition, resulting in continuous interference. Therefore, regarding Option B, we think the ambiguity issue should be firstly discussed. |
| MediaTek | YES | RAN2 can discuss higher layer signaling in Msg3 PUSCH. The 3rd bullet in FL proposal is not needed at this stage. |
| Ericsson | YES, but | the third bullet is not needed. |
| QC | No | Since Msg3 enhancement already fragment the PRACH resource, Option A should not be considered unless it is shown that the impact is acceptable. Suggest to agree to send an LS to RAN2 to discuss the feasibility of Option B first. |
| NEC | Yes | Support the proposal |
| Apple | No | We prefer Option A as it is a legacy/workable way. Regarding Option B, the ambiguity issue between a UE with Msg4 PUCCH repetition capability and a UE without Msg4 PUCCH repetition capability mentioned by OPPO may be examined first. Also, the signaling overhead/feasibility in Option B needs to be checked in RAN2 before RAN1 can agree on it. |
| Lenovo | Yes | We support the proposal and fine with option A. |
| Nokia, Nokia Shanghai Bell | No | We are still concerned of the implications of choosing this option, as described in our Tdoc. It is correct that using the DAI field (as hinted in Proposal 1_3-v0 below) would relax the concern, but that has not been agreed, especially in light of the still open point of whether the enhancement applies only to the Msg4 HARQ-ACK or other PUCCH before dedicated configuration, since the DAI field is not anymore reserved for the DCIs scheduling such other PUCCH (as outlined in our response in section 4.1.1.1). Can @FL also explain to us how the Msg3 collision concern can be resolved by gNB implementation? Additionally, we find that this Option B has some potential limitations with respect to the logical channels, since the current reserved LCIDs may also be used for other purposes, while at the same time there may be limitations to the actual logical channels that may be reported/indicated by the UE for the RRC connection setup message. For this reason we would be supportive of asking RAN2 if they acknowledge the risks associated with the use of R bits (lack of future updates), use of LCID codepoints (lack of flexibility for indication of actual logical channel to be indicated). |
| DCM | Yes | We are not sure Option B has critical issue, but discussion for the UE ambiguity issue raised by several companies may be necessary. |
| Samsung | No | We think that at least 8 LCID codepoints might be necessary because eRedCap will be also introduced in Rel-18 specification. Since only 7 LCID codepoints are available in MAC spec, option A will not be feasible solution. Although R bits can be considered further, it should be reserved for other potential important feature that cannot utilize other option(s). Since option A is still feasible (similar to legacy) and without any major drawback, it should be considered. |
| Xiaomi | Yes | We are fine with this proposal, but the last sentence that If Option B is infeasible from RAN2 perspective, Option A is supported instead should be removed. |
| China Telecom | Yes | We support the proposal. |
| Panasonic | NO | We prefer Option A as used in earlier releases. Too many PRACH resource partitioning can be avoided by restricting combinations among PRACH repetition, Msg3 PUSCH repetition and Msg4 PUCCH repetition. In addition, as OPPO mentioned, ambiguity issue between Msg4 PUCCH repetition capable UE and Msg4 PUCCH repetition incapable UE can be solved by Option A. |
| Spreadtrum | No | We think Option A should be first choice, because it is a legacy way and has less impact on spec. What’s more, R17 Feature combination framework can be used for avoiding further fragment of RACH resource. Considering the relationship between PUCCH and Msg3 PUSCH, combine the features of Msg.4 HARQ-ACK PUCCH repetition with Msg.3 PUSCH repetition which could also avoid the further fragment issue. For Option B, we should first send LS to RAN2 to confirm the feasibility of Option B. Then, discuss which one is preferable. |
| Huawei, HiSilicon |  | If we could not achieve consensus on the proposal, we can firstly exclude option C as progress, which clearly lacks sufficient support and received more concerns from companies. Also we agree with Apple that the ambiguity issue raised by OPPO should be clarified first before we agree on option B directly. |
| Intel | Yes | Support the proposal |
| CMCC | No | We slightly prefer Option A, it is feasible that UE requests repetition or reports capability dynamically through Msg1 by using a dedicated PRACH preamble or RACH occasion. For Option B, the feasibility is not clear yet, and the ambiguity issue raised by some companies also needs to be clarified |
| Sharp 2 |  | Could anyone elaborate the aforementioned ambiguity issue? Our understanding is that ACK for Msg4 is transmitted by only if the UE successfully completed contention resolution. Other UEs that did not successfully complete the contention resolution do NOT transmit either ACK or NACK. Therefore, the collision issue never happens. Do we miss anything? |
| Thales | No | First send an LS to RAN2 on the feasibility of Option B |
| Baicells | Yes | Support sending an LS to RAN2 to ask the feasibility of Option B. |
| ZTE | Yes | The majority choice is option B and can be agreed. The detailed design should be within RAN2 scope. The only thing we need to do is to inform RAN1 opinion on down-selection. For PUCCH repetition for Msg4 HARQ-ACK, support Option B to indicate the repetition request or capability report. - Option B: Higher layer signaling in Msg3 PUSCH - Send an LS to RAN2 to to specify the details - If any concerns on Option B is identified from RAN2 perspective, Option A is supported instead. |

FL’s observation

Support: 12 (with/without modification)

Not support: 9

Still there are supports of Option A from almost half number of companies. The main argument seems to be the ambiguity issue in Option B. Before updating the proposal, FL would like to ask companies whether this ambiguity issue is valid or not. FL’s assumption is that if the issue is valid and difficult to be solved, then what we can go with is only taking Option A. Otherwise, the proposal 1-2_v0 has no issue from RAN1 perspective, and thus it can be agreed with sending an LS to RAN2. Further discussion in the 2nd round will be necessary, and thus this proposal is not treated in the 1st GTW.

      - 2nd round

In Option B, the same PRACH resource may be used by two UEs at the same time. Then gNB receives the PRACH and transmits RAR to the two UEs without differentiation. After that, UE-1 and UE-2 receives the RAR and transmits Msg3 by using the same resource. They are collided, and gNB may decode only either Msg3. Then gNB transmits Msg4 to the two UEs without differentiation and corresponding PUCCH can be transmitted by the two UEs.

If UE-1 is incapable of this PUCCH rep and UE-2 is capable of this PUCCH rep, and when gNB detects Msg3 from UE-1, gNB schedules PUCCH without rep. However, UE-2 receives the Msg4 and may interpretate PUCCH rep is scheduled. As a result, UE-2 transmits PUCCH rep that is not intended by gNB.

Fig.1 in [4/OPPO]

This is FL’s understanding of what Option A supporters pointed out. Meanwhile, FL’s thinking is that this issue can be avoided by gNB implementation. For example, if dynamic indication is conveyed on DAI field in DCI scheduling Msg4, the field for UE-1 can be set as codepoint corresponding to repetition factor ‘1’ since this field is reserved and thus there is no impact on UE-1 behavior. Then UE-2 assumes no rep. Although this solution cannot be applied when only a single factor is configured, gNB can configure multiple factors if this issue is critical from NW perspective.

The above is just FL’s feeling, so let us discuss the above issue.

Q1. Do you think this above situation (as illustrated above) is valid?

Q2. If answer of Q1 is YES, do you think this issue can be solved in Option B by e.g., NW implementation?

| Company | Q1 (Y/N) | Q2 (Y/N) | Comment |
| --- | --- | --- | --- |
| FGI | N |  | Even if both UE1 (legacy UE) and UE2 (Rel-18 UE) decodes the DCI format 1_0 with CRC scrambled by TC-RNTI successfully, since the DCI is for UE1, UE2 may or may not decode the Msg4 PDSCH successfully, depending on which field in the DCI is used to indicate repetition factor. - If UE2 decodes Msg4 PDSCH successfully, as per TS 38.321, if the MAC PDU contains a UE Contention Resolution Identity MAC CE and if the UE Contention Resolution Identity in the MAC CE does not match the CCCH SDU transmitted in Msg3, the MAC entity shall consider this Contention Resolution not successful. If the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful, the MAC entity shall not instruct the physical layer to generate acknowledgement(s) of the data in this TB. Therefore, it is clear that UE2 shall not transmit the Msg4 PUCCH, although UE2 will incorrectly interpret the DCI field(s) of the DCI scheduling Msg4 PDSCH. - If UE2 does not decode Msg4 PDSCH successfully, as per TS 38.321, UE2 will not transmit Msg4 HARQ-ACK and will keep monitoring DCIs addressed by TC-RNTI until the CR timer expires. In both cases, UE2 does not transmit Msg4 HARQ-ACK and hence we think this is not an issue. |
| Sharp | N |  | We have same understanding as FGI. For the following explanation from FL, the yellow part is not correct. According to TS38.321, ACK for Msg4 is transmitted by only if the UE successfully completed contention resolution. Other UEs that did not successfully complete the contention resolution do NOT transmit either ACK or NACK. Therefore, PUCCH collision never happen. In Option B, the same PRACH resource may be used by two UEs at the same time. Then gNB receives the PRACH and transmits RAR to the two UEs without differentiation. After that, UE-1 and UE-2 receives the RAR and transmits Msg3 by using the same resource. They are collided, and gNB may decode only either Msg3. Then gNB transmits Msg4 to the two UEs without differentiation and corresponding PUCCH can be transmitted by the two UEs.. |
| LG | N |  | We have similar views with FGI and Sharp regarding above situation. However, Option B creates another restriction in the discussion of dynamic indication field in DCI format 0_1 (still discussed in Proposal 1-3). To be specific, if UE1 is incapable of PUCCH repetition and UE2 is capable of PUCCH repetition, and when gNB detects Msg. 3 PUSCH from UE2, then gNB schedules Msg. 4 HARQ-ACK PUCCH with repetition. In this situation, if a DCI field (which is not reserved) is used for dynamic indication (such as MCS, PRI, HPN etc.), UE1 may not successfully decode the DCI for Msg. 4 PDSDCH. This means that the dynamic indication field must be a DAI field since it is a reserved field. Therefore, we prefer to support Option A. If we go with Option A, UEs with incapable of PUCCH repetition will not use the same preamble index group as UEs with capable of PUCCH repetition, so clear UE/gNB behaviors are guaranteed in Tx/Rx of Msg. 3 PUSCH and Msg. 4 PDSCH. |
| OPPO | Y | N | Q1: In our understanding, TS 38.321 does not provide conclusive information on the time when Msg4 HARQ-ACK is transmitted, i.e., “if the MAC PDU is successfully decoded” or “if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3”. For the former one, the Msg4 HARQ-ACK is just the feedback of Msg4 PDSCH reception in the physical layer, but for the latter one, the disassembly and demultiplexing of the MAC PDU in the higher layer should also be involved. Furthermore, according to FGI’s interpretation, neither UE1 (legacy UE) and UE2 (Rel-18 UE) will transmit NACK in Msg4 PUCCH because Msg4 HARQ-ACK transmission happens only if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3, which seems a bit weird. If so, Msg4 PDSCH retransmission will not be scheduled due to lack of NACK feedback. Therefore, we think the Msg4 HARQ-ACK is the response of Msg4 PDSCH reception in the physical layer and the ambiguity issue may exist when the repetition request/capability report is transmitted by Msg3 PUSCH. Q2: Which DCI field is used for indication of Msg4 PUCCH repetition factor is still under discussion, so we cannot make a conclusion that the issue can be solved by gNB implementation. |
| Sharp2 |  |  | Regarding the restriction mentioned by LG: We don’t think any restriction would be necessary, though our preference is to use the DAI field for the dynamic repetition factor indication. Basically, when UE1 and UE2 had sent Msg3 based on the same RAR but the gNB d sent Msg4 to the UE2 but not to the UE1, one of the following would happen at the UE1 – (1) the UE1 fails to detect the DCI; (2) the UE1 detects the DCI but fails to decode the scheduled PDSCH; and (3) the UE1 detects the DCI and successfully decode the PDSCH but Contention Resolution Identity is not matched. If we use MCS field for the indication of repetition factor, the UE1’s interpretation is wrong, which results in (2). If PRI or HPN is used, it can decode PDSCH but results in (3). In any case, we see any problem. Regarding the interpretation of TS38.321 behavior mentioned by OPPO: both two conditions “if the MAC PDU is successfully decoded” and “if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3” need to be met to instruct the PHY layer to generate acknowledgement. In other words, at least one of these conditions is not met, the MAC shall not give the instruction to PHY. This means neither ACK nor NACK is generated in PHY. Regarding the Msg4 retransmission mentioned by OPPO: If the gNB had transmitted Msg4 but did not receive ACK on the PUCCH resource, then it can take Msg4 retransmission attempt unless the contention resolution timer is expired. When it retransmits Msg4, it would toggle the NDI field value in the DCI. |
| Ericsson | N |  | We have the same understanding as FGI and Sharp. Only the UE that successfully completes contention resolution (by successfully decoding Msg4 and finding a matching UE contention resolution identity) will transmit Msg4 HARQ-ACK PUCCH. If UE contention resolution identity does not match, the UE will not send HARQ-ACK. If decoding of Msg4 is unsuccessful, UE will not transmit NACK. Regarding LG’s concern, we do not think this is an issue either. We agree that “if UE1 is incapable of PUCCH repetition and UE2 is capable of PUCCH repetition, and when gNB detects Msg. 3 PUSCH from UE2, then gNB schedules Msg. 4 HARQ-ACK PUCCH with repetition. In this situation, if a DCI field (which is not reserved) is used for dynamic indication (such as MCS, PRI, HPN etc.), UE1 may not successfully decode the DCI for Msg. 4 PDSDCH.” But failed DCI decoding for UE1 is not an issue since the Msg4 transmission was not intended to UE1. |
| Huawei, HiSilicon | N |  | If the mentioned ambiguity case is as above, we don’t think it exists. In TS 38.213, there is description as following. The UE shall provide HARQ-ACK when contention resolution identity in Msg4 is checked matched. Therefore, there shall be no ambiguity issue as mentioned above. ## 8.4 PDSCH with UE contention resolution identity In response to a PUSCH transmission scheduled by a RAR UL grant when a UE has not been provided a C-RNTI, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity [11, TS 38.321]. In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same active UL BWP as the PUSCH transmission. A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding PUCCH transmission with the HARQ-ACK information is equal to NT,1+0.5 msec. NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].. |
| ZTE | No |  | Agree with FGI. When UE decode PDSCH of msg4, it is able to know whether this msg4 is for this UE by comparing the UE identity transmitted in msg3 and received in msg4. When UE does not decode PDSCH for msg4 successfully, the Contention Resolution is not yet successful so that no HARQ-ACK will be transmitted. Hence, there will be no ambiguity issue. |
| QC | No |  | If UE2 decodes Msg4, it knows that it is not for UE2; if it does not decode Msg4, it understands contention resolution fails and will not transmit HARQ-ACK. |
| Samsung |  |  | Q2: As FL said, NW may configure only 1 repetition that is same situation in legacy specification. However, we wonder why the PUCCH repetition enhancement is necessary if gNB would not configure multiple repetition factor. Also, we would like to point out again that using unused LCID code point is NOT valid since RAN2 agreed to support early indication of UE capable of “eRedCap” using msg. 3 PUSCH. Thus, from our assessment, 8 code points are necessary: 2 (different CCCH size) X 2 (Redcap or not) X 2 (eRedcap or not). This is impossible to do this because only 7 code points are reserved in Rel-17 MAC specification. |
| OPPO2 |  |  | If most companies agree that only ACK can be transmitted in Msg4 PUCCH by the UE whose Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3, we are also OK the aforementioned ambiguity issue will not happen. But the feasibility of Option B should be further confirmed by RAN2. |
| Spreadtrum | No |  | If UE2 decode PDSCH of msg4, it will know whether this PDSCH is for itself through comparing Contention Resolution Identity in the MAC CE of PDSCH and CCCH SDU transmitted by itself. If UE2 does not decode Msg4, contention resolution is failed and no HARQ-ACK will be transmitted. So there isn’t a ambiguity issue. |
| vivo | NO |  | In our understanding, UE is able to know whether this decoding msg4 is targeted for this UE by checking the decoded UE ID. Thus, this above situation is invalid. |
| Panasonic |  |  | The collision would not happen. But, this necessarily means that Option B is preferred option. Consulting with RAN2 is necessary. |
| Nokia, Nokia Shanghai Bell | No |  | We think that companies are assuming a certain UE behavior in case Msg4 is not successfully decoded that is neither specified in 38.321 nor in 38.213. Section 8.4 of 38.213 instructs the UE on what to do in case the UE ID is in Msg4, while section 5.1.5 of 38.321 instructs the UE on what to do in case Msg4 contains the UE ID and in case Msg4 does not contain the UE ID. Neither of the specifications actually give UE any instruction on what to do in case the decoding of the Msg4 fails. However, if all companies believe that such missing UE behavior implies that UE does nothing, we are ok with it and can move forward |
| Baicells | No |  | Agree with Huawei. |

FL’s observation

Valid: [1]

Invalid: 12

Given this situation, we can conclude that the raised ambiguity issue is not valid. In this case, the concern of Option B is only feasibility of RAN2 perspective. Meanwhile, the concern of Option A = PRACH resource fragmentation seems to be remaining. Therefore, FL suggests agreeing Option B in RAN1 and asking the feasibility to RAN2. The proposal is a bit updated based on comments in the first round.

To Samsung, although FL understands the overhead aspect, 8 codepoints are available. For example, one reserved bit in MAC sub-header is used to differentiate LCID codepoint table. ‘0’ means the existing table, and ‘1’ means another table. In this case, much more codepoints are available and thus UE capability of RedCap UE / eRedCap UE can be accommodated. Of course, any other mechanism can be considered in RAN2, but at least RAN1 cannot conclude that Option B is impossible.

Proposal 1-2_v1

For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE.

- Option B: Higher layer signaling in Msg3 PUSCH

  - E.g., LCID codepoints, R bits in the MAC subheader

- Send an LS to RAN2 to ask the feasibility of Option B, and if feasible, to specify the details of Option B.

  - Note: if feasible, which signaling of higher layer is used is up to RAN2 decision.

- If Option B is infeasible from RAN2 perspective, Option A is supported instead.

After the 2nd GTW session

The following working assumption was reached with an LS to RAN2.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE.

- Option B: Higher layer signaling in Msg3 PUSCH

Send an LS to RAN2 to ask the feasibility of Option B, and if feasible, to specify the details of Option B.

Comeback for LS – Shohei (Docomo)

      - 3rd round

For the 3rd round, details of LS shall be discussed. FL found that only the working assumption reached at this meeting is insufficient for RAN2, and thus one more additional working assumption is included as a kind of information note.

Title: LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG2 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH. It is noted that repetition request or capability report is transmitted from UE based on the following working assumption. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report. 2. Actions To TSG RAN WG2 ACTION: RAN1 respectfully asks RAN2 to provide feedback on whether or not Option B is feasible from RAN2 perspective, and if feasible, to specify Option B in RAN2 specification. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN1 Meeting #113 May 22nd-26th, 2023 Incheon TSG-RAN1 Meeting #114 Aug. 21st-25th, 2023 Toulouse

Q: Do you agree the above draft LS? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE |  | We think the second working assumption is not needed in the LS. The intention of the LS is that option 3 is to report in msg3 higher layer signaling and RAN2 should consider which field to contain the report. And only the first working assumption is enough, as the second working assumption is only about when to trigger the report instead of the signaling content. Moreover, the second working assumption is not stable (updates are still being discussed) now and we think RAN1 should discuss it clear before informing RAN2. |
| LG |  | Agree with ZTE. The second working assumption may not needed. In addition, we think “the details of” can be added in action part, as in the Chair’s note. |
| FGI |  | The second working assumption may help RAN2 understand the whole picture of this feature. We think it is OK to leave with it. |
| FL |  | Let me clarify the intention of adding the second working assumption. If the second one is not included here, RAN2 may not understand what is the meaning of ‘repetition request or capability report indicated by UE’ in the first working assumption, and thus they cannot discuss/decide the feasibility. As a result, RAN2 may ask it to us and thus RAN1 progress becomes slower. To avoid such a situation, I think some clarification is necessary. But if companies prefer to remove it, I can do that. RAN2 may check the second WA in RAN1 chair’s note. Let me hear further view. |
| Xiaomi | Yes | We support the draft LS. |
| Baicells | Yes | Agree |
| Samsung |  | Although we understand FL’s intention, it is not clear to us whether 2nd working assumption is clearly saying the meaning of “repetition request or capability report” since the WA is related to which condition(s) UE need to report “repetition request or capability report”. Actually, we think that RAN2 is able to directly check RAN1 chair note even if the 2nd working assumption is not included in this LS. However, if most companies think that it is beneficial, we are okay to keep it. |
| Ericsson | Yes with comment | Fine to have the second working assumption as background information since it helps explain the term ”repetition request or capability report” in the first working assumption. RAN2 also needs to know the amount of information to be included in Msg3 PUSCH. Clarify that the repetition request/capability report is a single bit of information. |
| Apple | Yes | We are fine to include the second working assumption as background information for RAN2 to understanding the trigger of this repetition request or capability report. |
| OPPO | Yes | Agree with Ericsson and Apple that the second working assumption should be provided as background information for RAN2 |
| Panasonic | Yes | OK to include the working assumption on “repetition request or capability”. In the action part, it would be good to use the same working as in the chairman note as mentioned by LG, i.e. “RAN1 respectfully asks RAN2 to provide feedback on the feasibility of Option B, and if feasible, to specify the details of Option B.” |
| Huawei, HiSilicon | Yes | Support the comments from Ericsson and Apple that the second working assumption should be included. |
| Lenovo | Yes | Support |

FL’s observation

It can be observed that all companies are OK with this draft CR in principle. Regarding the second working assumption, more companies believe that the information is necessary for corresponding RAN2 discussion. Let’s include it. Based on Ericsson’s comment, assumed number of bits is also informed but FL would like to hear other companies’ view. Besides, the action part is updated based on comment from LGE/Panasonic.

Further discussion is continued via reflector directly.

Title: LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG2 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH. It is noted that repetition request or capability report is transmitted from UE based on the following working assumption, and one bit information for each information is assumed. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report. 2. Actions To TSG RAN WG2 ACTION: RAN1 respectfully asks RAN2 to provide feedback on whether or not Option B is feasible from RAN2 perspective, and if feasible, to specify Option B in RAN2 specification. the feasibility of Option B, and if feasible, to specify the details of Option B. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN1 Meeting #113 May 22nd-26th, 2023 Incheon TSG-RAN1 Meeting #114 Aug. 21st-25th, 2023 Toulouse

Proposal 1-2a

R1-230xxxx              [Draft] LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK   Moderator (NTT DOCOMO, INC.)

Final LS is endorsed in R1-230xxxx.

      - 4th round

Discussed via reflector.

    - [Closed/High] Dynamic indication

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

At the last meeting, the above agreement was reached for dynamic indication. Summary of the inputs from companies is the following:

- For Alt 1-1, at least 13 companies support this option while there are explicit objections from 3 companies.

  - The concerns are scheduling flexibility, difficulty of reuse, ambiguity issue as explained in section 4.1. FL’s understanding is that appropriate sub-option can be selected to avoid these issues.

  - For Alt 1-1a: Although a lot of supporting companies (8) can be found, at the same time there are multiple companies (5) having concerns on this alternative. The argument is that DL may have better channel quality and thus higher MCS for PDSCH may be used, which is different from Msg3 PUSCH rep. FL feels this is valid.

  - For Alt 1-1b/Alt 1-1c/Alt 1-1e: 4/3/2 companies support while 3/3/3 companies raise concern, respectively. Flexibility issue seems to be the main concern.

  - For Alt 1-1d: there are many supporting companies (10). The main concern from 3 companies seems to be inapplicability to the case where PUCCH rep before dedicated PUCCH parameters are not provided. However, at least [21/QC] proposes a mechanism to solve the concern. In other words, this alternative can be used with some additional rule if the above case is considered in this WI. Besides, 1 company argues that these reserved bits should be kept for future release.

- For Alt 1-2/Alt 3, there is no/little supporter while a lot of companies (10) do object these options. FL suggest deprioritizing these options.

- For Alt 2, FL can find 3 companies as supporters while 11 companies raise concerns on this option based on multiple aspects:

  - Lack of possibility to use Msg3 PUSCH RX performance for determination of repetition factor

  - Inconsistency with Option B above, if Option B is supported

  - Inflexibility and complexity of DCI indication

- For Alt 4, 6 companies prefer this option while at least 8 companies raise explicit concerns as listed for Alt 2.

From the above summary, FL’s suggestion is to take Alt 1-1 firstly, and to continue discussion for down-selection of Alt 1-1a/b/c/d/e.

      - 1st round

Proposal 1-3_v0

For PUCCH repetition for Msg4 HARQ-ACK, support Alt 1-1 for dynamic indication of repetition factor from gNB. Further discuss which field is used.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

- FFS: how to decide/indicate repetition factor if PUCCH repetition corresponding to PDSCH reception before dedicated PUCCH configuration is received and after Msg4 PDSCH is received

Q: Do you agree the above proposal? If YES, which alternative of Alt 1-1 series is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Alt | Comment |
| --- | --- | --- | --- |
| Sharp | YES | d | We support the proposal. Since what kind of RRC configuration is put into Msg4 PUSCH is up to the gNB implementation, it is better to keep the current flexibility on MCS. Therefore, Alt 1-1a is not preferred. |
| LG | YES | Alt 1-1a, Alt 1-1d |  |
| OPPO | Yes | Alt 1-1b, Alt 1-1d |  |
| MediaTek | YES | 1-1d | We agree with FL comment on Re-interpreting DAI bits. |
| Ericsson | YES | 1-1b or 1-1d | We support Alt 1-1. Down-selection of which alternative of Alt 1-1 series can wait until discussion in section 4.1.1 is concluded. |
| QC | Yes | 1-1d | I-1d has the minimum impact. |
| NEC | Yes | 1-1b |  |
| Apple | No | Alt 4>Alt 1-1b | We are not convinced by the rational against Alt 4. First of all, Option B in Proposal 1-2 has not been agreed. We cannot rely on it as argument for disabling Alt 4. Also, Alt 4 has no impact on DCI 1_0 for Msg4, we do not think the complexity of DCI indication is increased. Actually, there is no DCI change in Alt 4. In case Alt 1 is applied, our preference is Alt 1-1b. By the way, we corrected the statistics on our position in Section 5.2 and Section 4.1.3.1. |
| Lenovo | Yes | Alt 1-1b, and/or Alt 1-1d |  |
| Nokia, Nokia Shanghai Bell | No |  | We believe that the down-selection depends on agreements on other aspects. The concern we raised in terms of Msg3 collision is not solved by any of these alternatives, in consideration of the open point that the enhancement applies to all PUCCH transmissions before dedicated configuration. We ask FL to start a discussion on the issue we raised, as we would like to hear other companies’ views as well. |
| DCM | Yes | 1-1d |  |
| Samsung | Yes | Alt. 1-1a/b/c/d | If more than one bit is used, it should be considered also that the bits can be from different fields. Thus, we propose the following change Alt 1-1: One or two bits of the existing fields |
| Xiaomi | Yes | 1-1a, 1-1d |  |
| China Telecom | Yes | Alt 1-1a, Alt 1-1b |  |
| Panasonic | Yes | Alt 1-b, 1-e | We support Alt 1-1. The down-selection in Alt 1-1, we can wait progress in section 4.1.1.1(applicability of PUCCH repetition before dedicated PUCCH resource) and 4.1.2.1(ambiguity issue). |
| Spreadtrum | No | Alt 2/4 | We think we should choose the solution which has less impact on DCI design, e.g., Alt2 or Alt4. It seems feasible that PUCCH repetition factor is indicated jointly with Msg3 repetition factor or implicit indication with mapping between Msg3 repetitions and PUCCH repetition for MSG4 HARQ ACK. When Msg3 PUSCH needn’t repetition, we support Alt1-1. |
| Huawei, HiSilicon | Yes | Alt. 1-1a. | But FFS is not needed for the timing being, we have not achieved conclusion on whether extending the scope of PUCCH repetition. Regarding the comments for Alt.1-1a, we want to pointed out that for NTN case, there is no need to have high MCS scheduled for Msg4 PDSCH. |
| Intel | Yes | Alt 1-1d |  |
| ETRI | Yes | Alt 1-1a or Alt 1-1d |  |
| CMCC | Yes | Alt1-1a |  |
| Thales | Yes | Alt 1-1a, Alt 1-1d |  |
| Baicells | Yes | Alt 1-1b, Alt 1-1c, Alt 1-1d |  |
| ZTE | Yes | Alt 1-1d | DAI field is reserved for DCI scheduling msg4. Hence can be directly reused for indication. |

FL’s observation

Support: 20

- 1-1a: 7

- 1-1b: 8

- 1-1c: 2

- 1-1d: 15

- 1-1e: 1

Not support: 3

It seems that most companies are fine to go with this proposal. For further down-selection, it may be related to discussion in section 4.1.1. Let us keep all possibilities in this stage.

Regarding comments from Apple/Spreadtrum, FL understands your preference while solution for concern of Alt 2/4 is not received from your side. For example, several companies supporting Alt 1 pointed out that Alt 2/4 cannot use Msg3 PUSCH RX performance for determination of repetition factor. Other concerns can be found in section 4.1.3. Then do you think other companies can accept your preference?

For comment from Nokia, FL does not understand what is relationship between dynamic indication and Msg3 conflict. This issue is to be discussed in section 4.1.2. Besides, regardless of the discussion outcome, it seems that at least your proposal (i.e., Alt 3) cannot be agreed since we can find concerns from so many companies.

Proposal 1-3_v1

For PUCCH repetition for Msg4 HARQ-ACK, support Alt 1-1 for dynamic indication of repetition factor from gNB. Further discuss which field is used.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

- FFS: how to decide/indicate repetition factor if PUCCH repetition corresponding to PDSCH reception before dedicated PUCCH configuration is received and after Msg4 PDSCH is received

      - 2nd round

Discussed via reflector.

After the discussion via reflector

The following agreement was reached. With that, this section was closed.

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, support Alt 1-1 for dynamic indication of repetition factor from gNB. Further discuss which field(s) to be used.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field(s)

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

    - [Email/High] Information report from UE

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

At the last meeting, the above working assumption was reached. FL observed that 5 companies recommend confirming this working assumption and 17 companies continue discussion based on this working assumption, e.g., down-selection of Alt A or Alt B. Meanwhile, 6 companies believe that this working assumption should not be confirmed or should be updated before confirmation. It seems that the main part of such an argument is necessity/usage of RSRP threshold. Given that at least several companies mention the possibility of the two ways are beneficial for NW perspective, FL assumes that update regarding necessity/usage of RSRP threshold is not essential and thus reverting the working assumption is unnecessary.

Regarding Alt A vs Alt B, companies’ views can be summarized as follows:

- Alt A: Supported by 3 companies. The main point seems to be no/little difference of pathloss experienced by UE.

- Alt B: Supported by 14 companies. The main point seems to be certain difference of required RSRP between Msg3 PUSCH and Msg4 HARQ-ACK PUCCH.

FL thinks that if different RSRP threshold is intended to track pathloss change during RACH procedure, the new one would be unnecessary as stated by Alt A supporters. Meanwhile, if the different RSRP threshold is used to consider performance difference between the two UL channels, Alt B would be necessary. At least more companies support Alt B, and also FL found that [14/Samsung] pointed out that Alt B can cover Alt A by gNB implementation. Given these aspects, FL’s suggestion is to take Alt B. For Alt B, there would be multiple ways to support new RSRP threshold as proposed by several companies. The details are put as FFS.

      - 1st round

Proposal 1-4_v0

Support Alt B for RSRP threshold to determine whether UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request or not.

- Alt B: New RSRP threshold is introduced.

  - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

  - FFS: how to define new RSRP threshold (e.g., absolute value, relative value to the RSRP threshold for R17 Msg3 repetition)

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Sharp |  | For the down-selection between Alt A and Alt B, we are fine to take Alt B. However, such decision does not lead to the confirmation of the working assumption. We believe, before confirming the working assumption, we should make the decision on Msg1 based or Msg3 based. The RSRP threshold does NOT bring any benefit for Msg3 based request. If we finally choose Msg 3 based scheme, we still think the reconsideration of RSRP threshold would be needed. Meanwhile, we see some benefit for Msg1 based requesting scheme, because it enables the reduction of the dedicated PRACH resources for Msg4 HARQ-ACK repetition. |
| LG | YES | We think the Note in first sub-bullet is not necessary. |
| OPPO | Yes |  |
| MediaTek | YES | We think the FL proposal is agreeable with the Note and FFS. |
| Ericsson | YES |  |
| QC | Yes |  |
| NEC | Yes | We support the proposal as it allows higher network flexibility |
| Apple | Yes | We think the differential RSRP threshold on top of RSRP threshold for Msg3 repetition could be used for saving signaling payload. |
| Lenovo | Yes | Agree with proposal |
| Nokia, Nokia Shanghai Bell | No | As stated in our contribution (and it was also raised several times at last meeting), the RSRP threshold based approach does not provide much benefit. The RSRP threshold was introduced for coverage enhancements to be able to filter/distinguish such that only UEs that are far away from the gNB would be providing indication of capability through preamble selection. The use case for NR over NTN is completely different and the RSRP variability over a cell covered by a NTN satellite is very small. Since this is still a working assumption, and we have seen no technical arguments for the benefit of introducing the RSRP threshold, we would suggest to dismiss this proposal. |
| DCM | Yes |  |
| Samsung | Yes |  |
| Xiaomi | Yes |  |
| China Telecom | Yes |  |
| Panasonic | Yes | Agree with FL’s view. |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes | We support the proposal. We are not quite sure on what Msg1-based and Msg3-based refer to in Sharp’s comments. We assume that refers to msg1-based repetition request or Msg3-based repetition request. But we don’t think RSRP threshold conflicts with Msg3-based repetition request. Even if e.g. higher layer signaling in Msg3 is used for repetition request, the RSRP threshold can be still used to trigger this request. |
| Intel | Yes |  |
| ETRI | Yes | Agree with the proposal & FL’s view |
| CMCC | Yes | We agree to introduce new RSRP threshold for PUCCH repetition for Msg4 HARQ-ACK |
| Thales | Yes | We support the proposal. |
| Baicells | Yes | We agree with the proposal. |
| ZTE | No | RSRP threshold to determine whether to perform repetition is not necessary: Firstly, the RSRP variation within an NTN cell is small, which is different from TN. Hence, RSRP is not an efficient parameter to distinguish UEs within an NTN cell, which could have similar RSRP. Secondly, the RSRP threshold configured by network is for DL RSRP measurement, while the PUCCH repetition is a UL transmission, which are mismatched. Since the RSRP variation is small in NTN, the mismatch may have further significant impact. We are discussing NTN specific enhancement. While no necessity or benefit of introducing a RSRP threshold can be observed in NTN scenario. Hence we do not support the proposal. |

FL’s observation

Support: 20

Not support: 3

It seems that most companies are fine with this proposal. FL suggests taking this proposal as agreement as it is.

For comment from Sharp/Nokia, FL’s understanding is the same as what HW commented above. We do not need to wait for progress of another discussion.

For comment from ZTE, we already agreed that dynamic indication is supported. This would mean that RSRP is different among UEs. Besides, if you think this mechanism is not beneficial, then RSRP threshold can be disabled. Without finding critical issue, agreed working assumption will not be reverted.

Proposal 1-4_v0 (No change)

Support Alt B for RSRP threshold to determine whether UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request or not.

- Alt B: New RSRP threshold is introduced.

  - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

  - FFS: how to define new RSRP threshold (e.g., absolute value, relative value to the RSRP threshold for R17 Msg3 repetition)

      - 2nd round

Discussed via reflector.

    - [Closed/Low] Frequency hopping

As raised at previous meetings, the current specification does not clear text to do ‘intra-slot’ FH, and hence anyhow spec change and an agreement for that seem to be necessary.

### 9.2.1 PUCCH Resource Sets … The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping. …

Intra-slot FH: 6 companies. The following concerns are raised for inter-slot FH.

- Capacity issue due to inapplicability of CDM between intra-slot FH without rep and inter-slot FH with rep

- Capability issue during initial access

- No gain of detection performance

Inter-slot FH: 4 companies. The motivation is better detection performance.

No FH: 1 company. The motivation is better detection performance.

Although the current situation is not so different from previous meetings unfortunately, FL cannot find in any contribution any solution on the issues of inter-slot FH other than detection performance. Given this situation and assuming that this situation will not be changed in future meeting, FL’s suggestion is to reuse legacy frequency hopping.

      - 1st round

Proposal 1-5_v0

For PUCCH repetition for Msg4 HARQ-ACK, reuse frequency hopping mechanism in R15/16/17.

- i.e., intra-slot frequency hopping is applied when repetition is performed.

Q: Please comment ONLY if you have strong concern. FL will not update this proposal if the comment is just repeat of your preference.

| Company | Comment |
| --- | --- |
| LG | Support |
| OPPO | Support |
| MediaTek | Support |
| Qualcomm | Not support. Based on simulations reported, intra-slot hopping leads to performance loss. |
| NEC | Support |
| Lenovo | Support |
| Nokia, Nokia Shanghai Bell | In principle OK, but a bit of cross-checking with 38.213 is needed to ensure that UE performs the intended intra-slot frequency hopping for each repetition. |
| Huawei, HiSilicon | Support. |
| Thales | Support |
| Baicells | We prefer not support intra-slot hopping when repetition is performed. Because intra-slot hopping has lower performance than without hopping. |
| ZTE | In NTN, the channel will be flat for a short-term transmission. Hence, intra-slot hopping is not needed since no gain can be expected. If hopping is to be supported, inter-slot hopping can be applied for long time interval, which may provide some gain, instead of intra-slot hopping. |

FL’s observation

Support: 8

Not support: 3

Given this situation, FL suggests taking this proposal as it is. For Nokia’s comment, if your or any other company find some issue in the existing 213, it can be discussed later. For comment from QC/Baicells/ZTE, you continue the same argument from performance perspective. However, more companies believe that 1/2/4/8 repetition factors with intra-slot FH are sufficient. In addition, you do not try to solve issues on CDM capacity with non-repetition UEs and capability aspect. Then can you believe that other companies accept your proposal by just what you mentioned so far?

Proposal 1-5_v0 (No change)

For PUCCH repetition for Msg4 HARQ-ACK, reuse frequency hopping mechanism in R15/16/17.

- i.e., intra-slot frequency hopping is applied when repetition is performed.

      - 2nd round

Discussed via reflector.

After the discussion via reflector

The following agreement was reached. With that, this section was closed.

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, apply frequency hopping mechanism in R15/16/17 defined for PUCCH transmission for Msg4 HARQ-ACK, in every slot.

    - [Email/Low] Remaining FFS

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Regarding configuration of only ‘1’, 3 companies suggest removing the possibility. Considering discussion in FL summary at the last meeting, this direction would be OK.

For UE behavior when repetition factor is not configured via SIB, although 8 companies believe that no repetition is the most reasonable behavior, 2 companies think dynamic indication should be considerable. [4/OPPO]’s comment is that even when repetition factor is not configured, if other parameter related to this repetition feature is configured, the situation can be interpreted like configuration of {1, 2, 4, 8}. Actually, in Msg3 rep in R17, dynamic indication from {1, 2, 4, 8} is applied when PRACH resource for Msg3 rep is configured. Similar mechanism may be considered; thus, FL would like to ask such a mechanism is necessary.

      - 1st round

Proposal 1-6a_v0

For PUCCH repetition for Msg4 HARQ-ACK, candidate values of only one repetition factor configuration via SIB are {2, 4, 8}.

- i.e., configuration of only ‘1’ is not supported.

Proposal 1-6b_v0

When repetition factor for PUCCH repetition for Msg4 HARQ-ACK is not configured via SIB, and if parameter Z related to PUCCH repetition for Msg4 HARQ-ACK is configured via SIB, dynamic indication from {1, 2, 4, 8} is applied.

- FFS: details of parameter Z

Q1: Do you agree Proposal 1-6a_v0? If NO, please share the reason and how the proposal should be updated.

Q2: Do you think Proposal 1-6b_v0-like mechanism is necessary? Please share the reason.

| Company | Q1 (Y/N) | Q2 (Y/N) | Comment |
| --- | --- | --- | --- |
| Sharp | Y |  | We are not sure what the parameter Z is. |
| LG | Yes | No | We prefer to support that if repetition factor for PUCCH repetition for Msg4 HARQ-ACK is not configured via SIB, single PUCCH transmission is used (legacy UE behavior) |
| OPPO | Y | Y | For proposal 1-6b, we note that in R17 Msg3 PUSCH repetition, the Msg3 PUSCH repetition factor is optionally configured. That is, if the UE sends Msg3 repetition request but no repetition factor is configured in SIB, a default set of Msg3 PUSCH repetition factor {1,2,3,4} is applied. In our understanding, the similar case may exist in Msg4 PUCCH repetition and should also be considered. |
| MediaTek | Y | N | On 1-6a, same view as LG On 1-6b, it is not necessary. It is sufficient if gNB only configures one repetition factor, then dynamic indication from {1, 2, 4, 8} cannot be applied. This is semi-static configuration of single repetition factor |
| Ericsson | Y | N | On 1-6b, RAN1 should focus on what we want to be able to signal (in this case, that all repetition factors are supported), and leave to RAN2 to decide how to efficiently signal it in SIB. |
| Qualcomm | N | N | Not sure if an agreement like 1-6a is needed. It’s already agreed that gNB is allowed to configure only 1 value. Whether rep “1” is allowed or not is not important. Proposal 1-6b is not necessary. |
| NEC | Y | N | Similar view as LG, if repetition factor is not configured in SIB then straightforward solution is to assume no repetition. It is not clear what is the advantage we are achieving from Proposal-1-6b. |
| Apple | Yes |  | On 1-6b, this design could be in RAN2. |
| Lenovo | Y | N | Agree with MediaTek for Proposal 1-6b. |
| Nokia, Nokia Shanghai Bell | No | No | For Proposal 1-6a_v0: We should acknowledge that configuring a single repetition value is simply a subset of configuring multiple potential values, so there is no need to have this distinction. When working with the general case of course the case of “single repetition” should also be part of the configuration. After all, the gNB may be in situations where it would like to also be able to indicate to a repetition capable UE that it should not perform repetitions (repetition factor 1). For Proposal 1-6b_v0: We are uncertain what the meaning of “when repetition factor is not configured by SIB”. After all, how would the UE know the configured repetition factors if they are not provided via SIB? |
| DCM | Y | N | 1-6b-like mechanism is unnecessary since there is no benefit. |
| Samsung | Y | [N] | For Q2, we think that it can be discussed later when discussing RRC parameters. |
| Xiaomi | Y | Y | We think 1-6b is necessary, the parameter Z for example could be the RSRP threshold. when threshold is configured while the repetition factor is not configured in the SIB, dynamic indication from {1, 2, [4], [8]} is assumed. |
| Panasonic | Y | N | We think that when repetition factor for PUCCH repetition for Msg4 HARQ-ACK is not configured via SIB, fallback to legacy behavior (i.e. no repetition) is sufficient. |
| Huawei, HiSilicon | N |  | For 1-6a: there are already 2 bits used for the configuration, why repetition 1 is excluded. For 1-6b: we are fine in general with the direction. But it is true that parameter Z is not clear. maybe it can be revised to “if PUCCH repetition for Msg4 HARQ-ACK is configured to be enabled”. |
| Intel | Y | Y |  |
| CMCC | Y | N | Proposal 1-6b_v0 seems unnecessary for the mechanism of PUCCH repetition for Msg4 HARQ-ACK |
| Baicells | Y | [N] | We are not sure what the parameter Z is. |
| ZTE | N | N | For 1-6a, there are complicated views on how network can indicate UE to perform only one repetition. To simplify the design, ‘1’ should be included in the repetition factor indication. For 1-6b, if repetition factor is not indicated in SIB, fallback behavior is preferred, i.e., no repetition is performed. |

FL’s observation

Proposal 1-6a

Support: 15

Not support: 4

More companies support removing ‘1’ for single factor configuration, while still there is concern of the removal. The main argument is that signaling bits exist, and thus the removal does not lead to any benefit.

Proposal 1-6b

Support: [4]

Not support: [13]

Most companies do not support 1-6b-like mechanism, while similar mechanism is available in R17 Msg3 rep and correspondingly it is proposed to follow this direction.

Based on several companies’ comment, FL suggests these RRC signaling-related parts are left to RAN2. RAN1 should not spend time for this kind of minor case. For the 1st online session, the proposal will not be treated since the proposal below is different from what we discussed in the 1st round.

      - 2nd round

Proposal 1-6c_v0

For PUCCH repetition for Msg4 HARQ-ACK, the following RRC signaling-related aspects are up to RAN2 decision.

- Whether the factor can be ‘1’ when only one repetition factor is configured via SIB

- Whether repetition is available when repetition factor is not configured via SIB

Q: Please comment ONLY if you have strong concern.

| Company | Comment |
| --- | --- |
| LG | Agree in principle. But, regarding second bullet, following modification can be acceptable. - Whether repetition is available UE behavior when repetition factor is not configured via SIB |
| OPPO | Support |
| Ericsson | Fine to let RAN2 decide the signaling-related aspects. Regarding first bullet: if configuring only repetition factor ‘1’ in SIB is allowed, the related UE behavior should be discussed in RAN1. Does it mean that the UE should still [request repetition/indicate repetition capability], or does it mean that the feature is disabled? |
| ZTE | OK to leave to RAN2 for details, if the case that network configuring all UEs not to perform repetition can be covered. |
| Apple | Agree that these signaling issues are up to RAN2 decision. |
| QC | Agree with the first bullet. For the second bullet, RAN1 can decide that UE follows legacy behavior. |
| Samsung | For second sub-bullet, we don’t understand the motivation because it should be no repetition as legacy behavior since any RRC value for repetition factor is not configured. Thus, the second sub-bullet should be removed. We think that it seems not necessary to discuss this issue for now because RRC parameter discussion will be happened from next meeting. Thus, we should discuss how design RRC parameters and the corresponding values from RAN1 perspective. In that sense, this proposal doesn’t help the discussion from our understanding. |
| vivo | We support the first bullet. As for 2nd bullet, we think the straightforward way is to support UE to perform legacy UE behavior without PUCCH repetition when repetition factor is not configured via SIB. RAN1 can decide this situation, which is unnecessary to be left to RAN2. |
| Panasonic | Support. For the 2nd bullet, legacy behavior when repetition factor is not configured via SIB is ok to us as suggested by several companies. |
| Nokia, Nokia Shanghai Bell | OK to leave signaling details to RAN2 |
| Baicells | Agree |

FL’s observation

First bullet

Support: 8

Not support: [1]

Second bullet

Support: 8

Not support: 3

For the first bullet, FL suggests taking it as an agreement. To reflect Ericsson’s comment, one additional sub-bullet can be added. For the second bullet, companies’ views are divergent. According to Samsung’s suggestion, FL suggests discussion the second bullet in future meeting.

Proposal 1-6c_v1

For PUCCH repetition for Msg4 HARQ-ACK, the following RRC signaling-related aspects is are up to RAN2 decision.

- Whether the factor can be ‘1’ when only one repetition factor is configured via SIB

  - If ‘1’ is configured, UE behavior is the same as in R17 specification.

- Whether repetition is available when repetition factor is not configured via SIB

      - 3rd round

Discussed via reflector.

    - [Closed/Low] Others

There are other discussion topics in companies’ contributions. They are listed here with FL’s observation for each.

- UE capability signaling details: Single capability (4)

  - [FL]: although it seems that proposals are aligned, capability issue can be discussed in UE feature session in future unless earlier discussion is required.

- DMRS bundling: Support (1), not support (3)

  - [FL]: Essentiality is unclear. FL suggests focusing on essential topic first.

- PUCCH resource candidates: Use only PUCCH with 14 symbol (3), Enhance the number of PUCCH resources in a cell (3)

  - [FL]: A new section for discussion is prepared if more inputs are found.

- Applicability for TN: Support (1)

  - [FL]: Such an aspect should be discussed in UE feature session as in R17.

- PUCCH slot count: Enhance (1), postpone discussion (1)

  - [FL]: Essentiality is unclear, especially FDD is assumed in NTN. FL suggests focusing on essential topic first.

- TX beam for rep: Clarify (1)

  - [FL]: Spec impact is unclear.

- Beam level rep config: Support (1)

  - [FL]: Essentiality is unclear. FL suggests focusing on essential topic first.

      - Comment

This section is prepared for discussion related to issues above or any others. Unless critical issue is found, no further section or no corresponding proposal is prepared at least in this meeting.

| Company | Comment |
| --- | --- |
| LG | PUCCH format 1 with 14 OFDM symbols can be a baseline for Msg.4 HARQ-ACK PUCCH repetition. |
| MediaTek | Capability signalling can only happen after MSG4. Some mechanisms needed to be discussed in RAN1 in NTN Session. |
| Ericsson | On “UE capability signaling details: Single capability”: We think single capability should be agreed since if more than one UE capability is needed, it may have impact on discussion in 4.1.2. |
| Nokia, Nokia Shanghai Bell | Agree with feature lead assessment. |
| Huawei, HiSilicon | Agree with FL’s assessment. |
| Thales | Agree with feature lead assessment |
| Qualcomm | Agree with FL. |
| Lenovo | Agree with FL. |

FL’s observation

For capability part, this topic is UE behavior during initial access and thus discussion in this agenda may be necessary as commented by MTK/Ericsson. Meanwhile, FL’s feeling is that the discussion is not urgent; firstly we can make basic functions, and then subsequent discussion including capability perspective can be prepared. In other words, FL will prepare discussion section for capability in future meeting (probably next meeting).

Views for the remaining seems to be OK. With these, this section is now closed.

  - DMRS bundling for PUSCH taking into account NTN-specifics

For discussion of DMRS bundling, the following RAN4 requirements are considered, as stated/used in contributions.

- Timing error limit (Table 7.1C.2-1 in 38.133)

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

- Phase difference limit (Table 6.4.2.5-1 in 38.101-1)

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

- Frequency error limit (Section 6.4.1 in 38.101-5)

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.] Requirement will be verified for at least two cases of which one has zero Doppler conditions.

    - [Closed/High] Phase rotation due to time drift

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit

Proposal 2-2_v5 (for observation) For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots for carrier bandwidth 5 MHz at the edge of the carrier bandwidth, if TA pre-compensation update, phase pre-compensation, and RX phase post-compensation are not assumed, and if 70.5 (us/s) timing drift rate is assumed. - FFS: whether to assume phase pre-compensation at UE and/or RX phase post-compensation at gNB

At the last meeting, the above observation for timing error limit and frequency error limit was agreed while impact of phase difference limit was not concluded. The main reason is to have time for further checking and to clarify corresponding RAN1 assumption/work. In this section, FL would like to treat this observation discussion and corresponding RAN1 assumption/work together in order to avoid misunderstanding of observation regarding phase rotation due to time drift.

From companies’ inputs, FL observed the following for phase difference limit. Note that LEO 1200 with elevation angle of 30 deg, SCS = 15kHz, and 5 MHz carrier bandwidth are assumed.

- At least 6 companies ([1/HW, HiSi] [2/Nokia, NSB] [4/OPPO] [12/Ericsson] [21/QC] [24/DCM]) shows that the phase difference limit cannot be satisfied e.g., at the edge of the carrier bandwidth.

  - [4/OPPO]’s observation is that if the PRB allocation is not within 6 PRBs from the DC carrier, the requirement is not met over multiple slots. FL believes that this type of observation is better for observation-type agreement compared to Proposal 2-2_v5 above since the observation explains the situation more accurately. It seems that [12/Ericsson]’s illustration intends the same observation.

    - Δφ = 2π (rad) × 70.5 (us/s) × 1 (ms) × 6 (PRB location from DC carrier) × 12 (subcarrier) × 15 (kHz) = 0.478 (rad) = 27 (deg)

- 4 companies ([3/vivo] [7/Baicells] [15/ETRI] [18/MTK]) show a bit different observation, but FL’s understanding of the reason is that they did not consider PRB allocation at the edge of carrier bandwidth as in the following illustration. The main point of phase difference due to timing drift is NOT channel bandwidth BUT distance from the DC carrier.

In summary, FL suggests agreeing observation as proposed by [4/OPPO] above to have the same understanding on this fact with respect to phase difference due to timing drift.

Then, under the observation, there are lots of inputs on what we should assume to avoid this issue. FL found that the inputs can be categorized into the following 2 options.

- Option 1: Pre-compensation is performed at UE side (5)

- Option 2: Post-compensation is performed at gNB side (4)

  - In this option, e.g., [12/Ericsson] commented that the phase difference due to timing drift is linear within the allocated subcarriers and thus CFO compensation can be applied at gNB side.

Besides, there are suggestion of sending LS to RAN4 to ask feasibility of these options and whether to update the RAN4 requirement. FL would like to ask which option should be assumed in RAN1. If both options should be considered, FL thinks that additional capability signaling to differentiate between Option 1 capable UE and Option 1 incapable UE.

In addition, it can be found that 7 companies propose not to perform TA pre-compensation update within each actual TDW if it causes phase discontinuity that may violate phase difference limit. This would be related to Option 1; if Option 1 is considered, time and/or frequency and/or phase pre-compensation to meet the phase difference limit will be performed at UE side. FL suggests considering this aspect together.

With respect to pre-compensation at the beginning of each TWD, 3 companies raise an issue of the initial timing error at the beginning of bundling. FL would like to ask to companies whether/how this issue to be solved.

      - 1st round

Proposal 2-1a_v0

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots for carrier bandwidth 5 MHz, if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed.

Proposal 2-1b_v0

For NTN-specific PUSCH DMRS bundling,

- Either or both of the following options is assumed to satisfy the phase difference limit.

  - Option 1: Pre-compensation of phase rotation due to timing drift is performed at UE side

  - Option 2: Post-compensation of phase rotation due to timing drift is performed at gNB side

- Send an LS to RAN4 to ask feasibility of [Option 1 / Option 2] and to ask whether/how to modify the phase difference limit for NTN.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - Note: If Option 1 is assumed, UE will perform pre-compensation of phase rotation due to timing drift, without causing phase discontinuity that may violate the phase difference limit.

Q1: Do you agree Proposal 2-1a_v0? If NO, please share the reason and how the proposal should be updated.

Q2: Do you agree Proposal 2-1b_v0? If NO, please share the reason and how the proposal should be updated. If YES, which option should be assumed?

| Company | Q1 (Y/N) | Q2 (Y/N) | Option | Comment |
| --- | --- | --- | --- | --- |
| LG | N | Y | Both | First of all, we think Proposal 2-1a should be proposed observation. We have similar view on Proposal 2-1a to the last meeting. Before agreeing with the proposal, the intention of the proposal should be clarified. If the observation just implies that a UE cannot perform DMRS bundling in certain situation, we think the proposal is not necessary since DMRS bundling should be configurable and performed by gNB’s decision. Moreover, if pre/post-compensation is assumed, the proposed observation couldn’t do anything to define DMRS bundling. We think proposal 2-1b need to be prioritized over 2-1a. |
| OPPO | Y | Y | Option 2 | Regarding Option 1, we have a concern that whether the phase rotation on the feeder link can be pre-compensated by the UE. |
| MediaTek | Y | Y | Option 1 | We had same understanding in our TDoc as FL that it is the distance between the DC carrier end the edge of the operating bandwidth that define the phase difference. |
| Ericsson | Y | Y |  | On 2-1b, ask RAN4 for feasibility of both options. For Option 1, pre-compensation of timing should also be considered. |
| Qualcomm | Y | Y |  | Precompensation by UE is always allowed. RAN1 should decide whether to support enhancements for UEs that cannot do pre-compensation during a TDW. |
| Apple | Y | Y | Option 2 | Proposal 2-1a is for observation. |
| Nokia, Nokia Shanghai Bell | No | No |  | Proposal 2-1a_v0 is valid in general. However, we do not see the benefit of capturing the proposed observation. Would the intention be to provide limitations to the gNB scheduling flexibility and/or operation? That is, gNB should not configure PUSCH larger than 6 PRBs and/or PUSCH with x PRBs at the edge of the UL BWP? Proposal 2-1b_v0: Sending LS to RAN4 to check the feasibility of option 1/2 may take some time and at the end, digital phase compensation without breaking phase continuity and power consistency is up to implementation. Further, we would once again like to highlight that post-compensation of (unknown) phase rotation at a very low SINR would not be possible at gNB side. After all, we are introducing the PUSCH DMRS bundling to address the issue of poor SINR conditions to allow the gNB to establish a decent channel estimate. |
| DCM | Y | Y |  | We would like to hear further views of option 1 vs option 2. |
| Samsung | Y | [N] |  | For Q2, we think that all bullets are needed to ask LS to RAN4. Thus, we would like to suggest following change. Actually, we think that further refinement might be necessary. For NTN-specific PUSCH DMRS bundling, - Send an LS to RAN4 to ask feasibility of [Option 1 / Option 2] and to ask whether/how to modify the phase difference limit for NTN. - Either or both of the following options is assumed to satisfy the phase difference limit. - Option 1: Pre-compensation of phase rotation due to timing drift is performed at UE side - Option 2: Post-compensation of phase rotation due to timing drift is performed at gNB side - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. Note: If Option 1 is assumed, UE will perform pre-compensation of phase rotation due to timing drift, without causing phase discontinuity that may violate the phase difference limit. |
| Xiaomi | Y | Y | Option 1 or Option 3: phase rotation compensated by UE and gNB. | We think the phase rotation can be partially compensated by UE and gNB, or totally compensated by UE. The phase rotation is impacted by TA drift which composed of UE-specific TA drift in the access link and common TA drift in the feeder link, and the UE-specific TA drift is unknown to gNB while the common TA drift is known to both gNB and UE. Therefore we support Option 1 and propose to add option 3 as following: Option 3: Pre-compensation of phase rotation due to timing drift in the access link is performed at UE side; Post-compensation of phase rotation due to timing drift in the backhaul link is performed at gNB side. |
| Panasonic | Y | Y | Both | Fine with proposal 2-1a as observation. On proposal 2-1b, both options can be considered. From specification point of view, post-compensation at gNB can be gNB implementation matter and would not impact on the specification. Pre-compensation at UE would depend on the UE capability. What factor is considered in a TDW for UE capability report should be clarified, e.g. UE transmitter impairments, UE autonomous timing update due to satellite movement and phase drift due to satellite movement. |
| Spreadtrum | Y | Y |  | Which one need further discussion. |
| Huawei, HiSilicon |  |  | Option 1 | - We agree in general with the content in Proposal 2-1a_v0. But it is also fair as Nokia requested, the intention of the observation should be firstly clarified. The limitation of gNB scheduling is not preferred. If all companies do not have issues on the issues identified in Proposal 2-1a_v0, maybe we could start from the discussion between option1 and option 2 directly. - Option 1 is preferred considering it can be taken in base band processing to compensate the phase error due to timing drift. If option 1 is adopted, a UE capability can be considered. - Option 2 is not preferred considering gNB does not know the UE’s location in a cell/beam. If option 1 is adopted, there is no need to modify the phase difference limit. Therefore, the LS is not preferred by us. |
| ETRI | Y | Y |  | To FL) Regarding the main point of phase difference, we also had the same understanding as FL. The phrase from our tdoc, “the farthest subcarrier from the center” (taken from the edge of the bandwidth) seems to be same as the phrase from FL, “the distance from the DC carrier”. |
| Thales | Y | Y |  |  |
| Baicells | Yes | Yes | Both | Apply Option1 (Pre-compensation) for service link, and Option2 (Post-compensation) for feeder link. |
| ZTE |  | Y but | Option 1 | For 2-1a, do not see the benefit of agreeing such proposal. For 2-1b, gNB cannot guarantee the phase different limit without knowing how UE performs TA pre-compensation updates. Option 1 should be considered. Moreover, UEs with different implementations may either support option 1 or not. We do not need to assume that option 1 is supported for all UEs. And the 3rd bullet of 2-1b is ambiguous, since how to determine actual TDW is not clear. As how to determine the actual TDW should be studied further. Suggest to add an FFS as following: FFS: how to determine actual TDW. |

FL’s observation

Proposal 2-1a

Support: 13

Not support: 2

Most companies are fine with this observation and the argument of objection is necessity of this proposal. In FL’s understanding, this observation is related to proposal 2-1b. Without 2-1a, why 2-1b is required is unclear. To reflect the concern, this proposal is combined with proposal 2-1b and one note is added.

Proposal 2-1b

Support: 14

- Option 1 only: 3

- Option 2 only: 2

- Both: [5]

Not support: [2]

Most companies are fine with this proposal and further refinement for options seem to be necessary. Based on the comment as UE pre-compensation for feeder link is impossible. Feeder link part is separately mentioned. For options, it seems that asking RAN4 is preferred by more companies.

For comment from Nokia/ZTE, at least one gNB vendor (Ericsson) assumes that Option 2 is possible from gNB perspective; please check it.

For ZTE’s comment, TDW determination is discussed at the next section. No need to include it here.

Proposal 2-1a+2-1b_v1

For NTN-specific PUSCH DMRS bundling,

- (Observation) in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

  - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots for carrier bandwidth 5 MHz, if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed.

  - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier.

- To satisfy the phase different limit,

  - For feeder link, post-compensation of phase rotation due to timing drift is performed at gNB side.

  - For service link, either or both of the following options is assumed to satisfy the phase difference limit. send an LS to RAN4 to ask feasibility of Option 1 / Option 2 and to ask whether/how to modify the phase difference limit for NTN.

    - Option 1: Pre-compensation of phase rotation due to timing drift is performed at UE side

    - Option 2: Pre-compensation of phase rotation due to timing drift is NOT performed at UE side

      - i.e., Post-compensation of phase rotation due to timing drift is performed at gNB side

    - Send an LS to RAN4 to ask feasibility of [Option 1 / Option 2] and to ask whether/how to modify the phase difference limit for NTN.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - Note: If Option 1 is assumed, UE will perform pre-compensation of phase rotation due to timing drift, without causing phase discontinuity that may violate the phase difference limit.

After the 1st GTW session

The following observation/working assumption were reached.

Observation

For NTN-specific PUSCH DMRS bundling,

- In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

  - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed.

  - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier.

Working assumption

For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - FFS: how to determine the actual TDW

- FFS: specification impact

- Send an LS to RAN4

      - 2nd round

For the 2nd round, details of LS shall be discussed. Regarding FFS, the next section will include them; no dedicated question/proposal are not made here.

Title: LS on UE antenna gain for NR NTN coverage enhancement Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG4 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed phase difference limit (Table 6.4.2.5-1 in 38.101-1) for DMRS bundling in NTN case and agreed the following observation and agreement. Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4. 2. Actions To TSG RAN WG4 ACTION: RAN1 respectfully asks RAN4 to take the above RAN1 observation and agreement into account. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN1 Meeting #113 May 22nd-26th, 2023 Incheon TSG-RAN1 Meeting #114 Aug. 21st-25th, 2023 Toulouse

Q: Do you agree the above draft LS? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| LG | Yes in principle | The wording “agreement” need to be changed as “working assumption” RAN1 observation and agreement RAN1 observation and working assumption |
| OPPO | YES |  |
| Ericsson | YES | Agree with LG’s comment. |
| Huawei, HiSilicon | Yes | We think the LS content is good and OK with LG’s comments. No need to add anything else that is not agreed. |
| ZTE | Yes | Agree with LG’s comment. |
| Apple | Yes | Agree with LG’s change. |
| Samsung |  | Why is the title “LS on UE antenna gain for NR NTN coverage enhancement”? It should be changed to something like “PUSCH DMRS bundling for NR NTN coverage enhancement”. Also, we prefer to add other observations for timing error limit/frequency error limit made in last RAN1 meeting as follows. This is because it is to convey the information to RAN4. Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit |
| Spreadtrum | YES | Agree with LG’s comment. |
| vivo | Yes | Agree with LG’s comment. |
| Panasonic | YES | We are ok to add the related observation made in RAN1#112 as Samsung suggested. |
| ETRI | YES | Agree with LG’s comment, and also OK with Samsung suggestion. |
| Nokia, Nokia Shanghai Bell | Yes | OK with proposed LS and agree with LG’s proposed change and also OK with Samsung’s suggestion (even that we may assume that RAN4 people are able to read earlier chairman minutes). |
| Baicells | Yes | Agree with LG and Samsung’s comments. |
| OPPO2 |  | After double-check, we have a question on the ACTION in the LS for clarification. What action does RAN1 expect RAN4 to perform after receiving the LS, check the feasibility of UE pre-compensation, or something else? If just for providing information, could FL clarify why we need the LS to RAN4? |

FL’s observation

Except for FL’s two mistakes, it seems the draft LS is agreeable. Regarding Samsung’s request, FL believes that the observation can additionally be added in this draft LS since this observation is also relative to RAN4 aspect. Several companies commented adding is OK/better, so it is added as below.

To OPPO, in FL’s understanding, several companies believe that feasibility and/or modification of RAN4 requirement should be checked by RAN4, but some companies were raised concern to say such concrete purposes. As a result, David decided that just sending an LS without any implication, and what to be done is completely up to RAN4. Sending LS was already agreed, so we need to do it. That’s all.

Title: LS on PUSCH DMRS bundling for NR NTN coverage enhancement Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG4 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed timing error limit (Table 7.1C.2-1 in 38.133), frequency error limit (Section 6.4.1 in 38.101-5), and phase difference limit (Table 6.4.2.5-1 in 38.101-1) for DMRS bundling in NTN case and agreed the following observations and working assumption. Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4. 2. Actions To TSG RAN WG4 ACTION: RAN1 respectfully asks RAN4 to take the above RAN1 observation and agreement into account. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN1 Meeting #113 May 22nd-26th, 2023 Incheon TSG-RAN1 Meeting #114 Aug. 21st-25th, 2023 Toulouse

Proposal 2-1c

R1-2304093	[Draft] LS on PUSCH DMRS bundling for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

Final LS is endorsed in R1-2304094.

After the 2nd GTW session

The following agreement was reached. With that, this section was closed.

Agreement

Final LS is endorsed in R1-2304094 with the following revision to the action:

ACTION: RAN1 respectfully asks RAN4 to take the above RAN1 observations and agreement working assumption into account.

    - [Closed/High] TDW determination – high level concept

Regarding TDW determination, although we tried to list all possibilities at the last meeting, agreeing the list was failed. FL can find that a lot of companies discuss TDW determination in their contributions, so anyhow discussion would be necessary in this meeting. Given that companies’ inputs can be categorized into some directions, FL suggests discussing high-level concept for TDW determination first. The categories can be made as follows:

- A: Indication from UE to gNB determines actual TDW without corresponding gNB indication/configuration (10)

  - It can be found that multiple companies believe that TA pre-compensation timing should be determined by UE as in Rel-17 NTN, and thus corresponding information is reported from UE to gNB. The reported information determines actual TDW, and it seems that corresponding gNB indication/configuration is not assumed. This UE-centric determination of actual TDW was not defined in Rel-17 DMRS bundling; FL would like to ask this way is feasible.

  - In FL’s understanding, all/most companies supporting this direction assume that nominal TDW is configured by gNB. That is, nominal TDW is not determined by UE side.

- B: Indication from gNB to UE determines actual TDW (11)

  - Companies supporting this direction would intend that gNB configures nominal TDW and also determines actual TDW by some indication to UE. That is, gNB-centric determination of actual TDW. This way is aligned with Rel-17 DMRS bundling mechanism, but Rel-17 NTN assumption of TA pre-compensation in any timing is not maintained.

  - FL’s understanding is that there are two sub-directions. One is just reusing Rel-17 configuration/indication, and the other is newly introducing NTN-specific configuration/indication for determination of nominal/actual TDW. However, discussion for this point is planned as the next step.

Given the current situation of divergent views, FL would like to ask which direction should be considered.

      - 1st round

Proposal 2-2_v0

For NTN-specific PUSCH DMRS bundling, down-select one of the following alternatives.

- Alt 1: UE-centric actual TDW determination

  - gNB configures nominal TDW to UE.

  - UE indicates NTN-specific information to gNB.

  - Actual TDW can be determined based on the UE indication without corresponding gNB configuration/indication.

  - FFS: details

- Alt 2: gNB-centric actual TDW determination

  - gNB configures nominal TDW to UE.

  - Actual TDW is determined based on gNB configuration/indication.

  - FFS: details

  - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN.

Q: Do you agree the above proposal? If YES, which alternative is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Alt | Comment |
| --- | --- | --- | --- |
| LG |  |  | Regarding Alt. 1, actual TDW determination should be based on both UE indication and gNB configuration, unless all event are indicated by UE. Actual TDW can be determined based on the UE indication without in addition to corresponding gNB configuration/indication. We think Alt. 2 should be baseline for where we cannot achieve the consensus on Alt. 1 |
| OPPO | Partially | Alt 2 | Regarding Alt 1, we are unclear what information is reported from the UE to the gNB and how to transmit it. On the one hand, the timing of TA pre-compensation update is left to implementation and is unpredictable in R17 NR NTN, so whether the information on the timing can be reported before TA pre-compensation update is not clear. On the other hand, if the information is reported after TA pre-compensation update, it can not be used for determining actual TDW. Therefore, we prefer Alt 2, where the gNB can provide an appropriate TDW as in segment transmission in R17 IoT NTN, and how to determine actual TDW can be FFS. |
| MediaTek | YES | Alt 2 | We prefer to keep same functionality as in Rel-17, where the actual TDW will be based on UE capability. The TDW size the UE can support while maintaining phase continuity and power consistency should be reported by the UE in capability signalling. |
| Ericsson | YES | Alt 2 | Alt 2 without specification impact (reuse of Rel-17 solution for DMRS bundling) is a feasible solution. An equivalent to Alt 2 was specified for Rel-17 IoT NTN. Alt 1 has larger impact since new signaling is needed. |
| Qualcomm | No | Alt2 | In Alt2, UE reports maximal TDW supported. |
| Apple | Yes | Alt 2 | We prefer that gNB determines/configures the nominal TDW, based on UE reported capability information and time drifting rate of the satellite. The determination of actual TDW follows legacy (Rel-17) design. |
| Lenovo | Yes | Alt2 |  |
| Nokia, Nokia Shanghai Bell | Yes | 2 | Although this is a high level proposal, we find it good if we could settle a direction for the management of the time domain window. For Alt 2 we would assume that enhancements to existing TDW mechanisms are needed for slightly more dynamic control of the TDW from gNB side. |
| DCM | Yes | Alt 2 | Alt 1-like approach is not aligned with normal 3GPP spec. |
| Samsung | [NO] |  | We think that this discussion is put on hold until getting reply LS from RAN4 because it is not clear what exact conditions are for NTN specific DMRS bundling. It is premature to discussion at this moment. Regarding alternatives, it should be under gNB control. |
| Xiaomi | Yes | Alt 2 | Alt 1 may cause large spec. impact. |
| China Telecom | Yes | Alt 2 | This somehow depends on the issues in 4.2.1. But reusing the existing nominal/actual TDW determination mechanism in the current specification is a default option. |
| Panasonic | Yes | Alt 2 | Alt 2 should be baseline. In order to allow gNB to properly determine actual TDW, what factor is considered in a TDW for UE capability report should be clarified by the specification, e.g. UE transmitter impairments, UE autonomous timing update due to satellite movement and phase drift due to satellite movement. |
| Spreadtrum | Yes | Alt 2 | We prefer Alt 2. We think gNB can configure actual TDW, but UE need to report some information, e.g., maximum duration and NTN-specific maximum duration may differ from TN’s. |
| Huawei, HiSilicon | Partially and needs some revisions | Revised Alt.2 | In our understanding, the Alt.1 adjust actual TDW dynamically by UE centric way. However, we should not exclude the solution which enables UE to report information to assist gNB to configure a proper NTDW or determine the actual TDW. Currently, Alt.2 does not include UE assistance of NTDW configuration and it is also not clear that actual TDW can be also based on UE assistance information. Therefore, we prefer to have some revision Alt.2 to allow this. Proposal 2-2_v0 For NTN-specific PUSCH DMRS bundling, down-select one of the following alternatives. - Alt 1: UE-centric actual TDW determination - gNB configures nominal TDW to UE. - UE indicates NTN-specific information to gNB. - Actual TDW can be determined based on the UE indication without corresponding gNB configuration/indication. - FFS: details - Alt 2: gNB-centric actual TDW determination - gNB configures nominal TDW to UE. - Actual TDW is determined based on gNB configuration/indication. - UE assistance information, e.g. the phase pre-compensation capability, antenna switching interval etc., is reported to assist gNB to configure nominal TDW or assist the indication/configuration to determine actual TDW. - FFS: details - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN. |
| Intel | Yes | Alt 2 | Alt 2 is preferred due to ambiguity of actual TDW duration in Alt 1. |
| CMCC | Yes |  | The nominal TDW is configured based on UE capability reporting, the details for actual TDW, such as whether UE assistance information needs to be reported can be further discussed. |
| Thales | Yes | Alt 2 |  |
| Baicells | [No] |  | In our understanding, actual TDWs are determined based on nominal TDW and ‘event’ in spec. Before that, nominal TDW should be determined by gNB and/or UE firstly. |
| ZTE | No | Alt 1 or Alt 2 with clarification | The alt-2 is too ambiguous. The gNB configuration/indication mentioned in second bullet of alt-2 should be clarified. Whether it is the nominal TDW shown in first bullet, or some new configuration? Moreover, we think the key point of actual TDW determination is whether DMRS bundling requirement can be satisfied when pre-compensation is updated. If UE does not have the capability to maintain DMRS bundling across pre-compensation segment, a new event for segment pre-compensation should be introduced for actual TDW determination. If UE have the capability, then new event is not needed. Hence, pure gNB configuration cannot work. UE should report some information, e.g., capability whether UE can maintain phase different limit across pre-compensation segments. So UE report is still needed for alt-2. Therefore, we support Alt 2 if it is clarified that gNB configuration/indication is bullet 2 is the segment length for pre-compensation, and UE will report of capability about cross segment pre-compensation, and new event for segment pre-compensation are introduced. Without above clarification, alt-1 is preferred over alt-2 since alt-2 ignore the impact of TA pre-compensation. |

FL’s observation

Support: [15]

- Alt 1: 1

- Alt 2: [16]

Not support: [4]

Most companies believe that this proposal is fine and Alt 2 should be agreed. FL suggests taking Alt 2 as the first step of TDW determination.

Regarding details of Alt 2, whether additional UE information report is introduced or not is the next discussion. What is clarified in Alt 2 is that even if UE information report is introduced, anyhow gNB configuration/indication determines nominal/actual TDW regardless of what information is reported.

      - 2nd round

Proposal 2-2_v1

For NTN-specific PUSCH DMRS bundling, down-select one of the following alternatives. support Alt 2 for TDW determination.

- Alt 1: UE-centric actual TDW determination

  - gNB configures nominal TDW to UE.

  - UE indicates NTN-specific information to gNB.

  - Actual TDW can be determined based on the UE indication without corresponding gNB configuration/indication.

  - FFS: details

- Alt 2: gNB-centric actual TDW determination

  - gNB configures nominal TDW to UE.

  - Actual TDW is determined based on gNB configuration/indication.

  - FFS: details

    - E.g., configuration of nominal TDW, gNB configuration/indication related to actual TDW determination, information report from UE including UE capability report

  - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| LG | Yes |  |
| OPPO | YES |  |
| Ericsson | YES |  |
| Huawei, HiSilicon | Yes with revision | We suggest the following revisions: - considering FL suggestion in the next section, include the discussion points in section 4.2.3 here; - In Rel-17, nominal TDW can be determined either explicitly by configuration or determined implicitly based on configurations. So revise the first sub-bullet a little bit to align with Rel-17 existing mechanism. - Alt 2: gNB-centric actual TDW determination - gNB configures nominal TDW to UE, or nominal TDW is determined implicitly based on configurations. - Actual TDW is determined based on gNB configuration/indication. - FFS: details - E.g., configuration of nominal TDW, gNB configuration/indication related to actual TDW determination, information report from UE including UE capability report and other information to assist nominal TDW configuration and/or actual TDW determination - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN. |
| ZTE | Yes with revision | Basically fine with the proposal. But since two gNB configurations are mentioned in the proposal, where one is for nominal TDW and the other is for actual TDW determination. What’s the relationship between them are not very clear. We think they can be different and suggest to add a note for clarification: Note: gNB configuration/indication related to actual TDW determination does not refer to the gNB configuration of nominal TDW |
| Apple | Yes | We are fine with Huawei’s modification. Also, the note can be moved under the second bullet of “Actual TDW is determined based on gNB configuration/indication” |
| Samsung |  | We prefer to remove all “e.g. configuration ~” part because all details can be further discussed when consensus is made. For main bullet, it should add “actual” before TDW determination. |
| Spreadtrum | YES |  |
| vivo | Yes | We suggest to remove “E.g., configuration of … ” for progress, and the FFS can be discussed further in another potential proposal. |
| Panasonic | Yes | We are basically fine with the proposal. In order to allow gNB to determine actual TDW (or slots used for joint channel estimation), gNB need to know what is taken into account for the TDW capability report from the UE, especially whether effect of satellite movement (e.g. necessary TA update due to satellite movement, phase rotation due to timing drift) should be taken into account in the TDW capability report. Such details on the TDW capability report also needs to be discussed. - FFS: details - E.g., configuration of nominal TDW, gNB configuration/indication related to actual TDW determination, information report from UE including UE capability report, factors to be taken into account for TDW capability report. |
| ETRI | YES | We are basically fine with the proposal 2-2_v1, and also OK to remove “E.g., configuration of … ” for progress. |
| Nokia, Nokia Shanghai Bell | Yes with comments | More clarity might be needed for the first two bullets of Alt2, but this may probably be resolved at next meeting. We are not OK with Huawei’s proposed modification. The reason being that we need to guarantee that UE and gNB have the same understanding. If we have some implicit association of conditions, it will still require the same understanding at both UE and gNB. Hence we prefer explicit signaling such that this is ensured. |
| Baicells | Yes | Agree with Huawei’s comment. |

FL’s observation

It seems that basically all companies are fine with the direction. Regarding FFS part, it seems that several companies want to have it and other several companies want to remove. From FL perspective, anyhow this is under FFS, so either way would not bring any difference. My suggestion is to keep it for easier understanding for  chair, and discuss it in the next online session.

To HW/Baicells, FL guesses for HW’s intention is that nominal TDW determination may be direct configuration as in R17 or may be implicit determination based on another configuration. To keep it open, FL updated the bullet accordingly. For FFS part, HW’s intention would be some kind of assistance information. FL added simpler text.

To ZTE, they may be related, may not. No need to describe details now since any details are FFS. ‘What’s the relationship between them are not very clear’ is correct and OK. From this reason, FL does not update this proposal for ZTE’s suggestion.

To Panasonic, FL understands Panasonic’s intention, but it seems that the point is already included in the part ‘including UE capability report’. Considering the situation that several companies prefer to remove entire ‘e.g.’ part, FL suggest skipping addition of further details.

To Nokia, probably the update first bullet under Alt 2 is fine, right?

Proposal 2-2_v2

For NTN-specific PUSCH DMRS bundling, down-select one of the following alternatives. support Alt 2 for actual TDW determination.

- Alt 1: UE-centric actual TDW determination

  - gNB configures nominal TDW to UE.

  - UE indicates NTN-specific information to gNB.

  - Actual TDW can be determined based on the UE indication without corresponding gNB configuration/indication.

  - FFS: details

- Alt 2: gNB-centric actual TDW determination

  - Nominal TDW is determined based on gNB configuration configures nominal TDW to UE.

  - Actual TDW is determined based on gNB configuration/indication.

  - FFS: details

    - [E.g., gNB configuration related to of nominal TDW determination, gNB configuration/indication related to actual TDW determination, information report from UE including UE capability report or assistance information]

  - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN.

After the 2nd GTW session

The following agreement was reached. Although we should discuss details of gNB configuration/indication and UE report, rushing the discussion in this meeting would be difficult and not so efficient. We can discuss them in the next meeting based on companies’ further considerations and corresponding inputs in their contributions. With this plan, this section was closed.

    - [Closed/High] TDW determination – UE information report (not determining actual TDW directly)

Regardless of outcome of discussion at the last section, another topic related to TDW determination seems to be UE information report for efficient gNB configuration/indication to determine nominal/actual TDW in NTN scenario.

In R17 Msg3 PUSCH, UE reports only the max bundling size as a UE capability and does expect to be configured with appropriate TDW by gNB. Meanwhile, there are several inputs (4) on support of additional UE information report. The motivation seems to be efficient gNB configuration/indication for nominal/actual TDW. At the same time, 2 companies believe that additional signaling from UE to gNB is unnecessary. FL would like to ask to companies whether another UE information report should be specified or not.

      - 1st round

Q: Do you think another UE information report should be defined for NTN-specific PUSCH DMRS bundling?

Note: this ‘another information’ does not include UE capability of pre-compensation of phase rotation due to timing drift. This point is to be discussed in section 4.2.1.

Note: this ‘another information’ does not determine actual TDW directly. This point is to be discussed in section 4.2.2 (as Alt 1).

| Company | YES/NO | Comment |
| --- | --- | --- |
| LG | YES | Regardless of phase rotation due to timing drift, we think it would be beneficial to indicate several UE capability per NTN platform type. Of course, it can alleviate the issues from phase drift or TDW determination. |
| OPPO | NO | We believe that the gNB can provide appropriate configuration for DMRS bundling. Besides, the information efficiency is time-varying since the satellite is always moving with high speed, resulting the information report mechanism is complicated. |
| MediaTek | NO | We repeat in previous section The TDW size the UE can support while maintaining phase continuity and power consistency should be reported by the UE in capability signalling as in Rel-17. This is sufficient information. |
| Ericsson | NO | Agree with OPPO and MediaTek. Unclear what other information is needed. |
| Qualcomm | Yes | Maximal TDW size needs to be reported. |
| Apple | No | Besides the UE capability reporting (e.g., max. TDW size) as in Section 4.2.1, we do not see other information to be reported from UE to gNB. |
| Lenovo | No | Agree with others, not clear what other information may be required. |
| Nokia, Nokia Shanghai Bell | No | The intention of this discussion is not clear. It is not clear to us what information is carried by the “another UE information report”, given the explanation of the two notes. |
| DCM | Yes | Max TDW size is dependent on multiple factors, e.g., satellite ephemeris, satellite elevation angle, etc. Additional information report from UE to gNB is beneficial to configure TDW size. |
| Samsung |  | We think that this can be discussed in UE feature session if possible. |
| Xiaomi | No |  |
| Panasonic |  | The necessity of another information would depend on Proposal 2-1b (UE pre-compensation/gNB post-compensation). If UE pre-compensation/gNB-compensation is feasible and assumed, another information would not be needed. But, it not, another information to estimate feasible TDW size may be useful. |
| Spreadtrum | Yes | NTN based maximum duration which may be different in different beams need to be reported. |
| Huawei, HiSilicon | Yes | We agree with DCM and Qualcomm, and actually the actual TDW or normal TDW should consider the following aspects: - The UE capability regarding the phase pre-compensation due to time drift. - Other aspects, e.g. UE preferred antenna switching interval for better performance (UE knows the antenna numbers equipped on UE), satellite elevation angle etc. The above aspects to impact the TDW size are clearly different capability from that is considered in Rel-17 TN. Therefore, separate UE reporting should be considered. |
| CMCC | Yes | We think that UE self-estimated TA to pre-compensate for the service link delay, which is calculated based on the UE position and the serving satellite ephemeris, may be transparent for gNB, assistance information may be needed for actual TDW determination. |
| Thales | No | Agree with OPPO and MediaTek |
| Baicells | No | The intention of “another UE information report” is not clear if this another information does not include UE capability of pre-compensation of phase rotation due to timing drift. We are open for further discussion. |

FL’s observation

YES: 6

NO: 9

Companies’ views seem to be divergent; no clear majority. In addition, detailed discussion would be involved with proposal 2-1a+2-1b or 2-2 as commented/implied by several companies. This aspect can be included in FFS of proposal 2-2, and thus this section is not pursed in this meeting.

    - [Closed/Low] Others

There are other discussion topics in companies’ contributions. They are listed here with FL’s observation for each.

- Msg3 PUSCH rep for CFRA is not supported in the current spec and is necessary in NTN. This information should be shared to RAN2 (1)

  - [FL]: Considering situation at the last meeting, LS from RAN2 would be necessary to trigger discussion at RAN1.

      - Comment

This section is prepared for discussion related to issues above or any others. Unless critical issue is found, no further section or no corresponding proposal is prepared at least in this meeting.

| Company | Comment |
| --- | --- |
| Ericsson | RAN1 could discuss the issue without an LS from RAN2. |
| Nokia, Nokia Shanghai Bell | Agree with feature lead assessment. |

FL’s observation

FL feels that this section is not meaningful for most companies. This section is now closed.

- Contribution summary

  - PUCCH enhancements for Msg4 HARQ-ACK

- High-level concept

  - Confirm WA

    - YES: [18/MTK]

- Information report from UE

  - Confirm WA

    - YES: [24/DCM] [19/Apple] [16/CMCC (?)] [12/Ericsson] [11/xiaomi]

    - NO/Update: [25/LGE] [23/Sharp] [17/ZTE] [5/Spreadtrum] [3/vivo] [2/Nokia, NSB]

      - [25/LGE] Proposal 12. Do not support the case when the RSRP threshold is not configured for PUCCH repetition for Msg4 HARQ-ACK.

      - [23/Sharp] Observation 6: Regarding the RSRP threshold,

        - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg1, the RACH resources partitioning based on the RSRP threshold has benefit for reducing the reservation of RACH resources for the Msg4 HARQ-ACK repetition.

        - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg3, the separation using RSRP threshold has no benefit.

      - [23/Sharp] Proposal 2: It should be discussed whether either Msg1 or Msg3 is used for the Msg4 HARQ-ACK repetition request, before discussing whether confirm the working assumption related to RSRP threshold configuration made in RAN1#112.

      - [17/ZTE]

        - Firstly, it should be noted that RSRP variation within an NTN cell is small, which is different from TN.

        - Secondly, the RSRP threshold configured by network is for DL RSRP measurement, while the PUCCH repetition is a UL transmission, which are mismatched

        - Thirdly, it has already been agreed that PUCCH repetition will be performed only when network broadcast the repetition factors in SIB. That is, there is already a mechanism to control whether to perform repetition.

      - [5/Spreadtrum] the RSRP threshold should always be configured by gNB and smaller than X.

      - [3/vivo] Proposal 5: Updates are needed before confirming the working assumption on RSRP threshold for repetition request or capability report of PUCCH repetition for Msg4 HARQ-ACK.

      - [2/Nokia, NSB] definition of such a threshold is not necessary for the targeted feature, as before the Msg4 HARQ-ACK the gNB has already received two UL transmissions from the UE (i.e. PRACH and Msg3), from which it can already derive an accurate estimate of the UE UL conditions and assign a proper repetition factor for the subsequent Msg 4 HARQ-ACK PUCCH transmission

    - Comment

      - [12/Ericsson] How to instruct the UE to always indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) is an RRC signaling decision that should be decided by RAN2.

        - The two options how to instruct the UE to indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) are equivalent from a functional point of view. Not configuring the RSRP threshold saves bits in SIB.

  - RSRP threshold for request

    - Alt A

      - YES: [25/LGE] [18/MTK] [6/CATT]

        - [25/LGE] We think that it is inefficient to provide multiple RSRP thresholds with similar functions to determine whether to perform repeated transmissions of UL channels included in RACH procedure in terms of signaling overhead at gNB side

        - [18/MTK] It is reasonable assumption that the path loss experienced by UE for MSG3 transmission is not expected to change significantly with MSG4 HARQ ACK transmission.

        - [6/CATT] Firstly, we do not believe that the channel condition will change significantly during access. Secondly, the RSRP threshold is used to determine whether the UE is sending repetition request or capability report. Even in some cases, UE requiring Msg3 repetition does not necessarily needs Msg4 PUCCH repetition.

      - NO

    - Alt B

      - YES: [24/DCM] [22/Panasonic] [21/QC] [19/Apple] [16/CMCC] [15/ETRI (?)] [14/Samsung] [13/China Telecom] [12/Ericsson] [11/xiaomi] [9/Intel] [4/OPPO] [3/vivo] [1/HW, HiSi]

        - [22/Panasonic] UE requiring Msg3 PUSCH repetition does not necessarily require Msg4 PUCCH repetition according to past evaluations.

        - [21/QC] Since the exact SNR level below which  Msg4 HARQ-ACK repetition is required differs from that for  Msg3 repetition, network should be allowed the flexibility to configure the RSRP threshold for Msg4 HARQ-ACK repetition independent of that for Msg3 repetition.

        - [16/CMCC] Considering that the payload of Msg4 HARQ-ACK is usually smaller than that of Msg3, and the coverage performance is different between PUCCH Msg4 HARQ-ACK and Msg3

        - [14/Samsung] Alt. B can provide Alt. A by gNB implementation.

        - [13/China Telecom] the coverage performance of Msg3 and Msg4 HARQ-ACK is different, i.e., the link budget of Msg4 HARQ-ACK is better than Msg3

        - [12/Ericsson] The SNR limit for when repetitions are needed is different for Msg4 HARQ-ACK PUCCH and Msg3 PUSCH. Rel-17 Msg3 PUSCH repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are optional features both for the network and the UE. Support for one of the features should not require support of the other.

        - [9/Intel] due to different SNR requirements for PUSCH and PUCCH reception at the gNB

        - [4/OPPO] It can be observed that the performance gap of Msg3 PUSCH is larger than that of Msg4 PUCCH

        - [3/vivo] Considering different UE features in different releases should be supported independently, the RSRP threshold should be independent from the RSRP threshold of Msg3 repetition

        - [1/HW, HiSi] PUCCH of Msg4 HARQ-ACK can have better link budget than Msg3

      - NO

      - Others

        - [24/DCM] [16/CMCC] [14/Samsung] [4/OPPO] [3/vivo] [1/HW, HiSi] The value range is defined by RSRP-range, i.e., X = 127

        - [19/Apple] RSRP threshold for Msg4 PUCCH repetition is rsrp-ThresholdMsg3-r17 minus an offset

        - [15/ETRI] By adding or subtracting some amount of constant to rsrp-ThresholdMsg3-r17, UE can interpret this calculation result as a threshold for PUCCH repetition for Msg 4 HARQ-ACK

    - Others

      - [22/Panasonic] Proposal 6: The same RSRP threshold for PRACH repetition and Msg4 PUCCH repetition should be considered.

      - [19/Apple] Considering both Msg3 PUSCH repetition and Msg4 PUCCH repetition are optional UE features, all the 4 cases are possible. Hence, the joint indication of Msg3 PUSCH repetition and Msg4 PUCCH repetition is unnecessary.

  - Others

    - [19/Apple] UE’s indication of required Msg4 PUCCH repetition factor is not supported.

    - [16/CMCC] No further PRACH resource partitioning is needed for indication of repetition request or capability report

      - The gNB is also aware of the UL channel condition more clearly since paired spectrum is applied in the NTN system. Besides, UE indicating repetition factor may also needs more signalling to report different number of repetitions, the increased payload should also be avoided considering the NTN UE is more likely suffered from a bad channel condition.

    - [8/NEC]

      - Proposal 1: Support only repetition request signalling for Msg4 HARQ-ACK.

      - Proposal 2: A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

    - [7/Baicells] Observation 1 : UE can only indicate whether or not the measured RSRP is lower than the RSRP threshold. UE indication of repetition factor is not feasible. Proposal 6 : UE indication of repetition factor is not assumed.

    - [6/CATT] The repetition factor for MSG4 HARQ-ACK should be determined by gNB.

    - [1/HW, HiSi] Proposal 3: Do not support repetition factor request in option A.

- Signaling for repetition request or capability report

  - Option A

    - YES: [25/LGE] [22/Panasonic] [20/Lenovo] [19/Apple] [16/CMCC] [15/ETRI] [14/Samsung] [6/CATT] [4/OPPO] [3/vivo] [1/HW, HiSi]

      - [25/LGE] RACH partitioning is a technique in which a gNB informs which RACH resource (e.g., preamble indexes) a UE should select when the UE has a specific feature or when the UE tries to request the feature

      - [25/LGE]

        - Option 1: Introduce a new parameter (e.g., pucch-Repetitions-r18) for the PUCCH repetition for Msg4 HARQ-ACK feature into the SIB (e.g., in FeatureCombination-r17).

        - Option 2: “msg3-Repetitions-r17” in FeatureCombination-r17 is used as a PUCCH repetition for Msg4 HARQ-ACK feature in addition to the existing Msg3 PUSCH repetition feature.

      - [22/Panasonic] The restriction of combinations among PRACH repetition, Msg3 PUSCH repetition and Msg4 PUCCH repetition should be applied in order to avoid too many PRACH resource partitioning. our understanding is when Msg.4 PUCCH repetition is required, Msg.3 PUSCH repetition would be also required

      - [19/Apple] In Rel-17 NR coverage enhancement, to request Msg3 PUSCH repetition, a UE uses separate PRACH preamble in a shared RACH occasion configured by the same PRACH configuration index with legacy UEs.

      - [16/CMCC] the mechanism of Rel-17 Msg3 repetition can be reused for PUCCH repetition for Msg4 HARQ-ACK. if using exactly the same PRACH preamble or if UE reports repetition capability based on the similar RSRP configuration, gNB can not figure out whether Msg3 repetition or PUCCH repetition for Msg4 HARQ-ACK, or both are required by UEs.

      - [15/ETRI] existing mechanism for Msg3 repetition can be reused

      - [15/ETRI] Proposal 2: In case of indication repetition request or capability report using PRACH preamble and/or occasion, a method for minimizing collision probability for more detailed fragmentation should be discussed.

      - [14/Samsung] the partitioning issue can be avoided by gNB implementation such as joint configuration (RACH resources associated with multiple features) and/or making small spot beam to minimize SNR variation.

      - [6/CATT] Resources combination of preamble resources can be used to resolve the preamble resources are further divided.

      - [4/OPPO] Observation 4: Associating a PRACH resource with the request of Msg3 PUSCH repetition and Msg4 PUCCH repetition simultaneously can mitigate PRACH resource partitioning issue.

      - [3/vivo] some rules can be introduced to alleviate this excessive partitioning. For example, a UE requesting msg3 repetition could also be considered as a UE requesting PUCCH repetition for Msg4 HARQ-ACK.

      - [1/HW, HiSi] it is quite straight forward to reuse the same mechanism. the added feature does increase the possibility of further segmentation but not necessarily contribute to Msg1 collisions very much

    - NO: [24/DCM] [23/Sharp] [18/MTK] [17/ZTE] [13/China Telecom] [12/Ericsson] [11/xiaomi] [9/Intel] [8/NEC] [5/Spreadtrum (?)] [2/Nokia, NSB]

      - [24/DCM] Option A leads to further fragmentation of PRACH resources or inefficient resource usage. Information transmission via PRACH should be used ONLY when the other mechanism is unavailable

      - [23/Sharp] Observation 1: If the RACH resources for Msg3 repetition request via Msg1 is used to also indicate Msg4 HARQ-ACK repetition request, the network need to differentiate between Rel-17 Msg3 repetition UE and Rel-18 UE that requests both Msg3 and Msg4 HARQ-ACK repetitions. For this differentiation, RACH resource segmentation for Rel-18 UE would be needed anyway.

      - [18/MTK] Further increasing RACH resource overhead for MSG4 HARQ repetition cannot be justified when there are other options without any impact on RACH resource efficiency and flexibility for configuration.

      - [17/ZTE] considering large preamble fragmentation already exists for the support of other features (RedCap,slicing,SDT,etc.) and the large cell sizes in NTN, furthermore, PRACH repetition which would be introduced in R18 coverage enhancement WI will consume more resources. Indication of PUCCH repetition capability via PRACH preamble/RO will result in further segmentation of the PRACH preambles which would lead to increased collision probability.

      - [13/China Telecom] There are already many features for PRACH resource partitioning. If PRACH resource partitioning is used for Msg4 HARQ-ACK repetition request, severe problems can be caused for PRACH resource usage, e.g., PRACH capacity, PRACH resource collision

      - [12/Ericsson] Excessive partitioning of the PRACH resources may cause capacity problems for PRACH and increases complexity. Knowledge of the UE’s need for Msg4 HARQ-ACK PUCCH repetition is not needed in the network until after Msg3 is received.

      - [11/xiaomi] Option A cause further PRACH resource fragmentation. It may not be possible to indicate all the repetition factors considering the limited PRACH resource.

      - [9/Intel] gNB should allocate additional PRACH resources either with separate PRACH preambles in case of shared ROs or separate ROs, which may not be desirable in term of spectrum efficiency.

      - [8/NEC] Indication of PUCCH repetition capability via PRACH will result in segmentation of the PRACH preambles which would lead to increased collision probability. Since large preamble fragmentation already exists for the support of other features and the large cell sizes in NTN, PUCCH repetition capability reporting via Msg1 is not preferred.

      - [5/Spreadtrum] Proposal 2: For PRACH (Preamble and/or occasion) based signaling for request or capability report, it is necessary for PRACH resource partitioning for PUCCH and how to avoid further fragmentation of PRACH resources needs to be considered.

      - [2/Nokia, NSB] Observation 14: Indication of PUCCH repetition capability via Msg1 generates segmentation of the configured preambles, leading to increased collision probability.

  - Option B

    - YES: [24/DCM] [23/Sharp] [21/QC] [20/Lenovo] [18/MTK] [17/ZTE] [16/CMCC] [13/China Telecom] [12/Ericsson] [11/xiaomi] [9/Intel] [8/NEC (?)] [5/Spreadtrum (?)] [3/vivo]

      - [23/Sharp] Observation 2: In case of the transmission of the Msg4 HARQ-ACK repetition request or capability report via the Msg3, the repetition request or capability report from Rel-18 UE does not have any impact on the legacy UEs.

    - NO: [19/Apple] [14/Samsung] [6/CATT] [4/OPPO] [2/Nokia, NSB]

      - [19/Apple] there are only 7 reserved LCID codepoints available. If LCID is to be used for PUCCH repetition request or capability report, then at least 4 codepoints will be used to additionally distinguish 2 CCCH sizes (48 bits or 64 bits) and RedCap/non-RedCap cases.

      - [14/Samsung] Option B and C require further gNB blind detections. Option B and C require larger specification impact(s) (possibly required for other working groups)

      - [6/CATT] Additional signaling added to Msg3 used for MSG4 HARQ-ACK repetition request or capability report will causes significant impact to existing specification.

      - [4/OPPO] Proposal 5: For Option B and C, RAN1 discusses whether/how to solve the ambiguity issue when UEs with and without Msg4 PUCCH repetition capability send Msg3 PUSCH simultaneously.

        - For example, UE1 incapable of Msg4 PUCCH repetition and UE2 capable of Msg4 PUCCH repetition select the same PRACH resource to initiate a random access procedure, then the same TC-RNTI is applied and the same time-frequency resource is used for Msg3 and Msg4 transmission. In this case, when the Msg3 PUSCH of UE1 is successfully received by the gNB, UE2 cannot realize it and will assume its Msg3 PUSCH was received. Then, if the repetition factor is indicated via the existing field in DCI, UE2 may mistakenly perform Msg4 PUCCH repetition, resulting in continuous interference.

      - [2/Nokia, NSB] This approach however is subject to the problems and limitation already highlighted in Section 2.2.1, for which in case of a collision with a UE without PUCCH repetition capability, there would be ambiguity in the UE behavior for subsequent Msg4 reception and decoding.

    - Details

      - LCID value: [24/DCM] [23/Sharp] [5/Spreadtrum]

      - R bit in the MAC subheader: [12/Ericsson]

        - [12/Ericsson] If LCID codepoints are used to request Msg4 HARQ-ACK PUCCH repetitions, four of the seven reserved LCID codepoints would be needed to also support simultaneous indication of CCCH for a RedCap UE and CCCH size of 48 and 64 bits.

      - LS to RAN2: [24/DCM] [23/Sharp] [22/Panasonic] [21/QC] [16/CMCC] [15/ETRI] [3/vivo] [1/HW, HiSi]

  - Option C

    - YES [21/QC] [8/NEC (?)] [2/Nokia, NSB]

    - NO: [24/DCM] [22/Panasonic] [19/Apple] [18/MTK] [16/CMCC] [14/Samsung] [12/Ericsson] [11/xiaomi] [9/Intel] [4/OPPO] [3/vivo] [1/HW, HiSi]

      - [22/Panasonic] DMRS and scrambling of CRC could be considered. Both methods would impose undesired complexity increase at gNB receiver

      - [19/Apple] This may include either a dedicated scrambling to Msg3 PUSCH or a dedicated DMRS port for Msg3 PUSCH. Either of these approaches increases network detection complexity and has significant specification impact.

      - [16/CMCC] such as the dedicated scrambling to Msg3 PUSCH or using a dedicated DMRS ports of Msg3 PUSCH, which will introduce network implementation complexity and significant standard effort.

      - [14/Samsung] Option B and C require further gNB blind detections. Option B and C require larger specification impact(s) (possibly required for other working groups)

      - [12/Ericsson] the proposed solutions require blind detection, which has negative impact on gNB complexity.

      - [11/xiaomi] Option C may cause negative impacts on channel estimation at gNB

      - [9/Intel] gNB needs to perform blind decoding of Msg3 PUSCH according to used DMRS port or other physical layer parameters (e.g., scrambling), which would increase implementation complexity at the receiver.

      - [4/OPPO] Proposal 5: For Option B and C, RAN1 discusses whether/how to solve the ambiguity issue when UEs with and without Msg4 PUCCH repetition capability send Msg3 PUSCH simultaneously.

      - [3/vivo] using DMRS port to carry the information may require Msg 3 blind detection and cause significant increasing on NW complexity

      - [1/HW, HiSi] it introduces unnecessary gNB blind detection of PUCCH.

    - Details

      - [21/QC] a UE can use DMRS port 1 instead of 0 for Msg3

  - Others

    - [18/MTK] Proposal 3: UE reports of a capability for MSG4 HARQ Ack repetitions is based on a RSRP threshold.

- Dynamic indication

  - Alt 1 [20/Lenovo] [14/Samsung] [8/NEC] [7/Baicells]

    - Alt 1-1

      - YES: [25/LGE] [24/DCM] [22/Panasonic] [21/QC] [19/Apple] [17/ZTE] [16/CMCC] [15/ETRI] [13/China Telecom] [12/Ericsson] [11/xiaomi] [4/OPPO] [3/vivo] [1/HW, HiSi]

        - [12/Ericsson] it has the advantage that the repetition factor can be chosen based on measurements on Msg3 PUSCH and selected as closely as possible before the Msg4 HARQ-ACK is to be transmitted by the UE.

      - NO: [6/CATT] [5/Spreadtrum] [2/Nokia, NSB]

        - When field in DCI is used for dynamic indication of repetition factor for Msg4 HARQ-ACK，existing fields are difficult to reuse, and adding additional field affects the DCI design which will involve a massive revision of the existing spec.

        - [5/Spreadtrum] it would influence the original indication of that field to some extent and reduce the scheduling flexibility

        - [2/Nokia, NSB] Observation 10: Repurposing of DCI fields creates ambiguity in the case of collision between UEs with and without capabilities of PUCCH repetitions.

      - Alt 1-1a: [25/LGE] [24/DCM] [19/Apple] [16/CMCC] [15/ETRI] [13/China Telecom] [4/OPPO] [3/vivo] [1/HW, HiSi]

        - [23/Sharp] For Msg4 HARQ-ACK repetition, unlike Msg3 repetition, the channel to which the MCS field value applies and the channel that the repetition is needed are different

        - [22/Panasonic] higher MCS may be used (e.g. with high satellite power)

        - [16/CMCC] To avoid significant payload increasing and DCI format spec impact, similar design spirit could be reused

        - [12/Ericsson] Limiting to PDSCH MCS selection

        - [9/Intel] the MCS field is not related to PUCCH and determines the MCS for PDSCH transmission, full range of MCS may be needed due to different DL coverage level comparing to UL coverage

        - [1/HW, HiSi] the modulation order and the coding rate for transmission of Msg4 in NTN does not need to be large, due to the relatively low link budget

      - Alt 1-1b: [22/Panasonic] [13/China Telecom] [12/Ericsson] [4/OPPO]

        - [23/Sharp] serious restriction of flexibility of resource assignment for network

        - [22/Panasonic] considering that all slots are available for PUCCH transmission in FDD which is assumed in NTN and PDSCH-to-HARQ timing indicator gives additional flexibility, restriction of PUCCH resource indication would not be crucial

        - [12/Ericsson] if the scope is repetition for any PUCCH when dedicated PUCCH is not configured.

        - [3/vivo] less flexibility and more specification impact

        - [1/HW, HiSi] PUCCH frequency resource scheduling flexibility

      - Alt 1-1c [24/DCM] [15/ETRI] [4/OPPO]

        - [12/Ericsson] Reducing number of HARQ processes may cause stalling if the RTT is large.

        - [3/vivo] HARQ process number is determined by higher layer and the relevant field would be on duty, which should not be changed

        - [1/HW, HiSi] Apart from the backward compatibility issue for the Rel-17 NR NTN UE, such choice impacts the flexibility for HPN configuration

      - Alt 1-1d: [25/LGE] [24/DCM] [23/Sharp] [21/QC] [18/MTK] [15/ETRI] [12/Ericsson] [9/Intel] [4/OPPO] [3/vivo]

        - [23/Sharp] DAI field is not reserved for DCI1_0 that is not scrambled by TC-RNTI. RAN agreed to specify only PUCCH enhancement for Msg4 HARQ-ACK and excluded to specify the other PUCCH transmission.

        - [22/Panasonic] if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, use of this field would not be preferable.

        - [12/Ericsson] if the scope is only repetition for Msg4 HARQ-ACK PUCCH

        - [1/HW, HiSi] these two bits are preferred to be kept for future purpose if MCS field can already support the indication of PUCCH repetition number

      - Alt 1-1e: [22/Panasonic] [15/ETRI]

        - [23/Sharp] serious restriction of flexibility of resource assignment for network

        - [22/Panasonic] considering that all slots are available for PUCCH transmission in FDD which is assumed in NTN and PDSCH-to-HARQ timing indicator gives additional flexibility, restriction of PUCCH resource indication would not be crucial

        - [22/Panasonic] if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, this field can not be used for the indication of the repetition factor.

        - [12/Ericsson] Constrains UL scheduling

        - [1/HW, HiSi] scheduling flexibility would be affected

      - Details

        - [21/QC] Proposal 4: The same repetition factor of the PUCCH for Msg4 HARQ-ACK applies to other PUCCH without additional signaling when dedicated PUCCH resource configuration is not provided.

    - Alt 1-2

      - YES

      - NO: [22/Panasonic] [15/ETRI] [13/China Telecom] [5/Spreadtrum] [2/Nokia, NSB] [1/HW, HiSi]

        - [22/Panasonic] If the DCI size is changed, UE capable of and/or requesting the repetition needs additional blind decoding (i.e. normal DCI size and increased DCI size), which is not preferable.

        - [13/China Telecom] The increase of the length of DCI should be avoided. CRC scrambling of DCI scheduling the Msg4 PDSCH will increase CRC missing detection probability and complexity.

        - [2/Nokia, NSB] Observation 11: Introduction of a new field in the DCI scheduling the Msg4 generates fallback DCI size misalignments in the case the PUCCH repetition feature is not configured by the gNB.

        - [1/HW, HiSi] it is not preferred to change the size of the DCI format 1_0

  - Alt 2

    - YES [16/CMCC] [5/Spreadtrum] [1/HW, HiSi]

      - [5/Spreadtrum] it is reasonable assumption that the path loss experienced by UE for MSG3 transmission would not change significantly with MSG4 HARQ ACK transmission.

    - NO [24/DCM] [23/Sharp] [22/Panasonic] [20/Lenovo] [18/MTK] [15/ETRI (?)] [14/Samsung] [13/China Telecom] [12/Ericsson] [8/NEC] [2/Nokia, NSB]

      - [24/DCM] these alternatives do not allow for gNB implementation to decide repetition factor based on Msg3 RX

      - [23/Sharp] the UE should indicate the repetition request via Msg3

      - [22/Panasonic] A drawback of Alt 2 and Alt 4 is that gNB can not use the reception results of Msg3 PUSCH (e.g. signal strength and/or TA report in Msg3 PUSCH if included).

      - [18/MTK] un-necessary complicate the DCI design

      - [17/ZTE] dynamic scheduling is not only inflexible but also complex

      - [16/CMCC] considering UL coverage limitation happens in both Msg3 PUSCH and Msg4 HARQ-ACK

      - [15/ETRI] there is a possibility that the required number of repeated transmissions for PUCCH for Msg4 HARQ-ACK may be incorrect when the number of repeated transmissions is indicated in Msg3 based on the signal measured by preamble only.

      - [14/Samsung] it cannot be applicable for UEs only supporting Msg 4 HARQ-ACK repetitions. Therefore, other complementary solution(s) are still needed.

      - [13/China Telecom] As the coverage performance of Msg3 and Msg4 HARQ-ACK is different, it’s not appropriate to indicate PUCCH repetition factor for Msg4 HARQ-ACK and number of Msg3 repetition jointly

      - [12/Ericsson] the repetition factor cannot be chosen based on measurements on Msg3 PUSCH, and the independent features Msg3 repetition and Msg4 HARQ-ACK PUCCH repetition are coupled.

      - [8/NEC] Since, repetition request/capability is proposed to be indicated using Msg3, the gNB only has the option to provide the indication within DCI for Mdg4 PDSCH

      - [2/Nokia, NSB] Observation 12: Alt 2 does not allow a gNB to optimize the number of repetitions for the UE dominating in Msg3 (in case of PRACH collision), defeating the purpose of a dynamic indication of the PUCCH repetition factor.

    - Others

      - [23/Sharp] Proposal 5: Regarding the agreement of RAN1#112 meeting, the following correction should be needed:

        - Alt 2: Field in DCI RAR UL grant scheduling Msg3 PUSCH

  - Alt 3

    - YES: [2/Nokia, NSB]

    - NO [24/DCM] [23/Sharp] [22/Panasonic] [20/Lenovo] [18/MTK] [17/ZTE] [14/Samsung] [12/Ericsson] [6/CATT] [1/HW, HiSi]

      - [23/Sharp] [22/Panasonic] [20/Lenovo] [18/MTK] [17/ZTE] [14/Samsung] there is a complexity issue

      - [14/Samsung] it is likely to increase false alarm probability compared to legacy behavior since it is similar to introduce new RNTI for special purpose (i.e., indicating repetition number using CRC bits).

      - [12/Ericsson] Indicating the repetition factor for PUCCH using CRC scrambling of DCI scheduling Msg4 (i.e., Alt 3 in the agreement from RAN1#112) has the disadvantage that it reduces the error correcting capability of the CRC and increases receiver complexity.

      - [6/CATT] high impact on complexity.

      - [1/HW, HiSi] usage of some bits of CRC shall impact the physical layer channel performance, e.g. the FAR of the DCI detection.

  - Alt 4

    - YES: [25/LGE] [20/Lenovo] [7/Baicells] [6/CATT] [5/Spreadtrum] [19/Apple]

      - [7/Baicells] if a fixed relationship between the PUCCH repetition factor and Msg3 repetition factor can be agreed

      - [6/CATT] the channel quality will not change dramatically during random access.

      - [5/Spreadtrum] it is reasonable assumption that the path loss experienced by UE for MSG3 transmission would not change significantly with MSG4 HARQ ACK transmission.

    - NO [24/DCM] [23/Sharp] [22/Panasonic] [17/ZTE] [14/Samsung] [8/NEC] [2/Nokia, NSB] [1/HW, HiSi]

      - [24/DCM] these alternatives do not allow for gNB implementation to decide repetition factor based on Msg3 RX

      - [23/Sharp] the UE should indicate the repetition request via Msg3

      - [22/Panasonic] A drawback of Alt 2 and Alt 4 is that gNB can not use the reception results of Msg3 PUSCH (e.g. signal strength and/or TA report in Msg3 PUSCH if included).

      - [17/ZTE] there may be situations where only PUSCH repetition is required without PUCCH repetition. In this case, it may cause unnecessary resource waste of PUCCH and access delay, and using other signaling, such as high-layer signaling, to define implicit mapping relationships can make dynamic scheduling inflexible

      - [14/Samsung] it cannot be applicable for UEs only supporting Msg 4 HARQ-ACK repetitions. Therefore, other complementary solution(s) are still needed.

      - [8/NEC] Since, repetition request/capability is proposed to be indicated using Msg3, the gNB only has the option to provide the indication within DCI for Mdg4 PDSCH

      - [2/Nokia, NSB] Observation 13: Alt 4 imposes a constraint to UE implementations and restricts the flexibility for the dynamic indication of the PUCCH repetition factor.

      - [1/HW, HiSi] the mapping relationship of repetition factors between two channels are fixed

- When no factor is configured

  - No repetition: [25/LGE] [24/DCM] [22/Panasonic] [18/MTK] [9/Intel] [8/NEC] [7/Baicells] [4/OPPO]

    - [25/LGE] if additional signaling for PUCCH repetition is not provided via SIB, transmitting a single PUCCH (i.e., legacy UE behavior) is the simplest and the most natural UE behavior

    - [9/Intel] PUCCH HARQ-ACK repetition in response to Msg4 PDSCH feature can be enabled or disabled by the gNB in a simpler way.

    - [4/OPPO] Proposal 4: When the RSRP threshold and repetition factor are not configured in SIB, the legacy UE behaviour is applied.

  - Dynamic indication: [11/xiaomi] [4/OPPO]

    - [4/OPPO] Proposal 3: For Msg4 PUCCH repetition, the case that the RSRP threshold is configured but the repetition factor is not configured in SIB should be considered.

      - Observation 3: In R17 Msg3 PUSCH repetition, if the UE sends repetition request but no repetition factor is configured in SIB, a default set of Msg3 PUSCH repetition factor {1,2,3,4} is applied.

BWP-UplinkCommon field descriptions

numberOfMsg3-RepetitionsList The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {n1, n2, n3, n4} (see TS 38.214 [19], clause 6.1.2.1).

- ‘1’ for a single factor configuration

  - Yes:

  - No: [25/LGE] [24/DCM] [9/Intel]

    - [9/Intel] PUCCH HARQ-ACK repetition in response to Msg4 PDSCH feature can be enabled or disabled by the gNB in a simpler way.

- FH

  - Intra-slot: [25/LGE] [24/DCM] [17/ZTE] [12/Ericsson] [11/xiaomi] [2/Nokia, NSB]

    - [25/LGE] We think it is natural to support intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK since this scheme was already supported for a single PUCCH transmission for Msg4 HARQ-ACK

    - [24/DCM] when a resource w/ inter-slot FH (e.g., resource with red square below) is used, two resources become unavailable for intra-slot FH. it would be true that the key point is that only max 16 PUCCH resources are available in an NTN-cell. As referred in the last section, capacity of common PUCCH is low.

    - [17/ZTE] frequency hopping within a slot is not enabled by signaling, but is enabled by default

    - [12/Ericsson] Inter-slot frequency hopping performs worse than intra-slot frequency hopping for Msg4 HARQ-ACK PUCCH repetitions.

    - [11/xiaomi] Observation 2: The performance gain of enabling inter-slot frequency hopping is negligible.

    - [2/Nokia, NSB] Except for some very specific scenarios, the NTN channel can generally be considered flat in the frequency domain, and therefore the expected gains from additional frequency hopping would be marginal and would not justify support of such a feature.

    - [2/Nokia, NSB] Further, as the inter-slot frequency hopping for PUCCH is a UE capability feature (indicated by the parameter interSlotFreqHopPUCCH-r17) the gNB would at this stage of the connection establishment procedure not be aware of the UE capabilities and hence not be knowing whether the UE performing the random acecss procedure supports such feature.

  - Inter-slot: [25/LGE] [22/Panasonic] [9/Intel] [3/vivo]

    - [25/LGE] because it is advantageous in terms of frequency diversity, the inter-slot frequency hopping was introduced in Msg3 PUSCH repetition in Rel-17 NR CE

    - [22/Panasonic] Observation 1: Inter-slot FH and DMRS bundling of PUCCH for Msg.4 HARQ-ACK has large gain in NACK-to-ACK error performance but only marginal gain in ACK miss-detection performance. ACK miss-detection is dominant in 4 or less repetitions case and NACK-to-ACK error is dominant in 8 repetitions case.

    - [9/Intel] to further improve the coverage performance for PUCCH for Msg4 HARQ-ACK

    - [3/vivo] Observation 1:

      - The performance gain with inter-slot FH and intra-slot FH decreases with the larger repetition number.

      - Compared to PUCCH repetitions for msg4 HARQ-ACK without FH, around 0.2dB and 0.4dB additional performance gain from PUCCH repetitions with inter-slot FH and intra-slot FH can be obtained, respectively.

    - Details

      - [25/LGE] Further discuss on which slots are applied if the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK is enabled

      - [3/vivo] Proposal 6: Support to enable/disable inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK considering the following alternatives

        - Alt 1: reusing the frequency hopping flag information field in UL RAR grant when the repetition number is configured to be more than 1

        - Alt 2: repurpose some fields in scheduling DCI corresponding to msg4 PDSCH

  - No FH: [7/Baicells]

    - [7/Baicells] Observation 2: For Msg4 HARQ ACK in NTN, intra-slot frequency hopping worsen the performance compared to no intra-slot frequency hopping.

- UE capability signaling details

  - Single: [24/DCM] [23/Sharp] [22/Panasonic] [8/NEC]

    - [23/Sharp] Proposal 4: The UE should support all of the configurable repetition factors.

    - [22/Panasonic] Proposal 10: The UE supporting cell specific indication of PUCCH repetition for msg4 should support dynamic indication of PUCCH repetition for msg4.

    - [8/NEC] In connected mode, PUCCH repetition capability indication applies to all the repetition factors in the set.

  - Multiple:

  - Zero:

  - Others

    - [12/Ericsson] A UE that supports the Rel-18 feature for PUCCH repetition when dedicated PUCCH is not configured shall also support the Rel-17 feature dynamic PUCCH repetition (slotBasedDynamicPUCCH-Rep-r17).

- DMRS bundling

  - Support: [17/ZTE]

    - [17/ZTE] Significant performance improvement can be achieved by DMRS bundling by enabling joint cross-slot channel estimation and frequency pre-compensation.

    - [17/ZTE] Proposal 5: A bit field in Msg3 can be defined to indicate whether to support DMRS bundling for PUCCH for Msg4 HARQ-ACK.

  - Not support: [22/Panasonic] [9/Intel] [2/Nokia, NSB]

    - [9/Intel] Since PUCCH repetition for Msg4 HARQ-ACK corresponds to initial access, this feature may not be supported for all the UEs and additional UE capability report before Msg4 transmission is not preferred.

    - [2/Nokia, NSB] support of such feature is not mandatory from a UE point of view, and therefore support of DMRS bundling for the PUCCH of the Msg4 HARQ-ACK would have a large specification impact since it would require additional capability signalling and network configurations in initial access.

    - [2/Nokia, NSB] if a UE is provided a PUCCH resource by pucch-ResourceCommon (i.e. initial access), UE automatically performs frequency hopping in the initial UL BWP, which would not allow a UE to maintain phase and power consistency and perform DMRS bundling across the PUCCH repetitions.

- PUCCH resource candidates

  - Enhance/Restrict: [25/LGE] [24/DCM] [17/ZTE] [14/Samsung] [12/Ericsson]

    - [25/LGE] Proposal 3. Additional PRB offsets can be supported for each PUCCH repetition factors for Msg4 HARQ-ACK.

      - if the legacy behavior is reused without any changes, the PRB location based on PRB offset (Table 9.2.1-1 in TS38.213 [2]) must be applied equally regardless of the repetition factor. gNB is forced to allocate resources for PUCCH repetition using different time domain resources (because sequence domain resources are limited), resulting in an increase in the average time delay of UEs in the cell.

    - [25/LGE] Proposal 6. The NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

    - [25/LGE] Proposal 7. It is possible to consider creating a new PUCCH resource set table consisting only of PUCCH format 1 with 14 OFDM symbols for Rel-18 NR NTN UE.

    - [24/DCM] Proposal 9: For PUCCH repetition for Msg4 HARQ-ACK, support more than 16 PUCCH resources in a cell.

      - Only 16 resources are available in an NTN-cell, and if e.g., 8 repetition is applied, only 16 resources are available within 8 slots. Our view is that this PUCCH capacity is not sufficient since a lot of users would exist in an NTN cell.

    - [17/ZTE] Proposal 3: The PUCCH format-1 with 14 symbols should be used as the baseline for enhancement.

    - [14/Samsung] Proposal 2: Support at least PUCCH repetition for PUCCH format 1.

    - [12/Ericsson] RAN1 needs to further study common PUCCH resource sets capacity problem.

- Terminology (Enhancement target)

  - ‘when dedicated PUCCH resource configuration is not provided.’: [25/LGE] [17/ZTE] [14/Samsung] [12/Ericsson]

    - [25/LGE] This is because if the PUCCH repetition is only supported for Msg4 HARQ-ACK, and is not supported for PDSCHs received before acquiring the dedicated PUCCH resource sets, the PUCCH for Msg4 HARQ-ACK may be received well, but the PUCCH for other PDSCHs may not be received well at the gNB.

    - [17/ZTE] whether to configure the dedicated PUCCH resource in the Msg4 PDSCH is determined by NW implementation. So even after the HARQ for Msg-4, there is still case that common PUCCH will be reused, e.g., the HARQ feedback for subsequent PDSCHs after successful random access will be carried by the common PUCCH.

    - [14/Samsung] “Msg4” itself is not defined in the specifications and the PDSCH providing contention resolution is not necessarily the first PDSCH after Msg3 PUSCH is correctly received.

    - [12/Ericsson] RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation. During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later.

  - PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI: [9/Intel]

    - [9/Intel] PUCCH transmission when dedicated PUCCH resource configuration is not provided may correspond to HARQ-ACK for a PDSCH transmission which does not carry Msg4. Considering the above, separate discussion is needed to decide if enhancements are required for transmission when dedicated PUCCH resource configuration is not provided.

  - Comment

    - [1/HW, HiSi] Proposal 1: RAN1 to clarify the DL messages that might be scheduled during the RRC procedure delay duration.

- Others

  - [12/Ericsson] The solution for Msg4 HARQ-ACK PUCCH repetition should target use in both NTN and TN.

  - [9/Intel] Specification update needs to be considered to support available slot counting based on existing principles (as in section 9.2.6 of TS 38.213) for the repetition of PUCCH in response to Msg4 HARQ-ACK

    - Section 9.2.6 of TS 38.213 is for PUCCH repetition in RRC connected mode, and both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationCommon are used to determine the available slot in that case

  - [9/Intel] Same Tx beam and same PUCCH resource are applied for all the repetitions of PUCCH transmission with HARQ-ACK in response to Msg4 PDSCH

  - [5/Spreadtrum] Proposal 6: Beam-level repetition value configuration of PUCCH for Msg4 HARQ-ACK can be supported.

  - [2/Nokia, NSB] Observation 16: No specification update to apply the existing mechanism on repetition slot counting to PUCCH repetition for Msg4 HARQ-ACK is needed.

  - [2/Nokia, NSB] Proposal 14: Postpone discussion on definition of N_PUCCH^repeat for PUCCH repetition for Msg4 HARQ-ACK to when the feature details are agreed and consolidated.

  - DMRS bundling for PUSCH taking into account NTN-specifics

- Phase rotation analysis

  - Observation

    - [24/DCM]

      - When the transmission is allocated at the center of the carrier bandwidth, the observed phase difference per 1 ms is approximately 4.6 deg.

        - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) can be satisfied within 6 slots in this case

      - When the transmission is allocated at the edge of the carrier bandwidth, the observed phase difference per 1 ms is approximately 114 deg.

        - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots in this case

    - [21/QC]

      - Observation 1: For LEO,  to meet the existing phase continuity requirement for DMRS bundling, the bundling window is less than 1 ms for 10 MHz carrier bandwidth at low elevation angles if transmit time and frequency remains unchanged during the window.

      - Observation 2: With segmented pre-compensation at UE and  post compensation of the timing error of the beam center at gNB in LEO, DMRS bundling with a large bundling size, e.g., 8, is feasible.

    - [18/MTK]

      - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) can be satisfied over TDW not larger than 4 ms and maximum operating bandwidth of 360 kHz (i.e. 2 RBs) without assuming phase pre-compensation at UE and/or RX phase post-compensation at gNB.

      - In case of PRB index that are not contiguous in an operating bandwidth larger than 360 kHz or a TDW larger than 4 ms, the phase difference limit requirement can only be met assuming phase pre-compensation at UE and/or RX phase post-compensation at gNB.

    - [15/ETRI]

      - Observation 7: The phase difference limit (38.101-1 Table 6.4.2.5-1) might be satisfied within the following value

        - UE velocity=1200 km/h

          - LEO-600, elevation=10°: L_(TDW,DMRS)≤(9.96 ms)/N_RB = 0.39 ms (@ 25 RB)~9.96 ms (@ 1 RB)

          - LEO-600, elevation=30°: L_(TDW,DMRS)≤(11.33 ms)/N_RB = 0.45 ms (@ 25 RB)~11.33 ms (@ 1 RB))

          - LEO-1200, elevation=30°: L_(TDW,DMRS)≤(12.79 ms)/N_RB = 0.51ms (@ 25 RB)~12.79 ms (@ 1 RB)

        - UE velocity=0 km/h

          - LEO-600, elevation=10°: L_(TDW,DMRS)≤(10.02 ms)/N_RB   = 0.40 ms (@ 25 RB)~10.02 ms (@ 1 RB)

          - LEO-600, elevation=30°: L_(TDW,DMRS)≤(11.6 ms)/N_RB = 0.46 ms (@ 25 RB)~11.6 ms (@ 1 RB)

          - LEO-1200, elevation=30°: L_(TDW,DMRS)≤(13.1 ms)/N_RB = 0.52 ms (@ 25 RB)~13.1 ms (@ 1 RB)

      - Note: the phase difference limit (38.133 Table 6.4.2.5-1) cannot be satisfied over multiple slots, if the farthest subcarrier from the center is taken from the edge of the 5 MHz channel bandwidth.

    - [12/Ericsson] Due to time drift, a phase difference between consecutive slots of more than 25° is experienced in a large part of a 5 MHz bandwidth.

    - [12/Ericsson] If the PRB allocation of a UE is narrow, the phase drift of the different subcarriers will be very similar and appear almost like a frequency offset of the whole signal from the receiver point of view.

      - DMRS bundling performance with CFO compensation is almost the same at the edge and at the center of a 5 MHz carrier, despite the large phase drift at the carrier edge.

      - Timing drift gives almost no degradation on DMRS bundling performance with CFO compensation for narrow (1-2) PRB allocations, but larger degradation for wider (4-8) PRB allocations.

    - [7/Baicells] Observation 7:  At the edge of N PRB bandwidth (away from the carrier center), in M slots, the phase difference due to timing error is 5.17 (deg) * N (PRB) * M (slot) for LEO-600, and 4.57 (deg) * N (PRB) * M (slot) for LEO-1200.

    - [4/OPPO] Observation 6: For DMRS bundling in LEO 1200 with elevation angle of 30 deg and SCS = 15kHz, the phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots if the PRB position is not within 6 PRBs from the directive current (DC) subcarrier.

    - [3/vivo] Observation 2: For DMRS bundling, the maximum duration due to phase difference is limited by 4ms for LEO-1200 with 2 PRBs scenarios and 2ms for LEO-1200 with 4 PRBs scenarios.

    - [2/Nokia, NSB] Observation 5: Impact of round trip time drift leads to the violation of the requirements for phase difference across DMRS symbols sooner than the requirement of timing error.

    - [1/HW, HiSi] Observation 3: Phase difference limit cannot be satisfied over multiple slots for carrier bandwidth 5 MHz, if TA pre-compensation update, phase pre-compensation, and RX timing post-compensation are not assumed, and if 70.5 (us/s) timing drift rate is assumed.

  - RAN1 assumption for this issue

    - A) UE pre-compensation: [24/DCM] [17/ZTE (?)] [12/Ericsson] [2/Nokia, NSB] [1/HW, HiSi]

      - [1/HW, HiSi] a new TDW capability can be reported to help gNB configure a proper bundling window depending on UE reports.

    - B) gNB post-compensation: [24/DCM] [21/QC] [12/Ericsson] [4/OPPO]

      - [21/QC] RAN1 asks RAN4 to consider associated requirement for DMRS bundling for segmented pre-compensation.

      - [12/Ericsson] The TDW length cannot be increased by post-compensation of the RX timing in gNB.

        - in the gNB receiver, the FFT may be applied to the whole UL signal with an RX window that cannot be adjusted individually per UE. The UEs in a spotbeam will not necessarily have aligned TDWs and will therefore autonomously update their TAs at different times.

      - [12/Ericsson] Post-compensation of phase drift can be implemented in gNB without specification impact.

      - [4/OPPO] Considering the timing drifts within a satellite beam are close, the RX timing post-compensation at gNB is possible to improve the performance of channel estimation.

    - C) gNB scheduling restriction: [24/DCM]

    - D) Ask RAN4 on the feasibility of A/B: [22/Panasonic]

    - E) Ask RAN4 on the following: [14/Samsung]

      - The feasibility of DMRS bundling for NTN when the UE applies TA pre-compensation.

      - Maximum allowable phase difference for NTN-specific DMRS bundling and related assumptions

    - F) Ask RAN4 on the following: [12/Ericsson]

      - whether the phase continuity requirement in clause 6.4.2.5 38.101-1 should be modified for NTN UE e.g. to allow for a linear phase drift that can be compensated by CFO compensation.

    - Others

      - [17/ZTE] UE reports capability on whether to support DMRS bundling across pre-compensation segments

      - [12/Ericsson] Consider defining a UE capability to distinguish between different UE implementations, such as:

        - 1) UE that can pre-compensate its local clock to cancel the Doppler shift effect at the start of the TDW

        - 2) UE that can update time and frequency compensation inside the TDW while maintaining coherent transmission

        - 3) UE that can only update time and frequency pre-compensation at TDW borders

- UE behavior within each actual TDW

  - No TA adjustment: [25/LGE] [21/QC] [20/Lenovo] [12/Ericsson] [11/xiaomi] [4/OPPO] [3/vivo]

    - [21/QC] Proposal 6: For DMRS bundling, support UE capability signalling of TDW based pre-compensation where time and frequency may be precomepsated at the beginning of a TDW but remain unchanged during the TWD.

  - [12/Ericsson] RAN1 to discuss assumption for initial timing error at the start of the TDW.

    - The timing accuracy requirement for NTN UE is 29Ts (assuming 15 kHz SCS) but not all of this timing error budget can be used for managing the timing drift due to paused updating of the open-loop TA within the TDW.

  - [6/CATT] The initial timing error at the beginning of TDW must be considered.

  - [2/Nokia, NSB] Observation 3: Initial timing error of UE at the beginning of bundling affects the maximum supported DMRS bundle size. However, it is difficult to have an assumption on this initial timing error value, as UE’s timing error may drift to positive or negative T_e, and various sources of error are contributing to the entire timing error budget.

- Additional gaps between two segments

  - [9/Intel] Additional gaps to adjust the UE pre-compensation values are not supported for NR NTN

- TDW determination

  - Actual TDW determination by UE indication

    - YES: [25/LGE] [23/Sharp] [22/Panasonic (?)] [20/Lenovo] [16/CMCC] [15/ETRI (?)] [13/China Telecom] [11/xiaomi] [10/Sony (?)]

      - [16/CMCC] a UE self-estimated TA to pre-compensate for the service link delay, which is calculated based on the UE position and the serving satellite ephemeris, may be transparent for gNB

      - [11/xiaomi] the length of the time window that UE can maintain phase continuity is varying with the elevation angle. Larger repetition factor can be configured with larger elevation angle between UE and satellite. Therefore, multiple nominal TDWs can be configured by gNB based on the variable elevation angles, and nominal TDW is determined and indicated by gNB in semi-static way.

    - Nominal TDW is configured by gNB as in the existing spec (?)

  - Nominal/Actual TDW determination by gNB configuration/indication

    - YES: [24/DCM] [21/QC] [14/Samsung] [13/China Telecom] [12/Ericsson] [20/Lenovo] [19/Apple] [17/ZTE] [3/vivo] [18/MTK (?)] [4/OPPO (?)]

    - For gNB configuration/indication

      - No enh

        - YES: [24/DCM] [21/QC] [14/Samsung] [13/China Telecom] [12/Ericsson]

          - [14/Samsung] it is not clear which aspects of current specifications cannot support proper DMRS bundling for NTN service

          - [14/Samsung] Observation 1: Rel-17 coverage enhancement design for DMRS bundling provides fully flexible window size for DMRS bundling.

          - [12/Ericsson] The Rel-17 feature DMRS bundling already supports configuration of transmission segments equivalent to Rel-17 IoT NTN segmented uplink transmission through UE-specific RRC configuration of the nominal TDW length. No enhancement of the configuration is necessary.

          - [12/Ericsson] The gNB can e.g. determine the max Doppler/timing drift rate in a cell based on serving satellite ephemeris, and/or estimate/predicte timing drift based on the UL signal from the UE, and configure the UE with a nominal TDW such that pre-compensation updates can be paused within the TDW without exceeding requirements on time/frequency accuracy.

        - NO: [25/LGE] [23/Sharp] [2/Nokia, NSB]

          - [25/LGE] it could result in performance degradation, especially when significant TA drift is predicted

          - [23/Sharp] since the timing of UE pre-compensation is determined by gNB, regarding TA pre-compensation update, implementation of the Rel-17 NR NTN chip-set may not be reused for that of the Rel-18 NR NTN chip-set

          - [2/Nokia, NSB] this may lead to constant RRC reconfigurations, which is not an attractive solution. To resolve this issue, more dynamic indication/adjustment of nTDW length is useful. Feasibility of this option, when configuration/indication of nTDW length is not associated with individual UEs is questionable, as different UEs may experience different timing drifts.

      - New gNB configuration/indication

        - YES: [20/Lenovo] [19/Apple] [17/ZTE] [3/vivo]

          - [19/Apple] Proposal 8: The nominal TDW size of PUSCH DMRS bundling (if configured) is upper bounded by the indicated PUSCH segmented transmission duration.

            - Update of PUSCH segmented transmission duration autonomously reconfigures nominal TDW size of PUSCH DMRS bundling for all UEs in coverage.

          - [17/ZTE] in R17 IoT-NTN, segment pre-compensation mechanism has already been agreed and specified, where eNB is responsible for determining the segment length for pre-compensation.

        - NO: [13/China Telecom]

          - [13/China Telecom] the configuration of nominal TDW in the current specification is very flexible. The length of a nominal TDW can be configured from 2 to 32 slots. In this sense, pre-compensation timing and segmented configuration can already be realized by configuration of different length of nominal TDWs.

        - Comment

          - [2/Nokia, NSB] Observation 7: Configuration of nTDW/aTDW as part of UL segment duration configuration does not provide additional functionality and benefits, as nTDW length can act as UL segment duration. Therefore, we do not see the benefits of this approach compare with option 2-1.

          - [2/Nokia, NSB] Observation 8: For option 2-2 of proposal 2-3_v5, gNB may provides TA update assisting information to NTN UE such as indicating the rate or periodicity of TA udates or instances of TA update. New event is also defined based on UE TA update.

  - For UE information report

    - No enh

      - YES: [18/MTK] [4/OPPO]

        - [4/OPPO] gNB can provide an appropriate TDW as in segment transmission in R17 IoT NTN.

      - NO:

    - UE indication of TA pre-compensation timing

      - YES: [25/LGE (?)] [22/Panasonic (?)] [3/vivo]

    - With new UE capability

      - YES: [25/LGE] [24/DCM] [22/Panasonic]

        - [25/LGE] Proposal 13: For DMRS bundling in NTN environment, introduce a new UE capability based on types of NTN platforms.

          - In NTN, this problem is magnified as it is highly dependent on NTN platform, rather than whether the UE can manage it or not.

        - [24/DCM] Proposal 12: Discuss new UE capability signaling to report the max TDW size where TA pre-compensation update causing phase discontinuity that may violate RAN4 requirement is not performed

        - [22/Panasonic] Proposal 13: Whether time and phase drift due to satellite movement is taken into account or not in the TDW reported should be specified.

  - Others

    - [19/Apple] Proposal 9: If the nominal TDW size of PUSCH DMRS bundling is not configured, then the actual TDW size is equal to the minimum of maximum duration reported by UE, PUSCH segmented transmission duration, PUSCH repetition duration and the frequency hopping interval for PUSCH DMRS bundling.

- Other event which causes power consistency and phase continuity not to be maintained

  - Antenna switching as the event

    - Yes: [11/xiaomi (?)] [1/HW, HiSi]

      - [11/xiaomi] Proposal 9: Antenna switching is supported when the TDW size of PUSCH DMRS bundling is lower than a threshold.

      - [1/HW, HiSi] Proposal 10: Support the mechanism to jointly utilize antenna switching and DMRS bundling for NTN.

    - No: [12/Ericsson]

      - For PUSCH DMRS bundling, TX antenna switching between the TDWs can provide a substantial gain.

      - Antenna switching can be implemented in the UE without specification impact.

  - Indication or update of epoch time: [9/Intel] [3/vivo]

  - Application of new common TA and satellite ephemeris: [9/Intel]

- Others

  - [12/Ericsson]

    - Conclude that there is a coverage gap for Msg3 PUSCH in case of CFRA that can be solved by Msg3 PUSCH repetition.

    - Send an LS to RAN2 informing them of the coverage gap for Msg3 in case of CFRA and recommend them to specify signalling support for Msg3 repetitions in this case.

  - [6/CATT]

    - The limitation of phase difference is affected by modulation mode.

    - Consider suitable TDW length for different modulation mode.

  - [5/Spreadtrum] Proposal 7: TDW size should be associated with PUSCH repetition number through configuration by gNB.

- References

- R1-2302364	Discussion on coverage enhancement for NR NTN	Huawei, HiSilicon

- R1-2302433	Further considerations on coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2302502	Discussions on remaining issues of coverage enhancements in NR NTN	vivo

- R1-2302564	Discussion on coverage enhancement for NR NTN	OPPO

- R1-2302616	Discussion on coverage enhancements for NTN	Spreadtrum Communications

- R1-2302719	Further discussion on UL coverage enhancement for NR NTN	CATT

- R1-2302738	Discussion on coverage enhancement for NR NTN	Baicells

- R1-2302748	Coverage enhancement for NR NTN	NEC

- R1-2302812	On coverage enhancement for NR NTN	Intel Corporation

- R1-2302857	On coverage enhancement for NR NTN	Sony

- R1-2302998	Discussion on coverage enhancement for NR-NTN	xiaomi

- R1-2303014	On coverage enhancements for NR NTN	Ericsson

- R1-2303032	Discussion on coverage enhancement for NR NTN	China Telecom

- R1-2303144	On coverage enhancement for NR NTN	Samsung

- R1-2303204	Discussion on coverage enhancement for NR NTN	ETRI

- R1-2303250	Discussion on coverage enhancement for NR NTN	CMCC

- R1-2303294	Discussion on coverage enhancement for NTN	ZTE

- R1-2303351	UL coverage enhancements	MediaTek Inc.

- R1-2303499	On Coverage Enhancement for NR NTN	Apple

- R1-2303534	Coverage enhancements for NR NTN	Lenovo

- R1-2303606	Coverage enhancements for NR NTN	Qualcomm Incorporated

- R1-2303625	Discussion on coverage enhancement for NR-NTN	Panasonic

- R1-2303643	Views on Coverage enhancement for NR NTN	Sharp

- R1-2303725	Discussion on coverage enhancement for NR NTN	NTT DOCOMO, INC.

- R1-2303748	Discussion on coverage enhancement for NR NTN	LG Electronics

- Appendix-1 (Copy from WID RP-230809)

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

- Appendix-2 (Outcomes of post meetings)

  - RAN1#109-e

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

R1-2205622	[Draft] LS on UE antenna gain for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

R1-2205623	LS on UE antenna gain for NR NTN coverage enhancement	RAN1, NTT DOCOMO, INC.

Final LS is endorsed in R1-2205623.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

  - RAN1#110

Conclusion

For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

Agreement

For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18.

- Potential enhancements for LEO can also apply to GEO and MEO

Agreement

For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services:

- VoIP using AMR 4.75 kbps.

- Low data rate of 3 kbps.

- Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2

Observation

For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS,

- Five sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

Conclusion

RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB

For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification can meet the performance requirement

- Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB

For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB

  - One company simulated by using 20 repetitions without DMRS bundling

  - Four companies simulated by using 20 repetitions with DMRS bundling

  - One company simulated by using 32 repetitions with DMRS bundling

    - Note: this is the only result using frame combining by application layer

- Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB

  - Eight companies simulated by using 20 repetitions without DMRS bundling

  - Seven companies simulated without frequency hopping

  - One company simulated by using 16 repetitions with DMRS bundling

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied.

Observation

For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB.

Conclusion

RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

  - RAN1#110bis-e

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

Agreement

For NTN-specific PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc.

  - Note: baseline performance for legacy UEs can include antenna switching

Agreement

For PUCCH transmission for Msg4 HARQ-ACK,

- Supported number of transmissions are 1, 2, 4, 8.

  - Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions.

  - FFS: whether larger number of transmissions is supported

  - FFS: whether/how single PUCCH transmission can be configured and/or indicated

  - RAN1#111

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Conclusion

RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP

- FFS: How to determine TDW size, including UE capability.

- Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

  - RAN1#112

Observation

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed.

  - FFS: whether/how to consider the initial timing error at the beginning

  - FFS: TA pre-compensation update is assumed

- Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed.

- FFS: impact of phase difference limit

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE.

- Option A: PRACH preamble and/or occasion

  - FFS: whether PRACH resource partitioning is needed for indication of [repetition request  or capability report]

  - FFS: whether or not indication of repetition factor is assumed

  - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings

- Option B: Higher layer signaling in Msg3 PUSCH

  - FFS: which signaling is used

  - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2.

- Option C: Physical layer signaling in Msg3 PUSCH

  - FFS: which signaling is used, e.g. DMRS ports

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

  - Alt 1-2: New field with one or two bits

- Alt 2: Field in DCI scheduling Msg3 PUSCH

  - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor

  - Note: it is assumed that there is impact on DCI design

- Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH

  - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc.

- Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

- Appendix-3 (Contact information)

| Company | Name | Email |
| --- | --- | --- |
| FL (DCM) | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Lenovo | Hongmei Liu | Liuhm6@lenovo.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunhai_yao@apple.com |
| Xiaomi | Min Liu | Liumin10@xiaomi.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| vivo | Yong Wang | wy.wang.5g@vivo.com |
| Nokia | Frank Frederiksen | Frank.frederiksen@nokia.com |
| OPPO | Hao LIN | lin.hao@oppo.com |
| OPPO | Zuomin WU | wuzuomin@oppo.com |
| OPPO | Nande Zhao | zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaiolei@huawei.com |
| Huawei, HiSilicon | Ying Chen | chenying18@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Ericsson | Stefan Eriksson Löwenmark | stefan.g.eriksson@ericsson.com |
| Thales | Mohamed EL JAAFARI | mohamed.el-jaafari@thalesaleniaspace.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Gilles Charbit | Gilles.charbit@mediatek.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Sony | Samuel Atungsiri | Sam.Atungsiri@sony.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| ETRI | Dukhyun You | dhyou@etri.re.kr |
| ETRI | Jung-Bin Kim | jbkim777@etri.re.kr |
| ETRI | Gyeongrae Im | imgrae@etri.re.kr |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Samsung | Sungjin Park | sj100.park@samsung.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| Omnispace | Ron Olexa | rolexa@omnispace.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Ligado | Clive Packer | clive@ligado.com |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com; munirajaffar@hughes.com |
| Qualcomm | Xiao Feng Wang | wangxiao@qti.qualcomm.com |
| Qualcomm | LiangPing Ma | lpma@qti.qualcomm.com |
| Novamint | Thierry Bérisot | tberisot@novamint.com |
| GateHouse | Robert van der Pool | rvp@gatehouse.com |
| FGI | YenHua Li | danielli@fginnov.com |
| LG | Haewook Park | haewook.park@lge.com |
| LG | Seokmin Shin | seokmin.shin@lge.com |
| LG | Duckhyun Bae | duckhyun.bae@lge.com |
| Baicells | Xiang Yun | yunxiang@baicells.com |
| Baicells | Yong Ding | dingyong@baicells.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| China Telecom | Jianchi Zhu | zhujc@chinatelecom.cn |