3GPP TSG RAN WG1 #112bis-e		R1-230xxxx

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.2

Source:	Moderator (vivo)

Title:	Summary #1 on Measurements and reporting for SL positioning

Document for:	Discussion and Decision

Introduction

It has been capture in “New WID on Expanded and Improved NR Positioning” [1] to specify measurements and reporting for SL positioning.

- Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

In this paper, we summarize some common elements in the contributions and identify some areas and positions where contributing companies are aligned from which some agreements could be derived at this meeting.

Discussion information

The proposals are ranked according to HIGH, MEDIUM, LOW. The intention is to start in the offline/online sessions from the HIGH proposals

Sidelink Positioning Measurements: Procedures and definitions

It was explicitly captured in the WID to focus on positioning methods including “RTT-type solutions using SL, SL-AoA, and SL-TDOA”. Moreover, the following agreements were reached in the study phase.

Agreement With regards to the Positioning methods supported using SL-PRS measurements - at least the following measurements should be introduced: - SL-PRS based Rx-Tx measurement - SL-PRS based RSTD measurement - SL-PRS based RSRP measurement - SL-PRS based RSRPP measurement - SL-PRS based RTOA measurement - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

Based on the submitted contributions and above agreements, discussion in this section is divided into the following sub-sections:

- SL-PRS based Rx-Tx measurement

- SL-PRS based RSTD measurement and RTOA measurement

- SL-PRS based RSRP measurement and RSRPP measurement

- SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

Corresponding issues for sidelink positioning measurements mentioned by companies are categorized into each of above subsection. During RAN1 #112bis, it is expected for companies to clarify the issues, debate on necessity of resolving the issues and progress on directions to address the high priority issues.

SL-PRS based Rx-Tx measurement

The following agreements were achieved in RAN1 #112.

Agreement

For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

Agreement

For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs.

- FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report

- FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders

  - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

Based on the submitted contributions, the following issues are identified to be related to this topic:

- Definition of Rx-Tx difference using actual SL-PRS Tx and Rx time

- Support of report-free SL-RTT

- PRS transmission order and differentiation of different PRS transmissions for multiple rounds of double sided RTT.

- Measurement procedures

[HIGH] Definition of Rx-Tx difference using actual SL-PRS Tx and Rx time

The following proposals are identified to be related to this issue.

| R1-2302327 FUTUREWEI | Proposal 1: Support either Alt 1 or Alt 3 for the definition of SL-PRS based Rx-Tx measurement. If Alt 3 is supported TA shall contain the value of time adjustment to the new synchronization source, if any. |
| --- | --- |
| R1-2302378 Huawei, HiSilicon | Proposal 1: For definition of SL-PRS based Rx-Tx measurement, support - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. |
| R1-2302491 vivo | - For RTT, Rx – Tx time difference is defined as TUE-RX – TUE-TX , where - TUE-RX is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time. - TUE-TX is the UE transmit timing of sidelink subframe #j considering the actual SL-PRS transmission time. - For RTT, if Alt 1 is adopted, downselect UE Rx-Tx time difference measurement as the following - Alt 1-1: report the Rx – Tx time difference as: Nms+X×2u×TC with DL reception time stamp - Alt 1-2: report the Rx – Tx time difference as: X×2u×TC with DL reception time stamp and UL transmission time stamp |
| R1-2302554 OPPO | - The definition of SL Rx-Tx time difference can be modified based on UE Rx-Tx time difference definition as follows: TSL-TX is the UE transmit timing of sidelink subframe #j that is closest in time to the subframe #i received from another UE. |
| R1-2302606 Spreadtrum Communications | Proposal 1: Actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement. |
| R1-2302709 CATT | Proposal 1: For definition of SL-PRS based Rx-Tx measurement for single-sided RTT, Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu (Alt3) should be reused. Proposal 2: For definition of SL-PRS based Rx-Tx measurement for double-sided RTT, actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement (Alt1). |
| R1-2302802 Intel | - For the transmission time considered for the Rx-Tx time difference measure the actual SL-PRS transmission time is used (Alt. 1). |
| R1-2302852 Sony | Proposal 2: Support using actual SL-PRS transmission time for definition of SL-PRS based Rx-Tx time difference measurement. |
| R1-2302989 xiaomi | Proposal 1: Definition of SL-PRS based Rx-Tx measurement is based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. |
| R1-2302927 LG Electronics | Proposal 3: The following definition is used for SL Rx-Tx time difference measurement by the UE that transmits the responding SL PRS in SL RTT. TUE-TX is the UE transmit timing of the start of SL subframe #j that is closest in time to the subframe #i received from the UE, measured based on the synchronization timing that was used for measuring TUE-RX. Proposal 4: The following definition is used for SL Rx-Tx time difference measurement by the UE that transmits the initiating SL PRS in SL RTT. Proposal 5: The above definitions are added as a new option for SL Rx-Tx time difference. Proposal 7: The displacement of UE between SL PRS transmission and reception is reported to the location calculation entity in SL RTT. |
| R1-2303134 Samsung | Proposal 2: For definition of SL-PRS based Rx-Tx measurement, SL-PRS transmission time based on the SL PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement. - The reporting range for the Rx-Tx difference for SL RTT is 1msec. - Reuse the resolution step defined in TS 38.133 for Uu Rx-Tx difference time. |
| R1-2303240 CMCC | Proposal 8: For definition of SL-PRS based Rx-Tx measurement, support Alt. 1 or Alt. 3 agreed in RAN1#112 meeting: - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. FFS measurement reporting details to minimize the impact of UE reference timing offset and mobility. |
| R1-2303264 Lenovo | Proposal 8: RAN1 to support Alt.1 in defining the SL PRS-based Rx-Tx time difference measurement, where the actual SL-PRS transmission times are incorporated in the definition. |
| R1-2303277 ZTE | Proposal 5: With regards to the RTT-type solutions using SL, the same definition of UE Rx-Tx time difference for SL positioning (TUE-RX – TUE-TX) is applied for both single-sided or double-sided RTT method: - TUE-RX is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time. - TUE-TX is the UE transmit timing of sidelink subframe #j based on actual transmission time |
| R1-2303489 Apple | Proposal 3-3: to minimize the impact of UE reference timing offset and mobility select Alt 1 i.e. actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement |
| R1-2303551 Ericsson | - For SL-PRS based Rx-Tx definition, support Alt 1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement. |
| R1-2303596 Qualcomm | Proposal 8: Actual SL-PRS transmission time is used for the definition of the UE SL Rx-Tx time difference measurement (i.e. Alt 1). |
| R1-2303842 MediaTek | Proposal 2-1: For SL RX-TX time difference definition, reuse the R16/R17 definition in Uu Proposal 2-2: Consider to have additional signaling to indicate the transmission timing change quantity with a timestamp |
| R1-2303415 Fraunhofer | Proposal 2: Support using the actual SL-PRS transmission time for the definition of SL-PRS based Rx-Tx time difference measurement. |

Based on companies TDoc, supporting company of Alt1 and Alt 3 is listed as following:

For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Supporting company: Futurewei, vivo, CATT(for DS-RTT), Intel, CMCC, Lenovo, ZTE, Apple, Ericsson, Qualcomm, Spreadtrum, Sony, Fraunhofer, Spreadtrum Communications - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Supporting company: Samsung - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. - Supporting company: Huawei/Hisi, OPPO, CATT(for SS-RTT), LGE, CMCC, MediaTek, Xiaomi

Based on FL’s understanding, this issue needs to be resolved.

#### [ROUND1][High] FL proposal 3.1.1-v1:

For definition of SL-PRS based Rx-Tx measurement, support the following Alt1 to minimize the impact of UE reference timing offset and mobility

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Note：This is at least applicable for double sided RTT definition, FFS on whether a different definition is needed for single sided RTT definitions.

- FFS whether to limit the reporting range to within 1msec.

Companies are invited for comments in the following table.

Table 3.1.1 Collection of views on FL proposal 3.1.1-v1

| Company | Views on FL proposal 3.1.1-v1 |
| --- | --- |
| CATT | We prefer to adopt Alt3 for the definiation of SS-RTT, and adopt Alt1 for the definition of DS-RTT. |
| CMCC | We are OK to support Alt. 1. |
| OPPO | Alt 3 Definition in current specification, i.e. Alt 3, is applicable. In RTT, the 2 involved UEs should use same sync-reference, if the refernence timing of one UE is changed, the other is changed with same timing offset, Alt 3 would not introduce additional error due to reference timing offset. We disagree with the Note, the distinction between double-sided and single-sided should be minimized. |
| Qualcomm | We support the proposal |
| Futurewei | Support Alt 3 and Alt 1 . In our understanding, the issue is a possible change in the synchronization source between the two instances (Rx and Tx). We note that a similar issue may happen when RSTD is reported (issue 3.2.1-v1), which require a similar solution. |
| DCM | OK |
| LGE | Alt 1 requires a wider range of Rx-Tx time difference than Alt 3 (legacy). We don’t see any benefit of reporting the actual time difference in “literal-sense”. The reported range should be suppressed within 1 msec, similar to the legacy operation. We have submitted a proposal for such enhancement of the legacy operation as follows. Proposal 3: The following definition is used for SL Rx-Tx time difference measurement by the UE that transmits the responding SL PRS in SL RTT. TUE-TX is the UE transmit timing of the start of SL subframe #j that is closest in time to the subframe #i received from a UE, measured based on the synchronization timing that was used for measuring TUE-RX. The above is equivalent to Alt 1 with the reported range suppression (or improved Alt 3). With this suppression, we don’t need FFS point any more. The proposed operation should be applied to both single- and double-sided RTT. We suggest to remove the note. Based on the observations, we suggest to add the following clarification. #### [ROUND1][High] FL proposal 3.1.1-v1: For definition of SL-PRS based Rx-Tx measurement, support the following Alt1 to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - TUE-TX is the UE transmit timing of the start of SL subframe #j that is closest in time to the subframe #i received from a UE, where TUE-TX is measured based on the synchronization timing that was used for measuring TUE-RX. - Note：This is at least applicable for double sided RTT definition, FFS on whether a different definition is needed for single sided RTT definitions. - FFS whether to limit the reporting range to within 1msec. |
| Samsung | For Alt 1, we think that reporting the interger subframe part is not necessary. Clearly, this has no benefit and will make an additional issue for deciding the range of Rx-Tx measurement reporting. Even though this issue is captured as FFS, we want to resolve this issue with further discussion since the meaning of wording ‘acutal’ in Alt 1 includes the interger subframe part in our understanding. If we modify Alt 2 as below, it becomes Alt 1 without integer subframe part and it is also OK for us. - Alt2-2: SL-PRS transmission time based on the sidelink PRS receiving symbol subframe is used for the definition of SL-PRS based Rx-Tx time difference measurement With this modification, we think there will be more supporting companies. |
| Intel | We support Alt. 1. In our understanding this definition should be adopted for both double and single sided RTT. |
| Lenovo | Support FL’s proposal and share the same view that it should be applicable to the single-sided RTT definition as well. |
| ZTE | We support Alt 1 for both SS-RTT and DS-RTT. |
| Xiaomi | Alt 3 is preferred to reuse the definition in Uu. We do not think UE synchronizaiton refrence change is an issue necessary to solve as the duration of RTT based measurement is rather short compared with SL synchronizaiton reference selection procedures. |
| Huawei, HiSilicon | With regards to Alt.1, we have a couple of questions for clarification: Q1: Does it assume that UE should use its own internal clock to tick the time, or does it require the target UE to use the sync reference timing at the Rx time and at the Tx respectively? Q2: Does it require UE to actually transmit the SL-PRS in order to generate the corresponding measurement? |
| vivo | We support the proposal For Q1 from Huawei, we hope the reception timing and transmission timing are based on its own internal clock, and then the clock offset can be removed by double-sided RTT. But, the transmission timing may be adjusted by sync reference timing, in this case, the offset should be considered on both sides. For Q2 from Huawei, yes, it requires. |
| Apple | We support the proposal. |
| Fraunhofer | Okay with the proposal. However, there are concerns regarding the clarity of both FFS(s). Firstly, it is not clear why the actual transmission time should not be applicable for single-sided scenarios. Secondly for the second FFS, the intuition behind limiting the reporting range seems to be to constrain the time between transmission and reception. If that is not the case, it would be helpful to further elaborate on the relevance of the reporting range in this context. |
| SONY | We support the proposal (Alt.1). Alt.1 should be compatible with both SS-RTT and DS-RTT |
| Nokia, NSB | OK with Alt1. |
| InterDigital | We support to keep FFS for 1ms restriction. We would like to clarify whether the 1ms constraint will limit performance of the alternatives in the FL’s proposal. |

[HIGH]PRS transmission order and differentiation of different PRS transmissions for multiple rounds of double sided RTT

The following proposals are identified for this issue.

| R1-2302294 Nokia, Nokia Shanghai Bell | - Proposal 7: In SL RTT, UE indicates to the other UE the need for switching from one SL RTT method to another |
| --- | --- |
| R1-2302378 Huawei, HiSilicon | - Proposal 2: With regards to flexible transmission orders of SL-PRS for RTT-type sidelink positioning - The time stamp in the measurement report with the existing UE Rx – Tx time difference measurement is sufficient to differentiate different SL-PRS transmissions. - An inter-UE SL-PRS transmission request, which can be based on either higher layer signaling or lower layer signaling, should be introduced. |
| R1-2302709 CATT | Proposal 3: For SL-PRS based double-sided RTT, time resource assignment (similar to the time resource assignment field of SCI format 1-A) is used to differentiate different SL-PRS transmissions for SL PRS Rx-Tx measurement. Proposal 4: For SL-PRS based RTT, RTT type (i.e. single-sided RTT, double-sided RTT and report-free RTT if supported) should be configured by higher layer signaling. Proposal 5: Transmission order for double-sided RTT should be configured by higher layer signaling. Proposal 6: For SL-PRS based RTT, RTT type and transmission order for double-sided RTT could be included in the measurement report. |
| R1-2302802 Intel Corporation | - A single report format is defined for single and double sided RTT |
| R1-2302927 LG Electronics | Proposal 18: The measurement report of double-sided SL RTT includes the information on which type of double-sided SL RTT methods and which type of SL PRS transmission order are used. |
| R1-2303277 ZTE | Proposal 4: With regards to double-sided RTT for SL positioning, - Introducing a DS-RTT round-trip indicator in SCI to indicate the transmission of the 1st and 2nd TX-to-RX round-trip. |
| R1-2303596 Qualcomm | Observation 3: The SCI accompanying SL-PRS can be used to distinguish the different SL-PRS transmissions in double-sided RTT without order restriction. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 3.1.2-v1:

- Support to use higher layer signaling to distinguish single-sided RTT and double-sided RTT.

- Support to use SCI accompanying SL-PRS to distinguish the different SL-PRS transmissions in double-sided RTT without order restriction.

Companies are invited for comments in the following table.

Table 3.1.2 Collection of views on FL proposal 3.1.2-v1

| Company | Views on FL proposal 3.1.2-v1 |
| --- | --- |
| CATT | Support. Regarding the second bullet(the SCI indication), for double-sided RTT without order restriction, time resource assignment (similar to the time resource assignment field of SCI format 1-A) can be used to differentiate different SL-PRS transmissions for SL PRS Rx-Tx measurement. |
| CMCC | We are not sure the exact meaning of the 1st bullet, does it mean to use higher layer signalling to configure the SS-RTT or DS-RTT method? If so, we are fine with it. Regarding the 2nd bullet, basically, we think SCI can be used for the UEs performing DS-RTT to coordinate with the SL PRS transmission resources and orders, including one UE schedules the other UE for SL PRS transmission. |
| OPPO | Specification change on top of single sided RTT should be minimized, only essential change is necessary. |
| Qualcomm | We are ok with the first bullet. For the second bullet, does this mean that additional fields in SCI are added to support this function or that identification information that would be included in SCI is sufficient? |
| Futurewei | Some clarifications are necessary on the SCI purpose and information carried. In principle OK with both. |
| DCM | We are not sure spec impact for double-sided mechanism. Motivation for these two bullets is unclear for us. |
| LGE | The most important advantage of double-sided SL RTT without order restriction is that the SL PRS resources can be dynamically selected by UE based on e.g. channel status. Instead of pre-defined fixed order of SL PRS, such relaxation provides a high degree of freedom to utilize the SL resources more flexibly. With this reason, instead of (pre-)configuring SL PRS order for double-sided SL RTT, we suggest UE to autonomously select SL PRS resources and order, and report the resultant order of SL PRS transmission in the measurement report. In this regard, we’re ok with the first bullet as SLPP as a higher layer signaling can decide the positioning method between single- and double-sided. We suggest to modify the second bullet as explained above. #### [ROUND1][High] FL proposal 3.1.2-v1: - Support to use higher layer signaling (SLPP) to determine distinguish single-sided RTT and double-sided RTT. - Support to use measurement report SCI accompanying SL-PRS to distinguish the different SL-PRS transmissions type and order in double-sided RTT without order restriction. |
| Samsung | As we explained our Tdoc (Proposal 1 in R1-2303134), the different SL-PRS transmission order in double-sided RTT (let’s call this as method 2) has better advantage compared with the original double-sided RTT. If we consider only method 2 together with single-sided RTT (let’s call this as method 1), we do not need to introduce explicit signalling for distinguishing these RTT methods. So, we suggest to discuss at first about whether to support both legacy double-sided RTT and different SL-PRS transmission order or only one of them. |
| Intel | We are fine with the directions of the agreement. |
| Lenovo | We are also not sure if the definition of Supportive of the first bullet. Double-sided RTT still holds true if the transmission order is not restricted. The bullets seem to contradict each other as the first bullet proposes higher-layer signalling while the 2nd bullet proposes to use lower layer signalling. Clarification may be needed. |
| ZTE | We are ok with this proposal. |
| Xiaomi | We are fine with the proposal. |
| Huawei, HiSilicon | It is not clear to us why we need higher layer signalling or SCI to distinguish single-sided RTT and double-sided RTT. In fact even for single sided RTT, there could be multiple rounds of RTT procedure. We do not think there should even be a clear boundary between single-sided RTT and double-sided RTT. |
| vivo | We support the first bullet for distinguishing single-sided and double-sided, based on my understanding, the second bullet is for distinguishing the transmission of SL-PRS |
| Apple | We are not clear why there needs to be a mix of higher layer and lower layer signaling for this. |
| Fraunhofer | Support the first bullet |
| SONY | We are OK with the proposal |
| Nokia, NSB | OK if the higher layer signling in first bullet allows a UE to indicate another UE on the RTT method selection, e.g. switching from one SL RTT method to another. Regarding second bullet, changes needed on top of the specification support for single-sided RTT is to be minimized (as agreed in RAN1#111). |

[MEDIUM]Support of report-free SL-RTT

The following proposals are identified to be related to this issue.

| R1-2302294 Nokia | Proposal 8: Introduce functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement. Proposal 9: Investigate possibility of having multiple fixed Rx-Tx time values for functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement. |
| --- | --- |
| R1-2302378 Huawei, HiSilicon | Proposal 3: Support report-free RTT based on adjusting the transmission time of SL-PRS. |
| R1-2302852 Sony | Proposal 3: Support report-free RTT measurement with preconfigured Rx-Tx time. |
| R1-2303415 Fraunhofer | With regards to RTT methods, SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement shall be supported: The mechanism shall not have any impact on the OFDM symbol timing requirements |

Based on the proposals identified above, the following FL proposal is provided.

#### [ROUND1][Meduim] FL proposal 3.1.3-v1:

For SL-PRS based Rx-Tx measurement, support the functionality of SL Rx-Tx measurement not being reported but the SL-PRS transmission time being adjusted based on the measurement.

Companies are invited for comments in the following table.

Table 3.1.3-v1 Collection of views on FL proposal 3.1.3-v1

| Company | Views on FL proposal 3.1.3-v1 |
| --- | --- |
| CMCC | Low priority. |
| OPPO | Support the proposal. |
| Qualcomm | We do not support the proposal |
| Futurewei | Low priority |
| DCM | Not support. |
| Intel | This solution does only omits a single report and the relation to the transmission timing requirement from RAN4 are unclear. Therefore, this is a minor optimization at best and in our view, should be deprioritized. |
| ZTE | Do not support this proposal. Just for clarification, the adjusted SL-PRS transmission can only be used for unicast link between two UEs. |
| Xiaomi | We do not support the proposal. The exsisting SL TX timing shall be reused to reduce unnecessary additional UE complexity. |
| Huawei, HiSilicon | Support. Support report-free RTT contributes to reducing positioning latency and signaling overhead. |
| Apple | Lower priority |
| Fraunhofer | Support. - In terms of the report-free approach, it only excludes a single report, so if M*RSUs are performing ranging with N*vehicles, it would result in M*N fewer reports. - As for the impact on RAN4, there should not be any concerns, and we support clarifiyinf it in the proposal. You can refer to our contribution for operational details as an example. @Xiaomi/Intel: could you please specify where you still require a change in SL TX timing? - @ZTE: Based on our understanding, the adjusted SL-PRS transmission can be used for multiple UEs when the reply time can be separated in time, similar to the concept of using different cyclic shifts for different antenna ports or UEs. |
| SONY | Support. With report-free RTT measurement, UE can preconfigure the Rx-Tx time difference such that the signalling overhead can be reduced. |
| Nokia, NSB | OK |

[MEDIUM]Measurement procedures

The following proposals are identified for this issue.

| R1-2303264 Lenovo | Proposal 11: Support Mult-UE SL-RTT within a SL Positioning Group, where both one-to-many (groupcast) and many-to-one (unicast) SL-PRS transmissions are enabled. |
| --- | --- |
| R1-2303551 Ericsson | For SL multi-RTT in NR Rel-18, support group-based SL multi-RTT where each UE in the group only transmits one single SL PRS and computes the SL UE Rx-Tx measurement corresponding to each other UE in the group. |

#### [ROUND1][Medium] FL proposal 3.1.4-v1:

Support group-based SL multi-RTT where each UE in the group only transmits one single SL PRS and computes the SL UE Rx-Tx measurement corresponding to each other UE in the group.

Companies are invited for comments in the following table.

Table 3.1.4 Collection of views on FL proposal 3.1.4-v1

| Company | Views on FL proposal 3.1.4-v1 |
| --- | --- |
| CMCC | To our understanding, the RTT-type solutions using SL includes both multi-RTT which can obtain the absolute location of a UE and also relative positioning/ranging between a pair of UEs. From the perspective of SL PRS measurements, I don’t see the difference between the two kinds of RTT solutions; on the other hand, we have agreed to specify UE behaviors to support unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions. In this sense, we think that such proposal is not needed. |
| Qualcomm | We support the proposal since it would significantly improve resource utilization efficiency. |
| Futurewei | We do not see necessary that everyone in the group computes measurements to everyone else. Therefore the proposal not needed. |
| Intel | This solution does only save a single report and the relation to the transmission timing requirement from RAN4 are unclear. Therefore, this is a minor optimization at best and in our view, should be deprioritized. |
| ZTE | We share similar understanding with CMCC. This proposal is not needed and what is the spec impact if agreed? |
| Xiaomi | We do not support the proposal. The proposed solution can be realized by implementation without specification impact. |
| Fraunhofer | Support |
| Nokia, NSB | Support. In case of a group of size N, pairwise RTT in unicast would require N*(N-1)/2 pairwise RTT procedures, using a group-based approach will be more efficient. |

[MEDIUM]Measurement window

The following proposals are identified for this issue.

| R1-2303277 ZTE | Proposal 6: With regards to UE Rx – Tx time difference for SL positioning, - For double-sided RTT, a single UE needs to report at least two Rx-Tx time difference and those two Rx-Tx time difference measurements should be within a certain timing window. |
| --- | --- |
| R1-2302852 Sony | Proposal 1: In case of SS-SL-RTT, introduce a time window between the first and second SL-PRS transmission in order to minimize the impact of the time drift. |

#### [ROUND1][Medium] FL proposal 3.1.5-v1:

With regards to UE Rx – Tx time difference for SL positioning,

- For double-sided RTT, a single UE needs to report at least two Rx-Tx time difference and those two Rx-Tx time difference measurements should be within a certain timing window.

Companies are invited for comments in the following table.

Table 3.1.5 Collection of views on FL proposal 3.1.5-v1

| Company | Views on FL proposal 3.1.5-v1 |
| --- | --- |
| CMCC | We admit that the two Rx-Tx time difference measurements should not be far away to avoid large timing drift and ensure the positioning accuracy, but there may have other solutions, for example, to further enhance the resource selection procedure, for example, the determination of resource selection windows in PHY layer, or the resource selection in MAC layer, which may need to coordinate with the progress in 9.5.3. |
| Futurewei | OK supporting timing window. |
| Intel | We understand the motivation behind having the different transmissions belonging to double side RTT not spaced too far about. However, to us it is unclear which concrete steps would the current proposal apply and in which signalling this is contained. Thus, we would like to discuss concrete solutions to achieve this. For example via assistance information exchanged before the RTT exchange. |
| ZTE | The “timing window” mentioned in our proposal means the time duration or a latency boundary for a DS-RTT procedure. Compared to SS-RTT, DS-RTT takes more time to complete the positioning procedure and thus resulting in higher probability of performance degradation caused by UE mobility or timing changes. |
| Apple | Do not see a need to have the window. In the paper “An Alternative Double-Sided Two-Way Ranging Method” by Dries Neirynck et. al., it is shown that the double-sided RTT eliminates the need for the transmissions to be close together based on error due to clock drift. Unless the goal is to make sure that there is no change in any of the UE positions. |

[MEDIUM] Indication of synchronization related information

The following proposals are identified to be related to this issue.

| R1-2302294 Nokia | Proposal 6: In SL RTT, UE reports expected error for Rx-Tx measurement, SyncRef change status and/or expected clock error to the other UE. Proposal 7: In SL RTT, UE indicates to the other UE the need for switching from one SL RTT method to another. |
| --- | --- |
| R1-2302327 FUTUREWEI | Proposal 2: Support in the Rx-Tx measurement report the indication of source of synchronization. |

Based on FL’s understanding, this issue needs to be resolved.

#### [ROUND1][MEDIUM] FL proposal 3.1.6-v1:

For SL-PRS based Rx-Tx measurement, support to indicate synchronization source related information in measurement report. Further study which information is indicated.

Companies are invited for comments in the following table.

Table 3.1.6 Collection of views on FL proposal 3.1.6-v1

| Company | Views on FL proposal 3.1.6-v1 |
| --- | --- |
| Futurewei | Support |
| Intel | In our understanding synchronization source related information is not necessary to be exchanged for RTT exchanges, as the positioning method itself is indpedenent of synchronization offsets. |
| Xiaomi | For SS-RTT and DS-RTT, imperfect synchronization may not be a big issue, and thus we do not think the proposal is necessary. |
| Huawei, HiSilicon | We suggest to delect “in the measurement report” for now. |
| Nokia, NSB | OK. In addition, quality of Rx-Tx measurement in terms of e.g. expected error range can be included in the measurement report. |

Others

Companies are encouraged to provide inputs in the following table for whether there is any other missing issues with high priority for SL-PRS based Rx-Tx measurement

Table 3.1.7 Collection of views on any other missing issues with high priority

| Company | Views on any other missing issues with high priority |
| --- | --- |

SL-PRS based RSTD and RTOA measurement

The following agreements were achieved in RAN1 #112

Agreement

SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where

- T0 is the nominal beginning time of SFN 0 or DFN0.

  - FFS on how to select between SFN 0 or DFN 0 for determination of T0.

  - FFS: the source for the reference timing

- tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

  - FFS on how to select between SFN or DFN

[HIGH]The definition of SL-PRS based RSTD

The definition of DL reference signal time difference (DL RSTD) is shown in the following for reference

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

The following proposals are identified for this issue.

| R1-2302554 OPPO | - The definition of SL RSTD can be modified based on DL RSTD definition which is as follows: Definition; Sidelink reference signal time difference (SL RSTD) is the SL relative timing difference between an anchor UE j and another anchor UE i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the target UE receives the start of one subframe from anchor UE j. TSubframeRxi is the time when the target UE receives the corresponding start of one subframe from anchor UE i that is closest in time to the subframe received from anchor UE j. Multiple SL PRS resources can be used to determine the start of one subframe from an anchor UE. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the SL RSTD shall be the antenna of the UE.. Applicable for; RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. |
| --- | --- |
| R1-2302802 Intel Corporation | Table 1 SL reference signal time difference definition. Definition; SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, Where: TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. Multiple SL PRS resources can be used to determine the start of one subframe from a UE. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE.. Applicable for; RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. - Adopt the definition for SL reference signal time difference in Table 1 |
| R1-2303134 Samsung | Proposal 5: SL reference signal time difference (SL RSTD) is defined as the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, Where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. - The reference UE i is determined by the UE performing the RSTD measurement and the reference UE i is selected with the highest SL PRS-RSRP. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. |
| R1-2303240 CMCC | Proposal 2: SL PRS-based RSTD measurement is the SL relative timing difference between UE j and the reference UE i. |
| R1-2303264 Lenovo | Proposal 4: SL RSTD may be defined as SL relative timing difference between the UE (node) j and the reference UE (node) i, defined as TSubframeRx,j – TSubframeRx,i, where: TSubframeRx,j is the time when the UE receives the start of one subframe from UE (node) j and TSubframeRx,i is the time when the UE receives the corresponding start of one subframe from UE (node) i that is closest in time to the subframe received from UE (node) j. |
| R1-2303277 ZTE | Proposal 3: SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, - Where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. - For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. |
| R1-2303307 CEWiT | Proposal 2: SL reference signal time difference (SL RSTD) is defined as the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, - Whereas, - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. - UE i is reference UE configured. - For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][HIGH] FL proposal 3.2.1-v1:

SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi,

- Where:

  - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

  - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

Companies are encouraged to share views in the following table.

Table 3.2.1Collection of views on FL proposal 3.2.1-v1

| Company | Views on FL proposal 3.2.1-v1 |
| --- | --- |
| CATT | Support. |
| CMCC | Support. |
| OPPO | Support. |
| Qualcomm | Support |
| Futurewei | Same comment as for Rx-Tx. What happens when the synchronization changes between the two instances? |
| DCM | Support |
| LGE | We support in general, but with one comment. The issue of the synchronization timing change in Rx-Tx time difference measurement should be valid also for SL RSTD measurement. The impact of UE reference timing offset and mobility should be minimized if the RX UE sync. timing changes between TSubframe_SL-Rxj and TSubframeSL-Rxi. Based on the observation above, we suggest the following modification. #### [ROUND1][HIGH] FL proposal 3.2.1-v1: SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, - Where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j, where TSubframeSL-Rxi is measured based on the synchronization timing that was used for measuring TSubframe_SL-Rxj. |
| Samsung | Unlike DL RSTD, we need to discuss about how to select the reference UE in SL RSTD. In this regard, we suggest to add the following The reference UE i is determined by the UE performing the RSTD measurement and the reference UE i is selected with the highest SL PRS-RSRP. |
| Intel | Support. |
| Lenovo | Support |
| ZTE | Support |
| Xiaomi | OK |
| vivo | Support |
| Apple | Support |
| SONY | OK |
| Nokia, NSB | OK |

[HIGH]Indication of synchronization related information for RSTD and RTOA

The following proposals are identified for this issue.

| R1-2302294 Nokia | Proposal 2: Anchor UEs can report synchronization information incl. synchronization source, co-synchronized other anchor UEs, synchronization with respect to other anchor UEs or its own synchronization source. |
| --- | --- |
| R1-2302378 Huawei, HiSilicon | Proposal 4: For alternatives to mitigate impact of synchronization errors, support Alt1 and Alt2. - For Alt2, send an LS to RAN2 to inform them that SL positioning measurement procedure can also be applicable between anchor UEs. |
| R1-2302491 vivo | Proposal 1: - To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, no physical layer synchronization procedures are introduced in Rel-18. - To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, the following information enhancement can be supported - Exchange of SFN/DFN Initialisation Time of anchor UEs between anchor UE and target/server UE/ LMF. - Exchange of synchronization source of anchor UEs between anchor UE and target/server UE/ LMF - Exchange of synchronization error (e.g., RTD) of anchor UEs |
| R1-2302606 Spreadtrum Communications | Proposal 2: Both Option 1and Option 2 can be supported to mitigate impact of synchronization errors between anchor UEs for RTOA measurement. |
| R1-2303134 Samsung | Proposal 6: For SL-TDOA, the clock source’s accuracy should be taken into account to mitigate impact of synchronization error between multiple anchor UEs. - FFS: (pre-)configuration of the clock source’s accuracy requirement/threshold or signalling of UE’s clock source accuracy status - FFS: How to select anchor UEs to mitigate impact of synchronization error |
| R1-2303240 CMCC | Proposal 7: The following can be considered to mitigate the impact of synchronization errors between the anchor UEs for SL TDOA measurements: - Alt. 1: Indication of synchronization source (e.g., GNSS), and timing adjustment information for in-coverage anchor UEs. - Alt. 2: SL PRS transmission between anchor UEs to estimate and compensate the synchronization errors for out-of-coverage anchor UEs. |
| R1-2303264 Lenovo | Proposal 6: RAN1 to support synchronization error compensation based on SL synchronisation source selection. FFS the details for synchronization error compensation e.g., sharing of RTD information between anchors and target-UE, SL synchronisation source selection, etc. Proposal 7: RAN1 to support the provision of SL RSTD and SL RTOA expected search window information for anchor UEs or target-UE. |
| R1-2303277 ZTE | Proposal 1: With regards to mitigate impact of synchronization errors between anchor UEs for SL-TDOA method, support reporting synchronization difference between reference UE and a list of anchor UEs. - The quality of the synchronization difference can also be reported. - Note: Clock source’s accuracy can be one of the factors that determine quality of the synchronization information. |
| R1-2303307 CEWiT | Proposal 3: For synchronization issues in RSTD and RTOA, support indication of source of synchronization like GNSS, cell or synchRefUE ID. This indication will be provided as capability information by the anchor UEs. Proposal 4: For synchronization error estimation and calibration in RSTD and RTOA, the Anchor UEs should be allowed to receive the SL-PRS from each other. Anchor UEs can report the sync offset wrt to another anchor. |
| R1-2303444 InterDigital, Inc. | Proposal 4: Support the UEs in a positioning group synchronize with a common SyncRef UE in the same group where the group can be formed to perform SL TDOA-based positioning. |
| R1-2303489 Apple | Proposal 9: - For timing-based positioning, the impact of synchronization errors between the anchor UEs should be studied to ensure accuracy of the estimated position. To mitigate the impact of synchronization errors RAN1 should study and adopt one or more of the following: - Enable a procedure to identify anchor UEs with a common synchronization reference. - Enable a procedure to allow for a change of anchor UEs to a common synchronization reference - Enable a procedure to estimate the target UE position in the presence of anchor UEs with different synchronization references and compensate for the difference. |
| R1-2303596 Qualcomm | Proposal 13: Methods to mitigate impact of synchronization error between multiple anchor UEs are not specified and are left up to deployment. FFS whether a synchronization quality metric is reported. |

Based on discussion from previous meetings and Tdocs submitted in this meeting, the following FL proposal is provided for company to start from the minimum.

#### [ROUND1][High] FL proposal 3.2.2-v1:

Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement

- Exchange of synchronization information of anchor UEs between anchor UE and target/server UE/ LMF

  - FFS detailed synchronization information. E.g: synchronization source, synchronization error, [SFN/DFN Initialisation Time]

- FFS other mechanisms

Companies are encouraged to share views in the following table.

Table 3.2.2 Collection of views on FL proposal 3.2.2-v1

| Company | Views on FL proposal 3.2.2-v1 |
| --- | --- |
| CATT | OK with the proposal. |
| CMCC | We are generally fine with the direction and to further study details. |
| OPPO | The sync. error among different anchor UEs are not specified and can be left to deployment. |
| Qualcomm | We are generally ok with the proposal, but would like to clarify that synchronization procedure is not changed: - Exchange of synchronization information of anchor UEs between anchor UE and target/server UE/ LMF - FFS detailed synchronization information. E.g: synchronization source, synchronization error, [SFN/DFN Initialisation Time] - No changes are introduced to sidelink synchronization procedure |
| Futurewei | OK |
| DCM | Not support. Sync error should be handled by implementation. If sync error is large, just RTT-based mechanism should be used instead. |
| LGE | We don’t support the first bullet. Even though the synchronization source/error of each anchor UE is exchanged each other, the timing offset caused by the different distance between anchor UE and SyncRef cannot be removed. The measurement report of round-trip-time between anchor UE and target anchor UE, and Rx-Tx time difference from anchor UE enable the cancellation of such timing offset. This is nothing but the required operation for Alt 2. The details on how RSTD can be measured regardless of the timing offset can be found in R1-2302928. We suggest the following proposal. #### [ROUND1][High] FL proposal 3.2.2-v1: Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement - Alt 2. SL PRS transmission/reception between anchor UEs - Report round-trip-time between anchor UE and target anchor UE - Report Rx-Tx time difference from the anchor UE |
| Samsung | The following was discussed in the last meeting. - Alt1: indication/reporting of synchronization source, synchronization error or quality of synchronization information We suggest to add ‘or quality of synchronization information in the example above since this can be a good alternative for synchronization information. |
| Intel | We can accept the proposal, but prefer to clarify further that this proposal does not intend to enhance synchronization procedures and is only limited to signalling of assistance information regarding synchronization quality. |
| Lenovo | Supportive of FL’s proposal |
| ZTE | Generally ok. We are fine with the change made by Qualcomm, but that part should be a note. |
| Xiaomi | We are fine with the proposal. |
| Huawei, HiSilicon | We suggest to also consider SL-PRS transmission between anchor Ues when anchor Ue’s synchronization performance does not satisfy the positioning requirement. |
| vivo | Support |
| Apple | Support |
| Nokia, NSB | OK |
| InterDigital | We should clarify under what condition the synchronization information is exchanged. If our understanding is correct, synchrozniation information is circulated among UEs who are participating in a SL-TDoA session and such mechanism should be clarified. Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement - Exchange of synchronization information of anchor UEs between anchor UEs and target/server UE/ LMF who are participating in SL-TDoA positioning - FFS detailed synchronization information. E.g: synchronization source, synchronization error, [SFN/DFN Initialisation Time], whether syncRef is one of the UEs participating in SL-TDoA positioning - FFS other mechanisms |

[HIGH]SFN v.s. DFN

The following proposals are identified for this issue.

| R1-2302378 Huawei, HiSilicon | - Proposal 6: Support the indication of whether SFN or DFN is used along with the RTOA measurement reporting. |
| --- | --- |
| R1-2302491 vivo | - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time if serving cell timing reference is in use, or DFN0 provided by DFN Initialization Time otherwise. |
| R1-2302709 CATT | Proposal 7: For SL RTOA measurement, whether T0 is the nominal beginning time of SFN0 or DFN0 can be recommended by the network side. But, it is up to anchor UE to select SFN 0 or DFN 0 for T0. |
| R1-2302554 OPPO | - Depending on the synchronization reference selected by the target UE and anchor UE, DFN or SFN is used to determine T0 and tSL-PRS. |
| R1-2302802 Intel | - When the UE uses the network as a synchronization source the SFN can be used for SL-PRS based RTOA. - When the UE uses SL synchronization or an external synchronization source like GNSS the DFN is used for the definition of the SL-PRS based RTOA. |
| R1-2302927 LG Electronics | Proposal 1: It is supported that at least when the anchor UEs are directly synchronized to gNB/eNB, SL RTOA T0+tSL-PRS is determined as follows. - T0 is the nominal beginning time of SFN0 - TA/2, where TA is the timing advance for UL transmission. - tSL-PRS = (10nf + nsf) x 10-3tSRS=10nf+nsf×10-3, where nf and nsfnsf are the SFN and the subframe number of the SL-PRS respectively, which are determined based on SFN timing - TA/2. - Applying TA for RTOA timing reference determination is (pre-)configured. Proposal 2: At least when anchor UEs are directly synchronized to GNSS, SL RTOA T0+tSL-PRS is determined as follows. - T0 is the nominal beginning time of DFN0. - tSL-PRS = (10nf + nsf) x 10-3tSRS=10nf+nsf×10-3, where nf and nsfnsf are the DFN and the subframe number of the SL-PRS respectively. |
| R1-2303134 Samsung | Proposal 4: For SL positioning, the synchronization reference source becomes the source for the reference timing and DFN is used when a UE selects GNSS as the synchronization reference source. |
| R1-2303240 CMCC | Proposal 6: Reuse the sidelink mechanism of UE selecting synchronization reference to select between SFN 0 or DFN 0 for determination of T0. |
| R1-2303264 Lenovo | Discussion on SL Positioning Measurement and Reporting Proposal 2: Support flexible selection of reference timing for deriving the SL RTOA measurements depending on the scenario, e.g., SFN 0 for all in-coverage group of UEs or DFN 0 for all out-of-coverage group of UEs. |
| R1-2303277 ZTE | Proposal 2: SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - tSL PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively |
| R1-2303307 CEWiT | Proposal 1: For SL-PRS based RTOA TSL-RTOA, T0 in the SL RTOA reference time is the nominal beginning time of DFN0 derived from the synchronization reference. |
| R1-2303551 Ericsson | - For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission. |
| R1-2303596 Qualcomm | Proposal 9: DFN is used for RTOA calculation when the UE would use DFN for sidelink communications. Proposal 10: SFN is used for RTOA calculation when the UE would use SFN for sidelink communications. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1] [High] FL proposal 3.2.3-v1:

- For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission.

  - When the UE uses the network as a synchronization source, SFN0/SFN is used for SL-PRS based RTOA.

  - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA.

- FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting

Companies are encouraged to share views in the following table.

Table 3.2.3 Collection of views on FL proposal 3.2.3-v1

| Company | Views on FL proposal 3.2.3-v1 |
| --- | --- |
| CATT | Support the proposal in principle. We prefer the following updated version: #### Updataed [ROUND1] [High] FL proposal 3.2.3-v1: - For SL-PRS based RTOA definition, the selection between SFN0/SFN vs DFN0/DFN and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission. - When the UE directly or indirectly synchronized to gNB/eNB uses the network as a synchronization source, SFN0/SFN is used for SL-PRS based RTOA. - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA. - FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting |
| CMCC | Support. |
| OPPO | We are OK with FL’s proposal. |
| Qualcomm | Support |
| Futurewei | OK |
| DCM | Can it be assumed that sync source is the same when SL UEs can communicate each other? |
| LGE | We support FL proposal with one comment. Even though anchor UEs are synchronized to same gNB, the exact timings of SFN0/SFN are different from different anchor UEs, which can cause the RTOA measurement error. This is because the distance between anchor UE and gNB is dfferent from each other. Such timing offset between anchor UEs are easily compensated by applying timing advance(TA) used for UL transmission. We suggest the following addition. #### [ROUND1] [High] FL proposal 3.2.3-v1: - For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission. - When the UE uses the network as a synchronization source, SFN0/SFN is used for SL-PRS based RTOA. The timing of SFN0/SFN for RTOA reference timing is determined by advancing DL SFN0/SFN with TA/2. - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA. - FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting |
| Samsung | In SL, a UE can select GNSS as the synchronization source even though the UE is in the network coverage. In order to follow the same principle with existing SL procedure, we suggest the proposal as: - For SL positioning, the synchronization reference source becomes the source for the reference timing and DFN is used when a UE selects GNSS as the synchronization reference source. |
| Intel | Support. |
| Lenovo | Support FL’s proposal |
| ZTE | Support. Withe regards to the last bullet, we do no think whether SFN or DFN should be reported along with RTOA measurements explicitly. Time stamp can imply such information. |
| Xiaomi | We are fine with the proposal. |
| Huawei, HiSilicon | Support. Indication of whether SFN or DFN is used is required. |
| Apple | OK |
| Nokia, NSB | OK |

[MEDIUM]Reference UE determination for RSTD and RTOA

The following proposals are identified for this issue.

| R1-2302709 CATT | Proposal 8: For SL RSTD measurement, reference UE can be recommended by the network. But, it is up to Rx (target) UE to decide which Tx UE is used as the reference UE. |
| --- | --- |
| R1-2303240 CMCC | Proposal 3: For sidelink positioning, whether and how the reference anchor UE/SL PRS resources is (pre-)configured should be discussed. Proposal 4: The UE can determine the reference anchor UE and/or SL PRS resource by itself and carry the corresponding information in the measurement reporting. |  |
| R1-2303264 Lenovo | Proposal 5: Selection of a reference UEs for performing SL RSTD measurements may use the same procedures used to select anchor UEs. FFS reference UE selection criteria. RAN2 coordination may also be required. |  |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][Medium] FL proposal 3.2.4-v1:

For SL-PRS based RSTD measurement and measurement report, the reference UE can be determined by the Rx UE, and the reference UE information will be included in measurement reporting.

- FFS on necessity of preconfiguraion/recommendation of reference UE information.

Companies are encouraged to share views in the following table.

Table 3.2.4 Collection of views on FL proposal 3.2.4-v1

| Company | Views on FL proposal 3.2.4-v1 |
| --- | --- |
| CATT | Support. |
| CMCC | Support. |
| OPPO | Support. |
| Qualcomm | Support |
| Futurewei | OK. Please see our comment for RSTD measurement. |
| Intel | Is this proposal in the RAN1 scope? We are unsure if this would collide with agreements in other WGs. In our understanding discussions on anchor node selection at the moment take place in RAN2. |
| ZTE | Support |
| Xiaomi | We also think how to determine the reference UE shall be up to RAN2. |
| Apple | OK |
| Nokia, NSB | OK |
| InterDigital | I’m not sure if we have an agreement on how the reference UE (assuming this is the UE based on which RSTD is computed) is determined for TDOA-like positioning methods. The entity which determines the reference UE may be different depending on in/out/partial coverage. Thus, we suggest the following: Modified proposal: For SL-PRS based RSTD measurement and measurement report, the reference UE can be determined by the Rx UE, and the reference UE information will be is included in measurement reporting report FFS : Reference UE determination |

[HIGH]Source Timing for RTOA

| R1-2302802 Intel | For the definition of SL-PRS based RTOA the reference timing is a UE reference timing defined for positioning that may be provided by server UE or by LMF when in coverage. |
| --- | --- |
| R1-2303264 Lenovo | Proposal 3: Sources of the reference timing for SL RTOA measurements may support network-based timing, e.g., gNB, GNSS-based timing and UE-based timing. FFS prioritization of reference timing sources. |
| R1-2303134 Samsung | Proposal 4: For SL positioning, the synchronization reference source becomes the source for the reference timing and DFN is used when a UE selects GNSS as the synchronization reference source. |

#### [ROUND1][HIGH] FL proposal 3.2.5-v1:

- SFN/DFN Initialisation Time can be provided by server UE or by LMF.

- For the definition of SL-PRS based RTOA,  update the definition of reference timing as:

- the reference timing is defined as T0+tSL-PRS, where

  - T0 is the nominal beginning time of SFN 0 or DFN0 provided by SFN/DFN Initialisation Time

    - FFS on how to select between SFN 0 or DFN 0 for determination of T0.

    - FFS: the source for the reference timing

  - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

    - FFS on how to select between SFN or DFN

Table 3.2.5 Collection of views on FL proposal 3.2.5-v1

| Company | Views on FL proposal 3.2.5-v1 |
| --- | --- |
| CATT | It seems that “server UE” has not been defined in RAN1. So it may be unclear for the “server UE” in the first bullet. Maybe a note on what is the server UE is needed. |
| OPPO | OK |
| Qualcomm | We are ok with the proposal, after replacing “server UE” with “UE”. This would also address CATT’s concern. |
| Futurewei | OK with Qualcomm comment |
| Samsung | It is not clear how server UE or LMF can provide the reference time properly. In addition, this method is feasible only in network coverage. So, we prefer that the synchronization reference source becomes the source for the reference timing. |
| Intel | Agree. |
| Lenovo | Agree that for purposes of RAN1 discussion “server UE” has not been defined and can support Qualcomm’s edit. |
| ZTE | OK |
| Xiaomi | Fine with the proposal. |
| vivo | OK with Qualcomm modification |
| Apple | OK |
| Nokia, NSB | OK with Qualcomm’s modification |

Others

Companies are encouraged to provide inputs in the following table for whether there is any other missing issues with high priority for SL-PRS based RSTD measurement and RTOA measurement.

Table 3.2.6 Collection of views on any other missing issues with high priority

| Company | Views on any other missing issues with high priority |
| --- | --- |
| R1-2302294 Nokia | Proposal 3: For the UE which can cancel time offset between different receive antennas, support time difference measurement between different receive antennas with respect to a SL PRS. |
| R1-2303240 CMCC | Proposal 5: RSTD search window is not required to be configured/provided to the UE. |
| R1-2303264 Lenovo | Discussion on SL Positioning Measurement and Reporting Proposal 1: RAN1 to support SL-RSTD and SL-RTOA physical layer measurements in any respective order within a single SL positioning session. Proposal 7: RAN1 to support the provision of SL RSTD and SL RTOA expected search window information for anchor UEs or target-UE. |

SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

The following agreements were achieved in RAN1 #112.

Agreement

Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement.

- FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

[HIGH] LCS & GCS, Azimuth and zenith of arrival

The following proposals are identified to be related to LCS & GCS, Azimuth and zenith

| R1-2302378 Huawei, HiSilicon | Proposal 7: Support AoA/ZoA relative to LCS without translation of the LCS to GCS. |
| --- | --- |
| R1-2302491 vivo | Discussion on measurements and reporting for SL positioning - The target UE can report the AoA/ZoA relative to LCS directly without translation of the LCS to GCS to LMF for Ranging/relative positioning service exposure through 5GC network |
| R1-2302709 CATT | Further discussion on measurements and reporting for SL positioning Observation 1: To use the SL AoA/ZoA measurements obtained by the anchor UEs for absolute positioning, the LCS to GCS translation matrixes need to be known for the anchor UEs. Observation 2: To use the SL AoA/ZoA measurements obtained by the target UE for absolute positioning, there is no need to know the LCS to GCS translation matrixes for any of the (target and anchor) UEs. |
| R1-2303240 CMCC | Proposal 10: At least relative positioning/ranging, in case that target UE performs SL PRS-based AoA/ZoA measurement, it is feasible that SL PRS-based AoA/ZoA relative to LCS without translation of the LCS to GCS. |
| R1-2303264 Lenovo | Discussion on SL Positioning Measurement and Reporting Proposal 12: For direct ranging for relative direction/orientation estimation with respect to another UE/node computed at the target-UE, SL-AoA and SL-ZoAs in LCS may be sufficient. Proposal 13: RAN1 to discuss the location of the LCS-to-GCS translation function, e.g., at the target-UE or anchor UE, which affect the signalling of the 3 rotation angles α, β and γ. |
| R1-2303489 Apple | On Measurements and reporting for SL positioning - Proposal 6-2: Anchor UE orientation: Given that the orientation of the positioning set at the time of measurement is important (e.g. a rotation occurs either during or after the transmission), the orientation of the measuring UE as well as a time stamp of when this measurement was made should be part of the measurement report. |
| R1-2303277 ZTE | Proposal 7: AoA/ZoA relative to LCS without translating to GCS is applicable at least in ranging and relative positioning cases. |
| R1-2303551 Ericsson | Measurements and reporting for SL positioning Support LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement both with and without translation of the LCS to GCS. |
| R1-2303596 Qualcomm | RAN1 to study reporting parameters needed for converting LCS to GCS, if conversion is necessary and when the required parameters are available. |

The following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 3.3.1-v1:

Support both the case with and without translation of the LCS to GCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement.

Companies are encouraged to share views in the following table.

Table 3.3.1 Collection of views on FL proposal 3.3.1-v1

| Company | Views on FL proposal 3.3.1-v1 |
| --- | --- |
| CATT | Support. |
| CMCC | Support. |
| OPPO | We support the case without translation of LCS to GCS for SL-PRS based AoA and ZoA measurement. |
| Qualcomm | Support |
| Futurewei | OK |
| Samsung | OK |
| Intel | Support. |
| Lenovo | Support |
| ZTE | Support |
| Xiaomi | Fine with the proposal. |
| Huawei, HiSilicon | Support. |
| vivo | Support |
| Apple | Support |
| Nokia, NSB | OK |

[HIGH]Definition for AoA and ZoA measurement

The following proposals are identified to be related to LCS & GCS, Azimuth and zenith

| R1-2302491 vivo | - SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. - The SL AoA is determined at measurement UE antenna for a SL channel corresponding to this UE. |
| --- | --- |
| R1-2302554 OPPO | Discussion on measurements and reporting for SL positioning - The definition of SL AoA can be modified based on UL AoA definition which is as follows: SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at a UE antenna for an SL channel corresponding to a target UE or anchor UE |
| R1-2302802 Intel | Measurements and reporting for SL positioning SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at the receiving UE’s antenna for an SL channel corresponding to the transmitting UE. |
| R1-2303134 Samsung | On Measurements and Reporting for SL Positioning Proposal 3: For SL AOA, both LCS and GCS are supported and the measurement is defined as: - SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The x, y and z axis form a right-handed co-ordinate system. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. - The SL AoA is determined at the UE antenna for an SL channel corresponding to a peer UE communicating each other. |
| R1-2303240 CMCC | Proposal 9: SL PRS-based AoA is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction. |
| R1-2303307 CEWiT | Proposal 7: For SL-AoA method, the definition of GCS and LCS will be as below: - In the global coordinate system (GCS), azimuth angle is measured relative to geographical North and is positive in a counterclockwise direction and vertical angle is measured relative to zenith and positive to horizontal direction. - In the local coordinate system (LCS), azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. |

Based on above proposals, before the discussion on SL-AoA definition, the issues on GCS and LCS should be clarified . So, the following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 3.3.2-v1:

SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined:

-	In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

-	In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901.

The SL AoA is determined at the receiving UE’s antenna for a SL channel corresponding to the transmitting UE.

Companies are encouraged to share views in the following table.

Table 3.3.2 Collection of views on FL proposal 3.3.2-v1

| Company | Views on FL proposal 3.3.2-v1 |
| --- | --- |
| CATT | OK with the proposal. |
| CMCC | Support. |
| OPPO | OK with the proposal. |
| Qualcomm | Support with correction: The SL AoA is determined at the receiving UE’s antennas for a SL channel corresponding to the transmitting UE. |
| Futurewei | OK, we think that in the main text instead “of a UE” should be “at a UE” |
| Samsung | OK |
| Intel | Support. |
| Lenovo | Support FL’s proposal |
| ZTE | Support |
| Xiaomi | Fine with the proposal. |
| vivo | Support, for the Qualcomm modification, we would like to confirm why NR definition is antenna other than antennas. |
| Apple | OK |
| SONY | Support |
| Nokia, NSB | OK. The first instance of “UE” could be replaced by “transmitting UE” for clarity and symmetry with respect to the later “receiving UE” |

Others

Companies are encouraged to provide inputs in the following table for whether there is any other missing issues with high priority for SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement.

Table 3.3.3 Collection of views on any other missing issues with high priority

| Company | Views on any other missing issues with high priority |
| --- | --- |

SL-PRS based RSRP measurement and RSRPP measurement

The definition of SL-PRS based RSRP measurement and RSRPP measuremen were agreed in RAN1 #112.

Agreement

SL PRS reference signal received power (SL PRS-RSRP)

- is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth

With regard to the reference point

- For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE.

- For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches.

Agreement

SL PRS reference signal received path power (SL PRS-RSRPP),

- is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

With regard to the reference point

- For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE.

- For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

Based on agreed definitions, the following issues were further raised.

[ROUND1][HIGH]FR2 SL-PRS based RSRP/RSRPP measurement

The DL PRS reference signal received power (DL PRS-RSRP) is shown in the following for reference

| Definition | DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

The following proposals are identified to be related to SL-PRS based RSRP definition

| R1-2302491 vivo | Discussion on measurements and reporting for SL positioning With regard to the reference point for frequency range 2 of SL-PRS RSRP - For frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. With regard to the reference point for frequency range 2 of SL-PRS RSRPP - For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. |
| --- | --- |
| R1-2303551 Ericsson | Measurements and reporting for SL positioning - For SL-PRS based RSRP definition in FR2, SL PRS-RSPR shall be measured based on the combined signal from the antenna elements corresponding to a given receiver branch. - For SL-PRS based RSRP definition in FR2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. - For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch |
| R1-2303596 Qualcomm | Measurements and Reporting for SL Positioning Proposal 6: For SL PRS-RSRP measurement and for frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. If receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches Proposal 7 For SL PRS-RSRP measurement and for frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. If receiver diversity is in use by the UE for SL PRS-RSRPP measurements, the reported SL PRS-RSRPP value shall be provided for the same receiver branch(es) as applied for SL PRS-RSRP measurements |

#### [ROUND1][High] FL proposal 3.4.1-v1:

- For definition of SL-PRS based RSRP measurement in frequency range 2

  - SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - If receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches.

- For definition of SL-PRS based RSRPP measurement in frequency range 2

  - SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - If receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

  - For frequency range 1 and 2, if receiver diversity is in use by the UE for SL PRS-RSRPP measurements, the reported SL PRS-RSRPP value for the first and additional measurements shall be provided for the same receiver branch(es) as applied for SL PRS-RSRP measurements.

Companies are encouraged to share views in the following table.

Table 3.4.1 Collection of views on FL proposal 3.4.1-v1

| Company | Views on FL proposal 3.4.1-v1 |
| --- | --- |
| CATT | Support the proposal in principle. For the last sub-bullet, since it mentioned that “For frequency range 1 and 2”, so it had better to put it in a new third main bullet, instead of current under the second main bullet “For definition of SL-PRS based RSRPP measurement in frequency range 2”. Our preferrd version as follows, #### [ROUND1][High] FL proposal 3.4.1-v1: - For definition of SL-PRS based RSRP measurement in frequency range 2 - SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - If receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. - For definition of SL-PRS based RSRPP measurement in frequency range 2 - SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - If receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. - For frequency range 1 and 2, if receiver diversity is in use by the UE for SL PRS-RSRPP measurements, the reported SL PRS-RSRPP value for the first and additional measurements shall be provided for the same receiver branch(es) as applied for SL PRS-RSRP measurements. - For definition of SL-PRS based RSRPP measurement: - For frequency range 1 and 2, if receiver diversity is in use by the UE for SL PRS-RSRPP measurements, the reported SL PRS-RSRPP value for the first and additional measurements shall be provided for the same receiver branch(es) as applied for SL PRS-RSRP measurements. |
| OPPO | Support. As SL PRS transmission on FR2 is allowed, measurement for FR2 should be specified such that the SL PRS transmitted on FR2 can be used for positioning. |
| Qualcomm | Support |
| Futurewei | OK |
| DCM | We do not think RAN1 should spend time for FR2. |
| Samsung | In our understanding, there was no consensus in the last meeting for capturing the definition for FR2. In addition, in RAN#99 meeting, there was a related discussion as below but no consensus. In this situation, we thint that we do not need to repeat the same discussion again. RP-230143 On support of measurement(s) for FR2 in sidelink positioning OPPO Proposal: FR2 specific reference point for the measurement(s) introduced in SL positioning should be specified, to make one of following clarifications in the WID: - “NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified (This does not preclude to specify FR2 specific reference point for the measurement(s)). “ Or, - “Specify measurements for both FR1 and FR2 to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].” Replaces AT&T: we had 1080min of online time for positioning at the last RAN1 meeting so we would like to keep the WI scope as it is ZTE: agrees with AT&T that we should avoid this upscoping Huawei: no urgency to include this, we can keep the scope as it is |
| Intel | We are fine with the proposal. However, this was already discussed last meeting and dropped as at this stage it is unclear when SL will support FR2. Thus, we think it is better to invest our time into other discussions. |
| Lenovo | Support with no further FR2 optimizations. |
| ZTE | Agree with Samsung’s comment and prefer not to further discuss this proposal. |
| Xiaomi | We support the discussion on FR2 as it is not precluded in WID. Howeverr, for SL RSRPP measurement, the following subbullet is not in the definition of DL RSRPP measurement, so we suggest to remove it: - If receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. |
| Apple | OK |
| Nokia, NSB | OK |

Others

Companies are encouraged to provide inputs in the following table for whether there is any other missing issues with high priority for SL-PRS based RSRP and RSRPP measurement

Table 3.4.2 Collection of views on any other missing issues with high priority

| Company | Views on any other missing issues with high priority |
| --- | --- |

[HIGH]Per panel/ARP measurement

Based on the on above proposals, FL think per ARP/ panel  measurement is logical especially for vehical UE. So, the following FL proposal is provided for company to share their views

| R1-2302378 Huawei, HiSilicon | - Proposal 8: Support per ARP/panel based measurement/transmission in sidelink positioning, where an indication is used to identify whether - Two measurements are from the same ARP - Two transmissions are from the same ARP - Rx time and Tx time are from the same ARP for UE Rx – Tx time difference |
| --- | --- |
| R1-2302491 vivo | Discussion on measurements and reporting for SL positioning |
| R1-2302709 CATT | Further discussion on measurements and reporting for SL positioning Proposal 11: Per-panel/subpanel (ARP) measurements such as Rx-Tx time difference, AoA/ZoA, RSTD, RTOA, RSRP and RSRPP should be supported for SL positioning. |
| R1-2302927 LG Electronics | Discussion on measurements and reporting for SL positioning Proposal 20: SL positioning supports per-panel location measurement if UE uses multiple panels. |

#### [ROUND1][High] FL proposal 3.5.1-v1:

Support per ARP based measurement in sidelink positioning. The ARP related information  can be reported along with the SL measurement.

- FFS on whether/how to define ARP ID

Companies are encouraged to share views in the following table.

Table 3.5.1 Collection of views on FL proposal 3.5.1-v1

| Company | Views on FL proposal 3.5.1-v1 |
| --- | --- |
| CATT | Support. |
| Qualcomm | Support |
| Futurewei | OK |
| LGE | Support |
| Samsung | OK |
| Intel | Support. |
| Lenovo | Support |
| ZTE | We understand the potential benefit of per panel measurement. However, in Uu positioning why does a UE report its measurements to LMF without associated ARP info? |
| Xiaomi | Multiple pannels for SL has not been discussed and defined in SL communication, the discussion can be for future release. |
| Huawei, HiSilicon | Support. |
| vivo | Support, for the ZTE and Xiaomi comments, we think vehicle UE is not considered in the NR scenario, and the different panel will impact the positioning severely. |
| Apple | OK |
| Fraunhofer | Support |
| Nokia, NSB | OK |

[LOW]Combination of Uu- and PC5-based positioning

The following proposals are identified for combination of Uu and PC5 based positioning.

| R1-2302378 Huawei, HiSilicon | - Proposal 9: Not to specify any new measurements for joint Uu and SL positioning. |
| --- | --- |
| R1-2302709 CATT | Proposal 9: Support hybrid Uu+PC5 UE-based positioning, in which gNBs and anchor UEs report, respectively, the UL RTOA measurements and SL RTOA measurements, to the target UE. |
| R1-2302852 Sony | Proposal 4: Introduce a new TDOA measurement report format that support hybrid Uu-PC5 measurements for absolute positioning |
| R1-2302989 xiaomi | Proposal 2: SL-DL RSTD is defined as the measurement of the timing difference between DL PRS and SL PRS. |
| R1-2303134 Samsung | Proposal 12: Consider for a UE to measure and report the time of arrival of a SL signal relative to time of arrival of a DL reception from a gNB. |
| R1-2303240 CMCC | Proposal 1: DL-SL RSTD measurement considering time difference of a DL PRS and a SL PRS for joint Uu- and PC5-based positioning is de-prioritized. |
| R1-2303264 Lenovo | Proposal 20: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for Uu and SL hybrid positioning. FFS whether such measurements are performed in a joint manner or separately measured and how these measurements are utilised. |
| R1-2303277 ZTE | Proposal 14: Support DL-TDOA-like joint SL & Uu positioning: - For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node. |
| R1-2303489 Apple | On Measurements and reporting for SL positioning Proposal 7: SL-TDOA definition: - SL-TDoA positioning is based on the time delay of arrival to or from a SL UE target with SL UEs in a positioning set (at least 2 for 2-D positioning and at least 3 for 3-D positioning). In hybrid SL-TDOA, one or more of the positioning set may be a gNB. |
| R1-2303551 Ericsson | Measurements and reporting for SL positioning - Hybrid ranging involving gNBs and UEs should be specified, e.g., exploiting SL RToA measurements and Uu UE Rx-Tx measurements. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1] [Low] FL proposal 3.6.1-v1:

Study the necessity of supporting the following joint Uu- and PC5-based positioning

- DL-SL RSTD

- Other joint Uu- and PC5-based positioning not precluded

Companies are encouraged to share views in the following table.

Table 3.6.1 Collection of views on FL proposal 3.6.1-v1

| Company | Views on FL proposal 3.6.1-v1 |
| --- | --- |
| ZTE | Firstly, joint SL&Uu positioning is agreed to be supported. We need specify how to use both SL measurements and Uu measurements and the potential impact of combing them. Then, usually anchor UEs and TRPs are not accurately synchronized and reference nodes for SL and Uu positioning are selected separately. The relative timing difference between SL reference node and Uu reference node should be reported to LMF for joint positioning accuracy improvement. |
| Huawei, HiSilicon | Prefer not to specify any new measurements. |

[HIGH]Measurements procedures

| R1-2302378 Huawei, HiSilicon | - Proposal 14: RAN1 should send an LS to RAN2/SA2 regarding the following questions: - Whether UE A can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C - Whether LMF can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C |
| --- | --- |
| R1-2302709 CATT | Proposal 10: Support target/anchor/server UE-initiated and LMF-initiated SL-PRS measurement procedures. |
| R1-2303134 Samsung | On Measurements and Reporting for SL Positioning Proposal 10: Support request signaling to trigger SL positioning measurement reporting, and study further trigger conditions to satisfy reliability requirement of SL positioning measurement. Proposal 14: In SL positioning, a target UE decides whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s). |
| R1-2303264 Lenovo | Discussion on SL Positioning Measurement and Reporting Proposal 21: RAN1 to further discuss the support of SL PRUs in SL positioning/ranging procedures. |
| R1-2303489 Apple | On Measurements and reporting for SL positioning Proposal 4: For single/double Sided Multi-RTT for absolute positioning, a procedure to select the candidate anchor UEs should be studied. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1] [HIGH] FL proposal 3.7.1-v1:

Working Assumption:

Support target UE/server UE-initiated and LMF-initiated SL-PRS measurement procedures.

- Send LS to RAN2 on this working assumption and ask them to confirm.

Companies are encouraged to share views in the following table.

Table 3.7.1 Collection of views on FL proposal 3.7.1-v1

| Company | Views on FL proposal 3.7.1-v1 |
| --- | --- |
| CATT | Support. |
| CMCC | It should be up to RAN2. |
| OPPO | It is up to RAN2’s discussion and determination. |
| Qualcomm | We generally support the proposal, but propose to replace “target UE/server UE” with “UE”: Support target UE/server UE-initiated and LMF-initiated SL-PRS measurement procedures. - Details are up to RAN2 |
| Futurewei | OK with initial proposal. Without details would be difficult for RAN2 to understand that the initiation can start from both ends. |
| DCM | Prefer QC’s version |
| Samsung | OK |
| Intel | Support. |
| Lenovo | Depends on which entity triggers the overall SL Positining procedures. RAN2 may decide and may not be a critical issue from RAN1 perspective |
| ZTE | Even though we agree with the content of Proposal 3.7.1, we also think it is a RAN2’s issue. |
| Xiaomi | As RAN1 has not introduce the definition of server UE, we suggest to simpliy state “UE initiated” as following: Support target UE/server UE-initiated and LMF-initiated SL-PRS measurement procedures. - Send LS to RAN2 on this working assumption and ask them to confirm. |
| Huawei, HiSilicon | Not sure why it is marked as working assumption. Any RAN1 spec impact or any further discussion needed on top of it. |
| SONY | We should aim to make an agreement (instead of working assumption). Suggest to remove the working assumption words. We suggest the following: In RAN1 understanding, Support target UE/server UE-initiated and LMF-initiated SL-PRS measurement procedures are supported. - Send LS to RAN2 on this working assumption and ask them to confirm. |
| Nokia, NSB | In general OK, but it is up to RAN2 to discuss further. |
| InterDigital | We are ok to discuss this in RAN2. |

[MEDIUM] TEG support

| R1-2302327 FUTUREWEI | Proposal 7: Consider supporting Rx, Tx and RxTx TEGs for SL-PRS and reporting their values to LMF and higher layers. |
| --- | --- |
| R1-2302802 Intel | A SL TEG for UE SL Rx – Tx time measures is defined. The definition is based on the TEG definition for Uu positioning. |

#### [ROUND1] [MEDIUM] FL proposal 3.8.1-v1:

Support to introduce Rx, Tx and Rx-Tx TEGs for SL-PRS based measurement and report.

Companies are encouraged to share views in the following table.

Table 3.8.1 Collection of views on FL proposal 3.8.1-v1

| Company | Views on FL proposal 3.8.1v |
| --- | --- |
| Intel | Support. |

Others

| R1-2303775 Locaila | (Proposal 1) Introduce new terminology RTP (Return Trip Phase) to avoid any confusion with RTT when discussing the frequency domain method such as the carrier phase positioning. (Proposal 2) Study new PRS signal efficiency for phase-based sidelink positioning methods such as carrier phase positioning. (Proposal 3) RAN1 should study the feasibility of carrier phase-based positioning for application to sidelink use cases. |
| --- | --- |
| R1-2303842 MediaTek | Measurement and reporting design for sidelink positioning |

Companies are encouraged to share views in the following table on any other missing issues.

Table 3.9.1 Collection of views on any other missing issues

| Company | Views on FL proposal 3.9.1 |
| --- | --- |

Sidelink positioning Report

Report signaling

[HIGH]Higer-layer report or lower-layer report

The following proposals are identified to be related to higher layer report or lower layer report.

| R1-2302927 LG Electronics | Proposal 8: SL measurement report is transmitted by the existing higher layer signaling (e.g. PC5-RRC, PC5-S or SL MAC CE). |
| --- | --- |
| R1-2303277 ZTE | Proposal 12: Only support higher layer signaling for sidelink positioning measurement report and report triggering. |
| R1-2302378 Huawei, HiSilicon | - Proposal 10: SL positioning measurement shall be treated as regular communication data, and be transparent to physical layer and transmitted over the PC5 communication resources. |
| R1-2302606 Spreadtrum Communications | Proposal 5: SL positioning measurement report through higher layer signaling (e.g., RRC or MAC CE) should be supported. |
| R1-2302989 xiaomi | Proposal 4: higher layer signalling shall be used to convey sidelink positioning measurement report. Proposal 5: From physical layer perspective, the sidelink positioning measurement report is transmitted in the same way as UE data. |
| R1-2303064 Sharp | Proposal 1: Higher layer signalling (i.e. SLPP) is used to convey SL positioning measurement reports to another UE. Proposal 2: Adopt at least physical layer triggering (e.g. SCI) of SL positioning measurement report. Proposal 4: Priority of the SL positioning measurement report is based on priority of the measured SL-PRS. |
| R1-2303134 Samsung | Proposal 9: Discuss the feasibility of reporting SL positioning measurement by using 2nd stage SCI in RAN1. |
| R1-2302491 vivo | Discussion on measurements and reporting for SL positioning - SLPP is used to carry the SL-PRS measurement report. - SL-PRS measurement reports can be triggered by SLPP - FFS whether SCI can be used to trigger feedback SL-PRS or measurement report. |
| R1-2302709 CATT | Proposal 13: High-layer signaling based report triggering should be supported. - FFS on SCI based report triggering and lower layer report. |
| R1-2303307 CEWiT | Proposal 9: Support higher layer signaling for sidelink positioning measurement report and report triggering. Lower layer signalling in SCI should be used for resource reservation for the reporting. - Measurement report signalling should be provide by SLPP/LPP message e.g., provide location information. - Report trigger should be provided by AS layer using MAC-CE. |
| R1-2303444 InterDigital, Inc. | Proposal 16: Support SL-PRS measurement report to be transmitted in PSSCH. Proposal 17: The transmission of SL Positioning Report can be triggered through a lower layer indication, such as an indication in the SCI. |
| R1-2303596 Qualcomm | Proposal 2: Measurement report is a higher-layer report in a resource pool that support sidelink communications. Proposal 3: Measurement report is triggered by higher layers. Proposal 5: Higher layers can deactivate a periodic (semi-persistent) reporting process. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 4.1.1-v1:

Support SLPP based higher layer signaling for sidelink positioning measurement report and report triggering.

- FFS on other higher layer signaling, e.g.,  RRC, MAC CE

- FFS on SCI based measurement report and report triggering

Companies are encouraged to share views in the following table.

Table 4.1.1 Collection of views on FL proposal 4.1.1-v1

| Company | Views on FL proposal 4.1.1-v1 |
| --- | --- |
| CATT | Support. |
| CMCC | We support that measurement reporting and reporting triggering should be via higher layer, and details should be determined by RAN2. |
| OPPO | Similar view with CMCC that it should be via high layer and up to RAN2’s determination. |
| Qualcomm | Support. We are also ok to leave details to RAN2 |
| Futurewei | OK |
| DCM | OK |
| LGE | Support |
| Samsung | We support SCI based approach also. |
| Intel | We prefer only higher layer signaling support for the report and report triggering as in our understanding the slightly lower latency of lower layer signalling is not a target of the SL positioning design. In addition, lower layer signalling would create significant, additional specification effort. |
| Lenovo | SLPP Request and Provide Location Information functionality has already been agreed by RAN2. So intention of second FFS bullet is not fully clear . We prefer to add “SLPP” on the first bullet as an example. |
| ZTE | Agree with CMCC’s comments. Details should be up to RAN2 and the two FFS can be deleted. |
| Apple | Support with details up to RAN2 |
| Nokia, NSB | In general OK, but it is up to RAN2 to discuss further on higher layer signalling. |
| InterDigital | We support higher layer reporting and signaling. We support SCI-based measurement report triggering (not SCI-based report) to reduce latency. We are ok to keep it as FFS but we suggest to remove SCI based measurement reporting. - FFS on SCI based measurement report and report triggering |

[MEDIUM]Time domain behavior

| R1-2302327 FUTUREWEI | Proposal 8: For SL-PRS measurement reports support aperiodic and periodic time distributions as the correspondent SL-PRS transmissions. |
| --- | --- |
| R1-2302606 Spreadtrum Communications | Proposal 6: Semi-persistent and aperiodic measurement report should be supported. |
| R1-2303064 Sharp | Proposal 3: Periodic, semi-persistent and aperiodic measurement reports are supported for SL positioning. |
| R1-2303264 Lenovo | Proposal 19: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports, where such reports may be based on higher-layer signalling. |
| R1-2303264 Lenovo | Discussion on SL Positioning Measurement and Reporting Proposal 19: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports, where such reports may be based on higher-layer signalling. |
| R1-2302709 CATT | Proposal 12: Periodic, semi-persistent and aperiodic measurement reporting should be supported in SL positioning. |
| R1-2303277 ZTE | Proposal 13: With regards to time domain behavior for sidelink positioning measurement report, LMF or UE may request a UE for either periodic reporting or triggered reporting. |
| R1-2303307 CEWiT | Proposal 10: In SL positioning measurement reporting, support aperiodic and periodic (semi-persistent) reports are supported. |
| R1-2303596 Qualcomm | Proposal 4: One-shot (aperiodic) and periodic (semi-persistent) reports are supported. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][Medium] FL proposal 4.1.2-v1:

Support at least one shot and periodic report for sidelink positioning measurement report.

Companies are encouraged to share views in the following table.

Table 4.1.2 Collection of views on FL proposal 4.1.2-v1

| Company | Views on FL proposal 4.1.2-v1 |
| --- | --- |
| Qualcomm | Support |
| Futurewei | There is no “one shot” concept defined in the spec. Maybe the proposal should clarify the meaning. |
| Intel | Support. |
| Apple | OK |
| Nokia, NSB | OK |

[HIGH]Unified or method specific report

The following proposals are identified to be related to this topic.

| R1-2303277 ZTE | #### Proposal 8: SL positioning measurements is not reported per SL positioning method. |
| --- | --- |
| R1-2302491 vivo | #### Discussion on measurements and reporting for SL positioning - A unified positioning method is supported, and to support one or more measurement reporting associated with the same identification can be reported together. |
| R1-2302709 CATT, GOHIGH | #### Proposal 14: Similar as Uu positioning, method-specific reporting for SL positioning should be supported in Rel-18. |
| R1-2303596 Qualcomm | #### Observation 1: A unified positioning method is limited in terms of forward compatibility and its ability to support devices implementing different releases. #### Proposal 1: Separate methods are specified for sidelink positioning. |

Before FL provide any proposal, companies are invited to share views on this issue(ie., the preference for unified or method specified report). Based on input from companies, FL would provide proposals to move forward.

#### [ROUND1][High]FL proposal 4.1.3-v1

Send LS to RAN2 to downselect one of the following

- Alt1: Support a unified positioning report where one or more measurement results of different methods can be reported together.

- Alt2: Support method-specific reporting for SL positioning

Companies are encouraged to share views in the following table.

Table 4.1.3 Collection of views on Issue 4.1.3-v1

| Company | Views on Issue 4.1.3-v1 |
| --- | --- |
| CATT | OK with the proposal. We support Alt.2, which is similar as Uu positioning. |
| CMCC | We are open for both alternatives and we think that it can be up to RAN2. |
| OPPO | We support Alt 2. |
| Qualcomm | We are ok with the proposal and prefer Alt 2 |
| Futurewei | OK, we support Alt 2. |
| DCM | Alt 2 |
| LGE | We support the method-specific reporting. It’s more optimized way of reducing signaling overhead. |
| Samsung | We prefer Alt 2. We suggest to check which alternative is the majority RAN 1 view at first. And then we may pick one alternative if possible. |
| Intel | We can accept the proposal. In our understanding there are a number of fields that can be common across the reports of multiple methods. Other information fields are present only for specific reports. |
| Lenovo | Support legacy structure with Alt.2. |
| ZTE | We support Alt.1 which is align with 38.455’s design. |
| Huawei, HiSilicon | We think this is generally related to multiple SL positioning methods (LPP like) or single SL positioning method (NRPPa). We prefer Alt.1 for SL, otherwise even for SL-TDOA, we may have define DL-like version and UL-like version. |
| vivo | Alt 1, and okay to send LS to RAN2. |
| Nokia, NSB | OK. We prefer Alt2. But, it is upto RAN2 to discuss further. |
| InterDigital | Alt. 2. Rx-Tx should be reported for RTT-based positioning methods and RSTD should be reported for TDOA-like positioning methods. |

Others

Companies are also encouraged to share views in the following table on any other missing issues for report signaling.

Table 4.1.4 Collection of views on any other missing issues for report signaling

| Company | Views on other missing issues for report signaling |
| --- | --- |

Contents of the positioning measurement report

The following contents were identified in the study phase.

Agreement For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement FFS any detail for the above

The following agreements were achieved in RAN1 #112.

Agreement

Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e.

Note: this does not imply a different measurement report content for reporting to LMF or to UE.

Table 6.2 Collection of measurement report content

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement |  |  |
| SL-PRS based RSTD measurement |  |  |
| SL-PRS based RSRP measurement |  |  |
| SL-PRS based RSRPP measurement |  |  |
| SL-PRS based RTOA measurement |  |  |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement |  |  |
| etc |  |  |

Agreement

Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE):

- SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE

  - FFS: whether SL-PRS resource set is supported

- Source ID and/or destination ID

- Other identification information not precluded

[HIGH]Identification information

| R1-2302378 Huawei, HiSilicon | Proposal 11: No need to provide ID for the UE receiving SL-PRS (destination ID) to either LMF or UE. - Proposal 13: RAN1 should send an LS to RAN2 regarding the potential difference in the UE ID in the measurement report to LMF or to the UE. |
| --- | --- |
| R1-2302491 vivo | - For identification information for a sidelink positioning measurement, the following options can be supported. - Source ID and/or destination ID of corresponding SL-PRS measurement. - ARP ID or panel Id if multiple panels are used. - SL-PRS resource ID if multiple resources are agreed to configure for a UE - Defer the discussion of unique UE identity in RAN1, and may reconsider the issue after SA2 is clear. |
| R1-2302709 CATT, GOHIGH | Proposal 22: Support the following identification information in sidelink positioning report - SL PRS resource set ID and SL PRS resource ID - Source ID and/or destination ID - ARP ID associated with measurement or SL-PRS resource |
| R1-2303444 InterDigital, Inc. | - The identification information should be designed to enable consistent identification of the UEs and the SL-PRS across all layers. |
| R1-2303064 Sharp | Proposal 5: “SL-PRS resource set” is not further considered as identification information in SL positioning measurement reports. |
| R1-2303277 ZTE | Proposal 10: UE identification information can be included in SL positioning measurement report. - How to define this UE identification information is up to RAN2 |
| R1-2303444 InterDigital, Inc. | Proposal 7: Identity of SL-PRS transmitting UE is included in the same measurement reporting. |
| R1-2303596 Qualcomm | Measurements and Reporting for SL Positioning Proposal 12: The measurement report contains the measurement, information to identify the resource pool where SL-PRS was received, source ID, destination ID, session ID, and quality metrics. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 4.2.1-v1:

Support to include the following identification information in sidelink positioning report

- SLPRS resource ID if multiple resources are agreed to be configured to a UE

- Source ID

  - FFS on whether the source ID belongs to UE receiving sidelink PRS or transmitting sidelink PRS, or whether the source ID belongs to anchor UE or target UE

- FFS destination ID

Companies are encouraged to share views in the following table.

Table 4.2.1 Collection of views on FL proposal 4.2.1-v1

| Company | Views on FL proposal 4.2.1-v1 |
| --- | --- |
| CATT | We think the SL PRS resource ID should be included, even a single resource are configured to a UE. In addition, the ARP ID associated with measurement or SL-PRS resource should also be included into the sidelink measurement report. |
| CMCC | OK. |
| OPPO | OK with the proposal. |
| Qualcomm | Destination ID should also be included. For example, it would help identify the group for groupcast transmission or the unicast link for unicast (since a UE can use multiple destination IDs for unicast) |
| Futurewei | OK |
| DCM | OK |
| LGE | Support with clarification. Source ID is intended to mean Layer-2 source ID, which needs to be clarified. #### [ROUND1][High] FL proposal 4.2.1-v1: Support to include the following identification information in sidelink positioning report - SLPRS resource ID if multiple resources are agreed to be configured to a UE - Layer-2 Source ID - FFS on whether the source ID belongs to UE receiving sidelink PRS or transmitting sidelink PRS, or whether the source ID belongs to anchor UE or target UE - FFS Layer-2 destination ID |
| Samsung | OK |
| Intel | As it is unclear which information fields are present in the dedicated and shared resource pool we think this cannot be discussed as we do not yet know which IDs are present. Thus, we would like to popsone discussing this topic until related fields are defined in agenda item 9.5.1.3. |
| Lenovo | Support 1st bullet. The existing source ID and destination ID definitions may need to take into account role reversal in SL Positioning procedures between Tx and Rx |
| ZTE | Support the first bullet. For source ID or destination ID, we prefer to leave this to RAN2. |
| Huawei, HiSilicon | We think that the terminology of source ID is not very accurate, and may be mixed with actual source ID for the SLPP message. The FFS point on the potential interpretation of source ID being the UE receiving the PRS is strengthens the confusion. We prefer to change source ID to UE ID transmitting SL-PRS, otherwise, the source ID itself should be further investigated. |
| vivo | We share the same view with Qualcomm that destination ID can be supported. |
| Nokia, NSB | Additionally, include ARP ID (of ARP associated with the measurement), Reference anchor UE ID. Regarding the FFS, we support reporting of both anchor UE IDs and/or target UE depending on positioning method. |
| InterDigital | We support the FL proposal. |

[MEDIUM]Timestamp information

| R1-2302927 LG Electronics | Proposal 12: Time stamp for SL positioning measurement report includes at least the followings. - SFN/DFN - Slot number - Symbol number (or SL PRS resource index within a slot) |
| --- | --- |
| R1-2302491 vivo | - The SL TimeStamp can be : - the SFN and the slot number relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if the serving cell timing reference is in use - And the nr-PhysCellID, nr-ARFCN, nr-CellGlobalID can be included to identify the serving cell - or the DFN and the slot number, otherwise - And the synchronization reference source indication (e.g., GNSS or UE), the DFN time (e.g., Tref, offset DFN) can be included |
| R1-2303277 ZTE | Proposal 11: Time stamp supported in SL positioning measurement report can be defined either relative to SFN or relative to DFN: - SFN: including SFN and slot number within the SFN and optionally including nr-PhysCellID, nr-ARFCN, nr-CellGlobalID, - DFN: including DFN and slot number within DFN or current UTC time obtained from GNSS - For either SFN or DFN, symbol number within a slot can also be included in time stamp |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][Medium] FL proposal 4.2.2-v1:

Time stamp for SL positioning measurement report includes at least the followings.

- SFN/DFN

- Slot number

- FFS associated reference of the time stamp

Companies are encouraged to share views in the following table.

Table 4.2.2 Collection of views on FL proposal 4.2.2-v1

| Company | Views on FL proposal 4.2.2-v1 |
| --- | --- |
| CMCC | OK. |
| OPPO | OK |
| Intel | Ok. |
| ZTE | Ok. We can also add symbol number for more precise time stamp. |
| Apple | Support |
| Nokia, NSB | OK |

[HIGH]Location information

The following proposals are identified to be related to location information

| R1-2303264 Lenovo | Proposal 16: SL Location Information transfer message may be used to request and transfer SL positioning measurements and location information and requested/reported per SL positioning method. Send LS to RAN2. |
| --- | --- |
| R1-2302491 vivo | - For Ranging/SL positioning service, the following information can be reported - Ranging - UE location - UE relative location. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][High] FL proposal 4.2.3-v1:

Location information of the target UE can be included in sidelink positioning measurement report.

- FFS: on whether quality information of location is included, e.g., uncertainty etc

Companies are encouraged to share views in the following table.

Table 4.2.3 Collection of views on FL proposal 4.2.3-v1

| Company | Views on FL proposal 4.2.3-v1 |
| --- | --- |
| CATT | OK with the proposal. |
| CMCC | To our understanding, position estimates of a UE is not carried in a measurement report in NR positioning, and we think that this issue belongs to RAN2. |
| Qualcomm | We support the UE reporting its location (i.e. UE based positioning). This could be part of the location information report as in Uu. |
| Futurewei | OK in principle. Needs to be confirmed by RAN2 |
| DCM | OK |
| Samsung | Is this for UE-based SL positioning? If yes, location information can be included when UE-based SL positioning is configured. |
| Intel | Ok. |
| Lenovo | Support, and think that the FFS enables the evaluation of the provided location information. |
| ZTE | Support. |
| Huawei, HiSilicon | We do not support this. It is not clear why the receiving end of SLPP needs the target UE location. Location server UE is used to calculate the target UE location, and LMF obtaining the target UE location should use common IE, instead of positioning method specific IE. |
| Apple | This is a RAN2 issue. Also, similar to Uu positioning, the location information should be sent to the “LMF” only for privacy reasons. |
| Nokia, NSB | OK |

[HIGH]Additional path

The following proposals are identified to be related to this topic.

| R1-2302709 CATT | Proposal 17: AdditionalPathList should be supported in sidelink positioning report, which includes one or more additional detected path timing values for SL PRS resource, relative to the path timing used for determining the SL-RSTD or SL-RTOA or SL-AOA measurements. |
| --- | --- |
| R1-2302491 vivo | - The first path and additional path can be supported for SL-based RSTD, Rx-Tx time difference, RToA, AoA measurement. - The additional path can be [2, 4, 8] for a SL-PRS. |
| R1-2303415 Fraunhofer IIS, Fraunhofer HHI | Proposal 4: Support additional path reporting for SL-PRS timing measurements. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][HIGH] FL proposal 4.2.4-v1:

Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path.

Companies are encouraged to share views in the following table.

Table 4.2.4 Collection of views on FL proposal 4.2.4-v1

| Company | Views on FL proposal 4.2.4-v1 |
| --- | --- |
| CATT | Support. In addition, we think additional detected path timing values for SL PRS resource should also be included in the sidelink measurement report. |
| CMCC | Support. |
| OPPO | OK |
| Qualcomm | We can accept the additional path as a compromise with the following changes: Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path. - No specification impact for how to set the additional path measurements. - From RAN1 perspective, no performance requirements are expected to be defined for setting the additional-path measurements in Rel-18. |
| Futurewei | OK |
| DCM | OK |
| LGE | Support as they are already supported in the legacy Uu link positioning. |
| Samsung | OK |
| Intel | Support. |
| Lenovo | Ok to support |
| ZTE | OK |
| Apple | OK |
| Nokia, NSB | OK |

[MEDIUM]Unified content for both reporting to LMF and reporting to UE

The following proposals are identified to be related to this topic.

| R1-2303596 Qualcomm Incorporated | Proposal 12: The measurement report contains the measurement, information to identify the resource pool where SL-PRS was received, source ID, destination ID, session ID, and quality metrics. Proposal 14: The same measurement report content is used for both reporting to LMF and reporting to UE and contains at least the following: |
| --- | --- |
| R1-2302491 vivo | - For measurement reporting to target UE or anchor UE for SL-PRS based Rx-Tx measurement, UE’s source ID and destination ID can be omitted since it has been transmitted by low layer. - The measurement report content for SL-PRS based Rx-Tx measurement to target UE or anchor UE same as the content to report to LMF except UE’s source ID and destination ID can be omitted. |

Based on above proposals, the following FL proposal is provided for company to share their views.

#### [ROUND1][Meduim] FL proposal 4.2.5-v1:

The same measurement report content is used for both reporting to LMF and reporting to UE

Companies are encouraged to share views in the following table.

Table 4.2.5 Collection of views on FL proposal 4.2.5-v1

| Company | Views on FL proposal 4.2.5-v1 |
| --- | --- |
| Qualcomm | Support. This provides better forward compatibility when additional functionality is added to the LMF or to the UE |
| Futurewei | OK as the starting point. More discussion needed about LMF/UE functionality. |
| Intel | Support. |
| ZTE | ok |
| Nokia, NSB | OK |

[MEDIUM]The summary content for the positioning measurement report

FL collects all the content information from companies contribution. FL would like to treat this part after stabilizing 4.2.1 and 4.2.2.

Reporting content for RTT

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| R1-2302294 Nokia | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. SyncRef change status. Expected clock error. | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. SyncRef change status. Expected clock error. |
| R1-2302327 FUTUREWEI | SL-PRS ID, SL-PRS resource ID, SL-PRS resource set ID for each measurement. SL-PRS-RSRP measurement. UE Rx-Tx time difference measurement. Time stamp of the measurement. Quality for each measurement. UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs associated with UE Rx-Tx time difference measurements. LOS/NLOS information for UE measurements. SL-PRS-RSRPP measurement. The association of UE Tx TEG ID and SL-PRS. Sync source information. Anchor quality information. | SL-PRS ID, SL-PRS resource ID, SL-PRS resource set ID for each measurement. SL-PRS-RSRP measurement. UE Rx-Tx time difference measurement. Time stamp of the measurement. Quality for each measurement. UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs associated with UE Rx-Tx time difference measurements. LOS/NLOS information for UE measurements. SL-PRS-RSRPP measurement. The association of UE Tx TEG ID and SL-PRS. Sync source information. Anchor quality information. |
| R1-2302378 Huawei, HiSilicon | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| R1-2302491 vivo | One or multiple measurement elements can be included, for each measurement element can include - （Optional）Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ） - ARP/Panel ID if multiple ARP/Panel are used for reception UE - Resource/set ID if multiple resources/sets are used - Rx-Tx timing difference which is measured by Rx UE - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| R1-2302554 OPPO |  |  |
| R1-2302606 Spreadtrum |  |  |
| R1-2302709 CATT | SL Rx-Tx time difference; RTT type and transmission order; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; Timestamp; Measurement Quality; Additional path; | SL Rx-Tx time difference; RTT type and transmission order; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; Timestamp; Measurement Quality; Additional path; |
| R1-2302802 Intel | UE Identification SL-PRS resource Identification SL Rx – Tx time difference for the first RTT exchange SL Rx – Tx time difference for the second RTT exchange (only for double side RTT) SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx + Tx + RxTx) | UE Identification SL-PRS resource Identification SL Rx – Tx time difference for the first RTT exchange SL Rx – Tx time difference for the second RTT exchange (only for double side RTT) SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx + Tx + RxTx) |
| R1-2302852 Sony |  |  |
| R1-2302927 LG Electronics | For SS-RTT - Received SL PRS ID (or target/anchor UE ID) - Received SL PRS resource ID - Time stamp of the received SL PRS resource - Measured RX-TX time difference - Expected RX-TX time difference and uncertainty For DS-RTT - Type of double-sided SL RTT method - Type of SL PRS transmission order - SL PRS ID (or target/anchor UE ID) - Received SL PRS resource ID #1 - Time stamp of the received SL PRS resource #1 - If two SL PRS resources are received - Received SL PRS resource ID #2 - Time stamp of the received SL PRS resource #2 - Measured RX-TX time difference #1 - Expected RX-TX time difference #1 and uncertainty - Measured RX-TX time difference #2 - Expected RX-TX time difference #2 and uncertainty | For SS-RTT - Received SL PRS ID (or target/anchor UE ID) - Received SL PRS resource ID - Time stamp of the received SL PRS resource - Measured RX-TX time difference - Expected RX-TX time difference and uncertainty For DS-RTT - Type of double-sided SL RTT method - Type of SL PRS transmission order - SL PRS ID (or target/anchor UE ID) - Received SL PRS resource ID #1 - Time stamp of the received SL PRS resource #1 - If two SL PRS resources are received - Received SL PRS resource ID #2 - Time stamp of the received SL PRS resource #2 - Measured RX-TX time difference #1 - Expected RX-TX time difference #1 and uncertainty - Measured RX-TX time difference #2 - Expected RX-TX time difference #2 and uncertainty |
| R1-2302989 xiaomi |  |  |
| R1-2303064 Sharp |  |  |
| R1-2303134 Samsung | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) |
| R1-2303240 CMCC |  | Proposal 13: For relative positioning/ranging, the SL Rx-Tx time difference measurement, SL AoA/ZoA measurement and/or SL RSRP/RSRPP measurement should be reported to from the anchor UE to the target UE, associated with SL PRS index, UE ID (i.e., source ID and/or destination ID) and time stamp. |
| R1-2303264 Lenovo |  |  |
| R1-2303277 ZTE | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |
| R1-2303307 CEWiT |  |  |
| R1-2303415 Fraunhofer |  |  |
| R1-2303444 InterDigital |  |  |
| R1-2303489 Apple |  |  |
| R1-2303551 Ericsson |  |  |
| R1-2303596 Qualcomm | SL-PRS Timestamp Identification information | SL-PRS Timestamp Identification information |
| R1-2303775 Locaila |  |  |
| R1-2303842 MediaTek |  |  |

Reporting content for RSTD

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| R1-2302294 Nokia | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. Anchor UE IDs. Reference Anchor UE for SL RSTD. Synchronization source. Time difference in synchronization with other anchor UEs and own synchronization source. | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. Anchor UE IDs. Reference Anchor UE for SL RSTD. Synchronization source. Time difference in synchronization with other anchor UEs and own synchronization source. |
| R1-2302327 FUTUREWEI | Latitude/Longitude/Altitude, together with uncertainty shape. PRS resource ID, SL-PRS resource set ID and SL-PRS ID for each measurement. SL RSTD measurement. SL-PRS-RSRP measurement. Time stamp of the measurements. Time stamp of location estimate. Quality for each measurement. UE Rx TEG IDs for SL RSTD measurements. SL-PRS-RSRPP measurement. LOS/NLOS information for UE measurements. Sync source information. Anchor quality information. | Latitude/Longitude/Altitude, together with uncertainty shape. PRS resource ID, SL-PRS resource set ID and SL-PRS ID for each measurement. SL RSTD measurement. SL-PRS-RSRP measurement. Time stamp of the measurements. Time stamp of location estimate. Quality for each measurement. UE Rx TEG IDs for SL RSTD measurements. SL-PRS-RSRPP measurement. LOS/NLOS information for UE measurements. Sync source information. Anchor quality information. |
| R1-2302378 Huawei, HiSilicon | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| R1-2302491 vivo | Reference anchor UE information includes: - Reference Anchor UE identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - RSTD measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| R1-2302554 OPPO |  |  |
| R1-2302606 Spreadtrum |  |  |
| R1-2302709 CATT | SL RSTD; Reference UE ID; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; Additional path; | SL RSTD; Reference UE ID; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; Additional path; |
| R1-2302802 Intel | UE Identification SL-PRS resource Identification SL-PRS based RSTD measurement SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx) | UE Identification SL-PRS resource Identification SL-PRS based RSTD measurement SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx) |
| R1-2302852 Sony |  |  |
| R1-2302927 LG Electronics | - Reference SL PRS ID (or the reference anchor UE ID) - Reference SL PRS resource ID - SL PRS ID (or the associated anchor UE ID) - SL PRS resource ID - Time stamp of SL PRS resource - Measured RSTD - Expected RSTD and uncertainty | - Reference SL PRS ID (or the reference anchor UE ID) - Reference SL PRS resource ID - SL PRS ID (or the associated anchor UE ID) - SL PRS resource ID - Time stamp of SL PRS resource - Measured RSTD - Expected RSTD and uncertainty |
| R1-2302989 xiaomi |  |  |
| R1-2303064 Sharp |  |  |
| R1-2303134 Samsung | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) |
| R1-2303240 CMCC | Proposal 12: For DL-like UE-assisted SL absolute positioning, the SL RSTD measurement and/or SL RSRP/RSRPP measurement should be reported to the LMF or location server, associated with SL PRS index, anchor unique UE ID, and time stamp. |  |
| R1-2303264 Lenovo |  |  |
| R1-2303277 ZTE | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |
| R1-2303307 CEWiT |  |  |
| R1-2303415 Fraunhofer |  |  |
| R1-2303444 InterDigital |  |  |
| R1-2303489 Apple |  |  |
| R1-2303551 Ericsson |  |  |
| R1-2303596 Qualcomm | SL-PRS Timestamp Identification information | SL-PRS Timestamp Identification information |
| R1-2303775 Locaila |  |  |
| R1-2303842 MediaTek |  |  |

Reporting content for RSRP/RSRPP

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| R1-2302294 Nokia | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. |
| R1-2302327 FUTUREWEI |  |  |
| R1-2302378 Huawei, HiSilicon | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| R1-2302491 vivo | Needs to be reported with other measurements |  |
| R1-2302554 OPPO |  |  |
| R1-2302606 Spreadtrum |  |  |
| R1-2302709 CATT | SL RSRP/ SL RSRPP LoS/NLoS indication UE location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; | SL RSRP/ SL RSRPP LoS/NLoS indication UE location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; |
| R1-2302802 Intel | No separate report for RSRP needed. | No separate report for RSRP needed. |
| R1-2302852 Sony |  |  |
| R1-2302927 LG Electronics |  |  |
| R1-2302989 xiaomi |  |  |
| R1-2303064 Sharp |  |  |
| R1-2303134 Samsung | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID |
| R1-2303240 CMCC |  |  |
| R1-2303264 Lenovo |  |  |
| R1-2303277 ZTE | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |
| R1-2303307 CEWiT |  |  |
| R1-2303415 Fraunhofer |  |  |
| R1-2303444 InterDigital |  |  |
| R1-2303489 Apple |  |  |
| R1-2303551 Ericsson |  |  |
| R1-2303596 Qualcomm | SL-PRS Timestamp Identification information | SL-PRS Timestamp Identification information |
| R1-2303775 Locaila |  |  |
| R1-2303842 MediaTek |  |  |

Reporting content for RTOA

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| R1-2302294 Nokia | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. Target UEs. Synchronization source. Time difference in synchronization with other anchor UEs and own synchronization source. | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. Target UEs. Synchronization source. Time difference in synchronization with other anchor UEs and own synchronization source. |
| R1-2302327 FUTUREWEI |  |  |
| R1-2302378 Huawei, HiSilicon | CN UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator |
| R1-2302491 vivo | Reference anchor UE information includes: - Reference Anchor UE identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - RSTD measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| R1-2302554 OPPO |  |  |
| R1-2302606 Spreadtrum |  |  |
| R1-2302709 CATT | SL RTOA LoS/NLoS indication UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; | SL RTOA LoS/NLoS indication UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; TimeStamp; Measurement Quality; |
| R1-2302802 Intel | UE Identification SL-PRS resource Identification SL-PRS based RTOA measurement SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx) | UE Identification SL-PRS resource Identification SL-PRS based RTOA measurement SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement UE SL TEG information (Rx) |
| R1-2302852 Sony |  |  |
| R1-2302927 LG Electronics | - SL synchronization source (gNB/eNB or GNSS) - SL PRS ID (or the associated anchor UE ID) - SL PRS resource ID - Time stamp of SL PRS resource - Measured RTOA - Expected RTOA and uncertainty | - SL synchronization source (gNB/eNB or GNSS) - SL PRS ID (or the associated anchor UE ID) - SL PRS resource ID - Time stamp of SL PRS resource - Measured RTOA - Expected RTOA and uncertainty |
| R1-2302989 xiaomi |  |  |
| R1-2303064 Sharp |  |  |
| R1-2303134 Samsung | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) | LOS/NLOS indicator time stamp SL-PRS resource ID/SL-PRS resource set ID quality of synchronization information (e.g., clock source’s accuracy) |
| R1-2303240 CMCC |  |  |
| R1-2303264 Lenovo |  |  |
| R1-2303277 ZTE | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |
| R1-2303307 CEWiT |  |  |
| R1-2303415 Fraunhofer |  |  |
| R1-2303444 InterDigital |  |  |
| R1-2303489 Apple |  |  |
| R1-2303551 Ericsson |  |  |
| R1-2303596 Qualcomm | SL-PRS Timestamp Identification information | SL-PRS Timestamp Identification information |
| R1-2303775 Locaila |  |  |
| R1-2303842 MediaTek |  |  |

Reporting content for AoA/ZoA

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| R1-2302294 Nokia | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. | LoS/NLoS indicator. Quality of measurement. Time stamp. Antenna panel ID. Antenna location information. |
| R1-2302327 FUTUREWEI | NCGI and TRP ID of the measurement. SL-RTOA. SL-PRS-RSRP. SL-PRS-RSRPP. SL Angle of Arrival (azimuth and/or elevation). Multiple SL Angle of Arrival (azimuth and/or elevation). PRS Resource Type. Time stamp of the measurement. Quality for each measurement. Beam Information for each measurement. LoS/NLoS information for each measurement. ARP ID of the measurement. Sync source information. Anchor quality information. | NCGI and TRP ID of the measurement. SL-RTOA. SL-PRS-RSRP. SL-PRS-RSRPP. SL Angle of Arrival (azimuth and/or elevation). Multiple SL Angle of Arrival (azimuth and/or elevation). PRS Resource Type. Time stamp of the measurement. Quality for each measurement. Beam Information for each measurement. LoS/NLoS information for each measurement. ARP ID of the measurement. Sync source information. Anchor quality information. |
| R1-2302378 Huawei, HiSilicon | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation |
| R1-2302491 vivo | One or multiple measurement elements can be included, for each measurement element can include - (Optional)Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - AoA measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| R1-2302554 OPPO |  |  |
| R1-2302606 Spreadtrum |  |  |
| R1-2302709 CATT | SL AoA/ZoA; translation of the LCS to GCS; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; Timestamp ; Measurement Quality; Additional path; | SL AoA/ZoA; translation of the LCS to GCS; LoS/NLoS indication; UE location information; ARP location information; Identification information{SL PRS resource ID, SL PRS resource set ID, source ID, destination ID, ARP ID}; Timestamp ; Measurement Quality; Additional path; |
| R1-2302802 Intel | UE Identification SL-PRS resource Identification SL AOA measurement (azimuth and/or elevation) SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement | UE Identification SL-PRS resource Identification SL AOA measurement (azimuth and/or elevation) SL-PRS RSRP SL-PRS RSRPP Time stamp of each measurement Quality of each measurement Los/NLOS information for each measurement |
| R1-2302852 Sony |  |  |
| R1-2302927 LG Electronics |  |  |
| R1-2302989 xiaomi |  |  |
| R1-2303064 Sharp |  |  |
| R1-2303134 Samsung |  |  |
| R1-2303240 CMCC |  | Proposal 13: For relative positioning/ranging, the SL Rx-Tx time difference measurement, SL AoA/ZoA measurement and/or SL RSRP/RSRPP measurement should be reported to from the anchor UE to the target UE, associated with SL PRS index, UE ID (i.e., source ID and/or destination ID) and time stamp. |
| R1-2303264 Lenovo |  |  |
| R1-2303277 ZTE | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |
| R1-2303307 CEWiT |  |  |
| R1-2303415 Fraunhofer |  |  |
| R1-2303444 InterDigital |  |  |
| R1-2303489 Apple |  |  |
| R1-2303551 Ericsson |  |  |
| R1-2303596 Qualcomm | SL-PRS Timestamp Identification information | SL-PRS Timestamp Identification information |
| R1-2303775 Locaila |  |  |
| R1-2303842 MediaTek |  |  |

Others

Companies are encouraged to share views in the following table on any other missing issues on sidelink positioning report.

Table 4.3 Collection of views on any other missing issues

| Company | Views on any other missing issue for sidelink positioning report |
| --- | --- |
| Apple | Anchor UE orientation: Given that the orientation of the positioning set at the time of measurement is important (e.g. a rotation occurs either during or after the transmission), the orientation of the measuring UE as well as a time stamp of when this measurement was made should be part of the measurement report |

Assistance data for SL positioning

Location information

| R1-2302709 CATT, GOHIGH | Proposal 21: ARP ID should be reported associated with ARP location information. |
| --- | --- |
| R1-2302852 Sony | Proposal 8: To facilitate SL positioning, support reporting of the location information of the RSUs involved in the ranging measurement. Proposal 5: For V2X ranging, support the reporting of the location of each antenna with respect to the reference point (e.g., geometry centre of the vehicle) to the LMF. In the case of a car equipped with DAS, multiple location reports may be required. Proposal 6: For V2X positioning, support reporting of the geometry information of a vehicle, such as the geometry boundary of the vehicle. |
| R1-2303134 Samsung | Proposal 13: The reliability of the anchor UEs’ location coordinates is indicated to the target UE for SL positioning. |
| R1-2303444 InterDigital, Inc. | Proposal 5: An anchor UE reports its location and associated uncertainty information to LMF (for in-coverage) and a target UE (for out-of-coverage). |

#### [High] FL proposal 5.1.1-v1:

For SL positioning, the anchor UE location information can be provided to LMF and target UE.

Companies are encouraged to share views in the following table.

Table 5.1.1 Collection of views on FL proposal 5.1.1-v1

| Company | Views on FL proposal 5.1.1-v1 |
| --- | --- |
| CATT | Support in principle. We think the anchor UE location information is essential for sidelink absolute positioning. We prefer the following revision. #### Updated [High] FL proposal 5.1.1-v1: For SL absolute positioning, the anchor UE location information should can be provided to LMF and target UE. |
| CMCC | We are generally OK. In addition, the location information of anchor UEs can also be provided to server UE, e.g., in out-of-coverage scenario, as the server UE at least have the functionality to calculate the location. |
| OPPO | OK with the proposal. |
| Qualcomm | We are generally ok with the proposal, but propose to replace “target UE” with “UE” |
| Futurewei | OK |
| LGE | Support with one comment. The anchor UE location is necessary only for absolute SL positioning. We suggest to add clarification on this. #### [High] FL proposal 5.1.1-v1: For absolute SL positioning, the anchor UE location information can be provided to LMF and target UE. |
| Samsung | Since UEs’ location information is not like TRPs’ location information, we suggest to modify the proposal as: For absolute SL positioning, the anchor UE location information and it’s quality information can be provided to LMF and target UE. |
| Intel | Support. |
| Lenovo | Support but also view that the quality/uncertainty of the anchor UE location information is equally needed to assess with the anchor UE requires any re-selection (if applicable) |
| ZTE | Also prefer to remove “target”. |
| Apple | Agree that provision to the target UE should be removed. |
| SONY | Support this proposal. SL absolute positioning relies on the anchor UE location information. |
| Nokia, NSB | OK in general. Consider reporting to server UE as well. |

Other

Companies are encouraged to provide inputs in the following table for whether there is any other missing issues with high priority for assistance data.

Table 5.2 Collection of views on other assistance data.

| Company | Views on on other assistance data |
| --- | --- |
| R1-2303240 CMCC | Proposal 5: RSTD search window is not required to be configured/provided to the UE. |
| R1-2303264 Lenovo | Proposal 7: RAN1 to support the provision of SL RSTD and SL RTOA expected search window information for anchor UEs or target-UE. |

Others

Companies are encouraged to share views in the following table on aspects other than measurement and report.

Table 6 Collection of views on aspects other than measurement and report

| Company | Views on aspects other than measurement and report |
| --- | --- |

OFFLINE Sessions Outcome

1st offline

2nd offline

Proposal for Online Discussion

1st online

#### FL proposal 3.5.1:

Support per ARP based measurement in sidelink positioning. The ARP related information  can be reported along with the SL measurement.

- FFS details on ARP related information.

#### FL proposal 3.2.3:

- For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission.

  - When the UE selects a cell as the synchronization reference source uses the network as a synchronization source, SFN0/SFN is used for SL-PRS based RTOA. [The timing of SFN0/SFN for RTOA reference timing is determined by advancing DL SFN0/SFN with TA/2.]

  - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA.

- FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting

#### FL proposal 3.2.5:

- SFN/DFN Initialisation Time can be provided by server UE or by LMF.

- For the definition of SL-PRS based RTOA,  update the definition of reference timing as:

- the reference timing is defined as T0+tSL-PRS, where

  - T0 is the nominal beginning time of SFN 0 or DFN0 provided by SFN/DFN Initialisation Time

    - FFS on how to select between SFN 0 or DFN 0 for determination of T0.

    - FFS: the source for the reference timing

  - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

    - FFS on how to select between SFN or DFN

#### FL proposal 3.2.2:

Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement

- Exchange of synchronization information of anchor UEs between anchor UE and target/server UE/ LMF

  - FFS detailed synchronization information. E.g: synchronization source, synchronization error, [SFN/DFN Initialisation Time]

  - [No changes are introduced to sidelink synchronization procedure]

- FFS other mechanisms

#### FL proposal 3.1.1:

For definition of SL-PRS based Rx-Tx measurement, support the following Alt1 to minimize the impact of UE reference timing offset and mobility

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Note：This is at least applicable for both double sided RTT definition, FFS on whether a different definition is needed for single sided RTT definitions.

- FFS whether to limit the reporting range to within 1msec similar as in NR Uu positionng.

2nd online

3rd online

Previous Agreements for Measurements and Reporting for SL positioning

Agreements from  RAN1 #112 (202302 Athens meeting)

Agreement

SL PRS reference signal received power (SL PRS-RSRP)

- is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth

With regard to the reference point

- For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE.

- For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches.

Agreement

SL PRS reference signal received path power (SL PRS-RSRPP),

- is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

With regard to the reference point

- For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE.

- For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

Agreement

SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where

- T0 is the nominal beginning time of SFN 0 or DFN0.

  - FFS on how to select between SFN 0 or DFN 0 for determination of T0.

  - FFS: the source for the reference timing

- tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

  - FFS on how to select between SFN or DFN

Agreement

Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement.

- FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

Agreement

For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

Agreement

Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE.

Agreement

For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs.

- FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report

- FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders

  - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

Agreement

Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE).

Agreement

Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE):

- SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE

  - FFS: whether SL-PRS resource set is supported

- Source ID and/or destination ID

- Other identification information not precluded

Agreement

LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE).

- LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible.

- No specification impact for how to set this indicator.

- From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18.

Agreement

Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e.

Note: this does not imply a different measurement report content for reporting to LMF or to UE.

Table 6.2 Collection of measurement report content

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement |  |  |
| SL-PRS based RSTD measurement |  |  |
| SL-PRS based RSRP measurement |  |  |
| SL-PRS based RSRPP measurement |  |  |
| SL-PRS based RTOA measurement |  |  |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement |  |  |
| etc |  |  |

Agreements from  RAN1 #112bis-e (202304 eMeeting)

References

      - RP-223549 “New WID on Expanded and Improved NR Positioning”, RAN 98e, Dec.12-16, 2022