3GPP TSG RAN WG1 #112bis-e                                 	       R1-2303961

e-Meeting, April 17th –26th, 2023

Agenda Item:			9.12.1

Source:			Moderator (China Telecom)

Title:			FL Summary#3 on PRACH coverage enhancements

Document for:		Discussion

- Introduction

In RAN #94 e-meeting, a new Rel-18 work item on further NR coverage enhancements was approved [1] and updated in RAN #96 [2]. The objective of the work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Detailed objectives are listed as follows:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beam for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

This contribution is a summary of companies’ contributions on PRACH coverage enhancements.

- Summary of contributions

2.1 Multiple PRACH transmissions with same beam

Based on companies’ contributions, sometimes the term “PRACH repetition” is utilized to indicate “multiple PRACH transmissions with same beam”. Thus, it needs to be clarified that the term “PRACH repetition” only indicates “multiple PRACH transmissions with same beam”, it doesn’t put any additional restrictions on multiple PRACH transmissions.

2.1.1 Resource configuration for multiple PRACH transmissions

Issue #1: Differentiation between single PRACH transmission and multiple PRACH transmissions

- In RAN1 #112 meeting [3], the following working assumptions were achieved for the differentiation between single PRACH transmission and multiple PRACH transmissions.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Based on the contributions, companies’ views are summarized as follows:

- Confirm both working assumptions:

Support (13): CATT, ZTE, Intel, Nokia, NSB, Panasonic, Lenovo, Quectel, CMCC, Apple, LG, Ericsson, Qualcomm(?)

- Confirm the working assumption with respect to “…support that multiple PRACH are transmitted on separate ROs…”:

Support (6): Spreadtrum, vivo, TCL, Fujitsu, China Telecom, NTT DOCOMO

- Confirm the working assumption with respect to “…support that multiple PRACH are transmitted with separate preamble on shared ROs…”:

Support (1): MediaTek

Regarding the FFS part, companies [CATT, Apple, Mavenir, Lenovo, Samsung, ETRI, Xiaomi] propose that Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions, while [ZTE, Nokia] proposes that it is up to RAN2 to decide whether the above Rel-17 framework can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. [Huawei] propose to consider the following methods of PRACH resource partitioning: Rel-17 framework of feature combination and additional RACH configuration can be used to realize the PRACH resource partitioning of transmitting multiple PRACH on separate ROs for gNBs with multiple simultaneous receiving analog beams; Rel-17 framework of feature combination and additional RACH configuration can realize the PRACH resource partitioning of transmitting multiple PRACH on shared ROs, where different repetition levels of multiple PRACH transmissions are regarded as different feature IEs in the RRC FeatureCombination-r17. If both working assumptions are confirmed, companies [Panasonic, LG] propose that it’s up to gNB whether to configure sperate ROs or separate preambles on shared ROs for multiple PRACH transmissions.

Moreover, [CATT, Apple, LG, NEC(?)] propose that multiple PRACH transmissions with same Tx beam is introduced as a new feature. Multiple values for the number of multiple PRACH transmissions can be regarded as different features when configuring the framework of feature combination and additional RACH configuration.

In addition, [Ericsson, China Telecom, Apple] propose that if the WA(s) is(are) agreed, send an LS to RAN2, informing RAN2 of RAN1’s decision and that it is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions.

Other companies’ views are summarized as follows:

- [Fujitsu] The WA with respect to shared RO case should be applied only for CFRA.

- [ZTE] Support separate ROs based on legacy PRACH configuration with additional new parameters. In addition, All the RO configurations for multiple PRACH transmissions can be individual parameters (including the T/F resource, repetition number, etc.), which are configured for different coverage levels.

- [Spreadtrum] At least SSB-RSRP threshold(s) are used to enable which option to differentiate the multiple PRACH transmissions with single PRACH transmission.

Issue #2: RO group

Different from legacy single PRACH transmission, where UE can randomly select one of the ROs associated with the selected SSB for PRACH transmission, if UE randomly select a given number of ROs from the ROs assigned for multiple PRACH transmission, gNB may be not able to perform joint detection since gNB is not sure which ROs are utilized by the UE for PRACH transmission. To deal with this issue, it’s necessary to align gNB’s and UE’s understanding about the potential time and frequency position of the corresponding ROs utilized for PRACH repetitions.

In RAN1 #112, the definition of “RO group” was discussed and the following agreement was achieved:

Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. - Note 1: All ROs in one RO group is associated with the same SSB(s). - Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - Note 3: whether/how to define “RO group” in specification will be discussed separately - Note 4: Valid RO(s) refers to what is defined in existing specification - FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. - FFS: the time span of RO group. - FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. - FFS: other details

Based on the contributions, the majority companies further discuss about the definition, configuration of RO group for multiple PRACH transmissions. Based on current definition, RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. But it is not clear whether these ROs can be located in the same time instance and whether the starting RB of the ROs within one RO group can be different. Notice that these two issues are related to the fundamental design of RO group, the summary will start from these two issues. Then, it goes for the discussion about details of RO group.

- Issue #2-1: Whether/how multiple PRACH transmissions located in the same time instance

The following agreement was achieved in RAN1 110b-e for ROs assignment for multiple PRACH transmissions [4]:

Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

- Since RAN1 #111 [5], companies make comprehensive discussion on the 2nd FFS parts and companies’ views are basically not changed, which are summarized as follows:

- Companies [Huawei, Fujitsu, Xiaomi, Intel, ETRI, Lenovo, LG, MediaTek, Nokia, Samsung, Spreadtrum, ZTE, China Telecom] propose not to support/ deprioritize multiple PRACH transmissions located at same time instance due to the following reasons: small benefit and high complexity; there may be more than one resource multiplexing modes in this network, which will greatly increase the complexity of the non-coherent combining detection at gNB side; only half of the deliverable power is available for each Tx chain.

- While companies [CMCC, Ericsson] think multiple PRACH transmissions located in the same time instance can be further studied for UE with 2 or more Tx chains. Moreover, [Ericsson] proposes to support simultaneous FDMed/CDMed (“CDMed” means multiple PRACH transmissions with different preambles in one RO) PRACH transmissions in Rel-18, including the association of different FDMed ROs/the association of different preambles in an RO within one RACH attempt.

In addition, [Ericsson] provides some simulation results as follow:

- FDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1.5 dB and a single PRACH with two antennas by 0.5dB.

- CDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1 dB and has nearly the same performance as a single PRACH transmission with two antennas.

- Issue #2-2: Whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

Based on the contributions, companies [vivo, Intel, OPPO, CATT, TCL, LG, NTT DOCOMO, Qualcomm, Nokia, Quectel, CMCC, FGI] propose to consider different starting RB of ROs at different time instances (RO frequency hopping) for multiple PRACH transmissions, while [Xiaomi] does not support different starting RB of ROs at different time instances for multiple PRACH transmissions.

Some simulation results are provided by companies as follows:

- [vivo] The gain of PRACH repetition with RO hopping is about 0.7dB compared to that of PRACH repetition without RO hopping in urban O2I scenario.

- [Intel] For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping. (@700MHz, PRACH format 0, TDL-C, 300ns)

Other views are summarized as follows:

- [Qualcomm] Study how to handle transmission at time having multiple ROs associated with the same SSB but the RO that would have been use for PRACH transmission is invalid.

- Issue #2-3: Definition and configuration of RO group

- Fundamental definition of RO group

Based on previous discussion and the agreement, current definition of RO group indicates the following things:

1. The ROs in one RO group is valid ROs, thus, it indicates validation rule is performed before determining the ROs in one RO group.

2. If multiple values of multiple PRACH transmissions are configured, one or multiple RO group(s) is configured for each configured number of multiple PRACH transmissions, and the number of valid ROs in one RO group is equal to the corresponding number of multiple PRACH transmissions.

However, based on the contributions, companies seem have divergent understanding about the second point. Thus, it is necessary to check and align companies’ understanding about the fundamental definition of RO group.

Companies [vivo, Lenovo, NTT DOCOMO, Nokia, Apple] propose that one RO group consists of TDMed ROs. In fact, this depends on the outcome of Issue #2-1, if multiple PRACH transmissions located in the same time instance is not supported, then one RO group consists of only TDMed ROs based on current agreements.

In addition, [Samsung] propose that it’s up to gNB implementation to ensure that there is at least one RO group available to use if the corresponding value of multiple PRACH transmissions is configured.

- How an RO group is defined and configured

Companies discuss about the details about how an RO group is defined. [LG] proposes that an RO group is defined by using starting RACH slot (and/or RO), its period, and repetition number for multiple PRACH transmissions, either based on PRACH configuration, SSB-to-RO mapping, and repetition number or provided by gNB via SIB independently for each repetition number. [China Telecom] proposes that if multiple PRACH transmission with frequency hopping is not supported or supported but not enabled, at least the following parameters: {start position in time domain, duration} are needed to determine the RO groups.

If multiple PRACH transmission with frequency hopping is supported, companies [China Telecom, Nokia, CATT, NTT DOCOMO] discuss about how an RO group is defined. [China Telecom] proposes that at least the following parameters: {start position in time domain, start position in frequency domain, duration per each hop, hopping offset} are needed to determine the RO groups. [NTT DOCOMO] propose that when there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS, different frequency domain locations for different ROs within one RO group can be achieved by indicating/defining a frequency domain index offset among different ROs within one group. As some company mentioned, it should also be noted that since RO group consists of valid RO(s) for multiple PRACH transmission, if frequency hopping is supported and enabled, when performing frequency hopping with a frequency offset (if defined), mod operation may need to be considered to determine the frequency hopping position to ensure that the RO locates within the configured ROs.

In addition, [CATT, Samsung] propose that RO group pattern within the SSB-to-RO association pattern period can be considered/supported. [Samsung] further point out that RO group pattern for each associated SSB could be repeated with any other SSB-to-RO association pattern period. If there is some leftover ROs which cannot form a complete RO group, these ROs are not used for multiple PRACH transmissions.

- How multiple PRACH transmissions are performed based on RO group

Companies [Nokia, Mavenir, China Telecom] discuss about how multiple PRACH transmissions are performed based on RO group. The general idea is similar, that is: UE performs the multiple PRACH transmission of one RACH attempt within one selected RO group, while multiple PRACH transmissions across different RO groups is not supported. [China Telecom] propose to support the following steps for the transmission of multiple PRACHs:

- Step. 1: UE determine the number of PRACH transmissions based on network configuration.

- Step. 2: UE determine the RO groups based on network configuration which satisfies the following two conditions: first, the number of ROs of the RO group is the same as the number of PRACH transmissions determined in Step. 1; second, the ROs within the RO group are associated with the same SSB as the SSB selects by the UE during cell search.

- Step. 3: UE select one RO group from the determined RO groups in Step. 2 to transmit multiple PRACH within the RO group.

Moreover, companies [vivo, Spreadtrum, Nokia, China Telecom, Panasonic, Mavenir, Qualcomm] discuss about UE behavior when some RO within the selected RO group is dropped due to e.g., dynamic SFI indication, result in that the number of actual PRACH transmissions is less than the number of configured one. Companies [vivo, Spreadtrum, Nokia, China Telecom, Panasonic, Mavenir, Qualcomm] propose that for the above case, UE then drop the PRACH transmission on this RO and doesn’t extend the PRACH transmission to the RO in another RO group, in other word, the number of multiple PRACH transmission is counted in valid ROs.

Regarding the collision issue, companies’ views are summarized as follows:

- [vivo] PRACH repetition in separate RO is not transmitted when the separate RO collides with Msg A PUSCH.

- [ZTE, Samsung, CATT] Collision between ROs for multiple PRACH transmissions and other PRACH transmission could be solved by implementation, in other words, it is not necessary to address the collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features. [ZTE] proposes that shifting the collided RO with a time domain or frequency domain offset is proposed to solve the collision of ROs as shown in the following figure.

- [Nokia] If RAN1 agrees to introducing collision rules between valid ROs for multiple PRACH transmissions and other existing ROs, applicability of such rules should be up to cell-specific configuration by gNB.

- [Panasonic] For a separate RO, support to determine it as a valid RO as follows:

- For paired spectrum or supplementary uplink band (except a HD-UE), a separate RO can be created as the same way as the shared RO such that all separate ROs are valid ROs.

- For paired spectrum of a HD-UE, a separate RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213.

- For unpaired spectrum a separate RO is allocated in UL slots as specified in Rel. 15/16 for the shared RO.

  - Different frequency-domain resource is used for the separate RO, compared to a shared RO.

  - A separate RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213.

- [Huawei] Some rules to determine the validity of resource over overlapping ROs should be supported when reuse Rel-17 framework of feature combination and additional RACH configuration to realize the PRACH resource partitioning of transmitting multiple PRACH on separate RO. For example, the maximum number of dropped ROs in RO groups should be specified, where the RO groups containing more dropped ROs than maximum should not be used.

- Whether the ROs can be shared between different RO groups for different numbers of multiple PRACH transmissions or for the same number of multiple PRACH transmissions

Based on the contributions, companies’ views are summarized as follows:

- ROs can be shared between different RO groups for different numbers of multiple PRACH transmissions, while separate preambles are used to differentiate the PRACH transmissions over different RO groups.
Support: Huawei, HiSilicon, OPPO, Intel, Nokia, NSB, China Telecom, Mavenir, Lenovo, LG, Sony(?)
Not Support: ZTE, Fujitsu, NTT DOCOMO, ETRI

- ROs can be shared between different RO groups for the same number of multiple PRACH transmissions, while separate preambles are used to differentiate the PRACH transmissions over different RO groups.
Support: Nokia, NSB
Not support: Fujitsu, NTT DOCOMO, China Telecom

- Other companies’ views are summarized as follows:

- [Sony] Multiple PRACH transmissions with different numbers of PRACHs per transmission share the same RO Group. The gNB blind decodes for the number of multiple PRACH transmission selected by the UE.

- [Panasonic] For CBRA and CFRA, support the following resource configuration for the multi-PRACH transmission: Dedicated n-th PRACH transmission resource: A specific PRACH resource is used for a specific number of the PRACH transmissions. Shared n-th PRACH transmission resource: A PRACH resource can be used for all possible numbers of PRACH transmissions.

- [Lenovo] To reduce PRACH transmission latency, study to use dense starting ROs of RO groups for PRACH repetition, where the periodicity of the starting ROs of the RO groups in terms of number of ROs is smaller than the number of ROs in the RO groups as shown in the following figure.

- [Samsung]RAN1 considers the following alternatives to differentiate multiple N values: Alt. 1: separate preamble with shared RO; Alt.2: separate RO; Alt. 3: FDMed RO (in same PRACH configuration)

- The time span of RO group

Companies [vivo, OPPO, Apple, Ericsson] propose that time duration of RO group for PRACH repetitions can span SSB-to-RO association period, while [vivo, Apple] think the time span should be no larger than one SSB-to-RO association pattern period.

[Nokia] thinks the time span of RO groups is a result of RO groups configuration by network and is not subject explicit restrictions.

[CATT] propose to down-select from the following options to determine the starting position and the time span of an RO group: Option 1: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from SFN #0; Option 2: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from the first system frame every 160ms. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions. And further discuss on whether the association period and association pattern period are needed for multiple PRACH transmissions.

[Samsung]RAN1 considers the following alternatives to ensure the time span of the RO group to be kept for a reasonable time duration: Alt. 1: leave to gNB configuration; Alt.2: the time span of one RO group is required not to exceed a certain time duration threshold

Issue #3: Same or different preamble(s) during multiple PRACH transmission

The following agreement was achieved in RAN1 110b-e:

Agreement - For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Regarding the FFS parts, based on the contributions since RAN1 #111, companies’ views are summarized as follows:

Companies [Huawei, vivo, TCL, Fujitsu, CATT, Xiaomi, Intel, Lenovo, CMCC, Mavenir, Panasonic, Samsung] propose not to support/ deprioritized utilize different preambles during the multiple PRACH transmissions in one attempt due to the following reasons: there is no additional performance gain and non-coherent combining gain cannot be obtained; the receiver complexity increases when combining PRACH repetitions using multiple preambles; higher standards impact; complicated gNB and UE behavior.

While [ZTE] proposes to consider different preambles during the multiple PRACH transmissions in one attempt in order to reduce or randomize the preamble collision with other UEs and reduce the interference.

Issue #4: SSB-to-RO mapping

Based on the contributions, companies [Nokia, Huawei, ZTE, Intel, OPPO, Spreadtrum, LG, China Telecom, Panasonic, Lenovo, Quectel, Huawei, CATT, Qualcomm, vivo] discuss the SSB-to-RO mapping for multiple PRACH transmsisions. For legacy SSB-to-RO mapping, SS/PBCH block indexes provided by ssb-PositionsInBurst are mapped to valid PRACH occasions in the following order: preamble indexes, FDMed RO indexes, TDMed RO indexes. The current mapping order leads to long latency between TDMed ROs for multiple PRACH transmission, since TDMed RO indexes has the lowest mapping priority. As Nokia point out: although via proper configuration, gNB is able to create multiple beams in different directions in a same time instance, however, for FR2 where gNBs typically operate with analog/hybrid beamforming and are only able to generate a limited set of beams in different directions in a same time instance and wherein the maximum number of SSB beams is equal to 64. Thus, this kind of configuration has its limiations.

Detailed companies views are summarized as follows:

Companies [Panasonic, Intel] propose to reuse the legacy mechanism for SSB-to-RO mapping for the multi-PRACH transmission. [Spreadtrum] proposes that for multiple PRACH transmission with separate preamble on shared ROs, support following the legacy mechanism between SSB and RO mapping.

Companies [Spreadtrum, vivo, LG, ZTE, Lenovo, Quectel] propose to consider SSB-to-RO group/set mapping, which replace the RO with a group/set of ROs. The mapping order between SSB and RO groups can follow the current rules. For example, as illustrated by [LG] in the following figure.

Companies [OPPO, China Telecom, Nokia] propose to consider SSB-to-RO mapping enhancement to allow more consecutive TDMed ROs for multiple PRACH transmission if separate/new ROs are configured, e.g., SSB are mapped to valid ROs for multiple PRACH transmission first in the time domain and then in the frequency domain.

[Huawei] SSB beams should be classified into different coverage enhancement levels, where SSB beams belonging to the same level are associated with the same number of ROs for repetition but SSB beams belonging to different levels are associated with different number of ROs for repetition. The SSB-RO pattern in which different SSBs associate with different maximum repetition level should be supported by RRC configuration.

[Qualcomm] propose that the counting of PRACH repetitions is based on the valid ROs. Moreover, [Qualcomm] propose that PRACH repetitions are only transmitted in the valid ROs associated with the same SSB at different time with the following order:

- First, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot.

- Second, in increasing order of indexes for PRACH slots.

- Third, in increasing order of indexes for PRACH association period.

[CATT] For multiple PRACH transmissions with same Tx beam on separate ROs, new SSB-to-RO mapping pattern is defined from one of the following options.

Option 1: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from SFN #0. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain.

Option 2: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from the first system frame every 160ms. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions.

2.1.2 RAR window and RA-RNTI calculation

Issue #5: RAR window and RA-RNTI

The following agreement was achieved in RAN1 112 for RAR monitoring:

Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

- Issue #5-1: RAR window

Based on the contribution, there are mainly two options for the start position of the RAR window as shown in the following figures.

Illustration of Option 1

Illustration of Option 2

Companies’ views for the start position of RAR window are summarized as follows:

- Option 1: the starting point of RAR window is after the last symbol of the last PRACH occasion corresponding to the multiple PRACH transmissions.

- Support (26): OPPO, Spreadtrum, ZTE, vivo, CATT, Intel, InterDigital, LG, MediaTek, NEC, Nokia, NSB, Xiaomi, China Telecom, Samsung, Huawei, HiSilicon, Panasonic, Fujitsu, Mavenir, Lenovo, CMCC, Apple, Sharp, NTT DOCOMO, ETRI

- Option 2: the start position of the RAR window is after the last symbol of the first PRACH occasion corresponding to the multiple PRACH transmissions.

- Support (5): Panasonic, ZTE, TCL, Lenovo, Quectel

Other companies’ considerations are summarized as follows:

- [Sony] The RAR window is located after the end of the RO Group, i.e. after the ROs for the highest configured number of multiple PRACH transmissions has passed.

- [Panasonic] Support to configure one of two schemes (after the first RO and after the last RO) for RAR window and RA-RNTI calculation in a semi-static manner.

- [Lenovo] For the case that the start position of the RAR window is reference to the first PRACH transmission, further study how to determine the length of the window. When there are multiple ROs used for PRACH transmissions, the timing of the UE starts the monitoring PDCCH for BFR should also be decided. Same method as RAR could be considered.

- [NEC] If RAR window longer than 10ms is supported for PRACH repetition, e.g., for shared spectrum PRACH repetition, UE determines LSBs of a SFN field by using the last PRACH repetition.

- Issue #5-2: RA-RNTI

According to current spec. TS 38.321, RA-RNTI is calculated as follows:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

For multiple PRACH transmissions, the RA-RNTI calculation is related to RAR window design. Based on the companies’ contributions, there are mainly two options proposed for RA-RNTI calculation as follows:

- Option 1: Multiple RA-RNTI candidates within one RAR window, i.e., UE attempts to detect a DCI format 1_0 with CRC scrambled by one of the multiple RA-RNTI candidates during a RAR window.

- Option 2: Single RA-RNTI within one RAR window, i.e., UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a RAA window.

  - Option 2-1: The corresponding RA-RNTI is calculated based on RO for the last PRACH repetition.

Support (16): CATT, vivo, Spreadtrum, ZTE, Intel, Panasonic (if RAR window starts after the last RO), Fujitsu, Xiaomi, China Telecom, Mavenir, Applem, NEC, Sharp, NTT DOCOMO, ETRI, LG

  - Option 2-2: The corresponding RA-RNTI is calculated based on RO for the first PRACH repetitions.

Support (11): CATT, Spreadtrum, ZTE, TCL, Panasonic (if RAR window starts after the first RO), Fujitsu, Xiaomi, Mavenir, Lenovo, Quectel, NTT DOCOMO

  - Option 2-3: The corresponding RA-RNTI is calculated as a function of the RO group used for the multiple PRACH transmissions.

Support (2): Nokia, NSB

Meantime, as some company points out, it needs to be clarified that whether the “RO” in the above Options means the configured one (consider the last or first PRACH repetition is dropped).

Other companies’ considerations are summarized as follows:

- [ZTE] UE can assume that multiple RA-RNTIs are calculated based on anyone of multiple ROs for the PRACH repetitions if RA-RNTI calculation cannot be decided to base on a predefined single RO.

- [Nokia] For the RA-RNTI calculation based on one RO of the RO group used for transmission of the PRACH repetitions. It has the drawback that gNB would not be able to disambiguate two UEs transmitting on two RO groups sharing the RO to be used for the calculation of the RA-RNTI, especially if the two UEs are using the same preamble. Moreover, this approach would expose gNB to ambiguities in case the RO used to calculate the RA-RNTI is dropped by the UE due to some collisions.

- [Samsung] The RA-RNTI calculation in legacy is not intended to separate them in RO, which means, as long as the UE, for whatever feature, selects the same RO, they will have same calculated RA-RNTI, and they will detect the same PDDCH no matter gNB detects whichever preamble in that RO. It not only consuming UE’s time and resource to detect and decode the PDDCH which is never intended for it, but in the given UE handling for current procedure, it could suspend the RAR monitoring in the window after the PDCCH-PDSCH decoding if RAPID is not matched. Thus, support to separate UE from selecting same RO but for different N value by RA-RNTI.

2.1.3 Determine the number of multiple PRACH transmissions

Issue #6: Determination of the number of multiple PRACH transmissions

In RAN1 #111, the following agreement was achieved for determination of the number of multiple PRACH transmissions:

Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

- Issue #9-1: Other factors other than SSB-RSRP threshold(s) for determination the number of multiple PRACH transmissions

Based on the contributions, companies think some other factors than SSB-RSRP needs to be considered when determine the number of PRACH transmissions, as summarized below:

Factor 1: The number of single PRACH attempts exceeds a threshold or the calculated power of single PRACH reaches the maximum output power of UE.

Support: ZTE, Nokia, OPPO, vivo, Qualcomm

Factor 2: UE power class.

Support: Qualcomm

Factor 3: UE power headroom.

Support: Ericsson

Factor 4: MPE or P-MPR.

Support: Fujitsu, Samsung

Companies [CATT, Xiaomi, China Telecom, vivo, Spreadtrum, ZTE, InterDigital, Apple, NEC, LG, Intel, Panasonic, Lenovo, MediaTek, NTT DOCOMO, ETRI] propose that the SSB-RSRP thresholds should be associated with the values configured for the number of multiple PRACH transmissions, which indicates SSB-RSRP threshold is configured per the value of multiple PRACH transmissions configured by the gNB. Furthermore, companies [CATT, Xiaomi, China Telecom, NTT DOCOMO] propose that for multiple PRACH transmissions with same beam, at least for the first RACH attempt, only SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions for CBRA.

Companies [CATT, NTT DOCOMO, Spreadtrum, FGI, Panasonic, NTT DOCOMO] proposes that different methods are applied for CBRA and CFRA, respectively. UE determination of the number of PRACH transmissions by at least SSB-RSRP threshold(s) only applies to CBRA. For CFRA, the number of PRACH transmission(s) is determined and indicated by gNB, e.g., via PDCCH order.

Regarding whether the SSB-RSRP threshold(s) can be shared with existing threshold, companies’ views are summarized as follows:

- [CATT] SSB-RSRP threshold(s) for determination of number of PRACH transmissions are separately configured from the SSB-RSRP threshold(s) for SSB selection and that for Msg3 repetition request.

- [OPPO, Apple, Intel] If a single PRACH repetition number is configured, RSRP threshold defined for Msg3 repetition request is re-used to determine the PRACH repetition.

Other companies’ considerations are summarized as follows:

- [Nokia] Determination by the UE of the number of multiple PRACH transmissions using at least SSB-RSRP threshold(s), is subject to the UE output power being above a certain value, e.g., based on UE’s maximum power. RAN1 to investigate aspects of UE selection of the SSB index based on expected UL link budget calculated as a function of e.g., SSB-RSRP measurements and expected link gain corresponding to the number of PRACH transmissions, wherein the gain from the multiple PRACH transmissions is integrated in the evaluation of the expected UL link budget as 10*log10(number_PRACH_transmissions). If separate preambles on shared ROs are used to differentiate the multiple PRACH transmissions with single PRACH transmission, derive the SSB-RSRP thresholds for PRACH repetitions from the Msg3 RSRP threshold. Details of the configuration and derivation can be left to RAN2.

- [Samsung] Further study multiple PRACH transmission enhancements when UE experiences MPE issues, e.g., impact of MPE on: number of multiple PRACH transmission, power settings, the trigger for multiple PRACH transmission, and SSB selection for PRACH association.

- [Fujitsu] The discussion on the condition for the P-MPR application to PRACH transmission is needed before investigating the impact on the MPE.

2.1.4 Power control

Issue #7: Power calculation

Companies [Huawei, ZTE, China Telecom] discuss the power calculation for multiple PRACH transmissions. Companies [Huawei, China Telecom] propose that the power control of multiple transmission specified in eMTC PRACH coverage enhancement can be starting point as follows:

Step 1: Calculate the total receive target power.

PREAMBLE_RECEIVED_TARGET_POWER=preambleInitialReceivedTargetPower+DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep

Step 2: Calculate the target receive power for one single transmission.

PREAMBLE_RECEIVED_TARGET_POWER=PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)

Step 3: Calculate the transmission power for one single transmission.

P_PRACH = min {P_CMAX, PREAMBLE_RECEIVED_TARGET_POWER + PL_c} [dBm]

- [Intel, Samsung, NTT DOCOMO, LG] Same transmit power is applied for multiple PRACH transmissions within one RACH attempt, in other word, The same measurement of the same reference signal to calculate the pathloss should be applied for each of the Multiple PRACH transmissions in one attempt.

- [ZTE] proposes that an additional power offset for multiple PRACH transmissions should be considered for PREAMBLE_RECEIVED_TARGET_POWER calculation.

- [Samsung] The multiple PRACH transmission use the same set of P0 and alpha configuration from single PRACH transmission.

- [NEC] A separate target power and/or power ramping step and/or maximum number of transmissions are configured for PRACH repetition. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5 [TS 38.213], or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1 [TS 38.213], down-select from

  - Option 1: the UE does not transmit all of PRACH attempts in a transmission occasion for PRACH repetition comprising of number of PRACH attempts, Layer 1 notifies higher layers to suspend the corresponding power ramping counter.

  - Option 2: the UE does not transmit any of a PRACH attempt in a transmission occasion for PRACH repetition comprising of number of PRACH attempts, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

- If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5 [TS 38.213], or due to power allocation in EN-DC or NE-DC or NR-DC operation,

  - the UE transmits any of a PRACH attempt with reduced power in a transmission occasion for PRACH repetition comprising of number of PRACH attempts, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

2.1.5 Retransmission of multiple PRACH transmissions

Issue #8: Retransmission of multiple PRACH transmissions

- Based on the contributions, companies discuss about the determination of the number of PRACH transmission(s). The potential issues include: the relation between the number of PRACH transmission in Re-attempts and that in first attempt; the relation between power ramping and increasing the number of PRACH transmissions. Detailed companies’ views are summarized as follows.

- Issue #8-1: The number of PRACH transmissions in RACH Re-attempts and power ramping

- Based on companies’ contributions, the main issues lies in the following aspects: whether the number of multiple PRACH transmission is increased in Re-attempts; whether power ramping is applied between RACH attempts for multiple PRACH transmissions, if applied, whether separate power ramping step is needed. Detailed companies’ views are summarized as follows:

- The number of PRACH transmissions in RACH Re-attempts

- [Huawei, China Telecom, CMCC, InterDigital, Sharp, Intel] proposes that the number of multiple PRACH transmission is increased in Re-attempts, but different conditions are considered by companies, including:

  - [Huawei, Sharp] when the transmission power of retransmission reaches the maximum transmission power.

  - [China Telecom] if SSB or CSI-RS selected is not changed from the selection in the previous multiple PRACH transmissions, the number of PRACH retransmissions in one RACH attempt is equal to or larger than that of the previous multiple PRACH transmissions. Else, the number of multiple PRACH retransmissions in one RACH attempt is determined in the same way as first RACH attempt.

  - [Intel] After a certain number of PRACH retransmissions for multiple PRACH transmissions on a given repetition level, and if UE still does not successfully receive RAR.

- [OPPO] proposes two options: Opt.1: Power ramping can be applied for the next attempt of multiple PRACH. The power of PRACH is ramped with the increase of multiple PRACH attempt; Opt.2: Power ramping is not applied for the next attempt of multiple PRACH. The number of multiple PRACH is ramped with the increase of multiple PRACH attempt.

- [CATT, Fujitsu] propose that for subsequent RACH attempt(s), power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met.

- [Xiaomi] proposes to use the same number of PRACH transmissions for subsequent RACH attempts.

- [ZTE] If multiple PRACH transmissions is used in the initial attempt, the retransmission should use multiple PRACH transmissions too.

- [Nokia] Define SSB-RSRP exception zone to allow a UE to increase the number of PRACH transmissions in case of PRACH re-attempt. Define a procedure for increasing the number of the multiple PRACH transmissions at different RACH attempts based on adapting the value of the measured SSB-RSRP.

- Power ramping

- [Huawei, NEC] propose to configure consider a separate power ramping step for multiple PRACH transmission in Re-attempts.

- [LG, Intel] If re-transmission may be required, the transmission power ramping can be applied between RACH attempts.

- [ETRI] The transmission power level and ramping step size can be affected by the repetition factor.

- [Xiaomi] Consider how to perform power ramping for multiple PRACH transmissions when one or several transmissions omission or power reduction occurs.

- Issue #8-2: The maximum number of retransmissions

- Companies [Huawei, China Telecom, NEC] propose that the maximum retransmission time of large repetition levels should be reduced to deal with the harmful retransmission, which indicates a separate maximum number of RACH attempts for multiple PRACH transmissions is configured.

- [Samsung] to constraint the total number of multiple PRACH transmission, considering following to alternatives:

- At.1: reuse preamble transmission counter by adding on the multiple transmission number for each attempt;

- Alt.2: introduce the PRACH attempt counter and adding one for each attempt.

2.1.6 Others

- Timing of UE starts the monitoring PDCCH for BFR

[Lenovo] When there are multiple ROs used for PRACH transmissions, the timing of the UE starts the monitoring PDCCH for BFR should also be decided. Same method as RAR could be considered.

- SSB/CSI-RS selection

[CATT] For multiple PRACH transmissions with same beam, at least for the first RACH attempt in a RACH procedure, SSB/CSI-RS is selected based on SSB/CSI-RS RSRP thresholds as in existing specifications. Further study whether to introduce separate RSRP thresholds for SSB/CSI-RS selection for PRACH repetitions from legacy PRACH transmission without repetition.

2.2 Multiple PRACH transmissions with different beams

In RAN1 #112, the following note about the potential gain of multiple PRACH transmissions with different beams is captured.

Note: It is summarized by FL that for the same number of PRACH transmissions per source, - 1 source [Ericsson] shows that: Multiple PRACH transmitted by beam sweeping, where a UE has no prior knowledge of channel and sweeps Tx beams across 360 degrees horizontally and 180 degrees vertically, outperforms multiple PRACH transmissions with the same Tx wide beam (omni direction) by at least 1 dB, provided gNB configures only one SSB and receives PRACH with a wide beam. - 3 sources [ZTE, Nokia, vivo] show that: A gain from about 1~3 dB of beam sweeping is observed if a UE is able to direct at least one of its Tx beams in the right direction or to narrow down the azimuth and/or zenith range of 360 degrees and/or 180 degrees for beam sweeping compared with multiple PRACH transmissions with the same Tx wide beam. - 1 source [Huawei] shows that: compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9~5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam. - 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams far apart is 3 dB worse than PRACH repetition with single best beam - 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam is 1dB worse than PRACH repetition with single best beam. - 1 source [vivo] shows that: PRACH repetition via random beam directions performs 1 dB worse than PRACH repetition with omni beam.

Based on the contributions, companies’ views about whether to support multiple PRACH transmissions with different beams are summarized as follows:

Companies [ZTE, Intel, TCL, Lenovo, Nokia, Ericsson, Samsung, NTT DOCOMO] propose to consider/support multiple PRACH transmissions with different beams, while [vivo] thinks PRACH repetition with different beams can be up to UE implementation and no specification work is needed on this in Rel-18 coverage enhancement work item. [Panasonic, LG, CMCC, China Telecom] prioritize to complete the basic design concept for multi-PRACH transmission with the same beam.

Other companies’ views are summarized as follows:

- [MediaTek] Multiple PRACH transmissions using different beams should be studied for the two following cases separately: A) For UEs without beamCorrespondence feature support, and B) For UEs with beamCorrespondence feature support.

- [Spreadtrum] Multiple PRACH transmissions with same beam can be used as a baseline for different beams.

- [CATT] The benefit and target scenario of multiple PRACH transmissions with different beams should be provided to justify the support of multi ple PRACH tra2nsmissions with different beams.

- [Panasonic] No need of the differentiation between same beam and different beams. The choice of beam can be up to UE implementation.

- [ETRI] If multiple beams are considered, study the need for a switching gap between consecutive PRACH transmission with different beams. Study the need for having a set of applicable Tx beams with given target gain.

If multiple PRACH transmission is supported, Companies [ZTE, Sharp] propose to consider PRACH resource partitioning between multiple PRACH transmissions with different beams and multiple PRACH transmissions with the same beam. Moreover, companies [ZTE, TCL, Sharp, Huawei] propose that the indication of best UL Tx beam based on the measurement of PRACH in each RO can be considered/supported.

2.3 Interaction between multiple PRACH transmissions and other transmissions

When multiple PRACH transmissions is enabled, it may have some interaction with other transmissions, e.g., Msg3 repetitions. The majority Companies think the coupling/interaction between PRACH repetitions, Msg.3 repetitions (and PUCCH repetitions for HARQ-ACK of Msg4) should be investigated.

- [ZTE, Panasonic, Qualcomm, CATT] The coupling between PRACH repetitions and Msg3 repetitions should be investigated. E.g., Msg3 repetition is mandatory if multiple PRACH transmissions is applied.

Besides, [Ericsson] has the following observations based on link-level simulation and propose to study how Msg3 performance can be improved by PRACH transmissions with different beams:

- In FR2, the required SNR for Msg3 with 8 repetitions and inter-slot frequency hopping at 10% BLER is 1.7 dB higher than that of a single PRACH transmission with a wide beam and 8 dB higher than a single PRACH transmission with the best beam for 1% missed detection. The gap could be 4.5 dB more for 10% mis-detection rate.

- With Rel-18 PRACH enhancement, the performance gap between Msg1 and Msg3 would grow. Msg3 needs further enhancement to be on par with Rel-18 PRACH.

- [OPPO] For legacy Msg3 transmission, the power is calculated based on the total preamble power ramping of performed PRACH transmission. When multiple PRACH transmissions is applied, how to calculate the total preamble power ramping during multiple PRACH transmissions attempts should be studied.

- [Panasonic] When multi-PRACH transmission is triggered, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

- [vivo] The combination of PRACH repetition and Msg3 repetition should be supported to achieve a robust random access procedure for coverage-constraint UEs.

2.4 CBRA and CFRA

Based on the contributions, companies [Spreadtrum, ZTE, vivo, Intel, NTT DOCOMO, Ericsson, Qualcomm, Huawei, HiSilicon(?)] propose to support multiple PRACH transmissions for both CBRA and CFRA. For CFRA, applying multiple PRACH transmissions can improve PRACH detection rate in SNR limited scenarios, which is essential to the cases of handover and beam failure recovery. Moreover, for CFRA, it is more flexible for network to configure the PRACH resources for PRACH repetition as dedicated signalling can be applied.

2.5 RRC parameters

Based on companies’ contributions [China Telecom, LG, Nokia, Xiaomi], the following RRC parameters for multiple PRACH transmissions can be considered:

- The values of multiple PRACH transmission configured by gNB (including {2, 4, 8}.

- The SSB-RSRP thresholds to determine the number of multiple PRACH transmissions.

- RO group related configurations (if RO group is supported to be specified).

2.6 Others

- RACH Procedure

[Samsung] The multiple PRACH transmissions is kept as one RACH procedure

- Frequency domain allocation of PRACH preamble

[Nokia] RAN1 to investigate mechanisms for transmission of subsets of the frequency representation of the PRACH preamble in ROs located at different time instances. For example, the mechanism is based on splitting the PRACH samples in frequency domain into two subsets, the two subsets being transmitted in different ROs in time domain. As shown below, UE1 and UE2 are transmitting the two subsets in two different portions of two different ROs, creating a sort of frequency hopping behavior within two TDMed ROs centred on the same frequency location.

- SUL

[ZTE] The multiple PRACH transmissions with same beam on SUL should be supported. Whether SUL carrier or NUL carrier will be selected firstly, and then whether to repeat PRACH is determined.

- SSB selection

[NEC] If none of SS-RSRP is above rsrp-ThresholdSSB (Rel-15 configuration used for SSB selection) and at least an SSB with SS-RSRP is above rsrp-ThresholdSSB-repetition (configuration value for PRACH repetition), select an SSB above rsrp-ThresholdSSB-repetition for PRACH repetition, else select any SSB for PRACH repetition.

- Email discussion (1st round)

3.1 Multiple PRACH transmissions with same beam

3.1.1 Resource configuration for multiple PRACH transmissions

Issue #1 Differentiation between single PRACH transmission and multiple PRACH transmissions

Proposal 1-1

Confirm the following working assumptions.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

- Confirm both working assumptions:

Support (13): CATT, ZTE, Intel, Nokia, NSB, Panasonic, Lenovo, Quectel, CMCC, Apple, LG, Ericsson, Qualcomm(?), Spreadtrum

- Confirm the working assumption with respect to “…support that multiple PRACH are transmitted on separate ROs…”:

Support (6): Spreadtrum, vivo, TCL, Fujitsu, China Telecom, NTT DOCOMO

- Confirm the working assumption with respect to “…support that multiple PRACH are transmitted with separate preamble on shared ROs…”:

Support (1): MediaTek

| Companies | Comments |
| --- | --- |
| CATT | Support |
| Panasonic | We support to confirm these two working assumptions. |
| LG | Support confirming both working assumptions |
| Apple | We support Proposal 1-1 |
| DOCOMO | OK to confirm both working assumptions if majority companies prefer this way, though our preference is to only configured the working assumption of separate ROs. |
| Lenovo | Support to confirm both the working assumptions. |
| Mavenir | Confirm both working assumptions. |
| ZTE | We support to confirm the two WFs. For the two WFs, we think gNB has the flexibility to configure one of them to match the different scenarios. No need to down select any on them. |
| TCL | Support |
| IDCC | Support |
| Spreadtrum | We support to confirm both the working assumptions. |
| ETRI | Support to conform both WAs. |
| Sharp | We are OK to confirm both WAs. |
| CMCC | Support. |
| Ericsson | Support to confirm both working assumptions |
| Fujitsu | Although we have some concerns on the shared ROs due to higher contention rate and lack of preambles, but we can accept to confirm both working assumptions if majority companies prefer. |
| Quectel | Support confirming both working assumptions |
| IDCC | Support |
| Sony | Support. |
| Qualcomm | Support |
| vivo | Considering the complexity of sharing ROs with legacy PRACH, we would prefer to prioritize working assumption for PRACH repetition on separate RO first. We can come back to discussing whether supporting shared RO case if time allows. |
| MediaTek | OK to support both WAs for progress. |
| Nokia/NSB | Support and agree with ZTE. |
| OPPO | Support |
| Intel | We support the proposal. |
| Samsung | Support both WA to be confirmed |
| Huawei, HiSilicon | Support |

Proposal 1-2

Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) is reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

| Companies | Comments |
| --- | --- |
| CATT | Support |
| Panasonic | We support the proposal 1-2. |
| LG | Support. Also, detail discussions are up to RAN2. |
| Apple | We support Proposal 1-2. |
| DOCOMO | Support. |
| Lenovo | Support this proposal 1-2. |
| Mavenir | Support |
| ZTE | We can agree this in RAN1 and send the LS to RAN2 for notification. |
| Spreadtrum | Support |
| ETRI | Support the proposal. |
| Sharp | We support the FL proposal 1-2. |
| CMCC | OK. Some additional parameter in Rel-18 could be further discussed. |
| Ericsson | Support in principle. Details are up to RAN2’s decision. |
| Fujitsu | Support |
| Quectel | Support. |
| Sony | Support. |
| Qualcomm | Support (details to be decided by RAN2) |
| vivo | We’re fine with this proposal in principle. However, when separate ROs are configured for PRACH repetition, additional signalling may also be needed to introduce additional ROs to reduce legacy as we proposed in our contribution. To move forward, we would propose to add an FFS bullet as following. Proposal 1-2 Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) is reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. - FFS whether additional signaling is needed to configure additional ROs to reduce the latency of PRACH repetition. |
| MediaTek | Support. |
| Nokia/NSB | Given that this is a RAN2 detail, at least according to our understanding, wouldn’t it be better to express a RAN1 understanding instead? What if RAN2 discovers that some incompatibility exists between the feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) framework and what will have to be done for supporting PRACH repetitions? Something like this seems more suitable to us: Proposal 1-2-v1 It is RAN1 understanding that at least Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be is reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning, and that final decision on how to realize PRACH resource partitioning is up to RAN2. |
| OPPO | Support |
| Intel | We are fine with Nokia’s update. |
| Samsung | Support And revise a little based on Nokia’s version, “that final decision” changes to “the details” |
| New H3C | Support |
| Huawei, HiSilicon | In general, the Rel-17 framework is a starting point for Rel-18. However, we prefer not to make such agreement at this stage for the following reasons, - The proposal is about RRC parameters and thus not urged to be concluded at this stage. It seems that no other key discussion point relies on this proposal. - There are many on-going discussion points that may result in new signalling based on the Rel-17 framework. It is too early to claim “is reused” or “can be reused”. For example, it is unclear yet how to handle the potential collision between legacy PRACH resources and R18 PRACH resources that are mapped to different SSB at the same symbols. It is also unclear whether additional signalling is needed to facilitate configurations of low latency of PRACH repetition. Therefore, more room should be reserved for any potential RRC signalling resulted by unsolved discussion points. In order to avoid discussions on new potential signalling, we suggest to postpone this proposal. Or a simple conclusion is sufficient that “Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) is assumed to be a baseline for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.” |

Discussion about separate RO configuration

FL comment: Besides reusing corresponding Rel-17 framework to realize corresponding PRACH resource partitioning, companies also propose to: configure separate ROs based on legacy PRACH configuration with additional new parameters; or configure separate ROs by individual parameters (including the T/F resource, repetition number, etc.); or configure additional RO resources relative to the separately configured ROs.

Companies are encouraged to provide your comments about whether other separate RO configuration methods are supported besides reusing corresponding Rel-17 framework.

| Companies | Comments |
| --- | --- |
| CATT | We do not think other separate RO configuration methods are needed in addition to Rel-17 framework. |
| Panasonic | Before any discussion on separate RO configuration, we should discuss and conclude time and frequency resources that can be used for separate RO. Our view for separate RO is that - For paired spectrum or supplementary uplink band (except a HD-UE), a separate RO can be created as the same way as the shared RO such that all separate ROs are valid ROs. - For paired spectrum of a HD-UE, a separate RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213 - For unpaired spectrum, there can be 2 alternatives for a separate RO. - Alt. 1: A separate RO is allocated in UL slots as specified in Rel. 15/16 for the shared RO. - Different frequency-domain resource is used for the separate RO, compared to a shared RO. - A separate RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213 - Alt. 2: A separate RO is allocated in a basis of available UL slots as specified for Rel. 17 PUSCH repetition type A - Same or different frequency-domain resources can be used for the separate RO, compared to a shared RO. - A separate RO can be validated as a valid RO based on Rel. 17 cancellation rule in addition to the existing validation rule according to Clause 8.1, TS 38.213. We think that Alt. 1 can be supported because Alt. 2 might be required to have more time to discussion in RAN1. Regarding the signalling for separate RO, we are open to discuss 3 options mentioned by FL - Option 1: Configure separate ROs based on legacy PRACH configuration with additional new parameters. - Option 2: Configure separate ROs by individual parameters (including the T/F resource, repetition number, etc.). - Option 3: Configure additional RO resources relative to the separately configured ROs For Option 3, the highlighted yellow part is unclear to us. Could you please clarify it @FL? |
| LG | We think that configuring additional RO resources (AdditionalRACH-Config-r17) is the simplest way to reuse the Rel-17 Feature Combination framework. |
| Lenovo | Not that clear the details of these proposals. But we prefer that the separate configured ROs can be shared by PRACH transmission with different repetition number. |
| Mavenir | It is preferred to configure separate ROs based on legacy PRACH configuration (r17) with additional new parameters. |
| ZTE | Actually the methods of above mentioned can all be referred to RAN2 for the detailed signalling design if proposal 1-2 is agreed. |
| TCL | We are open to discuss the RO resources for multiple PRACH transmission based on the following ways: - configure separate ROs based on legacy PRACH configuration with additional new parameters; - configure separate ROs by individual parameters (including the T/F resource, repetition number, etc.); - configure additional RO resources relative to the separately configured ROs |
| ETRI | We are open to discuss, and the third proposal (‘configure additional RO resources relative to the separately configured ROs’) can be more clarified. |
| CMCC | We are open to further discuss this solution, which seems more flexible. |
| Ericsson | Could FL please clarify that these options can’t be supported by Rel-17 RACH framework? If so, we would like to know what kind of new parameter is needed. |
| Sony | It isn’t clear what Option 3: Configure additional RO resources relative to the separately configured ROs. Please clarify. Also Option 1 and Option 2 may be the same: Option 1 says configure using legacy parameers + new parameers Option 2 says configure individually with time/freq resource and repetitions. Since legacy configuration have time/freq resource isn’t Option 2 just Option 1 with the new parameter being repetitions? |
| vivo | In our understanding, for separate RO configuration, additional parameters are always needed. These parameters may be different for different alternatives summarized by feature leader. In case separate ROs are only configured by existing Rel-17 framework, i.e., only based on the PRACH configuration table, following parameters are needed: - Number of PRACH repetitions; - SSB to RO mapping related parameters in case SSB to RO mapping is optimized (this is not preferred by us). In case separate ROs are configured by both existing Rel-17 framework and additional signaling to configure a set of ROs relative to separate ROs, following parameters are needed assuming the additional ROs are FDMed in the same way as the separately configured ROs based on Rel-17 frame work: - Slot level offset relative to e.g. a PRACH slot, - Number of ROs additionally configured in time domain. Our preference is to support separate RO configuration with additional ROs configured relative to the PRACH slot to not only reduce the PRACH repetition latency compared to PRACH retransmission, but also avoid the SSB to RO mapping optimization since all additional ROs would be mapped to the SSB mapped to the separate ROs configured by Rel-17 framework. |
| Nokia/NSB | Agree with ZTE. We think that agreeing on proposal 1-2, once its content will be stable, should be sufficient. |
| OPPO | We think configuring additional RO resources is clear and straightforward. It is not clear for other options what are the additional new parameters and additional RO resources. We can refer to RAN2 for the signalling of separate ROs. |
| Intel | Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) are sufficient for the configuration of multiple PRACH transmissions. |
| Samsung | Not needed. As the two WAs confirmed, additionally introducing RO which is not in the table causes problems. |

Issue #2 RO group

Proposal 2-1

FL comment: Considering that companies’ views about whether to support multiple PRACH transmissions located at same time instance is stable since RAN1 #111, no consensus can be achieved. The following conclusion is proposed:

Proposed conclusion:

There is no consensus to support multiple PRACH transmissions located at same time instance in Rel-18.

- Not support multiple PRACH transmissions located at same time instance (13): Huawei, Fujitsu, Xiaomi, Intel, ETRI, Lenovo, LG, MediaTek, Nokia, Samsung, Spreadtrum, ZTE, China Telecom

Support multiple PRACH transmissions located at same time instance (2): CMCC, Ericsson

| Companies | Comments |
| --- | --- |
| CATT | We support the proposed conclusion. |
| Panasonic | We are fine with the proposed conclusion. |
| LG | Agree. |
| Apple | We support the proposed conclusion. |
| DOCOMO | Fine with the proposed conclusion. |
| Lenovo | Support the proposed conclusion. |
| Mavenir | Fine with the proposed conclusion. |
| ZTE | Fine with the proposed conclusion. |
| TCL | Agree |
| Spreadtrum | Support the proposed conclusion. |
| ETRI | Support the conclusion. |
| Sharp | We support the proposed conclusion. |
| Ericsson | Thanks to FL for detailed summary of this issue in section 2.1.1. We copied related discussion in red below and added our comments in blue. In a summary, - 1.5dB performance gain of FDMed PRACH transmissions is not a small benefit to us. - Since gNB can avoid blindly detecting a multiplexing method by proper RACH configuration, there is no increase of gNB complexity for detecting FDMed PRACHs. - With half transmission power per PRACH on a Tx chain, the simultaneous PRACH transmissions with two Tx chains can achieve the full power transmission at a time instance. Therefore, we compared two FDMed PRACH transmissions with a single PRACH transmission, where the same total transmission power is assumed. The use of two FDM ROs can bring 1.5dB gain, with 1dB from spatial diversity of using the 2nd Tx chain and 0.5dB from frequency diversity gain. - The other discussions of RO group, e.g., proposal 2-2, 2-3, and 2-4, are common to TDMed and FDMed. Issue #2: RO group Based on the contributions, the majority companies further discuss about the definition, configuration of RO group for multiple PRACH transmissions. Based on current definition, RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. But it is not clear whether these ROs can be located in the same time instance and whether the starting RB of the ROs within one RO group can be different. Notice that these two issues are related to the fundamental design of RO group, the summary will start from these two issues. Then, it goes for the discussion about details of RO group. - [Ericsson] We don't see much difference between the configurations of a group of TDMed ROs and FDMed ROs by using the two working assumptions on separate ROs and shared ROs. An example is illustrated below. Anyway, it is up to RAN2 to decide how to configure RACH resources for TDMed and FDMed multiple PRACH transmissions of a RACH attempt. RO#1 Preamble index 10~19: two FDMed PRACHs. RO#0 Preamble index 10~19: two FDMed PRACHs; Preamble index 0~9: two TDMed PRACHs; RO#2 Preamble index 0~9: two TDMed PRACHs. - Issue #2-1: Whether/how multiple PRACH transmissions located in the same time instance - Companies [Huawei, Fujitsu, Xiaomi, Intel, ETRI, Lenovo, LG, MediaTek, Nokia, Samsung, Spreadtrum, ZTE, China Telecom] propose not to support/ deprioritize multiple PRACH transmissions located at same time instance due to the following reasons: small benefit and high complexity; there may be more than one resource multiplexing modes in this network, which will greatly increase the complexity of the non-coherent combining detection at gNB side; only half of the deliverable power is available for each Tx chain. - [Ericsson] We don’t think 1.5dB performance gain of FDMed PRACH transmission is a small benefit. - As to complexity, gNB doesn’t need to blindly determine a multiplexing mode, if separate RACH resources are configured for TDMed PRACHs and FDMed PRACHs. We would like to know why FDMed PRACH transmissions will greatly increase the complexity of the non-coherent combining detection at gNB side. - With half transmission power per Tx chain, the simultaneous PRACH transmissions with two Tx chains can achieve the full power transmission at a time instance. Therefore, we compared two FDMed PRACH transmissions with a single PRACH, as illustrated below. The use of two FDM ROs can bring 1.5dB ROs, with 1dB from using the 2nd Tx chain and 0.5dB from frequency diversity gain. |
| Fujitsu | We support the proposed conclusion. |
| Quectel | We are fine with the proposed conclusion. |
| Sony | Support |
| Qualcomm | Support the conclusion |
| vivo | Fine. |
| MediaTek | Support the conclusion. |
| Nokia/NSB | We agree with the spirit but think that the current wording of the proposal is ambiguous, since we have never defined what “multiple PRACH transmissions located at same time instance” are exactly. For instance, it could be understood that this also includes the case where two different UEs repeat PRACH over different ROs in the same time instance, but each UE using only TDMed ROs. We suggest rephrasing the conclusion as follows: Proposed conclusion: There is no consensus to support RO group configurations which result in FDMed ROs located in the same time instance to be part of the same RO group. |
| OPPO | Support |
| Intel | We are fine with the proposed conclusion. |
| Samsung | Fine with the proposal. |
| New H3C | Support |
| Huawei, HiSilicon | Support. Nokia’s proposal seems better. |

Proposal 2-2

RO group is specified for multiple PRACH transmissions. One or multiple RO group(s) is(are) configured for each configured number of multiple PRACH transmissions, and the number of valid ROs in one RO group is equal to the corresponding configured number of multiple PRACH transmissions.

| Companies | Comments |
| --- | --- |
| CATT | We do not think it needs to decide whether to specific RO group at this point given that the definition itself is not clear yet. In addition, is the intention of “One or multiple RO group(s) is(are) configured…” to say that RO group(s) are configured? At least, our current understanding is that ROs are configured and RO group(s) are derived based on the RO configuration and other parameters. |
| Panasonic | It seems companies have different understanding of fundamental definition of a RO group. We would like @FL to clarify the following points in this proposal to align understanding between companies. - Point 1: Are “valid ROs” in Proposal 2-2 referred to “possible ROs” that can be included in a RO group? - Point 2: Are “valid ROs” in Proposal 2-2 validated as ACTUAL “valid ROs” for actual PRACH transmissions due to any event of RO collision? - Point 3: In RAN1#112, it has been agreed that a RO group for a specific number of multiple PRACH transmissions. Regarding “one or multiple RO group(s) is(are) configured for each configured number of multiple PRACH transmissions”, does it mean that there could be more than one RO group for a configured number of multiple PRACH transmissions? For instance, for 2 PRACH transmissions, there could be RO group#1 {including RO#0 and RO#1) and RO group#2 {including RO#3 and RO#4}. We think Points 1 and 2 is directly related to how to count ROs in a RO group. If yes is answer for Points 1 and 2, we suggest FL to provide definitions to clarify “valid RO” in RAN1#112 agreement (copied in below) or in Proposal 2-2 as follows - “Valid ROs” in RAN1#112 agreement are referred to possible ROs that can be included in a RO group for a specific number of multiple PRACH transmissions. - “Valid ROs” in RAN1#112 agreement can be later validated as actual invalid RO(s) and actual valid RO(s), where actual invalid RO(s) are not used for actual PRACH transmission(s), while actual valid RO(s) are used for actual PRACH transmission(s). Agreement (RAN1#112) For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. - Note 1: All ROs in one RO group is associated with the same SSB(s). - Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - Note 3: whether/how to define “RO group” in specification will be discussed separately - Note 4: Valid RO(s) refers to what is defined in existing specification - FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. - FFS: the time span of RO group. - FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. - FFS: other details. |
| LG | We agree in principle. To be specific, we think the first sentence should be supported at this stage, and the rest parts can be discussed further. |
| Apple | To be clear, the multiple RO groups means the RO group repeated in time domain, not the different RO group pattern, is this understanding correct? |
| DOCOMO | We feel “one RO group” for a specific number of PRACH transmissions is not feasible? There always are multiple RO groups for a certain number of PRACH transmissions, e.g. at least considering different time spans (e.g. different mapping pattern periods). Our understanding is “RO groups” are determined based on configured ROs and configured/predefined parameters (e.g. possible starting time locations for RO groups for each repetition factor). Regarding “valid ROs”, we think the definition of “valid RO” can reuse legacy RO validation rule. Fine to clarify it first. |
| Lenovo | Not that clear the exact meaning of “one or multiple RO group(s) is (are) configured for each configured number of multiple PRACH repetitions”. Does it mean e.g., in the case there is only one system SSB, then there is “one RO group” for this SSB, and “multiple RO groups” is for the case that the system supports multiple SSBs, and each RO group is for one SSB? |
| Mavenir | Support most part of the proposal, only with the modification “…,the number of valid ROs in one RO group is equal to the corresponding configured maximum number of multiple PRACH transmissions” |
| ZTE | Generally fine with the proposal, but the definition of valid ROs should be clarified. Does the definition of valid RO follow the legacy R15? How to handle the RO if it collides with the dynamic SFI? |
| TCL | We agree in principle. To be clear, the ‘One or multiple RO group(s) is (are) configured for each configured number of multiple PRACH transmissions’ means the RO group(s) is (are) configured by gNB? Then UE choose a corresponding RO group based on the number of PRACH repetitions, is this understanding correct? |
| Spreadtrum | “One or multiple RO group(s) is(are) configured for each configured number of multiple PRACH transmissions” is not clear for us. We would greatly appreciate it if @FL could clarify this understanding. - One understanding is each type of RO group is determined by each configured number of multiple PRACH transmissions. For example, three candidate transmission values of {2, 4, 8} were configured by gNB, then three types of RO group are determined by three configured number of multiple PRACH transmissions. - The other understanding is for each configured number of {2, 4, 8}, there is(are) only one or multiple RO group(s) can be determined for multiple PRACH transmissions. We are fine to support the first understanding with the modification “…. One or multiple types of RO group(s) is(are) configured determined for each configured number of multiple PRACH transmissions,…” |
| ETRI | We support the proposal in principle. In our understanding, we can discuss further whether RO group may or may not be configured and in the latter case RO group may derived by some rule. We tend to agree Panasonics’ clarifications. |
| CMCC | Fine. |
| FL | @all, Some quick clarification. The main intention of this proposal is to clarify the fundamental definition of what RO group is. It includes two aspects: Firstly, for each configured number of multiple PRACH transmission, there is one or multiple RO groups, which means, RO group is per the configured number of multiple PRACH transmission. Second, one RO group for a configured number of multiple PRACH transmissions, the number of valid ROs within the RO group is the same as the number of configured multiple PRACH transmissions. In addition, collision handling can be dealt with separately. From FL understanding, the definition of valid RO follows 38.213 Section 8.1 as follows. |
| Ericsson | We would like to know if legacy RO validation rules and SSB-RO mapping rules are reused or not. |
| Quectel | Generally agree. In our understanding, the definition of valid RO should keep in line with legacy R15, and RO group can be derived by some rules as like as the RO determination in legacy R15. |
| Sony | I believe the “one or multiple RO groups” is required since: - For each repetition, we need different RO groups for different SSB. For example if we have {2 ,4, 8} repetitions, and we have 4 SSB, then we need at least 12 RO groups where for each repetition we have 4 RO groups for each SSB. - For each repetition, even for the same SSB, the gNB should be able to configure 2 more more RO groups. For example for 2x repetitions, gNB may configure two distinct RO groups to provide more RACH opportunities. If the above is the correct understanding then we can support this proposal (perhaps we should make it clear since there are so many question regarding the “one or more RO Group”) |
| Qualcomm | Fine in general |
| vivo | Regarding whether RO group term needs to be specified, it depends on the resource configuration discussions and RRC parameter discussions, therefore this proposal seems not necessary at this stage and it can be treated in a late stage in our view. |
| MediaTek | Thanks for FL’s clarification. We are OK with the proposal in spirit. Based on this proposal, will it be possible to configure multiple RO groups on some overlapping ROs where each RO group contains different numbers of ROs? In the last RAN1-112, we made the following agreement below about PRACH resource differentiation between multiple PRACH transmissions with different numbers of transmission numbers. So, one way to differentiate is by assigning different preambles on overlapping ROs. If this is still possible, we are fine with the proposal. Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details. |
| Nokia/NSB | Agree with the proposal. @TCL: Our understanding is that the proposal implies that for each configured cell-specific repetition number, e.g., 2 and 4, NW can configure more than one RO group if NW wants (say two groups for 2 repetitions and one group for 4 repetitions). One of the advantages of this approach could be, for example, to separate UEs repeating PRACH twice in the two different groups randomly (i.e., round robin selection at the UE). This would allow NW to reserve a lower number of preambles for the repetitions. Of course, NW could also be able to configure only one RO group per repetition number. |
| OPPO | We share the same understanding with FL’s summary. The proposal clarifies the definition of RO group. We can further discuss how to configure the RO groups, ome potential aspects include: - RO grouping rule - SSB-RO mapping rule - Time span of RO group - RO sharing among RO groups |
| Intel | It is not clear to us whether RO group needs to be defined in the specification. UE may simply determine the multiple ROs for multiple PRACH transmissions. We suggest to defer the discussion once the direction on how RO group is determined is clear. In addition, it would be good to clarify whether in case of multiple RO groups for a specific value, are these RO groups FDM’ed or TDM’ed, e.g., within association pattern period or across association pattern period? |
| Samsung | Support the proposal in principle. Change “One or multiple RO group(s) is(are) configured for” “One or multiple RO group(s) is(are) derived/determined for”; |
| New H3C | Fine in general |
| Sharp | We support with the FL proposal based on the FL clarification. We understand “valid RO” is based on the current 38.213 description. |

Discussion about how an RO group is defined and configured

FL comment: Similar to the design of SSB-to-RO mapping, where an association pattern period is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec, as some companies point out, an RO group pattern can be associated with the SSB-to-RO association pattern period. An RO group pattern consists of RO group(s) for each configured number of multiple PRACH transmissions. In this way, the RO group pattern can repeat with the SSB-to-RO association pattern period, thus we only need to focus on the design of RO group(s) within the first SSB-to-RO association pattern period, which start from system frame 0. Companies are encouraged to provide comments on the following aspects.

Issue-1. Whether to associate RO group pattern with the SSB-to-RO association pattern period, so that we only need to focus on the RO group design within one SSB-to-RO association pattern period.

Issue-2. If multiple PRACH transmission with frequency hopping is supported, whether an RO group can be determined based on the following parameters: {time and frequency start position (or start RO), frequency offset of second hop in unit of RO, the number of ROs within the RO group}, where frequency offset in unit of RO can be 0, which indicates the RO group consist of ROs with the same frequency position. If multiple PRACH transmission with frequency hopping is not supported, whether an RO group can be determined based on the following parameters: {time and frequency start position in unit of one RO, the number of ROs within the RO group}.

Assume the number of ROs within the RO group is K. Then, from the start position, the first K ROs which associates with the corresponding SSB and satisfies the frequency offset (if frequency hopping is supported) consist of an RO group.

As illustrated in the following figure (square with different color indicates the ROs are associated with different SSB), take msg1-FDM=4, configured number of multiple PRACH transmission =4, no frequency hopping as an example.

As illustrated in the following figure, take msg1-FDM=1, configured number of multiple PRACH transmission =4, no frequency hopping as an example. Notice that if there are some ROs left but the number is less than required ROs to construct an RO group for a configured value of multiple PRACH transmissions. Whether these ROs can be used for multiple PRACH transmissions depends on the outcome of discussion on the first aspect.

Issue-3. Whether/how multiple PRACH transmissions with frequency hopping is supported? Although many companies propose to support PRACH frequency hopping, the detailed design is not discussed extensively, e.g., whether the hopping is within the configured ROs, which indicates frequency offset and mod operation may be needed to determine the hopping position. Meantime, this issue is also related to the design of RO group. Thus, FL suggest companies to consider this issue together with RO group design and provide your comments.

Issue-4. Whether an RO group can be shared between different RO groups for different/same number of multiple PRACH transmissions. This can be realized by set proper start position of each RO group.

Issue-5. Any other consideration about the configuration of RO group.

Companies are encouraged to provide comments on the above aspects.

| Companies | Comments |
| --- | --- |
| CATT | Issue-1: For RO group pattern, we think an association pattern period similar as SSB-to-RO association pattern period may be needed. Issue-2/Issue-3: Frequency hopping has not been agreed yet. We think agreement is needed before discussion the RO group in case of FH. In addition, if FH is supported, the frequency offset can be absolute number of RBs, or in unit of RO, which needs to be discussed as well. Issue-4: we are not sure if we understand the question correctly. But at least considering that different numbers of PRACH transmission may share the same ROs with separate preambles, it is possible that a same set of ROs belong to different RO groups for different number of PRACH transmissions. |
| Panasonic | Issue 1: A RO group should span over multiple periods of SSB-to-RO mapping because the RO group is associated with the same SSB for a multi-PRACH transmission with same Tx beam. Issues 2 and 3: We think that the frequency hopping in a RO group can be supported. To realize this feature, we can introduce multiple RO bundles, each of which is assigned per frequency hop, within a RO group. For instance, RO group#1 (consisting of RO#0-RO#3) includes RO bundle #0 (consisting of RO#0 and RO#1) for frequency hop#0 and RO bundle #1 (consisting of RO#2 and RO#3) for frequency hop#1. Issue 4: It might be related to an assignment between a multi-PRACH resource and a specific number of the PRACH transmission. There are two design choices for the assignment as follows. - Dedicated n-th PRACH transmission resource (RO or preamble): To assign a dedicated multi-PRACH resource for a specific number of the PRACH transmissions, e.g., a specific n-th multi-PRACH transmission is transmitted only by using a specific n-th RO in a dedicated RO group. - Shared n-th PRACH transmission resource: To commonly use multi-PRACH resource for all possible numbers of the PRACH transmissions, i.e., the first PRACH transmission from multi-PRACH transmission can start any of the index of multi-PRACH resource. We support the above two resource configurations for the multi-PRACH transmission. |
| LG | Regarding issue-1, the time span of single RO group may vary depending on the RACH configuration, SSB-to-RO mapping, and repetition numbers. Regarding issue-2, the starting RO (or RACH slot), its period, and the repetition number should be provided to determine the RO groups. Regarding issue-3, if RO frequency hopping is supported, predefined hopping pattern can be used based on the first RO selected by UE. Regarding issue-4, we can support the RO can be shared with separated preamble for different number of multiple PRACH transmission. |
| DOCOMO | Issue-1: We are fine to introduce a SSB-to-RO mapping pattern period. Issue-2/Issue-3: We think supporting frequency hopping for multiple PRACH transmissions would be beneficial. To indicate/configure an offset for frequency domain index among multiple FDMed ROs associated with the same SSB/CSI-RS between multiple hops is straightforward. Issue-4: We are a little confused by “RO group shared between different RO groups for different/same number of multiple PRACH transmissions”. Is the intention “RO shared by different RO groups for different/same number of multiple PRACH transmissions”? For different number of PRACH transmissions, it depends on whether differentiation of different number of PRACH transmissions is relying on separate preamble resource on shared RO or relying on separate RO resource. For the same number of PRACH transmissions, we prefer to not allow shared RO between the different RO groups to reduce complexity. |
| Lenovo | Issue 1: basically fine. Issue 2: basically fine. This sentence “the RO group consist of ROs with the same frequency position” could be revised to “the RO group consist of ROs with the same relative frequency position” since with the current SSB to RO association (e.g., to support PRACH repetition in the shared ROs), it is NOT always the case that the ROs that are mapped to a same SSB has same (absolute) frequency position. |
| Mavenir | Issue-1: We support the associate RO group pattern with the SSB-to-RO association pattern period, so that we only need to focus on the RO group design within one SSB-to-RO association pattern period. Issue-2 & 3: In our view, frequency hopping is supported within one RO group only when msg1-FDM= 2,4,or 8, and ROs associated with the same SSB mapping to different frequency domain position at different time occasion, such configuration including such parameters: {time and frequency start position (or start RO), frequency offset of second hop in unit of RO, the number of ROs within the RO group}. Issue-4: an RO group can be shared for different/same number of multiple PRACH transmissions, within the group the number of RO is corresponding to the maximum number (i.e. K) of multiple PRACH transmissions in the current serving cell. The starting position i of the number of repetition (i.e. N) determined by the UE, could be i mod ([K/N]) = 0. |
| ZTE | Maybe in issue 1, the RO group pattern is better to be SSB-to-RO group association pattern period? I don’t see it is necessary to discuss RO group pattern but the discussion on SSB-to-RO group association pattern period is really helpful. For issue 2 and 3, the RO group design without FH can be as baseline. The first figure can be the starting point for study. For issue 4, the question may be revised as “Whether an RO group can be shared between different RO groups for different/same number of multiple PRACH transmissions.” We prefer not sharing the RO between different RO groups. |
| TCL | For issue-1, in our understanding, when RO group is introduced for multiple PRACH transmission, then a SSB need mapping to a RO group and the pattern of RO group is related to repetition number of PRACH tightly. For issue-2/3, if frequency hopping is enabled, then frequency offset and number of ROs/ number of repetitions of PRACH can be used to determine the RO groups. For issue-4, an RO group is shared between different RO groups for different/same number of multiple PRACH transmissions can be considered. |
| Spreadtrum | Issue-1: We are fine to introduce a SSB-to-RO mapping pattern period. Issue-2 & 3: We support frequency hopping for multiple PRACH transmissions. The following parameters can be introduced: {time and frequency start position (or start RO), frequency offset of second hop in unit of RO, the number of ROs within the RO group}, Issue-4: We support both solutions regarding whether to share ROs or not in different RO groups for different number of multiple PRACH transmissions. - On the one hand, ROs in an RO group can be shared between different RO groups for different number of multiple PRACH transmissions, but not for same number of multiple PRACH transmissions. For the same number of multiple PRACH transmissions between different RO groups, sharing ROs is not necessary as it increases gNB complexity to distinguish different RO groups. - On the other hand, if ROs are shared in different RO groups, separate preambles should be used to differentiate the PRACH transmissions over different RO groups, causing further division of preambles resources. However, in some delay insensitive services, it is feasible not to share ROs in different RO groups for different number of multiple PRACH transmissions. |
| ETRI | Issue 1: We think an RO group may be within the association pattern period. Issue 2 & 3: We prefer to conclude firstly whether to support frequency hopping. If freq hopping is supported, then it needs clarified whether frequency hoping within ROs or ROs are shifted in frequency domain. Issue 4: We think that sharing ROs with different repetition factors requires further clarifications. |
| FL | @ZTE, thanks for pointing out the mistake. @all, Sorry for the mistake, the 4th issue should be: Issue-4. Whether an RO group can be shared between different RO groups for different/same number of multiple PRACH transmissions. This can be realized by set proper start position of each RO group. |
| Ericsson | Issue-1: the time duration of association pattern period depends on not only the duration of association period but also SSB periodicity. If SSB periodicity is very large, like 160ms, but association period is small like 10ms, there will be too many RO groups in the association pattern period. There is no need to depend PRACH RO pattern on SSB periodicity. We see the need of considering an RO group across multiple association periods. For example, with two SSBs in the network and onehalf provided by ssb-perRACH-OccasionAndCB-PreamblesPerSSB, there are 4 ROs associated with each SSB in the association period of 10ms, which can’t support 8 PRACH transmissions. Therefore, one association period is sufficient for a group of 2 ROs and 4 ROs; two association periods in 20ms is sufficient for a group of 8 ROs. Going up to 160 ms of SSB periodicity is unnecessary. Table 2: SS/PBCH block indexes to PRACH occasions mapping with two SSBs RO index in a PRACH configuration period; 0; 1; 2; 3; 4; 5; 6; 7. associated SSB index; 1; 1; 2; 2; 1; 1; 2; 2. Issue-2 and 3: we agree that the RO determination of the first RO and remaining ROs may be different. The frequency resource of latter ROs depends on not only frequency hopping and but also the frequency resource of ROs associated with the selected SSB. For example, with one provided by ssb-perRACH-OccasionAndCB-PreamblesPerSSB, the number of SSBs is not divisible by the number of FDMed ROs. We can solve this issue first and then consider frequency hopping. Issue-4: yes, separate preambles in one RO can be configured for different PRACH repetition factors. |
| Apple | Issue 1: we agree that multiple PRACH transmissions should be finished in one SSB-to-RO association pattern period, it’s easier to find the starting RO for PRACH repetition. If RO group pattern is used, the definition should be defined first. In addition, considering the PRACH repetition overhead, Not all RO groups will be configured PRACH transmission, thus, the RO group periodicity can be configured by the network, for instance, configuring one RO group form five RO groups for PRACH repetition. Issue 2: To reduce the access delay due to PRACH repetition, time domain consecutive ROs associated with one SSB can be considered. Issue 3: Regarding the PRACH frequency hopping, it could be discussed later after we have clear picture of RO group. Issue 4: For the same PRACH repetition level, the RO should not be shared, i.e., for the RO groups associated with the same SSB, the RO is associated with only one RO group. For the case of RO is configured/shared for RO groups with different repetition numbers, this should be allowed if shared RO is allowed for different PRACH repetition numbers, i.e. the last meeting WA. |
| Quectel | Issue-1: We agree with ZTE. Issue-2/Issue-3: We support frequency hopping for multiple PRACH transmissions in the case that msg1-FDM is larger than 1. The frequency offset between neighboring ROs in time domain can be indicated in unit of RO. Issue-4: We support an RO can be shared between different RO groups for different/same number of multiple PRACH transmissions and these RO groups sharing at least one same RO can be differentiated by separate preambles. |
| Sony | Issue 1: This issue seems to be similar to that in Issue #4:SSB-to-RO mapping. Issue 2 & 3: We support frequency hopping and the details of its configuration can be futher discussed. Issue 4: We agreed that ROs in an RO Group can be shared for different PRACH repetition. |
| Qualcomm | Issue-1: We think that it may depend on whether separate ROs are used for PRACH repetition or not. If only legacy ROs are used, multiple periods of SSB-to-RO mapping may be needed to have enough ROs to finish all repetitions. Issue-2&3: We think this issue may be also dependent on whether we use separate ROs for repetition or not. If only shared ROs are used, then frequency offset in unit of ROs make sense, but if separate ROs are used, explicit frequency offset (in terms of RBs) may be more appropriate. Issue 4:Issue 4: We think RO groups associated with different repetition numbers can overlap with each other (e.g. in a nested structure). |
| vivo | Issue 1: Agree that all repetitions must be completed within an SSB to RO association pattern period given UE can already do reattempt after each SSB to RO association pattern period. Issue 2: In our view, in case of FH is supported, UE would select first RO in time and frequency domain randomly. The 2nd RO is TDMed with first RO and associated to the same SSB is determined by applying a frequency offset relative to the first RO. The ROs after 2nd RO are also determined by the frequency offset applied to the previous RO. Issue 3: FH within the configured ROs is preferred. Issue 4: Whether ROs for different number of repetitions can be shared depending on whether they are treated as independent feature combinations. If the answer is yes, PRACH resources for different number of repetitions should be configured independently. |
| Nokia/NSB | Issue 1 – Just to ensure we understand the purpose of this question/issue. Is this to set a maximum time duration of an RO group? If yes, the proposed approach seems fine (although not strictly necessary) since it would imply that the duration of an RO group cannot span a number of resources in time domain larger than an association pattern period, which is reasonable assumption. If no, can the FL please clarify? Issue 2 – In our view, RO groups should be configured by NW (RAN2 detail) and RAN1 should only discuss whether ROs of the same RO group can have different starting frequency index. Surely, we could call this FH if we wanted, but this is not necessary. It should also be noted that this would allow a much more flexible approach to the RO group creation with different starting frequency index among ROs of the same group, given that we would not need to discuss “hop length”, “number of hops”, “RO groups multiplexing” and so on. Overall, we think this issue does not exist if RAN1 agrees that RO groups are configured. This is one of the reasons why we do not understand why RAN1 should overcomplicate the design by going for a RAN1-only solution to group creation, where this can be left to RAN2 which could realize it in a more efficient and flexible way using RAN2 tools. Issue 3 – Differently, from the previous issue, here we think that RAN1 could discuss FH within an RO. This could bring additional frequency diversity and can be realized with a very simple RAN1 procedure, with no RAN2 impact. We do not see this as a paramount aspect to work on, however we are open to this discussion. Issue 4 – In our view, some ROs can be shared among RO groups to ensure that multiple RO groups can be configured for the same number of repetitions, all the while not using a number of resources that is proportional to the number of ROs per group (this does not scale very well). The rationale would be to allow the NW to decide the optimal trade off between the total number of configured RO groups and the total number of used ROs across all the groups. This allows NW to also trade off latency and collision probability depending on implementation constraints and deployment characteristics. We think it is fundamental for the NW to have maximum flexibility in this sense to increase the practical relevance of this feature and motivate its adoption. Please note that there not would be any need to configure different preambles for the groups sharing one or more ROs, exactly because the sequence of ROs in a group constitutes a pattern/signature that provides a lot of diversity across the groups. This diversity can be easily used at gNB to have a correct detection of two PRACH transmissions sent over two groups with some shared ROs between them. The principle is very simple: if you let a UE transmit over multiple instances, the sequence of instances can be used to identify that UE quite straightforwardly. This is a peculiar feature of PRACH repetitions that cannot be observed in the single PRACH transmission (for which you need different preambles). We think it is important to leverage it. |
| OPPO | Issue 1: We wonder whether the SSB-to-RO association pattern period is legacy or newly defined one. It seems that it is new SSB-to-RO association pattern period within which at least one RO group is associated with each SSB. If it is correct, we are fine with the definition. Issue 2&3: We support frequency hopping. The start RB of the first RO in the RO group can be configured. The start RB of the following ROs can follow a hopping pattern. Issue 4: It should be whether a RO can be shared between different RO groups for different/same number of multiple PRACH transmissions. The PRACH resources for multiple PRACH transmissions include RO group and preamble. gNB should differentiate PRACH resources with different number of multiple PRACH transmissions. For a shared RO, different preamble should be configured for PRACH resource PRACH resources with different number of multiple PRACH transmissions. |
| Intel | Issue 1: our understanding is highly dependent on the configuration of SSB periodicity, number of repetitions, etc., if the number of repetitions for multiple PRACH transmission is large, e.g., 8, and if shared ROs are used to differentiate the single and multiple PRACH transmissions, it is clear that RO group would span > 1 association periods. We are open to discuss whether RO group can span > 1 association pattern period. Issue 2 and 3: Frequency hopping for multiple PRACH transmissions should be supported. We clearly demonstrated the performance benefit compared to the case without frequency hopping. In addition, 1 > FDM’ed ROs need to be configured to enable frequency hopping for multiple PRACH transmissions. Frequency hopping needs to be configured within a RO group. Our view is that the frequency hopping pattern as defined for eMTC can be considered as a starting point. Issue 4: We prefer to only allow separate preamble on shared ROs to differentiate different number of multiple PRACH transmissions, in order to avoid fragmented PRACH resources and minimize the impact on the legacy system. |
| Samsung | Issue 1: yes Issue 2-3: FH support or not is still under discussion, delayed the FH related decision Issue 4: we are open to discuss separate RO (from different RACH configuration, or different FDMed RO) or separate preamble from shared RO; |

Proposal 2-3

UE determine the RO groups based on network configuration which satisfies the following conditions:

- The number of valid ROs in the RO group is the same as the number of multiple PRACH transmissions determined by the UE

- The ROs within the RO group are associated with the same SSB as the SSB selects by the UE during cell search.

If there are multiple RO groups satisfy the above condition for a UE, UE select one RO group to transmit multiple PRACH within the RO group.

Note: Multiple PRACH transmissions within one RACH attempt are only performed within the selected RO group.

| Companies | Comments |
| --- | --- |
| CATT | The intention of the proposal is not clear to us. It seems that the first bullet is covered by Proposal 2-2. Proposal 2-2 RO group is specified for multiple PRACH transmissions. One or multiple RO group(s) is(are) configured for each configured number of multiple PRACH transmissions, and the number of valid ROs in one RO group is equal to the corresponding configured number of multiple PRACH transmissions.. For the second bullet, why the SSB has to be the one selected by the UE during cell search? In addition, what “UE determine the RO groups…” means is not clear to us. Our understanding is that UE needs to determine RO groups associated with different SSB and then based on the selected SSB, one RO group associated with the selected SSB is used. |
| Panasonic | This proposal is strongly related to the proposal 2-4, we would prefer to conclude the discussion for the proposal 2-4 before we discuss the proposal 2-3. |
| LG | The starting RO (and/or RACH slot) and its period should be provided along with the repetition number to determine the RO groups. |
| Apple | It’s not clear the benefits to support multiple RO groups (pattern?) for one PRACH repetition level, this part can be FFS. |
| DOCOMO | Our understanding is that: UE determines possible RO groups (for different repetition number, for all SSB indexes) for multi-PRACH transmissions, and then select one RO group corresponding to the determined repetition factor and the selected SSB/CSI-RS, where the condition that “number of valid ROs in the RO group is the same as the number of multiple PRACH transmissions” is already covered by determination of RO groups. |
| Lenovo | Basically fine, but we are not clear what is intention to have this sentence, “If there are multiple RO groups satisfy the above condition for a UE, UE select one RO group to transmit multiple PRACH within the RO group.”. It is quite clear that anyway there would be multiple RO groups available. This is also related with the meaning of “one RO group” “multiple RO group” in proposal 2-2. |
| Mavenir | Please see our view for Proposal 2-2. That is one RO group can be shared for different number of multiple PRACH transmissions, and in such case preamble partition is applied for different number of multiple PRACH transmissions. |
| ZTE | Generally fine with the proposal, same clarification is needed as proposal 2-2, does the definition of valid RO follow the legacy R15? How to handle the RO if it collides with the dynamic SFI? |
| Spreadtrum | The first sub-bullet is covered by Proposal 2-2, there's no need to discuss it again. In addition, if there are multiple RO groups satisfy the above condition for a UE, how to select one RO group by UE to transmit multiple PRACH within the RO group need further study. Additionally, a minor clerical error in proposal 2-3 has been corrected. |
| ETRI | We support the proposal. |
| Ericsson | The two sub-bullets are the same as in Proposal 2-2. Please find our comments there. We support in principle the sentence below the two sub-bullets. A comment is that if the RO group has the same number of ROs as the number of PRACH transmissions, a UE doesn’t need to select within the RO group. |
| Quectel | In our understanding, the proposal provides the rules to determine the RO group(s) to be used from a number of RO groups configured by the network. We support this proposal. |
| Sony | We support the proposal. We think that the gNB can configure multiple RO groups for the same PRACH repetitions of the same SSB and here the UE selects (randomly) one of this RO Group. For example, the gNB may think that there are a lot of UEs that requires 2x PRACH repetitions and so it configures 2 RO groups for 2x repetitions per SSB. |
| Qualcomm | It is better to have restrictions for the starting RO (depending on the repetition number), to simply PRACH detection. |
| vivo | For a determined number of PRACH repetitions, UE would still select SSB first, then select the RO resource of first RO, ROs for repetitions other than first repetition are determined by the RO group pattern. The discussion on how to determine a PRACH resource from a set of PRACH resources with different number PRACH repetitions should be up to RAN2 since this is related to the feature combination definition. |
| MediaTek | Agree in principle. As pointed out, Proposal 2-2 seems already cover these sub-bullets. |
| Nokia/NSB | Support the proposal. @ZTE: we think interplay with SFI can be handled by network, which is one responsible of the configuration. We used this approach in R17 for Msg3 repetitions as well so hopefully we do not need to have that discussion again and can focus on other aspects. |
| OPPO | We support the proposal. |
| Intel | We understand the intention of the proposal. It would be good to clarify whether multiple RO groups can be FDM’ed for multiple PRACH transmissions? |
| Samsung | Maybe I mis-understood, these two are not the conditions to satisfy, but the rules to follow when determine the RO pattern over the valid ROs. |
| New H3C | General fine |
| Sharp | It is not clear what the proposal intends. The definition of RO group is clear enough based on proposal 2-2. In addition, the selection of one RO group based on the number of multiple PRACH transmissions is also clear based on the previous agreements. It should be clarified if there is additional intention. |

Proposal 2-4

If some RO within the selected RO group is dropped due to collision handling, UE then drop the PRACH transmission on the RO, which indicates that the number of actual PRACH transmissions may be less than the determined number of multiple PRACH transmissions.

- Current collision handling rule is reused.

FFS: whether to introduce some new collision handling rules.

| Companies | Comments |
| --- | --- |
| CATT | We are fine with the intention of the proposal in general. But we do not quite understand what “some RO within the selected RO group is dropped” means. We do not think a RO can be dropped. In addition, why is new collision handling rules needed? |
| Panasonic | We think this proposal is related to how to count RO in a RO group. There could be the following counting methods for a multi-PRACH transmission - “Actual valid RO only counting”: For a configured number of multiple PRACH transmissions, the counting is based on (actual) valid RO(s) only in a RO group, where valid RO(s) are used for actual PRACH transmissions. - “Any RO counting”: For a configured number of multiple PRACH transmissions, the counting is regardless of actual valid RO(s) or actual invalid RO(s) in a RO group, where a RO is identified as actual invalid RO due to an RO dropping event and is not used for an actual PRACH transmission, while actual valid RO is used for an actual PRACH transmission. Our view is that - A RO counting method for multi-PRACH transmission should be common among UEs in a cell. If the RO counting method is different among UEs in a cell, the first PRACH and the last PRACH transmission can be different among UEs, which makes impossible to have a combined detection over the multi-PRACH transmission at gNB. - The number of actual PRACH transmissions can be different between UEs. Especially, for a HD UE in paired band, a determination of valid RO from the shared RO can be based on its implementation according to Clause 17.2 of TS 38.213. Therefore, to have a common RO counting method among UEs in a cell, we support the “any RO counting method” because it would be reasonable and simple. |
| LG | We can discuss it further. |
| Apple | Ok with this proposal. |
| DOCOMO | We feel this proposal is also related with “RO group” definition. If “the number of valid ROs is equal to the specific number of PRACH transmissions”, RO collision with cell-specific DL symbol or SSB symbol is already addressed before determination of RO groups. What kinds of collision here needs to be clarified first. |
| Lenovo | Basically fine, but it seems the bullet is not needed since the drop rule has been described in the main proposal |
| Mavenir | Support the proposal. |
| ZTE | From my understanding, the valid ROs is determined based on the semi-static RRC configuration which is the legacy principle. If some ROs need drop due to collision, the collision is from the dynamic SFI or other non-semi-static signalling. At least two options for dropping can be considered, one is as the proposal that only the collided RO is dropped, the other is that the whole RO group is dropped. We slightly prefer the solution in the proposal. |
| TCL | Agree with the proposal. |
| Spreadtrum | We are fine with this proposal. |
| ETRI | We think that an invalid RO already drops PRACH transmission. We think that the proposal 2-4 is a way of counting valid ROs. |
| CMCC | The current collision handling rule should be firstly clarify. Then new collision handling rules should also be discussed. The gNB and the UE should have same understanding about the collision happened. |
| Ericsson | Support it for CBRA. gNB can configure the number of PRACH transmissions for CFRA. Suggest to add an FFS for CFRA or update the proposal depending on the discussion in section 3.4. |
| Fujitsu | We think that the discussion for the determination of valid ROs is firstly required prior to this issue. |
| Quectel | We are fine with the proposal. |
| Sony | Support |
| Qualcomm | We are fine with the proposal, but it may be better to discuss these details after agreements on how to configure “RO group”. |
| vivo | We’re fine to count the valid ROs for PRACH repetitions even if the transmission is cancelled. Regarding the new collision rules, in our view, Msg A PUSCH transmission should be prioritized over PRACH repetitions on separate ROs considering some UEs supporting 2-step RACH may not support PRACH repetition and have no idea on the ROs separately configured for PRACH repetition. |
| MediaTek | Support. |
| Nokia/NSB | Agree. |
| OPPO | We are fine with the proposal. |
| Intel | We have similar question as CATT and RO dropping concept is not clear to us. Our understanding is the procedure for multiple PRACH transmissions is as follows: We first determine the valid ROs based on current validation rule, and according to the association between SSB and RO group, UE may transmit the multiple PRACH in the RO group. However, if one or more of the PRACH transmissions is dropped due to collision, e.g., with dynamic SFI, UE drops the one of more of the PRACH transmissions in the corresponding ROs, but does not defer the multiple PRACH transmissions. |
| Samsung | We are fine to support at least the collided RO, we are open to discuss whether other ROs within the group are impacted even though there is no direct collision. |
| New H3C | If intention of proposal is that when some RO within the selected RO group is dropped due to collision handling, UE then drop the PRACH transmission on the remaining RO, we are fine with this proposal. |
| Sharp | We would like to clarify the dropping rule is not for RO but for PRACH transmission. We have same view with Intel on the proposal. We prefer not to defer the PRACH transmission when one of multiple PRACH transmissions is dropped. |
| Huawei, HiSilicon | It is unclear what the difference between “current collision rules” and the note “• Note 4: Valid RO(s) refers to what is defined in existing specification” in RAN1 agreement for RO group. Could FL clarify it. We think the rule that handles the potential collision between legacy PRACH resources and R18 PRACH resources that are mapped to different SSB at the same symbols, especially for gNBs with multiple simultaneous receiving analog beams, should be supported. The reason is given as follows. If Rel-17 framework for PRACH configuration is reused in Rel-18, then it is almost impossible for gNB to avoid overlapping in time between Rel-18 new ROs and legacy ROs. As shown in the following figures, a gNB has to configure one IE prach-ConfigurationIndex for legacy RO resource and another IE prach-ConfigurationIndex for new RO resources. Both PRACH indices refer to the same PRACH table. For a given format, e.g. C2, most of existing indices have slot#39, many of them have slot#35 or #34. It is worthy noting that only a small sub-set of C2 format can be configured for a TDD UL/DL configuration chosen by operators, e.g. only index #187, #188, #195 and #196 with a periodicity of 5 slots are suitable for TDD UL/DL configuration DDDSU. Therefore, it is impossible for a gNB to avoid the overlapping between new RO and legacy RO by indicating different PRACH indices. Two RO configurations are configured under BWP-UplinkCommon as illustrated below, Legacy RO configuration => prach-ConfigurationIndex New RO configuration=> prach-ConfigurationIndex (assuming Rel-17 framework) TS38.211 If overlapping ROs are always associated with the same SSB, then it would not put a restriction on the number of simultaneously gNB receiving beams. However, it seems not the case in this WI because multiple PRACH transmissions are associated with the same SSB but span multiple slots, which is different from previous PRACH features. The collision due to different SSB beams also seems to occur very frequently according to the Table 6.3.3.2.4. Therefore, the number of actual valid RO in a RO group could be very far less than expected. If a UE is not aware of it, then the actual number of PRACH repetition may be far from what the RSRP threshold for the PRACH repetition expects. |

Issue #3: Same or different preamble(s) during multiple PRACH transmission

Proposal 3-1

Proposed conclusion

There is no consensus to support utilizing different preambles during the multiple PRACH transmissions in one attempt.

Not Support to use different preambles during the multiple PRACH transmissions (12): Huawei, vivo, TCL, Fujitsu, CATT, Xiaomi, Intel, Lenovo, CMCC, Mavenir, Panasonic, Samsung, Spreadtrum

Support to use different preambles during the multiple PRACH transmissions (1): ZTE

| Companies | Comments |
| --- | --- |
| CATT | Support |
| Panasonic | We would prefer to conclude that “not support to use different preambles during the multiple PRACH transmissions” because it is reflected by majority view. |
| LG | Agree |
| Apple | Ok with the proposed conclusion. |
| DOCOMO | Support the conclusion. |
| Lenovo | Support the proposed conclusion. |
| Mavenir | Fine with the conclusion. |
| ZTE | It is encouraged for all companies to check the benefit of supporting to use different preambles during the multiple PRACH transmissions. But if most companies like the simple solution of same preamble, we will not block the proposed conclusion. |
| TCL | Support. |
| Spreadtrum | Support. |
| ETRI | Agree to the conclusion. |
| Sharp | We would like to understand whether this conclusion is for same Tx beam case or for both same/different TX beam cases. At this stage, we prefer that this conclusion is only for same Tx beam case since the design of different Tx beam case is not clear now. |
| CMCC | We do not support using different preambles in time domain ROs. If we agree to support multiple PRACH transmissions located at same time instance, we think using different preambles in multiple PRACH transmissions located at same time instance is feasible. |
| Ericsson | Support |
| Fujitsu | We support this proposal |
| Quectel | We support the proposed conclusion. |
| Sony | Support |
| Qualcomm | Support |
| vivo | Fine. |
| MediaTek | We support the proposed conclusion. |
| Nokia/NSB | Support. |
| OPPO | Support |
| Intel | Support |
| New H3C | Support |

Issue #4: SSB-to-RO mapping

Discussion about whether to introduce a new SSB-to-RO mapping mechanism

FL comment: Based on companies’ contributions, if multiple PRACHs are transmitted on the shared ROs with single PRACH transmission, the current SSB-to-RO mapping mechanism is reused. However, for multiple PRACH transmissions on separate ROs with single PRACH transmission, some companies propose to introduce a new SSB-to-RO mapping mechanism. There are mainly two options:

Option 1: SSBs are mapped to a set of valid ROs in time domain, while the mapping order is aligned with current SSB-to-RO mapping.

Option 2: SSBs are mapped to valid PRACH occasions in the following order

- First, in increasing order of preamble indexes within a single PRACH occasion

- Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

- Third, in increasing order of indexes for PRACH slots

- Fourth, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

An illustration of Option 1 is as follows:

Companies are encouraged to provide your comments on:

Q1. Whether to introduce a new SSB-to-RO mapping mechanism for multiple PRACH transmissions on separate ROs with single PRACH transmission?

Q2. If the answer for Q1 is yes, what’s your preference between Option 1 and Option 2 or other mechanism?

| Companies | Comments |
| --- | --- |
| CATT | We support to introduce a new SSB-to-RO mapping mechanism for multiple PRACH transmissions on separate ROs with single PRACH transmission to reduce the latency. The descriptions of Option 1 and Option 2 are not very clear to us. We do not know how RO group mapping in the figure is derived based on Option 1. What would be the mapping of Option 2? |
| Panasonic | We would prefer Option 1. |
| LG | Q1: Yes Q2: We prefer to support Option 1. |
| Apple | Q1: we support to introduce the new mapping mechanism to reduce the access delay. Q2: Option 1 is simple with small updates on existing mapping mechanism, frequency domain first, time domain second. Option 2, it need to make it clear when to finish the mapping in time domain, such as in one SSB-to-RO association pattern period. |
| DOCOMO | Q1: In our understanding, the new SSB-to-RO mapping mechanism is possible only when separate ROs for multi-PRACH is supported/configured. We are open to introduce new SSB-to-RO mapping rule, but the current option 2 seems not good to us. If the mapping of “frequency domain” is after PRACH slots, doesn’t it mean that all time occasions in time domain would be applied first? The question is when will UE start mapping in frequency domain? It seems at least limitation of PRACH slots (e.g. PRACH configuration period or a certain duration) needs to be considered… Our feeling is that we can first determine whether to introduce new SSB-to-RO mapping rule. The detailed new rule should be the next step after new rule is agreed. Q2: We slightly prefer to introduce new SSB-to-RO mapping rule for separate RO case. |
| Lenovo | Q1: Yes, we support to introduce a new SSB-to-RO mapping mechanism to reduce PRACH transmission latency. Q2: How the new mapping is designed could be FFS. For opt.1, it seems it means the SSBs are per-RO group mapped. If so, it would be good to revise the description to make opt.1 clearer. |
| Mavenir | Q1: we support to introduce a new SSB-to-RO mapping mechanism for multiple PRACH transmissions on separate ROs. Q2: We prefer Option 1 for the new SSB-to-RO mapping mechanism. |
| ZTE | We think it is necessary to introduce a new SSB-to-RO mapping mechanism for multiple PRACH transmissions on separate ROs with single PRACH transmission. Option 1 is preferred. I think it is indeed the SSB-to-RO group mapping. It is hardly to understand the third step in Option 2. Is there any limitation on the number of PRACH slots when mapping is considered? |
| TCL | Q1: Yes Q2: Both option 1 and option 2 are ok for us. |
| Spreadtrum | Q1: Yes Q2: We prefer to support Option 1. |
| ETRI | We prefer Option 1. If a new mapping mechanism is introduced, shared ROs case and separate ROs case seem to have different mecahnim. |
| Sharp | Q1: Yes, new SSB-to-RO mapping mechanism should be introduced for lower latency. Q2: Option 1 seems to be workable. |
| CMCC | Q1: We have some concern about introducing new mapping mechanism. Q2: Introducing new SSB-to-RO mapping mechanism may have difficulty about compatible with the legacy feature using PRACH for early identification. If we confirm both WAs for separate RO and shared RO, then it should be discussed unified mechanism for both situations. |
| Ericsson | Q1: no. If the purpose of Option 1 is to have multiple PRACH transmitted in time-domain consecutive ROs, gNB can set the value of ssb-perRACH-OccasionAndCB-PreamblesPerSSB to be one or larger. We don’t see the need to change the current SSB-to-RO mapping. |
| Fujitsu | Q1: We support to introduce a new SSB-to-RO mapping mechanism Q2: We prefer Option1 because we can minimize the specification impact. |
| Quectel | Q1: Yes. Q2: We support Option 1. |
| Sony | Q1: Yes we need new SSB-to-RO mapping Q2: Similar views as CATT. Unclear how Option 1 can lead to the mapping in the figure and unclear what Option 2 looks like. |
| Qualcomm | Q1: we need new mapping if separate ROs are used. Q2: We prefer Option 1. |
| vivo | Q1: We can understand the intention of the update of SSB to RO mapping. However, when SSBs are mapped to a group of ROs in time domain first, it would mean the SSB to RO mapping cycle is also increased and the SSB to RO association period would be increased as well. For a UE selecting one SSB that is mapped to ROs quite late in the association period, the latency is not reduced by increased. Given this consideration, we do not think optimization of SSB to RO mapping is needed. To really reduce the RA latency with PRACH repetition, additional ROs should be configured relative to the ROs configured by PRACH configuration table as we commented earlier and the additional ROs are mapped to the same SSBs as the RO used for first PRACH repetition transmission. Q2: See answer to Q1. |
| Nokia/NSB | We think this discussion can occur after the design of RO groups is finalized. Until then it is very hard to understand whether and how this should be done. Having said this, from our perspective Option 2 has the merit to have a lower latency for any given number of repetitions, if we agree on Proposal 2-1. Option 1 offers very limited benefits overall, so it could be an overoptimization that does not solve the problem completely and we prefer avoiding it. |
| OPPO | Q1: Yes Q2: We support Option 2. Option 1 cannot minimize the specification impact since it is also a new SSB-to-RO mapping mechanism. And it is not clear for us how to support RO group-wise mapping with multiple number of ROs in a RO group. |
| Intel | Q1: We can be open to consider a new SSB to RO group mapping. Q2: it is not very clear to us how the mapping is performed in Option 1. Does that mean for “SSBs are mapped to a set of valid ROs in time domain”, there are only 2 valid ROs in the figure? |
| New H3C | Q1: Yes. Q2: We support Option 1. |
| Ericsson | We added some more explanations below on Q1: No This new SSB-to-RO mapping rule can only work in the case that all preambles in an RO are dedicated to multiple PRACH transmissions. But this is a rare gNB configuration, causing low preamble utilization. More often, an RO, either a legacy RO or a separate RO, may have several preamble partitions for features, like multiple PRACH transmissions, Msg3 repetition, RedCap, etc. In this case, the new SSB-to-RO mapping rule doesn't work. We advise to focus on the most common RO/preamble configuration. On the other hand, if the intention is about low latency of multiple PRACH transmissions, PRACH configuration index, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and msg1-FDM are separately configured for separate/additional ROs in Rel-16/17. Even if a separate RO is purely for multiple PRACH transmissions, gNB configuration of these parameters can allow low latency for TDMed PRACH transmissions based on legacy SSB-to-RO mapping. If the intention of the proposal is to allow a UE to determine TDMed ROs for PRACH transmissions, this can be discussed in RO group, which consists of TDMed ROs. |

3.1.2 RAR window and RA-RNTI calculation

Issue #5: RAR window and RA-RNTI

FL comment: Companies’ views are listed as follows. Thus, FL propose we go with option 1, and has two versions of proposals.

- Option 1: the starting point of RAR window is after the last symbol of the last PRACH occasion corresponding to the multiple PRACH transmissions.

- Support (26): OPPO, Spreadtrum, ZTE, vivo, CATT, Intel, InterDigital, LG, MediaTek, NEC, Nokia, NSB, Xiaomi, China Telecom, Samsung, Huawei, HiSilicon, Panasonic, Fujitsu, Mavenir, Lenovo, CMCC, Apple, Sharp, NTT DOCOMO, ETRI

- Option 2: the start position of the RAR window is after the last symbol of the first PRACH occasion corresponding to the multiple PRACH transmissions.

- Support (5): Panasonic, ZTE, TCL, Lenovo, Quectel

Proposal 5-1-a

For multiple PRACH transmissions with same Tx beam, the corresponding RAR window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the multiple PRACH transmission.

Proposal 5-1-b

For multiple PRACH transmissions with same Tx beam, the corresponding RAR window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, that is at least one symbol, after the last symbol of the last PRACH occasion configured for corresponding multiple PRACH transmission.

Companies are encouraged to provide your preference of the above two versions and your comments.

| Companies | Comments |
| --- | --- |
| CATT | We support the proposal in principle. The difference between the two proposals is not very clear to us. |
| Panasonic | We support to configure one of two schemes (RAR window starts after the first RO and starts after the last RO) for RAR window and RA-RNTI calculation in a semi-static manner. |
| LG | Support. Either of them is fine. |
| Apple | Proposal 5-1-a is preferred. For Proposal 5-1-b, if the last RO is dropped due to collision, the starting point of RAR window is delayed compared with Proposal 5-1-a. |
| DOCOMO | We support the principle of the proposal. We are not sure about the motivation of the description difference between the two proposals. |
| Lenovo | Prefer proposal 5-1-a. For proposal 5-1-b, it is not clear why the last PRACH occasion is configured for the multiple PRACH repetition. Once an RO group is determined for PRACH repetition, the last PRACH occasion would be the last RO of the RO group (even it might be dropped). |
| Mavenir | We prefer Proposal 5-1-b. |
| ZTE | @FL: We think we can just simply discuss the RAR window starts at the first symbol of the first or last PRACH occasion. In this meeting, please defer the discussion related the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, because we find an issue that may affect the CORESET for RAR window starting. The issue is illustrated as below by an example: If the separate RO is used for multiple PRACH transmission, one UE selects single PRACH transmission and the other UE selects multiple PRACH transmission with repetition factor =2, it is possible the ROs used by the two UEs are FDMed in the same time instance as figure shown, the RA-RNTI calculation base on the RO B and RO D are the same, this means UE can’t identify the corresponding msg2 via RA-RNTI. Then different CORESETs for the different kinds of PRACHs may be needed. This is the reason why we suggest postponing the discussion on CORESET. |
| TCL | Proposal 5-1-a is preferred. |
| IDCC | Support. |
| Spreadtrum | We support the principle of the proposal. The key issue is whether UE and gNB have a common understanding about the dropping rule for ROs in the RO group. In other word, if gNB knows the last one or more RO(s) in the RO group might be dropped, Proposal 5-1-a is preferred to reduce the latency. Otherwise, it is meaningless to advance the starting point of RAR window. Proposal 5-1-b is preferred as RAR might be sent after the last configured RO in RO group. |
| ETRI | Two proposals may have different if some of last PRACH transmissions are dropped. In our view, the last PRACH occasion may be defined at the last RO in the RO group, and if this interpretation is correct, then we would prefer the proposal 5-1-a. |
| Sharp | We support the principle to use last RO for the multiple PRACH transmissions for RAR window. |
| CMCC | Prefer proposal 5-1-b. The only difference between two options is the case when transmission canceled. We think “the last PRACH occasion configured for” have a cleaer understanding between gNB and UE. For the 5-1-a, gNB and UE must have the same understanding about the dropping, otherwise the RAR-window can not be aligned. We should clarify the dropping or canceling rules firstly. |
| Ericsson | Support the two proposals and would like to know whether the difference is PRACH dropping. |
| Fujitsu | We support the proposal from either of them. |
| Quectel | Proposal 5-1-a is preferred. |
| IDCC | Support |
| Sony | It isn’t clear to us what’s the difference between the two. Does it mean: If gNB configures {2, 4, 8} repetitions and if the UE selects 4x repetitions then: - Proposal 5-1a: UE monitors the RAR after it completed the 4th PRACH repetition. - Proposal 5-2b: UE monitors the RAR after the RO for the 8th PRACH repetition has passed since 8x repetition is the configured PRACH repetition. We prefer Proposal 5-2b if the above is the correct understanding. |
| Qualcomm | Support (either one is fine) |
| vivo | RAR window should start from the last valid RO of the RO group in our view since this can be applied to all UEs and the collision rules are different for different UEs. And I would guess Proposal 5-1-b is for this intention, right? If the answer is yes, some wording updates may be needed. |
| MediaTek | Proposal 5-1-b is preferred. Whether UE drops the last PRACH transmission should not impact the RAR window position. |
| Nokia/NSB | Support Proposal 5-1-b. Agree with MediaTek: a configured RO can be dropped or not, but it is still configured (otherwise how could the UE drop it, if applicable?). Hence no ambiguity would exist even in case of dropping. |
| OPPO | Support. Proposal 5-1-a is preferred. |
| Intel | We do not see much difference between these two proposals. Is it correct understanding that the main difference between these two proposals is whether the last PRACH transmission is dropped due to collision handling? It would be good to clarify the intention. |
| New H3C | Support Proposal 5-1-b. Share this same concern on Proposal 5-1-a with MTK. |

3.1.3 Determine the number of multiple PRACH transmissions

Issue #6: Determination of the number of multiple PRACH transmissions

FL comment: During RAN1 #111 meeting, it was discussed about the trigger condition of multiple PRACH transmissions, companies’ views can refer to Section 4 proposal 4 in [6] copied as follows:

For multiple PRACH transmissions with same Tx beam, multiple PRACH transmissions is triggered based on at least one of the following options. - Option 1: SSB-RSRP threshold is utilized to trigger multiple PRACH transmissions. Support: Apple, OPPO, Lenovo, Xiaomi, ZTE, Sharp, ETRI, Samsung, LG, Spreadtrum, Sony, Fujitsu, NEC, DOCOMO, Panasonic - Option 2: The failure of single PRACH attempts reaches a threshold. Support: ZTE - Option 3: The calculated power of single PRACH attempt reaches the maximum output power of UE. Support: ZTE, OPPO

Then, we achieved the current agreement to use at least SSB-RSRP threshold(s) to determine the number of PRACH transmissions at least for the first RACH attempt. Thus, it makes no sense that we go back to discuss the trigger condition. In addition, since we have an agreement that gNB can configure one or multiple values for the number of multiple PRACH transmissions. The relationship between SSB-RSRP thresholds and configure values need to be discussed. Based on companies’ views, the following proposal is made.

Proposal 6

For CBRA, SSB-RSRP threshold is configured per each number of multiple PRACH transmissions configured by gNB.

| Companies | Comments |
| --- | --- |
| CATT | Support |
| Panasonic | We support Proposal 6. In addition, for RA with PDCCH order, we support that a number of multiple PRACH transmissions can be indicated in a DCI. - FFS on detailed indication. |
| LG | Support. |
| Apple | We support this Proposal. |
| DOCOMO | Support. |
| Lenovo | Support this proposal. |
| Mavenir | We are fine with Proposal 6. |
| ZTE | Maybe this proposal is not so accurate. If RACH in SUL is not considered, as we have the rsrp-ThresholdSSB for single PRACH, if repetition factor {2,4,8} are configured, maybe only two new RSRP thresholds are needed for the multiple PRACH transmission. Anyway, we can leave the RSRP thresholds issue for RAN2 to decide. |
| IDCC | Support. |
| Spreadtrum | Support |
| ETRI | Support the proposal. |
| Sharp | We support the FL proposal. |
| CMCC | OK. |
| Ericsson | We support the following agreement made in RAN1#111. However, the SSB-RSRP thresholds are not configured by gNB but determined by a UE. Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed. If a set of cell-specific SSB RSRP thresholds are configured, UEs may start to transmit multiple PRACH transmissions with different levels of positive power headroom. This is not an efficient use of RACH resources and contrary to the spirit of improving UE transmission power. This can be prevented with cell-specific power headroom thresholds. UEs determine their specific SSB RSRP thresholds based on the configured power headroom thresholds and their PCMAX to make the most of their transmission power before using RACH resources shared by all UEs in a cell. |
| Fujitsu | We are fine with the proposal in CBRA case. |
| Quectel | We support this proposal. |
| IDCC | Support |
| Sony | Support |
| Qualcomm | We support having triggering criteria based on SSB-RSRP, but we think other issues (such as the history of past PRACH attempt failures should be taken into account). So, we think it’s better to modify the proposal as following: Proposal 6 For CBRA, for the first PRACH attempt, SSB-RSRP threshold is configured per each number of multiple PRACH transmissions configured by gNB. FFS: How SSB-RSRP threshold is modified after each failed PRACH attempt. |
| vivo | Fine. |
| MediaTek | Support the proposal. |
| Nokia/NSB | RAN1 should just agree that one threshold per repetition number is needed. How this is configured should be left to RAN2 to decide, e.g., only one threshold plus some offsets, multiple thresholds and so on. |
| OPPO | We prefer the previous agreement. Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. For the first PRACH attempt, SSB-RSRP threshold can be used to determine the number of PRACH transmission. The measured SSB-RSRP may not always be a fine parameter to determine the multiple PRACH transmission. For example, UE initially determines single PRACH transmission based on the measured SSB-RSRP. However, it may retransmit the single PRACH with ramped power due to failed reception of RAR. It is a possible legacy situation for single PRACH transmission. Then UE continues the legacy PRACH transmission procedure until PCMAX is reached. It means that the coverage of PRACH is poor. Option3 can address this situation. If UE determines single PRACH for the first PRACH attempt, it still has chance to enhance coverage of PRACH after PCMAX is reached. |
| Intel | It would be good to clarify the following: Assuming two values, e.g., 2 or 4 are configured for multiple PRACH transmissions. In this case, only 1 SSB-RSRP threshold is needed to differentiate the multiple PRACH transmission with these two values, instead of two SSB-RSRP thresholds. Or does that mean two SSB-RSRP thresholds are configured to differentiate 1) between single and multiple PRACH transmissions and 2) between multiple PRACH transmissions with two values? |
| New H3C | OK |

Discussion about whether other factor besides SSB-RSRP is considered to determine the number of PRACH transmissions

FL comment: The following factors are proposed by companies to determine the number of PRACH transmissions besides SSB-RSRP: UE power class; UE power headroom; MPE or P-MPR.

Companies are encouraged to provide your comments about whether other factor besides SSB-RSRP is considered to determine the number of PRACH transmissions.

| Companies | Comments |
| --- | --- |
| CATT | We do not think other factor besides SSB-RSRP is considered to determine the number of PRACH transmissions. |
| Panasonic | We think at least UE power class can also be considered when to determine the number of PRACH transmissions besides SSB-RSRP. |
| LG | We don’t think these are necessary, but they can be discussed further. |
| Apple | As we agreed already the SSB-RSRP is to be used for determining the PRACH repetition number. The necessity of introducing other parameter needs to justify. |
| DOCOMO | Relying only on SSB-RSRP threshold is the simplest solution. |
| ZTE | No need to consider the additional factors besides SSB-RSRP. |
| Spreadtrum | Other factor besides SSB-RSRP are not necessary. |
| ETRI | In our view, RSRP may be sufficient, and we are open to discuss jointly, for example determining power level. |
| Sharp | We don’t find strong necessity on other factors besides SSB-RSRP. |
| CMCC | We want to make sure that UE transmit PRACH with max power before using additional PRACH resources, which benefits for saving gNB’s physical resources. This target can be achieved by some gNB proper configurations like a larger power ramping step or a higher PRACH target power. Introducing these new parameters are not necessary. |
| Ericsson | Please find our comments to Proposal 6. |
| Fujitsu | For initial access case, we don’t think that the UE’s Tx power information is needed to determine the number of PRACH transmission since it is difficult to assume that the UE’s Tx power is saturated to its maximum. For CFRA case, on the other hand, gNB needs to refer the UE’s Tx power in this case due to the fact that UE may be suffering from the saturated Tx power and thus performing power reduction such as power class change or P-MPR. If gNB has UE’s tx power information, the gNB will configure the PRACH for CFRA by referring it. Otherwise, we need to consider how to achieve it. |
| Sony | We don’t think there is a need to define other factors. |
| Qualcomm | We think at least the history of the past PRACH failures should be considered. Also, we prefer modifying the SSB-RSRP threshold according to UE power class and MPE condition. |
| vivo | RSRP based PRACH repetition request seems enough, similar to Msg3 repetition request. We do not see the need to consider power class/PHR/MPR etc., though we’re open to discuss whether repetition number can be increased during PRACH re-attempt. |
| MediaTek | We don’t see a need for additional factors. |
| Nokia/NSB | @FL: would this preclude the possibility to discuss whether some tolerance zone around the threshold can be defined to determine whether the number of PRACH transmissions? |
| OPPO | It is not cleat for us necessity on other factors. |
| Intel | No need to additionally consider these factors on top of SSB-RSRP thresholds. |

3.1.4 Power control

Issue #7: Power calculation

Proposal 7-1-a

The same measurement of the same reference signal to calculate the pathloss should be applied for each of the multiple PRACH transmissions in one attempt.

Proposal 7-1-b

Same transmit power is applied for multiple PRACH transmissions within one RACH attempt.

From FL perspective, the two versions indicate the same thing. Companies are encouraged to provide your preference and comments.

| Companies | Comments |
| --- | --- |
| CATT | We prefer proposal 7-1-b since proposal 7-1-a seems to imply that UE needs to calculate transmission power for each repetition independently although the results would be the same. |
| Panasonic | We support both Proposal 7-1-a and Proposal 7-1-b. |
| LG | Support. We slightly prefer proposal 7-1-a since it is a clear UE behaviour |
| Apple | Proposal 7-1-a is preferred. For Proposal 7-1-b, we are not sure whether same transmit power can be kept considering RAN4 required MPR and SAR limitations with multiple transmissions |
| DOCOMO | We understand the result of Proposal 7-1-a and Proposal 7-1-b would be the same. Prefer Proposal 7-1-b for simiplicity. |
| Lenovo | We agree with FL that they are same in essence. Proposal 7-1-b is clearer. |
| Mavenir | We support both Proposal 7-1-a and 7-1-b. |
| ZTE | No need to constrain the same measurement and same transmitting power as multiple PRACH transmission may span the period of the reference signal measurement. |
| TCL | We prefer proposal 7-1-b. |
| IDCC | Support. |
| Spreadtrum | We support both Proposal 7-1-a and Proposal 7-1-b. |
| ETRI | We slightly prefer proposal 7-1-b. |
| Sharp | We slightly prefer Proposal 7-1-b though either proposal is OK. |
| CMCC | The spirit of these two proposal are to make sure UE keeping the same transmit power during multiple PRACH transmissions. It is confusing why we should have the limitation for keeping the same transmit power during multiple PRACH transmissions should be clarified. For the 7-1-b, in some cases UE may not have ability to keep the same power, which seems putting too much restriction to UE. |
| Fujitsu | We support both options. |
| IDCC | Support |
| Sony | Both proposals seem to be the same thing. |
| Qualcomm | We Support 7-1-b. |
| vivo | Proposal 7-1-b is preferred and it means pathloss is estimated before the first PRACH repetition and applied for power determination of all PRACH repetitions. Regarding the “MPR and SAR limitations” mentioned by apple, in our understanding they cannot be different for different repetitions. |
| MediaTek | Support both proposals. |
| Nokia/NSB | Wouldn’t it be simpler to agree that the max power is used to transmit each repetition? This would also ease Apple’s concern since the max power the UE can use over time can change depending on RAN4 limitations and regulations. |
| OPPO | We support both options. |
| Intel | As FL mentioned, these two proposals would be equivalent. Our view is that we may only need to agree on e.g., Proposal 7-1-b. |
| New H3C | We support both Proposal 7-1-a and Proposal 7-1-b. |

3.1.5 Retransmission of multiple PRACH transmissions

Discussion about retransmissions of multiple PRACH transmissions

FL comment: based on companies’ contributions, companies are encouraged to answer the following questions.

Q1: If multiple PRACH transmissions are performed in the first RACH attempt, in RACH Re-attempts, whether multiple PRACH transmissions are performed? If yes, how to decide the number of multiple PRACH transmissions in RACH Re-attempts, e.g., the same number as the first RACH attempt; or the number can be increased based on some conditions.

Q2: Whether power ramping between RACH attempts for multiple PRACH transmissions is supported?

Q3: Whether separate parameters including {power ramping step, maximum number of transmissions, etc.} are needed for multiple PRACH transmissions？

| Companies | Comments |
| --- | --- |
| CATT | Our proposal is that for multiple PRACH transmissions with same Tx beam, for subsequent RACH attempt(s), power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met. |
| Panasonic | We think that a multi-PRACH transmission is to be used after UE’s transmit power reaches a maximum transmit power. In this case, power ramping is ONLY applied for single PRACH transmission case, and not applied during the multi-PRACH transmission. |
| LG | Q1: In RACH Re-attempts, the same number as the first RACH attempt should be used until some conditions are satisfied. (e.g., transmit power is larger than maximum RACH TX power) Q2: Power ramping between RACH attempts for multiple RACH transmission is supported. Q3: If multiple PRACH are transmitted with separate preamble on shared ROs, separate parameters may be provided for multiple PRACH transmissions. Otherwise, legacy parameters for a single PRACH transmission can be reused. |
| DOCOMO | Q1: We think either “repetition factor is independently determined for each RACH attempt” or “ramping based on the previous attempt” are fine. Q2: Open to further discuss whether power ramping is applied together with repetition factor is increased. Q3: Maybe it depends on how we design e power ramping and repetition factor determination issue for RACH retransmission. |
| Lenovo | There should be order for determination of ramping and repetition number increase. Q1: Yes, the number of PRACH repetition could be increased in the re-attempts. Q2: Yes, power ramping could be supported. Q3: Yes. |
| Mavenir | Q1: in our view, the number can be increased in RACH re-attempts after failure of first attempt, as long as the increased number doesn’t exceed the maximum number of multiple PRACH transmissions supported by the serving cell. Q2: Power ramping can be supported in RACH re-attempts. Q3: New separate parameters for power ramping can be defined for multiple PRACH transmissions. |
| ZTE | Q1: Yes, the multiple PRACH transmissions should perform if the first RACH attempt is multiple PRACH transmission. And the same number as the first RACH attempt should be used. The fackback mechanism such as increasing the repetition factor is not reasonable as it makes UE to do the PRACH resource set selection again, the complexity is not acceptable. Q2: If the first RACH attempt has not reached the maximum transmission power, the power ramping is allowed between RACH attempts. Q3: The benefit of separate parameters such as power ramping step, maximum number of transmissions, etc. should be investigated first. |
| Spreadtrum | Q1: Yes, and the number of PRACH repetition could be same with the first RACH attempt until some conditions are met. Q2: Yes, power ramping can be supported in RACH re-attempts. Q3: Yes. |
| ETRI | We think the retransmission is due to contention. We think for next attempt that the repetition factor is kept same while the power ramping can be considered. For Q3, we support either separate parameters are configured or derived from the legacy parameters. For example, repetition factors can be considered to determine the ramp step. |
| Sharp | Q1: Yes. Increase of the number of multiple PRACH for the PRACH attempt can be considered especially for maximum UE transmission power case. Q2: Power ramping can be supported in RACH re-attempts at least for the case that the repetition factor does not increase. Q3: The benefit of separate parameters is not clear now. It can be considered later based on the design. |
| CMCC | Q1: Yes. The number can be increased. When RACH attempt failed in previous, UE can use a larger number of repetition in next RACH attempt. Q2:No. The UE should firstly use it max power and then try to use PRACH repetitions. Q3:Not necessary. From our understanding, when using multiple PRACH transmissions, UE already using max transmit power. The legacy maximum number of transmissions can be reused, no need to introduce a additional max number. |
| Ericsson | Q1: does the question mean the number of PRACH transmissions in retransmission should always be no smaller than that of the previous attempt? In our view, we only need to determine some rules, with which the PRACH repetition factor is determined. Q2: yes Q3: could FL please clarify whether the intention is for retransmission of multiple PRACH transmissions to use a larger power ramping step and a larger maximum number of transmissions than the legacy? |
| Fujitsu | Our preference is to apply power ramping first until it reaches maximum power, and then increase the number of transmissions. Therefore, our answer is that Q1: Yes, UE can increase the number of PRACH transmissions based on the previous transmission number. Q2: Power ramping should be supported. Q3: We are open to discuss about separate parameters. |
| Sony | Q1: Use same repetition until a condition is reached (i.e. certain Tx power reached) Q2: Yes power ramp per PRACH attemp |
| vivo | Q1: Yes. Q2: Yes. Not sure why we should preclude power ramping for PRACH reattempts with PRACH repetition. Q3: yes. |
| MediaTek | Q1: Same number is kept for the second RACH attempt. Since power ramping will be applied, success probability will increase. Q2: Yes Q3: We don’t see the necessity for separate parameters. |
| Nokia/NSB | Q1: Number can be increased based on some conditions (FFS) Q2: Agree with Panasonic on the power ramping. Q3: there could be a parameter configured by NW to offset the thresholds for the PRACH Re-attempts to ensure that if the first measurement was just above a given threshold then the number can be increase for the Re-attempt, whereas if the number was far from the next threshold, then the UE would perform the same number of PRACH transmissions as for the initial attempt. Indeed, reasons of PRACH failure may be several and not simply related to the number of PRACH repetitions. This approach has the advantage of not letting the number of repetitions increase uncontrollably across different PRACH attempts. Example: X=2 dBs (offset) Threshold_A = N dBs P repetitions Threshold_B = (N-3) dBs M>P repetitions Threshold_C = (N-6) dBs R>M>P repetitions Case 1: UE measures a SS-RSRP that is N-0.5 dB for its initial attempt UE transmits a number of repetitions P associated with Threshold_A. The PRACH fails and the UE needs to retransmit. The offset is applied to obtain (N-2.5) dB, which is still larger than Thresholds B, then retransmission is performed with P repetitions. Case 2: UE measures a SS-RSRP that is N-1.5 dB for its initial attempt UE transmits a number of repetitions P associated with Threshold_A. The PRACH fails and the UE needs to retransmit. The offset is applied to obtain (N-3.5) dB, which is smaller than Thresholds B, then retransmission is performed with M repetitions (associated with Threshold_B). |
| OPPO | Q1: We think the number of PRACH transmissions can be increased in the next attempt based on some conditions. Q2: Power ramping between different attempts can be supported. Q3: Separate parameters can be further considered, if needed. |
| Intel | Q1: repetition level ramping needs to be supported for multiple PRACH transmissions, e.g., after certain number of attempts. Q2: Yes. Similar to single PRACH transmission. Q3: We are open to discuss these. However, maximum number of transmissions may be common multiple PRACH transmissions with different repetition levels. |
| New H3C | Q1: Yes, Q2: Yes, Q3: Open to discuss |
| Huawei, HiSilicon | Q1: If multiple PRACH transmissions are performed in the first RACH attempt, multiple PRACH transmissions are performed in RACH Re-attempts. And, the number of multiple PRACH transmissions in RACH Re-attempts can be increased when its power targeted by PRACH parameter P0 exceeds the maximum transmission power. Q2: yes, but increased number of repetitions is more important considering such UE requiring PRACH repetition is lack of UL power. Q3: Separate parameters including {power ramping step, maximum number of transmissions, etc.} are needed for multiple PRACH transmissions. |

3.2 Multiple PRACH transmissions with different beams

Proposal 9-1

For multiple PRACH transmission with different Tx beams, down-select one of the following options:

Option 1: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is not supported.

Option 2: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is supported.

FFS: whether/how to indicate best UL beam based on multiple PRACH transmissions for the subsequent UL transmissions.

Option 3: Multiple PRACH transmission with different Tx beams is not supported in Rel-18.

Note: If multiple PRACH transmission with different Tx beams is supported, the mechanism defined for multiple PRACH transmissions with the same beam should be reused as much as possible.

| Companies | Comments |
| --- | --- |
| CATT | We prefer Option 3. |
| Panasonic | We would prefer Option 1. |
| LG | We prefer Option 3. |
| Apple | We support Option 3. Defining two features having the same function is not preferred. the multiple PRACH transmission with the same beam should be prioritized. |
| DOCOMO | We support option 1. In our understanding, most specification impacts for multi-PRACH with same Tx beam can be reused for multi-PRACH with different Tx beams. |
| Lenovo | We prefer opt.2. As shown in companies’ evaluations, multiple PRACH transmissions with different finer beams could provide noticeable gain for PRACH detection. Differentiation between multiple PRACH transmissions with different beams and with same beams is beneficial for message detection, e.g., msg3, by somehow indicating the best beam for msg3 transmission. |
| Mavenir | We prefer Option 1. |
| ZTE | Option 2. Considering the two WFs in same Tx beam case, if the Proposal 1-2 is agreed, the same principle for PRACH resources differentiation can also be used for differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam easily. |
| TCL | Option 1 is ok for us. |
| Spreadtrum | We support option 1 if time permits, and multiple PRACH transmissions with same Tx beam can be used as a baseline for different Tx beams. |
| ETRI | We think that different Tx beams and same Tx beams based PRACH transmissions may not occur in the same serving cell. Option 1 is preferred but we are open to Option 2 as well. |
| Sharp | We are OK to consider different Tx beams case. However, to obtain the benefit, how to indicate a best beam needs to be carefully considered. If the indication is done through multiple PDCCHs with different RA-RNTIs, the UE is required to detect multiple RAR PDSCHs which are associated with different Msg2 PDCCHs. This parallel operation has a significant impact on the UE implementation. More clarification is needed. |
| CMCC | Option3. PRACH transmission with different UE beams may let gNB have opportunity to indicate UE which beam should be used in the msg3 transmission, but a lot of spec impact may be needed. If gNB cannot indicate this information, the gain is insignificant. |
| Ericsson | We support Option 2. In our view, spherical coverage requirement guarantees the minimum performance of beam sweeping and can prevent a UE from sweeping beams only in the wrong directions, which EIRP could be lower than the minimum EIRP requirement. 1dB gain provides the lower bound of beam sweeping's performance over PRACH repetitions with the same wide beam. gNB can have different detection methods for PRACH transmissions with the same beam and different beams. Option 1 excludes the possibility that gNB uses corresponding detection methods for PRACH transmissions and will cause performance loss, compared with Option 2. |
| IDCC | We perefer option 3. |
| Sony | Option 3. Multiple PRACH with different beam needs to perform better than multiple PRACH with same beam since multiple PRACH with different beams have higher specs impact. |
| Qualcomm | We agree with Ericsson and we support option 2. |
| vivo | For option1, we would assume PRACH TX beam would not be specified in Rel-18, which means we only introduce PRACH repetition feature and PRACH TX beam of each repetition can be up to UE implementation similar to what we have in current spec. for single PRACH transmission. If above understanding is correct, we’re fine with either option 1 or option 3 considering limited time we have in this release. |
| MediaTek | We prefer Option 3. |
| Nokia/NSB | Option 2. If We with Option 1, how can gNB know which detection algorithm to use? |
| OPPO | We prefer option 3. |
| Intel | We are open to discuss Option 2. However, given the limited TU and we do not even agree some fundamental design for multiple PRACH transmissions with same Tx beam yet, it may be realistic to consider Option 3. |
| New H3C | We prefer option 3. |

3.4 CBRA and CFRA

Proposal 10-1

Support multiple PRACH transmissions for both CBRA and CFRA.

| Companies | Comments |
| --- | --- |
| CATT | Support |
| Panasonic | Support. |
| LG | Support. We can further discuss how to configure the repetition number in CFRA case. |
| Apple | OK with this proposal. |
| DOCOMO | Support. |
| Lenovo | Yes, besides CBRA, support also PRACH repetition for CFRA |
| Mavenir | Support. |
| ZTE | Supporting CFRA for multiple PRACH transmissions will not cost much specification effort, so we support this proposal. |
| TCL | Support |
| IDCC | Support |
| Spreadtrum | Support. |
| ETRI | Support the proposal. |
| Sharp | Support |
| CMCC | Support. How to configure or indicate the repetition number for CFRA could be further discuss. |
| Ericsson | Support |
| Fujitsu | Support. |
| Quectel | Support. |
| IDCC | Support |
| Sony | Support |
| Qualcomm | Support |
| vivo | Support. |
| MediaTek | Support. |
| Nokia/NSB | Support |
| OPPO | Support |
| Intel | We are fine with the proposal. |
| New H3C | OK |
| Huawei, HiSilicon | We prefer not to have an agreement on support of CFRA without clear list of potential spec changes, which seems not minor changes. For example, new DCI field(s) to indicate - whether legacy RO or new R18 separate RO is applied to the indicated preamble index - which repetition level is applied when multiple levels are configured. - Whether the additional fields cause larger DCI size for normal DCI 1_0 due to DCI size alignment between PDCCH order and normal DCI 1_0. Without analysis and discussions on the potential spec impacts, it is too early to agree the proposal. |

- Official offline discussion on Tues.

4.1 Multiple PRACH transmissions with same beam

4.1.1 Resource configuration for multiple PRACH transmissions

Issue #1: Differentiation between single PRACH transmission and multiple PRACH transmissions

Proposal 1-2

It is RAN1 understanding that at least Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning, and that final decision details on how to realize PRACH resource partitioning is up to RAN2.

- Send LS to inform RAN2 this agreement and above 2 confirmed Working Assumptions.

- FFS: whether additional signaling is needed to configure additional ROs to reduce the latency of PRACH repetition.

FL comment: Based on the online and email discussion, it seems the main bullet and the first bullet is stable. Companies are encouraged to provide your comments on the FFS part.

Issue #2 RO group

Proposal 2-1

Proposed conclusion:

There is no consensus to support multiple PRACH transmissions located at same time instance in Rel-18.

Note: multiple PRACH transmissions located at same time instance includes multiple PRACH transmissions in FDMed ROs located at the same time instance and multiple PRACH transmissions with different preambles in the same RO.

FL comment: Based on the email discussion, all companies but one support the above proposed conclusion. To make the progress, companies please check Ericsson’s comment in the 1st round discussion and provide your additional comments if any. In addition, please check the note which clarifies the meaning of “multiple PRACH transmissions located at same time instance”. Since this issue is related to the fundamental design of RO group, if the conclusion is made, then it can make a further progress that “RO group consists of TDMed valid ROs”. FL suggests we have a conclusion first.

Combined Proposal 2-2v1, Proposal 2-3 and Proposal 2-4

- Multiple PRACH transmissions within one RACH attempt are only performed within one RO group.

  - The number of valid ROs in the RO group is equal to the number of multiple PRACH transmissions.

    - If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value.

    - If multiple values are configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to the number of multiple PRACH transmission determined by the UE.

  - If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, the PRACH transmission on the collision RO is dropped based on collision handling rules.

    - Existing collision handling rules in Rel-17 spec. is reused.

    - FFS: whether to introduce new collision handling rules.

FL comment: Since there are a lot of comments regarding the fundamental design of RO group. Clarifications are provided as follows:

1. The initial intention of related proposal indicates that for each configured number of multiple PRACH transmission e.g., {2,4}, there are corresponding RO groups, e.g., N RO groups, each of which has 2 valid ROs, corresponding to the configured 2 PRACH transmissions; and M RO groups, each of which has 4 valid ROs, corresponding to the configured 4 PRACH transmissions. These RO groups can be TDMed or FDMed, which depends on the configuration of ROs, e.g., msg1-FDM>1, then there will be FDMed RO groups.

2. From FL understanding, the definition of valid RO follows 38.213 Section 8.1, so that gNB and all type of UEs can have a same understanding about valid RO and SSB-to-RO mapping. Collision handling can be discussed separately with definition of RO group, collision may only affect the number of PRACH transmissions, result in less PRACH transmission than the configured/determined one.

3. ROs associated with different SSB consist of different RO groups.

4. The collision may include: dynamic SFI or other non-semi-static signalling. New collision handling may include: RO collides with Msg A PUSCH.

In addition, as companies pointed out that there are some overlapping between Proposal 2-2, 2-3 and 2-4. Thus, FL combines these proposals.

Proposal 2-X

Issue-1. Whether to associate RO group pattern with the SSB-to-RO association pattern period, so that we only need to focus on the RO group design within one SSB-to-RO association pattern period.

Support: CATT, DOCOMO, Lenovo, Mavenir, Spreadtrum, ETRI, vivo, Samsung.

FL comment: FL wants to first make some clarifications for this issue. Based on current spec., an association pattern period is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. Since RO group consists of valid ROs associated with the same SSB, a nature thought would be the pattern between RO group and ROs repeats in the same way as pointed out by some companies. Then, we only need to determine/configure RO group in one association pattern period, and the pattern is repeated align with the association pattern period, which can reduce the workload.

Proposal 2-X-1

RO group is [determined/configured] within one SSB-to-RO association pattern period, and the pattern between RO group and ROs are the same within any SSB-to-RO association pattern period.

Issue-2. How an RO group is determined.

FL comment: Based on companies’ comment, it can be seen that majority companies’ views on the spirit of RO group determination is kind of aligned as: Firstly, determine the first RO of an RO group; Secondly, determine the subsequent ROs according to whether frequency hopping is supported or not.

@Nokia, thanks for your comment and suggestion. From FL understanding, the determination of RO group may not just a RAN2 issue. But I’ll check companies views to see whether it can be left to RAN2.

Proposal 2-X-2

- If multiple PRACH transmission with frequency hopping is supported, RO group is determined by the following parameters {time and frequency start position (or start RO), frequency offset of second hop, the number of ROs within the RO group}.

  - FFS: the unit of frequency offset.

  - Note: Frequency hopping is performed within the configured ROs.

- If multiple PRACH transmission with frequency hopping is not supported, RO group is determined by the following parameters {time and frequency start position (or start RO), the number of ROs within the RO group}.

In addition, companies please check Nokia’s comment and are encouraged to provide your views on whether the design of RO group is better to be left to RAN2.

Issue-3. Whether/how multiple PRACH transmissions with frequency hopping is supported?

Support: Panasonic, DOCOMO, Spreadtrum, Quectel, Sony, OPPO, Intel

[ZTE] the RO group design without FH can be as baseline.

[ETRI] We prefer to conclude firstly whether to support frequency hopping. If freq hopping is supported, then it needs clarified whether frequency hoping within ROs or ROs are shifted in frequency domain.

[Apple, Samsung] delay the FH related decision.

FL comment: Based on companies’ comments, FL suggest we postpone the FH related discussion.

Issue-4. Whether an RO can be shared between different RO groups for different/same number of multiple PRACH transmissions. This can be realized by set proper start position of each RO group.

- RO can be shared by RO groups for same number of multiple PRACH transmissions:

Support: TCL, Nokia, NSB

Not Support: DOCOMO, ZTE, Spreadtrum, Apple, Intel

- RO can be shared by RO groups for different numbers of multiple PRACH transmissions:

Support: LG, TCL, Spreadtrum, Ericsson, Apple, Sony, Qualcomm, vivo (if different number of repetitions are treated as independent feature combinations), Nokia, NSB, Intel

Not Support: ZTE

[Panasonic] Support both dedicated and shared PRACH transmission resource.

FL comment: Based on companies’ comments in the 1st round discussion, FL has the following proposals.

Proposal 2-X-4-1

RO can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation.

Proposal 2-X-4-2

Proposed conclusion

There is no consensus that RO can be shared between RO groups for the same number of multiple PRACH transmissions in Rel-18.

- Email discussion (2nd round before online in Wed.)

5.1 Multiple PRACH transmissions with same beam

5.1.1 Resource configuration for multiple PRACH transmissions

Issue #1: Differentiation between single PRACH transmission and multiple PRACH transmissions

Proposal 1-2

Version 1

It is RAN1 understanding that at least Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning, and that final decision details on how to realize PRACH resource partitioning is up to RAN2.

- Send LS to inform RAN2 this agreement and above 2 confirmed Working Assumptions.

- FFS: whether additional signaling is needed to configure additional ROs.

Companies support to delete the FFS (9): Ericsson, Lenovo, Apple, Samsung, Intel, Nokia, ZTE, CATT, LG

Companies support to Keep the FFS (2): Huawei, vivo

Version 2

It is RAN1 understanding that Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) is assumed to be a baseline for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning, and details on how to realize PRACH resource partitioning is up to RAN2.

- Send LS to inform RAN2 this agreement and above 2 confirmed Working Assumptions.

- FFS: whether additional signaling is needed to configure additional ROs.

Version 3

Send LS to inform RAN2 about the 2 confirmed Working Assumptions, and details on how to realize PRACH resource partitioning is up to RAN2.

FL comment: Based on the offline discussion, some company shows some concern of the feasibility about just reusing Rel-17 framework. To make some progress, 3 versions are provided. Companies can have a check and provide your comments. Company can support multiple versions, e.g., support Version 1 and 3. Then, we can see if one of them is acceptable to all of us.

| Companies | Comments |
| --- | --- |
| ZTE | All the three versions are acceptable to us, if the FFS part cannot achieve consensus, we can go directly via version 3. |
| Mavenir | Prefer Version 1, and would like keep the FFS. |
| Samsung | General fine with the proposals, we concerned the FFS point may create too many other issues, like the alignment of SSB-RO association with legacy, and new implementation for these new RO reception at gNB etc. Thus, even though we can live with it as FFS, but it’s not our preference. |
| Panasonic | For the proposal 1-2, we are fine with either version 1 or version 3. |
| Ericsson | Support version 1 and 2. It is better to remove the FFS in the two versions. |
| Sharp | All the three versions are acceptable. |
| New H3C | Support version 1 and can live with FFS |
| CATT | We are fine with the proposals and prefer version 1 and 2 without FFS. |
| Fujitsu | We are fine with all three versions of the proposals. |
| CMCC | We are not sure what is the difference between version 1 and 2. From our understanding, Rel-17 feature combination framework should be a starting point, and try to reuse as much as possible. From our understanding, the feature combinations are most useful for the vertical scenarios/industry. The most useful scenario for PRACH repetition is also vertical scenarios or the To B scenarios. So we think the new design of PRACH repetitions should also compatible to Rel-17 feature combinations. We have no problem to send LS to inform RAN2 about our confirmed working assumptions. |
| Lenovo | We think whether additional ROs are needed or not can be discussed separately. Before that, we can go with either version 1 or version 2 without the FFS. |
| Nokia/NSB | What is the difference between Version 1 or Version 2? They look identical. Any of the versions is ok with us and we can live with FFS if needed. |
| Sony | Fine with all 3 versions. |
| Spreadtrum | We are acceptable with all three versions and we are appreciate if the difference between Version 1 and Version 2 will be clarified. |
| OPPO | Version 1 and 2 are fine for us. We can live with FFS. |
| Quectel | We are fine with Version 1 and 2. |
| Intel | After careful checking, there are indeed difference between Version 1 “can be reused” and Version 2 (is assumed to be a baseline). Given that, we think Version 1 is aligned with our understanding, although we do not have strong view on Version 2. Given the majority support, it would be good to remove FFS. If we cannot reach consensus, we do not think FFS should be included in the LS to RAN2 |
| IDCC | Fine with all. We can remove FFS if there is no consensus. |
| Qualcomm | Fine with version 1. |

Issue #2 RO group

Offline agreed conclusion

There is no consensus to support multiple PRACH transmissions within one RACH attempt located at same time instance in Rel-18.

Note: multiple PRACH transmissions within one RACH attempt located at same time instance includes multiple PRACH transmissions in FDMed ROs located at the same time instance and multiple PRACH transmissions with different preambles in the same RO.

Combined Proposal 2-2v1, Proposal 2-3 and Proposal 2-4

- Multiple PRACH transmissions within one RACH attempt are only performed within one RO group.

  - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions.

    - Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value.

    - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions.

  - If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collision RO is dropped based on collision handling rules, multiple PRACH transmissions are not postponed.

    - Existing collision handling rules in Rel-17 spec. is reused.

    - Note: Multiple PRACH transmissions are not postponed if some of the transmissions are dropped.

    - FFS: whether to introduce new collision handling rules.

FL comment: Based on the offline discussion, some refinement was done. Companies can check this proposal and provide your further comment. In addition, companies please check Huawei’s comment about the collision in Section 3 for Proposal 2-4.

| Companies | Comments |
| --- | --- |
| ZTE | Fine with the proposal. For the collision reason according to Huawei’s comment, the collision is due to the gNB capability restriction on the multiple simultaneous analog beams receiving, but this capability restriction is transparent to UE, and gNB cannot dynamically report the maximum number of simultaneous analog PRACH receiving beams to UE. So UE cannot drop any RO even if the collision occurs. For simplicity, to avoid the collision, one way is depending on gNB configuration on PRACH resources. If it is difficult, the other way, the SSB to RO mapping of single PRACH should applied to multiple PRACH, and some ROs pool, same mapping rule, same mapping period, etc. It seems the PRACH resource differentiated by different preambles on Shared RO is more reasonable to be applied? No dropping is needed due to the collision case mentioned by Huawei. |
| DOCOMO | We are fine with the proposal. |
| Mavenir | We agree Huawei’s comments on the collision between ROs for multiple transmissions and legacy single transmission. Thus we can discuss whether to exclude the collision ROs (with legacy RO) in RO group when UE and gNB determine the RO group. |
| Samsung | Fine with the proposal. For the collision part, we think there should be another FFS points on whether the rest of ROs in the RO group will be impacted by the dropped RO. The reason is that, if the RO is dropped due to collision with other UL tx or DL rx, then the such UE may not be able to quickly switch back to resume the RO transmission. Suggest to add FFS: “FFS: the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group” |
| Panasonic | We are fine with the combined proposal. |
| Ericsson | Fine with the proposal, except for a question for clarification. DCM asked in offline session whether validation rules of RO are reused. We advise to add a Note 3 under the first sub-bullet to make it clear whether the legacy RO validation rules are reused or not. |
| Sharp | WE support the latest combined proposal. |
| New H3C | OK |
| CATT | We are fine with the proposal. |
| Fujitsu | We are fine with the proposal. |
| CMCC | Thanks FL for the updates. For the 1st sub bullet we are general fine. But for the 2nd bullet, we do not think we have Rel-17 collision rule for RACH occasions. For the RACH occasion, we only have valid ROs or not valid ROs. Between the SFI and valid RO, we have had the discussion in the previous Releases. Most companies do not think the RO should be impacted by the SFI. And at the very beginning, the RO are only determined by the TDD-UL-DL config common. If there is any issue or the group consider it as a problem, it would be solved and captured in the spec since Rel-15. For the issue mentioned by Huawei, the spatial filter of the RACH occasions would conflict among different FDM ROs. This issue also exists even in Rel-15. All the short format preambles are designed for FR2. And the configurations of FDM ROs are allowed. No one complains the confliction between different ROs with different spatial filter. And we do not have any rules to avoid such situation. That is because it can be solved by the implementation or the configurations of gNB. For the overlapping between separate RO and the common ROs, we still think this can be solved through the configuration. And even we introduced separate RO for even two step RACH and other feature combinations, no rules are defined for the collision between them. Then our thinking is before we talk about collision rule or dropping of the ROs, we should first clarify in which a condition the collision would happen. Then we can solve the issue. But at this moment, the collision issues are not clear to us. |
| Lenovo | We are fine with the proposal. With the offline conclusion, we suggest to add a bullet to explicitly say “the RO group does not contain FDMed ROs in the same time instance”. |
| Nokia/NSB | Suggest a minor modification: - If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collision RO is dropped based on collision handling rules, the multiple PRACH transmissions over such ROs are not postponed. |
| Sony | The 1st sub-bullet sounds like the number of ROs in a RO Group can take only 1 value of the configured repetitions. That is if gNB configures {2, 4, 8} repetitions, then the number of RO in the RO Group can only be either 2, 4 or 8, and if it is configured as 2, then if 4x repetitions is required, somehow we have to combine two RO Groups to make 4x repetitions. I believe this is not the intention. Suggested clarification: - The number of valid ROs in a RO group can be equal to one of the configured number(s) of multiple PRACH transmissions. On the 2nd sub-bullet, there is some editorial mistake: - If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collided RO is dropped based on collision handling rules, multiple PRACH transmissions are not postponed. |
| Spreadtrum | We are fine with the proposal. |
| OPPO | We are fine with the proposal. |
| Quectel | We are fine with the proposal. |
| Intel | During the offline discussions, it is not clear to us why “determined by UE” in the second sub-bullet was removed. In our view, if this is removed, the whole “Note 1 and Note 2” seems meaningless. For the second bullet, “and” should be removed. If there is collision, PRACH should be dropped based on current collision rule. This is natural consequence of collision. |
| IDCC | Fine with the proposal. |
| Qualcomm | Fine with the proposal |

Issue 2-1. Whether to associate RO group pattern with the SSB-to-RO association pattern period, so that we only need to focus on the RO group design within one SSB-to-RO association pattern period.

Support: CATT, DOCOMO, Lenovo, Mavenir, Spreadtrum, ETRI, vivo, Samsung.

FL comment: FL wants to first make some clarifications for this issue. Based on current spec., an association pattern period is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. Since RO group consists of valid ROs associated with the same SSB, a nature thought would be the pattern between RO group and ROs repeats in the same way as pointed out by some companies. Then, we only need to determine/configure RO group in one association pattern period, and the pattern is repeated align with the association pattern period, which can reduce the workload.

Based on the offline discussion, some companies show concerns on the limitation of one SSB-to-RO association pattern period. For example, if the SSB-to-RO association pattern period is too short that the ROs within the period is not enough for, e.g., 8 PRACH transmission. In addition, some company propose to consider RO group determination within K SSB-to-RO association period. From FL perspective, the spirit of them are the same, which is to determine a time period, e.g., called “RO group pattern period”, so that the pattern between RO group and RO can be repeated to make the design easier. Thus, FL has the following proposal.

Proposal 2-X-1-1

RO group is determined within an RO group pattern period, starting from frame 0, and the determined RO group pattern repeats in every RO group pattern period.

  - FFS: whether RO group pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period.

    - K is a positive integer.

    - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period.

    - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213.

| Companies | Comments |
| --- | --- |
| ZTE | Fine with the proposal. As the collision due to the reason raised by Huawei, it seems much impossible to configure all the enough RO resource for multiple PRACH transmissions in the period of legacy SSB-to-RO association pattern period, so I think relax the RO group pattern period may be a better way. |
| DOCOMO | Fine with the proposal. |
| Mavenir | We are fine with the term ‘RO group pattern period’ or ‘SSB-to-RO group pattern period’. And we support the second FFS, i.e. K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. |
| Samsung | Support the proposal. And our preference is K association pattern period, since the length of the association period is uncertain, it’s not easy to determine a repeated pattern with such uncertain association period. |
| Panasonic | We understand a motivation of the proposal 2-x-1-1. We support this motivation, i.e., a RO group would span over multiple periods of SSB-to-RO association such that UE can determine enough ROs for a large number of PRACH transmission, e.g., 8. Regarding the proposal 2-x-1-1, - It is not clear that “frame 0” is absolute value or other value. We would recommend to replace “starting from frame 0” by “starting from a starting point of the RO group pattern”. - This statement “the determined RO group pattern repeats in every RO group pattern period” is not clear as the “RO group pattern” is not defined. Due to the above, we would like to update the proposal as follows The updated proposal 2-x-1-1: RO group is determined within a length of K SSB-to-RO association periods (or a K SSB-to-RO association pattern periods) - FFS: details on K, where K>=1. - Note: SSB-to-RO association period (or SSB-to-RO association pattern period) refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. |
| Ericsson | Support the proposal. In our view, K can be equal to or larger than one. The length of PRACH association period or pattern period depends on parameters of PRACH configuration index, SSB-to-RO mapping, etc. If we consider multiple PRACH transmissions as one of the multiple features in a feature combination, namely configured with one of the several preamble partitions in one RO, network will configure the abovementioned parameters for all the features in the feature combination, rather than for one feature only. That is to say, the number of ROs associated with one SSB in an association period or an association pattern period may be smaller than the configured PRACH repetition factor. @Panasonic, 38.213 says “An association period, starting from frame 0, …” An association pattern period should also start from frame 0. |
| Sharp | We support the FL proposal in principle. We would like to know whether multiple RO groups can be included in a RO group pattern period if the RO group pattern period is replaced by K SSB-to-RO association pattern period, since an SSB-to-RO association pattern period may include multiple ROs more than configured number of multiple PRACH transmissions. |
| New H3C | Fine with the proposal. |
| CATT | We are fine with the main bullet. For the FFS part, we prefer to remove that given that we prefer new SSB-to-RO mapping in case of separate RO and in this case we do not expect the RO group pattern period can be replaced. If it is kept, we would like to make the following change. RO group is determined within an RO group pattern period, starting from frame 0, and the determined RO group pattern repeats in every RO group pattern period. - FFS: whether RO group pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period in case of shared RO with single PRACH transmission. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. |
| CMCC | Do not support. Our thinking is the RO group should be defined within one PRACH configuration period. There is no need to further define or use the SSB-to-RO association pattern period. Any way there would be multiple SSB-to-RO association pattern period within the PRACH configuration period. As mentioned by FL and companies, if one SSB-to-RO association pattern period cannot support e.g. 8 repetitions. Any way we should extend the mapping period. And the legacy PRACH configuration period can be used as the boundary. What we need to define is how many ROs would be within the RO group. And how many groups would happen within the PRACH configuration period. |
| Lenovo | We are basically fine with the proposal. |
| Nokia/NSB | We would like to thank the FL for taking our comments into account. We do not agree with CATT modifications and would like to keep the original working of the FFS. We would also like to ensure this agreement does not imply that RO groups are determined and not configured. We understand that the majority of companies prefer determining the RO groups, but we have yet to receive sufficient guarantees that this will allow to have a simple but powerful framework. We thus prefer to replace “determined” with “[determined/configured]” @CMCC: how can PRACH repetitions be performed within a configuration period, if we are not even sure to be able to map all the SSBs to one RO in one configuration period (which is the reason why we define association period)? |
| Spreadtrum | Support the proposal. Our preference is RO group pattern period for the separate RO and K association period is preferred for the shared RO. We share the same view with CATT. In existing spec. i.e., Section 8.1 in TS 38.213, NTxSSB SS/PBCH block indexes are mapped at least once to the PRACH occasions within an association period. If K SSB-to-RO association period or K SSB-to-RO association pattern period is determined for the case of separate RO, it means the SSB-to-RO mapping rule may follow the legacy one, which is not acceptable by us. We would like to make the following change: Proposal 2-X-1-1 RO group is determined within an RO group pattern period, starting from frame 0, and the determined RO group pattern repeats in every RO group pattern period. - FFS: whether RO group pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period in case that multiple PRACH are transmitted with separate preamble on shared ROs. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note1: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. - Note2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. |
| OPPO | Fine with the proposal. A new RO group pattern period is needed to cover the number of ROs in a RO group. The RO group pattern period may span multiple legacy SSB-to-RO association pattern period. |
| Quectel | We are fine with the proposal. |
| Intel | We are generally fine with the proposal. It may be good to us “RO group association pattern period” to align the current definition for single PRACH transmission In our view, depending on the configuration, e.g., SSB to RO mapping and the number of FDM’ed RO, multiple RO groups may be multiplexed in a FDM manner. In this case, these multiple RO groups may be repeated across RO group pattern period. The details on how to determine K can be further studied. We suggest to remove the example. Also, similar to RO group, we suggest to further discuss the spec impact. Proposal 2-X-1-1 RO group is determined within an RO group association pattern period, starting from frame 0, and the determined RO group association pattern repeats in every RO group association pattern period. - FFS: whether RO group association pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. - Note: whether to define “RO group association pattern period” in specification will be discussed separately |
| IDCC | Fine with the proposal. |
| Qualcomm | Support |

Issue 2-4. Whether an RO can be shared between different RO groups for different/same number of multiple PRACH transmissions. This can be realized by set proper start position of each RO group.

FL comment: Based on companies’ comments in the 1st round discussion, companies’ views are summarized as follows.

- RO can be shared by RO groups for same number of multiple PRACH transmissions:

Support (3): TCL, Nokia, NSB

Not Support (6): DOCOMO, ZTE, Spreadtrum, Apple, Intel, CATT

- RO can be shared by RO groups for different numbers of multiple PRACH transmissions:

Support (12): LG, TCL, Spreadtrum, Ericsson, Apple, Sony, Qualcomm, vivo (if different number of repetitions are treated as independent feature combinations), Nokia, NSB, Intel, CATT

Not Support (1): ZTE

[Panasonic] Support both dedicated and shared PRACH transmission resource.

Proposal 2-X-4-1

- RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation.

- RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions.

| Companies | Comments |
| --- | --- |
| ZTE | We can live up with the proposal if majority support. For first bullet, as some companies intend to save the PRACH resources, I can accept the ROs can be shared between RO groups for different numbers of multiple PRACH transmissions for sake of progress. |
| DOCOMO | Support the proposal. |
| Mavenir | Support this proposal. |
| Samsung | The first bullet is fine. The second one is not so clear to us. We guess it’s to exclude some subsequent RO group determined with starting RO located in the preceding RO group. But this might be subjective to the decision on the RO group determination rules agreed eventually. |
| Panasonic | We are fine with the proposal |
| Ericsson | A question to the first sentence. Can shared preambles in a shared RO differentiate RO groups of different PRACH repetition factors? Is the intention like the following? Proposal 2-X-4-1 - RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation. - RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions. |
| Sharp | We support the FL proposal. |
| New H3C | Fine with the proposal |
| CATT | Fine with the proposal. |
| Fujitsu | We are fine with the proposal. |
| CMCC | We have no problem with the 2nd bullet. For the 1st bullet, currently we do not want to introduce the overlapped RO groups. Although the preambles can be used to differentiate different repetition numbers. But it will also complicated the gNB’s receiver. And we do not think this is a critical issue. It is a kind of optimization. We can discuss this later. |
| Lenovo | We support the first bullet. For second bullet, we prefer that an RO can be shared between RO groups for the same number of multiple PRACH transmissions. As explained in our contribution, this is beneficial in terms reducing the PRACH transmission latency. |
| Nokia/NSB | Not fine with this proposal, since it can significantly increase the number of preambles that NW needs to reserve for PRACH repetitions, when multiple numbers of repetitions are configured. We do not expect many UEs to support this feature in the early stage of its adoption and we do not know how much its adoption will be widespread. Hence accepting that the only way to separate UEs is through reserved preambles is not feasible. There is no valid technical reason to justify this, and forces only one gNB’s implementation. We can accept to have also this possibility if gNB can also solve the problem via implementation and separate UEs using the time correlation between ROs. After all, for single PRACH transmission we have 100% overlap for all the ROs, since all UEs can always pick all the ROs and the same preamble. The problem is solved through gNB’s implementation. Now, for multiple PRACH transmissions we even have the advantage of having multiple ROs used by the same UE, i.e., a time correlation between them that can be used to further distinguish two UEs. This implies that gNB’s complexity is not higher in this case, because the problem would be simpler than in Rel-15! This implies that the multiple PRACH transmissions are much more powerful than single PRACH transmission in terms of detection probability, by default. Why do we also need preamble separation? The technical foundation of the proposed direction is unclear and we respectfully ask to consider our concerns from the technical perspective. We suggest the following modifications: Proposal 2-X-4-1 - RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation. FFS: whether RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions. |
| Sony | Support the proposal. |
| Spreadtrum | Support this proposal. |
| OPPO | We are fine with the proposal. For the first bullet, we can also live with separate ROs between RO groups for different numbers of multiple PRACH transmissions |
| Quectel | We are fine with the first bullet. For the second bullet, we agree with Lenovo that RO(s) can be shared between RO groups for the same number of multiple PRACH transmissions by using separate preambles for differentiation. |
| Intel | We are fine with the proposal. |
| IDCC | Support the proposal. |
| Qualcomm | Support |

- Email discussion (3rd round)

6.1 Multiple PRACH transmissions with same beam

6.1.1 Resource configuration for multiple PRACH transmissions

Issue #1: Differentiation between single PRACH transmission and multiple PRACH transmissions

FL comment: The draft LS was uploaded in https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.12(NR_cov_enh2)/9.12.1/Draft%20LS.

Companies please have a check and provide your comments.

| Companies | Comments |
| --- | --- |
| Ericsson | Fine |
| ZTE | Fine with the LS. |
| LG | Support |
| DOCOMO | Fine with the LS. |
| Mavenir | Fine with the LS. |
| Panasonic | The draft LS is fine to us. |
| Sharp | Fine with the draft LS. |
| Nokia/NSB | Support |
| NewH3C | OK |
| CATT | Fine with the LS. |
| Lenovo | Fine with the LS |
| Spreadtrum | Support |
| Fujitsu | We are fine with the draft LS |
| CMCC2 | Fine with the draft LS |
| Sony | Fine with the LS |
| IDCC | Finw with the LS. |
| Intel | It would be good to also include the new agreement in the LS. Agreement Send LS to inform RAN2 about the 2 confirmed Working Assumptions, and details on how to realize PRACH resource partitioning is up to RAN2. |
| Qualcomm | Fine with the LS |
| Apple | Fine with the LS |

Issue #2 RO group

FL comment: Based on online discussion and previous discussion. The majority companies are fine with the first bullet. Thus, it seems better to divide the current proposal into two parts to make some progress.

@Intel, as clarified online, note 1 and note 2 are meaningful, cause if we only have the sub bullet, it may cause such case that 2 PRACH transmission performed on RO group with 4 valid RO(s). Thus, the proposal is clearer with the tow Notes.

Proposal 2-2v2

- Multiple PRACH transmissions within one RACH attempt are only performed within one RO group.

  - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions.

    - Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value.

    - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions.

    - Note 3: Valid RO(s) refers to what is defined in existing specification.

| Companies | Comments |
| --- | --- |
| Ericsson | Support |
| ZTE | Support. For Note 3, maybe we can make it clear. The wording of “refers to” has an uncertainty. Note 3: Valid RO(s) refers to what is has been defined in existing specification. |
| LG | Agree in principle. For the clarification, we can add some modifications as follows: - Note1: If only one value is configured for multiple PRACH transmissions in a cell, then the number of valid ROs in the RO group is equal to this value. - Note2: If multiple values are configured for multiple PRACH transmissions in a cell, for each value, the number of valid ROs in the each RO group is equal to the corresponding number of multiple PRACH transmissions. |
| DOCOMO | Support. |
| Mavenir | Fine with the proposal. |
| Panasonic | We are fine with the proposal 2-2v2. |
| Sharp | Support |
| Nokia/NSB | Support. Concerning Note 3, we think its meaning is clear, however if FL prefers addressing ZTE’s concern then our preference would be write it as follows: - Note 3: The definition of valid RO(s) is as per refers to what is defined in existing specification. |
| NewH3C | OK |
| CATT | Fine with the proposal. |
| Lenovo | We are fine with the proposal. It seems the main bullet is from UE point of view while the sub-bullets are more from gNB point of view. But anyway, the spirit of the proposal is clear. |
| Spreadtrum | Support |
| Fujitsu | Support. |
| CMCC2 | Support. Either Nokia’s version or ZTE’s is fine to us. |
| Sony | Share similar view with Lenovo, the 1st bullet is from UE perspective whilst the rest are from gNB perspective, which is why the description seems disjointed. Just to ensure I understand the second note: - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions. Can someone (i.e. FL) clarify that this means that if gNB configures 3 repetitions = {2, 4, 8} then it will have 3 different types of RO Groups that have 2, 4 and 8 ROs respectively? The gNB can have more than one type of RO Groups, e.g., it can have 3 RO Groups consisting of 2 ROs for 2x repetition and 4 RO Groups consisting of 8 ROs for 8x repetition. |
| IDCC | Support. |
| Intel | Although we still it is important to consider how to determine the number of repetitions for multiple PRACH transmissions, we can accept this proposal to move forward. |
| Qualcomm | Support |
| Samsung | Support, Note 3 is same as previous meeting, no need to change. |
| Apple | Support |

Proposal 2-4v2

If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collision RO is dropped based on collision handling rules, multiple PRACH transmissions are not postponed.

  - Existing collision handling rules in Rel-17 spec. is reused.

  - FFS: whether to introduce new collision handling rules.

  - FFS: the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group

Proposal 2-4v3

If PRACH transmission is dropped based on collision handling rules, multiple PRACH transmissions are not postponed.

  - Existing collision handling rules in Rel-17 spec. is reused.

  - FFS: whether to introduce new collision handling rules.

  - FFS: the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group

Companies please provide your comments on the above two versions and on the collision issues, including Huawei’s comment about the collision in Section 3 for Proposal 2-4, and CMCC’s comment in Section 5 for Combined Proposal 2-2v1, Proposal 2-3 and Proposal 2-4. In addition, companies are encouraged to discuss the detailed about where the collision will happen, and suggested to provide the section number of the corresponding spec to make it more clear.

| Companies | Comments |
| --- | --- |
| Ericsson | A question regarding the second FFS is whether the concern of TA only exists for RRC_Idle UEs. Another point is that after determining an RO group, if a UE realizes that it has to drop at least one PRACH transmission in the RO group, it can - (partially dropping) drop the PRACH transmission in the RO and transmit other PRACHs of the RACH attempt, or - (completely dropping and postponing) drop all PRACH transmissions in the RO group and postpone all PRACH transmissions in another RO group. The second behavior has no specification impact and is up to UE implementation. It is the only choice if all ROs in an RO group have collision, while the current proposals may prevent the second behavior. We provide the following update in blue so that a UE can transmit in non-collision ROs of a RO group. Proposal 2-4v2 If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collision RO is dropped based on collision handling rules, the dropped PRACH transmission is multiple PRACH transmissions are not postponed. [omitted] Proposal 2-4v3 If PRACH transmission is dropped based on collision handling rules, the dropped PRACH transmission is multiple PRACH transmissions are not postponed. [omitted] |
| ZTE | Fine to support version of 2-4v3. But actually the two versions are almost the same. For the collision, I don't think it is much serious. Most of the collisions may avoid by gNB implementation. For example, as the collision due to ROs in the same time instance mapping to different SSB, gNB can try to differentiate PRACH resources by different preambles on shared RO. Regarding the second behaviour mentioned by Ericsson, I have different view. In my understanding, if the whole RO group is dropped, it means the initial access is failed, and UE need to initialize another RACH procedure. |
| LG | We are fine with Proposal 2-4v2. Also, Proposal 2-4v3 can be acceptable with following modifications: Proposal 2-4v3 If one or more RO(s) of the multiple PRACH transmission is are dropped based on collision handling rules, multiple PRACH transmissions are not postponed. <omitted> |
| DOCOMO | Support Proposal 2-4v3. It seems companies have different views on UE behavior considering RO collision. For “If collision occurs on some RO(s) within the RO group where multiple PRACH are transmitted, and the PRACH transmission on the collision RO is dropped”, we don’t have any agreement on this “UE dropping PRACH transmissions on the collision RO” behavior. For example, Ericsson proposed other possible UE behaviors. It seems not good to put a not agreed behavior in the main bullet. Regarding whether any impact on ROs after the collision RO, we share CATT’s online comment that it seems unnecessary considering the current dropping of PUSCH/PUCCH repetitions doesn’t impact later repetitions. |
| Mavenir | We slightly prefer Proposal 2-4v2. |
| Panasonic | We are fine with either Proposal 2-4v2 or Proposal 2-4v3. Regarding the collision event, it can be happened for a HD UE in paired band. In particular, for a given valid ROs, the number of actual PRACH transmissions can be different between HD-UEs due to handling an event of collision with another UL/DL channel/signal according to Clause 17.2 in TS 38.213. We describe this possibility in our contribution [R1-2302885]. It is copied in below box for your convenient. If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks by ssbPositionsInBurst in SIB1 or in ServingCellConfigCommon in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks. If a HD-UE would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS based on a configuration by higher layers or is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, and the HD-UE would transmit PRACH or MsgA PUSCH triggered by higher layers starting or ending at a symbol that is earlier or later than Rx-Tx ⋅ c or Tx-Rx ⋅ c, respectively, from the last or first symbol in the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks. |
| Sharp | We support the Proposal 2-4v3. Following modification based on that by Ericsson and by LG would be clearer. Proposal 2-4v3 If one or more RO(s) PRACH transmission(s) of the multiple PRACH transmissions is are dropped based on collision handling rules, the dropped PRACH transmission(s) is multiple PRACH transmissions are not postponed. |
| Nokia/NSB | Fine with LG’s version, that is clear and unambiguous. With reference to the need to collision handling rules, we agree with ZTE that this problem seems minor and can likely be mitigated by proper configuration at gNB. By the way, this further shows why RO group configuration is better than RO group determination because it gives much finer control to gNB for configuring RACH resources without overcomplicating the design to account for corner cases and special features, e.g., FH. |
| NewH3C | OK with Proposal 2-4v3 with LG’s modification |
| CATT | We prefer Proposal 2-4v3 which is more accurate. For LG’s version, we do not think RO is dropped. We are still not clear about the last FFS, i.e. in which case drop a PRACH transmission would impact other PRACH transmissions. |
| Lenovo | We share similar view with DOCOMO. |
| Spreadtrum | Preference proposal 2-4v3, and we are fine with the modification by Sharp. |
| Fujitsu | We support Proposal 2-4v3 and prefer the modification version by Sharp. |
| CMCC2 | Thanks FL for updates and companies further clarification. Proposal 2-4v3 with Ericsson’s update is preferred. Our thinking is that the key point of this proposal is not to extend the ROs within one RO group or the increase the RO number of the RO group, when dropping of a PRACH transmission happens. Then the expression of the dropped PRACH transmission is not postponed seems more proper. And the issue of collision between the ROs, as we mentioned in the last round, this could be solved by gNB implementation. Currently our first priority is to have a clear definition of the RO group. For the 2nd FFS, our views is that we do not introduce new collision rules, unless there are some multiple PRACH transmission specific issues are found. For the 3rd FFS of the impact of the dropped PRACH transmission, based on current collision rules of PRACH transmission and other uplink transmissions, whether dropping the PRACH transmission or other UL transmission depends on UE implementation. Then gNB have no idea which PRACH transmission does not happen, and gNB may combine some noise when combing the N repetitions of PRACH. And if the detection or demodulation of the PRACH repetitions failed, the UE will do the autonomous retransmission. Then our think about the impact of the dropped RACH transmission to the remaining RACH transmission within the RO group should be trivial. |
| Sony | Share similar views with E/// & LG, it will be good to clarify that it is the dropped RO (or PRACH transmission) that is not postponed. We are fine with Sharp’s proposal which is a combination of E///’s & LG’s proposals. |
| IDCC | Fine with both proposals in general. Ericsson’s revision seems clearer. |
| Intel | We prefer Proposal 2-4v3 with update from Sharp. We suggest to add “in one RACH attempt” in the main bullet It is also not clear to us the last FFS. To move forward, we suggest the following: Proposal 2-4v3 If one or more RO(s) PRACH transmission(s) of the multiple PRACH transmissions is are dropped based on collision handling rules, the dropped PRACH transmission(s) is multiple PRACH transmissions are not postponed in one RACH attempt. - Existing collision handling rules in Rel-17 spec. is reused. - FFS: whether to introduce new collision handling rules. FFS: whether there is the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group |
| Qualcomm | We prefer LG’s version of the proposal |
| Samsung | One point is on using “collision handling rules”, because PRACH might be dropped is not only collided with other signals, we have rules like if PRACH and other UL signals in the same slot , PRACH might be dropped. This is not exactly like collision. We can be ok with Intel’s change on the FFS point. Regarding the change with adding “in one RACH attempt”, I hope it’s not triggering to compensation design towards these dropped PRACH transmission. For FFS point, we draw picture on potential case that the FFS is targeting to, there might be other cases as well. Suggested change on top of Intel’s: Proposal 2-4v3 If one or more RO(s) PRACH transmission(s) of the multiple PRACH transmissions is are dropped based on collision handling rules, the dropped PRACH transmission(s) is multiple PRACH transmissions are not postponed in one RACH attempt. - Existing the collision handling rules refer to the rules causing to drop PRACH transmission in Rel-17 existing spec. is reused. - FFS: whether to introduce new collision handling rules. FFS: whether there is the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group |
| Apple | We are not clear the FFS part as well. To move forward, we can live with it. We are ok with Samsung’s version. |
| Huawei, HiSilicon | As commented before, it is appreciate if any company could provide the source of spec text describing the existing “collision rules”. @Panasonic, Thanks a lot for providing a spec reference. HD-UE is a half-duplex RedCap UE operated on FDD bands only. However, this WI enhancement targets at TDD bands only because it is only for FR2. Therefore, it seems not the collision rules discussed by the current proposal. We think the rule that handles the potential collision between legacy PRACH resources and R18 PRACH resources that are mapped to different SSB at the same symbols, especially for gNBs with single simultaneous receiving analog beams, should be supported. The reason is given as follows. If Rel-17 framework for PRACH configuration is reused in Rel-18, then it is almost impossible for gNB to avoid overlapping in time between Rel-18 new ROs and legacy ROs. As shown in the following figures, a gNB has to configure one IE prach-ConfigurationIndex for legacy RO resource and another IE prach-ConfigurationIndex for new RO resources. Both PRACH indices refer to the same PRACH table. For a given format, e.g. C2, most of existing indices have slot#39, many of them have slot#35 or #34. It is worthy noting that only a small sub-set of C2 format can be configured for a TDD UL/DL configuration chosen by operators, e.g. only index #187, #188, #195 and #196 with a periodicity of 5 slots are suitable for TDD UL/DL configuration DDDSU. Therefore, it is impossible for a gNB to avoid the overlapping between new RO and legacy RO by indicating different PRACH indices. TS38.331 Two RO configurations are configured under BWP-UplinkCommon as illustrated below, Legacy RO configuration => prach-ConfigurationIndex New RO configuration=> prach-ConfigurationIndex (assuming Rel-17 framework) TS38.211 If overlapping ROs are always associated with the same SSB, then it would not put a restriction on the number of simultaneously gNB receiving beams, which can be achieved by requiring the new RPACH to guarantee the same PRACH index and the exactly same SSB-RO mapping pattern as the legacy PRACH. However, it seems not the case in this WI because multiple PRACH transmissions are associated with the same SSB but span multiple slots, which is different from previous PRACH features. Maybe FL could set up a question to get feedback from all companies whether it is agreeable that the new RO and the legacy RO should be always mapped into the same SSB when the two RO overlaps in time. The collision due to different SSB beams also seems to occur very frequently according to the Table 6.3.3.2.4. When the number of different SSB beams required by overlapping ROs exceeds the number of simultaneous analog beams of gNBs, especially for gNBs with single simultaneous receiving analog beams, the number of actual valid RO in a RO group reduces. Therefore, the number of actual valid RO in a RO group could be very far less than expected. If a UE is not aware of it, then the actual number of PRACH repetition may be far from what the RSRP threshold for the PRACH repetition expects. It could arise a terrible result, i.e., UE selects the only one RO group of largest repetition level, containing some invalid ROs, it could persistently fail due to the periodic SSB-RO group association pattern. But, if UE is aware of it, it can set a proper initial transmission power to get access to network. |

Issue 2-2. How an RO group is determined.

Proposal 2-X

Issue 2-1. Whether to associate RO group pattern with the SSB-to-RO association pattern period, so that we only need to focus on the RO group design within one SSB-to-RO association pattern period.

FL comment: FL wants to first make some clarifications for this issue. Based on current spec., an association pattern period is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. Since RO group consists of valid ROs associated with the same SSB, a nature thought would be the pattern between RO group and ROs repeats in the same way as pointed out by some companies. Then, we only need to determine/configure RO group in one association pattern period, and the pattern is repeated align with the association pattern period, which can reduce the workload.

Based on the offline discussion, some companies show concerns on the limitation of one SSB-to-RO association pattern period. For example, if the SSB-to-RO association pattern period is too short that the ROs within the period is not enough for, e.g., 8 PRACH transmission. In addition, some company propose to consider RO group determination within K SSB-to-RO association period. From FL perspective, the spirit of them are the same, which is to determine a time period, e.g., called “RO group pattern period”, so that the pattern between RO group and RO can be repeated to make the design easier.

Some quick clarification regarding companies’ comments in the 2nd round discussion.

@ Panasonic, as pointed out by Ericsson, the definition of an association period start from frame 0. In addition, since companies have different views about whether to use K SSB-to-RO association period or K SSB-to-RO association pattern period, it is better we replace them with a time period, e.g., “RO group pattern period”.

@ Sharp, yes, there maybe multiple RO groups within a RO group pattern period.

@ CATT, Spreadtrum, I think the FFS goes for both separate or shared case, since we don’t have an agreement to introduce new SSB-to-RO mapping mechanism, thus we need to consider a unified defination for both cases.

@ CMCC, from FL perspective, to define RO group within one PRACH configuration period is not reasonable, since there may be not enough even for one entire mapping cycle between SSB and RO.

@Intel, it is just a name and I think with the note, it is not a big deal. In addition, FL suggests to keep the details of the FFS, it really takes me a lot of time to work out such description. Anyway, it just provide some information, it doesn't harm anything, right?

Proposal 2-X-1-1

RO group is [determined/configured] within an RO group pattern period, starting from frame 0, and the  [determined/configured] RO group pattern repeats in every RO group pattern period.

  - FFS: whether RO group pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period.

    - K is a positive integer.

    - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period.

    - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213.

  - Note: whether to define “RO group pattern period” in specification will be discussed separately

| Companies | Comments |
| --- | --- |
| Ericsson | Support. |
| ZTE | Fine with the proposal. Maybe determined is better than configured, UE can indirectly derive the RO group location and the mapping relationship with SSB by the configured parameters, such as PRACH configuration index, SSB information, the mapping parameters, etc. |
| LG | Agree in principle. On the other hand, is it correct understanding that the “RO group pattern period” in this proposal is a cell common single value regardless of repetition number? |
| DOCOMO | Fine with the proposal. |
| Mavenir | Fine with the proposal. |
| Panasonic | Thank you, FL, for your reply to our comment. We are fine to introduce “an RO group pattern period”. However, it is not clear to us about this term “RO group pattern” because it is not defined in Proposal 2-X-1-1. In addition, @FL, can you clarify the meaning of “RO group pattern repeats in every RO group pattern period”? This statement will raise a question about how many times need to be repeated? |
| Sharp | We support the FL proposal. |
| Nokia/NSB | Support the FL’s proposal. Appreciate the fact that we are keeping the [configuration/determination] discussion open. We provide further details on the “configured RO groups” in our comment to Proposal 2-X-2-1 |
| NewH3C | OK |
| CATT | Same comment as before. We think FFS is only applicable if existing SSB-to-RO mapping is used. But we prefer new SSB-to-RO mapping for separate RO case. We are fine with the main bullet. |
| Lenovo | We are fine with the proposal. |
| Spreadtrum | Thanks FL’s clarification. We are fine with this proposal. We have a confusion that whether different RO groups within a RO group pattern period can associated only with the same SSB or associated with different SSB. |
| Fujitsu | We are fine with the proposal. |
| CMCC2 | Thanks FL’s for the clarification. We may share a similar view that other companies that we would hesitate to introduce a new definition which may introduce different understandings. The RO group pattern period would bring imaginations about whether there is an additional pattens under a same period. If the most companies have a same understanding on the proposal and would like to support this, we would like to move further to have more consolidated and clear description as below, Updated: RO group is [determined/configured] within an RO group pattern period of K SSB-to-RO association pattern periods, starting from frame 0, and the [determined/configured] RO groups pattern repeats in every RO group pattern period K SSB-to-RO association pattern periods. - FFS: whether RO group pattern period is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. Or if the proponents still want to discuss and make a choice between by K SSB-to-RO association period or K SSB-to-RO association pattern period, we can just replace it wither a Period X. The updates is as below. RO group is [determined/configured] within a period X an RO group pattern period, starting from frame 0, and the [determined/configured] RO group pattern repeats in every RO group pattern period period X. - FFS: whether RO group pattern period the period X is replaced by K SSB-to-RO association period or K SSB-to-RO association pattern period. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. - Note: whether to define “RO group pattern period” in specification will be discussed separately If the concept is simple as period X, and can be replaced by either K SSB-to-RO association period or K SSB-to-RO association pattern period, there is no need to define it in the spec. |
| Sony | This RO Group pattern is a new term and it will be good to know how a RO Group pattern looks like. Does the duration of a RO Group pattern equals to the longest RO Group (i.e. for 8x repetition)? Or can we have TDM-ed RO Groups, i.e. we have 2 x RO Group with 8x repetition? |
| IDCC | Fine with the proposal. |
| Intel | We still think RO group association pattern period would be proper terminology, similar to association pattern period as defined in 213. For the FFS, we do not think it is necessary to provide example for K. We can further study how to determine K. |
| Qualcomm | Fine with the proposal |
| Samsung | We are fine the proposal in general. We share intel’s view on the term in fine, since it can be replaced in other words if it’s specified or not. The key point if the function of such “period” as asked by CMCC. We think the key function is that we will have a period that such determined RO group pattern can repeat every such period, so it’s in the main bullet. The second function is that such RO group pattern contains at least the ROs which can complete at least one RO group for the largest N value configured by gNB, which is in FFS point. The FFS is more on the exact value, which is not to FFS such function. One wording change in green: RO group is [determined/configured] within an RO group pattern period, starting from frame 0, and the [determined/configured] RO group pattern repeats in every RO group pattern period. - FFS: whether RO group pattern period is replaced by consisting of K SSB-to-RO association period or K SSB-to-RO association pattern period. - K is a positive integer. - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period. - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213. - Note: whether to define “RO group pattern period” in specification will be discussed separately |
| Lenovo2 | Given that SSB to RO group association might be defined, it might be better to use the terminology “SSB to RO group association pattern period” than “RO group pattern period”, which is aligned with legacy terminology. So, we suggest adding an FFS, “FFS: whether “RO group pattern period” is replaced by other terminology, e.g., “SSB to RO group association pattern period”, depending on SSB to RO association for PRACH repetition. |
| Apple | Similar comments as Sony, it could be better to provide the explanation of the term ‘RO group pattern’. For example, four PRACH transmission case, the RO group pattern means the first four TDM ROs if the RO group pattern period provides 8 ROs? |

Issue 2-2. How an RO group is determined.

FL comment: Based on companies’ comment, it can be seen that majority companies’ views on the spirit of RO group determination is kind of aligned as: Firstly, determine the first RO of an RO group; Secondly, determine the subsequent ROs according to whether frequency hopping is supported or not.

Based on the offline discussion, FL check companies views on how RO group is realized. Companies’ views are summarized as follows:

A. RO group is explicitly configured by the network.

Support (5): Nokia, Sony, Panasonic, LG, CMCC

B. RO group is implicitly derived/determined based on network configuration.

Support (16): Samsung, Qualcomm, Intel, CATT, vivo, Spreadtrum, Ericsson, Huawei, ZTE, OPPO, MediaTek, Sharp, Xiaomi, DOCOMO, Fujitsu, ETRI

Thus, we may need to work this out in RAN1. Regarding how an RO group is determined. Companies please provide your comment on the following proposal.

Proposal 2-X-2-1

- If multiple PRACH transmission with frequency hopping is supported, RO group is determined by the following parameters {time and frequency start position (or start RO), frequency offset of second hop, the number of ROs within the RO group}.

  - FFS: the unit of frequency offset.

  - Note: Frequency hopping is performed within the configured ROs.

- If multiple PRACH transmission with frequency hopping is not supported, RO group is determined by the following parameters {time and frequency start position (or start RO), the number of ROs within the RO group}.

| Companies | Comments |
| --- | --- |
| Ericsson | If frequency hopping is supported by 3GPP and becomes an optional UE feature, for a given number of PRACH transmissions, there would be different RO groups and possibly preamble partitions for UEs supporting FH and UEs not supporting it. Literally, this intertwines with the FFS of Proposal 2-X-4-1 v1, copied below. Proposal 2-X-4-1 v1 [omitted] - FFS: whether RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions. Two outcomes of the FFS are illustrated below, where each arrow links two ROs of a RO group. If RO groups of the same length have no shared RO, RO group of FH and RO group of non-FH have to be FDMed (illustrated in Figure 1) or TDMed, requiring more RO resources. Otherwise, with a shared RO between RO group of FH and RO group of non-FH, separate preamble partitions are needed as illustrated in Figure 2. We suggest the following. Proposal 2-X-2-1 - If multiple PRACH transmission with frequency hopping is supported, RO group is determined by the following parameters {time and frequency start position (or start RO), frequency offset of second hop, the number of ROs within the RO group}. - FFS: the unit of frequency offset. - FFS: the differentiation between an RO group of frequency hopping and an RO group of non-frequency hopping - Note: Frequency hopping is performed within the configured ROs. - If multiple PRACH transmission with frequency hopping is not supported, RO group is determined by the following parameters {time and frequency start position (or start RO), the number of ROs within the RO group}. |
| ZTE | frequency offset of second hop -->> frequency offset of next hop Just to confirm the FH is only processed in RO group? Or the FH can across the two adjacent RO groups, if there are at least two RO groups corresponding to the same SSB within one RO group pattern period? |
| LG | Agree in principle but we think Proposal 2-X-1-1 should be discussed first. Regarding this proposal, it can be considered that the frequency position of starting RO may be randomly selected by the UE. In addition, we think that “the repetition number determined by UE” should be included in the parameters required for RO group determination. |
| DOCOMO | Fine with the principle of the proposal. Just one clarification question, is the proposal based on the assumption that there are multiple FDMed ROs for the same SSB/CSI-RS in the same time instance, and the frequency offset represents frequency index offset instead of RB offset (like PUCCH/PUSCH FH), is above correct understanding? |
| Mavenir | We support that the RO group is implicitly derived/determined based on network configuration. We support the first bullet of this Proposal, i.e. frequency hopping support case, which can be baseline for further specification. And further clarification may be needed. Does the ‘frequency hopping for multiple PRACH transmission’ mean the frequency domain positions of two consecutive RO in one RO group are shift by an offset if configured? |
| Panasonic | For the case of supporting FH in the first bullet, we think that it is necessary to define either a length of a frequency hop or a number of ROs (from the total number of ROs within a RO group) allocated in a frequency hop such that a UE can determine how many ROs to be used in a frequency hop. |
| Nokia/NSB | We do not support this proposal in its current form because it only relies on RO group determination, which in our view is a largely suboptimal approach for this features and exposes gNB to a lot of implementation constraints and limitations. Please note that this is also hinted at by Ericsson’s comments which highlight how challening it is to work out a RAN1-only mechanism for FH which does not imply a lot of performance trade-offs and implementation constraints. At trhe same time, we understand that the notion of “configuration up to RAN2” can sound vague and does not explain much, hence we would like to provide a simple example of a possible configuration of RO groups which can reuse mechanisms already existing in the specification (where details can be left, anyway, to RAN2). Once the raster of ROs has been determined by UE depending on PRACH configuration index, ssb-perRACH-OccasionAndCB-PreamblesPerSSB and msg1-FDM, all valid ROs belonging to a same SSB index can be enumerated by UEs following the natural ordering provided in TS 38.213, frequency time slot. At this stage the UE only needs to know which ROs are grouped together to determine one or more RO groups based on an additional higher-layer configuration (the RO group configuration). The problem is identical to what is found in the MU-CSI codebook in TS 38.214, where UE needs to report to gNB which spatial components and frequency components, out of a set of available ones, have been used to compress the CSI and obtain the content of the CSI report (e.g., Type 2). To this end TS 38.214 uses a combinatorial indicator providing the rank of the chosen component combination out of all the possible combination, which is well known efficient signaling scheme to convey such information. Similarly, in case of RO group configuration, NW could provide a set of combinatorial indicators providing the rank of one or more combinations of ROs out of all the possible combinations of ROs, wherein one combination indicates one group of ROs. In other words, this could be done using approaches already existing in the specification and logics already implemented in the products, and with low redundancy in SIB1. The effort would be minimal, if any. Tools and expertise already exist. RAN2 may eventually decide to resort to other possibly more efficient mechanisms of course, e.g., bitmaps, explicit enumeration and so on. What I wrote above should be considered only as an example for the sake of this discussion. The final outcome may indeed depend on whether groups can only consist of “ROs consecutively mapped to same SSB” or whether some gaps can exist between them, i.e., considering the first 5 ROs mapped to SSB#0 in an association pattern period, whether a group of 4 ROs can only be formed by: - [RO#0 RO#1 RO#2 RO#3] - [RO#1 RO#2 RO#3 RO#4] Or can also be, say, [RO#0 R0#2 RO#3 RO#4], i.e. RO#1 is skipped, for instance to avoid collisions. Now, what would the advantges of such approach be? Configuration of RO groups from gNB is necessary to allow a network to control and reduce at will the collision probability of the PRACH repetitions and of the gNB detection complexity based on deployment scenarios. Indeed, the larger the number of RO groups, the larger the number of hypotheses a gNB must test during PRACH to understand whether PRACH repetitions were transmitted in an RO group or not. Furthermore, via an explicit configuration of the RO groups, a gNB can autonomously determine for example the number of RO groups to configure based on aspects such as: - Its detection capabilities. - Receive buffer size - RO combination algorithm, if any (coherent, non-coherent) - Possible estimates of the number of UEs benefiting from specific number of repetitions, e.g., via UL RSRP measurements and so on. Configuration allows to realize: - Arbitrary frequency hopping patterns (see example below), without complicating the RAN1 spec with complex rules for the frequency hopping, by simply grouping ROs together as gNB deems fit. - Create very efficient patterns of RO groups which can occupy the least amount of ROs possible while guaranteeing arbitrary time and frequency diversity. - Reduce at will the overall number of ROs used across the groups by configuring any level of sharing between RO groups, partial with one or more shared ROs, complete sharing and so on - Reduce the need for additional collision handling rules significantly, since configuration would allow a much finer granularity for the groups and collisions could be avoided by design. Going for RAN1-only RO determination instead strips gNB of all these possibilities and de facto forces unique implementation choices for realizing the feature. This should be avoided in RAN1, since the spec should not mandate implementation implicitly or explicitly. Moreover, it would complicate the realization of all the operations I described above such as FH, partial RO sharing, collision handling and so on. Overall, we would like to invite companies to take some time and think about all the implications of this decision, which are huge. In this context, we are not against the RO group determination as such but are not ok with having it as the only possibility to realize the RO groups. This would be technically unjustified and philosophically against what we typically do in RAN1. Example of simple but arbitrary FH pattern (no overlap): |
| New H3C | Fine with the principle of the proposal |
| CATT | For the note, we need more discussions to decide whether FH is within configured ROs or not. |
| Lenovo | Maybe we can try the second bullet (i.e., w/o hopping) at first. It is not clear yet how hopping is performed, which may depend on how SSB to RO is associated. |
| Spreadtrum | We are fine with the modification with ZTE. If frequency hopping is supported, the unit of frequency offset can be absolute number of RBs, or ROs. In addition, we also need to consider that whether the frequency offset is a positive or negative value. For example, if the current RO is the last RO in the frequency domain. In this case, the frequency offset of next hop must be a negative value. |
| CMCC2 | More details should be discussed for this proposal. Our understanding is we should have a design of RO group without frequency hopping first. Anyway, if supported, frequency hopping of the PRACH transmission should be an optional UE feature. Thus, we should finish the basic design without FH first. If the FH for the multiple PRACH transmission is supported, a unified design of RO groups with and without FH should be supported. |
| Sony | From E///’s comment it seems the RO Group also needs to identify the set of preambles as it seems there is a possibility that two RO Groups may collide in some hops, even if they start with different RO Groups. If collision among frequency hopped ROs is an issue perhaps Nokia’s suggestion of providing flexibility for gNB to configure each RO for a RO Group is beneficial, if it doesn’t consumes too high overhead. |
| Intel | Our view is that frequency hopping may be configured when there are FDM’ed RO or RO groups for multiple PRACH transmissions, as shown in the figure below. In this case, a clarification on the meaning of “determined” in the proposal: in case of multiple FDM’ed RO groups, does that mean UE will randomly select or determine a starting RO similar to what was defined for single PRACH transmission? |
| Samsung | We are not supportive to have FH design without knowing the RO group pattern determination (Nokia’s way is one to determine from our point of view). Suggest to focus on other issues. |
| Apple | In our understanding, the current listed parameters are not enough to derive the RO group position in time domain. The parameter can only configure the first RO group relative to SFN0, so the distance or offset between two RO groups need to be configured as well. |

Issue 2-3. Whether/how multiple PRACH transmissions with frequency hopping is supported?

Support: Panasonic, DOCOMO, Spreadtrum, Quectel, Sony, OPPO, Intel

[ZTE] the RO group design without FH can be as baseline.

[ETRI] We prefer to conclude firstly whether to support frequency hopping. If freq hopping is supported, then it needs clarified whether frequency hoping within ROs or ROs are shifted in frequency domain.

[Apple, Samsung] delay the FH related decision.

FL comment: Based on companies’ comments, FL suggest we postpone the FH related discussion.

Issue 2-4. Whether an RO can be shared between different RO groups for different/same number of multiple PRACH transmissions. This can be realized by set proper start position of each RO group.

FL comment: Companies’ views are summarized as follows. The initial intention of the sentence “if shared…” is considering that we have an agreement that “If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.” But as Nokia point out, the RO sharing including fully shared and partially shared, if ROs are partially shared maybe separate preambles is not needed.

Proposal 2-X-4-1

- RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation.

Support (16): Qualcomm, IDCC, Intel, Quectel, OPPO, Sony, Lenovo, Fujitsu, CATT, New H3C,Sharp, Panasonic, Samsung, Mavenir, DOCOMO, ZTE

Not support (1): CMCC

- RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions.

Support (13): Qualcomm, IDCC, Intel, OPPO, Spreadtrum, CMCC, Fujitsu, CATT, New H3C, Sharp, Panasonic, Mavenir, DOCOMO

Not support (3): Nokia, NSB, Lenovo

Thus, companies are encouraged to consider whether sperate preambles are needed if partially ROs are shared between RO groups for different numbers of multiple PRACH transmissions. Currently, FL suggest we put this into FFS as follows.

Proposal 2-X-4-1 v1

- RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation.

- FFS: whether separate preambles are utilized for differentiation.

- FFS: whether RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions.

| Companies | Comments |
| --- | --- |
| Ericsson | As commented to Proposal 2-X-2-1, if frequency hopping is supported, we think shared RO with different preamble partitions is worth considering in order to differentiate between UEs supporting FH and those not. Differently, as per our understanding, the intention of 2nd FFS in Proposal 2-X-4-1 v1 is that the same preamble in an RO can be shared between RO groups of the same length. If it is correct, we suggest the following in blue. Proposal 2-X-4-1 v1 - RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are utilized for differentiation. - FFS: whether separate preambles are utilized for differentiation. - FFS: whether the same preamble in RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions. |
| ZTE | Separate preambles are needed for differentiation even if the partial shared case. Maybe there is other possible way to differentiate but way of separate preamble is the simplest and has been supported by the R17 feature combination. |
| LG | It is not clear how RO can be partially shared between different numbers of multiple PRACH transmissions. On the other hands, we think it is beneficial to discuss the RACH resource partitioning between different numbers of multiple PRACCH transmissions. If a similar approach discussed in two confirmed working assumptions is applied, following options would be the base line when multiple repetition values (e.g., R1, R2) are configured in a cell. - Option 1: Multiple PRACH with R1 are transmitted with a separate preamble on shared ROs with multiple PRACH with R2 using FeatureCombination-r17 in either legacy configuration or AdditionalRACH-Config-r17. - Option 2: Multiple PRACH with R1 are transmitted on separate ROs with multiple PRACH with R2 using FeatureCombination-r17 in AdditionalRACH-Config-r17. |
| DOCOMO | Fine with the principle of the proposal. We think separate preambles to differentiate different number of PRACH transmissions with shared RO resource is necessary. If shared ROs for RO groups of the same number of PRACH transmissions, further preamble resource partition is not preferred since there would be too many preamble resource partitions. If this direction is applied, we think careful design on configuring the partially overlapped RO groups would be needed. And potential gNB decoding complexity may be resulted. |
| Mavenir | We are fine with the proposal. Regarding the Nokia’s comments, for example, when one RO group with 2 ROs is overlapping with one RO group with 4 ROs (i.e. nested hierarchy for RO group configuration), if no separated preambles used to differentiate 2 PRACH transmissions from 4 transmissions, it may happen that the UE triggered 2 PRACH transmission over 2 overlapped ROs and started the RAR window after end of these 2 ROs, however gNB don’t know and may believe this RACH attempt is for 4 PRACH transmissions (since the 2 ROs are within a 4-RO group), thus would wait 2 more sequential ROs before replying RAR. In our view, it would increase the random access delay. |
| Panasonic | It seems the proposal 2-X-4-1 v1 goes one step back as compared with the previous version. We are fine with it if majority view prefers to do so. |
| Nokia/NSB | @Mavenir: that’s correct. The use-case you describe can only work with separate preamble. However, if only one RO is shared between the two groups, the same preamble can be used and gNB can differentiate the two UEs by leveraging the time correlation between the ROs of the same group. For instance, if a preamble is detected in a shared RO between group A and group B, gNB cannot know whether this is a preamble for group A or for group B or wtehre there are two UEs, one using group A and other using group B. However, by just looking at other ROs of two groups, gNB can easily understand what is going on, because ROs in the same group are not just random ROs, but are ROs related to each other. It is the whole point of having the group! It is unclear why we should introduce the group and then ask gNB not to be smart and avoid making use of all the advantages brought by the groups themselves. This is very counterintuitive to us and would force only “less smart” gNB implementations to be possible. This would be very bad for both NW vendors and operators. We are fine with Proposal 2-X-4-1 v1. Maybe we can reverse the order of the two FFS points to address Ericsson’s concern. |
| New H3C | OK |
| CATT | If different numbers of repetition share the same ROs, we think separate preambles are needed for differentiation. |
| Lenovo | We are fine with the proposal. |
| Spreadtrum | We share the same view with LG that how RO can be partially shared between different numbers of multiple PRACH transmissions. |
| Fujitsu | For the first FFS in the sub-bullet, we think that the separate preamble is needed on shared ROs. As Mavenir mentioned, especially for the case that the one RO group is totally included in the another RO group, gNB has a difficulty to detect the PRACH. |
| CMCC2 | Thanks FL’s updates searching for further compromise. As mentioned by E/// that the FH could be benefited by the share ROs. As we commented for the last proposal, the RO group design is the first priority issue. A common design should be supported for the RO groups with FH and without FH. Then we suggest to solve the Proposal 2-X-2-1 first. And furthermore, if shared ROs with different number of repetitions are supported, the preambles should be used to differentiate different repetition number within the partially shared RO groups. Considering we already have the portioning of the preambles for different usage, further partitioning for different repetitions may not be preferred. And further we have confirmed two working assumptions at the beginning of this meeting, we should put the proposal under both scenarios. We propose more discussions are needed. |
| Sony | Fine with proposal. If different numbers of repetitions share the same ROs, we think it can still use same set of preambles. The gNB can always attempt to blind decode (NOTE: All PRACHs are blind decoded anyway) for a PRACH after the end of an RO Group. For example in the figure below we can have RO Group 1 and RO Group 2 with 2x and 4x repetitions respectively. Since gNB knows when RO Group 1 ends, it knows when to combine the signal and attempt to decode it. Same thing with RO Group 2, gNB knows exactly when to start decoding it since it knows exactly when it ends. Hence there isn’t any really a need to further partition preambles but if the gNB wants to partition it, it can do so. Whether to partition preambles should be a configuration decision at the gNB rather than set in stone in the specs. |
| IDCC | Fine with the proposal. |
| Intel | It is not clear to us why “separate preambles” are in the FFS. Without separate preambles in case of shared ROs, how can we differentiate different numbers of multiple PRACH transmissions? It may result in consistent collision. |
| Qualcomm | Fine with the proposal |
| Samsung | We think the first FFS should be the case, we see Nokia’s comment on some potential gNB handling can solve the problem, but it is not fine to mandate the behavior at gNB and also some configuration. So we suggest some change in green: - RO(s) can be shared between RO groups for different numbers of multiple PRACH transmissions, if shared, separate preambles are can be utilized for differentiation. - FFS: whether separate preambles other methods can be are utilized for differentiation. - FFS: whether RO(s) cannot be shared between RO groups for the same number of multiple PRACH transmissions. |
| Apple | If the ROs are shared, separate preambles are needed. The main bullet should keep as it is. |
| Huawei, HiSilicon | Considering the limited capability of blind detection of gNB, when ROs are shared between RO groups for different numbers of multiple PRACH transmissions, separate preambles should be utilized for differentiation. |

Issue #3: Same or different preamble(s) during multiple PRACH transmission

Proposal 3-1

Proposed conclusion

There is no consensus to support utilizing different preambles during the multiple PRACH transmissions with the same Tx beam in one attempt.

To reduce companies’ work load. Please provide your comment only if you have some concerns.

| Companies | Comments |
| --- | --- |
| ZTE | We can live up with this proposal for sake of progress. |
| LG | Support |
| DOCOMO | Support. |
| Sharp | We support the updated proposal. Thanks. |
| New H3C | OK |
| CATT | Support |
| Spreadtrum | Support. |
| Sony | Support |
| Qualcomm | Support |
| Samsung | Support |
| Apple | Support |

Issue #4: SSB-to-RO mapping (Postponed)

FL comment: Based on companies’ comments, companies’ views are summarized as follows:

Introduce a new SSB-to-RO mapping mechanism for multiple PRACH transmissions on separate ROs with single PRACH transmission.

Support (19): CATT, Panasonic, LG, Apple, DOCOMO, Lenovo, Mavenir, ZTE, TCL, Spreadtrum, ETRI, Sharp, Fujitsu, Quectel, Sony, Qualcomm, OPPO, Intel, New H3C

Not support (3): Ericsson, CMCC, vivo

Option 1: SSBs are mapped to a set of valid ROs in time domain, while the mapping order is aligned with current SSB-to-RO mapping.

Support (12): Panasonic, LG, Mavenir,ZTE, TCL, Spreadtrum, ETRI, Sharp, Fujitsu, Quectel, Qualcomm, New H3C

Option 2: SSBs are mapped to valid PRACH occasions in the following order

- First, in increasing order of preamble indexes within a single PRACH occasion

- Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

- Third, in increasing order of indexes for PRACH slots

- Fourth, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

Support (2): TCL, OPPO

Companies’ concerns are copied as follows:

[CMCC] Introducing new SSB-to-RO mapping mechanism may have difficulty about compatible with the legacy feature using PRACH for early identification. If we confirm both WAs for separate RO and shared RO, then it should be discussed unified mechanism for both situations.

[vivo] We can understand the intention of the update of SSB to RO mapping. However, when SSBs are mapped to a group of ROs in time domain first, it would mean the SSB to RO mapping cycle is also increased and the SSB to RO association period would be increased as well. For a UE selecting one SSB that is mapped to ROs quite late in the association period, the latency is not reduced by increased.

[Ericsson] This new SSB-to-RO mapping rule can only work in the case that all preambles in an RO are dedicated to multiple PRACH transmissions. But this is a rare gNB configuration, causing low preamble utilization. More often, an RO, either a legacy RO or a separate RO, may have several preamble partitions for features, like multiple PRACH transmissions, Msg3 repetition, RedCap, etc. In this case, the new SSB-to-RO mapping rule doesn't work. We advise to focus on the most common RO/preamble configuration.

On the other hand, if the intention is about low latency of multiple PRACH transmissions, PRACH configuration index, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and msg1-FDM are separately configured for separate/additional ROs in Rel-16/17. Even if a separate RO is purely for multiple PRACH transmissions, gNB configuration of these parameters can allow low latency for TDMed PRACH transmissions based on legacy SSB-to-RO mapping. If the intention of the proposal is to allow a UE to determine TDMed ROs for PRACH transmissions, this can be discussed in RO group, which consists of TDMed ROs.

[Nokia/NSB] We think this discussion can occur after the design of RO groups is finalized. Until then it is very hard to understand whether and how this should be done.

FL comment: Based on current situation, FL suggest we postpone the discussion.

6.1.2 RAR window and RA-RNTI calculation

Issue #5: RAR window and RA-RNTI

FL comment: Sorry for cause the confusion, the difference of proposal 5-1-a and 5-1-b is considering the case that if the PRACH transmission on the last RO is dropped. If this happens, gNB may not know cause the dropping may depend on UE behavior, e.g., Red-cap UE. To align the gNB’s and UE’s understanding about the starting position of RAR window, the last valid RO needs to be considered as the reference point.

Take ZTE’s comment into account, FL suggest we simply the proposal as following.

Proposal 5-1v1

The starting point of RAR window is after the last symbol of the last valid RO corresponding to the multiple PRACH transmissions.

Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213

| Companies | Comments |
| --- | --- |
| Ericsson | It would be clearer to add “of a RACH attempt” at the end of the first sentence. Another suggestion is that if Proposal 2-2v2 can be agreed, the term RO group can be used here. |
| ZTE | Fine with the proposal. Even if the RO is dropped, the dropped RO can be regarded as the nominal RO as the reference of RAR window starting. |
| LG | Support. In addition, Ericsson’s second suggestion would be fine. |
| DOCOMO | Fine with the proposal. |
| Mavenir | Support the proposal. |
| Panasonic | We agree FL that the last valid RO can be dropped as one of the examples is RedCap UE in half duplex operation. The alignment of gNB’s and UE’s understanding about the starting position of RAR window is essential. Our understanding is, even with the current usage of valid RO, the valid RO can be dropped. Therefore, we propose to clarify it. Proposal 5-1v1 The starting point of RAR window is after the last symbol of the last valid RO corresponding to the multiple PRACH transmissions, regardless of that the last valid RO is dropped or not. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213 |
| Sharp | We support the FL proposal. |
| Nokia/NSB | Ok with the proposal, also with Panasonic’s change it this helps. |
| New H3C | OK |
| CATT | Support |
| Lenovo | We are fine with the proposal. |
| Spreadtrum | Support this proposal. |
| Fujitsu | We are fine with the proposal. |
| CMCC2 | The updates from E/// and Panasonic are fine to us. We share the same views that if we have a same understanding of the RO group, we can use it here. Base on Panasonic’s version, the updates is below. Proposal 5-1v1 The starting point of RAR window is after the last symbol of the last valid RO within the RO group corresponding to the multiple PRACH transmissions, regardless of that the last valid RO is dropped or not. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213 |
| Sony | To cater for the issue of dropped RO, maybe we just want to say the “last configured RO”. The starting point of RAR window is after the last symbol of the last configured valid RO corresponding to the multiple PRACH transmissions. |
| IDCC | Support. |
| Intel | We are fine with the proposal. |
| Qualcomm | Support |
| Samsung | Support, wording suggestion in green: Proposal 5-1v1 The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213 This suggestion can solve the problem on whether drop is happened, which targeting the resource part, it aligns with “regardless drop or not” |
| Apple | Support |

6.1.3 Determine the number of multiple PRACH transmissions

Issue #6: Determination of the number of multiple PRACH transmissions

FL comment: Based on companies comment, it seems the original proposal 6 is not that necessary considering that we already had an agreement that “at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt”.

The critical point is whether other factors are considered besides SSB-RSRP threshold(s) to determine the number of PRACH transmissions.

Based on companies’ comments in the 1st round, companies’ views are summarized as follows:

Not support other factors (13): CATT, LG, Apple (needs to justify the necessity), DOCOMO, ZTE, Spreadtrum, Sharp, CMCC, Sony, vivo, MediaTek, OPPO, Intel

Support consider UE power class: Panasonic, Qualcomm

Support to consider MPE condition: Qualcomm, Samsung

Support Power headroom threshold: Ericsson

@Nokia, from FL perspective, tolerance zone can be discussed in retransmission related issues, e.g., how to decide the number of multiple PRACH transmission in Re-attempts.

I’ll leave a door open for further discussion about whether other factor besides SSB-RSRP is considered to determine the number of PRACH transmissions in 2nd round email discussion. The proposal will be provided in next round.

| Companies | Comments |
| --- | --- |
| LG | Agree with FL comment. |
| Nokia/NSB | Ok (thx FL for the explanation, we are fine with the suggested approach) |

6.1.4 Power control

Issue #7: Power calculation

Proposal 7-1-a

The same measurement of the same reference signal to calculate the pathloss should be applied for each of the multiple PRACH transmissions in one attempt.

Support (11): Panasonic, LG, Apple, Mavenir, IDCC, Spreadtrum, Fujitsu, Sharp, MediaTek, OPPO, New H3C

Concerns (1):

[CATT] seems to imply that UE needs to calculate transmission power for each repetition independently although the results would be the same.

Proposal 7-1-b

Same transmit power is applied for multiple PRACH transmissions within one RACH attempt.

Support (17): CATT, Panasonic, DOCOMO, Lenovo, Mavenir, TCL, IDCC, Spreadtrum, ETRI, Sharp, Fujitsu, Qualcomm, vivo, MediaTek, OPPO, Intel, New H3C

Concerns (2):

[Apple]we are not sure whether same transmit power can be kept considering RAN4 required MPR and SAR limitations with multiple transmissions.

[CMCC] For the 7-1-b, in some cases UE may not have ability to keep the same power, which seems putting too much restriction to UE.

FL comment: Based on companies’ comment, FL suggest we goes with Proposal 7-1-b, maybe some wording refinement is needed.

@ CMCC, anyway we need to determine the transmission power of each PRACH transmission. Since power ramping within a RACH attempt is not supported, the thing left is whether the measurement of the reference signal is the same or not. Just as ZTE mentioned, another way is not to constrain the same measurement and same transmitting power as multiple PRACH transmission may span the period of the reference signal measurement. But based on the contributions, I think the majority companies think we goes the current discussion.

@ Nokia, I’m not sure companies are supportive to use max power to transmit each repetition.

Companies are encouraged to address CMCC and Apple’s concerns and discuss whether the max power is used to transmit each PRACH within one RACH attempt.

| Companies | Comments |
| --- | --- |
| Ericsson | We agree with Nokia that multiple PRACH transmissions is triggered when a UE has zero or a negative power headroom. Then it would transmit multiple PRACHs with its Pcmax. Pathloss is used in legacy for the determination of PRACH transmission power. So, Proposal 7-1-a is better than Proposal 7-1-b. |
| ZTE | I am open to this contribution. I also have the same view with Nokia, that it is better for UE to transmit with the maximum power as we mentioned the reason in previous meeting. But this is the issue of how to calculate the transmitting power, and this proposal 7-1-b seems not contradictory with the maximum transmitting power? |
| LG | Agree with Ericsson. We prefer to support Proposal 7-1-a. |
| DOCOMO | Support Proposal 7-1-b. |
| Nokia/NSB | Glad to see other companies share our understanding. Slight preference for Proposal 7-1-a due to what Ericsson explained. |
| New H3C | OK |
| Spreadtrum | Support Proposal 7-1-b. |
| CMCC2 | Share similar idea with Nokia and Ericsson that when multiple PRACH transmission is activated or requested, UE should have zero or negative power headroom. But for the proposal 7-1-a, we share a same feeling that it implies a multiple measurement within a multiple PRACH transmission. For the legacy behaviour, UE determine the transmit power for the 1st transmission. And if the PRACH transmission failed, a power ramping up happens. The power ramping up is based on the previous determined transmit power, but not the PL measurement. We may first clarify the procedure and find a proper description. |
| Sony | Typically, a UE would only use repetitions if it hits the max power, otherwise what’s the point of occupying extra ROs and prolonging a transmission when it could be done with just one RO. Hence, we do share similar views with Nokia on using max power. However, I believe the proposal is not about whether UE uses max power but how UE determines what power to use, and here we prefer Proposal 7-1-a as it offer more details on what are used to determine the power. |
| Qualcomm | Support Proposal 7-1-b. |
| Samsung | A suggested change on -a trying to address the concern from CMCC Proposal 7-1-a The same single measurement of the same reference signal to calculate the pathloss should be applied for determining Tx power each of the multiple PRACH transmissions in one attempt. |
| Apple | Support Proposal 7-1-a. |

6.1.5 Retransmission of multiple PRACH transmissions

Issue #8: Retransmission of multiple PRACH transmissions

Discussion about retransmissions of multiple PRACH transmissions

Q1: If multiple PRACH transmissions are performed in the first RACH attempt, in RACH Re-attempts, whether multiple PRACH transmissions are performed? If yes, how to decide the number of multiple PRACH transmissions in RACH Re-attempts, e.g., the same number as the first RACH attempt; or the number can be increased based on some conditions.

The number of PRACH repetition could be increased in the re-attempts.

Support (5): Lenovo, Mavenir, CMCC, vivo, New H3C

Support if some condition is met (11): LG, Spreadtrum, Sharp, Fujitsu, Sony, Nokia, NSB, OPPO, Intel, Huawei, HiSilicon

Not support (3): ZTE, ETRI, MediaTek

[CATT, Fujitsu] power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met.

Q2: Whether power ramping between RACH attempts for multiple PRACH transmissions is supported?

Support (18): CATT, LG, Lenovo, Mavenir, ZTE, Spreadtrum, ETRI, Sharp, Ericsson, Fujitsu, Sony, vivo, MediaTek, OPPO, Intel, New H3C, Huawei, HiSilicon

Not support (4): Panasonic, CMCC, Nokia, NSB

Q3: Whether separate parameters including {power ramping step, maximum number of transmissions, etc.} are needed for multiple PRACH transmissions？

Support (6): Lenovo, Mavenir, Spreadtrum, vivo, Huawei, HiSilicon

Not support (2): CMCC, MediaTek

[LG] If multiple PRACH are transmitted with separate preamble on shared ROs, separate parameters may be provided for multiple PRACH transmissions. Otherwise, legacy parameters for a single PRACH transmission can be reused.

[ZTE, Sharp]: The benefit of separate parameters such as power ramping step, maximum number of transmissions, etc. should be investigated first.

[Ericsson] could FL please clarify whether the intention is for retransmission of multiple PRACH transmissions to use a larger power ramping step and a larger maximum number of transmissions than the legacy?

FL comment: @ Ericsson, maybe a large power ramping step and a smaller maximum number of transmissions is needed based on companies’ contributions as summarized in Section 2.1.5.

Based on companies’ comments in the 1st round, one argument may be if UE perform multiple PRACH transmissions in the first RACH attempt, whether the maximum transmission power is applied. Following the discussion in issue #6, it makes no sense that we go back to discuss the trigger condition. Thus, From FL understanding, UE cannot alter single PRACH transmission to multiple PRACH transmissions.

Proposal 8-1

Support power ramping between PRACH attempts.

FFS: whether/how to increase the number of multiple PRACH transmissions in Re-attempts when maximum transmission power is reached.

Regarding whether the maximum transmission power is applied in the first RACH attempt for multiple PRACH transmissions, companies can further provide you comments, and considering the above proposal.

| Companies | Comments |
| --- | --- |
| Ericsson | Support the proposal. Regarding the FFS, our rationale is for a UE to transmit multiple PRACH transmissions with its Pcmax for one RACH attempt, and then more PRACH transmissions with Pcmax in a latter RACH attempt. A UE can determine the number of PRACH transmissions based on the power headroom of (PCMAX,f,ci-(PPRACH,target,f,c+PLb,f,c). For example, if {2, 4} are the configured numbers of PRACH transmissions, a UE transmits single PRACH if its power headroom is above 0dB, 2 PRACH transmissions if the value is e.g., between 0 and -3, and 4 PRACH transmissions with a lower value. The same as legacy, power ramping is applied to a RACH re-attempt for a UE to determine PPRACH,target,f,c. With that, the UE can obtain power headroom and the number of PRACH transmissions for the re-attempt. |
| ZTE | Support the main bullet. FFS is not needed. Our RAN2 colleagues have concern on the increasing number of multiple PRACH transmission in re-attempts as it means UE will do the PRACH procedure again from the PRACH resource set selection (The PRACH resource sets of different repetition factor are separate.) It is complicated for UE. |
| LG | Support. |
| DOCOMO | Fine with the proposal. |
| Mavenir | Support the proposal. Regarding the FFS, in our view, the UE can determine to increase the number of multiple PRACH transmissions as long as not exceed the maximum number of multiple transmissions configured by network when max tx power is reached. |
| Panasonic | As we commented in the 1st round, we think that a multi-PRACH transmission is to be used after UE’s transmit power reaches a maximum transmit power. In this case, power ramping is ONLY applied for single PRACH transmission case, and not applied during the multi-PRACH transmission and between PRACH attempts. |
| Sharp | In our understanding, power ramping should be applied, at least for the case the number of multiple PRACH transmissions does not change in Re-attempt. On the other hand, the power ramping (i.e. increase of the ramping counter)) may not be needed for the case with increase of the number of multiple PRACH transmissions in Re-attempt, if supported. |
| Nokia/NSB | Agree with Panasonic |
| Spreadtrum | Support this proposal |
| Fujitsu | Support the proposal and share same view with Mavenir. |
| CMCC2 | We share a similar view as Panasonic and Nokia. Our understanding is, UE can use additional physical resources when it reaches its maximum transmit power. Whether UE can reach the maximum transmit power depends on 1) the target power configured by gNB, 2)pathloss. Through implementation, gNB could configured a higher target power for the UE. On the other side, if UE cannot use maximum power for the 1st transmission of PRACH, and it still failed for the initial access, then the UE should do the power ramping up. And it finally can reach the max transmit power. Though it maybe a long procedure, but it also depends on gNB’s config. At last, when UE can request for a multiple PRACH transmission, it should already reach the maximum transmit power. And there will be no room to increase the power between the multiple attempts of multiple PRACH transmissions. With a proper configuration, the UE who request a multiple PRACH transmission based on the measurement of a RSRP, should reach the maximum transmit power for the first attempt of transmission. One more issue is that, the current proposal 8-1 covers both legacy behaviour of single PRACH transmission and also the Rel-18 multiple PRACH transmission. The proposal 8-1 needs updates. |
| Sony | Share similar views with Panasonic. UE should only perform repetitions, which occupies extra resources and prolonging the transmissions, if it has reached max power. Otherwise, it doesn’t make sense why UE wouldn’t just use higher power and complete the transmission in a single PRACH transmission. Hence we think that in a re-transmission, the UE increasers the number of reptitions, i.e. use the next higher number of repetitions. |
| IDCC | Support. |
| Intel | We are fine with the proposal. We also support repetition level ramping between RACH attempts. |
| Qualcomm | Support |
| Samsung | General support, but the FFS point needs some wording refinement. - Not always power ramping is applied, due to beam change, ssb change, or even new case that the number of multiple PRACH transmission increases. - “increase” should only allow among the values configured by gNB, e.g., if gNB only configures 2,8, the increase means 2 ->8, not 2->4->8; - Reach max power is not only condition, the max number for each configured number is possibly also one factor. Suggesting change in green. Proposal 8-1 Support power ramping can be supported between PRACH attempts. FFS: details including whether/how to increase the number of multiple PRACH transmissions within configured numbers in Re-attempts when maximum transmission power or maximum transmission/attempt time is reached. |
| Apple | Ok with this proposal. |
| Huawei, HiSilicon | To improve PRACH resource efficiency and reduce latency, we think the enhanced UEs (select multiple PRACH transmissions) are expected to get access to network as quickly as possible, especially for UEs selecting the largest repetition level. To achieve this goal, when a power control formula similar to that of eMTC PRACH coverage enhancement is used, setting a larger power ramping step than legacy can increase power and improve RACH success probability quickly. When transmit power comes up to maximum in retransmission, it is reasonable for UEs to increase the selected repetition level to the next larger repetition level to further improve RACH success probability. Moreover, we should handle the harmful side-effects of multiple PRACH transmissions. For example, UEs in the extremely bad channel, especially UEs in cell-edge, would persistently transmit PRACH with the largest power and the largest repetition number until the maximum number of retransmissions, which degrades collision performance and causes long-term large intercell interference. Such harmful lasting transmission should be avoided. Limiting the maximum number of retransmissions of enhanced UE selecting the largest repetition level is helpful to solve this problem. |

6.2 Multiple PRACH transmissions with different beams (Postponed)

Proposal 9-1

For multiple PRACH transmission with different Tx beams, down-select one of the following options:

Option 1: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is not supported.

Support (7): Panasonic, DOCOMO, Mavenir, TCL, Spreadtrum, ETRI, vivo (PRACH TX beam would not be specified in Rel-18)

Option 2: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is supported.

- FFS: whether/how to indicate best UL beam based on multiple PRACH transmissions for the subsequent UL transmissions.

Support (6): Lenovo, ZTE, Ericsson, Qualcomm, Nokia, NSB

Option 3: Multiple PRACH transmission with different Tx beams is not supported in Rel-18.

- Note: If multiple PRACH transmission with different Tx beams is supported, the mechanism defined for multiple PRACH transmissions with the same beam should be reused as much as possible.

Support (11): CATT, LG, Apple, CMCC, IDCC, Sony, vivo, MediaTek, OPPO, Intel, New H3C

FL comment: It seems the proposal is fine, but it can be seen that companies’ views are quite divergent. This may need to be discussed in online session.

6.4 CBRA and CFRA (Postponed)

Proposal 10-1

Support multiple PRACH transmissions for both CBRA and CFRA.

Support (27): CATT, Panasonic, LG, Apple, DOCOMO, Lenovo, Mavenir, ZTE, TCL, IDCC, Spreadtrum, ETRI, Sharp, CMCC, Ericsson, Fujitsu, Quectel, IDCC, Sony, Qualcomm, vivo, MediaTek, Nokia, NSB, OPPO, Intel, New H3C

Not support (2): Huawei, HiSilicon

FL comment: This may be decided in online session.

- Proposals for GTW

Draft LS R1-2304070.

Proposal 3-1

Proposed conclusion

There is no consensus to support utilizing different preambles during the multiple PRACH transmissions with the same Tx beam in one attempt.

Proposal 2-2v2

- Multiple PRACH transmissions within one RACH attempt are only performed within one RO group.

  - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions.

    - Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value.

    - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions.

    - Note 3: Valid RO(s) refers to what is defined in existing specification.

Proposal 2-X-1-1

RO group is [determined/configured] within a time period X an RO group pattern period, starting from frame 0, and the [determined/configured] RO group pattern repeats in every time period X RO group pattern period.

  - FFS: whether the time period X RO group pattern period is replaced by consists of K SSB-to-RO association period or K SSB-to-RO association pattern period or other value.

    - K is a positive integer.

    - FFS: the values of K. E.g., K is determined as a minimum positive integer so that at least one RO group corresponding to the largest number of multiple PRACH transmissions configured by the network can be determined within an RO group pattern period.

    - Note: SSB-to-RO association pattern period and SSB-to-RO association period refers to what is defined in existing spec. i.e., Section 8.1 in TS 38.213.

  - Note: whether to define “RO group pattern period” in specification will be discussed separately

Proposal 2-4v3

If one or more PRACH transmission(s) of the multiple PRACH transmission are dropped based on collision handling rules, the dropped PRACH transmission is not postponed.

  - Existing The collision handling rules refer to the rules causing to drop PRACH transmission in Rel-17 existing spec. is reused.

  - FFS: whether to introduce new collision handling rules.

  - FFS: whether there is the impact of the dropped RACH transmission to the remaining RACH transmission within the same RO group.

Proposal 5-1v1

The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions.

Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213.

- Agreements at RAN1#112b-e

Agreement

Confirm the following working assumptions.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Agreement

Send LS to inform RAN2 about the 2 confirmed Working Assumptions, and details on how to realize PRACH resource partitioning is up to RAN2.

Conclusion

There is no consensus to support multiple PRACH transmissions within one RACH attempt located at same time instance in Rel-18.

Note: multiple PRACH transmissions within one RACH attempt located at same time instance includes multiple PRACH transmissions in FDMed ROs located at the same time instance and multiple PRACH transmissions with different preambles in the same RO.

- Agreements at RAN1#112

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Conclusion

For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS.

Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

Agreement

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

Agreement

For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt.

- FFS: the start position of the RAR window.

- FFS: RA-RNTI.

Agreement

For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions.

- Note 1: All ROs in one RO group is associated with the same SSB(s).

- Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- Note 3: whether/how to define “RO group” in specification will be discussed separately

- Note 4: Valid RO(s) refers to what is defined in existing specification

- FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH.

- FFS: the time span of RO group.

- FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions.

- FFS: other details

Agreement

Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

Note: It is summarized by FL that for the same number of PRACH transmissions per source,

- 1 source [Ericsson] shows that: Multiple PRACH transmitted by beam sweeping, where a UE has no prior knowledge of channel and sweeps Tx beams across 360 degrees horizontally and 180 degrees vertically, outperforms multiple PRACH transmissions with the same Tx wide beam (omni direction) by at least 1 dB, provided gNB configures only one SSB and receives PRACH with a wide beam.

- 3 sources [ZTE, Nokia, vivo] show that: A gain from about 1~3 dB of beam sweeping is observed if a UE is able to direct at least one of its Tx beams in the right direction or to narrow down the azimuth and/or zenith range of 360 degrees and/or 180 degrees for beam sweeping compared with multiple PRACH transmissions with the same Tx wide beam.

- 1 source [Huawei] shows that: compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9~5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam.

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams far apart is 3 dB worse than PRACH repetition with single best beam

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam is 1dB worse than PRACH repetition with single best beam.

- 1 source [vivo] shows that: PRACH repetition via random beam directions performs 1 dB worse than PRACH repetition with omni beam.

- Agreements at RAN1#111

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

Study at least the following case for multiple PRACH transmissions with different Tx beams.

- UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

- FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

- Note: not related to decision on CFRA

Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

Working Assumption

Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

  - Focus on FR2.

    - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

  - Performance metric: 0.1% false alarm, 1% miss-detection

  - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

- Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

- Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

- Agreements at RAN1#110b-e

Agreement

- For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt.

  - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Agreement

- For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions.

- FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Agreement

For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options.

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design.

  - FFS: RA-RNTI.

- Option 2: Only one RAR window for all of the multiple PRACH transmissions.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

- Reference

- 3GPP RP-213579, “New WI: Further NR coverage enhancements”, China Telecom, RAN#94e, December 6-17, 2021.

- 3GPP RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom, RAN #96, Budapest, Hungary, June 6-9, 2022.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #112, Athens, Greece, February 27th – March 3rd, 2023.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #110b-e, October 10th – 19th, 2022.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #111, Toulouse, France, November 14th – 18th, 2022.

- 3GPP R1-2212568, “FL Summary#3 on PRACH coverage enhancements”, Toulouse, France, November 14th – 18th, 2022.

- R1-2302350	Discussion on PRACH coverage enhancements	Huawei, HiSilicon

- R1-2302509	Discussions on remaining issues of PRACH coverage enhancements	vivo

- R1-2302573	PRACH coverage enhancements	OPPO

- R1-2302623	Discussion on PRACH coverage enhancements	Spreadtrum Communications

- R1-2302690	PRACH coverage enhancements	CATT

- R1-2302759	Discussion on PRACH coverage enhancements	ZTE

- R1-2302818	Discussions on PRACH coverage enhancement	Intel Corporation

- R1-2302835	PRACH coverage enhancements	TCL Communication Ltd.

- R1-2302863	PRACH Coverage Enhancement using Multiple PRACH Transmissions	Sony

- R1-2302880	PRACH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2302885	Discussion on PRACH coverage enhancements	Panasonic

- R1-2302915	Discussion on PRACH coverage enhancements	Fujitsu

- R1-2302970	Discussion on PRACH coverage enhancements	xiaomi

- R1-2303034	Discussion on PRACH coverage enhancement	China Telecom

- R1-2303086	Discussion on solutions for NR PRACH coverage enhancement	Mavenir

- R1-2303090	PRACH coverage enhancements	Lenovo

- R1-2303153	PRACH coverage enhancements	Samsung

- R1-2303206	PRACH coverage enhancements	ETRI

- R1-2303209	Discussion on PRACH coverage enhancements	Quectel

- R1-2303256	Discussion on PRACH coverage enhancements	CMCC

- R1-2303353	On PRACH coverage enhancements	MediaTek Inc.

- R1-2303411	Discussion on CFRA for Multiple PRACH Transmission	FGI

- R1-2303453	Discussion on PRACH coverage enhancements 	InterDigital, Inc.

- R1-2303508	Discussion on PRACH coverage enhancement	Apple

- R1-2303615	PRACH Coverage Enhancements	Qualcomm Incorporated

- R1-2303640	Views on multiple PRACH transmission for coverage enhancement	Sharp

- R1-2303661	Discussion on PRACH coverage enhancement	Ericsson

- R1-2303681	Discussion on PRACH coverage enhancement	NEC

- R1-2303731	Discussion on PRACH coverage enhancements	NTT DOCOMO, INC.

- R1-2303750	Discussion on PRACH repeated transmission for NR coverage enhancement	LG Electronics