3GPP TSG RAN WG1 meeting #112bis		                                R1-2303985

E-meeting, Apr. 17th – Apr. 26th, 2023

Title: 	Discussion on NES techniques in spatial and power domains

Source: 	ZTE, Sanechips

Agenda item:	9.7.1

Document for:	Discussion and decision

- Introduction

In RAN1#112 meeting, some agreements about network energy saving in spatial and power domain were achieved as below [1]:

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account. Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included Agreement For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline Agreement Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

In this paper, the enhanced CSI measurement/ reporting and dynamic indication for spatial and power domain adaptation are discussed.

- Consideration of the necessity of multiple CSI reports

If the number of antennas for CSI-RS is changed dynamically, the CSI reported before the adaptation will be out-of-state for the data scheduling after adaptation. Therefore, it will be beneficial if UE report CSIs for different numbers of CSI-RS ports in advance so that gNB has more information about channel state of different number of ports, it has more choices to adapt the antenna patterns to fit into traffic arrival and channel condition without sacrificing network energy saving gain (ESG) and system performance.

In this section, the evaluation results of dynamic antenna adaptation with multi-CSI report and single CSI report are provided. FTP3 with 20KByte packet size is used. The simulation assumptions are provided in Appendix A.1.

In dynamic antenna adaptation with single CSI report, the gNB can dynamically change the TxRUs among {64TxRUs, 32TxRUs} based on the reported CSI and traffic arrival, where UE reports CSI for 64 ports or 32 ports CSI based on gNB antenna adaptation.

In dynamic antenna adaptation with multi-CSI report, the CSI of {64ports, 32ports, 16ports, 8ports} are reported, the gNB can dynamically change the TxRUs among {64TxRUs, 32TxRUs, 16TxRUs, 8TxRUs} for PDSCH transmission based on the reported multi-CSI.

Figure 1 Simulation results for spatial domain

According to simulation results in Figure 1, multi-CSI report provides additional 17.36% ESG compared with single CSI report. And multi-CSI report has less UPT loss than single CSI report.

Multi-CSI report provides additional 17.36% ESG and less UPT loss compared with single CSI report.

Multi-CSI report should be considered for network spatial adaptation for energy saving.

- Consideration of CSI-RS resource for different number of ports

In RAN1#112 meeting, the agreed CSI-RS resource configuration with different numbers of ports has two alternatives.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns”

For A1-1, multiple CSI-RS resource/resource set/resource setting should be configured separately and gNB indicates which CSI-RS resource/resource set/resource setting(s) is used for CSI measurement and reporting. The overhead of RRC signaling and CSI-RS transmission is increased for multiple CSI reports.

For A1-2, since the measurement resources for different port numbers are nested, only one CSI-RS resource/resource set/resource setting needs to be configured, the overhead of both RRC signaling and CSI-RS transmission is small. In addition, it provides more opportunities of network energy saving with less DL transmission. And as common resource is used for the CSI measurement of different CSI-RS port numbers, the measurement results will be highly correlated among the CSIs. The correlation can be used to reduce the overhead of CSI reports. Hence, A1-2 is preferred.

For A1-2 with nested structure for CSI-measurement, it has the following benefits:

- Less overhead of both RRC signaling and CSI-RS transmission

- More network energy saving with less DL transmission

- High correlation among multiple CSIs

For spatial element adaptation with multi-CSI report, each CSI-RS resource/resource set/resource setting can be associated with more than one spatial adaptation patterns

For the definition of “spatial adaptation patterns”, the factors which will be affected by spatial element adaptation should be considered. It is clear that the information related to the number of CSI-RS ports, such as port indexes for different port numbers, should be comprised in “spatial adaptation patterns”. Moreover, codebook configuration information, e.g., different codebook subset restrictions for different port numbers, also needs to be included.

The following can be considered to define “spatial adaptation pattern”

  - Codebook configuration,

  - Port indication for CSI with different number of ports.

It is noted that the CSI-RS resource/resource set/resource setting is configured by RRC signaling, if multiple spatial adaptation patterns are configured and the corresponding CSI report is activated, UE always needs to report multiple CSI reports regardless of channel quality and data traffic. In some cases, the required CSI reports can be different in different time period, and semi static configuration may not satisfy the requirement in time.

To better adapt channel states and reduce reporting, dynamic indication of which CSIs need to be reported can be considered. Since each CSI is associated with a spatial adaptation pattern, the indication can be spatial adaptation pattern information. The indication from gNB can be L1 signaling (e.g., a group common DCI). Moreover, since multi-CSI provides more NES gain, the indicated spatial adaptation pattern information should convey information of one or multiple spatial adaptation patterns to support multi-CSI when needed.

The required CSI reports can be different in different time periods, and semi static configuration may not satisfy the requirements in time.

To better adapt to CSI reporting requirements, dynamic indication of one or more CSIs can be considered, where the dynamic indication should convey information for one or multiple spatial adaptation patterns to support multi-CSIs.

- Consideration of UL overhead reduction

If UE needs to report multiple CSIs for NES, the UL overhead will increase accordingly. Therefore, the method to reduce the UL overhead of CSI reports should be studied.

  - PMI

The payload of PMI is shown in Table 1. It can be seen that the overhead of PMI is determined by N1, N2, O1, O2 and the number of subbands. For example, when UE needs to report CSIs of 32 ports, 24 ports and 16 ports with rank = 3, where (N1, N2) = (8,2) for 32 ports, (N1, N2) = (6,2) for 24 ports, (N1,N2)=(8,1) for 16 ports, codebookMode = 1, no subband PMI. The total payload of PMI is 10 + 10 + 8 = 28 bits. If subband PMI also needs to be reported, the overhead of PMIs can be even larger.

Table 1 PMI of codebookType=typeI-SinglePanel

|  | Information field for wideband PMI |  |  | Information field for wideband PMI or per subband PMI |  |
| --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |
|  | codebookMode=1 | codebookMode=2 |  | codebookMode=1 | codebookMode=2 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (,) | N/A | 2 | 4 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (, 0) | N/A | 2 | 4 |
| Rank=2 with 4 CSI-RS ports, | (,) | (, 0) | 1 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (,) | 2 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (, 0) | 2 | 1 | 3 |
| Rank=3 or 4, with 4 CSI-RS ports | (,) |  | 0 | 1 |  |
| Rank=3 or 4, with 8 or 12 CSI-RS ports | (,) |  | 2 | 1 |  |
| Rank=3 or 4, with >=16 CSI-RS ports | (, ) |  | 2 | 1 |  |
| ... |  |  |  |  |  |

The overhead of PMI is determined by N1, N2, O1, O2 and the number of subbands. If multiple PMIs need to be reported for multi-CSI, the overhead is large.

Hence, the compression of PMI payload is beneficial to UCI overhead reduction. In the design of precoding matrix, each row corresponds to a CSI-RS port. If a subset ports are used for CSI measurement with the help of resource configuration of A1-2, a promising solution to PMI overhead reduction is to derive the PMI of smaller number of CSI-RS ports via a sub-matrix of the precoding matrix corresponding a larger number of ports.

    - Correlation between precoding matrices of different port numbers

In this section, the correlation coefficient between precoding matrices of different of port numbers are evaluated.

The correlation coefficient shows the correlation between precoding matrices of different of port numbers, and is computed by , wherein C is the covariance matrix of Pa and Pb. Pa is the precoding matrix of a smaller number of ports derived by a precoding matrix of larger number of ports (e.g., deriving precoding matrix of 16 ports from precoding matrix of 32 ports), and Pb is the real precoding matrix of a smaller number of ports (e.g, 16 ports).

In the evaluation, it assumed that the gNB antenna configuration is adapted from 32 ports to 16 or 24 ports, where the muting pattern is shown in Figure 2, Figure 3, and Figure 4. For gNB antenna configuration adapts from 32 ports to 16 ports, both  and  antenna arrays are evaluated.

Figure 2 Antenna muting pattern 1: adaptation from 32 ports to 16 ports ()

Figure 3 Antenna muting pattern 2: adaptation from 32 ports to 24 ports

Figure 4 Antenna muting pattern 3: adaptation from 32 ports to 16 ports ()

The evaluated correlation coefficient between matrix Pa and matrix Pb for gNB antenna configuration adaption from 32 ports to 16 or 24 ports are shown in Figure 5, Figure 6, and Figure 7, respectively. The rank is fixed in the simulation in this section.

| (a) Rank = 1 | (b) Rank = 2 |
| --- | --- |
| (c) Rank = 3 | (d) Rank = 4 |

Figure 5 Correlation coefficient between 32 port-precoding matrices and 16 port-precoding matrices with different Ranks ()

| (a) Rank = 1 | (b) Rank = 2 |
| --- | --- |
| (c) Rank = 3 | (d) Rank = 4 |

Figure 6 Correlation coefficient between 32 port-precoding matrices and 24 port-precoding matrices with different Ranks

| (a) Rank = 1 | (b) Rank = 2 |
| --- | --- |
| (c) Rank = 3 | (d) Rank = 4 |

Figure 7 Correlation coefficient between 32 port-precoding matrices and 16 port-precoding matrices with different Ranks()

According to the simulation results in Figure 5 , Figure 6, and Figure 7, it can be seen for precoding matrices with 32 ports () and 16 ports, the probability of the correlation coefficient that is greater than 0.6 is about 90% for any rank of 1,2,3,4. For precoding matrices with 32 ports () and 16 ports, the probability of the correlation coefficient that is greater than 0.6 is about 90% for any rank of 1,3,4, and the probability of the correlation coefficient that is greater than 0.6 is about 80% for rank of 2. For precoding matrices with 32 ports and 24 ports, the probability of the correlation coefficient that is greater than 0.6 is about 80% for the rank of 1 or 2, the probability of the correlation coefficient that is greater than 0.6 is about 70% for the rank of 3 or 4. The correlation between these precoding matrices is high.

The correlation between the precoding matrices of different CSI-RS port numbers is high.

    - System performance using the correlation between the two precoding matrices

Since the precoding matrices of different CSI-RS port numbers are highly correlated, the signaling overhead of PMI can be saved by using one PMI, i.e., not transmitting the PMI with smaller number of ports. The throughput performance of exploiting this correlation are evaluation in both LLS with fixed rank and SLS with flexible rank.

- Fixed rank

For evaluation results in Table 2, Pa is a precoding matrix of 16 ports which is derived from a sub-matrix of the precoding matrix of 32 ports, Pb is real precoding matrix reported by PMI of 16 ports CSI-RS. For evaluation results in Table 3, Pa is a precoding matrix of 24 ports which is derived from a sub-matrix of the precoding matrix of 32 ports, Pb is real precoding matrix reported by PMI of 24 ports CSI-RS. The corresponding throughput performance is also provided in Figure A1, Figure A2, and Figure A3 in Appendix.

Table 2. Throughput loss of Pa compared with Pb for 16 ports (LLS, fixed rank)

| SNR(dB) cases | -5 | 0 | 5 | 10 | 15 | 20 | 25 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration: |  |  |  |  |  |  |  |  |
| Rank = 1 | -6.44% | -4.87% | -0.08% | -0.13% | -0.01% | 0.00% | 0.00% | -1.65% |
| Rank = 2 | -3.95% | -2.88% | -2.22% | -0.13% | -0.17% | -0.15% | -0.14% | -1.38% |
| Rank = 3 | 0.41% | -1.84% | -1.59% | 1.88% | 0.52% | -2.76% | -0.20% | -0.51% |
| Rank = 4 | -0.94% | -2.64% | -2.08% | -4.03% | -3.32% | -2.33% | -3.05% | -2.63% |
| Configuration: |  |  |  |  |  |  |  |  |
| Rank = 1 | 0.75% | 0.87% | -0.92% | -0.73% | -0.62% | -0.05% | 0.00% | -0.10% |
| Rank = 2 | -0.70% | 0.15% | -2.20% | -0.23% | -2.69% | -0.33% | -0.27% | -0.90% |
| Rank = 3 | -0.60% | 0.45% | -2.69% | -5.84% | -0.64% | -2.60% | -2.90% | -2.12% |
| Rank = 4 | 3.16% | 0.25% | -0.97% | -1.54% | -4.10% | -2.68% | -2.87% | -1.25% |

Table 3. Throughput loss of Pa compared with Pb for 24 ports (LLS, fixed rank)

| SNR(dB) cases | -5 | 0 | 5 | 10 | 15 | 20 | 25 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank = 1 | -0.82% | -0.85% | -0.82% | 0.04% | 0.00% | 0.00% | 0.00% | -0.35% |
| Rank = 2 | 0.79% | -0.39% | -1.15% | 1.05% | -0.53% | -0.02% | 0.01% | -0.03% |
| Rank = 3 | 2.97% | -0.28% | -2.56% | -1.19% | -1.20% | -3.75% | -2.04% | -1.15% |
| Rank = 4 | -0.92% | -1.49% | 1.12% | -0.81% | -2.56% | -1.77% | -5.96% | -1.77% |

According to the simulation results in the case of fixed rank, it can be seen that the average throughput loss of derived precoding matrices by a precoding matrix with a larger port number is less than 2% relative to that of using real precoding matrix, i.e., the throughput performance of using a derived precoding matrix and real precoding matrix is nearly the same.

The throughput performance of using a derived precoding matrix and using a real precoding matrix is nearly the same if rank is fixed.

- Flexible rank

Considering the high correlation and small throughput performance in LLS, a promising CSI overhead solution is to using one PMI, i.e., not transmitting the PMI with smaller number of ports.

The performance comparison of one PMI and multiple PMIs are evaluated in SLS. In the evaluation of using one PMI to assist gNB dynamic antenna adaptation, the PMIs of other smaller CSI-RS ports are derived by gNB from a sub-matrix of the precoding matrix of largest CSI-RS ports reported by UE. Meanwhile, for the antenna adaptation with multiple PMIs, the PMIs corresponding to all the adaptation patterns are reported.

Moreover, it should be noted that the RI may change when the gNB adapts TxRU patterns for energy saving or data transmission. When RI changes for different number of ports, the number of columns of a precoding matrix will change accordingly. Different from LLS, the RI changes due to spatial adaptation is also considered in simulations of dynamic gNB antenna adaptation among {32ports, 24ports, 16ports}. In this simulation, FTP 3 traffic (packet size = 4KByte, average interval time =10ms) is used.

The distribution of RI change is provided in Figure 8. It can be seen that in most cases (about 80%), the RI will not update due to gNB antenna adaptation, while in 20% cases, the RI will be impacted when gNB antenna number is dynamically adapted from 32 ports to 24 or 16 ports.

Figure 8 RI change distribution

Table 4 Throughput loss of single PMI reporting compared with multi-PMI reporting with flexible RI for dynamic gNB antenna adaptation among {32ports, 24ports, 16ports}

|  | Average UE throughput | 5% UE throughput | 50% UE throughput | 95% UE throughput |
| --- | --- | --- | --- | --- |
| Throughput loss | -2.39% | -4.74% | -2.66% | -1.96% |

The throughput loss of single PMI reporting compared with multi-PMIs reporting with flexible RI is provided in Table 4. According to SLS simulation results, compared with reporting multiple PMI, the average throughput loss of single PMI is small when rank is flexible.

Compared with reporting multiple PMI, the average throughput loss of single PMI is small when rank is flexible.

To this end, it can be concluded that performance of single PMI reporting and multiple PMI reporting is similar and CSI payload can be reduced with single PMI reporting. For dynamic gNB antenna adaptation among {32ports, 24ports, 16ports}, the payloads of single PMI reporting and multiple PMI reporting are shown in Table 5, where it is assumed that the bandwidth part includes 275 PRBs, subband size is 16 PRBs, the number of subband PMIs is 18, and rank=3. It can be observed that payload size of PMI reporting (including wideband PMI and sub-band PMI) can be reduced from 82 bits to 28 bits by single PMI, the payload size of PMI reporting can be reduced by 65%.

Therefore, one PMI reporting with the largest number of ports for multiple CSIs report can should be considered to reduce the UCI overhead.

Table 5 Payload of PMI reporting

| Scheme | Payload of wideband PMI reporting | Payload of subband PMI reporting | Total payload |
| --- | --- | --- | --- |
| Report one PMI | 10(bits) | 18(bits) | 28(bits) |
| Report multi-PMI | 10+10+8=28(bits) | 18*3=54(bits) | 82(bits) |

The performance of single PMI reporting and multiple PMI reporting is similar in both LLS and SLS.

When dynamic gNB antenna adaptation among {32ports, 24ports, 16ports}, the payload of PMI reporting can by reduced by 65% (i.e., from 82bits to 28bits) using single PMI reporting.

Reporting only one PMI with the largest number of ports for multiple CSIs report should be considered to reduce the UCI overhead.

    - Determination of a precoding matrix with smaller port numbers with single PMI reporting

When only one PMI is reported, it should be clarified how to use a precoding matrix of a larger port numbers to derive a precoding matrix of a smaller port numbers.

The rows of the precoding matrix of a smaller port numbers depends on the subsets of ports (or spatial adaptation patterns) selected by gNB. The rows are selected according to the port/spatial adaptation pattern indication by gNB.

For example, gNB indicates port information for different antenna adaptation, UE measures the CSI for the largest CSI-RS port number, while each row of the precoding matrix with smaller of port numbers are extracted from the indicated row (i.e., port information) of the PMI corresponding to the largest CSI-RS port number.

A bitmap can be used to indicate which ports are used for CSI calculation. For example, 32 bits are used for a maximum number of 32 CSI-RS ports, and each bit corresponds to one port. However, the overhead of the signaling should be considered if the bitmap is transmitted via DCI, especially if gNB needs to indicate for more than one number of ports. One way to reduce the overhead is that gNB configures multiple bitmap candidates by RRC signaling, and uses L1 signaling to select one or more from the multiple candidates.

If gNB transmits a signaling to indicate which ports are used for CSI calculation, the overhead of the signaling should be considered.

gNB can configure multiple bitmap candidates by RRC signaling, and use L1 signaling to select one or more from the multiple candidates.

One thing should be noted is that the two ports which are in same position but different polarization directions should be selected simultaneously. Otherwise, the system performance will be severely affected. To ensure that the two ports with different polarization directions are selected at the same time, each bit in the bitmap can be used to indicate dual polarization directions. For example, if 16 ports need to be selected via bitmap from 32 ports. A bitmap ‘1111 0000 0000 0000’ means ports with indexes of ‘{0,16}, {1,17}, {2,18}, {3,19}’ are selected. Using this method, the overhead of the bitmap configuration can be further reduced.

To reduce signaling overhead and guarantee performance, each bit in a bitmap corresponds to two ports with different polarization directions but in same position.

Another thing is that the port selection (i.e., via bitmap) should not violate the legacy configuration of (N1, N2).

Table 6: Supported configurations of and

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |
| 8 | (2,2) | (4,4) |
|  | (4,1) | (4,1) |
| 12 | (3,2) | (4,4) |
|  | (6,1) | (4,1) |
| 16 | (4,2) | (4,4) |
|  | (8,1) | (4,1) |
| 24 | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,1) |
| 32 | (4,4) | (4,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,1) |

The supported configurations of (N1, N2) are shown in Table 6. We should make sure that the selected ports satisfy one of the supported (N1, N2).

Moreover, the codebook design in TS 38.214 also should be considered. According to the structure of the precoding matrix, there are two types of PMI, one (which is composed of four parts) is used when number of layers is three or four, and number of ports (PCSI-RS) is not less than 16, and one (which is composed of two parts) is used for others. The derived precoding matrix with smaller number of CSI-RS ports should also follow these structures. Otherwise, the correlation between the derived precoding matrix and the real precoding matrix will poor, and the system performance will be seriously impacted.

For example, if the codebookType is set to ‘typeI-SinglePanel’ the number of layers is three or four, and number of ports (PCSI-RS) is not less than 16, the precoding matrix is: , where L is the number of layer, the number of rows in precoding matrix is same as PCSI-RS, the number of columns in precoding matrix is equal to the number of layers. The precoding matrix includes four parts, where the first two parts correspond to one polarization direction and the last two parts correspond to another polarization direction.

We further provide some simulation results of correlation between two precoding matrices, wherein one precoding matrix (e.g., 16 ports) is derived by a precoding matrix with larger number of ports (e.g., 32 ports) and another precoding matrix is the real precoding matrix with smaller number of ports (e.g, 16 ports). The rank is fixed in the simulation for the sake of simplification.

Different from the muting pattern in Figure 2 in section 4.1.1, the muting pattern of gNB antenna configuration adaptation from 32 ports to 16 ports in this section is shown in Figure 9.

Figure 9 Antenna muting pattern 3: adaptation from 32 ports to 16 ports

In this evaluation, the derived precoding matrix does not follow the structures used when number of layers is three or four, and number of ports (PCSI-RS) is not less than 16.

| (a) Rank = 3 | (b) Rank = 4 |
| --- | --- |

Figure 10 Correlation coefficient between 32 port-precoding matrices and 16 port-precoding matrices with different Ranks

According to the simulation results, the probability of the correlation coefficient of 16-port precoding matrices and 32-port precoding matrices that is greater than 0.6 is less than 30% for rank of 3,4. The correlation is much lower than the evaluation in Figure 5 due to the improper selection of ports.

The correlation between 16-port precoding matrix and 32-port precoding matrix with rank 3 or 4 is low with improper selection of ports.

The reason about the low correlation between two precoding matrices when rank = 3 or 4 is the derived precoding matrix does not follow the structure of the precoding matrix defined in TS 38.214. For example, as shown in Figure 11, the derived precoding matrix with 16 ports based on the port selection in the evaluation only comprises of part #1 and part #3 of the precoding matrix with 32 ports. Hence, the correlation is low.

Figure 11 One example of derived PMI based on the port selection in section 4.1.3

However, for the evaluation in Figure 4, the derived precoding matrix with 16 ports based on the port selection comprises a subset of each part of the precoding matrix with 32 ports, which follows the structure of precoding matrix in TS 38.214. Therefore, correlation and performance are better.

Figure 12 Another example of derived PMI based on the port selection in section 4.1.1

To summarize, if only one PMI is reported for multiple number of ports, the port selection should observe the following principles:

- The selected ports should follow a valid (N1, N2);

- The extracted precoding matrix should follow the structure of the precoding matrix in TS 38.214.

If only one PMI is reported for multiple number of ports, the port selection should observe the following principles:

- The selected ports should follow a valid (N1, N2);

- The extracted precoding matrix should follow the structure of the precoding matrix in TS 38.214.

  - RI

The results in Figure 8 show how RI value changes when the gNB antenna are dynamically adapted. According to the simulation results, in most cases, RI doesn’t change or just decreased by 1 when CSI-RS antenna ports are dynamically adapted.

In most cases, RI doesn’t change or just decreased by 1 when CSI-RS antenna ports are dynamically adapted.

Based on the observation, the overhead of RI can be reduced for multi-CSI reporting via differential RI. For example, RI with smaller number of ports can be reported using a differential RI value which is a differential between a RI value of larger port numbers and the RI value with smaller port numbers. The candidates of differential RI value can be small according to evaluation results in Figure 8.

Differential RI can be considered to reduce UCI overhead.

  - CQI

Similar as RI overhead reduction, the CQI with different number of ports will not change a lot when the CSI-RS resources with different port numbers are nested, thus a differential CQI can be considered for multi-CSI report which is a differential between a CQI value with larger number of ports and the CQI value with smaller number of ports.

Differential CQI can be considered to reduce UCI overhead.

- Discussion on the impact of spatial domain adaptation on beam management

For the CSI-RS resources configured for L1-RSRP and L1-SINR computation, the UE can only be configured with the same number (1 or 2) of ports for all CSI-RS resources within the CSI-RS resource set. In 3GPP RAN1#112 meeting, two types mapping methods between antenna elements and logical antenna port were discussed as follows.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

With type 1 mapping method, even the dynamic antenna port adaptation is used for network energy saving, the antenna port configuration of CSI-RS resources used for L1-RSRP and L1-SINR computation does not need to be impacted.

The spatial element adaptation with type 1 mapping method does not impact the antenna port configuration of CSI-RS resources for beam management.

For type 2 mapping method, the CSI-RS resources for beam management may be affected by the dynamic spatial element adaptation. However, this impact brought by the spatial element adaptation can be avoided by NW implementation. The gNB does not need to perform spatial element adaptation for the CSI-RS resources configured for beam management, and a small amount of energy saving gain is sacrificed to avoid the impact.

The impact on beam management with type 2 mapping method can be avoided by NW implementation.

In conclusion, the necessity of enhancement on beam management is unclear, and considering the limited TUs and heavy workload, we think the enhancement on beam management should be deprioritized.

The enhancement on beam management should be deprioritized.

- Consideration on CSI related procedure enhancement in power domain

The dynamic power adjustment can adapt to different channel conditions and bring gNB energy saving gain. While the semi-static power offset configuration does not match the dynamic power adjustment for PDSCH, so the enhancement on the power offset between PDSCH and CSI-RS should be considered.

Based on the motivation of the enhancement on spatial domain and power domain, it can be observed that both the enhancement for power domain and spatial domain have a similar purpose to provide more information for the base station, so that the base station can obtain proper and more accurate CSIs when network energy saving techniques are used, thereby ensuring that system performance of the base station is not affected. And both the antenna ports related information and the power offset between PDSCH and CSI-RS are critical for the UE to compute the CSI. Therefore, it’s better to use a same framework for the enhancement on power domain and the enhancement on spatial domain, so that workload can be reduced.

Same framework can be used for the enhancement on power domain and the enhancement on spatial domain.

In 3GPP RAN1#112 meeting, the enhancements on CSI related procedures in power domain was discussed. From the discussion, it can be learned that multiple power offset values between PDSCH and CSI-RS should be configured to match different power adaptation cases.

Obviously, when multiple power offset values are configured, UE can perform CSI measurements and derive CSI feedback using different power offset values. To ensure that the gNB can capture a relatively accurate CSI feedback results when dynamic power adjustment is performed, both multi-CSI report with different power offset values and dynamic indication of the power offset values can be considered.

Both the multi-CSI report with different power offset values and the dynamic indication of the power offset values is beneficial for the gNB to capture a relatively accurate CSI feedback.

For multi-CSI report with multiple CSI measurement results corresponding to different power offset values, the reporting information can be used to guarantee the network obtain the proper/accurate CSI measurement results and assist the network for the dynamic transmission power adjustment. Therefore, an enhanced CSI measurement and CSI report with multi-CSI reporting can be supported.

Enhanced CSI measurement and CSI report with multi-CSI reporting corresponding to different power offset values are beneficial for the network to dynamically adjust transmission power.

Multi-CSI report with multiple power offset values between PDSCH and CSI-RS should be supported.

To reduce the overhead caused by multi-CSI report, one effective method is to set multiple CSIs reported in one report. In this way, duplicated information of the measurement results that is not affected by the power offset values can be reduced, thereby reducing the reporting overhead.

In addition, since the power offset of the CSI-RS and PDSCH is only used to CQI calculation, the PMI and RI of different power offsets are the same. Therefore, differential CQI can be used to further reduce the UCI overhead of multi-CSI in one reporting.

Differential CQI should be can be adopted for multi-CSI in one reporting with multiple power offset values.

For CSI-RS resource configurations, each CSI-RS resource/resource set/resource setting can be associated with one or more power offset values, which can reduce the signaling overhead. For example, multiple power offset values can be configured as a list within one NZP-CSI-RS resource or multiple power offset values can be configured in multiple CSI-RS resources within one CSI-ResourceConfig. UE can perform CSI measurement and reporting with different power offset values using the same CSI-RS resources or different CSI-RS resources.

Each CSI-RS resource/resource set/resource setting can be associated with one or more power offset values.

Besides, the dynamic indication can also be used to compensate for an inaccurate CSI feedback caused by dynamic power adjustment. The indication information can be activation/deactivation of one or more power offset values. UEs can adjust CSI measurement and reporting by the indication information to adapt to the dynamic power adjustment.

Dynamic indication can be considered for power domain enhancement.

- Conclusion

In this contribution, we discuss the network energy saving techniques in spatial and power domain and have the following observations and proposals.

Observation 1: Multi-CSI report provides additional 17.36% ESG and less UPT loss compared with single CSI report.

Observation 2: For A1-2 with nested structure for CSI-measurement, it has the following benefits:

 Less overhead of both RRC signaling and CSI-RS transmission

 More network energy saving with less DL transmission

 High correlation among multiple CSIs

Observation 3: The required CSI reports can be different in different time periods, and semi static configuration may not satisfy the requirements in time.

Observation 4: The overhead of PMI is determined by N1, N2, O1, O2 and the number of subbands. If multiple PMIs need to be reported for multi-CSI, the overhead is large.

Observation 5: The correlation between the precoding matrices of different CSI-RS port numbers is high.

Observation 6: The throughput performance of using a derived precoding matrix and using a real precoding matrix is nearly the same if rank is fixed.

Observation 7: Compared with reporting multiple PMI, the average throughput loss of single PMI is small when rank is flexible.

Observation 8: The performance of single PMI reporting and multiple PMI reporting is similar in both LLS and SLS.

Observation 9: When dynamic gNB antenna adaptation among {32ports, 24ports, 16ports}, the payload of PMI reporting can by reduced by 65% (i.e., from 82bits to 28bits) using single PMI reporting.

Observation 10: If gNB transmits a signaling to indicate which ports are used for CSI calculation, the overhead of the signaling should be considered.

Observation 11: The correlation between 16-port precoding matrix and 32-port precoding matrix with rank 3 or 4 is low with improper selection of ports.

Observation 12: If only one PMI is reported for multiple number of ports, the port selection should observe the following principles:

 The selected ports should follow a valid (N1, N2);

 The extracted precoding matrix should follow the structure of the precoding matrix in TS 38.214.

Observation 13: In most cases, RI doesn’t change or just decreased by 1 when CSI-RS antenna ports are dynamically adapted.

Observation 14: The spatial element adaptation with type 1 mapping method does not impact the antenna port configuration of CSI-RS resources for beam management.

Observation 15: The impact on beam management with type 2 mapping method can be avoided by NW implementation.

Observation 16: Both the multi-CSI report with different power offset values and the dynamic indication of the power offset values is beneficial for the gNB to capture a relatively accurate CSI feedback.

Observation 17: Enhanced CSI measurement and CSI report with multi-CSI reporting corresponding to different power offset values are beneficial for the network to dynamically adjust transmission power.

Observation 18: Each CSI-RS resource/resource set/resource setting can be associated with one or more power offset values.

Proposal 1:	Multi-CSI report should be considered for network spatial adaptation for energy saving.

Proposal 2:	For spatial element adaptation with multi-CSI report, each CSI-RS resource/resource set/resource setting can be associated with more than one spatial adaptation patterns

Proposal 3:	The following can be considered to define “spatial adaptation pattern”

•	Codebook configuration,

•	Port indication for CSI with different number of ports.

Proposal 4:	To better adapt to CSI reporting requirements, dynamic indication of one or more CSIs can be considered, where the dynamic indication should convey information for one or multiple spatial adaptation patterns to support multi-CSIs.

Proposal 5:	Reporting only one PMI with the largest number of ports for multiple CSIs report should be considered to reduce the UCI overhead.

Proposal 6:	gNB can configure multiple bitmap candidates by RRC signaling, and use L1 signaling to select one or more from the multiple candidates.

Proposal 7:	To reduce signaling overhead and guarantee performance, each bit in a bitmap corresponds to two ports with different polarization directions but in same position.

Proposal 8:	Differential RI can be considered to reduce UCI overhead.

Proposal 9:	Differential CQI can be considered to reduce UCI overhead.

Proposal 10:	The enhancement on beam management should be deprioritized.

Proposal 11:	Same framework can be used for the enhancement on power domain and the enhancement on spatial domain.

Proposal 12:	Multi-CSI report with multiple power offset values between PDSCH and CSI-RS should be supported.

Proposal 13:	Differential CQI should be can be adopted for multi-CSI in one reporting with multiple power offset values.

Proposal 14:	Dynamic indication can be considered for power domain enhancement.

- References

Chair’s notes RAN1#112, 2023-03.

- Appendix

## A.1 Simulation assumptions

Table A1 BS antenna configuration for TxRU adaptation

| Simulation case | Description |
| --- | --- |
| 64TxRU | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1); Max transmission power=55dBm |
| 32TxRU | (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1); Max transmission power=52dBm |
| 16TxRU | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1); Max transmission power=49dBm |
| 8TxRU | (M, N, P, Mg, Ng) = (4, 2, 2, 1, 1); Max transmission power=46dBm |

Table A2 Evaluation assumptions for UMa scenario

| Parameters | Value |
| --- | --- |
| Duplex Mode / Simulation bandwidth | TDD/100 MHz |
| Scenario | Urban Macro; Hexagonal grid, 7 macro sites, 3 sectors per site |
| Inter-BS distance | 500m |
| Min. BS - UE distance | 35m |
| Carrier frequency | 4GHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 64Tx antenna ports, and (dH, dV) = (0.5λ, 0.8λ); 64Tx: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1; 4,8) The antenna tilt is 12 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Total DL power level | 55dBm |
| UE transmit power | 23 dBm |
| Antenna Height | 25 m for BS and 1.5 m for UE |
| Antenna Element Gain | 8dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE distribution | Uniform in cell; 80% of users are indoor, 20% of users are outdoor; Use 3km/h for modeling fading channel |
| Number of UEs per cell | Depends on RU requirement |
| Scheduling Algorithm | SU-MIMO + PF |
| HARQ/repetition | HARQ retransmission |
| Channel estimation | Realist |
| PDCCH overhead | 1/7 (2symbols per 14symbol) |
| DMRS overhead | 1 symbol per 14symbol |
| BS receiver | MMSE-IRC |

- A.2 Simulation results

| (a) Rank = 1 | (b)Rank = 2 |
| --- | --- |
| (c)Rank = 3 | (d)Rank = 4 |

Figure A1 Average throughput of using Pa and using Pb (Pa is precoding matrix derived from a precoding matrix of 32 ports and Pb is a real precoding matrix with 16 ports.) W16= Fnorm(W32([0:2:15 16:2:31],:))

| (a)Rank = 1 | (b)Rank = 2 |
| --- | --- |
| (c)Rank = 3 | (d)Rank = 4 |

- Figure A2 Average throughput of using Pa and using Pb (Pa is precoding matrix derived from a precoding matrix of 32 ports and Pb is a real precoding matrix with 24 ports.), W24= Fnorm(W32([2:13 18:29],:))

| (a) Rank = 1 | (b)Rank = 2 |
| --- | --- |
| (c)Rank = 3 | (d)Rank = 4 |

Figure A3 Average throughput of using Pa and using Pb (Pa is precoding matrix derived from a precoding matrix of 32 ports and Pb is a real precoding matrix with 16 ports.) W16= Fnorm(W32([0:4:15 16:4:31],:))