3GPP TSG-RAN WG1 Meeting #112bis-e	  R1-2303988

e-Meeting, April 17 – 26, 2023

Agenda Item:	9.2.2.1

Source:	Moderator (Huawei)

Title:	Summary#1 for [112bis-e-R18-AI/ML-02]

Document for:	Discussion and Decision

# Introduction

In RAN#94-e plenary meeting, a new SID on AI/ML for air-interface was approved for Rel-18 [1], where the use cases include CSI feedback enhancement, and evaluations would be performed for this use case:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This document summarizes the key issues discussed under agenda item 9.2.2.1, and aims to discuss a set of issues for the evaluation of the AI/ML-based CSI feedback enhancements in RAN1#112.

## 1st round email discussions

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table. Please update your contact information on top of the last meeting if needed.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Samsung | Ameha | amehat.abebe@samsung.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| ZTE | Lun Li | li.lun1@zte.com.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| AT&T | Isfar Tariq Salam Akoum | isfar.tariq@att.com salam.akoum@att.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Mavenir | Ali Fatih Demir | ali.demir@mavenir.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| IIT Kanpur | Abhishek Kumar Singh | Abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| vivo | Jianming Wu | jianming.wu@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| Mavenir | Fan Yang | fan.yang@mavenir.com |
| Fujitsu | Xin Wang Qun Zhang | wangxin@fujitsu.com zhangqun@fujitsu.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Apple | Huaning Niu | huaning_niu@apple.com |
| MediaTek | Gyubum Kyung Pedram Kheirkhah Sangdeh | gyubum.kyung@mediatek.com Pedram.kheirkhah@mediatek.com |
| InterDigital | MoonIl Lee | MoonIl.Lee@InterDigital.com |
| Nokia | Tosato, Filippo | filippo.tosato@NOKIA.COM |
| Xiaomi | Min Liu | liumin10@xiaomi.com |
| China Telecom | Bei Yang | yangbei1@chinatelecom.cn |
| Fraunhofer | Ebrahim Amiri | ebrahim.amiri@iis.fraunhofer.de |

# Generic issues on evaluation methodology

## Summary of views from companies

### 2.1-1: Remaining issues of the EVM table

N/A

### 2.1-2: Metrics

#### Intermediate KPIs

- Fraunhofer: If explicit CSI is provided at the gNB, the metrics based on similarity i.e., NMSE or GCS (SGCS) are proposed for performance evaluation and comparison

- Fraunhofer: If explicit CSI is not provided at the gNB, the metrics based on spectral efficiency (differential or relative) are proposed for performance evaluation and comparison

#### Capability/complexity related KPIs

- Processing complexity

  - Ericsson: For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values based on high level operations (HLO) representations (and not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included for a fair cross-company comparison.

  - NVIDIA: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for CSI feedback enhancements

    - Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year.

  - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered

    - Estimated raw channel matrix per each frequency unit as an input for pre-processing

    - Precoding vectors per each  frequency unit as an output of post-processing

    - Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively

  - ETRI: For the evaluation of CSI compression sub use-case, consider limiting the pre-processing operations as operations after the input data is prepared in the input format (e.g., precoding matrix, or explicit channel matrix).

  - ETRI: For the evaluation of CSI compression sub use-case, consider limiting the post-processing operations as operations after the output data is prepared in the output format (e.g., precoding matrix, or explicit channel matrix).

  - AT&T: For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, the boundary for calculating the complexity metric (i.e., FLOPs) is as

    - For the input of the CSI prediction, or input of the CSI generation part, the pre-processing starts at the raw channel matrix (i.e., includes the decomposition from channel matrix to eigenvectors)

    - For the output of the CSI generation part, the post-processing includes the quantization.

    - For the input of the CSI reconstruction part, the pre-processing includes the dequantization.

    - For the output of the CSI prediction or output of CSI reconstruction part, ends at the precoding vectors.

  - LG: It is up to company to report their calculation and assumption of pre/post-processing if applied, and it can be independently reported from that of AI/ML model

### 2.1-3: Dataset related issues

Source of dataset

- OPPO: Companies are encouraged to disclose their utilized dataset(s) and reference model(s)

  - FFS: to establish common dataset(s) and/or reference model(s) for performance calibration and drawing final conclusions.

- NVIDIA Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for CSI feedback enhancement

## 1st round email discussions

### 2.2-1: Metrics

#### Issue#2-1 (Medium priority) Clarification for processing complexity

Moderator note: In the 109-e meeting, we agreed in 9.2.2.1 that FLOPs is adopted as the metric to evaluate the complexity.

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

In 110b-e meeting, we agreed in 9.2.1 that the complexity includes pre/post-processing.

Agreement The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency)

In this meeting, Samsung [19] brings up that the idea that there may be some ambiguity on the boundary for the calculation of the pre-processing (see below).

[Samsung] However, while reporting the complexity of pre- and post-processing, the reference input and output format of the pre-processing and post-processing operations, respectively, could be ambiguous. For example, raw channel matrices or eigenvectors can be considered as input for pre-processing. In order to clarify this point we propose the following: Proposal #1: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered. - Estimated raw channel matrix per each frequency unit as an input for pre-processing - Precoding vectors per each frequency unit as an output of post-processing Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively.

This issue has been initially discussed in the last meeting. There are concerns on reporting the complexity of the pre/post processing, that complexity for pre-processing and post-processing would be very implementation-specific (e.g., depending on the algorithm for SVD decomposition, etc.). A compromised proposal is then proposed in below, to allow companies to separately report the FLOPs for AI/ML model and the pre/post processing. For how to calculate the FLOPs for pre/post processing, as the concern of opponent companies, there is no need to align the calculation method, and it is up to companies.

Proposed conclusion 2.2.1: For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing.

- How to calculate the FLOPs for pre/post processing is up to companies.

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, Spreadtrum, CATT, LG Electronics, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | As we have concerned in last meeting, the pre/post processing is implementation specific, the FLOPs reporting of pre/post processing is not very necessary. We prefer that the reporting of FLOPs of pre/post processing is optional. |
| Fujitsu | We suggest that it is up to companies to report the complexity of the pre/post-processing used. |
| ZTE | We think there is no need to calculate the FLOPs for pre/post processing since it is up to companies’ implementation and it is enough to only evaluate the calculation complexity of AI/ML model since we mainly focus on the AI/ML model performance calibration. |
| MediaTek | There is no clear boundary between pre-processing and AI/ML model. Depending on implementation, some operation can be either done as a AI/ML model’s part or as pre-processing. It is not possible to draw such a boundary unless we agree either on a nominal input to AI/ML models or standard operations of an AI/ML model. Finally, benefit of such a separation is not clear to us as finally the total FLOPs determine the computational burden on UE/gNB |
| Samsung | To be honest, we are not sure how reporting separately would resolve the concern from opponents. AI/ML model itself is implantation and we report the complexity for it. Why is it different for Pre/post processing? Besides, we have already made agreement in RAN1#110 for 9.2.1. to take complexity of pre/post-processing. |

### 2.2-2: Others

Question 2.2.2: Do you think there are additional high priority issues or EVM parameters which are generic to all sub use cases and have not been discussed/captured in previous sub-sections?

| Company | View |
| --- | --- |

## 2nd round email discussions

#### Issue#2-1 (Medium priority) Clarification for processing complexity

Moderator note: No change on the proposal. Some replies are given in the table.

Proposed conclusion 2.3.1: For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing.

- How to calculate the FLOPs for pre/post processing is up to companies.

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, Spreadtrum, CATT, LG Electronics, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, AT&T, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @OPPO @Fujitsu @ZTE @MediaTek We have agreement on 9.2.1 at 110b-e that pre/post processing complexity is part of inference complexity. Agreement The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency). @MediaTek The intention for separate report is to provide comprehensive view: you can sum model complexity and processing complexity during the comparison. @Samsung As said by opponents, the calculation of FLOPs may quite diverse over companies. That is why the separate report is proposed here. |

# Specific evaluation methodology for CSI compression sub use case

## Summary of views from companies

### 3.1-1: Evaluation findings with 1-on-1 joint training

CSI compression

#### Findings of companies

- Findings on spatial-frequency domain CSI compression

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 7%-10.3% gain on rank 1 under full buffer traffic.

    - 8.9%-14.8% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-2.5% gain on rank 1 under FTP traffic and 50% RU.

    - 3.2%-7.8% gain on rank 1 under FTP traffic and 70% RU.

    - 4.8%-8.8% gain on rank 1 under FTP traffic and 80% RU.

    - 5.4%-10.1% gain on rank 2 under FTP traffic and 50% RU.

    - 7.2%-14.4% gain on rank 2 under FTP traffic and 70% RU.

    - 7.5%-14.5% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-7% gain on rank 1 under FTP traffic and 50% RU.

    - 6.4%-14.6% gain on rank 1 under FTP traffic and 70% RU.

    - 7.1%-16.3% gain on rank 1 under FTP traffic and 80% RU.

    - 4.3%-13.3% gain on rank 2 under FTP traffic and 50% RU.

    - 8.1%-18% gain on rank 2 under FTP traffic and 70% RU.

    - 9.1%-23.7% gain on rank 2 under FTP traffic and 80% RU.

  - Qualcomm: AI/ML based CSI compression gives UL overhead gains of around 50% for mean throughputs (at a mean throughput around 14.3 Mbps) and 32% for cell-edge throughputs (around 2.6 Mbps)..

  - Qualcomm: For Bursty Traffic

    - At 10% RU, we see a 63% improvement in UL overhead at a mean user experience of around 150 Mbps. At 55% improvement in UL overhead, the edge user experience is around 80 Mbps.

    - At 40% RU, we observe a 60% improvement in UL overhead at a mean user experience of 88 Mbps. At an edge user experience of around 38 Mbps, the gain in UL overhead is 51%.

    - At 80% RU, we observe a 60% improvement in UL overhead at a mean user experience of around 45 Mbps, and 70% improvement in UL overhead at an edge user experience of around 11 Mbps.

  - Ericsson:	For Type 1 training, the intermediate KPI gains of AI based CSI compression is 0.7-3.9 % for layer 1

  - Ericsson:	For Type 1 training, the SLS gains of AI based CSI compression (limited to rank 1 and 2) is for 50% RU equal to 1-4% on mean UPT and 2-7 % on cell edge

  - Apple: Transformer based AE can achieve better SGCS consistently comparing to type II codebook. Around 40% overhead reduction is observed

  - ZTE: For Rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 6.43% SGCS gains; with the medium feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains; with the high feedback overhead, AI-based CSI recovery can obtain up to 7% SGCS gains

  - ZTE: For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead

  - ZTE: For Rank 1, for 5% tail UPT, AI-based CSI reconstruction can achieve up to 20.43% performance gain with the low feedback overhead, up to 10.13% performance gain with the medium feedback overhead, and up to 0.85% performance gain with the high feedback overhead

  - ZTE: For Rank 2, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 4.67% SGCS gains in Layer 1 and 5.92% SGCS gains in Layer 2; with the medium feedback overhead, AI-based CSI recovery can obtain up to 4.04% SGCS gains in Layer 1 and 5.90% SGCS gains in Layer 2; with the high feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains in Layer 1 and 9.95% SGCS gains in Layer 2.

  - ZTE: For up to Rank 2, under the case of 50% RU, AI based CSI reconstruction has about up to 4.09% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.08% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 4.55% average UPT gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 2, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 20.00% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.37% performance gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 2, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 48.60% overhead reduction with the low feedback overhead,up to 39.80% overhead reduction with the medium feedback overhead, and up to 40.22% overhead reduction with the high feedback overhead.

  - ZTE: For up to Rank 2, under the case of 70% RU, AI based CSI reconstruction has about up to 5.30% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.92% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.94% average UPT gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 2, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 10.26% performance gain over the Rel-16 eType II with the low feedback overhead, up to 15.02% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 16.26% performance gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 2, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 44.44% overhead reduction with the low feedback overhead,up to 38.12% overhead reduction with the medium feedback overhead, and up to 40.41% overhead reduction with the high feedback overhead.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

  - ZTE: For Rank 4, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 3.64% SGCS gains in Layer 1,  3.86% SGCS gains in Layer 2, 20.56% SGCS gains in Layer 3, and 53.21% SGCS gains in Layer 4; with the medium feedback overhead, AI-based CSI recovery can obtain up to 3.56% SGCS gains in Layer 1, 3.55% SGCS gains in Layer 2, 11.24% SGCS gains in Layer 3, and 26.99% SGCS gains in Layer 4; with the high feedback overhead, AI-based CSI recovery can obtain up to 5.80% SGCS gains in Layer 1, 5.41% SGCS gains in Layer 2, 13.53% SGCS gains in Layer 3, and 26.46% SGCS gains in Layer 4.

  - ZTE: For up to Rank 4, under the case of 50% RU, AI based CSI reconstruction has about up to 12.22% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.04% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.19% average UPT gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 11.98% performance gain over the Rel-16 eType II with the low feedback overhead, up to 6.38% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.12% performance gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 60.76% overhead reduction with the low feedback overhead, up to 34.31% overhead reduction with the medium feedback overhead, and up to 25.40% overhead reduction with the high feedback overhead.

  - ZTE: For up to Rank 4, under the case of 70% RU, AI based CSI reconstruction has about up to 14.89% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 6.64% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.88% average UPT gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 20.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 12.50% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.20% performance gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 64.38% overhead reduction with the low feedback overhead,up to 34.20% overhead reduction with the medium feedback overhead, and up to 24.81% overhead reduction with the high feedback overhead.

  - ZTE: For up to Rank 4, under full buffer traffic, AI based CSI reconstruction has about up to 10.67% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.68% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 9.98% average UPT gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under full buffer traffic, for 5% tail UPT, AI based CSI reconstruction has about up to 4.91% performance gain over the Rel-16 eType II with the low feedback overhead, up to 3.53% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.15% performance gain over the Rel-16 eType II with the high feedback overhead.

  - ZTE: For up to Rank 4, under full buffer traffic, AI-based CSI reconstruction can achieve up to 66.67% overhead reduction with the low feedback overhead,up to 45.48% overhead reduction with the medium feedback overhead, and up to 39.40% overhead reduction with the high feedback overhead.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank 4.

  - MediaTek: AI/ML model offers overhead reduction ranging from 17.9% to 42.5%, and 27.4% on average.

  - MediaTek: AI/ML model offers less than 1% gain in terms of spectral efficiency

  - Nokia: The transformer-based CSI compression model outperforms baseline Rel-16 eTypeII codebook performance based on both the SGCS metric and MU-MIMO throughput performance.  Gains up to the following are seen:

    - 6.5%/2.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 1.

    - 8.5%/3.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 2.

    - 4.5%/10.0% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 1.

    - 13.0%/22.5% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 2.

  - Xiaomi: CSI compression shows 9% performance loss compared

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with proposed scalable AI/ML model for rank=1, 2, 3, 4:

    - SGCS can be improved by 0.02~0.22 under the same CSI feedback payload;

    - Payload can be saved by 30%~60% bits under the same SGCS.

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same or similar number of feedback bits.

    - With the same or similar number of feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the square of generalized cosine similarity

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy

    - With similar performance in the square of generalized cosine similarity, AI based approach could reduce 30%~60% feedback bits

  - CAICT: From preliminary results, AI/ML based spatial-frequency domain CSI compression shows good SGCS performance for training type 1 with Max rank 1 and 2

  - ETRI: In the CSI compression sub-use case, with a Transformer based Autoencoder, there are no or slight (2~3%) improvements in terms of SGCS compared to the baseline (R16 eTypeII) for payload sizes X/Y/Z

  - Intel: ML based Autoencoder can outperform Rel-16 eType II codebook for Layer 1 and Layer 2 case in all overhead regimes for InH, Dense Urban Macro and Dense Urban Micro deployments

  - Intel:

    - Up to 13% gain for average UE throughput and up to 17% gain for cell-edge UE throughput can be achieved for AI-ML CSI comparing to eType II PMI codebook

    - Up to ~100 bits overhead reduction can be achieved using the AI-ML CSI with the same performance as for eType II PMI codebook

  - Spreadtrum: The AI based CSI compression and recovery can achieve better SGCS performance and lower feedback bits cost than Rel-16 eType II codebook

  - Fujitsu: Comparing with eType-II codebook, the throughput can be improved by AI/ML based CSI feedback:

    - 2%~9% gain for average UPT and 6.1%~20.9% gain for 5% UPT for rank=1 and full buffer traffic.

    - 7%~13.7% gain for average UPT and 3.5%~14.9% gain for 5% UPT for the maximum rank=2 and full buffer traffic.

  - FUTUREWEI: For CSI feedback compression sub use case, when using scalar quantization AI/ML-based approach outperforms Rel-16 Type II codebook-based approach in all 3 overhead categories, X, Y and Z with ~13.1% average Mean UPT gain and ~33.7% average 5% UPT gain.

  - FUTUREWEI: For CSI feedback compression sub use case, when using vector quantization, AI/ML-based approach with 13 CSI overhead bits achieves better Mean UPT and 5% UPT than Rel-16 Type II codebook-based approach with 10 times more overhead bits, e.g., 130 or 242 CSI bits.

- Findings on spatial-frequency domain CSI compression with additional past CSI (Temporal-spatial-frequency domain)

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 18.3%-25.4% gain on rank 1 under full buffer traffic.

    - 23.3%-30.2% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 7.7%-14.9% gain on rank 1 under FTP traffic and 80% RU.

    - 16.6%-28.6% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 10%-28.4% gain on rank 1 under FTP traffic and 80% RU.

    - 17%-39.2% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI,

    - Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively.

    - AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively.

    - AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively.

  - Huawei, Hisilicon: For achieving the same throughput, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

    - 73% in high overhead range for rank = 1.

    - 70% in high overhead range for rank = 2.

- Comparison with upper bound with ideal CSI without compression

  - Huawei, HiSilicon: With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI,

    - Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively

    - AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively

    - AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively

  - ZTE: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling

  - ZTE: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 7% for 5% tail UPT

  - ZTE: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 5.44% under the case of 50% RU.

  - ZTE: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 24.04% under the case of 50% RU.

  - ZTE: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 9.50% under the case of 70% RU.

  - ZTE: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 28.75% under the case of 70% RU.

  - ZTE: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.06% under the case of 50% RU.

  - ZTE: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 37.72% under the case of 50% RU.

  - ZTE: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.09% under the case of 70% RU.

  - ZTE: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 35.58% under the case of 70% RU.

  - ZTE: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 19.75% under full buffer traffic.

  - ZTE: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 45.21% under full buffer traffic.

- Findings on capability-related KPI

  - Samsung: The number of FLOPs to perform the AE operations is much larger than eType II

#### Preliminary trends observed by more than one company

  - Different effects of AI/ML on different layers: The benefit of AI/ML method is more/less obvious in higher rank CSI compression due to better efficiency on compression than legacy Type II. Huawei, Hisilicon, Ericsson, ZTE, OPPO Nokia, Google, InterDigital

    - Huawei, Hisilicon: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse, and AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more gains over the Rel-16 Type II on the 2nd layer than on the 1st layer.

    - ZTE: The case of rank>1 should be prioritized in future study.

    - OPPO: Compared to rank 1 achieving 5%~8% SGCS gain and 1%~3% SLS throughput gain, AI based CSI feedback achieves 8%~16% SGCS gain and 4%~10% SLS throughput gain for rank 2.

    - InterDigital: For the rank two case, the AI/ML-based CSI compression using transformer model achieves higher SGCS than the Rel-16 Type II with 20% reduction in overhead. Also, the achievable gain over Rel-16 Type II on the first layer is higher than that of the second layer

    - Google: For rank 1 case, the ML based CSI compression cannot provide significant gain on SGCS

    - Google: For rank >1 case, the ML based CSI compression can provide significant gain on SGCS.

  - Performance of different layers: 1st layer has higher SGCS than later layers Huawei, Hisilicon, ZTE, CATT

    - Huawei, Hisilicon: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse

  - Impact of CSI payload size: Compared to higher feedback overhead, AI based CSI feedback achieves larger gain with lower feedback overhead.

    - OPPO: Compared to higher feedback overhead achieving 1% for rank 1 and 4% for rank 2 SLS throughput gain, AI based CSI feedback achieves larger SLS throughput gain with lower feedback overhead, about 3% for rank 1 and 10% for rank 2

  - Impact of traffic load/RU: AI/ML approaches show better throughput performance over Rel-16 Type II CB with heavier/lighter traffic load. Huawei, Hisilicon, ZTE, OPPO, InterDigital

    - Huawei, Hisilicon: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook on high RU than low RU

    - ZTE: AI/ML approaches show better average throughput performance with heavy traffic load for up to rank 2

    - OPPO: Compared to FTP model achieving 1%~3% SLS throughput gain for rank 1, AI based CSI feedback achieves larger SLS throughput gain for full buffer model about 3%~6%.

    - InterDigital: The gain of the AI/ML compression over the baseline appears to decrease as the RU utilization increases. This trend is similar for both Mean UPT and 5% UPT

  - Effects on cell edge UEs: AI/ML can achieve more/less gains for cell edge UEs (than average). Huawei, Hisilicon, InterDigital

    - Huawei, Hisilicon: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook for cell edge UPT than average UPT

    - InterDigital: The gain of the AI/ML compression over the baseline appears to be smaller for low geometry UEs

Other views/findings

- NVIDIA: AI/ML based algorithms for CSI compression (e.g., using autoencoders) should be selected as a sub-use case for evaluation

### 3.1-2: AI/ML training methods

#### General findings/views

#### Unmatched/restricted model pairing

- Huawei, Hisilicon: For the performance comparison of training Type 3, the learning capability difference of model itself should not be attributed to the potential performance loss due to the specific training behaviors (e.g., dataset sharing).

- Apple: When mis-matched model is used, a simple fully connected encoder model with one hidden layer, together with transformer-based decoder, perform better than other combinations

#### Boundary between Type 2 and Type 3

- General views/findings

  - Huawei, HiSilicon: The boundary between Type 2 and Type 3 is whether the information exchange is required in each FP/BP loop or not:

- Type 2 training requires information exchange in each FP/BP loop.

- Type 3 training doesn’t require information exchange in each FP/BP loop.

  - Huawei, HiSilicon: If one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, it belongs to Type 2 training.

- Freeze and training [Type 4]

  - Ericsson: There is no noticeable performance degradation on using frozen/non-trainable decoder [Type 4 training] compared to the case of both encoder and the decoder are trainable

  - Qualcomm

    - gNB-first type3-alt training: gNB-Dec is trained first and then UE-Enc is trained with a frozen gNB-Dec

      - This approach may require exchange of activation (latent vector), gradient and corresponding target for loss function computation

    - UE-first type3-alt training: UE-Enc is trained first and then gNB-Dec is trained with a frozen UE-Enc

      - This approach may require exchange of activation (latent vector) and corresponding input

  - Qualcomm: gNB-first type3 training with dataset exchange performs worse compared to type1 and type3 with activation/gradient exchange (type3-alt), since type3-alt training is based on end-to-end loss minimization in contrast to latent space loss minimization which is used in type3

  - NTT DOCOMO: Type 3-B: Following the Type 3-B training paradigm, NW-side firstly uses UMa 4GHz dataset to train an encoder and decoder pair, then UE2 uses UMi 4GHz dataset to jointly train an encoder with the decoder on NW-side

  - NTT DOCOMO: Compared with Benchmark 2, when using Type 3-A training strategy for UE2 training, a significant decrease can be observed

  - NTT DOCOMO: Compared with Benchmark 2, when using Type 3-B training strategy for UE2 training, a slight decrease can be observed

  - NTT DOCOMO: When trainset on UE2 is different from dataset for NW-side training, the performance of Type 3-B training is better than that of Type 3-A training

#### Descriptions/Findings for Type 1

- vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

- Ericsson: The intermediate KPIs are not useful in drawing conclusion on expected system performance (that can be obtained in system level simulations).

#### Descriptions/Findings for Type 2

- General views/findings

  - Huawei, Hisilicon: For the evaluation of Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), analyze the feasibility for the following two cases of N>1 NW part models to M>1 UE part models (i.e., Case 4), and companies to report the training method/order, including the preference and details to achieve:

    - Case A: Simultaneously train all N Network part models and M UE part models involving all vendors

    - Case B: Sequentially train N Network part models and M UE part models in a pairwise manner

    - Note: The loss function should be separately generated for N>1 NW part models.

  - Huawei, Hisilicon: For Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), challenges are observed for both of the two training methods/orders of Case A (Simultaneously train all N Network part models and M UE part models involving all vendors) and Case B (Sequentially train N Network part models and M UE part models in a pairwise manner)

    - For Case A, there may be challenges on aligning the training timelines of all involved Network vendors and UE vendors.

    - For Case B, fairness may be harmed for later trained Network-UE pairs where only one side may be able to update parameters during the joint training.

- Samsung: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), for multi-vendors training Case 2, i.e., one NW part model and M>1 separate UE part models, and Case 3, i.e., one UE part model and N>1 separate NW part models, evaluate and compare the following two training scenarios

  - Scenario 1: training in a single session

  - Scenario 2: training in multiple sequential training sessions, i.e, one UE part and one network part at each training session.

- Different structures between NW part and UE part (NW and UE are not aware of the AI/ML model of each other) Huawei, Hisilicon, vivo, Qualcomm, MediaTek,

  - MediaTek: Define a mechanism/threshold to identify and avoid certain vulnerable pairings of encoders and decoders

Findings

  - Qualcomm: Type 2 offline training of the UE-side model and NW-side model is feasible even if the ML model structure of the UE-side and NW-side models are not matched.

  - MediaTek: On average over all unmatched pairs, UE loses 2.23% GCS performance and gNB loses 2.26%.

  - MediaTek: Overall, joint training on all pairs caused 1.68% performance loss

- One common NW part at NW to M>1 UE parts of different UEs Huawei, Hisilicon, Ericsson, vivo, Qualcomm, MediaTek, Fujitsu, InterDigital

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 1.1%~6.5% for one Network part model to M=3 UE part models with different backbones or different structures

  - Ericsson: Using intermediate KPIs, the joint multi-vendor training (based on Type 2 collaboration) with X payload, come with only some minor performance deterioration compared to the single vendor training (N=M=1) case.  For both N=1 and N=2 and M=2, the performance degradation in SGCS for layer 1 is less than 1%.

  - vivo: One common CSI reconstruction part could be trained to match multiple CSI generation parts of different UEs in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduces from 0.075 to 0.052, i.e., losing about 30% performance gain.

  - Qualcomm: It is feasible to use Type 2 offline training to train a common NW-side model together with separate UE-side models without any performance impact when compared to training a separate NW-side model for each UE-side model.

  - Fujitsu: For training Type 2, the significant SGCS loss can be observed for two UE part models and one NW part model, and the backbone is different, comparing with the joint training

- N>1 NW parts of different NWs to one common UE part at UE vivo, MediaTek

Findings

  - vivo: One common CSI generation part could be trained to match multiple CSI reconstruction parts of different networks in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain.

  - MediaTek: In single-encoder multi-decoder, we observe 0.3% performance loss compared to the joint training. The performance degradation for both UE and gNBs are almost equal to 2.0%, which again confirms superiority of single-encoder multi-decoder setting over multi-encoder single-decoder setting.

  - MediaTek: In both multi-decoder and multi-encoder training strategies, matched encoder-decoder pairs may promote themselves at the cost of degrading the performance of unmatched encoder or decoder

- N>1 NW parts of different NWs to M>1 UE parts at UE Huawei, Hisilicon, Ericsson, InterDigital

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 1.1%~6.5% for one Network part model to M=3 UE part models with different backbones or different structures.

  - Ericsson: Using intermediate KPIs, the joint multi-vendor training (based on Type 2 collaboration) with X payload, come with only some minor performance deterioration compared to the single vendor training (N=M=1) case.  For both N=1 and N=2 and M=2, the performance degradation in SGCS for layer 1 is less than 1%.

  - InterDigital: For the scenario analysed (M=2, N=2), only a marginal difference in the SGCS performance was found with respect to 1-on-1 joint training

  - InterDigital: further analysis over multiple random training initializations (higher Monte Carlo iterations) and with much larger M and N and may be required to draw a conclusion on the relative performance of Type 2 multi-vendor joint training.

- Loss function generation of N>1 CSI reconstruction parts of different NWs

  - MediaTek: Discuss how to calculate a joint loss to avoid adverse bias toward the matched pairs

    - In our pilot study, we use a joint loss that is simply calculated by averaging over all individual losses of gNBs

    - there are cases that the matched encoder-decoder pairs promote themselves at the cost of degrading the performance of unmatched encoders or decoders. In this regard, we believe a simple average has the risk of biasing the common encoder/decoder to its matched decoder/encoder.

- Dataset interaction/training order

  - MediaTek: Study different parameters’ update scheduling for multi-encoder or multi-decoder settings using any training strategy

    - The UEs can share a common dataset or employ UE-specific datasets, also UE/gNB vendors may undergo concurrent, alternating, and sequential updating schedule

#### Descriptions/Findings for Type 3

- Direction 1: Sequential training starting with Network side training (NW-first training) Huawei, HiSilicon, Qualcomm, Ericsson, MediaTek, Samsung, ZTE, CATT, Xiaomi, CMCC, OPPO Fujitsu, Nokia

Findings

  - Ericsson: For training Type 3 and N=M=1 case, NW-first training works well with a minor performance degradation compared to training Type 1. UE first training show in these evaluations a slight performance degradation.

  - Xiaomi: When AI model structure is known, separate training can achieve similar performance with joint training

  - CMCC: With large enough dataset samples at UE side, separate training could achieve similar SGCS as joint training

  - Fujitsu: For the Case 1 of Type 3 training, only a negligible SGCS degradation (0.0001~0.0011) is observed compared to joint training

  - ZTE: For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training

How to achieve N NW part models to M UE part models (Multiple NW parts of different NWs to one common UE part at UE) Huawei, HiSilicon, MediaTek, Samsung, Fujitsu, Apple

  - Findings

    - Huawei, HiSilicon: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training when the separate training considers N>1 Network part models to one common UE part model with a backbone with similar or stronger learning capability than any of the N Network part models.

    - MediaTek: On average, using gNB-first separate training for single-encoder multi-decoder setting degrades the performance by -58.97% compared to joint training for single-encoder multi-decoder setting

    - Samsung: In NW-first training, i.e., Type 3 training between one UE part model and N>1 separate NW part models, severe performance degradation is observed when the multiple NW parts are from different backbone

    - Fujitsu: For the Case 3 of Type 3 training, where the training at NW side is performed at first, the SGCS degradation is negligibly 0.003~0.006 compared to that of Case 1 of Type 3 training if the backbone of the AI/ML model is transformer.

    - Apple: For training collaboration type 3 with NW first training

      - One UE encoder to one NW decoder training achieve similar performance as joint training

      - One UE encoder to multiple NW decoders training observes large performance degradation.

- Direction 2: Sequential training starting with UE side training (UE-first training) Huawei, HiSilicon, vivo, Qualcomm, Ericsson, MediaTek, ZTE, Nokia, CATT OPPO Fujitsu, Lenovo, Apple

  - MediaTek: Give higher priority to UE-first separate training if separate training is adopted as the main training framework

    - gNB-first separate training has inferior performance compared to UE-first separate training for any number of encoders and decoders participating in the training session

Findings

  - Huawei, HiSilicon: For AI/ML-based CSI compression, there is minor margin (<1%) between the SGCS of the UE first separate training and the SGCS of the joint training even if the backbone of Network-side CSI reconstruction part has similar or stronger learning capability than that of UE side CSI reconstruction part.

    - This observation applies regardless when the dataset for the Network side is only a subset of or equal to the dataset for the UE side

  - Qualcomm: UE-first type3 training with dataset or activation exchange can achieve the same performance of Type1 training

  - Qualcomm: Joint and sequential training (e.g., UE-first type3) training achieves similar SGCS performance for rank >1

  - Ericsson: For training Type 3 and N=M=1 case, NW-first training works well with a minor performance degradation compared to training Type 1. UE first training show in these evaluations a slight performance degradation.

  - Lenovo: AI/ML-based CSI feedback models, trained using joint or separate training scheme, leads to better performance compared to the performance of eType II

  - MediaTek: it is evident that UE-first separate training strategy is not necessarily underperforming joint training. In fact, matched pairs achieved 1.2% higher reconstruction accuracy using UE-first separate training strategy.

  - Fujitsu: For the Case 1 of Type 3 training, only a negligible SGCS degradation (0.0001~0.0011) is observed compared to joint training

  - Apple: For training collaboration type 3 with UE first training

    - One UE encoder to one NW decoder training achieve similar performance as joint training

    - Multiple UE encodes to one NW decoder training achieve similar performance as joint training.

How to achieve N NW part models to M UE part models (One common NW part at NW to multiple UE parts of different UEs) vivo, Qualcomm, MediaTek, Fujitsu, Lenovo

  - Findings

    - vivo:	One common CSI reconstruction/generation part could be trained to match multiple CSI reconstruction/generation parts of different UEs in training collaboration type 3 at the cost of some performance loss (e.g., considering one common CSI reconstruction part to three CSI generation part and each UE sharing 50,000 samples with NW, the performance loss in SGCS is around 0.04).

    - vivo:	Performance of one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts of different networks/UEs is affected by the amount of exchanged data from each network/UE.

    - vivo:	Performance loss in supporting common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts gets worse as the number of supported UEs/networks increases

    - Qualcomm Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training

    - MediaTek: Using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -37.45% compared to single-encoder single-decoder setting

    - MediaTek: Using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -36.66% compared to joint training for multi-encoder single-decoder setting

    - MediaTek: Using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -37.45% compared to joint training in multi-encoder single-decoder setting

    - MediaTek: On average, UE’s gain is -37.84% from UE-first separate training for multi-encoder single-decoder setting

    - MediaTek: On average, gNB’s gain is -37.99% from UE-first separate training for multi-encoder single-decoder setting

    - Fujitsu: For the Case 2 of Type 3 training, where the training at UE side is performed at first, the SGCS degradation is 0.11~0.03 compared to that of Case 1 of Type 3 training if the backbone of the AI/ML model is transformer.

- Different structures between NW part and UE part for NW-first and UE-first (NW and UE are not aware of the AI/ML model of each other)

  - Solution description: 4 cases are raised by companies in the evaluations

    - Case 1 (baseline): Same backbone/hyperparameters/quantization method at NW ane UE Huawei, HiSilicon, vivo, MediaTek, CATT Xiaomi, CMCC OPPO Xiaomi ZTE Fujitsu

    - Case 2: Same backbone and quantization method at NW and UE, but different hyperparameters (e.g., different number of layers) between NW and UE Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT Nokia Fujitsu, Nokia

    - Case 3: Same quantization method at NW and UE, but different backbones between NW and UE (e.g., one side is Transformer, the other side is CNN/ResNet) Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT, Xiaomi, CMCC Xiaomi Fujitsu

    - Case 4: Different backbones (and hyperparameters)/quantization methods between NW and UE vivo Fujitsu

  - Solution description-different dataset size: dataset used for the second training entity is only a subset (e.g., including CSI only corresponding to the second training entity) of the dataset used for the first training entity (e.g., including CSI corresponding to multiple second training entities): Huawei, HiSilicon, vivo CATT, CMCC OPPO Lenovo

    - Huawei, HiSilicon: For NW-first training, two cases are evaluated:

      - Case 1: The shared dataset includes the full dataset (300K samples) at the training of the NW side, i.e., input and label of the NW side CSI generation part

      - Case 2: UE only uses its own CSI as labels (100K samples) and the corresponding CSI feedback shared by NW as inputs

    - Huawei, HiSilicon: For UE-first training, two cases are evaluated:

      - Case 1: The set of information includes the full dataset (300K samples) at the training of the UE side, i.e., the input and label of the UE side CSI reconstruction part

      - Case 2: Network only uses its own CSI as labels (100K samples) and the corresponding CSI feedback shared by UE as inputs

    - vivo: Each UE reports 10,000, 50,000, or 300,000 data samples for separate training, and the gNB combines all reported data to train the CSI reconstruction model

    - CATT For sequential training, we also provided simulation results with dataset in step 2 which is the only half size of dataset in step 1.

    - CMCC For case 2, 3 and 4, the decoder part at network side is the same as that of case 1, and the encoder parts at the UE side are trained based on Transformer, EVCsiNet and MLP-Mixer respectively using different number of dataset samples, i.e., 154K, 100K, 50K, 10K, 5K.

  - OPPO: for 600k, 300k, 100k and 50k training data, the total type 3 training overhead is about 1909M, 954M, 319M and 160M

Views on different structures/dataset sizes

  - Huawei, Hisilicon: For the evaluation of training Type 3 under CSI compression, the new case (1-on-1 joint training) benchmark/upper bound for performance comparison should separately take into account the following aspects:

    - Aspect 1: The same pair of Network part model and UE part model to the Type 3 case being compared.

    - Aspect 2: The same pair of Network part CSI reconstruction model and virtual Network part CSI generation model for NW first training Type 3 case being compared, or UE part CSI generation model and virtual UE part CSI reconstruction model for UE first training Type 3 case being compared.

  - Lenovo: Study mechanisms to reduce the degradation due to the difference between the structure of the “NW-side actual” and “the UE-side nominal” decoder model.

  - MediaTek: In the UE-first separate training strategy, UE should inform gNB about the type of its architecture

  - MediaTek: In the gNB-first separate training strategy, gNB should inform UE vendor at least about the type of its dropped encoder’s architecture

    - this problem is more severe for gNB-first separate training

  - Fujitsu: For the evaluation of the Type 3 training, evaluate the effect of the choice of backbone of AI/ML model on the performance of Type 3 training

  - Fujitsu: Evaluate the performance of separate training (training Type 3) for the case that the quantization and the dequantization are mismatched

Findings on different structures/dataset sizes

  - Huawei, HiSilicon: For separate training, good performance can be achieved by pairing with comparable learning capabilities between Network side model and UE side model, which may be easily aligned by, e.g., sharing the target performance metric of the models in together with the shared dataset.

  - Huawei, HiSilicon: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training if the backbone of UE-side CSI generation part has similar or stronger learning capability than that of Network side CSI generation part

    - This observation applies regardless when the input dataset for the UE side is only a subset of or equal to the input dataset for the Network side

  - Lenovo: The performance of the AI/ML-based CSI feedback models trained using separate training method degrades when there is mismatch between the structure of the “NW-side actual” and the “UE-side nominal” decoder model

  - Lenovo: The degradation performance is more significant if there is more considerable difference between the structure of the “NW-side actual” and the “UE-side nominal” decoder model. In one simulation, the degradation could be up to 78% of the gain

  - vivo: If the model structure is not aligned (e.g., dequantization method at decoder and the quantization method in encoder could not match), there will be an obvious performance loss compared with that in case where the dequantization and quantization method are matching

  - MediaTek: Even unmatched pairs experience 0.93% improvement compared to their performance when joint training is adopted.

  - MediaTek: In single-encoder single-decoder setting using gNB-first separate training, UE and gNB almost equally suffer from performance degradation in gNB-first separate training. UE and gNB respectively experience 1.52% and 1.42% performance loss compared to their corresponding matched designed trained via joint training strategy

  - CATT: For separate training for AI/ML based CSI compression, compared to joint training

    - Similar performance can be achieved when the UE side CSI generation part and network side CSI reconstruction part use aligned AI/ML model structure (both use transformer based AI/ML model with 6 layers);

    - Performance loss is tiny when both the UE side CSI generation part and network side CSI reconstruction part use transformer based AI/ML model, but with different number of layers;

    - Performance loss can be obtained (2% ~ 6.6%) when one of the UE side CSI generation part and network side CSI reconstruction part uses transformer based AI/ML model and the other one uses ResNet based AI/ML model

  - CATT: For sequential separate training, compared to dataset in step 2 has the same size as dataset in step 1, minor performance loss can be seen for dataset CSI reconstruction part has half size of dataset in step 1

  - Xiaomi: When different AI backbone is used for CSI generation part and CSI reconstruction part, performance of separate training decreased compared with joint training

  - CMCC: When the number of dataset samples at UE side decreases, the SGCS of separate training will also decrease.

  - CMCC: When the generation part at UE side and the reconstruction part at network side have the same AI algorithms or model structures, to ensure separate training achieve similar SGCS as joint training, the requirement of number of dataset samples at UE side is much lower than the requirement when the AI algorithm or model structure is different between UE side and network side.

  - OPPO: Compared with type 1 training, NW-first type 3 training has 0.9%~6.6% and UE-first type 3 training has 0.3%~3.6% SGCS performance loss with 160M~1909M overhead, respectively

  - OPPO: NW-first Type 3 training with unmatched AI/ML model pairing using ResNet encoder and Transformer decoder has 12.3% SGCS performance loss compared with Type 1 1-on-1 joint training Transformer baseline

  - Fujitsu: For the Case 1 of Type 3 training, by varying the backbones of AI/ML models and fixing other conditions, it is observed that the performance of transformer models is superior to that of a convolutional neural network (CNN).

  - Fujitsu: The significant SGCS performance loss can be observed for separate training (training Type 3) when the quantization and the dequantization are mismatched.

  - Nokia: The performance loss due to separate training with mismatched transformer-based models is small compared to joint training

  - Nokia: A common NW-side decoder can accommodate multiple UEs with different NN structures

- Direction 3: Parallel training

Solution description

  - CATT: Parallel training means the AI/ML model at the UE side and the AI/ML model at the network side are trained separately, with no distinguishable sequential order. In other words, parallel training is order-agnostic training. One example of parallel training is as follows:

    - UE side trains the UE side CSI generation part with dataset #A;

    - Network side trains the network side CSI reconstruction part based on dataset #B;

    - Dataset #A and dataset #B can be the same or not

  - CATT: Parallel training with both the UE side CSI generation part and the network side CSI reconstruction part trained with the same dataset of {Channel, target CSI}, where “channel” is used as the input, and “target CSI” is used as output for the training of the UE side CSI generation part; and “target CSI” is used as the input, and “channel” is used as output for the training of the network side CSI reconstruction part

Views/Findings

  - CATT: For separate training, similar performance can be achieved by parallel training and sequential training.

  - CATT: For separate training, parallel training is supported for further studied and evaluation

- Direction 4: Iterative separate training Lenovo

Solution description

        - UE-side trains the two models, encoder and UE-side nominal decoder, jointly

        - Each UE-side transmits samples representing the input and the expected output of the decoder to the NW-side.

        - The NW-side uses the samples received from all the UE-sides to train its decoder.

        - The NW-side sends samples to each UE-side representing the input and output of the decoder

        - Each UE-side retrains its respective “UE-side nominal decoder” using the received samples from the NW-side (or all samples received from all NW-sides in case there are multiple NW-sides)

        - Each UE-side retrains the “encoder” based on its updated “UE-side nominal decoder”.

        - The process can be stopped, or each UE-side can send another set of samples representing the input and expected output of the UE-side nominal decoder to the NW-side and repeat the process.

Findings

  - Iterative separate training recovers the loss due to mismatch between different vendors. For example, the performance of UE-1 Case-2 is back to 84.3% (from 81.5%) after application of iterative separate training

  - In some cases, the updated-decoder (using iterative separate training) surpass the performance of initial joint-training, e.g., UE-1 Case-1. It might be due to the availability of more data and the fact that the “actual decoder” at the NW-side is a more complex model that the “UE-side nominal decoder”. 
We note that, assuming the existence of a complex decoder and enough training samples we would expect the performance of the joint-training will be the upper-bound

Views

  - Study the performance gains and the extra training costs incurred by, “iterative separate training” as a potential method to improve the performance of cases with mismatch between different sides

- Direction 5: Share {Input of nominal encoder, CSI feedback, Output of decoder} to UE Nokia

  - Nokia: The first step is the same as in Method 1, but in the second step the network also saves the decoder output in the dataset.  The appended dataset is now V, qV,V'.  In the third step, the UE vendor trains a hypothetical decoder (DEC2) using the appended dataset.  Finally, the UE vendor freezes the hypothetical decoder (DEC2) and trains its encoder (ENC2).

  - Nokia: For NW-first training, the performance loss due to separate training is reduced when a hypothetical decoder is trained by the UE and used in the encoder training

- Other views/findings for Type 3

  - Lenovo: Study methods for generating representative datasets for evaluation of multi-vendor scenarios.

#### Impact of device types

- Qualcomm: The performance of AI/ML-based CSI compression using two-sided model can vary considerably if there is a discrepancy between the training data and inference data due to device-side variations

  - The data corresponding to different types of devices may have different characteristics. The source of such differences could be from device construction, RF aspects, implementation differences across vendors or device models or chipsets, etc.

- Qualcomm: Gradient-exchange based Type 3 training enables the training of encoders for new devices in a scalable and backward compatible manner without requiring updates of the NW-side models and avoids data distribution mismatch issues

- Lenovo: To study the performance of Multivendor separate training, consider different model structures and also datasets with different statistics for different vendors

- Lenovo: For Multi-vendor separate training, AI/ML-based CSI feedback models with separate training have better performance compared to the performance of Rel.16-eType II

- Lenovo: Multi-vendor separate training, experience performance loss compared to joint-training. The degradation is more significant when there is a mismatch between the model/data of different vendors. For example, the performance of Separate training for case-2 UE-vendorA is much less than the joint-training (it is almost as low as Rel.16-eType II.)

- Lenovo: In multivendor separate training case, study mechanisms to reduce the degradation due to the difference between the model structures and the training datasets at different vendors

- NTT DOCOMO: These differences include UE located in different environments, different UE antenna layouts, and UE antenna imbalance, i.e., there is a gap on the antenna gains of two UE antennas due to the different antenna positions on the UE or the shadowing by other UE components (e.g., PCB).

- NTT DOCOMO: There is no significant performance loss on SGCS when directly applying a well-trained model based on UMa/UMi dataset to the cases that UE is located in different environment and has different antenna layouts or antenna gain imbalance.

- NTT DOCOMO: For the cases that UE is located in different environment, Type 3-A or Type 3-B training for adaption to the environment has similar performance with the cases that directly apply the model.

- NTT DOCOMO: The CSI compression model is robust against non-ideal factors including different UE antenna layout, imbalance of UE antenna gains, or different channel environments.

#### Quantization / dequantization in training

- Quantization aware training/ Quantization non-aware training

Solution description

  - Case 0: Training & inference w/o quantization

    - Xiaomi: Training without quantization, inference without quantization

  - Case 1: Quantization non-aware training: where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training Huawei, Hisilicon, Ericsson, vivo, Qualcomm, MediaTek, Nokia, Apple, Mavenir

    - MediaTek: Adopt training-non-aware non-learnable scalar quantization (TNA-NL-SQ) as the baseline for evaluation of all other quantization methods

    - Apple: we train the transformer base auto-encoder without quantization, the floating-point latent dimension is set to 30. After that, we apply scaler codebook and vector quantization codebook which quantize one float point value to 2 bits

  - Quantization-aware training: where quantization/dequantization is involved in the training process.

    - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase Ericsson, vivo, Qualcomm, MediaTek, Nokia Apple Xiaomi, Mavenir FUTUREWEI

      - Apple: the VQ codebook is trained using Linde-Buzo-Gray (LBG) algorithm based on floating point encoder output

      - Xiaomi: Case 2-1: Scalar quantization

    - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase Huawei, Hisilicon, vivo, Qualcomm, MediaTek, Nokia Xiaomi, FUTUREWEI

      - Huawei, Hisilicon: the dictionary/codebook for the vector quantization is generated by applying the clustering algorithm to the outputs of the well-trained CSI generation part.

      - MediaTek: As the VQ is a training-non-aware method in essence, we only use it in the inference stage.

      - Xiaomi: Case 2-2: Vector quantization

Views

  - vivo: Quantization method at UE side and dequantization method at NW side should be aligned for training collaboration type2 and 3 to achieve a satisfying performance

  - Nokia: Define quantization loss as the difference between the reconstruction metric (SGCS or NMSE) obtained from a model trained without quantization and a model which incorporates quantization whether through quantization aware or unaware training. Report the loss in dB

  - Xiaomi: The case of training without quantization and inference without quantization should be evaluated as the upper bound

  - AT&T: For the evaluation of quantization aware/non-aware training, on top of the agreed cases of quantization non-aware training (Case 1) and quantization aware training (Case 2-1, Case 2-2), introduce an additional upper bound case of Case 0: non-quantized training and inference (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training and inference), to reflect the performance loss due to quantization

  - ETRI: For the evaluation of CSI compression sub use-case with quantization aware/non-aware training, consider not introducing training and inference using unquantized latent variables for the performance upper bound

  - OPPO: For training collaborative Type3, study whether/how to align the quantization and dequantization method between UE and NW

  - Fujitsu: Quantization aware training is considered as the priority training strategy for the evaluation of AI/ML based CSI compression feedback

  - Fujitsu: In quantization aware training, it is suggested that we increase the number of floating-point outputs for a fixed number of output bit numbers

  - Fujitsu: For the evaluation of the quantization aware training, study and compare the throughput achieved by the approaches that the quantizers are updated (Case 2-2) or not (Case 2-1) during the training phase. This provides evidence for studying which of the two should be considered as the priority method.

  - Qualcomm: Quantization method should be considered a part of the UE-side model and dequantization method should be considered a part of the NW-side model. The quantization method should be aligned for good performance, but there is no need for separate specification support to align the quantization method

  - ETRI: For the evaluation of AI/ML-based CSI compression use case, companies to report the details on the quantization/dequantization including:

    - Functional separability of compression and quantization

    - Configuration of quantization/dequantization block (Scalar or vector quantization, fixed or dynamic codebook)

    - Quantization aware/non-aware training of the AI models

  - Comba: Study the dequantizer and try to eliminate the noise introduced in quantization.

  - Comba: Research quantitative parameters and try to find the best parameter training method.

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, quantization non-aware training shows significant performance loss compared to quantization aware training

  - Ericsson: Quantization aware training provides better performance and/or fewer quantization bits for the same performance, compared to the quantization non-aware training

  - Ericsson: In training Type 3 quantization-aware-training performs better compared to the quantization non-aware training

  - vivo: Quantization non-aware training only achieves good performance when the averaged quantization bit is large (e.g., >= 4bits/float). When the averaged quantization bit is small (e.g., <= 2bits/float), the performance loss is significant

  - vivo: Performance of quantization non-aware training could be significantly lower than that of quantization aware training (more than 0.1 in SGCS in our considered configurations)

  - Qualcomm: Quantization non-aware training (case-1) leads to noticeable performance degradation compared with quantization aware training (case-2)

  - Qualcomm: For quantization aware training, fixed or pre-configured quantization (case 2-1) is more sensitive to quantization’s parameters/configuration compared with trainable quantization (case 2-2). That is, quantization’s parameters/configuration in case 2-1 need to be carefully chosen to align with statistical distribution of latent vector (z), otherwise performance is degraded.

  - Qualcomm: Trainable quantization offers more flexibility and better performance compared to fixed quantization, e.g., trainable vector quantization can improve the performance.

  - MediaTek: Training awareness improves the gain of quantization methods by 6.5%, Learnability improves the gain of quantization methods by 5.1%,

  - Nokia: Quantization-aware training performs better than quantization-unaware training due to the ability to optimize the model to the quantization method within the training process

  - Nokia: Case-1 training can provide acceptable performance and adapt easily to different feedback sizes without the need to change the AI model

  - Apple: For quantization aware training, fixed VQ codebook can achieve similar performance comparing to joint VQ codebook/auto-encoder design

  - Xiaomi: There is around 7% performance loss caused by quantization

  - Fujitsu: For scalar uniform quantization, the performance of quantization aware training is better than that of quantization non-aware training, and the performance loss of quantization non-aware training is reduced with the increasing quantization bits and feedback payloads

    - Compared with quantization aware training, there is a 0.01~0.03 SGCS loss for quantization non-aware training under various feedback payloads

  - NVIDIA: Both quantization aware training and quantization non-aware training have been agreed to be evaluated for AI/ML based CSI compression

  - InterDigital: Quantization aware training outperforms (in terms of SGCS) the quantization non-aware training, at the expense of lacking generalization on the payload size.

  - InterDigital: Quantization non-aware training has more flexibility on payload size generalization, at the expense of some SGCS performance degradation

  - Comba: Quantization introduces a lot of noise that affects decompression performance.

  - Comba: Quantization aware training can perform better than Quantization non-aware training.

Preliminary trends observed by more than one company

Comparison between quantization aware and non-aware

  - For AI/ML-based CSI compression, quantization non-aware training shows clear performance loss compared to quantization aware training Huawei, Hisilicon, Ericsson, vivo, Qualcomm MediaTek, Nokia, Fujitsu, InterDigital, Comba

    - InterDigital: at the expense of lacking generalization on the payload size

Impact of quantization granularity on payload scalability

  - Nokia: Continue to study quantization unaware and quantization aware training to assess the different approaches to payload scalability

  - MediaTek: Study alignment requirement for payload-scalability methods between CSI generation and CSI reconstruction parts of AI/ML models

    - payload-scalability at the encoder can be achieved by applying different quantization methods with various codeword lengths

  - ETRI: AI/ML models with a functionally separable quantization block can adjust CSI payload sizes by using different quantization configurations

Fine-tuning on quantization granularity

  - Solution design: The pair of UE part and NW part models are trained based on training dataset from Quantization granularity#A, and then the NW part model is updated based on a fine-tuning dataset with Quantization granularity#B where the UE part model is unchanged. After that, it is tested on Quantization granularity#B.

  - Ericsson:	By only fine-tuning the decoder, the performance degradation resulting from training with the wrong quantization, can be improved from -5.5% to -1.3%.

  - Comba: Model fine-tune can well adapt to changes in certain parameters and conditions of the model

  - Comba: Different quantization bits make the performance of the model different

  - Comba: Research on method of model fine-tune in practical applications

- Scalar quantization/vector quantization

Design of SQ

  - Nokia: For scalar quantization, we only consider uniform 2-bit quantization.

  - vivo: To define a specific scalar quantization rule, we can directly define the number of bits assigned to each float. For example, we can use a vector [2, 2, …, 2, 3, …, 3, …, 4] to express a scalar quantization method, which assigns 2 bits to the first several float number, 3 bits to the next several float number, and 4 bits to the last several float number. The most trivial scalar quantization method is to uniformly assign K bits for all float numbers in a sequence.

Views/Findings for SQ

  - Fujitsu: For a fixed number of payload size, the number of floating-point outputs of the neural network followed by a small-resolution quantizer is larger than that followed by a high-resolution quantizer. This is helpful in increasing the SGCS performance if quantization aware training is used.

  - FUTUREWEI: For CSI feedback compression sub use case, when utilizing scalar quantization (in quantization-aware training), SGCS performance improves with the number of quantization bits per code when lower number of bits are used (e.g., <= 6 bits per code observed in our study). However, SGCS performance saturates after quantization level reaches certain point and does not improve much beyond that point (e.g., around 6 bits per code observed in our study).

  - FUTUREWEI: For CSI feedback compression sub use case, when utilizing scalar quantization (in quantization-aware training), SGCS performance improves when codesize (output size of the CSI generation part before quantization) increases; more noticeable gain is observed in smaller codesize region, e.g., from codesize 8 to codesize 16, particular when smaller number of quantization bits are used in our study, and the performance gain becomes smaller in larger codesize region, e.g., from codesize 32 to codesize 64, particular when larger number of quantization bits are used.

  - FUTUREWEI: For CSI feedback compression sub use case, when utilizing scalar quantization method to quantize each code in the output of the CSI generation part, determine the optimal quantization level, i.e., number of bits per code, based on the performance saturation point where performance improvement is no longer significant and the total number of overhead bits.

  - InterDigital: The latent values have a non-uniform distribution

  - InterDigital: For quantization non-aware training: non-uniform quantization improves the SGCS performance significantly compared to uniform quantization

  - InterDigital: For quantization non-aware training: the per-latent quantization outperforms the all-latent quantization.

  - InterDigital: For quantization non-aware training: the clustering and CDF-based non-uniform quantization show similar SGCS performance

Design of VQ

  - MediaTek: Designing VQ should be done by UE, and gNB can optionally fine-tune its CSI reconstruction part.

  - MediaTek: The UE should inform gNB about its VQ design to maintain the alignment for quantization and dequantization parts.

  - MediaTek: Therefore, instead of having a one-to-one mapping from CSI generation part’s output to a sequence of quantized feedback, we segment the CSI generation part’s output into multiple segments, and design/apply VQ on each segment individually.

  - Nokia: For vector quantization, the dimension of each segment is given by D and B is the number of bits allocated to the VQ codebook, where B = D × Feedback bits/element.  For Case 1, the K-means algorithm is used to determine the VQ codebook. For Case 2-2, the VQ codebook elements are jointly adapted with the encoder and decoder parameters by incorporating the VQ performance into the loss function.

  - vivo: The whole sequence to be quantized will usually be partitioned into several segments, as it is difficult to directly quantize the whole sequence…For example, 80 float variables can be partitioned into 16 sub-sequences, each of which is of size 5. Correspondingly, we can set 16 quantization codebooks, each of which will be used to quantize one segment…The quantization procedure is to select one codeword in the codebook that most represents the input, and the most common criteria is to select the one with the least MSE distance to the input.

  - FUTUREWEI: UE uses a look-up-table delivered from the NW to generate the CSi feedback, rather than using an encoder.

    - During the training phase, the training entity, e.g., gNB generates a CSI generation part, the corresponding vector quantization codebook, and a CSI reconstruction part. The quantization codebook and the CSI reconstruction part are delivered to the UE side (e.g., via model transfer) after training/testing is finished vs. delivering the CSI generation part to UE side. At the UE side, once it receives the quantization codebook and the CSI reconstruction part, it derives a CSI codebook by using the vector quantization codebook entries as input to the CSI reconstruction part of the AI/ML model, which is referred as CSI look-up-table (LUT) later in the document.

    - During the inference/testing phase, for each testing sample/input (e.g., eigenvectors of channel matrix), instead of using the CSI generation part and the quantizer to generate the CSI feedback at the UE side, UE selects an entry from the LUT (derived during the training phase described above) based on the estimated CSI and a selection criterion, e.g., SGCS used in our study. The LUT entry that has the highest SGCS will be selected as the CSI feedback, and the index of the selected LUT entry will be reported to gNB

Views/Findings for VQ

  - MediaTek: VQ is very sensitive to small changes in the distribution of CSI generation part’s output.

  - MediaTek: Re-using VQ codebook even for the AI/ML models of the same structure degrades its gain by 79.6% on average

  - FUTUREWEI: For CSI feedback compression sub use case, when utilizing vector quantization (in quantization-aware training), SGCS performance improves with the number of quantization bits per CSI output while the codesize (output size of the CSI generation part before quantization) increase does not impact performance significantly even though gradual improvement is observed.

  - FUTUREWEI: For CSI feedback compression sub use case, when utilizing vector quantization method to quantize the entire output of the CSI generation part (i.e., a vector), determine the proper codesize and quantization level combination based on the balance/tradeoff between performance and overhead incurred in codebook storage and codebook transfer over the air interface

  - FUTUREWEI: For CSI feedback compression sub use case, determining CSI feedback by using a CSI look-up-table constructed from the CSI reconstruction part and a vector quantization codebook shows promising result compared to both Rel-16 Type II codebook-based approach and typical AI/ML model-based approach that uses the CSI generation part and vector quantizer/vector quantization codebook to predict the CSI feedback

  - FUTUREWEI: For CSI feedback compression sub use case, further study the robustness and feasibility of using a CSI look-up-table (constructed from the CSI reconstruction part and a vector quantization codebook) based approach to determine the CSI feedback.

Comparison between SQ and VQ

  - vivo: Vector quantization with optimized codebook can achieve slightly better performance (e.g., by about 0.009 in SGCS in our considered configurations) than scalar quantization with fixed codebook

  - vivo: Performance of vector quantization with randomly initialization and fixed codebook can be slightly inferior to that of scalar quantization with fixed codebook (e.g., by about 0.0065 in SGCS in our considered configurations)

  - MediaTek: Fine-tuning CSI reconstruction part of AI/ML model in the presence of TNA-VQ improves the CSI reconstruction accuracy gain over TNA-NL-SQ by 62.7%.

  - MediaTek: On average, TA-NL-SQ, TA-L-SQ, and TNA-VQ achieve 7.8%, 11.6%, and 4.1% gain over TNA-NL-SQ, respectively.

  - MediaTek: SQ methods of all kinds, on average, outperform VQ by 1.7%.

  - MediaTek: Give higher priority to SQ methods given their performance, less sensitivity, and less alignment efforts in multi-vendor ecosystems

  - Nokia: Vector quantization is found to have a smaller quantization loss than both uniform and non-uniform scalar quantization

  - Xiaomi: Vector quantization and scalar quantization can achieve similar performance

  - Fujitsu: Under the method of quantization aware training, compared with the fixed scalar uniform quantization method (Case 2-1), better performance is achieved in a updated quantization approach (Case 2-2), where the vector quantization is updated together with the AI/ML models during the training phase.

  - FUTUREWEI: utilizing vector quantization can more significantly reduce CSI overhead compared to scalar quantization-based approach, e.g., ~10 bits CSI overhead vs. > 100 bits CSI overhead

- Alignment on vector quantization/dequantization for separate training

Solution description

  - Approach 1 (Quantization non-aware training)

    - UE server trains the encoder without quantization and shares the dataset (z, Vtarget); Vector quantizer (VQ) is trained with the decoder at NW-side training entity Qualcomm Huawei, Hisilicon, MediaTek

      - Case 1: Upon designing TNA-VQ, the entire AI/ML model is fine-tuned in its presence. MediaTek

      - Case 2: Upon designing TNA-VQ, only CSI reconstruction part of AI/ML model is fine-tuned in the presence of quantization. MediaTek

      - Case 3: Upon designing TNA-VQ, the entire AI/ML model will be re-initialized and trained from scratch in the presence of TNA-VQ. MediaTek

  - Approach 2 (Quantization-aware training)

    - Case 1: output of CSI generation part (input of CSI reconstruction part) is shared before quantization Qualcomm ZTE Fujitsu

    - Case 2: output of CSI generation part (input of CSI reconstruction part) is shared after quantization Qualcomm ZTE Fujitsu, Nokia

      - Case 2-1: NW shares the dataset V, I and the finalized codebook C. Nokia

      - Case 2-2: NW shares the dataset V, I. The UE-proprietary encoder is trained to output the codeword I that closely matches the shared I Nokia

      - Case 2-3: NW shares the dataset V, I. The UE creates a hypothetical codebook and a hypothetical decoder to facilitate the UE-side encoder Nokia

Views

  - Fujitsu: For the evaluation of the Type 3 training, evaluate the effect of the choice of quantizer on the performance of Type 3 training, from the perspectives of

    - Training method: quantization aware training or quantization non-aware training.

    - Quantization method: scalar quantization or vector quantization

  - Nokia: For NW-first separate training with vector quantization, the UE does not need to have access to the codebook for retrieval of the quantized latent representations. The codebook for mapping the codeword to quantized latent representation can be NW-proprietary

Findings

  - Qualcomm: Training UE encoder without quantization and generating the separate training based on this encoder may lead to some performance degradation compared to encoder training with quantization

  - Qualcomm: Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training

  - Qualcomm: For approach 2, we observe that separate training based on (ze, Vtarget) or (zq, Vtarget) leads to almost same performance

  - Fujitsu: There is negligible performance difference between two quantization behaviors for both NW-first and UE-first separate training (Case 1 and Case 2)

  - Nokia: Case 2-2 & Case 2-3 demonstrate that the UE, without the knowledge of NW-side codebook for mapping codeword to latent vector, can still be effectively trained without performance degradation

  - MediaTek: Updating the entire AI/ML model through re-training/fine-tuning in the presence of TNA-VQ will degrade overall CSI reconstruction accuracy

  - MediaTek: Fine-tuning CSI reconstruction part of AI/ML model in the presence of TNA-VQ improves the CSI reconstruction accuracy gain over TNA-NL-SQ by 62.7%.  In overall, fine-tuning gain is not significant. It is 1.2% gain compared to non-tuned AI/ML model

### 3.1-3: AI/ML model settings

#### Remaining issues on AI/ML model settings for rank>1

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

  - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

  - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

Solution/views

  - Huawei, Hisilicon: For the rank >1 options under AI/ML-based CSI compression, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

    - Option 1-1 (rank specific): Max FLOPs over K rank specific models.

    - Option 1-2 (rank common): FLOPs of the rank common model.

    - Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

    - Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

    - Option 3-1 (layer common and rank common): K * FLOPs of the common model.

    - Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

  - Huawei, Hisilicon: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

    - Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

    - Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

    - Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

    - Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

    - Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

  - Huawei, Hisilicon: For a given rank, layer common options only need to run a single model in one processing where the multiple layers can be processed as one batch by making use of the GPU, which largely alleviates the complexity as opposed to the layer specific models.

  - MediaTek: we believe at least among the layer-specific and layer-common AI/ML models, the layer-common AI/ML model design is a more favorable option for both UEs and gNBs.

  - Apple: Rank common AI model needs further clarification

  - AT&T: For the complexity and storage report for the AI/ML based CSI compression sub use cases with rank >=1,

    - FLOPs

      - Option 2 (layer specific): summarized FLOPs over the layer specific models inferred for each layer

      - Option 3 (layer common): summarized FLOPs for the layer common model inferred for each layer

    - Memory storage/number of parameters

      - Option 1-1 (rank specific): summarized Memory storage/number of parameters over the rank specific models

      - Option 2 (layer specific): summarized Memory storage/number of parameters over the layer specific models

      - Option 3-2 (layer common and rank specific); summarized Memory storage/number of parameters over the rank specific models

  - LG: For AI/ML model setting for rank>1, option 3-1 (layer common and rank common) can be a baseline

Findings

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model

  - Apple: Layer specific AI model can achieve better performance comparing to layer common AI model, with 4 times storage overhead

  - Apple: Rank specific AI model perform depends on loss function. If average SGCS is used in loss function, the model averages out the SGCS of each layer. With rank 4 model, the layer 1 and layer 2 SGCS suffer significant loss comparing to layer specific and layer common.  Higher model complexity and higher storage requirement for rank specific model is observed.

  - MediaTek: Layer-common AI/ML model respectively achieves 1.18% and 1.55% SGCS gain for layer 0 and layer 1 of rank-2 channels compared to layer-specific AI/ML models.

  - ZTE: For rank =2, AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II.

  - ZTE: Case 1(layer common) can achieve better performance than Case 2 (rank specific).

  - InterDigital: For the rank one case, the AI/ML-based CSI compression using eigenvectors of channels as the input outperforms the Rel-16 Type II at similar overhead

  - InterDigital: For the rank one case, the AI/ML-based CSI compression using the eigenvector layer-common model achieves the best SGCS performance relative to the layer-specific and rank-specific models

  - InterDigital: The AI/ML-based CSI compression using the eigenvector layer-common model generalizes well when tested under two layers, and achieves the best SGCS performance relative to the layer-specific and rank-specific models for both rank one and rank two transmission

  - ETRI: Rank-common or layer-common AI/ML models can process inputs with different numbers of ranks using a single model, allowing for a smaller model size

#### Input/output CSI format

- Option 1: Raw channel matrix. NVIDIA, FUTUREWEI InterDigital

- Option 2: Eigenvector. Huawei, Hisilicon, vivo, CATT, Samsung, Nokia, MediaTek, OPPO, Xiaomi, Apple, ZTE, CMCC, FUTUREWEI, NVIDIA, ETRI, Google, Spreadtrum, Fraunhofer Intel InterDigital,

  - Option 2A: Legacy-like PMI (e.g., Type I-like, Type II-like CSI). Ericsson, Intel, vivo

  - Option 2B: Eigenvector of additional past CSI. Huawei, Hisilicon,

- Views/findings

  - Fraunhofer: Eigenvectors of the channel matrix shall be used for AI/ML encoding

  - InterDigital: The AI/ML-based CSI compression using eigenvectors as an input for the layer-common model consistently outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS under the rank one transmission case

  - NVIDIA: Both autoencoders with raw channel matrix as input and autoencoders with eigenvector(s) of raw channel matrix as input have been agreed to be evaluated

#### Model structure

- Fraunhofer: The CSI feedback compression based on Transformers e.g., TransNet, significantly outperforms CNN based architectures e.g., CsiNet and CRNet if NMSE is taken as performance metric

- Fraunhofer: Transformer-based methods are proposed at the UE-side and gNB-side, if the complexity of training and inference can be tolerated

- InterDigital: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of SGCS

- InterDigital: The AI/ML-based CSI compression using transformer model achieves a favorable performance-complexity-memory tradeoff

- InterDigital: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of average throughput

- InterDigital: The SGCS gain of the Transformer-based model over the CNN-based model (CsiNet) is 28% while the throughput gain is only 5%. The significant gap between the throughput and SGCS gain may suggest that using SGCS as an intermediate KPI is not a good indicator of the throughput gain

### 3.1-4: EVM related issues for CSI compression

#### Template for simulation results collection

- Intermediate KPI report for rank>1

  - Qualcomm: When providing results for the SGCS of the benchmark and the gain for SGCS for each layer, compute SGCS and the CSI feedback overhead assuming Max Rank for every UE for both the benchmark and AI/ML-based CSI feedback, i.e., Option 1

    - Option 1: Compute SGCS for each layer assuming all layers up to the maximum rank are compressed using the payload

    - Option 2: Compute SGCS for layer k only from channel samples when the rank is at least k

- CSI feedback payload

  - ZTE: It is necessary to add a note for the template: the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

  - ZTE: The X, Y and Z values for CSI feedback payload per layer are determined as:

    - For a max rank value of 1/2

      - X is <=80bits

      - Y is 100bits-140bits

      - Z is >=230bits

    - For a max rank value of 3/4

      - X is <=200/vbits

      - Y is 240/vbits-320/vbits

      - Z is >=500/vbits

  - Qualcomm: Using the maximum number of non-zero coefficients, the total PMI overhead for Release 16 eType II for parameter combination (PC) 6 is 557 bits. For Rank 4, this would imply the per-layer overhead is below 140 bits. Hence it would not be possible to fill the “Z” bin for this case using the current template.

  - Qualcomm: The definition of the bins (i.e., X, Y, Z ranges) for the result templates should consider whether a particular overhead is possible for the benchmark case for a given Max Rank.

    - Hence, the bin-identifiers (X, Y and Z) should be specified for each Max Rank value separately.

    - Alternatively, replace X/Y/Z with the overhead for eType II parameter combinations. For each Max Rank value, the overhead for Release 16 eType2 Benchmark could be computed for PC 1 through 6.

- CSI feedback overhead

  - Huawei, Hisilicon: For the template of AI/ML-based CSI compression of 1-on-1 joint training, the CSI feedback overhead range of eventual KPI is suggested as:

    - Low overhead: <=β* 80 bits.

    - Medium overhead: β* (100bits – 140 bits).

    - High overhead: >=β* 230 bits.

    - Note: β=1 for max rank = 1, andβ=1.4 for max rank = 2/3/4.

  - CATT: For AI/ML based CSI feedback, following current MIMO mechanism, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2.

  - CATT: For the template for simulation results collection for AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification, the values of CSI feedback overhead for the mean UPT and 5% UPT can be values of total payload size of all ranks

    - Imitating the per layer payload of X, Y, Z for intermediate KPI calculation, a total payload size of A, B and C for eventual KPI can be defined and reported.

    - As additional information, the payload size for each rank can be reported, and the payload size for each layer of each rank can be optionally reported.

  - ZTE: It is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows:

    - Value range#1: CSI feedback overhead <= CSI feedback payload X0*Max rank value;

    - Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead <= CSI feedback payload Y0*Max rank value;

    - Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead <= CSI feedback payload Z0*Max rank value;

    - where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results.

- CSI overhead reduction

  - Huawei, Hisilicon: For the template of AI/ML-based CSI compression of 1-on-1 joint training, 3 values of CSI overhead reduction can be reported corresponding to 3 CSI feedback overhead ranges to be determined in the template: “Low overhead”, “Medium overhead”, and “High overhead”, respectively.

    - For a given CSI feedback overhead range, it is calculated as the gap, between the CSI feedback overhead of the benchmark which should within the given CSI feedback overhead range, and the CSI feedback overhead of the AI/ML solution corresponding to the same eventual KPI.

      - E.g., for a given CSI feedback overhead range, a horizontal line is drawn on the figure which will have two cross points with two curves of benchmark and AI/ML, respectively, from which the CSI overhead reduction is derived. The cross point with the curve of benchmark should fall into the given CSI feedback overhead range.

  - Huawei, Hisilicon: For achieving the same throughput, AI/ML-based CSI compression with spatial-frequency domain CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

    - 42% in high overhead range and 46% in medium overhead range for rank = 1.

    - 48% in high overhead range and 43% in medium overhead range for rank = 2.

  - Huawei, Hisilicon: For achieving the same throughput, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

    - 73% in high overhead range for rank = 1.

    - 70% in high overhead range for rank = 2.

  - Qualcomm: For each bin that identifies the benchmark CSI feedback overhead, companies should report the reduction in CSI feedback overhead from the usage of AI/ML-based CSI feedback over the benchmark CSI feedback method

  - CATT: For the template for simulation results collection for AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification, the values of CSI overhead reduction can be provided for given SGCS value(s), with each SGCS value belongs to a given range

  - ZTE: The CSI overhead reduction and UPT gain are calculated based on:

    - Method 2: For a given overhead for benchmark scheme, calculate the overhead reduction between the benchmark scheme and AI scheme in which the AI scheme has the same throughput value as the benchmark scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead.

  - ZTE: The results of CSI overhead reduction should be captured in the template for different CSI feedback payload values X/Y/Z, respectively

- RI/CQI calculation method

Views

  - Apple: When e-type II codebook is used, two options for RI determination:

    - Option 1: Sequential approach. RI is determined based on ideal eigen-vector. PMI is searched based on RI.

    - Option 2: Joint approach. RI is determined based on the best PMI for each RI hypothesis.

  - Apple: When AI based CSI compression is used, two options for RI determination:

    - Option 1: RI is determined based on ideal eigen-vector. RI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation.

    - Option 2: RI is calculated based on the output of CSI reconstruction part from the realistic channel estimation. RI is determined based on the best PMI for each RI hypothesis.

  - Apple: In result reporting table, add a row to describe the RI determination method for AI and e-type II separately

  - Apple: In result reporting table, add a row to describe the CQI determination method for AI and e-tpye II separately

  - Google: for throughput evaluation, the rank adaptation and CQI measurement scheme should be clarified:

    - Scheme 1: RI/CQI is always measured based on the ideal precoders

    - Scheme 2: RI/CQI is measured based on the decompressed precoders

    - Scheme 3: RI is measured based on the ideal precoders and CQI is measured based on the decompressed precoders

    - Scheme 4: RI is measured based on the decompressed precoders and CQI is measured based on the ideal precoders

  - Google: The RI/CQI measurement scheme should be clarified for the system performance evaluation

Findings

  - Apple: System level performance gain depends on RI selection algorithm. With layer common RI specific AI method, model selection per RI also play important role is system performance

  - Apple: For e-type II parameter set Config 1

    - When RI selection for e-type II and AI based method are based best PMI for each RI hypothesis (joint RI/PMI search, with UE knows the AI decoder),

      - 5.8% cell edge gain and 3% cell average gain is observed at 74% RU.

      - 5% cell edge gain and 3% cell average gain is observed at 50% RU

      - 8% cell edge gain and 2.5% cell average gain is observed at 13% RU

    - When e-type II RI selection is based on selected codebook, and AI RI selection is based on ideal eigen-vector, performance loss is observed.

#### High resolution ground-truth CSI labels

Solutions/views Huawei, Hisilicon, vivo, Ericsson, Qualcomm, ZTE, Apple, Fujitsu, Lenovo, MediaTek, Intel

- Huawei, Hisilicon: evaluation results for the two quantization methods (scalar quantization and quantization by Rel-16 TypeII-like codebook generation method with new parameters) for the ground-truth CSI are provided

  - Float32/Float16/8 bits scalar quantization

  - Rel-16 TypeII CB with new parameters: L=10, p=0.9, beta=0.31, amplitude: 4 bits, phase: 6 bits

- Ericsson: we present results showing the accuracy (as measured in SGCS and RAR respectively) for different choices of L, M and subband size. L=4/6/8, M=5/10/15. The test is with both with β=0.5 and includes both unquantized coefficients (32 bit per real part and imaginary part for W2 entries, respectively) and with 7-bit quantization.

- vivo: Rel-16 TypeII CB with new parameters: L=12, M=6, beta=1.0. Rel-16 TypeII CB with legacy parameters: L=4, M=4, beta=0.75

- Qualcomm: For training data collection, specifying new/larger values of R16 Type II parameters to achieve higher resolution of the ground truth CSI needs clear justification

- ZTE:

  - eType-II PC9: L = 8, pv=0.75, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

  - eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

- Apple: Adopt 8-bit scaler quantization for ground true data quantization

- Fujitsu:

  - Parameter Set #1: L = 6, Mv=7, pv=0.5, β=0.5, N3=13, R=1, Reference amplitude = 4 bits, Differential amplitude = 4 bits, Phase = 4 bits

  - Parameter Set #2: L = 12, Mv=13, pv=0.95, β=0.5, N3=13, R=1, Reference amplitude = 4 bits, Differential amplitude = 4 bits, Phase = 4 bits

- Fujitsu: High-resolution codebook quantization of the right singular vectors of the spatial-frequency-domain channel matrix, e.g., Rel-16 type II-like method with new parameter values, can be used in the dataset construction for finetuning

- Fujitsu: In order to achieve a low-overhead dataset transferring in over-the-air-training/monitoring , the codebook-based quantization approach should be further studied.

- MediaTek: AI/ML model is trained with a dataset which consists of both ideal (float 32) and quantized CSI samples (ParComb 8 of eType II) while the total number of samples is kept fixed as 600k. For example, if quantized CSI ratio in dataset is 90%, it means 540k quantized CSI samples and 60k ideal CSI samples exist in the training dataset

- MediaTek: Need for resolutions beyond what is currently being offered by eType II should be justified by evaluation.

- MediaTek: Study and compare the following options for reducing overhead of data collection

  - Quantization with eType II-like framework with higher resolutions

  - Incorporation of ideal CSI samples for possible finetuning at the data collector side

- Intel: AI-ML models were trained with dataset quantized by

  - Float32 floating point value representation (baseline), 26624 bits

  - Fixed-point scalar quantization with 8 bits per scalar value, 6656 bits

  - eType II PMI with L = 16, pv = 1, beta = 1, 4 bits amplitude and phase, 3329 bits

  - eType II PMI with L = 16, pv = 1/2, beta = 1, 4 bits amplitude and phase, 1803 bits

  - eType II PMI with L = 16, pv = 1/2, beta = 1/2, 4 bits amplitude and phase, 1131 bits

  - eType II PMI with L = 8, pv = 1/2, beta = 1/2, 4 bits amplitude and phase, 588 bit

Findings

- Huawei, Hisilicon: For AI/ML-based CSI compression, compared with the training dataset quantized by Float32 format, other high resolution ground-truth CSI quantization methods with lower overhead show minor SGCS loss with remarkable overhead reduction,

  - Training dataset quantized by 8 bits scalar quantization has <0.9% SGCS loss but reduces 75% overhead of training dataset delivery.

  - Training dataset quantized by Rel-16 TypeII CB with new parameters has <0.7% SGCS loss but reduces 96% overhead of training dataset delivery.

- Huawei, Hisilicon: For AI/ML-based CSI compression, compared with the training dataset with Float32 format, training dataset quantized by a legacy quantization resolution of PC8 of Rel-16 Type II CB causes significant SGCS loss of around 3% when the CSI payload size is medium and large

- Ericsson: For higher layers (layer 3 and 4), the true Tx-eigenvector changes quickly over frequency, such that the eigenvectors extracted with 4 RBs/subband has lost 15-20% in SGCS. However, in terms of RAR the loss is only about 2-3%

- Ericsson: The legacy CSI reporting was shown by SLS to reach 70% UTP while increasing L and M can reach 80%. With non-quantized eType-II coefficients, 87% can be reached at a high overhead cost.

- Apple: For ground true data quantization, 8 bit quantization achieve the best tradeoff between overhead, performance and complexity

- vivo: High resolution R16-eType II codebook with large L, M, beta (for example, L=12, M=6, beta = 1.0) performs well for ground-truth CSI quantization

- Qualcomm: The SGCS achieved with a training dataset quantized using R16 Type II PC 8 is very close to the SGCS achieved using the ideal (floating point) training dataset for the rank 1 case with dense urban scenario.

- ZTE: The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead

- ZTE: New parameters combinations for enhanced R16 Type II method should be supported to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection.

- Fujitsu: For finetuning, an excellent performance can be achieved by the dataset composed by the high-resolution codebook quantization, i.e., Rel-16 type II-like method with new parameter values, of the right singular vectors of the spatial-frequency-domain channel matrix

- Fujitsu: The performance of finetuning is almost the same from using right singular vectors of channel matrices and their high-resolution codebook quantization, e.g., Rel-16 type II-like method with new parameter values

- Fujitsu: It is observed that there is a significant overhead reduction of transferring a codebook-based dataset than a dataset composed by channel vectors of floating-point numbers for separate training. So, it is worth to study codebook-based quantization method in order to achieve a low-overhead dataset transferring in separate training

- MediaTek: It is feasible to quantize entire dataset for training with negligible perf. loss. We observed 10x reduction in dataset size with only 3.1% SGCS performance loss

- MediaTek: Incorporation of few ideal samples into quantized dataset can mitigate perf loss from quantization. Incorporation of 2% ideal CSI samples in the dataset can compensate 38.7% of SGCS loss caused by quantization

- Intel: At least 8 times reduction of the number of bits required for ground truth CSI quantization for data collection comparing to float32 scalar quantization format can be done without performance loss

Preliminary trends observed by more than one company

- For AI/ML-based CSI compression, compared with the training dataset quantized by Float32 format, the high resolution quantization methods of R16 Type II-like method with new parameters show minor SGCS loss with remarkable overhead reduction. Huawei, Hisilicon, vivo, Ericsson, Qualcomm, ZTE, Apple, Fujitsu, Intel

  - Compared to the training dataset based on legacy quantization resolution of Rel-16 Type II CB (e.g., PC8), R16 Type II-like method with new parameters and higher resolution may/may not achieve better performance. Huawei, Hisilicon, vivo, Ericsson, Qualcomm, ZTE

#### CQI calculation

- ZTE: Companies should disclose its CQI calculation assumptions when submitting the simulation results.

- ZTE: Further study and evaluate different CQI calculation options and draw necessary conclusions (including the performance and feasibility) in Rel-18.

- ZTE: Option 2a-1 is not practical in reality, but it can provide a performance upper-bound to compare different options

- ZTE: With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme

- ZTE: Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

- Lenovo: Assuming two-sided AI models for CSI compression under training collaboration Type 3, further enhancements are needed to ensure precise CQI characterization in the presence of mismatch between the nominal decoder (at UE side) and the actual decoder (at network side).

- Lenovo: Consider Option 1b for CQI reporting, where the UE appends side information to the CQI calculated based on the CSI reconstruction output, where the side information helps quantify the encoder/decoder mismatch to better estimate the actual CQI value

- Lenovo: Considering Option 2a for CQI reporting, study the gain and the associated cost of using “updated nominal decoder” instead of the “nominal decoder”

- Xiaomi: Companies to report the specific CQI determination method(s) for AI/ML in the template table, e.g.,

  - CQI is calculated based on the output CSI from the realistic channel estimation

  - CQI is calculated based on the target CSI from the realistic channel estimation

  - CQI is calculated based on traditional codebook

  - CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

  - CQI is calculated based on using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

- Fraunhofer: For CQI calculation in Type 3 network-first training, side-information and a set of parameters of the decoder e.g., the accuracy or loss of the reconstruction should be revealed to the UE

- LG: For CQI calculation of CSI compression, prioritize following options

  - CQI is calculated based on target CSI with realistic channel measurement

  - CQI is calculated based on legacy codebook

#### Others

CSI dataset separation in evaluations

- ZTE: For CSI dataset construction, dataset should be generated in diverse drops rather than in divers TTIs at least for calibration purpose (e.g., only one TTI can be used for generating training dataset in a single drop).

### 3.1-5: Generalization for CSI compression

#### Generalization over scenarios (dimensions of the input/output of the AI/ML model are unchanged)

- General views/findings

  - vivo For a generic model (non-optimized for a specific area/cell), AI models perform well in generalization of carrier frequency, channel scenario, indoor/outdoor ratio.

  - vivo For a generic model (non-optimized for a specific area/cell), AI models perform bad in antenna spacing and antenna virtualization, which can be further studied.

  - FUTUREWEI: For CSI feedback compression sub use case, if the goal is to use the AI/ML model to perform inference in multiple target scenarios, then using a mixed training dataset that is constructed from multiple scenarios, i.e., from those target scenarios, can be considered to improve performance

  - Mavenir: Mixed dataset should be considered as a effective way to increase AI model generation

  - Mavenir: AI model performance should be evaluated on datasets with different scenario and configuration

- Various deployment scenarios (UMa/UMi/InH): Huawei, HiSilicon, vivo, Xiaomi, ZTE, OPPO, CATT, CMCC, Intel, NVIDIA, Spreadtrum, FUTUREWEI, InterDigital, Comba, Mavenir

Findings on generalization verification

  - Huawei, HiSilicon: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa/UMi) for inference (i.e., generalization Case 2), its performance may be degraded compared to inference under Scenario#A (i.e., generalization Case 1), but mixing the dataset over the three scenarios for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization

  - vivo: For a generic model (non-optimized for a specific area/cell) AI model can generalize across different scenarios with a mixed dataset. A reasonable mixing ratio can provide better performance for each scenario

  - ZTE: Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various scenarios included in the mixed deployment scenarios.

  - Xiaomi: AI/ML model shows good generalization performance in different scenarios

  - OPPO: For different scenarios, the SGCS degradation is slight (about 1%~3%) when training set and testing set are mismatching

  - OPPO: For different scenarios, training on mixing dataset can improve the generalization performance of AI/ML model

  - CATT: For the generalization of AI/ML based CSI feedback, the following is observed

    - For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small

    - For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH

    - For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance gap

  - Intel: If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is marginal comparing to training and testing on aligned dataset (Case 1)

  - CMCC: The AI model trained with mixed dataset across various scenarios might have some performance loss comparing with dedicated model.

  - NVIDIA: Evaluation results show that it is much easier for the autoencoders to compress CSI in CDL-C than in dense urban scenario, as the link level channel model CDL-C has fixed angle values and represents only a single channel realization while the system level channel in the dense urban scenario is much more sophisticated

  - NVIDIA: Evaluation results show the autoencoders trained in the sophisticated dense urban scenario perform well in CDL-C, illustrating the generalization capability of the AI/ML models

  - Spreadtrum: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook

  - Samsung: The following observation were made for generalization performance across deployment scenario

    - AI model generalizes well from UMa to UMi and vice versa

    - It is relatively difficult to generalize from UMa or UMi to InH

  - FUTUREWEI: For generalization Case 1, when the AI/ML model is trained using dataset generated from one scenario then performs inference on a dataset generated from the same scenario, performance (as measured by SGCS) is very decent, at least for UMa to UMa and UMi to UMi cases

  - FUTUREWEI: For generalization Case 2, when the AI/ML model is trained using dataset generated from one scenario then performs inference on a dataset generated from another scenario, significant performance degradation is observed compared to the baseline performance (generalization Case 1), at least for UMa to UMi and UMi to UMa cases

  - FUTUREWEI: For generalization Case 3, when the AI/ML model is trained using training dataset constructed by mixing datasets from multiple scenarios, then performs inference on one of the scenarios, the SGCS performance is comparable to the baselines (generalization Case 1), at least between UMa and UMi scenarios.

  - InterDigital: The AI/ML model trained on mixed datasets from UMa and UMi channel samples generalizes well when tested under each individual dataset. This suggests that using one model trained under mixed datasets can provide both performance gains and memory savings relative to using a separate model for each scenario

  - Comba: There are significant differences in the generalization performance of models for different scenarios

  - Mavenir: For different scenarios, the SGCS degradation is about 1%~2% when training set and testing set are mismatching.

  - Mavenir: Training on mixing dataset with CDL_A and CDL_B can improve the generalization performance (about 1.4%/0.6%) of AI/ML model for CDL_A and CDL_B.

Preliminary trends observed by more than one company

  - AI model generalizes well/not well from UMa to UMi and vice versa. Huawei, HiSilicon, vivo Samsung, ZTE, CATT, Xiaomi, Intel, FUTUREWEI

    - vivo: For a generic model (non-optimized for a specific area/cell), AI model performance does not degrade when generalized from UMi to UMa.

    - Samsung: AI model generalizes well from UMa to UMi and vice versa.

    - ZTE: The AI model trained in the scenario of UMa/UMi shows good generalization performance to InH dataset, while the AI model trained in the scenario of InH shows generalization degradation to UMa/UMi dataset.

    - CATT: For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small. For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH.

    - Intel: there is a very small performance loss for autoencoder with misaligned datasets for training and inference (Case 2) comparing to autoencoder with aligned datasets (Case 1)

  - It is relatively difficult to generalize from Uma/UMi to InH. vivo, Samsung

  - It is relatively difficult to generalize from InH to Uma/UMi. Huawei, HiSilicon, ZTE, CATT

  - Mixing the dataset between InH and Uma/UMi for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization under InH or UMa/UMi Huawei, HiSilicon, vivo, CATT, FUTUREWEI, InterDigital

- Various outdoor/indoor UE distributions for UMa/UMi: Huawei, HiSilicon, vivo, Qualcomm, Spreadtrum

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over indoor/outdoor UE distribution ratios,

    - AI/ML model trained by any indoor/outdoor UE distribution ratio shows similar performance on the outdoor testing dataset.

    - With the decrease of the indoor channel ratio for the training dataset, the trained AI/ML model brings decreased performance on the indoor testing dataset.

  - Qualcomm: Training on a dataset constructed by mixing the datasets of multiple scenarios enables the same ML model to perform well during inference in each of the scenarios

  - vivo: For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability.

  - vivo: The performance of AI model depends on the deployment environment

  - Spreadtrum: For the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions ratio, AI/ML model trained by 8:2 and 2:8 indoor/outdoor UE distribution ratio shows similar performance

Preliminary trends observed by more than one company

  - For generalization Case 2, AI model generalizes well for training with indoor (or higher indoor ratio) and inference with outdoor (or higher outdoor ratio), while a poor generalization performance is observed for the other way around. Huawei, HiSilicon, vivo, Qualcomm

- Various carrier frequency: vivo,

Findings on generalization verification

  - vivo: AI model performance does not degrade when a generic model (non-optimized for a specific area/cell) trained for a frequency is applied to another frequency

Preliminary trends observed by more than one company

  - AI model trained under one carrier frequency generalizes well over other carrier frequencies (e.g., from the set of 2GHz, 3.5GHz, 4GHz, 5.5GHz) vivo,

- Various Tx/Rx antenna spacing: vivo,

Findings on generalization verification

  - vivo: When compressing precoding matrix, CSI compression models generalize well over different RX antenna spacing (generalization case2).

  - vivo: For a generic model (non-optimized for a specific area/cell), there is obvious performance loss for antenna spacing mismatch of training data.

Effect of pre-processing vivo

  - Issue: Performance loss for antenna spacing mismatch of training data

  - Solution: Pre-processing to achieve a small AI models with spatial domain and frequency domain compression

  - Findings: For a generic model (non-optimized for a specific area/cell), the influence of mismatch of training data may be reduced by pre-processing

- Various antenna virtualization (TxRU mapping): Huawei, HiSilicon, vivo,

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over various TxRU mapping methods including (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8), the AI/ML model trained by the mixed datasets show moderate performance on each of the TxRU mapping methods.

  - vivo: For a generic model (non-optimized for a specific area/cell), SGCS performance of AI model may degrade slightly from 128 antennas with virtualization to 32 antennas without virtualization. While the SE performance may degrade heavily due to the less antennas

  - vivo: For a generic model (non-optimized for a specific area/cell), in the case of 32 antennas, AI model trained with 32 antennas may have similar SE performance compared with AI model trained with 128 antennas and settled in the case of 128 antennas, which is needed to be further studied.

#### Generalization over scenarios – fine-tuning

Views on fine-tuning

- OPPO: Regarding the EVM for fine-tuning, the following factors should be considered at least

  - Size of fine-tuning dataset

  - Sampling distribution of fine-tuning dataset

  - Diversity between fine-tuning dataset and original dataset

  - Fine-tuning delay

  - Performance gain

  - Other aspects related to fine-tuning

- OPPO: For the baseline of fine-tuning evaluation, direct training on fine-tuning dataset from random initialization and inference on the testing dataset should be considered as a benchmark

Findings on fine-tuning

- Huawei, HiSilicon: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa) for inference, its performance can be improved by applying fine-tuning using a relatively small dataset from Scenario#B.

- Xiaomi: The fine-tuning procedure can be used to improve the performance

- ZTE: The AI model trained in the scenario of InH and fine-tuned with the dataset from a scenario of UMa/UMi shows 1%-5% SGCS performance gain over the AI model trained only in the scenario of InH under the testing dataset from the scenario of UMa/UMi.

- ZTE: With the increased samples of fine-tuning dataset, the AI model shows better generalization performance

- OPPO: For AI model trained on UMa and fine-tuned on UMi, the fine-tuning gain on SGCS performance is small with insufficient UMi fine-tuning dataset

- OPPO: For AI model trained on UMa and fine-tuned on CDL-C, the fine-tuning gain on SGCS performance is about 3.6% for larger fine-tuning dataset, which vanishes when fine-tuning dataset is small

- OPPO: For AI model trained on UMa and fine-tuned on UMi and CDL-C, the fine-tuning with insufficient datasets cannot achieve the equivalent SGCS performance as upper-bound

  - Note: upper-bound indicates using sufficient training set, which is from the same scenario as testing set.

- OPPO: Fine-tuning converges faster than baseline for both CDL-C#2k and CDL-C#20k datasets. Compared with baseline, fine-tuning achieves about 27.9% SGCS performance gain on smaller dataset CDL-C#2k and similar SGCS performance gain on larger dataset CDL-C#20k.

  - Note: baseline indicates using directly trained on CDL-C#2k and CDL-C#20k with random AI model initialization as starting point.

- Fujitsu: There is a huge penalty of the performance if the AI/ML-based CSI generation part and the AI/ML-based CSI reconstruction part are mismatched in the sense that they are trained using the datasets from different scenarios

- Fujitsu: The performance of the finetuning is very similar to that of joint training in terms of the SGCS

#### Generalization over Configurations (dimensions of the input/output of the AI/ML model are different, i.e., scalability needed)

- General views

  - Solutions to achieve scalability

    - vivo: Study the following three methods for generalization of input dimension

      - Option 1: use large dimension AI/ML model in small dimension cases: zero-padding

      - Option 2: use small dimension AI/ML model in large dimension cases: grouping

      - Option 3: use pre-processing to fix the input dimension: angle-delay domain compression

    - OPPO: For scalability evaluation, zero-padding, clipping and truncation can be considered for pre-processing and post-processing

- Various bandwidths/frequency granularities: Ericsson, vivo, Qualcomm, OPPO, Xiaomi, ZTE, CMCC, ETRI, Fujitsu Nokia, InterDigital

Input/output dimension cases:

  - Case 1: One scalable CSI generation part with N>1 separate CSI reconstruction parts each with fixed dimensions OPPO

  - Case 2: M>1 separate CSI generation parts each with fixed dimensions to one scalable CSI reconstruction part OPPO

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - The experiments are conducted over 52 and 48 PRB datasets Nokia, CMCC

Solutions:

  - Solution 1: Pre-processing to angle-delay domain.

    - vivo: For the preprocessing AI/ML model, angle-delay compression is used for preprocessing and 4 top strong beams on each polarization and 4 top strong paths are selected, which means the input dimension is 8 * 4 complex coefficients

  - Solution 2: Training with mixed variable subband configurations. E.g., training the AI model using random subband patterns in addition to the full subband case. Qualcomm

  - Solution 3: Adapt the subband size according to BW size (to keep the input dimension unchanged). E.g., 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz. ZTE, Ericsson

    - Ericsson: The evaluation agreement is to double the number of RBs per subband for the ground-truth calculation when doubling the considered bandwidth from 10 to 20 MHz

  - Solution 4: Zero padding. vivo Xiaomi, OPPO, CMCC ETRI, Fujitsu, InterDigital

    - vivo: For the normal AI model, the input is 13 subbands and 32 ports and zero-padding is used for less input dimension.

    - OPPO: the AI/ML model trained on dataset from Configuration#A with 13 sub-band can be inferenced/tested on dataset from Configuration#B with 8 sub-band and zero-padding on the sub-band domain

    - CMCC: we train AI model with the samples composed of the eigenvectors of 12 subbands and apply this AI model to test the samples composed of the eigenvectors of 8 subbands. To achieve better generalization performance, we will pre-process the samples composed of the eigenvectors of 8 subbands before inputting these samples into generation part, i.e., we will perform padding zero at the end of each sample

    - Fujitsu: the input of CSI generation part will be filled with zero value along the sub-band dimension and/or the antenna port dimension if input dimensions are less than pre-configured values

  - Solution 5: PCA/restoration. ETRI

    - For PCA based AI model for CSI compression, restoration is performed at the last stage of the Decoder. The restoration is to remove noises of the reconstructed channel data, where the noises are induced during downsampling, dimension reduction, and quantization…Transformer network is one of Recurrent Neural Network (RNN) and RNNs can process variable length of data sequences. The restoration NN in Decoder gets (reconstructed) eigenvectors of subbands as an input sequence and puts restored eigenvectors of subbands as an output sequence... The PCA based AI/ML model in the section 2.2.3 supports various sizes of input and output because the restoration NN is based on Transformer network which can get and generate variable lengths of sequences.

  - Solution 6: Adaptation layer CMCC, NTT DOCOMO, Nokia

    - CMCC: a pair of down-sampling (DS-x block) and up-sampling (US-x block) is corresponding to one number of subbands while the EN block and the DE block are shared among all the configured subband numbers…We train the AI model with a mixed data, which compose of the eigenvectors of 10 subbands, 6 subbands and 4 subbands

    - NTT DOCOMO: the adaptation layer is introduced before the encoder and after the decoder

Findings/views on generalization verification

  - vivo: For case 2, zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization

  - vivo: For case 2, pre-processing performs well for both subband number generalization and port number generalization

  - Qualcomm: Training with mixed variable subband configurations achieve robust performance across all possible subband configurations including arbitrary number of subbands and arbitrary subband patterns

  - Qualcomm: Training with mixed variable subband configurations outperforms specific training with specific subband configuration

  - Ericsson:	The change in the intermediate KPI gains over legacy baseline are within 1%-unit, hence the adjustment of subband size in the pre-processing when bandwidth is scaled allows for bandwidth generalization of the AI/ML-model

  - Ericsson:	For generalization of bandwidth, the results need to be compared with legacy of the target bandwidth, i.e. the same bandwidth as the test case

  - Nokia: The universal model is capable of compressing and decompressing the CSI matrices of different bandwidth configurations without a significant loss of CSI reconstruction accuracy

  - Nokia: Continue to study the universal model approach to scalability for support of wider variation in bandwidth as well as for other scalability parameters and combinations of parameters

  - ZTE: AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size

  - Xiaomi: AI/ML model shows good generalization performance in different sizes of subband and subband number

  - CMCC: The AI model trained under one number of subbands might have some performance loss when performing interference on CSI compression of a different number of subbands

  - CMCC: For scalability over different number of subbands, the AI model with adaptation layers only suffers a little SGCS performance degradation compared with subband number-specific model

  - CMCC: For scalability over different numbers of subbands, the LSTM based AI model will suffer great SGCS performance degradation compared with subband number-specific model; however, when using mixed data to train a unified LSTM based AI model, it could achieve similar SGCS performance as subband number-specific model

  - Fujitsu: For generalization/scalability of AI/ML model over the different number of sub-bands, the zero-padding method can achieve good performances in terms of the SGCS when the AI/ML model is trained with mixed data

  - InterDigital: The AI/ML model trained on 10 MHz bandwidth generalizes well when tested under 5 MHz bandwidth. Further, we observed that increasing the number of zero padded input results in more performance degradation relative to the baseline

- Various Tx/Rx antenna port layouts: Huawei, HiSilicon, vivo, Qualcomm, ZTE, Apple, CATT, OPPO, NTT DOCOMO, Fujitsu,

Input/output dimension cases:

  - Case 1: One scalable CSI generation part with N>1 separate CSI reconstruction parts each with fixed dimensions

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

      - Findings: Using common encoder with gNB-specific decoder achieves higher SGCS than using common encoder with common decoder

  - Case 2: M>1 separate CSI generation parts each with fixed dimensions to one scalable CSI reconstruction part

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

      - Findings: Using common decoder with UE-specific encoder achieves higher SGCS than using common decoder with common encoder

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - X1/X2 =16/32 ports: Huawei, HiSilicon vivo CATT ZTE

    - 2x8, 4x4 and 2x4: Qualcomm

    - [8 2 2] and [4 4 2]: Apple

    - X1/X2/X3 =16 ports & 6 subbands/ 16 ports & 12 subbands / 32 ports & 12 subbands NTT DOCOMO

Solutions

  - Solution 1A: training a common AI model using mixed data set of 2x8, 4x4 and 2x4 antenna configurations. Qualcomm

  - Solution 1B: Case1: training on [8,2,2], testing on [8,2,2]; Case2: training on [8,2,2], testing on [4,4,2]; Case3: training on mixed dataset of [8,2,2] and [4,4,2], testing on [8,2,2]/[4,4,2]. Apple

  - Solution 2: Zero padding to 32 ports (and dataset mixing). Huawei, HiSilicon, ZTE, OPPO, Fujitsu

  - Solution 3: Adaptation layer CATT, Nokia

  - Solution 4: Pre-processing to angle-delay domain. vivo

Findings on generalization verification

  - Huawei, HiSilicon: For scalability of AI/ML-based CSI compression over various antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 2 different antenna port numbers is less than 0.9%.

    - Pre/post processing of padding/truncation is used to achieve the scalability.

  - vivo: Zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization

  - Qualcomm: Training with mixed antenna configuration achieves robust performance across all antenna configurations in the training

  - Qualcomm: Training with mixed antenna configuration outperforms specific training with specific antenna configuration

  - ZTE: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers

  - ZTE: AI model trained with the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

  - Apple: For generalization study case 2, when the autoencoder is trained in UMa with [8 2 2] antenna port layout, and test with [4 4 2] antenna port layout, large performance loss is observed

  - Apple: For generalization study case 3, when the autoencoder is trained in mixed dataset with [8 2 2] and [4 4 2] antenna port layout, and test with [4 4 2] antenna port layout, similar performance is observed as case 1.

  - OPPO: The scalability performance of AI/ML model for various number of antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - Fujitsu: For generalization/scalability of AI/ML model over the different number of antenna ports, the zero-padding method can achieve good performances in terms of the SGCS when the AI/ML model is trained with mixed data

- Various CSI feedback payloads: Huawei, HiSilicon, vivo, Qualcomm, Xiaomi, CATT, OPPO, CMCC, ETRI, NTT DOCOMO, Nokia, MediaTek, Indian Institute of Tech

Input/output dimension cases:

  - Case 1: One scalable CSI generation part with N>1 separate CSI reconstruction parts each with fixed dimensions

    - vivo: Y1/Y2/Y3 =176/199/223 bits or 132/176/199 bits or 132/176/223 bits

    - Xiaomi: Y1/Y2/Y3 =120/180/240 bits

    - CMCC: Y1/Y2=32/48 bits; Y1/Y2/Y3=32/48/120 bits

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

  - Case 2: M>1 separate CSI generation parts each with fixed dimensions to one scalable CSI reconstruction part

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

    - Indian Institute of Tech: Y1/Y2=512/256 bits

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - Huawei, HiSilicon: Y1/Y2/Y3/Y4=60/120/168/240 bits.

    - CATT: payload = 20, 40 … 320 bits

    - NTT DOCOMO: Y1/Y2/Y3=44/104/304 bits

    - Nokia: CR=1/8, 1/16, 1/32, and 1/64

    - MediaTek: Y1/Y2/Y3=52/104/208 bits

Solutions:

  - Solution 1: Payload truncation. vivo, Xiaomi OPPO CMCC

    - vivo: the output of the encoder is cut out from the beginning to the specific payload length.

    - Xiaomi: different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The loss function is the average results of different decoder parts

    - OPPO: When the AI/ML model trained on Configuration#A is adopted on Configuration#B, the first LB bits can be reserved, and the latter LA-LB bits can be truncated during the interface feedback. Then, the truncated  LA-LB bits can be regarded as default 0 or 1 for the decoder input.

    - CMCC: The dimensions of the output of generation part is designed based on the maximum feedback bits, and before outputting from the generation part, some extra bits will be dropped

  - Solution 2: Variable subband configurations with variable payload Qualcomm

    - Option 1-two payload configurations are considered (i.e., encoder output dimension = 32 and 64) and are trained at the same time;

    - Option 2-training using contiguous patterns with random number of subbands, and the number of subbands are randomly generated;

    - Option 3-Similar to Option 2 but arbitrary subband pattern is considered in the training

  - Solution 3: Adaptation layer Huawei, HiSilicon, CATT, NTT DOCOMO, Nokia, MediaTek

    - NTT DOCOMO three pairs of fully connected layers with different units are used

  - Solution 4: Adjusting quantization bit lengths ETRI

  - ETRI: The AE based AI Model for CSI compression can generate various CSI feedback payload sizes by controlling quantization bit lengths.

      - For example, the AE based AI Model can generates 86 bits of CSI feedback payload by quantizing 43 output nodes of Encoder by 2 bits each output. By changing quantization bit lengths of each output node of Encoder, the AI Model can generate different payload sizes

  - Solution 5: PCA/restoration. ETRI

    - ETRI: The PCA based AI Model can further change payload sizes by controlling number of reduced dimensions to represent the input data (e.g., eigenvector) and number of bits to quantize each dimension. For example, an input eigenvector can be reduced to 4 dimensions and quantized using 4, 4, 2 and 2 bits for each dimensions, respectively, then 12 bits of CSI payload can be generated

Views from companies

  - vivo: Study CSI payload truncation for the generalization of UCI payload

    - FFS the flexible truncation strategy and training parameters for more different payload.

    - FFS the payload truncation on different layers for rank > 1 cases.

  - Xiaomi: RAN1 study pre-processing mechanisms for the input of decoder to improve the AI model generalization performance on various feedback payloads

  - MediaTek: Discuss model ID assignment and relevant LCM issues for payload-scalable AI/ML models

    - it is not clear whether one or multiple model IDs should be assigned to a generalized model

Findings on generalization/scalability verification

  - Huawei, HiSilicon: For scalability of AI/ML-based CSI compression over various CSI payload sizes, the SGCS margin between the payload-specific models and the unified scalable model supporting 4 different CSI payload sizes is less than 1%.

    - Adaptation layers in the AI/ML model are used to achieve the scalability

  - vivo: Payload truncation, as a starting point, performs well in UCI payload size generalization

  - Qualcomm: Smaller number of subbands can achieve comparable results to the larger number of subbands with half of reporting payload

  - Xiaomi: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads

  - OPPO: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - CATT: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, 3%~11% throughput improvement under the same CSI feedback payload can be achieved by proposed scalable AI/ML model

  - CMCC: The unified AI model of one common encoder and multiple specific decoders performs well across different number of feedback bits.

  - Nokia: The universal model is capable of generating CSI feedback codewords with scalable compression ratios with a tolerable loss in CSI reconstruction accuracy

  - MediaTek: SGCS loss from payload scalability is only -1.6% and -2.5% for two and three rates, respectively. The SGCS loss increases as the difference between payload lengths increases

  - MediaTek: Payload scalability saves 59.4% and 57.0% storage for two and three rates, respectively

- Various ranks/layers: Huawei, HiSilicon, vivo, ZTE, Apple, MediaTek, NTT DOCOMO, ETRI, Intel

Solution:

  - Rank common model (Option 1-2) ETRI

    - ETRI: the rank-common AI/ML model operates on the entire input (input of multiple layers)

  - Layer common model (Option 3). Huawei, HiSilicon, vivo, ZTE, MediaTek ETRI NTT DOCOMO, Intel

Elaboration

    - Option 3-A: A unified AI/ML model is trained with the dataset for layer 1 (and 2) (Case 1/2) only; Apple CATT ZTE

    - Option 3-B: A unified AI/ML model is trained with the mixed dataset for multi-layers (Case 3) Apple CATT ZTE

Views

    - MediaTek: Between all options of AI/ML models settings for rank>1, down select layer-common setting.

    - NTT DOCOMO: the layer-common model trained with the rank 2 data set is applied to the inference of rank 1 channel dataset

Views from companies

  - vivo: For rank > 1 cases, study 	Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference).

    - FFS how to choose the layers for training data set

    - FFS how to deal with specific payload for each layer

Findings on generalization verification

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model.

  - vivo: Layer common model can achieve better SGCS with the same dataset

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at layer level causes 1.29% GCS degradation

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at rank level causes 1.13% GCS degradation

  - MediaTek: On average, a unified AI/ML model (generalization case 3) not only does not degrade the feedback accuracy, but it also achieves 0.46% higher GCS accuracy compared to the dedicated AI/ML models for both layers

  - MediaTek: On average, a unified AI/ML model (generalization case 3) shows 5.8% higher GCS accuracy for EVs of layer 0 compared to those belonging to layer 1. The similar trend has also been observed among the dedicated AI/ML models

  - ZTE: The AI/ML model shows a good generalization capability across layers in the following cases

    - Trained with data from all layers and tested for different layers

    - Trained with data from the first layer and tested for different layers

    - Trained with data from the first two layers and tested for different layers

  - Intel: Performance is comparable for layer common and layer specific models for both layer 1 and layer 2 evaluations

Preliminary trends observed by more than one company

  - Layer common model achieves good generalization performance. Intel, vivo, MediaTek, ZTE

- Overall proposals on scalability

  - Qualcomm: For the evaluation of generalization of AI model to variable configurations, consider the following in data set generation:

    - For subband generalization, generate N>=1 random patterns (either contiguous or non-contiguous) for each data sample in the training set. The full subband pattern can be used in addition.

    - For antenna configuration generalization, mix data sample generated based on M antenna configuration with equal proportion.

    - Same configuration in the testing set and training set

#### Per-area/localized model

- General views

  - vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

  - vivo: Consider to capture observations from field data test into TR

  - vivo: Further study the model update for per-cell (region) models

  - Qualcomm: Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

- Methodology on Per-area/localized model

  - Model size/structure

    - vivo: From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

  - Overhead of model update

    - vivo: If the model structure of CSI generation part is simple (e.g., one-layer MLP), overhead of the model updating procedure will be very small (probably less than 100kB).

  - Data collection

    - vivo: Further study the data collection for per-cell (region) models.

      - Training per-area models requires to enhance the data collection mechanism by some assistance information. Cell ID/sector ID or some other information that could represent the collecting area should be assigned to the corresponding data during dataset delivery. However, there could be some concerns on user privacy, UE storage, power consumption or overhead.

  - Channel model

    - vivo: Here we consider using data where the channel has spatial consistency characteristics. Each UE generates random variables with spatial consistency based on its own geographic location at the T=0, both the cluster specific random variables and the correlation distance for spatial consistency procedure a follow 38.901.

- Findings on performance

  - vivo: From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance when per cell model is used

  - vivo: Based on initial field test results, per-cell (region) models can provide more than 30%~50% improvement on SCGS of AI models

  - vivo: From initial results for field test, the model developed for Cell 1 shows robust performance for different moving routes.

  - vivo: Field test shows that model developed for Cell 1 does not perform well for Cell 2.

  - vivo: Field test shows that simple and small models work well for all different cases, at least for typical cell coverage.

  - Qualcomm: Localized models, which are developed specifically for use within a local region, provide better performance compared to global models of comparable size trained using data from the entire layout.

  - Qualcomm: Localized models provide similar performance with a smaller model size compared to global models

#### Other views/findings on generalization verification

- Nokia: Study various model architectures for generalization performance, including an assessment of the trade-off between performance and model complexity

- Indian Institute of Tech: Training UE part model and NW part model with datasets based on different scenarios will result in a depletion of model performance

- Indian Institute of Tech: For multi-vendor scenarios with different CSI compression models, sharing parameters at the NW reduces computation time and in turn power consumption. However, it may result in a performance drop of the system

- Indian Institute of Tech: For multi-vendor scenarios with different CSI compression models, sharing trainable parameters at the network side allows for a more efficient implementation of the CSI compression model

- Indian Institute of Tech: A specifier must be sent from all the UEs to the NW informing the type of CSI model used for compression. This will be used by the NW to optimize the number of parameters to be shared

- Indian Institute of Tech: Training UE part model and NW part model with datasets based on different output dimensions will result in a depletion of model performance

- Indian Institute of Tech: For multi-vendor scenarios with different output dimensions, sharing parameters at the NW reduces computation time and in turn power consumption. However, it may result in a performance drop of the system

- Indian Institute of Tech: For multi-vendor scenarios with different output dimensions, sharing trainable parameters at the network side allows for a more efficient implementation of the CSI compression model

- Indian Institute of Tech: A specifier must be sent from all the UEs to the NW informing the output dimension used for compression. This will be used by the NW to optimize the number of parameters to be shared

### 3.1-6: Monitoring

General principle

- Lenovo: To evaluate the performance of a “model”, we should evaluate how much the “output of the model” is inline with the “expected output”. To evaluate the performance of a “model monitoring” scheme, we should evaluate the rate can correctly detect the outputs which are deviating from the “expected output”.

- Lenovo: For evaluation of the performance of a monitoring scheme:

  - Generate dataset of “K” test samples:

  - The samples in this set are in the form of <input, expected output>. The samples should be drawn form:

    - The same scenario/configuration that the model is designed for

    - Also from other scenarios/configurations other than the ones used for training of the model: we need to have such samples to evaluate the performance of monitoring scheme in case the settings of the environment changes.

  - Determine an “Actual label” to each of the samples in the test set representing which of samples lead to a “not acceptable” model output. For example, samples for which the output of the model has low corelation with the expected output are labelled as “True” and the rest are “False”.

  - Use the proposed monitoring scheme to determine an “Estimated label” for each of the samples in the test-set based on the.

  - Compare the set of “Actual labels” and “Estimated labels” and report”: a) the “True positive rate” and “False positive rate” of the detection, b) The overhead and latency associated with the proposed scheme.

- vivo: The accuracy of a particular monitoring method can be evaluated via testing whether this method could lead to a proper model switching. To measure what is a proper model switching, some metrics can be considered as the starting points:

  - Counting the false alarm and misdetection compared with ideal switching

    - Ideal switching refers to the switching decisions guided by accurate performance KPIs of candidate models

    - false alarm refers to the case that ideal switching does not happen, but practical switching happens

    - misdetection refers to the case that switching happens but practical switching does not happen

  - Comparing the SGCS after switching for each monitoring method

- vivo: For the purpose of evaluating performance monitoring methods in CSI compression, the data to model a switching procedure can be collected from moving UEs across multiple scenarios

- ZTE: Intermediate KPI based monitoring can be a starting point for evaluation on model performance monitoring

- ZTE: For model performance monitoring, the following two cases can be considered

  - Case 1: Model performance monitoring at UE side

  - Case 2: Model performance monitoring at NW side

- ZTE: Study necessary KPIs for model performance monitoring, e.g., threshold to determine the monitoring accuracy

- Google: SGCS cannot indicate the status of performance gap for two CSIs

- Google: Model monitoring should not be based on SCS

Ground-truth CSI-based monitoring Huawei, HiSilicon, ZTE, Fujitsu

Methodology description

- ZTE: UE should report ground-truth CSI to for network to calculate the monitoring metrics. In order to improve the performance of network-based model monitoring, a higher resolution ground-truth label needs to be reported by UE.

- Huawei, HiSilicon: To evaluate the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI), the methodology can be considered in terms of the statistical gap between the actual intermediate KPI and the target intermediate KPI for a group of test data samples, where the actual intermediate KPI is calculated with the recovery CSI and the given ground-truth CSI format, and the target intermediate KPI is calculated with the same recovery CSI and the ground-truth CSI of Float32.

- Huawei, HiSilicon: For the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI) by using the statistical gap for a group of test data samples, the following metrics can be considered to describe the monitoring accuracy:

  - Metric 1: The mean gap over the group of test data samples

  - Metric 2: The percentage of the samples for which the gap in terms of an absolute value is smaller than a threshold

  - Metric 3: The percentage of the samples for which the gap in terms of a positive bias lower than a positive threshold, or in terms of a negative bias higher than a negative threshold, where bias is denoted as actual intermediate KPI - target intermediate KPI

    - Metric 3-1: if the positive bias is higher than the positive threshold, such inaccuracy case is deemed as misdetection.

    - Metric 3-2: if the negative bias is lower than the negative threshold, such inaccuracy case is deemed as false alarm.

Views/findings

- Huawei, HiSilicon: For AI/ML-based CSI compression, consider the UE report of high resolution ground-truth CSI using Rel-16 Type II CB with new parameters for the monitoring of intermediate KPI at the Network side.

  - Example 1: New parameters of Rel-16 Type II CB are {L=10, p=0.9, beta=0.31, amplitude quantization: 4 bits, phase quantization: 6 bits}.

  - Example 2: New parameters of Rel-16 Type II CB are {L=12, p=0.8, beta=0.5, amplitude quantization: 4 bits, phase quantization: 6 bits}.

- Huawei, HiSilicon: For AI/ML-based CSI compression, the monitoring performance is heavily deteriorated if the Rel-16 Type II CB with legacy resolution (e.g., PC8) is adopted. Introducing higher resolution ground-truth CSI format of Rel-16 Type II CB with new parameters can significantly improve the monitoring performance in terms of monitoring accuracy, false alarm, and misdetection. E.g., for threshold of absolute SGCS gap = 0.01/0.02/0.05, respectively:

  - When PC8 is adopted for monitoring, only 21%/42%/81% monitoring accuracy is achieved for the 60 bits CSI payload, and 8%/21%/67% monitoring accuracy is achieved for the 240 bits CSI payload.

  - When 1014 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 56%/82%/99% for 60 bits CSI payload and 63%/89%/100% for 240 bits CSI payload.

  - When 1610 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 63%/88%/100% for 60 bits CSI payload and 75%/97%/100% for 240 bits CSI payload.

- Fujitsu: For intermediate KPIs based model monitoring at the NW side, UE needs to report the target CSI to the NW side which may be realized by high resolution codebook quantization using Rel-16 type II-like method with new parameter values.

- ZTE: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is lower than 90% based on the reference CSI of eType-II PC8 under the case of SGCS threshold of 0.8, while for other cases, the monitoring accuracy is higher than 95%.

- ZTE: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy

Input-based monitoring

Methodology description

- Qualcomm: Training samples are partitioned into two groups … the distribution of the distance of a sample from samples belonging to an unmatched group is biased from the distribution of the distance to samples belonging to its matched group…we first divide the testing samples to matched samples and unmatched samples where matched samples are those belong to group 1 and unmatched samples are those do not belong to group 1. Then, we further divide matched samples and unmatched samples based on the distance to group 1.

Views/findings

- Qualcomm: Model monitoring based on metrics derived by comparison between input samples at inference and training samples can have strong relationship with the inference accuracy. As a result, input-based monitoring appears promising

- Qualcomm: Study specification impact of input-based model monitoring on the UE-side by comparing input samples at inference time to the training samples

Proxy model/nominal model-based monitoring vivo, MediaTek, Qualcomm, Lenovo, Fujitsu, ZTE

Methodology description

- MediaTek: Instead of traversing input CSI to gNB or exchanging AI/ML model between gNB and UE, a promising approach is using a proxy decoder at UE side.

- Qualcomm: For training the proxy model for the purpose of monitoring UE-side model only, UE may develop a private decoder which generates a reconstructed CSI, and the SGCS label is computed using this reconstructed CSI and its ground-truth…the training would need to use SGCS labels derived from the reconstructed CSI output by the actual CSI decoder deployed at the NW side.

- Lenovo: the UE performs monitoring without the need for the NW-side. More accurately, the UE uses its encoder and “UE-side nominal decoder” to generate the estimation of the “output” of the NW-side.

- Lenovo: the UE uses the “encoder” and the “updated UE-side nominal decoder” to generate the estimation of the “output” of the NW-side.

- Fujitsu: For intermediate KPIs based model monitoring at the UE side,… We consider the case that the same model structure (Transformer) but smaller model size (about 1/40 comparing with CSI reconstruction part) for the monitoring decoder and the case that the different model structure (CNN) for the monitoring decoder.

- ZTE: UE has its own reference reconstruction part in CSI generation model which is not the same as the actual CSI reconstruction model used at the NW side.

Views/findings

- MediaTek: The structures of the proxy and actual CSI reconstruction parts do not need to match either in size or type (which further secures proprietariness of gNB’s CSI reconstruction)

- MediaTek: If intermediate-KPI-based monitoring is adopted as the main monitoring method, prioritize UE-side monitoring with proxy CSI reconstruction.

- Qualcomm: Model monitoring using a proxy model that outputs the intermediate KPI directly shows an accurate inference accuracy prediction

- Qualcomm: Study specification impact of methods that directly outputs intermediate KPI at the UE side

- Lenovo: Genie-aided model monitoring Scheme A has the perfect TP and FP rates since it assumes the availability of the “expected output” at the NW-side. If we want to have this data, the downside is the amount of overhead that is associated with transmission of the “expected output” to the NW-side

- Lenovo: Model Monitoring Scheme B (based on “UE-side nominal encoder”) archives a good FP rate with much less overhead/latency compared to Scheme A. The TP rate, however, is a bit low in this scheme

- Lenovo: Model Monitoring Scheme C (based on “updated UE-side nominal decoder”) significantly improves the TP rate at the expense small degradation on False Positive Rate. This scheme does not have the overhead/latency of Scheme A as well.

- Lenovo: Study the performance of “UE-based” model monitoring with “UE-side nominal decoder” and with “updated UE-side nominal decoder”.

- Lenovo: Model Monitoring Scheme C leads to a better performance compared to the monitoring scheme B. This again show the benefit of using “updated UE-side nominal decoder”. Note that Scheme C need extra data exchange during the training phase but no extra overhead during the monitoring phase.

- Lenovo: Use ROC curves to better compare the performance of different monitoring schemes over different threshold values

- vivo: Monitoring based on proxy model at UE side in CSI compression can lead to near-ideal model switching when the scenario changes

- Fujitsu: For joint training, the SGCS of the monitoring decoder is like that of the CSI reconstruction part whether the structure between the monitoring decoder and the CSI reconstruction part is the same

- Fujitsu: For separate training, the SGCS of the monitoring decoder is like that of the CSI reconstruction part when the backbone of AI/ML models is similar; But the SGCS shift can be observed when the backbone is different

- ZTE: For model performance monitoring at UE side, assumed output CSI at UE can be used as CSI reconstruction part output to achieve a good monitoring accuracy

Other views on monitoring methods

- Qualcomm: Real-time performance monitoring that incurs overhead and/or additional processing complexity is unnecessary

- Qualcomm: Model monitoring based on ground-truth provided by UE to the network requires large signalling overhead and may be sensitive to large latency

- Qualcomm: For model performance monitoring, specification change for reporting the target CSI with high resolution from UE to network requires clear justification as it incurs additional overhead and may not be necessary

- Fujitsu: For AI/ML based CSI feedback using two-side model, study the feasibility and the performance of using UE-side monitoring decoder, taking at least the following aspects into account:

  - KPIs for performances

  - Complexity

  - Multi-vendor cases

## 1st round email discussions

### 3.2-1: AI/ML training methods

#### Issue#3-1 (Medium priority) Boundary between Type 2 and Type 3

Moderator note: In the last meeting, there is controversy on the boundary between Type 2 and Type 3. Background can refer to the question discussed in the last meeting, which is also pasted below:

#112 meeting Upd Question 3.3.1: For the evaluation of training types of CSI compression, which of Type 2 and Type 3 or a new type should it belong to, if - Step A: first side is trained, and after the training for the first side is finished - Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

To facilitate the discussions, Moderator copied the procedures for Type 2 and Type 3, respectively (Type 3 only NW first is provided as an example).

#110 meeting Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded.

#111 meeting Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies

Some understandings from Moderator:

- As shown in Type 2, in highlighted part it says NW trains the CSI reconstruction part, while this new behavior freezes the NW side parameter updating in Step B. However, in the implementation, whether NW parameters are updated or not, it is unseen by the UE – NW can also update the parameters if needed.

- As shown in Type 3, in highlighted part it says NW shares the UE side with the information, while this new behavior needs NW and UE to exchange FP and BP (rather than a one way sharing).

- From the example in the bracket of Type 3, this new behavior is more close to Type 2 as in Step B it also asks NW and UE to exchange FP/BP.

- From the interaction behavior between NW and UE, the “freeze and train” behavior asks the NW and UE to perform real time exchange of FP info. and BP info. over massive epochs, which is more like Type 2.

Therefore, it is Moderator’s feeling that this behavior is more like Type 2 than Type 3.

Question 3.2.1: For the evaluation of training types of CSI compression, which of Type 2 and Type 3 should it belong to, if

- Step A: first side is trained, and after the training for the first side is finished

- Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

| Type 2 | Support/Can accept | Futurewei (with comments) , vivo (with comments), ETRI Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, CATT, LG Electronics, Fraunhofer, Lenovo, Fujitsu, Apple, ZTE, Intel, NVIDIA, MediaTek, Ericsson, InterDigital, AT&T, Samsung |
| --- | --- | --- |
|  | Object/Concern | Qualcomm |
| Type 3 | Support/Can accept | Futurewei, Lenovo (comment), Ericsson, Qualcomm |
|  | Object/Concern |  |
| Type 4 | Support/Can accept | Ericsson (preferred), Qualcomm |
|  | Object/Concern |  |
| Not support this new behavior | Support/Can accept |  |
|  | Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Futurewei | Based on the description of Question 3.2.1, we think it may belong to Type 2 or Type 3. As FL explained, “However, in the implementation, whether NW parameters are updated or not, it is unseen by the UE – NW can also update the parameters if needed.”, in the case that the other part is NOT updated, then this procedure can be considered as Type 3 while in the other case that the other part is updated, then this procedure can be considered as Type 2. We think this type of training behavior may happen when initially both parts of the two-sided model may be trained as Type 3 (the first part is frozen while the second part is being trained), while the first part may be updated if needed (as FL put it) during the model update situation. Since Step 2 already indicates “the first side is frozen”, we think it can be considered as Type 3 based on the wording/description while we are ok also to consider this procedure as Type 3 if companies think the implementation cannot be determined. |
| vivo | We believe the method that freezes decoder and solely updates encoder via exchanging FP/BP information belongs to type2 training collaboration, and type3 refers to the framework of exchanging dataset to facilitate the training of models at other entities. It should be noted that, as FL mentioned, as the implementation, whether NW parameters are updated or not is unseen by the UE, and NW can also update the parameters if needed. |
| ETRI | We shares similar views from Moderator. |
| OPPO | We agree with Moderator that this new training behavior is more like Type 2 since it requires FP and BP exchange in Step B. |
| Xiaomi | We tend to agree with FL’s analysis that the new behavior belongs to Type 2. There is similar problem for both type 2 training and the new training behavior when multiple NW and multiple UE are trained. How to merge the FP/BP when simultaneous training performed and how to keep the fairness when sequentially training performed. |
| CATT | We have similar view as moderator that although it says the first side is frozen in Step B, in the implementation, NW can also update the parameters if needed. Therefore we think the behavior is more like Type 2. |
| LG Electronics | Agree with moderator. |
| Fraunhofer | Since there are multiple FP and BP exchanges required in step B for the training of the other side, it belongs to type 2. |
| Lenovo | Our understanding is that in Type-2 each iteration (epoch) of training (for updating the NN weights”) needs some communication between the two-sides, for example, the gradients of the error. In Type-3, however, each side can get perform multiple iterations epochs (update the weights of its local NN) without the involvement of the other-side just by getting a set of training data. In other words, in Type-2 we need to get “training related info” from the other side in each epoch but in type-3 we can gets lots of training data first and then perform the complete training loop for multiple epochs. Based on this, we believe the proposed scheme as described by FL is of Type-2. We note however, if in the proposed scheme, after training of the decoder, it sends a set of samples to the UE side (for examples similar to “iterative separate training” that we have proposed) using which the UE can train its encoder, then it will be a type-3 training. |
| Fujitsu | We agree with FL that it does not match exactly with either Type 2 or 3 training. However, considering the gradient exchange is involved in the training process for multiple epochs, it is more close to Type 2 training in terms of potential STD impacts, and hence can be categorized into Type 2 training. |
| ZTE | To our understanding, it belongs to Type 2 training, since the CSI generation model training needs FP/BP interaction with CSI reconstruction model for some epochs and it is not a one-way information sharing like Type 3 training. |
| Intel | Agree with the moderator (i.e., it is Type 2) |
| MediaTek | Our understanding is also aligned with Moderator. As the BP crosses two entity, it is more like Type 2. |
| Ericsson | This is not a big issue. Perhaps easiest to denote this Type 4 to avoid confusion with previous agreements. Also we anyway have to specify in more details what we mean when we say “Type 2” if we now include this in Type 2 as there would be two Type 2 schemes…. |
| Qualcomm | We can accept to call it Type 3 or Type 4 for the sake of progress. Type 2 definition requires joint training of NW-side and UE-side models. But in this case, first side is trained in step A, then second side is trained in step B. So it cannot be Type 2. The conclusion made in 9.2.2.1 says “for the evaluation of an example of Type 3”, and therefore it is not the definition of type 3. This training method shares a lot of similarities with Type 3 and has many differences from Type 2. Treating it as Type 2 training, will make the pros / cons / applicability discussion very confusing. |
| InterDigital | Share the same view with the moderator |

#### Issue#3-2 (Medium priority) Extendibility evaluation

Moderator note: QC [28], Ericsson [14] and DOCOMO [30] evaluated the cases of freeze-and-train. To Moderator’s understanding, such behavior is more likely to support the extendibility case, where after the NW#1/UE#1 has trained the model, and a new UE#2/NW#2 joins and wants to train a paired model with the NW#1/UE#1, with potentially new data distribution (to emulate a newly released UE/gNB with new RF/baseband processing algorithms which may bias the data distribution with legacy UE/gNB) for the new UE#2/NW#2 [28][30], then such freeze-and-train is applied with the target of not impacting the pairing of NW#1/UE#1 model with legacy UEs/NWs.

Based on the understanding above, we may construct an extendibility simulation case with a broader perspective, to evaluate the performance of different freeze-and-train behaviors on achieving the extendibility. In Step A, the NW/UE trains the model with the old dataset (e.g., Uma in [30]), then in Step B a new UE/NW joins with new dataset (e.g., Umi in [30]), then NW/UE needs to incorporate the new dataset and train the new UE/NW by freezing its decoder/encoder.

Two examples are shown in below: the left figure demonstrates the FB/BP loop-based freeze and train (method in Question 3.2.1), while the right figure demonstrates a Dataset delivery-based freeze and train, where in Step B1, the decoder is frozen while the nominal encoder at NW side is trained based on updated dataset, after which, in Step B2, the updated dataset including the input/output of the nominal encoder is delivered to UE for its encoder training.

| FP/BP loop-based freeze and train | Dataset delivery-based freeze and train |
| --- | --- |

Question 3.2.2: For the training methods of CSI compression, do you agree to establish the simulation cases to evaluate the extendibility as elaborated in the following steps?

- NW first

  - Step A: For a NW, its CSI reconstruction part is trained under Dataset#A.

  - Step B: For a newly joining UE, its CSI generation part is trained with the frozen NW side CSI reconstruction part under Dataset#B:

    - Option 1: its CSI generation part is trained with FP and BP exchange to the frozen NW side CSI reconstruction part, under Dataset#B.

    - Option 2: its CSI generation part is trained with a set of information (e.g., dataset) shared by the NW side, which is derived by the joint training of the NW side CSI generation part and the frozen NW side CSI reconstruction part under Dataset#B.

- UE first

  - Step A: For a UE, its CSI generation part is trained under Dataset#A

  - Step B: For a newly joining NW, its CSI reconstruction part is trained with the frozen UE side CSI generation part under Dataset#B, e.g.,

    - Option 1: its CSI reconstruction part is trained with FP and BP exchange to the frozen UE side CSI generation part, under Dataset#B, e.g.,

    - Option 2: its CSI reconstruction part is trained with a set of information (e.g., dataset) shared by the UE side, which is derived by the joint training of the frozen UE side CSI generation part and the UE side CSI reconstruction part under Dataset#B.

- FFS for Step A, which training type (Type 1/2/3) is applied for the training of the NW side CSI reconstruction part

- FFS how to construct Dataset#A and Dataset#B

| Support/Can accept | vivo (with comment) Huawei/HiSi, OPPO, Fraunhofer, Lenovo (comment), Fujitsu, Apple, NVIDIA, MediaTek, Qualcomm |
| --- | --- |
| Object/Concern | Intel |

| Company | View |
| --- | --- |
| vivo | First of all, we believe that the method of freezing NW side model and updating UE-side model can be also implemented in type1 training. Note: to our understanding, the principle of training type1 is training at a single entity and transferring models to other entities, where the training algorithm is highly flexible and not restricted to simultaneously tuning parameters of both CSI generation part and CSI reconstruction part. For example, in type1 training, the training entity can also only update parameters of CSI generation part while freezing those of CSI reconstruction part to fulfill the extendibility requirement. Also, we believe that this frozen NW side training method belongs to a finetune training mechanism. We can establish the simulation cases on top of training type1, 2, and 3 by freezing either CSI reconstruction part or CSI generation part depending on NW first or UE first. We can accept to evaluate the performance of freezing decoder and updating encoder without confining the training collaboration types. |
| OPPO | This case can be optionally simulated by companies. |
| Xiaomi | We are not clear about the motivation of this proposal, why different dataset have to be involved into the training type evaluation. We think we can firstly decide whether to evaluate the 1-1 training case as described in Question 3.2.1, then discuss the M-N case. |
| Fraunhofer | The simulations results can be optionally provided by the companies. |
| Lenovo | We are generally fine with this proposal with some modifications as below: - When a new UE or a new NW side is added we can still train it using the UE first or the NW first and also gor joint training the “first” and second terminology is not well defined. So we suggest to change the name of each group as below. - For option 2, we suggest to remove “which is derived by the joint training” since the dataset might be derived differently. Foe example, the NW send some samples representing its CSI reconstruction model to the UE so the UE can determine a local model of the CSI reconstruction model and then train its CSI generation model using that. This way, we can train them sequentially. For the training methods of CSI compression, do you agree to establish the simulation cases to evaluate the extendibility as elaborated in the following steps? - Frozen NW side NW first - Step A: For a NW, its CSI reconstruction part is trained under Dataset#A. - Step B: For a newly joining UE, its CSI generation part is trained with the frozen NW side CSI reconstruction part under Dataset#B: - Option 1: its CSI generation part is trained with FP and BP exchange to the frozen NW side CSI reconstruction part, under Dataset#B. - Option 2: its CSI generation part is trained with a set of information (e.g., dataset) shared by the NW side, which is derived by the joint training of the NW side CSI generation part and from the frozen NW side CSI reconstruction part under Dataset#B. - Frozen UE side UE first - Step A: For a UE, its CSI generation part is trained under Dataset#A - Step B: For a newly joining NW, its CSI reconstruction part is trained with the frozen UE side CSI generation part under Dataset#B, e.g., - Option 1: its CSI reconstruction part is trained with FP and BP exchange to the frozen UE side CSI generation part, under Dataset#B, e.g., - Option 2: its CSI reconstruction part is trained with a set of information (e.g., dataset) shared by the UE side, which is derived by the joint training of from the frozen UE side CSI generation part and the UE side CSI reconstruction part under Dataset#B. - FFS for Step A, which training type (Type 1/2/3) is applied for the training of the NW side CSI reconstruction part - FFS how to construct Dataset#A and Dataset#B |
| ZTE | We are not clear that Option 1 &2 in Step B belong to which training type. To our understanding, Option 1 is a kind of Type 2 training and Option 2 belongs to Type 3 training. |
| Intel | Companies can bring evaluation results, but in our view, there is no need to have separate agreements for each and every training/update/refinement procedure possible. Instead, we can focus on a list of basic cases which are already covered by agreements. |
| Ericsson | Agree with Intel |

#### Issue#3-3 (Medium priority) Modeling of various datasets from different UE types

Moderator note: In Issue#3-19 of the #112 FL summary, a new issue is raised on how to model the difference on the data distribution due to different UE types. In this meeting, some companies raised that the distribution can differ due to RF aspects, implementation differences, UE antenna layout, imbalance of UE antenna gains, or different channel environments, etc. E.g., DOCOMO [30] models different UE types with different Uma/UMi, UE antenna layouts, UE antenna power imbalance values. Lenovo [27] models different UE types with O2I and LOS channels, respectively.

Therefore, a question is then raised in below to collect views on how to model such UE type difference.

Note this issue could be a follow-up issue of Issue#3-2 on “FFS how to construct Dataset#A and Dataset#B”.

Question 3.2.3: For CSI compression evaluation, for the modeling of the variation among dataset distributions over different UE types (e.g., different UE vendors/UE versions of a single vendor), consider the following aspects

- Different deployment scenarios, e.g., UE#1 with UMa while UE#2 with UMi, or UE#1 with LOS while UE#2 with O2I.

- Different antenna deployments, e.g., UE#1 with (1,1,2,1,1,1,1) while UE#2 with (1,2,1,1,1,1,1) for 2Rx.

- Different antenna gain imbalance values.

- Other factors are not precluded.

| Support/Can accept | Futurewei, vivo, OPPO, Xiaomi(with comments) , Spreadtrum, LG Electronics, Fraunhofer, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, Qualcomm (comments), InterDigital, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please also share your views in below if you have a different method to model different UE types. |
| Xiaomi | In our understanding, the different UE types is a kind of generalization evaluation, and companies can provide the evaluation results in the generalization table based on different UE assumptions, e.g., different scenario, different antenna architecture. |
| ZTE | From our perspective, dataset construction for different UE types can refer to the generalization cases in CSI compression sub use case, and there is no need to discuss again. |
| Qualcomm | Regarding the first item, deployment scenarios should not be used to model UE types as they are a different aspect related to generalization. Another factor to model could be baseband implementation differences, e.g. in the precoder computation. Due to the phase ambiguity associated with the SVD operation, the phases of the precoders on different RBs may vary depending on the exact implementation of the SVD operation. |

#### Issue#3-4 (Low priority) Other Type 3 training methods

Moderator note: For most simulation methods evaluated by companies, it is Moderator’s understanding that the set of information shared from NW to UE is {input ground-truth CSI, output CSI feedback label}, and the set of information shared from NW to UE is {input CSI feedback, output ground-truth CSI}.

In this meeting, besides the above method, more variations of Type 3 training methods are raised, including parallel training raised by CATT [10], iterative separate training raised by Lenovo [27], and end-to-end dataset sharing (Method 2 in Figure 10) raised by Nokia [8]. These methods are brought up in the following question to see the views of companies on whether we need to align the methodology and simulation case to evaluate them.

Question 3.2.4: For the training methods of dataset sharing based Type 3 training, other than the dataset sharing of {input: ground-truth CSI, output label: CSI feedback} from NW to UE for NW first training and the dataset sharing of {input: CSI feedback, output label: ground-truth CSI} from UE to NW for UE first training, do we need to align the evaluation methodology/simulation cases for the following Type 3 training methods?

- Option 1: Parallel training [10]

- Option 2: Iterative separate training [27]

- Option 3: End-to-end dataset sharing [8]

- Option 4: Other dataset sharing based Type 3 training methods

| Option 1 | Support/Can accept |  |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | Lenovo |
|  | Object/Concern |  |
| Option 3 | Support/Can accept |  |
|  | Object/Concern |  |
| Option 4 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We can accept that the listed methods are considered as optional. |
| OPPO | These new Type 3 training options can be optionally simulated and reported by companies. No need to align the EVM for these cases. |
| CATT | No need to align the evaluation methodology/simulation cases. Anyway, they are optional. Companies can report which one is applied. |
| Lenovo | We are in favor of Option 2 and also agree that companies can chose and report any option if they want. |
| Fujitsu | We think that there is no need to align the evaluation method for these variations, considering that they are not widely adopted by majority companies. |
| ZTE | For the above Type 3 training method, it is up to companies to evaluate and report the performance and no need to align the simulation cases. |
| Ericsson | We can make an agreement on the definitions of these options for the completeness of the study. No need to discuss EVM etc. |
| Samsung | Same view as vivo. |

### 3.2-2: AI/ML monitoring

#### Issue#3-5 (High priority) Methodology-high level principle

Moderator note: In the last meeting, we have discussed the methodology for model the monitoring with no consensus. In this meeting, Huawei [3] and ZTE [4] proposed the methodology for monitoring accuracy for NW side monitoring of intermediate KPI. ZTE [4], Fujitsu [13], MediaTek [24], Lenovo [27], Qualcomm [28], vivo [5] proposed the methodology for monitoring accuracy for UE side monitoring of intermediate KPI. Moderator tries to summarize a unified methodology on modeling the monitoring methods under this issue, and provide elaborations for step 2 and step 3 in later issues of this sub section.

Question 3.2.5: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, the methodology can be considered in terms of the statistical gap of monitored intermediate KPI:

- Step1: Generate test dataset including K test samples

- Step2: For each K test sample, the gap of monitored intermediate KPI (KPIGap) is calculated as a function of KPIGap= f(KPIActual, KPIGenie) (e.g., KPIGap= KPIActual-KPIGenie in case of SGCS), where KPIActual is the actual intermediate KPI (e.g., actual SGCS), and KPIGenie is the genie-aided intermediate KPI (e.g., genie-aided SGCS).

- Step3: Calculate the statistical result of the KPIGap over K test samples which represents the monitoring accuracy performance.

| Support/Can accept | Futurewei (with comments), vivo (with comments), CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, CATT(with comments), Fraunhofer, Lenovo (comment), Fujitsu, NVIDIA |
| --- | --- |
| Object/Concern | Ericsson, Qualcomm (comments) |

| Company | View |
| --- | --- |
| Futurewei | It should be clarified that this question is to determine performance monitoring accuracy for study and evaluation purpose as the genie-aided ground truth will NOT be available during deployment phase. |
| vivo | We are okay to use the method of computing intermediate KPI gaps to evaluate the accuracy of monitoring at NW-side via CSI measurement reporting. For UE-side monitoring, we think requiring all monitoring methods to accurately provide the real SGCS is too stringent. In fact, we monitor CSI compression models mainly to make decisions on model switching or fallback. If a monitoring method could help to make correct and suitable switching/fallback decision (i.e., no false alarm and misdetection), its reliability is also acceptable. In addition, in Step1, K test samples should be time-related samples generated with spatial consistency and different wireless environments (e.g., from a LoS-dominant cell to a NLoS-dominant cell), to facilitate the necessity of performance monitoring accordingly. |
| ETRI | We think it would be better if Genie-aided intermediate KPI is further clarified. Is it SGCS of genie-aided CSI feedback? Then it will be always 1. We also think we can consider that comparing with the legacy method (e.g. eType2) for performance monitoring. |
| OPPO | We are general OK. But it should be clarified how to obtain the test dataset in Step1. |
| Xiaomi | We agree with the high level principle of monitoring evaluation. |
| CMCC | Support in principle if it is just for model monitoring evaluation. But for Step 1, we may need a separate discussion on how to get the dataset and dataset generation assumption. |
| Spreadtrum | We are OK with the proposal. |
| CATT | Similar view as Futurewei that the purpose of the evaluation shall be clarified. |
| LG Electronics | Better to have different term instead of genie-aided KPI, e.g. KPI_upperbound. According to Q3.2.6, it seems higher resolution ground-truth CSI (but have some quantization error) is employed as genie-aided KPI. But, genie-aided normally means perfect ground truth CSI. |
| Fraunhofer | We support the proposal, if the data set and the statistical features of the samples are explained by the companies. |
| Lenovo | We support this proposal in general with a comment that the in Step1, K test samples should cover samples from the same dataset used for training and also from dataset generated form other case like for other scenarios/configurations. We need to have such samples to evaluate the performance of monitoring scheme in case the settings of the environment changes. How to generate dataset that cover different cases is somehow related to Question 3.2.3 |
| Apple | How to define genie-aided KPI? Need clarification. Is genie-aided KPI defined as below proposal? “KPIGenie is calculated with the same output CSI and the ground-truth CSI of Float32”? |
| ZTE | We have a question about the statistical KPIGap calculation over K test samples, is it calculated for CDF or an average value? |
| Intel | We are not sure if the KPI gap metrics will reflect the actual impact of model performance monitoring on system performance. Thus, meaningful conclusion can’t be made based on the presented methodology. So, we don’t see the need to agree this methodology in the SI. We propose the following change for the methodology, which, in our view, better represents the impact of model performance monitoring on system performance. Question 3.2.5: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, the methodology can be considered in terms of the statistical gap of monitored intermediate KPI: - Step1: Generate test dataset including K test samples - Step2: For each K test sample, the gap of monitored intermediate KPIs KPIGenie and KPIActual (KPIGap) is are calculated as a function of KPIGap= f(KPIActual, KPIGenie) (e.g., KPIGap= KPIActual-KPIGenie in case of SGCS), where KPIActual is the actual intermediate KPI (e.g., actual SGCS), and KPIGenie is the genie-aided intermediate KPI (e.g., genie-aided SGCS). Step3: Calculate monitoring accuracy measured by ¦a-b¦/K, where a and b are the number of samples with KPIGenie and KPIActual larger or lower than a threshold T. |
| Ericsson | I think we need to first determine whether there is any intermediate KPI that can reflect the true performance well. What we have observed is that only for low load and rank 1, the SGCS can somewhat reflect the performance seen in system level simulations, for higher loads and when rank is not restricted to always 1, the SGCS poorly represent the SLS results. We even found cases where two models with the same SGCS for layer 1 and 2 gives different results in MU-MIMO SLS simulations. So the initial question is whether it is feasible to find an intermediate KPI that is good enough for performance monitoring? Or can we find an intermediate KPI that is not specified (but the steps outlined in this proposal are still valid and are specified)? |
| Qualcomm | The analysis of monitoring performance should include accuracy, overhead and complexity together to present a complete picture to compare the different options. The monitoring schemes considered in Question 3.2.6 each have a different implication on signaling overhead, and UE-side complexity. For example, reporting ground-truth CSI may incur high signaling overhead. Also, a UE may incur high processing complexity to produce both ML-based CSI and quantized ground truth CSI from a channel sample. Running a full CSI reconstruction model at the UE-side may also have complexity implications compared to running a model that estimates the SGCS directly. Regarding the accuracy aspect, in this proposal, for Step 3, absolute value of the KPI gap should be used. |
| InterDigital | Not sure if the genie-aided KPI is always available. Also, need clarification how to define the genie-aided KPI. |

#### Issue#3-5a (High priority) Methodology-per sample gap

Moderator note: Elaboration of the first issue on Step2. The methods of NW side monitoring and UE side monitoring are summarized as two cases. Proponent companies may also raise other cases for evaluation.

Question 3.2.6: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, for Step2, the per sample KPIGap is considered for

- Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with Float16, R16 Type II-like method, etc.), where

  - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format.

  - KPIGenie is calculated with the same output CSI and the ground-truth CSI of Float32.

- Case 2: UE side monitoring of intermediate KPI with a proxy CSI reconstruction part, where the monitoring accuracy is evaluated for the output CSI of the proxy CSI reconstruction part at UE:

  - KPIActual calculated with the output CSI of the proxy CSI reconstruction part at UE and the ground-truth CSI.

  - KPIGenie calculated with the output CSI at the NW side and the same ground-truth CSI.

  - FFS how to train the proxy CSI reconstruction part and the resulting monitoring performance.

  - FFS whether/how to evaluate the generalization performance of the proxy model.

- Case 3: others

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, LG Electronics, Fraunhofer, Lenovo (comment), ZTE, NVIDIA, InterDigital |
| --- | --- |
| Object/Concern | Apple (comment), Intel, Qualcomm |

| Company | View |
| --- | --- |
| Moderator | For the FFS how to train the proxy CSI reconstruction part and the resulting monitoring performance, Lenovo, Qualcomm and Fujitsu discussed the method based on separate training. Other companies may provide your views on candidate methods. |
| Futurewei | Same comments as indicated in our response for Question 3.2.5. |
| vivo | We are supportive of the proposal. In the performance evaluation, the overhead (e.g., ground-truth CSI for case 1, and the proxy model size for model transfer) should be reported by each proponent. |
| OPPO | We are general OK for both Case 1 and Case 2. But for Case 2 with proxy CSI reconstruction model, we have the following concern: 1. Whether the monitoring result of UE-side CSI generation part model + proxy CSI reconstruction part model can effectively reflect the actual performance of UE-side CSI generation part model + NW-side CSI reconstruction part model? This may highly depend on the specific implementation of the proxy CSI reconstruction model. |
| Xiaomi | We support this proposal. For case2, we think the ground-truth CSI for calculating both KPIActual and KPIGenie should be ground-truth CSI of Float32. |
| CMCC | Support in principle if it is just for model monitoring evaluation. |
| CATT | We still have some questions/concerns: 1) Whether Float32 can still be used for calculation of KPIActual in Case 1? 2) The feasibility and availability of ‘proxy CSI reconstruction part’ in Case 2. 3) UE-side monitoring may still use other methods, e.g. high resolution Output-CSI-UE from NW, which is not reflected here. |
| Lenovo | The proposal looks good to us. We also agree with vivo on the need to include overhead related metric. So, we suggest adding the following FFS: FFS: how to evaluate the overhead and latency requirement of ground-truth CSI transmission for Case-1 |
| Fujitsu | Support in general. Considering the fact that Int8 is more commonly used in the AI/ML model application, we suggest changing from Float16 to Int8. Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with Int8, Float16, R16 Type II-like method, etc.), where |
| Apple | Do we assume additional dataset labeling to train the UE side proxy de-coder? For example, in NW first training, NW provide dataset of target CSI, encoder output, and actual output CSI. UE can train the actual encoder, and the proxy decoder? This additional data label will enable UE to calculate KPI_actual, also can be used to calculate RI and CQI. |
| ZTE | We are fine with the proposal. |
| Intel | Given that NW performance monitoring methods are considered under AI 9.2.2.2, we think that this agreement is not necessary, especially considering that new aspects related to the proxy model are under FFS. |
| Qualcomm | In our proposal, the UE uses an SGCS estimator model that does not output the reconstructed CSI, but instead directly outputs an estimate of the intermediate KPI. This allows for a simpler model. Please include this case either in Case 2 by updating the wording, or as a new Case 3. Also, due to processing complexity concerns, a UE may not be able to provide ground truth CSI matched and ML-based CSI for the same channel sample. The impact of such a delay on the accuracy should be studied. |
| Samsung | We are fine with the proposal |

#### Issue#3-5b (High priority) Methodology-statistical result

Moderator note: Elaboration of the first issue on the metrics of Step3. 3 options are raised in forms of monitoring accuracy, misdetection and false alarm. Proponent companies may also raise other options for evaluation.

Question 3.2.7: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, for Step3, the statistical result of KPIGap over K test samples is considered in forms of

- Option 1: Monitoring accuracy: The percentage of the samples for which KPIGap<KPIth, where KPIth is a threshold of the intermediate KPI.

- Option 2: Misdetection: The percentage of the samples for which KPIGap>KPIth_pos, where KPIth_pos is a second threshold of the intermediate KPI.

- Option 3: False alarm: The percentage of the samples for which KPIGap<KPIth_neg, where KPIth_neg is a third threshold of the intermediate KPI.

- Option 4: other metrics.

- FFS the values of KPIth, KPIth_pos, and KPIth_neg.

| Support/Can accept | Huawei/HiSi, OPPO, Fraunhofer, Lenovo (comment) |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Moderator | The intention of Moderator is not to make down selection of the options. Companies may leave your comments if you want to make potential down selection. |
| Futurewei | This question should wait till some consensus is reached among companies for Question 3.2.5 and Question 3.2.6. |
| vivo | Agree Futurewei’s comment. |
| OPPO | We think that the definition of misdetection and false alarm should be clarified firstly. |
| Xiaomi | We share similar view with Futurewei that Question 3.2.7 can be discussed later. |
| CATT | Similar comment as Futurewei and vivo. Also, whether/how to define the threshold for the intermediate KPI gap is quite unclear. |
| LG Electronics | Agree with Futurewei’s comment. |
| Fraunhofer | Similar views as Futurewei. |
| Lenovo | In general we support this proposal with the following comments. - Option 2 and option 3 should be reported jointly as a pair (Misdetection, False alarm) - The evaluation of (Misdetection, False alarm) should be carried out for a range of KPIth_pos, and KPIth_neg. - As there will be (Misdetection, False alarm) for different threshold values, we suggest to use ROC curve and use AUC (area under the curve) related to ROC for comparing different scheme. |
| ZTE | We think KPI should be considered for model performance monitoring rather than KPIGap, since KPI gap can only reflect on the relative relation between actual SGCS and genie-aided SGCS, and it may not reflect the absolute value of current AI/ML performance. For example, we assume actual SGCS is 0.5 and genie-aided SGCS is 0.55, hence the SGCS gap is 0.05, which may satisfy the predefined monitoring threshold. However, it is meaningless for evaluating this monitoring performance because AI/ML model performance is so bad that it is not applicable for current situation. Therefore, we suggest first considering the relationship of actual KPI and genie-aided KPI with the threshold, Option 2 and Option 3 can be FFS. |
| MediaTek | We agree with Futurewei’s comment and some terminologies should be clarified first as OPPO mentioned. |
| Qualcomm | The definition of false alarm and misdetection depends on what should be considered the ground truth for model failure, and on what subsequent actions will be taken based on the monitoring result. It would be simpler to focus on the accuracy of the intermediate KPI estimation in isolation. The statistics of the absolute value of the KPI gap could be directly used as the result. |
| InterDigital | Agree with Futurewei’s comment |

#### Issue#3-5c (High priority) Methodology-scenario change

Moderator note: vivo [5] and Lenovo [27] raised to consider changed/various scenarios to evaluate the monitoring accuracy in case of changing environments. To Moderator, the straightforward way is to separately test the accuracy of the monitoring method with the same pair of models under different scenarios.

Question 3.2.8: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, the procedure of Step1-Step3 is performed for various scenarios (e.g., Uma/Umi/InH, etc.) to record the statistical result of KPIGap separately.

- FFS the set of various scenarios.

| Support/Can accept | Huawei/HiSi, Xiaomi, Fraunhofer |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | This question can be discussed till some agreement/consensus is reached among companies for Issue#3-5a and Issue#3-5b. |
| vivo | We believe that the K test samples are better to be time-related samples generated with spatial consistency and different wireless environments to construct a model switching (or fallback) procedure. |
| CAICT | We also fine to wait more progress on Issue#3-5b. |
| OPPO | Agree with Futurewei’s comments. |
| Xiaomi | Support in general, companies are encouraged to provide the dataset description in monitoring evaluation. |
| CATT | The question can be discussed after there is some conclusion on issue #3-5. |
| LG Electronics | Agree with Futurewei’s comment. |
| Lenovo | We believe that the K test samples should cover samples from - the same dataset used for training - also from dataset generated form other case like for other scenarios/configurations. We need to have such samples to evaluate the performance of monitoring scheme in case the settings of the environment changes. How to generate dataset that cover different cases is somehow related to Question 3.2.3. @vivo: we understand that your point on having time-related samples but as for each sample we assume that we have the “genie” result we can look at each sample separately. By allowing to have not time-related samples we can test more samples to better determine the performance of the monitoring scheme. |
| Fujitsu | We think that the mentioned procedure should also be performed for various configurations (e.g, antenna configuration, etc.), if it is performed for various scenarios separately as presented in this proposal. |
| ZTE | Further discuss this question till some agreement/consensus are reached for Issue#3-5. |
| MediaTek | Agree with Futurewei |
| Qualcomm | Is the proposal that to repeat only the test for different scenarios, or to also train a model for each scenario and then test in that scenario? Some clarification would help. |
| InterDigital | Agree with Futurewei’s comment |

### 3.2-3: Quantization/dequantization

#### Issue#3-6 (Medium priority) Quantization loss/quantization upper bound

Moderator note: In the last meeting, whether there is need to introduce quantization upper bound has been discussed yet with dispute. Due to relatively limited inputs in the last meeting, this issue is brought up again to collect more voices. In this meeting, Nokia [8], Xiaomi [15], and AT&T [29] believe this upper bound is needed, while ETRI [22] thinks it is not needed.

To Moderator’s understanding, it is not an essential issue, so if we cannot struggle a consensus at this meeting, then it will not be raised further.

Question 3.2.9: For the evaluation of quantization aware/non-aware training, on top of the agreed cases of quantization non-aware training (Case 1) and quantization aware training (Case 2-1, Case 2-2), do you think it is needed to introduce an additional upper bound case of Case 0: non-quantized training and inference (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training and inference), to reflect the performance loss due to quantization?

| Support/Can accept | Futurewei, vivo, Xiaomi, Ericsson, InterDigital, AT&T |
| --- | --- |
| Object/Concern | OPPO, Lenovo, ZTE |

| Company | View |
| --- | --- |
| Futurewei | We agree to consider evaluating performance loss due to quantization. |
| OPPO | It is not necessary to evaluate this upper bound. |
| Xiaomi | We support to evaluate the upper bound case. |
| CATT | OK if it is optional. |
| Lenovo | It is good to have an upper-bound value but we believe the upper-bound assuming that the gNB has access to the correct CSI form a genie. So, for now we do not see the value of evaluating the method using “no quantization” scheme. |
| Fujitsu | The payload size in case 0 is not clear. How to make a fair comparison of the performance of the two existing cases and the new case 0? |
| ZTE | It is not necessary to add an additional upper bound case of non-quantized training and inference, since quantization is mandatory in the actual communication system. |

#### Issue#3-7 (Low priority) Model represented by quantization/look up table

Moderator note: FUTUREWEI [2] raised a new quantization behavior for the small payload size case, where the CSI feedback is generated by using the look-up-table (generated from the quantization CB as well as the decoder) rather than using the encoder. In the inference phase, the UE generates the CSI feedback by directly using the look-up-table which can implicitly represent the model.

During the training phase, the training entity, e.g., gNB generates a CSI generation part, the corresponding vector quantization codebook, and a CSI reconstruction part. The quantization codebook and the CSI reconstruction part are delivered to the UE side (e.g., via model transfer) after training/testing is finished vs. delivering the CSI generation part to UE side. At the UE side, once it receives the quantization codebook and the CSI reconstruction part, it derives a CSI codebook by using the vector quantization codebook entries as input to the CSI reconstruction part of the AI/ML model, which is referred as CSI look-up-table (LUT) later in the document. During the inference/testing phase, for each testing sample/input (e.g., eigenvectors of channel matrix), instead of using the CSI generation part and the quantizer to generate the CSI feedback at the UE side, UE selects an entry from the LUT (derived during the training phase described above) based on the estimated CSI and a selection criterion, e.g., SGCS used in our study. The LUT entry that has the highest SGCS will be selected as the CSI feedback, and the index of the selected LUT entry will be reported to gNB.

To Moderator, it is a new quantization or CSI compression behavior than what we have discussed previously. Therefore, the question is raised in below:

Question 3.2.10: For the evaluation of the quantization codebook, do you think the following quantization behavior can be considered for aligned EVM?

- Training: A look-up-table is generated based on the CSI reconstruction part trained at NW side and the corresponding quantization CB.

- Inference: UE quantizes the measured CSI to an entry of the look-up-table and sends the entry index to gNB.

| Company | View |
| --- | --- |
| Futurewei | We appreciate that FL included our proposed approach in the summary. We would like to first clarify our approach that the look up table (LUT) can be generated via any training type, not just limited to model trained at NW side even though we used training type 1 for simplicity in our study. The major difference between the proposed LUT-based approach and others is that this approach allows VQ to generate relatively smaller number of CSI overhead bits, e.g., 10-14 bits (per layer) while achieving relatively more robust performance compared to typical (non-LUT-based) CSI feedback generated using the CSI generation part of the two-sided model and a quantizer which can be part of the CSI generation part or separate. In our study, the LUT-based approach with 13 CSI overhead bits achieves better mean UPT and 5% UPT than Rel-16 Type II codebook-based approach with 10 times more overhead bits, e.g., 130 or 242 CSI bits. When comparing with using typical autoencoder-based approach to predict the CSI feedback, significant performance improvement is also observed, particularly in the samples with worst performance, e.g., the bottom 10-20%. This approach shows very promising initial results while it is relatively easy to realize and suitable for any agreed training type that leverages VQ (assuming the associated VQ codebook can be generated), thus, we encourage companies to consider incorporating this approach as part of their approaches and evaluating this approach further for its potential benefit in reducing CSI feedback overhead and performance gain. |
| vivo | We are fairly interested in Futurewei’s proposal. From our understanding, the mentioned method further compresses the latent space (i.e., the output of the CSI generation part). However, we have some questions about the method to be clarified: 1. How to derive the look-up-table based on the vector quantization codebook is critical. It is better for the proponent to provide more details. We guess part of the VQ codebook entries are picked up to be the look-up-table. 2. It seems that deriving the look-up-table can also be done at training entity, in this case, at NW. Is it correct? |
| Lenovo | We can understand how the LUT can work at the decoder side but for CSI generation part it is unclear how we can use the LUT. If out undesrtatnign si correct, we use LUT (at the UE) to determine the “feedback data” to be send to the gNB based on the “input CSI”. The question is that the “input CSI” is continuous value so how we want to use it for determine the index of LUT. So, how the indexing of the LUT should be. Is it based on a particular quantization of the “input CSI”. In the description of the method, it is stated that “UE selects an entry from the LUT (derived during the training phase described above) based on the estimated CSI and a selection criterion, e.g., SGCS used in our study. The issue here is that the UE does not know reconstructed CSI so it does not have access to the SGCS. It only have the “input CSI” and “latent representation of the input CSI” (feedback data), so it is not clear to us how the UE can select from the LUT based on SGCS. Some clarification can help here. Thanks. |
| Fujitsu | No need to have such a specific approach for the evaluation. |
| MediaTek | This scheme is practical only if the CSI can be compressed into few bits (14 bits in Futurewei’s evaluation). For higher number of bits, it is not feasible to store hundred thousand or billions of CSI samples and measure SGCS w.r.t. to each element of LUT. We have not seen enough evidence so far on feedback with less than 15bits which result a good performance. |

### 3.2-4: AI/ML related EVM

#### Issue#3-8 (Medium priority) rank>1 options-complexity report

Moderator note: In the last meeting, we have categorized 6 options for rank>1 options. There is a FFS issue for each option on how to report the complexity and storage. In this meeting, Huawei [3] and AT&T [29] propose the calculation of FLOPs and memory storage for each option. The following proposal is therefore given for the calculation of FLOPs in below:

Proposal 3.2.1: For the rank >1 options under AI/ML-based CSI compression, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, CATT, LG Electronics, Fraunhofer, Fujitsu, Apple, ZTE, NVIDA, MediaTek, Ericsson, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-9 (Medium priority) rank>1 options-storage report

Moderator note: Similar with issue#3-8, the following proposal is given for the calculation of memory storage:

Proposal 3.2.2: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, CATT, LG Electronics, Fujitsu, ZTE, NVIDIA, MediaTek, Ericsson, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-10 (Medium priority) CQI calculation method

Moderator note: In the last meeting, we have categorized the options for CSI calculation and report. In this meeting, some companies discussed and evaluated some specific options. Moderator does not observe necessity to align the EVM over companies for the CQI calculation methods, so it is proposed to allow companies to report the method to calculate the CQI.

Question 3.3.11: For the evaluation of CSI compression, do you think companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g.,

- Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

- Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder

- Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation

- Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment

- Option 1c: CQI is calculated based on traditional codebook

| Support/Can accept | Futurewei, vivo, CAICT,, OPPO, Xiaomi, CATT, LG Electronics, Lenovo (comment), Fujitsu, Apple, ZTE, NVIDIA, MediaTek, Ericsson, InterDigital, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Xiaomi | We think that each company can provide multiple evaluation results, e.g., multiple columns in the table with different CQI calculation methods. |
| Lenovo | We generally support this proposal with one bullet added as • Other options for reporting of CQI is also allowed. |
| Apple | This will help a lot to further the CQI discussion in 9.2.2.2 |
| ZTE | We think it is necessary to introduce additional field for companies to report the specific CQI determination method, since different CQI calculation methods would influence the eventual UPT performance. So, we suggest to prioritize this issue to capture the CQI evaluation results in the template for companies to compare. In addition, for Option 2a, we suggest categorizing it into two options as - Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. - Option 2a-2: CQI is calculated based on CSI reconstruction output assumed at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW. |
| Qualcomm | In practice, CQI and RI determination may be optimized in implementation and simulation heuristics and results may not entirely reflect that. |

#### Issue#3-11 (Medium priority) Report of PMI/RI calculation

Moderator note: In the last meeting, this issue was raised for discussion, yet very limited feedbacks were received. In this meeting, Apple [26] and Google [16] proposed to report the RI, CQI determination method. Therefore, the same question as the last meeting. Continue to collect views from companies as very limited feedbacks are received in the last meeting.

Question 3.2.12: For the evaluation of CSI compression, do you think it is needed to allow companies to report (e.g., by introducing an additional field in the template to describe) the specific implementations (e.g., RI/PMI/CQI determination, etc.) for benchmark (e.g., Type II CB) and AI/ML, separately?

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi(with comments), CATT, LG Electronics, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, AT&T |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Xiaomi | We think the implementations for benchmark is not necessary. Companies can report the RI calculation method for AI/ML together with CQI calculation method described in Issue#3-10. |
| Fujitsu | It is up to companies to decide whether they want to disclose such details. |
| Apple | This will help discussion in 9.2.2.2 on RI determination. In 9.2.2.2, there were agreement to further discuss RI determination. The discussion can not move forward without a clear under-standing of RI impact for AI and e-type II performance. Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination |
| ZTE | We should prioritize the evaluation of CQI determination compared with RI/PMI determination. |
| Qualcomm | The eventual result may depend on several other scheduler choices and heuristics. It would be difficult to agree upon and/or capture in the results table all the details. It can be left to companies to ensure that the benchmark and AI/ML schemes are compared in a proper manner. In practice, CQI and RI determination may be optimized in implementation and simulation heuristics and results may not entirely reflect that. |
| InterDigital | Agree with Xiaomi’s comment |

#### Issue#3-12 (Low priority) Per area/localized model

Moderator note: In this meeting, vivo [5] and Qualcomm [28] provide simulation results with per area/localized model for UMa and indoor respectively, where the channel model with spatial consistency is considered. From the simulation results, per area/localized model shows a good performance. Therefore, a question is raised in below:

Question 3.2.13: For the evaluation of the AI/ML based CSI compression, do you think besides the channel model as currently agreed, the channel model with spatial consistency needs to be taken into consideration to evaluate the performance of per area/localized model?

- FFS deployment scenario, e.g., UMa, indoor etc.

| Company | View |
| --- | --- |
| vivo | We are supportive of this proposal. We believe that the study of per area model can provide good insights on how to deploy models in practice. In our simulations, we see that channels with spatial consistency are a good and simple way to construct data samples within a specific area. |
| ETRI | If per-cell/localized models can bring much gains in CSI compression, we think that it would be helpful to study on this in the study item. For this purpose, the channel model with spatial consistency can optionally be adopted and reported by companies. |
| OPPO | We agree that to evaluate the per-region model, the spatial consistency needs to be considered. |
| CATT | Although this might be useful in some scenario, it seems going to double the simulation effort so far, e.g. area/localized model with all training collaboration types, M-to-N mapping… tend to be up to companies interest optionally, or consider how to limit the simulated cases. |
| Qualcomm | To develop a model specifically for use in a small region, it would be important to use the model for spatial consistency defined in TR 38.901 in order to ensure accuracy in the results of an AI/ML-based approach. |
| InterDigital | Enabling spatial consistency may be important when studying the performance of ML models within a localized area or region; however we are concerned of the potential increase in the number of simulations. Additionally, several issues need clarification, for example: how to generate localized training data using the current EVM agreements and how to define the (small) regions. |

### 3.2-5: Observations

#### Issue#3-13 (Medium priority) Basic performance for CSI compression-mean UPT

Moderator note: From the collected results for Table 1,

Huawei [3], Nokia [8], FUTUREWEI [2], Lenovo [27], ZTE [4], vivo [5], OPPO [6], ETRI [22], Spreadtrum [7], Fujitsu [13], NTT DOCOMO [30], CMCC [23] have provided results for FTP traffic with Max rank 1;

Huawei [3], Ericsson [14], Nokia [8], ZTE [4], OPPO [6], Intel [11], Fujitsu [13], NTT DOCOMO [30], InterDigital [12] have provided results for FTP traffic with Max rank 2;

Qualcomm [28], ZTE [4], CATT [10], Apple [26] have provided results for FTP traffic with Max rank 4;

Huawei [3], Nokia [8], vivo [5], OPPO [6], Fujitsu [13], China Telecom [40], MediaTek [24] have provided results for full buffer with Max rank 1;

Huawei [3], Xiaomi [15], Nokia [8], Qualcomm [28], vivo [5], Intel [11], Fujitsu [13], InterDigital [12] have provided results for full buffer with Max rank 2;

ZTE [4] has provided results for full buffer with Max rank 4;

The following observation is given as:

Upd Proposed Observation 3.2.3: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, compared to the benchmark of R16 Type II codebook, under FTP traffic and in terms of mean UPT:

- 6 sources [3][8][4][5][6][7] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=1, among which:

  - 3 sources [3][8][5] show 0.2%~3% gains for RU<=39%

  - 4 sources [3][8][5] show 0.1%~6% gains for RU 40%-69%

  - 1 source [7] shows 12.77%~21.21% gains for RU 40%-69%

  - 6 sources [3][8][4][6][5][7] show 0.23%~9% gains for RU >=70%

  - 32 sources [2][3][7] show 8.1%11.23%~31.9%15% gains for RU >=70%

- 8 sources [3][14][8][4][6][11][12][30] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=2, among which:

  - 1 source [8] shows 0.3% loss for RU<=39%

  - 4 sources [3][14][8][11] show 0.5%~10% gains for RU<=39%

  - 2 sources [30][12] show 11.5%~30% gains for RU<=39%

  - 1 source [8] shows 0.5% loss for RU 40%-69%

  - 5 sources [3][14][8][4][11] show 1%~10% gains for RU 40%-69%

  - 3 sources [3][30][12] show 16.3%~23% gains for RU 40%-69%

  - 1 source [8] shows 0.2% loss for RU >=70%

  - 7 sources [3][14][8][4][6][11][12] show 1%~10% gains for RU >=70%

  - 5 sources [3][8][6][11][30] show 11%~29% gains for RU >=70%

- 4 sources [28][4][10][26] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=4, among which:

  - 1 source [26] shows 4% loss for RU<=39%

  - 2 sources [28][10] show 2%~7.4% gains for RU<=39%

  - 1 source [26] shows 1.8% loss for RU 40%-69%

  - 2 sources [28][4] show 2%~12.2% gains for RU 40%-69%

  - 1 source [26] shows 1% loss for RU >=70%

  - 2 sources [28][4] show 4%~14.89% gains for RU >=70%

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, ZTE, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the range of gains, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Futurewei | We noticed that our results were not captured in the Proposed Observation 3.2.3. In our reported results for Max rank = 1 case, the Mean UPT gain is between 8.1 – 31.9% for RU>=70% (for SQ, the gain is between 8.1 – 21.5% and for VQ is the gain is 31.9% for X category using LUT-based approach). |
| Apple | For rank-4, we submitted results to <80 bits and >230 bits. Only the 2nd part is capture. Based on the results, we actually see a pattern that AI performance is relatively better in low UCI bit range. [28] has similar trend. We recommend to capture the results separately for each size. |
| Qualcomm | The full-buffer results have not been captured in the observation. It would be useful to capture those results also. |

#### Issue#3-13a (Medium priority) Basic performance for CSI compression-5% UPT

Moderator note: The following observation is given as:

Proposed Observation 3.2.4: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, compared to the benchmark of R16 Type II codebook, under FTP traffic and in terms of 5% UPT:

- 6 sources [3][8][4][5][6][7] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=1, among which:

  - 3 sources [3][8][5] shows 0.8%~3% gains for RU<=39%

  - 1 source [3] shows 4%0.1%~8%7% gains for RU<=39%

  - 3 sources [3][8][5] shows 0.1%~6% gains for RU 40%-69%

  - 2 sources [3][7] show 8%~21.04% gains for RU 40%-69%

  - 5 sources [3][8][4][5][6] show 0.85%~7% gains for for RU >=70%

  - 43 sources [2][3][8][7] show 8%~28% gains for for RU >=70%

  - 1 source [2] shows 49.6%~69.5% gains for RU >=70%

- 8 sources [3][14][8][4][6][11][12][30] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=2, among which:

  - 1 source [8] shows 0.5%~2% loss for RU<=39%

  - 4 sources [3][14][8][11][12] show 1.1%~9% gains for RU<=39%

  - 2 sources [30][11] show 16%~23.3% gains for RU<=39%

  - 2 source [8][12] show 2%~8% loss for RU 40%-69%

  - 5 sources [3][14][8][11][12] show 0.3%~8% gains for RU 40%-69%

  - 3 sources [3][4][11][30] show 9%~29.7% gains for RU 40%-69%

  - 2 sources [8][12] show 1.3%~10% loss for RU >=70%

  - 4 sources [3][14][8][4] show 2%~12% gains for RU >=70%

  - 5 sources [3][8][4][11][30] show 13%~39% gains for RU >=70%

- 3 sources [28][4][26] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=4, among which:

  - 2 sources [28][26] show 1%~1.6% loss for RU<=39%

  - 2 sources [28][26] show 3%~8% gains for RU<=39%

  - 1 source [26] shows 1.7% loss for RU 40%-69%

  - 3 sources [28][4][26] show 2%~11.9884% gains for RU 40%-69%

  - 4 sources [28][4][26] show 2%~20.72% gains for RU >=70%

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, ZTE(comments), Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the range of gains, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Futurewei | We noticed that our results were not captured in the Proposed Observation 3.2.4. In our reported results for Max rank = 1 case, the 5% UPT gain is between 22.6 – 69.5% for RU>=70% (for SQ, the gain is between 22.6 – 49.6% and for VQ is the gain is 69.5% for X category using LUT-based approach). |
| Apple | Suggest to capture the observation of low UCI bits and high UCI bits separately. |
| ZTE | For the case of AI/ML-based CSI compression outperforming the benchmark at Max rank=1, the last sub-bullet should add ZTE[4] since we also achieve the 20.43% 5% UPT gain, which satisfies the corresponding range. Therefore, we suggest rewording it as - 4 sources [3][8][7][4] show 8%~28% gains for for RU >=70% For the case of AI/ML-based CSI compression outperforming the benchmark at Max rank=4, according to our results, the 4th sub-bullet should be reworded as - 3 sources [28][4][26] show 2%~11.84% 11.98% gains for RU 40%-69% |

#### Issue#3-13b (Medium priority) Basic performance for CSI compression-input type

Moderator note:

Huawei [3], Nokia [8], FUTUREWEI [2], Lenovo [27], ZTE [4], vivo [5], OPPO [6], ETRI [22], Spreadtrum [7], Fujitsu [13], NTT DOCOMO [30], CMCC [23] have provided results for FTP traffic with Max rank 1, all with the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI, showing more gains over eigenvector of current CSI.

Huawei [3], Ericsson [14], Nokia [8], ZTE [4], OPPO [6], Intel [11], Fujitsu [13], NTT DOCOMO [30], InterDigital [12] have provided results for FTP traffic with Max rank 2, wherein

- Ericsson [14] adopts the input type of W2 matrix after eType-II processing

- Other sources adopt the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI as model input, showing more gains over eigenvector of current CSI (1%-6% additional gain over current CSI for rank 1, 1%-14% additional gain over current CSI for rank 2).

Qualcomm [28], ZTE [4], CATT [10], Apple [26] have provided results for FTP traffic with Max rank 4, all with the input type of precoding matrix.

Huawei [3], Nokia [8], vivo [5], OPPO [6], Fujitsu [13], China Telecom [40], MediaTek [24] have provided results for full buffer with Max rank 1, all with the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI as model input, showing more gains over eigenvector of current CSI.

Huawei [3], Xiaomi [15], Nokia [8], Qualcomm [28], vivo [5], Intel [11], Fujitsu [13], InterDigital [12] have provided results for full buffer with Max rank 2, wherein

- Ericsson [14] adopts the input type of W2 matrix after eType-II processing

- Other sources adopt the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI, showing more gains over eigenvector of current CSI.

ZTE [4] has provided results for full buffer with Max rank 4, all with the input type of precoding matrix.

The following observation is given as:

Proposed Observation 3.2.5: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, for the AI/ML model input (for CSI generation part) and output (for CSI reconstruction part):

- 21 sources [3][8][2][27][4][5][6][22][7][13][30][23][6][11][12][28][10][26][5][40][24] adopt precoding matrix as the AI/ML model input and output, wherein

  - 1 source [3] adopts the eigenvector(s) with additional past CSI as the model input, showing 1%-14% additional gain over the model input with the eigenvector(s) of current CSI.

- 1 source [14] adopts channel matrix as the model input/output

| Support/Can accept | Futurewei, vivo, Huawei/HiSi, Lenovo, Fujitsu, ZTE, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Xiaomi | Xiaomi[15] also adopt precoding matrix as the AI/ML model input and output. We noticed that there are two [5] among the 21 sources, so the total number may not need to change. |

#### Issue#3-13c (Medium priority) Basic performance for CSI compression-rank>1 options

Moderator note:

- Option 1-1 (rank specific): ZTE [4],

- Option 1-2 (rank common):

- Option 2-1: layer specific and rank common: Ericsson [14], Intel [11], Qualcomm [28],

- Option 2-2: layer specific and rank specific: Ericsson [14],

- Option 3-1: layer common and rank common: Huawei [3], CMCC [23], ZTE [4], OPPO [6], Fujitsu [13], vivo [5],

- Option 3-2: layer common and rank specific: NTT DOCOMO [30], InterDigital [12], Apple [26]

The following observation is given as:

Proposed Observation 3.2.6: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, for the rank>1 options:

- 1 source [4] evaluates Option 1-1 (rank specific).

- 3 sources [14][11][28] evaluate Option 2-1 (layer specific and rank common).

- 1 source [14] evaluates Option 2-2 (layer specific and rank specific).

- 6 sources [3][23][4][6][13][5] evaluate Option 3-1 (layer common and rank common).

- 3 sources [30][12][26] evaluate Option 3-2 (layer common and rank specific).

- Note: no source submits evaluation results based on Option 1-2 (rank common).

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, ZTE, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. Limited submitted results for the comparison over schemes, so no performance comparison and down selection performed over submitted options in the observation (except Option 1-2). |
| Xiaomi | Xiaomi [15] also adopt Option 3-1: layer common and rank common. |
| CATT | Our design of scalable transformer ([10]) can be categorized as Option 3-1. |

#### Issue#3-13d (Medium priority) Basic performance for CSI compression-quantization training

Moderator note: Huawei [3], Ericsson [14], vivo [5], Qualcomm [28], MediaTek [24], Nokia [8], Fujitsu [13], InterDigital [12], Comba [20] observe that quantization aware training shows clear performance gain compared to quantization non-aware training.

The following observation is given as:

Proposed Observation 3.2.7: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, 9 sources [3][14][5][28][24][8][13][12][20] show that quantization aware training (Case 2-1/2-2) provides clear performance gain compared to quantization non-aware training (Case 1).

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-13e (Low priority) Basic performance for CSI compression-quantization and scalability

Moderator note: MediaTek [24], Nokia [8], ETRI [22] observe that by adjusting the quantization granularity, it can also achieve the scalability of CSI payload size, without impacting the dimension of the AI/ML model.

For example, to achieve CSI payload size of L, 2L, …, Q*L bits, two approaches can be considered:

- Consider scalable dimension of the latent variable as L, 2L, …, Q*L bits

- Applying 1,…, Q bits quantization for latent variable’s dimension of fixed L bits.

The following observation is given as:

Proposed Observation 3.2.8: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, 3 sources [24][8][22] show that scalability on CSI feedback payload size can be achieved by either training a scalable AI/ML model with various latent space dimensions or applying various quantization granularities on a AI/ML model with non-scalable latent space dimension.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-13f (Low priority) Basic performance for CSI compression-VQ and SQ

Moderator note: vivo [5], Fujitsu [13], and FUTUREWEI [2] observe that VQ outperforms SQ under the same CSI payload size.

MediaTek [24], Nokia [8] observe that SQ outperforms VQ under the same CSI payload size.

Xiaomi [15] observe that SQ has comparable performance with VQ under the same CSI payload size.

The following observation is given as:

Proposed Observation 3.2.9: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, between the quantization methods of scalar quantization and vector quantization under the same CSI payload size:

- 3 sources [5][13][2] show that vector quantization outperforms scalar quantization.

- 2 sources [24][8] show that scalar quantization outperforms vector quantization.

- 1 source [15] shows that scalar quantization and vector quantization have comparable performance.

| Support/Can accept | Futurewei (see comments) , Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Futurewei | We are ok with the observation but we would like to clarify that in our study VQ is only used for CSI bits belong to X category and we observed that VQ performs better than SQ for X. |
| Lenovo | In our simulation we have used both SQ and VQ for generation of the feedback data. So we suggest to add: - 1 source [27] shows the possibility of using both scalar and vector quantization for generation of the feedback data. |
| MediaTek | We have also two critical observations regarding our evaluations: 1) VQ is very sensitive and cannot be used at CB or segment level. 2) VQ makes finetuning more challenging as it is very sensitive to its input distribution. We should note that these observations are tied to our specific implementation and we cannot generalize it to all VQ methods. |
| Qualcomm | The results presented in our contribution (in Table 2) indicate that VQ outperforms SQ for Case 2-2 quantization aware training, but we did not include it in the excel file results. Can it still be captured in this observation? |

#### Issue#3-14 (High priority) Generalization for CSI compression – deployment scenario

Moderator note: From the collected results for Table 2,

Mavenir [18], Huawei [3], Ericsson [14], FUTUREWEI [2], Lenovo [27], Intel [11], vivo [5], OPPO [6], Fujitsu [13], CMCC [23], CATT [10], Xiaomi [15], ZTE [4], NTT DOCOMO [30], InterDigital [12] submitted the results on the generalization over deployment scenarios (UMa/UMi/InH, etc.).

The following observation is given as:

Proposed Observation 3.2.10: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#112bis-e meeting,

- 15 sources [18][3][14][2][27][11][5][6][13][23][10][15][4][30][12] show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

  - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, deployment scenario#A is UMi, deployment scenario#B is UMa, or deployment scenario#A is InH, deployment scenario#B is UMa/UMi.

  - 6 sources [3][5][4][10][15][11] observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor.

    - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, or deployment scenario#A is UMi, deployment scenario#B is UMa.

    - 1 source [2] observes that the degradation under the above combinations is also significant.

- 5 sources [3][5][10][2][12] show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#A and deployment scenario#B, and the trained AI/ML model applies inference on either deployment scenario#A or deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa and/or UMi.

- 3 sources [3][15][4] show that, compared to the case where the AI/ML model is trained on scenario#A and applied for inference on deployment scenario#B, the generalization performance can be improved, if the AI/ML model, after trained on deployment scenario#A, is updated based on a fine-tuned dataset subject to deployment scenario#B, and performs inference on deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa or UMi.

| Support/Can accept | vivo, Huawei/HiSi, CMCC, Fujitsu, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Futurewei | Please remove “1 source [2] observes that the degradation under the above combinations is also significant.” There may be some mis-interpretation of our observations/results. |

#### Issue#3-15 (Medium priority) Generalization for CSI compression – carrier frequency

Moderator note: From the collected results for Table 2,

Ericsson [14], vivo [5], NTT DOCOMO [30] submitted the results on the generalization over carrier frequencies (2GHz, 3.5GHz, etc.).

The following observation is given as:

Proposed Observation 3.2.11: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#112bis-e meeting,

- 3 sources [14][5][30] show that compared to the case where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B, it has almost the same performance if the AI/ML model is trained with carrier frequency#A and applied for inference with a different carrier frequency#B.

  - E.g., carrier frequency#A is 2GHz/5.5GHz, carrier frequency#B is 3.5GHz, or carrier frequency#A is 3.5GHz, carrier frequency#B is 2GHz.

  - Note: same antenna layout is assumed for carrier frequency#A and carrier frequency#B.

| Support/Can accept | vivo, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-16 (Low priority) Scalability for CSI compression – Bandwidths

Moderator note: From the collected results for Table 3,

Ericsson [14], ZTE [4], NTT DOCOMO [30], InterDigital [12] submitted the results on the generalization over various bandwidths.

The following observation is given as:

Proposed Observation 3.2.12: For the scalability verification of AI/ML based CSI compression over various bandwidth values, till the RAN1#112bis-e meeting,

- 2 sources [14][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain bandwidth#B and applied for inference with a same bandwidth#B, it has almost the same performance if the model is trained with bandwidth#A and applied for inference with a different bandwidth#B, and an appropriate scalability solution is performed.

  - 2 sources [14][4] adapt the subband size according to bandwidth value without scaling the dimension of the AI/ML model as the scalability solution.

- 2 sources [30][12] shows that generalized performance of the AI/ML model can be achieved for the inference on either bandwidth#A or bandwidth#B, if the training dataset is constructed with data samples subject to multiple bandwidth values including bandwidth#A and bandwidth#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 2 sources [30][12] adopt pre/post-processing of truncation/padding as the scalability solution.

| Support/Can accept | ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-17 (Medium priority) Scalability for CSI compression – Tx ports

Moderator note: From the collected results for Table 3,

Huawei [3], OPPO [6], Fujitsu [13], CATT [10], ZTE [4], NTT DOCOMO [30] submitted the results on the generalization over various Tx port numbers.

The following observation is given as:

Proposed Observation 3.2.13: For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#112bis-e meeting,

- 3 sources [6][13][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B, it has degraded performance if the model is trained with Tx port number#A and applied for inference with a different Tx port number#B

  - E.g., Tx port number#A is 32, Tx port number#B is 16.

- 6 sources [3][6][13][10][4][30] show that generalized performance of the AI/ML model can be achieved for the inference on either Tx port number#A or Tx port number#B, if the training dataset is constructed with data samples subject to multiple Tx port numbers including Tx port number#A and Tx port number#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 5 sources [3][6][13][4][30] adopt pre/post-processing of truncation/padding as the scalability solution.

  - 1 source [10] adopts adaptation layer in the AL/ML model as the scalability solution.

| Support/Can accept | Huawei/HiSi, Fujitsu, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-18 (High priority) Scalability for CSI compression – CSI payload sizes

Moderator note: From the collected results for Table 3,

Huawei [3], Ericsson [14], OPPO [6], Fujitsu [13], CMCC [23], CATT [10], MediaTek [24], NTT DOCOMO [30] submitted the results on the generalization over various CSI payload sizes.

The following observation is given as:

Proposed Observation 3.2.14: For the scalability verification of AI/ML based CSI compression over various CSI payload sizes, till the RAN1#112bis-e meeting,

- 2 sources [6][13] show that compared to the case where the AI/ML model is trained with dataset subject to a certain CSI payload size#B and applied for inference with a same CSI payload size#B, it has degraded performance if the model is trained with CSI payload size#A and applied for inference with a different CSI payload size#B.

- 8 sources [3][14][6][13][23][10][24][30] show that generalized performance of the AI/ML model can be achieved for the inference on either CSI payload size#A or CSI payload size#B, if the training dataset is constructed with data samples subject to multiple CSI payload sizes including CSI payload size#A and CSI payload size#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 3 sources [6][13][23] adopt pre/post-processing of truncation/padding as the scalability solution.

  - 1 source [14] adopt various quantization granularities as the scalability solution.

  - 4 sources [3][10][24][30] adopt adaptation layer in the AL/ML model as the scalability solution.

| Support/Can accept | vivo, Huawei/HiSi, CMCC, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-19 (On hold) Scalability for CSI compression – Rank>1 options

Moderator note: From the collected results for Table 3,

ZTE [4], CATT [10], MediaTek [24], NTT DOCOMO [30] submitted the results on the generalization over various rank/layer adaptation methods.

The following observation is given as:

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-20 (On hold) Multi-vendor joint training for CSI compression

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-21 (On hold) Separate training for CSI compression

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

### 3.2-6: Others

Question 3.2.14: For the evaluation of the AI/ML based CSI compression sub use cases, what other aspects related with evaluations or observations do you think is necessary to be discussed?

| Company | View |
| --- | --- |

## 2nd round email discussions

#### Issue#3-1 (Medium priority) Boundary between Type 2 and Type 3

Moderator note: There seems to be majority of companies who are in favor of categorizing the “Freeze-and-train” behavior to Type 2, yet Qualcomm has strong concern on that. It is Moderator’s understanding that this behavior is a special implementation for an already agreed Type other than a standalone training mode, so it is not desired to use a new Type to name it.

For Qualcomm’s concern on the “example” for Type 3, it is Moderator’s understanding that although the dataset sharing method is provided as an example, this is the major case that has been evaluated by companies who submit Type 3 results, and we have agreement to elaborate this behavior.

#111 Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2 - For NW-first training - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - For UE-first training - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

As we did not achieve consensus at the first round, following proposal is given to see if we can further clarify the boundary between Type 2 and Type 3:

Proposal 3.3.1: For the evaluation of training types of CSI compression, for the freeze-and-train behavior of:

- Step A: first side is trained, and after the training for the first side is finished - Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

Categorize this behavior as a special behavior of Type 2 (namely Type 2-alt), and clarify

- For the procedure of Type 2 in the 110bis-e conclusion: either Step 3 or “trains the CSI reconstruction part” in Step 2 is needed under Type 2-alt.

- For the procedure of Type 3 in the 110bis-e conclusion: the “a set of information (e.g., dataset)” in Step 2 is the dataset shared from UE to NW for UE first training, and dataset shared from NW to UE for NW first training.

| Support/Can accept | Intel (as conclusion), Fujitsu, CMCC |
| --- | --- |
| Object/Concern | NTT DOCOMO |

| Company | View |
| --- | --- |
| Moderator | 110bis-e conclusions for reference For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example: - For each FP/BP loop, - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side - Step 3: UE side trains the CSI generation part based on the BP information from NW side - Note: the dataset between UE side and NW side is aligned. - Other Type 2 training approaches are not precluded and reported by companies. Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies. Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training): - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly - Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part - Step3: NW side trains the NW side CSI reconstruction part based on the received set of information - Other Type 3 UE-first training approaches are not precluded and reported by companies. |
| Intel | We are fine to accept the proposal as a conclusion similar to other examples of training procedure. |
| NTT DOCOMO | We cannot accept calling this training as type 2. It was agreed to deprioritize type 2 training in other aspect agenda. If this training procedure is treated as type 2 training, it is deprioritized even though it has different pros and cons from the regular type 2 training. Also, this training procedure was not taken into account when discussing the prioritization/deprioritization of type 2. It is unfair to call it type 2 training procedure. |

#### Issue#3-2 (closed) Extendibility evaluation

Moderator note: As a number of companies have concern on the necessity of this simulation case, this issue is closed in this meeting.

#### Issue#3-3 (Medium priority) Modeling of various datasets from different UE types

Moderator note: In the first round, some companies hold the view that this can be represented by generalization. However, it is Moderator’s understanding that the UE type with respect to diverse UE algorithms/implementations is a separate perspective to impact the generalization, rather than what we have agreed for scenarios, carrier frequencies, etc., which are mainly decided by the network/gNB.

The question is updated according to the comments, and continue to discuss in this round.

Question 3.3.1: For CSI compression evaluation, for the modeling of the variation among dataset distributions over different UE types (e.g., different UE vendors/UE versions of a single vendor), consider the following aspects

- Different deployment scenarios, e.g., UE#1 with UMa while UE#2 with UMi, or UE#1 with LOS while UE#2 with O2I.

- Different antenna deployments, e.g., UE#1 with (1,1,2,1,1,1,1) while UE#2 with (1,2,1,1,1,1,1) for 2Rx.

- Different antenna gain imbalance values.

- FFS Phase ambiguity associated with the SVD operation

- Other factors are not precluded.

| Support/Can accept | Futurewei, vivo, OPPO, Xiaomi(with comments) , Spreadtrum, LG Electronics, Fraunhofer, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, Qualcomm (comments), InterDigital, AT&T, NTT DOCOMO, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Qualcomm: the phase ambiguity can be compensated by phase rotation? Would that be a problem deteriorating the quality of data samples? |

#### Issue#3-4 (Cloased) Other Type 3 training methods

Moderator note: As a number of companies believe these options can be optionally reported, they are not further discussed in this meeting. As long as there more than two companies submitting the results, we will come back and define the procedures of these options.

#### Issue#3-5 (High priority) Methodology-high level principle

Moderator note: Updates according to companies’ comments. Continue the discussion in this round.

BTW an example is given in below to elaborate how to model the monitoring.

- KPIGenie: The genie-aided SGCS between the recovery CSI and the ground-truth CSI. E.g., assume it is 0.8.

- KPIActual: The monitored SGCS using a specific monitoring method, e.g., Cases in Issue#3-6. Due to the robustness/quantization of the monitoring method, bias to the genie-aided SGCS may occur. E.g., it can be 0.75 (lower than KPIGenie) or 0.83 (higher than KPIGenie).

- KPIGap: The gap between genie-aided SGCS and monitored SGCS.

- By the statistic of KPIGap for all test samples, we can see how close it is between monitored SGCS and the genie-aided SGCS (i.e., monitoring accuracy), e.g., in terms of the percentage of |KPIGap| smaller than a threshold; other metrics see Issue#3-5b.

Proposal 3.3.2: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, the methodology is can be considered in terms of the statistical gap of monitored intermediate KPI:

- Step1: Generate test dataset including K test samples

  - FFS how to obtain the K test samples, e.g., K test samples are time-related samples

- Step2: For each K test sample, the gap of monitored intermediate KPI (KPIGap) is calculated as a function of KPIGap= f(KPIActual, KPIGenie) (e.g., KPIGap= KPIActual-KPIGenie in case of SGCS), where KPIActual is the actual intermediate KPI (e.g., actual SGCS), and KPIGenie is the genie-aided intermediate KPI (e.g., genie-aided SGCS).

- Step3: Calculate the statistical result of the KPIGap over K test samples which represents the monitoring accuracy performance.

- Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network.

| Support/Can accept | Futurewei (with comments), vivo (with comments), CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, CATT(with comments), Fraunhofer, Lenovo (comment), Fujitsu, NVIDIA, NTT DOCOMO |
| --- | --- |
| Object/Concern | Ericsson, Qualcomm (comments) |

| Company | View |
| --- | --- |
| Moderator | Please the “Object/Concern” companies see if the updates/clarifications can comfort you to accept the proposal? @ Futurewei see updates. @vivo “time-related samples” reflected as FFS under Step 1 (spatial consistency not reflected since it is not in EVM table). For “make decisions on model switching or fallback” – how to make EVM and derive the results as quantitative values? @ETRI @ InterDigital “genie-aided SGCS” is the SGCS between the recovery CSI and the genie-aided ground-truth CSI (without any quantization). It is the SGCS value reported in our inference performance evaluation results; but as it is End-to-end result, realistically neither NW and UE can obtain it (unless for Type 1 where one side have both models). @LG Clarification: In the next question, Case 1, higher resolution ground-truth CSI (but have some quantization error) is employed as actual KPI. Perfect ground truth CSI is used to generate genie-aided KPI @Lenovo see Issue#3-5c @Apple For Case 1, yes. See Issue#3-5a @ZTE see Issue#3-5b @Intel do not understand your proposed change. Elaborations are appreciated. What does a and b mean? What if a=b but both cause bias over the genie-aided KPI? In addition, only counting the number cannot reflect the variation of the bias, right? @Ericsson Intermediate KPI is not only for monitoring the system performance, but also for monitoring the accuracy of the model, right? If the performance is good/bad, how can I identify it attributes to the AI model? @ Qualcomm This question is about the high level methodology; if you have comments on the solution of monitoring, please comments under Issue#3-5a. Absolute KPI value is also included, see Issue#3-5b Option 1. |
| Intel | Please find the below figure which clarifies our proposal. Basically, metrics ¦a-b¦/K defines number of samples for which wrong decision is done for the model performance monitoring assuming that threshold T is the same for all the samples. This methodology is not ideal as well since T can be determined separately per sample (e.g., by comparison of SGCS with eType II PMI), but it better represents the impact on system performance than KPIGap in our view. Proposal: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, the methodology can be considered in terms of the statistical gap of monitored intermediate KPI: - Step1: Generate test dataset including K test samples - Step2: For each K test sample, the gap of monitored intermediate KPIs KPIGenie and KPIActual (KPIGap) is are calculated as a function of KPIGap= f(KPIActual, KPIGenie) (e.g., KPIGap= KPIActual-KPIGenie in case of SGCS), where KPIActual is the actual intermediate KPI (e.g., actual SGCS), and KPIGenie is the genie-aided intermediate KPI (e.g., genie-aided SGCS). - Step3: Calculate monitoring accuracy measured by ¦a-b¦/K, where a and b are the number of samples with KPIGenie and KPIActual larger or lower than a threshold T. |

#### Issue#3-5a (High priority) Methodology-per sample gap

Moderator note: Elaboration of the first issue on Step2. The methods of NW side monitoring and UE side monitoring are summarized as two cases. Proponent companies may also raise other cases for evaluation.

Proposal 3.3.3: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, for Step2, the per sample KPIGap is considered for

- Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar Float16, R16 Type II-like method, etc.), where

  - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format.

  - KPIGenie is calculated with the same output CSI and the ground-truth CSI of Float32.

  - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100%.

- Case 2: UE side monitoring of intermediate KPI with a proxy model CSI reconstruction part, where the monitoring accuracy is evaluated for the output CSI of the proxy model CSI reconstruction part at UE:

  - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated with the output CSI of the proxy CSI reconstruction part at UE and the ground-truth CSI.

  - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual)

  - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI.

  - FFS how to train the proxy model CSI reconstruction part and the resulting monitoring performance.

  - FFS whether/how to evaluate the generalization performance of the proxy model.

- Case 3: others

- Note: the overhead and latency are reported by companies.

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, LG Electronics, Fraunhofer, Lenovo (comment), ZTE, NVIDIA, InterDigital, Fujitsu |
| --- | --- |
| Object/Concern | Apple (comment), Intel, Qualcomm |

| Company | View |
| --- | --- |
| Moderator | Please the “Object/Concern” companies see if the updates/clarifications can comfort you to accept the proposal? @vivo see updates @OPPO Agree. The performance is reported up to companies by using the developed proxy model. In addition, the generalization of the proxy model may not be equal to the real decoder, which means when scenario changes, the real decoder is still robust while proxy model fails (false alarm). That is why a FFS “whether/how to evaluate the generalization performance of the proxy model” is added. @Xiaomi it is calculated at the UE side, so UE can use whatever format (by default Float32) @CATT Then the accuracy is logically 100%? Do we still need to run the simulation? A note is added accordingly. @ Fujitsu updated accordingly @Intel We can evaluate to justify the feasibility and performance to facilitate the discussions at 9222, right? @Apple How to train the proxy model is further discussed in the FFS part. Reusing it to calculate CQI/RI should be a separate issue? @Qualcomm Sorry for misunderstanding your solution. See updated Case 2-2; delay aspect please see the note part. |
| Intel | I agree with FL comment: evaluations can be done in 9.2.2.1. However, our comment is that separate agreement is not necessary since companies can bring evaluation results directly based on agreement in 9.2.2.2. We believe that the discussion in 9.2.2.2. is not finished yet – there are other proposals for the model performance monitoring. We proposed NW-based model performance monitoring which uses SRS for actual KPI determination and it was also captured under FFS in the last meeting. Given that we are now discussing the same thing as in 9.2.2.2, we propose the following changes (in green). Otherwise, we need to go back to 9.2.2.2 and finalize the list of options for further study. Proposal: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, for Step2, the per sample KPIGap is considered for - Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar Float16, R16 Type II-like method, etc.) or SRS measurements, where - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements. - KPIGenie is calculated with the same output CSI and the ground-truth CSI of Float32. - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100%. - Case 2: UE side monitoring of intermediate KPI with a proxy model CSI reconstruction part, where the monitoring accuracy is evaluated for the output CSI of the proxy model CSI reconstruction part at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated with the output CSI of the proxy CSI reconstruction part at UE and the ground-truth CSI. - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual) - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI. - FFS how to train the proxy model CSI reconstruction part and the resulting monitoring performance. - FFS whether/how to evaluate the generalization performance of the proxy model. - Case 3: others - Note: the overhead and latency are reported by companies. |

#### Issue#3-5b (Medium priority) Methodology-statistical result

Moderator note: Limited inputs in the last round. Continue discussion at this round. Priority set to “Medium”.

Proposal 3.3.4: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, for Step3, the statistical result of KPIGap over K test samples is considered in forms of

- Option 1: Monitoring accuracy: The percentage of the samples for which KPIGap<KPIth, where KPIth is a threshold of the intermediate KPI.

- FFS Option 2: Misdetection: The percentage of the samples for which KPIGap>KPIth_pos, where KPIth_pos is a second threshold of the intermediate KPI.

- FFS Option 3: False alarm: The percentage of the samples for which KPIGap<KPIth_neg, where KPIth_neg is a third threshold of the intermediate KPI.

- Option 4: other metrics.

- FFS the values of KPIth, KPIth_pos, and KPIth_neg.

- FFS the report of the options/combination of options

| Support/Can accept | Huawei/HiSi, OPPO, Fraunhofer, Lenovo (comment), Futurewei |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Moderator | @OPPO the bullet is self-explanatory on the definition of misdetection/false alarm. To elaborate: The physical meaning of misdetection is: the genie-aided SGCS is lower than the SGCS tolerance of the network/UE, but the monitored SGCS is higher than the genie-aided SGCS and also above the SGCS tolerance (so that NW/UE wrongly ignores the model failure). The physical meaning of false alarm is: the genie-aided SGCS is higher than the SGCS tolerance of the network/UE, but the monitored SGCS is lower than the genie-aided SGCS and also below the SGCS tolerance (so that NW/UE alarms the model failure that actually does not exist). @Lenovo see the updated FFS. For the curves, the intention of this proposal is to make quantitative values rather than curves – otherwise how to compare over companies? @ZTE: How to judge what absolute value is good, what absolute value is bad? The judgment of the specific value of the SGCS for performance should be Network/UE implementation, right? @Qualcomm Option 2/3 are moved to FFS. |
| Intel | In our view Issue#3-5 shall be discussed first. |

#### Issue#3-5c (Medium priority) Methodology-scenario change

Moderator note: Limited inputs in the last round. Continue discussion at this round. Priority set to “Low”.

Proposal 3.3.5: To evaluate the monitoring accuracy of the intermediate KPI for CSI compression, if the methodology in issue#3-5 is considered, the procedure of Step1-Step3 is performed for the same model(s) and monitoring method over various scenarios/configurations (e.g., Uma/Umi/InH, antenna layouts, etc.) to record the statistical result of KPIGap separately.

- FFS the set of various scenarios.

| Support/Can accept | Huawei/HiSi, Xiaomi, Fraunhofer |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @vivo@Xiaomi the generation of K test samples see changes on Issue#3-5. @Lenovo: the current proposal already capture these two cases, right? As companies will separately report multiple scenarios, you can see the monitoring accuracy for a single scenario, or compare them over different scenarios. @Fujitsu: updated @Qualcomm same model and same monitoring method, repeated over scenarios. No model retraining. |

#### Issue#3-6 (Medium priority) Quantization loss/quantization upper bound

Moderator note: Limited inputs in the last round. Continue discussion at this round. For proponents: please provide your reason (not simply “support”) on the necessity of introducing this new case to convince the opponents.

Question 3.3.2: For the evaluation of quantization aware/non-aware training, on top of the agreed cases of quantization non-aware training (Case 1) and quantization aware training (Case 2-1, Case 2-2), do you think it is needed to introduce an additional upper bound case of Case 0: non-quantized training and inference (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training and inference), to reflect the performance loss due to quantization?

| Support/Can accept | Futurewei, vivo, Xiaomi, Ericsson, InterDigital, AT&T, CMCC |
| --- | --- |
| Object/Concern | OPPO, Lenovo, ZTE |

| Company | View |
| --- | --- |
| Futurewei | We agree to consider evaluating performance loss due to quantization. |
| OPPO | It is not necessary to evaluate this upper bound. |
| Xiaomi | We support to evaluate the upper bound case. |
| CATT | OK if it is optional. |
| Lenovo | It is good to have an upper-bound value but we believe the upper-bound assuming that the gNB has access to the correct CSI form a genie. So, for now we do not see the value of evaluating the method using “no quantization” scheme. |
| Fujitsu | The payload size in case 0 is not clear. How to make a fair comparison of the performance of the two existing cases and the new case 0? |
| ZTE | It is not necessary to add an additional upper bound case of non-quantized training and inference, since quantization is mandatory in the actual communication system. |

#### Issue#3-7 (Low priority) Model represented by quantization/look up table

Moderator note: Limited inputs in the last round. Continue discussion at this round.

Question 3.3.3: For the evaluation of the quantization codebook, do you think the following quantization behavior can be considered for aligned EVM?

- Training: A look-up-table is generated based on the CSI reconstruction part trained at NW side and the corresponding quantization CB.

- Inference: UE quantizes the measured CSI to an entry of the look-up-table and sends the entry index to gNB.

| Company | View |
| --- | --- |
| Moderator | @Futurewei please provide the changes to the proposal in your replies, if you think the current description is not accurate enough. |
| Futurewei | We appreciate that FL included our proposed approach in the summary. We would like to first clarify our approach that the look up table (LUT) can be generated via any training type, not just limited to model trained at NW side even though we used training type 1 for simplicity in our study. The major difference between the proposed LUT-based approach and others is that this approach allows VQ to generate relatively smaller number of CSI overhead bits, e.g., 10-14 bits (per layer) while achieving relatively more robust performance compared to typical (non-LUT-based) CSI feedback generated using the CSI generation part of the two-sided model and a quantizer which can be part of the CSI generation part or separate. In our study, the LUT-based approach with 13 CSI overhead bits achieves better mean UPT and 5% UPT than Rel-16 Type II codebook-based approach with 10 times more overhead bits, e.g., 130 or 242 CSI bits. When comparing with using typical autoencoder-based approach to predict the CSI feedback, significant performance improvement is also observed, particularly in the samples with worst performance, e.g., the bottom 10-20%. This approach shows very promising initial results while it is relatively easy to realize and suitable for any agreed training type that leverages VQ (assuming the associated VQ codebook can be generated), thus, we encourage companies to consider incorporating this approach as part of their approaches and evaluating this approach further for its potential benefit in reducing CSI feedback overhead and performance gain. |
| vivo | We are fairly interested in Futurewei’s proposal. From our understanding, the mentioned method further compresses the latent space (i.e., the output of the CSI generation part). However, we have some questions about the method to be clarified: 1. How to derive the look-up-table based on the vector quantization codebook is critical. It is better for the proponent to provide more details. We guess part of the VQ codebook entries are picked up to be the look-up-table. 2. It seems that deriving the look-up-table can also be done at training entity, in this case, at NW. Is it correct? |
| Lenovo | We can understand how the LUT can work at the decoder side but for CSI generation part it is unclear how we can use the LUT. If out undesrtatnign si correct, we use LUT (at the UE) to determine the “feedback data” to be send to the gNB based on the “input CSI”. The question is that the “input CSI” is continuous value so how we want to use it for determine the index of LUT. So, how the indexing of the LUT should be. Is it based on a particular quantization of the “input CSI”. In the description of the method, it is stated that “UE selects an entry from the LUT (derived during the training phase described above) based on the estimated CSI and a selection criterion, e.g., SGCS used in our study. The issue here is that the UE does not know reconstructed CSI so it does not have access to the SGCS. It only have the “input CSI” and “latent representation of the input CSI” (feedback data), so it is not clear to us how the UE can select from the LUT based on SGCS. Some clarification can help here. Thanks. |
| Fujitsu | No need to have such a specific approach for the evaluation. |
| MediaTek | This scheme is practical only if the CSI can be compressed into few bits (14 bits in Futurewei’s evaluation). For higher number of bits, it is not feasible to store hundred thousand or billions of CSI samples and measure SGCS w.r.t. to each element of LUT. We have not seen enough evidence so far on feedback with less than 15bits which result a good performance. |
| Futurewei | We appreciate feedbacks/questions from companies. We would like to clarify a few questions raised by companies. Q1 (from vivo): How to derive the look-up-table based on the vector quantization codebook is critical. It is better for the proponent to provide more details. We guess part of the VQ codebook entries are picked up to be the look-up-table. R1: The LUT can be derived by using the VQ codebook entries (which are a set of real/floating-number vectors) as input to the decoder or CSI reconstruction part. LUT can be generated by either side. If it is generated by the training entity of the CSI reconstruction part then be delivered to the other entity (i.e., the CSI generation part/UE side). If it is generated by the training entity of the CSI generation side together with the training/learning of VQ codebook, only the VQ codebook needs to be delivered to the other entity (i.e., the CSI reconstruction part/NW side). Q2 (from vivo): It seems that deriving the look-up-table can also be done at training entity, in this case, at NW. Is it correct? R2: This is one option while it can also be generated at UE side from the VQ codebook (assuming VQ codebook is shared between entities) and the CSI reconstruction side (if available). Q3 (from Lenovo): The question is that the “input CSI” is continuous value so how we want to use it for determine the index of LUT. So, how the indexing of the LUT should be. Is it based on a particular quantization of the “input CSI”. R3: LUT is constructed by using VQ codebook entries as input to the CSI reconstruction part, thus, it consists of a set (e.g., N, which is determined by the VQ codebook size) of channels (eigenvectors or channel matrix). During the inference, UE side determines the best LUT entry to feedback to NW side based on SGCS (or other criterion) between the estimated channel and the entries in the LUT. Q4 (from Fujitsu): No need to have such a specific approach for the evaluation. R4: We think this method can be integrated into existing agreed-upon training options as long as vector quantization mechanism is leveraged. The only difference is to use LUT to determine the CSI feedback vs. based on encoder + vector quantizer (or VQ codebook). Regarding MediaTek’s concern, we understand it may have storage concern, thus we think using lower number of VQ codebook size will help, e.g., 10-14 bits. Even though the storage at the CSI generation part/UE side may still be significant, the benefit is that the CSI overhead reduction can be much higher. For companies who adopt vector quantization method to generate the final CSI feedback, using LUT-based method will outperform using CSI generation + VQ (this is the performance lower-bound compared to LUT). We understand it is early to say this method will definitely have better performance compared to Type II codebook or scalar quantization when using small VQ codebook size(s). Based on our study, using ~13-14 bits has shown promising results as it improves the tail performance significantly compared to typical “CSI generation + VQ” method. We encourage companies to try and evaluate it, so we can have more robust CSI feedback compression solution. |

#### Issue#3-8 (Medium priority) rank>1 options-complexity report

Moderator note: Seems relatively stable for the first round. To be presented on GTW if no further comments at the second round.

Proposal 3.3.6: For the rank >1 options under AI/ML-based CSI compression, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, CATT, LG Electronics, Fraunhofer, Fujitsu, Apple, ZTE, NVIDA, MediaTek, Ericsson, AT&T, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-9 (Medium priority) rank>1 options-storage report

Moderator note: Seems relatively stable for the first round. To be presented on GTW if no further comments at the second round.

Proposal 3.3.7: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, CATT, LG Electronics, Fujitsu, ZTE, NVIDIA, MediaTek, Ericsson, AT&T, NTT DOCOMO, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-10 (Medium priority) CQI calculation method

Moderator note: Updated from the comments of companies.

Proposal 3.3.8: For the evaluation of CSI compression, do you think companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g.,

- Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

  - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same to the actual CSI reconstruction part at the NW

  - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE different from the actual CSI reconstruction part at the NW

- Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder

- Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation

- Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment

- Option 1c: CQI is calculated based on traditional codebook

- Other options if adopted, to be described by companies

| Support/Can accept | Futurewei, vivo, CAICT,, OPPO, Xiaomi, CATT, LG Electronics, Lenovo (comment), Fujitsu, Apple, ZTE, NVIDIA, MediaTek, Ericsson, InterDigital, AT&T, NTT DOCOMO, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Fujitsu | Support in principle. We suggest the following wording for Option 2a-2: Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW. |
| Futurewei | We are ok with Fujitsu’s updates. We suggest minor wording change: Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW. |

#### Issue#3-11 (Medium priority) Report of PMI/RI calculation

Moderator note: Seems to have different voices in the first round. Continue the discussion at the second round, and please proponent companies provide reasons to convince Xiaomi, Qualcomm, and InterDigital.

Question 3.3.4: For the evaluation of CSI compression, do you think it is needed to allow companies to report (e.g., by introducing an additional field in the template to describe) the specific implementations (e.g., RI/PMI/CQI determination, etc.) for benchmark (e.g., Type II CB) and AI/ML, separately?

| Support/Can accept | Futurewei, vivo, ETRI, CAICT, Huawei/HiSi, OPPO, Xiaomi(with comments), CATT, LG Electronics, Lenovo, Apple, NVIDIA, MediaTek, Ericsson, AT&T, CMCC |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Xiaomi | We think the implementations for benchmark is not necessary. Companies can report the RI calculation method for AI/ML together with CQI calculation method described in Issue#3-10. |
| Fujitsu | It is up to companies to decide whether they want to disclose such details. |
| Apple | This will help discussion in 9.2.2.2 on RI determination. In 9.2.2.2, there were agreement to further discuss RI determination. The discussion can not move forward without a clear under-standing of RI impact for AI and e-type II performance. Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination |
| ZTE | We should prioritize the evaluation of CQI determination compared with RI/PMI determination. |
| Qualcomm | The eventual result may depend on several other scheduler choices and heuristics. It would be difficult to agree upon and/or capture in the results table all the details. It can be left to companies to ensure that the benchmark and AI/ML schemes are compared in a proper manner. In practice, CQI and RI determination may be optimized in implementation and simulation heuristics and results may not entirely reflect that. |
| InterDigital | Agree with Xiaomi’s comment |
| CMCC | It is up to companies to report this implementation information or not. |

#### Issue#3-12 (Low priority) Per area/localized model

Moderator note: Limited feedback in the first round. Continue discussion at this round.

Question 3.3.5: For the evaluation of the AI/ML based CSI compression, do you think besides the channel model as currently agreed, the channel model with spatial consistency needs to be taken into consideration to evaluate the performance of per area/localized model?

- FFS deployment scenario, e.g., UMa, indoor etc.

| Company | View |
| --- | --- |
| Moderator | Please proponent companies provide how to model the spatial consistency (procedure A/B, whether/how UE trajectory is modeled), and the deployment scenario (UMa, indoor?) |
| vivo | We are supportive of this proposal. We believe that the study of per area model can provide good insights on how to deploy models in practice. In our simulations, we see that channels with spatial consistency are a good and simple way to construct data samples within a specific area. |
| ETRI | If per-cell/localized models can bring much gains in CSI compression, we think that it would be helpful to study on this in the study item. For this purpose, the channel model with spatial consistency can optionally be adopted and reported by companies. |
| OPPO | We agree that to evaluate the per-region model, the spatial consistency needs to be considered. |
| CATT | Although this might be useful in some scenario, it seems going to double the simulation effort so far, e.g. area/localized model with all training collaboration types, M-to-N mapping… tend to be up to companies interest optionally, or consider how to limit the simulated cases. |
| Qualcomm | To develop a model specifically for use in a small region, it would be important to use the model for spatial consistency defined in TR 38.901 in order to ensure accuracy in the results of an AI/ML-based approach. |
| InterDigital | Enabling spatial consistency may be important when studying the performance of ML models within a localized area or region; however we are concerned of the potential increase in the number of simulations. Additionally, several issues need clarification, for example: how to generate localized training data using the current EVM agreements and how to define the (small) regions. |

#### Issue#3-13 (Medium priority) Basic performance for CSI compression-mean UPT

Moderator note: Updated to provide results for per CSI payload basis. Note that as the exact ranges for CSI overhead is still discussing.

Huawei [3], Nokia [8], FUTUREWEI [2], Lenovo [27], ZTE [4], vivo [5], OPPO [6], ETRI [22], Spreadtrum [7], Fujitsu [13], NTT DOCOMO [30], CMCC [23] have provided results for FTP traffic with Max rank 1;

Huawei [3], Ericsson [14], Nokia [8], ZTE [4], OPPO [6], Intel [11], Fujitsu [13], NTT DOCOMO [30], InterDigital [12] have provided results for FTP traffic with Max rank 2;

Qualcomm [28], ZTE [4], CATT [10], Apple [26] have provided results for FTP traffic with Max rank 4;

Huawei [3], Nokia [8], vivo [5], OPPO [6], Fujitsu [13], China Telecom [40], MediaTek [24] have provided results for full buffer with Max rank 1;

Huawei [3], Xiaomi [15], Nokia [8], Qualcomm [28], vivo [5], Intel [11], Fujitsu [13], InterDigital [12] have provided results for full buffer with Max rank 2;

ZTE [4] has provided results for full buffer with Max rank 4;

Proposed Observation 3.3.9: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, compared to the benchmark of R16 Type II codebook, under FTP traffic and in terms of mean UPT:

- 7 sources [2][3][8][4][5][6][7] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=1, among which:

  - 3 sources [3][8][5] show 0.29%~3% gains for [Low CSI overhead] RU<=39%

  - 3 sources [3][8][5] show 0.2%~1% gains for [Medium CSI overhead] RU<=39%

  - 3 sources [3][8][5] show 0.33%~2% gains for [High CSI overhead] RU<=39%

  - 3 sources [3][8][5] show 1.09%~6% gains for [Low CSI overhead] RU 40%-69%

  - 1 source [7] shows 12.77% gains for [Low CSI overhead] RU 40%-69%

  - 1 source [7] shows 15.18% gains for [Medium CSI overhead] RU 40%-69%

  - 1 source [7] shows 21.21% gains for [High CSI overhead] RU 40%-69%

  - 6 sources [3][8][4][5][6][7] show 0.38%~15% gains for [Low CSI overhead] RU >=70%

  - 1 source [2] shows 31.9% gains for [Low CSI overhead] RU >=70%

  - 5 sources [3][8][4][5][6] show 0.62%~8% gains for [Medium CSI overhead] RU >=70%

  - 2 sources [2][7] show 11.23%~15.8% gains for [Medium CSI overhead] RU >=70%

  - 5 sources [3][8][4][5][6] show 0.23%~8% gains for [High CSI overhead] RU >=70%

  - 2 sources [2][7] show 11.44%~15.8% gains for [High CSI overhead] RU >=70%

- 8 sources [3][14][8][4][6][11][12][30] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=2, among which:

  - 4 sources [3][14][8][11] show 0.5%~10% gains for [Low CSI overhead] RU<=39%

  - 2 sources [30][12] show 19.9%~30% gains for [Low CSI overhead] RU<=39%

  - 4 sources [3][14][8][11] show 0.5%~8% gains for [Medium CSI overhead] RU<=39%

  - 2 sources [30][12] show 11.5%~25% gains for [Medium CSI overhead] RU<=39%

  - 1 source [8] shows 0.3% loss for [High CSI overhead] RU<=39%

  - 4 sources [3][14][11][30] show 0%~7% gains for [High CSI overhead] RU<=39%

  - 1 source [12] shows 21% gains for [High CSI overhead] RU<=39%

  - 4 sources [3][14][8][4][11] show 3%~10% gains for [Low CSI overhead] RU 40%-69%

  - 3 sources [3][30][12] show 17%~23% gains for [Low CSI overhead] RU 40%-69%

  - 4 sources [3][14][8][4][11] show 1.2%~10% gains for [Medium CSI overhead] RU 40%-69%

  - 2 sources [30][12] show 21%~22% gains for [Medium CSI overhead] RU 40%-69%

  - 1 source [8] shows 0.5% loss for [High CSI overhead] RU 40%-69%

  - 5 sources [3][14][4][11][12] show 1%~9% gains for [High CSI overhead] RU 40%-69%

  - 1 source [30] shows 16.3% gains for [High CSI overhead] RU 40%-69%

  - 4 sources [14][4][11][12] show 5%~10% gains for [Low CSI overhead] RU >=70%

  - 4 sources [3][8][6][30] show 11%~23% gains for [Low CSI overhead] RU >=70%

  - 6 sources [3][14][8][4][6][12] show 3%~9% gains for [Medium CSI overhead] RU >=70%

  - 3 sources [3][11][30] show 16%~24.6% gains for [Medium CSI overhead] RU >=70%

  - 1 source [8] shows 0.2% loss for [High CSI overhead] RU >=70%

  - 5 sources [3][14][4][6][11][12] show 1%~12% gains for [High CSI overhead] RU >=70%

  - 2 source [3][30] shows 17%~26.8% gains for [High CSI overhead] RU >=70%

- 4 sources [28][4][10][26] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=4, among which:

  - 3 sources [28][10][26] show 2%~7.4% gains for [Low CSI overhead] RU<=39%

  - 1 source [28] show 2% gains for [Medium CSI overhead] RU<=39%

  - 1 source [26] shows 4% loss for [High CSI overhead] RU<=39%

  - 3 sources [28][4][26] show 3%~12.2% gains for [Low CSI overhead] RU 40%-69%

  - 2 sources [28][4] show 2%~7.04% gains for [Medium CSI overhead] RU 40%-69%

  - 1 source [26] shows 1.8% loss for [High CSI overhead] RU 40%-69%

  - 1 source [4] shows 8.19% gains for [High CSI overhead] RU 40%-69%

  - 2 sources [28][4][26] show 3%~14.89% gains for [Low CSI overhead] RU >=70%

  - 2 sources [28][4] show 4%~6.44% gains for [Medium CSI overhead] RU >=70%

  - 1 source [26] shows 1% loss for [High CSI overhead] RU >=70%

  - 1 source [4] shows 6.88% gains for [High CSI overhead] RU >=70%

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, ZTE, Qualcomm, Futurewei |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please companies provide your view on whether the eventual results should be captured per each CSI feedback overhead range (as updated). @Futurewei captured. @Apple separately captured @Qualcomm will visit full buffer later. The work load for drawing one observation is too high… |

#### Issue#3-13a (On hold) Basic performance for CSI compression-5% UPT

Moderator note: Will be updated later in the second round.

Proposed Observation 3.3.10: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, compared to the benchmark of R16 Type II codebook, under FTP traffic and in terms of 5% UPT:

- 6 sources [3][8][4][5][6][7] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=1, among which:

  - 3 sources [3][8][5] shows 0.8%~3% gains for RU<=39%

  - 1 source [3] shows 4%0.1%~8%7% gains for RU<=39%

  - 3 sources [3][8][5] shows 0.1%~6% gains for RU 40%-69%

  - 2 sources [3][7] show 8%~21.04% gains for RU 40%-69%

  - 5 sources [3][8][4][5][6] show 0.85%~7% gains for for RU >=70%

  - 43 sources [2][3][8][7] show 8%~28% gains for for RU >=70%

  - 1 source [2] shows 49.6%~69.5% gains for RU >=70%

- 8 sources [3][14][8][4][6][11][12][30] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=2, among which:

  - 1 source [8] shows 0.5%~2% loss for RU<=39%

  - 4 sources [3][14][8][11][12] show 1.1%~9% gains for RU<=39%

  - 2 sources [30][11] show 16%~23.3% gains for RU<=39%

  - 2 source [8][12] show 2%~8% loss for RU 40%-69%

  - 5 sources [3][14][8][11][12] show 0.3%~8% gains for RU 40%-69%

  - 3 sources [3][4][11][30] show 9%~29.7% gains for RU 40%-69%

  - 2 sources [8][12] show 1.3%~10% loss for RU >=70%

  - 4 sources [3][14][8][4] show 2%~12% gains for RU >=70%

  - 5 sources [3][8][4][11][30] show 13%~39% gains for RU >=70%

- 3 sources [28][4][26] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=4, among which:

  - 2 sources [28][26] show 1%~1.6% loss for RU<=39%

  - 2 sources [28][26] show 3%~8% gains for RU<=39%

  - 1 source [26] shows 1.7% loss for RU 40%-69%

  - 3 sources [28][4][26] show 2%~11.9884% gains for RU 40%-69%

  - 4 sources [28][4][26] show 2%~20.72% gains for RU >=70%

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, ZTE(comments), Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the range of gains, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Futurewei | We noticed that our results were not captured in the Proposed Observation 3.2.4. In our reported results for Max rank = 1 case, the 5% UPT gain is between 22.6 – 69.5% for RU>=70% (for SQ, the gain is between 22.6 – 49.6% and for VQ is the gain is 69.5% for X category using LUT-based approach). |
| Apple | Suggest to capture the observation of low UCI bits and high UCI bits separately. |
| ZTE | For the case of AI/ML-based CSI compression outperforming the benchmark at Max rank=1, the last sub-bullet should add ZTE[4] since we also achieve the 20.43% 5% UPT gain, which satisfies the corresponding range. Therefore, we suggest rewording it as - 4 sources [3][8][7][4] show 8%~28% gains for for RU >=70% For the case of AI/ML-based CSI compression outperforming the benchmark at Max rank=4, according to our results, the 4th sub-bullet should be reworded as - 3 sources [28][4][26] show 2%~11.84% 11.98% gains for RU 40%-69% |
| Futurewei | We thank FL for including our results. One minor comment: 6 7 sources [2][3][8][4][5][6][7] show that the AI/ML-based CSI compression outperforms the benchmark at Max rank=1 |

#### Issue#3-13b (Medium priority) Basic performance for CSI compression-input type

Moderator note: Continue discussion at the second round.

Huawei [3], Nokia [8], FUTUREWEI [2], Lenovo [27], ZTE [4], vivo [5], OPPO [6], ETRI [22], Spreadtrum [7], Fujitsu [13], NTT DOCOMO [30], CMCC [23] have provided results for FTP traffic with Max rank 1, all with the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI, showing more gains over eigenvector of current CSI.

Huawei [3], Ericsson [14], Nokia [8], ZTE [4], OPPO [6], Intel [11], Fujitsu [13], NTT DOCOMO [30], InterDigital [12] have provided results for FTP traffic with Max rank 2, wherein

- Ericsson [14] adopts the input type of W2 matrix after eType-II processing

- Other sources adopt the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI as model input, showing more gains over eigenvector of current CSI (1%-6% additional gain over current CSI for rank 1, 1%-14% additional gain over current CSI for rank 2).

Qualcomm [28], ZTE [4], CATT [10], Apple [26] have provided results for FTP traffic with Max rank 4, all with the input type of precoding matrix.

Huawei [3], Nokia [8], vivo [5], OPPO [6], Fujitsu [13], China Telecom [40], MediaTek [24] have provided results for full buffer with Max rank 1, all with the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI as model input, showing more gains over eigenvector of current CSI.

Huawei [3], Xiaomi [15], Nokia [8], Qualcomm [28], vivo [5], Intel [11], Fujitsu [13], InterDigital [12] have provided results for full buffer with Max rank 2, wherein

- Ericsson [14] adopts the input type of W2 matrix after eType-II processing

- Other sources adopt the input type of precoding matrix; on top of that, Huawei [3] adopts eigenvector with additional past CSI, showing more gains over eigenvector of current CSI.

ZTE [4] has provided results for full buffer with Max rank 4, all with the input type of precoding matrix.

Proposed Observation 3.3.11: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, for the AI/ML model input (for CSI generation part) and output (for CSI reconstruction part):

- 21 sources [3][8][2][27][4][5][6][22][7][13][30][23][6][11][12][28][10][26][15][40][24] adopt precoding matrix as the AI/ML model input and output, wherein

  - 1 source [3] adopts the eigenvector(s) with additional past CSI as the model input, showing 1%-14% additional gain over the model input with the eigenvector(s) of current CSI.

- 1 source [14] adopts channel matrix as the model input/output

| Support/Can accept | Futurewei, vivo, Huawei/HiSi, Lenovo, Fujitsu, ZTE, Qualcomm, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Xiaomi corrected! Sorry for the mistake! |

#### Issue#3-13c (Medium priority) Basic performance for CSI compression-rank>1 options

Moderator note: Continue discussion at the second round.

- Option 1-1 (rank specific): ZTE [4],

- Option 1-2 (rank common):

- Option 2-1: layer specific and rank common: Ericsson [14], Intel [11], Qualcomm [28],

- Option 2-2: layer specific and rank specific: Ericsson [14],

- Option 3-1: layer common and rank common: Huawei [3], CMCC [23], ZTE [4], OPPO [6], Fujitsu [13], vivo [5],

- Option 3-2: layer common and rank specific: NTT DOCOMO [30], InterDigital [12], Apple [26]

The following observation is given as:

Proposed Observation 3.3.12: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, for the rank>1 options:

- 1 source [4] evaluates Option 1-1 (rank specific).

- 3 sources [14][11][28] evaluate Option 2-1 (layer specific and rank common).

- 1 source [14] evaluates Option 2-2 (layer specific and rank specific).

- 8 sources [3][23][4][6][13][5][15][10] evaluate Option 3-1 (layer common and rank common).

- 3 sources [30][12][26] evaluate Option 3-2 (layer common and rank specific).

- Note: no source submits evaluation results based on Option 1-2 (rank common).

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, ZTE, Qualcomm, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Xiaomi @CATT corrected! |
| NTT DOCOMO | We [30] also submit the results for Option 3-1 to the result collection table. |

#### Issue#3-13d (Medium priority) Basic performance for CSI compression-quantization training

Moderator note: Continue discussion at the second round.

Huawei [3], Ericsson [14], vivo [5], Qualcomm [28], MediaTek [24], Nokia [8], Fujitsu [13], InterDigital [12], Comba [20] observe that quantization aware training shows clear performance gain compared to quantization non-aware training.

Proposed Observation 3.3.13: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, 9 sources [3][14][5][28][24][8][13][12][20] show that quantization aware training (Case 2-1/2-2) provides clear performance gain compared to quantization non-aware training (Case 1).

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-13e (Low priority) Basic performance for CSI compression-quantization and scalability

Moderator note: Continue discussion at the second round.

MediaTek [24], Nokia [8], ETRI [22] observe that by adjusting the quantization granularity, it can also achieve the scalability of CSI payload size, without impacting the dimension of the AI/ML model.

For example, to achieve CSI payload size of L, 2L, …, Q*L bits, two approaches can be considered:

- Consider scalable dimension of the latent variable as L, 2L, …, Q*L bits

- Applying 1,…, Q bits quantization for latent variable’s dimension of fixed L bits.

The following observation is given as:

Proposed Observation 3.3.14: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, 3 sources [24][8][22] show that scalability on CSI feedback payload size can be achieved by either training a scalable AI/ML model with various latent space dimensions or applying various quantization granularities on a AI/ML model with non-scalable latent space dimension.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-13f (Low priority) Basic performance for CSI compression-VQ and SQ

Moderator note: Continue discussion at the second round.

vivo [5], Fujitsu [13], and FUTUREWEI [2], Qualcomm [28]observe that VQ outperforms SQ under the same CSI payload size.

MediaTek [24], Nokia [8] observe that SQ outperforms VQ under the same CSI payload size.

Xiaomi [15] observe that SQ has comparable performance with VQ under the same CSI payload size.

The following observation is given as:

Proposed Observation 3.3.15: For the AI/ML based CSI compression, till the RAN1#112bis-e meeting, between the quantization methods of scalar quantization and vector quantization under the same CSI payload size:

- 4 sources [5][13][2][28] show that vector quantization outperforms scalar quantization.

- 2 sources [24][8] show that scalar quantization outperforms vector quantization.

- 1 source [15] shows that scalar quantization and vector quantization have comparable performance.

| Support/Can accept | Futurewei (see comments) , Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Lenovo How to use both? Did you elaborate the method in your Tdoc or spread sheet? @MediaTek what is your suggestion on the change of the observation? It is Moderator’s understanding that a limited companies simulated with the robustness of the SQ/VQ, which may not generate a part of the observation, but you may add some clarification as a note. @Qualcomm In principle it should not be captured. But this time is fine. |

#### Issue#3-14 (High priority) Generalization for CSI compression – deployment scenario

Moderator note: Continue discussion at the second round.

From the collected results for Table 2,

Mavenir [18], Huawei [3], Ericsson [14], FUTUREWEI [2], Lenovo [27], Intel [11], vivo [5], OPPO [6], Fujitsu [13], CMCC [23], CATT [10], Xiaomi [15], ZTE [4], NTT DOCOMO [30], InterDigital [12] submitted the results on the generalization over deployment scenarios (UMa/UMi/InH, etc.).

The following observation is given as:

Proposed Observation 3.3.16: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#112bis-e meeting,

- 15 sources [18][3][14][2][27][11][5][6][13][23][10][15][4][30][12] show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

  - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, deployment scenario#A is UMi, deployment scenario#B is UMa, or deployment scenario#A is InH, deployment scenario#B is UMa/UMi.

  - 6 sources [3][5][4][10][15][11] observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor.

    - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, or deployment scenario#A is UMi, deployment scenario#B is UMa.

    - 1 source [2] observes that the degradation under the above combinations is also significant.

- 5 sources [3][5][10][2][12] show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#A and deployment scenario#B, and the trained AI/ML model applies inference on either deployment scenario#A or deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa and/or UMi.

- 3 sources [3][15][4] show that, compared to the case where the AI/ML model is trained on scenario#A and applied for inference on deployment scenario#B, the generalization performance can be improved, if the AI/ML model, after trained on deployment scenario#A, is updated based on a fine-tuned dataset subject to deployment scenario#B, and performs inference on deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa or UMi.

| Support/Can accept | vivo, Huawei/HiSi, CMCC, Fujitsu, ZTE, Futurewei |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Futurewei done. Would you add Futurewei to “6 sources [3][5][4][10][15][11] observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor”? |
| Futurewei | We thank FL for considering our feedback. We don’t have sufficient result to support “if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor” at least for this meeting. We may report our results for next meeting. |

#### Issue#3-15 (Medium priority) Generalization for CSI compression – carrier frequency

Moderator note: Continue discussion at the second round.

From the collected results for Table 2,

Ericsson [14], vivo [5], NTT DOCOMO [30] submitted the results on the generalization over carrier frequencies (2GHz, 3.5GHz, etc.).

The following observation is given as:

Proposed Observation 3.3.17: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#112bis-e meeting,

- 3 sources [14][5][30] show that compared to the case where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B, it has almost the same performance if the AI/ML model is trained with carrier frequency#A and applied for inference with a different carrier frequency#B.

  - E.g., carrier frequency#A is 2GHz/5.5GHz, carrier frequency#B is 3.5GHz, or carrier frequency#A is 3.5GHz, carrier frequency#B is 2GHz.

  - Note: same antenna layout is assumed for carrier frequency#A and carrier frequency#B.

| Support/Can accept | vivo, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-16 (Low priority) Scalability for CSI compression – Bandwidths

Moderator note: Continue discussion at the second round.

From the collected results for Table 3,

Ericsson [14], ZTE [4], NTT DOCOMO [30], InterDigital [12] submitted the results on the generalization over various bandwidths.

Proposed Observation 3.3.18: For the scalability verification of AI/ML based CSI compression over various bandwidth values, till the RAN1#112bis-e meeting,

- 2 sources [14][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain bandwidth#B and applied for inference with a same bandwidth#B, it has almost the same performance if the model is trained with bandwidth#A and applied for inference with a different bandwidth#B, and an appropriate scalability solution is performed.

  - 2 sources [14][4] adapt the subband size according to bandwidth value without scaling the dimension of the AI/ML model as the scalability solution.

- 2 sources [30][12] shows that generalized performance of the AI/ML model can be achieved for the inference on either bandwidth#A or bandwidth#B, if the training dataset is constructed with data samples subject to multiple bandwidth values including bandwidth#A and bandwidth#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 2 sources [30][12] adopt pre/post-processing of truncation/padding as the scalability solution.

| Support/Can accept | ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-17 (Medium priority) Scalability for CSI compression – Tx ports

Moderator note: Continue discussion at the second round.

From the collected results for Table 3,

Huawei [3], OPPO [6], Fujitsu [13], CATT [10], ZTE [4], NTT DOCOMO [30] submitted the results on the generalization over various Tx port numbers.

The following observation is given as:

Proposed Observation 3.3.19: For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#112bis-e meeting,

- 3 sources [6][13][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B, it has degraded performance if the model is trained with Tx port number#A and applied for inference with a different Tx port number#B

  - E.g., Tx port number#A is 32, Tx port number#B is 16.

- 6 sources [3][6][13][10][4][30] show that generalized performance of the AI/ML model can be achieved for the inference on either Tx port number#A or Tx port number#B, if the training dataset is constructed with data samples subject to multiple Tx port numbers including Tx port number#A and Tx port number#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 5 sources [3][6][13][4][30] adopt pre/post-processing of truncation/padding as the scalability solution.

  - 1 source [10] adopts adaptation layer in the AL/ML model as the scalability solution.

| Support/Can accept | Huawei/HiSi, Fujitsu, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

#### Issue#3-18 (High priority) Scalability for CSI compression – CSI payload sizes

Moderator note: Continue discussion at the second round.

From the collected results for Table 3,

Huawei [3], Ericsson [14], OPPO [6], Fujitsu [13], CMCC [23], CATT [10], MediaTek [24], NTT DOCOMO [30] submitted the results on the generalization over various CSI payload sizes.

The following observation is given as:

Proposed Observation 3.3.20: For the scalability verification of AI/ML based CSI compression over various CSI payload sizes, till the RAN1#112bis-e meeting,

- 2 sources [6][13] show that compared to the case where the AI/ML model is trained with dataset subject to a certain CSI payload size#B and applied for inference with a same CSI payload size#B, it has degraded performance if the model is trained with CSI payload size#A and applied for inference with a different CSI payload size#B.

- 8 sources [3][14][6][13][23][10][24][30] show that generalized performance of the AI/ML model can be achieved for the inference on either CSI payload size#A or CSI payload size#B, if the training dataset is constructed with data samples subject to multiple CSI payload sizes including CSI payload size#A and CSI payload size#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model.

  - 3 sources [6][13][23] adopt pre/post-processing of truncation/padding as the scalability solution.

  - 1 source [14] adopt various quantization granularities as the scalability solution.

  - 4 sources [3][10][24][30] adopt adaptation layer in the AL/ML model as the scalability solution.

| Support/Can accept | vivo, Huawei/HiSi, CMCC, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

# Specific evaluation methodology for CSI prediction sub use case

## Summary of views from companies

### 4.1-1: Evaluation views/findings from companies

CSI prediction on time domain

- Views

  - OPPO: In R18 time domain CSI prediction, following two aspects should be studied to evaluate the performance gain and identify the potential spec impacts

    - Impact on throughput caused by scheduling delay and outdated CSI

    - Reduction of CSI-RS overhead

  - OPPO: For UE-sided CSI prediction, the following items should be considered

    - Ideal channel estimation for training stage and intermediate KPI calibration

    - Realistic channel estimation together with CSI feedback error for SLS performance evaluation

    - Possible CSI feedback enhancement for the output of CSI prediction (e.g. eType II based, AI based CSI feedback)

    - FFS: how to perform raw channel feedback in observation and prediction window

  - NVIDIA: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides evaluating the CSI prediction at UE side, companies are encouraged to evaluate the CSI prediction at gNB side to understand the potential gains of performing CSI prediction at gNB side vs. UE side

- Findings on CSI prediction on time domain

  - Huawei, Hisilicon: For AI/ML-based CSI prediction

    - It outperforms nearest historical CSI with 2.5%-4% gain in terms of mean UPT and with 9%-17% gain in terms of 5% UPT.

    - With the same configuration of the observation window, it outperforms non-AI/ML prediction with 2%-3% gain in terms of mean UPT and with 6%-10% gain in terms of 5% UPT.

  - vivo: The AI-based CSI prediction achieves higher spectral efficiency when compared with the nearest historical CSI w/o prediction and AR-based non-AI CSI prediction.

  - Samsung: Gains in terms of SGCS are observed from AI/ML-based CSI prediction as compared to the sample-and-hold baseline over various UE speeds and prediction window (application instances)

    - The observed gains are different across UE speeds

    - The observed gains are different across different offset values between the CSI measurement and the predicted CSI application instance

  - Samsung: The relevant prediction horizon (the delay between CSI application instances from last measurement) to achieve a certain SGCS level is related to UE’s speed. gNB’s estimation on the rate of channel change (UE’s relative speed) may be beneficial to adjust the prediction horizon accordingly

  - ZTE: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied

  - ZTE: After eTypeII quantization on the predicated CSI, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI

  - Nokia: System level simulations of AI/ML-based CSI prediction show gains of somewhat less than 5% in mean user throughput with SU-MIMO in full buffer traffic

  - OPPO: Regarding AI based CSI prediction in single-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

    - AI achieves higher CSI prediction performance gain for UE with fast channel variation

    - AI achieves higher CSI prediction performance gain with father CSI prediction instance within the same CSI-RS feedback period

  - OPPO: Regarding AI based CSI prediction in multi-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

  - MediaTek: Depending on the requirements on CSI prediction, for example the required prediction length, AI/ML-based solutions provide superior performance compared to classical non-AI based methods

  - MediaTek: The AI/ML-based CSI prediction with Rel-18 codebook can effectively solve the CSI aging problem under specific scenarios

  - MediaTek: The AI/ML-based CSI prediction performance can approach the ideal prediction performance when the CSI-RS periodicity is within the coherence time.

  - CATT: AI/ML based CSI prediction outperforms the baseline using nearest historical CSI

  - Apple: LSTM based AI model achieves more than 10dB gain for CSI prediction use case

  - Apple: For sample and hold method, the SGCS performance is much better than NMSE since only Doppler effect is modeled in the channel

  - Xiaomi: AI based CSI prediction outperforms the benchmark scheme

  - CMCC: AI/ML based CSI prediction can achieve very high prediction accuracy compared with baseline non-prediction in terms of NMSE

  - CMCC: The performance of both baseline and AI based CSI prediction will decrease when UE moves faster

  - InterDigital: The SGCS performance of Layer 1 is higher than that of Layer 2 for all considered CSI prediction methods

  - InterDigital: The SGCS gain of LR over KF and S&H increases when the UE speed increases

  - InterDigital: For the same observation window and prediction window, the SGCS performance decreases when the UE speed increases

  - ETRI: AI/ML based CSI prediction improves performance compared to the baseline

  - Fujitsu: AI/ML-based eigenvector prediction outperforms the sample-and-hold method for temporal domain

  - Fujitsu: For AI/ML-based eigenvector predictions, the performance gain is decreased as the UE speed increases, compared to the sample-and-hold method

  - NVIDIA: Evaluation results show that AI/ML based CSI prediction significantly outperforms the baseline case without prediction (sample-and-hold)

  - Spreadtrum: The AI based CSI prediction can achieve better SGCS performance than the nearest historical CSI w/o prediction

### 4.1-2: AI/ML model settings

#### Input/output CSI type

- Option 1: Raw channel matrix Huawei, Hisilicon, vivo, Nokia, ZTE, MediaTek, CMCC, NVIDIA Xiaomi InterDigital ETRI, Spreadtrum

  - Huawei, Hisilicon: The input/output type of CSI prediction is channel matrix, and SGCS is considered as the performance metric. SGCS is calculated based on the eigenvectors of the first layer decomposed from the predicted channel matrix, and the formula of SGCS calculation is same as the CSI compression sub use case… The frequency domain size of input/output channel matrix is 1RB in this simulation, after which the subband CSIs to be reported are calculated based on the predicted CSI per RB.

  - vivo: 15 raw historic channels in PRB

  - Nokia: complex CSI, 50 PRBs, 16 AP

  - MediaTek: each CSI instance is a complex-valued matrix with dimensions NRX×NTX×NF, where NRX and NTX are the numbers of RX and TX antennas, respectively, and NF is the number of elements in the frequency dimension, which could be on subcarrier or PRB level

  - MediaTek: we suggest converting the time-frequency domain channel into the Doppler-delay domain channel by simple two-dimensional (2D) discrete Fourier transform (DFT) matrix. Need to note that the phase shift in Doppler domain is needed to shift the zero-frequency component to the center.

    - Converting the time-frequency domain dense channels into Doppler-delay domain sparse channels can significantly improve the performance in the specific scenarios.

  - ZTE: historical channel measurements (i.e., raw channel in time domain) are used as the input of AI models for training, validation and testing.

    - During the training process, to avoid high AI model complexity, the model input only includes 8 PRBs from a sub-band in frequency domain

  - CMCC: the CSI information is the full channel in one RB

  - Spreadtrum: we use the historical channel matrix during the observation window as the input to predict the channel matrix in the future slot(s)

- Option 2: Eigenvector OPPO Google ZTE CATT Fujitsu

  - ZTE: In order to evaluate the performance influenced by the diverse input types, we adopt the samples of historical sub-band eigenvectors processed from the raw channel matrices as input and the corresponding predicted eigenvectors as output

  - CATT: both the input and output are in the format of eigenvectors

  - OPPO: For eigenvector prediction, the input/output of AI/ML model would be the eigenvector W∈CNt×Nsb, where Nsb is the number of sub-band.

  - Google: The study of the input of CSI prediction should prioritize the input based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1

  - Fujitsu: the historical eigenvectors are used to predict the future eigenvectors at one or multiple slots

- Views on input/output types

  - ZTE: Further study the input and output types for the sub-use case of AI/ML based CSI prediction

  - ETRI: For the evaluation of CSI prediction sub use-case, consider the input of AI/ML model for CSI prediction as RAW channel matrix

  - Google: Study the following output of CSI prediction

    - Predicted RI/PMI based on Type1 codebook

    - Predicted CSI dwelling time

- Findings for comparison of input types

  - ZTE: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input

  - ZTE: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms

  - ETRI: AI/ML based CSI prediction as input of eigenvectors improves performance compared to the baseline. However, performance improvement is slightly reduced compared to the AI/ML model as input of RAW channel matrix

#### Observation window

- Descriptions on the Observation window

  - Huawei, Hisilicon: The observation window is the latest k=4 observation instances with 5 slots distance to each other

  - vivo: the prediction is with 15 historical CSIs as the input

  - Samsung: A CSI measured from K=3 CSI-RS resources are used for CSI prediction.

  - Nokia: 4 / 5 ms

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions, respectively

    - we set diverse samples of historical CSIs as AI model inputs (3, 4, 6, 8 historical CSIs)

  - MediaTek: observation window is 10/15

  - CATT: In our design, we use the CSI in past three moments (i.e. t-2, t-1, t) to predict the CSI in one moment in the future (i.e. t+1)

  - Apple: Assuming CSI-RS periodicity of 5ms, the measurement window length 8

  - Xiaomi: In case 1, the input sample number is 8 and the prediction number is 1. In case 2, the input sample number is 4 and the prediction number is 1. And for case 3, the input sample number is 8 and the prediction number is 4, the 4 prediction samples are predicted one by one

  - OPPO: the input of CSI model includes K historic eigenvectors [Wt-5(K-1),…Wt-5,Wt] from K CSI-RS measurement

  - CMCC: we use 15 historic CSI samples to predict the 16th and 17th CSI in time domain

  - InterDigital:1~10ms

  - ETRI: we simulate 5/10 historical CSI information as the input of AI/ML model with CSI periodicity of 5ms/10ms/15ms

  - Fujitsu: The sampling interval of eigenvectors (v1, v2, v3, …) is 5ms, the slot number of AI/ML model input and output is 4 and 1, respectively.

  - NVIDIA: The raw channel matrices of the four latest CSI-RS measurement instances are used as the AI/ML model input. The raw channel matrices are associated with the first PRB

  - Spreadtrum: the observation window is 4, and the prediction window is 1. The CSI feedback periodicity is 5ms.

- Views on the Observation window

  - Nokia: Support high number of frequency-domain and of time-domain channel samples for training and inference of NNs for channel prediction

  - MediaTek: The tradeoff between the observation length and prediction length should be further studied

  - ZTE: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction

  - OPPO: Regarding the EVM on CSI prediction, evaluate the performance with different numbers of observation window K and prediction window T according to CSI-RS period

  - vivo: The performance impact of observation window on the AI-based CSI prediction should be studied

- Findings on the Observation window:

  - vivo: The observation window can be described by the number of historical CSIs and the spacing of the historical CSIs

  - vivo: The larger the number and the smaller the spacing of historical CSIs within the observation window, the better the prediction performance that can be achieved. However, this will in return increase the complexity and the storage (buffer) overhead of the model.

  - vivo: For different speeds, the requirement for the observation window is different

  - ZTE: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increasing number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs

  - ZTE: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input

  - Apple: LSTM based prediction achieve 20% SGCS performance gain at 5ms predict time, 23% at 7.5ms prediction time, and 10% at 10ms prediction time

  - Xiaomi: The prediction accuracy increased with larger observation window

  - ETRI: AI/ML based CSI prediction slightly improves performance as the number of historical input CSIs increase.

#### Prediction window

- Descriptions on the prediction window

  - Huawei, Hisilicon: The prediction window is 1 future slot

  - vivo: the future CSI at +4ms as output

  - Nokia: 1 / 4 ms

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions (+5ms, +10ms, +15ms), respectively

  - OPPO: The output of CSI prediction model includes 4 future eigenvectors Wt+1,Wt+2,Wt+3,Wt+4 on the next 4 interval slots. Then the observation window is K and the prediction window is 4.

  - MediaTek: the length of the prediction window is 5/1/3;

  - Apple: Prediction window is 10ms, where we predict the channel 2.5ms, 5ms, 7.5ms and 10ms ahead of the last CSI-RS measurement

  - Samsung: The CSI prediction is made to two application instances. Application instance 1 and Application instance 2 which are 10ms and 25ms after the last CSI-RS resource used for measurement, respectively.

  - CATT: In our design, we use the CSI in past three moments (i.e. t-2, t-1, t) to predict the CSI in one moment in the future (i.e. t+1)

  - Xiaomi: In case 1, the input sample number is 8 and the prediction number is 1. In case 2, the input sample number is 4 and the prediction number is 1. And for case 3, the input sample number is 8 and the prediction number is 4, the 4 prediction samples are predicted one by one

  - CMCC: we use 15 historic CSI samples to predict the 16th and 17th CSI in time domain. The time interval between two CSIs is 5ms.

  - InterDigital: 1,…10ms

  - ETRI: to predict 3 future CSIs

  - Fujitsu: The sampling interval of eigenvectors (v1, v2, v3, …) is 5ms, the slot number of AI/ML model input and output is 4 and 1, respectively.

  - NVIDIA: The AI/ML model output is the predicted raw channel matrix at 4 ms ahead

  - Spreadtrum: the observation window is 4, and the prediction window is 1. The CSI feedback periodicity is 5ms.

- Views on the prediction window

  - ZTE: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction

  - MediaTek: The trade-off between the length of observation window and the length of prediction window should be further studied

- Company findings on the prediction window:

  - ZTE: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction

  - Samsung: the SGCS gain is different over UE speeds and application instances. This suggests that the gNB can actively monitor the UE’s scenario, e.g., speed, and configure the number of CSI prediction instances and prediction window accordingly

  - Apple: Reduce the CSI-RS sample distance from 5ms to 2.5ms improve the SGCS by for LSTM based AI model. At 10ms prediction time, 17.5% SGCS improvement is observed over 5ms sample distance. Overall 30.6% SGCS improved at 10ms over sample and hold baseline

  - Xiaomi: The prediction accuracy decreased with larger prediction window

  - InterDigital: The SGCS performance decreases when the prediction time increases

#### Other views/findings

CSI-RS periodicity

- vivo: Using the preprocessing-based model scaling mechanism, the model trained at one speed can scales to other speeds

- MediaTek: The different CSI-RS periodicity should depend on the corresponding appropriate coherence time (UE speed and carrier frequency).

- Apple: Reduce the CSI-RS sample distance from 5ms to 2.5ms improve the SGCS by for LSTM based AI model. At 10ms prediction time, 17.5% SGCS improvement is observed over 5ms sample distance. Overall 30.6% SGCS improved at 10ms over sample and hold baseline

### 4.1-3: EVM related issues for CSI prediction

#### KPI

- Intermediate KPI

  - OPPO: NMSE performance cannot completely match to the SGCS performance, especially for different UEs.

  - OPPO: Suggest to use multiple kinds of intermediate KPIs (e.g. SGCS, NMSE) for CSI prediction evaluation, conclusions should be drawn based on eventual KPI (e.g. SLS throughput) instead of intermediate KPIs

- Other KPIs

  - Execution latency Nokia

    - Nokia: Adopt a measure of execution latency and include it in the performance reporting template for CSI prediction

      - One possible solution is to report a measure of the sequential calculation depth of the model, such as the number of layers in the model.

- Other views on KPI

  - ZTE: Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability.

#### Modeling for Spatial consistency

- vivo: The spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms.

- LG: For UE-sided AI/ML based CSI prediction, consider spatial consistency and companies can report one of spatial consistency procedures, Procedure A and/or B in TR 38.901

#### Benchmark for evaluation results comparison

General views on benchamrk

- InterDigital: To evaluate the AI/ML CSI prediction performance, a more realistic mobility model needs to be used

Modeling of level x based CSI prediction in Benchmark#2

- Huawei, Hisilicon: For the CSI prediction sub use case, if collaboration level x is reported as the benchmark, the simulation methodology to evaluate the AI/ML-based CSI prediction enabled by LCM can emulate the fine-tuning case, while the simulation methodology to evaluate collaboration level x CSI prediction can emulate generalization Case 2.

- vivo: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the comparison of level y/z AI-based CSI prediction and level x AI-based CSI prediction should be studied in the generalization aspects, e.g., collaboration level x is modeled as generalization Case 2, while collaboration level y/z is modeled as the generalization Case 1 or finetuning.

- vivo: For AI-based CSI prediction, with a level y/z collaboration, the speed-specific model can be switched according to the information associated with the UE change speed so as to guarantee the prediction performance for different speed. However, using a level x AI/ML model, it is hard to generalize well across different speeds

- Samsung: For the AI/ML based CSI prediction sub use case, if collaboration level x-based AI/ML CSI prediction is reported as the benchmark, the EVM to distinguish level x and level y/z-based AI/ML CSI prediction is considered from the generalization aspect

- Samsung: For the AI/ML based CSI prediction sub use case, if collaboration level x- based AI/ML CSI prediction is reported as the benchmark, model generalization performance across N scenarios/configurations to be evaluated:

  - For level y/z-based AI/ML CSI prediction, LCM aspects such as model switching, model selection, fall-back, achieve generalization Case 1

  - For level y/z-based AI/ML CSI prediction, LCM aspects such as model fine tuning achieves generalization Case 3

  - For level x-based AI/ML CSI prediction benchmark, performance of generalization Case 2 and Case 3 to be considered.

FFS: whether to introduce restriction on Case 3 for level x based AIML based CSI prediction.

Note: if different model size are assumed for level x and level y/z-based AI/ML prediction, the corresponding FLOPs, model size, parameters, to be reported.

- AT&T:For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

  - E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2.

#### Template for simulation results collection

### 4.1-4: Generalization study

#### Generalization over frequency PRBs

- Findings

  - MediaTek: in the whole frequency band, we can train only one AI model by a specific RB (or a subcarrier) and use the same AI/ML model for other RBs. In this way, we can save the memory and computation complexity in the UE side.

- NVIDIA: AI/ML training and inference are performed on the first PRB. Next, we use the AI/ML model trained on the first PRB to carry out inference/testing on different PRBs.

  - Findings: The AI/ML based CSI prediction model trained on a certain PRB can be generalized to perform inference on other PRBs

#### Generalization over UE speeds

- Views/solution description

  - vivo: The generalization of AI-based CSI prediction across speeds should be studied, relying on, e.g., level y/z collaboration-based model switching and model scaling mechanisms.

  - vivo: The monitoring of the AI-based CSI prediction should be studied

  - MediaTek: UE speed is leveraged for evaluation with 10, 20, 30, 60, 120km/h

    - Case 1/2: Training at 30km/h, inference at 10/20/30/60/120km/h; Training at 120km/h, inference at 10/20/30/60/120km/h

    - Case 3: Training at mixed [10, 20, 30, 60, 120] km/h, inference at 10/20/30/60/120km/h

  - Samsung: For the evaluation of AI/ML based CSI prediction sub use case, the following evaluation cases are considered,

    - Case 1: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a dataset from the same UE speed A

    - Case 2: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

    - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple UE speeds including UE speed A and a different dataset than UE speed A, e.g., UE speed B, and then the AI/ML model performs inference/test on a dataset from a single UE speed from the multiple UE speeds, e.g., UE speed A, UE speed B.

      - Note: Companies to report the ratio for dataset mixing

    - Note: UE speed includes 10Kmphr, 20kmphr, 30kmphr, 60kmphr and 120kmphr

  - Samsung: For UE-side AI/ML-based CSI prediction, study mechanisms to cope up with various UE speeds

    - Methods for the network to discover UE speed scenario

    - Network’s explicit and implicit indications for model/functionality activation, selection, switching

  - Apple: Case 3: the training dataset is a mixed dataset with both 30kmph speed and 60kmph speed UEs. The testing case is 30kmph data set and 60kmph data set separately.

  - CMCC: Case 3: we use a unified model trained with dataset containing samples with various speeds (10, 20, 30, 60 km/h), and during inference phase, this unified model is applied to separate test dataset with different UE speed

  - ETRI:

    - Case 1: Training at 30/20/10km/h, inference at the same UE speed of 30/20/10km/h.

    - Case 2: Training at 30/20/10km/h, inference at a different UE speed of 30/20/10km/h.

    - Case 3: Training at mixed dataset of 30/20/10km/h, and either 30/20/10km/h is adopted for testing.

  - ETRI: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed

  - LG: Consider various UE speed for model generalization performance of CSI prediction

- Findings

  - Huawei, Hisilicon: For generalization of AI/ML-based CSI prediction over UE speeds of 30km/h and 60km/h,

    - AI/ML model trained by low UE speed has poor performance when tested on high UE speed.

    - AI/ML model trained by mixed dataset shows moderate performance when tested on each of the UE speeds.

  - vivo: The generalization of AI-based CSI prediction over speed is not good if the training set contains only one speed

  - vivo: The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed, whose prediction accuracy is still worse than that of speed-specific models

  - MediaTek: For CSI prediction, the Doppler effect and the coherence time are critical factors for AI/ML model’s prediction accuracy

  - MediaTek: When the coherence time is less than the CSI-RS periodicity, the CSI prediction performance will degrade rapidly

  - MediaTek: For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds

  - Samsung: Severe performance degradation in terms of SGCS is observed when the AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

    - The degradation is sever when UE speed B < UE speed A.

  - Samsung: Considerable performance gain degradation in terms of SGCS is observed when the AI/ML model is trained based on training dataset from mixed dataset of different UE speeds, e.g., UE speed A + UE speed B + UE speed C, and then the AI/ML model performs inference/test on a test dataset from one of the UE speeds, UE speed A, UE speed B, or UE speed C.

    - The degradation is severe when the test UE speed is lower among the mixed set of UE speeds

  - Apple: LSTM based AI model performance has small performance degradation with mixed training dataset.  Model switching can be used to adapt to different mobility

  - CMCC: The unified AI model trained with mixed dataset achieve good generalization performance over different UE speeds for CSI prediction

  - ETRI: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds

  - ETRI: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

  - Fujitsu: For AI/ML based CSI prediction, the performance degradation can be observed when the AI/ML model trained by the dataset with UE speed X is tested on the dataset with UE speed Y(≠X).

  - Fujitsu: For AI/ML based CSI prediction, the AI/ML model trained by the mixed dataset has good generalization for various UE speeds

  - ZTE: The model trained with a high UE speed can achieve a good generalization performance to a low UE speed. However, on the contrary, the model trained with a low UE speed can achieve a poor generalization performance to a high UE speed.

  - ZTE: Case 3(AI model trained with mixed UE speeds) shows good generalization performance for various UE speeds

- Preliminary trends observed by more than one company

  - For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds. Huawei, Hisilicon, vivo, MediaTek, Samsung, ETRI, Fujitsu, ZTE

    - The degradation is severe when the UE speed in the training dataset < the UE speed in the inference dataset Huawei, Hisilicon, vivo, Samsung, MediaTek,

    - The generalization of AI-based CSI prediction over speed can be improved/still degrades using training set with mixed speed. Huawei, Hisilicon, vivo, MediaTek, Apple, CMCC, ETRI, Fujitsu, ZTE, Samsung

#### Generalization over carrier frequency

- MediaTek: Observe the performance changes under different carrier frequency values (including 2, 3 and 3.5GHz)

  - Case 1/2: Training at 3GHz, inference at 2GHz/3GHz/3.5GHz

  - Case 3: Training at mixed [2GHz, 3GHz, 3.5GHz], inference at 2GHz/3GHz/3.5GHz

- ETRI:

  - Case 1: Training at 2GHz/3GHz/4GHz, inference at 2GHz/3GHz/4GHz

  - Case 1: Training at 2GHz, inference at 3GHz/4GHz, Training at 3GHz, inference at 2GHz/4GHz, Training at 4GHz, inference at 2GHz/3GHz

  - Case 3: Training at mixed [2GHz, 3GHz, 4GHz], inference at 2GHz/3GHz/4GHz

Views/Findings

  - MediaTek: For CSI prediction, the AI/ML model trained on a certain carrier frequency may not be generalized on other carrier frequencies

  - ETRI: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of carrier frequencies

  - ETRI: Mixed datasets of different carrier frequencies can mitigate performance degradation compared to datasets of single carrier frequency.

  - ETRI: Further evaluate the AI/ML based CSI prediction over various carrier frequencies to overcome performance degradation in untrained carrier frequency

#### Generalization over deployment scenarios

- vivo: The generalization over scenarios (e.g., LOS/NLOS, Uma/Umi) and the model adjustment (e.g., model switching/selection, finetuning, deactivation, and fallback) of AI-based CSI prediction should be studied

- vivo: The generalization over the deployment scenarios, e.g., LOS/NLOS, Uma/Umi, is not good if the training set contains only one scenario

- MediaTek:

  - Case 1: Training at UMa, inference at UMa; Training at UMi, inference at UMi

  - Case 2: Training at UMa, inference at UMi; Training at UMi, inference at UMa

  - Case 3: Training at mixed [UMa, UMi], inference at UMa / UMi

Findings

  - For CSI prediction, the AI/ML model trained on a certain deployment (e.g., UMa/UMi) can be generalized and performed inference on other deployment (e.g., UMa/UMi).

### 4.1-5: Monitoring

Solutions

vivo: we randomly select N scenarios or configurations (each containing K samples) and concatenate samples of these N scenarios or configurations in the selected order to form a test set. Then, the prediction accuracy is continuously calculated based on this test set…Just for an example, we provide the monitoring results when considering multiple speeds including 30, 60, and 120 km/h. For comparison of schemes, we evaluated the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2) and the scheme with model selection.

Views

- vivo: The monitoring of the AI-based CSI prediction should be studied

- vivo: The monitoring dataset is formed by randomly selecting N scenarios or configurations (each containing K samples) and concatenating samples of these N scenarios or configurations in the selected order.

Findings

- vivo: During the monitoring process, the scheme with model selection achieves better prediction accuracy than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

- vivo: During the monitoring process, the fluctuation of the scheme with model selection is smaller than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

## 1st round email discussions

### 4.2-1: Remaining EVM issues

#### Issue#4-1 (High priority) Benchmark for performance comparison-modeling of level x

Moderator note: For how to model level x, the same proposal from the last meeting (which received relatively few replies) is raised.

Proposal 4.2.1: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2.

| Support/Can accept | vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, LG Electronics, Fujitsu, Apple, NVIDIA, MediaTek, AT&T, Samsung, Futurewei |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | Based on our simulation results, the generalization of UE speed and carrier frequency is poor. Even training at a mixed dataset with various scenarios may cause the significant degradation. Therefore, if we consider the collaboration level y/z based CSI prediction, UE can receive assistance signaling to switch to a specific model. In this case, the generalization Case 1 can be considered to achieve better performance. |

#### Issue#4-2 (Medium priority) Codebook type to report predicted CSI

Moderator note: In the last meeting, Qualcomm raised an issue which was missed for discussion, that “The scheme used to feedback the predicted CSI must be discussed. It must also captured in the results table template”. To Moderator’s understanding, it may mean the type of R16 CB, or R18 CB (please correct and clarify if it is incorrect understanding). So, the following question is raised to collect views from companies.

Question 4.2.1: For the AI/ML based CSI prediction, add an entry for “Table 6. Evaluation results for CSI prediction without model generalization/scalability” to report the Codebook type for CSI report.

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance [between prediction instances/distance from the last observation instance to the 1st prediction instance]) |
|  | Whether/how to adopt spatial consistency |
|  | Codebook type for CSI report |

| Support/Can accept | vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, LG Electronics, Apple, ZTE, NVIDIA, MediaTek, Ericsson, Qualcomm, AT&T, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### 4.2-2: Observations

#### Issue#4-3 (High priority) Basic performance gain for CSI prediction-benchmark 1

Moderator note: From the collected results, ZTE [4] Huawei [3] Spreadtrum [7] NVIDIA [25] ETRI [22]  Samsung [19] Fujitsu [13] CMCC [23] CATT [10] Apple [26] MediaTek [24] have shown results for FTP traffic;

Nokia [8], vivo [5] have shown results for full buffer traffic.

The following observation is given as:

Proposed Observation 4.2.2: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of the nearest historical CSI

- 13 sources [3][4][5][7][8][10][13][19][22][23][24][25][26] show that the AI/ML-based CSI prediction outperforms the benchmark

  - 8 sources [3][4][7][8][25][19][10][26] shows 10.04%~20.22% gains for layer 1 in terms of SGCS

  - 4 sources [4][19][13][26] show 2.4%~9.49% gains for layer 1 in terms of SGCS

  - 4 sources [4][19][19][26] show 22.05%~39.3% gains for layer 1 in terms of SGCS

  - 3 sources [22][23][24] show 1.06dB~9.45dB decrease in terms of NMSE

  - 3 sources [25][23][24] show 11.39dB~32.06dB decrease in terms of NMSE

  - 1 source [23] shows 38.4dB~52.53dB decrease in terms of NMSE

  - 2 sources [13][26] show 3.85%~27.43% gains for layer 2 in terms of SGCS

  - 1 source [26] shows 19.4%~140.6% gains for layer 3/4 in terms of SGCS

- 3 sources [3][5][8] show that the AI/ML-based CSI prediction outperforms the benchmark in terms of eventual KPI

  - 2 sources [3][8] show 1.2%~4.2% gains in terms of mean UPT

  - 1 source [5] shows 27.5% gains in terms of mean UPT

  - 2 sources [3][8] show 4.5%~18.3% gains in terms of 5% UPT

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, Fujitsu, ZTE, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the range of gains, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| Xiaomi | Xiaomi[15] shows 9.96dB decrease in terms of NMSE when the input sample number is 8, and 8.36dB decrease in terms of NMSE when the input sample number is 4. |
| CMCC | Our results in the row of “Gain for NMSE over Benchmark 1” is denoted as “gain over benchmark (absolute value of AI approach)”, so there might some misinterpretation. We think the following is captured correctly: - 1 source [23] shows 34.9dB~41.45dB decrease in terms of NMSE |
| Apple | Apple submitted eventual KPI in the spreadsheet directly. Can we capture it here as well? Sorry the results were not submitted in [26] on time. |
| MediaTek | We would like to mention two points: 1. In the Excel file (Table 6), the benchmark 1 we provide is based on non-AI/ML based results (auto-regression, not sample-and-hold). Therefore, please remove the results in this section, and we have provided the reorganized results in Issue#4-3a. 2. In the row “Gain for NMSE over Benchmark 1” in the Table 6, we provide the gain of NMSE, which is the decrease value of NMSE, in dB. The numbers in brackets are the absolute NMSE value obtained by AI/ML CSI prediction. Sorry for the misunderstanding. Not sure whether the meanings of CMCC and ETRI are the same with us? If so, the results need to be reorganized. |
| Qualcomm | It would be helpful to identify which results are from FTP traffic and which results are from full-buffer traffic in the observation wording. For CSI compression, full-buffer traffic results were not captured in the observation. |

#### Issue#4-3a (High priority) Basic performance gain for CSI prediction-benchmark 2

Moderator note: Huawei [3], ZTE [4], Nokia [8], vivo [5] have shown results for benchmark 2.

Proposed Observation 4.2.3: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of a non-AI/ML solution

- 2 sources [3][4] show 0.05%~12.92% gains in terms of SGCS

- 1 source [4] shows 0.3%~2.77% loss in terms of SGCS

- 3 sources [3][5][8] show 0.7%~9.7% gains in terms of mean UPT

- 2 sources [3][8] show 2.5%~15% gains in terms of 5% UPT

| Support/Can accept | vivo, Huawei/HiSi, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the range of gains, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |
| MediaTek | In the Excel file (Table 6), the benchmark 1 we provide is based on non-AI/ML based results (auto- regression, not sample-and-hold). Therefore, we reorganize our results as follows: 1 source [24] show 2.32dB~9.62dB decrease in terms of NMSE 1 source [24] show 10.1dB~14.6dB decrease in terms of NMSE |

#### Issue#4-4 (Medium priority) Generalization for CSI prediction – UE speed

Moderator note: Huawei [3], Apple [26], ZTE [4], ETRI [22], Samsung [19], Fujitsu [13], vivo [5], CMCC [23], InterDigital [12], MediaTek [24] have shown results for generalization verification over UE speeds.

show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

Proposed Observation 4.2.4: For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#112bis-e meeting,

- 7 sources [3][5][24][19][22][13][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, it has degraded performance if the model is trained with UE speed#A and applied for inference with a different UE speed#B.

  - 4 sources [3][5][24][19] show the degradation is more severe when the AI/ML model is trained with dataset subject to a smaller UE speed#A, and applied for inference to UE with a larger UE speed#B.

- 9 sources [3][5][24][26][23][19][22][13][4] show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#A and UE speed#B, and the trained AI/ML model applies inference on either UE speed#A or UE speed#B.

  - 1 source [19] observes that, if the training dataset is constructed with data samples subject to multiple UE speeds, there is still considerable degradation compared with the case where the model is trained with dataset subject to UE speed#A and applied for inference to UE with a same UE speed#A.

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, ZTE, MediaTek |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

### 4.2-3: Others

Question 4.2.2: For the evaluation of the AI/ML based CSI prediction sub use cases, what other aspects related with EVM or observation do you think is necessary to be discussed?

| Company | View |
| --- | --- |

## 2nd round email discussions

#### Issue#4-1 (High priority) Benchmark for performance comparison-modeling of level x

Moderator note: Seems relatively stable for the first round. To be presented on GTW if no further comments at the second round.

Proposal 4.3.1: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2.

| Support/Can accept | vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, CMCC, Spreadtrum, LG Electronics, Fujitsu, Apple, NVIDIA, MediaTek, AT&T, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | Based on our simulation results, the generalization of UE speed and carrier frequency is poor. Even training at a mixed dataset with various scenarios may cause the significant degradation. Therefore, if we consider the collaboration level y/z based CSI prediction, UE can receive assistance signaling to switch to a specific model. In this case, the generalization Case 1 can be considered to achieve better performance. |
| NTT DOCOMO | It is not reasonable to assume that training dataset and inference dataset are always different (generalization Case2) in collaboration level x. In the practical scenario, it is possible to collect the mixed dataset from different scenarios without any dedicated AI/ML-specific enhancement between UE and NW. Hence, generalization Case 3 seems closer to the collaboration level x where there is no dedicated AI/ML-specific enhancement such as model switching and performance monitoring. Then we suggest updating as follows. Proposal 4.3.1: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect. E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2 or Case 3. |

#### Issue#4-2 (Medium priority) Codebook type to report predicted CSI

Moderator note: Seems relatively stable for the first round. To be presented on GTW if no further comments at the second round.

Proposal 4.3.2: For the AI/ML based CSI prediction, add an entry for “Table 6. Evaluation results for CSI prediction without model generalization/scalability” to report the Codebook type for CSI report.

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance [between prediction instances/distance from the last observation instance to the 1st prediction instance]) |
|  | Whether/how to adopt spatial consistency |
|  | Codebook type for CSI report |

| Support/Can accept | vivo, CAICT, Huawei/HiSi, OPPO, Xiaomi, Spreadtrum, LG Electronics, Apple, ZTE, NVIDIA, MediaTek, Ericsson, Qualcomm, AT&T, Samsung, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#4-3 (High priority) Basic performance gain for CSI prediction-benchmark 1

Moderator note: Some updates based on comments of companies.

From the collected results, ZTE [4] Huawei [3] Spreadtrum [7] NVIDIA [25] ETRI [22]  Samsung [19] Fujitsu [13] CMCC [23] CATT [10] Apple [26] MediaTek [24] have shown results for FTP traffic;

Nokia [8], vivo [5] have shown results for full buffer traffic.

Proposed Observation 4.3.3: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of the nearest historical CSI

- 13 sources [3][4][5][7][8][10][13][19][22][23][24][25][26] show that the AI/ML-based CSI prediction outperforms the benchmark

  - 4 sources [4][19][13][26] show 2.4%~9.49% gains for layer 1 in terms of SGCS

  - 8 sources [3][4][7][8][25][19][10][26] show 10.04%~20.22% gains for layer 1 in terms of SGCS

  - 3 sources [4][19][19][26] show 22.05%~39.3% gains for layer 1 in terms of SGCS

  - 2 sources [13][26] show 3.85%~27.43% gains for layer 2 in terms of SGCS

  - 1 source [26] shows 19.42%~140.66% gains for layer 3/4 in terms of SGCS

  - 2 sources [22][15][23][24] show 0.66dB~9.96dB 1.06dB~9.45dB decrease in terms of NMSE

  - 2 sources [25][23][24] show 13.3dB~41.45dB11.39dB~32.06dB decrease in terms of NMSE

  - 1 source [23] shows 38.4dB~52.53dB decrease in terms of NMSE

- 4 sources [3][5][8][26] show that the AI/ML-based CSI prediction outperforms the benchmark in terms of eventual KPI

  - 2 sources [3][26][8] show 1.2%~4.2%8.5% gains in terms of mean UPT under FTP traffic

  - 2 sources [3] [26][8] show 4.5%~18.3%20.1% gains in terms of 5% UPT under FTP traffic

  - 2 source [5][8] shows 3%~27.5% gains in terms of mean UPT under full buffer

  - 1 source[8] show 6% gains in terms of 5% UPT under full buffer

| Support/Can accept | vivo, Huawei/HiSi, Spreadtrum, Fujitsu, ZTE, Samsung, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please companies submit your results to the spread sheet next time, otherwise your results will be missed from the observation! @Apple sorry for missing your results. Now updated. @MediaTek @CMCC corrected. @Qualcomm updated. For CSI compression, full buffer will also be drawn into observation later. |

#### Issue#4-3a (High priority) Basic performance gain for CSI prediction-benchmark 2

Moderator note: Some updates based on comments of companies.

Huawei [3], ZTE [4], Nokia [8], vivo [5], MediaTek [24] have shown results for benchmark 2.

Proposed Observation 4.3.4: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of a non-AI/ML solution, 5 sources [3][4][5][8][10][13][24] show that the AI/ML-based CSI prediction outperforms the benchmark

- 2 sources [3][4] show 0.05%~12.92% gains in terms of SGCS

- 1 source [4] shows 0.3%~2.77% loss in terms of SGCS

- 1 source [24] show 2.32dB~14.6dB decrease in terms of NMSE

- 1 source [3][5][8] shows 0.7%~9.7%3.1% gains in terms of mean UPT under FTP traffic

- 1 source [3][8] shows 2.5%~15%14.8% gains in terms of 5% UPT under FTP traffic

- 2 sources [5][8] show 2%~9.7% gains in terms of mean UPT under full buffer

- 1 source [5] shows 15% gains in terms of 5% UPT under full buffer

| Support/Can accept | vivo, Huawei/HiSi, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#4-4 (Medium priority) Generalization for CSI prediction – UE speed

Moderator note: Seems relatively stable for the first round. Continue discussion in the second round.

Huawei [3], Apple [26], ZTE [4], ETRI [22], Samsung [19], Fujitsu [13], vivo [5], CMCC [23], InterDigital [12], MediaTek [24] have shown results for generalization verification over UE speeds.

show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

Proposed Observation 4.2.4: For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#112bis-e meeting,

- 7 sources [3][5][24][19][22][13][4] show that compared to the case where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, it has degraded performance if the model is trained with UE speed#A and applied for inference with a different UE speed#B.

  - 4 sources [3][5][24][19] show the degradation is more severe when the AI/ML model is trained with dataset subject to a smaller UE speed#A, and applied for inference to UE with a larger UE speed#B.

- 9 sources [3][5][24][26][23][19][22][13][4] show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#A and UE speed#B, and the trained AI/ML model applies inference on either UE speed#A or UE speed#B.

  - 1 source [19] observes that, if the training dataset is constructed with data samples subject to multiple UE speeds, there is still considerable degradation compared with the case where the model is trained with dataset subject to UE speed#A and applied for inference to UE with a same UE speed#A.

| Support/Can accept | vivo, Huawei/HiSi, Fujitsu, ZTE, MediaTek, CMCC |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Please double check if your results are correctly categorized into the observation, and provide the changes in below if wrongly captured/missed. Highlight parts of cite “[x]” will be removed in the GTW version. |

# Templates for simulation results collection

## 1st round email discussions

#### Issue#5-1 (High priority) FFS issue on format of intermediate KPI results

Moderator note: For the templates of Table 2/3/4/5/7, there are two FFS issues on the intermediate KPI:

FFS whether the intermediate KPI results are gain over benchmark or absolute values

FFS whether the intermediate KPI results are in forms of linear or dB

From the collected results of majority of companies before this meeting, the SGCS results are reported as linear values, while the NMSE are reported in forms of dB values.

Proposed working assumption 5.1.1: For the forms of the intermediate KPI results for the following templates:

Table 2. Evaluation results for CSI compression with model generalization Table 3. Evaluation results for CSI compression with model scalability, Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, Table 7. Evaluation results for CSI prediction with model generalization

- The intermediate KPI results are in forms of absolute values (i.e., NOT the gain over benchmark)

- The intermediate KPI results are in forms of linear value for SGCS and dB value for NMSE

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, CMCC, Spreadtrum, CATT, Fraunhofer, Lenovo, Fujitsu, ZTE, NVIDIA, Ericsson, Qualcomm, AT&T |
| --- | --- |
| Object/Concern | Samsung |

| Company | View |
| --- | --- |
| Samsung | We prefer reporting the gain over the benchmark as that is easier to make a sensible comparison. A compromise would be to report both absolute values and percentage gains. |

#### Issue#5-2 (High priority) Template for CSI compression – per layer basis CSI feedback payload

Moderator note: In the last meeting, we have a WA on the ranges CSI feedback payload X/Y/Z in below:

Working Assumption For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as: - X is <=80bits - Y is 100bits-140bits - Z is >=230bits Working Assumption X, Y and Z are applicable for per layer

In this meeting, it is mentioned by Qualcomm [28] and ZTE [4] that the per layer basis X/Y/Z may be too large for rank 3/4.

Qualcomm [28] Using the maximum number of non-zero coefficients, the total PMI overhead for Release 16 eType II for parameter combination (PC) 6 is 557 bits. For Rank 4, this would imply the per-layer overhead is below 140 bits. Hence it would not be possible to fill the “Z” bin for this case using the current template. The definition of the bins (i.e., X, Y, Z ranges) for the result templates should consider whether a particular overhead is possible for the benchmark case for a given Max Rank. Hence, the bin-identifiers (X, Y and Z) should be specified for each Max Rank value separately. Alternatively, replace X/Y/Z with the overhead for eType II parameter combinations. For each Max Rank value, the overhead for Release 16 eType2 Benchmark could be computed for PC 1 through 6.

ZTE [4] However, the Z value is too large for Rank 3/4, since there are no parameter combinations of Rel-16 Type II compatible with the CSI feedback payload size Z. The X, Y and Z values for CSI feedback payload per layer are determined as: For a max rank value of 1/2 - X is <=80bits - Y is 100bits-140bits - Z is >=230bits For a max rank value of 3/4 - X is <=200/vbits - Y is 240/vbits-320/vbits - Z is >=500/vbits

The following proposal is provided accordingly.

Proposed working assumption 5.1.2: For the per layer CSI payload size X/Y/Z in the templates of CSI compression, as a clarification, the X/Y/Z ranges in the working assumption achieved in RAN1#112 meeting is applicable to Max rank = 1/2. For Max rank (v) = 3/4, the per layer basis X/Y/Z ranges are re-determined as:

- X is <=200/vbits

- Y is 240/vbits-320/vbits

- Z is >=500/vbits

| Support/Can accept | ZTE, Qualcomm (comment) |
| --- | --- |
| Object/Concern | Huawei/HiSi |

| Company | View |
| --- | --- |
| Moderator | If you are not in favor of the value ranges in the proposal, please provide your preferable value ranges. |
| Huawei/HiSi | The max CSI payload size for rank 3/4 should be comparable as the max CSI payload size for rank 2, so rank 3/4 X/Y/Z should be divide rank 2 X/Y/Z by Max rank (v). Therefore, the X/Y/Z ranges should be - X is <=160/vbits - Y is 200/vbits-280/vbits - Z is >=460/vbits |
| Qualcomm | We agree with reducing the ranges for larger max rank cases. As a suggestion, the per-layer ranges for the intermediate KPI results could be defined based on the average per-layer overhead of specific eType II parameter combinations for a rank equal to the given max-rank setting. |
| Samsung | We agree with Huawei/HiSi |

#### Issue#5-2a (High priority) Template for CSI compression–per layer basis CSI feedback payload interpretation

Moderator note: In this meeting, it is mentioned by ZTE [4] that for rank common/specific model (Option 1-1/1-2), it may not have a per layer basis CSI feedback payload X/Y/Z but rather an overall CSI feedback payload for the Max rank.

ZTE [4] For example, for a rank-common model, it’s hard to differentiate the CSI feedback payload per layer. Instead, it should be an averaged value per layer based on total CSI feedback overhead of a given max rank value. Hence, it is necessary to add a note for the template to clarify them, e.g., the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

To Moderator’s understanding, the WA already says the X/Y/Z are per layer CSI feedback payload, so it is straightforward to rank common/specific model to interpret them as the average value over the max rank. But it is no bad if we add a note to make it crystal clear.

Proposed working assumption 5.1.3: For the templates of CSI compression, add a note: for rank common/specific model (Option 1-1/1-2), the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

| Support/Can accept | Futurewei, vivo, Huawei/HiSi, CMCC, Spreadtrum, ZTE, Ericsson, Qualcomm (comment), Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Qualcomm | In our understanding, the note applies to all the rank > 1 options, so it would be more clear to just remove “for rank common/specific model (Option 1-1/1-2),” |
| Samsung | Agree with Qualcomm. |

#### Issue#5-3 (High priority) Template for the 1-on-1 table – CSI feedback overhead for eventual KPI

Moderator note: For Table 1: 1-on-1 joint training, there is a remaining issue to be revisited:

- Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting

During the discussion of the last meeting, we have aligned the CSI feedback payload X/Y/Z for per layer basis. However, as the CSI feedback overhead is calculated as the weighted average over ranks (in below), the total CSI feedback overhead also needs to be determined for reporting the eventual KPIs (e.g., mean/5% UPT).

Agreement The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

The intention of aligning the total CSI feedback overhead is to facilitate the cross check of results from companies. However, it may be inaccurate to simply multiply CSI feedback payload X/Y/Z with Max rank value, since the average rank value is smaller than the Max rank value, i.e., the weighted CSI feedback overhead is smaller than per layer basis X/Y/Z * Max rank value.

In this meeting, Huawei [3], ZTE [4], and CATT [10] raised to reconsider the values of CSI feedback overhead.

Huawei [3] For the template of AI/ML-based CSI compression of 1-on-1 joint training, the CSI feedback overhead range of eventual KPI is suggested as: Low overhead: <=β* 80 bits. Medium overhead: β* (100bits – 140 bits). High overhead: >=β* 230 bits. Note: β=1 for max rank = 1, andβ=1.4 for max rank = 2/3/4.

ZTE [4] - It is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows: Value range#1: CSI feedback overhead <= CSI feedback payload X0*Max rank value; Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead <= CSI feedback payload Y0*Max rank value; Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead <= CSI feedback payload Z0*Max rank value; - where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results.

CATT [10] Imitating the per layer payload of X, Y, Z for intermediate KPI calculation, a total payload size of A, B and C for eventual KPI can be defined and reported

Therefore, the following proposal is given:

Proposed working assumption 5.1.4: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as:

- [X*Max rank value] CSI feedback overhead A: <=β* 80 bits.

- [Y*Max rank value] CSI feedback overhead B: β* (100bits – 140 bits).

- [Z*Max rank value] CSI feedback overhead C: >=β* 230 bits.

- Note: β=1 for max rank = 1, andβ=[1.4] for max rank = 2/3/4.

| Support/Can accept | vivo, CAICT, Huawei/HiSi, CATT, Ericsson, Qualcomm (comment) |
| --- | --- |
| Object/Concern | ZTE |

| Company | View |
| --- | --- |
| Moderator | If you are not in favor of the value ofβin the proposal, please provide your preferableβvalue. |
| Futurewei | Please clarify the relationship between this proposal and Proposed working assumption 5.1.2. |
| Fujitsu | We are fine to introduce the parameter \beta. We suggest that it is up to companies to report the value of \beta in their simulation since the rank distribution is highly related to the scheduling algorithm used in simulation. |
| ZTE | We think the proposed working assumption is not a good method for CSI feedback overhead from the following two reasons: Firstly, for the CSI feedback overhead C, C >=β* 230 (322 bits) is a large range in which multiple ParamCombinations are included. For example, for Rank 4, eType II PC5 (>400 bits)-PC8(>700 bits) are included in the CSI feedback range C, and the results may differentiate among companies to a large extent and it is hard to compare the performance gain. Secondly, different companies’ simulation results have different rank distribution, which may follow different values of β. For accommodating all companies’ results, additional workload is needed for discussing the value β. Therefore, we suggest adopting the method mentioned in our contribution, - It is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows: Value range#1: CSI feedback overhead A <= CSI feedback payload X0*Max rank value; Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead B <= CSI feedback payload Y0*Max rank value; Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead C <= CSI feedback payload Z0*Max rank value; where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results. The above method can dismiss our concerns, and well echo the former mentioned ‘CSI feedback payload’ in the template. In addition, considering the values of CSI feedback payload may be differential, we suggest X0, Y0, Z0 are selected by each companies for CSI feedback payload for intermediate results, hence other companies can clearly identify the corresponding CSI feedback overhead A/B/C belonging to which ranges. |
| Intel | Does it lead to different overhead points for SGCS/NMSE and UPT? |
| Qualcomm | We have a similar question as Futurewei. The beta scaling should be applied to the same per-layer ranges for intermediate KPI results that are decided in 5.1.2. |
| Samsung | We are in general fine with the proposal. But we strongly recommend that companies additionally report the payload size they considered. Some companies have already reported the actual payload size. |

#### Issue#5-4 (High priority) Template for the 1-on-1 table – CSI overhead reduction for eventual KPI

Moderator note: For Table 1: 1-on-1 joint training in the last meeting, there is one FFS issue on how to capture the CSI overhead reduction.

- FFS: how to capture CSI overhead reduction to the template - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction.

In this meeting, the following companies provided views on how to capture CSI overhead reduction.

Huawei [3] For the template of AI/ML-based CSI compression of 1-on-1 joint training, 3 values of CSI overhead reduction can be reported corresponding to 3 CSI feedback overhead ranges to be determined in the template: “Low overhead”, “Medium overhead”, and “High overhead”, respectively. - For a given CSI feedback overhead range, it is calculated as the gap, between the CSI feedback overhead of the benchmark which should within the given CSI feedback overhead range, and the CSI feedback overhead of the AI/ML solution corresponding to the same eventual KPI. - E.g., for a given CSI feedback overhead range, a horizontal line is drawn on the figure which will have two cross points with two curves of benchmark and AI/ML, respectively, from which the CSI overhead reduction is derived. The cross point with the curve of benchmark should fall into the given CSI feedback overhead range.

ZTE [4] The CSI overhead reduction and UPT gain are calculated based on: - Method 2: For a given overhead for benchmark scheme, calculate the overhead reduction between the benchmark scheme and AI scheme in which the AI scheme has the same throughput value as the benchmark scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead. The results of CSI overhead reduction should be captured in the template for different CSI feedback payload values X/Y/Z, respectively

CATT [10] For the template for simulation results collection for AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification, the values of CSI overhead reduction can be provided for given SGCS value(s), with each SGCS value belongs to a given range

Qualcomm [28] For each bin that identifies the benchmark CSI feedback overhead, companies should report the reduction in CSI feedback overhead from the usage of AI/ML-based CSI feedback over the benchmark CSI feedback method

As Huawei, ZTE, and Qualcomm have the same view on the method, the following proposal is given based on this method.

The following proposal is given accordingly:

Proposed working assumption 5.1.5: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback reduction is provided for 3 CSI feedback overhead ranges, where for each CSI feedback overhead range of the benchmark, it is calculated as the gap between the CSI feedback overhead of benchmark and the CSI feedback overhead of AI/ML corresponding to the same mean UPT.

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, CMCC, CATT, ZTE(comments), Qualcomm, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | [X*Max rank value]/ [Y*Max rank value]/ [Z*Max rank value] in the table are to be determined by Issue#5-3, so no need to comments on those. |
| ZTE | Support with the proposal. The UPT gain calculation should be clarified for companies in addition to the CSI feedback reduction, as our figure shows. |
| Intel | We are fine/can accept if the CSI overhead reduction field is optional. |
| Ericsson | The RU is as usual valid for the baseline scheme, not for the enhancement which may result in another RU? |
| Samsung | In order to make comparison, as the Figure from ZTE shows, the UPT gain and feedback overhead reduction should be samples at the same payload size (given overhead) for benchmark scheme. Note: Companies to report the CSI feedback overhead reduction and gain for mean/5%tile UPT at the same payload size for benchmark scheme. |

#### Issue#5-5 (High priority) Confirmation of changes on the templates

Moderator note: For the templates, some post meeting changes/corrections are made by Moderator on top of the WA achieved in the 112 meeting. From the offline email discussion, no comments are received. So, these changes/corrections are brought here for a confirmation.

Proposed working assumption 5.1.6: For the initial templates achieved in the WA in RAN1#111 and RAN1#112, the following changes/corrections are made:

- For Table 1: Evaluation results for CSI compression of 1-on-1 joint training without model generalization scalability, following changes are made.

  - Sheets for FTP, Max rank=1,2,3,4

| Gain for Mean UPT (for a specific CSI feedback overhead) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |
| Gain for 5% UPT | [X*Max rank value], RU<=39% |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [X*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Y*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Z*Max rank value] RU<=39% / 40%-69% / >=70% |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [X*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Y*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Z*Max rank value] RU<=39% / 40%-69% / >=70% |

- For Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, following changes are made.

| Case 2-UE first training | UE#1 part model backbone/structure |
| --- | --- |
|  | UE#2 part model backbone/structure |
|  | UE#3 part model backbone/structure |
|  | UE#4 part model backbone/structure |
|  | NW part model backbone/structure (NW#1/NW#2/NW#3/NW#4) |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |

…

| Benchmark: NW#1-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
| --- | --- |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#2-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#3-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#4-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |

…

| Case 2-UE first training: Intermediate KPI (NW#1/NW#2/NW#3/NW#4, in case of more than one NW model) | X: <=80bits, NW-UE#1 |
| --- | --- |
|  | X: <=80bits, NW-UE#2 |
|  | X: <=80bits, NW-UE#3 |
|  | X: <=80bits, NW-UE#4 |
|  | Y: 100bits-140bits, NW-UE#1 |
|  | Y: 100bits-140bits, NW-UE#2 |
|  | Y: 100bits-140bits, NW-UE#3 |
|  | Y: 100bits-140bits, NW-UE#4 |
|  | Z: >=230bits, NW-UE#1 |
|  | Z: >=230bits, NW-UE#2 |
|  | Z: >=230bits, NW-UE#3 |
|  | Z: >=230bits, NW-UE#4 |
| Case 3-NW first training: Intermediate KPI (UE#1/UE#2/UE#3/UE#4, in case of more than one UE model) | X: <=80bits, NW#1-UE |
|  | X: <=80bits, NW#2-UE |
|  | X: <=80bits, NW#3-UE |
|  | X: <=80bits, NW#4-UE |
|  | Y: 100bits-140bits, NW#1-UE |
|  | Y: 100bits-140bits, NW#2-UE |
|  | Y: 100bits-140bits, NW#3-UE |
|  | Y: 100bits-140bits, NW#4-UE |
|  | Z: >=230bits, NW#1-UE |
|  | Z: >=230bits, NW#2-UE |
|  | Z: >=230bits, NW#3-UE |
|  | Z: >=230bits, NW#4-UE |

- For Table 6. Evaluation results for CSI prediction without model generalization/scalability, following changes are made.

  - FTP sheet

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |
|  | Whether/how to adopt spatial consistency |
| Dataset size | Train/k |
|  | Test/k |
| Benchmark 1 |  |
| SGCS of benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for SGCS over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for NMSE over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
| Benchmark 2 |  |
| SGCS of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain SGCS over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain NMSE over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU >=70%] |

  - Full buffer sheet

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |
|  | Whether/how to adopt spatial consistency |
| Dataset size | Train/k |
|  | Test/k |
| Benchmark 1 |  |
| SGCS of benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for SGCS over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for NMSE over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT [(CSI feedback payload X/Y/Z)] |
|  | 5% UPT [(CSI feedback payload X/Y/Z)] |
| Benchmark 2 |  |
| SGCS of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain SGCS over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain NMSE over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT [(CSI feedback payload X/Y/Z)] |
|  | 5% UPT [(CSI feedback payload X/Y/Z)] |

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, CMCC, Spreadtrum, CATT, Fujitsu, ZTE, NVIDIA, Qualcomm, AT&T, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#5-6 (High priority) Rank consideration for SGCS calculation

Moderator note: In this meeting, Qualcomm [28] raised an issue on how to sample the SGCS in case of rank>1:

When reporting results in this section of the table, there may be two approaches: - Option 1: Compute SGCS for each layer assuming all layers up to the maximum rank are compressed using the payload - Option 2: Compute SGCS for layer k only from channel samples when the rank is at least k

E.g., for the following table, when rank =4, Option 1 means the channel samples are collected to the testing dataset by assuming all the channel samples are subject to rank=4 (i.e., no rank selection), while Option 2 means a real SLS is run, and rank selection is performed, so that only a portion of data samples subject to the actually selected RI=4 samples are counted to the testing dataset.

| SGCS of benchmark, [layer 1] | X: <=80bits |
| --- | --- |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 2] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 3] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 4] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |

Question 5.1: For the calculation of per layer basis SGCS, compute the per layer SGCS of a given Max rank by assuming all data samples in the testing dataset is subject to the given Max rank, i.e., no rank selection is performed.

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, CMCC, Fujitsu, ZTE, Qualcomm, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| CATT | If so, this case is actually simulating per layer SGCS of a given rank. This seems different with the original intention of a ‘given max rank’. |
| Qualcomm | Is this issue not the same as Issue #5-2a? |
| NTT DOCOMO | We share the same view with CATT. We do not think it is useful to observe the performance of layers which are not compressed/reported in the actual CSI reporting. |

#### Issue#5-7 (On hold) Updated templates

Moderator note: Based on the discussions of the previous issues of this section, the 7 templates are updated as below:

Proposed working assumption 5.1.6: The templates for reporting the evaluation results of AI/ML-based CSI enhancements are updated as follows

- Evaluation results to be collected/updated before RAN1#113 and RAN1#114 meeting.

#### Issue#5-8 Other issues on the templates

Question 5.2: Except for the above issues raised in this section, do you have other comments on the correction of templates Table 1 [33] - Table 7 [39]?

| Company | View |
| --- | --- |
| Lenovo | In Table. 2, currently we have Generalization Case 1; Train (setting#A, size/k). Test (setting#A, size/k). We believe, it should be Generalization Case 1; Train (setting#AB, size/k). Test (setting#AB, size/k). Such that it has comparable results with the test-cases that we are reporting after for generalization. Also In Table. 5, we have: Case 1-NW first training; NW part model backbone/structure (NW#1/NW#2/NW#3/NW#4). UE#1 part model backbone/structure. UE#1 part training dataset description and size. UE#2 part model backbone/structure. UE#2 part training dataset description and size. UE#3 part model backbone/structure. UE#3 part training dataset description and size. UE#4 part model backbone/structure. UE#4 part training dataset description and size. [air-interface overhead of information (e.g., dataset) sharing]. We believe this table can be use for “UE first training” as well when different UE has different model backbone/structure or dataset. We understand that there is another rows for “case-1 UE first training” but they are mainly differ in the NW side not the UE side. For example, it is not possible to show the dataset of different UEs. So we suggest repeating “Case 1-NW first training” for “UE first training” as well. or adding similar rows of “Case 1-NW first training” to “case-1 UE first training” and similar rows of “case-1 UE first training to “Case 1-NW first training”. |
| ZTE | First, whether detailed CSI payload value X/Y/Z should be filled in the table accompanied with the intermediate/eventual KPI for more clear demonstration, e.g. Second, in the same sheet, whether the CSI payload value X/Y/Z are mandated the same for different fields in the table, e.g. SGCS and UPT gain. Third, for the table of generalization, how to fill in the table for generalization Case2? For example, generalization Case 1 is training with A and testing for A, while we find Case 2 has two options in the same column: - Option1: training with A, testing for B - Option2: training with B, testing for A We should have a common understanding on providing generalization/scalability results. |

## 2nd round email discussions

#### Issue#5-1 (High priority) FFS issue on format of intermediate KPI results

Moderator note: Continue discussion in the second round.

Proposed working assumption 5.2.1: For the forms of the intermediate KPI results for the following templates:

Table 2. Evaluation results for CSI compression with model generalization Table 3. Evaluation results for CSI compression with model scalability, Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, Table 7. Evaluation results for CSI prediction with model generalization

- The intermediate KPI results are in forms of absolute values (i.e., NOT the gain over benchmark)

- The intermediate KPI results are in forms of linear value for SGCS and dB value for NMSE

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, CMCC, Spreadtrum, CATT, Fraunhofer, Lenovo, Fujitsu, ZTE, NVIDIA, Ericsson, Qualcomm, AT&T |
| --- | --- |
| Object/Concern | Samsung |

| Company | View |
| --- | --- |
| Samsung | We prefer reporting the gain over the benchmark as that is easier to make a sensible comparison. A compromise would be to report both absolute values and percentage gains. |
| Moderator | @Samsung From Table 2-5, and Table 7, they are comparison over different AI/ML solutions (different from Table 1/6, which are used to compare AI/ML and non-AI benchmark). Is there strong need to introduce an absolute value for Table 2-5, and 7? |

#### Issue#5-2 (High priority) Template for CSI compression – per layer basis CSI feedback payload

Moderator note: Changes according to Huawei’s comments in the first round.

Proposed working assumption 5.2.2: For the per layer CSI payload size X/Y/Z in the templates of CSI compression, as a clarification, the X/Y/Z ranges in the working assumption achieved in RAN1#112 meeting is applicable to Max rank = 1/2. For Max rank (v) = 3/4, the per layer basis X/Y/Z ranges are re-determined as:

- X is <=160/vbits

- Y is 200/vbits-280/vbits

- Z is >=460/vbits

| Support/Can accept | ZTE, Qualcomm (comment) |
| --- | --- |
| Object/Concern | Huawei/HiSi |

| Company | View |
| --- | --- |
| Moderator | @ZTE see if the suggested values from Huawei is acceptable or not. @Qualcomm see if the current WA is acceptable. |

#### Issue#5-2a (High priority) Template for CSI compression–per layer basis CSI feedback payload interpretation

Moderator note: Changes according to Qualcomm’s comments in the first round.

Proposed working assumption 5.2.3: For the templates of CSI compression, add a note: for rank common/specific model (Option 1-1/1-2), the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

| Support/Can accept | Futurewei, vivo, Huawei/HiSi, CMCC, Spreadtrum, ZTE, Ericsson, Qualcomm (comment), Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Qualcomm | In our understanding, the note applies to all the rank > 1 options, so it would be more clear to just remove “for rank common/specific model (Option 1-1/1-2),” |
| Samsung | Agree with Qualcomm. |

#### Issue#5-3 (High priority) Template for the 1-on-1 table – CSI feedback overhead for eventual KPI

Moderator note: Two options are given based on the comments of companies in the first round.

Proposed working assumption 5.2.4: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as:

- Option 1:

  - [X*Max rank value] CSI feedback overhead A: <=β* 80 bits.

  - [Y*Max rank value] CSI feedback overhead B: β* (100bits – 140 bits).

  - [Z*Max rank value] CSI feedback overhead C: >=β* 230 bits.

  - Note: β=1 for max rank = 1, andβ=[1.4] for max rank = 2/3/4.

- Option 2: CSI feedback overhead A/B/C is calculated from the per layer basis X/Y/Z, respectively, based on the X/Y/Z and the rank distribution. Values of A/B/C are reported by companies.

  - E.g., assume MAX rank value is K, the percentage of rank k is distributed as Dk, the per layer CSI payload size is selected as Xi, i=1,…k, where i is the layer index, then the total CSI feedback payload calculated from per layer X is k=1K(Dk*i=1kXi)

| Option 1 | Support/Can accept |  |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | Fujitsu |
|  | Object/Concern |  |
| Other | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Futurewei The Issue#5-2 addresses the definition of the per layer basis CSI payload X/Y/Z range for rank 3/4; note this per layer basis X/Y/Z is used for the calculation of intermediate KPI for each layer. This issue addresses the alignment of the overall CSI feedback overhead for the eventual KPI. @Fujitsu @ZTE @Samsung as mentioned in the moderator note in the first round “The intention of aligning the total CSI feedback overhead is to facilitate the cross check of results from companies.” If we allow companies to report the CSI feedback overhead by per company basis, then how can we cross check the results? E.g., if one company simulate 10% gain on 100 bits overhead (with 80% rank1), while another company simulate 20% gain on 180 bits overhead (with 80% rank2), both based on the same per layer basis Y. See if Option 2 is your preference. @ZTE for PC5/8 – they are the max payload value, right? Here we are discussing the average weighted CSI feedback overhead. Would rank 3/4 occupy a big portion in the simulation to lead to such a high average overhead? @ Intel yes, it will do. We either choose to align the per layer basis X/Y/Z for intermediate KPI and apply it to the calculation of the eventual KPI, with the result that the average CSI feedback overhead over companies are not aligned (due to rank distributions), or, choose to decouple the CSI payload X/Y/Z for intermediate KPI and the CSI feedback overhead for UPT, where the overhead over companies can be somehow aligned. @Qualcomm then how would we decide the average CSI feedback overhead for the eventual KPI? The average rank value is not equal to the Max rank value, right? Gain for Mean UPT (for a specific CSI feedback overhead); [X*Max rank value], RU<=39%. [Y*Max rank value], RU<=39%. [Z*Max rank value], RU<=39%. [X*Max rank value], RU 40%-69%. [Y*Max rank value], RU 40%-69%. [Z*Max rank value], RU 40%-69%. [X*Max rank value], RU >=70%. [Y*Max rank value], RU >=70%. [Z*Max rank value], RU >=70%. |
| Fujitsu | @FL. “E.g., if one company simulate 10% gain on 100 bits overhead (with 80% rank1), while another company simulate 20% gain on 180 bits overhead (with 80% rank2), both based on the same per layer basis Y. See if Option 2 is your preference.” Under the same assumption, e.g., max rank, and Y, we think the results from companies can be averaged. In this example, an average gain of 15% can be reported. We also think that it is not necessary to clarify the gain achieved by each individual rank. The focus of this evaluation is not to align the rank distribution among companies’ SLS. If the gain obtained by each individual rank needs to be clarified, additional simulation assuming fixed rank can be considered. Therefore, we support Option 2 to save the efforts for aligning rank distribution (sensitive to scheduling method). |

#### Issue#5-4 (High priority) Template for the 1-on-1 table – CSI overhead reduction for eventual KPI

Moderator note: For Table 1: 1-on-1 joint training in the last meeting, there is one FFS issue on how to capture the CSI overhead reduction.

The following proposal is given accordingly:

Proposed working assumption 5.2.5: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback reduction is provided for 3 CSI feedback overhead ranges, where for each CSI feedback overhead range of the benchmark, it is calculated as the gap between the CSI feedback overhead of benchmark and the CSI feedback overhead of AI/ML corresponding to the same mean UPT.

- Note: the CSI feedback overhead reduction and gain for mean/5%tile UPT are determined at the same payload size for benchmark scheme

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, CMCC, CATT, ZTE(comments), Qualcomm, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @ZTE @Samsung Do we need to make a restriction to couple the UPT and the CSI overhead reduction? It is Moderator’ understanding that these two metrics can be separately counted. But anyway a note is added. @Ericsson Good question. It is Moderator’s understanding that RU is for benchmark. But typically the RU between two schemes will not differ a lot? |

#### Issue#5-5 (High priority) Confirmation of changes on the templates

Moderator note: Seems relatively stable for the first round. Continue discussion in the second round.

Proposed working assumption 5.2.6: For the initial templates achieved in the WA in RAN1#111 and RAN1#112, the following changes/corrections are made:

- For Table 1: Evaluation results for CSI compression of 1-on-1 joint training without model generalization scalability, following changes are made.

  - Sheets for FTP, Max rank=1,2,3,4

| Gain for Mean UPT (for a specific CSI feedback overhead) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |
| Gain for 5% UPT | [X*Max rank value], RU<=39% |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [X*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Y*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Z*Max rank value] RU<=39% / 40%-69% / >=70% |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [X*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Y*Max rank value] RU<=39% / 40%-69% / >=70% |
|  | [Z*Max rank value] RU<=39% / 40%-69% / >=70% |

- For Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, following changes are made.

| Case 2-UE first training | UE#1 part model backbone/structure |
| --- | --- |
|  | UE#2 part model backbone/structure |
|  | UE#3 part model backbone/structure |
|  | UE#4 part model backbone/structure |
|  | NW part model backbone/structure (NW#1/NW#2/NW#3/NW#4) |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |

…

| Benchmark: NW#1-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
| --- | --- |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#2-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#3-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Benchmark: NW#4-UE#1/UE#2/UE#3/UE#4 joint training: Intermediate KPI | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |

…

| Case 2-UE first training: Intermediate KPI (NW#1/NW#2/NW#3/NW#4, in case of more than one NW model) | X: <=80bits, NW-UE#1 |
| --- | --- |
|  | X: <=80bits, NW-UE#2 |
|  | X: <=80bits, NW-UE#3 |
|  | X: <=80bits, NW-UE#4 |
|  | Y: 100bits-140bits, NW-UE#1 |
|  | Y: 100bits-140bits, NW-UE#2 |
|  | Y: 100bits-140bits, NW-UE#3 |
|  | Y: 100bits-140bits, NW-UE#4 |
|  | Z: >=230bits, NW-UE#1 |
|  | Z: >=230bits, NW-UE#2 |
|  | Z: >=230bits, NW-UE#3 |
|  | Z: >=230bits, NW-UE#4 |
| Case 3-NW first training: Intermediate KPI (UE#1/UE#2/UE#3/UE#4, in case of more than one UE model) | X: <=80bits, NW#1-UE |
|  | X: <=80bits, NW#2-UE |
|  | X: <=80bits, NW#3-UE |
|  | X: <=80bits, NW#4-UE |
|  | Y: 100bits-140bits, NW#1-UE |
|  | Y: 100bits-140bits, NW#2-UE |
|  | Y: 100bits-140bits, NW#3-UE |
|  | Y: 100bits-140bits, NW#4-UE |
|  | Z: >=230bits, NW#1-UE |
|  | Z: >=230bits, NW#2-UE |
|  | Z: >=230bits, NW#3-UE |
|  | Z: >=230bits, NW#4-UE |

- For Table 6. Evaluation results for CSI prediction without model generalization/scalability, following changes are made.

  - FTP sheet

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |
|  | Whether/how to adopt spatial consistency |
| Dataset size | Train/k |
|  | Test/k |
| Benchmark 1 |  |
| SGCS of benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for SGCS over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for NMSE over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
| Benchmark 2 |  |
| SGCS of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain SGCS over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain NMSE over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | Mean UPT [(CSI feedback payload X/Y/Z), RU >=70%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU<=39%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU 40%-69%] |
|  | 5% UPT [(CSI feedback payload X/Y/Z), RU >=70%] |

  - Full buffer sheet

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |
|  | Whether/how to adopt spatial consistency |
| Dataset size | Train/k |
|  | Test/k |
| Benchmark 1 |  |
| SGCS of benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for SGCS over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for NMSE over Benchmark 1 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT [(CSI feedback payload X/Y/Z)] |
|  | 5% UPT [(CSI feedback payload X/Y/Z)] |
| Benchmark 2 |  |
| SGCS of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain SGCS over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| NMSE of Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain NMSE over Benchmark 2 (1,…N, N is number of prediction instances) |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT [(CSI feedback payload X/Y/Z)] |
|  | 5% UPT [(CSI feedback payload X/Y/Z)] |

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, Xiaomi, CMCC, Spreadtrum, CATT, Fujitsu, ZTE, NVIDIA, Qualcomm, AT&T, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#5-6 (High priority) Rank consideration for SGCS calculation

Moderator note: Seems relatively stable for the first round. Continue discussion in the second round.

Question 5.2.7: For the calculation of per layer basis SGCS, compute the per layer SGCS of a given Max rank by assuming all data samples in the testing dataset is subject to the given Max rank, i.e., no rank selection is performed.

| Support/Can accept | Futurewei, vivo, CAICT, Huawei/HiSi, CMCC, Fujitsu, ZTE, Qualcomm, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| CATT | If so, this case is actually simulating per layer SGCS of a given rank. This seems different with the original intention of a ‘given max rank’. |
| Qualcomm | Is this issue not the same as Issue #5-2a? |
| Moderator | @CATT but the SGCS is per layer value, so it does no matter on the rank distribution, right? @Qualcomm Both proposals are from Qualcomm’s Tdoc. It is Moderator’s understanding that this issue addresses the issue of how to generate the test dataset to calculate SGCS, while 5-2a addresses the issue of the per layer CSI payload interpretation. Please clarify your intention on the two proposals if there is misunderstanding. |

# Potential proposals for GTW/offline

## 19 Apr. (Tuesday) GTW

#### Issue#3-8 (Medium priority) rank>1 options-complexity report

Proposal 3.3.6: For the rank >1 options under AI/ML-based CSI compression, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

#### Issue#3-9 (Medium priority) rank>1 options-storage report

Proposal 3.3.7: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

#### Issue#4-3 (High priority) Basic performance gain for CSI prediction-benchmark 1

Moderator note: From the collected results, ZTE [4] Huawei [3] Spreadtrum [7] NVIDIA [25] ETRI [22] Samsung [19] Fujitsu [13] CMCC [23] CATT [10] Apple [26] have shown results for FTP traffic;

Nokia [8], vivo [5] have shown results for full buffer traffic.

Proposed Observation 4.3.3: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of the nearest historical CSI

- 12 sources show that the AI/ML-based CSI prediction outperforms the benchmark, wherein

  - 4 sources show 2.4%~9.49% gains for layer 1 in terms of SGCS

  - 8 sources show 10.04%~20.22% gains for layer 1 in terms of SGCS

  - 3 sources show 22.05%~39.3% gains for layer 1 in terms of SGCS

  - 2 sources show 3.85%~27.43% gains for layer 2 in terms of SGCS

  - 1 source shows 19.42%~140.66% gains for layer 3/4 in terms of SGCS

  - 2 sources show 0.66dB~9.96dB decrease in terms of NMSE

  - 2 sources show 13.3dB~41.45dB decrease in terms of NMSE

- 4 sources show that the AI/ML-based CSI prediction outperforms the benchmark in terms of eventual KPI, wherein

  - 2 sources show 1.2%~8.5% gains in terms of mean UPT under FTP traffic

  - 2 sources show 4.5%~20.1% gains in terms of 5% UPT under FTP traffic

  - 2 source shows 3%~27.5% gains in terms of mean UPT under full buffer

  - 1 source shows 6% gains in terms of 5% UPT under full buffer

#### Issue#4-3a (High priority) Basic performance gain for CSI prediction-benchmark 2

Moderator note: Huawei [3], ZTE [4], Nokia [8], vivo [5], MediaTek [24] have shown results for benchmark 2.

Proposed Observation 4.3.4: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, compared to the benchmark of a non-AI/ML solution, 5 sources show that the AI/ML-based CSI prediction outperforms the benchmark

- 2 sources show 0.05%~12.92% gains in terms of SGCS

- 1 source shows 0.3%~2.77% loss in terms of SGCS

- 1 source shows 2.32dB~14.6dB decrease in terms of NMSE

- 1 source shows 0.7%~3.1% gains in terms of mean UPT under FTP traffic

- 1 source shows 2.5%~14.8% gains in terms of 5% UPT under FTP traffic

- 2 sources show 2%~9.7% gains in terms of mean UPT under full buffer

- 1 source shows 15% gains in terms of 5% UPT under full buffer

#### Issue#3-14 (High priority) Generalization for CSI compression – deployment scenario

Moderator note: From the collected results for Table 2,

Mavenir [18], Huawei [3], Ericsson [14], FUTUREWEI [2], Lenovo [27], Intel [11], vivo [5], OPPO [6], Fujitsu [13], CMCC [23], CATT [10], Xiaomi [15], ZTE [4], NTT DOCOMO [30], InterDigital [12] submitted the results on the generalization over deployment scenarios (UMa/UMi/InH, etc.).

Proposed Observation 3.3.16: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#112bis-e meeting,

- 15 sources show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

  - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, deployment scenario#A is UMi, deployment scenario#B is UMa, or deployment scenario#A is InH, deployment scenario#B is UMa/UMi.

  - 6 sources observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor.

    - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, or deployment scenario#A is UMi, deployment scenario#B is UMa.

- 5 sources show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#A and deployment scenario#B, and the trained AI/ML model applies inference on either deployment scenario#A or deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa and/or UMi.

- 3 sources show that, compared to the case where the AI/ML model is trained on scenario#A and applied for inference on deployment scenario#B, the generalization performance can be improved, if the AI/ML model, after trained on deployment scenario#A, is updated based on a fine-tuned dataset subject to deployment scenario#B, and performs inference on deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa or UMi.

#### Issue#5-1 (High priority) FFS issue on format of intermediate KPI results

Proposed working assumption 5.2.1: For the forms of the intermediate KPI results for the following templates:

Table 2. Evaluation results for CSI compression with model generalization, Table 3. Evaluation results for CSI compression with model scalability, Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, Table 7. Evaluation results for CSI prediction with model generalization

- The intermediate KPI results are in forms of absolute values (i.e., NOT the gain over benchmark)

- The intermediate KPI results are in forms of linear value for SGCS and dB value for NMSE

#### Issue#4-1 (High priority) Benchmark for performance comparison-modeling of level x

Proposal 4.3.1: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2.

#### Issue#4-2 (Medium priority) Codebook type to report predicted CSI

Proposal 4.3.2: For the AI/ML based CSI prediction, add an entry for “Table 6. Evaluation results for CSI prediction without model generalization/scalability” to report the Codebook type for CSI report.

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance [between prediction instances/distance from the last observation instance to the 1st prediction instance]) |
|  | Whether/how to adopt spatial consistency |
|  | Codebook type for CSI report |

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2302319	Discussion and evaluation of AI/ML for CSI feedback enhancement FUTUREWEI

- R1-2302358	Evaluation on AI/ML for CSI feedback enhancement  Huawei, HiSilicon

- R1-2302437	Evaluation on AI for CSI feedback enhancement	ZTE

- R1-2302477	Evaluation on AI/ML for CSI feedback enhancement  vivo

- R1-2302540	Evaluation methodology and results on AI/ML for CSI feedback enhancement  OPPO

- R1-2302593  Discussion on evaluation on AIML for CSI feedback enhancement  Spreadtrum Communications, BUPT

- R1-2302628  Evaluation of ML for CSI feedback enhancement  Nokia, Nokia Shanghai Bell

- R1-2302637  Evaluation of AI/ML based methods for CSI feedback enhancement  Fraunhofer IIS

- R1-2302695	Evaluation on AI/ML for CSI feedback enhancement  CATT

- R1-2302790  Evaluation for CSI feedback enhancements  Intel Corporation

- R1-2302822	Evaluation on AI/ML for CSI feedback enhancement  InterDigital, Inc.

- R1-2302904  Evaluation on AI/ML for CSI feedback enhancement  Fujitsu

- R1-2302918	Evaluations of AI-CSI	Ericsson

- R1-2302975	Discussion on evaluation on AI/ML for CSI feedback enhancement  xiaomi

- R1-2303050  On Evaluation of AI/ML based CSI  Google

- R1-2303076  Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2303087 	Evaluation on AI for CSI feedback enhancement	Mavenir

- R1-2303120	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2303174	Evaluation of AI and ML for CSI feedback enhancement	RAN1, Comba

- R1-2303183  Some discussions on evaluation on AI-ML for CSI feedback	 CAICT

- R1-2303194  Evaluation on AI/ML for CSI feedback enhancement  ETRI

- R1-2303224  Discussion on evaluation on AI/ML for CSI feedback enhancement  CMCC

- R1-2303336  Evaluation on AI/ML for CSI feedback enhancement  MediaTek Inc.

- R1-2303435  Evaluation of AI and ML for CSI feedback enhancement  NVIDIA

- R1-2303475  Evaluation for AI/ML based CSI feedback enhancement  Apple

- R1-2303524  Evaluation on AI/ML for CSI feedback  Lenovo

- R1-2303582  Evaluation on AI/ML for CSI feedback enhancement  Qualcomm Incorporated

- R1-2303654	Discussion on AI/ML for CSI feedback enhancement	AT&T

- R1-2303705  Discussion on evaluation on AI/ML for CSI feedback enhancement  NTT DOCOMO, INC.

- R1-2303776  Evaluation on AI/ML for CSI feedback enhancement  Indian Institute of Tech (H)

- R1-2303849  Evaluation on AI/ML for CSI Feedback Enhancement  IIT Kanpur

- Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%201.%201-on-1%20joint%20training_v023_ChinaTelecom_MediaTek.xlsx

- Table 2. Evaluation results for CSI compression with model generalization, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%202.%20Evaluation%20results%20for%20CSI%20compression%20with%20model%20generalization_V018_CATT_InterDigital.xlsx

- Table 3. Evaluation results for CSI compression with model scalability, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%203.%20Evaluation%20results%20for%20CSI%20compression%20with%20model%20scalability_v012_InterDigital_MediaTek.xlsx

- Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%204.%20multi-vendor%20joint%20training_v007_InterDigital_MediaTek.xlsx

- Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%205.%20separate%20training_v011_CATT_MediaTek.xlsx

- Table 6. Evaluation results for CSI prediction without model generalization/scalability, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%206.%20CSI%20prediction%20without%20generalization_v015_Apple2_MediaTek.xlsx

- Table 7. Evaluation results for CSI prediction with model generalization, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/drafts/9.2(FS_NR_AIML_air)/9.2.2.1/CSIenh_eval_results_FIN/Table%207.%20CSI%20prediction%20with%20generalization_v010_InterDigital_MediaTek.xlsx

- China Telecom

# Appendix I: Agreement list

### Agreements of the 109-e meeting

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement (modified by May 23rd post)

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

Note: Above agreement is updated as follows

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

### Agreements of the 110 meeting

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

### Agreements of the 110bis-e meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded. |
| --- | --- |

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy,

- Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

  - The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook

Note: there is no restriction on model training

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook (if it outperforms Type II Codebook) can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the intermediate KPI is calculated for each prediction instance

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations:

- Raw channel matrixes

- Eigenvector(s)

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the evaluation of CSI prediction:

- Companies are encouraged to report the assumptions on the observation window, including number/time distance of historic CSI/channel measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the prediction window, including number/time distance of predicted CSI/channel as the output of the AI/ML model

Conclusion

If ideal DL channel estimation is considered (which is optional) for the evaluations of CSI feedback enhancement, there is no consensus on how to use the ideal channel estimation for dataset construction, or performance evaluation/inference.

- It is up to companies to report whether/how ideal channel is used in the dataset construction as well as performance evaluation/inference.

Conclusion

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example:

- For each FP/BP loop,

  - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

  - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

  - Step 3: UE side trains the CSI generation part based on the BP information from NW side

- Note: the dataset between UE side and NW side is aligned.

- Other Type 2 training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training):

- Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly

- Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the received set of information

- Other Type 3 NW-first training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training):

- Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

- Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part

- Step3: NW side trains the NW side CSI reconstruction part based on the received set of information

- Other Type 3 UE-first training approaches are not precluded and reported by companies

Working assumption

In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors

- Note: Eventual KPI can still be used to compare the performance

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2.

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Agreement

In CSI compression using two-sided model use case, evaluate and study quantization of CSI feedback, including at least the following aspects:

- Quantization non-aware training

- Quantization-aware training

- Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc.

- How to use the quantization methods

Agreement

For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options

- High resolution scalar quantization, e.g., Float32, Float16, etc.

    - FFS select one of the scalar quantization resolutions as baseline

- High resolution codebook quantization, e.g., R16 Type II-like method with new parameters

    - FFS new parameters

- Other quantization methods are not precluded

Agreement

For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement which is optionally considered by companies, the following case is taken

- The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

- Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

- Case 2: Not aligned AI/ML model structures between NW side and UE side

    - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers)

- FFS different sizes of datasets between NW side and UE side

- FFS aligned/different quantization/dequantization methods between NW side and UE side

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

Agreement

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

- Case 1 (baseline): Type 2 training between one NW part model to one UE part model

- Case 2: Type 2 training between one NW part model and M>1 separate UE part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models

- Case 3: Type 2 training between one UE part model and N>1 separate NW part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models

- FFS N NW part models to M UE part models

- FFS different quantization/dequantization methods between NW and UE

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

- FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models

- FFS: whether/how to report overhead

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations

- Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit

    - Companies to report the raw channel is in frequency domain or delay domain

- Precoding matrix

    - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation)

- Other input/output types are not precluded

- Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part),

    - Note: the input and output may be of different types

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for SLS, spatial consistency procedure A with 50m decorrelation distance from 38.901 is used (if not used, company should state this in their simulation assumptions)

- UE velocity vector is assumed as fixed over time in Procedure A modeling

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of intermediate KPI, the following is considered as the granularity of the frequency unit for averaging operation

- For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs

- For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs

- Note: Other frequency unit granularity is not precluded and reported by companies

### Agreements of the 111 meeting

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS the value or range of payload size X/Y/Z

- FFS the description and results for different training types/cases may need a separate table

- FFS: training related overhead

Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#1, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#2, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#2, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Gain for Mean UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for 5% UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors

- Case 1 (baseline): Type 3 training between one NW part model and one UE part model

  - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model

  - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model

  - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different

  - FFS: different quantization methods between NW side and UE side

- Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models

  - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models

  - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model

  - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models

- Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

  - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

  - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

  - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

- FFS: whether/how to report overhead of dataset

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example

  - It does not imply any restriction on future specification for CSI prediction

- FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

- Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

- Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

- Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

- Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization.

- Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison.

Agreement

For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies:

- Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

  - Fixed/pre-configured quantization method/parameters is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

- Case 2: Quantization aware training, where quantization/dequantization is involved in the training process

  - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

  - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase

    - Companies to report how to update the quantization method/parameters during the training

- Note: the above cases apply for training Type 1/2/3

- Others are not precluded.

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2

- For NW-first training

  - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization.

- For UE-first training

  - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Working Assumption

For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS whether/how to capture the muliptle predicted CSI instances and their mapping to slots

Table X. Evaluation results for CSI prediction without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | UE speed |  |  |
|  | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Benchmark 1 |  |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 1 |  |  |  |
| Intermediate KPI #2 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| Benchmark 2 |  |  |  |
| Intermediate KPI #1 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 2 |  |  |  |
| Intermediate KPI #2 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 2 |  |  |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| FFS others |  |  |  |

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated

- Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions.

- Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions

- Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions

- Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc.

### Agreements of the 112 meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Agreement

Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

Conclusion

For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Agreement

For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison

Agreement

Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

    - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, ect.

Agreement

For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison.

- FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

  - FFS: input/output type

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is

    - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

    - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is

    - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

    - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

- Other options not precluded.

Agreement

The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

- For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank.

- For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB

  - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank

Working Assumption

For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as:

- X is <=80bits

- Y is 100bits-140bits

- Z is  >=230bits

X, Y and Z are applicable for per layer

Working assumption

The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS the description and results for generalization/scalability may need a separate table

- Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting

- FFS: training related overhead

- FFS: how to capture CSI overhead reduction to the template

  - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction.

Table X. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AI/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AI/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
|  | Rank/layer adaptation settings for rank>1 |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) |  |  |  |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |  |  |
| SGCS of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| SGCS of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| NMSE of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| NMSE of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| Other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for Mean UPT (for a specific CSI feedback overhead) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for 5% UPT | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| [CSI feedback reduction (%)] |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

- Note: “Benchmark” means the type of Legacy CB used for comparison.

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working assumption

A separate table to capture the evaluation results of generalization/scalability verification for AI/ML-based CSI compression is given in the following initial template

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
| SGCS, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| SGCS, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other settings for Case 1) |  |  |  |
| … |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Case 2) |  |  |  |
| … (other settings for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
| … (results for Case 3) |  |  |  |
| … (other settings for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Fine-tuning) |  |  |  |
| … (other settings for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model description (e.g., backbone, structure) |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 1) |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | [Training method] |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | NW part model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | UE part model backbone/structure |  |  |
|  | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction par

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for sequentially separate training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of separate training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Shared output of CSI generation part/input of reconstruction part is before or after quantization |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| [Benchmark: NW#1-UE#1 joint training] | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
|  | Training dataset size |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AI/ML model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
|  | UE part model backbone/structure |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 2-UE first training | UE#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AI/ML model backbone/structure |  |  |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |  |  |
| Case 3-NW first training | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
|  | UE part training dataset description and size (e.g., description/size of dataset from N NWs and how to merge) |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| NW#1-UE#1 joint training: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other 1-on-1 NW-UE joint training combinations) |  |  |  |
| Case 1-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.