3GPP TSG RAN WG1 Meeting #112bis-e	R1-2304010

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #1 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

25 contributions [2-26] have been submitted to Agenda Item 9.1.3.2 of RAN1#112bis-e on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

## Comb offset hopping and cyclic shift hopping

### Time-domain hopping behavior

- We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

The general positions regarding the 3 options are:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

  - Supporting: Ericsson, Futurewei (FFS for comb offset hopping), InterDigital, Qualcomm, Samsung, Sharp, Spreadtrum, vivo (based on configuration), xiaomi (for comb offset hopping), ZTE.

  - 10 proponents.

Option 2: Within a slot, hopping based on only the symbol index l'.

  - Supporting: Apple, CATT, CMCC, ETRI, Futurewei, Google, Huawei, HiSilicon, Lenovo, NEC, New H3C, Nokia, Nokia Shanghai Bell, OPPO (prefer), Qualcomm (except for positioning R18 comb offset hopping), vivo (based on configuration), xiaomi (for cyclic shift hopping).

  - 16 17 proponents.

Option 3: No intra-slot hopping.

  - Supporting: Intel

  - Not supporting: CMCC, Ericsson

  - 1 proponent and 2 opponents

I suggest to narrow down to Options 1 and 2.

When repetition is not configured, i.e., R=1, both options have the same behavior, and it is already covered by agreement “the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l'”. If needed we can further confirm this in an agreement.

When repetition is configured, i.e., R>1, there are different considerations for the different preferences of Options 1 and 2:

  - Some companies are mainly concerned with the multiplexing orthogonality when hopping is enabled, for SRSs that may be configured with different periodicities, symbol locations, etc. They prefer the hopping pattern to be based on the frame/slot/symbol structures instead of UE-specific / SRS-specific configurations. An example of this design is to have the similar hopping behavior as SRS sequence hopping / sequence group hopping, which in the cases of SRS repetition, would still hop to different resources.

  - Some other companies are mainly concerned with the joint processing simplicity when hopping is enabled, and therefore, on the R symbols with SRS repetition, would use the same resources. This may be especially beneficial to comb offset hopping, but seems insignificant to cyclic shift hopping.

  - Qualcomm also discussed that Option 2 for Rel-18 comb offset hopping may not be applicable to positioning SRS. We note that positioning SRS already supports legacy comb offset hopping, so it does not need to support Rel-18 comb offset hopping.

The group can further discuss these two options for R>1. The following proposal is suggested as a starting point which attempts to incorporate both options as much as possible:

Proposal 2.1.1: For a SRS resource configured with comb offset hopping and/or cyclic shift hopping,

- If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l'.

- If the repetition factor R > 1, at least support Option 2: Within a slot, hopping based on only the symbol index l'.

  - In addition, for comb offset hopping, gNB can configure Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as l'/R.

  - FFS: Rel-18 comb offset hopping does not apply to positioning SRS.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support in principle. We think the FFS should be removed, we do not know why this is mentioned. |
| QC | - For Option 1, l'/R may not be integer, and it does not ensure “symbol index that is the same across the R repetitions” from the description and intention of Option 1 in the agreement. Instead, “symbol index l' of the first symbol across the R repetitions.” is needed. - For the FFS, we think the scheme can be also applicable to positioning with option 1 (instead of R, nrofSymbols can be used). We can instead say “Option 2 is not applicable to SRS for positioning” - @FL: Different comb offset in SRS for positioning in legacy is not for randomization. It has a specific uniform structure for SRS staggering (to sound as many comb offsets as possible within a SRS resource uniformly) |
| InterDigital | Agree with Qualcomm’s modification of Option 1. |
| OPPO | We prefer Option 2. If option 2 is mandatory, we don’t think a UE supporting option 2 is needed to additionally support option 1. |
| NEC | We prefer Option 2. |
| Lenovo | We prefer option 2. It is desirable to down-select one scheme for simplicity if there is not much performance difference. On account that this is a compromise proposal, we can live up with it. |
| Spreadtrum | Fine with QC’s modification. |
| ZTE | - Support proposal 2.1.1. Agree with Qualcomm’s modification of Option 1. - For CS hopping, Option 1 is beneficial for maintaining orthogonality among multiple ports over a common symbol. - For comb offset hopping, Option 2 is beneficial for channel estimation accuracy. |
| CMCC | - Support this proposal and agree with QC’s modification. |
| Huawei, HiSilicon | Prefer option 2. - Whether the enhancement introduced in this WID should be expanded to other scenarios should be separately discussed. |
| CATT | We prefer option 2, and don’t see the necessity to additionally support option 1. |
| Nokia/NSB | Support with QC’s modification. |
| Intel | Our preference is no intra slot hopping since it would be complicated to multiplex with legacy UE. But we could be fine with Option 2. |
| vivo | We also support Option 1 for CS hopping, since it can simplify the complexity of UE to generate too many SRS sequences with per symbol hopping. In our understanding, UE can report its capability, then gNB configures whether SRS is hopping per R symbols or each symbol. |
| Xiaomi | Support this proposal with QC’s modification. |
| MediaTek | Agree with QC’s modifications. |
| Sharp | Support with QC’s modification. |
| DOCOMO | Ok with QC’s modification |
| New H3C | Ok with QC’s modification |
| Samsung | Support the FL’s proposal with QC’s modification. |
| Apple | In principle, we support |
| FL | Minor update above on the supporting situation. Based on the discussions, the revision from Qualcomm seems to be agreeable by most companies. Regarding positioning SRS, since so far we have only agree to support SRS with antennaSwitching, I think we do not have to discuss positioning SRS here; I added an FFS in Sec. 2.1.6. Proposal 2.1.1A: For a SRS resource configured with comb offset hopping and/or cyclic shift hopping, - If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l'. - If the repetition factor R > 1, at least support Option 2: Within a slot, hopping based on only the symbol index l'. - In addition, for comb offset hopping, gNB can configure Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as l'/R symbol index l' of the first symbol across the R repetitions. - FFS: Rel-18 comb offset hopping does not apply to positioning SRS. @Qualcomm: Thanks for the comments and suggestions for making the proposal clearer. @ZTE: You summarized very well. I think Option 1 and Option 2 were flipped in your summary if I am not mistaken. @Intel: For multiplexing issue, Sec. 2.1.5 discusses a viable solution. Hope that can address your concern. |
| LGE | We prefer Option 2. Option 2 has more flexibility for multi-SRS scheduling compared to Option 1. |
| CATT | Ok with the revised proposal. |
| ZTE | @FL Yes, Option 1 and Option 2 were flipped. Thanks for the correction. |
| Ericsson | While our preference is Option 1, we can be fine with Option 2 to support intra-slot hopping. However, we don’t see the technical justification for supporting both options specifically for comb-offset hopping and prefer to down-select one scheme. Therefore, we propose to remove the “In addition” sub-bullet (or make it FFS). |
| FL2 | @Ericsson: Thank you for being flexible. Regarding the sub-bullet, we can check other companies’ views. Now the proposal is discussed in the email reflector for potential email endorsement. |
| Xiaomi | We think comb offset hopping should be based on the symbol index l' of the first symbol across the R repetitions. The reason is that repetition gain which is obtained through increasing receiver power at gNB side by SRS repetition can be remained even though comb offset hopping. |

### NW-configured ID(s)

We have the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

The following summarizes the NW-configured ID(s):

- Option 1: Reuse the SRS sequence identity nIDSRS.

  - Supporting: CMCC, Ericsson, Google, Intel, InterDigital, Lenovo (prefer), MediaTek, NEC, New H3C, OPPO, Qualcomm, Sharp, vivo, ZTE.

  - 10 14 proponents.

- Option 2: Introduce new ID(s).

  - Supporting: Apple, CATT, ETRI, Futurewei, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Spreadtrum, Xiaomi.

  - 9 proponents.

Either option in the above agreement has its merit, and companies have suggested different ways to utilize these two options for different scenarios. The numbers of proponents are roughly equal. Therefore, I suggest the following initial proposal which can incorporate the benefits of both options and leave the specific decision to implementation, i.e., the gNB can configure new ID(s) but if no ID is explicitly configured, the legacy SRS sequence identity is assumed as the default.

Proposal 2.1.2: For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID is down selected from the following options:

Option 1: cinit=nIDSRS.

Option 2: New ID(s)

  - Option 1-1: cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

  - Option 1-2:

    - cinit=nIDCS, where nIDCS is a new ID for cyclic shift hopping.

    - cinit=nIDCO, where nIDCO is a new ID for comb offset hopping.

    - [cinit=nIDCSCO, where nIDCSCO is a new ID for the combined cyclic shift and comb offset hopping, if supported.]

  - Default ID: If no ID is explicitly configured in RRC configuration, the SRS sequence identity nIDSRS is used.

  - FFS: the value range, e.g., 0~1023, 0~65535.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support option 1, which is a simple and clean solution |
| QC | - Support Option 1. We do not see the need for Option 2 as discussed in our Tdoc. |
| InterDigital | Support Option1 for less spec impact. |
| OPPO | Support Option 1. |
| NEC | Support Option 1. |
| Lenovo | Support the proposal. And we prefer Option 1 since the legacy SRS sequence ID can meet the requirement of UE specific initialization. |
| ZTE | - Support option 1. It was pointed out that, if more than one hopping algorithms are enabled simultaneously, each of them should use an individual ID/pseudo-random sequence to ensure their hopping patterns are independent from each other. However, it is not necessary, because multiple independent hopping patterns can be derived from a common pseudo-random sequence by using different segments of the sequence and different pattern generation formulas. Besides, option 1 is beneficial for designing the pattern of combined CS + comb offset hopping and combined CS + group/sequence hopping. |
| CMCC | - Option 1 is enough. |
| Huawei, HiSilicon | - Support Option1. |
| CATT | - Support option 2 to simplify the configuration of the initialization ID. |
| Nokia/NSB | - Support option 2. |
| Intel | - Support Option 1. |
| vivo | - Support Option 1. |
| Xiaomi | - Support the proposal and we prefer to Option2 for the flexibility of configuration. |
| MediaTek | - Support Option 1. |
| Sharp | - We slightly prefer Option 1 because Option 1 is simple, and Option 2 has no large benefit. |
| DOCOMO | - While we understand option 1 is majority, we are not sure if it is really practical. So we still prefer option 2. - Although the legacy SRS ID can be UE-specific, we think one basic purpose of SRS ID is to mitigate inter-cell interference. To achieve this, we believe different SRS ID should be used in different cells (at least in a single time occasion). Otherwise, we need to consider quite complicate policy for the usage of SRS ID together with SRS scheduling/configuration across many cells (which is very difficult, given it needs micro-optimization per cell), or we need to accept such occasional SRS inter-cell interference (which is not possible). Therefore, we would think the number of SRS IDs available per cell in the deployment is quite limited (at most 1024/3, in our view). - We’d like to note that the above should be considered even when sequence group is configured, as using the same ID and same time-domain occasion, the result of group hopping could be the same. Given sequence hopping is just a binary solution, we believe it should still be very careful on how to assign SRS ID. - Now, if option 1 is taken based on the majority, to mitigate (intra-cell) inter-TRP SRS interference, SRS ID should be per-UE. Meanwhile, the above consideration on inter-cell should be kept, since we believe introduction of CJT doesn’t mean inter-cell interference is a “trivial” issue. Plus, if adjacent cells deploy CJT using TDD with SRS based DL CSI acquisition, the issue seems worse. We’d also like to note that one SRS ID per cell to mitigate inter-cell interference is still a good implementation which makes operation quite easier. In this sense, option 1 may be something very restrictive for the operation to consider CJT introduction. - Option 1 camp’s feedback is highly appreciated. |
| New H3C | - Support Option1. |
| Apple | - We prefer option 2. - With option 1, within a cell, if different UEs are configured with different SRS sequence ID and multiplexed orthogonally in legacy system. How can option 1 be used to ensure that those UEs are still orthogonal when they start to hop? |
| FL | - I updated the supporting company list based on the discussions. Please continue to discuss, especially regarding NTT DOCOMO’s comment from an operator’s perspective and Apple’s question. |
| QC | - We are open to consider Option 2 if there are issues with Option 1, but we would like to first understand the issue(s) better. Hence, we have some questions: - @Docomo: Thank you for the analysis. In your explanations above, are you assuming that CJT is within a cell (multiple TRPs within a “cell” transmit jointly to the UE)? Or are you assuming that CJT is across cells (multiple TRPs corresponding to different “cells” transmit jointly to the UE)? Or both? - Also, “mitigate (intra-cell) inter-TRP SRS interference” is not very clear to us. In our understanding, the main use case of this interference randomization is for the case that SRS of two UEs are not orthogonal, which means that they belong to different (but neighboring) CJT clusters. If they are orthogonal, they should remain orthogonal irrespective of hopping. If they are not orthogonal (or if we rely in occasional orthogonalization as a result of hopping), these UEs should not be within a CJT cluster. Otherwise, the interference would be detrimental to CJT performance, and solely relying on randomization does not solve the issue. - @Apple: Thank you for the question. When you say “within a cell, if different UEs are configured with different SRS sequence ID and multiplexed orthogonally in legacy system”, do you mean UE1 is configured with nID,1SRS and UE2 is configured with nID,2SRS such that they have the same SRS sequence (nID,1SRS mod 30=nID,2SRS mod 30), and they are orthogonal in CS domain? Or do you mean they are orthogonal in comb offset domain? In either case, given these UEs are intra-cell, why they are not configured with the same nIDSRS? - Obviously, if the actual SRS sequence is different (nID,1SRS mod 30 ≠nID,2SRS mod 30), they are not orthogonal in the same REs. |
| NTT DOCOMO | - Reply to QC: - We thank QC for the question above. Indeed, it would be good to clarify the scenario in mind at first. - We assume CJT (irrespective of TDD or FDD) will basically be performed by multiple coherent TRPs within a cell. It means “CJT cluster” doesn’t comprise multiple TRPs across different cells. I think this scenario could be a simplest one. - Based on above let us depict what we are thinking for sTRP deployment and mTRP deployment: - Single TRP case: - In this case, we assume two approaches; - One is that a common SRS ID is configured in a cell (say cell#1 above), while other SRS IDs are used for other cells. - The other is that, on top of the first approach, UE#1 and UE#2 may be configured with different SRS IDs. - Both the two above achieve inter-cell interference mitigation in case when SRS resource collides between UEs in different cells. The difference is that intra-cell interference might be mitigated in the latter approach (though the usecase is a bit unclear in the sTRP case). The latter one needs sophisticated handling on SRS ID in operation to achieve the principle “different SRS ID in different cells”. - CJT (i.e., multi-coherent-TRP) case: - In case of CJT, the deployment will be like above in our view. - Here we understand UE#1 and UE#2 may transmit their SRS toward different target TRP, in which interference should be mitigated. That is why we will introduce comb offset hopping and CS hopping for CJT case in our understanding, and this is what we described as “(intra-cell) inter-TRP SRS interference” in the previous comment. We apologize for our unclear wording. - One may argue that such an inter-TRP interference within a cell should be avoided by NW implementation; we would say we tend to agree with such a logic. But if that is the case, then we must say we do not quite understand what scenario we will require to turn on comb offset hopping or CS hopping. (note that here may be what I misunderstand something compared with other companies☹ – please let us know if the scenario intended is different). So now, we assume the mitigation of inter-TRP interference within a cell is aimed by Rel-18 comb offset/CS hopping schemes. - Another point is that even in case above, we still need to consider inter-cell interference. It is assumed to be achieved by reusing SRS ID, as described above. Based on the analysis above, we feel the available SRS IDs in a cell might be quite limited, which may or may not be sufficient for CJT usage. Because in our understanding, SRS in TDD CJT case may need to be hopped based on different patterns if the SRS is toward different TRPs (even within a cell). Plus, neighbor cells may also deploy CJT. Option 1 means SRS ID will also be used in such neighbor cells to initialize hopping pattern. - Also, QC kindly remind us of another good point – # of CJT clusters may be one or more than one, even in a cell. We fully agree with QC comment that “In our understanding, the main use case of this interference randomization is for the case that SRS of two UEs are not orthogonal, which means that they belong to different (but neighboring) CJT clusters”. If multiple CJT clusters can be considered, then SRS between different CJT clusters may need to be hopped in different pattern, which also require different initialization value. In this case, the required # of initialization parameters would increase. Then the (remaining) capacity of SRS ID in legacy might be more questionable. - Final note is that we fully agree with the concept of “initialize hopping based on UE-dedicated RRC parameter” itself. Our question is that when considering the actual purpose of SRS comb offset/CS hopping for CJT in Rel-18, whether SRS ID can achieve what RAN1 wanted to achieve actually. If it does not achieve the purpose, then we’d like to suggest decoupling the initializing ID from legacy parameter which is option 2. |
| Nokia/NSB2 | - Following is our understanding about SRS ID. - - Rel-15 SRS ID (10bits): common ID for a cell. To mitigate inter-cell SRS interference, group hopping is supported, because ZC sequence has not even cross-correlation across the sequences. With Rel-18 schemes, this hopping should be supported for reducing inter-cell interference. (legacy function) - - Rel-16 SRS ID (16 bits): common ID for a TRP. (Rel-15 cell-ID 10bits, PDSCH sequence ID 16bits for multiple TRPs (not necessary for NC-JT). - -Rel-18 hopping ID: to compensate inter-SRS interference in a CJT groups. Full UE specific ID is required. - So, we don’t think the existing ID can support the randomization. |
| QC | - @Docomo: Thank you very much for the clarifications and explanations. In the scenario above, and as you also mentioned, we think inter-TRP interference within a cell/cluster should be avoided by orthogonalization (and not by randomization). In our mind, the primary use case for turning on CS / comb offset hopping is for randomization across different cells / CJT clusters. This is similar to legacy sequence / group hopping, but Rel-18 will allow for interference randomization in other domains as well. - We are also open to hear more views regarding the scenario that Docomo mentioned above. |
| FL2 | - Thank you for the detailed discussions. I think in general, interference randomization is done across different coordination areas, whereas within one coordination area, usually the SRS ports are orthogonal via TDM/FDM/CDM if there are sufficient orthogonal resources. - There may also be another case: some SRS ports are configured to be orthogonal but due to long delay spread or long delay, they still cause interference between the SRS ports. One example is shown below, in which SRS1 causes interference to SRS2 due to the long delay spread: - The persistent interference on SRS2 remains when comb offset hopping is configured for these SRSs and Option 1 is used. This is because these multiplexed SRSs have the same SRS sequence identity and hence the same hopping pattern: - But with Option 2, SRS1 and SRS3 can be configured with one initialization ID and hence one hopping pattern, and SRS2 and SRS4 can be configured with a different initialization ID and hence a different hopping pattern, leading to randomized interference onto SRS2 and SRS4. - Therefore, I think there may be some advantages of adopting Option 2, and Option 2 can always fall back to Option 1 if needed. Please consider this example, and we can further study this. |
| ZTE | - We appreciate the good discussion among DCM, QC, and FL. We hold the same view with QC and FL, that inter-TRP interference within a cell can be avoided by orthogonalization and inter-cell interference can be handled by legacy and new hopping algorithms. And we do not see the necessity of introducing new ID(s) based on DCM’s description. - Thanks for FL for introducing another considerable case. If my understanding is correct, this is an intra-cell case, because all SRS ports are configured with a same SRS sequence ID and SRS sequence, right? However, as mentioned before, the purpose of introducing new hopping algorithms is to mitigate inter-cell interference. |
| NEC | - Thanks for the good discussion. - Regarding option 2, we think if a new ID is introduced, the orthogonality between UEs should be further discussed, in current framework, SRS for different UEs can be orthogonal based on different comb offset values or based on different cyclic shift values with a same root sequence, where same root sequence is generated based on the nIDSRS, if newly introduced pattern of comb offset/CS hopping is based on a new ID, it will cause different hopping patterns for orthogonal UEs with same root sequence (configured with different CS values or different comb offset values), which may lead to a same CS value or a same comb offset value for the orthogonal UEs in some occasion. To solve this, a same new ID should be configured for the orthogonal UEs, while if going with this way, there is no difference to reusing nIDSRS. - In general, we are open if orthogonality can be guaranteed/further discussed if going with option 2, otherwise we still prefer option 1. |
| FL3 | - @ZTE: Actually the WID is about “manage inter-TRP cross-SRS interference”, and the long delay / long delay spread scenarios can be common to M-TRP CJT deployment, as many companies described. - @NEC: I think the above example I showed allows different new IDs to be configured to the orthogonally multiplexed SRSs, and they can allow decoupling from group/sequence hopping, providing more flexibility to network configuration. |

### Hopping pattern reinitialization

We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

Regarding the FFS points colored in red, the general positions are:

Support at least reinitialization at the beginning of each radio frame

  - Supporting: CATT, Futurewei, Huawei, HiSilicon, Lenovo, LG, NEC, New H3C, Samsung, Sharp, vivo

  - 8 10 proponents

Support reinitialization periodicity of N radio frames or reinitialization based on system frame number

  - Supporting: CATT, Futurewei, Huawei, HiSilicon, NEC, New H3C, Qualcomm, Spreadtrum, vivo (for N radio frames)

  - Against: Lenovo, CMCC, Intel, LG, MediaTek, Nokia, Nokia Shanghai Bell, Samsung, Sharp, vivo (for based on SFN), Xiaomi, ZTE

  - 6 9 proponents and 2 11 opponents

The benefits of introducing dependency on SFN include that, the hopping patterns can be different (i.e., further randomized) across different radio frames. Otherwise, the hopping pattern repeats for each system frame of 10 ms, which may not have sufficient randomness for some use cases. In addition, gNB can also configure parameters related to the SFN reinitialization so that the reinitialization is performed for every radio frame, similar to legacy designs. Based on the views, we have the following proposal.

Proposal 2.1.2: For SRS comb offset hopping / cyclic shift hopping, at least support reinitialization at the beginning of each radio frame.

- Additionally, support reinitialization periodicity of N radio frames or reinitialization based on system frame number (SFN), and FFS details.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Do not support. We do not see any benefit. |
| QC | - We are ok with introducing this functionality, but then the same should be introduced for legacy hopping (sequence / group hopping). If there is a need to consider longer duration for reinitialization, the same need is applicable to sequence/group hopping. |
| InterDigital | Support FL’s proposal. |
| NEC | Support |
| Lenovo | We prefer to make reinitialization at the beginning of each radio frame since we do not see the strong motivation to make reinitialization periodicity as N radio frames. If majority companies support reinitialization periodicity as N radio frames, we can live up with it on account of progress. |
| Spreadtrum | Support. Larger reinitialization periodicity basically means more randomization and better performance. |
| ZTE | - Support reinitialization at the beginning of each radio frame. - NOT support reinitialization periodicity of N radio frames or reinitialization based on system frame number (SFN). - Apparently, reintialization is beneficial for interference randomization. However, reinitialization at the beginning of each radio frame is sufficient, and reinitialization periodicity of N radio frames or reinitialization based on system frame number (SFN) is unnecessary. |
| CMCC | - For legacy sequence/group hopping, reinitialization will be performed at the beginning of each radio frame, so it can be reuse for R18 SRS hopping. But for reinitialization with periodicity of N radio frames or reinitialization based on system frame number (SFN), we are not clear with the benefit. |
| Huawei, HiSilicon | - Considering the relatively large SRS periodicity supported by the current spec. compared to the radio frame length, forcing the re-initialization granularity to be as small as radio frame will severely restrict the randomization effect. Rather than this, larger re-initialization granularity should be considered. For instance, N can be fixed as 1024, which corresponds to the whole SFN range. |
| CATT | - Support the proposal. |
| Nokia/NSB | - Support reinitialization of start of each radio frame. Benefits of enabling reinitialization with periodicity of N radio frames remains unclear for us. |
| Intel | - We think the main bullet is sufficient. There is no need for the sub-bullet. |
| vivo | - For hopping per symbol, reinitialization at the beginning of each radio frame may be enough, when R is large. However, when R is small, the randomization gain would be limited. For hopping per R symbols or even per multiple slots, reinitialization at the beginning of each radio frame would directly degrade hopping gain, since hopping pattern is fixed in each frame, if the periodicity of SRS is larger than one frame. Therefore, reinitialization with periodicity of N radio frames should be supported for better hopping gain. - Besides, the meaning of “reinitialization based on system frame number (SFN)” is not clear for us. What does it mean? Does it mean hopping pattern would be different in each frame due to different SFN? If yes, we don’t support, since it would increase the UE complexity to generate infinite SRS hopping pattern with SFN increasing. |
| Xiaomi | - We prefer to reinitialization at the beginning of each radio frame. It is not clear how much gain can be obtained through reinitialization based on SFN. |
| MediaTek | - We support reinitialization at the beginning of each radio frame only. |
| Sharp | - We think it is enough to re-initiate at every radio frame. |
| New H3C | - OK |
| Samsung | - We can live with re-initialization at the beginning of each radio frame only. |
| Apple | - More discussion is needed. - Each radio frame is only 10ms. For example, if SRS periodicity is multiple of 10ms, randomization will not work, right? |
| FL | - Updated the support list above. - @Google: At least the main bullet is needed, otherwise the reinitialization behavior is undefined. Please check. - @Qualcomm: Good suggestion, and we can consider it after we make a decision on this proposal. - @Apple: If reinitialization is done for every radio frame, then there is no randomization from one radio frame to another. |
| LGE | - Support reinitialization at the beginning of each radio frame only. We think that the benefits of reinitialization based on system frame number are not clear. |
| CATT | - Reinitialization at every N radio frame (N >> 1) is good for interference randomization. As pointed out by some companies, when the periodicity of SRS is 10ms or even larger, the interference randomization effect is rather limited. |
| QC | - @FL: It may be better to allow for this flexibility for all SRS hopping schemes in the same proposal so that we do not have to separately discuss the same issue again. This is because the issue/motivation itself is not limited to CS / comb offset hopping. |
| FL2 | - @Qualcomm: We can give it a try. Proposal 2.1.2A: For SRS comb offset hopping / cyclic shift hopping, at least support reinitialization at the beginning of each radio frame. - Additionally, support reinitialization periodicity of N radio frames or reinitialization based on system frame number (SFN) for SRS comb offset hopping / cyclic shift hopping / sequence hopping / group hopping, and FFS details. |
| Xiaomi | - In current specification, for group/sequence hopping, the randomization is initialed at the beginning of each radio frame. We think it has worked well in practical commercial scenarios even for larger periodicity of SRS. So, it is not clear why supports reinitialization periodicity of N radio frames or reinitialization based on system frame number (SFN) for sequence hopping / group hopping. |
| Huawei, HiSilicon2 | - For companies don’t see the benefit, let’s assume a simple scenario, where the SRS resource contains 1 symbol and the SRS period is configured as 10ms, under which randomization scheme reinitializing at the beginning of each radio frame cannot achieve any randomization effect due to the repeated randomization pattern. Similarly, any SRS period larger than 10ms will suffer from the deficiency of randomization effect. Very limited randomization effect can be obtained only if the repetition factor R is larger than 1, which obviously violates the original intention of randomization scheme design. If the time-domain hopping behaviour of comb offset hopping finally depends on the OFDM symbol index l’ of the first symbol across the R repetitions, the situation for comb offset hopping will be even worse. - Since until now we haven’t observed any technical concern about the re-initialization periodicity of N radio frames, we suggest the following: - Proposal 2.1.2B: For SRS comb offset hopping / cyclic shift hopping, support reinitialization periodicity of N radio frames. - FFS: N is fixed or changeable |
| LGE | - Do not support sub bullet. The benefit of reinitialization based on N radio frames or based on system frame number is still not clear, as companies mentioned. We think that reinitialization based on the frame number is sufficient. So, we don’t need to discuss on the enhancement of legacy sequence/group hopping. |
| vivo | - We share the same view with xiaomi. The main bullet says “For SRS comb offset hopping / cyclic shift hopping”, the sequence hopping / group hopping part should be removed, since we have only agreed to introduce the enhancement of CS hopping and comb offset hopping in the previous meeting. - Besides, does the sub-bullet mean that only one of reinitialization periodicity of N radio frames and reinitialization based on system frame number (SFN) is supported? If yes, we think it’s better to make it clearer. Proposal 2.1.2A: For SRS comb offset hopping / cyclic shift hopping, support one of the following reinitialization schemes - Alt 1: Reinitialization periodicity of N radio frames - Alt 2: Reinitialization based on system frame number (SFN) - FFS details. - We prefer Alt 1. We understand both Alt 1 and Alt 2 can achieve the similar randomization gain. The key point is that c(i) would be calculated only once per N frame in Alt 1, while c(i) would be calculated per frame for Alt 2. Therefore, Alt 1 requires less UE complexity. - We also fine with Huawei’s revision. |
| FL3 | - @Huawei, HiSilicon, vivo: As you can see, still there are companies not supporting the new behavior. I think we should stick with the legacy behavior as the baseline in the main statement, and further discuss the new behavior. Proposal 2.1.2B: For SRS comb offset hopping / cyclic shift hopping, at least support reinitialization at the beginning of each radio frame. Additionally, support at least one of the following alternatives for SRS comb offset hopping / cyclic shift hopping / sequence hopping / group hopping: - Alt 1: reinitialization periodicity of N radio frames - FFS: N is fixed or configurable. - Alt 2: reinitialization based on system frame number (SFN). |

### Whether to allow combining cyclic shift hopping and comb offset hopping

We had the following agreement:

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

The general positions regarding the FFS point colored in red are:

Only one of cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time

  - Supporting: CMCC, ETRI, Futurewei, InterDigital, LG, MediaTek, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo

  - 11 12 proponents

One or both of cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time

  - Supporting:  Apple (UE optional feature), CATT, Ericsson, Google, Huawei, HiSilicon, Lenovo, NEC, New H3C, Nokia, Nokia Shanghai Bell, Sharp, Spreadtrum, xiaomi (depending on UE capability), ZTE

  - 12 15 proponents

  - In addition, one joint/combined cyclic shift + comb offset hopping may be supported.

    - Supporting: Ericsson, ZTE

Based on the views, we have the following initial, compromised proposal. Since we already agreed “At least the two features can be separately configured”, the proposal only needs to cover the case where both feature are configured. Also we already had agreement to FFS UE capability, so we can further discuss the associated UE capability design but we do not have to repeat it in this proposal.

Proposal 2.1.4: Both cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time, depending on UE capabilities.

FFS: one joint/combined cyclic shift + comb offset hopping

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support. Without any restriction, this is supported by default. |
| QC | - We do not see the need to configure both at the same time. Even in legacy, both sequence hopping and group hopping cannot be configured at the same time. For this combination, the need is even less given that these two hopping schemes are for different purposes. |
| InterDigital | Do not support. |
| OPPO | Not support. Both schemes are beneficial for interference randomization of inter-TRP SRS transmission. However, the benefits of the other scheme would be very small if one scheme has been supported since most interference can already be restricted by the scheme. Combined scheme would also introduce additional scheduling complexity at gNB to avoid potential collision. |
| NEC | Support the proposal. |
| Lenovo | Support the proposal. For 4Tx, it is needed since different Comb offset and CS may be used for different antenna ports. |
| Spreadtrum | Support |
| ZTE | - Support proposal 2.1.4. Further support combined CS + comb offset hopping. Jointly designing CS + comb offset hopping pattern can fully utilize the 2-D CS / comb offset resources and better avoid the collision among SRS ports due to the usage of a same CS and a same comb offset. |
| CMCC | - Not clear about the benefit of combined hopping over CS hopping or Comb offset hopping. |
| Huawei, HiSilicon | - Considering the limited degree of freedom for randomization caused by the existence of legacy UE, configuring available CS hopping range per comb should be enabled besides the simultaneous cyclic shift and comb offset hopping to facilitate better randomization effect. |
| CATT | - Support the proposal. |
| Nokia/NSB | - Support proposal. |
| Intel | - The FFS bullet is not clear and confusing. What does it mean? The main bullet already says both can be configured. |
| vivo | - Don’t support. - As the simulation results shown in our tdoc, the additional gain of the combination of CS hopping and comb offset hopping is limited. - Configuration of cyclic shift hopping or comb offset hopping separately is enough. |
| Xiaomi | - Support |
| MediaTek | - Do not support. We do not see the benefits of combined hopping vs cyclic shift offing or comb offset hoping individually. |
| Sharp | - Support |
| New H3C | - OK |
| Samsung | - Do not support, separate configuration for one scheme among two is enough. |
| Apple | - In general, we are fine as long as it is UE optional |
| FL | - Updated the support list. - @Intel: The FFS is about a single hopping (one pseudo random sequence, one initialization, etc.) on a 2D space formed by comb offset values and CS values. It is different from 2 separate hopping enabled for the same SRS resource. |
| LGE | - Do not support. Benefit of combined scheme is not clear. |
| CATT | - Suggest to remove the FFS part. The benefit of joint configuration of the hopping is not clear. |
| ZTE | - @Intel @FL: To our understanding, the FFS means the hopping patterns of CS hopping and comb offset hopping are jointly designed. If so, we prefer to clarify this point in the FFS. And the joint design is beneficial for fully utilizing the CS and comb offset resources and increasing the degree of hopping freedom if configuring a subset for hopping is supported. |
| Ericsson | - Support the FL proposal. Hopping over both CSs and comb offsets can randomize both intra-TRP and inter-TRP interference. E.g., two SRS resources configured with same SRS sequence may use different CS hopping pattern (to randomize intra-TRP interference) and same comb-offset hopping pattern (to randomize inter-TRP interference). |
| FL2 | - As ZTE requested, we can further clarify the FFS. Further suggestions are welcome to make it clearer. Proposal 2.1.4A: Both cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time, depending on UE capabilities. FFS: one jointly designed hopping in joint/combined cyclic shift + comb offset domains hopping |
| Samsung | - We still don’t see the benefit of combined hopping with or without UE optional feature. Separate configuration either Comb-offset or CS hopping is enough. |
| Xiaomi | - We are fine with the updated proposal. |
| ETRI | - Do not support. We also don’t see the benefit and necessity of combining cyclic shift hopping and comb offset hopping. |
| Huawei, HiSilicon2 | - Support. - As discussed before, considering the limited degree of freedom (DoF) for randomization caused by the comb offset(s) and cyclic shift(s) occupied by legacy UE(s), simultaneous enabling CS hopping and comb offset hopping (with the assistance of subsets restriction) can achieve higher randomization DoF and harvest performance benefit. - Furthermore, regarding the FFS bullet, given that the comb offset(s) and cyclic shift(s) occupied by legacy UE(s) irregularly distributes among the 2D resource grid formed by all the comb offsets and cyclic shifts, joint subsets restriction can take maximum advantage of the fragmented resource. |
| LGE | - We still don't think the gain obtained by combining the two hopping methods is significant. It seems that there is no clear benefit compared to complexity of combining the two hopping schemes. |
| vivo | - Not support the updated proposal. - Firstly, we have given the simulation result showing that there is almost no additional gain of the combination, since separate hopping (CS hopping/comb offset hopping) can provide great randomization gain. - Secondly, we still don’t get the point what is the difference between “the combination of CS hopping and comb offset hopping” and “one jointly designed hopping in joint/combined cyclic shift + comb offset domains”. In our understanding, assume CS hopping and comb offset hopping can be configured together, then gNB can configure optimal CS/frequency/time resources for SRS by RRC to achieve a joint optimization. |
| FL3 | - As ZTE requested in Sec. 2.1.8, I added another FFS. Proposal 2.1.4A: Both cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time, depending on UE capabilities. FFS: one jointly designed hopping in cyclic shift + comb offset domains FFS: one jointly designed hopping pattern of cyclic shift hopping + one of sequence / group hopping |

### Subset(s) for cyclic shift hopping and comb offset hopping

We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Regarding the FFS point colored in red, the general positions are:

Support configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

  - Supporting: CATT, Ericsson, Futurewei, Google, Huawei, HiSilicon, Lenovo, New H3C, Samsung, ZTE

  - Low priority: Apple, MediaTek, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Spreadtrum, vivo, Xiaomi

  - 6 10 proponents

The benefits of configuring a subset of resources for hopping include potential collision avoidance between different SRS resources when hopping is enabled. For example, if a Rel-18 SRS resource configured with cyclic shifts hopping is CDMed with a legacy SRS resource, the Rel-18 SRS resource can be configured with a subset of cyclic shifts excluding the legacy SRS resource’s cyclic shifts, so that these two resources would never collide. Based on the views, we have the following proposal.

Proposal 2.1.5: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support. This is beneficial with regard to legacy UEs. |
| QC | - This is not high priority in our view. We do not need to make things unnecessarily complicated. For multiplexing legacy UEs, different cyclic shifts (when comb offset hopping is used) or different comb offsets (when cyclic shift hopping is used) can be utilized, or even different OFDM symbols can be utilized (when both are used). Also, it is not clear if the subset applies to all ports or whether it is only for the first port. |
| OPPO | Low priority. We think the collision can be avoided via gNB implementation. |
| Lenovo | Support. It is beneficial for interference coordination and multiplexing SRS between legacy UEs and CJT UE. |
| Spreadtrum | This issue is an optimization of comb offset/cyclic shift hopping, and better to be discussed after all is done. |
| ZTE | - Support proposal 2.1.5. Furthermore, we want to point out that, if CS / comb offset hopping within a subset of CSs / comb offsets is supported, combined CS + comb offset hopping is superior to separate CS / comb offset hopping. - For separate CS hopping/ comb offset hopping, the number of resources in the subset of CSs / comb offsets could be very limited, especially for KTC=2,4 and nSRSCS,max=6. - For combined CS + comb offset hopping, there could be far more available resources in a 2-D resource subset, wherein each resource is corresponding to a combination of a CS and a comb offset. From this perspective, combined CS + comb offset hopping can better enhance interference randomization. |
| Huawei, HiSilicon | Support. The benefit of subset(s) restriction for CS hopping and comb offset hopping has been discussed in our contribution: - Keep compatibility with legacy UEs Subset(s) restriction for CS/Comb offset hopping is an effective way to keep the compatibility between Rel.18 UEs and legacy UEs. Take CS hopping as an example, although the compatibility issue can be solved by mapping SRS of Rel.18 UEs and legacy UEs on different comb offsets, it may lead to the decrease of resource utilization efficiency. - Avoid interference incurred by delay difference for CS hopping For some scenario (joint orthogonal SRS resource allocation by coordinated TRPs, SRS interference comes from propagation delay difference), obvious performance loss lead by potential severe collision is observed in our simulations [4] without subset(s) restriction. |
| CATT | Support the proposal. |
| Nokia/NSB | Deprioritize this topic. It would be good to clarify further what are the specification impacts for enabling support for sub-set comb offset or whether this can be done without any specification impacts? |
| Intel | Fine to discuss. It’s also fine to de-prioritize. |
| vivo | Low priority. It can be handled by gNB implementation. |
| Xiaomi | We have similar view with OPPO. The collision can be avoided by gNB implementation. |
| MediaTek | We believe this issue should be prioritize as it can be handled by gNB without any specification impact. |
| Sharp | I n our view, the legacy SRS resource’s cyclic shift cannot be recognized due to other TRP’s SRS resources. Furthermore, for interference randomization, SRS resources are in shortage. For these reason, we think configuration of the subset is not needed. |
| DOCOMO | Should be low priority. Our understanding is that MTK also intends to say deprioritize. |
| New H3C | OK |
| Samsung | Support the proposal. if we don’t have such subset, at least for Comb-offset hopping, the only way to fully separate between R18 UE supporting hopping scheme and legacy UE is TDM (allocating in different symbols), which is significantly inefficient. |
| Apple | Low priority. |
| FL | Updated the support list. @QC, Sharp: My understanding is as follows. Using CS hopping as an example, if SRS1 and SRS2 are CDMed, they have to occupy different cyclic shifts to avoid interference. But if SRS1 is a legacy SRS and SRS2 is configured with CS hopping, SRS2 may hop onto the same CS(s) as SRS1, causing interference. Then the subset of CSs can be configured for SRS2’s CS hopping range, i.e., SRS2 can only hop on certain CSs, not all possible CSs, thus avoiding interference. In my opinion, the subset may be applied to all ports of a SRS resource or only some ports (e.g., for ports on one comb offset but not the others). |
| QC | If indeed the intention is to define this subset of CS / comb offset for randomization for a subset of ports, then it becomes even more complicated. Relying on gNB implementation to address the issue should be enough. In the example FL mentioned above, the SRS1 and SRS2 can be multiplexed on different comb offsets. |
| LGE | Low priority. Similar view as OPPO. |
| ZTE | @Sharp: The intention of configuring a subset is to avoid collision between legacy UEs and new UEs within one cell. Interference among SRSs transmitted from UEs in different cells is handled by the newly introduced hopping schemes. We agree with Samsung that, if the compatibility of legacy UEs is achieved by gNB implementation, the best gNB can do is to allocate legacy SRS ports and new SRS ports over different OFDM symbols, such that the flexibility is insufficient. |
| Ericsson | Support. In our view, this is essential for co-scheduling UEs with different hopping schemes and legacy UEs. |
| FL2 | @Qualcomm: Configuring the subset for all ports of a SRS resource is simpler and can be a starting point for further study. I agree in the above example, SRS1 and SRS2 can be FDMed, but with this proposal, they can be FDMed or CDMed, providing more flexibility, especially there can be cases where the system runs out of resources for FDM and has to rely on CDM. Therefore, I think there can be some advantages of supporting this, and we can further study this. |
| Xiaomi | In our view, the subset of comb offset or cyclic shift should be preconfigured. If the size of the configured subset is too larger, the available resources for legacy UEs will be less. Otherwise, the issue is still exist for Rel-18 UEs. So, the size of subset needs to FFS. |
| Huawei, HiSilicon2 | Support. Totally agree with FL’s assessment. Take a glance at the supporting situation you may find that most of the gNB vendors, who actually facing the SRS resource allocation problem, support this proposal. Compared with expecting gNB can always handle this problem by implementation, which strongly relies on the allocation algorithm and resource occupancy situation, why not introduce a simple mechanism to give gNBs more chance to allocate SRS resource to UEs? |
| vivo | We can understand the motivation for introducing the subset. However, introducing subset would degrade the gain of comb offset hopping and CS hopping, due to less randomization room. Besides, when there are enough SRS resources configured, configuring the subset can’t help anything. For example, assume comb 2, one R18 SRS occupies comb 0 and one R15 SRS occupies comb 1. Then to avoid the collision with R15 SRS based on the comb subset, there is no room for comb offset hopping. Therefore, it’s better to leave it to gNB implementation to make an optimal SRS assignment. |

### Applicability to SRS other than P/SP SRS for antennaSwitching

We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Regarding the FFS point colored in red, the general positions are:

Applicable to aperiodic SRS

  - Supporting: Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, Lenovo, MediaTek, New H3C, Nokia, Nokia Shanghai Bell, Qualcomm, Sharp, Spreadtrum, vivo, ZTE

  - Against: CMCC, Google, Intel

  - 4 15 proponents and 1 3 opponent

Applicable to SRS with usage codebook, nonCodebook, or beamManagement

  - Supporting: CMCC, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, MediaTek, New H3C, Nokia, Nokia Shanghai Bell, Qualcomm, Sharp, Spreadtrum, ZTE

  - Against: Google, Intel, vivo, NTT DOCOMO

  - 3 14 proponents and 4 opponents

Based on the views, we have the following proposal.

Proposal 2.1.6: SRS comb offset hopping / cyclic shift hopping can be configured for:

Aperiodic SRS

SRS with usage codebook, nonCodebook, or beamManagement

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Do not support. |
| QC | - Support. |
| InterDigital | Support FL’s proposal. |
| Lenovo | Support |
| Spreadtrum | Support |
| ZTE | - Support the proposal. - If CS hopping/ comb offset hopping cannot be configured for AP SRS, AP SRS may collide with P SRS due to the usage of a same CS and a same comb offset over a common OFDM symbol. - Besides, configuring CS hopping/ comb offset hopping for SRS with usage codebook, nonCodebook, or beamManangement is beneficial for interference randomization. |
| CMCC | - SRS interference randomization is used for avoiding continues SRS collision, while AP-SRS is not the case. But it seems CS/Comb offset hooping for AP-SRS do no harm for their performance, as long as UE capability support, then we are OK with this proposal. |
| Huawei, HiSilicon | - Support. |
| Nokia/NSB | - Support. |
| Intel | - We don’t see the strong need. |
| vivo | - Ok with aperiodic SRS. - However, for the second bullet, we don’t see the necessity and it seems out of scope. Since we are discussing TDD CJT scenario, the enhancement should focus on SRS for antenna switching. |
| MediaTek | - Support |
| Sharp | - Support |
| DOCOMO | - Open, while the same understanding as vivo. |
| New H3C | - OK |
| Samsung | - We don’t see any necessity. It could be deprioritized. |
| Apple | - Low priority for us |
| FL | - Updated the support list. - @vivo, NTT DOCOMO: My understanding is that SRS interference randomization to improve TDD CJT SRS could include other SRS usages, as those SRSs can also interference with TDD CJT SRS with antennaSwitching. Therefore, improving the randomization for codebook, nonCodebook, or beamManagement may also improve TDD CJT SRS performance. - As positioning SRS was also mentioned, I added it as an FFS here. Proposal 2.1.6A: SRS comb offset hopping / cyclic shift hopping can be configured for: Aperiodic SRS SRS with usage codebook, nonCodebook, or beamManagement FFS: Positioning SRS |
| LGE | - Same view as VIVO. |
| CATT | - We are ok with aperiodic SRS, but other usages are out the scope of Rel-18 MIMO. |
| Ericsson | - Support the first bullet. While we tend to agree that the second bullet is out of scope (not targeting CSI acquisition for TDD CJT), we don’t see an issue with supporting such hopping schemes for SRS resources in SRS resource sets with other usages (SRS resource configuration does not depend on which SRS resource set said SRS resource belongs to). Hence, support also the second bullet. Don’t support the third bullet. SRS for positioning is configured differently from SRS and does not support the hopping schemes discussed here. |
| vivo | - We understand that SRS with other usages would also cause cross-SRS interference on SRS for antenna switching. However, as long as SRS for antenna switching is configured with CS/comb offset hopping, then the interference would be temporary. Therefore, no need to enhance SRS with usage codebook, nonCodebook, or beamManagement. Regarding positioning SRS, it is out of scope. Even whether the randomization is useful for it to keep the positioning accuracy is not clear. |
| FL3 | - Based on the comments, I think we can remove positioning SRS. For the other usages, since there are at least 14 proponents, we can still keep it. Proposal 2.1.6B: SRS comb offset hopping / cyclic shift hopping can be configured for: Aperiodic SRS SRS with usage codebook, nonCodebook, or beamManagement FFS: Positioning SRS |

### Cyclic shift hopping with finer cyclic shift granularity

Regarding cyclic shift hopping with finer cyclic shift granularity, the general positions are:

Support cyclic shift hopping with finer cyclic shift granularity

  - Supporting: Futurewei, Huawei, HiSilicon, vivo

  - Against: Intel, MediaTek, NEC, Nokia, Nokia Shanghai Bell, OPPO, Qualcomm

  - 4 proponents

The benefits of introducing finer cyclic shift granularity include further randomization for cyclic shift hopping, which is also demonstrated using simulations. Based on the views, we have the following proposal.

Proposal 2.1.7: For SRS cyclic shift hopping, support k (k=1,2,…,K) times finer time-domain granularity for a cyclic shift in cyclic shift hopping.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | The proposal is somehow unclear to us. Any example to further clarify it? |
| QC | - Wouldn’t this make multiplexing legacy UEs even harder even if we allow a subset of cyclic shifts to be configured for hopping? For example, we have 12 cyclic shifts for comb 4. If K=4, this means we have 48 cyclic shifts. Then, it is not clear what would be the meaning of subset in proposal 2.1.5 for the purpose of multiplexing with legacy UEs. - Also, this goes against the following conclusion: Conclusion - No further discussion of increasing the maximum number of cyclic shifts for CJT SRS. - ... |
| OPPO | We don’t think it is needed. |
| NEC | We don’t think it’s needed either. |
| Lenovo | We are fine to make further study and discussion. If the benefit is clear and the impact on legacy UE is not large, we are fine to specify them. |
| ZTE | We are open to discuss this issue. |
| CMCC | We are open to further study it. |
| Huawei, HiSilicon | Support. As shown in our contribution [4], finer granularity can bring obvious performance gain under different scenarios. @Google, for finer CS hopping granularity, the CS used per SRS port at each SRS transmission can be defined as: αi=2πnSRScs,inSRScs,max+2πnSRScs,randomK∙nSRScs,max, where K≥1 is the parameter indicating the hopping granularity. By the introduction of finer granularity, higher degree of freedom can be obtained, which will bring better interference randomization effect. @QC, we don’t observe any multiplexing issue if both the subset and finer granularity for CS hopping are enabled. The definition of subset can depend on the hopping granularity K. For example, if the CS hopping subset is {0, 1, 2, 3} when K=1 (nSRScs,max=8), the corresponding subset can be configured as {0, 0.5, 1, 1.5, 2, 2.5, 3} when K=2, which means {4, 5, 6, 7} can still be allocated to legacy UEs. In terms of the conclusion mentioned, increasing the maximum number of cyclic shifts is precluded because it shrinks the zero correlation zone (ZCZ) and harms the SRS channel estimation performance, while finer CS hopping granularity only applies to the hopping offset, which is shared by all “orthogonal” SRS ports without shrinking the ZCZ. The technical essence is totally different. |
| Nokia/NSB | Agree with QC that proposal is against previous conclusion. Not to support. |
| Intel | Do not support. The proposal is not clear. |
| vivo | We support CS hopping with K oversampling of cyclic shifts, since it can bring additional randomization gain. Regarding the conclusion mentioned above, we understand the initial motivation is to exclude the following schemes that directly increases the number of integer cyclic shifts. - F: Increasing the maximum number of cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus, Spreadtrum, Google, Xiaomi, Futurewei) 6 Pros: Capacity enhancement Cons: Applicable only for cases with short enough delay spreads F1: Multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus) |
| Xiaomi | We are open to further study whether more gain can be obtained. |
| MediaTek | Not support |
| Sharp | We are OK to study. |
| DOCOMO | Open |
| New H3C | OK to further disucss |
| Apple | Need further clarification |
| LGE | Agree with QC. It has already been concluded that we will not discuss it, in the last meeting. |
| Ericsson | Don’t support. We agree with QC that this proposal does not adhere to the previous conclusion. K times finer granularity is the same as K times more cyclic shifts, except it is only for the case when CS hopping is configured. |
| Huawei, HiSilicon2 | Support. Hope after the clarification above and offline the proposal is now clear. Again, the previous conclusion is precluded due to the reduced zero correlation zone (ZCZ) length and degraded channel estimation performance, both of which is NOT TRUE for this proposal. The maximum number of cyclic shifts remains the same and the finer granularity only applies to the cyclic shift offsets utilized by a group of orthogonal SRS ports. @Ericsson, thanks for raising a good point, we can add a note clarifying that the finer granularity only applies to the cyclic shift offsets as below: Proposal 2.1.7: For SRS cyclic shift hopping, support k (k=1,2,…,K) times finer time-domain granularity for a cyclic shift in cyclic shift hopping. Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. |
| FL3 | We can try Huawei/HiSilicon’s suggestion, and I think it is more clear to say “time-delay-domain” rather than “time-domain”. Proposal 2.1.7A: For SRS cyclic shift hopping, support k (k=1,2,…,K) times finer time-delay-domain granularity for a cyclic shift in cyclic shift hopping. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. |

### Cyclic shift hopping / comb offset hopping combined with group / sequence hopping

Regarding cyclic shift hopping / comb offset hopping combined with group / sequence hopping, the general positions are:

Support cyclic shift hopping combined with group / sequence hopping

  - Supporting: CATT, Futurewei, NEC, Qualcomm, Spreadtrum, xiaomi, ZTE

  - Against: ETRI, vivo

  - 7 proponents and 2 opponents

Support comb offset hopping combined with group / sequence hopping

  - Supporting: CATT, Futurewei, NEC, Spreadtrum, xiaomi

  - Against: ETRI, vivo

  - 5 proponents and 2 opponents

Before we proceed, based on some detailed views from the contributions, a few clarification questions to ask and to align companies’ understanding are:

Q1: If not explicitly specified in the standards, does it mean that cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping based on gNB configuration and UE capability?

  - Yes: CATT, CMCC, Google, Intel, InterDigital, Lenovo, MediaTek, NEC, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, Spreadtrum, vivo, ZTE

  - No: xiaomi

Q2: If not explicitly specified in the standards, does it mean that cyclic shift hopping / comb offset hopping cannot be combined with group hopping / sequence hopping?

  - Yes: InterDigital, Lenovo, NTT DOCOMO, OPPO, Qualcomm, xiaomi

  - No: CATT, CMCC, Google, Intel, MediaTek, NEC Nokia, Nokia Shanghai Bell, Samsung, Sharp, Spreadtrum, vivo, ZTE, New H3C,

In other words, what should the gNB/UE assume as the default behavior if the standards do not explicitly support (or exclude) the new hopping be combined with legacy hopping?

Please feel free to comment on this topic and provide your views on Q1/Q2.

| Company | View |
| --- | --- |
| Google | Q1: Yes, it does. Q2: No, it does not. |
| QC | Q1: If “UE capability” refers to indicating support of this combination, then yes. Q2: Yes. At least a UE capability for supporting combined hopping would be needed. |
| InterDigital | Q1: Yes contingent on UE capability. Q2: Correct, the default ehavior is to not be configurable unless the UE supports it. |
| OPPO | Q1: yes Q2: yes if UE capability supports that. |
| NEC | Q1: Yes. Q2: No. |
| Lenovo | Q1: yes Q2: yes. It needs UE capability for supporting it. |
| Spreadtrum | Q1: Yes Q2: No |
| ZTE | Q1: Yes, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. Q2: Yes, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. Combined CS + group/sequence hopping should be supported, because: - Both CS hopping and group/sequence hopping are code-domain hopping, combing them can further enhance interference randomization; - Joint design of the patterns of CS hopping and group/sequence hopping is beneficial for avoiding the collision of two SRS ports due to the usage of a same CS and a same group/sequence number. |
| CMCC | Q1: Yes, it does. Q2: No, it doesn’t. |
| CATT | Q1: Yes, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. Q2: Yes, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. |
| Nokia/NSB | Q1: Yes Q2: No, from our perspective single UE capability for comb offset and cyclic shift hopping should also cover combination with legacy operation. (they are different functionalities. inter-cell vs. inter-UE) |
| Intel | Q1: Yes Q2: No |
| vivo | Q1: Yes Q2: No. We don’t see additional gain of the combination of new hopping and legacy hopping. These four hopping schemes should be configured separately. Besides, we would like to mention that in the current spec, sequence hopping can’t be configured with group hopping together. |
| Xiaomi | Q1: No Q2: Yes Cyclic shift hopping and group/sequence hopping are code domain hopping. If cyclic shift is hopped based on symbols in a slot, they are no much difference. Hence, we prefer cyclic shift hopping can combine with group/sequence hopping when cyclic shift is hopped inter-slot. |
| MediaTek | Q1: Yes, it does. Q2: Yes, it does, based on UE capability. |
| Sharp | Q1: Yes Q2: No |
| DOCOMO | Q1: Yes Q2: Yes |
| New H3C | Q1: Yes Q2: No |
| Samsung | Q1: Yes, without any specification restriction, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. Q2: Yes, without any specification restriction, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. |
| Apple | It is better to make explicit agreement. Otherwise, by default, simultaneous configuration is not allowed. If there is no consensus to support simultaneous configuration, then there is no consensus. |
| FL | @CATT, ZTE, Samsung: Since the original Q2 is ‘cannot’, I think your answer to the original Q2 should be ‘No’. Please correct me if I am misunderstanding. Based on the discussions, I believe this is more related to UE capability / feature design, which we will be focused on later, and there seems to be no issue regarding the combined hopping behavior. Of course we can still continue the discussion here. As most companies replied ‘yes’ for Q1, we can try to move forward based on Q1. Proposal 2.1.8: Whether SRS comb offset hopping / cyclic shift hopping can be combined with group / sequence hopping on a SRS resource depends on UE feature/capability design. FFS: UE feature/capability design details. |
| QC | We are ok with the proposal above in principle. |
| CATT | Ok with the updated proposal. |
| ZTE | @FL: Regarding Q1 and Q2, we believe that, without any specification restriction, cyclic shift hopping / comb offset hopping can be combined with one of group hopping / sequence hopping. Besides, we propose to add an FFS: Joint design the hopping patterns of CS hopping and sequence / group hopping, because of them are code-domain hopping, and the joint design is beneficial for avoiding two SRS ports collide due to the usage of a same CS and a same sequence / group number. |
| FL2 | @ZTE: For the first comment, I think that is what we are trying to solve in this proposal and we will try to account for all companies’ inputs. For the second comment, since this proposal is more related to UE feature/capability discussion, maybe the FFS can be added to Proposal 2.1.4A or as a separate proposal? |
| ZTE | @FL: Thanks for considering our request. We prefer to add an FFS to proposal 2.1.4.A. But we are also fine with a separate proposal. |
| Xiaomi | For group hopping and sequence hopping, we think at most one is supported. Thus, comb offset hopping and cyclic shift hopping should combine one of the group/sequence hopping and suggest the proposal is reworded as Proposal 2.1.8: Whether SRS comb offset hopping / cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. FFS: UE feature/capability design details. |
| OPPO | We are fine with the updated proposal. |
| vivo | We don’t see the benefit of the combination of new hopping with sequence/group hopping. Anyway, it should be discussed after the determination of issue 2.1.4. |
| FL3 | We can try Xiaomi’s version. Proposal 2.1.8A: Whether SRS comb offset hopping / cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. FFS: UE feature/capability design details. |

### Other proposals comb offset hopping / cyclic shift hopping:

The following proposals are also for comb offset hopping / cyclic shift hopping:

- Enh. 1: Combined with enhanced CS configurations, e.g., per-port CS assignment, non-equidistant CS assignment

  - Supported by: Samsung, Futurewei

Views can be provided for the above enhancements, and other designs can also be suggested.

| Company | View |
| --- | --- |
| Google | Do not support. In our view, we do not need to discuss this based on the conclusion below. Conclusion No consensus to support the following for TDD CJT SRS enhancement in Rel-18: - Further enhancements to frequency hopping - Sequence hopping/randomization, per-hop sequence from a long SRS sequence - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - Precoded SRS for DL CSI acquisition - Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Configuration of v (sequence index within a group) per SRS resource - Multiplying mask sequence to the legacy SRS sequence |
| ZTE | NOT support. We share the same view with Google. |
| Huawei, HiSilicon | Open to discuss. |
| MediaTek | Not support. Agree with comment made by Google. |
| New H3C | Open |
| Samsung | In R1-2301879, FL kindly captured some proposals for this meeting and this was captured in proposal 4. We support to discuss this issue at least with Proposal 2.1.5 (subset configuration). |
| FL | My understanding is that this can be discussed at least with Proposal 2.1.5 (subset configuration), especially with cyclic shift subset configuration when CS hopping is enabled. |
| LGE | Not support. Agree with Google. |
| CATT | Not support. |
| Xiaomi | Not support |
| ETRI | Not support. |

## Per-TRP power control and/or power control of one SRS towards to multiple TRPs

We have achieved the following agreement:

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

The general positions based on the contributions and previous inputs are:

Option 0: For legacy TRP-specific SRS using multiple SRS resource sets; no enhancement in Rel-18

Pros:

No spec impact

Cons:

The number of SRS resource sets is generally at most 2 per current standards

Support by: Samsung, vivo

Acceptable by: Qualcomm, OPPO, Sharp

Option 1: For TRP-common SRS using one SRS resource set

Pros:

Low SRS overhead; low latency

One source shows performance benefit

Cons:

Receive powers at the CJT TRPs are less accurate

Support by: CMCC, ETRI, Futurewei, KDDI, Lenovo, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Spreadtrum, ZTE, Google, xiaomi

Acceptable by:

Option 2: For TRP-specific SRS using one SRS resource set

Pros:

Receive powers at the CJT TRPs are accurate

Cons:

Higher SRS overhead and/or higher latency compared to Option 1

Support by: Futurewei, Google, Intel, InterDigital, Sharp

Acceptable by:

Option 3: For TRP-common and/or TRP-specific SRS using one SRS resource set

Pros:

Flexible; combines both Option 1 and Option 2

Cons:

Higher complexity than Options 1 or 2

Support by: Futurewei, ZTE

Acceptable by:

Option 1 has the most proponents. If no compromise is made, then we will end up with Option 0, i.e., no SRS power control enhancements in Rel-18. As some companies are concerned with insufficient SRS resource sets to cover up to 4 CJT TRPs, maybe the alternative is to increase the number of SRS resource sets which can be a UE feature/capability discussion without core RAN1 spec impact.

Proposal 2.2: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, for an SRS resource set, support Option 1 (for TRP-common SRS using one SRS resource set):

One power control process for all SRS resources of an SRS resource set where the power control process is based on one P0 value and one closed loop state and jointly on M (M > 1)  DL pathloss RSs and/or M alphas.

Each transmission occasion of the SRS resource is towards multiple TRPs.

Please share your view. If you can also accept an option that is not your first preference, please also indicate below.

| Company | View |
| --- | --- |
| Google | Support |
| QC | We are ok with Option 1, but overall, it seems that power control enhancement is not critical for this item. |
| InterDigital | Support Option 2, otherwise for the sake of progress Option 0 is acceptable. |
| OPPO | We intend to support on enhancement in Rel-18. |
| NEC | Support |
| Lenovo | Support. We can target to agree one enhanced SRS power control scheme since it can be thought as one scheme for SRS capacity enhancement, which is included in the scope of WID. |
| Spreadtrum | Basically fine with FL proposal. One issue left is how to use M (M > 1) DL pathloss RSs and/or M alphas? Is it a completely implementation issue? |
| ZTE | Support proposal 2.2. Option 1 is better than Option 2 in the aspects of acquiring DL precoder, saving transmission power and reducing inter-cell interference. |
| CMCC | Support. |
| Huawei, HiSilicon | Fine. |
| CATT | Support the proposal. |
| Nokia/NSB | Support proposal with Option 1. |
| Intel | Don’t support. We support Option 2 and can accept Option 0. |
| vivo | Not support. |
| Xiaomi | Support |
| MediaTek | Fine |
| KDDI | Support, and we also support Option 2. |
| Sharp | We can go with the majority. |
| DOCOMO | Support with Option 1. |
| New H3C | OK |
| Samsung | Do not support, we support Option 0. Option 1 cannot compensate any pathloss values among distances between all TRPs and UE. Option 2 may have phase continuity problem and gNB cannot recognize distance difference between TRPs and UE. |
| LGE | Do not support. |
| ETRI | Support. |

## Others

- Any other views, issues, potential enhancements, and clarifications can be provided.

| Company | View |
| --- | --- |

# SRS enhancements targeting 8 Tx operation

It is well known that increasing UE Tx antenna ports can significantly improve various performance metrics for UL/DL transmissions. 8 Tx transmissions can be feasible for at least CPE/FWA/vehicle/industrial devices and hence can be beneficial.

Some remaining issues on the number of SRS resources for 8 Tx SRS and the number of SRS resource sets for 8 Tx SRS will be discussed in agenda item 9.1.4.2 covering “SRI/TPMI enhancement for enabling 8 TX UL transmission; To support up to 4 or more layers per UE in UL targeting CPE/FWA/vehicle/industrial devices”.

## Non-TDMed 8 Tx SRS

We will continue to discuss some detailed designs for non-TDM 8 Tx SRS, such as port mapping to comb offsets, cyclic shift allocation, etc., based on legacy non-TDMed port mapping, i.e., one symbol, repetition, FH, RPFS.

### Port mapping on multiple comb offsets

Several options for 8-port mapping on multiple comb offsets have been provided. Specifically, a number of companies proposed the following:

For kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset.

For kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} are mapped on the third comb offset, and {1003, 1007} on the fourth comb offset.

Supporting: Apple, CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Intel, Lenovo (for comb 2), LG, MediaTek, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Samsung, OPPO, Qualcomm, Sharp, vivo, ZTE,Xiaomi.

10 23 proponents.

Therefore, we have the following proposal.

Proposal 3.1.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset

If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support |
| OPPO | Fine with the proposal. |
| NEC | Fine |
| Lenovo | We are fine for the case when kTC=2 is configured. When kTC=4 is configured, we prefer the following assignment Ports {1000, 1001} are mapped on the first comb offset, {1002, 1003} on the second comb offset, {1004, 1005} on the third comb offset, and {1006, 1007} on the fourth comb offset |
| CMCC | Support |
| ZTE | Agree with the scheme for kTC=4. For kTC=2, our first preference is ports {1000, 1001, 1004, 1005} are mapped to the first comb offset, and {1002, 1003, 1006, 1007} to the second comb offset, but considering less effort of spec drafting, we can live with the proposed combination. |
| Huawei, HiSilicon | Support. |
| CATT | Support |
| Nokia/NSB | Support proposal. |
| QC | Support the proposal |
| Intel | Generally fine. |
| vivo | Support |
| Xiaomi | Support the proposal |
| MediaTek | Fine |
| Sharp | Support |
| DOCOMO | Fine |
| New H3C | OK |
| Apple | Okay for us |
| FL | Most companies support this proposal. @Lenovo: As all other companies support it, is it possible that the proposal is acceptable for comb 4? Of course if you have any technical justification / concern please feel free to elaborate. |
| Lenovo2 | Our intention is to assign the antenna ports in a same antenna group with the same comb offset. But for the sake of progress, we can accept this proposal. |
| LGE | Support the proposal. |
| Ericsson | Support the proposal. |
| FL2 | @Lenovo: Many thanks for being flexible. Now the proposal is discussed in the email reflector for potential email endorsement. |

### Comb 2: Cyclic shift locations and the number of comb offsets

For comb 2, a number of companies have the following view:

For comb 2, extend the legacy mechanism, i.e., there is only one configured cyclic shift location nSRScs, and based on its value compared with nSRScs,max/2, the number of comb offsets is determined.

Supporting: CATT, CMCC, Ericsson, Futurewei, NTT DOCOMO, Sharp, vivo, ZTE

8 proponents

We have the following proposal:

Proposal 3.1.2: For an 8-port SRS resource configured with comb KTC=2 and with maximum nSRScs,max=8 cyclic shifts per comb offset, the number of comb offset(s) and the cyclic shift locations are determined based on the one RRC configured cyclic shift location nSRScs as follows:

If nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used.

The 8 cyclic shift locations for the 8 ports are {nSRScs, (nSRScs+1) mod nSRScs,max, …,(nSRScs+7) mod nSRScs,max}.

Views can be provided for the above proposal.

| Company | View |
| --- | --- |
| Google | In our view, if we use 2 comb offsets, the cyclic shifts for both comb offsets should be the same. The second-bullet defines different cyclic shifts for different comb offsets. In that case, it is unnecessary to define the first sub-bullet. |
| OPPO | We support the proposal to reuse current mechanism. |
| NEC | Support |
| Lenovo | Fine with Proposal 3.1.2 |
| CMCC | Support. In Rel-15, 4-port SRS with comb=2, if nSRScs<nSRScs,max/2, 1 comb offset is used, otherwise 2 comb offsets are used, and 4 different cyclic shifts are used for the 4 ports. The same scheme can be reused for 8-port SRS. |
| ZTE | Generally support, this is aligned with legacy scheme. Regarding whether the CSs for different comb offsets should be same, we are open to discuss. |
| Huawei, HiSilicon | Although already agreed, we’d like to check companies’ view about whether 1 comb offset for KTC=2 is still needed, given that TDM is already supported. The reason why multiple mapping schemes are introduced for non-TDM case is to make sure the gNB can obtain different capability against delay spread, which is already enabled by the multiple TDM mapping patterns. Over-complicated and non-uniform mapping pattern design under different KTC should be avoided. |
| CATT | Support. The second bullet can ensure a unified cyclic shift determination mechanism for comb KTC=2 with 1 or 2 comb offsets. |
| QC | Partially support. We support the first bullet of the proposal. We don’t support the second bullet. The second bullet needs more discussion. If >1 comb offsets are used, why “+1” per SRS port? We suggest to write down the equation for the CS determination in FL’s proposal, which can clarify the potential confusion. |
| Intel | Generally fine with the proposal. |
| vivo | Support |
| MediaTek | Fine |
| Sharp | Support |
| DOCOMO | Qualcomm has a point. Second bullet may require further discussion. The other parts are fine for us. |
| New H3C | OK |
| Apple | In principle, we are okay. But the second bullet needs clarification. We just need to agree on the CS/comb offset mapping to different SRS port for both 1 and 2 comb offsets |
| FL | Most companies support the proposal. @Google: The proposed is a straightforward extension of the legacy design, by putting the CSs on different locations and using the configured CS value to decide whether one or two comb offsets are used. @Huawei, HiSilicon: Is the comment to suggest reverting a previous agreement on support 1 comb offset for comb 2? @Qualcomm, DOCOMO: The CS locations are across 2 comb offsets but are still numbered by the same indexes. On each comb offset, it is “+2”, but across the comb offsets, it is “+1”. This is exactly the same as 38.211 6.4.1.4.2, the second set of equations. |
| LGE | Support. Reuse the legacy scheme. |
| Ericsson | Support to extend legacy scheme. |
| FL2 | Now the proposal is discussed in the email reflector for potential email endorsement. |

### Comb 4 and comb 8: Cyclic shift locations across the comb offsets and indication of the number of comb offsets

For comb 4 and comb 8, there are a few options proposed for how to determine the cyclic shift locations for each comb offset and across the multiple comb offsets, which is also related to how the number of comb offsets is determined. For example, some companies discussed how the existing cyclic shift/comb offset equations can be generalized, and in some proposals, the cyclic shifts may be aligned across the comb offsets, but in some other proposals, the cyclic shifts are maximally separated/spread and unaligned if possible. Another option is to explicitly configure multiple cyclic shift locations for the multiple comb offsets. These details can be subject to further study.

The general positions are:

The cyclic shifts are completely aligned across the multiple comb offsets

  - Supporting: CATT, CMCC, Huawei, HiSilicon, NTT DOCOMO, Qualcomm, Sharp

  - 7 proponents

The cyclic shift positions have minimum alignment across the multiple comb offsets

  - Supporting: Ericsson, Futurewei, ZTE

  - 3 proponents

  - Ericsson and Futurewei also conducted PAPR simulations, showing that aligned cyclic shifts across the comb offsets lead to increased PAPR

Based on the views, the group can first study the following proposal:

Proposal 3.1.3: For an 8-port SRS resource configured with multiple comb offsets, decide, based on at least PAPR study, whether to allow the cyclic shift positions to be completely aligned across the comb offsets on the same OFDM symbol.

Views can be provided for the above proposal.

| Company | View |
| --- | --- |
| Google | OK to study |
| OPPO | Fine to study. And we think the same CSs are sufficient. |
| Lenovo | OK with Proposal 3.1.3 |
| CMCC | Support. For comb=4, nSRScs,max=12, if 8 different cyclic shifts are used for 8 ports, then the interval distance of different cyclic shifts are not aligned. For comb=8, nSRScs,max=6, there is not enough cyclic shifts for 8 ports. In Rel-17, for comb=8 for 4-port SRS, since the interval distance of different cyclic shifts are not aligned, then same cyclic shifts are used across the multiple comb offsets, the same scheme can be reused for 8-port SRS. So, aligned cyclic shifts can be used for the SRS ports on different comb offsets for comb=4 and comb=8. |
| ZTE | Support to study. In the case of KTC =4 and nSRSCS=12, to reduce the PAPR issue and fully use the CS resources, it is better to allocate different CSs to different comb offsets (i.e., allocate 8 different CSs to 8 respective ports) than to allocate 4 common CSs to 2 comb offsets. In legacy scheme for 4-port SRS as shown below, in the case of KTC =8, nSRSCS=12, same CSs are allocated to 2 pairs of ports over different comb offsets, but now we have more ports, PAPR issue becomes more serious. Therefore, we prefer to at least support allocating different CSs to different comb offsets. Legacy scheme for 4 ports in one symbol: - For comb 2, i.e., KTC=2, =8 - If , 2 comb offsets and 4 different CSs - Otherwise, 1 comb offset and 4 CS - For comb 4, i.e., KTC=4, =12 - If , 2 comb offsets and 4 different CSs - Otherwise, 1 comb offset and 4 CS - For comb 8, i.e., KTC=8, =6 - 2 comb offsets and 2 CS, 2 ports with different comb offsets share the same CS. |
| CATT | In Rel-15, for 4-port SRS resource configured with comb KTC=2, the cyclic shift positions are completely aligned across the two comb offsets on the same OFDM symbol. We prefer to extend the principle to 8-port SRS resource. We are open to further study. |
| Nokia/NSB | Support to study |
| QC | We don’t support the proposal. There is no PAPR issue to begin with. Potential PAPR issue is due to precoding across SRS ports. But there is no precoding across SRS ports allowed in spec. Given different SRS ports are transmitted via different physical antennas, we don’t see the need to study PAPR issue which does not exist. |
| Intel | Open to study. |
| vivo | Not need to study. We think the cyclic shifts completely aligned across the multiple comb offsets are acceptable for 8-port SRS with comb 4/8. When there are not enough cyclic shifts with the same distance between different cyclic shifts for 8-port SRS, aligning cyclic shifts across the multiple comb offsets is a better way, like 4-port SRS with comb=8. There would be a tradeoff between PAPR and channel estimation performance, irregular distance between different cyclic shifts would degrade the channel estimation performance as it was also raised in R17 discussion. |
| Xiaomi | open to study |
| MediaTek | Do not support the study. Agree with the comment made by QC. We don’t think PAPR issue exists. |
| Sharp | We are OK to study. |
| DOCOMO | Open to study |
| New H3C | OK to study |
| Apple | We are open to discuss but it is too early to make agreement |
| FL | As discussed in several contributions, whether there is a PAPR issue depends on the UE implementation. We can hear more inputs from UE vendors. |
| LGE | Do not support to study. |
| Ericsson | Support the proposal. We would like to clarify that, in contrast to what is claimed above by some companies, there is nothing in the specification preventing mapping two or more SRS port to the same physical antenna port for an SRS resource in an SRS resource set with usage ‘codebook’ (e.g., beamforming). Hence, there is potentially a PAPR issue (e.g., as we have shown in our tdoc). Now, the question is whether a UE would actually map its SRS ports to physical antenna ports in this way. From a NW perspective, if no UE/chipset vendor will map two or more SRS ports to a same antenna ports in the above way, we don’t see an issue with extending legacy CS mapping for transmission comb 4 and 8 to 8 SRS ports, but we think it is important that the problem is properly understood (especially with companies claiming there is no issue) before making agreements and, hence, we are fine to study. We agree with vivo that irregular distance between cyclic shifts (on the same comb offset) degrades the channel estimation performance. However, PAPR issue can be mitigated without resorting to such approaches. |
| FL2 | @vivo: Could you please clarify “irregular distance between different cyclic shifts”? I am not quite sure how this is related to the discussion here, as the two options provided by companies are all based on equidistant CSs on each comb offset. |
| QC | To Ericsson: if UE “mapping two or more SRS port to the same physical antenna port for an SRS resource in an SRS resource set with usage ‘codebook’”, the Tx waveform sending from that physical antenna is not even ZC low PAPR waveform. We will have a bigger problem even fallback using Rel-15 SRS. No one will implement in that way. |
| OPPO | We also think there would not be PAPR issue since different antenna ports are mapped to different physical antennae. |
| vivo | @FL, “irregular distance between different cyclic shifts” may be caused based on rounding up or down, since 6 integer cyclic shifts can’t be divided evenly for 8 ports. After we double check the CS assignment given by companies considering PAPR issue, we understand there is a way to keep the same distance between different cyclic shifts on each comb offset. Thanks for reminding. But we fail to see the PAPR issue from the perspective of different ports transmitted in different antennas. |
| Intel2 | Thanks for the discussion. Now we tend to agree with QC and other companies that there is no PAPR issue. |
| FL3 | Most companies think there is no PAPR issue, but Ericsson/ZTE think otherwise. I revised the proposal below. Proposal 3.1.3A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide at least one of the following options: Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol. Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8. FFS: potential impact on PAPR, if any. |

## 8 Tx SRS with TDM

### Cyclic vs sequential mapping

- We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

The general positions are:

Cyclic mapping

  - Supporting: Apple, CATT, CMCC, Futurewei, Intel, LG, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, Spreadtrum, vivo, xiaomi, ZTE

  - 16 18 proponents

Sequential mapping

  - Supporting: Ericsson, Google, New H3C

  - 2 3 proponents

- As we have to down select one option, I suggest we go with cyclic mapping.

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Views can be provided for the above proposal.

| Company | View |
| --- | --- |
| Google | Compared to cyclic mapping, we think sequential mapping is better from UE implementations perspective. For example, when s=2, the UE needs to change the power for each port symbol by symbol like [on, off, on, off,…]. We are not sure whether this could work based on current time mask definition in RAN4. Probably we can ask RAN4 before making the decision. |
| OPPO | Support the proposal. Can google clarify why the UE needs to change the power for each port symbol by symbol in case that the power is the same for different ports? |
| Lenovo | Support. |
| Spreadtrum | Support FL proposal. |
| CMCC | Support. To consider the phase consistence, cyclically mapping is benefit for gNB to estimate each 8-port SRS with more precise channel estimation performance. To consider the high-speed scenario, cyclically mapping is also benefit for gNB to estimate the Doppler property. |
| ZTE | Support. Cyclic mapping is better than sequential mapping considering collision. |
| Huawei, HiSilicon | Fine. |
| CATT | Support. Compared to option 2-2, option 2-1 can complete the transmission of one repetition of all 8 ports earlier. Besides, when there is a large interference lasting for several OFDM symbols during the transmission of the SRS resource, it is possible that only a part of ports can be received with option 2-2, while one or more repetitions of all SRS ports can be received with option 2-1. |
| Nokia/NSB | Support proposal, cyclic mapping reminds legacy operation with repetition. |
| QC | Support the FL proposal. |
| Intel | Fine with the proposal. |
| Vivo | Support |
| Xiaomi | Support the proposal |
| MediaTek | Support |
| Sharp | Support |
| DOCOMO | Support |
| New H3C | OK |
| Samsung | Support |
| Apple | We are okay. But for intra-slot frequency hopping, we need to discuss more than 2 cycles. |
| FL | Most companies support this proposal. @Google: My understanding is that for the 8Tx capable Ues, they should be able to switch to any antenna port(s) for transmission without any issue. Also UL transmissions with cyclic mapping have already been supported, so this does not seem to be a problem. Can you please elaborate the time mask issue? @Apple: I think it is being discussed in Sec. 3.2.5. |
| LGE | Support |
| Ericsson | Support the above proposal. |
| FL2 | @Ericsson: Thank you for being flexible. Now the proposal is discussed in the email reflector for potential email endorsement. |

### TDM factor s

We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Regarding the FFS points colored in red, the general positions are:

Support s = 4

  - Supporting: Apple, CATT, CMCC, Google, Huawei, HiSilicon, KDDI, Lenovo, MediaTek, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Sharp, Spreadtrum, xiaomi, ZTE

  - Against: Ericsson, Intel, NTT DOCOMO, OPPO, Samsung, vivo

  - 11 18 proponents and 6 5 opponents

Support s = 8

  - Supporting: KDDI, New H3C, Nokia, Nokia Shanghai Bell, Sharp, Spreadtrum

  - Against: Ericsson, Intel, NTT DOCOMO, OPPO, Samsung, vivo, ZTE

  - 5 proponents and 6 7 opponents

Determination of s

  - Implicit determination based on configured parameters: Ericsson, Nokia, Nokia Shanghai Bell, OPPO

  - Explicit determination based on configured parameters: CATT, Nokia, Nokia Shanghai Bell

Given the large number of proponents for s=4 and 8, we can see if an agreement can be achieved to support one or both of them.

Proposal 3.2.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4 and s = 8.

Views can be provided for the above enhancements. Proponents for implicit determination based on configured parameters can further suggest proposals.

| Company | View |
| --- | --- |
| Google | We are fine with s=4. |
| OPPO | We can accept s=4 if companies prefer, but s=8 is unnecessary. |
| Lenovo | We support s=4 for Ng=4 and suggest [s = 8]. |
| Spreadtrum | Fine with FL proposal |
| CMCC | At least TDM-4 can be supported to transmit 2 SRS antenna ports on each symbol and to increase the scheduling flexibility of network. |
| ZTE | Support s=4, NOT support s=8. |
| Huawei, HiSilicon | Support s=4 and open to discuss s=8. |
| CATT | We are fine to support s =4, and also fine with only s =2. |
| Nokia/NSB | We are fine with s=4. |
| QC | We think that the main concern from the opponents to support s=4 and 8 is that some value of m cannot be divided by s=4 or 8. We think we can address that concern by excluding those pair of {m,s} where m cannot divide by s. |
| Intel | Do not support the proposal. Larger value of s require more OFDM symbols for the 8-port SRS operation and is not preferred considering the limited uplink symbols in TDD. We think s=2 is sufficient. |
| vivo | Only s=2 is preferred, which is similar with two 4-port SRS resources in adjacent symbol. It’s enough to handle the case of larger delay spread as legacy design. |
| Xiaomi | We are fine with s=4 |
| MediaTek | We support s=4 only. |
| KDDI | We are fine with FL’s proposal. |
| Sharp | We support the proposal but can accept s=4 only. |
| DOCOMO | Fine with s=4. |
| New H3C | OK |
| Samsung | Do not support 4 and 8, longer s could be suffered by collision, so s=2 is sufficient. One of main reason to support TDMed port mapping is a low per-port power level considering 8-port, so having s=2, we could boost per-port power level as 4-port, which is already supported by legacy. So supporting only s=2 is enough. |
| Apple | s=8 can work for 8 symbol SRS and then intra-slot HF cannot be supported at the same time. S=4 can be a compromise |
| FL | Updated the support list. @Qualcomm: It was already agreed that “m = 2,4,8, 10,12,14, and m is a multiple of s”. Please let me know if I missed anything. |
| LGE | We support only s=2. Using a larger value increases the probability of collision. |
| Ericsson | Do not support the FL proposal, we support only s = 2 which is the only value that supports all values of m > 1. |
| QC | Thank FL for clarifying we already had agreement “m = 2,4,8, 10,12,14, and m is a multiple of s”. With this, we can support s=4 or 8. |

### TDM power control related issues

- A few power control related issues for TDMed 8 ports were discussed by some companies:

The UE splits a linear value PSRS of SRS transmission power equally across the configured SRS ports within each OFDM symbol

  - Supporting: CATT, CMCC, Huawei, HiSilicon, Intel, OPPO, Qualcomm

  - 7 proponents

Maintain the same SRS transmission power over the m OFDM symbols

  - Supporting: OPPO, Qualcomm, Sharp

  - 3 proponents

Other power control related proposals supported by one or two companies:

  - Introduce PCMAX,f,cSRS(i)

  - Full power transmission capability reporting

  - Power scaling when SRS overlaps with another uplink signal

- We can first try to reach an agreement on the power splitting proposal and further study the other proposals.

Proposal 3.2.3: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | In our view, this should be based on a UE capability. We suggest the following revision. Proposal 3.2.3: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol subjected to the UE capability. - If the UE does not support the UE capability, the UE splits a linear value PSRS of SRS transmission power equally across the total SRS ports configured across the OFDM symbols |
| OPPO | Support the proposal. Regarding the issue raised by google, if UE doesn’t support this UE capability, what is the benefit to support TDMed SRS transmission? We think a UE supporting TDMed 8 ports should support this power scaling naturally. |
| Lenovo | We understand the motivation of TDM based 8 ports SRS is the gain in power domain. Thus, we support Proposal 3.2.3. |
| CMCC | If UE splits the transmit power equally across the configured antenna ports for each symbol, the transmit power for each antenna port may exceed the PA capability. So, UE firstly splits PCMAX across the symbols for SRS transmission, then UE splits the transmit power equally across the configured antenna ports for each symbol. |
| ZTE | Support. Google’s proposal should be discussed. To our understanding, TDM scheme aims to transmit SRS with less ports over one symbol to allocate higher power to each port, if a UE cannot support full-power transmitting with part of ports, e.g., 4 ports, over each symbol, it is better not configure TDM SRS to such UE. From this perspective, it can be precluded up to gNB implementation. |
| Huawei, HiSilicon | Support FL proposal and fine with Google’s revision. |
| CATT | Support. |
| QC | Support the FL proposal in principle. Regarding Google/ZTE comment on SRS max power, we partially agree but not fully agree. This is an SRS max power issue. The issue only exists when UE enter PA power saturation region, while there is no issue to adopt FL proposal when UE has not enter PA power saturation yet. In other words, when UE tx power is low (e.g. cell center UE), UE should always be able to split power PSRS based on the FL proposal. When UE tx power is high/close to PA saturation (e.g. cell edge UE), UE may not able to split power PSRS based on the FL proposal. Adding the static UE capability does not solve the issue. Introduce a new Pcmax for SRS which is low than legacy PUSCH Pcmax can solve this issue. Therefore, we propose the following to solve the SRS max power issue. Proposal: When 8-ports SRS in an SRS resource are mapped to m>1 OFDM symbols with TDM factor s, introduce PCMAX,f,cSRS(i) and modify SRS power control equation as the following. PSRS,b,f,ci,qs,l=minPCMAX,f,cSRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,cqs∙PLb,f,cqd+hb,f,ci,l[dBm] - FFS: whether include PCMAX,f,cSRS(i) in power headroom report, or report a difference between PCMAX,f,cSRS(i) and PCMAX, f,c(i) in UE capability report. Fig 3: An example of SRS max power being smaller than PUSCH max power. Of course, we understand the group need to discuss the max power issue more before we can agree a solution. The above is just one candidate solution. Anyway, for the FL proposal, we suggest the following update. Proposal 3.2.3: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol. - FFS: how to determine max SRS transmission power with the above proposal for TDM factor s>1 |
| Intel | Ok with the FL proposal. Regarding the issue mentioned by other companies, e.g., the transmit power exceed the PA capability, we think the linear value of SRS transmit power could be scaled by a factor, which is similar with the full power scaling factor as used for PUSCH power control. |
| Vivo | Support the FL proposal |
| MediaTek | Support |
| Sharp | In our view, the UE should split power equally across all the 8 SRS ports, not the SRS ports configured on each OFDM symbol (i.e., maintaining power consistency across s OFDM symbols is also needed) to estimate a channel for the 8 SRS ports. |
| DOCOMO | Perhaps multiple issues might be here. Following Qualcomm’s text, there are an issue on power division and on max. power limit. The first issue exists in 7.3 of 38213. We agree to solve that issue. The FL proposal is a good starting point, while its potential problem is it may have impacts on the second issue by saying “PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol”, because it is in some cases impossible. In summary, we support to consider how to divide power into multi symbols used by a SRS resource with TDM. We are also supportive to discuss the issue raised by QC, since from NW perspective to obtain DL CSI from SRS, we believe accurate awareness on UE power should be achieve as much as possible. |
| New H3C | OK |
| Samsung | Support FL proposal. Regarding UE capability, we have same understanding with OPPO, if a UE supports TDMed SRS port mapping, the UE shall support power splitting equally. |
| Apple | Needs more discussion. Not all PA can support it. |
| FL | I think the Pcmax issue can be further discussed, and it can be decoupled from Proposal 3.2.3 for the moment as Proposal 3.2.3 is about PSRS. How PSRS can be computed from Pcmax will be further discussed. Based on the discussions, the proposal is updated along the line of Google: Proposal 3.2.3A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, capped by the maximum SRS transmissions power according to UE capability. |
| ZTE | We don NOT agree to introduce a new term of max power for SRS dedicated. The max power for UL should be the same for PUSCH and SRS which depends on UE capability for PA capability of each physical antenna. Here we just need to discuss whether SRS with 4 ports can be transmitted by using all or part of the max power of PUSCH. There is no need to have different max power levels for PUSCH and SRS with a same set of ports. It is possible to reuse full power capability for 8Tx PUSCH. If a UE reports full power capability for a codebook with 4 NZP ports, it can support full power SRS transmission on same ports. |
| QC | We can support update FL proposal 3.2.3A, although the formulation is not what we preferred. To ZTE: It does not make sense to tie SRS max power with PUSCH max power in the scenario of TDMed SRS. Please notice that when 8-ports SRS is TDMed, PUSCH 8L transmission are not TDMed. Therefore, given a coherent 8 Tx UE with 8 PAs as below, SRS max power with TDM is lower than PUSCH max power. This is simply just a fact. |
| OPPO | We agree with QC that the max transmit power in one SRS symbol depends on the PA architecture at UE. A UE with 14dBm PA would have lower max transmit power in one symbol than a UE with 17/20/23 dBm PA. One further question for clarification: Is it common understanding that only the UE with PAs of the same max power is considered for TDMed SRS? Do we need to consider the UE with unbalanced PA (e.g. 20+20+14+14+14+14+14+14 dBm), which would lead to different max power in different symbols? |
| Intel2 | For TDMed SRS, when power boosting is applied, the SRS transmission power according to the power control formula may exceed the maximum power of the UE PA. With the PA example from QC, considering two subsets (s=2), the SRS power control output power might be larger than 14dBm, which exceeds the PA max output. We think the unbalanced PA architecture as mentioned by OPPO is also valid. The SRS transmission power could be scaled, similar with the power scaling factor as PUSCH full power operation. Or the Pcmax could be scaled, similar as defining a new Pcmax for SRS as proposed by QC. |

### Port splitting for TDM

- Regarding how the 8 ports are split into s subsets for TDM, some companies provided the following discussions.

Option 1: Within each of the s subsets, the port indexes are consecutive. For example, for s=2, the first subset has {1000,1001,1002,1003}, and the second subset has {1004, 1005, 1006, 1007}.

  - Supporting: CATT, CMCC, Ericsson, Futurewei, Nokia, Nokia Shanghai Bell, Samsung

  - 7 proponents

Option 2: Within each of the s subsets, the port indexes are not necessarily consecutive. For example, for s=2, the first subset has {1000,1001,1004,1005}, and the second subset has {1002, 1003, 1006, 1007}.

  - Supporting: ZTE

  - 1 proponent

- We can see if Option 1 is agreeable.

Proposal 3.2.4: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, within each of the s subsets, the port indexes are consecutive.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We can decide this after we make the decision of the supported value of s. |
| OPPO | Not support. We think the port splitting should consider the output of antenna port group indexing. One antenna port group can be allocated in the same symbol for Ng>1. For example, if port {0,1,4,5} and {2,3,6,7} are different antenna groups for Ng=2, option 2 should be applied. |
| Lenovo | We think the UE coherent capability should be considered. For partial coherent UE, the antenna ports within a same coherent antenna group should be in a same OFDM symbol. Thus, we can accept option 2. |
| Spreadtrum | Basically fine with FL proposal. For other options, e.g., {1000,1002,1004,1006}, {1001,1003,1005,1007}, can be further discussed. |
| CMCC | Support |
| ZTE | Not support. It is straightforward to use a common port index splitting scheme in TDM and non-TDM multiple-comb-offset cases, e.g., ports {1000, 1002, 1004, 1006} in the first subset and ports {1001, 1003, 1005, 1007} in the other subset. To our understanding, port index splitting actually reflects the mapping relationship between 8 port indexes and panels / port groups (Ng in AI 9.1.4.2), this relationship should maintain unchanged in both TDM and non-TDM multiple-comb-offset cases. |
| Huawei, HiSilicon | We think the FL proposal can be postponed. From our point of view, the ports within the same coherent group should be mapped to the same OFDM symbol if the number of ports within one OFDM symbol allows. Otherwise, if the ports within the same coherent group are mapped to multiple separated OFDM symbols, the coherency of the ports may not be maintained. Thus, it is better to first determine the port indexing of 8Tx UL codebook in 9.1.4.2. |
| CATT | We prefer to map ports for the same coherent group to the same OFDM symbol(s). Since there is no conclusion on which ports are for the same coherent group in AI 9.1.4.2 yet, maybe we can have a proposal on principle first. |
| Nokia/NSB | Support |
| QC | Support the FL proposal. |
| Intel | Agree with Google’s view. |
| vivo | Support |
| Xiaomi | Support the FL proposal. |
| MediaTek | Support |
| Sharp | We can wait for progress in AI9.1.4.2 because indexing of antenna port for each antenna port group is not defined yet. |
| New H3C | The same view with HW and sharp. |
| Samsung | Support in principle, but also fine with discussing after the progress on AI9.1.4.2, i.e., how to allocate ports within a coherent antenna group. |
| Apple | We are fine |
| FL | It seems there is a large number of proponents for the current proposal, and a large number of proponents for a proposal based on the coherent antenna group. Both can be considered. So I added an alternative proposal: Proposal 3.2.4A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1: For the SRS resource associated with coherent antenna group(s): the port indexes associated with one coherent antenna group are mapped to the minimum number of subsets of the totally s subsets. FFS details. For all other cases: the port indexes are consecutive within each of the s subsets. |
| LGE | Support the FL proposal. |
| CATT | Ok with the updated proposal. |
| ZTE | Regarding FL’s proposal, NOT support to have different solutions which causes unnecessary complexity. Port index grouping used for coherent antenna groups can also be reused for non coherent antenna ports. |
| QC | Thank FL for updating the proposal. But we don’t fully follow the language/formulation of the Proposal 3.2.4A. What does it mean by “mapped to the minimum number of subsets”? How to determine that minimum number. Things seem not clear here. |
| Samsung | Thank FL for the updated proposal, but we also have similar view with QC. Also, in the second bullet, “the port indexes are consecutive” means that port indexes are always consecutive? I think exact port indexes included in a coherent antenna group has not been agreed yet, so it may not be always consecutive. |
| OPPO | We have similar view as ZTE. We should strive to the same port mapping for different coherent cases. |
| Intel2 | The update proposal is confusing. What does it mean by “SRS resource associated with coherent antenna groups”? Also similar question as QC. The text “mapped to the minimum number of subsets” is confusing. |

### Resource allocation for the s subsets of ports over s OFDM symbols

Several companies discussed the resource allocation for the s subsets of ports over s OFDM symbols. The general positions are:

PRB resources when FH/RPFS/repetition are configured

  - The s subsets of ports have the same PRB resource allocation, i.e., the s OFDM symbols have the same PRB resource allocation and FH/RPFS is not utilized within the set of s OFDM symbols. Accordingly, the l'/R part in the expression of nSRS will need to be changed to l'/(sR). It is also suggested that the configured m (for the number of OFDM symbols) should be divisible by sR, which helps maintain the desired grouping of the subsets of ports when repetition is configured.

  - This may depend on the outcome of cyclic vs sequential mapping, so we can further discuss when an agreement is made in Sec. 3.2.1.

  - Supporting: Apple, CMCC, Qualcomm

RE or code-domain resource when sequence/group/comb offset/cyclic shift hopping is configured

  - Over one set of s OFDM symbols, sequence/group hopping behaves the same way as legacy design (i.e., hop on every OFDM symbol)

    - Supporting: Qualcomm

  - Over one set of s OFDM symbols, comb offset / cyclic shift may be different

    - Supporting: ZTE

  - Over one set of s OFDM symbols, comb offset / cyclic shift hopping remains unchanged (i.e., hop on every s OFDM symbol)

    - Supporting: ZTE

Views can be provided for the above enhancements, and companies can feel free to suggest proposals for the group to consider.

| Company | View |
| --- | --- |
| OPPO | We support the same PRBs for the s subsets of ports. |
| ZTE | Regarding the first bullet, we agree to keep same PRB resources for s symbols, so agree to change the l'/R part in the expression of nSRS to l'/(sR). Regarding the second bullet, this issue is similar to issue 2.1.1. We believe that, for code-domain hopping (i.e., CS/group/sequence hopping), the hopping pattern can be different over s symbols; for comb offset hopping, the hopping pattern should remain unchanged over s symbols. |
| Huawei, HiSilicon | Support the discussion about the PRB resources when FH/RPFS/repetition are configured. |
| CATT | For the first issue, support the s subsets of ports have the same PRB resource allocation. For the second issue, delay the discussion until there is conclusion on hopping pattern for comb offset/cyclic shift hopping. |
| QC | We support the spirit of the proposal FL put under “PRB resources when FH/RPFS/repetition are configured”. Our formulation of the proposal is as follows for the group to consider. Proposal: When 8-ports SRS in an SRS resource are mapped to m>1 OFDM symbols with TDM factor s, reuse the SRS frequency hopping location equation Fb(nSRS) as in TS 38.214. Redefine as the following. - For Aperiodic SRS, nSRS=l'sR within the slot in which the symbol SRS resource is transmitted. - For periodic and semi-persistent SRS, nSRS= Nslotframe,μnfμ+ns,fμ-ToffsetTSRSNsymbSRSsR+l'sR For sequence/group hopping, we support the legacy way (hop on every OFDM symbol). Actually, it seems that the second bullet (under sequence/group hopping) is proposing the same as (or at least similar to) the first bullet? For comb/CS hopping, we are ok to wait until the discussion in section 2 is matured. |
| Vivo | Support the first issue that the s subsets of ports have the same PRB resource allocation. |
| New H3C | Open to discuss |
| Samsung | Fine to discuss both 1st and 2nd bullet, but at least 2nd bullet seems similar with Issue 2.1.1, so we could utilize the outcome of 2.1.1. |
| FL | Most companies are fine with the line of the first issue. We can further discuss the other issues. Initial proposals related to the first issue are provided below, but the change from l'/R to l'/(sR) may need to wait after the decision on cyclic/sequential mapping. Proposal 3.2.5-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the s subsets of ports have the same PRB resource allocation. Proposal 3.2.5-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, with TDM factor s > 1, m ≥ 2 OFDM symbols in a slot, and repetition factor R ≥ 1, m is a multiple of sR. |
| LGE | Support the first issue that the s subsets of ports have the same PRB resource allocation. |
| CATT | There are two proposals numbered 3.2.5-1. I assume the second one is 3.2.5-2. We are ok with proposal 3.2.5-1 and 3.2.5-2. |
| FL2 | @CATT: Thanks for pointing out the typo. The proposal numbering is updated; no change in the content. Proposal 3.2.5-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the s subsets of ports have the same PRB resource allocation. Proposal 3.2.5-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, with TDM factor s > 1, m ≥ 2 OFDM symbols in a slot, and repetition factor R ≥ 1, m is a multiple of sR. |
| QC | We support the two proposals in general. |
| Samsung | Thank FL for updating the proposal. Support Proposal 3.2.5-1 and 3.2.5-2. |
| Xiaomi | Support both proposals |
| OPPO | Support the proposals. |
| Intel2 | We prefer to discuss this after the cyclic or sequential mapping pattern is determined. |

### Collision handling for TDMed ports

Several companies discussed potential collision handling rules for TDMed ports, such as dropping rules. The general positions are:

If the SRS transmission on one of the s OFDM symbols is dropped, the SRS transmission on the other s-1 OFDM symbols may also be dropped. Whether to drop on the other s-1 OFDM symbols may depend on the usage and coherence, repetition configuration, whether the UE can change the transmission order of the subsets of ports, etc.

  - Supporting/supporting to study: Google, Huawei, HiSilicon, MediaTek, New H3C, OPPO, Qualcomm, Samsung, vivo, Xiaomi

  - Against: CATT, Intel, ZTE

Views can be provided for the above enhancements, and companies can feel free to suggest proposals for the group to consider.

| Company | View |
| --- | --- |
| Google | Support to study this issue. Besides, we also think we need to study the power scaling for partial overlapping when the total Tx power for the SRS and other UL signals exceeds the maximum Tx power. |
| ZTE | Not support. If part of symbols are dropped due to collision, SRS ports over other symbols should be transmitted normally. Especially when repetition is enabled, if some symbols are dropped, the same SRS ports can be transmitted over other symbols. If cyclic mapping in 3.2.1 is supported, there is less possibility that multiple nonconsecutive symbols carrying a same subset of ports are all dropped. Even all symbols carrying a same subset of ports, e.g., first subset, are dropped, other subsets of ports, e.g., second subset, can be transmitted and estimated by gNB. At the next time, if the second subset of ports are dropped due to collision, then the first subset of ports can be transmitted normally. In a word, each symbol carrying SRS port(s) should be determined to be dropped or not independently. |
| Huawei, HiSilicon | Support to study. |
| CATT | Reusing legacy dropping rule is preferred, i.e., only the SRS transmission on overlapped symbol(s) is dropped. |
| QC | We agree with the general position FL mentioned. Specifically, proposal from QC is the following. Proposal: When an 8-ports SRS resource is mapped to m (where m>1) OFDM symbols, if a subset (denote as S) of OFDM SRS symbols are dropped, treat the other SRS OFDM symbols as the following. - If the usage of the 8-port SRS is “codebook” and the codebook is fully coherent, all the m SRS OFDM symbols are considered as dropped. - If the usage of the 8-port SRS is “codebook” and the codebook is partial coherent, only the SRS OFDM symbols whose SRS ports are coherent with the SRS ports in the subset S are considered as dropped, while other SRS OFDM symbols are considered as transmitted. - If the usage of the 8-port SRS is “codebook” and the codebook is non-coherent, only the SRS OFDM symbols in the subset S are considered as dropped, while other SRS OFDM symbols are considered as transmitted. - If the usage of the 8-port SRS is “antennaSwitching”, only the SRS OFDM symbols in the subset S are considered as dropped, while other SRS OFDM symbols are considered as transmitted. |
| Intel | We don’t see the strong need for this. |
| vivo | Open to study. |
| Xiaomi | Support to study |
| MediaTek | Open to further discussions. |
| New H3C | OK to study |
| Samsung | Support to study since this issue is a main drawback on TDMed SRS port mapping. |
| FL | A large number of companies support to study this issue. An initial proposal is suggested below: Proposal 3.2.6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, study, when the SRS transmission on one of the s OFDM symbols is dropped, whether the SRS transmission on the other s-1 OFDM symbols may also be dropped, such as based on the usage and coherence, repetition configuration, possibility of UE changing the transmission order of the subsets of ports, etc. |
| LGE | Support to study. If the SRS transmission on one of the s OFDM symbols is dropped, dropping the other s-1 OFDM symbols is needed for a full 8-port transmission. |
| CATT | Ok to study the issue. But we don’t have list all the factors, i.e., the following revision is suggested: Proposal 3.2.6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, study, when the SRS transmission on one of the s OFDM symbols is dropped, whether the SRS transmission on the other s-1 OFDM symbols may also be dropped, such as based on the usage and coherence, repetition configuration, possibility of UE changing the transmission order of the subsets of ports, etc. |
| FL2 | We can try CATT’s suggestion. Details can be further discussed. Proposal 3.2.6A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, study, when the SRS transmission on one of the s OFDM symbols is dropped, whether the SRS transmission on the other s-1 OFDM symbols may also be dropped. |
| QC | We think the dropping is not limited to only 1 OFDM symbol. We can formulate the proposal in a more generic way. We suggest the following Modified Proposal 3.2.6A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, study, when the SRS transmission on one a subset of the s OFDM symbols is dropped, whether or not a UE drop the SRS transmission on the other s-1 the rest OFDM symbols may also be dropped. |
| Samsung | Since this is for the purpose of study, we think that it is good for including some possible solutions. Actually our original proposal for handling collision was changing the transmission order of the subsets of ports at least when periodic overlapping happens. Hence, based on QC’s modified proposal (thank you), we would like to suggest adding “such as” part again as follows: Modified Proposal 3.2.6A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, study, when the SRS transmission on one a subset of the s OFDM symbols is dropped, study the following solutions: - Whether or not a UE drops the SRS transmission on the other s-1 the rest OFDM symbols may also be dropped, based on the usage, coherency, and/or repetition configuration - Whether or not a UE changes the transmission order of the subsets of ports |
| FL3 | We can try Samsung’s version. Proposal 3.2.6B: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the SRS transmission on a subset of the s OFDM symbols is dropped, study at least the following solutions: Whether or not a UE drops the SRS transmission on the rest of the s OFDM symbols, based on the usage, coherency, and/or repetition configuration Whether or not a UE changes the transmission order of the subsets of ports |

## Other proposals for 8Tx SRS

Several companies further detailed enhancements for the 8Tx SRS, and some of the enhancements may be related to the outcomes of above discussions.

Enh. 1: Port mapping for TDM based on antenna coherent group

  - Supported by: CATT, Huawei, HiSilicon, OPPO

Enh. 2: Maintain phase/beam consistency over multiple OFDM symbols for TDM

  - Supported by: OPPO, Sharp

Enh. 3: Whether to down select from existing resource mapping schemes for TDM

  - Discussed by: Ericsson, ZTE

Enh. 4: Whether to downgrade configuration of SRS for antenna switching

  - Supported by: CMCC

Enh. 5: Same number of symbols and TDM pattern for SRS resources for a SRS resource set

  - Supported by: Apple

Views can be provided for the above enhancements, and new details / proposals can also be included.

| Company | View |
| --- | --- |
| ZTE | For Enh. 1: Support. We believe this is relevant to 3.2.4, and can be discussed together. For Enh. 2: Open to discuss. For Enh. 3: No need to down-select. For Enh. 4: Open to discuss. Low priority. For Enh. 5: Open to discuss. |
| Huawei, HiSilicon | Enh.1: Support. Enh.2: Support. Enh.3: Support. Enh.4: Open to discuss. Enh.5: Open to discuss. |
| CATT | Enh. 1 is related to the issue in section 3.2.4. It can be discussed in section 3.2.4. |
| KDDI | We support Enh. 1, and we are open to discuss for the others. |

# Conclusions

## For Wednesday GTW

(Proposal 3.1.1A is also pending email endorsement)

Proposal 3.1.1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

- If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset

- If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.

(Proposal 3.1.2B is also pending email endorsement)

Proposal 3.1.2B: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with comb KTC=2 and with maximum nSRScs,max=8 cyclic shifts per comb offset, the number of comb offset(s) and the cyclic shift locations are determined based on the one RRC configured cyclic shift location nSRScs as follows:

- If nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used.

- The 8 cyclic shift locations for the 8 ports are {nSRScs, (nSRScs+1) mod nSRScs,max, …,(nSRScs+7) mod nSRScs,max}, reusing the existing equation nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max in 38.211 6.4.1.4.2.

Proposal 2.1.1C: For a SRS resource configured with comb offset hopping and/or cyclic shift hopping,

- If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

- If the repetition factor R > 1,

  - For cyclic shift hopping, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

    - FFS: the time-domain hopping behavior depends on the OFDM symbol index l' of the first symbol across the R repetitions.

  - For comb offset hopping, within a slot, the time-domain hopping behavior depends on one of the following alternatives:

    - Alt1: The OFDM symbol index l' of the first symbol across the R repetitions.

    - Alt2: The OFDM symbol index l' of each symbol.

    - Alt3: The OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on configuration, and FFS configuration details.

Proposal 3.2.1B: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols.

- FFS: For UE that supports TDMed SRS, {{1, 2, …, s}, … , {1, 2, …, s}} with more than one {1,2,…,s} mapped to the m OFDM symbols is supported as a UE optional feature.

Proposal 3.2.5-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the s subsets of ports have the same PRB resource allocation.

Proposal 3.2.5-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, with TDM factor s > 1, m ≥ 2 OFDM symbols in a slot, and repetition factor R ≥ 1, m is a multiple of sR.

Proposal 2.1.2B: For SRS comb offset hopping / cyclic shift hopping, at least support reinitialization at the beginning of each radio frame. Additionally, support at least one of the following alternatives for SRS comb offset hopping / cyclic shift hopping / sequence hopping / group hopping:

- Alt 1: reinitialization periodicity of N radio frames

  - FFS: N is fixed or configurable.

- Alt 2: reinitialization based on system frame number (SFN).

Proposal 2.1.2: For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID is down selected from the following options:

Option 1: cinit=nIDSRS.

Option 2: New ID(s)

  - Option 1-1: cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

  - Option 1-2:

    - cinit=nIDCS, where nIDCS is a new ID for cyclic shift hopping.

    - cinit=nIDCO, where nIDCO is a new ID for comb offset hopping.

    - [cinit=nIDCSCO, where nIDCSCO is a new ID for the combined cyclic shift and comb offset hopping, if supported.]

  - Default ID: If no ID is explicitly configured in RRC configuration, the SRS sequence identity nIDSRS is used.

  - FFS: the value range, e.g., 0~1023, 0~65535.

Proposal 3.1.3A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide at least one of the following options:

Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8.

FFS: potential impact on PAPR, if any.

Proposal 3.2.6B: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the SRS transmission on a subset of the s OFDM symbols is dropped, study at least the following solutions:

Whether or not a UE drops the SRS transmission on the rest of the s OFDM symbols, based on the usage, coherency, and/or repetition configuration

Whether or not a UE changes the transmission order of the subsets of ports

Proposal 3.2.3A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, capped by the maximum SRS transmissions power according to UE capability.

# References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#94-e.

R1-2302303, Enhancements on SRS for CJT and 8TX UEs, InterDigital, Inc.

R1-2302314, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2302374, Discussion on SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2302420, SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2302473, Further discussion on SRS enhancements, vivo.

R1-2302536, SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2302580, Discussions on SRS enhancement in Rel-18, New H3C Technologies Co., Ltd.

R1-2302589, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2302684, Discussion on Rel-18 SRS enhancement, CATT.

R1-2302727, Discussion of SRS enhancement, Lenovo.

R1-2302776, On SRS enhancements targeting TDD CJT and 8 TX operation, Oy LM Ericsson AB.

R1-2302784, Discussion on SRS enhancement in Rel-18, Intel Corporation.

R1-2302963, Discussion on SRS enhancements, Xiaomi.

R1-2303009, SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2303046, On SRS Enhancement, Google.

R1-2303072, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2303116, Views on SRS enhancements, Samsung.

R1-2303170, Views on SRS enhancement targeting TDD CJT and 8 TX operation, KDDI Corporation.

R1-2303181, SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2303192, Discussion on SRS enhancement targeting TDD CJT, ETRI.

R1-2303220, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2303471, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2303577, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2303679, Discussion on SRS enhancement, NEC.

R1-2303701, Discussion on SRS enhancement, NTT DOCOMO, INC.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |  |
| --- | --- | --- | --- |
| Parameter |  |  | Value |
| Duplex, Waveform |  |  | TDD, OFDM |
| Multiple access |  |  | OFDMA |
| Scenario |  |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  |  | Follow TR36.873 |
| UE receiver noise figure |  |  | 9dB |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 30kHz |  |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |  |
| Frame structure |  | DSUDD, or companies to state the used frame structure |  |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |  |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |  |
| Overhead |  | Companies shall provide the downlink overhead assumption |  |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |  |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |  |
| UE receiver |  | MMSE-IRC as the baseline receiver |  |
| DL Channel estimation |  | Realistic |  |
| Evaluation Metric |  | DL throughput |  |
| Baseline for performance evaluation |  | R17 SRS design |  |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |  |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## Appendix 3: Agreements from RAN1#110bis-e

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

## Appendix 4: Agreements from RAN1#111

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on:

Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID.

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

## Appendix 5: Agreements from RAN1#112

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- Option 2: For comb 4, do not support 4 comb offsets.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Conclusion

No consensus to support the following for TDD CJT SRS enhancement in Rel-18:

Further enhancements to frequency hopping

Sequence hopping/randomization, per-hop sequence from a long SRS sequence

Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

Precoded SRS for DL CSI acquisition

Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

Configuration of  (sequence index within a group) per SRS resource

Multiplying mask sequence to the legacy SRS sequence

Conclusion

No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.