3GPP TSG RAN WG1 #112bis-e		                                                      R1-2304025

e-Meeting, April 17th – April 26th, 2023

Source:	Moderator (CATT)

Title:	Summary of [112bis-e-AI7.1-05] on NFI definition

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions during RAN1#112bis-e on the following CRs.

[112bis-e-AI7.1-05] NR Rel-15/16 maintenance on NFI definition by April 21 – Yanping (CATT)

R1-2302677	Correction on NFI definition for Type-2 HARQ-ACK codebook in Rel-16	CATT

R1-2302678	Correction on NFI definition for Type-2 HARQ-ACK codebook in Rel-17	CATT

- Discussion

  - 1st round discussion

As defined in TS 38.212, there is only one NFI field in a DCI format. However, the description of NFI in TS 38.213 always assumes two NFI fields in a DCI format for different PDSCH groups, which is not aligned with TS 38.212. CRs in R1-2302677 and in R1-2302678 proposed corrections on NFI in TS 38.213 to align with TS 38.212.

Question 1: Do you agree with the CR in R1-2302677 for Rel-16? If not, please elaborate your reasons.

|  | Company |
| --- | --- |
| Agree | LGE |
| Not agree | Ericsson, Intel, LGE |

| Company | Comments |
| --- | --- |
| Ericsson | From the clause, it is clear that it is applicable when pdsch-HARQ-ACK-Codebook-r16 is configured, and from 38.212 that is only the case when the field is present. |
| Intel | The change is not necessary. With the specification in 38.212, “the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group”, it is already clear which bit in the 2-bit NFI should be applied to which PDSCH group. |
| LGE | We are open to either way. (the CR can be considered if clarification is needed among the group) |
| Samsung | Although 38.212 does provide the information mentioned by Ericsson and Intel, the CR does make the specifications clearer. Based on consideration of 38.212, it may not be argued as an essential correction but we would be OK to include it in an alignment CR (to align with 38.212). |
| ZTE, Sanechips | We are open to change spec to make it more clearer . |
| Huawei, HiSilicon | We are also open to the change |

Question 2: Do you agree with the CR in R1-2302678 for Rel-17? If not, please elaborate your reasons.

|  | Company |
| --- | --- |
| Agree | LGE |
| Not agree | Ericsson, Intel, LGE |

| Company | Comments |
| --- | --- |
| Ericsson | Same as above. |
| Samsung | We would be OK to include the proposed text in the Rel-17 alignment CR from RAN1#112bis as explained in our response to Q1. |
| ZTE, Sanechips | Same comments as above. |
| Huawei, HiSilicon | We are also open to the change |

  - 2nd round discussion

Based on the feedback in the 1st round discussion, Ericsson and Intel do not think the change is necessary while LG, Samsung, ZTE and Huawei are open to the change.

As a compromise, it is proposed to adopt the change as part of alignment CR. Based on the guidance from Mr. Chair, we do not have alignment CR for Rel-15 and Rel-16 from this meeting. Therefore, it is proposed to adopt the change from Rel-17. Hopefully it is acceptable to all the companies.

Proposal: Endorse the text proposal in R1-2302678 as part of Rel-17 TS 38.213 alignment CR.

| Company | Comments |
| --- | --- |
| Intel | We are fine to follow majority views. |
| Samsung | As we commented in the 1st round, we support the proposal. |
| LGE | OK with the proposal. |
| Huawei, HiSilicon | We are fine with the proposal. |
| ZTE, Sanechips | Fine |

- Conclusion

The conclusion of the discussion is as follows.

For the TS 38.213 editor:

The text proposal in R1-2302678 is provided to improve the clarity of the RAN1 specifications. Please include it in the Rel-17 alignment CR.