3GPP TSG RAN WG1 #112bis-e		                                                      R1-2304028

e-Meeting, April 17th – April 26th, 2023

Source:	Moderator (CATT)

Title:	Summary #1 of subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the inputs and the discussions on subband non-overlapping full duplex in RAN1#112bis-e.

- Proposals for online sessions

  - April. 18th (Tue)

Proposal 1-2a

Proposed Agreement:

One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration.

Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition.

  - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed

  - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required

Proposal 1-5

Proposed Conclusion:

At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any.

- Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Proposal 1-8a

Proposed Agreement:

Study the following options for dynamic SBFD.

- Option 1: Dynamic SBFD is achieved by flexible subband where RBs in flexible subband can be dynamically used for UL transmission and DL transmission.

- Option 2: Dynamic SBFD is achieved by scheduling DCI which indicates whether allowing DL receptions outside semi-statically configured DL subband and/or allowing UL transmission outside semi-statically configured UL subband.

  - Option 2-1: explicit indication in scheduling DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol, or whether DL receptions outside semi-statically configured DL subband and/or UL transmission outside semi-statically configured UL subband are allowed

  - Option 2-2: implicit indication in scheduling DCI based on resource allocation

- Option 3: Dynamic SBFD is achieved by non-scheduling DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol.

Proposal 1-12a

Proposed Agreement:

If PRG is determined as wideband, study the following two options:

- Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

  - Same precoding is assumed within each DL subband

  - FFS whether same precoding is assumed across two DL subbands

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

Proposal 1-18a

Proposed Conclusion:

For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed.

Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type.

The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2, which requires additional specification efforts.

Option 1 may increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may reduce the transmission/reception latency and improve coverage.

Proposal 3-4

Proposed Agreement:

For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering:

  - Necessity/benefit compared with measurement within DL subband

  - Whether/how to estimate inter-subband CLI from RSRP/RSSI measurements within UL subband

  - Whether UE is required to measure RSRP/RSSI within UL and receive DL in DL subband(s) simultaneously

  - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband

    - If not, identify the potential impact

Proposal 3-5

Proposed Agreement:

For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods:

  - Method#1: separate CLI-RSSI measurement resources/reports in each DL subband

    - Note: supported in existing specifications

  - Method#2: CLI-RSSI measure/report in one DL subband only

    - Note: supported in existing specifications

  - Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands

    - FFS: report single or separate report(s)

For Method#3, if agreed, consider the following options for non-contiguous CLI-RSSI resource allocation in frequency:

  - Option 1: Two contiguous CLI-RSSI resources configured to be linked to one CLI-RSSI report

  - Option 2: One CLI-RSSI resource

    - Option 2-1: Non-contiguous CLI-RSSI resource allocation

    - Option 2-2: One contiguous CLI-RSSI resource allocation with non-contiguous CLI-RSSI resource derived by excluding frequency resources outside DL subband (s)

Proposal 3-6

Proposed Agreement:

Study whether/how to support subband CLI-RSSI measurement and reporting.

- General aspects of SBFD schemes

This section discusses the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

  - Summary of input contributions

The inputs from companies’ contributions are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or are missed. Companies are encouraged to correct/update the summary with revision marks if needed.

    - SBFD operation

The following agreement was made in RAN1#112 to study whether or not a slot can consist of both SBFD and non-SBFD symbols.

Agreement Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration

Use cases

The following use cases are identified by companies for a slot consisting of both SBFD and non-SBFD symbols.

    - Slot of DL, UL, and flexible symbols with SBFD symbols and full UL symbols [8][12][14][15][18][20][23][24][33][34][35][36]

    - Use remaining symbols in slots with symbols cannot be used for SBFD operation e.g. SSB etc. for SBFD operation [11][15][20][24][25][27]

    - Some end symbols of DL slot can be configured with the UL subband [18][36]

Implementation complexity

In terms of implementation complexity, vivo thinks that it may not introduce much complexity at gNB side when switching between SBFD symbols and non-SBFD symbol is also allowed within a slot, since such switching is under gNB’s control. For SBFD aware UEs, if there is much complexity, the maximum number of allowed switching points between SBFD and non-SBFD symbols within a slot at UE side can be further studied similar to that for FG5-1 “at most one switch point per slot for actual DL/UL” and FG 5-1b “More than one DL/UL switch point in a slot”.

Intel thinks that if a UL transmission or a DL reception is only within single symbol type (SBFD or non-SBFD symbol), the complexity of symbol-level SBFD configuration is similar to the case of slot-level SBFD configuration.

CMCC thinks that whether per-symbol subband time location configuration granularity will reduce the system capacity or not is up to gNB’s antenna configuration implementation. For example, gNB doesn’t need to change the connection between Tx/Rx chain and panel groups between DL/UL and SBFD symbols if the SBFD antenna configuration Option-2 (Method 2-1) and SBFD antenna configuration Option-3 (Method 3-1) are implemented, which will not introduce the DL/UL/SBFD switching time.

Xiaomi doesn’t see additional difficulties in terms of implementation complexity from both gNB side and UE side.

Ericsson thinks that additional guard symbols will need to be added for transitions between SBFD symbols and non-SBFD symbols to allow the hardware reconfigurations to activate/deactivate and stabilize in practice. Large numbers of such SBFD/non-SBFD symbol transitions will incur significant radio resource losses from the required guard symbols and proposed to study the number of transitions between SBFD symbols and non-SBFD symbols to be supported in a TDD UL/DL cycle.

Qualcomm observed that from gNB perspective, switching between SBFD and non-SBFD symbols may require a transition guard period to switch the panels, tune filter and adjust timing which disrupt the transmission or reception. From UE perspective, SBFD-aware UE may need to do filter retuning and UL sampling rate adjustment from SBFD to UL-SB. For the other example of switching from DL to SBFD, SBFD-ware UE may adjust DL filtering between DL symbol and SBFD symbols which interrupts DL reception.

Samsung thinks that frequent symbol-level switching in a slot of the gNB-side SBFD antenna configuration and SIC implementation, i.e., from/to {DL-only, UL-only, SBFD} should not be expected.

Compatibility with legacy TDD DL/UL configuration

Vivo, OPPO, ZTE, Intel, Sony, Samsung, DOCOMO pointed out that the minimum granularity of TDD configuration, time domain resource allocation and collision handling etc. are symbol level and the same granularity should be followed for SBFD operation.

Companies’ views are summarized below.

- A slot can consist of both SBFD and non-SBFD symbols

  - Supported by: Huawei, TCL, vivo, IDC, OPPO, ZTE, Intel, Sony, CMCC, Panasonic, MediaTek, xiaomi, Nokia, Samsung, FGI, Lenovo, DOCOMO, LG

    - Forward compatibility [11]

- A slot cannot consist of both SBFD and non-SBFD symbols

  - Supported by:

Whether a physical channel can be mapped to SBFD and non-SBFD symbols within a slot

If a slot can consist of both SBFD and non-SBFD symbols, it needs to be discussed whether a PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot according to previous agreement in RAN1#111.

Ericsson proposed to study the signal continuity issues and solutions, if any, for a physical channel that is carried in both non-SBFD and SBFD symbols within a slot considering that the gNB and possibly also the UE may change/activate/deactivate several hardware configurations/operations during the transition between non-SBFD and SBFD symbols.

Spreadtrum proposed not to discuss PUCCH/PUSCH/PDSCH/PDCCH mapped to SBFD and non-SBFD regions in the same slot in SI stage and think scheduled PUSCH/PDSCH/PUCCH without repetition mapped to SBFD and non-SBFD regions in the same slot can be avoided by gNB scheduling.

Intel thinks that if a physical channel can be mapped to SBFD and non-SBFD symbols, it would lead to different performance in different symbols which degrades the performance and complicates link adaptation and the overall transmission/reception procedure. The proposal is to disallow a physical channel/signal to be mapped to both SBFD and non-SBFD symbols. To be more specific, it is avoided by gNB scheduling for scheduled transmission/reception without repetition or multi-slot scheduling and for configured transmission/reception or scheduled transmission/reception with repetition or multi-slot scheduling, the transmission/receptions across different symbols types are dropped/postponed.

Qualcomm has the similar proposal as Intel that UE doesn’t expect to be dynamically scheduled with a physical channel that is mapped to SBFD and non-SBFD symbols within a slot and UE drops or discard physical channel transmission or reception configured by higher layer that is mapped to SBFD and non-SBFD symbols in a slot.

ZTE proposed that PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD symbols and non-SBFD symbols in the same slot if configured.

Vivo proposed that dynamically scheduled UL transmissions and DL receptions is allowed to be overlapped with both SBFD symbol(s) and non-SBFD symbol(s) in the same slot, where dynamic overriding can be considered further. Semi-statically configured UL transmissions and DL receptions is not allowed to be overlapped with both SBFD symbol(s) and non-SBFD symbol(s) in the same slot or it is allowed only if consistent transmission parameters (such as frequency domain resource, beam related configurations, etc.) across all symbols that a UL transmission/DL reception is mapped to are ensured.

Samsung thinks that symbol-level resolution by RRC for the SBFD time-domain configuration does not imply that SBFD operation across SBFD and non-SBFD symbol must be supported by the UE implementation. The possibility to allocate and map a PDSCH, PUSCH or PUCCH across both non-SBFD and SBFD symbols in an SBFD slot depends on the gNB-side SBFD antenna configuration. It is the baseline that a channel/signal is not mapped to different symbol types within the same slot. A channel/signal mapped to different symbol types within the same slot can be supported under the condition that the same QCL and EPRE assumptions can be applied by the UE for the allocation on the non-SBFD and SBFD symbols.

- A physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot

  - Supported by: ZTE, vivo (dynamic), Samsung (under certain condition)

- A physical channel/signal cannot be mapped to SBFD and non-SBFD symbols within a slot

  - Supported by: Spreadtrum, vivo (semi-static), OPPO, Intel, QC, Samsung (baseline), FGI, Apple (PUSCH)

      - Semi-static configuration of SBFD subbands

The following agreements were made in RAN1#110 to study semi-static configuration of subband time and frequency location as baseline.

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

        - Subband time location indication

The following agreements were made in RAN1#110bis-e for semi-static configuration of subband time locations for SBFD operation.

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Periodicity of SBFD subband time location

Two options are identified by companies for the periodicity of SBFD subband time location configuration as summarized in [11] and companies’ views are summarized below:

- Option 1: The period is determined by periodicity of the configured TDD pattern

  - Supported by: Huawei, vivo, OPPO, ZTE, Ericsson, Spreadtrum, CMCC, FGI, Sharp, Qualcomm

- Option 2: The period is determined by a new configured periodicity.

  - Supported by: vivo, ZTE,

        - Subband frequency location indication

The following agreements were made for semi-static configuration of subband frequency locations for SBFD operation.

Agreement (RAN1#110) For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Agreement (RAN1#110bis-e) For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement (RAN1#111) For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Explicit/implicit indication of subband(s)

There are basically two options for DL/UL subband(s) and guardband(s) configuration as illustrated below.

Figure 3-1: Options for RB set and guard band configuration in SBFD symbols [19]

- Option 1: Frequency locations of DL subband(s) and UL subband are explicitly configured. Guardband(s) if any are implicitly derived.

  - Supported by: vivo, TCL, New H3C, MediaTek, ZTE, Intel, Nokia, Samsung, FGI, DOCOMO, LG

- Option 2: Frequency locations of UL subband and number of RBs for guardband(s) if any are explicitly configured. Frequency locations of DL subband(s) are implicitly derived.

  - Supported by: vivo, OPPO,ZTE, IDC, Ericsson, Intel, Fujitsu, Nokia, Qualcomm, DOCOMO, CEWiT

Huawei proposed to indicate frequency locations of DL subband, flexible subband and guard band as follows considering introduction of flexible subband.

- DL subband should be explicitly indicated, at least for the length of DL subband;

- Flexible subband can be implicitly indicated as a set of consecutive of RBs which are not included in UL subband and DL subband;

- Guard band can be implicitly indicated based on scheduling, i.e., gNB will not schedule UL and DL transmission on guard band.

- The subband frequency-domain location is indicated by granularity of RB.

        - Signalling design of subband location indication

Companies discussed whether semi-static time and frequency locations of SBFD subbands are signalled via cell-common signalling or UE-specific signalling and companies’ views are summarized below.

- Cell-common signalling

  - Supported by: Spreadtrum, MediaTek (frequency and time location), ZTE, CATT, Ericsson (frequency location), xiaomi, Nokia, Samsung (if SBFD in RRC_IDLE/INACTIVE is supported), Sharp, Qualcomm, Fujitsu

- UE-specific signalling

  - Supported by: vivo, Spreadtrum (time location), MediaTek (time location), ZTE, Ericsson (frequency location), OPPO (time location), Panasonic, Nokia (can be considered as additional option), Samsung (baseline for SBFD in RRC_CONNECTED)

    - Different configurations may be expected by SBFD aware UEs and non-SBFD aware UEs [11]

    - More flexibility and extendibility [11]

    - For potential different guardband sizes for different UEs [19]

      - Interaction with TDD-UL-DL-ConfigDedicated and SFI

The following agreement was made in RAN1#111 meeting for SBFD operation in DL and flexible symbols configured in TDD-UL-DL-ConfigCommon.

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

The above agreements define UE behaviors for SBFD operation in a symbol configured as DL and flexible in TDD-UL-DL-ConfigCommon. Vivo, CMCC, DOCOMO, xiaomi discussed the interaction with TDD-UL-DL-ConfigDedicated and SFI.

Vivo and DOCOMO think are that a symbol configured as flexible in TDD-UL-DL-ConfigCommon can be further overridden as DL/UL by TDD-UL-DL-ConfigDedicated with UE specific RRC signaling as in existing specification.

  - If the symbol i is overridden as DL by TDD-UL-DL-ConfigDedicated, SBFD operation for semi-static DL symbol is performed.

  - If the symbol i is overridden as UL by TDD-UL-DL-ConfigDedicated, DOCOMO thinks that this case should be deprioritized while vivo thinks that the symbol i should be used as a UL-only symbol.

  - If the symbol i remains flexible with TDD-UL-DL-ConfigDedicated, vivo thinks that SBFD operation in flexible symbols is performed and DOCOMO thinks that further interaction with SFI should be considered.

For interaction with dynamic SFI, vivo observed that at least when the detected DCI format 2_0 indicates the semi-static flexible symbol configured with UL subband as DL, UL transmission(s) may be allowed within the UL subband in this symbol, which is not consistent with legacy UE behavior where only DL reception(s) are allowed in this symbol.

      - Dynamic SBFD

It was agreed in RAN1#112 to further study dynamic SBFD.

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

Companies who support allowing DL receptions outside semi-statically configured DL subband and allowing UL transmission outside semi-statically configured UL subband have the following observations/views:

- Better resource utilization depending on the UL/DL traffic ratio [8][11][16][18][20][21][24][27][35][36]

- Better forward compatibility with subband overlapping full-duplex systems [8]

- Better performance especially in UL compared to semi-static SBFD [11]

- For UE-UE CLI measurement outside DL subbands [8][16]

- For gNB-gNB CLI measurement, a potential approach is for the victim gNB to measure the RSRP of the aggressor gNB within the DL subband. Additionally, for coordinated beamforming using gNB-gNB channel measurements, the measurements should also be done in the DL subband. Therefore, UL transmission outside the UL subband should be allowed from UE point of view. [8]

- DL throughput and spectral efficiency losses of semi-static SBFD are expected to be alleviated [21][25]

- Allowing legacy behaviour for supporting TDD operation (e.g., L3 measurement, time/frequency tracking using contiguous CSI-RS resources and/or SSB) [36]

Companies who do not support allowing DL receptions outside DL subband and allowing UL transmission outside UL subband have the following observations/views:

- Complicated/Significant spec efforts [14][19]

- No tangible benefits compared to dynamic TDD [13][15][19]

- Allowing DL receptions outside the DL subband(s) (i.e., within the UL subband) defies the principle of configuring an UL subband [19]

Vivo, CATT, Ericsson provided simulation results comparing dynamic SBFD with semi-static SBFD/dynamic SBFD.

Vivo’s evaluation results show that at least for frame structure#3 (XXXXX) and FR1 InH, and for asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to semi-static SBFD, dynamic SBFD can achieve higher performance in both DL and UL, especially significant gain can be obtained in UL.

CATT observed the following:

- Compared with semi-static SBFD, dynamic SBFD outperforms in DL but degrades the performance in  UL;

- Compared with dynamic TDD, dynamic SBFD does not bring benefit in downlink. For uplink, dynamic SBFD brings benefit for cell edge users but introduces performance loss for cell center users.

Ericsson’s results show that dynamic SBFD options 2 and 3 do not offer a performance advantage compared to the considerably simpler Dynamic TDD (dTDD) system.

Companies’ views on the options agreed in the last meeting are summarized below.

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

        - Supported by: TCL, OPPO, Spreadtrum, CATT, Ericsson, Apple

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

        - Supported by: Huawei, New H3C, vivo, MediaTek, NEC, ZTE, Intel, Sony, Fujitsu, xiaomi, Nokia, Samsung, ETRI, CMCC, Panasonic, FGI, Lenovo, DOCOMO, LG, ITRI, ASUSTEK, WILUS

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

        - Supported by: OPPO, Spreadtrum, CATT, Ericsson, Apple

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

        - Supported by: ZTE, Samsung, xiaomi, Lenovo

          - Same mechanism for DL and flexible symbols [ZTE, Samsung, xiaomi]

          - UL transmission outside the semi-statically configured UL subbands requires hardware enhancement of the UE, such as the design of filters for the UL subband [ZTE]

          - UE design complications, e.g. FDRA/FH for PUSCH repetition then needs to determine dynamically if a legacy F symbol where a PUSCH transmission of a PUSCH repetition occurs has been scheduled as UL-only or has been scheduled as SBFD symbol. [Samsung]

        - Not precluded at this stage: MediaTek

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

        - Supported by: Huawei, TCL, New H3C,vivo, MediaTek, NEC, Intel, Sony, Fujitsu, Nokia, ETRI, CMCC, FGI, DOCOMO, ITRI, ASUSTEK, WILUS

For details on how to achieve DL receptions outside DL subband and UL transmission outside UL subband, the proposals can be categorized into the following options.

- Option 1: flexible subband

  - Supported by: Huawei

  - Not supported by: Intel, Samsung

  - Beneficial but may not be necessary: ZTE

- Option 2: scheduling DCI

  - Option 2-1: explicit indication in scheduling DCI

  - Option 2-2: implicit indication in scheduling DCI

    - E.g. if a PUSCH/PUCCH transmission is scheduled by the DCI in a UL subband in an SBFD slot, but the symbol of the PUSCH/PUCCH transmission exceeds the SBFD symbol of the UL subband in the SBFD slot, then the SBFD symbol of the UL subband can be automatically expanded to include the symbol of the PUSCH/PUCCH transmission, or if a PDSCH reception is scheduled in the UL subband of the SBFD symbol by a DCI, then the UL subband in the SBFD symbols where the PDSCH reception is located is dynamically cancelled [18]

  - Supported by: vivo, ZTE (Option 2-2?), Intel, xiaomi

    - Not applicable to configured channels/signals. [20]

    - Option 2-1 avoids confusion on FDRA for dynamic channels/signals compared with Option 3 in case of miss-detection [20]

    - UE may not know how to interpret the indicated frequency resource allocation for Option 2-2, e.g. UE should perform RM for PDSCH around UL subband and guardband(s) in SBFD symbols and does not perform RM in full DL symbols. [20]

- Option 3: group-common DCI, e.g. SFI, enhanced SFI, new DCI etc.

  - Supported by: vivo, ZTE, Intel, CMCC, Nokia, xiaomi, WILUS

    - Applicable to both dynamic scheduled and configured channels/signals. [20]

    - Miss-detection should be carefully considered. [20]

    - Common to all UEs to avoid simultaneous transmissions and receptions from different UEs on the same resources [24]

      - SBFD operation in SSB symbols

It was agreed in RAN1#110bis-e to study whether SBFD operation in SSB symbols is supported or not.

Agreement Study whether SBFD operation in SSB symbols is supported or not.

Companies’ views are summarized below:

SBFD operation in SSB symbols is

  - Supported: Huawei, IDC, Sony, Qualcomm, Samsung, ITRI

    - Degradation of SBFD performance due to limited symbols for SBFD operation if SBFD operation on SSB symbols are not supported especially when the number of SSB symbols is relatively large [8][12][20]

    - Allowing for the maximum possible number of symbols to be configured with an SBFD UL subband maximizes the theoretically achievable UL coverage gain when SBFD is introduced. [25]

    - Except the UEs which are under initial access or cell reselection, a short SSB measurement period is not necessary and these UEs can perform the SBFD operation on SSB symbols to improve the performance [8]

    - UE-UE CLI between two UEs occurs rarely [37]

  - Not supported: Spreadtrum, vivo, OPPO, MediaTek, NEC, Ericssion, Nokia, CMCC, DOCOMO (and CORESET#0), CEWiT, FGI, WILUS (restricting UL transmission in SSB symbols)

    - Avoid impact on SSB detection/measurement from UE-to-UE CLI [11][13][16][19][20][24][27][30]

    - UL transmissions may not be supported by UE given that UE cannot transmit UL and measure SSB simultaneously and gNB does not know when UE is receiving SSB [14]

    - Self-interference from SSB is not easy to be cancelled considering that SSB signaling is a broadcast signaling with high power [13]

    - Impact and potential enhancements for transmissions and receptions

      - PDSCH/PUSCH

        - RBG

The following agreement was made in RAN1#112.

Agreement For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used FFS: The part of the RBG outside.

Companies’ views/observations on the two options for PDSCH and PUSCH are summarized below:

  - Option 1:

    - Supported by: Huawei, TCL, IDC, Spreadtrum, CATT, MediaTek, NEC, Ericsson, Intel, Nokia, Samsung, CMCC, Sharp, Apple, Lenovo, Qualcomm, xiaomi, LG, WILUS

      - Similar as RBG overlapping with the edge of the active BWP in the current spec [8][16][19][24][25] [34]

      - In order to utilize the resource more efficiently [27]

      - Simplest way would be rate matching around the UL subband [20]

      - Support a new configurable RBG set size [25]

      - Support non-continuous PRB-to-CRB mapping of DL subbands on SBFD slots [27]

  - Option 2:

    - Supported by:

      - Degrade spectrum efficiency/resource waste [8][12][14][15][20][34]

In addition, LG proposed that single value of RBG size for PDSCH RA type 0 is used for both SBFD symbol and non-SBFD symbol. CATT observed that RBG size determined based on size of DL/UL BWP is the same as in current specification with potential large RBG size in small DL/UL subband(s) in SBFD symbols. RBG size determined based on size of DL/UL subband(s) can avoid large RBG size in small DL/UL subband(s) in SBFD symbols.

  - If the RBG size is applied for both SBFD symbols and non-SBFD symbols, the FDRA field size may increase;

  - If the RBG size is applied for SBFD symbols only, the following issues need to be considered.

    - How to determine FDRA field size

    - RBG size if a PDSCH/PUSCH transmission can be mapped to SBFD symbols and non-SBFD symbols

        - PRG

The following agreement was made in RAN1#112.

Agreement For SBFD-aware UEs, study at least the following issues for PDSCH: - PRG(s) with size of 2 and 4 that overlaps with subband boundary - Wideband precoder in case of non-contiguous DL subbands

PRG size of 2 and 4

Similar to partial RBGs, partial PRGs are supported in current NR specs. The precoding granularity can be 2 PRBs, 4 PRBs or wideband. For 2 or 4 PRBs granularity, the first and the last PRG can be a partial PRG, depending on the size and the starting point of the BWP. There is no restriction on the size of a partial PRG in the exiting NR specs. If the BWP starts on an odd index, then the first PRG could be 1 or 3 PRBs if the precoder granularity is not wideband.

Huawei, New H3C, CATT, MediaTek, Ericsson, Nokia, xiaomi, Lenovo and WILUS proposed that partial DL PRGs inside the DL subband caused by unaligned boundaries between PRG and SBFD subbands are supported for precoder granularity of 2 and 4 PRBs.

Qualcomm thinks that the benefit of having extra partial PRGs across the DL subband boundaries is minimal. For example, when PRG size is 2, at most 1 extra RB is utilized. On the other hand, this may introduce extra UE complexity in terms of special DMRS channel estimation for handling up to four partial PRGs. It is considered that the expected gains don’t motivate the extra complexity at the UE side and the scenario can be avoided by proper configuration.

Wideband precoder

According to current specification, if PRG is determined as wideband, the UE is not expected to be scheduled with non-contiguous PRBs. For SBFD operation with {DUD} subband pattern, it needs to be discussed whether non-contiguous frequency resources across two DL subbands can be scheduled/configured for PDSCH in this case.

Huawei, CATT, Intel proposed that PDSCH across two DL subbands can be allocated. Huawei and CATT considered that the PRG size can be reinterpreted as subband. Nokia and LG proposed that for Physical Resource Block (PRB) bundling in case of SBFD symbol, assume wideband precoder with non-contiguous DL subbands derived by excluding frequency resource outside DL subband(s).

Qualcomm thinks that relaxing the current restriction will introduce extra UE complexity, e.g. double complexity of channel estimation and associated parameter estimation and there proposed that when PRG is set to "wideband”, SBFD-aware UE should be configured with contiguous PRBs in one DL subband. MediaTek also proposed that wideband precoding cannot be applied when PDSCH allocation is across two (or more) non-contiguous DL subbands.

In addition, MediaTek pointed out that for selecting wideband precoding in dynamic PRG bundling the allocated RBs should occupy more than half the BWP. This threshold may not be met when RB allocation is across non-contiguous DL subbands and a new threshold must be defined for the minimum RB size needed to select wideband precoding for dynamic PRG bundling.

CMCC proposed to support non-continuous PRB-to-CRB mapping of DL subbands on SBFD slots to solve precoder granularity issues where PRG is based on continuous PRB index across two DL subbands. The same solution is proposed for PDSCH FDRA.

        - Enhanced RM

According to current specification, PDSCH is rate matched around resources not available for PDSCH, including SSB, P/SP NZP CSI-RS, LTE CRS, resource configured by rate matching patterns etc. However, a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

Several companies [Huawei, Spreadtrum, CATT, Ericsson, Intel, Qualcomm, WILUS] discussed enhanced rate matching which relax the above restriction to rate match around UL subband and guardband(s) if any, for non-contiguous resource allocation for RA type 1 across two DL subband(s), unaligned RBG and subband boundaries, PDSCH resource allocation across SBFD and non-SBFD symbols etc.

        - Other enhancements

CMCC proposed to support non-continuous PRB-to-CRB mapping of DL subbands on SBFD slots to allocate non-contiguous frequency resources across DL subbands and to solve precoder granularity issues.

Figure 3-2: Non-continuous PRB-to-CRB mapping of DL subbands [27]

MediaTek proposed to support interleaved mapping of odd and even VRBs over physical RBGs allocated by Type-0 FDRA.

Figure 3-3: PDSCH FDRA Type-0 with fractional RBG allocation at DL subband edges and VRB-interleaving [16]

Sony proposed a mirror image FDRA, where the DL Grant indicates a 1st set of RBs and a 2nd set of RBs is determined by reflecting the 1st set of RBs across the middle of the BWP.  The scheduled PDSCH occupies the 1st set and the 2nd sets of RBs.  The Mirror Image FDRA can be enabled or disabled in the DL Grant.

Figure 3-4: Mirror Image FDRA [21]

Sony proposed to support non-uniform MCS in a PDSCH and PUSCH so that RBs of a PDSCH/PUSCH that are closer to an adjacent subband uses more robust MCS compared to RBs that are further away from an adjacent subband.

      - CSI-RS

        - Non-contiguous CSI-RS resource allocation

The following agreement was made in RAN1#112.

Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Companies’ views/observations are summarized below:

  - Option 1: Two contiguous CSI-RS resources that are linked

      - Supported by: Samsung, MediaTek

        - limits the flexibility of CSI-RS configuration due to the limitation of total number of CSI-RS resources [8][23][27]

        - signification specification impact on signaling design [11]

        - large overhead [27]

        - less change to implementation/signaling [16]

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

      - Supported by: Huawei, Ericsson, Panasonic, Qualcomm, DOCOMO, xiaomi, Lenovo

        - More flexibility compared with Option 2-2 that CSI-RS frequency-domain resources can be allocated in any positions within DL subbands [8]

        - Flexibility [19]

        - Better support flexible subband [8]

        - Not applicable for periodic CSI-RS across SBFD and non-SBFD slots [19][27]

        - RRC impact [16]

        - Impact on measurement accuracy/CSI-RS sequence generation need to be studied [16]

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

      - Supported by: vivo, IDC, Spreadtrum, CATT, NEC, Ericsson, Intel, Nokia, Samsung, CMCC, Panasonic, Apple, Qualcomm, xiaomi, Lenovo, CSI-RS, WILUS

        - Existing signaling design can be reused [11][15]

        - Simple [11]

        - Backward compatible [11]

        - Impact on measurement accuracy/CSI-RS sequence generation need to be studied [16]

        - Applicable for periodic CSI-RS across SBFD and non-SBFD slots [14][19][27]

        - Can resolve the issue when the DL subband boundary is not aligned with the boundary of RB group for CSI-RS configuration with granularity of 4 RBs [15]

In addition, Qualcomm proposed to study relaxed UE CSI processing and reporting timeline as the UE needs to double the complexity of CSI processing to estimate the channel per each subband.

        - Unaligned boundaries between CSI-RS resource configuration and SBFD subbands

It was agreed to study unaligned boundaries between CSI-RS resource configuration and SBFD subbands.

Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

In current specification, the CSI-RS resource frequency resources are configured with granularity of 4 RBs. The boundaries of DL subbands may not be aligned with the CSI-RS 4RB frequency grid, as shown below, similar as the unaligned boundaries of BWP and CSI-RS 4RB frequency grid. For the partial 4RB CSI-RS resources at the edges of DL BWP, UE assumes that the actual CSI-RS includes RBs within DL BWP only.

Figure 3-5: CSI-RS frequency resource granularity in SBFD symbols [34]

Vivo thinks that the issue of unaligned boundaries between NZP CSI-RS resources and SBFD subbands may be negligible.

Qualcomm discussed several solutions. One solution is to drop only the PRBs outside the DL subband within the edge 4RBs CSI-RS when they are misaligned with the inner edge of the DL subband or to drop the whole 4RBs. Alternatively, the configuration of the CSI-RS frequency resources may be enhanced by finer granularity of RBs (e.g., 1 or 2RBs).

It is also mentioned by companies that Option 2-2 for non-contiguous CSI-RS resource allocation can be applied to resolve this issue.

      - CSI report

        - CSI report associated with P-/SP-CSI-RS across SBFD and non-SBFD slots

The following agreement was made in RAN1#112.

Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Companies’ views and observations are summarized below.

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

    - Supported by: Huawei, vivo, Intel, CATT, Nokia, Samsung (baseline), CMCC, Qualcomm, DOCOMO, WILUS

      - reuse existing specification at a high degree [11]

      - potentially with higher UE capability requirement [11]

      - larger measurement delay [11]

      - have problems with averaging over too long a time gap when determining CSI-RS periods that fall in SBFD symbols and CSI-RS periods that fall in non-SBFD symbols [19]

      - different CSI reporting can be directly used as reference for adaptive scheduling for PDSCH with single reception occasion or PDSCH with multiple reception occasions confined within one symbol type [20]

      - different CSI reporting subband can be configured [20]

      - SBFD symbols and non-SBFD symbols are subject to different interference types [24]

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

    - Supported by: vivo, Spreadtrum, Ericsson, Samsung

      - some new rules or UE behavior may be needed [11]

      - keep UE capability requirement [11]

      - more flexibility [11]

      - gNB could learn about potential CSI differences between SBFD and non-SBFD symbols using existing tools in the spec [19]

      - UE will average the measurement results on multiple measurement occasions which causes the report CSI is a combined result of SBFD symbols and non-SBFD symbols. [27]

      - gNB has to rely on time domain restriction to obtain CSI for SBFD or non-SBFD from last CSI-RS occasion [20]

MediaTek observed that for periodic/semi-persistent CSI reporting, the existing specification can support separate CSI reporting for SBFD symbols and non-SBFD symbols.

Xiaomi observed that both option 1 and option 2 can be supported via current specification.

        - Unaligned boundaries between CSI reporting configuration and SBFD subbands

It was agreed to study unaligned boundaries between CSI reporting configuration and SBFD subbands.

Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

For CSI reporting configuration, csi-ReportingBand in the form of bitmap can be configured to indicate a contiguous or non-contiguous subset of subbands in the DL BWP which CSI shall be reported for. Besides, subbandSize can be configured to indicate one out of two possible BWP-dependent values for the subband size. The subband size is designed as an integer number of PRG (2 or 4 RBs) as well as an integer number of the RBG to avoid misalignment.

As the downlink subband(s) may not fully align with the CSI subband grid at the boundaries between DL-SB and guardband or UL subband, there could be partial CSI subband at the edges of the downlink subband.

Figure 3-6: Partial CSI subband [34]

Vivo thinks that with proper configuration of subbandSize, as well as the number of PRBs for the DL BWP, the issue of unaligned boundaries between CSI reporting configuration and SBFD subbands is under gNB’s control, which can be mitigated or resolved by gNB’s implementation.

Qualcomm thinks that it may be beneficial for the UE to measure CSI in partial CSI subband especially when the subband size is large so it is proposed to support SBFD-aware UE partial CSI subband reporting at the edges of the DL subband(s) boundaries.

Intel discussed the following options for unaligned boundaries between CSI reporting subband and DL subband(s):

- Option 1: UE does not expect to be configured with a CSI reporting subband which overlaps with DL/UL subband boundary.

- Option 2: A CSI reporting subband based on existing mechanism can overlap with DL/UL subband boundary.  UE derives CSI report for the CSI subband based on available CSI-RS resource received by the UE.

- Option 3: Introduce new mechanism for CSI reporting subband configuration, e.g., gNB can configure starting and ending PRBs for CSI subband and gNB ensures the configured CSI subband boundary is aligned with DL/UL subband.

      - Tx/Rx across SBFD slots and non-SBFD slots

The following agreements were made in RAN1#112 for transmissions and receptions across SBFD slots and non-SBFD slots.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement: Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation

Companies’ views and observations are summarized below.

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

      - Supported by: Huawei (except PDCCH), Nokia (SPS PDSCH/CG PUSCH, PDCCH), Intel (Periodic/semi-persistent CSI-RS), Samsung, QC (SPS PDSCH/CG PUSCH)

        - less spec impacts/simpler compared with Option 2 considering that the channel and interference are quite different in different symbols types [8][11][27][34]

        - contradictory with the purpose of latency reduction of SBFD  [11][18][20][27][35]

        - supported inherently by gNB implementation [19]

        - limited coverage gain [34][35]

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

      - Supported by: Huawei (PDCCH), NEC, ZTE, vivo, IDC, OPPO (SPS PDSCH/CG PUSCH), Spreadtrum, Intel, CMCC, xiaomi, Nokia (PUSCH/PDSCH repetitions and TBoMS), Ericsson, Samsung, QC (PUCCH/PUSCH repetition), MTK (PDCCH, multi-slot UL transmissions), DOCOMO, WILUS

        - Enhancements (e.g. separate frequency resources, MCS, TBS, power control parameters, and beam parameters) are needed [8][11]

FDRA across SBFD and non-SBFD slots

If Option 2 is agreed, the frequency resource may fully or partially overlap with RBs outside SBFD subbands which are invalid.

For PDSCH, CSI-RS, PUSCH, PUCCH, SRS, the proposals from companies can be summarized as below:

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

  - Option 1-2: Single FDRA configuration/indication for SBFD slots and non-SBFD slots

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

If search space is configured for SBFD and non-SBFD symbols and the CORESET overlaps with RBs outside DL subband(s) in some SBFD symbols, the following potential solutions are discussed by companies:

- Option 1 (CORESET adaptation): Adapting the valid resources for the CORESET in SBFD symbols/slots and remapping the CCE-to-REG for the CORESET accordingly.

- Option 2 (PDCCH rate matching/puncturing): Performing rate matching or puncturing on the REG(s) of a PDCCH which fall(s) outside the DL subband(s).

- Option 3 (PDCCH dropping): drop PDCCH candidates which REGs falls outside DL subband.

Frequency hopping

In case of frequency hopping, without enhancements, the UL transmission hop may be mapped to the resource out of the UL subband in SBFD slot and/or the resources in full-UL slot may cause UL resource fragmentation. The following solutions are proposed by companies:

- Option 1: Separate frequency hopping parameters are configured for UL transmission in SBFD and non-SBFD slots

  - Supported by: vivo, OPPO, Huawei, ZTE, Samsung, QC

- Option 2: Different reference RB/BW for FH in different slot types

  - Supported by: MediaTek, Ericsson, WILUS

UL power control

On whether to have separate UL power control parameters for SBFD slots and non-SBFD slots, vivo thinks that it depends on handling strategies of interferences, such as self-interference, inter-UE CLI, etc. ZTE thinks that independent power control should be used for UL transmission in the UL subband and in the UL BWP, respectively since UL transmission is subjected to interference of different intensities. However, shared power control should also be supported for both UL transmissions if the interference is within the control capability of the base station or within a controllable range. Therefore, ZTE proposed that for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, UL power control parameters can be shared or configured separately.

Ericsson and Intel proposed to further study separate UL power control in SBFD vs. non-SBFD symbols. In addition, Ericsson proposed to study how to signal this to the UE, e.g., by different configured power control parameters and/or different TPC commands. TCL, OPPO, Samsung, CMCC and LG support separate UL power control parameters in SBFD and non-SBFD slots. In addition, Samsung supports separately configured maximum UE output power.

Beam/spatial relation

On whether to have separate beam/spatial relation for SBFD slots and non-SBFD slots, vivo thinks that it depends on antenna settings for different symbol types, e.g. separate antennas for UL receptions and DL transmissions at gNB side, as well as handling strategies of interferences. Similar as UL power control, ZTE proposed that for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, beam/spatial relation can be shared or configured separately. Qualcomm proposed to study potential enhancement on beam configuration for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, e.g. different QCL type D for UL transmissions and DL receptions. Intel proposed to study separate beam/spatial relation for UL transmissions in SBFD and non-SBFD symbols. Samsung and LG support separate beam/spatial SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots. CMCC does not think it is so necessary to configure separate UL beam/spatial relation, since the difference between UL symbols and SBFD symbols is the available UL resource and the number of Rx elements. Although the Rx panels may be different between SBFD symbols and non-SBFD symbols, e.g., only half of the panels is used in SBFD symbols, the relative locations between gNB and UE are not changed and the same UL beam/spatial relation can be used for UE.

Similar as for UL, Qualcomm proposed to study the following for PDSCH and CSI-RS on SBFD symbols and non-SBFD symbols in different slots:

- Whether/how to have separate resources

- Whether/how to have separate time and frequency resources within the resource

- Whether/how to have separate beams/TCI states

      - Others

Huawei and Intel proposed to study potential enhancements for UCI multiplexing on SBFD symbols considering that UCI multiplexing on SBFD symbols has less reliability than UCI multiplexing on non-SBFD symbols.

Huawei and OPPO proposed to study the enhancement for the TBS determination for SBFD operation when the PDSCH overlaps with UL subband and the PRBs within the UL subband are not available for PDSCH.

CMCC proposed to study for a single DCI format used to schedule PDSCH/PUSCH in SBFD symbol/slot or non-SBFD symbol/slot, whether the same DCI size or different DCI sizes are used for scheduling SBFD symbol/slot and non-SBFD symbol/slot respectively

MediaTek proposed to study the feasibility of enabling two MCSs based on slot type to enable flexibility in MCS selection.

DOCOMO proposed to study SBFD impact on PUSCH repetition type B segmentation and SPS HARQ-ACK deferring.

    - UE collision handling

The following agreement was made in RAN1#110bis-e.

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

There are two types of collision between UL and DL for a SBFD aware UE as below.

- Type A: Collision between UL transmissions and DL receptions in the same SBFD symbol

- Type B: Collision between transmissions/receptions with transmission direction of subbands

For Type A collision, the following cases are discussed by companies.

- Collision between dynamic UL transmissions and dynamic DL receptions

  - IDC, ZTE, xiaomi (w/ different priorities),  DOCOMO (w repetition), ITRI (with different priorities)

    - Option 1: For same priority, UL transmission in UL subband is prioritized over DL reception in DL subband, and DL reception in UL subband is prioritized over UL transmission in UL subband; for different priorities, DL/UL transmission with higher priority is prioritized [ZTE]

    - Option 2: based on rules [DOCOMO]

    - Option 3: Consider configuration of physical channel priorities for handling the collisions in SBFD symbols [IDC]

  - Error case: vivo, Spreadtrum, CATT, Intel, CMCC, DOCOMO (w/o repetition), WILUS

- Collision between dynamic DL receptions and semi-statically configured UL transmissions

  - vivo, IDC, OPPO, CATT, NEC, ZTE, xiaomi, Intel, CMCC, ETRI, MTK, CEWiT, Apple, DOCOMO, Lenovo, ITRI, WILUS

    - Option 1: dynamic transmission/reception is prioritized over semi-static transmission/reception  [ZTE, Apple, CMCC, MTK, CEWiT]

- Collision between dynamic UL transmissions and semi-statically configured DL receptions

  - vivo, IDC, OPPO, CATT, NEC, ZTE, xiaomi, Intel, CMCC, ETRI, MTK, CEWiT, Apple, DOCOMO, Lenovo, ITRI, WILUS

    - Option 1: dynamic transmission/reception is prioritized over semi-static transmission/reception  [ZTE, Apple, CMCC, MTK, CEWiT]

- Collision between semi-statically configured UL transmissions and semi-statically configured DL receptions

  - New H3C, vivo, ZTE, CMCC, CATT, Intel, DOCOMO, Nokia, Apple, QC, xiaomi, Lenovo, WILUS

    - Option 1: Additional signalling is provided to indicate the transmission/reception direction [CMCC (explicit indication of link direction)]

    - Option 2: For same priority, UL transmission in UL subband is prioritized over DL reception in DL subband, and DL reception in UL subband is prioritized over UL transmission in UL subband; for different priorities, DL/UL transmission with higher priority is prioritized [ZTE]

    - Option 3: UL is prioritized [Apple]

    - Option 4:Based on priority (support priority for DL) [Nokia]

    - Option 5: based on rules [DOCOMO]

UE transmission direction determination

CMCC’s views are the UL/DL direction in symbols/slots overlapped with (enabled) SBFD subband in time-domain can follow the following two steps from UE’s perspective.

- Step 1 (semi-static/dynamic UL subband time location indication): The semi-static/dynamic UL subband time location indication signalling is used to configure the UL subband time location (SBFD symbol time location). This signalling can be cell-specific and the periodicity can be the same as the TDD periodicity configured in cell-specific TDD UL/DL configuration. This signalling can “override” the legacy UL/DL direction determining rule related to cell-specific TDD UL/DL configuration. For example, for a given SBFD symbol which is indicated as DL by cell-specific TDD UL/DL configuration, SBFD aware UE can determine it UL or DL based on the other indications as discussed in Step 2.

- Step 2 (UL/DL direction indication): For a given SBFD symbol, some legacy signalling e.g., SFI, scheduling DCI or new signalling, e.g., new DCI format can be reused to indicate the UL or DL direction. This function is similar to traditional TDD config dedicated RRC signalling/SFI/ scheduling DCI to override the flexible slots/symbols in while the only change is to use the UL/DL direction indication to override SBFD slots/symbols.

Figure 3-7: UL/DL direction determining in SBFD symbol/slot [27]

Spreadtrum and MediaTek proposed to discuss the following options for transmission direction indication in SBFD symbols

•	Option 1: Transmission direction is preconfigured by higher layer signalling

•	Option 2: Transmission direction is dynamically determined based on the scheduling DCI

    - SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

Ericsson does not support misaligned center frequencies between DL and UL BWPs as it doesn't seem to bring any benefits and would likely negatively impact UE complexity.

Qualcomm see useful scenario to support a single configured DL and UL BWP pair with unaligned center frequencies where network configures the UE with narrowband initial UL/DL BWP and then wideband first active BWP after RRC connection. If SBFD is indicated to only RRC connected UE, then during initial access, all UEs will be configured with center aligned narrowband UL/DL BWP as shown on left figure in the example below. This will restrict leveraging the UL-SB resources and hence don’t achieve UL coverage gain and latency reduction. On the other hand, if this restriction is lifted, two narrowband UL/DL BWP could be configured for the new UEs as shown in the right figure in the example below. This can be applicable for default BWP as well where UE could be scheduled only in one of the two DL subbands.

Figure 3-8: Narrowband UL/DL BWP with aligned and non-aligned center frequency [34]

    - SBFD operation for initial access

It was agreed to study SBFD operation at least for RRC_CONNECTED state.

Huawei, InterDigital, MediaTek, ZTE, CATT, NEC, Intel, Nokia, Samsung, LG, Qualcomm, DOCOMO observed that SBFD operation for initial access may offer the following potential benefits:

  - Increased RACH capacity [8][12][15][16][18][20][24][34]

  - Reduced initial access latency [8][12][15][16][18][20][24][25][34][35]

  - Improved UL coverage for Msg 3 and PRACH due to more UL resources [8][20][24][34][36]

  - Avoid UL resource fragmentation [25]

  - Reduced CLI if PRACH resource is not used by UE [25]

Therefore, the above companies proposed to study potential enhancement of initial access enhancement for SBFD operation.

Ericsson proposed that UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation.

    - SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies

InterDigital, ZTE, CEWiT proposed to study/support BWP-based solution.

Ericsson does not support misaligned center frequencies between DL and UL BWPs as it doesn't seem to bring any benefits and would likely negatively impact UE complexity.

Qualcomm observed that for SBFD operation using more than one configured DL and UL BWP pair where each subband configured as BWP, it requires a lot of specification impacts and complicate UE behavior.

- Requires two active BWPs at a time, each configured with its own DL/UL TDD pattern.

- Requires cross-BWP scheduling and cross-BWP HARQ feedback.

- Complicates BWP switching mechanism

- Requires non-aligned BWP center frequency

- Requires some restriction rules may be needed to have common parameters for both DL BWPs

- RRC signalling overhead

For SBFD operation using more than one configured DL and UL BWP pair where one BWP pair is configured for TDD operation and the pair is configured for SBFD operation, it requires less specification impact mainly to enhance the BWP framework (e.g. non-contiguous RB for BWP, non-aligned UL/DL center freq.)

- One UL/DL BWP pair is active at a time

- Semi-static configuration of BWP switching pattern to reduce BWP switching delay.

- Enable UE to enhance BWP-based selectivity/filtering to reduce inter-UE CLI.

Intel suggested to not further consider BWP-based SBFD in Rel-18 SI phase considering BWP-based SBFD may not work for a UE incapable of multiple BWPs and limited time for any enhancement for BWP to enable efficient SBFD operation based on BWP, e.g.

  - BWP switching delay enhancement

  - DL control overhead and latency reduction due to BWP switching (SPS PDSCH/type-2 CG PUSCH of previous BWP is released and SPS PDSCH/type-2 CG PUSCH in the new active BWP is usable only after new activation DCI is received)

  - DL performance improvement due to HARQ-ACK dropping (BWP switching leads to HARQ-ACK dropping for PDSCHs in previous BWP)

  - Repetition across different BWPs for coverage improvement

Xiaomi proposed that BWP-based SBFD operation is not supported in Rel-18 duplex enhancement.

    - SBFD operation across carriers

It was agreed in RAN1#109-e to at least study SBFD operation within a TDD carrier.

Agreement At least study SBFD operation within a TDD carrier

SBFD operation across carriers achieved by intra-band TDD CA with different TDD configurations is considered by Huawei as illustrated below. Huawei thinks that the support of SBFD operation across carriers does not seem to require any specific standardization effort compared to the support of SBFD operation within a carrier. It is proposed to study potential enhancements to collision handling for intra-band TDD CA with different TDD configurations to enable SBFD operation across carriers.

Figure 3-9: SBFD operation across carriers [8]

Qualcomm observed that SBFD operation across multiple CCs requires UE supports of CA as prerequisite while CA framework has some inherent UE complexity and compared to single-CC SBFD, CA-based SBFD has some limitation where DL and UL BW is restricted to the component carrier bandwidth while the inter-channel guardband can’t be utilized. While it is considered that CA-based SBFD operation is interesting for e.g. FR 2-1, it is proposed that SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier.

Intel proposed to not consider CA-based SBFD in Rel-18 SI due to the following reasons:

  - the rule to determine a reference cell and the prioritization between reference cell and other cells when collision happens may not be sufficient for SBFD operation

  - CA based SBFD operation suffers retransmission restriction in the same carrier

  - repetition transmission across carriers is not supported

  - CA is an optional UE feature

Xiaomi proposed that half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement.

    - SBFD operation in legacy UL sybmol

The following conclusion was made in RAN#96.

Conclusion: UL symbol as second priority is accepted, no intended suspension of continuation of work in WGs

Ericsson proposed to not support SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon with the following rationale:

- The premise for SBFD operation is that it enables improved UL coverage and/or latency by enabling more UL opportunities compared to the baseline time domain TDD pattern. Hence, it is not motivated to configure symbols configured as 'U' by TDD-UL-DL-ConfigCommon for SBFD operation, e.g., with frequency domain pattern U-D-U.

- SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon can create strong gNB-gNB cross-link interference to a legacy (static TDD) gNB configured with the same baseline time domain TDD pattern, hence such a configuration should be avoided.

Qualcomm observed that SBFD operation at legacy UL slot is beneficial in multiple deployment scenarios, e.g., greenfield deployment and UL heavy deployment (InH/InF) to reduce DL blockage and improve DL coverage.

Xiaomi proposed that for subband non-overlapping full duplex, it cannot be applied to UL symbols.

  - [Open] 1st round discussion

Proposal 1-1

Proposed Agreement:

Endorse the text proposal in R1-2303639.

|  | Company |
| --- | --- |
| Support | New H3C with clarification, TCL (add FFS in partial RBG related text), ZTE, xiaomi, IDC, Samsung, Intel, vivo,CMCC, Huawei, HiSilicon,OPPO, LG (with modification), Sharp, Ericsson (depending on if comments/questions are addressed), Panasonic, Qualcomm, WILUS, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| New H3C | Regarding section 6.1.1.2, proposed text is as follows: In addition, whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon for SBFD aware UEs are studied based on the following options: - Option 1 (semi-static SBFD): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed We suggest to delete “dynamic SBFD” in option 2 because agreement in RAN1#112 doesn’t mention the wording “dynamic SBFD”. |
|  | The following text of section 6.1.2 miss the FFS part as given in RAN1#112 agreement. For SBFD-aware UEs, at least the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands are studied. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used Agreement For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used FFS: The part of the RBG outside.. |
| Sony | It is strange that we put in all the options that has already been ruled out, for example in Section 6.1.1, we still have these alternatives where we already agreed to go for Alt-4: - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. I think the TR is a summary of the outcome of the Study rather than a compilation of a list of previous agreements. For example for the above text, we could say in the TR (instead of listing all the alternatives in previous agreement): The SI focused on SBFD time and frequency location, that are known to SBFD aware UEs. |
| LG Electronics | Generally fine with the proposal. Wondering to capture all agreements with the ‘FFS’ and ‘whether/how’ in the draft TR. Suggest to delete the sentences with ‘FFS’ and ‘whether/how’. |
| Ericsson | Agree with comment from Sony Also, in Section 6.1.1.3, Option 1 and Option 2 are reused twice which can create some confusion. Question: out of all the agreements we made so far, what rationale was used for deciding which agreements to list in the body of TR? |
| QC | One minor comment regarding SBFD operation in SSB symbols. The current wording may give the impression that RAN1 has already done the study. Suggest the wording below till RAN1 decide on this issue. Whether SBFD operation in SSB symbols is supported or not is being studied in RAN1. |
| Nokia, NSB | We are OK for the text. Considering concerns of some companies, we think we can keep the options in the text if only we provide clear summary/outcome of the discussion in SI phase. |

Proposal 1-2

Proposed Agreement:

It is beneficial that a slot can consist of both SBFD and non-SBFD symbols at least for compatibility with symbol-level TDD UL/DL configuration and use of non-SSB symbols for SBFD operation if SBFD operation in SSB symbols is not supported.

Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition.

  - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot and/or a TDD UL/DL pattern period are needed

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, xiaomi, IDC, Samsung, Intel (suggestion for wording revision), Spreadtrum(without SSB part), vivo, CMCC, Huawei, HiSilicon,OPPO, LG Electronics, Sharp, Panasonic, Lenovo, Qualcomm, NEC, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Ericsson (too early to agree before other aspects studied) |

| Company | Comments |
| --- | --- |
| Moderator | Based on companies’ inputs, majority company support that a slot can consist of both SBFD and non-SBFD symbols. Meanwhile, concerns on frequent switching between SBFD and non-SBFD symbols have been raised considering the potential implementation impact and interruption of transmissions/receptions during the transition. |
| ZTE | Overall, we support this proposal. From our perspective, the main motivation for supporting symbol-level SBFD configuration is for compatibility with symbol-level TDD UL/DL configuration. |
| Xiaomi | The first sentence is an observation instead of a proposed agreement. The following modification is proposed from our side: Proposed Agreement: It is beneficial that a A slot can consist of both SBFD and non-SBFD symbols at least for compatibility with symbol-level TDD UL/DL configuration and use of non-SSB symbols for SBFD operation if SBFD operation in SSB symbols is not supported. - Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot and/or a TDD UL/DL pattern period are needed |
| Sony | We support SBFD & non-SBFD symbols in a slot. However, I think it is good to separate the discussion of SSB in SBFD symbols. That is we can agree on the following: It is beneficial that a slot can consist of both SBFD and non-SBFD symbols at least for compatibility with symbol-level TDD UL/DL configuration. Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot and/or a TDD UL/DL pattern period are needed |
| IDC | Support the proposal and the modification by Sony. |
| Intel | For 1st paragraph, we agree that symbol-level SBFD is compatibility with symbol-level TDD UL/DL configuration, and can fully utilize symbols other than the symbol not allowed for SBFD operation in a slot. SSB symbol is one example which may not allow SBFD operation, but also other cases under study, e.g., symbol containing CORESET 0. Therefore, more general description rather than only mention non-SSB symbol is better, or at least the non-SSB/SSB case should be captured as an example. For 2nd paragraph, we agree the number of switch between SBFD and non-SBFD symbols should be limited, but for the sub-bullet, considering we did not agree the SBFD configuration period is same as TDD UL/DL pattern period, it is better to use more general wording, e.g., semi-static SBFD configuration period. |
| Spreadtrum | For SSB slot, SSB spans four symbols or eight symbols, if this SSB slot consists of both SBFD and non-SBFD symbols, at least two switching points would be in a slot. It is too early to list SSB as an aspect for SBFD and non-SBFD switching. A safer change is to only list TDD UL/DL configuration. |
| vivo | We share similar comments as Intel, another use case observed by many companies is to configure SBFD operation in the DL symbols in the DL heavy special slot. |
| CMCC | We support to add the SSB issue since it is one key use case to allow a slot consists both SBFD symbol and non-SBFD symbol. In addition, this sentence doesn’t hurt the discussion of SBFD operation on SSB, because it also says “if SBFD operation in SSB symbols is not supported” |
| LG Electronics | Fine with the proposal. |
| Ericsson | We think it is too early to agree that (a) slot can consist of SBFD and non-SBFD symbols, or (b) that it is beneficial to support. We also agree with other companies’ views that SSB aspect should be treated separately. We think an important issue that needs to be addressed first is the following related to how the slot will be used if a switch between SBFD and non-SBFD is allowed within a slot (see also Proposal 3): If SSB and non-SBFD symbols are allowed in the same slots, then signal continuity and/or QCL issues need to be further analyzed. For instance, the following cases need to be treated differently. - The slot starts with a few non-SBFD DL symbols carrying only PDCCH. PDSCHs are only transmitted in DL subbands during SBFD DL symbols. Similarly, a PUSCH or PUCCH is transmitted only during one type of symbols. - The PDCCH or PDSCH are transmitted during both non-SBFD and SBFD symbols. Or a PUSCH or PUCCH is transmitted during both non-SBFD and SBFD symbols. In parallel, how many switches between SBFD and non-SBFD symbols in a TDD cycle needs to be analyzed before deciding that SBFD and non-SBFD symbols in the same slot can be supported. |
| Lenovo | We support a slot can consist of both SBFD and non-SBFD symbols For a slot with DL/flexible/UL symbols, the UL symbols are non-SBFD symbols, while the DL and flexible symbols can be SBFD symbols. Hence, symbol-level SBFD configuration should be supported. |
| QC | We are generally fine with the proposal. Regarding the switching between SBFD and non-SBFD symbols, some guard period or transition time may be needed. For example, this time is needed for the gNB to adapt or tune any subband analog/digital filters, digital IC or to switch panels from Tx mode to Rx mode as the gNB may use one panel for UL reception in SBFD symbols while using two panels in the UL symbols. From UE perspective, SBFD-aware UE may need to do filter retuning and UL sampling rate adjustment from SBFD to UL-SB. For the other example of switching from DL to SBFD, SBFD-ware UE may adjust DL filtering between DL symbol and SBFD symbols which interrupts DL reception. Also, suggest removing the wording ‘and use of non-SSB symbols for SBFD operation if SBFD operation in SSB symbols is not supported’ as RAN1 is still discussing whether SSB symbols can be configured for SBFD operation or not. Proposed Agreement: It is beneficial that a slot can consist of both SBFD and non-SBFD symbols at least for compatibility with symbol-level TDD UL/DL configuration and use of non-SSB symbols for SBFD operation if SBFD operation in SSB symbols is not supported. Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot and/or a TDD UL/DL pattern period are needed - Further study whether a guard period or transition between SBFD and non-SBFD symbols and the length of the guard period and whether there are any scenarios that don’t require guard period |
| NEC | We are okay with the proposal however we would also need to study the need for gap between SBFD and non-SBFD symbols to allow for RF reconfigurations by gNB/UE as mentioned by Qualcomm. |
| Fujitsu | One more concern point on frequent transition between SBFD and non-SBFD symbols is to increase the resources wasting by using the guard symbols. |
| Moderator | Xiaomi proposed to directly agree that a slot can consist of both SBFD and non-SBFD symbols while Ericsson thinks it is too early to agree that (a) slot can consist of SBFD and non-SBFD symbols, or (b) that it is beneficial to support. Several companies [Sony, IDC, Spreadtrum, Ericsson, QC] proposed to remove the SSB part which is separately discussed while CMCC support to add the SSB case. Intel proposed to change “TDD UL/DL pattern period” to “semi-static SBFD configuration period” given that it has not been agreed that the SBFD configuration period is same as TDD UL/DL pattern period. Qualcomm and NEC proposed to study guard period or transition between SBFD and non-SBFD symbols. Based on above comments, the proposal is updated as follows: Proposed Agreement: It is beneficial that a slot can consist of both SBFD and non-SBFD symbols at least for compatibility with symbol-level TDD UL/DL configuration and use of non-SSB symbols for SBFD operation if SBFD operation in SSB symbols is not supported. Frequent switching between SBFD and non-SBFD symbols is not expected considering that it may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, and/or a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required |
| Nokia, NSB | We support the agreement, with the change proposed by Xiaomi. For SSB symbol, we think SBFD operation in SSB symbols is not supported. Further study on number of switching points should be needed to similar as TDD. |
| Ericsson 2 | We disagree with the wording “It is beneficial” given that there are open study items listed in this proposal (and Proposal 1-3) that have not yet been addressed. If the intention of the proposal is to capture status of the study so far, then alternative wording could be: It is beneficial that It is under study on whether or not a slot can consist of both SBFD and non-SBFD symbols, and one motivation for allowing it is at least for compatibility with symbol-level TDD UL/DL configuration |
| Apple | We don’t support the first part of proposal (a slot can consist of both SBFD and non-SBFD symbols). Required time gap to switch between SBFD and legacy symbols may be more than a slot. For example, not all DL UEs are capable to receive in both DL sub-bands where there is an UL sub-band, possibly making a big CLI, in the middle. For such a victim UE, tuning to one of DL subbands can be a solutions which effectively means a DL BWP switch (which generally speaking cannot be fit within a slot). |

Proposal 1-3

Proposed Agreement:

Study whether a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot including:

  - Phase continuity

  - Potential interruption of transmissions/receptions during transition

  - Potential impact on performance, link adaptation, procedures

.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, Samsung, Intel (clarification), vivo (Clarification), CMCC,OPPO, Sharp (w/ modification), Ericsson (with modifications) , Lenovo, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB (with modification) |
| Not support | Xiaomi, Spreadtrum, NEC |

| Company | Comments |
| --- | --- |
| ZTE | We understand the intention of this proposal. However, it seems the wording “physical channel/signal” itself may be confusing, e.g., whether the above proposal is applied to a PDSCH with 2 repetitions? We propose the following wording change: Study whether a physical channel/signal transmission occasion can be mapped to SBFD and non-SBFD symbols within a slot including: - Phase continuity - Potential interruption of transmissions/receptions during transition - Potential impact on performance, link adaptation, procedures |
| xiaomi | We don’t see the necessity and benefits to support such kind of scheduling. Flexibility is not an issue as gNB can schedule anything with any strategies it wants. That is, dynamic scheduling or RRC signaling can easily avoid the case that a transmission or reception across SBFD symbol and non-SBFD symbol. On the other hand, if transmission/reception across SBFD symbol and non-SBFD symbol within a slot is allowed, it means UE and gNB has to process a single TB with different assumptions. The performance cannot be guaranteed and the complexity goes up. |
| Sony | I think there are two scenarios where a transmission overlaps SBFD & non-SBFD symbols: - A DL transmission overlaps SBFD and non-SBFD symbols but the transmission is always in RBs configured as DL, i.e. it is in DL subband whilst in SBFD symbol and in DL only symbol whilst in non-SBFD symbol. Similarly, a UL transmission overlaps SBFD & non-SBFD symbols, where it is in UL subband whilst in SBFD symbols and in UL-only symbol whilst in non-SBFD symbol. Here there is no issue with phase discontinuity and I don’t think there is any interruption at the UE since it still transmitting/receiving in the same RBs across SBFD & non-SBFD symbols. - A DL transmission overlaps SBFD & non-SBFD symbols where the transmission is interrupted by UL symbols (whilst in non-SBFD symbol) or a UL transmission overlaps SBFD & non-SBFD symbols where the transmission is interrupted by DL symbols (whilst in non-SBFD symbol). We think that at least scenario 1 can be supported. Or perhaps we can clarify whether this proposal concerns scenario 2 where the transmission is interrupted by a symbol of the opposite link direction? |
| IDC | Support the proposal. In our opinion another aspect should be considered in the proposal. That is the cases where the DL resources being scheduled in non-SBFD symbols overlap with UL SBs in SBFD symbols. Whether/how the DL reception overlapped with UL SBs in SBFD symbols should be dropped or rate-matched? Or a different offset in DL SBs in SBFD symbols should be used instead of dropping? |
| Intel | Agree with ZTE, the wording ‘a physical channel/signal’ itself may be misleading. We suggest revision on top of ZTE’s version as below. Study whether a physical channel/signal transmission or reception occasion Besides, we’d like to clarify, whether the proposal covers some or all the cases listed below? - For resource configuration, an occasion can be mapped to both SBFD and non-SBFD symbol, but UE cannot transmit or receive such occasion. - For resource configuration, an occasion cannot be mapped to both SBFD and non-SBFD symbol. For UE transmission or reception, UE can transmit or receive such occasion |
| Spreadtrum | We do not support a physical channel/signal can be mapped to SBFD and non-SBFD symbols due to the following reasons. - First, timing advance for CLI handling in SBFD operation - Second, DL-UL switching is another issue - Third, some enhancements were agreed for SBFD regions, such as different frequency resource, time domain resource, power control, and spatial domain. |
| vivo | Good to clarify whether this proposal targets the both scenarios as Sony mentioned. We are fine with ZTE and Intel’s revision on the ‘‘a physical channel/signal’’. In addition, the study should also consider the operation compatibility with legacy UE or non-SBFD aware UE. |
| CMCC | We support the proposal and also fine with ZTE’s clarification. Regarding the scenario 1 in Sony’s comment, even the RB is always restricted in DL subband or UL subband, the other parameters can be different between SBFD symbol and non-SBFD symbol, e.g., UL power control, beam/spatial domain, and in this case the interruption of transmissions/receptions during transition may also occur. |
| Huawei, HiSilicon | Although we understand the proposal is to study the performance and impact to map a physical channels/signals to both SBFD and non-SBFD symbols within a slot. We do have a concern to support such operation such this will complicate the system design in quite some aspects such as potential transmissions/receptions interruption across SBFD and non-SBFD symbols due to the antenna switching, potential channel and interferences differences across SBFD and non-SBFD symbols, etc. |
| LG Electronics | We don’t see the benefit that a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot. Also it brings implementation complexity. The proposal seems unclear whether the examples are listed up from the UE perspectives or gNB perspectives. Also, in the first example, the other things needs to be included. - Phase continuity, power consistency, transmission/reception timing |
| Sharp | We have similar understanding as ZTE. “Channel” is ambiguous in the context of repetition. “a physical channel/signal” should be replaced by “transmission occasion”. |
| Ericsson | Support, with some modifications. Comment: the 3rd bullet seems to be incomplete. Suggest splitting performance and procedures aspects. Study whether a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot including: - Phase continuity - Potential interruption of transmissions/receptions during transition - Required guard time - Potential impact on performance - Impact on link adaptation and channel estimation procedures |
| Lenovo | Support the proposal with the “transmission or reception occasion” clarification. In our view, the study also includes whether the frequency domain resources of the physical channel/signal transmission or reception occasion can be determined separately for SBFD and non-SBFD symbols. |
| QC | We believe there are no benefits for mapping a physical channel across a combination of SBFD and non-SBFD symbols in a slot and should be avoided. In addition, it has some many complications on both UE and gNB side. Also, it is not guaranteed that QCL assumption, power, phase, timing can be maintained across both SBFD and non-SBFD symbols types. And if there is guard gap needed for switching, then a physical channel can’t be allocoated across the SBFD and non-SBFD symbol. So, we don’t think it is feasible to assume such operation and should be avoided. For dynamically scheduled channel, e.g., PDSCH or PUSCH, gNB can easily avoid this. However, for RRC-configured transmission and reception, the periodicity of the transmission or reception occasion may not well align with the slot periodicity and some occasion may map to combination of SBFD and non-SBFD symbols. In that case, the UE should discard the transmission or reception in that occasion. |
| NEC | We don’t understand the motivation to study this. A physical transmission containing both SBFD and non-SBFD symbols just increases the complexity without providing any apparent advantage. It should be up to gNB scheduling implementation to make sure that a physical channel transmission should consist of only SBFD or non-SBFD symbols but not both. |
| CEWiT | We support ZTE’s modification of the proposal. |
| Moderator | Several companies proposed to clarify the wording “physical channel/signal”. For the two scenarios mentioned by Sony, the intention is to cover both. Even for scenario 1, as commented by other companies, power, beam, interference etc. can be different in different symbols even if the RBs are the same. According to Intel’s question, the proposal is updated to clarify that the study includes both cases, i.e. whether a Tx/Rx occasion can map to different symbol types and whether UE can transmit/receive in such occasion. Ericsson proposed to add some FFS points Based on above comments, the proposal is updated as follows. Proposed Agreement: Study whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot, and whether UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including: - Phase continuity - Potential interruption of transmissions/receptions during transition - Required guard time - Potential impact on performance - Impact on link adaptation, channel estimation, and other procedures Meanwhile, xiaomi, Huawei, Spreadtrum, LG, Qualcomm do not see benefit for mapping a physical channel across different symbol types and thus proposed to not support. It is not clear to moderator whether that is acceptable to the group. An alternative proposal is provided to check companies’ views. Proposed Agreement: SBFD aware UE does not expect to be dynamically scheduled with a physical channel/signal without repetition that is mapped to SBFD and non-SBFD symbols within a slot. UE drops a physical channel/signal configured by higher layer or scheduled with repetition that is mapped to SBFD and non-SBFD symbols within a slot. |
| Nokia, NSB | Support for further study. We need to consider whether the listed are issue or not and whether there is performance benefit or limitation. Study whether a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot including: - Whether there is issue on Phase continuity - Potential interruption of transmissions/receptions during transition if not support the mapping to SBFD and non-SBFD symbols within a slot - Potential impact on performance, link adaptation, procedures |

Proposal 1-4

Proposed Agreement:

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within period(s) configured by TDD-UL-DL-ConfigCommon is the baseline.

  - FFS whether additional periodicity can be configured.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, Spreadtrum, vivo,CMCC, Huawei, HiSilicon,OPPO, Sharp, Ericsson, Panasonic, Qualcomm, WILUS, NEC, DOCOMO, Fujitsu, CATT, CEWiT, FGI |
| Not support | Xiaomi, Samsung, Intel, Nokia, NSB (before the real period for SBFD is clear) |

| Company | Comments |
| --- | --- |
| ZTE | We are fine with the direction of this proposal. However, we think the following two issues need to be clarified before agreeing this proposal: Issue#1: How to handle SSB with longer periodicity. For example, if the periodicity of SSB is 20ms and the periodicity for SBFD is 5ms (equal to the periodicity to the TDD pattern), due to the misalignment of periodicity of SSB and SBFD, does it mean that network has to keep the symbols unavailable for SBFD even if the SSB is not transmitted in these symbols? Issue#2: How to handle the case with dual TDD pattern periodicities? |
| Xiaomi | It is unclear to us the exact meaning for ‘within period(s)’. It is well known that a TDD-UL-DL-ConfigCommon can configure one period TDD pattern or two period TDD patterns. For single TDD pattern, it is clear from the current wording that the period of the TDD pattern should be assumed for SBFD subband configuration. If two TDD patterns are configured, what is the correct understanding on within period(s)? - Understanding#1: single configuration of SBFD subband time location is applied to period#1+period#2 - Understanding#2: separate configuration of SBFD subband time location is applied to period#1 and period#2 respectively. Besides, what if there is no TDD-UL-DL-ConfigCommon? |
| IDC | We support the proposal to further study the proposed concept. |
| Samsung | We do not think that a Rel-18 SID agreement is needed. This can be left to Rel-19 WID discussions. We assume that for backwards-compatibility reasons, any RRC provided SBFD subband configuration to be decided upon must remain “compatible” with the pattern1/pattern2 configured for the legacy UEs. This does not necessarily imply that the SBFD subband time locations “within period(s) configured by TDD-UL-DL-ConfigCommon” should be assumed baseline. Stage-3 signaling design aspects for SBFD subband configuration should be discussed in RAN2 during the WID stage. |
| Intel | We think this is WI issue, there is no need to discuss it in SI phase. |
| vivo | We are fine with the proposal, but also agree with Intel, this is more like WI details. |
| CMCC | From our understanding, this proposal just says a SBFD time location pattern can be configured with the same period as the configuration in by TDD-UL-DL-ConfigCommon, but the number of SBFD time location patterns can be further discussed. For example, different SBFD time location patterns for a TDD slot configuration period consist SSB and a TDD slot configuration doesn’t consist SSB, as well as SBFD time location patterns for a TDD slot configuration period P1 and P2. |
| OPPO | Clarification on period(s). There are two understandings Understanding 1: Multiple times of period configured by TDD-UL-DL-ConfigCommon. For example, when period is configured as 10ms by TDD-UL-DL-ConfigCommon, the SBFD period can be one value of 1*10ms, 2*10ms, 3*10ms... Understanding 2: Two period values maybe configured by TDD-UL-DL-ConfigCommon. When two period values, P1 and P2, are configured, then period for SBFD = P1 + P2 |
| LG Electronics | Have a question regarding the intention of the proposal 1-4. Is the intention of the proposal 1-4 is the period(s) of configuration of SBFD subband time location is aligned with the period(s) configured by TDD-UL-DL-ConfigCommon, and the period(s) of configuration of SBFD subband is not provided. |
| Ericsson | Support in principle, although we also recognize that this can be a WI detail. Regarding the FFS, what is the intention on other periodicities when already two periods can be configured with TDD-UL-DL-ConfigCommon? Suggest to remove FFS bullet. |
| Lenovo | We also think this is a WI details and does not need to be discussed in the SI. |
| QC | The FFS is not clear. |
| NEC | We are fundamentally okay with the proposal. However, we should also carefully consider the interaction between UL subband time configuration periodicity with SSB periodicity. This is because avoiding SSB occasions (with large SSB period) using UL subband time configuration can result in underutilization of the SBFD symbols. Consider the figure below for the scenario taken from our contribution where UL subband period is assumed to be 5ms duration and we can observe that significant number of DL symbols cannot be utilized for SBFD symbols due overlap of DL and SSB symbols during the first 5ms window |
| Nokia, NSB | One aspect that needs to be discussed in parallel with the periodicity of the semi-static configuration is whether SBFD can be configured in SSB symbols or not. If it cannot, it might be needed to further study how to handle the SSB periodicities with TDD-UL-DL-ConfigCommon and current values of the dl_UL_TransmissionPeriodicity. Before the real period for SBFD clear, we think we can not have agreement on this stage. |

Proposal 1-5

Proposed Conclusion:

At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any.

- Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

|  | Company |
| --- | --- |
| Support | New H3C (with Option1), TCL, ZTE (option1), Samsung (Option 1), Intel, ITRI(Option 1), Spreadtrum (Option 1), vivo, CMCC, Huawei, HiSilicon,OPPO, Sharp, Ericsson (prefer Option 1), Panasonic, Qualcomm, NEC, DOCOMO, Fujitsu (Option 2), CATT, CEWiT, FGI, Nokia, NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | We had an FFS in the previous agreement. Agreement (RAN1#110bis-e) For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.. At least for semi-static SBFD, it is widely agreed that either DL subband(s) or guardband(s) are explicitly configured and the other one is implicitly derived. For dynamic SBFD, further discussion is needed depending on whether flexible subband is introduced or not. |
| New H3C | We slightly prefer option 1 and the motivation and benefit of indication on guradband aren’t clear to us. |
| TCL | Prefer option 1 |
| Xiaomi | Basically, we think guard band can be guaranteed by gNB implementation as the main purpose of guard band is to suppress self-interference. UE is capable of either DL reception or UL transmission on SBFD symbol. It doesn’t care the resource type outside UL subband. Hence UL subband configuration only should be sufficient for SBFD aware UE. Considering it is the majority view that guard band should be configured explicitly or implicitly, we can live with the current direction. However, we would like to check with companies what is the value range of guard band? From our understanding, guard band is used to suppress self-interference and CLI, the determination of guard band may need RAN4 involvement. |
| Sony | I think there is a 3rd option where the guardband is implicitly and dynamically scheduled by gNB. That is gNB configures only UL subband and the understanding is that the remaining RBs can be used for DL transmission. The guardband can be made by the gNB not scheduling those RBs. This enables the gNB to extend or contract the DL subband depending upon the traffic in the UL subband. For example if there are a lot of UEs using the edge of the UL subband then gNB may want a bigger guardband and vice-versa. |
| IDC | This signaling detail can be discussed in WI phase |
| Huawei, HiSilicon | In our understanding, the basic principles for TDD UL-DL slot format configuration can be referred here. Option 1 is our preferred direction. The guard band can be implicitly determined by scheduling, i.e., the flexible RBs which are not scheduled for UL transmissions or DL receptions are guard band. |
| OPPO | We support this proposal and prefer to option 2. Frequency-domain structure of SBFD can be either {D1-U-D2} or {U,D}. What’s more, there can be various combinations of D1 and D2 for {D1-U-D2}. So it is not straight-forward and efficient in signaling overhead to explicitly indicate DL subbands. |
| LG Electronics | Clarification question about the option 1. LG proposes the explicit signaling to indicate a frequency resource that is not used as DL subbands. If not, frequency location not used as DL subband needs to be included as an ‘other subband types’. Suggest following modification: Option 1: Frequency locations of DL subband(s) or frequency locations not used as DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). |
| Ericsson | Support, since the intention of the agreement is to list two viable options. Which one to adopt can be decided in WI phase. |
| Lenovo | We also think this is a WI signaling detail. From a UE perspective, the UL subband information may be sufficient. |
| QC | It is good to list possible options during the study item phase and leave the down selection for the WI phase. |
| WILUS | We are generally fine with the proposal and support Option 1, excluding guardband(s) aspect. In our view, not all RBs outside DL/UL subbands are used for guardband(s) in flexible symbol by TDD-UL-DL-ConfigCommon, since it’s not concluded yet whether to allow or not DL reception and/or UL transmission outside DL/UL subbands. For guardband(s), at least minimum fixed number of RBs can be defined in specification based on the RAN4 study. Then, remaining RBs outside DL/UL subbands and fixed guardband(s) have possibility to be scheduled for DL reception and/or UL transmission, and it’s up to gNB scheduler. |
| NEC | We are okay to have this discussion but it would be preferable to follow the methodology of RB sets already present in RAN1 spec to specify subband and guardband location. |
| Fujitsu | We think either way of two options can result in same performance. From the signalling overhead perspective, however, the explicit indication of DL subband should have more information at least {the starting position and the number of RBs} while the explicit indication of the guardband case requires only {the number of RBs} since the guardband is always adjacent to UL subband. |
| Moderator | Moderator would like to clarify that the intention of the proposal is not for down-selection but to provide viable solutions for down-selection in WI phase. Some companies prefer to support configuring UL subband only and the guardband(s) if any are implicitly implemented by gNB. According to moderator’s understanding, that can be achieved by both options. To be specific, for Option 1, gNB can configure DL subband(s) as RBs excluding UL subband. For Option 2, gNB can configured guardband size of 0. No update to the proposal. |
| Nokia, NSB | We are fine with the proposed conclusion, and the details can be discussed in the WI. |

Proposal 1-6

Proposed Conclusion:

The main advantage of dynamic SBFD compared with semi-static SBFD is better resource adaptation between DL and UL based on UL/DL traffic ratio.

Intra-subband CLI can be avoided in semi-static SBFD if the SBFD subband configurations are aligned, but intra-subband CLI cannot be avoided in dynamic SBFD.

There would be additional specification efforts to support dynamic SBFD.

|  | Company |
| --- | --- |
| Support | New H3C, TCL(modification), Sony (agreed dynamic SBFD has better resource adaptation), IDC (only the first sentence), Intel (modification), Huawei, HiSilicon, WILUS (1st and 3rd bullet), NEC (modified), Fujitsu, CATT, CEWiT (partial) , Nokia, NSB (only after remove 2nd paragraph) |
| Not support | Xiaomi, Sony (don’t agree that intra-subband CLI cannot be avoided in dynamic SBFD), Samsung, ITRI, Spreadtrum, vivo, Sharp, Ericsson (support 3rd bullet), FGI |

| Company | Comments |
| --- | --- |
| TCL | It is too early to conclude that the intra subband CLI cannot be avoided in dynamic SBFD. In our view, in dynamic TDD intra-subband CLI can be further studied. |
| ZTE | Dynamic SBFD can be used in the scenario with smaller or controllable CLI. |
| Xiaomi | We want to understand of the motivation to have this conclusion. It seems quite obvious and the issues listed above will be considered in further study on dynamic SBFD. |
| Sony | We agree that dynamic SBFD has better resource adaptation compared to semi-static SBFD. We don’t agree that intra-subband CLI cannot be avoided in dynamic SBFD. In fact, if different gNB semi-statically configures their cell with different SBFD configurations, it is much harder to avoid intra-subband CLI. We have coordinated scheduling being discussed in Agenda Item 9.3.3 that may enable mitigation of different subband configurations among gNBs, which is faster if we have dynamic SBFD since gNB can change the configuration quickly compared to semi-static SBFD. |
| IDC | Share similar views as Sony. |
| Samsung | We think that it is too early for the FL proposed conclusion while SL evaluations in 9.3.1 for most scenarios/cases are still ongoing. We see benefits when supporting DL scheduling in configured SBFD UL subbands on legacy D/F symbols at least from the gNB perspective. However, SL performance of semi-static SBFD config options D-1/F-1 compared to dynamic SBFD D-2/F-2/F-3 and their comparison to Rel-15 TDD performance, potential UE modem impacts and resulting specification impacts when supporting any of these or a combination all still need to be discussed. |
| Intel | We share similar view with TCL and ZTE that CLI may or may not avoided depending on scenarios. |
| Spreadtrum | Agree with Samsung, suggest to postpone this conclusion. |
| vivo | The conclusions for dynamic SBFD vs. semi-static SBFD should be based on the evaluation results with explicitly stating the scenarios, relevant assumptions and analysis. For example, under which scenario, what assumptions, which factors (i.e., better resource adaptation, interference handling) bring the performance gain and which factors results in performance loss (i.e., intra-subband CLI). Current conclusion is applied for all scenarios, traffic load, all cases, which is not correct. Also share with other companies that depending on the traffic load, dynamic SBFD can even avoid the intra-subband CLI. |
| CMCC | It is too rough to say “intra-subband CLI cannot be avoided in dynamic SBFD”, for example, if the neighbor cells also disable the UL subband at the same time, the SBFD/TDD pattern are still aligned. |
| OPPO | Information from “better” is very limited and not clear. If we want to give some information or suggestion for further study, it’s better to list evaluation results. In addition, dynamic TDD is an existing technology, so the advantage of dynamic SBFD compared with dynamic TDD or dynamic TDD+semi-static SBFD needs to be considered. |
| LG Electronics | The intra-subband CLI is a common issue for both dynamic SBFD and dynamic TDD. If the co-channel CLI handling scheme(s) is/are applied for dynamic SBFD, the intra-subband CLI in dynamic SBFD can be mitigated. Suggest the following modification: Intra-subband CLI can be avoided in semi-static SBFD if the SBFD subband configurations are aligned, but inter-cell coordination may be necessary to mitigate intra-subband CLI in dynamic SBFD. There would be additional specification efforts to support dynamic SBFD. For example, indication for switching from SBFD operation and non-SBFD operation can be specified. |
| Sharp | Intra-subband CLI cannot be avoided when some of the gNBs does not have support for SBFD. To avoid it, all the gNBs in a frequency needs to support SBFD. Therefore, we suggest deleting the 2nd bullet. |
| Ericsson | We also feel that it is premature to make conclusions on potential advantages of dynamic SBFD. As our system level evaluations show, dynamic TDD (much simpler than dynamic SBFD) can be used to achieve resource adaptation with equal or superior performance compared to dynamic SBFD. Hence any conclusions made on dynamic SBFD must consider dynamic TDD as a comparison point, otherwise the conclusion is misleading. This being said, we agree that dynamic SBFD likely has (significant) spec impact compared to semi-static SBFD, hence we agree with the 3rd bullet. |
| Panasonic | We share with the view of TCL and other companies. It is too early to conclude that the intra subband CLI cannot be avoided in dynamic SBFD. |
| Lenovo | We should await conclusion on the simulation evaluations before concluding “intra-subband CLI cannot be avoided in dynamic SBFD” irrespective of the scenario and assumptions. |
| QC | The conclusion is mainly to list the pros/cons for dynamic SBFD versus semi-static SBFD. In general, Dynamic SBFD provides gNB flexibility on resource allocation and adaptivity to traffic pattern, however, it comes at cost of increased complexity, signaling overhead and CLI. |
| NEC | Intra-subband CLI for dynamic SBFD may not be a significant issue when dynamic SBFD is mainly used for mmwave based deployment scenarios like indoor hotspots. For e.g. when macro sites are deployed in FR1 while PSCell in FR2 operate with dynamic SBFD. In such situations, dynamic SBFD can provide higher returns due to dynamic traffic requirements of hotspots while ensuring lower CLI. We can understand that for macro sites, managing CLI for dynamic SBFD can be an issue. Hence, following modification is preferred: Intra-subband CLI can be avoided in semi-static SBFD if the SBFD subband configurations are aligned, but intra-subband CLI cannot be avoided in some scenarios in dynamic SBFD. |
| DOCOMO | Share similar view as TCL, ZTE and Intel. |
| Fujitsu | Like other companies thought, we don’t think that intra-subband CLI cannot be avoided in dynamic SBFD. |
| CEWiT | We only support “The main advantage of dynamic SBFD compared with semi-static SBFD is better resource adaptation between DL and UL based on UL/DL traffic ratio.” part of the proposal. It is too early to conclude on the second and third clauses. |
| FGI | We think it is too early to draw a general conclusion while evaluations are still in progress. |
| Nokia, NSB | We have a similar view as TCL. It is too early to conclude that intra subband CLI cannot be avoided in dynamic TDD. We support the proposed conclusion without the second paragraph. Actually intra-subband CLI also exist if there is different DL/UL SBFD subband in multiple cells even with semi-static SBFD. |

Proposal 1-7

Proposed Agreement:

For dynamic SBFD in the symbol configured as flexible in TDD-UL-DL-ConfigCommon, study Option 2 vs. Option 3 including:

  - Necessity and potential benefits of same mechanism as in symbol configured as DL in TDD-UL-DL-ConfigCommon

  - Additional benefit of allowing UL transmissions outside semi-statically configured UL subband

  - Potential additional complexity to allow UL transmissions outside semi-statically configured UL subband

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Xiaomi, Sony, IDC, Intel , CMCC, Huawei, HiSilicon, LG Electronics, Sharp, Panasonic, WILUS, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Samsung, Spreadtrum, vivo, Ericsson, Qualcomm |

| Company | Comments |
| --- | --- |
| ZTE | We support this proposal. Supporting UL transmission outside the UL subband requires network to adjust its UL subband filter dynamically, which complicates the network implementation. From this perspective, we don’t support UL transmission outside the UL subband. In addition, the flexibility can be already achieved if the DL transmission can be received within the DL subband. There is no motivation to further support UL transmission outside the UL subband. |
| xiaomi | We are supportive to further study option2 and option3. But the intention should be make a down selection between these options rather than provide a possibility that support both. To make it clear, we propose the following modification for the main bullet: Proposed Agreement: For dynamic SBFD in the symbol configured as flexible in TDD-UL-DL-ConfigCommon, study Option 2 vs. Option 3 including: down select between option 2 and option 3 at least considering the following aspects: - Necessity and potential benefits of same mechanism as in symbol configured as DL in TDD-UL-DL-ConfigCommon - Additional benefit of allowing UL transmissions outside semi-statically configured UL subband - Potential additional complexity to allow UL transmissions outside semi-statically configured UL subband |
| Samsung | In RAN1#112, we have already agreed that “Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.” for Options F-1 (semi-static), F-2 (DL in UL SB) and F-3 . What would be the purpose or the additional value of the FL proposed agreement? |
| Spreadtrum | Agree with Samsung, suggest to postpone this proposal. |
| vivo | Not sure what additional delta for this proposal compared to the agreements we made in the last meeting. |
| OPPO | Firstly, semi-static SBFD configuration is an baseline, so option 1 should not be excluded. Secondly,benefit of allowing UL transmissions outside semi-statically configured UL subband is the same as benefit of allowing DL transmissions outside semi-statically configured DL subband ,i.e. better resource allocation adapting to DL/UL traffic. So we do not see the reason to support different solutions for DL and UL. If concern is that various UL bandwidth is difficult to implement, but the whole bandwidth for UL is already supported for UL transmission, so i.e. UL subband converting to UL symbol, should be supported due to no extra complexity . So we prefer to narrowdown to option 3. If option 2/3 needs to further study, we’d better identify: - Additional benefit of allowing DL/UL transmissions outside semi-statically configured DL/UL subband - Potential additional complexity to allow DL/UL transmissions outside semi-statically configured DL/UL subband |
| Ericsson | We share a similar view as Samsung. We have already agreed to study Options 2 and 3, so it’s not clear why this proposal is needed. Moreover, Option 1 is missing as a comparison point, as is dynamic TDD. |
| QC | Regarding SBFD operation in ‘FL’ symbols, we think that option-2 is sufficient to have consistent behavior for SBFD-aware UE in ‘DL’ and ‘FL’ symbols. We fail to see motivation and benefits of option 3 versus configuring the symbol as a whole ‘FL’ symbol. |
| Nokia, NSB | We support for further study in the SI. |

Proposal 1-8

Proposed Agreement:

Study the following options for dynamic SBFD.

- Option 1: Dynamic SBFD is achieved by flexible subband where RBs in flexible subband can be dynamically used for UL transmission and DL transmission.

- Option 2: Dynamic SBFD is achieved by scheduling DCI which indicates whether allowing DL receptions outside semi-statically configured DL subband and/or allowing UL transmission outside semi-statically configured UL subband.

  - Option 2-1: explicit indication in scheduling DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol, or whether DL receptions outside semi-statically configured DL subband and/or UL transmission outside semi-statically configured UL subband are allowed

  - Option 2-2: implicit indication in scheduling DCI based on resource allocation

- Option 3: Dynamic SBFD is achieved by group-common DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, Intel, ITRI , vivo, CMCC, Huawei, HiSilicon, LG Electronics, Sharp, Panasonic, Lenovo, Qualcomm, WILUS, NEC (prefer Option-3), Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Xiaomi, Samsung, Spreadtrum, Ericsson |

| Company | Comments |
| --- | --- |
| ZTE | We support the above proposal. Regarding Option 2-2, we think it can be combined with rate-matching indication. For example, if the scheduling spans across two DL subbands and rate-matching indication is indicated, then the UL subband is not used for DL transmission; however if the rate-matching indication is not indicated, then the UL subband is used for DL transmission. |
| xiaomi | For option 1, the same functionality can be achieved by flexible symbol. We don’t see additional benefits compared with flexible symbol and hence don’t quite understand the motivation of option 1. For option 2-2, we don’t think it is workable as a UE cannot differentiate the following two cases if, for example, a DL RBG partially overlaps with UL subband - Case#1: UE only use the RBs not overlapping with UL subband for DL reception - Case#2: UE can use the RBs overlapping with UL subband for DL reception. For the other options, we are fine for further study. |
| IDC | Support the proposal in principle, and OK for further study on all 3 options. |
| Samsung | We cannot support the FL proposed agreement. In the Rel-18 SID, we have not concluded yet on the support of dynamic SBFD. It does not appear meaningful to us to discuss the signaling design options before making some more progress which SBFD scheduling options are actually beneficial and deemed feasible. |
| Intel | We tend to agree with Samsung that the signaling design option can be discussed after progress of whether dynamic SBFD is supported. Having said that, we are fine with high-level study for signaling design. In our view, functionality of flexible subband can be achieved by other options, we don’t see any benefit of option 1. |
| Spreadtrum | Agree with Samsung. |
| vivo | We are fine to list all the signaling details, although they may not be essential for the SI conclusion. |
| CMCC | We prefer option 3. For option 1 and option 2, one issue is other periodic physical channel/RS transmission assumption, e.g., periodic CSI-RS, how can UE know these periodic physical channel/RS are transmitted in a full D/U symbol manner or SBFD symbol manner, since the available RB resources and other configuration parameters are all different. |
| Huawei, HiSilicon | We are fine with above proposal and Option 1 is preferred. - For option 1, the SBFD symbols with flexible subband and the UL/DL symbols can be converted to each other by scheduling. - For option 2-1, a new field should be added into DCI to indicate the symbol type. - For option 2-2, it is not clear to us how the implicit indication can be achieved. - For option 3, an additional DCI format should be introduced. As discussed above, option 1 is more flexible and compared to other options, it has less spec impact and forward compatible to subband over-lapping full duplex. |
| OPPO | Dynamic SFI can be reused directly to support dynamic SBFD, especially flexible symbol with UL subband converting to full-DL/full-UL.So Dynamic SFI can be a baseline. The benefit and complexity of all options should be identified if support. |
| Ericsson | We think that whether or not there is a performance benefit of dynamic SBFD should be established first, especially in relation to dynamic TDD. Signaling design should be left to WI phase. |
| QC | Generally fine, however, we think RAN1 should discuss the signalling mechanism after we settle on Proposal 1-6 and Proposal 1-7. |
| NEC | For Option-3, signaling details can be discussed later whether it is group-common or UE-specific. So propose following rewording for Option-3: - Option 3: Dynamic SBFD is achieved by group-common DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol. |
| DOCOMO | We share same feeling as Samsung and Intel that the signaling design can be discussed in later stage. For option 2-1 and option 2-3, we have one question regarding “SBFD symbol or full-DL/full-UL symbol”. Is the intention to preclude “full-flexible symbol”? |
| Fujitsu | We support the proposal for further study. One comment on Option1 is that all transmission directions on the flexible subband should be same. |
| FGI | Option 2-2 may not be workable of different parameters, e.g., power control, beam, etc, are applied for SBFD and non-SBFD symbols. |
| Moderator | Several companies think that this can be discussed later after we conclude whether or not there is a performance benefit of dynamic SBFD. However, we only have two meetings left and the gap between this meeting and the next meeting is very short. I doubt whether we can conclude in the next meeting. In addition, moderator thinks that this is also part of the feasibility study of dynamic SBFD. Hope it clarifies the intention. To address NEC’s comments, group-common DCI is updated to non-scheduling DCI to include both group-common and UE-specific DCI. Proposed Agreement: Study the following options for dynamic SBFD. - Option 1: Dynamic SBFD is achieved by flexible subband where RBs in flexible subband can be dynamically used for UL transmission and DL transmission. - Option 2: Dynamic SBFD is achieved by scheduling DCI which indicates whether allowing DL receptions outside semi-statically configured DL subband and/or allowing UL transmission outside semi-statically configured UL subband. - Option 2-1: explicit indication in scheduling DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol, or whether DL receptions outside semi-statically configured DL subband and/or UL transmission outside semi-statically configured UL subband are allowed - Option 2-2: implicit indication in scheduling DCI based on resource allocation - Option 3: Dynamic SBFD is achieved by non-schedulinggroup-common DCI which indicates whether a symbol is SBFD symbol or a full-DL/full-UL symbol. |
| Nokia, NSB | We support the proposal. Our preference is Option 3 as it also allows for adaptation of semi-statically configured resources based on symbol or slot type. We are also open to discuss dynamic SBFD via implicit indication in scheduling DCI. The complexity and impact on collision handling should also be studied. |

Proposal 1-9

Proposed Conclusion:

Regarding whether SBFD operation in SSB symbols is supported or not, the following observations are agreed.

- If SBFD operation in SSB symbols is supported, UE-to-UE CLI may degrade the performance of SSB detection/measurement. Due to the uncertainly of when/which UE would detect/measure SSB, it is difficult for gNB to mitigate the performance degradation and UE may not be able to transmit UL in SSB symbols as expected by gNB if UE needs to receive SSB due to half-duplex operation at UE side.

- If SBFD operation in SSB symbols is not supported, the SBFD performance may be degraded due to less number of symbols for SBFD operation and there would be transition(s) between SBFD symbols and non-SBFD symbols within the slot if the non-SBFDSSB symbols are used for SBFD operation.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, Intel , vivo, Huawei, HiSilicon, Sharp, Panasonic, Qualcomm, NEC, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Samsung, Ericsson |

| Company | Comments |
| --- | --- |
| ZTE | Agree with the intention. However, the last sentence seems confusing, i.e., “if the non-SBFD symbols are used for SBFD operation.”. |
| Moderator | Correct the typo in the proposal in red. |
| Xiaomi | We don’t see the necessity to have this inter-mediate conclusion. The listed issues will be surely considered for the further study on whether allows SBFD operation on SBFD symbols. |
| Samsung | We think that we should try to reach an agreement if SBFD UL subband configuration on SSB symbols is supported or not. We do not necessarily agree with the FL proposed observations as is. For SBFD-aware UEs in RRC_CONNECTED mode, SSB-based measurements are not uncertain. SMTCs and measurement gaps apply. MGs and the UE scheduling gaps are needed and inherent to Rel-15 operation even for intra-frequency measurements where SSBs of serving/neighbor cell are not located on the same sync raster point or use different SCSs. For legacy or SBFD-aware UEs in RRC_IDLE/INACTIVE, it is mostly important to protect the serving cell measurements inside the SMTC. In our view, the possibility to configure the SBFD UL subband on the SSBs at all mostly depends on the gNB-side SBFD antenna configuration option. For Option 1, large/undesired impact on DL cell coverage (5-6 dB) can result. For Option 2, the SSB symbols can be used for as long as DL EPRE over the SSS is sufficiently stable. If an gNB can configure the UE if the SBFD UL subband uses the SSB symbols, the gNB implementation can decide depending on its antenna implementation. |
| CMCC | Support in principle, but regarding the “the SBFD performance” in the second bullet, it is much better to list which performance, e.g., the UL UPT rather than SBFD performance, because some performance metric, e.g., DL UPT may not be degraded. |
| OPPO | If SBFD operation in SSB symbols is supported, different antenna configurations may be applied for SBFD symbol and non-SBFD symbol, e.g. 8 Tx for DL in non-SBFD symbol and 4Tx for DL and 4Tx for UL in SBFD symbol. It leads different SSB coverage and decoding performance for SBFD symbol and non-SBFD symbol. |
| LG Electronics | The observation seems unclear. (1) If the performance of SSB detection/measurement is degraded, I think the reasons may be the intra-cell inter-subband UE-to-UE CLI and/or lack of measurement opportunity. (2) Even if the SBFD operation in SSB symbols is not supported, the enhancement of UL performance than legacy TDD can be achieved. |
| Ericsson | We don’t agree to the conclusion, and prefer to conclude first whether UL subband can be configured in SSB symbols. Agree with Samsung that it is not uncertain when UEs measure SSB. For RRM measurements, SMTC windows are configured, and the UE can be configured with a bitmask (ssb-ToMeasure) indicating which SSBs to measure. Related to whether or not UL subband can be configured in SSB symbols, we observe that in current specs, if the bitmask is not configured, then the UE is not allowed to transmit during the whole SMTC window which can be up to 5 ms. This is not an issue in conventional TDD, since SSBs are only allowed in DL symbols/slots. However, for SBFD where the goal is to provide additional UL opportunities, this can lead to an UL outage of approximately 5 / 20 ms = 25% which would impact the claimed latency benefits of SBFD. We think this should be considered when deciding if UL is allowed in SBFD symbols. Instead of simply allowing/disallowing UL in SSB symbols, we think it would be useful to have more configurability, so that either SSB measurements could be prioritized or UL transmission in SSB symbols are prioritized. To achieve this configurability, UL subband should be allowed to be configured in SSB symbols. Whether or not it is used for UL, depends on configuration. |
| QC | From RAN1 perspective, we think as a first step, SBFD subbands can be configured in SSB symbols. Whether to allow SBFD operation or not is next level of discussion. In other words, two options are on the table: - SSB configured in SBFD symbols, however, UL transmission is restricted - SSB configured in SBFD symbols with possible SBFD operation under some limits. |
| WILUS | Agree with Ericsson. Whether to receive SSB or transmit UL signal/channel can be differently handled according to SSB types (ssb-PositionsInBurst or ssb-ToMeasure). |
| Nokia, NSB | We do not support for SBFD operation in SSB symbols. SBFD operation in SSB symbols will impact SBFD aware UE as the first bullet and it may also impact to legacy UE for SSB reception if the guard band is not large enough. We want to improve UL performance but we think the number of SSB symbols is small to provide much benefit for UL performance. If we want to add the second bullet, it should also mention “the number of SSB symbols is small and the benefit of SBFD in SSB symbols may be limited.” |

Proposal 1-10

Proposed Agreement:

For SBFD-aware UEs, Option 1 is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization.

For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, xiaomi, Sony, IDC, Samsung, Intel, ITRI, Spreadtrum, vivo, CMCC, Huawei, HiSilicon,OPPO, LG Electronics, Sharp, Ericsson, Lenovo, Qualcomm, WILUS (w/modification), NEC, DOCOMO, Fujitsu, CATT, FGI, Nokia, NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Ericsson | Support Proposal 1-10. Minor editorial comment: add “The” before “Part” in both sub-bullets to clarify that all of the part of the DL/UL RBG that is inside the DL subband can be used, not just part of the part. |
| WILUS | We are fine to study Option 1. However, there was a “FFS: The part of the RBG outside” in the previous agreement. We propose to capture this FFS in Proposal 1-10: Proposed Agreement: For SBFD-aware UEs, Option 1 is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used FFS: The part of the RBG outside. |
| Moderator | Ericsson’s comment is valid so the proposal is updated accordingly. For the comment from WILUS, I think the FFS can be separately discussed which is in the previous agreement anyway. Proposed Agreement: For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1 (with update): - The Part of the DL RBG inside the DL subband can be used - The Part of the UL RBG inside the UL subband can be used |

Proposal 1-11

Proposed Agreement:

For a PRG that overlaps with subband boundary, partial DL PRG inside the DL subband can be used.

  - FFS additional restriction to address potential concerns on UE implementation complexity, e.g. only applicable to PRG size of 4.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, xiaomi, Sony, IDC, Intel, ITRI , vivo, CMCC, LG Electronics, Sharp, Ericsson (with modification) , Lenovo (without FFS). NEC, DOCOMO, CATT, FGI, Nokia, NSB |
| Not support | Samsung, Spreadtrum, Huawei, HiSilicon, Qualcomm, WILUS |

| Company | Comments |
| --- | --- |
| ZTE | We support the proposal. We don’t understand the motivation to limit it only to PRG size of 4. Maybe we can update it as a general FFS, e.g., For a PRG that overlaps with subband boundary, partial DL PRG inside the DL subband can be used. - FFS additional restriction to address potential concerns on UE implementation complexity, e.g. only applicable to PRG size of 4. |
| Samsung | We consider the potential benefits of using the partial PRG as marginal and are concerned about the potential impact to the UE channel estimation implementation. In our view, the FFS should be resolved first before deciding if partial PRG makes sense at all to consider. |
| Spreadtrum | First, we don’t understanding the FFS point. When PRG size is 2, UE does not use this partial PRG for channel estimation? If so, how UE to decode the PDSCH using the PRBs. Second, for the main bullet, it can be clear from the point of UE perspective. The change can be: For a PRG that overlaps with subband boundary, UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a partial DL PRG inside the DL subband can be used. - FFS additional restriction to address potential concerns on UE implementation complexity, e.g. only applicable to PRG size of 4. |
| Huawei, HiSilicon | We have a concern on the FFS. In current spec, the partial PRG has been supported regardless of PRG size is 2 or 4. So why we need to study such an additional restriction? |
| Ericsson | We support the main bullet, but we have similar concerns on the FFS point as Spreadtrum. We emphasize that partial PRGs are already supported in current specifications, and it is mandatory to support them (no UE capability defined). Hence, we don’t see why they should not be supported for SBFD Hence, we propose to remove the FFS bullet. Minor editorial comment: in the main bullet, change “partial” to “the part of the” for better readability. |
| QC | The benefits of having extra partial PRGs across the DL subband boundaries is minimal (1-3 RBs in spectrum utilization) and are not motivating the extra UE complexity for DMRS channel estimation. |
| WILUS | We share the similar view with Huawei. Partial PRG is already supported as denoted by “Actual number of consecutive PRBs in each PRG could be one or more” in TS 38.214. No additional handling is needed for PRG in SBFD operation. |

Proposal 1-12

Proposed Agreement:

If PRG is determined as wideband, study the following two options:

- Option 1: non-contiguous frequency resources across two DL subbands can be allocated

  - Same precoding is assumed within each DL subband

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

- FFS threshold for the minimum RB size needed to select wideband precoding for dynamic PRG bundling

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, Intel (with Xiaomi’s revision), ITRI, vivo, Huawei, HiSilicon, Sharp, Ericsson (with modification), Panasonic, Lenovo, Qualcomm, WILUS (with Xiaomi’s revision), NEC, DOCOMO, CATT, FGI |
| Not support | Xiaomi, Samsung, Spreadtrum |

| Company | Comments |
| --- | --- |
| Moderator | For wideband precoding, two options are identified which are proposed to be further studied. In addition, MediaTek pointed out the issue of threshold for the minimum RB size needed to select wideband precoding for dynamic PRG bundling. Please refer to section 3.1.2.1.2 or R1-2302736 for more details. |
| Xiaomi | From our understanding, the intention of option 2 is to fully reuse the current specification, i.e. non-contiguous RB allocation is not expected for wideband precoding. Hence the last FFS point should be applicable to option 1 only. We are OK with the proposal with the following formulation: Proposed Agreement: If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands can be allocated - Same precoding is assumed within each DL subband - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated - FFS threshold for the minimum RB size needed to select wideband precoding for dynamic PRG bundling |
| Samsung | We share a similar view as Xiaomi and support the proposed re-wording. |
| Spreadtrum | First, it should only limit to the effect of UL subband for PDSCH resource allocation in Option 1. Second, same precoding within each DL subband or across two DL subbands can be open to discuss. Last, also support to remove the FFS point. The proposal is updated in red words: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - FFS: Same precoding is assumed within each DL subband or across two DL subbands |
| CMCC | Fine with Xiaomi’s re-wording |
| LG Electronics | In case of option 1, frequency diversity gain can be achieved and frequency selective scheduling can be allowed. |
| Ericsson | Support removing the FFS as proposed by Xiaomi. For the sub-bullet of Option 1, we don’t understand why Spreadtrum changes this to FFS. However we agree that the wording is not entirely clear. Hence we propose the following modification: - Option 1: non-contiguous frequency resources across two DL subbands can be allocated Same precoding is assumed within each in both DL subbands |
| QC | Okay to study both options. Strong preference on option 2. There is no need for the FFS at this point of the study phase. |
| Moderator | Some companies commented that FFS is applicable to Option 1 only. However, moderator has a different understanding. The issue is that for selecting wideband precoding in dynamic PRG bundling the allocated RBs should occupy more than half the BWP. This threshold may not be met in SBFD symbol with UL subband. It is applicable to both cases. But on the other hand, Qualcomm’s comment that FFS is not needed in the study phase seems valid. So the FFS is removed. For Option 1, the original intention was that UE does not assume same precoding across DL subbands since UE does not expect to do joint channel estimation across two DL subbands. If some companies think same precoding is also assumed across two DL subbands, we can add an FFS. Proposed Agreement: If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - Same precoding is assumed within each DL subband - FFS whether same precoding is assumed across two DL subbands - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated - FFS threshold for the minimum RB size needed to select wideband precoding for dynamic PRG bundling |
| Nokia, NSB | For option 1, we think whether same precoding is assumed within each DL subband or same precoding is assumed for both DL subband should be studied. We proposed to remove the FFS as also proposed by other companies. |

Proposal 1-13

Proposed Agreement:

Study enhanced rate matching for PDSCH where resources outside DL subband(s) are considered as resources not available for PDSCH including PDSCH DM-RS. PDSCH DM-RS may overlap with the resources not available for PDSCH.

    - Note: a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH according to current specification.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, xiaomi, IDC, Intel, ITRI , vivo, Huawei, HiSilicon, Panasonic, Lenovo, Qualcomm, WILUS, NEC, Fujitsu, CATT, FGI, Nokia, NSB |
| Not support | Sony, Samsung, Spreadtrum, Sharp, Ericsson (proposal not clear) |

| Company | Comments |
| --- | --- |
| ZTE | Currently, both semi-static configured rate-matching pattern and dynamic indication of rate-matching pattern are supported. If the enhanced rate matching is introduced for SBFD, we assume both semi-static configuration and dynamic indication of rate-matching are supported as well. |
| Sony | Is this proposal for {DUD} subband configuration? If yes, we think enhanced Rate Matching is not the only solution there are other FDRA solutions that we have yet to consider. |
| Samsung | We have concerns about the associated UE and gNB modem design impact when new RE-level rate-matching functionality is introduced for SBFD. Changes to existing Rel-15 per-symbol/RE-level processing of PDSCH are high-impact. If needed, Rel-15 allows the possibility to configure RB level rate-match patterns for the UE. An alternative is the use of FDRA. Studying associated complexity and benefits of enhanced RM according to the FL proposal is meaningful for us only when compared to the existing spec functionality. |
| Intel | In our understanding, it is not RE-level rate matching, it is PRB-level rate matching, considering UL/DL subband is configured with granularity of PRB. |
| Spreadtrum | It is confused why PDSCH DM-RS can overlap with the resources not available for PDSCH, if the resources are not available for PDSCH including PDSCH DMRS, the DMRS only mapping without PDSCH? |
| CMCC | We also think other FDRA enhancement solution should be studied as well. |
| OPPO | We are fine with enhanced rate matching but the proposal is too narrow at SI stage. Moreover, rate matching related procedure, e.g. rate matching pattern and TBS calculation should be studied too. - Rate matching indicator is a configurable field and does not exist in fallback DCI, so resources outside DL subband(s) can not always be indicated by Rate matching indicator, especially for initial access stage when only fallback DCI can be used. - TBS is determined by higher layer configured parameter NohPRBto exclude unavailable resource .However, UL subband is not configured in each DL symbol, so higher layer parameter does not adapt to symbol/slot level UL subband configuration. Moreover, limited NohPRB value ,i.e.{0/6/12/18} does not correspond to various UL subband/guard band bandwidth. In addition, except DL transmission, there is still unavaliable resource for UL transmission, e.g. RB outside UL subband in RBG overlapping with subband boundary So we suggest to study enhanced rate matching procedure due to resources outside DL/UL subband(s) , at least including: - Rate matching pattern determination - TBS determination DMRS in unavailable resource |
| LG Electronics | The intention of the proposal 1-13 is not clear. Further clarification seems necessary for clear understanding the proposal. |
| Sharp | If DMRS REs are not considered, what is the motivation to discuss rate-matching for SBFD? Our understanding is rather that rate-matching is not applicable to avoid PDSCH allocation from UL subband. |
| Ericsson | Is this proposal regarding same or different slots? If it is about same slot, then there is another proposal on supporting partial RBGs, hence for Type 0 FDRA, it would be an error case if and RBG that is fully outside the DL subband is indicated. Furthermore for Type 1 FDRA, it would also be an error case if an RB outside DL subband is indicated. If it is about different slots (e.g., PDSCH repetition across SBFD and non-SBFD slots), then rate matching could be discussed, e.g., to handle a case where the FDRA is for the whole carrier (in non-SBFD symbols), but restricted to DL subbands in non-SBFD symbols. |
| DOCOMO | We are not very sure about the intention of the Proposal. Is the intention that unavailable resource outside DL subband is indicated by enhanced rate matching pattern? If {DUD} subband location is configured/indicated to UE, why such rate matching pattern is needed? |

Proposal 1-14

Proposed Conclusion:

For the options agreed in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed.

  - For all the options, there is no impact on CSI-RS sequence generation.

  - Option 1 configures CSI-RS resources in two DL subbands as two CSI-RS resources which may limit UE capability of maximum number of configured CSI-RS resources. Additional signalling is needed to link two CSI-RS resources. Option 1 is not applicable to periodic/semi-persistent CSI-RS across SBFD and non-SBFD symbols.

  - Option 2-1 requires new RRC structure to configure non-contiguous resource for one CSI-RS resource, which is more flexible but with higher signalling overhead. Option 2-1 is not applicable to periodic/semi-persistent CSI-RS across SBFD and non-SBFD symbols.

  - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. It is applicable to periodic/semi-persistent CSI-RS across SBFD and non-SBFD symbols and can be used to resolve the unaligned boundaries between CSI-RS resource configuration and SBFD subbands

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE (option 2-2), Sony, IDC (Option 2-2), Samsung, Intel, Spreadtrum, vivo , CMCC,OPPO, LG Electronics (option 2-2, adding a sentence), Sharp, Ericsson (support with modifications), Panasonic, Lenovo, Qualcomm, WILUS (Option 2-2), NEC, DOCOMO, Fujitsu (Option2-2), CATT, CEWiT (option 2-2), FGI, Nokia, NSB |
| Not support | xiaomi, Huawei, HiSilicon |

| Company | Comments |
| --- | --- |
| ZTE | From our perspective, option 2-2 is a clean and complete solution that can address almost all CSI-RS related issues in SBFD operation. |
| Xiaomi | We believe the above observation is the common understanding from the group. We don’t see the necessity to have such inter-mediate conclusion. Actually based on the proposed conclusion, option 2-2 should be the way to go forward. |
| IDC | Support the proposal Option 2-2. |
| Intel | We think it is quite obvious to go with option 2-2 with the benefit listed in the proposal . |
| CMCC | Also support Option 2-2 |
| Huawei, HiSilicon | For Option 2-1, it is also applicable to periodic/semi-persistent CSI-RS across SBFD and non-SBFD symbols. For example, [a1,b1] and [a2,b2] are two frequency ranges indicated by option 2 on SBFD symbols, and [min(a1,a2),max(b1,b2)] is the frequency range on non-SBFD symbols. In addition, [a3,b3] and [a4,b4] indicates two frequency ranges as [a3,a4-1] and [b3+1,b4] on SBFD symbols, and [a3,b3] is the frequency range on non-SBFD symbols. |
| OPPO | Support proposal and slightly prefer to option 2-2 |
| LG Electronics | Generally fine with making an observation for capturing the study result in the TR. In case of option 2-2, separate resource configuration can be provided for each symbols (i.e., SBFD symbols and non-SBFD symbols). In my understanding, one advantage of option 2-2 is same configuration method can be applied for both symbols. |
| Ericsson | Support the observations with the following updates. We point out that there is only an issue for periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS occurs in both SBFD and non-SBFD symbols. We point out that this can be avoided by configuration in many cases. Also the “higher signaling overhead” for Option 2-1 is a bit of an overstatement. It may be that only one additional parameter is needed, e.g., a frequency offset for where the 2nd DL subband starts. If DL subbands are configured separately, this additional parameter may not be needed at all. For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 configures CSI-RS resources in two DL subbands as two CSI-RS resources which may limit UE capability of maximum number of configured CSI-RS resources. Additional signalling is needed to link two CSI-RS resources. Option 1 is not applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols. - Option 2-1 requires new RRC structure to configure non-contiguous RBs resource for one CSI-RS resource, which is more flexible but may require additional with higher signalling overhead. Option 2-1 is not applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. It is applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols and can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands |
| QC | One note regarding the usability of a CSI-R across SBFD and non-SBFD. This will depend on gNB implementation on antenna/panels configuration and whether the same number of CSI-RS ports are used for SBFD and non-SBFD symbols and assumptions on power/QCL. |
| FGI | Support Option 2-2. |
| Moderator | Regarding the comment from Huawei that for Option 2-1, it is also applicable to periodic/semi-persistent CSI-RS across SBFD and non-SBFD symbols. It is not quite clear to me. Let’s check whether the update from Ericsson is agreeable. Proposed Conclusion: For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 configures CSI-RS resources in two DL subbands as two CSI-RS resources which may limit UE capability of maximum number of configured CSI-RS resources. Additional signalling is needed to link two CSI-RS resources. Option 1 is not applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols. - Option 2-1 requires new RRC structure to configure non-contiguous RBs resource for one CSI-RS resource, which is more flexible but may require additional with higher signalling overhead. Option 2-1 is not applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. It is applicable to periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both across SBFD and non-SBFD symbols and can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands |

Proposal 1-15

Proposed Agreement:

For unaligned boundaries between CSI-RS resource configuration and SBFD subbands, study the following options:

- Option 1: UE does not expect to be configured with a CSI-RS resource which overlaps with SBFD subband boundaries.

- Option 2: For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid.

- Option 3: Introduce new configurations of CSI-RS resource with finer granularity to ensure aligned boundaries between CSI-RS resource and SBFD subbands.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE (option2), Sony, IDC (Option 2 with modification), Intel (option 2), Spreadtrum (without Option 3), CMCC(option2), Huawei, HiSilicon, LG Electronics, Sharp, Ericsson (prefer Option 2), Lenovo, Qualcomm, WILUS (Option 2), NEC (prefer Option-2) , DOCOMO, Fujitsu, CATT, FGI, Nokia, NSB (need update for option 2) |
| Not support | Xiaomi, Samsung, vivo |

| Company | Comments |
| --- | --- |
| xiaomi | Whether guard band can be used or not is not concluded yet. In order to leave this issue open, we propose the following modification on option 2: - Option 2: For a CSI-RS resource which overlaps with SBFD subband boundaries, only at least CSI-RS resources within DL subband(s) are valid. - FFS: CSI-RS resources within guard band |
| IDC | Support the Option 2 in the proposal and agree with Xiaomi comment. |
| Samsung | We can support the FL proposed agreement for Option 1 and 2 but not for Option 3. The frequency occupancy of the CSI-RS resources is configurable in integer multiples of 4. We do not see meaningful potential for improved TP/SE if CSI-RS cannot be configured and CSI not reported on 1-3 RBs left/right of the SBFD UL SB. |
| Intel | Same single solution can be applied for both issues for proposal 1-14 and 1-15, i.e., option 2-2 in proposal 1-14 （option 2 in proposal 1-15）. |
| vivo | We share similar comments as Samsung. In addition, even if the finer granularity is introduced, we are not sure Option3 can resolve the issue completely. |
| CMCC | Regarding the FFS added by Xiaomi, we are not sure whether this discussion also applies for PDSCH or not. For option 1, it is conflict with option 2-2 in proposal 1-14. |
| Huawei, HiSilicon | Option 1 is preferred. See our comments for FL’s proposal 1-14. |
| LG Electronics | Generally fine with the options for study. |
| Ericsson | We don’t think it is motivated to increase the granularity of CSI-RS configuration (Option 3), so we think this option should be removed. Regarding Option 2, this seems the most natural, but we are confused about the FFS added by Xiaomi. Why would resources inside a guardband be valid? The whole point of a guard band is that no transmissions or receptions are expected within the guards. We think Option 1 is wasteful – why not use all RBs in a DL subband. We think it is better to follow analogous principle as partial RBGs for PDSCH to fully utilize all RBs. |
| FGI | Support Option 2. |
| Moderator | Several companies commented to remove Option 3 and the proposal is updated accordingly. Several companies explicitly support Option 2 and we can also check whether Option 2 is agreeable. Proposed Agreement: For unaligned boundaries between CSI-RS resource configuration and SBFD subbands, study the following options: - Option 1: UE does not expect to be configured with a CSI-RS resource which overlaps with SBFD subband boundaries. - Option 2: For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid. - Option 3: Introduce new configurations of CSI-RS resource with finer granularity to ensure aligned boundaries between CSI-RS resource and SBFD subbands. |
| Nokia, NSB | We are OK with the changes proposed by Xiaomi. |

Proposal 1-16

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS:

    - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

      - Option 1-1: One CSI reporting for SBFD/non-SBFD symbols is associated with one CSI-RS restricted to SBFD/non-SBFD symbols only;

      - Option 1-2: One CSI reporting for SBFD/non-SBFD symbols is associated with one CSI-RS which can be in SBFD symbols and non-SBFD symbols. For CSI reporting for SBFD/non-SBFD symbols, CSI report is derived based on CSI-RS in SBFD/non-SBFD symbols only.

    - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

      - Option 2-1: One CSI reporting for SBFD symbols and non-SBFD symbols is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived for SBFD symbols and for non-SBFD symbols based on CSI-RS in SBFD symbols and non-SBFD symbols respectively.

      - Option 2-2: One CSI reporting for SBFD symbols and non-SBFD symbols is associated with one CSI-RS which can be in SBFD symbols and non-SBFD symbols. CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

    - FFS impact on UE CSI processing and reporting timeline

Option 1-1 can be supported according to existing specification by gNB configuration to ensure that the CSI-RS associated with one CSI reporting is confined to either SBFD symbols or non-SBFD symbols. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type.

Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, TCL(prefer option 1), ZTE, Sony, Samsung, IDC, Intel, Spreadtrum (with change for Option 1), vivo, CMCC, Huawei, HiSilicon,OPPO, Sharp, Lenovo, Qualcomm, WILUS , NEC (prefer Option1-2) , DOCOMO, Fujitsu, CATT, FGI, Nokia, NSB |
| Not support | Ericsson (clarification needed) |

| Company | Comments |
| --- | --- |
| ZTE | Overall, we are fine with this direction. From our perspective, separate CSI report for SBFD symbols and non-SBFD symbols are beneficial to network scheduling. In this case, network can obtain the CQI/CLI/etc... info for slots without CLI and for slots with CLI. Hence the network can apply different scheduling mechanisms (e.g., AMC) for different slots |
| xiaomi | We prefer to reuse the current mechanisms. If the current mechanism is sufficient, we don’t see strong motivation to further enhance CSI reporting procedure. Clarification question: - For option 1-2: One CSI reporting is calculated based on single CSI-RS resource which can across SBFD symbol and non-SBFD symbol. Based on the description, it seems that a CSI-reporting happens on SBFD symbol is calculated based on the CSI-RS transmitted on SBFD symbol, and a CSI reporting happens on non-SBFD symbol is calculated based on the CSI-RS transmitted on non-SBFD symbol. Is this the correct understanding? - For option 2-1: One CSI reporting is associated with two CSI-RS resource which are located on SBFD symbol and non-SBFD symbol respectively. As the measurement is conducted only based on SBFD symbol or non-SBFD symbol, it means a CSI-reporting consists of two measurement results. Is this the correct understanding? |
| Sony | Since both options can be done with existing specs, why do we need to discuss this? |
| IDC | We support in principle Option 1-2 and Option 2-2 for further considerations, where both are based on one CSI-RS across SBFD and non-SBFD symbols which allows flexible and efficient CSI-RS resource management. Regarding CSI reporting configuration aspect (Option 1 vs Option 2), we think both options are feasible and better for further study. |
| Intel | We support option 1 to enable accurate CSI for SBFD and non-SBFD respectively and the design is simple and clean for both gNB and UE side. |
| Spreadtrum | For Option 1, one additional change is how to determine one CSI reporting is for SBFD symbols or non-SBFD symbols. |
| CMCC | We support to list all the solutions for further study. It is pre-mature to say there is no spec impacts. |
| Huawei, HiSilicon | Option 1-1 is preferred. |
| Ericsson | It is good to discuss options, but some further refinement may be needed. I find it difficult to differentiate between the options. I think some of the confusion comes with the word “same” used to describe Option 2. Our understanding of Option 2-2 is that it would be a way for the network to obtain separate CSI reports for SBFD and non-SBFD symbols using current specs, so in that sense it belongs under Option 1. Furthermore, Option 2-1 is another way the gNB could obtain separate CSI reports for SBFD and non-SBFD symbols, so it seems that this should also be under Option 1. Regarding Option 1-1, how can a single CSI-RS resource restricted to either SBFD or non-SBFD symbols be used to obtain separate CSI reports in SBFD and non-SBFD symbols? It seems like two resources are needed for this as in Option 2-1. Not sure we understand Option 1-2, especially the word “only” |
| QC | Generally oka, however, we have few comments: - Whether the NZP CSI-RS are for channel measurement only or for interference measurement as well? - Option 1-1 is the most simple and straightforward solution. - Options 1-2, 2-1 and 2-2 have may not work when gNB have different panels/antennas in SBFD and non-SBFD symbols which affect #CSI-RS ports, codebook, etc. Suggest to add a subbulet, whether same CSI-RS can be used across SBFD and non-SBFD symbols considering possible different panels and number of CSI-RS ports in both symbol types. |
| CEWiT | In our understanding, both option 1 and 2 can be implemented by the gNB using the current specification. So, we are not clear regarding the motivation behind studying this. Instead, we feel that it might be beneficial to discuss scenarios where the CSI-RS resources overlaps both SBFD and non-SBFD symbols and its impacts on the UE reporting. |
| FGI | Option 1 is preferred as measurement results with or without CLI can be obtained separately. |
| Moderator | @xiaomi, for option 1-2, if the CSI report is for SBFD symbol, UE should report CSI based on the measurement of CSI-RS in SBFD symbol and the same applies for non-SBFD symbol. For option 2-1, yes. @Ericsson, then what is same CSI reporting for SBFD symbols and non-SBFD symbols in your view? My understanding of Option 1 is that for a certain CSI report, only CSI report for SBFD symbols or non-SBFD symbols is reported. For Option 2, CSI report for both SBFD symbols and non-SBFD symbols can be reported for a CSI report simultaneously or in a TDM manner. For Option 1-1, two CSI reporting are configured for SBFD and non-SBFD symbols respectively. For Option 1-2, please see my reply to xiaomi above. |
| Nokia, NSB | We support further study on this, also we prefer option 1. We need to mention that CSI will be impacted by both CSI-RS and CSI-IM (optional), which may also be needed to consider. |
| Ericsson 2 | @Moderator My understanding of “same” is that the UE prepares a single CSI report, and there is no differentiation between SBFD and non-SBFD symbols, i.e., the UE just reports whatever it measures. I do not share your understanding that two reports are multiplexed in the same reporting instance or in a TDM manner. To me, that is what “separate” means for Option 1. I think we need to come to common understanding on this point before making an agreement. |

Proposal 1-17

Proposed Agreement:

For unaligned boundaries between CSI reporting subband and SBFD subbands, study the following options:

- Option 1: UE does not expect to be configured with a CSI reporting subband which overlaps with SBFD subband boundaries.

- Option 2: For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s).

- Option 3: Introduce new configurations of CSI reporting subband with finer granularity to ensure aligned boundaries between CSI reporting subband and SBFD subbands.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE (option2), Sony, IDC (Option2), Intel, Spreadtrum (without Option 3) , CMCC(option 2), Huawei, HiSilicon, Sharp, Ericsson (with modification) , Lenovo, Qualcomm, WILUS (Option 2), NEC (Option-2) , DOCOMO, Fujitsu, CATT, FGI (option 2) , Nokia, NSB (prefer Option 2) |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Xiaomi | Regarding option 2, considering there is no consensus whether guard band can be used for DL reception or not, it is not suitable to preclude guard band for CSI-RS transmission. Hence we have the following modification: - Option 2: For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding unavailable CSI-RS resources for DL reception outside DL subband(s). |
| Samsung | Similar to FL proposal 1-15, we can support the FL proposed agreement for Option 1 and 2 but not for Option 3. We do not see meaningful potential for improved TP/SE if CSI cannot be reported on a few RBs left/right of the SBFD UL SB. |
| IDC | Option 2 is sufficient (without introducing new configurations). |
| Intel | Option 2 would be sufficient, which is well-aligned with option 2-2 in proposal 1-14. |
| vivo | Not sure the relation with proposal 1-15. Whether the decision for Proposal 1-17 can be derived naturally based on the outcome of proposal 1-15. |
| CMCC | Can be discussed together with proposal 1-15 and the same solutions can be applied to both issues. |
| Huawei, HiSilicon | Option 2 is preferred. |
| Ericsson | Consistent with our comments on Proposal 1-15, we don’t think it is motivated to increase the granularity of CSI reporting granularity (Option 3), so we think this option should be removed. Regarding Option 2, this seems the most natural, but we are confused about the need for the edits from Xiaomi. The FL proposal seemed clear already We think Option 1 is wasteful – why not use all RBs in a DL subband. We think it is better to follow analogous principle as partial RBGs for PDSCH to fully utilize all RBs. |
| Moderator | Similar as Proposal 1-15, Option 3 is removed. And we can also check whether Option 2 is agreeable. Proposed Agreement: For unaligned boundaries between CSI reporting subband and SBFD subbands, study the following options: - Option 1: UE does not expect to be configured with a CSI reporting subband which overlaps with SBFD subband boundaries. - Option 2: For a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s). - Option 3: Introduce new configurations of CSI reporting subband with finer granularity to ensure aligned boundaries between CSI reporting subband and SBFD subbands. |

Proposal 1-18

Proposed Conclusion:

For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed.

Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type.

The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2, which requires additional specification efforts.

Option 1 may increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped.

|  | Company |
| --- | --- |
| Support | New H3C, xiaomi, Samsung, IDC(Option 2), Intel, ITRI , vivo, Huawei, HiSilicon, Ericsson (with modifications), Panasonic, Lenovo, NEC (with modification) , DOCOMO, CATT, CEWiT, FGI |
| Not support | Sony, Sharp |

| Company | Comments |
| --- | --- |
| ZTE | We propose to add the following for option 2: Option 2 may decrease the transmission/reception latency. |
| xiaomi | Fine with the conclusion. |
| Sony | Why is there only conclusion for Option 1? I think some companies want to consider Option 2. |
| IDC | We support in general Option 2, and we share similar comments as ZTE and Sony. |
| Intel | We are also fine with the suggestion from ZTE. Besides, in our view, different option may be applied to different signal/channels, for DG/CG, with or without repetition, etc. |
| CMCC | Support to add the conclusion for Option 2. |
| Huawei, HiSilicon | Both option 1 and 2 can be supported. And which one is used by be selected by scheduler. |
| LG Electronics | Generally fine with the proposal to make an agreement for the observation. Further clarification seems necessary for option 1. In my understanding, specification efforts for modifying the rule of dropping and/or postpone are required for option 1. |
| Sharp | Option 1 cannot be achieved by gNB configuration or scheduling. Considering repetitions, how does it avoid allocating one transmission occasion from non-SBFD region? |
| Ericsson | Agree with Sony that the conclusions need to cover Option 2 as well. For Option 2, we don’t think it necessarily requires additional specification efforts, hence that statement can be removed. For example, consider PDSCH with repetition where the FDRA indicates the same set of RBs in the frequency domain in both SBFD and non-SBFD symbols. It seems like that would be supported without spec enhancements. |
| Panasonic | We agree with Huawei that both option 1 and 2 can be supported. |
| QC | Generally fine with the proposal as a high-level trade-off between option 1 (SBFD only or non-SBFD only) or option 2 (across both SBFD and non-SBFD). However, as other companies pointed out, the pros of option-2 are missing in the conclusion. Option 2 enables gNB to leverage SBFD coverage and latency benefits, e.g UL repetition across SBFD and non-SBFD symbols improves UL coverage and improved UL duty cycle. |
| WILUS | No specification impact should be a baseline assumption on Option 2. It’s up to gNB whether to schedule or not across SBFD symbols and non-SBFD symbols. Different FDRA, power control, and beam/spatial relations across SBFD symbols and non-SBFD symbols are next step enhancement, and motivations should be clarified. |
| NEC | For Option-1 another point to note is that Option-1 would require higher signaling overhead as compared to Option-2 for semi-static channels because in Option-1 for each physical channel that needs to be supported in SBFD as well as non-SBFD two separate full configurations would need to be provided. While for Option-2, only delta configuration change needs to be signaled to enable Tx/Rx between SBFD and non-SBFD slots |
| Moderator | The advantages of Option 2 are added based on the comments. Proposed Conclusion: For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed. Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type. The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2, which requires additional specification efforts. Option 1 may increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may reduce the transmission/reception latency and improve coverage. |
| Nokia, NSB | As it is mentioned option 1 may increase the transmission/reception latency, we propose to add “option 2 may increase transmission/reception resource and decrease the transmission/reception latency” as it is benefit from option 2. |

Proposal 1-19

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study the frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS:

- Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

  - Option 1-2: Single FDRA configuration/indication for SBFD slots and non-SBFD slots

- Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

- Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, xiaomi, Sony, IDC, Intel, ITRI, Spreadtrum (with change for Option 2), vivo, CMCC, Sharp, Ericsson, Panasonic, Lenovo, WILUS, NEC, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| ZTE | We are basically fine with this proposal. From our perspective, we don’t think separate FDRA configuration/indications for SBFD slots and non-SBFD slots are needed due to the large overhead. To us, one FDRA configuration/indication plus one offset is sufficient. For example, if the UL transmission is transmitted in the UL subband and network prefers to move it to the up side of bandwidth to avoid frequency fragmentation, then an offset plus the previous FDRA indication is sufficient. One question to FL, is this kind of solution included in option 1-2? If no, we suggest to add another option 1-3 for this. |
| Samsung | We are somewhat concerned about the specification impact and DCI overhead associated with Option 1-1. We think that single FDRA and a configurable offset would be sufficient. Similar to ZTE comments, we should capture this explicitly under Option 1. Option 3, we should remove “postpone”. |
| IDC | Support the proposal in general. Agree with ZTE to add the option on considering an offset value which saves the signaling overhead compared to having separated FDRA indications. |
| Intel | Support the proposal. In our understanding, option 1-1 can be useful for higher-layer configured signals without DCI, e.g., CG PUSCH type-1, CSI-RS, SRS, etc. option 1-2 can be useful for DG case. Option 1-2 includes options, such as single FDRA is derived for SBFD and non-SBFD according to UL subband and UL BWP, single FDRA is derived for SBFD and non-SBFD with or without additional offset. For option 3, we think ‘postpone’ should be kept, considering available slot counting. |
| Spreadtrum | For Option 2, one operation is to do exclusion the RBs that cannot be used for Tx or Rx, in addition to RM or puncture. So : - Option 2: Perform rate matching or puncturing on the RBs or exclusion the RBs outside DL/UL subbands for DL/UL channels/signals. |
| Huawei, HiSilicon | Different channels/signals may have different designs. We prefer to have separate discussions and focus on PUSCH and PDSCH first. - For PDSCH, both option 1 and 2 should be studied. - For option 1, it has more flexibilities to allocate PDSCH away from UL subband to reduce UE-UE CLI. - For option 2, it can be used for UEs which has less UE-UE CLI. - For PUSCH, Option1 and Option 3 are preferred. |
| LG Electronics | Is the intention of the proposal 1-19 to study the options for each physical channel/signal? For example, configuration of FDRA and rate-matching can be applied for PDSCH/PUSCH, but these can’t be applied for CSI-RS, PUCCH and SRS. |
| Ericsson | Support |
| Panasonic | We also think different channels/signals may have different designs. |
| QC | Few comments on the proposal. - For option 1 (different FRDA), is the intention to have different start-RBs at SBFD and non-SBFD or to have both different start-RBs and different lengths (#RBs)? - For option 2 (RM/puncturing): concerns on RM for UL, which is new L1 behaviour. - For option 3: Does postponing refers to Slot being not available and UE needs to look for next available slot? |
| Moderator | Option 1-3 is added per ZTE and Samsung’s comments. For the questions from Qualcomm, 1) I assume Option 1-1 has separate starting RB and number of RBs. 2) understand. 3) yes. Proposed Agreement: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Single FDRA configuration/indication for SBFD slots and non-SBFD slots - Option 1-3: one FDRA configuration/indication and one offset - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. |
| Nokia, NSB | We support further study on this. There should be a rule to make it work but both the rule should consider both performance and complexity. |

Proposal 1-20

Proposed Agreement:

Study whether/how to support a search space configured for SBFD and non-SBFD symbols and one associated CORESET overlaps with RBs outside DL subband(s) in some SBFD symbols, considering the following options for potential enhancements:

- Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols.

- Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s).

- Option 3: Drop PDCCH candidates which REGs outside DL subband(s).

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Xiaomi(with modifications), Sony, IDC, Intel , vivo, CMCC, Huawei, HiSilicon, Sharp, Panasonic(with modification) , Lenovo, Qualcomm, NEC, DOCOMO, Fujitsu, CATT, CEWiT, FGI, Nokia, NSB (with update) |
| Not support | Samsung, Spreadtrum, Ericsson |

| Company | Comments |
| --- | --- |
| xiaomi | We are OK with the proposal. We have the following comments on the listed options: Option 1 will complicate operation on PDCCH mapping at both gNB side and UE side as additional mapping procedure for SBFD slot is needed. Actually there is no difference from introducing another CORESET for SBFD symbol. Option 2 define rate matching or puncture for PDCCH, it is different from current PDCCH operation which is not expected. Option 3 is actually quite similar as the current specification hence has least standard efforts. Especially we are discussing the enhancement on RMR for PDSCH, once we achieve consensus there, the following current behavior can be directly reused: The frequency resources occupied by CORESET are indicated via a 45-bit bitmap and the granularity is 6 continuous RBs. gNB has full power to avoid overlapping between CORESET and UL subband. TS38.213 If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate. Based on the above analyses, we have the following modification: Proposed Agreement: Study whether/how to support a search space configured for SBFD and non-SBFD symbols and one associated CORESET overlaps with unavailable RBs RBs outside DL subband(s) in some SBFD symbols, considering the following options for potential enhancements: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: Drop PDCCH candidates which REGs outside DL subband(s) UE does not expect to monitor the PDCCH candidate if it is mapped to one or more Res that overlaps with unavailable REs. |
| Sony | We also need to consider interleaved and non-interleaved CCE. If we puncture REGs in interleaved CCE we may end up with very few PDCCH candidates. It may be beneficial to convert an interleaved CCE CORESET to a non-interleaved one to reduce the number of invalid CCEs. |
| Samsung | We think that the gNB can configure the CORESETs (and CORESET#1) flexibly as by existing Rel-15 such that there is no need to study such PDCCH enhancements for SBFD-aware UEs. |
| Spreadtrum | Agree with Samsung, CORESET and PDCCH can reuse the legacy configurations. No need to do extra operation for SBFD. |
| CMCC | Support the enhancement on CORESET. The number of CORESETs UE supporting is limited, e.g., mandatary two CORESETs in FR1, if two CORESETs are configured for normal DL slots and SBFD slots respectively, the scheduling flexibility will be reduced. |
| Huawei, HiSilicon | For option 1, there are two alternatives - Option 1-1: Separate valid resources for CORESET in SBFD slots and non-SBFD slots, respectively - Option 1-2: One adaptive valid resource for CORESET across SBFD slots and non-SBFD slots, based on DL/UL subband configuration. In addition, option 3 may have an issue when interleaving is used. For example, the candidate REGs may be allocated in UL subband after interleaving; it will lead reduce the number candidate REGs, thus reducing the capacities of CORESET. |
| OPPO | Due to UL subband is configure semi-statically, CORESET configuration is easy to avoid overlapping with UL subband. So reasonable CORESET configuration by implementation is enough. The benefit from spec effort is not clear. So we prefer to add option 4: Implementation solution to avoid CORESET overlapping with unavailable resource. |
| LG Electronics | Generally fine with the study about the CORESET. |
| Ericsson | We think that spec enhancements are not needed, and thus we do not support the proposal. There is significant flexibility the FDRA for a CORESET, hence several implementation based solutions exist: (1) gNB can avoid configuration a search space that occurs in both SBFD and non-SBFD symbols; or, (2) restrict the RB allocation to a common set of RBs that occur in both SBFD and non-SBFD symbols, e.g., restricting to single DL subband or non-contiguous allocation in 2 DL subbands. |
| Panasonic | In addition to these options, we would like to add Option 4 as follows. For option 4, search space is dropped when the associated CORESET overlaps with RBs outside DL subband(s). When search space is dropped, another search space can be used if the associated CORESET does not overlapped with RBs outside DL subband(s) and the number of BDs/CCEs does not exceed the limit. Search space dropping for PDCCH overbooking is specified in the current specification. By using search space dropping, similar functionality to option 1 can be supported without additional complexity pointed out by Xiaomi. Proposed Agreement: Study whether/how to support a search space configured for SBFD and non-SBFD symbols and one associated CORESET overlaps with RBs outside DL subband(s) in some SBFD symbols, considering the following options for potential enhancements: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: Drop PDCCH candidates which REGs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) |
| QC | Fine with listing options for study purpose. The suggested schemes have different pros/cons (e.g PDCCH reliability, PDCCH capacity, UE complexity, etc). |
| NEC | We agree and support with the intention. However, these solutions are not expected to work for the case of CORESET0 which also needs to be monitored by the legacy UEs (which do not have knowledge of SBFD and hence will try to receive DCI with legacy procedure). So, additional note needs to be indicated Note: Whether/how to address the scenario of CORESET0 and CORESETs associated with common search spaces |
| FGI | We share similar concern on Option 1 due to limited number of CORESETs supported by the UE. |
| Moderator | I understand some companies do not see the need of any enhancements. Note we have ‘whether’ in the proposal. Option 3 is updated based on comments from xiaomi and Option 4 is added based on comments from Panasonic. Proposed Agreement: Study whether/how to support a search space configured for SBFD and non-SBFD symbols and one associated CORESET overlaps with RBs outside DL subband(s) in some SBFD symbols, considering the following options for potential enhancements: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: DropUE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REswhich REGs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) |
| Nokia, NSB | We are ok with the proposal. We prefer Option 1 and Option 2 (rate matching). We propose to add “Only PDCCH candidates with all resource as valid for PDCCH could be for UE blind detection”. |

Proposal 1-21

Proposed Agreement:

Study whether a SBFD-aware UE is preconfigured by higher layer signalling a transmission direction to either receive or transmit in a SBFD symbol.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Spreadtrum, Sharp, CATT, Nokia, NSB |
| Not support | Xiaomi, Sony, Samsung, IDC, Intel , vivo, Huawei, HiSilicon, Ericsson, Lenovo, WILUS, NEC, Fujitsu, CEWiT, FGI |

| Company | Comments |
| --- | --- |
| ZTE | We are fine with the intention of this proposal. However, we think clarification on “higher layer signaling” is needed. There are at least two different interpretations: - “higher layer signalling” refers to SPS/CG-PUSCH/CSI/PDCCH configuration etc. If there is only DL semi-static configured transmission, the symbol is for DL transmission; if there is only UL semi-static configured transmission, the symbol is for UL transmission. - “higher layer signalling” refers a specific configuration that is used to indicate the DL or UL transmission direction for each symbol/slot. |
| Xiaomi | For a SBFD aware-UE, if a UL subband is configured on DL symbols, it is capable to either transmit uplink or receive downlink based on scheduling or configuration. We don’t see the necessity to introduce such kind of restriction. If such restriction is really needed, a semi-static TDD is sufficient. |
| Sony | It isn’t clear what this proposal is for. This seems to defeat the purpose of having SBFD if the transmission direction is semi-statically configured by the gNB and overly restrictive. |
| Samsung | This proposal is very unclear to us. If the gNB configures the SBFD-aware UE with an SBFD UL subband on an SBFD symbol, the intent is that the gNB can then either schedule UL transmission from the UE or DL reception by the UE on that symbol according to traffic and needs. If scheduling the SBFD-aware UE in one or more system-wide SBFD slots is not desired, the gNB can choose to not configure the UE with an SBFD UL subband on these symbols. |
| IDC | We have similar concerns by Xiaomi and Sony. It should be determined in a dynamic way, e.g., by a scheduling DCI, a group-common DCI, etc. If not, a semi-static TDD is sufficient. |
| Intel | This proposal is unclear to us. We’d like to better understand the intension of restricting to one direction in a SBFD symbol. |
| vivo | Is it for RRC UL vs. RRC DL collision case? For RRC DL or UL vs. dynamic UL or DL collision, such higher layer signaling is not needed. |
| CMCC | We are fine with the intention of this proposal to determine the link direction of a given SBFD symbol, but other solutions, e.g., scheduling DCI or L1 signalling are not precluded. |
| Huawei, HiSilicon | In our understanding, this higher layer signaling is not required for SBFD since it is not required to determine the transmission direction of flexible symbols for dynamic TDD. |
| OPPO | “Preconfigured” is confusing. Does it mean that the transmission direction configured by higher layer signaling can be reconfigured, even overridden by dynamic? |
| LG Electronics | Fine with the study. |
| Ericsson | If an UL subband is configured, the UE needs to be prepared to either transmit in the UL subband or receive in the DL subband (but not simultaneously). This is the nature of SBFD, and to pre-configure a direction for UEs would remove the claimed benefits of SBFD. Significant scheduling flexibility would be lost. |
| Lenovo | The proposal is unclear and needs further clarification. |
| QC | It is better to discuss how would UE determine transmission or reception in SBFD symbols via different options. - Option 1: based on RRC signalling - Option 2: based on dynamic scheduling. |
| WILUS | The intention of the Proposal is unclear to us. If it’s to handle the collision between higher layer configured DL reception and higher layer configured UL transmission, UE behaviors can be studied without pre-configuration. |
| NEC | We think that transmission direction information can be implicitly determined by the UE based on pre-defined set of rules rather than explicit signaling. For e.g. UE monitors DL during an SBFD symbol if configured with a DL transmission or if no UL transmission is scheduled for UE during the symbol. We can further discuss how to handle the collision scenarios for more clarity but motivation of explicit signaling is not sufficient. |
| DOCOMO | We feel the intention of configuring/dynamically indicating direction in a SBFD symbol is to handle DL and UL collision in the symbol (e.g. collision cases in section 3.1.3)? |
| Fujitsu | We don’t understand the intention of this proposal. Is it related to the flexible subband? |
| CEWiT | The intention of this proposal is not clear. Why are we restricting to only “higher layer signalling” for setting a transmission direction to either receive or transmit in a SBFD symbol? |
| FGI | The benefit of restricting one transmission direction is not clear. |

Proposal 1-22

Proposed Agreement:

SBFD-aware UE does not expect to be dynamically scheduled UL transmission in UL subband and DL reception in DL subband(s) in the same SBFD symbol.

    - FFS the case that the time between UL transmission and DL reception is less than Tx-Rx or Rx-Tx switching time

|  | Company |
| --- | --- |
| Support | New H3C, TCL, CMCC, Huawei, HiSilicon,OPPO, Qualcomm, WILUS, NEC, CATT, CEWiT, FGI |
| Not support | Xiaomi, Sony, Samsung, IDC, ITRI, LG Electronics, Sharp, Ericsson, Nokia, NSB |

| Company | Comments |
| --- | --- |
| ZTE | We see some benefits to support collision handling for dynamic DL and dynamic UL especially in case of different priorities. But it this proposal is the majority view, we can go with this proposal. |
| Xiaomi | Theoretically, gNB has full power to avoid collision between dynamic DL and dynamic UL on the same symbol. Therefore, it may be a reasonable hypothesis that UE doesn’t expect gNB dynamically schedules uplink and downlink at the same time. On the other hand, dynamic downlink channel and dynamic uplink channel may have different priorities, i.e. one is associated with eMBB while the other is associated with URLLC. gNB may firstly schedule a PUSCH or PDSCH carrying eMBB traffic and later schedule a PDSCH or PUSCH carrying URLLC on the same symbol. In this sense, it make sense to consider collision between dynamic downlink and dynamic uplink with different priority respectively. |
| Sony | This is overly restrictive and unclear what the benefit is for. It also seems to defeat the purpose of SBFD. |
| Samsung | We don’t think a Rel-18 SID agreement is needed for that. SBFD-aware UEs as by SID mandate are HD like the legacy TDD UEs. If needed, such a collision / error case or need to update the existing 38.213 Section 11 should be discussed during the WID stage. |
| IDC | To allow flexible scheduling and reduce scheduling complexity, discussions on collision handling seem to be better and desirable, which are not urgent and can be discussed in WI phase. |
| Intel | We’d like to clarify, whether the dynamic scheduled UL transmission/DL reception includes the case for repetition, multi-PDSCH/PUSCH, TBoMS ? |
| vivo | Is the FFS for back-to-back scheduling that the DL and UL is not in the same SBFD symbol, but in adjacent symbols? In addition, any reason for the FFS given the collison is caused by dynamic scheduling, gNB should ensure the transition time? |
| LG Electronics | Seems not necessary to make an agreement on the issue of the dynamic scheduling for both directions. gNB may not provide this kind of scheduling information to the SBFD-aware UE. |
| Sharp | Note is about Tx-Rx switching. However, the main bullet is for Tx-Rx collision in one symbol. Note should be removed. |
| Ericsson | Why is this proposal needed? According to the SID, the scope of the SI is HD UEs, so it would be an error case if the gNB scheduled the UE to simultaneously transmit and receive. |
| Lenovo | Collision handling in the case of UL transmission in UL subband and DL reception in DL subband(s) in the same SBFD symbol should be studied. |
| QC | This scenario should be an error case and avoided by gNB scheduling. |
| DOCOMO | Fine with the principle, but we share similar view as Intel that the case of repetition, multi-PDSCH/PUSCH, TBoMS should be clarified. |
| Nokia, NSB | We do not think there should always be a strong limitation. It is beneficial to also study the case in which dynamically scheduled DL and UL have different priorities, therefore we do not support the proposal. |

Proposal 1-23

Proposed Agreement:

SBFD-aware UEs may be configured with UL transmission in UL subband and DL reception in DL subband(s) in the same SBFD symbol.

    - FFS how to handle such collision case

    - FFS the case that the time between UL transmission and DL reception is less than Tx-Rx or Rx-Tx switching time

    - Configured UL transmissions at least include CG PUSCH, configured PUCCH/SRS

    - Configured DL receptions at least include PDCCH, SPS PDSCH, configured CSI-RS

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, xiaomi, Sony, IDC, Intel, ITRI, CMCC, Huawei, HiSilicon, Sharp, Lenovo, Qualcomm, WILUS, NEC, DOCOMO, CATT, CEWiT, FGI, Nokia, NSB |
| Not support | Samsung, Ericsson |

| Company | Comments |
| --- | --- |
| Samsung | We don’t think a Rel-18 SID agreement is needed. We consider this WID-level signaling discussions. |
| Intel | We think high-level discussion can be pursued in SI, while details on the rule to determine the prioritization is to be discussed in WI. |
| vivo | Same clarification as for dynamic case in proposal 1-23 for the transition time |
| OPPO | The collision in proposal can be avoided by reasonable gNB configuration. |
| LG Electronics | Fine with the study. In my understanding, the intention of the proposal 1-23 is to study the case of collision between configure UL(/DL) and DL/UL. |
| Ericsson | We don’t support defining precise collision handling rules in SI phase. This can be left for WI phase. It is sufficient to identify collisions that can potentially happen, but not to define rules. |

Proposal 1-24

Proposed Agreement:

For a SBFD-aware UE, if a configured transmission/reception in UL/DL subband(s) collides with a scheduled reception/transmission in DL/UL subband(s) in the same SBFD symbol, scheduled reception/transmission is prioritized.

    - FFS the case that the time between UL transmission and DL reception is less than Tx-Rx or Rx-Tx switching time

    - Configured UL transmissions at least include CG PUSCH, configured PUCCH/SRS

    - Configured DL receptions at least include PDCCH, SPS PDSCH, configured CSI-RS

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, IDC, ITRI, CMCC, Huawei, HiSilicon,OPPO, Sharp Lenovo (with additional FFS modification), Qualcomm, NEC, DOCOMO, CATT, CEWiT, FGI |
| Not support | Xiaomi, Samsung, Ericsson, Nokia, NSB |

| Company | Comments |
| --- | --- |
| xiaomi | It is too arbitrary to assume that dynamic scheduling must have high priority once direction collision occurs. There is a possibility that the dynamic channel and the semi-static channel may have different priorities in nature. For example, gNB schedules a PUSCH with R=4 while the SPS periodicity equates two, which can be explained in Error: Reference source not found. On slot#0, SPS PDSCH collides with PUSCH repetition. If SPS PDSCH is configured with a high priority, i.e. its harq-CodebookID points to the HARQ codebook with high priority, it is hard to say the dynamic PUSCH should be prioritized. |
| Samsung | Similar to the other FL proposal on collision handling, we don’t think a Rel-18 SID agreement is needed. We consider this WID-level signaling discussions. |
| IDC | We support the proposal in general, but we are open for further discussions in details. |
| LG Electronics | Clarification between proposal 1-23 and proposal 1-24 seems necessary. |
| Ericsson | We don’t support defining precise collision handling rules in SI phase. This can be left for WI phase. It is sufficient to identify collisions that can potentially happen, but not to define rules. |
| Lenovo | Generally, ok with the scheduled reception/transmission having higher priority. But the priority also depends on the time gap between the scheduled reception in the DL subband and the configured transmission in the UL subband. So, need to the following FFS: FFS whether the priority depends on the time gap between the scheduled reception in the DL subband and the configured transmission in the UL subband. |
| QC | Dynamic scheduling should be prioritized over semi-static configuration. |
| WILUS | We are generally fine with the proposal. Regarding that collision occurs in symbol-level (not RE-level), enhancement can be studied in addition to prioritize scheduled Rx/Tx. For example, configured PUSCH can be rate-matched around symbols colliding with scheduled DL signal/channel to guarantee both DL Rx and UL Tx at least in different symbols. |
| Nokia, NSB | We have same view as Ericsson that we define detail collision rule in WI phase. |

Proposal 1-25

Proposed Agreement:

Support PRACH and Msg3 transmissions in UL subband for UEs can potentially reduce the latency of random access, reduce the collision probability, improve the coverage of PRACH and Msg3 and avoid the UL resource fragmentation in full UL symbols.

In order to support PRACH and Msg3 transmissions in UL subband for SBFD aware UEs in RRC_IDLE and RRC_INACTIVE states, SBFD time and frequency locations need to be configured in SIB.

Further study potential issues to support PRACH and Msg3 transmissions in UL subband and solutions to resolve the issues if any.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, Samsung, IDC, Intel, , Huawei, HiSilicon, LG Electronics, Sharp, Qualcomm, WILUS, NEC, DOCOMO, CATT, Nokia, NSB |
| Not support | Vivo, Ericsson, Fujitsu |

| Company | Comments |
| --- | --- |
| Xiaomi | It is too weak to be an agreement. We don’t think we need such an intermediate agreement. Instead, we can directly discuss the solutions for RRC_IDLE and RRC_INACTIVE state. |
| IDC | Support in principle the proposal for latency reduction benefits by indicating the SBFD time and frequency configuration during initial access (e.g., via SIB) and to transmit PRACH and Msg3 in UL subband of SBFD symbols. |
| vivo | Support PRACH and Msg3 transmissions in UL subband may be beneficial, but it incurs huge specification impacts for RO validation, SSB-to-RO association, co-existence with legacy UEs and SBFD non-aware UEs. High priority should be given to RRC connected mode UEs. So, the study to support PRACH and Msg3 transmissions in UL subband should be deprioritized in Rel-18 SI. |
| CMCC | We are fine to study the support PRACH and Msg3 transmissions in UL subband. BUT in addition to the SBFD time and frequency locations signalling issue, the inter-UE CLI handling should also be considered, for example, if RO is allowed to be configured in SBFD symbols, when the aggressor UE transmits PRACH, the inter-UE inter-subband CLI will impact the DL reception of the victim UE. For contention-base RACH, due to the uncertainty of UE transmitting PRACH, it is hard or not possible for gNB to mitigate the inter-UE inter-subband CLI. But for contention-free RACH, the PRACH transmission is controlled by gNB and the inter-UE inter-subband CLI can be avoided and mitigated by gNB’s proper scheduling. |
| OPPO | Uncertain PRACH and Msg3 transmissions in UL subband wastes UL resource and also impacts DL scheduling from gNB, e.g. gNB may reduce DL scheduling in SBFD symbol and increase DL latency. In addition, the number of Rx antenna in SBFD symbol may be smaller than the number of Rx antenna in non-SBFD symbol,PRACH and Msg3 coverage will decrease and the failure number of initial access will increase, which also increases initial access latency. Lastly, more collision cases, e.g. Collision with PRACH/Msg3 needs to be considered in P1-22,P1-23,P1-24 |
| LG Electronics | Fine with the proposal. |
| Ericsson | We have strong concern on this proposal. We don’t agree to support SBFD operation for UEs in IDLE/INACTIVE mode, and making an agreement to support this in SI is improper. This can be seen by the final bullet in the proposal which says study potential issues and solutions to resolve them. It is backwards to agree to support something, and then study the issues. We don’t agree that latency during initial access is a bottleneck, furthermore, there are no requirements on initial access latency to start with. We also don’t agree that there is a RACH capacity issue to solve. The primary focus of the study item should be UEs in CONNECTED mode; we think that handling IDLE/INACTIVE mode can and should be deprioritized. |
| Fujitsu | We have same views with vivo and Ericsson that there are critical issues to support PRACH Msg3 transmission in SBFD symbols on RRC_IDLE and RRC_INACTIVE states. The gNB cannot expect the timing when an SBFD UE transmits the PRACH in those state, consequently the gNB will be suffering to control the CLI. |

- gNB self-interference handling schemes

This section discusses gNB self-interference handling schemes for SBFD.

  - Summary of input contributions

Tx/Rx timing misalignment

BS self-interference related to timing and SCS was discussed in RAN4#104bis-e with the following agreement in R4-2217464.

## 4.1 BS self-interference related to timing and SCS Agreement: - For the BS self-interference issue related to timing and SCS of D and U for both BS and UE: - RAN4 understanding is that this issue will be studied in RAN1. - RAN4 has not evaluated the timing and SCS impacts to BS SI. RAN4 for now assumes they are negligible and do not impact RAN4 requirement work. - RAN4 will not consider this issue in the BS SI feasibility study if no request from RAN1.

This issue is discussed by many companies in previous meetings and in this meeting.

In legacy TDD system, a UE is provided a cell-specific TA offset NTA,offset. It can be provided in SIB1 and if not, UE assumes TA offset of 25600Tc or 39936Tc defined in Table 7.1.2-2 in 38.133.

The cell-specific TA offset is used to reserve enough time for UL/DL switching as illustrated below.

Figure 4-10: TA offset in legacy TDD [8]

For SBFD operation, gNB would transmit and receive simultaneously and the non-zero TA offset may cause inter-symbol/inter-slot interference as observed by Huawei, ZTE, Ericsson, CATT, Sony.

Furthermore, CATT and Ericsson provided some evaluation results in Error: Reference source not found and Error: Reference source not found respectively. CATT observed that the required SIC capability can be reduced by 4 dB by configuring NTA,offset=0  instead of NTA,offset=25600Tc. Ericsson observed that assuming no transmit side impairments, the misalignment creates a dramatic rise in interference received in the 20 MHz UL subband between the two 40 MHz DL subbands compared to the case of no misalignment. However, a very much reduced gap is observed in the presence of Tx side impairments and the combined effect of Tx side impairments and UL subband selection filter come very close to fully masking the misalignment effect.

Huawei proposed to configure NTA,offset=0.

ZTE, Nokia, observed that there may be potential backward compatibility issue since legacy UE may not support NTA,offset=0 in TDD system. In addition, with NTA,offset=0, the gap for UL-to-DL switching may disappear and the gap realized based on scheduling avoidance can be larger.

Alternatively, ZTE, Nokia proposed to consider two TA offset values for SBFD-aware UEs for UL transmissions in non-SBFD symbols and SBFD symbols. Samsung proposed that for SBFD-aware UEs, support configuration with different TA offset values or with multiple timing advance groups in the SBFD and non-SBFD slots. There can be potential overlap between UL transmission in a SBFD symbol and next UL transmission in a non-SBFD symbol. Nokia proposed to study how to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots.

Ericsson proposed that RAN1 should model practical transmitter and receiver components to enable a realistic assessment of the effect of UL/DL timing misalignment at the gNB.

Sony and Intel discussed larger TA value to align the symbol boundary for DL and UL with different symbol index. To be specific, Sony proposed to add a time alignment offset to the overall timing advance for UL transmission in SBFD slots, where time alignment offset is selected such that the 2nd or higher OFDM symbol of the UL transmission aligns with a potential DL interferer at the gNB and also provides at least NTX-RX at the UE between the end of an UL transmission and the start of a DL reception for UL to DL switching as shown below. Intel observed that at least one symbol overhead would be generated.

Figure 4-11: Adding TUL to the overall timing advance to align the 2nd OFDM symbol with the DL interferer [21]

Xiaomi thinks that TA enhancement is not necessary as it doesn’t resolve inter-slot interference issue.

Power control based solution

ZTE observed that the uplink transmissions in the UL subband are subject to different interference levels due to different frequency domain isolation between the uplink transmission and the DL subband and proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the different levels of inter-subband interference.

Nokia proposed that the potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

  - [Open] 1st round discussion

Proposal 2-1

Proposed Agreement:

Time misalignment between UL receptions and DL transmissions due to non-zero TA offset lead to increased interference assuming no transmit side impairments.

    - FFS the case with transmit side impairment

    - FFS whether/how to mitigate the interference increase

|  | Company |
| --- | --- |
| Support | TCL, ZTE (with comments), Sony (need clarification), Samsung, IDC, vivo, Panasonic, Qualcomm, NEC, Fujitsu, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Since there is only one FFS below, to be fair, the following change is proposed. Time misalignment between UL receptions and DL transmissions due to non-zero TA offset lead to increased interference at least for the case with assuming no transmit side impairments. - FFS the case with transmit side impairment - FFS whether/how to mitigate the interference increase |
| Sony | What does it mean by “side impairments”? |
| Huawei, HiSilicon | Support with some modifications: Time misalignment between UL receptions and DL transmissions when non-zero TA offset is configured which will due to non-zero TA offset lead to increased interference assuming no transmit side impairments. - FFS the case with transmit side impairment - FFS whether/how to mitigate the interference increase |
| LG Electronics | Fine with the study. But, clarification about the observation is necessary. In case of separation between DL/UL frequency subbands, the reduced self-interference from DL subband may affect to the UL subbands. The effect of TA offset (i.e., symbol boundary alignment between UL OFDM symbol and DL OFDM symbol) is not clear for further reduction of self-interference. |
| NEC | We agree with the issue. To address this issue, several companies have indicated configuring TA offset as 0 for UL transmission performed during SBFD symbols. However, as shown in (b) in below figure, we can see that keeping 0 TA offset also does not solve the problem as it allows no margin for transition from UL to DL for the gNB. Hence, to solve this problem, there is a need to provide a guard period after UL transmission in an SBFD symbol (in addition to keeping TA offset as 0) to allow sufficient margin time for gNB to switch its RF components from UL to DL appropriately. |
| Nokia, NSB | We think the main bullet is for observation. The FFSs are for study. We are fine with the study. |

- Inter-gNB and inter-UE CLI handling schemes

The guidance from Mr. Chair on discussions of CLI handling in AI 9.3.2 and AI 9.3.3 is as follows. Accordingly, this section discusses the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

In addition, according to the guidance from Mr. Chair, L1/L2 based CLI measurement/report for UE-to-UE CLI handling is to be discussed in AI 9.3.3 and exchange of intended subband configurations for SBFD operation across gNBs is to be handled in AI 9.3.2.

  - Summary of input contributions

The inputs from companies’ contributions on inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or missed and encourage companies to correct/update the summary with revision marks if needed.

    - UE-to-UE CLI handling

      - UE-to-UE CLI measurement/reporting

The following agreements were made in RAN1#112.

Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Among the three methods, it seems that all companies [Huawei, Spreadtrum, ZTE, Intel, DOCOMO, Lenovo] agree to support Method#1, i.e., RSSI measurement in DL subband. Some further enhancements e.g. non-contiguous CLI-RSSI resource allocation, subband CLI-RSSI measurement and reporting are proposed by companies and summarized below.

For the SINR measurement in Method#1, CMCC thinks that SINR can reflect the channel state information within DL subband more directly compared with RSSI and gNB can use the reported SINR to adjust the MCS for PDSCH scheduling. Xiaomi proposed that victim UE does not need to measure SINR within DL subband.

Regarding how to measure the SINR, Intel thinks that if ‘S’ is the signal strength of desired DL transmitted by serving gNB, UE may derive SINR based on RS transmitted by serving gNB in DL subbands and configured resource for interference from other gNB/UEs including CLI in DL subbands. CMCC thinks that the traditional CSI measurement and report framework can be reused. The only difference from traditional SINR measurement is that the received interference on IMR does not only include inter-cell interference but also includes inter-subband inter-UE CLI, which is transparent to UE. Qualcomm thinks that UE measures the DL channel based on NZP CSI-RS and interference based on the CLI Resources.

For Method#2 and Method#3, CMCC pointed out that it may need UE to receive in DL subband and measure CLI in UL subband simultaneously to avoid the performance degradation. Also it is considered that if the DL BWP contains the UL subband frequency region and if inter-UE CLI measurement outside DL subband is allowed, this function can be supported by Rel-16 inter-UE CLI measurement. CMCC proposed to study how to estimate the interference intensity of inter-UE inter-subband CLI from CLI-SRS RSRP/RSSI. Qualcomm and Nokia think that it is beneficial for gNB to have UE CLI reports based on the CLI measurements within the UL subband considering the impact on the AGC dynamic range for the reception of the DL signal at the victim UE and Rel-16 CLI measurement framework could be reused for CLI resources configured in the UL-subband.

Figure 5-12: Effect of inter-UE CLI on DL reception [34]

For Method#2, the intention from the proponent companies [ZTE, MediaTek, Nokia] is to identify/rank the aggressor UE(s).

Huawei thinks that CLI-RSSI measurement seems sufficient for interference measurement. In addition, Huawei thinks that the feasibility of DL measurement in UL subband has not been confirmed (there is no agreement yet to support DL reception within an UL subband) and the usage of such measurement should be clarified especially the benefit if any compared to the Rel-16 SRS-RSRP measurement mechanism.

Intel thinks that it does not only increase complexity for a measuring UE, it would lead to typically more conservative estimates than reality since it would not capture the true nature of leakage interference for inter-subband interference with non-overlapping SBFD operation. Therefore, Intel thinks that Method #2 may not be necessary.

LG thinks that SRS-RSRP measurement seems not feasible because UE monitors emission power from inter-subband. For operating SRS-RSRP of inter-subband CLI measurement, it seems some enhancement is required. It is proposed that for UE-to-UE SRS-RSRP measurement/report within UL subband, measurement gap for measurement inter-subband can be indicated. For UE-to-UE SRS-RSRP measurement within active DL BWP, SBFD symbol can be converted as non-SBFD symbol for operating within wider bandwidth.

For Method#3, ZTE thinks that there is no motivation to support Method#3.

In addition, Qualcomm proposed to consider a combination of methods where the gNB configure the CLI-Resource across both the DL and UL subbands and UE measures the CLI per subset of the frequency resource (i.e. subband) enabling CLI-RSSI reporting in both DL and UL subbands.

Non-contiguous CLI-RSSI measurement/report

The following agreement was made in RAN1#110bis-e.

Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

CATT think that if the CLI in two DL subbands is considered to be non-symmetric, separate CLI-RSSI measurement resources/reports can be configured in different DL subbands according to existing Rel-16 CLI-RSSI mechanism; otherwise if the CLI in two DL subbands is considered to be symmetric, i.e. CLI in RBs in two DL subbands with the same frequency separation from UL subband is the same, it seems sufficient to measure CLI in only one DL subband. So it is proposed to further discuss the motivation and potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

LG think that the measured strength of CLI may be different depending on the position of DL. But, if the UE-to-UE CLI-RSSI measurement/report is taken non-contiguous measurement resource in frequency into account, the measured strength of CLI is averaged over the non-contiguous measurement resource. Hence, it is hard to determine which part of DL subband is suffered from intra-cell inter-UE CLI. So it is proposed that the use cases and potential benefits for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency should be justified.

Some companies discussed how to support non-contiguous CLI-RSSI measurement resources across DL subbands and the views are summarized below.

  - Option 1: Two contiguous CLI-RSSI resources configured to be linked to one CLI-RSSI report

      - Supported by: Huawei

  - Option 2: One CLI-RSSI resource

    - Option 2-1: Non-contiguous CLI-RSSI resource allocation

      - Supported by: Huawei

    - Option 2-2: One contiguous CLI-RSSI resource allocation with non-contiguous CLI-RSSI resource derived by excluding frequency resources outside DL subband (s)

      - Supported by: Qualcomm (for Method#1)

CLI measurement and reporting considering non-uniform inter-subband CLI

In Rel-16 inter-UE CLI measurement, only wideband CLI measurement is reported. Subband CLI measurement and reporting are considered by NEC, ZTE, CMCC, Sony, Qualcomm, xiaomi considering CLI leakage to adjacent subbands is non-uniform over the measurement bandwidth. But it is also pointed out that existing configuration for L3 CLI-RSSI measurement resource already supports multiple resources in different frequency regions, which can be directly reused [20].

For subband CLI measurement/report, CMCC and Qualcomm proposed to support both uniform and non-uniform CLI subband configuration for Method#1. In addition, Qualcomm proposed that UE reports CLI measurements in specific subband(s) where CLI exceeds a configured CLI threshold and differential CLI reporting to reduce the CLI reporting overhead.

ZTE proposed that different frequency densities can be configured for reference signals (e.g., RS for UE-to-UE CLI measurement, RS for downlink transmission channel estimation) transmitted in different areas with different interference levels.

Information exchange between gNBs

Huawei thinks for UE-to-UE co-channel CLI measurement, at least the configuration of measurement sig-nals/channels should be exchanged between aggressor gNBs and victim gNBs. In addition, the CLI measurement reports can also be exchanged between aggressor gNBs and victim gNBs. Whether the information exchange needs to be specified can be further discussed.

Similarly, Qualcomm proposed that gNB to exchange information of the CLI resource configurations and/or CLI measurements.

Timing alignment aspects

ZTE proposed to consider timing alignment solution on measurement RS transmission for UE-to-UE CLI, e.g. exchange timing related information for reception of measurement RS.

Nokia proposed to study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement.

Other enhancements

InterDigital proposed to consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario. In addition, it is proposed to study measurement resources and reporting configurations for subband-edge CLI measurement and to study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

InterDigital proposed to study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Considering that the scheduler may not be able to discriminate between the inter-subband cross-link interference(s) and the intra-subband inter-cell interference (DL-DL or UL-DL) based on reported CLI-RSSI, MediaTek proposed to study the feasibility and cost of muting co-channel interferer for the assessment of inter-subband UE CLI using CLI-RSSI measurements.

MediaTek proposed to study optimized SRS configurations for CLI measurement within a guard band to reduce the resource overhead for SRS-RSRP measurements and make efficient use of GB resources.

MediaTek proposed to study CLI measurement on UL subband with frequency differentiation considering that there are multiple UE transmissions in the UL subband of a given slot, the current CLI-RSSI measurement doesn’t provide information on which UE(s) are the high aggressor. An example is shown in the figure below, in which the victim UE will report high UE-UE CLI for RB Groups 1 and 2, and no UE-UE CLI for RB Group 3. However, the current specification does not support such frequency differentiation for CLI measurement.

Figure 5-13: CLI measurement on UL subband with frequency differentiation [16]

      - Power control based solution

ZTE proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels. The resources contained in each area can be semi-statically configured by RRC or indicated by DCI.

As an example shown below, UL subband is divided into three areas in the frequency domain. A higher transmission power can be used for uplink transmission in Area 1 as it is far away from DL subband and a lower interference can be expected. For Area 2 and Area 3, a lower transmission power can be used for uplink transmission in them as they are closed to DL subband.

Figure 5-14: Different allowed maximum power for UL transmission in different areas [18]

InterDigital proposed to study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

Apple proposed that potential aggressor UE, UEA, is indicated to measure SRS (transmitted by potential victim UE) before PUSCH transmission. The indication can be through DCI scheduling the PUSCH for aggressor UE. Besides, DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s).

      - Others

ZTE proposed that different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

    - gNB-to-gNB CLI handling

      - gNB-to-gNB CLI measurement/reporting

For inter-gNB inter-subband CLI measurement, CMCC proposed to study the following two methods, where the measurement results of Method#1 can directly reflect the intensity of inter-subband CLI but cannot identify the strongest aggressor gNB, and the measurement results of Method#2 can identify the aggressor gNB based on accurate RS measurement without complicated inter-gNB coordination:

  - Method#1: victim gNB measures leakage interference strength from aggressor gNB within UL subband, e.g., RSSI;

  - Method#2: victim gNB measures RSRP of aggressor gNB within DL subband.

Figure 5-15: Inter-gNB inter-subband CLI measurement method#1 & method#2 [27]

Furthermore, it is proposed that inter-gNB transmission coordination in orthogonal time-domain, frequency-domain or spatial-domain resources can be supported to identify the strongest aggressor gNB in inter-gNB inter-subband CLI measurement Method#1.

Spreatrum supports Method#1 above for gNB-to-gNB inter-subband CLI handling.

Subband CLI measurement/reporting

Similar as subband CLI measurement/report for UE-to-UE CLI, Qualcomm proposed to support subband-based inter-gNB CLI reporting for accurate measurement of CLI leakage in SBFD.

Different RS frequency densities

Similar as UE-to-UE inter-subband CLI, ZTE proposed that different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels.

Resource muting

Huawei has the following observations.

Uplink resources muting provides at least the following benefits for gNB-to-gNB co-channel CLI measurement in SBFD operation

- Muting the REs on the DL subband in UL DMRS symbols and/or the REs on the UL subband in DL DMRS can improve channel estimation and inter-cell interference estimation and suppression.

- Introducing dedicated UL muting resources for gNB-to-gNB CLI measurement can improve the gNB-to-gNB co-channel and adjacent-channel inter-subband CLI estimation and suppression.

Huawei thinks that based on the above scheme, the accurate channel and interference covariance matrix can be measured. And these measurement quantities can be used to improve performance at the receiver, e.g. MMSE-IRC receiver, which can effectively suppress the gNB-gNB co-channel CLI.

Figure 5-16: Resource muting for gNB-gNB co-channel CLI measurement [8]

      - Beam nulling between gNBs

To suppress the DL blocking signal, Huawei proposed to study the feasibility and performance of beam nulling for gNB-to-gNB CLI suppression with gNB-to-gNB channel, e.g. if the channel between transmitter and receiver can be measured, the DL beamforming weights can be manipulated to avoid transmitting in the direction of the receiver by using coordinated beamforming (CBF).

Figure 5-17: CBF to suppress blocking interference [8]

      - Interference suppression based on analogue filter

Huawei proposed to study the feasibility and performance of applying filters at both transmitter and receiver sides in SBFD involving RAN4 on the following aspects.

- Filter at transmitter to suppress the leakage interference.

- Filter at receiver to suppress the blocking interference.

- Guard band for filters.

It is noted that Huawei thinks that the study should be started by RAN4 and the performances should be provided to RAN1 to check the feasibility as well as the performance of SBFD.

  - [Open] 1st round discussion

Proposal 3-1

Proposed Agreement:

Update the agreement in RAN1#112 as follows.

For inter-UE inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim UE measures RSSI within DL subband within DL BWP

    - FFS: Whether SINR can be measured

  - Method#2: victim UE measures RSRP of aggressor UE within UL subbandoutside DL subband within DL BWP

  - Method#3: victim UE measures RSSI within UL subbandoutside DL subband within DL BWP

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

|  | Company |
| --- | --- |
| Support | Xiaomi, Samsung, Intel , vivo, CMCC, Huawei, HiSilicon, Panasonic, DOCOMO, Nokia, NSB(with update) |
| Not support | NEC |

| Company | Comments |
| --- | --- |
| Moderator | It is based on the proposal from MediaTek to include measurement in guardband(s). In addition “within DL BWP” is added for Method#1 to be aligned with other methods. |
| ZTE | We propose to keep the existing agreement as it is. The guard band (if exist) is not used for transmission or reception. If UE has the capability to transmit or receive in the guard band, then the network doesn’t need to configure the guard band at the first place. |
| OPPO | In our understanding, there is no any RS/signal within guardband. So the RSRP can not be measured in guard band. Modification of method#2 is confusing. |
| LG Electronics | In case of measuring RSRP of aggressor UE, the victim UE can measure the RSRP within DL BWP if DL only is enabled. Is the Method#2 with modification explaining aligned with this understanding? |
| Sharp | We agree with ZTE. Does this update intend to perform measurement in guard bands? |
| Panasonic | We think that how to treat guardband is not clarified now. Then, modifications of the proposal would be OK. |
| QC | Further clarification is needed to update the agreement from RAN1 #112. Outside DL subband means UL-SB and guard band. It is not clear how the victim UE would measure RSRP in the guardband? It requires aggressor UE to transmit SRS in the UL-subband. In general, UEs shouldn’t transmit neither receive in the guardband. |
| NEC | We prefer the earlier wording of the agreement as the advantage of measuring CLI within guardband is not apparent to us. Further, we agree with Qualcomm’s comment for Method#2 how can a UE measure RSRP within guardband? Because this assumes that the transmitting UE is performing UL transmission within the guardband which is not the assumption we can agree to. |
| Nokia, NSB | We propose to remove “within DL BWP”, then we can agree on the update. |

Proposal 3-2

Proposed Agreement:

For inter-UE inter-subband CLI measurement, at least the following method is agreed.

  - Method#1: victim UE measures RSSI within DL subband within DL BWP

|  | Company |
| --- | --- |
| Support | xiaomi, Sony, Samsung, IDC, Intel , vivo, Huawei, HiSilicon,OPPO, LG Electronics, Sharp, Panasonic, Lenovo, NEC, DOCOMO, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We think both method#1 and method#2 should be supported. Method#1 can obtain the CLI strength directly while method#2 can identify the interfering source and identify the most strong aggressor(s) as well. |
| IDC | Support at least the Method #1 for inter-UE inter-SB CLI measurement. |
| CMCC | Also support option 2, this function has been supported in Rel-16, we don’t know why it is precluded from SBFD. |
| QC | RSSI measurements in DL subband within UE DL BWP are not accurate method for capturing inter-UE CLI or identify aggressor UEs. It will be contaminated by inter-cell interference. RSSI/RSRP measurement in the UL-SB within UE BWP should be starting point. |
| Nokia, NSB | We also think both method 1 and method 2 should be studied more before agreement, or both should be supported. |

Proposal 3-3

Proposed Conclusion:

No specific enhancement is needed for SINR measurement within DL subband for inter-UE inter-subband CLI measurement, if agreed.

|  | Company |
| --- | --- |
| Support | ZTE xiaomi, Samsung, Huawei, HiSilicon,OPPO, LG Electronics, NEC, DOCOMO |
| Not support | IDC, CMCC, Nokia, NSB |

| Company | Comments |
| --- | --- |
| IDC | It is too early to conclude for no enhancement in SI phase. This detail should be carefully checked during WI phase, e.g., whether/how to measure the interference part including CLI part based on which specific RS for the interference measurement, etc. Even for the channel measurement part, the channel measurement resource (e.g., CSI-RS) related issues need to be checked, including spatial-domain reference (e.g., QCL type-D) management, etc. |
| Intel | What does ‘specific enhancement’ mean, e.g., comparing with existing SINR measurement ? |
| vivo | Similar comments with Intel. |
| CMCC | Same view as IDC. As SINR is one of the metric of CSI measurement and report framework, and we are discussing proposal 1-14 and proposal 1-15, about the CSI-RS and CSI reporting band enhancement across SBFD and non-SBFD, these discussion may have spec impacts. |
| Huawei, HiSilicon | The proposal is not quite clear. We prefer to clarify how it is done and whether it has any spec impacts. |
| QC | Further clarification needed. In our understanding, for UE to measure SINR, UE needs CMR for DL signal and IMR for interference. Is the intention that IMR is reused to capture both legacy inter-cell interference and CLI as well? In our views, to measure accurate SINR reflecting CLI, additional CLI-resource should be used. |
| Nokia, NSB | L1 SINR measurement or CSI measurement for interference including CLI should be important assistance for resource utilization of SBFD and non-SBFD resource. |

Proposal 3-4

Proposed Agreement:

For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering:

  - Necessity/benefit compared with measurement within DL subband

  - Whether/how to estimate inter-subband CLI from RSRP/RSSI measurements within UL subband

  - Whether UE is required to measure RSRP/RSSI within UL and receive DL in DL subband(s) simultaneously

  - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband

    - If not, identify the potential impact

|  | Company |
| --- | --- |
| Support | xiaomi, Sony, Samsung, IDC, Intel , vivo, CMCC,OPPO, LG Electronics, DOCOMO, CATT, Nokia, NSB(with update) |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We think at least method#2 should be supported. Similar view as above. |
| Huawei, HiSilicon | We think it is important to justify the necessity/benefit not only by comparing them to Method#1 but also the existing Rel-16 UE-to-UE CLI measurement schemes. |
| QC | 2nd subbulet is not needed. CLI measurement in UL-SB is mainly intra-subband CLI where aggressor UE transmit in UL-SB and victim UE measures in same resources confined within the DL BWP. |
| NEC | For UE to measure CLI within the UL subband we think that existing CLI framework can be reused by conversion of symbol from SBFD to DL-only for the measuring UE and configuring normal CLI measurements within the frequency resource of the UL subband. |
| Nokia, NSB | We support further study, but with update For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering: - Necessity/benefit compared with measurement within DL subband - Whether/how to estimate inter-subband CLI from RSRP/RSSI measurements within UL subband or guard band - Whether UE is required to measure RSRP/RSSI within UL and receive DL in DL subband(s) simultaneously - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband - If not, identify the potential impact |

Proposal 3-5

Proposed Agreement:

For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods:

  - Method#1: separate CLI-RSSI measurement resources/reports in each DL subband

    - Note: supported in existing specifications

  - Method#2: CLI-RSSI measure/report in one DL subband only

    - Note: supported in existing specifications

  - Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands

    - FFS: report single or separate report(s)

For Method#3, if agreed, consider the following options for non-contiguous CLI-RSSI resource allocation in frequency:

  - Option 1: Two contiguous CLI-RSSI resources configured to be linked to one CLI-RSSI report

  - Option 2: One CLI-RSSI resource

    - Option 2-1: Non-contiguous CLI-RSSI resource allocation

    - Option 2-2: One contiguous CLI-RSSI resource allocation with non-contiguous CLI-RSSI resource derived by excluding frequency resources outside DL subband (s)

|  | Company |
| --- | --- |
| Support | ZTE, xiaomi, Sony, IDC, vivo, CMCC, Huawei, HiSilicon,OPPO, LG Electronics, Lenovo, Qualcomm, NEC (support Method#3) , DOCOMO, CATT |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| xiaomi | We prefer method#1 and method#2. |
| Sony | We think that all 3 methods are ok and should be supported and up to gNB to configure one of them. |
| Samsung | We only support the paragraph (up to “separate report(s)”). We think that second paragraph starting from “For Method #3, if agreed,…” to end, it does not make sense to agree on options if we don’t know yet if Method #3 can be supported “at all”. |
| IDC | Support the proposal to study the methods further. |
| LG Electronics | Generally fine with the Proposal 3-5. Clarification whether this handling scheme can be applied for only SBFD case, or for both SBFD and d/f TDD case (i.e., a cell in SBFD operation, B cell in d/f TDD operation) seems necessary. |
| Nokia, NSB | We support the study. Additionally, we propose inter-gNB coordination on the SBFD related information, as assistance on how to measure CLI, as gNB need to know whether same RSSI can be measured on different DL subbands. |

Proposal 3-6

Proposed Agreement:

Study whether/how to support subband CLI-RSSI measurement and reporting.

|  | Company |
| --- | --- |
| Support | ZTE, xiaomi, Sony, Samsung, IDC, Intel , vivo, CMCC,OPPO, Panasonic, Lenovo, Qualcomm, DOCOMO, CATT |
| Not support | Huawei, HiSilicon |

| Company | Comments |
| --- | --- |
| ZTE | we support to have finer reporting granularity, i.e., subband. |
| IDC | Support the proposal to enhance CLI-RSSI measurement and reporting based on subband-wise measurement and differential measurements. |
| Huawei, HiSilicon | It is not clear to us how subband CLI-RSSI can be utilized by the scheduler in particular in case of a smaller frequency granularity. |
| LG Electronics | In last meeting, there was a similar discussion in AI 933. (Moderator Proposal #21-3 (1) For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the benefit of applying narrower frequency granularity of CLI measurement reporting.) But, we are generally fine with the discussion whether/how to support subband CLI-RSSI measurement and report in AI932 or AI933. |
| QC | Subband CLI reporting is beneficial, for example when CLI characteristics are different across the frequency resources of the CLI resource. |
| Nokia, NSB | We support the study. Additionally, we propose inter-gNB coordination on the SBFD related information, as assistance on how to measure CLI. |

Proposal 3-7

Proposed Agreement:

For inter-gNB inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim gNB measures RSSI within UL subband

  - Method#2: victim gNB measures RSRP of aggressor gNB within DL subband

|  | Company |
| --- | --- |
| Support | ZTE, Sony, Samsung, IDC,CMCC,OPPO, Panasonic, Qualcomm, CATT, Nokia, NSB (with update) |
| Not support | Huawei, HiSilicon |

| Company | Comments |
| --- | --- |
| ZTE | We think both method#1 and method#2 should be supported. Method#1 can obtain the CLI strength directly while method#2 can identify the interfering source and identify the most strong aggressor(s) as well. |
| CMCC | Support both methods. |
| Huawei, HiSilicon | We think the above measurement can be done by gNB implementation. |
| Nokia, NSB | We support the study but with update - Method#2: victim gNB measures RSRP of aggressor gNB within DL subband or guard band Additionally, we propose inter-gNB coordination on the SBFD related information, as assistance on how to measure CLI. |

- Offline discussion

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| New H3C | Lei Kong | Kong.lei@h3c.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NEC | Pravjyot Singh Deogun Li Xincai | pravjyot.deogun@emea.nec.com li_xincai@nec.cn |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Panasonic | Hidetoshi Suzuki Tomoya Nunome Quan Kuang | suzuki.hidetoshi@jp.panasonic.com nunome.tomoya@jp.panasonic.com Quan.Kuang@eu.panasonic.com |
| Intel | Yi Wang | yi5.wang@Intel.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.org.tw |
| Lenovo | Hyejung Jung Vijay Nangia | hyejung@motorola.com vnangia@motorola.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| WILUS | David (Geunyoung) Seok | david.seok@wilusgroup.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| LG Electronics | Hyunsoo Ko | hyunsoo.ko@lge.com |
| SK Telecom | Sanghoon Cho | seanc.cho@sk.com |
| KDDI | Masahito Umehara | ma-umehara@kddi.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Fujitsu | Taewoo LEE | lee.taewoo@fujitsu.com |
| Apple | Ali Fakoorian | sfakoorian@apple.com |

- Reference

- RP-220633		Revised SID: Study on evolution of NR duplex operation	CMCC

- R1-2205399		TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205520		Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)	RAN1#109-e

- R1-2208122		Summary #5 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110

- R1-2210317		Summary #4 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110bis-e

- R1-2212735		Summary #3 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#111

- R1-2301935		Summary #3 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#112

- R1-2302348		Discussion on potential enhancement on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2302407		Discussion on subband non-overlapping full duplex	TCL Communication Ltd.

- R1-2302426		Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2302484		Discussion on subband non-overlapping full duplex	vivo

- R1-2302522		Discussion on subband non-overlapping full duplex	InterDigital, Inc.

- R1-2302547		Discussion on subband non-overlapping full duplex	OPPO

- R1-2302599		Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2302702		Discussion on subband non-overlapping full duplex	CATT

- R1-2302736		Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2302746		Discussion on subband non-overlapping full duplex	NEC

- R1-2302757		Discussion of subband non-overlapping full duplex	ZTE

- R1-2302770		Subband non-overlapping full duplex	Ericsson

- R1-2302795		On SBFD operation in NR systems		Intel Corporation

- R1-2302845		Considerations on Subband Full Duplex TDD operations	Sony

- R1-2302910		Discussion on subband non-overlapping full duplex	Fujitsu

- R1-2302982		Discussion on subband non-overlapping full duplex	xiaomi

- R1-2303016		On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2303127		On SBFD for NR duplex evolution	Samsung

- R1-2303197		Discussion on subband non-overlapping full duplex enhancements	ETRI

- R1-2303233		Discussion on subband non-overlapping full duplex	CMCC

- R1-2303262		Discussion on subband non-overlapping full duplex	Panasonic

- R1-2303303		Discussion on subband non-overlapping full duplex	CEWiT

- R1-2303408		Discussion on subband non-overlapping full duplex	FGI

- R1-2303459		Discussion on subband non-overlapping full duplex	Sharp

- R1-2303482		Views on subband non-overlapping full duplex	Apple

- R1-2303530		Subband non-overlapping full duplex	Lenovo

- R1-2303589		Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2303711		Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2303742		Study on Subband non-overlapping full duplex	LG Electronics

- R1-2303779		Discussion on sub-band non-overlapping full duplex	ITRI

- R1-2303825		Details of subband non-overlapping full duplex	ASUSTEK COMPUTER (SHANGHAI)

- R1-2303830		Discussion on subband non-overlapping full duplex	WILUS Inc.

Appendix: Previous agreements of SBFD

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

RAN1#110bis-e

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

- If there are, whether/how to avoid/handle such collision cases (as second step)

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Agreement

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above.

LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#111

Agreement

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Agreement

For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Agreement

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously.

Agreement

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH

RAN1#112

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

Agreement

Study whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

Agreement

For inter-UE inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim UE measures RSSI within DL subband

    - FFS: Whether SINR can be measured

  - Method#2: victim UE measures RSRP of aggressor UE within UL subband

  - Method#3: victim UE measures RSSI within UL subband

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - Study the following options for SBFD-aware UEs:

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

    - PDSCH/PUSCH/PUCCH repetitions

    - SPS PDSCH/CG PUSCH

    - TBoMS

    - Multi-PUSCH/PDSCH scheduled by a single DCI

    - Periodic/semi-persistent SRS/CSI-RS/PUCCH

    - PDCCH

Agreement

For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

FFS: The part of the RBG outside.

Agreement

For SBFD-aware UEs, study at least the following issues for PDSCH:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

Agreement:

Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation