3GPP TSG RAN WG1 #112bis-e			R1- 2304032

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.3.3

Source: 	Moderator (LG Electronics)

Title:	Summary #1 of potential enhancement on dynamic/flexible TDD

Document for:	Discussion and Decision

- Introduction

In this documentation, proposals based on the technical documentation submitted in RAN1#112-bis-e and the discussion on the potential enhancement of dynamic/flexible TDD are summarized.

- Moderator Proposals for On-line discussion

  - Tuesday

gNB-to-gNB inter-cell co-channel interference

1.1. gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

Moderator Proposal #11-1  (2)

Capture the text in the TR.

For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline. Both CD-SSB and NCD-SSB can be used.

It is assumed that exchange of configuration for NZP CSI-RS/SSB can be an enabler for gNB-to-bNB co-channel CLI measurement and/or channel measurement.

Moderator Proposal #11-2 (Performance metric) (2)

For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement baseline metric for the study

Moderator Proposal #11-3 (UL rate-matching/resource muting) (1)

For enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement, UL rate-matching/resource muting can be applied.

Following options can be studied.

- Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource, use UL muting patterns based on RB-level and symbol level muting)

- Option 2: Non-transparent UL resource muting method (e.g., define UL rate-matching/muting pattern with one or more RE muting patterns)

Moderator Proposal #12-1 (1)

For gNB-to-gNB and/or UE-to-UE co-channel CLI handling, exchange of SBFD time/frequency configuration can be an enabler.

- gNB-to-gNB inter-cell co-channel interference

1. gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

2. Coordinated scheduling for time/frequency resources between gNBs

3. Spatial domain coordination method for gNB-to-gNB CLI handling

4. UE and gNB transmission and reception timing

5. Power control based solution

  - 1st Round Discussion

    - [OPEN] gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

Summary

From RAN1#110 meeting to RAN1#112 meeting, there were discussions to determine which method(s) of gNB-to-gNB co-channel CLI handling is/are studied, and following(s) were agreed.

1. gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling RAN1#110 Agreement Study the feasibility and potential benefits of gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource configuration - Measurement details - Relevant information exchange - Usage of measurement. RAN1#110-bis-e Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not. Agreement For gNB-to-gNB co-channel CLI measurement, consider as baseline reusing existing DL channel(s)/signal(s)/measurement_resource(s) - For example, SSB, NZP/ZP-CSI-RS, DMRS for PDCCH/PDSCH, CSI-IM, RSSI measurement resource, etc. - FFS: Which type of DL channel(s)/signal(s) can be used for gNB-to-gNB co-channel CLI measurement - FFS: How resources are used/configured. RAN1#111 Agreement For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. - FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. RAN1#112 Agreement For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement.

In RAN1#112-bis-e meeting, followings are proposed.

Enhancement of CSI-RS for gNB-to-gNB CLI measurement

Huawei thinks that the potential enhancement to NZP CSI-RS for gNB-to-gNB channel measurement in FR1 for coordinated beamforming is necessary. It is proposed to study CSI-RS port expansion to support gNB-to-gNB channel measurement for SBFD and DTDD with considering following gNB-to-gNB channel characteristics to reduce the high overhead of CSI-RS caused by CSI-RS port expansion:

    - gNB-to-gNB channel has a larger coherent time than gNB-UE channel.

    - gNB-to-gNB channel has a larger coherent bandwidth than gNB-UE channel.

Also, it is proposed to study gNB-to-gNB channel measurement resource management, coordination, and configuration by OAM.

ZTE thinks that in order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives:

    - Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim.

    - Alt.2: Define NZP CSI-RS with up to 64 ports.

    - Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI.

Lenovo proposes to study enhancements to periodic RS for resource efficiency, scalability, and flexibility of gNB-to-gNB CLI measurement. Consider gNB-specific patterns of RS transmission and CLI measurement.

Nokia thinks allowing CSI-RS transmission during the guard period symbols for conducting CLI measurements while not impacting the downlink spectral efficiency on the aggressor gNB.

Qualcomm thinks that RAN1 shall study whether to reuse existing access link RS (e.g. at least periodic NZP CSI-RS/SSB) or introduce a dedicated RS (e.g. at least periodic NZP CSI-RS/SSB) for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

FL summary

Five companies are thinking enhancement of CSI-RS can be studied

- For the purpose of gNB -to-gNB co-channel channel measurement, enhancement of CSI-RS needs to be studied.

- For introducing a dedicated RS, enhancement of CSI-RS needs to be studied.

Beam sweeping

Huawei thinks that Beam sweeping among multiple gNBs can be studied for beam pairing.

InterDigital thinks that the gNB-to-gNB co-channel CLI mitigation can be based on spatial domain coordination, where the CLI measurement can be based on beam sweeping at both victim and aggressor gNBs.

vivo thinks For spatial domain coordination, beam sweeping procedure to identify preferred/non-preferred DL beams of aggressor gNB can bese on implementation.

Spatial Domain Coorination

InterDigital proposes to consider using spatial domain coordination for gNB-to-gNB co-channel CLI measurement and mitigation, where the victim gNB measures beam-swept CLI and sends, to the aggressor gNB, information on the SSB index or the CRI of the aggressor beams with the highest and/or lowest CLI in addition to the measured CLI.

CATT thinks that beam level measurement results and corresponding measurement resources should be exchanged among gNBs to achieve beam/spatial based CLI management.

Reference signal (SSB, CSI-RS) Information Exchange

New H3C thinks that the configuration of both CD-SSB and NCD-SSB, such as the number of SSB, time-frequency domain position of the SSB, the periodicity and the time offset of the first SSB, and so on, should be exchanged between gNBs for CLI measurement. The same slot of the victim gNB as the SSB slot in aggressor gNB should be configured to the UL slot.

New H3C thinks that the existing CSI-RS for interference measurement (CSI-IM) /ZP-CSI-RS can be reused for the CLI measurement and report in the D/F-TDD, the measurement resource can be periodic, aperiodic or semi-persistent. The mechanism of the CSI-RS for interference measurement (CSI-IM) can be extended to the multiple aggressor gNBs case. A central controller can be used to handle the information exchange between gNBs, it can be a CU, a master gNB, or OAM. The NZP-CSI-RS can also be used for CLI measurement in order to get more precise measurement results. The measurement resource can be periodic, aperiodic or semi-persistent. The NZP-CSI-RS for different aggressor gNBs should be different, and the configuration of the NZP-CSI-RS should be exchanged between gNBs by Xn interface, or handled by a central controller. In victim gNB, the PRACH/PUCCH/SRS should not use any resource that are overlapping with CSI-RS for CLI measurement, the PUSCH can perform rate matching around CSI-RS for CLI measurement.

CATT thinks that Neighboring gNBs should exchange measurement configuration information of SSB set and/or CSI-RS set (each SSB or CSI-RS in the set is associated with a specific beam) to enable beam level CLI measurement.

Intel thinks that for gNB-to-gNB CLI mitigation, the configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB) needs to be exchanged between gNBs. Measurement and reporting periodicity: at least periodic measurement resources and reporting are considered.

Nokia thinks that the measuring gNB should be informed about the CLI-RS configuration over the Xn interface. This applies to both CLI-RS candidates, the SSB-based and CSI-RS-based measurements.

Lenovo thinks upport exchange of reference signal configuration information among gNBs for the purpose of inter-gNB CLI measurement.

Samsung thinks that for gNB-to-gNB co-channel CLI and/or measurement enhancements, Xn/F1AP signaling can indicate NZP CSI-RS resource, SSB resources, DL muting patterns of the aggressor gNB to the victim gNB.

FL summary

Six companies are thinking that for gNB-to-gNB co-channel CLI measurement, the configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB) needs to be exchanged between gNBs.

Measreuement Resource (CSI-RS)

NEC proposes to study aperiodic or semi-persistent CSI-RS along with periodic CSI-RS for gNB-gNB CLI measurements

Measreuement Resource (SSB)

TCL proposes that the procedure of gNB-to-gNB co-channel CLI measurement based on the CD-SSB and NCD-SSB can be further studied.

    - CD-SBB or NCD-SSB are UE specific and may not be directly applicable to measure the gNB-to-gNB co-channel CLI.

    - To measure gNB-to-gNB CLI, it may be necessary for the victim gNB to identify the aggressor gNB.

Lenovo thinks that in order for each gNB to have a chance to measure CLI from any other gNB in its vicinity, support gNB-specific patterns for transmitting SSBs dedicated to CLI measurements. The SSBs can be configured as NCD-SSB.

Cooridnaition of Measreuement Resource (RSSI resource, Measurement/Transmission)

CMCC thinks that for inter-gNB intra-subband CLI measurement and reporting, the transmission of different aggressor gNBs are coordinated on different RSSI resources/occasions in TDM/FDM manner.

NTT DOCOMO thinks that measurement resource for gNB-to-gNB CLI measurement should be separated from the transmission resource of victim gNB for NZP CSI-RS and SSB.

Qualcomm thinks that for SSB serving as gNB-to-gNB co-channel CLI measurement and/or channel measurement RS, baseline proposal is to support the central NW can configure dedicated RS that is not used for access link for gNB-to-gNB CLI/channel measurement to guarantee TDMed CLI measurements across different gNBs to avoid Tx and Rx collisions. For CSI-RS serving as gNB-to-gNB co-channel CLI measurement and/or channel measurement RS, baseline proposal is to support reusing access link CSI-RS for gNB-to-gNB CLI/channel measurement with the assumption that gNB can coordinate with neighbor gNBs and configure CSI-RS to guarantee the TDMed fashion measurement among different gNBs for gNB-to-gNB CLI/channel measurement.

    - FFS: Study the receiving timing of CSI-RS for gNB-to-gNB CLI/channel measurement

Measurement Window

New H3C thinks that a measurement window can be introduced for improving the energy efficiency of the victim gNB. For the victim gNB, it can only measure the CLI measurement signals in the measurement windows, and ignore all the CLI measurement signals out the range of the measurement windows. Several measurement window can be configured, but only one is active. The measurement window is periodic, and its position is determined by the length, periodicity and offset.

NTT DOCOMO thinks that information for measurement window needs to be exchanged among gNBs via F1-AP.

Qualcomm proposes to investigate how resources are used/configured: e.g. how inter-gNB CLI measurement RS Tx and Rx time window configuration per cell. It can be considered gNB HD/FD capability in the inter-gNB CLI RS Tx and Rx time window configuration.  RAN1 can study whether to perform simultaneous Tx and Rx of CLI measurement RS for gNB-to-gNB CLI/channel measurement for SBFD capable gNB. It is proposed that RAN1 study exchanging the DL muting pattern among the gNBs to ensure the accurate inter-gNB CLI/channel measurement.

Performance Metric

New H3C thinks that the new RAN measurement abilities should be introduced for supporting the CLI measurement and reporting: CLI-RSSI and/or CLI RSRP.

OPPO thinks that a CLI-RSSI-alike resource, i.e. defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset, can be used for gNB-to-gNB CLI measurement. OPPO proposes that RAN1 targets to support L1 gNB-to-gNB CLI measurement. L1-based RSRP/RSSI can be considered;L1 gNB-to-gNB CLI measurement reporting with timestamp is exchanged over Xn interface.

CATT thinks that RSSI and RSRP can be considered as the gNB-gNB CLI measurement metric.

ZTE proposes that both RSRP measurement and RSSI measurement can be considered.

    - The existing measurement resource configuration for SSB/CSI-RS based RRM can be applied as baseline for gNB-to-gNB co-channel RSRP measurement.

    - The existing configuration of RSSI measurement resource can be applied as baseline for gNB-to-gNB co-channel RSSI measurement.

ZTE mentions below:

- RSRP measurement

Compared with RSSI measurement, it has the advantage of identifying interference sources, but the disadvantage is that it can only be used for intra-frequency CLI measurement, e.g., intra-subband CLI measurement. As agreed in RAN1#111 meeting, at least periodic NZP CSI-RS/SSB can be considered.

Regarding SSBs, they are transmitted periodically through sweeping mode, making them naturally suitable for obtaining per SSB interference. Further, the frequency domain location and time domain patterns are well designed, which benefits the information exchange of measurement resources between gNBs.

Regarding NZP-CSI-RS, it has the advantage of more flexible frequency resource configuration, so it can serve as an effective supplement to SSBs as a CLI measurement RS. In addition, it can also be used for channel measurement among gNBs for spatial domain coordination.

- RSSI measurement

For RSSI measurement, it can be based on a measurement resource and the measurement result represents received signal strength on the measurement resource. The interference source cannot be distinguished unless different measurement resources are associated with different aggressors. However, the advantage is that it can be used for both of intra-frequency CLI and adjacent-frequency CLI measurement, e.g., inter-subband CLI measurement.

Intel thinks that CLI measurements may be categorized as short-term and long-term interference measurements:

  - Short-term CLI metrics may be defined based on CSI/CQI- or L1-RSRP/RSSI/SINR-like measurements.

    - NZP-CSI-RS may be suitable candidates for CLI-RS for short-term CLI metrics.

    - Use of ZP CSI-RS or CSI-IM resources can be studied further for estimating L1-SINR under different interference hypotheses when considering coordination across more than two gNBs/TRPs.

  - Long-term CLI metrics may be defined based on CLI-RSRP- or CLI-RSSI-like measurements.

    - In addition to NZP-CSI-RS, CD-SSB and NCD-SSB may be CLI-RS candidates at least for long-term CLI measurements.

Xiaomi thinks RSRP and RSSI can be considered as baseline metrics for gNB-to-gNB CLI measurement.

CMCC thinks that for inter-gNB intra-subband CLI measurement and reporting, RSSI-like measurement can also be supported.

Nokia thinks that for gNB-to-gNB co-channel CLI measurement, the RSRP should be used as baseline measurement metric.

Samsung thinks that for gNB-to-gNB CLI and/or measurement enhancements, no new gNB-side measurement capabilities are introduced in 38.215. “Actual gNB-side co-channel CLI measurements on the NZP CSI-RS resources are difficult to exchange in real-time on existing network interfaces. We therefore consider the introduction or definition of additional NG-RAN, i.e., gNB-side, CLI measurements in 38.215 based on the NZP CSI-RS configured as CLI measurement resources for gNB-to-gNB co-channel measurements as not meaningful.”

FL summary

7 companies are thinking both RSRP measurement and RSSI measurement can be considered as the gNB-gNB CLI measurement metric

Measurement Reporting

xiaomi thinks periodic reporting for gNB-to-gNB CLI mitigation.

Others

NEC proposes to define CLI sensitivity level as a measurement metric for gNB-gNB CLI measurements.

Measurement Accuracy Enhancement (UL rate-mathing/puncturing)

In RAN1#110bis-e meeting, the following agreement about further study UL rate matching/cancellation/muting operation was made [6]:

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

CMCC thinks that for inter-gNB intra-subband CLI measurement and reporting, both transparent and non-transparent UL resource muting method should be considered, e.g., define UL rate-matching/muting pattern or avoid the scheduling on measurement resource.

- Support, but No specification impact

CATT thinks that for UL transmission muting, both gNB scheduling based solution and rate matching based solution can be used.

Samsung thinks that UL muting patterns based on RB-level and symbol level muting, if needed, are left to gNB implementation.

Panasonic thinks that for UL muting resource for gNB-to-gNB CLI measurement, UE transparent scheme should be supported.

- Support with specification impact

NEC proposes UL rate matching/puncturing procedures at least for CLI measurement based on CSI-RS, (FFS for SSB)

Intel thinks that UL resource blanking to enable measurements at a victim gNB can be explicitly realized via existing mechanisms in NR, e.g., UL CI, SFI, etc., or implicitly based on gNB scheduling.

Huawei thinks that Different uplink blank/muting resources can be used to measure spatial characteristics of gNB-to-gNB CLI caused by various DL signals and to avoid cross link interference. Uplink resources muting pattern can be different for various DL channel(s)/signal(s). For gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling support the following

  - Muting REs in UL slot at the position of part of REs of the SSB, SIB1, and broadcast PDCCH from aggressive cell are supported to measure the spatial characteristics of downlink broadcast interference.

  - Muting REs in UL slot at the position of part of REs of unicast PDSCH and PDCCH from aggressive cell are supported to obtain the spatial characteristics of unicast PDSCH and PDCCH CLI.

  - Muting REs in UL slot at the position of REs of NZP CSI-RS from aggressive cell are supported to avoid strong CLI.

UE non-transparent uplink muting resources is supported for cross link interference measurement and avoidance.

ZTE thinks that regarding UL resource muting, UL rate matching/cancellation mechanism can be defined for more accurate gNB-to-gNB co-channel CLI and/or channel measurement. Regarding UL resource muting, the rate matching resource for uplink transmission can be determined according to the measurement resources.

  - FFS whether a certain guard bands need to be reserved around the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB).

Sony thinks that for UL and DL transmissions, the gNB semi-statically configures one or more RE muting patterns for the UE, i.e. the UE is aware of which REs is muted. The gNB dynamically enables/disables RE muting for an UL/DL transmission and if multiple RE patterns are configured, the gNB indicates which RE muting pattern to apply in the dynamic grant. RE muting on REs containing gNB RS is conditional upon the transmission parameters, such as the L1 priority or MCS of the UL transmission.

NTT DOCOMO thinks that necessity of UL muting resource indication should be discussed based on typical scenarios for gNB-to-gNB measurement. And if we find the necessity of UE UL muting, UL muting resource indication with small granularity in time/ frequency domain can be considered.

WILUS think that RAN1 to study UE non-transparent UL muting for gNB-to-gNB CLI handling in terms of efficient resource utilization. (FFS: UE behaviors on UL muting resource with respect to the UL signal/channel and PHY priority.)

Qualcomm proposes that RAN1 study exchanging the UL muting pattern among the gNBs.

FL Summary

Majority view is that UL rate-matching/resource muting can be applied for enhancement of gNB-to-gNB co-channel CLI measurement. It is observed that companies proposes two different options (i.e., Transparent UL resource muting method, Non-transparent UL resource muting method). In this sense, these two option can be studied further.

CLI Measurement Procedure

Nokia thinks that gNB-to-gNB CLI measurements to follow a 2-step procedure. In the first step, gNBs use SSBs to obtain a course per-SSB CLI estimation. On a second step, CSI-RS are used to fine-tune the initially measured CLI levels.

Lenovo proposes assigning priorities to victim gNBs so that the aggressor gNB will be able to limit or avoid the CLI towards at least high-priority victim gNBs.

Proposals

Moderator Proposal #11-1 (configuration exchange) (1)

For gNB-to-gNB co-channel CLI / channel measurement, the configuration on the time/frequency/sequence/spatial information on the CLI-RS (e.g., NZP CSI-RS, both CD-SSB, andor NCD-SSB) needs to be exchanged between gNBs.

Moderator Proposal #11-1  (2)

Capture the text in the TR.

For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline. Both CD-SSB and NCD-SSB can be used.

It is assumed that exchange of configuration for NZP CSI-RS/SSB can be an enabler for gNB-to-bNB co-channel CLI measurement and/or channel measurement.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, NTT DOCOMO,OPPO, SPRD, TCL, CMCC, Huawei, HiSilicon, Xiaomi, Sony, CATT, NEC, Nokia/NSB, Ericsson (with revision), IDC, Intel, Lenovo/MM (with comment), QC |
| Not support | Samsung |

| Companies | Views |
| --- | --- |
| New H3C | In previous meeting, NZP CSI-RS, both CD-SSB and NCD-SSB are considered as the CLI-RS. Per our understanding, the other resources, such as ZP CSI-RS/CSI-IM are not excluded. So, we suggest the following updates For gNB-to-gNB co-channel CLI measurement, the configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB, and so on) needs to be exchanged between gNBs. |
| Vivo | Time/frequency/sequence/spatial information of CD-SSB exchange may not be needed.Simlar as UE, the gNB performing CLI measurement can aquire the information via SIB. |
| ZTE | In our understanding, the CLI measurement only includes the interference measurement. For gNB-to-gNB CLI handling, we think channel measurement is also very important and helpful. The configuration exchange between gNBs on the measurement channel is also needed. Therefore, we suggest removing ‘CLI’ to make the measurement more general to include both CLI measurement and channel measurement. In addition, for RSSI measurement, the measurement resource is a specific time and frequency resource. Therefore, we suggest adding ‘measurement resource’. We have many solutions to be studied for gNB-to-gNB CLI handing. To make this solution as optional for gNB CLI, we suggest the following change. For gNB-to-gNB co-channel CLI measurement, it can be enabled that the configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB) or measurement resource are needs to be exchanged between gNBs. |
| Huawei, HiSilicon | First, we tend to agree with ZTE that the gNB-gNB channel measurement is also important for gNB-gNB co-channel CLI handling. Moreover, we think the gNB-gNB co-channel CLI measurement should also cover spatial characteristics measurement of gNB-gNB co-channel CLI, which is beneficial for gNB-gNB CLI suppression at the receiver of victim gNB. Note that the above acutally has been brought up several times in previous discussions. We suggest the following modification to the proposal: For gNB-to-gNB co-channel CLI measurement including gNB-gNB channel measurement, gNB-gNB CLI RSSI/RSRP measurement and gNB-gNB CLI spatial characteristics measurement, study the neccessity and performance of sharing the configuration on the time/frequency/sequence/spatial information on the CLI-RS (e.g., NZP CSI-RS, CD-SSB, or NCD-SSB) needs to be exchanged between gNBs. |
| CATT | Support the current proposal. Channel measurement can be addressed via separate bullet |
| Samsung | We do not see the additional value of the moderator proposal when compared to the existing RAN1 agreements. We have already agreed that RAN1 assumes “exchange of configuration for NZP CSI-RS/SSB can be enabler” and that “both CD-SSB and NCD-SSB”. Configuration includes time/frequency/sequence/spatial information. The TDD system can function without additional & new gNB-to-gNB CLI measurements or can rely on gNB implementation-specific CLI estimation. In this sense, we can’t say the configuration needs to be exchanged. The next step would be to assess and quantify the benefits of the inter-gNB co-channel CLI measurements assuming gNBs can exchange configurations and if agreeable, document such observations in the SI TR. Agreement For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. - FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. RAN1#112 Agreement For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement. |
| Ericsson | Mostly agree with Huawei’s revision, although we would like to avoid the term “CLI-RS” since we are not inventing a new RS. Also, we think “configuration info” is sufficient, since different companies have different ideas on what should be shared. Similar to the comment from vivo, we don’t think configuration info on CD-SSB needs to be shared; it can be acquired by cell search. For gNB-to-gNB co-channel CLI measurement including gNB-gNB channel measurement, gNB-gNB CLI RSSI/RSRP measurement [and gNB-gNB CLI spatial characteristics measurement], study the potential need neccessity and performance of sharing the configuration info of on the time/frequency/sequence/spatial information on the CLI-RS measurement resource (e.g., NZP CSI-RS, CD-SSB, or NCD-SSB) needs to be exchanged between gNBs. Question to Huawei: Could Huawei please clarify what is meant by “gNB-gNB CLI spatial characteristics measurement?” Is this not captured already by “channel measurement”? |
| IDC | Support the proposal in general. As for the CLI-RS, since other reference signals (e.g., ZP CSI-RS) could also be used for CLI measurement, we suggest to add “at least” to the list of CLI-RS resources. That is: CLI-RS (at least NZP CSI-RS, both CD-SSB and NCD-SSB) |
| Lenovo, Motorola Mobility | Agree with New H3C that agreements so far do not preclude other types of CLI-RS or whether the existing RS needs enhancement. We support this proposal if slightly modified as: … (e.g., NZP CSI-RS, CD-SSB, NCD-SSB) … |
| QC | We are generally ok with the proposal. And we also agree with ZTE and HW that this could be useful for channel measurement as well. We think an easier way to capture it could be “gNB-to-gNB co-channel CLI/channel measurement”. And we have the same question as E/// on the part of “gNB-gNB CLI spatial characteristics measurement.” Regarding “spatial information” in the proposal, we think that is necessary to exchange. However, we want to clarify the meaning of the spatial information here. Does that align with our earlier agreement that “reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs”? We are fine if it is aligned. If it refers to additional info, then further discussion may be needed. |
| FL | The intention of the proposal is to capture the result of the study in the TR. In the previous agreements, ‘study, assume’ If companies thinks that the previous agreements are enough for capturing in the TR, this proposal seems unnecessary. Moderator Proposal #11-1 (configuration exchange) (2) Capture the text in the TR. For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline. Both CD-SSB and NCD-SSB can be used. It is assumed that exchange of configuration for NZP CSI-RS/SSB can be an enabler for gNB-to-bNB co-channel CLI measurement and/or channel measurement. |

Moderator Proposal #11-2 (Performance metric) (2)

For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement baseline metric for the study

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, Panasonic, NTT DOCOMO,OPPO, TCL,vivo, ZTE, CMCC, Xiaomi, Sony, CATT, NEC, Nokia/NSB, IDC, Intel, Lenovo/MM, QC |
| Not support | Samsung, Ericsson |

| Companies | Views |
| --- | --- |
| ZTE | We support this proposal due to both RSRP and RSSI have the its own applicable scenario. |
| Huawei, HiSilicon | To us, the following can be studied for gNB-gNB co-channel CLI measurement - For gNB-gNB co-channel CLI handling, the gNB-gNB CLI handling schemes based on gNB-gNB RSRP and RSSI measurement can be studied considering at least the following - Measurement resources which enables the gNB-gNB RSRP/RSSI measurement - Note: The RSRP and RSSI measurement may be up to gNB implementation. As also replied for proposal #11-1, channel measurement and gNB-gNB CLI spatial characteristics measurement are also important, if we could reach consensus the proposed modified proposal from us, maybe there is no need to agree on this proposal. |
| Samsung | We do agree that gNB-side RSRP based and/or RSSI based metrics are typical/expected measurements which the gNB-side measurement implementation would choose. We do not expect that a formal introduction and definition of such NG-RAN (=gNB-side) measurements is necessary, however. The moderator proposal should clarify if the gNB-side use of RSRP/RRSP is assumed for evaluation purposes in the study or if we actually intend to recommend new gNB-side measurements for CLI estimation in 38.215. |
| Ericsson | With Huawei’s revision of Proposal 11-1, this proposal seems not needed. We still want to better understand what is meant by “spatial characteristics.” See our question in the table below Proposal 11-1. |
| QC | Generally fine with the proposal. And suggested edit: For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement baseline metric for the study |
| FL | Thanks for the suggestion. |

Moderator Proposal #11-3 (UL rate-matching/resource muting) (1)

For enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement, UL rate-matching/resource muting can be applied. Following options can be studied.

- Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource, use UL muting patterns based on RB-level and symbol level muting)

- Option 2: Non-transparent UL resource muting method (e.g., define UL rate-matching/muting pattern with one or more RE muting patterns)

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, Panasonic, NTT DOCOMO,OPPO, TCL, CEWiT, Huawei, HiSilicon, Xiaomi, Sony (also consider puncturing in non-transparent mode), CATT, NEC, Nokia/NSB, Ericsson (with revision), IDC, Lenovo/MM, QC |
| Not support | Samsung |

| Companies | Views |
| --- | --- |
| Panasonic | We support Option 1. |
| Spreadtrum | We are OK with this proposal and think performance evaluation should be provided for those two options. |
| Vivo | We support option 1. There are different UL trasmisions, such as PUCCH, PUSCH, SRS, PRACH. It is impossible to support UL rate-matching/muting pattern for sequenced based PUCCH format, SRS and PRACH. For PUSCH, noting that only consective UL RA acllotion are supported for PUSCH considering low PAPR. We have concen on option 2 from both flexibility and complexity perspectives. |
| ZTE | First, we think what UL rate matching/muting resource is should be clarified so that people have the same understanding on the UL rate matching/muting. It is important to downselect the options. We prefer Option 2 because it is more efficient. For example, for CSI-RS measurement, not all the REs in the RB are used for CSI-RS transmission. For Option 2, the UL resource muting only around the CSI-RS and potential guard band is sufficient. It allows the UE can transmit uplink signals/channels on the remaining RE. It can improve the resource utilization. Option 1 cannot achieve this. |
| CMCC | From our understanding “use UL muting patterns based on RB-level and symbol level muting” in option 1 is also Non-transparanet method (with spec impacts) e.g.. the configuration of RB-level and symbol-level UL muting patterns |
| Huawei, HiSilicon | We are fine to study the two options further and support Option 2. For Option 1, UL resource muting can only be done on symbol and RB level which is less efficient than Option 2, i.e., RE level muting. As an example, transparent UL resource muting can be achived by not scheduling a certain UL symbol to the UE. However, this will degrade the UL coverage performance since the UE will lose the whole symbol for UL transmission. In addition, symbol-level muting may also lead to the unscheduled symbols before this muting symbol. This is because a UE cannot mute a specific symbol in the middle of the PUSCH. For example, to avoid the strong cross link interference caused by CSI-RS from an aggressor gNB, the UE needs to mute the symbol corresponding to the interference and symbols before and after the interference symbol, this will further degrade the uplink performance. For Option 2, we have provided some performance evaluations in our contribution R1-2302347 (section 2.3.2). It can be observed that the UL RE-level resource muting can help with the gNB-gNB CLI covariance matrix estimation which is beneficial for gNB-gNB CLI suppersssion at the victim gNB receiver. Note that this cannot be achieved by transparent UL resource muting. For example, the UL muting patterns based on RB-level and symbol level muting in current specification may not work since it is periodic and the UL scheduling may come at any time and it would be impossible to configure an efficient multing resource pattern which would help with gNB-gNB CLI covariance matrix estimation with moderate overhead. |
| Xiaomi | Support Option 2. gNB scheduling for transparent UL resource muting method will reduce the scheduling flexibility, especially for the periodic UL transmission scheduling. |
| Sony | In addition to rate matching, puncturing of the REs can also be done at the UE. |
| Catt | At this stage both options should be kept for FFS |
| NEC | We understand the intention of the proposal but what are we expecting by agreeing on Option-1? Because this is not expected to have any specification impact. So, I would suggest to only focus on Option-2 whether it is agreeable by companies or not. |
| Samsung | We do not see the additional value of the moderator proposal when compared to the existing RAN1 agreements. We have already agreed in RAN1 that proponents of UL resource muting can provide evaluation results to compare transparent and non-transparent. We have several technical concerns which make it overall questionable for us if further exploring UE non-transparent muting is meaningful to pursue. RE-level UL muting is high impact and results in high L1 UE modem impacts. Benefits of UE non-transparent UL muting depend on the penetration rate of later release UEs. RE-level UL muting is difficult to achieve with sub-carrier accuracy and within CP timing due to propagation and frequency-drift. Guard tones/symbols are required to absorb timing and frequency uncertainty. UE transparent muting (RB/symbol level) can rely on gNB scheduling and does not show such dependencies. It should also be considered that support for DFT-s-OFDM Ul transmissions is still required, where RE-level muting has challenges to respect the single-carrier properties and there is risk of higher resulting PAPR and reduced UL link budgets. Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not. |
| Ericsson | Okay to study the options. We think Option 1 is sufficient which can be done by gNB implementation. Option 2 clearly has specification impact, and this aspect should be studied. For enhancement of gNB-to-gNB co-channel CLI measurement, study the following options for UL rate-matching/resource muting including potential specification impact can be applied. Following options can be studied. - Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource, use UL muting patterns based on RB-level and symbol level muting) - Option 2: Non-transparent UL resource muting method (e.g., define UL rate-matching/muting pattern with one or more RE muting patterns) |
| Intel | In our view, Option 1 is sufficient. Moreover, we’d like to highlight that Option 1 may also include non-transparent but existing UL resource muting methods as well. For instance, the current specs supports the option of UL Cancellation Indication (UL CI) that can be used as well. Supporting RE-level (dynamic) rate matching for UL transmissions (e.g., PUSCH/PUCCH) as suggested by proponents of Option 2 would increase UE complexity significantly and not sufficiently justified for this purpose. |
| Lenovo, Motorola Mobility | Option 1 is preferred. |
| QC | From our understanding, UL muting is mainly for more accurate inter-gNB channel measurement, so suggest capturing it in the main bullet as: gNB-to-gNB co-channel CLI/channel measurement.” We are open to study both options at this stage. And E///’s edit below looks good to us: “study the following options for UL rate-matching/resource muting” |
| FL | Thanks for the suggestion and comment. |

    - [OPEN] Coordinated scheduling for time/frequency resources between gNBs

Summary

From RAN1#110 meeting to RAN1#112 meeting, there were discussions to determine which method(s) of gNB-to-gNB co-channel CLI handling is/are studied, and following(s) were agreed.

2. Coordinated scheduling for time/frequency resources between gNBs RAN1#110 Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange. RAN1#112 Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration

In RAN1#112-bis-e meeting, followings are proposed.

Time/frequency resource

TCL proposes exchanging the subbands pattern among gNBs to assist in mitigating gNB-to-gNB CLI during SBFD operation. The knowledge among gNBs about the SBFD time/frequency configuration may assist the gNBs to perform scheduling adaptation and mitigate the gNB-to-gNB co-channel CLI.  In addition to the SBFD time/frequency configuration, the exchanging of DL/UL subbands pattern may assist the gNBs for CLI mitigation. During the simultaneous existence of SBFD and dynamic TDD operations among gNBs, consider at-least the following information exchange among gNBs: TDD slot format, SBFD time/frequency configuration, Subbands pattern.

NEC thinks that for inter-gNB CLI mitigation, gNBs exchange with each other the UL subband frequency resource configuration and SBFD time occasions.

ZTE thinks that the related configuration (e.g., SBFD time/frequency, dynamic TDD) should be exchanged among gNBs for more accurate CLI measurement and more effective CLI handling.

Samsung thinks that for other gNB-to-gNB co-channel CLI enhancements, Xn/F1AP signaling can be used to indicate the intended SBFD time-/frequency-domain configuration of the NR TDD cell to other gNBs.

Schedulign inforamtion

CMCC proposes to enhance the backhaul signaling to exchange necessary information, e.g., scheduling information in time-domain, frequency-domain and power domain.

Qualcomm proposes that coordinated scheduling information for time/frequency/spatial domain can be exchanged via OTA or BH signalling for inter-gNB CLI mitigation. Qualcomm thinks that RAN 1 study semi-static or dynamic coordinated scheduling for inter-gNB CLI mitigation

OPPO thinks that coordinated scheduling between gNBs, more flexible configuration exchange over Xn/F1 interfaces should be studied, e.g. SBFD time/frequency configuration and TDD DL-UL configuration with periodicity longer than 10-ms.

DL resource blanking

Huawei thinks that for details of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB CLI handling, at least followings can be studied:

- DL resource blanking including time/frequency resource at aggressor gNB to avoid strong interference to UL DMRS

- UL resource restriction/blanking including time/frequency resources among gNBs to avoid UL performance degradation due to downlink CSI-RS etc

- Necessity of information exchange considering signaling overhead, latency and implementation flexibility

TCL thinks for coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider at least the following.

- RB based UL and DL Resource muting to support CLI mitigation in dynamic TDD and SBFD operation.

- Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation.

Figure 1 Scheduling adaptation of dynamic TDD (DL) at gNB1 and SBFD operation at gNB2

Spreadtrum proposes to use pseudo-sequence based muting scheme for inter-gNB CLI handling

CATT thinks that for details of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling, at least followings can be studied.

  - DL resource blanking including time/frequency resource at aggressor gNB

  - UL resource restriction including time/frequency resources among gNBs

  - Coordination of  SBFD configuration

Intel proposes that for coordinated scheduling for gNB-to-gNB CLI mitigation, study resource blanking and related information exchange between gNBs.

  - DL resource blanking at aggressor gNB help to protect the UL transmission at the victim gNB.

  - UL resource blanking at victim gNB can be supported by the existing mechanism on the UL resources that is interfered by the aggressor gNB.

  - Additional solutions for UL resource blanking by a transmitting UE may involve significant UE complexity and further justifications may be needed.

Qualcomm proposes coordinated scheduling on DL Tx restriction on UL resources between cells.

Sony thinks that blanking/restriction of resources for coordinated scheduling is not further considered unless the following concerns are addressed:

  - How does a gNB decides where and when to perform resource blanking/restriction?

  - How far ahead should a gNB blank/restrict a resource?

Xiaomi thinks that the RMP(rate matching pattern) can be considered with potential enhancement to support UL reserved resource indication.

Spatial domain, Power domain

Lenovo proposes a coordination/matching of TDD DL/UL on certain slots/symbols for use of high-interference beams to enable coordinated scheduling/beamforming. This information can be exchanged by adding spatial parameters to the Intended TDD DL-UL Configuration IE.

NEC proposes that information exchange, such as DL beam scheduling information and DL transmission power information, between gNBs is supported for coordinated inter-gNB scheduling

New RS

Sony thinks that Introduce new RS that can be used as Over-The-Air (OTA) physical layer signalling between gNBs for scheduling coordination. And the gNB-gNB RS is used to indicate the Slot & SBFD Format of the gNB transmitting the RS. The gNB-gNB RS is used to indicate L1 priority of a scheduled transmission.

From the input in documentations, following can be summarized.

Majority view is that exchanging SBFD time/frequency configuration among gNBs is necessary. In addition, DL resource blanking could be an enabler to mitigate co-channel gNB-to-gNB CLI.

For coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling, intended TDD-UL-DL configuration and/or intended SBFD time/frequency configuration can be exchanged among gNBs. These exchang information seems to be used for UE-to-UE co-channel CLI handling.

DL resource blanking including time/frequency resource at aggressor gNB can be further studied.

Proposals

Moderator Proposal #12-1 (1)

For coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB and/or UE-to-UE co-channel CLI handling, exchange of intended TDD-UL-DL configuration and/or intended SBFD time/frequency configuration can be an enabler. exchanged among gNBs.

Informaiton exchange for DL resource blanking including time/frequency resource at aggressor gNB can be studied further.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C,OPPO, TCL, CEWiT, Huawei, HiSilicon, Sony (support SBFD config exchange), CATT, NEC, Nokia/NSB (first sentence), IDC, Lenovo/MM (first paragraph), QC |
| Not support | Sony (do not support DL resource blanking), Samsung, Ericsson (needs clarification; DL resource blanking is separate topic) |

| Companies | Views |
| --- | --- |
| vivo | For DL resource blanking, it seems that the motivation to support the blankig is different for different companies. We hope FL/proponents can explain more regarding about this. |
| ZTE | We think more clarification on the intended TDD-UL-DL configuration is needed. Is it equal to the actual TDD-UL-DL configuration? In addition, is the intention that RRC configured TDD configuration is exchanged. If yes, we suggest removing the ‘intended TDD-UL-DL configuration and/or’ because it has been supported by current RAN3 spec. |
| CMCC | The “intended TDD-UL-DL configuration” has been supported in Rel-16. Does this proposal is to agree the informantion exchange of “intended SBFD time/frequency configuration” is supported in this SI? |
| Huawei, HiSilicon | It is not clear what is the difference between the intended TDD-UL-DL configuration and the TDD UL-DL configuration specified in Rel-16. For intended SBFD time/frequency configuration, this seems to imply that the SBFD time/frequency configuration will changes dynamically in time within a cell. We are not sure there is any agreement this will be supported even for dynamic SBFD operation as discussed in AI 9.3.2. For DL resource blanking including time/frequency resource at aggressor gNB, we propose to study the detailed scheme then decide whether the information exhcanege is needed nor not. One potential scheme is to study the DL symbol muting on UL DMRS which helps with the UL channel estimation so that the UL performance can be improved. Another potential scheme is that the aggressor gNB can use DL resource blanking to avoid strong CLI to periodic UL transmissions. |
| Xiaomi | The definition of intended TDD-UL-DL configuration and intended SBFD time/frequency configuration is not clear to us. More clarification would be appreciated. As for the DL resource blanking, we think it is a separate topic and should be discussed separately. |
| Sony | We support exchanging SBFD configuration information. We do not support DL resource blanking without addressing the following questions: - How does a gNB decides where and when to perform resource blanking/restriction? - How far ahead should a gNB blank/restrict a resource? The reason for this is, DL resource blanking will significantly impact URLLC operations. |
| CATT | We also think the following should also be included: UL resource restriction |
| Nokia/NSB | We share similar concern with other companies on DL blanking. Would the second sentence mean that it is FFS? Should we make it clear by adding it as FFS so that there won’t be ambiguity in interpreting it in the future. Thank you! |
| Samsung | We do not see the additional value of the moderator proposal when compared to the existing RAN1 agreements. We have already agreed in RAN1 that we can assume that the “intended SBFD time/frequency configuration can be exchanged among gNBs.” The “Intended TDD-UL-DL configuration” can already be indicated as by existing NR specs. We don’t need to re-agree. If would be more useful to identify and agree upon additional information to be included. Regarding the last sentence in the moderator proposal, it is not clear to us what “resource blanking” is intended to convey. Do we mean existing DL rate-match pattern(s) configurable for the UE, or rather time/frequency resources where gNBs indicate they do not expect to transmit? |
| Ericsson | Agree with other companies’ comments that “intended” is not clear, and it is not clear how this is different than Rel-16 (for TDD). Agree with comment from Huawei that for SBFD this proposal implies that dynamic SBFD is supported, and that is not agreed – it is currently being studied in 9.3.2. We think DL resource blanking is a separate topic and does not belong with this proposal. “can be exchanged” is too strong. As with other proposals, the wording should be “potential need and benefit of exchange can be studied” |
| Intel | It seems the first parts on TDD and SBFD configuration exchanged are already specified or agreed. We could focus only on the last part. |
| Lenovo, Motorola Mobility | These are two proposals. We support the first proposal here and suggest discussing the second proposal separately. |
| QC | First of all, for the first bullet, we want to clarify the additional information compared to our earlier agreement. “intended TDD-UL-DL configuration and/or intended SBFD time/frequency configuration can be exchanged among gNBs” has been agreed before. Is this the additional value here to use this for UE-to-UE CLI handling as well? Secondly, from our understanding, the second bullet is refer to that gNB restricts the DL Tx on certain time/frequency resource at aggressor gNB for CLI reduction purpose, correct? We are fine then to study this. Suggested minor edit: Information exchange for DL resource blanking/restriction including time/frequency resource at aggressor gNB can be studied further. |

    - [OPEN] Spatial domain coordination method for gNB-to-gNB CLI handling

Summary

From RAN1#110 meeting to RAN1#112 meeting, there were discussions to determine which method(s) of gNB-to-gNB co-channel CLI handling is/are studied, and following(s) were agreed.

3. Spatial domain coordination method for gNB-to-gNB CLI handling RAN1#110 Agreement Study the feasibility and potential benefits of spatial domain coordination method for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details for spatial domain coordination - Relevant information exchange Note1: Study can include method for FR1 and FR2. RAN1#110-bis-e Agreement For details of spatial domain coordination method for gNB-to-gNB co-channel CLI handling, at least followings can be studied. - Recommended/restricted Beams between gNBs - Beam nulling between gNBs - Beam pairing between gNBs - Other schemes are not precluded.. RAN1#111 Agreement For spatial domain coordination, the exchange of beam related information among gNB(s) (e.g., victim gNB(s) and aggressor gNB(s)) can be an enabler for inter-gNB co-channel CLI management. - For example 1 (from aggressor gNB to victim gNB), DL beam indication from aggressor gNB(s) - For example 2 (from victim gNB to aggressor gNB), preferred/restricted DL beam and associated resource configuration, beam based inter-gNB co-channel CLI measurement result from victim gNB - FFS: how to define DL beam indication - FFS: how to define DL beam Note: The above examples are only provided as starting point for further discussions Agreement For gNB-to-gNB co-channel CLI handling, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered for study. RAN1#112 Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs

In RAN1#112-bis-e meeting, followings are proposed.

Channel measurement and beam nulling

Huawei thinks that gNB-to-gNB instant channel is needed for beam nulling to suppress the gNB-to-gNB co-channel cross link interference. Huawei proposes to study solutions for channel measurement among multiple gNBs to enable beam nulling.

Preferred/non-preferred DL beam of the aggressor gNB, and beam information

Huawei proposes to study solutions for beam pairing considering CLI strength of beam pair over a threshold, Preferred Tx beams for each receive beam at the victim cell, Necessity of information exchange considering signaling overhead, latency and implementation flexibility.

TCL thinks that the information exchange of the preferred/restricted DL and UL beams of the aggressor and victim gNBs with each other, based on the beam ID and TCI state.

New H3C thinks that the beam information exchange can be handled by a central controller. The beam information consists of gNB ID+CLI measurement configuration which including the signal resource ID. For CSI-RS for CLI measurement, a dedicated indication, such as cli-info, can be introduced in the CSI-RS resource configuration to indicate the usage of this CSI-RS resource.

New H3C thinks All the CLI results of all beams should be reported in full report mode, while preferred beam set and non-preferred beam set are reported in partial report mode. The periodic or event-triggered report can be also used for the beam based CLI report. The central controller determines the non-preferred beam or preferred beam for aggressor gNB according to the dedicated algorithms. The number of the non-preferred beam for one aggressor gNB should not exceed maximum value. A restriction window can be introduced, where the aggressor gNB cannot use the non-preferred beams, but the victim gNB can use any beam. Several restriction window can be configured, but only one is active. The measurement window is periodic, and determined by the length, periodicity and offset.

Vivo thinks that assistance information exchange among gNBs can be considered for gNB-to-gNB CLI handling, including measurement reports of RSRP/RSSI, scheduling information.

Spreadtrum proposes to study the benefit and the procedure of information exchange of preferred/no-preferred DL beams considering the following

  - Determine preferred/non-preferred DL beams based on beam level RSRP or RSRQ measurements

  - A threshold can be used to determine preferred/non-preferred DL beams

ZTE thinks that a common understanding of the overall framework of spatial domain gNB-to-gNB CLI coordination should be made firstly. Rel-18 dynamic/flexible TDD can consider the following framework for CLI management,

  - Step 0: The victim identifies gNB-to-gNB CLI based on measurement of reference signal from the aggressor (e.g., SSB, CSI-RS or other measurement resource);

  - Step 1: The victim indicates interference information identified from Step 0, e.g., index of high-interference beam, channel state information for the interference channel, etc, to the aggressor via either air interface or backhaul;

  - Step 2: The aggressor and/or victim start to perform CLI handling schemes;

  - Step 3: The victim measures the reference signals sent by the aggressor to evaluate the CLI handling effect;

  - Step 4: The victim feedbacks the CLI mitigation effect of the different CLI handling schemes.

Spatial domain coordination can be considered by aggressor gNB and/or victim gNB for handling gNB-to-gNB CLI, e.g.,

  - Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information,

  - Resources to be used by the aggressor for downlink Tx and resources to be used by the victim for uplink Rx are determined according to the preset (or preconfigured) time domain pattern.,

  - Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, e.g., beam nulling.

Intel thinks that in addition to the preferred/not-preferred Tx/DL beams of the aggressor gNB that can be signalled from a potential victim gNB to a potential aggressor gNB, the intended Tx/DL beams or beam nulling information of aggressor gNB can be signalled from a potential aggressor gNB to a potential victim gNB.

Xiaomi thinks that Rx beam can be indicated by the associated RS (e.g., CSI-RS, SSB, SRS) for gNB-to-gNB CLI management. The restricted/recommended beam pairs, i.e., restricted/recommended Rx beams for victim gNB and restricted/recommended Tx beams for aggressor gNB, should be configured for gNB-to-gNB CLI mitigation.

Lenovo proposes victim gNB indicating high-interference (non-preferred) beams to the aggressor gNB or the core network and the victim gNB reporting the amount/level of excess interference corresponding to the high-interference beams. Also, victim gNB indicating preferred and high-priority Tx beams to the aggressor gNB, and aggressor gNB indicating information of using high-interference beams to victim gNBs are proposed.

Samsung thinks that for gNB-to-gNB co-channel CLI spatial domain enhancements, Xn/F1AP signaling is extended to indicate the reference signal resource ID (NZP-CSI-RS resource ID and SSB index) from the aggressor gNB to the victim gNB and to indicate the preferred/non-preferred DL beams of an aggressor gNB from the victim gNB to the aggressor gNB.

NTT DOCOMO thinks that information to be exchanged among gNBs should include spatial domain information.

Qualcomm proposes to investigate schemes for inter-gNB CLI mitigation in dynamic/flexible TDD and SBFD to identify compatible inter-gNB beam pairs, which can be based on inter-gNB CLI measurement and reporting per candidate DL/UL beam pair. If measurement CLI RS is configured with RS resource repetitions e.g. to allow victim gNB to scan different gNB Rx beams, then DL beam or DL beam indication can be represented via CSI-RS resource ID # plus repetition # with associated scrambling sequence ID / cell ID for inter-gNB co-channel CLI management. Qualcomm proposes that RAN1 to prioritize the study of example 2 in spatial domain coordination agreement for inter-gNB co-channel CLI management, to investigate measurement periodic or event triggered report with contents of allowed/disallowed (recommended/restricted) beams, to investigate related resources and corresponding required power backoff per allowed/disallowed beam. Furthermore, Qualcomm proposes that gNB adopts a slot-specific DL codebook restrictions, where a subset of PMI codebook is restricted in slots where a neighboring gNB has a conflicting traffic direction. Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding matrix For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots. For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Beam related coordination info can be sent between victim gNB and aggressor gNB

  - If the inter-gNB CLI RS is transmitted from aggressor gNB and measured by victim gNB, the coordination info can include allowed/disallowed aggressor gNB DL beam(s), corresponding Tx power backoff and time/frequency resources.

  - If the inter-gNB CLI RS is transmitted from victim gNB and measured by aggressor gNB, the coordination info can include the intended victim gNB UL beam(s), corresponding intended time/frequency resources and max allowed caused interference level.

Marity of companies are supporiting the exchange the preferred/non-preferred DL beam of an aggressor gNB from the victim gNB to the aggressor gNB. In addition, companies are thinking beam information (i.e., reference signal resource ID) needs to be indicated from the aggressor gNB to the victim gNB.

In case of SBFD opration, DL Tx beam could be different depending on the slot type (i.e., SBFD slot, non-SBFD slot). When two types of duplex operation (i.e., SBFD operation, TDD operation) are allowed, the victim gNB may need to report different DL Tx beam depending on the slot type. Furthermore, power domain method for CLI handling can be considered on top of the spatial domain schemes. For example, power back-off per beam could be requested to the aggressor gNB with preffered DL beam indication. In this aspect, followings can be studied further.

- Preferred/non-preferred DL bem Per slot/symbols (i.e., SBFD symbol, non-SBFD symbol)

- Power back-off per beam

Proposals

Moderator Proposal #13-1

For spatial domain handling of gNB-to-gNB co-channel CLI, Xn/F1AP signaling is extended to indicate the reference signal resource ID (NZP-CSI-RS resource ID and SSB index) from the aggressor gNB to the victim gNB and to indicate the preferred/non-preferred DL beams of an aggressor gNB from the victim gNB to the aggressor gNB.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, NTT DOCOMO, TCL,vivo, Sony, NEC, Nokia/NSB, IDC, Lenovo/MM, QC |
| Not support | Huawei, HiSilicon, Samsung, Ericsson, Intel |

| Companies | Views |
| --- | --- |
| Spreadtrum | We are generally fine with this proposal. But we want to confirm the necessarity of indication of the reference signal resource ID since it will be included in the configuration of CLI-RS which will be exchanged among gNBs. Or is it used for measurement and reporting based on aperiodic CLI-RS? |
| ZTE | We don’t think we need to spell out the interface name in RAN1 agreement because the RAN1 solution can be applied to any applicable scenario and the interface is out of RAN1 discussion. We just make it general, i.e., exchange between gNBs. In addition, we are not sure how perferred/non-preferred DL beam helps for gNB-to-gNB CLI. In our understanding, the victim gNB just reports the measurement result to the aggressor gNB as well as the corresponding beam. For example, the beam with higher interference and the beam with lower interference. The remaining work are left to aggressor gNB. |
| CMCC | What’s the difference from the following proposal? Moderator Proposal #11-1 (configuration exchange) For gNB-to-gNB co-channel CLI measurement, the configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB) needs to be exchanged between gNBs. |
| Huawei, HiSilicon | We are note sure about the increamental part of the above proposal compared to the RAN1#112 agreement below. Is it only the interface part? Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs |
| Xiaomi | We agree with ZTE that the interface signaling is out of RAN 1 discussion. Besides, we are also wondering how the indication of the preferred/non-preferred DL beams of an aggressor gNB from victim gNB and aggressor gNB works. In our understanding, the CLI level differs from different beam pairs (e.g., Rx & Tx beams). Even with same DL beam, the CLI level is also different with different Rx beams, e.g., Rx 2 & Tx 1 beam pair and Rx 0 & Tx 1 beam pair in following figure. Does it mean that the victim gNB indicates the preferred/non-preferred DL beams of an aggressor gNB corresponding to a certain Rx? If that is the case, how can the aggressor gNB to choose transmission DL beams with lower CLI as it does not know the current transmission Rx beams of victim gNB. |
| Sony | Similar view with ZTE, we don’t need to specify the backhaul. |
| Samsung | We do not see the additional value of the moderator proposals compared to the already existing RAN1#112 agreements. We have already agreed that for purpose of the study we can assume that the reference signal ID can be used. In the Rel-18 SID, we cannot make a decision to extend Xn/F1AP. This is RAN3. We can identify benefits and characterize system-level performance when assuming gNB-side spatial domain enhancements such as beam coordination, then document observations in the TR. Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. |
| Ericsson | Do not support. We also do not see the need for this proposal compared to what has already been agreed. Moreover, it cannot be agreed in an SI, especially without RAN3 involvement, that Xn/F1AP signaling is extended. |
| Intel | Same view as others that the ‘delta’ in this proposal compared to decisions from last two meetings seems negligible. |
| Lenovo, Motorola Mobility | Suggest breaking the items after “extended to” to bullet points for more clarity, or possibly splitting the proposal. |
| QC | This shall be up to RAN3 during WI phase. However, instead, we could say: RAN1 recommends exchanging “the reference signal resource ID (NZP-CSI-RS resource ID and SSB index) from the aggressor gNB to the victim gNB and the preferred/non-preferred DL beams of an aggressor gNB from the victim gNB to the aggressor gNB” over Xn or F1AP interference. |

Moderator Proposal #13-2

For spatial domain handling of gNB-to-gNB co-channel CLI, followings can be studied further.

- Preferred/non-preferred DL bem Per slot/symbols (i.e., SBFD symbol, non-SBFD symbol)

- Power back-off per beam

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Sony (with comments on time scale), QC |
| Not support | Samsung, Ericsson |

| Companies | Views |
| --- | --- |
| New H3C | Preferred/non-preferred DL bem Per slot/symbols will introduce more overhead in the Xn/X1AP interface, We prefer that the preferred/non-preferred beam applies to all slots/symbols, but introduce the restricted windows. During the active restricted window, the preferred/non-preferred beam apply, otherwise, all beams can be used. |
| Spreadtrum | Feasibility of DL beam Per slot/symbols should be studied first. |
| TCL | Share similar views with H3C regarding the increased overhead for preferred/non-preferred DL beams per slot/symbols. |
| vivo | Although Tx power reduction of aggressor gNB can reduce the CLI for victim gNB, it would aslo decrease the DL performance of it’s serving cell, which should be carefully studied. |
| ZTE | For the first bullet, we suggest changing ‘slot/symbol’ to ‘resource’ to make it broader since the different beams can be used for different frequency resource. For the second bullet, we suggest put the related discussion under the discussion of power control based solution. |
| CMCC | We support the study of exchange of Preferred/non-preferred DL beam, but it is not necessary to exchange the information per slot/symbol. |
| Huawei, HiSilicon | We would like to have some clarifications on the proposal to make sure we have the correct understanding: For the first subbullet, it is not clear to us why “Preferred/non-preferred DL beam” would be needed for non-SBFD symbols since there is no gNB-gNB CLI. Or is the proposal also targeting dynamic/flexible TDD? If so, there is no need to mention SBFD or non-SBFD symbols. For the second subbullet, we share similar view as ZTE that it belongs to power control-based solution, so it can be discussed in section 2.1.5. |
| Xiaomi | Except the overhead and feasibility as mentioned by other companies. The time-sensitivity is another issue. The slot/symbol-based beams are introduced based on the assumption that the preferred/non-preferred beams are changed via different slot/symbols. If that is the issue, preferred/non-preferred DL beams may be outdated considering that the delay for CLI measurement and beam indication, e.g., indicating the preferred/non-preferred DL beams of an aggressor gNB from the victim gNB to the aggressor gNB. |
| Sony | We think there need to be an expiry timer for this preferred/non-preferred beam, otherwise gNB would be stuck with a static list of preferred/non-preferred beam. Maybe the timescale on a per slot/symbol basis is too fine but nevertheless some aspect of time needs to be considered. |
| Nokia/NSB | Overhead brought by coordinating preferred/non-preferred beam per slot/symbols may be significant and can be seen as micro-optimization. In the end, it should be up to the aggressor gNB on whether/how to apply such preferred/non-preferred beams from victim gNB. |
| Samsung | We have questions to better understand the moderator proposal. (1) Is this proposal intended for both SBFD operation and df-TDD operation? For the SBFD case, what is the motivation to separately do beam coordination for the SBFD and non-SBFD symbols/slots? Based on our initial SBFD evaluation results, it appears un-warranted to attempt controlling inter-gNB/inter-UE interference in the system for the SBFD antenna configuration options separately on the symbols. If there is one strongest gNB aggressor out of many, it will dominate the interference paths every time it schedules. We can support preferred/non-preferred DL beam(s) when applied to all symbols. Since the gNB-to-gNB channel is mostly static, the preferred/non-preferred DL beam should not expected to be time-varying for relatively long time periods. (2) Is it the intend of “power back-off” to mandate gNB behaviour, i.e., require an aggressor gNB to back off power for X dB if high interference is indicated by a victim gNB? |
| Ericsson | Similar questions as above about whether this applies to SBFD of dynamic TDD. If SBFD, then no CLI in non-SBFD symbols. If dynamic TDD, then SBFD doesn’t need to be mentioned. Option 2 belongs in power domain discussion, not here. With these changes, it seems the proposal is not needed, since we have already agreed to study preferred/non-preferred beam solutions. |
| Intel | We are not convinced that the merits of per symbol/slot type indication are sufficient to justify the increased signalling complications and OH. As noted by Samsung, we do not expect much benefits in trying to optimize for differences in antenna configurations across SBFD/non-SBFD symbols for gNB-to-gNB links. |
| Lenovo, Motorola Mobility | Agree with ZTE to use “resource” in general for now. Open to discuss this proposal during the online session. |
| QC | 1). Would like to clarify the meaning of “Preferred/non-preferred DL bem Per slot/symbols” refer to “Preferred/non-preferred DL bem Per slot/symbol type”? e.g. non-perferred DL beam only for SBFD symbol and misaligned D-TDD symbol? 2) . We support the second bullet. It is a mixed feature that covers both spatial domain and power domain, so it makes sense to mention under Preferred/non-preferred DL beam proposal. Because the required power backoff per DL beam for aggressor cell could be different. With certain power backoff applied to a non-preferred DL beam, CLI could be under a threshold, and it could become an allowed DL beam for DL transmission. |

    - [OPEN] UE and gNB transmission and reception timing

Summary

From RAN1#110 meeting to RAN1#112 meeting, there were discussions to determine which method(s) of gNB-to-gNB co-channel CLI handling is/are studied, But there was no consensus about UE and gNB transmission and reception timing

In RAN1#112-bis-e meeting, followings are proposed.

Huawei thinks in current specification, the UL signal and downlink interference can be aligned (within CP) when proper TA offset is configured and/or proper overall timing of victim cell is applied. The necessity of further enhancement of UE and gNB transmission and reception timing is not clear.

Vivo thinks Transmission and reception timing adjustment can be supported in Rel-18 dynamic/flexible TDD to accurately estimate interference channel and effectively suppress CLI from aggressor gNB. For transmission and reception timing adjustment, victim gNB should adjust transmission timing of the served UEs to align with DL transmission signal arrival of aggressor gNB. A negative TA can be configured for UEs served by victim gNB. The timing adjustment is slot specific.

Spreadtrum proposes to study the necessity and benefit of adjusting the TA offset to resolve transmission and reception timing misalignment in gNB-to-gNB CLI measurement.

ZTE thinks RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary of the victim gNB and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Figure-3: Timing difference between different gNBs

Intel proposes to study timing synchronization assistance information exchange between gNBs to enable improved estimation of timing offsets between neighboring gNBs to enable better CLI estimation and its management.

Sony proposes to add a time alignment offset TUL to the overall timing advance, TTA = NTA + NTA,offset + TUL for UL transmissions so that the UL transmission is OFDM symbol aligned with any inter gNB DL CLI at the victim gNB’s receiver and at the same time provide sufficient time gap at the UE between the end of an UL transmission and the start of a DL reception for UL to DL switching.

Nokia proposes to study the limitations and trade-offs of adjusting the TA offset including the potential backward compatibility problems between legacy UEs and Rel-18 UEs.

CMCC thinks UE and gNB transmission and reception timing alignment can be further studied, e.g., set N_(TA,offset)=0 via information n-TimingAdvanceOffset or define negative N_(TA,offset).

Qualcomm proposes to investigate how to determine inter-gNB CLI RS Tx/Rx timing for accurate inter-gNB CLI measurement. Also, Qualcomm thinks that inter-gNB CLI can be mitigated by coordinating and configuring slot-specific TA. For SBFD, TA configured for SBFD slots can be different from those configured for HD slots. And for dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Simultaneous UL reception and inter-gNB CLI measurement can be achieved by configuring UE with zero or negative TA.

From the inputs in documentation, it is observed that the majory view is that the receving timing aligment at victim gNB between OFDM symbol from a UE and OFDM symbol from aggressor gNB is an enabler to accurately estimate co-channel gNB-to-gNB CLI at victim gNB and effectively suppress CLI from aggressor gNB. Following can be studied.

- Option 1: Proper TA offset is configured to the UEs.

- Option 2: Timing synchronization assistance information exchange between gNBs

Proposals

Moderator Proposal #14-1

Receving timing aligment at victim gNB between OFDM symbol from a UE and OFDM symbol from aggressor gNB is an enabler to accurately estimate co-channel gNB-to-gNB CLI at victim gNB and effectively suppress CLI from aggressor gNB. Following can be studied.

- Option 1: Proper TA offset is configured to the UE

  - FFS: additional TA offset or negative TA offset

- Option 2: Timing synchronization assistance information exchange between gNBs

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | TCL,vivo, CMCC, Nokia/NSB, Ericsson (with revision), IDC, Intel (also OK with Ericsson’s version). QC with edit |
| Not support | Samsung |

| Companies | Views |
| --- | --- |
| Spreadtrum | Evaluation on the degradation introduced by receving timing misalignment should be provided before study on it. |
| ZTE | For option 1, it is not clear whether a proper TA offset is feasible to align the receiving timing of the UL signal and signal from aggressor gNB, especially considering the legacy UE in the system. For option 2, the details of the assistance information should be clarified to see whether it works. So we suggest to add an FFS under option 2. For example, ‘FFS: details of timing synchronization assistance information’ At this stage, we think another solution should also be considered to reduce the impact to the legacy UE. For example, some special design for the measurement RS to handle the receiving timing misalignment. The measurement RS with longer CP similar to RIM-RS can be considered as another option. Option 3: RS enhancement to handle receiving timing misalignment |
| Huawei, HiSilicon | For Option 1, in current specification, the UL signal and downlink interference can be aligned (within CP) when proper TA offset is configured and/or proper overall timing of victim cell is applied. The necessity of further enhancement of UE and gNB transmission and reception timing is not clear. In addition, we understand the current gNB synchronization requirement for TDD is loose, i.e., up to 3us among different gNBs. However, we are not sure the discussion on potential enhancements should be conducted considering this loose requirement which are much better in realistic deployment. The timing synchronization assistance information in Option 2 is not clear to us. In addition, is the intention to change the DL Tx timing at the aggressor gNB? |
| Xiaomi | For the additional TA offset in Option 1, does it mean zero TA offset. If this is the case, clearer clarification would be appreciated. For Option 2, we prefer not to support it. Even the victim gNB can derive the arriving time of CLI RS from aggressor gNB by information exchange, the victim gNB has to determine another reception timing for CLI RS, except the reception timing for UL data. It brings additional complexity at gNB side. |
| Sony | Does the TA offset in Option 1 refers to the NTA,offset parameter or just an additional TA offset in general? We have concerns in messing around NTA,offset with as it has backward compatibility issues and may cause self interference in DL slot. It isn’t clear why there is something that needs to be received for timing alignment, what is being received? Is it some control information that the victim gNB needs to receive? I don’t think we need to list the options and if we have to, make it more general, the following is a suggested reivision: Timing alignment between the victim gNB UL OFDM symbol from a UE and DL OFDM symbol from aggressor gNB is an enabler to accurately estimate co-channel gNB-to-gNB CLI at victim gNB and effectively suppress CLI from aggressor gNB. Following can be studied. - Option 1: Enhancement to Timing Advance - Option 2: Timing synchronization assistance information exchange between gNBs |
| Samsung | We have a question to better understand the moderator proposal. (1) What type of “Timing synchronization assistance information exchange” is implied by Option 2? What would be typically expected frequency of the inter-gNB information exchange and what timing accuracy would be targeted? |
| Ericsson | As we discussed offline at the end of last meeting, we are okay to study the issue, but the proposal needs to be more clearly written, and we prefer to leave it general for now to study the impact of misalignment first, then potential solutions could be discussed later. We suggest the following generic proposal: Modified proposal: For gNB-gNB cochannel CLI measurement, study the impact on CLI measurement accuracy at victim gNB due to misalignment between UL timing at victim gNB and timing of DL reception at victim gNB of CLI measurement resource transmitted from aggressor gNB. |
| Intel | To clarify our thoughts on Option 2 (“timing synchronization assistance information exchange”, this can simply include indication of timing offsets for DL timing in each cell with respect to a reference time. The reference time may be coordinated within few to 100s of nanoseconds depending on particular PTP version used, etc. Such coordination allows for operation with some misalignment but the assistance information can be used by a measuring gNB to better position its Rx window, e.g., for CLI-RS reception for estimation of the cross-link. At this point, we can also accept the simplified version from Ericsson. |
| QC | We are fine with E/// edit of making it general at this first stage for the study. |

    - [OPEN] Power control based solution

Summary

From RAN1#110 meeting to RAN1#112 meeting, there were discussions to determine which method(s) of gNB-to-gNB co-channel CLI handling is/are studied, But there was no consensus about power control based solution.

In RAN1#112-bis-e meeting, followings are proposed.

UL power control

Vivo thinks that enhanced UL power control can be considered, e.g., different power control parameters can be used depending on resource allocation or the existence/strength of the CLI.

MediaTek proposal to study the feasibility of enabling two UL power control loops for gNB-gNB CLI handling in DTDD and SBFD.   Also, it is proposed to support the use of a bitmap for slot indication to the UE when two UL power control loops are enabled for gNB-gNB CLI handling in DTDD and SBFD.

Intel proposes to study configuration of separate open-loop power control parameters in different slot or symbols depending on the fixed or dynamic/flexible UL resource allocation.

Qualcomm thinks that Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific power control parameters. For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots. For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

DL power control

Xiaomi thinks that the power adaptation schemes to alleviate the CLI issue can be further studied.

Nokia thinks that enhancements on the signaling between gNBs is required to inform about the desired power reduction at the aggressor(s) cells. Also, it is mentioned that the IAB concepts of Desired DL Tx power adjustment and DL Tx power adjustment can be used as a starting point.

Qualcomm proposes gNB requesting another gNB to have X dB power backoff on time/frequency/spatial resources to mitigate inter-gNB CLI.

From the input in documentation, followings can be summaries.

Separate open-loop power control parameters in different slots/symbols for the uplink power control of a UE in victim gNB can be an enabler to overcome the co-channel gNB-to-gNB CLI. Following options can be studied.

- Option 1: Legacy parameters for openloop power control can be reused.

- Option 2: Additional parameters for openloop power control can be introduced.

DL Tx power adjustment of aggressor gNB can be an enabler to mitigate the co-channel gNB-to-gNB CLI. The IAB concepts of Desired DL Tx power adjustment and DL Tx power adjustment can be used. Following options can be studied.

- Option 1: gNB requests to another gNB for desired DL Tx power adjustment on specific time/frequency resources.

- Option 2: gNB informs DL Tx power adjustment to the UEs in a serving cell

Proposals

Moderator Proposal #15-1

Separate open-loop power control parameters in different slots/symbols for the uplink power control of a UE in victim gNB can be an enabler to overcome the co-channel gNB-to-gNB CLI. Following options can be studied.

- Option 1: Legacy parameters for openloop power control can be reused.

- Option 2: Additional parameters for openloop power control can be introduced.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C,OPPO, SPRD, TCL, ZTE, CMCC, Xiaomi, Sony, Nokia/NSB, Intel (also fine with the version from HW-HiSi) |
| Not support | Huawei, HiSilicon,CATT, Samsung, Ericsson (overlap with 9.3.2) |

| Companies | Views |
| --- | --- |
| New H3C | We support this proposal. From our point of view, we don’t see any necessary to introduce the additional parameters, the legacy parameters are enough. |
| Spreadtrum | We are fine with this proposal and prefer Option 1. |
| vivo | What is difference between the proposal and power control in current spec? |
| ZTE | We are fine with this proposal. |
| Huawei, HiSilicon | We think it is bit too strong to say that separate open-loop power control parameters in different slots/symbols can be an eabler to overcome the co-channel gNB-to-gNB CLI. Some futher studied can done. For example, for separate open-loop power control parameters in different slots/symbols for the uplink power control, although it may have benefit on the serving UE, however, it may also cause more interference on other UEs. It is not clear whether/how much gain it can be achieved. We suggest the following update to the proposal Study the performance and specification impact of applying separate open-loop power control parameters in different slots/symbols for the uplink power control of a UE in victim gNB can be an enabler to overcome the for co-channel gNB-to-gNB CLI considering the following options can be studied. - Option 1: Legacy parameters for openloop power control can be reused. - Option 2: Additional parameters for openloop power control can be introduced. |
| Sony | We have a preference for Option 1 but I take it we are not down selecting in this proposal so we are ok with this proposal. |
| CATT | Generally negative for this aspects as it will degrade system performance |
| Nokia/NSB | We prefer Option 1. |
| Samsung | We cannot support the moderator proposal. Proper use of UE UL transmit power control can be an enabler to improve inter-gNB CLI estimation by the victim gNB but does not overcome the co-channel gNB-to-gNB CLI, i.e., the DL transmissions from the aggressor gNB. For the proposed Option 2, what kind and type of “additional open-loop power control parameters” are intended? Existing NR open-loop parameter sets include the (normalized) nominal target receive power P0 and the fractional pathloss compensation coefficient alpha parameters. Do we mean new parameters or do we mean more flexibility in signalling from the open-loop sets? |
| Ericsson | Agree with comments from Huawei. Moreover, the issue of separate power control parameters for SBFD and non-SBFD slots is being discussed in 9.3.2. Hence, we think the discussion should not be repeated in two places. Agreement (from AI 9.3.2): Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation |
| Intel | On overlap with AI 9.3.2, in our understanding, the proposal here should be about “fixed” vs. “flexible” symbols in context of dynamic/flexible TDD operation. |
| QC | We would like to clarify that legacy parameters are P0, alpha, etc. and what does it mean by additional parameters? |

Moderator Proposal #15-2

DL Tx power adjustment of aggressor gNB can be an enabler to mitigate the co-channel gNB-to-gNB CLI. Following options can be studied.

- Option 1: gNB requests to another gNB for desired DL Tx power adjustment on specific time/frequency resources.

- Option 2: gNB informs DL Tx power adjustment to the UEs in a serving cell

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, TCL, NEC, IDC, QC |
| Not support | Huawei, HiSilicon, CATT, Samsung, Ericsson |

| Companies | Views |
| --- | --- |
| vivo | Although Tx power reduction of aggressor gNB can reduce the CLI for victim gNB, it would aslo decrease the DL performance of it’s serving cell, which should be carefully studied. |
| ZTE | We think the study of DL Tx power adjustment should be careful. It may affect the DL coverage to all the UE, including the legacy UE. In our understanding, DL Tx power adjustment has less/limited help for the gNB-to-gNB handling. We prefer to de-prioritize this study. It seems the two options are different steps of one solution, i.e., first, the gNB request another gNB to adjust the DL Tx powe in the specified resource. Then, the next step is gNB informs the adjusted Tx power ot the UE. |
| CMCC | The two options are not in the same level. The option 1 is the enabler to mitigate inter-gNB CLI, while the Option 2 is consequent hebaviours if agreesor gNB adjust its DL TX power but not the inter-gNB CLI handling enabler. |
| Huawei, HiSilicon | First of all, we feel it is a bit too strong to make the agreement that DL Tx power adjustment of aggressor gNB can be an enabler to mitigate the co-channel gNB-to-gNB CLI without detailed discussions on the evaluation in AI 9.3.1. Secondly, it is not clear what channels/signals are discussed in the proposal. Is it for PDSCH or other DL signals/channels? We also share some concern as ZTE that reducing DL Tx power would degrade the DL performance which is typically not recommended from DL coverage performance perspecitive. Finally, Option 1 and Option 2 are more related to signalling procedure in case reduce the gNB DL Tx power is beneficial which can be discussed in the WI phase. |
| Nokia/NSB | One question for clarification, gNB in Option 2 is victim gNB or aggressor gNB? It doesn’t seem like Option 2 is needed (or being alternative to Option 1), could you please help us to understand? Thank you. |
| Samsung | From the network deployment perspective, we do not expect that an operator will set gNB transmit power taking into account of victim gNBs. From the NR specification perspective, it is hard to envision that we can or should mandate gNB behaviour. When looking at the Rel-17 eIAB or Rel-18 NCR cases, it might be possible to allow for DL EPRE decoding assumptions, but context might to be considered. Regarding Option 2 where gNB informs DL Tx power adjustment to the UEs in a serving cell, what is the benefit of the UE knowing the DL Tx power restriction/limitation of the (non-serving) aggressor gNB? Does the UE participate in the gNB-side inter-gNB CLI estimation? To which DL channels/signals would the proposed Option 2 apply? |
| Ericsson | Agree with above comments, especially too strong to say “can be an enabler” given impact on DL. For Option 2, why is it needed to inform the DL Tx power to the UE? DL power is gNB implementation, and it is not specified that gNB indicates the power to UEs. |
| QC | We support to study the DL Tx power adjustment solution. And we need to clarify the two options are not mutual exclusive options. |

  - 2nd Round Discussion

    - gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

    - Coordinated scheduling for time/frequency resources between gNBs

    - Spatial domain coordination method for gNB-to-gNB CLI handling

    - UE and gNB transmission and reception timing

    - Power control based solution

  - 3rd Round Discussion

    - gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

    - Coordinated scheduling for time/frequency resources between gNBs

    - Spatial domain coordination method for gNB-to-gNB CLI handling

    - UE and gNB transmission and reception timing

    - Power control based solution

- UE-to-UE inter-cell co-channel interference

1 UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

2 Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling

3 Spatial domain coordination method for UE-to-UE co-channel CLI handling

4 UE and gNB transmission and reception timing

5 Power control based solution

From RAN1#110 to RAN1#112), there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied and followings were agreed.

1 UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB. RAN1#110-bis-e Agreement For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s) - For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement - FFS potential enhancements Agreement For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting - Note: Accounting for UE processing/reporting delay – companies to share their assumptions - Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result - Note: Accounting for information exchange delay between gNBs (if applicable). RAN1#111 Agreement For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: - For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting. - FFS: Event triggered reporting. - For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements. RAN1#112 Agreement For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics. Agreement For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied. Agreement For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded. 2 Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange (if needed). 3 Spatial domain coordination method for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes: - Details for spatial domain coordination by gNB - Relevant information exchange (if needed) Note1: Study can include method for FR1 and FR2. 4 UE and gNB transmission and reception timing So far, no consensus 5 Power control based solution Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline 6 Advanced Receiver RAN1#110-bis-e Conclusion Under AI 9.3.3, no further discussion on UE side advanced receiver for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD. 7 Sensing based mechanism RAN1#110-bis-e Conclusion No further discussion for sensing based mechanism (i.e. LBT) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD.

  - 1st Round Discussion

    - [HOLD] UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

Summary

From RAN1#110 to RAN1#112, there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied and followings were agreed.

1 UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB. RAN1#110-bis-e Agreement For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s) - For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement - FFS potential enhancements Agreement For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting - Note: Accounting for UE processing/reporting delay – companies to share their assumptions - Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result - Note: Accounting for information exchange delay between gNBs (if applicable). RAN1#111 Agreement For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: - For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting. - FFS: Event triggered reporting. - For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements. RAN1#112 Agreement For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics. Agreement For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied. Agreement For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded.

In addition, in RAN1#112 meeting, the final proposal was discussion in 3rd round, but there was no conclusion due to lack of discussion time.

Moderator Proposal #21-3 (1) For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the benefit of applying narrower frequency granularity of CLI measurement reporting.

In RAN1#112-bis-e meeting, followings are proposed.

TO BE UPDATED

Proposal

Moderator Proposal #21-1

.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support |  |
| Not support |  |

| Companies | Views |
| --- | --- |

Moderator Proposal #21-2

.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support |  |
| Not support |  |

| Companies | Views |
| --- | --- |

Moderator Proposal #21-3

.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support |  |
| Not support |  |

| Companies | Views |
| --- | --- |

    - [CLOSE] Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling

Summary

From RAN1#110 to RAN1#112, there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied and followings were agreed.

2 Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange (if needed).

In RAN1#112-bis-e meeting, followings are proposed.

Huawei thinks that for details of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling, at least DL/UL resource blanking/reservation/muting including time/frequency resource can be studied. Besides, followings can also be studied.

- Potential impact of traffic load.

- Necessity of information exchange considering signaling overhead, latency and implementation flexibility

CATT think that for details of coordinated scheduling for time/frequency resources between gNBs for UE-to-UE co-channel CLI handling, DL/UL resource blanking/reservation/muting can be studied.

Intel proposes to study coordinated scheduling schemes focusing on:

- Inter-gNB information exchange on user selection.

- Inter-gNB information exchange on DL/UL resource blanking/reservation/muting.

- Inter-gNB information exchange on scheduled PRBs, subbands, etc.

- Assistance information between UE and gNB to facilitate coordinated scheduling.

CMCC proposes to support to enhance the backhaul signaling to exchange necessary information, e.g.,

- Example 1 (2-step negotiation): CLI-SRS resource configuration and the request for scheduling avoidance of the aggressor UE (associate with certain CLI-SRS index) at certain pre-configured resources in time/frequency domain

- Example 2 (3-step negotiation): CLI-SRS resource configuration, the request for scheduling information of the aggressor UE (associate with certain CLI-SRS index) and the information of pre-choregraphed scheduling information of the aggressor UE (associate with certain CLI-SRS index)

- Example 3 (1-step negotiation): CLI-SRS resource configuration and the corresponding pre-configured candidate DL resources subset for the associated aggressor UE

Qualcomm thinks that coordinated scheduling information for time/frequency resources and corresponding UE information can be exchanged via OTA or BH signalling for inter-UE CLI mitigation.

    - [OPEN] Spatial domain coordination method for UE-to-UE co-channel CLI handling

Summary

From RAN1#110 to RAN1#112, there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied and followings were agreed.

3 Spatial domain coordination method for UE-to-UE co-channel CLI handling RAN1#110 Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes: - Details for spatial domain coordination by gNB - Relevant information exchange (if needed) Note1: Study can include method for FR1 and FR2.

In RAN1#110bis-e, the final moderator proposal for spatial domain enhancement was provided.

#### Moderator Proposal #2-7 For details of spatial domain coordination method for UE-to-UE co-channel CLI, followings can be studied. - Tx-Rx beam coordination/avoidance between UEs

In RAN1#112, the moderator proposal was discussed in 3rd round. But, there was no conclusion due to lack of discussion time.

Moderator Proposal #21-4 (1) For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the benefit of enhancement of UE assumption for CLI measurement (e.g., QCL-D)

In RAN1#112-bis-e meeting, followings are proposed.

Huawei proposes to study solutions for beam pairing in FR2 considering CLI strength of beam pair (RSRP, RSSI) that is over threshold A/below threshold B, Preferred/restricted beams between UEs, Preferred Tx beams of candidate scheduled aggressive UE and preferred Rx beams of the candidate scheduled victim UE, and Necessity of information exchange considering signaling overhead, latency and implementation flexibility.

InterDigital thinks two aspects: (1) Consider preventive aspects in spatial domain coordination by determining the most and least favourable beam pairings between the victim and aggressor UEs. (2) Consider CLI mitigation aspects in spatial domain coordination by determining beam pairing between victim UE and gNB based on directional CLI.

CMCC proposes that victim UE can report the recommended beams along with the CLI measurement results.

Spreadtrum proposes to study considering: Use results of beam management of gNB and UE as the baseline, and Exchange information of best UE pairs.

Intel proposes to study schemes focusing on: Inter-gNB information exchange on use of or intended Tx beams, Inter-gNB information exchange on preferred/not-preferred Tx beams, and Methods for identification of Tx beams (e.g., via mapping to SRS resource indices.)

Xiaomi proposes beam based CLI measurement for UE-to-UE CLI mitigation.

Panasonic proposes to study how to include spatial domain information to facilitate efficient UE pairing to avoid UE-to-UE CLI, and to study UE-to-UE reporting for spatial domain coordination using L1 or L2 reporting.

CEWIT proposes gNB configuring different Rx beams for UE-to-UE CLI measurement, and separate UE-to-UE CLI measurement report corresponding to different receive beam configurations.

Qualcomm proposes UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware gNB beam management for CLI mitigation, which can apply to L1/L2/L3 CLI measurement and reporting including P/SP/AP resource and report (corresponding TCI state/QCL-D can be configured, updated.)

Qualcomm thinks that UE can dynamically report to the gNB a set of recommended beams, not preferred beams, or both. gNB configures multiple Rx (QCL-D) beams for UE to measure, and UE determines the recommended and/or not preferred beams based on measurement of inter-UE CLI using different RX beams (QCL-D)

Qualcomm thinks that Inter-UE CLI can be mitigated by configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding codebook. That is, for SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots, and for dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Qualcomm thinks inter-CU/vendor coordination on exchanging scheduled data/control UE beams to mitigate inter-UE CLI, and inter-CU/vendor coordination exchange information on number of required CLI resources, e.g. the total number could be # of measured Tx beams of UL UE multiply # of Rx beams of DL UE.

InterDigital proposes a beam failure instance due to CLI may occur even when the signal received from gNB is not physically blocked, where the degradation in the DL radio link is mainly due to the interference from an aggressor UE, and enhancements in beam failure detection and recovery, in case the beam failure is caused by UE-to-UE CLI.

NEC thinks differentiation the BFR caused by CLI with the beam blockage is needed, and eliminate the effect of the CLI to BFR for BFD and NBI should be considered.

From the proposals in documentation, followings can be summarized.

For mitigating co-channel UE-to-UE CLI, spatial domain coordination method for UE-to-UE co-channel CLI can be enabled. Followings can be studied.

- Option 1: Victim UE can measure co-channel UE-to-UE CLI of multiple SRS resource(s) using different Rx beams

- Option 2: Victim UE can report preferred/non-preferred beams to the serving gNB

- Option 3: Victim UE can measure and report co-channel UE-to-UE CLI depending on the slot type (i.e., SBFD slot, non-SBFD slot)

- Option 4: The serving gNB can configure UE Rx beam (QCL-D) to the victim UE

Proposal

Moderator Proposal #23-1

For mitigating co-channel UE-to-UE CLI, spatial domain coordination method for UE-to-UE co-channel CLI can be enabled. Followings can be studied.

- Option 1: Victim UE can measure co-channel UE-to-UE CLI of multiple SRS resource(s) using different Rx beams

- Option 2: Victim UE can report preferred/non-preferred beams to the serving gNB

- Option 3: Victim UE can measure and report co-channel UE-to-UE CLI depending on the slot type (i.e., SBFD slot, non-SBFD slot)

- Option 4: The serving gNB can configure UE Rx beam (QCL-D) to the victim UE

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C, Panasonic, CMCC, Sony, IDC, QC |
| Not support | Samsung, Ericsson |

| Companies | Views |
| --- | --- |
| Spreadtrum | We agree with this proposal in principle. But we think the preferred/non-preferred beams selection can be done by UE implementation. |
| vivo | We think more clarifications are needed for the listed options. For example, for option 1, how does UE measure CLI using different Rx beams, what’s the different between option 1 and option 4. For option 3, it is not clear to us. |
| ZTE | Option 1 seems to be an implementation issue since in general which Rx beam is used for measurement is up to UE implementation. Is there any spec impact? For option 3, is one periodic resource configured in both SBFD slot and non-SBFD slot? For option 4, for what purpose does the gNB configure Rx beam to the victim UE. Is it used for CLI measurement or downlink reception, or both? We think Option 1 and Option 4 can be merged or just leave to UE implementation. |
| Huawei, HiSilicon | Option 1 to Option 3 are related to measurement and reporting while Option 4 seems to target Rx beam configuration which is already possible in current specification. In general, Option 1/2/4 seem to propose beam-pair based UE-UE CLI measurement and reporting. The overall measurement complexity as well as the overhead increase compared to the Rel-16 UE-UE CLI measurement is quite significat considering the multiple UEs in the network. In addition, it is still not clear how the gNB can acutally ultilize such sophisticated measurement and whether there is any additional benefit compare to Rel-16 UE-UE CLI measurement. In addition, for Option 3, the victim UE can only perform CLI measurement on SBFD slots, so the victim UE measure and report co-channel UE-to-UE CLI depending on the slot type seems not needed. |
| Xiaomi | Slightly prefer Option 4. Configuring the Rx beam by gNB is beneficial for the further beam enhancement. To be specific, once the UE reports the CLI results via different Rx beams, the gNB is able to know which Rx beam corresponds to lower CLI level. In this case, the Rx beam with lower CLI level can be scheduled for UE’s PDSCH/PDCCH reception. |
| Sony | Some of these options can work together for example, Option 1, UE measures aggressor beams and then use Option 2 to tell gNB which beams it preferred/not preferred. |
| CATT | At this stage we can only say: For mitigating co-channel UE-to-UE CLI, spatial domain coordination method for UE-to-UE co-channel CLI can be enabled further studied including the following:. Followings can be studied. |
| Nokia/NSB | Does it mean that we can select more than one option? It seems they are directions for further study so keeping bullet points may be sufficient rather than calling them options. In general, we think that Option 2 is the most straightforward to enable coordination among gNBs. |
| Samsung | We think that only Option 4 is needed. |
| Ericsson | Suggest to remove “can be enabled” and just keep “can be studied” Option 1 seems like it would require quite high overhead considering the number of UEs – multiple SRS resources would needed in multiple OFDM symbols to enable the victim UE to test different Rx beams. Option 2 is not clear. How does a UE report a preferred Rx beam? By the time it measures and resports, the UE has most likely moved or rotated, and the information is not valid anymore. Agree with Huawei that Option 3 seems not needed. Assuming gNBs co-ordinate their SBFD patterns (similar to co-ordinated TDD patterns in today’s deployments), then UE-UE CLI does not occur on non-SBFD slots. Option 4 is unclear – current speca already configures QCL-D to a UE for reception of various DL signals/channels. |
| IDC | We understand the intention of this proposal, and share similar views as Xiaomi and Sony. Our understanding on Option 1 is that the victim UEs measure SRS-RSRP of different SRS resources (beam swept). Option 2 implies the victim UE reports SRIs with the least and most CLI (e.g., based on measured SRS_RSRP) to the gNB. Regarding Option 4, the aggressor gNB can indicate its associated UE to avoid using the UL beam associated with the SRI with the high CLI, while the victim gNB can indicate a desired UE Rx beam (e.g., QCL-D) to the victim UE. |
| Intel | For Option 2, we do not think a victim UE needs to explicitly report preferred/non-preferred Tx UE beams to a serving gNB – this can be realized based on reported SRS-RSRP and serving gNB can identify the corresponding Tx beams via mapping from the SRS resources. Instead, what is needed is exchange of information between gNBs on preferred/non-preferred UE Tx beams based on, e.g., identification of SRS resources. |
| Lenovo, Motorola Mobility | We are generally fine with this proposal, but it seems the options are not necessarily mutual exclusive. |
| QC | We are ok with the proposal. We are okay to study Option 3 but it seems not directly related to spatial domain method. Or does this mean UE uses different beams on different slot type? Compared option 1 vs option 4, we support option 4, but we are open to study both, and reasons for option 4 are: - Rx beam for CLI measurement is up to UE implementation as QCL-D follows one of the latest received PDSCH and the latest monitored CORESET. However, there could be multiple active DL beams for the UE, and the latest used beam for CLI measurement may not be the beam used for future scheduling, and the CLI level per beam could be different especially for FR2. In turn, gNB cannot characterize CLI for different Rx beams in order to enable CLI-aware beam management. - For UE with two UE panels for FR2, there could be multiple active DL beams for the UE, and the latest used beam for CLI measurement may not be the beam used for future scheduling on a different UE panel, and the CLI level per beam per UE panel could be even largely different. Therefore, it is necessary to measure different Rx beams for different CLI levels on different UE beams and panels by gNB configuration. - Without configured Rx beam information per CLI measurement resource and reporting, there could be ambiguity for gNB to understand and make use of the CLI reporting for gNB scheduling, e.g. the CLI measurement on latest used PDSCH beam can be changed to other active DL beam in future transmission and reception, with using the current e.g. low CLI measurement result as assumption, the new active DL beam can be high CLI for future transmission and reception. There is no guarantee that UE will use same beam for Rx, it is important to have explicit RRC-configured QCL-D per each CLI resource to avoid ambiguity, and in this case, gNB has its control for CLI mitigation. |

    - [OPEN] UE and gNB transmission and reception timing

Summary

From RAN1#110 to RAN1#112, there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied. But there was no consensus about UE and gNB transmission and reception timing.

Moderator Suggestion #2-2 - Study the feasibility and potential benefits of ‘UE and gNB transmission and reception timing’ for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of timing alignment - Relevant information exchange Moderator Proposal #2-9-1 Study whether/how to enhance SRS-RSRP measurement accuracy considering on reception timing of SRS from aggressor UE for SRS-RSRP measurement - Note: if study of L1/L2 based CLI measurement is agreed, above is modified as following. For UE-to-UE co-channel CLI handling, study whether/how to enhance CLI measurement accuracy considering reception timing at victim UE for CLI measurements

Support

- Intel proposes to study

- assistance information from a serving gNB to a UE for adjustment of reception time window for CLI measurements, and

- timing synchronization assistance information exchange between gNBs to enable improved estimation of timing offsets between neighboring gNBs.

- the reception window for multiple aggressors

- InterDigital think that UE and gNB timing alignment could be effective in performance enhancement for UE-to-UE CLI measurement and accuracy. InterDigital proposes to study timing alignment issues including subband non-overlapping full duplex scenarios.

- Qualcomm think that the CLI measurement UE can recommend TA adjustment for aggressor UE corresponding to a particular CLI resource transmission. Also, Qualcomm thinks that the CLI measurement UE can report Rx timing difference between UE DL arrival timing and CLI RS arrival timing to help align the timing at the DL UE for inter-UE CLI reduction. Qualcomm thinks that inter-UE CLI can be mitigated by configuring slot-specific TA.

- For SBFD, TA configured for SBFD slots can be different from those configured for HD slots.

- For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

- vivo think that when the victim UE suffers CLI from multiple aggressor UEs with different TAs, CLI may exceed the CP duration of the victim UE which would bring severe interference on DL reception.

- Apple thinks that UEA is indicated to hold two different TAs to assure symbol level alignment at UEV.

  - one TA for symbols on which TRP is doing legacy TDD, another TA for symbols on which TRP is doing SBFD or dynamic TDD

Deprioritize

- Huawei think that the current timing scheme for UE-to-UE CLI measurement may be sufficient, and the benefits of enhancement on reception timing of SRS from one aggressor UE for SRS-RSRP measurement are not clear.

- Spreadtrum think that TA adjustment of UEs is deprioritized for transmission and reception timing of UE-to-UE CLI measurement in Rel-18 dynamic/flexible TDD.

- xiaomi think that the misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation.

From the inputs from the documentations, it was observed that

Reception timing misalignment is possible due to

- if TA in SBFD slot is different from those in half duplex slot.

Receiving timing alignment within CP duration between reference signal for CLI measurement and DL channel/signal at victim UE side can prevent the degradation of CLI measurement accuracy. In order for alignment of receiving timing within CP duration at victim UE side, following options can be studied.

- Option 1: UE implementation

- Option 2: Small cell with short propagation delay

- Option 3: a serving gNB provides assistance information to a UE for adjustment of reception time window for CLI measurements

- Option 4: A measurement UE can report Rx timing difference between UE DL arrival timing and CLI-RS arrival timing.

Proposal

Moderator Proposal #24-1

Receiving timing alignment within CP duration between reference signal for CLI measurement and DL channel/signal at victim UE side can prevent the degradation of CLI measurement accuracy. In order for alignment of receiving timing within CP duration at victim UE side, following options can be studied.

- Option 1: UE implementation

- Option 2: Small cell with short propagation delay

- Option 3: a serving gNB provides assistance information to a UE for adjustment of reception time window for CLI measurements

- Option 4: A measurement UE can report Rx timing difference between UE DL arrival timing and CLI-RS arrival timing.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | New H3C,vivo, Intel (not sure if Option 1 needs to be listed), Lenovo/MM (options to be discussed/refined), QC |
| Not support | Huawei, HiSilicon, CATT, Samsung, Ericsson |

| Companies | Views |
| --- | --- |
| Spreadtrum | Evaluation on the degradation introduced by receving timing misalignment should be provided before study on it. |
| vivo | For option 3 and 4. The propagation delay from a victim UE to its serving cell may be very different from that of an aggressor UE in a neighbouring cell. In some case, the CLI RS arriving may exceed CP of the victim UE which leads to stronger inter-symbol interference. One or more than one symbols should be reserved. The benefit and overhead generated from receiving timing alignment should be carefully evaluated. It is also unclear when a victim UE suffers CLI from multiple aggressor UEs with different TAs, how to adjust the receiving timing. |
| ZTE | Option 2 is gNB implementation and cannot applied to all the scenario. We can merge Option 1 and Option 2. For option 4, more clarification is needed. What is the purpose to report Rx timing difference between UE DL arrival timing and CLI-RS arrival timing. How does it resolve the issue of receiving timing misalignment? |
| CMCC | We are fine to further study, but one thing should be taken into account is the overhead and feasibility of handling different CLI-RS timing from multiple aggressor UEs |
| Huawei, HiSilicon | Option 1 and Option 2 can be the baseline. Option 3 and Option 4 are not needed. In the indoor deployment scenario, the current timing may not have problems due to the cell radius is small, most of the delay or delay difference are within the cyclic prefix. For a larger cell deployment, the problem caused by some misalignment between the measurement window and the received interference signal may not be critical for CLI measurement (e.g., RSRP, RSSI). It can be up to UE implementation to dertermine the time offset just as what has been specified in R16 CLI measurements. Besides, considering a victim UE may have lots of aggressor UEs, it is quite difficult for the victim UE to adjusting the Rx timing such that the DL signal is align with multiple aggressor UE. The benefits of enhancement on reception timing from one aggressor UE are not clear. Thus, in our view, Option 1 and Option 2 are enough, Option 3 and Option 4 are not needed. |
| Xiaomi | Prefer option 1. Currently, the CLI RS reception for L3 based UE-to-UE CLI measurement is by UE implementation. When it comes to L1/L2 UE-to-UE CLI measurement, the motivation of further enhancement is not observed. In addition, TA acquisition of candidate cells for L1/L2 cell switch has been supported in Rel-18 NR mobility enhancement topic. UEs with that capability can derive the receiving timing of CLI RS from different cells wherein the aggressor UEs locate. |
| Sony | Option 1 and Option 2 are both implementation, which suggest we don’t need to look into this issue further. Option 3 and Option 4 seem to assume there are only 1 victim UE and only 1 aggressor UE. It is unclear how this methods operate or how practical they are in real life where we have 100s of victim UE and 100s of aggressors. Which aggressor’s timing should the victim UE align with? |
| CATT | Need to justify the feasibility of the enhancement |
| Nokia/NSB | We are generally fine to list all options for further study. Some further clarifications/justifications on the options may be needed, as pointed out by other companies. |
| Samsung | We share Huawei’s view (and recommendations). |
| Ericsson | Share similar view with Sony. Options 3 and 4 seem impractical due to CLI from multiple aggressor UEs. It seems there is no timing issue with Options 1 and 2, so why do they need to be studied? |
| IDC | Support the proposal in general to further study the options to enhance timing alignment issues. However, the provided options need more clarifications. |
| Intel | It seems the logic from some companies is just that we continue with whatever we did (or actually didn’t do) in Rel-16. Regarding the question raised about multiple aggressor UEs, for SRS-RSRP measurements, a victim UE would anyway be measuring the cross link channel on different SRS resources; and in this case, multiple timings could be feasible. More importantly, not all aggressor UEs need to be received using different timings and it could still be up to UE implementation on where to position its Rx window. The key point is to provide assistance information from the serving cell to enable a UE decide whether and by how much to adjust its Rx timing window relative to the DL timing. |

    - [CLOSE] Power control based solution

Summary

From RAN1#110 to RAN1#112, there were discussions to determine which method(s) of UE-to-UE co-channel CLI handling is/are studied and followings were agreed.

5 Power control based solution Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline

  - Not support: [OPPO], [CATT], [App], [WILUS]

  - UL power reduction [QC], [intel], [zte], [vivo], [IDC], [SS], [CMCC], [Nokia]

  - DL power boost [QC], [IDC]

  - UL power limitation [QC], [SS], [intel]

Aggressor UE’s UL power control based CLI mitigation

CATT thinks that the benefit and loss generated from UL power control based solution should be carefully evaluated.

OPPO thinks existing power control mechanism with separate open loop power control parameters can be reused for UL transmissions with CLI and without CLI.

Apple thinks that reuse existing signaling and procedure to manage for UE-to-UE CLI by UL power control mechanism.

WILUS thinks that RAN 1 to study UL power control-based solution for UE-to-UE CLI handling based on L1/L2 UE-to-UE CLI measurement and reporting at aggressor UE side. And existing UL power control parameter set can be reused.

CMCC thinks that two power control loops can be used for different slot types, e.g., SBFD slot and normal UL slot in SBFD system or aligned and unaligned slot in flexible/dynamic TDD system.

ZTE thinks that regarding UE-to-UE CLI handling in power domain, it should be supported to configure separate sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power( ), etc, for UL transmission in different resources with/without UE-to-UE CLI.

InterDigital thinks Dynamic UL power control mechanisms based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL should be considered in performance enhancement for UE-to-UE CLI mitigation.

Intel proposes to study configuration of separate open-loop power control parameters in different slot or symbols depending on the fixed or dynamic/flexible UL resource allocation.

Nokia proposes to study autonomous adjustments of the aggressor UE transmit power to reduce the UE-to-UE CLI

Qualcomm proposes that CLI measurement UE can recommend UL power backoff for neighbor UL UE corresponding to a particular CLI resource. Qualcomm thinks that gNB may indicate UL power limit for certain interfering UE to ensure caused CLI is always under limit. Also, inter-UE CLI can be mitigated by configuring slot-specific power control parameters

- For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

DL power adjustment

Apple thinks that further study the feasibility, and impacts to legacy UE, for DL power adjustment

InterDigital thinks that Dynamic DL power backoff/control mechanisms at gNB could be used to deal with self-interference caused by the FD operation at the gNB, where such mechanism could impact UE behaviours including CSI-RS measurements depending on the amount of the power backoff.

InterDigital proposes to study power-control based mechanisms for UE-to-UE CLI mitigation and issues related to gNB’s transmission power backoff/adjustment.

Qualcomm thinks that CLI measurement UE can recommend DL power boost to cope with the CLI from neighbor UL UE corresponding to a particular CLI resource.

Summary

From the inputs of the companies, it was observed that

The UL tx power reduction of aggressor UE can be advantage of UE-to-UE CLI reduction. But, the UL coverage of the aggressor UE may be reduced.

The DL Tx power adjustment from the gNB may be helpful to overcome the UE-to-UE CLI from the aggresso UE. But, the impacts to legacy UE needs to be studied.

  - 2nd Round Discussion

    - UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

    - Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling

    - Spatial domain coordination method for UE-to-UE co-channel CLI handling

    - UE and gNB transmission and reception timing

    - Power control based solution

  - 3rd Round Discussion

    - UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

    - Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling

    - Spatial domain coordination method for UE-to-UE co-channel CLI handling

    - UE and gNB transmission and reception timing

    - Power control based solution

- Summary of discussion in RAN1#112-bis-e

- 1 gNB-to-gNB inter-cell co-channel interference

  - gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling

  - Coordinated scheduling for time/frequency resources between gNBs

  - Spatial domain coordination method gNB-to-gNB CLI handling

  - UE and gNB transmission and reception timing

  - Power control based solution

  - Advanced receiver

  - Others

- 2.  UE-to-UE inter-cell co-channel interference

- 2.1 UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

- 2.2 Coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling

- 2.3 Spatial domain coordination method for UE-to-UE co-channel CLI handling

- 2.4 UE and gNB transmission and reception timing

- 2.5 Power control based solution

- 2.6 Advanced Receiver

- 2.7 Sensing based mechanism

- Contact Person

Please provide the information of the contact person for the purpose of discussion facilitation

| Company | Name | Email address |
| --- | --- | --- |
| TCL Communication Ltd. | Shahid Jan | shahid.jan@tcl.com |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| Lenovo | Yuantao Zhang | zhangyt18@lenovo.com |
| New H3C | Lei Kong | kong.lei@h3c.com |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| Vivo | Na Li | Lina5g@vivo.com |
| NTT DOCOMO | Daisuke Kurita | kuritad@nttdocomo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| QC | Emily Zhang | qiaz@qti.qualcomm.com |
| Spreadtrum | Shuai Zhang | Shuai.Zhang6@unisoc.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| LG | Jaenam Shim | jaenam.shim@lge.com |
| Nokia/NSB | Quang Nhan | nhat-quang.nhan@nokia.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Panasonic | Tomohiro Inoue | inoue.tomohiro004@jp.panasonic.com |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |

Summary

[1]

Channel measurement is necessary for obtaining gNB-to-gNB instant channel. The measured gNB-to-gNB instant channel is used for beam nulling to suppress the gNB-to-gNB co-channel cross link interference.

For channel measurement, CSI-RS from aggressor gNB can be used.

Also, the enhancement of CSI-RS for co-channel gNB-to-gNB channel measurement provide a benefit of reducing the overhead.

Channel measurement

gNB-to-gNB instant channel is needed

Spatial domain enhancement

- beam nulling

- beam pairing

CLI strength of beam pair

Preferred Tx beam for each receiver beam

Necessity of information exchange. (considering signaling overhead, latency and implementation flexibility)

Timing

the UL signal and downlink interference can be aligned (within CP) when proper TAoffset is configured

- Submitted proposal

- Inter-cell gNB-to-gNB CLI

  - CLI measurement and reporting

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | gNB-to-gNB CLI CLI measurement and reporting Figure 1. The general framework of gNB-to-gNB CLI measurement Proposal 4: Consider the potential enhancement to NZP CSI-RS for gNB-to-gNB channel measurement in FR1 for coordinated beamforming. Proposal 5: Study CSI-RS port expansion to support gNB-to-gNB channel measurement for SBFD and DTDD with considering following gNB-to-gNB channel characteristics to reduce the high overhead of CSI-RS caused by CSI-RS port expansion: - gNB-to-gNB channel has a larger coherent time than gNB-UE channel. - gNB-to-gNB channel has a larger coherent bandwidth than gNB-UE channel. Proposal 6: Study gNB-to-gNB channel measurement resource management, coordination, and configuration by OAM. Proposal 7: Beam sweeping among multiple gNBs can be studied for beam pairing. Figure 6. Illustration of the uplink blank/muting resource Observation 4: Different uplink blank/muting resources can be used to measure spatial characteristics of gNB-to-gNB CLI caused by various DL signals and to avoid cross link interference. Observation 5: Uplink resources muting pattern can be different for various DL channel(s)/signal(s). Proposal 8: For gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling support the following - Muting REs in UL slot at the position of part of REs of the SSB, SIB1, and broadcast PDCCH from aggressive cell are supported to measure the spatial characteristics of downlink broadcast interference. - Muting REs in UL slot at the position of part of REs of unicast PDSCH and PDCCH from aggressive cell are supported to obtain the spatial characteristics of unicast PDSCH and PDCCH CLI. - Muting REs in UL slot at the position of REs of NZP CSI-RS from aggressive cell are supported to avoid strong CLI. Proposal 9: UE non-transparent uplink muting resources is supported for cross link interference measurement and avoidance. |
| TCL Communication Ltd. [R1-2302408] | gNB-to-gNB CLI CLI measurement and reporting Observation 1: CD-SBB or NCD-SSB are UE specific and may not be directly applicable to measure the gNB-to-gNB co-channel CLI. Observation 2: To measure gNB-to-gNB CLI, it may be necessary for the victim gNB to identify the aggressor gNB. Proposal 1: The procedure of gNB-to-gNB co-channel CLI measurement based on the CD-SSB and NCD-SSB can be further studied. |
| New H3C Technologies Co., Ltd. [R1-2302430] | gNB-to-gNB CLI CLI measurement and reporting Figure 2. The deployment scenario of the flexible TDD Observation 1: Using SSB for CLI measurement is not enough for the case that the bandwidth is larger than 20MHz. Proposal 1: The configuration of both CD-SSB and NCD-SSB, such as the number of SSB, time-frequency domain position of the SSB, the periodicity and the time offset of the first SSB, and so on, should be exchanged between gNBs for CLI measurement. The same slot of the victim gNB as the SSB slot in aggressor gNB should be configured to the UL slot Proposal 2: The existing CSI-RS for interference measurement (CSI-IM) /ZP-CSI-RS can be reused for the CLI measurement and report in the D/F-TDD, the measurement resource can be periodic, aperiodic or semi-persistent. Proposal 3: The mechanism of the CSI-RS for interference measurement (CSI-IM) can be extended to the multiple aggressor gNBs case. A central controller can be used to handle the information exchange between gNBs, it can be a CU, a master gNB, or OAM. Proposal 4: The NZP-CSI-RS can also be used for CLI measurement in order to get more precise measurement results. The measurement resource can be periodic, aperiodic or semi-persistent. Proposal 5: The NZP-CSI-RS for different aggressor gNBs should be different, and the configuration of the NZP-CSI-RS should be exchanged between gNBs by Xn interface, or handled by a central controller. Proposal 6: In victim gNB, the PRACH/PUCCH/SRS should not use any resource that are overlapping with CSI-RS for CLI measurement, the PUSCH can perform rate matching around CSI-RS for CLI measurement. Figure 3. CLI measurement window Proposal 7: A measurement window can be introduced for improving the energy efficiency of the victim gNB. For the victim gNB, it can only measure the CLI measurement signals in the measurement windows, and ignore all the CLI measurement signals out the range of the measurement windows. Several measurement window can be configured, but only one is active. The measurement window is periodic, and its position is determined by the length, periodicity and offset. Proposal 8: The CLI report including CLI-RSSI or RSRP (if any) can be exchanged between the aggressor gNB and victim gNB. The reported CLI results can be short term or long term. The report can be full report or partial report, and can be event-triggered or periodic. Proposal 14: The new RAN measurement abilities should be introduced for supporting the CLI measurement and reporting: CLI-RSSI and/or CLI RSRP |
| vivo [R1-2302485] | gNB-to-gNB CLI CLI measurement and reporting Proposal 1: Assistance information exchange among gNBs can be considered for gNB-to-gNB CLI handling, including measurement reports of RSRP/RSSI, scheduling information. |
| InterDigital, Inc. [R1-2302523] | gNB-to-gNB CLI CLI measurement and reporting Observation 1. The gNB-to-gNB co-channel CLI mitigation can be based on spatial domain coordination, where the CLI measurement can be based on beam sweeping at both victim and aggressor gNBs. Proposal 1. Consider using spatial domain coordination for gNB-to-gNB co-channel CLI measurement and mitigation, where the victim gNB measures beam-swept CLI and sends, to the aggressor gNB, information on the SSB index or the CRI of the aggressor beams with the highest and/or lowest CLI in addition to the measured CLI. Observation 2. The victim gNB could monitor to detect one or more events to trigger gNB-to-gNB co-channel CLI measurement. Proposal 2. Consider defining the events that may trigger the gNB-to-gNB CLI measurement. |
| OPPO [R1-2302548] | gNB-to-gNB CLI CLI measurement and reporting Proposal 1: A CLI-RSSI-alike resource, i.e. defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset, can be used for gNB-to-gNB CLI measurement. Proposal 2: RAN1 targets to support L1 gNB-to-gNB CLI measurement. - L1-based RSRP/RSSI can be considered; - L1 gNB-to-gNB CLI measurement reporting with timestamp is exchanged over Xn interface. |
| Spreadtrum Communications [R1-2302600] | gNB-to-gNB CLI Coordinated scheduling Proposal 2: Support to use pseudo-sequence based muting scheme for inter-gNB CLI handling |
| CATT [R1-2302703] | gNB-to-gNB CLI CLI measurement and reporting Proposal 1: Neighboring gNBs should exchange measurement configuration information of SSB set and/or CSI-RS set (each SSB or CSI-RS in the set is associated with a specific beam) to enable beam level CLI measurement. Proposal 2: Beam level measurement results and corresponding measurement resources should be exchanged among gNBs to achieve beam/spatial based CLI management. Proposal 3: For UL transmission muting, both gNB scheduling based solution and rate matching based solution can be used. Proposal 4: RSSI and RSRP can be considered as the gNB-gNB CLI measurement metric. Figure 4 Inter-subband CLI measurement |
| NEC [R1-2302745] | gNB-to-gNB CLI CLI measurement and reporting Proposal 1: Study aperiodic or semi-persistent CSI-RS along with periodic CSI-RS for gNB-gNB CLI measurements Proposal 2: Define CLI sensitivity level as a measurement metric for gNB-gNB CLI measurements Proposal 3: Support UL rate matching/puncturing procedures at least for CLI measurement based on CSI-RS - FFS for SSB |
| ZTE, China Telecom [R1-2302758] | gNB-to-gNB CLI CLI measurement and reporting Proposal 2: For gNB-to-gNB co-channel CLI measurement, both RSRP measurement and RSSI measurement can be considered. - The existing measurement resource configuration for SSB/CSI-RS based RRM can be applied as baseline for gNB-to-gNB co-channel RSRP measurement. - The existing configuration of RSSI measurement resource can be applied as baseline for gNB-to-gNB co-channel RSSI measurement. Observation 1: The existing CSI-RS can be configured with up to 32 ports, which is not sufficient for the gNB-to-gNB co-channel channel measurement for gNBs equipped with 64 antenna ports in the practice. Proposal 3: In order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives: - Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim. - Alt.2: Define NZP CSI-RS with up to 64 ports. - Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI. Proposal 5: Regarding UL resource muting, UL rate matching/cancellation mechanism can be defined for more accurate gNB-to-gNB co-channel CLI and/or channel measurement. Proposal 6: Regarding UL resource muting, the rate matching resource for uplink transmission can be determined according to the measurement resources. - FFS whether a certain guard bands need to be reserved around the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB). |
| Ericsson [R1-2302771] | gNB-to-gNB CLI CLI measurement and reporting (a) (b) Figure 5: Performance of dTDD and protected dTDD compared to sTDD in terms of mean user throughput for (a) UL, and (b) DL, at low, medium, and high loads. (a) (b) Figure 6: Performance of dTDD and protected-dTDD compared to sTDD in terms of 5th percentile user throughput for (a) UL, and (b) DL at low, medium, and high loads. Observation 1. Protected dTDD is a simple and robust scheme for mitigating the performance impact of CLI without requiring fast exchange of information between gNBs. The scheme is feasible for operation both within and between operators. Proposal 1. Capture the performance of protected dTDD in the TR as a beneficial CLI handling scheme under the umbrella of "co-ordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling." |
| Intel Corporation [R1-2302796] | gNB-to-gNB CLI CLI measurement and reporting Observation 1 - For gNB-to-gNB CLI mitigation, the necessity of configuration of ZP CSI-RS or CSI-IM resources measurements to improve accuracy of RSSI or RSRP type of measurements remains to be established. - However, use of ZP CSI-RS or CSI-IM resources may be considerable in estimating SINR under different interference hypotheses when considering coordination across more than two gNBs/TRPs. Proposal 1 - For gNB-to-gNB CLI mitigation, - The configuration on the time/frequency/sequence/spatial information on the CLI-RS (NZP CSI-RS, both CD-SSB and NCD-SSB) needs to be exchanged between gNBs. - Measurement and reporting periodicity: at least periodic measurement resources and reporting are considered. - CLI measurements may be categorized as short-term and long-term interference measurements: - Short-term CLI metrics may be defined based on CSI/CQI- or L1-RSRP/RSSI/SINR-like measurements. - NZP-CSI-RS may be suitable candidates for CLI-RS for short-term CLI metrics. - Use of ZP CSI-RS or CSI-IM resources can be studied further for estimating L1-SINR under different interference hypotheses when considering coordination across more than two gNBs/TRPs. - Long-term CLI metrics may be defined based on CLI-RSRP- or CLI-RSSI-like measurements. - In addition to NZP-CSI-RS, CD-SSB and NCD-SSB may be CLI-RS candidates at least for long-term CLI measurements. - UL resource blanking to enable measurements at a victim gNB can be explicitly realized via existing mechanisms in NR, e.g., UL CI, SFI, etc., or implicitly based on gNB scheduling. |
| Sony [R1-2302846] | gNB-to-gNB CLI CLI measurement and reporting Observation 1: For UL transmissions, the UE needs to know which REs to mute. Observation 2: RE muting may not be enabled for every UL/DL transmission. Proposal 1: For UL and DL transmissions, the gNB semi-statically configures one or more RE muting patterns for the UE, i.e. the UE is aware of which REs is muted. Proposal 2: The gNB dynamically enables/disables RE muting for an UL/DL transmission and if multiple RE patterns are configured, the gNB indicates which RE muting pattern to apply in the dynamic grant. Observation 3: RE muting on REs containing RS from multiple gNBs may degrade the reliability of UL transmissions. Proposal 3: RE muting on REs containing gNB RS is conditional upon the transmission parameters, such as the L1 priority or MCS of the UL transmission. |
| xiaomi [R1-2302983] | gNB-to-gNB CLI CLI measurement and reporting Observation 4：The above procedure of the gNB-to-gNB measurement and reporting can be a starting point. Proposal 9: Support periodic reporting for gNB-to-gNB CLI mitigation. Proposal 10: RSRP and RSSI can be considered as baseline metrics for gNB-to-gNB CLI measurement. |
| Nokia, Nokia Shanghai Bell [R1-2303017] | gNB-to-gNB CLI CLI measurement and reporting Observation 9: The limitations of the Xn interface should be considered when drawing conclusions on the benefits of SBFD time/frequency configuration information exchange. Proposal 11: The exchange of SBFD time/frequency configuration between gNBs is seen as beneficial and it should be supported. Observation 1: Using CD-SSBs for measuring CLI at the gNBs might require muting/skipping some of the CD-SSBs transmissions which ultimately impacts the UE information acquisition and/or UE measurements. Proposal 1: The measuring gNB should be informed about the CLI-RS configuration over the Xn interface. This applies to both CLI-RS candidates, the SSB-based and CSI-RS-based measurements. Proposal 2: gNB-to-gNB CLI measurements to follow a 2-step procedure. In the first step, gNBs use SSBs to obtain a course per-SSB CLI estimation. On a second step, CSI-RS are used to fine-tune the initially measured CLI levels. Proposal 3: For gNB-to-gNB co-channel CLI measurement, the RSRP should be used as baseline measurement metric. Proposal 4: Event-triggered reporting should be supported for gNB-to-gNB CLI measurements. Proposal 5: Consider allowing CSI-RS transmission during the guard period symbols for conducting CLI measurements while not impacting the downlink spectral efficiency on the aggressor gNB |
| Lenovo [R1-2303088] | gNB-to-gNB CLI CLI measurement and reporting Observation 1: Periodic RS (such as NZP CSI-RS and SSB) are not optimal for gNB-to-gNB CLI measurements. Using periodic RS without enhancements is wasteful and not easily scalable, especially for beam-based CLI measurement at FR2. Proposal 1: Study enhancements to periodic RS for resource efficiency, scalability, and flexibility of gNB-to-gNB CLI measurement. Consider gNB-specific patterns of RS transmission and CLI measurement. Proposal 2: In order for each gNB to have a chance to measure CLI from any other gNB in its vicinity, support gNB-specific patterns for transmitting SSBs dedicated to CLI measurements. The SSBs can be configured as NCD-SSB. Proposal 3: Support exchange of reference signal configuration information among gNBs for the purpose of inter-gNB CLI measurement. Proposal 8: Study unified inter-cell CLI handling through transmitting SRS by aggressor gNB/UE and measuring interference by victim gNB/UE. Proposal 9: Support assigning priorities to victim gNBs so that the aggressor gNB will be able to limit or avoid the CLI towards at least high-priority victim gNBs. Proposal 10: The impact on the PUSCH reception when receiving CLI measurement RS can be solved by gNB implementation. |
| Samsung [R1-2303128] | gNB-to-gNB CLI CLI measurement and reporting Proposal 4: For gNB-to-gNB co-channel CLI and/or measurement enhancements, Xn/F1AP signaling can indicate NZP CSI-RS resource, SSB resources, DL muting patterns of the aggressor gNB to the victim gNB. Proposal 5: UL muting patterns based on RB-level and symbol level muting, if needed, are left to gNB implementation. Proposal 6: For gNB-to-gNB CLI and/or measurement enhancements, no new gNB-side measurement capabilities are introduced in 38.215. Proposal 8: For other gNB-to-gNB co-channel CLI enhancements, Xn/F1AP signaling can be used to indicate the intended SBFD time-/frequency-domain configuration of the NR TDD cell to other gNBs. |
| Panasonic [R1-2303167] | gNB-to-gNB CLI CLI measurement and reporting Figure 7 Example of UL resources muting in UE transparent and non-UE transparent schemes Observation 1: When the number of RE in PUSCH allocation varies between symbols in non-UE transparent scheme, UE design becomes more complex since the total transit power or PSD among symbols within PUSCH varies with and without muting. Proposal 1: For UL muting resource for gNB-to-gNB CLI measurement, UE transparent scheme should be supported. |
| CMCC [R1-2303234] | gNB-to-gNB CLI CLI measurement and reporting Proposal 9: For inter-gNB intra-subband CLI measurement and reporting, RSSI-like measurement can also be supported. Proposal 10: For inter-gNB intra-subband CLI measurement and reporting, the transmission of different aggressor gNBs are coordinated on different RSSI resources/occasions in TDM/FDM manner. Proposal 11: For inter-gNB intra-subband CLI measurement and reporting, both transparent and non-transparent UL resource muting method should be considered, e.g., define UL rate-matching/muting pattern or avoid the scheduling on measurement resource. |
| Qualcomm Incorporated [R1-2303590] | gNB-to-gNB CLI CLI measurement and reporting Proposal 42: RAN1 shall study whether to reuse existing access link RS (e.g. at least periodic NZP CSI-RS/SSB) or introduce a dedicated RS (e.g. at least periodic NZP CSI-RS/SSB) for gNB-to-gNB co-channel CLI measurement and/or channel measurement. Proposal 43: For SSB serving as gNB-to-gNB co-channel CLI measurement and/or channel measurement RS, baseline proposal is to support the central NW can configure dedicated RS that is not used for access link for gNB-to-gNB CLI/channel measurement to guarantee TDMed CLI measurements across different gNBs to avoid Tx and Rx collisions. Proposal 44: For CSI-RS serving as gNB-to-gNB co-channel CLI measurement and/or channel measurement RS, baseline proposal is to support reusing access link CSI-RS for gNB-to-gNB CLI/channel measurement with the assumption that gNB can coordinate with neighbor gNBs and configure CSI-RS to guarantee the TDMed fashion measurement among different gNBs for gNB-to-gNB CLI/channel measurement. FFS: Study the receiving timing of CSI-RS for gNB-to-gNB CLI/channel measurement Figure 6-8 Inter-gNB CLI RS Tx/Rx Configuration Proposal 45: Investigate how resources are used/configured: e.g. how inter-gNB CLI measurement RS Tx and Rx time window configuration per cell. Proposal 46: Consider gNB HD/FD capability in the inter-gNB CLI RS Tx and Rx time window configuration. Proposal 47: RAN1 to study whether to perform simultaneous Tx and Rx of CLI measurement RS for gNB-to-gNB CLI/channel measurement for SBFD capable gNB. Figure 6-9 Simultaneous Tx/Rx of CLI measurement RS Figure 6-10 Inter-gNB/DU CLI Measurement Proposal 48: RAN1 to study inter-gNB/DU CLI measurement, where the gNBs/DUs may belong to same or different CUs and CU may provide DU the measurement configuration. Figure 6-11 Inter-gNB/DU CLI Report Proposal 49: RAN1 to study inter-gNB CLI report contents. Proposal 50: Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling. - Semi-static UL-muting patterns are configured to prevent UL transmissions from interfering with the inter-gNB CLI channel measurement. - Support non-UE transparent uplink resource muting scheme to configure RE-level UL muting patterns. Proposal 51: Support RAN1 study exchanging the UL muting pattern among the gNBs. Proposal 52: Support RAN1 study exchanging the DL muting pattern among the gNBs to ensure the accurate inter-gNB CLI/channel measurement. Proposal 53: RAN1 will study report based inter-gNB CLI measurement and report free inter-gNB CLI measurement. Proposal 54: Inter-gNB CLI measurement RS can be transmitted by aggressor gNB and measured by victim gNB, which will provide measurement results or DL Tx restriction info to aggressor gNB. Proposal 55: Inter-gNB CLI measurement RS can be transmitted by victim gNB and measured by aggressor gNB, which will derive caused CLI to victim gNB and corresponding DL Tx decision. Proposal 56: Inter-gNB CLI measurement RS can include transmitting cell ID and can be CDMed across multiple transmitting gNBs to save resource. Proposal 57: OAM or CU can configure the inter-gNB CLI transmission parameters, including time/frequency location, sequence ID, beam info, periodicity. Proposal 58: OAM or CU can configure the inter-gNB CLI monitoring parameters, including monitoring window location, beam info and periodicity. Proposal 59: Support to study OTA or backhaul information exchange for inter-gNB CLI reporting contents including inter-gNB CLI metric per Tx/Rx beam pair, allowed/disallowed beams, etc. Proposal 60: Support to study beam hierarchy information exchange for inter-gNB CLI measurement via SSB and CSI-RS. Observation 11: gNB-to-gNB CLI measurement and reporting can be used for gNB coordinated scheduling between gNBs and also can be used to facilitate inter-gNB Tx/Rx beamforming/nulling to reduce inter-gNB CLI. |
| NTT DOCOMO, INC. [R1-2303712] | gNB-to-gNB CLI CLI measurement and reporting Proposal 4: Measurement resource for gNB-to-gNB CLI measurement should be separated from the transmission resource of victim gNB for NZP CSI-RS and SSB. Proposal 5: Information for measurement window needs to be exchanged among gNBs via F1-AP. Proposal 7: Necessity of UL muting resource indication should be discussed based on typical scenarios for gNB-to-gNB measurement. And if we find the necessity of UE UL muting, UL muting resource indication with small granularity in time/ frequency domain can be considered. |
| WILUS Inc. [R1-2303831] | gNB-to-gNB CLI CLI measurement and reporting - Proposal 1: RAN1 to study UE non-transparent UL muting for gNB-to-gNB CLI handling in terms of efficient resource utilization. - FFS: UE behaviors on UL muting resource with respect to the UL signal/channel and PHY priority. |

  - Coordinated scheduling

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | gNB-to-gNB CLI Coordinated scheduling Figure 12. Downlink resource muting to avoid interference on uplink DMRS and uplink blank/muting resource to avoid strong downlink CSI-RS interference. Proposal 3: For details of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB CLI handling, at least followings can be studied - DL resource blanking including time/frequency resource at aggressor gNB to avoid strong interference to UL DMRS. - UL resource restriction/blanking including time/frequency resources among gNBs to avoid UL performance degradation due to downlink CSI-RS etc. - Necessity of information exchange considering signaling overhead, latency and implementation flexibility |
| TCL Communication Ltd. [R1-2302408] | gNB-to-gNB CLI Coordinated scheduling Observations 3: For coordinated scheduling of time frequency resources between the gNBs, it may necessary to consider the relevant information exchange between the aggressor and victim gNBs. Observation 4: The knowledge among gNBs about the SBFD time/frequency configuration may assist the gNBs to perform scheduling adaptation and mitigate the gNB-to-gNB co-channel CLI. Observation 5: In addition to the SBFD time/frequency configuration, the exchanging of DL/UL subbands pattern may assist the gNBs for CLI mitigation. Proposal 2: To assist in mitigating gNB-to-gNB CLI during SBFD operation, consider exchanging the subbands pattern among gNBs. Observation 6: An aggressor gNB performing dynamic TDD operation may exchange slot format with its victim gNBs, and an aggressor gNB performing SBFD operation may exchange SBFD slot format as well as the starting and numbers of RBs assigned for each DL and UL sub-bands. Proposal 3: During the simultaneous existence of SBFD and dynamic TDD operations among gNBs, consider at-least the following information exchange among gNBs: - TDD slot format - SBFD time/frequency configuration - Subbands pattern Figure 1 Scheduling adaptation of dynamic TDD (DL) at gNB1 and SBFD operation at gNB2 Figure 2 Scheduling adaptation of same direction dynamic TDD (DL) and SBFD operation at gNB1 and gNB2 Observation 7: In coordinated scheduling for time frequency resources between gNBs, muting the DL RBs or blanking the UL RBs can reduce the effect of gNB to gNB co-channel CLI. Observation 8: In simultaneous implementation of dynamic TDD and SBFD, operation at a gNB and its neighbor gNB the following scheduling adaptation, techniques can reduce or avoid the gNB to gNB co-channel CLI. - Each gNB can assign a time window to the dynamic TDD operation and a time window to the SBFD operation. - Allocating the same numbers of slots or symbols in the time windows assigned to the dynamic TDD or SBFD operation across the neighbor gNBs. Proposal 4: For coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider at least the following. - RB based UL and DL Resource muting to support CLI mitigation in dynamic TDD and SBFD operation. - Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation. |
| OPPO [R1-2302548] | gNB-to-gNB CLI Coordinated scheduling Proposal 5: To support coordinated scheduling between gNBs, more flexible configuration exchange over Xn/F1 interfaces should be studied, e.g. SBFD time/frequency configuration and TDD DL-UL configuration with periodicity longer than 10-ms. |
| Spreadtrum Communications [R1-2302600] | gNB-to-gNB CLI Coordinated scheduling Proposal 2: Support to use pseudo-sequence based muting scheme for inter-gNB CLI handling |
| CATT [R1-2302703] | gNB-to-gNB CLI Coordinated scheduling Proposal 5: For details of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling, at least followings can be studied. - DL resource blanking including time/frequency resource at aggressor gNB - UL resource restriction including time/frequency resources among gNBs - Coordination of SBFD configuration |
| NEC [R1-2302745] | gNB-to-gNB CLI Coordinated scheduling Proposal 4: Following information exchange between gNBs is supported for coordinated inter-gNB scheduling - DL beam scheduling information - DL transmission power information Proposal 5: For inter-gNB CLI mitigation, gNBs exchange with each other the UL subband frequency resource configuration and SBFD time occasions |
| ZTE, China Telecom [R1-2302758] | gNB-to-gNB CLI Coordinated scheduling Observation 3: The gNB-to-gNB CLI can be accurately measured and effectively coordinated only after the related configuration (e.g., SBFD time/frequency, dynamic TDD) of the neighbouring gNB is obtained. Proposal 10: The related configuration (e.g., SBFD time/frequency, dynamic TDD) should be exchanged among gNBs for more accurate CLI measurement and more effective CLI handling |
| Intel Corporation [R1-2302796] | gNB-to-gNB CLI Coordinated scheduling Proposal 2 - For coordinated scheduling for gNB-to-gNB CLI mitigation, study resource blanking and related information exchange between gNBs. - DL resource blanking at aggressor gNB help to protect the UL transmission at the victim gNB. - UL resource blanking at victim gNB can be supported by the existing mechanism on the UL resources that is interfered by the aggressor gNB. - Additional solutions for UL resource blanking by a transmitting UE may involve significant UE complexity and further justifications may be needed. |
| Sony [R1-2302846] | gNB-to-gNB CLI Coordinated scheduling Observation 4: Since the backhaul among gNBs has high latency, exchanging information between gNBs via the backhaul for coordinated scheduling has limited benefit in dynamic scheduling at each of the gNBs. Proposal 4: Introduce new RS that can be used as Over-The-Air (OTA) physical layer signalling between gNBs for scheduling coordination. Observation 5: Signalling of information on Slot & SBFD Format between gNBs is beneficial for coordinated scheduling. Proposal 5: The gNB-gNB RS is used to indicate the Slot & SBFD Format of the gNB transmitting the RS. Observation 6: Since URLLC traffic has ultra-low latency, the gNB may need to schedule a URLLC transmission in a slot even if the gNB is aware that that slot suffers from CLI. It is therefore beneficial that an aggressor gNB is aware of the L1 priority of a victim gNB’s transmission. Proposal 6: The gNB-gNB RS is used to indicate L1 priority of a scheduled transmission. Observation 7: In a sensible network, one gNB does not force another gNB to stop its transmissions/receptions since if every gNB forces every other gNB to blank/restrict its resources, then the entire network would fail to function. Observation 8: If the backhaul (X-interface) is used to signal the resources for blanking/restriction, then a gNB can only promise to blank/restrict resources on some distant future slots, since the backhaul (X-interface) is slow. Observation 9: It is not practical for one gNB to promise another gNB that it would blank/restrict its resources in some distant future slots, since the traffic/scheduling at each gNB occurs dynamically. Proposal 7: Blanking/restriction of resources for coordinated scheduling is not further considered unless the following concerns are addressed: - How does a gNB decides where and when to perform resource blanking/restriction? - How far ahead should a gNB blank/restrict a resource? |
| xiaomi [R1-2302983] | gNB-to-gNB CLI Coordinated scheduling Proposal 13: The RMP can be considered with potential enhancement to support UL reserved resource indication. |
| Lenovo [R1-2303088] | gNB-to-gNB CLI Coordinated scheduling Proposal 7: To enable coordinated scheduling/beamforming, support coordination/matching of TDD DL/UL on certain slots/symbols for use of high-interference beams. This information can be exchanged by adding spatial parameters to the Intended TDD DL-UL Configuration IE. |
| CMCC [R1-2303234] | gNB-to-gNB CLI Coordinated scheduling Proposal 12: For coordinated scheduling for inter-gNB intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g., scheduling information in time-domain, frequency-domain and power domain. |
| Qualcomm Incorporated [R1-2303590] | gNB-to-gNB CLI Coordinated scheduling Proposal 61: Coordinated scheduling information for time/frequency/spatial domain can be exchanged via OTA or BH signalling for inter-gNB CLI mitigation. Proposal 62: Support coordinated scheduling on DL Tx restriction on UL resources between cells. Proposal 63: RAN 1 study semi-static or dynamic coordinated scheduling for inter-gNB CLI mitigation. |

  - Spatial domain enhancement

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | gNB-to-gNB CLI Spatial domain enhancement Observation 1: gNB-to-gNB instant channel is needed for beam nulling to suppress the gNB-to-gNB co-channel cross link interference. Proposal 1: For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, study solutions for channel measurement among multiple gNBs to enable beam nulling. Proposal 2: For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, study solutions for beam pairing considering the following - CLI strength of beam pair over a threshold. - Preferred Tx beams for each receive beam at the victim cell. - Necessity of information exchange considering signaling overhead, latency and implementation flexibility - FR1 digital coordinated beamforming (b) FR2 analogue beam coordination Figure 13. Coordinated beamforming for FR1 and FR2. |
| TCL Communication Ltd. [R1-2302408] | gNB-to-gNB CLI Spatial domain enhancement Observation 9: the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs can help the victim gNB to identify which beam can create CLI. Proposal 5: Consider the information exchange of the preferred/restricted DL and UL beams of the aggressor and victim gNBs with each other, based on the beam ID and TCI state. |
| New H3C Technologies Co., Ltd. [R1-2302430] | gNB-to-gNB CLI Spatial domain enhancement Proposal 9: The beam information exchange can be handled by a central controller. The beam information consists of gNB ID+CLI measurement configuration which including the signal resource ID. Proposal 10: For CSI-RS for CLI measurement, a dedicated indication, such as cli-info, can be introduced in the CSI-RS resource configuration to indicate the usage of this CSI-RS resource. Proposal 11: All the CLI results of all beams should be reported in full report mode, while preferred beam set and non-preferred beam set are reported in partial report mode. The periodic or event-triggered report can be also used for the beam based CLI report. Proposal 12: The central controller determines the non-preferred beam or preferred beam for aggressor gNB according to the dedicated algorithms. The number of the non-preferred beam for one aggressor gNB should not exceed maximum value. Proposal 13: A restriction window can be introduced, where the aggressor gNB cannot use the non-preferred beams, but the victim gNB can use any beam. Several restriction window can be configured, but only one is active. The measurement window is periodic, and determined by the length, periodicity and offset. |
| vivo [R1-2302485] | gNB-to-gNB CLI Spatial domain enhancement Proposal 2: For spatial domain coordination, beam sweeping procedure to identify preferred/non-preferred DL beams of aggressor gNB can bese on implementation. |
| Spreadtrum Communications [R1-2302600] | gNB-to-gNB CLI Spatial domain enhancement Proposal 1: Study the benefit and the procedure of information exchange of preferred/no-preferred DL beams considering the following - Determine preferred/non-preferred DL beams based on beam level RSRP or RSRQ measurements - A threshold can be used to determine preferred/non-preferred DL beams |
| ZTE, China Telecom [R1-2302758] | gNB-to-gNB CLI Spatial domain enhancement Proposal 7: A common understanding of the overall framework of spatial domain gNB-to-gNB CLI coordination should be made firstly. Proposal 8: Rel-18 dynamic/flexible TDD can consider the following framework for CLI management, - Step 0: The victim identifies gNB-to-gNB CLI based on measurement of reference signal from the aggressor (e.g., SSB, CSI-RS or other measurement resource); - Step 1: The victim indicates interference information identified from Step 0, e.g., index of high-interference beam, channel state information for the interference channel, etc, to the aggressor via either air interface or backhaul; - Step 2: The aggressor and/or victim start to perform CLI handling schemes; - Step 3: The victim measures the reference signals sent by the aggressor to evaluate the CLI handling effect; - Step 4: The victim feedbacks the CLI mitigation effect of the different CLI handling schemes. Proposal 9: Spatial domain coordination can be considered by aggressor gNB and/or victim gNB for handling gNB-to-gNB CLI, e.g., - Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information, - Resources to be used by the aggressor for downlink Tx and resources to be used by the victim for uplink Rx are determined according to the preset (or preconfigured) time domain pattern., - Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, e.g., beam nulling. |
| Intel Corporation [R1-2302796] | gNB-to-gNB CLI Spatial domain enhancement Observation 2 - For spatial domain coordination for gNB-to-gNB CLI mitigation, - The combination of indication of the intended Tx/DL beams from aggressor gNB to victim gNB and the preferred/not-preferred Tx/DL beams of the aggressor gNB from victim gNB to the aggressor gNB can be beneficial by enabling coordinated scheduling decisions and appropriate user selection for beamformed Tx/Rx. Proposal 3 - For spatial domain coordination for gNB-to-gNB CLI mitigation, - In addition to the preferred/not-preferred Tx/DL beams of the aggressor gNB that can be signalled from a potential victim gNB to a potential aggressor gNB, the intended Tx/DL beams or beam nulling information of aggressor gNB can be signalled from a potential aggressor gNB to a potential victim gNB. |
| xiaomi [R1-2302983] | gNB-to-gNB CLI Spatial domain enhancement Proposal 11: Rx beam can be indicated by the associated RS (e.g., CSI-RS, SSB, SRS) for gNB-to-gNB CLI management. Proposal 12: The restricted/recommended beam pairs, i.e., restricted/recommended Rx beams for victim gNB and restricted/recommended Tx beams for aggressor gNB, should be configured for gNB-to-gNB CLI mitigation. |
| Nokia, Nokia Shanghai Bell [R1-2303017] | gNB-to-gNB CLI Spatial domain enhancement Observation 6: In scenarios where aggressor gNBs are using static DL-heavy TDD frame configurations, the victim gNB should measure the complex channel matrix and report it back to the aggressor for future precoding matrix adaptation/beam-nulling. Observation 7: Applying restrictions of a large set of the downlink beams might results large downlink performance degradation on the aggressor gNB. |
| Lenovo [R1-2303088] | gNB-to-gNB CLI Spatial domain enhancement Proposal 4: Support victim gNB indicating high-interference (non-preferred) beams to the aggressor gNB or the core network. Additionally, support the victim gNB reporting the amount/level of excess interference corresponding to the high-interference beams. Proposal 5: Support victim gNB indicating preferred and high-priority Tx beams to the aggressor gNB. Proposal 6: Support aggressor gNB indicating information of using high-interference beams to victim gNBs. |
| Samsung [R1-2303128] | gNB-to-gNB CLI Spatial domain enhancement Proposal 7: For gNB-to-gNB co-channel CLI spatial domain enhancements, Xn/F1AP signaling is extended to indicate the reference signal resource ID (NZP-CSI-RS resource ID and SSB index) from the aggressor gNB to the victim gNB and to indicate the preferred/non-preferred DL beams of an aggressor gNB from the victim gNB to the aggressor gNB. |
| Qualcomm Incorporated [R1-2303590] | gNB-to-gNB CLI Spatial domain enhancement Figure 6-14 Tx/Rx inter-gNB beam-pairs Proposal 64: Support to investigate schemes for inter-gNB CLI mitigation in dynamic/flexible TDD and SBFD to identify compatible inter-gNB beam pairs, which can be based on inter-gNB CLI measurement and reporting per candidate DL/UL beam pair. Proposal 65: If measurement CLI RS is configured with RS resource repetitions e.g. to allow victim gNB to scan different gNB Rx beams, then DL beam or DL beam indication can be represented via CSI-RS resource ID # plus repetition # with associated scrambling sequence ID / cell ID for inter-gNB co-channel CLI management. Proposal 66: Support RAN1 to prioritize the study of example 2 in spatial domain coordination agreement for inter-gNB co-channel CLI management. Proposal 67: Support to investigate measurement periodic or event triggered report with contents of allowed/disallowed (recommended/restricted) beams. Proposal 68: Support to investigate related resources and corresponding required power backoff per allowed/disallowed beam. Proposal 69: gNB adopts a slot-specific DL codebook restrictions, where a subset of PMI codebook is restricted in slots where a neighboring gNB has a conflicting traffic direction. Proposal 70: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding matrix - For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots - For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Proposal 71: Beam related coordination info can be sent between victim gNB and aggressor gNB - If the inter-gNB CLI RS is transmitted from aggressor gNB and measured by victim gNB, the coordination info can include allowed/disallowed aggressor gNB DL beam(s), corresponding Tx power backoff and time/frequency resources. - If the inter-gNB CLI RS is transmitted from victim gNB and measured by aggressor gNB, the coordination info can include the intended victim gNB UL beam(s), corresponding intended time/frequency resources and max allowed caused interference level. |
| NTT DOCOMO, INC. [R1-2303712] | gNB-to-gNB CLI Spatial domain enhancement Proposal 6: Information to be exchanged among gNBs should include spatial domain information. |

  - UE and gNB transmission and reception timing

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | gNB-to-gNB CLI UE and gNB transmission and reception timing Observation 2: In current specification, the UL signal and downlink interference can be aligned (within CP) when proper TAoffset is configured and/or proper overall timing of victim cell is applied. The necessity of further enhancement of UE and gNB transmission and reception timing is not clear. |
| vivo [R1-2302485] | gNB-to-gNB CLI UE and gNB transmission and reception timing Proposal 3: Transmission and reception timing adjustment can be supported in Rel-18 dynamic/flexible TDD to accurately estimate interference channel and effectively suppress CLI from aggressor gNB. Proposal 4: For transmission and reception timing adjustment, victim gNB should adjust transmission timing of the served UEs to align with DL transmission signal arrival of aggressor gNB. A negative TA can be configured for UEs served by victim gNB. The timing adjustment is slot specific. |
| Spreadtrum Communications [R1-2302600] | gNB-to-gNB CLI UE and gNB transmission and reception timing Proposal 3: Study the necessity and benefit of adjusting the TA offset to resolve transmission and reception timing misalignment in gNB-to-gNB CLI measurement. |
| ZTE, China Telecom [R1-2302758] | gNB-to-gNB CLI UE and gNB transmission and reception timing Figure-2: Timing difference showed via a field test Observation 2: Based on the field test, a clear timing difference is observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement. Figure-3: Timing difference between different gNBs Proposal 4: RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary of the victim gNB and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement. |
| Intel Corporation [R1-2302796] | gNB-to-gNB CLI UE and gNB transmission and reception timing Proposal 5 - For gNB-to-gNB CLI mitigation, study timing synchronization assistance information exchange between gNBs to enable improved estimation of timing offsets between neighboring gNBs to enable better CLI estimation and its management. |
| Sony [R1-2302846] | gNB-to-gNB CLI UE and gNB transmission and reception timing Observation 10: As per 38.211, a TDD UE expects a time gap of at least NTX-RX = 13 s or 7 s for FR1 and FR2 respectively between the end of an UL transmission and the start of a DL reception for UL to DL switching and this time gap is provided by setting NTA,offset = 13 s. Observation 11: Setting NTA,offset ≤ 0 to align an UL transmission with an aggressor gNB’s DL transmission, i.e. CLI, at a victim gNB’s receiver may lead to: - insufficient time gap (<NTX-RX) at the UE between the end of the UL transmission and the start of a DL reception for UL to DL switching - self-interference at the victim gNB for NTA,offset < 0 due to the UL reception extending beyond the UL slot and into a subsequent DL slot and a DL transmission starting at that DL slot. Figure 15: Adding alignment offset TUL to the overall timing advance Proposal 8: Add a time alignment offset TUL to the overall timing advance, TTA = NTA + NTA,offset + TUL for UL transmissions so that the UL transmission is OFDM symbol aligned with any inter gNB DL CLI at the victim gNB’s receiver and at the same time provide sufficient time gap at the UE between the end of an UL transmission and the start of a DL reception for UL to DL switching. |
| xiaomi [R1-2302983] | gNB-to-gNB CLI UE and gNB transmission and reception timing Observation 5: There is severe ISI between CLI RS and UL data at victim gNB side with non-zero NTA,offset . Observation 6: One CLI RS symbol may result in two UL symbol unavailable at victim gNB side due to the misalignment of timing between CLI-RS arrival and UL timing. Observation 7: For each UL/DL transition at victim gNB, at least one OFDM symbol is not available for the victim gNB if zeroNTA,offset is configured. |
| Nokia, Nokia Shanghai Bell [R1-2303017] | gNB-to-gNB CLI UE and gNB transmission and reception timing Observation 8: Differences in the reception timing of intended UL and interfering DL signals result in IRC receiver performance degradation. Prospoal 10: Study the limitations and trade-offs of adjusting the TA offset including the potential backward compatibility problems between legacy UEs and Rel-18 UEs. |
| CMCC [R1-2303234] | gNB-to-gNB CLI UE and gNB transmission and reception timing Proposal 5: For inter-gNB intra-subband CLI handling, UE and gNB transmission and reception timing alignment can be further studied, e.g., set NTA,offset=0 via information n-TimingAdvanceOffset or define negative NTA,offset. |
| Qualcomm Incorporated [R1-2303590] | gNB-to-gNB CLI UE and gNB transmission and reception timing Proposal 72: Investigate how to determine inter-gNB CLI RS Tx/Rx timing for accurate inter-gNB CLI measurement. Proposal 73: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific TA. - For SBFD, TA configured for SBFD slots can be different from those configured for HD slots. - For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Proposal 74: Simultaneous UL reception and inter-gNB CLI measurement can be achieved by configuring UE with zero or negative TA. |

  - Power control based solution

| Company | Proposals |
| --- | --- |
| vivo [R1-2302485] | gNB-to-gNB CLI Power control based solution Proposal 6: For dynamic TDD/SBFD CLI handling, enhanced UL power control can be considered, e.g., different power control parameters can be used depending on resource allocation or the existence/strength of the CLI. |
| MediaTek Inc. [R1-2302737] | gNB-to-gNB CLI Power control based solution Observation 1: Applying UL power boosting across all UL slots will cause power wastage on non-CLI slots. Proposal 7: RAN1 to study the feasibility of enabling two UL power control loops for gNB-gNB CLI handling in DTDD and SBFD. Proposal 8: Support the use of a bitmap for slot indication to the UE when two UL power control loops are enabled for gNB-gNB CLI handling in DTDD and SBFD. - (b) Figure 16: Post Processing UL SINR for non-CLI and CLI slots with different power offsets: (a) DTDD (b) SBFD - (b) Figure 17: Average UL UPT in victim cell with different power offsets: (a) DTDD (b) SBFD Observation 2: Enabling UL power boosting on CLI slots can significantly improve UL SINR and UL UPT in the presence of gNB-gNB CLI |
| Intel Corporation [R1-2302796] | gNB-to-gNB CLI Power control based solution Proposal 4 - For gNB-to-gNB CLI mitigation, study configuration of separate open-loop power control parameters in different slot or symbols depending on the fixed or dynamic/flexible UL resource allocation. |
| xiaomi [R1-2302983] | gNB-to-gNB CLI Power control based solution Proposal 14: The power adaptation schemes to alleviate the CLI issue can be further studied. |
| Nokia, Nokia Shanghai Bell [R1-2303017] | gNB-to-gNB CLI Power control based solution Observation 4: Uplink power control specifications have high degree of flexibility, current specifications allow a UE can be configured with multiple p0 values. Proposal 8: Enhancements on the signaling between gNBs is required to inform about the desired power reduction at the aggressor(s) cells. Proposal 9: The IAB concepts of Desired DL Tx power adjustment and DL Tx power adjustment can be used as a starting point. Observation 5: System-level simulations show that adjusting the gNB transmit power is a relevant scheme for gNB CLI mitigation. |
| Qualcomm Incorporated [R1-2303590] | gNB-to-gNB CLI Power control based solution Proposal 75: Support of gNB requesting another gNB to have X dB power backoff on time/frequency/spatial resources to mitigate inter-gNB CLI. Proposal 76: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific power control parameters - For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots - For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. |

  - Advanced receiver

| Company | Proposals |
| --- | --- |
| Nokia, Nokia Shanghai Bell [R1-2303017] | gNB-to-gNB CLI Advanced receiver Proposal 6: E-LMMSE-IRC should be considered as a possible solution for CLI mitigation, potentially assisted through information exchange of the CLI aggressor characteristics over the Xn interface (or the F1 interface in case of gNB-split architectures). Observation 2: Existing DL RSs (e.g., CSI-RS) can be used for gNB-to-gNB CLI channel interference measurements. Proposal 7: Signal UL muting patterns to UEs in the victim cell to enable interference channel estimation and cancellation schemes based on advanced receivers, potentially assisted through information exchange of the CLI aggressor characteristics over the Xn interface. Observation 3: Link-level simulations show that UL muting helps improving the accuracy of the receiver estimation to suppress or cancel the interference |

  - Sensing-based

  - Potential enhancement to Rel-16 RIM

| Company | Proposals |
| --- | --- |
| CEWiT [R1-2303304] | gNB-to-gNB CLI Potential enhancement to Rel-16 RIM Observation 11: Enhanced RIM-RS for gNB-to-gNB CLI measurement resistant to the timing synchronisation error that exists between two gNBs. Proposal 11: Support enhanced RIM RS for gNB-to-gNB co-channel CLI measurement. |

- Inter-cell UE-to-UE CLI

  - CLI measurement and reporting

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | UE-to-UE CLI CLI measurement and reporting Proposal 10: The beneficial scenario of the L1/L2 based UE-UE interference measurement and reporting should be studied before discussing the detailed mechanisms. Proposal 11: For L1/L2 based UE-to-UE CLI measurement reporting, the following aspects can be studied: - whether/how to multiplex CLI and legacy CSI metric(s) in a CSI report; - the priority of CLI reports relative to current CSI reports; - the trigger mechanism of semi-persistent, aperiodic CLI reports; - how to reduce the CLI measurements/reports and improve the measurement efficiency; |
| vivo [R1-2302485] | UE-to-UE CLI CLI measurement and reporting Proposal 9: For efficient UE-to-UE CLI measurement and reporting as well as coordinated scheduling, the following enhancements for Rel-16 CLI should be considered. - gNBs should exchange their cell or UE’s SRS configurations over the Xn/F1 interface. - gNBs should exchange the victim UE’s CLI measurement results and associated CLI-RS resources in case the victim UE suffers stronger CLI. Proposal 10: For UE-to-UE CLI measurement and reporting, the following enhancements can be considered. - The beam information can be configured per CLI measurement resource. |
| InterDigital, Inc. [R1-2302523] | UE-to-UE CLI CLI measurement and reporting Observation 3. CLI estimation and reporting at a potential victim UE based on distinguishing aggressor UEs can be used for enhancing CLI mitigation at the UE and further optimal scheduling at the gNB. Proposal 3. Consider enhancements to UE-to-UE co-channel CLI measurement based on supporting CLI measurement and reporting at the potential victim UE that includes distinguishing aggressor UEs. Observation 4. Layer 1 UE-to-UE CLI measurement and reporting could be used for performance enhancement by improving interference measurement accuracy and reducing the reporting overhead, respectively. Observation 5. Layer 1 UE-to-UE delta CLI measurement for the band-edge and the middle-band could be used for performance enhancement by UE reporting an indication if the difference between the two measurements is higher than a threshold. Proposal 4. Consider supporting Layer-1 UE-to-UE L1-CLI-RSSI along with delta-CLI-RSSI measurement and reporting. Observation 6. Joint beam management between victim UE and gNB taking into account beams from aggressor UE can be beneficial in dynamic beam selection for UE-to-UE co-channel CLI mitigation. Proposal 5. Consider enhancements in joint beam management for enhanced CLI measurement between gNB, victim UE, and aggressor UE for optimal beam selection or beam avoidance at the victim UE or aggressor UE, respectively. - Consider the victim UE reporting beams or panels that are preferred, as well as the ones that are not preferred. Figure 1. Directional CLI from aggressor UE in UL received at the victim UE in DL Observation 8. Techniques based on victim UE-initiated CLI reporting based on a configured condition or event to reduce UE complexity could be used to enhance spatial domain coordination in UE-to-UE interference mitigation. Proposal 8. In addition to periodic type of CLI reporting, study the event-based aperiodic CLI reporting to reduce UE complexity, since DL reception failures due to CLI may not happen regularly. |
| OPPO [R1-2302548] | UE-to-UE CLI CLI measurement and reporting Proposal 3: RAN1 targets to support L1-based SRS-RSRP and L1-based CLI-RSSI measurement for UE-to-UE CLI measurement. - R16 configuration of SRS and CLI-RSSI resources should be reused. Proposal 4: L1 UE-to-UE CLI measurement reporting can be a separated CSI report. - R17 CSI reference resource definition should be extended to include the SRS resource and CLI-RSSI resource for UE-to-UE CLI measurement; - R15/16 CSI processing delay should be satisfied. |
| Spreadtrum Communications [R1-2302600] | UE-to-UE CLI CLI measurement and reporting Observation 1: L1/L2 UE-to-UE CLI measurement cannot be performed accurately because of the timing unalignment issue. Proposal 4: Study the necessity and benefit of L1/L2 based UE-to-UE CLI measurement and reporting. Proposal 5: Taking CLI measurement reporting as a part of legacy CSI reporting in the study of L1/L2 based UE-to-UE CLI measurement and reporting. |
| CATT [R1-2302703] | UE-to-UE CLI CLI measurement and reporting Observation 1: Two candidate solutions can be used for beam based measurement. The first one is scheduling victim UE with Rx beam which suffers the least CLI, and the second candidate solution is scheduling aggressor UE with Tx beam which generates least CLI (to victim UE) or avoiding scheduling aggressor UE with Tx beam which generates largest CLI. The information exchange overhead could be significant for the second alternative solution. Proposal 6: Study beam based CLI measurement and reporting, and prioritize solutions with practical information exchange between gNBs. Observation 2: The existing Rel-16 CLI-RSSI measurement resource can be configured with finer granularity at the cost of reduced measurement range or increased signaling overhead and implementation complexity. Observation 3: Subband L1-CLI measurement and report will increase UE implementation complexity and L1 report overhead, and introduce significant specification impact. Proposal 7: Wideband measurement and report can be considered as the baseline of L1-CLI measurement and report, while subband measurement and report can be considered as optional UE capability. Proposal 8：Further for study reporting priority rule, reporting method, computation delay requirements, processing criteria for L1-CLI measurement and report. |
| MediaTek Inc. [R1-2302737] | UE-to-UE CLI CLI measurement and reporting Observation 3: UE-UE CLI-prediction based on measurement in reverse Tx-Rx direction is useful to protect legacy UEs not supporting such measurements. Proposal 11: RAN1 to study the feasibility of using “reverse” UE-UE CLI measurement to protect legacy UEs not supporting such measurements. Observation 4: SRS-RSRP measurement has the following limitations when used for reverse CLI-prediction: - Only the aggregate SRS-RSRP value is reported dropping the values measured per Rx antenna. - SRS-resources transmitted over switched antennae will be reported on separately by measuring UE, causing inefficiency. Proposal 12: UE can be configured to report SRS-RSRP (or CLI-RSSI) per Rx antenna separately. Figure 18: FR2 analog beam patterns influencing (forward/reverse) SRS-RSRP measurement. Observation 5: In FR2 reverse CLI-measurement scenario, measuring UE should be configured to use its Tx analog beam pattern (instead of Rx beam pattern). Proposal 13: SRS-RSRP measurement can be configured with QCL-TypeD (spatial relationship information). Observation 6: Autonomous UE-UE CLI detection can reduce measurement resource overhead and enable faster CLI reporting. Proposal 14: Allow autonomous UE-UE CLI detection and study the details of a corresponding CLI reporting framework. |
| NEC [R1-2302745] | UE-to-UE CLI CLI measurement and reporting Proposal 6: The configuration information for UE-to-UE CLI measurement should include a list of TCI states for CLI beam measurement. Proposal 7: The report configuration/indication information for UE-to-UE CLI should include K (K>=1) TCI states with highest L1-SRS-RSRP or L1-SINR or L1-CLI-RSSI. Proposal 8: Unified design for CLI RS for gNB-to-gNB and UE-to-UE measurement should be considered to reduce the RS overhead. |
| ZTE, China Telecom [R1-2302758] | UE-to-UE CLI CLI measurement and reporting Proposal 1: Take the existing CLI handling schemes defined in the Rel-16 as a starting point for Rel-18 enhancements on dynamic/flexible TDD. Observation 4: The UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange, especially in the typical deployment, e.g., HetNet, of Rel-18 dynamic/flexible TDD. Proposal 11: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18. - For example, exchange timing related information for reception of measurement RS. Proposal 12: L1-based reporting for UE-to-UE CLI should be considered for Rel-18 dynamic/flexible TDD. - Reporting according to defined conditions should be supported to reduce the reporting overhead and measurement effort. - FFS: whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other. Observation 5: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources. Proposal 13: Further study subband CLI measurement and reporting for UE-to-UE CLI handling, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc. Proposal 14: Both the CQI with CLI and CQI without CLI (e.g., CQI measured in case of aggressor’s muting) are reported to the gNB. |
| Intel Corporation [R1-2302796] | UE-to-UE CLI CLI measurement and reporting Proposal 6 - For UE-to-UE CLI mitigation, - Measurement resources and reporting periodicity: may be periodic, semi-persistent, or aperiodic. - Event-triggered reporting of CLI measurement reporting is not pursued further. - Beam information can be configured for a CLI measurement resource. - The measurement resources and L1 or L2 CLI measurement reports can be exchanged between the aggressor and victim gNBs. |
| Sony [R1-2302846] | UE-to-UE CLI CLI measurement and reporting Proposal 11: Study potential timing information that a gNB can provide to a victim UE that would aid the victim UE in time sychronising with an aggressor UE for SRS measurements. |
| xiaomi [R1-2302983] | UE-to-UE CLI CLI measurement and reporting Proposal 1: For L1/L2 based CLI measurement, at least periodic and aperiodic CLI measurement resource should be supported. Proposal 2: For L1/L2 based CLI reporting, at least periodic and aperiodic CLI reporting should be supported. Proposal 3: For L1/L2 based CLI reporting, the event-triggered reporting should be supported. Observation 1: CSI and CQI may need high calculation complexity with non-linear operations. Proposal 4: For L1/L2 based UE-to-UE CLI measurement and reporting, the configuration can be realized via updating CSI-ReportConfig: - Adding CLI-RSRP and CLI-RSSI as component of reportQuantity. - Adding CLI measurement resources as component of CSI-ReportConfig. - Adding event-triggered reporting as component of reportConfigType. Proposal 5: Subband CLI reporting can be considered for UE-to-UE CLI mitigation. |
| Nokia, Nokia Shanghai Bell [R1-2303017] | UE-to-UE CLI CLI measurement and reporting Proposal 12: For inter-cell UE-to-UE CLI measurements, the exchange of the SRS configuration between gNBs is needed to properly configure the CLI-SRS measurements Proposal 13: CSI reporting framework can be considered as starting point but adjustments/enhancements to support the new L1/L2 UE-to-UE CLI measurements might be required. Proposal 14: The UE-to-UE CLI framework to support and define new criteria for event triggered L1/L2 UE-to-UE CLI reporting. Proposal 15: Study autonomous adjustments of the aggressor UE transmit power to reduce the UE-to-UE CLI Proposal 16: Study increased flexibility on the CLI measurements and reporting to support different Rx beams for UEs with beamforming capabilities. Propsoal 17: Support the UE to report the applied timing offset on the CLI SRS-RSRP measurements Proposal 18: Study the benefits of the gNB controlling the time offset applied for the CLI SRS-RSRP measurements to compensate for the different TA configurations between UEs. |
| Lenovo [R1-2303088] | UE-to-UE CLI CLI measurement and reporting Proposal 11: Study to introduce coordination of SRS configurations for SRS-RSRP measurement. Proposal 12: Study benefits and mechanisms for sharing SRS resources among UEs in the aggressor cell. Proposal 13: For the UE-to-UE inter-cell co-channel and inter-subband CLI measurement, common schemes on coordination of SRS configurations and intended TDD DL-UL configurations should be studied. Observation 2: Observed interference level may vary significantly depending on Rx beams and Rx antenna panels. Proposal 14: Support spatially differentiated CLI measurement and reporting. Proposal 15: For L1/L2 based UE-to-UE co-channel CLI measurement and reporting, study periodic/aperiodic/semi-persistent CLI reporting over PUCCH or PUSCH. |
| Samsung [R1-2303128] | UE-to-UE CLI CLI measurement and reporting Proposal 1: UE-to-UE co-channel CLI measurement and reporting configurations should be enhanced to support L1 aperiodic CLI reports. Proposal 2: UE-to-UE co-channel CLI measurement and reporting configurations should be enhanced to support associated spatial domain information. Proposal 3: Xn/F1AP signaling is extended to indicate the configured periodic Rel-16 CLI measurement resource(s) in a cell to co-channel neighbor gNBs. |
| Panasonic [R1-2303167] | UE-to-UE CLI CLI measurement and reporting Figure 19 Example of conjunction with CSI report Proposal 2: For L1-based UE-to-UE CLI measurement and reporting, the conjunction between CLI measurement metrics and CSI measurement metrics should be discussed. Proposal 3: Study subband-based CLI measurement and reporting for UE-to-UE CLI handling. |
| CMCC [R1-2303234] | UE-to-UE CLI CLI measurement and reporting Proposal 1: For L1/L2 based periodic inter-UE CLI reporting, how to reduce the CLI report overhead should be studied. Proposal 2: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the information exchange time between gNBs should be considered for the design of CLI measurement and reporting timeline. Proposal 3: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the detailed SRS resource triggering signaling and report resource indication signaling should be studied, as well as how to reduce the signaling overhead. Proposal 4: For L1/L2 based inter-UE CLI measurement and reporting, event triggered reporting can be supported. The following reporting triggering method can be further studied as examples: - For L1 based event triggered reporting, SR resource can be used for UE to inform gNB the CLI measurement results reporting and PUCCH can be used as reporting resource. - Foe L2 based event triggered reporting, MAC-CE on CG PUSCH can be used by UE to convey measurement results. Proposal 5: For L1/L2 based inter-UE CLI measurement and reporting, the priority of CLI reports relative to the existing CSI reports should be studied. |
| CEWiT [R1-2303304] | UE-to-UE CLI CLI measurement and reporting Observation 1: Factors like synchronization errors between gNB, smaller CP length in higher numerologies, higher propagation delay between the UEs and implementation specific adjustment of reception timing causes the misalignment to go beyond CP duration while measuring the CLI on Rel. 16 SRS as both the UEs are not time synchronized. This will degrade the CLI measurement accuracy. Observation 2: Timing adjustment for transmission or reception by aggressor and victim UE respectively will restrict SRS RSRP measurement in scenarios of multiple aggressor and victim UEs. Observation 3: SRS RSRP measured on phase rotated SRS symbols (Enhanced Rel. 16 SRS) repeated in time domain (similar to RIM RS design principle) has the following advantages- - SRS RSRP accuracy improves as compared to accuracy using Rel. 16 CLI RSRP measurement method based on Rel. 16 SRS. - No need for TA adjustment at the aggressor UE. Thus, SRS RSRP measurement can be done by multiple victim UEs. - SRS RSRP measurement can be done by a single victim UE from multiple aggressor UEs. Observation 4: RIM RS design has already been proven to work in case of gNB-to-gNB interference measurement where there is timing synchronisation misalignment between the gNBs. The same design principle can be applied to enhance the Rel. 16 SRS by phase rotating the symbols and repeating them in time domain. Proposal 1: For L1/L2 based UE-to-UE CLI measurement, enhancement of SRS resource for improving the accuracy of SRS-RSRP measurement is supported. Proposal 2: The enhancement of SRS measurement resource is based on the Rel. 16 RIM RS design principle. Proposal 3: Support DL rate matching around SRS to further improve the CLI measurement accuracy. Observation 5: Periodic measurement and reporting of CLI is unnecessary since CLI is expected to be taken care of when reported. Proposal 4: Study semi-persistent and aperiodic measurement of UE-to-UE co-channel CLI and on-demand reporting of L1/L2 CLI based on the existing CSI framework. Proposal 5: Study enhanced CSI-IM resources with comb pattern that matches with the SRS comb pattern to measure L1/L2 CLI for accuracy improvement. Proposal 6: Consider the existing CSI processing delay for UE as a baseline for CLI measurement processing delay. Observation 6: L1/L2 CLI can be directly used by UE/serving gNB for dynamic handling of CLI and not necessarily shared with the adjacent gNB. Proposal 7: Information exchange delay between gNBs is necessarily not applicable for comparison between L1/L2 CLI and L3 CLI measurement and reporting. Observation 7: Rel. 16 CLI management does not specify required SRS configuration parameters for CLI measurement to be shared across gNBs. Observation 8: In case of partial overlap of BWPs, the victim UE receives only a part of the SRS transmitted by the aggressor UE for measurement of CLI RSRP leading to mismatch in how the SRS sequence is filled by the aggressor and how SRS sequence is interpreted by the victim. E.g., based on simulation analysis, a difference of 1RB between the 2 BWPs will result in an error of around 25 dB. Observation 9: When aggressor and victim UE are operating at different numerology, discrepancy arises in the transmitted and received SRS numerologies that will affect the accuracy of CLI RSRP measurement. Proposal 8: The following information exchange between gNBs is supported for efficient UE-to-UE CLI measurement. - Rel. 16 CLI management related SRS configuration parameters - Numerology of transmission of SRS - A common reference point for CLI RSRP measurement |
| Apple [R1-2303483] | UE-to-UE CLI CLI measurement and reporting Proposal 1: UE is RRC configured with M (M is subject to UE capability) CLI resources per active BWP within the SBFD symbol, where time domain CLI measurement resource configuration shall indicate at which slots and which symbols within that slot, CLI measurement is expected - A CLI measurement resource can be associated to a specific duration (number of slots) or it can be repeated periodically once activated/triggered Proposal 2: UE is indicated about which CLI measurement resource(s) or resource set(s) are activated/triggered as follows - Alt1: L2 based, i.e., through DL MAC-CE (preferred) - Alt2: UE specific DCI or GC-DCI activate the CLI resource(s) or CLI resource set(s) Proposal 3: If UE is aperiodically indicated to report CLI, each CLI report occasion may cover O CLI measurement occasions, where O>=1 and is subject to UE capability Proposal 4: If UE is aperiodically indicated through UL DCI to report CLI, UE capability signaling indicates whether or not UE can measure and report legacy CSI and CLI simultaneously - In case such simultaneous AP reporting of CSI and CLI is under UE capability, CLI is added to the legacy CSI and the encoded bits are multiplexed over PUSCH |
| Qualcomm Incorporated [R1-2303590] | UE-to-UE CLI CLI measurement and reporting Observation 9: Support L1/L2 based UE-to-UE CLI measurement and reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework. Proposal 3: RAN1 to study UE CLI processing timeline at least for separate CLI reporting starting with L1-CSI timeline as a baseline. - e.g. reuse AP CSI timeline as baseline with different value for timeline of L1-CLI. Proposal 4: Support L1 based UE-to-UE CLI measurement and reporting to reduce latency and facilitate gNB adjusting UE scheduling for inter-UE CLI reduction, even for latency stringent traffic. Proposal 5: L1 CLI measurement and report framework supports the following features - Support L1-CLI report with P/SP/AP CLI resources and P/SP/AP report types with more details Proposal 6: RAN1 to further study both schemes for CLI measurement and report: - Scheme 1: Implicitly capture CLI in existing CSI report e.g. via existing CQI and L1-SINR metrics - Scheme 2: Explicitly capture CLI in separate new CLI reportQuantity metrics, e.g. SRS-RSRP and CLI-RSSI Proposal 7: Enhance existing CSI framework by adding configuration of IMR dedicated for inter-UE CLI in a CSI-ReportConfig for scheme 1 of implicitly capture CLI in existing CSI report e.g. via existing CQI and L1-SINR metric. Proposal 8: Multiple CSI reports to learn the CSI metrics with and without considering inter-UE CLI from an aggressor UL UE. Proposal 9: Multiple CLI resources can be configured for multiple candidate UL UEs to measure different CLI levels from different aggressor UEs. Proposal 10: RAN1 to further study enhancement to support at least semi-persistent, or aperiodic measurement resource and reporting in addition to periodic resource and reporting. Proposal 11: RAN1 to further study L1 CLI aperiodic triggering mechanism including associated qcl_info for CLIs. Proposal 12: RAN1 to further study L1 semi-persistent CLI activation and reporting mechanism. Proposal 13: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware gNB beam management for CLI mitigation, for L1 CLI measurement and reporting including P/SP/AP resource and report. Figure 5-20 Subband based CLI reporting Proposal 14: RAN1 to further study L1 subband based or narrower frequency granularity based CLI reporting as a general framework commonly used by both SBFD and dynamic TDD. Proposal 15: RAN1 to further study L1 subband based CLI/CSI reporting configuration. Proposal 16: RAN1 to further study L1 differential subband based CLI/CSI reporting to save overhead. Proposal 17: RAN1 to study L1-CLI report priority, and multiplexing when reported as UCI. Proposal 18: To reduce L1 CLI DCI signalling overhead, a GC-DCI is introduced for triggering both AP SRS transmissions and AP CLI measurement/reporting from a group of UEs. Figure 5-21 GC-DCI for triggering AP SRS and AP CLI measurement and reporting Proposal 19: In addition to most interfering CLI resources, UE can be configured to report top X least interfering CLI resources for CLI report. Figure 5-22 Example L2 CLI framework Proposal 20: Support at least L2 event triggered CLI reporting Proposal 21: Inter-UE CLI measurement RS can be configured and transmitted by aggressor UE or victim UE, which will be measured at victim UE or aggressor UE and report measurement results to its serving gNB. Proposal 22: OAM or CU (if across different gNBs) or gNB (if within a gNB for example for SBFD) can configure the inter-UE CLI transmission parameters, including time/frequency location, RS sequence ID, beam info, periodicity between different UEs of different cells/gNBs, which can be exchanged by gNBs. Proposal 23: OAM or CU (if across different gNBs) or gNB (if within a gNB for example for SBFD) can configure the inter-UE CLI monitoring parameters, including monitoring window location and periodicity between different UEs of different cells/gNBs. Proposal 24: In addition to most interfering CLI resources, UE can be configured to report top X least interfering CLI resources for CLI report which can be applicable commonly for L1/L2/L3 reporting. Proposal 25: Support subband-based CLI reporting to facilitate subband based scheduling for both SBFD and dynamic TDD in which CLI could be non-uniform across the DL RBs for CLI based on L1, L2 or L3 reporting framework. Proposal 26: Support to study information exchange between gNBs for inter-UE CLI measurement and mitigation - UE-to-UE CLI measurement resource configuration between gNBs including time/frequency resources and beam indication for inter-UE CLI measurements between gNBs - UE-to-UE CLI reporting contents including CLI metric per CLI resource Observation 10: UE-to-UE CLI measurement and reporting can be used for gNB scheduling UE in the cell and for gNB coordinated UE scheduling between gNBs. Proposal 27: Signaling of inter-UE CLI measurement report between gNBs can include additional assistant information, such as aggressor UE ID/CLI resource ID, corresponding UE’s future data/control scheduling information, suggested UE power backoff value, beam ID, measured or applied on certain time/frequency resources. |
| NTT DOCOMO, INC. [R1-2303712] | UE-to-UE CLI CLI measurement and reporting Proposal 1: Measurement resource and reporting configuration with spatial information, and configuration for multiple beam measurement should be considered. Proposal 2: Following two options can be considered for the measurement resource for layer-1 measurement, and Option 1 can be baseline for the study. Option 1: Any measurement resource for layer-3 measurement can be used for layer-1 measurement Option 2: Measurement resource for layer-1 measurement is explicitly indicated Proposal 3: For L1 UE-to-UE CLI reporting, existing CSI reporting framework can be reused, and new report quantity is introduced, or layer-1 measurement results is jointly reported with existing CSI report quantity. |
| LG Electronics [R1-2303743] | UE-to-UE CLI CLI measurement and reporting Observation 1. Inter-cell UE-to-UE CLI with both of victim and aggressor UE located at the cell edge is common scenario to both SBFD and dynamic/flexible TDD. Proposal 1. For L1/L2 based UE-to-UE CLI measurement resource, consider aperiodic measurement resource configuration and/or shortened periodicity. Proposal 2. For L1/L2 based UE-to-UE CLI measurement resource, consider spatial domain configuration. Proposal 3. For UEs capable of TA acquisition of candidate cell, TA of candidate cell can be used to accurate inter-cell UE-to-UE CLI measurement. Proposal 4. Consider L1/L2 based UE-to-UE CLI measurement is reported via UCI - UCI for CSI part 1, UCI for CSI part 2, new type of UCI can be considered. Proposal 5. L1/L2 CLI measurement and report reusing existing CSI framework should be investigated in following aspects; - Measurement configuration perspective - Report configuration perspective Proposal 6. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as channel measurement. - The aggressor UE should be indicated to transmit reference signal when victim UE is indicated for L1/L2 CLI measurement. - The victim UE with advanced receiver (e.g., IRC) and capable of distinguishing aggressor UEs is assumed. Proposal 7. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as interference measurement. - Victim UE applies beam used for desired signal from gNB when L1/L2 CLI measurement is indicated. |
| WILUS Inc. [R1-2303831] | UE-to-UE CLI CLI measurement and reporting - Proposal 2: RAN1 to study UE-to-UE CLI measurement and reporting at aggressor UE side for UE-to-UE CLI handling. - Proposal 3: IEs (information elements) of L1/L2 UE-to-UE CLI measurement and reporting can be included in CSI reporting configuration (i.e., CSI-ReportConfig) with new report quantities to measure and report UE-to-UE CLI. |

  - Coordinated scheduling

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | UE-to-UE CLI Coordinated scheduling Observation 6: L3 based UE-to-UE CLI measurement and reporting in current specification may be sufficient for coordinated scheduling. Information exchange between gNBs are needed for the semi-static and dynamic coordinated scheduling. Proposal 13: For details of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling, at least DL/UL resource blanking/reservation/muting including time/frequency resource can be studied. Besides, followings can also be studied. - Potential impact of traffic load. - Necessity of information exchange considering signaling overhead, latency and implementation flexibility |
| CATT [R1-2302703] | UE-to-UE CLI Coordinated scheduling Proposal 9: For details of coordinated scheduling for time/frequency resources between gNBs for UE-to-UE co-channel CLI handling, DL/UL resource blanking/reservation/muting can be studied. |
| Intel Corporation [R1-2302796] | UE-to-UE CLI Coordinated scheduling Proposal 7 - For UE-to-UE CLI mitigation, study coordinated scheduling schemes focusing on: - Inter-gNB information exchange on user selection. - Inter-gNB information exchange on DL/UL resource blanking/reservation/muting. - Inter-gNB information exchange on scheduled PRBs, subbands, etc. - Assistance information between UE and gNB to facilitate coordinated scheduling. |
| CMCC [R1-2303234] | UE-to-UE CLI Coordinated scheduling Proposal 6: For coordinated scheduling for inter-UE intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g., - Example 1 (2-step negotiation): CLI-SRS resource configuration and the request for scheduling avoidance of the aggressor UE (associate with certain CLI-SRS index) at certain pre-configured resources in time/frequency domain - Example 2 (3-step negotiation): CLI-SRS resource configuration, the request for scheduling information of the aggressor UE (associate with certain CLI-SRS index) and the information of pre-choregraphed scheduling information of the aggressor UE (associate with certain CLI-SRS index) - Example 3 (1-step negotiation): CLI-SRS resource configuration and the corresponding pre-configured candidate DL resources subset for the associated aggressor UE Proposal 7: For coordinated scheduling for inter-UE intra-subband CLI handling in spatial domain, victim UE can report the recommended beams along with the CLI measurement results. |
| Apple [R1-2303483] | UE-to-UE CLI Coordinated scheduling Proposal 5: For co-channel CLI handling for dynamic TDD and/or SBFD, study feasibility and benefit of R17 IAB solutions for coordinated scheduling between gNBs, e.g., Desired and/or prohibited beams, associated with SBFD slots/symbols Coordinated scheduling on resources used for each link direction, associated with SBFD slots/symbols Proposal 6: To assure symbol level alignment at UEV, UEA is indicated to hold two different Tas - one TA for symbols on which TRP is doing legacy TDD, another TA for symbols on which TRP is doing SBFD or dynamic TDD |
| Qualcomm Incorporated [R1-2303590] | UE-to-UE CLI Coordinated scheduling Proposal 28: Coordinated scheduling information for time/frequency resources and corresponding UE information can be exchanged via OTA or BH signalling for inter-UE CLI mitigation. |

  - Spatial domain enhancement

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | UE-to-UE CLI Spatial domain enhancement Proposal 12: For spatial domain enhancement of UE-to-UE co-channel CLI handling, study solutions for beam pairing in FR2 considering the following - CLI strength of beam pair (RSRP, RSSI) that is over threshold A/below threshold B - Preferred/restricted beams between UEs - Preferred Tx beams of candidate scheduled aggressive UE and preferred Rx beams of the candidate scheduled victim UE - Necessity of information exchange considering signaling overhead, latency and implementation flexibility |
| InterDigital, Inc. [R1-2302523] | UE-to-UE CLI Spatial domain enhancement Observation 7. In spatial domain coordination, there are two aspects to be considered: - Preventive aspects, that is determining the victim and aggressor UEs beam pairs to be avoided - Beam pairing aspects, that is determining the gNB and victim UE beam pairs to be used based on directional CLI from the aggressor UEs. Proposal 6. Consider preventive aspects in spatial domain coordination by determining the most and least favourable beam pairings between the victim and aggressor UEs. Proposal 7. Consider CLI mitigation aspects in spatial domain coordination by determining beam pairing between victim UE and gNB based on directional CLI. Observation 13. A beam failure instance due to CLI may occur even when the signal received from gNB is not physically blocked, where the degradation in the DL radio link is mainly due to the interference from an aggressor UE. Proposal 12. Study enhancements in beam failure detection and recovery, in case the beam failure is caused by UE-to-UE CLI. - Consider panel switching mechanism as part of beam failure recovery procedure due to the nature of the UE-to-UE CLI. |
| Spreadtrum Communications [R1-2302600] | UE-to-UE CLI Spatial domain enhancement Proposal 6: Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method considering the following - Use results of beam management of gNB and UE as the baseline - Exchange information of best UE pairs. |
| NEC [R1-2302745] | UE-to-UE CLI Spatial domain enhancement Proposal 9: - Differentiation the BFR caused by CLI with the beam blockage is needed. - Eliminate the effect of the CLI to BFR for BFD and NBI should be considered. |
| Intel Corporation [R1-2302796] | UE-to-UE CLI Spatial domain enhancement Proposal 8 - For UE-to-UE CLI mitigation, study spatial domain coordination schemes focusing on: - Inter-gNB information exchange on use of or intended Tx beams. - Inter-gNB information exchange on preferred/not-preferred Tx beams. - Methods for identification of Tx beams, e.g., via mapping to SRS resource indices. |
| xiaomi [R1-2302983] | UE-to-UE CLI Spatial domain enhancement Figure 23 The reception of CLI RSs configured with different Rx beams Proposal 8: Support beam based CLI measurement for UE-to-UE CLI mitigation. |
| Panasonic [R1-2303167] | UE-to-UE CLI Spatial domain enhancement Proposal 4: Study how to include spatial domain information to facilitate efficient UE pairing to avoid UE-to-UE CLI Proposal 5: UE-to-UE reporting for spatial domain coordination using L1 or L2 reporting should be studied. |
| CEWiT [R1-2303304] | UE-to-UE CLI Spatial domain enhancement Observation 10: In Rel. 16, the gNB cannot configure a UE to measure CLI RS using beam sweeping/different Rx beams. Proposal 9: Support gNB configuring different Rx beams for UE-to-UE CLI measurement. Proposal 10: Support separate UE-to-UE CLI measurement report corresponding to different receive beam configurations. |
| Qualcomm Incorporated [R1-2303590] | UE-to-UE CLI Spatial domain enhancement Figure 5-24 Rx QCL-D for CLI Measurement with multiple UE panels Proposal 29: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware gNB beam management for CLI mitigation, which can apply to L1/L2/L3 CLI measurement and reporting including P/SP/AP resource and report. - For P CLI resource for L1/L2/L3, corresponding TCI state/QCL-D can be RRC configured. - For SP CLI resource for L1, corresponding TCI state/QCL-D can be dynamically updated via MAC-CE (de)activating the resource or resource set/list - For AP CLI resource for L1, corresponding TCI state/QCL-D can be RRC configured with each resource or resource set/list associated with a trigger state, which is further dynamically indicated in the triggering DCI, and current AP CSI triggering mechanism can be used as baseline. Proposal 30: UE can dynamically report to the gNB a set of recommended beams, not preferred beams, or both. - gNB configures multiple Rx (QCL-D) beams for UE to measure - UE determines the recommended and/or not preferred beams based on measurement of inter-UE CLI using different RX beams (QCL-D) Proposal 31: Inter-UE CLI can be mitigated by configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding codebook - For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots - For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. Proposal 32: Support inter-CU/vendor coordination on exchanging scheduled data/control UE beams to mitigate inter-UE CLI. Proposal 33: Support inter-CU/vendor coordination exchange information on number of required CLI resources, e.g. the total number could be # of measured Tx beams of UL UE multiply # of Rx beams of DL UE. |

  - UE and gNB transmission and reception timing

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon [R1-2302349] | UE-to-UE CLI UE and gNB transmission and reception timing Observation 7: The current timing scheme for UE-to-UE CLI measurement may be sufficient. Observation 8: The benefits of enhancement on reception timing of SRS from one aggressor UE for SRS-RSRP measurement are not clear. |
| vivo [R1-2302485] | UE-to-UE CLI UE and gNB transmission and reception timing Observation 7: When the victim UE suffers CLI from multiple aggressor UEs with different TAs, CLI may exceed the CP duration of the victim UE which would bring severe interference on DL reception. |
| InterDigital, Inc. [R1-2302523] | UE-to-UE CLI UE and gNB transmission and reception timing Observation 9. UE and gNB timing alignment could be effective in performance enhancement for UE-to-UE CLI measurement and accuracy. Proposal 9. Study timing alignment issues including subband non-overlapping full duplex scenarios. |
| Spreadtrum Communications [R1-2302600] | UE-to-UE CLI UE and gNB transmission and reception timing Proposal 7: TA adjustment of UEs is deprioritized for transmission and reception timing of UE-to-UE CLI measurement in Rel-18 dynamic/flexible TDD. |
| Intel Corporation [R1-2302796] | UE-to-UE CLI UE and gNB transmission and reception timing Proposal 10 - For UE-to-UE CLI mitigation, study: - assistance information from a serving gNB to a UE for adjustment of reception time window for CLI measurements, and - timing synchronization assistance information exchange between gNBs to enable improved estimation of timing offsets between neighboring gNBs. |
| xiaomi [R1-2302983] | UE-to-UE CLI UE and gNB transmission and reception timing Observation 2：The time offset between DL reception timing and CLI-RS arrival timing at victim UE side can be determined by UE’s TA information. Proposal 6: The misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation. - The reception window for multiple aggressors Observation 3：In most scenarios, the arriving time of CLI-RSs are within the same CP duration even the CLI-RSs are from multiple aggressor UEs. Proposal 7: The simultaneous reception of multiple SRSs from different aggressor UEs for CLI measurement can be realized by gNB. |
| Qualcomm Incorporated [R1-2303590] | UE-to-UE CLI UE and gNB transmission and reception timing Proposal 34: The CLI measurement UE can recommend TA adjustment for aggressor UE corresponding to a particular CLI resource transmission. Proposal 35: The CLI measurement UE can report Rx timing difference between UE DL arrival timing and CLI RS arrival timing to help align the timing at the DL UE for inter-UE CLI reduction. Proposal 36: Inter-UE CLI can be mitigated by configuring slot-specific TA. - For SBFD, TA configured for SBFD slots can be different from those configured for HD slots. - For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. |

  - Power control based solution

| Company | Proposals |
| --- | --- |
| InterDigital, Inc. [R1-2302523] | UE-to-UE CLI Power control based solution Observation 10. Dynamic UL power control mechanisms based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL should be considered in performance enhancement for UE-to-UE CLI mitigation. Observation 11. Dynamic DL power backoff/control mechanisms at gNB could be used to deal with self-interference caused by the FD operation at the gNB, where such mechanism could impact UE behaviours including CSI-RS measurements depending on the amount of the power backoff. Proposal 10. Study power-control based mechanisms for UE-to-UE CLI mitigation and issues related to gNB’s transmission power backoff/adjustment. |
| OPPO [R1-2302548] | UE-to-UE CLI Power control based solution Proposal 6: Existing power control mechanism with separate open loop power control parameters can be reused for UL transmissions with CLI and without CLI. |
| CATT [R1-2302703] | UE-to-UE CLI Power control based solution Proposal 10: The benefit and loss generated from UL power control based solution should be carefully evaluated. |
| ZTE, China Telecom [R1-2302758] | UE-to-UE CLI Power control based solution Proposal 15: Regarding UE-to-UE CLI handling in power domain, it should be supported to configure separate sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(PCMAX), etc, for UL transmission in different resources with/without UE-to-UE CLI. Proposal 16: The unified UL power control solution applied to both of gNB-to-gNB CLI and UE-to-UE CLI handling can be considered. |
| Intel Corporation [R1-2302796] | UE-to-UE CLI Power control based solution Proposal 9 - Consider a common UL PC framework to address gNB-to-gNB and UE-to-UE CLI mitigation. - For UE-to-UE CLI mitigation, study configuration of separate open-loop power control parameters in different slot or symbols depending on the fixed or dynamic/flexible UL resource allocation. |
| CMCC [R1-2303234] | UE-to-UE CLI Power control based solution Proposal 8: Two power control loops can be used for different slot types, e.g., SBFD slot and normal UL slot in SBFD system or aligned and unaligned slot in flexible/dynamic TDD system. - FFS: Enhancements on CG PUSCH or PUCCH/PUSCH repetition across different slot types. |
| Apple [R1-2303483] | UE-to-UE CLI Power control based solution Proposal 7: Reuse existing signaling and procedure to manage for UE-to-UE CLI by UL power control mechanism. Proposal 8: Further study the feasibility, and impacts to legacy UE, for DL power adjustment |
| Qualcomm Incorporated [R1-2303590] | UE-to-UE CLI Power control based solution Proposal 37: CLI measurement UE can recommend UL power backoff for neighbor UL UE corresponding to a particular CLI resource. Proposal 38: CLI measurement UE can recommend DL power boost to cope with the CLI from neighbor UL UE corresponding to a particular CLI resource. Proposal 39: gNB may indicate UL power limit for certain interfering UE to ensure caused CLI is always under limit. Proposal 40: Investigate UL UE autonomously adjust Tx power to limit inter-UE CLI caused to DL UE based on inter-UE pathloss measurement. Proposal 41: Inter-UE CLI can be mitigated by configuring slot-specific power control parameters - For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots - For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells. |
| WILUS Inc. [R1-2303831] | UE-to-UE CLI Power control based solution - Proposal 4: RAN 1 to study UL power control-based solution for UE-to-UE CLI handling based on L1/L2 UE-to-UE CLI measurement and reporting at aggressor UE side. - Existing UL power control parameter set can be reused. |

  - Advanced receiver

  - Sensing-based

| Company | Proposals |
| --- | --- |
| NEC [R1-2302745] | UE-to-UE CLI Sensing-based Proposal 10: Enhancement for the flexible symbols allocation can be studied, such as: - Methods to achieve different UE interpretation different slot format for flexible symbols can be studied. - LBT scheme can be applied to determine the flexible symbols used for DL or UL transmission. |

- Reference

[1] R1-2302349, "Study on potential enhancements on dynamic/flexible TDD", Huawei, HiSilicon

[2] R1-2302408, "Potential enhancement on dynamic/flexible TDD", TCL Communication Ltd.

[3] R1-2302430, "Discussion on potential enhancement on dynamic/flexible TDD", New H3C Technologies Co., Ltd.

[4] R1-2302485, "Potential enhancements on dynamic/flexible TDD", vivo

[5] R1-2302523, "Discussion on potential enhancements on dynamic/flexible TDD", InterDigital, Inc.

[6] R1-2302548, "Discussion on potential enhancements on dynamic/flexible TDD", OPPO

[7] R1-2302600, "Discussion on potential enhancements on dynamic/flexible TDD", Spreadtrum Communications

[8] R1-2302703, "Discussion on potential enhancements on dynamic/flexible TDD", CATT

[9] R1-2302737, "Discussion on potential enhancements for dynamic/flexible TDD", MediaTek Inc.

[10] R1-2302745, "Views on enhancements of dynamic/flexible TDD", NEC

[11] R1-2302758, "Discussion of enhancements on dynamic/flexible TDD", ZTE, China Telecom

[12] R1-2302771, "Potential enhancements on dynamic/flexible TDD", Ericsson

[13] R1-2302796, "On potential enhancements for dynamic/flexible TDD operations", Intel Corporation

[14] R1-2302846, "Considerations on Flexible/Dynamic TDD", Sony

[15] R1-2302983, "Discussion on potential enhancements on dynamic/flexible TDD", xiaomi

[16] R1-2303017, "Dynamic TDD enhancements", Nokia, Nokia Shanghai Bell

[17] R1-2303088, "Potential enhancements on dynamic/flexible TDD", Lenovo

[18] R1-2303128, "Dynamic and flexible TDD for NR duplex evolution", Samsung

[19] R1-2303167, "Discussion on potential enhancements on dynamic/flexible TDD", Panasonic

[20] R1-2303234, "Discussion on potential enhancements on flexible/dynamic TDD", CMCC

[21] R1-2303304, "Discussion on enhancements on dynamic/flexible TDD", CEWiT

[22] R1-2303483, "Views on potential enhancements on dynamic TDD", Apple

[23] R1-2303590, "On potential enhancements on dynamic/flexible TDD", Qualcomm Incorporated

[24] R1-2303712, "Discussion on potential enhancements on dynamic/flexible TDD", NTT DOCOMO, INC.

[25] R1-2303743, "Study on Potential enhancements on dynamic/flexible TDD", LG Electronics

[26] R1-2303831, "Discussion on potential enhancements on dynamic/flexible TDD", WILUS Inc.