3GPP TSG RAN WG1 #112b-e	R1-2304036

e-Meeting, Apr 17 - Apr 26, 2023

Agenda Item:	9.11.3

Source:	Moderator (Nordic Semiconductor ASA)

Title:	Summary #1 of discussions on L1 signal design and procedure for low power WUS

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

This contribution summarizes the discussions on L1 signal design and procedure for low power WUS in RAN1#112b-e.

Section 2 provides a summary of the outcome. Section 3 documents the detailed discussions. Companies’ proposals from the contributions are captured in the Section 5. TDOCs are referenced in Section 4.

At this point, please provide kindly input at least to proposals and questions marked with FL2-Higher, but it is allowed to comment to FL2-Lower as well.

# Outcome

## Quantitative analysis, results

- In general, companies are OK with approach to look on aspects which has been evaluated by more companies. It has been pointed out that S6 , Manchester coding was simulated almost by everyone. - We agreed last meeting on studying, also S observations could be assigned to below bullets - impact of timing error (O3) - impact of frequency error (O1) - impact of phase noise and I/Q imbalance, if applicable () - impact of ADC resolution and sampling rate (O4 + O5) - impact of interference - impact of delay spread/frequency-selectivity (O2) - impact of doppler spread It is suggested that having above, we could focus study on answering more detailed questions, which could facilitate getting more focused results in next meeting: FL2-Higher-Proposal-2b: To focus simulation effort further, focus on answering the following: - For each waveform scheme assess sensitivity to timing and frequency error - How frequency error sensitivity is impacted by choice of guard-bands at the edges and between segments. - Study sensitivity of segment to frequency-selectivity as function of its size. - What are sampling rate requirements and ADC dynamic range requirements for each waveform scheme. - Performance with and without Manchester coding - Whether ACI and phase-noise impacts different waveforms differently - Does sequence based (Option 2) perform better or worse than message+CRC (Option 3) - Anything else?

## Discuss further on KPI of waveform selection

Selection of waveform(s) should take into account. - Receiver power consumption and complexity - Latency and resource overhead - Coverage, i.e., required SNR at target MDR and FAR - Feasibility to generate waveform using existing hardware. - Capability to support flexible bitrate? - Capability to support different receiver structures? Few companies say it is too early to agree anything, surely FL agree, and we could stop here. Intension of FL is to encourage more than 2 companies to contribute on the aspect in the future, because this will be big headache of study item.

## Coverage

FL1-Higher-Question-6: Should RAN1 (a) discuss how coverage of NR could be met, (b) accept that coverage is unbalanced and discuss how such unbalanced coverage between LP-WUS and NR could be handled, or (c) wait for .1 agenda to form conclusion on coverage first.

Summary

a) Sony, LGE, QC (target first), CTC

b) LGE

c) Xiaomi, HW, Samsung, MTK, Apple, Intel, Nokia

Seems wait for 9.11.1 is majority

## OOK prioritization

OOK-2 seems still wanted to be studied by 3 companies.

OOK-3 only proponent wants further study. It is also pointed out the receiver is more of simplified FFT receiver and scheme could be regarded more of an OFDMA waveform.

FL2-Higher-Proposal-2a:

For OOK schemes, prioritize further study of OOK1/2/4. OOK-3 can be still optionally studied.

## Placement of LP-WUS

This could be useful for RAN4 LS

FL2-Higher-Proposal-8:

- The frequency location of LP-WUS is recommended to be configurable within carrier/BWP in a band.

  - band can be the same as band of MR

  - band can be TDD or FDD band

- Study further

  - band can be different than band of MR

  - LP-WUS can be configurable within guard-band of a band (like NB-IoT)

## BW of LP-WUS

This could be useful for RAN4 LS, it is still controversial how many BW to support, this starts from supporting one

FL2-Higher-Proposal-6

At least for IDLE/Inactive mode

- BW-size of LP-WUS is recommended to be less than 20MHz for FR1.

- at least one BW-size <=5MHz is recommended to be supported [to accommodate LP-WUS within all NR supported channel BWs]

## RRM measurements

I used word reference signal(s) to cover SSB or LP-SS. Intention is to focus study on serving cell relaxations first, which are not yet relaxed in NR.

FL2-Higher-Proposal-9:

Prioritize study on offloading of RRM measurements of serving cell to LP-WUR and relaxation of RRM measurements of serving cell in MR

- periodic reference signal(s) is/are used for measurements in LP-WUR.

- periodic reference signal(s) is/are cell dependent.

- MR performs serving cell measurements

  - Alt1: with relaxed periodicity.

  - Alt2: only when LP-SS based measurements are below threshold.

- FFS: relaxation of neighbour cell measurements

- FFS: reference signal(s) to measure, e.g. SSB, OOK/FSK sequence (LP-SS)

- FFS: periodicity

- FFS: content, e.g. cell ID

## FR1 pioritization

FL2-Higher-Proposal-20: Prioritize study of FR1

## Monitoring

Majority view is duty-cycle monitoring, some companies would like to support also continuous monitoring, e.g. once a slot. The following proposal could be tried.

FL2-Higher-Proposal-16:

- It is recommended to support configurable duty-cycled based LP-WUS monitoring.

  - Study further what should be upper bound of monitoring periodicity [which is impacted at least by accuracy of RTC clocks and periodicity of LP-SS (if supported).]

  - Study further what should be lower bound of monitoring periodicity [to enable low latency of paging]

# LP WUR signals and procedures

## Waveforms

Discussion on waveform has been part of almost every contribution, this is high priority topic.

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

### Waveform Generation

OOK-4 generation

For OOK-4 when all ones are input to DFT block the spectrum has single spike, which is sensitive to fading channel and may impose challenges in waveform generation at the gNB [12][2][5][20].

To resolve this issue, phase scrambling before DFT has been proposed in [2][5][13][20] [24] [27]. This flattens spectrum, it improves robustness to frequency-selective fading or improves frequency diversity [24]. However, it also increases PAPR in time domain which can cause trouble to envelop detector [13]. [27] points out that for OOK4, in general, PAPR is higher.

Contributions proposed the following phase scrambling methods:

- alternate “1” and “-1” [2]

- random QPSK [20]

- Zadoff-chu [2][5][12][20]

FL comment: On one hand, generation of the waveform could be left to gNB implementation, on the other hand, LP-WUR receiver could be optimized for a particular waveform generation and therefore it would be beneficial to specify the input signal of the DFT block.

FL1-Higher-Proposal-1: For OOK-4, study further the following phase scrambling methods for modulated input signal of the DFT/Least-Square block, and study PAPR of generated waveform.

- alternate “1” and “-1”

- (pseudo)-random QAM sequence

- Zadoff-Chu sequence

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Partial Y | Our understanding is that - Option (1), i.e., alternating between “1” and “-1” will simply result in a cyclic frequency shift at the output of the DFT, so it may not have the desired outcome of a flatter frequency spectrum. - Option (2) will require the identification of a sequence that will result in a flat frequency spectrum. - Option (3) should already result in a flat frequency response as a characteristic of the ZC sequence |
| EURECOM | Y | We agree that modulated signal should be specified, because it will impact the receive filter design. Note that the encoding scheme will also impact the PAPR. Moreover, we think that other options should not be excluded at this point. |
| Huawei, HiSilicon | Partially Y | First, we are fine to further study the waveform generation of the above three phase scrambling methods, but the study should not be limited to ‘OOK-4’. For any OOK option or FSK option, the above methods can be applied. Second, for the QAM sequence, it is not necessarily to be ‘random’. As the FL commented, UE can get benefit from the known waveform generation for specific receiver architecture. Third, the study on ‘PAPR’ is a little confusing. On the one hand, we agree with [27] that for OOK, the PAPR is generally high considering there is ON/OFF switching. On the other hand, all the mentioned three methods are keeping a constant amplitude with phase changing. Then the PAPR within ON symbol of OOK (or within FSK symbol) should be quite small. So we don’t see strong need to study the PAPR here. Therefore, we suggest the following modification in red. FL1-Higher-Proposal-1: For MC-OOK-4 and MC-FSK, study further the following phase scrambling methods for modulated input signal of the DFT/Least-Square block, and study PAPR of generated waveform. - alternate “1” and “-1” - (pseudo)-random QAM sequence - Zadoff-Chu sequence |
| Samsung | Y in principle | We don’t need to mention “phase scrambling” in the proposal, and the wording “study further the following phase scrambling methods for modulated input signal of the DFT/Least-Square block” is clear enough. |
| MTK | N | The issue is valid, but it can depend on gNB implementation for the following reasons. - There are many ways to implement phase scrambling by gNB. - Reducing PAPR and flattening the spectrum can conflict, so there may not be an optimal solution. - PAPR requirements and RE power limitation for a gNB is unclear to RAN1. - There is no evidence to show any benefit for LPWUR from knowing how gNB implements phase scrambling. If it is for LLS calibration, we recommend using a simple method like random QPSK proposed by [20]. |
| ZTE, Sanechips | Generally Y | Different gNB implementation can have different waveform performance, which impacts the WUR’s coverage. If it is only up to gNB implementation, the LP-WUS’s performance can not be guaranteed. Therefore, this issue should be studied. Add another subbullet as follows and other options are not precluded - (pseudo)-Random phase, for example, , where - Other phase scrambling methods are not precluded. Considering different methods may have different PAPR impacts and not all the methods can keep constant amplitude, the study for PAPR is needed. If there is no impacts finally, a relevant conclusion can be made. |
| OPPO | Partially Y | The general proposal for this problem is fine in the study. However, those solutions may not need to be standardized. Also, the impact to the receiver should be considered if we start to change the waveform defined. If there is no strong issue, the original options should be kept. |
| Sharp |  | We agree to study and specific some scrambling methods, and others schemes should not be precluded. |
| vivo | Partially yes | It is beneficial for phase scrambling to flatten spectrum for better frequency diversity. There may exist different candidate methods, among which a trade-off of the PAPR performance and frequency diversity performance can be achieved. Therefore, it is not necessary to limit the candidate schemes for study. We suggest to remove the sub-bullets and study the phase scrambling methods in terms of frequency diversity and PAPR performance. FL1-Higher-Proposal-1: For OOK-4, study further the following phase scrambling methods for modulated input signal of the DFT/Least-Square block, in terms of frequency diversity and PAPR performance. and study PAPR of generated waveform. - alternate “1” and “-1” - (pseudo)-random QAM sequence - Zadoff-Chu sequence |
| SONY |  | OK. We are OK to study these methods. It is unclear whether there would potentially be any specification impacts from these methods or whether these are just implementation methods of generating the OOK-4 signal. |
| Apple | Partially Y | We are fine with the spirit of proposal. Here are some comments: - It should be clarified whether this is specifically to consider the flattened spectrum, or it is intended to be a generic study for the waveform to achieve good performance overall. - If it is the former, we should add the motivation to the main bullet to clarify the purpose. - If it is the latter, the study should be applicable to all the OOK options and FSK options using a frequency segment instead of a single subcarrier. - It is not clear to us how the first option would result in flat frequency spectrum. Some explanation would be useful before we include it in the list. - Other options should not be excluded. |
| LGE | Partial Y | Agree with the main bullet. Regarding the listed options as sub-bullet, it may need to further study on it. For example, the purpose of the 1st sub-bullet seems not to flatten the spectrum. In addition, ZC sequence could give the flatter spectrum, but it may not be the only solution. We think it’s too early to conclude details. |
| NEC | Y | We agree to further study the three alternatives. |
| Ericsson1 |  | OK to study. Prefer formulation proposed by Samsung |
| Intel | Partial Y | Firstly, we would like to clarify that the phase scrambling respectively applies to each of the M OOK symbols in an OFDM symbol. The modulated signal aims to reduce PAPR in time domain as well as flat in frequency domain. It is unclear how Option 1 can flatten the frequency domain. We are fine with study on option 2 and option 3, with preference on option 3 which is well-studied since LTE. |
| Nokia/NSB | Y | Having 1 and -1 will cause spectral leakage compared to other techniques as it is like FFTSHIFT operation, which puts the stronger center peak (highlighted by many companies) to the edges, which leads to stronger spectral leakage. However, we should not restrict only to the given alternatives. |
| Lenovo | Yes | We agree to further study all three alternatives |
| DOCOMO | Y | We agree to further study the three alternatives. |
| QC1 QC2 |  | We are ok to study further about scrambling methods. The provided methods better to be captured as examples. For OOK-4, study further the following phase scrambling methods including following methods for modulated input signal of the DFT/Least-Square block, and study PAPR of generated waveform. - alternate “1” and “-1” - (pseudo)-random QAM sequence - Zadoff-Chu sequence Other phase scrambling sequences are not excluded. |
| Spreadtrum | Partial Y | Share similar view as MTK. It may be for evaluation purpose. We are not sure whether the TS should include the sequence for bit 1 for LP-WUS. If LP-WUS can be used for aperiodic synchronization, the TS should include the sequence for bit 1. |
| CTC | Principle Y | In Rel-16 e-MIMO enhancement, a scrambling is added in downlink 2 CDM group DMRS sequence generation process to lower down PAPR value, which means downlink low-PAPR transmission is meaningful. However, operation of PAPR reduction on BS side is generally dependent on the implementation, only a few studies have largely focused on it (e.g. NWS), and low PAPR transmission on the UE side is more common. Besides, coverage performance is currently the more important issue for WUS. Hence, it can be studied but should not spend much time and can not affect the demodulation performance of WUS. |
| FL2 |  | Thanks all for good comments. - It is pointed out that by several companies alternate “1” and “-1” will not flatten spectrum, it will move it to edges of spectrum. Causing spectral leakage. - Some companies think that specification of phase alternation is needed, some think it can be left up to gNB implementation. Some companies say selection of “modulation of input signal to DFL/LS block” would impact performance. - It should be clarified what is the main purpose of study - improving frequency diversity by flattening the spectrum - avoiding issues with dynamic range in frequency - keeping time domain PAPR low, to improve performance of reception - Companies do not want to lock details at this point. - It is pointed out that by one company that similar questions may arise for other scheme that directly populate Based on comments, I made the update of proposal FL2-Higher-Proposal-1a: Study further methods to modulate input signal of the DFT/Least-Square block for OOK-4, and to modulate input signal of N SCs for other MC-ASK/FSK schemes - study whether there is a need to specify input signal or it can be left up to gNB implementation - study methods with respect to - improving frequency diversity by flattening the spectrum - avoiding issues with dynamic range in frequency domain or spectral leakage at gNB - keeping PAPR of generated time domain signal low, to improve performance of reception. - improving robustness to timing error (Note: comes from Question 2) |

For OOK-4, benefits from using truncation block is not clear, while additional complexity caused by truncation is there [9]. On the other hand, [2][24] point out that signals can be pre-processed by gNB [2][24]. Therefore, there may not be any complexity from additional signal processing form truncation or pulse shaping. At the same time, proponents of truncations should explain benefit.

FL1-Lower-Question-1: What is benefit of generating N’ symbols instead of directly N, and using truncation.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | In our understanding, when truncation is used the N’ is greater than N, for example in this case N’ could be the length of FFT/IFFT instead of DFT, while N is equal to the number of REs (also the DFT length). One potential benefit is that gNB can choose N’=2α to enable FFT processing instead of DFT processing. But we also agree that the signal can pre-generated and stored in gNB. So no matter which way to generate OOK-4 is used, the complexity of gNB is expected to be the same. |
| Nokia/NSB | Y | There is no benefit of using N’ FFT (probably power of 2) with truncation compared to N (non-power of 2) in DFT process. However, we need to evaluate the performance of time domain waveform after truncation to see the distortion introduced. |
| Spreadtrum |  | It can be used for shaping of waveform at frequency domain? |
| FL2 |  | FL2-Higher-Proposal-1b For OOK-4, study the impact of truncation to generated waveform. |

In addition, the following new wave-form generating schemes were contributed:

- P1: Cconcentrated MC-ASK/FSK, transmitted OOK/FSK symbol is shortened from edges to improve robustness to multi-path [2].

- P2: Additional segments can be unmodulated or modulated with FSK [1].

- P3: FSK-3 could be defined as FSK-1 + OOK-4 [9]

- P4: FSK-4, FSK-1 can be used with OOK-4 to form M=2 [2]

- P5: FSK-5, IF segments without GBs are all modulated with sequence of good auto-correlation properties. Information is carried by number of segments used. Additionally linear phase can be applied to introduce PSK. [1]

FL1-Higher-Question-2:  Is it OK to add P1-P5 schemes to list of studied waveforms agreed in RAN1#112?

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Partial Y | - P1: we understand that it might be beneficial for robustness against timing errors and can improve SNR. So we suggest incorporating it as further details/note on how OOK pulses can be generated, not necessarily as a new scheme. - P2: this scheme is related to the 2 additional segments in RAN1#112 agreement on schemes, which might be more related to OOK-2, where OOK levels references can be inferred from frequency domain instead of time domain (using Manchester encoding). We suggest adding an additional note that the 2 segment can further be FSK modulated to carry one additional bit of information. - P3: In RAN1#112 agreement on FSK waveforms, the following note was present “Study how to generate segment in time domain, e.g. OOK-1 or OOK-4”, so we suggest adding the additional details on how this can be achieved. - P4: Our understanding is that this scheme is different than P3 and it entails some sort of coding over blocks of time and frequency within an OFDM symbol. It can be captured and further evaluated. - P5: this scheme can actually be used for both OOK and FSK. For OOK it can improve the performance over conventional OOK detection at DC. For FSK, it can eliminate the need for GBs between frequency segments and can enable detection with low complexity at BB without the need for multiple BPFs at IF for heterodyne receiver architectures. |
| EURECOM | Partial Y | We are OK with adding more schemes. However, the number of schemes to evaluate must remain reasonable. |
| Huawei, HiSilicon | Partially Y | We are OK to add P1, P3 and P4, as well as an additional one. For P2, we understand it is an OOK option rather than a FSK option, and it is already supported in the previous agreement by ‘[optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted]’. For P3 and P4, we understand both of them are aiming to carry additional information in time domain based on either FSK-1 or FSK-2, so they can be merged together as a single FSK-3: FSK-1/2 + OOK-4. For P5, the scheme is not clear to us and we want to first see some further clarification/explanation of this scheme. Beside the above P1-P5, in our contribution [2] we also suggested another FSK option in our observation 1 that for FSK-2 it can be decomposed into parallel channels with M=1 or M=2. To make it clear, we suggest the following description (similar as FSK-1): - Option FSK-X: N SCs of LP-WUS are separated to K sets of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a set of segments one segment is modulated, other segment is zero power (from base-band point of view) - where the number of segments in the i-th set is Li, and i=1Klog⁡(Li)=M Actually, the above method is a natural extension of FSK-1, i.e. from ‘pair’ to ‘set’ to allow more than two frequency segments in each parallel processing. We could reduce the number of FSK options by editing the definition of FSK-1, if preferred. However, for P1, we understand it may be better or additionally handled under the FFS: FFS modulated SCs are e.g. QAM symbols, sequences or other signals |
| Samsung | N | At this stage of study, we should focus on the alternatives already agreed (which are already many), and if essential issues are identified, we can consider further of the new proposals. |
| MTK | Partially Y | P1 is okay. To our understanding, P1 is adding guard time (like CP to OFDM) to improve robustness to multi-path. |
| ZTE, Sanechips |  | More details for each proposal should be clarified P1: at the receiver side, to avoid ISI by multipath, is it similar to drop some samples? Then there is no need to introduce new schemes for the waveform generation. P2: a diagram is needed to explain P2. how many additional segments are needed? How to implement ‘unmodulated or modulated with FSK’. P3: actually, besides FSK-1+OOK-4, there are different combinations can be considered, e.g., parallel OOK-4. we can separately discuss how many combinations can be considered. For P4 and P5, we hope more details including the diagrams should be clarified. |
| OPPO | N | We would like to minimize the solutions, as many of them is actually small alternation or combinations of other options. |
| vivo | N | First, we want to make sure the intention of this proposal. Does it intend to add more types of waveform generating schemes in addition to the existing schemes for OOK and FSK or add supplementary descriptions for the existing ones? - For P1, we consider it as kind of implementation for signal generation of MC-ASK/FSK covered by ‘signal generation and modification’ block, instead of a new waveform generating scheme. - For P2, not sure the relationship between P2 and the additional segments agreed in RAN1 #112 as shown below: [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - For P3 and P4, they have been already included in the agreement for FSK by generating segment in time domain via OOK-1 or OOK-4, new waveform generating schemes are not needed - For P5, for IF segments without GBs modulated with sequence, we wonder whether such IF operation requires new requirements for gNB implementation. |
| Apple |  | In general, we think we do not need to add an additional scheme for anything that falls into one of the current options. This is not saying that we don’t want to consider these schemes. We should be careful not to define a scheme for each small variation. In our understanding, - P1 falls under “FFS modulated SCs are e.g. QAM symbols, sequences or other signals” for OOK or “Study how to generate segment in time domain, e.g. OOK-1 or OOK-4” for FSK. - P2 is covered by “[optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted]” already? - P3 falls under “Study how to generate segment in time domain, e.g. OOK-1 or OOK-4” for FSK. - P4: is it somewhat similar to P3? - P5: we are not sure we understand the scheme. Would like a bit more clarification. |
| LGE | N | It would be better to further study/identify pros and cons for the agreed options in RAN1#112. After that, if needed, we can consider other options listed in the proposal. |
| Intel | Partial Y | For P1, the necessity depends on the number of OOK/FSK symbols per OFDMA symbol. We can discuss it later. For P2, we’d like to understand the relation between P2 and additional segment discussed in last RAN1 meeting. For P3 and P4, ok to add to address how to generate segment in time domain. For P3, if needed, we can add some details, e.g., 2 segment in frequency domain and each segment is modulated similar as OOK-4. For P5, more details would be helpful for better understanding. |
| Nokia/NSB | Y | We agree with FL. However, it should be noted that it incurs significant overhead on the evaluations in the next meeting. |
| QC1 QC2 |  | Having too many schemes could make it difficult to study. We recommend to limit the new schemes to add. Within the schemes P1-P5, we agree w/ Futurewei that P1 is beneficial for robustness against timing errors. |
| Spreadtrum |  | The enhancements or combination of waveforms/options can be further studied. |
| Futurewei2 |  | We would like to provide the following clarifications on P2 and P5: - P2: as already indicated by Huawei, vivo, Apple, and Intel, there is a relationship between this proposal and the agreement in the previous meeting ‘[optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted]’. - The difference is that we can add the capability that the 2 segments can further be FSK modulated to carry one additional bit of information. - The decision on the modulated bit from the two segments can then be used to invert the decision on each of the modulated bits for the other M segments in OOK-2. - Here is an example figure to illustrate the proposal - P5: as described earlier this proposal can apply to both OOK and FSK. - The main idea is that the LP-WUS is designed to generate a signal at a non-zero/DC frequency, i.e., low BB IF, after self-mixing envelope detection. Results in our contribution [R1-2302332] shows that this can improve the performance of OOK-1 by ~1.5dB. - For OOK, it can fall as further details under “FFS modulated SCs are e.g. QAM symbols, sequences or other signals” where the signal can be roughly described as follows: the N SCs of LP-WUS are separated into N0 segments of length L0 SCs each. - The number of segments N0=N/L0. - The segment length L0=IF0/SCS for a target low envelope IF of IF0. - Each segment is occupied by a sequence of length L0 and good autocorrelation properties . - For FSK, this scheme will not require explicit guard bands between segments and LP-WUS can be generated as described for OOK but for multiple IFn, n∈0, 1, …, 2M instead of a single IF0. |
| CTC |  | As the study on OOK/FSK waveform is still not completed, paying more attention on exist generation options is more reasonable. |
| FL2 |  | Thanks all for useful comments, I think we a forming a common understanding. FL is also concerned about amount of simulation overhead, even though own company does not provide results, per se. P1: It is understood that it falls under “signal generation and modification’ block” of OOK-4. I have added one sub-bullet to Proposal-1a P2: I suggest to remove [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] , indeed it is covers those 2 additional segments being FSK, which can aid time and frequency synch, etc. P3 and P4: Indeed it is covered by an example. - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol) Further there is small difference between P3 and P4, P4 codes time-frequency segments in OFDMA symbol jointly, while in P3, each OOK-4 time segment is treated separately. FL proposal would be P3 and P4 are added to #112 agreement, are optionally evaluated. P5: Companies please have a look at FW2 input above, and ask further clarifications if needed. P6: Actually, could we combine FSK1 and FSK2 to single scheme - Option FSK: N SCs of LP-WUS are separated to K sets of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a set of segments one segment is modulated, other segment is zero power (from base-band point of view) - where the number of segments in the i-th set is Li, and i=1Klog⁡(Li)=M Comment: in terms of evaluation overhead it does not help, since when M>2, number of simulation options grow. some values of Li’s could be down-selected. |

In addition, it has been pointed out that waveform generation should not require hardware change at Gnb [3]. Proposed has been also to use one sided LP-WUS instead of symmetric [1].

### Comparison of waveforms

#### Qualitative analysis waveforms

The following observation were contributed for the waveforms based on qualitative analysis.

OOK-2

- OOK-2 provides frequency diversity or more frequency resource [5]

- OOK-2 is more complex and thus not preferred [1][18]

- OOK-2 is acceptable. [10]

- OOK-2 has more complex receiver, parallel processing. [6]

OOK-3

- OOK-3 is sensitive to frequency offset [5]

- OOK-3 may violate RAN4 dynamic range requirement [5]

- OOK-3 is more complex and thus not preferred [1][18]

OOK1/4

- OOK-4 with M=4 and 8 is recommended [6]

- OOK-4/OOK1 recommended, because they are single segment, more segments need GB in-between [7]. OOK-4/OOK-1 are prioritized because OOK-1 is robust and OOK-4 offers flexibility in bitrate and frequency width [16]

- OOK-4/OOK-1 are simplest [18][21]

- OOK-1 is simplest for Gnb [27]

- OOK-1 is the simplest and highest priority [10]

- OOK-4 is the best in terms of system overhead, latency and low-power consumption [11]

- OOK-4 with same SCS is selected among OOK schemes [6], due to has good coexistence with NR and higher bit-rate.

- OOK over FSK due to complexity. [10][21]

- OOK-4+DFT is acceptable. [10]

FSK

- FSK-1 has issues with phase discontinuity, due to CP [27]

- FSK-1 also needs techniques to flatten time domain envelop. [27]

- FSK-2 is better than FSK-1 when M=2, after that use parallel FSK-1s.[2]

- With M=3 and more, FSK-2 eats too much BW. [2]

- FSK to show significant gain over OOK to be supported [17], increased complexity and power consumption.

- FSK receivers are nothing but parallel OOK receivers. Thus, the total power consumed by the LR may be doubled in most of the implementation. However, if FSK based LP-WUS signal uses same SCS as NR transmission, then it can be easily received and decoded if in case MR monitors LP-WUS under more stringent radio conditions when LR is not applicable. [12]

- FSK1/2 – Due to the narrowness of ON segment in FSK-1/2 schemes, they may suffer from fading, crystal stability of LR, and can also cause PAPR problems at the Gnb, if the total transmit power is allocated to a narrow section within LP-WUS BW. [12]

FL summary: When looking on above observations/proposals, majority companies state that OOK-1/4 is preferred for

- its simplicity of processing only single segment, and thus lower power consumption

- its robustness of OOK-1 and flexibility of bit-rate of OOK-4.

OOK-2 and OOK-3 are regarded as more complex schemes. OOK-3 may have issues with RAN4 requirements. OOK-2 and OOK-3 seems to be less attractive for the contributing companies.

FSK requires also parallel processing. With the fixed BW, segments are smaller and thus having less frequency diversity.

FL1-Higher-Proposal-2: Among agreed OOK schemes, prioritize further study of OOK1/4.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | N | We think that OOK2 should still be considered as it can provide a trade-off between robustness against timing errors lacked by OOK4, robustness against fading experienced by OOK1, and higher data rate provided by OOK4. |
| InterDigital | Y | We agree with the observation from the FL. |
| EURECOM | Partial Y | We are ok to keep OOK-2 for the moment. |
| Xiaomi | Y |  |
| Huawei, HiSilicon | N | We suggest to also prioritize OOK-2. Compared with OOK-4, OOK-2 is more robust to time error, frequency error and sampling rate reduction (see our simulation results in Figure 9, Figure 10 and Figure 11 of [2]). It is too early to exclude an option with good robustness. Another important benefit of OOK-2 is that with the parallel receiver architecture, UE is able to estimate and correct frequency error, while it is not clear how the architecture of OOK-1/4 can do that. |
| Samsung | Y | We are ok with the proposal. |
| MTK | Y | OOK-2 and OOK-3 are parallel structures of OOK-1/4. That means once companies make consensus on how to generate OOK-1/4, we can extend to OOK-2/3. We are supportive to prioritize OOK1/4 to facilitate the progress. |
| ZTE, Sanechips |  | Generally OK to prioritize the OOK-1 and 4. Additionally, when comparing with FSK or other waveform, OOK-2 also can be considered for fair comparison. |
| OPPO | Y | We are fine with that slightly focusing of options. |
| Sharp | Y | We support the proposal |
| vivo | Partially yes | OOK 1 and OOK 4 are two basic waveform generation methods for OOK, we agree to prioritize them for further study. Besides OOK 1 and OOK 4, OOK 2 provides multiple parallel transmissions for OOK, which should be also prioritized for study. FL1-Higher-Proposal-2: Among agreed OOK schemes, prioritize further study of OOK1/2/4. |
| SONY |  | OK |
| Apple | N | We think it is too early to down-select at this stage. All the options should be kept on the table for now, and the pros and cons will become clearer after more study. |
| LGE | Y | Agree with the proposal |
| NEC | Y | We agree to prioritize OOK1/4. |
| Ericsson1 |  | If prioritization for evaluations is needed at this stage, OOK-1 can be considered as baseline. The relative trade-offs for other schemes needs further study. |
| Intel | Y | Agree with the proposal |
| Nokia/NSB | Y | We agree with the FL proposal. OOK-2 uses partial LP-WUS BW for each segment, which widens the time domain waveform, however, suffers from frequency selective fading. OOK-3 suffers the most with frequency offset and frequency selectivity. OOK-3 as OFDM? |
| Lenovo | Yes | agree |
| DOCOMO | Y | Agree with the proposal |
| QC | Y | Agree with FL proposal |
| Spreadtrum | Y | OOK-2/3 is very like FSK since FDM of segments are used. They can be addressed in FSK. |
| CTC | Y | For option 1, one chip per symbol is the baseline solution, it should be preserved at this stage and the performance of any proposed schemes should be compared with such solution. For option 2, parallel OOK is a method to increase chip rate at the sacrifice of frequency resources. Smaller bandwidth is utilized for each chip and a guard band has to be inserted between each segment to avoid interference with each other. Spectrum efficiency and demodulation performance are both worse than option 4, hence not necessary to be preserved. For option 3, it can be regarded as a power and bandwidth saving option OOK-1 method since only part of the whole bandwidth is utilized, and demodulation performance is also degraded for the lower power, which is not comparable to option OOK-1. For option 4, it has both performance gains on both single-bit OOK options and multi-bit OOK options, should be proposed. |
| FL2 |  | OOK-2 seems still wanted to be studied by 3 companies. OOK-3 only proponent wants further study. It is also pointed out the receiver is more of simplified FFT receiver and scheme could be regarded more of an OFDMA waveform. FL2-Higher-Proposal-2a: For OOK schemes, prioritize further study of OOK1/2/4. OOK-3 can be still optionally studied. |

#### Quantitative analysis

FL Summary: I have extracted observations related to LLS in Section 6. Here is the summary of those.

- CP-OFDMA waveform

- O1: CP-OFDMA sequence receiver is shown to be tolerant to up to 10ppm frequency and 1us time error [2], while [5][13] shows degradation with 5ppm already. The performance of OFDMA sequence receiver is shown superior [2]. For CP-OFDMA with FFT or without, OFDMA receiver performs the same under ideal conditions [13].

- Drawback of multi-segment schemes

- O2: At fixed BW, multi-segment schemes suffer from loss of frequency selectivity due to reduced BW of segments [8][24][20][24]. Larger BW segment is also easier to deal with at gNB when it comes to PAPR [4]

OOK-4 is sensitive to timing error

- O3: OOK-4 is the most sensitive to timing error [2][20], but can be improved by concentrated waveform. OOK1,2,4, and FSK1,2 are shown to tolerate up to 4us.

- Sampling rate

- O4: When sampling rate reduction starts to approach LP-WUS BW, degradation of performance is observed [13][2]. Oversampling is essential for at least OOK [20]. Sampling should be comparable to effective BW of signal [24]

ADC dynamic range

- O5: ADC of 4bits [13][20] is sufficient. At least 4bits are needed [12].

Single company observations

- S1: New proposed FSK-5 [1] from sub-clause 3.1.1 is shown to be more spectral efficient than conventional FSK.

- S2: Sequence based LP-WUS performs better than coded message LP-WUS. [12]

- S3: For FSK, robustness to frequency offset is function of GBs used [2]

- S4: OOK with fixed threshold is more susceptible to frequency errors than FSK [3]

- S5: OOK-3 is not robust to frequency offset/error. [20]

- S6: OOK-1 Manchester may improve robustness to timing error [20]

- S7: ACI is shown to impact more OOK-4 than OOK-1 [20]

- S8:16bit sequence is shown to provide reliable synchronization performance [5]

- S9:LS and DFT has similar performance [27]

- S10: Multi-bit waveforms do not improve coverage, because performance degrades with increased number of bits. [27]

- FL1-Higher-Question-3: Do you agree to focus in RAN1#112b-e on discussing only observations which have been studied by at least two companies, i.e. O1-O5? Any important observation in LLS missed by moderator?

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | partial | We think that S1 is related to FL1-Higher-Question-2, so it would be appropriate to be discussed as part of the discussion on P5 in FL1-Higher-Question-2. |
| EURECOM | Y | We agree that O1-O5 should be discussed first. |
| Xiaomi | Y | Generally OK, but for “S6: OOK-1 Manchester may improve robustness to timing error [20]”, it seems lots of companies have considered Manchester coding, so it would be OK to also discuss it. |
| Huawei, HiSilicon | Y | We are fine to focus on the observations with more input first. But we have the following comments: - For O2, it is not reasonable to separately discuss the drawback of multi-segment scheme (i.e. narrower BW for each segment), since more energy can be concentrated to the segment with non-zero energy leading to boosting gain. We should review the final link performance instead of discussing only frequency diversity. - For O3, based on our simulation, OOK-4 with 30kHz SCS is only tolerant to 2us - For O4, we show OOK-1/2 and FSK-1/2 are more robust to sampling rate reduction than OOK-4. - Another important thing is that with the parallel receiver architecture (corresponding to OOK-2 and FSK-1/2), UE is able to estimate and correct frequency error. |
| Samsung | Y | First, we agree with moderator that we could focus more on the common observations from multiple sources, and come back to other observations later Then we want to clarify that the “Y” put in this comment meaning the agreement with moderator’s approach, and the wording and detailed observations of O1-O5 still need improvement. |
| MTK | Y | O1: The performance depends on T/F resources. For SSS-like WUS with one OFDM symbol, its performance degrades given 5ppm frequency error. O2: Okay. O3: It depends on the OOK duration. OOK-4 can support a shorter duration given 15/30kHz SCS. When the duration is shorter, it is sensitive to timing error. O4: Okay, it should not be less than Nyquist rate. O5: It depends on AGC is used or not. If AGC is used, 4-bit is sufficient. R4-2305641 |
| ZTE, Sanechips |  | According to the agreement, the following aspects also need to be considered for comparison: - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread Therefore, O1, O3, O4, O5 are based on the agreement, and can be discussed with higher priority. Besides, we would suggest, under each aspect, MC-ASK, MC-FSK, and CP-OFDMA can be analyzed separately. |
| OPPO | Y | We are OK with the O1/2/3/5, But wondering the O4 can bring any help to LP-WUS, and the sampling rate don’t need to be reduced. |
| Sharp | Y |  |
| vivo | Y | Agree to prioritize the aspects studied by more companies. |
| SONY |  | The observations supported by at least two companies don’t have observations from the other 25 companies, so there is hardly a quorum for making these observations in this meeting. For this meeting, maybe we should list the sorts of observations we would like to make in a future meeting. Companies would then know what to focus on at a future meeting. A possible list includes: - Sensitivity to timing and frequency error - Pros and cons of multiple-segment schemes - Sampling rate requirements ADC dynamic range requirements |
| Apple |  | We are generally fine to discuss the issues that have more input. Sony’s suggestion is also good in the sense that it provide a summary of directions that we need to look into. |
| LGE | Y | OK in general. Moreover, we may focus more on O2 and O3 for comparison between OOK/FSK waveform generation options. |
| NEC | Y | We are OK with the proposal. |
| Intel | Y | We share similar view with ZTE that O1-O5 is aligned with previous agreement. We agree to prioritize the discussion for O1-O5. |
| Nokia/NSB | Y | Focus currently with O1-O5 and if time permits, we’ll focus on S1-S10. |
| Lenovo | Y | We are fine with the proposal |
| DOCOMO | Y | We are OK with the proposal. |
| QC1 QC2 | Y | Agree with FL suggestion. Start with ones with multiple inputs and reach consensus. |
| Spreadtrum | Y |  |
| CTC | Y |  |
| FL2 |  | - In general, companies are OK with approach to look on aspects which has been evaluated by more companies. It has been pointed out that S6 , Manchester coding was simulated almost by everyone. - We agreed last meeting on studying, also S observations could be assigned to below bullets - impact of timing error (O3) - impact of frequency error (O1) - impact of phase noise and I/Q imbalance, if applicable () - impact of ADC resolution and sampling rate (O4 + O5) - impact of interference - impact of delay spread/frequency-selectivity (O2) - impact of doppler spread It is suggested that having above, we could focus study on answering more detailed questions, which could facilitate getting more focused results in next meeting: FL2-Higher-Proposal-2b: To focus simulation effort further, focus on answering the following: - For each waveform scheme assess sensitivity to timing and frequency error - How frequency error sensitivity is impacted by choice of guard-bands at the edges and between segments. - Study sensitivity of segment to frequency-selectivity as function of its size. - What are sampling rate requirements and ADC dynamic range requirements for each waveform scheme. - Performance with and without Manchester coding - Whether ACI and phase-noise impacts different waveforms differently - What size of guard-band is needed. - Does sequence based (Option 2) perform better or worse than message+CRC (Option 3) - Anything else? |

#### Waveform selection

Two companies contributed on how to select waveform for LP-WUS.

- Waveform selection should be decided based on receiver comparison [9].

- The modulation scheme used for LP-WUS must be chosen to reduce both power and latency of majority of UEs in the cell rather targeting a subset of UEs whose channel conditions are good. [12]

FL1-Higher-Question-4: Based on what criteria RAN1 should select waveform(s) for LP-WUS?

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei |  | We think that at least the following metrics should play a role in waveform selection: - Receiver power consumption - Latency and resource overhead - Coverage, i.e., required SNR at target MDR and FAR |
| InterDigital |  | We are fine with the proposed aspects from Futurewei, but prefer to include UE implementation complexity. |
| EURECOM |  | We agree with Futurewei and would also add capacity as a criteria. |
| Xiaomi |  | For the above two points, we are generally OK. But for the second point, it should be noted that, the modulation scheme may not be able to achieve whole cell coverage. |
| Huawei, HiSilicon |  | The following aspects should be considered: - The link level performance, including the robustness to non-ideal channel conditions (e.g. timing error, frequency error and etc) - The power consumption of the corresponding receiver, significant power saving gain is expected - The impact to gNB hardware, no new hardware should be required in our view |
| Samsung |  | Detection performance (FAR, MDR), resources allocated for LP-WUS, receiver energy consumption are the three most essential criteria for selecting the waveform. |
| MTK |  | We consider a waveform achieves low power and a reasonable coverage, potentially with a price of low data rate support. |
| ZTE, Sanechips |  | - Link level performance - UE’s power saving gain - UE complexity - gNB complexity - Unified waveform design |
| OPPO |  | We agree the power consumption and coverage/performance would be the most important ones |
| Sharp |  | We think following can be considered for waveform selection: 1. Capacity/bitrate in unit resource 2. Coverage 3. UE receiver complexity/power consumption |
| vivo |  | Currently RAN1 is studying different types of waveform including OOK, FSK, and CP-OFDM together with the corresponding receiver architecture, we can revisit this question after achieving more progress. |
| SONY |  | Combination of power consumption, latency and system overhead |
| Apple |  | This question seems to be very broad, and we think almost everything (e.g. the performance metrics agreed under AI 9.11.1 and the factors agreed under AI 9.11.2) that we discussed/agreed earlier should be considered for waveform selection. In addition, we have the following proposal in our contribution: Further study a harmonized WUS design that can be detected by both OOK-based receiver and sequence-based receiver, by defining the sequence used to generate the WUS signal. We would like to bring it to companies’ attention, and we think a harmonized design is worth considering, because it leaves some flexibility on UE implementation and different tradeoffs. |
| LGE |  | Fine with the formulation by Futurewei. In addition, above design criteria could also be applied for LP-SS waveform design. Thus, we suggest to add LP-SS in the proposal. |
| Intel |  | We are generally fine with the proposed criteria including power consumption and complexity as well as target coverage. |
| Nokia/NSB | Y | We agree with both the options listed above. In addition, we recommend the following. - It should be easy to generate at the gNB without additional HW requirements. Selecting the coverage impacts the architecture and waveform selection for LP-WUS. However, we agree it is a trade-off between power consumption and latency. |
| Lenovo | Yes | Waveform should be generated using the current OFDM without any additional HW requirement at gNB. Power consumption, coverage, interference are criteria to select waveform |
| QC |  | Sensitivity, LP-WUR power consumption, receiver implementation complexity, system resource overhead, and impact to gNB, performance need to be considered. |
| Spreadtrum |  | Based on the KPIs we identified, e.g. PSG, system overhead, latency, coverage, mobility… |
| Panasonic |  | Reasonable receiver power consumption/complexity and coverage performance. |
| CTC |  | Coverage performance is the more important thing. |
| FL2 |  | Selection of waveform(s) should take into account. - Receiver power consumption and complexity - Latency and resource overhead - Coverage, i.e., required SNR at target MDR and FAR - Feasibility to generate waveform using existing hardware. - Capability to support flexible bitrate? - Capability to support different receiver structures? Few companies say it is too early to agree anything, surely FL agree, and we could stop here. Intension of FL is to encourage more than 2 companies to contribute on the aspect in the future, because this will be big headache of study item. |

#### CP-issue

OOK-4 CP may be LP-WUS symbol-long when M is large, how to deal with this [12][21][25]. Should padding or zeroing of last samples be applied.

Another issue is how to handle CP at the receiver when LP-WUS spans over multiple symbols [17]. However, UE may always discard CP, because it knows its location within LP-WUS

FL1-Lower-Proposal-3: Capture in TR: For OOK-4, when M is large, handling of CP may require enhancement.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| InterDigital | Y | Fine |
| Huawei, HiSilicon | N | We think it is too early to capture this in the TR. If UE can sync in time domain, UE can remove the CP by implementation. |
| Samsung |  | Understand the intention of the proposal. It would be more clear to directly state for which value of M, handling of CP may require enhancement. For example, from our perspective, handling CP is always helpful, even for M=2. “M is large” is not delivering accurate and complete information in the TR. |
| MTK | Y | Agree. UE needs to discard CP once it knows CP location. |
| OPPO | Y | That is necessary and would be solved with no much complexity. |
| SONY |  | An alternative would be that OOK-4 is only supported for small M |
| LGE | Y | Agree with the proposal. Moreover, CP handling also needs to be studied in case the different SCS is applied for LP-WUS or LP-WUS payload is carried across multiple OFDM symbols. |
| NEC | Y | We are OK with the proposal. |
| Intel | Y |  |
| Nokia/NSB | Y | Agree with the proposal. It would be better to clarify on M, i.e., when the ON duration is comparable to the CP duration as it can affect the detection. |
| Spreadtrum |  | Can be discussed |
| QC2 |  | Could the FL clarify the intention of the proposal? Is the intention to introduce new CP lengths for LP-WUS that are different from the per-OFDM symbol, normal CP and extended CP duration for NR? |
| FL2 |  | FL think that this should be reminder for us for WID (if any), and if OOK-4 is relevant. I would not like to spend more airtime on this one in SID. For link simulation, companies may assume whatever they If we cannot agree on below, then I would deprioritize discussion on this topic in future. FL2-Lower-Proposal-3: Capture in TR: For OOK-4 M>1, handling of CP may require enhancement. However, if UE has good timing synchronization it may discard CP at the receiver. |

### Different SCS between NR signals/channels and LP-WUS

Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB

Design aspects of mixed SCS between LP-WUS and NR signals:

- Different SCS between LP-WUS and other NR signals can be supported, in fact, in this case LP-WUS does not need to have SCS at all [8].

- Specification supports already FDM and TDM of different SCS [9]

- Support 15 kHz, 30 kHz, 60 kHz, and 120 kHz, for FR1 and for MC-ASK/ MC-FSK, and no extended CP needed [17]

- Guard band is needed between NR signals and LP-WUS with mixed SCS, but Guard band is need for LP-WUS anyway irrespective of SCS [9]

- The SCS of LP-WUS should be equal to or larger than the SCS of DL BWP. [16]

  - CP from later symbol could omitted to match NR signal

- GB is needed, but GB is need for LP-WUS anyway [9]

Why not to support mixed SCS between LP-WUS and NR signals:

- Spectral leakage is shown [12], it can be alleviated by pulse shaping but not removed.

- Number of SCS gNB needs to perform grows exponentially with increased SCS of LP-WUS to NR difference. [12]

- Mixed SCS increases complexity to UE as well, if MR would detect LP-WUS in RRC connected [12].

- Different SCS between WUS and NR signals causes large impacts to network [15]

- Larger GBs would be needed if LP-WUS is higher SCS than NR signals.

- There is significant gNB impact from mixed SCS support [27].

- gNBs do not currently support mixed SCS [9].

FL summary: Mixed numerology could be supported by specification, and existing FR1 and FR2 supported SCSs could be re-used for LP-WUS, on the other hand, complexity of gNB implementation is pointed out by 3 companies. One company points out that complexity may increase also for UE in some scenarios.

FL1-Higher-Proposal-4:  Capture in TR:

SCS of LP-WUS can be different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with. SCS of LP-WUS should be higher than SCS of NR transmissions and 30 kHz, 60 kHz, and 120 kHz SCS can be considered in FR1 for LP-WUS.  However, mixed SCS between NR transmissions and LP-WUS increases complexity at least at the gNB and makes this deployment scenario less attractive than the deployment scenario where SCS of a CP-OFDM symbol used for LP-WUS generation is the same as SCS used for other NR transmissions.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Partial Y | Our understanding is that current specification does not preclude gNB’s support of TDM/FDM of mixed numerology BWPs. In terms of gNB complexity, it is also our understanding that in order to support high data rate LP-WUS we have at least the following two options: (1) higher SCS, and (2) DFT. Both options may require additional components/modules, i.e., an additional IFFT for options (1) and an additional DFT for option (2), or additional storage to store corresponding time/frequency domain samples to specific combinations of OOK bits per NR signal’s OFDM symbol for both options. So, we suggest further discussions on the statement “However, mixed SCS between NR transmissions and LP-WUS increases complexity at least at the gNB and makes this deployment scenario less attractive than the deployment scenario where SCS of a CP-OFDM symbol used for LP-WUS generation is the same as SCS used for other NR transmissions.” |
| InterDigital | Y in principle | We are fine in principle but prefer to include more benefits and drawback. |
| Xiaomi | Y |  |
| Huawei, HiSilicon | N | As we replied for FL1-Higher-Question-4, we think one important criterion for LP-WUS design is to ensure that no new hardware is required for gNB. If different SCS are used for LP-WUS and other legacy NR signal/channel, separate IFFT is required for these two kinds of signals, which makes increases the complexity and may require new hardware. Regarding the comment on guard band, we think the size of guard band is different for same SCS and different SCS case. If same SCS is used, there is no interference from LP-WUS to legacy NR signal/channel, and the interference from legacy NR signal/channel to LP-WUS can be suppressed by the filter of LP-WUR. However, if different SCS is used, there will also be interference from LP-WUS to legacy NR signal/channel, and the UE receiving legacy NR signal/channel has no filter or processing to deal with such kind of interference. Therefore, a larger guard band is required which leads to larger resource overhead. |
| Samsung | Y in principle | We only see a need for studying different SCS for OOK-1, which should be clarified at the beginning of the proposal. We also has some wording change suggestion as follow: For OOK-1, SCS of LP-WUS can be different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with. When the SCS of LP-WUS is different, SCS of LP-WUS should be higher than SCS of NR transmissions and 30 kHz, 60 kHz, and 120 kHz SCS can be considered in FR1 for LP-WUS. However, mixed SCS between NR transmissions and LP-WUS increases complexity at least at the gNB and makes this deployment scenario less attractive than the deployment scenario where SCS of a CP-OFDM symbol used for LP-WUS generation is the same as SCS used for other NR transmissions. |
| MTK | Y but | If larger GBs are needed, then mixed SCS not only increases gNB complexity but lowers spectrum efficiency. |
| ZTE, Sanechip |  | - We agree that SCS of LP-WUS can be larger than (instead of ‘should be’) SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with. - The SCS already supported by NR transmission can be the potential candidate SCS for LP-WUS. - Whether mixed SCS deployment is attractive or not depends on many aspects, not only rely on the SCS. |
| OPPO | Yes |  |
| Sharp | Y | We are fine with the proposal |
| SONY |  | OK. Can we update the first sentence to: SCS of LP-WUS can be different to SCS used for other NR transmissions in CP-OFDM symbols overlapping in time with that overlap in time. |
| Apple |  | We think it is fair at least to capture such an option in the TR. Pros and cons can be discussed, and we can then conclude on whether to recommend such an option or not. |
| LGE | Partial Y | Fine with the proposal with the following change SCS of LP-WUS can be different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with. For SCS of LP-WUS, should be higher than SCS of NR transmissions and 15 kHz, 30 kHz, 60 kHz, and 120 kHz SCS can be considered in FR1 for LP-WUS. However, mixed SCS between NR transmissions and LP-WUS may increases complexity at least at the gNB and makes this deployment scenario less attractive than the deployment scenario where SCS of a CP-OFDM symbol used for LP-WUS generation is the same as SCS used for other NR transmissions. |
| Ericsson1 | No | Suggest alternate formulation as below If SCS of LP-WUS can be is different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with other NR transmissions, then SCS of LP-WUS should be higher than SCS of NR transmissions and 30 kHz, 60 kHz, and 120 kHz SCS can be considered in FR1 for LP-WUS. However, mixed SCS between NR transmissions and LP-WUS increases complexity at least at the gNB and makes this deployment scenario less attractive than the deployment scenario where SCS of a CP-OFDM symbol used for LP-WUS generation is the same as SCS used for other NR transmissions. We note that supporting different SCS between LP-WUS and other NR transmissions has much more increased complexity compared to generation of regular NR transmissions (where multiple IFFTs are not needed) |
| Intel | Partial Y | For mixed SCS, if LP-WUS and NR transmission is transmitted by FDM, it would increase complexity for gNB, but not the case for TDM. Regarding the complexity, we share similar view with Futurewei that both OOK-1 with higher SCS and OOK-4 increase complexity for gNB comparing to existing gNB, i.e., additional IFFT or DFT. Regarding whether attractive deployment, we share similar view ZTE that this may not only depend on SCS. |
| Nokia/NSB | N | We should first consider formulating it as an observation to capture it in the TR. It could be noted that we are trying to reduce the complexity of the LRs by taxing gNB for that. The solution must address the computational complexity of both gNB and LR. We must evaluate the mixed numerology and try to capture both benefit and drawback of OOK-1 scheme. |
| QC | Y | okay |
| Spreadtrum |  | Different SCS seems not friendly for gNB. If we can use the same SCS, why we still focus on different SCS? If OOK-1 can use different SCS to support more bits, different SCS can be configured by gNB. Since OOK-1/4 is to be prioritized, both different and same SCS canbe supported by spec, but left to gNB selecting. |
| CTC | N | Different SCS between LP-WUS and other NR transmissions has much network overhead, better to focus on the same SCS solutions first. |
| FL2 |  | As suggested by some companies, lets agree first set of observation, we can start from the following list. FL2-Higher-Proposal-4 Agree on the following observations. O1: If SCS of LP-WUS is different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with other NR transmissions, then SCS of LP-WUS should be higher than SCS of NR transmissions. O2: NR supports FDM and TDM multiplexing of signals/channels generated with different SCS. O3: The case of LP-WUS is different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with other NR transmissions will require larger guard-bands due to spectral leakage. Spectral leakage can be alleviated partially by pulse shaping performed at gNB. Larger guard-band will negatively impact spectral efficiency. Spectral leakage will negatively impact performance of legacy UEs. O4: Several companies observed that the case of LP-WUS is different to SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with other NR transmissions will increase complexity at gNB and may require new hardware. |

### TF resource and SNR

Agreement - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - raw information bit-size - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - FFS: false alarm probability/rate - FFS: misdetection probability/rate are kept [comparable or fixed]. Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

Two companies show that T-F resources can be fixed [2][23]. One company TF resources can be fixed only within sub-options of the same waveform, e.g. OOK [5]. Several companies used also repetition as one way to match T-F resources and bitrate between waveforms.

Two companies also discussed SNR.

- Average EPRE is fixed [5].

- Manchester with OOK, pulling between symbols can be done [5]

- OOK-4 having an issue with keeping average EPRE in-between the symbols [5].

- SNR definition of OOK

- average EPRE over REs / noise variance [13]

- use only white modelling of phase noise [13],

Companies point out that in some cases EPRE cannot be kept constant among symbols, however on average across simulation EPRE would become constant.

FL1-Higher-Proposal-5:  Update the above RAN1#112 agreement as the following:

  - [time/frequency resources (including any guard bands), if applicable]

  - [total energy of LP-WUS across the time/frequency resources]

  - average EPRE within the time/frequency resources

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are OK with proposal. |
| InterDigital | Y | Fine |
| EURECOM | Y | OK |
| Huawei, HiSilicon | Y | We want to first clarify the technical difference between ‘total energy of LP-WUS across the time/frequency resources’ and ‘average EPRE within the time/frequency resources’. If no difference, we prefer to use the agreed wording (i.e. total energy). If the intention is to consider the case that T/F resources cannot be aligned (though in our contribution [2] we justified they can be aligned), we can also live with the new wording (i.e. average EPRE). |
| Samsung | Y | OK with the proposal. |
| MTK | Y | Okay |
| ZTE, Sanechips |  | It is hard to keep time/frequency resources and EPRE are the same for different schemes in some cases. For fair comparison among different schemes, it is more easier to implement the following: - time/frequency resources (including any guard bands)*average EPRE within the time/frequency resources are the same |
| OPPO | Y | OK |
| Sharp | Y |  |
| vivo | N | We think the T-F resources is related to the waveform design and keep the same T-F resources is too restrictive to the design. While on the other hand, fixed FAR/MDR and provide the T-F resources is also reasonable comparison. In the end, the overhead corresponds to the T-F resources can be calculated and compared. Also, the required SINR is added when report the results. - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - Alt 1: - raw information bit-size - average EPRE within the time/frequency resources - time/frequency resources (including any guard bands), if applicable - Alt 2: - raw information bit-size - average EPRE within the time/frequency resources - false alarm probability/rate - misdetection probability/rate are kept [comparable or fixed]. Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - required SINR - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS. |
| Apple |  | It is unclear to us whether we can assume the power can be pulled across different OFDM symbols. RAN4 specs seem to define the dynamic range based on one OFDM symbol. At least it is something to check. |
| LGE | Y | Fine with the proposal. One clarification question: the intention of the proposal is the fixed T/F resources or comparable T/F resources? |
| Ericsson1 |  | Prefer to clarify – “average EPRE within the time/frequency resources used for LP-WUS” to align with earlier text. |
| Intel | Y | We are OK with the proposal. |
| Nokia/NSB | Y | Agree with FL. No need to have new definitions for SNR. It must be noted that if, in some schemes, due to fewer number of tones in frequency domain, if the power is boosted to compensate the guard gaps/bands in the ON segment, PAPR in time domain may cause serious issue in the fidelity of transmission at the gNB. How to determine RE in EPRE? |
| QC | Y | OK |
| Spreadtrum | Y | Fine to make it simple |
| Panasonic | Y | We are okay. |
| CTC | Y | Seems fine |
| FL2 |  | It is pointed out that pulling power across OFDM symbols is not allowed. This fluctuation of power (due to power pulling) in time domain may cause issued to gNB due to increased PAPR. However, we agreed the following bullet When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additioal hardware for WUS. Two companies are not OK with the proposal. For ZTE proposal FL feels that “time/frequency resources (including any guard bands)*average EPRE within the time/frequency resources are the same” it is hard to compare SNR. Therefore, better to go with VIVO suggestion for now. For companies who simulated, would be a lot of overhead to get also PAPR of output waveform when running simulations? FL1-Higher-Proposal-5: Update the above RAN1#112 agreement as the following: - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - Alt 1: - average EPRE within the time/frequency resources - time/frequency resources (including any guard bands), if applicable - Alt 2: - average EPRE within the time/frequency resources - Companies to report PAPR of generated waveform. |

### OFDMA signals

Three companies point out that they would like to hear more about the low power receiver consumption, before they can study.

- New low-power OFDMA based receiver needs power model before it can be compared with other waveforms (MC-ASK, MC-FSK) [8].

- Hold on study for OFDM-based existing signals/channels as LP-WUS until detailed break-down for the components of LP-WUR on power consumption reduction compared to the MR is provided [9].

- Power gain is unclear as well as performance for OFDMA low power receiver [10].

It is shown that performance of OFDMA based receiver is superior, but already with 5ppm frequency offset, performance is degraded.

- OFDMA receiver with ZC sequences is sensitive to frequency error [5]

- [11] having evaluation of ZC performance.  [FL comment : OFDMA receiver details are missing.]

- LP-WUS using sequence-based CP-OFDM achieves better coverage and performance than the coded transmission. Multiple sequence or sequence properties can used to increase the information rate [12].

- More observations on CP-OFDMA are in sub-clause 3.1.2.2.

FL1-Lower-Question-5: Is there a need for more agreements in other sub-agendas in order to compare CP-OFDMA signals waveform with MC-ASK/FSK waveforms?

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | N - can proceed in parallel | There are already some proposals for CP-OFDMA signals, we can focus on them first. Another thing we want to emphasize is that we agree with FL’s comment in 3.1.1 that ‘LP-WUR receiver could be optimized for a particular waveform generation and therefore it would be beneficial to specify the input signal of the DFT block’. In this way, some signal, based on which UE can perform correlation detection, can be used together with MC-ASK/FSK. |
| Samsung | N | We didn’t see a need for further agreement. |
| SONY | N | The power consumption of an LR for an OFDMA-based receiver can be studied in another agenda item. The power consumption of the LR is not necessary for link level simulation of the OOK and FSK waveforms, so why is it necessary for the link level simulation of an OFDMA-based waveform??? |
| Apple | N | The study on different signals can proceed in parallel under different sub-agendas. In fact, there is some dependency, so it is preferred to discuss in parallel. E.g. the architecture for OFDMA-based signal and the corresponding power consumption can depend on the assumed frequency offset. |
| Spreadtrum | N | Too large workload |
| FL2 |  | thread still open for further comments |

## Bandwidth and location

Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI

### BW

#### BW size preferences

- 5MHz with 1RB GB on each side

- 6-24RB@15kHz [6]

- max 5MHz [14]

- 6RB seems to be saturation point for performance [15]

- BW of LP-WUS is fixed as a number of RBs [17]

- <20MHz [26]

- larger BW has better performance and is more robust against fading [12][15]

- larger BW makes it easier for gNB to lower PAPR and facilitates less power hungry implementations. [12]

- BW 1MHz, 5MHz, and 20 MHz [24]

FL summary: 1MHz, 5MHz and 20MHz could be good candidates for the LP-WUS BW.

#### Flexibility and configurability

- flexible BW of LP-WUS increases the receiver complexity [5]

- flexible BW is good for forward compatibility and different coverage[7][17]

- narrow BW allows for more deployments and eases the coexistence with other signals [12][14][15]

- configurable BW 1MHz, 5MHz, and 20 MHz [24]

FL summary: BW size may be dependent on deployment scenario, but the large number configurable BW size increases the implementation complexity in LP-WUR.

FL1-Higher-Proposal-6 Capture in TR:

BW of LP-WUS is recommended to be configurable, to accommodate LP-WUS in different deployment scenarios. Number of BW configurations should be minimized, because they increase the number of filters to be implemented in LP-WUR.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| InterDigital | Y | Fine |
| EURECOM | Y | We agree, although it is our understanding that a device may not have to support all configurable LP-WUS BW sizes. |
| Xiaomi | Y |  |
| Huawei, HiSilicon | Y | We are OK with the proposal. And propose to add the following For FR1, the BW is recommended to be less than 20MHz. Based on our previous agreement Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI. FL1-Higher-Proposal-6 Capture in TR: BW of LP-WUS is recommended to be configurable, to accommodate LP-WUS in different deployment scenarios. Number of BW configurations should be minimized, because they increase the number of filters to be implemented in LP-WUR. BW of LP-WUS is recommended to be be less than 20MHz for FR1. |
| Samsung | N | Our position is not captured correctly, and fixed in the summary now. We want to clarify first what is the unit of BW here. Is it in MHz or a number of RBs. Also, we don’t think this proposal makes sense, since the coverage of LP-WUS can be adjusted based on transmission power, then fixed BW as a number of RB is sufficient (and save the receiver complexity at the same time). |
| MTK | N | Benefits from increasing BW can be obtained by using more time domain resources. For example, applying different configurations in the time domain can accommodate different deployment scenarios, e.g., 1/2 or 1/4 Manchester coding. WE don’t see a strong support for configurable BW. |
| ZTE, Sanechips | Y |  |
| OPPO | N | We’d better to have limited number of BW. If we can not conclude, RAN4 recommendation in the LSR1-2302287 could be the starting point. |
| Sharp | Y | OK |
| SONY |  | We think this requires further study. Deciding that the LP-WUS BW needs to be configurable is likely to have implications on the LP-WUS receiver, which would presumably also have to be configurable. |
| TCL | Y | Fine with the proposal |
| Apple |  | We feel it is too early to make such recommendation. Companies have been evaluating different options. However, so far we are not aware of any compelling reason that the BW needs to be configurable. |
| LGE | Y | Fine with the proposal |
| Ericsson1 |  | In general agree that LP_WUS with configurable BW should be supported. Prefer following formulation, e.g., - “Having configurable BW for LP-WUS increases the applicable deployment scenarios. Number of BW configurations should be minimized, because they increase the number of filters to be implemented in LP-WUR. ” |
| Intel | N | We prefer fixed size for LP-WUS for simple implementation, but difference caused by different SCS can be ok. As commented by other companies, different coverage may not require different BW, e.g., it can be achieved by different power, coding rate, etc. |
| Nokia/NSB | N | We prefer to start with the baseline evaluation using 5MHz, which is supported by majority of UE categories. From NW perspective, if LP-WUS BW can be configurable, it is beneficial, however, this imposes UE capability requirements that should be carefully studied. |
| QC |  | We think 5MHz could be prioritized than other options. |
| Spreadtrum | Y |  |
| Panasonic |  | To agree on several recommended BWs including 5MHz would be more important than the configurability itself for SI, considering in different frequency ranges and bands. Whether and how to support configurability can be address later or in the WI phase. |
| CTC | Y | Generally fine, and the BWP size can not configured too small to cause excessive performance degradation and affect coverage. |
| FL2 |  | It is pointed that 5MHz could fit all channel BW. Coverage can be improved by more transmit power, or increasing time domain resource. I wonder whether this is the minimum we can agree at this point, given many “No’s” for configurability. FL2-Higher-Proposal-6 At least for IDLE/Inactive mode - BW-size of LP-WUS is recommended to be less than 20MHz for FR1. - at least one BW-size <=5MHz is recommended to be supported [to accommodate LP-WUS within all NR supported channel BWs] |

#### Size of GBs

Little input has been contributed on the size of GB. Study requires phase noise model. This would be in competence of RAN4.

- GB 1MHz 2MHz is FFS [28]

- GBs are included in LP-WUS BW and are up to RAN4 [5]

- 5MHz with 1RB GB on each side

- [13] showed that 2RB GB would be sufficient for 10dB ACI

- [20] studied impact of ACI as well

FL1-Higher-Proposal-7: GB size of LP-WUS is up to RAN4.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| EURECOM | Y | RAN4 should have the last word on this. |
| Xiaomi | Partially Y | Additionally, GB is related to the signal design of LP WUS, and signal design should be decided by RAN 1. |
| Huawei, HiSilicon | N | We do not agree with this proposal. RAN1 can also study the GB size. For example, companies can provide simulation results for different GB size, and RAN1 can recommend proper values to RAN4 based on its own evaluations with reasonable performance loss or reasonable suppression performance of the assumed filters. RAN4 can also further study these values. |
| Samsung | Y | OK with the proposal. We need to send an LS to RAN4 about this, and clarify that GB is included in the LP-WUS BW as agreed. |
| MTK | Y | Agree. RAN4 has discussed both guard bands between carriers and between LPWUS and other NR transmission. |
| ZTE, Sanechips | Y | If RAN1 can converge, RAN4 can have a double check. Or just leave it to RAN4. |
| vivo | Y | We agree with the proposal |
| SONY | Y |  |
| TCL | Y |  |
| Apple | N | We are not sure if we can completely leave this decision to RAN4. We feel it could be a joint decision of RAN1 and RAN4 in the end. In fact, we are already considering the GB impact in the LLS, meaning that RAN1 is already studying it. |
| Ericsson1 |  | At least in SI phase, RAN1 evaluations should include need and details for guard-bands required for different waveforms. We prefer to not limit the discussion to just RAN4. |
| Intel | Y | OK to leave to RAN4 for final decision, while we also think RAN1 can study and evaluate the GB size. |
| Nokia/NSB | Y | Agree with FL. Just noting that RAN4 uses guard RB instead of guard band / guard gap. |
| QC |  | RAN1 can also further study whether/how much GB is necessary. Especially, reduce the impact of ACI, GB from the edge of system bandwidth could be necessary in placing the WUS within system bandwidth. |
| Panasonic | N | It should be worked out together with RAN4 and further to be addressed and refined in WI, if necessary. |
| CTC | Y |  |
| FL2 |  | Few companies not OK with the proposal, say RAN1 should study as well. FL adds GBs to LLS study FL2-Higher-Proposal-2b: |

### Placement

LP-WUS being flexible and configurable within carrier/BWP has good support [18][1][4][5][9][21][16]. Restricting LP-WUS to the middle of carrier/BWP causes hole in spectrum and is bad for coexistence with NR signals [4,18]. Edges are better for coexistence [4]. Dedicated BWP for LP-WUS is proposed [4], but at this stage it is not clear whether LP-WUS is configured within BWP or not. It should be preferably outside of initial DL BWP, as contributed in [18].

In addition, both FDD and TDD modes should be supported [6]. LP-WUS and MR carrier can be different and association between LP-WUS and MR carrier(s) is needed [6][13][17][26].  LP-WUS could be configured in GB of a band as well, if fits. Finally, LP-WUS CA does not need to be supported [6]

FL1-Higher-Proposal-8:  Capture in TR:

- LP-WUS is recommended to be configurable within carrier/BWP in a band.

  - band can be different than band of MR

  - band can be TDD or FDD band

  - FFS configurable within guard-band of a band (like NB-IoT)

- It is NOT recommended to support CA for LP-WUR

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Partial Y | Our understanding is that NB-IoT required in general a small BW (180kHz) as opposed to what might be required by LP-WUS (options discussed above include a minimum of 1 MHz). Therefore, we doubt the possibility to consider operation within the guard-band of a band. |
| EURECOM | Y | We are OK with the proposal |
| Xiaomi | Y | Just a little modification, - LP-WUS is recommended to be configurable within carrier/BWP in a band. - band can be the same or different than band of MR - band can be TDD or FDD band - FFS configurable within guard-band of a band (like NB-IoT) - It is NOT recommended to support CA for LP-WUR |
| Huawei, HiSilicon | Y/partial | OK with the fist bullet and its sub-bullets. From a proposal point of view, it is not defined what CA for LP-WUR means. We suggest removing or clarifying that bullet before forming views. |
| Samsung | Y in general | Clarify the wording for the first main bullet: “The frequency location of LP-WUS is recommended to be configurable within carrier/BWP in a band.” |
| MTK | Yes but | Okay to support flexibility within BWP. But it may not be appropriate to rule out CA in RRC CONNECTED at this stage. |
| ZTE, Sanechips | Y |  |
| OPPO | Y/partial | We would like to focus more on same band with MR as this can easily adapted to different band with MR. Could we remove the bullet in the moment? |
| SONY | Y |  |
| TCL | Y |  |
| Apple |  | We are fine with the 2nd bullet. But for the 1st bullet, we prefer to leave it to RAN4 to decide, because it is very much related to the interference issue. |
| LGE | Partial Y | We share the view with Xiaomi |
| Ericsson1 |  | Prefer to have FFS for the first sub-bullet of the first bullet. - LP-WUS is recommended to be configurable within carrier/BWP in a band. - FFS: band can be different than band of MR - band can be TDD or FDD band - FFS configurable within guard-band of a band (like NB-IoT) - It is NOT recommended to support CA for LP-WUR |
| Intel |  | For 1st sub-bullet, we think the band same as MR should be baseline, and FFS band different than MR. |
| Nokia/NSB | Y(P) | - LP-WUS location is recommended to be configurable within a carrier and further we can study the placement outside the carrier but within the band. We can study the guard-band implementation of LP-WUS but need not to capture in TR |
| QC |  | We suggest to prioritize the case of transmitting LP-WUS in the same band as MR. Supporting LP-WUS in different band as that of MR may require further feasibility study. |
| CTC | Y |  |
| FL2 |  | FL2-Higher-Proposal-8: - Capture in TR: The frequency location of LP-WUS is recommended to be configurable within carrier/BWP in a band. - band can be the same as band of MR - band can be TDD or FDD band - Study further - band can be different than band of MR - LP-WUS can be configurable within guard-band of a band (like NB-IoT) |

## RRM measurements

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE) Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

Offload RRM of serving cell to LP-WUR and relax neighbor cell in MR could be the way to go. Supported by [2][3][5][13][24][28].

Use of LP-SS is a majority view  [11] [2][23][5][7][15], stating that periodic signal is needed, because LP-WUS may not be always transmitted. In addition, the following comments we made regarding using LP WUS:

- not based on LP-WUS due to need for cell ID(10) causing overhead [8]. Cell ID is carried in LP-WUS according to [10].

- preamble (of LP-WUS) can be UE/group specific, this causes complexity of detection for UEs, if preamble changes with UE/group ID [4]

Two companies would like to use SSB as measurement signal [7] [27]

There are few options [13] for serving cell measurements.

- SSB measurement by MR is relaxed. no need for LP-SS.

- no SSB is monitored by MR when LP-SS is good quality.

- SSB measurement by MR is relaxed, but LP-SS is monitored when MR sleeps.

Neighbor cell is FFS [2][7][17]. If neighbor cell measurements are supported by LP-WUR, it would need to support also re-selection [7]. Large specification effort is expected if neighbor cell measurements by LP-WUR would be supported [7][8]. In addition, neighbor cell baring information, threshold information, would be required, resulting in MR wake-up to receive SSB and SIB [13], because carrying such info in LP-SS is hardly feasible [13].

Several companies show that RRM offloading is beneficial.

- 90% power saving benefit by offload [5]

- RRM power consumption is dominant at low paging rates and low FAR [23]

- Power consumption benefit seen [11][17]

- static MR can be put to eDRX, RRM frequency can be reduced [24]

Relaxing RRM measurement requirements only cannot be done. It will impact quality and latency of service, e.g. due to missed pages or not camping on the best available cell. Design targets/KPIs should be discussed.  Long measurement durations to ensure certain quality, will impact capability of UE to cope with changing conditions and increased overhead [12]. Therefore, UE-group specific relaxation could target static UEs [4].

Given above,

FL1-Higher-Proposal-9: In R18 SID, study further only offloading of RRM of serving cell to LP-WUR and relaxation of neighbor cell in MR

- periodic LP-SS is used for measurements.

- periodic LP-SS is cell dependent

- MR performs serving cell measurements.

  - Alt1: periodically, e.g. depending on configured eDRX cycle

  - Alt2: only when LP-SS based measurements are low

- FFS: signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are in general OK with the proposal bu suggest the following modifications FL1-Higher-Proposal-9: In R18 SID, study further only offloading of RRM of serving cell RRM measurements to LP-WUR and relaxation of neighbor cell in MR - periodic LP-SS is used for measurements. - periodic LP-SS is cell dependent specific - MR optionally performsing serving cell measurements. - Alt1: periodically, e.g. depending on configured eDRX cycle - Alt2: only when LP-SS based measurements are low - FFS: signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID - FFS: MR’s neighbor cell RRM measurement relaxation |
| Xiaomi | Partially Y | We have the following modification to state that neighbor cell measurement is also possible if LP-SS coverage allows, In R18 SID, study further only offloading of RRM of serving cell/neighbor cell to LP-WUR and relaxation of serving cell/neighbor cell in MR - periodic LP-SS is used for measurements. - periodic LP-SS is cell dependent - MR performs serving cell measurements. - Alt1: periodically, e.g. depending on configured eDRX cycle - Alt2: only when LP-SS based measurements are low - MR/LP-WUR performs neighbor cell measurements when serving cell measurement based on LP-SS or by MR is low. - FFS: signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID |
| Huawei, HiSilicon |  | It is not clear what “In Rel18 SID” means so better to remove it. We think more discussion on neighbor cell measurement is needed and this proposal can focus on serving cell measurements and their offloading to LP-WUR as a first step. Additionally, it is not clear why MR need to perform serving cell measurement periodically since they are offloaded to LR. Based on above, we suggest the following modifications: FL1-Higher-Proposal-9: In R18 SID, study further only offloading of RRM of serving cell to LP-WUR and relaxation of neighbor cell in MR - periodic LP-SS is used for measurements. - periodic LP-SS is cell dependent - MR performs serving cell measurements : - Alt1: periodically, e.g. depending on configured eDRX cycle - Alt2: only when LP-SS based measurements are low - FFS: signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID |
| Samsung |  | We have several comments: - First, whether to support neighboring cell measurement should be subject to RAN2 and RAN4, since all the specification work needed is within RAN2 and RAN4 scope - Then, seems we don’t have agreement on the wording “LP-SS” yet – whether we support it or not is still under discussion in our view, so maybe we can come back to this aspect later. - The third bullet may need more discussion. To us, the Alt 1 and Alt 2 may not be alternatives to each other. - FFS is not quite clear. The “signal to measure” is for LP-WUR or MR? |
| MTK | Yes but | - “and relaxation of neighbor cell in MR” is misleading. If no enhancement is expected for the relaxation of neighbor cell in MR, we do not mention it. - Alt1 is the legacy behaviour. Maybe we do not need it. - Alt2 is a condition to wake up MR. It can also be an indication carried by LPWUS that request MR to perform RRM. We prefer to discuss wake-up conditions separately. |
| ZTE, Sanechips |  | If the LP-WUR is used for serving cell measurement, the cell ID seems is not so necessary but can be further discussed. We can remove the second sub-bullet currently. For the Alt1, it is not sure why the periodicity depends on the eDRX cycle. To be more general, it can be e.g. depending on configured eDRX cycle. For both Alt1 and Alt2, we think they are different methods to achieve serving cell measurement offloading and should be kept. For Alt1, if understand correctly, it means the serving cell measurement may be relaxed to a larger/relaxed measurement period. During the measurement period, using LP-SS for measuring and MR would not wake-up until the relaxed measurement period ends. For alt2, the MR may wake-up when the LP-SS signal has the bad performance. |
| Sharp |  | the measurement for serving cell and neighbour cell can be separately considered in two proposals. For serving cell, we are ok with the first two bullets. |
| vivo | Partially Yes | For the main bullet, the relaxation of MR should include both serving and neighbor cell measurements, to allow MR entering ultra-deep sleep as much as possible. So suggest the following update In R18 SID, study further only offloading of RRM of serving cell to LP-WUR and relaxation of serving and neighbor cell measurements in MR For Alt 1, we think only relies MR to eDRX is restrict such as large latency. We are open to discuss other alternatives. The meaning of Alt 2 “only when LP-SS based measurements are low” is a bit unclear, clarification would be needed. - MR performs serving cell measurements. - Alt1: periodically with a more relaxed periodicity, e.g. depending on a large period with comparable period like of configured eDRX cycle - Alt2: only when LP-SS based measurements are low |
| SONY |  | Should the bullet “MR performs serving cell measurements” read “MR performs neighbour cell measurements”? That would seem to be consistent with the moderator’s summary. However, we think it is too early to conclude whether LR or MR should perform neighbour cell measurements. We suggest the following update: In R18 SID, study further only offloading of RRM of serving cell to LP-WUR and relaxation of FFS whether neighbor cell is measured in MR or LP-WUR - periodic LP-SS is used for measurements. - periodic LP-SS is cell dependent - FFS: whether MR or LP-WUR performs serving cell measurements. - Alt1: periodically, e.g. depending on configured eDRX cycle - Alt2: only when LP-SS based measurements are low - FFS: signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID |
| TCL |  | We share similar questions as raised by Huawei that why MR need to perform serving cell measurement periodically since it is already performed by LR. In our view, for RRM measurement performed by LR can be reported through MR. |
| Apple |  | We also prefer to keep neighbor cell measurement open at this point. We generally support Huawei’s update, except that “only” can be removed from the main bullet. |
| LGE | Generally Y | FW’s modifications are fine to us. We doubt relaxation of neighbor cell RRM measurement is feasible when considering the limited coverage of LP-WUS. However, if majority of companies have interests in relaxation of neighbor cell RRM measurement so we are okay with keeping it FFS. |
| NEC | Y | We are OK with the proposal |
| Ericsson1 |  | We suggest below modifications “LP-SS” should be replaced with “reference signal(s)” in first three bullets. Last bullet should be updated to “FFS: reference-signal(s) to measure and content, e.g. SSB, OOK/FSK sequence (LP-SS), cell ID |
| Intel | Partially Y | We think neighbour cell measurement should be FFS. We have concern on complexity, standard effort (involvement of RAN2 and RAN4), and accuracy for neighbour cell measurement based on LP-SS. For the MR bullet, for Alt 1, it would be sufficient to consider minimum interval/measurement period to perform measurement by MR rather than restrict the measurement to be exactly periodical. Besides, eDRX cycle is just an example, no need to restrict the minimum interval/measurement period same as eDRX cycle. |
| Nokia/NSB | Y | We agree in principle with FL proposal. However, we would however propose to clarify the main bullet so that we consider only IDLE/Inactive state (and some additional clarifications): study further only offloading of RRM of serving cell measurements in IDLE/Inactive mode to LP-WUR and relaxation of MR-based neighbor cell measurements in MR with following assumptions: In relation to the fall-back to MR, the Alt2 could be maybe reworded/simplified for example: Alt2: based on some threshold e.g. LP-SS measurement results only when LP-SS based measurements are low There are various metrics considered in next proposal. The FFS bullet is not very clear as the afore bullets seem to refer to LP-SS. For clarification, is the intention to consider measurements on e.g., SSB with the LR? |
| QC | Y | We are in general okay with the proposal. We agree the “MR performs serving cell measurement” can be done optionally. For the FFS part, we think the SSB can be removed since the use of LP-SS for measurement has been already included in the first sub-bullet. We would also like to add another FFS regarding LP-SS periodicity. - FFS: the periodicity of LP-SS signal to measure and content, e.g. SSB, OOK/FSK sequence, cell ID |
| Spreadtrum | Partial Y | Whether LP-WUR can support neighbor cell should be further studied. If LP-WUR can process PSS/SSS, why does it not process the neighbor cell PSS/SSS? |
| Panasonic |  | We are supportive on the main bullet. The concrete definition of LP-SS and whether it is periodic can still be FFS and open. |
| FL2 |  | - generalized to reference signal(s) - clarified alternative - further suggest to keep neighbor cell relaxation as FFS FL2-Higher-Proposal-9: Prioritize study on offloading of RRM measurements of serving cell to LP-WUR and relaxation of RRM measurements of serving cell in MR - periodic reference signal(s) is/are used for measurements. - periodic reference signal(s) is/are cell dependent. - MR performs serving cell measurements - Alt1: with relaxed periodicity. - Alt2: only when LP-SS based measurements are below threshold. - FFS: relaxation of neighbour cell measurements - FFS: reference signal(s) to measure, e.g. SSB, OOK/FSK sequence (LP-SS) - FFS: periodicity - FFS: content, e.g. cell ID |

Metrics

Proposed metric to RAN1#112b-e:

- LP-RSRP/LP-RSSI

  - proposed in [5][10][21], and it is shown in [24] that RSRP can meet RAN4 accuracy.

  - can be defined as the received power within the BW of a LP-WUS and within a pre-defined time duration [2][10]

- LP-RSRQ, LP-SINR

  - depends on whether receiver has ADC, see details in [5][23]

- LP-WUS/LP-SS detection rate [5][23]

- Energy detection [10]

Further observations are:

- Architecture and RF impairments will impact the feasibility of measurements. [12]

- Selection of waveform impacts selection of metric.

- For OOK, RSRP, RSRQ, SINR definition could be adapted. [3]

- For MC-FSK with OOK, RSRP, RSRQ, SINR definition could be also adapted.  [3]

- For OFDM sequence-based receiver, discuss whether RSRP, RSRQ, SINR could be reused. [3]

FL1-Higher-Proposal-10: For the purpose of s RRM offloading to LP-WUR, candidate metrics for serving cell measurement identified by RAN1 are

- LP-RSRP/LP-RSSI applicable to all agreed architectures

- LP-RSRQ, LP-SINR, which may require architecture with ADC

- LP-WUS/LP-SS detection rate applicable to all agreed architectures

- Energy detection applicable to all agreed architectures

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are OK with proposal. |
| EURECOM | Y | OK |
| Xiaomi | Y? | Just a clarification question for the third sub-bullet, what is actually “detection rate”? Is it the times of detection that UE is trying within a certain duration? |
| Huawei, HiSilicon | Y | We are generally fine with the first three sub-bullets. But the last sub-bullet (energy detection) is not clear to use, for which more clarification is needed. |
| Samsung |  | Our understanding this list is for study purpose, and further downselection would be needed (since some metrics have overlapping functionality). Is it correct? Also, we didn’t get the point on why only LP-RSRQ and LP-SINR require ADC while LP-RSRP and LP-RSSI are not (even after reading the reference [5][23]). By definition, RSRQ=RSRP/RSSI, then if the UE is able to measure RSRP and RSSI without ADC, then why not RSRQ? |
| MTK | Not sure | To clarify, LP-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry LPWUS, where the LPWUS includes LP-SS and LP-WUS but is modulated by the OOK waveform only? |
| ZTE, Sanechips |  | We need to further clarify how the RSRP is calculated for OOK signals, since the adjacent subcarriers are not available for interference cancellation. Also, the detection rate definition needs to be clarified. |
| OPPO | OK |  |
| vivo |  | For LP-RSRP and LP-RSSI, architecture with ADC is also required in our understanding. It is not clear to us, how these metrics can be obtained without digital processing with multi-bit ADC. Last bullet “Energy detection applicable to all agreed architectures” seems to have some overlap with LP-RSSI or detection rate, some consolidation would be needed. |
| SONY | Y |  |
| Apple |  | The 3rd and 4th bullets are unclear to us. Some clarification is needed. We are also a bit confused about the claim on ADC. We would think even for LP-RSRP/RSSI, ADC might be needed. Maybe we can leave this out for now. At the same time, we should also keep it open that we may reuse the existing signal (SSB) and the existing metrics. |
| LGE | Y | Fine with the proposal. |
| NEC | Y | We are OK with the proposal |
| Ericsson1 |  | Suggest following modifications For the purpose of s RRM offloading to LP-WUR, candidate metrics for serving cell measurement identified by RAN1 are - LP-RSRP/LP-RSSI applicable to all agreed architectures - LP-RSRQ, LP-SINR, which may require architecture with ADC - LP-WUS/LP-SS detection rate applicable to all agreed architectures - Energy detection applicable to all agreed architectures Note: LP-RSRP, LP-RSSI, LP-RSRQ, LP-SINR refer to RSRP/RSSI/RSRQ/RS-SINR measurements performed by LP-WUR. FFS: Feasibility of different receiver architectures to support the above metrics. |
| Intel |  | We share similar view with other companies that ADC is needed for RSRP/RSSI too. For energy detection, what is difference between this one and RSSI? |
| Nokia/NSB | Y | LP-SINR depends on the oversampling factor. Does LP-RSRP, LP-RSSI considers the ON and OFF duration of MC encoding separately to calculate the energy? In all these options, other than LP-SS detection rate, ADC is required, e.g., at least 4 bits. |
| QC1 QC2 |  | LP-WUS is not always transmitted, and the consideration on its detection rate does not make sense. We think LP-RSRQ/LP-RSSI needs an architecture with ADC. We would like to clarify the difference of energy detection to LP-RSSI in the first sub-bullet. |
| Spreadtrum | Y |  |
| CTC |  | It seems more like a RAN determination thing. |
| FL2 |  | I tried to clarify metrics, please suggest better wording /correct definitions please. FL1-Higher-Proposal-10: For at least RRM serving cell measurement based on reference signals(s) performed by LP-WUR, RAN1 identified the following metrics for . - LP-RSSI/Energy detection = linear average of received power - LP-RSPR linear average of received power of reference signal(s) - RSRQ=RSRP/RSSI - Detection rate of always ON periodic reference signals(s) FFS: Feasibility of different receiver architectures to support the above metrics. |

## Synchronisation of LP-WUR

12 companies support at least periodic synch signal LP-SS received by WUR [2][23][4][5][7](must be SSB) [8] [11](for duty cycle only) [13][14](only periodic)[16] [22] [24]

Companies that are in favor of periodic LP-SS state that periodic LP-SS

- minimizes the LP-WUR ON duration, as discussed above. [11]

- simplifies LP-WUS decoding as the LR will already be synchronized when it attempts to decode LP-WUS [11]

- can be used for course synch [18]

- support RRM measurements (such as RSRP and RSRQ-like measurements) [11] [2][23][5][7][15]

- can be monitored within periodic window [13]

- is essential if LP-WUR monitors with duty cycle [24]

OFDMA-based signals/channels (e.g. SSS/PBCH-DMRS) are preferred over MC-ASK/FSK signal by 3 companies [7][19][27], reason is the large overhead of MC-ASK/FSK to get the same precision as with OFDMA-based signals/channels. Other companies argue that periodicity could be low [2][23][24] overhead insignificant. [27] points out that periodicity of 1.28s/iDRX (being max allowed periodicity of measurement according to [7]) may not be sufficient for RRM measurements as at least two sample averaging needed by RAN4, also synch is needed for measurements, etc.

Several companies point out that supporting additional aperiodic synch signal

- can be used for residual time/frequency error [7][8][18]

- can be used for comparator purpose, beneficial for some of agreed architectures [12][13]

- can be separate from LP-WUS [13] transmitted or not within a periodic candidate location

- Preamble with 16chips could be enough to provide synch [24]

On the other hand, one company [14] states that aperiodic synch signal is redundant if periodic LP-SS is supported. Additionally, if aperiodic synch signal is UE/group specific, causes detection increased complexity for other UEs [4].

[24] summarizes also nicely pros and cons in a table.

|  | Option 1: LP-SS | Option 2: Preamble only | Option 3: Hybrid LP-SS + preamble |
| --- | --- | --- | --- |
| Overhead | Fixed overhead; Depends on LP-SS periodicity. With LP-SS periodicity of 1.28 sec, the overhead is quite low compared to SSB. This is more predictable to gNB resource scheduling point of view. | Variable overhead; Depends on the # of UEs paged. On average, if there is more than two or more UEs paged during one DRX cycle, then, the overhead gets larger than LP-SS. | Somewhere in between the (a) and (b). When there are large number of UEs in the cell, shared LP-SS could save resources significantly, while helping sync for each UE based on preamble. |
| RRM offloading | Periodic LP-SS could directly used for offloading RRM. | Since WUS may or may not be transmitted, it cannot be used for RRM offloading. | Periodic LP-SS could be directly used for offloading RRM to LP-WUR. |
| Power consumption | Additional power consumption could be limited by duty cycled monitoring of LP-SS. | LP-WUR should continuously monitor WUS to detect preamble, which increases power consumption. | In between (a) and (b) |
| WUS monitoring scheme | Fit to duty cycled WUS monitoring | Fit to continuous WUS monitoring | Fit to duty cycled WUS monitoring |

Regarding the waveform used for time-frequency synchronization:

Time synch is easy based on MC-ASK, however how to do frequency synch [2]? Here FSK seems better [2]. [21] then points out that waveform could be different for LP-SS and LP-WUS. And [19][7] prefer SSB to be used as LP-SS.

Regarding the structure:

LP-SS signal structure alternative could be as shown in [5]:

- Alt 1: sequence(s) [11][13]

- Alt 2: sequence(s) followed by additional message [13]

- Alt 2a: additional message is carried by sequence(s) selection

- Alt 2b: additional message is carried by encoded bits ([with CRC]) [11][13]

Also it has been contributed that:

- Additional content may be included in LP-SS, e.g. SI update, ETWS [5]

- Information on time from LR to MR can reduce synch effort and power consumption at MR [2].

FL1-Higher-Proposal-11: If LP-WUR monitoring with duty cycle is supported, it is recommended that at least periodic LP-SS is supported. Study further

- required periodicity of LP-SS.

- waveform used for LP-SS, and whether it should be the same as used for LP-WUS

- structure of LP-SS

  - Alt1: sequence based only

  - Alt2: sequence + message with encoded bits

- supporting additionally aperiodic synch signal for fine synchronisation

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are OK with proposal |
| EURECOM | Y | OK |
| Xiaomi | Y |  |
| Huawei, HiSilicon | Y with comment | We are generally fine with the proposal with the following comments - For the 2nd sub-bullets, we’d like to clarify if different waveforms are used for LP-SS and LP-WUS, it is expected that the same receiver architecture can support to receive both. - We’d like to clarify that the receiver architecture of OOK-2 (i.e. parallel receiver) can also perform frequency error estimation/correction, while the architecture of OOK-1/4 need further discussion. This aspect should also be captured in the proposal. So we suggest the following FL1-Higher-Proposal-11: If LP-WUR monitoring with duty cycle is supported, it is recommended that at least periodic LP-SS is supported. Study further - required periodicity of LP-SS. - waveform used for LP-SS, and whether it should be the same as used for LP-WUS - structure of LP-SS - Alt1: sequence based only - Alt2: sequence + message with encoded bits - supporting additionally aperiodic synch signal for fine synchronisation - The feasibility of time/frequency estimation/correction for different waveforms/receivers architecture |
| Samsung | Y in principle | We don’t think “If LP-WUR monitoring with duty cycle is supported,” is needed. Also, we should at least clarify what LP-SS is short for, since this is first time it comes to an agreement (if agreed). |
| MTK | Yes but | If duty cycle and RRM offload are to be supported, it is recommended to consider monitoring PSS/SSS via LPWUR. This prevent introducing a new broadcast signal that has a tradeoff between NW spectrum efficiency and LPWUR sync. requirement. The monitoring occasion can be longer than 20ms and it can support multiple combining once cell ID detection is detected. |
| ZTE, Sanechips |  | If the following transmission are also viewed as the periodic signal We can further modify the wording as : required periodicity or pattern based on LP-SS, since periodicity of LP-SS seems preclude these cases. Also, we would suggest, how the periodic signal for sync is defined should be clarified firstly. At least there are 3 interpretations for the periodic signal for sync - The LP-SS is transmitted with a periodicity - The LP-SS is transmitted in a window wherein the window is with periodicity - The LP-SS is transmitted with a pattern wherein the pattern is with a periodicity |
| OPPO | OK |  |
| Sharp | Y |  |
| SONY | Y |  |
| TCL | Y |  |
| Apple |  | It is not clear what “sequence” means in Alt 1 and Alt 2. Does it mean multiple bits? Or it means something like CAZAC sequence? Regardless, we think the legacy SSB and CAZAC sequence-like signal should also be considered as candidates for SS. |
| LGE | Y | Fine with the proposal. |
| NEC | Y | We are OK with the proposal. |
| Ericsson1 |  | We suggest following updates.. At least for LP-WUR architectures that cannot receive existing OFDMA-based NR signals for synchronization/measurements, If LP-WUR monitoring with duty cycle is supported, it is recommended that at least periodic LP-SS is required supported. Study further - required periodicity of LP-SS. - waveform used for LP-SS, and whether it should be the same as used for LP-WUS - structure of LP-SS - Alt1: sequence based only - Alt2: sequence + message with encoded bits - supporting additionally aperiodic synch signal for fine synchronisation |
| Intel | Y | We are fine with the proposal. |
| Nokia/NSB | Y | Agree with FL. It is better to consider what is the maximum ppm offset LP-SS should target. If majority of UEs with LR gets assistance from MR, can the initial offset be reduced in case of duty cycled approach? |
| DOCOMO |  | Fine with the Ericsson’s modification since some LR architecture may be able to do synchronization by SSB. |
| QC | Y | We are OK with the proposal with the following modifications. If LP-WUR monitoring with duty cycle is supported, it is recommended that at least periodic LP-SS is supported for synchronization. |
| Spreadtrum | Y |  |
| Panasonic |  | We are supportive to study the synchronization of LP-WUR. In addition, depending on the bandwidth and location of LP-WUS/LP-SS, legacy PSS/SSS may also be considered for further study, when possible. |
| CTC | Y | To avoid extra time shift of WUS due to less accurate of LP-SS, additionally aperiodic synch signal should be added. |
| FL2 |  | We agreed we study Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS Option 3: Option1 + Option2 @MTK concern has been addressed by Ericsson wording update? @ZTE added pattern @ Samsung, would Ericsson wording be acceptable to you? FL2-Higher-Proposal-11: At least for LP-WUR architectures that cannot receive existing OFDMA-based NR signals for synchronization/measurements, and if LP-WUR monitoring with duty cycle is supported, at least periodic synchronization signal used by LP-WUR (LP-SS) is required. Study further - required periodicity of LP-SS. - waveform used for LP-SS, and whether it should be the same as used for LP-WUS, assuming that receiver can receive both. - structure of LP-SS - Alt1: sequence based only, e.g. CAZAC, M-sequence - Alt2: sequence + message with encoded bits - supporting additionally aperiodic synch signal for fine synchronisation - feasibility of time/frequency estimation/correction for different waveforms/receivers architectures |

## Content of LP-WUS

Target ID is an obvious essential content.

- UE-group [1][2][8](for IDLE)[9](or subgroup) [11][12][13][14](for IDLE)[15](or UE ID)[17] ](or UE ID, and trigger to wake-up MR) [18][20][22] (also UE ID) [24](subgroup) [26](subgroup and UE ID)

UE ID is to be supported only in CONNECTED [8][13][18]. In IDLE/Inactive mode it caused excessive overhead. Reduction of 48bit UE ID field is proposed in [2][13]. However, it is shown that even reducing to 10bits, false wake up is still 10-3 [24].

Other information carried in LP-WUS could be :

- Control data (change of resource, change of periodicity) [20]

- Other data, e.g. for actuator [20]

- Tracking area/RAN area information [2]

- Cell information [1][2][6][10][11][17] [13](In IDLE mode)

- SI change and ETWS/CMAS information [2][9][10][13](study)[17]

Among other content, Cell information and SI change and ETWS/CMAS information have the highest support among contributions. One company does not want to include anything else than Target ID [22] to keep overhead low.

FL1-Higher-Proposal-12: Capture in TR:

For at least IDLE/Inactive mode

- it is recommanded to include information of UE-group ID as the information to which users the information (e.g. wake-up information) in LP-WUS is targeted. Inclusion of sub-groups or directly UE ID can be furhter considerd in the work item.

  - Note : Information can be explicit or implicit, e.g. by LP-WUS monitoring location

- it is for further study whether to include also Cell-inform and SI change and ETWS/CMAS information in LP-WUS

- FFS content in RRC connected mode

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are in general OK with the proposal. |
| InterDigital | Y in principle | Fine in principle, but the first sub-bullet is not clear enough to us. Some update as shown in the below is needed. - it is recommaended to include information of UE-group ID as the information to inform which users are targeted for the information (e.g. wake-up information) in LP-WUS is targeted. Inclusion of sub-groups or directly UE ID can be furhter considerd in the work item. |
| EURECOM | Partial Y | For IDLE: FFS LP-WUS related control information |
| Xiaomi | Y |  |
| Huawei, HiSilicon | N | In our view, actually it is not clear enough how to define UE ID or UE-group ID. Does UE ID mean the 48-bit 5G-s-TMSI? Is it possible to define per UE information with shorter length (e.g. 32-bit or 24-bit)? Before answering this question, it is too early to conclude that ‘per UE indication causes excessive overhead’. On the other hand, talking about only overhead without considering power saving gain is not proper. As shown in our simulation results (see R1-2302339), per-group indication leads to power saving gain reduction. Per our understanding, a possible way is that to let RAN2 discuss what is the bit-length range of different kinds of ID, then we can further discuss which kind(s) of ID is/are supported. |
| Samsung | Y | OK with the proposal. |
| MTK | Y but | The note misleads LPWUR should monitor multiple occasions to detect information explicitly or implicitly transmitted by gNB. However, that is not the intention for this proposal, which considers TDD for UE grouping. To prevent misleading, we suggest removing the note. |
| ZTE, Sanechips |  | Recommendation can be done at the end of the SI. At current stage, we can say UE group ID or UE ID based information can be considered to avoid the confusion. It is not clear how Information can be explicit or implicit by LP-WUS monitoring location. Also this issue can be discussed in WI stage and it is not preferred to be captured currently. SI change and ETWS/CMAS information in LP-WUS should be prioritized to be considered, which helps UE and gNB has the same understanding for the current system configuration. |
| OPPO | Y | OK with the proposal. |
| Sharp | Y |  |
| vivo |  | For RRC connected mode, it has been evaluated by multiple proponent companies that LP-WUS is used to control the UE PDCCH monitoring, therefore suggest the following revision: For RRC CONNECTED mode, it is recommended to include at least information to wake-up UE PDCCH monitoring, with details FFS. |
| SONY | Y | OK with the outline of the agreement. - At the end of the first bullet, can we change “furhter considerd“ to “further considered” In the second bullet, what does “Cell-inform” relate to? |
| Apple |  | It is unclear what a UE-group ID (or a sub-group) is and what payload we are considering here. Without this understanding, it is not very useful to have such an agreement. |
| LGE | Y | The first bullet seems not clear. Fine with the InterDigital’s modification. |
| NEC | Y | We are OK with the proposal. |
| Ericsson1 |  | Our preference is to focus on small payload for LP-WUS. Given this relates to Idle mode operation, RAN2 inputs should be considered before discussing recommendations. |
| Intel | Y | We are generally fine with the proposal. In addition to UE group ID, sub-group ID can also be considered. |
| Nokia/NSB | N | Using 4 bits per CP-OFDM symbol for OOK/FSK may cost more symbols/slots to indicate UE ID(s). It would be better to study and evaluate the feasibility of having UE ID in the LP-WUS payload during the study item and make a conclusion on it rather leaving it for work item. |
| DOCOMO | Y | We are OK with the proposal. |
| QC | Y | We are in general OK with the proposal with the following modifications. FL1-Higher-Proposal-12: Capture in TR: For at least IDLE/Inactive mode - it is recommanded to include information of paging UE-group ID as the information to which users the information (e.g. wake-up information) in LP-WUS is targeted. Inclusion of sub-groups or directly UE ID can be furhter considerd in the work item. - Note : Information can be explicit or implicit, e.g. by LP-WUS monitoring location - it is for further study whether to include also Cell-information and SI change and ETWS/CMAS information in LP-WUS - FFS content in RRC connected mode |
| Panasonic |  | For now, we are supportive to LP-WUS carrying Cell ID, paging indication and optionally UE group-ID, in IDLE/Inactive mode. Other options are pre-mature to capture in the TR. |
| CTC | Y |  |
| FL2 |  | I tried to take comments into account by updating the proposal FL2-Higher-Proposal-12: For at least IDLE/Inactive mode, a LP-WUS includes at least information on which users are targeted by the LP-WUS. - Ask RAN2 what would be recommended information (if any) and its size. - including also Cell information and SI change and ETWS/CMAS information in LP-WUS may be strongly considered. - FFS content in RRC connected mode |

Coding/structure

LP-WUS signal structure options were proposed in [5]:

- Alt 1: WUS payload only

  - Alt 1a: WUS payload information is carried by sequence(s) selection

  - Alt 1b: WUS payload information is carried by encoded bits (with CRC)

- Alt 2: a preamble/delimiter/synch followed by WUS payload

  - Alt 2a: WUS payload information is carried by sequence(s) selection

  - Alt 2b: WUS payload information is carried by encoded bits (with CRC)

The following companies are interested in supporting message with payload [1][6][7][9][12][13][18]. Advantages are

- this allows content to be different for different use-cases and deployments

- has better transmission efficiency than sequence [18]

One company points out that message-only would need some info on where data start [13]

CRC for message is mentioned in [1][6][12](/FCS)[13][19]. The main motivator is to reduce FAR.

Preamble/delimiter (known signal) is mentioned in [1][7][9]

Spreading codes in [11] include interesting design analysis when correlating both Group ID sequence and cell ID sequence.

Synch-field separate from preamble is shown in [12]

- could be group-ID specific [12]

- it is better to get AGC/threshold settled, before performing synch

- re-use PSS as synch signal by LP-WUS, correlation signal is real and pre-processed by MR [19]

Manchester [6],[7](or codes<=1/2)[9][16][20]. Block codes [2][20] in general have the following advantages:

  - The power of the OOK symbols is independent of the payload [20]

  - Performance coding gain/coverage improvement since R<1 [20]

  - Simplified detection, no threshold detection, only comparison [20]

  - aids threshold estimation [2][12]

  - clock synch for OOK [20], but that can done from synch signal [if present] [12]

Repetition codes are used in [9][20](time domain cyclic shift repetition with cover code)

Selection may depend on payload to transmit, but preamble may be needed due to bad quality clock [5]

Based on above:

FL1-Higher-Proposal-13: Capture in TR:

- LP-WUS information can be carried:

  - Alt 1: by sequence(s) selection

    - spreading codes can be used to improve link performance

  - Alt 2: by encoded bits with CRC/FCS for at least FAR reduction

    - payload can be encoded using Manchester encoding or block codes in general can be used to improve link performance.

      - Manchester encoded LP-WUS may not need additional signal for detection threshold estimation in case of OOK

      - Manchester encoded LP-WUS may not need additional signal for time synchronization.

  - Repetition can be used to improve link performance.

- LP-WUS can be preceded by known [FFS one or more] sequence(s). The following functionality can be provided by known sequences(s).

  - start of LP-WUS.

  - fine synchronization.

  - threshold estimation.

  - AGC settling.

  - information, e.g. control information.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are in general OK with the proposal |
| EURECOM | Y | Generally OK with the proposal. Not sure what is meant by “Manchester encoded LP-WUS may not need additional signal for time synchronization”, our simulations show that Manchester coding improves resilience to timing errors but good synchronization is still required (especially for OOK-4). |
| Xiaomi | Partially Y | For LP WUS preceded by known sequence, what is “start of sequence”？from our understanding, if it is a time domain sequence occupying multiple OFDM symbols， the starting time of the sequence should be clear since it is “known”. But if the LP WUS is message-based, that is the sequence for time domain is not known, then some preamble is needed to cope with the ambiguity if LP WUS starts with off- waveform. |
| Huawei, HiSilicon |  | It is too early to conclude on coding, since the coverage performance is not well discussed yet. Beside block coding, other coding schemes with simple decoding complexity can also be considered. To make progress, we can first discuss the main bullets: FL1-Higher-Proposal-13: Capture in TR: - LP-WUS information can be carried: - Alt 1: by sequence(s) selection - spreading codes can be used to improve link performance - Alt 2: by encoded bits with CRC/FCS for at least FAR reduction - payload can be encoded using Manchester encoding or block codes or other codes in general can be used to improve link performance. - Manchester encoded LP-WUS may not need additional signal for detection threshold estimation in case of OOK - Manchester encoded LP-WUS may not need additional signal for time synchronization. - Repetition can be used to improve link performance. The intention of the second bullet is not clear to us. Is this the preamble? If so, it should be discussed under 3.4. |
| Samsung |  | For the first bullet, the last sub-bullet should be FFS. We already have ways (e.g. block codes) to improve the link performance, then the need of repetition should be FFS. For the second bullet, the whole bullet should be FFS, since all the listed functions may not be essentially required until further study reveals LP-WUS with the first bullet cannot work well. |
| MTK | Y but | “Manchester encoded LP-WUS may not need additional signal for time synchronization.” It is not possible. It is true that Manchester code can be decoded by a clock, but the clock must be synchronized with the transmitter. Also, the start of frame must be detected to know whether it is 01 or 10. |
| ZTE, Sanechips |  | We suggest to list several options for carrying the information, instead of describing the details. For example, LP-WUS information can be carried via sequences, by encoded bits. For functionality provided by known sequences, the following modification is suggested: - Start or end of LP-WUS. - fine synchronization. - measurement - threshold estimation. - AGC settling. - information, e.g. control information. |
| vivo |  | In the 2nd bullet, is the know sequences supposed to be the same as LP-SS? |
| SONY | Y |  |
| TCL | Y |  |
| Apple |  | Again, we would like to clarify the meaning of sequence here, whether it means multiple bits or something like CAZAC sequence. Overall we think it can be premature to make conclusion already. We would like more clarification on Alt 1, and how it is related to different waveform options that we have agreed. Alt 2 includes many detailed aspects that we haven’t got chance to discuss yet. |
| LGE |  | For the 1st main bullet, we have the same view with Huawei that Alt 1 and Alt 2 without sub bullets are fine to support. Regarding the 2nd main bullet, we are fine in general. But, more clarifications mentioned by companies should be clarified first. |
| NEC | Y | We are OK with the proposal. |
| Ericsson1 |  | Prefer following updates to first bullet - LP-WUS information can be carried: - Alt 1: by sequence(s) selection - spreading codes can be used to improve link performance - Alt 2: by encoded bits with CRC/FCS for at least FAR reduction - payload can be encoded using Manchester encoding or block codes in general can be used to improve link performance. - Manchester encoded LP-WUS may not need additional signal for detection threshold estimation in case of OOK - Manchester encoded LP-WUS may not need additional signal for time synchronization. - Repetition can be used to improve link performance. |
| Intel | Y | We are fine with the proposal, except ‘start of sequence’. We share similar view with Xiaomi that it be longs to part of time sync. |
| Nokia/NSB | N | Is the intention being to recommend supporting two LP-WUS designs in terms of information encoding? Should we aim to down select from the alternatives at the end of this SI to simplify the LP-WUS design? |
| DOCOMO |  | We have same view as HW that it is too early to conclude on coding, so we support HW’s modification. |
| QC1 QC2 |  | In general OK. For the sub-bullet in Alt 1: the term “Spreading code” may have specific meanings in the context Rel-16 NOMA, where 1 bit is repeated M times with a spreading factor M. While this is a valid sequence generation scheme, it appears to us to be too specific, and it doesn’t seem to capture the intention of [11], where the “spreading code” was proposed. Indeed, in [11], the following was explained “The address spaces of these two sequences are typically much smaller than the M and L bits (i.e., the length of the sequences) in the sequences, providing a degree of spreading that reduces error probabilities” The intention was not to use a spreading code, but rather to use sequences to convey information where the length of the sequence is larger than the #info bits conveyed by the sequence. In addition, “Alt 1: by sequence selection” could also have Manchester encoding for individual OOK symbol. Therefore, we suggest to modify the subbullet as - in case the sequence is a binary sequence, the number of sequences can be smaller than 2^L, where L is the length of the sequence. - Manchester code may be used together with the (binary) sequence. |
| Spreadtrum | Y |  |
| FL2 |  | Updated based on comments received, very much stripped down FL2-Higher-Proposal-13: Capture in TR: - LP-WUS information can be carried: - Alt 1: by sequence(s) selection - FFS sequence type - Alt 2: by encoded bits - FFS: encoding - [FFS]: CRC/FCS can be included for FAR reduction - FFS: need for repetition to improve link performance. - FFS: LP-WUS need to be preceded by known [FFS one or more] sequence(s). The following functionality can be provided by known sequences(s). - fine synchronization. - threshold estimation. - AGC settling. - information, e.g. control information . |

## Coverage

Comparable with PDCCH/PUSCH [3][18][21], but [21] points out that it may come at large cost at which it can be achieved. [24](PDCCH). Whether and how to live with smaller coverage should be studied [21]. It is partially studied in activation procedures. Coverage of LP-WUS should be that of PSS, otherwise network topology will change [6]

Study techniques to improve coverage [2] techniques are listed in [2]. Reliability of reception of LP-WUS should be measured due to coverage being worse than in MR, study how to [11]. Determine the coverage target, and only after discuss whether coverage enhancement is needed [14].

FL summary: Companies have very diverse view on what should be target coverage. Coverage can match the coverage of NR, but it seems that overhead needed for that would be large or payload of LP-WUS would need to be very small. Coverage enhancement techniques could be studied, but those typically also result in increased overhead or complexity. On the other hand, if coverage is not matched, specification effort will be need to specify procedures on how to deal with unbalanced coverage between LP-WUS and e.g. Paging PDCCH.  Waiting for

FL1-Higher-Question-6: Should RAN1 (a) discuss how coverage of NR could be met, (b) accept that coverage is unbalanced and discuss how such unbalanced coverage between LP-WUS and NR could be handled, or (c) wait for .1 agenda to form conclusion on coverage first.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | Since currently companies’ views are quite diverse, we support “(c) wait for .1 agenda to form conclusion on coverage first”. |
| Huawei, HiSilicon | Comments | We suggest to first discuss this issue in evaluation AI 9.11.1. Based on the discussion of BW, it is possible that the BW can be configurable for LP-WUS, which results in different coverage. In this study, we can list different design targets of coverage and the prices to achieve them. If operators choose to deploy LPWUS with the coverage smaller than the radius of a cell considering overhead issue, some simple fallback mechanism can be considered in spec. |
| Samsung |  | We should discuss the performance evaluation first and then come back to the proposal. |
| MTK |  | (c) depends on LLS results and details are discussed here. We prefer both (a) and (b) considering potential support of RRM offload can wake up MR if needed. The need can be at the cell edge with low LP-RSRP or strong interference that LPWUR cannot handle at any point of the cell. |
| ZTE, Sanechips |  | Better than PUSCH could be acceptable as the starting point |
| OPPO |  | We should discuss a) first as this will impact the effectiveness of LP-WUS |
| SONY |  | Is this meant to be a down-selection exercise? We think that RAN1 needs to (1) discuss how coverage of NR can be met. If the coverage of NR cannot be met with an LP-WUS/LP-WUR combination, we should prioritise LP-WUS/LP-WUR combinations that can meet the coverage requirement. As a last resort, (2) RAN1 could consider how to handle unbalanced coverage between LP-WUS and NR. |
| Apple |  | We probably should discuss the evaluation results first. |
| LGE |  | In our view, both (a) and (b) should be discussed. Target coverage can be comparable to NR coverage, but for this case, unbalanced coverage should be considered according to the different LP-WUS configuration. No need to wait the conclusion from evaluation results. If some observations are available from AI .1, we can use them during the discussion. |
| Intel |  | We should discuss LLS evaluation first, so we support (c) |
| QC |  | RAN1 should discuss coverage target – with its necessity in use cases, implication on data rate, system overhead, etc. Once this is determined then (b) could be discussed. |
| CTC |  | a) can be discussed first. |
| FL2 |  | a) Sony, LGE, QC (target first), CTC b) LGE c) Xiaomi, HW, Samsung, MTK, Apple, Intel, Nokia Seems wait for 9.11.1 is majority |

## Multiplexing and resource allocation

Inter-cell interference

- Interference mitigation by muting would be of low complexity [1][9]

- Interference randomization among cells [3][9]

  - different base sequence or scrambling according to different cell ID [9]

  - signal modification can be used to mitigate inter-cell interference [5]. Some more explanation from [5] is needed to understand how

- Inter-cell interference if not handled may cause high FAR [12]

FL1-Lower-Proposal-14: Support of inter-cell interference mitigation is needed for LP-WUS. Study further at least the following mitigation techniques

- muting

- interference randomization

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | Agree. Inter-cell interference is a serious issue in mobile networks and agree that methods to mitigate its impact for LP-WUS is needed. |
| Samsung |  | We agree inter-cell interference issue should be addressed. Other than the listed methods, the issue can also be resolved by designing low cross-correlation sequences, if LP-SS/LP-WUS is sequence based. We could come back to this issue after the design of LP-SS/LP-WUS is more clear. |
| MTK | Y but | Agree the need, but the interference randomization requires LPWUS to carry cell ID information, which we have not agreed how and whether to support. |
| SONY | Y |  |
| TCL | Y |  |
| Intel | Y |  |
| Nokia/NSB | Y | We prefer (c). The coverage is the important factor that determines the benefit of having LP-WUS. |
| Spreadtrum |  | Some issues are coupled, i.e. coverage, neighbor-cell measurement. If coverage is small, MR should be open at cell edge. It can be fact that LR is only used at non-cell edge and does not support the mobility case. The use case of LP is very limited and PSG is not justified in real deployment. We don’t see the commercialization benefits. |
| Panasonic | Y | Agree. |
| QC2 |  | For inter-cell interferences, there can be different cases. Case 1: the non-serving cell(s) is transmitting legacy CP-OFDM signal other than LP-WUS Case 2: the non-serving cell(s) is transmitting LP-WUS. |
|  |  | Different mechanisms may be used to deal with interferences in Case 1 and Case 2. In our view, LP-WUS should at least support mitigation against Case 1 inter-cell interference. Whether/how to support mitigation against Case 2 inter-cell interference can be further studied, since it largely depends on the outcome of Proposal-13. |
| FL2 |  | FL1-Lower-Proposal-14: Support of inter-cell interference mitigation is needed for LP-WUS. Study further at least the following mitigation techniques - muting - interference randomization - low cross-correlation sequences, if LP-WUS is sequence-based - other options not precluded |

Intra-cell interference

- GB and pulse-shape filter should be used to mitigate intra-cell interference [8]

LP-WUS to LP-WUS

- TDM, FDM, CDM [2]

LP-WUS and NR

- TDM/FDM [5][6][8][9][16][17]

  - semi-static and dynamic [5]

  - beam-sweeping to be supported [16]

- Reuse of unused LP-WUS resource for NR should be allowed [1][4][5][9]

  - study how to reuse [9]

FL1-Lower-Proposal-15: TDM/FDM multiplexing of LP-WUS and NR signals and channels. Existing semi-static and dynamic PDSCH rate-matching can be utilized. It is to be further studied whether any additional enhancements are needed/beneficial to enable better reuse of unused LP-WUS resources by NR signals and channels.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | N More discussion is needed | We think this proposal combines (as the FL show just above) many issues which does not need to be combined. Additionally, separation between connected mode and Idle/inactive mode is needed for some parts of the proposals. For example, Existing semi-static and dynamic PDSCH rate-matching can be utilized for connected mode only?. The meaning of the first sentence is not clear. Hence, we do not support the proposal as it is. |
| Samsung |  | We could come back to this issue after the design of LP-SS/LP-WUS is more clear. |
| MTK | Yes but | Okay. But if LP-WUS is flexible and configurable within carrier/BWP, it is up to gNB implementation for TDM/FDM multiplexing. |
| TCL | Y |  |
| FL2 |  | Apologize, the proposal was not very clear. But perhaps these aspects are more of Work-item business anyway. Thread is stopped. |

## LP-WUS monitoring

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

LP-WUS duty cycle can be associated with iDRX/eDRx periodicity which is followed by MR [4][23][8]

Periodicity and offset can be configurable: [9](for continuous symbols or slots)[15](allow short cycles)[16][17][26][27]

It is pointed out that continuous monitoring is possible to achieve continuous monitoring by duty cycle[11].

Duty cycle LP-WUS monitoring [9][8][4]

- Pros:

  - reduces FAR and higher power saving gain [9][11][12][13][5][22][24][26][27]

  - enables time domain TDM between UE groups, while using same preamble. [21][22][26]

  - better sensitivity receivers with higner power consumption can be used in the filed [12] while still providing power benefit

- Cons:

  - less NW flexibility and larger latency [5][9][11][13][16][21]

  - need for re-synch due to drifting oscillator [8][13] [8][13] this requires LP-SS [13]

Continuous LP-WUS monitoring

- Pros:

  - more NW flexibility and small latency [5][9][11][13][16][21][26], but latency is anyway lower bounded by MR wake-up form ultra-deep sleep [27]

- Cons:

  - FAR is expected to be worse [9][11][12][13][5][22][24][26][27]

  - expected to have larger power consumption [23][4][8][11][12] [16][21][22][24][26][27]

RTC precision is the one determining wake-up for duty cycle [11][23]. Oscillator precision determines duty-cycle [22]. RTC inaccuracy will increase the LP-WUR ON duration and this will negatively impact LR power consumption [11]. Therefore LP-SS is needed.

FL summary: There is a large amount of companies which see strong benefits from supporting monitoring duty-cycle for LP-WUS. Duty-cycle may “by configuration” also support continuous monitoring. Upper bound of duty-cycle periodicity depends on periodicity of LP-WUS if supported and [LP-WUR] RTC clock accuracy.

FL1-Higher-Proposal-16: It is recommended to support configurable duty-cycled based LP-WUS monitoring. Configuration should support the case of continuous monitoring. Study further what should be upper bound of monitoring periodicity which is impacted at least by accuracy of RTC clocks and periodicity of LP-SS (if supported).

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are OK with proposal |
| EURECOM | Y | OK |
| Xiaomi | Y |  |
| Huawei, HiSilicon | Y | In our view, both continuous monitoring and duty-cycle monitoring can be supported. One thing we want to clarify is that if slot format is introduced for LP-WUS, the FAR for continuous monitoring can be reduced. |
| Samsung | Y | OK with the proposal. |
| MTK | N | Okay to support duty-cycled monitoring. But we have concerns on continuous monitoring without given any monitoring occasion. It is painful for UE to perform non-stopping blind detection, leading to high FAR and additional power consumption. We propose to deprioritize continuous monitoring without monitoring occasion. That means UE should know at least when and where gNB will transmit LPWUS before reception. |
| ZTE, Sanechips | Y | According to the simulation results, in some cases, continuous monitoring still has the power saving gain in idle/inactive mode. Moreover, to satisfy the latency requirement of XR traffic, continuous monitoring is necessary. |
| OPPO | Y | OK with the proposal |
| Sharp |  | we are not sure how to configure a duty-cycled monitoring to continuous monitoring, is it by setting cycle duration = 0? |
| SONY | Y |  |
| TCL | Y |  |
| Apple | Y in principle | We support duty-cycle based LP-WUS monitoring. However, for the proposal, the exact meaning of “Configuration should support the case of continuous monitoring.” is not clear to us. Do we intend to say something like “Configuration should support small periodicity which allows the LP WR to perform continuous monitoring”? The last sentence is not clear to us either. Why is the upper bound of the monitoring periodicity impacted by accuracy of RTC clocks and periodicity of LP-SS? |
| LGE | Y | Fine with the proposal. |
| NEC | Y | We are OK with the proposal. |
| Ericsson1 |  | We are ok with “It is recommended to support configurable duty-cycled based LP-WUS monitoring.”. Further study on feasibility/benefits of continuous monitoring is needed before it can be recommended. |
| Intel | Partially Y | We agree that the configuration of duty-cycle can cover continuous monitoring, but it’s up to gNB to decide whether to configure such duty-cycle to enable continuous monitoring. |
| QC |  | We agree to support duty cycled based monitoring. We don’t see the strong need for the configuration to support the case of continuous monitoring, which will significantly increase the FAR for LP-WUS monitoring. |
| Spreadtrum | Y |  |
| Panasonic |  | We are okay to start with periodic monitoring for Idle/Inactive mode. |
| CTC | Y |  |
| FL2 |  | Some concerns are raised against continuous monitoring FL2-Higher-Proposal-16: - It is recommended to support configurable duty-cycled based LP-WUS monitoring. - Study further what should be upper bound of monitoring periodicity [which is impacted at least by accuracy of RTC clocks and periodicity of LP-SS (if supported).] - Study further what should be lower bound of monitoring periodicity [to enable low latency of paging] |

## Procedures for MR upon wake-up from ultra-deep sleep

Proposal-13(in RAN1#112): Study further pros and cons of the following procedures of MR wake-up from ultra-deep sleep - Option 1: perform PO monitoring, and afterwards follow legacy procedures - Option 2: perform PEI monitoring, and afterwards follow legacy procedures - Option 3: transmit PRACH for initial access, and follow legacy procedures

Option 2: Perform PEI monitoring, and afterwards follow legacy procedures

Several companies state that LP-WUS should be rather an alternative to PEI.

- PEI increases power consumption, LP-WUS should be alternative to PEI [4][23][24]

- largest latency: wake-up latency + PEI monitoring latency + paging latency [5][8][21]

- Once sub-grouping indication is carried by LP-WUS, UE does not need to monitoring PEI. [5]

Option 3: transmit PRACH for initial access, and follow legacy procedures

This option has quite many challenges

- Plenty of spec impact and workload. [5]

- Increased overhead

  - PRACH option not because the content is too large, which would increase power consumption and degrade performance [9]

  - Required LP-WUS contents for PRACH: all paging message carried in paging DCI and paging PDSCH. SI update info etc. [5]

  - Full ID is needed [13][18][22][26]

- FAR / large-UE-groups would waste network resource [13][21][26].

If Option 3 is supported, one company states that with valid TA UE could start from receiving MSG2 [26].

It is generally accepted that Option 3 going directly to PRACH reduces latency and companies are interested to further study, but it is pointed out that this option comes at cost of overhead and more complicated LP-WUS design [12]. It is also pointed out that latency is anyway lower-bounded by MR wake-up form ultra-deep sleep [27]. Is suitable for continuous monitoring [21].

Several companies point out that having a dynamic PO, i.e. PO outside of regular paging frame would reduce latency [13][14][22]

Above options are function of latency, overhead and specification complexity of LP-WUS. [12]. Option 1 can be a good compromise solution [18]. Many companies state that Option 1 should be baseline [5][4][23][9][16](unified among DRX and eDRX)[24][27].

Given above,

FL1-Lower-Proposal-17: At least “Option 1: perform PO monitoring, and afterwards follow legacy procedures” is recommended to be supported.

- Study further whether LP-WUS should be an alternative to PEI, or configuration of PEI (Option 2) and LP-WUS can be configured together.

- To reduce latency of paging reception study further whether

  - Alt 1: to recommend support of dynamic PO, i.e. PO outside of regular paging frame.

  - Alt 2: to recommend support of “Option 3: transmit PRACH for initial access, and follow legacy procedures.”

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are in general OK with the proposal, but would like to indicate that if SI change notification is included as part of the LP-WUS content, then the MR can receive SI upon wake-up due to LP-WUS without the need for PO monitoring. |
| Huawei, HiSilicon | N | We think this issue is related to the content of LP-WUS. For example, if per-UE indication is carried in LP-WUS, there is no need to monitor PO. So we need to wait for more progress on the content before making further conclusion on the options. But we are OK to have some progress on the detailed design for option 1. Therefore, we suggest the following: FL1-Lower-Proposal-17: At leastFor “Option 1: perform PO monitoring, and afterwards follow legacy procedures” is recommended to be supported. - Study further whether LP-WUS should be an alternative to PEI, or configuration of PEI (Option 2) and LP-WUS can be configured together. - To reduce latency of paging reception study further whether - Alt 1: to recommend support of dynamic PO, i.e. PO outside of regular paging frame. - Alt 2: to recommend support of “Option 3: transmit PRACH for initial access, and follow legacy procedures.” |
| Samsung |  | OK with the proposal. |
| MTK | Yes But | PEI can carry up to 43 bits configured by payloadSizeDCI-2-7-r17. To be an alternative to PEI, LPWUS may need to carry the same number of bits. Overhead will be an issue. Option 2 will require the minimum payload size, and the increased latency on MR can be ignored comparing to its long sleep, e.g., eDRX of 61.44s. |
| Sharp | Y | We are OK with the proposal, we believe that the legacy PO monitoring should be basically supported. |
| SONY | - | LP_WUS should be alternative to PEI, otherwise the wake-up time will increase. For Alt2, we need to be able to address UEs individually. |
| TCL | Y | We are fine with the intention of this proposal. |
| LGE | Y | Fine with the proposal. |
| NEC | Y | We are OK with the proposal. |
| DOCOMO | Y | Fine with the proposal. |
| QC |  | We think the proposal may be overlapped with FL1-Higher-Proposal-12. If LP-WUS carries UE-group ID information, then UE will perform PO monitoring after MR is woken up. The “configuration of PEI and LP-WUS can be configured together” in the first sub-bullet is ambiguities. If the activation/deactivation LP-WUS is configured vis SIB, then from the NW point of view, PEI and LP-WUS can be configured together for different UEs. But from the UE point of view, it needs to discuss whether to support both. If direct UE ID is not carried by LP-WUS, then using Option 3 to reduce latency is not preferred since it may reduce power saving gain and cause too much overhead for preamble transmission |
| Panasonic | Y |  |
| FL2 |  | Continue collecting input |

## Activation/Deactivation procedures of LP-WUS

Proposal-14(in RAN1#112): Study further the following LP-WUS monitoring activation and deactivation procedures - Option 1: LP-WUS is activated/deactivated by gNB - e.g. SIB or Dedicated RRC - Option 2: LP-WUS activated/deactivated implicitly based on criteria - Option 3: LP-WUS activated by UE autonomously

One company states Option 2/3 is not feasible [4],  company most likely assumed that gNB transmits LP-WUS or paging. However, gNB can transmit paging and LP-WUS at the same time.

Option2/3 may be useful if LR coverage not fully matched to MR. [18][21]. Opiton2/3 is supported or OK to be studied by [5][9][13][22] [18][21]

One company points out that MR should not turn-off before LP-WUR is in synch. [19]

Large among of companies see Option 1 as baseline option to be supported [4][8][9][13] [22] (based on UE report). In any case, some form of configuration of LP-WUS is needed first.

Given above,

FL1-Lower-Proposal-18: At least Option 1: LP-WUS is activated/deactivated by gNB is recommended to be supported, e.g. SIB or Dedicated RRC

- FFS based on UE report.

- Study further whether to support also the following options.

  - Option 2: LP-WUS activated/deactivated implicitly based on criteria.

    - FFS: need for informing gNB

  - Option 3: LP-WUS activated by UE autonomously.

    - FFS: need for informing gNB

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Partial Y | We are in general OK with the proposal but have the following comments/clarifications: - For “FFS based on UE report”, is this specific to LP-WUS reception in RRC connected state? - Our understanding is that Option 2 and 3 are the same unless by Option 3 we are referring to a UE satisfying a criteria (i.e., ability to receive LP-WUS and gNB’s support) but decides not to activate LP-WUS monitoring, in which case, this can be up to UE implementation and informing gNB can be done if needed. Therefore, we suggest to only keep Option 2. |
| Huawei, HiSilicon | N | Clarification on the meaning of ‘activated/deactivated’ is needed. And How SIB can be used for activation/deactivation? What information to put in the SIB? Is it just informing that WUS is available in the cell and UE can camp on it or not? Without further clarification is hard to support or not at this phase. For example, for IDLE/INACTIVE mode, LP-WUS can be configured by gNB via SIB, but this does not mean that UE MUST monitor LP-WUS when it is camping in the current cell. UE has the flexibility to choose to use MR for paging monitoring when channel state is not good. Hence, we suggest to further discuss this issue. And maybe sub-divided the discussion to - sub-Issue 1: LP-WUS monitoring activation/deactivation in connected mode - sub-Issue 2: LP-WUS monitoring activation/deactivation in Idle/inactive mode |
| Samsung |  | Ok with the proposal in general. We want to clarify the first bullet (first FFS). We don’t quite understand the wording “based on UE report”. In our understanding, there are two things worthy further discussion: 1) whether the UE report feedback as confirmation of whether the activation/deactivation signal is received, 2) whether the UE can request LP-WUS to be activated/deactivated. |
| MTK | Yes but | We are supportive but RAN2 has the same discussion with more detailed procedures. If we agree anything here, then it will impact RAN2’s decision. |
| SONY | Y |  |
| TCL |  | Regarding this proposal our view is not correctly observed. As mentioned in our TDoc that the SIB or dedicated RRC may be feasible for de-activating the LP-WUS monitoring but not feasible for activating the LP-WUS monitoring. This is because before LP-WUS monitoring is activated, the UE's MR is in ultra-deep sleep and cannot receive SIB or dedicated RRC signals. In addition, option 2 may not be feasible for the activation of LP-WUS monitoring, since the activation requires a trigger from gNB. For option 3, UE may not be able to activate the LP-WUS monitoring autonomously, since the UE do not know when the gNB will trigger the LP-WUR of UE to wake up the MR. Since the activation and de-activation of LP-WUS are two independent procedures. Therefore the methods of activation and activation of LP-WUS can be discussed independently. |
| LGE |  | We are generally okay with the proposal and suggest minor update. At least Option 1: LP-WUS is activated/deactivated by gNB is recommended to be supported, e.g. SIB or Dedicated RRC |
| NEC | Y | We are OK with the proposal. |
| DOCOMO |  | Generally OK, but prefer to LGE’s suggestion. |
| QC |  | We think the activation and deactivation of LP-WUS monitoring can be different. For example, the activation can be cell specific based on SIB, and the deactivation can be UE based following criteria. We suggest to discuss separately the activation and deactivation of LP-WUS monitoring. |
| FL2 |  | Continue collecting input |

## Connected mode LP-WUS

Unified LP-WUS design between IDLE and Connected Mode [2]. Different design for IDLE and Connected, because of different use-case [21].

BW

BW is the same as BW in IDLE due to reduction in LP-WUS implementation complexity [5][28]

Same or different BW between RRC_CONNECTED and RRC_IDLE/INACTIVE mode should be supported [8]. Difference since UE-specific coverage in Connected mode, contrary UE-group coverage in IDLE.

RLM/BFD/CSI

For LP-WUS monitoring in RRC connected mode, RLM/BFD/CSI measurements are performed by UE Main Radio (MR) and the legacy procedures is reused [5][13]. Reason is that it may not be feasible due to narrow-band nature of LP-WUS. Once LP-WUS is detected by LP-WUR, UE resumes PDCCH monitoring from micro/light/deep sleep of MR. During LP-WUS monitoring, UE MR is not expected to enter the ultra-deep sleep to avoid long wake up delay [5]. And finally [12] states: If CONNECTED mode operation with LP-WUS is considered, the link quality measurements and reporting need to be accounted for in evaluations. [12]

Functionality

Some companies think functionality is to replace DCP / or PDCCH skipping, i.e. to control monitoring [18][21]. Other companies think LP-WUS is envisioned to be interacting with existing PDCCH monitoring procedure [22], such as R16 DCP, R17 PDCCH skipping and R17 SSSG switching, C-DRX.  LP-WUS may contain information related to power saving mechanism, e.g. start of C-DRX [13]

Given above:

FL1-Higher-Proposal-19: For RRC connected mode LP-WUS,

- LP-WUS BW can be configured to be different from BW configured in IDLE/Inactive mode, set of configurable BWs size is the same for IDLE/Inactive and RRC connected modes.

- RLM/BFD/CSI measurements are performed by UE Main Radio (MR) as consequence ultra-deeps sleep state is not allowed for MR.

- Further study functionality of LP-WUS, e.g. whether LP-WUS is to replace existing PDCCH monitoring power saving features or LP-WUS is to enhance those features further in terms of latency and power saving.

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y | We are in general OK with proposal. |
| Xiaomi | Y |  |
| Huawei, HiSilicon |  | First, we support to discuss CONNECTED mode since we had agreement saying that both IDLE/INACTIVE mode and CONNECTED mode should be studied. For the BW, it is reasonable to have same set of candidate BW for IDLE/INACTIVE mode and CONNECTED mode, in order to reduce the work load. Beside the BW, some other design can also be shared to CONNECTED mode. For example, the modulation/waveform of LP-WUS should be the same to enable the same receiver architecture. For the same reason, LP-SS design can also be shared. For RLM/BFD/CSI measurement, we are OK to let them performed by MR. And per our understanding, MR cannot be in ultra-deep sleep not only due to the measurement, but also due to the latency requirement. For the functionality, we can further study the relationship between LP-WUS and legacy UE power saving techniques. Besides, some procedures of LP-WUS monitoring activation/deactivation (i.e. how to start/stop LP-WUS monitoring) should also be studied. |
| Samsung |  | We believe this proposal (especially the first bullet) needs further study. If ultra-deep sleep state is not applicable for connected mode, we want to see the gain first before drawing any conclusion (e.g., the first bullet). |
| MTK | N | We have the same concern to support configurable BW like the proposal for IDLE/Inactive mode. It increases hardware complexity, and the benefit is unclear to us at this stage. |
| ZTE, Sanechips |  | Obviously, in different deployment, the LP-WUS coverage performance is different. It does not make sense to use only one BW size for all the cases. For the first sub-bullet, Since the carried information may be different in idle mode or connected mode, it is not sure whether the set of configurable BWs size is the same. For example, the potential candidate BW size may be {10,8,6} PRBs, however, in connected mode, only {8,6} is used and in idle mode only {10,8} is used. Therefore, whether the set of configurable BWs size is the same or not does not need to be considered in SI stage. But we are OK with ‘LP-WUS BW can be configured to be different from BW configured in IDLE/Inactive mode’ |
| OPPO |  | We prefer to further discuss the issue. No conclusion in this stage. |
| vivo |  | For the second bullet, RRM should also be performed by MR for the third bullet, the study of the functionality of LP-WUS for RRC connected mode does not need to be restricted with the usage of the existing power saving features. Besides, companies also showed their views that LP-WUS can be interacting with the existing power saving features. Hence, the following revised for the proposal can be considered. - RLM/BFD/CSI/RRM measurements are performed by UE Main Radio (MR) as consequence ultra-deeps sleep state is not allowed for MR. - Further study functionality of LP-WUS, e.g. how LP-WUS is to replace existing PDCCH monitoring power saving features or LP-WUS is to interact with other UE power saving features and enhance those features further in terms of latency and/or power saving. |
| SONY | N | RRC_IDLE/INACTIVE should be prioritized |
| Apple |  | For the 1st sub-bullet, we are not sure if we should make the BW configurable. We are fine to have MR perform RLM/BFD/CSI measurement. The 3rd bullet probably should cover all the PHY layer procedure aspects? |
| LGE | Y | Fine with the proposal. |
| NEC | Y | We are OK with the proposal. |
| Intel |  | If LP-WUS for connected mode is considered, we prefer to reuse the design from idle/inactive mode as much as possible. The same BW is slightly preferred. |
| QC |  | We suggest to further study LP-WUS in connected mode. It is too early to conclude to support LP-WUS in connected mode. One modification: RRM can be done LP-WUR in connected (as done in Idle mode). |
| FL2 |  | BW seems controversial similarly as in IDLE . So lets continue discussion under IDLE. For reminder we have the following agreement Agreement Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI. - FFS: Further prioritization if needed during the study item. It would be good to prioritize discussion on what CONNECTED mode LP-WUS is good for first, what is its purpose of functionality FL1-Higher-Proposal-19: - If RRC connected mode LP-WUS would be supported: - RLM/BFD/CSI/RRM measurements are performed by UE Main Radio (MR). - Ultra-deep sleep state is not allowed for MR. - Study RRC connected mode LP-WUS functionality/purpose - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW |

## Other

T1: Implicit LP-WUS ACK sent by MR is sent [4]

- FFS: before signalling or after RRC signalling

T2: Study LP-WUS receiver autonomously determine the sampling rate, which can be beneficial from reducing power consumption perspective. [5]

T3: Define UE capability to inform about sensitivity [8]

T4: Capability of duty-cycle and contiguous? [13]

T5: Network should support LP-WUS waveform [8]

T6: How to deal with receiving two wake-up after each other, how to deal with missed wake-up [13][19]

T7: Study enhanced beam related procedures for supporting LP-WUS in FR2 should be studied.  [10]

T8: Prioritize discussion on FR1 [17]

FL1-Lower-Question-7: Any of above topics deserves to be prioritized in discussion in RAN1#112b-e?

| Company | Agree Y/N | Comments |
| --- | --- | --- |
| Samsung |  | At least T8 should be discussed since it’s a question from RAN4 LS. |
| MTK | Yes | T8: this can reply RAN4 LS (if 9.11.2 has no concluded) T1, T3 and T4 can leave to RAN2’s discussion. |
| SONY |  | T6: It should be possible for the system to control the rate of missed wake-up (whether by power control, reconfiguration or some other mechanism), so we should look at how to deal with missed wake-up. |
| TCL | Yes | T1 to T6 can be further studied. |
| FL2 |  | FL2-Higher-Proposal-20: Prioritize study of FR1 |

# References

- R1-2302332	LP-WUS Physical Signal Design	FUTUREWEI

- R1-2302341	Signal design and procedure for LP-WUS	Huawei, HiSilicon

- R1-2302392	Discussion on low power wake up signal design	Panasonic

- R1-2302409	L1 signal design and procedure for low power WUS	TCL Communication Ltd.

- R1-2302508	Discussion on physical signal and procedure for low power WUS	vivo

- R1-2302572	Design consideration on lower power wake-up signal and procedure	OPPO

- R1-2302622	Discussion on L1 signal design and procedure for low power WUS	Spreadtrum Communications

- R1-2302689	Physical layer signals/procedures and higher layer protocol for Low-Power WUR	CATT

- R1-2302817	Discussions on L1 signal design and procedure for LP-WUS	Intel Corporation

- R1-2302829	Discussion on L1 signal design and procedure for LP-WUS	InterDigital, Inc.

- R1-2302862	On L1 signal design and procedures for low power WUS	Sony

- R1-2302892	L1 signal design and procedures for low power WUS	Nokia, Nokia Shanghai Bell

- R1-2302950	LP-WUS design and related procedure	ZTE, Sanechips

- R1-2302969	Discussions on L1 signal design and procedure for low power WUS	xiaomi

- R1-2303033	Discussion on signal design and procedure for LP-WUS	China Telecom

- R1-2303061	L1 signal design and procedure for low-power WUS	Sharp

- R1-2303152	Signal design and procedure for LP-WUS	Samsung

- R1-2303255	Discussion on L1 signal design and procedure for LP-WUS	CMCC

- R1-2303334	L1 signal design and procedure for low power WUS	MediaTek Inc.

- R1-2303421	Discussion on L1 signal design and procedure for low power WUS	EURECOM

- R1-2303430	Discussion on L1 signal design and procedure for LP-WUS	LG Electronics

- R1-2303507	On the L1 signal design and procedures for low power wake-up signal	Apple

- R1-2303538	On LP-WUS signal design	Nordic Semiconductor ASA

- R1-2303614	L1 signal design and procedures for LP-WUR	Qualcomm Incorporated

- R1-2303673	Discussion on L1 signal design and procedure for LP-WUS	NEC

- R1-2303730	Discussion on L1 signal design and procedure for low power WUS	NTT DOCOMO, INC.

- R1-2303761	L1 signal design and procedure for low power WUS	Ericsson

- R1-2303808	Discussion on the L1 signal design and procedure for low power WUS	Lenovo

# Proposals from contributions

## R1-2302332_FUTUREWEI.docx

LP-WUS Design Considerations

Observation 1: The LP-WUS can be scheduled in any time-frequency resources within the configured BWP i.e., no dedicated time-frequency resources.

Proposal 1:  The LP-WUS should carry information indicating UE/group/cell ID or their possible combinations.

Proposal 2: For message-based LP-WUS, a CRC of proper length should be added to improve FAR and reduce its impact on power saving gain without degrading the preamble MDR.

Proposal 3: Separate and joint performance KPIs for preamble and message parts should be defined. For the preamble, at least miss-detection and false-alarm rate should be adopted as KPI and similarly decoding error rate for the message part.

LP-WUS Generation and Transmission

Observation 2: A minimum sampling frequency of ~8× the LP-WUS transmission rate of 56kbps is needed for DFT-S-OFDM OOK to achieve a similar MDR performance as the OFDM-OOK transmission scheme at a sampling frequency of only ~2× the LP-WUS transmission rate under the TDL-C 300ns channel.

Proposal 4: Study a dynamic design of LP-WUS with single-spectrum allocation to enable good and resource efficient in-band selectivity for RF/IF/BB envelope detection receiver architectures.

Observation 3: Out of M-bits LP-WUS generation Options OOK-2, FSK-1, and FSK-2, OOK-2 requires the least number of segments (M) but may require Manchester encoding, i.e., achieve half the information bit rate, for a comparable performance as the other two options.

Observation 4: For M>2, additional 2 segments for Option OOK-2 acting as ON/OFF levels reference may help achieve similar performance to Options FSK-1 and FSK-2 without information bit rate loss or significant increase in number of segments.

Observation 5: The additional 2 segments for Option OOK-2 can further be FSK modulated to carry one additional bit of information.

Proposal 5: Support the additional 2 segments for Option OOK-2 as ON/OFF levels reference with the following two alternatives:
Alt 1: the two segments carry known reference signals that are not modulated.
Alt 2: the two segments carry a signal that is FSK modulated.

Observation 6: The 2M frequency components of an M-bit FSK signal can be generated as low envelope IFs via the repetition of a sequence (of good autocorrelation properties and of length corresponding to the target IF) throughout the LP-WUS bandwidth.

Proposal 6: Study an Option FSK-3 for M-bit MC-FSK generation where the N SCs of LP-WUS are separated into Nn, n∈1, 2, …,2M, segments of length Ln SCs each.
- The number of segments Nn=N/Ln. 
- The segment length Ln=IFn/SCS for a target low envelope IF of IFn.
- Each segment is occupied by a sequence of length Ln and good autocorrelation properties.
- Linear phases may be applied to the N SCs to generate an M-bit PSK signal.

Link-Level Simulation Results

Observation 7: The performance of conventional OOK LP-WUR can be improve by ~2.5 dB by switching the LP-WUS bandwidth from Option 2 (2.88 MHz) to Option 1 (4.32 MHz).

Observation 8: Envelope IF signal generation/reception has a ~1.5 dB performance improvement over conventional OOK LP-WUR with the proper selection of IF.

Observation 9: Envelope IF signal generation/reception can support higher bit rate, i.e., using Option FSK-3,  than conventional OOK LP-WUR without a considerable impact on performance.

Proposal 7: Support a design of Option OOK-1 for 1-bit MC-ASK generation where the N SCs of LP-WUS are separated into N0 segments of length L0 SCs each.
- The number of segments N0=N/L0. 
- The segment length L0=IF0/SCS for a target low envelope IF of IF0.
- Each segment is occupied by a sequence of length L0 and good autocorrelation properties.

## R1-2302341_Huawei, HiSilicon.docx

- For the two options for FSK signal generation, the following observations are made

- For M=1, option FSK-1 and option FSK-2 are the same

- For M=2, option FSK-2 outperforms FSK-1 due to higher power boosting

- For M>2,

  - The segments required for option FSK-1 grows linearly with M.

  - For option FSK-2, the segments required grows exponentially with M. In this case, it is suggested to decompose the transmission into parallel channels with M=1 or M=2. With this method, segments required are the same as option FSK-1.

- A joint modulation of OOK-4 and FSK-2 can be more robust to timing error than OOK-4 due to its longer symbol duration.

- A joint modulation of OOK-4 and FSK-2 can have better frequency diversity than FSK-2.

- The parallel receiver architecture can be used by the joint modulation to enable both OOK and FSK.

- OOK and FSK detection can be made more robust with respect to interference and noise by modulating with sequences having good correlation properties. Sequences suitable for existing gNB transmitter, such as ZC sequences, have such properties.

- Options OOK-1 and OOK-2 experience marginal performance loss when ADC sampling rate is reduced to 960 kHz. Option OOK-4 sees 2 dB performance loss when sampling is reduced from 1.92MHz to 960kHz.

- For Option OOK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.5, 1.6 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 1.4 dB respectively.

- For Option OOK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.2, 0.4, 0.5 dB respectively.

- For Option OOK-4 with channel model of TDL-C 300ns, the performance loss caused by 1, 2 μs timing error are 2.5, 4.8 dB respectively. Timing error of 4 μs causes error floor. For channel model of TDL-C 1000ns, the performance loss caused by 1 μs timing error is 2.7 dB. Timing error of 2 μs causes error floor.

- If the guard bands can cover frequency offset, Options OOK-1, OOK-2 and OOK-4 are insensitive to frequency offset.

- ZC sequence-based generation of FSK/OOK waveform outperforms significantly the OOK modulation with rectangular waveform in fading channel.

- For OOK-4, concentrated OOK provides 0.5dB gain for ideal timing case compared with the normal sequence-based waveform because ISI between OOK symbols are mitigated.

- Concentrated OOK significantly improves robustness against timing error, e.g. at least 2dB improvement under 2us timing error.

- Concentrated waveform is applicable to OOK and FSK to improve their respective performance.

- Both FSK-1 and FSK-2 sees no performance loss when ADC sampling rate is reduced to 960 kHz.

- For Option FSK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 2 dB respectively.

- For Option FSK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.5, 1.7 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.6, 2.4 dB respectively.

- If the guard bands can cover frequency offset, Options FSK-1, FSK-2 are insensitive to frequency offset.

- For sequence-based detection, it can be observed that the performance would be degraded by 0.5dB due to the phase noise and I/Q imbalance when 120 uW oscillator is used.

- Sequence-based OOK/FSK modulation has better coverage performance than energy detection, even if phase noise and I/Q imbalance, frequency error within 10ppm and 1us timing error are considered.

- The target data rate for LP-WUS design can be x101~x102 kbps level.

- To indicate paging information by LP-WUS, there is trade-off between power saving gain/latency and required data rate/coverage performance.

- Defining slot format for LP-WUS is helpful to reduce the FAR, and thus increase the power saving gain.

- Parallel OOK/FSK receiver architecture is capable to do frequency error estimation and correction.

- The requirement of periodicity is usually dominated by the time error, where the residual frequency error contributes most of the time error.

- The resource overhead of periodic LP-SS is marginal for the following reasons: 1) based on the requirement of time/frequency estimation, the overhead can be low to ~0.01%, and 2) the overhead can be reduced by gNB configuration.

- A unified LP-WUS signal design for CONNECTED mode and IDLE/INACTIVE mode can avoid supporting two kinds of LP-WUS receiver architecture for different RRC states.

- Study joint modulation of OOK and FSK by e.g. multiple segments in time domain for FSK.

- Study carrying information via different sequences or cyclic shifts for the OFDM based signal/channel for LP-WUS.

- Study how to improve the coverage performance of LP-WUS.

- Study methods to extend coverage and reduce misdetection probability by block coding.

- For multiplexing of LP-WUSs for different UEs, study at least TDM, FDM, CDM.

- At least paging information is carried in LP-WUS, where LP-WUS can indicate per-UE information or per-group information.

- Further study how to indicate paging information to get a good balance between low data rate requirement and low false wakeup rate.

- The following information can be indicated by LP-WUS in addition to paging information:

  - Tracking area/RAN area information

  - Cell information

  - SI change and ETWS/CMAS information

- Study the slot format of LP-WUS.

- At least continuous monitoring of LP-WUS should be supported.

- At least support periodically transmitted LP-SS for LP-WUS/WUR.

- Further study the periodicity of LP-SS based on the requirement of max time error and/or frequency error.

- If there is time and frequency relationship between LP-WUR and MR, then LP-WUR can assist the re-sync procedure of MR, which can reduce the power consumption and latency.

- At least serving cell measurement is supported by LP-WUR using LP-WUS/LP-SS.

- Further discuss how to define and use the measurement quantities by LP-WUR.

- Further study how to support neighbor cell measurement.

- Further study how to define activation/deactivation conditions for monitoring LP-WUS.

- A unified LP-WUS signal design should be considered for CONNECTED mode and IDLE/INACTIVE mode.

## R1-2302392_Panasonic.docx

Proposal 1: LP-WUS needs to co-exist with other NR signals. The signal design should not mandate gNB hardware change.

Proposal 2: LP-WUS should not require re-planning of the cell deployment. LP-WUS coverage performance should be guaranteed in the existing deployment. Further discussion is needed on the coverage performance of LP-WUS should match to which bottleneck channel, e.g. PDCCH or PUSCH.

Proposal 3: Study some kind of cell specific randomization/scrambling of LP-WUS.

Proposal 4: For power saving gain in realistic operation, LP-WUR/WUS should be used for RRM measurement at least for serving cell.

Proposal 5: LP-WUR/WUS should support the functionality of time/frequency tracking to maintain serving cell quality measurement in both cell edge and center.

Proposal 6: RRC IDLE/INACTIVE has higher priority in the study.

Proposal 7: Target Oscillator max frequency error should be clearly defined and optimal design of filter passband and GB for it can be studied.

Observation 1: Both OOK-2 and FSK-1/2 based LP-WUS with small M can provide workable link performance under the used simulation assumption, at least with 24 PRB in 14 symbols .

Observation 2: When the value of M is high, in higher SNR, FSK-1/2 can provides better BLER performances than OOK-2 and FSK-1 has best performances among them because of the usage of relative comparison.

Observation 3: In higher SNR region, regardless of M , BER 1-to-0 of OOK-2 is better than that of FSK-1/2. This is because energy per 1 segment of OOK-2 is higher than FSK-1/2.

Observation 4: Regarding FSK-1/2, FSK-1 has better BLER/BER performance frequency usage efficiency than FSK-2 under the conditions of same transmit bits and LP-WUS bandwidth.

Observation 5: OOK with fixed threshold is more susceptible to frequency errors than FSK.

Observation 6: FSK-1 has more tolerance to frequency error than FSK-2 and OOK.

## R1-2302409_TCL Communication Ltd..docx

Observation 1: Aperiodic signal transmitted as part of LP-WUS may contains UE ID/UE group ID payload which could increase complexity and overhead if multiple UEs need to synchronize, and the synchronization may vary slightly between UEs.

Observation 2:  Periodic signal transmitted separately from LP-WUS reduces the overhead of synchronization signals and allows all the UE’s LP-WURs to receive the same synchronization signals.

Observation 3: Duty cycle based monitoring of LP-WUR for LP-WUS detection may encounter a mismatch between the LP-WUR ON duration and the periodic LP-WUS or configured LP-WUS in a specific occasion.

Observation 4: The monitoring behavior of LP-WUR for LP-WUS detection greatly impact the power consumption of LP-WUR, and the activation/de-activation of LP-WUS monitoring can avoid continues monitoring, and unnecessary decoding of each LP-WUS.

Observation 5: The following options may not be feasible for LP-WUS activation.

- Before the LP-WUS activation the UE's MR is in ultra-deep sleep and cannot receive SIB or dedicated RRC signals.

- LP-WUS activated implicitly based on criteria is not feasible, since the activation of LP-WUS monitoring may requires a trigger from gNB

- UE may not be able to activate the LP-WUS monitoring autonomously, since the UE do not know when the gNB will trigger the LP-WUR of UE to wake up the MR.

Observation 6:  The following options may be feasible for LP-WUS de-activation

- SIB or dedicated RRC

- LP-WUS monitoring de-activated based on criteria or autonomously such as the UE can use the MR ON status to de-activate the LP-WUS

Observation 7: In idle/inactive state the MR of UE maybe trigger by two times, one time by LP-WUS to wake up and one time by PEI to indicate paging, which may increase the UE power consumption.

Observation 8: In idle/inactive state the MR trigger of a UE via the LP-WUS can be consider as an indication for a UE to monitor the upcoming PO, which can replace the Rel-17 PEI.

Observation 9: The implicit or explicit indication for the LP-WUS reception from the MR of a UE may enhance the LP-WUS reliability.

Observation 10: The flexible bandwidth configuration of LP-WUS in the carrier BWP, or restricting the LP-WUS to the lower edge or upper edge of the carrier BWP, may avoid holes between the carrier's bandwidth for the legacy NR channel.

Observation 11: Restricting the placement of LP-WUS to the middle of carrier BWP, creates holes in the middle of the carrier bandwidth for NR legacy channel resulting in frequency resource wasting.

Observation 12: The transmission of LP-SS and LP-WUS in different BWP, may let the LP-WUR to perform RF retuning for LP-SS, and increase its power consumption.

Observation 13: LP-WUR based dedicated RRM measurement relaxation may reduce the LP-WUR power consumption.

Proposal 1: Support both options for the LP-WUR synchronization and prefer option 2:

- Option 1: Aperiodic signal transmitted as part of LP-WUS

- Option 2: Periodic signal transmitted separately from LP-WUS

Proposal 2: For Duty cycle based LP-WUS monitoring, the LP-WUR ON/OFF duration can be associated with the current DRX and e-DRX durations.

Proposal 3: For activation/de-activation of LP-WUS monitoring, study an activation/de-activation trigger from gNB, such as a payload in the form of bitmap carries in the LP-SS.

Proposal 4: For De-activation of LP-WUS monitoring at least the following options can be studied

- SIB or dedicated RRC

- Implicit or UE autonomous De-activation based on the MR ON status

Proposal 5: Consider the LP-WUS as an alternative option for paging early indication.

Proposal 6: The following procedures can be considered after the MR of UE wake-up from ultra-deep sleep

- Perform PO monitoring, and afterwards follow legacy procedures

- Transmit PRACH for initial access, and follow legacy procedures

Proposal 7: Study an indication method for LP-WUS reception to ensure the successful detection of LP-WUS and improve its reliability.

Proposal 8: Consider at least the following two options for the placement of LP-WUS in the carrier bandwidth.

- Placement within carrier/BWP is flexible.

- Placement within carrier BWP is restricted to at the edge(s) of a carrier

Proposal 9: Study a dedicated BWP for the placement of LP-WUS and LP-SS, with the maximum bandwidth less than or equal to the required bandwidth of the LP-WUS.

Proposal 10: Study at least the following RRM measurement relaxation for the LP-WUR based RRM measurement.

- RRM measurement without reporting

- RRM measurement based on the UEs group

- RRM measurement based on the LP-WUS

## R1-2302508_vivo.docx

Observation 1: For option OOK-1, the data rate/chip rate highly depends on the SCS of an OFDM symbol used for LP-WUS generation.

Observation 2: For OOK-1, new IFFT branch corresponding to high SCS is needed to achieve higher data/chip rate.

Observation 3: For OOK-1, uniformly distributed frequency spectrum density can be achieved.

Observation 4: OOK-2 provides additional degrees of freedom to exploit more frequency resources, the parallel transmissions can be used to achieve either high data rate or frequency diversity.

Observation 5: Option OOK-3 detection is susceptible to frequency error.

Observation 6: It is not clear how to fulfill RAN4 requirement on RE dynamic range at gNB side in option OOK-3.

Observation 7: Regarding the block ‘signal generation and modification’ in option OOK-4, signal modification can be used for making PSD of LP-WUS flatter and mitigating inter-cell interference.

Observation 8: For sequence-based OFDM signal, e.g., ZC sequence, the detection performance is more sensitive to frequency error compared with OOK waveform.

Observation 9: If the BW of LP-WUS is configurable, BPFs with different bandwidth should be implemented to better reject the interference, causing higher receiver complexity.

Proposal 1: Guard gap is included within LP-WUS bandwidth.

Proposal 2: The size of guard gap is not determined in RAN1.

Observation 10: If the BW of LP-WUS can be different for each RRC states, BPFs with different bandwidth should be implemented to better reject the interference, causing higher WUR complexity.

Proposal 3: The same BW of LP-WUS should be designed for different RRC states for simple implementation

Proposal 4: Study LP-WUS signal structure including:

- Alt 1: WUS payload only

- Alt 1a: WUS payload information is carried by sequence(s) selection

- Alt 1b: WUS payload information is carried by encoded bits (with CRC)

- Alt 2: a preamble (for sync) followed by WUS payload

- Alt 2a: WUS payload information is carried by sequence(s) selection

- Alt 2b: WUS payload information is carried by encoded bits (with CRC)

Proposal 5: Study LP-SS signal structure including:

- Alt 1: sequence(s)

- Alt 2: sequence(s) followed by additional message

- Alt 2a: additional message is carried by sequence(s) selection

- Alt 2b: additional message is carried by encoded bits (with CRC)

Observation 11: For LP-WUS structure with preamble using OOK waveform, the preamble with at least 16 chips can achieve reliable synchronization performance.

Proposal 6:

  - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

    - raw information bit-size

    - [time/frequency resources (including any guard bands), if applicable]

    - [total energyEPRE of LP-WUS across the time/frequency resources]

    - FFS: false alarm probability/rate

    - FFS: misdetection probability/rate

    - are kept [comparable or fixed].

Companies to report the time/frequency resources (including any guard bands), if applicable

Observation 12: For LP-WUS generated by reusing OFDM generator, the SINR can be defined per RE level, as that for OFDM based signal/channels.

Observation 13: For option OOK-1, assuming ‘on’ and ‘off’ chips are equally distributed, the power allocated to ‘on’ chip can be doubled on the average RE power to achieve certain average SINR.

Observation 14: For option OOK-4, if ‘on’ and ‘off’ chips are equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is consistent across OFDM symbols, the SINR in each OFDM symbol is equivalent to average SINR.

Observation 15: For option OOK-4, if ‘on’ and ‘off’ chips are not equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is not consistent across OFDM symbols, additional scaling on each OFDM symbol is needed to align the amplitude/power for ‘on’ chip.

Proposal 7: Study LP-WUS receiver autonomously determine the sampling rate, which can be beneficial from reducing power consumption perspective.

Observation 16: For duty cycle LP-WUS monitoring

- Pros: better MDR performance under certain FAR and higher power saving gain

- Cons: less NW flexibility and larger latency

Observation 17: Following pros and cons are observed for continuous LP-WUS monitoring

- Pros: higher NW flexibility and lower latency

- Cons: degraded MDR performance under certain FAR, which relies on signal/channel design, especially for long DRX cycle and non-ideal timing.

Proposal 8: It is beneficial to support RRM measurements based on LP-WUR, and the following enhancements can be considered.

- RRM measurements are totally offloaded to LP-WUR.

- LP-WUR performs RRM measurements, and MR performs relaxed RRM measurements.

Observation 18: Design for a unified LP-SS performed by LP-WUR for both synchronization and measurement is beneficial.

Proposal 9: Study suitable transmission periodicity of the LP-SS performed by LP-WUR for the following cases,

- LP-SS case 1: LP-SS is used for synchronization and measurement purpose

- LP-SS case 2: LP-SS is used for measurement purpose

Proposal 10: Following measurement metrics can be considered for measurement using WUR

- LP-RSRP;

- LP-RSRQ;

- LP-SINR;

- Detection error rate.

Observation 19: For coexistence between legacy PDSCH and LP-WUS,

- Semi-static resource sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering LP-WUS related signals can be used to improve the spectral efficiency.

- Dynamic resource sharing can be used if PDSCH is scheduled by DCI format 1_1; If PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how LP-WUS related signal is transmitted in PDSCH resource.

Proposal 11: Study both TDM and FDM multiplexing for co-existence between LP-WUS and legacy signals/channels, with both semi-static and dynamic manner.

Proposal 12: Reusing the LP-WUS resources for other NR signal/channel transmissions should be allowed.

Proposal 13: UE performs PO monitoring after main radio is waken up from ultra-deep sleep.

Proposal 14: For RRC idle/inactive mode, UE entering or exiting LP-WUS monitoring can be triggered:

- Alt1: via explicit network indication;

- Alt2: based on pre-configured condition(s). And a UE does not need to send notifications to gNB about the updates on LP-WUS monitoring behavior.

Proposal 15: For LP-WUS monitoring in RRC connected mode, RLM/BFD/CSI measurements are performed by UE Main Radio (MR) and the legacy procedures is reused.

Proposal 16: Study the procedures of LP-WUS monitoring interacting with the following for RRC connected mode:

- R16 DCP, R17 PDCCH skipping and R17 SSSG switching

- C-DRX

Proposal 17: For RRC connected mode, once LP-WUS is detected by LP-WUR, UE resumes PDCCH monitoring from micro/light/deep sleep of MR.

- During LP-WUS monitoring, UE MR is not expected to enter the ultra-deep sleep to avoid long wake up delay.

## R1-2302572_OPPO.docx

Regarding the LP-WUS signal design, we have following proposals:

Proposal 1: For the frequency band, RAN1 study the case that DL carrier of NR frequency band is used to transmit very low power wake-up signal.

The single WUS carrier for a UE low power receiver is associated with a DL carrier which the main radio will be waken upon.

The WUS signal should coexist with other NR signals, support cellular operation, and support main radio in FDD or TDD mode.

Proposal 2: For the OOK waveform, RAN1 can assume the DFT based multiple bits OFDM based WUS signal generation.

Proposal 3: For the FSK waveform, the size of sub-carrier group and the occupation bandwidth should be defined and taken into account into the evaluation.

Proposal 4: Wake-up signal consider OOK or FSK as basic detection wave form. NR OFDM symbol duration without CP symbol is N times of WUS symbol duration. The candidate value could be start from N = {4, 8}. Transmission bandwidth can be 6~24 PRB @ 15kHz.

Compatible NR CP is also inserted.

Guard band for WUS can be considered to be included in the transmission bandwidths, to support multiplexing with NR signals/channels.

In the case of FSK, frequency shifting should be in number of transmission bandwidth.

Proposal 5: UE group ID information and/or Cell related information should be carried by wake-up signal. Wake-up signal support Manchester code and CRC.

Proposal6: Study the scenarios with very low power wake-up signal carrying a WUS ID and mapped to a cell ID.

Proposal7: Study the mechanism for very low power wake-up signal has similar coverage as normal PSS/SSS.

Study the power saving procedure in case of wake-up signal outage.

Proposal8: Study whether the very low power WUS signal measurement can facilitate the main radio wake-up and supporting the mobility of UE.

## R1-2302622_Spreadtrum Communications.docx

Sync and RRM measurement at the LP-WUR

Proposal 1: Whether the LP-WUR can support neighboring-cell measurement can be discussed further.

Proposal 2: At least, sync and serving-cell measurement is supported by the LP-WUR.

Proposal 3: For sync signal, at least periodic signal is studied.

Proposal 4: OFDMA-based signals/channels (e.g. SSS/PBCH-DMRS) is prioritized for periodic sync signal.

Design of LP-WUS

Proposal 5: MC-ASK waveform is preferred, and Option OOK-1/4 is preferred.

Proposal 6: The code rate can be equal to or lower than 1/2 (e.g. Manchester code).

Proposal 7: The BW of one LP-WUS can be scalable.

Proposal 8: The structure of the LP-WUS does not mean a new slot format.

Proposal 9: The LP-WUS can contain two part, i.e. an aperiodic signal and a channel.

## R1-2302689_CATT.docx

Observation 1: OOK-1 brings better link performance than OOK-2 due to the high frequency diversity in OOK-1.

Observation 2: The SCS has negligible impacts on the link level performance due to the frequency diversities remains the same within the BW of LP-WUS regardless of the SCS used.

Observation 3: Higher sampling rate brings better performance due to the less error happens.

Proposal 1: The power model of the OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example.

Proposal 2: It is expected to support TDM and FDM multiplexing between LP-WUS and NR legacy channel/signal. The LP-WUS design should take into account inter-channel interference mitigation techniques when LP-WUS and NR signals are multiplexed in the same NR carrier.

Proposal 3: The LP-WUS should take the pulse shape filter into consideration as the requirement to limit the ICI to the adjacent channel.

Proposal 4: The behavior of  LP-WUS monitoring can be rely on the UE configuration of monitoring the LP-WUS transmission occasions with duty-cycle to align the duty-cycle with the periodicity of DRX for RRC_CONNECTED mode UEs or PO for RRC_IDLE/INACTIVE mode UEs.

Proposal 5: The timing error/frequency error caused by drift and wander of reference clock would impact the system design for LP-WUS monitoring and detection. The enhancement to reduce the timing error/frequency error for synchronization and LP-WUS monitoring window should to be studied.

Proposal 6: The SCS used for LP-WUS generation and the SCS used for other NR channels/signals in the same carrier can be different.

Proposal 7: The network support of UE wakeup mechanism by LP-WUR needs to inform all UEs by broadcast the configuration of wakeup signals through SIB-1 or SIB-x at a given cell.

Proposal 8:  UE capability includes the receiver sensitivity of the low-power wakeup receiver.  UE will report it’s supported of low-power wakeup receiver in the UE capability.

Proposal 9:  Network should support the LP-WUS waveform configuration.

Proposal 10:  Cell-specific or UE-specific configuration of LP-WUS resource could be supported.

Proposal 11:  Configuration of wake-up signal with unique UE identification should be supported.

Proposal 12: Same or different BW between RRC_CONNECTED and RRC_IDLE/INACTIVE mode should be supported.

Proposal 13: The RF envelop detector in the LP-WUR architecture does not require accurate timing information for the detection of LP-WUS. Option 3 provides the flexibility in the network configuration of synchronization signals and preambles to achieve obtain more accurate synchronisation performance with less resource overhead.

Proposal 14: For a UE support LP-WUR in RRC_IDLE/INACTIVE mode, RRM measurements on LP-WUS should not be supported.

## R1-2302817_Intel Corporation.docx

Observation 1: To support different SCS for LP-WUS and NR transmission,

  - From standard point of view, the support of different SCS for LP-WUS and NR transmission is similar to the support of different SCS for different NR transmissions in legacy system.

    - LP-WUS and NR transmission can be multiplexed by TDM or FDM.

    - Guard band is needed to avoid interference to LP-WUS reception or NR transmission reception, which can be supported by proper frequency resource allocation.

  - From gNB implement point of view, the support of different SCS for LP-WUS and NR transmission increases gNB complexity, if typical deployment for existing NR system only supports single SCS.

Proposal 1: Hold on study for OFDM-based existing signals/channels as LP-WUS until detailed break-down for the components of LP-WUR on power consumption reduction compared to the MR is provided.

Proposal 2: For MC-ASK waveform generation, Option OOK-1 can be the baseline, in case of small payload of LP-WUS. If larger payload is needed, option OOK-4 with same SCS as NR transmission or option OOK-1 with larger SCS than NR transmission can be considered, with reasonable complexity and cost.

Proposal 3: For MC-ASK waveform generation, further study option FSK-1 and FSK-3

- For FSK-3, N SCs of LP-WUS are separated to one pair of segments with potential guard-bands in-between and around.

  - Segment comprises one sub-carrier or multiple contiguous SCs

  - For each segment, N/2 SCs are generated by a transformation (DFT/Least square)

    - N/2 samples are generated from M-bits

    - In the pair of segments for each bit, one segment is modulated, other segment is zero power (from base-band point of view)

Proposal 4: Study coding scheme for LP-WUS, taking Manchester code and repetition code as starting point.

Proposal 5: Study LP-WUS structure based on two parts,

- 1st part is at least for LP-WUS presence detection and frequency/time synchronization.

  - 1st part is a known-sequence.

- 2nd part is for wake-up message.

  - 2nd part consists of a string of information bits.

  - The information bits include at least LP-WUS target ID, e.g., paging group, paging subgroup ID.

Proposal 6: Study synchronization signal used by LP-WUR, based on periodic signal with or without 1st part of LP-WUS for paging.

Proposal 7: Study LP-WUR monitoring based on duty-cycle

- The periodicity and offset for LP-WUS occasions can be configured by gNB.

- The duration of a LP-WUS occasion can be one or multiple consecutive OFDM symbols within a slot or over multiple slots.

Proposal 8: Study in-band LP-WUS which can be multiplexed with NR channels/signals in different PRBs within a carrier.

- LP-WUS bandwidth around 5MHz including one PRB guard band at each side can be the baseline.

- Study mechanism to utilize unused LP-WUS resource for NR DL/UL signal/channels

Proposal 9: Evaluate the impact of inter-cell interference and study mechanism for inter-cell interference handling.

Proposal 10: For activation/deactivation of LP-WUS monitoring, study the activation/deactivation mechanism based on gNB configuration and UE initiated procedure with or without report to gNB.

Proposal 11: Study at least following UE procedures upon detection of LP-WUS in RRC idle/inactive mode

- Option 1: Main radio is off until UE identifies its LP-WUS. UE may still need to monitor PEI after turning on the main radio, assuming UE can obtain group information of PO but without sub-group information by LP-WUS.

- Option 2: Main radio is off until UE identifies its LP-WUS. UE may directly decode Paging PDCCH/PDSCH without PEI after turning on the main radio, assuming UE can obtain at least sub-group information of PO by LP-WUS.

- Option 4: Main radio is off until UE identifies its LP-WUS. UE may turn on radio to receive system information update.

- FFS how to differentiate different cases by LP-WUS.

Proposal 12: Study relaxed RRM measurements for RRC idle/inactive mode.

- Study RRM measurements by LP-WUR for serving cell, based on RSRP, RSRQ-like metric of OOK/FSK symbols

  - Study the criterion to offload RRM measurement to LP-WUR.

- Study potential relaxation of RRM measurement requirement for main radio.

- Any optimization requiring large effort among different working groups, e.g., deep involvement of RAN1, RAN2 and RAN4, should be avoided.

Proposal 13: Further discuss the need of LP-WUS for RRC connected mode.

## R1-2302829_InterDigital, Inc..docx

Observation 1: Single bit per OFDM symbol can provide better reliability which is compromised for reduced power consumption.

Observation 2: Multi-bit per OFDM symbol can provide better system capacity when there’s no coverage issues.

Observation 3: Although FSK provides more reliable performance, FSK generally requires more complex implementation and more bandwidth than ASK as described in FSK-1 and FSK-2.

Observation 4: If OFDMA with existing signals/channels is utilized with existing UE receiver architecture, power reduction gain is doubted.

Observation 5: If OFDMA with existing signals/channels is utilized with a separate UE receiver architecture including components for power reduction, performance gain compared to OOK and FSK is doubted.

Observation 6: LP-WUS can be used to offload main radio’s RRM measurement for mobility support during idle/inactive state.

Observation 7: As LP-WUR already supports LP-WUS, RRM measurement based on LP-WUS provides efficient RRM measurement without additional UE complexity.

Observation 8: LP-WUS assisted paging during idle/inactive state may provide power saving benefits.

Observation 9: Utilization of LP-WUS/LP-WUR may not always be beneficial considering additional power consumption and low coverage/sensitivity of LP-WUR.

Observation 10: Conditions such as coverage/sensitivity for enabling/disabling LP-WUR may dynamically change due to UE movement and blockage.

Proposal 1: Consider both single bit and multi-bit per OFDM symbol for LP-WUS design.

Proposal 2: Considering limited UE implementation complexity scenarios such as IoT use cases for LP-WUR, it is preferred to prioritize OOK than FSK.

Proposal 3: Carefully evaluate performance and power reduction gain by utilizing OFDMA based receiver with existing signals/channels.

Proposal 4: For single bit per OFDM symbol, OOK-1 is prioritized considering its simple structure.

Proposal 5: For multi-bit per OFDM symbol, OOK-2 and OOK-4 with DFT precoding are prioritized.

Proposal 6: Key design parameters and aspects of LP-WUS including LP-WUS symbol length, supported SCSs, number of modulated bits per LP-WUS symbol, whether to use dedicated/standalone bands, bandwidth, transmitter architectures, coverage (or receiver sensitivity) and multiplexing should be carefully decided considering the design trade-off between the key parameters/aspects of LP-WUS.

Proposal 7: Benefits from introducing additional signal such as LP-SS should be carefully evaluated.

Proposal 8: Considering limited payload size of LP-WUS, indication of SI change via LP-WUS should be considered.

Proposal 9: For measurement metric, RSRP can be a starting point as it is simple and already supported by a main receiver. On the other hand, using LP-WUS can be also considered for reusing LP-WUR implementation.

Proposal 10: For cell identification, providing cell ID in LP-WUS can be considered. Detailed design of cell ID indication is FFS.

Proposal 11: Mobility to neighboring cells which is capable of LP-WUS transmission should be supported.

Proposal 12: Relaxation of existing RRM measurement requirements should be considered for LP-WUR. How to appropriately handle limited coverage of LP-WUR can be further discussed.

Proposal 13: Study procedures to use LP-WUS to facilitate power-efficient paging operation during idle/inactive state and evaluate the potential power saving gains.

Proposal 14: Study efficient configuration, activation and deactivation mechanisms for LP-WUS and LP-WUR.

Proposal 15: Although FR1 may be a main frequency range for LP-WUS, FR2 should be considered for LP-WUS design.

Proposal 16: Study enhanced beam related procedures for supporting LP-WUS in FR2 should be studied.

## R1-2302862_Sony.docx

Observation 1 – To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

Observation 2: RTC is used to control the time at which the LR needs to wake up to monitor the LP-WUS monitoring occasion.

Observation 3: The length of the LR’s LP-WUS monitoring window increases due to the timing inaccuracy of the RTC in the duty-cycled monitoring case.

Observation 4: The length of the LR’s LP-WUS monitoring window is minimized when the LP-WUR has recently synchronised to the network.

Observation 5: LP-SS reduces LR power consumption. The power consumption reduction is most pronounced when the paging rate is lower.

Observation 6 - Currently, the signals available for cell re-selection evaluation procedure, i.e., the synchronization signals (SSB) or reference signals of the neighbor cells, can only be measured by the main radio.

Proposal 1 – For OOK-based LP-WUS, support LP-WUS designs compensating for LP-WUR performance loss using spreading.

Proposal 2 – For OOK-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 3 – Support MC-OOK option 4 mechanism to embed LP-WUS within OFDM transmissions without creating interference to other OFDM transmissions.

Proposal 4 – For OFDMA-based LP-WUS, support LP-WUS designs consisting of N WUS preamble / detection symbols and M WUS information symbols. Values of N and M can be chosen for a suitable SNR operating point.

Proposal 5 – For OFDMA-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 6 – Support an adaptive configuration of LR monitoring behaviour, where the UE, depending on its delay requirement, can operate based on a continuous or a duty-cycle scheme.

Proposal 7: LP-SS is supported for duty-cycled monitoring of LP-WUS.

Proposal 8: For continuously monitored of LP-WUS, consider synchronisation based on an aperiodic signal transmitted as part of LP-WUS.

Proposal 9: Study the following structures for LP-SS:

- Sequence based LP-SS

- LP-SS signalled as a bit string within the regular LP-WUS

Proposal 10: RAN1 studies the need to monitor the reliability / performance of the LP-WUS.

Proposal 11 – Consider low-power mechanisms such as transmission of low-power reference signal to support mobility and cell re-selection mechanism for UEs with LP-WUR.

## R1-2302892_Nokia, Nokia Shanghai Bell.docx

Observation 1:  	Wider LP-WUS BW can offer robustness against fading, but impacts the spectrum efficiency and feasibility of multiplexing with other signals

Observation 2: 	Having limited LP-WUS BW could allow adoption in different device types and enable use in different type of deployments.

Observation 3:  	Wider LP-WUS BW could enable support of more flexibility in signal generation and different signal characteristics and improve the tolerance to implementation impairments.

Observation 4:  	The effects of assumed SCS on the LP-WUS symbol duration should be studied.

Observation 5:  	It may be beneficial for some HW implementations to have a preamble for comparator threshold settling.

Observation 6:  	Sync field may be added to help the LR to find the beginning of the WUS payload. The sync field pattern can be also used to distinguish content and target group.

Observation 7:  	Payload field can be used to carry relevant information and have different content and/or sizes based on the use case/procedure.

Observation 8:   	FCS/CRC check field calculated on the wake-up message payload will lower the FAR and ensure the integrity of the payload.

Observation 9: 	To achieve a good trade-off between sensitivity and power consumption, it may be necessary to introduce DRX on the wakeup receiver (as opposed to an always on receiver).

Observation 10: 	The LP-WUS signal reception can suffer from intercell interference and other serving cell signals, which may degrade the false detection performance thereby reducing the power consumption savings from the introduction of the LP-WUS.

Observation 11: 	The number of FFT/IFFT operations used at the gNB to generate LP-WUS with different SCS than the NR signal scales up exponentially with the ratio between SCS numerology used for LP-WUS and NR signal.

Observation 12: 	Mixed numerology may incur additional efforts to MR if it decides to monitor and detect LP-WUS in certain connected mode cases, wherein MR is already active.

Observation 13: 	If DFT-s-OFDM type of modulation is considered, effect of CP in the transmission should be studied if LP-WUS spans multiple OFDMA symbols.

Observation 14:	The performance of multi-bit OOK degrades significantly in comparison with that of AWGN channel due to multi-path effect as expected.

Observation 15: 	Oversampling at the LR is essential to improve the performance of OOK reception and to reduce the probability of false alarm.

Observation 16: 	Timing accuracy requirement for OOK detection is quite restrictive, which can only be achieved by using a preamble associated with each LP-WUS payload if LP-WUS synchronization has low periodicity

Observation 17: 	The performance degradation of OOK in TDL-C 1000ns is noticeable but not detrimental to render it useless.

Observation 18: 	The LP-WUS modulation should be resource efficient accounting need for possible guard bands, device BW restrictions and efficient multiplexing with other LP-WUS and other legacy signals.

Observation 19:	At least few bitwidth for ADCs, i.e., 4 or above, should be considered for evaluation as it benefits both the LR and the network power saving target.

Observation 20: 	FSK receivers are nothing but parallel OOK receivers. Thus, the total power consumed by the LR may be doubled in most of the implementation. However, if FSK based LP-WUS signal uses same SCS as NR transmission, then it can be easily received and decoded if in case MR monitors LP-WUS under more stringent radio conditions when LR is not applicable.

Observation 21: 	Due to the narrowness of ON segment in FSK-1/2 schemes, they may suffer from fading, crystal stability of LR, and can also cause PAPR problems at the gNB, if the total transmit power is allocated to a narrow section within LP-WUS BW.

Observation 22: 	LP-WUS using sequence-based CP-OFDM achieves better coverage and performance than the coded transmission. Multiple sequence or sequence properties can used to increase the information rate.

Observation 23: 	The modulation scheme used for LP-WUS must be chosen to reduce both power and latency of majority of UEs in the cell rather targeting a subset of UEs whose channel conditions are good.

Observation 24: 	If eDRX based evaluation rate is assumed, the mobility of the devices should be assumed to be restricted to semi-stationary to avoid service interruptions due to delayed mobility evaluations. Alternatively, if higher mobility (than semi-stationary) is assumed, the service should be able to tolerate longer latency.

Observation 25: 	If LP-WUS is considered for mobility measurements, the LP-WUS design would need to account the requirement to enable good accuracy within reasonable evaluation period. This may imply increase in the overhead.

Observation 26: 	RX chain imperfections of LP-WUR, as well the baseline reference architecture affects the feasibility of LP-WUS based mobility measurements.

Observation 27: 	Reducing the measurement activity in CONNECTED mode can have negative impact on service quality.

Observation 28: 	LP-WUS can be sent as an indication for paging and can enable UE to start RRC state transition to RRC-Connected in different ways depending on the payload size and content.

Proposal 1: 	The SI evaluates the LP-WUS BW accounting performance/robustness and applicability to different device types and deployments.

Proposal 2: 	The SI evaluates the benefits and the cost of supporting different LP-WUS fields.

Proposal 3:	The SI should prioritize duty cycled operation for LP-WUS monitoring as it may widen the scope of receiver architectures by studying the trade-off between power vs efficiency.

Proposal 4:	The SI should consider techniques to improve the robustness of the LP-WUS to inter-cell and intra-cell interference.

Proposal 5: 	Clarify the mobility assumption for the purpose of LP-WUS evaluations and design and ensure that assumptions are aligned with other assumptions.

Proposal 6: 	Evaluate the possible alternatives and feasibility to reduce the need of MR based RRM measurements with limited mobility performance impact.

Proposal 7: 	Evaluate new LR/LP-WUS performance measurements/alarms to assist the network to optimize the LP-WUS configuration.

Proposal 8: 	If CONNECTED mode operation with LP-WUS is considered, the link quality measurements and reporting need to be accounted for in evaluations.

Proposal 9:   	Consider different alternatives for LP-WUS payloads to support or replace paging PDCCH monitoring.

## R1-2302950_ZTE, Sanechips.doc

Observation 1: The problem of strong spectral line can cause the degradation of LP-WUS detection performance.

Observation 2: If phase randomization for generation of LP-WUS waveform is supported, the PAPR of the waveform is increased.

Observation 3: Considering the problem of Ripple & Peak clipping effects for envelope in reality, LP-WUS waveform with higher PAPR may have impact on envelope detection performance.

Observation 4: For Option OOK-4, based on the restriction of constant transmitting energy within LP-WUS payload transmission,

- For the same code rate, the priority of value of M is (M=4) > (M=8) > (M=2);

- For the same value of M, the priority of Manchester code rate is (Code Rate=1/2) > (Code Rate=1/4)

Observation 5: LP-WUS detection performance with ADC bits number >= 4 is very close to that of Ideal ADC.

Observation 6: For LP-WUS with bandwidth of 4.32MHz (i.e.,12 RBs with SCS=30KHz),

- LP-WUS detection performance with sampling rate=3.84MHz has a significant gap to that of No downsampling;

- LP-WUS detection performance with sampling rate=15.36 or 7.68MHz is very close to that of No downsampling.

Observation 7: For LP-WUS detection, guardband of 11SCs can mitigate up to 3dB ACI.

Observation 8: For LP-WUS detection, Guardband=24SCs@ACI=10dB has only 3dB performance loss at BLER=1% compared with the case of Guardband=11SCs@ACI=0dB.

Observation 9: Under the restriction of constant time-frequency resources and transmitting power, assuming No Time/Frequency error,

- For Option OOK-1, since CP is added for each OOK symbol, ISI caused by multi-path delay of fading channel can be solve and the performance of the Option OOK-1 is slightly better than that of the Option OOK-4.

- For Option OOK-2, since the bandwidth of LP-WUS is obviously reduced, the envelop performance will be obviously affected. Therefore, LP-WUS detection performance will be degraded obviously.

Observation 10: For LP-WUS with Option OOK-1, OOK-2 and OOK-4, the capability of mitigating impact of frequency error is, Option OOK-1 > Option OOK-4 > Option OOK-2.

Observation 11: Under the restriction of constant time-frequency resource allocation and same transmitting power, if no time/frequency error is assumed, the MDR of longer LP-WUS preamble sequence and shorter LP-WUS preamble sequence are nearly the same.

Observation 12: For MC-FSK with parallel homodyne receiver architecture, if M>=2, MC-FSK detection performance will be degraded obviously.

Observation 13: It is observed that the link level performance at BLER=10% based on receiver 1 and receiver 2 is almost same for the case of the ZC sequence length of 23 under ideal cases.

Observation 14: For ZC sequence-based OFDM, the link level performance is not impacted by the aspect of sequence correlation in frequency domain or in time domain.

Observation 15: It is observed that the SNR values at BLER=10% based on OFDM receiver 1 without down sampling and with down sapling factor of 1/4 are almost same for the case of ZC sequence length of 23.

Observation 16: It is observed that the SNR values at BLER=10% based on receiver 1 decreases with the increase of the length of ZC sequence of 23, 47 and 83.

Observation 17: The link level performance is seriously impacted by the aspect of frequency offset for sequence-based OFDM receiver.

Proposal 1: Adding phase randomization for LP-WUS waveform generation can be used to solve the problem of strong spectral line.

Proposal 2: Discuss and decide whether PAPR should be considered for envelope detection performance.

Proposal 3: SNR definition of MC-OOK in time domain should be specified as follows:

SNR =E[|S|2] / E[|N|2]

Where E[|S|2]= Num1*E[|s1|2] +Num0* E[|s0|2], E[|N|2] = (Num1+Num0)*E[|n|2]

Wherein,

-  Num1 is the number of MC-OOK symbols with bit “1” during LP-WUS transmission;

-  E[|s1|2] is the energy for a MC-OOK symbols with bit “1”;

-  Num0 is the number of MC-OOK symbols with bit “0” during LP-WUS transmission;

-  E[|s0|2] is the energy for a MC-OOK symbols with bit “0”;

-  E[|n|2] is the energy of white noise per MC-OOK symbols within the LP-WUS bandwidth.

Proposal 4: Phase noise modeling with only “white frequency noise” is supported for LP-WUS.

Proposal 5: At least 4-bit ADC is supported for MC-OOK based LP-WUS transmission.

Proposal 6: For LP-WUS with bandwidth of 4.32MHz (i.e.,12 RBs with SCS=30KHz), sampling rate 7.68MHz is prioritized.

Proposal 7: For LP-WUS detection, guardband with at least 1PRB should be configured at each side of LP-WUS and detailed guardband size can be configured by gNB in accordance with the strength of ACI.

Proposal 8: Further clarify the difference in LLS for OFDM receiver with FFT or without FFT

Proposal 9: Do not study the cell selection directly based on the RSRP/RSRQ of LP-SS in Rel-18

- LP-SS is only used for measurement assistance, which means if LP-SS performance becomes worse, the MR wake up to fallback to legacy measurement procedure.

Proposal 10: Consider the LP-SS for serving cell measurement assistance and don’t consider LP-SS for neighbor cell measurement.

Proposal 11: Regarding measurement metrics

- For OOK, RSRP, RSRQ, SINR definition could be adapted.

- For MC-FSK with OOK, RSRP, RSRQ, SINR definition could be also adapted.

- For OFDM sequence-based receiver, discuss whether RSRP, RSRQ, SINR could be reused.

Proposal 12: Consider three relaxing RRM mechanisms

- Mechanism 1: UE performs relaxed RRM measurement for serving cell based on SSB and LP-SS.

- Mechanism 2: UE performs relaxed RRM measurement for serving cell based on LP-SS assistance

- Mechanism 3: UE performs relaxed RRM measurement for serving cell based on SSB without LP-SS assistance

Proposal 13: Activation/deactivation of LP-WUS function via gNB is the baseline.

- Further consider UE based criterion for activation/deactivation of LP-WUS function

Proposal 14: Consider to monitor PEI or PO after UE receives LP-WUS.

Proposal 15: Consider the procedure of dynamically monitoring PO to reduce the latency.

Proposal 16: Take table 4 into account for duty cycle and continuous monitoring comparison.

Proposal 17: LP-WUS in connected mode also can be considered in SI stage.

Proposal 18: For different power saving mechanisms in connected mode, take the following into consideration,

- Power saving gain

- UPT or capacity

- Overhead

- Coverage

Proposal 19: In connected mode, study on LP-SS for replacing SSB/CSI for RLM or BFR is not considered in R18.

Proposal 20: For idle/inactive mode, the information carried via LP-WUS

- Include UE group ID or UE ID

  - FFS how to reduce the overhead for carrying UE group ID or UE ID

- Study whether system information modification, ETWS is included

Proposal 21: For connected mode, the information carried via LP-WUS

- Include the information corresponding to the power saving mechanisms via LP-WUS

Proposal 22: Study the following signal structure for LP-WUS

- Alt 1: LP-WUS consists of only preamble.

- Alt 2: LP-WUS consists of preamble and corresponding payload.

- Alt 3: LP-WUS consists of only payload.

Considering the following aspects

- UE specific or group specific

- Synchronization

- Training

- Check bits

- Boundary positioning

Proposal 23: Study the following signal structure for LP-SS

- Alt 1: LP-SS consists of only preamble.

- Alt 2: LP-SS consists of preamble and corresponding payload.

Proposal 24: LP-SS information may include system information modification, ETWS.

Proposal 25: for synchronisation signal used by LP-WUR,

- Aperiodic synchronisation signal can be transmitted separately from LP-WUS

- Synchronisation signal with periodic monitoring window is periodic signal

- For aperiodic synchronisation+ periodic synchronisation signal, both aperiodic LP-SS as part of periodic LP-SS or separate from periodic LP-SS can be considered.

Proposal 26: LP-WUS transmission in a band/carrier/BWP is up to gNB

- LP-WUS transmitted in another band is feasible.

Proposal 27: Discuss whether to continue to monitor the LP-WUS and corresponding UE behavior when MR is on after detecting LP-WUS.

Proposal 28: Discuss the procedures when the LP-WUS is missing.

## R1-2302969_xiaomi.doc

Proposal 1: Support periodical synchronization signals for synchronization between LP WUR and gNB.

Proposal 2: Support BW of one LP-WUS is not greater than 5 MHz for FR1

Proposal 3: Support BW of one LP-WUS is configurable, and support LP WUS BW being different for RRC idle/inactive and RRC connected states.

Proposal 4: UE group ID and/or UE ID should be supported to be indicated by LP WUS.

Proposal 5: Whether the coverage of LP WUS is whole cell or only cell centre should be determined and study whether coverage enhancement is needed.

Observation 1: If LP WUS coverage is the whole cell, both intra-cell and inter-cell measurement are possible, but if LP WUS coverage is only cell centre, then inter-cell measurement is not possible and intra-cell measurement is also restricted in cell centre.

Proposal 6: Enhanced paging mechanism can be studied to reduce overall latency.

## R1-2303033_China Telecom.docx

Observation 1: Larger signal bandwidth is beneficial for demodulation performance.

Observation 2: Smaller signal bandwidth should be considered for network deployment flexibility

Observation 3: Demodulation performance over 6RB bandwidth no longer offers significant performance gain.

Observation 4: Combining synchronization sequence with the WUS signal is an appropriate solution to improve timing accuracy and reduce network overhead.

Observation 5: Different SCS between WUS and NR signals can increase the chip rate of WUS signal and improve the generation flexibility.

Observation 6: Different SCS between WUS and NR signals causes huge impacts on the network.

Observation 7: DRX operation can be utilized for LP-WUS monitoring.

Observation 8: DRX cycle needs to be small and configurable, and can be utilized when the main radio is in the sleep state.

Observation 9: The synchronization part can be utilized as the measure signal for relaxed RRM measurement.

Proposal 1: 6RB WUS bandwidth can be regarded as the starting point.

Proposal 2: Option 1, i.e., aperiodic signal transmitted as part of LP-WUS, is prioritized as the WUS signal structure.

Proposal 3: UE paging indication or UE group paging indication can be carried in the information part of the WUS.

Proposal 4: Different SCS between WUS and NR signals is deprioritized.

Proposal 5: DRX operation can be prioritized in the monitor mechanism.

## R1-2303061_Sharp.docx

Proposal 1: Further study on option OOK-1 and OOK-4 should be prioritized for LP-WUS.

Proposal 2: Manchester encoding should be considered for OOK options.

Proposal 3: The SCS of WUS should be equal to or larger than the SCS of DL BWP.

Observation: Two types of CP alignment patterns can be considered for further study.

Proposal 4: Resource allocation for LP-WUS should keep flexible with configurable bandwidth and symbols.

Proposal 5: Support FDM/TDM for LP-WUS resource allocation.

Proposal 6: At least for idle/inactive UE, the LP-WUS should support beam sweeping.

Proposal 7: Support periodic LP-SS transmitted separately from LP-WUS.

Proposal 8: LP-SS should keep the same coverage as LP-WUS.

Proposal 9: LP-WUS for idle/inactive UE should be prior in the study.

Proposal 10: Support periodic-on mode for WUS monitoring.

Proposal 11: Unified procedure can be considered for UEs provided or not provided with eDRX.

Proposal 12: The performance and overhead should be compromised for LP-WUS design.

## R1-2303152_Samsung.docx

Proposal 1: Prioritize FR1 in the study, from RAN1 perspective.

Proposal 2: LP-WUS located in a band separate from the UE’s NR band is not precluded, and for in-band operation, power boosting of LP-WUS is feasible, from RAN1 perspective.

Proposal 3: For LP-WUS waveform, deprioritize MC-FSK if no significant performance gain is observed.

Proposal 4: For LP-WUS numerology:

- Different subcarrier spacing is only applicable to MC-OOK and MC-FSK based waveforms;

- Candidate subcarrier spacings for LP-WUS are 15 kHz, 30 kHz, 60 kHz, and 120 kHz, for FR1;

- Extended CP is not supported.

Proposal 5: For resource allocation of LP-WUS:

- Frequency location of LP-WUS shall be flexible;

- FDM and TDM with other signal(s) and channel(s) shall be supported;

- BW of LP-WUS is fixed as a number of RBs;

- Potential CP rejection at the receiver should be addressed, when LP-WUS spans over multiple OFDM symbols.

Proposal 6: Information carried by LP-WUS (and LP-SS if supported) can be studied, including:

- Identification (cell ID, UE ID, or UE-group ID);

- Timing information;

- Indication on whether MR is triggered to operate;

- Other information to support additional functionality such as system information modification indication, paging information.

Proposal 7: Study different options or their combinations for supporting synchronization using LP-WUS and potentially LP-SS.

- Option 1: LP-WUS supporting both synchronization and waking-up functionalities;

- Option 2: Each transmission instance including two parts: LP-SS for synchronization and LP-WUS for wake-up indication;

- Option 3: Two types of signals: LP-SS for synchronization and LP-WUS for wake-up indication.

Proposal 8: Study explicit and implicit triggering mechanisms for the LR, and potential confirmation message(s) from the MR to the gNB.

Proposal 9: Study DRX operation for the LR, wherein the DRX configuration can be provided to the UE by system information or dedicated RRC parameter.

Proposal 10: Study explicit and implicit triggering mechanisms for the MR, and potential confirmation message from the MR to the gNB.

Proposal 11: Study LP-WUS/LP-SS based RRM measurement by the LR, for both serving cell RRM measurement and neighboring cell RRM measurement.

Proposal 12: Study relaxation of RRM measurement requirement by the MR, such that the MR is not required to perform RRM measurement when it operates with low power.

## R1-2303255_CMCC.docx

Proposal 1. Multi-bit OOK options can be considered as the high-priority option to be studied.

Proposal 2. Support Option OOK-4 for further study.

Proposal 3. Manchester Encoding can be the candidate method to enhance the reliability of MC-ASK(OOK) based LP-WUS.

Proposal 4. Support flexibility configuration of LP-WUS bandwidth location. Both inside and outside initial DL BWP can be considered.

Proposal 5. Support payload based LP-WUS structure and study the payload size.

Proposal 6. Support Option 3 as the synchronization scheme to further study.

Proposal 7. For UEs in RRC_IDLE/RRC_INACTIVE state, LP-WUS can replace PEI to trigger MR whether to monitor PO.

Proposal 8. For UE in RRC_CONNECTED state, LP-WUS is used to indicate UE whether to monitor PDCCH or not similar to Rel-16 DCP and Rel-17 PDCCH skipping/SSSG switching and strive a joint LP-WUS design for all functions.

Proposal 9. Coverage of LP-WUS/WUR should be comparable with the main radio.

Proposal 10. LP-WUS jointly applied with other paving saving techniques to overcome the coverage hole issue.

## R1-2303334_MediaTek Inc..docx

Observation 1	By monitoring PSS and SSS, LPWUR can prevent broadcasting periodic LPSS and NR SSB, thus achieving timing synchronization, maintaining a duty cycle, and monitoring serving cell RRM.

Proposal 1	RAN1 should take advantage of LPWUR to monitor periodic NR PSS and SSS for a duty cycle and serving cell relaxation, while also monitoring OOK-based LPWUS to reduce hardware complexity and power consumption. This would ensure the best balance between signaling overhead and energy saving.

Observation 2	The OOK LPWUR can maintain its low complexity by using an independent receiver to provide timing synchronization and the DC level estimation, to decode the OOK-based LPWUS.

Proposal 2	An OOK-based LPWUR should have an independent receiver to provide timing synchronization and the DC level estimation, as well as an AGC module to estimate the ADC range for improved signal reception.

Observation 3	Lowering the LO accuracy from 10ppm to 200ppm results in higher power consumption and a risk of false alarms.

Proposal 3	To reduce the power consumption and the risk of false alarms, the main radio (MR) should generate the PSS candidates with fewer hypotheses and stay awake before LPWUR has built up its synchronization.

Observation 4	After LPWUR continues monitoring over 104.2 s at a sampling rate of 3.84 MHz, LPWUR will most likely wake MR up due to a false alarm with a 98.2% FAR.

Proposal 4	To prevent the risk of a high FAR, UE should not turn MR off before LPWUR receives the first LPWUS or synchronizes with the network successfully, and LPWUR should not expect to receive another LPWUS after a certain time.

## R1-2303421_EURECOM.docx

Proposal 1: Consider encoding of the WUS payload, e.g. Manchester code, for improved performance.

Proposal 2: Consider an overlay code in time-domain to increase spectral efficiency of the WUS.

Proposal 3: Consider encoding information via different sequences.

Proposal 4: Consider similar WUS configuration as in LTE-M/NB-IoT.

Proposal 5: Consider the WUS to carry other message types besides wake-up message.

Observation 1: The WUS BW is a trade-off between, payload capacity, multipath diversity and filter complexity.

Observation 2: In AWGN, decreasing the number of SCs of OOK symbols improves performance because less noise if captured.

Observation 3: OOK-2 cannot fully exploit frequency diversity because OOK symbols are confined to a subset of the WUS bandwidth.

Observation 4: Given the same bandwidth, OOK-1 is more robust to ACI than OOK-4 because the interference is averaged over more samples.

Observation 5: Given ideal AGC, an ADC with 4-bit resolution is sufficient for close to optimal performance.

Observation 6: For OOK-1, Manchester coding improves robustness to timing inaccuracies.

Observation 7: OOK-4, shorter the OOK symbols increase sensitivity to timing inaccuracies.

## R1-2303430_LG Electronics.docx

Proposal #1: Focus on studying MC-OOK waveform generation based on the legacy OFDM transmitter due to its simplicity and low power consumption.

Proposal #2: Study on whether/how LP-WUS transmission can be multiplexed with the legacy NR signals/channels in time and/or frequency domain.

Proposal #3: Define OOK-1 and OOK-4 as the baseline for discussions on other aspects for LP-WUS

- OOK-1 for low data rate

- OOK-4 for high data rate

Proposal #4: Discuss on whether LP-WUS is aligned with the symbol boundary of CP-OFDM for NR signal/channel which is overlapping with.

Proposal #5: LP-SS is generated by MC-OOK/MC-FSK and the same sampling rate (or SCS) as that for LP-WUS can be considered

Proposal #6: Discuss whether LP-SS is generated by MC-OOK/MC-FSK waveform which is the same or different from LP-WUS waveform generation

- Alt-1: LP-SS is generated based on the same MC-OOK/MC-OOK as LP-WUS

- Alt-2: LP-SS is generated based on MC-OOK/MC-OOK different from LP-WUS

Proposal #7: Study on how to achieve the comparable coverage of LP-WUR as the NR paging PDCCH

Proposal #8: Study on whether/how to handle the smaller coverage of LP-WUR than the NR paging PDCCH

Proposal #9: Consider the procedures for IDLE/INACTVE mode UEs for studying pros and cons of monitoring behaviors of LP-WUR.

Observation #1: What should be UE behaviour for LP-WUS is interrelated to LP-WUS performance requirement

Proposal #10: Study whether to support that UE monitors PEI or paging after the UE receives LP-WUS in IDLE/INACTIVE, considering wake-up latency.

Proposal #11: Study whether to support that UE attempt random access after the UE receives LP-WUS in IDLE/INACTIVE, considering false alarm rate and the number of UE group per MO.

Proposal #12: Study whether/how to support more than one UE behaviour for LP-WUS in IDLE/INACTIVE state.

Proposal #13: Study whether/how to introduce PDCCH monitoring occasion for LP-WUS monitoring, which can be enabled temporally based on LP-WUS reception.

Proposal #14: Study on the fallback operation where a UE cannot receive an LP-WUS signal, including how the UE can determine whether to perform fallback operation.

Proposal #15: Study the LP-WUS functionalities for CONNECTED mode for UE power saving.

Proposal #16: LP-WUS should be designed assuming the different procedures and functionalities of LP-WUS between CONNECTED and IDLE/INACTIVE modes.

Proposal #17: Study RRM measurement by LP-WUR for relaxing or offloading RRM measurement by the main radio.

Proposal #18: Study on how to utilize RRM measurement by LP-WUR as a complementary for RRM measurement by the main radio.

Observation #2: Potential measurement metric used for RRM measurement performed by LP-WUR can vary on which waveform of LP-WUS and receiver architecture are adopted.

## R1-2303507_Apple.docx

Proposal 1: Duty cycle operation of WUR should be prioritized in the LP WUS design.

Observation 1: If synchronization signal is needed, Option 1 (aperiodic signal transmitted as part of LP-WUS) is not sufficient as a standalone solution, while the comparison between Option 2 (periodic signal) and Option 3 (Option 1 + Option 2) requires quantitative analysis based on the synchronization requirements.

Proposal 2: Both UE-specific and group-common WUS should be considered to be supported.

Proposal 3: The additional information carried for WUR should be avoided minimized.

Proposal 4: Further study a harmonized WUS design that can be detected by both OOK-based receiver and sequence-based receiver, by defining the sequence used to generate the WUS signal.

Proposal 5: Study the mechanisms to enable and disable LP WUS/WUR for a UE, including the following two options:

- Option 1: the UE determines whether to enable or disable WUS/WUR

- Option 2: gNB determines whether whether to enable or disable WUS/WUR for a UE based on UE report

Proposal 6: For WUS monitoring with duty cycle operation, consider defining periodicity, offset and ON duration for the monitoring window.

Observation 2: Transmitting PRACH directly after receiving WUS may not be desirable due to the overhead associated with the false alarm.

Proposal 7: For PEI or paging DCI monitoring after WUS reception, consider starting the monitoring after an offset from WUS monitoring/detection, without waiting for the legacy PO.

Proposal 8: At least the following procedures should be studied for LP WUS/WUR for connected UEs:

- UE procedures for transitioning into and out of WUS monitoring

- The interaction with C-DRX, i.e., the procedures with or without C-DRX

- The interaction with other UE power saving features, such as R16 WUS, R17 PDCCH skipping, and R17 SSSS switching

- RLM procedures

## R1-2303538_Nordic Semiconductor ASA.docx

Proposal-1: Define reference FAR per reference time duration, this then allows to scale target FAR to target duration, e.g. monitoring cycle.

Proposal-2: Update the following in RAN1#112 agreement:

- [time/frequency resources (including any guard bands), if applicable]

- [total energy of LP-WUS across the time/frequency resources]

- time/frequency resources (including any guard bands) of a cluster

  - number of clusters per scheme is reported along with the results, e.g. 1,2,4

  - total energy of LP-WUS across the time/frequency resource cluster is constant

Proposal-3: Capture in TR: Support of a LP-WUS monitoring duty cycle is beneficial for reduction of false-alarm rate at the LP-WUR.

Observation-1: Synchronization signal for LP-WUS is beneficial to ensure that LP-WUS can hear the camping cell and to correct timing and frequency offset when LP-WUR is ON. Such signal should be known to UE, cell-specific and always present. Option2 is a baseline.

Observation-2: Along with LP-SS additional information could be sent to assist LP-WUR, such as SFN and/or cell identification.

Observation-3:  LP-SS periodicity of 50s could be sufficient for RRM measurement of serving cell in low mobility case, as well as, to keep timing within 1ms precision, At the same time impact on gNB power consumption and overhead would be insignificant.

Observaton-4: Aperiodic preamble preceding LP-WUS data is beneficial for AGC settling and would further improve timing and frequency synchronization of WUR.

Observation-5: When determining baseline information size for LP-WUS, for the purpose of simulations, candidate values could be 1,2,4,8,16 bits, these corresponding to number of sub-groups indicated in the LP-WUS.

Observation-6: When determining additional information size for LP-WUS, for the purpose of simulations, candidate values could be N*39 or N*24, where 39 corresponds to bit-size of NR UE ID in Idle, 24 corresponds to bit-size of NR UE ID in Inactive and N is max number of UE IDs indicated by the LP-WUS.

Obsevation-7: For overall energy consumption of WUR and MR, RRM measurements performed by MR become dominant energy consumption contributor when paging rate and FAR is low.

Observation-8: Feasibility of LP-SS receive signal power and signal quality may depend on whether ADC and AGC is part of the receive architecture.

Observation-9: Every agreed architecture will be capable to determine whether LP-SS has been detected in know occasion or not.

Proposal-4: For LP-SS consider m-Sequence, as a candidate OOK signal.

Observation-10: Configurability of LP-WUS BW would increase LP-WUR complexity. Target a single BW size.  Different BW for FR2, if supported, could be considered.

Observation-11: LP-WUS BW requirement should take into account a frequency error of the receiver.

Observation-12: A UE should subscribe for reception of LP-WUS in a camping cell.

Proposal-5: After MR is waken-up by LR, study the transition of MR from MICO deep sleep to

- PO monitoring

- transmitting directly PRACH, if including an UE ID in LP-WUS is deemed feasible.

## R1-2303614_Qualcomm Incorporated.docx

Observation 1: For Option OOK-4, randomize the phase of the pre-DFT OOK signal may help achieve a uniform power spectrum density, thereby making the OOK transmission more robust against frequency selective fading (or equivalently increasing the frequency diversity).

Observation 2: For OOK (Option OOK-4) and FSK (Option FSK-1) , on-the-fly DFT processing may be avoided by pre-storing the frequency domain (post-DFT) signals corresponding to different information bits (within one OFDM symbol) at the gNB.

Observation 3: at the same data rate, OOK (with Option OOK-4) is more robust against frequency error than FSK (with Option FSK-1), and FSK is more robust against timing error than OOK.

Observation 4: For MC-OOK (with Option OOK-4) and MC-FSK (with Option FSK-1), the sampling rate at the receiver needs to be set according to the bandwidth of the effective signal, which can be much larger than the information/data rate of the OOK/FSK transmission.

Observation 5: at the same data rate, MC-OOK (with Option OOK-4) is more robust against frequency domain selectivity (i.e., having a larger frequency diversity gain) than MC-FSK due to larger effective bandwidth.

Proposal 1: The LP-WUS coverage shall strive to match the coverage of NR (e.g., NR paging PDCCH coverage).

Proposal 2: RAN1 shall study the minimum amount of information conveyed by LP-WUS to support necessary functionalities, and the bandwidth configuration for LP-WUS.

- The LP-WUS shall at least support multi-user capability to be able to support addressing to more than one UE/UE groups per cell

Proposal 3: Study sequence design for LP-WUS for OOK-based waveform and CP-OFDM waveform, that can be detected by the LP-WUR with low complexity.

Observation 6: LP-SS can help in synchronization of the LP-WUR and reduce complexity of buffering for an entire WUS periodicity, which is in order of seconds or minutes.

Observation 7: LP-sync-preamble signal can be used for synchronization. However, it requires continuous monitoring by WUR, which is power consuming at the WUR.

Observation 8: LP-sync-preamble signal can be used with LP-SS to further help in reducing synchronization errors.

Proposal 4: Study pros and cons of LP-sync-preamble signal vs LP-SS schemes as methods for synchronization for WUR.

Observation 9: Continuous LP-WUS monitoring is not power efficient.

Proposal 6: A duty cycled monitoring scheme is the baseline for LP-WUR monitoring. RAN1 should further study the duty cycle configuration for monitoring LP-WUS considering target power and latency requirement.

Observation 10: Continuous monitoring of LP-WUS results in very high false alarm. Having even CRC of 24bits would results in false alarm probability of ~ 0.3 during 1.28sec, which is unacceptably high. High false wake up means unnecessary power consumption and increased average power.

Proposal 7: De-prioritize continuous WUS monitoring.

Observation 11: It is difficult to indicate full UE ID information in LP-WUS due to the max rate limitation, and if partial UE ID is used then the required bit width should be at least 10 to achieve a target PFA of 0.1%.

Observation 12: LP-WUS for paging monitoring can indicate the group information of the associated PO similar to Rel-17 PEI and the false wakeup rate can be reduced based on UE subgrouping.

Proposal 8: A baseline procedure for using LP-WUS for paging indication can be same as R17 PEI. RAN1 should study whether UE can be indicated by LP-WUS to perform RACH without processing the upcoming PO or monitor Rel-17 PEI firstly when MR is waked up.

Observation 13: Combination of LP-WUS and eDRX is beneficial for stationary UE use cases that do not require frequent RRM measurement.

Proposal 9: RAN1 study how to configure LP-WUS with DRX.

Observation 14: A low-power synchronization signal (LP-SS) needs to be defined for supporting RRM measurement performed by LP-WUR.

Proposal 10: LP-WUR can support measurements and cell selection criteria evaluation only on the serving cell. How to balance between power saving and measurement complexity should be studied.

Observation 15:  ±2 dB delta RSRP relative to genie RSRP may be achievable 90% of the time using OOK based LP-SS at SNR=-3 dB and realistic clock model.

## R1-2303673_NEC.docx

Observation 1: DFT-s-OFDM based OOK symbol generation is beneficial for the scaling of OOK symbol duration for different SCSs.

Observation 2: dropping CP at transmitter side may lose the orthogonality between LP-WUS signal and legacy OFDM symbol.

Observation 3: CP removing at receiver side may increase the complexity and power consumption of LP-WUR and lead to low spectrum efficiency.

Observation 4: CP can be enhanced as a useful part of OOK symbol, e.g., a CP with non-zero power can be seen as a useful part of an OOK-ON symbol, and a CP with zero power can be seen as a useful part of an OOK-OFF symbol.

Proposal 1: study DFT-s-OFDM based OOK symbol generation, to support transmit multiple bits by a single OFDM symbol.

Proposal 2: study CP enhancement to utilize the CP as a useful part of OOK symbols.

Proposal 3: study synchronization design for UE in low power mode, consider SSB based synchronization and low power synchronization signal (LP-SS) based synchronization.

Proposal 4: study RRM measurement for UE in low power mode, consider LP-SS based RRM and RRM relaxation for SSB based measurement.

Proposal 5: study wake-up procedure for UE in low power mode, consider sequence based LP-WUS (one or two sequences) and code block based LP-WUS for wake-up indication, and take the miss-detection and overhead issues into account.

## R1-2303730_NTT DOCOMO, INC..docx

Proposal 1: Further study duty-cycled monitoring of LP-WUS taking into account latency performance.

Proposal 2: The bandwidth of LP-WUS sequence should be assumed to be less than 20MHz for the evaluation purpose.

Proposal 3: UE (group)-specific information should be baseline for the contents of LP-WUS

‒	i.e., UE ID, UE group, and/or UE subgroup ID

Proposal 4: Further study the mechanisms to switch power state between Ultra-deep sleep and other states.

Proposal 5: UE procedure for paging and Msg. 1-4 after LP-WUS reception should be further discussed.

## R1-2303761_Ericsson.docx

Observation 1	With a single IFFT block, using a larger SCS for WUS for generating multi-bit OOK signal results in the coexistence issue and impact on other NR transmissions due to the time-domain operation (e.g., masking parts of the samples).

Observation 2	If WUS SCS is different from SCS used for other NR transmissions, FDM multiplexing between WUS and other NR transmissions cannot be enabled without significant gNB impact.

Observation 3	OOK WUS generation with single bit per OFDM symbol is straightforward with minimum impact on the OFDM transmitter.

Observation 4	M-bit OOK WUS generation (with multiple bits per OFDM symbol) using waveform fitting or DFT-based approach increases gNB complexity compared to single bit per OFDM symbol OOK WUS generation.

Observation 5	Phase discontinuous FSK modulation caused by CP, causes power leakage and higher requirement of timing synchronization in receiver.

Observation 6	For FSK-1, compensation of the phase shift caused by CP before OFDM modulation may be needed.

Observation 7	For the case of each segment comprises multiple subcarriers, the sequence mapped on the subcarriers impacts the envelop of time domain WUS signal. For the energy detection method such as envelop detector in the receiver, flat envelop signal should be designed to improve the detection performance.

Observation 8	For FSK-1, the complex sequence mapped on the subcarriers for WUS transmission should be studied further.

Observation 9	FSK-2 needs more segments than FSK-1 when M≥3 to carry same information.

Observation 10	For modulating ON symbols in OOK WUS, a sequence and random QPSK provide the same BER performance for the same signal power.

Observation 11	Adjacent cell interference can result in a significant coverage degradation when the interference is stronger than the signal.

Observation 12	Increasing ADC bit width is beneficial in terms of coverage, interference mitigation, and dynamic range of signal reception.

Observation 13	The coverage performance of single-bit ADC is extremely poor.

Observation 14	UE speed has a minor impact on the link performance considering a relatively large channel coherence time.

Observation 15	For non-coherent WUS detection, every repetition factor of 2 improves the link performance by around 2 dB.

Observation 16	For payload-based OOK WUS a relatively large number of WUS repetitions might be needed to match the PDCCH coverage. The total WUS duration with repetition is in order of tens of slots.

Observation 17	For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

Observation 18	Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Observation 19	For M-bit OOK generation, the link-level performance significantly degrades for M=8.

Observation 20	For M-parallel-OOK approach (OOK-2), guardband between segments is needed to improve the link performance especially when M >2.

Observation 21	For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

Observation 22	Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Observation 23	The main receiver transition time, 400 ms in ultra-deep sleep and 20 ms in deep sleep, is a lower bound for the achievable DL latency. Much more frequent WUR monitoring that this is only expected to increase the UE energy consumption without any noticeable improvement for the DL latency.

Observation 24	False-alarms are problematic for Continuous-WUR and cause the energy consumption reduction to be smaller than for Duty-cycled WUR, and even smaller than the DRX baseline for longer DRX cycles.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It should be possible to generate LP-WUS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

Proposal 2	It should be possible to multiplex the LP-WUS with other NR transmissions in time or frequency domain without causing interference.

Proposal 3	It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Proposal 4	Target the same coverage for LP-WUS as for Paging PDCCH.

Proposal 5	Consider the case of one (coded) bit per OFDM symbol as baseline for evaluation of OFDM based OOK waveforms.

Proposal 6	For M-bit OOK generation using waveform fitting or DFT-based approach, consider mapping of generated frequency domain values (before IFFT) to existing sequences and QAM constellations (e.g., 64-QAM) to minimize the gNB impact.

Proposal 7	For M-bit OOK generation with DFT-s or waveform fitting (i.e., least square), the PAPR aspects should be studied.

Proposal 8	WUR should support multi-bit ADC (e.g., bit width 4 or 8) to improve its sensitivity.

Proposal 9	Consider WUS with small payload (e.g., one bit or few bits) triggering legacy paging procedure as baseline for the study.

Proposal 10	For evaluation of RRM measurement impact on UE energy consumption, serving cell measurements are considered.

Proposal 11	RRM measurements by LP-WUR using existing OFDMA based signals (SSB) should be considered.

Proposal 12	Feasibility for LP-SS periodicity should be determined from, at least, RRM measurement considerations, UE distribution over paging frames, and synchronization requirements.

## R1-2303808_Lenovo.docx

Proposal 1: Evaluate candidate waveform such as MC-OOK and FSK for target data rate and latency requirement for LP-WUS transmission/reception

Proposal 2: Evaluate the relative coverage loss of the LP-WUR compared to main receiver and methods to recover the coverage loss should be studied

Observation 1: Guard band is required when LP-WUR and MR are deployment in the same NR frequency bands

Proposal 3: Consider LP-WUS guard band size 1MHz and other values such as 2MHz are FFS

Proposal 4: Consider same BW for LP-WUS for idle, connected and inactive modes

Proposal 5: Study various LP-WUR deployment options

- LP-WUR and MR are in the same NR frequency band

- LP-WUR and MR are in different NR frequency band

- LP-WUR deployed in the guard band

Proposal 6: Study if the Guard band is required when LP-WUR and MR are deployed in different frequency bands

Proposal 7: The procedure upon detecting WUS can consider the design complexity of the LP WUS (e.g., payload, coverage), latency effects and synchronization precision from LP-WUR.

Proposal 8: LP-WUR performs serving cell measurement from LP-SS while wakes up the MR for cell reselection

# Observations extracted from LLS

[1]

- The performance of conventional OOK LP-WUR can be improve by ~2.5 dB by switching the LP-WUS bandwidth from Option 2 (2.88 MHz) to Option 1 (4.32 MHz).

- Envelope IF signal generation/reception has a ~1.5 dB performance improvement over conventional OOK LP-WUR with the proper selection of IF.

- Envelope IF signal generation/reception can support higher bit rate, i.e., using Option FSK-3, than conventional OOK LP-WUR without a considerable impact on performance.

[2]

- Options OOK-1 and OOK-2 experience marginal performance loss when ADC sampling rate is reduced to 960 kHz. Option OOK-4 sees 2 dB performance loss when sampling is reduced from 1.92MHz to 960kHz.

- For Option OOK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.5, 1.6 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 1.4 dB respectively.

- For Option OOK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.2, 0.4, 0.5 dB respectively.

- For Option OOK-4 with channel model of TDL-C 300ns, the performance loss caused by 1, 2 μs timing error are 2.5, 4.8 dB respectively. Timing error of 4 μs causes error floor. For channel model of TDL-C 1000ns, the performance loss caused by 1 μs timing error is 2.7 dB. Timing error of 2 μs causes error floor.

- If the guard bands can cover frequency offset, Options OOK-1, OOK-2 and OOK-4 are insensitive to frequency offset.

- ZC sequence-based generation of FSK/OOK waveform outperforms significantly the OOK modulation with rectangular waveform in fading channel.

- For OOK-4, concentrated OOK provides0.5dB gain for ideal timing case compared with the normal sequence-based waveform because ISI between OOK symbols are mitigated

- Concentrated OOK significantly improves robust against timing error, e.g. at least 2dB improvement under 2us timing error.

- Concentrated waveform is applicable to OOK and FSK to improve their respective performance.

- Both FSK-1 and FSK-2 sees no performance loss when ADC sampling rate is reduced to 960 kHz.

- For Option FSK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 2 dB respectively.

- For Option FSK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.5, 1.7 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.6, 2.4 dB respectively.

- If the guard bands can cover frequency offset, Options FSK-1, FSK-2 are insensitive to frequency offset.

- For sequence-based detection, it can be observed that the performance would be degraded by 0.5dB due to the phase noise and I/Q imbalance when 120 uW oscillator is used.

- Sequence-based OOK/FSK modulation has better coverage performance than energy detection, even if phase noise and I/Q imbalance, frequency error within 10ppm and 1us timing error are considered.

[3]

- Both OOK-2 and FSK-1/2 based LP-WUS with small M can provide workable link performance under the used simulation assumption, at least with 24 PRB in 14 symbols .

- When the value of M is high, in higher SNR, FSK-1/2 can provides better BLER performances than OOK-2 and FSK-1 has best performances among them because of the usage of relative comparison.

- In higher SNR region, regardless of M , BER 1-to-0 of OOK-2 is better than that of FSK-1/2. This is because energy per 1 segment of OOK-2 is higher than FSK-1/2.

- Regarding FSK-1/2, FSK-1 has better BLER/BER performance frequency usage efficiency than FSK-2 under the conditions of same transmit bits and LP-WUS bandwidth.

- OOK with fixed threshold is more susceptible to frequency errors than FSK.

- FSK-1 has more tolerance to frequency error than FSK-2 and OOK.

- Target Oscillator max frequency error should be clearly defined and optimal design of filter passband and GB for it can be studied.

[5]

- For sequence-based OFDM signal, e.g., ZC sequence, the detection performance is more sensitive to frequency error compared with OOK waveform.

- For LP-WUS structure with preamble using OOK waveform, the preamble with at least 16 chips can achieve reliable synchronization performance.

[8]

- OOK-1 brings better link performance than OOK-2 due to the high frequency diversity in OOK-1.

- The SCS has negligible impacts on the link level performance due to the frequency diversities remains the same within the BW of LP-WUS regardless of the SCS used.

[11]

- To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

[12]

- The performance of multi-bit OOK degrades significantly in comparison with that of AWGN channel due to multi-path effect as expected.

- Oversampling at the LR is essential to improve the performance of OOK reception and to reduce the probability of false alarm.

- Timing accuracy requirement for OOK detection is quite restrictive, which can only be achieved by using a preamble associated with each LP-WUS payload if LP-WUS synchronization has long periodicity.

- The LP-WUS modulation should be resource efficient accounting need for possible guard bands, device BW restrictions and efficient multiplexing with other LP-WUS and other legacy signals.

- At least few bitwidth for ADCs, i.e., 4 or above, should be considered for evaluation as it benefits both the LR and the network power saving target.

- FSK receivers are nothing but parallel OOK receivers. Thus the total power consumed by the LR may be doubled in most of the implementation. However, if FSK based LP-WUS signal uses same SCS as NR transmission, then it can be easily received and decoded if in case MR monitors LP-WUS under more stringent radio conditions when LR is not applicable.

- Due to the narrowness of ON segment in FSK-1/2 schemes, they may suffer from fading, crystal stability of LR, and can also cause PAPR problems at the gNB, if the total transmit power is allocated to a narrow section within LP-WUS BW

- LP-WUS using sequence-based CP-OFDM achieves better coverage and performance than the coded transmission. Multiple sequence or sequence properties can used to increase the information rate.

[13]

For Option OOK-4, based on the restriction of constant transmitting energy within LP-WUS payload transmission,

  - For the same code rate, the priority of value of M is (M=4) > (M=8) > (M=2);

  - For the same value of M, the priority of Manchester code rate is (Code Rate=1/2) > (Code Rate=1/4)

- LP-WUS detection performance with ADC bits number >= 4 is very close to that of Ideal ADC.

- For LP-WUS with bandwidth of 4.32MHz (i.e.,12 RBs with SCS=30KHz),

  - LP-WUS detection performance with sampling rate=3.84MHz has a significant gap to that of No downsampling;

  - LP-WUS detection performance with sampling rate=15.36 or 7.68MHz is very close to that of No downsampling.

- For LP-WUS detection, guardband of 11SCs can mitigate up to 3dB ACI.

- For LP-WUS detection, Guardband=24SCs@ACI=10dB has only 3dB performance loss at BLER=1% compared with the case of Guardband=11SCs@ACI=0dB.

- Under the restriction of constant time-frequency resources and transmitting power, assuming No Time/Frequency error,

  - For Option OOK-1, since CP is added for each OOK symbol, ISI caused by multi-path delay of fading channel can be solve and the performance of the Option OOK-1 is slightly better than that of the Option OOK-4.

  - For Option OOK-2, since the bandwidth of LP-WUS is obviously reduced, the envelop performance will be obviously affected. Therefore, LP-WUS detection performance will be degraded obviously.

- For LP-WUS with Option OOK-1, OOK-2 and OOK-4, the capability of mitigating impact of frequency error is, Option OOK-1 > Option OOK-4 > Option OOK-2.

- Under the restriction of constant time-frequency resource allocation and same transmitting power, if no time/frequency error is assumed, the MDR of longer LP-WUS preamble sequence and shorter LP-WUS preamble sequence are nearly the same.

- For MC-FSK with parallel homodyne receiver architecture, if M>=2, MC-FSK detection performance will be degraded obviously.

- It is observed that the link level performance at BLER=10% based on receiver 1 and receiver 2 is almost same for the case of the ZC sequence length of 23 under ideal cases.

- For ZC sequence-based OFDM, the link level performance is not impacted by the aspect of sequence correlation in frequency domain or in time domain.

- It is observed that the SNR values at BLER=10% based on OFDM receiver 1 without down sampling and with down sapling factor of 1/4 are almost same for the case of ZC sequence length of 23.

- It is observed that the SNR values at BLER=10% based on receiver 1 decreases with the increase of the length of ZC sequence of 23, 47 and 83.

- The link level performance is seriously impacted by the aspect of frequency offset for sequence-based OFDM receiver.

[15]

- Demodulation performance over 6RB bandwidth no longer offers significant performance gain.

[19]

- Shows that at 200ppm frequency error OFDMA signal (PSS) with non-I/Q OOK receiver can be received with the same performance as MC-ASK. MR may help with preprocessing of PSS to look like OOK.

[20]

- In AWGN, decreasing the number of SCs of OOK symbols improves performance because less noise if captured.

- OOK-2 cannot fully exploit frequency diversity because OOK symbols are confined to a subset of the WUS bandwidth.

- Given the same bandwidth, OOK-1 is more robust to ACI than OOK-4 because the interference is averaged over more samples.

- Given ideal AGC, an ADC with 4-bit resolution is sufficient for close to optimal performance.

- For OOK-1, Manchester coding improves robustness to timing inaccuracies.

- OOK-4, shorter the OOK symbols increase sensitivity to timing inaccuracies.

- OOK-3 has good performance, but requires low frequency error.

[24]

- at the same data rate, OOK (with Option OOK-4) is more robust against frequency error than FSK (with Option FSK-1), and FSK is more robust against timing error than OOK.

- For MC-OOK (with Option OOK-4) and MC-FSK (with Option FSK-1), the sampling rate at the receiver needs to be set according to the bandwidth of the effective signal, which can be much larger than the information/data rate of the OOK/FSK transmission.

- at the same data rate, MC-OOK (with Option OOK-4) is more robust against frequency domain selectivity (i.e., having a larger frequency diversity gain) than MC-FSK due to larger effective bandwidth.

[27]

- For modulating ON symbols in OOK WUS, a sequence and random QPSK provide the same BER performance for the same signal power.

- Adjacent cell interference can result in a significant coverage degradation when the interference is stronger than the signal.

- Increasing ADC bit width is beneficial in terms of coverage, interference mitigation, and dynamic range of signal reception.

- The coverage performance of single-bit ADC is extremely poor.

- UE speed has a minor impact on the link performance considering a relatively large channel coherence time.

- For non-coherent WUS detection, every repetition factor of 2 improves the link performance by around 2 dB.

- For payload-based OOK WUS a relatively large number of WUS repetitions might be needed to match the PDCCH coverage. The total WUS duration with repetition is in order of tens of slots

- For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).  [27]

- Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.