3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2304041

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.8

Source:	Moderator (Ericsson)

Title:	Moderator Summary#1 – PDCCH monitoring resume after UL NACK

Document for:	Discussion, Decision

# 1	Introduction

In RAN plenary 98-e, the Rel-18 WI on eXtended Reality (XR) was agreed and was further revised in RAN#99, with the following objectives:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Note that in addition to these objectives, it was agreed that two additional power saving enhancements can be discussed in working groups:

- PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts (RAN1, see e.g. [3]); and

- CG without retransmissions for uplink XR traffic (RAN2, see e.g. [4]).

With respect to the first power saving enhancement scheme above based on PDCCH monitoring resumption, the following email discussion is assigned by the RAN1 Chair:

[112bis-e-R18-XR-02] Email discussion on XR-specific capacity enhancements by April 26 – Sorour (Ericsson)

- Check points: April 21, April 26

This documents summarizes the inputs in contributions and facilitate the corresponding discussion.

# 2	Background

## 2.1	Proposal and summary of views

Proposal: Resume PDCCH monitoring if UE transmits NACK after PDCCH skipping starts.

With respect to this enhancements, proponents in [1], [2], [3], [4] support the proposed enhancement, while [5] does not support introduction of this enhancement.

- Support ([1]-[4]): vivo, MediaTek, Google, Qualcomm, Panasonic, Meta, Ericsson, China Unicom, Huawei, HiSilicon, Xiaomi, Apple, China Telecom, ZTE/Sanechips, Nokia/NSB

- Not Support ([5]): CATT

## 2.2	Motivations

### Reason to support the enhancement ([2])

In Rel-17, the PDCCH skipping feature has been introduced under the power saving enhancements. The UE can be indicated to skip monitoring for a duration by the scheduling DCI format X-1/X-2. Although this Rel-17 feature was introduced for power saving purposes, it also impacts scheduling latency especially for retransmission requests. Figure 1. Impact of PDCCH skipping on DL retransmission However, in the current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. As shown in Figure 1, if a PDSCH is not decoded successfully right before the PDCCH skipping, the retransmission would be delayed until the PDCCH skipping duration is ended. This brings in severe impact to services with low latency requirement, e.g., XR and URLLC, where instant HARQ retransmission is critical.

### Reason to not support the enhancement ([5])

Observation 1: The PDCCH skipping scheme enhanced with continuous PDCCH skipping and go-to-sleep indication can achieve 29.4% power saving gain with negligible capacity degradation compared with that of always-on. Observation 2: PDCCH skipping interaction with HARQ scheme with short skipping duration can hardly provide any power saving with 7.1% power saving gain over that of always-on. Observation 3: Although the longer skipping duration of PDCCH skipping can provide similar or slightly higher UE power saving with 31.5% power saving gain comparing to that of PDCCH skipping enhancement with MAC-CE based go-to-sleep signaling, the XR capacity performance would suffer significant loss. In RAN#112, some companies proposed PDCCH monitoring adaptation schemes [2] with the evaluation results of enhanced PDCCH skipping with HARQ interaction scheme showing relatively good performance in UE power saving and system capacity. However, the proposed scheme does not consider the SSSG switching delay and the addition power consumption for detecting SSSG switching indication. The MAC-CE based go-to-sleep signaling scheme considers the 3ms processing delay after HARQ-ACK transmission and provide 19.3%~20.8% power saving gain by terminating the drx-onDuartion Timer and drx-Inactivity Timer for UE to go to sleep mode directly. Proposal 1: PDCCH skipping scheme with resuming PDCCH monitoring after NACK provides only negligible UE power saving gain with short skipping duration and have severe degradation in system capacity for longer skipping duration. PDCCH skipping scheme with resuming PDCCH monitoring after NACK should not be supported.

## 2.3	Proposed draft CR for the enhancement

### 2.3.1	TP1 ([2] vivo, et. al)

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell, and additionally according to the procedures described in [11, TS 38.321] if DRX is configured. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. *** Unchanged text is omitted ***

### 2.3.2	TP2 ([3] ZTE/Sanechips)

10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. If the UE transmits a NACK after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE terminates PDCCH skipping for the serving cell. <Unchanged parts are omitted>

### 2.3.3	TP3 ([4] Nokia/NSB)

## 10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged part omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. If UE transmits PUCCH or PUSCH providing a NACK on the serving cell, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, the UE resumes PDCCH monitoring at the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH providing the NACK on the SpCell. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. <Unchanged part omitted>

# 3	Discussions

Moderator’s note regarding TP1:

The highlighted blue text above was added due to an offline discussion before submission as explained by [1]. However, after the submission and further offline discussion, it was clarified that after NACK the UE is intended to start monitor any PDCCH, and not start monitoring with PDCCHs that schedule a retransmission.

Hence, from source [1] point of view, considering that clarification, the highlighted blue text is no further needed. Therefore, Moderator suggests considering TP1 without the highlighted blue text.

## 3.1	Initial Discussions

### 3.1.1	Questions and Answers

#### Question 1:

Please indicate your view whether the proposes enhancement should be supported-

| Company | Comment |
| --- | --- |
| vivo | We support the proposed enhancement. The reason to support the enhancement has been discussed, i.e., the current PDCCH skipping feature is difficult to apply with HARQ retransmission, especially for XR services with stringent latency and reliability requirement. In addition, performance of the proposed enhancement was evaluated by SLS in R1-2212518 and it can be seen that performance benefits can be achieved. Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Notes. Always On; -; -; -; 10; 10; 92.50%; -; Note1. -; -; -; 10; 10; 97.45%; -; Note1,7. -; -; -; 10; 10; 93.12%; -; Note1,6. -; -; -; 10; 10; 97.45%; -; Note1,6,7. -; -; -; 10; 10; 91.83%; -; Note2. R17 PDCCH monitoring adaptation; 16.67; 16; 4; 10; 10; 91.05%; 12.26%; Note2,4,5. 16.67; 8; 4; 10; 10; 1.11%; 35.21%; Note1,3,5. 16.67; 8; 4; 10; 10; 2.22%; 35.21%; Note1,3,5,7. 16.67; 8; 4; 10; 10; 2.08%; 32.77%; Note1,3,5,6. 16.67; 8; 4; 10; 10; 94.91%; 32.77%; Note1,3,5,6,7. 16.67; 8; 4; 10; 10; 92.22%; 19.28%; Note1,4,5. 16.67; 8; 4; 10; 10; 97.45%; 19.28%; Note1,4,5,7. 16.67; 8; 4; 10; 10; 92.78%; 19.48%; Note1,4,5,6. 16.67; 8; 4; 10; 10; 97.45%; 19.48%; Note1,4,5,6,7. Enhanced PDCCH skipping with HARQ interaction; 16.67; 8; 4; 10; 10; 92.22%; 32.18%; Note1, 3,5. 16.67; 16; 4; 10; 10; 92.20%; 23.67%; Note2, 3,5. 16.67; 8; 4; 10; 10; 97.45%; 32.18%; Note1, 3,5,7. 16.67; 8; 4; 10; 10; 92.78%; 32.49%; Note1, 3,5,6. 16.67; 8; 4; 10; 10; 97.45%; 32.49%; Note1, 3,5,6,7. Note1: jitter range = [-4, +4]ms, STD = 2ms Note2: jitter range = [-8, +8]ms, STD = 5ms Note3: the network indicates PDCCH skipping in the DCI that schedules the initial PDSCH transmission of the last DL packet of an XR traffic burst Note4: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note5: applying R17 sparse SSSG with PDCCH monitoring every 2 slots when DRX Onduration starts and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note6: initial BLER is reduced from 10% to 1% Note7: satisfaction metric as 95% packet successful rate. |
| Panasonic | We support the proposal. |
| Futurewei | Based on performance impacts on XR traffic, we support the proposed enhancement. |
| ZTE, Sanechips | We support the proposal. |
| CATT | CATT’s contribution in R1-2302676 [5] showed that the UE power saving gain will be degraded with short skipping duration. The short skipping duration, e.g., 3 slot, is commonly used for the periodic traffic, such as XR traffic, with network delay jitter. We also showed that long skipping duration of PDCCH skipping would degrade the system capacity. Regarding the XR performance shown in R1-2212518 only indicate the performance in a few selected cases without the full system capacity (XR system capacity is more than 10 UEs per cell). We can NOT agree with the proposal to have inferior UE power saving performance for XR. |
| Qualcomm | We support the proposed enhancement that UE stops PDCCH skipping if UE transmits a NACK due to a failed PDSCH decoding. This is a fix to the misalignment issue that when gNB triggers UE PDCCH skipping, it may not know there is a pending retransmission for a latency sensitive traffic. The proposal directly addresses the issue with a simple solution. |
| Xiaomi | We support the proposal. |
| Samsung | We support the proposal. |
| Spreadtrum | We think it is common understanding that a NACK value in the proposal only refers to a NACK due to a failed PDSCH decoding. However, there are some NACK value are generated due to padding, such as only one CW is scheduled when two CWs are allowed, scheduled CBG number is less than maximum CBG number in a TB, no PDSCH is scheduled when Type-1 HARQ-ACK codebook, etc. So a safer solution is NACK value only limited to the following two cases in section 9.1 and 9.1.1 as the blue words blow. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value if the UE does not correctly decode the transport block/CBG according to clause 9.1, 9.1.1, 9.1 HARQ-ACK codebook determination For a HARQ-ACK information bit, a UE generates a positive acknowledgement (ACK) if the UE detects a DCI format that provides a SPS PDSCH release or detects a DCI format that does not schedule PDSCH reception and indicates a TCI state update or correctly decodes a transport block, and generates a negative acknowledgement (NACK) if the UE does not correctly decode the transport block. A HARQ-ACK information bit value of 0 represents a NACK while a HARQ-ACK information bit value of 1 represents an ACK. 9.1.1 CBG-based HARQ-ACK codebook determination The UE generates an ACK for the HARQ-ACK information bit of a CBG if the UE correctly received all code blocks of the CBG and generates a NACK for the HARQ-ACK information bit of a CBG if the UE incorrectly received at least one code block of the CBG. If the UE receives two transport blocks, the UE concatenates the HARQ-ACK information bits for CBGs of the second transport block after the HARQ-ACK information bits for CBGs of the first transport block.. |
| vivo2 | On CATT’s comment. In R1-2212518, the evaluations are performed using the evaluation methodology and assumptions in Rel-17. Please note that for “AlwaysOn” case, the satisfied UE ratios are close to 90%, which show that these cases are full system capacity. In addition, the proposed enhancement are also evaluated by other companies and are showing performance benefit, as captured in B.2.11 in TR38.835 (Rel-18 SI TR). On Spreadtrum’s comment. Regarding Spreadtrum’s comment to improve the TP (related to Question 2?), we think it is typical case that when a NACK for a PDSCH decoding is sent, UE starts to monitor PDCCH. However, UE may send NACK due to failure of decoding PDSCH or failure of detecting the scheduling DCI. So we think it is better to keep it as general. |
| LGE | We agree to address the issue Qualcomm and Spreadtrum mentioned that NACK can be generated due to padding. Also, we have to consider that HARQ-ACK codebook includes multiple ACK/NACKs across multiple cells. As PDCCH monitoring adaptation indication is only applied to individual scheduling cell, only NACK for the PDSCH scheduled from the scheduling cell indicated with PDCCH skipping is able to trigger cancellation of the PDCCH skipping indication. In other words, NACK for other PDSCH scheduled from other cell should not trigger the cancellation of the PDCCH skipping indication. |
| OPPO (update) | 1st comment: We share the views from Spreadtrum and LGE that the NACK transmission in the current spec may refer to something other than PDSCH failure. Taking Type-1 CB as an example, NACK value is set for the PDSCH with k1 corresponding to the other slot, which will lead to unnecessary resuming of PDCCH skipping if the CR is adopted. ### 9.1.2 Type-1 HARQ-ACK codebook determination A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release or TCI state update only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format as described in clause 9.2.3. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.. 2nd comment: In the new sentence: “If UE transmits PUCCH or PUSCH providing a NACK on the serving cell, after the UE detects a DCI format ...”, the wording “after” here seems to say the method in the CR does not guard the failed PDSCH reception in a case, where: - The UE sends a NACK in slot n for a PDSCH (#1) received in slot n-k; and - The UE receives a PDCCH triggering PDCCH skipping also in slot n (assuming FDD). This triggering PDCCH schedules another PDSCH (#2), which succeeds at UE reception. According to the CR, PDCCH monitoring would not resume (as NACK does not occur “after” triggering PDCCH), and PDSCH #1 cannot be recovered until end of PDCCH skipping. In fact, any PDSCHs having NACK transmitted no later than PDCCH skipping triggering but having their HARQ re-transmission not scheduled before PDCCH skipping triggering has a risk of not being recovered until end of PDCCH skipping. 3rd comment: For the issue targeted by the CR, the earlier RAN1 discussion once gave a work-around solution, which lets the PDCCH that triggers PDCCH skipping to schedule dummy PDSCH and this PDCCH is transmitted when gNB knows all pending PDSCH’s are all received and acknowledged by UE. However, the method proposed by the CR seems to make this work-around method partly problematic, since a NACK on dummy PDSCH may just delay the power saving procedure for nothing. If the proposed solution would somehow disqualify an existing work-around, it is better to be solid and sound, rather having issues as mentioned in earlier two comments. |
| vivo3 | On LGE’s comment UE may send NACK due to failure of decoding PDSCH or failure of detecting the scheduling DCI. Besides, in case cross-carrier scheduling, PDCCH skipping is applied to the scheduling cell. Therefore, NACK corresponding to the PDSCH from scheduled cell would trigger PDCCH monitoring resumption in the scheduling cell. On OPPO’s comment2 The CR is intended to address PDCCH monitoring resumption if NACK is transmitted after PDCCH skipping indication. For the case mentioned in you comment, it seems not the case that UE transmit NACK after PDCCH skipping indication. Besides, in such case, if gNB would send a PDCCH skipping indication, UE can follow gNB’s indication to skip PDCCH monitoring, or if gNB can trigger PDCCH skipping in next slot. On OPPO’s comment3 Without the proposed enhancment, gNB may trigger PDCCH skipping by scheduling a dummy PDSCH after ACK. If the proposed enhancement is adopted, there is no need to send PDCCH skipping by DCI scheduling dummy PDSCH. |
| TCL | We support the proposal. |
| Panasonci2 | This proposal addresses the situation of the last DL packet to be sent to UE as other than last DL packet situation would be multiple HARQ process occupies the timeline. So for eMBB of relatively larger data packet with relatively infrequent transmission, we don't see the need of this proposal (although it can be used if the CR can be agreed). For XR traffic of relatively smaller pacekt with more frequent transmision, the situation of the last DL packet to be sent to UE can be more frequent. Therefore, we see the merit of the propsal for XR. |
| MediaTek | We support the proposal. |
| Huawei/HiSilicon | We support the proposal. |
| Nokia1 | We support introducing the enhacement. |
| Intel | We do not think the enhancement is critically needed. It is expected to result in less power saving than when PDCCH skipping is not interrupted. For latency critical services, short PDCCH skipping duration such as 2ms or 4ms can be used. It is gNB implementation to ensure appropriate configuration is applied for a given service. From our perspective, higher power saving gain is expected to be a necessary condition to introduce any power saving enhancement. |
| Ericsson | We support the proposal. Regarding NACK, our understanding from the relate doflfine discusson prior to the meeting is that the NACK, refers to any NACK. Regaridng resuming PDCCH monitoring, if refers to any PDCCH after NACK in the nest slot, and not the PDCCH scheudulign retransmisison. |
| Moderator | Summary of views: - Support: vivo, MediaTek, Google, Qualcomm, Panasonic, Meta, Ericsson, China Unicom, Huawei/HiSilicon, Xiaomi, Apple, China Telecom, ZTE/Sanechips, Nokia/NSB, Futurewei, Samsung, TCL - [Spreadtrum], [LG], [OPPO] - Not Support : CATT, Intel Summary of discussion: Reasons for not suppoting (CATT, Intel): - CATT: Concern to cause inferior power saving performance for XR referring to evalautions in R1-2302676. Also questioning evaluation results showing gain. - Intel: Not critical enhancement. For latency critical short skipping duraiton can be used. Reasons to support (in responce): - QC/Panasonic: Pending retransmission not useful for latency critical services (when PDCCH skipping is triggered, gNB may not know of the need for retransmisison) - Vivo: Showing power saving perofrmance gain for XR. Explain Rel-17 assumption and „always on“ is used. Also similar results by other companies evalautions captured in in B.2.11 in TR38.835 (Rel-18 SI TR) showing benefits. Discussion on TP: - Clairficaiton on NACK - Spreadtrum, LG, OPPO, - vivo (in responce) |

#### Question 2:

Please indicate your view that if the proposes enhancement were supported, which TP in section 2.3 is preferred. Please include any suggestion to improve the TP. Please consider Moderator’s note regarding TP1 in your comment.

| Company | Comment |
| --- | --- |
| vivo | We support FL’s assessment that after NACK the UE is intended to start monitor any PDCCH, and not start monitoring with PDCCHs that schedule a retransmission. In fact, the proposed TP is intended to introduce the behavior to start to monitor PDCCH after NACK as per MAC behavior. Therefore, we support TP1 without the highlighted blue text. |
| Panasonic | Our thinking is following. - the proposal intends to just to finish „PDCCH skipping“ at „the beginning of a first slot that is after a last symbol ft he PUCCH or PUSCH transmission in the serving cell“. - Therefore, - if „the beginning of a first slot that is after a last symbol ft he PUCCH or PUSCH transmission in the serving cell“ is active time (e.g. OnDuration timer or retransmission timer is running), resume PDCCH monitoring immediately - if „the beginning of a first slot that is after a last symbol ft he PUCCH or PUSCH transmission in the serving cell“ is not active time (e.g. OnDuration timer is expired and RTT timer is running)), PDCCH monitoring is paused until the start of retransmission timer (= RTT timer is expired). This means PDCCH monitoring is paused until next active time. Based on above, we think „the UE resumes PDCCH monitoring“ is not proper wording as it can imply the contradiction with MAC specification. As suggested offline, „UE terminates PDCCH skipping …” would be better wording as it describes only RAN1 part ft he behaviour. |
| Futurewei | As assessed by FL “it was clarified that after NACK the UE is intended to start monitor any PDCCH, and not start monitoring with PDCCHs that schedule a retransmission”, which means that UE doesn’t resume PDCCH monitoring, e.g., before drx-HARQ-RTT-TimerDL expires, the blue text is not necessary in that sense. We support TP1 without the highlighted blue text. |
| ZTE, Sanechips | We support to remove the blue text. I see people don’t intend to cause confusion with MAC layer impact. I also share the clarifications/understanding from Panasonic and prefer to merge the wording “UE terminates PDCCH skipping..” into TP1 if the change can address the concern. |
| CATT | No CR is needed for the inferior proposal to degrade the UE power saving performance. |
| Qualcomm | Technically the UE only needs the rescheduling DCI for the PDSCH retransmission. The proposal is hence not fully optimized for power saving but it is simple for implementation. The unrealized power saving could be negligible though. We can support the majority view. Also agreed with Panasonic and ZTE that “UE terminates/cancels/stops PDCCH skipping…” (as if the UE never receives the PDCCH skipping indication) could be better than “UE resumes PDCCH monitoring…”. |
| Xiaomi | Agree with Panasonic that „the UE resumes PDCCH monitoring“ is not proper wording as it can imply the contradiction with MAC specification. As suggested offline, „UE terminates PDCCH skipping …” would be better wording as it describes only RAN1 part ft he behaviour, if the blue text in TP1 should be removed. |
| Samsung | OK with the CR including the blue text as it is anyway the MAC that determines PDCCH monitoring in case of DRX. A somewhat better wording may be: and, if DRX is configured, additionally according to the procedures described in [11, TS 38.321] if DRX is configured |
| vivo2 | We are also fine to change the wording to “UE terminates PDCCH skipping …”, thus only focusing on the RAN1 part ft he behavior. |
| LGE | The condition of NACK triggering to resume PDCCH monitoring (e.g. the NACK is based on PDSCH decoding and it corresponds ft he PDSCH scheduled from the scheduling cell indicated with PDCCH skipping) should be addressed. |
| OPPO | Same view as LGE. |
| Vivo3 | On LGE’s comment UE may send NACK due to failure of decoding PDSCH or failure of detecting the scheduling DCI. Besides, in case cross-carrier scheduling, PDCCH skipping is applied ft he scheduling cell, i.e., NACK corresponding ft he PDSCH from scheduled cell would trigger PDCCH monitoring resumption in the scheduling cell. In ft heo cover all these cases for the condition of NACK to trigger PDCCH monitoring resumpition, using general description as in current TP1 could be sufficient. However, if there is any better wording for the TP, we are also fine to consider. |
| TCL | Similar view as Panasonic. |
| MediaTek | Based on moderator’s note, we support TP1 without the highlighted blue text. |
| Huawei/HiSilicon | We share similar view with Panasonic that “the UE resumes PDCCH monitoring…” may cause some confusion. We suggest the following TP: “… the UE terminates PDCCH skipping, starting from the beginning of a first slot that is after a last symbol ft he PUCCH or PUSCH transmission in the serving cell.” |
| Nokia1 | Firstly, we fine to consider RRC parameter for this functionality, as in TP1, accounting/assuming that UE terminates the skipping fully, not just for the duration of drx-RetransmissionTimerDL. On the functionality, while both wordings have been used in 38.213, it would seem best to use the wording ‘terminate skipping’ (as per TP#2) rather than ‘resumes PDCCH monitoring’ (as per TP#1/#3), to avoid any risk of confusion with MAC (if any). We somewhat prefer (for clarity) that the timing (e.g. ‘at the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH providing the NACK’) when UE resumes/terminates the skipping is defined as in TP#1/#3. On overall slightly leaning towards wording as in TP#1/#3. Then on the matter of scheduling cell, I think this would need to be considered bit furhter in the wording. I understood from my colleague that that in Rel-17 SCell scheduling PCell and PCell scheduling itself is possible. Thus it would make sense from network perspective, if UE would terminate skipping on both/all of the candidate scheduling cells to maintain the scheduling flexibility. In relation to the RRC parameter to enable/disable this functionality, wonder if we should also consider the need of UE capability or can this be assumed to be mandatory for all the Rel-18 UEs supporting PDCCH skipping? |
| Intel | Please see the response in Q.1. |
| Lenovo | If the proposed enhancement to be supported, disabling PDCCH skipping should be RRC configured. |
| Ericsson | Regarding TP: We agree to use RRC parameter, and avoid using <resume> Regaridng Nokia’s comment: our understanding is that this CR is per cell that PDCCH is configured and can be skipped. Good comment from Nokia whether this feature should be associated to a separate capability. |
| Moderator | Comments on TP if the enhancement is supported: - Use “UE terminates PDCCH skipping“ instead of “UE resumes PDCCH monitoring“: OK with companies - Remove blue text in TP1: OK with companies. However, Samsung commented to keep and update the text. - Check if Samsung OK to remove in updated TP by Moderator - Clarifcaiton of NACK: LGE, OPPO, [Spreadtrum] to clairfy NACK correpsonds to scheuled PDCCH. Others clarified not nessesary. - Check if OPPO, LG, Spreadtrum OK w updated TP by Moderator - RRC parameter in TP1: OK to consider (Nokia, Lenovo) - Need for UE capability? (Nokia) - Check companies‘ view - Not supporting the TP (as in Q1): CATT & Intel Updated TP1 based on comments (Check companies view): When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumesterminates PDCCH monitoring, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell, and additionally according to the procedures described in [11, TS 38.321] if DRX is configured. |

### 3.1.2	1st Online discussion

Discussion points for online session:

- Whether the proposed feature is supported (Proposal 1)

- Whether the proposed TP reflects the proposed feature in case the feature is supported (Proposal 2)

  - Check if concern on “NACK” is resolved

- Whether an RRC parameter and a UE capability corresponding to the feature should be introduced in case the feature is supported (Proposal 2)

Proposal 1: Resume PDCCH monitoring if UE transmits NACK after PDCCH skipping is started.

- Support: vivo, MediaTek, Google, Qualcomm, Panasonic, Meta, Ericsson, China Unicom, Huawei/HiSilicon, Xiaomi, Apple, China Telecom, ZTE/Sanechips, Nokia/NSB, Futurewei, Samsung, TCL

  - [Spreadtrum], [LG], [OPPO]

- Not Support : CATT, Intel

Proposal 2: For support of the feature “to resume PDCCH monitoring if UE transmits NACK after PDCCH skipping is started”:

- The following TP for TS 38.213 is endorsed.

- A new RRC parameter (e.g., PdcchMornitoringResumptionAfterNack) is introduced.

- An optional UE capability for the feature is introduced.

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE terminates PDCCH monitoring, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. *** Unchanged text is omitted ***

Proposal 3:

# 4	Conclusion

TBD

# References

| 1 | R1-2302398 | PDCCH monitoring resumption after UL NACK | Ericsson |
| --- | --- | --- | --- |
| 2 | R1-2302500 | Draft CR on PDCCH monitoring resumption after NACK | vivo, MediaTek, Google, Qualcomm, Panasonic, Meta, Ericsson, China Unicom, Huawei, HiSilicon, Xiaomi, Apple, China Telecom |
| 3 | R1-2302946 | Discussion on PDCCH monitoring resumption after reporting NACK | ZTE, Sanechips |
| 4 | R1-2303312 | Draft CR for Introducing PDCCH monitoring resumption after UL NACK | Nokia, Nokia Shanghai Bell |
| 5 | R1-2302676 | PDCCH Skipping interaction with HARQ | CATT |