3GPP TSG-RAN WG1 Meeting #112-bis-e		R1- 23xxxxx

Online, April 17th – April 26th, 2023

Agenda item:	9.2.1

Source: 	Moderator (Qualcomm)

Title: 	Summary#1 of General Aspects of AI/ML Framework

Document for:	Discussion/Decision

# Introduction

In this contribution, we summarize issues regarding AI/ML general aspects agenda in RAN1 #111.

## Contact information

Please provide/update your contact information.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (Qualcomm) | Taesang Yoo | taesangy@qti.qualcomm.com |
| vivo | Peng SUN | sunpeng@vivo.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| OPPO | Jia SHEN | sj@oppo.com |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Futurewei | Chunhui (Allan) Zhu | chunhui.zhu@futurewei.com |
| Ericsson | Henrik Ryden | henrik.a.ryden@ericsson.com |
| NTT DOCOMO | Haruhi Echigo | haruhi.echigo.fw@nttdocomo.com |
| NEC | Peng Guan | guan_peng@nec.cn |
| LG Electronics | Jiwon Kang | jw.kang@lge.com |
| Lenovo | Jianfeng Wang Vahid Pourahmadi | wangjf20@lenovo.com vpourahmadi@lenovo.com |
| Mediatek | Yuanyuan Zhang | Yuany.zhang@mediatek.com |
| ZTE | Guozeng, ZHENG | zheng.guozeng@zte.com.cn |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |
| Intel | Bishwarup Mondal | bishwarup.mondal@intel.com |

| AT&T | Salam Akoum Isfar Tariq | Salam.akoum@att.com Isfar.tariq@att.com |
| --- | --- | --- |
| Fujitsu | Wang Xin | wangxin@fujitsu.com |
| Spreadtrum | Hualei Wang | Hualei.wang@unisoc.com |
| ETRI | Hanjun Park Anseok Lee | hjunpark@etri.re.kr alee@etri.re.kr |
| CMCC | Yuhua Cao Yi Zheng | caoyuhua@chinamobile.com zhengyi@chinamobile.com |
| Panasonic | Hidetoshi Suzuki | suzuki.hidetoshi@jp.panasonic.com |
| Nokia | Keeth Jayasinghe | Keeth.jayasinghe@nokia.com |
| Samsung | Ameha | amehat.abebe@samsung.com |
| Intel Corporation | Bishwarup Mondal | bishwarup.mondal@intel.com |
| SEU | Jiajia Guo | jiajiaguo@seu.edu.cn |
| Sony | Hiroki Matsuda | hiroki.matsuda@sony.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Apple | Huaning Niu | huaning_niu@apple.com |
| Rakuten Mobile | Koichiro Kitagawa | koichiro.kitagawa@rakuten.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Fraunhofer | Georgios Kontes Ebrahim Amiri | georgios.kontes@iis.fraunhofer.de ebrahim.amiri@iis.fraunhofer.de |
| Indian Institute of Tech (M) | Jeeva Keshav S Sai Prasanth Anil Kumar Yerrapragada Rohit Singh | jeevak@5gtbiitm.in saiprasanth@5gtbiitm.in anil@5gtbiitm.in rohit@5gtbiitm.in |
| TCL | Jack Ahmed Mikaeil Qu Miao | xiaoyong.tnag@tcl.com ahmed.mikaeil@tcl.com miaoqu.tcl.com |
| China Telecom | Bei Yang | yangbei1@chinatelecom.cn |

# Summary of contributions in RAN1#112

## Definitions of terms, symbols and abbreviations

### Proposals for changes

Fraunhofer:

Proposal 1: Change the Reinforcement Learning (RL) Definition as follows:

Reinforcement Learning (RL): A process of training an AI/ML model (policy) to interact with an environment and take actions (model’s output) based on the environment’s current state (model’s input), with the goal of maximizing the expected cumulative reward (feedback signal). For the AI/ML model (policy) training, direct interaction with the environment, available logged data from the environment, or a combination of both can be used.

### New terminologies

Fraunhofer:

Proposal 2: The following concepts/terms shall be introduced:

- Fault: a specific problem caused by the performance degradation of the AI/ML model. For example, for a beam management model, the RSRP/SINR values of the chosen beams are declining.

- Fault indication: signs that could imply the existence of a fault. For example, a mismatch between the statistics of input data in the AI/ML model during Inference and the training data for the specific AI/ML model, could indicate a problem on the model’s performance.

- Fault type or root cause of a fault: the underlying reason a fault is observed. For example, we have a blockage or reflections in the radio environment and the AI/ML model’s performance degrades, as it is not trained for this.

## General AI/ML Framework

### Description of the stages of Machine Learning

FUTUREWEI:

Model Inference

Model Management (monitor, select, activate, deactivate, switch, fallback)

output

model inference control

inference data

Data Collection

training data

monitoring data

Model Training

model training control

Figure 3. Proposed framework diagram

Proposal 1: Take the functional framework shown in Figure 3 as the starting point for RAN1 functional framework discussion. On top of the functional framework defined by RAN3, this revised framework diagram adds a functional block for model management.

Huawei, HiSilicon:

Figure 1 Functional Framework for RAN Intelligence defined in TR 37.817

Figure 2 Diagram of the general framework for air-interface AI/ML

Proposal 2: Figure 2 can be considered as the diagram for high-level general AI/ML framework.

Spreadtrum:

Figure 2. Functional Framework for AI/ML enabled air interface

Proposal 3: The general framework of Figure 2 can be considered as the AI functional framework for the air interface.

CATT:

Figure 3 Functional framework of AI/ML in NR air interface

Proposal 2: Adopt Figure 1 as the diagram for AI/ML framework in NR air interface.

Proposal 3: Discuss whether to introduce functional framework diagram for functionality-based LCM.

- If introduced, it is preferred to reuse the one for model-ID based LCM as much as possible.

Intel:

Figure 4: Functional framework

Proposal-1: Support a modification of the RAN3 functional framework (37.817) by using components according to the agreed terminology in RAN1.

Ericsson:

- Adopt the functional framework in Figure 1 for anchoring discussions related to NW-UE collaborations.

Figure 5. A functional framework for discussing model LCM aspects.

Xiaomi:

Figure 6 Framework of AI/ML in air interface

LG:

Proposal #1: Adopt the 4 functions and their relation defined in TR37.817 as a starting point for AI/ML functional framework.

- The functional framework may be modified later based on the progress of LCM

Proposal #2: AI/ML model can be categorized based on different scenarios in that which entity (i.e. either UE or NW) has which AI/ML function(s).

CAICT:

Fig.1 Sample of general AI/ML framework

Proposal 1: the general framework presented in Fig.1 could be considered.

CMCC:

Proposal 13: On Rel-18 AI/ML for air interface, whether a new framework based on the functional framework for RAN intelligence is needed can be studied.

MediaTek:

Model Inference

Model Training

Model Monitoring

Model delivery/ transfer

Model Performance Feedback

Output

Training data

Inference data

Data collection

Monitoring data

Model control

Figure 1 Example of Functional Framework for AI/ML over air interface

Proposal 1: The AI/ML functional framework for air interface includes the functions of data collection, model training, model inference and model monitoring. Take Figure 1 as starting point for functional framework discussion.

Nvidia:

Proposal 1: The defining stages of AI/ML related algorithms, including the model generation, e.g., model training (including input/output, pre-/post-process, online/offline, etc.), model validation, model testing, the model inference operation, e.g., input/output, pre-/post-process, and the associated complexity, need to be analysed case by case.

Apple:

Fig 2: Proposed functional framework with performance monitoring

Proposal 10: Use TR 37.817 functional framework as the starting point for RAN1 functional framework discussion.

Proposal 11: Considering additional performance monitoring block in the functional framework.

Lenovo:

Figure 7 A high-level functional framework for the study on AI/ML for NR air interface.

Adopt Figure 1 as the general functional framework, including data collection, model training, model management and model inference.

Qualcomm:

Figure 8: A general AI/ML framework

Proposal 1: Adopt the general AI/ML framework diagram shown in Figure 1.

NEC:

Figure 1 AI/ML Framework Description

Proposal 1: Adopt Figure 1 as starting point for discussion for AI/ML functional framework and discuss the potential enhancements to support different use cases/variations of AI/ML models.

TCL Communication:

features of these three use cases. Hence, it is better to study a new common functional framework of AI/ML for NR air-interface.

- Proposal 1: A new common functional framework of AI/ML for air-interface need to be studied.

Fig.1. Common functional framework of AI/ML for NR air-interface

- Proposal 2: A common functional framework of AI/ML over air-interface may include the following functions: data collection, model training. model inference, model monitoring and actor.

##### Issue 6-1: framework diagram

RAN1 could define a functional framework to identify functional requirements of an AI/ML architecture with adequate AI/ML model characterization, start studying different aspects of LCM and anchor discussions related to NW-UE collaborations. A basic functional framework of AI/ML for NR RAN in TR 37.817 can be considered as a starting point for RAN1 study. One block that RAN1 could add is a model management block endowed with many functionalities such as functionality/model monitoring, selection, activation, deactivation, switching and fallback. The Actor block may or may not be needed. RAN1 may want to add a model storage block to capture model transfer/delivery that was not needed in the RAN3 framework.

###### [FL1] Proposal 6-1a:

Include the following blocks as a starting point for high-level AI/ML framework diagram.

- Data collection

- Model training

- Performance monitoring

- Model inference

- FFS: Actor

- FFS: Model storage

Notes

- A block may be implemented in one or multiple entities, each of which may be 3gpp or non-3gpp entities.

- The interactions between blocks may or may not have impact on 3gpp signaling.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | Panasonic |
| Company | Comments |  |
| vivo | Model delivery/transfer seems not included in the block diagram? |  |
| Google | Support in principle. What does the “actor” indicate? |  |
| Panasonic | Current performance monitoring block has the function of model inference control (activate, deactivate, switch, fallback). It is beyond the performance monitoring. Model management including performance monitoring would be more reasonable. |  |
| LG | Supportive in general. The diagram may need some further refinement later so it would be safer to agree this as a working assumption. |  |
| ETRI | We are generally fine with the direction but the diagram is incomplete. To complete the diagram, the output of the ‘Model Training’ block and where it is used should be described. |  |
| CATT | Generally OK. Additionally: - The link from ‘model inference’ to ‘performance monitoring’ may better be connected from ‘model inference’ to ‘data collection’ (The ‘data collection’ is responsible to collect output data and provide it to performance monitoring). But we can accept current form if the majority thinks OK. - Model delivery/transfer may be needed (especially for UE-sided model). If presented, a link should be created from ‘model training’ to ‘model inference’. |  |
| Samsung | Fine in general. |  |
| Xiaomi | - Model delivery/transfer and model/functionality identification is not included. - For the model inference control (activate, deactivate, switch, fallback), the operation of fallback should be handled separately, since the subsequent operation is not model inference for fallback. In our view, for fallback operation, the subsequent operation is performance monitoring for the inactive AI model |  |
| NEC | Support in general. But like Vivo, model delivery and transfer seem to be missing in the diagram. But so far as it is not agreed whether model delivery/transfer will have impact on 3GPP signaling or not, we would like this to be pointed in the Notes that this description is only applicable for the scenario where model transfer/delivery is not within 3GPP signaling. |  |
| ZTE | Generally fine with the diagram. One question for clarification: Is the ‘output’ between model inference and performance monitoring for output based model monitoring? Our understanding is that the inference output should be fed back to ‘actor’ (e.g., gNB). |  |
| OPPO | First, we doubt we really need a diagram to illustrate the functional framework. In the one year study, the group has got a clearer understanding than the diagram. Many details have been identified in the previous agreements. It may be difficult to accurately describe the framework in a diagram. And the diagram will not be present in the future standards. For the diagram provided by the FL, we have several comments: 1. The input to Model Inference block is from data collection. We do not think it is correct. Data Collection is for training or monitoring, not for inference. The input to Model Inference block can be simply named “Input”. It is the normal signal stream in the 5G air interface, not a special “collected data”. 2. The output from Model Inference block should not only for Performance Monitoring. It is firstly for outputting the normal signal stream in the 5G air interface. 3. Model Inference can be placed in the kernel of the diagram because it actually replaces a feature in the 5G air interface processing chain, with Input on left and Output on right. Then Performance Monitoring, Training, Data collection blocks can be listed surrounding Model Inference. 4. So far we do not see strong needs to include Actor and Model Storage. |  |
| Rakuten Mobile | - In our opinion, Model Inference and Model Training can be triggered not only by Performance KPI, but other reasons like validity timer, NW update, and so on. Therefore, current description about the trigger entity of AI/ML operation is not sufficient. - Definition of Actor is not clear. |  |
| Lenovo | Though the ‘Performance Monitoring’ could be the core function of management, we think it is better to replace it with ‘Model Management’ to cover more possible functions as a more general framework for air interface enhancement. In addition, an arrow from ‘Model Training’ to ‘Model Management’ can be added as an action of model deployment after verification for further inference. |  |
| CAICT | General fine. Model transfer/delivery could be added. |  |
| Huawei, HiSilicon | Support in principle. For the note part, we think some of them may be non-3GPP entity, but not “each”. E.g., for inference, it must be 3GPP entity, right? Change as below A block may be implemented in one or multiple entities, each some of which may be 3gpp or non-3gpp entities. |  |
| Fujitsu | Generally, it is fine for us. We have several suggestions as follows: 1. model transfer/delivery can be included in the diagram. 2. performance monitoring seems too specific. Even in model monitoring, there is an alternative to monitor data drift. One option would be change “performance monitoring” to “model management” |  |
| NVIDIA | Support in principle. |  |
| Ericsson | Support in general. It seems that performance monitoring box is responsible of handling the model LCM aspects. We prefer the figure from Futurewei, rename “Performance monitoring” to “model management (monitoring, training/inference control)”. Moreover, we don’t think there is a need for an “output” arrow from inference to performance monitoring. The necessary data for monitoring could be collected via the data collection functionality. Consider adding an arrow from model inference to data collection labelled “inference output data”. |  |
| Nokia/NSB | It is not clear that the above figure is compatible with the “high-level AI/ML framework” and planned use of such figure. For now, to our reading, the AI/ML framework shall be based on functionality LCM, (and optionally with model LCM (to be discussed still)). The above figure does not seem to be capturing that and not fully inline with the agreements made in the last RAN1 meeting. Overall, we are fine to discuss this framework bit later when the exact details on functionality and model LCMs, and related interactions are clear. |  |
| Apple | Support in principle |  |
| KDDI | Support in principle. |  |
| Fraunhofer | Support in general, but we also agree to rename the model/performance monitoring box to “model management” (that includes the model/performance monitoring functionality) |  |
| Sony | We are fine with 2nd bullet of CATT’s comment. Model delivery/transfer would be needed. |  |
| AT&T | Support in general. We can have this as working assumption and update as needed. Also it is better to rename performance monitoring block to model management. |  |
| Futurewei | Support in principle |  |
| InterDigital | OK at high level. Some updates can be made to improve clarity. Performance monitoring seems to be overloaded with model control. It may be cleaner to show model control/management separately, since model activation/deactivation may be triggered by reasons other than performance monitoring. The output of model inference is not just for performance monitoring, the output of inference can be shown to be used for use case specific actions. |  |
| Spreadtrum | Generally we are fine. In addition, model identification may be also needed between model training and model inference, since only after model identification, one trained model can be utilized to be for reference. |  |
| Intel | Support, not sure if we need new terms like model training control and model inference control simply the terms inside parenthesis may be fine. |  |
| Mod | RAN2 has some agreement. We can reconcile with RAN2 agreement. |  |

###### [FL2] Proposal 6-1b:

FL Note: RAN2 has agreed that

The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage. Chair: the following was almost agreed (leave it FFS for now): AI/ML functional architecture in Figure 1 in R2-2303674 is the baseline with the modification that Performance Monitoring is changed to Model Mgmt / Performance Monitoring. It is noted that the exact interactions may need some modification depending on how each piece of functionality is specified.

The proposal has been modified according to the RAN2 agreements and company comments.

Take the following as a starting point for high-level AI/ML framework diagram.

Notes

- A block may be implemented in one or multiple entities, some of which may be 3gpp or non-3gpp entities.

- The interactions between blocks may or may not have impact on 3gpp signaling.

- Other blocks may be added later.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

### Collaboration levels

Spreadtrum

Proposal 1: Confirm the working assumption on the definition of Level y-z boundary.

Sony

Proposal 1: RAN1 should prioritise the study of collaboration level y&z between transmitter and receiver to identify issues and solutions.

Proposal 2: RAN1 should study what signalling information would be needed for training and how to transfer an AI/ML model.

Google

Proposal 1: Do not confirm the working assumption on level y-z boundary and maintain the level y-z boundary based on the agreement in RAN1 #109.

LG

Proposal #9: For collaboration level z, RAN1 should focus on RAN1-specific issues only or deprioritize it for further study.

CMCC

Proposal 1: Confirm the working assumption on Level y-z boundary.

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

Nvidia

Proposal 2: RAN1 to further clarify the meaning of “dedicated AI/ML-specific enhancement.”

For example, if RAN1 introduced the feature that “UE to report the measurement results of more than 4 beams in one reporting instance” for AI/ML based beam management, would the feature be qualified as “dedicated AI/ML-specific enhancement”?

Rakuten

Proposal 1: Confirm the following working assumption:

•	Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

•	Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

•	Clarifying note: Level y includes cases without model delivery.

Proposal 2: Further clarification of the AI/ML collaboration Level y includes:

•	Level y-1: NW based AI/ML application

•	Level y-2: Dual-sided AI/ML application

•	Level y-3: UE based AI/ML application

The above clarification can be independently defined as framework, instead of clarification of the collaboration levels.

Proposal 3: For collaboration level z, controllable model parameters should be aligned with collaboration level y, at least

NEC

Proposal 18: Support to define network-UE collaboration levels based on one-sided AI/ML model or two-sided AI/ML model.

##### Issue 6-2: (placeholder)

### ML model Life Cycle Management

RAN1 #110 Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

#### Data collection

Conclusion from RAN#1 110-bis-e Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN2 #121 agreement Proposal 1: - RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity.​ Proposal 2: - Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc.​ Proposal 3: - RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted.​ Proposal 4: - Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics.​ Proposal 5: - When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.​ Proposal 6: - Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.​ Proposal 7: - Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required. Proposal 8: - For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary P1-P8 are loosely endorsed with the understanding that we can also go beyond, e.g., analyse other methods. RAN2 #121 agreement R2 may consider including the existing EVEX framework for this SI, FFS exactly what this means, can discuss next meeting

FutureWei

Proposal 2: When studying data collection from two directions, study the method of indicating the capabilities of one side to the other side, in a way that reflect its storage capacity. In addition, study the mechanisms of reducing the size of data needs to be transmitted over the air interface considering the balance between performance and the overhead.

Huawei, HiSilicon

Proposal 5: Study the potential spec impact of data collection from realistic networks for supporting the LCM of AI/ML model, including at least:

- Enhanced/dedicated RS design.

- Enhanced UE measurement/report.

- Type/format of the data sample(s).

- Signaling for indicating/requesting data collection.

Proposal 6: For data collection, study the procedure/signaling to generate/carry data sample(s), including both L1 and L3 measurement/reporting.

Proposal 7: Study the following aspects to improve the quality of dataset during data collection:

- Improving the quality of data samples, e.g., improving the accuracy of the measured labels.

- Indicating the quality requirement of data samples to be reported.

Proposal 8: The necessity of introducing new assistance information for data collection/categorization needs to be clarified/justified, considering:

- UE can sense the scenario autonomously without being notified by gNB or with legacy signaling.

- The categorization or granularity of the scenarios identified by Network vendor may not match the categorization principle of the UE side.

- Generalized model can be trained over scenarios/configurations.

Proposal 9: The assistance information for data collection/categorization, if studied, should be in forms of virtualized ID to avoid the disclosure of proprietary.

- Such assistance information can be sent from Network to UE or from UE to Network.

ZTE

Proposal 1: 	 When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side, the specification enhancements should focus on RS configurations for data collection.

Proposal 2: 	Current RS configurations should be reused to define the associated scenarios/configurations of the collected data.

Proposal 3: 	 The sharing of assistance data for data collection should keep private/proprietary information.

Proposal 4: 	 When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on different sides, further study:

- Signaling enhancements on configuration of reference signals and measurement report;

- Enhance existing measurements or define new measurement types to be discussed per use case.

Proposal 5: For the purpose of identifying a dataset between network side and UE side, further define the following terminology:

- Dataset identification: A process/method of identifying a dataset for the common understanding between the NW and the UE

- Note: The process/method of dataset identification may or may not be applicable.

- Note: Information regarding the dataset may be shared during dataset identification.

Vivo

Proposal 18: Study the following assistance information for data collection:

- General description of collected data, such as purpose, size and configuration;

- UE hardware information (meta data), such as antenna information;

- Applicable conditions, such as cell ID, scenario ID and SNR.

Proposal 19:	Study the two following reporting formats for a large number of collected data samples:

- Large number of samples in one report with low reporting frequency;

- Small number of samples in one report with high reporting frequency.

Proposal 20: Study the data compression for multiple samples in collected data reporting.

Proposal 21: Study ways for UE to report its capability for data collection regarding expected pre-processing, data storage, feature extraction and report for data collection.

Intel

Proposal-2: Support assistance information from NW to UE and UE to NW to facilitate data collection for training at the UE and at the NW respectively.

Ericsson

Proposal 15:	 RAN2 should determine how the data collection should be introduced within the functionality-based LCM framework. RAN1 should focus on potential requirements for such data collection.

Proposal 22: Study 3GPP data collection mechanisms to support UE performing data logging/collection and reporting the collected data to NW over the air-interface for model training.

Proposal 23: Recommend RAN2 to study specific details of an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training.

Proposal 24: Study in RAN1 the requirements of data collection for model inference per AI on PHY use case.

Proposal 25: Recommend both an RRC-message based (slower) and L1 (i.e., similar to aperiodic CSI reporting) based (faster) data collection methods for NW side model monitoring, if L1 UE measurement is needed.

Proposal 26: RAN1 to study specific requirements on RRC-message based data collection method for NW side model monitoring, if L1 UE measurement is needed. When conclusions are reached on the requirements, RAN1 would indicate that to RAN2 so that RAN2 can study the corresponding RAN2 aspects.

Proposal 27: RAN1 to study specific requirements and specification impacts of an L1 based (i.e., CSI reporting) data collection method for NW side model monitoring, if L1 UE measurement is needed.

Xiaomi

Proposal 1: Data collection from network to UE and potential network operation to facilitate the data collection on UE side should be supported.

Proposal 2: AI framework agenda provides guidance to the data collection in each use case in order to share necessary information to RAN2

- What is the requirements of the data collection for model training (focus on the offline training ), model inference and performance monitoring, respectively

  - Data amount, latency, privacy, …

- What is the input and output of the AI model

- What is the associated information should be collected

- What is the assistance information would be involved

The detailed procedure/signalling for the data collection is discussed per use case based on the guidance.

Google

Proposal 2: Rel-18 should prioritize the data collection for model interference, model monitoring and model selection, and the data collection for model training and update should be deprioritized.

Samsung

Proposal#9: Study the necessity, requirement and specification of data collection for two cases of purposes

- Case1: : Real-time purposes, e.g., model monitoring, inference, selection, switching, etc.

- Case2:  Non-real-time purposes, e.g., model training, update

Proposal#10: Study the following two directions of data collection where applicable, assess their pros and cons and specification impact:

- Network-side data collection and assistance information from UE

- UE-side data collection and assistance information from network

Proposal#11: Study per each use case the feasibility/frequency/overhead of dataset collection before designing the dataset sharing framework

CAICT

Proposal 2: Data collection signaling for multiple use cases should be considered to simplify signaling design.

CMCC

Proposal 3: For data collection, study the potential spec impact of dataset delivery based on 3GPP signaling.

Proposal 4: To enable the development of a set of specific models, study the way to associate the dataset with a specific scenario/configuration/site.

Proposal 5: To further improve the system performance, study the mechanism to reduce overhead of data collection in LCM.

MediaTek

Proposal 5: Data collection comprises multiple functional entities serving for different purposes of functions. The functional entity of data collection is co-located with the function for which the dataset is used.

Proposal 6: Data collections should consider the requirements of data size, latency, validity, security and privacy. Consider distinct data requirements for different purposes, e.g., offline training and model monitoring/inference.

Proposal 7: RAN1 consider UE data transfer from UE to the OTT server with the following ways. FFS on others.

- Option 1: Data transfer to UE to OTT server with RAN awareness

- Option 2: Data transfer from UE to OTT server without RAN awareness

Proposal 8: Study the following mechanisms for data collection: utilize existing or extension of existing L1/L3 measurement and report procedure or utilize procedure particular for data collection request and control.

Nvidia

Proposal 9: For AI/ML LCM, study potential specification impact related to data collection for different purposes, including model training, model inference, model monitoring, model selection, model update, etc.

Proposal 10: For AI/ML model training in each NR air interface enhancement, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Lenovo

Proposal 12: Study the requirements with respect to the payload and delay to collect data for model training, management and inference per sub use case.

Proposal 13: Associate the dataset for the AI/ML model with scenario/configuration/site-specific setting, at least for performance conformance evaluation.

Qualcomm

Proposal 14: For data collection for model training, RAN1 should focus on what data should be collected. Mechanism for training data collection needs architectural considerations and should be handled by other working groups.

Proposal 15: Data collection for real-time performance monitoring should be avoided.

NEC

Proposal 5: Study whether and how the legacy CSI framework, BM framework and positioning framework can provide sufficient data for model training (including fine-tuning) and model inference.

Proposal 6: For model monitoring based on inference accuracy, study methods of ‘ground truth’ data collection.

NTT Docomo

Proposal 15: Assistance signalling including scenario/configuration ID should be assumed for dataset collection, model training, and model inference of scenario/configuration-specific AI models.

Proposal 16: Study DL RS request for UE side data collection.

TCL Communication

Proposal 4: As a step of data collection, the quantization can be done with different requirements for different purposes.

##### Issue 6-3: Data collection requirements

###### [FL1] Proposal 6-3a:

Study the requirements associated with data collection for different purposes such as training, monitoring, and inference:

- Amount of data, latency, validity, privacy, security

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic |  |
| Company | Comments |  |
| vivo | Seems this need to be studied for each use case? We are also fine for this as guidance for each use case study. |  |
| Google | We are not sure whether this is needed at current stage or not. It seems the requirement related aspects (RAN4 requirement?) are usually after we have defined some mechanisms. |  |
| LG | Similar feeling as vivo that this study may better fit to each use case that already have done so far. If this proposal is for general guidance or principle, it needs to be more specific. |  |
| CMCC | Support this proposal as a guidance for each sub use case. Different use case could have different requirement, and it is related with the training collaboration type, model complexity, etc. |  |
| ETRI | For training, if only offline training is considered in R18, there is no need to discuss it because it is an area of offline engineering or RAN4. For the rest, there seems to be a mix between what is needed for monitoring and what is needed for inference. For example, latency requirements may be only needed in the case of inference. Also, neither privacy nor security is considered as a scope of RAN1. |  |
| CATT | Support. RAN2 may also need this information for procedure/protocol discussion on data collection. |  |
| Samsung | Ok. |  |
| Xiaomi | Support. The data collection is also under discussion in RAN2. It would facilitate the discussion in RAN2 as well. |  |
| NEC | We need to first have to candidate methods for data collection and then we can discuss about requirements. Thus we suggest to change the main bullet to “Study the methods and requirements associated with data collection” to include also the proposal 6-4a. |  |
| ZTE | It’s not necessary for this general proposal. Besides, some requirements are not in RAN1 scope (e.g., validity, privacy and security)。 |  |
| OPPO | We do see the clear needs for data collection for inference. Can FL further elaborate it? |  |
| Rakuten Mobile | As mentioned by vivo, actual data to be collected should be discussed in the use case discussion. In general aspects agenda, high layer requirements may be studied to trigger discussion in other WGs. |  |
| Lenovo | Agree, but we suggest studying such requirements per use case as updated below: Study the requirements associated with data collection for different purposes such as training, monitoring, and inference per use case: - Amount of data, latency, validity, privacy, security |  |
| CAICT | Fine with the proposal. |  |
| Fujitsu | Generally, it is fine for us. In addition, we suggest adding how often (frequency) of data collection. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | Agree that this could be a guidance for each use case. |  |
| Nokia/NSB | Proposal seems to be very generic and use-case level discussions went further ahead with data collection studies. We do not see any new detail is getting agreed by this. RAN1 also made following in RAN#110bis under 9.2.1. Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) |  |
| Fraunhofer | Support |  |
| Sony | Share the vivo’s comment |  |
| AT&T | Support |  |
| Futurewei | Support in principle. Agree with ZTE that some items in the list are not in RAN1 scope. But this can be a starting point. |  |
| InterDigital | The requirements depend on the purpose and the (sub)use case. The aspects like privacy, security could be discussed in other WGs. |  |
| Spreadtrum | Quality of data may be also needed to be considered. The quality level of data would bring impact on the training stage at least. |  |
| Intel | We are fine in general, privacy, security aspects may not be RAN1 aspects, validity may need a definition as well. |  |

###### [FL2] Proposal 6-3b:

(FL Note: This is provided as a general guidance to each sub use case and RAN2. As mentioned by a few companies, if RAN1 agrees on requirements for data collection for different purposes, it could help RAN2 discussion on this topic.)

Study the requirements associated with data collection for different purposes such as training, monitoring, and inference:

- Amount of data, latency, frequency, validity, privacy, security

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-4: Data collection study aspects

###### [FL1] Proposal 6-4a:

Study the following aspects and the corresponding potential specification impact related to data collection for training:

- RS configuration enhancements and measurement report enhancements

  - Note: The study should consider whether existing framework can be reused

- Contents, e.g., type and format of data samples

  - E.g., data related to model input, data related to ground truth output, assistance information for categorizing the data, other information

  - Note: Use-case specific details can be studied in respective agenda items

- Signaling of assistance information for categorizing the data (e.g., configuration ID)

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure, e.g., request / trigger, configuration, quality requirement, etc.

  - Note: Signaling mechanism details can be studied by RAN2

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | Panasonic |
| Company | Comments |  |
| vivo | We are fine with study on the third bullet. Other parts seem better studied for each use case. Study the following aspects and the corresponding potential specification impact related to data collection for training and inference: - RS configuration enhancements and measurement report enhancements - Note: The study should consider whether existing framework can be reused - Contents, e.g., type and format of data samples - E.g., data related to model input, data related to ground truth output, assistance information for categorizing the data, other information - Note: Use-case specific details can be studied in respective agenda items - Signaling of assistance information for categorizing the data (e.g., configuration ID) - Note: The study should consider the feasibility of disclosure of proprietary information - Signaling for data collection procedure, e.g., request / trigger, configuration, quality requirement, etc. |  |
| Google | For RS enhancement, we think we can add “coverage enhancement for the RS”, which is important to achieve high reliable measurement results. |  |
| Panasonic | On the first bullet of " - RS configuration enhancements and measurement report enhancements ", although RS is one aspect of the configuration but not required to be limited to RS. RS is too specific and the other configuration can be also related. Therefore, we propose to modify as following. - RS c Configuration enhancements and measurement report enhancements o E.g., RS configuration o Note: The study should consider whether existing framework can be reused |  |
| LG | This topic has been studied in each use case. Not sure urgent need for this agreement in this agenda. |  |
| NTT DOCOMO | Support the proposal. Especially, categorization for the data collection should be studied as it enables training of configuration/environment specific models. |  |
| CMCC | Support. Some similar agreements have been agreed in some use case specific agendas. We can agree here as a general guidance. |  |
| ETRI | The hierarchy seems to be different between the sub bullets. To our understanding, the fourth sub could cover the rest. It would be better to break down the data collection process. For example, it can be divided into 1) signaling for data collection resources, 2) signaling for data collection reporting, and 3) signaling for data collection assistance information. |  |
| CATT | Support the intension. But the detailed aspects may be (or perhaps only be) studied/concluded in each agenda. FYI, the following agreement has already been achieved in 9.2.2.2 in RAN1#112: Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection. |
| Samsung | Ok with the direction. |  |
| Xiaomi | We think it is better to discuss these specific issues in each sub-use case. |  |
| NEC | We support first two sub-bullets. For the third bullet, we don’t know what a configuration ID is. |  |
| ZTE | Fine with the proposal in general. In addition, we have two comments: - Replace ‘configuration ID’ with ‘categorization ID’ since the configuration may be used for indicating something that can be clearly defined by specification. However, we think proprietary information is hard to be specified. Therefore, ‘categorization ID’ is more general. - Data collection should include another important aspect: dataset exchange. Our thinking is that dataset alignment is to ensure NW and UE have the same understanding on the categorized data. We propose to have another bullet: - Singalling of dataset alignment between NW side and UE side. - E.g., Dataset identification(a process/method of identifying a dataset for the common understanding between the NW and the UE) |  |
| OPPO | We are generally OK for the study aspects. But they actually depend on the detailed use cases. |  |
| Rakuten Mobile | We are fine with the 3rd and 4th bullets to study in this agenda, at least. |  |
| Lenovo | Agree in general, and we suggest adding more clarification on the 2nd bullet as below: - Contents, e.g., type and format of data samples - E.g., data related to model input, data related to ground truth output, assistance information for categorizing the data, quality of the data sample, other information |  |
| CAICT | We are fine to have some high-level guideline on data collection for training. |  |
| Huawei, HiSilicon | We have a similar feeling that this can be studied per use case. If really needs to be discussed here, some comments in below: In addition, the report is missed from the signaling; the “configuration ID” is not clear, so change it to “data categorization ID”. Study the following aspects and the corresponding potential specification impact related to data collection for training: - RS configuration enhancements and measurement report enhancements - Note: The study should consider whether existing framework can be reused - Contents, e.g., type and format of data samples - E.g., data related to model input, data related to ground truth output, assistance information for categorizing the data, other information - Note: Use-case specific details can be studied in respective agenda items - Signaling of assistance information for categorizing the data (e.g., configuration data categorization ID) - Note: The study should consider the feasibility of disclosure of proprietary information - Signaling for data collection procedure, e.g., request / trigger, configuration, report, quality requirement, etc. Note: Signaling mechanism details can be studied by RAN2 |  |
| Fujitsu | Considering relevant discussions had been conducted in each use case, and some agreements had been reached on data collection, if the intention of this proposal is to give guidance to future use cases, it can be concluded later based on the agreements per use case. |  |
| NVIDIA | These topics are much use case specific, and have been discussed in the respective agenda items. Use case specific agenda items can continue to make progress in these aspects. |  |
| Ericsson | Same view as LG, not sure if this needs to be agree since it is already studied in each use case. |  |
| Nokia/NSB | Several comments, - The intension of the proposal is unclear as these details are under discussion in use cases for multiple meetings. - Needs to be clarified where the training takes place: UE or NW, or both. And, which ML model/functionality is being trained: UE or NW, or both - Not clear why model training is mentioned as the data collection purpose. - Not clear what is intended with “configuration ID” - High-level texts like “The study should consider the feasibility of disclosure of proprietary information” shall not be mentioned at this stage of the SI. |  |
| KDDI | Same view with LG and Ericsson. We should clarify why this should be discussed in this agenda. |  |
| Sony | We are fine with the proposal as a general guidance for each use case agenda. |  |
| AT&T | Support. However, only general aspects regarding data collection should be discussed with details to be agreed within each use case discussion. |  |
| Futurewei | Agree with most companies that all these can be studied in use cases. |  |
| InterDigital | The split between use cases specific aspects and general aspects should be clarified. |  |
| Spreadturm | Fine with the direction. |  |
| Intel | Support, we don’t think this is inconsistent with the discussions for the sub use-cases, this provides an umbrella for the detailed discussions under use-cases |  |

###### [FL2] Proposal 6-4b:

(FL Note: The intent of this proposal is to provide general guidance on how to categorize the study on data collection for training into different aspects that are common to the different use cases. Aspects specific to each use-case can be studied in the respective agenda item. The proposal has been updated to incorporate comments from the first round.)

Study the following aspects and the corresponding potential specification impact related to data collection for training:

- Measurement configuration and reporting enhancements

  - E.g., RS configuration

  - Note: The study should consider whether existing framework can be reused

- Contents, e.g., type and format of data samples

  - E.g., data related to model input, data related to ground truth output, assistance information for categorizing the data, quality of the data, other information

- Signaling of assistance information for categorizing the data

  - E.g., ID for data categorization based on scenario/configuration/etc.

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

  - E.g., request / trigger, configuration, quality requirement, report, etc.

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by RAN2.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

#### Model development and training

Huawei, HiSilicon

Proposal 10: The discussion of online/offline training should be decoupled with whether the data collection/dataset delivery is performed via air-interface or non-air-interface.

Proposal 11: For the study of one-sided AI/ML model, model training without model transfer/delivery should be considered as a starting point, i.e.,

- On-Network training for Network-side model.

- On-UE training for UE-side model.

Vivo

Proposal 10:	To fight against the AI/ML generalization problem, generic model would typically have larger computation complexity and storage overhead, while zone/site specific models would need simple model structure and small model size.

Oppo

Proposal 12: In the early stage of Rel-18 study, prioritize study of the AI/ML inference over the study of AI/ML training.

- Study offline training with high priority and as the default training type.

Spreadtrum

Proposal 4: For model training for one-sided model, the following model training types can be further discussed:

- Type 0: Training at a single side/entity without model transfer

- Type 1: Training at a single side/entity, and model transfer to another side/entity

Proposal 5: For one-sided AI/ML model, model training and model inference at the same side should be considered as a starting point, i.e.,

- Network-side training for Network-side model

- UE-side training for UE-side model

Proposal 6: Offline AI/ML model training is the first priority.

Ericsson

Proposal 16: Deprioritize studies and discussion on over-the-air training between NW and UE.

Xiaomi

Proposal 3: Prioritize the study of offline training in Rel-18

Qualcomm

Proposal 12: To avoid performance issues due to training and target platform differences, it is highly desirable for the trained model to be converted (quantized, compiled) and tested offline prior to being delivered to UE.

Proposal 17: Model training needs to consider feasibility of disclosing proprietary model information to the other side.

Proposal 18: Model structure design needs to consider device-specific design optimization and capability.

Proposal 20: Model training and conversion to executable involves device-specific optimization.

NTT Docomo

Proposal 9: Consider the LCM framework with and without online training, separately.

##### Issue 6-5: training type de-prioritization

There are many opinions on deprioritizing certain training aspects. The FL captured them in a proposal in Section 2.6.

#### Two-sided model development and training

## Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at Network side and UE side, respectively. - Type 3: Separate training at Network side and UE side, where the UE-side CSI generation part and the Network-side CSI reconstruction part are trained by UE side and Network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with Network side training [, or parallel training] at UE and Network - Other collaboration types are not excluded.

FutureWei

Proposal 10: For the three types of two-sided model training, study and compare their performance, signaling overhead and potential standard impacts.

Proposal 11: For Type 1 two-sided training, when the joint training is done at the network side, make the perform-at-network the baseline solution.

Huawei, HiSilicon

Proposal 12: For training Type 1 (joint training of the two-sided model at a single side/entity), prioritize the study of joint model training at Network side and transfer/deliver the model to the UE side.

Samsung

Proposal #10: Deprioritize two-sided model training collaboration that requires extensive sharing over the air interface of training, validation and testing dataset.

Proposal #7: Consider the following and study their impacts for the two-side model development approaches,

- Requirements on privacy-sensitive dataset sharing

- Scalability, i.e., whether the number of models one vendor should develop increases with the collaborating vendors

- Whether the model development approaches adhere to 3GPP’s open and fair framework

CAICT

Proposal 3: For two-sided AI/ML model, joint training and separate training should be supported.

MediaTek

Proposal 9: Study the proprietary way, 3GPP specified way and combination of them for dataset exchange for two-sided model Type 3 training.

Qualcomm

Proposal 22: Adopt the following two-sided model development/training framework:

Case 1: Initial (non-backward-compatible) development/training of “nominal encoder + nominal decoder”

  - The use of the nominal encoder at the UE-side is not mandated

    - If needed, UE-side may implement a different proprietary encoder based on this decoder using Case 2.

    - As the encoders are only nominal, input used in the training process is only a nominal input. The actual input to the CSI encoders may be different and of proprietary choice.

  - The use of the nominal decoder at the NW-side is not mandated

    - If needed, NW-side may implement a different proprietary decoder based on this encoder using Case 3.

  - Case 2: Encoder development/training to be interoperable with existing decoders (e.g., encoders for new UEs or updating encoders for existing UEs):UE-side vendor trains new encoders based on the existing decoders. Infra vendor should make the existing decoders available (via either a run-time image or an API for training) for the encoder training.

Case 3: Decoder development/training to be interoperable with existing encoders (e.g., decoders for new cell sites or updating decoders for existing cell sites):

  - Network-side vendor trains new decoders based on the existing encoders.

  - FFS: Need for encoder availability for decoder training

##### Issue 6-6: (placeholder)

#### Functionality/model identification and methods of LCM

##### Previous agreements

RAN1 #110-bis-e Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models - FFS: Detailed discussion of model ID with associated information and/or model functionality. - FFS: usage of model ID with associated information and/or model functionality based LCM procedure - FFS: whether support of model ID FFS: the detailed applicable AI/ML operations RAN2 #119bis-e agreement R2 assumes that a model is identified by a model ID. Its usage is FFS. RAN2 #119bis-e agreement R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. RAN1 #111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Funtionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs RAN1 #111 Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. RAN2 #120 agreement R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. RAN2 #120 agreement R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN1 #112 Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model RAN1 #112 Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. RAN1 #112 Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. RAN2 #120 agreement R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. RAN2 #120 agreement R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN2 #121 agreement RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified

Proposal from RAN1 #112 that were discussed but not agreed ###### Proposal 5-8i: At least for UE-side models and UE-part of two-sided models, RAN1 to study - How to define and study a (set of) applicable conditions for functionalities/[models]. - Note: Applicable conditions may be used to enable development of scenario/configuration/[site]-specific models [and, if needed, report the models’ applicability to the Network]. - Whether and how to define performance targets (possibly as a part of applicable conditions) for functionality/[models] - Whether and how UE reports a (set of) applicable conditions for supported functionalities (and if needed, for supported models) and/or supported set of functionalities. ###### Proposal 5-13b: In terms of applicability of functionality-based LCM and mode-ID-based LCM, - For Collaboration Level y, - Functionality-based LCM is applicable. If model identification is supported, Model-ID-based LCM is applicable. - For Collaboration Level z, - Alt 1: Functionality-based LCM is applicable. If model identification is supported, Model-ID-based LCM is applicable. - Alt 2: Model-ID-based LCM is applicable. Functionality-based LCM is not applicable. The following is identified for further discussion of pros and cons of functionality-based LCM and model-ID-based LCM: - UE-side implementation flexibility/optimization in model applicability granularity, e.g., Doppler-specific models - Test coverage and performance monitoring - LCM operation complexity at the Network - Model transfer support - Two-sided model inference support - Flexibility and scalability ###### Proposal 5-14e: An AI/ML model being identified by a model ID may be provided with some model description during model identification. Model description regarding the AI/ML model shared between the NW and the UE-side during model identification may include: - Applicable AI/ML-enabled Feature - Applicable conditions of the model - E.g., via Applicable functionality/functionalities Notes: Other aspects may be added as discussion progresses ###### Proposal 5-15d: Assistance information from Network to UE for at least training data collection, inference, and various other LCM purposes may be implied by a functionality itself or via configurations within a functionality. Note: Other ways of providing assistance information can be further discussed in each use case agenda. Note: The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

##### Company proposals

FUTUREWEI:

Proposal 3: A model ID is a unique index that differentiates one model from other models within a network. The model IDs may or may not be globally unique.

Proposal 4: Study the following two model identification approaches, as well as their pros and cons.

- Model ID identifies the model structure as well as the parameters associated with it.

- Model ID identifies only the model structure; model parameters are indicated via other methods.

Proposal 5: Each model ID should be associated with a list of meta information that describes the functionalities, associated features, and other characteristics etc. of the model.

Proposal 6: Model ID and UE capability may not have direct relationship or dependency. Model IDs may be carried in UE capability reports to inform the network about models that the UE supports.

Proposal 7: An AI/ML functionality ID is a unique index/number that differentiates one AI/ML-related functionality of model from other AI/ML-related functionalities of models within a network.

Proposal 8: RAN1 focuses on model identification in the SI phase and defer the study of functionality identification details till Rel-19 work item phase.

Proposal 9: Model registration is a process wherein UEs/vendors make a newly developed model known to the network by registering it. The model may be assigned a globally or locally unique model ID as a result of the registration. The registration process may also populate meta information of the model, including the functionality of the model, applicable conditions and so on.

Huawei, HiSilicon:

Proposal 13: The granularity of functionality can be smaller than sub use case, so that Network can better be aware and interact with the UE-side/UE part model, e.g., reconfiguration of specific RRC parameters may result in potential model switching.

Proposal 14: Categorize the functionality identification modes into the following two modes

- Mode 1: Functionality identification-basic, where NO globally unique ID is needed.

- Mode 2: Functionality identification-enhanced, where globally unique scenario ID and/or globally unique dataset ID is needed, which may have SA impact.

Proposal 15: Consider a unified design to use model ID to differentiate different models regardless of whether they are subject to the same or different model structures.

- Regardless the UE-side/UE part model to be registered only updates the parameters or adopts a new model structure with new parameters, it is regarded as a new model at the Network side, and is assigned with a new model ID.

Proposal 16: The model ID should take logic ID/pairing ID as a starting point; whether a logic ID/pairing ID corresponds to one or more physical UE part model is implementation.

Proposal 17: Consider model identification as a parallel mode with functionality identification (i.e., model identification mechanism is not supported on top of functionality identification mechanism).

Proposal 18: For model identification, study the mechanism to allow UE to timely report the list of currently supported UE part/UE-side models after identification, where the supported models may be a subset of all identified models.

Proposal 19: Send LS to SA2/SA3/SA5 for clarifying the feasibility and potential SA impact on functionality identification-enhanced and model identification.

- For functionality identification-enhanced, the potential SA impact from the aspects of globally unique scenario ID, and globally unique dataset ID.

- For model identification, the potential SA impact from the aspects of globally unique model ID, and model registration

Proposal 29: Study UE capability for the following procedures of the LCM:

- Capability of dataset delivery

- Capability of data collection

- Capability of model training

- Capability of inference latency

- Capability of monitoring

- Capability of models switching

- Capability of model updating

ZTE:

Proposal 7: From RAN1 perspective, model/functionality identification should focus on UE-side model or UE part model of a two-sided model.

Proposal 8: To understand the functionality identification and its applicable conditions, further study the pros and cons of the following options:

• Option 1(UE feature based method): Like legacy UE capability report, UE may report its supported features/components (i.e., applicable conditions) for a given AI/ML-enabled feature (i.e., sub use case).

• Option 2(Functionality ID based method): For a given AI/ML-enable feature (e.g., a sub use case), there are multiple functionalities reported by UE to the AI/ML-enabled feature, e.g., reported by different functionality IDs. Different functionalities have different applicable conditions, which enables UE to train and deploy various models.

• Option 3(Functionality ID based method + Model-level operations within a functionality): In addition to Option 2, model-level operations within a functionality are enabled to address the dynamic parts (e.g., dataset alignment, model pairing information, applicable scenarios) that are hard to be quantized and specified by static UE features.

Proposal 9: Functionality identification should consider the solutions to incorporate both static parts (e.g., UE features by some candidate values) and dynamic parts (e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios).

Proposal 10: Model identification should be discussed separately from other LCM procedures.

Proposal 11: To better understand the model identification process, further study following model identification types:

• Type A1: Model is identified offline to NW (if applicable) and UE (if applicable)

• Type A2: Model is identified via signaling from UE to NW

• Type B: Model is identified via signaling from NW to UE

Proposal 12: Send LS to RAN2/SA2 to study further details of the model identification Type A1, Type A2 and Type B (e.g., how to register/de-register the model and how to conduct certification test on the model).

• Similar specification impacts as both procedures need to report the applicable conditions (e.g., defined by UE features) of a model or a functionality.

• The functionality ID is reported by UE, which is a local ID within an RRC configuration. No specification impacts beyond the UE capability report is foreseen. The model ID is assigned by network side, which can be global.

• Depend on whether a full model or a model structure is identified, the model description information of Type A2 may include other information apart from applicable conditions.

Proposal 13: The model ID used in signaling between UE and network can refer to a physical model or a logical model:

• Physical model: Refer to a model file (or an algorithm) either in proprietary-format or open-format;

• Logical model: Refer to an identifier for a model that may be used for LCM purposes. A logical model may be implemented by one or multiple physical models, e.g. multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms or multiple physical models transparent to NW that are signaled as a single model ID.

Proposal 14: Study whether and how a model (regardless of a physical model or a logical model) identified to NW should satisfy some predictable performance target.

Vivo:

- AI/ML-enabled feature reporting is needed for both functionality identification and model identification and can follow legacy procedure.

- For the purpose of discussion on aligning applicable conditions between entities, define the terminology of “applicability identification” as following.

- Applicability identification: A process/method of identifying an AI/ML applicable condition for the common understanding between the NW and the UE.

  - Note: Information regarding the AI/ML applicable condition may be shared during applicability identification.

- Study “logical model” identification or applicability identification based LCM procedures.

- Consider applicable condition update for an existing model in “logical model” identification or applicability identification procedure.

- Functionality identification based LCM contains two steps as legacy UE features, UE capability report and RRC enabling/disabling procedures.

- Study LCM based on AI/ML-enabled feature with applicability identification, in which applicability ID is used for LCM.

- Consider to define the procedures as in Figure 2-1 for model/applicability identification.

Figure 2-1. The generally procedure of the model-based LCM.

- Consider the following model identification types.

- Type A (Step0 in Figure 2-1): Model is identified offline to network (if applicable) and/or UE (if applicable).

- Type B (Step1, Step2b and Step4 in Figure 2-1): Model is identified via signaling from UE to network.

- Type C (Step2a, Step2b and Step4 in Figure 2-1): Model is identified via signaling from network to UE.

Oppo:

Proposal 1: Support both functionality-based LCM and model-ID-based LCM.

Proposal 2: Local ID is supported for indication of model selection/activation/deactivation/switching/fallback within an AI/ML functionality.

- FFS: Local ID-based model indication across multiple functionalities.

Proposal 3: Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, not on “physical level”.

- Focus on the study on LCM for model on “logical” level.

- No need to explicitly introduce new concepts (e.g. logical model, physical model, source level, binary level) only when the necessity is justified.

Proposal 4: Support Local ID-based model identification.

- Focus on design of Local ID-based model identification assuming non-3GPP-based model transfer.

- FFS: 3GPP-based model transfer/training.

  - First focus on following aspects:

    - Required KPI (e.g. packet size, data rate, latency, reliability), so to select the design (e.g. in which layer/channel).

    - Model transfer format (if needed).

  - Study AI/ML model transfer for training with lower priority.

Proposal 5: At least for LCM with non-3GPP-based model transfer,

- Local ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., use case, scenario, configuration.

- FFS: Whether Global ID is needed and whether the Global ID needs to be defined in 3GPP specification.

Proposal 6: For LCM with 3GPP-based model transfer,

- Local ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., use case, scenario, configuration.

- Global ID may contain the information about the model (explicitly or implicitly).

  - FFS the information, e.g., use case, scenario, configuration.

- FFS: The mapping between the two types of IDs.

Spreadtrum:

Proposal 7: For AI/ML model/functionality identification, the following information can be considered to be included into model description, but need further discussion:

    - Model ID;

    - Model function;

    - Model applicable condition;

    - Model complexity;

    - Model input;

    - Model output

Proposal 8: For functionality identification, there can be one functionality within an AI/ML-enabled feature, and other information such as applicable condition can be denoted in the form of components of one feature.

Proposal 9: For model identification,

- Regarding UE-sided model, model description along with (local) model ID can be reported by UE, and then global or local model ID can be allocated by NW;

- Regarding UE part of two-sided model, model ID is reported by UE with the assumption that offline coordination and/or model transfer are considered.

Proposal 10: Capability reporting for Model identification can be corresponding to one subset of capability reporting for functionality identification.

Nokia:

Proposal 2: At least for UE-side models and UE-part of two-sided models, define the sub-use case specific applicable conditions for functionalities and identify the common applicable conditions for functionalities across different sub-use cases.

Proposal 3: At least for UE-side models and UE-part of two-sided models, the applicable conditions for functionalities shall contain the following (see Table 4),

- Conditions on inference (use case specific)

- Conditions on Performance monitoring (use case specific)

- Conditions on functionality configurations (generic)

- Conditions on functionality validation procedure (use case specific)

- Conditions on supporting fallback (use case specific)

- If applicable context information (use case specific)

Table 4: Framework for applicable conditions for all ML-enabled use cases/Features

| Condition | Description |
| --- | --- |
| 1. Conditions on inference (use case specific) | Indicates the capabilities associated with configurations/parameters (use-case specific) for functionalities of ML-enabled feature. |
| 2. Conditions on performance monitoring (use case specific) | Indicates UE support for NW-sided functionality monitoring, and conditions on related configuration options for functionality performance monitoring. If applicable, indicates UE support for UE-sided functionality performance monitoring, and conditions on feedback/reporting (triggers, events, proxy KPIs, reporting mechanisms, etc.) |
| 3. Conditions on functionality configurations (generic) | Indicates the max number of configured/activated functionalities, delays in activating/switching of functionalities, and Generalization condition of functionalities |
| 4. Conditions on functionality validation procedure (use case specific) | Indicates UE support for Functionality validation procedure (NW-initiated, UE-triggered). Indicates conditions for validation procedure (delays, measurement configurations, reporting configurations) |
| 5. Conditions on supporting fallback (use case specific) | Indicates UE support for one or more fallback features (triggers/events, delays, etc.) |
| 6. Context information (use case specific) - OPTIONAL | If applicable, indicates UE support for monitoring and reporting conditions for UE-side inference context e.g., radio KPIs not explicitly linked to the ML Functionality, and/or non-radio metrics (position, movement, temperature, etc.) |

Proposal 6: At least for UE-side models and UE-part of two-sided models, the NW creates/configures functionalities to the UE with each functionality referring to a configuration message (e.g., RRC or LPP) that contains NW-selected applicable conditions (according to the UE capability).

- Further study whether an additional step of reporting UE-preferred functionalities is needed after NW configures functionalities towards the UE (e.g., prior to any selection/activation of a functionality).

CATT:

Proposal 1: Add the following definition of functionality switching in the terminology list:

| Functionality switching | Deactivating a currently active AI/ML functionality and activating a different AI/ML functionality for a specific AI/ML-enabled feature. |
| --- | --- |

Proposal 4: Functionality identification and model identification are two separate procedures, which does not couple with each other.

Proposal 7: For model identification, the following information can be considered as the starting point for model description information when provided from UE to NW:

- Information on model functionality,

- Information on model input/model nominal input,

- Information on model output/model nominal output,

- Information on assistance information for inference,

- Information on model performance,

- Information on concurrent use with other AI/ML models and/or non-AI/ML features,

- Information on applicable conditions,

- Information on pairing information for two-sided model.

Proposal 8: Metadata of model for model identification can be reused for model transfer.

Proposal 9: A local model ID can be allocated to an AI/ML model for model LCM in physical layer, rather than directly apply the global ID assumed by RAN2.

Proposal 10: For model identification, both offline option and online option can be further studied.

Proposal 11: Consider the following principles for functionality identification:

- Functionality is reported by UE after initial access.

- A set of functionalities with finite number are pre-defined for a given sub use case.

- Within a given sub use case, different functionalities can be distinguished by large granularity characteristics, allowing network have correct understanding on ‘what is required’ and ‘what can be provided’, e.g. input type and/or output type.

- For a specific functionality, applicable condition can be provided as supplementary information, which provide the knowledge on ‘how to achieve good performance’ or ‘what case will lead to bad performance’.

Proposal 12: Information on ‘applicable condition’ for model identification can be reused for functionality identification, or vice versa.

Intel:

Proposal-3: Interpretation of a “model” (physical or logical) depends on context of usage. Model-ID from a model identification process could be associated with a physical or a logical model. From RAN1 perspective there is no need for uniqueness (global or otherwise) of such a model-ID

Proposal-6: Model identification is a process through which NW acquires meta information associated with a physical or a logical model. There is no need to define another model identification process from NW to UE

Proposal-7: A model-ID based LCM requires a model identification process – in contrast, a functionality based LCM is not associated with an identification procedure, it is not applicable for model transfer/delivery (collaboration level-z) and it is not applicable to two-sided models

Proposal-8: we suggest to develop model-ID based LCM and functionality based LCM procedures separately – it is not critical to address how they work with each other at this time.

InterDigital:

Proposal 1: Study following options for functionality-based LCM  1) One AIML functionality per AIML enabled feature 2) Multiple AIML functionality per AIML feature with static applicable conditions 3) Multiple AIML functionality per AIML enabled feature with dynamic applicable conditions. FFS details of applicable conditions.

Proposal 2: At least for some LCM procedures (e.g., model (de)activation, model switching) Model ID may be associated with the logical model rather than physical model.

Proposal 3: The model ID used for model transfer/update and the precise definition of logical model is FFS.

Proposal 4: Study the scope of model ID for different LCM procedures.

Sony:

Proposal 5: RAN1 should consider supporting the individual functionality-based LCM and the common functionality-based LCM for indication of activation/deactivation/switching/fallback.

Ericsson:

Proposal 2	For use cases with one-sided model, responsibility for model LCM is clearly on the side that implements functionality for making model inference. Study, on a per use case basis, the necessity of assistance for model LCM.

Proposal 3	Conclude that two-sided model LCM must enable a gNB to operate with a single gNB-side model that works with different UE-side models.

Proposal 7 Conclude that a possible frequent update of models or applicable conditions (if needed) does not imply frequent updates in UE capabilities within the existing capability framework.

Proposal 8	Model identification (“physical model” ID or “logical model” ID) for a model trained and monitored at the UE-side is not needed.

Proposal 9	Functionality identification is sufficient for a model trained at UE-side but monitored (if needed) at the NW-side. Model identification (“physical model” ID or “logical model” ID) is not needed.

Proposal 10	Methods for supporting pairing of compatible UE part and NW part of a two-sided model should be studied (e.g., for CSI-compression, selecting an encoder of a connected UE to pair with a decoder used by the serving gNB of a network vendor).

Proposal 11	Model identification (“physical model” ID or “logical model” ID) is not needed for two-sided models. Study methods to support pairing of the UE-part and NW-part of a two-sided model.

Proposal 12	RAN1 to outline the requirements to RAN2 on a use-case basis on potential applicable conditions. RAN2 to decide whether applicable condition needs to be explicitly defined and signalled, and if so, the best solution for indicating/reporting the applicable condition.

Proposal 13	For UE-side models, functionality activation/deactivation requires network-UE interaction, whereas individual model activation/deactivation/switching for an activated functionality is transparent to the NW.

Proposal 14	Conclude that for a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification for the applicable use case of the SI.

Proposal 31	Support of ML model-based functionalities, but not related model details, may be reported using the UE capability framework.

Fujitsu:

Proposal-1: To facilitate NW-side model monitoring and model management operations, both the update of model structure and/or the update of model parameters should be identified as a new model. In this sense, the term “model” means model + model parameters.

Proposal-2: To align the understanding on the meaning and assumptions of a model among companies in relevant model LCM discussions, it is suggested to introduce the concept of physical model and the concept of logical model in the discussions of this SI.

Proposal-3: In the cases where either using physical model or using logical model makes no difference to the discussions, only use “model” to cover both possibilities. Physical model and logical model can be described as:

- Physical model:

  - Proprietary-format model.

  - Open-format model.

- Logical model:

  - Logical model described by an explicit dataset.

  - Logical model described by an applicable condition.

  - A model is identified via signaling.

Proposal-4: Considering different requests on the usage of model ID in different LCM procedures, it is suggested to study other types of model ID besides the global model ID. Meanwhile, a hierarchical model indication mechanism needs to be studied if multiple-level model ID is defined.

Proposal-5: Considering the importance to guarantee AI/ML performance and the gain over conventional methods, at least the following issues should be studied and clarified:

- The relationship between model ID and the model related RAN4 test.

- The relationship between model functionality/logical model and the model related RAN4 test.

Proposal-6: For model or model part at UE side, three model identification levels can be considered for further study:

- MID-1: model is identified based on legacy UE-feature-based functionality identification (Level-1), wherein the functionality level refers to pre-defined configurations.

- MID-2: model is identified based on either functionality identification level-2 or logical model identification, wherein the functionality or the logical model links to its applicable condition.

- MID-3: model is identified based on model ID, which is assumed to be assigned to a physical model.

Proposal-7: To have a unified model identification in LCM procedures, an explicit ID for functionality identification or logical model identification can be introduced.

Xiaomi:

Proposal 4: one feature refers to one sub-use case.

Proposal 5: One model could support one or more than one functionalities.

Proposal 6:

- Divide the application conditions into two categories. One category determine the core part of the functionality and another category determine the performance of the functionality

- The functionality should at least be pre-defined based on the application condition category which determine the core part

- For the application condition category which determine the performance of the functionality, further discuss the following options

  - Option 1: Transparent to network

  - Option 2: Predefined in each functionality

  - Option 3: Dynamically reported during the functionality identification

Proposal 7: For model identification

- Some of the information can be shared in RAN1 transparent way

- The other information should be shared via air interface

Panasonic:

Proposal 1: For the discussion purpose, following terminology can be used when necessary. Detailed definition may not be required.

- Physical model consists of binary model and source code model.

- Binary model is executable with specific implementation. Note it can be proprietary or open-format.

- Source code model is before the compilation. Note it can be proprietary or open-format.

- Logical model is the model defined by the input and output relation and test.

Proposal 2: In 3gpp RAN1 discussion perspective, the case that does not produce inference output i.e. the AI/ML model structure without parameters, are not called as a AI/ML model. An AI/ML mode is used only when the certain inference output is generated.

Proposal 5: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

Google:

Proposal 3: The model identification should be based on a model ID indicating a reference model, and study the following options for model identification:

- Option 1: The model identification is based on the model ID for a reference UE-side model

  - The network selects the network-side model to match the corresponding reference UE-side model

  - The UE can identify a UE-side model based on the reference UE-side model

- Option 2: The model identification is based on the model ID for a reference network-side model

  - The network selects a network-side model based on the reference network-side model

  - The UE can select one of the UE-side models that matches the reference network-side model

LG:

Proposal #3: Define the following three stages of AI/ML algorithms

- Model training & deployment stage

- Model inference stage

- Model monitoring & update stage

Proposal #4: Functionality-based LCM should be adopted as a baseline approach, which is applicable for most cases. On top of that, model-based LCM can be considered for some special cases, e.g. two-sided model and/or model transfer scenario, with more focused work scope.

Proposal #5: For UE-sided models, model selection/activation/deactivation/switching should be decided by the UE and no need to inform NW on the number of AI/ML models and the currently activated AI/ML model among them. Instead, UE may report updated UE capability/functionality and/or reliability/confidence of the reported values for NW to decide whether or not to use it.

Proposal #6: For the granularity of functionality, start from FG defined for UE capability, and further consider whether a functionality can cover multiple performance reference of the same feature or not.

Proposal #7: Consider dynamic reconfiguration of UCI reporting for the case of functionality switch/update for the same sub-use-case.

Samsung:

Proposal #3: Study functionality-based LCM for UE-side model where

Alt 1: UE reports the supported AI/ML functionalities

Alt2: UE reports the supported AIML functionalities by mapping them to logical models. i.e., by indicating the group of functionalities supported by a single model.

Note: Logical model implies a reported AI/ML model. UE may transparently have multiple implementations of a logical model.

Proposal #4: For UE side and UE part of two-sided models, study mechanisms to manage

- Timeline and delay requirements for AIML operations, e.g., AIML model/functionality activation, switching,

- Processing capability for concurrently activated AIML models/functionalities

Proposal#5: For UE-sided AI/ML models functionality based LCM is adopted.

  - Network provides model LCM assistance on the basis of specified AI/ML functionalities.

  - Capability reporting relies on the specified list of functionalities.

FFS: whether UE reports the supported AI/ML functionalities or the supported AIML functionalities by mapping them to logical models. i.e., by indicating the group of functionalities supported by a single model.

Proposal#6: For two-sided AI/ML models, differ the conclusion on whether to adopt model-ID or functionality based LCM after evaluating the feasibility of

  - Case1: Two-sided models that work in a vendor-specific manner.

  - Case2: Two-sided models that work in a vendor-agnostic manner.

CAICT:

Proposal 4: A full set for different AI/ML-enabled Features could be defined, which including all item for different functionalities reporting. The reporting details of different functionalities could be flexible based on the full set.

ETRI:

Proposal 1: For the LCM of AI/ML in NR air interface, study the following two-step identification process:

- Step 1: AI/ML functionality identification step

  - The gNB sends a UE capability enquiry to the UE.

  - The UE reports UE capability information to the gNB.

  - UE capability information includes supportable configurations for each AI/ML functionality.

- Step 2: AI/ML model identification step

  - The gNB sends configurations related to AI/ML functionalities.

  - Configurations related to AI/ML functionalities include functionality ID

  - The UE reports model ID(s) that can be supported for the configurations/functionality ID.

Proposal 2: For the LCM of AI/ML in NR air interface, study AI/ML operation scenario and/or operation area identification during the AI/ML model identification.

Proposal 3: For the LCM of AI/ML in NR air interface, study the following two options for AI/ML model information report procedure during/after the AI/ML model identification.

- Option 1: AI/ML model information report in response to the AI/ML related configuration

- Option 2: AI/ML model information report in response to the AI/ML model enquiry

Proposal 4: For the LCM of AI/ML in NR air interface, study a unified procedure to support both functionality-based LCM and model-ID based LCM.

Proposal 5: For the LCM of AI/ML in NR air interface, study AI/ML functionality ID and model ID based activation/deactivation of AI/ML functionality/model.

Proposal 6: For the LCM of AI/ML in NR air interface, study AI/ML functionality/model activation and/or deactivation reflecting other UE status (e.g., DRX).

Proposal 7: For the LCM of AI/ML model in NR air interface, study UE capability for AI/ML model inference performance report:

- Capability A: Not capable to provide AI/ML model inference performance

- Capability B: Capable to provide AI/ML model inference performance with GT

- Capability C: Capable to provide AI/ML model inference performance estimate without GT

CMCC:

Proposal 6: For the model description information during model identification, the following aspects could be considered:

- Model functionality

- Model applicability scenarios, configurations

- Information on model input

- Information on model output

- Information on assistance information

Proposal 7: For functionality identification, the functionality ID can be assigned by the network to facilitate functionality-based LCM procedure.

Proposal 8: For the description information during functionality identification, the following aspects could be considered:

- Model functionality

- Applicability scenarios, configurations of models for the functionality

- Information on model input type(s)

- Information on model output type(s)

- Information on assistance information

Proposal 9: For functionality identification, there could be at least one functionality defined within an AI/ML-enabled sub use case:

MediaTek:

Proposal 2: The mechanisms for data collection, model training, model monitoring and model inference are use case specific and studied for each use case.

Proposal 3: The mechanisms for model transfer, model configuration, model selection, model switching, model activation/deactivation, fallback and UE capability reporting can be common for different use cases and be studied in the general aspect.

Proposal 4: RAN1 doesn’t intend to design two distinct mechanisms for functionality-based and model-ID-based LCM. Functionality-based and model-ID-based LCM procedures should share as much commonality as possible.

Proposal 17: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 18: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is supported by the UE

- Model transfer to indicate which AI/ML model is being transferred to the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 19: A model index is assigned to each model for model activation/deactivation/switch/selection/fallback by the network through model configuration.

Proposal 20: Study what associated information needs to be provided through model identification for the case that UE sided model is generated and training at the UE side and leave the model identification procedure to RAN2 discussion.

Proposal 21: For each AI/ML model, at least following associated information needs to be known. FFS on other information.

- Model functionality (e.g., beam management, CSI compression, positioning)

- Applicable scenario/configuration and site

- Information on model input

- Information on model output

- Information on pairing between UE-sided part and network-sided part of two-sided model

Nvidia:

Proposal 5: From a common framework’s perspective, introduce “in-distribution generalization” and “out-of-distribution generalization” in the terminology list and leave the details of generalization types to the discussion of each use case.

Proposal 6: In-distribution generalization: training and test data have the same distribution.

Proposal 7: Out-of-distribution generalization: training and test data do not have the same distribution.

Proposal 8: Coordinate with SA5 on AI/ML model life cycle management.

Apple:

Proposal 1: Define functionality-based LCM and model ID based LCM independently.

Proposal 2: Functionality granularity is sub-use case dependent and can be defined in later stage of WI similar to legacy UE feature discussion.

Proposal 3: Functionality based LCM procedure can be used for one sided model without model transfer.

Proposal 4: RAN1 discussion focus on the use cases and requirement for model ID.

Proposal 5: Model ID based LCM procedure can be used for two-sided model, and one-sided model with model transfer.

Proposal 6: AI model identification can be done between vendors/operators during product development phase as part of feature alignment and training process for two-sided model, or part of feature alignment for one sided model for known model structure.

Proposal 7: UE capability report can be used to report the UE supported AI model ID, using legacy UE capability framework.

Proposal 8: 3GPP define model ID and model description. Model ID may include PLMN ID, vendor ID, functionality ID and version number etc.

Proposal 9: Model description include scenarios/configurations for model inferencing, model input/output information, model file type/size/compression status etc.

Lenovo:

Model identification would have more precise granularity on the information to be shared than Functionality identification.

To align the understanding on ‘AI/ML Model’, ‘Functionality’ and the information to be shared in the identification procedure.

Confirm the working assumption on ‘AI/ML Model’ that is a kind of algorithm without specification impact, and the ‘identification’ procedure is used to facilitate any necessary operations over air interface.

Not use Model ID to identify a physical AI/ML model, represented by either a model structure with parameters or a compiled binary file.

Model ID can at least facilitate the model-related information sharing between NW and UE in the model identification procedure.

The model-related information regarding the AI/ML model needs to be discussed per sub use case, which can facilitate the operations over air interface in Model ID-based LCM, e.g., to support the mechanisms of decision by the network for the model selection, activation, deactivation, switching, and fallback.

The functionality to be identified during ‘Functionality identification’ could be the sub use cases with some pre-defined configurations and/or application conditions with some pre-defined scenarios, which can be identified at NW side.

Introduce a hierarchical Model ID, one level is for functionality indication and the other level is for the multiple models within the functionality, which can be applied for both Model ID based LCM and Functionality based LCM.

The information and signaling exchanging between NW and UE for LCM can be tagged with the Model ID to potentially manage multiple models.

Functionality-based LCM can provide feasibility for UE to select AI/ML model, based on the information shared during the identification procedure.

Qualcomm:

Proposal 2: Study the mechanism/signaling to handle UE capabilities dynamically for AI/ML enabled features, e.g., in the presence of handover.

Proposal 3: Introduce the following terminologies.

| Physical AI/ML model | A model that tangibly exists, e.g. proprietary-format model, open-format model |
| --- | --- |
| Logical AI/ML model | A model that is visible and identified (via a model ID) in the air interface and hence used in signaling. Note: A logical model may be implemented by one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single (logical) model ID. |

Proposal 4: It is acknowledged that multiple physical models may exist transparently under one model ID.

Proposal 5: A pairing ID is a logical ID that indicates compatibility between the UE-side and NW-side model of a two-sided model.

- For example, all encoders developed resulting from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training, UE-side encoders trained based on the same NW-side model may be associated with a single pairing ID.

- The pairing ID may also serve as a model ID. That is, all the encoders associated with a pairing ID may be identified as a single model ID.

Proposal 6:

| Meta information | Supplemental information being provided about a model during model identification process. |
| --- | --- |

Proposal 7: Functionality-based LCM is applicable to UE-side models in Collaboration Level y.

Proposal 8: Models are identified by model IDs, and associated meta information known at the network is used for selection of the right model at the inference time.

Proposal 9: Model-ID-based LCM is applicable for both one-sided and UE-part of two-sided models.

Proposal 10: Model-ID-based LCM is applicable to Collaboration Level z.

KDDI:

Proposal 1:

The acknowledgement that a model that the NW is aware of may be logical.

Proposal 2:

We should study the need for the NW to identify physical models.

Proposal 3:

Clarify boundaries and commonalities among AI/ML enabled features, functionality, and applicable conditions.

Rakuten:

Proposal 4
Model ID should be changed upon the model update so that UE could know the model is updated.

Proposal 5
Performance monitoring should be done with awareness of model ID.

Proposal 6

Both gNB and UE can indicate model ID depending on the framework deployed.

Proposal 7

Model ID should be able to be indicated separately with functionality ID, in addition to the indication within functionality ID.

Proposal 8

Functionality should be related with the model ID and the collaboration level.

- Solution 1: UE driven - identify model structure identically in functionality identification

- Solution 2: NW driven - identify model structure identically in functionality identification

- Solution 3: identify model structure by differentiating model ID with consideration of model structure

- Solution 4: UE indicate deployed model structure to the network, separately.

- Solution 5: Model performance requirements are specified in spec. for each use case. UE selects appropriate model structure that satisfies specified requirements, without awareness from the network.

ATT:

Proposal 1: Study a common framework for model identification to indicate a common understanding between network and UE across all sub-use cases.

Proposal 2:  Characterize AI/ML model identification as a function of the associated information needed for the common understanding between the UE and the network, used for AI/ML LCM procedure

Proposal 3: Define three levels of AI/ML model identification categories: model functionality, AI/ML untrained model structure, and AI/ML trained model.

Proposal 4: Functionality based LCM procedure is used for both UE-sided and two-sided models.

Proposal 5: For UE-side models and UE-part of two-sided models, functionality-based LCM is used.

Proposal 6: For UE-sided models, model-ID-based LCM procedure can be optional.

Proposal 7: For two-sided models, both functionality and model-ID-based LCM are used.

Proposal 8: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Proposal 9: Study the necessity for model-ID based LCM to support any additional functionalities that will not be supported through functionality-based LCM.

NEC:

Proposal 2: Support model-ID based lifecycle management also for NW-sided model.

Proposal 3: Model-ID can be explicitly included in LCM signaling to activate/deactivate/switch/select a specific AI/ML model.

Proposal 4: For a two-sided model, study methods to align NW part and UE part of one AI/ML model, e.g., assigning NW part ID and UE part ID in addition to model-ID.

NTT Docomo:

Proposal 3: Functionality should include all AI/ML-enabled feature related information that NW should be aware of in the NW operation.

Proposal 4: Functionality should be associated with applicable condition (e.g., applicable configuration/scenario/deployment and paired model), nominal input information (e.g., required measurement and assistance information for model input) and nominal output information (e.g., derived information from model output)

Proposal 5: AI/ML enabled feature can be defined by input type and output type.

Proposal 6: Applicable condition (e.g., configuration/scenario/deployment) should be verified by the test.

Proposal 7: Clarify what is identified by model ID.

Proposal 9: Consider the LCM framework with and without online training, separately.

Proposal 10: Model-ID-based LCM should be considered for models trained by NW side.

Proposal 11: Functionality-based LCM should be considered for models trained by NW side.

Proposal 12: Check how often the availability of UE side model/functionality could be changed.

Proposal 13: Model level LCM within a functionality should not include the fallback operation.

Proposal 14:  Further clarify what procedure corresponds to the model identification.

Proposal 15: Assistance signalling including scenario/configuration ID should be assumed for dataset collection, model training, and model inference of scenario/configuration-specific AI models.

TCL Communication:

- Proposal 3: The generalization of an ML model is needed to be discussed, according to model deployment, model switching, and alignment of applicable settings.

##### Issue 6-7: Overall framework of functionality and model operation

From the company proposals, the following 4 cases are identified for LCM operations.

Case-1: Functionality based LCM, in which functionality = feature group

In RAN1-112 meeting, it was agreed to reuse legacy 3GPP framework of features as a starting point for discussion for AI/ML functionality identification and hence for functionality-based LCM. Associated with this agreement, it has been observed that some companies consider functionality-based LCM at feature level by interpretating functionalities as feature groups and functionality identification as the index of the feature group as presented in Table 1. This apparently results in performing LCM at feature level.

| Features | Index (Functionality identification) | Feature group (Functionality) | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| X. AI/ML features | X-1 | AI/ML based beam prediction in temporal domain | - UE can perform T (ms) prediction - Configuration of Set A is xxx and Set B beams are yyy. | Component 1: candidate values for T = {10, 20} (ms) | Optional with capability signaling |
| X. AI/ML features | X-2 | AI/ML based beam prediction in spatial domain | … | … | … |

Table 1 Example of feature group indicating model based on TR 38.822

Case-2: Functionality based LCM, in which functionality = static RRC configurations

Instead of considering the granularity of functionality as feature group, it has been seen that many companies argue that functionality can be more granular than a feature group or a sub-use case. Hence, a functionality corresponds to a combination of specified RRC configurations that each UE defines/reports without a need of global ID.

Case-3: Functionality based LCM, in which functionality = static RRC configurations + dynamic scenario/site ID

This point of view incorporates the definitions of applicable conditions to functionalities. According to that, a functionality corresponds to specific combinations of scenario, RRC configuration, and sites, that is identified offline by a vendor or via collaboration across vendors and assigned a (global) ID. This approach is also well-aligned with the concept of logical model ID discussed thoroughly via emails. There have been observations from companies that logical models and functionalities are similar or the same.

Case-4: Model-ID based LCM

From model ID based LCM, it is possible to perform LCM at model level instead of UE level observed in functionality-based LCM. Here, what a model means has been extensively discussed during the offline email discussions. In the offline email discussions, logical model and physical model were introduced such that a physical model refers to a model that tangibly exists, e.g., proprietary-format model, open-format model, and a logical model refers to a logical unit whose applicability may be specific combinations of RRC configurations, scenarios, and sites, as determined offline by a vendor or via collaboration across vendors. A logical model may be implemented by one or multiple physical models, e.g. multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms or multiple physical models transparent to NW that are signaled as a single model ID. This interpretation of a model is very similar to or the same as the functionality interpretation in Case-3. Functionality and Model have the same motivation of creating a common understanding between the NW and the UE to facilitate LCM. Given this observation, the FL would like to proceed by introducing the notion of logical model and capture Case-3 as a part of (logical)-model-ID-based LCM, limiting the interpretation of Functionality to only Case-1 and Case-2.

For aligned discussion in the functionality/model terminologies, the above observations can be summarized as

- Feature-based operations (Case-1 and Case-2): This can be seen as a common denominator where UE capability is reported for each Feature or for a combination of specified RRC configurations of each Feature.

- ID-based operations (Case-3 and Case-4): This is more granular and may be needed, where companies used various terms like functionality ID, model ID, logical model ID, pairing ID, applicability ID, etc. to denote/achieve similar concepts. An ID may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors.

What an ID represents in the “ID-based operations” may be dependent on various factors (e.g., collaboration level, UE-side or two-sided operations, use cases). For example,

  - For UE-side models in Collaboration Level y, an ID may be assigned to a logical unit, which companies variously refer to functionality, logical model, applicability condition, etc.

  - For two-sided models in Collaboration Level y, a pairing ID may be used.

  - For Collaboration Level z, an ID may correspond to a physical model that is transferred from NW to UE.

Thus, the FL proposes to use the term functionality and model in the following ways:

- Functionality is something that is defined based on pre-defined configurations in the specification, e.g., legacy-like Features/FGs and their components.

  - Case-1 and Case-2 correspond to functionality-based LCM.

- Model is something that is created by a vendor or via alignment/collaboration across vendors and identified between NW and UE.

  - Case-3 and Case-4 correspond to model-ID-based LCM.

###### [FL1] Proposal 6-7a:

Align the functionality and model terminologies such that:

- Functionality refers to either an entire Feature/FG or a combination of specified RRC configurations of a Feature.

- Correspondingly, functionality-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations of a Feature.

- It is acknowledged that a model that the NW is aware of may be logical and that multiple AI/ML models may exist transparently to NW under a model ID.

- Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors.

- Which aspects should be specified as components of a Feature/FG available for functionality, and which other aspects should be considered as signaling such as assistance information that are made available for model training and identification will be discussed in each sub-use-case agenda.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | Panasonic, LG, Samsung (Question to FL) |
| Company | Comments |  |
| vivo | We are fine in general. Some wording update considering how offline is done need to be studied further. Align the functionality and model terminologies such that: - Functionality refers to either an entire Feature/FG or a combination of specified RRC configurations of a Feature. - Correspondingly, functionality-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations of a Feature. - It is acknowledged that a model that the NW is aware of may be logical and that multiple AI/ML models may exist transparently to NW under a model ID. - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors. - Which aspects should be specified as components of a Feature/FG available for functionality, and which other aspects should be considered as signaling such as assistance information that are made available for model training and identification will be discussed in each sub-use-case agenda. |  |
| Google | Support in principle. |  |
| Panasonic | On the following bullet point, - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors. we are not sure the meaning of "may be determined offline by a vendor or via collaboration across vendors". We think it also can be specified in 3gpp. The description may mean to exclude the case to be specified and also excluded the involvement of the network operator. Similar to case y to z5 discussion, just UE-side / NW-side would be sufficient as it is not limited to vendors. Therefore, we propose to modify as following. - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors UE-side / NW-side. |  |
| LG | If we go this way, the boundary between functionality and model seems very unclear. The proposed difference seems that functionality-based LCM is static (RRC-based) and model-based LCM is dynamic but this is not aligned with our understanding of previous agreements. The key difference between the two approaches is whether NW is aware of UE-side/part model or not, not about whether the relevant signaling is dynamic or static. [Mod] If we acknowledge that a model may be logical (in the sense that a UE may declare the entire “functionality” as a single model), indeed the boundary between functionality and model overlaps. Please see the FL discussion on Case 3 and Case 4 within this Issue. That’s precisely the motivation behind this proposal. Given that Case 3 can be covered by a (logical) model, we can narrow down the interpretation of functionality to avoid the overlap between the two notions. |  |
| NTT DOCOMO | It is better to differentiate feature-based operation and ID-based operation. We understand the intention of this proposal is to exclude the applicable scenario or configuration information from the functionality. However, the current description is not clear, because even scenario ID or configuration ID also can be reported as a component of feature. Hence, it should clearly mentioned that scenario ID or deployment ID is not associated with feature-based operation. Also, the functionality is the granularity that NW can be aware of about UE side models, as well as the unit of NW control of UE side models. If the UE reports the AI/ML enabled feature without applicable information of scenario/deployment/NW configuration, NW assumes that UE can handle all scenarios/deployments/NW configuration in that AI/ML enabled feature. However, it is almost impossible as observed in the generalization evaluation results of each sub use case. As it is not typical that one UE handles the AI/ML enabled feature to all scenarios/deployments, the functionality should be associated with applicable information (e.g., scenario ID, deployment ID, NW configuration ID). Otherwise, each reported AI/ML enabled feature is required to be generalizable to all scenarios/deployments. From the above two points, we suggest modifying the proposal as follows Modification A) - AI/ML enabled feature functionality refers to either an entire Feature/FG or a combination of specified RRC configurations of a Feature without applicable information about scenarios and sites. - Correspondingly, functionality feature-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations of a Feature without applicable information about scenarios and sites. - FFS the granularity of functionality. Modification B) - Functionality refers to either an entire Feature/FG or a combination of specified RRC configurations, applicable scenarios, and applicable sites of a Feature. Correspondingly, functionality-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations, applicable scenarios, and applicable sites of a Feature. |  |
| ETRI | This is a very complex issue. Therefore, to avoid confusion, we believe that functionality-based LCM and model-ID-based LCM should be separated into different bullets. For the functionality-based LCM, the 1st, 2nd, and 3rd sub-bullets can be applicable, and we are supportive for this. However, model-ID-based LCM requires more discussion. It is not even clear whether the model-ID-based LCM mentioned here means an LCM that operates only with model-ID without defining functionality. |  |
| CATT | Generally OK with the categorization. As this proposal is to align the understanding of functionality and AI/ML model, we think vivo’s update (on the forth sub-bullet) is better. Prefer not debate whether/how a model or functionality is identified or determined here. |  |
| Samsung | Thank you FL. We are generally find with the direction of discussion. But we have to following clarification questions, comments. - Functionality in Case 1 and Case 2 could also include scenario information, if a specified AIML feature has association with certain scenarios, e.g., CSI prediction features (groups) for high and medium mobility could be distinguished as two feature groups. - For Case 2, in what basis the UE group functionalities and report them? UE can report configurations and scenarios that it can support by a single model (logical). In this case, a reported functionality (scenario + configurations sets) maps to a logical model. - Moreover, it is worth mentioning Case 2 with model transfer can support site-specific models, e.g., Case Z1: UE-side trains a model store at the network in proprietary format. Network just use local ID to identify the models. UE reports the supported functionalities of the transferred models. [Mod] This is categorized as a model ID based LCM in the Proposals. - In principle, we agree Case 3 is advanced approach for LCM assistance towards site specific models. However, Rel-18 is not evaluating site-specific models. It is not clear whether Case 3 is needed to be considered at this stage. |  |
| Xiaomi | - In current discussion, what is one AI/ML feature refers to should be clarified as well - We are OK with the 1st and 2nd bullet - For the 3rd bullet, we could understand FL’s intension. But current statement it would make the boundary between functionality and logic model blurry. For example, in which case, one set of AI models can be viewed as one logic model. For this bullet, we suggest to handle it until we reach consensus on the understanding of logic AI model - For the 4th bullet, we share similar consideration with vivo and Panasonic For the 5th bullet, generally we are OK |  |
| NEC | For the first sentence, we would like to understand more about the granularity of “Feature” and “(sub-) use case”. - For the rest, we are generally having the same understanding. |  |
| ZTE | We have following suggestions: - Functionality refers to either an entire Feature/FG(i.e., a sub use case) or a combination of specified RRC configurations of a Feature(i.e., a sub use case). - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors static parts(e.g., specified RRC configurations) and dynamic parts(e.g.,scenarios,sites and datasets). The dynamic parts may be determined offline/online by a vendor or via collaboration across vendors - Which aspects should be specified as components of a Feature/FG available for functionality, and which other aspects should be considered as signaling such as assistance information that are made available for model training and model description information available for model identification will be discussed in each sub-use-case agenda. One question for clarification: If functionality identification only includes Case-1 and Case-2, does it preclude dynamic parts(e.g.,scenarios,sites and datasets) for a functionality? [Mod] We can say that model ID is a UE/NW-side alignment way of handling dynamic parts, but I don’t think we’re precluding the handling of dynamic parts in functionality-based LCM via other means. See Proposal 6-12. |  |
| OPPO | We are generally OK for the proposal. But we do not think the third bullet is needed. We agree this is true. But with no specification impacts, it does not need to be captured in an agreement. If needed, the 3rd bullet can be agreed as a separate conclusion. |  |
| vivo | Also for the third bullet. The wording “transparently” needs to be deleted since the existence of multiple physical models under a model ID may not be transparent. This can be further studied. - It is acknowledged that a model that the NW is aware of may be logical and that multiple AI/ML models may exist transparently to NW under a model ID. |  |
| Lenovo | In general, we agree to discuss such alignment. If we want to clearly define the AI/ML functionality and model, we suggest following updating based on our understanding: - Functionality refers to either an entire AI/ML-enabled Feature/FG or a combination of specified RRC configurations of an AI/ML-enabled Feature. - Correspondingly, functionality-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations of a Feature. - FFS: the specified RRC configurations - It is acknowledged that a A model that the NW is aware of may be logical and that multiple physical AI/ML models may exist transparently to NW under an identified model ID. - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and the association may be determined offline by a vendor or via collaboration across vendors. - FFS: the specified RRC configurations - Which aspects should be specified as components of a Feature/FG available for functionality, and which other aspects should be considered as signaling such as assistance information that are made available for model training and identification will be discussed in each sub-use-case agenda. |  |
| CAICT | We are fine with vivo’s update. |  |
| Huawei, HiSilicon | In our understanding, the difference between Case 3/4 and Case 1/2 is that a globally unique ID is needed for Case 3/4, while Case 1/2 does not need it. The difference between Case 3 and Case 4 is that for Case 4, the NW and UE builds an aligned and unique anticipated performance for per model basis, while for Case 3, the scenario ID/dataset ID is only applicable for data categorization, while NW has no knowledge on the specific model that is developed at UE, e.g., UE can develop TF, or CNN, using the dataset collected based on the same scenario ID/dataset ID but shows totally different target performances. Therefore, for Case 4, there needs a model registration procedure to let NW know this unique model ID at UE side. We are fine to FFS Case 3, but for Case 4, we may need to emphasize that the model ID indicates the information of anticipated performance, and that the model registration procedure is needed. In addition, the potential SA impact is also noted. For whether the registration is online or offline, the definition is not clear, so removed from the bullet. - For functionality identification (Case 1/2) - Functionality refers to either an entire Feature/FG or a combination of specified RRC configurations of a Feature. - Correspondingly, functionality-based LCM operates based on specified Feature/FG or a combination of specified RRC configurations of a Feature. - No globally unique ID is needed. - For model identification (Case 4) - It is acknowledged that a model that the NW is aware of may be logical and that multiple physical AI/ML models may exist transparently to NW under a model ID, but with comparable anticipated performance. - Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of RRC configurations, scenarios, and sites and may be determined offline by a vendor or via collaboration across vendors. - SA impact may be needed for aligning the model ID and model related information between UE side and NW side. - Which aspects should be specified as components of a Feature/FG available for functionality, and which other aspects should be considered as signaling such as assistance information that are made available for model training and identification will be discussed in each sub-use-case agenda. FFS for functionality identification introduced with globally unique scenario ID/dataset ID (Case 3). |  |
| Fujitsu | We are fine with the proposal in general. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | Support in general the original proposal. Share the view that “may be determined offline by a vendor or via collaboration across vendors" can be removed, how the model IDs are shared and defined can be FFS. Support the modifications by DCM. Any LCM need to take into account applicable scenario/condition of a trained model, since AI/ML model cannot work always as in existing non-AI algorithms. Functionality-based LCM need to include applicable scenario/condition also, although exactly how to describe the applicable scenario/condition can be FFS. The difference between functionality-based and model-ID-based is, functionality-based does not use model ID, and models are not identified at the network. |  |
| Nokia/NSB | Not OK. Comments as below, - First bullet: Functionality should not refer to the entire feature. If companies have that proposal, we would like to know why to define another terminology to replace AI/ML-enabled feature. To our reading, it is up to the network to define any number of functionalities based on UE-reported applicable conditions. We are open to discussing the number of supported functionalities if supporting multiple functionality configurations are problematic to the UE, but that shall be discussed separately. - First bullet: the wording “Combinations of specified RRC configuration” is not clear. We shall discuss functionalities using/considering the applicable conditions reported by the UE. Otherwise, this statement is not helping much. - Second bullet: Functionality LCM can also be dynamic signalling. What is the reason to limit this to RRC? Functionality LCM will be clearer if we first discuss applicable conditions and how to create functionalities around them. - Not clear why to brin-up a logical model to this discussion. For us, logical model = functionality. Let’s not go back to the wording of using “model” in unnecessary form when there is no clear need for that. If the need is to define an ID based LCM, then functionality LCM can also refer to functionality-ID-based LCM. Also, Logical model is not agreed term and functionality is introduced to this purpose. - 4th bullet: unclear to us. Is this for the two-sided case or in general? Better to clarify what is referred by the model-ID discussed here. [Mod] If we acknowledge that a model may be logical (due to transparent model operations at UE), a lot of what you’re saying (i.e., logical ID-based operations) is now covered by (logical) mode-ID-based LCM. It’s just a naming issue, whether we call such ID-based operations as functionality-ID-based LCM or (logical)-model-ID-based LCM. |  |
| Apple | We agree the understanding of case 1, case 2 and case 4. For case 3, we do not see the need for a scenario ID and site ID. It is also not clear how such global unique ID can be defined. In simulation, scenario refers to UMa, Umi, indoor etc, site refers to (one factory with fixed seed). However we do not see this is part of signaling as UE can figure out indoor/outdoor, location by itself. From signaling point of view, it will be the same as case 1 and case 2. In addition, case 3 include the proposals on dataset ID, which indicates the paring information for two-sided model. In this case, we believe it is part of case 4. |  |
| KDDI | Support the proposal in principle. |  |
| Sony | We think the vivo’s update is fine. |  |
| AT&T | We support the proposal and are fine with vivo edit for the 4th bullet. |  |
| Futurewei | We are fine with the direction in general. |  |
| InterDigital | We are generally OK with the proposal to align the terminologies. |  |
| Spreadtrum | Generally we are fine |  |
| Intel | Support the original proposal except just delete the term “offline” – the rest looks ok |  |

###### [FL2] Proposal 6-7b:

Align the functionality terminology such that:

- Functionality refers to either an entire AI/ML-enabled Feature/FG (i.e., a sub use case) or a combination of specified RRC configurations of an AI/ML-enabled Feature.

- Correspondingly, functionality-based LCM operates based on specified AI/ML-enabled Feature/FG (i.e., a sub use case) or a combination of specified RRC configurations of an AI/ML-enabled Feature.

  - FFS: Signaling to identify dynamic parts (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

- FFS: the granularity of functionality

- FFS: which aspects should be specified as components of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

Align the model terminology such that:

- It is acknowledged that a model that the NW is aware of may be logical and that multiple physical AI/ML models may exist transparently to NW under a model ID.

- Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of static parts (e.g., specified RRC configurations) and dynamic parts (e.g., scenarios, sites, and datasets) and. The dynamic parts may be determined offline/online between UE-side and NW-side by a vendor or via collaboration across vendors.

- FFS: Which aspects should be considered as signaling such as assistance information that are made available for model training and identification to be included into model description information during model identification will be discussed in each sub-use-case agenda.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-8: Model definition clarification

AI/ML model does not necessarily refer to a single physical model since there always exists implementation freedom to implement more than one physical model under the AI/ML model visible to NW. Thus, an AI/ML model can refer to a number of physical models grouped according to a rule/applicable condition.

AI/ML model refers to a logical unit whose applicability may be specific combinations of RRC configurations, scenarios, and sites, as determined offline by a vendor or via collaboration across vendors, and identified by an ID.

###### [FL1] Proposal 6-8a:

Extend the notion of a model into a logical model.

- Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model.

- Logical AI/ML model is a model that is identified (e.g., via a model ID) and used in signaling.

- Note: A logical model may be implemented by one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single model ID.

- Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format).

Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model.

- In particular, model-level-LCM is based on logical models, and UE may have physical model operations transparently to the NW.

- FFS: NW's awareness of physical model operations, such as for monitoring, test, certification, and interruption during physical model switching

A logical model identified by a model ID may refer to different units for different cases. More precisely, a model ID may correspond to

- For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable scenarios, configurations, sites.

- For two-sided models in Collaboration Level y: A pairing ID indicating compatibility between UE-side and NW-side models

- For Collaboration Level z: A physical model that is transferred from NW to UE.

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic | LG |
| Company | Comments |  |
| vivo | We support this proposal to move forward with the following part deleted. The spec impact or signaling design need to be considered further. We can focus on the basic understanding of logical model. Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model. - In particular, model-level-LCM is based on logical models, and UE may have physical model operations transparently to the NW. - FFS: NW's awareness of physical model operations, such as for monitoring, test, certification, and interruption during physical model switching |  |
| Google | Support the proposal in principle. |  |
| LG | Similar feeling as previous proposal. This would blur the boundary between functionality-based LCM and model-based LCM. Functionality-based LCM is based on what we have in 3GPP (e.g. UE feature, RRC/MAC-CE/DCI), so it is something related to ‘enhancement’ but model-based LCM is something ‘new’. Thus, before getting into details of model-based LCM, we suggest that we should try to agree on whether model-based LCM is needed on which case. [Mod] Whether model-ID-based LCM is needed depends on the clarification on the model. |  |
| NTT DOCOMO | Generally fine with the proposal. However, we think it is better not to use the applicable “condition” to avoid the misleading. If “condition” is used, it gives the impression that the model is activated/deactivated when that condition is satisfied like conditional handover. However, we believe that the supporting conditional activation/deactivation is not intended in this proposal. Then, the following modification should be made to avoid that confusion. Applicable conditions information of the model [Mod] The proposal does not mention applicable condition. |  |
| ETRI | We understand the background of this topic, but we do not think it is necessary to introduce a specific definition of an AI/ML model. Rather than this, it seems better to describe whether the identifier of the AI/ML model is a global model ID or a local model ID. For example, in Type A1 of Proposal 6-11a, the meaning of model ID is ambiguous. However, considering that it is an offline method, a more precise expression would be the global model ID. This makes Type A1 more explicit. Otherwise, we have to discuss how the model ID for the physical AI/ML model and the model ID for the logical AI/ML model should be different, which adds to the confusion of the discussion. [Mod] The proposal is more of an acknowledgement that a model may be logical, that a model ID is assigned to a logical model, and that the logical model may or may not be physical. These are all inevitable facts that we will have to deal with. |  |
| CATT | Fine with the first part, the clarification on logical model and physical model. For the second and third part, from specification point of view, we only need one definition, i.e. either logical model or physical model. Not prefer to incorporate both for further study. For example, if we go with logical model, even for model transfer when multiple physical models are transferred, they can still be viewed as one logical model (and associate with one model ID). [Mod] Yes, the proposal says that an ID is assigned to a logical model. |  |
| Samsung | In order to avoid misconception, we suggest addition of this: - Logical AI/ML model is a model that is identified (e.g., via a local or global model ID) and used in signaling. |  |
| Xiaomi | We share similar consideration with vivo. We could align the basic understanding first |  |
| NEC | We are supportive to have a logical model ID concept which would be useful for LCM discussion. However, in our understanding, physical model ID is evitable if considering model transfer/delivery. That is the reason why we cannot support stating that “model-level-LCM is based on logical models”. [Mod] Some misunderstanding here in the “logical model” interpretation. The proposal is basically saying that a model ID is assigned to a model, and the model may or may not be physical, and we call whatever is assigned a model ID a “logical model”. It’s clearly stated in the proposal that logical and physical models are not mutually exclusive, and that in case of collaboration level z, a logical model identified by a model ID may refer to a physical model. Also, it is not clear what does “UE may have physical model operations transparently to the NW” means. What kind of physical model operations are we discussing here? [Mod] physical model switching, physical model update. |  |
| ZTE | Better to have separate proposals for above discussion points. In general, we can start with the understanding on logical model and physical model. |  |
| OPPO | Firstly, we do not think this proposal should be related to model transfer. The logical model concept was clarified during the meeting interval email discussion. But during model transfer, the “model” is only a non-executive compressed file, which is not a physical or logical model in inference/monitoring procedure. In model transfer procedure, the signaling is for transferring a model file, which is different from the signaling activating/de-activating a logical model. So the transferred model is not the activated logical model. Do we need to use the “logical model” in model transfer procedure? Secondly, do we need to explicitly introduce the new concepts “logical unit” and “pairing ID”? We suggest to avoid them. The modification we suggested is as below: Extend the notion of a model into a logical model. - Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model. - Logical AI/ML model is a model that is identified (e.g., via a model ID) and used in signaling. - Note: A logical model may be implemented by one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single model ID. - Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format). [Mod] Some misunderstanding here in the “logical model” interpretation. The proposal is basically saying that a model ID is assigned to a model, and the model may or may not be physical, and we call whatever is assigned a model ID a “logical model”. Therefore, logical and physical models are not mutually exclusive, and that in case of collaboration level z, a logical model identified by a model ID may refer to a physical model. Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model. - In particular, model-level-LCM is based on logical models, and UE may have physical model operations transparently to the NW. - FFS: NW's awareness of physical model operations, such as for monitoring, test, certification, and interruption during physical model switching A logical model identified by a model ID may refer to different unitmodels for different cases. More precisely, a model ID may correspond to - For UE-side models in Collaboration Level y: A logical unitmodel that is identified between UE and NW based on applicable scenarios, configurations, sites. - For two-sided models in Collaboration Level y: An pairing ID indicating compatibility between UE-side and NW-side models - For Collaboration Level z: A physical model that is transferred from NW to UE. [Mod] “unit” was used as a generic word to describe what a logical model may mean. A “unit” is not a new concept. Pairing ID was removed in the new proposal. |  |
| Vivo2 | We would like to further point out that physical model can also be identified and used in signaling. Thus prefer the following update Extend the notion of a model into a logical model. - Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model. Physical model can be identified and used in signaling. - Logical AI/ML model is a model that is identified (e.g., via a model ID) and used in signaling. - Note: A logical model may be implemented by one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single model ID. - Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format). Another example is that the physical model can be associated with a logical model for LCM control. [Mod] Some misunderstanding here in the “logical model” interpretation. The proposal is basically saying that a model ID is assigned to a model, and the model may or may not be physical, and we call whatever is assigned a model ID a “logical model”. It’s clearly stated in the proposal that logical and physical models are not mutually exclusive, and that in case of collaboration level z, a logical model identified by a model ID may refer to a physical model. If we go with the route where a logical model or a physical model may be identified, it leads to two different LCM discussions. So, better to say that whatever is identified is logical, and that a logical model is sometimes also a physical model. |  |
| Lenovo | It is a long proposal, and many issues are coupled, though we understand FL’s intention to have a aligned understand on ‘model’. In general, we think the model should be a logical one for management, and physical model can be included with related meta information is provided. In addition, we don’t think ‘pairing ID’ is needed, since well-designed model ID can be reused for two-sided models. |  |
| CAICT | We are general fine with the proposal. We share the similar view as vivo2’s comment that physical model could be identified and used for signaling. [Mod] Please see response to vivo2. |  |
| Huawei, HiSilicon | As the comments in the last proposal, a key difference of the model identification, as opposed to the functionality identification, is the anticipated performance. In addition, we think for Level y, there is no need to identify the site? We think the site-specific model is only applicable to model transfer mode. A logical model identified by a model ID may refer to different units for different cases. More precisely, a model ID may correspond to - For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable scenarios, configurations, sites and aligned anticipated performance. - For two-sided models in Collaboration Level y: A pairing ID indicating compatibility between UE-side and NW-side models to achieve anticipated performance of pairing - For Collaboration Level z: A physical model that is transferred from NW to UE. [Mod] Why is site-specific model not applicable in Level y? Why can’t we develop site-specific UE-side models and store them in a UE-side server for Level y operations? |  |
| Fujitsu | We suggest this proposal can be discussed part by part. The first part and the second part are fine for us. Regarding third part, we think the meaning of “model ID” should be clarified. Does it the same as model ID agreed in RAN2, which is a global ID? Besides, pairing ID for two-sided model seems need further discussion. [Mod] Pairing ID has been removed. Whether an ID is global or local will need further clarification. |  |
| NVIDIA | Support the proposal in principle. It however would be easier for the group to converge if the different parts of the proposal are discussed separately. |  |
| Ericsson | It is unclear what is meant by “units” in the last paragraph. Consider change to "A logical model identified by a model ID may correspond to:" We don’t agree with the following, and it should be deleted. Logical model is a new concept that just surfaced due to recent discussion on model transfer/delivery. Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model. In our understanding, “model” refers to a physical model in 3GPP discussion thus far. For example, the example terminologies below clearly describe the model based on the understanding that it’s a physical model. AI/ML Model; A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.. AI/ML model training; A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. AI/ML model Inference; A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. For the progress of discussion, we could first agree to the first paragraph and 4 bullets prior to discussing the two remaining paragraphs. |
| Nokia/NSB | We think that the logical model and functionality talk about the same thing. We are open to discussing something like below, A functionality can be related to the notion of a logical model. - Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model. In earlier RAN1 agreements, “model” referred to mainly as a “physical model” - Logical AI/ML model is a high-level view of the use/task of one or more physical models and identified to be related to a functionality (e.g., via an ID or functionality ID). - Note: A logical model may be related by one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single ID. - Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format). [Mod] I believe the last Note is helpful. Otherwise, some readers may think that a model is either physical or logical. Though it’s true that a logical model may be the same as some notion of functionality (Case 3 in the Issue 6-7 discussion), maybe it’s better not to tie this proposal with functionality which is not clearly defined yet. |  |
| Apple | Agree on the 1st part. 2nd part seems redundant. On 3rd part, similar as proposal 6-7a, we do not see how do we define global unique ID for applicable scenarios/configuration/site. We think those are part of functionality based LCM, particularly site info which is based on UE location. And UE location is privacy related information and should not be disclosed to NW without user consent. Pairing ID is one example of model ID for two sided model. Other examples include dataset ID used for training type 3 of two sided model, encoder ID, decoder ID etc. Suggest to modify as: A logical model identified by a model ID may refer to different units for different cases. More precisely, a model ID may correspond to - For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable scenarios, configurations, sites. - For two-sided models in Collaboration Level y: A pairing ID model ID indicating compatibility between UE-side and NW-side models - For Collaboration Level z: A physical model that is transferred from NW to UE. [Mod] Changed the descriptions in the new proposal |  |
| AT&T | Support the proposal. However, it will be easier to converge if the parts of the proposal are discussed separately in particular, regarding model ID and logical model. We understand that the physical model is associated with a global model ID. However, for collaboration level z after the model is identified (and transmitted), we may use a shorter logical model ID (that can be a part of global model ID) for other LCM procedures. [Mod] Clarified that a model ID may be local or global |  |
| Futurewei | We are not sure about the “different units” as it has not been carefully discussed. We can agree with the proposal with the following text removed. A logical model identified by a model ID may refer to different units for different cases. More precisely, a model ID may correspond to - For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable scenarios, configurations, sites. - For two-sided models in Collaboration Level y: A pairing ID indicating compatibility between UE-side and NW-side models For Collaboration Level z: A physical model that is transferred from NW to UE. [Mod] Changed the descriptions. They are provided as examples for better understanding. |  |
| InterDigital | We agree with first 4 bullets. The last paragraph needs further discussion. |  |
| Spreadtrum | We are fine. |  |
| Intel | Support, the only modification that we propose is pairing ID model ID |  |

###### [FL2] Proposal 6-8b:

Extend the notion of a model into a logical model.

- Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model. Physical model can be identified and used in signaling.

- Logical AI/ML model is a model that is identified (e.g., via a local of global model ID) and used in signaling.

- Note: A logical model may be implemented by related to one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single model ID.

- Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format).

Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model.

- In particular, model-level-LCM is based on logical models, and UE may have physical model operations transparently to the NW.

- FFS: NW's awareness of physical model operations, such as for monitoring, test, certification, and interruption during physical model switching

A logical model identified by a model ID may correspond to,. More precisely, a model ID may correspond to For for example,

- For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable conditions scenarios, configurations, sites. A group of physical models that is identified as one model ID for LCM purposes.

- For two-sided models in Collaboration Level y: An ID indicating compatibility between UE-side and NW-side models

- For Collaboration Level z: A physical model that is transferred from NW to UE.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-9: Model description information

In the proposal 5-14 of the last meeting, it was highlighted that some model description information or meta information may be provided to a Model ID. Model description information or meta information is the supplemental information being provided about a model during model identification process to figure out applicability of the model. This may include things like intended sub-use-case and applicable Features.

###### [FL1] Proposal 6-9a:

Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point:

- Applicable AI/ML-enabled Feature(s)

- Applicable conditions of the model

  - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID

- Information on nominal model input

  - Note: Physical model input is UE implementation and does not need standardization.

- Information on nominal model output

  - Note: Physical model output is UE implementation and does not need standardization.

- Assistance information for inference

- Expected model performance for monitoring

Notes: Other aspects may be added as discussion progresses

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic, DCM |  |
| Company | Comments |  |
| vivo | We prefer the following update to make it general enough. Further clarification on expected model performance for monitoring is needed. Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point: - Applicable AI/ML-enabled Feature(s) - Applicable conditions of the model - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID, scenarios/site information, training data information - Information on nominal model input - Note: Physical model input is UE implementation and does not need standardization. - Information on nominal model output - Note: Physical model output is UE implementation and does not need standardization. - Assistance information for inference - [Expected model performance for monitoring] Notes: Other aspects may be added as discussion progresses |  |
| Google | We think the last sub-bullet on model performance monitoring should be removed. |  |
| CMCC | The definition of functionality is still under discussion. So we prefer the following update: - Applicable conditions of the model - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID |  |
| ETRI | Each step of the function/model identification process should be defined first. Then, the above topics can be discussed, referring to the steps of conveying model description information. Without information about the before and after stages, it is impossible to predict what model description information will be needed. For example, if the model description includes the applicable conditions, it can imply that the UE reports to the NW all applicable conditions for the model. However, such information may not be necessary if the NW configures applicable conditions before the reporting of model description information. |  |
| CATT | Generally OK. We also think the ‘Expected model performance for monitoring’ is needed and prefer to keep it. In addition, if the UE cannot always guarantee the concurrent use with any other non-AI/ML features or other AI/ML models, they should also provide related information. |  |
| Samsung | Is this for Case 3 and Case 4 of LCM cases? Or does it apply for Case 1 and Case 2? [Mod] As this is regarding model identification, it’s applicable only for model-ID-based LCM. |  |
| Xiaomi | - In our view, we should clarify the relationship between model identification and functionality identification first. Currently, it seems that there are two kinds of understanding - Type 1 understanding: model identification and functionality identification are decoupled. - Type 2 understanding: model identification is dependent on the functionality identification For type 1 understanding, this proposal may be good for start the discussion. But for type 2 understanding, it seems some information is already revealed during the functionality identification procedure. - From Xiaomi’s view, we support type 1 understanding, and based on this assumption, We support vivo’s revision. In addition, we suggest the following revision on the main part since the some of the listed information is mandatory not supplemental. Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point: [Mod] “Supplemental” was meant to signify that the information is NOT about the model itself such as the model structure and parameters. |  |
| NEC | Like Google and Vivo, we also think that last subbullet “Expected model performance for monitoring” should be removed or kept FFS. It is not clear whether this information should be part of model description or should be part of signaling for model monitoring. It would be preferable if we can discuss this aspect together with model monitoring procedure. |  |
| ZTE | We’re fine with the first two bullets. - For model input and output, it’s not clear whether we need to define actual model input/output when model transfer happens. - For assistance information, it’s not clear whether the assistance information is used for model input or simply RRC configurations in second bullet. - Our understanding is that the last bullet should be discussed by RAN4. What RAN1 can discuss is that the conditions or configurations required for model performance monitoring. |  |
| OPPO | The last two bullets are not clear now, which can be left for FFS. And we do not understand how “model pairing ID” can be a part of the information. If the information is used for model identification, the ID has not exist when the information is reported. Between the two names, we prefer “model description information”. |  |
| Lenovo | Generally OK. |  |
| CAICT | Fine with the proposal. |  |
| Huawei, HiSilicon | 1) Pairing ID belongs to model ID but not meta information. If we want to describe the applicable scenario, we may use dataset ID/scenario ID. 2) As we are talking about model identification, “Applicable functionality/functionalities” is removed to avoid the tangling between the two. 3) The last two bullets are not clear, so move to FFS Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point: - Applicable AI/ML-enabled Feature(s) - Applicable conditions of the model - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID, applicable dataset ID, scenario ID, etc. - Information on nominal model input - Note: Physical model input is UE implementation and does not need standardization. - Information on nominal model output - Note: Physical model output is UE implementation and does not need standardization. - FFS Assistance information for inference, if applicable - FFS Expected model performance for monitoring Notes: Other aspects may be added as discussion progresses |  |
| Fujitsu | We also think “model paring ID” need to be got agreement/common understanding first. Besides, “Expected model performance for monitoring” seems unclear and may put it as FFS. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | The “assistance information for inference” could be removed. This information should anyway be part of the nominal input to the model. We prefer to keep a bullet on the expected model performance. Accuracy/Uncertainty is a key notion in AI/ML and accuracy/uncertainty quantification is a key element for trustworthy and explainable AI/ML. It is important that the NW can retrieve information on how accurate/uncertain a prediction from a UE-side model is. Consider the following rewording: - Assistance information for inference - Expected model performance for monitoring - Information on (expected) model performance - e.g., based on model training |  |
| Nokia/NSB | Not clear why to change the direction we had in RAN1 #112 of applicable conditions for functionalities to models now. We shall first discuss functionality and its relation of that to the logical model to make the separation or forget about discussion of logical model (and model IDs) to replace earlier agreements with new terminology. Also, please refer to Nokia contribution on directions that RAN1 shall identify as applicable conditions. |  |
| Apple | We agree with Xiaomi’s comment, also support type 1 undertanding. In addition, pairing ID is one form of model ID. Suggest to remove. - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID |  |
| Sony | We are fine with the proposal. |  |
| AT&T | We support the proposal. We prefer to keep the last bullet. |  |
| Futurewei | Agree in principle, with the comments similar to HW. - The Model’s function has been indicated by the “Applicable AI/ML-enabled Feature(s)” so the “Applicable functionality/functionalities” can be removed from the examples of the second bullet. - “Assistance information for inference” and “Expected model performance for monitoring” can be FFS; these have not been carefully discussed and we are not sure whether they are needed. |  |
| InterDigital | We think agree with other companies that Assistance information for inference is not clear and should be FFS. |  |
| Spreadtrum | We have one clarification on the last bullet ‘Expected model performance for monitoring’. It is not clear about model performance, for example, what the mode performance is. |  |
| Intel | Support, we suggest to remove pairing ID. Also, it may be worthwhile to mention that a model identification process is 1-1 associated with a model-ID |  |

###### [FL2] Proposal 6-9b:

FL note: RAN2 has an agreement.

R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point:

Model description information or meta information is the supplemental information being provided about a model during model identification process. For the model description information, consider the following aspects as the starting point:

- Applicable AI/ML-enabled Feature(s)

- Applicable conditions of the model

  - E.g., Applicable functionality/functionalities, applicable RRC configurations, model pairing ID, scenarios, sites, and datasets

- Information on model input, if applicable

  - Note: Input may be nominal Physical model input is UE implementation and does not need standardization.

- Information on nominal model output, if applicable

  - Note: Output may be nominal Physical model output is UE implementation and does not need standardization.

- Assistance information for inference, if applicable

- FFS: Expected model performance for monitoring

Notes: Other aspects may be added as discussion progresses

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-10: Local Model index

According to the RAN2#121 agreement, “RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified”.  This notion of global Model ID can be utilized in model transfer and/or test certifications. On the other hand, it is questionable whether a global Model ID is needed for Model inference in model activation/deactivation/selection/switching since for these purposes it may not be efficient to use global model ID. This brings the question if there can be assigned local (or temporary). Model index for model management after identification/registration.  Note that the concept is similar to many legacy operations where an index used in DCI during an RRC connection may be configured by higher-layer signaling.

###### [FL2] Proposal 6-10a:

Study the necessity, use case, and specification impact of a local model index.

A local model index refers to a temporary model index for model control after Model identification.

- Local Model index may be allocated between NW and UE and utilized for various LCM signaling purposes such as activation/deactivation/selection/switching.

- Local Model index should ambiguously identify an AI/ML model between NW and UE.

- FFS: Associated mechanism and signaling

- FFS: For which LCM procedures a model ID or a local model index may be used

|  | Yes | No |
| --- | --- | --- |
| Agree? | DCM |  |
| Company | Comments |  |
| LG | General direction is ok but this discussion would be fit to WI phase. |  |
| NEC | We are going into the details which should be discussed in RAN2. From RAN1 perspective we only need to determine whether LCM procedures are performed using model id. The remaining discussion on how to compress the signaling during actual LCM procedures falls into RAN2 domain. Any discussion on this should have proper justification on how it impacts RAN1 procedures. |  |
| ZTE | Fine to clarify the understanding. Maybe we don’t need FFS for this proposal as it can be further discussed in WI phase. |  |
| Ericsson | This does not seem like a RAN1 issue, topic should be handled in other working groups. |  |
| Nokia/NSB | Need further discussion on what is refer by model-ID LCM here with local ID compared to the functionality LCM. |  |
| Intel | This seems like a type of logical Model and its corresponding Model-ID ? |  |

##### Issue 6-11: Model identification types

Clarification on “model identification”: Model identification is a “naming” process (i.e., process of assigning a model ID). Model identification itself doesn’t imply any model transfer/delivery nor UE capability, although it’s possible that 3gpp define a combined signaling procedure if necessary.

###### [FL1] Proposal 6-11a:

Categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact.

- Type A1: Model is identified offline to NW (if applicable) and UE (if applicable)

  - The model is assigned with a model ID. Then, the model ID is accessible to both NW side and UE side. The model description information about UE-side/part model being identified is provided during the model identification.

  - The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is transparent to RAN1 specification. The model identification type A1 may have RAN2/SA2 impact.

  - Note: An example of the model identification Type A1 is the pairing ID for the two-sided model training.

  - Note: In Type A1, model is identified to NW and UE separately (say from vendor to NW, and via vendor’s firmware update at UE), rather than directly between NW and UE. As a result, NW is not aware of the supported model IDs at an individual UE, until the UE reports the supported model IDs via UE capability.

- Type A2: Model is identified via signaling from UE to NW

  - Type A2 is similar to Type A1, except that the model identification is performed between NW and a designated UE via over-the-air signaling.

- Type B: Model is identified via signaling from NW to UE

  - Type B is used in conjunction with model transfer from NW to UE to identify a new model of the previously identified structure with new parameters.

  - Note: A new model of the previously identified structure with new parameters may also be identified offline via Type A1.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | Panasonic |
| Company | Comments |  |
| vivo | We prefer the following update. The second one is in fact more like the third one rather than the first one. Categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact. - Type A1: Model is identified offline to NW (if applicable) and UE (if applicable) - The model is assigned with a model ID. Then, the model ID is accessible to both NW side and UE side. The model description information about UE-side/part model being identified is provided during the model identification. - The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is transparent to RAN1 specification. The model identification type A1 may have RAN2/SA2 impact. - Note: An example of the model identification Type A1 is the pairing ID for the two-sided model training. - Note: In Type A1, model is identified to NW and UE separately (say from vendor to NW, and via vendor’s firmware update at UE), rather than directly between NW and UE. As a result, NW is not aware of the supported model IDs at an individual UE, until the UE reports the supported model IDs via UE capability. - Type B1A2: Model is identified via signaling from UE to NW - Type A2 is similar to Type A1, except that the model identification is performed between NW and a designated UE via over-the-air signaling. - Type B2: Model is identified via signaling from NW to UE - Type B is used in conjunction with model transfer from NW to UE to identify a new model of the previously identified structure with new parameters. - Note: A new model of the previously identified structure with new parameters may also be identified offline via Type A1. |  |
| Google | Support in principle |  |
| Panasonic | At this moment, it is not yet so clear whether A1 is transparent to RAN1 specification for example of two-sided models in Collaboration Level y. Therefore, we propose to modify as following. - The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is may be transparent to RAN1 specification. The model identification type A1 may have RAN2/SA2 impact. On the type A2, whether the whole signaling procedure is initialized from UE to NW or some of the signaling content is suggested from UE are unclear at this moment. To say "from UE to NW" is ambiguous. Alignment with the subbullet would be better like following. - Type A2: Model is identified via over-the-air signaling signaling from UE to NW On the type B, "a new model of the previously identified structure with new parameters" or not can be clarified by the following z4/z5 boundary. Therefore, it is not required to clarify it here. - Type B is used in conjunction with model transfer from NW to UE to identify a new model of the previously identified structure with new parameters. - Note: A new model with model transfer of the previously identified structure with new parameters may also be identified offline via Type A1. |  |
| LG | Type A1 and A2 are for NW to identify UE-side/part model but Type B is for UE to identify UE-side/part model. Not sure whether it is beneficial to categorize these different things in the same level, but if so, this difference needs to be clearly described. |  |
| ETRI | We don't think the discussion is enough to define the model identification type. The model identification may not be unidirectional. For example, a UE can report model information to the NW along with a global model ID, and the NW can assign a local model ID to the reported model. This process includes handshake between UE and NW. This topic can be discussed after the model identification process has been sufficiently refined. |  |
| CATT | The original Type A2 and Type B are more close to each other, especially from view of potential RAN specification impact. Thus we prefer vivo’s naming. Or simply use Type A, Type B and Type C, which do not hint anything. No strong view on removing the notes. |  |
| Samsung | Does this apply for Case 3 and Case 4 or Case 2 and Case 1 too? [Mod] As this is regarding model identification, it’s applicable only for model-ID-based LCM. |  |
| ZTE | - The second note of Type A1 is not clear to us. We may don’t need to mention the first sentence as it somehow refers to model transfer/delivery. In addition, when the model ID report belongs to UE capability report can be further discussed. - Note: In Type A1, model is identified to NW and UE separately (say from vendor to NW, and via vendor’s firmware update at UE), rather than directly between NW and UE. As a result, NW is not aware of the supported model IDs at an individual UE, until the UE reports the supported model IDs via signaling from UE to NW via UE capability. - Type B seems to preclude the case z4(model transfer in open format of an unknown model structure at UE) we agreed in last meeting. It’s better to conclude the feasibility of z5 first. - We have similar comment as Samsung that whether Case 3 should include model identification. |  |
| Rakuten Mobile | We agree in principle, but regarding detail described, companies seems to have different understanding. |  |
| Lenovo | We think Type A2 and Type B can be merged in an ‘online’ way (compared with the ‘offline’ used in Type A1). Thus, they can be categorized to be two types as - Type A1: Model is identified offline to NW (if applicable) and UE (if applicable) - (skip) - Type B: Model is identified via signaling from between NW to and UE - The model is assigned with a model ID via model identification performed between NW and UE with over-the-air signaling. - The model transferred or updated from NW to UE can be identified as a new model in conjunction with model identification. - Note: A new model of the previously identified structure with new parameters may also be identified offline via Type A1. |  |
| CAICT | We also prefer to renaming Type A1/A2/B to Type A/B/C. |  |
| Huawei, HiSilicon | Fine in general. Some comments: 1) For Type A1, SA3/SA5 impact may also be needed. 2) For Type A2, as the registration is via air-interface signaling, NW should be gNB. 3) For model transfer of Type B, whether Case z4 is the only feasible option may be too early to decide. - Type A1: Model is identified offline to NW (if applicable) and UE (if applicable) - The model is assigned with a model ID. Then, the model ID is accessible to both NW side and UE side. The model description information about UE-side/part model being identified is provided during the model identification. - The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is transparent to RAN1 specification. The model identification type A1 may have RAN2/SA2/SA3/SA5 impact. - Note: An example of the model identification Type A1 is the pairing ID for the two-sided model training. - Note: In Type A1, model is identified to NW and UE separately (say from vendor to NW, and via vendor’s firmware update at UE), rather than directly between NW and UE. As a result, NW is not aware of the supported model IDs at an individual UE, until the UE reports the supported model IDs via UE capability. - Type A2: Model is identified via signaling from UE to NW - Type A2 is similar to Type A1, except that the model identification is performed between gNB NW and a designated UE via over-the-air signaling. - Type B: Model is identified via signaling from NW to UE - Type B is used in conjunction with model transfer from NW to UE to identify a new model of the previously identified structure with new parameters. - Note: A new model of the previously identified structure with new parameters may also be identified offline via Type A1. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | It’s not clear why Type B is only used for “previously identified structure with new parameters”. It should be more generic, similar to Type A. Consider clarifying that Type B is only applicable for model transfer. For Type A2, how is the model ID assignment done? How is A2 different from A1? “UE reports the supported model IDs via UE capability” of A1 can be understood as A2 “Model is identified via signaling from UE to NW”. |  |
| Nokia/NSB | Not Ok. RAN1 first need to clarify the difference of model terminologies used in different proposals. |  |
| Sony | We think the vivo’s update is clearer. |  |
| AT&T | Support in principle. |  |
| Futurewei | We understand the purpose of this categorization is to faciliate discussion. For this purpose, suggest we call A1 “offline identification”, and A2/B “online identification”. |  |
| Intel | We support Type A1, A2 and the associated text but not sure about Type B. We support to describe a process to update parameters of an identified model structure by a NW to a UE but not sure it should be called model identification. This is confusing – its cleaner to call Type B as a model update process or something different from model identification. |  |

###### [FL2] Proposal 6-11b:

Categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact.

- Type A: Model is identified offline to NW (if applicable) and UE (if applicable)

  - The model is assigned with a model ID. Then, the model ID is accessible to both NW side and UE side. The model description information about UE-side/part model being identified is provided during the model identification.

  - The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is may be transparent to RAN1 specification. The model identification type A may have RAN2/SA2/SA3/SA5 impact.

  - Note: An example of the model identification Type A is models identified via offline the pairing ID for the two-sided model training.

  - Note: In Type A, model is identified to NW and UE separately (say from vendor to NW, and via vendor’s firmware update at UE), rather than directly between NW and UE. As a result, NW is not aware of the supported model IDs at an individual UE, until the UE reports the supported model IDs via over-the-air signaling (e.g., UE capability report) from UE to NW UE capability.

- Type B: Model is identified via over-the-air signaling from UE to NW

  - Type B is similar to Type A, except that the model identification is performed between gNB NW and a designated UE via over-the-air signaling.

- Type C: Model is identified via over-the-air signaling from NW to UE

  - Type C is used in conjunction with model transfer from NW to UE to identify a new model of the previously identified structure with new parameters.

  - Note: A new model in model transfer of the previously identified structure with new parameters may also be identified offline via Type A.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-12: Dynamic UE capability reporting

Legacy UE capability reporting is used only for reporting static applicable conditions/RRC configurations. Therefore, if a legacy-like UE capability reporting is used for functionality-based LCM (as defined in Issue 6-7), the functionality-based LCM cannot handle dynamic scenarios/sites. For example, in functionality-based LCM, if UE supports AI/ML models for the functionality in some sites but not in other sites, we may need a mechanism that enables UE to dynamically update its capability for supporting the functionality.

Unlike the functionality-based LCM, the model-ID-based LCM provides mechanism for handling dynamic scenarios/sites. However, various implementation restrictions at UE (such as model availability, model concurrency issue, etc.) may render UE to be unable to support the given AI/ML-enable Feature temporarily or for an extended duration.

###### [FL2] Proposal 6-12a:

Study the mechanism to allow UE to timely report the list of currently supported UE part/UE-side functionalities/models, where the supported functionalities/models may be a subset of all identified functionalities/models.

|  | Yes | No |
| --- | --- | --- |
| Agree? | LG, ZTE |  |
| Company | Comments |  |
| LG | In our view, the key potential enhancement for functionality-based LCM is to support more dynamic update of UE capability. In this aspect, it needs to be clarified how to handle varying performance of a same feature/FG (e.g. separate them as different functionalities or handle them within a same functionality). For example, if UE supported up to X1 ms TD prediction but now its capability is reduced up to X2 ms (X2<X1) TD prediction. In conventional UE capability report, this is one feature/FG so the UE may report NW that the UE can support up to X2 ms prediction for the feature in conservative way (i.e. based on the worst case scenario assumption). For performance improvement via AI/ML, whether/how to handle this aspect needs to be studied. We suggest the following update: Study the mechanism to allow UE to timely report the list of currently supported UE part/UE-side functionality(es)/model(s), where the supported functionalities/models may be a subset of all identified functionalities/models. FFS: how to handle varying performance of a same feature/FG (e.g. separate them as different functionalities or handle them within a same functionality) |  |
| Samsung | It is not clear why such dynamic indication is needed. What is the scale of site? Is it cell? |  |
| NEC | Support in principle |  |
| Lenovo | Generally OK. |  |
| Nokia/NSB | Disagree with the FL comment above “Unlike the functionality-based LCM, the model-ID-based LCM provides a mechanism for handling dynamic scenarios/sites.”. Ran1 agreed functionality LCM is based on 3GPP signalling and its functionality selection can also be dynamic. In summary, what matters now is to identify applicable conditions. If there are dynamic applicable conditions, of course those do not fit the UE capability signalling, but functionality selection/activation does not always have to be depended on only on the capability signalling. |  |
| Apple | Clarification question: Type A1 and A2 are two step approach for model ID identification? In type A1, last sentence, “until the UE reports the supported model IDs via UE capability.” Is this UE capability reporting type A2 identification? In other word, type A1 is first step which is offline, and type A2 is 2nd step which is over the air interface for model identification? |  |
| InterDigital | OK to study |  |

##### Issue 6-13: Pairing ID

###### [FL1] Proposal 6-13a:

A pairing ID is a logical ID that indicates compatibility between the UE-side and NW-side model of a two-sided model.

- For example, all encoders developed resulting from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training, UE-side encoders trained based on the same NW-side model may be associated with a single pairing ID.

- The pairing ID may also serve as a model ID. That is, all the encoders associated with a pairing ID may be identified as a single model ID.

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic | Vivo |
| Company | Comments |  |
| vivo | This ID seem to serve the same purpose as model ID. We may not need another ID. |  |
| Google | It seems model ID should be sufficient. |  |
| LG | Same view as vivo |  |
| NTT DOCOMO | We are fine with introducing the paring ID that indicates the compatibility between the UE-side and NW-side model. However, a paring ID should not be equal to logical model ID. There is a situation where multiple logical models are paired to the same physical NW side models. In that case, a paring ID cannot differentiate two logical models. |  |
| CMCC | It seems we introduce another ID in addition to model ID, we are not sure it is needed or not. Besides, for the first sub-bullet, regarding the second example of multi vendors training case, we think multiple encoders based one same NW-side model can still be assigned with multiple paring IDs. |  |
| ETRI | We share similar view with vivo |  |
| CATT | Support in intention. If the group wants to support two-side model paring in functionality-based LCM, a pairing ID is needed (since no model ID in this case). |  |
| Samsung | Ok to study but better if this wait until some clarity on Model LCM is achieved. |  |
| Xiaomi | Whether the terminology of pairing ID is needed depends on how to interpret currently defined model ID. If the model ID refers both the encoder and decoder, then we think there is no need for the pairing ID. |  |
| NEC | In our understanding, to support two-sided model, the model ID should be clear enough for example by including NW part ID and UE part ID, and thus pairing ID is not needed. |  |
| ZTE | Our understanding is that, for training collaboration Type 3 of CSI compression, UE and NW only need to align the dataset rather than a model ID(which may refer to a specific model). Therefore, we propose the following revisions: A pairing ID is a logical ID that indicates compatibility between the UE-side and NW-side model of a two-sided model. - For example, all encoders developed resulting from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training on training collaboration Type 3 of CSI compression sub use case, UE-side encoders trained based on the same dataset NW-side model may be associated with a single pairing ID. - The pairing ID may also serve as a model ID/dataset ID. That is, all the encoders associated with a pairing ID may be identified as a single model ID/dataset ID. |  |
| OPPO | We do not think the new concept is needed. If the multiple encoders are invisible to NW, they can share the same logical model ID. In which case, not all the encoders associated with a pairing ID can be identified as a single model ID? |  |
| Lenovo | As we comment above, we don’t understand the motivation to introduce ‘pairing ID’ before clearly defining and designing the ‘model ID’. |  |
| CAICT | We could have some consensus on Model ID first and then decide whether pairing ID is required or not. |  |
| Huawei, HiSilicon | We agree that the pairing ID means the logical model ID, which may correspond to one or more physical models at UE side. But the examples are problematic: how if UE trains with totally different model backbones which lead to different performance when doing the pairing? Are they still subject to the same model ID? “ For example, all encoders developed resulting from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training, UE-side encoders trained based on the same NW-side model may be associated with a single pairing ID. ” |  |
| Fujitsu | Same view as vivo. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | We prefer pairing ID over model ID. This would be more general than a model ID, and would be the preferred solution for two-sided models. Otherwise, the number of possible model IDs needed is expected to be large. The pairing ID indicates a reference to a prior training session (e.g. using an API) between NW and UE, associated with a specific dataset. Hence the ID doesn’t refer to a model but instead a process used to obtain the models at each side. |  |
| Nokia/NSB | OK in general. However, more discussion is needed to align among companies. |  |
| Apple | Same view as vivo. |  |
| AT&T | It will be beneficial to first agree on model ID before this discussion as pairing ID can be just used as model ID. |  |
| Futurewei | Agree with many companies that if pairing ID serves the same purpose as model ID, then we don’t need it. |  |
| InterDigital | We think the pairing ID may be OK for synchronizing with the right decoder model at the NW. But it is not clear how two different UE encoders (with potentially different performance) resulting from same multi-vendor training session should be identified for model transfer/update/switching/performance monitoring etc. |  |
| Spreadtrum | Understand the intention of the proposal. But we also think the pairing ID may be not needed for model ID can be served as the same purpose. |  |
| Intel | Its premature in our view – pairing ID does nothing that a model-ID cannot do as per this proposal. |  |

###### [FL2] Proposal 6-13b:

Consider the logical ID that indicates compatibility between the UE-side and NW-side model of a two-sided model. For example, all encoders developed resulting from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training, UE-side encoders trained based on the same NW-side model may be associated with a single pairing ID.

- The pairing ID may also serve as a model ID. The ID may be used as a model ID. That is, all the encoders compatible with a decoder associated with a pairing ID may be identified as a single model ID.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-14: Applicability of LCM methods

###### [FL2] Proposal 6-14a:

For UE-side models in Collaboration Level y

- Further study whether to adopt functionality-based LCM, model-ID-based LCM, or both.

For two-sided models in Collaboration Level y

- Study Model-ID-based LCM. Functionality-based LCM is NOT applicable.

  - Note: A model ID may be a pairing ID.

For Collaboration Level z

- Study Model-ID-based LCM. Functionality-based LCM is NOT applicable.

  - Note: A model indicated by a model ID is both logical and physical.

For Network-side model and Network-part of two-sided model, functionality and model activation/deactivation/selection/switching/fallback, if applicable, is up to Network implementation.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | LG, AT&T |
| Company | Comments |  |
| LG | Functionality-based LCM is based on what we have in 3GPP (e.g. UE feature, RRC/MAC-CE/DCI), so it is something related to ‘enhancement’ but model-based LCM is something ‘new’. In this regard, we should firstly conclude that UE-side models in Collaboration Level y, no model-ID based LCM is necessary, i.e. functionality-based LCM is enough. After that, we can further study whether functionality-based LCM can still be applied for two-sided model/level z or not. If not, we can study by focusing on what additional aspect needs to be supported for these cases, which will make our discussion focused/easier. |  |
| NTT DOCOMO | It should be discussed after proposal 6-7. |  |
| NEC | Support in principle |  |
| Nokia/NSB | Functionality-LCM is agreed in RAN1 #112 and there is no limitation of that to any collaboration level or one/two sided cases. It shall be base of 3GPP signalling design. As model terminology is not clarified yet, it is not suitable to mixing thing yet on that. |  |
| AT&T | Our understanding is that functionality based LCM is based on the 3GPP signaling while model ID based LCM is necessary to perform model level control for instance identification/transfer/switching/update etc. However, after both sides have common understanding regarding model and its supported functionalities we can use functionality based LCM to change configurations. For instance, if NW know UE is running model 1(which support configuration A and B) with configuration A we can use functionality based LCM to switch to configuration B. |  |

##### Issue 6-15: (placeholder)

##### Issue 6-16: (placeholder)

##### Issue 6-17: (placeholder)

#### Model configuration

Huawei / HiSilicon

Proposal 21: Whether to consider model configuration as an individual procedure in LCM can be postponed until its definition is clear.

#### Model deployment

Huawei / HiSilicon

(From Proposal 1):  Define the following terminologies if needed:

- Model deployment: Process of converting an AI/ML model into an executable form and deploy it to a target device where inference is to be performed. Note: The conversion may happen before or after delivery.

#### Model delivery and transfer

Working Assumption from RAN1 #111 Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared RAN2 #120 agreement For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions.

##### FL recommendation 3-53d: FL recommendation: Consider the following aspects for discussion of model delivery format options and their feasibility, pros, and cons. The list is provided as a guideline for companies to bring discussion in the next meeting. - Interoperability: does a model transferred from NW/UE side to UE/NW side work in a plug-and-pay manner? - Device capability for compiling and running the model - Hardware efficiency (device-specific optimization) - Proprietary information disclosure across vendors - Testability aspects: including how to involve testing equipment - Offline co-engineering efforts - Feasibility for deployment involving multiple vendors - Flexibility for model update - Model performance - If AI/ML model is transferred from one node to other, which entity guarantees performance, e.g., inference latency? - Impact on other common KPIs (e.g., over-the-air overhead, inference complexity, training complexity, LCM related complexity and storage overhead, [latency]) - Specification effort Model delivery format options under consideration include, at least - Vendor-specific format (e.g., device-specific run-time binary image) - 3GPP-standardized/adopted model representation format (MRF) (e.g.,.h5, .pt, .mlmodel, ONNX, or custom 3GPP-developed MRF) - Any other aspects Note: The discussion is about model delivery format. Note: For the 3GPP-standardized/adopted MRF, if adopted, the choice of format is FFS. Note: Some aspects may belong to RAN2 discussion, in which case RAN1 can give appropriate guidance based on RAN1 understanding. FFS: Applicability to collaboration level y and z

##### Proposed conclusions 4-19b from RAN1 #111 (not agreed): Conclusion on device-specific vs. agnostic model: Concerns with a model developed without device specific considerations: - Suboptimal performance due to unoptimized model design for the device (e.g., device specific input, device specific pre-processing, device specific power/complexity/performance trade-off considerations) - Hardware efficiency (lack of device-specific optimization for hardware implementation) - Interoperability issue: Models may not run in a plug-and-play manner at the device. - Model performance (e.g., inference latency) at the target device may not be guaranteed. - Lack of vendor differentiation, potentially hindering innovation Benefits of a vendor-/device-agnostic model: - Less offline engineering effort across vendors for two-sided model training Conclusion on model delivery using an open-format model compared to proprietary-format models Concerns with model delivery in an open format: - It may require device capability for compiling and running the model. - FFS: device capability for parameter-only update - Proprietary model information is disclosed across vendors. - Specification impact Benefits of model delivery in an open format: - Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing Conclusion on model parameter update after deployment Concerns - Unlike in offline training where the trained model can go through extensive functionality and performance testing, model parameter update after deployment may lead to unoptimized and/or unexpected device behavior/performance.

FL comment from RAN1 #111 There are varying opinions on the need of model delivery/transfer. FL encourages - Proponents to bring discussions on why model delivery/transfer may be useful and their use cases - Opponents to bring discussions on why model delivery/transfer is not needed - In which scenarios model delivery/transfer may or may not be needed

Agreement from RAN1 #112 To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary RAN2 #121 agreement Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. Solution 4: Server (e.g., OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g., transparent to 3GPP)

Huawei, HiSilicon

(From Proposal 1):  Define the following terminologies if needed:

- AI/ML model transfer: Delivery of an AI/ML model over the air-interface signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

Proposal 3: Deprioritize the model transfer/delivery from UE to Network

Proposal 4: For the study of model transfer/delivery from Network to UE, small model size (e.g., to ensure no strong impact to legacy RRC signaling) should be assumed as a starting point for the potential spec impact analysis.

ZTE

Proposal 15: RAN1 concludes the pros and cons of supporting “proprietary model” and “open-format model”. Then, RAN1 should send LS to other working groups (e.g., RAN2 and SA2) to check the feasibility of supporting open-format models.

Proposal 16: Leave the discussion on model storage and delivery options to RAN2. RAN1 can proceed current work assuming model delivery may be from a 3GPP entity (e.g., gNB, LMF and CN) or from a non-3GPP entity (e.g. OAM, OTT).

Vivo

Proposal 11:	 Model delivery/transfer is necessary due to flexibility for model update and per cell or area optimization.

Proposal 12:	 Study open format with widely-used model structures and parameter-only updating, which can obtain the most gain of open format and proprietary format.

- Using widely-used model structures to avoid the concern of disclosing model structure.

- Using parameter-only updating to avoid additional compiling at UE side.

Proposal 13: Use pros and cons in Table 5.3-1 for the model transfer/delivery discussion.

|  | Flexibility for model update and per cell or area optimization | Offline co-engineering efforts and feasibility for deployment involving multiple vendors | Device capability and delay for compiling, updating and running the model | Participation in model optimization |
| --- | --- | --- | --- | --- |
| Case y | Low | Large offline co-engineering efforts | Low device capability, small delay | Only UE side |
| Case z1 | High | Large offline co-engineering efforts | Low device capability, small delay | Only UE side |
| Case z2 | High | Large offline co-engineering efforts | Low device capability, small delay | Both UE side and network side |
| Case z3 | High | Small offline co-engineering efforts | Dependent on whether the model structure is known at UE | Both UE side and network side |
| Case z4 | High | Small offline co-engineering efforts | Low device capability, small delay | Both UE side and network side |
| Case z5 | High | Small offline co-engineering efforts | High device capability, large delay | Both UE side and network side |

Proposal 14: Support to reuse the mechanism defined in SA2 (interoperability token) for aligning model description format for model transfer.

Proposal 15:	RAN1 concludes typical model size, frequency of model transfer/update and latency requirement and send LS to RAN2 to facilitate the discussion of solutions for the model transfer.

Proposal 16:	Model transfer capability may consider the alignment between UE and network on supported structures, quantization and processing.

Spreadtrum

Proposal 2: Suggest to consider the following terminology definition:

- AI/ML model transfer: Delivery of an AI/ML model over the air interface with non-transparent 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

CATT

Proposal 14: Leave the discussion of model delivery/transfer to RAN2. RAN1 may provide some input by sending LS if relative agreements are reached.

Proposal 15: If RAN1 sends an LS to RAN2 on the size of UE-side models, capture CATT’s input in Table 1 in this contribution.

Table 2 Sizes of UE-sided models from CATT

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | CSI compression | The model is designed based on transformer model. | 2.51 M | 10 Mbytes |
| CATT | CSI prediction | The model is designed based on Conv-LSTM model. | 41 k | 0.16 Mbytes |
| CATT | BM-Case1 | The model is designed based on ResNet18 model. | 206 k | 0.8 Mbytes |
| CATT | BM-Case2 | The model is designed based on MLP-Mixer model. | 2.2 M | 8.5 Mbytes |
| CATT | Positioning Case 1 (UE-based positioning) | For direct AI/ML positioning, the model is designed based on ResNet18 model. | 11.2 M | 44 Mbytes |
| CATT | Positioning Case 1 (UE-based positioning) | For AI/ML assisted positioning, ToA is estimated. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |
| CATT | Positioning Case 2a (UE-assisted/LMF-based positioning) | For AI/ML assisted positioning, ToA is estimated. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |
| CATT | Positioning Case 2a (UE-assisted/LMF-based positioning) | For AI/ML assisted positioning, LOS/NLOS is identified. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |

Ericsson

Proposal 4: Model transfer is deprioritized for use cases using one-sided model.

Proposal 5: For CSI with two-sided model, alternatives that do not require model transfer are prioritized in this SI.

Proposal 6: For model delivery/transfer, consider only model delivery (case y). Model transfer (case z1-z5) are not supported.

Xiaomi

Proposal 8: At least model delivery/transfer from network to UE should be considered.

Google

Proposal 11: Since AI/ML models are not expected to be specified, the model transfer and update procedure could be deprioritized.

Samsung

Proposal #1: Concerning with the feasibility and practicality of AI/ML model transfer, RAN1 should study aspects such as

- Interoperability: does a model transferred from one node to another node work in a plug-and-pay manner, i.e., without extensive receiving-node-specific optimization, compiling and testing?

- Proprietary issues: If AI/ML models are considered proprietary assets, model transfer in an open format discloses them.

- Model transfer format (MTF): does RAN1 need to adopt a common MTF so that a model exchanged between two nodes from different vendors compiles and runs?

- Performance guarantee: If AI/ML model is transferred from one node to the other, which entity guarantees performance, e.g., inference latency?

Proposal#2: In regards to model delivery/transfer deprioritize cases z1, z2, z3 ,z4, z5 in this study item

- Cases z2, z3, z4 and z5, do not allow the UE to receive and run AIML models in a ‘plug-and-play’ manner, i.e., without additional steps for UE-specific compilation and optimization.

- Specification support for case-z1 is not justified as the same UE vendor would train the model. Hence, proprietary solutions, e.g., case-y, can be used.

CMCC

Proposal 2: Study the following options and potential spec impact of model delivery.

- Opt1. Proprietary-format based model transfer

- Opt2. Open-format based model transfer

- Opt3. OTT/OAM based model delivery

MediaTek

Proposal 22: For model transfer, RAN1 focuses on what kind of information needs to be delivered for model transfer. RAN1 leaves model transfer channel (CP or UP) and model format to RAN2 discussion. RAN1 should provide inputs for the requirements of model transfer/delivery in terms of model size, latency and model update frequency to RAN2 to evaluate the applicability of different model transfer/delivery solutions.

Apple

Proposal 12: For level z, 3GPP consider endorse a few existing AI model formats. 3GPP does not specify its own model format for model delivery.

Lenovo

Proposal 14: Study the requirements with respect to the payload and delay of different model delivery/transfer contents, including model structure, a set of parameters or a whole model in either open or proprietary format.

Proposal 15: The preferred model structure at UE can be reported to NW as the known model structure, which can be realized within UE capability report or model/functionality identification procedure.

Qualcomm

Proposal 16: Define the boundary between Case z4 and z5 as follows.

- z4: The model structure has been previously identified between NW and UE, and UE has indicated in its capability report that the UE supports the model structure and supports parameter updates for the structure.

- z5: The model structure has not been previously identified between NW and UE, or UE has not indicated in its capability report that the UE supports the model structure and supports parameter updates for the structure.

Proposal 19: Deprioritize Case z5.

Proposal 21: Prioritize proprietary format (Cases z1 and z2) over open-format model (Cases z3 and z4) for model transfer.

AT & T

Proposal 4: Consider “proprietary AI/ML model structure” and “open-source AI/ML model structure” as two separate AI/ML model structure categories for RAN1 discussion,

| Proprietary AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and/or input/outputs that are proprietary from 3GPP perspective. |
| --- | --- |
| Open-source AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and input/outputs that are mutually recognizable across vendors and can allow interoperability, from 3GPP perspective. |

NEC

Proposal 14: Study AI/ML model transfer with 3GPP network assistance at least for the case of two-sided AI/ML model or when online training is required for an AI/ML model.

Proposal 15: Study AI/ML model transfer using open AI/ML format. FFS details of open format, support of vendor specific algorithms.

NTT Docomo

Proposal 1: Deprioritize the case z5 model transfer in Rel-18 AI/ML SI discussion.

Proposal 2: Discuss the feasibility of case z4 model transfer before further studying it.

##### Issue 6-18: Model delivery and transfer discussion

(placeholder)

##### Issue 6-19: Model delivery/transfer Case z4/z5 clarification

This is a continuation of Proposal 5-16f of the previous meeting.

###### [FL1] Proposal 6-19a:

In model delivery/transfer Case z4, the “known model structure” means a model structure that has been previously identified and for whose model ID the UE has explicitly indicated support in its capability reporting.

In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not appearing in z4.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  | Panasonic |
| Company | Comments |  |
| vivo | Support. |  |
| Google | Support |  |
| Panasonic | What part of a AI/ML model is called as parameter and what part of the AI/ML mode is structure is not required to be discussed. Important is whether the AI/ML is capable to UE is known to the NW. Even if the parameter only, depending on the value of the parameters, UE may not be capable. In addition, the AI/ML is capable to UE is not always required to be explicitly indicated in its capability reporting. The capability may be limited by the specification. Therefore, we propose to modify as following. In model delivery/transfer Case z4, the “known model structure” means a model NW knows the UE is capable of structure that has been previously identified and for whose model ID the UE has explicitly indicated support in its capability reporting. [Mod] It will be good not to blur the boundary between z4 and z5. Case z4 is intended for parameter update by the NW on the structure that UE declares “I support the structure and parameter updates on the structure.” |  |
| CMCC | Support. |  |
| CATT | OK. But can we clarify this is (not?) intended to design a separate procedure dedicated for model structure identification? [Mod] Case z4 is, as its definition says, mainly intended for model transfer in open format where NW transfers a model with known structure at UE but new parameters. For this to happen, the model structure (along with initial parameters) should have been identified and UE should have reported its capability for it. |  |
| Samsung | Ok. |  |
| Xiaomi | Support |  |
| ZTE | Support |  |
| Lenovo | We are OK to indicate the structure support in its capability reporting, but not together with 'model ID'. So we suggest the following update for z4: In model delivery/transfer Case z4, the “known model structure” means a model structure that has been previously identified and for whose model ID the UE has and explicitly indicated support in its UE capability reporting. |  |
| CAICT | Support |  |
| Huawei, HiSilicon | To what granularity the “known model structure” is reported? Backbone, number of layer/neurons, activation function, etc., or fully known? An FFS is added. In model delivery/transfer Case z4, the “known model structure” means a model structure that has been previously identified and for whose model ID the UE has explicitly indicated support in its capability reporting. In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not appearing in z4. FFS granularity of “known model structure”, e.g., Backbone, number of layer/neurons, activation function, etc., or fully known. [Mod] In Case z4, the model struct means “exactly known model structure”, which will allow UE-side to fully go through compiling, quantization decision, mapping to hardware operations, and full HW/SW testing, so that UE may indicate its capability for it. Any “roughly but not exactly known model structure” belongs to Case z5, as it needs further discussion on feasibility. For the sake of progress, FL suggests such a clear demarcation between z4 and z5. If we include “roughly but not exactly known model structure” into Case z4, then it will be difficult to advance the Case z4 as Case z4 will invite feasibility questions. |  |
| Fujitsu | Support |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | Fine with above clarification. However, we haven’t seen evidence that ‘known model structure’ means plug-and-play. Since the model is trained at the network, the network need to share model input pre-processing info, so that UE can be sure that it uses the same model input data as network used when training. UE also needs to receive quantization calibration data from NW so that UE knows how to quantize the received parameters (e.g., value range, quantization step size) |  |
| Nokia/NSB | This proposal is not aligned with earlier FL proposals on model ID. [Mod] Could you clarify? In the earlier proposal, a model ID refers to a logical model, which is also (quite obviously) a physical model in case of model transfer. This was clarified in the earlier model ID proposals. |  |
| Apple | Support |  |
| Sony | Support |  |
| AT&T | Support |  |
| Futurewei | Support in principle |  |
| InterDigital | OK |  |
| Intel | Support |  |

###### [FL2] Proposal 6-19b:

In model delivery/transfer Case z4, the “known model structure” means a model structure that has been previously identified and for whose model ID the UE has and explicitly indicated support in its UE capability reporting.

In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not appearing in z4.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

##### Issue 6-20: Proprietary model clarification

This is a continuation of Proposal 5-17a of the previous meeting.

###### [FL1][FL2] Proposal 6-20a:

Confirm the following working assumption from RAN#1 111 with the updates in Red.

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion:

| Proprietary models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format NOTE: A proprietary model can be supplemented with metadata that allows third parties to manage those with respect to the air interface without changing the model itself. |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

  - FFS: what a model’s metadata should contain

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared.

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic |  |
| Company | Comments |  |
| Google | Support |  |
| LG | If model is in proprietary-format, why the information of metadata needs to be studied in 3GPP? |  |
| NTT DOCOMO | We think even open-format model can be supplemented with metadata. Why is it only for proprietary model? |  |
| CMCC | Not sure why metadata is needed for proprietary-format model. |  |
| ETRI | We have similar view with LG. |  |
| CATT | Do not see strong need since metadata should anyway be discussed in model identification, for both proprietary format and open format. But we are not against the update since it seems not wrong. |  |
| Samsung | It is ok but this is somehow stating the obvious. |  |
| NEC | Like NTT DOCOMO, metadata information is also required for open-format model. It is not expected that 2 nodes can exchange the open-format model and without any other assistance information the nodes can understand the applicability of the model. |  |
| ZTE | Not necessary. Both proprietary-format model and open-format model should be associated with corresponding model description information for LCM purposes, which can be discussed in Issue 6-9. |  |
| Lenovo | The definition of the metadata is not clear to us at this point. |  |
| CAICT | We are not sure the process of third party to use metadata for LCM yet but fine for FFS. |  |
| Huawei, HiSilicon | Agree with ZTE, NEC, and DOCOMO, that: How to send meta data can be a separate discussion, and may be applicable to both Case z1/z3 and Case z3/z4/z5. |  |
| Fujitsu | Share the view of LG. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | We don’t see the need for the new note. Also share the view by Doccomo that “metadata”, when defined, can be valid for open-format as well. |  |
| Nokia/NSB | Similar view as some other comments above. |  |
| Apple | Agree with DOCOMO. Open format model also requires meta data. In fact, we think model ID based LCM require meta data. |  |
| Fraunhofer | Support |  |
| AT&T | Same view as Apple and DOCOMO. |  |
| Futurewei | Support |  |
| InterDigital | We agree with others, metadata should be defined first. |  |
| Intel | Not support, same question as DOCOMO |  |

##### Issue 6-21: Revision on AI/ML model transfer terminology

This is a continuation of 5-18 of the previous meeting.

Based on the WA,

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

the definition of model transfer shall be revised to reflect that the model is delivered via 3GPP signaling in collaboration Level z. This change is to reflect the group’s understanding of model transfer into the terminology definition. Otherwise, the definition is not consistent with the above working assumption.

###### [FL1] Proposal 6-21a:

The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface via 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic, LG, DCM, ETRI,Ericsson |  |
| Company | Comments |  |
| vivo | Support |  |
| Google | OK |  |
| LG | OK to clarify the terminology difference between delivery and transfer |  |
| CMCC | Support. |  |
| CATT | Support. |  |
| Samsung | We slightly prefer the following wording to be aligned with above WA. Delivery of an AI/ML model over the air interface in a non-transparent manner to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |  |
| Xiaomi | We support Samsung’s revision considering the previous WA regarding the boundary between level y and level Z Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z. - Clarifying note: Level y includes cases without model delivery. In our understanding, the wording of “transparent to 3gpp signaling” and wording of “via 3gpp signaling” have different meaning. We suggest to make update to align the WA |  |
| NEC | Support in principle |  |
| ZTE | OK with Samsung’s version. |  |
| Lenovo | OK |  |
| CAICT | Support. |  |
| Huawei, HiSilicon | OK |  |
| Fujitsu | OK. |  |
| NVIDIA | Support |  |
| Nokia | Not sure this is critical to clarify. RAN2 discusses possible options of transfer and RAN1 does not need to change terminologies. |  |
| Apple | Support |  |
| Fraunhofer | Support |  |
| Sony | Support |  |
| AT&T | Support. Fine with Samsung version. |  |
| Futurewei | Support |  |
| InterDigital | OK |  |
| Spreadtrum | Support |  |
| Intel | Support |  |

###### [FL2] Proposal 6-21b:

The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

#### Model inference operation

Huawei, HiSilicon

Proposal 20: Study the following aspects for pre/post-processing:

- Pre/post-processing methods, e.g. scalability to different input/output dimensions, channel conversion, quantization methods, etc.

- Potential spec impact on how to align the pre/post-processing methods between Network and UE.

ZTE

Proposal 17: 	 For model inference operation, further study:

- Data required for model input, e.g., reference signal configuration and assistance information delivery

- Report feedback based on the model output, e.g., quantization methods, UCI mapping order and priority

- Inference latency, e.g., the relationship between inference latency and CSI reference resource

Vivo

Proposal 22:	Study the assistance information needed for model inference.

Proposal 23:	Study ways for UE to report its capability for latencies with respect to the model inference.

Proposal 24:	Study UE capability on supported quantization levels.

Google

Proposal 4: For 1-side mode, Rel-18 should focus on the scenario that the model inference and training are in the same side.

Proposal 5: Study parallel model inference based on the same or different AI/ML models.

Nvidia

Proposal 13: For AI/ML based enhancements for NR air interface, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing.

Proposal 14: For AI/ML based enhancements for NR air interface, study potential specification impact related to report/feedback of model inference output and post-processing.

NEC

Proposal 7: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

NTT Docomo

Proposal 19: Study the processing unit framework for model inference, when multiple models are compiled at UE.

TCL Communication

Proposal 5: To reduce the signaling overhead between the UE and the gNB, a rule is needed to roughly classify the model complexity.

Proposal 6: Some constraints shall be added on the post-processing, in order to avoid obtaining an oversimplified low-performance model from post-processing.

##### Issue 6-22: (placeholder)

#### Functionality/model selection, activation, deactivation, switching, and fallback operation

RAN1 #110bis-e Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

Huawei, HiSilicon

Proposal 27: For model selection, activation, deactivation, switching, and fallback at least for UE-side models and two-sided models, take the following modes as baseline:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network, and the network indicates UE to execute the decision accordingly

Proposal 28: Study the case where Network pre-monitors the performance of the UE part model or UE-side model before model activation/switching for guaranteeing the network performance.

ZTE

Proposal 20: Network should make final decision on model/functionality selection, activation, deactivation, switching, and fallback for UE side/part model.

Vivo

Proposal 25:	Study event triggered model selection, activation, deactivation, switching, and fallback.

Proposal 26:	Study applicable conditions or applicability ID assisted model selection, activation, deactivation, switching, and fallback.

Proposal 27:	Study the mechanism for dynamic UE reporting of whether it has enough resources to run AI/ML models.

Oppo

Proposal 7: Besides generalized AI/ML models, scenario-dependent AI/ML models should be supported.

- Model switching should be supported because its specification impact is limited if the Local ID is supported.

- FFS: Specification impacts of model update.

Proposal 8: In Rel-18, for model selection, activation, deactivation, switching, and fallback,

- The NW-sided model is decided by NW.

  - The decision may be informed to UE, if needed.

- The UE-sided model is decided by UE.

  - The decision may be informed to NW, if needed.

- For two-sided models, focus on the mechanisms based on network decision.

Spreadtrum

Proposal 11: For network sided model, the following mechanisms can be further studied for model selection, activation, deactivation, switching, and fallback for network sided models:

- Decision by the network

  - Network-initiated

Nokia

Proposal 9: RAN1 to prioritize Network-controlled Functionality selection, activation, deactivation, switching, and fallback, and focus only on the following variants:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE – mainly for deactivation of an activated functionality

  - Event-triggered as configured by the network, UE’s decision is reported to network

Sony

Proposal 4: RAN1 should consider supporting the network initiated AI/ML model switching and event-trigger based AI/ML model switching for AI/ML model switching.

Xiaomi

Proposal 11: Study processing time for the AI model activation/deactivation/switch/fallback

Google

Proposal 6: For 1-side mode, the model selection/switching could be transparent.

Proposal 7: For 2-side mode, the model selection/switching can be configured by the NW or reported by the UE

Proposal 8: Consider to use lower layer signaling, e.g., MAC CE, for model activation/deactivation/fallback operation.

LG

Proposal 5: For UE-sided models, model selection/activation/deactivation/switching should be decided by the UE and no need to inform NW on the number of AI/ML models and the currently activated AI/ML model among them. Instead, UE may report updated UE capability/functionality and/or reliability/confidence of the reported values for NW to decide whether or not to use it.

Proposal 8: For two-sided models, model selection/activation/deactivation/switching can be decided by the NW.

CAICT

Proposal 5: The definition of model selection should be selection of one or more AI/ML models for activation among multiple models for the same functionality

Proposal 6: The definition of functionality selection could be selection of one or more functionalities for activation among multiple functionalities.

CMCC

Proposal 10: For the mechanism of model selection, activation, deactivation, switching, and fallback, if the decision is made by UE, UE’s decision should be reported to the network.

MediaTek

Proposal 10: Use ‘model control’ to stand for model selection, activation, deactivation, switching, and fallback to facilitate the discussion.

Proposal 11: Clarify model selection as the operation to select the first AI/ML model at the very beginning when AI/ML is enabled.

Proposal 12: Clarify the difference between model switching and model activation/deactivation and discuss the need to keep both.

Nvidia

Proposal 11: For AI/ML based enhancements for NR air interface, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 12: For AI/ML based enhancements for NR air interface, study potential specification impact related to assistance signalling and procedure for model performance monitoring, model update/tuning, and model selection/switching.

Qualcomm

Proposal 11: Model selection / switching should be based on well-defined conditions that are available during inference. Model monitoring is not a desirable mean for model selection / switching.

NEC

Proposal 8: Study adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling.

Proposal 9: Study autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

NTT Docomo

Proposal 8: Define model selection, model conversion, and model delivery as follows.

| Model selection | Activate one AI/ML model among multiple models within the same functionality. |
| --- | --- |
| Model conversion | Process of converting a trained AI/ML model into an executable form for inference at a target device. |
| Model delivery | Process of both model conversion and model delivery, where model conversion may or may not be performed before model delivery. |

TCL Communication

Proposal 3: The generalization of an ML model is needed to be discussed, according to model deployment, model switching, and alignment of applicable settings.

Proposal 8: During model switching, a backup model can be randomly selected, or according to the preference of UE or gNB.

##### Issue 6-23: functionality/model control

###### [FL1][FL2] Proposal 6-23a:

For functionality selection, activation, deactivation, switching, and fallback, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

For logical model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

UE may autonomously select and switch physical models within a logical model.

- FFS: NW’s awareness of UE’s physical model operations for certain purposes

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic, DCM, CAICT, Sony | LG |
| Company | Comments |  |
| vivo | Seems to be better discussed in each use case except the third bullet. |  |
| Google | Support |  |
| LG | Similar view as vivo. The last bullet already been addressed in 6-8. |  |
| ETRI | We have similar view with vivo. |  |
| CATT | In previous meeting, we think we already agreed that functionality related management is always indicated by NW in functionality-based LCM? Agreement For UE-side models and UE-part of two-sided models: … - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM ….. Is the intention of this proposal to distinguish ‘indication’ and ‘decision’? |
| Samsung | Ok but functionality selection and logical model selection may not be two different things. |  |
| NEC | Support in principle |  |
| ZTE | Not necessary as already have the following agreement. We can discuss whether the agreement can be extended to functionality based LCM in WI phase. Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network |  |
| OPPO | We are generally fine with the proposal. But for functionality, do we need mention functionality selection and switching? |  |
| Rakuten Mobile | Support |  |
| Lenovo | OK |  |
| Huawei, HiSilicon | OK in principle. |  |
| NVIDIA | Support |  |
| Ericsson | Share the view by vivo. |  |
| Nokia/NSB | Second part of the proposal is not needed. Also, as pointed by the CATT, most of the control is up to the gNB. We are ok to relax “Decision by the UE” is only for deactivation, not for other cases. |  |
| Apple | Support. Not critical given previous agreements. |  |
| Sony | Support. |  |
| AT&T | Support in principle. |  |
| Futurewei | Support in principle |  |
| InterDigital | Agree with Vivo. |  |
| Intel | Support |  |

##### Issue 6-24: Model selection terminology

This is the same as 5-21a of the previous meeting.

###### [FL1] Proposal 6-24a:

| Model selection | Selection of an AI/ML model for activation among multiple models for the same functionality Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |

|  | Yes | No |
| --- | --- | --- |
| Agree? | ETRI | Panasonic |
| Company | Comments |  |
| vivo | We prefer the following wording update. Selection of an AI/ML model for activation among multiple models for the same functionality AI/ML enable features. |  |
| Google | Support the original version. |  |
| Panasonic | In "for the same functionality", is it same meaning of " Case-1 and Case-2 correspond to functionality-based LCM" and exclude case -3 and 4 in Issue 6-7? We think some local model change in case 3 and 4 may be also called as model selection. |  |
| CATT | Fine in principle. But since the scope of functionality is still not clear, vivo’s update seems safer. |  |
| Samsung | Let us have common understanding on what functionality means first. |  |
| ZTE | As noted in our contribution, the model selection may depend on signaling framework design(e.g., RRC, MAC CE and DCI), which can be further discussed in WI phase. If the group thinks it’s necessary to define this terminology, we prefer the following revision: Selection of an one or multiple AI/ML models for activation among multiple models for the same functionality Note: Model selection may or may not be carried out simultaneously with model activation |  |
| OPPO | Generally fine. |  |
| Lenovo | OK |  |
| CAICT | Fine. |  |
| Huawei, HiSilicon | Agree with vivo’s version. We’d better decouple the two identification modes at this stage. |  |
| Fujitsu | Similar view as Panasonic. Model-ID-based model selection should be covered as well. |  |
| NVIDIA | Support |  |
| Ericsson | Support |  |
| Nokia/NSB | We assume this “model” usage is generic and not referring to logical model. |  |
| Fraunhofer | Support |  |
| AT&T | Support. Fine with vivo version. |  |
| Futurewei | As the term “functionality” has specific meaning in this environment, support vivo’s revision. |  |
| InterDigital | OK |  |
| Spreadtrum | Fine with vivo version |  |
| Intel | Support, vivo version is slightly preferable |  |

###### [FL2] Proposal 6-24b:

| Model selection | Selection of an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

#### Functionality/model monitoring

RAN1 #110bis-e Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN1 #110bis-e Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE RAN1 #110bis-e Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

##### FL comment 4-24 (closed) There are quite a few proposals on which entity should be monitoring the KPIs and which entity should be making monitoring decisions (activation/deactivation/switching/fallback) for one-sided and two-sided AI/ML models. As an example, some companies believe the monitoring decisions for one-sided AI/ML models should be exclusively made by the side at which the AI/ML inference is happening, whereas some other companies promote the prospect of (also) making monitoring decisions at NW side for UE-side AI/ML models. A similar trend holds for which side should be monitoring KPIs for one-sided and two-sided AI/ML models. From FL’s perspective, it would be more efficient and constructive if these discussions continue in the context of each respective use case, and FL believes more progress can be made by carefully analyzing the pros and cons of each methodology per use case, as opposed to exhaustively listing all the possible options in this agenda item.

Huawei, HiSilicon

Proposal 22: Study the potential procedures included by model monitoring, including data collection, measurement and report, AI/ML and non-AI/ML co-existence.

Proposal 23: For the container of Network side monitoring based on intermediate KPI, at least consider L1 signaling for the UE report to enable fast identification of AI/ML model failure.

Proposal 24: The input or output data based monitoring should be evaluated before being further discussed for potential spec impact, including: what metrics can be adopted for evaluating the distribution, how to generate the distribution of data, how accurate the data drift reflects the AI/ML model performance.

Proposal 25: Study the following three modes of model monitoring:

- Mode 1: Network collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision.

  - This case is applicable to Network-side model and two-sided model.

- Mode 2: UE collects inputs for monitoring, calculates monitoring KPIs which are then fed back to Network, and Network makes monitoring decision.

  - This case is applicable to Network-side model, UE-side model, and two-sided model.

- Mode 3: UE collects inputs for monitoring, calculates monitoring KPI, makes monitoring decision, and reports the decision to Network; Network will indicate UE to execute the decision accordingly.

  - This case is applicable to UE-side model.

  - Network may configure a threshold criterion (e.g., threshold throughput/RSRP, or threshold intermediate KPIs) to facilitate UE to make decision.

Proposal 26: Study the benchmark solutions for model monitoring, including at least:

- Non-AI/ML solution, to make the decision of deactivation/fallback based on the performance comparison with the undergoing AI/ML solution.

- AI/ML solution subject to an inactive model, to make the decision of model switching based on the performance comparison with the undergoing AI/ML solution.

ZTE

Proposal 18: Model selection/activation/deactivation/switching/fallback operation should be discussed separately from model monitoring.

Proposal 19: For UE-side/part model, depending on which side to calculate the model monitoring metrics and whether the model monitoring metrics should be reported, further study following options:

- UE-side model monitoring: model monitoring metrics are calculated by UE, and the model monitoring metrics are not reported to network side.

- Network-side model monitoring: model monitoring metrics are calculated by network (with/without the potential to inform UE about the model monitoring metrics).

- Hybrid model monitoring: monitoring metrics are calculated by UE, and then the model monitoring metrics are reported to network side.

Oppo

Proposal 9: Target to design a unified AI/ML inference monitoring mechanism supporting AI/ML model switching, 3GPP-based model transfer and model re-training.

•	Consider communication performance-based metrics (e.g. MSE, BLER, throughput) as starting point. Complexity and overhead are not considered as metrics for model performance monitoring.

•	Study performance prediction mechanism for an unused model.

•	Model re-training is considered with low priority.

•	Study evaluation methodology for performance monitoring approaches.

Proposal 10: Study on AI/ML training performance monitoring is low priority.

Proposal 11: For AI/ML model performance monitoring,

•	The performance monitor of a UE-sided model is the UE.

•	The performance monitor of a NW-sided model is the NW.

Nokia

Proposal 7: At least for UE-side models and UE-part of two-sided models, functionality or functionalities associated with the ML-enabled feature are always monitored based on an NW-controlled manner.

Proposal 8: At least for UE-side models and UE-part of two-sided models, to ensure performance is not impacted due to UE’s autonomous model-LCM changes (e.g., model update, switch) for a used/activated functionality, RAN1 to support NW to track the performance variations due to the UE’s model-LCM changes and partially control UE’s model-LCM changes.

- Study the exact performance monitoring framework and control mechanism to enable such a controllability

CATT

Proposal 13: For model monitoring, further study how to monitor the performance of inactive AI/ML models for better LCM. The following method can be considered as starting point:

- Monitoring based on input data distribution or applicable condition,

- Activating the inactive AI/ML models in turn, and monitors their performance based on inference accuracy/system performance.

Sony

Proposal 3: RAN1 should consider supporting both of periodic and aperiodic feedback some assistance information from UE-side for AI/ML model monitoring.

Ericsson

Proposal 17: Add the following as model monitoring KPIs: false alarm rate (FAR) and missed detection rate (MDR).

Proposal 18: One-sided models at the NW-side are monitored by the NW. Study data collection mechanisms to support inference-accuracy based model monitoring for NW-sided models if needed for a given use case.

Proposal 19: Study triggering conditions/events/signalling for UE-sided model fallback per use case.

Proposal 20: One-sided models at the UE-side are monitored by the UE.

Proposal 21: The performance monitoring of two-sided models is performed at the NW side. Study mechanisms to support inference-accuracy based model monitoring for two-sided models at the NW side, including UE reporting ground-truth together with UE-part model output to NW, periodic and event-triggered model monitoring procedures.

Xiaomi

Proposal 9: study the performance monitoring from the following two aspects

- Monitor the performance of activated AI model to assess whether to deactivate this model or update this model

- Monitor the possible performance of AI model not activated to assess whether to activate the AI model.

Proposal 10: Study the mechanism to enable fast performance report.

Google

Proposal 9: For 1-side mode, the model monitoring should be performed at the same side with the model inference and training, and study necessary information from the other side to assist the model monitoring

Proposal 10: For 2-side mode, further study the following options for model monitoring

- Option 1: The model monitoring is based on the input for the AI/ML model in transmitter side and the output for the AI/ML model in receiver side

- Option 2: The model monitoring is based on some performance related metric, e.g., hypothetical BLER, based on the output for the AI/ML model

Samsung

Proposal #8: Study direct model monitoring, e.g., monitoring based on inference latency, and indirect monitoring, e.g., monitoring based on system performance, input/output data distribution, application condition, per use case.

- Prioritize methods that do not require specifying monitoring metrics unless justified.

CMCC

Proposal 11: For NW-sided AI/ML model, study the following mechanism for model monitoring

- Atl1. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

MediaTek

Proposal 13: For network-decided and network-initiated mechanism, model monitoring is performed at the network side.

Proposal 14: For network-sided and UE-initiated mechanism, consider the cases that model monitoring is performed at the UE side or at both UE and network side.

Proposal 15: For UE-decided mechanisms, model monitoring is performed at the UE side.

Proposal 16: If model control is UE-autonomous without reporting the decision to the network, model monitoring and model control at the UE side is implementation specific and will not be specified.

Fraunhofer

Proposal 3: The AI/ML monitoring is extended to AI/ML monitoring and fault management. It comprises of a Fault Detection and a Fault Diagnosis function.

Proposal 4: Consider the requirements and applicability for simultaneous inference and monitoring at the UE.

Proposal 5: The AI/ML monitoring at the UE, can provide information to the NW at least on AI/ML model functionality, detected fault indicators and associated recommended actions, even for models identified only as logical models in the NW.

Proposal 6: In two-sided model for CSI feedback use case, model monitoring is performed at the gNB-side based on the transmitted CSI report from the UE.

NEC

Proposal 10: Consider to support model monitoring of multiple AI/ML models for the same functionality.

Proposal 11: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal 12: Study L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

NTT Docomo

Proposal 17: In performance monitoring, (near) real time-scale performance metrics should be provided to NW for reliable model activation/deactivation/switching/fallback in case of NW-controlled models

Proposal 18: Study the performance monitoring mechanism for inactive models and fallback operation in addition to active models.

TCL Communication

Proposal 7: At the inference stage, the AI/ML model has to be monitored. If the AI/ML model does not work properly, it can be replaced by a backup AI/ML model or fall back to the non-ML working way.

##### Issue 6-25: Model monitoring

###### [FL1] Proposal 6-25a:

For the purpose of model selection/switching, study methods to monitor the performance of inactive AI/ML models, including:

- Monitoring based on applicable condition

- Monitoring based on input data distribution

- Monitoring based on activating the inactive model and measuring the inference accuracy/system performance

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic, ETRI, CAICT | LG |
| Company | Comments |  |
| Google | We think we should add the following bullet. For example, for CSI report, different models correspond to different output size. With regard to the UCI payload size limitation, a proper model can be selected. - Monitoring based on the output data, e.g., output data size |  |
| LG | Performance monitoring of inactive models may be beneficial for fast model switching but it may induce a larger overhead/complexity. Before studying ‘methods’, we need to study its necessity and analyze pros/cons. |  |
| NTT DOCOMO | The clarification on “monitoring based on the applicable condition” is necessary to further study it. |  |
| CMCC | Support in principle. |  |
| CATT | Support. Minor update: - Monitoring based on activating the inactive model(s) and measuring the inference accuracy/system performance |  |
| Samsung | ###### We propose minor update For the purpose of model selection/switching, study methods to monitor the performance of inactive AI/ML models/functionalities, including: - Monitoring based on applicable condition - Monitoring based on input data distribution - Monitoring based on activating the inactive model and measuring the inference accuracy/system performance |  |
| Xiaomi | ###### Support in principle and OK with CATT’s update and SS’s update |  |
| NEC | Support in principle. But we may require further discussion on the third sub-bullet. It is indeed correct that an inactive model can be activated to identify its performance and use that information for model switch/selection. But such kind of trial and error approach also means that if the model is not performing well then it will deteriorate system performance. We would also like to study a scenario where a model can be operated at UE for only monitoring, while actual radio operation is performed using legacy procedure or using a different model id. For e.g., for CSI compression/decompression use case, model id-X can be used for monitoring purpose (i.e., CSI compression output by UE can be reported via RRC which is used by network for its inference) but at the same time UE may using legacy CSI reporting procedure so that system performance remains stable. |  |
| ZTE | Our understanding is that applicable conditions are attributes of a model, which is to define a scope of the model and to differentiate different models/functionalities. However, model monitoring is to monitor the performance of multiple models in the same applicable conditions. Therefore, the applicable conditions can be further discussed in model/functionality identification. For model monitoring, we should focus on the last two bullets. |  |
| Huawei, HiSilicon | 1) We understand that the 1st and 2nd bullets are no spec impact? If so, merge into one bullet. 2) For the 3rd bullet, we are not sure whether running an inactive model should be called as “activating”, so use a more generic wording of “perform inference”. 3) the intention of monitoring the inactive model is added as “for performance comparison with the currently active model” For the purpose of model selection/switching, study methods to monitor the performance of inactive AI/ML models, including: - Monitoring based on applicable condition - Monitoring based on input data distribution - Implementation based monitoring, e.g., data distribution, applicable condition, etc. Monitoring based on performing inference on activating the inactive model and measuring the inference accuracy/system performance, for potential comparison with the currently active model |  |
| Fujitsu | Support in principle. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | We don’t see the need to study the aspect of monitoring “inactive” models. This should be handled transparently at the UE. We should first progress on studying monitoring active models in each use case prior to also studying inactive model monitoring. |  |
| Nokia/NSB | Not required to be discussed as this is an implementation issue. |  |
| Apple | OK |  |
| Fraunhofer | Support the proposal and the update from CATT |  |
| AT&T | Support in principle. Fine with CATT and Samsung edits. |  |
| Futurewei | Monitoring performance of something inactive (i.e., not in operation) sounds strange. Suggest change “inactive” to “stand-by”, as these models are waiting to be used. |  |
| InterDigital | The notion of activation could be reserved only for inference purpose. There may be other means to determine performance of inactive model without activating it. • Monitoring based on activating the inactive model and measuring the inference accuracy/system performance of inactive model |  |
| Spreadtrum | We are generally fine. |  |
| Intel | We think this is premature, its not clear what specification is needed for this |  |

###### [FL2] Proposal 6-25b:

(FL note: The proposal wording has been updated to add clarity, and to account for the comments.)

For the purpose of model selection/switching, study necessity and potential specification  impact for methods to evaluate the applicability of inactive AI/ML models/functionalities, including:

- Evaluation by comparing the model's applicability condition to the current conditions

- Evaluation based on input data distribution

- Evaluation through model monitoring by using the inactive model(s) for inference and measuring the inference accuracy/system performance

  - FFS: how to mitigate resulting system performance impact if any

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

#### Model update

FL comment from RAN1 #111 ##### Online training Online training at the network: FL believes that this is a matter of implementation at the network side, and there is no strong reason to explicitly discuss offline/online training separately. Online training at the UE: FL thinks that the group can first discuss the LCM framework not involving online training at the UE, and treat online training at the UE as additional discussions. Over-the-air training between NW and UE: Given the amount of specification impact and practical value of such specification, FL thinks that we can deprioritize it.

ZTE

Proposal 6: For further study on model update, model parameter update is preferred than model structure update.

Panasonic

Proposal 3: Not to confirm working assumption of model parameter update.

Proposal 4: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

CAICT

Proposal 7: The process of model updating only applies for deactivated model. If an activated model needs to be updated, it should be deactivated first.

NEC

Proposal 13: Study the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

##### Issue 6-26: (placeholder)

#### UE capability

FUTUREWEI:

Proposal 12: When studying UE AI/ML related capabilities, separate physical capabilities from functional capabilities.

Proposal 13: For UE physical capabilities, consider categorizing them that reflects their ability in handling various AI/ML complexities, including pre- and post-processing.

Huawei, HiSilicon:

Proposal 30: Study the reporting mechanism due to varying UE capability for a specific AI/ML model or for an AI/ML functionality.

ZTE:

- Proposal 21: Further study UE capability to support AI/ML models, at least consider:

Dynamic UE capability report in addition to conventional fixed UE capability report;

Concurrent UE capability for conventional method and AI based method.

- Proposal 22: Discussions on LCM-related UE capabilities can be postponed or studied per use case basis.

Vivo:

Proposal 21: Study ways for UE to report its capability for data collection regarding expected pre-processing, data storage, feature extraction and report for data collection.

Proposal 23: Study ways for UE to report its capability for latencies with respect to the model inference.

Proposal 24: Study UE capability on supported quantization levels.

Proposal 27: Study the mechanism for dynamic UE reporting of whether it has enough resources to run AI/ML models.

Nokia:

Proposal 4: UE capability reporting to be used only for reporting static applicable conditions.

Proposal 5: RAN1 to study whether an additional reporting method is needed for reporting applicable conditions in addition to UE capability reporting framework.

CATT:

Proposal 16: For support of AI/ML, consider defining several levels of UE capabilities

based on one or more following aspects:

- Hardware related capability

  - Storage,

  - Computation power,

- LCM related capability

  - Capability of online training,

  - Capability of data collection for model training,

  - Capability of implementing downloaded AI/ML model (i.e. collaboration Level z).

Google:

Proposal 12: For AI/ML based operation, the following UE types should be considered:

- Type 1 UE (low performance UE): AI/ML based operation is based on general processing unit (GPU)

- Type 2 UE (high performance UE): AI/ML based operation can be based on neural processing unit (NPU)

CMCC:

Proposal 12: For AI-related UE capability, how to define and report the capability of training, power, computation, storage should be studied.

Nvidia:

Proposal 15: For AI/ML based enhancements for NR air interface, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

NEC:

Proposal 16: Introduce AI/ML processing units (APUs) to reflect UE capability of AI/ML operations.

Proposal 17: Support UE capability reporting on the supported number of AI/ML models for parallel model monitoring and on the supported methods for model monitoring.

##### Issue 6-27: (placeholder)

#### Interoperability and testability aspects

Agreement (RAN1 #110bis-e) Study various approaches for achieving good performance across different scenarios/configurations/sites, including - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.] - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example. FL recommendation 3-73d from RAN1 #110bis-e Companies are encouraged to bring discussion on interoperability and testability aspects, including, but not limited to, the following: - Discussion on testing model generalization performance - Discussion on two-sided AI/ML model interoperability and testing - Discussion on involvement of multiple parties including UE, NW, and TE vendors how to support full NW-UE interoperability - Discussion on how to handle multiple models (e.g., model switching, model selection) - Discussion on how to handle model update (e.g., offline and online model update) - Whether and how to test LCM This discussion can also serve as an input for later RAN4 study.

FutureWei

Proposal 14: Study common assumptions, topics, and guidelines for the discussion of interoperability.

Note: this may be use case dependent.

Huawei, HiSilicon

Proposal 31: The performance test at RAN4 can consider the following two options

- Option 1: Static scenario, where a fixed scenario is considered to construct the testing dataset.

- Option 2: Variable scenario, where more than one scenario is considered to construct the testing dataset to separately test the same AI/ML model.

Proposal 32: RAN1 may need to provide information on the typical scenarios for performance test in RAN4, e.g., by referring the set of scenarios for the generalization verification.

Proposal 33: RAN1 may need to provide training dataset related information to RAN4 for determining the performance requirements. The following two options can be considered:

- Option 1: Construct a common training dataset for each scenario.

- Option 2: The methodology of generating the training dataset, e.g., the EVM of simulation for each scenario, and the method to sample the data in simulation.

Proposal 34: To facilitate RAN4 to perform the test on training, RAN1 may need to provide information on the procedures of training types that have been identified.

Observation 25: RAN4 testing is applied before model deployment and is to ensure the AI/ML model can reach the minimal requirement,

- RAN4 testing does not necessarily be applied to each single model that has been developed.

- After model deployment in the realistic network, the performance and robustness of the model is guaranteed by model monitoring.

Vivo

Proposal 9:	Consider to define reference model structure in RAN4 or RAN1.

Ericsson

Proposal 30: RAN1 should consider requirement setting and testing feasibility of proposed AI PHY solutions but any requirement- and testing-related decisions should be taken by RAN4.

Panasonic

Proposal 6: The need of test or pre-commercial deployment after online training before the deployment needs some discussion.

Qualcomm

Proposal 13: Study feasibility of RAN4-like tests over-the-air on deployed UEs.

##### Issue 6-28: model validation

###### [FL1] Proposal 6-28a:

Study methods to validate a UE-side model or two-sided model prior to deployment for active use, including:

- Validation based on applicable condition

- Validation based on activating the model for testing and measuring the inference accuracy/system performance (e.g., RAN4-like test)

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic, CAICT |  |
| Company | Comments |  |
| vivo | We can send LS to RAN4 and ask them to take into account. |  |
| Google | We think this can be deprioritized. |  |
| NTT DOCOMO | Support the principle of studying the validation of models before the active use. However, validation based on applicable condition is not clear. If it means the validation can be per applicable scenario, configuration, or deployment, we prefer to update as follows. Study methods to validate a UE-side model or two-sided model prior to deployment for active use, including: - Validation based on applicable condition - Validation based on activating the model for testing and measuring the inference accuracy/system performance (e.g., RAN4-like test) - Validation can be per scenario, configuration, or/and deployment. |  |
| ETRI | Prior to the discussion, there should be a commendation about what the purpose of validation is, at what stage and how often it happens. |  |
| CATT | So far we consider model validation as part of model training/development, which is likely up to implementation. Not sure what study can do/provide. |  |
| Samsung | What is the difference between this and extended monitoring? Is it to mean the inference result will not be utilized for other purposes than monitoring? What are the use cases? |  |
| Xiaomi | In our view, this proposal may fall into RAN4 scope. And In addition, it is also similar to the monitoring of inactive model for activation purpose. |  |
| ZTE | We think the validation is necessary. However, it’s better to check RAN4’s views. |  |
| Lenovo | It is necessary to clearly identify the motivation for such ‘validation’ in context. If it is for active use, we suggest considering this issue within the mechanism for ‘model selection’. |  |
| Fujitsu | We think model validation is important. Considering the limitation of RAN4 test, and number of models being used for a UE feature, there is a high possibility that many model have no chance to be tested (IOT test) before using in network. But it would be better to check RAN4’s view first. |  |
| NVIDIA | Support the proposal in principle. |  |
| Ericsson | This could be handled in RAN4. |  |
| Nokia/NSB | OK |  |
| Apple | RAN4 scope |  |
| Fraunhofer | The meaning of validation based on applicable condition is not very clear for us. |  |
| NVIDIA | Support the proposal in principle. |  |
| InterDigital | Prefer to check with RAN4 |  |
| Intel | Is it intended to have RAN1 specification support for validation ? |  |

###### [FL2] Proposal 6-28b:

(FL note: While there are similarities to monitoring, the goal of the validation discussed in this proposal is to determine whether a deployed model performs well in an applicable condition before it can be activated. RAN4 tests occur prior to a UE being deployed, and cannot test every model in different scenarios and conditions. Unlike a RAN4 test which tests a UE in controlled conditions, the validation procedure may test a model in live conditions, and can possibly involve multiple UEs. The proposal below has been updated based on comments.)

Study methods to validate a UE-side model or two-sided model after deployment but prior to active use, including:

  - Validation based on activating the model for testing and measuring the inference accuracy/system performance

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |

## Use cases

<empty>

## Evaluations

### Common evaluation methodology and KPIs

#### Dataset and model disclosure

Samsung

Proposal #12: For evaluation purpose, companies should report their results with, at least, a higher level description of their AI/ML model. higher level description includes

- Types of neural network for AI/ML model, e.g., CNN, LSTM, transformer, etc.

- Number of layers

CMCC

Proposal 14: A common data set for each use cased could be encouraged to be constructed for evaluation and cross-checking of performance.

Nvidia

Proposal 3: Companies are encouraged to contribute real data to the 3GPP Rel-18 AI/ML study for NR air interface to help start to build up sets of real data in 3GPP.

#### Model generalization

Samsung

Proposal #13: The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Proposal #14: For approaches to achieve good performance across different scenarios/configurations/sites, e.g., model generalization, model switching, model update, etc., study

- Approaches to discover and/or report scenarios/configurations/sites

- Model input/output pre/post-processing and the additional side-information  required to achieve           model  generalization

CMCC

Proposal 15: The average performance under multiple configurations / scenarios should be evaluated to evaluate the generalization capability of AI/ML model.

Proposal 16: The performance loss of intermediate or eventual performance KPIs using configurations / scenarios-common models over configurations / scenarios-specific models can also be adopted as the metric for evaluating the generalization performance.

Nvidia

Proposal 5: From a common framework’s perspective, introduce “in-distribution generalization” and “out-of-distribution generalization” in the terminology list and leave the details of generalization types to the discussion of each use case.

Proposal 6: In-distribution generalization: training and test data have the same distribution.

Proposal 7: Out-of-distribution generalization: training and test data do not have the same distribution.

#### Common KPIs

Agreement from RAN1 #110 The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIs Note: Use-case specific KPIs may be additionally considered for the given use-case. Agreement from RAN1 #110-bis-e The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency) Conclusion from RAN#1 110-bis-e This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP. Agreement from RAN1 #112 For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

Huawei, HiSilicon

Proposal 35:  Adopt power consumption in common KPI for evaluating the performance benefit of AI/ML. Companies are encouraged to report power consumption for the AI/ML model as part of the evaluation.

Ericsson

Proposal 28: For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values based on HLO representations before optimization (i.e., not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included for a fair cross-company comparison.

Proposal 29: To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined per use case, as follows:

- Small models (e.g., < 1 M model parameters for positioning use cases)

- Medium-size models (e.g., 1 – 8 M model parameters for positioning use cases)

- Large models (e.g., > 8 M model parameters for positioning use cases)

CMCC

Proposal 17: The model size can be adopted as one representative KPI to evaluate the overhead of model delivery/transfer.

Proposal 18: The inference latency can be adopted as one common KPI when evaluating the performance of AI/ML model.

Proposal 19: When the performance monitoring metric/method is the input or output data-based monitoring method, such as data drift between training dataset and observed dataset or out-of-distribution detection, the overhead of storing these data and the complexity to compute the input or output data-based KPIs need to be considered.

Nvidia

Proposal 4: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for NR air interface.

Lenovo

Proposal 16: Consider latency as one of the KPIs/Metrics for the common aspects of an evaluation methodology:

- Latency

  - Latency for data collection for model training and update.

  - Latency for LCM procedures, e.g., model monitoring, update, training data transfer, model activation, deactivation, switching, and fallback operation.

Proposal 17: 	Evaluations of an AI/ML scheme should include analysis of the latency/delays introduced by the AI/ML procedures (e.g., model training, update) and comparisons with the latency requirement of the system and latency for baseline Rel-17 schemes.

NTT Docomo

Proposal 20: Companies can voluntarily provide their models estimating power consumption model based on FLOPs with their expected implementations.

##### Issue 6-29: (placeholder)

## Potential Specification Impact Assessment

<empty>

### General observations

<empty>

### PHY layer aspects

<empty>

### Protocol aspects

<empty>

### Interoperability and testability aspects

<empty>

## SI structure

##### Issue 6-30: Rel-18 SI prioritization

Note: This is continuation from 5-30b of the previous meeting.

Considering time constraints in Rel.18 and current progress, RAN1 may need to narrow down the scope of the study and deprioritize the most challenging topics, leaving those for study in future releases. At the same time, it is important to ensure that the AI/ML framework produced in Rel.18 is future-proof, i.e., it can be extended with more disruptive features in the future without reverting to agreed solutions.

###### [FL1][FL2] Proposal 6-30a

To ensure timely completion of Rel-18 study, focus Rel-18 discussions on core aspects necessary for Rel-19 WI specification, while treating challenging topics as supplementary discussions for assessment. Rel-18 study on core aspects should strive for future-proof AI/ML framework.

Assumptions for core aspects

- Offline model development and training

  - Note: This does not imply that data collection/delivery is offline.

- Model conversion and UE capability verification prior to delivery/transfer to UE

Topics for supplementary discussions:

- Online training at the UE side

- Model transfer for one-sided models

- Model transfer via open format models

Topics to deprioritize:

- Model delivery/transfer Case z5

- Over-the-air training between NW and UE

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | Immature timing and inappropriate place to discuss this. |  |
| Google | Agree with vivo |  |
| Panasonic | Although it looks reasonable, we would like to conclude the meaning of each as the meaning of each are under the discussion in this agenda. |  |
| LG | We support prioritization discussion for timely completion of this SI. Our preference is to deprioritize all the model transfer related topics. |  |
| CATT | OK for the deprioritizing part. Would like to continue study of others. |  |
| ZTE | We understand the intention here. However, the proposal only includes the prioritization on model training and model transfer/delivery. Do we also need to the discuss the prioritization on other aspects(e.g., data collection, model/functionality identification, model monitoring and decision on model activation/deactivation/selection/switching. |  |
| OPPO | Generally fine. |  |
| Huawei, HiSilicon | In general, we share a similar view with vivo, Google that the deprioritize may be decided after we make further investigation. See some views/comments in below: Offline model development and training – we think there is no issue/proposal discussing to how to perform model development or the on-line learning method such as reinforcement learning? If so, there seems to be no specific need to highlight this issue. Model conversion and UE capability verification prior to delivery/transfer to UE – agree. Online training at the UE side – same to the “Offline model development and training”, that there is no issue/proposal discussing it naturally? Model transfer for one-sided models - agree Model transfer via open format models – we think Case z4/z5 are still on the table, so there is no need to deprioritize these two modes. Model delivery/transfer Case z5 – same view as above, that we are hesitate whether it needs to deprioritize z5 so urgently. In our understanding, if UE side and NW side can align somehow on the profile of the model, such as the backbone, layers, etc. (rather than exactly the full structure), NW side can develop a unified model supported by all/multiple UE vendors. Otherwise the models are totally customized which imposes a big burden to NW side to store/maintain. Over-the-air training between NW and UE – Type 2 training is already deprioritized in 9.2.2.2, so there seems to be no redundant conclusion here. |  |
| Fujitsu | It can be concluded later based on the study result per use case. |  |
| Ericsson | Support the proposal. Given the amount of time left in the study item, it would make sense to select which topics to prioritize. |  |
| Nokia/NSB | It is not clear what model conversion means here. |  |
| Futurewei | We prefer to discussing this later in the process after we clarify some issues currently be discussed. |  |
| Intel | We are open to discuss this but we also need to be careful here – we can and should accept a broader study in Rel-18 (even if not fully mature) and not reject aspects simply because there is urgency of Rel-19. TU for Rel-19 should be part of Rel-19 discussion and should not impact Rel-18. |  |

### RAN1 sub-agendas

### Coordination with RAN2 and SA

Huawei:

Proposal 19: Send LS to SA2/SA3/SA5 for clarifying the feasibility and potential SA impact on functionality identification-enhanced and model identification.

- For functionality identification-enhanced, the potential SA impact from the aspects of globally unique scenario ID, and globally unique dataset ID.

- For model identification, the potential SA impact from the aspects of globally unique model ID, and model registration

vivo:

Send LS to SA2 and SA4 to study the potential specification impact of at least model transfer/deliver, model training, data collection and model identification.

Nvidia:

Proposal 8: Coordinate with SA5 on AI/ML model life cycle management.

ZTE:

Send LS to RAN2/SA2 to study further details of the model identification Type A1, Type A2 and Type B (e.g., how to register/de-register the model and how to conduct certification test on the model).

RAN1 concludes the pros and cons of supporting “proprietary model” and “open-format model”. Then, RAN1 should send LS to other working groups (e.g., RAN2 and SA2) to check the feasibility of supporting open-format models.

CATT:

Proposal 14: Leave the discussion of model delivery/transfer to RAN2. RAN1 may provide some input by sending LS if relative agreements are reached.

##### Issue 6-31:

###### [FL1][FL2] Discussion 6-31a:

Please share your views on sending LS to RAN2, SA2, SA4, and/or SA5 on model transfer/delivery, two-sided model training, UP-based data collection, and model identification.

| Company | Comments |
| --- | --- |
| ZTE | We’re fine to send LS to other WGs. However, we should have some common understandings in RAN1 before sending the LS. For example, pros and cons of different model transfer/delivery options, the feasibility of two-sided model training methods(it’s under discussion in 9.2.2.2) and types for model identification. |
| vivo | Support |
| Huawei, HiSilicon | Support. The globally unique ID (e.g., scenario ID, dataset ID, etc.), model identification Type A1 need to involve SA2/SA3/SA5. For some model transfer cases with offline delivery (e.g., Case [y]/z1/z2/z3). |
| Fujitsu | Support |
| NVIDIA | Support |
| Nokia/NSB | No clear details mentioned on what shall be sent to other WGs. |
| AT&T | Support |
| Futurewei | Support. In fact RAN2 is doing something in parallel with RAN1, for example, defining the functional framework. A better communication with RAN2 and other related groups is needed to coordinate the project. We need to tell other groups clearly what we wish they can do and what they better wait for our decisions. |
| Intel | Depends on the kind of LS – is it just agreements or something else, better to discuss in context |

### Coordination with RAN4

## Others

<empty>

# GTW session (4/28/2023 Tuesday)

###### Proposal 6-7b:

Align the functionality terminology such that:

- Functionality refers to either an entire AI/ML-enabled Feature/FG (i.e., a sub use case) or a combination of specified RRC configurations of an AI/ML-enabled Feature.

- Correspondingly, functionality-based LCM operates based on specified AI/ML-enabled Feature/FG (i.e., a sub use case) or a combination of specified RRC configurations of an AI/ML-enabled Feature.

  - FFS: Signaling to identify dynamic parts (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

- FFS: the granularity of functionality

- FFS: which aspects should be specified as components of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

Align the model terminology such that:

- It is acknowledged that a model that the NW is aware of may be logical and that multiple physical AI/ML models may exist transparently to NW under a model ID.

- Correspondingly, model-ID-based LCM operates based on identified models, where a model may be associated with specific combinations of static parts (e.g., specified RRC configurations) and dynamic parts (e.g., scenarios, sites, and datasets) and. The dynamic parts may be determined offline/online between UE-side and NW-side by a vendor or via collaboration across vendors.

- FFS: Which aspects should be considered as signaling such as assistance information that are made available for model training and identification to be included into model description information during model identification will be discussed in each sub-use-case agenda.

###### Proposal 6-8b:

Extend the notion of a model into a logical model.

- Physical AI/ML model is a model that tangibly exists, e.g., proprietary-format model, open-format model. Physical model can be identified and used in signaling.

- Logical AI/ML model is a model that is identified (e.g., via a local of global model ID) and used in signaling.

- Note: A logical model may be implemented by related to one or multiple physical models, e.g., multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms, or multiple physical models transparent to NW that are identified as a single model ID.

- Note: Physical and logical models are NOT mutually exclusive. For example, in model transfer, the model being transferred is both logical (as it is identified and used in signaling) and physical (as it tangibly exists as either proprietary-format or open-format).

Clarify that the “model” discussed for potential 3GPP specification impacts in the Study Item is the AI/ML model described on “logical” level, unless specifically mentioned as a physical model.

- In particular, model-level-LCM is based on logical models, and UE may have physical model operations transparently to the NW.

- FFS: NW's awareness of physical model operations, such as for monitoring, test, certification, and interruption during physical model switching

A logical model identified by a model ID may correspond to,. More precisely, a model ID may correspond to For for example,

- For UE-side models in Collaboration Level y: A logical unit that is identified between UE and NW based on applicable conditions scenarios, configurations, sites. A group of physical models that is identified as one model ID for LCM purposes.

- For two-sided models in Collaboration Level y: An ID indicating compatibility between UE-side and NW-side models

- For Collaboration Level z: A physical model that is transferred from NW to UE.

# GTW session (placeholder)

# GTW session (placeholder)

# GTW session (placeholder)

# GTW session (placeholder)

# FL notes for the next meeting

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- 3GPP TR 37.817, Technical Specification Group RAN; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancement for Data Collection for NR and EN-DC (Release 17)

- R1-2205522, “Summary of General Aspects of AI/ML Framework”, RAN WG1 #109-e, Moderator (Qualcomm)

- Draft Report of 3GPP TSG RAN WG1 #109-e

- R1-2208178, Summary#1 of General Aspects of AI/ML Framework, RAN WG1 #110, Moderator (Qualcomm)

- Draft Report of 3GPP TSG RAN WG1 #110

- R1-2210708, Summary#5 of General Aspects of AI/ML Framework, RAN WG1 #110bis-e, Moderator (Qualcomm)

- R1-2210801, Report of RAN1#110bis-e meeting

- R1-2213003, Revised final summary of General Aspects of AI/ML framework, RAN WG1 #111, Moderator (Qualcomm)

- R1-xxxxxx, Report of RAN1#111

- R1- 2301868, Final Summary of General Aspects of AIML Framework, RAN WG1 #112, Moderator (Qualcomm)

# Working list of terminologies

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table 3: Working list of terminologies

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model download | Model transfer from the network to UE |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model parameter update | Process of updating the model parameters of a model |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model upload | Model transfer from UE to the network |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Unsupervised learning | A process of training a model without labelled data. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

# Agreement from RAN#1 109-e

Agreement

- Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

- Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

# Agreement from RAN#1 110

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIs

Note: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

# Agreement from RAN#1 110-bis-e

Summary#1 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)	(rev of R1-2210375)

From Oct 11th GTW session

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

R1-2210472	Summary#2 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)

From Oct 13th GTW session

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

- FFS: Detailed discussion of model ID with associated information and/or model functionality.

- FFS: usage of model ID with associated information and/or model functionality based LCM procedure

- FFS: whether support of model ID

- FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

R1-2210661	Summary#3 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

From Oct 18th GTW session

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

- Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

- Monitoring based on inference accuracy, including metrics related to intermediate KPIs

- Monitoring based on system performance, including metrics related to system peformance KPIs

- Other monitoring solutions, at least following 2 options.

  - Monitoring based on data distribution

    - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

    - Output-based: e.g., drift detection of output data

  - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

From Oct 19th GTW session

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

- Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

- Overhead (e.g., signaling overhead associated with model monitoring)

- Complexity (e.g., computation and memory cost for model monitoring)

- Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

- FFS: Power consumption

- Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

- Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

- Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

  - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

- Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

# Agreement from RAN#1 111

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

# Agreement from RAN#1 112

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

# Agreement from RAN#1 112-bis-e