3GPP TSG RAN WG1 #112bis-e			R1-2304069

e-Meeting, April 17th- April 26th, 2023

Agenda item:	9.10.2

Source: 	Moderator (CATT)

Title: 	Moderator summary on Timing advance management for LTM: Round 3

Document for:	Discussion and Decision

- Introduction

In RAN #94e, the Rel-18 WID of Further NR mobility enhancements are approved [1]. In the approved WID, Timing Advance management is a part of RAN1 objectives,

To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

This summary includes the following:

- Summary of companies’ views on each of open issues raised by interested companies

- Observations and recommended proposals based on the summary of companies’ views

- Issue 1: TA acquisition of RACH-based solutions

Open issues on RACH-based solutions for TA acquisition of candidate target cell(s) and company views are summarized below.

Table 1. Summary of views on Issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | When reception of RAR is not configured for RACH-based solution, whether UE should initiate re-transmit PRACH - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax | Alt1: Huawei, Ericsson, CATT, Spreadtrum, Lenovo, Intel, CMCC, CAICT, KDDI, Interdigital, NTTDoCoMo, MTK, Qualcomm, Apple, ITRI, KDDI, Nokia, Xiaomi Alt2: ZTE, vivo, Oppo, Transsion Holdings, Google |
| 1.2 | If Alt1 is supported for Issue #1.1, how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - Alt 1: UE transmits PRACH with power ramping for subsequent PRACH triggered by PDCCH order for the same candidate cell - Alt 2: Whether power ramping is performed or not is indicated in PDCCH order - Alt 3: Tx power is indicated in PDCCH order - Alt 4: Use the same Tx power as the first PRACH transmission triggered by PDCCH order | Alt1: KDDI Alt2: NTTDoCoMo, Huawei, MTK, Apple, Qualcomm Alt3: Interdigital, Ericsson, ITRI Alt4: |
| 1.3 | When reception of RAR is configured, whether RAR is received from serving cell or candidate cell - Alt1: RAR is received from serving cell - Alt2: RAR is received from candidate cell | Alt1: Huawei, Ericsson, CATT, Futurewei, ZTE, vivo, Oppo, Spreadtrum, Intel, Samsung(baseline), CMCC, CAICT, KDDI, Transsion Holdings, Interdigital, Apple(intra-DU), Qualcomm, NTTDoCoMo(Spcell), ITRI, Nokia, LGE Alt2: Huawei(for inter-DU scenario), ZTE(configure Type-1 CSS for each candidate cell), Samsung |
| 1.4 | When reception of RAR is configured, whether other parameters such as UE ID, candidate cell ID is contained in RAR - Alt1: additional parameters are not needed - Alt2: candidate cell ID/UE ID is contained in RAR - Alt3: others. E.g. introduce a new designed MAC CE for RAR | Alt1: ZTE, CATT, Samsung, vivo, Spreadtrum Alt2: CMCC(Candidate Cell ID), NTTDoCoMo(UE ID), OPPO(candidate cell indicator), MTK Alt3: Ericsson(a new MAC CE including additional bits to indicate it is transmitted in response to a PRACH received in a candidate cell, the new MAC-CE would be transmitted using C-RNTI to reach the desired UE), Qualcomm, Nokia(depends on the RA-RNTI/C-RNTI used) |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P1-1: closed

Round 1

Proposal 1-1: For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed. To this end, the following alternatives are to be discussed and down-selected.

- Alt 1: if reception of RAR is not configured, the number of allowed PRACH transmission is always set to PreambleTransMax=1

- Alt 2: if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Support. Support Alt2: PreambleTransmax cannot be set to 1 since it’s applicable to any RACH procedure. |
| Apple | Support. Specifically, prefer Alt.2 to avoid unnecessary signaling overhead. Note that this is for a candidate cell and CFRA configuration is provided additionally for it. Alt.1 needs to add ‘PreambleTransMax’ with a ‘fixed’ value, which simply increase overhead without any benefit. |
| QC | Support and prefer Alt2 |
| Futurewei | Support Proposal 1-1 and prefer Alt2. |
| OPPO | Support, and prefer Alt2. |
| CMCC | Support. Prefer Alt 2. If the UE autonomous re-transmission of PRACH is not allowed, there is no need to further configure the value of PreambleTransMax in this case. |
| Nokia | Support Alt2 as this is a simpler option and does not require any additional signaling overhead. |
| Lenovo | Support and prefer Alt2. |
| ZTE | Alt 2 is slight preferred due to it is unnecessary to introduce new value for PreambleTransMax. But in our view, this proposal mainly has impacts on section 5.1 of TS 38.321, and while for RAN1 spec, there is less impact regardless Alt 1 or Alt 2. |
| vivo | Our preference is to support UE autonomous re-transmission of PRACH. If majority of companies prefer “UE autonomous re-transmission of PRACH is not allowed”, then we are fine to support alt2. |
| Xiaomi | Support. And prefer Alt.2 |
| Samsung | Support with Alt 2 |
| ITRI | Support and prefer Alt.2. |
| MTK | Support, and prefer Alt2. |
| Spreadtrum | Support. Alt 2 is preferred. |
| NTT DOCOMO | Support and prefer Alt2. |
| Huawei, HiSilicon | Support the FL proposal and Alt 2 is preferred. |
| IDCC | Support and prefer Alt2. |
| LGE | Support and Alt 2 is slightly preferred. |
| Mod | Based on feedback provided by companies, P1-1 is updated accordingly: Proposal 1-1: For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax. |

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.

P1-2

Round 1

Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured, on how to determine the transmit power of subsequent PRACH triggered by PDCCH order, discuss and down-select among the following alternatives:

- Alt 1: UE transmits PRACH with power ramping for subsequent PRACH triggered by PDCCH order for the same candidate cell

- Alt 2: Whether power ramping is performed or not is indicated in PDCCH order

- Alt 3: Tx power is indicated in PDCCH order

- Alt 4:  Use the same Tx power as the first PRACH transmission triggered by PDCCH order

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Alt2 and Alt3 can probably be combined into one proposal |
| Apple | Alt.2 (Preferred) or Alt.3. In our view, the Alt.2 and Alt.3 are identical in terms of function i.e., power ramping for retransmitted PRACH. The difference maybe in signal details. For example, even Alt.2, power ramping or not is controlled by PDCCH order, which is same as Alt.3. Alt.2 tries to mimic the current power ramping procedure defined in the existing spec with way to inform UE whether the PRACH is an initial or retransmitted PRACH. Alt.1 seems always increases the PRACH power even for an ‘initial’ PRACH transmission, which is not aligned with the existing PRACH ramping operation. Technically, it is also problematic as in many cases, there is no need of power ramping for initial PRACH transmission. Transmission power of initial PRACH purely determined based on the measured PL and target receiving power is sufficient. The power ramping was introduced to deal with dynamic interference e.g., cross-cell. In addition, Alt.1 results in always Pc.max transmission once UE hits the maximum transmission power as there is no resetting. Alt.4 also has clear drawbacks. For instance, power ramping is not supported and consequently the UL sync latency becomes even worse than legacy in some cases. |
| QC | Prefer Alt3. For Alt2, if it is a binary indicator, it may not work well if UE missed the 1st PDCCH order scheduling the initial PRACH Tx, since UE has no clue on the initial Tx power for further ramping |
| Futurewei | Support Alt1. Although retransmission is triggered by PDCCH order, the UE retransmission power ramping rules and procedure can still be like the existing MAC RACH procedure. This can reduce the standards impact. Anyway, regardless PDCCH ordered or UE autonomous preamble reTX, a larger RACH/TA response window than legacy RACH window should be specified to consider backhaul delay and support the retransmissions. |
| OPPO | Prefer Alt3 Alt1,2 or 4 might not work since there is no association between previous PDCCH order DCI and current PDCCH order DCI. The UE might miss some DCI, even through the chance is not large. Therefore, it is only feasible for the UE to determine the Tx power based on the current DCI and the latest measurement of associated SSB |
| CMCC | Alt 2. Is slightly preferred. Alt 1 cannot distinguish the situation of PRACH retransmission due to a poor channel quality or the TA value is outdated. If it is an outdated TA case, there is no need to increase the power. If the case is that the PRACH received by the candidate cell is too poor to derive the TA values, then a power ramping up is needed. Alt 2 and Alt 3 both can work for this purpose. If it is a initial transmission of PRACH, the power ramping up can be set as zero which means no power ramping up. And if it is a retransmission of PRACH for TA, the bit field in DCI could be one which means a power ramping up is required. From our understanding, Alt 3 should not be indicating the exact transmit power for the Ues. Instead, it should be a TPC command with increasing or not increasing the power. The initial transmit power of PRACH should follow the legacy rules that it is decided by the target power and PL. |
| Nokia | We prefer an implicit way for power ramping using PDCCH order which is Alt 1, but with an associated timer, i.e., if the UE receives a PDCCH order for the same candidate cell within a ‘X’ ms then the UE may be configured to increase the power by ‘Y’ value, where X and Y can be configured to the UE. This will mitigate the need to any additional dynamic signaling via PDCCH order. Also, note that we have agreed to use the reserved bits (10 bits) to indicate the candidate cell ID in the PDCCH order; therefore, an additional indication (as in Alt 2/3) will need to be fit in to that. |
| Lenovo | Support Alt1 since UE knows whether a PRACH transmission is retransmission or new transmission which is triggered by a PDCCH order, therefore, the UE retransmission power ramping rules and procedure can still be like the existing MAC RACH procedure where the spec impact will be minimal. |
| ZTE | Both Alt 1 and Alt 2 are fine to us, and we prefer Alt 2 slightly. For Alt 1, as agreed before that TA update can be triggered by PDCCH order as well, hence UE might could not identify the received PDCCH order is for initial TA acquisition or TA update. To avoid such ambiguity, it should be specified that UE transmits PRACH with power ramping for subsequent PRACH triggered by PDCCH order indicating the same SSB index for the same candidate cell. In addition, PRACH transmission power transmitted by UE and expected by gNB might be different due to missing of the PDCCH order. For example, network transmits 3 PDCCH order for a candidate cell, and UE misses the second PDCCH order, power ramping value for transmission power of the third PRACH will be less than what expected by network. For Alt 2, PDCCH order missing issue can be realized by UE based on the indication fields in PDCCH order. |
| Xiaomi | We are OK with both Alt.2 and Alt.3.And we slightly prefer Alt.3, in which, from our understanding, a power offset/adjustment is indicated to UE. |
| Samsung | We also think alt 2 and 3 can be merged as PDCCH based power indication, and prefers these two alts |
| ITRI | Both Alt. 2 and Alt.3 are fine for us. And we slightly prefer Alt.2. |
| MTK | Support Alt2. In LTM, UE needs to re-transmit PRACH not because the uplink power is too small to be received by gNB, but because the TA monitoring toward candidate cell must be kept until the reception of the cell switch command. Therefore, we should allow the Network to control whether UE needs to do the power ramping. Re comment from QC and OPPO, we don’t see why it is an issue for Atl2 if UE misses the 1st PDCCH oder. For Alt2, if UE misses the 1st PDCCH order scheduling the initial PRACH Tx, UE can still send the PRACH based on the initial Tx power when it receives the 2nd PDCCH order. For initial transmission from UE’s perspective, UE may ignore the field for power ramping indication. The concept is that the UE gradually increases its Tx power until the network can receive, and we think it is a reasonable way. On the other hand, DCI missing may casue issue for Alt3 since UE may use larger Tx power (indicated by NW) for trigged PRACH transmission even UE I perform initial PRACH transmission before. If UE misses the 1st PDCCH order scheduling the initial PRACH Tx, and gNB indicates 2nd PDCCH order with larger fixed Tx power. It means that that gNB misunderstands that the first Tx power is too small for that UE, but it is actually caused by UE not receiving the 1st PDCCH order at all. |
| Spreadtrum | Since UE cannot distinguish initial PRACH or subsequent PRACH, PDCCH order can indicate this new information. So Alt 2 is preferred. |
| NTT DOCOMO | Prefer Alt.2 or Alt.3. Regarding Alt.2 and Alt.3, our view is the same with Apple. Which is better would depend on whether next PRACH for other candidate cells is sent before one RACH procedure completes. If next PRACH for other candidate cells is not sent before one RACH procedure completes, both alternatives are good. In terms of the number of DCI bits used (i.e., Alt.2: 1 bit, Alt3: 2 bits), a little Alt2 may be better. While if next PRACH for other candidate cells is sent before one RACH procedure completes, Alt.3 would be better because managing power with one bit for multiple PRACHs can be complex. For Alt.1, if next PRACH for other candidate cells is not sent before one RACH procedure completes, it may be good. In this case, it is also necessary to narrow down the target PDCCH order. If only the same candidate cell is used for condition, there is a possibility of an initial transmission using a different beam. Therefore, it is better to restrict the case where SS/PBCH index is the same. Also, the UE should reset the power counter when PDCCH order with different conditions is received. Therefore, we propose the following update. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured, on how to determine the transmit power of subsequent PRACH triggered by PDCCH order, discuss and down-select among the following alternatives: - Alt 1: UE transmits PRACH with power ramping for subsequent PRACH triggered by PDCCH order for the same candidate cell and the same SS/PBCH index. UE resets the power counter when PDCCH order with different conditions is received. For Alt.4, we are not sure whether it is useful for subsequent PRACH. |
| Huawei, HiSilicon | The Alt 2 need to be clarified as the current version is quite similar as Alt 3. To our understanding, PDCCH order should indicate whether the current PRACH is the retransmission or initial retransmission. Based on such indication, UE can follow the existing power ramping mechanism. So we propose to adjust the Alt 2 as following “Alt 2a: Whether PRACH is a retransmission is indicated in PDCCH order” Explicit indication of (re)transmission in DCI is preferred as it cost least standard effort and there is sufficient reserved bit in DCI format |
| IDCC | We perefer Alt 1 or Alt 2. For Alt 1, the UE can determine if the PRACH transmission is an initial transmission or a retransmission (for eample using a method similar to Nokia proposed). For Alt 2, an explicit trigger can be used in the PDCCH order (details FFS). |

Round 2

| Mod | As shown in round-1 discussion, positions of companies can be summarized as follows: - Alt.1(5): Futurewei, Nokia, Lenovo, ZTE(2nd preference), IDCC - Alt.2(10): ZTE(1st preference), Spreadtrum, IDCC, Apple(1st preference), CMCC, Xiaomi(2nd preference), ITRI(1st preference), MTK, DCM, HW - Alt.3(6): Apple(2nd preference), QC, OPPO, Xiaomi(1st preference), ITRI(2nd preference), DCM - Merging Alt.2 & 3(2): Ericsson, Samsung, [Apple] - Alt.4: none Apparently, majority companies are supportive to Alt.2 or 3, and among the proponents of Alt.1, ZTE and IDCC are also agreeable to Alt.2 actually. Given the situation and considering the limited time slots we can have in this release, I suggest we focus on Alt.2 and 3 in this round of discussion. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured, on how to determine the transmit power of subsequent PRACH triggered by PDCCH order, discuss and down-select among the following alternatives: - Alt 2: Whether power ramping is performed or not is indicated in PDCCH order - If power ramping is performed, whether PRACH is an initial transmission or retransmission is [explicitly] indicated in PDCCH order - Alt 3: Tx power is indicated in PDCCH order For proponents to Alt2, please check whether current update is acceptable to you or not. @Ericsson @Samsung @Apple: regarding whether Alt.2 and 3 can be merged, seems different companies still have different view. |
| --- | --- |
| OPPO | We are also fine to merge Alt2 and Alt3. One solution to merge can be: - The PDCCH order DCI indicates whether power ramping is performed or not and also the power ramping values. |
| QC | Fine with FL’s proposal, and also fine with merging Alt2 and 3. To our understanding, the two options are tradeoff between # of bits and power adjustment accuracy. For example, if 1st and 2nd PDCCH orders are missed but 3rd is received, Alt2 will boost for X dB, while Alt3 will boost for the indicated amount, which will be greater than X dB, since gNB knows the retransmission #. To MTK: we do think Tx power should be boosted for every missed DL, since it is hard to identify the reason. This is same as legacy, i.e. boost for every missed RAR. |
| NTT DOCOMO | Support. We slightly prefer Alt2. And we think Alt3 means TX power ramping value can be indicated instead of absolute TX power. Thus, we suggest following revision on Alt3. Alt 3: Tx power ramping value is indicated in PDCCH order |
| Futurewei | Based on the text reading of Alt 2, it only specified the source cell instruct to the UE whether the RACH order is an initial TX or a reTX. It is not a complete solution to including how reTX ramping power is determined. We also support Alt2 and think almost all companies fine with it. The difference is how the ramping power to be determined. It appears to us there are two options: - Alt2 + 3: if the RACH order indicates a preamble reTX, XdB of power up step is also indicated in the PDCCH order. (note: considering UE open loop PC for the candidate cell is also impact final UE TX power, I presume the “TX power” in current Alt 3 text meat X dB power step and we are not requesting the source cell to mimic open loop PC to determine the UE TX power). - Alt2 + 1: if the RACH order indicates a preamble reTX, the UE performs the power ramping based on existing RACH reTX procedure with configured power up step. We think both above potion 1) and 2) are doable. We still slightly prefer the option of Alt2 +1, since we can save some bits in DCI and minimize the standards efforts at both UE and network. |
| ZTE | For Alt 3, we are not sure if Tx Power is a cumulative value, or an absolute value. if it is former, what is specific value. If it is latter, it is not clear if there are enough bits to indicate TX power value in DCI since reserved bits are only 10 bits in the existing DCI for PDCCH order based PRACH. |
| Samsung | We are basically O.K. with FL’s new proposal. But as small modification, indication of exact tx power should not be practical, PDCCH may deliver ‘ramping up’ indication or ‘ramping-up value’. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured, on how to determine the transmit power of subsequent PRACH triggered by PDCCH order, discuss and down-select among the following alternatives: - Alt 2: Whether power ramping is performed or not is indicated in PDCCH order - If power ramping is performed, whether PRACH is an initial transmission or retransmission is [explicitly] indicated in PDCCH order - Alt 3: Tx power ramping-up value is indicated in PDCCH order |
| Xiaomi | Support the new proposal 1-2. And between alt2 and alt3, we support Alt.3. First, for Alt.3, UE does not have to know the current triggered PRACH is a subsequent PRACH or not and does not need to know the number of “retransmissions” for current triggered PRACH. Only a power offset/adjustment needs to be indicated to decide the transmit power for the subsequent PRACH triggered by PDCCH order at t2 and t3 in the figure 1. Fig.1 In addition, for alt3, the transmit power of current PRACH, like the RACH at t2 in figure 2, can be adjusted based on the received power of last RACH, that is the RACH at t1, even there is no failed RACH. Fig.2 |
| CMCC | Thanks FL’s updates. We are fine with current version of proposal. One question should be clarified, as we mentioned in the last round, what is exactly the Tx power means in the Alt 3 ? TPC command or the real transmit power of UE’s ? Currently we have the power control procedure for legacy PRACH transmission, the transmit power of the PRACH could base on that. And we usually do not indicated a exactly transmit power value to the UE. Then the wording of the Alt 3 is ambiguous. if Alt 3 means the step of power ramping up, then we think the power ramp up steps of current serving cell can be used. |
| Spreadtrum | From our understanding, Alt 2 means to use 1 bit to indicate initial Tx or re-Tx, UE would calculate the power according to the counter. Alt 3 means to include a power value in PDCCH order, However, we don’t understand how to indicate this, a value to replace step *(counter -1)? |
| FGI | We support Alt 2. We share a similar view to CMCC, i.e., the initial transmit power of PRACH is better to follow the legacy rules that it is decided by the target power and PL. In addition, we don’t think merge Alt2 and Alt3 is necessary since they are different. |
| Huawei, HiSilicon | We are fine with FL’s proposal to have two alternatives for further down selection. Our preference is Alt 2 and suggest following update if acceptable to the group - Alt 2: Whether power ramping is performed or not is determined from the PDCCH order - If power ramping is performed, whether PRACH is an initial transmission or retransmission is [explicitly] indicated in PDCCH order |
| vivo | Although our preference is Alt.1, we can accept majority views and accept the proposal. Compared to explicitly indicating Tx power offset or absolute Tx power in PDCCH order for PRACH re-transmission, we prefer the power ramping to be executed based on the existing RACH reTX procedure with configured power-up step, i.e., Alt.2, which has less specification impact. |
| Nokia | Our preference is Alt1, but based on the majority view, we support Alt2 between Alt2 and Alt3. With Alt2, the legacy power ramping mechanism should be followed. |
| IDCC | We are fine to consider with Alt 2 or Alt 3. We prefer Alt 2. Our understanding is that the details are FFS. For example, we think for Alt 2, the PDCCH order may indicate a transmission index (e.g., 0 for initial transmission, 1 for the retransmisison, etc.) and the power ramping step has been preconfigured. |
| ITRI | We support Alt.2. One bit in the PDCCH order to indicate whether the PRACH is re-transmission or not, then UE can determine the transmission power with a pre-configured power ramping step. |
| Mod | Thanks for the continuous discussion. @CMCC: regarding your question, seems majority companies believe that power ramping-up value is indicated in Alt-3. @Spreadtrum: as shown in revised Alt-3, Tx power ramping-up value is indicated in PDCCH order. @Nokia: thank you for your flexibility. Based on comments from companies, three alternatives on the table are summarized as follows. - Alt. I (revision of Alt-2): Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping is performed with configured power ramping-up value - Alt. II (revision of Alt-3): Tx power ramping-up value is indicated in PDCCH order - Alt. III (merged Alt-2 and 3): The PDCCH order DCI indicates whether power ramping is performed or not and also the power ramping-up values. After listing all the three alternatives together, high similarity among them can be observed. It seems that the major difference is whether the ramping-up values is configured or indicated. So, as shown in the following revision of P1-2, to make progress and move forward, my suggestion is to further merge these alternatives. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping is performed with configured or indicated power ramping-up value |
| Google | We can support current FL proposal for further down-selection. But one clarification question, why do we need explicit indication of initial transmission or retransmission? If the RO and RA preamble indicated in PDCCH order is the same as last one, doesn’t it already mean retransmission? |
| MTK | We also support current FL proposal, although we do not think it is necessary to explicitly indicate initial transmission or retransmission. |
| Ericsson | We think that the latest FL proposal is a good step forward. Our thinking was that the PDCCH order would include a single-bit indication if the Tx power should be increased (with a configurable step, likely a new parameter), or reset. If this is the intention of the latest FL proposal, the proposal could be shortened: Proposal 1-2A: For PDCCH ordered-RACH, if reception of RAR is not configured - The PDCCH order explicitly indicates if the power should be - increased with a configured or indicated value, or - reset to its starting value, determined by the open-loop power control |
| QC | Fine with latest FL proposal 1-2 |
| Mod | @Google @MTK: regarding the clarification question from Google, as it’s possible for UE to miss the first PDCCH order, implicit identification of initial or retransmission by comparing RO and RA preamble with previous PDCCH order may not work. @Ericsson: thank you for the simplification of this proposal. To me, they are the same. However, to be safe, I prefer to keep the latest FL proposal for now. I hope it’s also acceptable to you. |

Round 3

| Company | Input |
| --- | --- |
| Mod | In our second GTW session on Wednesday, one company proposed to use absolute power instead of ramping-up value. So, as shown below, this alternative is also included for further discussion. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1 - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured or indicated - Alt. 2 - Absolute power is indicated - else, the power should be reset to its starting value, determined by open-loop power control |
| Xiaomi | First, between alt.1 and alt.2, we support Alt.1. Secondly, for the solution in Proposal 1-2, UE has to remember the transmission power of last retransmitted RACH or UE needs to count how many times the RACH is retransmitted. And whether UE have to remember the transmission power of last retransmitted RACH, or UE needs to count how many times the RACH is retransmitted should be specified to make the solution more complete. Proposal 1-2A: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1 - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured or indicated - Note: UE needs to remember the transmission power of last retransmitted RACH - Alt. 2 - Absolute power is indicated - else, the power should be reset to its starting value, determined by open-loop power control Or, Proposal 1-2B: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1 - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured or indicated - Note: UE needs to count how many times the RACH is retransmitted - Alt. 2 - Absolute power is indicated - else, the power should be reset to its starting value, determined by open-loop power control In addition, as we mentioned in the GTW, there is another solution that a power offset/adjustment is indicated in PDCCH order. From our understanding, it is more appropriate than the solution in Proposal 1-2. If current triggered RACH is a retransmission, an appropriate power offset/adjustment decided by NW can be indicated to UE. And even current triggered RACH is not a retransmission, a power offset/adjustment decided by NW based on the received power of previous RACH can also be indicated to UE. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Option 1: Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping is performed with configured or indicated power ramping-up value - Note: UE needs to remember the transmission power of last retransmitted RACH (or, UE needs to count how many times the RACH is retransmitted) - Option 2: A power offset/adjustment value is indicated in PDCCH order Well, to make a progress, Option 2 can be excluded is most companies don’t support it. But at least we should make down selection between proposal 1-2A and proposal 1-2B to make the solution more complete. We are fine with both proposal 1-2A and proposal 1-2B, and slightly prefer proposal 1-2B. |
| Lenovo | We are OK with the two alternatives. |
| ZTE | For the latest proposal 1-2, we have some comments as follows: Comment #1: for “else, the power should be reset to its starting value, determined by open-loop power control”, we don’t understand why transmission power should be reset to initial value depends on “open-loop power control”. Comment #2: for Alt.1. it is not clear to us on “power ramping-up value” is indicated. In our understanding, If PDCCH order indicates transmission power needs to be increased, then UE will determine transmission power based on ramping-up value, where this value is directly configured by RRC. With this consideration, we would like to know how to understand the case that ramping-up value is indicated. Comment #3: regarding one issue raised by Xiaomi, from our point of view, we think that UE itself maintain the transmission power of last retransmitted RACH or count the times of re-transmission PRACH can be left to UE implementation. Comment #4: Compared with “absolute power value”, we prefer to carry an indication on whether transmission power needs to be increased in PDCCH order, mainly considering the limited available bits in DCI. |
| Ericsson | We support P1.2, with Alt1. We are not sure if the meaning of Alt2 is to really set the absolute Tx power? In dBm? There is no such power control in NR today, right? How would the NW know what absolute Tx power to set? The final row in the proposal 1-2 is good for clarity. @Xiaomi: How the UE implements the power ramping (counting the number of transmissions or remembering the latest Tx power) would be up to UE implementation |
| Mod | @Xiaomi: thanks for the detailed elaboration of options/alternatives and the comments/suggestions. Regarding whether UE needs to remember the number of transmission/power of previous transmission, I hope the explanation from ZTE and Ericsson can convince you. @ZTE: Regarding comment 1, the operation described in “else…” means ramping is not performed. Regarding comment 2, I agree with you that approach of configuring the value is more aligned with current mechanism. However, as also mentioned by some other companies, it’s also possible to indicate the value. The two options are tradeoff between # of bits and power adjustment accuracy. |
| Futurewei | We do not support adding Alt 2 in Proposal 1-2. UE preamble TX power is determined by UE open loop power control w.r.p.t. candidate cell. We don’t think the source cell can determine the absolute power for the UE. What the source cell can do is only provide the delta power up step and the UE used its own determined power plus the delta step for its preamble TX. For network configured power step, the UE accumulates them with number of retransmissions and apply the accumulated steps on top or the UE currently determined TX power. For PDCCH order carried power value, the source cell has the flexibility to accumulate the steps and change the step size. Both options are feasible, for saving DCI bits and reusing existing procedure, we slightly prefer network configured power ramping value. |
| OPPO | We support Alt2. The Alt1 has problem sue to DCI mis detection because Alt1 needs to build associate between PDCCH order DCI. If UE miss one DCI detection, error propagation would happen. That is not acceptable to us. |
| Nokia | Ae don’t support Alt2. We can support Alt1, but we have a couple of questions to clarify. - What do we mean by “power ramping-up value is configured or indicated”, do we support both options - RRC configured and indicated in the PDCCH order? Which option will be selected in which scenario? We prefer to down select only one option for simplicity, i.e., power ramping-up value is configured. - When the PDCCH order contains an indication of “retransmission”, how the UE will know for which previous transmission this retransmission refers to? Which of the PDCCH order field will be compared to make that decision: - Candidate cell is the same - Both candidate cell and the SSB are the same - Candidate cell, SSB, and RO are the same |
| QC | Fine for FL’s Alt1 as compromise. |
| NTT DOCOMO | We do not support Alt2 as it cannot work. NW never knows the absolute power used by UE, which should be decided by UE open loop power control. We support Alt1. And in our understanding, the wording ‘power ramping is performed with configured or indicated power ramping-up value’ has similar meaning as ‘a power offset/adjustment value is indicated in PDCCH order’, which was proposed by Xiaomi. |
| Apple | Support P1-2 in generical. On ‘configured or indicated’, our understanding is that RAN1 will further down-select one of them in the next meeting. However, current formulation maybe interpreted as support both. To avoid this confusion, suggest adding the following sub-bullet: - Down selection of one of ‘configured or indicated’ in RAN1 113 meeting. In legacy power control of PRACH, a power ramping step is configured by higher-layer signaling, we do not see motivation to introduced new power control framework as it complicates the NW implementation at least to implement new power control scheme for PRACH. In addition, with configured power ramping step size, 1-bit field in PDCCH order is sufficient to indicate whether it is a retransmitted PRACH. While it seems challenging for ‘indicated’ Option. Anyway, we can have more discussions in next meeting. On Xiaomi’s comment, as captured in TS 38.213, the current dynamic power control for PRACH is sort of delta signaling as the transmission power for a given time instance ‘i’ is always determined on top of the previous accumulated power ‘i-1’. In other words, even in legacy, UE always accumulates power for each transmission instance for the later transmission. The difference between Alt.1 and Alt.2, in our opinion, is whether allows NW to control the transmission power of initial PRACH transmission. For example, NW triggers the PRACH transmission as ‘initial, retransmission’ after time gap, and then ‘initial PRACH’. For the second initial PRACH before cell switching, it is unnecessary to increase the power on top of the previous retransmitted power, which is same as the current CFRA procedure, i.e., for each new PDCCH-order CFRA, the power for the initial transmission is reset. |
| FGI | We are fine with FL’s proposal and prefer Alt.1. In addition, since power ramping-up value is configured by higher-layer singalling in legacy, we prefer to use the configured power ramping-up value. However, we share the same view as Apple, i.e., we can down-select configured or indicated power ramping-up value in RAN1#113 meeting. |
| Xiaomi | @ ZTE @Ericsson @Apple @FL, Thanks, every one, for your kindly response. Our intention is to make the method of determining the Tx power of PDCCH order RACH more clear. And if from most companies’ understanding, it can be up to UE implementation, then we can live with it. @ NTT DOCOMO, The wording ‘power ramping is performed with configured or indicated power ramping-up value’ and the wording ‘a power offset/adjustment value is indicated in PDCCH order’ are different. They are totally different solutions. First, power ramping is added to the transmission power when the RACH is a retransmission. While, a power offset/adjustment value in PDCCH order will be used to determine the Tx power as formula (1) no matter the RACH is retransmission or not. PPRACHi=min⁡{Pcmaxi, PPRACH,target+PL+∆} (1) From our understanding, it is simpler than the solution in proposal 1-2. UE does not have to know whether the current RACH is retransmission or not. And UE does not need to store the transmission power of last retransmitted RACH or count how many times the RACH is retransmitted. An appropriate power offset/adjustment value decided by NW is used to adjust the Tx power as in formula (1). Therefore, we do hope you guys can think about the option 2 in the proposal 1-2 below. Please do not exclude it right away. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Option 1: Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1 - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured or indicated - Alt. 2 - Absolute power is indicated - else, the power should be reset to its starting value, determined by open-loop power control - Option 2: A power offset/adjustment value is indicated in PDCCH order |
| Samsung | We are not O.K. to include Alt2, since it is totally new concept of power control and we don’t see a reason to introduce it. In legacy operation, network does not need to know absolute or exact tx power of UE and UE always remember the latest transmission power at least for PRACH. When cumulative TPC is configured, UE remember latest transmission power even for PUSCH/SRS/PUCCH separately. |
| Huawei, HiSilicon | We do not support to add alt 2 in proposal 1-2. The bit for power control need to share 10 reserved bits DCI 1-0 with candidate cell ID of RO and absolute value of transmit power cost large DCI overhead. As for the power ramping value, we preferred it to be configured together with RO configuration as the legacy behavior, e.g. powerRampingStep in RACH-ConfigGeneric. We do not think it is necessary to dynamically change the ramping step in DCI. If the option of “indicated” is kept in 2nd sub-sub-bullet, it contradicts with the 1st sub-sub bullet. As for remembering previous transmit power, I think it is a regular behavior for close loop power control as the new power is increased/decreased from the previous value. As for using implicit method to differentiate initial or re transmission of PRACH, we think it may not be efficient. As we are using CFRA, the available preamble or RO configured for a UE is very limited. If a gNB just hope to update previous TA, instead of, it must use a different preamble or RO which increase requirement for CFRA resource. As for the power resetting behavior, it can be achieved by indicating initial transmission. then open loop power control value can be used. No need to define specific procedure for that. Thus, we would suggest the proposal update as follows Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1 - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured together with other RACH parameters for a candidate cell or indicated |
| Mod | @Futurewei @Nokia @Apple: Regarding how ramping-up value is determined, the intention of this proposal is to include all the options for discussion. I hope down-selection can be made after our discussion in this meeting or the next meeting. @Xiaomi: thanks for your comments. From my understanding, option 2 proposed by Xiaomi in included in Alt.1. Also, considering the limited time for discussion and we already have too many things on the table, let’s focus our discussion and try to reach convergence on this issue. @HW: thanks for the comments and suggestions. Regarding the power resetting behavior, I think the option proposed by you is one of potential options. So, my suggestion is to keep this part more general, next level details can still be discussed after we agree on this proposal. Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - Alt. 1: Xiaomi, Futurewei, Ericsson, FGI, Samsung, HW, QC, CMCC - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order - power ramping-up value is configured or indicated, - note: down-select one option later in this meeting or RAN1 #113 meeting - Alt. 2: OPPO - Absolute power is indicated - else, the power should be reset to its starting value, determined by open-loop power control |
| CMCC3 | Thanks for discussion. We are fine with current formula of the proposal 1-2. As commented by many companies and in our last round, we do not support alt 2 for the absolute value. Within the Alt 1, power ramping up value should be configured as legacy. We do not understand the reason of indicating it in DCI. We may not need this kind of flexibility. Also commented by other companies that, if the transmission is indicated as a retransmission as the 1st bullet in the Alt1, the power ramping up can work. If it is a initial transmission, the power ramping up is not needed and the power could be determined according to the open loop power control. |
| QC | We prefer Alt1 with indicated power. The benefit is that gNB knows how many retrans have happened, and hence can indicate the power accordingly. Letting UE to ramp up the power PRACH by PRACH not only wastes DCI/PRACH resource but also increases interruption for serving cell. We don’t see the need for indicating initial or retrans but can leave with that as compromise |

P1-3: closed

Round 1

Proposal 1-3: When reception of RAR is configured, support RAR is received from serving cell

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Support |
| Apple | Fine for progress. For intra-DU case, it should be ok. For inter-DU case, this may result in large latency caused by RAR forwarding across different DU, even involving CU. |
| QC | Support |
| Futurewei | Not support. True options we have are either the target TA is sent to the UE from the serving cell or from the candidate cell. If the target TA is from the serving cell, we have all the options under discussion: TA via cell switch command –> non autonomous reTX, TA via “RAR” or a “new MAC CE” UE autonomous reTX … . Therefore, without-RAR and with-RAR from the serving cell are essentially the same thing. If target TA is from the candidate cell, we have RAR from candidate cell to carry TA following legacy RACH procedure for CFRA and preamble reTX with conventional RACH response window. We support that if RAR is configured, RAR is only from the candidate cell. Since RACH configuration is per cell, network implementation can ensure with-RAR is configured to only inter DU candidate cells. For MIMO intra DU candidates, they can be configured as without-RAR. The use case for RAR from inter-DU-candidate(s) should be for more delay sensitive use cases where cell switch command can be issued any time after early RACH is ordered and backhaul delay can be avoided. Especially, in DC enabled scenarios, RACH on candidate PSCell or Scell does not have interruption on both DL and UL of the serving cell. |
| OPPO | Support |
| CMCC | Support |
| Nokia | Support |
| Lenovo | Fine with the proposal. |
| ZTE | Support in principle. As analyzed in our contribution, we hold the view that RA response window related issue should be further considered. |
| Vivo | Support |
| Xiaomi | Fine. But we believe it has great influence on RAN2. Should this be decided by RAN2? |
| Samsung | Support. Or at least at UE’s perspective, it should be received without defining target cell specific CSS. |
| ITRI | Support |
| MTK | Support |
| Spreadtrum | Support. |
| NTT DOCOMO | Support. And we can further clarify whether the serving cell for RAR reception is SpCell. |
| Huawei, HiSilicon | We hope the proposal should clarify the applicable scenario. Suggest following updates “When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case” For the inter DU case, we share the similar view as FW and Apple, the latency in higher layer signaling exchange might cause impact of RAR reception if it is from the serving cell. Further study should be considered. |
| IDCC | Support |
| LGE | Support. |
| Mod | If the concern on receiving RAR from serving cell is latency in inter-DU case, can we consider to agree on intra-DU first? Proposal 1-3: When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. In inter-DU case, the following alternatives are listed for discussion and down-selection: - Alt 1: RAR is received from serving cell - Alt 2: RAR is received from candidate cell |

Agreement

When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case.

Round 2

| Company | Input |
| --- | --- |
| Mod | Regarding the alternative of RAR reception from serving cell, I can fully understand the concern from the opponents on latency issue in inter-DU case. Also, as mentioned by Futurewei, in some cases, receiving RAR from candidate cell is beneficial to the system from interruption avoidance perspective. However, in my opinion, a unified solution for both inter- and intra-DU cases is more desirable, especially considering the possible impact to spec and the limited time for discussion. For example, if we intend to introduce an additional RAR reception mechanism for inter-DU case, we may need separate discussions on the contents of RAR (i.e. the issue of P1-4) for both inter and intra-DU cases. In addition, the spec impact of introducing RAR reception from candidate cell needs to be considered as well. So, it would be preferable if we can focus on one unified solution and start this round of discussion with the following proposal. Proposal 1-3-1: When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues |
| OPPO | Support the proposal 1-3-1. The FFS sub-bullet can be good starting point for resolving the latency issue in inter-DU case. |
| QC | Support FL’s proposal for unified solution |
| Lenovo | Support the proposal for unified solution. |
| NTT DOCOMO | Support. |
| Futurewei | We respect the majority view and decision. In last online meeting I intend to share two points: - If RAR is from the serving cell, I am not sure what benefit we can get from the with-RAR option comparing with the without-RAR option. If no additional benefit, we do not need to make effort on the with-RAR option. - We also do not like to have different handling for intra-DU and inter-DU scenarios for with-RAR option. This can be resolved by requiring that with-RAR option is only configured to a specific scenario e.g., inter-DU scenarios. We are OK with proposal 1-3-1 if majority companies want to go this way. We are also OK for pending the option of with-RAR. |
| ZTE | support |
| Samsung | Support |
| Xiaomi | Fine |
| CMCC | Fine with the proposal. Unified design is preferred. For the FFS part, does it mean that gNB would configure different RAR window length according to whether the cell switch happens within a DU or inter DUs ? |
| Spreadtrum | Support |
| FGI | Support the proposal 1-3-1. |
| LGE | Support the proposal 1-3-1. |
| Huawei, HiSilicon | We can live with the proposal. |
| ZTE2 | Share the information from RAN2: R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. Response to CMCC: from our point of view, on whether to need configuring different RAR window for inter- and intra UE will depend to the delay required by information coordination among source DU, target DU and CU in inter-DU case. For example, if the required delay is relatively large, it may be necessary to consider some RAR window related enhancements in order to avoid not being able to receive RAR within the existing RAR window. Otherwise, it may not be necessary. If we don’t expect to have different RAR window for intra-and inter-DU, an potential method is to adjust the timing of starting point of RAR window to resolve the issue raised by delay. |
| vivo | Support the proposal 1-3-1. |
| Nokia | Support the unified solution for both intra and inter-DU scenarios. |
| IDCC | Fine with the proposal. |
| ITRI | Support |
| Mod | Thanks for the constructive discussion. @Futurewei: thanks for sharing your understanding. Also, your flexibility is appreciated. @CMCC: to my understanding, that’s one of the possible solutions. @ZTE: thank you so much for the information about agreement from RAN2. |
| Google | We agree with FL to have a unified solution and support FL proposal. For the FFS part, it should also involve when to start the RAR window? |
| MTK | Support proposal 1-3-1 |
| Ericsson | Support proposal 1-3-1. We also noted the agreement from RAN2, and propose to agree on 1-3-1 also for inter-DU. |

Agreement

When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

P1-4

Round 1

Proposal 1-4: When reception of RAR is configured, on whether other parameters such as UE ID, candidate cell ID is contained in RAR, discuss and down-select among the following alternatives:

- Alt1: additional parameters are not needed

- Alt2: candidate cell ID/UE ID is contained in RAR

- Alt3: others. E.g a new designed MAC CE

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Additional parameters are needed. Remember that RA-RNTI/RAPID is only unique within a cell. |
| Apple | Based on the agreed functions for LTM, we do not see the need of UE ID. On candidate cell ID, it is indeed a valid point that RAPID is cell-specific. First, to avoid RAR confusion, it seems dedicated CFRA PRACH resource needs to be reserved for both serving cell and candidate cell. Otherwise, if the CFRA PRACH is also used by a legacy UE in serving cell, ambiguity of RAR is inevitable even candidate cell ID is provided in RAR payload. The reason is that legacy UE cannot comprehend this new IE and then continue transmits Msg3 even this RAR intends to address LTM UE for the target cell PRACH. Second, there are different options to convey the candidate cell ID, e.g., DCI with RA-RNTI or RAR payload. For any of these solutions, the first issue is not solved and need to clarify first. |
| QC | Support Alt3, which does not need to restrict to CSS Type 1 and does not need Msg3. DCI is scrambled by C-RNTI, so no need UE ID as well. Candidate cell ID is also not needed if there is only one ongoing RACH procedure |
| Futurewei | If RAR is from the inter-DU candidate cell, legacy RAR can be used without any additional change. |
| OPPO | Candidate cell ID is needed but UE ID is not. As in proposal 1-3, the RAR is received from serving cell, preamble ID and candidate cell ID are enough. Do not see the need for UE ID. Support revise Alt2: candidate cell ID/UE ID is contained in RAR. |
| CMCC | We do not see the need for the UE id indication. If TA is required for only one candidate cell for each time, there is also no need to indicate candidate cell. If the UE has sent multiple PRACH to multiple candidate cell, the candidate cell id or configuration id of the candidate cells are needed within the RAR. |
| Nokia | We suggest delaying the discussion on this proposal until we decide on the proposal 1-3. We support RAR via serving cell. If that is the scenario we refer here, then we should first discuss if it is received by type-1 CSS or USS (or new MAC-CE) on the serving cell. If RAR from the serving cell is received via type-1 CSS, then we need candidate cell ID in the MAC payload as there can be other Ues in the same serving cell which might have triggered the PDCCH order for another cell (another candidate cell or serving cell) using the same RA-RNTI and RAPID (as E/// and Apple also mentioned, these are cell-specific parameter). If RAR from the serving cell is received via USS (or new MAC-CE) then we don’t need any candidate cell ID. We don’t need UE ID in any of the options. |
| ZTE | Support Alt 1. Regarding the UE ID (i.e., C-RNTI), we prefer to include it in cell switch command to indicate UE to update the C-RNTI. |
| Vivo | First of all, P1-4 should be discussed with the combination of P1-3. If the proposal 1-3 is agreed, that the TA value of candidate cell(s) is received from the serving cell, whether the TA value of candidate cell(s) is carried by RAR or new MAC CE needs further discussion. According to the current specification, UE is not required to simultaneously monitor PDCCHs addressed to RA-RNTI for RAR and C-RNTI for data transmission. Hence, if TA value for candidate cell(s) is carried by RAR, when UE monitors the RAR from the serving cell, the normal data transmission in the serving cell will be interrupted. To reduce data interruption, a new MAC CE should be designed to carry the TA value for candidate cell (i.e., Alt3). As for the contents of the new MAC CE, if there is only one ongoing RACH procedure, additional parameters are not needed except the TA value for the candidate cell. |
| Xiaomi | Agree with Nokia. |
| Samsung | We slightly supports Alt 1. OPPO’s modified Alt 2 is our 2nd priority. |
| ITRI | Share same view with ZTE. |
| MTK | Support Cell ID (explicit/implicit indication) in Alt2. If there is no cell ID information, UE can’t distinguish from which cell the current RAR is sent. We would like to modify our proposal, the C-RNTI should be given in the cell switch command. |
| Spreadtrum | We support Alt 1. Since UE only has one RACH procedure at one time, so candidate cell ID does not need. For UE ID, we do not see any need. |
| NTT DOCOMO | OK to further consider Alt2 or Alt3. We think a relevant issue that should be discussed is whether PRACH to another candidate cell can be sent before RAR reception of PRACH to a prior candidate cell. |
| Huawei, HiSilicon | This issue should be deferred after determination of from where the RAR is received and PDCCH is in which search space. If the RAR is from serving cell and share same type 1 CSS in the serving cell, the cell ID might be needed. However, if it is from candidate cell or separate SS in serving cell, the cell ID is redundant. Considering legacy CBRA is not supported in last meeting, the UE ID seems not necessary no matter whether RAR is scheduled by PDCCH scrambled with RA-RNTI as regular RACH procedure or C-RNTI as CFRA for BFR. |
| IDCC | Ok with the alternatives. |
| LGE | OK to further discuss on Alt 2 and/or Alt 3. As some companies mentioned, this proposal is quite related with the consequence of P1-3, we are also OK to discuss this proposal after P1-3 is agreed. |
| Mod | As mentioned by some companies, this proposal is related to whether RAR is received from serving cell or candidate cell. It would be better if we defer the discussion on P1-4, and based on the progress we may achieve for P1-3, reconsider the alternatives of this proposal. |

Round 3

| Company | Input |
| --- | --- |
| Mod | After discussion in FL summary and GTW session, P1-3 and P1-3-1 have been agreed. So, based on these agreements we can resume our discussion on P1-4. |
| Lenovo | We prefer Alt-2, since RAR is transmitted from serving cell, therefore, candidate cell ID should be included. |
| ZTE | For this issue, we tend not to change the existing RAR structure to avoid mismatch for gNB and UE. |
| Ericsson | We prefer Alt-3: it is more efficient, less complex for the UE and the NW, avoids collision issues w.r.t. RA-RNTI. The contents of the MAC CE can be discussed later. |
| Futurewei | We think in this case, Alt-3 makes more sense. We consider it is a variant of without RAR. |
| OPPO | We support Alt2 with removing UE ID. Candidate cell ID is sufficient: - Alt2: candidate cell ID/UE ID is contained in RAR Alt3 shall be removed since we should discuss the contents of MAC CE, instead of whether new MAC CE or not. How to implement the MAC CE shall be up to RAN2. We RAN1 only agree the MAC CE function. |
| Nokia | First of all, we think along with the estimated TA, the value of RA-RNTI or/and RAPID used by the UE should be transmitted by the candidate cell to the source cell, so that source cell can identify for which UE the given TA is valid. Otherwise, if only TA is transmitted from the candidate cell to source cell, there is no way the source cell will know about the UE. Second (collision across different cells), first we need to answer this question: is it possible that multiple UEs in the source cell may use the same RAPID and RA-RNTI to trigger a PRACH towards different cells (candidate cell or/and the source cell itself) and waiting for a RAR during the same time window? Not sure if we can make sure unique RAPIDs across multiple candidate cells and the source cell. Third (collision within the same candidate cell), at least to have this scheme work, the source cell can make sure that there are no two UEs which have used the same RAPID and RA-RNTI for the same candidate cell and waiting for a RAR at the same time. If we agree that the above issues (second and third) are valid, we can have the following solution: we prefer the legacy way of RAR reception, via monitoring type-1 CSS over the source cell, with DCI scrambled with RA-RNTI (source cell has this information), and candidate cell ID in the MAC payload. With candidate cell in the MAC payload, the issue mentioned in the second point will be resolved, and the issue mentioned in the third point can be handled by the source cell itself. Summary: RAR should contain candidate cell ID and TA; RAR is received by monitoring type-1 CSS (DCI scrambled with RA-RNTI). |
| QC | Support Alt3, which does not need to restrict to CSS Type 1 and does not need Msg3. DCI is scrambled by C-RNTI, so no need UE ID as well. Candidate cell ID is also not needed if there is only one ongoing RACH procedure |
| NTT DOCOMO | OK to further consider Alt2 or Alt3. We think a relevant issue that should be discussed is whether PRACH to another candidate cell can be sent before RAR reception of PRACH to a prior candidate cell. (i.e., whether there is only one going RACH procedure.) |
| FGI | We are ok with Alt2 and Alt3. Regarding Alt2, UE ID is not needed. |
| Samsung | We are O.K. with Alt2 if UE ID is removed. We are also fine with Alt 3 |
| Huawei, HiSilicon | We need to first determine the RAR reception procedure and container of RAR before we discuss the information. For example, whether the RAR should still be received in RAR window, how the PDCCH scheduling PDSCH carrying RAR is scrambled, whether the RAR information is carried in MAC PDU (like RAR MAC PDU in 4 step RACH) or MAC CE (like Absolute TAC in MsgB in 2 step RACH or like regular TAC). To us, the current Alt 2 and Alt 3 is not mutual exclusive. |
| LGE | We have similar view as Huawei that Alt 2 and Alt 3 are not mutually exclusive. We can further discuss considering entire RACH procedure perspective. First, regarding how the PDCCH scheduling RAR PDSCH is scrambled, as Nokia mentioned, there is a possibility that RA-RNTI of different UE can collide within a serving cell type-1 CSS. So, to avoid detection error, UE ID for the PDCCH scrambling should be carefully investigated (which can be different RA-RNTI from serving cell) and prefer to keep UE ID in Alt 2. Second, candidate cell ID can be included in RAR PDSCH to indicate TA value of the candidate cell which requires new RAR MAC CE design. Alternatively, candidate cell ID can be indicated in PDCCH scheduling RAR PDSCH. Third, QCL assumption for PDCCH scheduling RAR PDSCH and RAR PDSCH should be discussed. Especially for RAR PDSCH, in legacy, UE assumes that RAR PDSCH is QCLed with SSB used for the corresponding RACH transmission. However, in Rel-18 LTM, RAR is transmitted from serving cell, so the legacy QCL assumption cannot be assumed anymore, e.g., the SSB for RACH transmission is associated with candidate cell while RAR is transmitted from serving cell. It should be further considered. Lastly, considering forwarding delay of RAR (especially in case of inter-DU scenario), starting point of RAR window and/or length of RAR window should be further studied. |
| CMCC3 | Fine with Alt 2 without UE ID, but with candidate cell ID and corresponding TA. Since if multiple TAs can be acquired for multiple candidate cells, then the identification of candidate cell would help to indicate the TA is from which cell. And if the candidate cell ID is introduced, then it is also within the scope of Alt 3. If only legacy RAR is allowed, then the acquisition of multiple TA from multiple candidate cell would induce a long delay, since RAR only carries one TA from one candidate cell. |
| Mod | Based on feedback shown above, the positions of companies are summarized as follows. - Alt1: additional parameters are not needed - ZTE - Alt2: candidate cell ID/UE ID is contained in RAR - Lenovo, DOCOMO, Samsung/OPPO/FGI(without UE ID), CMCC(without UE ID, with TA), Nokia(RAR should contain candidate cell ID and TA; RAR is received by monitoring type-1 CSS (DCI scrambled with RA-RNTI)), LGE(keep UE ID) - Alt3: others. E.g a new designed MAC CE - Ericsson, QC, DOCOMO, FGI It’s clear that majority companies see the need of additional parameters in RAR. So, I think we can focus our discussion on Alt 2 and 3 in the next step. @HW @LGE: I agree with you that RAR window related issues should also be considered, and we already captured this in P1-3-1. Also, as commented by HW and LGE, seems Alt.2 and 3 are not mutually exclusive. Regarding other related issues such as scrambling, QCL assumption and container of RAR, they’re listed in the following revision of P1-4. Proposal 1-4: When reception of RAR is configured, - The following information provided in RAR - candidate cell ID and corresponding TA value - FFS: whether UE ID is also contained in RAR - FFS - how the PDCCH scheduling PDSCH carrying RAR is scrambled - whether the RAR information is carried in MAC PDU (like RAR MAC PDU in 4 step RACH) or MAC CE (like Absolute TAC in MsgB in 2 step RACH or like regular TAC) - QCL assumption for PDCCH scheduling RAR PDSCH and RAR PDSCH |
| QC | For Proposal 1-4, not support. Candidate cell ID is not needed, since there is only one ongoing RACH procedure is allowed in current spec. After PDCCH ordered PRACH, UE just waits for the RAR, so no confusion. Unless RAN1 allows multiple ongoing RACH in parallel. If so, we need to discuss that first. Otherwise, candidate cell ID is not needed |
| Nokia | We think whether we need candidate cell ID or not depends on what we select for first/second FFS. Therefore, we suggest we first discuss whether we support type-1 CSS (DCI scrambled with RA-RNTI) or new MAC-CE based design? Then we should discuss the contents. |

- Issue 2: TA acquisition of RACH-less solutions

Open issues on TA acquisition of the candidate target cell(s) for RACH-less solutions and company views are summarized below.

Table 2. Summary of views on Issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | Alt 2 of RACH-less solutions (RACH-less mechanism as in LTE) | Support Alt 2: ZTE, Samsung, Spreadtrum, Intel, CATT |
| 2.2 | Alt 3 of RACH-less solution Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec | Confirm the working assumption: Ericsson, CATT, Futurewei, vivo, OPPO, CMCC, Google, Nokia(by configuration), Qualcomm Concern on Alt3: Huawei(DL synchronization errors, SSB bandwidth for timing difference estimation etc) , ZTE(wait until RAN4 has a preliminary result), Spreadtrum, Xiaomi(when and how to trigger UE-based TA measurement) |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P2-1

Round 1

Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA=0 or the latest TA value of the source cell in the cell switch command.

Note: this doesn’t mean to preclude TA values other than 0 and the latest TA value of the source cell.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Do not support. RACH-less in LTE implies that the UE directly sends Msg3. |
| Apple | Support. It is unclear for us why RACH-less implies Msg3 transmission. Our understanding on RACH-less is that it means UE directly uses the pre-determined TA value for UL transmission towards target cell without need of RACH procedure to obtain TA. Specifically, for RACH-less procedure, there is no RAR and it is unclear UE can get the UL grant for Msg3. |
| QC | Fine for the conclusion |
| Futurewei | We agree the two LTE cases should be covered. But we don’t see separate effort is needed. Note: serving cell does not store initial absolute TA but only tracking the delta TA of the UE. |
| OPPO | Looks like the what the proposal 2-1 tries to say is that: if the MAC CE cell switch command contains latest TA value or TA = 0, that means RACH-less handover and the UE does not conduct RACH procedure. If so, suggest to change the text of proposal as follows. To support RACH-less LTM, the MAC CE shall indicate the TA of target cell, not the TA = 0 or the source cell. Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE LTM can be supported by indicating TA=0 or the latest TA value of the source target cell in the cell switch command. |
| Nokia | We do not support the “Note” in the proposal. It makes it confusing. With the “Note” it means, the NW can always indicate any TA value in the cell switch command for the RACH-less case (without the any prior PDCCH order). Note that we have already supported indicating the TA of the target cell when RAR is not configured for the PDCCH order. Here, for RACH-less, we should only focus on the TA = 0 and the same TA (as the source cell) case. We can support the proposal 2-1 without the “Note”. Another note: In RAN2, RACH after the cell switch has also been considered as a fallback operation. To support that in RAN1, the UE may also be configured in the cell switch command that there is no TA in the cell switch command, and if the UE has not acquired the TA before the cell switch command (e.g., via RAR) then the UE may initiate the RACH after the cell switch command. |
| Lenovo | Fine with the conclusion. |
| ZTE | Support in principle. |
| Xiaomi | Not support. We only need to decide whether RACH-less mechanism as in LTE should be supported or not. It is not necessary to discuss how to support it. If it is supported, UE and NW just assume that the TA of candidate cells is same with that of serving cell. Then, there is no TA measurement and TA indication. Before TA=0 is indicated, the TA measurement should be performed. Then it is totally different from RACH-less mechanism as in LTE. And, by the way, we do not support RACH-less mechanism as in LTE. |
| Samsung | It is not clear what this conclusion proposing in addition to the WA in proposal 2-2. Considering with proposal 2-2 together, should this conclusion mean TA=0 is indicated to allow UE-based TA measurement? |
| MTK | We support the updated proposal provided by OPPO. Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE LTM can be supported by indicating TA=0 or the latest TA value of the source target cell in the cell switch command. |
| Spreadtrum | We support the proposal in general, OPPO’s version is more preferred, that UE does not need to do TA acquisition in advance. |
| NTT DOCOMO | OK to support in principle. Our understanding is that current cell switch command design has already supported the RACH-less mechanism. For the conclusion itself, it is not clear how to understand ‘the latest TA value of the source cell’. |
| Huawei, HiSilicon | We support the FL proposal. We do not see any reason to exclude the scenario which can be applicable to LTE RACH less case. As for the standard impact, we think a special value indicating “same as source cell” should be introduced in TA indication in CSC, which might different from other cases where absolute TA value might be indicated. |
| IDCC | Support the conclusion. |
| Mod | Based on comments from OPPO, Nokia, MTK and Huawei, the proposal is updated as follows. @OPPO @MTK: in previous meeting, it’s agreed to study RACH-less solutions including RACH-less mechanism as in LTE. So, to avoid any misunderstanding, I prefer to keep this terminology. @Nokia: hopefully the revision of P2-1 can address your concern on introducing more TA values in RACH-less solutions. Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Please share your views on the following proposal. Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. |
| OPPO | So, now, the proposal is actually designing the MAC CE cell switch command. It proposes that setting TA field in MAC CE command to be 0 or = the TA value of current serving cell indicates RACH-less LTM. The design of MAC CE field shall be part of RAN2, right? Furthermore, RAN2 also discuss how to indicate RACH-less LTM, shall we leave this discussion to RAN2? |
| QC | Fine with FL’s conclusion |
| Lenovo | Share a similar view with OPPO, it can be up to RAN2’s discussion. |
| NTT DOCOMO | OK to support in principle. |
| Futurewei | Fine with the principle, Share similar view as OPPO. Suggest to leave the detailed MAC CE design to RAN2. |
| ZTE | Support |
| Samsung | Support in principle, and we share similar view with OPPO, Lenovo, and Futurewei. |
| Xiaomi | Does Proposal 2-1 mean RACH-less mechanism as in LTE can be supported via RACH-based mechanism with indicating TA value of target cell as TA=0? From our understanding, they are different. For RACH-less mechanism as in LTE, UE assumes the TA of candidate cell is same with that of serving cell. While, for RACH-based mechanism with indicating TA value of target cell as TA=0, at least TA measurement should be performed, which is totally different with RACH-less mechanism as in LTE. Maybe we can just conclude that RACH-less mechanism as in LTE is not supported/considered in LTM. |
| CMCC | Fine with the proposal. |
| Spreadtrum | Support the proposal. We think it is needed due to RAN1 can make decision and send it to RAN2 that even without TA acquisition before LTM CSC, TA value (0) can be in the LTM CSC. It can help RAN2 to do signaling design. |
| FGI | We are fine with FL’s proposal. |
| LGE | We are also fine with the suggestion to leave it up to RAN2. |
| Huawei, HiSilicon | Fine with the proposal. RAN1 can suggest the value indicated by MAC CE, although signaling design is RAN2‘s task. |
| vivo | Fine with the proposal. |
| Nokia | Fine with the updated FL’s conclusion. |
| IDCC | Fine with the proposal. |
| Mod | @Xiaomi: to my understanding, the procedure may not be exact the same as what we have in LTE. However, if 0 or the same value as source cell can be acquired by UE for candidate cell without performing RA, to some extent, equivalent effect as RACH-less in LTE is achieved. In this sense, we term this scheme as RACH-less mechanism as in LTE. To address such kind of concerns and avoid any possible ambiguity, P2-1 is revised as follows. Proposal 2-1 (proposed conclusion): without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. Regarding whether this is totally a RAN2 issue, I share the same view as HW that RAN1 can suggest the value indicated by MAC CE and leave the signaling design to RAN2. |
| Google | We are OK with the updated conclusion. |
| MTK | Fine with FL’s conclusion |
| Ericsson | After checking with RAN2, we are still concerned about the expression “RACH-less as in LTE”. RACH-less in LTE means that the UE does not send PRACH, but it also means that it instead sends Msg3, with the Msg3 content. The grant for that transmission is provided either in DCI from target, or by configuration, We also do not see why “as in LTE” is needed: the proposed conclusion is fine without it: Proposal 2-1 (proposed conclusion): without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. |
| QC | Fine with latest FL’s conclusion |
| Mod | @Ericsson: thanks for your comment and suggestion on P2-1. As I mentioned, I agree with you that the procedure may not be exactly the same as RACH-less in LTE. So, if this is also agreeable to other companies, we can revise P2-1 according to your suggestion. Proposal 2-1 (proposed conclusion): without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. |

Round 3

| Company | Input |
| --- | --- |
| Mod | Please share your views on the following proposal. Proposal 2-1 (proposed conclusion): without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. |
| ZTE | Support the updated proposal 2-1 |
| Ericsson | Support |
| Futurewei | Fine with Proposal 2-1. |
| OPPO | We can not support this. Our concerns were not addressed. As we commented in last round, the proposal is actually designing the MAC CE cell switch command. It proposes that setting TA field in MAC CE command to be 0 or = the TA value of current serving cell indicates RACH-less LTM. The design of MAC CE field shall be part of RAN2, right? RAN2 also discuss how to indicate RACH-less LTM, shall we leave this discussion to RAN2? |
| Nokia | Support |
| QC | Fine for the conclusion |
| NTT DOCOMO | Support. |
| Apple | Support |
| FGI | Support |
| Samsung | Support |
| Huawei, HiSilicon | support |
| Mod | @OPPO: I agree with you that the signalling design is a part of RAN 2 work. However, we may have our preference on the values we need. To address you concern, let’s check the following update. Proposal 2-1 (proposed conclusion): From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. - Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. - Note 2: The feasibility can be further concluded by RAN2 |
| CMCC3 | Support the latest version from FL. |
| QC | Fine for the latest FL’s proposal |

P2-2

Round 1

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption:

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Support |
| QC | Support |
| Futurewei | Support. We consider this working assumption is a high-level principle. More details will be worked out. The network assistance for compensation of source and target node asynchronous timing offset should be included to make the scheme practical and accurate. |
| OPPO | Support |
| CMCC | Support |
| Nokia | Support |
| ZTE | Do not support. It confuses us why to confirm the WA so early without evaluation and analysis regardless of RAN1 and RAN4. In our view, UE based TA measurement is more related to RAN4, i.e., MTTD or MRTD, it makes no sense to confirm the WA without any conclusion and feedback from RAN4. If RAN4 agrees that UE-based TA measurement is workable and should be supported, we are fine to confirm it and further consider RAN1 spec efforts. With the above consideration, we don’t expect to confirm Working assumption agreed in the last meeting until receiving RAN4 reply LS and conclusion or evaluation and analysis. |
| Vivo | Support. @ZTE, thanks for kind explanation. As we discussed in extensively in the last meeting and explained multiple times, this proposal is not about RACH-less, we have agreed to support PDCCH order RACH already. The benefit, for example when there are multiple candidate cells (for example cell1, cell2, cell3, …) to which network triggers PDCCH order RACH sequentially before cell switch, then network may choose one of the candidate cells as the target cell for cell switch for which the PDCCH order RACH was performed earlier (for example the first one, cell1), if the UE has capability to maintain TA then network can send CSC without needing to perform PDCCH order RACH again for this cell (cell1). And, according to the discussion in ZTE RAN4 contribution, it was claimed that FR1 is fine but could be problematic for FR2. According to our understanding of the problem statement on difference between MRTD and MTTD, if the error is there then even RACH cannot compensate it, which means this problem is not unique to UE based TA measurement |
| Xiaomi | Support We want to clarify that we do not have concern on Alt3. We were trying to analysis the impacts on RAN1 spec., like the configuration of the inherent time difference, if exists, and when and how to trigger UE-based TA measurement. |
| Samsung | Support |
| MTK | Do not support. We agree that it is a RAN4 related work. An LS is needed to ask for RAN4’s opinion. |
| Spreadtrum | We prefer to have more discussion before confirm the WA, such as below • Issue1: When UE has to do the TA acquirement • Issue2: Whether or not this type of TA acquisition has further trigger or just by UE decision, or is based on certain conditions • Issue3: When serving cell and candidate cell have different DL timing and/or UL timing, whether UE-based TA measurement through Rx timing difference can be supported |
| NTT DOCOMO | Support. |
| Huawei, HiSilicon | We do not object to confirm the WA. But as we mentioned in our tdoc, the accuracy of TA acquired by UE itself should be further evaluated. Its usage scenario should be further studied, e.g. whether it is used independently or together with RACH based scheme, which UL channels can use TA acquired by UE itself. |
| IDCC | Support. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Please share your views on proposal 2-2. |
| QC | We suggest to confirm WA and carry on discussion from RAN1 point of view. The feasibility can be later decided by RAN4. There is no time for LS iterations. RAN1 should be the initiator. In addition, RAN4 also needs RAN1 initial discussions to further check feasibility. We think this is the right order to go |
| Futurewei | Share the same view as QC. Support proposal 2-2. We should just follow the normal process: RAN1 starts to work then RAN4 follows up. RAN1 do need to start discussion then provide the initial framework for RAN4 to evaluate. Based on companies’ proposals we think the accuracy and reliability issues are taken into consideration from RAN1 perspective. |
| ZTE | Our position is the same as in Round 1 |
| Samsung | Support |
| Xiaomi | Support |
| CMCC | We support to confirm the WA. The accuracy of TA acquired by UE should be also considered as one of the aspect of UE capabilities. |
| Vivo | Support to confirm the working assumption first and then discuss the details. |
| IDCC | Fine with the proposal. |
| Google | Support. Perhaps one WA can be done as the following revision. Working Assumption From RAN1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec |
| Ericsson | Support |
| Mod | Based on discussions and comment from Google, P2-2 is revised as follows. Working Assumption From RAN1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec |

Round 3

| Company | Input |
| --- | --- |
| Mod | Please share your views on proposal 2-2. Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption with the following update: Working Assumption From RAN1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec |
| ZTE | As said in round 1 and 2, we don’t support confirming this WA at this stage. |
| Ericsson | Support |
| Mod | @ZTE: I can fully understand your concern. Also, I agree with you that this is related to other working groups. However, there’re still many issues related to RAN2/3/4, and we cannot just wait the confirmations from all the groups before moving forward. |
| Futurewei | Support |
| OPPO | Support Even though we think adding “From RAN1 perspective” is not needed. |
| Nokia | Support |
| QC | Support. To address ZTE’s concern, perhaps add a note to say “The feasibility can be further decided by RAN4”. |
| NTT DOCOMO | Support. |
| FGI | Support. |
| Samsung | Support |
| Huawei, HiSilicon | Maybe we can add the following FFS into the agreement in case there is concern on the accuracy FFS: whether additional mechanism(s) is required to meet the UL synchronization requirement FFS: applicable signals/channels |
| Mod | @HW @ Spreadtrum: thanks for your comments, as mentioned in this WA, other impacts on RAN1 spec are FFS. So, it would be appropriate to have further discussion on the issues raised from you after the confirmation of this WA. |
| CMCC3 | Support. |

- Issue 3: Association between TA/TAG and candidate target cell

Open issues on TA indication and company views are summarized below.

Table 3. Summary for Issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1 | Association between TA and candidate cell - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - The association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly - Alt 2.1. The association is provided as part of candidate cell(s) configuration - Alt2.2 The association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration - Alt3: Do not support associating the TA with a candidate cell | Alt1: Samsung, MTK Alt2: Alt2.1: OPPO, Spreadtrum, CMCC, Transsion Holdings, NTT DoCoMo, LGE,CATT, QC Alt2.2: Huawei, Futurewei Alt3: Google |

P3-1

Round 1

Proposal 3-1:  Explicit association of TA/TAG and candidate cell is provided as a part of candidate cell(s) configuration

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | First of all, we note that this is only needed when RAR is configured. We do not see that we can reuse the TAG for this purpose: in most cases, every candidate CG will only have one TAG. Still, some association is needed. |
| Apple | We may need to first clarify the usage of TAG for target cell. In current spec, the pTAG for Pcell is hard-encoded in specification as TAG 0. Then, it means, for Pcell switching, the TAG ID is not changed, instead of update of ‘TA value’ from serving cell to target cell. TA update can be achieved by using cell switch command. Before cell switch command, the RAR includes the target cell ID. Is it sufficient to maintain the mapping between TA value and target cell ID at UE without additional TAG information? Then, when NW transmits the cell-switch command, UE can use/update TA for the pTAG accordingly. For the case of LTM involving sTAG of target cell group, more discussion maybe needed regarding TAG association. In any case, it is worth noting that the two Tas here is a bit different with that in MIMO agenda in our view since UE would not transmit UL data towards two gNBs at the same time in LTM. Specifically, the 2nd TA is purely for handover preparation. Hence, we should seek for keeping the existing design e.g., the maximum number of ‘4’ TAGs for LTM and TA value can be updated in case of switching from serving cell to target cell. |
| QC | Suggest to say “is provided as a part of configuration for candidate cell(s)”, since the original wording may imply the info is configured under the candidate cell. Other options are under discussion in RAN2 |
| Futurewei | We support Alt2.2. Multiple TRPs in cell is now a very normal scenario. Beams from a TRP can form a TAG with a TA value largely different from that of a different TRP/TAG. Candidate beams from a candidate cell configured to the UE can from different TRPs. They cannot be associated with one TAG. Therefore, the association should be at SSB level not at the cell level. |
| OPPO | Each candidate cell can be associated with one TA value. And each cell is associated with one TAG, when that cell is serving cell. However, when the cell is only a candidate cell, it is not clear why a TAG is needed and also not clear how the TAG is used. |
| Nokia | First of all, we would like to clarify that this is only for PDCCH order RACH mechanism? As we highlighted in our contribution, we first need to discuss if we need a TAG ID or not. The candidate cell identify information is already provided to the UE. When RAR reception is configured, the TA can be associated with the candidate cell for which the identify is provided in the PDCCH order, and when RAR reception is not configured, the TA given in the cell switch command can be associated with the candidate cell for which the identity information is provided in the cell switch command. Then which additional association are we referring with this proposal? Is it to associate the given TA with a TAG ID (which is configured in RRC, in the candidate cell configuration)? We don’t oppose such association but would like to clarify our understanding first. Also, if we provide TAG ID configuration to the UE, somehow we need to provide the associated TAG ID to the UE, which is not supported based on the current agreements as TAG ID is not given in the RAR or cell switch command. |
| ZTE | Support. |
| Xiaomi | Support |
| Samsung | Not support. For multi-TRP, it is agreed to make association between TCI state and TA value. We prefer to have one unified solution at managing multiple TA. In current release, multiple TA is discussed for LTM and MDCI based multi-TRP scenario separately, where possible scenario extension to SDCI multi-TRP is visible. Case specific solution cannot be helpful. |
| MTK | We would like to clarify the question we discuss. First, we agree Apple’s comment that TAG ID is not changed, what we need is to update its TA value. Originally, what we want to discuss is how to indicate the candidate cell’s ID in the reserved bit(s) in DCI format 1_0 for PDCCH order (based on RAN1 #112 agreement). Considering that it needs 11 bits to bring the completed Cell ID and the reserved bits in DCI format 1_0 is not long enough, we suggest to use an implicit method, i.e., indicating the TCI state ID or the index of the activated TCI state in the DCI format 1_0 of the PDCCH order. |
| Spreadtrum | Support |
| NTT DOCOMO | We prefer Alt2.1. Multiple candidate cells may exist in the same TAG (i.e., share the same TA), thus, it is important to support association between a candidate cell and a TAG. And explicit association is a simple and clean configuration. |
| Huawei, HiSilicon | First, we would like to echo QC’s comment that TA association may NOT be necessarily in CellGroupConfig for candidate cell. As for the association, we prefer Alt 2.2 especially for UE acquire TA from RAR before CSC. Before UE transmit PRACH to candidate cell, it should first be indicated with candidate cell ID and SSB index in the PDCCH order, which can be used to associated with TA. As for the TAG mentioned, it is not necessary before UE receives CSC as UE will only use it in the target cell after cell switch, except for the cases of mTA mTRP/CA. |
| LGE | Support the proposal. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Please share your views in this table. |
| Xiaomi | For now, we have concern on this proposal. At first, for the TA of target cell, it is indicated in cell switch command. If TAG ID is included in cell switch command, then Proposal 3-1 is acceptable to us. Otherwise, the association of TA and target cell might be provided explicitly in cell switch command based on the cell id or cell logical id. Therefore, there is another solution about the association of TA and target cell which is provided explicitly in cell switch command/TA indication signaling. Secondly, for the TA of candidate cell, we even do not know whether the TA of candidate cell should be indicated to UE or not. Maybe we can discuss it first. If the TA of candidate cell should be indicated, the association of TA and candidate cell can also be provided explicitly in TA indication signaling or provided as a part of candidate cell(s) configuration. In short, we can discuss the association between TA and target cell first. Proposal 3-1A: The association of TA/TAG and target cell is explicitly provided - Alt.1: as a part of candidate cell(s) configuration - Alt.2: as a part of cell switch command/TA indication signaling And for the TA of candidate cell, we can discuss whether the TA of candidate cell should be indicated to UE or not first. Proposal 3-1B: whether the TA of candidate cell should be indicated to UE (every time the TA of candidate cell is measured) - Alt.1: The TA of candidate cell should be indicated to UE - FFS: the association between TA and candidate cell - Alt.2: The TA of candidate cell does not need to be indicated to UE |
| Nokia | Based on the comments from the companies in round 1, we think we should first discuss whether we need to indicate a TAG ID or not; if needed, then how we support that. |
| IDCC | Fine with the proposal. |
| Google | We don’t support this proposal in current form, which is unclear to us. What’s the intended use case and benefit? Does it mean we support 2TA operation for candidate cell? |
| MTK | We prefer to clarify the intension first. Do we need to indicate TAG ID or candidate cell ID implicitly/explicitly in PDCCH order? |
| Ericsson | We are not sure why we need this. For sure, one or more TAGs will be configured for each candidate cell. The question is how the UE handles the TA it receives in the RACH procedure before the cell-switch. |
| QC | Support Alt2.1 with wording modification to say “is provided as a part of configuration for candidate cell(s)” |

Round 3

| Company | Input |
| --- | --- |
| Mod | Please share your views in this table. |
| Ericsson | The use case must be clarified: we assume that the UE receives a TA value for a certain candidate cell. If the TA is received in a normal RAR, the UE would reset the TA timer corresponding to TAG 0. Should we instead focus on the corresponding actions when a TA for a candidate cell is received? |
| Nokia | Agree with Ericsson (also raised by Google, MTK, Apple, Xiaomi, OPPO in the previous rounds) that the issue or use case is not clear for this proposal. As far as we understand, we are trying to associate/indicate TAG ID in addition to the candidate cell ID for a TA. Maybe the proponents may try to highlight what are we trying to solve here, please? |
| NTT DOCOMO | We’d like to clarify whether it is possible to get the same TA for different candidate cells. If not, then the association is not needed. On the other hand, if there is a possibility (and we believe so), wouldn't it be beneficial from a latency perspective to already know the TA when, for example, after a cell switch, next cell switch happens again (i.e., subsequent LTM)? In that case, how to provide the association should be discussed. |
| Apple | Similar as other companies, we also would like to clarify the intended use case/function. |
| LGE | It can be further discussed when the reception of RAR is configured. In that case, if multiple TAs for multiple candidate cells are stored by UE, each of TAs can be maintained per TAG which is associated with each candidate cells. We are open to here other companies’ view/clarification. |
| Mod | As shown in tables above, positions of companies can be summarized as follows: - Association is needed: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,the association between TA/TAG and TCI states can be configured - Samsung - Alt2: Associate TA/TAG and candidate cell explicitly - Alt 2.1. The association is provided as part of configuration for candidate cell(s) - QC, ZTE, Spreadtrum, DOCOMO, LGE, IDCC - Alt2.2 The association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration - Futurewei, HW, - Association is not needed: Google, MTK, Apple, Xiaomi, OPPO, Nokia, Ericsson It’s clear that views from companies are still diverged. However, regarding the usage of TAG in TA management for LTM, DOCOMO’s clarification sounds reasonable. In addition, as mentioned at least by QC and HW, seems we don’t need to restrict that it’s configured under the candidate cell. So, for further discussion, P3-1 is revised as follows. Proposal 3-1: Explicit association of TA/TAG and candidate cell is provided as a part of configuration for candidate cell(s). |
| QC | Support FL’s proposal 3-1 |
| Nokia | We would like to clarify that the we didn’t say that the association is not required. The current wording of proposal is not clear enough. TA and TAG is not the same thing. What do we want with this proposal? - Association between TA and TAG? - Association between TA and candidate cell – we think that is already supported. No additional agreements are required. - Association between TAG and candidate cell? |
| Mod | @Nokia: thanks for the questions. To make the proposal clear, P3-1 is revised as: Proposal 3-1: Explicit association of TAG and candidate cell is provided as a part of configuration for candidate cell(s). |

- Other issues

Table 5. Summary for other issues

| # | Issue | Companies’ views |
| --- | --- | --- |
| 4.1 | Number of Tas associated with candidate cell(s) can be handled by UE. | Maximum number of TA associated with candidate cell(s) could be based on UE capability: Huawei, CATT, Spreadtrum, Qualcomm |
| 4.2 | For PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx | - Support additional time gap between the PDCCH order and PRACH(detailed value depend on RAN4): Qualcomm, Huawei, MTK, Lenovo, Apple - Defer the impact/interruption on UL Tx interruption of serving CCs since current RAN2 is not sure whether to maintain CA during the LTM process: MTK |
| 4.3 | Whether two TA(s)/TAG(s) for a candidate cell can be configured | Yes: Nokia/Google(up to two), Huawei, Qualcomm No: LG(up to one), ZTE |

P4-1: closed

Round 1

Proposal 4-1:  For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA

- The maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Propose a small clarification: Proposal 4-1A: For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA - The maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability. - Support of more than one of the candidate cells can be associated with acquired Tas is up to UE capability |
| Apple | Support. The wording from FL is more aligned with our original intention to limit the Tas that UE can maintains for LTM before receiving cell switching command. The revised wording from Ericsson is not very clear for us as it seems what UE capability reports is ‘number of candidate cells’, instead of maintained Tas. FL proposal is clearer for us and therefore is preferred. |
| QC | Support |
| Futurewei | We are fine with Proposal 4-1. |
| OPPO | Support the FL’s version, where the sub-bullet means the total number of TA of candidate cells the UE can handle. We are not ok with Ericsson’s change, which changes the meaning of the sub-bullet. |
| CMCC | Support. |
| Nokia | Support |
| Lenovo | Support. |
| ZTE | We are generally fine with the proposal. But we prefer to clarify whether candidate cell configuration should take the UE capability into account or whether dropping rules are needed in case of TA values exceeds the UE capability. |
| Vivo | Support. |
| Xiaomi | Fine |
| MTK | Support |
| NTT DOCOMO | Support. |
| Huawei, HiSilicon | support |
| LGE | OK. |
| Mod | @Ericsson: as mentioned by Apple and OPPO, based on discussion we had in previous meeting, the intention of the sub-bullet is to clarify number of Tas that can be maintained, rather than acquired, by UE for LTM before receiving cell switching command @ZTE: regarding your question, from my understanding, yes, the configuration should take the UE capability into account. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Seems the main bullet of original version of P4-1 caused some confusion. Actually, the sub-bullet is the essential part of this proposal. So, as shown below, can we just remove the main bullet from the original version of P4-1 and reflect our view on related UE capability directly. Proposal 4-1: For TA management in L1/L2 based mobility, the maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability |
| QC | Fine with FL’s proposal |
| NTT DOCOMO | Support in principle. It may be difference between main bullet and sub-bullet. Main bullet indicates that some candidate cell may have same TA as other candidate cells. Sub-bullet indicates UE capability. We think main bullet is also important. For proposal 4-1, we suggest a minor change following. Proposal 4-1: For TA management in L1/L2 based mobility, at least one TA with candidate cell(s) should be handled by UE and the maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability |
| Futurewei | Fine with Proposal 4-1. |
| ZTE | We are fine with the proposal updated by FL. |
| Samsung | Support |
| CMCC | Fine with FL’s Proposal 4-1. |
| FGI | Support. |
| LGE | Fine with Proposal 4-1. However, somehow it seems there are two verbs in a sentence, “can” and “is”. (I’m not a native English speaker, so correct me if I missed something..) So, the proposal can be revised as: Proposal 4-1: For TA management in L1/L2 based mobility, the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability |
| Huawei, HiSilicon | support |
| vivo | Fine with Proposal 4-1. |
| Nokia | Fine with FL’s Proposal 4-1. |
| IDCC | Fine with the proposal. |
| ITRI | Support |
| Mod | Based on comments from DOCOMO and LGE, P4-1 is revised as follows. Hopefully, this update is acceptable to companies. Proposal 4-1: For TA management in L1/L2 based mobility, support at least one TA associated with one candidate cell and the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability. |
| Google | From the discussion, we suggest the following revision. Proposal 4-1: For TA management in L1/L2 based mobility, support at least one TA can be associated with more than one candidate cell and the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability. |
| Ericsson | On high level, it would seem that the NW sends a TA value, and that TA value is associated with a certain candidate cell. It would seem likely that there is a one-to-one mapping between a TA value and a candidate cell: it is not likely that one TA value is applicable to two different candidate cells. And vice versa, it is realistic that two TA values are applicable to one candidate. So the association is always one-to-one. Formulations on “associated” are then mis-leading. So the question would be: for how many candidate cells can the UE store/remember/maintain a TA value? Here we think that the UE should always be capable of storing/remembering/maintaining this for one candidate cell, but it would be up to UE capability to store/remember/maintain the TA for more than one candidate cell. But again: the limitation will be on the number of candidates. But the formulation is still confusing: the limitation is on how many candidate cells the UE can handle TA. I assume that no one questions what we try to describe: but the word “associate” makes the proposal unclear, and as long as “associate” stays, it will remain unclear. Could we try this formulation: Proposal 4-1: For TA management in L1/L2 based mobility, when RAR is configured, - the UE stores/remembers/maintains the TA for at least one candidate cell - storing /remembering/maintaining the TA for more than one candidate cell is up to UE capability support at least one TA can be associated with more than one candidate cell and the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability. |
| Mod | Thanks Google and Ericsson for the comments and suggestions. To avoid any ambiguity, let’s focus on the capability of UE to handle TA in this proposal. As shown below, P4-1 is revised accordingly. Proposal 4-1: For TA management in L1/L2 based mobility, when RAR is configured, - the UE stores/remembers/maintains/handles the TA for at least one candidate cell - storing/remembering/maintaining/handling the TA for more than one candidate cell is up to UE capability support at least one TA can be associated with more than one candidate cell and the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability. |

Agreement

For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured,

- the UE stores(remembers/maintains/handles) a TA for at least one candidate cell

- storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability

  - detailed number of candidate cell is up to UE capability

P4-2

Round 3

Proposal 4-2:  For PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured

- Support additional time gap between the PDCCH order and PRACH(detailed value depend on RAN4)

- FFS: Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

| Company | Input |
| --- | --- |
| Mod | Please share your views in this table. |
| Lenovo | RF retuning is needed for inter-frequency candidate cell which is similar with the case that the candidate cell is not UL serving cell, therefore, we propose to support the case too. And a revised proposal is provided as following. Revised |
| Ericsson | In principle, we understand the issue. But we don’t recognize the concept “UL serving cell”, and the explanation is non-intuitive: the candidate cell can have PUCCH/PUSCH configured but will in most cases not be a serving cell. It could be that the carrier is the same, but that’s something different If the issue is inter-frequency, we should write that. We assume this is the same issue that Nokia raises under P4-x (1). But overall, this feels like a RAN4 issue. Maybe we could write an LS to RAN4 saying that: - RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. - RAN1 believes that this will require that the time gap is increased - RAN1 relies on that RAN4 describes the situations where this is needed and the corresponding relaxations. |
| Futurewei | Fine with Proposal 4-2. Depends on RAN4. |
| OPPO | “not UL serving cell”: our understanding can be that the PRACH is configured in UL that is not part of UL CA. We did not agree to support this case. The previous agreement only says to “study the issue..” Before we can agree something like the proposal 4-2, we shall first discuss and agree whether PDCCH order PRACH for candidate cell can be configured on UL that is not part of the UL CA. |
| Nokia | Agree with Ericsson, we need to consult with RAN4 for this issue. As we are not sure even some of the current delay elements in section 8.1, 38.213, i.e., ΔBWPSwitching, Tswitch can be reused as it is in case of a LTM candidate cell. We should discuss the need to additional time gap, but we should first make sure if no change/enhancement is required to an existing delay element. |
| QC | For proposal 4-2, support and we are fine to let RAN4 decide the value. @All: Pls check our issue description/proposal (Proposal 2 & 3) provided below for more background @Lenovo @E///, understood inter-frequency may be more intuitive to understand. The issue is that we don’t have inter-frequency defined for UL. There is only intra/inter-frequency measurement defined for DL, which may not say anything for UL. For example, if a candidate cell is intra-frequency for DL measurement, then it has same SSB frequency as the serving cell, but it does not imply the candidate cell has same center-frequency and BW as the serving cell in UL. However, we are also fine for Lenovo’s revision if it is easy to understand @E/// thanks for the LS draft points. However, they may not provide concrete example for RAN4 to understand the issue. RAN4 may further ask for detailed issue. Slightly prefer FL’s Proposal 4-2 at least as one example @OPPO, RAN2 119 has agreed non-serving cell can be candidate cell. Btw, our issue description/proposal has also been provided below for more background R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) b) FFS the target PCell is a current SCell c) FFS the target SCell is the current PCell. ------------------------------Issue description and proposal in QC Tdoc ( R1-2303611)---------------------------- As described in RAN1 #112 FL summary (R1-2302165), for PRACH on candidate cell not part of UL CA, UE needs to prepare/store separate RF config for each such candidate cell. This is because UE needs to change RF config from UL CA to that candidate CC for PRACH. The RF config includes the Tx filter to satisfy the corresponding CA/single CC emission mask requirement. In the existing time gap between PDCCH order DCI and PRACH in R17, the UE capability term Tswitch is introduced to account for the RF config switching action latency, which has max value of 210 us and assumes the corresponding source/target RF config has been generated/stored in memory. However, in case of LTM, the candidate cells across all frequencies may have a large number, and generating/storing RF config in advance for every candidate cell for potential PDCCH order may consume significant amount of memory resource, especially when those candidate cells have no traffic. Therefore, we propose for PDCCH-order based PRACH on a candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured, an additional UE capability latency component Tprepare can be added to the existing time gap between the DCI and PRACH as defined in 38.213->8.1. The corresponding UE capability candidate value should at least include 0 and 7 ms with the non-zero value accommodating the RF config generation latency when UE does not in advance generate/store the corresponding RF config for PRACH. 38.213->8.1: Existing time gap between PDCCH order DCI and PRACH - NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec 38.331: UE capability on Tswitch ULTxSwitchingBandPair-r16 ::= SEQUENCE { bandIndexUL1-r16 INTEGER(1..maxSimultaneousBands), bandIndexUL2-r16 INTEGER(1..maxSimultaneousBands), uplinkTxSwitchingPeriod-r16 ENUMERATED {n35us, n140us, n210us}, uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL } ULTxSwitchingBandPair-v1700 ::= SEQUENCE { uplinkTxSwitchingPeriod2T2T-r17 ENUMERATED {n35us, n140us, n210us} OPTIONAL }. Proposal 2: For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH, an additional UE capability latency component Tprepare can be added to the existing time gap between the PDCCH order DCI and PRACH as defined in 38.213->8.1 - NT,2+ ∆BWPSwitching+∆Delay+Tswitch+Tprepare msec - The UE capability candidate values of Tprepare at least include 0 and 7 ms - The non-zero candidate value is to accommodate the RF config generation latency when UE does not in advance generate the corresponding RF config for PRACH In addition, the PRACH on candidate cell that is not current UL serving cell may interrupt UL Tx on serving cells with PUCCH/PUSCH. Similar issue exists for SRS carrier switching, where the SRS Tx on a serving cell without PUCCH/PUSCH will interrupt UL Tx of certain serving cells with PUCCH/PUSCH, which are indicated by gNB based on UE capability report. In addition, prioritization rule has been specified if the SRS Tx on a serving cell without PUCCH/PUSCH overlaps with PUCCH/PUSCH on other serving cells. 38.214->6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carriers in the set S(c2).. In case of LTM, to simplify the design, we propose to prioritize the PDCCH order based PRACH on candidate cell that is not current serving cell with PUCCH/PUSCH when the PRACH Tx overlaps with UL Tx of interrupted serving cells with PUCCH/PUSCH, since the PRACH Tx is dynamically scheduled by gNB. In this case, UL Tx on any interrupted serving cell is dropped at least during the PRACH transmission plus required switching time before and after the PRACH Tx. Detailed definition of interruption start/end time and corresponding interrupted serving cell can be decided by RAN4. Proposal 3: For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH, the PRACH is prioritized if overlapping in time with UL Tx on any interrupted serving cell - In this case, UL Tx on any interrupted serving cell is dropped at least during the PRACH Tx plus required switching time before and after the PRACH Tx - Detailed definition of interruption start/end time and interrupted serving cell can be decided by RAN4 |
| vivo | We are fine with the proposal and sending LS to RAN4. CC without PUSCH/PUCCH transmission is what SRS carrier switching was trying to address for gNB obtaining CSI from SRS measurement. |
| NTT DOCOMO | Agree with Ericsson and Nokia, we need to consult with RAN4 for this issue. And agree with Nokia’s comments on delay elements. |
| Huawei, HiSilicon | As for the gap between PDCCH order and RACH, the formula is defined in RAN1 spec but some values are referred to RAN4 spec. we tend to agree with Ericsson that an LS should be sent to RAN4 any additional factor should be added into the existing formula in 213 spec. as well as the scenario mentioned by QC for inter frequency case, we also see the problem in intra frequency asynchronous deployment scenario, where the DL timing need to be adjusted before UE transmit PRACH to candidate cell. We hope it can be raised although to RAN4. As for the FFS, we agree with the issue. However, it is purely RAN4 task. |
| LGE | Agree with Ericsson. |
| Mod | As explained by QC and some other companies, this may not be purely a RAN4 issue. Regarding the FFS part, can we simply remove it? Proposal 4-2: For PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured, or inter-frequency cell - Support additional time gap between the PDCCH order and PRACH(detailed value depend on RAN4) - FFS: Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx Regarding the LS to other working groups, as usual, an LS including all the agreements in this meeting will be drafted. |
| QC | We think FL’s Proposal 4-2 is more efficient. We think it is mainly RAN1 issue. As described in our Proposal 2, the additional latency depends on how fast UE generates the RF config for the candidate cell. It depends on UE processing power, which may vary across UEs. So we think it is better to be UE capability. Super UE may even have 0 additional latency. This is a normal UE capability issue like other RAN1 UE features, and may not be related to RAN4 testing requirement, and is also not related to other existing delay terms. It does not preclude any potential change for the existing terms. If companies still don’t quite understand the issue, we are fine to totally leave the issue to RAN4 with clear description of the issue/proposal to avoid multiple iterations for clarification LS to RAN4 can include the following info - RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. - RAN1 believes that this will require that the time gap is increased at least for the following scenario - [R1-2303611] For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH, an additional UE capability latency component T_prepare can be added to the existing time gap between the PDCCH order DCI and PRACH as defined in 38.213->8.1 - N_(T,2) + ∆_BWPSwitching + ∆_Delay + T_switch + T_prepare msec - The UE capability candidate values of T_prepare at least include 0 and X ms, e.g. X=7 - The non-zero candidate value is to accommodate the RF config generation latency when UE does not in advance generate the corresponding RF config for PRACH - RAN1 relies on RAN4 to verify the need for the above additional latency and, if so, the corresponding value and whether UE capability is needed - RAN1 relies on RAN4 to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH |
| Nokia | In the above proposal, as we commented above, we need to confirm if the existing delay elements can be reused as it is? We want to add the following to the list: - Confirm with RAN4 whether any update is required to ΔBWPSwitching, ΔDelay - FFS: whether any update requires to 𝑁𝑇,2 and Tswitch. |
| Mod | As recommended by QC, we have two options now. One is to treat it as mainly a RAN1 issue, or we may also consider to leave this to RAN4 and send the LS including the issues identified by us. For option 1, the proposal is updated according to Nokia’s suggestion. Companies are encouraged to provide your views on the following options. Option 1: discuss in RAN1 based on the following proposal Proposal 4-2: For PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured, or inter-frequency cell - Support additional time gap between the PDCCH order and PRACH(detailed value depend on RAN4) - Confirm with RAN4 whether any update is required to ΔBWPSwitching, ΔDelay - FFS: whether any update requires to 𝑁𝑇,2 and Tswitch. Option 2: send an LS to RAN4 with the following info - RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. - RAN1 believes that this will require that the time gap is increased at least for the following scenario - [R1-2303611] For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH, an additional UE capability latency component T_prepare can be added to the existing time gap between the PDCCH order DCI and PRACH as defined in 38.213->8.1 - N_(T,2) + ∆_BWPSwitching + ∆_Delay + T_switch + T_prepare msec - The UE capability candidate values of T_prepare at least include 0 and X ms, e.g. X=7 - The non-zero candidate value is to accommodate the RF config generation latency when UE does not in advance generate the corresponding RF config for PRACH - RAN1 relies on RAN4 to verify the need for the above additional latency and, if so, the corresponding value and whether UE capability is needed - RAN1 relies on RAN4 to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH |

P4-X

Round 1

Views on other issues can be provided in the following table.

| Company | Input |
| --- | --- |
| Mod | Please share your views in this table. |
| Nokia | We think the following RAN1 issues are relevant and important to be discussed at some point (in this meeting if time allows or in the next meeting): - the timeline (gap) between the PDCCH order DCI and PRACH transmissions for a PRACH transmission to a LTM candidate cell as it may be a different scenario than a CA; this may require spec updates to section 8.1, 38.213. (this may require some coordination with RAN4) - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed as it may be a different scenario than a CA; this may require spec updates to section 7.5, 38.213. - prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission. |

Round 3

| Company | Input |
| --- | --- |
| Mod | In round-1 discussion, Nokia raised the following issues for discussion. If companies have any comments, questions or proposals on these issues, please feel free to provide your feedback in this table. - the timeline (gap) between the PDCCH order DCI and PRACH transmissions for a PRACH transmission to a LTM candidate cell as it may be a different scenario than a CA; this may require spec updates to section 8.1, 38.213. (this may require some coordination with RAN4) - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed as it may be a different scenario than a CA; this may require spec updates to section 7.5, 38.213. - prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission. |
| Ericsson | The issues seem valid. In our understanding, (1) is the same issue described in P4.2. |
| Futurewei | Issues (2), (3) can be further discussed in this group. |
| Nokia | Ericsson is right that (1) is same as P4.2. In this meeting at least we as a group should understand and agree that these issues are important and should be studied. Maybe we can have detailed discussion in the next meeting if don’t have enough time in this meeting. |
| QC | In our view, they are all valid issues. Suggest to discuss (1) together with P4.2, and separate discussions for (2) and (3) |
| vivo | We fine to discuss, as commented by others (1) need not be discussed separately from P4.2. For (2), (3) it would be better to see concrete proposals. |
| Samsung | Support to discuss. And we also think (1) can be discussed with P4.2 |
| Huawei, HiSilicon | For issue (1), we had similar feeling as Ericsson. is it similar as P4.2 For issue (2), maybe we need to first discussion whether it is allowed that UE is transmitted to different cells simultaneously on different carrier For issue (3), why source gNB will trigger the simultaneous UL transmission in the same time/frequency? I think such kind of scheduling should be avoided. |
| CMCC3 | For issue 1 and 2, we share the same view as majority. And for the issue 3, we have the similar idea as HW that, it can be avoided through gNB scheduling. Anyway, the UL transmission and PDCCH order RACH for TA are scheduled by the same gNB. It should be further clarify in which scenario it will happen. |
| Mod | Based on feedback of companies, at least we can identify the issues to be discussed/studied. And then, we may come up with more concrete proposals later. Proposal 4-3: for PDCCH-order based PRACH for candidate cell study the following issues: - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission |
| QC | Fine for the Proposal 4-3 |

- Issues to be discussed in online session

6.1 Round 1

Proposal 1-1: For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.

Proposal 1-3: When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. In inter-DU case, the following alternatives are listed for discussion and down-selection:

- Alt 1: RAR is received from serving cell

- Alt 2: RAR is received from candidate cell

Proposal 4-1:  For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA

- The maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability

Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command.

- Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption:

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

6.2 Round 2

Proposal 1-3-1: When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured

- Whether power ramping is performed or not is determined from PDCCH order

  - If power ramping is performed,

    - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order

    - power ramping is performed with configured or indicated power ramping-up value

Proposal 4-1:  For TA management in L1/L2 based mobility, when RAR is configured,

- the UE stores/remembers/maintains/handles the TA for at least one candidate cell

- storing/remembering/maintaining/handling the TA for more than one candidate cell is up to UE capability support at least one TA can be associated with more than one candidate cell and the maximal number of TA associated with candidate cell(s) which can be handled by UE is up to UE capability.

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption with the following update:

Working Assumption

From RAN1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

Proposal 2-1 (proposed conclusion): without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as in LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command.

- Note: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

6.3 Round 3

Proposal 1-2: For PDCCH ordered-RACH, if reception of RAR is not configured

- Whether power ramping is performed or not is determined from PDCCH order

  - If power ramping is performed,

    - Alt. 1: Xiaomi, Futurewei, Ericsson, FGI, Samsung, HW, QC, CMCC

      - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order

      - power ramping-up value is configured or indicated,

        - note: down-select one option later in this meeting or RAN1 #113 meeting

    - Alt. 2: OPPO

      - Absolute power is indicated

  - else, the power should be reset to its starting value, determined by open-loop power control

Proposal 2-1 (proposed conclusion): From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command.

- Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

- Note 2: The feasibility can be further concluded by RAN2

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption with the following update:

Working Assumption

From RAN1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

Proposal 4-3: for PDCCH-order based PRACH for candidate cell study the following issues:

- whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed

- whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission

Proposal 3-1:  Explicit association of TAG and candidate cell is provided as a part of configuration for candidate cell(s).

- References

- RP-222332	Revised WID on Further NR mobility enhancements		                MediaTek (Moderator)

- R1-2302316	On TA acquisition schemes of UE based RACH-less and early RACH with RAR	FUTUREWEI

- R1-2302369	Timing advance management to reduce latency	Huawei, HiSilicon

- R1-2302414	Timing advance management for L1/L2 Mobility	Ericsson

- R1-2302424	Enhancements on TA management to reduce latency	ZTE

- R1-2302505	Discussion on TA management for L1/L2 mobility	vivo

- R1-2302569	Discussions on Timing Advance Management	OPPO

- R1-2302620	Discussion on timing advance management to reduce latency	Spreadtrum Communications

- R1-2302731	Timing advancement management for L1L2 mobility	Lenovo

- R1-2302814	On Timing Advance Management	Intel Corporation

- R1-2302831	Timing Advance Management for L1/L2-triggered Mobility	Nokia, Nokia Shanghai Bell

- R1-2302869	Discussion on time advance management to reduce latency	CATT

- R1-2302967	Discussion on Timing advance management	xiaomi

- R1-2303083	Enhancements on TA management for mobility	LG Electronics

- R1-2303149	Candidate cell TA acquisition for NR L1/L2 mobility enhancement	Samsung

- R1-2303254	Discussion on timing advance management to reduce latency	CMCC

- R1-2303260	Discussion on Timing advance management to reduce latency	CAICT

- R1-2303290	Discussion on Timing advance management to reduce latency	KDDI Corporation

- R1-2303362	UL Timing management to reduce handover latency	MediaTek Inc.

- R1-2303381	Discussion on TA management for L1/L2 mobility	Transsion Holdings

- R1-2303456	Discussion on timing advance management to reduce latency	InterDigital, Inc.

- R1-2303504	Timing advance management for L1/L2 Mobility	Apple

- R1-2303519	Discussion on timing advance management to reduce latency	Google

- R1-2303611	TA management to reduce latency for L1/L2 based mobility	Qualcomm Incorporated

- R1-2303728	Timing advance enhancement for inter-cell mobility	NTT DOCOMO, INC.

- R1-2303782	Discussion on TA management to reduce latency	ITRI

- Previous agreements

RAN1 #110bis-e

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

•         RACH-based solutions

e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

•         RACH-less solutions

e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- the association is provided as a part of candidate cell(s) configuration

- the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

RAN1 #111

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

Agreement (Made in RAN1#110b-e)

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

    - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

RAN1 #112

Agreement

For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

Note: the detailed signalling is left to RAN2

Agreement

The PDCCH order from the source cell contains the indication of candidate cell.

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

Agreement

- For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported

Agreement

on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives.

- Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

  - Alt 2: UE autonomous Re-transmission of PRACH is allowed,

    - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

Agreement

If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell.

- The maximum number of TA values memorized by UE is a UE capability

- FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

Agreement

Whether RAR needs to be received is configured by RRC.

Agreement

study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured

- Whether gap between the DCI and PRACH longer than timeline defined in spec is needed

- Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec