3GPP TSG-RAN WG1 #112bis-e	R1-2304071

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0465 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the mapping of PDSCH-to-HARQ_feedback timing indicator field values in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator(CATT), Huawei, HiSilicon, ZTE, Sanechips, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following text proposal based on TS 38.213 version 16.6.0 was agreed in R1-2108595 in RAN1#106-e to clarify the PDSCH-to-HARQ_feedback timing indicator field values for a DCI triggering Type-3 HARQ-ACK codebook report without scheduling PDSCH in Clause 9.2.3. 9.2.3 UE procedure for reporting HARQ-ACK …… For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, scheduling a PDSCH reception or a SPS PDSCH release or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1. ……. However, the above text proposal is not correctly captured in TS 38.213 version 16.7.0 as below. To be specific, ‘, scheduling a PDSCH reception or a SPS PDSCH release’ is missing. 9.2.3 UE procedure for reporting HARQ-ACK …… For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0 or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, as defined in Table 9.2.3-1. ……. Based on current specification, it seems that the a DCI triggering Type-3 HARQ-ACK codebook without scheduling a PDSCH reception is excluded in the mapping of PDSCH-to-HARQ_feedback timing indicator field values for a DCI format, which is contradictory with the intention. In addition, following the same change logic as in R1-2108595, a DCI indicating SCell dormancy without scheduling a PDSCH reception is missing for mapping of PDSCH-to-HARQ_feedback timing indicator field values as well. Since for current specficiation, a DCI format is not restricted as a DCI scheduling a PDSCH reception, which means all kinds of DCIs that have a PDSCH-to-HARQ_feedback timing indicator field could be included by the description of “For a DCI format…. the PDSCH-to-HARQ_feedback timing indicator field values, if present”, the simple way to fix the issues is to just delete the description “or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4” so as to capture all kinds of DCI with assosicated HARQ-ACK feedback, which is future-proof as well. |
| Summary of change: |  | Clarify that a DCI triggering Type-3 HARQ-ACK codebook without scheduling a PDSCH reception and a DCI indicating SCell dormancy without scheduling a PDSCH reception are included in the mapping of PDSCH-to-HARQ_feedback timing indicator field value for a DCI format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCI formats for mapping of PDSCH-to-HARQ_feedback timing indicator field value is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis: This CR has isolated impact on mapping of PDSCH-to-HARQ_feedback timing indicator field values for a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. The CR reflects RAN1 common understanding and does not expect to impact gNB or UE implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, as defined in Table 9.2.3-1.

<Unchanged parts are omitted>