3GPP TSG-RAN WG1 #112bis-e	  R1-2304084

E-meeting,  17 – 26 April, 2023

Agenda Item:	7.2

Source:	Moderator (Huawei)

Title:	FLS#1 on the HARQ-ACK related issues for Rel-17 NR MBS

Document for:	Discussion and Decision

# Introduction

This FLS provides the discussion summary for the HARQ-ACK related issues for Rel-17 NR MBS. Section 2 provides overview for all submitted CRs and FL’s assessment. Section 3 will document the discussions for each individual issue.

[112bis-e-R17-MBS-01] Email discussion on Rel-17 MBS maintenance (MBS HARQ) by April 26 – Jinhuan (Huawei)

- Determine by April 18 which issues to handle in RAN1#112bis-e

- If needed, additional email threads will be used to organize the discussions

# Overview on all submitted CRs

Based on the submitted contributions, all the issues submitted are summarized in the following table with FL initial assessment. Companies can check each subsequent section for each issue in details and moderator CRs if provided.

Note: Companies may also indicate whether they think the proposed CR is essential or not essential, including the technical reason, as part of the preparation phase.

Table 1: Summary of HARQ-ACK related issues

| Issue# | Issue description | References | Views for preparation phase |
| --- | --- | --- | --- |
| 1-1 | HARQ CB for multicast SPS repetition: For the pseudo-code under type-1 CB, NPDSCHrepeat is not defined if neither pdsch-AggregationFactor-r16 is provided in SPS-Config nor pdsch-AggregationFactor is provided in PDSCH-Config for unicast SPS PDSCH. FL note: new issue. CR seems needed. Please check section 3.1 for details and draft moderator CR. | Samsung-CR-x3107 | Nokia: Ok to clarify the behavior. ZTE: OK to discuss. CATT: OK to discuss. However, as discussed in our Tdoc (R1-2302669), the description of pdsch-AggregationFactor for multicast SPS in TS 38.214 is not aligning with the corresponding parameter description in TS 38.331. Both of the issues on pdsch-AggregationFactor for multicast SPS should be discussed. Qualcomm: ok to discuss Samsung: Should be clarified – can be for an alignment CR (no technical correction). Ericsson: OK to discuss DCM: OK to discuss and probably included in alignment CR vivo: ok to discuss. MediaTek: ok to discuss ASUSTeK: ok to discuss |
| 1-2 | HARQ CB for ‘maxNrofCodeWords”: maxNrofCodeWordsScheduledByDCI can be independently configured for unicast and multicast. The impact to Type-1/2/3 HARQ-ACK codebook is addressed by setting the max as the number configured for unicast or for multicast or as the max of the number for unicast and multicast. FL note: new issue. The issue needs to be discussed. Please check section 3.2 for details. | CATT-Dis-x2665, CATT-CR-x2666 | Nokia: Ok to discuss the behavior. ZTE: OK to discuss. CATT: Support FL initial assessment. When both of maxNrofCodeWordsScheduledByDCI are configured for unicast and multicast, for multiplexing the HARQ-ACK feedback for multicast and unicast, the value used for Type-1/ 2/ 3 CB generation should be determined. Qualcomm: ok to discuss Samsung: OK to discuss if the proposed clarification is needed. If so, a more generic statement may be added - e.g. “…UE separately applies the procedures in this clause per G-RNTI for multicast or per G-CS-RNTI based on corresponding higher layer parameters, except the procedures for SPS PDSCHs and determines the Type-2 HARQ-ACK codebook …” - seems more appropriate as there are other RRC parameters, in addition to maxNrofCodeWordsScheduledByDCI, that are specific to multicast (e.g. search space sets). Ericsson: OK to discuss DCM: OK to discuss vivo: ok to discuss. MediaTek: ok to discuss |
| 1-3 | Condition for not providing Type-1 CB: in 9.1.2, it states if a Type-1 CB would not include any HARQ-ACK information for TB with enabled HARQ-ACK, then UE would not provide it. The case of ‘for SPS release’ is missing. FL note: CR may be needed. Please check section 3.3 for details and draft moderator CR. | CATT-CR-x2658 | Nokia: Ok to leave it to spec editor. ZTE: OK to discuss. CATT: Support FL initial assessment. Qualcomm: ok to discuss Samsung: OK to discuss. The suggested text needs an amendment as Type-1 is not supported for DCI indicating dormancy. Also, the scenarios for which the UE report HARQ-ACK for Type-1 are captured in the 5th paragraph of 9.1.2 - a possible clarification should be considered jointly with the suggested one in the 2nd paragraph of 9.1.2. Ericsson: OK to discuss DCM: OK to discuss vivo: ok to discuss. MediaTek: ok to discuss |
| 1-4 | Type-1 CB on PUSCH Clarify in 9.1.2.2 HARQ-ACK for data scheduled by DCI format 4_1 in PUSCH should mention ‘having enabled HARQ-ACK as described in clause 18’. FL note: CR seems ok. Please check section 3.4 for details and draft moderator CR. | ZTE-CR-x3388 | Nokia: Agree with FL. ZTE: OK to discuss. CATT: Agree with FL. It can be left to spec editor. Qualcomm: not needed. The spec for the fallback operation is clear, which includes the case that UE can receive DCI format 4_1 for a SPS PDSCH release. Samsung: We think the CR is not needed. There exists corresponding text in 9.1.2 of 38.213 (same paragraphs as commented for the above draft CR from CATT). Ericsson: OK to discuss DCM: Our feeling is similar to QC/Samsung, but OK to discuss. vivo: ok to discuss. MediaTek: ok to discuss |
| 1-5 | type1-Codebook-GenerationMode TS38.213 clause 9.1.2.1 taking ‘mode2’ as the default configuration by stating ‘not provided type1-Codebook-GenerationMode = 'mode1'’ is not aligned with the ASN.1 Need code ‘S’ FL note: CR seems needed. Please check section 3.5 for details and draft moderator CR. | Huawei-CR-x3801 | Nokia: Not sure. As this is RAN1 functionality, shouldn’t RAN2 change their spec? ZTE: may not be essential and the current spec is in line with RAN1 agreement. CATT: Agree with FL. Qualcomm: RAN1 spec seems clear, saying if fdmed-ReceptionMulticast is not configured and type1-Codebook-GenerationMode is not 'mode1', ‘mode2’ is used for Type-1 CB. Samsung: Not an essential correction – OK without a change or with the proposed change included in the alignment CR. Ericsson: agree with Nokia’s comment. DCM: seems to be non-essential correction vivo: agree with Qualcomm MediaTek: not an essential issue |
| 1-6 | TDRA for ‘fdmed-ReceptionMulticast’ How to determine TDRA when UE is provided ‘fdmed-ReceptionMulticast’ is missing for Type-1 CB in clause 9.1.2.1 FL note: new issue. CR seems needed. Please check section 3.6 for details and draft moderator CR. | CATT-CR-x2662 | Nokia: Agree with FL. ZTE: OK to discuss. CATT: Support FL initial assessment. Qualcomm: ok to discuss Samsung: Do not agree with the proposed CR. We think it is incorrect as all related text is applicable to the case that fdmed-ReceptionMulticast is not provided (the case that it is provided is separately addressed). An alignment CR may be considered to clarify that fdmed-ReceptionMulticast is not provided for the related text. Ericsson: OK to discuss DCM: OK to discuss vivo: ok to discuss. MediaTek: ok to discuss |
| 1-7 | K1 timing for DCI 1_0: Addressing the issue: For Type-1 CB multiplexing, for TDM-ed mode1, TDM-ed mode2, or FDM-ed multiplexing, K1 set could be the intersection, the union of k1 sets for unicast and multicast. The question is which k1 set is used to be taken intersection with {1, 2, 3, 4, 5, 6, 7, 8} to determine the k1 for DCI format 1_0. FL note: new issue. CR seems needed. Please check section 3.7 for details and draft moderator CR. | CATT-Dis-x2663, CATT-CR-x2664 | Nokia: Agree with FL. ZTE: OK to discuss. CATT: Support FL initial assessment. The K1 set used for intersection should be clarified for the case of TDM-ed mode1, TDM-ed mode2 and FDM-ed multiplexing. Qualcomm: ok to discuss Samsung: Do not agree with the proposed CR. We think it is incorrect (e.g. no need to consider FDMed unicast and multicast as K1s are allowed to be different). A similar statement for multicast as the current one for unicast (with DCI 4_1/4_2 instead of DCI 1_0/1_X) may be considered. Ericsson: OK to discuss DCM: OK to discuss vivo: ok to discuss. MediaTek: ok to discuss |
| 1-8 | Referred PUCCH resources The submitted CRs mainly address the issues which configuration is used to draw the PUCCH resources for multiplexing HARQ-ACK information, especially when parameters A is not configured but B is, or both parameters A and B are not configured. FL note: The moderator draft CR is provided based on the input for Cat2(x3636, x3389, and x3880). With it, companies can also comment any CR is needed for Cat1(x3633, x3634, and x3635). Please check section 3.8 for details and draft moderator CR. | MediaTek-CR-x3633, MediaTek-CR-x3634, MediaTek-CR-x3635, MediaTek-CR-x3636, ZTE-CR-x3389, Huawei-CR-x3880 | Nokia: Agree with x3633, x3634. We do not see the need for the rest and those could be left to spec editor to decide. ZTE: OK to discuss. CATT: OK to discuss. Qualcomm: ok to discuss Samsung: OK with the CR from x3389 for alignment CR (not OK for individual CR). Also, unclear why 3 draft CRs in x3633/x3634/x3635 were submitted on one same issue. Ericsson: OK to discuss DCM: OK to discuss, but redundant text should not be agreed vivo: ok to discuss. MediaTek: we think the the CRs of x3633, 3634,3635,3636 is needed. Otherwise, the UE’s behavior is not clear on how to determine the corresponding PUCCH resource. The reason why we submit the 4 separate CRs is that the four different scenarios are described. |
| 1-9 | Disabled HARQ-ACK not applied to the first PDSCH after activation: Disabling HARQ-ACK is not applied to the first PDSCH after SPS activation, i.e., always providing HARQ-ACK feedback. FL note: discussed earlier and proposed by the most number of companies again as in the last meeting. Companies can check section 3.9.1 for details and provide your views for the question wherein. | vivo-CR-x2462, CATT-CR-x2659, Lenovo-CR-x3317, ZTE-CR-x3386, Ericsson-CR-x3547, Qualcomm-CR-x3570, MediaTek-CR-x3637, DOCOMO-CR-x3696, Huawei-CR-x3793 | Nokia: Support the functionality. ZTE: OK to discuss. CATT: Support FL initial assessment. Qualcomm: ok to discuss Samsung: No need to re-discuss. It is not an essential correction, was discussed multiple times already, and in our opinion a change would be detrimental as it will significantly constraint multicast SPS deployments since a network must have HARQ-ACK resources for all UEs. Ericsson: OK to discuss DCM: Should be discussed vivo: ok to discuss. MediaTek: Ok to discuss Lenovo: OK to discuss Spreadtrum: Ok to discuss. Since we have agreed feedback for SPS release is mandatory for UE supporting SPS, we have not seen other additional complexity to support this. ASUSTeK: Ok to discuss. |
| 1-10 | Disabled HARQ-ACK not applied to SPS release: FL note: when discussing the controversial issue of disabling HARQ-ACK not applied to SPS activation, there is consensus HARQ-ACK feedback should be reported but proponents view that such point is not clear from the spec. Companies can check section 3.10.1 for details and provide your views for the question wherein. | Qualcomm-CR-x3569 Huawei-CR-x3876 | Nokia: Support the functionality. ZTE: OK to discuss. CATT: Support FL initial assessment. Qualcomm: ok to discuss Samsung: We think x3876 is incorrect (and related aspects are also captured elsewhere). The text from x3569 is not agreeable. If any clarification is needed based on the argument in the cover page of x3569, a statement such as “For a DCI format indicating SPS PDSCH release, the UE provides the associated HARQ-ACK information based on the first reporting mode” is appropriate. Ericsson: OK to discuss DCM: OK to discuss x3569. We do not support x3876; HARQ feedback for DCI format without PDSCH scheduling is not related to feedback-enabling/disabling signaling. No discussion is necessary for x3876. vivo: ok to discuss. Spreadtrum: Ok to have the discussion ASUSTeK: Ok to discuss. |
| 1-11 | Intra-UE HARQ-ACK multiplexing: Addressing: - Add the description of UE behavior for NACK-only PUCCH overlapping with the other PUCCH or PUSCH with different priorities.( ZTE-CR-x3391) - Add the description of UE behavior for PUCCH with HARQ-ACK information for MBS overlapping with another PUCCH or PUSCH with different priorities. (vivo-CR-x2463, ZTE-CR-x3390) - Add the description of UE behavior for PUCCH for multiplexing different priorities, for the case when provided a PUCCH-sSCell by pucch-sSCell and a periodic cell switching pattern, and for the case when provided sps-HARQ-Deferral for multicast SPS PDSCH (LGE-CR-x3423) FL note: Intra-UE multiplexing was introduced by URLLC in parallel with discussing URLLC, for which it was clarified intra-UE multiplexing was not considered when discussing MBS. Now both features are specified. The question is whether any CR is needed to clarify intra-UE multiplexing or combination with other features is supported or not supported for MBS. Companies can check section 3.11.1 for details and provide your views for the two questions wherein. | vivo-CR-x2463 ZTE-CR-x3390 ZTE-CR-x3391 LGE-CR-x3423 | Nokia : We do not need to enhance current MBS functionality with new Rel-17 functionality.It is clear from the specifications what is the UE behavior for MBS. ZTE : should be discussed. The UE behavior is not clear when combining the two features. For example, the NACK-only PUCCH, which is a bit different from ACK/NACK PUCCH discussed in URLLC. In addition, it is not clear Rel-17 URLLC feature can be combined with MBS. CATT : It is not an essential issue for Rel-17 MBS. Qualcomm: not clear why need CR. Samsung: We do not think any CR is needed. Current text in TS 38.213 does not differentiate unicast and multicast HARQ-ACK and the proposed CRs have a same effect. Ericsson : agree with other, this is not essential. DCM : Not necessary. If spec impact is really necessary, it should be concluded that MBS + intra-UE mux with different priorities is not supported in R17. vivo: the current spec only specify UE behavior for the case with same priority, UE behavior for the multiplexing case with different priorities is not capatured. MediaTek: we agree with other companies that this is not an essential issue Spreadtrum: Agree with DCM |
| 1-12 | Type-3 CB for Ucast/Mcast: Addressing: what does UE fill for Type-3 CB for TB indicated with disabled HARQ-ACK. - x3318: up to UE as in clause 18 - x3392: real ACK/NACK based on decoding. FL note: for the case of a TB indicated ‘disabled HARQ-ACK’ by ‘dci-enabler’, CR may be needed. Companies can check section 3.12.1 for details and provide your views for the two questions wherein. | Lenovo-CR-x3318, ZTE-CR-x3392 | Nokia: We do not think that it is esential to specify UE behavior here, but a note could be added. ZTE: This issue should be discussed. Otherwise, the UE behavior is not clear. CATT: OK to discuss, and we prefer x3318. Qualcomm: ok to discuss Samsung: OK to discuss. Prefer x3318 as it also addresses the case of disabled HARQ-ACK. Ericsson: OK to discuss DCM: No additional text will mean x3318 behavior. We do not see essentiality. vivo: ok to discuss. Lenovo: OK to discuss Spreadtrum: Ok to discuss |
| 1-13 | RRC parameters correction: Correct the parameter of ‘SPS-PUCCH-AN’ to ‘n1PUCCH-AN’ in clause 18. FL note: seems straightforward. Please check section 3.13.1 for details and draft moderator CR. | vivo-CR-x2458, Huawei-CR-x3877 | Nokia: Agree with FL. ZTE: Agree with FL. CATT: It can be left to spec editor. Qualcomm: could be alignment CR Samsung: OK for alignment CR (not OK for individual CR). DCM: OK for alignment CR vivo: Agree with FL. Spreadtrum: Ok for alignment CR ASUSTeK: Agree with FL |
| 1-14 | NACK-only for Table 18 Describing the PUCCH resources with PUCCH format 0/1 is for the second HARQ-ACK reporting mode. FL note: the entire paragraph describing the second reporting mode. The specification is clear without the draft CR. CR seems not needed. FL proposes this draft CR is not pursued as in section 3.14.1 | MediaTek-CR-x3631 | Nokia: Agree with FL. ZTE: Agree with FL. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: No need for a CR. It is clear the paragraph is for the second HARQ-ACK reporting mode. DCM: Same view with Samsung. vivo: Same view with Samsung. MediaTek: we also think the intention of the paragraph is for the second reporting mode. However, the current description is not clear. In this paragraph, there are four sentences, for the first two sentences, there is one description restriction, e.g., second HARQ-ACK reporting mode, but for the last two sentences, there are not corresponding description and will make the other people confused, e.g., not familiar with MBS spec/issue discussion. Since we have the similar understanding, maybe it is simple to add the corresponding restriction and make the issue clear. A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For the second HARQ-ACK reporting mode, for a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For the second HARQ-ACK reporting mode, for a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. Spreadtrum: not needed |
| 1-15 | n1PUCCH-AN for multiplexing n1PUCCH-AN should be added as the resource for multiplexing multicast SPS and multicast DG, unicast SPS and multicast DG, multicast SPS. FL note: To FL’s understanding, n1PUCCH-AN is configured for reporting HARQ-ACK information for one SPS configuration without multiplexing with other UCI, since n1PUCCH-AN at most carries two HARQ-ACK bits. CR seems not needed. FL proposes this draft CR is not pursued as in section 3.15 | MediaTek-CR-x3688 | Nokia: Agree with FL. ZTE: Agree with FL. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: No need for a CR. DCM: Agree with FL vivo: Agree with FL. MediaTek: From our understanding, n1PUCCH-AN also can be used for multiplexing case. For example, in legacy unicast procedure, the UE will choose n1PUCCH-AN if SPS-PUCCH-AN-List is not provided. Copy from 38.213 Spreadtrum: Agree with FL |
| 1-16 | Type-1 CB generation: When UE provided fdmed-ReceptionMulticast and when UE does not received any DCI for unicast or multicast, whether the Type-1 CB includes the HARQ-ACK bits for unicast or multicast. FL note: discussed in the last meeting. Same CR from proponent. No CR input from other companies. FL proposes this draft CR is not pursued as in section 3.16.1. | ZTE-CR-x3387 | Nokia: Agree with FL. ZTE: This issue was discussed in the last meeting without any conclusion. It should be clarified. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: Support the CR in principle. As the issue is same with the case of two coresetPoolIndex in the previous paragraph, our suggestion would be to postpone and consider both cases jointly next time with both Rel-16 and Rel-17 draft CRs. DCM: The same mechanism with the case of two coresetPoolIndex should be used. If CR for the case is agreed, we can have the same correction for MBS can be agreed. vivo: Agree with FL. |
| 1-17 | PTP retransmission: The retransmission of SPS when configured with NACK only is only over PTM FL Note: same case as DG. As clarified in the last meeting. The current spec could allow PTP retransmission for the case of NACK-only transformed into ACK/NACK. However, if using Tale 18 for NACK-only, PTM is the only practical retransmission scheme. Seems No CR needed. FL proposes this draft CR is not pursued as in section 3.17.1. | Ericsson-CR-x3546 | Nokia: Agree with FL. ZTE: Agree with FL. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: Not an essential correction. Also, although theoretically possible, it will not have practical value as there is no point for a NW to configure. DCM : Agree with FL. vivo: Agree with FL. |
| 1-18 | Overlapping SR+NACK-PUCCH+other PUCCH/PUSCH FL note: discussed in earlier meetings. Since multiplexing NACK-only and SR is not supported as agreed, then the key issue is whether support multiplexing NACK-only, SR and others. The current spec does not state such case. In FL’s understanding, such multiplexing is not supported and UE behavior by default is dropping SR when there is positive SR. FL proposes this draft CR is not pursued as in section 3.18.1 | vivo-Dis-x2460, vivo-CR-x2461, vivo-CR-x2459, Qualcomm-Dis-x3568 ZTE-CR-x2766 Huawei-Dis-x3792 | Nokia: Ok to pursue the CR. ZTE: OK to discuss CATT: Agree with FL. Qualcomm: Agree with FL Samsung: OK with the draft CR in x2459/x2766 for clarity of specifications that Rel-15 HARQ-ACK/SR/CSI multiplexing remains applicable. DCM: If this CR is discussed, NACK-only + SR should also be discussed. vivo: draft CR in x2459/x2766 should be discussed ath least. |
| 1-19 | NACK-only mode2 for >1 G-RNTI: Support NACK-only mode2 for more than one G-RNTI/G-CS-RNTI. FL note: discussed many times with no consensus. FL proposes this draft CR is not pursued as in section 3.19.1. | DOCOMO-CR-x3695, Ericsson-CR-x3545, | Nokia: Support the functionality as it requries minimal changes from the specification perspective.. ZTE: Agree with FL. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: OK with the draft CRs but, as noted, they were already discussed several times but are not an essential correction and were not agreed. However, we would request the moderator to apply a uniform handling of proposals for such cases as the situation in 1-19 was essentially same as in 1-9 but the suggestions are different. Ericsson: we support the CR. DCM: support this CR. We have same comment with Samsung; why 1-9 is suggested for discusión while why 1-19 is not suggested for discusión should be clarified. Double standard is not preferred. vivo: Agree with FL. |
| 1-20 | Same ‘dci-enabler’ indication: A UE does not expect to be receive different indications in multicast DCIs scheduling multicast reception with the same uplink feedback slot. FL note: this issue was discussed earlier without consensus. FL proposes this draft CR is not pursued as in section 3.20.1. | Ericsson-CR-x3548 | Nokia : Agree with FL. ZTE: Agree with FL. CATT: Agree with FL. Qualcomm: Agree with FL Samsung: No need for a CR. There is nothing that the UE cares about. DCM: Agree with FL. vivo: Agree with FL. |

## Conclusions from preparation phase

Issue (1-1) HARQ CB for multicast SPS repetition

Discuss for alignment CR including addressing the preparation phase comments to improve the formulation of the incoming draft CR, but start the discussion only after a conclusion on [112bis-e-AI7.1-09].

Issue (1-2) HARQ CB for ‘maxNrofCodeWords”

Correction seems needed. Discuss at RAN1#112bis-e aiming for a draft CR/CR addressing the preparation phase comments to improve the formulation of the incoming draft CR.

Issue (1-3) Condition for not providing Type-1 CB

Correction seems needed. Discuss at RAN1#112bis-e aiming for a draft CR/CR addressing the preparation phase comments to improve the formulation of the incoming draft CR.

Issue (1-4) Type-1 CB on PUSCH

Continue discussing whether this issue is an essential correction, and if it is aim for a draft CR/CR.

Issue (1-5) Type1-Codebook-GenerationMode

Continue discussion potentially for alignment CR.

Issue (1-6) TDRA for ‘fdmed-ReceptionMulticast’

Discuss whether a draft CR/CR is needed, including addressing the preparation phase comments to improve the formulation of the incoming draft CR.

Issue (1-7) K1 timing for DCI 1_0

Correction seems needed. Discuss at RAN1#112bis-e aiming for a draft CR/CR addressing the preparation phase comments to improve the formulation of the incoming draft CR.

Issue (1-8) Referred PUCCH resources

Discuss and refine the formulation of a potential draft CR/CR.

Issue (1-9) Disabled HARQ-ACK not applied to SPS activation

No consensus to change the specs. The common understanding in RAN1 on the current specifications is that disabling HARQ-ACK (when configured) applies to the first PDSCH after SPS activation.

Issue (1-10) Disabled HARQ-ACK not applied to SPS release

Correction seems needed. Discuss at RAN1#112bis-e aiming for a draft CR (and final CR) addressing the preparation phase comments to improve the formulation of the incoming draft CR in R1-2303569.

Issue (1-11) Intra-UE HARQ-ACK multiplexing

Majority think it is not an essential correction but adding a late functionality. This issue is not further pursued in Rel-17.

Issue (1-12) Type3 CB for Ucast/Mcast

Discuss to conclude on whether a CR is needed or whether the UE behaviour is already clear in the specification.

Issue (1-13) Editorial correction

Draft CRs in R1-2302458 and R1-2303877 (which result in the same final text) are endorsed for the editor’s alignment CR for TS38.213.

Issue (1-14) NACK-only for Table 18

Not an essential correction, RAN1 specification is already clear. This issue is not pursued in Rel-17.

Issue (1-15) n1PUCCH-AN for multiplexing

Not an essential correction, RAN1 specification is already clear. This issue is not pursued in Rel-17.

Issue (1-16) Type-1 CB generation

This issue was already discussed at RAN1#112 (without conclusion) and still only proposed by a single company. The majority of companies still don’t consider this as an essential correction, so the issue is no longer pursued in Rel-17. It is concluded that this is aligned with the fact that such behavior is not supported for two coresetPoolIndex case.

Issue (1-17) PTP retransmission

Not an essential correction. This issue is not pursued in Rel-17.

Issue (1-18) SR+NACK-PUCCH+other PUCCH/PUSCH

Not an essential correction. It is concluded that multiplexing NACK-only, SR and other contents in PUCCH is not supported in Rel-17.

Issue (1-19) NACK-only mode2 for >1 G-RNTI

Not an essential correction. It is concluded that NACK-only mode2 for more than one G-RNTI/G-CS-RNTI is not supported in Rel-17.

Issue (1-20) Same ‘dci-enabler’ indication

Not an essential correction. This issue is not pursued in Rel-17.

# Discussion for each individual issue

## (1-1)HARQ CB for multicast SPS repetition

| Samsung-CR-x3107 | #### 9.1.2 Type-1 HARQ-ACK codebook determination - *** Unchanged text is omitted *** In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell - while c<NcellsDL - Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-Config; otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1, for multicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config; otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; - end while |
| --- | --- |

### Round-1(CR)

FL’s analysis:

The draft corrects RRC parameters and also address the cases when NPDSCHrepeat is not provided for unicast and also for multicast.

The moderator draft CR is provided based on the input.

#### Proposal 3.1.1

The draft CR in Moderator draft CR on issue 1-1_v000_Mod  is endorsed.

Company views:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Samsung |  | Support – can be either an individual CR or an alignment CR. If an individual CR, “Samsung” should be the first company name as that is not a “Moderator/Huawei” draft CR. |  |
| vivo |  | It could be counted as alignment CR. |  |
| FL response |  | To Samsung, The draft CR x3107 was modified because 1. The ‘otherwise, if NPDSCHrepeat is not provided’ is not quite clear to me though I understand the intention, so I changed it to ‘or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, NPDSCHrepeat=1,’ as in the moderator CR. 2. The consequences if not approved from x3107 was refined with more details. With these, I changed it to moderator CR. Of course, these are improvements from moderator perspective. If companies disagree with moderator’s modification and prefer the original version x3107, it could be surely endorsed as individual draft CR. |  |

## (1-2)HARQ CB for ‘maxNrofCodeWords”

| CATT-Dis-x2665, CATT-CR-x2666 | #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: <Unchanged Text Omitted> If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats - a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and - a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U with applying the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and by setting NcellsDL=NcellsDL,M with applying the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits. If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R, and for each of the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast as the value of maxNrofCodeWordsScheduledByDCI to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook. If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI and is not provided type1-Codebook-Generation-Mode = 'mode1' and is not provided fdmed-ReceptionMulticast, the UE generates a Type-1 HARQ-ACK codebook by applying the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast in the following pseudo-code. If timeDomainHARQ-BundlingType1 is provided - set RT'=R - set RT to the set of row indexes that include the last SLIV of each row of set RT' <Unchanged Text Omitted> <Unchanged Text Omitted> #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs for multicast or G-CS-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause per G-RNTI for multicast or per G-CS-RNTI with applying maxNrofCodeWordsScheduledByDCI configured in pdsch-ConfigMulticast except the procedures for SPS PDSCHs and applies the procedures in this clause with applying maxNrofCodeWordsScheduledByDCI configured in pdsch-Config for unicast DCI formats, and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats excluding the unicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-CS-RNTI values excluding the multicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for unicast and multicast SPS PDSCH receptions. A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on - PDSCH-to-HARQ_feedback timing indicator field values, or a dl-DataToUL-ACK, dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17 value if the PDSCH-to-HARQ_feedback timing indicator field is not present in a DCI format, for PUCCH transmission with HARQ-ACK information in slot n, as described in clause 9.2.3, in response to PDSCH receptions, or in response to a DCI format having associated HARQ-ACK information without scheduling PDSCH reception - slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided. <Unchanged Text Omitted> <Unchanged Text Omitted> ### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated numbers of HARQ processes NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero. Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI which is configured in PDSCH-config or maximum parameter value configured in PDSCH-config and in PDSCH-configMulticast if configured for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1 <Unchanged Text Omitted> |
| --- | --- |

### Round-1(Question+CR)

FL’s analysis:

The parameter maxNrofCodeWordsScheduledByDCI can be independently configured for unicast and multicast. When generating Type-1/2/3 HARQ-ACK codebook for multiplexing HARQ-ACK information for unicast and multicast, which value of maxNrofCodeWordsScheduledByDCI should be taken is the issue intended to be addressed, the changes from the draft CR x2666 imply:

- For Type-1 TDM-ed mode1: the max of {maxNrofCodeWordsScheduledByDCI for unicast, maxNrofCodeWordsScheduledByDCI for multicast} for the first K1,UM set, maxNrofCodeWordsScheduledByDCI for unicast for the second K1,U\M set, maxNrofCodeWordsScheduledByDCI for multicast for the third K1,M\U set

- For Type-1 TDM-ed mode2: the max of {maxNrofCodeWordsScheduledByDCI for unicast, maxNrofCodeWordsScheduledByDCI for multicast}

- For Type-1 FDM-ed: taking maxNrofCodeWordsScheduledByDCI separately from unicast for unicast subcodebook and from multicast for multicast subcodebook

- For Type-2 codebook: taking maxNrofCodeWordsScheduledByDCI separately from unicast for unicast subcodebook and from multicast for multicast subcodebook

- For Type-3 codebook: the max of {maxNrofCodeWordsScheduledByDCI for unicast, maxNrofCodeWordsScheduledByDCI for multicast}

Before diving into the change details, companies can comment the high level principle regarding the draft CR from proponent as asked in the subbullet for the following proposal:

#### Question

Based on the draft CR in x2666,

- Do you agree with the issues to be addressed?

- Do you agree with the changes made in the CR?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | If a clarification is needed, prefer a generic statement - e.g. “…UE separately applies the procedures in this clause per G-RNTI for multicast or per G-CS-RNTI based on corresponding higher layer parameters, except the procedures for SPS PDSCHs and determines the Type-2 HARQ-ACK codebook …” as there are other RRC parameters, in addition to maxNrofCodeWordsScheduledByDCI, specific to multicast (e.g. search space sets). A possible CR should be an alignment CR as only a clarification is intended. |
| vivo | We agree to address the issue. maxNrofCodeWordsScheduledByDCI for multicast would not be larger than that for unicast, the configuration for unicast can always be applied for type 1 and type 3. So, it can be no change for type 1 and type 3 CB case. For type 2, CB for unicast and multicast are separately generated, it is ok to take maxNrofCodeWordsScheduledByDCI separately from unicast for unicast subcodebook and from multicast for multicast subcodebook |

## (1-3)Condition for not providing Type-1 CB

| CATT-CR-x2658 | ### 9.1.2 Type-1 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. In clauses 9.1.2, 9.1.2.1, and 9.1.2.2, if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static for only one of unicast or multicast HARQ-ACK codebook, the Type-1 HARQ-ACK codebook is generated considering only one of respective unicast or multicast configurations for PDSCH receptions or for PDCCH monitoring for detection of DCI formats. If a Type-1 HARQ-ACK codebook would not include any HARQ-ACK information for transport blocks with enabled HARQ-ACK information or HARQ-ACK information for DCI formats without scheduling PDSCH reception, the UE does not provide the Type-1 HARQ-ACK codebook and does not transmit a corresponding PUCCH. |
| --- | --- |

### Round-1(CR)

FL’s analysis:

In TS38.213 Clause 9.1.2, it states if a Type-1 CB would not include any HARQ-ACK information for TB with enabled HARQ-ACK, then UE does not provide the Type-1 HARQ-ACK codebook and does not transmit a corresponding PUCCH it. The case of ‘for SPS release’ is missing.

The proponent draft CR is not in the latest CR format. The moderator draft CR is provided based on the input.

#### Proposal 3.3.1

The draft CR in Moderator draft CR on issue 1-3_v000_Mod  is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not agree with the proposed draft CR. The suggested text needs an amendment as Type-1 is not supported for DCI indicating dormancy. Also, the scenarios for which the UE report HARQ-ACK for Type-1 are captured in the 5th paragraph of 9.1.2 - a possible clarification for an alignment should be considered jointly with the suggested one in the 2nd paragraph of 9.1.2. |
| vivo | Ok to add SPS release case here. It should be ‘nor HARQ-ACK information for DCI formats without scheduling PDSCH reception’. |

## (1-4)Type-1 CB on PUSCH

| ZTE-CR-x3388 | #### 9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel < Unchanged parts are omitted > If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, and - is not provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, or - is provided pdsch-HARQ-ACK-Codebook = 'semi-static' only for one of unicast and multicast HARQ-ACK information the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a TCI state update, or only a PDSCH that is scheduled by a DCI format 1_0 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or scheduled by a DCI format 4_1 having enabled associated HARQ-ACK information report as described in clause 18 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2. < Unchanged parts are omitted > |
| --- | --- |

### Round-1(CR)

FL’s analysis:

The change seems ok especially considering the following paragraph in the same clause 9.1.2.2:

If a UE is provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH - for the first set SU of NcellsDL,U serving cells if a value VT-DAIUL,U of the DAI field associated with unicast HARQ-ACK information is VT-DAIUL,U=1 [5, TS 38.212] - for the second set SM of NcellsDL,M serving cells if a value VT-DAIUL,M of the DAI field associated with multicast HARQ-ACK information is VT-DAIUL,M=1 [5, TS 38.212] The UE does not generate unicast or multicast HARQ-ACK information for multiplexing in the PUSCH transmission when VT-DAIUL,U=0 or VT-DAIUL,M=0, respectively, unless the UE receives respectively - only a unicast or a multicast SPS PDSCH release, or - only unicast SPS PDSCH(s) or multicast SPS PDSCH(s) having enabled associated HARQ-ACK information reports as described in clause 18, or - only a TCI state update or a PDSCH that is scheduled by a DCI format 1_0 with a counter DAI field value of 1 on the PCell, or only a PDSCH that is scheduled by a DCI format 4_1 having enabled associated HARQ-ACK information report, as described in clause 18, and counter DAI field value of 1 on the PCell

The moderator draft CR is provided based on the input.

#### Proposal 3.4.1

The draft CR in Moderator draft CR on issue 1-4_v000_Mod  is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not agree with the proposed draft CR. There exists corresponding text in 9.1.2 of 38.213 (same paragraphs as commented for the above draft CR from CATT for issue 1-3). |
| vivo | ok |

## (1-5)type1-Codebook-GenerationMode

| Huawei-CR-x3801 | #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP on the primary cell or, if the PUCCH transmission is indicated by a DCI format to be on the PUCCH-sSCell as described in clause 9A, on a set of slot timing values K1 associated with the active UL BWP on the PUCCH-sSCell - If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 for serving cell c, or the active DL BWP for serving cell c is dormant BWP, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6 - If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 - If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If an inapplicable value in dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 is provided, the value is excluded from K1 - If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c - - if the UE is provided fdmed-ReceptionMulticast, K1 for multicast is provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-MulticastDCI-Format4-1 - - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4-1, K1 is provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - - else if the UE is provided type1-Codebook-GenerationMode = 'mode2', K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-MulticastDCI-Format4-1 - - if the UE is not provided dl-DataToUL-ACK-MulticastDCI-Format4-1, K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - - else if the UE is provided type1-Codebook-GenerationMode = 'mode1', the UE - - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by - - the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is provided type1-Codebook-GenerationMode = 'mode2', or, if any, for the first K1,UM set - - the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c for the second K1,U\M set, if any - - the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c for the third K1,M\U set, if any - if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 |
| --- | --- |

### Round-1(CR)

FL’s analysis:

From TS38.331:

MulticastConfig-r17 ::=                 SEQUENCE {

pdsch-HARQ-ACK-CodebookListMulticast-r17    SetupRelease { PDSCH-HARQ-ACK-CodebookList-r16}         OPTIONAL,   -- Need M

type1CodebookGenerationMode-r17             ENUMERATED { mode1, mode2}                              OPTIONAL    -- Need M

}

The need code is M, i.e., maintaining the configuration from last time. To keep backward compatibility, RAN1 specification needs to be aligned with TS38.331, i.e., mode2 will NOT be the default mode when UE is NOT provide ‘mode1’.

In addition, the RRC parameter also needs to be corrected to ‘type1CodebookGenerationMode’.

The moderator draft CR is provided based on the input.

#### Proposal 3.5.1

The draft CR in Moderator draft CR on issue 1-5_v000_Mod  is endorsed.

Company views:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Samsung |  | Do not support endorsing the draft CR. The draft CR is obviously for alignment. It is not strictly essential but OK with agreeing/capturing the proposal for the alignment CR. |  |
| FL response |  | This should be essential because RAN1 needs to be aligned with 331 regarding UE behaviour, RAN2 put need code M means no default mode2 as RAN1 understood. The Need code S is not supposed to be changed because otherwise it would be NBC issue. |  |

## (1-6) TDRA for ‘fdmed-ReceptionMulticast’

| CATT-CR-x2662 | #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by - the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is not provided type1-Codebook-GenerationMode = 'mode1' and is not provided fdmed-ReceptionMulticast, or, if any, for the first K1,UM set - the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c if the UE is provided fdmed-ReceptionMulticast, or, if any, for the second K1,U\M set. - the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is provided fdmed-ReceptionMulticast a, or, if any, for the third K1,M\U set. - if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 |
| --- | --- |

### Round-1(CR)

FL’s analysis:

For Type-1 codebook generation, how to determine the TDRA set for the case of UE provided fdmed-ReceptionMulticast is missing from the current specification, which is the intention for the proposed CR.

The moderator draft CR is provided based on the input.

#### Proposal 3.6.1

The draft CR in Moderator draft CR on issue 1-6_v000_Mod  is endorsed.

Company views:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Samsung |  | Do not agree with the proposed CR. We think it is incorrect as all related text is applicable to the case that fdmed-ReceptionMulticast is not provided (the case that it is provided is separately addressed). An alignment CR may be considered to clarify that fdmed-ReceptionMulticast is not provided for the related text. |  |
| vivo |  | ok |  |
| FL response |  | Moderator CR is the updated based on x2662 considering the change as discussed in issue 1-5, means if accept the change ‘is not provided type1-Codebook-GenerationMode = 'mode1'’ is changed to “is not provided type1-Codebook-GenerationMode = 'mode2', in which case, fdmed-ReceptionMulticast will not be provided. |  |

## (1-7) K1 timing for DCI 1_0

| CATT-Dis-x2663, CATT-CR-x2664 | #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP on the primary cell or, if the PUCCH transmission is indicated by a DCI format to be on the PUCCH-sSCell as described in clause 9A, on a set of slot timing values K1 associated with the active UL BWP on the PUCCH-sSCell - If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 for serving cell c, or the active DL BWP for serving cell c is dormant BWP, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6 - If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 - If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If an inapplicable value in dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 is provided, the value is excluded from K1 - If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c - if the UE is provided fdmed-ReceptionMulticast, K1 for multicast is provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-MulticastDCI-Format4-1 - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4-1, K1 is provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - else if the UE is not provided type1-Codebook-GenerationMode = 'mode1', K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-MulticastDCI-Format4-1 - if the UE is not provided dl-DataToUL-ACK-MulticastDCI-Format4-1, K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - else if the UE is provided type1-Codebook-GenerationMode = 'mode1', the UE - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM <Unchanged Text Omitted> A UE does not expect to detect a DCI format switching a DL BWP within N3 symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where N3 is defined in clause 9.2.3. If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6, and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell. If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c and is provided fdmed-ReceptionMulticast or is provided type1-Codebook-GenerationMode = 'mode1', the set of slot timing values provided by K1 refers to K1 set for unicast DCI formats except for DCI format 1_0; otherwise the the set of slot timing values provided by K1 refers to the union of K1 set for both of unicast DCI except for DCI format 1_0 and multicast DCI formats. |
| --- | --- |

### Round-1(CR)

FL’s analysis:

Following the procedure for generating Type-1 CB in clause 9.1.2.1, for UE provided type1-Codebook-GenerationMode = 'mode2', K1 is extended to additionally include the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-MulticastDCI-Format4-1 or K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}.

Then when determining the slot timing for DCI format 1_0 in the subsequent procedure as follows: this K1 set should be

- for the case of UE provided type1-Codebook-GenerationMode = 'mode2' (or not provided ‘mode1’ per the current spec), the K1 set extended as above paragraph stated. In other words, this K1 set includes the k1 values configured for unicast and also k1 values configured for multicast.

- for the case of UE provided fdmed-ReceptionMulticast or is provided type1-Codebook-GenerationMode = 'mode1', the k1 set configured for unicast DCI formats.

If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6, and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell.

The similar discussion happened in RAN1#109-e meeting for NR_ext_to_71GHz-Core, which ended up with the following conclusion:

Conclusion - If type-1 HARQ-ACK codebook is configured, the intersection of predefined K1 values for DCI 1_0 and the extended K1 set for DCI 1_1/1_2 is applicable to DCI 1_0. - The current specification already covers the behavior. Hence no specification change is necessary

With all this information, the intention of the draft CR should be acknowledged but the wording needs refinement.

The moderator draft CR is provided based on the input.

#### Proposal 3.7.1

The draft CR in Moderator draft CR on issue 1-7_v000_Mod  is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not agree with the proposed CR. We think it is incorrect (e.g. no need to consider FDMed unicast and multicast as K1s are allowed to be different). A similar statement for multicast as the current one for unicast (with DCI 4_1/4_2 instead of DCI 1_0/1_X) may be considered. |
| vivo | We think K_1 can always refersto K_1 set for unicast DCI formats except for DCI format 1_0. |

## (1-8)Referred PUCCH resources

| MediaTek-CR-x3633 | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** If the UE multiplexes in a PUCCH only multicast HARQ-ACK information of same priority that is according to both the first and second HARQ-ACK reporting modes, the last DCI format that the UE uses to determine the PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided, otherwise, from pucch-Config/pucch-ConfigurationList, as described in clause 9.2.3, is a last DCI format associated with multicast HARQ-ACK information that is according to the first HARQ-ACK reporting mode. *** Unchanged text is omitted *** |
| --- | --- |
| MediaTek-CR-x3634 | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** If a UE multiplexes in a PUCCH HARQ-ACK information of same priority associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource from pucch-Config/pucch-ConfigurationList, as described in clause 9.2.3, is a last unicast DCI format. *** Unchanged text is omitted *** |
| MediaTek-CR-x3635 | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH from pucch-ConfigMulticast1, if provided, otherwise, from pucch-Config to provide the HARQ-ACK information as described in clause 9.2.1 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. *** Unchanged text is omitted *** |
| MediaTek-CR-x3636 | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** If a UE is provided pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for each G-RNTI for multicast or G-CS-RNTI that the UE provides associated HARQ-ACK information according to the first HARQ-ACK reporting mode or the second HARQ-ACK reporting mode, respectively. For HARQ-ACK information associated only with the second HARQ-ACK reporting mode, when the UE is not provided moreThanOneNackOnlyMode and the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode, the UE determines a PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided; otherwise, the UE determines a PUCCH resource from pucch-Config/pucch-ConfigurationList. For HARQ-ACK information associated only with the first reporting mode, the UE determines a PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided; otherwise, the UE determines a PUCCH resource from pucch-Config/pucch-ConfigurationList. *** Unchanged text is omitted *** |
| ZTE-CR-x3389 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.1 or 9.2.3 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. <Unchanged parts are omitted> If a UE is provided pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for each G-RNTI for multicast or G-CS-RNTI that the UE provides associated HARQ-ACK information according to the first HARQ-ACK reporting mode or the second HARQ-ACK reporting mode, respectively; otherwise, the UE determines a PUCCH resource from pucch-ConfigurationList as described in clause 9.2.3. For HARQ-ACK information associated only with the second HARQ-ACK reporting mode, when the UE is not provided moreThanOneNackOnlyMode and the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode, the UE determines a PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided; otherwise, the UE determines a PUCCH resource from pucch-Config/pucch-ConfigurationList as described in clause 9.2.3. <Unchanged parts are omitted> |
| Huawei-CR-x3880 | # 18 Multicast Broadcast Services This clause is applicable only for PDCCH receptions, PDSCH receptions, and PUCCH transmissions for MBS on a serving cell. DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI scheduling PDSCH receptions are referred to as multicast DCI formats and the PDSCH receptions are referred to as multicast PDSCH receptions. DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast scheduling PDSCH receptions are referred to as broadcast DCI formats and the PDSCH receptions are referred to as broadcast PDSCH receptions. HARQ-ACK information associated with multicast DCI formats or multicast PDSCH receptions is referred to as multicast HARQ-ACK information. < Unchanged parts are omitted > For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.3 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. |

### Round-1(CR)

FL’s analysis:

The submitted CRs mainly address the issues which configuration is used to draw the PUCCH resources for multiplexing HARQ-ACK information, especially when parameters A is not configured but B is, or both parameters A and B are not configured.

All submitted CRs could fall in two categories:

- Cat1(x3633, x3634, and x3635): the PUCCH resources regarding the last DCI. It should be clear if it is last unicast DCI, PUCCH resources configured for unicast with clause 9.2.3 should be meant; if it is multicast DCI, PUCCH resources configured for multicast should be meant.

- Cat2(x3636, x3389, and x3880): PUCCH configurations for the first report mode and for the second report mode should be used for HARQ-ACK information associated only with the first reporting mode and HARQ-ACK information associated only with the second reporting mode, respectively. If not configured, the PUCCH configuration for unicast should be taken.

Moreover, if CR is made for Cat2 then no more CR is needed for Cat1.

The moderator draft CR is provided based on the input for Cat2(x3636, x3389, and x3880). With it, companies can also comment any CR is needed for Cat1(x3633, x3634, and x3635).

#### Proposal 3.8.1

The draft CR in Moderator draft CR on issue 1-8_v000_Mod  is endorsed.

With it, companies can also comment any CR is needed for Cat1(x3633, x3634, and x3635).

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not support endorsing an individual CR. OK with agreeing/capturing the proposal for the alignment CR. |
| vivo | For the first change, 9.2.3 is for the case UE is not provided SPS-PUCCH-AN-List and 9.2.1 is for the case a UE is provided SPS-PUCCH-AN-List. Both should be referred. It should be alignment CR |

## (1-9)Disabled HARQ-ACK not applied to SPS activation

| vivo-CR-x2462 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first SPS PDSCH reception after activation and mulitcast PDCCH for releasing SPS PDSCH, UE provides HARQ-ACK information as described in Clause 9.2.3 and 10.2 respectively. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or for a G-CS-RNTI associated with a multicast SPS PDSCH without PDCCH scheduling and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or for a G-CS-RNTI associated with a multicast SPS PDSCH without PDCCH scheduling, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. |
| --- | --- |
| CATT-CR-x2659 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions, except for the first SPS PDSCH reception after an activation of SPS PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled or PDSCH receptions activated by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212] , except for the first SPS PDSCH reception after an activation of SPS PDSCH receptions. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. A UE always provides HARQ-ACK information for the first SPS PDSCH reception after an activation of SPS PDSCH receptions, and HARQ-ACK information for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. < Unchanged parts are omitted > |
| Lenovo-CR-x3317 | # 18 Multicast Broadcast Services This clause is applicable only for PDCCH receptions, PDSCH receptions, and PUCCH transmissions for MBS on a serving cell. DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI scheduling PDSCH receptions are referred to as multicast DCI formats and the PDSCH receptions are referred to as multicast PDSCH receptions. DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast scheduling PDSCH receptions are referred to as broadcast DCI formats and the PDSCH receptions are referred to as broadcast PDSCH receptions. HARQ-ACK information associated with multicast DCI formats or multicast PDSCH receptions is referred to as multicast HARQ-ACK information. <Unchanged part omitted> In clauses referring to a higher layer parameter value provided by a first or second PUCCH-Config, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast PDCCH or PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by SPS-PUCCH-AN or SPS-PUCCH-AN-List, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast SPS PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by pdsch-HARQ-ACK-Codebook or pdsch-HARQ-ACK-CodebookList, when applicable a corresponding higher layer parameter value for HARQ-ACK codebooks associated with multicast HARQ-ACK information is provided as described in [12, TS 38.331]. A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1. The UE provides HARQ-ACK information with ACK value when the UE correctly decodes the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration or detects a DCI format indicating release of a SPS configuration, and provides HARQ-ACK information with NACK value when the UE does not correctly decode the first SPS PDSCH reception after activation of SPS PDSCH receptions for the SPS configuration. A UE can be configured by harq-FeedbackOptionMulticast, for a G-RNTI for multicast or for a G-CS-RNTI, to provide HARQ-ACK information for a transport block reception associated with the G-RNTI for multicast or with the G-CS-RNTI, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The UE determines a priority for a PUCCH transmission with multicast HARQ-ACK information according to any HARQ-ACK reporting mode as described in clause 9 for a PUCCH transmission with unicast HARQ-ACK information. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. or the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. <Unchanged part omitted> |
| ZTE-CR-x3386 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> A UE can be configured by harq-FeedbackOptionMulticast, for a G-RNTI for multicast or for a G-CS-RNTI, to provide HARQ-ACK information for a transport block reception associated with the G-RNTI for multicast or with the G-CS-RNTI, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The UE determines a priority for a PUCCH transmission with multicast HARQ-ACK information according to any HARQ-ACK reporting mode as described in clause 9 for a PUCCH transmission with unicast HARQ-ACK information. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. If the second HARQ-ACK reporting mode is configured for a G-CS-RNTI, the first HARQ-ACK reporting mode should be used for the first SPS PDSCH reception after activation of SPS PDSCH receptions, and for DCI format deactivating the SPS configuration associated with the G-CS-RNTI. <Unchanged parts are omitted> |
| Ericsson-CR-x3547 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE can be configured by harq-FeedbackOptionMulticast, for a G-RNTI or for a G-CS-RNTI, to provide HARQ-ACK information for a transport block reception associated with the G-RNTI or with the G-CS-RNTI, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The UE determines a priority for a PUCCH transmission with multicast HARQ-ACK information according to any HARQ-ACK reporting mode as described in clause 9 for a PUCCH transmission with unicast HARQ-ACK information. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. The First HARQ-ACK reporting mode is always applied to a transport block received in a PDSCH associated with a PDCCH scrambled with CS-RNTI or G-CS-RNTI, and to DCI formats indicating an SPS PDSCH release. < Unchanged parts are omitted > |
| Qualcomm-CR-x3570 | ### 9.1.2 Type-1 HARQ-ACK codebook determination ## *** Unchanged text is omitted *** If a UE is provided downlinkHARQ-FeedbackDisabled indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE reports a NACK value for a HARQ-ACK information bit corresponding to the transport block in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE reports NACK values for HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-1 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.2.1. If the UE is provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. ## *** Unchanged text is omitted *** ### 9.1.3 Type-2 HARQ-ACK codebook determination ## *** Unchanged text is omitted *** If a UE is provided downlinkHARQ-FeedbackDisabled indicating disabled HARQ-ACK information for a HARQ process associated with a transport block for PDCCH monitoring occasion m or for SPS PDSCH receptions on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.3.1. If the UE is provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. |
| MediaTek-CR-x3637, | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI associated with multicast SPS PDSCH without PDCCH scheduling and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI , the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI associated with multicast SPS PDSCH without PDCCH scheduling [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. If a UE is scheduled a multicast SPS PDSCH by PDCCH with CRC scrambled by G-CS-RNTI, the UE always provides HARQ-ACK information for the first multicast PDSCH reception after the multicast PDCCH activation. *** Unchanged text is omitted *** |
| DOCOMO-CR-x3696, | # 18 Multicast Broadcast Services - *** Unchanged text is omitted *** A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. If the UE is provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. - *** Unchanged text is omitted *** |
| Huawei-CR-x3793 | < Unchanged parts are omitted > A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If the UE is also provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers for the G-CS-RNTI a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. < Unchanged parts are omitted > |

### Round-1(question+CR)

FL’s analysis:

This issue is still the one of most interest from submitted papers (9 in total).

Technically, the view from the papers is that only ACK/NACK reporting is supported for SPS activation/deactivation as agreed, so the CR is needed given the current spec states the disabling HARQ-ACK is applied to PDSCH which in general includes the first PDSCH after activation.

Agreement: Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling. - FFS for SPS activation/deactivation. Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.

The counter argument from the last meeting is that the first PDSCH after activation is also PDSCH, so the HARQ-ACK information report can be disabled per the current specification but nothing is broken. The issue could be closed with such controversy.

In addition, as three companies point out in this meeting (x3570, x3696, and x3793) that for NTN one parameter of harq-feedbackEnablingforSPSactive = 'enabled' can specifically enable HARQ-ACK reporting for the first PDSCH after activation when ‘disabling HARQ-ACK’ turns off HARQ-ACK reporting for all SPS PDSCHs. MBS being configured with such parameter can be a compromise.

Before discussing the detailed CR, let’s answer the first question which option from the following proposal is preferred/agreeable to the group?

#### Question:

Which option is agreeable?

- Opt1: The disabling HARQ-ACK feedback is not applied to multicast SPS activation.

- Opt2: The disabling HARQ-ACK feedback is applied to multicast SPS activation. The parameter of harq-feedbackEnablingforSPSactive can be configured for multicast SPS.

- Note: a UE feature needs to be defined for option2.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | No need to discuss any option as the specifications are complete. The issue has already been discussed, it was not agreeable, it is not an essential correction and, in our opinion, it would be detrimental to a NW’s capability to deployment multicast SPS PDSCH. There are also other differences between NTN and MBS operation and RAN1/MBS made corresponding agreements when it was known that NTN would operate differently (those agreements were proposed/supported by the companies now raising that issue in x3570/x3696/x3793). |
| vivo | We prefer option 1. |

## (1-10)Disabled HARQ-ACK not applied to SPS release

| Qualcomm-CR-x3569 | - 18 Multicast Broadcast Services ## *** Unchanged text is omitted *** A UE can be configured by harq-FeedbackOptionMulticast, for a G-RNTI for multicast or for a G-CS-RNTI, to provide HARQ-ACK information for a transport block reception associated with the G-RNTI for multicast or with the G-CS-RNTI, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The UE determines a priority for a PUCCH transmission with multicast HARQ-ACK information according to any HARQ-ACK reporting mode as described in clause 9 for a PUCCH transmission with unicast HARQ-ACK information. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration. ## *** Unchanged text is omitted *** An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. For the DCI format associated with a G-CS-RNTI without scheduling a PDSCH reception, the UE provides the associated HARQ-ACK information based on the first reporting mode. A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. ## *** Unchanged text is omitted *** |
| --- | --- |
| Huawei-CR-x3876 | < Unchanged parts are omitted > A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions and for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. < Unchanged parts are omitted > |

### Round-1(question+CR)

FL’s analysis:

To FL’s understanding, the HARQ-ACK reporting mode is not restricted to PDSCH only but also applied to DCI formats having associated HARQ-ACK information without scheduling a PDSCH. Hence, no need to delete ‘for DCI format indicating an SPS release’ from the paragraph describing the first reporting mode and the second reporting mode. Instead, what is missing from the paragraph describing the function of ‘enabling/disabling HARQ-ACK’ by harq-FeedbackEnablerMulticast is for the ‘DCI format indicating an SPS release’.

#### Question:

Do you agree to take the draft CR in R1-2303876 as baseline or agreed?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not agree. We think x3876 is incorrect (and related aspects are also captured elsewhere). If any clarification is needed based on the argument in the cover page of x3569, a statement such as “For a DCI format indicating SPS PDSCH release, the UE provides the associated HARQ-ACK information based on the first reporting mode” is appropriate. |
| vivo | No. for SPS PDSCH release, disabling HARQ-ACK is not applicable. |

## (1-11)Intra-UE HARQ-ACK multiplexing

| vivo-CR-x2463 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs. If a UE determines that a first PUCCH transmission with multicast HARQ-ACK information that has a priority index overlaps in time domain with a second PUCCH transmission or a PUSCH transmission that have a different priority indexes, including repetitions if any, - if the UE is not provided uci-MuxWithDiffPrio, the UE cancels the PUCCH transmission or PUSCH transmission of priority index 0 following the procedures in clause 9. - if the UE is provided uci-MuxWithDiffPrio, the UE resolves the overlapping following the procedures in clause 9. <Unchanged parts are omitted> |
| --- | --- |
| ZTE-CR-x3390 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs. If a UE determines that a first PUCCH transmission with multicast HARQ-ACK information that has a priority index overlaps in the time domain with a second PUCCH transmission or a PUSCH transmission that have a different priority indexes, including repetitions if any, - if the UE is not provided uci-MuxWithDiffPrio, the UE cancels the PUCCH transmission or PUSCH transmission of priority index 0 following the procedures in clause 9. - if the UE is provided uci-MuxWithDiffPrio, the UE multiplexes UCI in the PUCCH transmission or PUSCH transmission following the procedures in clause 9, and determine the PUCCH resource following the procedures in clause 9.2.5.3. <Unchanged parts are omitted> |
| ZTE-CR-x3391 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> If a UE would multiplex second multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with first multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the second HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE would only transmit a first PUCCH with only positive SR and a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode, where the first and second PUCCHs would overlap in time in a slot and have same priority index, it is up to UE implementation for the UE to transmit either the first PUCCH or the second PUCCH. When a UE determines that a first PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode that has a priority index overlaps in the time domain with other PUCCHs or PUSCHs that has a different priority indexes, the UE follows the procedure for a first PUCCH with SR overlapping with a second PUCCH or PUSCH in the time domain, as described in clause 9, with replacing a first PUCCH with SR by a first PUCCH with HARQ-ACK information. <Unchanged parts are omitted> |
| LGE-CR-x3423 | <Unchanged parts are omitted> If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs. A UE is not required to multiplex in a PUCCH multicast HARQ-ACK information of a priority and unicast UCI of the priority if the UE is provided subslotLengthForPUCCH for PUCCH transmissions with unicast UCI of the priority. If a UE multiplexes in a PUCCH HARQ-ACK information of different priorities associated with unicast DCI formats and with multicast DCI formats according to the first HARQ-ACK reporting mode in a same PUCCH, the UE uses to determine the PUCCH resource with higher priority. If a UE is be provided a PUCCH-sSCell by pucch-sSCell and a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines PUCCH transmissions for multicast HARQ-ACK information according to the first HARQ-ACK reporting mode as currently specified in clause 9.A. If a UE is provided sps-HARQ-Deferral for multicast SPS PDSCH, the UE determines PUCCH transmissions for the HARQ-ACK information associated with multicast SPS PDSCH receptions according to the first HARQ-ACK reporting mode as currently specified in clause 9.2.5.4. < End > |

### Round-1(question and CR)

FL’s analysis:

Company views from the last meeting for reference:

| Samsung | No need - Clause 9 does not differentiate unicast and multicast for intra-UE multiplexing. Adding a general reference to Clause 9 for intra-UE multiplexing is OK (in an alignment CR) although not really needed. |
| --- | --- |
| vivo | No need. When it is ACK, UE does not transmit the PUCCH, there is no PUCCH transmission and no overlapping. |
| ZTE | Since physical priority can be configured for MBS, then when the HP channels and LP channels for MBS overlaps in the time domain, what the UE behavior is should be clarified. We think all the UE behavior defined in Rel-16/17 URLLC can be reused. However, in current spec, the UE behavior of intra-UE multiplexing does not cover the PUCCH for MBS. Therefore, the spec updates are needed to cover the PUCCH for MBS. For the NACK-only PUCCH for MBS, since it may or may not be transmitted by the UE, depending the status of HARQ-ACK information bits, it is similar as the SR transmission depending on the status of SR. Therefore, the UE behavior of intra-UE multiplexing involving SR can be reused for intra-UE multiplexing involving NACK-only PUCCH. |
| LG Electronics | We generally think that a CR is needed. |

It seems when UE supports intra-UE multiplexing and when UE is indicated or not indicated with uci-MuxWithDiffPrio, what is UE behaviour for HARQ-ACK reporting needs clarified somehow, although we kind of agreed that no need to discuss intra-UE multiplexing at the beginning of normative work since intra-UE multiplexing was discussed in parallel in another work item.

The same situation remains when discussing periodic cell switching pattern and sps-HARQ-Deferral for multicast SPS PDSCH as proposed in x3423.

#### Question 1:

Do you think CR is needed in Clause 18 regarding description for the case of UE configured or not configured with uci-MuxWithDiffPrio? Can the draft CR in x2469 be the baseline if a CR is needed?

#### Question 2:

Do you think CR is needed in Clause 18 regarding description for the case of UE configured pucch-sSCell and pucch-sSCellPattern, or configured sps-HARQ-Deferral for multicast SPS PDSCH? Can the draft CR in x3423 be the baseline if a CR is needed?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | A CR is not needed. Current text in TS 38.213 does not differentiate unicast and multicast HARQ-ACK and the proposed CRs have a same effect. |
| LG Electronics | We think that both CRs are needed to clarify the cases. |

## (1-12)Type3 CB for Ucast/Mcast

| Lenovo-CR-x3318 | ### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated numbers of HARQ processes NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero. If a UE would report HARQ-ACK information associated with a G-RNTI for multicast or a G-CS-RNTI with disabled HARQ-ACK information, as described in clause 18, a value of the HARQ-ACK information is a UE implementation choice. Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1 <Unchanged part omitted> |
| --- | --- |
| ZTE-CR-x3392 | ### 9.1.4 Type-3 HARQ-ACK codebook determination <Unchanged parts are omitted> Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1 Set c=0 – serving cell index in the set of serving cells Set h=0 – HARQ process number index in the set of numbers of HARQ processes Set t=0 – TB index Set g=0 – CBG index Set j=0 while c<NcellsDL while h<NHARQ,cDL if downlinkHARQ-FeedbackDisabled is not provided, or is provided and indicates enabled HARQ-ACK information for h, or HARQ-feedbackEnablingforSPSactive is provided and enabled and h corresponds to a transport block in a first SPS PDSCH reception after an activation of SPS PDSCH receptions if NDIHARQ=0 if NHARQ-ACK,cCBG/TB,max>0 while t<NTB,cDL while g<NHARQ-ACK,cCBG/TB,max ojACK= HARQ-ACK information bit for CBG g of TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c, if any; else, ojACK=0 j=j+1 g=g+1 end while ojACK= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c, if any; else, ojACK=0 g=0 j=j+1 t=t+1 end while else while t<NTB,cDL ojACK= HARQ-ACK information bit for TB t for HARQ process index h in the set of numbers of HARQ processes of serving cell c, if any; else, ojACK=0 j=j+1 ojACK= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c, if any; else, ojACK=0 j=j+1 t=t+1 end while end if t=0 else if NHARQ-ACK,cCBG/TB,max>0 while t<NTB,cDL if UE has obtained HARQ-ACK information for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception, and the DCI or harq-FeedbackEnablerMulticast indicates enabled HARQ feedback for TB t for HARQ process number index h for multicast only while g<NHARQ-ACK,cCBG/TB,max ojACK= HARQ-ACK information bit for CBG g of TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c j=j+1 g=g+1 end while else while g<NHARQ-ACK,cCBG/TB,max ojACK=NACK j=j+1 g=g+1 end while end if g=0 t=t+1 end while else while t<NTB,cDL if UE has obtained HARQ-ACK information for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception, and the DCI or harq-FeedbackEnablerMulticast indicates enabled HARQ feedback for TB t for HARQ process number index h for multicast only if harq-ACK-SpatialBundlingPUCCH is not provided ojACK= HARQ-ACK information bit for TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c else ojACK= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process number index h in the set of numbers of HARQ processes of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block end if j=j+1 t=t+1 else ojACK=NACK j=j+1 t=t+1 end if end while end if t=0 end if end if h=h+1 end while h=0 c=c+1 end while If NTB,cDL>1, when a UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block. <Unchanged parts are omitted> |

### Round-1(question and CR)

FL’s analysis:

For Type-3 HARQ-ACK CB, what UE should report when a TB is indicated with ‘disabled HARQ-ACK feedback’ was not concluded yet. Now

- Lenovo CR views it is up to UE implementation to report ACK or NACK.

- ZTE CR views UE should report the real decoding result.

#### Question 1:

Do you think CR is needed in Clause 9.1.4 for Type-3 CB generation for the case of a TB indicated with disabled HARQ-ACK by ‘dci-enabler’?

#### Question 2:

If yes for Q1, which CR should be taken as the starting point? x3318 or x3392?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK to discuss – prefer the CR from x3318 as it also addresses the case of disabled HARQ-ACK. |

## (1-13)Editorial correction

| vivo-CR-x2458 | # 18 Multicast Broadcast Services In clauses referring to a higher layer parameter value provided by a first or second PUCCH-Config, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast PDCCH or PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by n1PUCCH-AN or SPS-PUCCH-AN-List, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast SPS PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by pdsch-HARQ-ACK-Codebook or pdsch-HARQ-ACK-CodebookList, when applicable a corresponding higher layer parameter value for HARQ-ACK codebooks associated with multicast HARQ-ACK information is provided as described in [12, TS 38.331]. |
| --- | --- |
| Huawei-CR-x3877 | In clauses referring to a higher layer parameter value provided by a first or second PUCCH-Config, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast PDCCH or PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by n1PUCCH-AN or SPS-PUCCH-AN-List, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast SPS PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by pdsch-HARQ-ACK-Codebook or pdsch-HARQ-ACK-CodebookList, when applicable a corresponding higher layer parameter value for HARQ-ACK codebooks associated with multicast HARQ-ACK information is provided as described in [12, TS 38.331]. |

### Round-1(CR)

FL’s analysis:

RRC parameter correction. Looks straightforward.

Companies can comment whether the moderator CR considering both sources is agreeable.

#### Draft CR 3.13.1

The draft CR in Moderator draft CR on issue 1-13_v000_Mod  is endorsed.

Company views:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Samsung |  | Not OK endorsing “Moderator draft CR …”. OK with agreeing the change from x2458/x3877 for the alignment CR. |  |
| FL response |  | Since we have two individual CRs taking about the same change, does ‘no moderator CR’ from Samsung’s comment mean the CRs would be individually endorsed? |  |

## (1-14)NACK-only for Table 18

| MediaTek-CR-x3631 | # 18 Multicast Broadcast Services *** Unchanged text is omitted *** A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For the second HARQ-ACK reporting mode, for a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For the second HARQ-ACK reporting mode, for a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. *** Unchanged text is omitted *** |
| --- | --- |

### Round-1(Conclude not pursued)

FL’s analysis:

The entire paragraph describes the case of UE indicated the second HARQ-ACK reporting mode as well as the resources to be used for the HARQ-ACK report. To FL’s understanding, the current specification is clear without ambiguity, so such CR seems not needed.

FL suggests closing the issue by not pursue the draft CR.

#### Proposal 3.14.1

The draft CR in R1-2303631 is not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | No need for a CR. It is clear the paragraph is for the second HARQ-ACK reporting mode. |

## (1-15)n1PUCCH-AN for multiplexing

| MediaTek-CR-x3688 | - 18 Multicast Broadcast Services *** Unchanged text is omitted *** If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and the first and second HARQ-ACK information are indicated by harq-FeedbackOptionMulticast different HARQ-ACK reporting modes, the UE determines the PUCCH resource based on: - sps-PUCCH-AN-ListMulticast, or n1PUCCH-AN/sps-PUCCH-AN-List if sps-PUCCH-AN-ListMulticast is not provided, if the first HARQ-ACK information is according to the first HARQ-ACK reporting mode or - the last multicast DCI format, as described in clause 9.2.3, if the second HARQ-ACK information is according to the first HARQ-ACK reporting mode. If a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from - if provided, n1PUCCH-AN/SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1; - else, if provided, PUCCH-Config/PUCCH-ConfigurationList for multicast PDSCH receptions; - else, PUCCH-Config/PUCCH-ConfigurationList for unicast PDSCH receptions. If a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast SPS PDSCH receptions and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1. If a UE multiplexes in a PUCCH only HARQ-ACK information associated with multicast SPS PDSCHs receptions of same priority that is according to the first HARQ-ACK reporting mode and is not provided sps-PUCCH-AN-ListMulticast, the UE determines the PUCCH resource from the n1PUCCH-AN/sps-PUCCH-AN-List provided for unicast SPS PDSCH reception as described in clause 9.2.1. *** Unchanged text is omitted *** |
| --- | --- |

### Round-1(Conclude not pursued)

FL’s analysis:

n1PUCCH-AN is configured for reporting HARQ-ACK information for one SPS configuration without multiplexing with other UCI, since n1PUCCH-AN at most carries two HARQ-ACK bits, so the draft CR does not seem correct.

FL suggests closing the issue by not pursue the draft CR.

#### Proposal 3.15.1

The draft CR in R1-2303688 is not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | No need for a CR. |
| LG Electronics | Ok with FL’s suggestion |

## (1-16)Type-1 CB generation

| ZTE-CR-x3387 | #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats - a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and - a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates - a Type-1 HARQ-ACK codebook for the set SU of serving cells by setting NcellsDL=NcellsDL,U in the following pseudo-code, if the unicast DCI format is only received from the set SU of serving cells, or - a Type-1 HARQ-ACK codebook for the set SM of serving cells by setting NcellsDL=NcellsDL,M in the following pseudo-code, if the multicast DCI format is only received from the set SM of serving cells, or - a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code, if both the DCI formats 1_0/1_1/1_2 and the DCI format 4_1/4_2 is received from the set SU and the set SM of serving cells, respectively. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits. <Unchanged parts are omitted> |
| --- | --- |

### Round-1(Conclude not pursued)

FL’s analysis:

This issue was discussed in the last meeting. As concluded in offline in the last meeting:

The current specification implies the same Type-1 codebook generation for mTRP or FDMed codebook multiplexing for unicast and multicast. The issue expected to be fixed is what if UE does not detect any DCI from either mTRP, for unicast or multicast.

Alt1: keep the current spec and no CR, meaning though no DCI detected, the HARQ-ACK for that CC will be provided.

Alt2: Change the Type-1 codebook generation as suggested by the proposal, and make the change for both multiplexing HARQ-ACK for unicast and multicast, and for mTRP.

Alt3: only change made for unicast&multicast.

From the submitted papers, no other company proposes CR to address the issue. Based on the current situation, FL suggests closing the issue by not pursue the CR.

#### Proposal 3.16.1

The draft CR in R1-2303387 is not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | We support the CR in principle. As the issue is same with the case of two coresetPoolIndex in the previous paragraph, our suggestion would be to postpone and consider both cases jointly next time with both Rel-16 and Rel-17 draft CRs. The proposal from the FL need not be agreed or can be agreed with “in RAN1#112bis-e” at the end. |

## (1-17) PTP retransmission

| Ericsson-CR-x3546 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > A PDSCH reception providing an initial transmission of a transport block is scheduled only by a multicast DCI format. For the first HARQ-ACK reporting mode, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a multicast DCI format using a same G-RNTI as the G-RNTI of the initial transmission of the transport block, or by a unicast DCI format using a C-RNTI [6, TS 38.214]. An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. For the second HARQ-ACK reporting mode and when the UE is indicated moreThanOneNackOnlyMode, for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. < Unchanged parts are omitted > |
| --- | --- |

### Round-1(Conclude not pursued)

FL’s analysis:

As clarified in the last meeting that the PTP retransmission can be supported if the NACK-only is transformed into ACK/NACK, so that network is aware which UE reports ACK or NACK, but such clarification does not need CR. The reason is the statement saying ‘for the first reporting mode, xxx’ and when NACK-only is transformed into ACK/NACK, the reporting mode then is the first reporting mode.

The draft CR from proponent is specifically for SPS with the second reporting mode and is configured moreThanOneNackOnlyMode, then the retransmission should be scheduled by a DCI format scrambled by G-CS-RNTI, i.e., PTM retransmission. When UE is provided moreThanOneNackOnlyMode for the second reporting mode, UE shares the same resource for reporting HARQ-ACK according Table 18. In such a case, PTM should be the practical retransmission scheme. No CR seems needed.

#### Proposal 3.17.1

The draft CR in R1-2303546 is not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Not an essential correction. Also, although theoretically possible, it will not have practical value as there is no point for a NW to configure separate/non-overlapping PUCCH resources for HARQ-ACK to each UE and “NACK-only”. |

## (1-18)SR+NACK-PUCCH+other PUCCH/PUSCH

| vivo-Dis-x2460 vivo-CR-x2461 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex second multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with first multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the second HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE would only transmit a first PUCCH with only positive SR and a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode, where the first and second PUCCHs would overlap in time in a slot and have same priority index, it is up to UE implementation for the UE to transmit either the first PUCCH or the second PUCCH. If a UE would transmit a first PUCCH with only positive SR and a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode, where the first and second PUCCHs would overlap in time in a slot and have same priority index, if - either the first PUCCH or the second PUCCH would overlap in time with a third PUCCH with the same priority and the three PUCCHs are not in a group overlapping PUCCHs as discribled in Cluase 9.2.5, or - the seond PUCCH would overlap with a PUSCH transmission with the same priority, or - either the first PUCCH or the second PUCCH or both the first and the second PUCCH would overlap in time with a third PUCCH/PUSCH with different priority, the UE does not transmit the positive SR. |
| --- | --- |
| vivo-CR-x2459 | # 18 Multicast Broadcast Services - < Unchanged parts are omitted > If a UE multiplexes in a PUCCH only HARQ-ACK information associated with multicast SPS PDSCHs receptions of same priority that is according to the first HARQ-ACK reporting mode and is not provided sps-PUCCH-AN-ListMulticast, the UE determines the PUCCH resource from the sps-PUCCH-AN-List provided for unicast SPS PDSCH reception as described in clause 9.2.1. If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports and, if any, SR, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs. - *** Unchanged text is omitted *** |
| Qualcomm-Dis-x3568 | NO CR is needed. |
| ZTE-CR-x2766 | # 18 Multicast Broadcast Services <Unchanged parts are omitted> If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports and, if any, SR, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs. <Unchanged parts are omitted> |
| Huawei-Dis-x3792 | NO CR is needed. |

### Round-1(Conclude not pursued)

FL’s analysis:

For the case of NACK-only overlapping with CSI (‘if any, SR’ was included in the specification but was removed), it was agreed NACK-only is transformed into ACK/NACK before multiplexing with CSI. The ‘if any, SR’ was removed because it was realized how NACK-only multiplexing with SR is not straightforward as ACK/NACK multiplexing with SR.

After further discussion, it was agreed that it is up to UE to either drop SR or drop NACK-only when SR overlaps NACK-only, since it is of low probability anyway that UE has positive SR at the same time when UE fails decoding a TB.

In addition, the current statements in TS38.213 only specify the case of UE multiplexing NACK-only and CSI, which could implicitly mean UE will drop SR when UE needs to report the multiplexed NACK-only and CSI because multiplexing three is not supported per the specification.

Given the issue was also discussed several meetings without consensus for the draft CR, FL suggests concluding the issue by not pursued.

#### Proposal 3.18.1

The draft CRs in R1-2302461, R1-2302459, and in R1-2302766 are not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK with the draft CR in x2459/x2766 for clarity of specifications that Rel-15 HARQ-ACK/SR/CSI multiplexing remains applicable as the current text includes CSI but excludes SR. At a minimum, a RAN1 conclusion is needed that Rel-15 remains applicable. |
| LG Electronics | We think that it is good to clarify that SR is not transmitted in the concerned case either by a CR or by RAN1 conclusion. |

## (1-19)NACK-only mode2 for >1 G-RNTI

| DOCOMO-CR-x3695 | # 18 Multicast Broadcast Services - *** Unchanged text is omitted *** A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. - *** Unchanged text is omitted *** |
| --- | --- |
| Ericsson -CR-x3545 | # 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE that is indicated the second HARQ-ACK reporting mode can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. < Unchanged parts are omitted > |

### Round-1(conclude not pursued )

FL’s analysis:

The same CR has been proposed for many meetings but the change is not agreeable with consensus in RAN1.

FL suggests closing this issue but not pursuing the related CRs.

#### Proposal 3.19.1

The draft CRs in R1-2303695 and in R1-2303545 are not pursued.

Company views:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Samsung |  | OK with the draft CRs but, as noted, they were already discussed several times but are not an essential correction and were not agreed. We would request the moderator to apply a uniform handling of proposals for such cases as the situation in 1-19 was essentially same as in 1-9 but the suggestions are different. |  |
| FL response |  | To Samsung, some points are different for two issues. Issue1-9, 1. some companies say there is agreement clearly supporting the CR. 2. Issue 1-9 is continuously the most demanded CR from companies (with the most number of support). |  |
| LG Electronics |  | Agree with FL’s suggestion. |  |

## (1-20)Same ‘dci-enabler’ indication

| Ericsson-CR-x3548 | The specification currently supports that all PDCCH scheduling PDSCH with HARQ feedback in the same UL slot may have a different HARQ enable/disable bit value in DCI. This create ambiguity for the payload size of the type-2 HARQ codebook, since the DAI counter is increased for each PDSCH scheduled, but feedback maybe disabled. In that case, a missed PDCCH may create a misalignment between the network and the UE and in turn, unnecessary retransmission. In order to ensure a common view on the codebook size between the UE and the network, we propose to make the enable / disable HARQ bit in DCI have the same value for all PDCCH scheduling PDSCH with UL feedback in the same slot. # 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212]. A UE does not expect to be receive different indications in multicast DCIs scheduling multicast reception with the same uplink feedback slot. < Unchanged parts are omitted > |
| --- | --- |

### Round-1(conclude not pursued)

FL’s analysis:

The reason as claimed on the coversheet to have this CR is to avoid CB size not alignment due to DCI missing. The change is to add ‘A UE does not expect to be receive different indications in multicast DCIs scheduling multicast reception with the same uplink feedback slot.’

The issues was discussed in earlier meetings without consensus.

Company views from the last meeting for the draft CR:

| Samsung | No need. A UE does not care. In general, gNB misconfigurations and “UE does not expect …” statements are not corrections particularly when the UE behavior remains undefined with or without such statements. |
| --- | --- |
| vivo | No need |
| ZTE | Seems no need since if the DCI indicates no HARQ feedback, the UE may ignore the k1 value indication and there is no PUCCH slot. |
| LG Electronics | Not needed. |

#### Proposal 3.20.1

The draft CR in R1-2303548 is not pursued.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | No need for a CR. There is nothing that the UE cares about. |

# Set#1 CRs for checkpoint

# References

- R1-2302458	Draft editorial CR on RRC signaling for SPS HARQ-ACK for MBS	vivo

- R1-2302459	Correction on multiplexing NACK-only, CSI and SR in a PUCCH	vivo

- R1-2302460	Discussion on overlapping of NACK-only,SR and other PUCCH/PUSCH	vivo

- R1-2302461	Draft CR on overlapping among NACK-only, SR and other PUCCH/PUSCH	vivo

- R1-2302462	Draft CR on not applying disabled HARQ-ACK to multicast SPS PDSCH activation/deactivation	vivo

- R1-2302463	Draft CR on intra-UE multiplexing for MBS	vivo

- R1-2302658	Draft CR on the condition of not providing Type-1 HARQ-ACK codebook	CATT, CBN

- R1-2302659	Draft CR on not applying enabled/disabled HARQ-ACK feedback to multicast SPS activation/deactivation	CATT

- R1-2302662	Draft CR on a set of row indexes R of TDRA tables for Type-1 HARQ-ACK codebook multiplexing of unicast and multicast	CATT

- R1-2302663	Discussion on restriction for a slot timing value indicated by DCI format 1_0 when multicast is configured	CATT, CBN

- R1-2302664	Draft CR on restriction for a slot timing value indicated by DCI format 1_0 when multicast is configured	CATT, CBN

- R1-2302665	Discussion on Type-1/2/3 HARQ-ACK codebook generation with both of maxNrofCodeWordsScheduledByDCI for unicast and multicast	CATT, CBN

- R1-2302666	Draft CR on Type-1/2/3 HARQ-ACK codebook generation with both of maxNrofCodeWordsScheduledByDCI for unicast and multicast	CATT, CBN

- R1-2302667	Draft CR for Type-2 HARQ-ACK codebook generation with both of maxNrofCodeWordsScheduledByDCI for unicast and multicast	CATT

- R1-2302668	Draft CR for Type-3 HARQ-ACK codebook generation with both of maxNrofCodeWordsScheduledByDCI for unicast and multicast	CATT

- R1-2302766	Draft CR on UCI multiplexing for SR and NACK-only PUCCH for MBS	ZTE

- R1-2303107	Correction on HARQ-ACK codebook for multicast SPS PDSCH receptions	Samsung

- R1-2303317	Draft CR on HARQ-ACK feedback for multicast SPS activation/deactivation	Lenovo

- R1-2303318	Draft CR on multicast HARQ-ACK information in a Type-3 HARQ-ACK codebook	Lenovo

- R1-2303386	Draft CR on feedback mode for SPS for MBS	ZTE

- R1-2303387	Draft CR on the generation of the Type1 HARQ-ACK codebook for MBS	ZTE

- R1-2303388	Draft CR on HARQ-ACK multiplexed in PUSCH for MBS	ZTE

- R1-2303389	Draft CR on PUCCH resource determination for MBS	ZTE

- R1-2303390	Draft CR on intra-UE multiplexing for MBS	ZTE

- R1-2303391	Draft CR on intra-UE multiplexing with NACK-only PUCCH	ZTE

- R1-2303392	Draft CR on the Type-3 codebook for MBS	ZTE

- R1-2303423	Draft CR on multicast HARQ-ACK	LG Electronics

- R1-2303545	Draft CR to 38.213 for NACK only mode 2 configuration with multiple G-RNTI	Ericsson

- R1-2303546	Draft CR to 38.213 for PTM retransmissions of SPS multicast	Ericsson

- R1-2303547	Draft CR to 38.213 for HARQ for SPS activation and release	Ericsson

- R1-2303548	Draft CR to 38.214 on DCI for enable/disable HARQ for MBS	Ericsson

- R1-2303549	Maintenance on NR Multicast and Broadcast Services	Ericsson

- R1-2303568	Remaining issues for Rel-17 MBS maintenance	Qualcomm Incorporated

- R1-2303569	Draft CR on feedback for multicast SPS release	Qualcomm Incorporated

- R1-2303570	Draft CR on feedback for first PDSCH after multicast SPS activation	Qualcomm Incorporated

- R1-2303631	Correction on multicast second HARQ-ACK reporting mode	MediaTek Inc.

- R1-2303633	Correction on PUCCH resource determination for multiplexing multicast first and second HARQ-ACK reporting mode	MediaTek Inc.

- R1-2303634	Correction on PUCCH resource determination for multiplexing unicast and multicast HARQ-ACK	MediaTek Inc.

- R1-2303635	Correction on PUCCH resource determination for NACK-only mode 1 with one bit HARQ-ACK	MediaTek Inc.

- R1-2303636	Correction on PUCCH resource determination for the multicast first HARQ-ACK reporting mode	MediaTek Inc.

- R1-2303637	Correction on HARQ-ACK for multicast SPS	MediaTek Inc.

- R1-2303688	Correction on n1PUCCH-AN for multicast HARQ-ACK multiplexing case	MediaTek Inc.

- R1-2303695	Draft CR to support NACK-only feedback with multiple G-RNTIs	NTT DOCOMO, INC.

- R1-2303696	Draft CR on HARQ feedback for the initial SPS PDSCH	NTT DOCOMO, INC.

- R1-2303792	Remaining issues for Rel-17 MBS	Huawei, HiSilicon, CBN

- R1-2303793	Draft CR on disabling HARQ-ACK for multicast SPS	Huawei, HiSilicon, CBN

- R1-2303801	Draft CR on Type-1 HARQ-ACK codebook for multicast	Huawei, HiSilicon, CBN

- R1-2303876	Draft CR on HARQ-ACK for SPS release	Huawei, HiSilicon

- R1-2303877	Draft CR on PUCCH resource for multicast SPS	Huawei, HiSilicon

- R1-2303880	Draft CR on PUCCH resources for multicast HARQ-ACK	Huawei, HiSilicon