3GPP TSG RAN WG1 Meeting #112bis-e	R1-2304090

e-Meeting, April 17th – 26th, 2023

Source:	Moderator (Fujitsu, MediaTek)

Title:	FL summary 3 on L1 enhancements for inter-cell beam management

Agenda Item:	9.10.1

Document for:	Information

Introduction

This contribution is a Feature Lead (FL) summary for A.I. 9.10.1: L1 enhancements for inter-cell beam management.

Plan for GTW/Online discussion

[Proposals for Monday GTW]

FL Proposal 5-5-1-v2

FL Proposal 1: For DL synchronization timing when Rel-17 unified TCI is used for LTM beam indication

- Two-step timing DL synchronization procedure is defined:

  - Step 1: UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement

    - Total number of SSBs where DL synchronization timing information is stored/maintained is a UE capability

      - FFS how SSB down-selection is performed

    - Step 1 does not imply UE will derive QCL-TypeA properties from the SSBs used for L1 measurement.

  - Step 2: gNB activates TCI state(s), and then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the TCI states

FFS:

- Necessity for DL synchronization for TA: whether and how DL synchronization is performed before TA

- Applicability of CSI-RS (if agreed) in addition to SSB

- RAN1 spec impact (UE capability, configuration, activation etc)

- Timing of TCI state activation, i.e. whether TCI state activation is performed before TCI state indication or together with TCI state indication.

FL Proposal 2: For R-18 LTM introduce a mechanism to enable the UE to select a subset of the reported SSB for storing/maintaining DL synchronization timing information.

- Alt 1: SSB subset selection is done explicitly by gNB

  - FFS details of the signalling, whether subset selection is done at RRC or MAC CE level

- Alt 2: SSB subset selection is performed by the UE

  - UE would store/maintain DL synchronization timing information for the N strongest SSBs (based on UE capability) based on the measurement results

- Alt 3: UE would store/maintain DL synchronization timing information for the SSB(s) given in the PDCCH orders for early TA acquisition

FL Proposal 3: TCI state activation:

- Alt 1: Is performed before TCI state indication

- Alt 2: Is performed together with TCI state indication

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

[FL Proposal 5-1-7-v2]

- For SSB-based L1-RSRP measurement RS configuration, at least the following RRC parameters are provided to a UE, and can be updated depending on RAN4 decision:

  - For intra- and inter- frequency:

    - PCI (whether/how to use logical ID is up to RAN2)

    - Time domain information of SSB

      - Alt. 1: ssb-Periodicity, ssb-PositionsInBurst [FL note: it is pointed out if this mechanism works for asynchronous case? ]

        - Supported by ZTE, vivo, CATT, Fujitsu, MediaTek, Apple, Lenovo, Samsung, Nokia, CATT

      - Alt. 2: smtc, [ssb-ToMeasure and deriveSSB-IndexFromCell]

        - Supported by Ericsson, Spreadtrum, Qualcomm, CMCC

      - [FFS: restriction to perform L1 measurement from ssb-PositionsInBurst – FL note: discuss in L1 measurement configuration or in reporting?]

    - Note: Necessity of Transmit power of SSB will be discussed under AI 9.10.2

    - Transmit power of SSB

      - ss-PBCH-BlockPower

  - For inter-frequency:

    - Time domain (SFN) information

      - [halfFrameIndex, sfn0-Offset (sfn-Offset and integerSubframeOffset) – FL note: Some companies are not convinced with the necessity. If necessary, is this also necessary for intra-frequency?]

    - Frequency domain information:

      - ssb-Freq, ssbSubcarierSpacing

    - measurement gap (details are up to RAN4)

[FL Proposal 5-3-7-v2]

- For scenario 2 (and scenario 1 if agreed), the beam indication for target is applied to target Scell(s) multiple target cells included in the list of simultaneous TCI state, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM

  - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells.

  - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2

[FL Proposal 5-3-1a-v2]

- For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets where no RRC configuration for the TCI state is provided or followUnifiedTCI-state is not enabled or not provided, the following alternatives are further studied, and one alternative will be down-selected at RAN1#113. (FL note: partial down-selection in RAN1#112bis-e is appreciated)

  - Alt.1: Follow the indicated TCI state until configured a new TCI state is configured

    - Supported by DCM, Futurewei, Nokia, ZTE, Lenovo, Fujitsu, FGI, Huawei, Samsung

  - [Alt.2: An additional TCI state is signalled in the cell switch command

    - Supported by Samsung]

    - FL note – can we remove this alternative due to minority support?

  - [Alt.3: Follow the SSB index indicated by the cell switch command or PDCCH order for candidate cells

    - Supported by Fujitsu]

    - FL note – can we remove this alternative due to minority support?

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. RRC configuration for TCI state is always available under candidate cell configurations, the network schedules transmission only based on the CORESET following Rel-17 unified TCI, and/or the corresponding beam information would be configured by the target cell after cell switch

    - Supported by Ericsson, Apple, Nokia, vivo, LG, OPPO

[Proposals for Wednesday GTW]

[FL Proposal 5-3-1b-v3]

- Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) ,

  - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

    - Supported by Ericsson, Apple

  - Alt. 2: By indicating Rel-17 TCI state index, i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB

    - Supported by DCM, Google, Futurewei, Qualcomm, Nokia, ZTE, Fujitsu, CMCC, FGI, Huawei, OPPO, Samsung, CATT, vivo, IDCC, CATT, Lenovo

[FL Proposal 5-2-1a-v3]

For the beam selection for SSB based L1-RSRP measurement report, adopt option 1-3:

- [Option 1-2: FL has suggested dropping it after 1st round]

  - Beam selection is performed across all configured (or activated, if introduced),

  - M beams are reported in a single report instance

  - Support: Nokia, MTK, LG, Samsung (Nokia, MediaTek, ZTE, Lenovo, vivo, Samsung, CATT from first round)

    - Similar approach with ICBM

    - Less parameters for configuration (simple)

    - Less overhead

  - Concern: Ericsson, CMCC, [Futurewei?]

    - The beams may consist of only one cell, which read to have multiple report configuration

- Option 1-3

  - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

  - FFS whether and how to define the L best cell criteria

  - FFS whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting

  - Support: Nokia, Huawei, Xiaomi, CATT, Lenovo, Futurewei (Qualcomm, Apple, Nokia, ZTE, CMCC, FGI, Huawei from first round?)

    - The definition best-cell criteria is not necessary, UE implementation

  - Concern: MTK

    - Potential lengthy discussion to define best-cell criteria

- Option 1-4

  - Beam selection is performed for each group of cells, i.e. M best beams for each group

  - M x O beams are reported in a single report instance, where O is the number of groups

  - Support: DCM (Ericsson, Fujitsu, IDCC, FGI from first round)

    - To differentiate frequency

    - Depending on the configuration, Option 1-2 is also possible

  - Concern: ZTE, Huawei

    - Necessity to differentiate the frequency is not clear

    - Overdesign(complexity)

- Note: Value of L and M, N and O, configurability and UE capability are FFS

- Note: How to include the measurement result for serving cell are further discussed depending on the decision on option 2-1 or 2-2 below:

[FL Proposal 5-2-1b-v3]

For the inclusion serving cell measurement results for gNB scheduled L1 measurement reporting, both Option 2-1 and 2-2 are adopted, and it’s configurable by the network

- Option 2-1: P best serving cell beams are always included in a report instance

  - Value of P, configurability and UE capability are FFS

  - How to include the serving cell measurement result in Option 1-X is FFS

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

[5-4-1-v3]

Proposal 1: From RAN1 point of view, at least the following information needs can to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - TA related information (details up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

    - FFS: signaling details of beam indication, e.g., implicitly or explicitly

  - Active DL and UL BWPs for the target Spcell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

  - FFS: C-RNTI

- FFS: the presence of each field (i.e. always present or configurable)

[5-5-1-v3]

FL Proposal 3: For the Rel-17 unified TCI based beam indication in Rel-18 LTM:

- Alt 1: TCI state activation of the target cell is done before TCI state beam indication of the target cell,

  - FFS: signalling details for TCI state activation

- Alt 2: TCI state activation of target cell is done together with TCI state beam indication of the target cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

[5-5-1-v3]

Proposal 1:

At least when the candidate cells are current serving cells, the following procedures are supported, prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM

- CSI acquisition for candidate before reception of cell switch command

- TRS tracking for candidate cells before the reception of cell switch command

Proposal 2:

There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM, except that the candidate cells are current serving cells

- CSI acquisition for candidate before reception of cell switch command

[Proposals for Friday GTW]

[FL Proposal 5-4-1-v3]

- Proposal: From RAN1 point of view, at least the following information needs can be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - TA related information (details up to the discussion in A.I. 9.12.2)

  - Beam Indication TCI State for the target SpCell

    - FFS: signaling details of beam indication, e.g., implicitly or explicitly

  - Active DL and UL BWPs for the target Spcell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

  - FFS: C-RNTI

- FFS: the presence of each field (i.e. always present or configurable)

[FL Proposal 5-5-1-v3]

Proposal: For the Rel-17 unified TCI based beam indication in Rel-18 LTM:

- Alt 1: TCI state activation of the target cell is done before TCI state beam indication of the target cell,

  - FFS: signalling details for TCI state activation

- Alt 2: TCI state activation of target cell is done together with TCI state beam indication of the target cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

[FL Proposal 5-5-1-v3]

Proposal:  There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM, except that the candidate cells are current serving cells

- CSI acquisition for candidate before reception of cell switch command

[FL Proposal 5-2-1-v6]

For the beam selection for SSB based L1-RSRP measurement report,

- Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells

  - How to select the L cells and M beams per cells is up to UE [FL note: is Qualcomm OK?]

- M x L beams are reported in a single report instance

  - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS

    - FFS if UE is allowed to report less than M x L beams [FL note: proposal by Samsung]

  - The values of M and L are configured to the UE in the reporting configuration

- The following configurability is introduced in the report configuration

  - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2

  - 2) FFS at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency [FL note: concern by multiple companies were shown]

  - FFS: 1) and 2) are exclusive

Contact Person

Please input the contact information for each company below:

| Company | Name | Email |
| --- | --- | --- |
| NTT DOCOMO | Jing WANG | wangj@docomolabs-beijing.com.cn |
| Google | Alex Liou | alexliou@google.com |
| Futurewei | Kai Xu | kxu3@futurewei.com |
| Nokia | Sanjay Goyal | Sanjay.goyal@nokia.com |
| Samsung | Emad Farag | e.farag@samsung.com |
| Panasonic | quan Kuang | quan.kuang@eu.panasonic.com |

List of Contributions

| R1-2302315 | L1 enhancements for inter-cell beam management | FUTUREWEI |
| --- | --- | --- |
| R1-2302368 | L1 enhancements for inter-cell beam management | Huawei, HiSilicon |
| R1-2302413 | L1 enhancements to inter-cell beam management | Ericsson |
| R1-2302423 | L1 enhancements for inter-cell beam management | ZTE |
| R1-2302504 | Discussion on L1 enhancements for L1/L2 mobility | vivo |
| R1-2302568 | Discussions on Inter-cell beam management enhancement | OPPO |
| R1-2302619 | Discussion on L1 enhancements for inter-cell beam management | Spreadtrum Communications |
| R1-2302730 | L1 enhancements for inter-cell beam management | Lenovo |
| R1-2302752 | Discussion on L1 enhancements for inter-cell beam management | NEC |
| R1-2302819 | L1 Enhancements for Inter-cell Beam Management | Intel Corporation |
| R1-2302830 | Layer-1 Enhancements for L1/L2-triggered Mobility | Nokia, Nokia Shanghai Bell |
| R1-2302860 | Discussion on L1 enhancements for inter-cell beam management | Sony |
| R1-2302867 | Discussion on L1 enhancements for L1L2-triggered mobility | Panasonic |
| R1-2302870 | Discussion on L1 enhancements for inter-cell beam management | CATT |
| R1-2302914 | Views on L1 enhancements for inter-cell beam management | Fujitsu |
| R1-2302966 | Discussion on L1 enhancements for inter-cell beam management | Xiaomi |
| R1-2303148 | On L1 enhancements for inter-cell beam management | Samsung |
| R1-2303253 | Discussion on L1 enhancements for inter-cell beam management | CMCC |
| R1-2303288 | Discussion on L1 enhancements for inter-cell mobility | KDDI Corporation |
| R1-2303331 | L1 enhancements for inter-cell beam management | MediaTek Inc. |
| R1-2303410 | Discussion on L1 enhancements for inter-cell beam management | FGI |
| R1-2303455 | Discussion on L1 enhancements for inter-cell beam management | InterDigital, Inc. |
| R1-2303503 | L1 enhancements to inter-cell beam management | Apple |
| R1-2303518 | Discussion on L1 enhancements for inter-cell beam management | Google |
| R1-2303610 | L1 Enhancements for Inter-Cell Beam Management | Qualcomm Incorporated |
| R1-2303727 | Discussion on L1 enhancements for inter-cell mobility | NTT DOCOMO, INC. |
| R1-2303082 | Enhancements on inter-cell beam management for mobility | LG Electronics |

Discussion

L1 measurement

[Closed] L1 Intra-frequency measurement

[Conclusion at RAN1#110b-e]

Agreement

- For Rel-18 L1/L2 mobility, L1 intra-frequency measurement for candidate cell is supported

  - At least the following aspects are for RAN1 further study:

    - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

    - Whether and how to apply relaxation for the restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133, where RAN4 impact is foreseen, e.g.

      - SFN offset alignment compared with serving cell

      - BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP

      - Introduction of symbol level gap or SMTC for larger Rx timing difference (i.e. larger than CP length)

    - Commonality with intra-frequency L3 measurement

    - Commonality with L1 inter-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 to ask RAN4 if the restriction on e.g., SFN offset alignment, BWP setting and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

  - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

[Conclusion at RAN1#111]

No discussion as LS from RAN4 had not been received at that time.

[Conclusion at RAN1#112]

No discussion as LS from RAN4 requires no RAN1 discussion.

[Summary of Contributions]

Based on RAN4 LS, the follow is proposed.

- Intel: extension of Rel-17 ICMB mechanism in case where SSB RTD is larger than CP length

[FL observation]

Even though the reply LS from RAN4 is received and their view on the definition of intra- and inter- frequency was provided there, RAN4 also mentioned that their discussion has not been concluded yet. In addition, it is not clear yet what is the RAN1 spec impact driven by RAN4 agreements. Given this situation, FL thinks RAN1 needs no discussion in this meeting.

Given the observation above, no FL proposal is made.

[Comments if any]

| Company | Comment |
| --- | --- |

[Closed] L1 Inter-frequency measurement

[Conclusion at RAN1#110b-e]

Agreement

- For Rel-18 L1/L2 mobility, further study the potential RAN1 spec impact of L1 inter-frequency measurement

  - The definition and scenarios of L1 inter-frequency measurement is determined by RAN4, and RAN1 assumes at least the following until receiving their confirmation

    - The scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

      - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

      - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - At least the following aspect is studied:

    - Commonality with L1 intra-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 would like to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

    - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

    - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - It is RAN1 understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue

  - Note: this content is included in the LS agreed for intra-frequency L1 measurement

[Conclusion at RAN1#111]

Agreement

- For Rel-18 LTM, L1 inter-frequency measurement is supported from RAN1 point of view.

[Conclusion at RAN1#112]

No discussion as LS from RAN4 requires no RAN1 discussion.

[Summary of Contributions]

Some companies discussed about the necessity of measurement gap in their contributions:

  - Huawei: A UE capability of L1 intra-frequency measurement without gap can be introduced for LTM. And L1 measurement gap should be configured for the UE in asynchronous measurement scenario if UE do not indicate the capability.

  - Huawei: Support UE to report the RTD exceed CP or L1 measurement cannot be handled without gap by its capability

  - Lenovo: Introduction of measurement gap

  - Xiaomi: L1 measurement gap/window needs to be introduced to support inter-frequency beam measurement.

  - FGI: For inter-frequency measurement, measurement gap information needs to be provided to a UE.

  - Google: Support introducing symbol-level L1 measurement gap for SSB/CSI-RS from the neighbouring cells configured for L1-RSRP/L1-SINR report or CBD.

[FL observation]

Even though there is some possibilities to introduce (enhanced) measurement gap for L1 inter-frequency measurement, FL’s understanding is that the details of measurement gap is RAN4 issue, and the optimization from RAN1 POV (if necessary) can be discussed after RAN4 design is finalized. In this sense, FL sees no necessity to discuss the measurement gap issue in this meeting.

With this observation, no FL proposal is made in this meeting.

[Comments if any]

| Company | Comment |
| --- | --- |

void

[Low] Measurement RS

[Conclusion at RAN1#110b-e]

Agreement

- For Rel-18 L1/L2 mobility,

  - SSB is supported for L1 intra-frequency measurement

  - SSB is supported for L1 inter-frequency measurement if inter-frequency L1 measurements are supported

- Further study the following L1 measurement RS for candidate cell

  - CSI-RS for tracking, beam management, CSI and mobility, CSI-IM, which is for L1 intra-frequency and L1 inter-frequency (if supported)

[Conclusion at RAN1#111]

The FL proposal 1-4-v3 was not due to the lack of time during RAN1#111. Companies are encouraged to perform their analysis based on the latest proposal below:

- For Rel-18 LTM,

  - L1 measurement based on CSI-RS for beam management for candidate cells is supported for L1 intra-frequency measurement and L1 inter-frequency measurement if supported in RAN4

    - The definition of intra- and inter- frequency for CSI-RS is defined in RAN4

    - The CSI-RS is explicitly linked to a candidate cell

    - Applicability to L1-RSRP and/or L1-SINR is separately discussed.

    - FFS for the support of other CSI-RS types (i.e. tracking, CSI, mobility and CSI-IM).

[Conclusion at RAN1#112]

The following FL was not able to be discussed again because of the lack of time.

- [Working assumption: CSI-RS is introduced for L1-RSRP measurement from RAN1 point of view

  - Intra- and inter- frequency is supported

  - At least CSI-RS for BM [mobility] is supported

  - Send an LS to RAN4 to explicitly ask their feasibility to finalize their work in Rel-18]

[Summary of contributions]

Many companies discussed about the introduction of CSI-RS, and the types of CSI-RS supported for Rel-18 LTM. The companies’ views are summarized below:

CSI-RS for at least L1-RSRP measurement:

- Support (18): Futurewei, Huawei, ZTE, vivo, OPPO, Lenovo, Intel, Nokia, Sony, CATT, Samsung, CMCC, KDDI, FGI, IDC, Apple, LGE, DCM

- Not support (2): Ericsson, MediaTek

Types of CSI-RS:

- Beam management (6): Huawei, ZTE, Intel, CATT, CMCC, DCM

- Mobility (4): Sony, CATT, Samsung, LGE

- TRS (2): Huawei, ZTE

- CSI (1): Huawei

The potential benefits have already been pointed out: flexibility of configuration, beam refinement and potential higher L1-RSRP thanks to the narrow beam.

The reason why CSI-RS should not be supported in Rel-18 is provided by 2 companies:

- MediaTek: First, the definition of intra and inter frequency measurement based on CSI-RS is still missing. Compared with SSB-based L1 intra-frequency measurement discussion where L3 measurement definition of intra-frequency is used as the baseline for discussion, CSI-RS based L1 intra-frequency measurement definition can’t easily follow the same discussion pattern. In particular, as specified in TS 38.133 clause 9.10.2, part of the definition of CSI-RS based L3 intra-frequency measurement is the center frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the center frequency of the CSI-RS resource of the serving cell indicated for measurement. Such statement can’t be directly used for L1 intra-frequency measurement since several CSI-RS resources can be configured in a serving cell for L1 measurement. Without a fundamental description of CSI-RS based L1 intra-frequency measurement, the discussion on support of the feature might not be efficient and the benefit of flexibility of configuration of CSI-RS is not justified.

- Ericsson: If additional measurement possibilities are introduced, the configuration overhead grows. For example, the CSI-RS configuration is quite flexible, resulting in a much larger overhead. Based on this, our preference is not to support CSI-RS as a measurement target for LTM In Rel-18:

[FL observation]

The introduction of CSI-RS has been discussed for long time, and some offline time was spent in RAN1#112 meeting. However, no consensus has been achieved due to the reasons above (RAN4 impact, complexity to handle many beams and CSI-RSs).

FL’s view is that it wouldn’t be good idea to spend more time on this issue because there are so many essential issues while CSI-RS isn’t due to the presence of SSB-based L1 measurement. Considering the many supporting companies for the introduction of CSI-RS for L1 measurement, FL would like to propose the following as a WF, and to discuss by email to avoid wasting GTW/Teams time.

[FL Proposal 5-1-4-v1]

- Working assumption: CSI-RS is introduced for L1-RSRP measurement from RAN1 point of view

  - Intra- and inter- frequency L1 measurement is supported

  - At least CSI-RS for BM is supported

  - Send an LS to RAN4 to explicitly ask their feasibility to finalize their work in Rel-18, and the WA is confirmed when positive feedback is received from RAN4.

[Comments to FL Proposal 5-1-4-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support FL proposal. |
| Ericsson | Do not support. There is very little difference between an agreement and a working assumption. |
| Google | We share similar views as MTK and Ericsson. We also agree with FL’s assessment that introduction of CSI-RS is not essential at this point. |
| Futurewei | From performance perspective for LTM as discussed before, CSI-RS needs to be supported. We support FL proposal. |
| QC | Support. CSI-RS is beneficial for beam refinement |
| Nokia | Support |
| MediaTek | Similar to Google and Ericsson, we don’t support the working assumption. Without basic assumption for intra/inter frequency L1 measurement based on CSI-RS, we are afraid that we can’t even provide necessary information to RAN2 for them to complete the measurement config design. |
| ZTE | To improve system performance after handover, we support FL proposal. |
| Lenovo | Support |  |
| Fujitsu | Support |  |
| vivo | Support |  |
| CMCC | Support FL proposal |
| IDCC | Support |
| LG | Fine with the proposal |
| FGI | Support |
| Huawei, HiSilicon | Support. CSI-RS is essential to maintain the throughput during LTM and reduce latency before data transmission with fine beam. Without CSI-RS, the current QCL chain might be broken as PDCCH/PDSCH cannot use SSB as QCL source in current releases. The definition of inter/intra-frequency measurement on CSI-RS for L3 mobility can be taken as reference to reduce the work load. |
| Samsung | Fine with the proposal |

[FL observation]

- Support Proposal 5-1-4-v1 (14)

  - DCM, Futurewei, Qualcomm, Nokia, ZTE, Lenovo, Fujitsu, vivo, CMCC, IDCC, LG, FGI, Huawei, Samsung

- Not support (3)

  - Ericsson, Google, MediaTek

The situation is unchanged from the previous meeting. FL proposal tries to follow the majority view to introduce CSI-RS based L1 measurement while addressing the concern on RAN4 workload. However, 3 companies showed their strong concern on the workload issue and the usefulness itself. Also, no compromise was proposed during the email discussion. Thus, FL sees the difficulties to find a middle ground between two parties.

[FL Proposal 5-1-4-v2]

FL will bring the following proposal to the GTW session this week (may be to the last GTW session in this meeting) to make the binary decision.

Working assumption: CSI-RS is introduced for L1-RSRP measurement from RAN1 point of view

  - Intra- and inter- frequency L1 measurement is supported

  - At least CSI-RS for BM is supported

  - Send an LS to RAN4 to explicitly ask their feasibility to finalize their work in Rel-18, and the WA is confirmed when positive feedback is received from RAN4.

- Support Proposal 5-1-4-v1 (14)

  - DCM, Futurewei, Qualcomm, Nokia, ZTE, Lenovo, Fujitsu, vivo, CMCC, IDCC, LG, FGI, Huawei, Samsung

- Not support (3)

  - Ericsson, Google, MediaTek

FL recommendation is not to introduce this feature in Rel-18 in order to reduce RAN4 workload. Also, even for RAN1, there is no room for this WI to introduce optimization feature, unfortunately. It would be good to specify performance optimization features in Rel-19.

[Comment if any]

| Company | Comment |
| --- | --- |
| QC | Can we at least confirm WA and put lower priority? Perhaps this way is more likely to continue in R19 [FL reply] As long as I see the situation so far, FL feels confirming WA will just make the situation more complicated. Let’s check online if your proposal is acceptable to everyone. |
| ZTE | Agree with the suggestion raised by QC. |
| LG | Fine with it |
| NTT DOCOMO | Agree with QC. |
| Huawei, HiSilicon | Support the WA. Maybe we can also add “up to UE capability” and leave vendor flexibility to implement. [FL reply] FL agrees this functionality can be UE capability. On the other hand, FL is not sure if this proposal can alleviate the concern from opponents. Let’s discuss this online. |
| Xiaomi | We slightly prefer to support SSB only. From our understanding, SSB can provide enough information for LTM. To make sure UE can switch to target cell successfully with less delay based on L1 measurement is the core of this WI. Including too much procedures, like beam refinement to provide higher accuracy beam, before the cell switch is not a good idea. While, if most companies want to support CSI-RS, then we can live with it. |
| Samsung2 | Support WA |
| Futurewei | Support the WA, and suggest revising the first bullet as below: Working assumption: CSI-RS is introduced at least for L1-RSRP measurement from RAN1 point of view |
| FL | Let’s continue the discussion as not many companies (especially the opponents) have provided their view. |
| MediaTek | We don’t support the WA. Our concern on the definition of intra and inter frequency has not been resolved yet. |
| Fujitsu | Support |
| Spreadtrum | Support WA. |
| vivo | Support the WA. |

[Closed] Measurement quantity

[Conclusion at RAN1#110b-e]

Agreement

- For candidate cell measurement for Rel-18 L1/L2 mobility,

  - L1-RSRP is supported for intra-frequency candidate cell measurement.

  - Further study the following measurement quantities for candidate cell measurement

    - L1-RSRP for inter-frequency (if supported)

    - L1-SINR for intra-frequency and inter-frequency (if supported)

- FFS: to assess the use case and the benefit of UL measurement instead of/in addition to DL L1 measurement, which includes:

  - How the UL measurement result is used, e.g. handover decision

  - Signals/channels used for UL measurement, e.g. SRS

  - Spec impact including other WGs, e.g. definition of gNB measurement, interface to transfer RS configuration or measurement results

  - Note: The next discussion will take place based on companies’ contribution in future meeting.

[Conclusion at RAN1#111]

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

[Conclusion at RAN1#112]

The following FL was not able to be discussed again because of the lack of time.

- [CSI-RS based L1-SINR (with channel measurement and interference measurement using CSI-RS) is introduced from RAN1 point of view

  - If supported, both intra- and inter-frequency L1-SINR is supported

  - Send an LS to RAN4 to explicitly ask their availability in Rel-18]

It was pointed out that the benefit of SINR cannot be achieved without CSI-RS. In addition, some companies showed the concern on the L1-SINR.

[Summary of contributions]

Similar to the discussion in the last meeting, there are a lot of contributions talking about the necessity of L1-SINR measurement

Introduction of CSI-RS based L1-SINR for intra- and inter-frequency

- Support (8) : Futurewei, Huawei (SSB as well), vivo, CMCC (inter-frequency), KDDI, FGI, Google, DCM(addressing the concern)

- Not support/deprioritize(6): OPPO, Intel, CATT, MediaTek, IDC, Apple

- For further study(1): Samsung

Note: suggestion by DCM to address the concern (i.e. usefulness of instantaneous interference measurement) :

- Use L1-RSRP and L1-SINR together to select the beams to report.

- Introduce UE/event triggered reporting, with the new event(s) considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams.

[FL observation]

Similar to the discussion in the last meeting, the views on the introduction of L1-SINR is still split, and the same concern is shared by some companies (i.e. UE complexity, Usefulness of instantaneous interference results (as interference varies depending on neighbor cell scheduling), and is L3 filtered L1-SINR in preparation phase is sufficient to choose the best frequency?)

Also, FL believes that the benefit of SINR can be achieved with CSI-RS, as pointed out in RAN1#112. Thus, we should pause the discussion until the final decision on CSI-RS (for L1-RSRP) is made, but can gather the views from companies.

[FL Proposal 5-1-5-v1]

- Working assumption: if CSI-RS is introduced for L1-RSRP measurements, CSI-RS based L1-SINR (with channel measurement and interference measurement using CSI-RS) is introduced from RAN1 point of view

  - If supported, both intra- and inter-frequency L1-SINR is supported

  - Send an LS to RAN4 to explicitly ask their feasibility to finalize their work in Rel-18, and the WA is confirmed when positive feedback is received from RAN4.

FL note: FL will not request email approval until the proposal on CSI-RS based L1 measurement is agreed.

[Comments to FL Proposal 5-1-5-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support FL proposal. |
| Ericsson | Do not support |
| Google | Our position is mistaken. We support introducing L1-SINR, which is useful especially for inter-frequency scenario. However, we don’t think “CSI-RS is introduced for L1-RSRP measurements” is a necessary condition. It seems the middle ground could be to first introduce L1-SINR with interference measurement by IMR. We then suggest the following revision. - Working assumption: if CSI-RS is introduced for L1-RSRP measurements, CSI-RS based L1-SINR (with channel measurement and interference measurement using CSI-RS) is introduced from RAN1 point of view - If supported, both intra- and inter-frequency L1-SINR is supported - Send an LS to RAN4 to explicitly ask their feasibility to finalize their work in Rel-18, and the WA is confirmed when positive feedback is received from RAN4. |
| Futurewei | At least for inter frequency scenario with different interference conditions, L1-SINR needs to be supported. We support FL proposal. |
| QC | Not support L1-SINR. Short-term interference is hard to predict. Not good for L1 beam/cell selection |
| Apple | Not support. Comparing to L1-RSRP, the L1-SINR reflects the instant interferences at the cost of extra computation complexity. In addition, the benefit is not very clear for LTM operation as ‘one-shot’ measurement result is used for LTM triggering determination. In addition, L1-SINR has been supported by L3 measurement, which is always available for NW as a metric for LTM operation. |
| Nokia | We do not support L1-SINR without having detailed analysis in terms of benefits and configuration complexity, and which we think cannot be done until we know CSI-RSs are supported or not. Therefore, we support to pause the discussion on this topic until we make any decision on CSI-RS. |
| MediaTek | Not support. L1-SINR support definitely has implementation impact and the benefit is still not clear us compared to L3-SINR. Therefore, RAN4 input won’t resolve the concerns raised by several companies. |
| ZTE | Do not support L1-SINR since there is no see a significantly benefit. |
| Lenovo | More evaluation is needed for L1-SINR based LTM. |
| Fujitsu | Support |
| vivo | Support FL proposal. |
| CMCC | Support. Our position is to support the intra-frequency L1-SINR as an extension of ICBM, which is updated with remarks. But support of inter-frequency scenario is also acceptable to us. |
| IDCC | We think L1-RSRP is sufficient. It can be considered later is there is any benefit. |
| FGI | Support L1-SINR but it is not necessary to bundle the discussion with the result of CSI-RS based measurement. |
| Huawei, HiSilicon | Support FL proposal. SINR is beneficial for gNB to make decision on the HO according to both signal and interference level. As for the fluctuation of interference measurement, it can be left for gNB’s implementation. Considering L1-SINR for serving cell and L3 measurements on SS/CSI-SINR have been supported in legacy release, we do not think there is any issue to support L1-SINR in LTM |
| Samsung | This issue requires further study. We think that it is not critical for this feature. Therefore, we prefer to defer the discussion until more essential aspects are agreed. |

[FL Observation]

- Support the introduction of L1-SINR (basically OK with FL proposal) (8)

  - NTT DOCOMO, Google(modification suggested), Futurewei, Fujitsu, vivo, CMCC, FGI, Huawei

- Not support (or further study is necessary) (9)

  - Ericsson, Qualcomm, Apple, Nokia, MediaTek, ZTE, Lenovo, IDCC, Samsung

The number of supporting/not-supporting companies are almost even. However, most of the not-supporting companies see the necessity of the study on the benefit of L1-SINR. Also, many companies see the potential benefit of L1-SINR conditioned on the introduction of CSI-RS. Thus, FL thinks the remaining time in Rel-18 is not sufficient to finalize the discussion on L1-SINR, irrespective of the decision on CSI-RS. Also, impact to RAN4 workload is something we need to consider.

Having said this, FL thinks more discussion on the benefit L1-SINR and/or the measures to address the concern (e.g. DCM has already provided their ideas) is very useful. However, there is less chance in Rel-18. FL would like to keep the discussion open, but this topic needs to be handled on low priority basis. No further FL proposal is not provided in this meeting.

With this understanding, the email discussion in this section is closed.

[Comments if any]

| Company | Comment |
| --- | --- |

[Closed] Filtering for L1 measurement results

[Conclusion at RAN1#110b-e]

FL proposal below was not agreed and postponed to the further RAN1 meeting.

- For Rel-18 L1/L2 mobility, [study the importance of mitigating the ping-pong issue for L1/L2 mobility, which is expected to align with RAN2. If important yes,] further study at least the following mitigation aspects: FL: A concern was raised if RAN1 can perform a proper study on ping-pong issue, CATT, Samsung, LG wants to keep it. FL thinks we can keep it as long as companies say they can perform their analysis,

  - UE-based filtering to the L1 measurement results, where the definition of filtering includes:

    - Time domain filtering: e.g. exact definition of time domain filtering, and/or

    - Cell-level (spatial domain) filtering: e.g. how many beams are averaged, and/or how the beams are chosen.

  - Applicability to L1-RSRP and L1-SINR (if supported)

  - Applicability to intra-frequency and inter-frequency (if supported)

  - Necessity to be specified in standard considering the presence of alternative implementation-based solutions, e.g. gNB-based filtering and/or L3 measurement (when involved)

[Conclusion at RAN1#111]

Given the comments from companies, FL believes no positive result can be obtained on this topic even when we discuss the discussion in this meeting. Therefore, FL would like to take approach 1 this meeting and to encourages to have offline discussions for the next meeting.

With this analysis, the discussion on this section is closed. If companies have any comments, the following table can be used or further input.

[Conclusion at RAN1#112]

No discussion was held as no progress was expected

[Summary of contributions]

While some companies still propose to introduce time domain and/or cell domain filtering for L1 measurement result, FL thinks this is not an essential feature for Rel-18 LTM, and the discussion will not take us anywhere given the past RAN1 discussions (as no consensus on the ping-pong issue and the feasibility of RAN1 evaluation etc).

Therefore, no FL proposal is made in this section.

[Comments if any]

| Company | Comment |
| --- | --- |

[Email approval] Configurations for L1 measurement

[Conclusion at RAN1#111]

The following proposal was not treated in RAN1#111 and postponed to next meeting. Since this functionality is essential for Rel-18 LTM, companies are encouraged to perform their analysis based on the final proposal (i.e. FL proposal 1-7-v3) as well as the companies comments for this proposal above. Especially, there was a big discussion on the difference between Option 2 and 3. Also, it was pointed out (from October meeting) that Option 4 for intra-frequency will violate our former agreement on the “Rel-17 ICBM baseline”. They are key discussion points in the next meeting.

- For Rel-18 LTM, further study the following structure for L1 measurement configurations.

  - Option-1: Based on CSI measurement configuration specified in Rel-17 ICBM

    - CSI-MeasConfig for serving cell and candidate cell(s), which requires inter-DU coordination

    - For inter-frequency, at least the frequency information, SMTC or measurement gap (MG) with candidate cell are additionally introduced

  - Option-2: Introduce an independent a measurement configuration for candidate cell(s) [and serving cell] from serving cell configurations, which, and the measurement configuration is decoupled with serving cell configuration.Introduce an independent measurement configuration for all candidate cell(s) and serving cell(s) (FL note: Suggested by DOCOMO and their intention is to enable reporting for candidate cells with that for serving cell ) from serving cell configurations, which is decoupled with serving cell configuration.

    - L1 measurement resource set can be configured outside candidate cell configurations (i.e. ServingCellConfig or CellGroupConfig)

    - Separate CSI-MeasCofig is configured for candidate cell from the CSI-MeasConfig for serving cell

  - Option-3: Use measurement configuration for each candidate cell

    - L1 measurement resource set can be configured inside candidate cell configurations (i.e. ServingCellConfig or CellGroupConfig)

  - Option-4: Do not include RS information or cell information in measurement configurations

    - For intra-frequency, neither SSB/RS indices nor PCI is configured.

    - For inter-frequency, neither SSB/RS indices nor PCI is configured, but frequency information is configured

  - Note: Proponents of each option are encouraged to bring the detailed explanation in RAN1#112.

[Conclusion at RAN1#112]

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

[Summary of contributions]

Cell identification

- PCI

  - Ericsson: the UE would have to map that to a PCI, and the mapping between the logical ID and the PCI must also be provided to the UE. In other words, the UE must be provided a PCI identifying the measurement target one way or another. To clarify the agreement, the logical ID could be removed.

  - MediaTek: Introduce a physical cell indicator field to reference signal configuration to specify the physical cell of a reference signal.

    - The mapping between indicator filed contents and physical cells is fixed before and after cell switch

Time domain information

- L3-like approach (2): SMTC is configured (Spreadtrum), and ssb-ToMeasure and deriveSSB-IndexFromCell (Ericsson)

- L1-like approach (6): ssb-Periodicity, ssb-PositionInBurst  (ZTE, vivo, CATT, Fujitsu, MediaTek, Apple)

Time domain (SFN) information

- halfFrameIndex, sfn0-Offset(sfn-Offset and integerSubframeOffset) (ZTE, DCM)

Frequency domain information for inter-cell measurement

- ssb-Freq, ssbSubcarierSpacing (ZTE, Fujitsu)

Tx power for SSB

- ss-PBCH-BlockPower are configured (ZTE, vivo, OPPO, Spreadtrum, Nokia, CATT, Xiaomi, Samsung, Apple)

Measurement gap for inter-frequency measurement

- The details are up to RAN4

[FL observation]

As pointed out in the agreement at RAN1#112, RAN1 still needs some discussions to finalize the RRC parameters even though the RRC structure is discussed in RAN2.

PCI or logical ID: as pointed out by one company, just saying “logical ID” is somewhat ambiguous as the RRC design is decided by RAN2 and it is not necessary to add redundant information. Regarding whether logical ID for PCI can be kept regardless of serving cell change, FL suggestion is to discuss this in RAN2.

Time domain information: limited number of companies showed their preference on the time domain information, i.e. L3-like approach vs L1-like approach. The benefit of L3-like approach is that serving cell need not to know SSB setting of candidate cells, while the drawback is blind detection of SSB at UE side. For L1-like approach the pros/cons are opposite from those for L3-like approach. FL views both mechanisms work, and then the question is who will be sacrificed, gNB or UE. It is hence suggested to go with majority view.

SFN information: it is proposed to provide additional time domain information such as halfFrameIndex, sfn0-Offset (sfn-Offset and integerSubframeOffset). The proponents suggest that they are useful for asynchronous network, but FL wonders if this information is always available at serving cells.

Frequency domain information for inter-cell measurement: no change from the previous agreement, SCS for SSB and center frequency are necessary and sufficient.

Tx power for SSB: this is a FFS from previous meeting, and many companies see the necessity for pathloss calculation.

[FL Proposal 5-1-7-v1]

- For SSB-based L1-RSRP measurement RS configuration, the following RRC parameters are provided to a UE:

  - For intra- and inter- frequency:

    - PCI (whether/how to use logical ID is up to RAN2)

    - Time domain information of SSB – FL suggests to adopt Alt. 1 due to majority (6 vs 2)

      - Alt. 1: ssb-Periodicity, ssb-PositionInBurst

      - Alt. 2: smtc, [ssb-ToMeasure and deriveSSB-IndexFromCell]

    - Transmit power of SSB

      - ss-PBCH-BlockPower

  - For inter-frequency:

    - Time domain (SFN) information

      - [halfFrameIndex, sfn0-Offset (sfn-Offset and integerSubframeOffset) – FL note: need more discussion]

    - Frequency domain information:

      - ssb-Freq, ssbSubcarierSpacing

[Comments to FL Proposal 5-1-7-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We think the time domain (SFN) information can be provided for both intra- and inter-frequency measurement, to support the unsync. candidate cells. |
| Ericsson | Do not support. Keep in mind that the system is asynchronous, so ssb-Periodicity and ssb-PositionInBurst does not help ss-PBCH-BlockPower is not needed for the measurements, propose to discuss this in AI 9.10.2 The UE does not need halfFrameIndex or sfnOffset to perform measurements |
| Google | Support in principle |
| Futurewei | From L1-RSRP measurement perspective, introducing ss-PBCH-BlockPower is not necessary, and not justified. |
| QC | We think SFN may not be needed. SMTC or measurement gap is sufficient. Also, UE can know the SSB burst location from L3 measurement |
| ZTE | We generally support FL proposal, but we understand that measurement gap also needs to be provided in meas information especially for inter-frequency scenario, while whether it is needed for intra-frequency case can be discussed further. |
| Lenovo | We have the following comments: - We prefer to adopt Alt1 to provide the time domain information of SSB, i.e., by ssb-Periodicity, ssb-PositionInBurst as that used for ICBM. - It seems ss-PBCH-BlockPower is unnecessary for SSB based L1-RSRP measurement. On inter-frequency measurement, we suggest to firstly agree the parameters on frequency domain information |
| Fujitsu | Support in principle. As for SFN information, it may need to be discussed for both inter-frequency and intra-frequency scenarios. If ssb-PositionInBurst is provided, it is also suggested to discuss whether a subset of SSB indexes within ssb-PositionInBurst should be provided for measurement. Actually, this subset of SSB indexes corresponds to the following “Resource for channel measurement” for the report configuration in Proposal 5-2-3-v1. [Proposal 5-2-3-v1] - For L1 measurement report configuration for gNB scheduled reporting, - As a starting point, at least the following information needs to be provided to a UE, e.g. - Resource for channel measurement, i.e. SSB and CSI-RS if agreed |
| vivo | Fine with the above RRC parameters for L1 intra-frequency measurement in FR2, which has been agreed to perform L1 measurement based on fine beam in RAN4#106 meeting. While for inter-frequency cases, how to perform beam measurement is still in discussion, so the corresponding RRC parameter should be discussed after the RAN4-related discussion is complete. |
| Xiaomi | For L1-RSRP measurement, ss-PBCH-BlockPower is unnecessary indeed. But that does not mean it should not be configured. As shown in Figure below, after UE receives cell switch command, for RACH-less cell switch, it needs to send acknowledgement to targe cell. Then, the transmission power used to estimate pathloss is needed to decide the transmit power of this acknowledgement. |
| CMCC | Our understanding is ss-PBCH-BlockPower should be supported, which is useful for the following procedure, such as PL measurement which could be used for the PRACH transmission. For the time domain information, we are slightly supportive of Alt 2. In the current spec, the ssb-Periodicity and ssb-PositionInBurst could also be included in the SMTC configuration, just like SSB-MTC-AdditionalPCI-r17. |
| IDCC | Support the alternatives. |
| LG | Fine with the PCI. But for the SFN, it seems not clear to be needed |
| FGI | For frequency domain information, we suggest that measurement gap information should be included. |
| Huawei, HiSilicon | We support the transmit power of SSB as it is left over from last meeting. As for the rest, we already had high level agreement and agreed to leave it for RAN2 in last meeting. They are not urgent for now. |
| Nokia | For RAN1, it is sufficient to list the necessary parameters (as in Alt1), then RAN2 may decide if an existing IE like SSB-MTC-AdditionalPCI-r17 could be used, or a new IE will be needed. We agree that power information is not needed for measurement, but it is needed for a PRACH transmission later. We can add a note there or remove it from the list but discuss it in 9.10.2. |
| OPPO | We support to include the Tx power of SSB and also we shall include the measurement gap at least for inter-freq measurement. |
| Samsung | For intra-frequency, we should keep the Rel-17 structure/parameters for SB-MTC-AdditionalPCI-r17. We don’t see a need to change that. This includes: - additionalPCI - periodicity - ssb-PositionsInBurst - ss-PBCH-BlockPower If CSI-RS is agreed as a measurement RS, we can revisit and decide on additional parameters for CSI-RS. For inter-frequency, in addition to the intra-frequency parameters, listed above, we should include: - Frequency information (e.g., ARFCN) - Sub-carrier spacing. |
| CATT | We share the same view as FL that Alt1 can be adopted. We also support the transmit power of SSB. |

[FL Proposal 5-1-7-v2]

- For SSB-based L1-RSRP measurement RS configuration, at least the following RRC parameters are provided to a UE, and can be updated depending on RAN4 decision:

  - For intra- and inter- frequency:

    - PCI (whether/how to use logical ID is up to RAN2)

    - Time domain information of SSB

      - Alt. 1: ssb-Periodicity, ssb-PositionsInBurst [FL note: it is pointed out if this mechanism works for asynchronous case. Can proponents explain the feasibility? ]

        - Supported by ZTE, vivo, CATT, Fujitsu, MediaTek, Apple, Lenovo, Samsung, Nokia, CATT

      - Alt. 2: smtc, [ssb-ToMeasure and deriveSSB-IndexFromCell]

        - Supported by Ericsson, Spreadtrum, Qualcomm, CMCC, NTT DOCOMO

      - [FFS: restriction to perform L1 measurement from ssb-PositionsInBurst – FL note: Can we discuss in reporting section?]

    - Note: Necessity of Transmit power of SSB will be discussed under AI 9.10.2

    - Transmit power of SSB

      - ss-PBCH-BlockPower

  - For inter-frequency:

    - Time domain (SFN) information

      - [halfFrameIndex, sfn0-Offset (sfn-Offset and integerSubframeOffset) – FL note: Some companies are not convinced with the necessity. If necessary, is this also necessary for intra-frequency?. Can proponents explain the motivation?]

    - Frequency domain information:

      - ssb-Freq, ssbSubcarierSpacing

    - measurement gap (details are up to RAN4)

- FL note – “at least ~ can be updated depending on RAN4 decision” is added to address vivo’s concern. Waiting for RAN4 just delays our discussion, which is not good for RAN1 progress

- FL note – Comments to companies input

  - Xioami: ACK to cell switch command is RAN2 region. Off course, HARQ-ACK will be sent in reply to PDCCH carrying MAC CE, but it is sent to source cell

  - Fujitsu: I put a placeholder for the restriction ssb-PositionsInBurst, but FL recommendation is to discuss this issue under reporting section as we can see similar motivation.

[Comments to FL Proposal 5-1-7-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | First, for time domain information of SSB, if Alt.1 is applied, SFN related information is also needed to support measurement of async. candidate cells. Otherwise, Alt.2 should be applied. We added DCM under Alt.2. Please note that in Rel-17 ICBM we assumed all the candidate cells are synchronized with serving cell, that’s why only ssb-PositionsInBurst and periodicity were provided in Rel-17 ICBM. Second, for SFN related information, we think it is beneficial for measurement of async. candidate cells. We could understand that if measurement gap is used, the measurement can be performed within measurement gap. But measurement gap is to be decided by RAN4. In addition, there is statement of ‘can be updated depending on RAN4 decision’, thus, we think SFN related information can be kept in the proposal, and RAN4 can make further decision anyway. |
| Fujitsu | It is OK to discuss the restriction on ssb-PositionsInBurst in the reporting section. |
| CMCC | For the SFN information, we are a bit confused how the SFN or sfn-Offset can benefit or improve the inter-frequency measurement. If the measurement gap is configured, UE will take measurement within the gap for the inter frequency. The time period for measurement is fixed if the gap is indicated. Then it seems SFN and sfn-Offset does not provide additional information to facilitate the inter-frequency measurement. For the transmit power of SSB, it is deleted with addition note that this depends on the discussion of AI 9.10.2. But I think it is the same situation that AI 9.10.2 thinks this issue belongs to 9.10.1. Our proposal is to discuss this issue here and support to provide the power information. |
| Samsung | Prefer to keep ss-PBCH-BlockPower |
| Ericsson | Do we really need this proposal? If so, we propose to split it in several parts. |
| vivo | For the transmit power of SSB, it can not only be used to calculate path loss and then determine the transmission power of PRACH, but also can be used to select the best K beams across frequency/cells. Therefore, it should be determined in this agenda. |
| QC | Fine with FL’s proposal, and prefer keep SSB Tx power |
| ZTE | Regarding “Transmit power of SSB”, we tend to discuss it in this agenda item because although this parameter is related to agenda item 9.10.2, it mainly discusses some designs and mechanisms of TA acquisition and does not involve much content on parameter configuration. Besides, we think that parameter “sfn-SSB-Offset” may also need to added in Time domain (SFN) information. Actually, it is different with “sfn0-Offset”, it is used to indicate SFN offset of SSB of candidate cell with respect to SFN 0 of serving cell. |
| MediaTek | We also have question regarding the necessity of SFN related information for L1 measurement. Can proponent elaborate a little bit more? On the other hand, we agree SFN related information of target cell need to be obtained by UE before cell switch; otherwise, how does UE obtain those information without further PBCH decoding or RRC configuration update? |
| LG | Generally fine with the FL’s proposal but for the SFN information in inter-frequency, it is still unclear to take it |
| Huawei, HiSilicon | We share similar view as Ericsson. Except for the transmit power, we already had sufficient information of L1 measurement configuration provided to RAN2. |
| Xiaomi | For the time domain information of SSB, we prefer to support Alt.2. In current spec., periodicity of ssb is included under the configuration of SMTC. And other information, if necessary, can also be included in SMTC in LTM. Therefore, we believe Alt.2 is more appropriate. |
| CATT | Fine with FL’s proposal. For transmit power of SSB, we agree with vivo that it should be determined in this agenda and we support to add SSB Tx power. |
| Lenovo | To support intra-frequency asynchronous case with Alt1, sfn-Offset may be further provided. |
| Futurewei | Both alternatives can work for time domain information of SSB. If considering asynchronization issue, we slightly prefer Alt.2, especially for inter frequency case. For intra frequency case, Alt. 1 may work based on UE capability. However, because this is for LTM, a different SMTC (if supported) in Alt.2 from that of L3 may be needed for latency reason. |
| ZTE2 | Regarding SSB power information, we think that SSB is considered as a measurement reference signal and used to determine pathloss for subsequent UL transmission of target cell. So we don’t think that SSB power information is not related to measurement. Furthermore, if SSB power information is not configured in the measurement configuration, we would like to know where it should be configured. Regarding on whether async scenario is related to inter-frequency measurement, we understand that whatever for inter frequency or intra frequency measurement, it involves two scenarios: sync and async. So far, there is no any conclusion that we should not consider async case. Thus, it is obvious that asycn case cannot be ruled out unless we first have a conclusion. If async case is considered, then it is difficult for us to assume to have the same SFN related information for the serving cell and candidate cells. With this consideration, we cannot directly assume that the time offset of SFN0 slot0 between the serving cell and the candidate cell (i.e., sfn0-Offset (sfn-Offset and integerSubframeOffset)) is the same. After determining SFN information for serving cell and candidate cell, we need to further determine SFN offset of SSB transmission of candidate cell, which can be done by“sfn-SSB-Offset”. Further, UE needs to know SSB is transmitted in the first half frame or the second half frame by “halfFrameIndex”. As for which SSB index will be transmitted, it will be determined by “ssb-PositionsInBurst”. Regarding SMTC and related L3 parameters, what we want to say that current working item is discussing L1 measurement rather than L3. besides, In similar or related topic such as Rel-17 ICBM and even in Rel-16 positioning, L1 measurement information is determined based on the above mentioned parameter, while not reuse L3 parameter. So we think that it is an reasonable way to introduce some additional parameter based on existing L1 parameters to accommodate the requirement of inter-frequency and async scenarios. |

[FL observation]

FL sees a difficult situation from companies reply:

- On SSB power, FL thinks the motivation should be discussed under AI 9.10.2, but some (many?) companies request to handle this issue under this AI.

  - Vivo: It is pointed out that SSB Tx Power is useful for best K beams across frequency/cells for reporting (performed by UE implementation)

    - FL is not sure if this is a valid use case. For example, {cell#1_RSRP=-50dBm with TxPower=43dBm} {cell#2 RSRP=-50dBm with TxPower=40dBm} then, which cell is better then?

  - ZTE: although this parameter is related to agenda item 9.10.2, it mainly discusses some designs and mechanisms of TA acquisition and does not involve much content on parameter configuration.

    - FL believes parameter acquisition is also a procedure of TA. Again, FL doesn’t understand why the discussion on “necessity of SSB power” cannot be done in AI 9.10.2.

  - ZTE2: if SSB power information is not configured in the measurement configuration, we would like to know where it should be configured.

    - FL agrees this is the valid point. If it is the necessity SSB Tx power information is necessary for some purpose, then FL is happy to discuss in this AI. However, FL haven’t got such information yet.

  - It is clear and common understanding that SSB power information is not necessary for L1 measurement purpose

- Proponents were requested to explain the background of their proposals on parameters, but no clear answer yet

  - ssb-Periodicity, ssb-PositionsInBurst: does this mechanism work for ansync case

  - Necessity of SFN information: some companies still wonders if this information is necessary for L1 measurement.

    - Lenovo: intra-frequency asynchronous case with Alt1, sfn-Offset may be further provided

      - FL is not sure if SFN offset is really used among intra-frequency cells. It looks weird

- Some companies requested not to discuss this issue at all (i.e. task for RAN4, RAN2), but FL’s understanding was that RAN1 should decide the decision on time domain information

Compiling the situation above, FL suggestion is to discuss the following issues in Teams offline (hopefully on Thursday):

- Question 1: Time domain information

  - No answer clear answer for the applicability to ansyc network yet

  - Taking into account the support of async network case, it would be reasonable to consider smtc/measurement gap as time domain information, which falls into RAN4 (and RAN2) region

  - FL suggestion

    - Go with Alt.2: smtc is configured for time domain information for intra- and inter-frequency L1 measurement RS, and the detailed RRC parameter and structure design is left to RAN2.

      - This is captured as “conclusion” in the minute

    - This message is included in the LS (to RAN2 and RAN4)

      - Capture the agreement in the LS, and no explicit question is necessary

- Question 2: ss-PBCH-BlockPower for intra-and inter-frequency

  - To confirm this information is not necessary for L1 measurement

  - To confirm that check if the necessity of SSB Tx power information for candidate cell PRACH transmission has been agreed confirmed in AI 9.10.2

    - If yes, continue the discussion under this AI

    - If no (or not yet), this issue should be discussed under AI 9.10.2 first

  - To check how many companies support to introduce ss-PBCH-BlockPower for candidate cell RACH

    - No/Concern/More study: None

  - To check if there are any other use cases for SSB Tx Power information

    - For K best beam/cell selection?

  - FL suggestion

Offline consensus

  - ss-PBCH-BlockPower is included in the LTM configuration.

- Question 3: Other RRC parameters (esp. SFN and measurement gap)

  - The open points are SFN and measurement gap

  - According to the comment from DOCOMO, the necessity of SFN can be discussed in RAN4

  - Measurement gap is clearly RAN4 issue

  - FL suggestion

    - With this understanding, RAN1 needs no further discussion, and no additional conclusion is needed in this meeting

With this, the discussion of this section is suspended from 8:30 to 21:30UTC on Thursday.

If you have any further comment, please use [Comments to FL Proposal 5-1-7-v2] table above.

[FL observation after Teams offline on Thu]

Only one issue has been discussed during the teams offline on Thursday, and the following offline consensus has been achieved

Offline consensus

  - ss-PBCH-BlockPower is included in the LTM configuration.

To clarify the common understanding, this consensus has been achieved under “Configuration for L1 measurement RS” section, but this does not preclude the further discussion on the RRC structure, i.e. whether this parameter should be included under L1 measurement RS configuration. What is confirmed in the teams offline is that this parameter is necessary at least for the purpose of candidate cell RACH.

Meanwhile, the discussion of time domain information was not take place due to the lack of time. The situation is like below:

- Time domain information

  - Companies view is split for intra-frequency measurement, “ssb-Periodicity, ssb-PositionsInBurst” or “smtc”

  - FL originally thought that RAN1 needs to define the RRC parameters if “ssb-Periodicity, ssb-PositionsInBurst” based approach is adopted. Otherwise, RAN4 and RAN2 would be more appropriate to do so.

- Necessity of SFN

  - Many companies are not convinced the necessity at this moment, while ZTE has provided their view. FL thinks companies need more time to look at this.

  - RAN4 might be more appropriate to discuss this issue

To wrap up the discussion in RAN1#112bis-e, FL would like to suggest simplified proposals as follows, i.e. SFN issue is simply postponed and focus on SSB power and time domain information for intra-frequency:

[FL Proposal 5-1-7a-v3]

  - RRC parameter ss-PBCH-BlockPower for candidate cells is included in the LTM configuration.

    - FFS: the RRC structure, i.e. where and how this parameter is included in the LTM configuration.

[FL Proposal 5-1-7b-v3]

Choose one option from the following:

- For the configuration of intra-frequency time domain information for L1 measurement RS:

  - Option 1: the choice of “ssb-Periodicity, ssb-PositionsInBurst” or “smtc” is up to RAN2 and/or RAN4, and RAN1 will not work on defining the RRC parameters

  - Option 2: adopt “ssb-Periodicity, ssb-PositionsInBurst” based approach, and RAN1 will work on defining the RRC parameters

  - Option 3: adopt “smtc” based approach, and RRC parameter definition is up to RAN2 and/or RAN4

[Comments to FL Proposal 5-1-7a-v3 and 5-1-7b-v3]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Proposal 5-1-7a-v3 Agree with the main bullet. For the FFS, does it mean we need to spend time to further discuss this? We think it can be left to RAN2. [FL reply] We have to finalize RAN1 RRC parameter list, and this is the only thing RAN1 should do. It seems FFS just gives a wrong expression, so let’s remove this to avoid the confusion. Proposal 5-1-7b-v3 We can accept Option 1 or Option 3. |
| FL | FL would suggest adding “for candidate cells” for further clarity. This has been added in Proposal 5-1-7a-v3. Also, FFS part is removed to avoid the misunderstanding. |
| Lenovo | Proposal 5-1-7a-v3: We support the proposal without FFS. Proposal 5-1-7b-v3: We are fine to leave this discussion to RAN2 and RAN4. We support option 1. |
| Fujitsu | Proposal 5-1-7a-v3: Support. Proposal 5-1-7b-v3: Support. Option 1 is slightly preferred. |
| Spreadtrum | Proposal 5-1-7a-v3: Support Proposal 5-1-7b-v3: We support Option 3 for unified configuration of intra and inter-frequency, can live with Option 1. |
| Huawei, HiSilicon | Support 5-1-7a-v3 For 5-1-7b-v3, the smtc is quite ambiguous. Is it for SSB-MTC/SSB-MTC2 in MO or SSB-MTC-AdditionalPCI in MIMOParam. Some information are overlapped, like the SSB periodicity, while some are different, like PCI in SSB-MTC2 or ssb-PositionsInBurst in SSB-MTC-AdditionalPCI, which seems all necessary from our perspective. For option 2, even if it adopted finally, but I do not think it is RAN1’s expertise to design RRC signalling. So, if we hope to move step forward compared with the agreement in last meeting on the configuration in time domain, maybe just list the description of configuration instead of name of IE. According to my observation, it seems no one object to have “periodicity of SSB” and “indication of SSB(s) to be measured”. We are open to discuss others. |
| vivo | Proposal 5-1-7a-v3: Support. Proposal 5-1-7b-v3: slightly prefer Option 1. |
| CATT | Proposal 5-1-7a-v3: Support. Proposal 5-1-7b-v3: Prefer Option2. We don’t think RAN2 or RAN4 can make a decision on the choice of “ssb-Periodicity, ssb-PositionsInBurst” or “smtc”. We need give the guideline from RAN1 perspective. |
| CMCC5 | Support 5-1-7a-v3 For the 5-1-7b-v3, we are fine with HW’s proposal to further discuss the function but not limited to the terminology. If this way does not work, or not supported by the majority, we can come back to FL’s proposal 5-1-7b-v3 and make a decision. |
| Nokia | Proposal 5-1-7a-v3: Support. Proposal 5-1-7b-v3: slightly prefer Option 1 |
| ZTE | Proposal 5-1-7a-v3: We tend to add a note: how it this parameter captured or which RRC structure this parameter will be captured is up to RAN2. Proposal 5-1-7b-v3: Our 1st preference is option 2. But if it is difficult to choose between option 2 and option 3 in RAN1, we can live with option 1 that left it for RAN2 decision. If we go to option 1, we think it would be better to tell RAN2 which measurement information we want to will be included in the measurement configuration, but not to mention specific parameter names. |
| QC | Support Option 3, which works for both sync and async scenarios |
| Futurewei | We are fine with proposal 5-1-7a-v3. We are fine with proposal 5-1-7b-v3, and both option 1 and 3 are preferred. |

[FL Observation]

FL Proposal 5-1-7a-v3 looks acceptable to the companies who participate to the discussion. Even though addition of a note is proposed, it is not a big deal. Thus, FL would like to propose to go to approval phase.

As for FL Proposal 5-1-7b-v3, it seems companies have no strong motivation to continue RAN1 discussion to define RRC parameters for time domain information of candidate cell SSB. On the other hand, Huawei mentioned that list the description of configuration can be discussed in RAN1 to move one step further from the RAN1#112 agreement. This might be good approach, but there are no contributions proposing the detailed design of RRC parameters for SSB time domain information. Given this situation, FL expects no further progress on Proposal 5-1-7b-v3 in this meeting. Companies are encouraged to check the RAN2 situation, and bring a contribution to RAN1#113, if deemed necessary. With this understanding, the discussion on FL Proposal 5-1-7b-v3 is suspended.

[FL Proposal 5-1-7a-v4 for email approval]

  - RRC parameter ss-PBCH-BlockPower for candidate cells is included in the LTM configuration.

    - Note: how to capture this parameter and RRC structure are up to RAN2

void

L1 measurement reporting

[High] Contents of gNB scheduled L1 measurement reporting

[Conclusion at RAN1#110b-e]

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, further study the following mechanisms:

  - Report as UCI on PUCCH or PUSCH

    - Periodic report on PUCCH, semi-persistent report on PUCCH/PUSCH, and aperiodic report on PUSCH

    - Potential enhancements to Rel-17 ICBM report format to accommodate Rel-18 scenarios, e.g.

      - Inter-frequency measurement, if supported

      - Increasing the maximum number of reported beams, which is 4 for Rel-17 ICBM

      - Flexible size beam report, e.g., two-part UCI (e.g., the 1st part contains the best beam/cell and the number (e.g., N) of reported beams/cells, the 2nd part contains the rest (N-1) beams/cells

      - Reducing the reporting overhead by e.g. choosing beams/cells per frequency or across frequencies to report (FFS how)

  - Report on MAC CE

    - Both gNB scheduled and/or UE initiated (if supported) report are studied

[Conclusion at RAN1#111]

Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

[Conclusion at RAN1#112]

The last version of FL proposal:

- For L1 measurement reporting for LTM,

  - At maximum [4] beams (4 is a starting point, FFS: the values and UE capabilities) from candidate cell(s) [and serving cells] configured for measurement & reporting can be reported in a single report instance

    - FFS whether the configured candidate cell(s) can be activated

    - FFS how to choose the beams to be reported from multiple candidate cells, e.g. from all configured/activated candidate cells, from each candidate cell, from each group of candidate cells, from selected candidate cells

  - [Additionally/At least]1 beam from the serving cell is included in the report instance

    - FFS: always included or depending on the gNB configuration

It was pointed out that the light blue part of 5-2-1-v4 is the most important issue, and the agreement on the numbers is meaningless without knowing how to choose the beams to be reported. In other word, the consensus of the group was that the following discussion should be resolved in the next meeting, and then we can go to the next step discussion (e.g. number of beams, necessity of 2-part report).

Important discussion in RAN1#112bis-e

- FFS whether the configured candidate cell(s) can be activated

- FFS how to choose the beams to be reported from multiple candidate cells, e.g. from all configured/activated candidate cells, from each candidate cell, from each group of candidate cells, from selected candidate cells

[Summary of contributions]

How to choose the beams to be reported by a single report instance

- The measurement resources in a resource set could be divided into multiple groups according to its associated PCI and UE reports for the best N resources in each group at a single reporting instance. (Huawei)

- M strongest beams over all the candidate cell (associated with the same or different) – Nokia

- One beam from each of the M strongest candidate cells. – Nokia

- In a reporting instance, beams from the synched candidate cells (for which the valid TA information is available) are prioritized over the non-synced candidate cells (for which the valid TA information is not available) – Nokia

  - FL note: FL wonders if L1 measurement is performed before synchronization. This aspect needs to be discussed first.

- multiple groups of SSBs among multiple candidate cells are configured for measurements, and one pair of L1-RSRP and SSB index is reported for each group of SSBs. – Fujitsu

- K×M beams, where K is the number of cells need to be reported and M is the number of beams for each cell should be reported – Xiaomi

- choosing the beam for reporting from multiple candidate cells, either from all configured/activated candidate cells or from a selected candidate cells – CMCC

- includes a best L1-RSRP of the current serving cell, ‘M’ best candidate cells with ‘N’ largest measured L1-RSRP value for each cell – Apple

- Intra-freqeuncy: report X best candidate cells with Y best beams per reported candidate cell in the same report – Qualcomm

  - L1 cell-level metric can be linear average

- inter-frequency, report X best candidate cells across all measured frequencies with Y best beams per reported candidate cell in the same report – Qualcomm

  - L1 cell-level metric can be linear average

Whether the report for serving cell beam(s) is always included in a report instance

- Yes: Ericsson

- Up to gNB configuration: CMCC

- No: NTT DOCOMO

Activation of measurement RS

- The purpose is to reduce the number of RSs to be measured from RRC configured RSs (more than 7 cells can be configured, and [7] cells are activated by MAC CE)

  - Support: ZTE, Spreadtrum, Intel, Nokia, Panasonic, CATT, IDC, Apple (for aperiodic CSI), DCM (but larger than 7 for inter-frequency)

Number of beams reported in a single report instance

- Same as Rel-17 ICBM (i.e. 4): Nokia(starting point), Fujitsu (starting point), DCM(for intra-frq)

- Larger than 4: ZTE, vivo, Intel, CATT, DCM (for inter-freq)

Weather intra- and inter-cell are reported in the same report instance

- Yes: Ericsson, vivo, Samsung?, FGI (UE capability), IDC (condition needed)

- No: Samsung?

- FL note: the agreement in RAN1#111 says “In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included”. So, the answer should be yes.

How to differentiate the report: which cell(including inter-frequency)/beam the report is for?

- Index for candidate cell + index of detected SSB (Ericsson)

  - This mechanism is necessary of SSB index for L1 measurement is explicitly configured to a UE

- Bitmap of candidate cells (ZTE)

- RS indices (cell identity is implicitly included): vivo, Xiaomi

Support of 2-stage reporting

- Different level of protection: Samsung

- Achieving flexible-size beam report: vivo, OPPO, [CATT]

  - UE can report the L1 measurement result for the beams whose L1-RSRP is above an L1-RSRP threshold

Quantization of measurement result

- Same as legacy releases: 7 bits for absolute value and 4 bits for differential value (ZTE, Nokia)

Additional information to be reported

- timing offset between different cells (vivo)

[FL observation]

Since the companies proposal are widely spread, we can firstly focus on the very fundamental part of reporting issues. FL would like to propose the following topics for the 1st round discussion, which is aligned with the conclusion of RAN1#112:

- How to choose the beams to be reported by a single report instance

- Whether the report for serving cell beam(s) is always included in a report instance

Based on the input from companies, the proposals can be categorized as follows:

- Option 1-1:

  - Beam selection is performed for each configured (or activated, if introduced) cell, i.e. M best beams for N cells

  - M x N beams are reported in a single report instance

- Option 1-2:

  - Beam selection is performed across all configured (or activated, if introduced),

  - M beams are reported in a single report instance

- Option 1-3:

  - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

- Option 1-4:

  - Beam selection is performed for each group of cells, i.e. M best beams for each group

  - M x O beams are reported in a single report instance, where O is the number of groups

Additionally, we have additional options (2-X) to handle the beams for serving cells.

- Option 2-1: P best serving cell beams are always included in a report instance

  - FFS: value of P, how to include, and necessity of configurability to include or not

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

Since companies’ view are not clear yet, it is not easy to check the majority view. FL plans to gather companies preference on these options, and perform down-selection in this meeting, hopefully.

FYI, the diagram explaining the concept of option 1-X is shown below:

Option 1-1                                 Option 1-2

Option 1-3                                 Option 1-4

[FL Proposal 5-2-1-v1]

For the beam selection for SSB based L1-RSRP measurement report, down-select from the following options:

- Option 1-1:

  - Beam selection is performed for each configured (or activated, if introduced) cell, i.e. M best beams for N cells

  - M x N beams are reported in a single report instance

- Option 1-2:

  - Beam selection is performed across all configured (or activated, if introduced),

  - M beams are reported in a single report instance

- Option 1-3:

  - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

- Option 1-4:

  - Beam selection is performed for each group of cells, i.e. M best beams for each group

  - M x O beams are reported in a single report instance, where O is the number of groups

- Note: Value of L, M, N and O are FFS

- Note: How to include the measurement result for serving cell are further discussed depending on the decision on option 2-1 or 2-2 below:

For the inclusion of serving cell for SSB based L1-RSRP measurement report, down-select from the following options:

- Option 2-1: P best serving cell beams are always included in a report instance

  - FFS: value of P, how to include, and the configurability to include or not

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

Companies are encouraged to provide their views for down-selection.

[Comments to FL Proposal 5-2-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | For the beam selection, we’d like to add other options. The reason is that, for inter-frequency, if only L1-RSRP is used for beam selection, considering potential difference on PL due to different frequencies, it may happen that always the candidate cells on the lowest frequency are selected and reported. Thus, we’d like to consider beam selection per frequency. Two options are added below, but we’re open to consider other detailed options as long as frequency is considered in beam selection. - Option 1-5: - Beam selection is performed per frequency, N frequencies and M beams per frequency are reported - M*N beams are reported in a single report instance - Option 1-6: - Beam selection is performed per frequency - M beams are reported in a single report instance (NW configures the restriction that the N beams should include the beams on at least N frequencies) |
| Ericsson | Option 1-1 is ok, but the overhead may be too large if M is large Option 1-2 is not ok, this would simply lead to that the NW would have to configure more reports – one per cell Option 1-3 – what’s the difference compared to 1-1? Option 1-4 – this could be configured to be 1-1 or 1-2, right? Could be OK, but in the end, we need to design a fixed format that fits in UCI Support option 2-1. P=1 is enough. By NW configuration is OK. |
| Google | One quick question on Option 1-3: What’s the criteria to consider a cell is a best/good cell? |
| Futurewei | Option 1-1 is not preferred, because of singling overhead of measurement reporting for each configured cell, and if some configured candidate cells have much worse signal quality, measurement reports for the corresponding candidate cells are not necessary. Option 1-2 is not clear, this option depends on how to perform the selection, further clarification is needed. We are ok for Option 1-3. And Option 1-2 looks like a special case of Option 1-3. We think no big differences between Option 1-4 and Option 1-3 except for the concept of “group”, which makes it more complicated and not preferred. We are ok for Option 2-1, because the cell switching decision is made based on measurements of both serving cell and candidate cell(s). |
| QC | Support Option 1-3, which reduces overhead by reporting only best X candidate cells Support Option 2-2. Whether reporting serving cell can be up to gNB configuration |
| Apple | Support Opt.1-3. Opt.1-1 is not so clear for us, which seems to limit cell numbers to be configured for measurement based on UE capability. Then, UE reports all measured results in a single report. Opt.1-3 has no limitation on number of configured Cells for measurement, but ‘M*L’ L1-RSRP results are fixed to be included in report. Would better to be clarified by FL. Support Opt.2-1. Share a same view as Ericsson that P=1 is sufficient for HO decision and can be fixed in specification without need of configuration. One potential compromise WF, as QC comments, whether serving cell measurement is included or not is up to NW configuration. |
| Nokia | Option 1-2 with the correct format is not clear to us – does it mean to select M best beams from all the configured cells? If yes, then we can clarify it and then we will be fine with this option. We are OK with Option 1-3, but it needs to be clarified how to determine the quality of a cell, e.g., using the best beam measurement or an average metric calculated using multiple beams? Support Option 2-1, and where the value of P can be configured by the NW. |
| MediaTek | We have a general question on this proposal. For each option, the proposal has statement like “Beam selection is performed for each configured (or activated, if introduced) cell”, and what is the meaning of “configured”? is it “measured”? for example, UE may be configured with several beams and cells for measurement but gNB might only trigger measurement on some of them. In such case, should UE report all the configured cell? or measured cells? The other question is for option 1-1, option 1-3, and option 1-4. For those options, it seems to assume the beams measured in each of the cells will be the same? What if UE only measure cell 1 for 4 beams and cell 2 for 1 beam? To us, probably option 1-2 is the simplest one. To Ericsson: our understanding on the difference between option 1-1 and option 1-3 is option 1-1 reports all the N cells and option 1-3 has a smaller number of L<N cells. For Option 2-1 and option 2-2, we wonder gNB configuration and report scheduling can achieve the same consequence? For example, assume option 1-1 is adopted. Then any report will include serving cell? It seems to us that discussion of Option 2-1 and 2-2 might need to wait for the outcome of option 1-X. |
| ZTE | Regarding Option 1-1, reporting the beam of each configured cell invisibly increases the overhead of reporting signaling and reporting measurement results of certain configured cells to NW may not be useful for making handover decisions at NW side. Regarding Option 1-2, we are fine with this option but some clarifications seem to be needed, such as whether all M beams are from the same cell, or different cell and for the latter, whether the same or different number of beam is for each potential configured or activated cell. Or, whether M beams is from the same or different cells depends on the implementation. Regarding Option 1-3, we are also fine with this option and we tend to support M best beams for each cell of L best cells, not for L best cells. Regarding Option 1-4, it is actually similar to the method that L cells and M beams per cell, e.g., Option 1-1 or Option 1-3. For serving cell, we understand that it can be handed using same method with candidate cell and tend to support that serving cell is always included in a single report instance. |
| Lenovo | - For the first bullet: We are OK with Option 1-2, which is used in R17 ICBM. The configuration and report overhead for Option 1-1 is higher. For Option 1-3, we are not sure how to select the L best cells for configuration. - For the second bullet: We support 2-2 |
| Fujitsu | Support Option 1-4. We think grouping is a flexible way to do the report. Possibly, it can also include per frequency report, e.g., by treating each frequency as a group. Support Option 2-2. In our view, serving cell beams do not have to be always included. |
| vivo | Support Option 1-2. Support Option 2-2. Whether reporting beam(s) from the serving cell can be up to gNB configuration and UE beam selection. If the beam quality of the serving cell is poor, it is unnecessary to report it. |
| Xiaomi | Among all these options, there are two alternatives about the number of beams that should be reported in one report instance: - Alt.1: N beams, no matter how many candidate cells should be reported (option 1-2) - Alt.2: K×M beams, where K is the number of cells that need to be reported and M is the number of beams for each of K candidate cells (option1-1, 1-3, 1-4) To make the discussion easier, maybe we can decide the number of beams should be reported in one report instance first. Then, further discuss how to select these beams. If Alt.1 is supported, then no need to discuss option1-1, 1-3, and 1-4. For issue 2, we support option 2-2. |
| CMCC | Option 1-3 is preferred. The best L cells could be determined by the cell quality which is derived from beam-specific measurement. Option 1-2 is not preferred. If only one beam from Cell A is contained in the M beams report, serving cell still cannot determine whether the Cell A is suitable for the cell switch. The average beam quality or the cell level beam quality is important for the cell switch. If option 1-3 is not supported, option 1-1 could be a 2nd preference. The grouping rule of option 1-4 is not clear and needs clarification. For the inclusion of serving cell, option 2-1 is preferred. The serving cell beam quality could be a reference for cell switch determination. |
| IDCC | We are ok to discuss these options. We think there should be also a distinction between intra and inter-frequency cells. For example, in a report, inter-frequency cells can be reported if certain conditions hold (e.g., inter-frequency cell RSRP is above a threshold). |
| LG | Regarding Option 1-2, the clarification is needed. To our understanding, it seems to reuse the reporting format in Rel-17 ICBM with the maximum number of measurements M (=4) and, for intra-/inter-frequency, M value is to consider the larger value. |
| FGI | Option 1-1 is not preferred due to large signalling overhead. Support Option 1-3 and option 1-4 and the definition of L best cells or grouping criteria might need further discussion. For example, - Option 1-3: - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells - M x L beams are reported in a single report instance - FFS: The definition of best beam and best cell. - Option 1-4: - Beam selection is performed for each group of cells, i.e. M best beams for each group - M x O beams are reported in a single report instance, where O is the number of groups FFS: The definition of each group. |
| Huawei, HiSilicon | For option 1-1 and option 1-2, the meaning of configured cells is not clear. Whether the configured cell is just for the report or it is equal to the cells to be measured. For option 1-3, we are generally fine with it. to our understanding, best cells and best beams are one of the choices. Maybe we just need to specify that a report can include L candidate cells and M beams per candidate cell. For option 1-4 looks like a generalization of option 1-3, but seems no additional benefit compared with option 1-3. For serving cell, option 2-1 is preferred. The exact number of P can be left for FFS. |
| OPPO | It looks like none of the Option 1-1 ~4 will work well since the in all the option, the “best” beams are selected blindly. For intra-cell BM, the UE can report the K best beam no matter the absolute RSRP level. But that is not the case for LTM measurement, which is supposed to be used to find good candidate cells. Suggest to update the Options as follows. With the revised options, we prefer Option 1-1. - Option 1-1: - Beam selection is performed for each configured (or activated, if introduced) cell, i.e. M best beams for N cells which satisfy some L1-RSRP condition. - M x N beams are reported in a single report instance - Option 1-2: - Beam selection is performed across all configured (or activated, if introduced), - M beams are reported in a single report instance which satisfy some L1-RSRP condition. - Option 1-3: - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells which satisfy some L1-RSRP condition. - M x L beams are reported in a single report instance - Option 1-4: - Beam selection is performed for each group of cells, i.e. M best beams for each group which satisfy some L1-RSRP condition. - M x O beams are reported in a single report instance, where O is the number of groups |
| Samsung | We should select the option that has the least overhead and provides sufficient information to the network. This seems to be option 1-2, which considers the best M beams across all cells. This would include the serving cell as well as the non-serving cells. Reporting M beams for each cell or for the M best cells, might lead to large overhead without providing additional reporting information that is beneficial. For issue 2, we prefer option 2-2. |
| CATT | Support Option 1-2, since the number of reported beam is less than other options. The Option 1-2 has less spec impact. But the current Option 1-2 is incomplete. I try to complete it as following: - Option 1-2: - Beam selection is performed across all configured (or activated, if introduced) cells, i.e. M best beams for all cells - M beams are reported in a single report instance Regarding the inclusion of serving cell, we prefer Option 2-2. |
| NTT DOCOMO | We check the input from companies. And it seems that other companies did not consider the impact of frequency on beam selection. On the other hand, we realized that Option 1-4 can be used for per-frequency beam selection by grouping concept. Thus, we’d like to add an example on the grouping in Option 1-4. - Option 1-4: - Beam selection is performed for each group of cells, i.e. M best beams for each group (e.g., candidate cells on the same frequency can be configured in one group) - M x O beams are reported in a single report instance, where O is the number of groups |

[FL observation]

For Option 1-X discussion, Companies view for each option (with clarification/modification) is summarized as follows:

- Option 1-1:

  - OK (2): Ericsson, CMCC(2nd)

  - Concern (4): Futurewei, ZTE, Lenovo(Overhead), FGI

  - Beam selection is performed for each configured to be measured (or activated, if introduced) cell, i.e. M best beams for N cells

  - M x N beams are reported in a single report instance

- Option 1-2:

  - OK (7): Nokia, MediaTek, ZTE, Lenovo, vivo, Samsung, CATT

  - Concern (3): Ericsson, Futurewei(not clear), CMCC

  - Beam selection is performed across all cells configured to be measured (or activated, if introduced),

  - M beams (irrespective of number of cells, N) are reported in a single report instance

- Option 1-3:

  - OK(8): Futurewei, Qualcomm, Apple, Nokia, ZTE, CMCC, FGI, Huawei

  - Concern:

  - Beam selection is performed across the L best cells from configured to be measured (or activated, if introduced) cells, i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

- Option 1-4:

  - OK(7): Ericsson, ZTE?, Fujitsu, IDCC(distinction between intra- inter-frequ), FGI, Huawei (but no benefit over Option 1-3?), DCM(w/ addition of e.g. part)

  - Concern(1): Futurewei (complicated)

  - Beam selection is performed for each group of cells to be measured, i.e. M best beams for each group (e.g., candidate cells on the same frequency can be configured in one group)

  - M x O beams are reported in a single report instance, where O is the number of groups

Also, FL reply to the questions/proposals from companies are provided below:

- DCM: Option 1-5/1-6  Option 4 with clarification

  - FL: thanks for the compromise! The clarification to Option 1-4 looks fine

- Ericsson: Difference between Option 1-1 and 1-3?

  - FL: For option 1-3, firstly L best cells are chosen by a criteria (FFS the exact definition). Then M best beams can be reported for the L cells. Meanwhile option 1-1, L best cell selection is not performed, thus the beams of all the configured (or activated, if introduced) cells will be eventually reported.

- Google, Nokia: criteria to choose best/good cells?

  - FL: You can check the contribution from Qualcomm. For example, cell average can be used. But FL thinks this can be FFS and the details can be left to the next meeting.

- Futurewei, Nokia: Option 1-2 is not clear

  - FL: It’s very clear and easy, just choose the beams among cells which RSRP values are the best. In this case, the report beam may be chosen from the same cell. This situation may happen

- Apple: Option 1-1 is not clear

  - FL: “Opt.1-1 is not so clear for us, which seems to limit cell numbers to be configured for measurement based on UE capability. Then, UE reports all measured results in a single report” is also FL’s understanding. However, limitation by UE capability is common to all the options, in my understanding.

- MediaTek, Huawei: “Beam selection is performed for each configured (or activated, if introduced) cell”, and what is the meaning of “configured”? is it “measured”?

  - FL: For option 1-1, it should be “configured to be measured”, but “configured to be measured” and “configured to report” should be identical. Otherwise, selection from measured cell is needed, then it would be Option 1-3. For option 1-2, is should be “configured to be measured” as well.

- MediaTek: What if UE only measure cell 1 for 4 beams and cell 2 for 1 beam?

  - FL: Restriction on the beams from a cell is something we can further discuss, but FL thinks this is an exceptional case, so let’s split the discuss to make the argument easier.

- MediaTek: Option 2-1 and 2-2 might need to wait for the outcome of option 1-X.

  - FL: agree in some sense. Maybe the impact/mechanism of option 2-X is deferent depending on the choice of Option 1-X. FL believes that 2-X is a kind of “concept/spirit”, so separate but parallel discussion would be more efficient.

- Xioami: To make the discussion easier, maybe we can decide the number of beams should be reported in one report instance first.

  - FL: No. It is not recommended. The current discussion formulation was decided in the previous meeting.

- CMCC: The grouping rule of option 1-4 is not clear and needs clarification.

  - FL: it is up to the scheduler implementation. For example, as DCM mentioned, two groups can be made: one is for intra-frequency and the other is for inter-frequency.

- LG: it seems to reuse the reporting format in Rel-17 ICBM

  - FL: the answer may be yes or no. It also depends on other discussion, e.g. necessity of cell index. However, it is true that Option 1-2 is “closer” to the legacy format than others.

- Oppo: introduction of “which satisfy some L1-RSRP condition.”

  - FL: this is a separate issue let’s decouple the discussion. Otherwise, the discussion will be just complicated.

Counting the number of support and concern, FL would like to suggest performing further down-selection from Option 1-3 and Option 1-4.

For Option 2-X discussion, Companies view for each option (with clarification/modification) is summarized as follows:

- Option 2-1: P best serving cell beams are always included in a report instance

  - OK (7): Ericsson (P=1, configurability is OK) , Futurewei, Qualcomm (configurability is OK), Apple (P=1, configurability is OK), Nokia(P=configurable), ZTE(always included), Huawei (P=FFS)

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

  - OK (7) Qualcomm, Lenovo, Fujitsu, vivo, Xioami, Samsung, CATT

  - Concern

It is not an easy decision because the companies view is equally split. However to move forward, FL would like to suggest a compromise based on Option 2-1.

[FL Proposal 5-2-1a-v2]

For gNB scheduled SSB-based L1 measurement reporting, choose one option from Option 1-3 and 1-4

- Option 1-1:

  - OK (2): Ericsson, CMCC(2nd)

  - Concern (4): Futurewei, ZTE, Lenovo(Overhead), FGI

  - Beam selection is performed for each configured to be measured (or activated, if introduced) cell, i.e. M best beams for N cells

  - M x N beams are reported in a single report instance

- Option 1-2:

  - OK (7): Nokia, MediaTek, ZTE, Lenovo, vivo, Samsung, CATT

  - Concern (3): Ericsson, Futurewei(not clear), CMCC

  - Beam selection is performed across all cells configured to be measured (or activated, if introduced),

  - M beams (irrespective of number of cells, N) are reported in a single report instance

- Option 1-3:

  - OK (8): Futurewei, Qualcomm, Apple, Nokia, ZTE, CMCC, FGI, Huawei

  - Concern (0):

  - Beam selection is performed across the L best cells from configured cells to be measured (or activated, if introduced), i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

  - FFS: the criteria how to choose the L best cells

- Option 1-4:

  - OK (7): Ericsson, ZTE?, Fujitsu, IDCC(distinction between intra- inter-frequ), FGI, Huawei (but no benefit over Option 1-3?), DCM(w/ addition of e.g. part)

  - Concern (1): Futurewei (complicated)

  - Beam selection is performed for each group of cells to be measured, i.e. M best beams for each group (e.g., candidate cells on the same frequency can be configured in one group)

  - M x O beams are reported in a single report instance, where O is the number of groups

- Note: Value of L, M, N and O, configurability and UE capability are FFS

- Note: How to include the measurement result for serving cell are further discussed depending on the decision on option 2-1 or 2-2:

Companies are encouraged to provide their preference on Option 1-3, 1-4, or modification for compromise proposal.

[FL Proposal 5-2-1b-v2]

For the inclusion serving cell measurement results for gNB scheduled L1 measurement reporting, Option 2-1 and 2-2 are adopted, and it’s configurable by the network   is adopted [FL note: modified after QC’s comment to make to FL intention clear]

- Option 2-1: P best serving cell beams are always included in a report instance

  - The inclusion of serving cell measurement result is configurable

  - Value of P, configurability and UE capability are FFS

  - How to include the serving cell measurement result, if configured, in Option 1-X is FFS

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

[Comments to FL Proposal 5-2-1a-v2 and 5-2-1b-v2]

| Company | Comment |
| --- | --- |
| QC | For 1a, fine to down select among the two For 1b, since both options are half to half , suggest to support both options with gNB to configure which option to use [FL reply] For 1b, Yes, That’s definitely the FL’s intention by the fisrst sub-bullet, but the wording was not crystalclear. FL proposal 5-2-1b-v2 is not modified with blue font. |
| Nokia | For the 5-2-1a-v2, we support Option 1-2 or Option 1-3. Based on our reading of the above comments from several companies, the Option 1-2 seems to be preferred by more companies than the Option 1-4, so not sure why Option 1-2 was not short listed. Between option 1-3 and option 1-4, we support option 1-3. For the 5-2-1b-v2: Support, but slightly prefer only Option 2-1. |
| ZTE | From our point of view, we are not clear about the benefits of Option 1-4 compared to Option 1-3. Option 1-4 gives the impression that a group is defined based on frequency. We are not sure we really need to consider the frequency information of cell in report. Actually, it seems sufficient to inform gNB which cells and which beams in the reported cell are relatively best. |
| MediaTek | We still believe option 1-2 should be baseline since it is legacy mechanism. In fact, Option 1-2 is a special case of option 1-4 when there is only one group? Among those options, option 1-2 has less parameter to be configured and it should be the baseline. For option 1-3, the discussion of criteria for L is expected to be lengthy and we don’t prefer to support such option in this late WI stage. For option 2-1 and 2-2, if option 1-4 is adopted, then both options will be supported by proper grouping configuration and we don’t need addition proposal? |
| LG | Regarding 1a-v2, we have a similar view with MediaTek and Nokia. As FL assessed, Option 1-2 is closer to the legacy format than others and it could be considered as a baseline. |
| NTT DOCOMO | For 5-2-1a-v2, we prefer Option 1-4. The reason to consider frequency in beam selection is that, since only L1-RSRP is used for beam selection, it may happen that only the candidate cells on the lowest frequency are selected and reported, considering potential PL difference brought by frequency. In addition, in Option 1-4, if NW decides to configure one group only, it is equivalent to Option 1-2. So Option 1-2 is included in Option 1-4. For 5-2-1b-v2, Option 2-2 is slightly preferred. We think there will be L1 beam measurement/reporting for serving beams only under ServingCellConfig. So beam reports for serving cells can be obtained by NW anyway. |
| Huawei, HiSilicon | For 1a-v2, fine to keep two options for further down selection. We support option 1-3 and do not support option 1-4. If there is only one cell per group in option 1-4, it falls back to option 1-3. We think option 1-4 is overdesign. As for the criterion for “best”, we do not think it necessary to be specified. It is UE’s implementation. For 1b-v2, if there no differentiation for serving cell and candidate cell, does current report format in R17 ICBM can be reused? |
| Xiaomi | For 5-2-1a-v2: We support option 1-3. For 5-2-1b-v2: We support option 2-2. From our understanding, whether serving cell is one of the L cells can be used to make cell switch decision for NW. Because whether the results of serving cell is reported or not already provided the information of the results of serving cell, which is the results of serving cell are worse than that of these L cells. |
| IDCC | 1a: Fine with the alternatives. 1b: Support option 2-2. |
| Samsung2 | For Proposal 5-2-1a-v2. We still prefer option 1-2. We think this is alternative that provides the necessary solutions with the least overhead. We have concern on the overhead of the other alternatives. For Proposal 5-2-1b-v2, we prefer to only support option 2-2. |
| CATT | For Proposal 5-2-1a-v2, we can live with the down selection. Regarding the Option 1-3 and 1-4, we prefer Option 1-3. We don’t think it is necessary to divide cells into different groups, i.e., Option 1-4. For Proposal 5-2-1b-v2, we don’t think it is necessary to support both options. Suggest to down selection either Option 2-1 or Option 2-2. |
| Lenovo | For 5-2-1a-v2: We prefer option 1-3. But an FFS is needed on the selection or L best cells. - Option 1-3: - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells - M x L beams are reported in a single report instance - FFS: How to select the L best cells. For 5-2-1b-v2: We support 2-2. |
| Futurewei | We are ok with FL Proposal 5-2-1a-v2. More specifically, we prefer Option 1-3. We think no big differences between Option 1-4 and Option 1-3 except for the concept of “group”, which makes it more complicated and not preferred. We are ok with FL Proposal 5-2-1b-v2. More specifically, we prefer Option 2-1, because the cell switching decision is made based on measurements of both serving cell and candidate cell(s). |

[FL observation]

For proposal 5-2-1b-v2, companies views are summarized as follows

- [Option 1-2: FL suggested dropping it]

  - Support: Nokia, MTK, LG, Samsung (Nokia, MediaTek, ZTE, Lenovo, vivo, Samsung, CATT from first round)

    - Similar approach with ICBM

    - Less parameters for configuration (simple)

    - Less overhead

  - Concern: Ericsson, CMCC, [Futurewei?]

    - The beams may consist of only one cell, which read to have multiple report configuration

- Option 1-3

  - Support: Nokia, Huawei, Xiaomi, CATT, Lenovo (Futurewei, Qualcomm, Apple, Nokia, ZTE, CMCC, FGI, Huawei from first round?)

    - The definition best-cell criteria is not necessary, UE implementation

  - Concern: MTK

    - Potential lengthy discussion to define best-cell criteria

- Option 1-4

  - Support: DCM (Ericsson, Fujitsu, IDCC, FGI from first round)

    - To differentiate frequency

    - Depending on the configuration, Option 1-2 is also possible

  - Concern: ZTE, Huawei

    - Necessity to differentiate the frequency is not clear

    - Overdesign(complexity)

Seeing from the companies view and reasons why the companies support/concern, FL thinks going with option 1-3 would be best way. Meanwhile, FL see the need to address the concern from DCM, i.e. the cells with lower frequency is always chosen due to the use of RSRP. FL can add this aspect for further study for option 1-3.

Regarding the issue on overhead, FL is not quite sure if we can really keep the same overhead as ICMB because the number of cells naturally increase due to the support of inter-frequency.

For proposal 5-2-1b-v2, companies views are summarized as follows

- Option 2-1 and 2-2 (configurable)

  - QC, Nokia (OK), Futurewei

- Option 2-1 only

  - Nokia (slightly prefer), CATT(choose one option), Futurewei

  - Note: result from 1st round (Ericsson (P=1, configurability is OK) , Futurewei, Qualcomm (configurability is OK), Apple (P=1, configurability is OK), Nokia(P=configurable), ZTE(always included),)

- Option 2-2 only

  - DCM (serving cell report can be obtained by different way), Xiaomi (if serving cell report is not included, then it means the quality of serving cell is bad), IDCC, Samsung, CATT(chose one option), Lenovo

  - Note: result from 1st round (Qualcomm, Lenovo, Fujitsu, vivo, Xioami, Samsung, CATT)

After seeing the discussion so far, FL didn’t quite catch the reason why Option 2-2 alone can be the best solution:

- DCM’s comment (serving cell report can be obtained by different way) is somewhat understandable. However, for the gNB, it may be useful so some implementation if the necessary information is obtained from one report instance. That’s why FL proposed the configurability between 2-1 and 2-2.

- The comment from Xiaomi (if serving cell report is not included, then it means the quality of serving cell is bad) may not be valid for some case. One good example is inter-frequency LTM. Better RSRP for the cell with different frequency wouldn’t justify inter-freq handover. It is important to know how bad the serving cell quality is (i.e. service continuity by the serving cell can be guaranteed or not)

- The argument by CATT is understandable as well, but FL cannot find any good way forward.

Given this analysis, FL believes 5-2-1b-v2 would be the best what we can do.

[FL Proposal 5-2-1a-v3]

For the beam selection for SSB based L1-RSRP measurement report, adopt option 1-3:

- [Option 1-2: FL has suggested dropping it after 1st round]

  - Beam selection is performed across all configured (or activated, if introduced),

  - M beams are reported in a single report instance

  - Support: Nokia, MTK, LG, Samsung (Nokia, MediaTek, ZTE, Lenovo, vivo, Samsung, CATT from first round)

    - Similar approach with ICBM

    - Less parameters for configuration (simple)

    - Less overhead

  - Concern: Ericsson, CMCC, [Futurewei?]

    - The beams may consist of only one cell, which read to have multiple report configuration

- Option 1-3

  - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells

  - M x L beams are reported in a single report instance

  - FFS whether and how to define the L best cell criteria

  - FFS whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting

  - Support: Nokia, Huawei, Xiaomi, CATT, Lenovo, Futurewei (Qualcomm, Apple, Nokia, ZTE, CMCC, FGI, Huawei from first round?)

    - The definition best-cell criteria is not necessary, UE implementation

  - Concern: MTK

    - Potential lengthy discussion to define best-cell criteria

- Option 1-4

  - Beam selection is performed for each group of cells, i.e. M best beams for each group

  - M x O beams are reported in a single report instance, where O is the number of groups

  - Support: DCM (Ericsson, Fujitsu, IDCC, FGI from first round)

    - To differentiate frequency

    - Depending on the configuration, Option 1-2 is also possible

  - Concern: ZTE, Huawei

    - Necessity to differentiate the frequency is not clear

    - Overdesign(complexity)

- Note: Value of L and M, N and O, configurability and UE capability are FFS

- Note: How to include the measurement result for serving cell are further discussed depending on the decision on option 2-1 or 2-2 below:

[FL Proposal 5-2-1b-v3]

FL suggests the same proposal, i.e. both option 2-1 and 2-2 with configurability because the companies comments for down-selection were not convincing.

For the inclusion serving cell measurement results for gNB scheduled L1 measurement reporting, both Option 2-1 and 2-2 are adopted, and it’s configurable by the network

- Option 2-1: P best serving cell beams are always included in a report instance

  - Value of P, configurability and UE capability are FFS

  - How to include the serving cell measurement result, if configured, in Option 1-X is FFS

- Option 2-2: do not differentiate the beams depending on serving or candidate cell

[FL observation]

In Wed GTW, A good progress was achieved on this topic, i.e. now option 1-3 can be the way to go. Meanwhile, we have a lot of issues to address with Option 1-3 based solution. FL thinks the final form of the proposal in the GTW session can be the starting point for the next round of discussions.

[FL Proposal 5-2-1-v4]

Starting point of the next round of discussion

For the beam selection for SSB based L1-RSRP measurement report,

- Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells

- M x L beams are reported in a single report instance, and M and L is configurable based on UE capability of maximum value of M and L

- FFS whether and how to define the L best cell criteria

- FFS whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting

- Open issue 1: L best cell criteria

  - We should avoid the potential lengthy discussion to define this criteria

  - Can “leave it to UE implementation” be a solution?

- Open issue 2: whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting

  - This has already been captured in 2nd FFS, but more detail discussion in this meeting is highly appreciated for the progress.

- Open issue 3: ease the UE implementation by reusing the mechanism of Rel-17 ICBM L1-measurement reporting

  - Option 1-3 may increase the complexity for UE side, so some baseline would be useful not to require drastic change for UE implementation

  - One solution would be limiting the value of M, L plus introducing UE capability

  - Another solution is to introduce both option 1-2 and 1-3. This can ease UE implementation, but may not be useful for the network because of the functional fragmentation of the UE.

- Open issue 4: whether or how to include the serving cell report in a single report instance

  - This is the issue discussed under FL Proposal 5-2-1b, but FL thinks it is time to merge the discussion

  - For example, if the minimum UE capability is limited to L=1, there is no room to include serving cell report in the report. This is not useful for some scheduler implementation.

  - Or, Option 1-3 can be simply extended such that serving cell report is always included, if the necessity is agreed.

Companies are encouraged to provide their views how to modify the starting point proposal to address the open issues above.

[Comments to FL Proposal 5-2-1-v4]

| Company | Comment |
| --- | --- |
| NOKIA | Considering all the open issues mentioned by the FL, we propose the following proposal. We think this can at least resolve issue 3 and issue 4. Issue 1 and issue 2 are already in FFS. For the beam selection for SSB based L1-RSRP measurement report, - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for each of the L best cells - M x L beams are reported in a single report instance, and M and L is configurable based on UE capability of maximum value of M and L - Max values of M and L are based on UE capability, - The values of M and L are configured to the UE in the reporting configuration - Whether beams (M beams) from the serving cell need to be included in the report is configured to the UE in the reporting configuration - FFS whether and how to define the L best cell criteria - FFS whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting |
| NTT DOCOMO | We respect FL’s proposal on discussion based on Option 1-3. We just tried to raise another possibility to resolve the 4 issues for discussion. For Issue 1/2/4: In the current proposal, if we can change ‘L best cells from configured cells’ to ‘L best groups from configured groups’, actually it covers all of Option 1-2, 1-3 and 1-4. And NW has the full flexibility in configuration. And we can leave the criteria of L best groups to UE implementation. In addition, whether serving cell report is included or not can be also RRC configured. In this case, both NW and UE have good flexibility. - E.g., if NW configures one group only (all the configured cells in one group, L=1), it becomes Option 1-2. - E.g., if NW configures one group to include one cell only, it becomes Option 1-3. For Issue 3: We understand the preference of reusing the mechanism of Rel-17 ICBM L1-measurement reporting. In our understanding, only beam selection rule is changed in above proposal, while the beam reporting format could reuse the Rel-17 ICBM beam reporting, regardless of whether grouping is configured or not. |
| MediaTek | Thanks to FL’s new proposal and Nokia’s suggestion. We are still not convinced that the new beam selection for reporting rule is useful for all the scenario and we don’t need the legacy rule in any case. In addition, just want to remind companies that Rel-17 ICBM can’t support such reporting format and it might not be possible to support simultaneous operation of Rel-17 ICBM and Rel-18 LTM. On the other hand, Nokia’s proposal is a good suggestion to address our concern on option 1-3. On top of that, we suggest the selection of M beams and L cells is up to UE implementation to minimize the discussion. As for the value of M and L, we would like to emphasis that 4 is the max number of reported beam in a single report in Rel-17 ICBM and we think M*L should keep the same ue capability maximum to avoid large variant of payload size(even though we haven’t agreed on the using PUCCH as container) |
| Huawei, HiSilicon | For issue 1: we think the best beam selection is up to UE implementation. Similar concept is adopted in r17 ICBM where how to select 4 beams in one report is not specified. For issue 2: The current wording is not clear or at least not reflect what DCM hope to express. To my understanding, it is also a best beam selection criterion but in inter frequency scenario. So we would suggest following updates to leave how UE select beam to implementation - Beam selection is performed across the L best cells from configured (or activated, if introduced) cells, i.e. M best beams for L best cells - M x L beams are reported in a single report instance, and M and L is configurable based on UE capability of maximum value of M and L - How to select the L cells and M beams per cells is up to UE implementation - FFS whether and how to define the L best cell criteria - FFS whether and how to address situation where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting For issue 3: We assume the report mechanism and result for r17 ICBM can also be used in r18 LTM, especially if ICBM and LTM can operate simultaneously. The introduction option 1-3 is to ensure there is a report mode to ensure UE to report measurement result for candidate cells. For issue 4: We are fine with direction of compromise mentioned by FL. However, whether the result of serving cell is included should depend on the configuration instead of capability. For example, if UE is configured with L>1, results for serving cell should be included. If L=1, then only candidate cell result can be configured. |
| LG | As Huawei mentioned, the selection of the best beam is based on UE implementation and the selection of L best cells also be the same (i.e., only for the number of M/N is mentioned if needed). Based on UE capability, to support Rel-17 ICBM and Rel-18 LCM, the max number of measurement is maintained up to 4. |
| Lenovo | We agree with companies that the beam and cell selection can be leaved to UE implementation to simplify the discussion. Further, we think Rel-17 ICBM should be the baseline the beam report for LTM. Then the max number of reported MxL beams is 4. Another comment is that we are not sure whether Rel-18 differential L1-RSRP report can be reused since the beam quantities of different cells may be quite different especially for inter-frequency measurement. |
| ZTE | For this proposal, we have the following comments: Comment #1: we would like to confirm that M beams is from L cells or each of L cells. Comment #2: For issue 1, if we do not define whether M beams are the best in each cell or the best in L cells, it may result in reported measurement results that are not optimal, which may affect NW’s decision switching to relatively good cells. So we prefer to add like“best” wording. Comment #3: Regarding M and L value, we support the modification raised by Nokia. Comment #4: For issue 4, regarding whether measurement result from serving cell is included in report, we tend to adapt same operation as L3 report that measurement result from serving cell is always included. Comment #5: For issue 2, to be honest, I really don't understand what this situation (i.e., where the cells with the same, e.g. lowest, frequency tends to be chosen for reporting) is. Comment #6: For issue 3, Does “reusing the mechanism of Rel-17 ICBM L1-measurement reporting” refer to the report containing only beam information and corresponding RSRP, not including cell information? |
| Futurewei | For issue 1, how to select L best cell really can be based on UE implementation, but some basic rules need to be guided by RAN1, for example, based on L1-RSRP value, L1-SINR, or other type of values, etc. For issue 4, serving cell report is definitely needed for making cell switching decision, we prefer in the same report instance from latency perspective, if possible, e.g., measurement report resources can carry all the measurement results in one instance. |

[FL Proposal 5-2-1-v5]

For the beam selection for SSB based L1-RSRP measurement report,

- Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells

  - How to select the L cells and M beams per cells is up to UE

- M x L beams are reported in a single report instance

  - Max values of M and L are based on UE capability, and at least M*L=4 is supported as a UE capability, other UE capabilities are FFS

  - The values of M and L are configured to the UE in the reporting configuration

- The following configurability is introduced in the report configuration

  - 1) serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2

  - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency

  - FFS: 1) and 2) are exclusive

[Comments to FL Proposal 5-2-1-v5]

| Company | Comment |
| --- | --- |
| Lenovo | We agree with companies that the beam and cell selection can be leaved to UE implementation to simplify the discussion. Further, we think Rel-17 ICBM should be the baseline the beam report for LTM. Then the max number of reported MxL beams is 4. Another comment is that we are not sure whether Rel-18 differential L1-RSRP report can be reused since the beam quantities of different cells may be quite different especially for inter-frequency measurement. |
| Fujitsu | Support |
| Spreadtrum | We support the first two bullet. According to the last bullet, it needs further clarify: - If two of them are configured, how to do best cell selection. - It can also up to UE implementation to select Inter-frequency cells, such as in the scope of “How to select the L cells and M beams per cells is up to UE”, why need to do extra configuration? |
| Huawei, HiSilicon | We are generally fine with the proposal except the subbullet of 2) under the 3rd bullet. In my opinion, the inter frequency cell issue may not be solved using the similar way as the issue for serving cell. If we forced UE to report a inter frequency cell, overhead of feedback is increased in order indicate large dynamic range of RSRP if differential RSRP from largest RSRP are reported (similar coding scheme as in r17 ICBM), considering the large pathloss difference. |
| vivo | We are fine with the first two bullet. As for the second bullet, we do understand why the measurement result of serving cell must be reported when the value of L is larger than 1. In our view, if no measurement result of the serving cell is selected and reported, it means the quality of serving cell is poor and the network needs to perform handover based on the reported measurement results. |
| CATT | The first two bullets are fine for us. For the 3rd bullet, we wonder why at least one of the inter-frequency cells is always selected when the value of L is larger than 1. We think if the best L cells are all belong to intra-frequency cells, it’s not necessary to report a inter-frequency cell. Moreover, we don’t think the configurability method is needed. We can just select one in the report configuration. |
| CMCC5 | Thanks for FL’s updates. For the 1st and 2nd bullets we are generally fine. We support to define the Max values of M and L as a UE capability, and at least M*L=4 is supported as a UE capability. If the M*L is larger than 4, we may need some new design for L1-RSRP reporting. For the 3rd bullet, we have no problem to always include the serving cell for the measurement and report. But for the inter-frequency cell, we do not support to mandate at least one inter-frequency cell in the report. Firstly, if we reuse the capability of ICBM of 4 beam for reporting, then we have one L1-RSRP from serving cell, at least one from inter-frequency cell, then 2 L1-RSRP left. As we commented in the previous round, only one beam is reported from one cell. Then the gNB have no observation for the beam quality in the cell level. One question to FL, based on current proposal, it seems we do not strive for UE to report the best M beams from the best L cells ? The selection of M beams and L cells depends on UE’s implementation. Is this understanding correct ? For the inter-frequency beam measurement, it can further depend gNBs’ configuration, whether a inter-frequency cell should be always reported. Since we begin to consider the difference of different frequency, are we going to consider the different transmit power of different cells ? Furthermore, if we support to report both serving cell beams and candidate cell beams, it should be clarify that how can we distinguish which beam is from the serving cell and which is from the candidate cells. |
| Nokia | Fine with the first two bullets. For the 3rd bullet, as other companies also mentioned, why there is a specific need to ask the UE to do inter-frequency measurement? We don’t need the 2nd and 3rd sub-bullet of the 3rd bullet. Otherwise, the proposal looks good. |
| ZTE | We are generally fine with the proposal, except the 2) of 3rd bullet. We don’t think it is necessary to reflect inter-frequency for reported cells. |
| Samsung3 | We have concern for overhead. We prefer to have M beams across all candidate cells. The report should include beams for L cells, it is up to the UE determine the number of beams for each of the L cells. Always having M beams for each reported cells might lead too high overhead. For the beam selection for SSB based L1-RSRP measurement report, - Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams across for each of the L cells - How to select the L cells and M beams per cells is up to UE - M x L beams for L cells are reported in a single report instance - Max values of M and L are based on UE capability, and at least M*L=4 is supported as a UE capability, other UE capabilities are FFS - The values of M and L are configured to the UE in the reporting configuration - The following configurability is introduced in the report configuration - 1) serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency - FFS: 1) and 2) are exclusive |
| QC | For Issue 1: We prefer NOT to leave to UE implementation. Otherwise, UE can report 4 best beams from a single cell. One simple definition is to rank based on best beam per candidate cell. For Issue 2: We think it may not be very critical. Picking best X cells across all frequencies seem reasonable. Otherwise, gNB can use separate report per frequency based on implementation For Issue 3: limiting M and L is fine. Introducing Option 1-2 and 1-2 does not fit into the purpose of cell switch well For Issue 4: We think supporting both options should be flexible for gNB scheduling. Agree that if serving cell is included, L should at least be 2 |
| Futurewei | We are fine with the proposal, except for 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency, which means one inter-frequency cell must be selected even not being one of the best cells, and it doesn’t make sense. Can you please clarify it? |

[FL Observation]

FL reply to the companies’ comments is available below:

- Lenovo

  - we are not sure whether Rel-18 differential L1-RSRP report can be reused since the beam quantities of different cells may be quite different especially for inter-frequency measurement

    - FL: This is not the scope of this discussion, but something RAN1 should study in the future meeting.

- Spreadtrum

  - If two of them are configured, how to do best cell selection.

    - FL: It’s up to UE implementation

  - It can also up to UE implementation to select Inter-frequency cells, such as in the scope of “How to select the L cells and M beams per cells is up to UE”, why need to do extra configuration

    - FL: The proposals in the 3rd sub-bullets are allow certain level of gNB control.

- Vivo

  - we do understand why the measurement result of serving cell must be reported when the value of L is larger than 1.

    - FL: Firstly, this is an additional feature configured by gNB. This will happen only when gNB configures to do so. In my understanding, the HO event, if the gNB want to use Event A2 (Serving becomes worse than threshold) like trigger for LTM, gNB needs to know RSRP for serving cell because event triggered report is not supported at this moment.

- CATT, (Huawei, CMCC, Nokia, Qualcomm, Futurewei and ZTE have similar comment)

  - we wonder why at least one of the inter-frequency cells is always selected when the value of L is larger than 1

    - FL: it seems companies are not convinced to separate the report based on frequency. FL suggestion is to put FFS.

- CMCC

  - One question to FL, based on current proposal, it seems we do not strive for UE to report the best M beams from the best L cells ? The selection of M beams and L cells depends on UE’s implementation. Is this understanding correct ?

    - FL: Yes. The decision is up to UE. My understanding is that the specs (and UE tests) cannot guarantee that UE always choose the best cells/beams, but the UE implementation does the best.

- Qualcomm

  - We prefer NOT to leave to UE implementation. Otherwise, UE can report 4 best beams from a single cell. One simple definition is to rank based on best beam per candidate cell.

    - FL: unfortunately, this is a single companies view. Let’s check if companies are OK to specify something.

- Samsung

  - We have concern for overhead. We prefer to have M beams across all candidate cells. The report should include beams for L cells, it is up to the UE determine the number of beams for each of the L cells. Always having M beams for each reported cells might lead too high overhead.

    - FL cannot accept this proposal because the idea is Option 1-2, which take us one step back. Also, the concern on overhead has already been addressed by MxL=4, which is identical to ICBM.

    - Regarding “the UE determine the number of beams”, this should be separately discussed. Mixing the issues wouldn’t be good for the progress.

[FL Proposal 5-2-1-v6]

For the beam selection for SSB based L1-RSRP measurement report,

- Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells

  - How to select the L cells and M beams per cells is up to UE [FL note: is Qualcomm OK]

- M x L beams are reported in a single report instance

  - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS

    - FFS if UE is allowed to report less than M x L beams [FL note: proposal by Samsung]

  - The values of M and L are configured to the UE in the reporting configuration

- The following configurability is introduced in the report configuration

  - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2

  - 2) FFS at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency [FL note: concern by multiple companies were shown]

  - FFS: 1) and 2) are exclusive

[Mid] Container of gNB scheduled L1 measurement reporting

[Conclusion at RAN1#110b-e]

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, further study the following mechanisms:

  - Report as UCI on PUCCH or PUSCH

    - Periodic report on PUCCH, semi-persistent report on PUCCH/PUSCH, and aperiodic report on PUSCH

    - Potential enhancements to Rel-17 ICBM report format to accommodate Rel-18 scenarios, e.g.

      - Inter-frequency measurement, if supported

      - Increasing the maximum number of reported beams, which is 4 for Rel-17 ICBM

      - Flexible size beam report, e.g., two-part UCI (e.g., the 1st part contains the best beam/cell and the number (e.g., N) of reported beams/cells, the 2nd part contains the rest (N-1) beams/cells

      - Reducing the reporting overhead by e.g. choosing beams/cells per frequency or across frequencies to report (FFS how)

  - Report on MAC CE

    - Both gNB scheduled and/or UE initiated (if supported) report are studied

[Conclusion at RAN1#111]

Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

[Conclusion at RAN1#112]

Due to the lack of time, the following FL proposal was not treated in RAN1#112.

- Periodic and semi-persistent PUCCH are also supported for gNB scheduled L1-measurement reporting.

[Summary of contributions]

Support report on PUCCH

- Periodic: Huawei, Ericsson, ZTE, Nokia, CATT, Samsung, CMCC, Google, LGE, DCM

- Semi-persistent: Huawei, Ericsson, ZTE, Nokia, CATT, CMCC, Google, LGE, DCM

- Not support: None

Support report by MAC CE

- Yes: Intel (if both inter- and intra-frequency measurements are reporting in a single reporting instance), IDC

- No: CATT

[FL observation]

Based on the input from companies, majority supports to introduce periodic and semi-persistent report on PUCCH for gNB scheduled L1-measurement report. In the meantime, it would be good to wait for the initial agreement on the contents discussed in section 5.2.1 because the size of L1-measurement might be too large compared with PUCCH capacity. Therefore, FL suggests to have an email discussion first, then check at the end of RAN1#112bis-e if this proposal agreeable or not.

[FL Proposal 5-2-2-v1]

- Periodic and semi-persistent PUCCH are also supported for gNB scheduled L1-measurement reporting.

[Comments to FL Proposal 5-2-2-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support FL proposal. |
| Ericsson | Support |
| Google | Support FL proposal |
| Futurewei | The exact payload size of measurement report needs to be determined firstly, and then we can decide whether support it or not |
| QC | Support |
| Apple | Support. |
| Nokia | Support |
| ZTE | Support |
| Lenovo | Support |
| Fujitsu | Support |
| vivo | Support. |
| Xiaomi | Support |
| CMCC | Support |
| IDCC | Support |
| LG | Support |
| Huawei, HiSilicon | support |
| Samsung | Support |

[FL Observation]

- Support FL Proposal 5-2-2-v1 (16)

  - NTT DOCOMO, Ericsson, Google, QC, Apple, Nokia, ZTE, Lenovo, Fujitsu, vivo, Xiaomi, CMCC, IDCC, LG, Huawei, Samsung

- Need to wait the discussion on the report contents (1)

  - Futurewei

Clear majority support FL Proposal 5-2-2-v1, i.e. the introduction of PUCCH for gNB scheduled L1 measurement report while one company proposed to check the size of report contents.

FL sees no necessity to further consolidate the FL proposal. And FL would bring this proposal to GTW in this meeting.

[FL Proposal 5-2-2-v2]

No change from FL proposal 5-2-2-v1, and to be proposed hopefully at GTW on Mon 2nd week.

Almost stable proposal

- Periodic and semi-persistent PUCCH are also supported for gNB scheduled L1-measurement reporting.

[Comments if any]

| Company | Comment |
| --- | --- |
| Spreadtrum | Support |
| CMCC5 | Support |
| Samsung3 | Support |

[Mid] Configuration for gNB scheduled L1 measurement report

[Conclusion at RAN1#112]

- Agreement related to an LS to RAN2

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

[Summary of contributions]

- Nokia

  - Reporting configuration for LTM can be a cell specific configuration which includes:

    - reporting configuration providing the uplink resource configuration

    - [measurement gap configuration for inter-frequency measurements ]

      - FL note: is this RAN4 region?

    - a set of initial activated RSs/cells (which can be updated later with MAC-CE) from the common configuration of L1 measurement RSs to consider for measurements.

- DCM

  - Support to explicitly configure cell index for SSB/CSI-RS in CSI configuration signaling.

[FL observation]

While FL believes that the configuration of L1 measurement report is an essential part, no details on are provided in this meeting. Thus, FL would like to kick-off the email discussion aiming at the finalization until RAN1#113. The agreement in RAN1#112 on RS configuration can be the starting point of our discussion.

[FL Proposal 5-2-3-v1]

- For L1 measurement report configuration for gNB scheduled reporting,

  - As a starting point, at least the following information needs to be provided to a UE, e.g.

    - Resource for channel measurement, i.e. SSB and CSI-RS if agreed

      - a set of initial activated RSs/cells, if activation of RSs/cells are introduced

    - Type of report configuration, i.e. periodic/semi-persistent/aperiodic, uplink resource configuration

    - Quantity, if agreed

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are applicable

    - Option 1) Configurations for L1 measurement report is provided under ServingCellConfig for the serving cells

    - Option 2) Configurations for L1 measurement report is provided separately from ServingCellConfig for the serving cells and RRCReconfiguraiton for the candidate cells

    - Option 3) Configurations for L1 measurement report is provided under RRCReconfiguraiton for the candidate cells

Companies are encouraged to provide their view on the FL proposal above

[Comments to FL Proposal 5-2-3-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | OK in principle. Suggest following revision. And the number of beams/cells/groups/frequencies for beam selection/reporting in FL proposal 5-2-1-v1 should be also configured. - Resource for channel measurement with cell index indication, i.e. SSB and CSI-RS if agreed |
| Ericsson | RAN2 is already discussing this. “Resource for channel measurement, i.e. SSB and CSI-RS if agreed” – we think this is part of FL proposal 5.1.7 Only option 1 is reasonable here: the report configuration is clearly part of the serving cell configuration |
| Futurewei | Please let RAN2 make the decision for different options of RRC structure. |
| QC | Fine with FL’s proposal |
| Apple | Our understanding on FL proposal is that it intends to formulate ‘what RRC parameters’ we need to introduce for a ‘L1 measurement report for candidate cell’, which is similar like CSI reporting defined for serving cell: - RS for measurement: clearly needed. - On the report configuration, this is transmitted to serving cell even the content includes candidate cells measurement results. The UL resources e.g., PUCCH or PUSCH have to be reserved by NW under serving cell as the measurement results for candidate cells are reported to serving cell. We may need to discuss some aspects e.g., how to configure these resources even under serving cell as they are used for candidate cell report. ‘Type of report’ is another IE to provide for each report associated with candidate cell. - On ‘Quantity’, so far only SSB-based RSRP is agreed. Unless other metric is specified, this can be fixed in spec. Of course, we can also include it as part of configuration for the future extension e.g., introducing new measurement quantity in Rel-19. |
| Nokia | Fine with the FL proposal |
| ZTE | Suggest to leave it for RAN2 decision. |
| Lenovo | Fine with the FL proposal |
| Fujitsu | Support |
| vivo | Fine with the revision from NTT DOCOMO. |
| CMCC | Fine with the proposal. For the RRC configuration, option 1 is our first preference and then option 2. The issue of option 3 is that the RRC reconfiguration of the candidate cell would be activated after the cell switch command is received, which means the early measurement will not work in this situation. |
| IDCC | Support |
| LG | Fine with the proposal |
| Huawei, HiSilicon | It should be up to RAN2, similar as L1 measurement configuration |
| OPPO | Ok with the proposal |
| Samsung | Fine with proposal. Prefer option (1) |

[FL Observation]

Most of the companies are OK for the direction, while the following comments were provided.

Up to RAN2 (Note: this is also FL’s intention)

- Ericsson, Futurewei, ZTE

Option 1 is preferred (configured under servingCellConfig)

- Ericsson, CMCC, Samsung

Modification to FL proposal

- “Resource for channel measurement with cell index indication”

[FL Proposal 5-2-3-v2]

- For L1 measurement report configuration for gNB scheduled reporting,

  - As a starting point, at least the following RRC parameters information needs to be provided to a UE, e.g.

    - Resource for channel measurement with cell index indication, i.e. SSB and CSI-RS if agreed

      - a set of initial activated RSs/cells, if activation of RSs/cells are introduced

    - Type of report configuration, i.e. periodic/semi-persistent/aperiodic

    - Uplink resource configuration in order to report serving cell and candidate cell L1 measurement result to serving cell

    - Quantity, if agreed

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are applicable are further considered in RAN2 [FL note: some companies think Option 1 is the only way to go. So “applicable” is avoided here]

    - Option 1) Configurations for L1 measurement report is provided under ServingCellConfig for the serving cells

    - Option 2) Configurations for L1 measurement report is provided separately from ServingCellConfig for the serving cells and RRCReconfiguraiton for the candidate cells

    - Option 3) Configurations for L1 measurement report is provided under RRCReconfiguraiton for the candidate cells

    - [FL note: companies don’t see the necessity to list the details of each option. Let’s remove them]

[Comments to FL Proposal 5-2-3-v2]

| Company | Comment |
| --- | --- |
| QC | Fine for FL’s proposal |
| Nokia | OK |
| ZTE | Fine with FL’s proposal |
| LG | Fine with the proposal |
| Huawei, HiSilicon | For the bullet of “Resource for channel measurement with cell index indication, i.e. SSB and CSI-RS if agreed” No sure what does “with cell index indication” refer to? Does it mean cell index associated with the SSB and CSI-RS to be measured? |
| Samsung2 | Fine with latest proposal, with the following comment: Cell index can be applicable to SSB. In some cases, the cell index of CSI-RS is implicit (e.g., based on source RS of TCI state) and not explicitly indicated. |
| Futurewei | We are fine with FL’s proposal. |

[FL observation]

All the companies are OK with the FL proposal, while where is a clarification question on “with cell index indication” which is a proposal by DCM. FL’s understanding is that the intentions is to clarify which candidate cell the measurement resource (i.e. SSB or CSI-RS if agreed) belongs to. FL can try to further clarify the intention

[FL Proposal 5-2-3-v3]

- For L1 measurement report configuration for gNB scheduled reporting,

  - As a starting point, at least the following RRC parameters needs to be provided to a UE, e.g.

    - Resource for channel measurement (i.e. SSB and CSI-RS if agreed) with the indication of associated cell

      - a set of initial activated RSs/cells, if activation of RSs/cells are introduced

    - Type of report configuration, i.e. periodic/semi-persistent/aperiodic

    - Uplink resource configuration in order to report serving cell and candidate cell L1 measurement result to serving cell

    - Quantity, if agreed

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are further considered in RAN2

[Comments to FL Proposal 5-2-3-v3]

| Company | Comment |
| --- | --- |
| Nokia | Just a note here that if we decide to include some parameters in the reporting config based on the proposal 5-2-1, then those need be added as well. |
| NTT DOCOMO | OK |
| Fujitsu | Support |
| Spreadtrum | Support, also agree with Nokia’s comments. |
| Huawei, HiSilicon | support |
| vivo | Support the proposal in principle. |
| ZTE | Support |
| Samsung3 | Fine in principle, with the following update: - Resource for channel measurement (i.e. SSB and CSI-RS if agreed) with the indication of associated cell. Indication can be explicit or implicit. |

[FL observation]

All the companies are OK in principle:

- Nokia

  - if we decide to include some parameters in the reporting config based on the proposal 5-2-1, then those need be added as well.

    - FL agrees. FL believes “at least” in the 1st sub-bullet allows to include the additional parameters. So there would be no problem.

- Samsung

  - Adding “Indication can be explicit or implicit.”

    - FL: This proposal looks reasonable.

[FL Proposal 5-2-3-v4]

- For L1 measurement report configuration for gNB scheduled reporting,

  - As a starting point, at least the following RRC parameters needs to be provided to a UE, e.g.

    - Resource for channel measurement (i.e. SSB and CSI-RS if agreed) with the indication of associated cell, and the indication can be explicit or implicit.

      - a set of initial activated RSs/cells, if activation of RSs/cells are introduced

    - Type of report configuration, i.e. periodic/semi-persistent/aperiodic

    - Uplink resource configuration in order to report serving cell and candidate cell L1 measurement result to serving cell

    - Quantity, if agreed

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are further considered in RAN2

[Comments to FL Proposal 5-2-3-v4]

| Company | Comment |
| --- | --- |

[Low] UE/event triggered report for L1 measurement results

[Conclusion at RAN1#111]

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

    - At least the following aspects may be considered

      - How to define UE event and exact definition of events,

      - Report container

      - Resource allocation/assignment for UE event triggered report

      - Necessity of indication to gNB when the condition UE event is met, and how

      - Necessity to define the condition to start/stop the reporting,

      - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

      - The interaction with filtered L1 measurement results (if supported)

      - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

      - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

      - Benefit when L3 measurement is involved

[Conclusion at RAN1#112]

The following FL proposal was not treated during RAN1#112 because of the lack of time

Alt 1

- UE event triggered report for L1 measurement is supported with the following design principle:

  - Supported the following trigger events (FFS on the necessity of modification) where the threshold and offset value (if needed) is configured by RRC

    - A2 based/ Serving becomes worse than absolute threshold;

    - A3 based/Neighbor becomes amount of offset better than Pcell/PSCell;

    - A4 based/ Neighbor becomes better than absolute threshold;

    - A5 based/Pcell/PSCell becomes worse than absolute threshold1 and neighbor/Scell becomes better than another absolute threshold2;

    - Scell BFR framework

  - As for Start/Stop condition:

    - Time To Trigger (TTT) is introduced and time duration is configured by RRC, where UE event triggered report is performed when the configured event is continuously fulfilled within the configured time duration.

    - The report is performed only once after the fulfillment of the event, i.e. no stop condition is defined

  - No indication to notify the fulfillment of the event condition to gNB is introduced

  - MAC CE is used to convey the UE event triggered report

    - The scheduling of PUSCH is up to gNB

  - Contents/format defined for gNB scheduled reporting is reused as much as possible (FFS the modifications)

  - No filtering mechanism in time domain and cell level is introduced for L1 measurement results

  - No specific enhancement on report destination is necessary, i.e. UE follow the gNB indication of Tx spatial filter and pathloss reference RS using the existing mechanism

  - gNB scheduled reporting and UE event triggered reporting can be simultaneously configured

Alt 2. (if Alt 1 is not agreeable)

  - No consensus to introduce UE event triggered report for L1 measurement results in Rel-18

[Summary of contributions]

Many companies are interested in the introduction of UE/event triggered L1 measurement report, and the companies’ views are summarized as follows

Introduction of UE/event triggered L1 measurement report

  - Support (20) : Futurewei, ZTE, vivo, OPPO, Spreadtrum, Lenovo, NEC, Intel, Nokia, Sony, CATT, Fujitsu, Xiaomi, Samsung, FGI, IDC, Apple, Google, LGE, DCM

  - Not support (3) : Huawei, Ericsson, MediaTek

[FL observation]

Despite the majority support for UE/event triggered L1 measurement report is not supportive to introduce this functionality because the details of the companies’ proposals are quite different/diverse, and non-negligible discussion would be required to finalize this functionality. In this sense, FL has a strong concern to agree on the introduction of UE/event triggered report without detailed design.

Given the level of interest and the complexity of the discussion, FL would like to suggest the same proposal as in RAN1#112, which is the simplest approach and based on the majority view. If this proposal is not agreeable in this meeting. FL would like to suggest “no consensus in Rel-18” on this topic.

FL plan is to treat this topic in email only, i.e. no GTW/Teams discussion is planned.

[FL Proposal 5-2-4-v1]

Alt 1

- UE event triggered report for L1 measurement is supported with the following design principle:

  - Supported the following trigger events (FFS on the necessity of modification) where the threshold and offset value (if needed) is configured by RRC

    - A3 based/Neighbor becomes amount of offset better than Pcell/PSCell;

    - A5 based/Pcell/PSCell becomes worse than absolute threshold1 and neighbor/Scell becomes better than another absolute threshold2;

  - As for Start/Stop condition:

    - Time To Trigger (TTT) is introduced and time duration is configured by RRC, where UE event triggered report is performed when the configured event is continuously fulfilled within the configured time duration.

    - The report is performed only once after the fulfillment of the event, i.e. no stop condition is defined

  - No indication to notify the fulfillment of the event condition to gNB is introduced

  - MAC CE is used to convey the UE event triggered report

    - The scheduling of PUSCH is up to gNB

  - Contents/format defined for gNB scheduled reporting is reused as much as possible (FFS the modifications)

  - No filtering mechanism in time domain and cell level is introduced for L1 measurement results

  - No specific enhancement on report destination is necessary, i.e. UE follow the gNB indication of Tx spatial filter and pathloss reference RS using the existing mechanism

  - gNB scheduled reporting and UE event triggered reporting can be simultaneously configured

Alt 2. (if Alt 1 is not agreed in this meeting)

  - No consensus to introduce UE event triggered report for L1 measurement results in Rel-18

[Comments to FL Proposal 5-2-4-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | OK in principle for Alt1. Small comments. - Not sure to call it TTT or a new parameter. - Add FFS for ‘No filtering mechanism in time domain and cell level is introduced for L1 measurement results’. |
| Ericsson | Support Alt2 |
| Futurewei | Currently, we do not support Alt 1, because the definition of event is not clear and further evaluation results for the event are needed before making the decision. |
| QC | Fine for Alt1 |
| Nokia | Support Alt1 in general, but we have the following clarification questions: For the event definition based on A3 or A5 type events, what kind of measurements are used (L1 or L3)? For example, for A3 (Neighbour becomes amount of offset better than PCell/PSCell), will the best beam or an average quality over multiple beams be compared? |
| ZTE | Support Alt 1 |
| Lenovo | We are fine with Alt1. |
| Fujitsu | Support |
| vivo | Support Alt 1 |
| CMCC | In general we support the Alt 1. And we open for further refinement of the details. |
| LG | Support Alt 1 |
| FGI | Support Alt1 in general, however we share Nokia’s view. Which (L1 or L3) measurements are used should be further discussed. |
| Huawei, HiSilicon | Support alt 2 |
| OPPO | Support Alt1 |
| Samsung | Supportive of Alt 1, we can further discuss the details. |

[FL observation]

Companies’ views are summarized as follows:

  - Support UE/event triggered report with minimum set of functionalities (including modification/clarification proposals) (i.e. Alt.1) – (11)

    - NTT DOCOCMO, Qualcomm, Nokia, ZTE, Fujitsu, vivo, CMCC, LG, FGI, OPPO, Samsung

  - Not support (i.e. Alt.2) – (3)

    - Ericsson, Futurewei, Huawei,

Comment to modification:

- No filtering mechanism in time domain and cell level is introduced for L1 measurement results

  - DCM: this should be FFS

  - Nokia: L1 or L3 measurement for A3/A5?

- Terminology of TTT

Regarding this comment, FL is afraid that keeping FFS point will just cause the time consumption of our precious time. Regarding the terminology of TTT, FL thinks it is OK at this moment the detailed description is also provided in the FL proposal.

Again, FL believes this is purely an optimization functionality, so Rel-18 LTM can be completed without this feature. In the updated FL proposal (5-2-4-v2), FL has highlighted the potential open issues with yellow shadow, so it would be appreciated if the companies, especially opponents, could check them and make a final decision.

In any case, this topic will be handled with low priority.

[FL Proposal 5-2-4-v2]

Alt 1 (Open issues are highlighted with yellow shadow)

- UE event triggered report for L1 measurement is supported with the following design principle:

  - Supported the following trigger events (FFS on the necessity of modification) where the threshold and offset value (if needed) is configured by RRC

    - A3 based/Neighbor becomes amount of offset better than Pcell/PSCell;

    - A5 based/Pcell/PSCell becomes worse than absolute threshold1 and neighbor/Scell becomes better than another absolute threshold2;

  - As for Start/Stop condition:

    - Time To Trigger (TTT) is introduced and time duration (FFS exact values) is configured by RRC, where UE event triggered report is performed when the configured event is continuously fulfilled within the configured time duration.

    - The report is performed only once after the fulfillment of the event, i.e. no stop condition is defined

  - No indication to notify the fulfillment of the event condition to gNB is introduced

  - MAC CE is used to convey the UE event triggered report

    - The scheduling of PUSCH is up to gNB

  - Contents/format defined for gNB scheduled reporting is reused as much as possible (FFS the modifications)

  - FFS: No filtering mechanism in time domain and cell level is introduced for L1 measurement results, i.e. use L1 measurement result or L3 measurement result (i.e. time domain filtering and/or cell level filtering)

  - No specific enhancement on report destination is necessary, i.e. UE follow the gNB indication of Tx spatial filter and pathloss reference RS using the existing mechanism

  - gNB scheduled reporting and UE event triggered reporting can be simultaneously configured

Alt 2. (if Alt 1 is not agreed in this meeting)

  - No consensus to introduce UE event triggered report for L1 measurement results in Rel-18

Companies, especially opponents of Alt.1, are encouraged to provide their view.

[Comments to FL Proposal 5-2-4-v2]

| Company | Comment |
| --- | --- |
| QC | Those values can take candidate values for L3 as baseline. We are fine for no filtering FL reply: reducing the number of open issues is always helpful. It’s OK if other companies can accept it |
| ZTE | We have a minor question on “The report is performed only once after the fulfillment of the event, i.e. no stop condition is defined”. Does it means UE will continuously report measurement results or only report one time. [FL reply]: as implied by “once” in the proposal, the report is performed only one time. |
| FL | Let’s continue the discussion as not many companies have provided their views. |
| NTT DOCOMO | Generally support. We have a question for clarification. For the last FFS, if ‘L3 measurement result’ is used, what’s the difference from L3 event triggered reporting? |
| Fujitsu | Support |
| CMCC5 | Support Alt 1. The open issue we can discuss further. |
| Samsung3 | Supportive of Alt 1, we can further discuss the details. |

Beam indication

[Email approval] Beam indication mechanism based on Rel-17 unified TCI framework

[Conclusion at RAN1#110b-e]

Agreement

- RAN1 to further study if the beam indication of candidate cell(s) L1/L2 mobility should be designed for a specific TCI framework below, and their potential RAN1 spec impact.

  - Option A:  Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-17 TCI framework mechanism

  - Option B: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-15 TCI framework mechanism

  - Option C: Beam indication for Rel-18 L1/L2 mobility is designed based on both Rel-15 and Rel-17 TCI framework mechanisms

[Conclusion at RAN1#111]

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

[Conclusion at RAN1#112]

- Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

[Summary of contributions]

- UE behavior for channels/signals which does not follow Rel-17 unified TCI states

- Summary of the current UE behaviour (e.g. provided by vivo R1-2302504)

  - Note: PDCCH in the table means (in FL understanding) CORESET#0, Type 0A/1/2-PDCCH CSS sets

    - This is also pointed out by many companies.

  - Note: FL believes the signals which QCL is set by “RRC configuration” have no problem because RRC configuration can be given by RRCReconfiguration for candidate cells.

- Solution for CORESET#0, Type 0A/1/2-PDCCH CSS sets

  - Extended to follow Rel-17 unified TCI states (ZTE, Fujitsu, Samsung, Google)

    - follow the unified TCI state of the target cell until configured a new TCI state (Samsung)

    - An additional TCI state is signalled in the cell switch command (Samsung)

  - the network schedules transmission only based on the CORESET following Rel-17 unified TCI before the dedicated beam indication signaling for PDCCH configured to not follow the unified TCI and PDSCH scheduled by the PDCCH is received. (vivo)

  - the corresponding beam information would be configured by the target cell after cell switch. (CATT)

  - Follow the SSB index indicated by the cell switch command (Fujitsu, Samsung)

  - Follow the SSB index indicated by the PDCCH order for the early uplink synchronization. (Fujitsu)

(2) Details of the signaling of beam indication:

For beam indication scenario 2, the beam indication is carried by cell switch command, and hence the details of beam indication mechanism, i.e. which information is used, needs to be decided. It is noted that the same discussion applies if scenario 1 and/or TCI state activation are introduced.

- By indicating RS identifier

  - Ericsson: UE maps the RS identifier to a TCI state in the correct candidate configuration for SpCell. The UE chooses the TCI state that has the signalled SSB as QCL Type-C source.

  - Apple: At least for inter-DU case, it seems the serving DU cannot know the measurement RS configuration, e.g., SSB or CSI-RS and TCI-state configuration of candidate cell served by another DU. RS-based beam indication design seems applicable for Rel-15 TCI framework and Rel-17 unified TCI frame for TCI-state derivation and activation.

  - Apple: the SSB in a PDCCH order DCI can be used for DL TCI-state activation as well.

- By indicating Rel-17 TCI state index

  - Qualcomm: at least support TCI state configured for the new cell as the beam indication

  - Huawei: For the beam indication in the MAC CE used for LTM triggering, TCI state ID is preferred.

The difference of these two approaches is described in Ericsson’s Tdoc, and the diagram is captured below:

(3) details of TCI state indication

- Samsung: Target serving cell TCI indication should include joint TCI or both of separate DL/UL TCI states except for DC scenario.

  - i.e. only one separate TCI state via non-serving cell TCI indication is not supported

[FL observation]

For issue (1) UE behaviour for channels/signals which does not follow Rel-17 unified TCI states, there is a common understanding that CORESET#0, Type 0A/1/2-PDCCH CSS sets may have a problem on the application of Rel-17 unified TCI state. However, it was confirmed that companies view on the solution is quite diverse at this moment. Therefore, it would be good at this stage to gather the companies’ view and narrow down the solutions in this meeting.

For issue (2) Details of the signalling of beam indication, the choice of the solution highly depends on other part, i.e. introduction of TCI state. However, it would be good to gather companies’ view because the concept of beam indication is quite deferent, i.e.

- If RS identifier is used for beam indication, mapping between RS identifier and Rel-17 unified TCI state is done by a UE

- If TCI state index is used for beam indication, mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB

For issue (3), FL thinks this is obvious because the TCI state of all channels/signals needs to be updated. However, FL would suggest to focus on other important issue at this stage.

[FL Proposal 5-3-1-v1]

- For beam indication based on Rel-17 unified TCI framework applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets, the following alternatives are further studied and one alternative will be down-selected at RAN1#113. (FL note: partial down-selection in RAN1#112bis-e is appreciated)

  - Alt.1: Follow the indicated TCI state until configured a new TCI state

  - Alt.2: An additional TCI state is signalled in the cell switch command

  - Alt.3: Follow the SSB index indicated by the cell switch command or PDCCH order for candidate cells

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. the network schedules transmission only based on the CORESET following Rel-17 unified TCI and the corresponding beam information would be configured by the target cell after cell switch

- One alternative will be down-selected for beam indication and TCI state activation (if supported):

  - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

  - Alt. 2: By indicating Rel-17 TCI state index, i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB

Companies are invited to provide the views to narrow-down the options.

[Comments to FL Proposal 5-3-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support in principle. For the options, we support Alt.1 for the first bullet, and Alt.2 for the second bullet. |
| Ericsson | Propose to split in two proposals: the two bullets are independent First bullet: after the cell-switch, the UE has a new RRC configuration, and the UE should follow that. That RRC configuration tells the UE how to operate with the indicated TCI state Second bullet: prefer Alt1 – it is simpler for the UE and the NW. Keep in mind that if the UE gets the TCI state, the UE would need to look into the RRC configuration of the candidate to find the SSB, which is the QCL source of the TCI state. |
| Google | Support FL proposal in principle. On the second bullet, we support Alt2. |
| Futurewei | We have the same view as NTT DOCOMO. We are ok with Alt 1 for the first bullet and Alt.2 for the second bullet because TCI state has much more useful information than RS only, not just beam information. |
| QC | For 1st bullet, support Alt4, i.e. which SSB to monitor the CSS is selected by UE as in initial access till legacy TCI is indicated. For 2nd bullet, support Alt2 |
| Apple | On the 1st bullet: - Alt.4. - At least one TCI-state is activated before cell switch command, and one is indicated by Cell switch command. In addition, which CORESTs that follow the indicated TCI-state have been pre-configured for target cell and known by the UE. NW can use these CORESET for scheduling. - For CORESET configured to ‘not follow’ unified TCI-state, NW can indicate TCI-state after UE switches to target cell as in Rel-17. On the 2nd bullet: - Prefer Alt.1 as it is a unified solution for both Rel-15 and Rel-17 TCI framework. - Also fine with Alt.2 if majority companies prefer it to make progress. |
| Nokia | Fort the first bullet, we slightly prefer Alt1, but Alt 4 is also fine if majority prefers that. We do not support Alt 2 and Alt 3. We are wondering if Alt 3 is supported then why wouldn’t we use the same indication for all the channels/signals. It would rather be a simpler mechanism, and this is similar to what we have been proposing from last two meetings. There is no benefit of supporting a separate (SSB based indication) only for a sub-set of channels/signals. For the second bullet, we support Alt 2. We don’t think that providing TCI state configuration to the source DU/cell would be an issue as there is anyways other information like L1 measurements configuration, RACH configuration (for PDCCH order) is going to be provided to the source DU, and we haven’t seen any objection from RAN2/3 on this. This can be further clarified with them. |
| MediaTek | We also prefer to split into two proposals. In our view, we should first decide what is the beam indication in the cell switch command, i.e, we should first discuss the following two alternatives. - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index, i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB If Alt2 is adopted, then we don’t think Alt1 and Alt2 under the first bullet is valid. Similar situation for Alt1 is adopted. On the other hand, Alt4 under the first bullet is acceptable to us without discussing what is the beam indication in the cell switch command. |
| ZTE | For 1st bullet, we would like to confirm what “For beam indication based on Rel-17 unified TCI framework applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets” means. Form our point of view, beam indication in Rel-18 LTM can be directly applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets without being controlled by RRC parameters “followUnifiedTCIstate”, after all the application scenarios of Rel-17 ICBM and Rel-18 LTM are different. With such understanding, the indicated TCI state in cell switch command (if only scenario 2 is supported in Rel-18 LTM) can be applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets. For 2nd bullet, we prefer Alt.2 but whether to need introducing mapping between RS and TCI state should be left for implementation. |
| Lenovo | For the first bullet, we support Alt.1 For the second bullet, we support Alt.2. |
| Fujitsu | On the first bullet, we prefer Alt.1 or Alt.3. On the second bullet, we prefer Alt.2. In our view, a TCI state also includes useful information other than a RS ID. |
| Vivo | For the first bullet, support Alt.4. For the second bullet, support Alt.2. |
| Xiaomi | This proposal is talking about the beam indication of target cell, right? We notice that there is wording “cell switch command” in some alternatives. Then it is better to make it more it clear in the main bullet: - For beam indication of target cell based on Rel-17 unified TCI framework … |
| CMCC | Could the proponent further clarify the difference between Alt 1 and Alt 4? does Alt 1 mean that the CORESET#0 and Type 0A/1/2-PDCCH CSS sets which are configured not to follow the unified TCI will follow the Rel-17 beam indication in the cell switch command ? For the 2nd bullet, based on the reformulation from FL, the 2nd bullet is preferred. The mapping of Rel-17 unified TCI and the reference RS should be determined by gNB not UE. Our preference is that the LTM based on the Rel-15 TCI framework should be also supported, including the part of the Rel-17 unified TCI that the CORESET#0 and Type 0A/1/2-PDCCH CSS which do not follow the unified TCI states. |
| IDCC | Support |
| LG | Support Alt4 for the first bullet. It is at least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2). The applicability of indicated TCI state to PDCCH reception and the respective PDSCH is per-CORESET determination and NW can indicate TCI-state after cell switch. |
| FGI | For the first bullet, we support Alt.1 and Alt.2. For the second bullet, we support Alt.2. |
| Huawei, HiSilicon | Share similar view as Ericsson that the two main bullets should be separated into two proposals. For the first main bullet, we prefer Alt 1 due to its simplicity. For the 2nd main bullet, we prefer Alt 2. Current TCI state can carry two QCL source RS and corresponding QCL types, power control parameter and pathloss RS, which cannot be reflected only by RS index. |
| OPPO | For 1st bullet, support Alt4 For 2nd bullet: support Alt2 |
| Samsung | For first bullet, we are fine with Alt1 or Alt2. For second bullet, we prefer Alt2. |
| CATT | For the first bullet, we prefer Alt.4. For the second bullet, we prefer Alt.2. We are also fine to down select in the next meeting. |

[FL Proposal 5-3-1a-v2]

- For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and Type 0A/1/2-PDCCH CSS sets where no RRC configuration for the TCI state is provided or followUnifiedTCI-state is not enabled or not provided, the following alternatives are further studied, and one alternative will be down-selected at RAN1#113. (FL note: partial down-selection in RAN1#112bis-e is appreciated)

  - Alt.1: Follow the indicated TCI state until configured a new TCI state is configured

    - Supported by DCM, Futurewei, Nokia, ZTE, Lenovo, Fujitsu, FGI, Huawei, Samsung

  - [Alt.2: An additional TCI state is signalled in the cell switch command

    - Supported by Samsung]

    - FL note – can we remove alt 2 due to minority support?

  - [Alt.3: Follow the SSB index indicated by the cell switch command or PDCCH order for candidate cells

    - Supported by Fujitsu]

    - FL note – can we remove alt 3 due to minority support?

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. RRC configuration for TCI state is always available under candidate cell configurations, the network schedules transmission only based on the CORESET following Rel-17 unified TCI, and/or the corresponding beam information would be configured by the target cell after cell switch

    - Supported by Ericsson, Apple, Nokia, vivo, LG, OPPO, QC

FL note: The the second bullet in FL proposal 5-3-1-v1 is now split. Note that the split part (second bullet, to be 5-3-1b-v2) is aimed at agreement in RAN1#112bis-e and discussed later in this meeting, while this proposal (5-3-1a-v2) is for the next meeting. In this sense, FL believes concern from MediaTek can be addressed.

FL note: Regarding the question from CMCC (Could the proponent further clarify the difference between Alt 1 and Alt 4? does Alt 1 mean that the CORESET#0 and Type 0A/1/2-PDCCH CSS sets which are configured not to follow the unified TCI will follow the Rel-17 beam indication in the cell switch command ?), FL’s understanding is that the companies view are not so clear at this moment: In this sense, CMCC’s question is valid and should be clarified in the proposal.

- There are companies who want to define a behaviour when “not to follow unified TCI” is configured. However, FL guesses some companies think that such configuration is not a valid case.

- There are companies who think there is no configuration or no SSB to follow e.g. for “follow unified TCI”, or “SSB identified during a most recent random access procedure”. However, some companies believe such a configuration is available in the candidate cell configurations.

[FL Proposal 5-3-1b-v2]

- One alternative will be down-selected for beam indication and TCI state activation (if supported):

  - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

    - Supported by Ericsson, Apple

  - Alt. 2: By indicating Rel-17 TCI state index, i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB

    - Supported by DCM, Google, Futurewei, Qualcomm, Nokia, ZTE, Fujitsu, CMCC, FGI, Huawei, OPPO, Samsung, CATT

FL note: Clear majority supports Alt.2. Can we go with majority view. Please provide your view if everyone is OK with this direction

[Comments to FL Proposal 5-3-1a-v2 and 5-3-1b-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support the two proposals. |
| Fujitsu | Proposal 5-3-1a-v2: We are OK to down select from the alternatives supported by majority. “followUnifiedTCI-state is not enabled or not provided” should always be a condition. So, it is suggested to delete “or” before “followUnifiedTCI-state”. The sentence “no RRC configuration for the TCI state is provided” may not be accurate. Form our understanding, CORESET 0 shares a TCI state pool with PDSCH. Since the TCI state pool for PDSCH will be configured in the candidate cell configuration, RRC configuration for the TCI state for CORESET 0 will be always available. Therefore, it is suggested to delete “no RRC configuration for the TCI state is provided” or replace it with “no TCI state activation is provided”. Similarly, as explained before, “RRC configuration for TCI state is always available under candidate cell configurations” is true for both Alt.1 and Alt.4. It may not be clear why this part is only highlighted for Alt.4. As for Alt.1, our interpretation is that “not to follow unified TCI state” is configured for CORESET 0. After cell switch and before receiving TCI state activation from the target cell, a UE will apply the indicated TCI state in cell switch command. After receiving TCI state activation, the UE will apply the activated TCI state. |
| CMCC | For Proposal 5-3-1b-v2, we support the Alt2 and fine to do the down selection. For the Proposal 5-3-1a-v2, thanks for FL’s further clarification. It seems that the difference between Alt 1 and Alt 4 is exactly that whether the channels/signals which are not intended to follow the unified TCI would follow the beam indication in the CSC. Alt 1 break the legacy rule of unified TCI. But Alt 4 cannot support the transmission of those signal/channels especially CORESETs who do not follow the unified TCI. In Alt 4, the UE has to wait until new configuration of the target cell to be activated. If the delay due to the activation of the target cell is acceptable, Alt 4 with only UE specific transmission is also acceptable. For the Alt 1, our thinking is that the new configured TCI is configured or activated by the target cell. Then the Alt 1 is updated as below, - Alt.1: Follow the indicated TCI state until configured a new TCI state is configured by the target cell |
| Samsung | For Proposal 5-3-1a-v2, prefer Alt1. Just a small wording change: - Alt.1: Follow the indicated TCI state until configured a new TCI state is configured or activated For Proposal 5-3-1b-v2, the motivation for Alt1 is not clear, i.e. “mapping between RS identifier and Rel-17 unified TCI state is done by a UE”. This seems to depart from earlier NR releases where the mapping is done by the gNB. Is there a benefit in doing the mapping by the UE? |
| Nokia | For 5-3-1b-v2, since we have agreed to use the Rel-17 uTCI framework, it would make sense to use the existing mechanism of TCI state configuration, i.e., Alt.2. If we choose Alt.1 where the mapping is done by the UE, the same mapping may need to be done by the target cell to derive the TCI state for the UE (target cell needs to be aware of the TCI state to configure a proper beam for the UE); therefore, inter-DU/cell coordination on the beam indication (and maybe for activation, if agreed) will be needed, especially when there is no RACH after the cell switch. If we agree with that then why not follow the existing TCI state configuration mechanism where the UE is directly provided with a TCI state. |
| Ericsson | Support both FL proposals. For 5-3-1b-v2: The reason why we support Alt2 is that the candidate and the UE need to know the mapping, but the serving cell does not need to know the mapping. We also note that if the beam indication is in the form of an SSB, the UE can use it directly without unpacking the RRC configuration of the candidate. This becomes even more of a burden for activation. We could potentially select between Alt1 and 2 during this meeting. |
| vivo | Proposal 5-3-1a-v2: First of all, we share a similar view with Fujitsu that “RRC configuration for TCI state is always available under candidate cell configurations” is true for both Alt.1 and Alt.4. The related contents should be modified. Secondly, in our view, the difference between Alt.1 and Alt.4 is listed as follows: - Alt.1: CORESET#0 and CORESETs (other than CORESET#0)associated with Type 0A/1/2-PDCCH CSS sets follow the TCI state indicated in the cell switch command and common messages can be scheduled and transmitted based on the indicated TCI state before the corresponding TCI state activation/indication is received by UE. - Alt.4: if followUnifiedTCI-state is not enabled or not provided for CORESET#0 and CORESETs (other than CORESET#0) associated with Type 0A/1/2-PDCCH CSS sets, any data transmission is not expected to be scheduled by the CORESETs until the corresponding TCI state activation/indication is received by UE. Compared with Alt.1, which changes the beam indication mechanism of the Rel-17 unified TCI framework, we prefer Alt.4. Due to the system information can be configured in candidate cell configuration, there is no strong demand for data transmission on non-UE-dedicated channels. When the system information needs to be updated, it can be achieved after the corresponding TCI state activation/indication is received by UE. Proposal 5-3-1b-v2: Support to down-select with the majority views. |
| QC | For 1a, we prefer Alt4 For 1b, support Alt2 |
| ZTE | We are fine with 1a. - For 1b, we think that “mapping between RS (SSB) and Rel-17 unified TCI state” in Alt 2 can be done by gNB implementation. And it would be better to remove “i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB” to avoid confusion. |
| LG | Fine with two updated proposals. |
| Huawei, HiSilicon | For 5-3-1a-v2, support the proposal. we prefer Alt 1. For 5-3-1b-v2, we hope to down select in this meeting. Our preference is Alt 2 |
| Xiaomi | Proposal 5-3-1a-v2: Fine with this proposal. And both alt.1 and alt.4 are OK to us. |
| IDCC | 1a: Support alternative 4. 1b: Support alternative 2. |
| CATT | For 1a, support Alt4 For 1b, support Alt2 |
| Lenovo | For 5-3-1a-v2, we support Alt.1. For 5-3-1b-v2, we support Alt.2 |
| Futurewei | We are fine with FL proposal 5-3-1a-v2, and we prefer Alt 1. We are fine with FL proposal 5-3-1b-v2, and we prefer Alt 2, because TCI state has much more useful information than RS only, not just beam information. |

[FL Proposal 5-3-1a-v3]

- For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and CORESETs (other than CORESET#0) associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided, followUnifiedTCI-state is not enabled or not provided, the following alternatives are further studied, and one alternative will be down-selected at RAN1#113.

  - Alt.1: Follow the indicated TCI state until a new TCI state is configured or activated by the target cell

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. the network schedules transmission only based on the CORESET following Rel-17 unified TCI, and/or the corresponding beam information would be configured by the target cell after cell switch

[FL Proposal 5-3-1b-v3]

- Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) ,

  - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

    - Supported by Ericsson, Apple

  - Alt. 2: By indicating Rel-17 TCI state index, i.e. mapping between RS (SSB) and Rel-17 unified TCI state is done by a gNB

    - Supported by DCM, Google, Futurewei, Qualcomm, Nokia, ZTE, Fujitsu, CMCC, FGI, Huawei, OPPO, Samsung, CATT, vivo, IDCC, CATT, Lenovo

[FL observation]

FL Proposal 5-3-1b-v3 was concluded in GTW on Wed thanks to the constructive discussion. (Alt. 2 was approved). Then, let’s focus on 5-3-1a for now.

[FL Proposal 5-3-1a-v4]

- For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and CORESETs (other than CORESET#0) associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided, followUnifiedTCI-state is not enabled or not provided, the following alternatives are further studied, and one alternative will be down-selected at RAN1#113.

  - Alt.1: Follow the indicated TCI state until a new TCI state is configured or activated by the target cell

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. the network schedules transmission only based on the CORESET following Rel-17 unified TCI, and/or the corresponding beam information would be configured by the target cell after cell switch

FL note: It is not clear from the discussion so far why Alt.1 supporters think Alt.4 does not work. In this round, it would be apricated if Alt.1 supporters could provide the reason why Alt.4 cannot be accepted.

[Comments to FL Proposal 5-3-1a-v4]

| Company | Comment |
| --- | --- |
| Nokia | For the down selection, we slightly prefer Alt1 than Alt4. |
| NTT DOCOMO | Support. |
| Fujitsu | Support |
| Spreadtrum | Support proposal, and prefer Alt 4 to avoid specification change. |
| Huawei, HiSilicon | Support the proposal and prefer Alt 1 for down selection. |
| vivo | Support and prefer Alt.4. |
| CMCC5 | Support the proposal. Alt 4 is slightly preferred which does not break the specification of unified TCI. The CORESETs/channels/signals that not follow the unified TCI can work after the RRC configuration of the candidate cell are activated. That is our understanding. |
| ZTE | Support the proposal but prefer Alt 1. |
| Samsung3 | OK for progress. Prefer Alt1. |
| QC | Support Alt4. To our understanding, CORESET not following unified TCI and without activated TCI will follow legacy rule, e.g. UE will receive CORESET 0 for those CSSs in the same way as in initial access, and will not receive other CORESETs. We don’t see any issue for that |

[FL proposal]

Even though companies have their preference, it seems that the proposal is now stable if we don’t perform down selection in this meeting. Thus FL would like to propose to go to email approval process

[FL Proposal 5-3-1a-v5 for email approval]

- For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and CORESETs (other than CORESET#0) associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided, followUnifiedTCI-state is not enabled or not provided, the following alternatives are further studied, and one alternative will be down-selected at RAN1#113.

  - Alt.1: Follow the indicated TCI state until a new TCI state is configured or activated by the target cell

  - Alt.4: No new behaviour is introduced on top of Rel-17 unified TCI

    - i.e. the network schedules transmission only based on the CORESET following Rel-17 unified TCI, and/or the corresponding beam information would be configured by the target cell after cell switch

[Mid] Configuration for TCI states based on Rel-17 unified TCI framework

[Conclusion at RAN1#110b-e]

Agreement

- RAN1 to further study if the beam indication of candidate cell(s) L1/L2 mobility should be designed for a specific TCI framework below, and their potential RAN1 spec impact.

  - Option A:  Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-17 TCI framework mechanism

  - Option B: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-15 TCI framework mechanism

  - Option C: Beam indication for Rel-18 L1/L2 mobility is designed based on both Rel-15 and Rel-17 TCI framework mechanisms

[Conclusion at RAN1#111]

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

[Conclusion at RAN1#112]

- Agreement related to an LS to RAN2

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

[Summary of contributions]

Structure of configuration

  - Independent TCI state pool (for candidate cell and serving cell) and mixed TCI state pool (vivo)

  - Similar approach as L1 measurement RS configuration for TCI state pool (NEC, MediaTek)

    - FL understanding is that the RRC structure discussion is up to RAN2 (same approach as L1 measurement RS configuration)

      - Option 1) Configurations for TCI state pool is provided under servingCellConfig for the BWP of serving cells

      - Option 2) Configurations for TCI state pool is provided separately from ServingCellConfig for the serving cells and RRCReconfiguraiton for the candidate cells

      - Option 3) Configurations for TCI state pool is provided under RRCReconfiguraiton for the BWP of candidate cells

  - With a common configuration of TCI states associated with multiple candidate cells, a MAC-CE is used to select sub-set of TCI states from the common configuration (Nokia)

    - FL note: FL’s understanding is that this activation procedure can be achieved by TCI state activation, if supported, or need to be a separate procedure

  - Support TCI state configuration per CSI-RS resource at least for L1 inter-frequency measurement – Detailed configuration can be up to RAN2 (Qualcomm)

TCI states mode:

- Huawei: Per candidate cell TCI state mode should be provided to UE to determine the type of TCI state indication in the cell switch command.

[FL observation]

Similar to the measurement report configuration, the same approach as L1 measurement RS is necessary for this topic: i.e. RAN1 needs to list all the necessary parameters while the detailed RRC structure is up to RAN2.

For the configuration of TCI states mode, FL thinks this is a necessary clarification if the same mechanism as Rel-17 is used.

[FL Proposal 5-3-1-v1]

- For the configuration of TCI states for candidate cells,

  - Per candidate cell TCI state mode, i.e. unifiedTCI-StateType, is be provided to UE to determine the type of TCI state indication in the cell switch command

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are applicable

    - Option 1) Configurations for TCI state pool is provided under servingCellConfig for the BWP of serving cells

    - Option 2) Configurations for TCI state pool is provided separately from ServingCellConfig for the serving cells and RRCReconfiguraiton for the candidate cells

    - Option 3) Configurations for TCI state pool is provided under RRCReconfiguraiton for the BWP of candidate cells

  - FFS to introduce a MAC CE to select sub-set of TCI states from TCI state pool for beam indication or TCI state activation (if supported)

[Comments to FL Proposal 5-3-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support in principle. We think the MAC CE for TCI state activation is needed. |
| Ericsson | The need for this is not clear to us. Clearly, there will be TCI states in each the candidate cell. What else is needed? |
| Google | Support in principle. However, we prefer not to list those options under the second bullet. Anyway, it is up to RAN2. For the FFS, isn’t it just TCI state activation? What else do we need to study? Last, the index of this FL proposal seems to be typo? |
| Futurewei | Please let RAN2 make the decision for different options of RRC structure. |
| QC | Prefer no signaling of unifiedTCI-StateType in CSC. RRC pre-configured is sufficient as in R17 |
| Apple | Support. On MAC-CE to activate TCI-state before cell-switch command, we agree the function is needed i.e., activating TCI-state before cell-switch command such that DL sync latency can be reducing as the indicated TCI-state by Cell-switch command can be immediately used without need of one SSB reception. However, the detailed signal for activation maybe FFS e.g., whether reuses the Rel-17 TCI-state activation MAC-CE or use the PDCCH-order DCI that triggers the CFRA operation. |
| Nokia | Support, but we should clarify the FFS. We think this FFS is not on the TCI activation, instead this is an intermediate step to select the limited number of TCI states initially configured for LTM. For example, if there is a large set of TCI states associated with 7 candidate cells is configured to the UE (e.g., in the HO preparation phase), this set may contain more than, e.g., 128 TCI states, then we would need a long bit codepoint in MAC CE activation command to indicate a TCI state; therefore we may need an intermediate step to dynamically select a sub-set of TCI states (e.g., up to 64) from the initial configured large set for LTM. However, the need of such step depends on how the configuration of TCI states is finalized, so we can wait for that first. |
| ZTE | We tend to let RAN2 to determine TCI state configuration. But for activation of TCI state, we support that some TCI states are activated from configured TCI state pool before cell switch command. |
| Lenovo | We also think the detail configuration should be discussed in RAN2. For the first bullet, we understand it’s better to provide TCI state mode along with the TCI state pool configuration for each candidate cell. |
| Fujitsu | Support in principle. As for unifiedTCI-StateType, it may have already been included in the candidate cell configuration. As for the FFS, our understanding is that it is not the TCI state activation before cell switch command. |
| vivo | Fine with the FL proposal. |
| CMCC | If the proposal is to define a unified TCI state pool contains all the TCI states for UE to communicate to all the candidate cells, we are fine with it. For the 3rd bullet, we kind of have a similar understanding as Apple. An MAC CE to activate some TCI states before cell switch command would be helpful for the DL synchronization and early acquisition of TA. But for the beam indication, the legacy behaviour is to indicate a beam in DCI from the activated beams through MAC CE. Currently, the cell switch command is carried in a MAC CE as agreed in the last meeting. Then it would be that a MAC CE indicates a beam from the beam set activated by another MAC CE. Our assumption is that such a MAC CE activation can happen before the early DL synchronization and TA acquisitions to reduce the workload of UEs. |
| LG | We have a similar view with Google that It is not necessary to list options and needs to leave up to RAN2. |
| Huawei, HiSilicon | Support the proposal in general. Response to QC: we are not proposing to put unifiedTCI-StateType in CSC. The text of “in the cell switch command” is just for the TCI state indication. May following update can be used - Per candidate cell TCI state mode, i.e. unifiedTCI-StateType, is be provided before cell switch command to UE to determine the type of TCI state indication in the cell switch command For the last FFS, not sure why it is necessary to have additional MAC CE to select subset of TCI states for further activation. To our understanding, MAC CE for activation itself implies select of subset to be used. |
| OPPO | Ok with the proposal in principle. |
| Samsung | Intention of this proposal is not clear. The TCI state configuration for candidate cells are needed. The TCI state source RS can indicate the candidate cell that the TCI state belongs to. |

[FL observation]

The comments from companies are summarized below:

- Necessity sub-set selection of TCI states

  - DCM, Apple, ZTE: sees the necessity of TCI state activation

  - Google, Huawei: necessity of sub-set selection of TCI state is not clear

  - Nokia: the intention is intermediate step (not TCI state activation), and this might be necessary if large number of TCI states can be activated, i.e more than 128  this is also FL’s intention

- Do not see the strong necessity of this proposal itself

  - Ericsson, Samsung

- No strong necessity to list RRC structure options

  - Google, Futurewei, ZTE, Lenovo, LG

- Necessity of unifiedTCI-StateType

  - QC (RRC pre-configuration is enough)

  - Fujitsu (Candidate cell configuration is available)  FL’s understanding is UE need to decode candidate cell configuration before processing cell switch command

  - Huawei:

    - Per candidate cell TCI state mode, i.e. unifiedTCI-StateType, is be provided before cell switch command to UE to determine the type of TCI state indication in the cell switch command

- Interaction with TCI state activation, UL sync and DL sync

  - CMCC  this topic is covered by other section, and FL intention is not TCI state activation (see the comment from Nokia)

[FL Proposal 5-3-1-v2]

- For the configuration of TCI states for candidate cells,

  - Per candidate cell TCI state mode, i.e. unifiedTCI-StateType, is be provided before cell switch command to UE to determine the type of TCI state indication in the cell switch command

    - [FL note: FL’s understanding is the intention is to provide unifiedTCI-StateType for candidate cell separately from the RRCReconfiguration for the candidate cell]

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are further considered in RAN2 [FL note: the same wording in 5.3.2 applied]

  - The detailed design of RRC structure is up to RAN2, and the similar options for L1 measurement RS are applicable

    - Option 1) Configurations for TCI state pool is provided under servingCellConfig for the BWP of serving cells

    - Option 2) Configurations for TCI state pool is provided separately from ServingCellConfig for the serving cells and RRCReconfiguraiton for the candidate cells

    - Option 3) Configurations for TCI state pool is provided under RRCReconfiguraiton for the BWP of candidate cells

  - FFS to introduce a MAC CE to select sub-set of TCI states from TCI state pool for beam indication or TCI state activation (if supported)

    - [FL note: seeing from companies view and the proponent’s comment, FL feels that we can delete this at this moment, and come back at the next meeting based on the contributions]

[Comments to FL Proposal 5-3-1-v2]

| Company | Comment |
| --- | --- |
| QC | Fine with FL’s proposal |
| Nokia | We don’t think so we need to discuss this proposal at this moment, also if we support Rel-17 uTCI based beam indication, unifiedTCI-StateType should be part of the TCI state configuration (RRC). In terms of the configuration of TCI states, from RAN1 perspective, we need to make sure, that Rel-17 unified TCI state-based configurations of LTM candidate cells are provided. The details can be left to RAN2. Also, such configuration is also needed at the serving cell or not will depend on the outcome of proposal 5-3-1b-v2 (if beam indication is a TCI state) and proposal 5-5-1 (if TCI activation is supported). |
| ZTE | Fine with FL’s proposal |
| MediaTek | We originally thought the bullet - Per candidate cell TCI state mode, i.e. unifiedTCI-StateType, is be provided before cell switch command to UE to determine the type of TCI state indication in the cell switch command means unifiedTCI-StateType will be pre-configured to UE in the same way as other candidate cell configurations in RRCReconfiguration for the candidate cell. However, the note from FL - [FL note: FL’s understanding is the intention is to provide unifiedTCI-StateType for candidate cell separately from the RRCReconfiguration for the candidate cell] means there is a separated signaling for such configuration? Can FL clarify? FL’s answer is yes. Otherwise, this proposal is meaningless as the configuration in RRCReconfiguration for candidate cells is quite normal thing. |
| LG | Fine with the proposal. |
| Huawei, HiSilicon | Fine with FL’s proposal |
| Samsung2 | The intention of this proposal is not clear as we have already agreed to support unified TCI state for all candidate cells. So the meaning or reason for providing unifiedTCI-StateType is not clear. FL: please check the contribution from Huawei, which is quoted below. - In Rel-17 unified TCI state framework, there two TCI state modes called joint and separate. A UE can be provided with TCI state lists for both modes at the same time, i.e. dl-OrJoint-TCIStateList and TCI-UL-State. And gNB use a flag called unifiedTCI-StateType-r17 configured per cell to determine UE’s TCI mode. For the beam indication in LTM triggered MAC CE, it is necessary for UE to be aware of the TCI state mode of corresponding beam indication. For example, the unifiedTCI-StateType-r17 can be configured associated to candidate cells before cell switch. Detailed signalling design is up to RAN2. |
| Futurewei | Fine with FL’s proposal |
| FL | It seems that the intention of the proposal in 1st bullet may not be correctly understood by companies. Let’s continue the discussion |
| Nokia | Agree with Samsung, it should be a part of the Rel-17 TCI state configuration. We don’t think so it needs to specified again for LTM. We think the more important issue for this section (Configuration of TCI states….) is to make aware RAN2/3 that serving cell needs to be aware of TCI state configurations of candidate cells. Especially in light of our agreement that a TCI state is used as the beam indication. |
| NTT DOCOMO | Agree with SS/Nokia. |
| Fujitsu | In our view, 1st bullet is needed if TCI state activation before cell switch command is agreed. Otherwise, UE may just wait until receiving cell switch command and then acquire unifiedTCI-StateType from decoding candidate cell configuration. |
| Huawei, HiSilicon2 | We think it should specify for LTM because it is for candidate cell, although similar mechanism as r17 may be used for serving cell. If all companies share the similar view that indication of unifiedTCI-StateType-r17 will be inherited directly to candidate cell, at least we need a conclusion to mark it down in order to avoid confusion in the future. |
| FGI | We are ok with FL’s proposal. |
| vivo | Fine with FL’s proposal. |
| Nokia | Fine with Huawei’s proposal to mark it down as a conclusion. |

[FL observation]

Considering the comment from companies, it would be best way for now to go with the Huawei’s updated proposal.

[FL Proposal 5-3-1-v3]

Conclusion

- Indication of unifiedTCI-StateType-r17 in the serving cell will be inherited directly to candidate cells

[Comments to FL Proposal 5-3-1-v3]

| Company | Comment |
| --- | --- |

[Closed] Beam indication mechanism applicable to gNBs not supporting Rel-17 TCI framework

[Conclusion at RAN1#110b-e]

Agreement

- RAN1 to further study if the beam indication of candidate cell(s) L1/L2 mobility should be designed for a specific TCI framework below, and their potential RAN1 spec impact.

  - Option A:  Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-17 TCI framework mechanism

  - Option B: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-15 TCI framework mechanism

  - Option C: Beam indication for Rel-18 L1/L2 mobility is designed based on both Rel-15 and Rel-17 TCI framework mechanisms

[Conclusion at RAN1#111]

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

[Conclusion at RAN1#112]

The following FL proposal was made, and no offline/online discussion was held at RAN1#112

- Interested companies are encouraged to check the proposal on by proponent companies, and to input their contribution to the future meeting as necessity.

[Summary of contributions]

Support mechanism which is applicable to Rel-15 TCI framework

- Yes (by indicating SSB index): Nokia

  - a TCI state or a QCL source RS associated with a target cell can be provided before cell switch

- Yes (by indicating TCI states for each channel/signal): NEC

- Yes (designed based on Rel-15 TCI framework): CATT

- Yes (No concrete proposal)

- Low priority: vivo, DCM

- No in Rel-18: Futurewei, Spreadtrum, Intel, Samasung, Google, Qualcomm

[FL observation]

Even though three companies showed their interest in the necessity to support the scenario where one from serving cell and candidate cell supports only Rel-15 TCI framework, a greater number of companies (8) thinks the discussion is unnecessary or low priority.

Given this situation, FL sees no necessity to make a FL proposal in this meeting, and come back to this topic after the progress of other important discussions.

[Comments if any]

| Company | Comment |
| --- | --- |
| Google | Agree with FL’s assessment. |
| CMCC | Please add CMCC as one of the supporting company for the LTM design based on Rel-15 TCI. Both indicating SSB index and designed based on Rel-15 (the 3rd bullet) are acceptable to us. |

[Closed] Timing of beam indication – scenario 2 except TCI state activation

[Conclusion at RAN1#110b-e]

Agreement

- From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart

  - Scenario 1: Beam indication before cell switch command

  - Scenario 2: Beam indication together with cell switch command

  - Scenario 3: Beam indication after cell switch command

- Interested companies are encouraged to further study the validity of the scenarios and the potential spec impact.

[Conclusion at RAN1#111]

Agreement

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

[Conclusion at RAN1#112]

Agreement

- RAN1 shares the same understanding as RAN2 on agreement:

  - The LTM mobility trigger information is conveyed in a MAC CE

- The same MAC CE is used for the LTM triggering.

Agreement

- The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

  - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

[Summary of contributions]

MediaTek: TCI state indicated in the cell switch command is in the activated TCI state lists before cell switch and in the activated TCI state lists after cell switch.

[FL observation]

Even though the relationship between TCI state activation and beam indication in cell switch command needs clarification, the decision on TCI state activation should be made first. Therefore, FL sees no urgent issues under this section in this meeting.

With this understanding, no FL proposal in this section is made, and this section is closed.

Comments if any]

| Company | Comment |
| --- | --- |
| Nokia | FL mentions that “the decision on TCI state activation should be made first”, but we don’t see any dedicated discussion on that, and we think this is one of the key issues, at least for the agreed scenario 2. I.e., - Do we support TCI activation before the beam indication in the cell switch command or not? Is yes, details of mechanism (existing MAC-CE or new MAC-CE), what is the UE assumption on the activated TCI states after the cell switch? |
| CMCC | As we commented in section 5.3.2, we are supportive of the MAC CE activation of a subset of the TCI states configured for the candidate cells. But it is not only for the down scope the indicated TCI states but also facilitate other behaviours. An MAC CE to activate some TCI states before cell switch command would be helpful for the DL synchronization and early acquisition of TA. But for the beam indication, the legacy behaviour is to indicate a beam in DCI from the activated beams through MAC CE. Currently, the cell switch command is carried in a MAC CE as agreed in the last meeting. Then it would be that a MAC CE indicates a beam from the beam set activated by another MAC CE. Our assumption is that such a MAC CE activation can happen before the early DL synchronization and TA acquisitions to reduce the workload of UEs. |

[Closed] Timing of beam indication – scenario 1 and/or 3

[Conclusion at RAN1#110b-e]

Agreement

- From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart

  - Scenario 1: Beam indication before cell switch command

  - Scenario 2: Beam indication together with cell switch command

  - Scenario 3: Beam indication after cell switch command

- Interested companies are encouraged to further study the validity of the scenarios and the potential spec impact.

[Conclusion at RAN1#111]

Agreement

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

[Conclusion at RAN1#112]

The following FL proposal was not discussed at RAN1#112 due to the lack of time.

- On scenario 1 for the timing of cell switch command, companies are encouraged to study further the following aspects:

  - which kind of enhancement is needed for scenario on top of the simultaneous operation of Rel-17 ICBM and Rel-18 LTM, and

  - the necessity of enhancements for scenario 1 when an activation procedure before cell switch command reception is introduced for scenario 2.

[Summary of contributions]

Regarding the support of scenario 1:

- scenario 1 should be supported

  - Support simultaneous operation with Rel-17 ICBM and Rel-18 LTM (Futurewei, Huawei, vivo, Nokia, CATT, Samsung, CMCC, FGI, IDC, Apple, DCM)

    - A combined TCI state pool is provided for Rel-17 ICBM and Rel-18 LTM (Nokia)

    - TCI state activation differentiate Rel-17 ICBM or Rel-18 LTM (Nokia)

    - No specific enhancement is needed. For a candidate cell with the same PCI configured in both Rel-17 ICBM configuration signaling and Rel-18 LTM configuration signaling, both Rel-17 ICBM and Rel-18 LTM can be performed for this cell.(DCM)

  - If TCI state activation is supported and only one TCI state can be activated, then it is scenario 1(Apple)

  - Applicable only when target cell is the current Scell: (Huawei)

  - Should be supported For network flexibility (Google)

  - Should be independent from Rel-17 ICBM:

    - The network could delay the cell-switch command when measurement reports validate that the UE is under the coverage of the target cell. Cell switch command cannot update the indicated TCI (Ericsson)

    - The beam is indicated first to target cell; this is then followed by the cell switch command, for e.g. inter-frequency scenario (Samsung)

- Low priority

  - ZTE

- No support for scenario 1:

  - OPPO, Xiaomi, MediaTek(FFS)

Regarding the support of scenario 3:

- Support of scenario 3: Nokia (as a fallback operation), Google

[FL observation]

Many companies view that scenario 1 is supported in terms of the simultaneous operation of Rel-17 ICBM and Rel-18 LTM, which doesn’t require any spec change except potential UE capability. This is useful to achieve a smooth transition from serving cell to target cell. In addition, some companies see the necessity of scenario 1 with spec impact. This needs further discussion as option 1 is an optional feature given the presence of scenario 2.

As for scenario 3, it would be useful as a fallback operation. However, it is also pointed out that the latency/interruption time by LTM cannot be reduced with scenario 3. FL sees the necessity of a greater number of companies to start the discussion. Therefore, no FL proposal is made for scenario 3 in this meeting.

FL plan is to perform email discussion only.

[FL Proposal 5-3-5-v1]

- Simultaneous operation of Rel-17 ICBM and Rel-18 LTM is supported

  - FFS: Spec impact, e.g. TCI state pool for Rel-18 LTM can include TCI states for Rel-17 ICBM operation

  - FFS: UE capability

- Scenario 1 beam indication, i.e. beam indication before cell switch command, is supported

  - Beam indication for a target cell is indicated to a UE after TCI state activation (if supported) and before cell switch command.

  - Cell switch command validates the beam indication

  - FL note – companies’ comments are appreciated as it is not clear if this is a majority view.

[Comments to FL Proposal 5-3-5-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support FL proposal. |
| Ericsson | Suggest to make it two separate proposals. First bullet: Simultaneous operation of ICBM and LTM is supported – unless excluded Second bullet: Does this mean that scenario 1 is supported without ICBM? |
| Google | Support FL proposal in general. However, we suggest removing the first sub-bullet under the second bullet, which could be incorrect in some cases. For example, if TCI state activation activates only one TCI state, then the activated TCI state is the indicated TCI state. No further beam indication would be received. Last, the index of this FL proposal seems to be typo? |
| Futurewei | Support simultaneous operation of ICBM and LTM. Not support the second bullet, because the motivation is not clear. |
| QC | First, we don’t think ICBM + LTM implies Scenario 1 is supported without any spec change. For example, before CSC, the source cell with ICBM indicates a TCI #3 with CSI-RS #1 QCLed with target cell SSB #5. However, both TCI #3 and CSI-RS #1 are configured under the source cell. After CSC, UE has no clue which TCI to use if there are multiple TCIs configured under the target cell and with same root QCL source RS as target cell SSB #5. Second, separate mechanism is needed in absence of ICBM for complete solution for Scenario 1. So our view is that Scenario 1 is both not free and not very critical => lower priority or even no support |
| Apple | First bullet: On high-level from function perspective, Rel-17 ICBM is a subset of Rel-18 LTM procedure without triggering cell switching and UL sync. On the other hand, not sure the meaning of ’simultaneous’ since anyway Rel-17 ICBM and Rel-18 LTM are two separate features and indicated by separate UE capabilities. The candidate cell’s configuration, including measurement and TCI-state configuration, maybe different between these two, e.g., one configures candidate cell’s measurement and RS under the serving cell but the other maybe not. From this detailed design perspective, they are separate designs. These two features can be operated simultaneously if UE supports both, but it does not ‘have to’. Second bullet: For scenario 1, our understanding is that one or more than one TCI-states is activated first and then one of these activated TCI-states are selected by cell-switch command. We modify the FL proposal based on the explanation above: - Scenario 1 beam indication, i.e. beam indication before cell switch command, is supported - Beam indication for a target cell is indicated to a UE after TCI state activation (if supported) and before cell switch command. - Cell switch command indicates validate the beam indication. |
| Nokia | For the first bullet, we share Apple’s view. For the second bullet, we support Apple’s modification, but the second bullet should be FFS, as we think for scenario 1, there is no need to provide the beam indication again in the CSC; there can be other simpler mechanism, e.g., a flag to indicate if the MAC-CE contains a beam indication or not. Also, for scenario 3: we agree that supporting scenario 3 will not provide much benefit in terms of latency. However, we think other WGs (at least RAN2) thinks RACH after cell switch as a fall-back operation. We should first clarify if we have the same understanding in RAN1 or not? If yes, then why not scenario 3 as a fall-back operation? |
| MediaTek | We don’t support the second main bullet: - Scenario 1 beam indication, i.e. beam indication before cell switch command, is supported As we mentioned in our Tdoc, benefits of such two stage cell switch procedure is not clear to us and the design will be complicated compared to scenario 2. For the first main bullet, we prefer to wait till Rel-18 LTM design is mature. At this moment, how these two features can function together is not clear to us since many LTM designs are still open, e.g., configuration, beam indication, TCI state activation mechanism are all missing in Rel-18 LTM feature. Therefore, we prefer to deprioritize the discussion on simultaneous operation of Rel-17 ICBM and Rel-18 LTM. After all, simultaneous operation of Rel-17 ICBM and Rel-18 LTM won’t reduce handover latency and more like a optimization to us. |
| ZTE | We don’t support simultaneous operation of Rel-17 ICBM and Rel-18 LTM. For Scenario 1, we tend to discuss it until all designs related to scenario 2 are completed since discussing scenario 1 together with scenario 2 will make all discussions more complex and even delay progress of entire work item. |
| Lenovo | We suggest taking the two bullets as separate proposals. For the first bullet, Rel-17 ICBM is the starting point of Rel-18 LTM, therefore, both features can be concurrently configured to a UE. For the second bullet, we are fine to support scenario 1 in addition to scenario 2 since the handover latency can be reduced as well. How to indicate the TCI state or beam for a candidate cell can be FFS. |
| Fujitsu | Support the first bullet. For the second bullet, the meaning of “Cell switch command validates the beam indication” may need to be clarified. One interpretation could be that the beam will be applied after cell switch command. |
| vivo | Support FL proposal. |
| CMCC | For the first bullet, in general we share a similar idea as Apple. Further, we are not sure the ICBM and LTM would share exactly the same TCI state pool or the same size of the configured TCI state pool for the candidate cells which depends on the discussion of section 5.3.2. Considering there are other behaviours such as early DL synchronization and TA acquisition for LTM, those two are different features. And of course that if UE supports both feature, they could be configured simultaneously. For the updates of 2nd bullet from Apple, if the before cell swich command is removed, then does this scenario also contain the scenario 2 ? |
| IDCC | Support |
| LG | Fine only with the first bullet of the proposal. |
| FGI | We support the proposal in general. However, we think the first bullet and the second bullet can be discussed separately. For the first bullet, we share the similar view with Apple, i.e., Rel-17 ICBM and Rel-18 LTM are two separate features and indicated by separate UE capabilities. Therefore, we may discuss the details based on the following cases: - Simultaneous operation of Rel-17 ICBM and Rel-18 LTM - Operation of Rel-18 LTM without Rel-17 ICBM For the second bullet, due to the HO latency, we are fine to support Scenario 1. |
| Huawei, HiSilicon | We support the first main bullet and regard it as scenario 1. As far as I remember, the cell switch is originally in the scope of R17 ICBM but finally not specify due to time limit. Additionally, a UE operating with ICBM is usually at cell edge which has more chances to switch to other cell. If we only allow either ICBM or LTM, it leaves operator make a difficult choice between interruption and throughput. Compare the functions of ICBM and LTM, most the functions can be shared (e.g. L1 measurement and report, beam indication). If the measurement results and can be shared between ICBM and LTM, overhead can be significantly reduced. As for the standard impact, the beam indicated in ICBM can be contiguously used if the target cell is one of the candidate cells in ICBM even without explicitly indicated in CSC. The only thing needs to change is the mapping of TCI index from the source cell to target cell, which can be adjusted after cell switch. For the 2nd bullet, we are not sure why a sperate indication is needed before CSC as it is not used immediately. |
| Samsung | OK in principle. We don’t see a need for the following sub-bullet “Cell switch command validates the beam indication”. The cell switch command, would just complete the cell switch, before the cell switch command the cell the UE is communicating with is a non-serving cell, after the cell switch command, it becomes a serving cell. |

[FL observation]

Comments received from the companies (Other than support comments)

- Split the proposal (Ericsson, FGI)

- For the first proposal,

  - Add “unless excluded” (Ericsson)

  - Spec change may be necessary (QC)

  - Simultaneous operation is not a must (Apple, Nokia, CMCC, FGI)

  - Wait for the maturity of Rel-18 LTM, and it is optimization (MTK)

- For the second proposal,

  - Beam indication is not necessary if only TCI state is activated (Google)

  - Not support this proposal (Futurewei, Qualcomm, MediaTek, LG, Huawei)

    - large spec impacts foreseen

    - Is this essential?

    - What is the benefit to use separate indention?

  - Should be FFS (Nokia)

  - Meaning of “validation” is not clear (Apple, Fujitsu, Samsung)

- Other proposal

  - Introduction of scenario 3 for fallback operation (Nokia)

    - think other WGs (at least RAN2) thinks RACH after cell switch as a fall-back operation

    - FL note: Not sure if this should be discussed here, or another AI (9.10.2) is appropriate.

It is confirmed that a number of companies are not convinced with the benefit of scenario 1 independent from Rel-17 ICBM. Also, it was also commented that non negligible spec impact is foreseen with out significant benefit. FL thinks we can focus on the simultaneous operation of Rel-17 ICBM and Rel-18 LTM at this stage. Meanwhile, it is confirmed that at least two companies showed their concern on the simultaneous operation. Thus FL would like to propose a “softened” proposal.

In addition, the necessity of scenario 3 as a fallback operation was not discussed deeply yet. So, FL can gather the companies view.

[FL Proposal 5-3-5a-v2]

- Companies are encouraged to further study the spec impact when Rel-17 ICBM and Rel-18 LTM can be operated simultaneously, e.g.

  - TCI state pool for Rel-18 LTM can include or have any dependency of TCI states for Rel-17 ICBM operation

  - UE capability

  - Note: this topic will be handled with low priority in Rel-18

[FL note: the second proposal in FL proposal 5-3-5-v1 is dropped to focus on only one issue]

[FL Proposal 5-3-5b-v2]

- Companies are encouraged to consider the necessity of fallback operation for Rel-18 LTM where beam indication and/or RACH for candidate cell cannot be performed before/together with cell switch command

  - E.g. legacy L3 handover can be used for fallback operation and/or Rel-18 LTM with scenario 3 needs to be specified

[FL note: companies view on this new proposal 5-3-5b-v2 is appreciated]

[Comments to FL Proposal 5-3-5a-v2 and Proposal 5-3-5b-v2]

| Company | Comment |
| --- | --- |
| QC | For 5b, wonder any case that the beam indication or RACH cannot be performed? I guess serving cell dwelling time should be long enough for at least one RACH? |
| Nokia | For 5-3-5a-v2: Support For 5-3-5b-v2: we think it would be beneficial for the feature to support scenario 3 as a fallback mechanism to provide more flexibility to the network and this will not require any significant spec change. For example, we may just need to include an indication in the cell switch command if the UE is required/expected to perform RACH after the cell switch to acquire TA and initial QCL reference. |
| LG | Fine with proposals |
| NTT DOCOMO | For 5-3-5b-v2, not sure about the case ‘where beam indication and/or RACH for candidate cell cannot be performed before/together with cell switch command’. |
| Huawei, HiSilicon | For 5-3-5a-v2, we are fine to have the proposal without the note. It is not efficient for a UE not able to operate both ICBM and LTM simultaneously in the intra DU intra frequency case. For 5-3-5b-v2, is it about UE capability of LTM, like some UE can only perform pre DL sync and some UE can perform both DL and UE pre-sync? |
| Samsung2 | For Proposal 5-3-5a-v2, we think that is should be able to operate ICBM and LTM together as they provide complementary functions. For Proposal 5-3-5b-v2, use case is not clear. |
| Futurewei | We are fine with FL Proposal 5-3-5a-v2, but we suggest revising as below, ##### [FL Proposal 5-3-5a-v2] - Companies are encouraged to further study the spec impact when Rel-17 ICBM and Rel-18 LTM can be operated simultaneously for intra DU and inter DU cases, e.g. - TCI state pool for Rel-18 LTM can include or have any dependency of TCI states for Rel-17 ICBM operation - UE capability - Note: this topic will be handled with low priority in Rel-18 We are fine with the main bullet of FL Proposal 5-3-5b-v2, but we do not support scenario 3 is taken as a latency reduction solution for Rel-18 LTM. FL: for inter-DU case, Rel-17 ICBM is not supported for this case, so this modification goes beyond the original FL intention. Can you clarify the motivation? Otherwise, it is not easy to adopt this proposal. [FW] since Rel-18 LTM can cover Rel-17 ICBM, if it can be classified as intra DU and inter DU, it is much easy to differentiate the exact features for different scenarios (Rel-17 ICBM and real Rel-18 LTM). Does it make sense? |
| FL | Let’s continue the discussion to check the stability of FL Proposal 5-3-5a-v2 |
| Nokia | For 5-3-5b-v2, although we don’t have a strong opinion, but for the companies which don’t want to support, our question is this: Since RAN2 assumes RACH after cell switch as one option (e.g., when TA is not available before the cell switch because of any reason. Why can’t we use that scenario at least as a fallback operation even to acquire the initial QCL assumption, e.g., from the RACH procedure? It does not mean this will always be enabled. We don’t think supporting scenario 3 as a fall-back operation can create any issues or significant specification work. |
| NTT DOCOMO | Support 5-3-5a-v2. |
| Fujitsu | For 5-3-5a-v2: Support For 5-3-5b-v2: The scenario may need more clarification, e.g., whether due to UE capability. |
| FGI | We are ok with 5-3-5a-v2. |
| vivo | Support 5-3-5a-v2 without the note. Although ICBM is not the prerequisite of LTM, it is the easiest case to achieve LTM for intra-frequency and intra-DU scenarios. Since the priori procedure of LTM can reuse that of ICBM as much as possible, such as inter-cell L1 measurement, TCI activation and beam indication. Only the cell switch command needs to be introduced. Therefore, ICBM and LTM are operated simultaneously should be supported and emphasized. |
| ZTE | For 5-3-5a-v2: As comments in Round 1, We don’t support and discuss simultaneous operation of Rel-17 ICBM and Rel-18 LTM at this stage. For 5-3-5b-v2: More clarifications are needed. |

[Conclusion]

For 5-3-5a-v2 without note would be acceptable except one company (ZTE). Since this is a proposal for study, it is not necessary to modify and/or purse this proposal in this meeting because companies can anyway continue the study until the next meeting considering the final form of the proposal below

- Companies are encouraged to further study the spec impact when Rel-17 ICBM and Rel-18 LTM can be operated simultaneously, e.g.

  - TCI state pool for Rel-18 LTM can include or have any dependency of TCI states for Rel-17 ICBM operation

  - UE capability

For 5-3-5b-v2, it seems companies needs more time to think about the necessity of fallback operation. Thus, FL would like to close the discussion in this meeting, and come back at RAN1#113 based on the companies’ contribution.

With this understanding FL would close the discussion in this meeting.

[Comments if any]

| Company | Comment |
| --- | --- |

[Email approval] Beam application time

[Conclusion at RAN1#112]

The following FL proposal was made, but not discussed due to the lack of time

- Beam application time may be different from that for Rel-17 ICBM, FFS the exact value(s)

[Summary of contributions]

- Beam application time is different from Rel-17 ICMB (as inter-DU and/or inter-frequency scenario is supported)

  - ZTE, vivo, Fujitsu

- Ericsson: After the beam indication has been applied, the UE can receive all DL signals/channels, and transmit all UL signals/channels.

- IDC: Beam application time may be different from that for Rel 17 ICBM, FFS the exact value(s).

- Google: Support introducing a new beam application delay for unified TCI indication with serving cell change.

- Qualcomm: Application time should be specified for the SpCell/CG update command. Application time can be different for previously activated and deactivated new SpCell/CG

[FL observation]

Even though many companies have pointed out the necessity of the discussion on beam application time for Rel-18 LTM, which may be different from that for Rel-17 ICBM, no details are provided. FL understands it is obvious because the UE procedure before/after receiving cell swich command is not clarified yet. Therefore, FL would like to propose the same proposal as in RAN1#112, which intends to keep the discussion open.

[FL Proposal 5-3-6-v1]

- Beam application time may be different from that for Rel-17 ICBM, FFS the exact value(s) and condidtion

[Comments to FL Proposal 5-3-6-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support in principle. |
| Google | Suggest some wording revisions to make it general and fix typo. Beam application time for LTM may be different from that for Rel-17 ICBM without serving cell change, FFS the exact value(s) and condidtion |
| Futurewei | More details and evaluation results are needed. |
| QC | It would be beneficial to first align the definition of beam application time for LTM. Does it from end of CSC or corresponding ACK in case of Scenario 2? Then the exact value can be FFS or up to UE capability |
| ZTE | Support FL proposal |
| Lenovo | The proposal is not clear to us. If the beam application time refers that for the beam indication carried in the MAC CE for CSC, legacy MAC CE application time is enough. |
| Fujitsu | Support |
| vivo | We share a similar view with QC that the timeline should be clarified first and then discuss the exact value. |
| CMCC | Although we share the similar idea that the application time for LTM should be discussed and maybe different from that of ICBM, we prefer to discuss it later when the group has a clear picture of how the LTM operate. And if the beam indication is carried in the cell switch command which is in a MAC CE, then the beam should be applied after gNB receives the ACK of the MAC CE. |
| LG | The clarification on the BAT for LTM is pre-requisite for further discussion. |
| Huawei, HiSilicon | Support in general. RAN4 input is necessary. |
| Samsung | OK in principle. We should also clarify the definition of beam application time and whether this agreement is only for scenario 2 or other scenarios as well (if agreed). |

[FL observation]

It looks that most of the companies are fine with the direction, but some modification was proposed:

- Typo correction

- Definition of beam application time (QC, Lenovo, vivo, LG)

  - Start from End of CSC or corresponding ACK?

  - CMCC proposes to use ACK timing

- This can be discussed after the overall picture is clarified (CMCC)

- RAN4 input is necessary (Huawei)

From the companies view, it is obvious that RAN1 is not ready to start the discussion beam application time. Therefore, FL proposal is kept as a starting point for the future discussion without official agreement.

[FL Proposal 5-3-6-v2]

- Companies are encouraged to study the beam application time for Rel-18 LTM, which may be different from that without serving cell change (i.e. Rel-17 ICBM)

  - Definition of beam application time, i.e. start from cell switch command or corresponding ACK

  - The exact value(s), condition and UE capability

[FL note: no agreement is necessary in this meeting. This is a starting point for the next meeting]

[Comments to FL Proposal 5-3-6-v2]

| Company | Comment |
| --- | --- |
| QC | To our understanding, item to study can also be agreement. There are plenty examples for this. This is to better remind companies to comment in next meeting |
| Nokia | Support |
| LG | Fine with the proposal |
| Huawei, HiSilicon | “start from cell switch command or corresponding ACK” seems not the definition of beam application time. It is the start of beam application time. |
| Samsung2 | Fine with proposal |
| Futurewei | We are fine with FL Proposal 5-3-6-v2. |

[FL observation]

- Qualcomm: item to study can also be agreement

  - FL: OK, let’s try to agree!

- Huawei: “start from cell switch command or corresponding ACK” seems not the definition of beam application time. It is the start of beam application time

  - FL: OK. Let’s apply this!

[FL Proposal 5-3-6-v3]

- Companies are encouraged to study the beam application time for Rel-18 LTM, which may be different from that without serving cell change (i.e. Rel-17 ICBM)

  - Definition of the start of beam application time, i.e. start from cell switch command or corresponding ACK

  - The exact value(s), condition and UE capability

[Comments to FL Proposal 5-3-6-v3]

| Company | Comment |
| --- | --- |
| Nokia | Support the study proposal. |
| NTT DOCOMO | Support to study |
| Fujitsu | Support |
| Spreadtrum | Support to study. |
| Huawei, HiSilicon | Support 5-3-6-v3 |
| vivo | Support. |
| ZTE | Support. But wondering how to define the gap relative to the start of beam application time. |
| Samsung3 | Support |
| QC | Minor wording suggestion Definition of the start of beam application time, i.e. start from end of cell switch command or corresponding ACK |

[FL observation]

It seems the proposal is now close to stable (only editorial change were proposed). So, FL would like to go to an email approval process.

[FL Proposal 5-3-6-v4 For email approval]

- Companies are encouraged to study the beam application time for Rel-18 LTM, which may be different from that without serving cell change (i.e. Rel-17 ICBM)

  - Definition of the start of beam application time, i.e. end of cell switch command or corresponding ACK

  - The exact value(s), condition and UE capability

[Mid] Beam indication for multiple cells for CA

[Conclusion at RAN1#112]

The following proposal was not discussed in RAN1#112 due to the lack of time.

- The existing mechanism, i.e. simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2, is reused to indicate TCI states for multiple target cells

[Summary of contributions]

- Vivo: Only the beam indication for target Pcell is included in the cell switch command, and the beam indication can be applied to a set of intra-band CCs, which are included in the same list of simultaneous TCI state updating as that including target Pcell.

- Fujitsu: when a TCI state ID is indicated for a target cell, the TCI states with the same TCI state ID apply to the target cells in the same simultaneousU-TCI-UpdateList

- IDC: To indicate TCI states for multiple target cells, existing mechanisms are reused.

- Qualcomm: For the beam indication together with the cell switch command, support that the same indicated TCI state can be applied to multiple new cells in the same CC list

[FL observation]

Four companies discussed about this issue, and all companies are willing to reuse the existing mechanism. Therefore, FL would like to simply propose the following.

[FL Proposal 5-3-7-v1]

- For scenario 2, the beam indication for target Pcell is included in the cell switch command, and the beam indication is also applied to target Scell(s) included in the list of simultaneous TCI state, i.e. the same mechanism as simultaneousTCI-UpdateList is reused for Rel-18 LTM

[Comments to FL Proposal 5-3-7-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support in principle. |
| Ericsson | Support |
| Google | The parameter name should be “simultaneousU-TCI-UpdateList”. |
| Futurewei | We are ok with FL proposal. |
| QC | Support |
| Apple | Support. This should be also applicable for scenario 1. |
| Nokia | This is on supporting the CA scenario in the target cell, which is fine, but for completeness, shouldn’t we also to discuss the case when different beam indications are used for SCell(s). Otherwise, this overall topic of beam indication for multiple cells for CA can be discussed later after all the essential features are finalized. |
| ZTE | Support FL proposal. |
| Lenovo | We are fine with this proposal. |
| Fujitsu | Support |
| vivo | Support and the mechanism can be also applicable for scenario 1 if supported. |
| CMCC | Fine with the proposal. One question for clarification, should the simultaneousTCI-UpdateList be configured before the cell switch command or part of configuration of the candidate cells which would be activated after the cell switch? |
| IDCC | Support |
| LG | Fine with the proposal |
| Huawei, HiSilicon | We share similar view as Nokia. In our tdoc R1-2302368, we had following two proposal, Proposal 13: The beam indication in the cell switch command can be applied to a set of CCs, if simultaneous TCI state updating is configured for more than one target candidate cell. Proposal 14: One or more than one beam indication for the target cell(s) can be conveyed in the MAC CE used for LTM, each beam indication can be applied to a CC or a set of CCs which share the beam indication. As for FL proposal, we think it is too early to decide whether the beam indication in CSC is for Pcell or SCell. According to R17 unified TCI framework, the TCI is configured for a reference cell which can be applied to a set of CCs configured by simultaneousTCI-UpdateList. The reference cell can be PCell or SCell. Moreover, if not all CCs are in the same simultaneousU-TCI-UpdateList, multiple TCIs should be included in the same CSC. For scenario 2, the beam indication is included in the cell switch command, and the beam indication is applied to cell(s) included in the list of simultaneous TCI state, i.e. the same mechanism as simultaneousTCI-UpdateList is reused for Rel-18 LTM. FFS: when there are multiple simultaneousTCI-UpdateLists configured for the target cells. |
| OPPO | Ok with the proposal |
| Samsung | OK |

[FL Proposal 5-3-7-v2]

- For scenario 2 (and scenario 1 if agreed), the beam indication for target is applied to target Scell(s) multiple target cells included in the list of simultaneous TCI state, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM

  - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells.

  - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2

[Comments to FL Proposal 5-3-7-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We think the first ‘for target’ should be kept. Otherwise, it is not clear what ‘beam indication’ we’re discussing. In addition, we think the intention is to say, the beam indication for target cell is applied to the multiple cells in the same list as target cell. But we’re not sure the meaning of ‘multiple target cells’. Does it mean the case when cell group switch is indicated? |
| Fujitsu | Support in principle. Maybe we can just address scenario 2 for now. for scenario 1, since it has not been decided whether cell switch command can also carry beam indication, it may be ambiguous whether the beam indication refers to that before cell switch command or that together with cell switch command. |
| CMCC | In general, we are fine with the proposal. Whether scenario 1 is included we are open to discuss. |
| Ericsson | Support |
| vivo | For LTM, the key is the PCell switch. Thus beam indication for the target PCell should be signaled before/in the cell switch command, and the simultaneous TCI states update which reference cell is the target PCell can be achieved naturally. For the simultaneousU-TCI-UpdateList whose reference cell is target SCell, it needs further study. As beam indication is based on reported measurement results in general, if separate beam indication for the target SCell is supported, UE needs to perform L1 measurement on the potential target SCell(s) before the cell switch. However, whether/how to perform L1 measurement on potential target SCell(s) has not been discussed yet. To make the proposal clearer, we revise proposal 5-3-7-v2 as follows: ##### [FL Proposal 5-3-7-v2] - For scenario 2 (and scenario 1 if agreed), the beam indication for target PCellfor target is applied to target SCell(s)target Scell(s) multiple target cells included in the same list of simultaneous TCI state as target PCell, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells. Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2 |
| QC | Fine for FL’s proposal |
| ZTE | Fine with the updated proposal |
| MediaTek | We suggest to have a separate discussion on scenario 1 when scenario 1 is clarified and agreed. We don’t have concern on extending Rel-17 unified TCI functionality to Rel-18 but we would like to be specific on the applied scenario. In particular, we would like to limit the “multiple target cells” to be active target cells. At this stage, we don’t have agreement to allow UE to handover to target Pcell+scells only based on the cell switch command and we would like to decouple this aspect from the proposal. For example, we are not sure how to apply the indicated beam in a deactivated cell when UE handover to a target Pcell with other deactivated Scells. Therefore, we suggest the following wording - For scenario 2 (and scenario 1 if agreed), the beam indication in the cell switch command for target is applied to target Scell(s) multiple target cells included in the list of simultaneous TCI state when the target cells are active, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells. - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2 |
| LG | Fine in principle. For the FFS, there are up to 4 CC list as in Rel-17. Would someone clarify the issue the FFS? |
| Huawei, HiSilicon | Fine for FL Proposal 5-3-7-v2 and also the changes by MTK |
| Samsung2 | Fine with Proposal 5-3-7-v2 |
| Futurewei | We are fine with FL Proposal 5-3-7-v2. |

[FL Observation]

The issues pointed out is summarized as follows

- Meaning of “multiple target cells”

  - FL’s original intention is just the CA case, i.e. Scell(s) are activated by SCS

- Excluding scenario 1 for now

  - FL agrees that this is useful to ease the discussion.

- Beam indication is for Pcell or cell(s) in general?

  - FL tend to understand the concern, i.e. The intention of FL initial proposal was the same as vivo. On the other hand, it was pointed out by Huawei that “According to R17 unified TCI framework, the TCI is configured for a reference cell which can be applied to a set of CCs configured by simultaneousTCI-UpdateList. The reference cell can be PCell or SCell.” To move forward, how about putting FFS, i.e. “FFS the reference cell is PCell or SCell” ?

- Beam indication for deactivated scell(s)?

  - FL has not such intention.

- More than 4CC case

  - FL had no intention to discuss this issue because nobody touched this issue. It can be FFS.

[FL Proposal 5-3-7-v3]

- For scenario 2, the beam indication in the cell switch command is applied to multiple cells included in the list of simultaneous TCI state when the cells indicated in the list are active, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM

  - FFS: the reference cell is Pcell or Scell

  - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells.

  - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2

  - FFS how many cells (i.e. more than 4) can be supported with this mechanism

[Comments to FL Proposal 5-3-7-v3]

| Company | Comment |
| --- | --- |
| Nokia | Support |
| NTT DOCOMO | The meaning of ‘reference cell’ in first FFS is unclear. The meaning of ‘how many cells’ in last FFS is unclear. Does it mean how many CC list? |
| Fujitsu | Support |
| Huawei, HiSilicon | Suggest following updates in main bullet to resolve DCM’s concern. The reference cell is a term by MIMO that TCI state pool is defined on a reference cell and can be applied to other cells. - For scenario 2, a TCI state indicated in the cell switch command is applied to multiple cells included in the list of simultaneous TCI state when the cells indicated in the list are active, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM - FFS: the reference cell of the TCI state is Pcell or Scell - FFS: when there are multiple simultaneousU-TCI-UpdateList configured for the target cells. - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2 - FFS how many cells (i.e. more than 4) can be supported with this mechanism |
| vivo | First at all, sorry for confusion on the reference cell in the last round. In Rel-17 unified TCI framework, the reference cell is the CC which is provided with TCI state list and as a reference to certain CCs not configured with TCI state list. And the reference cell has no direct relationship with the list of simultaneous TCI state update, since the number of reference cell in the simultaneous TCI state update can be zero or more than 1. Therefore, we suggest deleting the first bullet to avoid confusion. Secondly, this issue should be discussed with the combination of beam indication in the cell switch command in 5.4.1. In our view, the key of LTM is the PCell switch. Thus beam indication for the target PCell should be signaled before/in the cell switch command, and the simultaneous TCI states update for the active target SCell which is included in the same list as the target SpCell can be achieved naturally. As for other active target SCell, to indicate the corresponding TCI state in the cell switch command, L1 measurement and report for the target active SCell needs to be performed before cell switch. However, whether/how to perform L1 measurement on potential target SCell(s) has not been discussed yet. Therefore, we only support simultaneous TCI state update is performed for the active target SCell(s) which is included in the same list as the target SpCell, it configured. |
| Nokia | We suggest the following changes to avoid confusion: - Delete the first FFS (vivo is fine with that) - Merge the fourth FFS to second FFS, i.e., in the second FFS change “multiple” “multiple (up to 4)” |
| ZTE | Support that beam indication is replaced with TCI state indication since such clarification is done. Besides, does “the cells indicated in the list are active” refer to the cells mentioned in here is from the activated cells from configured cells? Finally, we are not sure why we need to define the number of cells, is it related to cell activation? |
| Samsung3 | Fine in principle |
| QC | Two comments: (1) Suggest to add “after cell switch command” if that is the intention; (2) Suggest to add “if reference cell is configured”, since gNB may not enable this feature - For scenario 2, the beam indication in the cell switch command is applied to multiple cells included in the list of simultaneous TCI state when the cells indicated in the list are active after cell switch command, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM FFS: the reference cell is Pcell or Scell or any cell if reference cell is configured |

[FL Observation]

FL proposal is updated based on the companies’ proposals.

[FL Proposal 5-3-7-v4]

- For scenario 2, a TCI state indicated in the cell switch command is applied to multiple cells included in the list of simultaneous TCI state when the cells indicated in the list are active after cell switch command, i.e. the same mechanism as simultaneousU-TCI-UpdateList is reused for Rel-18 LTM

  - FFS: the reference cell of the TCI state is Pcell or Scell or any cell if reference cell is configured

  - FFS: when there are multiple (up to 4) simultaneousU-TCI-UpdateList configured for the target cells.

  - Note: RRC structures (i.e. under serving cell configuration or candidate cell configuration, etc) are up to RAN2

  - FFS how many cells (i.e. more than 4) can be supported with this mechanism

[Comments to FL Proposal 5-3-7-v4]

| Company | Comment |
| --- | --- |

[Closed] Beam indication for mTRP

[Conclusion at RAN1#112]

Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

[Summary of contributions]

- OPPO: For the beam indication of LTM in the case of mTRP:

  - The TCI state(s) indicated through inter-cell beam management is applied to UE-specific PDCCH/PDSCH, PUSCH and PUCCH, as specified in Rel-17

  - The TCI state indicated in LTM cell switch is applied to UE-common PDCCH/PDSCH.

- Nokia: Enabling Rel-17 group-based beam reporting for a candidate cell in order to trigger a mTRP operation may be useful.

- CMCC: The beam indication for multiple TRP under LTM can be discussed later.

- Qualcomm: To facilitate mTRP operation on the new cell, R17 group-based beam report can be extended to LTM to identify a group of simultaneously receivable DL beams for a candidate cell

  - Two CMR resource sets can be configured with each set including RS configured for LTM L1 measurement

- DCM: Do not consider mTRP case in Rel-18 LTM mobility procedure.

[FL observation]

Even though some companies provided necessary discussion points for mTRP, two companies (operators) mentioned that this is a low-priority issue. Therefore, FL suggest closing the discussion in this meeting, and come back in the future meeting based on the companies’ contributions.

With this understanding, the discussion in this section is closed.

[Comments if any]

| Company | Comment |
| --- | --- |
| Google | Agree with FL’s assessment |

Cell switch command

[High] Information included in Cell switch command

[Conclusion at RAN1#110b-e]

Agreement

- Interested companies are encouraged to perform technical analysis of the cell switch command from a RAN1 point of view, e.g.

  - Necessary information included in the command, which is relevant for RAN1 discussion

  - Necessary number of bits for the information

  - L1 impact or concern to use DCI or MAC CE for L1/L2 cell switch command

[Conclusion at RAN1#111]

Due to the lack of time, FL proposal 4-1-v4 was not discussed during the online session. Companies are encouraged to perform their analysis based on the final proposal in this meeting (which is FL proposal 4-1-v4 below).

- The following contents are further studied for the contents included in the cell switch command from layer 1 perspective. The bit number required for the contents and the condition of existence needs further discussion.

  - Cell identity / Cell group identity – (ID or index?, what is the necessity from physical layer POV)

  - TCI state ID/Beam indication –FL note: the relationship with the timing discussion (i.e. beam indication before cell switch command) need to be discussed

  - DL/UL BWP indication

  - Differentiation between Rel-17 ICBM and Rel-18 LTM (if the DCI for Rel-17 ICBM is reused for cell switch command) FL note: RAN2 agreed to use MAC CE for cell switch triggering on Tuesday in this meeting

  - TA value and/or TA acquisition indication

  - [UL resource indication for sending acknowledgement of LTM (if RAN1 identify the necessity from L1 point of view) ]

  - Triggered aperiodic CSI-RS resource indice(s)/ CSI-RS resource set ID/CSI report setting ID

    - e.g. for gNB/UE beam refinement, TRS tracking after cell switch command

  - Triggered aperiodic SRS resource set ID

FL note: it was agreed in RAN2 that MAC CE is used for triggering cell switch. This means that this discussion is not urgent in this meeting.

The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch.

LTM cell switch is supervised by a timer

UE arrival in the target cell need to be indicated (somehow)

[Conclusion at RAN1#112]

The following proposal was not treated at RAN1#112 because of the lack of time.

- From RAN1 point of view, at least the following information needs to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - FFS: TA related information (up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

  - ID of the active DL and UL BWPs for the target SpCell

- [Study further the necessity/effectuality and benefit of the following field and corresponding UE procedure]

  - Triggering of aperiodic TRS transmitted from the target cell

  - Triggering the CSI acquisition of the target cell and reporting to the target cell

  - Triggering of aperiodic SRS transmission to the target cell

- FFS: the presence of each field (i.e. always present or configurable)

- FFS: the bit size of each field, or can be felt to RAN2

FL note: yellow part can be removed if we can achieve the consensus during offline discussion

[Summary of contributions]

Number of companies discussed the parameters needed to be included in the cell switch command. Based on the contributions companies agreed with the majority of the information summarized for cell switch command in RAN1#112 conclusion with minor updates.

Number of companies noted that triggering aperiodic CSI by including a field in the cell switch command can be beneficial as it allows for early CSI acquisition, and that it can be an alternative solution for supporting CSI acquisition of the target cell before cell switch, similar benefits were discussed for triggering A-TRS for the purpose of reducing the latency of synchronization.

Discussions also included whether some parameters included in the previous meeting conclusion should be signalled explicitly or conveyed implicitly. For example, regarding signalling the active DL/UL BWP   Huawei/HiSi proposed this information can provided by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig for target cell, similar as in L3 handover. If the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig, the active BWP ID associating with the indicated TCI state can be additionally carried in cell switch command.

Similarly, Apple proposed that ‘additionPCIIndex’ (in Rel-17 IBCM) value configured by RRC can be used to identify the target cell in CSC MAC-CE.

[FL Proposal 5-4-1-v1]

- Proposal 1: From RAN1 point of view, at least the following information needs to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - C-RNTI

  - TA related information (details up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

  - Active DL and UL BWPs for the target SpCell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

- FFS: the presence of each field (i.e. always present or configurable)

Proposal 2: On the presence of TA related information and Beam indication for the target SpCell at the following information within cell switch command is dependent on the beam switch timing scenarios, i.e., if the beam switch is carried out before, during or after cell switch command:

- Alt 1: TA related information and Beam Indication for the target SpCell fields are always present in the cell switch command independent of the beam switch timing scenarios, i.e., if the beam switch is carried out before, during or after cell switch command

- Alt 2: TA related information and Beam Indication for the target SpCell fields are Only Present in joint beam switch and cell switch command scenario, i.e., Scenario 2

Proposal 3: The active DL and UL BWP for the target SpCell, within the cell switch command, will be indicated:

  - Alt 1: Explicitly, through active DL and UL BWP ID

  - Alt 2: Implicitly through the active BWP ID associating with the indicated TCI state

  - Alt 3: Implicitly through:

    - firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig

- and

- the active BWP ID associating with the indicated TCI state

[Comments to FL Proposal 5-4-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | For Proposal 2, we’d like to separate the discussion and options on the presence of ‘TA related information’ and ‘Beam Indication’. |
| Ericsson | Proposal 1: Not C-RNTI (static information, part of RRC) – otherwise OK Proposal 2: This would require that we support scenario 1 and 3, which we have not agreed Proposal 3: We prefer Alt1. We do not see how Alt2 would work. Alt3 would work. |
| Google | Regarding Proposal 1: - On information to identify the target cell(s), RAN2 has already agreed that CSC contains candidate configuration index. - Why we need C-RNTI in CSC? |
| Futurewei | For Proposal 1, C-RNTI is not included, which can be preconfigured in CellGroupConfig of candidate cell(s). For proposal 2, we only support Scenario 1 [ICBM] and 2. For proposal 3, we are ok with Alt 3. Alt 1 and Alt 2 have some additional singling overhead. |
| QC | For Proposal 1: C-RNTI can be RRC configured. Also, suggest to add back the FFS for triggering A-SRS. The use case is for target cell to refine the TA based on the A-SRS, if the last TA acquisition is not fresh or may not be accurate. For Proposal 2, good to discuss after support for Scenario 1 and 3 is concluded For Proposal 3, support Alt1. We don’t think Alt2 can work. The BWP ID in TCI is for the QCL source RS’s BWP, not that for the TCI. For Alt3, the wording is a bit confusing to us. Suggest to say that using firstActiveDownlinkBWP-Id if no explicit BWP indicated in CSC, if that is the intention |
| Apple | Proposal 1: No C-RNTI. Proposal 2: Our preference is Alt.1. Proposal 3: Alt.3 or Alt.1. |
| Nokia | Proposal 1: C-RNTI should be removed or FFS. Proposal 2: We should separate “TA” from the proposal because the issue of adding the TA or not depends on the discussions in 9.10.2; here, we can only focus on adding beam switch in the cell switch command or not. Also, we can discuss this issue once we make decision on supporting scenario 1 and 3. Proposal 3: Support Alt 1 or Alt 3. |
| MediaTek | Proposal 1: ok with current format. However, since TA related information should be discussed under 9.10.2, we suggest to remove it from Proposal 1 or at least add FFS or bracket before any concrete decision made under 9.10.2. In that case , we can discuss Proposal 2 later. Proposal 3: feasibility of Alt2 depends on the discussion of beam indication content (TCI state or SSB index) under 5.3.1. Therefore we suggest to deprioritize Proposal 3 discussion. |
| ZTE | Proposal 1: regarding C-RNTI, although it may be configured in RRC pre-configuration, but it has not been discussed in RAN2 and no any conclusions on it so far. So from our point of view, whether C-RNTI is included in the RRC pre-configuration is still unknown. Furthermore, even if RAN2 ultimately agrees to configure C-RNTI in RRC pre-configuration, we understand that RRC is a semi-static configuration and updates are relatively slow and this way will not only increase the burden and overhead of network configuration, but also may cause unnecessary waste since configured C-RNTI for each candidate cell is reserved and cannot be used for other cell or UE. Compared with RRC configuration method, we think it will provide more flexibility for NW to dynamically indicate C-RNTI of target cell to UE without bringing large signaling overhead, instead of configuring C-RNTI of all candidate cells. On the other hand, even if RAN2 will have a consensus on the C-RNTI of candidate cell included in RRC pre-configuration in the future, it also has no harms to provide C-RNTI in cell switch command since C-RNTI field can be considered as an optional in cell switch command. Proposal 2: Since Scenario 1 and Scenario 3 have not been supported in Rel-18 LTM, we don’t tend to discuss related information together with scenario. Proposal 3: Support Alt 1 or Alt 3 |
| Lenovo | Proposal 1: Suggest removing C-RNTI. Proposal 2: We prefer Alt1 to simply UE behaviour. Proposal 3: We support Alt1. |
| Fujitsu | Proposal 1: Support Proposal 2: TA may have more alternatives than those of TCI state. As for TA, whether it is indicated by cell switch command or not is up to configuration. Even for Scenario 2 alone, whether to have the TA field could also depend on the configuration. Proposal 3: Alt 1 or Alt 3 but not deriving BWP ID from the TCI state. |
| vivo | Proposal 1: C-RNTI should be RRC configured in the candidate cell configuration, rather than being indicated in the cell switch command. Proposal 2: TA-related issues should be discussed separately from the beam indication. Proposal 3: Support Alt 3. |
| Xiaomi | Proposal 2: First, the wording needs to modified as following: Proposal 2: … within cell switch command is dependent on the beam switch indication timing scenarios, … Secondly, we prefer to delay the discussion because we only support Scenario 2. |
| CMCC | Support proposal 1 without C-RNTI or put it into FFS. Whether there is a need to update the C-RNTI for the UE or through other procedure should depends on other groups discussion. For the proposal 2, whether the TA related information should always be in the cell switch command depends on the discussion in 9.10.2. For the proposal 3, we are fine with the Alt 1 and Alt 3. Since as the UE received the CSC, either the BWP indicated in the CSC will be activated or the BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id should be. It seems both alternatives have the same function. Then the active DL and UL BWP id may not be necessarily included in the CSC. If my understanding is correct, we have not concluded that the RRCreconfiguration of the candidate cells should be contained in the CellGroupConfig ? |
| LG | Proposal 1: C-RNTI is not needed since it is configured via RRC Proposal 2: Similar view with ZTE |
| Huawei, HiSilicon | For Proposal 1, it should be clarified whether all items listed are to be always included if agreed. Our understanding is only “Information to identify the target cell(s)” is mandatory required, or the others depends on network configuration. For example, TA is only required if RAR is not configured. Beam indication might not be necessary, if default beam is defined as a fallback mechanism. C-RNTI is not necessary as it can be provided to UE in candidate cell configuration. For proposal 2, we suggest discuss TA and beam indication separately. For TA, it should be covered in 9.10.2, and we think TA is needed when RACH less procedure is adopted and RAR is not configured. For beam indication, we need to figure out which scenarios will be supported as well as scenario 2. For proposal 3, Alt 1 or Alt 3 depends on the scenario. Alt 3 is a fallback. If the TCI state is associated with BWP different from firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id provided in CellGroupConfig, explicitly BWP ID should be provided in CSC. The text after “and” under alt 3 is not clear to us. |
| OPPO | Proposal 1: C-RNTI is not needed. Proposal 2:Alt 2 is supported. The description in Alt1 seems not correct, at least not clear. Proposal 3: Alt3 is supported. |
| Samsung | Proposal 1: Prefer to keep FFS on A-SRS Proposal 2: Prefer Alt2 Proposal 3: Prefer Alt1 |
| CATT | Proposal 1: C-RNTI is not necessary. Proposal 2: We prefer Alt2. Proposal 3: We support Alt1 or Alt3. But we share the same view as Huawei, that the text after “and” under alt 3 is not clear. |

[FL Proposal 5-4-1-v2]

- Proposal 1: From RAN1 point of view, at least the following information needs to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - C-RNTI

  - TA related information (details up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

  - Active DL and UL BWPs for the target SpCell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

- FFS: the presence of each field (i.e. always present or configurable)

Proposal 2: On the presence of TA related information and Beam indication for the target SpCell at the following information within cell switch command is dependent on the beam switch timing scenarios, i.e., if the beam switch is carried out before, during or after cell switch command:

- Alt 1: TA related information and Beam Indication for the target SpCell fields are always present in the cell switch command independent of the beam switch timing scenarios, i.e., if the beam switch is carried out before, during or after cell switch command

- Alt 2: TA related information and Beam Indication for the target SpCell fields are Only Present in joint beam switch and cell switch command scenario, i.e., Scenario 2

Proposal 3: The active DL and UL BWP for the target SpCell, within the cell switch command, will be indicated:

  - Alt 1: Explicitly, through active DL and UL BWP ID

  - Alt 2: Implicitly through the active BWP ID associating with the indicated TCI state

  - Alt 3: Implicitly through:

    - firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig

- and

- If the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig

  - Alt 3.1: The active BWP ID associating with the indicated TCI state will be used

  - Alt 3.2: An explicit active DL and UL BWP ID will be included in cell switch command

[Comments to FL Proposal 5-4-1-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Support the 3 proposals. |
| Fujitsu | Support in principle. For Proposal 3 Alt 3, the part “and the active BWP ID associating with the indicated TCI state” should be removed? [Mod] This is to cover the case if the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig, the active BWP ID. Added some clarification to capture this. |
| CMCC | Thanks FL for the updates. For the proposal 1, the 4th bullet of Active DL and UL BWPs for the target SpCell depends on the conclusion of Proposal 3. For the proposal 2, we share similar views that we should decide whether only scenario 2 is supported or not. - For the proposal 3, we suggest to remove and and make the last sentence (the active BWP ID associating with the indicated TCI state) as a sub sub-bullet of Alt 3, which guarantee that the indicated TCI state should be within the configuration of the active BWP. |
| Samsung | For proposal 1, suggest to keep C-RNTI as FFS for now. |
| Nokia | For proposal 2, we support Alt2. The reason is that at some point, if we agree to support the scenario 3, then a beam indication cannot be provided in the cell switch command. As we mentioned earlier, in order to align with other WGs, we should support the scenario 3 as a fallback mechanism (e.g., RAN2 supports RACH after cell switch as a fallback mechanism). |
| Ericsson | Proposal 1: Support Proposal 2: Wouldn’t be better to conclude that only Scenario 2 is supported? Proposal 3: Alt3 is unclear. We not agreed there is a TCI state in the cell-switch command. |
| LG | Support proposal 1 |
| Futurewei | We are fine with Proposal 1, because C-RNTI can be preconfigured in CellGroupConfig of candidate cell(s). For proposal 2, we only support Scenario 1 [ICBM] and 2. For proposal 3, we are ok with Alt 3. Alt 1 has some additional singling overhead. |

[FL Observation]

Proposal 1 in v3 should be agreeable to companies who raised concerned as the comments have been addressed.

In general Companies are ok with proposal 2 and 3 apart from some editorial changes which has been addressed in v3. Alt 3 in Proposal 3 was reworded to capture more detailed description.

[FL Proposal 5-4-1-v3]

FL Proposal 5-4-1-v3]

- Proposal 1: From RAN1 point of view, at least the following information needs can to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - TA related information (details up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

    - FFS: signaling details of beam indication, e.g., implicitly or explicitly

  - Active DL and UL BWPs for the target Spcell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

  - FFS: C-RNTI

- FFS: the presence of each field (i.e. always present or configurable)

Proposal 2: On the presence of TA related information and Beam indication for the target Cell the following information within cell switch command is dependent on the beam switch timing scenarios, i.e., if the beam switch is carried out before, during or after cell switch command:

- Alt 1: TA related information and Beam Indication for the target cell fields are always present in the cell switch command independent of the beam switch timing scenarios, i.e., if the beam switch is carried out before, if supported, during or after cell switch command

- Alt 2: TA related information and Beam Indication for the target cell fields are Only present in joint beam switch and cell switch command scenario, i.e., Scenario 2

Proposal 3: The active DL and UL BWP for the target SpCell, within the cell switch command, will be indicated:

  - Alt 1: Explicitly, through active DL and UL BWP ID

  - Alt 2: Implicitly through the active BWP ID associating with the indicated TCI state

  - Alt 3: Implicitly through:

    - firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig if no BWP is indicated in cell switch command

- If the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig

  - Alt 3.1: The active BWP ID associating with the indicated TCI state will be used

  - Alt 3.2: An explicit active DL and UL BWP ID will be included in cell switch command

[Comments on FL Proposal 5-4-1-v3]

| Company | Comment |
| --- | --- |
| vivo | Proposal 1: Support. Proposal 2: Support Alt 2. Proposal 3: For Alt 3, “The active BWP ID associating with the indicated TCI state” is confusing. Whether it is the BWP ID indicated in the QCL-info or the BWP ID whose corresponding BWP is configured with a TCI state list and the indicated TCI state belongs to the TCI state list. It needs further clarification. [Mod] Please see modified Alt 3, after discussion with few companies it seems Alt 3.1 if not a viable option. |
| ZTE | For proposal 1: according to discussion of beam indication in section 5.3.7, at this stage, we cannot first distinguish whether cell is SpCell or SCell. So suggest the following change: - Beam Indication for the target SpCell - FFS: signaling details of beam indication, e.g., implicitly or explicitly - Active DL and UL BWPs for the target SpCell [Mod] Please see updated proposal. For Proposal 2: Comment#1:Due to scenario 1: beam indication is indicated before cell switch command is not supported, so we tend to add some wording like “.... carried out before if supported, during,.....” Comment#2: a typo in main sentence, such as “of” is mistakenly removed. Comment#3: similar comment with proposal 1, whether we need to distinguish Cell type at this stage. [Mod] Please see updated proposal. For proposal 3: similar comment with proposal 1, whether we need to distinguish Cell type at this stage. |
| MediaTek | For proposal 2, we suggest to wait for the discussion outcome on the support of scenario 1 and scenarios 3. If both scenarios are excluded in Rel-18 LTM, then we can only have Alt2. [Mod] ok - For proposal 3, we need to first agree on the beam indication in the cell switch command is TCI state. However, even after we agree beam indication is TCI state ID, does Alt3 fall back to Alt1 when TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig? - [Mod] That is my understanding for Alt. 3. Proponent companies can explain further on their proposal. |
| Huawei, HiSilicon | Sorry for not give feedback in 2nd round. For proposal 1, the Beam indication and BWP ID should be target cell in order to keep consistent with other bullet. At least whether SpCell/PCell is under discussion in 5.3.7. [Mod] Please see updated proposal. For proposal 2, currently we only have scenario 2. Maybe we can defer this proposal after we have new scenario supported. [Mod] ok Proposal 3, the sentences after the “and” can be a separate proposal. it is mechanism to resolve conflicting indication. [Mod] not sure which “and” you are referring to can you please let me know your rewording proposal. |
| CMCC3 | Thanks FL’s updates. For the proposal 1, we suggest putting the “Active DL and UL BWPs for the target SpCell” into FFS. If the RRC configuration of the candidate cell or target cell, UE can get the the active BWPs information. Then there is no need to include this information in the CSC. [Mod] There is an FFS on how this information will be conveyed explicitly or implicitly, this should address your concern For the proposal 3, do we have a common understanding that all the configurations of candidate cell is provided by CellGroupConfig ? |
| IDCC | Fine with the proposals. |
| Samsung2 | For proposal1, suggest to change “needs to be included” to “can be included”, the reason is that in some cases, some parameters might not be included, for example, for TA, if there is a RAR, the UE might already have the TA, and for scenario 1, the beam indication might not be included. [Mod] Done Fine with proposal 2 to discuss the two alternatives. Support Alt2. I assume that down selection will be in RAN1#113. Fine with proposal 3 to discuss the alternatives. Support Alt1. I assume that down selection will be in RAN1#113. |
| CATT | Proposal 1: support Proposal 2: support Alt 2 Proposal 3: support Alt 1 |
| FGI | Proposal 1: We are fine with the proposal after updating the beam indication for the target ‘SpCell’ to target ‘cell’, which needs further discussion in 5.3.7. Proposal 2: We share the similar view with QC and MTK, i.e., we can wait for the discussion outcome on the support of scenario 1 and scenarios 3. Proposal 3: We support Alt1. |
| Lenovo | Proposal 1: Fine Proposal 2: Prefer Alt1. Proposal 3: Support Alt1. |
| Futurewei | For Proposal 1, C-RNTI is not included, which can be preconfigured in CellGroupConfig of candidate cell(s). For proposal 2, we only support Scenario 1 [ICBM] and 2. For proposal 3, we are ok with Alt 3. Alt 1 has some additional singling overhead. |
| Nokia | Proposal 1: Support Proposal 2: Support Alt 2, and we also support to discuss this later (as mentioned at least by MTK, HW, Q) when support of an additional scenario (scenario 1/3) gets agreed. Proposal 3: Support both Alt 1 and Alt 3 |
| NTT DOCOMO | Support the 3 proposals. |
| Fujitsu | Proposal 1, 2: Support Proposal 3: We have a similar view with Huawei. The first half of Alt 3 as following can already be an independent alternative. - Alt 3: Implicitly through: firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig |
| Spreadtrum | Proposal 1: Support. Proposal 2: Support Alt 2. Since only scenario 2 is agreed now. Proposal 3: Support Alt 3. firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id |
| Huawei, HiSilicon2 | Support proposal 1. We still think proposal 2 should be deferred. For proposal 3, are we going to down-select or support both? We support Alt 3. From my perspective, the current Alt3 include both implicit and explicit indication mechanism and further provide when implicit and explicit method are to be used. |
| vivo | Proposal 1: First of all, as mentioned by Futurewei that C-RNTI should not be included. For beam indication for target cell, we only support beam indication for the target SpCell included in the cell switch command. Combined with the discussion in 5.3.7, if simultaneous TCI state update is supported, the beam indication for the active target SCell included in the same list as the target SpCell also can be determined naturally. As for other active target SCell, it is unnecessary to indicate corresponding TCI state in the switch command. Otherwise, L1 measurement and report for target SCell needs to be performed before cell switch, which will increase UE computation overhead. It is the same for the DL/UL active BWP id. So, we suggest revising the wording to the old version: - Beam Indication for the target SpCell - FFS: signaling details of beam indication, e.g., implicitly or explicitly - Active DL and UL BWPs for the target SpCell Proposal 2: Support Alt 2. Proposal 3: Support only implicit indication in Alt 3, i.e., firstActiveDown linkBWP-Id and firstActiveUplin kBWP-Id |
| ZTE | For proposal 1: Comment#1: a typo should be fixed, that is, remove “to” in main sentence. Comment#2: according to the following agreement reached online session, it seems that beam indication can be replaced with “TCI state index” to avoid being misunderstood as “RS index” Agreement - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) , - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index |
| QC | Proposal 1: Support Proposal 2: Better wait for decision of Scenario 1, and coexistence of different Scenarios. If different scenarios can be enabled simultaneously, we think Alt1 is better to always keep fixed MAC-CE size Proposal 3: Support Alt1, which should be sufficient |

Preparation for LTM before reception of cell switch command

[High] Details on DL synchronization to candidate cell(s)

[Conclusion at RAN1#110b-e]

Agreement

- RAN1 to further study the potential RAN1 enhancements and spec impact to perform at least the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay / interruption

  - DL synchronization for candidate cell(s)

  - TRS tracking for candidate cell(s)

  - CSI acquisition for candidate cell(s)

  - Activation/Selection of TCI states for candidate cell(s), if feasible

  - Note: Uplink synchronization aspect will not be discussed under this A.I.

  - FFS: Whether the above procedures prior to the reception of L1/L2 cell switch command can be performed on candidate cell when it is deactivated Scell (if defined in RAN2)

- Detailed discussion will be commenced after receiving RAN2 LS.

[Conclusion at RAN1#111]

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

[Conclusion at RAN1#112]

The following FL proposal was made and discussed:

- Companies are encouraged to study the following aspects related to the DL synchronization and TCI state activation when Rel-17 unified TCI is used for LTM beam indication:

  - Timing to perform DL synchronization

    - Alt.1 Two-step DL synchronization procedure

      - UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement and then

      - gNB activates TCI state(s), and then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the TCI states

    - Alt.2-1 One-step DL synchronization procedure

      - UE maintains DL synchronization with SSB after L1 measurement

    - Alt.2-2 One-step DL synchronization procedure

      - gNB activates TCI state(s), and then UE starts DL synchronization with the QCL source of the TCI states

  - Necessity for DL synchronization for TA: whether and how DL synchronized is performed before TA

  - Applicability of CSI-RS (if agreed) in addition to SSB

  - RAN1 spec impact (UE capability, configuration, activation etc)

  - Timing of TCI state activation, i.e. whether TCI state activation is performed before TCI state indication or together with TCI state indication.

The important aspects for the next meeting are clarified during the session as follows:

- Alternatives are just for study, and other alternatives are not precluded

- DL synchronization in alt 2-1 is to find frame boundary and for TA management

- RAN1 spec impact includes, e.g. gNB indication of the cell(s) to maintain DL synchronization

- Agreement on this proposal is not necessary as the list of alternatives is not well formulated. Instead, it can be captured in the FL summary and used for the discussion in the next meeting.

[Summary of contributions]

Number of companies discussed DL synchronization issue in the contributions submitted for RAN1-112bis. Based on the contributions, majority of the companies believe a Two-step DL synchronization procedure should be considered, i.e.,

- Timing to perform DL synchronization

  - Alt.1 Two-step DL synchronization procedure

    - UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement and then

    - gNB activates TCI state(s), and then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the TCI states

Few companies also pointed out that the natural interpretation of the agreement below is that the UE can directly receive the PDCCH/PDSCH from target without waiting for QCL-TypeA source as specified in the PDCCH/PDSCH TCI state. The consequence of this interpretation is that it would require the UE also estimates the QCL-TypeA properties from the SSB, as the UE does before receiving the RRC configuration in legacy. And given this interpretation a One-step step DL synchronization procedure based on SSB may be sufficient. Hence, no additional dedicated signalling apart from SSB is needed to support DL pre-synchronization

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

Companies unfirmly agree on the impact of UE computational complexity on the number of SSBs for which the UE can store the QCL properties. Hence, the need for a mechanism to enable the UE to select a subset of the reported SSB for maintaining QCL properties.

Furthermore, the UE may have limited capability in terms of tracking/maintaining DL synchronization with multiple candidate cells. Hence, the need for UE capability reporting, where the UE reports the maximal number of candidate cells that it can support synchronization before cell switch command. Hence, the need for a mechanism to inform the UE about the candidate cells with which the DL synchronization maintenance need to be performed.

[FL Proposal 5-5-1-v1]

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

Question 1: Does the above agreement require the UE (capable of Rel-18 LTM) to also estimates the QCL-TypeA properties from the SSB and furthermore PDxCH reception can be done directly following DL synchronization performed using the SSB only?

FL Proposal 1: For DL synchronization timing when Rel-17 unified TCI is used for LTM beam indication

- Two-step timing DL synchronization procedure is defined:

  - UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement and then

  - gNB activates TCI state(s), and then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the TCI states

- Necessity for DL synchronization for TA: whether and how DL synchronization is performed before TA

- Applicability of CSI-RS (if agreed) in addition to SSB

- RAN1 spec impact (UE capability, configuration, activation etc)

- Timing of TCI state activation, i.e. whether TCI state activation is performed before TCI state indication or together with TCI state indication.

FL Proposal 2: For R-18 LTM introduce a mechanism to enable the UE to select a subset of the reported SSB for storing/maintaining DL synchronization timing information.

- Alt 1: SSB subset selection is done explicitly by gNB

  - FFS details of the signalling, whether subset selection is done at RRC or MAC CE level

- Alt 2: SSB subset selection is performed by the UE

  - UE would store/maintain DL synchronization timing information for the N strongest SSBs (based on UE capability) based on the measurement results

Note: Total number of SSBs where store/maintain DL synchronization timing information is maintained is a UE capability

FL Proposal 3: DL synchronization for candidate cells before cell switch command (if an agreed scenarios for Rel-18 LTM), is a UE capability.

FL Proposal 4: Maximum number of candidate cells where DL synchronization is carried out before cell switch command, is a UE capability.

FL Proposal 5: UE is expected to perform DL synchronization at least based on SSB, before TA operation.

[Comments to FL Proposal 5-5-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | For FL Proposal 1, I do not understand the following bullet. We think DL synchronization is needed before TA. - Necessity for DL synchronization for TA: whether and how DL synchronization is performed before TA Generally okay with other FL proposals. |
| Ericsson | Proposal 1: Do not support additional signalling to activate TCI states. From the proposal, it looks like the UE only needs an indication on which SSB to use. Is that true? Proposal 2: Fine to study both options until the next meeting Proposal 3: Do not support. Without DL pre-synchronization, LTM performance would be no better than L3 HO. Proposal 4: Support, as long as the minimum capability is 1. Proposal 5: Support. |
| Futurewei | For Proposal 1, unless we agree that CSI-RS can be used for measurement of candidate cell(s), we do NOT see the benefits of Two-step timing DL synchronization procedure, because only SSB(s) can be used for DL synchronization. For Proposal 2, we are ok with Alt 2 because it is an implementation issue. We are ok with Proposal 3. We are ok with Proposal 4. We are ok with Proposal 5. |
| QC | For Q1, no. To our understanding, DL sync means cell level sync, i.e. frame boundary timing based on earliest arrival path of all SSBs. It is not beam level sync, i.e. QCL-A for every SSB For Proposal 1: Fine for such 2 steps to combine both cell level and beam level timings For Proposal 2: Not support. SSB for timing can only achieve initial access performance, which could be even worse than source cell with refined beam. Prefer TCI activation, which can provide time/freq. tracking for refined DL beam For Proposal 3: Support For Proposal 4: Support For Proposal 5: Support |
| Apple | Proposal 1: We do not support UE blindly maintains DL sync for all reported SSBs after measurement report as it may be too many. Instead, UE can maintain the TO/FO based on the reported RS that are selected by NW for DL sync and UL sync operation. This minimizes the UE complexity for DL sync maintaining and still can achieve fast cell switch purpose. Therefore, we support the following two steps: - gNB activates TCI state(s) based on L1 measurement report, and - then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the activated TCI states where the number of candidate cells with activated TCI-states are subject to UE capability.. Proposal 2: Fine to study both. Our current preference is Alt.1 where SSB is selected by NW and indicated to UE by TCI-state activation MAC-CE or PDCCH order DCI. Proposal 3: DL sync before cell-switch command is a key component to achieve the design goal of LTM. Otherwise, at least DL sync latency is same as in L3 handover. In this sense, scenario 1 should be a basic design. Even for scenario 2, what indicated in cell-switch command is an activated TCI-state, instead of ‘deactivated’ TCI-state; Otherwise, at least one SSB period is required after cell switch command. Proposal 4: Support. Proposal 5: Support. |
| Nokia | For proposal 1: Support For Proposal 2, it would be good if it can be clarified that if this proposal is only for the first step of the proposal 1 (DL synchronization with SSB)? So given that we agree on SSB based DL synchronization then we can discuss the proposal 2. In general, we are fine with Proposal 2; however, under Alt 2: we should not preclude other options at this stage. We suggest adding the following option under Alt 2: - UE would store/maintain DL synchronization timing information for the SSB(s) given in the PDCCH orders for early TA acquisition So, we consider both proposal 1 and proposal 2, that means that, first the UE will be indicated (explicitly/implicitly) with SSBs to be used for first-step DL synchronization and then there will be a TCI activation which will be used to further select a sub-set of SSBs (because so far we have agreed to only support SSBs) for the second-step synchronization. Is that the correct understanding? Support Proposal 3, proposal 4, proposal 5. |
| MediaTek | For Q1, we share the same view with Qualcomm. For Proposal 1 step 1: UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement, we wonder what is the RAN1 impact? Our understand is the cells for L1 measurement should be a subset of cells for L3 measurement? If that’s the case , then UE should maintain those cell information(timing information) for all the cells UE performed L3 measurement since L3 measurement should be performed periodically? Therefore, we are not sure what is the DL sync information mentioned in the step 1 which is not used for L3 measurement? For step 2, we share similar view with Apple. However, it should be sufficient to say : activation of TCI state (s) associated with SSBs of target and/or candidate cells before cell switch is supported subject to UE capability what is the UE behavior after a TCI is activated should be clear. For Proposal2, as our comment on Proposal 1, we need to know what is the DL sync information mentioned in the proposal which is not used(maintained) for L3 measurement? For Proposal 4, we don’t support the current format. In our view, number of candidate beams is more crucial to implementation than the number of candidate cells. Also, we may need to discuss candidate cell number for intra and inter frequency separately. In our view, we can discuss Proposal 4 in UE feature session. For Proposal 5, general ok but can we elaborate a little bit more on TA operation? |
| ZTE | Proposal 1: For 2nd bullet, we have same concern with DOCOMO. Proposal 2: These two alternatives can be further studied and slightly prefer Alt1. Proposal 3/4/5: Support. |
| Lenovo | Proposal 1: SSB based DL synchronization is enough. On the TCI state indication, as it is indicated by the CSC MAC CE, the indicated TCI state is just the single activate TCI state for the target cell. Proposal 2: Fine to study both alternatives. Proposal 3: Support Proposal 4: Support Proposal 5: Support |
| Fujitsu | Proposal 1: Support. Proposal 2: Support. Alt 2 is preferred. Proposal 3, 4, 5: Support. |
| vivo | For Proposal 1: Support For Proposal 2: if this proposal is only for the first step of proposal 1, we are fine and prefer Alt 1. For Proposal 3: Support For Proposal 4: Support For Proposal 5: Support |
| Xiaomi | Proposal 1: For the first step, does it mean that UE needs to maintains DL synchronization with SSB every time the L1 measurement is performed? If it does. Then, we only support One-step DL synchronization procedure that UE maintains DL synchronization with SSB after L1 measurement. Proposal 2: Ok to further study. Proposal 3: Support Proposal 4: Support Proposal 5: Support |
| CMCC | In general we are fine with proposal 2,3,4 and 5. For the proposal 3, is it a general description of the UE capability supporting the DL sync to candidate cells before cell switch ? if that is the intention we have no problem. For the proposal 2, Alt 1 is preferred from our side since it should be decided by gNB which subset of SSB would the UE take the measurements. For Q1, no. Our understanding is that the agreements do not require the UE to estimates any QCL type from the SSB and to be used for the further reception of the PDXCH. But according to the legacy procedure and behaviour, if the assumption of QCL stands, the SSB can be used to facilitate the demodulation of the PDxCH and solve the issue of Rel-15 TCI framework and the channel and signals that not follows the Rel-17 unified TCI state. For the proposal 1, we kind of share the similar view as Apple that gNB should active a subset for the DL sync and measurement of the candidate cell. And for the 2nd sub-bullet under the 1st bullet (gNB activates TCI state(s), and then the UE starts DL synchronization..) it should be further discuss if this is the same procedure of cell switch command. If only one TCI state is indicated for the 2nd bullet, it is same as cell switch command. |
| LG | Proposal 1: Similar view with Futurewei that it is unclear for the benefit coming from two-step timing DL sync when using SSB only and it would be taken CSI-RS into account for that. |
| Huawei, HiSilicon | For proposal 1, we support the main bullet and 2 step ways. For the rest of bullets, are these for FFS or for support? For proposal 2, fine to support the mechanism. Both Alternatives listed by FL and mentioned by Nokia can be FFS. For proposal 3, we think perform DL sync before CSC should be basic feature group. We echo Ericsson comment that there is no benefit compared with L3 HO if UE perform DL sync after CSC. For proposal 4, 5, we support them. |
| OPPO | Porposal 1: do not support Proposal 2: ok to study. Suggest to change the main bullet to: FL Proposal 2: For R-18 LTM, study whether/how to introduce a mechanism to enable the UE to select a subset of the reported SSB for storing/maintaining DL synchronization timing information. Proposal 3: ok Proposal 4: ok Proposal 5: do not support. The proposal is not clear. What is “TA operation”? If it means the PDCCH order RACH preamble transmission, it can be up to UE implementation, why do we need an agreement to restrict the UE behaviour. |
| Samsung | OK with proposals 3, 4 and 5. |
| CATT | For Proposal 1: Support For Proposal 2: OK to study both Alternatives. But we prefer Alt.1. For Proposal 3: Support For Proposal 4: Support For Proposal 5: Support |
| Panasonic | For Q1, we think the confusion comes from the meaning of “DL synchronization” in LTM context. Maybe we can take QC’s comment to distinguish between cell-level DL sync and beam-level DL sync, where cell-level DL sync means to find frame boundary and for TA management and beam-level DL sync means to estimates the QCL-TypeA properties for PDCCH/PDSCH reception? We think such distinguish might also be helpful for the discussion on Proposals 1,2,3,4,5. For Proposal 1, the first step is to achieve cell-level sync and the second step is for the beam-level sync. As mentioned by Apple in the above reply, such two-step approach is useful to reduce UE’s complexity/effort compared to the case that UE has to maintain beam-level sync for all SSBs measured/detected. On the other hand, even without the two-step approach, similar saving effect can be achieved by not requiring UE to maintain beam-level sync for all SSBs measured/detected. For example, UE is only required to maintain beam-level sync for at least one SSB beam based on UE selection (similar to Proposal 2, Alt 2). For Proposal 2, it is beam-level sync. We think both alternatives can be studied. For Proposal 3, is it a beam-level or cell-level sync intended in this proposal? We think Gnb should be able to indicate which cell(s) and which beam(s) that UE has to maintain sync. LTM-capable UE should be able to obtain sync to at least one SSB of one candidate cell before cell switch. For Proposal 4, is DL sync here cell-level? For Proposal 5, is DL sync here beam-level? |

[FL Proposal 5-5-1-v2]

FL Proposal 1: For DL synchronization timing when Rel-17 unified TCI is used for LTM beam indication:

- Two-step timing DL synchronization procedure is defined:

  - Step 1: UE maintains DL synchronization (cell level time/frequency synchronization) with SSB

    - Total number of SSBs used for cell level synchronization and total number of candidate cells where cell level synchronization in Step 1 is a UE capability

      - Details of the UE capability will be handled during UE capability session

    - FFS: how and when candidate cells and SSB in candidate cell are determined

    - Step 1 does not imply UE will derive QCL-TypeA properties from the SSBs used for L1 measurement.

  - Step 2: gNB activates TCI state(s), and then the UE starts DL synchronization for PDSCH/PDCCH reception (beam level time/frequency synchronization)

    - Total number beam level synchronization used in Step 2 is a UE capability

      - Details of the UE capability will be handled during UE capability session

    - FFS: scenarios where Step 2 is applicable

FL Proposal 2: For R-18 LTM introduce a mechanism to enable the UE to select a subset of the SSB used for cell level DL synchronization timing information.

- Alt 1: SSB subset selection is done explicitly by gNB

  - FFS details of the signalling, whether subset selection is done at RRC or MAC CE level

- Alt 2: SSB subset selection is performed by the UE

  - UE would store/maintain DL synchronization timing information for the N strongest SSBs (based on UE capability) based on the measurement results

- Alt 3: UE would store/maintain DL synchronization timing information for the SSB(s) given in the PDCCH orders for early TA acquisition

FL Proposal 3: TCI state activation:

- Alt 1: Is performed before TCI state indication

- Alt 2: Is performed together with TCI state indication

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FL Proposal 4: Maximum number of candidate cells where DL synchronization is carried out before cell switch command, is a UE capability.

- Minimum supported value for this feature will be discussed in the UE capability session

FL Proposal 5: UE is expected to perform cell level DL synchronization at least based on SSB, before TA operation.

[Comments to FL Proposal 5-5-1-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | There are two UE capabilities in Proposal 1. Then Proposal 4 for similar UE capability is not needed? [Mod] Good point. Will remove proposal 4. |
| Vivo | On proposal 1, in step 1 the main bullet say “UE maintains ..” then he UE capability is about how many candidate cells the UE can maintain DL synchronization. And, it should be “candidate cells”, even in the CA scenarios if the SCell is serving cell it can be candidate cell in LTM [Mod] As the sub-bullet mentions we will not be discussing the UE capability description on this track. Details of UE capabilities and exact wording for capability can be discussed and finalized during the UE capability session. However, for clarification I added more description. in step 2, I think the question is whether TCI states for the target cell are activated by serving gNB, this means this step happens before cell switch, now the question is how this is signalled, either with separate signalling before cell switch or signalled in cell switch command. Regarding newly added sub-bullet on “total number of DL …in step 2 is UE capability”, I don’t understand the motivation. TCI states are activated for the target cell not for candidate cells [Mod] We still need a UE capability to determine how many DL synchronization timing information used in Step 2 independent if its serving cell or candidate cell. UE may want to support different number of active TCI in R19 LTM compared to legacy for serving cell. Again, we are not going to discuss the details of UE capability in this session. |
| Fujitsu | Proposal 1: Support in principle. As for “Total number of DL synchronization timing information”, it may not be clear what the information is, and what is the relationship between the information and the candidate cells in Step 1. Can we just say something like “number of candidate cells” in Step 1 and “number of beams” in Step 2? [Mod] Thank you for the comments, made some modification. For Step-1 UE capability some companies wanted to capture both umber of SSBs and Candidate cells during GTW. For Proposal 2: We are OK to study these alternatives. Alt 2 is preferred. For Proposal 3: We are OK to study these alternatives, although we think Alt 1 should be supported. For Proposal 5: Support |
| Panasonic | For Proposal 1, we have the same question as Fujitsu, and we support the changes suggested by Fujitsu: - Total number of candidate cells where UE maintains DL synchronization in Step 1 is a UE capability - Total number of beams where UE maintains DL synchronization in Step 2 is a UE capability [Mod] Some companies want to also limit the total number of SSBs across candidate cells not just candidate cells. I did further rewording. I want to emphasis again we are not trying to design UE capabilities here, just a general note which can be used in UE capability discussion sessions For Proposal 2, we suggest to add “beam level” in the main bullet. As from our understanding, this proposal is related to step 2 in Proposal 1. [Mod] This is related to Step 1 in Proposal 1. For Proposal 5, we suggest to add “cell level”, and delete “at least”. - UE is expected to perform cell level DL synchronization at least based on SSB, before TA operation. [Mod] Done |
| CMCC | Thanks for the updates. For the two steps, whether there is a need to differentiate cell level or beam level T/F synchronization needs more clarification. For the DL synchronization we do not differentiate them. And beam level or cell level may not needed. And as there is not any behavior under step 1, we are not sure if we need to maintain the DL synchronization. Or we can update the step1 as follows - Step 1: UE maintains DL synchronization (cell level time/frequency synchronization) with SSB and take L1 measurement. And our thinking about the UE capability of the number of maintaining the DL synchronization should be the same capability. For the 1st sub bullet under step 1 and 2, there is no need to differentiate the capability is for step 1 or step 2. The newly added Alt 3 seems do not have strong relations with the other alternatives. But we consider it is a fact that UE have to store or maintain the DL synchronization before TA acquisition. In proposal 3, if the TCI state indication refers to the beam indication, only alt 1 is aligned with the current specification. We do not think the alt 2 is possible. Proposal 5 is kind of overlapped with the Alt 3 under proposal 2. |
| Samsung | For step 2 of proposal 1, if the intention is to only have TCI states for one candidate cell activated (in addition to those of the serving cell). In this case, we don’t see a need for “Total number beam level synchronization used in Step 2 is a UE capability”. If the intention is to activate TCI states for more than one cell, we should first discuss if this is supported and what are the use cases, when this would be applicable. In step 2, timing of when the synchronization would be done is also FFS. Step 2 (i.e., synchronization for PDxCH) is something that UE needs to perform to receive PDxCH. However, the details are up to the UEs implementation. The key aspect for the network is that before a DL/Joint TCI state is indicated to the UE, the UE should have achieved synchronization and be ready to receive the PDxCH channel. This can be achieved by having a large enough time between the activation of TCI state and indication of TCI states for candidate cell. |
| Nokia | Thanks for the updated proposal. We have the following comments. As we realized that some companies do not have the same understanding on the need of TCI activation, and without that it will be really heard to have consensus on the step 2 of proposal 1. Therefore, we recommend discussing the Proposal 3 before proposal 1. [Mod] Proposal 1 tried to describe the DL sync framework, there is an FFS when step 2 is needed. Which can be clarified later. Let’s try to agree on the framework. For Proposal 3: We suggest the following modification: FL Proposal 3: For the Rel-17 unified TCI based beam indication in Rel-18 LTM, TCI state activation: - Alt 1: Is performed before TCI state beam indication, FFS: signalling details for TCI state activation - Alt 2: Is performed together with TCI state beam indication - Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability [Mod] Thank you for rewording, updated the proposal in v3. For Proposal 1, step 1, in the first sub-bullet, we can simply say “Total number of cell level synchronization in step 1 is a UE capability”, this will be aligned with the sub-bullet in step 2. Otherwise, the current wording where we use both SSB and candidate cell is not clear. For Proposal 2: The step 2 of proposal 1 (using activated TCI states) is also one of the alternatives when only SSB are supported. |
| Ericsson | Proposal 1: Do not support. What happened to the version in the chair notes from yesterday? We will not support that the gNB activates TCI states. TCI state activate is something the UE does. For the capabilities, we want to clarify if 0 is a valid value for the capabilities. [Mod] Changed the wording for step 2 to remove the explicit mention of gNB activating TCI. We are not discussing UE capability values can be discussed in UE capability session. Proposal 2: Why do we need this proposal? It is not clear what the UE can do with the cell-level synchronization. So why does it matter if the UE can support 0,1, or 100 candidate cells? Proposal 3: TCI state indication is done via DCI, and there is no DCI involved here. [Mod] Proposal is about timing, not the signaling part. I added an FFS for signaling in v3. Proposal 5: Does this mean there is no capability involved? Otherwise, we already have this agreement. |

[FL Observation]

In general companies seem to agree with the 2-step DL sync procedure in proposal 1. Main open items remaining is on the step 2, e.g., whether it applies to all scenarios or how the TCI activation would be done. FL tried to capture the open items through newly added FFSs. DL Sync is an high priority item so its vital to have at least some high level agreement on the procedure.

Remaining proposals are quite stable with some terminology/editorial update requests from concerned companies

[FL Proposal 5-5-1-v3]

FL Proposal 1: For DL synchronization timing when Rel-17 unified TCI is used for LTM beam indication:

- Two-step timing DL synchronization procedure is defined:

  - Step 1: UE maintains DL synchronization (to find frame boundary and for TA management) with SSB [after L1 measurement]

  - Total number of SSBs and candidate cells in step 1 is a UE capability

      - Details of the UE capability will be handled during UE capability session

    - FFS: how and when candidate cells and SSB in candidate cell are determined

    - FFS: Timing of Step 1 synchronization

      - E.g., after L1 measurement

    - Step 1 does not imply UE will derive QCL-TypeA properties from the SSBs used for L1 measurement.

  - Step 2: UE starts DL synchronization for PDSCH/PDCCH reception with the QCL source of the activated TCI states

    - Total number activated TCI states and/or QCL sources used in Step 2 is a UE capability

      - Details of the UE capability will be handled during UE capability session

    - FFS: scenarios where Step 2 is applicable

  - FFS: how the TCI state(s) are activated

  - FFS: QCL source and QCL type details.

  - FFS: When DL synchronization for PDSCH/PDCCH reception is complete

FL Proposal 2: For R-18 LTM introduce a mechanism to enable the UE to select a subset of the SSB used in Step 1 of DL synchronization timing information.

- Alt 1: SSB subset selection is done explicitly by gNB

  - FFS details of the signalling, whether subset selection is done at RRC or MAC CE level

- Alt 2: SSB subset selection is performed by the UE

  - UE would store/maintain DL synchronization timing information for the N strongest SSBs (based on UE capability) based on the measurement results

- Alt 3: UE would store/maintain DL synchronization timing information for the SSB(s) given in the PDCCH orders for early TA acquisition

FL Proposal 3: For the Rel-17 unified TCI based beam indication in Rel-18 LTM:

- Alt 1: TCI state activation of the target cell is done before TCI state beam indication of the target cell,

  - FFS: signalling details for TCI state activation

- Alt 2: TCI state activation of target cell is done together with TCI state beam indication of the target cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FL Proposal 4: UE is expected to perform Step 1 of DL synchronization at least based on SSB, before TA operation.

[Comments to FL Proposal 5-5-1-v3]

| Company | Comment |
| --- | --- |
| Nokia | Support all the proposals in general; We may not need proposal 4, but if majority wants to have this as an agreement, we are fine. |
| ZTE | Proposal 1: For Step 2, we suggest to add a FFS: what is QCL source and QCL type. [Mod] done Proposal 2: To avoid confusion, “ to enable the UE” can be removed from the main sentence, such wording seems to imply RS subset selection is done by UE. [Mod] please see modified proposal Proposal 3: For alt2, we are quite confused about whether the existing spec can support such behavior that activating a set of TCI state simultaneously with indicating a TCI state from the activated TCI state set. |
| LG | Regarding proposal 2, the main sentence can be modified since the selection for subset of SSB is performed by gNB or UE depending on alts. [Mod] please see modified proposal |
| Panasonic | For Proposal 2: We are now confused. We thought that it is intended for beam level synchronization, i.e. a subset of SSB beams can either be selected by gNB or by UE in order to be ready for PDCCH/PDSCH reception. If it is intended for cell level synchronization as currently proposed, we have the same question as Ericsson: UE behaviour is unclear with the synchronized cells. If UE is not required to maintain beam level synchronization with the selected SSBs, why we need to introduce a mechanism to select the SSBs? [Mod] As discussed during online session, even for step 1 a UE can’t be expected to maintain DL sync with all the SSBs in a cell, e..g, if the cell has 64 SSBs, a UE can perhaps support maintaining DL sync with X SSBs where X<64. This sub selection mechanism is for the case where UE supported number of SSBs in Step 1 is less than total number of SSB in the candidate cells. For other proposals, we are fine. |
| Huawei, HiSilicon | For proposal 1, the meaning of “cell level” is still not clear. To my understanding, the DL sync in Step 1 is used for PRACH transmission to candidate cell. As SSB index will be indicated, then the symbol boundary should be that corresponding to the indicated SSB instead of earliest path of all SSBs in the cell. I think it should be up to UE implementation. we suggest modify the main bullet as UE maintains DL synchronization (cell level synchronization) with SSB can be used for TA operation [Mod] please see updated proposal. It also mentions step 1 is not QCL type A. then can it be others, like QCL type-C. For proposal 2, we have concern on the “cell level” similar as proposal 1. The rest are fine. Support proposal 3 and 4. If our changes on proposal 1 is acceptable, the proposal 4 seems not necessary anymore. |
| Xiaomi | Proposal 3: We are confusing about the wording, before beam indication, in both alt.1 and alt.2. Is it beam indication of target cell or candidate cell? In the two steps of DL synchronization, there is TCI states activation of candidate cell but no beam indication of candidate cell. So, it is target cell. However, the beam of target cell is indicated in cell switch command. Then, for alt.1, step2 is performed before cell switch command and for alt.2, step2 is performed after cell switch command. Apparently, alt.2 is not feasible because we are talking about DL synchronization for LTM before reception of cell switch command. In short, it should be clarified that “beam indication” in alt.1 and alt.2 is beam indication of target cell, or candidate cell. [Mod] please check latest proposal wording |
| Nokia | Support all the proposals in general; We may not need proposal 4, but if majority wants to have this as an agreement, we are fine. |
| CMCC3 | Thanks for FL’s update. For the proposal 1, we have to repeat our comments in the last version. We do not have any cell level synchronization or beam level synchronization in RAN1. what we have is just cell level measurement or beam level measurement which is mentioned in 38.300. What we have are only coarse synchronization through PSS and SSS, and finer synchronization or T/F tracking through TRS which is still under discussion. So we propose to remove the undefined concepts of cell level sync and beam level sync. For any SSB measured or synchronized by UE will be send though a beam. Then there is no beam level or cell level synchronization. They are all beam level synchronization. The difference between step 1 and 2 is whether the TCI states will be used. For this part, it is clearly stated in the step 2. Then we do NOT need the new concepts of the beam level sync or cell level sync. [Mod] please check the updated proposal. If those two concepts are just to facilitate the discussion, explicitly use step 1 and step 2 is acceptable. Secondly, I think the main behaviour of step 1 also contains L1 measurement. If there is no need to do the measurement, we do not think there is a need to split step 1 and step 2. Since step 1 only contains sync through SSB, and step 2 is also sync through SSB and then activate the TCI states. Then step 1 and the first part the step 2 is the same. If step 1 contains a measurement or L1 measurements, it should be mentioned clearly. Also we have proposed a update for step 1 as below, - Step 1: UE maintains DL synchronization (cell level time/frequency synchronization) with SSB and take L1 measurement. If there is any mis-understanding, please correct me. [Mod] During online discussion few companies mentioned they don’t want to tie down step 1 to L1 measurement timing, since same synchronization can be achieved via L3 measurement. For clarification I added FFS on the timing which step 1 is expected to be completed, please check updated proposal. We finalize this aspect once we agree on the framework. For the proposal 2 and 3, we also do not need the cell level. We can use Step 1 synchronization instead. |
| IDCC | Fine with the proposals. |
| Samsung2 | As mention before for proposal 1: For step 2 of proposal 1, if the intention is to only have TCI states for one candidate cell activated (in addition to those of the serving cell). In this case, we don’t see a need for “Total number beam level synchronization used in Step 2 is a UE capability”. If the intention is to activate TCI states for more than one cell, we should first discuss if this is supported and what are the use cases, when this would be applicable. Therefore, we prefer to remove “Total number beam level synchronization used in Step 2 is a UE capability” [Mod] Updated the UE capability wording for Step 1 and 2 please check. Since many companies want to keep the UE capability sub-bullet in step 2 I will keep it we can check anyone else also has concern with keeping it. In step 2, timing of when the synchronization would be done is also FFS. Please add timing FFS: “When DL synchronization for PDSCH/PDCCH reception is complete” [Mod] Done. Step 2 (i.e., synchronization for PDxCH) is something that UE needs to perform to receive PDxCH. However, the details are up to the UEs implementation. The key aspect for the network is that before a DL/Joint TCI state is indicated to the UE, the UE should have achieved synchronization and be ready to receive the PDxCH channel. This can be achieved by having a large enough time between the activation of TCI state and indication of TCI states for candidate cell. Please add. “FFS: Spec impact” [Mod] we can assess the Spec impact once we resolve the FFS items. Fine with proposal 2 to discuss the alternatives. Support Alt2. I assume that down selection will be in RAN1#113. Fine with proposal 3 to discuss the alternatives. I assume that down selection will be in RAN1#113. We would like add note, different alternatives can be supported for scenario 2 and scenario 1, if supported. Don’t see a need for proposal 4, this can be left to the UE’s implementation |
| CATT | For proposal 1: support For proposal 2: Alt 3 seems not reasonable since the storage of DL synchronization should be performed before PDCCH orders. For proposal 3: we prefer Alt 1. For Alt 2, is the correct understanding that TCI state activation would be included in the cell switch command? For proposal 4: support |
| FGI | For Proposal 1: Support For Proposal 2: We are fine with FL’s proposal. In addition, we slightly prefer Alt 2. For Proposal 3: Support. We prefer Alt3, but Alt.1 is also acceptable to us. For Proposal 4: Support. |
| Lenovo | For proposal 1: - We are fine to leave the first subbullet of step 1 to UE capability session. However, the wording “Total number of cell level synchronization” is confused to us. Does it mean the number of candidate cells for the UE to perform DL synchronization? [Mod] Please see updated proposal. - Does step 2 imply that TCI states activation is always needed for a candidate cell before the cell switching command? |
| Futurewei | For Proposal 1, unless we agree that CSI-RS can be used for measurement of candidate cell(s), we do NOT see the benefits of Two-step timing DL synchronization procedure, because only SSB(s) can be used for DL synchronization. For Proposal 2, we are ok with Alt 2, because it is an implementation issue. We are ok with Proposal 3, and we prefer Alt 2. We are not ok with Proposal 4, because of the term “Step 1 of”, and we suggest cancelling it as two steps of synchronization is not agreed yet. |
| NTT DOCOMO | We suggest following revision on the FFS. Because we think it is clear the procedure is complete before cell switch command, but the unclear point is when to start the step 2 DL synchronization. - FFS: When Timing of DL synchronization for PDSCH/PDCCH reception is complete |
| MediaTek | Thanks to FL’s latest proposal. As we commented online, we are not sure what information need to be maintained by (to find frame boundary and for TA management) and we are confused on the necessity of Step 1. Based on the discussion, we are afraid that companies still have different understandings on step 1. On the other hand, we are not sure what is the impact if we only specify step 2 without step 1 before cell switch? UE only needs to perform L1 measurement and gNB will inform UE which beam to use for sync in order for DL reception (e.g., TCI activation). We are not sure what is missing between L1 measurement and TCI activation. Also, not sure why step 1 can help handover latency reduction. [Mod] we already have agreement that at least SSB should be used before cell switch command are you suggesting we should revert the agreement? Regarding the missing part between L1 measurement and TCI activation currently based on the specification is that UE is not to maintain any Sync related parameters based on SSB configured for L1 measurement, it simply measures and reports the RSRP/SINR and that’s all. However, by introducing this 2-step Sync we are asking UE to not only measure RSRP but also maintain Sync information about each SSB used in step 1. Agreement - Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command - Further study the necessary mechanism, e.g. signaling and UE capability Therefore, we suggest to remove step1 and only capture step 2 as one UE behavior before cell switch. In that case, whether we need Proposal 2 needs further discussion. |
| LG | Regarding step 1, it is still confusing that the SSBs for DL sync are the same or different used for L1 measurement? [Mod] It seems majority of the companies assume it’s the SSBs used for L1 measurements. I keep it in [], we can discuss. |
| ZTE2 | Thanks to FL for considering our proposals. For step 1 in proposal 1, it is not clear for us why Coarse DL sync of first step will be used for TA management? |
| Huawei, HiSilicon2 | For proposal 1, step2, as TCI state is agreed in last GTW for beam indication, I am wondering the procedure of TCI state activation already implies the DL sync for data reception. Current version reads like after TCI state activation, there is still a procedure for DL sync. [Mod] TCI activation and indication are different. UE will need to perform some DL sync after activation, since it be informed about QCL source that need to be tracked once it gets the activation command, but it won’t need any extra DL sync after indication, since it has already done so during the activation. Step 2 is about UE behaviour once TCI is activated. For proposal 3, maybe we can change TCI state back considering the latest agreement achieved. |

[FL observation after Teams offline on Thu]

Based on the offline discussion there is no consensus on 1-step vs 2-step DL Synchronization procedures. Companies are now divided in the middle on whether 2-steps are needed or not. Given the situation, FL observation is as following:

Conclusion: There is no consensus to define details on DL synchronization for R18 LTM. There will be no specification enhancements at least for DL Synchronization procedure in R18 LTM aiming to reduce handover delay / interruption.

[Comments on FL observation after Teams offline on Thu]

| Company | Comment |
| --- | --- |
| Panasonic | After hearing all the comments in yesterday’s offline, maybe we can focus on Step 2 only as a way forward. The main concern for deleting step 1 that I heard in offline discussion is that step 1 is a first necessary step in order to step 2 to work. I don’t deny that. However, this is already reflected by previous agreement as follows: Agreement - Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command - Further study the necessary mechanism, e.g. signaling and UE capability There seems nothing new we can achieve by agreeing on step 1, compared to previous agreement. Therefore, I suggest we take a small step to agree on Step 2 first, then in future meetings proponents of Step 1 can bring up refined version of Step 1. Some minor changes of wording are suggested as follows, if we consider Step 2 only. FL Proposal 1: For DL synchronization timing when Rel-17 unified TCI is used for LTM beam indication: - Two-step timing DL synchronization procedure is defined: - Step 2: UE starts maintains DL synchronization for PDSCH/PDCCH reception with the QCL source of the activated TCI states in one or more candidate cells - Total number activated TCI states and/or QCL sources used in Step 2 is a UE capability - Details of the UE capability will be handled during UE capability session - FFS: scenarios where Step 2 the activation is applicable - FFS: how the TCI state(s) are activated - FFS: QCL source and QCL type details. - FFS: When DL synchronization for PDSCH/PDCCH reception is complete |
| FGI | We don’t support the proposed conclusion. Although there is no consensus on 1-step vs 2-step DL Synchronization procedures, we can still discuss the enhancement of early DL synchronization (i.e., performing DL synchronization for multiple candidate target cell(s) before the cell switch command). Perhaps the proposals related to Step 1 (e.g., how to select a subset of the SSB) can be considered for further discussion regarding the previous agreement as follows: Agreement - Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command - Further study the necessary mechanism, e.g. signaling and UE capability |
| CMCC5 | We cannot support the conclusion after multiple rounds of discussion on this proposal. The issue here is that the discussion did not clarify what has happened before the two/single step synchronization. For the agreements mentioned by Panasonic, in our view for most time, it is a function level agreement. And under this framework, the group are talking about the details and how to realize the functions. From our understanding, before we discuss the single/two steps synchronization, the group should have a common understanding on what behaviour the UE has performed. |
| Nokia | We should not make the conclusion on proposal 1 of no consensus. We can come back to this if needed in the next meeting or for example, if we agree to support CSI-RS. Also, we think the more important proposal in this section is TCI activation (proposal 3). We have to make decision on that. Proposal 3 can be discussed independent of the proposals 1/2/4. |
| ZTE | Do not support the conclusion and support updated proposal by Panasonic |
| Samsung3 | Support conclusion |
| QC | Not support. We think at least Step 2 significantly saves TCI activation latency and have spec impact. Agree Step 1 has no spec impact |
| Futurewei | IF step 2 needs to be kept, we suggest revising is as below, - Two-step timing DL synchronization procedure is defined: - Step 1: UE maintains DL synchronization (to find frame boundary and for TA management) with SSB [after L1 measurement] - Total number of SSBs and candidate cells in step 1 is a UE capability - Details of the UE capability will be handled during UE capability session - FFS: how and when candidate cells and SSB in candidate cell are determined - FFS: Timing of Step 1 synchronization - E.g., after L1 measurement - Step 1 does not imply UE will derive QCL-TypeA properties from the SSBs used for L1 measurement. - Step 2: UE starts DL synchronization for PDSCH/PDCCH reception with TRS in the QCL source of the activated TCI states - Total number activated TCI states and/or QCL sources used in Step 2 is a UE capability - Details of the UE capability will be handled during UE capability session - FFS: scenarios where Step 2 is applicable - FFS: how the TCI state(s) are activated - FFS: QCL source and QCL type details. - FFS: When DL synchronization for PDSCH/PDCCH reception is complete |
| MediaTek | We don’t support the proposal conclusion. Based on offline discussion, we have a feeling that we can’t achieve consensus on what is the definitional of DL sync and what is associated UE implementation on DL sync. As mentioned by MTK, Apple, and Ericsson during offline discussion, not all of the details can have spec impact. However, it seems like companies are clear that activating TCI state before beam indication carried in the cell switch command is beneficial for handover latency reduction. In that sense, we suggest formulate the proposal as following: Proposal: For scenarios 2, activation of TCI state (s) before the beam indication in cell switch command is supported. - Total number activated TCI states and/or QCL sources is a UE capability - Details of the UE capability will be handled during UE capability session FFS: how the TCI state(s) are activated FFS: QCL source and QCL type details of the activated TCI states FFS: activation of TCI state (s) for other scenarios |

[Low] Other procedures to reduce handover latency/interruption time

[Conclusion at RAN1#110b-e]

Agreement

- RAN1 to further study the potential RAN1 enhancements and spec impact to perform at least the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay / interruption

  - DL synchronization for candidate cell(s)

  - TRS tracking for candidate cell(s)

  - CSI acquisition for candidate cell(s)

  - Activation/Selection of TCI states for candidate cell(s), if feasible

  - Note: Uplink synchronization aspect will not be discussed under this A.I.

  - FFS: Whether the above procedures prior to the reception of L1/L2 cell switch command can be performed on candidate cell when it is deactivated SCell (if defined in RAN2)

- Detailed discussion will be commenced after receiving RAN2 LS.

[Conclusion at RAN1#111]

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

[Conclusion at RAN1#112]

The following proposal was not treated due to the lack of time.

- TRS tracking for candidate cells before the reception of cell switch command is supported

- CSI acquisition for candidate before reception of cell switch command is supported

FL note: before going to the proposal, it is suggested checking the companies’ view on the criticality of this functionality for LTM. FL’s understanding is that these techniques requires non-negligible spec impact (especially for CSI acquisition), and hence this topic is handled on best effort basis.

[Summary of contributions]

Number of contributions did consider the topic of triggering CSI acquisition and TRS tracking prior to cell switch command.  The companies were divided in opinion on whether these two mechanisms should be introduced part of Rel-18 LTM. Companies in favor of enabling these features discussed how TRS tracking for candidate cell(s) is necessary to get QCL-typeA information for demodulation and the benefits in terms of ensuring quality of PDxCH reception specially in scenarios where frequent handovers are likely to take place. In addition, the benefits of CSI acquisition for candidate cells were discussed where it will allow enabling appropriate MCS choice based on the UE reported CQI.

The proponents of these enhancements discussed how the same similar latency reduction can be achieved by having CSI acquisition and tracking triggering through cell switch command and furthermore raised concern on the workload introducing these procedures will lead to.

[FL Proposal 5-5-2-v1]

Proposal 1: There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM

- TRS tracking for candidate cells before the reception of cell switch command

- CSI acquisition for candidate before reception of cell switch command

[Comments to FL Proposal 5-5-2-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We need to clarify that, at least for the candidate cells which are current serving cells, the two procedures prior to cell switch command have been supported. |
| Ericsson | OK. This would mean that the UE can receive PDCCC/PDSCH based on QCL properties derived from SSB? |
| Futurewei | We are ok with FL proposal if candidate cells are active. |
| QC | Not support. Without those early preparations, wonder why we need LTM at all. Early switch with non-optimized performance may not have benefit compared with even L3 HO, since the old cell with refined beam can be better. We think early switch with optimized performance should not be excluded in LTM. At least TRS tracking can be considered, e.g. via TCI activation. Note: part of functions of TCI activation is for TRS tracking in DL, where TRS is signalled in the TCI |
| Apple | OK. On DCM comment, as mentioned in main bullet, this is limited ‘aiming at the reduction of handover delay/interruption in Rel-18 LTM’. Therefore, for the current serving cell with TRS tracking and CSI acquisition, they are not intended by this proposal. It should be fine. On SSB-based switch, our view that cell-switch is a ‘temporary’ period and finer beam refinement can be performed after cell switch as in legacy. For this transit period, SSB-based rough sync is sufficient to minimize the HO latency. |
| MediaTek | Sure the same view with Apple. However, we do need to discuss Ericsson’s question: UE can receive PDCCC/PDSCH based on QCL properties derived from SSB? |
| ZTE | We don’t agree with this proposal. At least for TRS tracking, we think that it should be done before cell switch command since TRS is a reference RS with QCL Type A for DMRS of PDSCH/PDCCH for target cell. If TRS is not supported before cell switch command, we would like to know if there is a TRS tracking operation after cell switch command and before data transmission. |
| Fujitsu | We support the proposal |
| vivo | Not support. We share similar views with QC that at least early TRS tracking should be considered for LTM to improve data transmission performance. |
| Huawei, HiSilicon | We should discuss the support of CSI-RS at first. |
| Samsung | OK |

[FL Proposal 5-5-1-v2]

Proposal 1: There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM

- TRS tracking for candidate cells before the reception of cell switch command

- CSI acquisition for candidate before reception of cell switch command

Note: At least for the candidate cells which are current serving cells, the CSI acquisition prior to cell switch command will be supported

[Comments on FL Proposal 5-5-1-v2]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | OK. |
| Fujitsu | A minor comment is that “the two procedures” under “Note” is not aligned with the proposal, since one procedure has been deleted in the proposal. [Mod] Done |
| CMCC | Should we split this proposal into two? One is to support both procedures (CSI early acquisition and TRS tracking) when the candidate cells are current serving cells The other one is in other cases, CSI early acquisition are not supported. [Mod] Done |
| LG | Fine with the proposal |

[FL Observation]

This section seems quite stables. As proposed by one company in v3 FL has proposed a two separate proposal to capture TRS and CSI acquisition separately. Hopefully this would allow for smoother resolution.

[FL Proposal 5-5-1-v3]

Proposal 1:

At least when the candidate cells are current serving cells, the following procedures are supported, prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM

- CSI acquisition for candidate before reception of cell switch command

- TRS tracking for candidate cells before the reception of cell switch command

Proposal 2:

There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM, except that the candidate cells are current serving cells

- CSI acquisition for candidate before reception of cell switch command

[Comments on FL Proposal 5-5-1-v3]

| Company | Comment |
| --- | --- |
| ZTE | For Proposal 2, we understand that it is related to step 2 in DL sync procedure. If rocedure is done before cell switch command, while step 2 will be done after cell switch command if step 2 is required in DL sync procedure. Considering that it is related to other proposals and related issues have not been clarified, it is not recommended to have such a conclusion so early. |
| NTT DOCOMO | Support Proposal 1. Postpone the conclusion of Proposal 2. |
| CMCC3 | Sorry if I did not express clearly. My intention is that, if the group can accept to support - CSI acquisition for candidate before reception of cell switch command - TRS tracking for candidate cells before the reception of cell switch command under the situation that the candidate cells are current serving cells. We can reformulate it as below, Proposal 1a: At least when the candidate cells are current serving cells, the following procedures are supported, prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM - CSI acquisition for candidate before reception of cell switch command - TRS tracking for candidate cells before the reception of cell switch command Proposal 2a: Proposal 1: There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM, except that the candidate cells are current serving cells - CSI acquisition for candidate before reception of cell switch command Sorry for the confusion. [Mod] Thank you for clarification. |
| MediaTek | For proposal 1, if we replace candidate cell with current serving cell, the the whole proposal is to describe legacy behavior? We are not sure why should we capture it? |
| Samsung3 | The need for proposal 1 is not clear. If the candidate cells are current serving cells, wouldn’t the UE already ben CSI acquisition and TRS tracking for these cells? What else does the UE need to do. |
| QC | For Proposal 1: Support For Proposal 2: Fine |

Cross A.I. issue

[High] L1 LTM procedures including TA

[Summary of contributions]

Apple proposed to combine two UE procedures (candidate cell RACH by PDCCH and TCI state activation) order to reduce the latency. The validity depends on the order of the procedures.

  - Using the SSB(s) indicated in PDCCH order DCI for both TCI-state mapping and activation as well as the CFRA procedure (i.e., Alt.3)

Meanwhile, MediaTek proposed the interaction between TCI state activation and beam indication. FL understands the presence of RACH procedure is not considered in this proposal.

  - TCI state indicated in the cell switch command is in the activated TCI state lists before cell switch and in the activated TCI state lists after cell switch.

Furthermore, Nokia proposed to consider UL synchronization status for beam selection on measurement report, which (in FL’s understanding) assumes that candidate cell RACH is performed in early stage of overall L1 procedure.

  - In a reporting instance, beams from the synched candidate cells (for which the valid TA information is available) are prioritized over the non-synced candidate cells (for which the valid TA information is not available).

Given the proposals above, FL thinks the necessity to discuss the overall L1 procedures for LTM to avoid the misunderstanding among the companies.

A possible overall L1 procedure is depicted in the figure below. FL thinks this is the most likely procedure, but is not quite sure if this is a common understanding. Also, FL agrees that different interpretation is also possible because RAN1 had no such discussion so far.

Based on the background above, FL would like to suggest an email discussion to achieve a common understanding on:

  - Timing relationship among the L1 procedures, i.e.

    - L1 measurement

    - DL synchronization for frame boundary identification, which is maintained after L1 measurement (under discussion)

    - Reception of candidate cell PDCCH order, and Msg.1 Tx followed by RAR reception (if configured)

    - TCI state activation (under discussion)

  - Relationship among candidate cells, i.e.

    - Candidate cell associated with L1 measurement RS

    - RACH candidate cell

    - Candidate cells associated with activated TCI sates

    - Candidate cells associated with indicated beam

[FL Proposal 5-6-1-v1]

Potential Conclusion

  - For a candidate cell, RAN1 assumes following timing relationship for the UE procedures of Rel-18 LTM

    - L1 measurement and report is performed before candidate cell RACH

    - L1 measurement and report is performed before TCI state activation

    - TCI state activation is performed before or together with candidate cell RACH (FFS details)

    - Beam indication is performed after candidate cell RACH

    - (FL note: timing relationship between candidate cell RACH and DL synchronization)

  - For candidate cells and target SpCell, RAN1 assumes following relationship for the UE procedures for Rel-18 LTM

    - Candidate cells associated with activated TCI states shall be a subset of candidate cells which DL synchronization is maintained by a UE

    - Candidate cell RACH shall be performed to the beams of candidate cells associated with activated TCI states

    - Indicated beam of a target cell shall be a subset of beams of candidate cells where candidate cell RACH is performed.

Companies are encouraged to provide their view, especially if they have a different understanding on the overall procedure.

[Comments to FL Proposal 5-6-1-v1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We have several comments. First, ‘candidate cell RACH’ may be or may be not performed, depending on whether the TA of the candidate cell is known by NW/UE or not by other methods. Second, if ‘candidate cell RACH’ is needed, the time relationship between TCI state activation and candidate cell RACH seems flexible. Thus, the candidate cells set for TCI state activation and the candidate cells set for candidate cell PRACH seems also flexible. |
| Ericsson | OK. |
| Futurewei | We think the timing relationship among L1 measurement and report, candidate cell RACH, TCI state activation should be more flexible, even after candidate cell RACH and TCI state activation, L1 measurement and report is still needed for serving cell. |
| QC | Some bullets in the potential conclusion can only be examples. To our understanding, TCI activation/indication can happen before, together, or after PRACH. They don’t have strong dependency in time order. But if we say those are just implementation examples in the main bullet, then it should be fine - TCI state activation is performed before or together with candidate cell RACH (FFS details) - Beam indication is performed after candidate cell RACH |
| Apple | Support. On the timing of ‘’candidate cell RACH’ and ‘TCI-state activation’, our understanding is that DL timing is needed for PRACH transmission in UL sync. In this sense, TCI-state activation for DL sync should be performed at least not late than RACH procedure. This is also assumed in legacy handover operation and initial access e.g., first DL sync and then followed by UL sync operation. |
| Nokia | We agree that L1 meas and reporting should be performed to enable RACH and TCI activation. But note that L1 meas and reporting may go on continuously even during and after the RACH and TCI activation. Not sure why TCI activation needs to be done before/together with the RACH? It can be done after the RACH as well. If that is true, then the second bullet needs to be updated accordingly. |
| ZTE | For 1st bullet: for 3rd sub-bullet, we have same question with Nokia. For fourth sub-bullet, it is related to scenario. For scenario 2, we think that RACH for candidate cell is done before cell switch command and beam indication is included in cell switch command. While for scenario 1, since such scenario has not been supported, so we don’t expect to mix it with related discussion together. For 2nd bullet, for 2nd and 3rd sub-bullet, it is related to the clarification on relationship between activation of TCI states and PRACH operation. |
| Lenovo | For the first bullet: - We prefer the following wording for the first sub-bullet: - Candidate cell RACH, if configured, is expect to be ordered after at least one L1 measurement and report. - TCI state activation should only depend on the L1 measurement; therefore, the last three sub-bullets are unnecessary for the first bullet. For the second bullet: we fail to see the motivation of the last two bullets. |
| Fujitsu | We don’t think TCI state activation has to be performed before candidate cell RACH. In a legacy RACH procedure, there seems to be no such restriction. Besides, it is suggested to add the following blue part into the second bullet. - For candidate cells and target SpCell, RAN1 assumes following relationship for the UE procedures for Rel-18 LTM - … - Indicated beam of a target cell shall be a subset of beams of candidate cells where candidate cell RACH is performed and where DL synchronization is maintained by a UE. |
| vivo | We are fine with the L1 measurement and report being performed before RACH and TCI activation. But we are also confused about why TCI activation needs to be done before/together with the RACH. |
| Xiaomi | For first bullet, we believe the restriction in first sub-bullet is enough. |
| CMCC | In general we are fine. For the 3rd sub-bullet of the 1st bullet, we do not see the strong relationship between TCI state activation and the cell candidate cell RACH. With the DL synchronization and the L1 measurement results, serving cell can trigger a PDCCH ordered RACH to the measured candidate cells for TA no matter if the TCI states is activated or not. But both TCI activation and candidate cell RACH should after the DL sync and L1 measurement and before the beam indication. For the 2nd sub-bullet of the 2nd bullet, it seems that we do not have the conclusion require that the Candidate cell RACH should be based on the activated TCI states. |
| Huawei, HiSilicon | For the first main bullet, we are not sure the objective of the discussion on this topic. Is it going to input to RAN2 on their running CR for 38.300. In general, the time relation is quite related to the usage scenario. If the UE is in the deployment scenario similar as R17 ICBM, i.e. sync and intra-frequency, the RACH procedure seems not necessary. For the 1st sub-bullet, the procedure of L1 measurement and DL/UL sync are mixed, the early result can be updated by the later measurement. For the 3rd and 4th sub-bullets, we are not sure why TCI state or beam indication should be performed early than RACH. In the current TCI state for UL, only spatial relation is provided. To our understanding, TCI state should be activated before beam is indicated. The other can be left for implementation. The 2nd main bullet and sub-bullets under it looks fine to us. |
| OPPO | Generally, we are not ok with the proposed conclusion and we do not think it is needed. For the 3rd sub-bullet: the TCI state activation is not related with RACH to candidate cell. Why such a timing relationship shall be assumed? The same problem for 4th sub-bullet. The three sub-bullets under the 2nd bullet: they are not something that shall be discussed for specification. |
| Samsung | We don’t see a strong need for this conclusion. These timing aspects can be left to implementation. For example, if the network wants to perform a PDCCH order triggered RACH before getting L1 measurements, for any reason, this can be allowed. As this has no spec impact, we don’t see a strong need to discuss. |
| CATT | For the first main bullet, we think it's necessary to discuss the timing relationship for the UE procedures and we support currently procedures. We are fine with the 2nd main bullet and sub-bullets. |

[FL Observation]

For L1 measurement, it is pointed out that measurement and reporting is performed continuously, so it may happen even after DL sync, TCI state activation and candidate cell RACH, which FL agrees.

The issue on candidate cell RACH was pointed out by many companies:

  - Candidate cell RACH may not be performed, e.g. in sync cells

  - Candidate cell RACH is an independent procedure from TCI state activation

    - Order of candidate cell RACH may be before and after TCI state activation

    - Meanwhile, DL sync (one company mentioned that this is TCI state activation) should be done before candidate cell RACH

While having one common understanding can ease our future discussion, FL is OK to leave the timing of candidate cell RACH open.

Also, it was also concerned that the restriction in the second main bullet (i.e. relationship between DL synced cell, cell where TCI state is activated and cells where RACH is perform). It is noted, again, that this issue is brought up by a company to have such a restriction.

Finally, it was pointed out that any agreement related to this discussion is necessary. FL is OK to conclude this discussion without any official agreement as companies assumption was clarified with this discussion.

The companies’ views are summarized as follows.

For a candidate cell, RAN1 assumes following timing relationship for the UE procedures of Rel-18 LTM with scenario 2 (i.e. Beam indication together with cell switch command)

Summary of the discussion in this section

  - It is assumed that L1 measurement and report shall be performed at least before candidate cell RACH (if performed) and TCI state activation (if agreed), and may go on continuously irrespective of candidate cell RACH and TCI state activation.

  - Many companies viewed that candidate cell RACH is independent from TCI state activation (if agreed) and left for implementation, while some see the dependency

    - TCI state activation is performed before, together with, or after candidate cell RACH (if performed) – further discussion on down-selection will be based on companies’ input

  - It is assumed that beam indication is performed after candidate cell RACH if candidate cell RACH is required. It is noted that beam indication scenario 1 and 3 are not considered in this discussion.

  - Some companies are OK to consider some dependencies among candidate/target cells/beams and TCI state e.g. below. However, there is no consensus among the companies as of this meeting.

    - Candidate cells associated with activated TCI states and candidate cells which DL synchronization is maintained by a UE

    - Candidate cell where candidate cell RACH is performed and beams of candidate cells associated with activated TCI states

    - Indicated beam of a target cell and beams of candidate cells where candidate cell RACH is performed.

  - Timing relationship between candidate cell RACH and DL synchronization is not discussed in this section, and covered under section 5.5.1

With this, the discussion in this section is closed, unless there is a misunderstanding.

[Comment if any]

| Company | Comment |
| --- | --- |
| QC | For the sentence below, is candidate cell RACH for early TA acquisition before cell switch? If so, we think beam indication can happen before RACH, e.g. if Scenario 1 is agreed. It is assumed that beam indication is performed after candidate cell RACH if candidate cell RACH is required. It is noted that beam indication scenario 3 is not considered in this discussion. [FL summary] The point Qualcomm raised was not discussed and not pointed out. Thus, FL applies the following modification. This has also applied to the summary above It is assumed that beam indication is performed after candidate cell RACH if candidate cell RACH is required. It is noted that beam indication scenarios 1 and 3 are not considered in this discussion. |

LS

[Mid] LS to RAN2,3 and 4

[FL observation]

As usual, it would be helpful for RAN2, 3 and 4 to know the RAN1 agreements in RAN1#112bis-e. The final decision will be made at the final session of RAN1#112bis-e.

Also, it is pointed out that the following agreement on Wed GTW needs an explicit message to RAN3 because RAN3 spec impact is foreseen. Therefore, the following proposal is made.

[FL proposal 5-1-8-v1]

- Send an LS to RAN2,3,4 on the RAN1 agreements in this meeting

  - All agreements in AI 9.10.1 and 9.10.2 in RAN1#112bis-e are included

  - The following contents are included in the LS

    - RAN1 has made the following agreement in RAN1#112bis-e

      - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) ,

        - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

        - Alt. 2: By indicating Rel-17 TCI state index

    - This agreement implies that the source cell needs to know the Rel-17 unified TCI states at the target cell(s) and candidate cell(s) (if TCI state activation is supported) even when source cell and target/candidate cell(s) belongs to different DU. RAN1 understands that this agreement has a RAN3 spec impact, and hence RAN1 respectfully asks RAN3 to perform the necessary specification work to support this functionality.

[Comments to FL proposal 5-1-8-v1]

| Company | Comment |
| --- | --- |

Other topics

The following proposals are the topics which FL see the difficulties to categorize. Also, FL doesn’t see the necessity of urgent discussion in this meeting as well as the discussion in RAN1. Companies are encouraged to perform their further analysis until the next meeting.

Subsequent handover

- Huawei: Proposal 18: To achieve the low latency of potential subsequent LTM, at least TA and L1 measurement results can be kept for original serving cell and part of candidate cells configured when UE is associated with original serving cell.

- Nokia: RAN1 to select one or more of the following alternatives on the UE assumption for activated TCI states upon the cell switch.

  - Alt-1: Upon the cell switch to the target cell, UE may consider the TCI states of the target cell activated before the cell switch as valid for intra/inter-cell beam management within the target cell. For all other candidate cells, the TCI states are not considered to be active anymore.

  - Alt-2: Upon the cell switch, UE may retain the all the activated TCI states given before the cell switch.

  - Alt-3: The UE may be configured (e.g., in RRC or in the cell switch command) whether the activated TCI states are to be maintained/retained after the cell switch. The configuration can be cell specific.

LTM completion

- Huawei: UE could be configured with PUSCH/PUCCH resource in the target cell before cell switch command is issued. FFS: beam application, timing, power control etc.

- CATT: Confirmation (i.e. cell switch complete) to the cell switch command is sent to the target cell using the indicated beam by the serving cell.

LTM/beam failure

- Huawei: UE can additionally determine the LTM failure according to L1 measurement of DL RS and success of UL transmission on preconfigured resource in the target cell. UE can recover the link with source cell when LTM failure is detected without waiting for expiration of T304.

- IDC: Support beam failure recovery on resources of non-serving cell.

- Google: Support the UE updates the beam for channels including both dedicated and non-dedicated channels based on the newly reported beam requiring serving cell change after 28+X symbols after the UE receives the BFR response,

  - X is the delay for serving cell change.

HARQ reset

- Qualcomm: To reduce latency/throughput impact due to HARQ process reset, support NW can maintain HARQ continuity after cell switch at least in certain scenarios, e.g. intra-frequency intra-DU cell switch

Further latency reduction

- Qualcomm: In case of inter-DU LTM, study mechanism to reduce the latency for target DU to prepare the indicated beam after the cell switch command is sent

- Annex

  - WID in RP-222332

The detailed objective of this work item is captured below:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

- To specify data forwarding optimizations for CHO including target MCG and target SCG in NR-DC [RAN3].

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

- To specify RRM core requirements for the following, as necessary [RAN4]:

- L1/L2-based inter-cell mobility

- Enhanced CHO configurations addressed by this WI

- To specify RF requirements to cover inter-frequency L1/L2-based mobility, as necessary [RAN4].

- To study the following, with completion targeted by RAN#98 meeting [RAN4]:

- The impact of FR2 RRM mobility measurement acquisition and reporting on FR2 SCell/SCG setup/resume delay for a UE connecting from idle/inactive mode.

- The level of feasible improvement in FR2 SCell/SCG setup delay from defining new UE measurement procedures and RRM core requirements, and whether additional information from the network would help the UE to perform those measurements effectively. The following sequence of events should be assumed.

    - The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

    - After acquiring those improved measurements, the UE subsequently reports those measurements to the network to support SCell/SCG setup.

  - TU allocation

  - Agreements at RAN1#112

Agreement

- RAN1 shares the same understanding as RAN2 on agreement:

  - The LTM mobility trigger information is conveyed in a MAC CE

- The same MAC CE is used for the LTM triggering.

Agreement

- The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

  - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

Agreement

- Send an LS to RAN2,3,4 on the RAN1 agreements in this meeting

  - All agreements in AI 9.12.1 and 9.12.2 in RAN1#112 are included

  - The LS contents agreed in AI 9.12.1 (on L1 measurement configuration) and AI 9.12.2 (on RAR) are also included

Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

Agreement

Draft LS R1-2302193 is endorsed in principle by appending latest agreements.

Agreement

Final LS R1-2302194 is endorsed.

  - Agreements at RAN1#111

Agreement

- For Rel-18 LTM, L1 inter-frequency measurement is supported from RAN1 point of view.

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

  - At least the following aspects may be considered

    - How to define UE event and exact definition of events,

    - Report container

    - Resource allocation/assignment for UE event triggered report

    - Necessity of indication to gNB when the condition UE event is met, and how

    - Necessity to define the condition to start/stop the reporting,

    - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

    - The interaction with filtered L1 measurement results (if supported)

    - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

    - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

    - Benefit when L3 measurement is involved

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

Agreement

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

Agreement (Made in RAN1#110b-e)

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

- Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

  - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

  - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

  - Agreements at RAN1#110b-e

Agreement

- For Rel-18 L1/L2 mobility, L1 intra-frequency measurement for candidate cell is supported

  - At least the following aspects are for RAN1 further study:

    - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

    - Whether and how to apply relaxation for the restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133, where RAN4 impact is foreseen, e.g.

      - SFN offset alignment compared with serving cell

      - BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP

      - Introduction of symbol level gap or SMTC for larger Rx timing difference (i.e. larger than CP length)

    - Commonality with intra-frequency L3 measurement

    - Commonality with L1 inter-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 to ask RAN4 if the restriction on e.g., SFN offset alignment, BWP setting and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

  - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

Agreement

- For Rel-18 L1/L2 mobility,

  - SSB is supported for L1 intra-frequency measurement

  - SSB is supported for L1 inter-frequency measurement if inter-frequency L1 measurements are supported

- Further study the following L1 measurement RS for candidate cell

  - CSI-RS for tracking, beam management, CSI and mobility, CSI-IM, which is for L1 intra-frequency and L1 inter-frequency (if supported)

Agreement

- For candidate cell measurement for Rel-18 L1/L2 mobility,

  - L1-RSRP is supported for intra-frequency candidate cell measurement.

  - Further study the following measurement quantities for candidate cell measurement

    - L1-RSRP for inter-frequency (if supported)

    - L1-SINR for intra-frequency and inter-frequency (if supported)

- FFS: to assess the use case and the benefit of UL measurement instead of/in addition to DL L1 measurement, which includes:

  - How the UL measurement result is used, e.g. handover decision

  - Signals/channels used for UL measurement, e.g. SRS

  - Spec impact including other WGs, e.g. definition of gNB measurement, interface to transfer RS configuration or measurement results

  - Note: The next discussion will take place based on companies’ contribution in future meeting.

Agreement

- For Rel-18 L1/L2 mobility, further study the potential RAN1 spec impact of L1 inter-frequency measurement

  - The definition and scenarios of L1 inter-frequency measurement is determined by RAN4, and RAN1 assumes at least the following until receiving their confirmation

    - The scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

      - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

      - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - At least the following aspect is studied:

    - Commonality with L1 intra-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 would like to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

    - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

    - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - It is RAN1 understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue

  - Note: this content is included in the LS agreed for intra-frequency L1 measurement

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, further study the following mechanisms:

  - Report as UCI on PUCCH or PUSCH

    - Periodic report on PUCCH, semi-persistent report on PUCCH/PUSCH, and aperiodic report on PUSCH

    - Potential enhancements to Rel-17 ICBM report format to accommodate Rel-18 scenarios, e.g.

      - Inter-frequency measurement, if supported

      - Increasing the maximum number of reported beams, which is 4 for Rel-17 ICBM

      - Flexible size beam report, e.g., two-part UCI (e.g., the 1st part contains the best beam/cell and the number (e.g., N) of reported beams/cells, the 2nd part contains the rest (N-1) beams/cells

      - Reducing the reporting overhead by e.g. choosing beams/cells per frequency or across frequencies to report (FFS how)

  - Report on MAC CE

    - Both gNB scheduled and/or UE initiated (if supported) report are studied

Agreement

- RAN1 to further study if the beam indication of candidate cell(s) L1/L2 mobility should be designed for a specific TCI framework below, and their potential RAN1 spec impact.

  - Option A:  Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-17 TCI framework mechanism

  - Option B: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-15 TCI framework mechanism

  - Option C: Beam indication for Rel-18 L1/L2 mobility is designed based on both Rel-15 and Rel-17 TCI framework mechanisms

- Agreement

-  Send an LS to RAN2/RAN3 asking the clarification on intra-/inter-DU scenario:

-      RAN1 has started the discussion on the configuration for L1 measurement and TCI states for candidate cells. Regarding the following RAN2 agreements captured in RAN2 LS (R1-2208331/R2-2209257), it is not clear for RAN1 which kind of information/configuration for candidate cell(s) are available at a serving cell for inter-DU case for Rel-18 L1/L2 mobility. Thus, companies have different understanding on the implication of the sentence “as much commonality as reasonable” in the LS.

-      The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

- 	RAN1 respectfully asks RAN2 and RAN3 if the serving DU knows the measurement RS configuration and TCI state configuration of cells served by another DU

- Agreement

- Send an LS to RAN2, 3 and 4 to inform them of the agreements under A.I 9.12.1 and A.I. 9.12.2

- If the LS related proposal under A.I 9.12.1 and 9.12.2 are agreed, the contents are also included.

Agreement

- RAN1 to further study the potential RAN1 enhancements and spec impact to perform at least the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay / interruption

  - DL synchronization for candidate cell(s)

  - TRS tracking for candidate cell(s)

  - CSI acquisition for candidate cell(s)

  - Activation/Selection of TCI states for candidate cell(s), if feasible

  - Note: Uplink synchronization aspect will not be discussed under this A.I.

  - FFS: Whether the above procedures prior to the reception of L1/L2 cell switch command can be performed on candidate cell when it is deactivated SCell (if defined in RAN2)

- Detailed discussion will be commenced after receiving RAN2 LS.

Agreement

- From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart

  - Scenario 1: Beam indication before cell switch command

  - Scenario 2: Beam indication together with cell switch command

  - Scenario 3: Beam indication after cell switch command

- Interested companies are encouraged to further study the validity of the scenarios and the potential spec impact.

Agreement

- Interested companies are encouraged to perform technical analysis of the cell switch command from a RAN1 point of view, e.g.

  - Necessary information included in the command, which is relevant for RAN1 discussion

  - Necessary number of bits for the information

  - L1 impact or concern to use DCI or MAC CE for L1/L2 cell switch command

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

•         RACH-based solutions

e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

•         RACH-less solutions

e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- the association is provided as a part of candidate cell(s) configuration

- the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

  - Agreements at RAN2#121

Agreed: Usage of reference configuration:

- 	Candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration (FFS if done at cell switch or before the cell switch)

- 	The complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP.

-	To support reconfigurations that requires reset of RLC PDCP, this should be possible (in principle same a full config)

-	FFS if more than RLC PDCP should be kept and how much of “replacing” need to be specified.

-	FFS if the reference configuration can be derived from the current UE configuration at some point of time.

Potentially: R2 assumes that LTM without a separate reference configuration (if agreed) could work something like this:

-	Alt A: The candidate configuration (which need to be complete) is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP. (Same procedure as above)

-	Alt B: The candidate configuration (which can be a delta config) is applied to the current UE configuration (at the time of reconfiguration execution/cell switch), by legacy RRC reconfiguration procedure (it is assumed that the network need to coordinate if subsequent reconfigurations shall work, FFS feasibility).

agree to use Model 1: One RRCReconfiguration message for each candidate target configuration RRCReconfiguration to configure target candidate cells

Reference config can be empty

In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check.

In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration.

No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem).

To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

  - Agreements at RAN2#120 (From RAN2 chair notes)

R2-2211201	Discussion on RAN1 LS on measurement and configurations for L1L2-based inter-cell mobility	CATT, Fujitsu	discussion	Rel-18	NR_Mob_enh2-Core

RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e. measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam mgmt (to the extent it is supported) may be by the source cell. RAN2 understands that this may require cooperation source DU CU target DU and/or OAM coord. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

R2-2213332	38.300 running CR for introduction of NR further mobility enhancements	MediaTek Inc.	draftCR	Rel-18	38.300	17.2.0	B	NR_Mob_enh2-Core

Endorsed as baseline for further update

R2-2211202	On Procedure Descriptions	CATT	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

Include a procedure in the MTK stage-2 offline (e.g. acc to proposal and comments)

R2-2212438	Qualitative analysis on what to include in the RRC model for LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

P1	RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration.

P3	The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

P5	The MeasConfig IE can be optionally supported in an LTM candidate configuration.

P8	The OtherConfig IE is not required to be part of the LTM candidate cell configuration.

P9	The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure.

P10	The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID.

R2-2211456	Discussion on configurations for multiple candidate cells of L1 L2 mobility	Intel Corporation	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

On Delta Configuration

A UE stores the reference configuration as a separate configuration.

The reference configuration is managed separately

R2-2211487	Trigger and Execution of LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch.

LTM cell switch is supervised by a timer

UE arrival in the target cell need to be indicated (somehow)

R2-2213335	Report of #033 on Partial MAC reset for intra-DU LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable)

R2-2213336	Potential Partial MAC Reset for intra-DU LTM	vivo, MediaTek, Xiaomi	discussion	Rel-18

Noted

The summary in [R2-2213336] could be considered as the starting point for partial reset in intra-DU.

R2-2212865	Discussion on security issue in cell switch	NTT DOCOMO INC.	discussion	Rel-18

Permanent Identities such as PCI will not be used in L1 L2 signalling, instead L1 L2 signalling will use temporary identities configured by RRC.

  - Agreements at RAN2#119b-e(R2-2211061)

- Terminology

RAN2 to use “LTM” as term for the L1/L2-triggered mobility.

Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature

Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between.

Target performance enhancements

No security update support in Rel-18 with L1/L2 based mobility.

FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified.

For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command:

- MAC/RLC reset (when configured)

- RF retuning (e.g. needed for inter-frequency), baseband retuning

R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific):

- - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell.

- - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command

L1L2 based mobility supports the following CA scenarios:

- PCell change without SCell change

- PCell change with SCell change

Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN.

- L1 measurements and beam indication

RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed.

Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1.

RAN2 assumes that whether to use the unified TCI framework as the baseline for beam indication for L1L2 mobility is up to RAN1 (RAN2 observes that L1/L2 mobility need to support inter-freq cases).

- RRC

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered.

For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported.

- Dynamic cell switching

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering.

RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index.

FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this.

RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS).

FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress.

R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

  - Agreements at RAN2#119-e (R1-2208331/ R2-2209257)

Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further).

Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded).

Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios).

The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility.

RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration.

Measurement delay can/may be considered in this work

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

R2 will initially focus on PCell mobility.

R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases

- a) the target PCell/target SCell(s) is not a current serving cell (CA  CA scenario with PCell change)

- b) FFS the target PCell is a current SCell

- c) FFS the target SCell is the current PCell.

DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS).

Current options on the table: to configure a L1/L2 inter-cell mobility candidate cell:

- a.	One RRCReconfiguration message for candidate target cell

- b.	One CellGroupConfig IE for each candidate target cell

- c.	One SpCellConfig IE for each candidate target cell

Will send an LS to RAN1 and RAN3 on the progress of this meeting.