3GPP TSG-RAN WG1 #112bis-e	        		   R1-2304096

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on enhanced type-3 HARQ-ACK codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics), CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Enhanced Type-3 HARQ-ACK codebook cannot be supported for a serving cell configured with up to 32 HARQ processes. - In Rel-16, Type-3 HARQ-ACK codebook was introduced for NR-U, which was supported in FR1 only. Accordingly, only subcarrier spacings of 15, 30 and 60kHz (μ=0,1,2) are considered for processing time of Type-3 HARQ-ACK codebook. However, Type-3 HARQ-ACK codebook is supported for HARQ-ACK retransmission in Rel-17, which should be supported in FR1, FR2-1 and FR2-2. Hence, μ=3,5,6 should also be considered for processing time of Type-3 HARQ-ACK codebook. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In order to support up to 32 HARQ processes for enhanced type-3 HARQ-ACK codebook, NHARQ,cDL can be set to the value of nrofHARQ-ProcessesForPDSCH-v1700 for serving cell c, if provided. - Remove the limitation that only μ=0,1,2 is considered for processing time of Type-3 HARQ-ACK codebook. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear UE behavior to generate enhanced type-3 HARQ-ACK codebook for a serving cell if nrofHARQ-ProcessesForPDSCH-v1700 is provided for the serving cell. - There is no processing time defined for Type-3 HARQ-ACK codebook in FR2-1 and FR2-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated numbers of HARQ processes NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero.

Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700 for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind

*** Unchanged text is omitted ***

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and the DCI format includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is provided by the value of the MCS field for transport block 1 in the DCI format 1_1 or the MCS field in the DCI format 1_2. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N μ=0,1,2is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".