3GPP TSG RAN WG1 Meeting #112bis-e 														R1-2304102

e-Meeting, April 17th – April 26th, 2023

Source: 	Moderator (vivo)

Title:	FL summary #3 of other aspects on AI/ML for positioning accuracy enhancement

Agenda item:	9.2.4.2

Document for:	Discussion and decision

# Introduction

In this contribution, we summarize issues regarding other aspects on AI/ML for positioning accuracy enhancement in RAN1 #112bis-e for the following discussion.

[112bis-e-R18-AI/ML-07] Email discussion on other aspects of AI/ML for positioning accuracy enhancement by April 26 – Huaming (vivo)

- Check points: April 21, April 26

Note that the scope of agenda 9.2.4.2 including discussions on potential specification impact of AI/ML for positioning accuracy enhancement.

# Potential specification Impact

In this section, we provide a summary of issues, observations and proposals related to specification impact for positioning accuracy enhancements in the submitted contributions.

As in the SID, the related objectives are the following.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. For the use cases under consideration: - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signaling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) – RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) – RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

## Individual observations/proposals

The following are individual observations and proposals from the contributions.

| Sources | Observations/proposals |
| --- | --- |
| [1, Ericsson] | Observation 1 For model inference of Case 1/2a/3a, the same entity generates measurement data for model input and performs model inference. It is up to UE or gNB implementation to decide which type of information to use: CIR, PDP, or DP. Observation 2 For Case 3a (NG-RAN assisted positioning with gNB-side model, AI/ML assisted positioning): AI/ML model inference is up to network implementation and transparent to the UE and LMF. Observation 3 For AI/ML assisted positioning methods, self-model monitoring method does not require ground truth label and has no signaling overhead. The complexity, power consumption, and latency for obtaining one model monitoring sample are equal to one round of model inference. Observation 4 Performance monitoring of AI/ML assisted positioning (e.g., Case 2a/3a) can be achieved by evaluating the residual loss from the triangulation-based error minimization positioning algorithm (i.e., conventional positioning methods). No need to collect labelled data for model monitoring purpose. Observation 5 For Case 3a (NG-RAN assisted positioning with gNB-side model, AI/ML assisted positioning): If LMF is the network entity with ground truth label knowledge, the NRPPa protocol needs to be extended to support transmission of ground truth label from LMF to the training data collection entity. Observation 6 For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning): The existing reporting from gNB to LMF may need to be enhanced to support model inference due to potentially new information type (e.g., CIR) and/or a larger size of measurement report. Observation 7 For direct AI/ML positioning methods, self-model monitoring method does not require ground truth label and has no signaling overhead. The complexity, power consumption, and latency for obtaining one model monitoring sample are equal to one round of model inference. Observation 8 For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning): The same standard impact (if any) is expected to support data collection for model training and model inference. Observation 9 Existing PRS configuration is sufficient for data collection purposes for Case 1/2a/2b. Observation 10 For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning): Model inference can be supported with existing signalling, where the reporting of model output to LMF reuses the existing LPP IEs (e.g., LoS/NLOS indicator, ToA/RSTD, DL-PRS-RSRP, DL AoD). Observation 11 If supporting Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning): The existing measurement reporting from UE to LMF may need to be enhanced to support model inference due to potentially new information type (e.g., CIR) and/or a larger size of measurement report. Observation 12 If supporting Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning), it is necessary to define ML model input which is to be carried by LPP from UE to LMF. The extent of specification impact depends on the type and size of measurement results that are required as the model input. Observation 13 If supporting Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning): Model monitoring is left for implementation. No specification impact is expected. Observation 14 If supporting Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning): If label information is not needed from UE for training data collection, the same standard impact is expected to support data collection for model training and model inference. If label information is needed from UE, signaling enhancements may be necessary to support training data collection. Proposal 2 For the positioning use case, model ID is not defined. Model ID based LCM is not supported for UE-side model. Proposal 3 For the positioning use case, functionality ID and functionality ID based LCM are supported for UE side model. Proposal 6 For model inference of Case 2b/3b, PDP and DP are prioritized over CIR considering the smaller model input size and the limited specification impact. Proposal 7 For model inference of Case 2b/3b, study how to specify the signalling of measurement data for model input, including PDP and DP. Proposal 8 For all Cases, design choice for model input (including information type and size) need to consider all LCM stages, not just model inference stage. Proposal 9 For Case 2b/3b, study how to adjust the timing value range, and format the timing measurement information for signalling over the interfaces (LPP, NRPPa), where the timing measurements are used directly or indirectly as input to the AI/ML model in LMF. Proposal 10 For training data collection of all Cases, data format information (e.g., granularity factor k) is reported together with the timing related measurement data. Proposal 11 At least for AI/ML models residing at network side (Case 2b, 3a, 3b), it is outside RAN1 scope to discuss whether/how to map the AI/ML functional entities to network nodes. Proposal 12 For Case 3a (NG-RAN assisted positioning with gNB-side model, AI/ML assisted positioning): The input to the AI/ML model does not need to be specified. The model output can be reported from gNB to LMF using existing signaling. No specification impact is expected for model inference. Proposal 13 Conclude that for Case 3a, model monitoring metric is calculated without collecting test data. No signalling needs to be specified to collect test data for model monitoring purpose. Proposal 14 For Case 3a (NG-RAN assisted positioning with gNB-side model, AI/ML assisted positioning): Study signaling enhancements for the LMF to provide the ground truth label (e.g., ground truth direct path ToAs or UE locations) to support the training data collection. Proposal 15 For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning): Postpone the final specification impact discussion until it has been concluded based on evaluations the suitable information to report as model input. Proposal 16 For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning): Conclude that for Case 3b, model monitoring metric is calculated without collecting test data. No signalling needs is to be specified to collect test data for model monitoring purposes. Proposal 17 For Case 1/2a, for model inference as well as training data collection, the benefits of adding support for assistance information should be proven with evaluations before RAN1 discuss what assistance information to support and the potential specification impact. Proposal 18 For Case 1 (UE-based positioning with UE-side model): Conclude that model monitoring is handled on the UE side. Proposal 19 Conclude that there is no need for PRS configuration enhancements for data collection purposes for Case 1/2a/2b. Proposal 20 For Case 1, introduce support for the UE to request PRS transmissions for training data collection purposes. Proposal 21 For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning): No specification impact for model inference is expected when the model output is fully aligned with existing measurement report. Proposal 22 For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning): Conclude that model monitoring is handled on the UE side. Available residual loss information could be used as assistance data from the network to the UE for model monitoring purposes. Proposal 23 For Case 2b, the IE for reporting model input should kept small (e.g., DP only). Full-size CIR is not considered. Proposal 24 If supporting Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning): Study the requirements for training data logging and reporting using 3GPP protocols. |
| [2, Huawei] | Proposal 1: For Case 1 (UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning), the model training/updating and inference are performed all at UE side. Proposal 2: Study the following modes of model monitoring and the potential spec impact for Case 1: - UE collects inputs for monitoring, calculates monitoring KPIs which are then fed back to NW, and NW makes monitoring decision. - UE collects inputs for monitoring, calculates monitoring KPI, makes monitoring decision, and reports the decision to NW; NW will indicate UE to execute the decision accordingly - NW may configure a threshold criterion (e.g., threshold RSRP/SINR or threshold intermediate KPIs) to facilitate UE to make decision. Proposal 3: For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at UE side. Proposal 4: Study the following modes of model monitoring and the potential spec impact for Case 2a: - UE collects inputs for monitoring, calculates monitoring KPIs which are then fed back to NW, and NW makes monitoring decision. - UE collects inputs for monitoring, calculates monitoring KPI, makes monitoring decision, and reports the decision to NW; NW will indicate UE to execute the decision accordingly - NW may configure a threshold criterion (e.g., threshold RSRP/SINR or threshold intermediate KPIs) to facilitate UE to make decision. Proposal 5: For Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF side. Proposal 6: For Case 2b, if justified by overhead and performance evaluation, to facilitate the data collection for initial model training and model updating of the AI/ML-model for positioning, study to support: - Measurements (e.g. CIR, PDP) performed at PRU/UE and sent to LMF - Note: Labels (PRU locations) known by definition in LMF, no spec impact Proposal 7: For inference with LMF based direct positioning for Case 2b, if justified by performance and overhead evaluation, support the transfer of new measurement reports to carry e.g. CIR or PDP from UE to the LMF. Proposal 8: Study the following modes of model monitoring and the potential spec impact for Case 2b: - NW collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision - UE/PRU collects inputs for monitoring, calculates monitoring KPIs which are then fed back to NW, and NW makes monitoring decision. Proposal 9: For Case 3a (NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at gNB side. Proposal 10: Study the following modes of model monitoring and the potential spec impact for Case 3a: - NW collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision Proposal 11: For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF side. Proposal 12: For Case 3b, to facilitate the data collection for initial model training and model updating of the AI/ML-model for positioning, study to support: - Measurements (e.g. CIR, PDP) performed at gNB and sent to LMF - Note: Labels (PRU locations) known by definition in LMF, no spec impact Proposal 13: For inference with LMF based direct positioning for Case 3b, support the transfer of new measurement reports to carry e.g. CIR or PDP from gNB to the LMF. Proposal 14: Study the following modes of model monitoring and the potential spec impact for Case 3b: - NW collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision Proposal 15: For training data collection for AI/ML based positioning support assistance signaling of the requested quality of training data. Proposal 16: Study the potential spec impact of data collection from realistic network for supporting the model training and updating of the AI/ML model, including at least: - Signaling for indicating/requesting data collection. - Feedback of channel measurements. - Methods of improving data quality. Proposal 17: At least for the direct AI/ML positioning, since the required measurement payload size to achieve a given accuracy target varies depending on deployment scenario and channel conditions, measurement reporting with flexible payload size should be supported. Observation 1: If the data collection for the model training/updating in UE-based positioning with UE-side model (Case 1 for direct positioning) is to be specified, a potential procedure could be - Entities used for training data generation (e.g. PRUs/UEs) perform measurements on PRS and report them to the gNB or LMF. - The PRU location is known by the NW per definition - The NW collects the measurement reports from the entities used for training data generation and labels them with their UE locations. - The NW sends the collected channel measurements together with the label (coordinates) to the training entity. Observation 2: If the data collection for the model training/updating in UE-based positioning with UE-side model (Case 1 for assisted positioning) is to be specified, a potential procedure could be - Entities used for training data generation (e.g. PRUs/UEs) perform measurements on PRS and report them to the gNB or LMF. - The positioning label is obtained autonomously at the NW, e.g., LMF. - The NW collects the measurements reports from the entities used for training data generation. - The NW sends the collected channel measurements together with the labels (e.g. LOS tags or TOAs) to the training entity. Observation 3: For the model inference in UE-based positioning with UE-side model (Case 1 for direct positioning), - The gNB sends PRS to the UE that contains the AI/ML model. - The UE infers the position based on the channel measurements obtained from PRS. Observation 4: For the model inference in UE-based positioning with UE-side model (Case 1 for assisted positioning), - The gNB sends PRS to the UE that contains the AI/ML model. - The UE infers the information needed for final positioning (e.g. LOS/NLOS states, TOA). - The UE performs the final positioning. Observation 5: If the data collection for the model training/updating in UE-assisted/LMF-based positioning with UE-side model (Case 2a) is to be specified, a potential procedure could be - Entities used for training data generation (e.g. PRUs/UEs) perform measurements on PRS and report them to the gNB or LMF. - The positioning label is obtained autonomously at the NW. - The NW collects the channel measurements and sends them together with the labels (e.g. LOS tags or TOAs) to the training entity. Observation 6: For the model inference in LMF-based positioning with LMF-side model (Case 2a), - The gNB sends PRS to the UE that performs the channel measurements. - The UE uses its channel measurements to infer the model output (e.g. e.g. LOS/NLOS states, TOA). - The UE transmits the model output to the LMF that performs the final positioning. Observation 7: For the data collection for model training/updating in LMF-based positioning with LMF-side model (Case 2b), - Entities used for training data generation (e.g. PRUs/UEs) perform measurements on PRS and report them to the LMF. - The positioning labels (coordinates) are known to the LMF. No signaling needed. - The LMF uses the received measurement data together with the labels to train the AI/ML model. Observation 8: For the model inference in LMF-based positioning with LMF-side model (Case 2b), - The gNB sends PRS to the UE that performs the channel measurements. - The UE sends the channel measurement results to the LMF where they are used as input to the AI/ML model for inference of the UE position. Observation 9: For the model training/updating in NG-RAN node assisted positioning with gNB-side model (Case 3a), - PRUs/UEs transmit SRS to the gNB. - The training data labels (e.g. LOS/NLOS state, TOA) are obtained autonomously at the network. - The gNB performs channel measurements based on SRS and uses them for training as input to the AI/ML model together with labels such as LOS/NLOS states or TOAs. Observation 10: For the model inference in NG-RAN node assisted positioning with gNB-side model (Case 3a), - The UE transmits SRS to the gNB. - The gNB performs channel measurements based on SRS and delivers them as input to the AI/ML-model where the LOS/NLOS or TOA is inferred. - The gNB transmits the results of the inference to the LMF where the final positioning is performed. Observation 11: For the model training/updating in NG-RAN based positioning with LMF-side model (Case 3b), - The PRU/UE transmits SRS to the gNB. - The LMF knows the PRU location by definition and is using it as label. - The gNB performs channel measurements based on SRS from various PRUs/UEs. - The gNB sends the obtained channel measurements results to the LMF. Observation 12: For the model inference in NG-RAN based positioning with LMF-side model (Case 3b), - The UE sends SRS to the gNB that performs the channel measurements. - The gNB sends the channel measurement results to the LMF where they are used to infer the UE position. Observation 13: By setting requirements for the data characteristics for training and model updating, training efficiency and performance can be optimized, e.g. improving generalization characteristics, reducing training effort and reporting overhead. |
| [3, ZTE] | Observation 1: For Case 1 and Case 2a, if model transfer/delivery is not considered, UE side can collect the training data through a specification-transparent way. Observation 2: The current associated information to the DL PRS (e.g., TRP ID, TRP location, carrier frequency etc) is already defined in TS 37.355. Additional association information is not necessary to be defined for UE side data collection. Observation 3: If AI model input includes channel observations from a two-port PRS, positioning accuracy is apparently improved with the same amount of training samples compared to single-port PRS. Observation 4: Measurements on multi-port PRS not only increase representative channel observations between UE and TRP, but also maintain spatial consistency between neighbor UEs. In addition to that, another advantage is to reduce the efforts to collect training dataset in reality due to a variety of channel observations per UE location. Observation 5: AI/ML assisted positioning is more appropriate to be implemented at UE/TRP side since the motivation is to increase reliability and accuracy of UE/TRP measurement based on a raw channel. Observation 6: At least for supervised learning, one important thing is that ground-truth labels should be accessible. If training an AI model has to rely on a non-reliable ground-truth labels, it cannot be expected that AI model can infer a more reliable intermediate result than the non-reliable ground-truth labels. Proposal 1: Reuse on-demand PRS mechanisms defined in Rel-17 for PRS request of data collection by UE side. Proposal 2: For Case 3a, model training and model deployment are left up to network implementation. There is no specification impact on data collection. Proposal 3: At least for data collection of Case 2b and Case 3b, support UE/TRP to report more than 8 additional path timings and RSRPPs. Proposal 4: At least for data collection of Case 2b and Case 3b, support UE/TRP to report path phase of a channel path in addition to path power and path timing. Proposal 5: At least for data collection of Case 2b and Case 3b, study and support multi-port SRS/PRS in order to collect enriched channel observations. Proposal 6: For AI/ML assisted positioning, intermediate results of AI/ML model should consider the accessibility to ground-truth labels. Proposal 7: For AI/ML assisted positioning, at support following intermediate results as the model output: • DL-RSTD values for first detected path; • LOS/NLOS indicator; • DL PRS-RSRPP values for first detected path Proposal 8: For AI/ML assisted positioning, study measurement report enhancement for AI/ML assisted intermediate results under both single TRP and Multi-TRP construction. Proposal 9: For functionality identification, AI/ML-enabled feature is equal to a functionality. • Direct AI/ML positioning is an independent functionality. • For AI/ML assisted positioning, the granularity of the functionality is determined by the model output type Proposal 10: For model identification, study how to report model capability for a specific AI/ML-enabled feature. |
| [4, vivo] | - Support time domain CIR as one model input for training of AI/ML model for positioning. When the entity conducting model training and data collection is not the same, collected data should be delivered from the data-collection entity to the model-training entity. - Some dedicated reference signals should be configured to support data collection if necessary, such as PRS configuration for downlink positioning and SRS configuration for uplink positioning. - Further study related assistance information at least consisting of RS configuration and data collection indication to support data collection. For ground truth label collection, to improve the quality of labels, indicate UE the criteria or requirement for data labeling or indicate UE to report label quality indicator. - For measurement collection, at least measurement type and format should be indicated, depending on to model input. For label quality indicator reporting, study how and to what extent label quality indicator can improve model training. - Both PRUs and regular UEs can be used to perform data collection. - Further study the specification impact of data collection for semi-supervised learning. - Real-time on-device model training with a large-scale dataset should be avoided at UE side. - Other potential issues on data quality and terminal capability should be considered for data collection. - Model information should contain meta-information indicating model capability and the physical and network environment or condition under which the model is suitable for operation. - The process of model activation and deactivation is needed to flexibly control the model's lifecycle, so as to ensure positioning performance. Network side should deploy a model pool containing multiple models with same structure but different parameters to UE in advance for model selection. - Network side could send a model selection instruction to instruct the target UE to select a suitable model from the model pool, when the current model does not work well. Model monitoring can be achieved based on the following ideas. - Monitor covariate (input) shift: detecting whether or to what extent the distribution of model inputs in the test dataset is consistent with the training dataset. - Monitor concept (translation) shift: detecting whether or to what extent the mapping between model inputs and model outputs in the test dataset is consistent with the training dataset. Monitoring covariate (input) shift can achieve model input based model monitoring, and can be used as an indirect indicator of degraded performance due to its accessibility without the needs of model output related information, such as ground truth labels. Monitoring concept (translation) shift can achieve model output based model monitoring, and can provide a definite answer on whether the deployed model is still accurate or not, with the needs of model output related information, such as ground truth label. Multiple monitoring methods can be integrated together to make a model monitoring decision with the considerations of overhead and accuracy. - Study specific model monitoring schemes and their specification impacts for both model input based model monitoring and model output based model monitoring with considering the shift of covariate (input) and concept (translation). The classification criteria for self-monitoring and non-self-monitoring methods is whether the entire process of model monitoring is implemented on the physical entity where the AI/ML model is deployed without the assistance of other physical entities. According to the functionality involved in the process of model monitoring, model monitoring can be divided into three entities: - Entity of data collection for model monitoring - Entity of metric calculation for model monitoring - Entity of monitoring decision for model monitoring For model monitoring, the specific mapping between the logic entities to the physical entities is case-by-case. Dedicated reference signal may be configured to support data collection for model monitoring. Under the general model monitoring framework, the further analysis of specific specification impact on model monitoring should be case-by-case, including the mapping between the logic entities to the physical entities and the specific signaling. Further study specification impact of the shift detection of dominant feature distribution based model monitoring, may include - the type(s) of dominant feature(s) for input data - the reference distribution(s) of dominant feature(s) Further study specification impact of AI/ML based adversarial validation based model monitoring, may include - The input data collection for monitoring entities for adversarial validation During training the AI/ML model for positioning, a seperate model could be tailored to continually detect whether the model input is out-of-distribution, so as to monitor the performance of the AI/ML model for positioning, with limited resource consumption. Further study specification impact of AI/ML based out-of-distribution based model monitoring, may include: - AI/ML model for monitoring transferred from NW side to UE side. The main specification impact of ground truth label based model monitoring is the procedure of collecting the samples with ground truth labels, which can be divided into the scope of data collection Further study specification solution and impact on model monitoring for AI/ML assisted positioning, may include - The error threshold between AI output and estimated timing based on calculated location - The timing measurement enhancement indication based AI/ML assisted positioning Further study specification impact of motion sensors assisted model monitoring considering the UE privacy, may include: - Indication to inform UE side to collect motion sensor information. During training the AI/ML model for positioning, a separate ranging model could be tailored to continually monitor the performance of the AI/ML model for positioning, with limited resource consumption. With the assistance of the ranging model, efficient model monitoring can be achieved without the needs of other measurement related to model output. Further study specification impact of ranging model assisted model monitoring, may include： - Ranging model transferred from NW side to UE side for monitoring. - When fine-tuning is conducted at UE side, UE capability corresponding to fine-tuning is required. - To enable model fine-tuning when AI/ML model inference is at UE side, support assistance information to the target UE about pre-trained model and training configuration. - Training data collection request for model fine-tuning and feedback from the target UE is required to support model fine-tuning at network side. - The result of model monitoring and the achievability of model updating should be jointly considered as the condition of model updating. - Support time domain CIR as one model input for AI/ML based positioning. - For direct AI/ML positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed. - For AI/ML assisted & UE assisted positioning, support the target UE to report the output of AI/ML model inference (intermediate feature for positioning) when model inference is at UE side. - For AI/ML assisted positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed. - TRP-related information, such as encoded TRP ID and TPR’s location, should be incorporated into the model input for Construction 2 (Single model, same TRP for N model) of AI/ML assisted positioning without additional signaling overhead. - For Construction 1 (single TRP, N model for N TRPs), uplink positioning should be primarily considered, and AI/ML models should be deployed at gNB side. - For Construction 2 (single TRP, same model for N TRPs), downlink positioning should be primarily considered, and AI/ML model should be deployed at UE side. - For Construction 3 (Multi-TRP, one model for N TRPs), uplink positioning should be primarily considered, and AI/ML model should be deployed at LMF side. Support TOA as a new measurement reporting from UE side to NW side. - A general model management procedure should be specially studied for AI/ML based positioning accuracy enhancement. - Support to study the detailed assistance signaling configuration when the model management procedure for AI/ML based positioning is agreed. |
| [5, OPPO] | Observation 1: For UE based and UE-assisted positioning method, if the model for direct AI/ML positioning is trained based on the labels that obtained by NR DL-TDOA scheme, - there is significant performance degradation compared to the ideal ground-truth labels - there is no obvious performance gain compared to legacy NR positioning methods Observation 2: For UE-assisted positioning method, if the model for AI/ML assisted positioning is trained by UE/chipset vendor and generates UE measurement results for some existing type(s), the reporting can reuse existing NR signaling and there is no strong motivation to specify the input of AI/ML model Observation 3: For UE-assisted positioning method, if the outputs of the model for AI/ML assisted positioning are some new type(s) of UE measurement, specification enhancement will be needed. - e.g., new reporting format, new type of measurement and corresponding requirement Observation 4: For UE-based positioning method, if the model for direct AI/ML positioning is trained by UE/chipset vendor, the reporting can reuse existing NR signaling and there is no strong motivation to specify the input Observation 5: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on existing UE measurement and reporting, the AI operations at network side can be transparent to UE. Observation 6: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on new type(s) of UE measurement/reporting, specification enhancement will be needed. - e.g., new reporting format, new type of measurement and corresponding requirement Proposal 2: For AI/ML based positioning, regarding the data collection for AI model training, NOT support UE to report the ground-truth labels of its location(s) Proposal 3: For the training data collection at UE side (e.g. UE-based positioning with UE-side model (Case 1), UE-assisted positioning with UE-side model (Case 2a)), study the potential spec impact on that UE sends request of preferred or supported configurations to LMF - FFS: Whether this request is sent by UE capability signaling or other signaling. Proposal 4: For the training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), study the potential spec impact (including necessity and benefit) of the following options - PRU: PRU reports measurement results (corresponding to model input) optionally with labels via LPP or other new signaling defined by RAN2. - Other network entity (e.g., LMF) maybe know the label(s) in advance - UE: UE reports measurement results (corresponding to model input) without labels via LPP or other new signaling defined by RAN2 - This kind of data is used for semi-supervised learning. Proposal 5: For the training data collection at NW side NG-RAN node assisted positioning with gNB-side model (Case 3a) or NG-RAN node assisted positioning with LMF-side model (Case 3b), study the potential spec impact (including necessity and benefit) of the following options - TRP can collect the measurement results (corresponding to model input) based on PRU’s transmission and report them via LPP or other new signaling defined by RAN2. - Other network entity (e.g., LMF) generates the associated labels based on the know location of the corresponding PRU - TRP can collect the measurement results (corresponding to model input) based on UE’s transmission and report them via LPP or other new signaling defined by RAN2. - This kind of data is used for semi-supervised learning. Proposal 6: For the data collection used for AI model inference - When direct AI/ML positioning is used for UE-base positioning method (Case 1) or AI/ML assisted positioning is used for UE-assisted positioning method (Case 2a), the UE will collect measurement for the input of AI model - If the model is trained at the same side, the inputs/data collection are up to UE implementation and transparent from the perspective of air interface - If the model is trained at NW side and AI model inference is performed at UE side (when model transfer is supported), the size/contents of inputs will need to be pre-defined or pre-configured. - When direct AI/ML positioning is used for UE-assisted positioning method (Case 2b), the target UE will report the measurement results to LMF via LPP signaling - FFS: type of measurement (e.g., existing measurement type, new measurement type), RS configuration for measurement - When AI/ML assisted positioning is used for NG-RAN node assisted positioning method (Case 3a), the TRP will collect measurement for the input of AI model by implementation and transparent from the perspective of specification - When direct AI/ML positioning is used for NG-RAN node assisted positioning method (Case 3b), the TRP will report the measurement results to LMF via NRPPa signaling Proposal 9: For AI/ML based positioning, - if UE-based positioning method is used (Case 1), study the following aspect on spec impact - support to report the confidence of the AI estimated location on top of location information (LPP signaling from UE to LMF) - if UE assisted positioning method is used (Case 2a, 2b), study the following aspects on spec impact - Support of the existing reporting via LPP signaling from UE to LMF - whether/what enhancement, e.g., finer granularity for the measurement result quantization) - whether/what new type of measurement /reporting (LPP signaling from UE to LMF) - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered. - if NG-RAN node assisted positioning method is used (Case 3a, 3b), study the following aspects on spec impact - Support of the existing reporting via NRPPa signaling from gNB to LMF - whether/what enhancement, e.g., finer granularity for the measurement result quantization) - whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF) - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered. Proposal 10: For UE-side model is used for AI/ML based positioning (e.g., Case 1, Case 2a), support implicit or explicit information from LMF to UE to indicate/identify the scenarios/configuration so that UE can choose a suitable AI model matching the target case(s) - For NW-side model, such type of signaling may not be needed. Proposal 11: Regarding AI model monitoring for AI/ML based positioning, Not support UE to report “target output” or “label” for the comparison with the output of the AI/ML model Proposal 12: For AI/ML based positioning, if PRU is utilized to collect data for AI model performance monitoring, at least study the following aspects - evaluate/justify whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar - availability of PRU for typical deployment |
| [6, Spreadtrum] | Observation 2: Input data based monitoring can be considered for the evaluation of positioning monitoring, and it seems no spec enhancement is needed. Observation 3: For AI/ML assisted positioning, monitoring based on model output is not necessary. Observation 4: The integrity mechanism can be considered as one tool to evaluate/monitor the performance of AI/ML model. Proposal 2: For case 2a and case 3a, the output of AI/ML model can be considered as new metric, e.g., LOS/NLOS hard/soft judgement, Path phase. Whether to support it depends on the evaluation of AI9.2.4.1. Proposal 3: For case 2b and case 3b, the input of AI/ML model can be considered as new metric, e.g., CIR/PDP. Whether to support it depends on the evaluation of AI9.2.4.1. Proposal 4: Whether/How to define/reflect the complexity of the AI/ML operation in the specification can be studied. Proposal 5: For direct AI/ML positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label for output based monitoring metric. Proposal 6: For AI/ML assisted positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label to evaluate the positioning accuracy. Proposal 7: For all cases, ground-truth labels can be reported to LMF, and it is up to LMF to monitor/evaluate the performance of AI/ML model. For case 1 with direct positioning, other mechanisms also can be considered. |
| [7, Nokia] | Observation 1: In the scope of functionality identification framework, the new set of methods and their respective UE’s capability conditions can be mapped using as a reference the current LPP framework defined in TS 3GPP 37.355 for AI/ML positioning cases. Observation 2: Considering as starting point, the LPP protocol as the 3GPP legacy framework of the functionality identification aspects on AI/ML enhanced positioning cases is beneficial for the current study item. Besides that, it is expected to include new positioning methods based on AI/ML and new reporting IEs. Observation 3: Regarding data collection, the noisy label evaluation can be assisted by the LMF. Samples with one or several sources of noisy labels should be pre-evaluated as part of data collection for model updating and model retraining. Observation 4: In case of the opportunity to label a sample with multiple positioning sources, one may exploit all the information from different positioning sources and incorporate them in the labels for training the ML model. Observation 5: Any given positioning measurement that does not represent well the expected distribution for a set of TRPs relative to the UE location may be seen as abnormal/rare events. Nevertheless, some rare events may prove useful to the ML positioning function, while others may be entirely detrimental. Observation 6: Because of RF limitations, some impairments are generated, such as phase rotation and delays of the positioning signal by the RF chain at the baseband receiver. As a result, a positioning entity (UE, TRP, etc.) hosting the ML positioning function could experience certain RF-based signal distortions which are not considered explicitly or characterized and compensated for updating the model. Observation 7: RF-based signal imperfections are different between host-type devices. For example, a PRU or gNB hosting the ML model would require adapting the model to its RF-specific characteristics. Observation 8: Performance monitoring may be done proactively (e.g. on a periodic basis), or reactively (over demand basis). Any entity (the UE or the LMF) that detects an environment change may trigger a reactive performance monitoring. Observations 9: A new measurement report such as monitoring metric can be used to switch the positioning functionalities (direct AIML, AI/ML assisted). The monitoring metric can be considered as an associated information to the given inference output. Observation 10: In Case 2a, depending on the type of intermediate feature reported to the LMF, the UE may need specific network assistance. The assistance depends on the type of intermediate feature (e.g. range/distribution). Observation 11: NW may assist performance monitoring using both labelled and unlabelled test data for case 3a using NRPPa interface. The NW may clean the test data prior to sending it to the gNB via NRPPa. Observation 12: For case 3b, when the performance monitoring is triggered, the LMF may collect monitoring data from selected gNBs using the existing positioning legacy 3GPP protocols. Observation 13: Output-based performance monitoring for cases 2b, 3b can be implemented autonomously by the LMF and does not require standardization. Observation 14: Output-based performance monitoring for case 3a may follow proposal 25 the difference being that the request is sent over a different legacy 3GPP framework (NRPPa). Observation 15: The existing legacy 3GPP framework may be extended with possible new methods and IEs to support and report the inference output of AI/ML positioning enhancement. Observation 16: The solution approaches presented so far as part of this study have considered only one-sided models, with the AI/ML model output directly indicating the UE location or providing intermediate features that are used by classical positioning approaches to estimate the UE location. However, for some cases using CIR and PDP with high dimensionality as model input could cause significant overhead for data collection for model training and inference. It is also unclear how an AI/ML model for positioning can cope with variable CIR/PDP size and shape. Observation 17: A positioning reference signal receiver (e.g. UE or TRP) that uses AI/ML positioning (direct or assisted) must ensure that the input to the AI/ML block is of fixed characteristics (e.g. size and shape), regardless of the size and shape of the received positioning reference signal. Observation 18: The performance of LOS/NLOS classification using AI/ML has a strong dependency on the environmental setting as well as the bandwidth capabilities of the UE. Observation 19: Using the above information, the LMF may select a quantization strategy including e.g.: scalar quantization (SQ) and vector quantization (VQ). Observation 20: For soft RSTD, model complexity cannot be ignored while studying the trade-off between the performance gain and the reporting overhead. Observation 21: The uncertainty involved with the ToA/RSTD estimation also depends on the operating SNR conditions and freshness of collected samples distribution. Proposal 1: For understanding various AI/ML positioning cases, it would be beneficial to include illustrative figures in the technical report (TR 38.843). Proposal 2: Regarding functionality identification framework, to study and discuss the potential specification impact for functionalities and reporting of UE’s applicable conditions. Proposal 3: RAN1 to study and discuss the specification impact of selected positioning anchors indication as intermediate feature on AI/ML assisted positioning. Proposal 4: RAN1 to consider at least the following mandatory UE’s applicable conditions on supporting ML functionalities for all AI/ML positioning enhancement cases (Case 1, Case 2, and Case 3): - Max number of supported functionalities (1, 2, 4, 8, ...) Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE - Delay in activating a functionality (2 ms, 4 ms, ...) Indicates the delay required when activating or switching a functionality - Generalization condition of functionalities (true, false) Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. Proposal 5: RAN1 to consider at least the following mandatory UE’s applicable conditions for Cases 1 and Case 2a: - Supported N’t: To indicate the N’t values that the UE is capable to consider (N’t = 64, 128, 256, 512). - Supported N_port: To indicate N_port, which is the number of transmit/receive antenna port pairs to consider (N_port = 1,2,4). - Supported N_TRP: To indicate N_TRP, which is the number of TRPs to (N_TRP = 1, 2, …, 72) Proposal 6: RAN1 to consider at least the following optional UE’s applicable conditions for Cases 1 and Case 2a: - Supported on collecting and labeling dataset for training, updating, and monitoring: It defines the support of the UE to receive data collection assistance from the LMF. - Supported anchor indication from the LMF: It indicates whether anchors selection by the LMF is supported. - Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements. - Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift. - Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring. - Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR. Proposal 7: RAN1 to consider at least the following mandatory UE’s applicable condition for AI/ML assisted positioning in Case 1 and Case 2a: - Supported intermediate_feature: To indicate the intermediate feature (ToA, LOS/NLOS indication, AoA, Selected positioning anchors). Proposal 8: RAN1 to consider at least the following optional UE’s applicable condition for Case 2a: - For LOS/NLOS classification - supported ue_channel features: Indicates which UE channel features can be used for determining LOS/NLOS indication (UE_channel_features = {CIR_energy, CIR_maximun_amplitude, CIR RMS delay spread, CIR skewness, CIR_kurtosis}). Proposal 9: RAN1 to deprioritize the study and discussion of the specification impact of data collection for model training for the current functionality identification framework. Proposal 10: RAN1 to prioritize the study and discussion of the specification impact of data collection for model update/re-tunning and performance monitoring considering legacy 3GPP framework for positioning. Proposal 11: For data collection, RAN1 to study and discuss the potential specification impact of at least the following challenges: data availability, noise ground truth, presence of abnormal propagation conditions, and RF imperfection. Reusing as much as possible the legacy 3GPP framework on positioning. Proposal 12: For ground truth labels, RAN1 to study and discuss the potential specification impact of at least the UE distribution used for model updating and performance monitoring for positioning cases. Reusing as much as possible the legacy 3GPP framework on positioning. Proposal 13: For UE-sided model (Case 1 and Case 2a), RAN1 to study solutions and their respective specification impact, as part of data collection, using network driven evaluation to verify the consistency of labels. Proposal 14: RAN1 to define means on identification and management of abnormal propagation conditions during data collection and the potential specification impact (e.g., define conditions to identify abnormal propagation conditions, define actions to manage such as whether to discard associated measurement, etc.). Proposal 15: RAN1 to study the specification impact on using the existing positioning framework, through which a generic ML positioning model can be customized to the specific NR elements host types - including target UE, PRU, or gNBs and their RF chain imperfections. Proposal 16: For Case 1, RAN1 to study the specification impact of the NW indication to UE requirements and parameters related to data collection (e.g., IPD threshold, data augmentation configuration, abnormal propagation condition). Reusing as much as possible the legacy 3GPP framework on positioning. Proposal 17: For Case 2a and Case 2b, RAN1 to study the specification impact of NW selecting a set of PRS configurations related to data collection. Proposal 18: For Case 3a and Case 3b, RAN1 to study the specification impact of gNB selecting a set of SRS configurations related to data collection. Proposal 19: Regarding performance monitoring, RAN1 to study the specification impact of using PRU through LPP protocol. Proposal 20: For Case 1 and Case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contains monitoring data for AI/ML model performance (including samples with corresponding labels). Proposal 21: RAN1 to further study and discuss the possible specification impact considering additional model outputs, which can deliver new monitoring metric to trigger model updating/switching. Proposal 22: For Case 1, RAN1 to consider the NW assistance for performance monitoring using both labeled and unlabeled data. Such data may be collected from PRUs and/or other UEs (in case of unlabeled data) by the NW. Proposal 23: For Case 2a, RAN1 to study and discuss at least potential specification impact when labelled data is used for monitoring. The labels are the intermediate features, which are extracted by the PRU. PRU data is collected by the NW, potentially cleaned, and then transferred to the target UE. Proposal 24: For Case 1, Case 2a (UE-side model) considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contain data for monitoring the AI/ML model performance (including samples with corresponding labels). Proposal 25: For Case 1 AI/ML direct and assisted AI/ML positioning, RAN1 to study the potential specification impact when UE triggers a request to the NW based on its self-evaluation of the model performance (e.g., model output related performance). This request can be also used to trigger any LCM step (enable model updating within a functionality, functionality switching) and may be set over a legacy 3GPP framework. Proposal 26: RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples. Proposal 27: RAN1 to study solutions that enable the UE to report positioning measurements in a fixed format characteristics (e.g., size and shape) independent of the PRS configuration, including what type of assistance the LMF may provide for enabling the fixed format. Proposal 28: For Case 1, Case 2a, and Case 3a, the network should be able to assist UE and/or gNB for LOS/NLOS classification by providing a ranked list of channel features such as containing CIR characteristics, RMS delay spread, etc. The ranked list should be based on the UE and/or gNB capabilities in terms of bandwidth, computation, and environmental setting. Proposal 29: In Case 1 and Case 2a, a UE-sided model may conduct anchor selection for positioning. For this, LMF may request UE to do anchor selection, and provide any necessary assistance data (e.g., containing a list of candidate anchors with their locations). To train the ML model, UE may request information from LMF on the positioning QoS resulting from the selected anchor(s). Proposal 30: RAN1 to study intermediate feature quantization impact on localization accuracy. Proposal 31: RAN1 to assess what type of assistance information LMF can provide to the UE regarding the quantization of the intermediate feature. |
| [8, CATT] | Observation 1: If all UEs/gNBs always provide the whole measurements and ground truth labels with quality indicator to LMF side for selecting the high quality training samples, the transmission of discarded samples with low quality indicator increases unnecessary resource overhead. Observation 2: Training AI/ML model for positioning at network side is more feasible due to easier data collection and stronger computational resources. Proposal 1: Regarding data collection for AI/ML model training, the ground truth labels are generated by the following entity in addition to PRU with known location: - The UEs with non-NR positioning capability such as GPS or GNSS; - The UEs or network entity with high confidence degree positioning results based on existing NR RAT-dependent positioning methods. Proposal 2: Regarding data collection for AI/ML model training, whether and how to select/use the partial and/or noisy ground truth labels to improve the performance of AI/ML model can be further studied. Proposal 3: For case 1, case 2a and case 3a, if UE-side model and gNB-side model is trained at UE and gNB side respectively, LMF side can collect a large-scale dataset from numerous UEs/PRUs/gNBs and transmits the dataset to UE/gNB side for AI/ML model training. Proposal 4: For case 1, case 2a and case 3a, if UE-side model and gNB-side model is trained at LMF side, LMF can collect a large-scale dataset from numerous UEs/PRUs/gNBs and transfer the trained AI/ML model to UE/gNB side. Proposal 5: For case 2b and case 3b, when LMF-side model is trained at LMF side, LMF can collect a large-scale dataset from numerous UEs/PRUs/gNBs for model training. Proposal 6: For the ground truth label generated by the UE/gNB based on non-NR and NR RAT-dependent positioning methods, the quality indicator for ground truth label can be used to determine whether the ground truth label meets the required quality for AI/ML model training. Proposal 7: When LMF side collects training data from UEs/PRUs/gNBs, LMF side can indicate the conditions or criteria such as the threshold of quality indicator. Proposal 8: For case 2b, if PRU/UE generates and reports new measurement such as CIR to LMF side as the input of AI/ML model, two-sided AI/ML model can be considered. Proposal 9: Ground truth labels and high-quality noise ground truth labels are used to monitor the AI/ML model performance: - Ground truth labels provided by the following entities in addition to PRU with known location: - The UEs with satellite positioning capability such as GPS or GNSS; - The UEs or network entity with high confidence degree positioning results based on existing RAT-dependent positioning methods; - High-quality noise ground truth labels provided by some UEs around the entities with ground truth labels. Proposal 10: The relative displacement between time T1 and time T2 estimated by motion sensor method can be used to monitor the performance of AI/ML model. Proposal 11: Regarding AI/ML model monitoring, if the AI/ML model is inferred and monitored at the same side, the following procedures for UE-side performance monitoring, gNB-side performance monitoring and LMF-side performance monitoring are considered: - UE-side performance monitoring: - For case 1 and case 2a with UE-side model, UE compares the results estimated by AI/ML model (e.g. estimated UE’s position, estimated timing/angle of measurement) with ground truth label or relative displacement estimated by motion sensor method for AI/ML model monitoring, and UE side makes decisions of model selection/activation/deactivation/switching/fallback operation then reports the decisions to gNB or LMF side; - gNB-side performance monitoring: - For case 3a with gNB-side model, gNB compares the results estimated by AI/ML model (e.g. estimated timing/angle of measurement) with ground truth label or relative displacement estimated by motion sensor method for AI/ML model monitoring, and gNB side makes decisions of model selection/activation/deactivation/switching/fallback operation. - LMF-side performance monitoring: - For case 2b and case 3b with LMF-side model, LMF compares the results estimated by AI/ML model (e.g. estimated UE’s position) with ground truth label or relative displacement estimated by motion sensor method for AI/ML model monitoring, and LMF side makes decisions of model selection/activation/deactivation/switching/fallback operation. Proposal 12: Regarding AI/ML model monitoring, if the AI/ML model is inferred and monitored at different sides, at least LMF-side performance monitoring should be supported. Proposal 13: Regarding AI/ML model monitoring, if the AI/ML model is monitored at LMF side and inferred at the other sides, the following procedures for LMF-side performance monitoring are considered: - For case 1 and case 2a with UE-side model: - UE reports the estimated results (e.g. estimated UE’s position, estimated timing/angle of measurement) to the LMF side for AI/ML model monitoring, and the LMF side make decisions of model selection/activation/deactivation/switching/fallback operation; - For case 3a with gNB-side model: - gNB reports the estimated results (e.g. estimated timing/angle of measurement) to the LMF side for AI/ML model monitoring, and the LMF side make decisions of model selection/activation/deactivation/switching/fallback operation. |
| [9, Baicells] | Observation 1: Assume the training process happens in a centralized fashion (e.g. at NW side), model input data (e.g. CIR) which usually has a large data size should be collected by UE or gNB and sent to NW. Then if the model input data is incorrect due to reasons such as synchronization error, or mismatching with the label, it would be a waste of bandwidth over the wireless/wired link. Observation 2: UE/gNB can make use of assistance information to enable data processing such as data binding, data validation/filtering, which has several benefits: Reduce the pre-processing workload of the network entity before model training. Improve the transmission effectiveness over the link. Suitable for scenarios when multiple PRUs or UEs are deployed collecting data in parallel, even for large-scale commercial deployment in the future. Observation 3: UE should report UE capability in advance if a AI/ML model needs to be transferred from network and deployed at UE. Proposal 1: Consider using assistance information to support data binding, data validation/filtering at UE/gNB side, to reduce incorrect model input data (e.g. CIR) transfer over wireless/wired link and achieve high transmission effectiveness as well as computation offloading from the network training entity. Assistance information may include: Geographical coordinates information of UE/gNB (with preserved UE privacy), environment information from sensors or other entities. Proposal 2: Support both quality indicator from the data collection entity and requested quality indicator from the training/management entity. Study how to represent quality indicator as a information type. Proposal 3: Assistance information such as time stamp, sequence number of the training data should be collected to support training data association, false data removal, thus ensuring high-quality training dataset generation. Proposal 4: Study the procedure of training data collection with time stamp/sequence number to facilitate training data pre-processing (e.g. data binding). Proposal 5: Consider out of distribution detection as a model monitoring method. Proposal 6: For Case 2b and Case 3b, the model and its monitoring are deployed within network side (e.g. LMF), hence model monitoring is up to implementation regarding monitoring decision. Proposal 7: Enhance LPP to support model monitoring decisions, as well as the transmission of model input related data, model output, and ground truth labels (e.g. UE position) . Proposal 8: Enhance NRPPa to support model monitoring decisions, as well as the transmission of model input related data and model output. Proposal 9: Study and define data format to support flexible training/inferencing data transmission with diverse assistance information. Proposal 10: Study and enhance LPP/NRPPa to support diverse variants channel observation measurements e.g. CIR/PDP as model input. Proposal 11: Discuss whether/how to indicate AI/ML capability of UE considering both training and inferencing aspects. If training is needed at UE side, training related system requirements (e.g. minimal training requirements) should be indicated via model information. Proposal 12: Discuss whether/how to indicate NG-RAN node AI/ML capability. If training is needed at gNB side, training related system requirements should be indicated via model information. Proposal 13: Regarding model transfer, geographical information e.g. the valid geographical boundary of the model can be attached as assistance information to better support UE-side model activation, switching, deactivation and performance monitoring etc. |
| [10, Sony] | Observation 1: The multiple paths reporting from UE/TRP to LMF could assist network-side (e.g., LMF) to make its own decision on LOS path selection. Observation 2: The procedure of AI/ML for positioning can be at least divided in three phases: - Data generation/collection with data processing and validation, - Model Training and updating, - Model deployment. Observation 3: The channel observation (e.g., in a form of CIR, SNR, RSRP) is used as part of the data generation/collection in the creation of training model Observation 4: Distributed learning model can achieve a better positioning accuracy based on training/inference by specific propagation channel environment. Proposal 1: Support AI/ML with model transfer in which the inference model is either in UE or gNB and LMF to create and train the AI/ML model. Proposal 2: Support the location information (e.g., location of PRU) and LOS information as the ground truth label. Proposal 3: Support UE generates ground truth label based on NR RAT-dependent and/or NR RAT-independent positioning methods. Proposal 3: Support channel observation as part of the data generation/collection from UE and gNB for downlink and uplink-based positioning, respectively. Proposal 5: Support AI/ML Positioning with model training at LMF and model inference at the UE side. Proposal 6: On AI/ML model indication, define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB. Proposal 7: Distributed learning model for positioning accuracy improvements can be considered. |
| [11, Fujitsu] | Observation 1 There are multiple entities which are able to generate training data (including labels) for AI/ML positioning models for each sub use case. Observation 2 Model ID and functionality ID can be both studied for further details, the co-existence of these two mechanisms is not precluded. Proposal 1 Entity capabilities information are important for AI/ML positioning data generation, it is suggested to enhance the current UE capability and other capability reporting mechanisms to support AI/ML positioning methods. Proposal 2 Study potential enhancements of at least the following existing signaling to determine the data collection entity for AI/ML positioning models per sub use cases: - SIB or pos-SIB - RRC Message - LPP - NRPPa Proposal 3 Study the potential enhancements of UE feature/behavior to support PRU information reporting. Proposal 4 Study the overhead requirements for configuration signaling and data transmission among entities per sub use case. Proposal 5 Study at least the following aspects for the data quality indication: - Data accuracy information - Data timing information - Data source information - Data applicability information Proposal 6 Not to preclude network entities for calculating monitoring metrics for at least UE-side AI/ML positioning models. Proposal 7 Study at least the following aspects for input-driven monitoring method: - Statistics (e.g., deviations or distributions) of input data obtained from different sources or positioning methods. - Additional information (e.g., timing, source, or imperfection level) of the input data. Proposal 8 Study specification impacts of model identification procedure for at least AI/ML positioning sub use cases 1 and 2a, and functionality identification procedure for sub use case 2a. Proposal 9 The format of the model identifier or functionality identifier is supposed to be studied in detail, the contents of the identifier are based on the model related information reported by UE, overhead issue should be considered during the reporting. |
| [12, xiaomi] | Proposal 1: UE capability related to the input collection, label collection should be studied Proposal 2: Consider label collection by UE or network by using position methods Proposal 3: For the AI model delivery to UE, delivering from LMF to UE is considered as the baseline Proposal 4: In AI-based positioning, features are defined from the perspective of output parameters Proposal 5: Functionalities are defined at least based on the factors which determine whether the functionality is workable or not Proposal 6: The monitor of the application condition or scenario could be considered for the performance monitoring Proposal 7: UE group-based LCM operation (e.g., model activation/deactivation/switch) can be considered Observation 1: The following options of collecting input and labels and corresponding specification impact on the air interface are identified - Model training on the network side - Option 1: UE/PRU collect the input and labels - Potential specification impact on the report the collected input and labels - Option 2: UE collect the input .The LMF generate the labels - Potential specification impact on the report of input - Potential specification impact on how to map the input from UE/PRU and labels from LMF - Option 3: TRP collect the input and UE/PRU collect the labels - Potential specification impact on the report of labels - Potential specification impact on the mechanism to maintain synchronized operation of input generation and label generation between different nodes - Option 4: Network entities generate the input and labels - Little specification impact on the air interface - Model training on the UE side - Option 1: UE collect the input and UE collect the labels - Little specification impact - Option 2: UE collect the input and LMF collect the labels. - Little specification impact Observation 2: Whether functionality identification is necessary depends on how the functionality is defined and whether the network want to control the functionality Observation 3: - For case 1 , no specification impact over the interface is foreseen for the inference phase - For case 2a, if the output parameter is exiting defined parameter, then little specification impact is foreseen. Otherwise, signalling to deliver the new parameter is needed Observation 4: For case 2b, new signalling to feedback the input of the inference may be needed for the inference phase Observation 5: For case3a - No specification impact over the interface is foreseen for the inference phase - Potential specification impact on the interface between gNB and LMF is expected if the output parameter is new parameters Observation 6: For Case 3b - No specification impact over interface is foreseen for the inference phase - Specification impact on the input report may be incurred between gNB and LMF Observation 7: - The performance monitoring can be separated as data collection, metric calculation and decision making and the following entities are possible - Data collection for performance monitoring - UE/TRP/LMF - Metric calculation - UE/TRP/LMF - Decision making - UE/ TRP - The following specification impact may be involved - Configuration of the performance monitoring, e.g., time occasions to perform the model monitoring - Request/report signalling of data collection and/ or metric calculation - Assistance signalling and procedure to facilitate the data collection or metric calculation - Signalling to inform/report the decision |
| [13, Google] | Proposal 1: The model monitoring for UE-side ML-based positioning should be transparent. Proposal 2: Study coverage enhancement for PRS to improve the measurement accuracy for CIR/PDP, which could be used as the input of ML based positioning. Proposal 3: Study aspects on CIR measurement and report Proposal 4: The model training in the NW-side ML-based positioning should be transparent. Proposal 5: The model monitoring for NW-side ML-based positioning should be transparent. |
| [14, LG] | Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE). Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE. Proposal #1: Consider AI/ML model fine-tuning or update based on model monitoring performance metric by taking into account the intermediate performance and output performance jointly. - Condition of fine-tuning or update with respect to a quality of intermediate/output performance Proposal #2: Consider the followings for potential specification impact on AI/ML model monitoring. - Assistance signalling for UE-sided model (e.g. distance between TRPs, beam information per TRP) - Contents of model switching/update (e.g. AI/ML model itself, AI/ML model parameter or structure only) Proposal #3: Consider the quality of ground truth label generation based on the measurement reports from TRPs for LMF/NW with known UE/PRU case. - Based on this, study AI/ML model training and monitoring methods further Proposal #4: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification at least for Case 2a - Assistance information: LOS/NLOS identifier with hard/soft value and the corresponding statistical information. Proposal #5: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication. Proposal #6: Consider PRS power control and PRS muting pattern on MTRP scenario via LOS/NLOS based PRS configuration. Proposal #7: Consider PRU prediction on LMF-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination at least for Case 2a/2b. Proposal #8: Consider a normal UE an entity used to obtain ground truth label based on the AI/ML based PRU prediction |
| [15, Samsung] | Observation 1: the use cases in which legacy positioning methods cannot work well could be prioritized to check whether AI based methods could work. Proposal 1: RAN1 to study the training data collection criteria, e.g., the qualified training device determination. Proposal 2: Current signaling framework of the measurement-report could be used as starting point to enable training data collection Observation 2: a UE claimed to be PRU is not always enough for data collection purpose. Proposal 3: PRU/UE/TRP could be used to generate the ground truth label under certain condition, including: whether the status of the PRU/UE/TRP matches the requirement of the model training, FFS details. Proposal 4: RAN1 to study the validation of the trained/obtained AI/ML model before actually apply it, consider following: - validity performance metric, e.g., positioning error between the model output (given input of PRU) and PRU’s location. - Validation data collection Proposal 5: other measurement metrics like L1 RSRP/SNR level could be considered as monitoring metric; Proposal 6: monitoring operation related aspects needs to be considered, including: - Potential monitoring specific resource determination - Monitoring procedure (e.g., initialization, periodic/a-periodic) Proposal 7: RAN1 to study the condition/methods to recovery/update a AI/ML model for positioning, e.g., event based condition or timer/counter based condition. |
| [16, CAICT] | Proposal 1: For NW-side AI.ML model training, data from some UEs with high reliability and accuracy location information could be considered for model update with UE transparent way. Proposal 2: For AI/ML model monitoring at UE/NW side, the comparison of positioning results from different sources and the deviation of positioning results over a period of time can be used. Proposal 3: In order to support AI/ML model monitoring at UE side, high accuracy positioning results from NW could be considered. Proposal 4: NW could provide area-based model monitoring results for AI/ML model update at UE. Proposal 5: For UE-based positioning with UE-side model, AI/ML model delivery from NW should be considered for Case1. Proposal 6: For AI/ML based positioning with UE-side model (Case 1 and 2a), full set for functionality identification could be defined. Flexible functionality reporting mechanism could be considered to allow partial elements reporting within the full set. Proposal 7: Proposed gNB set could be considered as part of assistant information to reduce AI model input. |
| [17, CMCC] | Proposal 2: For AI/ML based positioning, whether it is feasible to obtain the ground-truth labels via PRUs is related to the training dataset size. Proposal 3: For AI/ML based positioning, the potential spec impact of CIR report can be studied, and the impact of dimension of CIR on positioning accuracy can be evaluated in AI 9.2.4.1. Proposal 4: For AI/ML based positioning, the following two different options can be considered as the performance metrics for model monitoring based on model output. - Option1: The metrics of performance monitoring is based on the ground-truth labels - Option2: The metrics of performance monitoring is based on the results of traditional positioning techniques and/or the results of AI/ML model Proposal 5: For AI/ML based positioning, study the following model monitoring options for both UE-sided and LMF-sided model. - Atl1. UE-side model monitoring - Atl2. LMF-side model monitoring - Alt3. Hybrid model monitoring Proposal 6: For AI/ML based positioning, the relationship between model monitoring and positioning integrity can be considered. Proposal 7: For UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the reliability or the positioning accuracy should also be reported. Proposal 8: The data from PRU could be used for model monitoring. Proposal 9: UE could report its attitude and motion status to increase gNB awareness. |
| [18, MediaTek] | - UE-based positioning with UE-side model and direct AI/ML positioning has the potential to easily generalize only at UE side, with the help of the assistance information from NW. - In UE-based positioning with UE-side model and AI/ML assisted positioning, UE can report intermediate results to NW to speed up training and improve the performance of model monitoring. - The model for UE-based positioning with UE-side model and AI/ML assisted positioning can be trained by UE itself with small training effort and made UE-specific, which reduces the effort of LCM. - The model for UE-assisted/LMF-based positioning with UE-side mode and AI/ML assisted positioning can be trained by the UE itself and it can be UE-specific, which has a potential to generalize well even without model monitoring and update. - In UE-assisted/LMF-based positioning with LMF-side model and direct AI/ML positioning, LMF can be deployed with several models, each model can take its own input and has its own performance. UE/NW can choose what UE reports to the LMF and which model to use. - The number of fixed PRUs is limited in deployment. Moving PRUs could be considered as an alternative way collecting data. - A UE can be a PRU and a normal UE, depending on the UE capability and the positioning performance. If the UE can provide labeled data, the UE can be upgraded to PRU or downgraded back to facilitate the data collection. - A UE can provide training data even the positioning performance is not good, then the label data is not accurate or there are no labels at all. In this case, some method such as semi-supervised training can be used to improve the performance. - Moving PRUs need precise location information, so dedicated assistance information is needed to label data. - PRU should know its own locations if it labels data itself. - The existing measurement singling, and procedure like in LPP has been proved to be able to handle the requirements of various positioning methods. They can be also used directly or with some extension to facilitate the data collection and at the same time maintain the compatibility. - Depending on which entity needs data, and PRU/UEs’ data collection capability, and even there are other ways to transfer data, the training data collection procedure for each case could be different. However, there should be a core procedure that can be acted like a core module of every unique data collection procedure. - For Case 1 model inference, the data collection is just for a single UE and only a few measurements needed. The inference delay is also a key parameter that should be included in assistance information. - For UE-based positioning with UE-side model and direct AI/ML positioning, study the spec impact of fine-tuning only at UE side. - For UE-assisted/LMF-based positioning with UE-side model and AI/ML assisted positioning, study the spec impact of a UE-specific model without model monitoring and update. - For UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, study the spec impact of a model pool at LMF, where each model has its own inputs and performance. - Study the capability of a normal UE being upgraded to PRU and downgraded back. The upgraded UE could be assigned by NW as Auxiliary PRU (APRU), to distinguish it from already-have PRUs. - Study the granularity of UE capability of data collection, in terms of labels are present or not, and how much the label is impaired. - When a PRU is used to collect data, study the feasibility of PRU is moving and the related assistance information. - When a UE/PRU collects data, it should know its own location if it labels data itself. - Maintain the compatibility with existing measurement signaling and procedures when designing the data collection signaling and procedures. - Study a core data collection procedure at least for training data collection. This core procedure can be applied to various training procedures as a common procedure module. - Before training an AI/ML model, enough implementation imperfections should be introduced. The imperfections consist of channel estimation error, network synchronization error, UE and gNB timing error, etc. - Support to collect scenario identifier and related information (e.g., LOS probability) in training data collection. - For Case 1 model inference, data collection should consider narrowing down the measurements for a single UE and a specific model, and some inference information, e.g., inference delay of the model. - For Case1, Case 2a, Case2b, support collecting at least the following data: - PDP, or truncated PDP - If PDP is not enough, it can be CIR, or truncated CIR, or compressed CIR if UE performs CIR compression, extracting features from the CIR. - Enough implementation imperfections on PDP or CIR. - RSRP. - Horizontal location. - LOS/NLOS condition, TOA, DOA, and other intermediate metrics. - Scenario identifier - For model monitoring, study using model configurations (e.g., model ID, model version) as a way of monitoring models. - For model monitoring, study scenario monitoring (e.g., LOS probability monitoring, serving cell-based monitoring) as a way of monitoring models. - For model monitoring of direct AI/ML positioning, support the soft information associated to the estimated location as model output. - For direct AI/ML positioning with UE-side model and NW-side monitoring, the UE should report the estimated position and associated soft information to NW. - For positioning with UE-side model and UE-side monitoring, further study some simpler AI/ML model monitoring methods, e.g., serving-cell based monitoring and model configuration monitoring. |
| [19, Fraunhofer] | Observation 1: The AI/ML model for measurement enhancements can be trained on simulated data, the AI/ML model resulting from this use case can be generalized. Observation 2: For Positioning ML approaches trained with environment information a high accuracy is achievable, if the evaluation areas was covered by the training data. Proposal 1: Define the AI/ML model monitoring functionality w.r.t. fault indications applicable for the positioning use cases. Consider monitoring for at least the following fault indications: - Inference input and training data mismatch - Inference output inconsistency - Drop in QoS - AI/ML model/concept drift Proposal 2: The AI/ML model monitoring metric shall include information on the fault detection or fault diagnosis performed by the monitoring entity. Proposal 3: Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas. Proposal 4: Define new measurements for model inference input IQ reporting for the CIR. Proposal 5: Study in Rel-18 the following aspects to support AI/ML in challenging positioning environments: - Identification of AI/ML assisted areas - Additional signaling needed for making use of Virtual-TRPs - Temporal PRUs/anchors to enhance accuracy and maintain the AI/ML model |
| [20, NVIDIA] | Observation 1: AI/ML techniques can be used to learn the mapping of RF measurements to position. Proposal 1: High accuracy positioning in heavy NLOS scenarios should be the target of using AI/ML for positioning enhancement. Proposal 2: AI/ML techniques used to learn the mapping of RF measurements to position (i.e., direct AI/ML positioning) should be studied for positioning enhancement. Proposal 3: AI/ML techniques used to provide intermediate estimates such as LOS/NLOS classification (i.e., AI/ML assisted positioning) should be studied for positioning enhancement. Proposal 4: For AI/ML model training for positioning enhancements, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection at UE side or network side. Proposal 5: For AI/ML model training for positioning enhancements, study potential specification impact related to ground truth label determination and noisy level of the ground truth labels. Proposal 6: Study potential specification impact of different data collection methods (e.g., utilizing digital twin technology) for obtaining training data set with high user density. Proposal 7: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection. Proposal 8: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report. Proposal 9: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing for UE side or network side inference. Proposal 10: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model inference output and post-processing for UE side or network side inference. Proposal 11: For AI/ML based positioning enhancements, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring. |
| [21, InterDigital] | Observation 1: For Case 1, a potential specification impact is signaling from the LMF to the UE Observation 2: For Case 2a, a potential specification impact is signaling from the LMF to the UE Observation 3: For Case 2a, if new measurement types are specified, there may be specification impacts on measurement report from the gNB and LMF Observation 4: For Case 2b, Specification impacts related to signaling and measurements may be minimal or none Observation 5: For Case 3a, Signaling enhancements between gNB and LMF may be needed Observation 6: For Case 3a, if new measurement types are specified, there may be specification impacts on measurement report from the gNB and LMF Observation 7: For Case 3b, Specification impacts related to signaling and measurements may be minimal or none Observation 8: Ground truth label associated with UE location can be associated with quality/uncertainty of a location estimate Observation 9: Quality indicator for LOS/NLOS indicator is needed to generate a reliable training data set for AIML positioning Proposal 1: For UE-based inference generation, study a framework to initiate direct AI/ML positioning where the network can assist/trigger training and/or inference generation at the UE Proposal 2: For UE-based/assisted positioning, study a framework to initiate AI/ML assisted positioning where the network can trigger training and/or inference generation at the UE Proposal 3: For UE-assisted positioning and existing measurements (e.g., RSTD), study a mechanism to indicate whether measurements reported by the UE contain measurements estimated/predicted by the AIML Proposal 4: Study a framework to monitor for possible degradation in AIML performance Proposal 5: Study feasibility of deriving CIR based on measurements specified in Rel. 17 and identify missing measurements to derive CIR Proposal 6: Study how to indicate quality for CIR Proposal 7: Study direct AI/ML positioning where at least RSRP, RSRPP for PRS resources and RSTD are used as inputs for AI/ML models Proposal 8: Study AI/ML assisted positioning where timing measurements are generated based on RSRP fingerprints Proposal 9: Support different labels for information associated with PRU and non-PRU (e.g., normal UE) Proposal 10: Support labels associated with uncertainty of the ground truth Proposal 11: To develop a quality indicator for a LOS/NLOS indicator, support a verification procedure to verify the LOS/NLOS indicator generated by the UE or network. Proposal 12: Support to use the verified LOS/NLOS indicator as the ground truth during training |
| [22, Apple] | Proposal 1: Discussion of the details Model Capability, Indication and Configuration should wait till the a framework has been decided in the general session. For Model Training and Data Collection - Proposal 2: training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed - Proposal 3: Measurement Data (e.g. UE/PRU/TRP): Training data may be sourced from the actual network. The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may also depend on beam correspondence Support may be needed to enable a central data collection entity transfer data to the training entity.(e.g. from PRU to LMF to UE). - Proposal 4: assistance signalling and procedure for training data collection: The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may need positioning protocol based signaling to trigger feedback of training data to the training device. In addition, some assistance information may be needed for the availability and quality of noisy ground truth labels. - Proposal 5: Ground Truth Quality: The quality of the ground truth labels should be signaled to assist the selection and monitoring of the AI/ML model. - Proposal 6: Summary of specification impact for data collection: Ground Truth Label generation entity (RAN1 #112); Measurement entity (RAN1 #112); Measurement Data; Measurement quality (Ground Truth, measurement). Case 1 (Direct); PRU; PRU/UE; CIR, PDP, L1-RSRP; Ground Truth. Case 1 (UE-assisted); PRU; PRU/UE; CIR, PDP (TOA), CIR (NLOS); Ground truth. Case 2a; PRU; PRU/UE; CIR (TOA, NLOS); Ground Truth. Case 2b; LMF with know PRU location; PRU/UE; CIR, PDP, L1-RSRP; Ground Truth and measurement. Case 3a; Network entity with known PRU location; TRP; CIR (TOA, NLOS); Ground Truth. Case 3b; LMF with know PRU location; TRP; CIR, PDP, L1-RSRP; Ground Truth and measurement. Model Inference, Model Monitoring and Model Monitoring Response - Proposal 7: Monitoring Entity and Metric : For direct AI positioning or AI-assisted monitoring based on the estimated position, an accurate position ground truth obtained by the traditional location services of a UE, a PRU or a UE with GPS-based location services may be used to calibrate the AI-based location and vice versa. For AI-assisted positioning, intermediate KPI outputs may be used as the monitoring reference. For both direct AI positioning and AI-assisted positioning, the monitoring may be based on the properties and characteristics of the input e.g. a Doppler estimate on an input CIR may indicate the model is not appropriate. Entity to derive monitoring metric (RAN1 #112); Metric; Others (1); Others (2). Case 1 (Direct); UE, PRU; Ground Truth Difference; General statistics of input/output/scenario. Case 1 (UE-assisted); UE, PRU; Ground Truth Difference; Location Difference. Case 2a; UE, PRU; Ground Truth Difference; Location Difference. Case 2b; LMF; Ground Truth Difference. Case 3a; gNB; Ground Truth Difference; Location Difference. Case 3b; LMF (no gNB); Ground Truth Difference. - Proposal 8: Input for Inferencing and Monitoring : model input acquisition and pre-processing will depend on if the AI model is UE based, network based and on beam correspondence. An additional input capturing the parameters needed for monitoring (e.g. the ground truth) may be needed. A summary of the RS configuration and Feedback for the different cases is found below: RS Configuration; Feedback. Case 1 (Direct); PRS configuration; None. Case 1 (UE-assisted); PRS configuration; None. Case 2a; PRS configuration; TOA, NLOS. Case 2b; PRS configuration; CIR, PDP, L1-RSRP, quality. Case 3a; SRS configuration; TOA, NLOS. Case 3b; SRS configuration; CIR, PDP, quality. - Proposal 9: Update/ UE-network interaction: Monitoring Response: The Monitoring error may serve as input into Monitoring response. This response may be result in an action by the system including fallback to traditional methods, model retraining, model re-tuning or model switching. It could also result in a network-UE interaction model monitoring decision indication between UE and network. |
| [23, Lenovo] | Observation 1: MDT for normal UEs has already been utilised to collect UE measurement and location data for the purposes of network maintenance and operations by MNOs. Observation 2: For positioning, three entities in the RAN/CN require tight coordination and collaboration including LMF, NG-RAN nodes (serving and neighbouring gNBs) and the target-UE. Observation 3: Rel-17 focused on reporting enhancements for NLOS and multipath effects. Proposal 1: Training dataset acquisition, training dataset construction and actual training of a model may be considered as separate processes may not necessarily take place in the same entity. Proposal 2: Existing LPP/NRPPa signalling may be used to provide labelled/unlabelled data indication to different UEs/network entities. Proposal 3: Data labels used for AI/ML positioning may include at least location information and timing information associated with each collected data point as well as Label quality in at least Cases 1-2b. Proposal 4: Support Direct AI/ML and Assisted AI/ML positioning configurations for RAT-dependent and RAT-independent positioning measurements to generate training data. FFS the differences between existing non-AI/ML and AI/ML positioning configurations, e.g., provision of reference locations along with the measurement configuration. Proposal 5: Evaluate schemes related to transfer of positioning-dataset for different stages of the LCM. Proposal 6: Evaluate the following schemes for transfer of positioning-dataset: - Alt. 1 - Proprietary signaling. The Positioning-dataset is transferred without specification impact using non-3GPP technologies - Alt. 2 - Positioning-dataset transfer using 3GPP-signaling. Proposal 7: For NG-RAN node assisted positioning, further study the role of reference TRPs/PRU as TRPs and UEs as data sources to extend data collection in a distributed manner for Cases 3a and 3b. Proposal 8: In order to increase flexibility of the derivation of the model monitoring metric, support PRU UE as an entity for Cases 1 and 2a, while support gNB as an entity for case 3b. Proposal 9: RAN1 to further confirm that the entity deriving the model monitoring metric is the same entity that may also perform model monitoring. Proposal 10: Support model monitoring using model output statistics with and without ground truth label information. Proposal 11: Support model transfer using associated Model IDs and meta-information to help facilitate the transfer to other network entities/UEs. Proposal 12: Support new measurements such as CIR and PRS RSSI for model inference inputs, while existing Rel-16/Rel-17 positioning measurements may also be re-used. Proposal 13: Support new measurement report elements such as TOA for both PRS and SRS while existing Rel-16/Rel-17 positioning measurement reporting elements may also be re-used, e.g., RSTD, LOS/NLOS indicator, RSRPP, etc.. Proposal 14: Consider enhanced RS configurations to enable AI/ML positioning measurements for model inference, e.g., mechanism to measure PRS at specific reference locations. Proposal 15: Consider the following additional aspects with respect to the network-UE collaboration levels y and z including the associated sub-levels: - Data collection for training/inference - Model Life Cycle Management (including model acquisition, activation/deactivation of AI/ML models, model monitoring and update at the LMF, serving and neighbouring gNBs, and target-UE) - Model inference Proposal 16: Further study mechanisms to enable efficient positioning AI/ML model transfer between UE, gNB and LMF. Proposal 17: Study fingerprinting under the Direct AI/ML positioning sub-use case, whereby channel observations/RS measurements, e.g., CIR, RAT-dependent and RAT-independent positioning measurements serve as unique RF signatures to train an AI/ML model to determine the target-UE’s location estimate. Proposal 18: Further study fingerprinting in at least in following cases, where inference is being performed: - Case 1: UE-based positioning with UE-side model - Case 2b: UE-assisted/LMF-based positioning with LMF-side model - Case 3b: NG-RAN node assisted positioning with LMF-side model Proposal 19: RAN1 to consider LOS/NLOS identification under the AI/ML assisted positioning sub-use case for timing-based and angular-based positioning techniques, where the input data may comprise of all currently supported DL-based, UL-based, (DL+UL) measurements and the corresponding output comprises classification of measurements in terms of LOS and NLOS. Proposal 20: Further study LOS/NLOS identification at least in terms of: - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning |
| [24, Qualcomm] | Observation 1: A combination of the defined cases (i.e., Case1 to Case3b) may be used to achieve improved positioning in a defined setting. Observation 2: Model development is best to be done by the vendor who implements the device where the AI/ML model inference runs. Observation 3: Procedures for moving training data (i.e., measurements, ground truth labels, assistance information) from source entities to training entities is out the scope of RAN1 and can be studied by RAN2/3 and SA. Observation 4: For data collection in Case3a, gNB/TRP can schedule UE to send SRS positioning resources. Observation 5: For data collection in Case2b and Case3b, LMF can use existing LPP and NRPPa configuration and reporting procedures for configuring resources for data collection. Observation 6: For data collection in Case2b and Case3b, LMF can leverage information obtained using existing LPP and NRPPa procedures to compute ground truth labels. Observation 7: The following are information to be associated with data collected at UE/PRU side: - Timestamping of measurements and labels: UE side needs to tie labels with their corresponding measurements (e.g., when labelling is provided from LMF). Examples of timestamping includes UTC timing and/or indices (i.e., SFN, slot, OFDM) of resources used to compute the label. - Indication of how PRS resources map to physical anchor location/angles: UE side need to know how different PRS resource sets/resources are mapped to physical anchor location and beam angles. - Indication of timing errors at network side: UE side can benefit from knowing expected ranges/distributions/indexing of timing errors at network side (e.g., TRP TX timing errors, inter-TRP synchronization errors) for better training and model development. - Indication of LOS/NLOS maps for each TRP: UE side can benefit from knowing the map of potential LOS/NLOS states for each combination of TRP and PRS resource (if available from LMF side). Observation 8: Monitoring measurements and their labels can be available at LMF side with help of PRUs. LMF may share both measurements and labels (location or intermediate quantities) with the side handling model management (e.g., UE-side or gNB-side). Observation 9: For Case2b and Case3b, LMF can do label-based model monitoring with help of existing measurements and reporting, and it is not expected to incur additional specification impacts. Observation 10: For inference in Case1/Case2a, both measurements and model are obtained at UE side and no strong need to specify model input measurements. Observation 11: For inference in Case2b, reporting complex CIR measurements from UE to LMF incurs high OTA reporting overhead and need to be deprioritized. Observation 12: For inference in Case2b, at a given reporting overhead, reporting multipath measurements achieves higher accuracy than reporting an optimized complex CIR measurement. Observation 13: For inference in Case3a, both measurements and model are obtained at gNB/TRP side and no strong need to specify model input measurements. Observation 14: For inference in Case3b, reporting has less dependence on OTA resources and can include both existing measurements and new measurements. Observation 15: For inference in Case2a/Case3a, UE/TRP may report new measurement reports such as soft info to timing and angle to help LMF improve positioning accuracy. Proposal 5: Consider the following additional entities and mechanisms to generate ground truth label: - For direct AI/ML positioning, ground truth label is UE location - Case1 - UE generates location based on non-NR and/or NR RAT-dependent positioning methods with network assistance (as applicable). - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameters(s) corresponding to AI/ML model output - Case1/Case2a - UE generates label directly or calculates based on measurement/location with network assistance. Proposal 6: For data collection in Case1 and Case2a, consider UE/PRU to request, from LMF, configuring PRS resources as one of the assistance signaling for training data collection. Proposal 7: For data collection in Case1, Case2a, and Case3a, consider signaling for labelling assistance from LMF. Proposal 8: Consider the following associated information for data collection at UE/PRU (Case1 and Case2a): - Timestamping of measurements and labels: E.g., timestamping includes UTC timing and/or indices (i.e., SFN, slot, OFDM symbol) of resources used to compute the label. - Indication of how PRS resources map to physical anchor location/angles - Indication of timing errors at network side: E.g., TRP TX timing errors, inter-TRP synchronization errors. - Indication of LOS/NLOS maps for each TRP: E.g., map of potential LOS/NLOS states for each combination PRS/TRP (if available from LMF side). Proposal 9: For AI/ML positioning model monitoring (Case1 to Case3b), study the following aspect to enable model monitoring: - Model monitoring based on joint model input and output (ground truth-based monitoring): Ground truth label and AI/ML model input measurements can be made available at monitoring/model management side Proposal 10: For Case1 and Case2a, the following are required signaling for label-based monitoring: - signaling for letting UE side (or model management entity of UE side) request monitoring data from LMF and providing monitoring data from LMF to model management entity of UE side - signalling for letting UE side (or model management entity of UE side) and LMF communicate monitoring outcome/metric back and forth (if needed). Proposal 11: For Case3a, the following are required signaling for label-based monitoring: - Signaling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF request/report monitoring measurements - Signalling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF communicate monitoring outcome. Proposal 12: For inference in Case1/Case2a, no need to specify type of measurements. Proposal 13: For inference in Case2b, deprioritize CIR/PDP as new measurements due to their significant reporting overhead and minimal/comparable gain when compared to existing multipath reporting. Proposal 14: For inference in Case2b, type of measurements reported from UE to network are either existing measurements or minor enhancements of existing measurements. Proposal 15: For inference in Case3a, no need to specify type of measurements. Proposal 16: For inference in Case2b, study type of measurements while including both existing measurements, enhancements of existing measurements, or new measurements. Proposal 17: For inference in Case2a/3a, consider existing/enhanced measurements and new measurements (e.g., soft-info of time/angle) Proposal 18: Study the specification impact for the reporting of soft information associated with positioning measurements, derived using machine learning. |
| [25, NEC] | Proposal 1: Collaboration level x is recommended for UE-based positioning with UE-side model (case 1) for both direct AI/ML positioning and AI/ML assisted positioning. This is because collaboration level x is more likely to support the use of AI models, which have the potential to replace traditional position calculation methods and do not require any changes on the network side. Proposal 2: Collaboration level x and z is recommended for UE-assisted/LMF-based positioning with UE-side model. This is because collaboration level y and collaboration level z enables the LMF to access the information from the AI/ML model on the UE side, which can improve the accuracy of the positioning calculation. Proposal 3: Collaboration level x, y and z all are suitable for case2b, UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning. Proposal 4: Collaboration level x, y and z all are suitable for case 3a, NG-RAN node positioning with gNB-side model, AI/ML assisted positioning. Proposal 5: Collaboration level x, y and z all are suitable for case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. Proposal 6: RAN1 should specify an information interaction mechanism to assist the entities (UE/PRU/gNB/LMF) in collecting a suitable and balanced dataset from other entities where the data is transferred at the physical layer or higher layer. Proposal 7: Provide a mechanism to improve the reliability of model monitoring by requiring the entities that collect field data for AI/ML model positioning to indicate the reason why the AI/ML model was triggered. This information can be used to initiate the appropriate positioning procedure for field data collection, ensuring that the collected data is relevant to the reason for triggering the AI/ML model. |
| [26, NTT DOCOMO] | Proposal 1: For direct AI/ML positioning (case 2b and 3b), study the report format of UE/gNB reporting CIR/PDP to LMF considering the signaling overhead reduction. Proposal 2: The generation of ground truth label by UE/NW with RAT-dependent positioning methods could be considered with lower priority. - If high confidence level estimation can be achieved or quality indicator is reported, this method could be feasible. Proposal 3: For training data collection for AI/ML based positioning on UE side, DL RS (e.g., PRS) configuration dedicated for the data collection can be considered. Proposal 4: Additional parameters can be configured in DL RS configuration for data collection purpose, - E.g., associated measurement information (e.g., quality indicator, timing information, … of each measurement) and scenario/environment information (e.g., deployment scenario, TRP info., UE distribution, …), etc. Proposal 5: For performance monitoring of AI/ML based positioning, - For case 1,2a, prioritize the performance metric calculation on UE - For case 2b,3b, prioritize the performance metric calculation on LMF - For case 3a, performance metric calculation on both gNB and LMF can be considered Proposal 6: For case 1 and 2a, when monitoring entity is UE, UE should calculate monitoring metric following NW indication if NW makes decision of upcoming model operation based on performance monitoring. - The indication includes at least model ID/functionality, monitoring type (e.g., input-based, output-based), performance metrics/threshold. Proposal 7: For the decision of upcoming operations/LCM procedures, - For model/functionality activation/deactivation/switching/selection/fallback operation, - Regarding model ID-based LCM, NW should determine the upcoming operation at model level scale - Regarding functionality-based LCM, NW should determine the upcoming operation at functionality scale - For model update, the upcoming operation is determined by the training entity. Proposal 8: For the decision of upcoming operations/LCM procedures (e.g., model/functionality update) by UE, - Regarding functionality-based LCM, UE should report the decision to the NW at the functionality scale - Regarding model ID-based LCM, UE should report the decision to the NW at model level scale Proposal 9: The indication of AI/ML functionality includes at least functionality information, which includes - The nominal output of the AI/ML model - Essential information to derive the model inference - Validity condition for the functionality Proposal 10: Regarding LCM for AI/ML based positioning, ‘Network’ may be replaced by ‘LMF’ |

## Data generation and collection

In RAN1#112, the following were agreed.

Agreement

Regarding training data generation for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified

  - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)

  - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label

    - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

    - Network entity generates ground truth label based on positioning methods

- The following options of entity to generate other training data (at least measurement corresponding to model input) are identified

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

- Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study

Agreement

Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects

- Associated information of training data

  - Quality indicator at least for ground truth label (if needed)

  - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc.

- Assistance signaling and procedure to facilitate generating/collecting training data

  - Potential determination of the UE/PRU/TRP which can provide the training data

  - Configuration of reference signal (for measurement and/or label)

  - Signaling other than above 2 for data collection

    - E.g., requested quality of training data

Regarding the FFS point in the above first agreement (whether to support UE/network entity generate ground truth label based on positioning methods), many companies provided further inputs.

For ground truth label collection, to improve the quality of labels, [4, vivo] proposed to indicate UE the criteria or requirement for data labeling or indicate UE to report label quality indicator. For measurement collection, at least measurement type and format should be indicated, depending on to model input. [4, vivo] also proposed that both PRUs and regular UEs can be used to perform data collection.

[5, OPPO] proposed that regarding the data collection for AI model training, NOT support UE to report the ground-truth labels of its location(s).

[8, CATT] proposed that in addition to PRU, the ground truth labels are generated by UEs with non-NR positioning capability such as GPS or GNSS and/or UEs or network entity with high confidence degree positioning results based on existing NR RAT-dependent positioning methods.

[10, Sony] proposed to support UE generates ground truth label based on NR RAT-dependent and/or NR RAT-independent positioning methods.

[12, Xiaomi] proposed to consider label collection by UE or network by using position methods.

[14, LG] proposed to consider the quality of ground truth label generation based on the measurement reports from TRPs for LMF/NW with known UE/PRU case and to consider a normal UE an entity used to obtain ground truth label based on the AI/ML based PRU prediction.

[15, Samsung] proposed that PRU/UE/TRP could be used to generate the ground truth label under certain condition, including: whether the status of the PRU/UE/TRP matches the requirement of the model training.

[16, CAICT] proposed that for NW-side AI.ML model training, data from some UEs with high reliability and accuracy location information could be considered for model update with UE transparent way.

[17, CMCC] thought for AI/ML based positioning, whether it is feasible to obtain the ground-truth labels via PRUs is related to the training dataset size.

[18, MediaTek] proposed to study the capability of a normal UE being upgraded to PRU and downgraded back. The upgraded UE could be assigned by NW as Auxiliary PRU (APRU), to distinguish it from already-have PRUs.

[21, InterDigital] proposed to support different labels for information associated with PRU and non-PRU (e.g., normal UE) and to support labels associated with uncertainty of the ground truth.

[24, Qualcomm] proposed that for case 1 and 2a, UE generates location (label) based on non-NR and/or NR RAT-dependent positioning methods with network assistance.

[26, NTT DOCOMO] proposed that the generation of ground truth label by UE/NW with RAT-dependent positioning methods could be considered with lower priority. If high confidence level estimation can be achieved or quality indicator is reported, this method could be feasible.

Moderator’s observation and comment:

Companies’ view on whether to support UE and/or NW entity generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods for AI/ML based positioning.

Yes: [4, vivo], [8, CATT], [10, Sony], [12, Xiaomi], [14, LG], [15, Samsung], [16, CAICT], [18, MediaTek], [21, InterDigital], [24, Qualcomm], [26, NTT DOCOMO] (lower priority, feasible if high confidence level estimation can be achieved or quality indicator is reported)

No: [5, OPPO]

Regarding using UE to generate ground truth label at least for case 1 and 2a, as summarized above, [5, OPPO] is the only company which does not support to use UE to generate ground truth label for the concern of performance loss compared to the ideal ground truth label while many other companies proposed that UE can be used to generate ground truth label with further study on label quality. Moderator formulate the following proposal given this vast majority view from companies.

Proposal 1-1-1

Regarding ground truth label generation for AI/ML based positioning, the following options of entity to generate ground truth label are identified (in addition to entities from previous agreement)

- UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

  - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - FFS applicable conditions and potential specification impact, e.g., required label quality, necessary assistance signaling to UE

- Network entity generates ground truth label based on positioning methods

  - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b),  NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - FFS applicable conditions and potential specification impact (if any)

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Fujitsu | We agree with DCM that non-NR and/or NR RAT-dependent positioning methods can be applied only if high credibility or quality indicator are reported. |
| CMCC | We think that for the UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the label quality should be reported. |
| NEC | Support. Case 2b can also be concluded, i.e., the UE generates a measurement (CIR/RSRP/AOD/TDOA/…) by RAT-independent methods, and send measurements to the positioning server for position inference by AI/ML model. |
| NOK | OK with the direction of the proposal. However, we should consider that the labeling should be somewhat controlled e.g. use only those labels that have a max target uncertainty, or combine labels from multiple sources. In the specific case of NR RAT-dependent positioning methods, this method will ensure that the AIML method will learn to emulate the legacy positioning method (i.e. makes same mistakes as legacy), and there is a risk that the AIML method will be just as good as the legacy, but being more complex. Conversely, if we consider that each observation can have multiple labels e.g. a GNSS loc, a Wifi location, then the AIML final label can be a combination of the multiple labels. |
| Ericsson | Do not support - For scenarios that existing positioning methods works, AI/ML is likely not needed. - For scenarios that existing positioning methods do not work (e.g., heavy NLOS), it is not possible to use them to provide ground truth label. Evaluation results by OPPO R1-2302544 show that if using existing methods to provide label, then AI/ML performance is just as bad as existing methods. |
| Lenovo | Supportive of FL’s proposal and ok with the FFS including label quality on the first bullet |
| NVIDIA | Support the proposal in principle. |
| LG | Fine with the proposal |
| vivo | Support. The nature of AI/ML relies on large amount of data. This proposal gives flexibility to enable more entities for data collection. Respond to Ericsson’s argument. When UE report the ground truth label with a quality indicator, it's up to the training entity whether to use that training data or not. If that quality indicator indicating a good enough ground label in some scenarios, why to prevent such opportunity? It is Ericsson’s opinion that AI/ML is likely not needed for scenarios that existing positioning methods works. However, we believe AI/ML based positioning can be applied for a variety of scenarios not just for heavy NLOS. We don’t see the SID limit the usage of AI/ML positioning to a particular scenario. |
| Intel | Support the proposal. |
| Samsung | Supportive of FL’s proposal and ok with the FFS including label quality on the first bullet. To following E///’s comment, we think LOS/NLOS level is per UE based, in the same factory, some UE might in heavy NLOS state, but some other UE may not. We use heavy NLoS scenario is for the evaluation convenience. They will be a certain percentage of UE can still have relatively good condition. |
| CATT | Agree. We also think only the labels with high quality/confidence should be collected. Additionally, supporting this proposal does not mean this becomes the only way for ground truth label collection. |
| CAICT | Support FL’s proposal. |
| NTT DOCOMO | Support the proposal. The required label quality in the dataset generated by UE/NW should be further discussed. |
| Xiaomi | Support the proposal |
| OPPO | Agree with Ericsson. As shown in our evaluation, if the label for the training data set of AI model is obtained by traditional NR DL-TDOA scheme, the AI model inference performance will suffer larger performance loss for clutter parameters {60%, 6, 2} and {40%,2, 2}. It seems how to achieve high quality label based on existing positioning methods and only collect high quality label (with the help of quality indicator) for training are important. We think that the feasibility/performance need to be evaluate first. |
| HW/HiSi | For Case 2b and Case 3b the mechanism seems clear, but a question for clarification in the on the cases: For Case1 and Case 2a, the AI/ML model is at the UE side. And the output of the inference will be e.g. LOS states or TOAs. If other UE provide this information to the UE side model for training, then it needs to be discussed how measurement results and labels are transferred to the UE-side model. Can this please be clarified? For Case 1 with direct positioning, other UEs would need to disclose their position, we are not sure if this is acceptable from the privacy perspective. |
| ZTE | Support the proposal. Agree with other companies, if non-NR and/or NR RAT-dependent positioning methods, the high quality data is required for model training. |
| Baicells | Support. |
| Apple | Do not support especially for the UE generated GT. Agree with Ericsson and Oppo. Also, concerns that a UE has to expliclity opt-in for this to be valid if agreed. |
| Fraunhofer | Supportive the proposal and ok with the FFS including label quality on the first bullet. Reliability of the AI/ML positioning will benefit from more information. Additionally, we see that the non-RAT methods in future releases may support high accuracy methods (like UWB) which can make it applicable even in challenging NLOS scenarios. |
| InterDigital | We think that labels can be associated with PRU or normal UE, and similar to CMCC view, these labels should be accompanied with an uncertainty level/quality indicator. |
| Moderator | To Huawei: This proposal is about the entity generating ground truth label. If UE generates for Case 1 and 2a, it may transfer that to a training entity. To Huawei and Apple: Right now, the discussion on training data is about ground truth label and measurement. It does not mean training data collection would require UE ID with those training data. To Ericsson and OPPO: The FFS point is there to address the concern on label quality. To all: Wording update into proposal 1-1-1a below. |

##### Proposal 1-1-1a

Regarding ground truth label generation for AI/ML based positioning, the following options of entity to generate ground truth label are identified (in addition to entities from previous agreement)

- UE generates ground truth label when label quality satisfy the requirement

  - based on non-NR and/or NR RAT-dependent positioning methods

  - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - FFS potential specification impact, e.g., the required label quality, necessary assistance signaling to UE

    - Note: user data privacy needs to be preserved

- Network entity generates ground truth label when label quality satisfy the requirement

  - based on positioning methods

  - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b),  NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - FFS potential specification impact, e.g., the required label quality

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | Support. The argument of objecting company on “NLoS-heavy UE should not be able to generate ground truth label” should not apply here. NLoS heavy is just to describe the evaluated scenario. It doesn’t mean all UE is under that condition. It doesn’t mean that scenario is only target for AI based method. |
| NEC | We support the proposal in principle. However, we have two comments. Firstly, we would like to know if the moderator intends to limit the methods for the second bullet point. Will it only include RAT-dependent methods(our preference), or both RAT-dependent and RAT-independent methods? It should be clarified at least. Secondly, Case 2b can also be covered in the first bullet point, where the UE generates the label (UE location) through RAT-independent methods such as GNSS, WIFI, Bluetooth, etc., and sends the labels to the positioning server for position inference by an AI/ML model. Therefore, we suggest modifying the proposal to ##### Proposal 1-1-1a Regarding ground truth label generation for AI/ML based positioning, the following options of entity to generate ground truth label are identified (in addition to entities from previous agreement) - UE generates ground truth label when label quality satisfy the requirement - based on non-NR and/or NR RAT-dependent positioning methods - At least for UE-based positioning with UE-side model (Case 1), and UE-assisted positioning with UE-side model (Case 2a), and UE-assisted positioning with LMF-side model (Case 2b) - FFS potential specification impact, e.g., the required label quality, necessary assistance signaling to UE - Note: user data privacy needs to be preserved - Network entity generates ground truth label when label quality satisfy the requirement - based on NR RAT-dependent positioning methods - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - FFS potential specification impact, e.g., the required label quality |
| Ericsson | Do not support. Based on generalization evaluation, AI/ML based positioning does not generalize well. This is a big disadvantage compared to existing positioning methods. It’s very complex to ensure that the model would work, e.g., model training, model monitoring, model update/fine-tuning/re-training/switching. In a word, it’s costly to use AI/ML compared to existing methods. Thus it make sense to use AI/ML based method only when existing methods fail. With this understanding, the label quality by existing method does not satisfy the requirement. In general, the positioning accuracy achievable by the AI/ML method is worse than the label quality. If existing positioning method provides label, then this guarantees that AI/ML method will have worse accuracy than existing positioning method. What’s the reason to use AI/ML then? |
| ZTE | Support |
| Xiaomi | Support |
| NTT DOCOMO | Support |
| MediaTek | Support |
| Fujitsu | Support. |
| CATT | Support |
| vivo | Support On Ericsson’s comment, we don’t share such logic. It’s costly to upgrade 5G network, should we stop development for future 6G? In Ericsson’s own evaluation, it is observed that “Different ML positioning approaches can exhibit different levels of sensitivity to labeling errors. Semi-distributed ML assisted positioning approaches exhibit lower sensitivity to labeling errors than centralized ML positioning approaches.” In many evaluation contributions, companies have shown that AI/ML positioning is robust to some extent of labelling error. Ericsson cited OPPO’s results of DL-TDOA. That’s one method in one evaluation case of heavy NLOS. There’re other non-NR and/or NR RAT-dependent positioning methods for other scenarios. Furthermore, we have not decide on the requirement. But it seems Ericsson already made the judgement regardless of the requirement. The SID does not limit the study where AI/ML positioning can only be applied to heavy NLOS. |
| InterDigital | Support |
| Lenovo | Supportive of FL’s revised proposal. The proposal allows for more flexibility in terms of ground truth label generation whenever the label quality meets the desired requirements. |
| NOK | Support. We only suggest adding the following note: Note: The combination of both entities (UE and NW) generating ground truth for the same sample is not precluded. |
| Intel | Support the proposal. |
| CAICT | Support. |
| LG | Fine with the updated proposal |
| OPPO | Agree with Ericsson. According to the evaluation results by companies, the positioning accuracy by AI/ML model is approximate or worse than label error L (truncated Gaussian distribution with zero mean and standard deviation of L meters). In the real deployment, when a normal UE (other than PRU) generates label using NR existing positioning methods, we are not sure how could the UE decide label quality satisfy the requirement. Besides, the scenarios with heavy NLOS conditions are not the only target scenarios for AI/ML positioning, but very important application scenarios. |
| Qualcomm | The requirement on label quality is too restrictive. Noisy ground truth can still be helpful for weakly supervised and semi-supervised approaches. We suggest to take off the red part (i.e., “when label quality satisfy the requirement”). The proposal already has the FFS on required data quality. |
| CMCC | Support. |
| HW/HiSi | Support. |
| Fraunhofer | Support |
| Moderator | Status of companies’ view (based on comment to proposal 1-1-1 and/or 1-1-1a): Support: Fujitsu, CMCC, NEC, NOK, Lenovo, NVIDIA, LG, vivo, Intel, Samsung, CATT, CAICT, NTT DOCOMO, Xiaomi, ZTE, Baicells, Fraunhofer, InterDigital, HW/HiSi, Qualcomm (w/o label quality requirement in the bullet) (20) No: Ericsson, OPPO, Apple (3) To Qualcomm: Even for weakly supervised and semi-supervised learning, I assume we may still have some requirement on label quality, which may or may not be the same for all scenarios/cases. As you said, the detail of it is FFS. To NOK: Currently, we don’t have any agreement/conclusion to exclude that if this proposal were agreed even without your proposed note. To NEC: On your first comment, it seems you are referring to model inference “UE generates the label (UE location) through RAT-independent methods such as GNSS, WIFI, Bluetooth, etc., and sends the labels to the positioning server for position inference by an AI/ML model” for case 2b. I don’t think we need label for inference. Note that this proposal is about label generation for training and/or monitoring etc. If you are indeed meant for training, data transfer is a must when UE generate label while the model is on LMF-side. I believe that should be further discussed. On your 2nd question, my understanding is that typically 3GPP does not limit what a network entity can or cannot do. Given RAN1 does not decide which exact entity for that function, it is safe to live to other WGs. |
| Ericsson | We still don’t get the logic of this proposal at all. If this proposal works, then the non-AI/ML positioning method is able to provide ground truth which AI/ML attempts to match. This means the performance of non-AI/ML method is superior to AI/ML method. Then, what’s the reason to introduce AI/ML? About the label quality criteria: how to guarantee the label quality indicator is reliable? Since the TRUE label is not known, how to reliably judge that the estimated UE location by the conventional algorithm is within X meters of the TRUE label? |
| Samsung | Regarding E///’s comment, we have two different understanding, - Let’s say in an environment, which is more like the NLoS-heavy scenario, there are quite large percentage of link between TRP and UE is NLoS, but this did not prevent there are still certain number of UE can have LoS link to TRP, maybe not all link for these UE are NLoS, so that they can still achieve good enough positioning accuracy (by integrity check with legacy methods), these UE and their corresponding positioning estimates can still be used label; AI/ML will not be used NLoS only case, because in real deployment, there is no clear boundary on what is counted as NLoS-heavy and even such determination is changing since the UE might be moving as well. So the label could be based on the UE with good enough estimate in the same coverage of cell. |
| Moderator | To Ericsson:: This proposal is saying UE/NW can generate label in addition to what we have agreed. For example, PRU generates some theoretically TRUE label, while UE can also generate label (when satisfying requirement). A model can be trained to utilize both of them. I don’t see why restrict to only one type of label. On your argument, “If this proposal works, then the non-AI/ML positioning method is able to provide ground truth which AI/ML attempts to match. This means the performance of non-AI/ML method is superior to AI/ML method.” I don’t agree with that. OPPO’s evaluated two options: all labels are from existing DL-TDOA and 50% ideal + 50% from DL-TDOA. The latter option has much better performance than the former option and convention DL-TDOA. Many companies already evaluated label error impact where observation (accuracy in proportion to label error) is discussed in 9.2.4.1. A model can be trained with different label quality to obtain a given positioning accuracy target. On your question of label quality criteria, that’s part of FFS. In existing specification, we already have those quality for measurement report even when UE has no TRUE information. E.g., nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. |
| CATT | Support. Do not see the harm for UE/Network to generate high-quality label even if by chance. And let’s say, non-AI/ML method may perform well in some time/scenario, but this is not always valid. UE is moving, or the radio environment is changing. Generating labels for model training for a rainy day is still meaningful. On quality criteria, it is safe since it is also FFS. |
| OPPO | Sorry for the late response. The evaluation results of most companies show that the positioning accuracy by AI/ML model is approximate or worse than label error L (when the label error is based on truncated Gaussian distribution with zero mean and standard deviation of L meters). For example, the results from vivo, MTK, NVIDIA, IDC, Nokia, SS. We also copy the evaluation results below for reference. That is to say the error of AI-based positioning are not improved compared to the label error. It also means, if the labels are obtained by a scheme, the AI-based positioning cannot achieve better performance than this scheme, no matter this scheme is legacy NR positioning or is non-RAT based positioning. About the label quality criteria, we share same view with Ericsson. Regarding the evaluation option “50% ideal + 50% from DL-TDOA.” in our contribution, which is “40000 ideal labels + 40000 error labels based on DL-TDOA”, the positioning accuracy is 4.63m. That is still a large performance loss. If we only use 40000 ideal labels are training as copied below, the positioning accuracy is 0.38m. That is to say the labels obtained from DL-TDOA would harm the performance. |
| Moderator | To OPPO: The results you quoted observed that “the positioning accuracy by AI/ML model is approximate or worse than label error L” when ALL labels for training having the same label error. You didn’t copied your results of labeling error with 100% DL-TDOA. That’s more than 8 meter, compared to 4.63m for 50% ideal + 50% DL-TDOA. So your own results showed that 50% ideal + 50% DL-TDOA is better than 100% DL-TDOA. Yes, labeling error degrade performance compared the ideal case. But training data with say 50% ideal + 50% w/ L = 1 most likely still provide positioning accuracy within 1 m. A model can be trained with different label quality to obtain a given positioning accuracy target. You argued that “if the labels are obtained by a scheme, the AI-based positioning cannot achieve better performance than this scheme, no matter this scheme is legacy NR positioning or is non-RAT based positioning” assuming AI model is trained on all labels from a scheme (with all the same labeling error). See also my reply to Ericsson, I don’t see why we should put any restriction on what type of label quality for model training when it can select the desired quality for a particular accuracy target. |
| Apple | Support with the data privacy sub-bulletl. |
| Moderator | Updated status of companies’ view (based on comment to proposal 1-1-1 and/or 1-1-1a): Support: Fujitsu, CMCC, NEC, NOK, Lenovo, NVIDIA, LG, vivo, Intel, Samsung, CATT, CAICT, NTT DOCOMO, Xiaomi, ZTE, Baicells, Fraunhofer, InterDigital, HW/HiSi, Qualcomm (w/o label quality requirement in the bullet), Apple (21) No: Ericsson, OPPO (2) |

On the potential specification impact for data collection, multiple companies expressed their views.

[1, Ericsson] proposed that for training data collection of all Cases, data format information (e.g., granularity factor k) is reported together with the timing related measurement data. For Case 1/2a, for model inference as well as training data collection, [1, Ericsson] proposed that the benefits of adding support for assistance information should be proven with evaluations before RAN1 discuss what assistance information to support and the potential specification impact. It proposed that for Case 3a, study signaling enhancements for the LMF to provide the ground truth label (e.g., ground truth direct path ToAs or UE locations) to support the training data collection. It also proposed to conclude that there is no need for PRS configuration enhancements for data collection purposes for Case 1/2a/2b. It proposed that for Case 1, introduce support for the UE to request PRS transmissions for training data collection purposes. If supporting Case 2b, [1, Ericsson] proposed to study the requirements for training data logging and reporting using 3GPP protocols.

[2, Huawei] proposed that for training data collection for AI/ML based positioning support assistance signaling of the requested quality of training data.

[3, ZTE] proposed to reuse on-demand PRS mechanisms defined in Rel-17 for PRS request of data collection by UE side. It also proposed that for Case 3a, model training and model deployment are left up to network implementation with no specification impact on data collection. [3, ZTE] proposed that at least for data collection of Case 2b and Case 3b, support UE/TRP to report more than 8 additional path timings and RSRPPs, and to report path phase of a channel path in addition to path power and path timing. [3, ZTE] also proposed to study and support multi-port SRS/PRS in order to collect enriched channel observations at least for data collection of Case 2b and Case 3b.

[4, vivo] proposed to support time domain CIR as one model input for training of AI/ML model for positioning. It proposed to further study related assistance information at least consisting of RS configuration and data collection indication to support data collection. For ground truth label collection, to improve the quality of labels, [4, vivo] proposed to indicate UE the criteria or requirement for data labeling or indicate UE to report label quality indicator. For measurement collection, at least measurement type and format should be indicated, depending on to model input.

[5, OPPO] proposed that for the training data collection at UE side, study the potential spec impact on that UE sends request of preferred or supported configurations to LMF.

[7, Nokia] proposed RAN1 to deprioritize the study and discussion of the specification impact of data collection for model training for the current functionality identification framework. [7, Nokia] proposed RAN1 to prioritize the study and discussion of the specification impact of data collection for model update/re-tunning and performance monitoring considering legacy 3GPP framework for positioning. For data collection, it also proposed that RAN1 to study and discuss the potential specification impact of at least the following challenges: data availability, noise ground truth, presence of abnormal propagation conditions, and RF imperfection. Reusing as much as possible the legacy 3GPP framework on positioning.

For all cases, [8, CATT] proposed that LMF side can collect a large-scale dataset from numerous UEs/PRUs/gNBs. Furthermore, [8, CATT] proposed that for the ground truth label generated by the UE/gNB based on non-NR and NR RAT-dependent positioning methods, the quality indicator for ground truth label can be used to determine whether the ground truth label meets the required quality for AI/ML model training. When LMF side collects training data from UEs/PRUs/gNBs, [8, CATT] proposed that LMF side can indicate the conditions or criteria such as the threshold of quality indicator.

[9, Baicells] proposed to support both quality indicator from the data collection entity and requested quality indicator from the training/management entity. It proposed that assistance information such as time stamp, sequence number of the training data should be collected to support training data association, false data removal, thus ensuring high-quality training dataset generation and to study the procedure of training data collection with time stamp/sequence number to facilitate training data pre-processing (e.g. data binding).

[11, Fujitsu] proposed to study at least the following aspects for the data quality indication: data accuracy information; data timing information; data source information and data applicability information.

[13, Google] proposed that the model training in the NW-side ML-based positioning should be transparent.

[14, LG] proposed to consider the quality of ground truth label generation based on the measurement reports from TRPs for LMF/NW with known UE/PRU case.

[15, Samsung] proposed that current signaling framework of the measurement-report could be used as starting point to enable training data collection.

[18, MediaTek] proposed to support to collect scenario identifier and related information (e.g., LOS probability) in training data collection. It also proposed to that for Case1, Case 2a, Case2b, support collecting at least: PDP/CIR (or truncated, compressed); RSRP; Horizontal location; LOS/NLOS condition, TOA, DOA, and other intermediate metrics; scenario identifier.

[21, InterDigital] proposed to support different labels for information associated with PRU and non-PRU (e.g., normal UE) and to support labels associated with uncertainty of the ground truth.

[22, Apple] proposed that the quality of the ground truth labels should be signaled to assist the selection and monitoring of the AI/ML model.

[23, Lenovo] proposed that training dataset acquisition, training dataset construction and actual training of the model may or may not take place in the same entity. It also proposed that data labels used for AI/ML positioning may include at least location and timing information associated with each collected data point as well as Label quality in at least Cases 1-2b.

[24, Qualcomm] proposed that LMF provides the following assistance information for UE/PRU: timestamping of measurements and labels; indication of how PRS resources map to physical anchor location/angles; indications of timing errors at network side; indication of LOS/NLOS maps for each TRP.

[26, NTT DOCOMO] proposed additional parameters can be configured in DL RS configuration for data collection purpose, e.g., associated measurement information (e.g., quality indicator, timing information, … of each measurement) and scenario/environment information (e.g., deployment scenario, TRP info., UE distribution, …), etc.

Moderator’s comment:

Companies’ views on potential specification impact for data collection.

Data quality request and/or indicator: [2, Huawei], [4, vivo], [8, CATT], [9, Baicells], [11, Fujitsu], [21, InterDigital], [14, LG], [22, Apple], [23, Lenovo] (at least for Case 1-2b), [26, NTT DOCOMO],

Time stamp of data: [9, Baicells], [11, Fujitsu], [23, Lenovo], [24, Qualcomm], [26, NTT DOCOMO],

RS configuration(s) request and/or indication for data collection: [1, Ericsson], [4, vivo], [5, OPPO], [24, Qualcomm], [26, NTT DOCOMO],

Scenario identifier: [18, MediaTek], [26, NTT DOCOMO],

LOS/NLOS condition: [18, MediaTek], [24, Qualcomm] (assistance information from LMF for each TRP to UE/PRU)

Data format information (e.g., granularity factor k) for timing related measurement: [1, Ericsson]

Timing error at network side: [24, Qualcomm] (assistance information from LMF to UE/PRU),

Note that potential specification impact regarding types of measurement (i.e., potential new measurement and/or enhancement to existing measurement) in collected data is summarized in section 2.5.

The following proposal is formulated for discussion.

Proposal 1-1-2

Regarding data collection for AI/ML based positioning, at least the following information elements of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from data generation entity

- Quality indicator

  - At least for and/or associated with ground truth label for model training

  - Report from data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

- Time stamp

  - At least for and/or associated with training data for model training

  - Report from data generation entity together with training data and/or as LMF assistance signaling

- Note1: whether the above information elements can be applied to other aspects of AI/ML model LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note2: transfer of data from the entity generating data to a different entity is not precluded and associated potential specification impact is for further study

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Fujitsu | 1. We suggest adding “and model monitoring” at the end for both the first sub bullets of “ground truth label” and “Quality indicator”. 2. For the third bullet “Quality indicator”, the quality indicator for input data other than model GT label should be considered. |
| MediaTek | Agree Fujitsu for the quality indicator for input data should be considered. |
| CMCC | Support. |
| NOK | We are ok with the direction of the proposal. We have some suggestions: - First main bullet: we should use the plural “ground truth labels”, because it is beneficial to consider/allow multiple labels for one observation. - Second main bullet: to include explicitly both measurements classes: new measurements (CIR, PDP) and already existing measurements (RSRP, RSRPP, RSTD). - Third main bullet: Is this indicatory supposed to be binary? Is it related to a soft measurement report instead? In Note 1, we should also consider the functionality identification framework already agreed upon in the general aspects agenda item. |
| Ericsson | The proposal seems to imply there is a single “data generation entity”. But the data for model input and model output may be provided by different entities. For RS configuration: existing configuration can be reused, and no spec impact. Suggested edits: - Ground truth label - At least for model training - Report from the label data generation entity - Measurement (corresponding to model input) - At least for model training - Report from the measurement data generation entity - Quality indicator - At least for and/or associated with ground truth label for model training - Report from the label data generation entity and/or as request from a different (e.g., data collection, etc.) entity - RS configuration(s) - At least for deriving measurement - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP |
| Lenovo | Supportive of FL’s proposal. Spec impact for RS configuration can be provided in terms of different signalling between non-AI/ML and AI/ML UEs. |
| NVIDIA | Support the proposal in principle. |
| LG | Fine with the proposal |
| vivo | Support |
| Intel | Support the proposal. |
| Samsung | Support the proposal in principle. Regarding E///’s comments, we think many of the item can consider reuse/extend current signaling framework. Here is the list identified for useful. |
| CATT | We support the proposal in general. For Quality indicator, it is unnecessary to collect low-quality labels. A more reasonable way is to indicate the required criteria of quality indicator to data generation entity. Is it a correct interpretation that the above consideration is already captured by ‘At least for and/or associated with ground truth label for model training’ and ‘Report from data generation entity and/or as request from a different (e.g., data collection, etc.) entity’. |
| CAICT | Support. |
| NTT DOCOMO | Support the proposal. |
| Xiaomi | Support the proposal. And also OK with Ericsson’s update for the first 3 bullets. For the RS configuration, we are OK to keep it. Whether existing signaling or new signaling is used, we could have further discussion. |
| OPPO | Fine with the proposal. |
| HW/HiSi | Agree with Fujitsu to add “for model monitoring” to the first two sub-bullets. The quality indicator could be associated with the reported data/labels or it could be configured by the network or both. We think a configuration form the NW and letting the UE only report the data that meets this quality is preferable. But we are open to both approaches. A question for clarification here: The “At least for and/or associated” in the bullet for quality indicator, is the intention that “associated with” means the indicator is sent alongside with the label? And “for” means it is configured by the network, and only labels that satisfy the requirement are reported? We think that the association is not only needed for labels, but importantly also measurements, since not only cases where the label is coming from the PRU/UE should be covered. Also when the label is generated e.g. at the network, only measurements satisfying a pre-defined quality should be reported. We would therefore make the following suggestion for modification. - Ground truth label - At least for model training and monitoring - Report from data generation entity - Measurement (corresponding to model input) - At least for model training and monitoring - Report from data generation entity - Quality indicator - At least for and/or associated with ground truth label and for measurements for model training - Report from data generation entity and/or as request from a different (e.g., data collection, etc.) entity |
| Qualcomm | - For Note2, it seems to be out of RAN1 scope to study transferring data from generating/source entities. - For ground truth label, the labeling assistance from LMF to UE also has specification impact. - Quality indicator can also be considered for measurements - Timing errors need also to be included |
| ZTE | We have several comments: - For first bullet and second bullet, better to provide explicit measurements as commented by Nokia. - RSRP, RSRPP, RSTD and UE location as label - CIR and PDP as measurement corresponding to model input - For RS configuration, we think the on-demand PRS can be reused. No enhancements are required. - For time stamp report, current specification already supports reporting time stamp of measurement. It’s not clear what is the additional specification impact. - For note1, we also think data collection can be used for model inference. For example, measurements should be reported from UE to LMF for LMF-side model. |
| Baicells | We share the same concern with CATT that for a request, it may be not directly associated with ground truth (e.g. SINR ) , thus we suggest as follow: - Quality indicator - At least for and/or associated with ground truth label for model training when report from data generation entity - At least for the quality requirement of measurements/ground truth label as request from a different (e.g., data collection, etc.) entity |
| Apple | Agree on a need to add model monitoring to this list or repeat the agreement for monitoring (from the Section 2.3 this is not done). |
| Fraunhofer | Agree with the comments from Fujitsu |
| InterDigital | Agree with the proposal in principle. For quality indicator, we would like to clarify which data it is applicable. From our perspective, LOS/NLOS indicators, if used for training should be verified. We don’t think a quality indicator is applicable for LOS /NLOS indicator for training purpose. |
| Moderator | To Nokia and ZTE: Types of measurement is in section 2.5. No need to have them listed here. To Fujitsu and Huawei: Depends on monitoring methods, it is not always true to collect ground truth label for monitoring. However, note1 still allows to further study. To Qualcomm: Assistance from LMF to UE on labeling is covered in proposal 1-1-1. Wording update into proposal 1-1-2a below to address comments. |

Proposal 1-1-2a

Regarding data collection for AI/ML based positioning, at least the following information elements of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note: there may not be specification impact on top of existing RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note: there may not be specification impact on top of time stamp report in existing positioning measurement

- Note1: whether the above information elements can be applied to other aspects of AI/ML model LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note2: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | Fine with proposal. |
| Ericsson | - We still have concern about RS configuration. Since it’s acknowledged that existing RS configuration can be adequate, it shouldn’t be included. If future discussion proves that new RS configuration is needed, it can always be added. - Regarding time stamp, since measurement of model input and ground truth label are likely generated by two different entities, then each of them should be time stamped, so that they can be paired up at training data collection entity. Thus, suggest: “At least for and/or associated with training data for model training. One time stamp for measurement, another time stamp for ground truth label, when measurement and ground truth label are generated by different entities.” |
| ZTE | Support |
| Xiaomi | We are OK with Ericsson’s update |
| NTT DOCOMO | Support |
| MediaTek | Support |
| Fujitsu | Support. |
| CATT | By further check, we think ‘as request from a different (e.g., data collection, etc.) entity’ for Quality Indicator already cover our view in previous round. So OK. |
| Vivo | Support |
| InterDigital | We support the proposal in principle. We do not want to make any further changes to the proposal for a progress. We have a question for FL. As mentioned in the previous round, we are not sure how quality indicator can be applied to LOS/NLOS indicators for data collection. Are there proposals to discuss the details of the quality indicator? |
| Lenovo | Generally supportive of the revised proposal. On the two notes in the RS configuration and Timestamp bullets, it is mentioned that there may be no spec. impact and while we tend to somewhat agree, if existing functionality is re-used for another purpose, i.e., in this case AI/ML positioning, there may be spec. impact depending on AI/ML Pos. signalling framework. Perhaps it would be more accurate to mention that adapt the two notes as follows in terms of no new enhancements or new RS configuration(s): - Note: There may not be any enhancements specification impact on top of existing RS configuration(s) or any new RS configurations for positioning measurement |
| NOK | We request a clarification to FL. It is not clear in the text if it is an Observation or a Proposal to study potential specification impacts. However, in general, we agree with the content. We have only a rewording suggestion on Note1. - Note1: whether the above information elements can be applied to other aspects of LCM of AI/ML functionality identification and/or AI/ML model identification (e.g., updating, monitoring, etc.) can also be discussed based on agreements done in the agenda item 9.2.1. |
| Intel | Support the proposal. |
| CAICT | Support. |
| LG | Fine with the proposal |
| Qualcomm | - The heading statement of the proposal can mention “information” instead of “information elements”. Deciding on the fitting to specifications can be discussed later. - Regarding Note2, it is still not clear to us whether RAN1 has scoping on data transfer for positioning. Please clarify (if any), if there is no expected scoping, the Note2 is not needed. - We also find it is important to include timing errors. Many evaluations showed the sensitivity of timing errors and importance to account for them during training. |
| CMCC | Fine. |
| HW/HiSi | Support |
| Fraunhofer | Support |
| Apple | Support. We are fine with Lenovo’s update to the RS configuration note. |
| Moderator | To InterDigital: My understanding is that whether and how we define quality indicator for LOS/NLOS indicator (whether it’s the ground truth label or measurement) can still be discussed later. For instance, depends on LOS/NLOS indicator type (e.g., hard), it is still possible to define some quality estimation for it. For soft value LOS/NLOS indicator, that can be discussed and decided later. To Ericsson: I don’t understand what your concern on RS configuration is. The main sentence says “the following information elements” which are necessary for data collection purpose. If we remove the bullet of RS configuration, that can be interpreted that RS configuration is not necessary for data collection. Is that Ericsson’s intention? We have ‘potential’ in the main sentence and a note under RS configuration explicit mention “may not have specification impact”. I don’t see that can be interpreted as endorsement of enhancement for RS configuration. To Qualcomm: We have a similar note to Note2 in last meeting’s agreement. Just one example why it’s necessary to have the note. Right now, we only agreed PRU to generate label for Case 1 and Case 2a. Without transferring data, PRU would the only entity to train model in those cases. At least from RAN1 perspective, we should NOT have such restriction. On timing error information, please see my summary of companies’ view on necessary information of training data (I copied blow). The proponent company need to convince the group that such information is necessary. Data quality request and/or indicator: [2, Huawei], [4, vivo], [8, CATT], [9, Baicells], [11, Fujitsu], [21, InterDigital], [14, LG], [22, Apple], [23, Lenovo] (at least for Case 1-2b), [26, NTT DOCOMO], Time stamp of data: [9, Baicells], [11, Fujitsu], [23, Lenovo], [24, Qualcomm], [26, NTT DOCOMO], RS configuration(s) request and/or indication for data collection: [1, Ericsson], [4, vivo], [5, OPPO], [24, Qualcomm], [26, NTT DOCOMO], Scenario identifier: [18, MediaTek], [26, NTT DOCOMO], LOS/NLOS condition: [18, MediaTek], [24, Qualcomm] (assistance information from LMF for each TRP to UE/PRU) Data format information (e.g., granularity factor k) for timing related measurement: [1, Ericsson] Timing error at network side: [24, Qualcomm] (assistance information from LMF to UE/PRU), Wording update below to address comments from Ericsson, Lenovo, NOK and Qualcomm. |

##### Proposal 1-1-2b (closed)

Regarding data collection for AI/ML based positioning, at least the following information of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

    - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

- FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection

- Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Companies are encouraged to provide comments especially if they object this proposal.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | Discussion is closed. Refer to Chairman’s notes for the relevant working assumption. |

## Model monitoring

In RAN1#112, the following were agreed.

Agreement

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects

- Entity to derive monitoring metric

  - UE at least for Case 1 and 2a (with UE-side model)

    - FFS PRU for Case 1 and 2a

  - gNB at least for Case 3a (with gNB-side model)

    - FFS gNB for Case 3b (with LMF-side model)

  - LMF at least for Case 2b and 3b (with LMF-side model)

  - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b)

- If model monitoring does not require ground truth label (or its approximation).

  - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc.

  - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc.

  - report of the calculated metric and/or model monitoring decision

- If model monitoring requires and is provided ground truth label (or its approximation)

  - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc.

  - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc.

  - report of the calculated metric and/or model monitoring decision

- Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Many companies discussed aspects related to model monitoring.

[1, Ericsson] proposed that for Case 3a and 3b, model monitoring metric is calculated without collecting test data and no signalling needs to be specified to collect test data for model monitoring purpose. For Case 1 and 2a, it proposed that model monitoring is handled on the UE side. It also proposed that for case 2a, available residual loss information could be used as assistance data from the network to the UE for monitoring purposes.

[2, Huawei] proposed to study model monitoring and the potential spec impact: NW collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision (applicable to Case3a and Case3b); UE collects inputs for monitoring, calculates monitoring KPIs which are then fed back to NW, and NW makes monitoring decision (applicable to Case1, Case2a and Case2b); UE collects inputs for monitoring, calculates monitoring KPI, makes monitoring decision, and reports the decision to NW where NW will indicate UE to execute the decision accordingly (applicable to Case1, Case2a). Furthermore, [2, Huawei] also proposed that NW may configure a threshold criterion (e.g., threshold RSRP/SINR or threshold intermediate KPIs) to facilitate UE to make monitoring decision.

[4, vivo] observed that monitoring data shift can be based on model input and/or model output. It proposed to study monitoring based on model input and output. It proposed that dedicated reference signals may be required to obtain performance metrics so as to support model monitoring.

[5, OPPO] proposed that for AI model monitoring for AI/ML based positioning, not support UE to report “target output” or “label” for the comparison with the output of the AI/ML model. It also proposed to study whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar and availability of PRU for typical deployment if PRU is utilized to collect data for AI model performance monitoring.

[6, Spreadtrum] proposed that for direct AI/ML positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label for output based monitoring metric. It also proposed that for all cases, ground-truth labels can be reported to LMF, and it is up to LMF to monitor/evaluate the performance of AI/ML model.

[7, Nokia] proposed that for Case 1 and Case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contains monitoring data for AI/ML model performance (including samples with corresponding labels).

[8, CATT] proposed that ground truth labels and high-quality noise ground truth labels are used to monitor the AI/ML model performance. It also proposed that the relative displacement between time T1 and time T2 estimated by motion sensor method can be used to monitor the AI/ML model. It further proposed that if the AI/ML model is inferred and monitored at different sides, at least LMF-side performance monitoring should be supported.

[9, Baicells] proposed to consider out of distribution detection as a model monitoring method.

[11, Fujitsu] proposed not to preclude network entities for calculating monitoring metrics for at least UE-side AI/ML positioning models. It also proposed to study on statistics (e.g., deviations or distributions) and additional information (e.g., timing, source, or imperfection level) of input data obtained from different sources or positioning methods for input-driven monitoring method.

[12, Xiaomi] proposed that the monitor of the application condition or scenario could be considered for the performance monitoring.

[13, Google] proposed that the model monitoring for UE-side sand NW-side ML-based positioning should be transparent.

[14, LG] proposed to consider assistance signalling for UE-sided model (e.g. distance between TRPs, beam information per TRP) and contents of model switching/update (e.g. AI/ML model itself, AI/ML model parameter or structure only) for potential specification impact on AI/ML model monitoring.

[15, Samsung] proposed that other measurement metrics like L1 RSRP/SNR level could be considered as monitoring metric.

[16, CAICT] proposed that for AI/ML model monitoring at UE/NW side, the comparison of positioning results from different sources and the deviation of positioning results over a period of time can be used. It also proposed that NW could provide area-based model monitoring for AI/ML model update at UE.

[17, CMCC] proposed to consider two options as the performance metrics for model monitoring: based on the ground-truth labels; based on the results of traditional positioning techniques and/or the results of AI/ML model. It also proposed to study UE-side model monitoring, LMF-side model monitoring and Hybrid model monitoring for both UE-sided and LMF-sided model. It also proposed that UE could report its attitude and motion status to increase gNB awareness.

[18, MediaTek] proposed that for model monitoring, study using model configurations (e.g., model ID, model version) and scenario monitoring (e.g., LOS probability monitoring, serving cell-based monitoring) as ways of monitoring models. It also proposed that for model monitoring of direct AI/ML positioning, support the soft information associated to the estimated location as model output.

[19, Fraunhofer] proposed for positioning use cases, consider the AI/ML model monitoring for inference input and training data mismatch; inference output inconsistency; drop in QoS; AI/ML model/concept drift.

[22, Apple] proposed that for both direct AI positioning and AI-assisted positioning, the monitoring may be based on the properties and characteristics of the input e.g. a Doppler estimate on an input CIR may indicate the model is not appropriate.

[23, Lenovo] proposed that in order to increase flexibility of the derivation of the model monitoring metric, support PRU as an entity for Cases 1 and 2a, while support gNB as an entity for case 3b. It proposed that RAN1 to further confirm that the entity deriving the model monitoring metric is the same entity that may also perform model monitoring. It also proposed to support model monitoring using model output statistics with and without ground truth label information.

[24, Qualcomm] proposed that for AI/ML positioning model monitoring (Case1 to Case3b), study model monitoring based on joint model input and output (ground truth-based monitoring) where Ground truth label and AI/ML model input measurements can be made available at monitoring/model management side.

[25, NTT DOCOMO] proposed that for case1and 2a, prioritize the monitoring metric calculation on UE; for case 2b and 3b, prioritize the monitoring metric calculation on LMF; for case 3a, performance metric calculation on both gNB and LMF can be considered. It also proposed for case1, 2a, when monitoring entity is UE, UE should calculate monitoring metric following NW indication if NW makes decision of upcoming model operation based on model monitoring where the indication includes model ID/functionality, monitoring type (e.g., input-based, output-based), performance metrics/threshold.

Moderator’s comment:

Regarding the entity to derive monitoring metric, it seems all companies are ok with the candidate list in previous agreement. [23, Lenovo] proposed to confirm it. Furthermore, [23, Lenovo] also proposed to support PRU as an entity for Cases 1 and 2a, while support gNB as an entity for case 3b. [26, NTT DOCOMO] proposed for case 3a, performance metric calculation on both gNB and LMF can be considered. [6, Spreadtrum], [8, CATT], [11, Fujitsu] and [17, CMCC] all proposed to consider LMF to derive monitoring metric for UE-side model.

The following proposal is formulated to agree the candidate list of entities from previous agreement and to identify some FFS points.

##### Proposal 1-2-1 (closed)

Regarding AI/ML model monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric

- UE at least for Case 1 and 2a (with UE-side model)

- gNB at least for Case 3a (with gNB-side model)

- LMF at least for Case 2b and 3b (with LMF-side model)

- FFS by RAN1#113, whether to also support the following options to derive monitoring metric

  - PRU for Case 1 and 2a (with UE-side model)

  - gNB for Case 3b (with LMF-side model)

  - LMF for Case 1 and 2a (with UE-side model), Case 3a (with gNB-side model)

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Fujitsu | Support. |
| MediaTek | Support PRU for Case 1 and 2a (with UE-side model). |
| CMCC | Fine. |
| NEC | Support |
| NOK | Support |
| Ericsson | - Do not support sub-bullet “PRU for Case 1 and 2a (with UE-side model)”. PRU does not have knowledge of each individual UE’s model. The PRU cannot be put at the same location at the same time as an individual UE to provide its location as ground truth, when a UE wishes to monitor its model. Thus it does not make sense to use PRU to derive model monitoring metric for UE side model. - Do not support LMF for Case 1 in last sub-bullet. The LMF at best has the estimated UE location for Case 1. The LMF has no information about model input, and has no information about ground truth label. We don’t see how LMF can derive model monitoring metric for Case 1. |
| Lenovo | Support |
| NVIDIA | Support |
| LG | Fine with the proposal |
| vivo | Support |
| Intel | Support the proposal. |
| Samsung | Support proposal. |
| CATT | Agree. |
| CAICT | Support. |
| NTT DOCOMO | Support the proposal. |
| Xiaomi | Support Regarding the PRU for Case 1 and Case 2a, our understanding is as follows - Regarding performance monitoring for model activation, deactivation, switch, fallback maybe PRU is not feasible considering different wireless channel condition between UE and PUR - Regarding performance monitoring for the overall model update, PRU can be considered. In this case, the PRU could be used to monitor whether there is any shift of the overall data distribution So our suggestion to handle the model monitoring for activation/deactivation/switch/fallback and model monitoring for model update separate when we discuss the FFS part. |
| HW/HiSi | For FFS, since they are not proposed to be discussed this meeting, it might be better to delete the bullet, to not stall the progress on the bullets without FFS (note: the FFS aspects were already agreed as FFS last meeting). |
| Qualcomm | We are generally OK with first three bullets. The FFS bullet needs further clarification as some options may not be feasible. For example, it is not clear how the model developed for UE can be available at PRU side. For LMF to compute monitoring metric in Case1 and 2a, it is not clear what the LMF has on top of UE to compute better monitoring metrics. |
| ZTE | - The same comments as Ericsson on the following sub-bullet: - PRU for Case 1 and 2a (with UE-side model) - For the following subbullet, it’s not clear to us. Why gNB has to get the monitoring metrics for LMF side model? Our understanding is that LMF can derive monitoring metrics directly if LMF knows the PRU location. - gNB for Case 3b (with LMF-side model) |
| Apple | Fine with proposal. |
| Fraunhofer | Support. Also agree with Xiaomi’s suggestion to handle the model monitoring for activation/deactivation/switch/fallback and model monitoring for model update separate when we discuss the FFS part. |
| Moderator | To Ericsson, Qualcomm and ZTE: PRU may be deployed by the operator where the UE-side model could be download from e.g., LMF or an OTT server for monitoring purpose. Why is this not feasible? FFS means we study the feasibility and potential benefits first then to decide later whether to support them or not. I don’t see what’s the harm to have FFS. |
| Moderator | Discussion is closed. Refer to chairman’s notes for relevant agreement. |

Summary of companies’ view on model monitoring methods:

- Model monitoring based on provided ground truth label (or its approximation)

  - Support: [2, Huawei], [4, vivo], [6, Spreadtrum], [8, CATT], [16, CAICT], [17, CMCC], [22, Apple], [23, Lenovo], [24, Qualcomm]

  - Concern: [5, OPPO] (not support UE to report label, further study on whether PRU’s label can be used for UE model monitoring)

- Model monitoring based on model input (measurement) and/or statistic of model input

  - Support: [2, Huawei], [4, vivo], [9, Baicells], [11, Fujitsu], [15, Samsung], [19, Fraunhofer], [22, Apple], [24, Qualcomm]

- Model monitoring based on applicable condition or scenario of model

  - [4, vivo], [12, Xiaomi], [18, MediaTek]

- Self-model monitoring

  - [1, Ericsson], [4, vivo], [7, Nokia],

- Model monitoring based on model output and/or statistic of model output without ground truth label

  - [19, Fraunhofer], [23, Lenovo],

- UE motion sensor based model monitoring or as assistance for network-side model monitoring

  - [4, vivo], [8, CATT], [17, CMCC]

- Residual loss based model monitoring

  - [1, Ericsson]

- Model monitoring based on a separate ranging model or classifier model

  - [4, vivo]

- Network (e.g., LMF) to assist model monitoring for UE-side model

  - [1, Ericsson] (for case 2a), [2, Huawei], [7, Nokia], [11, Fujitsu], [14, LG],

Moderator’s comment:

As summarized above, the first two methods have majority support. The following proposal is formulated based on majority view for discussion.

Proposal 1-2-2

Regarding AI/ML model monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact

- Assistance signaling from LMF to UE/PRU/TRP for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on measurement (corresponding to model input) and/or statistics of measurement

  - Monitoring metric: measurement statistics as compared to the model input statistics of the training data

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label

- Note: other options (of monitoring methods, monitoring metrics) are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Fujitsu | For the fourth bullet, maybe the quality indicator or related information can be added together with the approximation of GT label. |
| MediaTek | We propose to update as follows: Regarding AI/ML model monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact - Assistance signaling from LMF to UE/PRU/TRP for UE/gNB-side model monitoring - Assistance signaling from UE/PRU for UE/network-side model monitoring - Model monitoring based on measurement (corresponding to model input) and/or statistics of measurement without ground truth label - Monitoring metric: measurement statistics as compared to the model input statistics of the training data - Model monitoring based on soft information of model output without ground truth label - Monitoring metric: soft information of model test output as compared to soft information of model train output - Model monitoring based on provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label - Provisioning of ground truth label - Note: other options (of monitoring methods, monitoring metrics) are not precluded |
| CMCC | Fine. |
| NEC | Regarding the third and fourth bullet on model monitoring, they appear to be consistent with the agreement reached during the previous meeting. However, could the moderator please provide more details on the specific objectives of these two bullets? Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded. |
| Moderator | To NEC: Previous agreement you quoted says “to study and provide inputs” and all those monitoring metrics are listed as examples for monitoring methods. The third and fourth bullets of this proposal are the identified methods (based on majority views) with the corresponding monitoring metric defined (not as example any more). Hope this clarifies. |
| NOK | We agree with the direction of the proposal. However, we have the following concerns: Third main bullet: the monitoring based on measurements applies for both input and output model. In the proposal it is only considered the model input. Please check our contribution (R1-2302633) Fig. 9 describing an example of model output used for monitoring purpose. |
| Ericsson | First, need to add “for each case”, so the analysis is done separately for each case. Also: agree with NEC (3rd/4th bullet seem to duplicate previous agreement) and MTK comment (if 3rd/4th bullet are kept) About moderator’s explanation, “identified for further study” is not clearly different from “to study and provide inputs”. Maybe updates are needed. |
| Lenovo | Support |
| NVIDIA | Support the proposal in principle. |
| LG | Similar view with MediaTek to revise the proposal. In addition, consider the quality of the ground truth label based method on fourth-bullet |
| vivo | Support in principle. We suggest to remove “feasibility, necessity” from the main sentence as those for model monitoring have already been demonstrated by multiple contributions in 9.2.4.1 and 9.2.4.2. |
| Samsung | We are concerning on the part “statistics of measurement” which by FL’s description is only targeting to the measurement of model input. The consideration like SNR level or RSRP level may not from the measurement used for model input. |
| CATT | Fine with the proposal. These two methods (ground truth label-based and input-based monitoring) gain majority’s support. Also fine to add ‘without ground truth label’ to the third bullet. |
| CAICT | Support. |
| NTT DOCOMO | Support in principle. It’s better to study the NW indication of monitoring metrics that UE monitors for label-based and label free monitoring, if the performance metric can be determined by NW for UE side monitoring metrics calculation. |
| Xiaomi | We have similar concern with Samsung. |
| OPPO | For the fourth bullet, if the ground truth label is generated by UE based on existing positioning methods, monitoring result maybe incorrect because of the positioning error caused by existing positioning methods. Same view with Ericsson to add “for each case”. |
| HW/HiSi | Support in principle. For the first bullet, can it please be clarified for “Assistance signaling from LMF to UE/PRU/TRP for UE/gNB-side model monitoring”: Why is it distinguished in the same sentence between TRP and gNB, aren’t they the same here? Should TRP be changes to gNB? |
| ZTE | Agree with comments from NEC and Ericsson. If this proposal is intended to identify or confirm something new, we should reuse the same descriptions in last meeting. - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric: measurement statistics as compared to the model input statistics of the training data - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label |
| Fraunhofer | Based on our understanding, the assistance signaling can potentially include the LMF or a PRU indicating the monitoring of (one or more) potential performance degradation indicators. These indicators can then be monitored by the designated monitoring entity. If this is accurate, could we consider adding a clarification note specifying the types of assistance signaling, such as statistics, performance degradation indicators, or other relevant information that may aid in positioning accuracy assessment and monitoring? Additionally we agree with the changes suggested by MediaTek. Also ok to consider the quality of the ground truth label for monitoring. |
| Moderator | To NEC, Ericsson and ZTE: This proposal is intend to reduce the study scope from last agreement. Wording update below into proposal 1-2-2a to address comments. |

Proposal 1-2-2a

Regarding AI/ML model monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s) and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on measurement (corresponding to model input) and/or statistics of measurement without ground truth label

  - Monitoring metric: measurement statistics as compared to the model input statistics of the training data

- Model monitoring based on statistics of model output without ground truth label

  - Monitoring metric: statistics of model output as compared to the model output statistics of the training data

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Note: other options (of monitoring methods, monitoring metrics) are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | We are concerning on the part “statistics of measurement” which by FL’s description is only targeting to the measurement of model input or output. The consideration like SNR level or RSRP level may not from the measurement used for model input nor counted as model output. |
| Moderator | Wording update into proposal 1-2-2b to address Samsung’s comment |

Proposal 1-2-2b

Regarding AI/ML model monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s) and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on statistics of measurement on RS without ground truth label

  - Monitoring metric: measurement statistics as compared to the statistics associated with the training data

  - Note: the measurement on RS may or may not be the same as model input

- Model monitoring based on statistics of model output without ground truth label

  - Monitoring metric: statistics of model output as compared to the model output statistics of the training data

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Note: other options (of monitoring methods, monitoring metrics) are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Ericsson | We don’t agree with 3rd and 4th bullet. It is not reliable to perform model monitoring based on model input statistics only, or model output statistics only, as compared to training data. A simple example: training data is collected for the entire floor area. A UE stays at one smaller area on the floor (e.g., a corner area). Then UE’s model input statistics is very different from that of training data. Its output statistics is also very different from that of training data. But the UE’s model can still function quite well. Thus model monitoring based on model input statistics only, or model output statistics only, would give high false alarm. This is not acceptable. Thus 3rd and 4th bullet should be combined, i.e., monitoring based on model input and model output jointly. |
| ZTE | We have concern on the following bullet. As model output only includes a small amount of channel information(e.g., UE location or some intermediate results) compared to model input(e.g., CIR, PDP), how statistics of model output can be used for model monitoring? The feasibility is unclear to us. It may need to be evaluated in 9.2.4.1 first. - Model monitoring based on statistics of model output without ground truth label - Monitoring metric: statistics of model output as compared to the model output statistics of the training data |
| NTT DOCOMO | We suggest using ‘performance monitoring’ as the monitoring may be performed at model level scale or functionality scale. Meanwhile, as the NW indication of the performance metrics for UE side monitoring metrics calculation may be considered, the potential applicable monitoring metrics could be identified/specified. |
| MediaTek | Model output can be used as model monitoring and we have evaluated in section 3.2.13 R1-2303340. |
| Fujitsu | We suggest to modify the wording as “statistics of current and historical measurements or model input/output compared with the statistics of training data”, because only the snapshot input/output of the model inference moment may not be such reliable. In terms of Ericsson’s concern, the statistics may not be only one absolute value, it may be a certain range or sort of credibility information which give a hint for the monitoring entity to make decision, so the validity depends on the monitoring method. |
| CATT | We have questions for the new bullet (‘statistics of model output’) as ZTE. It only gains very few proponents from the summary list. What is the threshold (# of supporting companies?) to include a monitoring method in this proposal? At least vivo and CATT provide simulation result for motion sensor based model monitoring, which shall also be added here. We can only agree on the original two methods based on obvious majority’s view. Or we should include all methods which are supported/evaluated by certain number of companies fairly. |
| Vivo | We share the concern from ZTE and CATT. Either we list all options or revert back to original proposal 1-2-2. |
| Lenovo | Generally supportive, but for further progress we can also accept bullet 3 and 4 as an FFS |
| NOK | If the word “for further study” is maintained, we support the current version of the proposal with a rewording in the 4th main bullet: - Model monitoring based on statistics of model output without ground truth label - Monitoring metric: statistics of model output as compared to a reference (e.g., model output statistics of the training data). |
| CAICT | We are general fine with the proposal. |
| LG | Generally fine with the proposal. Regarding the monitoring methods above, is it also consider a certain threshold(s) to execute model fine-tuning/update based on the performance metric? |
| OPPO | Similar view with ZTE. We suggest the fourth bullet can be an FFS for further check. |
| Qualcomm | - The previous proposal heading included “feasibility, necessity” but it got taken off without notification. Our understanding that some monitoring options may not be always feasible as discussed by other companies above. - It is better to keep the wording consistent with agreement from previous session (i.e., “Regarding monitoring for AI/ML based positioning” instead of “Regarding AI/ML model monitoring for AI/ML based positioning”) |
| HW/HiSi | Tend to agree with ZTE, CATT and vivo on model output based monitoring without ground truth label. More evaluation is needed. Otherwise fine. |
| Fraunhofer | Support the proposal direction and ok with Fujitsu’s modification on wording. We are also ok with bullet 4 as FFS. One high-level comment: in our view, there is no reason why the monitoring should focus and rely only on one ground truth label-free method at the time. It can investigate signs of problems in the input / output and at the same time incorporate assistance information from motion sensors for example. For us, the proposal provides a list of possible points for monitoring, and these can be monitored in any combination. Also, the assistance signaling can already contain monitoring indications. The network might possess environmental data or multiple UE reports that are pertinent to the inference performance. In such a scenario, the network can alert for case 1 and case 3a the UE or NG-RAN node about any potential problem or monitoring opportunity. Finally, as other companies have mentioned, further study is required on how the monitoring can reason about the best suggested course of action, based on the monitoring performance indicators detected: - How can we distinguish between temporal vs permanent changes in the radio environment, that would require different handling? - How can we identify a sub-region where the model is no longer valid and possibly collect new data only there and not in the entire area? Based on this, we would propose the following changes (in blue color): Regarding AI/ML model monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s) and potential specification impact for each case (Case 1 to 3b) - Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring - Assistance signaling from UE/PRU for network-side model monitoring - Model monitoring based on statistics of measurement on RS without ground truth label - Monitoring metric: measurement statistics as compared to the statistics associated with the training data - Note: the measurement on RS may or may not be the same as model input - Model monitoring based on statistics of model output without ground truth label - Monitoring metric: statistics of model output as compared to the model output statistics of the training data - Model monitoring based on provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label - Provisioning of ground truth label and associated label quality - Note: other options (of monitoring methods, monitoring metrics) are not precluded - Note: monitoring methods that don’t rely on ground truth labels can be applied in parallel - Note: Assistance signaling can include the network indicating monitoring alerts for case 1 and case 3a - FFS: monitoring methods to analyze the detected indicators of model performance degradation and suggest short-term (e.g., model switching) or long-term (e.g., model fine-tuning) course of action |
| Apple | On this bullet: - Model monitoring based on statistics of measurement on RS without ground truth label - Monitoring metric: measurement statistics as compared to the statistics associated with the training data - Note: the measurement on RS may or may not be the same as model input It evolved from this: - Model monitoring based on measurement (corresponding to model input) and/or statistics of measurement without ground truth label - Monitoring metric: measurement statistics as compared to the model input statistics of the training data In the update, it seems that the statistics of the input has been modified to the statistics of the RS only. In cases we have mixed input e.g. CIR, RSRP,SNR, the update is not applicable. We would prefer to revert to the original text. |
| Moderator | To Ericsson: All the bullets are listed for further study. This proposal does not say any of the bullet will be supported. There’s nothing prevent monitoring jointly based on model input and output. Status of companies’ view (based on comment to proposal 1-2-2/1-2-2a/1-2-2b). Company prefer to have the 4th bullet: MTK, LG, NOK Company prefer not to have the 4th bullet: ZTE, CATT, vivo, OPPO, HW/HiSi Assuming other companies’ view is the same based on their contribution/proposal, we have - Model monitoring based on provided ground truth label (or its approximation) - Support: Huawei, vivo, Spreadtrum, CATT, CAICT, CMCC, Apple, Lenovo, Qualcomm (9) - Concern: OPPO - Model monitoring based on model input (measurement) and/or statistic of model input - Support: Huawei, vivo, Baicells, Fujitsu, Samsung, Fraunhofer, Apple, Qualcomm (8) - Concern: Ericsson - Model monitoring based on model output and/or statistic of model output without ground truth label - Support: Fraunhofer, Lenovo, MTK, LG, NOK (5) - Concern: Ericsson, ZTE, CATT, OPPO, HW/HiSi - UE motion sensor based model monitoring - Support: vivo, CATT, CMCC - Joint monitoring based on model input and output - Support: Ericsson, Fraunhofer The intention of this proposal is still listing aspects for further study but with a reduced and focused scope compared to last meeting’s agreement. With that, wording update to address comments and to reflect the status quo. |

##### Proposal 1-2-2c (closed)

Regarding monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Model monitoring based on statistics of measurement(s) without ground truth label

  - Monitoring metric: statistics of measurement(s) compared to the statistics associated with the training data

  - Note1: the measurement(s) may or may not be the same as model input

- Note2: other monitoring methods (e.g., based on statistics of model output without ground truth label, based UE motion sensor and/or jointly based on multiple monitoring metrics) are not precluded

Companies are encouraged to provide comments especially if they object this proposal.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | Discussion is closed. Refer to Chairman’s notes for the relevant agreement. |

## Model/functionality identification

In RAN1#110b-e, the following were agreed.

Agreement

Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement

- Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc.

- Model capability, e.g., positioning accuracy quality and model inference latency

- Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information

- Note: other aspects are not precluded

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

FFS: Detailed discussion of model ID with associated information and/or model functionality.

FFS: usage of model ID with associated information and/or model functionality based LCM procedure

FFS: whether support of model ID

FFS: the detailed applicable AI/ML operations

In RAN1#111, the following were agreed.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Several companies discussed detailed aspects related to model identification.

[1, Ericsson] proposed that for the positioning use case, model ID is not defined. Model ID based LCM is not supported for UE-side model. It also proposed that functionality ID and functionality ID based LCM are supported for UE side model.

[3, ZTE] proposed that for functionality identification, AI/ML-enabled feature is equal to a functionality. Direct AI/ML positioning is an independent functionality. For AI/ML assisted positioning, the granularity of the functionality is determined by the model output type.

[4, vivo] proposed that model information should contain meta-information indicating model capability and the physical and network environment or condition under which the model is suitable for operation.

[5, OPPO] proposed that for UE-side model is used for AI/ML based positioning (e.g., Case 1, Case 2a), support implicit or explicit information from LMF to UE to indicate/identify the scenarios/configuration so that UE can choose a suitable AI model matching the target case(s).

[7, Nokia] proposed that regarding functionality identification framework, to study and discuss the potential specification impact for functionalities and reporting of UE’s applicable conditions. It proposed RAN1 to consider max number of supported functionalities, delay in activating a functionality and generalization condition of functionalities as mandatory UE’s applicable conditions on supporting ML functionalities for all cases. For case 1 and 2a, it proposed supported N’t, supported N_port, supported N_TRP, supported intermediate_feature (for assisted AI/ML positioning only) as mandatory UE’s applicable condition. It also proposed some optional UE’s applicable conditions for case 1 and 2a.

[9, Baicells] proposed that if training is needed at UE side, training related system requirements (e.g. minimal training requirements) should be indicated via model information. If training is needed at gNB side, training related system requirements should be indicated via model information. It also proposed that regarding model transfer, geographical information e.g. the valid geographical boundary of the model can be attached as assistance information to better support UE-side model activation, switching, deactivation and performance monitoring etc.

[10, Sony] proposed that for AI/ML model indication, define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB.

[11, Fujitsu] proposed to study specification impacts of model identification procedure for at least AI/ML positioning sub use cases 1 and 2a, and functionality identification procedure for sub use case 2a. It also proposed that the format of the model identifier or functionality identifier is supposed to be studied in detail, the contents of the identifier are based on the model related information reported by UE, overhead issue should be considered during the reporting.

[12, xiaomi] proposed that in AI-based positioning, features are defined from the perspective of output parameters. It also proposed that functionalities are defined at least based on the factors which determine whether the functionality is workable or not.

[16, CAICT] proposed that for AI/ML based positioning with UE-side model (Case 1 and 2a), full set for functionality identification could be defined. Flexible functionality reporting mechanism could be considered to allow partial elements reporting within the full set.

[19, Fraunhofer] proposed to support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas.

[22, Apple] proposed that discussion of the details Model Capability, Indication and Configuration  should wait till the a framework has been decided in the general session.

[26, NTT DOCOMO] proposed that the indication of AI/ML functionality includes at least functionality information, which includes the nominal output of the AI/ML model, essential information to derive the model inference and validity condition for the functionality.

Moderator’s comment:

Summary of companies’ view on high level principle:

Functionality identification: [1, Ericsson], [3, ZTE], [7, Nokia], [11, Fujitsu] (for case 2a)

Model identification: [4, vivo], [10, Sony], [11, Fujitsu] (for case 1 and 2a),

It is moderator’s understanding that high level principle of whether to support functionality identification and/or model identification is more appropriate for agenda 9.2.1 general framework discussion.

On the other hand, several companies proposed 2nd level details of necessary information elements, which in moderator’s understand, would be applicable for either functionality identification and/or model identification for AI/ML based positioning.

Summary of companies’ view on information elements for identification:

Applicable (valid) area/scenario/environment: [4, vivo], [5, OPPO], [9, Baicells], [12, xiaomi], [19, Fraunhofer], [26, NTT DOCOMO]

Information related to measurement for inference: [7, Nokia], [10, Sony], [26, NTT DOCOMO]

Information related to model output: [3, ZTE] (for granularity of functionality identification), [7, Nokia] (for assisted AI/ML positioning), [26, NTT DOCOMO]

Note that discussion related to model input and output with associated potential specification impact are captured in section 2.5 and not duplicated here. The following proposal is formulated for discussion.

Proposal 1-3

Regarding LCM of AI/ML based positioning accuracy enhancement,

- At least for UE-side model (Case 1 and Case 2a), information related to applicable (valid) area/scenario/environment is beneficial if shared between the network and the UE

  - FFS potential specification impact (including details if explicit signaling or implicit with no specification impact)

- Note: other type of information is not precluded for further study

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Fujitsu | Support. In addition, shall we start some discussions on meta info or ID hierarchy for AI/ML positioning? It seems that 9.2.1 has a long way to figure out the ID type first but RAN2 7.16.2.1 needs some urgent input per use case to continue their study. |
| NEC | Support |
| NOK | This topic is aligned with the current discussion on LCM (functionality or model) in 9.2.1. We can start on the current agreement related to functionality identification framework. Thus, we agree with the direction, but we should be more explicit in what LCM framework we should center the discussion/study. |
| Ericsson | Do not support. It’s not clear how and why is the info shared? In general the UE should be responsible for checking that its model is working properly in the deployment environment. We agree that UE-side model works only in certain area/scenario/environment it was trained for. However, we don’t see how such condition can be neatly packaged and shared, since many factors are involved, including implementation related factors (channel estimation error, UE/gNB Tx/Rx timing etc), see the generalization evaluation results. It’s also not clear what network can do about such information, if UE manages to send it. Even if all reported info matches (e.g. cell ID), it does not guarantee that the model would work well (e.g., clutter parameter has changed since training). Evaluations show that the model only works for the specific factory floor it’s trained for. In this case, cell ID(s) can be used by the UE to check that it’s in the cell(s) that the model was trained for. But UE does not need to share the set of applicable conditions of its model with network. Suggested edits: Regarding LCM of AI/ML based positioning accuracy enhancement, - At least for UE-side model (Case 1 and Case 2a), UE ensures that its model works properly, including using information related to applicable (valid) area/scenario/environment of its model |
| Lenovo | Support of FL’s proposal, however on first bullet it is not full clear how a scenario can be shared between UE and network |
| NVIDIA | Support the proposal in principle. |
| LG | Fine in principle but the clarification is required to share the scenario btw UE and NW as some companies mentioned |
| vivo | Support in principle. We believe a common understanding of applicable scenario for UE-side model between the UE and network is critical for model deployment. If RAN1 agree such information is beneficial, then the next step (FFS point) can follow. |
| Samsung | This proposal seems only picked out the examples for “validity condition”, could FL clarify? Even that’s the case, is FL intention to say the validity condition should be known by Network and UE. |
| CATT | Agree that high level principle of model/functionality identification is discussed in 9.2.1. This may be a common issue for all use cases. For positioning, it is more meaningful to identify/discuss what information (area? scenario? environment?) may be feasible to get standardized for higher positioning accuracy or LCM. So far it seems difficult to use spec language to describe them (other than ‘configuration’, but it is not captured in the proposal). |
| CAICT | Support. |
| NTT DOCOMO | Support to start discussion from this proposal. In our view, the validity condition information could be regarded as part of identification information. |
| Xiaomi | We support this proposal |
| OPPO | Fine with the proposal. |
| HW/HiSi | We agree with Ericsson’s comment. Since for Case 1 and 2a the NW does not seem to be involved in the monitoring (See proposal 1-2-1), then it should not be the NW’s obligation to share scenario/environment information. |
| ZTE | The current formulation is like an observation rather than a proposal. If the intention is to study the applicable conditions of a functionality or a model that are specific to positioning use case. We propose the following revisions: Regarding LCM of AI/ML based positioning accuracy enhancement, at least for UE-side model (Case 1 and Case 2a), further study how to provide the applicable conditions (e.g., applicable configurations/areas/scenarios/environments) of a model/functionality to network during model/functionality identification process - FFS potential specification impact on the applicable conditions (including details if explicit signaling or implicit with no specification impact) |
| Apple | To clarify, is this from the UE to the network or vice versa ? |
| Moderator | Wording update into proposal 1-3a based on ZTE’s comment. |

Proposal 1-3a

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for UE-side model (Case 1 and Case 2a), further study how to provide the applicable conditions (e.g., applicable configuration/area/scenario/environment) of a model/functionality to network during model/functionality identification process

- FFS potential specification impact (including details if explicit signaling or implicit with no specification impact)

- Note: other type of information is not precluded for further study

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | To clarify, for UE based pos and UE side model, does it need to provide the applicable conditions to network or UE to decide by itself? |
| Moderator | To Samsung: It’s part of our FFS to decide. |
| NEC | Support in principle |
| Ericsson | Do not support. We don’t see that the UE has to provide the applicable condition to network. The UE can make sure its applicable condition matches that of current network/environment. |
| ZTE | Support. In addition, we may need to consider the progress in 9.2.1(if any). Our understanding is that the applicable condition may finally be defined as UE features. It may be new UE features compared to existing UE features. |
| Xiaomi | It seems that current proposal requires UE report the applicable conditions mandatorily . We think it is too strong. We propose the following update Regarding LCM of AI/ML based positioning accuracy enhancement, at least for UE-side model (Case 1 and Case 2a), further study whether and how to provide the applicable conditions (e.g., applicable configuration/area/scenario/environment) of a model/functionality to network during model/functionality identification process - FFS potential specification impact (including details if explicit signaling or implicit with no specification impact) - Note: other type of information is not precluded for further study |
| NTT DOCOMO | Support |
| MediaTek | Support |
| Fujitsu | Support |
| CATT | Support. This version looks better. Xiaomi’s update is also OK, which seems more or less alleviate Ericsson’s concern. |
| vivo | We think model/functionality identification process may not necessarily be one way (from UE to NW only). So we suggest the following: Regarding LCM of AI/ML based positioning accuracy enhancement, at least for UE-side model (Case 1 and Case 2a), further study whether and how to provide the applicable conditions (e.g., applicable configuration/area/scenario/environment) of a model/functionality to network during model/functionality identification process - FFS potential specification impact (including details if explicit signaling or implicit with no specification impact) - Note: other type of information is not precluded for further study |
| InterDigital | Support |
| Lenovo | Support Xiaomi’s revision. No strong view on the note |
| NOK | Support the last suggestion proposed by vivo. |
| CAICT | Support vivo’s update. |
| LG | Fine with the vivo’s update |
| OPPO | Support vivo’s revision. |
| Qualcomm | We share some of the views listed by Ericsson in the previous round. However, the UE still needs to inform the LMF it has support for AI/ML positioning in an area (with probably some recommended conditions). The LMF can then activate AI/ML positioning for the UE side. This would require some indications between the LMF and UE. Please check the following suggested rewording to avoid confusions: Regarding LCM of AI/ML based positioning accuracy enhancement, at least for UE-side model cases (Case 1 and Case 2a), further study how and whether to provide the applicable conditions (e.g., applicable configuration/area/scenario/environment) of a model/functionality to network during model/functionality identification process - FFS potential specification impact (including details if explicit signaling or implicit with no specification impact) - Note: other type of information is not precluded for further study |
| CMCC | We are open to further discuss whether this is a two-way procedure. |
| HW/HiSi | We agree with what Ericsson also said, for the UE side model, every conditions, definition of scenario, etc, are up to the UE. This is transparent to the network (it seems there is not so much that the NW could do more than transmitting PRS). Therefore, we do not see the need for this proposal. |
| Apple | We think Vivo’s update with the “whether” and a discussion on whether it is one or two way needs to be discussed. |
| Moderator | Status of discussion: Ericsson and HW/HiSi are the two companies objecting to this proposal 1-3a. All other companies are OK with it. To Ericsson: It seems to me you assumed UE knows the environment by your comment “The UE can make sure its applicable condition matches that of current network/environment.” In that case, how does UE know the environment? Get that information from the network? In your previous comment toward proposal 1-3, you seemed acknowledging “information related to applicable (valid) area/scenario/environment of its model”. Now this proposal is asking to study “whether and how to” provide that information. Wording update below to address comments. |

##### Proposal 1-3b

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study whether and how to provide information related to the applicable conditions (e.g., applicable configuration/area/scenario/environment)

- FFS whether from UE to the network only or two way information exchange between UE and the network

- FFS potential specification impact (including details if explicit signaling or implicit with no specification impact)

- Note: other type of information is not precluded for further study

Companies are encouraged to provide comments especially if they object this proposal.

| Company Name | Comments/Views |
| --- | --- |
| Ericsson | Do not support. The UE has full knowledge of its models, its implementation capability, and the applicable condition for its models. If the UE has access to multiple models, the UE has the full knowledge of each. If a model is trained with mixed dataset, the UE knows what the mixture is. If the UE can fine-tune its model, the UE knows exactly how much, how fast it can update the model. In contrast, the network does not have any of such knowledge. Also, the network needs to serve a wide variety of UEs at the same time, it is difficult to keep track of each UE’s models even if the UE reports it. Also the generalization study indicates that there are many factors that affect the model performance. It is not practical for the UE to report the set of applicable conditions. For those condition that are easily matched with existing indication (e.g., cell ID, tracking area ID), the UE can check and match itself. For conditions that cannot be easily checked, e.g., different drops, the network is in a worse position than the UE to tell if the current drop is the same drop as that of training. The UE is in a better position to check, e.g., the UE can use its model monitoring mechanism to check if the current condition is acceptable. |
| HW/HiSi | Tend to agree with Ericsson that for the UE side model, the UE has full knowledge about the applicable conditions which are transparent to the network. Maybe for progress, it would be good if proponents could give an example could be given for an potential information exchange from NW to UE? |
| Samsung | At least for case 1, we hesitate to support this further study. As for UE based pos and also UE side model, UE should control/determine whether to use it, e.g., totally implementation-based usage of AI. Unless the use of AI is mandated is always controlled by network side, which we did not see such rules now. |
| Nokia/NSB | In Agenda item 9.2.1 two frameworks are being discussed. - AI/ML functionality identification; and - AI/ML model identification. We tend to agree with E// for the case of AI/ML model identification. However, for the case of AI/ML functionality identification, we believe that applicable conditions should be reported from UEs to NW, for example in Case 1 and Case 2a positioning may need disclosure by the UE if the UE itself requires some assistance from the NW to manage its own AI/ML models that support a specific functionality. For example, the UE may need help with: - Collecting and labeling training data: - When to trigger model updates: Our suggestion is to mention explicitly functionality-based LCM, because in 9.2.1 is not yet clear the framework related to AI/ML model identification. Thus, we propose the following rewording: “Regarding functionality-based LCM [and model-based LCM] of AI/ML based positioning accuracy enhancement …” |
| CATT | Support the latest version. This is aligned with the following part of a latest agreement achieve in 9.2.1, which implies discussion in each sub-use-case agenda, for both model-ID based and functionality-based LCM: Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.. So far, it is hard to say that there will be absolutely no impact in positioning use case. |
| ZTE | The same view as CATT, a new agreement was reached in 9.2.1. This proposal is to echo the yellow parts below, we need to study the positioning-specific parts for model/functionality identification. Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. |
| Apple | Same view as CATT and ZTE. |

## Model input and output

In RAN1#112, the following was agreed.

Agreement

Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement

- For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead

  - Potential new measurement: CIR/PDP

  - existing measurement: e.g., RSRP/RSRPP/RSTD

  - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied

  - Note2: study the impact of model input for other cases are not precluded

- For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF

  - new measurement report: e.g., ToA, path phase

  - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

  - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - RS configurations

  - Other assistance information is not precluded

Multiple companies discussed detailed aspects related to model input and output.

[1, Ericsson] proposed that for model inference of Case 2b/3b, PDP and DP are prioritized over CIR considering the smaller model input size and the limited specification impact. It proposed that for Case 2b, the IE for reporting model input should kept small (e.g., DP only). Full-size CIR is not considered. It proposed that for Case 3b, postpone the final specification impact discussion until it has been concluded based on evaluations the suitable information to report as model input.

[2, Huawei] proposed that for Case 2b, if justified by overhead and performance evaluation, to facilitate the data collection for initial model training, model updating and model inference of the AI/ML-model for positioning, study to support measurements (e.g. CIR, PDP) performed at PRU/UE and sent to LMF. It also proposed that for Case 3b, to facilitate the data collection for initial model training, model updating and model inference of the AI/ML-model for positioning, study to support measurements (e.g. CIR, PDP) performed at gNB and sent to LMF. It also proposed that at least for the direct AI/ML positioning, since the required measurement payload size to achieve a given accuracy target varies depending on deployment scenario and channel conditions, measurement reporting with flexible payload size should be supported.

[3, ZTE] proposed that at least for data collection of Case 2b and Case 3b, support UE/TRP to report more than 8 additional path timings and RSRPPs and to report path phase of a channel path in addition to path power and path timing. It also proposed for AI/ML assisted positioning, support following intermediate results as the model output: DL-RSTD values for first detected path; LOS/NLOS indicator; DL PRS-RSRPP values for first detected path. It also proposed for AI/ML assisted positioning, study measurement report enhancement for AI/ML assisted intermediate results under both single TRP and Multi-TRP construction.

[4, vivo] proposed to support time domain CIR as one model input for AI/ML based positioning. It also proposed that TRP-related information, such as encoded TRP ID and TPR’s location, should be incorporated into the model input for Construction 2 (Single TRP, same model for N TRP) of AI/ML assisted positioning without additional signaling overhead. It also proposed to support TOA as a new measurement reporting from UE side to NW side.

[6, Spreadtrum] proposed that for case 2a and case 3a, the output of AI/ML model can be considered as new metric, e.g., LOS/NLOS hard/soft judgement, path phase and for case 2b and case 3b, the input of AI/ML model can be considered as new metric, e.g., CIR/PDP.

[7, Nokia] proposed RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples.

[8, CATT] proposed that for case 2b, if PRU/UE generates and reports new measurement such as CIR to LMF side as the input of AI/ML model, two-sided AI/ML model can be considered..

[9, Baicells] proposed to study and enhance LPP/NRPPa to support diverse variants channel observation measurements e.g. CIR/PDP as model input.

[13, Google] proposed to study coverage enhancement for PRS to improve the measurement accuracy for CIR/PDP, which could be used as the input of ML based positioning and to study aspects on CIR measurement and report.

[17, CMCC] proposed that for AI/ML based positioning, the potential spec impact of CIR report can be studied, and the impact of dimension of CIR on positioning accuracy can be evaluated in AI 9.2.4.1.

[18, MediaTek] proposed that for Case1, Case 2a, Case2b, support collecting at least: PDP/CIR (or truncated, compressed); RSRP; Horizontal location; LOS/NLOS condition, TOA, DOA, and other intermediate metrics; scenario identifier.

[19, Fraunhofer] proposed to define new measurements for model inference input IQ reporting for the CIR.

[21, InterDigital] proposed to study feasibility of deriving CIR based on measurements specified in Rel. 17 and identify missing measurements to derive CIR. It also proposed to study how to indicate quality for CIR.

[22, Apple] proposed potential specification impact of CIR/PDP/L1-RSRP as model input for inference of direct AI/ML based positioning and of LOS/NLOS tap identification or TOA estimation as model output for AI/ML assisted positioning.

[23, Lenovo] proposed to support new measurements such as CIR and PRS RSSI for model inference inputs, while existing Rel-16/Rel-17 positioning measurements may also be re-used. It also proposed to support new measurement report elements such as TOA for both PRS and SRS while existing Rel-16/Rel-17 positioning measurement reporting elements may also be re-used, e.g., RSTD, LOS/NLOS indicator, RSRPP, etc..

[24, Qualcomm] proposed that for inference in Case2b, deprioritize CIR/PDP as new measurements due to their significant reporting overhead and minimal/comparable gain when compared to existing multipath reporting. It proposed that for inference in Case3b, study type of measurements while including both existing measurements, enhancements of existing measurements, or new measurements since reporting has less dependence on OTA resources. It also proposed that for inference in Case2a/3a, consider existing/enhanced measurements and new measurements (e.g., soft-info of time/angle).

[26, NTT DOCOMO] proposed that for direct AI/ML positioning (case 2b and 3b), study the report format of UE/gNB reporting CIR/PDP to LMF considering the signaling overhead reduction.

Summary of companies’ views on model input for direct AI/ML positioning (Case 2b and 3b)

Study to support CIR/PDP: [2, Huawei] (if justified by overhead and performance evaluation), [3, ZTE] (report in terms of path timing, RSRPP and path phase), [4, vivo], [6, Spreadtrum], [7, Nokia] (w/ overhead reduction), [8, CATT] (for two sided model), [9, Baicells], [13, Google], [17, CMCC], [18, MediaTek] (w/ overhead reduction), [19, Fraunhofer], [21, InterDigital], [22, Apple], [23, Lenovo], [26, NTT DOCOMO] (w/ overhead reduction)

PDP and DP are prioritized over CIR: [1, Ericsson]

Deprioritize CIR/PDP for case 2b: [24, Qualcomm]

Moderator’s comment:

Moderator’s understanding of information contained by different types of channel measurement

CIR: timing of path delay, power of path, phase of path

PDP: power and delay (timing) of path

DP: delay timing of path

RSRPP: the power of the linear average of the channel response at the i-th path delay

Note that Rel-17 specification supports reporting up to 8 additional detected path timing values.

The following objective is included in Rel-18 positioning WID (RP-230328)

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

In RAN1#112, it has been agreed in Rel-18 positioning WI to introduce carrier phase measurement in principle and the definitions of carrier phase for DL-PRS and UL SRS have been agreed. It can be safely expected that new measurement of carrier phase will be specified in Rel-18.

Considering exiting Rel-17 measurement report (power and timing for multiple path) and expected Rel-18 measurement report (path phase), it does not make sense to restrict future AI/ML model development (which may be specified in Rel-19 or later) to not utilize all those available measurements that contains more information of channel which in turn improves the AI/ML based positioning accuracy.

Given the above majority companies’ view, the following proposal is formulated to recognize those candidate measurements for model input and to focus further study on potential specification impact of signaling overhead reduction.

Proposal 1-4-1

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified for AI/ML based positioning accuracy enhancement

- CIR, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account exiting Rel-16/17 measurement and/or expected Rel-18 measurement

- PDP, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement (RSRPP/RSRP/RSTD), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) in addition to for model inference and/or for other cases can also be discussed

- Note2: combinations of multiple measurements are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | To all: Please read all the paragraphs under “Moderator’s comment” before Proposal 1-4-1 for summary of companies’ view and the motivation of proposal 1-4-1. |
| Fujitsu | Support. |
| CMCC | Support. |
| NEC | Support |
| NOK | We are ok with the direction of the proposal. However, we should indicate in an extra note that CIR/PDP are new measurements, and the calculation methodology calculation is for further study. |
| Ericsson | - For Case 2b: we think CIR should not be supported unless it’s proven that the signaling overhead for sending it to LMF is not excessive. The AI/ML situation is different than Rel-18 discussion of carrier phase, since phase info from many TRPs, many paths, multiple RX ports are needed according to existing evaluations. - Need to add DP, which has been shown by R1-2302335 evaluation to work. In fact, DP is in existing spec. For example, for downlink, PRS-RSRPP is optional. When RSRPP is absent, PDP becomes DP. For uplink, UL-SRS-RSRPP is also optional. If not including DP, then RSRPP is mandatorily provided, which is not true. NR-AdditionalPath-r16 ::= SEQUENCE { nr-RelativeTimeDifference-r16 CHOICE { k0-r16 INTEGER(0..16351), k1-r16 INTEGER(0..8176), k2-r16 INTEGER(0..4088), k3-r16 INTEGER(0..2044), k4-r16 INTEGER(0..1022), k5-r16 INTEGER(0..511), ... }, nr-PathQuality-r16 NR-TimingQuality-r16 OPTIONAL, ..., [[ nr-DL-PRS-RSRPP-r17 INTEGER (0..126) OPTIONAL ]] } |
| Lenovo | Supportive of the proposal in general. One typo correction in the Note: “Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement” |
| NVIDIA | Support. |
| LG | Fine with the proposal |
| vivo | Support. |
| Intel | Support the proposal. |
| Samsung | I think we have potential post processing on measurement like CIR for further studying. So the note 2 could be revised as “• Note2: potential combinations of multiple measurements or post processing of the measurement(s) are not precluded” |
| CATT | We are fine with this proposal. |
| CAICT | Support. |
| NTT DOCOMO | Support |
| Xiaomi | Support |
| OPPO | Fine with the proposal. |
| HW/HiSi | Support |
| Qualcomm | This should be postponed until we finish evaluating performance vs. reporting overhead tradeoff in the evaluation item 9.2.4.1 and draw related conclusions on necessity and feasibility. The discussion is still at early stage in the evaluation item and many aspects for evaluating reporting overhead needs to be agreed yet. We also share same thought as Ericsson’s first bullet. |
| ZTE | Support |
| Baicells | Support. |
| Apple | Support |
| Fraunhofer | CIR and PDP are widely adapted terminologies, which do not perfectly fit the intention as per Moderator definition: so PDP provides a statistical representation of the power distribution of the received signal over different time delays and CIR is the time delay representation. Therefore, we question the need to include terms such as 'CIR/PDP' in the proposal if the only difference is the phase information. For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified for AI/ML based positioning accuracy enhancement - New measurements CIR, which contains: - path timing, power and phase information of the channel response - FFS potential specification impact of measurement request and report including overhead reduction - Note: take into account exiting Rel-16/17 measurement and/or expected Rel-18 measurement - PDP, which contains path timing and power information of the channel response - FFS potential specification impact including enhancement to existing measurement (e.g., RSRPP and timing for multi-path) and/or overhead reduction - Existing measurement (RSRPP/RSRP/RSTD), which contains path timing or power information of the channel response - … |
| InterDigital | Support |
| Moderator | To Ericsson: On your comment of DP, your argument is not correct and misleading. I copied below from 37.355. nr-AdditionalPathList-r16 and nr-AdditionalPathListExt-r17 for additional path timing are also optional. If you think RSRPP is not always available, then the same applies to DP. -- ASN1START NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE { dl-PRS-ReferenceInfo-r16 DL-PRS-ID-Info-r16, nr-DL-TDOA-MeasList-r16 NR-DL-TDOA-MeasList-r16, ..., [[ nr-UE-RxTEG-TimingErrorMargin-r17 TEG-TimingErrorMargin-r17 OPTIONAL -- Cond UERxTEG ]] } NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-Result-r17 INTEGER (0..126) OPTIONAL, nr-los-nlos-Indicator-r17 CHOICE { perTRP-r17 LOS-NLOS-Indicator-r17, perResource-r17 LOS-NLOS-Indicator-r17 } OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL, nr-DL-TDOA-AdditionalMeasurementsExt-r17 NR-DL-TDOA-AdditionalMeasurementsExt-r17 OPTIONAL ]] } NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-ResultDiff-r16 CHOICE { k0-r16 INTEGER (0..8191), k1-r16 INTEGER (0..4095), k2-r16 INTEGER (0..2047), k3-r16 INTEGER (0..1023), k4-r16 INTEGER (0..511), k5-r16 INTEGER (0..255), ... }, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-ResultDiff-r16 INTEGER (0..61) OPTIONAL, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 INTEGER (0..61) OPTIONAL, nr-los-nlos-IndicatorPerResource-r17 LOS-NLOS-Indicator-r17 OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL ]] } -- ASN1STOP To Ericsson and Qualcomm: Even your evaluation results on CIR/PDP showed performance benefits. Are you questioning your own results? All the concern on signaling overhead is part of FFS on overhead reduction. But why prevent AI/ML model to utilize information contained in measurement which is already or expected to be available in specification? Wording update below into proposal 1-4-1a. |

Proposal 1-4-1a

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified for AI/ML based positioning accuracy enhancement

- Potential new measurement, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement

- Potential new measurement, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement (RSRPP/RSRP/RSTD, ), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) in addition to for model inference and/or for other cases can also be discussed

- Note2: combinations of multiple measurements are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | Wording update below into proposal 1-4-1b. |

Proposal 1-4-1b

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified for AI/ML based positioning accuracy enhancement

- Potential new measurement, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement

- Potential new measurement, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement (RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) in addition to for model inference and/or for other cases can also be discussed

- Note2: combinations of multiple measurements are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | I think we have potential post processing on measurement like CIR for further studying. So the note 2 could be revised as “• Note2: potential combinations of multiple measurements or post processing of the measurement(s) are not precluded” |
| Moderator | Wording update to address Samsung’s comment |

Proposal 1-4-1c

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified for AI/ML based positioning accuracy enhancement

- Potential new measurement, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement

- Potential new measurement, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement (RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) in addition to for model inference and/or for other cases can also be discussed

- Note2: potential combinations of multiple measurements and/or post processing of the measurement(s) are not precluded

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | To Ericsson and Qualcomm regarding CIR for Case 2b: Ericsson commented that “The AI/ML situation is different than Rel-18 discussion of carrier phase, since phase info from many TRPs, many paths, multiple RX ports are needed according to existing evaluations.” Qualcomm said “shared thought as Ericsson’s first bullet” 1. First of all, no multi-port PRS as of now. Is it Ericsson’s view that multi-port PRS is necessary for AI/ML positioning? 2. It seems you are assuming carrier phase positioning in Rel-18 can be done with a single path measurement for a single TRP. I don’t want to give wrong answer since that’s part of discussion on going in Rel-18 positioning WI now. Just one example on existing Rel-16/17 RSTD report. For NR DL-TDOA, UE can report multiple RSTDs per TRP pair for multiple TRPs as requested by NW. If there’re multiple PRS resource per TRP, that measurement report can be even larger. So LMF configures UE to report RSTD for multiple TRPs, can also configure whether to have RSRPP for each path and/or additional path, whether to have AdditionalPath or not. I don’t see any difference on the usage of measurement report between convention positioning and AI/ML based positioning. 3. The specification provides multiple options of measurement. I believe it is developed to support multiple positioning methods as well as flexible deployment. You are arguing “performance vs. reporting overhead tradeoff”. Exactly because of that, we should have multiple options for our usage. For example, when positioning accuracy requirement is not high or not in a challenging environment, an AI/ML model with maybe first path timing is enough. On the other hand, when accuracy requirement is high and/or in a challenging environment, an AI/ML model with the most information of channel response gives the best performance. Same thing for overhead. When we have overhead constraint, we may use model with less overhead. However, in case we don’t have very strict overhead constraint, why restrict ourselves on the type of measurement used for AI/ML model especially if such measurement already or expected to be in the specification? |
| Ericsson | One clarification - It is true that evaluation demonstrates CIR can be used to get higher accuracy. However, this does not mean that CIR need to be used for Case 2b/3b. It can be used by Case 1/2a/3a. On difference between measurement report for convention positioning and AI/ML based positioning: based on PDP evaluation so far, more taps are needed for AI/ML than in existing spec. We speculate that the same will be true for CIR, assuming Rel-18 defines new measurement for CIR. Thus measurement report size for Case 2b/3b can be many times bigger than conventional method. Our analysis indicate that CIR is the worst in terms of signalling overhead. Suggest to wait for more signaling overhead analysis from 9.2.4.1 before making a decision on Case 2b/3b. Between Case 2b/3b, we have to be very careful about 2b since measurement report is sent over the air. Case 3b can tolerate bigger measurement report. |
| ZTE | Support. We think it’s an important issue so that NW can get enriched channel information. Regarding the overhead issue, we can further study ways ti reduce overhead. It cannot the reason to block the measurement enhancement as specifications are always flexible. When/how to support such features is still subject to UE capability and NW scheduling. But the specification should allow such flexibility. In addition, we suggest to reorder the third bullet as following: - FFS potential specification impact including enhancement to existing measurement - Note: Existing measurement (RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response |
| Xiaomi | Support in principle |
| NTT DOCOMO | Support |
| Fujitsu | Basically support. We also support ZTE’s proposed reorder. And in general, we think “e.g.,” can be added in the bracket or remove the bracket because all sort of existing measurement defined in TS37.355 can be enhanced to support AI/ML positioning so no need to point out specific examples. |
| CATT | Support. |
| vivo | Support |
| InterDigital | Support |
| Lenovo | Supportive of FL’s proposal. On the 3rd bullet, with the inclusion of Additional Path, it is not exactly an existing measurement but a measurement reporting option, e.g., first path or additional paths can apply to RSRP and RSTD. |
| NOK | Support. If possible, we suggest merging bullet 1 and bullet 2. We have a rewording for Note1 to be aligned with agreements done in the agenda item 9.2.1: - Note1: whether such measurement request and report can be applied to other aspects of AI/ML model/functionality-based LCM (e.g., training, updating, monitoring, etc.) in addition to for model inference and/or for other cases can also be discussed. |
| CAICT | Support |
| LG | The difference of the first/second bullet is only for phase information and it would be better to merge for potential new measurment |
| OPPO | Support. |
| Qualcomm | We do not support the first two bullets. Reasons are explained in our previous comment. To Moderator: What we observe is that for a given overhead, the existing measurements has better accuracy than the new optimized CIR measurements. Thus, the reporting overhead of new measurements is not justified, and comparable performance gains can be claimed using existing reporting with smaller overhead. In addition, most companies have not yet tried training Case2b and Case3b with existing multipath measurements and they have not yet provided overhead calculations. It is too early to support the two first bullets until we see satisfactory justifications for any additional reporting head required by the new optimized measurements. The first bullets need to be postponed until we finish evaluations and draw conclusions in 9.2.4.1. |
| HW/HiSi | Support. Even fine to remove the word “potential” from the first two bullets. In our view, companies have provided a large number of results showing that the overhead is controllable. |
| Fraunhofer | Support |
| Apple | Support. Fine with the word potential. |
| Moderator | Status of companies’ view (based on comment toward proposal 1-4-1 to 1-4-1c): Support: Fujitsu, CMCC, NEC, NOK, Lenovo, NVIDIA, LG, vivo, Intel, Samsung, CATT, CAICT, NTT DOCOMO, Xiaomi, OPPO, HW/HiSi, ZTE, Baicells, Apple, Fraunhofer, InterDigital (21) No CIR for Case 2b: Ericsson, Qualcomm To Qualcomm and Ericsson: You have your observation based on your results. They are multiple other companies who also provided results to show performance gain of CIR/PDP vs. existing measurement for a comparable overhead. It seems to me your focus in on overhead while performance is lower priority. As I previously commented, overhead reduction is already captured as the aspect for FFS. I don’t agree with you that overhead concern should block the possibility of utilizing potential new measurement for AI/ML when overhead is not the first priority to begin with. Wording update to address comments from ZTE, Fujitsu, Lenovo, NOK and another attempt to move forward. |

##### Proposal 1-4-1d

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified as candidates providing performance benefits for AI/ML based positioning accuracy enhancement

- Potential new measurement, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement

- Potential new measurement, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement report (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement and/or existing measurement report (e.g., RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement report

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML LCM (e.g., monitoring, etc.) and/or for other Cases can also be discussed

- Note2: potential combinations of multiple measurements and/or post processing of the measurement(s) are not precluded

Companies are encouraged to provide comments especially if they object this proposal.

| Company Name | Comments/Views |
| --- | --- |
| Ericsson | We still have strong concern on the measurement report size, especially for CIR. A typical model input companies evaluated is, CIR of size 18 (TRP) * 2 (ports) * 256 * 2 (I/Q), that’s 18,432 real values to report for one instance of position estimation. This is substantially bigger than any of the existing measurement. We don’t agree that the size can be ignored, because the measurement report from UE has to be sent over the air using LPP, and LPP is on top of RRC. The measurement size cannot be so enormous. Also: we think delay profile is useful measurement. Thus for PDP bullet, power info may or may not be included, the same as in existing spec. In 9.2.4.1, several companies show analysis of measurement report size for different ways of building the AI/ML model. Recommend to delay this topic till a good amount of analysis is available for discussion. |
| HW/HiSi | No concern. But as stated earlier, we think the word “potential” could be removed from the first 2 bullets. Disagree with Ericsson that the typical input is 18 TRPs and the CIR length is 256. In our simulations we have shown that 4 TRPs and 32 taps can be sufficient in many cases (which would result into 512 values (instead of 18,432 according to E///’s), or when using PDP, we would end up with 256 values |
| Samsung | Even though we share the same view as E/// on that the raw CIR data with high dimension is not practical for OTA transfer, we think this proposal is still valuable, and that’s also the reason we insists to adding the post processing in the Note2, because the dimension/size reduction should be played from our point of view to really make AI being attractive to practical usage. This is also one important evaluation aspect in 9.2.4.1; To solve the concern for DP without power infor, we think it might be ok to change “path timing, power and phase information” to “path timing and/or power and/or phase information”; |
| Moderator | To Ericsson: 1. There’s no CIR mentioned in this proposal. If you are referring to the 1st bullet. Is it correct understanding that Ericsson objects this statement “Potential new measurement, which contains path timing, power and phase information of the channel response, as candidates for direct AI/ML positioning with LMF-side model (Case 2b and 3b), providing performance benefits for AI/ML based positioning accuracy enhancement”? 2. You argued based on your assumption of measurement report size for a potential new measurement. There’re already evaluation results from companies to show that’s not the case. FFS is there on how we can reduce the measurement report size. 3. In your previous comment, you said DP is carried in AdditionalPath in existing specification. That’s already captured in the 3rd bullet “Existing measurement and/or existing measurement report (e.g., RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response” |
| Qualcomm | The first two bullets are already listed as potential new measurements in a previous agreement (RAN1-112). The current version does not seem to add much compared to the previous agreement. Once again, let’s postpone this until we finish the evaluation on performance and reporting overhead tradeoff. We also suggest companies be considerate on the need to align companies on calculating reporting overhead for different potential new and existing measurements in the other item (9.2.4.1). We suggest the following rewording if Moderator keeps insisting on not postponing the discussion: For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified as candidates providing performance benefits for AI/ML based positioning accuracy enhancement - Potential new measurement, which contains path timing, power and phase information of the channel response - FFS potential specification impact of measurement request and report including overhead reduction - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement - Potential new measurement, which contains path timing and power information of the channel response - FFS potential specification impact including enhancement to existing measurement report (e.g., RSRPP and timing for multi-path) and/or overhead reduction - Existing measurement and/or existing measurement report (e.g., RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response - FFS potential specification impact including enhancement to existing measurement report - Note1: whether such measurement request and report can be applied to other aspects of AI/ML LCM (e.g., monitoring, etc.) and/or for other Cases can also be discussed - Note2: potential combinations of multiple measurements and/or post processing of the measurement(s) are not precluded - Note3: Other candidates (e.g., new measurements) are not precluded |
| Moderator | To Qualcomm: Previous agreement Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded This proposal is one step further on top of last meeting’s agreement. It acknowledges the performance benefits of all those measurements after our study, so next meeting we can focus on signaling overhead. I don’t see how your suggestion is constructive when there’re more than 20 companies support this proposal with the first two bullet included. |
| CATT | Support. This is a small step further from previous agreement based on observation through many companies’ papers. |
| ZTE | No concern. Better to remove ‘potential’. Majority companies’ simulation results show the need on measurement enhancement. Our evaluation results show the obvious performance increase on large number of Nt’. The concern on overhead is invalid. One example is that Nt’=32 shows better positioning accuracy that Nt’=32(currently supported by spec). Another example, in reality, it doesn’t mean 18* TRPs are always necessary. AI/ML positioning can even rely on single TRP. The situation is that the more enriched channel information being reported, the more performance gain is observed and less labeling data is required. |
| Apple | Fine with this. There is a possibility of mixed inputs where we have CIR for a subset of the TRPs and RSRP/PDP/(DP) for others. |

Moderator’s comment:

Several companies indicated their preference of model output for case 2a and 3a. Among them, TOA and LOS/NLOS indicator are those two preferred by more companies than other measurements. This is also aligned with available evaluation results in agenda 9.2.4.1 for AI/ML assisted positioning where vast majority of evaluation results are output TOA and LOS/NLOS indicator and showing positioning accuracy improvement. The following proposal is formulated to recognize majority view and to focus further study on potential specification impact. Other measurement as model output are still open for further study if justified by performance evaluations

Proposal 1-4-2

For assisted AI/ML positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), at least the following are identified as measurement report to carry model inference output to LMF

- TOA

  - FFS potential specification impact including details of measurement request and report, e.g., soft information report

- LOS/NLOS indicator

  - FFS potential specification impact (if any w.r.t. existing measurement report)

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) can also be discussed

- Note2: other measurement as model inference output for Case 2a and 3a are not precluded for further study

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| NEC | It seems to this proposal is coincident with the agreement from last meeting. Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded. |
| Moderator | To NEC: Previous agreement you quoted says “to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for …” where ToA, LOS/NLOS indicator are listed as examples. The intention of this proposal is to agree on these two measurement report (ToA, LOS/NLOS indicator) when it says “the following are identified as measurement report to carry model inference output to LMF”. Hope this clarifies. |
| NOK | Some clarification is needed: - First main bullet. Can we include RSTD?, at the end RSTD contains TOA information. |
| LG | Fine with the proposal |
| vivo | Support |
| CATT | We are fine with the proposal. |
| CAICT | Support. |
| NTT DOCOMO | Support |
| OPPO | Fine with the proposal. |
| HW/HiSi | Support |
| Qualcomm | - For ToA reporting, it is not clear to us whether ToA can produce better gain than RSTD measurements while introducing UE timing errors. - We also propose to include soft information for RSTD measurements. - We also share the view of NEC |
| ZTE | Prefer to add RSTD as commented by Nokia. It’s also evaluated in 9.2.4.1 by companies. In addition, we also think it’s beneficial to increase RSRPP reliability by using AI/ML model although there is no specific evaluation results in 9.2.4.1. |
| Apple | Fine with proposal |
| Fraunhofer | For the NG-RAN Case3-a, AoA should also be included |
| Moderator | Wording update below into proposal 1-4-2a. |

Proposal 1-4-2a

For assisted AI/ML positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), at least the following are identified as measurement report to carry model inference output to LMF

- TOA

  - FFS potential specification impact including details of measurement request and report, e.g., soft information report

- LOS/NLOS indicator

  - FFS potential specification impact (if any w.r.t. existing measurement report)

- FFS RSTD (including soft information report) for Case 2a

- FFS RSRPP for Case 2a and Case 3a

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML model LCM (e.g., training, updating, monitoring, etc.) can also be discussed

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| NEC | Agree. |
| Ericsson | Strong concern on TOA. When considering measurement report, we think the way timing info is sent in existing spec should be used. It’s not practical to send actual clock time of arrival, since this requires TRPs and UEs to have their clocks tightly synchronized, which is not true in real network. Timing info will need to be sent in the format of relative timing somehow, similar to RSTD and UE RxTxTimeDiff for DL, and RTOA and gNB RxTxTimeDiff for UL. Suggest to change TOA to “Timing information” to be generic. |
| ZTE | The RSTD should not be FFS. It can be described the same as TOA. - RSTD - FFS potential specification impact including details of measurement request and report, e.g., soft information report |
| NTT DOCOMO | Support |
| CATT | Support |
| vivo | We have concern on supporting RSTD as model output for Case 2a. 1. The original motivation to have RSTD for DL-TDOA is to address UE timing error in conventional positioning. However, such motivation/reason for RSTD is not necessary any more for AI/ML based positioning. As many companies have shown in their evaluations, the issue of UE timing error can be handled in AI/ML based positioning with mixed training dataset and/or fine-tuning etc. 2. We don’t see the benefit of RSTD reporting when the model is trained with UE channel measurement for each TRP. We showed in R1-2302481 (Table 9 and 10), single TRP, N models, positioning accuracy of RSTD (calculated by difference of ToAs) is much worse than that of direct ToA for the same number of TRP with and without UE timing error. 3. If a model is special trained with measurement on a pair of TRP (to align with the definition of RSTD), then we believe the training data would be much larger. Take the example of 18 TRPs in the evaluated deployment, there’re 153 combinations of TRP pair. Assuming the same UE density, that’s 8.5 times of training data compared to 18 TRP measurement. On the added FFS bullet, can the proponent of RSTD explain what’s the technical benefits of RSTD compared to TOA? Otherwise, we don’t know why and what to study on RSTD. |
| Lenovo | Ok to support latest revised proposal. |
| NOK | Ok with the proposal. We have only a rewording in second main bullet: - LOS/NLOS indicator (hard and soft) - FFS potential specification impact (if any w.r.t. existing measurement report) |
| CAICT | Support |
| LG | To our understanding, all the listed measurement reports can be soft information report |
| Qualcomm | We think the benefits of ToA and its gain on top of any other timing reporting option requires additional evaluation. We propose to postpone ToA until more evaluations are done to gauge its gain over RSTD reporting, especially when timing errors and clock mis-synchronization are included. We also share ZTE’s view on RSTD. The RSTD should be the baseline for timing reporting. |
| HW/HiSi | Support |
| Apple | Okay |
| Moderator | To NOK: Current wording does not limit the type of LOS/NLOS indicator. Unless we have new agreement to remove one or the other from specification, I don’t think we need to add “(hard and soft)”. To Ericsson: ToA measurement is well understood and assumed in existing specification. For instance, nr-TimeStamp This field specifies the time instance at which the TOA and DL PRS-RSRP/RSRPP (if included) measurement is performed. The nr-SFN and nr-Slot in IE NR-TimeStamp correspond to the TRP provided in dl-PRS-ReferenceInfo as specified in TS 38.214 [45]. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. In fact, all these DL-RSTD, RTT, UL-RTOA and Additional Path report are derived based on TOA measurement by their definitions. I also checked Ericsson’s evaluation in agenda 9.2.4.1, it seems you also indicated ToA as model output for AI/ML assisted positioning. As I mentioned in my summary, this proposal is made based on majority companies’ view (including their evaluations). Using a generic term “timing information” which could cover any timing information is not accurate or correct. To Qualcomm, ZTE: Qualcomm and ZTE evaluated RSTD w/o comparison to TOA. vivo raised concerns to RSTD based on their evaluation comparison of TOA vs. RSTD. It seems the proponent of RSTD have not provided answer to vivo on their question. I don’t understand the argument of RTSD as baseline, RSTD is not even defined for RTT, UL-RTOA. Also see my above comment to Ericsson, RSTD is derived based on UE ToA measurement of DL-PRS. It’s up to the proponent company of RSTD to convince the group the benefits of RSTD. Before that, I think it’s fair to put FFS on it. To all: If companies don’t want to agree on the report for now, then I suggest we at least confirm the type of model output and the details of potential specification impact (details of report) is FFS, wording update below into proposal 1-4-2b. |

##### Proposal 1-4-2b

For assisted AI/ML positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), at least the following types of model inference output are identified as candidates providing performance benefits

- TOA estimation at least for a TRP or a resource

  - FFS potential specification impact including details of report to LMF, e.g., time difference relative to a reference time, soft information report

- LOS/NLOS indicator

  - FFS potential specification impact (if any w.r.t. existing measurement report)

- FFS RSTD as output where estimation is for a pair of TRPs or resources

  - For Case 2a DL positioning

  - Note: soft information report is not precluded

- FFS RSRPP for Case 2a and Case 3a

- Note1: whether the above can be applied to other aspects of AI/ML LCM (e.g., training, updating, monitoring, etc.) can also be discussed

Companies are encouraged to provide comments especially if they object this proposal.

| Company Name | Comments/Views |
| --- | --- |
| Ericsson | We still have strong concern on TOA as model output to be specified. In evaluation, yes, TOA is used in our simulation. However, that’s just for running simulation where real life issue like UE/TRP clock synchronization is ignored. Also in simulation, it’s unnecessary and difficult to exactly simulate RSTD or RTOA according to their definition (e.g., start of subframe j carrying SRS). For actual measurement reporting, we intend that TOA should be translated to RSTD or UE RxTxTimeDiff or RTOA or gNB RxTxTimeDiff by applying their formal definition. Thus, we support to reuse existing timing measurement defined in 38.215 as a baseline, i.e., DL RSTD and UE RxTxTimeDiff for UE side timing measurement, and UL RTOA and gNB RxTxTimeDiff for gNB side. BTW: we are confused about the IEs listed by Moderator. Even though the spec uses ‘TOA’ here and there in explanation text, the actual timing IE being reported are the measurement quantities defined in 38.215. |
| Moderator | To Ericsson: This main sentence stated “the following types of model inference output are identified as candidates providing performance benefits”. It does NOT endorse measurement report to LMF when “details report to LMF, e.g., time difference relative to a reference time, soft information report” are in FFS. You said “we intend that TOA should be translated to RSTD or UE RxTxTimeDiff or RTOA or gNB RxTxTimeDiff by applying their formal definition”. Let’s look at all the reports by definitions. RSTD = TOA (Rx of PRS from TRP i) – TOA (Rx of PRS from TRP j) UE RTT = TOA (Rx of PRS from TRP) – Tx UL-RTOA = TOA (Rx of SRS at TRP) gNB RTT = TOA (Rx of SRS at TRP) – Tx The Tx time is deterministic and not by AI model estimation. TOA is the inference output from AI model. |
| Moderator | Corrected a typo in above reply to Ericsson. |
| Qualcomm | We suggest the Moderator to elaborate on the physical meaning of ToA. Is ToA meant as the absolute timing of arrival? The estimated ToA can be floating due to UE clock drift. The UE and network clock’s are also not tightly coupled and synchronized. We understand that many companies trained their models with ToA info as output, but that is basically a time of flight (ToF) information (the simulation methodology does not account for UE-network clock synchronization issues). We suggest to first align and clarify understanding of ToA by different companies and do more evaluations to understand ToA/ToF vs. RSTD gains when timing errors and clock drift are included. |
| Moderator | To Qualcomm: This is the 6th RAN1 meeting since the start of SI. We made multiple agreements in all these meeting in 9.2.4.1 and 9.2.4.2 with the term TOA. |
| CATT | Support current version which is more cautious. Combining the main bullet, it just means ‘TOA is identified as a candidate of model output’. It is not saying TOA is going to be reported to LMF. If needed, it may be transformed to other format, which is exactly the FFS part. |
| ZTE | We would agree with Qualcomm that the simulation conducted so far is time of flight (ToF). TOA may be impacted by clock drift. RSTD has its advantage to eliminate some timing errors. We don’t agree the following statement. If all RSTD values are estimated relatively to the same reference TRP, there are only 17 TRP pairs. If a model is special trained with measurement on a pair of TRP (to align with the definition of RSTD), then we believe the training data would be much larger. Take the example of 18 TRPs in the evaluated deployment, there’re 153 combinations of TRP pair. Assuming the same UE density, that’s 8.5 times of training data compared to 18 TRP measurement. In the yellow part below, if the reference time is determined by a reference TRP, it’s the same as RSTD. - TOA estimation at least for a TRP or a resource - FFS potential specification impact including details of report to LMF, e.g., time difference relative to a reference time, soft information report The issue is that AI/ML model training require normalization on label. If the model output is RSTD values relatively to the same reference TRP, it naturally applies the normalization to eliminate the bias. There is no need to define another reference time. |
| vivo | To ZTE: On your comment, “If all RSTD values are estimated relatively to the same reference TRP, there are only 17 TRP pairs.”. Yes, it is true when you always fix the reference TRP. Note that the reference TRP for UE RSTD is configured by LMF and not necessarily the serving gNB. It would be up to the proponent of RSTD estimation for a pair of TRPs to show whether such restriction on training data collection (with a fixed reference TRP) may provide performance benefits or not. |
| Moderator | To all: It seems my previous comment at the end of proposal 1-4-2a may be missed by some companies. I copied below again. “If companies don’t want to agree on the report for now, then I suggest we at least confirm the type of model output and the details of potential specification impact (details of report) is FFS” Please read this proposal word by word. This proposal is only saying “types of model inference output are identified as candidates providing performance benefits”. Potential specification impact on what and how to report to LMF is all for further study. |

# Discussion on prioritization

## Individual observations/proposals

The following are individual observations and proposals from the contributions.

| Sources | Observations/proposals |
| --- | --- |
| [1, Ericsson] | Proposal 1 For the use case of positioning accuracy enhancement, model transfer (case z1-z5) is not considered. Proposal 4 For the use case of positioning accuracy enhancement, prioritize the study of offline AI/ML model training in Release 18. Proposal 5 For the positioning use case, online fine-tuning of models is not considered. |
| [2, Huawei] | Proposal 1: For Case 1 (UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning), the model training/updating and inference are performed all at UE side. Proposal 3: For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at UE side. Proposal 5: For Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF side. Proposal 9: For Case 3a (NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at gNB side. Proposal 11: For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF side. |
| [4, vivo] | Observation 1: Model transfer over air interface can be achieved with extremely low signaling overhead by combining small-parameter model design and advanced model quantization technologies. Proposal 12: Further study the overhead of model transfer, and support model transfer over air interface for AI/ML based positioning. Proposal 13: For the case where model is developed at network side and deployed at UE side, network side should transfer the model information to the target UE. |
| [5, OPPO] | Proposal 1: For AI/ML assisted positioning, the following alternative should be prioritized if the TOA-like output is used for AI/ML model - The measurement results corresponding to all TRPs are used as the input for AI/ML model inference (i.e., Multi-TRP construction). Proposal 7: For both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18 - Enhancements dedicated to online training can be discussed in the future release(s) Proposal 8: For both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer - AI model training and inference at UE side, or - AI model training and inference at NW side - Study model transfer in future release(s). Proposal 13: For UE-assisted positioning method with AI/ML assisted positioning (Case 2a), collaboration level y is prioritized. Proposal 14: For NG-RAN node assisted positioning method with AI/ML assisted positioning (Case 3a), collaboration level y is prioritized. Proposal 15: For UE-based positioning method with direct AI/ML positioning (Case 1), collaboration level y is prioritized for direct AI/ML positioning in Rel-18 Proposal 16: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing UE measurement and reporting (e.g., the scheme “Direct: DL RSTD +RSRP”) - Note: Only from the perspective of AI model inference Proposal 17: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of UE measurement/reporting (e.g., the scheme “Direct: Normalized CIR + RSRP”) - Note: Only from the perspective of AI model inference Proposal 18: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing TRP measurement and reporting - Note: Only from the perspective of AI model inference Proposal 19: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of TRP measurement/reporting - Note: Only from the perspective of AI model inference |
| [6, Spreadtrum] | Observation 1: For both of direct AI/ML positioning and AI/ML assisted positioning, AI/ML model can be delivered or not. It can wait for the progress of AI9.2.1. Proposal 1: For both of direct AI/ML positioning and AI/ML assisted positioning, suggest to focus on the training and inference located at the same side at present. |
| [8, CATT] | Observation 2: Training AI/ML model for positioning at network side is more feasible due to easier data collection and stronger computational resources. Proposal 4: For case 1, case 2a and case 3a, if UE-side model and gNB-side model is trained at LMF side, LMF can collect a large-scale dataset from numerous UEs/PRUs/gNBs and transfer the trained AI/ML model to UE/gNB side. |
| [10, Sony] | Proposal 1: Support AI/ML with model transfer in which the inference model is either in UE or gNB and LMF to create and train the AI/ML model. Proposal 5: Support AI/ML Positioning with model training at LMF and model inference at the UE side. Proposal 6: On AI/ML model indication, define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB. |
| [12, xiaomi] | Proposal 3: For the AI model delivery to UE, delivering from LMF to UE is considered as the baseline |
| [16, CAICT] | Proposal 5: For UE-based positioning with UE-side model, AI/ML model delivery from NW should be considered for Case1. |
| [17, CMCC] | Proposal 1: For AI/ML enabled positioning accuracy enhancement, all the collaboration levels (Level x/y/z) defined in AI 9.2.1 can be considered. The details of model transfer can be discussed in AI 9.2.1. |
| [20, NVIDIA] | Observation 1: AI/ML techniques can be used to learn the mapping of RF measurements to position. Proposal 1: High accuracy positioning in heavy NLOS scenarios should be the target of using AI/ML for positioning enhancement. Proposal 2: AI/ML techniques used to learn the mapping of RF measurements to position (i.e., direct AI/ML positioning) should be studied for positioning enhancement. Proposal 3: AI/ML techniques used to provide intermediate estimates such as LOS/NLOS classification (i.e., AI/ML assisted positioning) should be studied for positioning enhancement. |
| [23, Lenovo] | Proposal 11: Support model transfer using associated Model IDs and meta-information to help facilitate the transfer to other network entities/UEs. Proposal 15: Consider the following additional aspects with respect to the network-UE collaboration levels y and z including the associated sub-levels: - Data collection for training/inference - Model Life Cycle Management (including model acquisition, activation/deactivation of AI/ML models, model monitoring and update at the LMF, serving and neighbouring gNBs, and target-UE) - Model inference Proposal 16: Further study mechanisms to enable efficient positioning AI/ML model transfer between UE, gNB and LMF. |
| [24, Qualcomm] | Proposal 1: For AI/ML positioning model training, study model training at the side at which the inference for that model is expected to be performed: - Case1: Model training and inference at UE side - Case2a: Model training and inference at UE side - Case2b: Model training and inference at network side - Case3a: Model training and inference at network side - Case3b: Model training and inference at network side Proposal 2: Modes of operation in which model training and inference happen at different sides are deprioritized for the current AI/ML positioning study. Proposal 3: For AI/ML positioning (Case1, Case2a, and Case2b), consider Level-x and Level-y collaboration between UE and LMF. Proposal 4: For AI/ML positioning (Case3a and Case3b), consider Level-x and Level-y collaboration between gNB/TRP and LMF. |
| [25, NEC] | Proposal 1: Collaboration level x is recommended for UE-based positioning with UE-side model (case 1) for both direct AI/ML positioning and AI/ML assisted positioning. This is because collaboration level x is more likely to support the use of AI models, which have the potential to replace traditional position calculation methods and do not require any changes on the network side. Proposal 2: Collaboration level x and z is recommended for UE-assisted/LMF-based positioning with UE-side model. This is because collaboration level y and collaboration level z enables the LMF to access the information from the AI/ML model on the UE side, which can improve the accuracy of the positioning calculation. Proposal 3: Collaboration level x, y and z all are suitable for case2b, UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning. Proposal 4: Collaboration level x, y and z all are suitable for case 3a, NG-RAN node positioning with gNB-side model, AI/ML assisted positioning. Proposal 5: Collaboration level x, y and z all are suitable for case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. |

## Model transfer and collaboration levels

In RAN1#109-e, some terminologies were agreed as working assumption to be used for RAN1 AI/ML air interface SI discussion. Some relevant to AI/ML model inference and transfer are copied below.

Working Assumption

| Terminology | Description |
| --- | --- |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

In RAN1#109-e, the following were agreed.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreement

Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels.

- Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2.

- Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement.

- Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case

In RAN1#110, it concluded that

Conclusion

Defer the discussion of prioritization of AI/ML positioning based on collaboration level until more progress on collaboration level discussion in agenda 9.2.1.

In RAN1#110b-e, it was further agreed that

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

In RAN1#111, the following agreement was agreed.

Agreement

For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

In RAN1#112, the following agreement was agreed in agenda 9.2.1.

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Several companies discussed further related to AI/ML model transfer and/or collaboration Level-y and Level-z.

[1, Ericsson] proposed that for the use case of positioning accuracy enhancement, model transfer (case z1-z5) is not considered.

It is proposed in [2, Huawei] that the model training/updating and inference are performed all at the same side for Case 1, 2a, 2b, 3a and 3b.

It is observed in [4, vivo] that model transfer over air interface can be achieved with extremely low signaling overhead by combining small-parameter model design and advanced model quantization technologies. [7, vivo] then proposed further study the overhead of model transfer, and support model transfer over air interface for AI/ML based positioning. It also proposed that when AI/ML model is deployed at UE side, network side should transfer the model information to the target UE.

[5, OPPO] proposed that for both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer where AI model training and inference at UE side, or AI model training and inference at NW side. [5, OPPO] also proposed to study model transfer in future release(s).

[6, Spreadtrum] proposed that for both of direct AI/ML positioning and AI/ML assisted positioning, whether AI/ML model can be delivered or not can wait for the progress of AI9.2.1. It is also proposed in [6, Spreadtrum] that that for both of direct AI/ML positioning and AI/ML assisted positioning, suggest to focus on the training and inference located at the same side at present.

It is observed in [8, CATT] that training AI/ML model for positioning at network side is more feasible due to easier data collection and stronger computational resources. [8, CATT] proposed that for case 1, case 2a and case 3a, if UE-side model and gNB-side model is trained at LMF side, LMF can collect a large-scale dataset from numerous UEs/PRUs/gNBs and transfer the trained AI/ML model to UE/gNB side.

[10, Sony] proposed to support AI/ML with model transfer in which the inference model is either in UE or gNB and LMF to create and train the AI/ML model. It also proposed to support AI/ML Positioning with model training at LMF and model inference at the UE side. On AI/ML model indication, [10, Sony] also proposed to define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB.

[12, Xiaomi] proposed that for the AI model delivery to UE, delivering from LMF to UE is considered as the baseline.

[16, CAICT] proposed that for UE-based positioning with UE-side model, AI/ML model delivery from NW should be considered for Case1.

[17, CMCC] proposed that all collaboration levels defined in AI 9.2.1 can be considered in Rel-18 SI for AI/ML-based positioning.

[23, Lenovo] proposed to support model transfer using associated Model IDs and meta-information to help facilitate the transfer to other network entities/UEs. [23, Lenovo] also proposed to consider the some additional aspects with respect to the network-UE collaboration levels y and z including the associated sub-levels. [23, Lenovo] also proposed to further study mechanisms to enable efficient positioning AI/ML model transfer between UE, gNB and LMF.

[24, Qualcomm] proposed that for AI/ML positioning model training, study model training at the side at which the inference for that model is expected to be performed and to deprioritize modes of operation in which model training and inference happen at different sides for the current AI/ML positioning study.

[25, NEC] proposed that collaboration level x is recommended for UE-based positioning with UE-side model (case 1); collaboration level x and z is recommended for UE-assisted/LMF-based positioning with UE-side model (case 2a); collaboration level x, y and z all are suitable for case 2b, 3a and 3b

Moderator’s observation and comment:

Companies’ view on whether to study/consider further on model transfer (i.e., model trained on one side and deployed to the other side for inference) for AI/ML based positioning.

Yes: [4, vivo], [8, CATT], [10, Sony], [12, Xiaomi], [16, CAICT], [17, CMCC], [23, Lenovo], [25, NEC]

No: [1, Ericsson], [5, OPPO]

Wait for progress of AI 9.2.1: [6, Spreadtrum]

Prioritize study on model training and inference at the same side: [2, Huawei], [6, Spreadtrum], [24, Qualcomm]

It has been more than three RAN1 meetings since companies presented their views/preference toward this topic. Compared to previous RAN1 meetings, the situation does not change dramatically. It is still observed that majority of companies support to study/consider further on model transfer (i.e., model trained on one side and deployed/delivered to the other side for inference) for AI/ML based positioning. Given this majority view, moderator does not think worthwhile time wise to discuss prioritization of collaboration levels (i.e., with model transfer or not) for AI/ML based positioning in AI 9.2.4.2 in this meeting for a chance to reach an agreement to deprioritize study on model transfer/delivery for AI/ML based positioning. Rather, moderator believe capture the study outcome on the detailed technical reasons to why and/or why not support model transfer for AI/ML based positioning is essential and necessary for the completion of this SI.

Detailed reasons (from contributions) to deprioritize or against study model transfer in AI/ML based positioning

- not possible to train and compile a binary executable model that is optimized for a given UE’s hardware; a burden to 3GPP network to store, register, maintain, retrieve, and transfer UE models; to avoid the concern of how one side can be responsible for the performance of a model deployed at the other side ([1, Ericsson])

- discussion of the spec impacts (deliver AI/ML model and how to define an AI/ML model representation format (MRF), etc.) is been included in agenda 9.2.1 and/or other WGs ([2, Huawei], [6, Spreadtrum])

- more standardization efforts including interoperability and the corresponding test and advanced UE capability ([5, OPPO])

- the model development is best to be done by the vendor who will implement the device where the AI/ML model inference runs ([24, Qualcomm])

Detailed reasons (from contributions) to support study model transfer in AI/ML based positioning

- model transfer over air interface can be achieved with extremely low signaling overhead by combining small-parameter model design and advanced model quantization technologies ([4, vivo])

- LMF is one of the most essential network entities for positioning which controls the positioning related procedure the configurations. It is straightforward that LMF is responsible for the AI model delivery to UE or TRP with small standardization effort ([12, Xiaomi])

- considering the model training at UE/gNB side requires large amounts of training data, computational resources and large resource overhead, it is preferred to train AI/ML model at network side, e.g. LMF side which also provide good generalization performance ([8, CATT], [10, Sony], [16, CAICT])

- Availability of ground truth label and/or other information at LMF to enable LMF monitoring for UE/gNB-side model inference ([8, CATT])

Moderator formulate the following proposal to capture the aspects for/against model delivery/transfer for AI/ML based positioning. Reading the above reasons from contributions for/against the study on model transfer for AI/ML based positioning, it is moderator’s understanding that all reasons against study on model transfer are actually generic and not specific for AI/ML based positioning. Note that since the same and/or similar reasons are made to the prioritization discussion on model transfer in general framework agenda 9.2.1, moderator put them in bracket for now. On the other hand, there’re several technical reasons for model transfer specific to AI/ML based positioning.

Proposal 2-1

Regarding model delivery/transfer for AI/ML based positioning, the following aspects/issues have been studied and identified

- Availability of ground truth label and/or other information at LMF to enable LMF monitoring for UE/gNB-side model inference

- Feasibility of LMF for the AI model delivery to UE or TRP with small standardization effort

- Feasibility and potential benefits (e.g., good generalization performance) of network side (e.g., LMF) training considering the amount of training data, computational resources and large resource overhead

- [model transfer signaling overhead considering small-parameter model design and advanced model quantization technologies

- concern on the feasibility to train and compile a binary executable model that is optimized for a given UE’s hardware

- concern on the complexity for 3GPP network to store, register, maintain, retrieve, and transfer UE models

- concern on the standardization efforts including interoperability and the corresponding test and advanced UE capability]

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Moderator | To Qualcomm: It’s not clear to me what is the criteria for your statement “the model development is best to be done by the vendor who will implement the device where the AI/ML model inference runs” in [24, Qualcomm]. ‘Best’ in terms of what? I guess you were referring to some comparison when use the word ‘best’. However, I didn’t find such comparison/study in [24, Qualcomm]. Please point to me the relevant paragraph in case I missed. Please elaborate your technical reasoning and/or provide technical arguments instead of pure subjective statement so that I may capture as part of study. |
| Fujitsu | Generally supportive, however we are wondering what the relationship between model transfer/delivery and the first bullet is. |

| NEC | Firstly, we think whether support model transfer/delivery should align with other use case(CSI/beam). It would be inconsistent to support transferring/delivering of AI/ML models for CSI/beam enhancement while not supporting transferring/delivering of AI/ML models for positioning enhancement. Therefore, the decision to support model transfer/delivery should be left to the discussion of AI 9.2.1. Secondly, model transfer/delivery is closely related to data transfer, especially for training data. If model transfer/delivery is not supported, training data should be supported since it is highly inflexible (and high-load for UE) for an entity to be responsible for both training data collection and model training (and model inference subsequently). Therefore, we suggest adding a bullet point to discuss the feasibility of entities being responsible for both training data collection and model training, especially for UE if it is not intend to leave it to 9.2.1. |
| --- | --- |
| Moderator | To Fujitsu: The first bullet is corresponding to an argument against model transfer raised in [1, Ericsson] “to avoid the concern of how one side can be responsible for the performance of a model deployed at the other side”. To NEC: This proposal does not intend to make decision on whether to support model transfer or not. Rather, as I commented, this proposal is to capture what have been studied on model transfer for AI/ML based positioning. Hope this clarifies. |
| NOK | There is a strong dependency on the current discussion on Functionality identification framework and model ID framework in 9.2.1 to start considering a specific discussion on AI/ML positioning about model delivery/transfer. Thus, we should wait for the progress on 9.2.1. |
| Ericsson | It seems that in this proposal moderator summarizes the reasons provided by companies why to support or not to support model delivery/transfer. It is not clear what is accomplished by this proposal. Suggest to just keep it as an informal list of pros and cons for discussion, not something to be endorsed. |
| vivo | Support. We think it’s better to capture explicitly the pros and cons as our study outcome into TR. This gives clear technical background which can also help even if later RAN1 decides whether to support model transfer or not (or whether to include model transfer for AI/ML based positioning in a potential Rel-19 (or later) WID). |
| CATT | We agree with the proposal, maybe as conclusion. As part of the SI outcome, we are open to capture this potential conclusion, or the list of pros and cons, or both. |
| CAICT | Support and we also think list the pros and cons of model transfer/delivery could help further decision. |
| NTT DOCOMO | Agree with Ericsson’s suggestion. |
| Xiaomi | We support this direction. But on the other hand, for the following 3 bullets, they are common issues for all use cases and it is also under discussion in 9.2.1. We could wait for the progress - [model transfer signaling overhead considering small-parameter model design and advanced model quantization technologies - concern on the feasibility to train and compile a binary executable model that is optimized for a given UE’s hardware - concern on the complexity for 3GPP network to store, register, maintain, retrieve, and transfer UE models |
| OPPO | We are ok to capture this proposal as a conclusion in the TR. |
| Hw/HiSi | Agree with Ericsson and NTT DOCOMO. This is a discussion point not a proposal to be endorsed. |
| Qualcomm | To Moderator: Just to better align and clarify our position, we find the model needs to be developed by the same side developing the target. There are many technical reasons that have been already discussed in our other aspect paper and our companion 9.2.1 contribution paper in previous meetings. To summarize, model development requires careful implementation consideration for power consumption, hardware area, latency, and concurrency with other PHY/MAC functionalities and require extensive testing. This needs to be done by the entity developing the target device running the inference. We would like also to mention that the advantages that the Moderator mentioned for having NW train the UE-sided models can still be claimed when the model is developed by the target vendor. For example, the availability of ground truth labels and generalizability are well-applicable to both training cases (i.e., NW-sided training and UE-sided training) and UE sided training is no exception to these advantages. Feasibility and potential benefits (e.g., good generalization performance) of network side (e.g., LMF) training considering the amount of training data, computational resources and large resource overhead Why is LMF better suited to handle computation compared to OTT training servers? At least, it seems there is no strong benefit to support model transfer from the AI/ML positioning use case point of view. |
| # ZTE | # It’s more like a conclusion rather than an agreement. The whole list of pros and cons is common to all use cases. Therefore, it’s better to conclude those aspects in 9.2.1. |
| # Moderator | # To ZTE: # I don’t think aspects related to LMF (first three bullets) are common to all use cases. # To all: # This proposal is proposed to be a conclusion. |

##### Proposal 2-1a as conclusion (closed)

Regarding model delivery/transfer for AI/ML based positioning, the following aspects/issues have been studied and identified

- Availability of ground truth label and/or other information at LMF to enable LMF monitoring for UE/gNB-side model inference

- Feasibility of LMF for the AI model delivery to UE or TRP with small standardization effort

- Feasibility and potential benefits (e.g., good generalization performance) of network side (e.g., LMF) training considering the amount of training data, computational resources and large resource overhead

- [model transfer signaling overhead considering small-parameter model design and advanced model quantization technologies

- concern on the feasibility to train and compile a binary executable model that is optimized for a given UE’s hardware

- concern on the complexity for 3GPP network to store, register, maintain, retrieve, and transfer UE models

- concern on the standardization efforts including interoperability and the corresponding test and advanced UE capability]

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Samsung | This is more like a noted information. |
| NEC | Since the moderator mentioned that this proposal is meant to capture what has been studied on model transfer for AI/ML based positioning, we believe that data transfer should also be considered since it is closely related to model transfer. In other words, if we intend to study model transfer for AI/ML based positioning, we cannot overlook the study of data transfer, especially for training data. As mentioned in our first round input, model transfer/delivery is closely related to data transfer. If model transfer/delivery is not supported, training data should be supported since it can be highly inflexible and taxing for the UE to be responsible for both training data collection and model training (and subsequent model inference). Therefore, we suggest adding a sub-bullet point • Feasibility of transferring the data at least for model training between UE and network side. |
| Ericsson | We don’t see the need to formally endorse such discussion notes. |

| ZTE | We can agree with the first and the third bullet. However, for the remaining bullets, it’s common to all use cases. For example, 9.2.1 has discussed the pros and cons of open-format model and proprietary-format model (not concluded yet). Concerns with model delivery in an open format: Require advanced UE capability for compiling and running the model; Proprietary model information disclosed across vendors; Model performance (e.g., intermediate KPIs, end-to-end performance, inference latency) is not guaranteed; Large specification impacts. Benefits of model delivery in an open format: Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing. Joint model development across vendors. |
| --- | --- |
| CATT | In our understanding, the first three bullets are positioning-specific, which may be feasible to conclude in 9.2.4.2. Others do not provide additional information than in 9.2.1. Suggest the following form: Regarding model delivery/transfer for AI/ML based positioning, the following aspects/issues have been studied and identified in addition to those in general framework agenda (9.2.1): - Availability of ground truth label and/or other information at LMF to enable LMF monitoring for UE/gNB-side model inference - Feasibility of LMF for the AI model delivery to UE or TRP with small standardization effort - Feasibility and potential benefits (e.g., good generalization performance) of network side (e.g., LMF) training considering the amount of training data, computational resources and large resource overhead - [model transfer signaling overhead considering small-parameter model design and advanced model quantization technologies - concern on the feasibility to train and compile a binary executable model that is optimized for a given UE’s hardware - concern on the complexity for 3GPP network to store, register, maintain, retrieve, and transfer UE models - concern on the standardization efforts including interoperability and the corresponding test and advanced UE capability] |
| vivo | We support this conclusion or the revision from CATT. |
| CAICT | We also fine with the update from CATT. |
| LG | Generally fine with the update from CATT but the added wording in the main bullet seems not needed since it already describes ‘the model delivery/transfer for AI/ML based positioning’ |
| Qualcomm | We share Ericsson’s view. |
| HW/HiSi | Don’t see the need to officially endorse this discussion as a conclusion. |
| Nokia/NSB | Agree with E//, QC |
| Apple | Also do not see a need for this |
| Moderator | Status of discussion (based on comments to proposal 2-1 and/or 2-1a on whether to endorse this as a conclusion): OK: Fujitsu, NEC (adding one more bullet on feasibility of data transfer), vivo, CATT, CAICT, Xiaomi, OPPO, ZTE (for the 1st and 3rd bullet only), LG (9) No need: Ericsson, NTT DOCOMO, HW/HiSi, Qualcomm, Samsung, Nokia, Apple (7) Although there’re a couple more companies support to capture this conclusion, due to our limited GTW time, moderator does not think a wise usage of time to debate in GTW whether to capture some conclusion on model transfer study or not in this meeting. Moderator’s suggestion is to close it for this meeting. However, moderator open discussion point 2-1 below to collect companies’ further input. |

Moderator’s comment:

We are approaching the end of this SI (2 more meeting after RAN1#112bis-e), it’s still unknown whether and when any conclusion/agreement on model transfer can be made in general framework 9.2.1. Furthermore, even if there’s some agreement in 9.2.1 on model transfer, we may still need to discuss aspects specific to AI/ML positioning in 9.2.4.2. Moderator would like to get input from companies (especially those saying no need to capture any conclusion/observations on this study) on what we need to do on this aspect for the completion of the SI.

##### Discussion point 2-1

Q1: Do you think it’s necessary or not to capture the study outcome regarding the study of model transfer and/or delivery for AI/ML based positioning based on input from contributions for the completion of SI?

Q2: In the answer to Q1 is yes, when and on what aspects of study outcome regarding the study of model transfer and/or delivery for AI/ML based positioning?

Q3: If the answer to Q1 is no, what do you think we need to study more in agenda 9.2.4.2 regarding the study of model transfer and/or delivery for AI/ML based positioning?

Companies are encouraged to provide their answers with elaborations to the above questions and/or other comments (if any).

| Company Name | Comments/Views |
| --- | --- |

## Online and offline training

In RAN1#110, some terminologies were agreed as working assumption to be used for RAN1 AI/ML air interface SI discussion. Some relevant to AI/ML model training are copied below.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

In RAN1#110b-e, it was concluded that

Conclusion

- Defer the discussion of prioritization of online/offline training for AI/ML based positioning until more progress on online vs. offline training discussion in agenda 9.2.1.

Regarding online/offline training for AI/ML based positioning, [1, Ericsson] proposed that for the use case of positioning accuracy enhancement, prioritize the study of offline AI/ML model training in Release 18. Their stated reason is that online learning primarily refers to reinforcement learning solutions where agents learn in an online manner through action exploration and reward observation. It is Ericsson’s opinion that reinforcement learning problems are notorious for slow convergence times, instability, and sensitivity to the reward function. [1, Ericsson] also proposed that for the positioning use case, online fine-tuning of models is not considered. (Note: [1, Ericsson] stated their view that fine-tuning of models should also be handled offline and deployment procedures are the same regardless of whether the model is fine-tuned (before or after initial deployment) or not. However, moderator did not find any reasons mentioned by [1, Ericsson] as why so).

[5, OPPO] proposed that for both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18 and study online training in the future release(s). The stated issues for online training in [5, OPPO] are how to evaluate/justify the performance/benefit of online training; more spec impact; potential more overhead due to data collection/information sharing; feasibility for UE/gNB to deploy an updated AI/ML models in (nearly) real-time manner.

Moderator’s observations and comment:

Reading from the above agreed/assumed definitions of online vs. offline training, it is moderator’s understanding that the definitions of online/offline training are mainly differed by when the dataset for training is collected and used (i.e., (near) real-time or not). There’s also a note on the definition of online training where companies may have different understanding on whether data collection/training for model updating/fine-tuning can be done via online training or not.

Looking at the arguments against online training raised by [1, Ericsson] and [5, OPPO], first of all, it’s not clear to the moderator what is the actual difference between online and offline training in terms of potential specification impact. Furthermore, their arguments are actually generic (apply to general framework discussion in agenda 9.2.1) but not specific to AI/ML based positioning. Given that the focus of agenda 9.2.4.2 is on the potential specification impact specific for AI/ML based positioning, moderator would encourage companies to focus on the study of pros/cons and potential specification impact of both online and offline training specific for AI/ML based positioning in agenda 9.2.4.2.

To the best knowledge of moderator, there is no progress or agreement regarding prioritization between online vs. offline training in agenda 9.2.1 yet. Considering the conclusion made in RAN1#110b-e, moderator does not think the situation changes compared to RAN1#110b-e and hence suggest to de-prioritize the discussion of prioritization of online/offline training for AI/ML based positioning in agenda 9.2.4.2.

##### Discussion point 2-2

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| NEC | Support to deprioritize the discussion. |
| NOK | Agree with NEC and Moderator. To deprioritize the discussion on online/offline. |
| Ericsson | Fine to deprioritize |
| LG | Fine |
| Samsung | Agree it’s not the unique issue for AI for positioning. However, different cases may have different sensitivity to the size needed for training, inference, or finetuning, that may be need to study in future. |
| CATT | Fine |
| NTT DOCOMO | Support to deprioritize the discussion. |
| OPPO | Agree. |
| ZTE | Fine to deprioritize. |
| Baicells | Fine to deprioritize. |
| Apple | Agree |
| Fraunhofer | Support |
| CAICT | Support |
| HW/HiSi | Ok |

# For GTW

##### Proposal 1-1-1a

Regarding ground truth label generation for AI/ML based positioning, the following options of entity to generate ground truth label are identified (in addition to entities from previous agreement)

- UE generates ground truth label when label quality satisfy the requirement

  - based on non-NR and/or NR RAT-dependent positioning methods

  - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - FFS potential specification impact, e.g., the required label quality, necessary assistance signaling to UE

    - Note: user data privacy needs to be preserved

- Network entity generates ground truth label when label quality satisfy the requirement

  - based on positioning methods

  - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b),  NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - FFS potential specification impact, e.g., the required label quality

##### Proposal 1-3b

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study whether and how to provide information related to the applicable conditions (e.g., applicable configuration/area/scenario/environment)

- FFS whether from UE to the network only or two way information exchange between UE and the network

- FFS potential specification impact (including details if explicit signaling or implicit with no specification impact)

- Note: other type of information is not precluded for further study

##### Proposal 1-4-2b

For assisted AI/ML positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), at least the following types of model inference output are identified as candidates providing performance benefits

- TOA estimation at least for a TRP or a resource

  - FFS potential specification impact including details of report to LMF, e.g., time difference relative to a reference time, soft information report

- LOS/NLOS indicator

  - FFS potential specification impact (if any w.r.t. existing measurement report)

- FFS RSTD as output where estimation is for a pair of TRPs or resources

  - For Case 2a DL positioning

  - Note: soft information report is not precluded

- FFS RSRPP for Case 2a and Case 3a

- Note1: whether the above can be applied to other aspects of AI/ML LCM (e.g., training, updating, monitoring, etc.) can also be discussed

##### Proposal 1-4-1d

For direct AI/ML positioning with LMF-side model (Case 2b and 3b), at least the following type of measurement(s) are identified as candidates providing performance benefits for AI/ML based positioning accuracy enhancement

- Potential new measurement, which contains path timing, power and phase information of the channel response

  - FFS potential specification impact of measurement request and report including overhead reduction

    - Note: take into account existing Rel-16/17 measurement and/or expected Rel-18 measurement

- Potential new measurement, which contains path timing and power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement report (e.g., RSRPP and timing for multi-path) and/or overhead reduction

- Existing measurement and/or existing measurement report (e.g., RSRPP/RSRP/RSTD/AdditionalPath), which contains path timing or power information of the channel response

  - FFS potential specification impact including enhancement to existing measurement report

- Note1: whether such measurement request and report can be applied to other aspects of AI/ML LCM (e.g., monitoring, etc.) and/or for other Cases can also be discussed

- Note2: potential combinations of multiple measurements and/or post processing of the measurement(s) are not precluded

# Conclusion

TBD

# Reference

- R1-2302336	Other Aspects of AI/ML Based Positioning Enhancement	Ericsson

- R1-2302363	Discussion on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

- R1-2302442	Discussion on other aspects for AI positioning enhancement	ZTE

- R1-2302482	Other aspects on AI/ML for positioning accuracy enhancement	vivo

- R1-2302545	On sub use cases and other aspects of AI/ML for positioning accuracy enhancement	OPPO

- R1-2302597	Discussion on other aspects on AIML for positioning accuracy enhancement	Spreadtrum Communications

- R1-2302633	Other aspects on ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

- R1-2302700	Discussion on AI/ML-based positioning enhancement	CATT

- R1-2302739	Other aspects on AI-ML for positioning accuracy enhancement	Baicells

- R1-2302844	Discussions on AI-ML for positioning accuracy enhancement	Sony

- R1-2302909	Discussions on specification impacts for AIML positioning accuracy enhancement	Fujitsu

- R1-2302980	Views on the other aspects of AI/ML-based positioning accuracy enhancement	xiaomi

- R1-2303055	On Enhancement of AI/ML based Positioning	Google

- R1-2303081	Other aspects on AI/ML for positioning accuracy enhancement	LG Electronics

- R1-2303125	Discussion on potential specification impact for Positioning	Samsung

- R1-2303188	Discussions on AI-ML for positioning accuracy enhancement	CAICT

- R1-2303229	Discussion on other aspects on AI/ML for positioning accuracy enhancement	CMCC

- R1-2303341	Other Aspects on AI ML Based Positioning Enhancement	MediaTek Inc.

- R1-2303413	On potential AI/ML solutions for positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-2303440	AI and ML for positioning enhancement	NVIDIA

- R1-2303451	Designs and potential specification impacts of AIML for positioning	InterDigital, Inc.

- R1-2303480	On Other aspects on AI/ML for positioning accuracy enhancement	Apple

- R1-2303529	AI/ML Positioning use cases and associated Impacts	Lenovo

- R1-2303587	Other aspects on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

- R1-2303675	Discussion on AI/ML for positioning accuracy enhancement	NEC

- R1-2303709	Discussion on other aspects on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.