3GPP TSG RAN WG1 #112bis-e                                                             R1-2304117

e-Meeting, April 17th – 26th, 2023

Agenda item:	7.1

Source:	Moderator (Samsung)

Title:	Summary of [112bis-e-AI7.1-09] NR Rel-15/16 maintenance on HARQ-ACK codebook for SPS PDSCH receptions

Document for:	Discussion and Decision

# Introduction

This document summaries email discussion “[112bis-e-AI7.1-09] NR Rel-15/16 maintenance on HARQ-ACK codebook for SPS PDSCH receptions”.

Please consider entering the contact information below for better coordination for this email discussion.

| Company | Contact(s) | Email address(es) |
| --- | --- | --- |
| Samsung (Moderator) | Sa Zhang | sa.zhang@samsung.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| vivo | Na Li | lina5g@vivo.com |
| Nokia/NSB | Klaus Hugl | klaus.hugl@nokia.com |
| LG Electronics | Sukchel Yang | suckchel.yang@lge.com |
| Intel | Debdeep Chatterjee | debdeep.chatterjee@intel.com |

# Background

The following two issues of HARQ-ACK codebook for SPS PDSCH receptions were brought up in R1-2303101 [1].

Issue#1: NPDSCHrepeat  is not defined if neither pdsch-AggregationFactor-r16 is provided in SPS-Config nor pdsch-AggregationFactor is provided in PDSCH-Config for SPS PDSCH.

Issue#2: RRC parameters SPS-Config and PDSCH-Config are not correctly captured in TS 38.213.

The following TP is proposed to address the above two issues.

TP#1

9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH reception on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-Config; otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while *** Unchanged text is omitted ***

The changes are summarized below,

- Clarify that NPDSCHrepeat =1 if neither pdsch-AggregationFactor-r16 is provided in SPS-config nor pdsch-AggregationFactor is provided in PDSCH-Config for SPS PDSCH.

- Align the RRC parameters SPS-Config and PDSCH-Config with 38.331.

# Discussion

## Company input for the first round discussion

Please provide your feedback on the following questions before UTC 16:59, April 18th (Tuesday).

Q1: Do you agree that Issue#1 is valid? If not, please elaborate your reasons.

| Company | View |
| --- | --- |
| CATT | Agree |
| vivo | Agree. In TS 38.331, the following was captured for pdsch-AggregationFactor in PDSCH-config. The CR is aligned with 38.331. pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1. |
| ZTE | Agree. |
| Huawei, HiSilicon | Partially. As pointed out by vivo, it is clear from TR38.331 that the UE applied the value of 1 when pdsch-AggregationFactor is absent. Therefore, the case “when neither pdsch-AggregationFactor-r16 is provided in SPS-Config nor pdsch-AggregationFactor is provided in PDSCH-Config for SPS PDSCH” has been covered by current specification already. We don’t see the need to further clarify this in TS 38.213. pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1. |
| Nokia/NSB | Maybe not an essential correction (as vivo pointed out), but it seems we have covered this case already in 9.1.2 here: Anyhow, if we are going to change the RRC parameter name of Issue #2, then better to handle Issue #1 at the same time. Similar as in Sec. 9.1.2, we think that just having the value there in otherwise to be sufficient. otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1 |
| Ericsson | No. The spec is clear from description in 38.331. We should consider all specifications, if we we observe conflicting information or missing information, use a CR. We don’t see that is the case here. |
| LGE | Similar view with HW/Nokia/Ericsson that the relevant description in 331 spec seems already sufficient. Nevertheless, if it is decided that clarification to 213 spec is also needed, then Nokia’s simplified version in above is preferred. |
| Apple | No, same view as Ericsson (and similar to HW/HiSi, Nokia/NSB, LGE, vivo) that spec is already clear. |
| Intel | Not an essential correction. The UE behavior should be clear from 331 specs as pointed out by others. |
| QC | No. Same view as many other companies that 38.331 already covered this case. |
| MTK | Agree. If to purse a more clear spec, Nokia’s version is preferred. |

Q2: Do you agree with that Issue#2 is valid?  If not, please elaborate your reasons.

| Company | View |
| --- | --- |
| CATT | Agree |
| vivo | Agree |
| Huawei, HiSilicon | Agree |
| Nokia/NSB | Agree |
| Ericsson | Agree. Can be handled by Editor. |
| LGE | Agree. |
| Apple | Hanlde by Editor |
| Intel | Yes, and can be addressed by the Editor. |
| QC | This can be an editorial change handled by editor. |
| MTK | Agree |

Q3: Do you agree with TP#1? If not, please elaborate your reasons.

| Company | View |
| --- | --- |
| CATT | We agree with the TP in principle. A minor comment is that “; otherwise,” should not be in italic. |
| Vivo | Fine. |
| ZTE | Partially agree. The alignment of term of RRC parameters is fine. But “; otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1” seems redundant, as the definition of “pdsch-AggregationFactor” in 38.331 has already declared the handling when the field is absent. But I will not block this TP if majority companies are OK with it. |
| Huawei, HiSilicon | As replied above, we are fine with the parameter alignment but do not see the need to clarify “NPDSCHrepeat =1 if neither pdsch-AggregationFactor-r16 is provided in SPS-config nor pdsch-AggregationFactor is provided in PDSCH-Config for SPS PDSCH.” |
| Nokia/ NSB | Italic of otherwise, the and middle sentence (aligned with 9.1.2) is not needed, i.e. otherwise, if NPDSCHrepeat is not provided, NPDSCHrepeat=1 .. on the redundant, it is also in the other part already (in Sec. 9.1.2 since Rel-15) |
| Ericsson | No. |
| LGE | If necessary, Nokia’s simplified version in above is preferred. |
| Apple | If majority agree with the change, Nokia’s version is preferred |
| Intel | Not necessary. |
| QC | NO. But we are fine to align the parameters name in an editorial CR. |
| MTK | Fine while Nokia’s version is preferred. |

Q4: Any other comments (including comments to the cover pages to the draft CR)?

| Company | View |
| --- | --- |
| Ericsson | Please see our comments to Q1 and Q2. |

## Summary of the first round discussion

In the first round discussion, half of the companies do not think Issue#1 is valid. From moderator’s perspective, it would be good if we can better clarify Issue#1, however, considering there is no misalignment, it would also be fine to keep the spec as it is.

For Issue#2, all the companies agree that Issue 2 is valid. A few companies prefer an alignment CR, however, as guided by Mr. Chair, we do not have alignment CR for Rel-15/16 issues from this meeting. An individual CR would be proposed to resolve Issue#2.

## Company input for the second round discussion

Please provide your feedback on the following proposal before UTC 23:59, April 19th (Wednesday).

Proposal 1: Adopt the following TP#2 for Rel-16 TS 38.213.

Reason for change: RRC parameters SPS-Config and PDSCH-Config are not correctly captured in TS 38.213 clause 9.1.2.

Summary of change: Correct the RRC parameters SPS-Config and PDSCH-Config in TS 38.213.

Consequences if not approved: Unaligned parameters from different specifications.

TP#2

9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH reception on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-Config, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while *** Unchanged text is omitted ***

| Company | View |
| --- | --- |
| QC | While, if no editor’s alignment CR is allowed in this meeting, we don’t want to adopt this CR as individual CR. At this stage, individual CR for Rel-16 is for critical issue, the TP is hard to be justified as a fix for critical issue. |
| Huawei, HiSilicon | When we focus on correcting the RRC parameter in this round, I would suggest this change could be merged into issue 1-1 in 112bis-e-R17-MBS-01 since wherein the same RRC parameter will be necessarily corrected, so that the issue could be resolved with the concern of being non critical for individual CR alleviated. |
| LGE | Similar view with other companies. Merging the TP into another (alignment) CR is reasonable rather than preparing an individual CR only on the TP itself. |
| Moderator | Based on the input so far, moderator’s suggestion is to include TP#2 in alignment CR for Rel-17. Companies are encouraged to check Proposal 2. |
| Nokia, NSB | Agree on R16 with comments by QC, HW/HiSi & LG – but seems this seems to be moved to Rel-17 already in the meanwhile (with respect to Proposal 2) |
| vivo | Agree with Nokia that this is be moved to Rel-17 already in the meanwhile (with respect to Proposal 2) |

Proposal 2: Adopt TP#2 for alignment CR in Rel-17 TS 38.213.

| MTK | Agree |
| --- | --- |
| Nokia, NSB | Support |
| Ericsson | Agree |
| LGE | OK with the proposal. |
| Samsung | Support |
| vivo | The TP can be handled together with other issues in Rel-17 MBS session. |
| Moderator | @vivo The following is suggested by FL of MBS HARQ-ACK, if you do not have technical concern, can you be flexible to accept the proposal to make progress? Conclusion from preparation phase: Discuss for alignment CR including addressing the preparation phase comments to improve the formulation of the incoming draft CR, but start the discussion only after a conclusion on [112bis-e-AI7.1-09]. Put on hold as suggested.. |
| vivo | @ Moderator thanks for your response. We think FL in MBS is waiting the discussion result here, it could be an endorsed CR, or conclusion, or consensus, or even no conclusion. Our original intention is a CR is anyway needed in MBS session in the same paragraph for the issue of pdsch-AggregationFactor for SPS, separate CR may be not needed. For progress, if no company object to agree this RRC parameter correction with an alignment CR first. We are fine |

# Conclusion

The following is agreed.

For the TS 38.213 editor:

The following editorial changes are provided to improve the clarity of the RAN1 specifications. Please include them in the Rel-17 alignment CR.

9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in spsSPS-Config or, if pdsch-AggregationFactor-r16 is not included in spsSPS -Config, by pdsch-AggregationFactor in PDSCH-Configpdsch-config, and *** Unchanged text is omitted ***

# Reference

[1] R1-2303101	Correction on HARQ-ACK codebook for SPS PDSCH receptions	Samsung