3GPP TSG-RAN WG1 #112bis-e	R1-2304201

E-meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of multi-carrier enhancements. |
| --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of multi-cell PDSCH / PUSCH scheduling using DCI format 0_3 & 1_3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of multi-cell PUSCH scheduling using DCI format 0_3 in clauses Introduction of multi-cell PDSCH scheduling using DCI format 1_3 in clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-cell PDSCH / PUSCH scheduling using DCI format 0_3 & 1_3 not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.1.2.1, 5.1.2.1.1, 5.1.2.2, 5.1.2.2.1, 5.1.2.2.2, 5.1.2.3, 5.1.3, 5.1.3.1, 5.1.3.2, 5.1.4.1, 5.1.4.2. 5.1.5, 5.1.6.2, 5.1.6.3, 5.2.1, 5.2.1.4, 5.2.3, 5.3.1 6.1, 6.1.1, 6.1.1.1, 6.1.1.2, 6.1.2.1, 6.1.2.1.1, 6.1.2.2, 6.1.2.2.1, 6.1.2.2.2, 6.1.2.3.1, 6.1.2.3.2, 6.1.4, 6.1.4.1, 6.1.4.2, 6.1.7, 6.2.1, 6.2.2, 6.2.3, 6.2.3.1, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

# 5	Physical downlink shared channel related procedures

## 5.1	UE procedure for receiving the physical downlink shared channel

For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700, and when no configuration is provided the UE may assume a default number of 8 processes.

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1 or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided. When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA’, the PDSCH includes two PDSCH transmission occasions. For each PDSCH, if either PDSCH occasion overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, the PDSCH is not received and HARQ process ID is not increment for the PDSCH. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, N=24 for =3, N=96 for m=5, and N=192 for m=6.

<omitted text>

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field 'Antenna Port(s)' and it is not configured with higher layer parameter sfnSchemePdsch, the UE may expect to receive a single PDSCH where the association between the DM-RS ports and the TCI states are as defined in Clause 5.1.6.2.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1.

When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and

-	if the UE reports its capability of sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or

-	otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and

the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5.

When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of sfn-SchemeA-PDCCH-only, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeA' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeB' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePdsch and/or sfnSchemePdcch, the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all DL BWP within a CC other than initial BWP, and the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all CCs in a same frequency band if the UE is configured with CA.

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below.

‒	Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot

‒	Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH.

‒	Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q.

‒	Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE

<omitted text>

### 5.1.2	Resource allocation

#### 5.1.2.1	Resource allocation in time domain

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m for the scheduled PDSCH on the serving cell provides a row index m + 1 to a resource allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

<omitted text>

When receiving PDSCH scheduled by DCI format 1_1, 1_2 or 1_3 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the same symbol allocation is applied across the pdsch-AggregationFactor, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the UE is not expected to be configured with the time duration for the reception of pdsch-AggregationFactor repetitions, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, larger than the time duration derived by the periodicity P obtained from the corresponding sps-Config. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

<omitted text>

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs  have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH.

<omitted text>

When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1, 1_3, if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than  K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

<omitted text>

##### 5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH

Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForMultiPDSCH-r17 or pdsch-TimeDomainAllocationListDCI-1-2 is applied. For operation with shared spectrum channel access in frequency range 1, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7.

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 1_3, 4_0, 4_1 and 4_2

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | pdsch-ConfigMCCH / pdsch-ConfigMTCH includes pdsch-TimeDomainAllocationList Or pdsch-ConfigMulticast includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH-r17 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | - | - | Default A for normal CP |
|  |  | 2 | - | - | - | - | Default B |
|  |  | 3 | - | - | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| RA-RNTI, MSGB-RNTI, TC-RNTI | Type1 common | 1,2,3 | No | - | - | - | Default A |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMTCH, if configured, otherwise TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI for multicast, G-CS-RNTI | Type 3 common search space for multicast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMulticast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs and G-CS-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.2.2	Resource allocation in frequency domain

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, 4_0 or 4_1 then downlink resource allocation type 1 is used.

If the scheduling DCI is configured to indicate the downlink resource allocation type as part of the 'Frequency domain resource assignment' field by setting a higher layer parameter resourceAllocation in PDSCH-Config to 'dynamicSwitch', for DCI format 1_1 or setting a higher layer parameter resourceAllocationDCI-1-2 in PDSCH-Config to 'dynamicSwitch' for DCI format 1_2 or setting a higher layer parameter resourceAllocationDCI-1-3 in PDSCH-Config to 'dynamicSwitch' for DCI format 1_3 or setting a higher layer parameter resourceAllocation in pdsch-ConfigMulticast to 'dynamicSwitch' for DCI format 4_2, the UE shall use downlink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the downlink frequency resource allocation type as defined by the higher layer parameter resourceAllocation in PDSCH-Config for DCI format 1_1 or by the higher layer parameter resourceAllocationDCI-1-2 for DCI format 1_2 or by the higher layer parameter resourceAllocationDCI-1-3 for DCI format 1_3 or by the higher layer parameter resourceAllocation in pdsch-ConfigMulticast for DCI format 4_2.

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the downlink bandwidth part and then the resource allocation within the bandwidth part.

For a PDSCH scheduled with a DCI format 1_0 in any type of PDCCH common search space, regardless of which bandwidth part is the active bandwidth part, RB numbering starts from the lowest RB of the CORESET in which the DCI was received; otherwise RB numbering starts from the lowest RB in the determined downlink bandwidth part. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the downlink RB set of a PDSCH when scheduled by DCI format 1_0, the CORESET with lower ID among two CORESETs associated with two PDCCH candidates is used.

##### 5.1.2.2.1	Downlink resource allocation type 0

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured by PDSCH-Config for DCI format 1_1 or 1_2 or by higher layer parameter rbg-SizeDCI-1-3 configured by PDSCH-Config for DCI format 1_3 and the size of the bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 | Configuration 3 |
| --- | --- | --- | --- |
| 1 – 36 | 2 | 4 | 8 |
| 37 – 72 | 4 | 8 | 16 |
| 73 – 144 | 8 | 16 | 32 |
| 145 – 275 | 16 | 16 | 32 |

The total number of RBGs (NRBG) for a downlink bandwidth part i of size NBWP,isize PRBs is given by NRBG=NBWP,isize+NBWP,istartmodP/P, where

-	the size of the first RBG is RBG0size=P-NBWP,istartmodP,

-	the size of last RBG is RBGlastsize=NBWP,istart+NBWP,isizemodP if NBWP,istart+NBWP,isizemodP>0 and P otherwise,

-	the size of all other RBGs is P.

In downlink resource allocation of type 0 scheduled using a DCI with CRC scrambled by G-RNTI for multicast or G-CS-RNTI, the resource block assignment information bitmap is calculated based on the description above with the following changes: the parameter NBWP,istart is the starting PRB of the CFR, NBWP,isize is the size of the common frequency resource (CFR) and the value of the higher layer parameter rbg-Size is configured by pdsch-ConfigMulticast.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and starting at the lowest frequency of the bandwidth part. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

##### 5.1.2.2.2	Downlink resource allocation type 1

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in any common search space in which case the size of CORESET 0 shall be used if CORESET 0 is configured for the cell and the size of initial DL bandwidth part shall be used if CORESET 0 is not configured for the cell.

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

When the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP with size of , a downlink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks , where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell;

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

The resource indication value is defined by:

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

When the scheduling grant is received with DCI format 1_2 or 1_3, a downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0, 1, …, NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=1, …, NRBG, where the resource block groups are defined as in 5.1.2.2.1 with P defined by resourceAllocationType1GranularityDCI-1-2 for DCI format 1_2 and resourceAllocationType1GranularityDCI-1-3 for DCI format 1_3 if the UE is configured with higher layer parameter resourceAllocationType1GranularityDCI-1-2 or resourceAllocationType1GranularityDCI-1-3, and P=1 otherwise. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

<omitted text>

#### 5.1.2.3	Physical resource block (PRB) bundling

The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of prb-BundlingTypeDCI-1-2 instead of prb-BundlingType as well as vrb-ToPRB-InterleaverDCI-1-2 instead of vrb-ToPRB-Interleaver. The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3. The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of prb-BundlingType given by pdsch-ConfigMulticast as well as vrb-ToPRB-Interleaver given by pdsch-ConfigMulticast.

<omitted text>

### 5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Clause 5.1.3.1, and

-	read 'redundancy version' field (rv) in the DCI to determine the redundancy version.

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Clause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH.

When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field.

The UE is not expected to handle any transport blocks (TBs) in a 14 consecutive-symbol duration for normal CP (or 12 for extended CP) ending at the last symbol of the latest PDSCH transmission within an active BWP on a serving cell whenever

2max⁡(0,μ-μ').i∈SCi'Lixi.Fi>X4.1RLBRM.TBSLBRM

where, for the serving cell,

-	S is the set of TBs belonging to PDSCH(s) that are partially or fully contained in the consecutive-symbol duration

-	for the ith TB

-	Ci' is the number of scheduled code blocks for as defined in [5, 38.212].

-	Li is the number of OFDM symbols assigned to the PDSCH

-	xi is the number of OFDM symbols of the PDSCH contained in the consecutive-symbol duration

-	Fi=maxj=0,...,J-1(min(k0,ij+Eij, Ncb,i)) based on the values defined in Clause 5.4.2.1 [5, TS 38.212]

-	k0,ij is the starting location of RV for the jth transmission

-	Ei j=min(Er) of the scheduled code blocks for the jth transmission

-	Ncb,i is the circular buffer length

-	J-1 is the current (re)transmission for the ith TB

-	μ' corresponds to the subcarrier spacing of the BWP (across all configured BWPs of a carrier) that has the largest configured number of PRBs

-	in case there is more than one BWP corresponding to the largest configured number of PRBs, µ' follows the BWP with the largest subcarrier spacing.

-	μ corresponds to the subcarrier spacing of the active BWP

-	RLBRM = 2/3 as defined in Clause 5.4.2.1 [5, TS 38.212]

-	TBSLBRM as defined based on the parameters for unicast in Clause 5.4.2.1 [5, TS 38.212]

-	X as defined for downlink max MIMO layer for unicast in Clause 5.4.2.1 [5, TS 38.212].

If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

Within a cell group, a UE is not required to handle PDSCH(s) transmissions including unicast and/or multicast/broadcast in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate

where,

-	J is the number of configured serving cells belonging to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot sj. If there are two PDSCH transmission occasions of the same TB (in time domain or in frequency domain) in the slot sj, each transmission occasion is counted separately.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PDSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PDSCH-ServingCellConfig is configured for the serving cell and set to 'enable', or if at least one IMCS > W for a PDSCH for unicast or multicast, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS table 5.1.3.1-2, and W = 26 for MCS table 5.1.3.1-4, or for a j-th serving cell where UE supports FDM-ed unicast and MBS PDSCH, the UE is not required to handle PDSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PDSCH(s). For a PDSCH that consists of two PDSCH transmission occasions in time domain in one slot, L is the number of symbols of one transmission occasion. For FDMed unicast and MBS PDSCHs in one slot, L is the total number of symbols of the unicast and MBS PDSCHs with fully or partially-overlapped in time domain.

-	M is the number of TB(s) in the PDSCH(s)

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PDSCH(s)

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

#### 5.1.3.1	Modulation order and target code rate determination

For the PDSCH scheduled by a PDCCH with DCI format 1_0, format 1_1, format 1_2, format 1_3, format 4_0, format 4_1 or format 4_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, MSGB-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or P-RNTI, or for the PDSCH scheduled without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration SPS-Config,

if the higher layer parameter mcs-Table-r17 given by PDSCH-Config is set to 'qam1024', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 or 1_3 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif mcs-TableDCI-1-2-r17 given by PDSCH-Config is set to 'qam1024', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 or 1_3 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMulticast is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI for multicast

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMCCH and pdsch-ConfigMTCH is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMulticast is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI for multicast

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with a DCI format other than DCI format 1_2 in a UE-specific search space with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with MCS-C-RNTI, and the PDSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-Table-r17 given by PDSCH-Config is set to 'qam1024',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_1 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-TableDCI-1-2-r17 given by PDSCH-Config is set to 'qam1024',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_2 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_2 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-Config, and the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_1 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-Config set to 'qam64LowSE'

-	if the PDSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if the PDSCH is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-Config or mcs-Table of pdsch-ConfigMulticast in the same CFR-ConfigMulticast set to 'qam64LowSE'

-	if the GC-PDSCH is scheduled by a GC-PDCCH with CRC scrambled by G-CS-RNTI or

-	if the GC-PDSCH is scheduled without corresponding GC-PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

end

The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

For a UE configured with the higher layer parameter repetitionScheme set to 'fdmSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the determined modulation order of PDSCH transmission occasion associated with the first TCI state is applied to the PDSCH transmission occasion associated with the second TCI state.

<omitted text>

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in PDSCH-config indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 or 1_3 if IMCS = 26 and if rvid = 1 for the corresponding transport block. In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in pdsch-ConfigMulticast indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 4_2 if IMCS = 26 and if rvid = 1 for the corresponding transport block. When the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH-r17, either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI format 1_1 if IMCS = 26 and if rvid = 2 for the corresponding transport block of all scheduled PDSCHs. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1 or 4_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, else if Table 5.1.3.1-4 is used and 0≤IMCS≤26 or a table other than Table 5.1.3.1-2 and Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1 or 1_3, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1, 1_2 or 1_3 or as described for format 1_0 in Clause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI, MSGB-RNTI or P-RNTI, NohPRB is assumed to be 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by G-RNTI for multicast or G-CS-RNTI or PDSCH without PDCCH is activated by PDCCH with a CRC scrambled by G-CS-RNTI, NohPRB NohPRBis the overhead configured by higher layer parameter xOverhead-Multicast in pdsch-ConfigMulticast. If the xOverhead-Multicast in pdsch-ConfigMulticast is not configured, the NohPRB is set to 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

<omitted text>

### 5.1.4	PDSCH resource mapping

<omitted text>

#### 5.1.4.1	PDSCH resource mapping with RB symbol level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying only the parameters of rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2 instead of rateMatchPatternGroup1 and rateMatchPatternGroup2. The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3. The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_0, by applying only the parameters of rateMatchPatternToAddModList configured in pdsch-ConfigMCCH or pdsch-ConfigMTCH.

<omitted text>

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList. The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in pdsch-ConfigMulticast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

<omitted text>

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

The UE can be configured with a list of up to 128 TCI-State configurations, within the higher layer parameter dl-OrJointTCI-StateList in PDSCH-Config for providing a reference signal for the quasi co-location for DM-RS of PDSCH and DM-RS of PDCCH in a BWP/CC, for CSI-RS, and to provide a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a BWP/CC, and SRS.

If the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or TCI-UL-State configurations from a reference BWP of a reference CC. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJointTCI-StateList or TCI-UL-State in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJointTCI-StateList or TCI-UL-State in any CC within the same band in the CC list.

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State and/or TCI-UL-State to only one TCI codepoint, the UE shall apply the indicated TCI-State and/or TCI-UL-State to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies.

When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJointTCI-StateList with activated TCI-State or TCI-UL-State receives DCI format 1_1/1_2/1_3 providing indicated TCI-State and/or TCI-UL-State for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2/ is without DL assignment, the UE can assume the following:

-	CS-RNTI is used to scramble the CRC for the DCI

-	The values of the following DCI fields are set as follows:

-	RV = all '1's

-	MCS = all '1's

-	NDI = 0

-	Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

<omitted text>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling a PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 or format 1_3 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

When a UE is configured with both sfnSchemePdcch and sfnSchemePdsch scheduled by DCI format 1_0 or by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable:

-	if the UE supports sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE should be activated with the CORESET with two TCI states.

-	else if the UE does not support sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present.

For PDSCH scheduled by DCI format 1_0, 1_1, 1_2, when a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' and sfnSchemePdsch is not configured, and there is no TCI codepoint with two TCI states in the activation command, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the first TCI state or QCL assumption which is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH-r17, the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is not provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets,

-	the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same value of coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)

-	If a UE is not configured with sfnSchemePdsch, and the UE is configured with sfnSchemePdcch set to 'sfnSchemeA' and there is no TCI codepoint witih two TCI states in the activation command and the CORESET with the lowest ID in the latest slot is indicated with two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the first TCI state of two TCI states indicated for the CORESET. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET with single active TCI state. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH is configured with qcl-Type set to 'typeD', the UE shall obtain the other QCL assumptions from the indicated TCI state(s) for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State,

-	if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception.

-	if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

If the PDCCH carrying the scheduling DCI is received on one component carrier, and a PDSCH scheduled by that DCI is on another component carrier:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	When the UE is configured with enableDefaultBeamForCCS, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, or if the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

A UE that has indicated a capability beamCorrespondenceWithoutUL-BeamSweeping set to 'supported', as described in [13, TS 38.306], can determine a spatial domain filter to be used while performing the applicable channel access procedures described in [16, TS 37.213] prior to a UL transmission on the channel as follows:

-	if UE is indicated with an SRI corresponding to the UL transmission, the UE may use a spatial domain filter that is same as the spatial domain transmission filter associated with the indicated SRI,

-	if UE is configured with SRS-spatialRelationInfo for the UL transmission, the UE may use a spatial domain filter that is same as the spatial domain filter associated with referenceSignal in the corresponding SRS-spatialRelationInfo,

-	if UE is configured with TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE may use a spatial domain transmit filter that is same as the spatial domain receive filter the UE may use to receive the DL reference signal associated with the indicated TCI state.

When the PDCCH reception includes two PDCCH from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the time offset between the reception of the DL DCI and the corresponding PDSCH, the PDCCH candidate that ends later in time is used. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the configuration of tci-PresentInDCI or tci-PresentDCI-1-2, the UE expects the same configuration in the first and second CORESETs associated with the two PDCCH candidates; and if the PDSCH is scheduled by a DCI format not having the TCI field present and if the scheduling offset is equal to or larger than timeDurationForQCL, if applicable, PDSCH QCL assumption is based on the CORESET with lower ID among the first and second CORESETs associated with the two PDCCH candidates.

<omitted text>

### 5.1.6	UE procedure for receiving reference signals

<omitted text>

#### 5.1.6.2	DM-RS reception procedure

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB. The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3.

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in pdsch-ConfigMulticast instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in PDSCH-Config.

When receiving PDSCH scheduled by DCI format 1_0, 4_0, or 4_1, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A and type B, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

When receiving PDSCH scheduled by DCI format 1_1 or 1_3 by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or DCI format 4_2 by PDCCH with CRC scrambled by G-RNTI for multicast or G-CS-RNTI,

-	the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Clause 7.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength given by DMRS-DownlinkConfig..

-	if maxLength is set to 'len1', single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0', 'pos1', 'pos2' or 'pos3'.

-	if maxLength is set to 'len2', both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0' or 'pos1'.

-	and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Clause 7.4.1.1.2 of [4, TS 38.211].

For the UE-specific reference signals generation as defined in Clause 7.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PDSCH mapping Type A and Type B.

A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 1_1 as described in Clause 7.3.1.2 of [5, TS 38.212].

For DM-RS configuration type 1,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

For DM-RS configuration type 2,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2  or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or a UE receiving PDSCH scheduled by DCI format 1_0, 1_1 or 1_3 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH:

-	any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and

-	PT-RS is transmitted to the UE.

<omitted text>

#### 5.1.6.3	PT-RS reception procedure

The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and to a UE receiving PDSCH scheduled by DCI format 1_0, 1_1 or 1_3 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 4_1 or 4_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB in pdsch-ConfigMulticast.

<omitted text>

#### 5.1.6.5	PRS reception procedure

<omitted text>

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the priority between DL PRS and SSB is defined in [11, TS 38.133] and the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability, except for SSB:

-	with value 'st1' where the DL PRS is higher priority than all the DL signals and channels, or

-	with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1, 1_2 or 1_3 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels, or

-	with value 'st3' where the DL PRS is lower priority than all the DL signals and channels.

Inside one DL-PPW-PreConfig the UE is only expected to measure a single DL PRS positioning frequency layer.

<omitted text>

## 5.2	UE procedure for reporting channel state information (CSI)

### 5.2.1	Channel state information framework

The procedures on aperiodic CSI reporting described in this clause assume that the CSI reporting is triggered by DCI format 0_1, but they equally apply to CSI reporting triggered by DCI format 0_2, by applying the higher layer parameter reportTriggerSizeDCI-0-2 instead of reportTriggerSize. The procedures on aperiodic CSI reporting described in this clause assume that the CSI reporting is triggered by DCI format 0_1, but they equally apply to CSI reporting triggered by DCI format 0_3.

The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indicator (LI), rank indicator (RI), L1-RSRP, L1-SINR or CapabilityIndex.

For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig.

<omitted text>

#### 5.2.1.4	Reporting configurations

<omitted text>

For a semi-persistent or aperiodic CSI report on PUSCH, the allowed slot offsets are configured by the following higher layer parameters:

-	if triggered/activated by DCI format 0_2 and the higher layer parameter reportSlotOffsetListDCI-0-2 is configured, the allowed slot offsets are configured by reportSlotOffsetListDCI-0-2, and

-	if triggered/activated by DCI format 0_1 or 0_3 and the higher layer parameter reportSlotOffsetListDCI-0-1 is configured, the allowed slot offsets are configured by reportSlotOffsetListDCI-0-1, and

-	otherwise, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetList.

The offset is selected in the activating/triggering DCI.

For CSI reporting, a UE can be configured via higher layer signaling with one out of two possible subband sizes, where a subband is defined as  contiguous PRBs and depends on the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2.

Table 5.2.1.4-2: Configurable subband sizes

| Bandwidth part (PRBs) | Subband size (PRBs) |
| --- | --- |
| 24 – 72 | 4, 8 |
| 73 – 144 | 8, 16 |
| 145 – 275 | 16, 32 |

The reportFreqConfiguration contained in a CSI-ReportConfig indicates the frequency granularity of the CSI Report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates:

-	the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported.

-	A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource.

-	If a CSI-IM resource is linked to the CSI Report Setting, a UE is not expected to be configured with csi-ReportingBand which contains a subband where not all PRBs in the subband have the CSI-IM REs present.

-	wideband CQI or subband CQI reporting, as configured by the higher layer parameter cqi-FormatIndicator. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band.

-	wideband PMI or subband PMI reporting as configured by the higher layer parameter pmi-FormatIndicator. When wideband PMI reporting is configured, a wideband PMI is reported for the entire CSI reporting band. When subband PMI reporting is configured, except with 2 antenna ports, a single wideband indication (i1 in Clause 5.2.2.2) is reported for the entire CSI reporting band and one subband indication (i2 in clause 5.2.2.2) is reported for each subband in the CSI reporting band. When subband PMIs are configured with 2 antenna ports, a PMI is reported for each subband in the CSI reporting band.

-	a UE is not expected to be configured with pmi-FormatIndicator if codebookType is set to 'typeII-r16' or 'typeII-PortSelection-r16' or 'typeII-PortSelection-r17'.

A CSI Reporting Setting is said to have a wideband frequency-granularity if

-	reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or

-	reportQuantity is set to 'cri-RI-PMI-CQI', codebookType is set to 'typeII-PortSelection-r17' with M=1 and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RI-i1' or

-	reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' or 'cri-RSRP-Index' or 'ssb-Index-RSRP-Index' or 'cri-SINR-Index', or 'ssb-Index-SINR-Index'

otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity.

A CSI Reporting Setting with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, can be configured with wideband frequency-granularity only if csi-ReportMode is set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 is set to X=0, as described in clause 5.2.1.4.2.

If the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency-granularity, and, if applicable, the higher layer parameter codebookType is set to 'typeI-SinglePanel'.

The first subband size is given by  and the last subband size given by  if  and if

If a UE is configured with semi-persistent CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic or semi-persistent. If a UE is configured with aperiodic CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic, semi-persistent or aperiodic.

A UE configured with DCI format 0_1, 0_2 or 0_3 does not expect to be triggered with multiple CSI reports with the same CSI-ReportConfigId.

<omitted text>

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II, Enhanced Type II and Further Enhanced Type II Port Selection CSI.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities, Enhanced Type II and Further Enhanced Type II Port Selection CSI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

<omitted text>

## 5.3	UE PDSCH processing procedure time

<omitted text>

### 5.3.1	Application delay of the minimum scheduling offset restriction

When the UE is scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with a 'Minimum applicable scheduling offset indicator' field in slot n, it shall determine the K0min and K2min values, if configured respectively, to be applied, while the previously applied K0min and/or K2min values are applied until the new values take effect. If the DCI in slot n also indicates an active DL (UL) BWP change for a serving cell, the indicated K0min (K2min) value in the new active DL (UL) BWP, if configured, is applied from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. Otherwise, change of applied minimum scheduling offset restriction indication carried by DCI in slot n, shall be applied in slot n+X of the scheduling cell. The UE does not expect to be scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating another change to K0min or K2min for the same active BWP of the scheduled cell before slot n+X of the scheduling cell.

When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating a change to the applied K0min or K2min is contained within the first three symbols of slot n, the value of application delay X is determined by, X=maxK0minOld∙2μPDCCH2μPDSCH,Zμ where  K0minOld is the currently applied K0min value of the active DL BWP in the scheduled cell and is zero, if minimumSchedulingOffsetK0 is not configured for the active DL BWP in the scheduled cell, Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1, and µPDCCH and µPDSCH are the sub-carrier spacing configurations for PDCCH of the active DL BWP in the scheduling cell and PDSCH of the active DL BWP in the scheduled cell, respectively, in slot n. After indication of a change to the applied K0min or K2min of the scheduled cell in slot n of the scheduling cell, if there is an active DL BWP change in the scheduling cell before slot n+X, the new K0min and/or K2min values are applied from the first slot no earlier than the start of slot n+X based on the sub-carrier spacing configuration of the active DL BWP in the scheduling cell in slot n.

When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, value of Zµ from Table 5.3.1-1 is incremented by one before determining the application delay X. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, the PDCCH candidate that ends later in time is used.

Table 5.3.1-1: Definition of Zµ

| µ | Zµ |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 2 |
| 5 | 8 |
| 6 | 16 |

<omitted text>

# 6	Physical uplink shared channel related procedure

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS.

For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant. When the UE is configured dl-OrJointTCI-StateList or TCI-UL-State, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in clause 6.1.4.1 and transformPrecoder described in clause 6.1.3.

For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1, 0_2 or 0_3 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. Upon detection of a DCI format 0_3 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_3, the UE ignores all fields for the scheduled cell with the smallest serving cell index in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH on the serving cell with the smallest serving cell index as indicated by this DCI format 0_3. When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPUSCH-r17 applied if nrofHARQ-ProcessesForPUSCH-r17 is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1, 0_2 or 0_3 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI.

<omitted text>

### 6.1.1	Transmission schemes

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission. The UE is configured with codebook based transmission when the higher layer parameter txConfig in pusch-Config is set to 'codebook', the UE is configured non-codebook based transmission when the higher layer parameter txConfig is set to 'nonCodebook'. If the higher layer parameter txConfig is not configured, the UE is not expected to be scheduled by DCI format 0_1, 0_2 or 0_3. If PUSCH is scheduled by DCI format 0_0, the PUSCH transmission is based on a single antenna port. Except if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE shall not expect PUSCH scheduled by DCI format 0_0 in a BWP without configured PUCCH resource with PUCCH-SpatialRelationInfo in frequency range 2 in RRC connected mode.

#### 6.1.1.1	Codebook based UL transmission

For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder(s) based on SRI(s), TPMI(s) and the transmission rank, where the SRI(s), TPMI(s) and the transmission rank are given by DCI fields of one or two SRS resource indicators and one or two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2  or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 according to clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of one SRS resource indicator and one Precoding information and number of layers in clause 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'.

When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', SRI and TPMI are given by the DCI fields of one SRS resource indicator and one Precoding information and number of layers in clause 7.3.1.1.2, 7.3.1.1.3 and 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_1. 0_2 and 0_3 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. When two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook', SRI and TPMI are given by the DCI fields of one SRS resource indicator and one Precoding information and number of layers in clause 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_3 and the UE applies the indicated SRI and TPMI to one or more PUSCH repetitions according to the first SRS resource set. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', one or two SRI(s), and one or two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2.  The UE applies the indicated SRI(s) and TPMI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. Each TPMI, based on indicated codepoint of SRS Resource Set indicator, is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set, or if a single SRS resource is configured for the applicable SRS resource set TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two SRIs are indicated, the UE shall expect the nrofSRS-Ports for the two indicated SRS resources to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the most recent transmission of SRS resource identified by the SRI, the PDCCH candidate that starts earlier in time is used.

For codebook based transmission, the UE determines its codebook subsets based on TPMI(s) and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 or 0_3 and codebookSubsetDCI-0-2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 or 0_3 and maxRankDCI-0-2 for PUSCH scheduled with DCI format 0_2.

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-2 with 'fullyAndPartialAndNonCoherent'.

A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-2 with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'.

A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two.

For codebook based transmission, only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubsetDCI-0-2 set to 'fullAndPartialAndNonCoherent' simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1, 0_2 or 0_3 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'.

#### 6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the SRI(s) is given by one or two SRS resource indicator(s) in DCI according to clause 7.3.1.1.2, and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by one SRS resource indicator in DCI according to clause 7.3.1.1.4 of [5, 38.212] for DCI format 0_3, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3.. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 and the UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. When two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', SRI is given by the DCI field of one SRS resource indicator in clause 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_3 and the UE applies the indicated SRI to one or more PUSCH repetitions according to the first SRS resource set. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 4. Each of the indicated SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the most recent transmission of SRS resource(s) identified by the SRI, the PDCCH candidate that starts earlier in time is used.

For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for each of the SRS resource set(s) with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured.

-	If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), as well as DCI format 0_3 and 1_3, where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1, 0_3, 1_1 and 1_3 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModListDCI-0-2. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42∙2max⁡(0,μ-3) OFDM symbols, where the SCS configuration μ is the smallest SCS configuration between the NZP-CSI-RS resource and the SRS transmission.

-	If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. If UE is configured with minimumSchedulingOffsetK0 in the active DL BWP and the currently applicable minimum scheduling offset restriction K0,min is larger than 0, the UE does not expected to receive the scheduling DCI with the SRS request field value other than '00'. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'.

-	If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet.

The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) and their corresponding PUSCH layers {0 … ν-1} given by DCI format 0_1, 0_2 or 0_3 or by configuredGrantConfig according to clause 6.1.2.3 in increasing order.

The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as .

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS 38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-ResourceSet for SRS resource set.

For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 or 0_2 when at least one SRS resource is configured in SRS-ResourceSet with usage set to 'nonCodebook'.

### 6.1.2	Resource allocation

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report by a DCI or by a RAR UL grant or fallbackRAR UL grant, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the 'Time domain resource assignment' field value m for the scheduled PUSCH on the serving cell of the DCI or the PUSCH time resource allocation field value m of the RAR UL grant or of the fallbackRAR UL grant provides a row index m + 1 to a resource allocation table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, the number of slots used for TBS determination (if numberOfSlotsTBoMS is present in the resource allocation table), and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17 is configured;

-	reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-1-r17, if PUSCH is scheduled by DCI format 0_1 or 0_3 and reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-1-r17 is configured;

-	reportSlotOffsetList or reportSlotOffsetList-r17, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorDCI-0-1  is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH, by RAR UL grant, or by fallbackRAR UL grant.

-	for PUSCH scheduled by DCI format 0_1 or DCI format 0_2, if numberOfSlotsTBoMS is present and larger than 1, the UE applies TB processing over multiple slots procedure when determining the time domain resource allocation.

-	For PUSCH repetition Type A and TB processing over multiple slots, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol and length of the indexed row of the resource allocation table, respectively.

-	For PUSCH repetition Type A and TB processing over multiple slots, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

-	For PUSCH repetition Type B, the PUSCH mapping type is set to Type B.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For TB processing over multiple slots, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1,

-	the number of slots used for TBS determination N is indicated by numberOfSlotsTBoMS.

-	the number of repetitions K of the number of slots N used for TBS determination is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	otherwise, K=1.

-	when the UE supports repetition of TB processing over multiple slots, the UE does not expect that N∙K is larger than 32.

When configured with m= 5 or 6 the UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, where multiple DCIs are not associated with CORESETs having different coresetpoolIndex.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1, 0_2 or 0_3 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

-	the number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3-RepetitionsList is configured. The number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3-RepetitionsList is configured. The number of slots used for TBS determination N is equal to 1.

Table 6.1.2.1-1A: Number of repetition K as a function of 2 MSBs of MCS information field

| numberOfMsg3-RepetitionsList is configured |  |  | numberOfMsg3-RepetitionsList is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | K |  | Codepoint | K |
| 00 | First value of numberOfMsg3-RepetitionsList |  | 00 | 1 |
| 01 | Second value of numberOfMsg3-RepetitionsList |  | 01 | 2 |
| 10 | Third value of numberOfMsg3-RepetitionsList |  | 10 | 3 |
| 11 | Fourth value of numberOfMsg3-RepetitionsList |  | 11 | 4 |

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, when any two UL DCIs end in the same symbol and at least one of the DCIs scheduling multiple PUSCHs, the UE does not expect that the any scheduled multiple PUSCHs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PUSCH till the end of the last scheduled PUSCH.

For unpaired spectrum:

-	When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

-	The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, and the TDRA information field value in the RAR UL grant.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst and the TDRA information field value in the DCI scheduling the PUSCH.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 scrambled by TC-RNTI, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For paired spectrum and SUL band:

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3.

-	For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH.

If a UE would transmit a PUSCH of PUSCH repetition Type A when AvailableSlotCounting is enabled and K>1 or a TB processing over multiple slots over N∙K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the N∙K slots, according to Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], the UE counts the slots in the number of N∙K slots.

For PUSCH repetition Type A, in case K>1,

-	If the PUSCH is scheduled by DCI format 0_1 or 0_2

-	if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	Otherwise, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	Else if the PUSCH is scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For TB processing over multiple slots:

-	For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the  N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2.

For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ((n-(n mod N))/N) mod 4 = 0 | ((n-(n mod N))/N) mod 4 = 1 | ((n-(n mod N))/N) mod 4 = 2 | ((n-(n mod N))/N) mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

When transmitting MsgA PUSCH on a non-initial UL BWP, if the UE is configured with startSymbolAndLengthMsgA-PO, the UE shall determine the S and L from startSymbolAndLengthMsgA-PO.

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS 38.213].

For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions. For the n-th nominal repetition, n = 0, …, numberOfRepetitions - 1,

-	The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by .

-	The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by .

Here is the slot where the PUSCH transmission starts, and is the number of symbols per slot as defined in Clause 4.3.2 of [4, TS38.211].

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For a reduced capability half-duplex UE in paired spectrum, symbols that do not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index indicated by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	For operation with shared spectrum channel access with semi-static channel occupancy, symbols in an idle duration associated with a periodic channel occupancy as described in Clause 4.3.1.1 of [16, 37.213], or in an idle duration in a period associated with an initiated channel occupancy as described in Clause 4.3.2. of [16, TS 37.213] are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	The UE may be configured with the higher layer parameter invalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by invalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by invalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of msec. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows:

-	if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorDCI-0-2  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	otherwise, the UE applies the invalid symbol pattern.

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

and

-	a symbol is considered as an invalid symbol in another cell among the set of serving  cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2, where N=1.

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a 'CSI request' field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, if a row indicates resource allocation for two to eight contiguous PUSCHs and extendedK2 is not configured, K2 given by k2-r16 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1.

For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, if a row indicates resource allocation of more than one PUSCH and extendedK2 is configured, each PUSCH has a separate SLIV, mapping type and K2 given by extendedK2. If a row indicates resource allocation of a single PUSCH, the PUSCH has a single SLIV, mapping type, and K2, where K2 is given by extendedK2, if configured, otherwise K2 is given by k2-r16. The number of scheduled PUSCHs is signalled by the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, and the UE is indicated re-transmission of PUSCH by DCI format 0_1, where the PUSCH is correspond to a configured grant Type 1 or Type 2, the UE does not expect that the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH by the DCI is more than one.

When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3 if the same field is available. When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI, MCS-C-RNTI or SP-CSI-RNTI with K2 smaller than K2min⋅2μ'2μ, where K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PUSCH transmission is scheduled by RAR UL grant or fallbackRAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows:

-	if a DCI format 0_3 schedules the PUSCH, the first SRS resource set is associated with all K consecutive slots,

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "00" for the SRS resource set indicator, the first SRS resource set is associated with all K consecutive slots,

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all K consecutive slots,

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to K consecutive slots is determined as follows:

-	When K = 2, the first and second SRS resource sets are applied to the first and second slot of 2 consecutive slots, respectively.

-	When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots.

-	When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second slots of K consecutive slots, and the second SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots.

-	Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to K consecutive slots is determined as follows,

-	When K = 2, the second and first SRS resource set are applied to the first and second slot of 2 consecutive slots, respectively.

-	When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots.

-	When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second slot of K consecutive slots, and the first SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots.

For PUSCH repetition Type B, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', the SRS resource set association to nominal PUSCH repetitions follows the same method as SRS resource set association to slots in PUSCH Type A repetition by considering nominal repetitions instead of slots.

For both PUSCH repetition Type A and PUSCH repetition Type B, when a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" or "11" for the SRS resource set indicator, the redundancy version to be applied on the nth transmission occasion (for PUSCH repetition Type A) of the TB, where n = 0, 1, … K-1, or nth actual repetition (for PUSCH repetition Type B, with the counting including the actual repetitions that are omitted) is determined according to Table 6.1.2.1-2 and Table 6.1.2.1-3. For all PUSCH repetitions associated with the SRS resource set of the first transmission occasion or actual repetition, the redundancy version to be applied is derived according to Table 6.1.2.1-2, where n is counted only considering PUSCH transmission occasions or actual repetitions associated with the same SRS resource set as the first transmission occasion or actual repetition. The redundancy version for PUSCH transmission occasions or actual repetitions that are associated with an SRS resource set other than the SRS resource set of the first transmission occasion or actual repetition is derived according to Table 6.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV in PUSCH-Config and n is counted only considering PUSCH transmission occasions or actual repetitions that are not associated with the SRS resource set of the first transmission occasion or actual repetition.

Table 6.1.2.1-3: Applied redundancy version for the other SRS resource set (SRS resource set not associated with the first transmission occasion or actual repetition) when sequenceOffsetforRV is present

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

For PUSCH repetition Type A, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, the CSI report(s) multiplexing is determined as follows

-	if higher layer parameter AP-CSI-MultiplexingMode in CSI-AssociatedReportConfigInfo is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately only on the first transmission occasion associated with the first SRS resource set and the first transmission occasion associated with the second SRS resource set.

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH transmissions of TB processing over multiple slots, when a DCI format 0_1 and DCI format 0_2 schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, the CSI report(s) is transmitted only on the first slot of the 𝑁 ∙ 𝐾 slots determined for the PUSCH transmission.

For PUSCH repetition Type B, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, CSI report(s) multiplexing is determined as follows

-	if higher layer parameter AP-CSI-MultiplexingMode in CSI-AssociatedReportConfigInfo is enabled and the first actual repetition associated with the first SRS resource set and the first actual repetition associated with the second SRS resource set have the same number of symbols and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is multiplexed separately only on the first actual repetition associated with the first SRS resource set and first actual repetition associated with the second SRS resource set.

-	otherwise, the CSI report(s) is multiplexed only on the first actual repetition.

The UE does not expect a different number of actual PT-RS ports for the two actual repetitions when the CSI report(s) is transmitted separately on two actual repetitions.

For PUSCH repetition Type A, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of repetitions is assumed to be 2 regardless of the value of numberOfRepetitions or pusch-AggregationFactor (if numberOfRepetitions is not present in the time domain resource allocation table), and transmission of CSI report(s) is determined as follows

-	if higher layer parameter AP-CSI-MultiplexingMode in CSI-AssociatedReportConfigInfo is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first transmission occasion and the second transmission occasion

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH repetition Type B, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions, and the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, and transmission of CSI report(s) is determined as follows:

-	if higher layer parameter AP-CSI-MultiplexingMode in CSI-AssociatedReportConfigInfo is enabled for aperiodic CSI report(s) or higher layer paremeter SP-CSI-MultiplexingMode in CSI-SemiPersistentOnPUSCH-TriggerStateList is enabled for semi-persistent CSI report(s) and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first actual repetition and the second actual repetition

-	otherwise, the CSI report(s) is transmitted only on the first actual repetition.

The UE does not expect a different number of actual PT-RS ports for the two actual repetitions when the CSI report(s) is transmitted separately on two actual repetitions.

For PUSCH repetition Type A, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and activate semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions or pusch-AggregationFactor (if numberOfRepetitions is not present in the time domain resource allocation table), and transmission of CSI report(s) is determined as follows

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistenetOnPUSCH-TriggerStateList is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first transmission occasion and the second transmission occasion

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH repetition Type B, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and the PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions, and transmission of CSI report(s) is determined as follows

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistenetOnPUSCH-TriggerStateList is enabled and one of the first or second nominal repetition is the same as corresponding first or second actual repetition, the nominal repetition that is not having same actual repetition is omitted and the CSI report(s) is transmitted on the actual repetition that is not omitted.

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistenetOnPUSCH-TriggerStateList is enabled and the first and second nominal repetitions are the same as the first and second actual repetitions and the UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first actual repetition and the second actual repetition

-	otherwise, the CSI report(s) is transmitted only on the first actual repetition.

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

Table 6.1.2.1.1-1, Table 6.1.2.1.1-1A, Table 6.1.2.1.1-1B and Table 6.1.2.1.1-1C define which PUSCH time domain resource allocation configuration to apply.

Table 6.1.2.1.1-4 defines the subcarrier spacing specific values j. j is used in determination of K2 in conjunction to table 6.1.2.1.1-2, for normal CP or table 6.1.2.1.1.-3 for extended CP, where µPUSCH is the subcarrier spacing configurations for PUSCH.

Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of PUSCH scheduled by the RAR or by the fallbackRAR. When the UE transmits a PUSCH scheduled by RAR or by the fallbackRAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallbackRAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationListForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No | - | Yes | pusch-TimeDomainAllocationListForMultiPUSCH provided in pusch-Config |

Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-2 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | pusch-TimeDomainAllocationListDCI-0-2 provided in pusch-Config |

Table 6.1.2.1.1-1C: Applicable PUSCH time domain resource allocation for DCI format 0_3 in UE specific search space scrambled with C-RNTI or MCS-C-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-3: Default PUSCH time domain resource allocation A for extended CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 5 | 11 |
| 6 | 21 |

Table 6.1.2.1.1-5: Definition of value Δ

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |
| 5 | 24 |
| 6 | 48 |

#### 6.1.2.2	Resource allocation in frequency domain

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI except for a PUSCH transmission scheduled by a RAR UL grant or fallbackRAR UL grant, in which case the frequency domain resource allocation is determined according to clause 8.3 of [6, 38.213] or a MsgA PUSCH transmission with frequency domain resource allocation determined according to clause 8.1A of [6, 38.213]. Three uplink resource allocation schemes type 0, type 1 and type 2 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 and type 2 are supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the 'Frequency domain resource' assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to 'dynamicSwitch', for DCI format 0_1 or setting a higher layer parameter resourceAllocationDCI-0-2 in pusch-Config to 'dynamicSwitch' for DCI format 0_2 or setting a higher layer parameter resourceAllocationDCI-0-3 in pusch-Config to 'dynamicSwitch' for DCI format 0_3, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation for DCI format 0_1 or the higher layer parameter resourceAllocationDCI-0-2 for DCI format 0_2. The UE shall assume that when the scheduling PDCCH is received with DCI format 0_1 and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, uplink type 2 resource allocation is used.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used, except when any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured in which case uplink resource allocation type 2 is used.

The UE expects that either none or both of useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured.

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active bandwidth part change via DCI, the RB indexing for uplink type 0, type 1 and type 2 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active bandwidh part change via DCI, the RB indexing for uplink type 0, type 1, type 2 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part. RB numbering starts from the lowest RB in the determined uplink bandwidth part.

##### 6.1.2.2.1	Uplink resource allocation type 0

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size for DCI format 0_1/0_2 or rbg-SizeDCI-0-3 for DCI format 0_3 configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 | Configuration 3 |
| --- | --- | --- | --- |
| 1 – 36 | 2 | 4 | 8 |
| 37 – 72 | 4 | 8 | 16 |
| 73 – 144 | 8 | 16 | 32 |
| 145 – 275 | 16 | 16 | 32 |

The total number of RBGs () for a uplink bandwidth part i of sizePRBs is given by  where

-	the size of the first RBG is ,

-	the size of the last RBG is if and P otherwise.

-	the size of all other RBG is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency of the bandwidth part and starting at the lowest frequency. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

In frequency range 1, only 'almost contiguous allocation' defined in [8, TS 38.101-1] is allowed as non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM.

In frequency range 2, non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported.

##### 6.1.2.2.2	Uplink resource allocation type 1

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the size of the initial UL bandwidth part  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

When the DCI size for DCI format 0_0 in USS is derived from the initial UL BWP with size  but applied to another active BWP with size of , an uplink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks .

The resource indication value is defined by

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

When the scheduling grant is received with DCI format 0_2 or 0_3, an uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0, 1, …, NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=1, …, NRBG, where the resource block groups are defined as in 6.1.2.2.1 with P defined by resourceAllocationType1GranularityDCI-0-2 for DCI format 0_2 and by resourceAllocationType1GranularityDCI-0-3 for DCI format 0_3 if the UE is configured with higher layer parameter resourceAllocationType1GranularityDCI-0-2 or resourceAllocationType1GranularityDCI-0-3, and P=1 otherwise. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

<omitted text>

#### 6.1.2.3	Resource allocation for uplink transmission with configured grant

<omitted text>

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

<omitted text>

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1, 0_2 or 0_3, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to '1', and if in this DCI the UE detects ACK for the HARQ process corresponding to that transport block.

<omitted text>

##### 6.1.2.3.2	Transport Block repetition for uplink transmissions of PUSCH repetition Type B with a configured grant

<omitted text>

For any RV sequence, the repetitions shall be terminated after transmitting K nominal repetitions, or at the last transmission occasion among the K nominal repetitions within the period P, or from the starting symbol of an actual repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1, 0_2 or 0_3, whichever is reached first. The UE is not expected to be configured with the time duration for the transmission of K nominal repetitions larger than the time duration derived by the periodicity P.

<omitted text>

### 6.1.4	Modulation order, redundancy version and transport block size determination

To determine the modulation order, target code rate, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field in the DCI scheduling PUSCH or provided in a DCI activating a configured grant Type 2 PUSCH, or as provided by mcsAndTBS as described in Clause 6.1.2.3 for a configured grant Type 1 PUSCH to determine the modulation order  and target code rate (R) based on the procedure defined in Clause 6.1.4.1

-	read redundancy version field (rv) in the DCI to determine the redundancy version for PUSCH scheduled by DCI, or determine the redundancy version according to Clause 6.1.2.3.1 for configured grant Type 1 and Type 2 PUSCH,

and second

-	use the number of layers , the total number of allocated PRBs  to determine the transport block size based on the procedure defined in Clause 6.1.4.2.

When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field.

Within a cell group, a UE is not required to handle PUSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate,

where

-	J is the number of configured serving cells belong to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot-sj. For PUSCH repetition Type B, each actual repetition is counted separately.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PUSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C'is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5,38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to 'enable', or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, or if it is an actual repetition for PUSCH repetition Type B, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PUSCH

-	M is the number of TB in the PUSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PUSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i)

-	each actual repetition for PUSCH repetition type B is treated as one PUSCH.

#### 6.1.4.1	Modulation order and target code rate determination

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by a fallbackRAR UL grant or

for a MsgA PUSCH transmission, or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or

for a PUSCH scheduled by a DCI format 0_3 with CRC scrambled by C-RNTI, MCS-C-RNTI, or

for a PUSCH with configured grant using CS-RNTI, and

if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3

-	if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel;

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2  in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 or 0_3 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant,

-	the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3-Repetitions is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI,

-	the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3-Repetitions is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

else

-	if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 or 0_3 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant,

-	the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3-Repetitions is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI,

-	the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3-Repetitions is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

end

For Table 6.1.4.1-1 and Table 6.1.4.1-2, if higher layer parameter tp-pi2BPSK is configured, q = 1 otherwise q=2.

<omitted text>

#### 6.1.4.2	Transport block size determination

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by fallbackRAR UL grant or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_3 with CRC scrambled by C-RNTI, MCS-C-RNTI, or

for a PUSCH transmission with configured grant, or

for a MsgA PUSCH transmission,

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

- The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH,  is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1, 0_2 or 0_3 or as described for DCI format 0_0 in Clause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 6, 12, or 18), the  is assumed to be 0. For Msg3 or MsgA PUSCH transmission the  is always set to 0. In case of PUSCH repetition Type B,  is determined assuming a nominal repetition with the duration of L symbols without segmentation.

-	A UE determines the total number of REs allocated for PUSCH  as follows

-	For TB processing over multiple slots, NRE=N*min156,NRE'⋅nPRB where  is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS.

-	Otherwise, NRE=min156,NRE'⋅nPRB.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

-	For a PUSCH scheduled by fallbackRAR UL grant, UE assumes the TB size determined by the UL grant in the fallbackRAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission.

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 P

<omitted text>

### 6.1.7	UE procedure for determining time domain windows for bundling DM-RS

For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows:

-	For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is:

-	Given by PUSCH-TimeDomainWindowLength, if configured.

-	Computed as min (maxDurationDMRS-Bundling, M), if PUSCH-TimeDomainWindowLength is not configured, where maxDurationDMRS-Bundling is maximum duration for a nominal TDW subject to UE capability [13, TS 38.306], M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where:

-	For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1 or in Clause 6.1.2.3.

-	For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1 or in Clause 6.1.2.3.

-	For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1 or in Clause 6.1.2.3.

-	For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is:

-	Given by PUCCH-TimeDomainWindowLength, if configured.

-	Computed as min (maxDurationDMRS-Bundling, M), if PUCCH-TimeDomainWindowLength is not configured, where maxDurationDMRS-Bundling is maximum duration for a nominal TDW subject to UE capability [13, TS 38.306], M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213].

-	For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots:

-	The start of the first nominal TDW is the first slot determined for the first PUSCH transmission.

-	The end of the last nominal TDW is the last slot determined for the last PUSCH transmission.

-	The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW.

-	For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B:

-	The start of the first nominal TDW is the first slot for the first PUSCH transmission.

-	The end of the last nominal TDW is the last slot for the last PUSCH transmission.

-	The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW.

-	For PUCCH transmissions of a PUCCH repetition:

-	The start of the first nominal TDW is the first slot determined for the first PUCCH transmission.

-	The end of the last nominal TDW is the last slot determined for the last PUCCH transmission.

-	The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW.

For PUSCH transmissions of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, a nominal TDW consists of one or multiple actual TDWs. The UE determines the actual TDWs as follows:

-	The start of the first actual TDW is the first symbol of the first PUSCH transmission in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW.

-	The end of an actual TDW is

-	The last symbol of the last PUSCH transmission in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, if the actual TDW reaches the end of the last PUSCH transmission within the nominal TDW.

-	The last symbol of a PUSCH transmission before the event, if an event occurs which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots.

-	When pusch-WindowRestart is enabled, the start of a new actual TDW is the first symbol of the PUSCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots.

For PUCCH transmissions of PUCCH repetition, a nominal TDW consists of one or multiple actual TDWs. The UE determines the actual TDWs as follows:

-	The start of the first actual TDW is the first symbol of the first PUCCH transmission in a slot determined for PUCCH transmission within the nominal TDW.

-	The end of an actual TDW is

-	The last symbol of the last PUCCH transmission in a slot determined for transmission of the PUCCH within the nominal TDW, if the actual TDW reaches the end of the last PUCCH transmission within the nominal TDW.

-	The last symbol of a PUCCH transmission before the event, if an event occurs which causes power consistency and phase continuity not be maintained across PUCCH transmissions of PUCCH repetition within the nominal TDW, and the PUCCH transmission is in a slot determined for transmission of the PUCCH.

-	When pucch-WindowRestart is enabled, the start of a new actual TDW is the first symbol of the PUCCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUCCH transmissions of PUCCH repetition within the nominal TDW, and the PUCCH transmission is in a slot determined for transmission of the PUCCH.

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are:

-	A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum.

-	The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix.

-	The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions.

-	For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

-	For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213].

-	For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1.

-	For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213].

-	Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213].

-	Frequency hopping.

-	For reduced capability half-duplex UEs,

-	a dropping or cancellation of a PUSCH or PUCCH transmission according to clause 17.2 of [6, TS 38.213] or

-	an overlapping of the gap between two consecutive PUSCH or two consecutive PUCCH transmissions and any symbol of downlink reception or downlink monitoring

The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled.

<omitted text>

## 6.2	UE reference signal (RS) procedure

### 6.2.1	UE sounding procedure

<omitted text>

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2  symbols and an additional time duration  Tswitch. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 +14 symbols and an additional time duration Tswitch. The minimal time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min(µPDCCH, µUL) where µUL is given by min(µUL,carrier1, µUL,carrier2, µSRS) when the UE is configured with the higher layer parameter uplinkTxSwitchingOption set to 'dualUL' for uplink carrier aggregation, and by µSRS otherwise. µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively.

-	Tswitch, µUL,carrier1 and µUL,carrier2 are defined in clause 6.4.

-	When UE reporting [Triggering SRS only in DCI 0_1/0_2], the UE can be indicated with DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI as described in clause 7.3.1.1 of TS38.212. Otherwise, except for DCI format 0_1/0_2 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1/0_2 with UL-SCH indicator of "0" and CSI request of all zero(s) as described in clause 7.3.1.1 of [5, TS 38.212].

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource,

-	If ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot ,

-	otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

<omitted text>

The SRS request field [5, TS38.212] in DCI format 0_1, 1_1, 0_2 (if SRS request field is present), 1_2 (if SRS request field is present), 0_3, 1_3 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38.212]. The 2-bit SRS request field in DCI format 2_3 indicates the triggered SRS resource set given in clause 7.3 of [5, TS 38.212] and defined by the entries of the higher layer parameter srs-ResourceSetToAddModList if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB', or indicates the SRS transmission on a set of serving cells configured by higher layers if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'.

<omitted text>

### 6.2.2	UE DM-RS transmission procedure

The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 and dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_3.

When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and

If frequency hopping is disabled:

-	The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or

If frequency hopping is enabled:

-	The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration.

When transmitted PUSCH is scheduled by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type provided by higher layer parameter dmrs-Type in DMRS-UplinkConfig on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and additional DM-RS with dmrs-AdditionalPosition from ConfiguredGrantConfig can be transmitted according to the scheduling type and the PUSCH duration as specified  in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled.

For the UE-specific reference signals generation as defined in Clause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B.

When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure,

-	for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRSports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213].

-	the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Clause 6.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig, or by higher layer parameter msgA-MaxLength in msgA-DMRS-Config,

-	if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3'.

-	if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'.

-	for MsgA PUSCH for Type-2 random access procedure the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter msgA-DMRS-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3' for single-symbol DM-RS or 'pos0', 'pos1' for double-symbol DM-RS.

-	and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Clause 6.4.1.1.3 of [4, TS 38.211].

If a UE transmitting PUSCH scheduled by DCI format 0_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 or dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2, or a UE transmitting PUSCH scheduled by DCI format 0_0, 0_1 or 0_3 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH

-	any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE.

For PUSCH scheduled by DCI format 0_1, by activation DCI format 0_1 with CRC scrambled by CS-RNTI, or configured by configured grant Type 1 configuration, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Clause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, the UE shall assume that the number of DM-RS CDM groups without data is 3 which corresponds to CDM group {0,1,2} for the case of PUSCH scheduled by activation DCI format 0_0 and the dmrs-Type in DMRS-UplinkConfig equal to 'type2' and the PUSCH allocation duration being more than 2 OFDM symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-DMRS-CDM-group, the UE shall assume that 2 DM-RS CDM groups are configured. Otherwise, msgA-PUSCH-DMRS-CDM-group indicates which DM-RS CDM group to use from the set of {0,1}.

For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-NrofPorts, the UE shall assume that 4 ports are configured per DM-RS CDM group for double-symbol DM-RS. Otherwise, msgA-PUSCH-NrofPorts with value of 0 indicates the first port per DM-RS CDM group, while a value of 1 indicates the first two ports per DM-RS CDM group.

For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor  specified in clause 6.4.1.1.3 of [4, TS 38.211] is given by .

Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

For PUSCH repetition Type B, the DM-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. A UE is not expected to be indicated with an antenna port configuration that is invalid for the allocated duration of any actual repetition.

### 6.2.3	UE PT-RS transmission procedure

The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 or dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, to PUSCH transmissions scheduled by DCI format 0_0, 0_1 or 0_3 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PT-RS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition.

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

<omitted text>

For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field(s) in DCI format 0_1, 0_2 and 0_3. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212].

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0.

For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1, 0_2 or 0_3 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers:

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as:

-	PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1.

-	UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 and 0_3 described in Clause 7.3.1 of [5, TS38.212].

When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

<omitted text>

## 6.3	UE PUSCH frequency hopping procedure

### 6.3.1	Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots

For PUSCH repetition Type A other than the PUSCH scheduled by RAR UL grant or fallbackRAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission, multi-slot PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3, each of multiple PUSCH transmissions on a serving cells scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured and each of multiple configured grant PUSCH transmissions in a configuration where the higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot are provided.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

For operation with shared spectrum channel access in FR1, the UE does not expect that two hops of a PUSCH transmission are in different RB sets.

In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping.

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1/0_3 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

<omitted text>