3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2304202

# e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of dynamic spectrum sharing enhancements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh-Core |  |  |  | Date: |  | 2023-04-27 |
| Category: | B |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of dynamic spectrum sharing enhancements |  |  |  |  |  |
| Summary of change: |  | Introduction of additional resource mapping procedures. |  |  |  |  |  |

| Consequences if not approved: | Incomplete support for Rel-18 enhancements to dynamic spectrum sharing |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 5.1.4.2 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38.211, 38.213 |
| affected: |  | X | Test specifications |  |
| (show related CRs) |  | X | O&M Specifications |  |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in pdsch-ConfigMulticast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for broadcast PDSCH. The total number of RateMatchPatternLTE-CRS for broadcast reception that a UE can be configured with is the same as for unicast in Rel-15.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’;

-	otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts in CSI-RS-ResourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type in CSI-RS-ResourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping in ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP CSI-RS.

-	For the UE in RRC_CONNECTED mode for multicast reception, p-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH. The REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config and pdsch-ConfigMulticast, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config.

-	For the UE in RRC_CONNECTED mode for multicast reception, sp-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by sp-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH when their triggering and activation delivered by unicast PDSCH are applied. The REs indicated by sp-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by sp-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of semi-persistent ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast.

The UE may be configured with a DCI field for triggering the aperiodic ZP CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of 'ZP CSI-RS' trigger in DCI format 1_1 triggers one aperiodic 'ZP-CSI-RS-ResourceSet' in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '1', the DCI codepoint '10' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '2', and the DCI codepoint '11' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '3'. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList or in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for PDSCH.

When the UE is configured with multi-slot and single-slot PDSCH scheduling or pdsch-TimeDomainAllocationListForMultiPDSCH-r17, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the PDSCH(s) scheduled or the PDSCHs with SPS activated by the PDCCH containing the trigger.

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

<omitted text>