3GPP TSG RAN WG1 #112bis-e		                                   					       												                  R1-2304217

e-Meeting, April 17th – April 26th, 2023

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.17.15

Agenda Item:	9.17.15

Document for: 	Endorsement

### UE features for BWP without restriction

Initial input on UE features for BWP without restriction to be provided by rapporteur.

[112bis-e-R18-UE_features-04] Email discussion on UE features for BWP without restriction by April 26 – Ralf (AT&T)

- Check points: April 21, April 26

Agreement: Introduce the following new FG/row, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for SCell] | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new FG/row, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured] [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for SCell] | [6-1a, [1-7, 2-24, 2-31, 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of [the corresponding carrier(s) to be measured.] [UE indicates at most one of FG 53-1 and 53- 2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new FG/row, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for Scell] | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | [Per band] | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in red, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for SCell] | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured] [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for SCell] | [6-1a, [1-7, 2-24, 2-31, 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of [the corresponding carrier(s) to be measured.] [UE indicates at most one of FG 53-1 and 53- 2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. [2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include SSB for Scell] | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | [Per band] | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |

- FFS: further clarify relationship to existing FG 6-1a for Option A (RLM/BM/BFD measurements based on CSI-RS within active BWP)

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | [Per band] | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:  Adopt the following changes highlighted in red, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell | [1-7, 2-24, 2-31, 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of [the corresponding carrier(s) to be measured.] [UE indicates at most one of FG 53-1 and 53- 2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |

R1-2302415	UE features for BWP without restriction	Ericsson

R1-2302517	Discussion on UE features for BWP without restriction	vivo

R1-2302765	Discussion on UE feature for BWP without restriction	ZTE

R1-2302876	Preliminary UE Features List for BWP Without Restriction	Vodafone Italia SpA, vivo

R1-2302895	Initial views on UE features for BWP without restriction	Nokia, Nokia Shanghai Bell

R1-2303350	UE features for BWP without restriction	MediaTek Inc.

R1-2303623	UE features for BWP without restriction	Qualcomm Incorporated

R1-2303653	Summary of UE features for BWP without restriction	Moderator (AT&T)

R1-2303738	Discussion on UE features for BWP without restriction	NTT DOCOMO, INC.

R1-2303865	UE features for BWP without restriction	Huawei, HiSilicon