3GPP TSG RAN WG1 #112bis-e	R1-2304233

e-Meeting, April 17th - April 26th, 2023

Agenda item:	9.5.1.3

Sourcce: 	Moderator (Qualcomm)

Title: 	Moderator Summary #4 on resource allocation for SL PRS

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved [1]. The relevant WID objective is copied here for convenience:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre)-configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present the summary of the documents submitted in this subagenda

# Discussion Information

The proposals are ranked according to HIGH, MEDIUM, LOW. The intention is to start in the offline/online sessions from the HIGH proposals.

# SL Positioning Resource Allocation

## 3.1 General Aspects on Resource Pools

### 3.1.1 Slots for resource pool for positioning

The following agreements have been reached:

- Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. - Agreement For a dedicated resource pool for positioning: - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). - FFS: additional slots that can be used for SL PRS is not precluded - Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

The following related proposals were found in the submitted papers:

| Huawei, HiSilicon | Proposal 2: Support using the reserved slots to transmit SL-PRS in dedicated resource pool for positioning. |
| --- | --- |
| CATT, GOHIGH | Proposal 1: For a dedicated resource pool for positioning, the set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool, no other additional slots can be used for SL PRS, e.g., the reserved slots. |
| Xiaomi | Proposal 1: The UE determines the set of slots that may belong to a SL PRS dedicated resource pool in the same way as for SL communication pool. Proposal 2: The set of slots assigned to a SL PRS dedicated resource pool are (pre)configured. |
| ZTE | Proposal 4: For the support of dedicated SL-PRS resource pool: - Pool level TDMed restriction by RRC signaling between SL-PRS resource pool and SL communication pool should be avoided. - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214) - The set of slots (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS) that may belong to a dedicated resource pool includes all the slots except NS-SSB slots, NnonSL slots and reserved slots. - According to (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS), the set of slots assigned to a dedicated resource pool is based on a bitmap configured in the resource pool.c |
| Apple | Proposal 1: It is necessary to clarify if a slot dedicated for SL-PRS transmission within a SL communication resource pool or the SL-PRS requires its own dedicated RP to be defined as a dedicated RP. |

We make the following observations:

- In addition to the set of slots already agreed, add the reserved slots for SL-PRS transmission

  - Huawei, HiSilicon

- No additional slots need to be supported

  - CATT, GOHIGH, Xiaomi, ZTE

[CLOSED] Feature Lead Proposal 3.1.1-v0

For a dedicated resource pool for positioning:

- No additional slots are needed to be supported

Companies views

| CATT | Support. |
| --- | --- |
| vivo | Support. |
| Sharp | We support the proposal. |
| Qualcomm | Support |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | Support |
| Spreadtrum | Support |
| CMCC | Support. |
| Intel | Agree |
| ZTE | Support. We do not support any additional slots, For example, reserved slots defined in SL communication can not be used in SL-PRS resource pool. The number and location of reserved slots of SL communication resource pool is highly related to the bitmap (e.g. the length of bitmap) defined per each pool. A UE can be (pre-)configured with multiple SL communication resource pools where each may have different reserved slots. |
| Fraunhofer | We recognize the potential benefit of SL-PRS reserved slots, but we are also open to accept the majority view. |
| Lenovo | Support |
| Huawei, HiSilicon | We can accept it for the sake of progress. |
| Toyota | Support. |
| Futurewei | Support |
| Apple | We are fine with the proposal and appreciate the clarification. |
| Nokia, NSB | Support |
| Ericsson | Support |

### 3.1.2 Configuration parameters for Resource Pool for Positioning

The following related proposals were found in the submitted papers:

| LGE | Proposal 3: The resource pool option for SL positioning are (pre-)configured. Proposal 19: SL positioning resource pool configuration includes at least the following parameters. - Frequency domain resource granularity - PSCCH configuration - PSSCH configuration - Allowed SL positioning method (e.g. SL RTT, SL TDOA, SL AoA) - SL PRS resource configuration - Allowed multiplexing scheme (e.g. comb-based, TDM) - Enable/disable of each multiplexing scheme - Enable/disable of the feedback-based retransmission - Associated resource pool information (e.g. for SLPP, measurement report transmission) |
| --- | --- |
| CATT, GOHIGH | Proposal 2: For a dedicated resource pool for positioning, the following information should be (pre-) configured: - Time-domain bitmap for available slots - Start PRB position and the number of contiguous PRBs - Sub-channel size and sub-channel count - SL-PRS configuration - Resource allocation Scheme 2 related configuration - Power control configuration. - Reporting configuration. |
| Samsung | Proposal 1: Both in the dedicated and shared resource pool, a UE receives the following SL PRS configuration parameters from higher layers as - SL PRS starting sub-channel and its length (i.e., the number of sub-channels for SL PRS) - Candidate values for SL PRS comb size (N) - SL PRS comb offset from the reference sub-carrier - SL PRS starting symbol and symbol length (M) within a slot - Time offset (Toffset) from the reference slot - SL PRS time domain behaviour (periodic or aperiodic) and periodicity (Tperiod) for periodic transmission - SL PRS repetition period (Nrep) and time gap (Tgap) between repetitions - SL PRS muting pattern - SL PRS max power - Latency bound for SL PRS transmission or measurement report - Note: This includes the pre-configuration case also. |
| China telecom | Proposal 2: The configurable number of PRBs for a dedicated resource pool larger than 100 can be considered. |
| lenovo | Proposal 10: RAN1 to define the SL PRS resource hierarchy in terms of SL Carrier SL BWPSL Resource PoolSL PRS Resource Set SL PRS Resources. |
| ZTE | Proposal 5: A dedicated SL-PRS resource pool should at least include the following configurations: - SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool - SL-PRS configuration: the period of SL PRS resource/SL PRS resource set, number of symbols that a SL PRS occupied in a slot, start symbol of SL PRS in a slot, comb size and RE offset, priority of SL PRS resource/SL PRS resource set - PSCCH configuration: candidates of time resource and frequency resource of PSCCH - Synchronization configuration: indicates the allowed synchronization reference(s) which is (are) allowed to use the configured SL-PRS resource pool - Resource allocation scheme 2 related configuration: CBR config and its association relationship with SL-PRS transmission parameter, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold - Power control configuration: maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control |
| CEWIT | Proposal 8: Dedicated resource pool (pre)-configuration should include - the time domain bit map of slots, - reserved slots bit map, - SL-start symbol per slot, - start PRB position, - the number of contiguous PRBs, - sub-channel size and sub-channel count, - SL-PRS configuration, - SL-PSCCH configuration, - SL-PSSCH configuration related to SL-PRS, - power control configuration, - reporting configuration, I - UC configurations. |
| Ericsson | The following SL PRS parameters are configured in the pool containing the SL PRS (dedicated or shared) - Starting PRB for the FDRA (lowest RB index of the subchannel with the lowest index) - Subchannel size (at least for shared resource pool) - Maximum bandwidth The listening UEs receives the SL PRS configuration via higher layers, i.e. from the LMF when the UE is in LMF coverage. When out of coverage, the listening UE receives the SL PRS configuration via higher layers, i.e., from the transmitting UE’s higher layer, via SLPP. For the UE receiving SL PRS, periodicity (if any) is included in assistance data (LPP/SLPP) and reflected in the SCI reservation field. |
| Qualcomm | Proposal 16: Higher layer signaling (SLPP) is used for configuration of SL-PRS. Proposal 17: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources. |
| Xiaomi | Proposal 3: the start PRB position, the number of contiguous PRBs, the start symbol and the length of symbols in a slot are (pre)configured for a SL PRS dedicated resource pool. Proposal 4: SL PRS comb size, SL PRS subchannel size, and the number of TDM multiplexing SL PRS resources in a slot of dedicated resource pool shall be (pre)configured. |

### 3.1.3 Dedicated and Shared resource pool in the SL BWP of a carrier

| CEWiT | Proposal 6: For SL-PRS transmission, it is allowed to (pre-)configure both dedicated resource pool and shared resource pool in the SL-BWP of a carrier optionally. - Note 1: It does not preclude the case of only either of the resource pool is configured in the SL-BWP - Note 2: It does mandate the use of dedicated resource pool only fo SL-PRS transmission and reception. Proposal 7: For a dedicated resource pool for positioning, multiple resource pool for positioning can be (pre-)configured associated with the one SL BWP of the carrier. Proposal 9: Shared resource pool can be multiplexed in TDMed manner with dedicated resources in SL-BWP.c |
| --- | --- |
| Nokia, NSB | Proposal 1: For SL PRS transmission, support (pre-)configuration of either both dedicated and shared resource pools with (Rel-16/17/18) SL communication, or only one of them. |
| Ericsson | - The network is allowed to configure SL PRS resources in both the shared resource pool and the SL PRS dedicated resource pool in the SL BWP of a carrier. - The network assigns SL PRS resource pools for scheme 1 and scheme 2, similarly to sl-TxPoolScheduling (mode 1 resource pools) and sl-TxPoolSelectedNormal (mode 2 resource pools) for legacy sidelink. Note that: - each resource pool can be only associated with either scheme 1 or scheme 2. - an SL PRS resource set can be assigned to both a scheme 1 and a scheme 2 resource pool simultaneously, which may result in an overlap between the two pools. - Support configuration of multiple dedicated resource pool for positioning. - PRS Resource allocation for a PRS in a dedicated pool using a reservation from a shared pool is not supported. |
| NEC | Proposal 3: At least when a dedicated resource pool is used only for SL-PRS transmission and the associated SCI transmission without other channel/signal, the association between the dedicated resource pool and one or multiple shared resource pools should be considered. |
| ZTE | Proposal 1: A UE can be (pre-)configured with one or more SL resource pools for SL-PRS transmission. - The SL resource pools can be dedicated resource pool, shared resource pool. |

[CLOSED] Feature Lead Proposal 3.1.3-v0

- For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier.

  - A UE can be (pre-)configured with one or more dedicated SL resource pools.

  - A UE can be (pre-)configured with one or more shared SL resource pools.

Companies views

| CATT | Support. |
| --- | --- |
| vivo | support. |
| Sharp | We support the proposal. |
| Qualcomm | Support |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | Support |
| Spreadtrum | Support |
| CMCC | Support. |
| Intel | Agree |
| ZTE | Support. It is up to flexible (pre-)configuration. |
| Fraunhofer | Support |
| Lenovo | Support |
| Huawei, HiSilicon | Support. |
| Toyota | Support. |
| Futurewei | Support |
| Apple | Support |
| Nokia, NSB | Support |
| Ericsson | Support |

## 3.2 Dedicated Resource Pool for Positioning

### 3.2.1 General

| OPPO | Proposal 10: For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation and slots configured with PSFCH resources can be included. |
| --- | --- |

### 3.2.2 Slot structure

The following agreement was reached with regards to this topic:

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above - Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 4: Dedicated resource pool for SL positioning should not include any other channel or transmission beyond SL-PRS and PSCCH that carries associated SCI. FFS whether PSFCH is included. Proposal 5: Any signaling associated with a SL PRS transmission in a dedicated resource pool that does not fit into associated control channels (e.g., PSCCH or PSFCH) can be transmitted in an associated resource pool (which could be a shared resource pool or SL-communications-only resource pool). Proposal 6: Consider supporting SL PRS slots with/without PSCCHs in a dedicated resource pool: a SL PRS slot with PSCCHs can be configured occurring once in every K slots; the parameter K is (pre-)configurable and can be configured as 1. |
| --- | --- |
| Futurewei | Proposal 4: Support Option 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) |
| Huawei, HiSilicon | Proposal 1: For the slot structure of SL-PRS dedicated resource pool, - Support to include SL-PRS and PSCCH, i.e., Option 2 - Do not support to include PSSCH, i.e., Option 3. |
| vivo | - PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the dedicated resource pool (i.e., Option 2 is supported). |
| Spreadtrum | Proposal 1: PSCCH and PSCCH which carry SCI associated with SL-PRS transmission(s) should be included in the dedicated resource pool. |
| Intel | - For dedicated resource pool for SL-PRS, Option 2, i.e., PSCCH which carries SCI associated with SL-PRS transmission(s) is included, is supported. - For dedicated resource pool for SL-PRS, - SL-PRS resource pool and SL communication resource pools can be multiplexed in a TDM manner. Support multiplexing of PSCCH and SL-PRS transmission in a TDM manner. |
| Sony | Proposal 5: Support a dedicated resource pool to include PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) (known as Option 3). Proposal 6: The PSSCH associated with SL-PRS transmission(s) contains SL-PRS configuration. FFS the SL-PRS parameters in each channel(s). Proposal 7: Support a dedicated resource pool to include PSFCH-like channel to be used for positioning purpose. |
| Xiaomi | Proposal 5: For a dedicated resource pool for SL positioning, PSCCH which carriers SCI associated with SL PRS transmission(s) is included. |
| LGE | Proposal 2: In a dedicated resource pool, the following channels associated with SL PRS are transmitted. - PSCCH and PSSCH carrying SCI - PSFCH for SL PRS retransmission - FFS: PSSCH carrying SL positioning-related data, such as SLPP or the measurement report |
| China Telecom | Proposal 1: With regards to which channels can be included in the dedicated resource pool in addition to SL-PRS, we support opt. 2 (PSCCH which carries SCI associated with SL-PRS transmission(s) is included). |
| Sharp | In a dedicated resource pool for positioning there is no need to support PSSCH transmission, and hence no need to reuse the 2-stage SCI design. Therefore, we support Alt. 1. Proposal 4: In a dedicated resource pool for positioning, a new single-stage SCI in PSCCH is introduced. |
| CMCC | Proposal 1: RAN1 needs to further study whether 2nd SCI is needed for SL-PRS, when making a comparison b/w Opt. 2 and Opt. 3. - PSSCH can only be used for conveying 2nd SCI in Opt.3; - Other channels, e.g. PSFCH, is not necessary to be included in the dedicated resource pool. |
| ZTE | Proposal 2: For a dedicated resource pool for SL positioning, in addition to SL-PRS, PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the resource pool. - PSSCH is not included in a dedicated resource pool |
| CEWiT | Proposal 10: The dedicated SL resource pool will include configuration of SL-PRS, corresponding PSCCH and PSSCH. PSCCH and PSSCH will be carrying the positioning protocol and SL-PRS related messages. |
| Interdigital | Proposal 25: Include the following channels in a dedicated resource pool in addition to SL-PRS: - PSCCH which carries SCI associated with SL-PRS transmission - PSFCH which carries feedback information associated SL-PRS transmission |
| Apple | Observation 1: With regards to which channels can be included in the resource pool in addition to SL-PRS, the following options require some specification updates: • Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included o Observation 1-1: should support a single stage SCI, and sensing-based and random resource allocation. • Option 3 (PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included ) o Observation 1-2: may be needed to support IUC feedback. Proposal 2: With regards to which channels can be included in the resource pool in addition to SL-PRS, at least Option 2 should be supported. Option 3 is supported if Inter-UE coordination is supported in the dedicated resource pool. |
| Ericsson | - A dedicated pool for positioning does not include any channel for payload data, e.g. PSSCH. |
| Qualcomm | Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool. |
| NEC | Proposal 4: Regarding a dedicated resource pool for SL positioning, only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) is expected to be transmitted. |
| Mediatek | Proposal 2-1: A slot for dedicated resource pool contains the control region for PSCCH and the symbols for allocating SL-PRS |

From the submitted documents we observe the following:

For a dedicated resource pool for SL positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS and PSCCH:

| - Do not include PSSCH and only a 1st stage SCI is used | Nokia, NSB, Huawei, HiSilicon, vivo, Intel, Xiaomi, China Telecom, Sharp, ZTE, Interdigital, Apple, Ericsson, Qualcomm, NEC, Mediatek |
| --- | --- |
| - PSSCH associated with SL-PRS transmission(s) is also included such that: - Alt. 1: 2nd stage SCI is included in the PSSCH | Futurewei, Spreadtrum, LGE, Samsung, Panasonic, CATT, GOHIGH |
| - PSSCH associated with SL-PRS transmission(s) is also included such that: - Alt. 2: 2nd stage SCI and SL-positioning-related information is included in the PSSCH | Sony, CEWiT, Apple (if IUC is supported) |
| - PSFCH-like channel can also be included | Sony, LGE, Interdigital |
| - Continue the study whether a 2nd stage SCI is needed or not | CMCC |

There is relatively strong support for only including PSCCH with single-stage SCI and SL-PRS. The main argument against this approach is:

- 2nd-stage SCI may be needed since the 1st-stage SCI payload is limited (e.g. sidelink scheduling information, source and destinations ID, zone indicator and inter-UE coordination indicator)

This concern has been the same the last couple of meetings that this topic is being discussed and the majority of companies do not seem to consider it a big issue, and therefore, the moderator proposal is to follow this solution:

[CLOSED] Feature Lead Proposal 3.2.2-v0

For a dedicated resource pool for SL positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS and PSCCH:

- Do not include PSSCH. Only a single stage SCI is used for reservation.

  - PSCCH and SL-PRS are TDMed in a slot

- Study further if PSFCH-like channel is needed

Companies views

| CATT | Do not support. In our view , PSCCH and PSSCH which carries SCI should be included if the two stages SCI for SL-PRS were agreed. We slightly prefer two stages SCI, in order to keep the same SCI design with the legacy Rel-16 V2X and keep the same design between dedicated RP and shared RP. |
| --- | --- |
| vivo | Support. In the dedicated resource pool, the single stage SCI can be re-design. And one SCI size can be defined in the dedicated resource pool, which can reduce the complexity of the blind detection of the PSCCH. |
| Sharp | We support the proposal. |
| Xiaomi | Support |
| OPPO | Support the main bullet and affiliated sub-bullets, PSSCH would complicate the slot strucuter significantly and meanwhile degarade the performance of SL positioning, there is no back-ward compatibility constraint in dedicated resource pool, all control information can be put into PSCCH as a new SCI format. We propose following changes, as what the PSCCH looks like is unclear for now, it may not be able to occupy the same bandwidth as the associated SL PRS, we should also consider FDM between PSCCH and associated SL PRS to avoid PSD imbalance among the OFDM symbols. - PSCCH and associated SL-PRS are TDMed in a slot - FFS whether PSCCH and associated SL-PRS can be FDMed in the OFDM symbol(s) for the PSCCH. |
| Qualcomm | Support |
| InterDigital | We support the proposal. |
| NEC | Support the proposal, and we also think PSFCH-like channel is needed. |
| Panasonic | Do not support. We are not sure how a standalone PSCCH is to be designed without PSSCH. We also share similar views as CATT. |
| Spreadtrum | We shared the similar views with CATT. |
| DCM | Similar view with CATT/Panasonic. Clarification is necessary. |
| CMCC | Support. For dedicated RP, support only PSCCH is feasible and is also the most simple solution considering the design of multiplexing of PSCCH and SL PRS in the pool. |
| Samsung | During Rel-16 V2X discussion, there was an observation that two-stage SCI can provide a more reliable sensing accuracy as compared to the single-stage SCI. We understand there can be pros and cons for single and two stage SCI (There was big debate also in Rel-16 V2X). Since we already have the two-stage SCI structure, it is easy to extend for SL positioning. |
| Intel | We are fine with the proposal. For FFS, our view is that it is not reasonable to support HARQ-ACK feedback for SL PRS transmission. This is different from PSSCH, where CRC can be used to determine whether UE successfully receives the PSSCH. We suggest to remove the FFS. |
| ZTE | Support in general. Regarding which channels can be included in a dedicated resource pool, we support only include SL-PRS and PSCCH. As for the last bullet, we suggest to remove it since we have another Proposal 3.6 discussing whether feedback mechanism is needed for SL positioning. For a dedicated resource pool for SL positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS and PSCCH: - Do not include PSSCH. Only a single stage SCI is used for reservation. - PSCCH and SL-PRS are TDMed in a slot - Study further if PSFCH-like channel is needed |
| Fraunhofer | Support the proposal. |
| Lenovo | Support proposal, for dedicated pool PSCCH and SL-PRS are sufficient |
| Huawei, HiSilicon | Support. We believe for SL-PRS transmission and sensing for dedicated resource pool, only the resource reservation related information should be included in SCI. Hence, we do not see the necessity of introducing PSSCH to the SL-PRS dedicated resource pool. |
| LGE | We support the transmission of PSSCH carrying 2nd-stage SCI in a dedicated resource pool. It is aligned with the legacy SCI signaling method, which is used in a shared resource pool. Apart from the 2nd-stage SCI, we need to determine whether or not to support a standalone SL positioning with a dedicated resource pool only. In other words, whether an associated shared resource pool is always needed for SL PRS transmission in a dedicated resource pool. This topic is independent from the first topic above, and needs to be determined to finalize the slot structure. We prefer a standalone operation in a dedicated resource pool. Not to reduce the available SL PRS symbols in a slot, PSSCH carrying SL positioning data and SL PRS need to be TDMed in a slot basis. We think that the same kind of slot-based TDM can also be adopted in a shared resource pool. We support PSFCH transmission in a dedicate resource pool for feedback-based SL PRS retransmission. |
| Continental Automotive | We do not support the proposal in its current form. Instead we support the following alternative proposal indicated in the preceding summary: - PSSCH associated with SL-PRS transmission(s) is also included such that: - Alt. 2: 2nd stage SCI and SL-positioning-related information is included in the PSSCH We feel that if there is only PSCCH and SL PRS, other data related to positioning (e.g., measurements) would need to be transmitted over a separate SL communication channel in a SL communication or shared RP, which introduces unnecessary complexity. E.g., if there is a UE which needs to use the SL only for positioning, ideally we expect that it would need to be allocated only a dedicated RP. With this proposal it seems that there should always be an associated SL communication RP, which appears redundant to us. We also support to study further if a PSFCH-like channel is needed. We feel including it would be useful for a receiver UE to give feedback to a transmitter UE in positioning techniques where a sequence of transmissions need to take place (e.g., RTT-based). |
| Futurewei | Support |
| SONY | Do not support. Our preference is the legacy way of SCI signalling method (PSSCH carrying 2nd-stage SCI). We also support PSFCH-like transmission for SL-PRS retransmission. Basically we have similar view as LGE. |
| Apple | We are fine with the proposal. With this proposal, we are eliminating IUC scheme 1 and scheme 2 from consideration. |
| Nokia, NSB | Support. We think that PSFCH-like channel is beneficial. |
| Ericsson | Support. Regarding a feedback channel, we don’t think that is necessary. Retransmission can be handled over higher layers. |

Current status

- Support:

  - vivo, Sharp, Xiaomi, OPPO, Qualcomm, Interdigital, NEC, CMCC, Intel, ZTE, Fraunhofer, Lenovo, Huawei, HiSilicon, Futurewei, Apple, Nokia, NSB, Ericsson (17)

- Do not support (i.e. Add also PSSCH with 2nd stage SCI):

  - CATT, Panasonic, Spreadtrum, DCM, Samsung, LGE, Continental, Sony (8)

FL comment: There is significant majority with the current proposal (i.e. not including PSSCH in dedicated RP). Based on that, the plan is to try the following online:

[CLOSED] Feature Lead Proposal 3.2.2-v1

For a dedicated resource pool for SL positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS and PSCCH:

- Do not include PSSCH. Only a single stage SCI is used for reservation.

  - PSCCH and associated SL-PRS are TDMed in a slot

    - [FFS whether PSCCH and associated SL-PRS can be FDMed in the OFDM symbol(s) for the PSCCH]

  - [Study further if PSFCH-like channel is needed]

Companies views

| LGE | We do not support FDM between PSCCH and SL PRS. We suggest to remove FFS point. Regarding PSFCH, we understand that the purpose is not same as in HARQ process for SL communication. But we still believe that a feedback can be useful to indicate the retransmission of SL PRS if the reception of SL PRS fails (e.g. due to half duplex), or do not meet the required quality of reception (e.g. in terms of positioning accuracy). We suggest to remove the bracket on last sub-bullet. |
| --- | --- |
| vivo | Support and prefer to remove the FFS and last bullet, the feedback(e.g MR) can be included in high layer, and retransmission or latency bound can reduce the riskiness of fail reception. |
| Ericsson | Support. We also agree with LGE regarding FDM of PSCCH and SL PRS. For the last bullet, we think it can be excluded from the proposal and if needed, we can discuss lower-layer feedback separately (we do not support it, by the way). |
| OPPO | We support the proposal, including the FFS and the last sub-bullet. Typically, the bandwith of PSCCH would be much smaller than SL PRS, if FDM between PSCCH and SL PRS is not allowed, either AGC or transient period may be needed, while quite a few PRBs in the OFDM symbols for PSCCH are left unused. Similar issue was also discussed in Rel-16 SL, which leads to the current SL slot structure. Hence we propse to remove the brackets around the FFS. In SL there would be much more resource collision, transmission dropping, and reception dropping(e.g., due to half duplex) than Uu, we see the necessity to further consider about feedback based SL PRS retransmission. |
| Feature Lead | Companies are encouraged to see the status and are kindly asked to reconsider if they can be flexible: Current status - Support: - vivo, Sharp, Xiaomi, OPPO, Qualcomm, Interdigital, NEC, CMCC, Intel, ZTE, Fraunhofer, Lenovo, Huawei, HiSilicon, Futurewei, Apple, Nokia, NSB, Ericsson (17) - Do not support (i.e. Add also PSSCH with 2nd stage SCI): - CATT, Panasonic, Spreadtrum, DCM, Samsung, LGE, Continental, Sony (8) The main discussion is not about the “FFS” points of this agreement. The focal point is whether the PSSCH with 2nd stage SCI is needed or not. As a feature lead, i note that i believe that technically both solutions can work, however there has been a persistent majority of going with the current proposal and i kindly ask the other companies to reconsider. |
| CATT | Do not support. In our view , PSCCH and PSSCH which carries SCI should be included if the two stages SCI for SL-PRS were agreed. We slightly prefer two stages SCI, in order to keep the same SCI design with the legacy Rel-16 V2X and keep the same design between dedicated RP and shared RP. |
| Continental Automotive | We are OK with not having 2nd-stage SCI. However, it is not clear why PSSCH or a SL positioning data channel should not be allowed in the dedicated RP. A data channel could be used in addition to PSCCH to transmit data related to positioning in the dedicated RP (e.g., measurement reports). For the first bullet we would like to add: - PSCCH and associated SL-PRS are TDMed in a slot - [FFS whether PSCCH and associated SL-PRS can be FDMed in the OFDM symbol(s) for the PSCCH] - FFS: need for PSSCH or new data channel in addition to PSCCH We also prefer to remove the brackets around the 2nd bullet. |
| CMCC | Support in general, we don’t support PSCCH and its associated SL PRS to be FDM nor including PSSCH, but the updated proposal can be a compromise at this stage. To CATT: During SI, the following agreement was made, the note clearly says that same design for shared RP and dedicated RP are not implied: Agreement With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility In sidelink, 1st stage SCI carriers necessary information for sensing and 2nd stage SCI carriers necessary information for Rx UE to decode data, for SL PRS, the necessary information for UE to detect the sequence is much less that that to decode data. In addition, as proposed by many companies, some SL PRS information such as BW, comb size, number of symbols, etc, can be pre-configured in a RP level, and in this sense, the information that are carried in SCI is quite limited, the payload of 1st stage SCI is enough. |
| Samsung | Even though our 1st preference is two stage SCI, we can accept the proposal if two brackets are deleated. In this release, we want to minimize the issues for designing the dedicated resource pool for SL positioning. |
| ZTE | Support in general. We prefer to remove two sentences in brackets but for the sake of progress, we are ok to leave them as FFS and agree with the main part: Only single stage SCI is introduced and not include PSSCH in a dedicated resource pool. |
| Fraunhofer | Support the FL proposal |
| CEWiT | We do not support this proposal. Further, PSSCH is important to support over dedicated resource pool. First thing, it will mandate to have two type of resource pool configuration should be closely associated because the positioning configuration messages will come on shared channel where as SL-PRS will come may come on dedicated channel. Second, It will be wastage of resources and resource fragmentation as may be number of symbols used for SL_PRS will be less that the configured resource pool. As an illustration lets say we deployed RSU dedicated for positioning purpose and V2X UE doing positioning using RSU, why do we need shared resource to be configured for RSU for positioning as well. We can use dediated resource for this purpose solely. Therefore we adavocated to have dedicated resource pool self sufficient. |
| NEC | Support in principle, same as the similar views of the others companies, we also think the bracket on the last sub-bullet should be removed. |
| Intel | Support the proposal. In our understanding the additional overhead associated with including PSFCH doesnot justify the benefit of “HARQ like” operation for SL-PRS. In addition HARQ like operation for positioning reference signals has never been studied before, thus we would like to invest the valuable discussion time into other topics. Therefore, we desire to remove the FFS on PSFCH. |
| Toyota | Support FL’s proposal. |
| Qualcomm | We support the proposal. We also share the preference to remove the two FFS point. |
| Nokia, NSB | Support the proposal including FFS bullet and study PSFCH-like channel bullet. |
| Sharp | We are fine with the proposal. We do not support FDM between PSCCH and SL-PRS, but are fine with keeping the FFS for progress. Fine also with keeping the last sub-bullet. |
| InterDigital | We support the proposal. We do not support FDM-based multiplexing between PSCCH and the associated SL-PRS and therefore we suggest to remove this FFS. We do see a benefit to include feedback information for SL-PRS transmission, particularly in Scheme 2. SL-PRS RA Scheme 2 is based on Mode 2 SL communication sensing and thus is also subject the issues of hidden node, half-duplex, etc. Mechanisms such as HARQ and IUC are available to address the issues and improve PSSCH/PSCCH transmission reliability and thus we think a PSFCH-like channel to request re-transmision and/or provide SL-PRS reception feedback should be considered. For a dedicated resource pool for SL positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS and PSCCH: - Do not include PSSCH. Only a single stage SCI is used for reservation. - PSCCH and associated SL-PRS are TDMed in a slot - [FFS whether PSCCH and associated SL-PRS can be FDMed in the OFDM symbol(s) for the PSCCH] FFS [Study further if PSFCH-like channel, e.g., to request SL-PRS re-transmission is needed] |
| Lenovo | Support FL’s proposal, we tend to agree with other companies that FFS bullets may not be needed. |

Support the proposal:

- Vivo, Ericsson, OPPO, CMCC, Samsung (compromised with the bullets removed), Fraunhofer, NEC, Intel, Toyota, Qualcomm, Nokia, NSB, Sharp, Interdigital, Lenovo

Do not support the proposal:

- LGE, CATT (“slight prefer another option”), Continental, CEWiT

FFS on the FDM of PSCCH SL-PRS

- Support: OPPO, Nokia, NSB

- Remove: Samsung, ZTE, LGE, vivo, Ericsson, Qualcomm, Sharp, interdigital, Lenovo

Study bullet on PSFCH

- Support: OPPO, Nokia, NSB

- OK to keep: Sharp

- Remove: Vivo, Ericsson, NEC, ZTE, Samsung, Intel, Qualcomm, Lenovo

### 3.2.3 SCI for SL-PRS

Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

| Huawei, HiSilicon | Proposal 3: In the dedicated resource pool, support single stage SCI for resource reservation/indication and reception for SL-PRS. |
| --- | --- |
| vivo | - For the SCI used for the reservation/indication of SL-PRS, the following information should be carried in the SCI. - SL PRS timing/frequency resource, destination ID, source ID, period indication, priority, RE offset, and starting symbol. - In the dedicated resource pool, the following options can be taken into consideration in PSCCH design - Option 1. New PSCCH includes destination ID and source ID at least; - Option 2. 1st PSCCH and 2nd PSCCH is defined, and the 2nd PSCCH carries the destination ID and source ID. |
| Spreadtrum | Proposal 5: A new single-stage SCI in PSCCH is introduced for SL PRS transmission in dedicated resource pool. |
| CATT, GOHIGH | Proposal 16: For a dedicated resource pool for positioning, with regards to which channels can be included in the resource pool in addition to SL-PRS: - Option 2 (PSCCH which carries SCI is included) should be adopted if the single stage SCI for SL-PRS was agreed. - Option 3 (PSCCH which carries SCI and PSSCH are included) should be adopted if the two stages SCI for SL-PRS were agreed. Proposal 17: Whether PSFCH could also be included will be decided according to the IUC design. |
| Panasonic | Proposal 7: The reservation could be with both 1st stage SCI and 2nd stage SCI - Proposal 8: A unified reservation scheme should be designed for both aperiodic and periodic transmissions |
| LGE | Proposal 1: The legacy 2-stage SCI format is reused for SCI associated with SL PRS in both a dedicated and a shared resource pool. Proposal 11: The location of PSCCH and PSSCH for SCI transmission is determined based on TDM index and/or comb RE offset of the associated SL PRS resource within a slot in a dedicated resource pool. |
| Sharp | In a dedicated resource pool for positioning there is no need to support PSSCH transmission, and hence no need to reuse the 2-stage SCI design. Therefore, we support Alt. 1. Proposal 4: In a dedicated resource pool for positioning, a new single-stage SCI in PSCCH is introduced.c |
| Samsung | Proposal 2: For a dedicated resource pool for positioning, PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included. - Only 2nd SCI is transmitted in PSSCH. Proposal 10: Two stage SCI is supported for SL PRS resource(s) indication/reservation - FFS: details contents |
| lenovo | Proposal 12: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc. |
| ZTE | Proposal 7: In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS: - A new single-stage SCI in PSCCH is introduced - Information carried in SCI for SL-PRS should at least include: SL-PRS priority, source ID, destination ID, cast type, resource reservation period, SL-PRS configuration information. Regarding the SL-PRS configuration information, down-select between the following two options: - SCI indicates SL-PRS resource ID: this field can implicitly indicate number of symbols, comb size and RE offset, symbol offset of SL-PRS, bandwidth, SL-PRS sequence ID - SCI indicates time and frequency resource assignment: apply similar design as the SCI 1-A in SL communicationc |
| CEWiT | Proposal 11: In the dedicated SL resource pool, dedicated SCI format in PSCCH should be defined for SL-PRS configuration, indication and/or triggering. Only new first stage SCI can be used for SL-PRS configuration in dedicated resource pool. Proposal 12: For Dedicated SL resource configuration for SL positioning, reporting of the measurement can be triggered by TX UE in the first stage SCI. Resource reservation mechanism of first stage SCI (e.g. SCI 1-A) can be used to decide the measurement reporting resources using resource reservation field from stage 1 SCI and providing PRS report trigger field as additional signal using reserved bit. |
| Fraunhofcer | Proposal 1: For the dedicated resource pool for positioning a new single-stage SCI is introduced. Proposal 2: For the dedicated resource pool the number of symbols assigned to PSCCH shall be kept to minimum in order to provide more capacity for the SL-PRS. Proposal 3: For the OFDM symbols including the PSCCH are allocated in a single subchannel, whereas for the SL-PRS several subchannels can be allocated. |
| Interdigital | Proposal 14: Support a new SCI-1 format including resource reservation information of associated SL-PRS transmissions in a dedicated resource pool. |
| Apple | Proposal 3: Single Stage SCI should be supported for PSSCH signaling for SL-PRS. |
| Ericsson | - In the dedicated resource pool, we support having only single-stage SCIs. - SCI for SL PRS in dedicated resource pool is used to assist sensing and includes at least time domain resource allocation and resource reservation periods. - FFS: additional fields, including priority, frequency domain resource allocation and SL PRS resource indication |
| Qualcomm | Proposal 18: In a dedicated resource pool, a new SCI-1 format is introduced and includes SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index. |
| Mediatek | Proposal 3-1: A single SCI could be considered for SL-PRS allocation in dedicated resource pool |

With regards to the single-stage or single/two-stage SCI, the following is observed from the contributions

| Single stage SCI | Huawei, HiSilicon, Spreadtrum, Sharp, ZTE, CEWiT, Fraunhofer, Interdigital, Ericsson, Qualcomm, Mediatek |
| --- | --- |
| 2-stage SCI | Panasonic, LGE, Samsung, CATT, GOHIGH |

FL Comment: Note that this has been merged in the previous Section.

### 3.2.4 Mapping Relationship b/w PSCCH and SL-PRS resources

| CMCC | Proposal 2: Mapping relationship b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning. - PSCCH-PSSCH mapping relationship can be a reference; - Comb-based multiplexing of SL PRS should be considered when designing the mapping relationship; TDM-based multiplexing of SL PRS should also be considered when designing the mapping relationship. |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: Support to introduce an association scheme between SL-PRS resources with PSCCH resources for the dedicated resource pool. Proposal 6: Support to indicate the associated PSCCH resources as part of the SL-PRS resources (pre-)configuration for the dedicated resource pool. |
| Xiaomi | Proposal 6: A mapping relationship is defined between resource of PSCCH and the associated SL PRS resource in a dedicated resource pool. |
| CATT, GOHIGH | Proposal 15: For the mapping rule between PSCCH and SL-PRS in dedicated resource pool, the method of determination for PSFCH based on associated PSSCH can be as a starting point. |
| vivo | - For the support of the multiple SL-PRS transmitted in a slot, the following parameters should be defined to determine the resource of SL-PRS. - The RE offset, PRB/subchannel offset, and/or starting symbol. - The parameters can be determined based on the following options: - Option 1. Based on the pre-defined mapping rule of PSCCH and SL-PRS; - Option 2. Indicated in the PSCCH. |
| ZTE | Proposal 6: In order to avoid SL-PRS collision, support the mapping between each SCI and each SL-PRS configuration for a dedicated resource pool, where: - Frequency resource occupied by SCI is limited to a range of bandwidth of dedicated resource pool and multiple SCIs can be multiplexed in FDM manner. - The mapping relationship of “SCI location and SL-PRS configuration” can be configured in each dedicated resource pool. - SCI location refers to frequency range of SCI (e.g. starting PRB of SCI, PRB number of SCI) - SL-PRS configuration refers to comb offset or symbol offset. |
| Spreadtrum | Proposal 12: The association between PSCCH frequency domain monitoring ranges and SL PRS in a slot should be supported for SL PRS resource selection. |
| LGE | Proposal 11: The location of PSCCH and PSSCH for SCI transmission is determined based on TDM index and/or comb RE offset of the associated SL PRS resource within a slot in a dedicated resource pool. |

Several companies propose to have a mapping relationship specified between PSCCh and SL-PRS to avoid SL-PRS collisions and reducing the PSCCH monitoring range. Lets see if the proposal below can be acceptable:

[CLOSED] Feature Lead Proposal 3.2.4-v0

For a dedicated resource pool for SL positioning, support a mapping relationship between PSCCH and SL-PRS in a slot

- FFS: Details

Companies views

| CATT | Support. In addition, for the detailed mapping rule, we prefer considering to reuse the mapping method of determination for PSFCH based on associated PSSCH. |
| --- | --- |
| vivo | Support. If a UE decodes a PSCCH, the UE can determine the resource of SL-PRS(e.g., RE offset, starting symbol, PRB/subchannel offset) based on the location of the PSCCH, and no additional bits need to be carried in the SCI. |
| Sharp | We support the proposal. |
| Xiaomi | Support |
| InterDigital | We support the proposal. We prefer to spell out that the mapping is specific to the resource for a PSCCH and the resource of its associated SL-PRS. The PSCCH resource can be a frequency allocation (e.g. RB/sub-channel) and a SL-PRS resource can be a combination of SL-PRS BW, comb pattern (RE offset/N/M) and start symbol. Thus we propose the following modification. For a dedicated resource pool for SL positioning, support a mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a slot - FFS: Details |
| NEC | Support. At least for agreed TDM-based and comb-based SL PRS multiplexing within a slot, a mapping relationship between PSCCH and SL PRS can be benefical to the detection of SL PRS. |
| Panasonic | Support |
| Spreadtrum | Support |
| DCM | OK |
| CMCC | Support. |
| Intel | In our understanding this mapping relation is dependent on the decision on the SL-PRS resource granularity. Thus, it is at this stage not clear if for a 1:1 mapping there are sufficient amount of candidates. Therefore, we would like to postpone this discussion till the SL-PRS resource granularity in a dedicated resource pool reached an agreement. |
| ZTE | Support, in detail, multiple pairs of “mapping between SCI location and PRS configuration” can be configured in a dedicated resource pool. For a dedicated resource pool for SL positioning, support a mapping relationship between PSCCH and SL-PRS in a slot - Multiple pairs of “mapping between SCI location and PRS configuration” can be configured in a dedicated resource pool - FFS: Details |
| Fraunhofe | Support |
| Lenovo | Supportive the proposal in principle |
| Huawei, HiSilicon | Support. We think the mapping relationship is needed to simplify the resource allocation. |
| Futurewei | Support |
| Apple | We are fine with this as it is needed to support a single Scheme 2 resource allocation for both channels. |
| Nokia, NSB | Support |
| Ericsson | We are not sure we understand the meaning of mapping relationship. If it means that the location of the SCI infers the PRS resource mapping, we agree with intel that it may conflict with support of comb based muxing or FDM. |

Supported by: CATT, vivo, Sharp, Xiaomi, Interdigital, NEC, Panasonic, Spredtrum, DCM, CMCC, Fraunhofer, Lenovo, Huawei, HiSilicon, Futurewei, Apple, Nokia, NSB

Postpone the discussion: Intel, Ericsson

[CLOSED] Feature Lead Proposal 3.2.4-v1

For a dedicated resource pool for SL positioning, support a mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a slot

- FFS: Details

Companies views

| LGE | Support |
| --- | --- |
| OPPO | OK |
| CATT | Support In addition, for the detailed mapping rule, we prefer considering to reuse the mapping method of determination for PSFCH based on associated PSSCH. |
| Continental Automotive | OK with the proposal. |
| CMCC | Support |
| Xiaomi | support |
| Samsung | We share the view with Intel and Ericsson (Postpone the discussion) |
| ZTE | Support. |
| Fraunhofer | Support |
| Spreadtrum | Support |
| NEC | Support. |
| Intel | As there are multiple topics that have an influence on this proposal we would still like to wait till these are clear. To be clear we are not negative regarding the solution in this proposal. |
| Qualcomm | We are ok with the proposal. We are also ok with postponing the discussion until other agreements have been made. |
| Nokia, NSB | Support |
| Sharp | We support the proposal. |
| InterDigital | We support the proposal. |
| Lenovo | Supportive of FL’s proposal |

### 3.2.5 Fields in the SCI

| Huawei, HiSilicon | Proposal 4: In the dedicated resource pool, SCI for SL-PRS should include following required fields: - Priority; - Time resource assignment; - SL-PRS resource indication; - Resource reservation period; - Source ID; - Destination ID. |
| --- | --- |
| vivo | - For the SCI used for the reservation/indication of SL-PRS, the following information should be carried in the SCI. - SL PRS timing/frequency resource, destination ID, source ID, period indication, priority, RE offset, and starting symbol. |
| Spreadtrum | Proposal 6: The contents of the new single-stage SCI should include time/trequency resource assignment; resource reservation period, SL-PRS resource indication, priority, source ID, destination ID. |
| CATT, GOHIGH | Regarding the contents of the SCI, the information transmitted by legacy 1st-stage SCI and 2nd-stage SCI defined in Rel-16/17 V2X can be a starting point. At least the following information should be considered: - Priority - Frequency resource assignment - Time resource assignment - Resource Reservation period - SL-PRS pattern - Source ID - Destination ID - Cast type indication - SL-PRS request/indication |
| lenovo | Proposal 12: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc. |
| ZTE | Proposal 7: In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS: - A new single-stage SCI in PSCCH is introduced - Information carried in SCI for SL-PRS should at least include: SL-PRS priority, source ID, destination ID, cast type, resource reservation period, SL-PRS configuration information. Regarding the SL-PRS configuration information, down-select between the following two options: - SCI indicates SL-PRS resource ID: this field can implicitly indicate number of symbols, comb size and RE offset, symbol offset of SL-PRS, bandwidth, SL-PRS sequence ID - SCI indicates time and frequency resource assignment: apply similar design as the SCI 1-A in SL communicationc |
| Ericsson | - SCI for SL PRS in dedicated resource pool is used to assist sensing and includes at least time domain resource allocation and resource reservation periods. - FFS: additional fields, including priority, frequency domain resource allocation and SL PRS resource indication |
| Qualcomm | Proposal 18: In a dedicated resource pool, a new SCI-1 format is introduced and includes SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index. |

We observe the following fields are being proposed in a few papers:

| Fields in the SCI | Supported based on the contributions |
| --- | --- |
| - Source ID - Destination ID | Huawei, HiSilicon, vivo, ZTE, CATT, GOHIGH, spreadtrum |
| - SL-PRS Priority | Huawei, HiSilicon, vivo, ZTE, spreadtrum |
| - Time resource assignment (e.g. starting symbol) | Huawei, HiSilicon, vivo, Ericsson |
| - Frequency resource assignment | Lenovo, CATT, GOHIGH, Ericsson |
| - SL-PRS resource indication | Huawei, HiSilicon, CATT, GOHIGH, spreadtrum |
| - Resource reservation period | Huawei, HiSilicon, vivo, ZTE, Qualcomm |
| - RE offset | Vivo |
| - Reservation Indication | Lenovo |
| - Cast type | ZTE, CATT, GOHIGH |
| - SL-PRS configuration / pattern | CATT, GOHIGH |
| - SL-PRS request | CATT, GOHIGH |
| - Positioning Session ID | Qualcomm |

[CLOSED] Feature Lead Proposal 3.2.5-v0

In the dedicated resource pool, SCI for SL-PRS should at least include the following fields:

- Source ID

- Destination ID

- Resource reservation period

- SL-PRS Priority

- Cast type

- SL-PRS configuration and/or time-frequency assignment information:

  - Consider further the following options (or combination of them):

    - Opt. 1: SCI includes a SL-PRS resource ID that indicates one of the SL-PRS resource(s) that have been provided to the UE with higher layer signaling

    - Opt. 2: Explicit field(s) for the indicated SL-PRS pattern & assignment

      - E.g., number of symbols, comb size and RE offset, sequence ID, Time-domain resource assignment, Frequency-domain resource assignment

- FFS: SL-PRS request

- FFS: Positioning Session ID

- FFS: Additional fields

Companies views

| CATT | Support. For the SL-PRS configuration, we prefer both Opt.1 and Opt.2 should be included. |
| --- | --- |
| vivo | Firstly, we have some concerns about cast type, in SL communication, the cast type is needed because of different HARQ mechanisms. Could the proponent provide more information on why cast type needs to be indicated in SCI Secondly, SL-PRS configuration and/or time-frequency assignment information should be divided into two sub-bullet. For example, time assignment information can be used to indicate up to 3 slots for SL transmission. SL-PRS configuration is used to indicate the SL PRS pattern in slot(E.g., number of symbols, comb size and RE offset,) In addition, SL-PRS configurations like RE offset, starting symbols and/or PRB/subchannel offset can be explicitly or implicitly determined, and it should be further studied. For example, if the mapping relationship between PSCCH and SL-PRS is pre-defined, these parameters can be implicitly obtained according to the location of PSCCH. If the mapping relationship is not pre-defined, some explicit field(s) need to be carried in the SCI. |
| Sharp | We are fine with the first 5 bullets and propose to put them (or, those with rough consensus) into a separate proposal. Regarding the 6th bullet, we propose to add one alternative of Opt. 1 that the SL-PRS resource ID is derived by the UE (i.e. similarly to a PSFCH resource index in a slot in Rel-16). The reason is that UEs other than the source and destination have to be able to correctly interpret the “SL-PRS resource ID” as well, for the purpose of sensing. |
| Qualcomm | Support. We prefer Option 2 to limit SCI size |
| InterDigital | We support the proposal. We prefer Option 1 as the SL-PRS resource (part or all of it) can be determined by a UE based on the detected PSCCH resource and the mapping discussed in question 3.2.4. |
| NEC | We think maybe it’s better to decide the definination of SL PRS resource or SL PRS resource set firstly, and then the content of SCI scheduling SL PRS. |
| Panasonic | OK |
| Spreadtrum | For cast type, it need FFS. For SL-PRS configuration and/or time-frequency assignment information, we prefer Opt. 1. |
| DCM | Proposal 3.2.2 should be concluded first. |
| CMCC | We are generally fine with the direction with the following comments: Regarding cast type, we are wondering why it is needed in the SCI. Different with SL, where different cast types have different HARQ feedback mechanisms, but for SL PRS, we don’t think such field is needed. Regarding the SL PRS configuration, we share similar views as vivo that some SL PRS configuration parameters such as BW, comb size, number of symbols, etc. can be per resource pool level pre-configured and no need to be indicated by SCI. In addition, regarding Option 1 that SCI to indicate SL PRS resource ID, I think whether a SL PRS resource ID is defined/needed is still open for now and is under AI 9.5.1.1 for discussion. |
| Intel | In addition to the fields we think 24-bit CRC should be added. For the time-frequency resource assignment in our understanding this depends also on the discussion under section 3.2.4 in this document. Thus, at this stage the options for the time-frequency resource assignment should not be limited to these options and we would like to remove these. |
| ZTE | Regarding the cast type indicator, we think it is necessary to include it in SCI. According to the agreement made in RAN1#112, SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. SCI which can contain cast type and source/destination UE information can be seen as physical filtering (similar as SCI in ), for example: - If the cast type indicated in Tx UE’s SCI is unicast, Rx UE should determine if its source ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI. - If the cast type indicated in Tx UE’s SCI is groupcast or broadcast, Rx UE should determine if its destination ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI. Among Opt.1 and Opt.2, it is hard to down-select without any details agreed for Proposal 3.2.4. If the mapping relationship is between PSCCH’s location and certain SL PRS configuration/patterns, then no explicit fields are needed. For the 1st FFS, we prefer to de-prioritize including SL-PRS request in SCI. We should wait for the outcome of Proposal 5.1. For the 2nd FFS, we do not support and suggest to delete it. Clarifications are needed regarding the definition of positioning session and why we need this field in SCI, we failed to find any definitions in RAN1’s spec. |
| Lenovo | Support with Option 2 for SL-PRS configuration |
| Futurewei | Support. Isn’t the cast type and destination ID redundant ? We do not see it necessary. |
| Apple | We are fine with the proposal with Option 2. |
| Nokia, NSB | Support |
| Ericsson | OK in principle, but we would like to keep the details of - SL-PRS configuration and/or time-frequency assignment information FFS. We can discuss the option later. Regarding the FFS, we prefer removing all except the FFS on additional fields. |
| LGE | Regarding Opt 1 and Opt 2, we think that SCI should contain the index (e.g. SL PRS configuration ID) that indicates at least comb size and the number of symbols. We don’t think every configuration field needs to be included in SCI. We suggest a new option. - Opt. 2: SCI includes - SL-PRS configuration ID that indicates the static parameters including at least comb size and number of symbols - Dynamic parameters including at least comb RE offset and starting SL PRS symbol In addition, comb RE offset and starting symbol index, which can be determined after sensing in scheme 2, should be included in SCI We do’t think that positiong session ID needs to be included in SCI. We’re ok with other fields. |

Support proposal:

- CATT, Nokia, NSB, Apple, Ericsson

Uclear about the need of cast type

- Spreadtrum, vivo, CMCC, Futurewei

A few companies suggested to remove the details on the PRS config and time/Frequency assignment. Lets continue with the following:

[CLOSED] Feature Lead Proposal 3.2.5-v1

In the dedicated resource pool, SCI for SL-PRS should at least include the following fields:

- Source ID

- Destination ID

- Resource reservation period

- SL-PRS Priority

- FFS: Cast type

- SL-PRS configuration and/or time-frequency assignment information:

  - FFS: Details

  - Consider further the following options (or combination of them):

    - Opt. 1: SCI includes a SL-PRS resource ID that indicates one of the SL-PRS resource(s) that have been provided to the UE with higher layer signaling

    - Opt. 2: Explicit field(s) for the indicated SL-PRS pattern & assignment

      - E.g., number of symbols, comb size and RE offset, sequence ID, Time-domain resource assignment, Frequency-domain resource assignment

- 24-bit CRC Field

- FFS: SL-PRS request

- FFS: Positioning Session ID

- FFS: Additional fields

Companies views

| Feature Lead | Please comment whether the “cast type” is needed or not with a technical justification |
| --- | --- |
| OPPO | OK. Cast type is needed, because the destination ID space for different cast types may not be clearly separated. |
| CATT | Cast type indicator in the SCI can be used to indicate the Unicast, Groupcast and Broadcast of SL-PRS transmission even without the HARQ mechanism, just like the Rel-16 V2X. We are open to discuss whether the destination ID can be used for this purpose. At this stage, we prefer to keep the cast type indicator in the proposal. Table 8.4.1.1-1: Cast type indicator Value of Cast type indicator; Cast type. 00; Broadcast. 01; Groupcast when HARQ-ACK information includes ACK or NACK. 10; Unicast. 11; Groupcast when HARQ-ACK information includes only NACK. |
| LGE | We support in general with some comments. We prefer SCI to carry full source and destination ID because there can be no MAC-PDU transmission together with SL PRS in the same slot. This minimizes the ambiguity due to partial ID in UE identification. Regarding FFS, there were same discussion in SL communication, and the conclusion was that cast type is needed because destination ID is not sufficient for cast type differentiation. We remember it was RAN2 decision. So we suggest to follow the legacy operation in sidelink. It’s not still clear why postioning seesion ID in higher layer should be conveyed by SCI in physical layer. We suggest to delete the FFS. - 24 bits Source ID - 24 bits Destination ID - FFS: Cast type - FFS: Positioning Session ID |
| Continental Automotive | Cast type should be included since it has been already agreed to support unicast/groupcast for SL PRS (with broadcast as working assumption) and for receiving UEs, knowing the cast type would help to determine whether the SL PRS is meant for them or not. |
| CMCC | Support. We don’t think cast type is needed, OK to leave it as FFS for now. |
| xiaomi | The reason why “cast type” is introduced for SL communication is because that HARQ-ACK for unicast, groupcast and broadcast are different. The UE PHY layer needs to know the cast type to know how to preform HARQ-ACK. However, we are not sure what UE PHY layer behavior will be different according to different cast types. So we think it is still early to remove FFS for cast type. Certainly if we can identify different PHY behavior of different cast type, we can agree to add cast type then. |
| ZTE | Comment 1: We support including “cast type” in SCI. Referring to legacy SCI design in SL communication, we think cast type was not just introduced for ACK/NACK feedback. Different cast type will use different “filtering” mechanism (unicast is different from groupcast and broadcast). SCI which can contain cast type and source/destination UE information can be seen as physical filtering (similar as SCI in ), for example: - If the cast type indicated in Tx UE’s SCI is unicast, Rx UE should determine if its source ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI. - If the cast type indicated in Tx UE’s SCI is groupcast or broadcast, Rx UE should determine if its destination ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI. Moreover, to echo LGE’s comment, we also open to discuss 24 bits source/destination ID. 38.321 3> if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or 3> if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:. Comment 2: - FFS: SL-PRS request We do not see clear benefit of introducing lower layer SL-PRS request. Anyway we are ok with FFS. Comment 3: - FFS: Positioning Session ID Clarifications are needed regarding the definition of positioning session and why we need this field in SCI, we failed to find any definitions in RAN1’s spec. Again we can compromise for it as FFS. |
| Huawei, HiSilicon | Why is CRC field explicitly listed? It should be generically appended based on the payloads after padding, prior to Polar encoding, right? CRC should not be counted as the SCI content. The reasoning is not clear from Intel’s comment from the last round. |
| Spreadtrum | Support the proposal. |
| Ericsson | Agree with Huawei’s comment that CRC is not part of SCI, but appended to the payload prior to encoding and modulation. On the FFS for PRS request and positioning session ID: - We can discuss PRS request separately. - We don’t think the session ID is needed for lower layer signaling, it will be in the higher layers messages. |
| Intel | We support including cast type. However, we would like to clarify that connectionless groupcast is not standardized for SL-PRS as in our understanding this is mainly beneficial for HARQ operation, which is not present for SL-PRS. |
| Qualcomm | We agree with the comments that cast type is needed. Destination ID is insufficient to determine cast type. This has been discussed multiple times in RAN1 and the conclusion has always been that cast type indication is needed as reflected in the SCI formats. We would like to revise the wording from “include the following fields” to “indicate the following values”. Some indirect or joint signaling might be needed to keep the SCI size small. |
| Nokia, NSB | Agree with Huawei on CRC field.. Regarding FFS on cast type, we agree with LGE’s reasoning why “FFS” should be removed, cast type is needed. Otherwise, OK in general. Agree with QC’s comment on “indicate” instead of “include” |
| Sharp | Agree with other companies that cast type is needed. We also don’t think positioning session ID is needed but are fine to keep it as FFS for progress. |
| InterDigital | We support the proposal. |
| Lenovo | Supportive in general and agree that cast type is needed in SCI and ok FFS details on the PRS configuration. Also not clear on the the motivation behind 24-bit CRC indication in SCI |

### 3.2.6 Bandwidth of the SL-PRS Transmission

The following agreement was reached with regards to this topic:

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 2: For both dedicated and shared resource pools for SL positioning, the bandwidth of SL-PRS shall be (pre)-configurable, e.g., depending on positioning accuracy requirements, which can be same or smaller than that of the resource pool. |
| --- | --- |
| Spreadtrum | Proposal 2: The bandwidth of PRS can be configured at the resource pool level or can be fixed to the bandwidth of dedicated resource pool. |
| CATT, GOHIGH | Proposal 3: For a dedicated resource pool for sidelink positioning, the bandwidth of SL-PRS should be (pre-)configurable and it can be same or smaller than that of the resource pool in order to get a right balance between positioning accuracy and SL-PRS overhead. |
| Sony | Proposal 4: In both shared and dedicated resource pool, the bandwidth of SL-PRS can be configured, such that it can be the same as the sub-channel bandwidth within a resource pool. |
| ZTE | Proposal 3: For a dedicated resource pool for positioning, with regards to the bandwidth of SL-PRS transmission, support the following option: - Option (2): SL PRS bandwidth is up to higher layer configuration in a resource pool. - Option (2a): The bandwidth of SL-PRS can be configured per dedicated resource pool. In other words, the bandwidth of SL-PRS is the same as that of the resource pool.c |
| CEWiT | Proposal 13: For a dedicated resource pool for positioning, the bandwidth of SL-PRS transmission can be same or smaller than that of the resource pool. |
| Apple | Proposal 15: The frequency resources used for SL positioning can be set equal or smaller than the shared channel frequency domain allocation size for both the shared and dedicated resource pools. |
| Ericsson | - For dedicated pool m the bandwidth of SL-PRS can be smaller than the resource pool - FFS: The starting subchannel and number of subchannel occupied by the SL PRS resource is either provided in the SL PRS resource configuration or allocated when the resource is in use (via e.g. SCI or higher layer signalling). |
| NEC | Proposal 5: The bandwidth of SL-PRS can be same or smaller than that of the dedicated resource pool for positioning. |
| Asustek | Proposal 1: For dedicated resource pool, bandwidth of SL-PRS can be same or smaller than that of the resource pool. |
| Mediatek | Proposal 2-2: Within the dedicated resource pool, the SL-PRS transmission BW is expected to be equal to that of the dedicated resource pool. If not, the resource pool’s bandwidth could be reduced Proposal 2-3: Within the dedicated resource pool, the measurement bandwidth of a RX UE is up to the UE capability |

We make the following observations:

| Alt. 1 (same or smaller than that of the RP) | Nokia, NSB, spreadtrum, CATT, GOHIGH, Sony, CEWiT, Apple, Ericsson, NEC |
| --- | --- |
| Alt. 2 (same as the BW of the RP) | ZTE, Mediatek |

[CLOSED] Feature Lead Proposal 3.2.6-v0

At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission:

  - The bandwidth of SL-PRS can be same or smaller than that of the resource pool

Companies views

| CATT | Support. Such configuraton can improve the flexibility of bandwidth configuration and get a right balance between positioning accuracy and SL-PRS overhead. |
| --- | --- |
| vivo | Support. |
| Sharp | We support the proposal. |
| Xiaomi | support |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | Support |
| Spreadtrum | Support |
| DCM | OK |
| CMCC | Same issue is under discussion of AI 9.5.1.1 and further coordination is needed to use which AI to treat this issue. We support the BW of SL PRS in dedicated RP to be the same as the BW of the pool, which guarantees higher positioning accuracy. Allowing the BW of SL PRS to be smaller than the BW of the pool may leave configuration flexibility of the SL PRS BW, however, it may scatter the resources and further reduce the resource utilization efficiency. |
| Intel | The bandwidth of the SL-PRS is the same as the bandwidth of the dedicated resource pool. |
| ZTE | We prefer to only have one bandwidth configured for a dedicated resource pool (similar as the bandwidth of DL PRS within a single PFL is the same). |
| Fraunhofer | Similar observation as CMCC. Support the proposal. |
| Lenovo | Prefer to be the same BW, but can live with proposal. |
| Huawei, HiSilicon | We prefer Alt. 2 (same as the BW of the RP). Since the issue is also discussed in 9.5.1.1, better to harmonize which agenda this issues should be discussed, e.g., in 9.5.1.1. |
| Toyota | Support. |
| Futurewei | Prefer Alt 2, i.e., the same BW as RP. Multiple RPs may be configured with different BW. |
| Apple | Fine with proposal |
| Nokia, NSB | Support |
| Ericsson | Support |

FL: Continue the discussion in 9.5.1.1

### 3.2.7 Resource allocation granularity

The following agreement was reached with regards to this topic:

Agreement Study further the granularity of time-domain resource allocation for SL-PRS transmission.

The following proposals were found in the contributions:

| vivo | - For the support of the multiple SL-PRS transmitted in a slot, the following parameters should be defined to determine the resource of SL-PRS. - The RE offset, PRB/subchannel offset, and/or starting symbol. - The parameters can be determined based on the following options: - Option 1. Based on the pre-defined mapping rule of PSCCH and SL-PRS; - Option 2. Indicated in the PSCCH. |
| --- | --- |
| CATT, GOHIGH | Proposal 6: SL-PRS resource or SL-PRS resource set should be the resource allocation unit in time-domain for SL-PRS transmission. - The definition of SL-PRS resource and SL-PRS resource set should be specified firstly. |
| Sony | Proposal 8: support slot- or sub-slot-based SL-PRS resource allocation |
| LGE | Proposal 13: SL PRS resource is the granularity of the SL PRS resource allocation. |
| Sharp | Proposal 5: In a dedicated resource pool for positioning, the minimum resource allocation granularity is one SL-PRS resource corresponding to one RE offset (of the first SL-PRS symbol) within a comb-based SL-PRS pattern. Proposal 6: In a dedicated resource pool for positioning, multiple granularities are supported in SL-PRS resource allocation. |
| ZTE | Proposal 8: With regards to the granularity of time-domain resource allocation for SL-PRS transmission - Support SL-PRS-resource-based allocation, i.e. M <=14 symbols with comb size N within a slot are configured for each SL-PRS resource |
| Interdigital | Proposal 3: (Pre)configured slot- or sub-slot-based SL-PRS resource allocation in a dedicated resource pool. |
| Ericsson | - The resolution of frequency domain allocation for the SL PRS is 1 subchannel. |
| Qualcomm | Proposal 3: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size and comb offset. |
| lenovo | Proposal 11: Further consider the SL PRS resource allocation granularity in terms of one or more sub-channels and/or slots/mini slots. |

[CLOSED] Feature Lead Proposal 3.2.7-v0

For a a dedicated resource pool,

- with regards to the time-domain resource allocation support a

- Alt. 1: slot-based allocation

  - FL Example (not part of the proposal): Allocate the whole slot, but eventually a SL-PRS resource may be transmitted in a subset of the symbols of the slot

- Alt. 2: sub-slot-based allocation

  - FL Example (not part of the proposal): Allocate a subset of the symbols of the slot (e.g. 7 symbols), but eventually a SL-PRS resource may be transmitted in a subset of those symbols

- Alt. 3: SL-PRS-resource-based allocation

  - FL Example (not part of the proposal):: The OFDM symbols that the SL-PRS resource occupies are being allocated

- with regards to the frequency-domain resource allocation granularity support allocating only the resource elements according to the SL-PRS resource’s RE-offsets for one or more contiguous sub-channels.

Companies views

| CATT | Support. We prefer Alt.3. In addition, if SL PRS resource set is also agreed to be defined, it can also be the Resource allocation unit. |
| --- | --- |
| vivo | Regards to the time-domain resource allocation, we support alt. 3 In our opinion, the number of SL-PRS should be configured by higher layer, and only single value of M can be configured. The OFDM symbols of SL-PRS transmitted in the resource pool should occupies the same symbols as M. It could reduce the complexity of resource allocation in the dedicated resource pool. Regards to the frequency-domain resource allocation granularity, we support allocating the resource elements based on the SL-PRS resource’s RE-offsets. |
| Sharp | We propose to support both Alt. 2 and Alt. 3, and we propose to add an Alt. 4 that the time-domain resource allocation granularity can be (pre-) configured between Alt. 2 and Alt. 3. In our view Alt. 1 can be a special case of Alt. 2. |
| xiaomi | Alt 3 is not very clear for us since the concept of SL PRS resource is still unclear. From our opinion, the time domain starting symbol position(s) and length of symbols of a SL PRS resource in a slot shall be (pre)-configured, and the resource allocation shall follow the (pre)configurations. If our understanding of alt 3 is correct, we prefer alt 3. |
| InterDigital | We support at least Alt 1 and Alt 2 (pre)configured in a resource pool. We think Alt 3 may have an AGC issue. If each UE is allowed to select SL-PRS resource with different M values and also SL-PRS resource selection allows FDMed SL-PRS transmissions by different UEs in the same slot, AGC issue can occur, e.g. when UE1 use the entire slot and UE2 and UE3 TDMed in the same slot and UE1 and UE2/3’ SL-PRS transmissions are FDMed. So we propose to study further Alt. 3. We support comb-patter-level (RE-level) multiplexing in frequency domain and thus we prefer the frequency domain resource granularity is based on RE-offset specific to a SL-PRS BW (one or more contiguous sub-channels) |
| NEC | We share the similar view as that of Sharp. Alt.2 seems to be a tradeoff between flexible and efficient resouce utilization, at least considering the alignment of comb-based multiplexing SL PRS from different UEs. |
| Panasonic | Alt 2 or 3 |
| Spreadtrum | We prefer Alt. 3. |
| DCM | Question for clarification: Alt 1/2 includes FDM or preclude this? If precluded, Alt 3 should be supported. If included, Alt 1 is better than Alt 2 for simplicity. |
| CMCC | We think that Alt. 3 includes both Alt. 1 and Alt. 2, as we have agreements on allowing TDM-based multiplexing of SL PRS from different UEs in a slot, which refers to a sub-slot resource allocation; otherwise, the baseline slot-bsaed resource allocation is used. |
| Intel | In our understanding coexistence of these alternatives is not possible for scheme-2 operation in the same dedicated resource pool due to sensing resource allocation purposes. In our understanding the last main bullet does also imply only sub-channel based allocation is possible. In our understanding there is not agreement on this yet. Therefore, we would like to modify the proposal to the following: ##### [MEDIUM] Feature Lead Proposal 3.2.7-v0 For a a dedicated resource pool, - with regards to the time-domain resource allocation support a - Alt. 1: slot-based allocation - FL Example (not part of the proposal): Allocate the whole slot, but eventually a SL-PRS resource may be transmitted in a subset of the symbols of the slot - Alt. 2: sub-slot-based allocation - FL Example (not part of the proposal): Allocate a subset of the symbols of the slot (e.g. 7 symbols), but eventually a SL-PRS resource may be transmitted in a subset of those symbols - Alt. 3: SL-PRS-resource-based allocation - FL Example (not part of the proposal):: The OFDM symbols that the SL-PRS resource occupies are being allocated - with regards to the frequency-domain resource allocation granularity support allocating only the resource elements according to the SL-PRS resource’s RE-offsets for one or more contiguous sub-channels. - For a dedicated resource pool with scheme-2 resource allocation only one of these alternatives is configured. |
| ZTE | For time-domain resource allocation, we support Alt.3. For frequency-domain resource allocation granularity, in our understanding, the first thing we should discuss is whether the concept of subchannel is still needed in dedicated SL PRS resource pool. Given the situation that maybe in a dedicated resource pool, all SL PRS resources have the same bandwidth, there is no need to introduce frequency domain assignment in SCI and also no need to introduce subchannel. |
| Lenovo | Support FL’s proposal with the listed Alts |
| Huawei, HiSilicon | Intention is still not clear, is it talking about resource pool configuration in time domain or SL-PRS reservation in time domain within the pool? |
| Futurewei | We may need to discuss the support of TDM and FDM in the dedicated RP first. Alt 3 may be a natural choice then. |
| Apple | Alt 2 or Alt 3 |
| Nokia, NSB | Support; prefer Alt 2 and Alt 3, but definition of SL-PRS in the SL-PRS AI may be needed first before this can be finalized. |
| Ericsson | Preference for alt2, or alt3. |
| LGE | We support Alt 3 for the 1st main bullet. The 2nd main bullet is not clear. We need further clarification on FL proposal. |

We observe the following:

- Alt. 1

  - Interdigital

- Alt. 2

  - Sharp, Panasonic, Apple, Interdigital, Ericsson

- Alt. 3

  - CATT, vivo, Sharp, Xiaomi, Panasonic, Spreadtrum, Apple, ZTE, CMCC, Ericsson

- Alt. 4 time-domain resource allocation granularity can be (pre-) configured between Alt. 2 and Alt. 3

  - Sharp

It was also pointed out that TDM of UEs has already been agreed and therefore Alt.1 would not be appropriate. It was also pointed out that one solution should be preconfigured in the resource pool. This is my understanding and we can try to clarify it by adding clearly: “with regards to the SL-PRS time-domain resource allocation within the resource pool”.

Based on the above, the proposal is updated as follows:

[CLOSED] Feature Lead Proposal 3.2.7-v1

For a a dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a (downselection needed):

- Alt. 1: slot-based allocation

- Alt. 2: sub-slot-based allocation

  - Note: This may include the cae of sub-slot equals a slot.

- Alt. 3: SL-PRS-resource-based allocation

- Alt. 4: time-domain resource allocation granularity can be (pre-)configured between Alt. 2 and Alt. 3

- with regards to the frequency-domain resource allocation granularity support allocating only the resource elements according to the SL-PRS resource’s RE-offsets for one or more contiguous sub-channels.

Companies views

| OPPO | The proposal is a bit confusing to us. In our understanding, in dedicated resource pool one or multiple TDMed SL PRS resources would be (pre-)configured, in time domain a SL PRS resource may occupy part of symbols (sub-slot) or all OFDM symbols of a slot, given that we have not seen the difference between Alt. 2 and Alt.3. |
| --- | --- |
| CATT | Support. We prefer Alt.3. |
| Feature Lead | To OPPO: My understanding is that in Alt. 2, the sub-slot may not be the same numbers of symbols as that of the SL-PRS resource (e.g., define a concept of a mini-slot/sub-slot within a slot and allocate that part; then a SL-PRS resource would be inside that portion of the slot). In Alt. 3, an actual SL-PRS resource will be allocated (e.g. if SL PRS is 4 symbols, then these 4 symbols are goin to be allocated). |
| LGE | Given clarification from FL above, we support Alt 3. Alt 2 will allow the transmission of incomplete SL PRS resource, which is not recommended. |
| Continental Automotive | We are OK with the proposal and would prefer Alt. 3. |
| CMCC | Support Alt. 3. |
| Xiaomi | We are fine with the proposal. |
| Samsung | We suggest to capture the different between Alt 2 and Alt 3 clearly in the proposal at first. |
| ZTE | Support Alt.3. |
| Huawei, HiSilicon | The proposal is confusing to us. Our understanding is - The resource partition in a slot (including start symbol index, symbol duration) is (pre)configured in the resource pool (RRC) or perhaps indicated by SLPP as commented by others. - When it comes to the resource allocation in PC5, i.e. using SCI, it should indicate the transmitted/reserved SL-PRS resource per slot. It appears that our understanding is more or less aligned with OPPO, which is why both of us are confused at the current wording. How about the following change For a a dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a (downselection needed): - Alt. 1: slot-based allocation - Alt. 2: sub-slot-based allocation - Note: This may include the cae of sub-slot equals a slot. - Alt. 3: SL-PRS-resource-based allocation - Note: An allocated SL-PRS resource may occupy a subslot or a slot. In addition, this proposal does not necessarily mean different UEs should use the same configuration of (M, N) in the same resource pool and using different (M, N) is still allowable? [FL response]: Yes this is not being discussed here. |
| Fraunhofer | Okay with the proposal. Prefer Alt.3 |
| Spreadtrum | We are fine with the proposal. We prefer Alt 3. |
| Ericsson | Considering the additional explanation, alt3 seems more appropriate in order not to over complicate the design – we do not want to break down a slot into subslots with separate control channels, for example. We are ok with adding a note clarifying that a resource may occupy only a fraction of the symbols in a slot with variable starting position and length, according to its configuration. |
| NEC | For Alt. 2, the definition of sub-slot/mini-slot is not clear, does it mean a slot may comprise multiple mini-slots? |
| Intel | The difference between Alt. 2 and Alt.3 is not clear to us. If for example SL-PRS resources are defined in terms of sub-slots they would be the same. Thus, probably it is best with finalizing this proposal until we have defined SL-PRS resources. |
| Qualcomm | Not support The starting symbol and the number of symbols used for all SL-PRS overlapping in time have to be the same across all UEs. Sidelink does not support dynamic transmission duration because that would not work with the UE-autonomous resource allocation. |
| Nokia, NSB | Support. We prefer Alt 2 and Alt 3, but as noted before the definition of SL-PRS resource in the SL-PRS AI may be needed first. |
| Sharp | We prefer Alt. 4, but could be fine with Alt.2 or Alt. 3 if supported by majority. |
| InterDigital | We support the proposal. We prefer Alt.2 or Alt. 4. |
| Lenovo | Lenovo, Also support Alt.2 and Alt.3 , but may have dependence on parallel discussion on SL-PRS resource granularity. |

This doesn’t seem to converge easily and some companies argue that it depends on the definition of SL-PRS resource. I ll try once more to see if the following could be acceptable:

[CLOSED] Feature Lead Proposal 3.2.7-v1

For a a dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - Alt. 3: SL-PRS-resource-based allocation

  - Note: An allocated SL-PRS resource may occupy a subslot or a slot.

Companies views

| vivo | We agree the intention of FL, but we wonder “ time-domain” is needed in the main-bullet. |
| --- | --- |
| Huawei, HiSilicon | Ok. By the ‘subslot’ in the Note: An allocated SL-PRS resource may occupy a subslot or a slot, we are saying the case one SL-PRS resource does not occupy the whole slot but not the case of subslot based allocation. |
| CATT | Support the proposal in principle with a comment on the note. Regarding the note, the subslot is not clear for us. We prefer to use the wording of “at least one OFDM symbol” instead of “a subslot or a slot” in the note. We prefer the following revision: ##### Updated [MEDIUM] Feature Lead Proposal 3.2.7-v1 For a a dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a - Alt. 3: SL-PRS-resource-based allocation Note: An allocated SL-PRS resource may occupiesy at least one OFDM symbol a subslot or a slot. |
| OPPO | Support, we prefer to keep “time-domain”, if SL PRS bandwidth is too flexible as discussed in AI 9.5.1.1, SL-PRS-resource-based allocation may not be applicable to frequency domain. |
| LGE | Support |
| Samsung | We think that the following proposal does not provide something meanful information. - Alt. 3: SL-PRS-resource-based allocation - Note: An allocated SL-PRS resource may occupy a subslot or a slot. We need to discuss exactly about how/when SL-PRS resoruce(s) can occupoy a sublot or a slot. However, this proposal is unclear about this. |
| ZTE | With Huawei’s clarification on “sub-slot”, we support this proposal in general. While we generally agree with this proposal, it may be best to wait for progress in AI 9.5.1.1, it is not agreed yet on whether the definition of a SL-PRS resource is limited within a slot for a resource pool. Or do we mean the same “SL-PRS resource” in two agendas? |
| Xiaomi | We are fine with the proposal. But we also share ZTE concern that it is better to treat the proposal after the definition of SL PRS resource is clarified in the other agenda. |
| Panasonic | Ok |
| Continental Automotive | We are in principle OK, but prefer not to use the word “subslot” since it is not defined before. CATT’s modification seems appropriate. |
| InterDigital | We are fine with the proposal. |
| Qualcomm | Not support While the note helps clarify the issue of dynamic duration, it is not clear that it precludes it. As we mentioned in the previous round, we do not think dynamic durations are workable in NR sidelink. ##### [MEDIUM] Feature Lead Proposal 3.2.7-v1 For a a dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a - Alt. 3: SL-PRS-resource-based allocation - Note: An allocated SL-PRS resource may occupy a subslot or a slot, where the number and start of OFDM symbols for a resource is semi-statically defined. - FFS when an SL-PRS resource may occupy a subslot. |
| Intel | We share Samsungs understanding. |
| Nokia, NSB | OK |
| Spreadtrum | We are fine with Samsung’s version. |
| CMCC | We support QC’s revision. This proposal is just talking about the SL PRS resource granularity in the time domain, it can be a slot, or a sub-slot in case of TDM-based multiplexing of SL PRS from different UEs. But anyways, the starting symbol and length should be pre-configured. |

FL comment: This is not converging this meeting. Hopefully it can be discussed next meeting after the SL-PRS resource definition is more clear.

### 3.2.8 Additional Proposals

The following proposals were found in the contributions:

| LGE | Proposal 6: TDM of multiple SL PRS resources from a single UE in a slot is supported in a dedicated resource pool. Proposal 12: The candidate positions of SL PRS resource in a slot are determined based on the number of symbols of SL PRS resource. |
| --- | --- |

## Shared Resource Pool

The following agreement was reached with regards to this topic:

Agreement With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 Ues should be ensured.

### 3.3.0 Meaning of backward compatibility

The following agreement was reached with regards to this topic:

Agreement With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 Ues should be ensured.

The related discussion can be found here:

R1-2210598     Moderator Summary #4 on potential solutions for SL positioning   Moderator (Qualcomm Incorporated)

The following was explained (at least from FL side) on what we were meaning by the proposal that was eventually agreed  in the same meaning as an outcome of this email discussion.

| FL Update | To ZTE: We are trying to define the differences betweens dedicated and shared resource pool by describing a few main properties. One main property of shared resource pool is whether backward needs to be taken into account. It was also the comment that was discussed online and the chair’s suggestion to try address what dedicated and shared resource pool mean. For dedicated resource pool, Feature Lead Proposal 5.1.1.A-v4 addresses what a dedicated resource pool could contain. For shared resource pool, backward compatibility with rel-16/17 does NOT mean that the Rel-16/17 UEs are going to transmit or receive SL-PRS. It means that a Rel-18 UE that supports SL-PRS could transmit/receive SL-PRS in a resource pool that contains Rel-16/17/18 SL Data communications and that the Rel-16/17 UEs can avoid resources to be used by SL-PRS with existing signaling. For example, Rel-18 UE reserves a slot with SCI-1 that Rel-16/17 can decode, and then the Rel-16/17 UEs avoid the resources reserved for SL-PRS without needing to know that those resources are for SL-PRS. Note that at least one company proposed that a shared resource pool for SL-PRS could correspond to a Rel-18 shared resource pool and will not accommodate the Rel-16/17 UEs. To MTK: Chair suggested that we need to provide more clarity about dedicated resource and shared resource pool before trying to downselect. I think, one of the main things to understand is the backward compabilitiy aspect of shared resource pool. It is important to arrive at a common understanding on a few main items to see how we can proceed on this debate. |
| --- | --- |

Companies are encouraged to comment whether they have a different understanding for backward compability than the one described above.

[HIGH] Feature Lead Question 3.3.0-v0

In a shared resource pool for positioning, with regards to the meaning of backward compatibility requirement, when SL-PRS is transmitted in a resource,

- Alt. 1: Legacy UEs should be able to avoid such a resource

- Alt. 2: Legacy UEs should be able to decode data PSSCH in that slot

Companies views

| ZTE | Alt.1 with comment. Do FL try to say when SL-PRS is transmitted in a slot, Alt.1: Legacy UEs should be able to avoid such a slot? |
| --- | --- |
| Lenovo | Aligned with Alt. 1 understanding, that Legacy UEs have no choice but to avoid such resource indicated in SCI-1 |
| InterDigital | We prefer Alt. 1. We think the backward compability primarily entails that R16/R17 and R18 UE (SL-PRS transmission) reserve and use resources in the shared resource pool based on the sensing. In other words, the R18 UE’s transmission of PSSCH and/or SL-PRS in a slot is transparent to R16/R17 UEs and they only decodes SCI-1 for resource reservation information. |

### 3.3.1 SCI for SL-PRS

| Futurewei | Proposal 6: In shared resource pools support Option 1, i.e., SCI-1A and a new SCI-2 for SL-PRS related L1 signaling. |
| --- | --- |
| Huawei, HiSilicon | Proposal 12: For a shared resource pool, support to use some reserved bits in the SCI-1 to indicate a PSSCH only transmission or a SL-PRS transmission with slot structure Option 1 or Option 2. Proposal 13: For SL-PRS transmissions with slot structure Option 2 (SL-PRS without SL-SCH) in a shared resource pool, support a two-stage SCI with the inclusion of a second stage SCI format to indicate the reservation of SL-PRS with a granularity of SL-PRS resource. |
| Continental | Proposal 2.4: Introduce new 3rd-stage SCI for Scheme 2 SL positioning resource allocation/reservation in a shared RP by a Rel-18 Tx UE. Proposal 2.5: The new 3rd-stage SCI format would contain information in one of the following formats - Option 1: Explicit indication of the priority, time and frequency resource assignment and resource reservation period for SL positioning. - Option 2: Indication of resource allocation via a resource index which maps to a combination of resource allocation parameters. Proposal 2.6: Backward compatibility with Rel-16/17 UEs can be ensured by having the Rel-18 Tx UEs indicate their resource reservation for SL positioning or communication in the 1st-stage SCI. |
| vivo | - For the SCI in the shared resource pool, - SCI format 1-A can be reused. - SL-PRS flag indication can be indicated by the SCI. |
| Spreadtrum | Proposal 8: A new Second-order SCI can be introduced for R18 UE for SL-PRS transmission in shared resource pool. |
| CATT, GOHIGH | Proposal 19: For the backward compatibility of shared resource pool, the PSCCH design for SL-PRS should keep the same as that of Rel-16/17. |
| Intel | - For shared resource pool for SL-PRS and SL communication, - SL-PRS transmission is associated with PSSCH and occupies same BW as the PSSCH. - SL-PRS and PSSCH/PSCCH can be multiplexed in a TDM manner |
| Panasonic | Proposal 7: The reservation could be with both 1st stage SCI and 2nd stage SCI Proposal 8: A unified reservation scheme should be designed for both aperiodic and periodic transmissions Proposal 10: Either new or re-used 2nd stage SCI can be used for backward compatibility with R16/17 UEs |
| LGE | Proposal 1: The legacy 2-stage SCI format is reused for SCI associated with SL PRS in both a dedicated and a shared resource pool. Proposal 10: SCI indicates the information of all the SL PRS resources reserved by a UE, which are TDMed within a slot in a shared resource pool. Proposal 14: The 1st-stage SCI associated with SL PRS carries at least the following information. - Resource reservation information of PSSCH carrying the 2nd-stage SCI - Indication of a new 2nd-stage SCI format for SL PRS resource information Proposal 15: New 2nd SCI format is introduced for indicating the SL PRS resource reservation, which includes at least the followings. - SL PRS resource ID - SL PRS resource TDM index within a slot - RE offset of the first symbol of SL PRS resource - Full Layer-2 source/destination IDc |
| Sharp | Proposal 3: In a shared resource pool, a SL-PRS is transmitted within a resource reserved/allocated by SCI format 1-A, i.e. reservation of the resource is same as in Rel-16 NR SL. Proposal 8: In resource allocation scheme 2, SL-PRS resources in a resource pool are reserved/allocated/coordinated by control signalling (e.g. SCI) in the same resource pool. |
| Samsung | Proposal 12: In a shared resource pool for SL PRS and SL communication, TX UE indicates whether SL PRS is transmitted or not for co-existence with SL communication. - The presence of SL-PRS is indicated in the 1st SCI by using the reserved bits.c |
| ZTE | Proposal 10: In a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication - Use one bit to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are transmitted - Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication - Extra bits are needed to indicates: whether only SL-PRS, only SL-data, or both SL-PRS and SL-data are transmitted - At least the following information is transmitted by means of SCI: priority of SL-PRS, SL-PRS resource time and frequency indication (e.g. reservation period), source ID, destination ID, cast type indicator. |
| CEWiT | Proposal 14: For legacy SL resource configuration for SL positioning, SL-PRS will be indicated using first stage SCI and new second stage SCI. Proposal 15: For legacy SL resource configuration for SL positioning, first stage SCI will be reinterpreted to convey subchannel is used for SL positioning if no legacy channel is multiplied in the subchannel. Otherwise, reserved bit can be used to indicate the presence of SL-PRS and other channels. Proposal 16: For legacy SL resource configuration for SL positioning, Subchannel will carry the SL-PRS, PSSCH with SL positioning protocol message, PSCCH with first stage SCI and second stage SCI piggybacked on PSSCH. The PSSCH will be multiplexed with SL-PRS in TDM manner only Proposal 17: For legacy SL resource configuration for SL positioning, reporting of the measurement can be triggered by TX UE in the first stage SCI. Resource reservation mechanism of first stage SCI (SCI 1-A) can be used to decide the measurement reporting resources using resource reservation field from stage 1 SCI and providing PRS report trigger field as additional signal using reserved bit. |
| Apple | Proposal 10: For Scheme 2 SL-PRS allocation in the shared resource pool, the sensing-based resource pool may reuse the Stage 1 (legacy) SCI with the introduction of a new Stage 2 SCI format to support the indication of both SL communications and SL-PRS parameters. |
| Ericsson | - Resource allocation for the SL PRS in a shared resource pool uses SCI for the purpose of sensing. SCI for SL PRS allocation contains the same resource allocation information as legacy V2X: - Frequency resource assignment in subchannels - Time resource assignment - Resource reservation period - FFS: use of additional fields for positioning |
| Qualcomm | Proposal 19: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity. Proposal 20: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index. |

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

| - Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication | - Futurewei, Spreadtrum, LGE, CEWiT, Apple, Qualcomm, CATT, GOHIGH |
| --- | --- |
| - Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication |  |
| - Eitehr new or the same 2nd stage SCI | - Panasonic, ZTE |

During previous meeting, there was majority support for option 1 already, so hopefully it can be agreeable now.

[CLOSED] Feature Lead Proposal 3.3.1-v0

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication

    - FFS: Details

    - Note: Data can also be indicated using the 2nd stage SCI in addition to SL-PRS

Companies views

| CATT | Support. |
| --- | --- |
| vivo | For us, the SCI 1-A should be reused to guarantee backward compatibility. For the 2nd stage SCI, either new or the same 2nd stage SCI can be used based on different configurations and can be further studied. For example, Whether SL PRS is present can be indicated in the SCI format 1-A (indicated by reserved bits). If the SL PRS present in the slot, the starting symbols of SL-PRS/SL PSSCH can be configured by the higher layer. In this case, the same 2nd stage SCI can be reused. Another way is that the starting symbol of SL PRS is indicated in the 2nd stage SCI, then new 2nd stage SCI should be defined to indicate the resource of SL-PRS. |
| Sharp | We are fine with the proposal except the note. For backward compatibility we think the note should rather say “data is also indicated using the 2nd stage SCI, as specified in Rel-16/17”. |
| Xiaomi | support |
| OPPO | This proposal should be made as Working assumption rather than an agreement. As we commented on Proposal 3.2.2, multiplxing PSSCH and SL PRS in same slot would complicated the slot structure and degrade the performance of SL positioning, from out perspective, 2nd SCI should not be considered unless the number of bits for SL PRS indication exceeds the capacity of SCI 1-A, but we totally have no idea of the number now. Next, we propose to consider exiting 2nd SCI format also. Furthermore, the last Note should be removed, we do not think data can be transmitted with SL PRS in the same slot due to back-ward compatibility restriction. With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a new 2nd stage SCI format are used for SL-PRS indication - FFS: Details - Note: Data can also be indicated using the 2nd stage SCI in addition to SL-PRS |
| Qualcomm | Support, we are also ok with Sharp’s addition |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | OK |
| Spreadtrum | Support |
| DCM | OK |
| CMCC | OK |
| Samsung | Main bullet is OK. For the note, our preference to use 1st SCI (by using reserved bits). We can discuss it separately as the first FFS. |
| Intel | We are fine with the proposal. |
| ZTE | Support. Resource exclusion in sensing mechanism is based on information indicated in the 1st stage SCI. Therefore we also support the SCI 1-A should be reused (especially the time/frequency resource assignment and resource reservation period should cover both SL-data and SL-PRS). |
| Fraunhofer | Support |
| Lenovo | Support FL’s proposal |
| Toyota | Support. |
| LGE | We support FL proposal. The 2nd-stage SCI format field in SCI1-A needs to be updated. |
| Continental Automotive | We have reservations about how backward compatibility with Rel-16/17 UEs could be achieved with the current form of the proposal. Suppose the reserved bits in SCI1-A are used to indicate whether the resources are used for SL communication only or both SL positioning and communication. In the latter case, the rest of the SL communication indications need to be transmitted using the existing 2nd-stage SCI (2-A or 2-B). In this case, the same 2nd-stage SCI cannot be used for indicating the SL positioning resources, since Rel-16/17 UEs would require all the fields in the 2nd-stage SCI for SL communication related indications. Thus, a new 3rd-stage SCI in addition to the already existing SCI1-A, SCI2-A and SCI2-B as in our proposal would be required. This would also make the development future proof since if there are additional types of indications for SL positioning in future, the new 3rd-stage SCI can be modified accordingly, whereas a new 2nd-stage SCI may be inflexible in size. |
| Futurewei | OK |
| SONY | Support |
| Apple | Support |
| Nokia, NSB | Support |
| Ericsson | Ok if PSSCH is also scheduled with PRS. Otherwise we do not support a 2nd stage SCI. not that this is also discussed in 9.5.1.1. |

Current status

Support the proposal

- CATT, Sharp, Xiaomi, Qualcomm, Interdigital, NEC, Panasonic, Spreadtrum, DCM, CMCC, Samsung, Intel, ZTE, Fraunhofer, Lenovo, Toyota, LGE, Futurewei, Sony, Apple, Nokia, NSB (21)

Has reservations

- Continental

Only if PSSCH is scheduled with PRS

- Ericsson

Might accept as WA with removing the note and leaving new/existing 2nd stage SCI open

- OPPO

Leave new/existing 2nd stage SCI open (?)

- vivo

FL comment: FL’s understanding on the “backward compability requirement” for shared resource pools is that the legacy UEs should be able to avoid a reserved slot that has SL-PRS. However, I think that maybe 2 companies (OPPO and Continental) assume that “backward compatibility requirement” also means that the legacy UEs should be able to decode correctly the data in the slot that has SL-PRS. My understanding is that this is not the case of the majority of companies that support this proposal. The removal of the “Note” below, doesn’t mean that we agree that PSSCH is not transmitted in the slot, it just means that it is discussed separately (See 3.3.2).

[CLOSED] Feature Lead Proposal 3.3.1-v1

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a new 2nd stage SCI format are used for SL-PRS indication

    - FFS: Details including whether existing 2nd stage SCI could also be used

    - Note: Data can also be indicated using the 2nd stage SCI in addition to SL-PRS

Companies views

| LGE | We do not support to reuse the existing 2nd-stage SCI, which was not designed for signaling SL PRS. We suggest to remove the new text in FFS. |
| --- | --- |
| Vivo | If multiplexing is not supported in the shared resource pool, what SL-PRS-specific information is needed to be indicated compared to the existing 2nd SCI. All listed information except FFS information in Proposal 3.2.5-v0(SCI for SL-PRS) can be provided by the existing 1st SCI+ reserve bit +2nd SCI, or could the proponent provide more information on why we need to design a new second SCI. |
| Ericsson | Do not support. For sensing, the first stage SCI should be sufficient for the V2X UEs to evaluate slot availability. for the measuring UEs, we can rely on assistance data. In the current state of the proposal, it seems to be impossible to transmit PRS in the shared pool without PSSCH, which we do not support. |
| OPPO | We propose the following, original proposal may lead to both new and existing 2nd SCI are used for SL PRS indication. As SCI 1-A can only indicate 4 2nd SCI formats, we already have 3 now, if we introduce one more here there is no room to intoduce additional 2nd SCI anymore, hence we propose to consider existing 2nd SCI as much as possible. - Opt. 1: SCI1-A & a new 2nd stage SCI format are used for SL-PRS indication - FFS: Details including whether existing 2nd stage SCI could also be used |
| Feature Lead | Companies are encouraged to see the status and are kindly asked to reconsider if they can be flexible: Current status Support the proposal - CATT, Sharp, Xiaomi, Qualcomm, Interdigital, NEC, Panasonic, Spreadtrum, DCM, CMCC, Samsung, Intel, ZTE, Fraunhofer, Lenovo, Toyota, LGE, Futurewei, Sony, Apple, Nokia, NSB (21) Has reservations - Continental Only if PSSCH is scheduled with PRS - Ericsson Might accept as WA with removing the note and leaving new/existing 2nd stage SCI open - OPPO Leave new/existing 2nd stage SCI open (?) - vivo I would like to ask the 21 companies that supported the proposal whether they will be OK to step back on the decision of “new 2nd stage SCI” and only progress that there will be “a 2nd stage SCI” and leave it open whether it is new or one of the existing. ##### [HIGH] Feature Lead Proposal 3.3.1-v2 With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a new 2nd stage SCI format are used for SL-PRS indication - FFS: Details including whether existing 2nd stage SCI could also be used - Note: Data can also be indicated using the 2nd stage SCI in addition to SL-PRS |
| CATT | Support the updated proposal 3.3.1-v2. |
| LGE2 | Support the original FL proposal 3.3.1-v1. With the existing 1st SCI, there is no field to indicate e.g. comb RE offset and SL PRS starting symbol for multiplexing. New informations including above needs to be transmitted by a new 2nd SCI because the existing 2nd SCI does not have such fields. |
| Continental Automotive | Following the FL’s comment above, our understanding of backward compatibility is that legacy UEs should also be able to decode data in PSSCH in a shared RP. Since even the multiplexing of SL PRS with PSSCH within a slot in a shared RP is still under discussion in Feature Lead Proposal 3.3.2-v1, we do not see how the common understanding is otherwise. If this multiplexed data is only meant for Rel-18 UEs, we do not understand the utility of a shared RP. Rel-18 UEs can just use the dedicated RP for SL positioning and legacy RP for SL communication. To seek common ground, we request to have a separate proposal on the meaning of backward compatibility and whether legacy UEs should be able to decode PSSCH data in a shared RP. This proposal could be considered after gaining clarity on that. |
| CMCC | Support the updated proposal v2. |
| Xiaomi | Can accept the updated proposal. |
| Samsung | Support the updated proposal 3.3.1-v2. |
| ZTE | Support the updated proposal. |
| Huawei, HiSilicon | OK with the updated proposal v2. One question we’d like to ask but missing from last round due to copy-paste mistake maybe is that whether we are assuming both data and SL-PRS should target the same Rx UE or not? |
| Spreadtrum | We are fine with the updated proposal. |
| CEWiT | Fine with updated proposal. |
| Ericsson | Firstly, our understanding to backward compatibility is the same as the FL. The legacy UEs should be able to skip SL PRS slots when detecting an SCI they don’t identify as SL for communication, and should be able to measure the slot availability. It does not mean the UEs are supposed to be able to decode or a PSSCH meant for positioning UEs (if present). This is why we think that a solution without SCI2 should be feasible. For the sake of progress, we are ok with option1. But we think it makes the shared resource pool a lot less attractive and means the dedicated pool will in practice always be used – which means the SL legacy pool will have less resources. |
| NEC | Support the updaed proposal 3.3.1-v2. |
| Intel | We support the proposal in the latest update of the feature lead in this table. |
| Toyota | We support the updated proposal 3.3.1-v2. |
| Feature Lead | To Huawei, HiSilicon: - It hasn’t been discussed yet, and to some extend related to 3.3.2 . My understanding is that one potential simple solution is to have a 2nd stage SCI that could schedule both SL-PRS and SL-SCH in a slot; in such a case, typically the 2nd stage SCI would have a single source and destination ID, and therefore it will be the same Rx UE. To Continental: - Please see section 3.3.0. Even though, my expectation is that, since most companies support the current proposal, they have the same understanding on what “backward compatibility” means. I am fine though to check further. To Ericsson: - Thank you for accepting the majority view. From my side, yes likely SCI-1A could also work; some arguments in favor of current approach seem to be that it is more forward compatible, and allows more flexibility compared to only having SCI-1A. To LGE: - I understand that there is strong majority to go for “new 2nd stage SCI”. However, it should be OK to do some progress and go with v2, and finalize this issue in May meeting. |
| Qualcomm | We prefer v1, but can accept v2. |
| Nokia, NSB | OK with either version |
| Sharp | We are fine with FL proposal 3.3.1-v2 to move forward. Same understanding as FL on backward compatibility (i.e. a legacy UE should be able to avoid selecting a resource reserved for a SL transmission containing SL-PRS). |
| InterDigital | We are fine with the updated proposal 3.3.1-v2. |
| Lenovo | We support the updated FL’s proposal 3.3.1-v2 with FFS on details |

### SL-PRS multiplexing within the S-RP

| Futurewei | Proposal 5: For shared resource pools SL-PRS transmission, support Option 1 and Option 2. |
| --- | --- |
| Huawei, HiSilicon | Proposal 7: Support TDM between SL-PRS and PSSCH when SL-PRS and PSSCH are transmitted in the same slot for the shared resource pool. Proposal 8: For the shared resource pool, the following two options on the slot structure are supported: - Option 1 (SL-PRS with SL-SCH): SL-PRS multiplexed with PSSCH carrying SL-SCH for the same Tx UE in a slot, where SL-PRS is multiplexed with PSSCH in a similar manner as the multiplexing of SL CSI-RS - Option 2 (SL-PRS without SL-SCH): SL-PRS and PSSCH carrying SL-SCH for the same Tx UE multiplexed on different slots. Proposal 9: For slot structure Option2 (SL-PRS without SL-SCH) for the shared resource pool, support SL-PRS with or without PSCCH multiplexing in a slot. Proposal 10: For slot structure Option2 (SL-PRS without SL-SCH), support TDM-based multiplexing and Comb-based multiplexing of SL PRS from different UEs in a slot in a shared resource pool. |
| vivo | - For a shared resource pool, - The PSSCH and SL-PRS should be TDMed in the slot for a UE. - The starting symbol and/or the number of SL-PRS should be configured by higher layer signaling or indicated by SCI. |
| Spreadtrum | Proposal 7: For multiplexing of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH), the TDM method is baseline. |
| CATT | Proposal 4: For shared resource pool, SL-PRS should be TDMed with PSSCH in the slot-level or symbol-level. Proposal 5: For shared resource pool, SL-PRS should be configured with the same bandwidth with PSSCH, when SL-PRS and PSSCH occupy different symbols in the same slot. |
| Sony | Proposal 1: In a shared resource pool, SL-PRS can be allocated within a resource pool and separated with a guard period. Proposal 3: In a shared resource pool, SL-PRS is time domain division multiplexed (TDM) with the legacy sidelink PHY channel(s).c |
| Samsung | Proposal 3: In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot. - The PSSCH and SL-PRS bandwidth is the same. - With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported. |
| ZTE | Proposal 9: In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot. - With regards to the bandwidth of SL-PRS, down-select between (1) bandwidth is indicated in SCI; (2) bandwidth is up to higher layer configuration in a shared resource pool - Only TDM between SL-PRS transmission and SL-data transmission is supported |
| Interdigital | Proposal 6: Sub-channel-based SL-PRS resource allocation for SL-PRS transmission multiplexed with PSSCH transmission in a shared resource pool. Proposal 11: Study enhancements for SL Mode 1 resource allocation for multiplexed SL-PRS and PSSCH transmission in a shared resource pool. |
| Apple | Proposal 4: For the shared resource, a discussion is needed on the modifications to the SL-slot frame structure including: - the P-SRS placement within the slot (e.g., the PSSCH region, the PSFCH region or in an independent region of the SL slot). The SL PRS can be transmitted as a few symbols or occupy the entire slot. They can be: - transmitted within the PSSCH - transmitted within dedicated slots within an existing resource pool containing the PSSCH/PSSCH/PSFCH - transmitted within the PSFCH - transmitted in an independent section within the slot similar to the PSFCH - The need for a new AGC symbol and additional gap symbols between the different signals/channels Proposal 5: For the shared resource, the SL positioning symbols can use the same resource allocation method as PSSCH/PSCCH/PSFCH Proposal 6: To support backwards compatibility for Rel-17/Rel-17 Ues for the shared resource pool, the following options should be considered: • specifying that only slots that are used for transmission to Rel-18 and above Ues may use a shared resource pool o signaling to indicate the resources that the PSSCH/PSFCH should avoid due to use by the positioning reference signal • limiting shared resources to aperiodic SL-PRS transmission and/or • modifying the behavior of shared periodic or semi-persistent SL-PRS in shared resources |
| Intel | - For shared resource pool for SL-PRS and SL communication, - SL-PRS transmission is associated with PSSCH and occupies same BW as the PSSCH. - SL-PRS and PSSCH/PSCCH can be multiplexed in a TDM manner |

We observe a lot of companies suggested to TDM SL-PRS with PSSCH in a shared RP:

- TDM of SL-PRS with PSSCH in shared resource pool

  - Huawei, HiSilicon, vivo, Spreadtum, CATT, Sony, Samsung, ZTE, Intel

- Sub-channel-based multiplexing of SL-PRS with PSSCH

  - Interdigital

However, I think more views are needed before trying to downselect.

[CLOSED] Feature Lead Proposal 3.3.2-v0

In a shared resource pool, at least support the following:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot

    - With regards to PSSCH and SL-PRS multiplexing

      - Alt. A.1: Only TDMing is supported

      - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported

    - With regards to PSCCH and SL-PRS multiplexing

      - Alt. B.1: Only TDMing is supported

      - Alt. B.2: TDMing or sub-channel-based FDMing is supported

- Study further the following option:

    - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot

Companies views

| CATT | Support. We prefer Alt. A.1 and Alt. B.1. |
| --- | --- |
| vivo | For PSSCH and SL-PRS multiplexing, only TDMing is supported (i.e., Alt. A.1). For PSCCH and SL-PRS multiplexing, only TDMing is supported(i.e., Alt. B.1). Only support SL-PSR with associated PSCCH in the slot, which is beneficial to reduce the half-duplex impact in the resource pool. |
| Sharp | We support Alt. A.1 and Alt. B. 1. We think the 2nd bullet is not backward compatible with legacy Ues and should not be pursued. |
| Xiaomi | We are open to alt A.1 and A.2. And we prefer to Alt B.1. |
| OPPO | Firstly we propose to add a note to clarify that the PSSCH is used to convey 2nd SCI only. In addition, in shared resource pool one slot may contain PSCCH, PSSCH, DMRS for PSSCH, SL-PRS, etc., we should discuss the multiplexing scheme with all the siganls/channels being taken into account, at this stage we prefer to keep the multiplexing scheme open. In a shared resource pool, at least support the following: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot - Note: The PSSCH is used to convey 2nd SCI only. - FFS how to multiplex the signal(s) and channels in a slot. - With regards to PSSCH and SL-PRS multiplexing - Alt. A.1: Only TDMing is supported - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported - With regards to PSCCH and SL-PRS multiplexing - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or sub-channel-based FDMing is supported - Study further the following option: - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot |
| Qualcomm | We are ok with the proposal and support Alt A2 and Alt B1. |
| InterDigital | We support the proposal. We prefer Alt. A. 1 and Alt. B. 1. |
| NEC | Support Alt. A.1 and Alt. B.1. For Opt. 2, we also think the PSCCH associated with SL PRS is necessary. |
| Panasonic | OK |
| Spreadtrum | Support. We prefer Alt. A.1 and Alt. B.1. |
| DCM | OK and we prefer Alt A2+B1. |
| CMCC | We prefer Alt. A1 and Alt. B1 to support only TDM-based multiplexing between SL PRS and other SL channels to simplify the design. |
| Samsung | We support Alt A.1 and Alt. B.1. |
| Intel | We are fine with the proposal. We support Alt. 1 and Alt B.1 for multiplexing between SL PRS and PSSCH/PSCCH, respectively. For FFS, it is not clear to how this would work if without associated PSCCH. We are open to discuss the case without PSSCH in a slot. |
| ZTE | We support Alt.A.1 and Alt.B.1. Regarding the last bullet for study, we think resource collision may happen if SL-PRS without associated PSCCH in a slot is allowed. |
| Lenovo | Supportive of the Alts for further down-slection with the following preferences: For PSSCH and SL-PRS multiplexing- Alt. A.1: only TDMing is supported For PSCCH and SL-PRS multiplexing- Alt. B.1: only TDMing is supported |
| Huawei, HiSilicon | For Opt.2, it can be considered for SL-PRS transmission decoupled of data transmission. If a UE is triggered to transmit SL-PRS and may not have data to transmit, or a UE may not always need to transmit data to Rx UE of SL-PRS, Option 2 could be a suitable choice. |
| Toyota | Support FL’s proposal. |
| LGE | For the 1st sub-bullet, we support Alt. A.1 because it’s simple and there is no IBE issue between SL PRS and PSSCH. For the 2nd sub-ublle, we support Alt. B. 1 because it’s backward compatible in that only PSSCH can be FDMed with PSCCH. If SL PRS is FDMed with PSCCH, it unnecessarily complicates the SL PRS resource allocation because the number of SL PRS REs per symbol will be different. For the 2nd bullet, we don’t support to study Opt.2. It seems related to cross-slot scheduling, which is not backward compatible design. |
| Continental Automotive | We support the proposal with the following preferences - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot - With regards to PSSCH and SL-PRS multiplexing - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported - With regards to PSCCH and SL-PRS multiplexing - Alt. B.2: TDMing or sub-channel-based FDMing is supported Our opinion is that at least PSCCH should be associated with SL-PRS and PSSCH or a new data channel may be required for transmitting other data related to positioning. |
| Futurewei | The text is not clear, it implies that all of these alternatives will be supported “at least support the following”. Do we select some alternatives/option or support all of them? Some clarification is necessary before adopting. |
| Apple | Alt A.1 and Alt B.1. |
| Nokia, NSB | Support. Prefer Alt A.2 and Alt B.1 |
| Ericsson | Support. For option 2, we do not support sending SL PRS without PSCCH. |

Current status

In a shared resource pool, at least support the following:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot

    - With regards to PSSCH and SL-PRS multiplexing

      - Alt. A.1: Only TDMing is supported

        - CATT, vivo, Sharp, Xiaomi, Interdigital, NEC, Spreadtrum, CMCC, Intel, ZTE, Lenovo, LGE, Apple

      - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported

        - Xiaomi, OPPO, Qualcomm, Continental, Nokia, NSB

    - With regards to PSCCH and SL-PRS multiplexing

      - Alt. B.1: Only TDMing is supported

        - CATT,vivo, Sharp, Xiaomi, Qualcomm, Interdigital, NEC, Spreadtrun, CMCC, Intel, ZTE, Lenovo, LGE, Apple, Nokia, NSB

      - Alt. B.2: TDMing or sub-channel-based FDMing is supported

        - Continental

    - Option 2

      - Support: Huawei, HiSilicon

      - Do not support: Sharp, NEC, Intel, ZTE, LGE, Ericsson

- OK as is without downselection: Panasonic, Toyota

- OPPO suggests to remove all subbullets from the PSSCH/SL-PRS and that only 2nd SCI is included in the PSSCH.

Based on the above, I propose the following:

[CLOSED] Feature Lead Proposal 3.3.2-v1

In a shared resource pool:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot

    - With regards to PSSCH and SL-PRS multiplexing, pick one of the following alternatives (downselect next meeting):

      - Alt. A.1: Only TDMing is supported

      - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported

    - With regards to PSCCH and SL-PRS multiplexing, pick one of the following alternatives

      - Alt. B.1: Only TDMing is supported

      - Alt. B.2: TDMing or sub-channel-based FDMing is supported

    - The PSSCH is used to (pick one of the following alternatives)

      - Alt. C.1: convey 2nd SCI only

      - Alt. C. 2: convey 2nd SCI and/or SL-SCH

- [Study further the following option:

    - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot]

Companies views

| FL | Lets focus the replies on the following: - Whether more companies want to study Option 2 - Whether Alt. C1 or C2 should be supported |
| --- | --- |
| LGE | Regarding PSSCH, we support slot-based TDM for SL PRS and SL-SCH transmision. In one type of slot, SL PRS is transmitted with PSCCH/PSSCH that only carries SCI. In the other type of slot, PSCCH and PSSCH carrying SCI+SL-SCH are transmitted. So we suggest to add Alt. C.3: - Alt. C. 3: convey 2nd SCI only in a slot where no SL-SCH but SL PRS is transmitted. convey 2nd SCI+SL-SCH where no SL PRS but SL-SCH is transmitted. where, SL-SCH carries SL positioning related data (e.g. SLPP, measurement report) Regarding Opt. 2, we do not support transmission of SL PRS alone in a slot (or cross-slot scheduling). SL PRS should be transmitted together with PSCCH/PSSCH. |
| Ericsson | We support inclusion of both option 1 or option 2 with the modification: - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot] |
| OPPO | As majority companies prefer to have the sub-bullets on multiplexing, we can live with them for sake of progress, but we believe Alt. A.2 should be generalized as below, as the structure of SL-SCI-RS is much different from SL PRS, we do not think multiplexing scheme similar can be used for SL PRS. - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported As commented on Proposal 3.2.2, we propose to keep Alt. B.2. We do not think Alt. C.2 is acceptable. Firstly, if multiplexing SL-SCH and SL PRS within one slot, the SL-SCH can only target to Rel-18 UE, but we have not seen Rel-18 specific traffic defined so far. Next, even there was Rel-18 specific traffic, as resource selection is triggered by MAC layer, we need a mechanism to identify this type of traffic from APP layer to MAC layer. Furthermore, to support this feature may have significant impact on the entire resource selection framework, as in the current specificaiotn resource selection is design for one TB transmission only, we will have another resource selection scheme for SL PRS only, to support this feature we need one more resource selection scheme which is kind of the mixture of the two above, every component of resource selection from higher layer to physical layer would be impacted. We support Opt. 2, companies object this due to the concern of standalone SCI or impact to resource selection, to address the concern, we propose to focus on Scheme 1 resoruce selection only. In Scheme 1 resoruce selection, anchor UE may transmit always-on SL PRS on periodic resources provided by configured SL grant, it is not efficient to transmit SL PRS always with SCI, higher layer siganling can be considered to indicate the SL PRS transmission. - [Study further the following option: - Opt. 2: For Scheme 1 resoruce selection, SL-PRS with or without associated PSCCH and without PSSCH in a slot] |
| CATT | Support. We prefer Alt. A.1, Alt. B.1 and Alt. C. 2. |
| LGE2 | Regarding the last bullet, RX-UE side cannot differentiate between scheme 1 and shceme 2 resource allocation. All the dynamic/configured grant resources are converted to the same format as scheme 2 resource. We’re reluctant to have separate signaling desing for scheme 1 and scheme 2. As a conclusion, we suggest to delete the last bullet. |
| Continental Automotive | We suggest that this proposal be treated after gaining clarity on the meaning of backward compatibility (whether PSSCH in shared RPs can be decoded by legacy UEs). |
| CMCC | Regarding the content of PSSCH, prefer Alt. C1. Regarding Option 2 of SL PRS without associating PSCCH, it may over complicate the SCI indication and resource selection procedure including how sensing window is determined, resource exclusion steps, etc. We only have 2 meetings left, we suggest not to consider it. |
| Xiaomi | We do not think option 2 needs to be further investigated. For alt c.1 and alt c.2, there is another alternative c.3: convey 2nd stage SCI and MAC CE. In SL communication, the IUC message can be transmitted like this way. We prefer to be open at the current stage. |
| Samsung | It seems that our view is not captured in the 1st round of discussion. Support Alt. A.1, Alt. B.1, and Alt C..2 |
| ZTE | For Alt. C: We support Alt.C-2. In our understanding, shared resource pool allow: - SL-PRS and SL-data transmitting in a slot, - only SL-data transmitting in s slot - Only SL-PRS transmitting in a slot For Option 2: We do not support only transmit SL-PRS in a slot. |
| Huawei, HiSilicon | One question for the following - The PSSCH is used to (pick one of the following alternatives) - Alt. C.1: convey 2nd SCI only - Alt. C. 2: convey 2nd SCI and/or SL-SCH For shared pool, Alt.C.1 PSSCH is used to only convery 2nd SCI, are we assuming no data will be transmitted in the shared pool or we actually assume SL data and SL-PRS will not be transmitted in the same slot? |
| Spreadtrum | We are fine with the proposal. We prefer Alt. A.1, Alt. B.1 and Alt. C. 2. |
| NEC | We support Alt. A.1, Alt. B.1 and Alt C. 2. |
| Intel | Support the updated proposal. If downselection can be discussed during this meeting we support Alt A.1, Alt B.1, and Alt C.2 . |
| Feature Lead | To Huawei, HiSilicon: C.1 assumes that there is no data at all; only 2nd SCI. Based on the above, let me update the Alternatives C as following, and bring back B.2 in brackets (for now only OPPO has expressed support). ##### [HIGH] Feature Lead Proposal 3.3.2-v2 In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot - With regards to PSSCH and SL-PRS multiplexing, pick one of the following alternatives (downselect next meeting): - Alt. A.1: Only TDMing is supported - Alt. A.2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported - With regards to PSCCH and SL-PRS multiplexing, pick one of the following alternatives (downselect next meeting): - Alt. B.1: Only TDMing is supported - [Alt. B.2: TDMing or sub-channel-based FDMing is supported] - The PSSCH is used to (pick one of the following alternatives) - Alt. C.1: 2nd SCI only - Alt. C. 2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” OR “2nd SCI and SL-SCH” - [Study further the following option: - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot] |
| Qualcomm | We support Alt A.2 and the Alt C.3 to minimize changes to legacy design and minimize impact on the performance of sidelink communications in the resource pool. For example, if PSCCH is transmitted without FDMed SL-SCH when allocation is larger than one sub-channel, there would be impact on the AGC of other UEs. |
| Nokia, NSB | Support. We prefer Alt A.2, Alt B.1 (only remaining Alt anyway) and Alt C.2. Also, the last bullet is OK. Current wording of Alt C.2 with “and/or” would allow PSSCH carrying SL-SCH without 2nd stage SCI – is that the intention? |
| Sharp | We are fine with the first main bullet. We do not support Opt. 2. We share the latest comment from LGE. |
| InterDigital | We support the proposal and our preference is as below - We support study Opt. 2 as below - Study further the following option: - Opt. 2: SL-PRS with or without associated PSCCH and without PSSCH in a slot] - We prefer Alt. A.1. - We prefer Alt. C.2. |

### 3.3.3 Bandwidth of the SL-PRS Transmission

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 7: SL PRS can be transmitted in multiple segments whereby different SL PRS segments - may be transmitted in different time slots - partially overlap in frequency domain - jointly cover wider bandwidth whereby the following transmission options are considered: - SL PRS segments are FDM’ed with data (PSSCH). SL PRS segments are FDM’ed with PSFCH. |
| --- | --- |
| Huawei, HiSilicon | Proposal 11: For shared resource pool configured with SL-PRS, the BW of the reserved SL-PRS follows the FDRA field in the first stage SCI. |
| CATT, GOHIGH | Proposal 20: The bandwidth of SL-PRS should be equal to that of associated PSSCH in shared resource pool. |
| Sony | Proposal 4: In both shared and dedicated resource pool, the bandwidth of SL-PRS can be configured, such that it can be the same as the sub-channel bandwidth within a resource pool. |
| Fraunhofer | Proposal 4: For the shared resource pool the bandwidth of the SL-PRS shall be configurable and independent from the bandwidth selected for the PSCCH and PSSCH. |
| Apple | Proposal 15: The frequency resources used for SL positioning can be set equal or smaller than the shared channel frequency domain allocation size for both the shared and dedicated resource pools. |

[CLOSED] Feature Lead Proposal 3.3.3-v0

In a shared resource pool, the bandwidth of indicated SL-PRS and the PSSCH scheduled by the PSCCH in a slot:

  - Alt. 1: is the same (i.e., follows the FDRA field in the 1st stage SCI)

  - Alt. 2: can be same or different (e.g. bandwidth is up to higher layer configuration in a shared resource pool)

Companies views

| CATT | Support. We prefer Alt.1. |
| --- | --- |
| vivo | We support alt. 1. The bandwidth of SL-PRS should be the same as the indicated bandwidth of SL PSSCH to guarantee backward compatibility for the legacy SL communication transmission. |
| Sharp | We support Alt. 1. It is unclear how to achieve backward compatibility with legacy Ues with Alt. 2. |
| Xiaomi | Alt 1. |
| Qualcomm | Alt 1 Since the resources are reserved by the UE, it would be more efficient to full utilize them. |
| InterDigital | We support the proposal. We prefer Alt. 1. |
| NEC | We prefer Alt. 1. |
| Panasonic | Alt 1 |
| Spreadtrum | We prefer Alt. 1. |
| DCM | Alt 1 |
| CMCC | Alt. 1. Similar comments on Proposal 3.2.6, same issue is also treated under AI 9.5.1.1, some coordination is needed. |
| Intel | We support Alt. 1 as this does limit the amount of signalling in the 2nd stage SCI SL-PRS resource configuration signalling. |
| ZTE | As long as the frequency resource assignment is larger than or can cover the frequency range of SL PRS, there should be no backward compatibility issue. It is feasible to configure the bandwidth of SL PRS in a shared resource pool. |
| Fraunhofer | Prefer Alt A.1 and Alt B.1. The backward compatibility of Alt-2 may depend on whether it assumes the same SL CSI-RS allocation. If the SL CSI-RS allocation differs for SL-PRs, we may have concerns regarding the backward compatibility of this alternative. |
| Huawei, HiSilicon | Alt.1, with a modification of the proposal so that it deos not imply that the SL-PRS and PSSCH are in the same slot In a shared resource pool, the bandwidth of indicated SL-PRS and the PSSCH scheduled by the PSCCH in a slot: |
| Futurewei | OK. Alt 1. |
| Apple | Alt1 |
| Nokia, NSB | We understand this will be discussed in the SL-PRS AI |

FL: Continue the discussion in 9.5.1.1

### 3.3.4 Resource allocation granularity

The following agreement was reached with regards to this topic:

Agreement Study further the granularity of time-domain resource allocation for SL-PRS transmission.

| Interdigital | Proposal 2: Support slot-based SL-PRS resource allocation in a shared resource pool. Proposal 5: SL-PRS bandwidth allocation is based on sub-channel in a shared resource pool. |
| --- | --- |
| Qualcomm | Proposal 3: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size and comb offset. |

[CLOSED] Feature Lead Proposal 3.3.4-v0

For a a shared resource pool for SL positioning, with regards to the time-domain resource allocation support a

- slot-based SL-PRS resource allocation

Companies views

| vivo | Support. Remove “a” before shared resource pool |
| --- | --- |
| Qualcomm | Support |
| InterDigital | We support the proposal. |
| Intel | Agree. In our understanding there due to legacy resource allocation there is no other option for shared pools. |
| ZTE | Support. |
| Apple | Okay |
| Nokia, NSB | Not sure about the exact implication of this. Legacy SL uses slot-based resource allocation and we’ll keep that. However, in terms of SL-PRS, we can still accommodate multiple SL-PRS resources for TX by the same UE which are TDMed in the slot – is that the correct understanding? |
| OPPO | OK. |
| CATT | Support. For shared RP, the legacy RA mechanism should be reused. |
| LGE | Support. |
| Spreadtrum | Support |
| Intel | Support |
| Feature lead | To Nokia: Yes, this is up to the discussion of TDMing SL-PRS in a slot in a shared RP which is being discussed in 9.5.1.1. My understanding however is that currently there is a proposal for Email Endorsement that precludes TDM-based multiplexing in a Shared RP. Correction: Correcting my previous response since the question was from a single Tx UE. This is discussed in 9.5.1.1 : [Medium] FL1 Question 2.4-3, so it is not precluded by this proposal. |
| Qualcomm | Support |

### 3.3.5 Additional Proposals

| Nokia, NSB | Proposal 3: A SL PRS transmission may occupy a wide bandwidth spanning across multiple SL resource pools. For monitoring and reserving SL resources across multiple resource pools, study methods that enable coordination between UEs that are configured with different resource pools. |
| --- | --- |
| Continental | Proposal 3: To enable Scheme 1 SL positioning resource allocation for Ues belonging to different cells, consider the following options: - Option 1: Indirect resource allocation with one UE acting as initiator/mediator for all Ues - Option 2: Direct resource allocation where Ues first coordinate among each other for resources required and individually request their own serving cells for resources - Option 3: Indirect or direct resource allocation depending on factors such as the participation of out-of-coverage Ues in the SL positioning operation. |
| Xiaomi | Proposal 7. The start symbol or the number of symbols for SL PRS transmitted together with PSCCH/PSSCH is (pre)configured for the shared resource pool. |
| LGE | Proposal 7: TDM of the multiple SL PRS resources from different Ues within a slot in a shared resource pool is not supported. Proposal 8: Comb-based multiplexing of the multiple SL PRS resources from different Ues within a slot in a shared resource pool is not supported. Proposal 9: TDM of multiple SL PRS resources from a single UE in a slot is supported in a shared resource pool. |
| Sharp | Proposal 2: A legacy resource pool can be configured to be either a shared resource pool (for SL-PRS transmissions), or not a shared resource pool (for SL-PRS transmissions). - By default, a legacy resource pool is not a shared resource pool. Proposal 9: For resource allocation scheme 2, specify mechanisms to cap the overall delay of a positioning service due to autonomous SL-PRS resource selection respectively by each anchor UE. |
| CMCC | Proposal 3: For a shared resource pool, the available symbols for SL-PRS and data should be distinguished by semi-static configuration in Rel-18. |

## Scheme 1

The following agreement was made with regards to the topic of SL Positioning Resource Allocation Modes:

Agreement With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. Gnb, LMF, Gnb & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting Ues (e.g. IUC-like solutions). - Note: Other Schemes are not precluded to be studied - FFS how to handle resource allocation of SL-Positioning measurement report Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. Gnb, LMF, Gnb & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from Gnb Up to further discussion which one or more of these shall be applicable Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

| Futurewei | Proposal 7: For Scheme 1 (Mode 1) SL PRS resource allocation use DCI Format 3_0 signalling resource allocation as a starting point. |  |
| --- | --- | --- |
| Huawei, HiSilicon | Proposal 14: For resource allocation scheme 1, support gNB to grant Ues the SL-PRS resource for transmission at least by a DCI. Proposal 15: For resource allocation scheme 1, support an in-coverage UE to request resource for SL-PRS transmissions from gNB by UL MAC CE, which also includes the requested SL-PRS characteristics. |
| Continental | Proposal 2.1: Introduce new DCI format for Scheme 1 resource allocation to Rel-18 and beyond Ues by a network gNB for SL positioning in a shared RP. Proposal 2.2: The new DCI format would contain one of the following alternatives in any combination with already existing fields in DCI format 3_0. - Option 1: Explicit indication of time and frequency resource assignment for SL positioning, and configuration indexes to indicate the type of resource allocation. - Option 2: Indication of resource allocation via a resource index which maps to a combination of resource allocation parameters. |
| vivo | - Dynamic grant, or through configured grant type 1/type 2 from gNB should be used for scheme 1 SL-PRS resource allocation. - New DCI format higher layer configuration parameters should be defined for scheme 1 SL-PRS resource allocation. |
| OPPO | Proposal 1: Regarding Scheme 1 SL PRS resource allocation, resource allocation Mode 1 for SL communication is used as starting point, and some enhancement should be considered if new time/frequency resource granularity is introduced for SL positioning. |
| Spreadtrum | Proposal 9: Dynamic grant and configured grant Type 1/Type 2 should be supported for network-centric operation SL-PRS resource allocation. |
| CATT, GOHIGH | Proposal 7：Existing resource allocation Mode 1 for PSSCH in Rel-16/17 NR V2X should be the starting point for resource allocation Scheme 1 for SL-PRS. Proposal 8: Resource allocation Scheme 1 should support both SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation. Proposal 9: Resource allocation Scheme 1 for SL-PRS should also support two kinds of scheduling grants for different application scenarios: SL-PRS dynamic grant and SL-PRS configured grant. Proposal 10: A new DCI format (e.g., DCI format 3-2) should be introduced in order to support the scheduling of SL-PRS with the resource allocation Scheme 1. Proposal 11: Resource allocation Scheme 1 for dedicated resource pool should be reused for shared resource pool. |
| Intel | - For SL-PRS transmissions in a shared resource pool using scheme-1, the same resource allocation via DCI signaling is considered for transmissions containing SL-PRS as the starting point. - This includes use of SCI format to signal SL-PRS for a shared resource pool allocation. |
| Sony | Proposal 12: RAN1 to define new DCI for SL-PRS resource allocation purpose.c |
| Panasonic | Proposal 2: Both higher layer signaling from UE’s own or gNB and lower layer signaling from gNB are supported for triggering SL-PRS transmission in Scheme 1. |
| Xiaomi | Proposal 8: For gNB based scheduling, CG type 1, CG type 2 and dynamic grand based scheduling are all supported. |
| China telecomc | Proposal 3: Regarding SL-PRS resource allocation of scheme 1 (Network-centric SL-PRS resource allocation), consider whether reuse the existing DCI 3_0 with necessary fields extension, or define a new DCI format for aperiodic scheduling. |
| Sharp | Proposal 7: For resource allocation scheme 1, specify mechanisms for LMF to assist a target UE to identify anchor UE(s), e.g. by providing a list of candidate anchor Ues. |
| CMCC | Proposal 11: For SL-PRS, in Scheme 1, semi-persistent transmission with a transmission periodicity after activation and until deactivation can be supported. - A DCI similar to format 3_0 is used for the activation/deactivation. Proposal 12: R16 DL PRS like always-on periodic transmission of SL-PRS is applicable for Scheme 1 only. |
| Lenovo | Proposal 1: In the case of Scheme 1, RAN1 to support the transmitting UE receiving a SL-PRS resource allocation information via the gNB through a dynamic grant, or through a configured grant type 1/type 2 from the gNB. Proposal 2: In the case of Scheme 1, RAN1 to support uni-directional and bi-directional request and allocation of resources by the gNB. In case of uni-directional resource allocation, SL-PRS resource from Tx UE to Rx UE is allocated. In case of bi-directional resource allocation, SL-PRS resource from 1) Tx UE to Rx UE 2) Rx UE to Tx UE is allocated. FFS types of resource allocation grants supported by bi-directional resource allocation by the gNB. |
| ZTE | Proposal 12: For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB. Proposal 13: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from gNB through Dynamic grant, or through configured grant type 1/type 2 - Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed. At lease the following should be added in either DCI format 3-0 or a new DCI format: - Resource pool index (either dedicated resource pool or shared resource pool for SL-PRS) - SL-PRS resource configuration index (SL-PRS resource ID or SL-PRS resource ID + SL-PRS resource set ID if resource set is introduced) - Time gap indicating the gap between DCI reception and the first SL-PRS transmission scheduled by DCI - Configuration index indicating which configured grant type 2 is activated by the DCI when UE is configured with multiple CG |
| CEWiT | Proposal 1: In scheme 1 type of resource allocation, gNB provides the suitable resource allocation information to the Ues using dynamic grant and configured grant type 1 & 2. Proposal 2: In scheme 1 type of resource allocation, gNB may provide resource configuration information to the positioning server/LMF. |
| Fraunhofer | Proposal 8: Support Type 1 CG, Type 2 CG and DG in SL-PRS resource allocation Scheme 1 to support periodic, semi-persistent and aperiodic SL-PRS transmissions. Proposal 9: Study a SL-PRS resource allocation Scheme 1 in which a UE requests and receives resources for SL-PRS transmissions by peer Ues in a SL positioning group. Proposal 10: Strive for a unified SL-PRS resource allocation Scheme 1 between a dedicated resource pool and at least stand-alone SL-PRS transmissions in a shared resource pool. |
| Apple | Proposal 7: For dynamic resource allocation and type 2 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the SL-PRS grant can be indicated by: - Option 1: Updating DCI 3_0 with a new field such as a “SL-PRS resource index” at least for the shared resource pool. - Option 2: Introducing a new DCI format at least for the dedicated resource pool. Proposal 8: For type 1 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the “sl-ConfiguredGrantConfig” may have a new field such as “sl-PRS_resources”, which indicates the SL-PRS resources in the grant. |
| Ericsson | - When the transmitting UE is operating with Scheme 1, resource allocation is controlled by the gNB. - For SL-PRS in Scheme 1, support aperiodic (DCI based), semi-persistent and periodic (RRC configured) resource allocation of the SL PRS to the transmitting UE - FFS: how to start and stop semi-persistent transmission - FFS: whether this affects the SCI sent by the transmitting UE. - For the measuring Ues, SLPP / LPP assistance data includes periodicity or start/stop information for the SL PRS resource. |
| Qualcomm | Proposal 5: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information. Proposal 6: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation. Proposal 7: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging. Proposal 8: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications. |
| Asustek | Proposal 2: In scheme 1, introduce SL PRS grant for allocating SL PRS resource(s) with scheduling information of RE offset and SL PRS occasion indication. |

Several companies propose supporting all 3 optiosn (DCI-based, CG type1 and CG type 2). With regards to the DCI used in Scheme 1, there seems to be good support to introduce a new DCI:

- Futurewei, Continental, vivo, CATT, GOHIGH, Sony, CMCC

Also a few companies seem to be OK with either introducing a new DCI or reusing the DCI 3_0. Based on the above, my proposal is the following:

[CLOSED] Feature Lead Proposal 3.4-v0

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - Support activating the configured grant using the  DCI format 3_X

- Configured grant type 2

  - the SL-PRS transmission(s) follows the higher layer configuration

Companies views

| CATT | Support. We prefer to introduce a new DCI 3_2. |
| --- | --- |
| Vivo | Support. Considering the comb-based multiplexing and TDM-based multiplexing of SL-PRS in the dedicated resource pool, the new DCI format and configured grant should be defined for SL-PRS in scheme 1. |
| Sharp | We are fine with supporting all of dynamic grant and type 1/2 configured grants, but we don’t see a need for a new DCI format. With a new DCI format, the UE may have to be configured with 2 SL DCI formats just for SL transmissions on a same set of (shared) resource pools, and furthermore, extra workload has to be introduced e.g. on size alignment of SL DCIs. |
| Xiaomi | Support |
| OPPO | Basically fine with the proposal, however, seems we should swap the sub-bullets for Configured grant Type1 and Type 2 to align with existing definitions. - Configured grant type 1 - Support activating the configured grant using the DCI format 3_X - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - the SL-PRS transmission(s) follows the higher layer configuration - Support activating the configured grant using the DCI format 3_X |
| Qualcomm | We generally support the proposal, but prefer to not introduce a new DCI format. For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - Support activating the configured grant using the DCI format (downselect) 3_X or 3_0 - Configured grant type 2 - the SL-PRS transmission(s) follows the higher layer configuration |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | Support |
| Spreadtrum | Support |
| DCM | OK |
| CMCC | Support. |
| Samsung | OK with the proposal. Considering some design may be applied to dedicated pool only, we also would like to add a that: Further discuss whether same/different DCI format(s) are applied for shared pool and dedicated pool. |
| Intel | We are fine with the proposal. |
| ZTE | Support. Moreover, current sidelink communication does not need LMF’s participation and clearly LMF does not have any sidelink related information. In order to support LMF’s involvement for sidelink positioning and hybrid positioning, sidelink related information transfer between LMF and gNB should be supported. For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - Support activating the configured grant using the DCI format 3_X - Configured grant type 2 - the SL-PRS transmission(s) follows the higher layer configuration - Support SL-PRS configuration transfer between LMF and gNB |
| Fraunhofer | Support |
| Lenovo | Support |
| Huawei, HiSilicon | We agree with Qualcomm’s comments that a new SCI format may not be needed. |
| LGE | In SL mode-1 resource allocation, dynamic grant, configured grant type 1 and 2 are provided to UE by gNB scheduling. In SL dynamic grant, gNB schedules the dynamic resources, where DCI only indicates the resources. In SL configured grant type 1, gNB schedules the semi-persistent resources, where RRC indicates the periodicity and offset of the resources, as well as activates/deactives the resources. In SL configured grant type 2, gNB schedules the semi-persistent resources, where RRC indicates the periodicity and offset of the resources, but DCI activates/deactives the resources. We prefer to reuse the above mechanism of SL mode-1 resource allocation for scheme 1. With this understanding, we suggest the following modification. ##### [HIGH] Feature Lead Proposal 3.4-v0 For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - Support a new DCI format (3_X) that indicates the dynamic resources, and consider DCI format 3_0 as a starting point - Configured grant type 1 - Support RRC indicates the periodicity and offset of the semi-persistent resources, and activating/deactivating the configured grant using the DCI format 3_X - Configured grant type 2 - Support RRC indicates the periodicity and offset of the semi-persistent resources, and DCI activates/deactivates the configured grant - the SL-PRS transmission(s) follows the higher layer configuration |
| Continental Automotive | We support the first bullet. For the two bullets after, we have the same comment as OPPO above. |
| Futurewei | Support in principle. |
| SONY | Support. |
| Apple | OK |
| Nokia, NSB | Support; configured grant types are the wrong way round |
| Ericsson | OK |

FL comment: Seems there is generally alignment. Most comments appeared to be in the details and restructuring of the bullets. A few companies suggested to leave it open whether to reuse existing DCI format. Please see below the updated proposal that we can try online:

[CLOSED] Feature Lead Proposal 3.4-v1

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - FFS Reuse DCI format 3_0 or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - the SL-PRS transmission(s) follows the higher layer configuration

- Configured grant type 2

  - Support activating the configured grant using the  DCI format (downselect) 3_X or 3_0

- [The above mechanisms use NR Rel-16 mode-1 signaling as baseline]

- [Support SL-PRS configuration transfer between LMF and gNB]

- FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool.

- FFS: Further details

[CLOSED] Feature Lead Proposal 3.4-v1

For Scheme 1 SL-PRS resource allocation

- Support SL-PRS configuration transfer between LMF and gNB

  - FFS: Details, e.g., LMF sending a request to serving gNB requesting SL-PRS transmission properties, serving gNB responding with a SL-PRS resource allocation, LMF providing SL-PRS resource allocation to other gNBs.

Companies views

| FL comment | My understanding on the “SL-PRS configuration transfer between LMF and gNB” is similar to what is specified for SRS configuration transfer: - LMF sends a request of “SRS transmission properties” - gNB responds with configured SRS - LMF provides this SRS configuration to other gNbs in order for them to measure Please comment if you are in principle support of such procedure, or any clarifications/suggestions to understand whether someone supports a different approach. I added the above 3 steps also in the FFS; i am fine to fully remove it, but it may be useful to understand whether some company envisions a very different solution so that we can capture it. |
| --- | --- |
| OPPO | The issue is out of RAN1’s scope, proponrnts should trigger this discussion in RAN3. |
| CATT | Low priority. |
| LGE | We share the view with OPPO. It’s not RAN1 scope. |
| Continental Automotive | Prefer to not discuss this in RAN1. |
| CMCC | We would like to know the intention of the SL-PRS configuration transfer between LMF and gNB. In Uu positioning, gNB provides DL PRS configuration to LMF because it is LMF to provide assistance data to the Rx UE. For Scheme 1, we have agreed that gNB will allocation resources to SL PRS Tx UEs, then in in-coveage scenarios, it may be the case that the LMF will provide some configuration information to Rx UE, where LMF needs to be provided with SL PRS configuration from gNB? But anyways, it is out of RAN1 scope. Regarding the detailed examples in the proposal, it’s not clear to me why the SL-PRS configuration information should be provided to LMF and then provided to other gNBs, is it for further resource coordination? If so, we don’t think it is needed. |
| Samsung | Support the proposal. |
| ZTE | Support the proposal. As what we explained during GTW session, enabling that LMF request SL-PRS configuration and gNB provides SL-PRS configuration to LMF is beneficial. For example, if UE report a SL-PRS resource ID to LMF in a measurement report, without any knowledge of SL-PRS configuration, how can LMF tell the difference between SL-PRS resource 1 and SL-PRS resource 2? In Uu positioning, we get the point of CMCC’s comment. However, similarly, as point out by Alex, even though LMF do not need to allocate positioning SRS resources for UE, LMF can still request gNB for SRS configuration and gNB can provide the SRS configuration. This procedure is clearly specified in TS 38.455. Therefore in SL, we also think it is reasonable to support this proposal. Regarding whether this proposal is out of RAN1 ‘s scope, we think generally a lot of agreements we made is reflected in RAN2 or RAN3’s spec. I do not think RAN1 has nothing to do with this discussion. Anyway if we have consensus, we can send an LS to RAN3 and ask them whether they think it is ok. |
| Huawei, HiSilicon | We do not support this. The motivation is not clear. If it is about LMF to coordinate the SL-PRS resource allocation, we haven’t even discussed that in LPHAP multi-cell SRS, and it is better RAN3 to discuss it. If it is for LMF identifying the SL-PRS in the UE SL positioning measurement report, we do not believe it is needed. - For Uu positioning, the LMF obtaining the DL-PRS configuration is mainly for assistance data distribution, or identified that UE is within a specific PRS beam (identified by the boresight direction). However for SL-PRS, none of the use cases applicable to Uu need to be considered. From LMF point of view, a ToA/RSRP/angle measurement between two Ues with the corresponding time stamp is sufficient to calculate the UE location. I suggest proponents to explain why the SL-PRS information is needed to calculate the UE location? |
| Ericsson | We support the proposal. For a measuring UE in coverage, this would avoid having to get multiple batches of assistance data from all the tx UEs, in our understanding. |
| Intel | In our understanding this information exchange does not need to be discussed in RAN1. For the phy it is sufficient to know which information is exchanged between the gNB and the UE. If the information is only forwared by the gNB is in our understanding not relevant for the physical layer design. |
| Toyota | No strong opinion but it should be noted that confgurations are generally subject to extensions/corrections, gNBs and LMFs can be based on different releases and versions of the standard but neither LMFs nor gNBs have any kind of release or version indicators. It raises some questions (which are however not a subject for discussion in RAN1) such as: how can it be ensured that LMF can comprehend the SL PRS configuration and how the LMF is supposed to behave if the gNB’s response message contains a non-comprehended configuration, extension, or correction? |
| Qualcomm | Support the proposal |
| Nokia, NSB | Ok in general. But, we share OPPO’s view that this needs to be discussed in RAN3. |
| Sharp | Agree with other companies that this could be treated with low priority. |
| InterDigital | We may need further clarification on which SL positioning scenario the SRS configuration example may apply to. Is this to consider e.g. a scenario where Anchor UEs are in different cells, i.e., receiving this configuration from different gNB? If all anchor UEs and target UE are in the same cell, the SL-PRS configuration can also between LMF and gNB, but no other gNBs are involved. |
| ZTE2 | Thanks so much for sharing your understanding. I will try to clarify our thoughts as follows: 1. The motivation Regarding Huawei’s comment: “I suggest proponents to explain why the SL-PRS information is needed to calculate the UE location?” I agree with your point that basically all LMF need for calculating location is timing/angle measurement, time stamp, UE ID and timing quality (mandatory field in 37.355 or 38.455 measurement report). What I tried to say is if LMF do not receive any SL-PRS configuration from gNB, the “SL-PRS resource ID” (which companies support including you) in a measurement report sent to LMF seems useless and non-comprehended. Moreover, if we do not support this proposal, LMF can not do anything when the requirement can not be met. If SL positioning requirement is high, LMF may request gNB to allocation a SL-PRS resource with large bandwidth. (similar as LMF sends a request of “SRS transmission properties”. not LMF coordinate) We actually can not understand the logic here, given the situation that current NRPPa spec already support SRS configuration transfer between LMF and gNB, why can’t we further consider the similar mechanism for SL-PRS configuration. (Please correct me if my understanding is wrong) 2.RAN1’s involvement We can tell multiple companies have concerns on discussing this proposal in RAN1. However since RAN1 is the leading group, it is natural for RAN1 to trigger such discussion and study the potential signaling. We understand SL-PRS configuration parameters are still under discussion and we only try to use the generic wording and see if we all notice the need to support LMF knowing some SL-PRS configurations. Just put an agreement we made in RAN1 Rel-17, which was also regarding some signalings between LMF and gNB: Agreement: NR supports at least the following additional assistance signaling from LMF to gNB/TRP to facilitate UL measurements of UL-AOA - Indication of expected AoA/ZoA value and uncertainty (of the expected AoA/ZoA value) range(s) - FFS: Details of procedure for providing the assistance - FFS: Reference angle of expected AoA/ZoA 3.Scenarios Regarding InterDigital’s comment, we are open to further discuss the scenarios and probably no other gNBs will be involved. In my understanding, Alex just put a possible example. Overall, we support this proposal. And if companies are not ready for supporting this proposal. We are ok to soft the wording (e.g. change support to study). Again we feel it is RAN1’s responsibility to trigger the discussion: either reach an agreement in RAN1 or have an conclusion in RAN1 and ask RAN3 to further investigate. |
| Lenovo | We think this issue can be deprioritized for now and revisited later , once the role of the LMF is clearer. We agree that in an LMF-depedent architecture the LMF can “recommend” some ideal SL-PRS configuration when certain requirements are not met but this is a best effort recommendation as the final decision in anyway up to the gNB. We also understand that SL positioning can work without LMF involvement even in in-coverage, so this scenario outlined in the proposal caters to one of the scenarios and may not necessarily have a critical impact on the overall Mode 1 design. |

Support

- Samsung, ZTE, Ericsson, Qualcomm

Do not support

- Huawei, HiSilicon

Low priority / not in RAN1 scope

- OPPO, CATT, LGE, Continental, Intel, Lenovo, Nokia, NSB

FL comment: It doesn’t seem that this will converge to something concrete in this meeting. Lets have this discussion as a starting point for next meeting.

## Scheme 2

#### 3.5.1 Resource allocation/selection – General

The following agreement was reached with regards to this topic:

Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

| Nokia, NSB | Proposal 12: Study the resource allocation mechanisms that allow one UE to allocate multiple SL PRS transmission opportunities for itself and/or other UE(s) in a SL positioning session. Proposal 13: In the case of a SL PRS allocation for multiple TX and/or RX Ues or under strict timing constraints, study how the priority of resource allocation can be raised as a function of the number of involved Ues and/or timing constraints. Proposal 15: Study at least the following methods for SL PRS conflict avoidance: comb offset coordination, SL PRS sequence selection, and CDM to share the same time-frequency resource (e.g., via IUC) and SL PRS muting patterns. |
| --- | --- |
| Futurewei | Proposal 1: Confirm the working assumption. Proposal 2: Investigate the necessary changes of the resource selection and allocation procedures to support comb-based SL-PRS multiplexing. Proposal 3: For the coexistence between random and sensing-based resource selection in the same resource pool, the random-based selection resources are separate or a subset of the sensing-based resources. |
| Huawei, HiSilicon | Proposal 21: Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - No enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool. |
| vivo | - In Scheme 2, confirm the working assumption that sensing-based and random selection can be allowed in the same resource pool. - No other enhancements are needed for the coexistence of random selection and sensing-based resource selection. - In Scheme 2, the full sensing mechanism should be supported at least, whether partial sensing should be supported can be further studied. |
| OPPO | Proposal 2: No enhancement is needed for coexistence of random selection and sensing-based resource selection in a resource pool. |
| Toyota | Observation 1: Studies of co-existence of random and sensing-based resource allocation in the same resource pool is supported in earlier Releases. Proposal 5: Confirm the working assumption and allow co-existence of random selection and sensing-based resource selection. |
| Spreadtrum | Proposal 10: the following Working assumption should be confirmed. - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool Proposal 14: UE A can select PRS resources in dedicated resource pool for UE B should be supported. |
| CATT, GOHIGH | Proposal 12: Confirm the working assumption: sensing-based and random selection can be allowed in the same resource pool. - It will be similar to NR Rel-16 or NR Rel-17 sidelink communication. - No enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool. Proposal 14: The sensing-based resource allocation and random resource selection of SL-PRS in dedicated resource pool should be modified according to the minimum resource allocation unit of SL-PRS. |
| Intel | - For SL-PRS transmissions in a shared resource pool using scheme-2, the resource allocation methods defined for SL communication for single shot transmissions are considered as a starting point. - Depending on the required mobility and allowed clock drift enhancements for RTT based exchanges are necessary. |
| CMCC | Proposal 4: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered as a candidate resource allocation mechanism for SL-PRS in Rel-18. Proposal 6: Some enhancements on resource allocation mechanism for RTT type solution should be introduced in Rel-18. - The motivation is to guarantee the interval b/w the SL-PRS resources selected by the two Ues small enough and mitigate the impact on clock drift. Proposal 9: The working assumption in RAN1#112 meeting should be confirmed, i.e., sensing-based and random resource selection can be allowed in same resource pool. - No further enhancements are needed for coexistence of sensing-based and random resource selection in a same resource pool. |
| lenovo | Proposal 3: RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS in Scheme 2. |
| ZTE | Proposal 15: Confirm the following working assumption: - Sensing-based and random selection can be allowed in the same resource pool |
| CEWiT | Proposal 5: For operations in scheme 2, support that an UE can suggest (and pre-reserve) SL-PRS resources for other UE(s) (e.g. responding Ues). |
| Interdigital | Proposal 15: Study Scheme 2 sensing enhancements to improve SL-PRS transmission reliability, e.g., prioritizing TDMed candidate resources. Proposal 16: Support a UE to transmit SL-PRS in resources selected and indicated by a peer UE in a position group using SL-PRS resource allocation Scheme 2. Proposal 17: Design a common SL-PRS resource allocation Scheme 2 for stand-alone SL-PRS transmissions in a shared resource pool and SL-PRS transmissions in a dedicated resource pool. Proposal 18: Support a joint resource allocation Scheme 2 for multiplexed SL-PRS and PSSCH/PSCCH transmissions based on legacy Mode 2 sensing procedure. Proposal 19: Confirm WA to allow both sensing-based and random selection in the same resource pool. |
| Apple | Proposal 9: For Scheme 2 SL-PRS resource allocation, the working assumption from RAN1 #112 shown below should be agreed to: • Working assumption: Sensing-based and random selection can be allowed in the same resource pool We do not see a need for any enhancements for coexistence of random selection and sensing-based resource selection in a resource pool similar to the Rel-16/Rel-17 design. |
| Ericsson | - For the applicable resource allocation schemes, support re-using the different schemes in AllowedResourceSelectionConfig for legacy sidelinks. |
| ITL | Proposal 1: With regards to resource allocation for sidelink RS with Operation Scenario 1(PC5-only-based positioning): - A master UE among multiple anchor Ues should be considered where sidelink RS is being transmitted from multiple anchor Ues to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor Ues (i.e. UL-TDOA-like operation) - The master UE can transmit resource allocation information for sidelink RS (which includes resource allocation information for all of the multiple anchor Ues) to the target UE |
| Mediatek | Proposal 3-2: When a UE reserves the resource for future transmission in SCI, the UE may also reserve the resource for the transmission of another UE. It can be done at least for the case without periodic transmission that, when 3 transmissions are reserved by UE A, one of them could be used for the transmission by UE B. In this way, the double sided SL-RTT could be realized |

[CLOSED] Feature Lead Proposal 3.5.1-v0

Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool.

- No enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool.

Companies views

| CATT | Support. |
| --- | --- |
| vivo | Support. |
| Sharp | We support the proposal. |
| Xiaomi | support |
| Qualcomm | Support |
| InterDigital | We support the proposal. |
| NEC | Support. |
| Panasonic | Support |
| Spreadtrum | Support |
| DCM | OK |
| CMCC | Support. |
| Intel | We are fine with the prosal. We would like to add that there should be an option to configure only sensing-based resource allocation in a resource pool. Therefore, we would like to revise the proposal to the following: ##### [MEDIUM] Feature Lead Proposal 3.5.1-v0 Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - No enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool. - It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation. |
| ZTE | Support. |
| Lenovo | Support |
| Huawei, HiSilicon | Support. |
| Toyota | Support. |
| Futurewei | It needs further discussion. How this feature will be supported together with SL PRS multiplexing? It is not clear. We prefer to be discussed after we decide on the resource allocation granularity. Will be random based and sensing based selection allowed in the same slot? |
| Apple | Support |
| Nokia, NSB | Support confirming the working assumption. Regarding “no enhancements needed” – prefer to not agree this now, may require study. |
| Ericsson | ok |
| LGE | Support |

Support

- CATT, vivo, Sharp, Xiaomi, Qualcomm, Interdigital, NEC, Panasonic, Spreadtrum, DCM, CMCC, ZTE, Lenovo, Huawei, HiSilicon, Toyota, Apple, Ericsson, LGE

Remove the subbullet on “no ehancements”

- Nokia, NSB

Add a subbullet

- Intel

Prefer to defer this

- Futurewei

FL comment: Wondering if removing the previous subbullet and agreeing on the main bullet would address Futurewei and Nokia’s concern. Also, Intel’s comment looks also natural, I am adding it but if it results to more questions, I don’t see it being necessary.

[CLOSED] Feature Lead Proposal 3.5.1-v1

Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool.

- No enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool.

- It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation.

Companies views

| OPPO | OK |
| --- | --- |
| CATT | Support We think the sub-bullet in red had better to be a note. |
| LGE | It is already possible to (pre-)configure a resource pool with sensing-based only or random selection only or both, according to the working assumption, which is also aligned with the case of Rel.17 SL communicatin resource pool. We don’t see the necessity of the last bullet, but it’s ok to leave it as a note for clarification, as suggested by CATT. |
| Continental Automotive | OK with the proposal. Make bullet in red a note. |
| CMCC | Support in general, the newly added bullet in red is not needed. |
| Xiaomi | OK |
| ZTE | Support. |
| Spreadtrum | OK |
| Ericsson | OK |
| NEC | Support. |
| Toyota | Support the proposal and share LGE’s view regarding the last bullet. |
| Qualcomm | We are ok with the proposal and echo the comments about the last bullet |
| Nokia, NSB | OK |
| Sharp | Agree with other companies to make the last bullet a note. |
| InterDigital | We are fine with the proposal. |

#### 3.5.2 Sensing-based Resource Allocation

Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective Related Agreement from a 9.5.1.1: Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

| Nokia, NSB | Proposal 9: For Scheme 2 resource allocation in at least dedicated resource pools, UE performing sensing SL PRS may use comb size and comb offset as parameters during sensing to determine if reuse of the same time resources via RE-level multiplexing is possible. This sensing operation may be (pre)-configurable. |
| --- | --- |
| Futurewei | Proposal 8: For multiple SL-PRS multiplexed in a single slot, extend the granularity of the resource selection procedure. Proposal 13: For a dedicated resource pool support only SL-PRS with the same priority. Observation 1: If only SL-PRS traffic with the same priority is supported in a dedicated resource pool, congestion control may not be necessary. |
| Huawei, HiSilicon | Proposal 16: For shared resource pool option 1 (SL-PRS with SL-SCH), - The SL-PRS bandwidth is the same with PSSCH. - No need to define independent scheme 2 resource selection for SL-PRS, i.e. the legacy Rel-16/Rel-17 PSCCH/PSSCH mode 2 resource selection scheme is used to determine the resource both for PSCCH/PSSCH and PRS. Proposal 17: PSCCH-RSRP should be configured for the sensing procedure in a shared resource pool when the slot structure Option2 (SL-PRS without SL-SCH) is applied. Proposal 18: For resource selection Scheme 2 of shared resource pool option 2 (SL-PRS without SL-SCH) from perspective of R18 UE, - The slot and subchannel-based resource first determined by legacy Rel-16/Rel-17 PSCCH/PSSCH mode 2 resource selection scheme to support the coexistence with communication UEs. - SL-PRS resource can be further determined within the selected slot and subchannel. Proposal 19: PSCCH-RSRP should be configured for the sensing procedure in a dedicated resource pool. Proposal 20: The SL-PRS comb-based resource selection should be supported for dedicated resource pool. - The comb-based SL-PRS resource should be operated during the sensing detection, candidate resource determination and resource exclusion. |
| vivo | - The SL-PRS can be used for RSRP measurement for sensing in scheme 2. - In the dedicated resource pool, the following information about SL-PRS can be taken into consideration in resource exclusion and allocation: RE offset, starting symbol. |
| OPPO | Proposal 3: The priority of SL PRS provided by higher layer and the priority of SL data should be comparable. Proposal 4: In the Scheme 2 resource pool, the parameters used to determine a SL PRS resource for sensing based resource selection should be (pre-)configured. Proposal 5: When TDM-based multiplexing of SL PRS from different UEs in a slot is supported, one UE can only use/reserve one and the same one SL PRS time resource in a slot. |
| Spreadtrum | Proposal 11: One SL-PRS resource or one SL-PRS resource set should be used as the granularity of resource allocation for SL-PRS transmission. |
| CATT, GOHIGH | Proposal 13: Resource selection mechanism for PSCCH/SL-PRS in the dedicated resource pool should reuse the existing resource selection mechanism for PSCCH/PSSCH in Rel-16/17 NR V2X as much as possible to reduce the workload of standardization and the complexity of UE implementation. |
| Intel | - Rel-16 resource (re)-selection procedure is the starting point for the design of the scheme 2 resource (re)-selection procedure in a dedicated SL-PRS resource pool. - The following topics should be considered for the scheme 2 resource (re)-selection procedure - SL PRS transmission all have the same priority and no pre-emption is considered. - Resource exclusion is considering SL-PRS resources in dedicated resource pools. - PSCCH DMRS RSRP is used for resource exclusion. |
| Xiaomi | Proposal 9: A candidate SL PRS resource is determined by the time/frequency position/size and comb offset in a dedicated resource pool. Proposal 10: S-RSRP measurement used to exclude SL-PRS candidate resource in scheme 2 resource selection is obtained - Option 1: from measuring DMRS of PSCCH conveying the SCI indicating the reserved SL PRS resource; - Option 2: from measuring SL PRS associated with the SCI indicating the reserved SL PRS resource. Proposal 11: For candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset, resource exclusion is performed with an additional S-RSRP threshold. |
| LGE | Proposal 16: In scheme-2, the SL PRS resources are selected based on the sensing results over the 2nd-stage SCI. - FFS whether the sensing results over the SL PRS resources (e.g. SL PRS RSRP) are additionally used for resource selection. |
| Samsung | Proposal 4: Comb based sensing and SL PRS selection mechanism are supported when comb-based multiplexing of SL PRS from different UEs in a slot is configured. Proposal 5: For Scheme 2, L1 SL-RSRP is measured on SL PRS. Proposal 6: For Scheme 2, L1 priority of SL-PRS is provided by the higher layer of UE and it is indicated by 1st SCI. |
| ZTE | Proposal 11: In a shared resource pool, priority indicator of SL-PRS is indicated in SCI format 1-A from Tx UE to Rx UE - Priority value of SL-PRS can be integer, non-integer, or discontinuous integer, or discontinuous decimal Proposal 14: For Scheme 2, with regards to sensing-based resource selection for SL-PRS, sensing is based on SCI: - For shared resource pool, Rel-16/17 resource (re)-selection procedure is reused for SL-PRS - For dedicated resource pool, the basic procedure of Rel-16 resource (re-)selection should be applied with minor modifications: - Reservation interval of SL-PRS: the determination of reservation interval can refer to the periodicities defined for DL-PRS. - Sensing window: Sensing window size should be equal to or larger than the reservation interval. - Selection window: the selection window size is related to the Qos requirement for SL-PRS. - Resource exclusion mechanism: whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction - Resource(s) exclusion is based on resources overlapping with reserved resource(s) of other UE whose RSRP measurement is larger than a RSRP threshold. - FDMed PRS multiplexing between UEs having positioning serving interaction should NOT be supported. |
| CEWiT | Proposal 3: For dedicated resource pool, T2 value in resource selection procedure should be dependent on location delay budge (LDB). LDB is latency budge for localization set by higher layer. |
| Interdigital | Proposal 13: Consider at least the following parameters for SL-PRS resource allocation Scheme 2. - SL-PRS L1 priority - SL-PRS bandwidth and comb pattern - SL-PRS time resource, e.g. TRIV - SL-PRS delay budget - Reservation interval - Resource selection mechanism |
| Apple | Proposal 12: For Scheme 2 SL-PRS allocation with Rel-16 type sensing in the dedicated resource pool, the following options should be considered: - Option 1: SL-PRS and PSCCH are allocated in the same section of the slot and can be scheduled together: a single resource allocation similar to the Rel-16 mode 2 resource allocation is sufficient. - Option 2: SL-PRS and the PSCCH are allocated in separate sections of the slot and cannot be scheduled together (e.g. a PSSCH section and a SL-PRS section): the resource allocation will depend on the mapping between the PSCCH and the SL-PRS. - Option 2-1: if an implicit mapping between the PSSCH resource and the SL-PRS resource is used , then a single resource allocation similar to the Rel-16 mode 2 allocation is sufficient. - Option 2-2: if explicit mapping in which the PSSCH indicates the SL-PRS resource is used, then the PSSCH and the SL-PRS may need separate resource allocations. |
| Ericsson | - Include PRS in the list sl-RS-ForSensing for candidate reference signals for RSRP measurement in the sensing procedure - FFS: include a DMRS+PRS RSRP measurement in the list of possible RS for sensing. - Priority handling of SL positioning and other V2X services, and potential modifications to resource allocation procedures, need to be studied. |
| Qualcomm | Proposal 9: Partial sensing procedure from Rel-17 is supported in the dedicated resource pool. Proposal 10: Higher layers directly indicate the SL-PRS resource selection window to lower layers. Proposal 11: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources. |
| Asustek | Proposal 3: In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level. |

[CLOSED] Feature Lead Proposal 3.5.2.A-v0

For the scheme 2 sensing-based resource allocation, resolve the brackets below by:

- Alt. 1: Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is also a starting point for the design

- Alt. 2: Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

Companies views

| CATT | Support. |
| --- | --- |
| vivo | In our opinion, the work on partial sensing should be postponed in this release. We can define SL-PRS resource allocation based on partial sensing after we complete SL PRS resource allocation based on the full sensing mechanism. So, Rel-16 can be as starting point |
| Sharp | We support Alt. 2. We think the support for partial sensing is not urgent can be considered after the basic SL positioning functionality is completed. |
| xiaomi | As SL positioning considers use cases including commercial and IIoT, partial sensing based resource selection would be benefitial to save the energy consumption. Therefore alt 1 is preferred. As we only need to study necessary changes, the workload to additionally support partial sensing based RA for SL PRS would not be high. |
| OPPO | Support Alt 1, partial sensing should be supported from the beginning, otherwise a UE except vehicle UE cannot perform sensing-based resource selection for SL-PRS due to its limited battery capacity. |
| Qualcomm | We support the proposal with Alt 1 |
| InterDigital | We support the proposal. We prefer Alt. 2 to start with R16 full sensing first. |
| Panasonic | Alt 2 |
| Spreadtrum | We support the proposal with Alt 1 |
| DCM | Alt 1 |
| CMCC | We support Alt. 2. We should focus on the basic functionalities of SL PRS resource allocation, while partial sensing is an enhanced mechanism for power saving UEs. |
| Samsung | Alt 2. We prefer to consider full sensing as a starting point. Since Rel-17 features also include IUC, we prefer to add one bullet to clarify that support of inter-UE coordination will be separately discussed. |
| Intel | We are fine with the proposal and support Alt. 2 as in our understanding a full sensing solution should be standardized before discussing any low power variants at this stage. |
| ZTE | We prefer Alt.2. |
| Lenovo | Support Alt. 2 with priority on the Full sensing scheme |
| Toyota | We support the FL’s proposal and prefer Alt 1. R18 should make use of functionality specified in earlier Releases. |
| LGE | We’re generally fine with FL proposal, but wonder if Rel.18 SL positioning workload is too heavy when partial sensing based resource allocation is supported for SL positioning. We prefer to remove or at least deprioritize the partial sensing issue. We can revisit them only after we finish the full sensing and random selection case for SL positioning. |
| Continental Automotive | We’re fine with the proposal and feel that it is better to prioritize Alt. 2. |
| Futurewei | OK with FL’s proposal |
| Apple | Prioritize Alt 2 |
| Nokia, NSB | Support; prefer Alt. 1 |
| Ericsson | Support, preference for alt2. |

Current status

- Alt. 1 supported by: 	Xiaomi, OPPO, Qualcomm, Spreadtrum, DCM, Toyota, Nokia, NSB (7)

- Alt.2 supported by: 		vivo, Sharp, Interdigital, Panasonic, CATT, CMCC, Samsung, Intel, ZTE, Lenovo, LGE, Continental, Apple, Ericsson (14)

- OK with proposal but Alt not picked: Futurewei

[CLOSED] Feature Lead Proposal 3.5.2.A-v1

For the scheme 2 sensing-based resource allocation, resolve the brackets below by:

- Alt. 1: Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is also a starting point for the design

- Alt. 2: Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

Current status

- Alt. 1 supported by: 	Xiaomi, OPPO, Qualcomm, Spreadtrum, DCM, Toyota, Nokia, NSB (7)

- Alt.2 supported by: 	vivo, Sharp, Interdigital, Panasonic, CATT, CMCC, Samsung, Intel, ZTE, Lenovo, LGE, Continental, Apple, Ericsson (14)

- OK with proposal but Alt not picked: Futurewei

Companies views

| Feature Lead | Companies are encouraged to see the status above and are kindly asked to reconsider if they can be flexible. |
| --- | --- |
| CATT | Support. We prefer Alt 2. |
| LGE | Support Alt 2. |
| Continental Automotive | We are fine with the proposal and can be flexible about Alt. 1 or Alt. 2. |
| CMCC | Alt. 2 |
| ZTE | We prefer Alt.2. at least we should firstly finish the design referring to Rel-16 resource selection procedure. |
| Fraunhofer | Our preference is for alternative 1 but we are open to consider alternative 2 if the majertoy prefer to consider full sensing only |
| Spreadtrum | We prefer Alt 1. |
| Intel | Alt. 2 |
| Qualcomm | We can compromise and accept Alt 2. Given the amount of work left to ensure basic functionality, we can accept not introducing partial sensing for the dedicated resource pool. Even though we view partial sensing as a very important feature to balance power savings with performance. |
| Nokia, NSB | Support. We prefer Alt 1, can accept Alt 2 |
| Sharp | Alt 2. |
| InterDigital | We support the proposal. We prefer Alt.2. Given the work load and historically slow progress in RAN1 discussion, we prefer to focus the R16-based framework. Note a lot of time was spent to work on paritial sensing alone in R17 SL comm work. |
| Lenovo | Support Alt. 2 |

| 2nd round | Alt. 1 supported by: - Fraunhofer (1st choice), Spreadtrum, Qualcomm (1st choice), Nokia, NSB (1st choice) Alt.2 supported by: - CATT, LGE, CMCC, ZTE. Fraunhofer (2nd choice), Intel, Qualcomm (2nd choice), Nokia, NSB (2nd choice), Sharp, Interdigital, Lenovo Can be flexible - Continental |
| --- | --- |

[CLOSED] Feature Lead Proposal 3.5.2.B-v0

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following modifications:

- With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support:

      - Option 1: SL-PRS

      - Option 2: PSCCH DMRS

      - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- With regards to the resource selection window:

      - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitude the Packet Delay Budget (PDB) with a new delay budget

      - Option 2: the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter)

- With regards to the SL-PRS priority:

      - Option 1: A single L1 SL-PRS priority allowed in for each resource pool

      - Option 2: SL-PRS transmissions can have different priority in a resource pool

- With regards to the definition of a candidate resource within the resource selection window:

      - Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set

- With regards to the reservation interval of SL-PRS:

  - Option 1: Provided by higher layers with values TBD

- With regards to the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

- Note 1: Other modifications and/or other options within each modification are not precluded

- Note 2: Companies are encouraged to comment whether any of the above modifications can also be used for shared resource pool

Companies views

| Feature Lead | The intention of this proposal is to start writing down a few categories where changes are envisioned over the legacy sensing procedure, together with corresponding potential modification options. Please suggest additional categories of potential that i may have missed, and comment whether the options are clear or further clarification/rewording is needed. |
| --- | --- |
| CATT | Our preference as follows: - With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support: both Option 1and Option 2 - With regards to the resource selection window: Option 2: - With regards to the SL-PRS priority: Option 2 - With regards to the definition of a candidate resource within the resource selection window: Option 1 - With regards to the reservation interval of SL-PRS: Option 1 - With regards to the sensing window length (T0): Option 1 |
| xiaomi | From our opinon, different S-RSRP threshold can be defined for SL PRS resources which is FDM/TDMed with reserved SL PRS resource or comb-FDMed with reserved SL PRS resource. Comb based FDMed SL PRS resource may have higher interference due to IBE, etc. and thus the selection of these resource can be deprioritized compared to FDM/TDM resource. Therefore, we suggest to add a bullet - With regards to the S-RSRP threshold - Option 1: additional S-RSRP threshold for candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset |
| OPPO | Propose to remove the bullet on SL PRS priority, as how may prioirity levels for SL PRS is determined by other working group, and the number of priority levels may not impact the sensing based resource selection procedure. As to Note 2, in our view, none of above can be used for sahred resource pool, to ensure back-ward compatibility, everyting should be the same as legacy in shared resource pool. |
| Qualcomm | We generally agree with the proposal. The definition of candidate resource should also include any other channels in the pool. Comb-offset does not need to be considered when comb-based multiplexing is not enabled. - With regards to the definition of a candidate resource within the resource selection window: - Option 1: The resource elements according to the {OFDM symbols, comb-offsets if comb-based multiplexing is enabled} occupied by a SL-PRS resource and associated channel(s) or SL-PRS resource set and associated channel(s) |
| InterDigital | We support the proposal and prefer the options as discussed below. For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following modifications: - With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support: - Option 1: SL-PRS - Option 2: PSCCH DMRS - With regards to the resource selection window: - Option 2: the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter) - With regards to the SL-PRS priority: - Option 2: SL-PRS transmissions can have different priority in a resource pool - We think the candidate resource (time domain granularity) should be studied after we reach a concludtion as discussed in 3.3.4. the SL-PRS candidate resource can be a single-slot or single-sub-slot resource. Also, we think we should discuss the definition and configuration PRS resource set included in Option 1. With regards to the definition of a candidate resource within the resource selection window: - Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set - With regards to the reservation interval of SL-PRS: - Option 1: Provided by higher layers with values TBD - With regards to the sensing window length (T0): - Option 2: Equal to or larger than the largest reservation interval |
| Spreadtrum | With regards to the RS used to derive L1 SL-RSRP for resource exclusion, we prefer Option 1. With regards to the resource selection window, we prefer Option 2. With regards to the SL-PRS priority, we prefer Option 1. With regards to the definition of a candidate resource within the resource selection window, we support Option 1. With regards to the reservation interval of SL-PRS, we support that it provided by higher layers with values TBD With regards to the sensing window length (T_0), we prefer Option 2. |
| Samsung | If the intention of this proposal is only to list possible modificiations rather than make down-selection now, I’m OK with the proposal. |
| Intel | In our understanding no modification should be done for the resource allocation in a shared resource pool as this would influence rlegacy behavior. We would like to clarify that we would like to avoid defining behavior for pre-emption for dedicated resource pools. |
| ZTE | With regards to the sensing window length, we support Option 2. In order to make sure at least one occasion appears in a sensing window and avoid potential resource collision, sensing window size for SL-PRS should be equal to or larger than the reservation interval. For example, if the maximum reservation interval of SL-PRS is 10240ms, sensing window size can be set to 10240ms. |
| Lenovo | Ok with FL’s proposal as a starting point. |
| Huawei, HiSilicon | - With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support: at least Option 2 for dedicated RP; and Option 2 for shared RP. - For SL-PRS based SL-RSRP, one question from our side is which sequence ID of the SL-PRS the sensing UE should use when deriving the RSRP measurement. - With regards to the resource selection window: Option 1 - With regards to the SL-PRS priority: Option 2 - With regards to the definition of a candidate resource within the resource selection window: Option 1 for dedicated RP. For shared resource pool of SL-PRS with PSSCH transmission in the same slot, no need to define any candidate resource for SL-PRS as it is covered by PSSCH candidate resource anyhow; for shared resource pool of SL-PRS without PSSCH in the same slot, option 1 can also be supported. - With regards to the reservation interval of SL-PRS: Option 1. - With regards to the sensing window length (T0): Option 1 |
| Toyota | Support FL’s proposal. |
| LGE | Basically SL positioning is one kind of of the SL operations. So we prefer to reuse the legacy SL mode-2 resource allocation. In this regard, we suggest the following comments/modifications. 1st bullet: We support PSCCH/PSSCH DMRS RSRP for sensing if they are transmitted together with SL PRS in a slot. This is aligned with the legacy operation. If we agree to the transmission of SL PRS only in a slot, then SL PRS RSRP can be used for sensing. 2nd bullet: We support Option 1 based on the legacy concept of the delay budget. 3rd bullet: We support Option 2. SL PRS priority can be different from each other, and the priority can be determined based on the priority of the associated SL positioning service. 4th bullet: We support FL proposal with comment. The resource allocation granularity should be a time/freq. resource of SL PRS resource, which is included in the SL PRS resource configuration in a resource pool. We suggest the following clarification. - With regards to the definition of a candidate resource within the resource selection window: - Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set, which is (pre-)configured in a resource pool. 5th bullet: We understand the proposal is aligned with SL mode-2 operation. UE’s higher layer can provide the periodicity to MAC layer. Then MAC layer indicates it as the resource reservation interval in SCI. We suggest to add the following clarification. - With regards to the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD 6th bullet: We support Option 1. We see no reason not to follow the legacy SL mode-2 operation. |
| Continental Automotive | Our preference is as follows (colored): - With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support: - Option 1: SL-PRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - With regards to the resource selection window: no preference - With regards to the SL-PRS priority: - Option 2: SL-PRS transmissions can have different priority in a resource pool - With regards to the sensing window length (T0): Consider a third option as below - Option 3: Provided by higher layers with values TBD |
| Futurewei | OK. - The RS used to derive L1 SL-RSRP for resource exclusion, support: both Option 1 and Option 2 - The resource selection window: Option 1 - The SL-PRS priority: Option 1 - The definition of a candidate resource within the resource selection window: Option 1 - The reservation interval of SL-PRS: Option 1 The sensing window length (T0): Option 1 |
| Apple | OK - With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support: Option 2: PSCCH DMRS - With regards to the resource selection window: Option 1: for the derivation of the window, using the legacy approach as a starting point, substitude the Packet Delay Budget (PDB) with a new delay budget - With regards to the SL-PRS priority:Option 2: SL-PRS transmissions can have different priority in a resource pool - With regards to the sensing window length (T0): Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) |
| Nokia, NSB | Support; with preferences resource selection window: Option 1 SL-PRS priority: Option 2 Sensing window length: Option 2 (with minimum value e.g. 100 ms if periodic reservations are not allowed in a pool). |
| Ericsson | Ok with the proposal, as it will help further discussion next meeting. |

[HIGH] Feature Lead Proposal 3.5.2.B-v1

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following modifications:

- With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support:

      - Option 1: SL-PRS

      - Option 2: PSCCH DMRS

      - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- With regards to the resource selection window:

      - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitude the Packet Delay Budget (PDB) with a new delay budget

      - Option 2: the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter)

- [With regards to the SL-PRS priority:

      - Option 1: A single L1 SL-PRS priority allowed in for each resource pool

      - Option 2: SL-PRS transmissions can have different priority in a resource pool]

- With regards to the definition of a candidate resource within the resource selection window:

      - FFS: potential options

      - Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set

- With regards to the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD

- With regards to the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- With regards to the S-RSRP threshold

  - Option 1: additional S-RSRP threshold for candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset

- Note 1: Other modifications and/or other options within each modification are not precluded

- Note 2: Companies are encouraged to comment whether any of the above modifications can also be used for shared resource pool

Companies views

| Feature Lead | There is NO intention to downselect in this meeting. Please only comment if your preferred options are listed and especially if your preferred potential modification areas are listed. This will leave time for companies to think about them for the next meeting. |
| --- | --- |
| LGE | Regarding the candidate resource, we prefer the original wording. The SL PRS resource should be a granularity for resource selection. - With regards to the definition of a candidate resource within the resource selection window: - FFS: potential options - Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set Regarding the RSRP threshold, we suggest that RSRP threshold stepsize for increment and resource ratio X% are separately configured for SL positioning. Regarding Option 1, if it is intended to allow some overlapping between candidate resources and the reserved resources, we need to determind how much overlap is allowed. We suggest to add this point. The overlap may happen in RE level not PRB level. - With regards to the S-RSRP threshold & stepsize, target resource ratio X(%) - S-RSRP threshold & stepsize, target resource ratio X(%) for SL PRS resource (re)selection are (separately pre-)configured from those for SL data resource (re)selection. - Option 1: additional S-RSRP threshold and the level of overlap for candidate SL-PRS resource which occupies overlapping REs PRB with reserved resource but with different comb offset. Finally, we also need to consider the resource (re)selection and pre-emption procedure. We suggest to add a new bullet. - With regards to pre-emption of the reserved resources: - Pre-emption priority for SL PRS resource (re)selection is (separately pre-)configured from that for SL data resource (re)selection. |
| OPPO | We propose to change the main bullet to “consider at least the following potential modifications”, as the list is for companies to further think about, it does not mean at least one option of each modification would be adopted. |
| Continental Automotive | We are OK with the proposal. |
| xiaomi | To LG: Regarding the RSRP threshold, the intention of option 1 is to set two thresholds, one for RE level overlapping and the other is for RB level overlapping without RE level overlapping. The idea is that except excluding candidate resource with RE level overlapping, we also need to preclude comb FDMed candidate resource if the interference is strong enough. |
| ZTE | Ok to list all the options for now. |
| Ericsson | Ok with the proposal, as it will help further discussion next meeting. |
| Intel | We are fine with the proposal. We would like to mention that discussing the RSRP thresholds would require a decision on how SL-PRS resources are defined. |
| Qualcomm | For the selection window, we’d like to clarify the dependence part. Our intention was that higher layers provide the beginning and end of the window. Higher layers could still take a delay budget or a minimum window size into account. To avoid confusion, we’d like to update as follows: - Option 2: the start and end of the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter) Our understanding is that the reservation interval is still signalled from a (pre-)configured set: - With regards to the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. We would like to note that the provided RSRP thresholds are all initial values, there’s a 3dB increment procedure. We’d like to update to clarify that aspect. We also would like to add the option to use the same initial RSRP threshold regardless of comb. - With regards to the initial S-RSRP threshold - Option 1: additional initial S-RSRP threshold for candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset - Option 2: the same initial S-RSRP threshold is used regardless whether a candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset |
| Nokia, NSB | OK |
| InterDigital | We agree with the proposal. |
| Lenovo | Support |

[HIGH] Feature Lead Proposal 3.5.2.B-v2

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- With regards to the RS used to derive L1 SL-RSRP for resource exclusion, support:

      - Option 1: SL-PRS

      - Option 2: PSCCH DMRS

      - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- With regards to the resource selection window:

      - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

      - Option 2: the start and end of the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter)

- With regards to the SL-PRS priority:

      - Option 1: A single L1 SL-PRS priority allowed in for each resource pool

      - Option 2: SL-PRS transmissions can have different priority in a resource pool

- With regards to the definition of a candidate resource within the resource selection window:

      - [Option 1: The resource elements according to the {OFDM symbols, comb-offsets} occupied by a SL-PRS resource or SL-PRS resource set]

      - FFS: other potential options

- With regards to the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- With regards to the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- With regards to the initial S-RSRP threshold

  - Option 1: additional initial S-RSRP threshold for candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset

  - Option 2: the same initial S-RSRP threshold is used regardless whether a candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset

- With regards to the S-RSRP threshold & stepsize, target resource ratio X(%)

  - [S-RSRP threshold & stepsize, target resource ratio X(%) for SL PRS resource (re)selection are (separately pre-)configured from those for SL data resource (re)selection.]

  - Option 1: additional S-RSRP threshold and the level of overlap for candidate SL-PRS resource which occupies overlapping REs PRB with reserved resource but with different comb offset.

- With regards to pre-emption of the reserved resources:

  - Pre-emption priority for SL PRS resource (re)selection is (separately pre-)configured from that for SL data resource (re)selection.

- Note 1: Other modifications and/or other options within each modification are not precluded

Companies views

| Feature Lead | There is NO intention to downselect in this meeting. Please only comment if your preferred options are listed and especially if your preferred potential modification areas are listed. This will leave time for companies to think about them for the next meeting. |
| --- | --- |
| Vivo | We prefer to remove the fourth and last three sub-bullet to discuss in the next meeting since it is not premature to agree with those bullets |
| Huawei, HiSilicon | Ok to list the options for further discussion in the next meeting. Note that in our understanding some bullets are not clear about the meaning of ‘same’ or ‘different’, is it comparing to other UEs in the dedicated pool or comparing to the values defined for sidelink communication. |
| CATT | OK with the proposal. We can down select the options at next meeting. |
| OPPO | Fine with the proposal in general, but the last 2 options need further clarification: For stepsize and X%, no data is transmitted in the dedicated RP, how to understand “separately (pre-)configured”? For affiliated Option 1, how to understand “occupies overlapping REs PRB with reserved resource but with different comb offset”. Similarly, no data is transmitted in dedicated RP, how to understand separately (pre-)configuring pre-emptin priority? |
| LGE | Two bullets (7th and 8th) regarding S-RSRP threshold assumes that different comb sizes are allowed in a same slot, which is not agreed yet from our understanding. There will be no overlapping issue if only a single (M,N) is allowed in a resource pool. We suggest to add this point for each bullet in proposal. Both 1st sub-bullet of 7th bullet and the 2nd sub-bullet of the 8th bullet describes the sam S-RSRP threshold. FL seems to intend to differentiate between them via ‘initial’ or not, but there is no separate S-RSRP threshold. S-RSRP threshold will just consist of the initial value and the step size. We suggest to merge both 7th and 8th bullets together to for simplicity by avoding such overlaps. We suggest the following merged text. - With regards to the initial S-RSRP threshold & stepsize, target resource ratio X(%) - Initial S-RSRP threshold & stepsize, target resource ratio X(%) for SL PRS resource (re)selection are separately (pre-)configured from those for SL data resource (re)selection. - If multiplexing SL PRS resources with different comb sizes in a slot is supported - Option 1: additional initial S-RSRP threshold, step size and the level of overlap are (pre-)configured for candidate SL-PRS resource which occupies overlapping REs PRB with reserved resource but with different comb size offset. - Option 2: the same initial S-RSRP threshold & step size for SL PRS resource (re)selection are is used regardless of whether a candidate SL-PRS resource which occupies overlapping REs PRB with reserved resource but with different comb size offset |
| Samsung | Direction seems OK for this proposal (i.e. listing the potential modifications for Scheme 2 procedure). However, the current proposal seems too lengthy. Considering the additional comments above, it maybe difficult to address a small controveral point if any. So, to make a progress how about treat by one-by-one? |
| ZTE | With regards to selection window, we prefer to delete “start and and of the” in option 2. In current SL communication, selection window is (n+T1, n+T2)and the T1 is not indicated by higher layer but up to UE implementation based on SCS. - With regards to the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the start and end of the selection window is provided by higher layers (no direct dependence on the packet delay budget, or T2,min parameter) Regarding the S-RSRP and pre-emption bullet, why comb offset is related to S-RSRP threshold? We do not see any benefit doing so and suggest to put an FFS for those three bullets. |
| Xiaomi | We are fine with the proposal. |
| ZTE2 | For sensing window length, we would like to emphasize again for the motivation of option 2, which is to ensure there is one PRS occasion in the sensing window. But the window length can still be provided by higher layer. For example, if the PRS periodicity is 200ms, but the sensing window length is configured with 100ms, it is most likely UE cannot sense other UE’s PRS transmission in the sensing window which may cause resource collision in the selection window. - With regards to the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - The values can still be provided by higher layers - Option 3: Provided by higher layers with values TB |
| Panasonic | Ok for the listed candidates |
| Continental Automotive | We are generally fine with the proposal, especially since the note mentions that other options are not precluded; we have a similar comment like Samsung that perhaps later it is better to split each item. |
| InterDigital | We agree with the proposal. |
| Qualcomm | We agree with LGE that “With regards to the S-RSRP threshold & stepsize, target resource ratio X(%)” also needs “initial” A resource in sidelink is a time-frequency resource. The following bullet is missing the frequency aspect and we propose to add that: - With regards to the definition of a candidate resource within the resource selection window: - [Option 1: The resource elements according to the {OFDM symbols, comb-offsets, allocated bandwidth} occupied by a SL-PRS resource or SL-PRS resource set] - FFS: other potential options In reply to ZTE, For request-based preferred resources introduced in Rel-17, the requestor can provide the start and end of the resource selection window. Processing timelines still need to be satisfied. We prefer to keep “start and end” for clarity of proposal, but can accept the change from ZTE if that’s the majority view. |
| Intel | We are in general fine with the direction of the proposal. Similar to Samsungs understanding we could have a general agreement on Rel-16 procedure as a starting point followed by a topic by topic discussion discussion of the modifications. This does make the discussion focues on the specific points. |
| Nokia, NSB | OK |
| Spreadtrum | We are fine with the proposal. |

#### 3.5.3 Congestion control

| Nokia, NSB | Proposal 19: UEs involved in SL positioning may exchange information on the channel congestion they observe, so as to adapt their SL PRS configuration, e.g., SL PRS bandwidth depending on their relative congestion status. |
| --- | --- |
| Futurewei | Proposal 12: For SL-PRS resource allocation in shared resource pools support the congestion control in sidelink resource allocation Scheme 2. |
| Huawei, HiSilicon | Proposal 27: Congestion control is supported for SL-PRS transmission. Proposal 28: Reuse the definition of CR and CBR for SL communication, but change the subchannel to SL-PRS resource. |
| vivo | The number of RE(s)/comb patterns can be used in the calculation CR/CBR for SL-PRS at least in the dedicated resource pool. The resource of SL-PRS transmitted by anchor UE can be calculated in the CR/CBR of the target UE at least in the unicast scenario. |
| OPPO | Proposal 6: Use the following principles for congestion control in SL positioning: - Congestion control processing time is same as SL communication; - Up to 16 CBR ranges as SL communication are defined; - CBR measurement time window is [100ms or 100 slots]; - CR measurement time window is [1000ms or 1000 slots]; - In shared resource pool, the definition of CBR and CR are same as SL communication; - In dedicated resource pool, the definition of CBR and CR should be adapted to the definition of SL PRS resource. |
| CATT, GOHIGH | Proposal 29: Congestion control mechanism for SL-PRS should be studied after the resource allocation mechanism of SL-PRS was determined. |
| LGE | Proposal 27: SL PRS resource is the granularity of RSSI measurement for CBR estimation in SL positioning. Proposal 28: CBR estimation for SL positioning can be based on the followings. - RSSI measurement on SL PRS - RSSI measurement on PSCCH/PSSCH Proposal 29: RSSI measurement for CBR estimation in SL positioning needs to be improved when SL PRS and PSCCH/PSSCH are transmitted in a same resource pool. |
| Samsung | Proposal 8: In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. - The priority value for SL PRS is provided by high layer of UE - If the priority of SL PRS and PSSCH is different, max function can be used. - CBR measurement for SL PRS can be reported to gNB. - Congestion control for SL PRS can restrict CR limit of SL PRS by CBR and priority. - FFS: The definition of CBR/RSSI/CR for SL positioning. Proposal 9: When determining the channels of a radio access technology to receive or transmit and when an overlap in time occurs, and one of the overlapped signals is SL PRS, the priority of SL PRS is considered in the prioritization. |
| ZTE | Proposal 16: For congestion control mechanism in shared resource pool: - Either a new SL-PRS RSSI is introduced or existing SL RSSI is reused with slight modification - The definition of CR and CBR should reuse the legacy definition in Rel-16/17 SL communication. - CR can be evaluated separately for SL-PRS and PSSCH Proposal 17: For congestion control mechanisms in dedicated resource pool: - SL-PRS CR and SL-PRS CBR should be based on - Alt.1: PSCCH which is associated with SL-PRS transmission - Alt.2: SL-PRS - Alt.3: both SL-PRS and PSCCH which is associated with SL-PRS transmission - The definition of SL-PRS CBR and CR is associated with SL-PRS resource allocation granularity and the SL-PRS resource configuration in a SL-PRS resource pool. - For comb-based multiplexing, SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and comb size. - For TDM-based multiplexing, the definition and configuration of SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and the number of symbols for each SL-PRS resource. - There are mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters. |
| Interdigital | Proposal 26: Introduce a CBR measurement for SL-PRS transmission. Proposal 27: Study separate CR measurement for SL-PRS and PSSCH transmission in a shared resource pool. |
| Apple | Proposal 14: For Scheme 2 SL-PRS resource allocation, support both congestion control and inter-UE coordination. |
| Ericsson | - For the dedicated pool, redefine CBR and CR based on the granularity of the resource reservation. - To adjust the load of the transmission on a pool the UE can switch the resource reservation between the pools |
| Qualcomm | Proposal 21: Congestion control is introduced for transmissions with SL-PRS and the existing congestion mechanism from sidelink communications is reused with updates if necessary. |

[CLOSED] Feature Lead Proposal 3.5.3-v0

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. Study at least the following aspects:

- CBR and CR definition for SL-PRS

- Which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

- CR and CBR measurement time window

- Congestion control processing time

- Number of CBR ranges

Companies views

| CATT | In our point of view, congestion control mechanism for SL-PRS should be studied in order to maintain the congestion of SL-PRS within a reasonable level. However, since the definitions of CBR and CR are closely related to resource occupation, we prefer congestion control mechanism for SL-PRS should be studied after the resource allocation mechanism of SL-PRS was determined. |
| --- | --- |
| vivo | The CBR and CR definition for SL-PRS should be defined as TDM-based multiplexing and comb-based multiplexing has been supported in the dedicated resource pool at least. The SL-PRS resource can be used in CBR/CR calculation. |
| Qualcomm | We support the proposal. Congestion control is important to ensure good system performance in a distributed system. |
| InterDigital | We support the proposal. |
| Samsung | OK. We think this is high priority issue since congestion control is very important for positioning performance. |
| Intel | Agree, in our understanding this does only target dedicated resource pools. Thus, we would like to add this to the main bullet. |
| ZTE | Support in general. Agree with Intel’s comment that this proposal should be intended for dedicated resource pool. Moreover, there are relationships between SCI and its associated SL-PRS resource(s) (e.g. one SCI resource/config is associated with one SL-PRS resource). In our understanding, SL-PRS CR and SL-PRS CBR can be based on either PSCCH/SCI or SL-PRS, or both PSCCH/SCI and SL-PRS. Take an example in our contribution, Suppose there is only 4 SCI configurations and associated 4 SL-PRS resources in a slot, CR and CBR can be either calculated based on how many SCI configuration are occupied/busy or how many SL-PRS resources are occupied/busy. Therefore we suggest the following update: For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. Study at least the following aspects: - CBR and CR definition for SL-PRS - SL-PRS CR and SL-PRS CBR can be based on either PSCCH/SCI or SL-PRS, or both PSCCH/SCI and SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges |
| Huawei, HiSilicon | Support |
| Toyota | Support FL’s proposal. |
| Continental Automotive | We support the proposal with the following addition: - Study definitions for SL positioning CBR and CR for both dedicated and shared RPs |
| Futurewei | Support FL’s proposal. |
| Apple | Support |
| Nokia, NSB | Support. |
| LGE | Support. This is for study and the details can be discussed later. |

There is good support, with the comments mainly be related to additional details. My suggestion is companies to hold on those details for later, since we are still missing a generic agreement that congestion control is supported.

[CLOSED] Feature Lead Proposal 3.5.3-v1

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point.

- Study at least the following aspects on potential changes over the existing congestion control mechanisms:

  - CBR and CR definition for SL-PRS

  - Which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

  - CR and CBR measurement time window

  - Congestion control processing time

  - Number of CBR ranges

  - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Companies views

| OPPO | OK |
| --- | --- |
| CATT | OK with the proposal. |
| LGE | Support |
| Continental Automotive | Support the proposal. |
| Xiaomi | OK |
| Samsung | Support |
| ZTE | OK. We may discuss details later as per FL’s suggestion. |
| Spreadtrum | Support |
| Intel | Support |
| Toyota | Support |
| Qualcomm | Support |
| Nokia, NSB | OK |
| InterDigital | We support the proposal. |
| Lenovo | Support |

#### 3.5.4 Inter-UE coordination for SL-PRS

| Nokia, NSB | Proposal 10: For code-domain multiplexing (CDM) of SL PRS transmissions, study IUC like mechanism to determine if SL PRS sequence multiplexing is acceptable at the receiver UE. Proposal 11: For frequency-domain multiplexing of SL PRS transmissions, study IUC like mechanism to determine if comb-based multiplexing of SL PRS is acceptable at the receiver UE. Proposal 14: Study SL PRS resource conflict avoidance/resolution for IUC scenarios involving a UE transmitting (multicast) SL PRS to multiple UEs based on IUC assistance received from them. Proposal 16: A UE (e.g., target UE) may assist configuring parameters of SL PRS (e.g., bandwidth, periodicity, etc.) for other UEs (e.g., anchor UEs) such as by providing (non-)preferred set of SL PRS parameters. |
| --- | --- |
| Futurewei | Proposal 10: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission. Proposal 11: Use the IUC Scheme 2 to provide the preferred resources for multiple SL-PRS same slot multiplexing. |
| Huawei, HiSilicon | Proposal 22: Only Rel-17 inter-UE coordination Scheme 1 is considered for the support of SL-PRS resource transmission. - The inter-UE coordination signaling will be indicated in SCI for both shared and dedicated resource pool. Proposal 26: To enable the inter-UE coordination Scheme-1 mechanism for SL-PRS transmission, the SCI can include: - Providing/Requesting indicator in SCI - Allocated SL-PRS resource information when the Providing/Requesting indicator indicates ‘Providing’ - The request for SL-PRS transmission from the responding UE when the Providing/Requesting indicator indicates ‘Requesting’. Proposal 24: For DL-like SL-TDoA, the inter-UE coordination mechanism can be used to indicate the associated SL-PRS transmission among different UEs e.g. - The target UE can allocate/recommend the associated SL-PRS resources for the responding UE within the reserved period. Proposal 25: For RTT-type positioning, the inter-UE coordination mechanism can be used to indicate the associated SL-PRS transmission among different UEs, e.g. - The transmitting UE can reserve the next associated SL-PRS resources for itself within the reserved period; and - The transmitting UE can request the responding UE to transmit SL-PRS based on the responding UE sensing. |
| vivo | - The preferred resource sets and non-preferred resource sets can be considered as the IUC information in scheme 2 SL-PRS resource allocation. |
| OPPO | Proposal 7: IUC scheme 1 should be supported for SL PRS resource selection at least when SL PRS is transmitted within the shared resource pool with SL communication. FFS for dedicated SL PRS resource pool. Proposal 8: IUC scheme 2 should be applied to SL PRS resource selection in shared resource pool and dedicated resource pool for SL positioning. Proposal 9: Conflict indication for the already used SL PRS resource(s) should be supported for SL positioning. |
| Spreadtrum | Proposal 13: Inter-UE coordination (IUC) in Mode 2 should be supported for SL-PRS resource allocation. |
| CATT, GOHIGH | Proposal 26: In order to solve the issue of SL-PRS resource confliction, the inter-UE coordination mechanism for SL-PRS should be introduced in the dedicated resource pool. Proposal 27: Inter-UE coordination for SL-PRS in the dedicated resource pool should reuse the existing inter-UE coordination mechanism for PSSCH in Rel-17 NR sidelink enhancements as much as possible to reduce the workload of standardization and the complexity of UE implementation. Proposal 28: Existing inter-UE coordination Scheme 1 and Scheme 2 for PSSCH in Rel-17 NR sidelink enhancements should be the starting point for inter-UE coordination for SL-PRS in the dedicated resource pool. |
| Xiaomi | Proposal 12: IUC message from a UE to the other UE to assist SL PRS resource allocation shall be supported. |
| LGE | Proposal 22: The resource selection based on the inter-UE coordination message defined in Rel.17 SL is supported for SL PRS resource selection. Proposal 23: In IUC-based SL PRS resource selection for SL TDOA positioning, the handling of the multiple IUC messages are needed. |
| lenovo | Proposal 5: RAN1 to further consider IUC scheme 1 and 2 for dedicated SL-PRS resource pool and study details. Proposal 6: RAN1 to support enhanced resource allocation/coordination schemes for a SL Positioning Group that are based on the existing Scheme 1 and 2 procedures, e.g., resource configuration sharing by a member UE with other member UE(s), sensing information coordination, inter-UE coordination by member UE with other member UE(s), server UE coordinating SL-PRS resources for other UEs within a SL positioning group. |
| ZTE | Proposal 18: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts. - Reuse the Rel-17 Scheme 1 and 2 IUC procedure - Do not support using IUC scheme to allocate SL-PRS resource(s) |
| CEWiT | Proposal 6: Support the exchange of assistance data (AD) for the selection of preferred resources or the exclusion of resources. |
| Interdigital | Proposal 20: A transmitting UE selects SL-PRS resources in a received resource set based on IUC Scheme 1. Proposal 21: Support conflict indication for SL-PRS resource reservation based on IUC Scheme 2. |
| Apple | Propsal 13: For Rel-17 type sensing, the IUC schemes supported will be determined by the channels can be included in the resource pool in addition to SL-PRS in the dedicated resource pool: • IUC Scheme 1 in which the set of preferred and/or non-preferred resources are indicated, requires that a PSSCH is present in the dedicated resource pool for the IUC message contents. • IUC Scheme 2 in which the presence of expected/potential resource conflict is signaled, requires the presence of a PSFCH in the dedicated resource pool for the IUC message contents. This is directly related to the decision on Option 3 : PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included. Proposal 14: For Scheme 2 SL-PRS resource allocation, support both congestion control and inter-UE coordination. |
| Qualcomm | Proposal 12: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete. |
| NEC | Proposal 8: Both Scheme 1 IUC procedure and Scheme 2 IUC procedure can be considered as the starting point for SL-PRS transmission on at least dedicated resource pool for positioning. Proposal 9: The IUC procedure could be applied based on necessary enhancement on the positioning related indication. Proposal 10: The SL PRS resource conflict indication should be considered, and corresponding conflict avoidance/solution mechanism should be studied. |

It seems there is strong majority to introduce IUC signaling for SL-PRS. A few companies seem to not differentialte between shared and dedicated resource pool, and others are not clear whether their proposals are for both.

| Use IUC scheme 1 as starting point (preferred/non-preferred resources) | Huawei, HiSilicon, vivo, Futurewei |
| --- | --- |
| Use IUC scheme 2 as starting point (future conflicts) |  |
| Both IUC scheme 1 & 2 are starting point | OPPO, CATT, GOHIGH, Lenovo, ZTE, Interdigital, NEC |
| Deprioritize IUC for SL-PRS | Qualcomm |
| Support also conflict indication for the already used SL PRS resource(s) | OPPO |

Seems there is some majority to use both IUC schemes as a starting point, and a few companies suggesting to only use Scheme 1. So, lets start the discussion from the following:

[CLOSED] Feature Lead Proposal 3.5.4-v0

For Scheme 2, support inter-UE coordination signaling for SL-PRS

- Consider the following options:

  - Option 1: Use Rel-17 Scheme 1 IUC procedure as a starting point

  - Option 2: Use both Rel-17 Scheme 1 or and 2  IUC procedure as a starting point

- Note: Companies are encouraged to describe the applicability of their proposals with regards to dedicated and shared resource pool, and the changes that may be needed

Companies views

| CATT | Support. |
| --- | --- |
| vivo | We prefer option 1. |
| Xiaomi | Option 1 |
| Qualcomm | Not support We do not support considering IUC in the dedicated resource pool at this stage. In our view, basic system functionality should be relatively complete before considering enhancements. For the shared resource pool, we are ok reusing the Rel-17 inter-UE coordination since it would not require changes. |
| InterDigital | We support the proposal and prefer Option 2. Note in a shared pool, since Scheme 2 is already supported for PSSCH and thus PSSCH+SL-PRS transmissions, we think it can be supported for SL-PRS only transmission in a shared pool too. In a dedicated resource pool, we think it is beneficial to support PSFCH for both SL-PRS feedback and Scheme 2 IUC to improve SL-PRS transmission reliability. Note these are the features that are applied to improve PSSCH/PSCCH transmission reliability in Mode 2 and considering SL-PRS transmissions in Scheme 2 can have similar reliability requiremend (if not more stringent), such mechanisms should be enabled for SL-PRS transmissions too. |
| NEC | We prefer Option 2. |
| Panasonic | Option 2 |
| DCM | Option 2 |
| CMCC | Option 1. |
| Intel | Disagree, we would like to progress scheme-2 before discussing IUC. During Rel-17 our extenstive simulations of scheme 1 showed that in many cases this is not beneficial for overall system performance. This means that for scheme-2 if comparing the performance with IUC scheme-1 or directly allocating the resource used for IUC to the dedicated pool it is in our understanding better to allocated more resource to the dedicated pool instead of having IUC in the communications resource pool. In the case of dedicated resource pools this also creates a couping of IUC resource in the resource pool used for communication and the SL-PRS resource in the dedicated resource pool. |
| Huawei, HiSilicon | We prefer Option 1. For IUC Scheme 1, it can help the reception UE to select SL-PRS transmission resource. While, for IUC scheme 2, different with data transmission, the SL-PRS can be robust to potential interference. We see less benefits or necessity to use scheme 2. |
| Futurewei | OK. We support Option 1 but we could live with Option 2 also. |
| Apple | Support for a shared pool. For a dedicated pool, to support IUC,will need to support PSSCH and/or PSFCH. |
| Nokia, NSB | Support in principle; prefer Option 2 But it seems a bit premature to agree on this, the basic resource allocation should be defined first. |
| LGE | We’re generally fine with IUC for SL positioning, but similar to partial sensing case, we have a concern on the workload caused by IUC. We suggest at least deprioritize this issue in Rel.18. |

Support proposal as is

- CATT, Apple (for shared RP)

Support with Option 1:

- Vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei

Support with Option 2:

- Interdigital, cNEC, Panasonic, DCM

Do not support / deprioritize / wait

- Qualcomm, Intel, LGE, Nokia, NSB

Overall, there is majority support to support IUC:

- In favor: CATT, Apple, vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei, Interdigital, NEC, Panasonic, DCM (11)

- Against: Qualcomm, Intel, LGE, Nokia, NSB (5)

[CLOSED] Feature Lead Proposal 3.5.4-v0

For Scheme 2, support inter-UE coordination signaling for SL-PRS

- Consider the following options:

  - Option 1: Use Rel-17 Scheme 1 IUC procedure as a starting point

  - Option 2: Use both Rel-17 Scheme 1 or and 2  IUC procedure as a starting point

- Note: Companies are encouraged to describe the applicability of their proposals with regards to dedicated and shared resource pool, and the changes that may be needed

| Feature Lead | Companies are encouraged to see the status and are kindly asked to reconsider if they can be flexible: Current status Support proposal as is - CATT, Apple (for shared RP) Support with Option 1: - Vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei Support with Option 2: - Interdigital, NEC, Panasonic, DCM Do not support / deprioritize / wait - Qualcomm, Intel, LGE, Nokia, NSB Overall, there is majority support to support IUC: - In favor: CATT, Apple, vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei, Interdigital, NEC, Panasonic, DCM (11) - Against: Qualcomm, Intel, LGE, Nokia, NSB (5) |
| --- | --- |
| CATT | Support. We prefer Option 2. |
| LGE | We still prefer not to discuss IUC in Rel.18 SL positioning or at least deprioritize it. But if group decides to adopt IUC, we think Option 2 should be supported. In sidelink, for groupcast and broadcast, only IUC scheme 2 is used. As we agreed GC and BC type of SL PRS transmission, it’s reasonable to support both schemes. |
| Continental Automotive | In principle, we support the proposal with Option 2 at least in the shared RP. |
| Samsung | We think it would be better to focus on the use case for IUC rather than selecting option. Since comb-based and TDM-based muiplexing are supported, I think that target UE can provide comb offset and/or time offset to anchor UEs as IUC information. |
| ZTE | We prefer Option 2. but we are ok to postpone the discussion. |
| Spreadtrum | We prefer Option 2. |
| CEWiT | We prefer Option 2. |
| Intel | We still think this topic should be deprioritized. In our understanding this is an optimization and similar to the case of SL communication it is not clear if this is beneficial for the system. |
| Qualcomm | Not support. The existing IUC framework does not work in the dedicated resource pool. It would require cross-resource pool signaling and indication, which is very a difficult problem to solve and was not solved for SL communications. We do not think that RAN1 time would be well spent working on the optimization, while baseline functionality is far from complete, especially in the absence of evaluations. |
| Nokia, NSB | OK in general. We are not against IUC, but feel that it is a bit premature to agree on this since we do not yet have the basic resource allocation defined. |
| InterDigital | We support the proposal. We prefer Option 2. |
| Lenovo | Support and prefer Option 2 for better flexibility. |

Support with Option 1:

- Vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei

Support with Option 2:

- Interdigital, NEC, Panasonic, DCM, CATT, LGE (2nd choice), Continental, ZTE, Spreadtrum, CEWiT, lenovo

Do not support / deprioritize / wait

- Intel, LGE (1st choice), Nokia, NSB, Qualcomm

Do not focus on Option 1 and Option 2 rather on the IUC functionality

  - Samsuing

[HIGH] Feature Lead Proposal 3.5.4-v1

For Scheme 2,

- Alt. 1: support inter-UE coordination signaling for SL-PRS

  - FFS: Details

- Alt. 2: deprioritize it for this release

- Alt. 1: Huawei, HiSilicon, CATT, Samsung, ZTE, Xiaomi, Panasonic, Continental, Interdigital, Nokia, NSB

- Alt. 2: LGE, Qualcomm, Intel

Companies views

| vivo | the inter-UE coordination signaling is unclear for us, we prefer only support option 1 |
| --- | --- |
| Huawei, HiSilicon | Alt.1 we tend to believe it could be a useful mechanism for RTT based positioning. It is true the discussion should build the basic functionalities first. However, we don’t have to explicitly agree to deprioritize it for this release when it is helpful for a better design of the basic functions. |
| CATT | We prefer Alt .1. |
| OPPO | We also prefer to separate the discussion for shared pool and dedicated resource pool. In shared resoruce pool both IUC scheme 1 and IUC scheme 2 should be supported, as this is a low-hanging fruit in our view. For dedicated resource pool, we have not seen too much specification impact to support IUC scheme 2 as long as PSFCH resource is configured. For IUC scheme 1, we agree with QC that cross-resource pool operation is needed, we are ok to deprioritize it in Rel-18. |
| LGE | We can accept Alt. 2. |
| Samsung | Alt 1 As we commented above, we see an useful scenario for IUC to enable comb-based and TDM-based muiplexing of UEs. For example, a target UE can provide comb offset and/or time offset to anchor UEs as IUC information. |
| ZTE | We prefer Alt.1. Regarding the comment from Qualcomm, could you please give us an example? I can not fully understand “require cross-resource pool signaling and indication” for dedicated resouce pool and why this issue does not exist in SL communication IUC. |
| Xiaomi | We prefer Alt 1 if time is allowed. |
| Panasonic | Alt 1 |
| Continental Automotive | We prefer Alt. 1. |
| InterDigital | WE prefer Alt. 1. |
| Qualcomm | Alt 2 @ZTE, the resources indicated in an IUC message are in the same resource pool as the message. Rel-17 sidelink does not support transmitting IUC information in one resource pool with the included resource sets pointing to a different resource pool. That why the cross-resource pool signaling issue is not there in IUC for SL communications. |
| Intel | We support Alt. 2. Considering how much time it took to design IUC during Rel-17 and that in our understanding the benefit and design of cross resource pool IUC is not clear it is in our understanding desirable to deprioritize it for this release and concentrate on the scheme-2 design. |
| Nokia, NSB | We prefer Alt. 1. But we prefer to define the resource allocation for SL PRS first before discussing IUC aspects. |
| Spreadtrum | We prefer Alt. 1. |

#### 3.5.5 UE-A reserving SL-PRS resources for UE-B

| Nokia, NSB | Proposal 12: Study the resource allocation mechanisms that allow one UE to allocate multiple SL PRS transmission opportunities for itself and/or other UE(s) in a SL positioning session. |
| --- | --- |
| Spreadtrum | Proposal 14: UE A can select PRS resources in dedicated resource pool for UE B should be supported. |
| CMCC | Proposal 4: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered as a candidate resource allocation mechanism for SL-PRS in Rel-18. |
| CEWiT | Proposal 5: For operations in scheme 2, support that an UE can suggest (and pre-reserve) SL-PRS resources for other UE(s) (e.g. responding UEs). |
| Interdigital | Proposal 15: Study Scheme 2 sensing enhancements to improve SL-PRS transmission reliability, e.g., prioritizing TDMed candidate resources. Proposal 16: Support a UE to transmit SL-PRS in resources selected and indicated by a peer UE in a position group using SL-PRS resource allocation Scheme 2. Proposal 17: Design a common SL-PRS resource allocation Scheme 2 for stand-alone SL-PRS transmissions in a shared resource pool and SL-PRS transmissions in a dedicated resource pool. Proposal 18: Support a joint resource allocation Scheme 2 for multiplexed SL-PRS and PSSCH/PSCCH transmissions based on legacy Mode 2 sensing procedure. Proposal 19: Confirm WA to allow both sensing-based and random selection in the same resource pool. |
| ITL | Proposal 1: With regards to resource allocation for sidelink RS with Operation Scenario 1(PC5-only-based positioning): - A master UE among multiple anchor Ues should be considered where sidelink RS is being transmitted from multiple anchor Ues to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor Ues (i.e. UL-TDOA-like operation) - The master UE can transmit resource allocation information for sidelink RS (which includes resource allocation information for all of the multiple anchor Ues) to the target UE |
| Mediatek | Proposal 3-2: When a UE reserves the resource for future transmission in SCI, the UE may also reserve the resource for the transmission of another UE. It can be done at least for the case without periodic transmission that, when 3 transmissions are reserved by UE A, one of them could be used for the transmission by UE B. In this way, the double sided SL-RTT could be realized |
| Futurewei | Proposal 9: For sensing-based resource allocation a SL UE may reserve SL-PRS resources for other SL Ues participants in the positioning process. |

[CLOSED] Feature Lead Proposal 3.5.5-v0

For scheme 2,

- Alt. 1: support that a UE can reserve a SL-PRS resource for the transmission of another UE.

- Alt. 2: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE.

Companies views

| CATT | Support. We prefer Alt.1. |
| --- | --- |
| vivo | We support alt. 2. In our opinion, UE 1 determines the reserved resource for SL PRS based on itself. The resource reserved by other UE(e.g., UE 2) is not considering the transmission and reception of UE 1. It would lead to conflict in UE 1 if UE 1 needs to perform reception in the resource reserved by UE 2. Therefore, we prefer the reserved resource of UE 1 is up to itself, and other UE just provides recommend resources to UE 1. |
| Sharp | We support Alt. 1, which we think is crucial for reducing the overall delay of a SL positioning session with multiple anchor Ues involved. |
| InterDigital | We support the proposal and prefer Alt. 1. We think Alt.1. is important to enable low latency required in SL positioning, e.g. in RTT-based solution. |
| NEC | We prefer Alt. 1, since the Xiaomio reservation for another UE is benifical and feasible based on (pre)configuration and coordination among the Ues in a positioning group. |
| Panasonic | Alt 1 |
| Spreadtrum | Support. We prefer Alt. 1. |
| DCM | Alt 2. Channel condition around another UE will not be known at the reserving UE. |
| CMCC | Alt. 1. Alt. 1 is applicable to RTT-based solution using SL, where UE-A can schedules resources for UE-B to further reduce latency and avoid potential resource collision. In addition, Alt. 1 is also applicable to the case when a server UE can schedules SL PRS resources to the anchor/target Ues. |
| Intel | We support Alt. 2. In our understanding Alt. 1 does create an unnecessary lower layer couplingof multiple Ues similar to one UE scheduling other Ues that was extenstively discussed in the past. |
| ZTE | We support Alt.2. Agree with vivo’s understanding, technically speaking it is hard for a UE to be known about all the resource transmission information and resource conflict information (in one positioning session). One UE can recommend some SL-PRS resource or SL-PRS configurations, but it is up to transmission UE (e.g. UE-B) itself to determine how to use this information (e.g. whether directly use the resource recommended by UE-A or combine the IUC with its sensing results). |
| Fraunhofer | Support Alt 1 |
| Lenovo | Support Alt. 1 |
| Huawei, HiSilicon | We prefer Alt.1. For example, for RTT based positioning, typically it needs two Ues take turns to transmit SL-PRS and measure the signal. In order to ensure the RTT functionality for the SL-PRS transmission, the transmitting UE-A should request UE-B for the SL-PRS reverse transmission with the reserved SL-PRS resource of UE-A. |
| Futurewei | Support Alt 1 |
| Apple | Support Alt 1 e.g. for RTT based positioning |
| Nokia, NSB | Support Alt. 1. This can be beneficial e.g. for RTT to reduce latency of the procedure. |
| LGE | Suport Alt. 1. We think that alt. 1 is useful at least for SL RTT, as commented by other companies. |

Summary of the 1st round:

Alt. 1

- CATT, Sharp, Interdigital, NEC, Panasonic, Spreadtrum, CMCC, Fraunhofer, Lenovo, Huawei, HiSilicon, Futurewei, Apple, Nokia, NSB, LGE (15)

Alt. 2

- Vivo, DCM, Intel, ZTE (4)

FL comment: There is strong majority for Alt. 1 More companies are encouraged to provide their views whether they can accept Alt. 1

[CLOSED] Feature Lead Proposal 3.5.5-v1

For scheme 2,

- Alt. 1: support that a UE can reserve a SL-PRS resource for the transmission of another UE.

- Alt. 2: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE.

Companies views

| OPPO | We do not support the proposal, it is against the fundamental principle of sensing-based resource selection. |
| --- | --- |
| CATT | Support |
| LGE | Support |
| Continental Automotive | We support the proposal with the addition of: FFS: details regarding the conditions and signaling mechanism. |
| CMCC | Support. |
| Xiaomi | OK |
| Samsung | In the proposal, is “a UE” the target UE? |
| ZTE | Support |
| Fraunhofer | Support |
| Spreadtrum | Support |
| CEWiT | Support |
| NEC | Support. |
| Intel | We understand the proposal as one UE (UE-1) reserving physical layer SL-PRS resources for another UE (UE-2). For scheme-2 the UE (UE-2) that is receiving the resource reservation from another UE (UE-1) might perceive a totally different interference and SL-PRS resource occupation scenario. To take this into account this would mean that the UE-2 needs to first forward its own understanding of the SL-PRS resource occupation to UE-1 before UE-1 can proceed with the resource reservation. Further, the reserved resource by UE-1 may conflict with some other Tx/Rx occasion at UE-2. So, that needs to be pre-coordinated or additional handling needs to be defined as well. All of these essentially requires additional and significantly complex coordination and a complete handshake of information before the each transmission. To address RTT, the only reasonable option would be to request for a transmission from UE-1 to UE-2 via indication of a time window within which UE-2 is expected to respond. We cannot accept Alt. 1 which is a significantly complex mechanism that was extensively discussed for SL communications and not agreed, that would lead to significant more specification efforts, and fundamentally, introducing hierarchy between two Ues at PHY. |
| Qualcomm | Not support At the beginning of the study item, we supported this idea. However, at this point in the WI, we think it is too late to start. As Intel noted, the amount of required work is extensive and many topics on basic functionality are still open. |
| Nokia, NSB | Support |
| Sharp | We support the proposal. |
| InterDigital | We support the proposal. |
| ZTE2 | Somehow I misread it and sorry for my mistake. We do not support this proposal and agree with Intel’s understanding. |
| Lenovo | Support, we think that SL communication use cases are different from SL positioning which requires more tighter coorindation mechanisms for RA especially in Mode 2. It is also known that for methods such as RTT, there is a guaranteed SL-PRS reply required so this can be coordinated beforehand. The pre-coordination aspect highlighted by an Intel can be a further FFS aspect . |

c

#### 3.5.6 Random Resource selection

Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed

| CATT | Proposal 14: The sensing-based resource allocation and random resource selection of SL-PRS in dedicated resource pool should be modified according to the minimum resource allocation unit of SL-PRS. |
| --- | --- |

## Feedback mechanisms

| Nokia, NSB | Proposal 20: Support ACK/NACK feedback for unicast and groupcast SL PRS transmissions to allow or prevent retransmission of a SL PRS transmission. Proposal 21: Support employing different SL PRSs (e.g., two different cyclic shifts of a SL PRS sequence) from a UE to convey ACK/NACK feedback to trigger retransmission of SL PRS from the corresponding UE or not. Proposal 8: For Scheme 2 resource allocation in shared pools, UE transmitting SL PRS may provide feedback to other transmitting Ues to avoid their resource preemption and reuse the same time/frequency resources via RE-level multiplexing. |
| --- | --- |
| LGE | Proposal 18: SL PRS repetition or retransmission is supported as follows. - Blind repetition - Feedback-based retransmissionc |
| NEC | Proposal 6: The feedback of SL-PRS reception from receiving UE should be considered in either shared resource pool or dedicated resource pool to request the quick SL-PRS retransmission. |

[CLOSED] Feature Lead Proposal 3.6-v0

Study further feedback mechanisms for SL-PRS including:

- ACK/NACK feedback for unicast and groupcast SL PRS transmissions to

- Feedback-based Retransmissions

Companies views

| vivo | In our view, no PSFCH channel in the dedicated resource pool |
| --- | --- |
| Qualcomm | In our view, the measurement report itself can serve as an implicit feedback. |
| InterDigital | We support the proposal to study further SL-PRS feedback. |
| NEC | We support to introduce the feedback mechanism for SL PRS, at least for latency sensitive positioning method, a feedback-based retransmission is benefical and has less impact on legacy spec. |
| Spreadtrum | We support the proposal. |
| Intel | Agree to postpone discussion. |
| ZTE | Do not support introducing feedback mechanism for SL-PRS. |
| Lenovo | Do not support |
| Huawei, HiSilicon | Need more clarification on the benefits of PSFCH based feedback for SL-PRS transmssions and re-transmssion. |
| Futurewei | OK to study, no clear the advantages. |
| SONY | Support. |
| Nokia, NSB | Support to study |
| LGE | We support the feedback-based SL PRS retransmission. Because of imperfect sensing, a feedback mechanism is useful for efficient SL PRS transmission. Measurement report can do a similar role in e.g. SL TDOA, but it doesn’t help for the 1st SL PRS transmission in SL RTT because there is no measurement report only for the 1st SL PRS transmission. |

Support to study further:

- Interdigital, NEC, Spreadtrum, Futurewei, Sony, Nokia, NSB, LGE

Do not support or postpone:

- Vivo, Qualcomm, Intel, ZTE, Lenovo, Huawei, HiSlicon

[CLOSED] Feature Lead Proposal 3.6-v1

Study further feedback mechanisms for SL-PRS including:

- ACK/NACK feedback for unicast and groupcast SL PRS transmissions

- Feedback-based Retransmissions

Companies views

| OPPO | We support feedback based re-transmission, but we do not support ACK/NAC feedback for unicast and groupcast SL PRS transmission, SL PRS has no CRC, ACK/NACK feedback for it is not feasible. |
| --- | --- |
| CATT | Do not support. There is no need to study the SL-PRS HARQ and retransmission mechanism, since the DL-PRS and SRS do not have the HARQ and retransmission mechanism in NR Uu. |
| LGE | We support FL proposal. Sidelink transmission/Xiaomion is different from that in Uu link. In Uu link, every transmission and resource is scheduled by a network so there is much less probability in resource collsion than sidelink. However, the resource collsion happens more frequently in sidelink especially when UE autonomouslty selects resources based on sensing. UE sensing cannot be perfect due to various reasons including e.g. half-duplex or hidden node issue. This causes resource collision in sidelink, and we believe that feedback-based retransmission can provide a smarter way of resource utilization, shortened latency and improved availability/reliability. |
| Continental Automotive | Support in principle, but not sure how ACK/NACK could be implementd for SL PRS. |
| Samsung | Do not support. Considering work load, we do not want feedback mechanism in this release. |
| ZTE | Do not support. |
| NEC | We support the feedback mechanism considering the resource allocation of SL PRS is quite different from that of DL PRS at least for Scheme 2. |
| Intel | We think this is an optimization with unclear benefits. We would like to priotize other aspects of the design before discussing this topic. |
| Nokia, NSB | Support to study. |
| InterDigital | We support the proposal. ##### As discussed in [HIGH] Feature Lead Proposal 3.2.2-v1,we think L1-feedback for SL-PRS transmission will enhance Schme 2 SL-PRS transmission reliability in the same way HARQ does Mode 2 SL communication due to inherent sensing issues. In addition, comb(RE)-level multiplexing between SL-PRS transmissions may increase interferences. Thus, a SL-PRS re-transmission would be useful e.g. based on the RSRP of the previous received SL-PRS. |

Support

- OPPO, LGE, Continental, NEC, Nokia, NSB, Interdigital

Do not support

- CATT, Samsung, ZTE, Intel

# SL-PRS cast types

| OPPO | Proposal 13: “Cast type indicator” and “Destination ID” fields as SL communication should be included in SCI to support unicast, groupcast and broadcast, where the destination ID is provided by higher layer. |
| --- | --- |
| CATT, GOHIGH | Proposal 30: “Cast type indicator”, “Source ID”, “Destination ID” fields (similar to SL communication) are included in SCI to support unicast, groupcast and broadcast SL-PRS transmissions.c |
| ZTE | Proposal 21: Support using SCI to indicate the cast type of SL-PRS transmission: - Rx UE uses the cast type and corresponding source/destination ID filtering to decide whether to process the SL-PRS associated with Tx UE’s SCI. |
| Apple | Proposal 16: The SL PRS may be unicast, broadcast or multi-cast. For broadcast and multicast SL PRS/SRS in the shared resource pool, RAN1 should study the following two options: • Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together • Option 2: Grouping is independent of the PSSCH Proposal 17: Broadcast and multicast transmission in the dedicated resource pool can use the design for SL communications as a starting point in the new SL-PRS SCI. |

FL comment: Proposal merged with 3.2.5

# Configuration/activation/deactivation/triggering of SL-PRS

## Triggering of SL-PRS

Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement. Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

| Huawei, HiSilicon | Proposal 23: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, where the lower layer signaling can be SL MAC-CE or SCI sent by UE-A. |
| --- | --- |
| OPPO | Proposal 11: In Scheme 1 or Scheme 2, support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers, this also includes higher layer triggering from another UE or gNB. Proposal 12: In Scheme 1 or Scheme 2, support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, the lower layer signaling is SCI. |
| Spreadtrum | Proposal 15: With regards to the triggering of SL-PRS, both Option 1 and Option 2 should be supported. |
| CATT, GOHIGH | Proposal 25: For SL-PRS resource allocation scheme 2, with regards to the triggering of SL-PRS, support both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Option 2: Support UE-A to request UE-B to transmit SL-PRS via SCI sent by UE-A. |
| Intel | - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layer (Option 1). |
| Sony | Proposal 9: Support high-layer and lower-layer signalling involvement in the SL-PRS configuration. |
| Panasonic | Proposal 1: Both UE’s own higher layer signaling and other UE’ lower layer signaling are supported for triggering SL-PRS transmission in Scheme 2. |
| LGE | Proposal 17: Both option 1 and 2 are supported for SL PRS resource selection triggering. - UE’s higher layer when SL positioning service is triggered - Other UE’s physical layer request on SL PRS (re)transmissionc |
| China Telecom | Proposal 4: For SL-PRS triggering in scheme 2 (UE autonomous SL-PRS resource allocation)，we prefer opt. 1. (triggering by the UE’s own higher layers). Proposal 5: If opt. 2 (via lower layer signaling sent by another UE) is supported, at least SCI should be permitted as a lower-layer signaling. |
| CMCC | Proposal 7: With regards to the triggering of SL-PRS, support both of the options: - Option 1 should be the baseline for SL-PRS transmission, at least support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers; - Option 2 is applicable for some specific cases, e.g., for SL RTT-type positioning procedure. - The lower layer signaling should be SCI. |
| ZTE | Proposal 19: In Scheme 2, with regards to the triggering of SL-PRS: - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes another UE requests UE to transmit SL-PRS and UE’s own higher layer then triggers the corresponding SL-PRS transmission. |
| CEWiT | Proposal 5: In Scheme 2, with regards to the triggering of SL-PRS, support the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signalling sent by UE-A. - lower-layer signalling is one of the SCI and SL MAC-CE. |
| Interdigital | Proposal 23: Support both Option 1 and Option 2 regarding the triggering of SL-PRS. Proposal 24: Study UE request SL-PRS transmission using a SCI and/or PSFCH. |
| Ericsson | - For triggering of SL PRS under Scheme 2, we support option 1; The triggering of the SL PRS transmission is over either UE’s own higher layers or another UE’s higher layer after performing the authentication procedure. - For the SL-PRS transmissions without periodic reservation, use the maximum number of transmission from legacy sidelink (i.e. up to 3 occasions in time resource assignment). - For periodic reception of SL PRS, reservation periodicity values are the same as for legacy sidelink. |
| Qualcomm | Proposal 13: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers only (i.e. Option 1 for Scheme 2). Proposal 14: Requests for transmission of SL-PRS are introduced as higher-layer messages and no explicit support is needed in lower layers. |
| ITL | Proposal 2: In Scheme 2, with regards to the triggering of SL-PRS, at least following option 2 should be supported. - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. ▪ UE-A: target UE, UE-B: only master UE among multiple anchor Ues |
| Mediatek | Proposal 3-3: Support both options, - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. And SL MAC-CE is preferred |

We make the following notes:

| Option 1 (through own UE’s higher layer) | Intel, China Telecom, Ericsson, Qualcomm, ZTE |
| --- | --- |
| Option 2 (lower layer) | Huawei, HiSilicon, ITL |
| Support Both | OPPO, Spreadtrum, CATT, GOHIGH, Sony, Panasonic, LGE, CMCC, CEWiT, Interdigital, Mediatek |

Based on the above, there is a clear majority to support both options.

- One compromised approach (appeared in ZTE’s paper) would be that, when UE-B receives the lower-layer request, it sends it to its own higher layers, and then the UE-B’s higher layers decide to trigger SL-PRS at the physical layer.

Lets give a try such a compromised solution:

[CLOSED] Feature Lead Proposal 5.1-v0

In Scheme 2, with regards to the triggering of SL-PRS, support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - Lower layer signaling corresponds to SCI

Companies views

| CATT | We think two sub-bullets ared needed for the Option 1 and Option 2. We prefer the following revision: ##### Updated [HIGH] Feature Lead Proposal 5.1-v0 In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - Lower layer signaling corresponds to SCI |
| --- | --- |
| vivo | We are okay with the CATT modification |
| Sharp | We support the proposal. |
| OPPO | Basically fine with the proposal. But as stated in Proposal 3.3.2, SL PRS may be transmitted w/o PSCCH, lower layer siganling corresponds to SL PRS should not be precluded for the time being. - Lower layer signaling corresponds to SCI and/or SL PRS |
| Qualcomm | We prefer to not introduce lower layer signaling since the need has not been fully justify. We can accept the proposal as a compromise but leaving it FFS the details of the lower-layer signaling. |
| InterDigital | We support the proposal. We agree with CATT’s modification. |
| NEC | Support. |
| Panasonic | Supprot |
| Spreadtrum | Support |
| DCM | Prefer CATT’s ver. |
| CMCC | Support. |
| Samsung | OK with the proposal. |
| Intel | We would like to avoid the lower layer signalling of the transmissions as this does create the necessaity for additional SCI signalling for the shared resource pool. In our understanding even if dedicated resource pool is configured this would need to be signalled in the shared resource pool. There is only a minor latency benefit of signalling this in the SCI (processing time of SCI vs decoding SL MAC CE or SL RRC), as there is no target to optimize latency for SL positioning we do not think this should be priortitized. |
| ZTE | We still prefer only support Option 1. |
| Lenovo | Support FL’s proposal |
| Toyota | Support. |
| LGE | We prefer the original wording instead of the compromised one, and support both option 1 and option 2. Especially for lower layer triggering, the scenario of interest is SL RTT. When UE1 transmits SL PRS#1 to UE2, definitely it is assume that all the necessary SL positioning configurations are shared between two Ues. In this case, SCI associated SL PRS#1 just trigger the timing of SL PRS#2 transmisstion, based on the configured parameters. In the above case, there is no need to deliver the physical layer request to a higher layer. There is no remaining parameters to be exchanged. There are only L2 source and destination ID, so no more UE idenfication can be obtained from higher layer for any confirmation, which was not the case in SL communication because there is MAC PDU transmission. With the reason above, we support the original two options as they are. ##### [HIGH] Feature Lead Proposal 5.1-v0 In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. FFS: Whether lower-layer signaling is SCI or SL MAC-CE |
| Continental Automotive | We are fine with the proposal. |
| Futurewei | Support |
| SONY | We are fine with the proposal |
| Apple | Support |
| Nokia, NSB | Support; prefer CATT’s formatting |
| Ericsson | Do not support. Positioning requests should follow the positioning architecture, i.e. via higher layers |

Current status

- Support original options before the merging:

  - LGE (1)

- Support the proposal:

  - CATT, vivo, Sharp, Interdigital, NEC, Panasonic, Spreadtrum, DCM, CMCC, Samsung, Lenovo, Toyota, Continental, Futurewei, Sony, Apple, Nokia, NSB (17)

- Can accept with the last bullet in FFS:

  - OPPO, Qualcomm

- Do not support:

  - Intel, ZTE, Ericsson (3)

[CLOSED] Feature Lead Proposal 5.1-v1

In Scheme 2, with regards to the triggering of SL-PRS,

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - [FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS]

Companies views

| LGE | As commented, we’re not so sure if the received lower layer request should always be delivered to higher layer for confirmation. Source/destination ID filtering can be done by MAC layer, and SL PRS transmission can be decided by MAC layer. This should be possible especially when all the SL PRS configurations are exchanged between Ues via SLPP in advance. UE-A just trigger the transmission that are supposed to be done by UE B (that is, the transmission timing is decided by triggering). We’re ok to further study this point. We prefer to take the first sub-bullet as FFS. - FFS: whether it is up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A |
| --- | --- |
| OPPO | We are fine with the proposal with the brackets around the FFS removed. |
| CATT | Support the proposal. |
| Continental Automotive | OK with the proposal. |
| CMCC | Support |
| ZTE | Do not support. We still do not see the need to intrduce lower layer requesting SL-PRS transmission. If the use case if RTT, once the higher layer configured that the method is SL-RTT, a SL-PRS resource with dense repetition behaviour can be configured. Lower layer does not help to reduce too much latency. |
| Fraunhofer | Support |
| Spreadtrum | Support |
| CEWiT | Support |
| Ericsson | We still have an issue with lower layer triggers /requests. Ok with the first bullet, but not the second. |
| NEC | Support. |
| Intel | Support |
| Toyota | Support the proposal, and prefer to keep lower layer signaling at least as a subject for discussion because latency is essential for automotive applications. Square brackets should be removed since there is no reason to have both square brackets and FFS. |
| Qualcomm | We can accept the proposal for progress as we noted in the previous round. Our first preference is to not introduce lower layer requests. |
| Nokia, NSB | Support |
| Sharp | We support the proposal. |
| InterDigital | We support the proposal. |
| Lenovo | Support |

# SL-PRS Time domain behavior

The following agreement has been made:

Agreement With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering

The following related proposals were found in the contributions:

| Nokia, NSB | Proposal 17: Periodic or aperiodic (pre-)configured SL PRS can be activated or deactivated based on indications coming from higher layers, e.g., based on parameters such as distance and/or time thresholds that are (pre-)configurable. |
| --- | --- |
| CATT, GOHIGH | Proposal 22: For SL-PRS transmissions without periodic reservation, the maximum number of reservations and transmission after triggering should be 3, which is similar with Rel-16 V2X design. Proposal 23: SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation should be applicable to both resource allocation scheme 1 and scheme 2. Proposal 24: The SL-PRS transmissions without periodic reservation should be triggered explicitly by the related signalling procedure. |
| Intel | - For a shared resource pool, periodic, aperiodic, and semi-persistent transmissions of SL-PRS are supported for both resource allocation schemes 1 and 2. |
| ZTE | Proposal 20: For SL PRS transmissions without periodic reservation or within a periodic instance, support the same maximum number, i.e. 3 of reservations as the legacy SL communications. |
| CEWiT | Proposal 4: For SL-PRS transmissions without periodic reservation, the repetition should be associated with PRS resource configuration as well TB (if slot includes PSSCH). Further the number of reservations should depend on number of SL-PRS configurations to be measured. |
| Fraunhofer | Proposal 12: Re-use SCI indications of resource reservation interval and TRIV in SL communication for periodic and aperiodic SL-PRS resource reservation, respectively. |

[LOW] Feature Lead Proposal 6-v1

For SL-PRS transmissions without periodic reservation, the maximum number of reservations and transmission after triggering should be (pre-)configurable with a value of 2 or 3, which is similar with Rel-16 V2X design.

- SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2.

Companies views

| CATT | In our view, the maximum number of reservations and transmission after triggering should be (pre-)configurable with a value of 3, instead of 2 or 3. |
| --- | --- |
| Qualcomm | We are ok with reusing the existing design |
| InterDigital | We support the proposal. |
| Intel | In our understanding the proposal does not reflect Rel-16 V2X design. The number 2 or 3 reflects the number of resource that can be signalled in a single SCI not the (potentially larger) total number of reservations that can be transmitted after a trigger. We are thus unsure what is the target of this proposal. Is it to configure the number of resource in different slots that can be signalled in a dedicated resource pool in a signel SCI, or the number of retransmission of a single SL-PRS transmission. |
| Huawei, HiSilicon | Support. |
| Futurewei | Support |
| Nokia, NSB | Agree with Intel’s comment |
| CEWiT | Agree with proposal in principle but main bullet and sub bullet are indepent so sub-bullet can be made as second main bullet. |

FL comment: I am adjusting the wording to clarify Intels/Nokia question.

[LOW] Feature Lead Proposal 6-v1

For SL-PRS transmissions without periodic reservation, the maximum number of reservations signaled in an SCI  is (pre-)configurable with a value of 2 or 3, which is similar with Rel-16 sidelink.

- SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2.

Companies views

| OPPO | OK |
| --- | --- |
| LGE | Given clarification from FL, we suggest to remove the first part of the main sentence: For SL-PRS transmissions without periodic reservation. This is because there will be no difference in SCI between periodic and without periodic case, except the reservation interval field. To differentiate the proposal from the reservation interval field for periodic transmission, we suggest to replace ‘reservations’ to ‘reserved resources’. We think the main sentence and the bullet point are independent. We suggest to just to make two bullets. ##### [LOW] Feature Lead Proposal 6-v1 - For SL-PRS transmissions without periodic reservation, the maximum number of reservations reserved resources signaled in an SCI is (pre-)configurable with a value of 2 or 3, which is similar with Rel-16 sidelink. - SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2. |
| Continental Automotive | We have the same concern as LGE above. |
| ZTE | Agree with LGE’s modification. |
| Intel | We are fine with the changes proposed by LGE. |
| Feature Lead | Please comment on LGE’s version |
| Qualcomm | We prefer to also introduce the value 1. This is for the case where no SL-PRS repetition is enabled and only periodic reservations would be used. For such cases, (pre-)configuring value 1 would reduce the SCI payload size. |

[LOW] Feature Lead Proposal 6-v1

For SL-PRS transmissions without periodic reservation, the maximum number of reservations signaled in an SCI  is (pre-)configurable at least with a value of 2 or 3, which is similar with Rel-16 sidelink.

- SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2.

- FFS: value of 1

Companies views

| vivo | OK |
| --- | --- |
| Huawei, HiSilicon | Ok. For value of 1, I guess it could the default value if 2 or 3 is not configured. |
| CATT | We think LGE’s revision in previous round looks better, since the number of reserved resources is signaled in an SCI for Rel-16 V2X. |
| OPPO | The proposal should be applicable to dedicated resource pool only, for shared resource the number indicated by prarameter sl-MaxNumPerReserve should be used. We also think the first sub-bullet should be a independent bullet. |
| LGE | Same as previous comment. There are two fields in SCI for resource reservation in SL communication. One for dynamic resources that are used for HARQ retransmission, the other for periodic resources that are used for another packet transmission. For main sentence, we assume that 2 or 3 reserved resources are for SL PRS retransmission/repetition. This should be same for both periodic and aperiodic SL PRS transmission. So we remove ‘For’ phrase in the main sentence. If the wording ‘reservations’ in the main sentence means the reserved resources for SL PRS retransmission/repetition, it may be confused with the ‘resource reservation interval’ for indicating periodic SL PRS transmission. So we suggest to replace with ‘reserved resources’ for clarification. We still think that the 1st sub-bullet is not something belonging to the main bullet, but we’re ok with the current version. We’re also fine with FFS for value of 1 for no repetition case suggest by Qualcomm. ##### [LOW] Feature Lead Proposal 6-v1 For SL-PRS transmissions without periodic reservation, the maximum number of reservations reserved resources signaled in an SCI is (pre-)configurable at least with a value of 2 or 3, which is similar with Rel-16 sidelink. |
| Samsung | We think the value of 2 is enough for SL-PRS. In this regards, we suggest the following. ##### [LOW] Feature Lead Proposal 6-v1 For SL-PRS transmissions without periodic reservation, the maximum number of reservations reserved resources signaled in an SCI is (pre-)configurable at least with a value of 2. FFS: the necessary of the value of 3. |
| ZTE | Generally OK For SL-PRS transmission with periodic reservation, and for one periodic instance the maximum number of reservations signaled in an SCI is (pre-)configurable at least with a value of 2 or 3. |
| Xiaomi | We are fine with the proposal. For LG’ version, maybe we can change the wording to clarify the reserved resource is for aperiodic reservation: For SL-PRS transmissions without periodic reservation, the maximum number of reservations aperiodic reserved resources signaled in an SCI is (pre-)configurable at least with a value of 2 or 3, which is similar with Rel-16 sidelink. |
| Panasonic | OK |
| Continental Automotive | Generally OK. Will the case of SL PRS with periodic reservation be treated separately? |
| InterDigital | We support the proposal. |
| Qualcomm | We just noticed that the main bullet does not mention the resource pool type. Our understanding is that this is for the dedicated resource pool since the shared resource pool has to follow the legacy design and configuration. ##### [LOW] Feature Lead Proposal 6-v1 In a dedicated resource pool and fFor SL-PRS transmissions without periodic reservation, the maximum number of reservations signaled in an SCI is (pre-)configurable at least with a value of 2 or 3, which is similar with Rel-16 sidelink. - SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2. - FFS: value of 1 |
| Intel | We are fine with the proposal. |
| Nokia, NSB | OK. |
| Spreadtrum | We are fine with the proposal. |

# Other Proposals

| Nokia, NSB | Proposal 18: SL-PRS transmission can be re-configured in response to changing target-anchor link conditions as per channel information report from receiver UE. Proposal 22: SL PRS transmissions can be re-configured based on higher layer feedback from the intended receiver that accounts for changing link conditions and positioning QoS requirements. |
| --- | --- |
| Continental Automotive | Proposal 1.1: [Definition of dedicated RP for SL positioning] An RP in which only signals or data strictly necessary for SL positioning (e.g., SL PRS, SL positioning measurement data) may be exchanged. If there are data channels defined within such an RP, they may only carry reference signals and/or control and/or measurement information necessary for an ongoing, completed, or future positioning operation. Proposal 1.2: [Definition of shared RP for SL positioning] An SL communication RP as defined in 3GPP TS 38.211 [4], which may also be used for exchanging reference signals and/or control and/or measurement information necessary for an ongoing, completed, or future positioning operation, while using only the existing control and data channels. |
| Sony | Proposal 2: In both shared and dedicated resource pool, SL-PRS from different UEs can be multiplexed in TDM-ed and/or FDM-ed within a sidelink resource. |
| Futurewei | Proposal 15: For RTT-based positioning a maximum time gap (delay) of the SL-PRS transmission from the responding SL UE should be supported. |
| Toyota | Proposal 1: Specify a solution to assist the UE in the selection of SL PRS bandwidth. Proposal 2: Provide the UE with a set of bandwidth allocations with information about where and how the allocation can be used, e.g., environment type information, accuracy requirements, horizontal and vertical information whether joint-Uu/SL or SL-only positioning should be used. Proposal 3: The information in Proposal 2 is provided as (pre-)configurations. Proposal 4: The environment type is inferred by combining the UE speed, RSRP variations, and LOS/NLOS variations. |
| Panasonic | Proposal 3: The reception of SL-PRS could be always-on or triggering-based for power saving. Proposal 4: The reception of SL-PRS may be associated with ha reception window. Proposal 5: The triggering of SL-PRS transmission or reception could be with different signaling for different use cases. Proposal 6: For a SL UE requests other SL UE(s) to transmit or receive SL-PRS, the requesting UE should also indicate the resources to be used for SL-PRS. |
| LGE | Proposal 20: It is supported that UE1 reserves UE2’s resources used for SL positioning (e.g. SL PRS or measurement report) and sends the SL PRS resource reservation information to UE2, at least when UE1 and UE2 are involved in the same SL positioning service. Proposal 21: When UE-A transmits SL PRS to UE-B, UE-A indicates to UE-B the required maximum responding time, within which e.g. the responding SL PRS in SL RTT or the measurement report in SL TDOA should be transmitted to UE-A. Proposal 25: When TX and RX SL PRS resources collide each other, a prioritization rule is necessary to drop either operation. Proposal 26: When SL PRS resource and UL transmission resource collide each other, a prioritization rule is necessary to drop either operation. Proposal 30: SL synchronization procedure is performed by the UE that performs SL positioning. Proposal 31: The synchronization errors between anchor Ues for SL TDOA shall be less than a (pre)configured threshold. Proposal 32: For RSTD-based SL TDOA, If anchor UE is directly synchronized to Gnb/Enb, anchor UE transmits SL PRS with advancing the transmission time by TA/2, where TA is the timing advance used for UL transmission. Proposal 33: In RSTD-based SL TDOA, RTT measurement between anchor Ues and Rx-Tx time difference measurement of anchor UE are reported to a target UE. Proposal 34: UE mobility and GDOP can be considered as the accuracy indicator of the measurement.c |
| Sharp | Proposal 1: Muting is supported for SL-PRS. - The muting functionality as specified for DL-PRS can be considered as a starting point. Proposal 10: SCI is used for the signaling of the parameters of broadcast SL PRS. |
| Samsung | Proposal 7: Two RTT methods are supported for SL positioning where RTT method 1 transmits one SL PRS resource and RTT method 2 transmits two SL PRS resource based on Sensing based or random selection in Scheme 2. |
| CMCC | Proposal 8: SL-PRS muting mechanism can be introduced to further alleviate the congestion condition. - The corresponding enhancements on resource selection procedure for Scheme 2 may be required. |
| Lenovo | Proposal 4: RAN1 to consider Scheme 1 and Scheme 2 for transmission and reception of SL positioning messages. FFS whether SL-PRS dedicated resource and/or shared resource pool may be tilized. Proposal 9: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, Gnb, LMF and the type of trigger, e.g., based on a higher-layer SL positioning/ranging service request. Send LS to RAN2 in relation to agreed SL-PRS triggering mechanism. |
| Interdigital | Proposal 1: Consider the following terminology for the purpose of continued RAN1 discussion: - Reference anchor UE: An reference entity e.g., anchor UE, based on which a target UE’s relative position and/or ranging information is determined. - Sidelink positioning group: A group of Ues including a target UE and one or more anchor Ues, who participate in a SL positioning session using one of the supported SL positioning method. Proposal 4: Study how to use unassigned symbols in a slot or a sub-slot allocated for SL-PRS transmission. |
| Ericsson | - To reduce the probability of collisions, coordinated transmission of reservation messages in SCI should be studied |
| NEC | Proposal 1: At least for OOC scenarios, non-contiguous frequency resource could be considered for the SL positioning. Proposal 2: Considering the combination of shared resource pools to provide wideband frequency resource for SL positioning. Proposal 7: The movement of reference points should be taken into account for the study of SL positioning methods including OTDOA, RTT, AOA/D, etc., especially for DL positioning. |
| ITL | Proposal 3: The periodicity of DL PRS allocation based on unit of slots should be changed. Following could be starting points for each of options for resource allocation mechanism for SL-PRS. - For Option 1(sensing based resource allocation) : the periodicity of SL-PRS allocation is based on P_reserve - For Option 2(random resource selection) : the periodicity of SL-PRS allocation is based on sl-TimeResource (length = L_bitmap)c |
| Intel | - Agree on the following design principles: - No hierarchy between different Ues at the physical layer. - Sensing and random resource selection can be configured to be the only resource allocation modes in a resource pool. - SL PRS unicast/groupcast/broadcast can occur in the same resource pool and the related reception procedures are the same at physical layer. Reception procedures for the SL PRS reception in a dedicated SL PRS resource pool are the same for scheme 1 and scheme 2. |

# 8 Proposals for Round 4

[MEDIUM] Feature Lead Proposal 3.5.2.B-v3

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

      - Option 1: SL-PRS

      - Option 2: PSCCH DMRS

      - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

      - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

      - Option 2: the selection window is provided by higher layers

- Modification 3: For the SL-PRS priority:

      - Option 1: A single L1 SL-PRS priority allowed in for each resource pool

      - Option 2: SL-PRS transmissions can have different priority in a resource pool

- Modification 4: For the definition of a candidate resource within the resource selection window:

      - Option 1: The resource elements according to the {OFDM symbols, comb-offsets, allocated bandwidth} occupied by a SL-PRS resource

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%):

  - Options are TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Pre-emption priority for SL PRS resource (re)selection is (separately pre-)configured from that for SL data resource (re)selection.

- Note 1: Other potential modifications and/or other options within each modification are not precluded

- Note 2: Multiple options for each potential modification may be supported

Companies views

| Feature Lead | It appeared to me that the mod.7 had more comments on what it really meant and It may be making the whole proposal unstable. My suggestion is for now to put the Options as TBD for that. The rest looked more stable. It will be really useful for next meeting to have some potential modifications written down. Please point out which modifications you think are clear to be kept to this proposal. Please do NOT comment which one you support, but rather, comment if you are very confident that a specific modification is not reasonable, or if you find the modification reasonable but the options are unclear/unreasonable (in which case we could remove the Options and put “TBD”). |
| --- | --- |
| OPPO | For Modification 8, pre-emption priority is (pre-)configured per resource pool, and likely SL data would not be transmitted in dedicated resource pool, “(separately pre-)configured from that for SL data resource (re)selection” is confusing. |
| vivo | For modification 4, we prefer to remove the sub-bullet since SL PRS resource defined in AI 9.5.1.1 can be used. For modification 8, the pre-emption priority for SL PRS is unclear to us, whether it is the same as the PRS priority or the same as the data for shared resource pool. Maybe we can remove the sub-bullet of modification 8. |
| Feature Lead | Please continue to comment using the following changes in modification 4 & 8: - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD Option 1: The resource elements according to the {OFDM symbols, comb-offsets, allocated bandwidth} occupied by a SL-PRS resource - Modification 8: For the pre-emption of the reserved resources: - Options TBD Pre-emption priority for SL PRS resource (re)selection is (separately pre-)configured from that for SL data resource (re)selection. |
| Qualcomm | We are ok with the proposal and the changes to Modifications 4 and 8. |
| Nokia, NSB | OK |

[LOW] Feature Lead Proposal 6-v2

In a dedicated resource pool, the maximum number of reserved resources signaled in an SCI  is (pre-)configurable at least with a value of 2 or [3], which is similar with Rel-16 sidelink.

- SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2.

- FFS: value of 1

Companies views

| Samsung | Some question and comment in the below as 1. Why did this proposal suggest only for the dedicarted resource pool? 2. We think the first subbullet is not related to the proposal in the main bullet. In this regard, how about the following version? In both shared and dedicated resource pool, the maximum number of reserved resources for SL-PRS transmission signaled in an SCI is 2. - FFS: (pre-)configurable with other maximum values of 1 and 3 |
| --- | --- |
| CATT | This proposal seems to be applicable for both the dedicated resource pool and shared resource pool, because such mechanism is similar to Rel-16 V2X behavior. We think Samsung’s version above is better. |
| OPPO | Fine in general, but we agree with Samsung that the 1st sub-bullet is irrelevant and should be an independent bullet. We agree to add “in a dedicated resource pool”, in shared resource pool, for back-ward compatibility it should be the same as Rel-16. |
| vivo | For back-ward compatibility of shared resource pool, 3 should be supported For the proposal, we prefer to support 3, and remove the bracket of 3 |
| Qualcomm | To ensure backward compatibility in a shared resource pool (legacy Rel16/17 and R18 devices without SL positioning support), the maximum number of reserved resources should use existing (pre-)configuration parameters and signaling. In our view, the proposal cannot be generalized to the shared resource pool. For the dedicated resource pool, this issue does not exist and we would be ok with putting both 1 and 3 in an FFS as suggested by Samsung or both in brackets. |
| Feature Lead | Lets try to go one step at a time and try to agree “at least” for dedicated RP and we can put an FFS for the shared RP. At least for the a dedicated resource pool, the maximum number of reserved resources signaled in an SCI is (pre-)configurable at least with a value of 2 or [3], which is similar with Rel-16 sidelink. - SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2. - FFS: value of 1 - FFS: shared resource pool |
| Feature Lead2 | I missed one comment to put 1 and 3 in the same FFS or both in brackets: ##### [LOW] Feature Lead Proposal 6-v4 At least for the a dedicated resource pool, the maximum number of reserved resources signaled in an SCI is (pre-)configurable at least with a value of 2, which is similar with Rel-16 sidelink. - SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2. - FFS: value of 1, 3 - FFS: shared resource pool |
| Qualcomm | We prefer to remove “FFS: shared resource pool” since that is already determined by the legacy configuration. However, we can accept the proposal as-is for now. |
| Nokia, NSB | Ok in general. “At least” can be removed from the proposal. Since “which is similar with Rel 16 sidelink” is now removed and value 3 is still open, legacy behaviour may not strictly be adopted. Therefore, the proposal can be decoupled for dedicated and shared RPs. Concerning 1st bullet, we note that legacy mode 1 does not use periodic reservation. gNB can schedule periodic resources, but these are not reserved by SCI (i.e., field “Resource reservation period” in 1st stage SCI is not set). |

# Proposal for Online Discussion

This section will contain proposals for discussion during online time:

## Tuesday session (04/25)

##### [HIGH] Feature Lead Proposal 3.5.4-v3

For Scheme 2,

- Alt. 1: support inter-UE coordination signaling for SL-PRS at least for shared resource pool

  - Both Rel-17 Scheme 1 and 2  IUC are used as a starting point

  - Note: For the dedicated resource pool, decide the support of IUC, by RAN1 #113 meeting

- Alt. 2: deprioritize it for this release

| 1st round | Overall, there is majority support to support IUC: - In favor: CATT, Apple, vivo, Xiaomi, CMCC, Huawei, HiSilicon, Futurewei, Interdigital, NEC, Panasonic, DCM (11) - Against: Qualcomm, Intel, LGE, Nokia, NSB (5) |
| --- | --- |
| 2nd round | - Alt. 1: Huawei, HiSilicon, CATT, Samsung, ZTE, Xiaomi, Panasonic, Continental, Interdigital, Nokia, NSB, Spreadtrum - Alt. 2: Qualcomm, Intel, LGE - Low-hanging fruit for Shared RP: OPPO |

##### [MEDIUM] Feature Lead Proposal 3.2.4-v1

For a dedicated resource pool for SL positioning, support a mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a slot

- FFS: Details

| 1st round | Supported by: - CATT, vivo, Sharp, Xiaomi, Interdigital, NEC, Panasonic, Spredtrum, DCM, CMCC, Fraunhofer, Lenovo, Huawei, HiSilicon, Futurewei, Apple, Nokia, NSB Postpone the discussion: - Intel, Ericsson |
| --- | --- |
| 2nd round | Support: - LGE, OPPO, CATT, Continental, CMCC, Xiaomi,, ZTE, Fraunhofer, Spreadtrum, NEC, Nokia, NSB, Sharp, Interdigital, Lenovo, Panasonic Postpone: - Intel, Samsung, Ericsson, Qualcomm (OK to postpone), Continental (OK to postpone) |

##### [MEDIUM] Feature Lead Proposal 3.2.5-v1

In the dedicated resource pool, SCI for SL-PRS should at least include the following fields [indicate the following values]:

- [24 bits] Source ID

- [24 bits] Destination ID

- Resource reservation period

- SL-PRS Priority

- FFS: Cast type

- SL-PRS configuration and/or time-frequency assignment information:

  - FFS: Details

- 24-bit CRC Field

- FFS: Additional information, e.g. SL-PRS request, Positioning Session ID

| 2nd round | Cast Type - Needed: - OPPO, CATT, LGE, Continental, ZTE, Intel, Qualcomm, Nokia, NSB, Sharp, Lenovo - Not Needed: - CMCC, Xiaomi, vivo |
| --- | --- |

##### [LOW] Feature Lead Proposal 3.6-v1

Study further feedback mechanisms for SL-PRS including:

- ACK/NACK feedback for unicast and groupcast SL PRS transmissions

- Feedback-based Retransmissions

Support - OPPO, LGE, Continental, NEC, Nokia, NSB, Interdigital Do not support - CATT, Samsung, ZTE, Intel, vivo

## Wednesday session (04/26)

##### [MEDIUM] Feature Lead Proposal 3.5.2.B-v4

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

      - Option 1: SL-PRS

      - Option 2: PSCCH DMRS

      - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

      - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

      - Option 2: the selection window is provided by higher layers

- Modification 3: For the SL-PRS priority:

      - Option 1: A single L1 SL-PRS priority allowed in for each resource pool

      - Option 2: SL-PRS transmissions can have different priority in a resource pool

- Modification 4: For the definition of a candidate resource within the resource selection window:

      - Options TBD

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%):

  - Options TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Options TBD

- Note 1: Other potential modifications and/or other options within each modification are not precluded

- Note 2: Multiple options for each potential modification may be supported

##### [HIGH] Feature Lead Proposal 5.1-v2

In Scheme 2, with regards to the triggering of SL-PRS,

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

| 1st round | - Support original options before the merging: - LGE (1) - Support the proposal: - CATT, vivo, Sharp, Interdigital, NEC, Panasonic, Spreadtrum, DCM, CMCC, Samsung, Lenovo, Toyota, Continental, Futurewei, Sony, Apple, Nokia, NSB (17) - Can accept with the last bullet in FFS: - OPPO, Qualcomm - Do not support: - Intel, ZTE, Ericsson (3) |
| --- | --- |
| 2nd round | - Support original options before the merging: - LGE - Support the proposal: - OPPO, CATT, Continental, CMCC, Fraunhofer, Spreadtrum, CEWiT, NEC, Intel, Nokia, NSB, Sharp, Interdigital, Lenovo, OPPO, Qualcomm (can compromise), Panasonic (16) - Do not support: - Intel, ZTE, Ericsson (3) |

##### [HIGH] Feature Lead Proposal 3.5.5-v1

For scheme 2,

- Alt. 1: support that a UE can reserve a SL-PRS resource for the transmission of another UE.

  - FFS: details regarding the conditions and signaling mechanism

- Alt. 2: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE.

| 2nd round | Support: - CATT, LGE, Continental, CMCC, Xiaomi, Fraunhofer, Spredtrum, CEWiT, NEC, Nokia, NSB, Sharp, Interdigital, Lenovo, Panasonic (14) Do not support: - OPPO, Intel, Qualcomm, ZTE, vivo, Samsung (6) |
| --- | --- |

## Potential update in company’s views from Section 8.1

Companies views

| Feature Lead | Companies are encouraged to check the status of the above proposals, and the previous replies and rounds, and kindly suggest whether they will be flexible to accept the majority view in any of the proposals. This includes whether you have a “slight preference”, or “ you would be OK either way”. Similarly please comment if i have wrongly understood your view. Please do not repeat the technical arguments that were provided in the detailed sections, unless you have additional/different technical arguments that might persuade the majority to change view. |
| --- | --- |
| OPPO | For Proposal 3.5.2.A-v2 on partial sensing, as a compromise we can accept to use Rel-16 full sensing as starting point, but we hope partial sensing could also be considered in Rel-18 when full sensing mechanism for SL PRS is stable. Based on Rel-17 partial sensing mechanism, we believe the work load to support partial sensing on top of full sensing for SL PRS is manageable. We propose to add the following bullet at the end of the proposal: Partial sensing should also be considered when full sensing mechanism for SL PRS resource allocation is stable. ##### For Proposal 3.6-v1 on feedback mechanism, we still do not understand why “ACK/NACK feedback” is included in addition to “Feedback-based Retransmissions”. In our view “Feedback-based Retransmissions” can cover “ACK/NACK feedback”. Furthermore, as SL PRS has no CRC, how to support ACK/NACK feedback for it is questionable to us. |

# 10 Proposals for Offline Discussion – Monday 04/24 14:00-15:00

This section will contain proposals for discussion during offline time:

[HIGH] Feature Lead Proposal 3.3.2-v4

In a shared resource pool:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot

    - With regards to PSSCH and SL-PRS multiplexing, pick one of the following alternatives:

      - Alt. A.1: Only TDMing is supported

      - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported

        - FFS: Rate-matched around SL-PRS REs and/or PRB/subchannel-level FDMing are supported potentially for different cases

        - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS

      - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification

    - With regards to PSCCH and SL-PRS multiplexing, pick one of the following alternatives:

      - Alt. B.1: Only TDMing is supported

      - [Alt. B.2: TDMing or sub-channel-based FDMing is supported]

    - The PSSCH is used to:

      - Alt. C.1: 2nd SCI only

      - Alt. C.2: 2nd SCI and SL-SCH

      - Alt. C.3: “2nd SCI only” OR “2nd SCI and SL-SCH”

    - FFS: Handling of PT-RS and PRS

# 10 References

- R1-2302295	On resource allocation for SL positioning reference signal	Nokia, Nokia Shanghai Bell

- R1-2302328	Discussion on resource allocation for SL PRS	FUTUREWEI

- R1-2302379	Discussion on SL-PRS resource allocation	Huawei, HiSilicon

- R1-2302387	Considerations on resource allocation for sidelink positioning reference signal	Continental Automotive

- R1-2302492	Discussion on resource allocation for SL positioning reference signal	vivo

- R1-2302555	Discussion on resource allocation for SL positioning reference signal	OPPO

- R1-2302584	Discussion on resource allocation for SL positioning reference signal	TOYOTA Info Technology Center

- R1-2302607	Discussion on resource allocation for SL positioning reference signal	Spreadtrum Communications

- R1-2302710	Further discussion on resource allocation for SL positioning reference signal	CATT, GOHIGH

- R1-2302803	On resource allocation for SL positioning	Intel Corporation

- R1-2302853	Discussion on resoucrce allocation for SL positioning reference signal	Sony

- R1-2302875	Discussion on Resource Allocation for SL-PRS 	Panasonic

- R1-2302928	Discussion on resource allocation for SL positioning reference signal	LG Electronics

- R1-2302990	Discussion on resource allocation for SL positioning reference signal	xiaomi

- R1-2303028	Discussion on SL positioning reference signal resource allocation	China Telecom

- R1-2303065	Resource allocation for SL positioning reference signal	Sharp

- R1-2303135	On Resource Allocation for SL Positioning Reference Signal	Samsung

- R1-2303241	Discussion on resource allocation for SL positioning reference signal	CMCC

- R1-2303265	Discussion on SL Positioning Resource Allocation	Lenovo

- R1-2303278	Discussion on resource allocation for SL positioning reference signal	ZTE

- R1-2303308	Discussion on SL positioning resource allocation for SL positioning reference signal	CEWiT

- R1-2303416	Considerations on SL-PRS resource allocation	Fraunhofer IIS, Fraunhofer HHI

- R1-2303445	Resource allocation for SL positioning reference signal	InterDigital, Inc.

- R1-2303490	On Resource allocation for SL positioning reference signal	Apple

- R1-2303552	Resource allocation for SL positioning reference signal	Ericsson

- R1-2303597	Resource Allocation for SL-PRS	Qualcomm Incorporated

- R1-2303684	Discussion on resource allocation for SL positioning reference signal	NEC

- R1-2303786	Discussion on resource allocation for SL PRS	ASUSTeK

- R1-2303823	Considerations on resource allocation for SL positioning reference signal	ITL

- R1-2303838	Resource allocation for SL-PRS	Mediatek (Chengdu) Inc.

# 11 Previous Agreements for SL Positioning Solutions

## RAN1 #109-e

Agreement

Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

Agreement

With regards to the Positioning methods supported using SL measurements study further the following methods:

  - RTT-type solutions using SL

    - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT

  - SL-AoA

    - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study

  - SL-TDOA

  - SL-AoD

    - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu.

    - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

- Consider in the study at least the following aspects:

  - Definition(s) of the corresponding SL measurements for each method

  - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed.

  - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities

  - Per-panel location, if UE uses multiple panels.

  - UE’s mobility, especially for V2X scenarios

  - Impact of synchronization error(s) between UEs

  - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used.

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method.

- Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review.

- Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method.

Agreement

With regards to the numerologies of the SL-PRS, limit the study to those supported for NR Sidelink.

- Note 1: NR Sidelink supports {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2

- Note 2: This doesn’t imply that SL-PRS FR2-specific optimization(s) are expected to be studied

Agreement

Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point.

- The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options.

- Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements.

Agreement

With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options:

- Option 1: High-layer-only signaling involvement in the SL-PRS configuration

  - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS.

  - Based on the study, this option may correspond to

    - A SL-PRS configuration that is a single-shot or multiple shots

    - A high-layer configuration that may be received from an LMF, a gNB, or a UE

- Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

  - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

- Option 3: Only lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

- Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

Agreement

With regards to the Sidelink Positioning measurement report,

- Study the contents of the measurement report  (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc)

- Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic)

- FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity.

- Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

  - FFS: clarification of the knowledge of the location of the anchor UE

Agreement

With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects:

- N>=1 (where N=1 corresponds to full RE mapping pattern)

- Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1)

- The number of symbols of SL-PRS within a slot

  - Any relation to the comb-N option

  - RE offset pattern repetitions within a slot

- FFS: Other frequency domain pattern(s)

Agreement

For a potential new SL PRS, study further the following

- Number of symbol(s) for AGC and/or Rx-Tx turnaround time

- Conditions under which AGC training and/or Rx-Tx turnaround time are needed

Agreement

With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration:

- Option 1: Dedicated resource pool for SL-PRS

  - Include in the study at least the following aspects:

    - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot)

    - positioning measurement report

    - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS

    - resource allocation procedure(s) of SL-PRS

    - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation

- Option 2: Shared resource pool with sidelink communication.

  - Include in the study at least the following aspects:

    - co-existence between SL communication and SL positioning, backward compatibility

    - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure

Agreement

With regards to the SL-PRS resource allocation, study the following two schemes:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

  - Applicable regardless of the network coverage

- FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions).

- Note: Other Schemes are not precluded to be studied

- FFS how to handle resource allocation of SL-Positioning measurement report

## RAN1 #110-e

Agreement

With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- Note: other methods can still be studied

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

Agreement

A new reference signal should be introduced for supporting SL positioning/ranging.

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

Agreement

With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging:

- Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

- Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

Agreement

For the content of the sidelink positioning measurement report, potential elements may include at least the following:

- One or more sidelink positioning measurement(s)

- Timestamp(s) associated with a sidelink positioning measurement

- Quality metric(s) associated with a sidelink positioning measurement

- Identification Information for a sidelink positioning measurement

- FFS any detail for the above

Agreement

For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2:

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

- Alt. 2: ZC-based (SRS sequence as a starting point)

Agreement

With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning

- Note: there could be multiple values for M, N

Agreement

Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects:

- Resource selection mechanism for SL-PRS

- Inter-UE coordination

- Aspects for congestion control mechanisms for SL-PRS

Agreement

- With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further.

- With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

Agreement

With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS.

- Note: this does not preclude comb N=1

- FFS: single symbol SL-PRS, if supported

## RAN1 #110-bis-e

Agreement

- With regards to the RTT-type solutions using SL, down-select between the following 2 alternatives:

  - Alt. 1: it corresponds to a single-sided RTT method

  - Alt. 2: it may correspond to either a single-sided or double-sided RTT method

    - With regards to the double-sided RTT,

      - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift

      - Study the order of the SL-PRS transmissions for double-sided RTT

      - Study the impact of UE mobility

    - FFS study whether there is or what is the spec impact of double-sided RTT method

- Note: the above may correspond to RTT with one or multiple devices

Agreement

For the study of SL-AoA positioning method,

- Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement should be included

  - FFS: Definition of the measurements

- Study further whether other measurements can be applicable

- Study further the frame of reference (LCS or GCS)

Agreement

With regards to SL-TDOA positioning method for SL-only positioning,

- It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE

- Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed.

Agreement

- From the potential candidate Positioning methods using at least SL measurements, at least the following should be introduced:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- FFS: SL-AoD

Agreement

Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options:

- Opt. 1: through higher layers from the LMF

- Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

  - Up to further discussion which one or more of these shall be applicable

Agreement

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

Agreement

From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one)

- Broadcast (as a working assumption).

- FFS: Applicability of the above cast types

Agreement

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb size) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: the values considered as potential candidate values for M

  - FFS1: Whether to consider N>12 as a potential candidate value(s)

- The symbols of a SL-PRS resource within a slot are consecutive symbols

  - FFS: consecutive and/or non-consecutive symbols for shared resource pool (if supported)

- FFS: RE-Offset sequence within a SL-PRS resource, including whether to have in the end of the SL-PRS pattern a symbol with the same RE-offset as the first symbol, for phase-tracking purpose

Agreement

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

    - FFS: Details

    - FFS: definition of PSSCH associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above

Agreement

With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

Agreement

With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning:

- Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s)

  - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration.

  - FFS: Whether SCI is single stage SCI or two stage SCI

- FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements are considered:

  - SL Rx-Tx measurement

  - SL RSTD measurement

  - SL RSRP measurement

  - SL RSRPP measurement

  - SL RTOA measurement

  - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor) and provide their analysis into why they are needed in light of the above measurements.

- Companies are encouraged to study potential enhancements, such as SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement

- FFS any additional measurements

Agreement

At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives:

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

- FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Agreement

Study further the granularity of time-domain resource allocation for SL-PRS transmission.

Agreement

With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective:

- Periodic SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity

  - FFS: any additional details, including whether or not higher layers can start/stop transmission.

- Semi-persistent SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

  - FFS: any additional details

- Aperiodic SL-PRS

  - SL-PRS is transmitted at least once after [triggering/request]

    - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions.

  - FFS: any additional details

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

- FFS: Rx UE behavior is separately discussed.

- FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

## RAN1 #111

Agreement

SL-AoD is included as a potential candidate positioning method, and

- SL-AoD should be deprioritized over the remaining methods that have been recommended to be introduced.

Agreement

With regards to the SL Positioning resource allocation, support

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

- Note: this does not imply that the design is the same for both types of resources pools

- Note: shared resources pool(s) should be supported with backward compatibility

Agreement

From RAN1 perspective, at least the following 2 operation scenarios are recommended for normative work:

- Operation Scenario 1: PC5-only-based positioning.

- Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Agreement

For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options:

- Option 1: A sensing based resource allocation should be introduced

- Option 2: A random resource selection should be introduced

- In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

Agreement

With regards to the RTT-type solutions using SL, both single-sided and double-sided RTT methods should be introduced

- Strive to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT.

- Note: a UE should be able to support single-sided RTT without having to support double-sided RTT

Agreement

Capture the following TP into the TR 38.859 as a conclusion:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

Agreement

A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier.

Agreement

With regards to the power control for SL-PRS at least Open Loop PC should be introduced.

Agreement

For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced.

- A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements should be introduced:

  - SL-PRS based Rx-Tx measurement

  - SL-PRS based RSTD measurement

  - SL-PRS based RSRP measurement

  - SL-PRS based RSRPP measurement

  - SL-PRS based RTOA measurement

  - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

Agreement

Update the agreed TP into the conclusion section of the TR 38.859 as follows:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

  - For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation is recommended for normative work.

  - For the support of the above methods the following measurements are recommended for normative work:

    - SL-PRS based Rx-Tx measurement

    - SL-PRS based RSTD measurement

    - SL-PRS based RSRP measurement

    - SL-PRS based RSRPP measurement

    - SL-PRS based RTOA measurement

    - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

  - With regards to the power control for SL-PRS at least Open Loop PC is recommended for normative work.

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

  - For resource allocation mechanism for SL-PRS in Scheme 2, a sensing based resource allocation, or a random resource selection, or both, should be introduced, where the legacy designs for UE autonomous resource allocation are used as a starting point.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

## RAN1 #112

- Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

- Agreement

For a dedicated resource pool for positioning:

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214).

  - FFS: additional slots that can be used for SL PRS is not precluded

- Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

- Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

Agreement

Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication).

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

  - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool

- FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

Agreement

With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications.

  - Study if any changes are needed

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options:

- Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes higher layer triggering from another UE

- Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement

For SL-PRS transmission, at least support the following

- SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication

  - FFS: whether/what changes are needed

- SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation.

  - FFS: Maximum number of reservations and transmissions after triggering

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective