3GPP TSG-RAN WG1 Meeting #112-bis-e	        	R1-2304255

# e-Meeting, April 17th - April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0426 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SFN configuration for HST-SFN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Intel Corporation), Google, Samsung, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TS 38.331, sfnSchemePDCCH and sfnSchemePDSCH are configured in ServingCellConfig, i.e., per-CC configuration. Hence, RAN1 specification should be updated to align with it. It should also be clarified that the SFN configuration is the same for all DL BWPs within a CC and the UE does not expect to be configured with sfnSchemePDCCH and sfnSchemePDSCH in the initial BWP as per the following agreement: Agreement Enhanced SFN (scheme 1 or TRP-based pre-compensation scheme) for PDCCH and PDSCH is configured by using separate per-BWP RRC parameters - In Rel-17, all downlink BWPs (except initial BWP and FFS: BWP-DownlinkCommon) within a CC should be the same configuration of SFN scheme. |
| Summary of change: |  | Remove description that sfnSchemePDCCH or sfnSchemePDSCH is configured for a BWP and add clarification that UE does not expect SFN configuration in initial BWP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 38.214 and TS 38.331 and misinterpretation of SFN configuration in initial BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

## 5.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' and

-	if the UE reports its capability of sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or

-	otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and

the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5.

When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' and activated with two TCI states by MAC CE, and the UE does not report its capability of sfn-SchemeA-PDCCH-only, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeA' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' and activated with two TCI states by MAC CE, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeB' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePdsch and/or sfnSchemePdcch, the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same within a CC, and the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all CCs in a same frequency band if the UE is configured with CA, where the UE does not expect to be configured with sfnSchemePdsch and/or sfnSchemePdcch in initial BWP in each CC.

< Unchanged parts are omitted >