3GPP TSG RAN WG1 #112-bis-e			            R1-2304259

e-Meeting, April 17th – 26th, 2023

Agenda Item: 7.2

Source: Moderator (MediaTek)

Title: Summary for maintenance on UE power saving enhancements

Document for: Discussion and Decision

# Introduction

This summary is for maintenance of R17 UE power saving enhancements based on companies’ contributions [1] - [4] and the summary of the maintenance in RAN1#112 [5] (as reference of what has been discussed before). Specifically, each of the submitted CRs is discussed in each sub-section of Section 2, and the final outcomes are summarized in Section 3.

# Maintenance on PDCCH Monitoring Adaptation

## Correction on mixed R16 SSSG switching and R17 PDCCH monitoring adaptation [1]

From the CR in R1-2302456 by vivo, the following are identified for the proposed correction to TS 38.213:

| Reason for change: | Background: Normally, Rel-16 SSSG switching is used for UE to immediately switching from a dense PDCCH monitoring (SSSG#0) outside the COT to a sparse PDCCH monitoring (SSSG#1) inside the COT. It is practically useful since outside the COT, the gNB need to perform LBT and occupy the channel frequently. Once the channel is occupied, the gNB has to send PDCCH immediately and delivery data to UE, otherwise, the occupancy will be expired. And once the UE enters the COT, for power saving purpose, UE is usually indicated to switch to another SSSG with sparse PDCCH monitoring periodicity. However, Rel-17 PDCCH monitoring adaptation has different purpose. When there is no requirement of traffic transmission, normally gNB will dynamically indicate UE to do PDCCH skipping. Issues: It has been agreed in RAN1 #108e meeting that R17 SSSG and R16 SSSG switching features do not support to be configured simultaneously. So, there will not be the case that R17 SSSG timer expires outside the COT duration. However, the current specification does not restrict the application of R17 PDCCH skipping outside COT. This means that a UE can still perform PDCCH skipping outside COT duration, resulting in unacceptable delay for channel occupancy. Solution: To not delay channel occupation, R17 PDCCH skipping should not be applied outside the COT duration, which means PDCCH skipping should be terminated once out of the COT duration. |
| --- | --- |
| Summary of change: | Correct the current specfication that R17 PDCCH skipping should not be applied outside the COT duration. |
| Consequences if not approved: | Incorrect UE behaviour if R17 PDCCH monitoring adaptation is applied outside the channel occupancy duration. |

where the proposed correction is quoted below:

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets. If a UE is provided with PDCCHSkippingDurationList and searchSpaceGroupIdList, the UE does not apply following procedures related to skipping of PDCCH monitoring outside the channel occupancy duration. A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. *** Unchanged text is omitted ***

In the following table, companies please input your views for deciding this CR:

Table 1 (1st_Round): Companies' views for correction on mixed R16 SSSG switching and R17 PDCCH monitoring adaptation (CR in R1-2302456 [1])

| Company name | Support to adopt the CR? (Y/N) | Views on the proposed correction |
| --- | --- | --- |
| CATT | N | There is no agreement that PDCCH skipping can not be used outside COT when Rel-16 SSSG switching is configured. Moreover, PDCCH skipping would be useful for reducing the power consumption of PDCCH monitoring outside COT. Thus, the restriction in this CR is not needed. |
| Samsung | N | Already discussed. There is no essential correction to make. |
| Nokia1 | N |  |
| OPPO | N | Understand the intention, but it is not agreed before. |
| vivo | Y | The intention of the CR is clear i.e., to not delay channel occupation before obtaining COT, R17 PDCCH skipping should not be applied outside the COT duration, which means PDCCH skipping should be terminated once out of the COT duration. Indeed, the above proposal is not agreed before, so there is the necessity to correct the UE behaviour outsider COT. |
| Huawei, Hisilicon | N | The issue was discussed in last meeting. No additional behaviour is needed for COT. |
| Xiaomi | N | Currently we are not convinced why PDCCH skipping can not be applied outside COT duration. We think it is still applicable and cause no harm. UE can acquire a new COT if needed after PDCCH skipping duration end, since it is gnB’s inetention to schedule like this. |
| Ericsson1 | N | This was already discussed in last meeting. |
| Intel | N |  |

Moderator would like to thank companies’ valuable inputs. From the table, it looks most companies’ think the proposed CR/restriction on PDCCH skipping outside COT is not necessary. Given it is gNB flexibility in whether to indicate PDCCH skipping outside COT, the intention of vivo proposal is not precluded. In this regard, moderator suggests no further discussion on the CR for RAN1#112-bis-e meeting.

## UE Behavior for PDCCH Monitoring Adaptation and BWP Switching Indicated in The Same DCI [2]

From the CR in R1-2303037 by Xiaomi, the following are identified for the proposed correction to TS 38.213:

| Reason for change: | When PDCCH monitoring adptation indication and BWP switching are indicated in the same DCI, a very power saving method is to apply the PDCCH monitoring adaptation on the new BWP once the BWP switching finishes. But current 38.213 does not specify UE behavior for this case, detailed analysis can see in our companion contribution R1-2209251. |
| --- | --- |
| Summary of change: | Add a description of UE hehavior for the case when PDCCH monitoring adaptation and BWP switching indicated in the same DCI. UE will apply the PDCCH monitoring adaptation after BWP switching on the new BWP. |
| Consequences if not approved: | The case that PDCCH monitoring adptation indication and BWP switching are indicated in the same DCI is not supported, and cost power waste at UE side |

where the proposed correction is quoted below:

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer, the UE - resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP - monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17. If UE receives PDCCH monitoring adaptation indication and BWP switching indication in the same DCI, the PDCCH monitoring adaptation will be applied on the new BWP starting from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell *** Unchanged text is omitted ***

In the following table, companies please input your views for deciding this CR:

Table 2 (1st_Round): Companies' views on UE behavior for PDCCH monitoring adaptation and BWP switching indicated in the same DCI (CR in R1-2303037 [2])

| Company name | Support to adopt the CR? (Y/N) | Views on the proposed correction |
| --- | --- | --- |
| CATT | N | This issue had been discussed for several meeting. There is no need of further discussion on this issue. |
| Samsung | N | Already discussed. There is no essential correction to make. |
| Nokia1 | N | I think we have already attempted to reach conclusion on this several times. |
| OPPO | Y | We actually think this is valid point to be clarified. The last meeting discussion unfortunately leave the specification a grey area. Better to solve this. |
| vivo | N | As we commented in the last several meeting, in case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously, it is in principle to apply the SSSG or PDCCH skipping to the target BWP. Besides, the current spec is workable, since most time the BWP switching delay is larger than the application delay for PDCCH skipping and SSSG switching. And it is not clear for us to understand why the mentioned power waste at UE side will be cost. |
| Huawei, Hisilicon | Y | First of all, we should firstly clarify that whether PDCCH skipping or SSSG switching could be indicated in a DCI where a BWP switching is indicated. We don’t have any agreement to say PDCCH monitoring adaptation is not supported in a DCI when a BWP switching is indicated in the same DCI. We strongly prefer to clarify whether PDCCH monitoring adaptation is supported or not in a DCI when a BWP switching is indicated in the same DCI, especially from the companies who are against the discussion of this issue for several meetings. For the information interpretation, the legacy principle is to interpretation based on the configuration of target BWP. And it is already clearly specified by current specification according to our analysis in last meeting. We are supportive in general with the proposed CR from Xiaomi. |
| Xiaomi | Y | As the proponent we support the CR. And for further explanation， 1, whether it is reasonable gNB behavior to indicate BWP switching and PDCCH switching in the same DCI? We think it is reasonable, because gNB can spare one DCI transmission compared to indicating BWP switching in one DCI and PDCCH skipping in ganother DCI, thus reduce resource overhead and also UE detection power consumption. 2, How PDCCH skipping would be applied? We believe it is reasonable to apply PDCCH skipping only on the new BWP, since anyhow the original BWP can not be used for transmission after the BWP switching indication. |
| Ericsson1 |  | As this issue was discussed last two meetings, it is not clear whether any new issue is being raised. |
| Intel | N |  |

Moderator would like to thank companies’ valuable inputs. The CR proposed by Xiaomi include two parts: 1) whether the indication refers to the setting of the target BWP of the BWP change indication, and 2) whether the application timing is aligned with that of active BWP change. For 1), multiple companies feedback that specification has been clear, and moderator would like to quote the following text from clause 12 of TS 38.213, which (implicitly) states interpretation of the DCI format is according to the setting of the BWP which the DCI format indicates:

Clause 12, TS 38.213 … If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the DCI format - if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively - if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively

For the second part of change, moderator thinks it is equivalent to check and align companies’ views on what is the “first slot” UE applies the PDCCH monitoring adaptation for the case where DCI indicates both PDCCH monitoring adaptation and active BWP change. It is noticed that, if companies have different understanding, BS and UE will be inconsistent in the timing when UE should resume PDCCH monitoring, which may cause missing of scheduling.

Clause 10.4, TS 38.213 - SSSG Switching (Rel-17) When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication - at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ ∈ {0, 1, 2, 3} , - at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ ∈ {5, 6}

Clause 10.4, TS 38.213 - PDCCH Skipping When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. …

Based on the above spec text, moderator would like to check whether the following conclusion is aligned with companies’ understanding and thus agreeable to close this issue:

Conclusion 2.2-1:

For the case a DCI format indicates both PDCCH monitoring adaptation and active BWP change, the first slot UE applies the PDCCH monitoring adaptation is the slot indicated by the slot offset value of the time domain resource assignment field in the DCI.

Table 3 (2nd_Round): Companies' views on Conclusion 2.2-1

| Company name | View(s) on Conclusion 2.2-1 |
| --- | --- |
| Spreadtrum | We don’t think spec change is necessary as mentioned by the 1st round companies’ view. We support FL’s conclusion. UE PHY layer can do the separate things for separate field according to Spec. The pure PHY behaviours may be simple enough without considering coupling things. The effective time of PDCCH adaptation and the effective time of BWP switch can be two different time point. UE follows the specific time point to perform PDCCH adaptation and/or BWP switch. The spec is not broken. |
| Xiaomi | OK with Conclusion 2.2-1 |
| Ericsson2 | For this issue, we do not prefer specification via only a conclusion in chair notes – any change should be reflected in the specifications. The proposed conclusion also seems to contradict below specification text (from 38.213-h20). Thus, we are not convinced of taking this conclusion. When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. |
| Huawei, HiSilicon | We support the conclusion. The target BWP is active from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI, hence the conclusion is a straightforward way. Based on the discussion in the inputs table, it is clear that companies can have difference views on this. A conclusion is clearly essential and we can align the view/conclusion first and then check whether specification change is needed. |
| Moderator2 | @Ericsson2 In reading the quoted text for PDCCH skipping, it looks the PDCCH monitoring to be skipped is in the active DL BWP. Given the DCI also indicates active BWP change, the active DL BWP is regarded as the target BWP for the active BWP change. If moderator’s understanding is correct, the first PDCCH monitoring to be skipped will be in the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. Since there is no spec change expected, moderator suggests a conclusion. But any specific change that can better clarify the understanding is highly appreciated. Companies are encouraged to provide your understanding on the first slot for this case. When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field.. |
| CATT | We don’t see the need of specification change since the PDCCH skipping duration is configured per BWP. The BWP switching would reset the value of PDCCH skipping to new BWP even it is triggered at the original BWP. |
| Intel2 | As vivo mentioned in first round, current spec procedure works even if active BWP changes. BWP switching delay is larger than the application delay for PDCCH skipping. Although we agree that the text was written considering PDCCH skipping applies to active BWP where indication was received, the procedure does not cause any ambiguity or problematic behavior from UE due to BWP change. If the first slot after the slot where indication is received falls within BWP switch, effective PDCCH skipping from UE perspective is just delayed. So we do not see strong need for conclusion or spec change. |
| Moderator3 | @CATT: Moderator would like to first clarify there is no intended specification change. From reading your feedback, it seems you expect to resume PDCCH monitoring after BWP switch (same behavior as BWP switch is due to BWP timer expiration). But, this interpretation is different from the understanding from other companies (e.g. Huawei, E///, Xiaomi etc.). Given there will be no change to current specification, it will be useful to capture a conclusion to align companies’ understanding. @Intel: If the start time of PDCCH monitoring adaptation is interpreted differently by UE and BS, this issue is different expected timing on resuming PDCCH monitoring by UE and BS, which can cause missing of scheduling. |
| vivo | Sorry for the late reply. From our understanding, the description of current spec of “… a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell…” does not mean UE need to apply PDCCH skipping after the finishing of BWP switching. A UE can know the active BWP is the indicated one right after it receives the BWP indication. Then UE applies PDCCH skipping by following the current spec i.e., “the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field.” Despite we agree with FL that a conclusion is needed for the purpose of aligning the understandi.ng among companies, but in fact we do not share the same view with FL that the current conclusion suggest by FL can be able to avoid causing any spec impact. So, we propose the following version for consideration. Conclusion 2.2-1: For the case a DCI format indicates both PDCCH monitoring adaptation and active BWP change, the first slot UE applies the PDCCH monitoring adaptation is the slot indicated by the slot offset value of the time domain resource assignment field in the DCI at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. |

Moderator would like to companies’ valuable inputs. Further email discussion in RAN1 email reflector cannot reach consensus (please refer THIS LINK to The Email Thread for the complete information of the discussion). The following are finally summarized for companies’ information:

The concerned case is when a scheduling DCI indicates both PDCCH monitoring adaptation and active BWP change. According to clause 12 of TS 38.213, the interpretation of this case is based on the setting of the target BWP, but different companies have different interpretations on the application timing of the PDCCH monitoring adaptation. This could lead to misalignment of SSSG timer/skip duration counting between gNB and UE, and result in data scheduling missing. While everyone seems able to agree no additional specification change needed, there are different views on the current spec text:

1. (HW, Xiaomi) The first slot UE applies the PDCCH monitoring adaptation is the slot indicated by the slot offset value of the time domain resource assignment field in the DCI

2. (Nordic, vivo, Spectrum) The PDCCH monitoring adaptation applies since the first slot after the application delay as described in clause 10.4 of TS 38.213

3. (CATT) No need to handle such case, and UE resumes PDCCH monitoring (as the behavior with BWP timer expiration)

The above are captured in this summary for potential future discussion as per company CR. However, it is a bit strange that companies say spec is clear while there are 3 different interpretations. Moderator would like to encourage the companies to align one common spec interpretation before further raising this issue.

## Correction on Timer Expiration for SSSG Switching [3]

From the CR in R1-2303103 by Samsung, the following are identified for the proposed correction to TS 38.213:

| Reason for change: | - It is ambiguous whether switching to SSSG 0 applies upon timer expiration in searchSpaceGroupIdList-r17 The specifications capture the following: “When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.” and “When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 starting in a second slot that - is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3, - is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6, - is not earlier than a slot where a PDCCH skipping duration expires, if applicable” Although the latter text is generic, the former text is ambiguous as it includes only searchSpaceGroupIdList and does not clarify that the reset is upon the timer expiration. It is suggested to add “if provided by searchSpaceGroupIdList” in the latter text and to remove the earlier text. - The Rel-17 parameter, searchSpaceGroupIdList-r17, is missing from the following statement “If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList.” - Incorrect RRC parameter name for PDCCHSkippingDurationList. |
| --- | --- |
| Summary of change: | - Remove duplicated/incomplete text to clarify applicability of the timer. - Add searchSpaceGroupIdList-r17 as a condition for applicability of the procedures. - Correct RRC parameter name from PDCCHSkippingDurationList to pdcch-SkippingDurationList. |
| Consequences if not approved: | Ambiguous/incomplete specifications, incorrect reference to an RRC parameter. |

where the proposed correction is quoted below:

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text omitted *** If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set. A UE can be provided a set of durations by pdcch-SkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided PDCCHSkippingDurationList, the following procedures related to skipping of PDCCH monitoring are not applicable. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17. A UE can be provided by searchSpaceSwitchDelay a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2. *** Unchanged text omitted *** If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0, if provided by searchSpaceGroupIdList, starting in a second slot that - is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3, - is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6, - is not earlier than a slot where a PDCCH skipping duration expires, if applicable *** Unchanged text omitted ***

In the following table, companies please input your views for deciding this CR:

Table 4 (1st_Round): Companies' views for correction on timer expiration for SSSG switching (CR in R1-2303103 [3])

| Company name | Support to adopt the CR? (Y/N) | Views on the proposed correction |
| --- | --- | --- |
| CATT | N | Current spec is clear. The CR makes it confusion. |
| Samsung | Y | At least the last 2 changes mentioned in the cover sheet should be made – can be included in the alignment CR for Rel-17. The first change should also be made as the current text is ambiguous and duplicated. |
| ZTE, Sanechips |  | For the first change, we don’t agree. The specification of “When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.” is used to determine a SSSG group when searchSpaceGroupIdList is configured but UE not receives a DCI to indicate one group ID. Furthermore, this wording is captured for UE behavior of SSSG switching for NRU in Rel-16. Hence, it can not be deleted. For the second change, “searchSpaceGroupIdList or searchSpaceGroupIdList-r17”, these two parameters have the same meaning in 213, except for the case with ambiguity. So we don’t think the update is needed. For others, we are OK. |
| Nokia1 |  | On the first change, have similar understanding as ZTE, hence change not needed. For the second change “if provided by searchSpaceGroupIdList”, if the text is read together with the afore text in the section, not sure if this is text is necessary. Parameter naming can be considered in alignment CR. |
| OPPO | N | Current one is good enough. |
| vivo |  | Only the modification of “by pdcch-SkippingDurationList” is OK for us. |
| Huawei, Hisilicon | N, the alignment of parameter name can be left to editors. | For the 1st change in the cover sheet, it is mainly for RRC configuration or reconfiguration of SSSG in Rel-16. Therefore, we don't support the 1st change. For the 2nd change in the cover sheet, the corresponding paragraphs are for R16 SSSG switching, and the cellGroupsForSwitchList is introduced for Rel-16 SSG switching. We have never discussed cell group switching in Rel-17. Therefore, we should not have the change#2 in the cover sheet. For the 3rd change, we are fine to align the parameter name. Also note that there are other places need to correct the name. But we think this minor change can be handled by editor. |
| Ericsson1 |  | 1st change – Not support. It is legacy Rel-16 text and applies to RRC (re)configuration case and for NR-U functionality from Rel-16, and hence should not be deleted. 2nd change – OK to consider below change (perhaps as an alignment CR) as it clarifies the “otherwise” branch given the phrase highlighted in blue. The “otherwise” branch should apply to a cell with either r16 or r17 parameter. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17. 3rd change: OK for parameter name update via alignment CR. |
| Intel | N | Current specification seems clear |

Moderator would like to thank companies’ valuable inputs. From the table, it looks only the alignment changes are likely accepted. Accordingly, moderator would like suggest companies’ further review the following proposal for providing a simple alignment TP:

Proposal 2.3-1:

Adopt the following TP to section 10.4 of TS 38.213 for alignment purpose.

- Note: Alignment CR to TS 38.213 to be integrated by editor of TS 38.213

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text omitted *** If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set. A UE can be provided a set of durations by pdcch-SkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided PDCCHSkippingDurationList, the following procedures related to skipping of PDCCH monitoring are not applicable. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17. *** Unchanged text omitted ***

Table 5 (2nd_Round): Companies' views on Proposal 2.3-1

| Company name | View(s) on Proposal 2.3-1 |
| --- | --- |
| Spreadtrum | Fine for FL’s revision. |
| Ericsson2 | OK. |
| Huawei, HiSilicon | For the first one, it is alignment of parameter names. But if this change is made in the TP, the name of PDCCHSkippingDurationList in the second line of the same paragraph should be also corrected. Or would it be better to make a note or conclusion to suggest editor to change all the PDCCHSkippingDurationList in the spec? For the second one, we originally prefer not to change it considering it belongs to Rel-16 feature. However, if we need some change to clarify clearer with respect to “other” branches, we should consider the two following changes also to avoid the extension of current behavior of UEs: - Besides search space group switching, Rel-17 also introduces PDCCH skipping procedure. Therefore, we should make clear that only search space group switching procedures are applicable for this paragraph. Therefore, a condition of “that are based on search space set group switching” for the procedure is added; - We should avoid people mis-understand that cellGroupsForSwitchList could be used with searchSpaceGroupIdList-r17. Therefore, “for which the UE is provided searchSpaceGroupIdList” is added to make clear that only Rel-16 SSSG switching procedure is applicable to all cells within cellGroupsForSwitchList. A suggestion of modification is as following, which we think is essential to avoid the change of scope for Rel-16/17 SSSG switching features. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures that are based on search space set group switching apply to all serving cells for which the UE is provided searchSpaceGroupIdList within each group; otherwise, the following procedures that are based on search space set group switching apply only to a serving cell for which the UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17. |
| Moderator2 | Moderator would like to thank the careful check by Huawei & HiSilicon. There are indeed 3 ‘PDCCHSkippingDurationList’ in clause 10.4 of TS 38.213 that should be fixed altogether. Given companies’ feedbacks in 1st round of discussion indicate only alignment change is acceptable, moderator would like to update Proposal 2.3-1 to focus on the alignment change. Companies please check and comment the following updated proposal: Proposal 2.3-1 (updated): In clause 10.4 of TS 38.213, ‘PDCCHSkippingDurationList’ is corrected to ‘pdcch-SkippingDurationList’ for alignment with TS 38.331. |
| CATT | OK |
| Intel | OK |

Given there is no objection received on the updated proposal, the updated proposal is finally approved by in THIS Email:

Agreement

In clause 10.4 of TS 38.213, ‘PDCCHSkippingDurationList’ is corrected to ‘pdcch-SkippingDurationList’ for alignment with TS 38.331.

## Correction on Interpretation of PDCCH Skipping Indication [4]

From the CR in R1-2303811 by ZTE, the following are identified for the proposed correction to TS 38.213:

| Reason for change: | The case when UE receives one indication of “no skipping in PDCCH monitoring” and another indication of “skipping for a duration” in one slot are not included in TS38.213 h50 that UE does not expect to receive different PDCCH monitoring adaptation indication during application delay. |
| --- | --- |
| Summary of change: | Clarify that UE does not expect to receive one indication of “no skipping in PDCCH monitoring” and another indication of “skipping for a duration” in one slot. |
| Consequences if not approved: | If UE receives a “no skipping in PDCCH monitoring” indication after a “skipping for a duration” indication and these two indications are received in a same slot, UE behavior of PDCCH monitoring is not clear. |

where the proposed correction is quoted below:

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When a UE receives - a first PDCCH in a first slot that provides a DCI format with a PDCCH monitoring adaptation field having a first value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to a search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index, for an active DL BWP and - a second PDCCH that provides a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring, or indicating no skipping in PDCCH monitoring, or indicating start of PDCCH monitoring according to search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index different than the first group index, for the active DL BWP where the second PDCCH is received - in the first slot if the first value indicates skipping PDCCH monitoring - before a slot that is at least Pswitch symbols after the first slot if the first value indicates start of PDCCH monitoring according to search space sets with a first group index the UE does not expect the second value to be different than the first value. *** Unchanged text is omitted ***

In the following table, companies please input your views for deciding this CR:

Table 6 (1st_Round): Companies' views for correction on interpretation of PDCCH skipping indication (CR in R1-2303811 [4])

| Company name | Support to adopt the CR? (Y/N) | Views on the proposed correction |
| --- | --- | --- |
| CATT | N | No skipping is one of the code points in the skipping duration configuration of PDCCH skipping. There is no need to single out the behavior of no skipping. |
| Samsung |  | Same understanding with CATT. |
| ZTE, Sanechips | Y | After UE receives an indication of skipping for a duration, UE does not expect to receive inconsistent indications(e.g., skipping for another duration or not skip). The current specification of the corresponding paragraph only considers “a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring” . The case that DCI indicating “no skipping ” is missed. |
| Nokia1 |  | I think this has also been discussed in the past, but no strong view. |
| OPPO | N | Behavior is clear. |
| Huawei, Hisilicon | Y | We are fine with the CR. |
| Xiaomi | N | Our initial thinking is, why would gNB transmit a “no skipping in PDCCH monitoring” indication and a “skipping for a duration” indication in the same slot? The gNB behavior is unreasonable from our view. We do not see it necessary to define a UE behavior for unreasonable gNB behavior. |
| Ericsson1 | N | This was already discussed in last meeting – we do not see need for this. |
| Intel | N |  |
| Spreadtrum | N or | “a second value indicating skipping PDCCH monitoring” => “a second value for skipping of PDCCH monitoring”, since the title is “skipping of PDCCH monitoring”. Nothing change is also OK. |

Moderator would like to thank companies’ valuable inputs. Given there are 5 of 9 companies explicitly object this CR, moderator suggests no further discussion for RAN1#112-bis-e meeting. Proponent of the CR is encouraged to discuss with the objection companies if desiring to bring this change again.

# Summary

In RAN1#112-bis-e meeting, there achieves the following agreement based on the discussion as captured in Section 2.3:

Agreement

In clause 10.4 of TS 38.213, ‘PDCCHSkippingDurationList’ is corrected to ‘pdcch-SkippingDurationList’ for alignment with TS 38.331.

While companies think current specification description for the case where a scheduling DCI indicates both PDCCH monitoring adaptation and active BWP change is clear, there, however, capture 3 different interpretations in Section 2.2, as quoted below for ease of reference. Moderator would like to encourage the companies to align one common spec interpretation before further raising this issue.

The concerned case is when a scheduling DCI indicates both PDCCH monitoring adaptation and active BWP change. According to clause 12 of TS 38.213, the interpretation of this case is based on the setting of the target BWP, but different companies have different interpretations on the application timing of the PDCCH monitoring adaptation. This could lead to misalignment of SSSG timer/skip duration counting between gNB and UE, and result in data scheduling missing. While everyone seems able to agree no additional specification change needed, there are different views on the current spec text: 1. (HW, Xiaomi) The first slot UE applies the PDCCH monitoring adaptation is the slot indicated by the slot offset value of the time domain resource assignment field in the DCI 2. (Nordic, vivo, Spectrum) The PDCCH monitoring adaptation applies since the first slot after the application delay as described in clause 10.4 of TS 38.213 3. (CATT) No need to handle such case, and UE resumes PDCCH monitoring (as the behavior with BWP timer expiration) The above are captured in this summary for potential future discussion as per company CR. However, it is a bit strange that companies say spec is clear while there are 3 different interpretations.

# Reference

R1-2302456, “Correction on mixed R16 SSSG switching and R17 PDCCH monitoring adaptation”, vivo, RAN1#112-bis-e

R1-2303037, “UE hehavior for PDCCH monitoring adaptation and BWP switching indicated in the same DCI”,	Xiaomi EV Technology, RAN1#112-bis-e

R1-2303103, “Correction on timer expiration for SSSG switching”, Samsung, RAN1#112-bis-e

R1-2303811, “Correction on Rel-17 PDCCH skipping”, ZTE, Sanechips, RAN1#112-bis-e

R1-2302033, “Summary#1 for maintenance on UE power saving enhancements”, MediaTek, RAN1 #112