3GPP TSG RAN WG1 #112b-e	R1-2304269

e-Meeting, Apr 17 - Apr 26, 2023

Agenda Item:	9.11.3

Source:	Moderator (Nordic Semiconductor ASA)

Title:	Collection of LLS results on low power WUS

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

# LLS results

This TDOC captures results submitted to RAN1#102b-e by 14 companies [1][2][3][5][8][9][12][14][15][17][19][20][24][27].

The simulation assumptions along with FAR and MDR/BLER performance are captured in the enclosed Excel document “LP-WUS_LLS_simulation_results_v18_EURECOM_SS_final”.

The observations contributed by companies are in section 4.

# References

- R1-2302332	LP-WUS Physical Signal Design	FUTUREWEI

- R1-2302341	Signal design and procedure for LP-WUS	Huawei, HiSilicon

- R1-2302392	Discussion on low power wake up signal design	Panasonic

- R1-2302409	L1 signal design and procedure for low power WUS	TCL Communication Ltd.

- R1-2302508	Discussion on physical signal and procedure for low power WUS	vivo

- R1-2302572	Design consideration on lower power wake-up signal and procedure	OPPO

- R1-2302622	Discussion on L1 signal design and procedure for low power WUS	Spreadtrum Communications

- R1-2302689	Physical layer signals/procedures and higher layer protocol for Low-Power WUR	CATT

- R1-2302817	Discussions on L1 signal design and procedure for LP-WUS	Intel Corporation

- R1-2302829	Discussion on L1 signal design and procedure for LP-WUS	InterDigital, Inc.

- R1-2302862	On L1 signal design and procedures for low power WUS	Sony

- R1-2302892	L1 signal design and procedures for low power WUS	Nokia, Nokia Shanghai Bell

- R1-2302950	LP-WUS design and related procedure	ZTE, Sanechips

- R1-2302969	Discussions on L1 signal design and procedure for low power WUS	xiaomi

- R1-2303033	Discussion on signal design and procedure for LP-WUS	China Telecom

- R1-2303061	L1 signal design and procedure for low-power WUS	Sharp

- R1-2303152	Signal design and procedure for LP-WUS	Samsung

- R1-2303255	Discussion on L1 signal design and procedure for LP-WUS	CMCC

- R1-2303334	L1 signal design and procedure for low power WUS	MediaTek Inc.

- R1-2303421	Discussion on L1 signal design and procedure for low power WUS	EURECOM

- R1-2303430	Discussion on L1 signal design and procedure for LP-WUS	LG Electronics

- R1-2303507	On the L1 signal design and procedures for low power wake-up signal	Apple

- R1-2303538	On LP-WUS signal design	Nordic Semiconductor ASA

- R1-2303614	L1 signal design and procedures for LP-WUR	Qualcomm Incorporated

- R1-2303673	Discussion on L1 signal design and procedure for LP-WUS	NEC

- R1-2303730	Discussion on L1 signal design and procedure for low power WUS	NTT DOCOMO, INC.

- R1-2303761	L1 signal design and procedure for low power WUS	Ericsson

- R1-2303808	Discussion on the L1 signal design and procedure for low power WUS	Lenovo

# Observations extracted from LLS

[1]

- The performance of conventional OOK LP-WUR can be improve by ~2.5 dB by switching the LP-WUS bandwidth from Option 2 (2.88 MHz) to Option 1 (4.32 MHz).

- Envelope IF signal generation/reception has a ~1.5 dB performance improvement over conventional OOK LP-WUR with the proper selection of IF.

- Envelope IF signal generation/reception can support higher bit rate, i.e., using Option FSK-3, than conventional OOK LP-WUR without a considerable impact on performance.

[2]

- Options OOK-1 and OOK-2 experience marginal performance loss when ADC sampling rate is reduced to 960 kHz. Option OOK-4 sees 2 dB performance loss when sampling is reduced from 1.92MHz to 960kHz.

- For Option OOK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.5, 1.6 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 1.4 dB respectively.

- For Option OOK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.2, 0.4, 0.5 dB respectively.

- For Option OOK-4 with channel model of TDL-C 300ns, the performance loss caused by 1, 2 μs timing error are 2.5, 4.8 dB respectively. Timing error of 4 μs causes error floor. For channel model of TDL-C 1000ns, the performance loss caused by 1 μs timing error is 2.7 dB. Timing error of 2 μs causes error floor.

- If the guard bands can cover frequency offset, Options OOK-1, OOK-2 and OOK-4 are insensitive to frequency offset.

- ZC sequence-based generation of FSK/OOK waveform outperforms significantly the OOK modulation with rectangular waveform in fading channel.

- For OOK-4, concentrated OOK provides0.5dB gain for ideal timing case compared with the normal sequence-based waveform because ISI between OOK symbols are mitigated

- Concentrated OOK significantly improves robust against timing error, e.g. at least 2dB improvement under 2us timing error.

- Concentrated waveform is applicable to OOK and FSK to improve their respective performance.

- Both FSK-1 and FSK-2 sees no performance loss when ADC sampling rate is reduced to 960 kHz.

- For Option FSK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 2 dB respectively.

- For Option FSK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.5, 1.7 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.6, 2.4 dB respectively.

- If the guard bands can cover frequency offset, Options FSK-1, FSK-2 are insensitive to frequency offset.

- For sequence-based detection, it can be observed that the performance would be degraded by 0.5dB due to the phase noise and I/Q imbalance when 120 uW oscillator is used.

- Sequence-based OOK/FSK modulation has better coverage performance than energy detection, even if phase noise and I/Q imbalance, frequency error within 10ppm and 1us timing error are considered.

[3]

- Both OOK-2 and FSK-1/2 based LP-WUS with small M can provide workable link performance under the used simulation assumption, at least with 24 PRB in 14 symbols .

- When the value of M is high, in higher SNR, FSK-1/2 can provides better BLER performances than OOK-2 and FSK-1 has best performances among them because of the usage of relative comparison.

- In higher SNR region, regardless of M , BER 1-to-0 of OOK-2 is better than that of FSK-1/2. This is because energy per 1 segment of OOK-2 is higher than FSK-1/2.

- Regarding FSK-1/2, FSK-1 has better BLER/BER performance frequency usage efficiency than FSK-2 under the conditions of same transmit bits and LP-WUS bandwidth.

- OOK with fixed threshold is more susceptible to frequency errors than FSK.

- FSK-1 has more tolerance to frequency error than FSK-2 and OOK.

- Target Oscillator max frequency error should be clearly defined and optimal design of filter passband and GB for it can be studied.

[5]

- For sequence-based OFDM signal, e.g., ZC sequence, the detection performance is more sensitive to frequency error compared with OOK waveform.

- For LP-WUS structure with preamble using OOK waveform, the preamble with at least 16 chips can achieve reliable synchronization performance.

[8]

- OOK-1 brings better link performance than OOK-2 due to the high frequency diversity in OOK-1.

- The SCS has negligible impacts on the link level performance due to the frequency diversities remains the same within the BW of LP-WUS regardless of the SCS used.

[11]

- To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

[12]

- The performance of multi-bit OOK degrades significantly in comparison with that of AWGN channel due to multi-path effect as expected.

- Oversampling at the LR is essential to improve the performance of OOK reception and to reduce the probability of false alarm.

- Timing accuracy requirement for OOK detection is quite restrictive, which can only be achieved by using a preamble associated with each LP-WUS payload if LP-WUS synchronization has long periodicity.

- The LP-WUS modulation should be resource efficient accounting need for possible guard bands, device BW restrictions and efficient multiplexing with other LP-WUS and other legacy signals.

- At least few bitwidth for ADCs, i.e., 4 or above, should be considered for evaluation as it benefits both the LR and the network power saving target.

- FSK receivers are nothing but parallel OOK receivers. Thus the total power consumed by the LR may be doubled in most of the implementation. However, if FSK based LP-WUS signal uses same SCS as NR transmission, then it can be easily received and decoded if in case MR monitors LP-WUS under more stringent radio conditions when LR is not applicable.

- Due to the narrowness of ON segment in FSK-1/2 schemes, they may suffer from fading, crystal stability of LR, and can also cause PAPR problems at the gNB, if the total transmit power is allocated to a narrow section within LP-WUS BW

- LP-WUS using sequence-based CP-OFDM achieves better coverage and performance than the coded transmission. Multiple sequence or sequence properties can used to increase the information rate.

[13]

For Option OOK-4, based on the restriction of constant transmitting energy within LP-WUS payload transmission,

  - For the same code rate, the priority of value of M is (M=4) > (M=8) > (M=2);

  - For the same value of M, the priority of Manchester code rate is (Code Rate=1/2) > (Code Rate=1/4)

- LP-WUS detection performance with ADC bits number >= 4 is very close to that of Ideal ADC.

- For LP-WUS with bandwidth of 4.32MHz (i.e.,12 RBs with SCS=30KHz),

  - LP-WUS detection performance with sampling rate=3.84MHz has a significant gap to that of No downsampling;

  - LP-WUS detection performance with sampling rate=15.36 or 7.68MHz is very close to that of No downsampling.

- For LP-WUS detection, guardband of 11SCs can mitigate up to 3dB ACI.

- For LP-WUS detection, Guardband=24SCs@ACI=10dB has only 3dB performance loss at BLER=1% compared with the case of Guardband=11SCs@ACI=0dB.

- Under the restriction of constant time-frequency resources and transmitting power, assuming No Time/Frequency error,

  - For Option OOK-1, since CP is added for each OOK symbol, ISI caused by multi-path delay of fading channel can be solve and the performance of the Option OOK-1 is slightly better than that of the Option OOK-4.

  - For Option OOK-2, since the bandwidth of LP-WUS is obviously reduced, the envelop performance will be obviously affected. Therefore, LP-WUS detection performance will be degraded obviously.

- For LP-WUS with Option OOK-1, OOK-2 and OOK-4, the capability of mitigating impact of frequency error is, Option OOK-1 > Option OOK-4 > Option OOK-2.

- Under the restriction of constant time-frequency resource allocation and same transmitting power, if no time/frequency error is assumed, the MDR of longer LP-WUS preamble sequence and shorter LP-WUS preamble sequence are nearly the same.

- For MC-FSK with parallel homodyne receiver architecture, if M>=2, MC-FSK detection performance will be degraded obviously.

- It is observed that the link level performance at BLER=10% based on receiver 1 and receiver 2 is almost same for the case of the ZC sequence length of 23 under ideal cases.

- For ZC sequence-based OFDM, the link level performance is not impacted by the aspect of sequence correlation in frequency domain or in time domain.

- It is observed that the SNR values at BLER=10% based on OFDM receiver 1 without down sampling and with down sapling factor of 1/4 are almost same for the case of ZC sequence length of 23.

- It is observed that the SNR values at BLER=10% based on receiver 1 decreases with the increase of the length of ZC sequence of 23, 47 and 83.

- The link level performance is seriously impacted by the aspect of frequency offset for sequence-based OFDM receiver.

[15]

- Demodulation performance over 6RB bandwidth no longer offers significant performance gain.

[19]

- Shows that at 200ppm frequency error OFDMA signal (PSS) with non-I/Q OOK receiver can be received with the same performance as MC-ASK. MR may help with preprocessing of PSS to look like OOK.

[20]

- In AWGN, decreasing the number of SCs of OOK symbols improves performance because less noise if captured.

- OOK-2 cannot fully exploit frequency diversity because OOK symbols are confined to a subset of the WUS bandwidth.

- Given the same bandwidth, OOK-1 is more robust to ACI than OOK-4 because the interference is averaged over more samples.

- Given ideal AGC, an ADC with 4-bit resolution is sufficient for close to optimal performance.

- For OOK-1, Manchester coding improves robustness to timing inaccuracies.

- OOK-4, shorter the OOK symbols increase sensitivity to timing inaccuracies.

- OOK-3 has good performance, but requires low frequency error.

[24]

- at the same data rate, OOK (with Option OOK-4) is more robust against frequency error than FSK (with Option FSK-1), and FSK is more robust against timing error than OOK.

- For MC-OOK (with Option OOK-4) and MC-FSK (with Option FSK-1), the sampling rate at the receiver needs to be set according to the bandwidth of the effective signal, which can be much larger than the information/data rate of the OOK/FSK transmission.

- at the same data rate, MC-OOK (with Option OOK-4) is more robust against frequency domain selectivity (i.e., having a larger frequency diversity gain) than MC-FSK due to larger effective bandwidth.

[27]

- For modulating ON symbols in OOK WUS, a sequence and random QPSK provide the same BER performance for the same signal power.

- Adjacent cell interference can result in a significant coverage degradation when the interference is stronger than the signal.

- Increasing ADC bit width is beneficial in terms of coverage, interference mitigation, and dynamic range of signal reception.

- The coverage performance of single-bit ADC is extremely poor.

- UE speed has a minor impact on the link performance considering a relatively large channel coherence time.

- For non-coherent WUS detection, every repetition factor of 2 improves the link performance by around 2 dB.

- For payload-based OOK WUS a relatively large number of WUS repetitions might be needed to match the PDCCH coverage. The total WUS duration with repetition is in order of tens of slots

- For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).  [27]

- Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.