3GPP TSG RAN WG1 #112bis-e		                                                       R1-2304282

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.17

Source: 	Moderators (AT&T, NTT DOCOMO, INC.)

Title: 	Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e

Document for:	Endorsement

- Introduction

This contribution includes the initial RAN1 UE features list for Rel-18 NR after RAN1 #112bis-e meeting.

- NR_MIMO_evo_DL_UL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_netcon_repeater

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. NR_NTN_enh | 44-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. NR_NTN_enh | 44-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. NR_NTN_enh | 44-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_UAV

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_UAV | 46-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 47. NR_SL_enh2 | 47-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 47. NR_SL_enh2 | 47-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_redcap_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 48. NR_redcap_enh | 48-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 48. NR_redcap_enh | 48-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}. FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}.] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 | [At least one of {49-1, 49-1b, 49-2, 49-2b}] | Yes |  |  | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | [At least one of {49-1, 49-1b, 49-2, 49-2b}] | Yes |  |  | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5 | Type 2 HARQ CB support for DCI format 1_3 | HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination] | [Per band pair per band combination, details up to RAN2] | [N/A] | [N/A] | [N/A] | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 [Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination] [Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {[0us], 500us} - The reported value X is applied to both one TAG case and two-TAG case (if UE supports two-TAG case) FFS: Note: If the UE reports [0us], the minimum separation time is not applied | 49-X | Yes |  | [two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands] | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |

- NR_XR_Enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_FR1_lessthan_5MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 51. NR_FR1_lessthan_5MHz_BW | 51-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 51. NR_FR1_lessthan_5MHz_BW | 51-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- NR_DSS_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-1 | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern [via lte-CRS-ToMatchAround] [2) PDCCH-DMRS channel estimation Value 1: PDCCH-DMRS channel estimation based on legacy CE assumption Value 2: for CORESET duration greater than 1 symbol, PDCCH-DMRS channel estimation based on PDCCH-DMRS REs in symbols not overlapping with LTE CRS Other values FFS] [3) Reception of NR PDCCH candidates that overlap with LTE CRS REs only on the 2nd symbol of an NR slot] [4) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within the first 3 OFDM symbols of a slot] | 5-28 | Yes | N/A | UE is not required to support reception of NR PDCCH candidates overlapping with LTE CRS REs when it is provided with LTE CRS RM pattern by higher layers | [Per Band] | [No] | [No] | [N/A] | [Component 2 candidate value set: {Value 1, Value2, other values FFS}] [Component 3 candidate value set: {Value 1, other values FFS}] [Note: From UE perspective, PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15] [Note: For component 2, RAN1 consider support legacy CE only if no RAN4 performance requirements are defined for this option] Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1a | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of multiple non-overlapping CRS rate matching patterns [via lte-CRS-PatternList1-r16] | 52-1, 14-1 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] | Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 1) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 52-1, 22-12 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] | 1) Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS: {2,3,4,5,6} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 6 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS: {1,2,3} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 3 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | [5-28] | Yes | N/A | [UE support for two overlapping LTE-CRS RM patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier when UE is not supporting or configured with multi-TRP is not enabled] | [Per Band] | [No] | [No] | [N/A] | FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | Optional with capability singaling |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier FFS additional clarification of relation between Rel-16 capability | 52-2, 16-2a | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability singaling |
| 52. NR_DSS_enh | 52-3 | FFS: Support reception of NR PDCCH candidates overlapping with LTE CRS REs when two LTE-CRS overlapping rate matching patterns are configured |  | 52-1, at least one of 52-2 or 14-1a | Yes | N/A |  | [Per Band] | No | No | N/A |  |  |

- BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell | [1-7, 2-24, 2-31, 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of [the corresponding carrier(s) to be measured.] [UE indicates at most one of FG 53-1 and 53- 2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per band | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e]

- TEI18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-1 | additionalSR-Periodicities-r18 | Indicates whether the UE supports the following SR periodicities in the periodicityAndOffset parameter as specified in TS 38.331: -5sl for 30 kHz SCS -5sl and 10sl for 120 kHz SCS Candidate values {30 kHz SCS, 120 kHz SCS, both} |  | Yes | N/A | If the network implements the TS 38.331 CR on new SR periodicities and the UE does not according to the capability indication, the network will not assign the new SR periodicities. Legacy behaviour applies. | Per UE | No | No | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-2a | 1-symbol PRS for MG-based measurement in RRC_CONNECTED state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot inside a MG in RRC_CONNECTED state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 13-1 | No | n/a | 1-symbol PRS is not supported for MG-based measurement in RRC_CONNECTED state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2b | 1-symbol PRS for outside MG in RRC_CONNECTED state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot outside a MG in RRC_CONNECTED state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-3 | No | n/a | 1-symbol PRS is not supported for outside MG in RRC_CONNECTED state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2c | 1-symbol PRS in RRC_INACTIVE state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot in RRC_INACTIVE state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-6 | No | n/a | 1-symbol PRS is not supported in RRC_INACTIVE state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2d | 1-symbol PRS for PDC | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot for PDC FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 25-19a | Yes | n/a | 1-symbol PRS is not supported for PDC | per band | n/a | n/a | n/a |  | Optional with capability signaling |