3GPP TSG RAN WG1#112bis-e															         R1-2304287

e-Meeting, April 17th – April 26th, 2023

Source:	Moderator (vivo)

Title:		Final summary of evaluation methodologies on LP-WUS/WUR

Agenda Item:	9.11.1

Document for:	Discussion and Decision

Introduction

This document summarizes the contributions [1 - 19] for AI 9.11.1 and email discussions.

The issues in this document are tagged and color coded with [H] or [M].

Please refer to section 5 for remaining issues.

Evaluation methodologies

Issue 1: Simulation assumptions

- 1A: Link-level assumptions

- 1A-1: FAR/MDR

In last meeting, following agreements are made, and definition of FAR is still not converged.

Agreement - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials

Summary of company views:

Alt 1: FAR target is determined per single WUS attempt/trial,

- Supported by Futurewei (For Coverage evaluation), OPPO, interdigital, LG (for duty cycle monitoring), Ericsson, MTK

Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials

- Supported by Futurewei (For power evaluation), Huawei, vivo, LG (for continuous monitoring), ZTE, Samsung

Proposals from contributions are as follows,

| Futurewei | - Number of false alarms in any duration T cannot simply be determined as the product of FAR and number of LP-WUS monitoring occasions in that duration T for ‘always-on’ and short ‘duty-cycled’ monitoring modes. - For power evaluation, only one false alarm is expected in any duration corresponding to MR’s wake-up, determination of false alarm, and return back to sleep. - Support definition of a simulation step size of, e.g., one subframe, for the evaluation of ‘always-on’ LP-WUS power saving gain using system level simulation, where the step size corresponds to the LP-WUS duration. - For the same FAR and reference time duration in FAR Alt 2, LP-WUS design under ‘always-on’ monitoring may require a longer sequence and/or a longer CRC compared to the LP-WUS design under ‘duty-cycled’ monitoring. - For power saving gain, support using FAR Alt 2 to simplify the evaluation and consider the impact on LP-WUS design for ‘always-on’ and ‘duty-cycled’ monitoring modes. |
| --- | --- |
| Huawei | - if the FAR value per single WUS attempt/trial is p, and during a time duration there are N attempts/trials, the joint FAR value for this time duration is 1-(1-p)N since the attempt/trials are independent. - Alt.2 can be generally used to compare different designs, e.g. the continuous monitoring and duty cycle based monitoring. The main impact of FAR is the false-wakeup of MR which consumes additional power. By setting the FAR target within a duration for different modulation/waveform or operation mode (e.g. continuous monitoring or duty cycle mode monitoring), the power saving gain can be aligned by maintaining proper FAR target with in the same duration for fair comparison of other metrics, e.g. miss-detection performance of the LP-WUS. |
| vivo | - Reporting per attempt FAR is not necessary - UE power consumption mainly depends on reference duration FAR. - The detection results from multiple attempts may be highly correlated, per attempt FAR cannot be easily derived from the per reference duration FAR. - The reference time duration can be T, e.g., one DRX cycle, during which UE detect N WUS occasions, - For always on monitoring, N>1, is the number of occasions within T. - For duty cycled monitoring, value N depends on WUS periodicity. N=1, if reference time duration is WUS periodicity. |
| OPPO | For evaluation of power consumption, FAR target is determined per single WUS attempt/trial. |
| interdigital | Alt 1 provides a simple FAR target per single WUS attempt/trial, Alt 2 defines an FAR target across a reference time duration. Alt 2 requires further discussion on possible values for reference time durations as captured as FFS, benefits of Alt 2 are not clear. |
| LG | For evaluation, only one alternative for FAR should be considered for each monitoring behavior of LP-WUR. E.g., Alt 1 for duty cycle based monitoring and Alt 2 for continuous monitoring. |
| Ericsson | False alarm probability is defined per detection attempt/trial. And if each WUR detection duration is D ms, then during T ms active time the WUR has [T/D] attempts for detecting WUS, where false alarm can occur in each attempt with a certain probability. |
| ZTE | FAR P is defined in a time duration T - The FAR for each detection is LLS is less than x0, where x0 is the FAR for the detection when the step size for sliding is the sequence length L, satisfying P=1-1-x0T/L - The reference time duration is a DRX cycle. - MDR has the similar definition. |
| Samsung | - When the finer sliding granularity for LP-WUS detection is considered, how to count the number of wake-up can affect to calculate FAR. - For calculation of FAR, FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - The number of wake-up within the reference time is 1 even if the multiple wake-ups are indicated by multiple attempts/trials. - Propose to consider three cases for something that gNB can transmit within the LP-WUS monitoring window: 1) LP-WUS, 2) Other NR signal(s)/channel(s) (PDSCH, PDCCH, …), 3) the absence of gNB transmission (only considering channel noise). |
| MTK | - Clarify the baseline of [0.1%, 1%] for the false-alarm rate (FAR) of LP-WUS agreed in RAN1#112 is defined by FAR per single WUS attempt/trial, otherwise, non-feasible FAR targets will be included. - Support of Alt 2 (FAR target is determined across a reference time duration of one or multiple WUS attempts/trials) may result in the target FAR values below 1% or below 0.1%, which may not be feasible if there is no CRC for further confirmation. |

Based on company inputs, moderator would like to align the understanding on UE behaviors and terminologies to facilitate further discussion and down selection on Alt-1/Alt-2

Firstly, moderator would like to align the understanding on detection behavior for duty cycle and continuous monitoring for LP-WUS.

- Time drifting: considering time drifting, UE may start detection of LP-WUS earlier than the actual transmission occasion of LP-WUS. For LP-WUS with sequence part, either sequence only or with subsequent payload, UE performs correlation to search for the sequence part within certain sliding window, in which the correlation is performed multiple times

- Continuous monitoring: It means WUR does not know when gNB will start transmission of LP-WUS, and UE monitors the LP-WUS in continuous manner, UE will monitor the LP-WUS multiple times within certain durations. Note that, although UE monitors continuously, it does not mean gNB can start WUS transmission anytime, e.g., sample Ts level. The gNB transmission still follows existing time domain resource grid, e.g., slot can be considered transmission unit for a LP-WUS for evaluation purpose. In other words, the continuous monitoring is actually short duty cycled monitoring. Thus, the number of LP-WUS transmission occasions can be obtained within a time duration.

Further, moderator would like to align following understanding and terminologies among companies.

- Per single WUS attempt/trial, means ONE trial/attempt to detect ONE LP-WUS transmission occasion.

  - The output of Multiple correlations/hypothesis may be highly correlated due to oversampling of OOK chips. Hence, it is difficult to derive per correlation FAR from per attempt FAR. [R1-2302948, ZTE], [R1-2303150, Samsung], [R1-2302526, vivo]

  - Note: If UE performs multiple correlations for detection the potential LP-WUS transmission in that transmission occasion, it can be considered as UE implementation in ONE trial/attempt, rather than multiple attempts/trials.

- Number WUS attempts/trials across a reference time duration, if each detection duration/LP-WUS occasion duration is D ms, then during T ms reference time duration, and the WUR has [T/D] attempts for detecting WUS.

  - Similarly, each detection duration/LP-WUS occasion duration may require multiple correlations/hypothesis at WUR receiver.

  - Similar view can be found in [R1-2302339, Huawei], [R1-2303759, Ericsson], [R1-2302506, vivo], [R1-2302948, ZTE], [R1-2302331 FUTUREWEI].

Based on above clarification, moderator have the following understanding

- For duty cycled LP-WUS monitoring, and if the reference time duration is equal to LP-WUS periodicity and only one LP-WUS transmission per LP-WUS period, UE only need to monitor LP-WUS once. In this case, the per single attempt FAR(Alt-1) is identical to per reference duration FAR(Alt-2).

- For always-on/continuous monitoring, if false alarm occurs at any occasion within the reference time duration, UE would wake up. Hence, Alt 2, FAR target determined across a reference time duration of one or multiple WUS attempts/trials is more intuitive for UE power consumption for Always on/Continuous monitoring.

- For always-on/continuous monitoring, each attempt on different WUS occasions can be assumed as independent, the per single attempt FAR and per duration FAR can be derived from each other.

- The per attempt FAR would be varied if companies report different assumptions on the duration for each attempt, which depends on WUS configurations. FAR target determined across a reference time duration of one or multiple WUS attempts/trials can be more easily aligned.

For the reference time duration used for evaluation

  - Typical DRX cycle can be assumed as the reference time duration. [R1-2302506, vivo], [R1-2302339, Huawei], [R1-2302948, ZTE].

A brief depicts of FAR calculation Alt-1 and Alt-2 is provided in the following figure.

Hence, moderator have the following proposal which covers both duty-cycled and continuously monitoring,

- [H] Proposals 1A-1-v1:

For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials,

- UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T.

  - N is the number of attempts within T.

  - where T is {1.28s, 2.56, …}

  - Company to report (FAR, T, N)

    - Note: FAR = {0.1%, 1%} as agreed in RAN1#112

- Note 1: For example, if UE performs multiple correlations for sequence part for potential LP-WUS transmission in that monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.

- Note 2: If UE performs multiple non-overlapping attempts within the reference time duration, the false alarm event for the attempts are assumed as independent.

- Note 3: Number of Attempts per second (λ) can be calculated from T and N, i.e., λ=NT.

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the proposal. |
| OPPO | OK with the proposal. |
| Spreadtrum | This FAR definition may be used for continuous monitoring. We don’t see the need to change the FAR definition for duty cycle. For evaluation purpose of continuous monitoring of LP-WUS, |
| Huawei, HiSilicon | We are fine in general with moderator’s analysis and the proposal, with some following: - Considering it was not agreed that LP-WUS always includes a sequence/preamble, therefore Note 1 should be updated as e.g.: Note 1: For example, if LP-WUS transmission is agreed to include a sequence/preamble part, for which UE performs multiple correlations for sequence part for potential LP-WUS transmission in that monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. |
| QC | Agree on this proposal. With this proposal, we think we have no need to discuss Alt 1 and Alt 2 any more. We think correlation can be done to detect a sequence only (which does not necessarily require preamble). |
| ZTE, Sanechips | We are generally OK. Additionally, in a time duration, we think 10% also can be optionally considered. In our simulation, in some cases, FAR 10% still has some power saving gain. |
| Xiaomi | Fine with the proposal. |
| Nokia1 | As discussed in last meeting, we should have some common understanding of the side-conditions that are needed FAR e.g. in terms of LP-WUS design. The assumed design for (FAR=0.1%, T=1.28s, N=1) would be very different that the design (FAR=0.1%, T=1.28s, N=1280). Thus if the resulting false alarm probability per one detection attempt (of the N) is very low, the assumed side conditions should also be disclosed. Minor note that N is defined at end of first sub-bullet as number of transmissions occasions, and in second sub-bullet (and implied start of first sub-bullet) as number of attempts within T. While these could be the same, would it be simplest just define N as the number of WUS detection attempts within T. |
| Intel | We are OK with the proposal. Regarding N, we’d like to clarify whether N is non-overlapped attempts. In our understanding, if N can be overlapped attempts, there is no simple relation between per-attempt FAR and the FAR in duration T. In other words, reporting N in the 3rd sub-bullet doesn’t help, if N can be overlapped attempts. |
| InterDigital | We understand the point of the FL, but the proposal needs some clarification. In our understanding, Note 1 and Note 2 are important information, so they shouldn’t be notes but a part of agreements. In addition, “N is the number of attempts within T.” is duplicated information. Therefore, it’s better to remove as well. Having said that, the below is our updated proposal. - [H] Proposals 1A-1-v1: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T. - N is the number of attempts within T. - where T is {1.28s, 2.56, …} - Company to report (FAR, T, N) - Note: FAR = {0.1%, 1%} as agreed in RAN1#112 - Note 1: For example, iIf UE performs multiple correlations for sequence part for potential LP-WUS transmission in that monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - Note 2: If UE performs multiple non-overlapping attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. - Note 3: Number of Attempts per second (λ) can be calculated from T and N, i.e., λ=NT. |
| Samsung | For Note 1, our understanding is that multiple attempts for single LP-WUS transmission due to time drifting are treated as single attempt and the wording “monitor occasion” is not clear to us. Therefore, we suggest to modify Note 1 for clarification as follow: - Note 1: For example, if UE performs multiple correlations for sequence part for potential single LP-WUS transmission in that monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. For Note 2, we would like to clarify the definition of non-overlapping attempts. Considering the Note 1, non-overlapping attempts can be defined as attempts performed for different potential LP-WUS transmission (occasion). In our understanding, it does not mean that LP-WUS detection duration is not overlapped in time domain. For example, it is assumed that LP-WUS can be transmitted from the first OFDM symbol for every slot and time duration for LP-WUS is longer than 1 slot, then multiple attempts for different LP-WUS occasion can be overlapped in time domain. Is our understanding correct? For understanding the figure to explain FAR calculation, we would like to check that “FAR across k monitor occasion” in (c) can be calculated using the same equation in (a) e.g., 1%=1-(1-x)k |
| CATT | We are not OK with Proposals 1A-1-v1. - Regarding the align the understanding on detection behavior: Per our understanding, monitoring behavior is related with the behavior of receiver. The difference between duty cycle monitoring and continuous monitoring is whether UE LP-WUR would turn ON/OFF at each duty cycle or stay ON to monitor at the event. - The false alarm probability is the collected event of false outcome of wakeup when the transmitted LP-WUS is to indicated not to wakeup over total events. Thus, we don’t need to define a reference time duration |
| Apple | We are generally fine with proposal in principle. But Note 1 is confusing for continuous monitoring. |
| LGE | We don’t have strong preference for which Alt should be used but think only one Alt should be used for each monitoring behavior for fair evaluation. We are okay with moderator’s analysis and proposal. As moderator analyzed, we also think duty-cycle monitoring is super-set of continuous monitoring. So, if we can evaluate both monitoring behaviors with common definition of FAR target, it would be the best for collecting observations. |
| FL | @ZTE, in RAN1#112, we agreed the followings - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies So perhaps we can remove Note: FAR = {0.1%, 1%} as agreed in RAN1#112 @Spreadtrum, to address your comment, can we add a note saying it is at least for @Huawei, I modified the note 1 to be more generic. Hope that address your comment. @Nokia, InterDigital, removing “N is the number of attempts within T.”, it is duplicated information. @Nokia, Yes, side conditions can be reported by other assumptions such as payload, CRC and etc. and I think it is part of the LLS assumptions which has been included in the Karol’s provided excel sheet. @Samsung, Intel, regarding non-overlapped attempts, it does not mean the duration can or cannot be overlapped. @ InterDigital, removing note according to your suggestion. @CATT, I understand your intension. If companies want’s to keep the same FAR for each attempts irrespective of the transmission occasions, the current proposal does not preclude to do so. For example, for duty-cycled monitoring, you can report (FAR=0.1%, T=1.28s, N=1), and for continuous monitoring, you can report - (FAR=0.1%, T=1ms, N=1) when LP-WUS monitoring occasion are back-to-back monitored with each duration is 1ms, or - (FAR=0.1%, T=1.28s, N=100) when every 100ms out of 1.28s are continuously monitored for potential LP-WUS. Not sure which continuous monitoring case you are referring, but the current proposal does not preclude to do so. @Apple, perhaps we can take note 1 as example. If there are particular unclear thing for continuous monitoring, we can fixed it later. Please see a revised version Proposals 1A-1-v2 |

- [H] Proposals 1A-1-v2:

For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials,

- UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T.

  - N is the number of attempts within T.

  - where T is {1.28s, 2.56, …}

  - Company to report (FAR, T, N)

    - Note: FAR = {0.1%, 1%} as agreed in RAN1#112

- Note 1: For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.

- Note 2: If UE performs multiple non-overlapping attempts within the reference time duration, the false alarm event for the attempts are assumed as independent.

- Note 3: Number of attempts per second (λ) can be calculated from T and N, i.e., λ=NT.

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the proposal. |
| Ericsson1 | We have below comments: - “monitor occasion” needs clarification – is it from UE perspective and if so, how it is linked to N transmission occasions within time duration T? - “non-overlapping attempts” and link to monitor occasion needs clarification – does it refer to overlap/not of “time domain samples” used in a detection attempt? If so, also need to clarify partially overlapping case? - What is considered as “ONE trial/attempt” should be left for proponent to describe with justification. |
| Spreadtrum2 | Fine. Sorry, I miss that N=1 means duty cycle based monitoring. |
| OPPO | OK with the proposal. |
| Samsung | We are fine with the proposal. |
| CATT | We thank FL’s responds and explanation. Per our understanding, T can be 1ms regardless of the monitoring behaviors. If we are the only company has concern, we can accept the proposal with following updated version: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T. - N is the number of attempts within T. - where T is {1.28s, 2.56, …}0. Other values are not precluded for evaluation. - Company to report (FAR, T, N) - Note: FAR = {0.1%, 1%} as agreed in RAN1#112 - Note 1: For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - Note 2: If UE performs multiple non-overlapping attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. - Note 3: Number of attempts per second (λ) can be calculated from T and N, i.e., λ=NT. |
| MTK | Okay for evaluation. Maybe “Note 3: Number of attempts per second (λ) can be calculated from T and N, i.e., λ=N⁄T.” is not needed. |
| vivo | @ Ericsson For monitoring occasion, in our understanding, it is the same as the transmission occasion, but defined from UE perspective. For ‘the non-overlapping attempts’ issue, in our understanding, it is similar to the question from Samsung in last round ‘whether the different LP-WUS occasion can be overlapped in time domain?’ Such overlapped occasions may lead to overlapping samples across different occasions, however, the false alarm event for the attempts can be still assumed as independent with proper WUS channel design. Although we consider this as corner case, we can be flexible to include this case, with change mark in blue. ##### [H] Proposals 1A-1-v2: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T. - N is the number of attempts within T. - where T is {1.28s, 2.56, …} - Company to report (FAR, T, N) - Note: FAR = {0.1%, 1%} as agreed in RAN1#112 - Note 1: For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - Note 2: If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. - Note 3: Number of attempts per second (λ) can be calculated from T and N, i.e., λ=NT. For third comment from Ericsson, we feel that some alignment on terminology is needed, since in last meeting, companies have varied terminologies, understanding of attempt/correlation is not aligned among companies, this example provided here can help to clarify the difference between one trial/attempt and one sequence correlation. Companies can report more detailed information on what is performed within one trial/attempt, e.g., how many correlations are performed within one attempt to detect the sequence of the LP-WUS. Other details on receiver procedure/algorithm/settings can be reported as well. |
| Nokia2 | We would also propose to add a note in the context of power saving evaluations that companies provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design. |
| Intel | We are fine with the proposal. note 3 seems straightforward and is not necessary. |
| Huawei, HiSilicon | We are fine. |
| ZTE, Sanechips | Maybe the example could be removed to address Huawei’s concern? We are also not clear about this. |
| FL2 | By addressing CATT, vivo, Ericsson, MTK, Nokia’s comment, FL suggest the following modifications, see Proposals 1A-1-v3. The last added bullet from Nokia’s comment. |

- [H] Proposals 1A-1-v3:

For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials,

- UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T.

  - N is the number of attempts within T.

  - where T is {1.28s, 2.56, …}. Other values are not precluded for evaluation.

  - Company to report (FAR, T, N)

    - Note: FAR = {0.1%, 1%} as agreed in RAN1#112

- Note 1: For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.

- Note 2: If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent.

- Note 3: Number of attempts per second (λ) can be calculated from T and N, i.e., λ=NT.

Power saving evaluations that companies provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design

| Company | Comments |
| --- | --- |
| Spreadtrum3 | Fine |
| CATT | Firstly, we thank the update on the first sub-bullet. But it still confuses us. From the current wording, it means that the T can be any values e.g. 1.28second, 2.56 seconds, 1millisecond, or 1second, right? Thus, We suggest that the sub-bullet can be removed and do not give any restrict on T values if we want consider T. We think companies report the values of FAR, T and N is enough. Again, we want clarify that the definition of T for FAR determination is not needed technically. From probability theory, FAR is the ratio of error event and the total event. It is not related with any time definition |
| InterDigital | Fine |
| OPPO | Fine. |
| Apple | We are fine with the principle. However, we still think the following is confusing: “For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.” This does not seem necessary, as long as the FAR over the time duration T is reported. Does it make any difference if the UE considers it as a single attempts or multiple attempts? For the bullet “If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent.”: the revision is unclear to us. The original version seems to make more sense, to assume they are independent if they are non-overlapping. But if there is significant overlap between two attempts, it may not be appropriate to assume they are independent any more. This is especially true for the case with continuous monitoring. The question is: do we really need these two bullets? Can we simply let companies report all the details for now, e.g., including how the FAR for the duration T is derived? |
| ZTE, Sanechips | Fine. Maybe the following can be modified to address Apple’s second concern. If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. For the first one, it can be deleted and up to company report. |

- 1A-2: Frequency error/drifting

In RAN1 #112 meeting, the following working assumption was made [1].

Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4, - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option 3/4 is not applicable - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than [TBD] power unit, - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR. - assumptions important for achieving performance by using MR clock for LR should be declared - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm). - Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr - Model 2: random time drifting, FFS details - FFS: Phase noise model

Huawei: For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is:

  - 0.1 ppm, if MR can assist to calibrate LP-WUR to correct the frequency error

  - 5 ppm, if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

Vivo: Option 3/4 cannot be used when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state.

Interdigital: For high frequency error/drift case, there may be increased power consumption and therefore the agreed power consumption for MR and LP-WUR may not be applicable. However, there is no clear evidence that only Option 3/4 are not applicable with the current assumption. In that regard, applying the current assumptions for Option 3/4 as well as Option 1/2 is preferred.

Qualcomm: The accuracy of clock mostly depends on the cost or clock. The clock with higher power accuracy does not necessarily consumes higher power. Therefore, we suggest removing following text from working assumption. The frequency of clock correlates more with power consumption. So, it would be good to remove following FFS on option 3 and 4.

- [H] Proposals 1A-2-v1:

For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is:

a) 0.1 ppm, if MR can assist to calibrate LP-WUR to correct the frequency error

b) 5 ppm, if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the proposal |
| OPPO | OK with the proposal. |
| Huawei, HiSilicon | We support the proposal. |
| QC | It is not clear why the Fr should be modeled as 0.1ppm or 5ppm for each case. The Fr depends on LP-SS detection and clock calibration mechanism. So, we think these should be reported by individual company. |
| ZTE, Sanechips | We are fine with the main bullet. For the subbullets, clarification is needed. For a), we assume that MR should wake-up and receive SSB for calibration to achieve 0.1 ppm. For b), For the OFDM receiver, SSS monitored by WUR also is included in the LP-WUS synchronization signal |
| Xiaomi | We agree to consider both 0.1 ppm and 5 ppm as Fr, which can provide more choices for UE’s implementation. But comments for each option are not necessary. We suggest the following modifications: For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is: a) 0.1 ppm, if MR can assist to calibrate LP-WUR to correct the frequency error b) 5 ppm, if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal Note: Both cases in which MR can and cannot assist to calibrate LP-WUR to correct the frequency error are taken into account. |
| Nokia1 | In principle we are fine to consider the residual frequency error. On the exact numbers, specifically point b), we share the view with QCM that the value of Fr is heavily dependent on the (possible) LP-SS design as well as on the (possible) LP-WUS design (e.g. preamble), thus the exact value could be left for FFS. |
| Intel | What is the relation between this proposal and option 1/2/3/4 in the WA? Does it intended to say, with help of MR or LP-SS, the frequency error (the first value in each value pair of Option 1/2/3/4) can be replaced by 0.1 or 5 ppm ? Then, Option 1/2/3/4 is for the case if oscillator is free run? |
| Samsung | We think that it can be also related to power model of LR, therefore we suggest to discuss power model of LR including OFDM-based receiver first. |
| CATT | OK with the proposal. However, the time interval of 3 SSB for MR clock to calibrate the MR clock frequency stability at 0.1 ppm needs to be included in the model when MR is waken up from out of sync. |
| Apple | We are fine with the main bullet. But it is not clear why we use 5ppm for case b). We also think it should be left to FFS or companies to report. |
| FL1 | It seems the main bullet is OK for all which the detail value of Fr is controversial. FL recommends the followings |

- [H] Proposals 1A-2-v2:

For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is:

a) 0.1 ppm, if MR can assist to calibrate LP-WUR to correct the frequency error

b) 5 ppm, if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

- Companies to report Fr

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the update. |
| Ericsson1 | OK. |
| OPPO | OK with the update. |
| Samsung | We think that Fr can be assumed as 0 for ideal case. But, for cases that Fr is not 0, it may be depending on the synchronization signal or/and assumed architecture. Therefore, the reason why Fr has such an assumption should also be provided by companies. |
| Xiaomi | Fine with the update. |
| CATT | OK with the update proposal. |
| MTK | Ok. But the working assumption is not agreed yet. We cannot make new agreements based on the working assumption. |
| Vivo | OK |
| Nokia2 | OK. |
| Intel | OK |
| Huawei, HiSilicon | It seems companies mainly not align with the exact values, but in general the two cases seems OK for companies. Can we progress a little bit: For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is: a) x ppm, if MR can assist to calibrate LP-WUR to correct the frequency error b) y ppm, if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal - Companies to report Fr |
| ZTE, Sanechips | OK. |
| FL2 | @MTK, maybe we can take it as working assumption @Samsung, adding a sentence saying companies to report important assumptions important for achieving Fr @Huawei, adding two sub-bullet back as example for companies to consider as the important assumptions when reporting Fr. Hope it is acceptable. |

- [H] Proposals 1A-2-v3:

Working assumption

For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is:

- Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

| Company | Comments |
| --- | --- |
| CATT | Ok with the proposal. |
| InterDigital | Fine |
| Apple | OK |

- [H] Proposals 1A-3-v1:

Option 3/4 clocks is not applicable to be used when MR is in ’ultra-deep sleep state’ with [0.015] power units and LR is in off state

| Company | Comments |
| --- | --- |
| Futurewei | To address Qualcomm’s point, we can specify that this is true for low cost devices used for IoT use cases. |
| OPPO | OK with the proposal. |
| Huawei, HiSilicon | The proposal may need to be clarified first. Should it be “not applicable……when MR is in ’ultra-deep sleep state’ with [0.015] power units or LR is in off state”? |
| QC | Given that clock accuracy highly depends on cost of clock, we think option 3 and 4 clock should be able to be considered when MR is in ultra deep sleep. |
| ZTE, Sanechips | We are OK with this proposal. |
| Xiaomi | We are fine with the proposal. |
| Nokia1 | Firstly, agree with the need to clarify as Huawei commented. Also as noted by QCM, if the cost penalty is taken in the LR, there should not be a reason to omit this when MR is in ultra deep sleep. |
| Intel | We are OK with the FL proposal |
| InterDigital | We believe that option 3/4 clocks should be applicable with ultra-deep sleep state in general. |
| Samsung | For LR in on-state, can option 3/4 clocks be applicable regardless of ‘on’ power of LR? I think that this proposal can be handled with proposal 1C-2-v1 together. We suggest the following change to the proposal. Option 3/4 clocks is not applicable to be used when MR is in ’ultra-deep sleep state’ with [0.015] power units and LR is in off state |
| CATT | OK with the proposal |
| Apple | Many companies are assuming free-running ring oscillator. But our understanding is that crystal+PLL can potentially be an option, and the power consumption can be reduced significantly compared to the main radio if the phase noise requirement is relaxed. Therefore, we do not agree with the proposal at this time. |
| FL1 | @Huawei, Nokia, the original intension is ‘and’ but not ‘or’. If LR is ‘OFF’ but the MR is deep sleep/micro sleep/active, it is still possible to provide accurate clock from MR. Considering Qualcomm’s and Samsung’s comment, we can wait a bit by proposal 1C-2-v1. Therefore, I didn’t update the proposal. And companies can provide comments below. |

| Company | Comments |
| --- | --- |
| SONY | Sound reasonable. Also, it is the power consumption of LR that determines these values, if the power consumption of LR is below a certain level, then it means we cannot have clocks as accurate as oprtion3/4 |
| Ericsson1 | We do not agree with the proposal. Have similar view as Apple. No need to link Option 3 or 4 only to MR. Can be applicable independently by LR. Also, do not follow meaning/applicability of clock accuracy assumption when LR is in off state. |
| Spreadtrum2 | Fine |
| vivo | We think at least the following cases, clock option 3/4 is not possible Option 3/4 clocks is not applicable to be used when - MR is in ’ultra-deep sleep state’ with 0.015 power units and - LR is in off state with 0.001 power unit. |
| FL2 | It seems the discussion is also related to the power model discussion. FL suggest to temporarily stop this proposal and coming back later. |

In the last meeting, the WA for frequency error model is agreed, companies to check whether it is agreeable this meeting. Not the FFS part is discussed separately.

- [H] Proposals 1A-4-v1:

Confirm the WA for the followings

Working Assumption

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |
| --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |
| RTC max frequency error [ppm] | 20 |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than [TBD] power unit,

    - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

    - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

    - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

    - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

    - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

    - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

    - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

    - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

    - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

    - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

- FFS: Phase noise model

| Company | Comments |
| --- | --- |
| OPPO | Support. |
| Xiaomi | Fine with the proposal. |
| CATT | OK with the proposal. |
| Apple | OK. |
| ZTE, Sanechips | OK |

- 1A-5: Others

Huawei

  - Huawei propose to Adopt the phase noise model defined by IEEE 802.11ba for LP-WUS study.

  - Huawei propose the random 16-QAM symbols can be mapped on the REs of the PDSCH to model the neighboring subcarrier interference.

  - Huawei propose to consider one or two neighboring cells to transmit random 16QAM symbols on REs within the cell bandwidth in the link simulation.

MTK

  - In the assumption of delay spread corresponding to the TDL models, the value of 1000ns has the max delay span of 8.6us, three times larger than the CP length of 2.34us for 30kHz SCS.

  - Change the assumption of delay spread from optional 1000ns to optional: 100ns, considering the agreed values of 300ns and 1000ns result in larger delay spans than the CP length for 30kHz SCS.

Qualcomm propose to discuss LLS assumptions for FR2.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 28GHz |
| SCS | 120KHz |
| Channel Model | CDL-C, TDL-A |
| Delay spread | 30ns, 100ns |
| UE speed | 3Km/h |
| Antenna configuration: | gNB antenna: 2 UE antenna: 1 |
| LP-WUS bandwidth | TBD |
| LP-WUS payload | Same as FR1 |
| LP-WUS raw data rate | Same as FR1 |
| Receiver Model | [Same as FR1] |
| Performance metrics | Same as FR1 |

Moderator suggest companies to provide feedback with respect to the above proposals

| Company | Comments |
| --- | --- |
| Futurewei | We are OK to adopt proposals from Huawei and MTK. However, we feel that FR2 should be deprioritized for now given the limited discussion on that aspect. |
| OPPO | OK with proposals from Huawei and MTK. |
| Xiaomi | Considering the poor coverage of LP WUS signal, we think that discussion of LP WUS in FR2 should be postponed. |
| CATT | OK with proposals. |
| Apple | We are open to consider the proposals from Huawei and MTK. But would like to deprioritized the FR2 discussion. |
| FL2 | Suggest the following proposals for Huawei and MTK’s proposal |

- [M] Proposals 1A-5-v1:

  - As start point, adopt the phase noise model defined by IEEE 802.11ba for LP-WUS study

  - Change the assumption of delay spread from optional 1000ns to optional: 100ns, considering the agreed values of 300ns and 1000ns result in larger delay spans than the CP length for 30kHz SCS.

  - Regarding the adjacent subcarrier interference, on resources mapped with PDSCH, the random 16-QAM symbols can be mapped on the REs of the PDSCH to model the neighboring subcarrier interference.

  - Regarding other cell interference, it can be modelled by considering one or two neighboring cells to transmit random 16QAM symbols on REs within the cell bandwidth in the link simulation.

| Company | Comments |
| --- | --- |
| OPPO | OK with the proposal. |
| CATT | Fine with the proposal. |
| InterDigital | Fine in principle, but better to clarify that this is for LLS evaluation. |
| ZTE, Sanechips | Generally fine. |

- 1B: Coverage evaluation assumptions

- [Close]1B-1: Noise Figure

Vivo propose to assume 12dB noise figure, because 8.7dB and 11 dB NF can be achieved according to literatures.

Samsung: The presence of LNA should be reflected to select the NF value for link budget evaluation.

Ericsson:

- WUS1: sequence-based OOK WUS (1 slot WUS), WUR noise figure 6 dB worse than main receiver

- WUS2: SSS-based signal detection based WUR capable of processing I/Q samples in time-domain (4 OFDM symbols WUS), WUR noise figure 3 dB worse than main receiver

Moderator suggest to handle the value of NF corresponds to each receiver in AI9.11.2

| Company | Comments |
| --- | --- |
| Futurewei | We agree with moderator’s proposal. |
| DOCOMO | We support this proposal. |
| Spreadtrum | It can be left to RAN4 discussion and it is architecture specific. |
| Huawei, HiSilicon | It is fine for us to map the noise figure and power consumption assumption for each receiver type in AI 9.11.2. |
| QC | Given that NF depends on the choice receiver types, implementation, and power consumption, it would be enough to determine two/three representative NF values to cover different receiver type/implementations. |
| ZTE, Sanechips | We are Okay. |
| Xiaomi | We are fine with the proposal. |
| Nokia1 | Fine to concentrate the RAN1 discussion to 9.11.2, with the note that RAN1 also asked RAN4 view on the NF. |
| Intel | Agree |
| Samsung | We are OK. |
| CATT | We agree with moderator’s suggestion. |
| Apple | OK |
| LGE | Fine with the proposal. |
| FL1 | Close this discussion. |

- 1B-2: Others

  - Nokia propose to consider whether the LR has it’s own separate antenna or whether (one of) the main receiver antenna is shared. If antenna is shared, additional loss due to possible switch could be considered, (while this could be assumed to be relatively low i.e. ~0.2dB). If separate antenna is used, it would need to be considered whether additional antennal efficiency should be considered for LR to reflect the possible constraints set by the ID of the device to the antenna design/dimensions.

  - Nokia There maybe different type of deployments, which assume different type of receiver baseline for cell coverage. Redcap UE and Normal UE have different references in coverage evaluation.

- 1C: Power model

- 1C-1: power model for MR ramp-up transition time and transition energy

| ZTE | Proposal 3: The relative power of ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation. Proposal 4: The ramp up and down transition energy and ramp-up time of ultra-deep sleep is 15000 units*ms and 400ms for IoT/Wearable cases and 40000 units*ms and 800ms for eMBB cases for power consumption evaluation. |
| --- | --- |
| MTK | Confirm Alt 2: (40000, 800ms) being the transition energy and transition time for FR1 MR ultra-deep sleep state. |
| Qualcomm | Proposal 3: For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) confirm following assumptions. - Alt 1: (15000, 400ms) - Alt 2: (40000, 800ms) |
| InterDigital | Proposal 2: Confirm the relative power value 0.015 for Ultra-deep sleep of MR. Proposal 3: For ramp-up/down transition energy and ramp-up time of MR, support Alt 1 (15000, 400ms) and deprioritize Alt 2 ([40000], 800ms)). |
| LG | Proposal 2: Confirm Alt 2 of (Ramp-up and down transition energy, ramp-up time) for evaluation at least for FR1 MR ultra-deep sleep state. - It can be used as an option only applicable for eMBB cases. |

- [H] Proposal 1C-1-v1:

Confirm Alt 2 in the following agreement

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms)

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

| Company | Comments |
| --- | --- |
| OPPO | OK with FL’s proposal. |
| Huawei, HiSilicon | We are fine to confirm the working assumption of Alt.2, but no need to further map the alternatives to specific device types. |
| QC | agree |
| ZTE, Sanechips | Fine with the proposal. Alt 1 for IoT/Wearable cases and Alt 2 for eMBB cases |
| Xiaomi | Fine with the proposal. |
| Nokia1 | We are OK to confirm |
| Intel | Agree |
| Samsung | We would like to clarify cases that Alt 2 is applied. If cases are not critical, we prefer to use Alt1 as baseline. |
| Apple | OK |
| LGE | Fine with the proposal. |
| FL1 | Based on the comments received, FL asked the followings before making any updates, Q1: can company go with the Proposal 1C-1-v1 Q2: can you accept taken Alt 1 as baseline considering if cases are not critical. Q3: are you OK to ‘Alt 1 for IoT/Wearable cases and Alt 2 for eMBB cases’ |

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with Alt 1 being baseline. |
| Ericsson1 | OK to confirm Alt2. |
| Spreadtrum2 | Alt 2 means MR cannot enter ultra-deep-sleep to get positive PSG for normal DRX case. It will limit the application of LR. It is questionable whether the transition energy/time of Alt 2 is actually feasible… In general view, here, ramp-up/down energy is the energy difference/gap between ultra-deep-sleep and micro sleep. Comparing wearables and eMBBs, the power for micro sleep may not be so different. Hence, we support Alt 1 as baseline. |
| Samsung | We do not support to limit to use alternative according to use case. We prefer to take Alt 1 as baseline. |
| Vivo | We are Ok to Q1-Q3. |
| Nokia2 | Q1: We are OK to confirm the proposal Q2: Alt1 can be the baseline Q3: If Alt1 is the baseline, not sure if categorizing is needed, but no strong view |
| Intel | OK to confirm Alt 1 as baseline |
| FL2 | According to the comments received, it seems majority are OK to take Alt 1 as baseline. FL suggest to take Alt 1 as baseline and confirm WA for Alt 2 |

- [H] Proposal 1C-1-v2:

Confirm Alt 2 in the following agreement and update as follows

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms) as baseline

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

| Company | Comments |
| --- | --- |
| OPPO | OK with the proposal. |
| Xiaomi | Fine with the proposal. |
| Spreadtrum3 | Fine |
| InterDigital | We are okay if Alt 2 is optional. |
| Apple | We prefer not to have Alt 1 as the baseline, but otherwise fine. Many companies seem to think Alt 1 is for IoT and Alt 2 is for wearable/eMBB. We do not see why IoT should be prioritized over wearable/eMBB. |
| ZTE, Sanechips | Fine. Since IoT use case has less requirement on mobility, it would be challenging for eMBB/XR to UE have significant power saving, and keeping latency low, UPT loss low. |

- 1C-2: power model for OFDM-based LP-WUR

| ZTE | Proposal 6: For OFDM based receiver, WUR ‘on’ relative power should be no less than 20 and WUR ‘off’ relative power should be 0.01. Proposal 7: The relative power of WUR on is 0.01, 0.5, 1, 20 and 40 for LP-WUS power consumption evaluation. |
| --- | --- |
| Samsung | Proposal 5: To reflect the higher power consumption of receiver architectures for various waveform e.g., OFDMA-based signal, FSK waveform, the following approaches should be considered for LR power model. - Company can use higher on/off power for LR, and the details for assumed receiver architecture should be provided. - Candidates of LR power model for higher power-consumed LR can be added. e.g., 10, 20, 40 for on-state of LR. |
| MTK | The current relative power values for LPWUR are up to 4 and they cannot reflect the need to support OFDMA-based LPWUR. Add relative power values 20 and 40 for the power state ON to support OFDMA-based LPWUR and set ramp-up time to zero for OOK/FSK LPWUR with relative power values smaller than 20. |
| Qualcomm | For study purpose and given that it is well-understood that OFDM-based receivers will consume higher power than OOK/FSK-based receivers, we may need to consider higher range of monitoring power values for LP-WUR. Proposal 5: For study, add following additional power numbers (0, 10,20,40) for LP-WUR power consumption in On state. Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0/0.01/0.05/0.1/0.5/1/2/4/10/20/40 FFS: If other values are needed. - Note: 0 is for Genie LP-WUR which can be used to show the lowest power consumption (or highest PSG). - Note: Ramp-up time for LP-WUR must be much lower than 15 ms since deep sleep (DS) ramp-up + ramp-down time in 38.840 is 20 ms - TLR, ramp-up is FFS, and company to report TLR, ramp-up - FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up - FFS: whether further categorization/sub-categorization is needed and how. |
| vivo | Proposal 6: Adopt the following power model for LP-WUR for OFDMA-based signals/channels detection: - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: 0.01unit - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: 20 or 40units |

The following power model for OFDM-based LP-WUR is used for evaluation:

- Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: [0.01 unit]

- Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: 10, 20 or 40units

While in 9.13.2 AI , there is some proposals also related to OFDM receiver, e.g.,

- Two company propose 1-5 unit,

- One company propose 0.15-0.2 unit(only for LO with 200ppm or 50ppm, i.e., clock option 3 and 4)

- [H] Proposal 1C-2-v1:

The following TPs is proposed for TR38.869v0.1.0 section 6.3.2

----------------------------TP start-------------------------------------------

6.3.2	Power model for LP-WUR (LR)

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) |  | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- | --- |
| Off[1] | 0.001 | [0.01] | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On[2] | 0.01/0.05/0.1/0.5/1/2/4 FFS: If other values are needed | OFDM-cat 1: FFS 5, 10, 20, 40 FFS: OFDM-cat 2: 0.2 |  |  |

- FFS: whether further categorization/sub-categorization is needed and how.

- FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

- FFS: LP-WUR power consumption values for FR2.

- Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

- Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

- Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

- Note4: for OFDM-cat1, clock error option 3 or 4 is assumed, for OFDM-cat2, clock error option 1 or 2 is assumed

[1] Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring

[2] Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring

----------------------------TP End-------------------------------------------

| Company | Comments |
| --- | --- |
| Futurewei | We are in general OK with the proposal. |
| OPPO | OK with the proposal. |
| Spreadtrum | For OFDM-based, it can be discussed in AI 9.11.2, since some companies have provided values. Anyway, we are fine for using two categories for OFDM-based. The specific values can be added with bracket. |
| Huawei, HiSilicon | We are not fine with the current proposal. - We agreed the list of “0.01/0.05/0.1/0.5/1/2/4” for LP-WUR receiver, and we have never agreed that “0.01/0.05/0.1/0.5/1/2/4” is only for envelope-detection based receivers, and in fact the values are general across the sequence-based correlation receivers also. The higher values proposed for proposed OFDM cat 1 can be added to the end of the existing list, and the Rx architectures agenda item is the better place to perform the mapping of power consumption to architecture. - We should also clarify that whether the LP-WUR “On” state includes the power consumption of synchronization. Based on the discussion in previous meetings, it seems current list of “0.01/0.05/0.1/0.5/1/2/4” only considers the detection power but not consider the synchronization power consumption. On the other hand, some companies propose very high power consumption of OFDM based receiver for the purpose of only time/frequency synchronization. Therefore, there exists mis-alignment among companies. In our view, it is better to separately discuss the LP-WUS signal detection power and the synchronization power consumption, considering the power consumption of these two operations on UE shall be very different and the synchronization power consumption can be actually averaged over the long synchronization signal period with a small impact on the final power consumption. In summary, we propose separately discuss the power consumption of LP-WUS signal detection and time/frequency synchronization: - Clarify that the agreed that the agreed value range of “[0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4]” only considers the power consumption due to LP-WUS signal detection. This can be reflected actually in the text in the RAN1#110bis agreement: “Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring”; - Further discuss the power consumption due to time/frequency synchronization, this power consumption value could be much higher considering UE may need to under different hypothesis to detect the correct timing and frequency (as analyzed in MTK’s contribution.). However, it should be noted that this power consumption shall be averaged over the periodicity the UE needs to do the synchronization. |
| QC | Although we know that typical OFDM receiver implementation will have higher power consumption than OOK receiver, making hard boundary between the two types of receiver does not make strong sense. We prefer to just add additional power numbers in the list instead of categorizing them as separate column. |
| ZTE, Sanechips | 20, 30, 40 are possible values for OFDM receiver. 5 and 10 are more appropriate for FSK or other modulation schemes. For OFDM-cat 2, it is strange why the relative power could be lower than OOK. The difference between Zero-IF architecture for OFDMA based signal and Zero-IF architecture for OOK based signal should be clarified. |
| Xiaomi | Fine with the proposal. |
| Nokia1 | OK in principle. Noting also that in our paper for the architecture discussion we touched upon the possible saving that could be achieved. Based on that and taking the relative power consumption of 50 (for SSB/CSI processing), we considered 12 in our evaluations. Thus at least relative power of 10 could be considered. |
| Intel | We are Ok with adding relative power values for OFDM receiver. |
| InterDigital | Support |
| Samsung | We suggest to listing all of relative power candidates together regardless of the type of receiver in this stage. We think whether the relative power value for OFDM-based receiver can be under 4 will be discussed receiver architecture agenda. And, for MC-OOK/FSK receiver, categorization of power model for each architecture can be provided by receiver architecture agenda, as well. Therefore, only larger relative power value which is not agreed yet can be discussed in this stage and we suggest to remove all FFS value e.g., 5, 0.2 in this proposal. On the other hands, Note 4 is also revised according to relative on-state power of LR. |
| CATT | Generally OK with this proposal. |
| Apple | We also prefer to just add additional numbers in the list. The power consumption of each receiver architecture can be discussed under AI 9.11.2. |
| LGE | Fine with the proposal. |
| FL1 | @Spreadtrum, adding [] for the new values @Huawei, removing ‘OFDM-cat’, thus companies can picked up values based on its own architecture. Also, adding a note saying ‘Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.’ @Qualcomm, in principle it’s true, but it is not easy to know the added value. Current way is easier to proceed since companies have reported the value of OFDM receiver. @ZTE, adding 30 for cat1a. @Samsung, Apple, removing the receiver type. Since different OFF values are used, Cat 0 and Cat 1. In principle it is OK to be also discussed in AI9.11.2. Since quite a lot of contributions submitted, and we discussed ED detector power model in 9.11.1, we can also make comments here. The following modified proposal does not say receiver architecture any more. FL make the following update of the proposal. Please see [H] Proposal 1C-2-v2 |

- [H] Proposal 1C-2-v2:

----------------------------TP start-------------------------------------------

6.3.2	Power model for LP-WUR (LR)

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) |  | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- | --- |
| Off[1] | 0.001 | [0.01] | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On[2] | Cat0: 0.01/0.05/0.1/0.5/1/2/4 FFS: If other values are needed | Cat 1a: - [10, 20, 30, 40] - FFS: 5 FFS: Cat 1b: - 0.2 |  |  |

- FFS: whether further categorization/sub-categorization is needed and how.

- FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

- FFS: LP-WUR power consumption values for FR2.

- Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

- Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

- Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

- Note4:

  - For cat1a, clock error option 3 or 4 is assumed,

  - FFS for cat1b, clock error option 1 or 2 is assumed

- Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

[1] Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring

[2] Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring

----------------------------TP End-------------------------------------------

| Company | Comments |
| --- | --- |
| Futurewei | We are not sure why we would have a separate Cat 1b when its relative power (0.2) is very close to values already part of Cat 0 and is restricted to the same clock options 1 and 2. We would suggest updating Cat 0 with the value of 0.2 and keep only Cat 0 and Cat 1. |
| Ericsson1 | We prefer the approach suggested by Qualcomm. OK to add additional OFF power value of 0.01 for purpose of evaluation. Some proponents suggest value 40. Given below information from 38.875 (Redcap TR), even MR power consumption for 1Rx and 20MHz seems to 35. We wonder why higher value should be assumed for LP-WUR that will have lower BW and complexity compared to MR. Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001/0.01; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.01/0.05/0.1/0.5/1/2/4/10/20 FFS: If other values are needed. - Note: 0 is for Genie LP-WUR which can be used to show the lowest power consumption (or highest PSG). - Note: Ramp-up time for LP-WUR must be much lower than 15 ms since deep sleep (DS) ramp-up + ramp-down time in 38.840 is 20 ms - TLR, ramp-up is FFS, and company to report TLR, ramp-up - FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up - FFS: whether further categorization/sub-categorization is needed and how. Overall, we would be OK with below updates. |
| Spreadtrum2 | Fine. Also agree with E/// that value of 30 and 40 may not be feasible. |
| OPPO | OK for purpose of evaluation. |
| Samsung | We prefer to remove Cat1b or include to Cat0. According to proposal, Cat 0 can consume higher on-state power compared to Cat 1b (e.g., 0.5, 1, 2, 4), but lower off-state power than that of Cat 1b. We think that it is not reasonable. Considering the proposal 1A-3-v1 and 1C-2-v2 proposals, our understanding is that clock 3/4 can be assumed only when LR with Cat 1 is on-state or MR is not ultra-deep sleep state. Otherwise, we wonder that clock 3/4 can be assumed for Cat0 with higher on-state power, as well. Current proposal seems that clock 3/4 is only applicable to Cat 1a. |
| CATT | We are OK with Ericsson’s version for simplicity. |  |
| MTK | Okay with Ericsson’s proposal. In our previous t-doc, we suggested 40 because considering a longer monitoring occasion 1ms than monitoring SSB. The power value can be smaller if the monitoring occasion can be shorter. For FL’s proposal, we wonder why Cat 0/1a/1b is needed. If the only intention is to associate with clock error, then clock error is still the working assumption. We need to agree it before using it. |  |
| vivo | - For progress, we can accept merging cat 1 into cat 0. For Off value of WUR, we would like to add another value to represent the case that oscillator is maintained for time/frequency tracking, y (e.g., y >= 0.1), because for Off value of 0.001 and 0.01, they are too low for keeping oscillator, but keeping a RTC. Further, among these Off values, companies can report which one is used. - For ON value, our assumptions mainly come from investigation from literature, if companies have concerns on 40, then 30 can be used instead, which is smaller than the relative power consumption for SSB/CSI processing by 1RX REDCAP. <Update - vivo> ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [0.01]/ [y, e.g., y>=0.1]; [0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; Cat0: 0.01/0.05/0.1/0.5/1/2/4/10/20/30 FFS: If other values are needed; Cat 1a: - [10, 20, 30, 40] - FFS: 5 FFS: Cat 1b: - 0.2. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: - For cat1a, clock error option 3 or 4 is assumed, - FFS for cat1b, clock error option 1 or 2 is assumed - For WUR Off value 0.001 and [0.01], oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value y [y, e.g., y>=0.1], option 1,2,3,4 can be assumed. - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. |
| Nokia2 | We also think the updated proposal from Ericsson would be a good way forward. I agree also with the point on the value 40 (missed the 0.7 scaling in my previous comment). |
| Huawei, HiSilicon | - As we commented in the first round, we should clarify that the LP-WUR “On” state is for “the power consumption of LP-WUS monitoring”. And add a note to say that the power that is assumed for synchronization shall be discussed/reported separately; - 0.02 should be also added in the list, considering it presents the power of sequence based correlation; - “Cat0” should be removed. We can discuss the power consumption of each receiver type in AI 9.11.2. no need to introduce category now. - For the WUR off value larger than 0.1, we didn’t see any company to propose it. And for WUR off of 0.001 and 0.01 (especially 0.01), we don’t get the point why option 1 and option 2 oscillator is not possible. |
| ZTE, Sanechips | We are generally fine with some clarification. After the bandwidth and the Rx is reduced to 20MHz and 1Rx, actually it is more harder to have power reduction when the bandwidth or other components is further reduced without significant structure change. Moreover, actually if WUR is used for measurement, the relative power should be higher, therefore, relative power 30 at least would be reasonable. Therefore, for Cat 1a, we would suggest [20, 30]. As for 10 and other values between 4 and 20, we think there should be a gap between OOK receiver and OFDM receiver, which could be applicable for other receivers with a medium complexity between OOK receiver and OFDM receiver, e.g., PSK. |  |
| FL2 | @ Ericsson, now the table is merged as follows. Since the intension of new added 10/20/30 are for OFDM receiver, I add a sub-bullet ‘10/20/30 for LP-WUR ON power state are used for OFDM receiver’ @vivo, per vivo’s suggestion (adding y for WUROFF), I understand the original intension is to describe the WUR power value when the LP-WUR oscillator clock is maintained in ‘off’ period for a duty-cycled monitoring. So far this is missing. Suggest to consider it. However, do we really need so many values other than 0.001? Suggest to simplify to Y as start point for discussion. @Huawei, - My intension for this table for ‘WUR ON’ includes WUS monitoring and sync/measurement, and whether they are using different value can be up to companies report. I have already updated the proposal to add a note saying ‘Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.’ - I think the current added value 10/20/30 is the values which are not in the range previously. However, 0.02 is already in the range. - Removed“Cat0” - The Y value is related to this. Thus can discuss the value Y in details. Please see[H] Proposal 1C-2-v3 |

- [H] Proposal 1C-2-v3:

----------------------------TP start-------------------------------------------

6.3.2	Power model for LP-WUR (LR)

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off[1] | 0.001/ [Y, e.g., Y>=0.1 or 0.01] | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On[2] | 0.01/0.05/0.1/0.5/1/2/4/10/20/30 - FFS: If other values are needed |  |  |

- FFS: whether further categorization/sub-categorization is needed and how.

- FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

  - 10/20/30 for LP-WUR ON power state are used for OFDM receiver

- FFS: LP-WUR power consumption values for FR2.

- Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

- Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

- Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

- Note4:

  - For cat1a, clock error option 3 or 4 is assumed,

  - FFS for cat1b, clock error option 1 or 2 is assumed

  - For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed.

- Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

----------------------------TP End-------------------------------------------

| Company | Comments |
| --- | --- |
| Spreadtrum3 | Fine |
| CATT | OK with the proposal. |
| OPPO | OK. |
| Apple | OK in principle. On “10/20/30 for LP-WUR ON power state are used for OFDM receiver”, I assume the intention is to say these values are not to be used for other receiver types. But it may be misunderstood as OFDM receiver can only take these values. Suggestion: “10/20/30 for LP-WUR ON power state are not applicable to the receiver architectures for MC-OOK and MC-FSK.” For the highlighted note on oscillator, is the intention to say that “For WUR Off value 0.001, oscillator is assumed to be not running and only RTC is maintained”? |
| ZTE, Sanechips | We are fine and think 10/20/30 is only applied for OFDM based receiver. It is not true that the OFDM receiver can use other relative power, e.g., 1. If it is possible, please clarify how to achieve this. |

- 1C-3: LR Ramp-up time

| CATT | Ramp up time from LP-WUS off to ON is assumed as 1ms with the relative power of LP-WUS on to be 0.01 – 0.1. Proposal 8: The suggested power model for LP-WUR is as follows: Power State; Characteristics; Relative Power; Ramp-up time. Periodic low power WUS “ON” state; Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO.; [0.01 – 0.1]; ---. Periodic low power WUS “OFF” state; Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. Otherwise, the wakeup receiver is shut down.; [0.001]; [1ms]. Continuous low-power WUS monitoring; Front end wakeup receiver with free-running clock in the active device or passive device monitoring of wakeup signals continuously.; [0.001 – 0.01]; [0ms]. |
| --- | --- |
| ZTE | 10ms ramp-up time of WUR could be used as the starting point. |
| Samsung | Proposal 3: When the relative power value for the on-state of LP-WUR is chosen for the evaluation, the characteristics of the assumed LR architecture should be reflected. - E.g., the types of receiver architecture, the presence of LNA/AMP, the type of oscillator, the type of BPF/LPF filter and etc. - The details of LR assumed for the evaluation are up to each company. Proposal 4: Ramp-up time and transition energy from ‘off’ to ‘on’ states should be different according to the power level of ‘on’ state for LR. - E.g., on state for 1/2/4 relative power unit, ramp-up time should not be neglected. |
| Ericsson | 10ms is assumed for ramp up time of LP-WUR on-off |
| vivo | 10ms |
| Nokia | Table 2. WUR power consumption assumptions LP-WUR; Relative power; Transition time and energy (if applicable). LP-WUS monitoring, always-on receiver; [0.1]*. LP-WUS monitoring, duty cycled receiver; Monitoring: [4.0] or [12.0] Off: [0.001]; { 10ms, TLR, ramp-up *(PON - POFF)/2 }. []* : Values are preliminary and to be considered further based on the LP-WUR architecture discussion.. |

- [H] Proposal 1C-3-v1:

When the relative power of LP-WUR “on” is no less than 1unit, the ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as 10ms for evaluation.

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the proposal |
| OPPO | OK with the proposal. |
| Huawei, HiSilicon | For the LP-WUR power consumption, currently the agreed power is 2 and 4, and some companies are proposing larger values, e.g. 20 and 40. It is not clear regarding the assumption of power unit for LP-WUR when we propose the corresponding ramp-up time is 10ms. Therefore, we prefer to firstly discuss Proposal 1C-2-v1 clearly, then discuss the ramp-up latency for the considered values. |
| QC | It is not clear why the boundary is “1 unit”. |
| ZTE, Sanechips | From relative power 0.01 to 40, we do not think the 10ms is enough, since for RedCap UE from deep sleep 0.8 to PDCCH only for cross-slot scheduling with relative power 40, the transition time is 20ms. |
| Xiaomi | Fine with the proposal. |
| Nokia1 | OK. On relation to ZTE comment, in our view the transition time is more defined by the change in processing, and not solely determined by the (relative) power difference. |
| Samsung | For completeness of the proposal, ramp-up time for relative on-state power less than 1 unit should also be provided. e.g., 0ms |
| CATT | We are not OK with this proposal. We think the LP-WUR with less than 1 unit relative power should be prioritized. And, the characteristics of the LP-WUR with no less than 1 unit relative power should be clarified before the discussion of the corresponding ramp-up time. |
| Apple | The number seems to be somewhat arbitrary, and it is not clear if assuming the same ramp-up time for all values is reasonable. We think there should be some correlation between the ramp-up time and the power consumption of LP-WUR ON. E.g. we assume 5ms for relative power 1 and 10ms for relative power 4. But if majority companies prefer this way for simplicity, we could be fine with it. |
| LGE | Fine with the proposal. |
| FL1 | As commented by Nokia, if the transition time is more defined by the change in processing, can we make the following proposal for simplicity. |

- [H] Proposal 1C-3-v2:

When the relative power of LP-WUR “on” is no less than 1unit, The ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as 10ms for evaluation. Other values are not precluded for evaluation.

| Company | Comments |
| --- | --- |
| Futurewei | We are Ok with the update. |
| Spreadtrum2 | Fine |
| OPPO | OK with the update. |
| Samsung | We are fine to proposal. |
| xiaomi | Fine with the update. |
| ZTE, Sanechips | From our understanding, the ramp-up time may consist of hardware tune on e.g., boot, memory load and etc, not related to the processing, as shown in the following Note: Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.], Different receiver with different complexity actually has different ramp-up time. It is not fair to set all the receiver has the same ramp-up time. At least, for Cat 1a and Cat0, two set of values should be defined. For example, Cat 0: [6ms, 8ms, 10ms] Cat 1a:[10ms, 12ms, 14ms] Other values are not precluded. Actually, if the WUR goes to deeper sleep as relative power 0.01, the power consumption and ramp time would be higher. Therefore, we doubt the ramp time 15ms from 0.01 to 20 or 30 can not be realized. For comparison, the MR from deep sleep (power:1) to MR-on (the power may be less than 35), the transition time is 20ms. And now, the simplified OFDM receiver from deeper sleep (power:0.01) to WUR-on (the power may be 20 or 30) is less than 15ms. We are not sure it is possible. Hope more evidence could be clarified. |
| CATT | OK |
| Vivo | We are OK. |
| Nokia2 | OK |
| Intel | OK |
| Huawei, HiSilicon | We think the ramp up time relates with the assumption of the state of LP WUR ON. We are not fine to just simply say it is 10ms w/o any details. |
| FL3 | @Huawei and other companies, can we go with other way such as different ramp-up time values for different power. Such as the values proposed by Apple, 5ms for relative power 1 and 10ms for relative power 4. Proposed update as follows, - When the relative power of LP-WUR ON is no more than 1unit, - The ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as 5 ms for evaluation. - When the relative power of LP-WUR ON is more than 1unit, - The ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as 10 ms for evaluation. - Other values are not precluded for evaluation. |
| Spreadtrum3 | Fine |
| CATT | When the relative power of LP-WUR ON is no more than 1unit, we think the ramp-up time from LP-WUR ‘off’ to ‘on’ can be 1ms with considering the RF receiver. Thus, the proposal can be updated as follow: - When the relative power of LP-WUR ON is no more than 1unit, - The ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as no larger than 5 ms for evaluation. - When the relative power of LP-WUR ON is more than 1unit, - The ramp-up time from LP-WUR ‘off’ to ‘on’ is assumed as 10 ms for evaluation. - Other values are not precluded for evaluation. |
| Apple | We are fine with the updated proposal. |
| ZTE, Sanechips | NO, a set of values should be assumed for each switch. It does not make sense to have the same ramp time for relative WUR-on power 4 and 30, which is applied for different architectures. For no more than 1 unit case, the ramp up time could be referred to {3, 5, 7} For more than 1 unit case, the ramp up time could be referred to {10, 12, 14} Moreover, we should assume that the relative power for WUR-on should satisfy: OFDM-based receiver >FSK based receiver>OOK based receiver |

- 1C-4: measurement assumptions details

| vivo | Proposal 7: For RRM measurement assumptions in RRC idle/inactive mode, the following options can be considered. - Option 1: RRM measurement is only performed by MR. - Option 2: LP-WUR performs RRM measurement based on periodic lower power signal e.g., LP-SS. MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. - Option 3: RRM measurement is only performed by LP-WUR. Vivo suggest 1.28s in contributions submitted in AT9.11.3 |
| --- | --- |
| Sony | Proposal 4 – Consider low-power mechanism to support mobility and cell re-selection mechanism for UEs with LP-WUR. |
| Huawei | The periodicity of LP-SS can be assumed to be 400 ms, which can be used for the evaluation of resource overhead and network energy consumption. |
| Qualcomm | Figure 21 CDF of Delta RSRP = Genie RSRP – LP-SS based RSRP Observation 15: ±2 dB delta RSRP relative to genie RSRP may be achievable 90% of the time using OOK based LP-SS at SNR=-3 dB and realistic clock model. |
| Nokia | Now as the LP-WUS maybe sent rather infrequently (depending on the paging rate and number of UEs addressed by the LP-WUS), maintaining a frequency syncronisation during these periods would still be necessary. Hence, as discussed in past meetings considering some form of periodic LP signal to facilitate maintaining the frequency synchronisation would be beneficial. However, based on the performance evaluations [5] and assumed assumed drift and maximum error models agreed in last meeting (below), the periodicity of the LP syncronisation signal can be rather long, e.g. in order of 10s. Proposal 11: Consider LP synchronisation signal for (at least) for frequency tracking. |

- [H] Proposal 1C-4-v1:

The period of low power synchronization signal for evaluation can be {400ms, 1.28s, 10s(at least for frequency tracking) }

- Note: the purpose of the low power synchronization signal can be for LR synchronization (i.e., time and/or frequency tracking) or measurement.

| Company | Comments |
| --- | --- |
| OPPO | Generally OK with the proposal. |
| Spreadtrum | Periodic synchronization can be relaxed for LP-WUS. Periodic measurement can be in the order of I-DRX cycle like that of main radio. Therefore, we think it should be no smaller than 1.28s. |
| Huawei, HiSilicon | We are in general fine with the proposal with the following comments: - The periodicity relates to the design of LP-SS and also the required timing error/frequency error budget for LP-WUS signal detection. Based on our study, 400ms is needed for the synchronization. Larger value may be possible if the LP-SS can provide better synchronization performance and the LP-WUS signal waveform is designed more robust w.r.t time/frequency error. Therefore, we prefer to keep 400ms as a starting point and say larger value can be considered if justified; - Regarding the note, it should be “Note: the purpose of the low power synchronization signal can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement” |
| QC | Agree on the proposal. Other values should not be precluded. |
| ZTE, Sanechips | Serving cell measurement period is based on DRX cycle, i.e., 0.32s,0.64s,1.28s, 2.56s. To offload the measurement to WUR, similar measurement period can be assumed for LP-SS. Therefore, we would suggest the following values for LP-SS evaluation: {160ms, 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} Regarding 160ms, we think if LP-SS can be used to replace SSB, the performance may be worse than SSB. Therefore, more frequent LP-SS could be considered to guarantee the measurement performance. |
| Xiaomi | Generally fine. But other values for periodicity should not be precluded and can be reported by companies. |
| Nokia1 | (Based on agenda) the proposal seems to be for the purpose of power saving evaluations. We think that it would be good to clarify the purpose e.g. (…for power saving evaluation..). As noted by others, the sync performance etc. the performance together with the assumed clock drift would determine the needed periodicity, thus these could be preliminary values for power saving evaluation. A minor note that our note “(at least for frequency tracking)” related more to the purpose of the LP-SS rather than the period, thus is not needed as it is covered in sub-bullet. |
| Intel | We are fine to take the proposed values as start point. other values should not be excluded which are subjected to further study. For the note, it may be revised as ‘for LR synchronization (i.e., time and/or frequency tracking) and/or measurement’ since a LP-SS may be used for both sync and RRM too. Then, what will be the metric in the study of proper periodicity? The example may be timing/frequency error, RRM accuracy, power consumption, and etc. |
| Samsung | We think that we do not need to restrict the period of low power synchronization signal for evaluation. It can be reported by company. |
| Apple | We wonder why 400 ms is chosen. It does not fit into NR numerology very well. The gap between 1.28 s and 10 s seems too large. We prefer the values proposed by ZTE. |
| LGE | We share the view with ZTE, Sanechips. It would be better to reflect current periodicity of DRX. |
| FL1 | @Spreadtrum, considering the i_DRX cycle 320ms, it should be OK to also include that value as well. @ZTE, considering some companies are concerning the value less than i-DRX, perhaps it’s better to focused on the values for i-DRX as start point, but other values are not precluded. @Nokia, adding for power evaluation to address your comment. @Intel, adding ‘and’ @Samsung, the purpose of the proposal to reduce the variance of the assumptions. And Other values are not precluded Make update as follows |

- [H] Proposal 1C-4-v2:

The period of low power synchronization signal for power evaluation can be {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)}

- Other values are not precluded

- Note: the purpose of the low power synchronization signal can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

| Company | Comments |
| --- | --- |
| SONY | OK with these values for evaluation. The periodicity should be decided based on the receiver architecture and possible frequency/clock accuracy and not independent from them. |
| Futurewei | We are OK with updates. |
| Ericsson1 | The period should include 20ms to reflect use of SSB. Our preference is removing second sub-bullet (Note), but if it is to be kept, suggest to leave the purpose description up to the proponent. <Updates> The period of low power synchronization signal for power evaluation can be - 20ms for evaluations assuming SSB - For evaluations assuming LP-SS - { 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)} - Other values are not precluded - Note: companies to report the purpose of the low power synchronization signal along with evaluations , e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. |
| Spreadtrum2 | OK for evaluation purpose |
| OPPO | OK with these values for evaluation. |
| Samsung | We think that the period of synchronization signal can affect the results of LLS when time/frequency errors are modeled considering the periodicity of synchronization signal. Therefore, these values can be used for detection performance evaluation as well as power evaluation. |
| xiaomi | Fine with the update. |
| CATT | OK for evaluation purpose. |
| MTK | Agree with Ericsson. Suggest including SSB and the period: ms20, ms40, ms80, ms160. |
| Vivo | In principle fine with the change by Ericsson. It should be clarified other SSb period should also be allowed. <Updates - vivo> The period of low power synchronization signal for power evaluation can be - {20ms, 40ms, 80ms, 160ms} for evaluations assuming SSB - For evaluations assuming LP-SS - { 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)} - Other values are not precluded - Note: companies to report the purpose of the low power synchronization signal along with evaluations , e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. |
| Nokia2 | We are OK with the proposal, and also with the updates proposed by Ericsson/MTK |
| Intel | OK |
| Huawei, HiSilicon | We are fine with it. Regarding the periodicity of SSB, we prefer not to include in this proposal, considering this focuses on the assumed periodicity of LP-SS signal. For the use of SSB, it is up to companies' report considering SSB periodicity exists in legacy NR system and no need to agree it in this proposal for the Low power synchronization signal. |
| Futurewei | Just to clarify, is the purpose of including the SSB as a synchronization signal to capture (a) a LP-WUR that is capable of receiving SSB, (b) usage of MR with relaxed periodicity to assist in LP-WUR synchronization? Also, if SSB is being added as a synchronization signal, I think the main text needs to change to generalize to any synchronization signal instead of a low power synchronization signal. We suggest the following edits: [H] Proposal 1C-4-v2: For power evaluation, Tthe period of low power the synchronization signal used for LP-WUR synchronization for power evaluation can be follow one of the following options - Option 1: Considering SSB as the synchronization signal - {20ms, 40ms, 80ms, 160ms, 320ms} - FFS: whether LP-WUR is directly used to receive SSB - Option 1: Considering a low power synchronization signal - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)} - Other values are not precluded - Note: the purpose of the low power synchronization signal can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. |
| ZTE, Sanechips | We are fine with the proposal. We think the LP-SS includes the sync signal received by WUR, also including OFDM based WUR. Therefore, there is no need to remove ‘low power’. |
| FL2 | @Ericsson, make changes according to your comments. @vivo, MTK, adding 20ms, 40ms, 80ms, 160ms, @FutureWei, adding 320ms as suggested. Regarding whether (a) or (b) is used can be separately discussed and reported. The last note also mentions a bit. But here the proposal only focused on the possible values for each synchronization signal for power evaluation. Make changes as follows. Please see [H] Proposal 1C-4-v3 |

- [H] Proposal 1C-4-v3:

The period of low power synchronization signal for power evaluation can be

- {20ms, 40ms, 80ms, 160ms, 320ms} for evaluations assuming SSB

- For evaluations assuming LP-SS

  - { 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)}

- Other values are not precluded

Note: companies to report the purpose of the low power synchronization signal along with evaluations , e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

- [H] Proposal 1C-4-v4(after Friday offline):

The period of low power synchronization signal that LP-WUR used for at least power evaluation can be

- Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR

- For evaluations assuming LP-SS

  - { 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)}

  - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy

- Other values are not precluded

Note: companies to report the purpose of the low power synchronization signal along with evaluations , e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

| Company | Comments |
| --- | --- |
| FL3 | Please companies to comment [M] Proposals 2C-v3(after Friday offline) |
| xiaomi | fine |
| Spreadtrum3 | Fine |
| CATT | The first bullet in the proposal implies that LP-WUR can receive SSB and use it for synchronization. If the LP-WUR could receive SSB, why is the LP-SS used for? The second bullet assumes LP-SS with long periodicity larger than the time interval 160 ms for UE maintaining in sync to the network. Thus, the out-of-sync LP-WUR would take a long time at each LP-SS cycle to achieve the synchronization using LP-SS. The number of slots of the LP-SS transmission at each cycle to achieve T/F synchronization should be justified. We would suggest including 160ms periodicity for the LP-SS in the evaluation assumption. |
| OPPO | Fine. |
| Apple | Fine |
| ZTE, Sanechips | We are fine to include 160ms as CATT suggested, and our motivation also has been stated in previous round. |

- 1C-5: Sync/re-sync assumption:

| Futurewei | LP-WUS may imply/convey some timing and serving cell information to MR. And study two alternatives for MR sync/re-sync: - MR Sync Alt 1: PSS/SSS search for, e.g., [40, 80, 120]ms, with no LP-WUS assistance. - MR Sync Alt 2: No PSS/SSS search with LP-WUS assistance. |
| --- | --- |
| CATT | at least 3 SSBs are needed for MR acquiring Time/Frequency synchronization after wakeup |
| ZTE | at least 3 SSBs are required for sync/re-sync when main radio wakes up from the state of ultra-deep sleep. |
| Qualcomm | 1 or 3SSBs for sync/re-sync considering different SNR. Proposal 4: Use the following values for additional X time units required for sync/re-sync of the MR: - X = 50 ms for low SNR - X= 20 ms for high SNR |
| vivo | Table 1. The assumptions of Sync/re-sync time and energy Sync/re-sync time [ms]; Sync/re-sync energy [ms*units]. Number of SSBs for sync/re-sync =3[1]; 60; 2180. Number of SSBs for sync/re-sync =5[1]; 100; 3340. Note 1: 10ms continuous monitoring from the beginning of sync-re-sync time is assumed.. Proposal 4: For sync/re-sync time and energy, Table 1 of R1-2302506 can be assumed. |
| Spreadtrum | Proposal 1: 3/6/9 SSBs for sync/re-sync for ultra-deep sleep can be assumed for different channel condition respectively. |
| InterDigital | Proposal 4: For total time for sync/re-sync of MR, support up to 10 SSBs for FR2 as well as FR1. |

- [M] Proposal 1C-5-v1:

For FR1 evaluation,

- Number of SSBs for sync/re-sync for MR can be at least 3

  - Companies to report sync/re-sync time and energy consumption

| Company | Comments |
| --- | --- |
| Futurewei | The minimum number of SSBs required for sync/re-sync and timeline will depend on whether LP-WUR can assist the MR in time and frequency synchronization or not. We suggest to wait till discussions on LP-WUR synchronization and assistance to MR is progressed under AI9.11.3. |
| DOCOMO | We agree with Futurewei the number of SSBs needed depends on whether there is the ability of the WUR to transfer information about synchronization to the MR. Thus, we suggest the following modification. For FR1 evaluation, - Number of SSBs for sync/re-sync for MR can be at least 3 in cases where LP-WUR does not assist the MR in time and frequency synchronization, - Companies to report sync/re-sync time and energy consumption - FFS: Number of SSBs for sync/re-sync for MR in cases where LP-WUR assist the MR in time and frequency synchronization |
| Spreadtrum | Fine |
| Huawei, HiSilicon | Not sure what’s the purpose of this proposal since it implies a large range of sync/re-sync time. The number of SSB needed could be reduced to less than 3 if LP-WUR can assist the synchronization of MR. |
| QC | As HW pointed, LP-WUR could also help MR sync. Prefer to leave company to report. |
| ZTE, Sanechips | OK with the proposal. |
| Xiaomi | Fine with the proposal. |
| Nokia1 | As noted others, it would be good to clarify whether this assumption is independent of e.g. possible existence of LP-SS and related assistance information from LR. As discussed earlier, the RAN4 requirements (for CONNECTED mode UEs) give somewhat pessimistic picture of the timeline/number of SSBs, thus these numbers seem rather aggressive, but if companies are fine to commit to these from latency perspective, no strong concern. Also the time for continuous search would be good to align e.g. 20ms prior the 3 SSBs. |
| Intel | After waking-up, the MR needs to do sync/resync and do RRM too. Is the current proposal of 3 SSBs shared for both sync/resync and RRM (for both serving cell and intra-frequency cells) ? We share views from FutureWei and DoCoMo. With assistance of LP-WUS, UE can know the timing of MR (assuming LP-WUS timing is configured related to MR timing), though there is still a need for frequency synchronization. |
| Samsung | We think that the number of SSBs can be different according to SINR and whether assistant from LP-WUR on timing determination is used (which is up to implementation), thus lower than 3 SSB can be also used for sync/re-sync of MR waked up from ultra-deep sleep state. |
| CATT | OK with the proposal. |
| Apple | OK |
| LGE | Fine with the proposal. |
| FL1 | At least two discussion point - Q1: Do you think the number of SSB depending on whether LP-WUR assist the MR in time and frequency synchronization? [DOCOMO][Huawei][Nokia] - Q2: Do you think whether RRM for MR is performed for sync/resync process? [Intel] Based on that, Q3 is asked Q3: which is preferred for you, - Alt 1: Companies to report assumptions asked in Q1 and Q2 and the number of SSBs for sync/re-sync for MR - Alt 2: Companies to report assumptions asked in Q1 and Q2, agree on value(s) for each case - Alt 3: Companies to report the number of SSBs for sync/re-sync for MR without clarifying anything. |

| Company | Comments |
| --- | --- |
| Futurewei | We would prefer Alt 2. |
| Ericsson1 | We are OK with Alt1 at this stage. |
| Spreadtrum2 | Alt 2 is encouraged, but Alt 1 is enough. In our assumption, 3 SSBs is suitable for good channel condition, without assumption of LP-WUR assistance, and with assumption of sync/resync is based on these 3 SSBs. In PEI discussion, 1 SSB is assumed for good channel condition, so we think there could be 2 more SSBs used for sync/resync after wakeup from ultra-deep-sleep. |
| Samsung | We prefer Alt 1. For Q1 and Q2, our answer is Yes for both question. |
| CATT | Alt2. |
| MTK | Q1: No. LPWUR can assist SFN to reduce MR blind timing search. To save SSB, LPWUR must maintain clock and PLL when MR enters to ultra-deep sleep. However, even that MR still needs 3 SSB for AGC settling. Considering at least 3 SSBs is reasonable. Q2: Yes Q3: Alt2. |
| Vivo | Either Alt 1 or Alt 2 is OK |
| Nokia2 | Alt1 would be aligned in our thinking. On Q2, in my recollection in earlier evaluations it was assumed that the serving cell evaluation was included in pre-PO SSB detection. |
| Intel | Alt 2 is preferred. But Alt 1 is acceptable if hard to converge on the values |
| Huawei, HiSilicon | Alt.2 is preferred, but if not achieved in the meeting, Alt.1 is acceptable. |
| FL2 | Q3: - Alt 1: Ericsson, Spreadtrum, vivo, Nokia, Intel(2nd), Huawei(2nd) - Alt 2: Futurewei, Spreadtrum, CATT, MTK, vivo, Intel, Huawei It seems still controversial. FL encourage companies to further comment |
| OPPO | ‘Yes’ for both Q1 and Q2. For Q3, we prefer Alt 2, and Alt1 is acceptable. |
| Xiaomi | Fine with either Alt1 or Alt2. |
| Apple | Q1: Most likely no. It may help the MR define a smaller search window, but sufficient number of SSBs would still be necessary to achieve the required synchronization accuracy. Q2: Yes, at least for serving cell measurement. Q3: no strong view. We are fine either way. |
| ZTE, Sanechips | Currently, we only have time to go with alt1. |

- 1D: Others

- Discussion on resource overhead or NW power consumption:

need to be considered and modeled: Futurewei, Ericsson, ZTE,vivo, Sony

not support to consider NW power consumption as performance metric: InterDigital

- Evaluation assumption on MR sleep state during LP-WUS monitoring for RRC connected mode

Vivo: In RRC connected mode, Ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring.

- Correct per UE paging probability to RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF

Nordic

- Number of UE in each group:

CATT: less than 8

Ericsson: 10

| Futurewei: | - Study the impact of defining a shorter DRX cycle (<320 ms), i.e., for the MR to monitor POs after waking up due to reception of LP-WUS, on latency and overall paging resource overhead. - Resource overhead: LP-WUS design options/functions and mapping to time/frequency resource requirements need to be defined for proper evaluation of the network resource overhead in support of LP-WUS/WUR. - Consider LP-WUR monitoring of at least a tracking and/or a RAN notification area level beacon that is transmitted with reasonable periodicity to alleviate the impact of MR’s low periodicity RRM measurements on latency. - Latency definition: For a LP-WUS carrying a UE unique ID, the latency is defined as the average time between the arrival of data at gNB and the UE’s completion of MR synchronization upon detection of a corresponding LP-WUS. - Ignore the latency impact of SI update on paging procedure due to its infrequent occurrence as part of paging procedure. |
| --- | --- |
| CATT | Proposal 4: The Latency_2 is mainly affected by the MR ramp-up and preparation for paging message reception with the addition of the RACH response time. The Latency_2 is suggested in a range of 500ms~2200ms. Proposal 5: The total latency can be divided into three independent parts associated with different dependent components: Latency_0 for gNB preparation, Latency_1 for LP-WUR monitoring the LP-WUS and Latency_3 for MR waking up and detecting paging message, the expression of latency can be formulated as: Total Latency = Latency _0 + Latency_1 + Latency _2. Discussion on the maximum number of UE in each group should be less than 8 Proposal 6: The number of UE in the same group should be less than 8 for 14.8% paging group rate under 1% paging rate per UE. Proposal 7: The number of UE in the same group should not be more than 8 for i-DRX and e-DRX with RE, REF below 1%. |
| Ericsson | For evaluations, use N=10 and the resulting range of RG={10%, 1%, 0.1%, 0.01%} for the per group paging probability. The following general framework should be used as starting point for WUS evaluations: - Transmission of LP-WUS should not require new gNB hardware and should not trigger new emissions/compliance requirements for gNBs. - It should be possible to dynamically reuse unused LP-WUS resources for other NR transmissions (i.e., dedicated time/frequency resource reservation for WUS should be avoided). - It should be possible to multiplex LP-WUS with other NR transmissions in time or frequency domain without causing interference. - LP-WUS is transmitted on Uu interface from gNB to UE. Impact of LP-WUS/WUR operation on NW Energy Efficiency should be considered especially if LP-WUS transmissions require significantly more time/frequency resources compared to PDCCH or require additional always-on transmissions (e.g., LP-SS) from gNB. |
| ZTE | Discuss the assumption for system overhead and NW power evaluation. Proposal 1: The following KPIs on LP-WUS should be further evaluated • System overhead • Network power consumption • Co-existence impacts Proposal 2: Clarify the latency definition for different RRC states. the latency means the time interval between LP-WUS and PRACH in idle/inactive mode. In connected mode, the latency means the time interval between LP-WUS and PDSCH with data. Proposal 13: Discuss the LP-WUS transmission assumption in idle/inactive mode for evaluation and capture how to calculate the system overhead. Proposal 14: For NW power evaluation, discuss the LP-WUS or LP-SS transmission assumption, and load scenarios in idle/inactive mode. |
| Samsung | Proposal 2: The latency for RRC_CONNECTED state is defined as the time interval between the data arrival time at the gNB and the time of the first UE specific data channel reception. Proposal 6: The power model in the Table 3.2 should be considered as a baseline to evaluate i-DRX/e-DRX operation for eMBB case. |
| Qualcomm | Proposal 2: Following KPIs are evaluated: data rate, false wakeup probability (due to grouping and false alarm), and misdetection probability. Proposal 5: For study, add following additional power numbers (0, 10,20,40) for LP-WUR power consumption in On state. Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0/0.01/0.05/0.1/0.5/1/2/4/10/20/40 FFS: If other values are needed. - Note: 0 is for Genie LP-WUR which can be used to show the lowest power consumption (or highest PSG). - Note: Ramp-up time for LP-WUR must be much lower than 15 ms since deep sleep (DS) ramp-up + ramp-down time in 38.840 is 20 ms - TLR, ramp-up is FFS, and company to report TLR, ramp-up - FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up - FFS: whether further categorization/sub-categorization is needed and how. Observation 1: Possible options for LP-WUS BW configuration include 1MHz, 5MHz, and 20MHz. Proposal 8 : Prioritize 5MHz for LP-WUS bandwidth. Proposal 10: For both link level and power evaluations LP-WUS, the following false-alarm rate (FAR) of LP-WUS can be assumed: [0.1%, 1%, 10%]. Proposal 11: Target false alarm probability of LP-WUS is at most 1%. |
| Nokia | Proposal 1: Down prioritize the sidelink related studies for time being. Proposal 2: Consider implications to network energy efficiency in studied LP-WUS related designs. Proposal 3: LP-WUS design and LP-WUR architecture should support flexible placement in frequency domain. Proposal 4: The wake-up signal design and wake up receiver architecture defined, allows efficient reuse of gNB hardware for signal generation. Proposal 5: The LP-WUS/WUR design should ensure that legacy receiver performance is not affected and efficient multiplexing with existing NR signals and channels is possible to limit the resource reservation. - Proposal 6: Coverage and mobility implications should be accounted for in LP-WUS design and LP-WUR architecture assumptions. |
| vivo | Observation 28: Additional network energy consumption caused by LP-WUS/WUR operation can be minimized, e.g., gNB can transmit LP-WUS in the slot with existing NR signal that to be transmitted. Proposal 8: In RRC connected mode, Ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring. |
| Xiaomi | Proposal 1: For RRC idle/inactive state, two use cases can be considered for evaluation: Case 1, LP WUS combined with legacy paging mechanism; Case 2, LP WUS combined with enhanced paging mechanism. |
| Nordic | Proposal-3: For e-DRX, per UE paging probability should corrected to RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF |
| Sony | Observation 5 – LP-WUS length, the amount information it carries, and the technique used for its multiplexing in an OFDM transmitter impact the number of resources for LP-WUS transmission and its associated system overhead. Proposal 5 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme. Average delay = ½ sleep time + transition time + signal miss detection × average time for re-transmission |
| InterDigital | Proposal 1: NW power consumption/energy efficiency is not adopted as a performance metric. Proposal 5: Confirm the current definition of transition energy (i.e., TLR, ramp-up *(PON – POFF)/2) for LP-WUR. |
| Huawei | If UE is not required to monitor a PO after wake-up, latency is the time interval between the data arrival time at the gNB, and the time of the first RO UE can transmit PRACH in after LP-WUS detection. |

- [M] Proposal 1D-1-v1:

Update as followings for the e-DRX paging probability

Note:

- For i-DRX with cycle duration Y second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

- For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW

  - L=4

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the update. |
| DOCOMO | We support this proposal. |
| OPPO | OK with the proposal. |
| Huawei, HiSilicon | OK |
| QC | We prefer to have L=1 and 4. L=1 would correspond to the case of having i-DRX with much larger cycle than allowed for now. This is useful configuration for study purpose. |
| ZTE, Sanechips | OK |
| Xiaomi | Fine with the proposal. |
| Nokia1 | OK |
| Intel | We support this proposal. |
| Samsung | We are fine to update. |
| CATT | Agree with the updated proposal. |
| Apple | OK |
| LGE | Fine with the proposal. |
| Ericsson1 | OK |
| MTK | Okay |
| FL1 | @Qualcomm, the purpose of the proposal is correct the formula. I would like to know if other companies can accept adding L=1 before updating. |

| Company | Comments |
| --- | --- |

- [M] Proposal 1D-2-v1:

Update the transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model.

- Note: this assumes the power consumption during the transition time is sum of transition energy and LP-WUR OFF energy, e.g., similar definition as the transition energy in TR38.840

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the update. |
| DOCOMO | We support this proposal. |
| OPPO | OK with the proposal. |
| Huawei, HiSilicon | We are fine with the update. |
| QC | okay |
| ZTE, Sanechips | OK |
| Xiaomi | Fine with the proposal. |
| Nokia1 | OK |
| Intel | We support this proposal. |
| Samsung | Considering the background of update for the equation, we suggest to change wording from “the transition energy” to “additional transition energy” as follows: Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model. - Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 |
| CATT | Agree. |
| Apple | OK |
| LGE | Fine with the proposal. |
| Ericsson1 | OK |
| MTK | Ok |
| FL1 | It seems the wording proposed by Samsung is reasonable. The proposal is updated as follows, |

- [M] Proposal 1D-2-v2:

Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model.

- Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840

| Company | Comments |
| --- | --- |
| OPPO | Support. |
| Xiaomi | fine |
| Apple | OK |
| ZTE, Sanechips | OK |

Issue 2: Others

- 2A: Remaining issues for use case descriptions

| Qualcomm | Proposal 1: - Latency requirement for IoT Idle mode cases including e.g., industrial wireless sensors, controllers, actuators and etc. - Order of seconds (0.64, 1.28 sec) - Latency requirement for wearable Idle mode cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - Order of seconds (0.64, 1.28 sec) - Latency requirement for eMBB Idle mode cases including e.g., smart phones and etc., - Latency requirement for eMBB/XR Connected mode cases including - Order of milliseconds [0.5, 1, 2ms] |
| --- | --- |
| Ericsson | Observation 1 Latency requirements for use cases mentioned in the SID such as industrial controllers, actuators etc., and wearables range from tens of milliseconds, hundreds of milliseconds to several seconds. For XR, the requirements are in few milliseconds to few tens of milliseconds range. Proposal 1 Study the following further: • Applicability of RRC IDLE/INACTIVE vs. RRC CONNECTED mode operation of LP-WUS/WUR considering latency requirements and expected data activity for different use cases mentioned in the SID. • Feasible latency at which LP-WUR can wake up MR while still providing power saving gain. Proposal 2 Include ‘latency’ as a use case characteristic for IoT, Wearables, and eMBB. |
| vivo | Proposal 2: Consider the following as the latency target by applying LP-WUS/WUR: - Within 1 or 2 seconds for latency-sensitive IoT cases and within several or tens of seconds for other IoT cases; - Within several seconds for wearable cases; - Within several milliseconds for XR use case and within tens of milliseconds for other eMBB use cases; |
| Sony | Proposal 1 – Prioritize LP-WUS/WUR for power-sensitive, low-traffic, small form factor devices as in IoT use cases (such as industrial sensors, controllers) and wearables where delay requirement or device reachability in time is short. |
| LG | Proposal 1: Update the following latency characteristic for target use cases. - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - latency-tolerable (e.g., the order of seconds) - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - latency- tolerable (e.g., the order of seconds) - eMBB cases including e.g., XR/smart glasses, smart phones and etc., latency-sensitive (e.g., the order of milliseconds) |
| Nordic | Proposal-1: For the latency of industrial IoT use-case, consider 100ms to 10s as design criteria. |

- [M] Proposal 2A-v1:

The latency for the target use cases are considered as follows:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - Latency for RRC CONNECTED mode is in the order of milliseconds

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the proposal |
| OPPO | OK with the proposal. |
| Spreadtrum | Latency for RRC CONNECTED mode should be stringent for indication in scheduling PDCCH, e.g. PDCCH skipping, SSSG switching, but it can be relaxed for WUS before C-DRX like DCI format 2_6. For RRC IDLE/INACTIVE, some companies proposed to use LP-WUS for IoT case to enable latency reduction. In this case continuous monitoring is efficient. - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - Non time critical: Latency for RRC IDLE/INACTIVE mode is in the order of seconds - Time critical: Latency for RRC IDLE/INACTIVE mode is in the order of milliseconds - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - Latency for RRC IDLE/INACTIVE mode is in the order of seconds - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - Latency for RRC IDLE/INACTIVE mode is in the order of seconds |
| Huawei, HiSilicon | It is already clear enough for the use cases discussion, and we have already used much time to discuss without a consensus to includes these latency target. No need to use our limited time to repeat the discussion. |
| QC | Agree with FL w/o change. |
| ZTE, Sanechips | For eMBB especially for XR traffic, to be more specific, the latency should be order of several milliseconds |
| Nokia1 | We are OK with the FL proposal. |
| Intel | We support this proposal. |
| InterDigital | Support |
| Samsung | eMBB cases for RRC IDLE/INACTIVE mode should be added “in the order of seconds” |
| CATT | OK with the proposal. |
| Apple | We would like to add the sub-bullet “Latency for RRC IDLE/INACTIVE mode is in the order of seconds” for eMBB case as well, which is a reasonable target. |
| LGE | Fine with the proposal. |
| FL1 | It seems most companies are OK in principle. FL addressed the following comments, @Samsung, Apple, eMBB cases for RRC IDLE/INACTIVE mode added “in the order of seconds” @Spreadtrum, it is a bit confused for RRC IDLE/INACTIVE mode, how can the latency be achieved in the order of milliseconds. Perhaps more clarifications needed before I can update. Please see a revised [M] Proposal 2A-v2 |

- [M] Proposal 2A-v2:

The latency for the target use cases are considered as follows:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - Latency for RRC CONNECTED mode is in the order of milliseconds

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

| Company | Comments |
| --- | --- |
| Futurewei | We are OK with the update. |
| Ericsson1 | It is not clear to us why the time-critical scenarios that need tighter latency than ‘order of seconds’ are skipped for IoT and wearables. Could the proponents clarify? If going by FL suggested approach, suggest adding separate main bullet at the end that applies all the cases, i.e. as follows: - For above cases - Latency for RRC CONNECTED mode is in the order of milliseconds - Latency for RRC IDLE/INACTIVE mode is in the order of seconds |
| Spreadtrum2 | Fine. We assume continuous monitoring for IoT case, so there could be several millisecond to detect LP-WUS and initiate RACH and then complete RACH within 100ms, if mission-critical is optimized for IoT use case. However, we miss to calculate the transition time 400ms. In our memory, IEEE 802.11ba can support mission-critical use case, since the transition time is not assume so large. We kept in mind the mission-critical use case in one of our targets. The current proposal looks like we do not pursue the latency KPI any more… From our side, we support latency reduction for IoT use case which may not have additional PSG than before. For example, it can be used for LPHAP to reduce latency. |
| Samsung | We are fine to the update. |
| CATT | Ok with the update. |
| MTK | Okay |
| vivo | we are fine with the modification by Ericsson |
| Nokia2 | Just a note that for eMBB and IDLE/Inactive, I’m not sure if we can assume as relaxed call set-up delays as for more M2M type of use cases, thus I would be rather cautious to extend it much beyond second |
| Intel | Ok with the update. |
| Huawei, HiSilicon | Before the agreement, is this as a evaluation target or not? If not, why we need to it? |
| ZTE, Sanechips | We are generally fine. Similar with Spreadtrum, the current proposal seems does not pursue the latency KPI. Therefore, we suggest to add a Note: Note: the reduced latency is still a pursued KPI. |
| FL2 | @Huawei, we have FFS for latency in the last meeting agreement. This is proposal is try to solve this. @Spreadtrum, even without 400ms transition time, it’s still need more than several milliseconds to transmit data in IDLE. @Nokia, understand your concern. FL has no idea how to modify. Perhaps what Ericsson proposed change can be considered, since it is a general description to all. Considering majority companies are OK with this. I made changes as follows, |

- [M] Proposal 2A-v3:

The latency for the target use cases are considered as follows:

  - Latency for RRC CONNECTED mode is in the order of milliseconds

  - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

| Company | Comments |
| --- | --- |
| Apple | OK |
| ZTE, Sanechips | OK |

- [Suspend]2B: target power for LP-WUR

| vivo | Observation 1: In the case that the relative power of LP-WUR “on” state is 0.03~0.5 unit, substantial power saving gain e.g., up to 80% can be achieved. Observation 2: LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB, thus keeping the “ON” state power as low as possible is important for increasing the battery life of the device. Proposal 1: The target relative power of LP-WUR “on” state should be less than 1 unit. |
| --- | --- |
| OPPO | Proposal 2: For I-DRX cycle, the different value of “additional transition energy from ultra-deep sleep” will case different conclusion of whether LP-WUR has power saving gain compared to I-DRX with PEI or not. Prioritize the case ‘LP-WUR on state’ is smaller than 1. |
| Apple | Proposal 1: Do not set a tight power consumption target for LP WUR at this stage. The tradeoffs should be carefully considered. |

Moderator encourage companies to express more views on this topic and how to proceed.

- [M] Proposals 2B-v1:

| Company | Comments |
| --- | --- |
| Futurewei | In our observations, we noticed that even with duty cycled monitoring, considering LP-WUR relative “ON” power greater than 4 units can result in a noticeable reduction in the power saving gain. Therefore, we suggest considering a target maximum “ON” relative power <4 units. |
| Spreadtrum | There are values for evaluation purpose already. The power saving gain is more important. |
| QC | RAN1 is in the middle of studying various LP-WUS power numbers (including 1, 4, 10, 20) with duty cycled based monitoring scheme. So, we think there is no need to make premature conclusion especially toward “1 unit”. |
| Intel | The targeted relative power should be discussed together with OFDM based receiver. From the power consumption study, the high relative power cause smaller power reduction gain, and even larger power consumption compared with the baseline for certain value of relative power. |
| Samsung | Taking into account much larger relative power values than 4 are still being discussed, we don’t prefer setting a specific relative value as the target. |
| Apple | We do not see a need to agree on a target at this point. We can further discuss the power saving evaluation results before making such a decision now. |
| FL1 | Considering the comments received, FL suggest to deprioritize this discussion |
| SONY | To have a fair evaluation, we need to have mapping between LR architecture, and its corresponding NF and power consumption. |
| MTK | Prefer less than 1 to support RRC CONNECTED use cases. In this case, LPWUR needs to monitor at least 1 occasion per OFDM slot. Low monitoring power consumption is needed. |

- 2C: target coverage for LP-WUS

Ericsson propose that, for coverage evaluations, LP-WUS/WUR designs that strive to match the coverage for NR PDCCH should be considered.

Samsung: The coverage for LP-WUS/WUR should be comparable to at least that of the NR downlink channel.

Huawei propose to use PUSCH or Msg3, as the reference for coverage evaluation of LP-WUS.

Qualcomm

Proposal: RAN1 strives to design LP-WUS to have a similar coverage as NR channel X.
 FFS: The NR channel X is either PDCCH or PUSCH.
 FFS: Channel configuration of X

ZTE propose that the target coverage of LP WUS should be better than PUSCH.

Proposal 12: RAN1 strives to design LP-WUS to have a similar coverage as NR [PDCCH] channel.

Moderator encourage companies to express more views on this topic and how to proceed.

- [M] Proposals 2C-v1:

| Company | Comments |
| --- | --- |
| Futurewei | We suggest to consider a coverage target that is at least equivalent to PUSCH and strives to approach that of PDCCH, but not necessarily set PDCCH coverage as target to have more flexibility to trade-off power saving gain, resource overhead, and latency. |
| Spreadtrum | Strive to paging PDCCH, otherwise LP-WUS cannot be used at cell edge and transition of main radio may happen often, and thus the power saving gain in real deployment will be gone. |
| Huawei, HiSilicon | The bottleneck channel, i.e. PUSCH, is preferred to be as the coverage target. For LP-WUS used in IDLE/inactive mode, we can be flexible to further compromise to set the target as the coverage of PUSCH for Msg3 considering the Msg3 is the bottleneck of coverage during the IDLE/inactive states. We don’t see a need to over-optimize the LP-WUS to have the same coverage of PDCCH. |
| QC | @FL, please capture QC view (above) as well. We encourage companies to do detailed study on coverage, data rate and overhead. We propose to make an agreement on following proposal as a first step. Proposal: RAN1 strives to design LP-WUS to have a similar coverage as NR channel X. FFS: The NR channel X is either PDCCH or PUSCH. FFS: Channel configuration of X |
| ZTE, Sanechips | Better than PUSCH would be enough, which can cover most of the UEs in a cell. Moreover, this is a good starting point, since we still can pursue the coverage similar with PDCCH. |
| Xiaomi | The bottleneck channel needs to take PUSCH into account and the LP WUS coverage should be superior to the bottleneck channel. |
| Nokia1 | Thus the question is whether we will aim for full coverage or partial coverage. There is evidently cost associated to requiring LR (and LP-WUS) to support higher/full coverage. If we on the other hand in practice require MR to be used at the cell edge for mobility evaluations e.g. for neighboring cells, implying more power consuming design for LR might not be justified. From this perspective, it might be more practical to consider partial coverage. However, it would increase the feasible deployments for LP-WUS if better coverage can be attained. |
| Intel | From our evaluation, the MIL for LP-WUS can be better than PUSCH but is much worse than common PDCCH. Since PUSCH or msg3 is involved in cell section/tracking area update, it would be fine to take PUSCH or msg3 as baseline. The assumed data rate for PUSCH should be further clarified. |
| Samsung | Our preference is to set the target as PDCCH. On the other hand, if it is difficult to achieve consensus, something like “strive to approach PDCCH” would be fine. |
| Apple | We are fine with the spirit of the proposal, but we prefer QC’s version. We may need to be a bit cautious about the various tradeoff. |
| LGE | We believe that the target coverage of LP-WUS is not necessarily PDCCH. We may lose power saving gain to target PDCCH coverage to strive to design LP-WUS to have a similar coverage of NR. So, we can further study and discuss the case of the coverage of LP-WUS being smaller than the target coverage of NR and how to handle the unbalanced coverages between LP-WUS and NR. |
| FL1 | Supported companies are as follows, PDCCH: Spreadtrum, Nokia(?), Samsung PUSCH: Futurewei, Huawei/ HiSilicon (msg3), ZTE/ Sanechips, Xiaomi, Intel(msg3), LGE(?) Considering this, FL suggest the WF1 for PDCCH, WF2 for PUSCH,and WF3 to proceed as compromise. Please see proposal 2C-v1 |

- [M] Proposals 2C-v2:

RAN1 strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is

- Option 1: PDCCH

- Option 2: PUSCH,

  - FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3

- FFS other options

| Company | Comments |
| --- | --- |
| SONY | OK with the proposal, however what do “WF1”, “WF2”, “WF3” refer to in the FL1 comment? Our basic view is that for coverage evaluations, LP-WUS/WUR designs should have a comparable coverage to PUSCH and strive to match the coverage for NR PDCCH. |
| Futurewei | We are OK with the proposal |
| Ericsson1 | LP-WUS controls paging PDCCH reception RAN1 should strive to design LP-WUS to not impact paging. We do not follow why RAN1 should strive to have a target that is significantly worse. |
| Spreadtrum2 | OK. It can be scalable, e.g. PUSCH for IoT, PDCCH for wearables/eMBBs. We still think there could be at least two categories for design. IoT: - Continuous monitoring - Low power value for LR - No RRM: stationary or low mobility - Initiate RACH after wakeup to reduce latency - Small MR transition energy/time for special device, e.g. LPHAP - DRX for MR, entering deep-sleep usually Wearables/eMBBs - Duty cycle - High power value for LR - RRM: up to medium mobility - Monitor PO after wakeup to reduce overhead of LP-WUS - Large MR transition energy/time - eDRX for MR, entering ultra-deep-sleep usually |
| Samsung | We are OK to proposal in principle, and these options can be down-selected further. We think that the assumed aggregation level (AL) of PDCCH for Option 1 can be specified as FFS, because AL can affect the coverage performance. |
| xiaomi | Generally fine. And AL 8 and AL 16 should be prioritized for PDCCH. |
| CATT | OK with the intention of this proposal. |
| MTK | Okay |
| vivo | @Ericsson, system should be affected by bottleneck channel. For IDLE state, msg3 coverage is even worse than paging PDCCH, so it ‘s not useful to target LP-WUS even better than msg3. So we support the proposal [M] Proposals 2C-v1 as a compromise to proceed. |
| Nokia2 | At least we have seem to have common understanding that the LP-WUS coverage does not need to exceed the PDCCH coverage. 😊 Evidently whatever we choose, should not affect the system paging performance. So, if the LP-WUS coverage is less than paging coverage, we would need to have mechanism in place for the UE to gracefully fallback to normal paging monitoring. This maybe required also if paging PDCCH and LP-WUS coverage are roughly equal, as due to larger diversity order (freq/spatial) the PDCCH coverage may be occasionally better. Hence a question from myside would be that do we take similar assumption as in Rel-17 that it is up to the UE whether it monitors PEI or directly to PO, but paging performance must not be impacted. I.e. is the UE mandated to monitor LP-WUS (if configured/supported)? |
| Intel | Ok with the update. |
| Huawei, HiSilicon | We are fine in general. Actually, it is possible to set both channels as targets, i.e. target#1 and target#2. As a work in study phase, the techniques to achieve each target can be identified. Maybe this can be as some compromise. |
| ZTE, Sanechips | We want to provide a medium option: Option 3: at least comparable or better than PUSCH |
| FL2 | It seems in principle fine. @Nokia, from FL perspective, I have no idea to your question. Let’s see more comments. @SONY, sorry for typo, WF1/2/3 should be option 1/2/3 @Samsung, suggest to solve the details later. |
| OPPO | Share similar views as SONY. LP-WUS/WUR designs should have a comparable coverage to PUSCH at least, and strive to match the coverage for NR PDCCH. If the coverage of LP-WUS/WUR can’t match the PDCCH’s, it is better to reduce the coverage gap as much as possible. |

- [M] Proposals 2C-v3(after Friday offline):

RAN1 further consider the design of strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is

- Option 1: PDCCH for broadcast

- Option 2: PUSCH,

  - FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3

- FFS other options

- The final design will jointly consider the coverage target with other KPI

| Company | Comments |
| --- | --- |
| FL3 | Please companies to comment [M] Proposals 2C-v3(after Friday offline) |
| Xiaomi | Generally fine. And AL 8 and AL 16 should be prioritized for PDCCH in order to reduce the workload of simulation. The updated proposal with modifications as follows: RAN1 further consider the design of strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Option 1: PDCCH with AL 8 or 16 for broadcast - Option 2: PUSCH, - FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options - The final design will jointly consider the coverage target with other KPI |
| FL4 | @Xiaomi, there could be many details behind each channel. If I remember correctly, in the last meeting when we discuss the evaluation assumptions for PDCCH, we had some discussion related to the AL. instead of debating which AL(e.g., AL8 or AL16), I think so far we can keep it open for companies to report. RAN1 further consider the design of strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Option 1: PDCCH - Option 2: PUSCH, - FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options - The final design will jointly consider the coverage target with other KPI |
| Spreadtrum3 | Fine |
| CATT | The current assumption of coverage of PDCCH and PUSCH are not clear for us. The coverage of PDCCH or PUSCH in TR38.830 for coverage enhancement includes several values for the target coverage in Tables 5.1.1.3-1 to 5.1.1.3-6. For example, in Tables 5.1.1.3-1, the gNB Tx power is 33 dBm/MHz, and the maximum coupling loss (MCL) of PUSCH has values for eMBB and VoIP with different TDD configurations range from 137.58 to 143.79. Thus, we need to agree on a single number of MCL for PDCCH and PUSCH for the evaluation. |
| OPPO | Fine. |
| Apple | Fine. Maybe “KPI” -> “KPIs” 😊 |
| ZTE, Sanechips | We would like to suggest to change ‘similar’ as ‘comparable’, since if we use ‘similar’, then we should have a similar coverage performance with one of the NR channel, instead of a medium coverage performance between two NR channels. Additionally, a FFS is added to address CATT’s concern. RAN1 further consider the design of strives to design LP-WUS to have a similar comparable coverage as NR channel X. The NR channel X is - Option 1: PDCCH for broadcast - Option 2: PUSCH, - FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - Option3: Option1+Option2 (better than PUSCH, worse than PDCCH) - FFS other options - The final design will jointly consider the coverage target with other KPI - FFS the details assumption for NR channels |

Evaluation results

Moderator: To be handled in another documents after consolidating the results.

Please see R1-2304151

For your information, FL have uploaded a summary in the inbox for the LP-WUS evaluation results handled in 9.11.1 this meeting. You can download from here R1-2304151

The following is included in R1-2304151

- FL summary of evaluation results (V009)

- Excel sheet for collecting power/latency evaluation (V031)  (Some new columns are added, please see ‘change history’ for detail information)

- Excel sheet for collecting coverage evaluation (V009)

Note: you can also find these attachments in the draft folder.

Email discussion during meeting

During the email discussion, the following is agreed,

- [M] Proposal 1D-1-v1:

Update as followings for the e-DRX paging probability

Note:

- For i-DRX with cycle duration Y second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

- For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW

  - L=4

- [M] Proposal 1D-2-v2:

Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model.

- Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840

- [H] Proposals 1A-2-v3:

Working assumption

For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is:

- Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

- #10: [H] Proposal 1A-4-v8

Confirm the WA from RAN1#112 and update as followings

Working Assumption

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |  |
| --- | --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |  |
| RTC max frequency error [ppm], FFS: RTC frequency drift [ppm/s] | (20 , FFS:[0.1]) |  |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than [TBD] power unit,

  - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

    - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

    - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

    - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

    - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

    - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

    - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

    - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

    - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

    - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

- FFS: Phase noise model

Working assumption: The following for usage of the clock is assumed for LP-WUR OFF/ON

| Assumption on LP-WUR OFF power | Assumptions on the clock usage |
| --- | --- |
| 0.001 | When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done. |
| TBD: value(s) | For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration |

[Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power]

FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1

Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

Remaining issues

Evaluation results

See R1-2304151

Evaluation methodologies

- #8: [H] Proposal 1C-2-v12:

- For LP-WUR power evaluation,

  - Relative Power (unit) for LP-WUR OFF state,

    - 0.001/ Y1/ [Y2], [FFS value(s) of Y1, Y2 e.g., 0.1, 0.01, 0.005]

  - Relative Power (unit) for LP-WUR ON state,

    - 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30

      - 10/20/30 for LP-WUR ON power state are not used for receiver types based on envelope detection for MC-ASK and MC-FSK,

      - For other values, the mapping between power value and receiver type are FFS

  - Note: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

- [M] Proposal 2A-v3:

The latency for the target use cases are considered as follows:

o    Latency for RRC CONNECTED mode is in the order of milliseconds

o    Latency for RRC IDLE/INACTIVE mode is in the order of seconds

- [M] Proposals 1A-5-v1:

  - As start point, adopt the phase noise model defined by IEEE 802.11ba for LP-WUS study

  - Change the assumption of delay spread from optional 1000ns to optional: 100ns, considering the agreed values of 300ns and 1000ns result in larger delay spans than the CP length for 30kHz SCS.

  - Regarding the adjacent subcarrier interference, on resources mapped with PDSCH, the random 16-QAM symbols can be mapped on the REs of the PDSCH to model the neighboring subcarrier interference.

  - Regarding other cell interference, it can be modelled by considering one or two neighboring cells to transmit random 16QAM symbols on REs within the cell bandwidth in the link simulation.

Summary of the previous agreements

- RAN1#110bis-e

For future meetings on LP WUS:

Use the following terminology for future discussion,

- Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up

- LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

Agreement

For evaluation, 1 Rx chain for LP-WUS receiver is baseline.

Agreement

Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI.

- FFS: Further prioritization if needed during the study item.

Agreement

Take the following power model for main radio for evaluation in LP-WUS/WUR SI,

- For IoT and wearable cases, reuse TR38.875 power model as baseline.

- For eMBB and other cases, reuse TR38.840 power model as baseline.

- Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver

  - FFS: The details of ‘Ultra-deep sleep’ power state

R1-2210512	FL summary#2 of evaluation on low power WUS	Moderator (vivo)

Agreement

- The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation

| Power State | Relative Power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | [0.015] | [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection | [400ms], FFS: 100ms | X |

Note1:

  - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.],

  - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.],

    - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio

    - Companies can report the assumption of X in the initial evaluation.

  - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated.

- The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync.

  - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption

Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state.

Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS

Agreement

The following power model for LP-WUR/WUS evaluation is considered,

  - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

    - [0.001]

  - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

    - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4]

    - Other values are not precluded to be evaluated.

    - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

  - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed.

Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

FFS: LP-WUR power consumption values for FR2.

Agreement

For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered,

- XR traffic model with evaluation methodologies and assumptions captured in TR 38.838.

- eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840

- Heartbeat traffic models in 3GPP TR 38.875.

- Other models are not precluded.

Company to further provide the followings,

- Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.)

- How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring

- Other details if any

Agreement

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Agreement

For the performance evaluations of LP-WUS candidate designs, it is assumed that

- The miss-detection rate (MDR) of LP-WUS [1%],

- The false-alarm rate (FAR) of LP-WUS

  - [0.1%, 1%, 10%]

  - Other values are not precluded for studying reported by companies

- Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly

- The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode.

- FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead

- FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details.

Agreement

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| FFS: Capacity impact | [Evaluate the system capacity impact due to introducing of LP-WUS] |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided.

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD |
| FFS: UPT | FFS Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement

The following is assumed for RRC IDLE/INACTIVE evaluation,

| Parameters | Value |
| --- | --- |
| i-DRX cycle length | 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI |
| e-DRX cycle length | 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE |
| Number of POs in Paging Frame | 1 |
| Number of DRXs per PTW | 4 |
| Number of SSB before PO / PEI | 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’ |
| Sync/re-sync after ultra-deep sleep | companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync |
| RRM Measurement | Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not. |
| LP-WUS monitoring | Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle. |
| Traffic | Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies. |
| Others | Reported by companies |

Agreement

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

- RAN1#111

Agreement

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| Capacity impact | Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR. |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included |
| UPT | The definition is the same as in [TR38.840] Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement

Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2,

- The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where

  - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

    - FFS: Value of N

- For LP-WUS

  - Both per group and UE paging can be assumed.

Note：

- For i-DRX with i-DRX cycle duration Y second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

- For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

  - L=4 (as agreed in RAN1#110bis)

Agreement

For MR, at least for FR1 evaluation,

- Number of SSBs for sync/re-sync for MR is up to 10

  - Companies to report timeline and energy consumption

- Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

Agreement

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off | 0.001 | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On | 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed |  |  |

FFS: whether further categorization/sub-categorization is needed and how.

- RAN1#112

Conclusion:

The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group

Agreement

The following characteristics for target use cases are considered in the study item:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - FFS: latency

  - primary for small form devices

  - power-sensitive

  - static, nomadic or limited mobility

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - FFS: latency

  - primary for small form devices,

  - power-sensitive

  - low/medium speed, FFS: high speed

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - FFS: latency

  - devices form is various and not restricted

  - power-sensitive

  - low/medium speed, FFS: high speed

Note: other use cases/characteristics are not precluded if any.

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms)

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

Agreement

For coverage evaluation, the following is used,

| Number of RX chains at the UE’s MR antenna elements for UE | Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS. |
| --- | --- |
| Number of RX chains antenna elements for LP-WUR | 1 Rx Note: agreed in RAN1#110bis |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Reference data rates for MR eMBB | Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Reference PDCCH configuration | SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | Urban: NloS Rural: NloS and LoS |
| Bandwidth | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| Delay spread | Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

Agreement

For link-level simulation of LP-WUS, the following table is used as starting point,

- FFS for other assumptions if any

- Note: The assumptions are not intended to limit the scope of the study or the design.

Table XX. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used |
| Channel structure | - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence). |
| SCS of OFDM generator for NR signal | 30kHz/15KHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials |
| Code scheme | Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….) |
| gNB Channel BW | 20MHz, FFS other values |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| Filter | X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional |
| Sampling Rate | - Companies to report. |
| ADC bit width | 1-bit, 4-bit, 8-bit, ideal and other options are not precluded |
| Channel Model | See link coverage assumption table (will copy and paste here) |
| Impairment modelling | - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report |

Working Assumption

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |
| --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |
| RTC max frequency error [ppm] | 20 |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than [TBD] power unit,

    - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

    - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

    - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

    - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

    - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

    - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

    - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

    - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

    - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

    - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

- FFS: Phase noise model

- RAN1#112-bis-e

R1-2304076	FL summary #1 of evaluation methodologies on LP-WUS/WUR	Moderator (vivo)

Agreement

Update as followings for the e-DRX paging probability

Note:

- For i-DRX with cycle duration Y second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

- For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW

      - L=4

Agreement

Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model.

- Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840

Working Assumption

For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm),

- Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

R1-2304150	FL summary #2 of evaluation methodologies on LP-WUS/WUR	Moderator (vivo)

Agreement

The period of synchronization signal that LP-WUR used for at least power evaluation can be

- Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR

- For evaluations assuming LP-SS

  - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms}

  - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy

- Other values are not precluded

Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

Working Assumption

For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials,

- UE have N attempts within T,

  - Company to report (FAR target, T, N)

- For example,

  - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.

  - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt.

- If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent.

Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design.

Agreement

RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is

- Option #1: PDCCH for paging

- Option #2: PUSCH for message3

- FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH)

- The final design will jointly consider the coverage with other KPIs

- FFS additional detail assumptions for NR channels, e.g., the message size for MSG3 and etc.

Agreement

Confirm Alt 2 in the following agreement and update as follows

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms) as baseline

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

Agreement

Confirm the WA from RAN1#112 and update as followings

Working Assumption

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |  |
| --- | --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |  |
| RTC max frequency error [ppm], FFS: RTC frequency drift [ppm/s] | - FFS:[0.1]) |  |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than [TBD] power unit,

  - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

      - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

      - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

      - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

      - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

      - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

      - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

      - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

      - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

      - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

- FFS: Phase noise model

Working Assumption

The following for usage of the clock is assumed for LP-WUR OFF/ON

| Assumption on LP-WUR OFF power | Assumptions on the clock usage |
| --- | --- |
| 0.001 | When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done. |
| TBD: value(s) | For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration |

[Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power]

FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1

Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

SID

RP-222644

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

Reference

The following contributions are submitted in RAN1#112-bis in AI 9.11.1,

- R1-2302331	Evaluation of LP-WUS and Performance Results	FUTUREWEI

- R1-2302339	Evaluations for LP-WUS	Huawei, HiSilicon

- R1-2303897	Evaluation methodologies for R18 LP-WUS/WUR	vivo

Revised from R1-2302506

- R1-2302570	Evaluation for lower power wake-up signal	OPPO

- R1-2302621	Discussion on evaluation on low power WUS	Spreadtrum Communications

- R1-2302687	Remaining issues of Deployment scenarios and evaluation methodologies and preliminary performance results of LP-WUR	CATT

- R1-2302815	Evaluations on LP-WUS	Intel Corporation

- R1-2302827	Discussion on evaluation on LP-WUS	InterDigital, Inc.

- R1-2302861	Evaluation of low power WUS	Sony

- R1-2302890	Low power WUS Evaluation Methodology 	Nokia, Nokia Shanghai Bell

- R1-2302948	Evaluation on LP-WUS	ZTE, Sanechips

- R1-2302968	Evaluation on low power WUS	xiaomi

- R1-2303150	Evaluation on LP-WUS/WUR	Samsung

- R1-2303332	Evaluation on low power WUS	MediaTek Inc.

- R1-2303429	Discussion on evaluation for LP-WUS	LG Electronics

- R1-2303505	On performance evaluation for low power wake-up signal	Apple

- R1-2303537	On LP-WUS evaluation	Nordic Semiconductor ASA

- R1-2303612	Evaluation methodology for LP-WUS	Qualcomm Incorporated

- R1-2303759	Low power WUS evaluations	Ericsson

History

- R1-2210437	FL summary#1 of evaluation on low power WUS	Moderator (vivo), RAN1#110bis

- R1-2210512	FL summary#2 of evaluation on low power WUS	Moderator (vivo) , RAN1#110bis

- R1-2210668	FL summary#3 of evaluation on low power WUS	Moderator (vivo) , RAN1#110bis

- R1-2212768FL summary#1 of evaluation on low power WUS	Moderator (vivo) , RAN1#111

- R1-2212899	FL summary#2 of evaluation on low power WUS	Moderator (vivo) , RAN1#111

- R1-2213005	Final FL summary of evaluation on low power WUS	Moderator (vivo) , RAN1#111

- R1-2302006 FL summary#1 of evaluation on low power WUS	Moderator (vivo) , RAN1#112

- R1-2302140 FL summary#2 of evaluation on low power WUS	Moderator (vivo) , RAN1#112

- R1-2302212 FL summary#3 of evaluation on low power WUS	Moderator (vivo) , RAN1#112

- R1-2302251	FL summary #4 (final) of evaluation on low power WUS Moderator (vivo) , RAN1#112

- R1-2304076	FL summary #1 of evaluation methodologies on LP-WUS/WUR	Moderator (vivo), RAN1#112bis

- R1-2304150	FL summary #2 of evaluation methodologies on LP-WUS/WUR	Moderator (vivo), RAN1#112bis

- R1-2304151	FL summary #1 of evaluation results on LP-WUS/WUR	Moderator (vivo), RAN1#112bis

Annex: Comments in the email discussion

| # | Proposals | Companies comments | FL comments |
| --- | --- | --- | --- |
| 8 | [H] Proposal 1C-2-v3: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For cat1a, clock error option 3 or 4 is assumed, o FFS for cat1b, clock error option 1 or 2 is assumed o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed. - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v4: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver [o 10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o For other values, the mapping between power value and receiver type are FFS] - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v5: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 and /FLL/PLL are not assumed and only RTC is maintained; o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 and whether Y1=Y2 or not - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v6: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types based on envelope detection for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For LP-WUR Off value 0.001, oscillator option 1, 2, 3, 4 and /FLL/PLL are not assumed and only RTC is maintained; Only RTC is running during sleep. Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power. o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 and whether Y1=Y2 or not - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v7: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ Y1/ [Y2], [FFS value of Y1, Y2 e.g., 0.1, 0.01, 0.005]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not specializedused for receiver types based on envelope detection for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For LP-WUR Off value 0.001, oscillator option 1, 2, 3, 4 and /FLL/PLL are not assumed and only RTC is maintained; Only RTC is running during sleep. Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power. o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 and whether Y1=Y2 or not - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- | Intel: We are OK for the proposal relative power 10/20/30 for OFDM receiver. We are also fine to introduce parameter Y. Note4 overlaps with the sub-bullet for Option 3/4’in the second bullet in Proposal 1A-v-v1. It is better to have a single proposal Ericsson: Not OK with below part in the proposal second FFS point. We suggest to remove it. FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver Huawei, HiSilicon: we have a couple comments as following: 1. 0.2 should be added considering we have proposal of sequence based receiver with power of 0.2; 2. We don’t need the sub-bullet of “10/20/30 for LP-WUR ON power state are used for OFDM receiver”. It is anyway to be handled by AI 9.11.2. And we actually have some proposal of OFDM based receiver, i.e. sequence based detection, to be with power consumption smaller than 1. 3. Thanks for adding the note5, but we should still clarify that all the values in the table is the power consumption for LP-WUS monitoring based on the following agreement. Therefore, Note 5 should be updated with the green revisions to “Note5: the power consumption value of LP-WUR ON is for LP-WUS monitoring, and Up to companies to report whether same or different values with justifications are assumed for WUS monitoring and time/frequency synchronization. Agreement - The following power model for LP-WUR/WUS evaluation is considered, o Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: o [0.001] o Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: o [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] o Other values are not precluded to be evaluated. o FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation …….. 4. for the bullet “For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed.”: a) option 1/2 and option 3/4 should be separated listed, considering they have very different assumption of frequency error and drift. E.g. if Y=0.1, it is sure that option 1/2 can be used because 0.1 is the same as power value of ON state, where option 1/2 can be used. B) For the detailed value of Y, if we just have examples in square brackets, we prefer to keep it as FFS . Apple OK in principle. On “10/20/30 for LP-WUR ON power state are used for OFDM receiver”, I assume the intention is to say these values are not to be used for other receiver types. But it may be misunderstood as OFDM receiver can only take these values. Suggestion: “10/20/30 for LP-WUR ON power state are not applicable to the receiver architectures for MC-OOK and MC-FSK.” For the highlighted note on oscillator, is the intention to say that “For WUR Off value 0.001, oscillator is assumed to be not running and only RTC is maintained”? MTK: Agree to add 0.2 considering we have proposed an OOK-OFDMA hybrid receiver with a power of 0.225. Nokia: In principle fine, but noting that, if Proposal 1D-2-v2 is agreed, should be accounted in this. HW&HiSi#2 on Proposal 1C-2-v4 1. On the updated wording regarding the power value of “10/20/30”, the current wording is still not clear enough from our point of view. we would be fine with Apple’s proposal with some revision: “10/20/30 for LP-WUR ON power state are not applicable to the receiver architectures of envelope detection for MC-OOK and MC-FSK.” ZTE/Sanechips We are fine and think 10/20/30 is only applied for OFDM based receiver. It is not true that the OFDM receiver can use other relative power, e.g., 1. If it is possible, please clarify how to achieve this. ZTE, Sanechips: We think 10/20/30 for OFDM based receiver should be kept. If 9.11.2 discuss the relatibe power for each architecture, the agreement in 9.11.1 also can be the reference. Some further clarification is clarified in the draft folder document. MTK2: We wonder why 10/20/30 cannot be used for MC-OOK and MC-FSK. It should be the opposite that “at least 0.01/0.05/0.1 cannot be used for OFDM receivers.” QC: We think it is not good having hard restriction for power numbers and receiver types. We propose to remove as follows. [o 10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, On Note 4, we are fine to add one more values. For simplicity, we propose to add one value with following clarification. 0.001: Only RTC is running during sleep. But, any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power. [0.005]: Option 1,2,3,4 can be assumed (always on LO). Apple2 (on v4) OK in principle. It appears that the sub-bullet in bracket or any proposed modification is still controversial. In this case, we would suggest removing it. We do not think it adds much value at this point. The power consumption will be discussed in 9.11.2 anyway. On note 4, we can be fine with it. But want to confirm if our understanding is correct: for WUR Off value 0.001, oscillator/FLL/PLL is assumed to be not running. So when WUR wakes up, it starts from the maximum frequency offset? Xiaomi: OK in principle. For WUR ON value, we do not think that restrictions are necessary for 10/20/30 and these restrictions can be reported by companies as additional assumptions. HW&HiSi#3 on Proposal 1C-2-v4: @FL2, our previous comments is 10/20/30 is NOT applicable to envelope detection of OOK/FSK. But it seems companies cannot align with the bullet of “10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, For other values, the mapping between power value and receiver type are FFS]”. We prefer to remove it as suggested by Qualcomm and Apple. Agree with Apple that the power consumption mapping shall be discussed anyway in 9.11.2. Ericsson2 (on v5): Prefer QC suggested updates to Note 4. As mentioned earlier, power mapping to different LP-WUR architectures can be done after there more convergence in 9.13.2 agenda item. QC (on V5): As pointed out by Eicsson2, We think hard restriction is not necessary and boundary/mapping between is arbitrary at this stage. Instead, we prefer to have qualitative description/comparison of power consumption values across different receiver types. o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS vivo2: Regarding the values 10/20/30, we think the justification to add them on is that receivers for OFDMA signals/channels have been agreed in the last meeting. We don’t see the any justification of these values for envelop detection for MC-ASK and MC-FSK. We are ok with either alt 1 or alt2, but we don’t agree to add 10/20/30 arbitrarily without solid justification. For the other values, we are ok to discuss further the mapping between power value and receiver type. HW&HiSi#4： In Alt.2, it should be “Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types based on envelope detection for MC-ASK and MC-FSK”, because we can be sure that envelope detection shall not have this high value. But maybe the receiver type with 10/20/30 can have a way to detect MC-ASK and CM-FSK. However, our first preference is remove the purple part considering there are too many misalignment here. it is better to be discussed in detail in AI 9.13.2. Intel: We are OK for the bullet on relative power 10/20/30. On Note4, option 3/4 may be only applicable when both conditions of off power and on power are met. We prefer the following update to align with 1A-4-v3. The first two sub-bullets are fine o For WUR Off value >= Y2, option 3,4 can be assumed when the condition on LR on value if agreed is also met. FFS: Y2 and whether Y1=Y2 or not Apple3 (on v6) We can be fine with purple text, and we are also fine with removing purple text completely, as this discussion is overlapping with the discussion under AI 9.11.2. A clarification question: is it correct that Y value is applicable to any type of receivers, if the receiver intends to keep LO/FLL/PLL running during WUR off? We think the highlighted yellow text related to option 1/2/3/4 should be removed from this proposal because it is related to the oscillator and covered in the next proposal, so that we do not have two places discussing the same issue. Samsung: 1) We prefer to remove purple text as suggested by other companies. We think that there is no sufficient discussion in 9.11.2 yet to provide information for mapping from power values to LR architecture. Proper power value not only for OFDM receiver but MC-OOK/FSK receiver can be provided together after discussion in 9.11.2. Therefore, it is enough to keep the second FFS without purple text. 2) In the table, a column for transition energy can be updated according to the agreement from the last online GTW session. Transition energy→Additional transition energy, and [TLR, ramp-up *(PON+POFF)/2]→[TLR, ramp-up *(PON-POFF)/2]. 3) We also think that we can remove the overlapped information in proposal #8 and #10. Note 4 in proposal #8 and Table (+Note 2) are partially overlapped. The mapping between the oscillator state and power value can be handled in proposal #10, not proposal #8. vivo3: We prefer to keep either Alt 1 or Alt 2 in purple, as it provides justification on why we add 10/20/30 in this meeting. HW’s modification on Alt 2 is ok to us. We fully agree we can further discuss the mapping between the set of power values to LR receiver architecture types as indicated by the last purple bullet, however, what we discuss here is to add new values 10/20/30, and solid justification is needed. If companies hesitate to add alt 1 or alt 2 here for providing justification, we suggest not to add 10/20/30 and leave the discussion to next meeting. MTK3 1. Architecture mapping can be done in AI 9.11.2, where recommendations about relative power and noise figure will be discussed properly. No need to consider any mapping/limitation here. We suggest removing 10/20/30 for LP-WUR ON power state are not specializedused for receiver types based on envelope detection for MC-ASK and MC-FSK, 2. Why Y1/Y2. If more options are needed, it is sufficient to say FFS: other values vivo 4:  For alt 2, we prefer the previous version ‘10/20/30 for LP-WUR ON power state are not used for receiver types based on envelope detection for MC-ASK and MC-FSK’, we think it is good enough to not preclude the case that the receiver type with 10/20/30 can have a way to detect MC-ASK and MC-FSK. Furthermore, when we look at the relative power values for OFDM receiver proposed by companies in this meeting,, 10, 20, 40 are typically proposed, that’s why we add 10/20/30 here. We insist alt 1 or alt 2 is needed to provide the justification for adding 10/20/30.  For Y1, Y2, we share similar view with HW that the 2nd and 3rd bullet in note 4 should be kept to provide illustration, however, for the values of Y1 and Y2, we prefer to discuss them further by considering detailed hardware accuracy and performance requirement. Therefore, we suggest to delete the examples, [FFS value of Y1, Y2 e.g., 0.1, 0.01, 0.005] CATT – Since we don’t have agreement on the power model of OFDM receiver, we should reuse the power model of sequence detection (e.g., CSI-RS with value 100) in Rel-16 TR38.840. Similarly, the power model and transition time for the off state of LP-WUR for OFDM waveform should reuse the deep sleep power model before we agree on the power model of OFDM waveform. Please adding the following values with note  Off – value [1] is used for OFDM receiver  ON – value [100] is used for OFDM receiver Apple4 (on v7) If Alt is to be kept, “specialized” is a confusing term. We would prefer the previous wording, or removing it completely. On vivo’s comments, we do not necessarily need to capture all the justifications in the agreements. We did not provide any justification when agreeing to the earlier set of power values either. The mapping will need to be done anyway, and there is no way around it. Maybe it is a bit unnecessary concern in my view. On the Y1, Y2, originally Y1, Y2 were included in the brackets, but now only Y2 is in the brackets. Maybe a typo here? If we are not providing any description for Y1 and Y2 here, it may be better not to differentiate here, and if needed, details can be added in the next proposal. Suggestion: “0.001/ [Y, FFS value(s) of Y]” QC on v7: We think the evaluation methodology should allow wide range of power numbers which we can freely evaluate regardless of choice of receiver architectures. The range should be wide enough to cover all the receiver types in study and the allowed set of values within the range should be limited to reduce required evaluation workload and allow easy comparison across data points from different companies. The mapping from specific values to certain receiver types are second issue, probably could be done in 9.11.2. We think 9.11.1 should provide evaluation methodology only so that companies can freely evaluate various values, rather than enforcing certain type of mapping. Therefore, we think the purple text are not necessary in this sense. Please remove all purple text other than last FFS. o 10/20/30 for LP-WUR ON power state are used for OFDM receiver o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not specializedused for receiver types based on envelope detection for MC-ASK and MC-FSK, o The mapping between power values and receiver type are FFS in 9.11.2. For Off power, we can simply as follows. Please remove Y1, Y2, etc.. given that Note 4 is now removed. 0.001/ FFS e.g., 0.005, 0.01 | FL1: @Ericsson, Apple, Huawei, clearly the intension of adding 10/20/30 is for OFDM receiver for MC-OOK/FSK. However to move forward, FL add [] for the sub-bullet. And add FFS for other values. @Huawei, (1) added 0.2 (3) no need to say the WUR ON power is only for monitoring, the T/F sync power value can be picked up from these values and reported by companies if they use different values. (4) Make sense. Suggest separate bullet for option 1/2 and 3/4. Please check v4 FL2: @MTK, the value 0.2 is already in V4 @Nokia, OK @Huawei, not sure which is not clear. Are you going to suggest 10/20/30 for OOK/FSK? I don’t see any intension to do so. By adding the setence ‘For other values, the mapping between power value and receiver type are FFS’, for the rest of other values 0.01/0.05/0.1/0.2/0.5/1/2/4, I is still open to discuss the mapping.. if you have any concerns, I think we can consider to remove the []. FL doesn’t change anything in the previous v4. FL3: Again, I would like to note that the intension we discuss to add 10/20/30 is because in the last meeting we agree to study OFDM receiver. It make no sense to add 10/20/30 for any OOK/FSK based on my observation from contributions or practical usage. So as a matter of fact, I do not see any wrong if 10/20/30 is for OFDM receiver. If people really wants to discuss whether 10/20/30 is for OFDM receiver or can be applicable for OOK/FSK for evaluation in 9.11.2, I can remove 10/20/30 now and wait until discussion in 9.11.2 to finalize. As the FL, I would like to move us forward instead of waiting for the next meeting(the last second meeting for SI in RAN1). In V5, I listed two Alternatives for 10/20/30 and let’s see which one is preferred by companies. Please companies be constructive and practical. @Apple2, My understanding is same with you. for WUR Off value 0.001, oscillator/FLL/PLL is assumed to be not running. So when WUR wakes up, it starts from the maximum frequency offset. I updated note4 to address your comment @Qualcomm, I also add FFS for whether Y1=Y2 or not to address your another comment Please see [H] Proposal 1C-2-v5 FL6 Two comments are addressed. 1. Regarding Note 4, For LP-WUR OFF 0.001, modified according to Qualcomm’s suggestion (also preferred by E///) For LP-WUR OFF Y, Qualcomm’s revision (one Y value) and Huawei’s revision (two Y value) are different. And the Y value are quite divergent. So FL suggest to keep the current text which are open for the number of Y and its value . (2)Regarding Alt1/2, based on the suggestion from Xiaolei’s revision on Alt 2. FL provide the compromise proposal V6 Please see [H] Proposal 1C-2-v6 FL7 @Apple3, Yes, Y value is applicable to any type of receivers @Apple3, Huawei, Would it be OK to move the sub-bullets of note4 in proposal#8 to proposal#10. Sub-bullet 1 of note 4 has been addressed by note1 in proposal#10 Sub-bullet 2 and 3, I modified 0.001/ Y1/ [Y2], [FFS value of Y1, Y2 is e.g., 0.1, 0.01, 0.005], and option 3/4 part are moved to proposal #10 note 2 - Note4: o For LP-WUR Off value 0.001, oscillator option 1, 2, 3, 4 and /FLL/PLL are not assumed and only RTC is maintained;Only RTC is running during sleep. Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power. o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 and whether Y1=Y2 or not Based on that, please see FL7 and corresponding updated proposals. I modified the following part Off[1]; 0.001/ Y1/ [Y2], [FFS value of Y1, Y2 is e.g., 0.1, 0.01, 0.005]. @ Samsung, vivo To compromise and proceed, I suggest the Alt 2 with the following changes, hope this is acceptable. I don’t think open another discussion for 10/20/30 specialized for envelope detector is necessary (in the contribution not mentioned at least). The reason we’re here discussing 10/20/30 is because we agree to study OFDM receiver last meeting. Otherwise, we may not need to discuss 10/20/30 at all. 10/20/30 for LP-WUR ON power state are not specializedused for receiver types based on envelope detection for MC-ASK and MC-FSK, Please see Proposal 1C-2-v7 |
| 10 | [H] Proposals 1A-4-v2: Confirm the WA for the followings Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. • Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS • Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4,  FFS applicability when LP-WUR ON power is less than[TBD] power unit  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD] power unit,  Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.  assumptions important for achieving performance by using MR clock for LR should be declared  Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).  Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] <image002.png> - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr  Model 2: random time drifting, FFS details - FFS: Phase noise model [H] Proposal 1A-4-v3 Confirm the WA for the followings and update as follows Working Assumption • For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm], RTC frequency drift [ppm/s]; (20 , FFS:[0.1]). • Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS • Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4,  FFS applicability when LP-WUR ON power is less than[TBD] power unit  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD] power unit,  Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.  assumptions important for achieving performance by using MR clock for LR should be declared  Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option • Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1: • The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 • When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error • T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. • FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).  Model 2: random frequency drifting, FFS details • Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1 [R1-2301438] [R1-2301558][R1-1714993]: • The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) • The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) • The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] <image003.png> • T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach • FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr  Model 2: random time drifting, FFS details • FFS: Phase noise model • The following usage of the clock is assumed for LP-WUR OFF/ON LP-WUR OFF power; Assumptions. 0.001; When LP-WUR OFF - Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR switches to ON - Frequency offset calculation follows option 1/2 and option 3/4[Note2](first value in the value pair).. Y - value(s) are TBD; For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated by option 1/2 and option 3/4[Note2]. RTC can be used too. - Frequency offset calculation follows option 1/2 and option 3/4[Note2](cumulating the second value in the value pair until reaching maximum).. Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=[TBD2] Note3. The clock error (of both RTC and LO) could be improved beyond less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble. [H] Proposal 1A-4-v4 Confirm the WA for the followings and update as follows Working Assumption • For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm], RTC frequency drift [ppm/s]; (20 , FFS:[0.1]). • Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS • Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4,  FFS applicability when LP-WUR ON power is less than[TBD] power unit  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD] power unit,  Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.  assumptions important for achieving performance by using MR clock for LR should be declared  Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option • Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1: • The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 • When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error • T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. • FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).  Model 2: random frequency drifting, FFS details • Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1 [R1-2301438] [R1-2301558][R1-1714993]: • The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) • The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) • The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] <image003.png> • T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach • FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr  Model 2: random time drifting, FFS details • FFS: Phase noise model • Working assumption: The following is example for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameter of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When at the time point LP-WUR switches from OFF to ON - Frequency offset calculation can follow the parameter of the oscillator option 1/2 and option 3/4[Note2] (first value in the value pair). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration When LP-WUR is ON: - Frequency offset and time offset calculation can follow the parameter of the oscillator option 1/2 and option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). Y - value(s) are TBD >0.001; For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 and or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 and or option 3/4[Note2] (cumulating based on the second value in the value pair until reaching maximum and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power [Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2] Note3. The clock error (of both RTC and LO) could be improved beyond less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble. | Intel: the sub-bullet ‘for Option 3/4’in the second bullet has overlap with Note4 in Proposal 1C-2-v3. It is better to have a single proposal Ericsson: Following part should be deleted if confirming the WA given inputs provided to this meeting regarding clock power consumption. i.e., as follows - For Option 3/4,  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD]power unit, MTK: Using Oscillator error to model CFO is not enough. To maintain stable frequency output, at least FLL/PLL cannot be turned off, which is power-consuming. Suggest adding a note below. option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies Note. At least FLL/PLL cannot be turned off for LR to support options 1/2/3/4. Nokia: A question for clarification, that for RTC case we should assume always random offset at 20ppm range upon LR wake-up? Samsung: According to the proposal #8, some options for oscillator cannot be assumed for WUR off-state. In our understanding, it means that oscillator is turned off and frequency drift of oscillator cannot be used for calculation of time/frequency error such as Model 1 in the proposal #10. Therefore, we suggest to add Note for clarification, “Note: assumption for frequency drift cannot be used when the oscillator is assumed to be turned off” QC: We are find removing red FFS related to option 3 and 4. Apple: we are getting a bit confused on how it works. Assuming the LR OFF power of 0.001, do we assume oscillator is still running, just FLL/PLL are turned off? I think everything should be turned off, except for the RTC. In this case, what happens when the LR is turned on again? Maximum frequency offset should be assumed? Then the drifting model does not apply any more. QC: We think t/f drifting model will apply to RTC clock. Depending on design, LO could be always on or shutdown during sleep duration. The FLL/PLL could start when LP-WUR wakes up to ON duration. The power of LO/PLL/FLL should be already included in LP-WUR On power. Intel: considering 1C-2 and 1A-4 jointly, we think it ends up behaviors of 4 combinations in the following table. We add our current understanding in the table now. On power < [TBD]; On power >= [TBD]. Off power 0.001;  Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used  Frequency offset when LP-WUS switches to on follows option 1/2 (first value in the value pair). FFS option 3/4;  Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used  Frequency offset when LP-WUS switches to on follows option 1/2/3/4 (first value in the value pair).. Off power Y (e.g., Y>=0.1 or 0.01);  Time offset cumulated in the off period can be calculated by option 1/2. RTC can be used too [*1]. FFS option 3/4.  Frequency offset when LP-WUS switches to on follows option 1/2 (cumulating the second value in the value pair). FFS option 3/4;  Time offset cumulated in the off period can be calculated by option 1/2/3/4. RTC can be used too. [*2]  Frequency offset when LP-WUS switches to on follows option 1/2/3/4 (cumulating the second value in the value pair).. *1: since RTC is better, so RTC can be used by UE *2: up to UE to select a better way, the accuracy is option 3/4 < RTC < option 1/2 Ericsson2: “FFS applicability when LP-WUR ON power is less than[TBD] power unit” and the other note on FLL/PLL are not needed. The architecture agenda item already covers clock accuracy vs. power consumption study. QC : We think Note 1 is not sufficient. We suggest to remove Note 1 or add additional Note 2 and 3 w/ following modification. [ Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain always on for when monitoring receiving LP-WUS. (Always on monitoring) Note2. FLL/PLL could be turned on only when LP-WUR is on for LP-WUS monitoring (duty cycled monitorig). Note3. The clock error (of both RTC and LO) could be improved beyond max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.] For completeness of model, we suggest to add drift of 0.1(ppm/sec) for RTC RTC max frequency error [ppm]: (20, 0.1) Samsung2: We have a similar question with Apple. We want to know and clarify how to calculate time/frequency error model for duty-cycle based LP-WUS monitoring when oscillator/FLL/PLL can be turned off during LR off-state. Apple2: QC’s proposal on adding drift for RTC seems to make sense, to cover the cases where the LO is off. But I am still not sure that I completely understand the model. Intel’s table seems to be a good starting point for clarification. Our preference would be not to confirm the WA before understanding these details. Intel: Thanks moderator adding the new last bullet. We are generally fine with it. some clarification questions - Not sure to how to understand ‘additional’ LO/FLL/PLL. Is it for the following two cases, 1) LO/FLL/PLL is off in LR off state and 2) a different/better LO/FLL/PLL is switched on in LR on state? - Is there a case for LR that LO is still on, but FLL/PLL is off? Not sure if we miss some part of Note2 in QC’s reply. If there is such case, can we assume same value pair (frequency error, frequency drift) in option 1/2/3/4 for 1) LO is on + FLL/PLL is off, and 2) LO/FLL/PLL are all on ？ Apple3: (on v3) I wonder if time offset should always follow the RTC, even if the oscillator/FLL/PLL is running? I thought RTC is used to control the timing, and the oscillator control the frequency. Of course RTC can also have timing correction after receiving sync signal, and then it drifts until receiving the next sync signal. HW&HiSi#5 on Proposal 1A-4-v3： 1. We think the following condition of “When LP-WUR switches to ON” is not clear. Only at the time point when LP-WUR switches from off to on, the frequency error is based on the first value in the value pair. When LP-WUR is on after synchronization with either LS-SS or SSB or assist from the MR, the frequency error is the Fr cited in the agreed formula “Fd (ppm)=ΔF (ppm) +Fr(ppm)”, which is the residual frequency error. When LP-WUR switches to ON Frequency offset calculation follows option 1/2 and option 3/4[Note2] (first value in the value pair).. 2. If we want to list all cases, we should also list the case when LP-WUR is on. 3. The claim of “(cumulating the second value in the value pair until reaching maximum)” in the second row of the table is not accurate. LP-WUR can use LP-SS or SSB to synchronize and not necessarily reaching maximum frequency offset. 4. I added also time error in some place of the table. 5. Why we restrict the sentence of “For both LP-WUR OFF and ON, Time offset cumulated in the off period can be calculated by option 1/2 and option 3/4[Note2]. RTC can be used too.” to off period only, the timing error in ON state should be also taken into account considering the frequency is not accurate and therefore the sampling is not accurate. 6. Regarding Note2: “Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=[TBD2]”, is there some typo here? There are two values compared with one TBD2 value. 7. There are Y1/Y2 in proposal 8, so the table should be also updated accordingly, e.g. change “option 1/2 and option 3/4” or “option 1/2 or option 3/4” depending on the value of Y. I am not sure whether it is good idea to try to list all the cases within this short time email discussion, but I tried to make some revisions as below: • The following usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When at the time point LP-WUR switches from OFF to ON - Frequency offset calculation follows option 1/2 and option 3/4[Note2](first value in the value pair). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration When LP-WUR is ON: - Frequency offset and time offset calculation follow option 1/2 and option 3/4[Note2] (cumulating based on the second value in the value pair). Y1 or Y2 - value(s) are TBD; For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated by option 1/2 and or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation follows option 1/2 and or option 3/4[Note2] (cumulating based on the second value in the value pair until reaching maximum). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. Samsung: 1) We think that the last table for the usage of the clock can be assumed as one example for LP-WUR OFF/ON given various implementations are possible. So, we suggest the following revision, “As one example, Tthe following usage of the clock can be isassumed for LP-WUR OFF/ON“. Also, we want to suggest some wording change in the table as follows: LP-WUR OFF power; Assumptions. 0.001; When LP-WUR OFF - Time offset cumulated in the off period cannot be calculated bybased on the parameter of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR switches to ON - Frequency offset calculation can follows the parameter of the oscillator option 1/2 and option 3/4[Note2] (first value in the value pair). (cumulating frequency drift the second value in the value pairuntil reaching maximum frequency error).. Y - value(s) are TBD; For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator by option 1/2 and option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follows the parameter of the oscillator option 1/2 and option 3/4[Note2] (cumulating frequency drift the second value in the value pair until reaching maximumfrequency error).. Otherwise, we are not sure whether Note1/3 are necessary. 2) We are not sure that we can align the understanding on the clock error modeling for oscillator off duration within the limited discussion time. So we prefer to keep it as working assumption and continue discussion based on the given table in the next meeting if it is hard to align company’s view. MTK2: For the case of LP-WUR OFF power > 0.001, we cannot justify clock assumption given a relative power range. For example, what the assumption should be for LP-WUR OFF power = 0.0011. we suggest considering the case of 0.001 only. vivo 4: regarding the LP-WUR OFF power > 0.001, we think a rough range may not help, we prefer to change back to ‘value(s) are TBD’ and discuss the values later as in proposal 8. CATT: The frequency drift is derived from frequency stability and clock rate, which is at least the sampling rate as follows, <image006.png>. We don’t think that we need to have the frequency drift value in the evaluation assumption when we have defined the frequency stability in ppm. We need to define the clock frequency, e.g., 4 GHz, instead. Apple4 (on v4): We are fine with the overall direction, but prefer to keep the entire proposal as the WA instead of just the example, as companies are still exploring how to evaluate. Also, it seems rather strange to have a WA on an example. We also wonder if the description can be improved a bit more for clarify (see below for some suggestions). What seems to be missing is the initial assumption for time offset when WUR switches on for OFF power of 0.001. Does it follow RTC or use the max freq error of the oscillator? If it follows the max freq, it can be quite large. But it seems also strange to use RTC if the subsequent tracking is done based on the oscillator. We think the notation of Y is still needed to connect it to the previous proposal. We wonder if Note 1 is for the first case with OFF power of 0.001, where LO/FLL/PLL is assumed to be not running in OFF state. Should clarify. We also suggested some changes to Note2 to capture the discussion related to Y1 and Y2 in the previous proposal. Maybe Note3 is no longer needed? Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON - The initial frequency offset when LP-WUR switches on can be set to the maximum frequency error following the parameters of the oscillator option 1/2/3/4[Note2]. - During the ON duration, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 (cumulating based on the frequency drift and not exceed maximum frequency error) - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. | FL1: @Intel, @Ericsson, removing ‘for option 3/4…’ But the last sub-bullet is related to whether Oscillator option 3/4 is applicable for WUR ON power. It is not covered in Proposal 1C-2. FL suggest to keep it FL2 @MTK, the added note is not clear since some ED detector doesn’t need to have PLL/FLL? But I understand your intension, perhaps we can modified a bit saying Note. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS @Samsung, I think what you proposed is relavant to the [H] Proposal 1C-2-v4. For example, if the WUR OFF is 0.001, the frequency drifting is not assumed to be used for calculating time/frequency error. We can solve this in that proposal or later. Since the value Y in H] Proposal 1C-2-v4is not yet done. Now changed to [H] Proposals 1A-4-v2 FL4 Thanks Intel for the summary, As suggested also by Apple, maybe we need more clarification. Therefore I suggest to take it for further discussion, e.g., - start from Intel’s table for further clarification. - Also discussing Qualcomm’s RTC assumption for adding drifting 0.1ppm/s FL6 @Qualcomm, revised RTC (20,FFS:0.1) according to Qualcomm’s suggestion. Companies please consider and provide comments. @Intel, Thanks Intel for the clarification. Basically it’s true proposal1C-2 and 1A-4 can be discussed jointly. I provide a version including note4 in proposal1C-2 if companies are OK to discuss here. Also I added some clarification. - Note 1 and 3 is suggested from Qualcomm. - Note 2 is following the principle as in Proposal#8(Note4), i.e., option 3/4 is assumed when power is larger than a value Please see [H] Proposal 1A-4-v3 FL7 @Apple3, my understanding is that time offset is not follow the RTC if the oscillator/FLL/PLL is running. When WUR OFF is Y, Time offset cumulated in the off period can be calculated by option 1/2 and option 3/4[Note2]. RTC can be used too. @Huawei3, 1. I agree. the reason I didn’t capture the related cases for residual frequency is because we have agree that in another agreement. But I would be fine to your revision if no one against. 2. OK to add 3. The original intension ‘until reaching maximum’ is to be assistant to ‘When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error’ 4. Ok 5. OK 6. Modified according to your comment (6) and (7) @Samsung, Fine to make it example and WA Regarding the comments to Note2, my observation is that it is related to the following assumptions, - Assumption 1: 5ppm/10ppm(option3/4) and 50ppm/200pm(option1/2) has same minimum required power - Assumption 2：5ppm/10ppm(option3/4) and 50ppm/200pm(option1/2) has different minimum required power Different companies may have different opinions so far as I observed. Maybe we can keep note2 in [] now and further come back in the next meeting. @All, Merged the change from Huawei and Samsung, Please see [H] Proposal 1A-4-v4 |

| # | Proposals | Companies comments | FL comments |
| --- | --- | --- | --- |
| 1 | Proposal 1D-1-v1: Update as followings for the e-DRX paging probability Note: - For i-DRX with cycle duration Y second, o Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second o Per UE paging probability is  RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW  RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW o L=4 |  | FL: This proposal was sent in the email last Friday. |
| 2 | [M] Proposal 1D-2-v2: Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model. - Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 |  | FL: This one is sent in the email last Friday. |
| 3 | [M] Proposals 2C-v3(after Friday offline): RAN1 further consider the design of strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Option 1: PDCCH for broadcast - Option 2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options - The final design will jointly consider the coverage target with other KPI [M] Proposals 2C-v4: RAN1 further study the designs/techniques consider the design of LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Coverage option #1: PDCCH for broadcast paging - Coverage option #2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options - The final design will jointly consider the coverage target with other KPIs [M] Proposals 2C-v5: RAN1 further study the designs/techniques of LP-WUS to have a similar comparable coverage as NR channel X. The NR channel X is - Coverage option #1: PDCCH for paging - Coverage option #2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs [M] Proposals 2C-v6: RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Coverage option #1: PDCCH for paging - Coverage option #2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels [M] Proposals 2C-v7: RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Option #1: PDCCH for paging - Option #2: PUSCH, o FFS PUSCH for data with data rate defined in the coverage SI or PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels, e.g., the date rate for PUSCH, the message size for MSG3 and etc. | Ericsson: Not OK. LP-WUS impacts paging PDCCH and so RAN1 should consider designs that can match paging PDCCH coverage. We do not follow why LP-WUS designs that aim to have significantly worse coverage than PDCCH for paging should be considered. If companies want to study trade-offs, we are OK with below formulation. RAN1 to further consider study the design ofstrives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Option 1: PDCCH for paging broadcast - Option 2: PUSCH for msg3, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options - The final design will jointly consider the coverage target with other KPIs Huawei, HiSilicon: as we commented during the offline, two targets could be even considered as compromise to move forward, i.e. the study item identifies techniques of LP-WUS to have a similar coverage with the two targets. Some revisions are suggested in green: RAN1 further consider the design/techniques of strives to design LP-WUS to have a similar coverage as NR channel X. The NR channel X is - Option Target#1: PDCCH for broadcast - Option Target#2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options The final design will jointly consider the coverage target with other KPI MTK: Based on companies’ results, at least one NR channel’s coverage can be achieved. We wonder whether RAN1 can confirm at least PUSCH (FFS: msg3) can be achieved. HW&HiSi#2 on Proposals 2C-v4 We noticed that all “targets” are removed from the proposal. So the proposal now means just two coverage options that may or may not be achieved? And there is no coverage target? We think it would be better to have the coverage target and at least we share the same view with MTK that the PUSCH coverage target should be able to be achieved. To further achieve the coverage target of PDCCH, the SI should identify the potential techniques so that the group can evaluate the tradeoff between the coverage and the cost/complexity/resource etc. - Coverage target option#1: PDCCH for broadcast paging - Coverage target option#2: PUSCH, Spreadtrum: Support to keep “target”. Support to have two options. We do care about system overhead and NES, but also care about the application/commercialization of LP-WUR (which means LP-WUS cannot have too much limitation on use cases, otherwise it will not be deployed in realistic)… ZTE, Sanechips: After the UE receive the LP-WUS, the UE need to wake up to receive the PDCCH and start the RACH procedure. If the LP-WUS has similar performance with PUSCH and can complete the RACH procedure perfectly, we do not think the LP-WUS coverage should be matched with PDCCH. Additionally, we have some suggestion and modification for the proposal, which is uploaded in the draft folder. ZTE, Sanechips: We would like to suggest to change ‘similar’ as ‘comparable’, since if we use ‘similar’, then we should have a similar coverage performance with one of the NR channel, instead of a medium coverage performance between two NR channels. Additionally, a FFS is added to address CATT’s concern. RAN1 further consider the design of strives to design LP-WUS to have asimilar comparable coverage as NR channel X. The NR channel X is - Option 1: PDCCH for broadcast - Option 2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - Option3: Option1+Option2 (better than PUSCH, worse than PDCCH) - FFS other options - The final design will jointly consider the coverage target with other KPI - FFS the details assumption for NR channels MTK2: It can be beneficial to RAN2 if RAN1 can confirm at least PUSCH can be achieved and PDCCH needs further study. [M] Proposals 2C-v4: At least PUSCH (FFS: msg3) can be achieved, RAN1 further studies the designs/techniques and consider the design of LP-WUS to have a similar coverage as NR PDCCH for paging. - FFS other options - The final design will jointly consider the coverage target with other KPIs QC: We think it would be good to have “target” coverage in terms of NR channel X. We are ok to support option 3 considering both aspects of coverage and overhead. The to be designed LP-WUS is better to support multiple data rates to support the coverage up to PDCCH and potentially higher rate matching PUSCH. Xiaomi: We are fine to add option3. From our point of view, there are three potential target performances of LP WUS coverage, that is, 1) similar to PDCCH, 2) similar to PUSCH; 3) Better than PUSCH and worse than PDCCH. From the current discussion, it is difficult for us to achieve the performance of LP WUS similar to PDCCH. We suggest to prioritize option 2 and option 3. Of course, it is necessary to keep option1 at this stage. CATT : Even the assumption on NR channels were discussed in last meeting, no related agreement was achieved. We prefer ZTE’s version with adding a FFS i.e. the details assumption for NR channels. HW&HiSi#3 on Proposals 2C-v5: We see some companies also think the target should be kept similarly as us. We would like to echo this again. Again, as a study item work, we can identify the mechanism needed to fulfill the two difference targets. We can further trade off and design what would be specified. Ericsson2: For Option 2, we are not OK with “FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3”. RAN1#110b-e already agreed that companies can compare LP-WUS with at least PDCCH for paging, PUSCH. Unless current discussion narrows down evaluation scope, we do not see much value of the proposal over previous agreement. Option 2, should be narrowed to PUSCH for msg3. On discussion related to ‘target’, LP-WUS tells UE whether/not to receive paging PDCCH. We do not support to include text suggesting that such LP-WUS design is targeted to have at least 15+dB worse coverage than PDCCH. Samsung2: We think that FFS for option #2 can be included in FFS for the last bullet (additional detail assumptions for NR channels), so we prefer to remove "FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3". If so, we are ok with proposals 2C-v6 Apple (on 2C-v6): on the newly added FFS, we do not think it is necessary, as we have the assumptions agreed already. The only thing that is missing is the FFS under option 2. If indeed the last FFS is to be added, we would like to understand what we should agree on further. vivo2：We share similar view with companies to keep target in the main bullet. Regarding the data rate of PUSCH and the message size of msg3, to move forward, we are ok to discuss them further. @Ericsson, the additional value by this proposal compared to previous agreement is 1) We are now discussing which existing channel shall be the coverage target for LP-WUS, not just allow company to compare with as in previous agreement 2) we now include MSG3 in PUSCH case. Thus, we suggest the following modification below, highlight in yellow: [M] Proposals 2C-v6: RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Coverage option #1: PDCCH for paging - Coverage option #2: PUSCH, o FFS PUSCH with data rate defined in the coverage SI or PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels, including the date rate for PUSCH and the message size for MSG3 CATT(on 2C-v6): The mentioned assumption on PUSCH and PDCCH were agreed for coverage evaluation, what we discussed here is about the coverage target of LP WUS. As we commented in the server (Xiaodong may miss it), the coverage of PDCCH or PUSCH in TR38.830 for coverage enhancement includes several values for the target coverage in Tables 5.1.1.3-1 to 5.1.1.3-6. For example, in Tables 5.1.1.3-1, the gNB Tx power is 33 dBm/MHz, and the maximum coupling loss (MCL) of PUSCH has values for eMBB and VoIP with different TDD configurations range from 137.58 to 143.79. Thus, we may need to agree on a single number of MCL for PDCCH and PUSCH for the evaluation. | FL: Copy/paste the proposal after Friday offline discussion : PDCCH: Spreadtrum, Nokia(?), Samsung, Qualcomm(?) PUSCH: Futurewei, Huawei/ HiSilicon (msg3 or data), ZTE/ Sanechips, Xiaomi, Intel(msg3 or data), LGE(?) FL1: I am try to merge Ericsson and Huawei’s comments together. It may not be perfect but to move forward as a compromise. FL2: @MTK, this can be confirmed after the results being discussed. @Huawei, Spreadtrum, keeping ‘target’ may not move us forward after reviewing the comments from other companies. Hopeful this is compromise way to move us forward. @ZTE, - changes ‘similar’ to ‘comparable’ - For your suggested option3, it is not clear criteria for determine the exact value. Considering there is a FFS, I suggest to add it as example for FFS. People can come back next meeting. - Assumption for NR channels had been discussed last meeting, No need to repeat here. Please see V5 FL3: @Qualcomm, Huawei2, considering supporting companies, I modified the proposal in V6 and add ‘target’ in [] in the main bullet. We can further discuss online for this controversial part. @CATT, ZTE, In RAN1#112, we agree the followings for PDCCH and PUSCH. That’s the reason I think no need to add ‘ FFS the details assumption for NR channels’. I can put it the last bullet in V6 to see if companies have strong opinions and objections. Reference data rates for MReMBB; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional). Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Please see [M] Proposals 2C-v6 FL4: @vivo, if removing ‘PUSCH with data rate defined in the coverage SI‘, then the detailed assumptions for PUSCH will be various a lot. In that sense I agree with you that additional FFS is needed. Perhaps it’s better to rephrase as follows for better understanding, - option #2: PUSCH, o FFS PUSCH for data with data rate defined in the coverage SI or PUSCH for message3 @Ericsson, with the modification from vivo, I think we can further discuss the assumption for PUSCH, 15dB worse may not be always true. @Samsung, removing all the FFS in option 2 seems to be less informative. Reading the contributions from companies, most companies are considering the PUSCH for data and msg3. Therefore I kindly ask you to consider keep it. @CATT, yes you’re true. You mentioned TR38.830, I copy/paste some MIL results as follows for your reference. We can discuss the details Urban 4GHz; Representative value; Reference. PUSCH for eMBB DDDSU; 139.01; Table 5.1.1.1-2 of TR38.830. PUSCH for eMBB DDDSUDDSUU; 140.33. Broadcast PDCCH(PDCCH of Msg.2); 150.19. Urban 2.6GHz; Representative value; Reference. PUSCH for eMBB DDDDDDDSUU; 139.66; Table 5.1.1.2-2 of TR38.830. Broadcast PDCCH(PDCCH of Msg.2); 151.62. Rural 700MHz; Representative value; Reference. PUSCH for eMBB UUUUU; 144.76; Table 5.1.1.6-2 of TR38.830. Broadcast PDCCH(PDCCH of Msg.2) 146.15; 157.16. Please see [M] Proposals 2C-v7 |
| 4 | [H] Proposal 1C-4-v4(after Friday offline): The period of low power synchronization signal that LP-WUR used for at least power evaluation can be  Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR  For evaluations assuming LP-SS o { 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms 400ms, 1.28s, 10s(at least for frequency tracking)} o Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the low power synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. [H] Proposal 1C-4-v5 The period of synchronization signal that LP-WUR used for at least power evaluation can be  Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR  For evaluations assuming LP-SS o {[160ms],320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} o Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. [H] Proposal 1C-4-v6 The period of synchronization signal that LP-WUR used for at least power evaluation can be  Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR  For evaluations assuming LP-SS o {[160ms if sufficient necessity], 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} o Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. [H] Proposal 1C-4-v7 The period of synchronization signal that LP-WUR used for at least power evaluation can be  Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR  For evaluations assuming LP-SS o {[160ms if sufficient necessity is justified], 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} o Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy  Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. | CATT: The first bullet in the proposal implies that LP-WUR can receive SSB and use it for synchronization. If the LP-WUR could receive SSB, why is the LP-SS used for? The second bullet assumes LP-SS with long periodicity larger than the time interval 160 ms for UE maintaining in sync to the network. Thus, the out-of-sync LP-WUR would take a long time at each LP-SS cycle to achieve the synchronization using LP-SS. The number of slots of the LP-SS transmission at each cycle to achieve T/F synchronization should be justified. We would suggest including 160ms periodicity for the LP-SS in the evaluation assumption. MTK: agree to add 160ms to support RAN4 measurement requirements, if any. HW&HiSi#2 on Proposal 1C-4-v5 According to our evaluation, 320ms/400ms should be enough. However, to keep 160ms in square brackets is acceptable to us. vivo: We don’t see the necessity to include 160ms, which is smaller than i-DRX cycle, it is unclear for us how this brings additional benefit for sync and measurement accuracy, however, shorter periodicity will cause larger burden to the network including the network power consumption and network overhead. Considering the benefit is not clear and to minimize network impact as much as possible, we suggest add FFS on 160ms. @CATT, when you talk about the time interval 160 ms for UE maintaining in sync to the network, would you please elaborate why 160ms is required for UE maintaining in sync, from our understanding, 320ms is adequate for that. @MTK, we would like to understand which RAN4 measurement requirement do you refer to. ZTE: We are fine to include 160ms as CATT suggested, and our motivation also has been stated in previous round. MTK2: @HW, 160ms is used as a condition to determine AGC setting and time/frequency tracking for FR1 known/unknown cell. But it is for the MR rather than LR. So, it can be relevant only if we consider reusing the MR requirements to LR. However, there is no harm to provide options for power evaluation. Xiaomi: We are fine with Proposal 1C-4-v6. CATT: We can accept Proposal 1C-4-v5 for progress. HW&HiSi#4 on Proposal 1C-4-v6: We are general fine, but according to the FL’s comment, it seems FL missed “is justified” in the proposal. We should add them otherwise the sentence is not complete. | FL: Copy/paste the proposal after Friday offline discussion FL1: @CATT, the listed values here for LP-SS are no less than the minimum i-DRX cycle (i.e., 1 - N times than i-DRX cycle). For 160ms (ZTE also commented), it is dense than the minimum i-DRX cycle which is quite different. But to proceed, would it be OK to add [160ms]? FL2: It seems a bit controversial on 160ms. FL suggest to keep it [] and adding ‘if sufficient necessity is justified’ to address companies’ concerns. Please see [H] Proposal 1C-4-v6 FL3 @Huawei, adding the missing part and revised in v7 |
| 5 | [H] Proposals 1A-1-v3: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T.  N is the number of attempts within T.  where T is {1.28s, 2.56, …}. Other values are not precluded for evaluation.  Company to report (FAR, T, N) o Note: FAR = {0.1%, 1%} as agreed in RAN1#112 - Note 1: For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - Note 2: If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. - Note 3: Number of attempts per second (<image001.png>) can be calculated from T and N, i.e., <image002.png>. Power saving evaluations that companies provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design [H] Proposals 1A-1-v4: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T. · where T is {1.28s, 2.56, …}. Other values are not precluded for evaluation. · Company to report (FAR target, T, N) - For example, if UE performs multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - If UE performs N non-overlap attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. Power saving evaluations that companies provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design [H] Proposals 1A-1-v5: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T. · where T is {1.28s, 2.56, …}. Other values are not precluded for evaluation. · Company to report (FAR target, T, N) - For example, if UE performs multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - If UE performs N non-overlap attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. Power saving evaluations that cCompanies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design [H] Proposals 1A-1-v6: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple WUS attempts/trials, - UE have N attempts within T, · Company to report (FAR target, T, N) - For example, · if UE performs multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. · if UE performs decoding (including e.g., CRC check) in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design [H] Proposals 1A-1-v7: Working assumption: For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, · Company to report (FAR target, T, N) - For example, · if UE performs make a single decision (e.g., comparison with threshold) based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. · if UE performs decoding (including e.g., CRC check) in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design | Ericsson: 1) Update to “Company to report (FARtarget, T, N)” to align terminology with first bullet. 2) It is not clear how following is possible when UE does not have sync – “UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T”.Suggest updating first bullet as “UE have N attempts within T, where N is the number of LP-WUS transmission occasions with in T” and also the third bullet “If UE performsmultiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent” CATT Firstly, we thank the update on the first sub-bullet. But it still confuses us. From the current wording, it means that the T can be any values e.g. 1.28second, 2.56 seconds, 1millisecond, or 1second, right? Thus, We suggest that the sub-bullet can be removed and do not give any restricton T values if we want consider T. We think companies report the values of FAR, T and N is enough. Again, we want clarify that the definition of T for FAR determination is not needed technically. From probability theory, FAR is the ratio of error event and the total event. It is not related with any time definition Apple We are fine with the principle. However, we still think the following is confusing: “For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.” This does not seem necessary, as long as the FAR over the time duration T is reported. Does it make any difference if the UE considers it as a single attempts or multiple attempts? For the bullet “If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent.”: the revision is unclear to us. The original version seems to make more sense, to assume they are independent if they are non-overlapping. But if there is significant overlap between two attempts, it may not be appropriate to assume they are independent any more. This is especially true for the case with continuous monitoring. The question is: do we really need these two bullets? Can we simply let companies report all the details for now, e.g., including how the FAR for the duration T is derived? Nokia: Maybe reword the last note: Power saving evaluations that cCompanies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design ZTE, Sanechips: we have some suggestion to address apple's concern, please see the draft folder document. ZTE/Sanechips: Fine. Maybe the following can be modified to address Apple’s second concern. If UE performs multiple non-overlapping N attempts for the N occasions within the reference time duration, the false alarm event for the attempts are assumed as independent. For the first one, it can be deleted and up to company report. Samsung: To Apple, we would like to keep the second bullet “For example, if UE performs multiple correlations for a sequence part for potential LP-WUS transmission in that in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.” We think that the definition of attempt is not clear if this bullet is removed. For example, when UE try to detect LP-WUS by sliding over time domain with the finer granularity (e.g., sample-level), it can be also multiple attempts within single potential LP-WUS transmission. However, these attempts are highly dependent and it can cause multiple wake-up. The intention is to avoid multiple wake-up at the single potential LP-WUS reception by treating it as ONE trial/attempt. If multiple attempts are independent from enough sliding granularity, then it can be considered as multiple attempts. Apple2 (on v5) We would like to clarify what is considered as a monitoring occasion? Is it correct that this concept only exists for duty cycle monitoring, not continuous monitoring? So the “for example” sub-bullet is intended for the duty cycle monitoring? I wonder if companies intend to use link level sims to evaluate FAR, without doing any post-processing, the last two sub-bullets should not matter, because all the effect is captured in the sims already. Are these two sub-bullets intended for the case where companies may derive the FAR based on the single-attempt LLS result? Sorry for the questions, but I just feel it is overall very confusing. Maybe we can also add “company to report (FAR target, T, N, definition of an attempt)”. QC: In example, only sequence based detection method is captured. It would be better to capture decoding based detection as well. - For example, if UE performs make a single decision (e.g., comparison with threshold) based on multiple correlations for a sequence in thea monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - For example, if UE performs decoding (including e.g., CRC check) in a monitor occasion, a single decoding is considered as ONE trial/attempt. CATT: We thank FL’s update, the Proposals 1A-1-v5 is OK for us. HW&HiSi on v5 For apple’s question on “duty cycle” and “continuous monitoring”, we think this does not preclude anything about continuously monitoring or duty cycle monitoring. If UE monitors every monitoring occasions with T, it is continuous monitoring. V5 proposal looks OK for us Samsung2: We think that QC's modification is better for understanding, " For example, if UE performs make a single decision (e.g., comparison with threshold) based on multiple correlations for a sequence in thea monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt.." With the update, we are fine with the latest proposal. Related to Apple2 comment, we have same view on HW's comments. Regardless of the monitoring schemes for LP-WUS, this sentence comes from the following issue (provided by FL summary): • The output of Multiple correlations/hypothesis may be highly correlated due to oversampling of OOK chips. Hence, it is difficult to derive per correlation FAR from per attempt FAR. [R1-2302948, ZTE], [R1-2303150, Samsung], [R1-2302526, vivo] • Note: If UE performs multiple correlations for detection the potential LP-WUS transmission in that transmission occasion, it can be considered as UE implementation in ONE trial/attempt, rather than multiple attempts/trials. Apple3 (on v6) Thanks for the response on our questions. But some issues are still not very clear to me. E.g. the definition of a monitoring occasion, whether it is a concept only for duty cycle monitoring. If it is continuous monitoring, the monitoring occasion really does not matter for the UE. The definition of an attempt for continuous monitoring is not clear either. However, we do not want to block the proposal. Given the good discussion so far, we are fine to take it as working assumption. As companies start to report next meeting, we will know whether this is sufficient, or maybe some aspects need further clarification. | FL1: make the following changes 1. Update to “Company to report (FARtarget, T, N)” @Ericsson 2. To address Apple’s comment, adding non-overlap back,“If UE performsmultiple non-overlapping N non-overlap attempts for the N occasions within the reference time duration” 3. Making change according to Ericsson’s 2nd comment @CATT, removing restriction of T value FL2: @Nokia, Modified according to suggestion @ZTE, ‘non-overlapping’ is added Please see [H] Proposals 1A-1-v5 FL3 @Apple2, I clarified the attempt in a Figure in the FL summary. Basically, the examples provides an explanation of the attempt. And what Qualcomm proposed to add is regarded as the explanation for decoding based detection @Qualcomm, added what you proposed as another example. Please see [H] Proposals 1A-1-v6 FL4 @Samsung, Qualcomm, Sorry to miss the revision by Qualcomm for the first example. I revised in v7. @Apple3, make it WA Please see [H] Proposals 1A-1-v7: |
| 6 | [H] Proposal 1C-1-v2: Confirm Alt 2 in the following agreement and update as follows Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases. [H] Proposal 1C-1-v3: Confirm Alt 2 in the following agreement and update as follows Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: (40000, 800ms) Company to report which alternative they use for which use cases. | Apple We prefer not to have Alt 1 as the baseline, but otherwise fine. Many companies seem to think Alt 1 is for IoT and Alt 2 is for wearable/eMBB. We do not see why IoT should be prioritized over wearable/eMBB. MTK: Agree to remove “as baseline”. At 2 is used to support more use cases and it is still unclear what will be the baseline/main use cases at this stage. Spreadtrum: We suspect the feasibility of Alt-2 currently. We prefer to keep “baseline” for Alt-1 vivo: we think current proposal doesn’t force the mapping between alternatives and use cases. Considering alt1 is better aligned with what has been agreed in LPHAP, we prefer to keep baseline for Alt1, otherwise we don’t agree to remove the brackets of Alt2. ZTE Fine. Since IoT use case has less requirement on mobility, it would be challenging for eMBB/XR to UE have significant power saving, and keeping latency low, UPT loss low. Apple2 Our view still stays. We understand we do not have any mapping to the use cases in the agreement right now, but that seems to be what many companies have in mind. So practically speaking, by adopting Alt 1 as the baseline, it potentially means that IoT is prioritized. So we still support v2, not v3. HW&HiSi#3: we don’t think there is any implicit mapping between alternatives and use cases/devices. We don’t see issue on v2. | FL1: @Apple, no clear statement saying which alternative is IoT or eMBB now. Many companies don’t support to limit to use alternative according to use case. But to accommodate Apple’s comment, FL suggest to check whether v3 is OK for you. FL2 After checking the stability of v2 and v3, Actually - [H] Proposal 1C-1-v2:: Futurewei, Spreadtrum, vivo, Samsung, Nokia, Intel, InterDigital - [H] Proposal 1C-1-v3:: Apple, MTK FL suggest to go [H] Proposal 1C-1-v2 |
| 7 | [H] Proposals 1A-2-v3: Working assumption For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), where Fr is: o Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal |  |  |
| 8 | [H] Proposal 1C-2-v3: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For cat1a, clock error option 3 or 4 is assumed, o FFS for cat1b, clock error option 1 or 2 is assumed o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed. - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v4: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver [o 10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o For other values, the mapping between power value and receiver type are FFS] - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- [H] Proposal 1C-2-v5: ----------------------------TP start------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001/ [Y, e.g., Y>=0.1 or 0.01]; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: o For WUR Off value 0.001, oscillator option 1, 2, 3, 4 and /FLL/PLL are not assumed and only RTC is maintained; o For WUR Off value >= Y1, option 1,2 can be assumed. FFS: Y1 o For WUR Off value >= Y2, option 3,4 can be assumed. FFS: Y2 and whether Y1=Y2 or not - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- | Intel: We are OK for the proposal relative power 10/20/30 for OFDM receiver. We are also fine to introduce parameter Y. Note4 overlaps with the sub-bullet for Option 3/4’in the second bullet in Proposal 1A-v-v1. It is better to have a single proposal Ericsson: Not OK with below part in the proposal second FFS point. We suggest to remove it. FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation o 10/20/30 for LP-WUR ON power state are used for OFDM receiver Huawei, HiSilicon: we have a couple comments as following: 1. 0.2 should be added considering we have proposal of sequence based receiver with power of 0.2; 2. We don’t need the sub-bullet of “10/20/30 for LP-WUR ON power state are used for OFDM receiver”. It is anyway to be handled by AI 9.11.2. And we actually have some proposal of OFDM based receiver, i.e. sequence based detection, to be with power consumption smaller than 1. 3. Thanks for adding the note5, but we should still clarify that all the values in the table is the power consumption for LP-WUS monitoring based on the following agreement. Therefore, Note 5 should be updated with the green revisions to “Note5: the power consumption value of LP-WUR ON is for LP-WUS monitoring, and Up to companies to report whether same or different values with justifications are assumed for WUS monitoring and time/frequency synchronization. Agreement - The following power model for LP-WUR/WUS evaluation is considered, o Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: o [0.001] o Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: o [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] o Other values are not precluded to be evaluated. o FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation …….. 4. for the bullet “For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed.”: a) option 1/2 and option 3/4 should be separated listed, considering they have very different assumption of frequency error and drift. E.g. if Y=0.1, it is sure that option 1/2 can be used because 0.1 is the same as power value of ON state, where option 1/2 can be used. B) For the detailed value of Y, if we just have examples in square brackets, we prefer to keep it as FFS . Apple OK in principle. On “10/20/30 for LP-WUR ON power state are used for OFDM receiver”, I assume the intention is to say these values are not to be used for other receiver types. But it may be misunderstood as OFDM receiver can only take these values. Suggestion: “10/20/30 for LP-WUR ON power state are not applicable to the receiver architectures for MC-OOK and MC-FSK.” For the highlighted note on oscillator, is the intention to say that “For WUR Off value 0.001, oscillator is assumed to be not running and only RTC is maintained”? MTK: Agree to add 0.2 considering we have proposed an OOK-OFDMA hybrid receiver with a power of 0.225. Nokia: In principle fine, but noting that, if Proposal 1D-2-v2 is agreed, should be accounted in this. HW&HiSi#2 on Proposal 1C-2-v4 1. On the updated wording regarding the power value of “10/20/30”, the current wording is still not clear enough from our point of view. we would be fine with Apple’s proposal with some revision: “10/20/30 for LP-WUR ON power state are not applicable to the receiver architectures of envelope detection for MC-OOK and MC-FSK.” ZTE/Sanechips We are fine and think 10/20/30 is only applied for OFDM based receiver. It is not true that the OFDM receiver can use other relative power, e.g., 1. If it is possible, please clarify how to achieve this. ZTE, Sanechips: We think 10/20/30 for OFDM based receiver should be kept. If 9.11.2 discuss the relatibe power for each architecture, the agreement in 9.11.1 also can be the reference. Some further clarification is clarified in the draft folder document. MTK2: We wonder why 10/20/30 cannot be used for MC-OOK and MC-FSK. It should be the opposite that “at least 0.01/0.05/0.1 cannot be used for OFDM receivers.” QC: We think it is not good having hard restriction for power numbers and receiver types. We propose to remove as follows. [o 10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, On Note 4, we are fine to add one more values. For simplicity, we propose to add one value with following clarification. 0.001: Only RTC is running during sleep. But, any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power. [0.005]: Option 1,2,3,4 can be assumed (always on LO). Apple2 (on v4) OK in principle. It appears that the sub-bullet in bracket or any proposed modification is still controversial. In this case, we would suggest removing it. We do not think it adds much value at this point. The power consumption will be discussed in 9.11.2 anyway. On note 4, we can be fine with it. But want to confirm if our understanding is correct: for WUR Off value 0.001, oscillator/FLL/PLL is assumed to be not running. So when WUR wakes up, it starts from the maximum frequency offset? Xiaomi: OK in principle. For WUR ON value, we do not think that restrictions are necessary for 10/20/30 and these restrictions can be reported by companies as additional assumptions. HW&HiSi#3 on Proposal 1C-2-v4: @FL2, our previous comments is 10/20/30 is NOT applicable to envelope detection of OOK/FSK. But it seems companies cannot align with the bullet of “10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, For other values, the mapping between power value and receiver type are FFS]”. We prefer to remove it as suggested by Qualcomm and Apple. Agree with Apple that the power consumption mapping shall be discussed anyway in 9.11.2. Ericsson2 (on v5): Prefer QC suggested updates to Note 4. As mentioned earlier, power mapping to different LP-WUR architectures can be done after there more convergence in 9.13.2 agenda item. QC (on V5): As pointed out by Eicsson2, We think hard restriction is not necessary and boundary/mapping between is arbitrary at this stage. Instead, we prefer to have qualitative description/comparison of power consumption values across different receiver types. o Alt1:10/20/30 for LP-WUR ON power state are used only for receiver types for OFDMA-based signal/channel, o Alt2: 10/20/30 for LP-WUR ON power state are not used for receiver types for MC-ASK and MC-FSK, o For other values, the mapping between power value and receiver type are FFS vivo2: Regarding the values 10/20/30, we think the justification to add them on is that receivers for OFDMA signals/channels have been agreed in the last meeting. We don’t see the any justification of these values for envelop detection for MC-ASK and MC-FSK. We are ok with either alt 1 or alt2, but we don’t agree to add 10/20/30 arbitrarily without solid justification. For the other values, we are ok to discuss further the mapping between power value and receiver type. | FL1: @Ericsson, Apple, Huawei, clearly the intension of adding 10/20/30 is for OFDM receiver for MC-OOK/FSK. However to move forward, FL add [] for the sub-bullet. And add FFS for other values. @Huawei, (1) added 0.2 (3) no need to say the WUR ON power is only for monitoring, the T/F sync power value can be picked up from these values and reported by companies if they use different values. (4) Make sense. Suggest separate bullet for option 1/2 and 3/4. Please check v4 FL2: @MTK, the value 0.2 is already in V4 @Nokia, OK @Huawei, not sure which is not clear. Are you going to suggest 10/20/30 for OOK/FSK? I don’t see any intension to do so. By adding the setence ‘For other values, the mapping between power value and receiver type are FFS’, for the rest of other values 0.01/0.05/0.1/0.2/0.5/1/2/4, I is still open to discuss the mapping.. if you have any concerns, I think we can consider to remove the []. FL doesn’t change anything in the previous v4. FL3: Again, I would like to note that the intension we discuss to add 10/20/30 is because in the last meeting we agree to study OFDM receiver. It make no sense to add 10/20/30 for any OOK/FSK based on my observation from contributions or practical usage. So as a matter of fact, I do not see any wrong if 10/20/30 is for OFDM receiver. If people really wants to discuss whether 10/20/30 is for OFDM receiver or can be applicable for OOK/FSK for evaluation in 9.11.2, I can remove 10/20/30 now and wait until discussion in 9.11.2 to finalize. As the FL, I would like to move us forward instead of waiting for the next meeting(the last second meeting for SI in RAN1). In V5, I listed two Alternatives for 10/20/30 and let’s see which one is preferred by companies. Please companies be constructive and practical. @Apple2, My understanding is same with you. for WUR Off value 0.001, oscillator/FLL/PLL is assumed to be not running. So when WUR wakes up, it starts from the maximum frequency offset. I updated note4 to address your comment @Qualcomm, I also add FFS for whether Y1=Y2 or not to address your another comment Please see [H] Proposal 1C-2-v5 |
| 9 | [M] Proposal 2A-v3: The latency for the target use cases are considered as follows: o Latency for RRC CONNECTED mode is in the order of milliseconds o Latency for RRC IDLE/INACTIVE mode is in the order of seconds | Huawei, HiSilicon: are these latency set to be the KPI for the evaluations? HW&HiSi#2: Thanks for the clarification. We are fine with latency value for connected mode. But for IDLE mode state, we do not think such a relaxed target should be set before a proper study on the feasibility of achieving a tighter target. Therefore, we prefer to keep IDLE mode open for FFS for the time being. [M] Proposal 2A-v3: The latency for the target use cases are considered as follows: o Latency for RRC CONNECTED mode is in the order of milliseconds o Latency for RRC IDLE/INACTIVE mode is in the order of seconds: FFS: whether to set a latency target; if so, FFS: the detailed value Spreadtrum: Agree with HW. As mentioned by some companies, the order of second in idle/inactive state will be harmful for phone call. vivo: Considering the typical value of I-DRX cycle, i.e., 1.28s, we think it is reasonable to keep the Latency for RRC IDLE/INACTIVE mode in order of second. ZTE/Sanechips: OK Apple: OK Xiaomi: Fine. QC: Support HW&HiSi#3: Some feedback on vivo’s comments: 1.28 seconds is one of the I-DRX cycle length. Specification also allows smaller values for shorter latency. Also, UE may have UE specific DRX cycle that is smaller than the one configured by gNB. Therefore, it is not reasonable to conclude this latency target based on one of the DRX cycle length. We see other companies also prefer to keep FFS for IDLE mode now. QC: We think the latency requirement should be determined from use case rather than technical feasibility. Even if a strict requirement is technically feasible, if it is not necessary in real use case, then, there is no need to have such strict requirement. We think having relaxed requirement which can met all the use case is better in the sense that it could further relax design requirement in terms of power, sync, overhead, complexity, etc. We support current FL’s proposal. | FL1: For KPI FL2: Not clear why in the order of second is wrong since legacy typical i-DRX allows to do so. If companies still have comments on [M] Proposal 2A-v3, FL suggest to drop this proposal. Hopefully companies can explain why the order of second is inappropriate FL3: As I suggest, If companies still have comments, let’s drop this proposal. Now I see Huawei still have, so let’s consider to stop discussing this proposal. |
| 10 | [H] Proposals 1A-4-v2: Confirm the WA for the followings Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. • Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS • Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4,  FFS applicability when LP-WUR ON power is less than[TBD] power unit  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD] power unit,  Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.  assumptions important for achieving performance by using MR clock for LR should be declared  Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).  Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,  The following are examples for consideration, other approaches are not precluded,  Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] <image002.png> - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr  Model 2: random time drifting, FFS details - FFS: Phase noise model | Intel: the sub-bullet ‘for Option 3/4’in the second bullet has overlap with Note4 in Proposal 1C-2-v3. It is better to have a single proposal Ericsson: Following part should be deleted if confirming the WA given inputs provided to this meeting regarding clock power consumption. i.e., as follows - For Option 3/4,  FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,  e.g., option 3/4 is not applicable  when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,  when LR monitoring power less than [TBD]power unit, MTK: Using Oscillator error to model CFO is not enough. To maintain stable frequency output, at least FLL/PLL cannot be turned off, which is power-consuming. Suggest adding a note below. option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies Note. At least FLL/PLL cannot be turned off for LR to support options 1/2/3/4. Nokia: A question for clarification, that for RTC case we should assume always random offset at 20ppm range upon LR wake-up? Samsung: According to the proposal #8, some options for oscillator cannot be assumed for WUR off-state. In our understanding, it means that oscillator is turned off and frequency drift of oscillator cannot be used for calculation of time/frequency error such as Model 1 in the proposal #10. Therefore, we suggest to add Note for clarification, “Note: assumption for frequency drift cannot be used when the oscillator is assumed to be turned off” QC: We are find removing red FFS related to option 3 and 4. Apple: we are getting a bit confused on how it works. Assuming the LR OFF power of 0.001, do we assume oscillator is still running, just FLL/PLL are turned off? I think everything should be turned off, except for the RTC. In this case, what happens when the LR is turned on again? Maximum frequency offset should be assumed? Then the drifting model does not apply any more. QC: We think t/f drifting model will apply to RTC clock. Depending on design, LO could be always on or shutdown during sleep duration. The FLL/PLL could start when LP-WUR wakes up to ON duration. The power of LO/PLL/FLL should be already included in LP-WUR On power. Intel: considering 1C-2 and 1A-4 jointly, we think it ends up behaviors of 4 combinations in the following table. We add our current understanding in the table now. On power < [TBD]; On power >= [TBD]. Off power 0.001;  Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used  Frequency offset when LP-WUS switches to on follows option 1/2 (first value in the value pair). FFS option 3/4;  Time offset cumulated in the off period cannot be calculated by option 1/2/3/4. RTC should be used  Frequency offset when LP-WUS switches to on follows option 1/2/3/4 (first value in the value pair).. Off power Y (e.g., Y>=0.1 or 0.01);  Time offset cumulated in the off period can be calculated by option 1/2. RTC can be used too [*1]. FFS option 3/4.  Frequency offset when LP-WUS switches to on follows option 1/2 (cumulating the second value in the value pair). FFS option 3/4;  Time offset cumulated in the off period can be calculated by option 1/2/3/4. RTC can be used too. [*2]  Frequency offset when LP-WUS switches to on follows option 1/2/3/4 (cumulating the second value in the value pair).. *1: since RTC is better, so RTC can be used by UE *2: up to UE to select a better way, the accuracy is option 3/4 < RTC < option 1/2 Ericsson2: “FFS applicability when LP-WUR ON power is less than[TBD] power unit” and the other note on FLL/PLL are not needed. The architecture agenda item already covers clock accuracy vs. power consumption study. QC : We think Note 1 is not sufficient. We suggest to remove Note 1 or add additional Note 2 and 3 w/ following modification. [ Note1. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain always on for when monitoring receiving LP-WUS. (Always on monitoring) Note2. FLL/PLL could be turned on only when LP-WUR is on for LP-WUS monitoring (duty cycled monitorig). Note3. The clock error (of both RTC and LO) could be improved beyond max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.] For completeness of model, we suggest to add drift of 0.1(ppm/sec) for RTC RTC max frequency error [ppm]: (20, 0.1) Samsung2: We have a similar question with Apple. We want to know and clarify how to calculate time/frequency error model for duty-cycle based LP-WUS monitoring when oscillator/FLL/PLL can be turned off during LR off-state. Apple2: QC’s proposal on adding drift for RTC seems to make sense, to cover the cases where the LO is off. But I am still not sure that I completely understand the model. Intel’s table seems to be a good starting point for clarification. Our preference would be not to confirm the WA before understanding these details. | FL1: @Intel, @Ericsson, removing ‘for option 3/4…’ But the last sub-bullet is related to whether Oscillator option 3/4 is applicable for WUR ON power. It is not covered in Proposal 1C-2. FL suggest to keep it FL2 @MTK, the added note is not clear since some ED detector doesn’t need to have PLL/FLL? But I understand your intension, perhaps we can modified a bit saying Note. FLL/PLL cannot be turned off for LR to support options 1/2/3/4 if it is required to maintain for receiving LP-WUS @Samsung, I think what you proposed is relavant to the [H] Proposal 1C-2-v4. For example, if the WUR OFF is 0.001, the frequency drifting is not assumed to be used for calculating time/frequency error. We can solve this in that proposal or later. Since the value Y in H] Proposal 1C-2-v4is not yet done. Now changed to [H] Proposals 1A-4-v2 FL4 Thanks Intel for the summary, As suggested also by Apple, maybe we need more clarification. Therefore I suggest to take it for further discussion, e.g., - start from Intel’s table for further clarification. - Also discussing Qualcomm’s RTC assumption for adding drifting 0.1ppm/s |