3GPP TSG RAN WG1 #112bis-e			R1-2302275

e-Meeting, April 17th – April 26th, 2023

3GPP TSG RAN#99			RP- 230780

Rotterdam, Netherlands, 20th – 23rd March, 2023

Title:	Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

Response to:	R1-2302186/RP-230033

Release:	Rel-18

Work Item:	          NR_FR1_lessthan_5MHz_BW-Core

Source:	RAN

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Matthew Baker

E-mail Address:	<Matthew.Baker@nokia.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN would like to thank RAN WG1 for their LS on transmission bandwidths for NR on dedicated spectrum less than 5 MHz.

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, and concluded the following:

- For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4):

  - PBCH transmission bandwidth is 12 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands.

- For the 5MHz channel bandwidth:

  - PBCH transmission bandwidth is 20 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- Other details (including sync raster details) are to be progressed in the WGs.

2. Actions: RAN respectfully requests RAN1 to take the above information into account for their further work.

3. Dates of Next RAN Meetings:

TSG RAN Meeting #100		June 12th – 14th, 2023	Taipei

TSG RAN Meeting #101		September 11th – 15th, 2023	TBC, India