3GPP TSG RAN WG1 #112bis-e 		                                    		       R1-2304147

e-Meeting, April 17th – 26th, 2023

# Title:	Reply LS to RAN2 on error source distributions

# Response to:	R1-2302282/ R2-2302271

# Release:	Rel-18

# Work Item:	NR_pos_enh2

# Source:	RAN WG1

# To:	RAN WG2

Cc:	-

Contact Person:

Name:	Fumihiro Hasegawa

E-mail Address:	fumihiro.hasegawa@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R1-2302282/ R2-2302271 on error source distributions.

RAN1 would like to provide the answers to the following questions:

Q1: RAN2 would like to confirm with RAN1 on the agreement “RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.” regarding the distribution of error sources

Reply to Question 1:

From RAN1’s perspective, the RAN2 agreement “the error sources are overbounded by a Gaussian distribution” can be confirmed for the error sources listed in Table 6.1.1-2 in TR 38.859 if the error source follows a Gaussian distribution.

Q2: RAN2 respectfully ask RAN1 to provide the parameters (e.g. mean and standard deviation) for the overbound Gaussian distribution.

Reply to Question 2:

Parameters for the overbound Gaussian distribution can be mean and standard deviation.

From RAN1’s perspective, zero is a valid possible option for the mean value for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859.

From RAN1 perspective, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859.

2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #113              22 May – 26 May, 2023        Incheon, Korea

TSG RAN WG1 Meeting #114              21 August – 25 August, 2023        Toulouse, France