3GPP TSG RAN WG1 Meeting #112bis-e	R1-2304156

e-Meeting, April 17th – April 26th, 2023

Title:	LS to RAN2 on introduction of one new RRC parameter and one new UE capability for Rel-17

Release:	Release-17

Work Item:	TEI17, NR_newRAT_Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact person:

Name:		Mostafa Khoshnevisan

E-mail Address:	mostafak@qti.qualcomm.com

# Overall Description

RAN1 agreed to the following in RAN1 #112bis-e related to Type1 HARQ-ACK codebook generation. Per the agreement, one new RRC parameter and one new UE capability are introduced for Rel-17 specifications. RAN1 respectfully asks RAN2 to implement these in the Rel-17 specifications 38.331 and 38.306. Furthermore, RAN1 suggests RAN2 to capture Note 2 and Note 3 in the description of the new RRC parameter.

Agreement Support to introduce one RRC parameter in Rel-17 related to the condition of “receiving more than one unicast PDSCH or multicast PDSCH per slot” in Type1 HARQ-ACK CB generation. - A UE capability is introduced which indicates UE supports receiving this new RRC parameter. - The new RRC parameter if configured, is per CC: - If the new RRC parameter is not configured, the legacy text in 38.213 Section 9.1.2.1 applies - If the new RRC parameter is configured and is set to ‘disabled’, the procedures of the “if” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation. - If the new RRC parameter is configured and is set to ‘enabled’, the procedures of the “else” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation. - The following TP for 38.213 Section 9.1.2.1 is agreed in principle to reflect the above: - Note 1: The name of this new RRC parameter is temporary and will be updated once it is introduced by RAN2. - Note 2: This new RRC parameter does not impose a general scheduling restriction on number of PDSCHs that can be received by the UE in a slot beyond the impact on Type 1 HARQ-ACK codebook generation in 38.213 Section 9.1.2.1. - Note 3: when this new RRC parameter is configured, and disables receiving more than one PDSCH per slot for Type1 HARQ-ACK codebook generation: - It does not impose a restriction on the number of PDSCHs that can be received per slot, if their HARQ-ACK’s are not reported in the same PUCCH resource in the same slot. - UE does not expect to receive more than one PDSCH per slot with the HARQ-ACK’s reported on the same PUCCH resource in the same slot. - When two coresetPoolIndex values are configured, the one PDSCH is per coresetPoolIndex value per slot. - When UE generates two HARQ-ACK codebooks for two priorities, the one PDSCH is per priority per slot. - When fdmed-Reception-Multicast is configured, the one PDSCH is per traffic type (unicast / multicast) per slot. <unchanged text omitted> if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB = ‘disabled’] and R≠∅, MA,c=MA,c∪j; j=j+1; else Set CR to the cardinality ofR <unchanged text omitted> end if <unchanged text omitted> Send an LS to RAN2 for introducing the new RRC parameter and the new UE capability in Rel-17, and suggest RAN2 to capture Note 2 and Note 3 in the description of the new RRC parameter.

# Actions

To TSG RAN2:

Action: RAN1 respectfully asks RAN2 to introduce the new RRC parameter and the new UE capability in Rel-17 specifications according to the above RAN1 agreement.

# Dates of Next TSG-RAN1 Meeting:

RAN1 #113			22 May – 26 May 2023			Incheon, KR

RAN1 #114			21 Aug – 25 Aug 2023			Toulouse, FR