3GPP TSG RAN WG1 #112bis-e		R1-2304211

e-Meeting, April 17th – April 26th, 2023

Title:	Reply LS on comparison of SL-RSRP and SD-RSRP measurements

Response to:	R1-2302280/R2-2302234

Release:	Rel-18

Work Item:	NR_SL_relay_enh-Core

Source:	RAN1, Nokia

To:	RAN2

Cc:	RAN4

Contact person:	Torsten Wildschek

torsten.wildschek@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN1 would like to thank RAN2 for the LS R1-2302280/R2-2302234 on comparison of SL-RSRP and SD-RSRP measurements. In the LS, RAN2 asked the following question:

Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

RAN1 informs RAN2 that comparison of SL-RSRP and SD-RSRP measurement is affected by at least the following issues

- Unicast is subject to transmit power control based on SL pathloss if parameter sl-P0-PSSCH-PSCCH is provided, while broadcast is not subject to this power control mechanism.

- Due to CBR-based power control, if configured, maximum transmit power may depend on the priority of the transmission. The priority for discovery messages may be different from the priority used for the transmissions over which SL-RSRP is measured.

- Transmission of discovery messages can take place in dedicated discovery pools; these may be configured differently, e.g. with respect to power control settings, from the pool(s) on which SL-RSRP is measured.

- Note: Sidelink carrier aggregation, for which an objective was added to Rel-18 NR_SL_enh2 at RAN#99, may additionally impact SL transmit power. 
Note: RAN1 has not started work on SL carrier aggregation.

RAN1 reply: RAN1’s understanding is that comparison of SL-RSRP and SD-RSRP measurement cannot be used for the purposes of triggering a measurement report at least due to the above outlined issues, and the decision on whether to use comparison of SL-RSRP and SD-RSRP measurement is up to RAN2.

# 2	Actions

To RAN2: RAN1 respectfully asks RAN2 to take the above response into account in the future work.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#113	22-26 May 2023   	Incheon, KR

RAN1#114	21-25 August 2023   	Toulouse, FR