3GPP TSG RAN WG1 #113	R1-2304356

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.11.3

Source:	Futurewei

Title:	LP-WUS Physical Signal Design and Performance

Document for:	Discussion/Decision

# Introduction

A new study item on low-power wake-up signal and receiver for NR was approved in RAN#94e [1] and revised in RAN#97e [2]. The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, and the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In prior RAN1 meetings, a set of agreements were reached and are available in Appendix A for reference. In this contribution, we discuss the design considerations for the Low-Power (LP) Wake-Up Signal (WUS), generation and transmission schemes, and performance results.

# Discussion

Duty-cycled operations in the form of Discontinuous Reception (DRX) and extended Discontinuous Reception (eDRX) are defined for power consumption reduction in NR RRC_IDLE and RRC_INACTIVE states through the reduction of the number of Paging Occasions (POs) monitored by the UE. Further power consumption reduction can be achieved through Paging Early Indication (PEI) in NR RRC_IDLE and RRC_INACTIVE states, which is still subject to the duty-cycled operations. Similar power saving techniques are defined for NR RRC_CONNECTED state in the form of connected mode DRX (C-DRX) and Wake-Up Signal (WUS). Both PEI and WUS can be received by UEs as DCIs over the PDCCH.

The DRX, eDRX, and C-DRX can provide higher power saving gain by increasing the duty cycle duration (putting the UE in a deep or ultra-deep sleep for as much as possible) at the expense of higher latency to be expected by the UE. PEI and WUS can provide more power saving gain without an impact on latency, but the gain is limited by the power consumption required to decode a DCI over PDCCH.

Existing PEI/WUS designs require coherent reception which might have better performance than non-coherent counterparts but can be limited in meeting ultra-low power consumption requirements, e.g., ~100 μW power consumption, in deep/ultra-deep sleep states [3]. A new WUS (LP-WUS) that can be received with significantly lower power consumption than existing PEI/WUS designs may enable new trade-off regions of Latency versus Power but will require a dedicated Low-Power Wake-Up Radio/Receiver (LP-WUR) with a simple architecture [3]. The simple architecture may ideally use passive circuit at the RF front-end and apply non-coherent reception where only energy detection is performed. On-Off Keying (OOK) type of modulation usually works well in generating WUS for energy detection type of receivers. The detection output is then used as input to the digital signal processing (DSP) unit for further digital domain processing. The processed result determines whether the WUS is targeting the receiver and if indeed it is the case, corresponding operations and procedures will be performed.

LP-WUS design considerations such as multiplexing, resource allocation, structure and content, interference mitigation, and performance KPIs are discussed in Section 1.1. Further, approaches to LP-WUS generation and transmission considering aspects related to conformance to existing OFDM transmitter architectures and accommodation of low-power receiver architectures’ limitations are discussed in Section 1.2. Link-level simulation results considering OOK-1, OOK-2, OOK-4, and FSK-2 waveform options identified in RAN1#112 are presented in Section 1.3.

## LP-WUS Design Considerations

### Multiplexing and Resource Allocation

A specific frequency band/resource can be dedicated to the transmission of LP-WUS which may simplify the design of low-power WUS receiver (LP-WUR) and eliminate/reduce interference to be experienced by the LP-WUR. For a dedicated frequency resource, LP-WUR could have a more certain idea on where to receive the WUS and make its RF front-end less tunable to reduce complexity and cost. However, dedicated frequency resources only for LP-WUS transmissions imposes higher resource overhead and less network resource utilization, which is a much more noticeable concern in cellular network than in unlicensed networks, e.g., WiFi. Further, if the dedicated frequency band is not at the edge of system bandwidth, it can create holes/fragmentation in the system frequency resources, putting more restrictions on resource scheduling and reducing system resource allocation flexibilities. It is possible that some sort of DRX operation may be considered to further reduce the power consumption of WUS reception and, therefore, during the off-duty cycle of the DRX for one LP-WUR, the frequency resources (i.e., dedicated for LP-WUS transmission) could be used for the transmission of LP-WUSs targeting other receivers. However, in case there are not enough LP-WUS transmissions to occupy the dedicated time-frequency resources, there will be a waste of resources. A more favorable idea is to allocate dedicated time-frequency blocks for LP-WUS transmissions. The time-frequency block allocation should be configurable and adjustable based on the resource demand of the LP-WUS transmission. The time-frequency blocks could be utilized to transmit other legacy NR channels or signals when there is no need for LP-WUS transmission. In NR, BWP concept seems to fit the WUS time-frequency block idea very well and can be considered for the resource allocation of LP-WUS transmission.

Observation 1: The LP-WUS can be scheduled in any time-frequency resources within the configured BWP i.e., no dedicated time-frequency resources.

Further, LP-WUS can be transmitted on frequency tones corresponding to one or more allocated resource blocks (RB). The number of RBs should be configurable to provide frequency diversity in case of frequency selective channel. Accordingly, filtering at the LP-WUR must be tunable to accommodate variable LP-WUS transmission bandwidth, causing an increase of LP-WUR hardware complexity and power consumption. Tones at the edge of the allocated RBs may not be used to carry LP-WUS transmission for the purpose of filter bandage roll off. The amount/fraction of unused tones may be determined considering the filter requirements which could be studied during the SI.

### Structure and Content/Information

To reduce the cost and power consumption of LP-WUR, a low-accuracy oscillator may be used which may result in large time drifting during the silent periods of LP-WUS transmissions. Therefore, two different design approaches may be considered: sequence-based design or message-based design preceded by a preamble. The sequence-based design might limit the amount of information that can be transmitted by the LP-WUS since the sequence detection complexity can increase exponentially with the information size. On the other hand, e.g., for a use case such as carrying certain system information to a group of LP-WURs, the message-based design that includes a preamble as a precursor to aid in preparing the RF front-end, e.g., AGC and filtering, and acquiring more accurate timing synchronization for the LP-WUS message part reception may be needed. The preamble may also be used to reduce the false alarm rate (FAR) of LP-WUS, which can have a negative impact on the power saving gain experienced by LP-WURs compared to legacy schemes, but at the expense of increased preamble miss-detection rate (MDR). The LP-WUS information is contained in the message part which may contain a UE ID or a UE group ID. To support mobility, cell ID may also be included along with UE/group IDs. Alternatively, it is also possible that cell ID, along with other system information, may be transmitted independently as broadcast information to assist UE mobility, i.e., same design can be used to carry information (UE ID, UE sub-group/group ID, System Information change notification indication, and emergency message notification indication) and synchronization signal (cell ID, timing stamps, … etc.).

Further, a CRC of the information bits may be transmitted, alongside the message-based LP-WUS, to effectively reduce the FAR of LP-WUS without impacting the preamble MDR. However, it should be noted that increasing the LP-WUS length may have negative impacts on its overall MDR for LP-WURs that rely on the energy detection of received signals as it becomes more prone to performance degradation caused by high power interference signals. For example, at the cell edge, the interference of data transmission or neighboring cell LP-WUS transmissions could be stronger than target LP-WUS. Transmission of longer sequence increases the chance of LP-WUS being exposed to high power interfering signal which may result in higher miss detection rate. Therefore, evaluations can be done to investigate the trade-off between LP-WUS length and its performance during the SI phase.

FEC coding may effectively help combat transmission errors. Depending on the LP-WUR reference receiver discussion, UE may perform hard or soft FEC decoding. In the SI phase, the type of FEC code, code rate etc. could be investigated and evaluated through simulation.

Proposal 1:  The LP-WUS can carry information indicating: 
- UE, UE sub-group, or UE group ID(s)
- cell information (ID)
- System information change and/or emergency message indication(s)

Proposal 2: For message-based LP-WUS, a CRC of proper length should be supported to improve FAR and reduce its impact on power saving gain without degrading the preamble MDR.

### Interference Mitigation

For LP-WUS targeting cell edge receivers, LP-WURs may experience relatively strong interference from neighboring cells. Additionally, OOK modulated LP-WUS is received with energy detection which is less resilient to interference. To ensure satisfactory coverage for the cell edge receivers, the network may need to utilize some interference mitigating schemes. Base stations coordination could be considered such that when one base station is transmitting in certain time-frequency resources, the other surrounding base stations perform muting on these time-frequency resources to reduce the interference dramatically. Base station coordination increases network complexity and muting decreases the system efficiency. The other way to mitigate the interference impact is to employ more complicated LP-WUR and/or LP-WUS design at the cost of increased hardware complexity and power consumption.  An example LP-WUS design that utilizes low envelope IFs and can mitigate the impact of inter-cell interference without increasing the complexity of LP-WURs is discussed in Section 1.1.7. Studies should be done to investigate the optional schemes to deal with inter-cell interference.

For LP-WUS coexistence with legacy channels/signals, intra-cell interference also needs to be considered in some scenarios. In one case the low/band pass filtering is not providing enough suppression on co-scheduled signals, interference from other channels/signals degrade the LP-WUS detection performance. In another case, if LP-WUR is allowed to relax its filtering bandwidth over carrier frequency requirement, portion of legacy channels/signals may pass through the filtering along with LP-WUS, then more advanced schemes may be employed to prevent it from degrading LP-WUS energy detection. The LP-WUS design utilizing low envelope IFs discussed in Section 1.1.7 can also provide robustness against intra-cell interference.

Proposal 3: Study LP-WUS design utilizing low envelope intermediate frequencies (IFs) to mitigate the impacts of intra-/inter-cell interference.

### Performance KPIs

For a LP-WUS consisting of a preamble sequence and message bits, performance requirements should consist of two parts, preamble detection performance and message body decoding performance. The requirements for the preamble sequence part may include miss-detection rate and false-alarm rate, e.g., 10-2 and 10-3, whereas decoding error rate could be considered for the message part, say, e.g., 10-2.

Proposal 4: Separate and joint performance KPIs for preamble and message parts should be defined. For the preamble, at least miss-detection rate should be adopted as KPI and similarly decoding error rate for the message part.

## LP-WUS Generation and Transmission

There are several ways to transmit OOK-modulated signals within 3GPP systems. One option is to use a separate hardware module in gNB to generate the LP-WUS which can then be mixed with other NR signals before transmission. Since a separate hardware module is used for LP-WUS, the design of the LP-WUS is more flexible. For example, the on-off duration can be quite flexible and doesn’t need to be restricted to, e.g., OFDM symbol boundary of legacy NR signals. Signal content in the ON duration can also be flexibly designed and optimized for certain receiver assumptions. While additional hardware does increase the cost and complexity of base station hardware, another drawback is that the arbitrary signal content does not coexist well with legacy channels/signals. Loss of orthogonality from LP-WUS to legacy channels/signals may occur and system performance may degrade. Therefore, sufficient guard bands may be required around the LP-WUS to maintain the orthogonality of subcarriers allocated for other NR signals/channels. The other option to transmit the LP-WUS within the OFDM framework is to generate a signal that conforms with the OFDM transmitter architecture as discussed in Section 1.1.5. Further, pre-processing of LP-WUS, i.e., before transmission, may be considered to assist LP-WURs achieve good in-band selectivity without significant penalty on power consumption as discussed in Section 1.1.6 or to eliminate the need for significant guard bands between frequency domain multiplexed segments as discussed in Section 1.1.7.

### Conformance to OFDM Transmitter Architecture

In one scheme to transmit LP-WUS within OFDM framework is to generate on-off (OOK) signal based on OFDM symbols (OFDM-OOK). LP-WUS is transmitted in ON state and not transmitted in OFF state for current OFDM symbol duration. This scheme fully conforms with the OFDM transmitter architecture, the OFDM-OOK operates on the OFDM symbol boundaries, and there is not much need to change base station hardware. However, in FR1 the sub-carrier spacing (SCS) can either be 15kHz, 30kHz, or 60kHz, thus OFDM symbol duration can be quite long for the LP-WUS transmission on-off granularity, i.e., only a data rate of 14kbps, 28kbps, or 56kbps can be supported.

In another scheme that can be considered for OOK signal generation while still conforming with the OFDM transmitter architecture is the one that utilizes DFT as pre-processing before OFDM symbol generation, i.e., DFT-s-OFDM based OOK (DFT-S-OFDM-OOK) or (DFT-OOK). In this scheme, the OOK modulated signal is generated in the time domain then transformed into frequency domain through DFT and mapped to desired tone locations, i.e., sub-carriers (SCs). Then the frequency domain LP-WUS signal, along with other legacy NR channels/signals go through IFFT to get time domain baseband signal for further processing and transmission. This transmission scheme also allows full reuse of base station hardware thus low cost and complexity. One advantage of this scheme over OFDM-OOK is that it can divide the OFDM symbol duration into finer granularities enabling one OOK signal to occupy smaller duration than the OFDM symbol duration. Thus, LP-WUS data rate could be increased and LP-WUS transmission time can be shortened which helps to reduce the chance of it being interfered with other high-power signals. However, OFDM-OOK detection performance can be superior to the DFT-S-OFDM scheme and our initial evaluation confirms their performance gap.

In Figure 1, initial evaluation of sequence miss-detection rate (MDR) is provided where OFDM-OOK and DFT-S-OFDM-OOK are compared for different values of LP-WUR sampling frequency, i.e., Fs∈112.12, 224.23, 445.22, 903.53 kHz, and at a LP-WUS transmission rate of 56 kbps. The transmission bandwidth is assumed to be ~2.9MHz and the TDL-C channel model is considered at a delay spread of 300 ns. The 32-bit long sequence, S, used by IEEE802.11ba as a synchronization preamble is considered.

S=1 0 1 0 0 1 0 0 1 0 1 1 1 0 1 1 0 0 0 1 0 1 1 1 0 0 1 1 1 0 0 0

A SCS of 240kHz is considered for OFDM-OOK whereas a SCS of 15kHz is considered for DFT-S-OFDM-OOK. Additionally, a carrier frequency of 4GHz, a UE mobility speed of 3km/h, and a detection threshold of 85% of the sequence length are considered. The results show less performance variation for OFDM-OOK than DFT-S-OFDM-OOK in response to the change in the LP-WUR sampling frequency. Further, the results suggest that a minimum sampling frequency of ~8x the LP-WUS transmission rate is required for DFT-S-OFDM OOK to achieve a similar performance to that of the OFDM-OOK transmission scheme.

Figure 1: Performance Comparison between DFT-S-OFDM-OOK and OFDM-OOK.

Observation 2: A minimum sampling frequency of ~8× the LP-WUS transmission rate of 56kbps is needed for DFT-S-OFDM OOK to achieve a similar MDR performance as the OFDM-OOK transmission scheme at a sampling frequency of only ~2× the LP-WUS transmission rate under the TDL-C 300ns channel.

One disadvantage of OFDM-OOK compared to DFT-S-OFDM based OOK lies in the relative longer OFDM symbol duration which reduces OFDM-OOK maximum data rate and is more prone to interference. One way to deal with this issue is to allow more SCS numerologies for FR1, in addition to SCS 15kHz, 30kHz, and 60k Hz already defined in the standard, e.g., 120k/240k Hz or even 480k/960k Hz may also be considered. The higher SCS for LP-WUS transmission, the shorter OFDM symbol duration and thus the higher maximum data rate. While shorter OFDM symbol duration also means less signal energy and less multi-path delay resistance which results in performance degradation. So, the trade-off between SCS and detection performance needs to be studied.

### In-band Selectivity Aware LP-WUS Design

In RAN1#110bis-e, three types of receiver architectures are agreed to be considered for the LP-WUR as suitable for OOK modulation.

Agreement Study at least the following three types of receiver architectures for LP-WUR: - Architecture with RF envelope detection - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection - Note: The details of each type of receiver architecture are discussed separately. - Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures can be applicable for other modulations such as FSK.

The RF envelope detection architecture achieves low-power consumption by avoiding the utilization of Local Oscillators (LOs) and Phase-Locked Loops (PLLs) whereas the IF/Baseband envelope detection architectures achieve low-power consumption by relaxing the accuracy and stability requirements of the LO. The removal of LOs creates another challenge for RF envelope detection architectures where suppression of interference from legacy NR signals/channels or other LP-WUSs on adjacent subcarriers may require very high-Q matching networks and/or RF BPFs. Implementations of high-Q matching networks and/or RF BPFs can be costly and/or bulky, e.g., require off-chip components. On the other hand, the use of low accuracy and stability LOs for IF/Baseband envelope detection architectures creates uncertainty in the IF/Baseband frequencies occupied by the LP-WUS. The uncertainty requires sufficient guard bands around the LP-WUS to suppress interference from any signal multiplexed on adjacent subcarriers.

Therefore, a solution that can aid RF/IF/Baseband envelope detection architectures to suppress interference from signals on adjacent subcarriers while relaxing the requirements on matching networks/RF BPFs for RF envelope detection architecture and/or guard bands for IF/Baseband envelope detection architectures is needed.

Two approaches that can be considered are described below.

- Fixed LP-WUS Design with Dual Spectrum Allocation:

A dual-spectrum allocation scheme, as shown in  Figure 2 , can be considered for RF envelope detection architectures to aid in interference suppression. In this scheme, LP-WUS frequency resources are allocated at the two edges of the channel with unallocated resources, i.e., guard bands, between the LP-WUS resources and resources allocated to any other signal. Since RF signals, xt, are converted into baseband signal directly via the non-linear operation, e.g., self-mixing xt×x*t, of RF envelope detection, the resulting spectrum of the baseband signal is Fxt⊛Fx*t=Xf⊛X*-f which is equivalent to the autocorrelation of the RF signals’ spectrum, where ⋅* is the conjugation operation, F⋅ is the Fourier transform operation, and ⊛ is the convolution operation. The LP-WUS can then be extracted in baseband as the signal centered at a known/fixed frequency (based on design of LP-WUS and guard bands at RF) of, e.g., B1-B2, and have a passband bandwidth of, e.g., 2B2, where B1 is the channel RF bandwidth and B2 is the LP-WUS RF bandwidth. With this spectrum allocation, the existing NR signals or any interfering signals falling within the spectrum occupied by the NR signals do not produce baseband envelop signals at the frequencies of the desired signal, therefore do not interfere with the reception of the desired signal, thus achieving in-band selectivity.

Figure 2 : Example frequency response of a single OFDM symbol under LP-WUS dual-spectrum allocation.

It is noted that while the dual-spectrum allocation scheme can solve the in-band selectivity problem, i.e., suppression of interference from signals on adjacent subcarriers, it requires sufficient guard band around frequency resources allocated to LP-WUS. Therefore, the dual-spectrum allocation scheme might not be a spectrally-efficient solution, but it can be suitable for use when LP-WUS is expected to share the channel only with low resource utilization signals/transmissions such as SSB. Further, this solution can be used for both OFDM-OOK and DFT-OOK.

- Dynamic LP-WUS Design with Single Spectrum Allocation:

As opposed to the dual-spectrum allocation scheme with fixed LP-WUS design, another scheme can consider only single contiguous spectrum allocation to LP-WUS at one of the channel edges. However, the LP-WUS generated to occupy that spectrum is dynamically designed to account for the interfering signal occupying the other edge of the channel as shown in Figure 3 .

Figure 3 : Example frequency response of a single OFDM symbol under LP-WUS dynamic design with single-spectrum allocation.

As mentioned earlier, the use of low accuracy and stability LOs for IF/Baseband envelope detection architectures creates uncertainty in the IF/Baseband frequencies occupied by the LP-WUS. The uncertainty requires sufficient guard bands around the LP-WUS to suppress interference from any signal multiplexed on adjacent subcarriers which may not be a resource efficient solution.

The dual-spectrum allocation scheme may not provide any resource efficiency for IF/Baseband envelope detection receiver architectures since it requires itself enough guard bands, i.e., comparable to LP-WUS allocated bandwidth, around each single allocated spectrum to eliminate interference. It may, however, relax the requirements on the IF BPF characteristics, e.g., for IF envelope detection receiver architectures, which might result in a lower implementation cost and/or better integration capability.

On the other hand, the dynamic LP-WUS design described above can be used to provide a resource efficient solution by generating the target signal at specific/fixed and accurate frequency after envelope detection where the guard bands around NR carriers can be reused, i.e., to minimize the requirements on any additional resources needed to mitigate the LOs uncertainty. Further, the design enables the relaxation of the IF BPF design requirements, e.g., for IF envelope detection receiver architectures. The scheme is illustrated in Figure 4 for IF envelope detection receiver architectures where the frontend (FE) RF matching network/BPF is used to select a desired NR band, an IF BPF is then used at IF to select the whole desired serving carrier, and an IF envelope detection is used to translate the signal to baseband. The dynamic LP-WUS design then results in a target/desired OOK signal that is located/centered at a specific (and accurate) frequency, IF2, with sufficient guard band for BB filtering. It is worth noting that the design requirements for the BB filter should be more manageable than IF BPF design requirements due to the expected lower center frequency of IF2 than at IF.

Figure 4 : An illustration of feasibility/benefit of dynamic LP-WUS design for IF envelope detection receiver architectures.

### Resource-Efficient Parallel OOK/FSK LP-WUS Design

In RAN1#112, the following two agreements are achieved to evaluate parallel OOK (Option OOK-2) and FSK modulation schemes. For the transmission of M bits per OFDM symbol, Option OOK-2, Option FSK-1, and Option FSK-2 require M, 2M, and 2M segments of frequency resources, respectively, with potential guard bands in-between and around.

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded.

As discussed in Section 1.1.6, the potential guard bands can account for a considerable number of wasted frequency resources due to the use of low accuracy and stability LOs for IF/Baseband envelope detection architectures and/or reduction in LP-WUS transmission power due to PSD limitations. In this section, we describe two approaches for LP-WUS design that can minimize or eliminate the need for the guard bands in-between the frequency resources segments without the requirement of a procedure for frequency error estimation and mitigation.

- Parallel OOK with Reference Segments

Out of the three LP-WUS generation options described above, parallel OOK (Option OOK-2) is the one that requires the least number of segments M, subsequently, the least number of guard bands in-between, to carry the same number of M bits. However, to achieve a comparable performance as the other two options, time-domain Manchester encoding may be required which can reduce the effective/information bit rate by half. Alternatively, the number of segments for Option OOK-2 can be increased to M+2 where the two additional segments can be used as ON/OFF levels reference in replacement to time-domain Manchester encoding. A corresponding example receiver operation is shown in Figure 5, but it should be noted that the parallel OOK branches do not necessarily have to be associated with a single LP-WUR.

Figure 5: An example 4-bit receiver utilizing two reference segments M+2, M=4.

Observation 3: Out of M-bits LP-WUS generation Options OOK-2, FSK-1, and FSK-2, OOK-2 requires the least number of segments (M) but may require Manchester encoding, i.e., achieve half the information bit rate, for a comparable performance as the other two options.

Observation 4: For M>2, additional 2 segments for Option OOK-2 acting as ON/OFF levels reference may help achieve similar performance to Options FSK-1 and FSK-2 without information bit rate loss or significant increase in number of segments.

The two reference segments may further be utilized to carry one additional bit of information where a 1-bit FSK receiver can be used to decode this additional bit. The decoded bit can then be used to toggle the decision of the M-bits and obtain the correct information bits as shown in Figure 6.

Figure 6: An example 4-bit receiver utilizing two FSK-modulated reference segments M+2, M=4.

Observation 5: The additional 2 segments for Option OOK-2 can further be FSK modulated to carry one additional bit of information.

Proposal 5: Support the additional 2 segments for Option OOK-2 as ON/OFF levels reference with the following three alternatives:
Alt 1: the two segments carry known reference signals and have fixed location.
Alt 2: the two segments carry known reference signals and follow a frequency hopping pattern.
Alt 3: the two segments carry a signal that is FSK modulated .

- FSK/OOK using Envelope IFs

The main drawback of the described parallel OOK with reference segments above is that it still requires the segmentation of the LP-WUS allocated bandwidth which can result in a lower transmission power for each segment, i.e., due to PSD limitation, and less robustness against channel fading. In this other approach, ‘almost’ the whole LP-WUS bandwidth can be used to generate an FSK signal after envelope detection using low envelope IFs as illustrated in Figure 7 where only one frequency component f ∈f1, …, fm, …,f2M is generated/detected at a time. In the example illustrated in Figure 7, a signal at a target IFn is generated by first splitting the LP-WUS bandwidth into Nm segments without any guard bands in-between such that fm= (LP-WUS BW)/Nm , then a sequence (Sm) of length Lm=fm/SCS and good autocorrelation properties is repeated over the Nm segments. Note that an OOK signal (e.g., Option OOK-1) can also be generated using this approach where only one IF is turned ON and OFF throughout the transmission of the LP-WUS, i.e., no switching between low envelope IFs is performed. Further note that due to the usage of a low envelope IF, PSK modulation can be considered to increase the transmission bit rate by applying a linear phase over the N SCs allocated to the LP-WUS bandwidth.

Figure 7: An illustration of M-bit MC-FSK generation using envelope IFs.

In Figure 8, another example illustrates how a 1-bit MC-FSK signal can be generated using low-envelope IFs where a LP-WUS bandwidth of 2 RBs, 2 sequences of lengths {6, 8} SCs, and a mapping of Bit 0→180kHz, Bit 1→240kHz are considered. The two frequencies are detected by the LP-WUR after self-mixing envelope detection. In Figure 9, example details of digital baseband processing for the detection of a LP-WUS generated using envelope IF design are presented when a heterodyne receiver architecture with IF envelope detection is considered. In this example, a set of Goertzel filters are used to detect signals at specific frequency tones, e.g., corresponding to target envelope IF(s).

Figure 8: An illustration of 1-bit MC-FSK generation using two envelope IFs 180kHz, 240kHz and considering LP-WUS bandwidth of 2 RBs.

Figure 9: Example Digital Baseband Processing for Envelope IF Detection using Heterodyne Receiver Architecture.

Observation 6: The 2M frequency components of an M-bit FSK signal can be generated as low envelope IFs via the repetition of a sequence (of good autocorrelation properties and of length corresponding to the target IF) throughout the LP-WUS bandwidth.

Proposal 6: Support a design of Option OOK-1 for 1-bit MC-ASK generation where the N SCs of LP-WUS are separated into N0 segments of length L0 SCs each.
- The segment length L0=IF0/SCS for a target low envelope IF of IF0.
- The number of segments N0=N/L0. 
- Each segment is occupied by a sequence of length L0 and good autocorrelation properties.

Proposal 7: Study Option FSK-2 for M-bit MC-FSK generation based on envelope IFs where the N SCs of LP-WUS are separated into Nm, m∈1, 2, …,2M, segments of length Lm SCs each.
- The segment length Lm=fm/SCS for a target low envelope IF of fm.
- The number of segments Nn=N/Ln. 
- Each segment is occupied by a sequence of length Lm and good autocorrelation properties.

## Link-Level Simulation Results

In this section, the link-level performance of Option OOK-1, Option OOK-2, Option OOK-4, and Option FSK-2 is presented. The impacts of frequency and timing offset are evaluated where a fixed frequency offset Δf∈0, 10, 50, 100 ppm and a fixed timing offset (with respect to timing obtained from 32-bit preamble detection) ΔT∈0, 1, 2, 4 μs are considered. The impact of sampling frequency is also considered for Option OOK-4 for sampling frequency Fs∈1.12, 3.4MHz. The detailed list of assumptions in Table 1, based on the latest agreement in RAN1#112, are used for the link-level simulations unless stated otherwise.

Table 1: Link-Level Simulation Assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Waveform | Option OOK-1 Option OOK-2 Option OOK-4 Option FSK-2 |
| Channel structure | - Option 1: Sync signal /sequence (32 chips) + payload (16 bits) + CRC (8 bits) |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2b: M =4/8 for SCS = 30 kHz (same as NR signal) |
| WUS duration | 32 (Preamble) + 6/12/48 (Payload+CRC) OFDM symbols |
| Code scheme | 1/2 Manchester code for OOK Options 1/2/4 |
| gNB Channel BW | 20MHz |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - 4.32 MHz including subcarriers for guard band - 2.88MHz (i.e.{8} RBs) for LP-WUS transmission for 30kHz SCS Option 3: - 1.8 MHz including subcarriers for guard band - 1.08MHz (i.e.{3} RBs) for LP-WUS transmission for 30kHz SCS GB is symmetrically placed on each side of LP-WUS |
| Filter | 3-rd Order filter (Butterworth) with Y MHz bandwidth, - Y = 4.32 MHz for LP-WUS BW Option 1 - Y = 2.88 MHz for LP-WUS BW Option 2 - Y = 1.08 MHz for LP-WUS BW Option 3 |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; - EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline |
| Sampling Rate | 1.12 MHz |
| ADC bit width | 1-bit (Preamble), Ideal (Payload+CRC) |
| Channel Model | TDL-C, Delay Spread = 300 ns, UE velocity = 3 km/h |
| Impairment modelling | - Fixed frequency error - Timing acquisition using preamble (Sync signal) detection + Fixed timing error |

### Option OOK-1

In this section, we evaluate the link-level performance of Option OOK-1 where two approaches are used for OOK signal generation and detection. In the first approach (Conventional OOK), the OOK signal is generated by modulating all N LP-WUS SCs, i.e., OOK=1, using a length-N sequence of good autocorrelation properties. The LP-WUR detects/decodes the OOK signal by tuning to DC (i.e., 0 Hz) after envelope detection. In the second approach (envelope IF OOK), the OOK signal is generated according to description in Section 1.1.7 and formulation in  and the LP-WUR detects/decodes the OOK signal by tuning to low envelope IF (i.e., ≠0Hz) after envelope detection.

Impact of LP-WUS Bandwidth

In Figure 10, the impact of LP-WUS bandwidth is evaluated for Option OOK-1 with bandwidths Option 1, Option 2, and Option 3. The figure shows that increasing the LP-WUS bandwidth from Option 3/Option 2 (1.08MHz/2.88 MHz) to Option 1 (4.32 MHz) can help improve the performance of conventional OOK LP-WUR (i.e., detection/decoding at DC) by ~8.5dB/~2.5 dB, respectively.

Figure 10: Impact of LP-WUS bandwidth on link-level performance considering LP-WUS waveform Option OOK-1.

Observation 7: The performance of conventional OOK LP-WUR can be improve by ~8.5 dB/~2.5 dB by switching the LP-WUS bandwidth from Option 3/Option 2 (1.08 MHz/2.88 MHz) to Option 1 (4.32 MHz).

Impact of Frequency Error

In Figure 11, the impact of a fixed frequency error of Δf=100 ppm on conventional and envelope IF OOK-1 is evaluated. The figure shows that a guard band of 1 RB (at 30kHz SCS) is sufficient to mitigate the impact of a 100 ppm frequency error at 2.6 GHz carrier frequency for both conventional and envelope IF OOK-1. Figure 11 and Figure 12 also show that with proper selection of IF for OOK signal generation and detection/decoding, the performance can be improved compared to conventional OOK LP-WUR by ~1 dB/~1.5 dB at 2.6 GHz and 4 GHz carrier frequency, respectively.

Figure 11: Impact of frequency error on LP-WUS waveform Option OOK-1 performance.

Observation 8: Option OOK-1 with conventional and envelope IF generation/reception can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Further, it should be noted that, compared to conventional OOK detection at DC, signal generation based on low envelope IFs can support the generation of M-bit FSK as discussed later.

Figure 12: Comparison of Conventional and envelope IF considering LP-WUS waveform Option OOK-1 at 4 GHz carrier frequency.

Impact of Timing Error

In Figure 13, the impact of a fixed timing error of ΔT=+4 μs on conventional and envelope IF OOK-1 is evaluated. The figure shows that both conventional and envelope IF OOK-1 schemes are robust against timing errors of up to +4 μs with a performance degradation <~0.5 dB. The robustness against timing error comes from the fact that Option OOK-1 has a long OOK pulse duration (~35μs at 30 kHz SCS).

Figure 13: Impact of timing error on LP-WUS waveform Option OOK-1 performance.

Observation 9: Option OOK-1 with conventional and envelope IF generation/reception can tolerate up to 4μs timing error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Observation 10: Envelope IF signal generation/reception has ~1 dB / ~1.5 dB performance improvement over conventional OOK LP-WUR with the proper selection of IF at 2.6 GHz / 4 GHz carrier frequency.

### Option OOK-2

In this section, we evaluate the link-level performance of Option OOK-2 where M=4 frequency segments are considered and a total LP-WUS bandwidth Option 1 is split across the 4 segments with 4 SCs as guard band between the segments as shown in Figure 14. A Butterworth filter of order l∈3, 5 and bandwidth of 10.125 MHz is considered for each of the 4 segments to reject interference from neighboring segments.

Figure 14: Resource configuration for LP-WUS waveform Option OOK-2.

Impact of Frequency Error

In Figure 15, the impact of fixed frequency error Δf∈10,50 ppm on LP-WUS waveform Option OOK-2 is evaluated. The figure shows that a guard band of 4 SCs (at 30kHz SCS) between frequency segments can only tolerate a 10 ppm frequency error at 2.6 GHz carrier frequency and using a 5th order filter. A frequency error ≥50 ppm at 2.6 GHz carrier frequency or a 3rd order filter can result in a considerable interference leakage from neighboring frequency segments which can degrade LP-WUS detection performance significantly.

Figure 15: Impact of frequency error on LP-WUS waveform Option OOK-2 performance.

Observation 11: Interference leakage between Option OOK-2 frequency segments can degrade LP-WUS detection performance, ≤ 10 ppm frequency error at 2.6 GHz carrier frequency can be tolerated using 4 SCs (at 30kHz SCS) guard band between segments and a 5th order filter.

Impact of Timing Error

In Figure 16, the impact of fixed timing error ΔT∈+1, +2 μs on LP-WUS waveform Option OOK-2 is evaluated. The figure shows that LP-WUS Option OOK-2 can tolerate timing errors of up to +2 μs with a performance degradation <~1 dB. Similar to LP-WUS Option OOK-1, timing error tolerance comes from the fact that Option OOK-2 has a long OOK pulse duration (~35μs at 30 kHz SCS).

Figure 16: Impact of timing error on LP-WUS waveform Option OOK-1 performance.

Observation 12: LP-WUS waveform Option OOK-2 can tolerate up to 2μs timing error at 2.6 GHz carrier frequency and 4 SCs (at 30kHz SCS) guard band between frequency segments.

### Option OOK-4

In this section, we evaluate the link-level performance of Option OOK-4 where M∈4,8 time segments are considered and CP is handled by the LP-WUR. Two sampling frequencies Fs∈1.12, 3.4 MHz are considered and their impact is illustrated in Figure 17. The figure shows that a sampling frequency of Fs=1.12 MHz is sufficient for LP-WUS waveform Option OOK-4 when M=4 is considered, however, a sampling frequency of Fs=3.4 MHz may be required for LP-WUS waveform Option OOK-4 when M=8 is considered. The figure also shows that LP-WUS waveform Option OOK-4 with M=8 (raw data rate of 224 kbps) experiences ~2 dB loss compared to M=4 (raw data rate of 112 kbps) with proper handling of CP at LP-WUR.

Figure 17: Impact of sampling frequency on LP-WUS waveform Option OOK-4 performance.

Observation 13: A sampling frequency of 1.12 MHz may be sufficient for LP-WUS waveform Option OOK-4 with M=4 segments but a sampling frequency of 3.4 MHz may be required when M=8 segments are considered.

Observation 14: LP-WUS waveform Option OOK-4 with M=8 segments at raw data rate of 224 kbps experiences ~2 dB performance loss compared to M=4 segments at 112 kbps.

Impact of Frequency Error

In Figure 18, the impact of a fixed frequency error Δf=100 ppm on LP-WUS waveform Option OOK-4 is evaluated. The figure shows that a guard band of 1 RB (at 30kHz SCS) is sufficient to mitigate the impact of a 100 ppm frequency error at 2.6 GHz carrier frequency for the two raw data rates of 112kbps M=4 and 224kbps M=8.

Figure 18: Impact of frequency error on LP-WUS waveform Option OOK-4 performance.

Observation 15: LP-WUS waveform Option OOK-4 with M∈4, 8 segments can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Impact of Timing Error

In Figure 19, the impact of fixed timing error ΔT∈+1, +2 μs on LP-WUS waveform Option OOK-4 is evaluated. The figure shows that LP-WUS waveform Option OOK-4 can tolerate timing errors only up to +1 μs when M=4 with a performance degradation <~0.5 dB whereas a timing error of +1 μs can result in ~5 dB LP-WUS detection performance degradation when M=8 segments are considered. As opposed to LP-WUS waveform Option OOK-1 where 1 μs timing offset account for ~3% of the OOK pulse duration, it accounts for ~12.5% of the OOK pulse duration when M=4 and ~25% when M=4 for LP-WUS waveform Option OOK-4 at 30 kHz SCS.

Figure 19: Impact of timing error on LP-WUS waveform Option OOK-1 performance.

Observation 16: A timing error of 1 μs can result in ~5dB LP-WUS detection performance degradation for LP-WUS waveform Option OOK-4 with M=8 segments at 2.6 GHz carrier frequency and 30kHz SCS.

Observation 17: LP-WUS waveform Option OOK-4 with M=4 segments can only tolerate up to 1 μs timing error at 2.6 GHz carrier frequency and 30kHz SCS.

### Option FSK-2

In this section, we evaluate the link-level performance of LP-WUS waveform Option FSK-2 with M=2 bits and a signal that is generated based on the envelope IFs scheme described in Section 1.1.7.

Impact of Sampling Frequency and ADC Bit Width

Three sampling frequencies Fs∈0.96, 1.92, 3.84 MHz are considered and their impact is illustrated in Figure 20. The figure shows that the difference in LP-WUS detection performance is minimal between sampling frequencies of Fs=1.92 MHz and Fs=3.84 MHz and that detection performance degradation of only <1 dB is experienced when a sampling frequency of Fs=0.96 MHz is considered.

Figure 20: Impact of the sampling frequency on the performance of LP-WUS waveform Option FSK-2 with envelope IFs.

Observation 18: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits at information data rate of 56 kbps experiences only <1 dB performance loss using sampling frequency of 0.96 MHz compared to 3.84 MHz.

The impact of ADC bit width is also evaluated in Figure 21 where ADC bit widths ∈2, 3, 4 bits are considered. The figure shows that 3-bit and 4-bit ADCs may be sufficient to approximately achieve the performance of an ideal ADC whereas a 2-bit ADC may result in a LP-WUS decoding performance loss of ~1 dB.

Figure 21: Impact of ADC bit width on performance of LP-WUS waveform Option FSK-2 with envelope IFs.

Observation 19: An ADC bit width of 3 or 4 are sufficient for LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits at information data rate of 56 kbps to achieve close to ideal ADC performance.

Impact of Frequency Error

In Figure 22, the impact of a fixed frequency error Δf=100 ppm on LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits is evaluated. The figure shows that a guard band of 1 RB (at 30kHz SCS) is sufficient to mitigate the impact of a 100 ppm frequency error at 2.6 GHz carrier frequency.

Figure 22: Impact of the frequency error on the performance of LP-WUS waveform Option FSK-2 with envelope IFs.

Observation 20: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Impact of Timing Error

In Figure 23, the impact of fixed timing error ΔT∈+2, +4 μs on LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits is evaluated. The figure shows that LP-WUS waveform Option FSK-2 based on envelope IFs can tolerate timing errors up to +4 μs with a performance degradation ∼0.5 dB whereas a timing error of up to +2 μs can be tolerated without a degradation in LP-WUS detection performance. Similar to LP-WUS waveform Option OOK-1, 4 μs timing offset account for only ~11% of the OOK/FSK symbol duration at 30 kHz SCS.

Figure 23: Impact of the timing error on the performance of LP-WUS waveform Option FSK-2 with envelope IFs.

Observation 21: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits can tolerate up to 2 μs timing error at 2.6 GHz carrier frequency and 30kHz SCS without performance degradation and up to 4 μs with ∼0.5 dB degradation.

# Conclusion

This contribution discusses the LP-WUS potential designs, design considerations, and link level simulation results. The following summarizes our observations and proposals.

LP-WUS Design Considerations/Generation and Transmission

Observation 1: The LP-WUS can be scheduled in any time-frequency resources within the configured BWP i.e., no dedicated time-frequency resources.

Proposal 1:  The LP-WUS can carry information indicating: 
- UE, UE sub-group, or UE group ID(s)
- cell information (ID)
- System information change and/or emergency message indication(s)

Proposal 2: For message-based LP-WUS, a CRC of proper length should be supported to improve FAR and reduce its impact on power saving gain without degrading the preamble MDR.

Proposal 3: Study LP-WUS design utilizing low envelope intermediate frequencies (IFs) to mitigate the impacts of intra-/inter-cell interference.

Proposal 4: Separate and joint performance KPIs for preamble and message parts should be defined. For the preamble, at least miss-detection rate should be adopted as KPI and similarly decoding error rate for the message part.

Observation 2: A minimum sampling frequency of ~8× the LP-WUS transmission rate of 56kbps is needed for DFT-S-OFDM OOK to achieve a similar MDR performance as the OFDM-OOK transmission scheme at a sampling frequency of only ~2× the LP-WUS transmission rate under the TDL-C 300ns channel.

Observation 3: Out of M-bits LP-WUS generation Options OOK-2, FSK-1, and FSK-2, OOK-2 requires the least number of segments (M) but may require Manchester encoding, i.e., achieve half the information bit rate, for a comparable performance as the other two options.

Observation 4: For M>2, additional 2 segments for Option OOK-2 acting as ON/OFF levels reference may help achieve similar performance to Options FSK-1 and FSK-2 without information bit rate loss or significant increase in number of segments.

Observation 5: The additional 2 segments for Option OOK-2 can further be FSK modulated to carry one additional bit of information.

Proposal 5: Support the additional 2 segments for Option OOK-2 as ON/OFF levels reference with the following three alternatives:
Alt 1: the two segments carry known reference signals and have fixed location.
Alt 2: the two segments carry known reference signals and follow a frequency hopping pattern.
Alt 3: the two segments carry a signal that is FSK modulated .

Observation 6: The 2M frequency components of an M-bit FSK signal can be generated as low envelope IFs via the repetition of a sequence (of good autocorrelation properties and of length corresponding to the target IF) throughout the LP-WUS bandwidth.

Proposal 6: Support a design of Option OOK-1 for 1-bit MC-ASK generation where the N SCs of LP-WUS are separated into N0 segments of length L0 SCs each.
- The segment length L0=IF0/SCS for a target low envelope IF of IF0.
- The number of segments N0=N/L0. 
- Each segment is occupied by a sequence of length L0 and good autocorrelation properties.

Proposal 7: Study Option FSK-2 for M-bit MC-FSK generation based on envelope IFs where the N SCs of LP-WUS are separated into Nm, m∈1, 2, …,2M, segments of length Lm SCs each.
- The segment length Lm=fm/SCS for a target low envelope IF of fm.
- The number of segments Nn=N/Ln. 
- Each segment is occupied by a sequence of length Lm and good autocorrelation properties.

Link-Level Simulation Results/Observations

Option OOK-1

Observation 7: The performance of conventional OOK LP-WUR can be improve by ~8.5 dB/~2.5 dB by switching the LP-WUS bandwidth from Option 3/Option 2 (1.08 MHz/2.88 MHz) to Option 1 (4.32 MHz).

Observation 8: Option OOK-1 with conventional and envelope IF generation/reception can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Observation 9: Option OOK-1 with conventional and envelope IF generation/reception can tolerate up to 4μs timing error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Observation 10: Envelope IF signal generation/reception has ~1 dB / ~1.5 dB performance improvement over conventional OOK LP-WUR with the proper selection of IF at 2.6 GHz / 4 GHz carrier frequency.

Option OOK-2

Observation 11: Interference leakage between Option OOK-2 frequency segments can degrade LP-WUS detection performance, ≤ 10 ppm frequency error at 2.6 GHz carrier frequency can be tolerated using 4 SCs (at 30kHz SCS) guard band between segments and a 5th order filter.

Observation 12: LP-WUS waveform Option OOK-2 can tolerate up to 2μs timing error at 2.6 GHz carrier frequency and 4 SCs (at 30kHz SCS) guard band between frequency segments.

Option OOK-4

Observation 13: A sampling frequency of 1.12 MHz may be sufficient for LP-WUS waveform Option OOK-4 with M=4 segments but a sampling frequency of 3.4 MHz may be required when M=8 segments are considered.

Observation 14: LP-WUS waveform Option OOK-4 with M=8 segments at raw data rate of 224 kbps experiences ~2 dB performance loss compared to M=4 segments at 112 kbps.

Observation 15: LP-WUS waveform Option OOK-4 with M∈4, 8 segments can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Observation 16: A timing error of 1 μs can result in ~5dB LP-WUS detection performance degradation for LP-WUS waveform Option OOK-4 with M=8 segments at 2.6 GHz carrier frequency and 30kHz SCS.

Observation 17: LP-WUS waveform Option OOK-4 with M=4 segments can only tolerate up to 1 μs timing error at 2.6 GHz carrier frequency and 30kHz SCS.

Option FSK-2 (based on envelope IFs)

Observation 18: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits at information data rate of 56 kbps experiences only <1 dB performance loss using sampling frequency of 0.96 MHz compared to 3.84 MHz.

Observation 19: An ADC bit width of 3 or 4 are sufficient for LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits at information data rate of 56 kbps to achieve close to ideal ADC performance.

Observation 20: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits can tolerate up to 100 ppm frequency error at 2.6 GHz carrier frequency and 1 RB (at 30kHz SCS) guard band on each side of the LP-WUS.

Observation 21: LP-WUS waveform Option FSK-2 based on envelope IFs with M=2 bits can tolerate up to 2 μs timing error at 2.6 GHz carrier frequency and 30kHz SCS without performance degradation and up to 4 μs with ∼0.5 dB degradation.

# References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#94e, Dec. 6-17, 2021

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#97e, Sept. 12-16, 2022

R1-2208379, “Low Power WUS Receiver Architectures Considerations and Modeling”, FutureWei, RAN1#110-bis-e, October 10-19, 2022

# Appendix A: RAN1 Agreements

### RAN1#111

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

### RAN1#112

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol) Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2 Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded. Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements Agreement - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - raw information bit-size - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - FFS: false alarm probability/rate - FFS: misdetection probability/rate are kept [comparable or fixed]. Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

### RAN1#112bis-e

Agreement - Capture in TR: From RAN1 perspective, LP-WUS and signals/channels used by MR can be within the same FR1 band. - At least LP-WUS and signals/channels by MR can be on the same carrier in the band - Study further - Whether LP-WUS and signals/channels used by MR can be different carriers in the band - Details on the LP-WUS location within a carrier - Band can be different than band of signals/channels used by MR - LP-WUS association with BWP - LP-WUS can be configurable within guard-band of a band (like NB-IoT) Agreement Update the RAN1#112 agreement as the following: - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] Working assumption: In place of the above deleted bullets: - Alt 1: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands) - time/frequency resources used for LP-WUS (including any guard bands) - Alt 2: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands) - SNR is calculated as average EPRE divided by power of noise [and interference]. - Companies to report whether and how power pooling across and within MR OFDMA symbols is used. - FFS: PAPR applicable to LP-WUS Agreement Replace in RAN1#112 agreement Companies to report - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms with - receiver architecture type and its relative power consumption Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information - For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated. Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques. Agreement - Study further following alternatives to carry the LP-WUS information using: - Alt 1: by sequence(s) detection/selection - FFS sequence type - Alt 2: by encoded bits - FFS: what type of encoding scheme - FFS: with or without other bits (e.g. CRC/FCS) - Other alternatives are not precluded - Study whether LP-WUS information needs to be preceded by known one or more sequence(s). Agreement At least for IDLE/Inactive mode, at least one BW-size <=5MHz is recommended to be supported for FR1 - Other BW sizes are not precluded - if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz - LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs Agreement Study further methods to modulate input signal of the DFT/Least-Square block for OOK-4, and methods to modulate input signal of N SCs for other MC-ASK/FSK schemes - study methods with respect to - improving frequency diversity by flattening the spectrum, frequency repetition and frequency hopping - impact to dynamic range of RE power in frequency domain - FFS: impact to PAPR of generated time domain modulated MC-ASK/FSK symbol - improving robustness to timing error necessary spectrum adjustment for compatibility with CP-OFDM generation Agreement - Study techniques/mechanisms to enhance coverage performance of LP-WUS - Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc… - Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.