3GPP TSG RAN WG1 #113																     R1-2304422

Incheon, Republic of Korea, May 22nd – 26th, 2023

Agenda Item:		9.1.1.2

Source:				InterDigital, Inc.

Title:						Multiple TA for mTRP Operation

Document for:		Discussion and Decision

# Background

RAN plenary #94e approved the WID in [1] for Rel-18 MIMO enhancements. As described in WID, one of the goals in Objective 7 is to study and specify the operation related to multi-DCI multi-TRP deployment allowing for two timing advance TAs loops. In RAN1 meeting #112bis-e discussions on this topic continued. As a result, certain agreements and further discussions points were outlined in the meeting that are listed below for convenience.

Working Assumption For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY. - FFS: details of PRACH power control Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support at least RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS - RAR based - FFS: RAR-less solution reusing the solution agreed in Rel-18 Mobility Enh Agreement For intercell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order. - FFS: Whether additionalPCI or a generic identifier is indicated in PDCCH order - FFS: The detail of the indication in PDCCH order in terms of whether to support PRACH triggered for inactive additionalPCI. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP. Agreement For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, for a CFRA based PDCCH order from one TRP triggering PRACH towards another TRP, study whether and, if needed, how to determine the transmit power of the triggered PRACH preamble

# Discussion

In this contribution, we share our analysis of the remaining discussion points mentioned in the meeting minutes and make further proposals. Additionally, we will address the questions from the LS sent by RAN2 on two TAs topic.

# PDCCH order multi-TRP case

In the last meeting it was agreed to support cross TRP PRACH PDCCH order. This is a CFRA and there are two cases inter-cell and intra-cell multi-TRP that must be addressed with slightly different requirements in terms of spatial filter indication and subsequent PRACH power control.

As captured in Table 1, the DCI format 1_0 that carries a PDCCH order may carry the related information for this process. When the PDCCH order is scrambled with C-RNTI, the frequency domain allocated resources are all set to “1” and the rest of information fields are interpreted for PRACH resources.

For intra-cell, with some modification, the same DCI may be used for PDCCH RACH order. The indicated SS/PBCH index in the DCI is still valid and relevant for the UE. Further, the UE can use the same pool of RACH resources, as RACH resources are configured by the same gNB for both TRPs. However, what is not supported by the existing DCI is the information related to the configured CSI-RS as a source RS for PRACH spatial filter to be used for TRP identification for such intra-cell case.

Since there are 10 reserved bits available for future use in DCI format 1_0 scrambled with C-RNTI, some of these bits may be used as an indication of configured CSI-RS for the purpose of PRACH spatial filter determination for the intra-cell case.

| DCI Fields | Number of bits within DCI for C-RNTI PDCCH order |
| --- | --- |
| DCI format identifier | 1 |
| Frequency domain resource allocation | All bits set to ‘1” |
| Random Access Preamble Index | 6 |
| UL/SUL indicator | 1 |
| SS/PBCH index | 6 |
| PRACH Mask index | 4 |
| Reserved bits | 10 |

Table 1. DCI format 1_0 for PDCCH order

Observation 1: There are 10 reserved bits available for future use in DCI format 1_0 scrambled with C-RNTI.

The PRACH transmission power is given by the following equation:

[dBm].

The Pathloss requires the correct DL RS association that will be used to calculate the filtered RSRP according to the filter configuration provided in 38.331.

Currently, in 38.213 the PRACH pathloss estimation is given by the RS QCL “type D” with the PDCCH even when there two RS indicated in the PDCCH order, as shown in the paragraph below:

If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1, referenceSignalPower is provided by ss-PBCH-BlockPower or, if the UE is configured resources for a periodic CSI-RS reception or the PRACH transmission is associated with a link recovery procedure where a corresponding index is associated with a periodic CSI-RS configuration as described in clause 6, referenceSignalPower is obtained by ss-PBCH-BlockPower and powerControlOffsetSS where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB. If the active TCI state for the PDCCH that provides the PDCCH order includes two RS, the UE expects that one RS is configured with qcl-Type set to 'typeD' and the UE uses the one RS when applying a value provided by powerControlOffsetSS.

In both cases, inter and intra cell multi-TRP, the fact that cross TRP PRACH order is supported, there is a need to clarify the PRACH power control appropriate RS for pathloss calculation and spatial filter to follow.

Observation 2: Since cross TRP PRACH order is supported for both inter and intra cell multi-TRP, there is a need to clarify the PRACH power control appropriate RS for pathloss calculation and spatial filter.

# PDCCH order multi-TRP inter-cell case

In the RAN1 meetings #110bis-e and #112, the following agreement has been reached:

Agreement (#110b-e) For multi-DCI based inter-cell multi-TRP operation with two TA enhancement, support PRACH configuration associated with additional configured PCIs different from the PCI of the serving cell. - FFS: details Agreement (#112) Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Consequently, looking at the DCI format 1_0 for the PDCCH PRACH order and having all the PRACH configurations available per configured and additional PCI, the only missing information for the inter-cell case is the target PCI. This can be coded as a bit since we have two TRPs or the target PCI can be sent in clear.

Observation 3: For the inter-cell multi-TRP case, the only missing information from DCI format 1_0 is the target PCI.

In our opinion, a common design supporting both inter-cell and intra-cell would be the best outcome. Thus, the PRACH PDCCH order shall support the indication or correct association of the RS to be used in the PRACH power control equation for pathloss measurement and spatial filter.

Proposal 1: For the cross TRP PDCCH PRACH order use a bit for PCI target indication, while the rest of the DCI format 1_0 fields can be maintained with the current significance.

# PDCCH order multi-TRP intra-cell case

For the intra-cell multi-TRP case we don’t have a second PCI. The RS supposed to be used by UE for the pathloss calculation for the cross TRP case must be defined within the PDCCH order. For intra-cell case, if the PRACH resources are shared/partitioned, this can be used as PRACH target TRP indication. For example, if the Preamble index belongs to the second TRP configuration, while the PDCCH order has been received on the first TRP, then the SS/PBCH field can be re-interpreted as the RS configuration for the special filter and pathloss calculation and its QCL “typeD” association must be with the second TRP RSRP configured RS for RSRP measurements.

Observation 4: For intra-cell case, if the PRACH resources are shared/partitioned, the PRACH preamble index that belongs to a group or the other can be used as PRACH target TRP indication.

For the cross TRP intra-cell RS for pathloss determination and calculation, the RS configured for RSRP measurements can be used for this purpose by default.

If there is no PRACH resource partitioning, all the PRACH resources in the pool for CFRA can be used for both TRPs, then the specific target TRP indication must be used. In this case the RS indication for pathloss RSRP calculation must be indicated separately if the SSBs are not used for the beam measurements.

Observation 5: If there is no PRACH resource split, all the PRACH resources in the pool for CFRA is used for both TRPs, then a specific TRP indication must be used.

Proposal 2: Use a specific TRP indication bit for the PRACH target in the DCI format 1_0 along with the PRACH resources to be used. This bit can be even a TAG Id pointer that identifies the second TRP.

Proposal 3: If there is no PRACH resources partitions, then a TRP indication for PRACH target is needed. This can be either a bit from the 10 reserved bits from DCI format 1_0, or UL/SUL bit can be re-interpreted when multi-TRP is configured for the cell.

# Reply LS for RAN2

RAN2 sent an LS in [3] to RAN1 asking the following questions that we reproduce here for convenience:

There is general understanding in RAN2 that, in the functionality RAN1 is designing, two time-alignment timers are used for one serving cell. One timer is associated to UL towards one TRP and the other timer is associated to UL towards the other TRP. Question 1 on TAG grouping RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs. Q1a: For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs? Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Question 2 on operation Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

Regarding Questions 1a and 1b: Q1a is related to the current maximum supported TAGs (4) being insufficient to support two TAs per serving cell, possibly requiring restrictions on the association of serving cells and TAGs. As such, the answer to Q1a is related to RAN1 conclusion on Q1b. As noted in the LS, NR does not currently impose any restrictions in configuring the association of serving cells and TAGs, and to avoid introducing such restrictions it is proposed to increase the number of TAGs per cell group to 8. An impact of this agreement is the TAGID field from the existing Timing Advance Command MAC CE is currently insufficient to support 8 TAG IDs, so a new MAC CE or enhancement to the existing MAC CE will necessary to support the increased TAG ID field (which can be further discussed by RAN2).

Observation 6: Increasing the number of TAGs per cell group to 8 avoids the need to introduce restrictions on the association of serving cells and/or TRPs to a TAG, thus maintaining legacy behaviour.

Proposal 4: No restrictions are placed on the association of serving cells and/or TRPs to a TAG (as in legacy).

Proposal 5: The number of TAGs per cell group is increased to 8.

Regarding Question 2: For beam failure detection in Rel-17 multi-TRP operation, the gNB configures the UE with two sets of beam failure detection reference signals, and the UE declares beam failure for a TRP / BFD-RS set when the number of beam failure instance indications associated with the corresponding set of beam failure detection reference signals from the physical layer reaches a configured threshold before a configured timer expires. This means that R17 beam failure detection is perform independently per TRP, and the same principle should also apply for time-alignment timer expiry.

Observation 7: Beam failure detection is performed independently per TRP in Rel-17, and the same principle should also apply for time-alignment timer expiry.

Proposal 6: When the time-alignment timer associated with one of the TRPs of a serving cell expires, UL or DL operation only impacted towards that TRP.

# Conclusions

This contribution discussed the issues raised in the last meeting for intra-cell and inter-cell M-TRP operation with the two TAs. Based on the presented discussion, we make the following observations and proposals:

Observation 1: There are 10 reserved bits available for future use in DCI format 1_0 scrambled with C-RNTI.

Observation 2: Since cross TRP PRACH order is supported for both inter and intra cell multi-TRP, there is a need to clarify the PRACH power control appropriate RS for pathloss calculation and spatial filter.

Observation 3: For the inter-cell multi-TRP case, the only missing information from DCI format 1_0 is the target PCI.

Observation 4: For intra-cell case, if the PRACH resources are shared/partitioned, the PRACH preamble index that belongs to a group or the other can be used as PRACH target TRP indication.

Observation 5: If there is no PRACH resource split, all the PRACH resources in the pool for CFRA is used for both TRPs, then a specific TRP indication must be used.

Observation 6: Increasing the number of TAGs per cell group to 8 avoids the need to introduce restrictions on the association of serving cells and/or TRPs to a TAG, thus maintaining legacy behaviour.

Observation 7: Beam failure detection is performed independently per TRP in Rel-17, and the same principle should also apply for time-alignment timer expiry.

Proposal 1: For the cross TRP PDCCH PRACH order use a bit for PCI target indication, while the rest of the DCI format 1_0 fields can be maintained with the current significance.

Proposal 2: Use a specific TRP indication bit for the PRACH target in the DCI format 1_0 along with the PRACH resources to be used. This bit can be even a TAG Id pointer that identifies the second TRP.

Proposal 3: If there is no PRACH resources partitions, then a TRP indication for PRACH target is needed. This can be either a bit from the 10 reserved bits from DCI format 1_0, or UL/SUL bit can be re-interpreted when multi-TRP is configured for the cell.

Proposal 4: No restrictions are placed on the association of serving cells and/or TRPs to a TAG (as in legacy).

Proposal 5: The number of TAGs per cell group is increased to 8.

Proposal 6: When the time-alignment timer associated with one of the TRPs of a serving cell expires, UL or DL operation only impacted towards that TRP.

# REFERENCES

[1]	RP-213598, “New WID: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, December 6-17, 2021.

[2]	Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112bis-e, April 2023

[3]	R2-2304342 LS on 2TA for multi-DCI multi-TRP, Ericsson, 3GPP RAN2 Meeting #121bis-e, April 2023