3GPP TSG RAN WG1 #113			      R1-2304423

Incheon, Korea, May 22nd  – May 26th, 2023

Agenda Item:		9.1.2

Source:				InterDigital, Inc.

Title:						Remaining Details on CSI for CJT and Medium/High UE Velocities

Document for:		Discussion and Decision

# Introduction

The Rel-18 MIMO WID [1] proposes to study enhancements to the CSI measurement and reporting. The WID scope includes the following objectives on CSI enhancements:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

The WID has outlined two specific deployment scenarios for study: High Doppler, and Coherent Joint Transmission (C-JT). In the previous meeting [2], discussion continued on Objectives 1 and 4. In this contribution, we provide our views on Type-II codebook enhancements for these two scenarios.

# Background

For Rel-18, the WID is focused on CSI enhancements to the Type-II codebook. NR Rel-17 supports two types of codebooks, i.e., Type-I and Type-II codebooks. Type-I codebook is mainly designed for single-user multiple-input-multiple-output (SU-MIMO), whereas the motivation behind Type-II codebook is multi-user (MU-MIMO) operation. The structure of both codebooks, i.e., Type-I and Type-II codebooks in matrix form is as W=W1W2. For both codebooks, the wideband channel state information (CSI) is contained in the space domain (SD) matrix W1, whereas the subband CSI is captured in W2. NR supports two variants of the Type-II codebook that are supported in Rel-17. The first variant enables the UE to report multiple spatial basis vectors and linear combination coefficients from a set of quantized codebooks to approximate the channel matrix and can support up to two-layer transmissions. The second variant, an enhanced version of the Type-II codebook named Enhanced Type-II (eType-II) codebook, was introduced in Rel-16. In eType-II codebook, the maximum number of supported layers is increased to four. To prevent excessive growth of the UCI payload size, a frequency domain (FD) compression of W2 information is introduced.

However, the feedback overhead of Type-II codebooks remains significant and approximately linearly increases with the number of subbands. The issue is exacerbated with the Rel-18 scenarios as the number of reports increases with High Doppler as well as the number of TRPs in a C-JT.

# High Doppler Enhancements

Rel-18 MIMO work aims at enhancing CSI for UEs moving with high/medium velocity. Particularly, Rel-18 aims at using time/Doppler domain (DD) information to assist downlink (DL) Type-II precoding. For a medium/high velocity scenario, the CSI application windows can be chosen such that the spatial domain (SD) basis and the frequency domain basis remains the same throughout the window, but the co-phasing coefficients changes several times. Assuming fixed SD and FD basis, with time varying co-phasing coefficients the existing precoder structure of Rel-16/17 changes from W=W1W2WfH to a time series of precoders, i.e., W=W1W2WfH, where W2=[W20,W21,⋯,W2N4-1], N4 is the number of component- precoding matrix indicators (PMIs). Since the payload size of a Type-II CSI report is dominated by W2, reporting a time-series of precoders, i.e., N4 component PMIs with fixed W1 and Wf will generate a massive feedback overhead.

CSI omission

On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support reusing the legacy UCI omission mechanism with (Alt3) the following priority function: Prioλ,l,m,q=2L RI Mvq+2L RI Pm+RI l+λ where P(m) = m - Note: This implies that DD basis is designated the least priority - FFS: Details on the location of the new UCI parameters in G0/1/2

Rel-16 uses the expression 2L RI Pm+RI i+l for assigning priority value to the CSI coefficients for CSI omission, where RI is the number of layers l=1, ⋯, RI, λ=0, ⋯, 2L-1, L is the number of beams, and Pm is a rule for mapping the FD basis, m=0,⋯,N3-1. Rel-18 adds another dimension, i.e., the DD basis q=0, ⋯, Q-1 which need to be considered for CSI omission. It was further agreed on how to assign priority value to a CSI coefficient on a certain layer, beam/polarization, FD basis and DFT basis considering Rel-18 enhancements. In RAN1 112bis-e, Alt3 was agreed as a UCI omission rule where the lowest priority is given to the Doppler domain basis with a mapping rule Pm=m. The details on the location of the UCI in a CSI report are yet to be sorted out. Figure 1 depicts an exemplary priority values assignment to the CSI’s based on the priority function in Alt3, i.e., Prioλ,l,m,q=2L RI Mvq+2L RI m+RI l+λ for L=2, Q=2 and M=4. From Fig. 1, it can be observed that the highest priority (smaller Prioλ,l,m,q value) is assigned to the first FD basis on the first SD and the first DD basis, whereas the least priority (highest Prioλ,l,m,q values) is assigned to the highest FD basis on the highest SD and DD basis. Based on Alt3, the CSI priorities decreases (Prioλ,l,m,q value increases) when increasing DD basis indices. Therefore, CSI’s pertaining to smaller DD basis should be placed in group 1 and higher DD basis should be placed in group 2.

Figure 1. Exemplary priority values assignment to L=2 beams, M=4 FD basis, and Q=2 DD basis

Observation 1: CSIs reporting priorities decreases( i.e., priority values increases) when increasing DD basis.

Proposal 1: Support placing CSI’s pertaining to smaller DD basis in group 1 and higher DD basis in group 2 of a CSI report.

TDCP report

For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y, in addition to Y=1, support Y=2, 3, 4 - FFS: Whether Y=7 is also supported For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, - At least the following size-Q quantization alphabet is supported: 1-2-N-qs where q=0,1,…,2Q-1 - TBD: supported value(s) of N (e.g. 2Q-1 or a larger value), Q, s (e.g. ½, ¼, 1/8, …), whether a center threshold is also supported (and if so, higher-layer configured) - FFS: Whether different schemes can be supported for different use cases For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, down-select (by RAN1#113) from the following candidates: - Alt1: N=2Q-1 where Q=5, s={1/5, ¼, 1/3} - Alt2: N=2Q where Q=3, s={¼, 1/3, ½, 2/3, ¾} - Alt3: N=2Q where Q=4, s={¼, ½, 2/3, ¾} - Alt4: N={2Q –1, …, 2Q+1 –1} (i.e., 7-15) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} - Alt4A: N={2Q , 2Q+0.5,…, 2Q+1 - 0.5} (i.e., 8, 8.5,…,15.5) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} Once an alternative is selected, reducing the number of candidate values for s is not precluded. Companies can simulate each alternative with and without a configurable center threshold For the Rel-18 TRS-based TDCP reporting, - Support the following D (delay) values: 4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots - Working assumption: Support the following D (delay) values in a separate UE Feature Group: 6 slots, 10 slots FFS: The value of Dbasic FFS: Applicability of each D value candidate for different SCS values and/or other parameters (e.g. Y, quantization)

In our understanding, the main motivation for TDCP report is to have a correct configuration of CSI process that corresponds to the time variation of the wireless channel, e.g., a TDCP report could assist a gNB with codebook determination and SRS periodicity. The TDCP is a quantized wideband normalized amplitude/phase of the time-domain correlation profile determined using one or more delays. To estimate quantized wideband normalized amplitude/phase of the time-domain correlation, it was agreed to support using Y=1, 2, 3 and 4 delay values.

Regarding the quantization of wideband normalized amplitude values of the TDCP, a size-Q quantization alphabet, i.e., 1-2-N-qs was supported, where q=1,⋯,2Q-1 and the values N, Q and s are to be decided. In our view, the TDCP amplitude quantization range can be closer around 1, sparser around 0, and the range can vary based on the speed of the UE and the number of delay values. The quantization range of the TDCP can be adjusted using different values of s.

Observation 2: Different TDCP amplitude quantization ranges are needed for different UE speeds and delay values, where the range can be closer around 1 and sparser around 0

Proposal 2: Support Alt4A with multiple values of s for different UE speeds.

At high UE speed, the wireless channel is expected to vary at a high rate. Therefore, the lag for correlation measurement/TDCP estimation does not need to be too long. Also, longer lags could have implications on the UE memory usage by forcing it to hold samples for an unreasonably extended period of time. Therefore, a Dbasic value in the range of  1 or 2 slots should be sufficient.

Observation 3: Longer lags for TDCP may cause inefficient estimation and could cause UE complexity issues.

Proposal 3: Support defining Dbasic in the range of 1 or 2 slots.

With the current Rel-18 design, a UE is obliged to report a TDCP even if the accuracy of its measurement is erroneous. The accuracy could be impacted by low SNR, impairments, or the channel variations are fast enough such that the estimated TDCP using the basic feature is not an accurate or sufficient measure of the variations in the wireless channel. When the gNB receives the TDCP report, it cannot determine whether the TDCP report is useful or reliable. This affects the gNB’s decisions with respect to configuration because there will be a mismatch between the gNB’s perceived UE conditions, and the actual UE conditions. Therefore, from our perspective, TDCP could work efficiently if we define a UE behaviour that considers the accuracy of TDCP measurements.

Observation 4: UE is obliged to report TDCP, even if due to low SNR or other impairments, the accuracy of its measurement is erroneous and not reasonable.

Proposal 4: RAN1 studies enhancements which consider the accuracy of TDCP measurements.

One alternative is to introduce an additional indication with the TDCP report to help the gNB assess the quality of TDCP report for CSI configuration and UE/gNB prediction. For instance, a UE can determine a confidence level (CL) of prediction or quality of prediction and report it as part of a TDCP report. The CL of prediction or quality of prediction can be determined by considering a measurement, comparing the predicted CSI and actual CSI measurement, etc.

Observation 5: UE side CSI prediction may not always be accurate and therefore some indication of the CSI prediction quality is needed at the gNB.

Proposal 5: Include an indication of confidence level (CL) of the estimated TDCP in the TDCP report.

# C-JT Enhancements

FD basis selection

On the Type-II codebook refinement for CJT mTRP, for mode-1, support the use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources, i.e. (example formulation) Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where: - Wf is commonly selected across N CSI-RS resources - φn is the layer-common FD basis selection offset for CSI-RS resource n relative to a layer-common reference CSI-RS resource n with φn=0 - Therefore, (N – 1) FD basis selection offset values φnn≠n are reported - Basic feature: φn∈0,1,2,…,N3-1 - Optional feature: φn∈0,14,12…,N3-14 - FFS: UCI design details, details on n

Rel-18 Type-II CJT codebook supports two modes of operation. In Mode 1, the FD basis selection is done per TRP/TRP-group. In Mode 2, the FD basis is jointly selected across the TRPs/TRP-groups. The CJT channel consists of one aggregated channel received from multiple TRPs, so in our view one Wf as in mode 1 captures most of the compression for the aggregated channel. In mode 1, one Wf is commonly selected across N CSI-RS resources, and the FD basis Wf,n for the nth CSI-RS resource is generated as a function of Wf and phase offsets φn for secondary adjustments per TRP. The phase offset φn is chosen relative to a reference CSI-RS resource n with φn=0. The reference CSI-RS resource n is defined as the first of the N selected CSI-RS resources. A total of N-1 FD basis selection offset values, i.e., ϕn for n=1, ⋯, N-1 and n≠n needs to be reported. Phase offset values, i.e., ϕn∈{0,1,⋯,N3-1} is supported as a basic feature and ϕn∈{0,1/4, 1/2,⋯,N3-1/4} is supported as an optional feature. UCI design of ϕn and its placement in a CSI report are the remaining issues. The phase offsets are used to capture the phase variations in the signals received from the different TRPs so they are essential to guarantee an accurate co-phasing at the receiver. Since the throughput performance directly depends on the phase offset value ϕn, its reporting should be treated as a high priority UCI within part 2 of a CSI report. Therefore ϕn can be placed in group 0 of a CSI report.

Observation 6: The phase offsets are used to capture the phase variations in the signals received from the different TRPs so they are essential to guarantee an accurate co-phasing at the receiver.

Proposal 6: Support reporting phase offset values as a high priority UCI with in part 2 of a CSI report, i.e., place ϕn in group 0.

# Conclusion

In this contributions, we provided our views regarding CSI enhancements for Rel-18 MIMO. Based on the presented discussion, we make the following observations and proposals.

CSI Enhancement for High Doppler

Observation 1: CSIs reporting priorities decreases, i.e., priority values increases when increasing DD basis.

Observation 2: Different TDCP amplitude quantization ranges are needed for different UE speeds and delay values, where the range can be closer around 1 and sparser around 0

Observation 3: Longer lags for TDCP may cause inefficient estimation and could cause UE complexity issues.

Observation 4: UE is obliged to report TDCP, even if due to low SNR or other impairments, the accuracy of its measurement is erroneous and not reasonable.

Observation 5: UE side CSI prediction may not always be accurate and therefore some indication of the CSI prediction quality is needed at the gNB.

Proposal 1: Support placing CSI’s pertaining to smaller DD basis in group 1 and higher DD basis in group 2 of a CSI report.

Proposal 2: Support Alt4A with multiple values of s, for different UE speeds.

Proposal 3: Support defining Dbasic in the range of 1 or 2 slots.

Proposal 4: RAN1 studies enhancements which consider the accuracy of TDCP measurements.

Proposal 5: Include an indication of confidence level (CL) of the estimated TDCP in the TDCP report.

CSI Enhancement for CJT

Observation 6: The phase offsets are used to capture the phase variations in the signals received from the different TRPs so they are essential to guarantee an accurate co-phasing at the receiver.

Proposal 6: Support reporting phase offset values as a high priority UCI with in part 2 of a CSI report, i.e., place ϕn in group 0.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN Meeting #94e, December 2021

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112b-e, April 2023