3GPP TSG RAN WG1 #113                      	R1-2304469

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	vivo

Title:	Further discussion on enabling 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Introduction

In this contribution we discuss details of partial coherent precoders for both Ng=2 and 4, also provide precoders for non-coherent codebook subset. We also discuss TPMI indexing for full-coherent precoders.

Discussion

Codebook-based transmission

Antenna port indexing

Agreement

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7}

- Note: Other alternatives which are not foreseen are not precluded

Following alternatives on antenna port indexing for Ng=2 and 4 were agreed in previous meeting.

A UE with higher coherence capability can be configured with codebook subset containing lower coherent precoders. For example, according to current spec, a full-coherent capable UE can be configured with PartialNon-coherent codebook subset, and a partial-coherent capable UE can be configured with Non-coherent codebook subset. Similarly, for 8Tx, a UE capable of full-coherent transmission can be configured with partial-coherent codebook subset etc. As SRS ports are one to one mapped with PUSCH ports, a common design of antenna ports mapping for full, partial coherent with Ng=2 and Ng=4 is preferred. Antenna port mapping shown in figure 1 below shall be supported,

0

1

4

5

2

3

6

7

antenna port mapping

For full-coherent UE, antenna layout in figure 1 corresponds to N1=4, N2=1 where DFT precoders are applied on antenna port 0, 1, 2 and 3, and 4, 5, 6 and 7.

For partial-coherent UE, Ng=2, there are two coherent antenna groups {0, 4, 1, 5} and {2, 6, 3, 7}.

For partial-coherent UE, Ng=4, there are four coherent antenna groups {0, 4}, {1, 5}, {2, 6} and {3, 7}.

With this antenna ports mapping convention, full and partial-coherent precoders can be designed. Hence, we support alt 2 for Ng=2 and alt1 for Ng=4.

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

Codebook configuration

Agreement

For codebook -based 8TX PUSCH transmission, down-select from,

- Alt1

  - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more  Ng cases, i.e., Ng= 4, 8

    - FFS which combinations of Ng value(s), if any, to be considered

  - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8

- Alt2

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8

    - FFS which Ng value(s), to be considered

  - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8

  - FFS whether/how the configuration can be done via RRC or MAC-CE.

- Alt3

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1

  - A partially-coherent UE , with Ng =2, can only use precoders considered for Ng =2

  - A partially-coherent UE , with Ng =4, can only use precoders considered for Ng =4

  - A non-coherent UE , with Ng =8, can only use precoders considered for Ng = 8

- Other alternatives are not precluded

Note: For an 8TX UE, Ng =8 can represent a non-coherent UE.

Following 3 alternatives on codebook configuration were agreed in RAN1#112b-e.

With alt1, if the precoders from only one Ng value are configured, then it could be same as either alt2 or alt3. If precoders from more than one Ng values are configured, then the question is whether all the precoders from all Ng values are configured or further precoders selection is to be considered.  If all the precoders from all Ng values are included in the codebook, then codebook size will be very big especially for full-coherent codebook. If further precoders selection is to be done, then it takes huge effort to agree on a common set of precoders, which could jeopardize the completion of WI.

On the other hand, alt3 is too restrictive. For some reason, if gNB wants to configure codebook subset with lower coherence precoders to a higher capable UE should be allowed.

In this sense, alt2 provides the balanced approach, a UE with higher coherence capability can be configured with codebook subset with lower coherence precoders. Whether MAC CE can switch the codebook subset can be further discussed.

For codebook -based 8TX PUSCH transmission, support alt2.

- Alt2

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8

    - FFS which Ng value(s), to be considered

  - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8

  - FFS whether/how the configuration can be done via RRC or MAC-CE.

Full-coherent codebook

Conclusion

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), there is no consensus to support any optional over-sampling ratio.

Following conclusion is made in RAN1#112b-e, which means the full coherent precoding matrices are ready for both {N1, N2} = {2,2} and {4,1} cases.

Now, the remaining issue is how to indicate the TPMI for full coherent precoders. The precoders for DL Type I codebook are indexed using parameters i1,1, i1,2, i1,3, i2. And, for UL MIMO in current spec, the TPMI jointly indicates transmission rank and the corresponding precoder.

The precoders can be indexed starting from rank1 then followed by precoders from higher rank values. Within a specific rank value, the precoders can be first indexed according to the parameters i1,1 then i1,2 then i1,3 then i2 or the sequence of precoders can be indexed according to the parameters i1,1 then i1,2 then i2 then i1,3.

The TPMI for full-coherent precoders are indexed starting from rank 1 precoders then followed by precoders from higher rank values. Within a specific rank value, the precoders can be first indexed according to the parameters i1,1 then i1,2 then i1,3 then i2 or the sequence of precoders can be indexed according to the parameters i1,1 then i1,2 then i2 then i1,3.

Partial-coherent codebook

For rank>4, following agreement has been made in previous meeting on codeword to layer mapping:

If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission.

However, precoding matrix to layer mapping is yet to be discussed, that means, for dual CW transmission, whether one CW is transmitted from one antenna group or both antenna groups. Below, we provide simulation results comparing following two alternatives:

Alt1: for rank>4, one CW is transmitted from one antenna group

Atl2: for rank>4, one CW is transmitted from both antenna groups

In the figure 2, two codebook structures are considered for two alternatives. For Alt1, diagonal codebook structure is assumed, where all layers from one CW is mapped to one antenna group. For Alt2 cyclic codebook structure is assumed, where every other layer is mapped to other antenna group, for example in the case rank=5, layers 1,3,5 are mapped to one antenna group and layers 2, 4 are mapped to another antenna group.

performance evaluation of cyclic and diagonal precoder structure

From the evaluation results, we observe there is virtually no difference in performance between two codebook structures. Considering simple codebook structure, it beneficial to keep diagonal codebook structure. Hence, we have following proposal

for rank>4, if dual CW is supported, support one CW is transmitted from one antenna group

      - Partial coherent codebook design for Ng=2

Working Assumption

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

Following working assumption was agreed in RAN1#112b-e, with a FFS bullet on whether all layer for each CW is mapped to only antenna group.

As shown in section 2.1.4, there is no benefit of spreading layers from a CW to different antenna groups. Hence, we propose to confirm the above working assumption with following revision:

Confirm the working assumption, including the table, with following revision

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group

For partial-coherent UEs, simply concatenating legacy 4 Tx codebook by indicating two 4Tx precoders in DCI (using 2 TPMI fields) works well to indicate 8Tx precoder without new design. Reusing Rel-17 signaling design will simplify the work and specification effort. If a single TPMI field in DCI is to be considered, precoders for each rank should be carefully designed in order not to compromise the performance.

Option1: two TPMI fields in DCI

Precoder design and indication for Rank<=4

Rank=1~4 precoders can be constructed by using legacy 4Tx rank=1 and 2 full-coherent precoders.

There are 16 rank=1 precoders and 8 rank=2 precoders, hence total 24 precoders in the codebook subset. Two TPMI fields in DCI are used to indicate overall 8Tx precoders, each TPMI field is 5 bits in this example. To further reduce the TPMI size, for example, 8 rank=1 precoders can be restricted, leading to total 16 precoders in the subset and each TPMI field is 4bits. Furthermore, there is third field in DCI (2bits), similar to SRS resource set indicator. This field can indicate effective TPMI field and/or antenna group mapping scheme for rank up to 4.

Precoder design and indication for Rank>4

According to current working assumption, 2 CWs are used when transmission rank is larger than 4, and same principle as in DL is used for enabling/disabling of the second CW. When the second CW is enabled by indicated DCI, that means transmission rank is larger than 4, hence separate codebook subset can be used which includes legacy 4Tx rank=2, 3, 4 full-coherent precoders only.

There are 8 rank=2 precoders, 4 rank=3 precoders and 2 rank=4 precoders, hence total 14 precoders in the codebook subset. Two TPMI fields in DCI are used to indicate overall 8Tx precoders, each TPMI field is 4 bits in this example.

Option2: one TPMI field in DCI

Although there are ways to reduce the overhead with “two TPMI fields in DCI”, we also provide precoders design for a 8Tx codebook. As it has been agreed to use 4Tx full-coherent precoders to construct a 8Tx codebook, following precoders for rank 1~8 should be supported.

At least following precoders shall be supported for Ng=2 partial coherent 8Tx codebook.

Rank =1, Ng=2 partial-coherent 8Tx precoders

11001100，1100jj00，1100-1-100， 1100-j-j00，1j001j00，1j00j-100，1j00-1-j00，1j00-j100，1-1001-100，1-100j-j00，1-100-1100，1-100-jj00，1-j001-j00，1-j00j100，1-j00-1j00，1-j00-j-100；

00110011，001100jj，001100-1-1， 001100-j-j，001j001j，001j00j-1，001j00-1-j，001j00-j1，001-1001-1，001-100j-j，001-100-11，001-100-jj，001-j001-j，001-j00j1，001-j00-1j，001-j00-j-1

Rank =2, Ng=2 partial-coherent 8Tx precoders

11001100001100-j-j, 11001100001100jj，1100jj00001100-1-1, 1100jj0000110011，1j001j00001j00j-1, 1j001j00001j00-j1 , 1j00j-100001j001j，1j00j-100001j00-1-j，1-1001-100001-100j-j , 1-1001-100001-100-jj，1-100j-j00001-100-11, 1-100j-j00001-1001-1，1-j001-j00001-j00j1，1-j001-j00001-j00-j-1, 1-j00j100001-j001-j，1-j00j100001-j00-1-j

Rank =3, Ng=2 partial-coherent 8Tx precoders

110011001100-1-100001100-j-j， 1100jj001100-j-j00001j00-1-j，1j00j-1001j00-j100001-100-11，1j00j-1001j00-j100001-100-11， 1-100j-j001-100-jj00001-j00-1j，1-j001-j001-j00-1j00001-j00-j-1，1-j00j1001-j00-j-100001100-1-1

Rank =4, Ng=2 partial-coherent 8Tx precoders

110011001-1001-100001100-1-1001-100-11，1j001j001-j001-j00001j00-1-j001-j00-1j，1100jj001-100j-j00001100-j-j001-100-jj，1j00j-1001-j00j-100001j00-j1001-j00-j-1

Rank =5, Ng=2 partial-coherent 8Tx precoders

110011001100-1-100 001-1001-1001-100-1100110011 ， 1100jj001100-j-j00 001-1001-1001-100-1100110011

Rank =6, Ng=2 partial-coherent 8Tx precoders

110011001100-1-1001-1001-100001-100-1100110011001100-1-1,  1100jj001100-j-j001-1001-100001-100-1100110011001100-1-1,  1j001j001j00-1-j001-j001-j00001-j00-1j001j001j001j00-1-j,   1j00j-1001j00-j1001-j001-j00001-j00-1j001j001j001j00-1-j

Rank =7, Ng=2 partial-coherent 8Tx precoders

110011001100-1-1001-1001-10000110011001100-1-1001-1001-1001-100-11， 1100jj001100-j-j001-100j-j0000110011001100-1-1001-1001-1001-100-11

Rank =8, Ng=2 partial-coherent 8Tx precoders

1100jj001-100j-j001100-j-j001-100-jj00001100jj001-100j-j001100-j-j001-100-jj，110011001-1001-1001100-1-1001-100-110000110011001-1001-1001100-1-1001-100-11

      - Partial coherent codebook design for Ng=4

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where

- Precoding design is based on Rel-15 UL 2TX codebook,

  - Full-coherent precoders are used

- Further study codebook size reduction

Following agreement on 8Tx codebook with Ng=4 was agreed in RAN1#112b-e,

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- The following rank and layer splitting cases are supported,

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups |
| --- | --- | --- |
| 1 | (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1) |  |
| 2 | (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2) |  |
| 2 |  | Transmission by 2 of the 4 antenna groups: (1,1,0,0), (1,0,1,0), (1,0,0,1) (0,1,1,0), (0,1,0,1), (0,0,1,1) |
| 4 |  | (1,1,1,1) |
| 4 |  | Transmission by 2 of the 4 antenna groups: (2,2,0,0), (2,0,2,0), (2,0,0,2) (0,2,2,0), (0,2,0,2), (0,0,2,2) |
| 8 |  | (2, 2, 2, 2) |

Note: Above is not relevant to how precoders are indicated.

Furthermore, following layers split for rank=1,2,4, and 8 were agreed in RAN1#112b-e.

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- In addition to the previously agreed cases, down-select from the rank and layer splitting cases listed below

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 4 |  | Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1) |
| 5 |  | Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2 |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2) |

Furthermore, a big list of layers split for rank3~7 was agreed in RAN1#112b-e for down selection.

Ng=4 partial-coherent codebook size will be huge if all the combinations in the above table are supported.

In our view, for rank=3, transmission from 3 of 4 antenna groups should be sufficient. For rank =4, transmission from 3 of 4 antenna groups doesn’t provide gain, since transmission from 2 and 4 groups has been agreed. For rank=5, there are too many combinations for transmission from 3 antenna groups, hence we support transmission from 4 antenna groups only. Similarly, for rank=6, there are too many combinations for transmission by 4 antenna groups. Hence, we propose to consider the list below and further down select if necessary.

- For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- In addition to the previously agreed cases, support following cases

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 5 |  | Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2) |

- Further down select among the layer split combinations for rank 1~8, if necessary

Considering above layers split and 2Tx full-coherent rank=1 and rank=2 precoders to construct 8Tx precoders for Ng=4 , we provide following set of precoders for each rank value,

At these precoders shall be supported.

Rank =1, Ng=4 partial-coherent 8Tx precoders

| 10001000 | 1000-1000 | 1000j000 | 1000-j000 | 01000100 | 01000-100 | 01000j00 | 01000-j00 | 00100010 | 001000-10 | 001000j0 | 001000-j0 | 00010001 | 0001000-1 | 0001000j | 0001000-j |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Rank =2, Ng=4 partial-coherent 8Tx precoders

| 1000100001000-j00 | 1000100001000j00 | 1000-j00001000100 | 1000-j00001000-100 | 1000-100001000-j00 | 1000-100001000j00 | 1000j00001000100 | 1000j00001000-100 | 10100000010-j0000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Rank =3, Ng=4 partial-coherent 8Tx precoders

| 1000100001000-j00001000-10 | 1000100001000j00001000-10 | 1000-j00001000100001000j0 | 1000-j00001000-100001000j0 | 1000-100001000-j0000100010 | 1000-100001000j0000100010 | 1000j00001000100001000-j0 | 1000j00001000-100001000-j0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Rank =4, Ng=4 partial-coherent 8Tx precoders

| 1000100001000-j00001000-100001000j | 1000100001000j00001000-100001000-j | 1000-j00001000100001000j00001000-1 | 1000-j00001000-100001000j000010001 | 1000-100001000-j00001000100001000j | 1000-100001000j00001000100001000-j | 1000j00001000100001000-j00001000-1 | 1000j00001000-100001000-j000010001 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Rank =5, Ng=4 partial-coherent 8Tx precoders

| 100010001000-10000100010001000-10000100010 | 1000j0001000-j00001000j0001000-j00001000j0 | 100010001000-10000100010001000-100001000j0 |
| --- | --- | --- |

Rank =6, Ng=4 partial-coherent 8Tx precoders

| 100010001000-10000100010001000-10000100010001000-10 | 1000j0001000-j00001000j0001000-j00001000j0001000-j0 | 100010001000-10000100010001000-100001000j0001000-j0 |
| --- | --- | --- |

Rank =7, Ng=4 partial-coherent 8Tx precoders

| 100010001000-10000100010001000-10000100010001000-1000010001 | 1000j0001000-j00001000j0001000-j00001000j0001000-j00001000j | 100010001000-10000100010001000-100001000j0001000-j00001000j | 10100000 10-10000001010000010-1000000001010000010-1000000101 | 10j00000 10-j00000010j0000010-j0000000010j0000010-j00000010j | 10100000 10-10000001010000010-10000000010j0000010-j00000010j |
| --- | --- | --- | --- | --- | --- |

Rank =8, Ng=4 partial-coherent 8Tx precoders

| 100010001000-10000100010001000-10000100010001000-10000100010001000-1 | 1000j0001000-j00001000j0001000-j00001000j0001000-j00001000j0001000-j | 100010001000-10000100010001000-100001000j0001000-j00001000j0001000-j | 10100000 10-10000001010000010-1000000001010000010-10000001010000010-1 | 10j00000 10-j00000010j0000010-j0000000010j0000010-j00000010j0000010-j | 10100000 10-10000001010000010-10000000010j0000010-j00000010j0000010-j |
| --- | --- | --- | --- | --- | --- |

Non-coherent codebook

For non-coherent codebook (precoders), it doesn’t matter what is the value of Ng, or it can be considered as Ng=8 for 8Tx, as the precoders are port selection matrices. There could be two different approaches for TPMI indication, one TPMI field and two TPMI fields in DCI

Agreement

For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission.

10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001

with the scaling factor of 122.

Following precoders for rank=1 were agreed in RAN1#112.

Furthermore, following agreements were reached in RAN1#112b-e:

Agreement

For non-coherent uplink precoding with rank≤8 by an 8TX UE, down-select from

- Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported

- Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders

It was discussed whether all 255 combination from 8 non-coherent rank1 precoders shall be supported. From TPMI indication point of view, it could be similar to SRI indication in non-codebook based since all precoders are antenna selection precoders whereas SRI in non-codebook is the selection of single port SRS resources. One could argue that there is no rationale in supporting 255 precoders for non-coherent subset while only 128 precoders are supported for full-coherent subset. If a subset of all combinations is considered at least following precoders for rank2~8 shall be supported

- At least support following precoders for rank2~8

Rank=2 non-coherent precoders:

1000000001000000, 0100000000100000, 0010000000010000, 0001000000001000, 0000100000000100, 0000010000000010, 0000001000000001, 1000000000100000, 0100000000010000, 0010000000001000, 0001000000000100, 0000100000000010, 0000010000000001, 1000000000010000，0100000000001000，0010000000000100，0001000000000010，0000100000000001 1000000000001000, 0100000000000100, 0010000000000010, 0001000000000001, 1000000000000100, 1000000000000010, 1000000000000001, 0100000000000001

Rank=3 non-coherent precoders:

100000000100000000100000, 010000000010000000010000, 001000000001000000001000, 000100000000100000000100, 000010000000010000000010, 000001000000001000000001, 100000000010000000001000, 010000000001000000000100, 001000000000100000000010, 000100000000010000000001, 100000000001000000000010, 010000000000100000000001

Rank=4 non-coherent precoders:

10000000010000000010000000010000, 01000000001000000001000000001000, 00100000000100000000100000000100, 00010000000010000000010000000010, 00001000000001000000001000000001, 10000000010000000001000000001000, 01000000001000000000100000000100, 00100000000100000000010000000010, 00010000000010000000001000000001,  10000000001000000000100000000010, 01000000000100000000010000000001

Rank=5 non-coherent precoders:

1000000001000000001000000001000000001000, 0100000000100000000100000000100000000100, 0010000000010000000010000000010000000010, 0001000000001000000001000000001000000001, 1000000001000000000001000000001000000001, 1000000001000000001000000000001000000001, 1000000001000000000100000000100000000001, 1000000000100000000100000000001000000001

Rank=6 non-coherent precoders:

100000000100000000100000000100000000100000000100, 010000000010000000010000000010000000010000000010, 001000000001000000001000000001000000001000000001, 100000000100000000010000000010000000001000000001, 100000000100000000100000000001000000001000000001，010000000010000000010000000001000000001000000001

Rank=7 non-coherent precoders:

10000000010000000010000000010000000010000000010000000010

Rank=8 non-coherent precoders:

1000000001000000001000000001000000001000000001000000001000000001

Two TPMI fields in DCI

It would be straightforward for non-coherent codebook subset constructed by simply concatenating current 4Tx codebook subsets and UL 8Tx precoder is indicated using 2 TPMI fields in the DCI. This will also allow a common design with simultaneous 2 panels transmission. For the 8Tx UEs reporting non-coherent capability, only non-coherent 4Tx precoders are applicable, hence gNB shall configure 4Tx non-coherent codebook subset for such UEs. Total transmission rank is determined by combined rank indicated by 2 TPMI fields. For transmission rank=1, how to indicate/select one the of 2 TPMI fields should be further discussed. One way is to reuse Rel-17 mechanism by using additional field in DCI to indicate/switch one or both TPMIs.

Support indicating 8Tx non-coherent precoder by 2 TPMI fields in DCI, further study how to select one of the 2 TPMI fields, e.g. for rank=1 transmission

Dual CW enabled

Agreement

For 8TX UE supporting dual CW PUSCH (Maximum number of layers configured for the UE is larger than 4)

- Alt1 – DL principle is reused for disabling transmission of a transport block, where

- The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding TB

- The enabled transport block is mapped to the first CW.

- Note: When the transmission of a transport block is disabled, the number of layers is ≤ 4.

- Note: the first CW refers to the enabled CW.

Following agreement dual CW enabling/disabling was made in RAN1#112b-e.

For 8Tx UE supporting dual CW PUSCH, it can be observed that the UE can determine whether the scheduled rank is higher than 4 or not by indicated values of IMCS and rvid, which enables or disables the second CW. In this sense, the precoders can be separated into two subsets, one subset includes rank1~4 precoders and another subset includes rank 5~6 precoders. Total number of precoders for rank1~4 is larger than the total number of precoder for rank 5~6, which means that when dual CW is enabled, if only rank5~8 are indicated by TPMI field, there is redundant bit in the TPMI. This redundant bit can be used as NDI bit for the second CW, which saves the DCI overhead.

for 8Tx UE supporting dual CW, when the second CW is enabled, the redundant bit in TPMI field is used as NDI bit.

UL full power transmission

Agreement

Framework for full power PUSCH transmission by an 8TX UE

- To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used.

  - FFS if any change is required in the specifications.

- Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used.

  - FFS if more than one of the 8TX full coherent precoders is used per rank.

- Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used.

  - FFS definition of precoder groups (G0, G1, …)

  - FFS enhancements for SRS configuration

Following agreements were reached in RAN1#112b-e:

For CPE/FWA type of devices depending on UE capability it can be assumed that full-power mode0 is supported, on the other hand for the devices supporting full-coherent precoders only (such as Rel-15 DL type 1 codebooks), full power modes are irrelevant. Full-power mode1 can be supported by introducing non-antenna selection precoders in non-coherent and partial-coherent codebook subsets, which is less dependent on UE PA architectures. Full-power mode2, especially precoder group indication, depends on UE PA architectures.

Consider the following full power enhancement for CPE/FWA 8 Tx operation.

  - Depending on UE capability, UL full-power mode1 can be supported by introducing non-antenna selection matrices

    - For Ng=2 partial-coherent case, only full power transmission precoder for rank=1 is needed

    - For Ng=4 partial-coherent case, only full power transmission precoders for rank=1, 2 and 3 are needed

  - Depending on UE capability, further discuss UL full-power mode2 for partial and non-coherent UEs

    - For partial-coherent codebook, take Ng values {2, 4} into account for full-power precoders grouping

Other issues

Other related specification enhancement needed to support 8Tx UL are configuration and signaling of SRS, DMRS, PTRS.

Details on SRS, DMRS configuration/indication are discussed in [4][3], for DMRS port indication the entries for DL 8Tx transmission can be taken as starting point.

In current spec, maximum of 2 ports PTRS is supported, which is configured by RRC. And, association of PTRS port to DMRS port is indicated in DCI. For the case of supported layers >4, PTRS to DMRS association table shall be extended.

Further discuss PTRS-DMRS association indication when rank>4, if supported

Conclusions

In this contribution, we provided details on 8Tx partial-coherent and non-coherent codebooks and other relevant issues. We also provided evaluation results showing performance of two codebook structures for partial coherent UE. We have the following proposals:

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

For codebook -based 8TX PUSCH transmission, support alt2.

- Alt2

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8

    - FFS which Ng value(s), to be considered

  - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8

  - FFS whether/how the configuration can be done via RRC or MAC-CE.

- The TPMI for full-coherent precoders are indexed starting from rank 1 precoders then followed by precoders from higher rank values. Within a specific rank value, the precoders can be first indexed according to the parameters i1,1 then i1,2 then i1,3 then i2 or the sequence of precoders can be indexed according to the parameters i1,1 then i1,2 then i2 then i1,3.

- for rank>4, if dual CW is supported, support one CW is transmitted from one antenna group

- Confirm the working assumption, including the table, with following revision

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group

At least support precoders listed in section 2.1.4.1 for Ng=2 partial coherent 8Tx codebook.

- For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- In addition to the previously agreed cases, support following cases

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 5 |  | Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2) |

- Further down select among the layer split combinations for rank 1~8, if necessary

At least support precoders listed in section 2.1.4.2 for Ng=4 partial coherent 8Tx codebook.

At least support precoders listed in section 2.1.5 for non coherent 8Tx codebook.

- Support indicating 8Tx non-coherent precoder by 2 TPMI fields in DCI, further study how to select one of the 2 TPMI fields, e.g. for rank=1 transmission

- for 8Tx UE supporting dual CW, when the second CW is enabled, the redundant bit in TPMI field is used as NDI bit.

- Consider the following full power enhancement for CPE/FWA 8 Tx operation.

  - Depending on UE capability, UL full-power mode1 can be supported by introducing non-antenna selection matrices

    - For Ng=2 partial-coherent case, only full power transmission precoder for rank=1 is needed

    - For Ng=4 partial-coherent case, only full power transmission precoders for rank=1, 2 and 3 are needed

  - Depending on UE capability, further discuss UL full-power mode2 for partial and non-coherent UEs

    - For partial-coherent codebook, take Ng values {2, 4} into account for full-power precoders grouping

- Further discuss PTRS-DMRS association indication when rank>4, if supported

- Appendix

Table A1: simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS, 30 KHz |
| Scenario | Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h |
| Channel model | 38.901 |
| System bandwidth | 20 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | Outdoor FWA : (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| gNB antenna radiation pattern parameters | Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | (1,4,2,1,1,1,4) with (𝑑H, 𝑑V) = (0.5, 0.5)𝜆 |
| Traffic model | Full buffer (optional) |
| Suggested benchmarking | R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | 32 dBm (FWA, 38.101) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Table A2: Antenna layouts

| Case | Ng | (M, N, P) per group | Antenna Layout | Antenna Pattern/Antenna Element Gain |
| --- | --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2), (1, 4, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 2 | 2 | (1, 2, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 3 | 4 | (1, 1, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial |