3GPP TSG RAN WG1 #113                                                             R1-2304473

Incheon, Korea, May 22nd – May 26th, 2023

Source:	vivo

Title:	Evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

Introduction

- In RAN 94-e meeting [1], a new SID on artificial intelligence (AI) / machine learning (ML) for multiple use cases was approved considering aspects such as performance, complexity, and potential specification impact. In this contribution, we focus on the use case of beam management, including spatial domain beam prediction and time domain beam prediction. Simulation results, corresponding comparisons and observations are provided to verify the rationality and validity of the proposed beam management enhancements based on artificial intelligence (AI) / machine learning (ML).

KPI discussion

Clarification on Rx assumption for label data in DL Tx beam prediction

In RAN1#112bis meeting, following agreement was approved for DL Rx beam assumption for Set B,

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

  - Companies report how to select the “best” Rx beam(s)

- Option 2: Measurements of specific Rx beam(s)

  - Companies report how to select specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

Other options are not precluded and can be reported by companies.

The purpose of this agreement is to define the assumption of Rx beam for providing input for AI/ML model for training and/or inference. Since the AI/ML model can only predict Tx beam for DL Tx beam prediction, it is necessary to determine the Rx beam assumption for providing label data for KPI calculation, and different Rx beam assumptions for label data may result in different performance. This is an issue common for BM Case 1 and Case 2. For BM case 2, it particularly means what Rx beam is used for calculating KPI in T2 time duration.

Figure 2.1-1 Rx assumption for label data in DL Tx beam prediction

In above figure, model input includes 8 L1-RSRP values with the best Rx beam assumption searched from the best Tx beam within Set B, and its output includes L1-RSRP values for 32 Tx beams. However, when calculating KPIs, it is necessary to determine which Rx beam assumption for label data is used. Currently, only Top-1 genie-aided Tx beam is defined as the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams corresponding to the best Rx beam searched from the best Tx beam within total 256 beam pairs for baseline.

For example, if Rx beam-8 is the best Rx beam that results in the largest L1-RSRP over all Tx and Rx beams, it is possible to use the model output results and the 32 Tx beams results with Rx beam-8 for Top-1 related KPI calculation. However, when it comes to calculating the top-k beams, there is no definition for Rx assumption. Therefore, we suggest using the same Rx assumption in Top-1 beams for calculating the top-k beams as well.

- For DL Tx beam prediction, the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1[/K] predicted Tx beam and the ideal L1-RSRP of the same Tx beam with the best Rx beam that results in the largest L1-RSRP over all Tx and Rx beams.

Rx/Set B pattern assumption for beam pair prediction

Three Set B assumptions may be used for input of beam pair prediction,

Alt1: down sample in Tx beams only (measurements from all Rx beams)

Alt2: down sample in Tx beams and in Rx beams (same Rx beams for each Tx beam)

Alt3: comb-like structure for both Tx beams and Rx beams (different Rx beam(s) for each Tx beam)

From our perspective, beams in Set B for beam pair prediction should be selected from all beam pairs, the different options above can be understood as optimization selection methods for Set B pattern. It is foreseeable that if only one Rx is used in Set B for beam pair prediction, the performance should be poor, and if continuous beams in Set B is used, the performance will not be good either. Thus, how to choose patterns within Set B should be implementation of AI/ML model owner, and no need to be discussed.

- Rx/Set B pattern assumption for beam pair prediction should be implementation of AI/ML model owner, and no need to be discussed. Companies can report the used pattern assumption for EVM.

Remaining issues on evaluation Methodologies

Different output label

In RAN1#112bis meeting, one agreement was approved to further study performance with different types of label,

Agreement

- For AI/ML in beam management, further study performance with different types of label, considering the following:

  - Option 1a: Top-1 beam(pair) in Set A

  - Option 1b: Top-K beam (pair)s in Set A

  - Option 2a: L1-RSRPs per beam of all the beams(pairs) in Set A

  - Option 2b: Top-K beam(pair)s in Set A and the corresponding L1-RSRPs

  - Option 2c: Top-1 beam(pair) in Set A and the corresponding L1-RSRP

  - Other options are not precluded and can be reported by companies.

In relative evaluation section, we will conduct some simulation evaluations for option 1a and option 2a, which will serve as preliminary results for performance comparison of classification and regression models. However, in reality, the usage of classification and regression model differs, and theoretically, regression model is expected to produce more outcomes than classification models. In beam prediction use-cases, classification model is mainly used to obtain beam prediction accuracy while regression model can extra provide beam quality information. From our perspective, conducting a direct performance comparison between classification and regression models would be unfair and unreasonable.

Besides, based on our simulation results, the performance of option 1a is worse than that of option 2a. Thus, we propose to compare results of different labels under the same model type.

As classification model, i.e. option 1a/1b, is mainly used to obtain beam prediction accuracy while regression model, i.e. option 2a/2b/2c, can extra provide beam quality information, conducting a direct performance comparison between classification and regression models would be unfair and unreasonable.

- Prefer to compare results of different labels under same model type.

Quantization/Measurement error

In RAN1#112 meeting, one agreement was approved for evaluating quantization error,

Agreement

- Further study the impact of quantization error of inputted L1-RSRP (for training and inference) for AI/ML model for beam management.

  - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model.

The quantization overhead cost can be affected by two factors, which are quantization granularity and quantization range. The current NR specification employs 1 dB quantization granularity with a 7-bit quantization range from -140 dBm to -44 dBm for the best measurement beam, while a 2dB quantization granularity with a 4-bit quantization range is used to other beams differentially to the best beam. To clearly evaluate quantization error, it is important to analyze each of these quantization aspects separately.

To simplified issues, 7-bit quantization range from -140 dBm to -44 dBm will be applied with considering quantization range for both the best beam and other differential beams. Further, to reduce quantization overhead, we attempt to lower overhead of L1-RSRP for partial measurement beams via increasing quantization step.

According to our simulation results, similar performance can be observed from existing quantization methods and non-quantitative methods without considering quantization range, while existing quantization methods lead to significant performance degradation if quantization range from -140dBm to -44dBm is considered. For the case considering quantization range, the input L1-RSRPs lower than -140dBm are quantized as the lowest L1-RSRP level, and the input L1-RSRPs higher than -44 dBm are quantized as the highest level. For the case without considering quantization range, all the input L1-RSRPs are quantized to the specified L1-RSRP levels, where a larger quantization range is considered, e.g., -200dBm to 0 used in our evaluation in section 4.1.6. Additionally, whether or not quantization range is considered, the impact of increasing quantization steps for the differential beams from 3dB to 5dB performance loss is negligible. We, thus, propose,

- Further study on minimizing quantization overhead with similar beam prediction performance as legacy NR beam report quantization, e.g., increasing quantization step of differential beams.

Evaluation for LCM/Model monitoring

In RAN1#112 meeting, following agreement was approved for model monitoring,

Agreement

Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection:

- Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

- Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- Alt.3: Performance metric based on input/output data distribution of AI/ML

- Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

- Other alternatives are not precluded

- Note: At least the performance and spec impact should be considered

To ensure the effective monitoring of AI/ML models, we place a high emphasis on selecting performance metrics that accurately reflect the model's overall accuracy and reliability. As such, our preferred metrics include the beam prediction accuracy related KPIs and L1-RSRP difference, i.e. Alt.1 and Alt.4, which have been evaluated across companies and are directly aligned with the model's performance. While Alt.2 and Alt.3 offer potential value in terms of assessing model performance, we recognize that further study is needed to fully establish their feasibility.

Further, in RAN1#112 meeting, one agreement was approved for the definition of predicted L1-RSRP difference,

Agreement

- For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

The definition of predicted L1-RSRP difference of Top-1 beam is the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam, which is identical to the definition of Alt 4 in performance metrices for model monitoring. From our perspective, there is no need to know whether Top1 beam is correctly predicted or not in Alt4. Thus, we propose,

- Prefer Alt.1 and Alt.4, i.e. beam prediction accuracy related KPIs and predicted L1-RSRP difference, as performance metrics of AI/ML model monitoring for BM-case1 and BM-case2.

Clarification on Option D for various Set B

In RAN1#112 meeting, following agreement was approved for further study Set B selected from Set C,

Agreement

- Additionally, study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable)

  - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

- Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

Based on the above agreement, Set B is a subset of measurement beams in Set C, which means that the number of beams in Set C should be greater than or equal to the number of beams in Set B. Further, Set B is defined as the beam set used at model input. Therefore, there are two different interpretations for the usage of Set B with Set C.

The first interpretation is the number of beams in Set B is equal to the number of beams required from model input. In this case, the number of beams in Set C will be greater than or equal to the number of beams used at model input. This method can improve the beam quality of measurement data with ensuring the number of beams used for model input.

The second is the number of beams in Set C is equal to the number of beams required at model input. Thus, the number of beams in Set B will be less than or equal to the number of beams used at model input. For this method, it can reduce feedback overhead, but it requires some implementation methods to supplement the missing input data.

From our perspective, both of these interpretations are valid, and therefore further clarification is needed regarding Option D for various Set B.

- Further clarify regarding Option D for various Set B based on one of the following two.

- Option D1: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C, and the number of beams in Set B is equal to the number of beams used at model input.

- Option D2: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C, and the number of beams in Set C is equal to the number of beams used at model input.

Quasi-optimal Rx beams assumption for DL Tx beam prediction

In RAN1#112bis meeting, one agreement was approved for Rx beam assumption for DL Tx beam prediction,

Agreement

For performance evaluation of AI/ML based DL Tx beam prediction for BM-Case1 and BM-Case2, optionally study the performance with a quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the “best” Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping.

- At least the following options can be considered:

  - Opt A: Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements.

    - Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use.

    - Companies report how to find the quasi-optimal Rx beam with “previous measurement”

  - FFS: Opt B: The Rx beams for measuring Set B/Set C consist of the X% of “best” Rx beam exhaustive Rx beam sweeping and (1-X%) of random Rx beams [or the adjacent Rx beam to the “best” Rx beam].

    - X%= 80% or 90%, or other values reported by companies.

    - Note: X% is the percentage of measurements with “best” Rx beams out of all measurements

  - Other options are not precluded.

- Companies report the measurement/RS overhead together with beam prediction accuracy.

We think the intention of the proposal is to see AI/ML based beam prediction with quasi-optimal Rx beams that is closer to real system. However, for option B, the value of X becomes the most influential factor in achieving realistic performance for AI/ML based beam prediction. It is foreseeable that the smaller value of X used in DL Tx beam prediction, the poorer the performance is observed. It is unclear how this foreseeable result would be helpful in beam prediction. Further, determining the value of X is also a challenging issue that is difficult to reach a consensus. Thus, we propose,

- Deprioritizes Option B in DL Tx beam prediction with quasi-optimal Rx beams.

Clarification on Set B assumption for BM-case2

In RAN1#110bis meeting, one conclusion was agreed to clarify the relationship between Set B and model inputs of the AI/ML model.

Conclusion

For the sub use case BM-Case1 and BM-Case2, Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

According to the above conclusion, Set B is defined as the measurement results used for model inputs. From our perspective, it can be noticed that Set B should include measurement results of all measurement occasions within T1. Since some companies use a different Set B to describe model input, which implies that Set B corresponds to a single measurement occasion within T1, we believe this interpretation of Set B is inconsistent with the above conclusion. However, some company define set B per occasion. Therefore, we suggest to further clarify the meaning of Set B in BM-case2.

- For EVM of BM Case 2, companies to report one of the following options for size of Set B and pattern assumption over multiple measurement occasions in T1,

- Option 1: different pattern is assumed for different measurement occasion within T1,

        - Option 1a: size of Set B is number of unique beams in all occasions.

        - Option 1b: size of Set B is number of unique beams per occasion.

- Option 2: same pattern is assumed for each measurement occasion within T1, and size of Set B is number of unique beams in all occasions/per occasion.

Beam management procedures

For system evaluation purpose, some options were discussed in FL summary of RAN1#112 meeting on beam management procedure after AI/ML based beam prediction. To determine the most appropriate beam management procedure after AI/ML-based beam prediction, it is important to consider both output type and DL prediction type. RSRP-based prediction (producing predicted L1-RSRP for set A beams) and beam-based prediction (producing predicted Top-K beam ID) are the two main output types discussed in EVM, while DL beam pair prediction and DL Tx beam prediction are agreed as two basic beam prediction solutions. Thus, we can identify four possible combinations of beam management procedures,

- Beam management procedure 1: DL Tx beam prediction and RSRP-based prediction

- Beam management procedure 2: DL Tx beam prediction and beam-based prediction

- Beam management procedure 3: DL beam pair prediction and RSRP-based prediction

- Beam management procedure 4: DL beam pair prediction and beam-based prediction

For beam management procedure 1, utilizing AI/ML to predict Top-k Tx beams with corresponding RSRP results based on Set B measurement is recommended. The output results can be directly used following same Rx beam assumption as set B measurement for data transmission, e.g., using the best Rx beam for set B measurement. However, for beam management procedure 2, prediction of only Top-k beams without RSRP results would require a mandatory extra P2 procedure to obtain RSRP of predicted beams. Further, to improve performance, a P3 procedure may also be configured to get the best Rx beam.

Similarly, for beam management procedure 3, AI/ML can be used to predict Top-k Tx-Rx beam pairs with corresponding RSRP results based on Set B measurement. Data transmission can directly use the predicted Tx-Rx beam pairs. By contrast, when applying procedure 4 beam-based DL beam pair prediction, an extra P2+P3 procedure should be configured to obtain corresponding RSRP.

In conclusion, depending on the specific use case and the desired output type/prediction type, one of the four suggested beam management procedures can be chosen to evaluate beam management after AI/ML-based beam prediction. We, thus, propose,

- For the evaluation of AI/ML in beam management, considering the following beam management procedures:

- Option 1: AI/ML can be used to predict Top-K Tx beams with corresponding RSRPs based on Set B. The output results can be directly used following same Rx beam assumption as the set B measurement.

- Option 2: AI/ML can be used to predict only Top-K Tx beams based on Set B. A mandatory extra P2 procedure to obtain RSRP of predicted beams is needed and a P3 procedure may also be configured for performance improvement.

- Option 3: AI/ML can be used to predict Top-K Tx-Rx beam pairs with corresponding RSRPs based on Set B. The output results can be directly used.

- Option 4: AI/ML can be used to predict only Top-K Tx-Rx beam pairs. An extra P2+P3 procedure should be configured to obtain corresponding RSRPs.

- Other options are not precluded and can be reported by companies.

Proprietary protection for assistance information in BM-case1

The assistance information used as part of model input provides significant performance and generalization improvement. However, proprietary information, such as beam angle information, maybe implementation sensitive information, and most companies are cautious to disclose such proprietary information. Thus, a proprietary protection mechanism shall be studied in order not to disclose proprietary information for a given assistance information. 3 examples of such proprietary protection mechanism are shown in the following figures.

Figure 3.8-1: proprietary processing for model training and model inference at UE-side

In Figure 3.8-1, a proprietary processing module belongs to NW can be applied to map the real beam angles to proprietary processed beam angles or IDs. Take UE side model as an example. Beam resources with proprietary processed Tx beam angle information are transmitted to the UE, and then measurement RSRPs + corresponding proprietary processed Tx beam angles + corresponding Rx beam angles without proprietary processing can be used as AI input for a UE-side AI/ML model.

Figure 3.8-2: proprietary processing for model training and model inference at NW-side

Similarly, for NW-side model in Figure 3.8-2, measurement RSRPs with proprietary processed Rx beam angle information are transmitted to gNB, and then revived RSRPs + corresponding Tx beam angles without proprietary processing + corresponding proprietary processed Rx beam angles can be used as AI input for a NW-side AI/ML model.

Thus, such proprietary information can be hided for a one-side AI/ML model training/inference. The model executor does not know the real information before proprietary processing of the other side.

Figure 3.8-3: proprietary processing for model training at NW-side and model inference at UE-side

In Figure 3.8-3, an AI/ML model can be trained at NW-side with proprietary processed Tx beam information and proprietary processed Rx beam information, which data protection of Tx beam information and Rx beam information is corresponding to different proprietary processing module, and then the trained model can be transferred from NW-side to UE-side. At model inference stage, such beam resources with proprietary processed Tx beam angle information are transmitted to the UE, and then measurement RSRPs + corresponding proprietary processed Tx beam angles by proprietary processing module-1 + corresponding proprietary processed Rx beam angles by proprietary processing module-2 can be used as AI input.

Therefore, the delivered information from NW can be a virtual Tx beam information mapped from the real Tx beam information, where the mapping between the real Tx beam information and the virtual Tx beam information can be known only by NW. Then NW does not need to disclose any privacy information if it does not want to. Such mapped information can still be useful for generalization performance at UE side. The key point is a same mapping function shall be maintained across training and inference.

- Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM Case 1. Detailed proprietary protection mechanism can be FFS.

- Support to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case1, where a same mapping function is maintained for training and inference.

Proprietary protection for assistance information in BM-case2

Assistant information, e.g. beam angle, helps to improve the prediction accuracy for generalization, i.e. different gNB/UE antenna configurations, in temporal domain beam prediction. However, beam angle information is implementation sensitive information, and there may be a risk of privacy leakage if exposed to the other side. One approach to address this issue is to provide assistant information based on proprietary processing, as in Figure 3.9-1. The proprietary processing shall have the following properties.

- Hide original information by specific processing, e.g, a function or a neural network

- Proprietary processing methods used in inference and training are the same.

- Proprietary processing method is known by processing side only

Figure 3.9-1: beam pair prediction with proprietary processing

An example of proprietary processing of beam angle information is to map Tx beam angle to a beam ID based on proprietary pattern, and UE only see the mapped Tx beam ID, as in Figure 3.9-2.

Figure 3.9-2: An example of proprietary processing of beam angle information

For the study on mapping order based proprietary protection for assistant information, we evaluate the generalization performance applying a trained AI model learned from a certain set of local beam ID, beam angle or proprietary processed beam angle based on a certain number of antennas for unlearned beam shape.

Figure 3.9-3: different mapping methods: local beam ID vs beam pointing angle vs proprietary processed beam angle

For the case using proprietary processed beam angle, the training dataset 32x8 has 32 Tx beam angles, and they are proprietarily mapped to a local beam ID, e.g. Tx beam ID = 0 ~ 31, and for validation dataset 16x8, the same proprietary mapping pattern is used to map beam angle to Tx local beam ID. The difference between ID-angle mapping based on local beam ID, beam angle (global beam ID) and proprietary processed beam angle is displayed in Figure 3.9-3.

Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM-Case 2. Detailed proprietary protection mechanism can be FFS.

- Suggest to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case2, where a same mapping function is maintained for training and inference.

Solutions of generalization improvement for various beam configurations

- In practice, output size of an AI model is associated with the total number of Tx beams and the total number of Rx beams for beam pair prediction scheme, which limits AI model deployed in different UE capabilities with distinct numbers of Tx/Rx beams. To address this issue, we propose to use the expected output Tx and/or Rx beam information as a part of the input to the AI model.

Consideration of this scheme is to use the AI model to predict the performance of expected Tx and/or Rx beam information. For the example of using expected Rx beam information, if all Rx beam information could be searched and the best RSRP/beam pairs are selected based on the per beam information prediction, then the model would be applicable for arbitrary number of Rx beams. Accordingly, the output size of the AI model is only associated with the number of total Tx beams by input expected Rx information into the model. Similarly, the AI output of expected Tx beam is L1-RSRP with all Rx beams and the expected Tx beam, and one predicted L1-RSRP can be obtained in an AI model running cycle by AI input with 1 expected Tx beam information and 1 expected Rx beam information. Figure 3.10-1 and Figure 3.10-2 depicts the details about using expected Rx beam information and expected Tx beam information, respectively.

To sum up, expected Rx beam scheme is adaptable to AI/ML operations on numerous UE antenna configurations, and expected Tx beam scheme can be used in a UE without any AI model changing even switching to a cell with a different number of Tx beams. Generalization performance can be further improved by using both expected Tx beam information and expected Rx beam information.

Figure 3.10-1: beam pair prediction with expected Rx beam information

For example, as figure 3.10-1, Pre-configured 50 patterns + Tx/Rx beam information with additional 1 expected Rx beam information is fed into AI model, and an expected output of L1-RSPR with all Tx beams and the expected Rx beam indicated in AI input can be obtained. Then, other expected outputs can be acquired by feeding same Pre-configured 50 patterns + Tx/Rx beam angle + different expected Rx beam information per running cycle. After running all cycles which may equal to the number of Rx beams, all the Tx and Rx beam information (L1-RSRP) can be predicted based on this trained AI model. As a consequence, the number of AI model output per running cycle is decoupled with the number of UE Rx beams, which takes significant generalization performance improvement if we need to apply AI/ML operations on numerous UE antenna configurations.

Figure 3.10-2: beam pair prediction with expected Tx beam information

Similarly, additional 1 expected Tx beam information can be fed into AI model to predict L1-RSPR of 8 Rx beams with indicated expected Tx beam per running cycle. Consequently, L1-RSRP of total 256 beam pairs can be obtained after combining all predicted results of 32 running cycles which each running cycle has same L1-RSRP and corresponding Tx/Rx beam information as a part of AI input.

- In performance evaluation, Pre-configured 50 patterns + Tx/Rx beam information is used as baseline AI model input in expected beam information study. Three types of expected information, including expected Rx beam information, expected Tx beam information, and expected Tx/Rx beam information, will be studied, which represent relative expected beam information to be predicted. For AI model simplification, we assume 1 expected beam information applied in AI model input in the following expected beam-based simulation, and more expected beam information simultaneously used in each input sample can be further studied if needed.

Study beam pair prediction with expected Tx/Rx beam information as the AI input as one of the solutions for generalization to different number of Tx/Rx beams in BM-Case1.

Further study expected information method in BM-Case2.

Further study multiple expected beam information simultaneously used in AI input.

Evaluation results without generalization

Evaluation results for BM-Case1 when Set B is subset of Set A

AI model structure

In comparison with fully-connection neural network, superior AI model, such as transformer, convolution neural network, LSTM and so on, may increase performance gain and/or decrease model size/computation. However, the main purpose of the SID is to find an effective AI/ML algorithm with acceptable AI generalization, complexity and performance in beam prediction rather than to find an optimal AI model. Thus, a fully-connected AI model with 2 hidden layers and 1000 parameters per hidden layer is used in spatial domain beam prediction, whereas for the neural network structure in temporal domain beam prediction, MLP-mixer is attempted to obtain considerable gain in the following simulations.

Data generation

373800 samples are generated, which are based on assumptions in appendix A for spatial domain beam prediction. 87.5% of samples are used to model training, and 12.5% of samples are used for validation, which are generated from different simulation drops compared with the training dataset. For temporal domain prediction, 418000 samples are generated, and 80% of data and 20% of data is used for model training and model validation respectively.

More simulation assumptions can be obtained in appendixes.

Data processing

To address the issue of using one AI model for multiple numbers of Tx and/or Rx beams, we study the performance of using expected information in AI model input, where expected Tx/Rx beam angle is the expected beam angle that the AI model want to predict. For example, if one UE has 8 Rx beams, but the AI model is just trained to output 4 Rx beams. Then introducing 4 expected Rx beam information into the input of the AI model, and the AI model can run twice with different sets of 4 expected Rx beam information as the input and output to get all the Rx beam RSRPs. To simplify solution for performance evaluation, we assume 1 expected beam information applied in this expected beam information-based scheme. As a consequence, the total number of samples should be multiplied by 8, 32 and 256 after introducing expected RX beam information, expected Tx beam information and expected TX/RX beam information in AI model input, respectively.

### Performance with fixed pattern in Set B

      - DL Tx beam prediction

Evaluation cases for different fixed patterns in Set B of beams that of 1/8 of Set A of beams:

- L1-RSRP of 4 beams with fixed pattern 1

- L1-RSRP of 4 beams with fixed pattern 2

- L1-RSRP of 4 beams with fixed pattern 3

- L1-RSRP of 4 beams with fixed pattern 4

Evaluation cases for different fixed patterns in Set B of beams that of 1/4 of Set A of beams:

- L1-RSRP of 8 beams with fixed pattern 5

- L1-RSRP of 8 beams with fixed pattern 6

- L1-RSRP of 8 beams with fixed pattern 7

- L1-RSRP of 8 beams with fixed pattern 8

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.1.1-1: evaluation results for different fixed patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction Different fixed patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 1 | 51.19 | 56.77 | Top2/1: 68.08 Top4/1: 76.93 Top6/1: 84.06 | Top1/1:6.33 Top2/1:4.02 Top4/1:2.65 Top6/1:1.71 | 5.22 |
| Fixed pattern 2 | 63.76 | 68.92 | Top2/1: 76.91 Top4/1: 87.74 Top6/1: 91.84 | Top1/1:4.07 Top2/1:2.54 Top4/1:1.28 Top6/1:0.82 | 1.56 |
| Fixed pattern 3 | 71.33 | 76.62 | Top2/1: 81.96 Top4/1: 89.31 Top6/1: 92.71 | Top1/1:3.16 Top2/1:2.02 Top4/1:1.11 Top6/1:0.72 | 1.75 |
| Fixed pattern 4 | 68.1 | 73.2 | Top2/1: 82.6 Top4/1: 90.7 Top6/1: 94.0 | Top1/1: 3.45 Top2/1: 1.88 Top4/1: 0.97 Top6/1: 0.60 | 0.81 |

- Table 4.1.1.1-2: evaluation results for different fixed patterns in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction Different fixed patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 5 | 68.2 | 72.2 | Top2/1: 78.0 Top4/1: 84.8 Top6/1: 89.8 | Top1/1: 3.61 Top2/1: 2.53 Top4/1: 1.65 Top6/1: 1.06 | 1.53 |
| Fixed pattern 6 | 75.8 | 79.6 | Top2/1: 84.7 Top4/1: 92.9 Top6/1: 94.9 | Top1/1: 2.85 Top2/1: 1.77 Top4/1: 0.85 Top6/1: 0.54 | 0.64 |
| Fixed pattern 7 | 74.4 | 78.4 | Top2/1: 84.7 Top4/1: 92.3 Top6/1: 94.8 | Top1/1: 3.03 Top2/1: 1.79 Top4/1: 0.85 Top6/1: 0.54 | 0.63 |
| Fixed pattern 8 | 76.4 | 80.2 | Top2/1: 86.2 Top4/1: 93.0 Top6/1: 95.2 | Top1/1: 2.70 Top2/1: 1.53 Top4/1: 0.77 Top6/1: 0.51 | 0.34 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample, the performance differences brought about by different fixed patterns in Set B are significant in the KPI of beam prediction accuracy and L1-RSRP difference for both Set B = 1/8 Set A and Set B = 1/4 Set A.

Thus, the best fixed pattern is used in following studies in DL Tx beam prediction for both Set B = 1/8 Set A and Set B = 1/4 Set A.

Evaluation cases for the best fixed pattern in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with fixed pattern 4

- L1-RSRP of 4 beams with fixed pattern 4 + Tx beam information

- L1-RSRP of 4 beams with fixed pattern 4 + Rx beam information

- L1-RSRP of 4 beams with fixed pattern 4 + Tx beam information + Rx beam information

Evaluation cases for the best fixed pattern in Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with fixed pattern 8

- L1-RSRP of 8 beams with fixed pattern 8 + Tx beam information

- L1-RSRP of 8 beams with fixed pattern 8 + Rx beam information

- L1-RSRP of 8 beams with fixed pattern 8 + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.1.1-3: evaluation results for the best fixed patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 4 | 68.1 | 73.2 | Top2/1: 82.6 Top4/1: 90.7 Top6/1: 94.0 | Top1/1: 3.45 Top2/1: 1.88 Top4/1: 0.97 Top6/1: 0.60 | 0.81 |
| Fixed pattern 4 + Tx beam information | 67.8 | 72.6 | Top2/1: 82.5 Top4/1: 91.1 Top6/1: 94.5 | Top1/1: 3.46 Top2/1: 1.88 Top4/1: 0.92 Top6/1: 0.55 | 0.77 |
| Fixed pattern 4 + Rx beam information | 67.9 | 71.8 | Top2/1: 82.7 Top4/1: 90.7 Top6/1: 94.0 | Top1/1: 3.44 Top2/1: 1.87 Top4/1: 0.96 Top6/1: 0.60 | 0.94 |
| Fixed pattern 4 + Tx beam information + Rx beam information | 67.9 | 73.4 | Top2/1: 82.7 Top4/1: 91.0 Top6/1: 94.3 | Top1/1: 3.45 Top2/1: 1.86 Top4/1: 0.94 Top6/1: 0.57 | 0.81 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 83%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/1.9/1.0/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.1.1-4: evaluation results for the best fixed patterns in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 8 | 76.4 | 80.2 | Top2/1: 86.2 Top4/1: 93.0 Top6/1: 95.2 | Top1/1: 2.70 Top2/1: 1.53 Top4/1: 0.77 Top6/1: 0.51 | 0.34 |
| Fixed pattern 8 + Tx beam information | 76.8 | 79.1 | Top2/1: 86.5 Top4/1: 92.8 Top6/1: 95.1 | Top1/1: 2.66 Top2/1: 1.53 Top4/1: 0.79 Top6/1: 0.53 | 0.62 |
| Fixed pattern 8 + Rx beam information | 76.8 | 80.8 | Top2/1: 86.5 Top4/1: 93.0 Top6/1: 95.4 | Top1/1: 2.64 Top2/1: 1.53 Top4/1: 0.77 Top6/1: 0.51 | 0.37 |
| Fixed pattern 8 + Tx beam information + Rx beam information | 76.0 | 80.2 | Top2/1: 86.4 Top4/1: 92.9 Top6/1: 95.3 | Top1/1: 2.71 Top2/1: 1.51 Top4/1: 0.78 Top6/1: 0.50 | 0.4 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 76% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.6/1.5/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

      - Beam pair prediction

Evaluation cases for different fixed patterns in Set B of beams that of 1/16 of Set A of beams:

- L1-RSRP of 16 beams with fixed pattern 1

- L1-RSRP of 16 beams with fixed pattern 2

- L1-RSRP of 16 beams with fixed pattern 3

- L1-RSRP of 16 beams with fixed pattern 4

Evaluation cases for different fixed patterns in Set B of beams that of 1/8 of Set A of beams:

- L1-RSRP of 32 beams with fixed pattern 5

- L1-RSRP of 32 beams with fixed pattern 6

- L1-RSRP of 32 beams with fixed pattern 7

- L1-RSRP of 32 beams with fixed pattern 8

Evaluation assumption for label:

- L1-RSRP of 256 beams in Set A

- Table 4.1.1.2-1: evaluation results for different fixed patterns in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction Different fixed patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 1 | 32.7 | 37.9 | Top2/1: 48.1 Top4/1: 64.8 Top6/1: 73.7 | Top1/1: 6.76 Top2/1: 4.45 Top4/1: 2.51 Top6/1: 1.70 | 6.31 |
| Fixed pattern 2 | 50.3 | 60.1 | Top2/1: 67.4 Top4/1: 80.7 Top6/1: 86.6 | Top1/1: 2.75 Top2/1: 1.55 Top4/1: 0.78 Top6/1: 0.51 | 3.08 |
| Fixed pattern 3 | 52.6 | 62.2 | Top2/1: 70.1 Top4/1: 83.5 Top6/1: 89.3 | Top1/1: 2.46 Top2/1: 1.29 Top4/1: 0.58 Top6/1: 0.35 | 3.04 |
| Fixed pattern 4 | 56 | 65.2 | Top2/1: 72.9 Top4/1: 84.7 Top6/1: 89.7 | Top1/1: 2.01 Top2/1: 1.06 Top4/1: 0.52 Top6/1: 0.33 | 2.48 |

- Table 4.1.1.2-2: evaluation results for different fixed patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction Different fixed patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 5 | 17 | 21.8 | Top2/1: 30.8 Top4/1: 48.7 Top6/1: 59.6 | Top1/1: 8.69 Top2/1: 6.28 Top4/1: 3.89 Top6/1: 2.57 | 4.76 |
| Fixed pattern 6 | 33.3 | 43.7 | Top2/1: 54.1 Top4/1: 72.2 Top6/1: 81.3 | Top1/1: 4.02 Top2/1: 2.31 Top4/1: 1.10 Top6/1: 0.67 | 2.9 |
| Fixed pattern 7 | 71.9 | 82.8 | Top2/1: 87.1 Top4/1: 95.0 Top6/1: 97.3 | Top1/1: 0.68 Top2/1: 0.28 Top4/1: 0.10 Top6/1: 0.05 | 1.43 |
| Fixed pattern 8 | 74.5 | 84.5 | Top2/1: 89.1 Top4/1: 96.1 Top6/1: 98.0 | Top1/1: 0.56 Top2/1: 0.21 Top4/1: 0.07 Top6/1: 0.04 | 1.27 |

For beam pair prediction, the performance differences brought about by different fixed patterns in Set B are significant in the KPI of beam prediction accuracy and L1-RSRP difference for both Set B = 1/16 Set A and Set B = 1/8 Set A.

Thus, the best fixed pattern is used in following studies in beam pair prediction for both Set B = 1/16 Set A and Set B = 1/8 Set A.

Evaluation cases for the best fixed pattern in Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with fixed pattern 4

- L1-RSRP of 16 beams with fixed pattern 4 + Tx beam information

- L1-RSRP of 16 beams with fixed pattern 4 + Rx beam information

- L1-RSRP of 16 beams with fixed pattern 4 + Tx beam information + Rx beam information

Evaluation cases for the best fixed pattern in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with fixed pattern 8

- L1-RSRP of 32 beams with fixed pattern 8 + Tx beam information

- L1-RSRP of 32 beams with fixed pattern 8 + Rx beam information

- L1-RSRP of 32 beams with fixed pattern 8 + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams in Set A

- Table 4.1.1.2-3: evaluation results for the best fixed pattern in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 4 | 56 | 65.2 | Top2/1: 72.9 Top4/1: 84.7 Top6/1: 89.7 | Top1/1: 2.01 Top2/1: 1.06 Top4/1: 0.52 Top6/1: 0.33 | 2.48 |
| Fixed pattern 4 + Tx beam information | 55.9 | 65.6 | Top2/1: 72.5 Top4/1: 84.5 Top6/1: 89.4 | Top1/1: 2.01 Top2/1: 1.08 Top4/1: 0.53 Top6/1: 0.34 | 2.45 |
| Fixed pattern 4 + Rx beam information | 55.5 | 66 | Top2/1: 72.4 Top4/1: 84.5 Top6/1: 89.6 | Top1/1: 2.04 Top2/1: 1.07 Top4/1: 0.52 Top6/1: 0.33 | 2.49 |
| Fixed pattern 4 + Tx beam information + Rx beam information | 55.6 | 65.7 | Top2/1: 72.5 Top4/1: 84.6 Top6/1: 89.7 | Top1/1: 2.03 Top2/1: 1.06 Top4/1: 0.52 Top6/1: 0.33 | 2.5 |

For beam pair prediction with Set B = 1/16 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For beam pair prediction with Set B = 1/16 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL beam pair

- 66% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 72%/84%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 2.0/1.0/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 2.5 dB predicted L1-RSRP difference for Top-1 DL beam pair

- Table 4.1.1.2-4: evaluation results for the best fixed pattern in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Fixed pattern 8 | 74.5 | 84.5 | Top2/1: 89.1 Top4/1: 96.1 Top6/1: 98.0 | Top1/1: 0.56 Top2/1: 0.21 Top4/1: 0.07 Top6/1: 0.04 | 1.27 |
| Fixed pattern 8 + Tx beam information | 74.1 | 83.9 | Top2/1: 88.9 Top4/1: 96.2 Top6/1: 98.0 | Top1/1: 0.56 Top2/1: 0.21 Top4/1: 0.07 Top6/1: 0.04 | 1.32 |
| Fixed pattern 8 + Rx beam information | 74.5 | 84.7 | Top2/1: 89.0 Top4/1: 96.2 Top6/1: 98.0 | Top1/1: 0.56 Top2/1: 0.21 Top4/1: 0.07 Top6/1: 0.04 | 1.28 |
| Fixed pattern 8 + Tx beam information + Rx beam information | 74 | 84.5 | Top2/1: 88.6 Top4/1: 95.9 Top6/1: 97.9 | Top1/1: 0.57 Top2/1: 0.22 Top4/1: 0.08 Top6/1: 0.04 | 1.29 |

For beam pair prediction with Set B = 1/8 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For beam pair prediction with Set B = 1/8 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 74% beam prediction accuracy of Top-1 DL beam pair

- 84% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 89%/96%/98% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 0.5/0.2/0.07/0.04 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 1.2 dB predicted L1-RSRP difference for Top-1 DL beam pair

### Performance with random patterns in Set B

      - DL Tx beam prediction

Evaluation cases for random pattern in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with random pattern

- L1-RSRP of 4 beams with random pattern + Tx beam information

- L1-RSRP of 4 beams with random pattern + Rx beam information

- L1-RSRP of 4 beams with random pattern + Tx beam information + Rx beam information

Evaluation cases for random pattern in Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with random pattern

- L1-RSRP of 8 beams with random pattern + Tx beam information

- L1-RSRP of 8 beams with random pattern + Rx beam information

- L1-RSRP of 8 beams with random pattern + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.2.1-1: evaluation results for random pattern in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Random pattern | 38.2 | 42.7 | Top2/1: 66.3 Top4/1: 86.5 Top6/1: 92.5 | Top1/1: 7.91 Top2/1: 3.88 Top4/1: 1.37 Top6/1: 0.71 | 6.24 |
| Random pattern + Tx beam information | 56.5 | 61.8 | Top2/1: 75.2 Top4/1: 85.6 Top6/1: 90.3 | Top1/1: 5.00 Top2/1: 2.77 Top4/1: 1.52 Top6/1: 0.97 | 3.73 |
| Random pattern + Rx beam information | 38.2 | 42.2 | Top2/1: 66.5 Top4/1: 86.0 Top6/1: 91.2 | Top1/1: 7.92 Top2/1: 3.85 Top4/1: 1.47 Top6/1: 0.86 | 6.2 |
| Random pattern + Tx beam information + Rx beam information | 55.2 | 60.7 | Top2/1: 74.6 Top4/1: 85.4 Top6/1: 90.3 | Top1/1: 5.17 Top2/1: 2.83 Top4/1: 1.54 Top6/1: 0.98 | 3.91 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, random pattern in Set B with Tx beam information brings significant performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, random pattern in Set B with Tx beam information as assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL Tx beam

- 61% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 75%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 5.0/2.7/1.5/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.2.1-2: evaluation results for random pattern in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Random pattern | 48 | 52.4 | Top2/1: 72.2 Top4/1: 86.9 Top6/1: 91.7 | Top1/1: 6.14 Top2/1: 2.81 Top4/1: 1.34 Top6/1: 0.80 | 5.24 |
| Random pattern + Tx beam information | 66.6 | 70.2 | Top2/1: 81.3 Top4/1: 89.1 Top6/1: 92.6 | Top1/1: 3.52 Top2/1: 2.01 Top4/1: 1.14 Top6/1: 0.75 | 2.6 |
| Random pattern + Rx beam information | 47.6 | 53.8 | Top2/1: 75.0 Top4/1: 87.7 Top6/1: 92.0 | Top1/1: 6.01 Top2/1: 2.70 Top4/1: 1.23 Top6/1: 0.75 | 5.3 |
| Random pattern + Tx beam information + Rx beam information | 66.9 | 72.2 | Top2/1: 82.0 Top4/1: 89.6 Top6/1: 93.0 | Top1/1: 3.48 Top2/1: 1.95 Top4/1: 1.09 Top6/1: 0.71 | 2.54 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, random pattern in Set B with Tx beam information brings significant performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, random pattern in Set B with Tx beam information as assistance information achieves approximately,

- 66% beam prediction accuracy of Top-1 DL Tx beam

- 70% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 81%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/2.0/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

      - Beam pair prediction

Evaluation cases for random pattern in Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with random pattern

- L1-RSRP of 16 beams with random pattern + Tx beam information

- L1-RSRP of 16 beams with random pattern + Rx beam information

- L1-RSRP of 16 beams with random pattern + Tx beam information + Rx beam information

Evaluation cases for random pattern in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with random pattern

- L1-RSRP of 32 beams with random pattern + Tx beam information

- L1-RSRP of 32 beams with random pattern + Rx beam information

- L1-RSRP of 32 beams with random pattern + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams in Set A

- Table 4.1.2.2-1: evaluation results for random pattern in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Random pattern | 8.9 | 10.4 | Top2/1: 17.6 Top4/1: 33.2 Top6/1: 45.2 | Top1/1: 12.44 Top2/1: 9.59 Top4/1: 6.32 Top6/1: 4.45 | 6.96 |
| Random pattern + Tx beam information | 13.4 | 19.2 | Top2/1: 26.0 Top4/1: 45.6 Top6/1: 57.8 | Top1/1: 9.50 Top2/1: 6.74 Top4/1: 3.91 Top6/1: 2.63 | 6.41 |
| Random pattern + Rx beam information | 16.2 | 20.6 | Top2/1: 28.4 Top4/1: 45.2 Top6/1: 57.1 | Top1/1: 9.68 Top2/1: 6.93 Top4/1: 4.27 Top6/1: 2.88 | 6.74 |
| Random pattern + Tx beam information + Rx beam information | 21.2 | 27.7 | Top2/1: 36.8 Top4/1: 55.5 Top6/1: 66.7 | Top1/1: 7.55 Top2/1: 4.98 Top4/1: 2.79 Top6/1: 1.83 | 5.84 |

For beam pair prediction with Set B = 1/16 Set A, random pattern in Set B with both Tx beam information and Rx beam information brings considerable gain compared to the same Set B without assistance information.

For beam pair prediction with Set B = 1/16 Set A, random pattern in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 21% beam prediction accuracy of Top-1 DL beam pair

- 27% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 36%/55%/66% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 7.5/4.9/2.7/1.8 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 5.8 dB predicted L1-RSRP difference for Top-1 DL beam pair

- Table 4.1.2.2-2: evaluation results for random pattern in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Random pattern | 13.2 | 16.6 | Top2/1: 24.7 Top4/1: 43.6 Top6/1: 57.0 | Top1/1: 10.1 Top2/1: 7.32 Top4/1: 4.35 Top6/1: 2.81 | 6.41 |
| Random pattern + Tx beam information | 18.8 | 25.0 | Top2/1: 35.1 Top4/1: 57.5 Top6/1: 69.3 | Top1/1: 7.57 Top2/1: 4.92 Top4/1: 2.47 Top6/1: 1.54 | 5.47 |
| Random pattern + Rx beam information | 21.1 | 26.0 | Top2/1: 36.6 Top4/1: 55.6 Top6/1: 67.6 | Top1/1: 7.61 Top2/1: 5.02 Top4/1: 2.74 Top6/1: 1.69 | 5.9 |
| Random pattern + Tx beam information + Rx beam information | 29.0 | 36.2 | Top2/1: 47.5 Top4/1: 66.8 Top6/1: 77.0 | Top1/1: 5.50 Top2/1: 3.30 Top4/1: 1.65 Top6/1: 1.01 | 4.97 |

For beam pair prediction with Set B = 1/8 Set A, random pattern in Set B with both Tx beam information and Rx beam information brings considerable gain compared to the same Set B without assistance information.

For beam pair prediction with Set B = 1/8 Set A, random pattern in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 29% beam prediction accuracy of Top-1 DL beam pair

- 36% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 47%/66%/77% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 5.5/3.3/1.6/1.0 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 4.9 dB predicted L1-RSRP difference for Top-1 DL beam pair

### Performance with pre-configured patterns in Set B

      - DL Tx beam prediction

Evaluation cases for 5 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 5 pre-configured patterns

- L1-RSRP of 4 beams with 5 pre-configured patterns + Tx beam information

- L1-RSRP of 4 beams with 5 pre-configured patterns + Rx beam information

- L1-RSRP of 4 beams with 5 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 10 pre-configured patterns

- L1-RSRP of 4 beams with 10 pre-configured patterns + Tx beam information

- L1-RSRP of 4 beams with 10 pre-configured patterns + Rx beam information

- L1-RSRP of 4 beams with 10 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 20 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 20 pre-configured patterns

- L1-RSRP of 4 beams with 20 pre-configured patterns + Tx beam information

- L1-RSRP of 4 beams with 20 pre-configured patterns + Rx beam information

- L1-RSRP of 4 beams with 20 pre-configured patterns + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.3.1-1: evaluation results for 5 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 5 pre-configured patterns | 64.5 | 69.5 | Top2/1: 79.8 Top4/1: 87.5 Top6/1: 91.2 | Top1/1: 3.93 Top2/1: 2.26 Top4/1: 1.31 Top6/1: 0.87 | 2.83 |
| 5 pre-configured patterns + Tx beam information | 69.2 | 73.8 | Top2/1: 82.2 Top4/1: 89.5 Top6/1: 92.8 | Top1/1: 3.35 Top2/1: 1.97 Top4/1: 1.11 Top6/1: 0.73 | 1.32 |
| 5 pre-configured patterns + Rx beam information | 64.2 | 69.3 | Top2/1: 79.7 Top4/1: 87.6 Top6/1: 91.5 | Top1/1: 3.97 Top2/1: 2.26 Top4/1: 1.29 Top6/1: 0.83 | 2.85 |
| 5 pre-configured patterns + Tx beam information + Rx beam information | 68.9 | 74 | Top2/1: 82.2 Top4/1: 89.6 Top6/1: 92.9 | Top1/1: 3.37 Top2/1: 1.97 Top4/1: 1.10 Top6/1: 0.71 | 1.34 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 69% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.3/1.9/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.1-2: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 10 pre-configured patterns | 60.8 | 66 | Top2/1: 78.8 Top4/1: 86.9 Top6/1: 90.6 | Top1/1: 4.42 Top2/1: 2.36 Top4/1: 1.37 Top6/1: 0.93 | 2.99 |
| 10 pre-configured patterns + Tx beam information | 68.1 | 72.8 | Top2/1: 82.1 Top4/1: 89.6 Top6/1: 92.9 | Top1/1: 3.50 Top2/1: 1.98 Top4/1: 1.09 Top6/1: 0.71 | 1.27 |
| 10 pre-configured patterns + Rx beam information | 60.7 | 67.4 | Top2/1: 78.8 Top4/1: 87.3 Top6/1: 90.9 | Top1/1: 4.42 Top2/1: 2.35 Top4/1: 1.32 Top6/1: 0.89 | 3.02 |
| 10 pre-configured patterns + Tx beam information + Rx beam information | 68 | 73.1 | Top2/1: 82.2 Top4/1: 89.5 Top6/1: 92.9 | Top1/1: 3.49 Top2/1: 1.98 Top4/1: 1.11 Top6/1: 0.72 | 1.25 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/1.9/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.1-3: evaluation results for 20 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 20 pre-configured patterns | 59.2 | 65.2 | Top2/1: 78.2 Top4/1: 86.9 Top6/1: 90.8 | Top1/1: 4.65 Top2/1: 2.42 Top4/1: 1.36 Top6/1: 0.90 | 3.24 |
| 20 pre-configured patterns + Tx beam information | 68.1 | 72.5 | Top2/1: 82.1 Top4/1: 89.5 Top6/1: 92.8 | Top1/1: 3.52 Top2/1: 2.01 Top4/1: 1.11 Top6/1: 0.73 | 1.26 |
| 20 pre-configured patterns + Rx beam information | 58.9 | 65.1 | Top2/1: 78.3 Top4/1: 86.9 Top6/1: 90.5 | Top1/1: 4.70 Top2/1: 2.41 Top4/1: 1.36 Top6/1: 0.94 | 3.3 |
| 20 pre-configured patterns + Tx beam information + Rx beam information | 68 | 74.1 | Top2/1: 82.3 Top4/1: 89.6 Top6/1: 92.9 | Top1/1: 3.51 Top2/1: 1.98 Top4/1: 1.10 Top6/1: 0.72 | 1.46 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/2.0/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/8 Set A, we have,

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, the performance improvement from assistance information becomes more apparent with the number of pre-configured patterns in Set B increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, same performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx beam information as assistance information.

Evaluation cases for 5 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 5 pre-configured patterns

- L1-RSRP of 8 beams with 5 pre-configured patterns + Tx beam information

- L1-RSRP of 8 beams with 5 pre-configured patterns + Rx beam information

- L1-RSRP of 8 beams with 5 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 10 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 10 pre-configured patterns

- L1-RSRP of 8 beams with 10 pre-configured patterns + Tx beam information

- L1-RSRP of 8 beams with 10 pre-configured patterns + Rx beam information

- L1-RSRP of 8 beams with 10 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 20 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 20 pre-configured patterns

- L1-RSRP of 8 beams with 20 pre-configured patterns + Tx beam information

- L1-RSRP of 8 beams with 20 pre-configured patterns + Rx beam information

- L1-RSRP of 8 beams with 20 pre-configured patterns + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.3.1-4: evaluation results for 5 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 5 pre-configured patterns | 72.5 | 76.4 | Top2/1: 83.0 Top4/1: 90.6 Top6/1: 93.6 | Top1/1: 3.14 Top2/1: 1.93 Top4/1: 1.01 Top6/1: 0.66 | 0.97 |
| 5 pre-configured patterns + Tx beam information | 73.2 | 78.6 | Top2/1: 83.4 Top4/1: 90.7 Top6/1: 93.7 | Top1/1: 3.06 Top2/1:1.90 Top4/1: 1.01 Top6/1: 0.66 | 0.82 |
| 5 pre-configured patterns + Rx beam information | 72.6 | 77 | Top2/1: 83.2 Top4/1: 90.9 Top6/1: 94.0 | Top1/1: 3.10 Top2/1: 1.90 Top4/1: 0.97 Top6/1: 0.61 | 1.07 |
| 5 pre-configured patterns + Tx beam information + Rx beam information | 73.3 | 78.6 | Top2/1: 83.5 Top4/1: 90.7 Top6/1: 93.6 | Top1/1: 3.06 Top2/1: 1.89 Top4/1: 1.01 Top6/1: 0.67 | 0.79 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 5 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 5 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 83%/90%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.9/1.0/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.1-5: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 10 pre-configured patterns | 71.3 | 76.8 | Top2/1: 83.7 Top4/1: 91.1 Top6/1: 94.1 | Top1/1: 3.16 Top2/1: 1.88 Top4/1: 0.93 Top6/1: 0.59 | 1.25 |
| 10 pre-configured patterns + Tx beam information | 72.8 | 78.4 | Top2/1: 84.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.04 Top2/1: 1.81 Top4/1: 0.89 Top6/1: 0.57 | 0.89 |
| 10 pre-configured patterns + Rx beam information | 71.1 | 76.2 | Top2/1: 83.0 Top4/1: 91.2 Top6/1: 94.2 | Top1/1: 3.20 Top2/1: 1.88 Top4/1: 0.91 Top6/1: 0.58 | 1.23 |
| 10 pre-configured patterns + Tx beam information + Rx beam information | 72.8 | 77.7 | Top2/1: 83.6 Top4/1: 91.5 Top6/1: 94.2 | Top1/1: 3.05 Top2/1: 1.85 Top4/1: 0.94 Top6/1: 0.61 | 0.79 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 10 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 10 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.8/0.9/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.1-6: evaluation results for 20 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 20 pre-configured patterns | 70.2 | 74.6 | Top2/1: 83.4 Top4/1: 91.5 Top6/1: 94.6 | Top1/1: 3.20 Top2/1: 1.78 Top4/1: 0.88 Top6/1: 0.53 | 1.64 |
| 20 pre-configured patterns + Tx beam information | 73.3 | 77.7 | Top2/1: 84.5 Top4/1: 92.0 Top6/1: 94.8 | Top1/1: 2.92 Top2/1: 1.72 Top4/1: 0.84 Top6/1: 0.53 | 0.79 |
| 20 pre-configured patterns + Rx beam information | 70 | 73.5 | Top2/1: 82.9 Top4/1: 91.2 Top6/1: 94.3 | Top1/1: 3.22 Top2/1: 1.83 Top4/1: 0.90 Top6/1: 0.56 | 1.79 |
| 20 pre-configured patterns + Tx beam information + Rx beam information | 73.8 | 78.6 | Top2/1: 84.7 Top4/1: 92.3 Top6/1: 95.1 | Top1/1: 2.86 Top2/1: 1.69 Top4/1: 0.81 Top6/1: 0.50 | 0.83 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 20 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 20 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 77% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/92%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.9/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/4 Set A, we have,

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, the performance improvement from assistance information becomes more apparent with the number of pre-configured patterns in Set B increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, same performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx beam information as assistance information.

      - Beam pair prediction

Evaluation cases for 5 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 5 pre-configured patterns

- L1-RSRP of 16 beams with 5 pre-configured patterns + Tx beam information

- L1-RSRP of 16 beams with 5 pre-configured patterns + Rx beam information

- L1-RSRP of 16 beams with 5 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 10 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 10 pre-configured patterns

- L1-RSRP of 16 beams with 10 pre-configured patterns + Tx beam information

- L1-RSRP of 16 beams with 10 pre-configured patterns + Rx beam information

- L1-RSRP of 16 beams with 10 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 20 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 20 pre-configured patterns

- L1-RSRP of 16 beams with 20 pre-configured patterns + Tx beam information

- L1-RSRP of 16 beams with 20 pre-configured patterns + Rx beam information

- L1-RSRP of 16 beams with 20 pre-configured patterns + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams L1-RSRP in Set A

- Table 4.1.3.2-1: evaluation results for 5 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 5 pre-configured patterns | 47.4 | 55.5 | Top2/1: 65.5 Top4/1: 80.2 Top6/1: 86.6 | Top1/1: 3.07 Top2/1: 1.76 Top4/1: 0.89 Top6/1: 0.57 | 3.40 |
| 5 pre-configured patterns + Tx beam information | 53.4 | 64.1 | Top2/1: 71.5 Top4/1: 84.8 Top6/1: 90.3 ' | Top1/1: 2.20 Top2/1: 1.12 Top4/1: 0.52 Top6/1: 0.31 | 2.86 |
| 5 pre-configured patterns + Rx beam information | 53.4 | 62.0 | Top2/1: 71.3 Top4/1: 84.8 Top6/1: 90.0 | Top1/1: 2.18 Top2/1: 1.11 Top4/1: 0.52 Top6/1: 0.32 | 2.83 |
| 5 pre-configured patterns + Tx beam information + Rx beam information | 53.4 | 64.6 | Top2/1: 71.4 Top4/1: 84.5 Top6/1: 89.9 | Top1/1: 2.20 Top2/1: 1.14 Top4/1: 0.53 Top6/1: 0.33 | 2.86 |

For beam pair prediction with Set B = 1/16 Set A, 5 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 5 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 53% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 71%/84%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.2/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.2-2: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 10 pre-configured patterns | 40.9 | 48.1 | Top2/1: 59.6 Top4/1: 75.1 Top6/1: 82.6 | Top1/1: 4.06 Top2/1: 2.42 Top4/1: 1.28 Top6/1: 0.83 | 4.07 |
| 10 pre-configured patterns + Tx beam information | 52.3 | 62.5 | Top2/1: 70.7 Top4/1: 83.9 Top6/1: 89.6 | Top1/1: 2.30 Top2/1: 1.19 Top4/1: 0.56 Top6/1: 0.34 | 2.90 |
| 10 pre-configured patterns + Rx beam information | 51.8 | 61.8 | Top2/1: 70.3 Top4/1: 83.7 Top6/1: 89.5 | Top1/1: 2.32 Top2/1: 1.19 Top4/1: 0.56 Top6/1: 0.35 | 2.91 |
| 10 pre-configured patterns + Tx beam information + Rx beam information | 51.9 | 62.1 | Top2/1: 70.2 Top4/1: 83.8 Top6/1: 89.4 | Top1/1: 2.31 Top2/1: 1.20 Top4/1: 0.56 Top6/1: 0.36 | 2.92 |

For beam pair prediction with Set B = 1/16 Set A, 10 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 10 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 52% beam prediction accuracy of Top-1 DL Tx beam

- 62% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 70%/84%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.3/1.2/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.2-3: evaluation results for 20 pre-configured patterns in Set B of beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 20 pre-configured patterns | 36.2 | 43.9 | Top2/1: 54.1 Top4/1: 70.9 Top6/1: 79.7 | Top1/1: 4.81 Top2/1: 2.99 Top4/1: 1.60 Top6/1: 1.03 | 4.43 |
| 20 pre-configured patterns + Tx beam information | 49.8 | 60.2 | Top2/1: 68.4 Top4/1: 82.6 Top6/1: 88.4 | Top1/1: 2.53 Top2/1: 1.33 Top4/1: 0.63 Top6/1: 0.39 | 3.00 |
| 20 pre-configured patterns + Rx beam information | 49.7 | 58.8 | Top2/1: 68.1 Top4/1: 82.2 Top6/1: 88.3 | Top1/1: 2.55 Top2/1: 1.34 Top4/1: 0.64 Top6/1: 0.40 | 3.06 |
| 20 pre-configured patterns + Tx beam information + Rx beam information | 49.5 | 58.7 | Top2/1: 67.9 Top4/1: 82.2 Top6/1: 88.3 | Top1/1: 2.57 Top2/1: 1.36 Top4/1: 0.65 Top6/1: 0.40 | 3.11 |

For beam pair prediction with Set B = 1/16 Set A, 20 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 20 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 50% beam prediction accuracy of Top-1 DL Tx beam

- 60% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 68%/82%/88% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.5/1.3/0.6/0.4 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/16 Set A, we have,

For beam pair prediction with Set B = 1/16 Set A, the performance improvement from assistance information becomes larger with the number of pre-configured patterns in Set B increases.

For beam pair prediction with Set B = 1/16 Set A, closed performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx/Rx beam information as assistance information.

Evaluation cases for 5 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 5 pre-configured patterns

- L1-RSRP of 32 beams with 5 pre-configured patterns + Tx beam information

- L1-RSRP of 32 beams with 5 pre-configured patterns + Rx beam information

- L1-RSRP of 32 beams with 5 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 10 pre-configured patterns

- L1-RSRP of 32 beams with 10 pre-configured patterns + Tx beam information

- L1-RSRP of 32 beams with 10 pre-configured patterns + Rx beam information

- L1-RSRP of 32 beams with 10 pre-configured patterns + Tx beam information + Rx beam information

Evaluation cases for 20 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 20 pre-configured patterns

- L1-RSRP of 32 beams with 20 pre-configured patterns + Tx beam information

- L1-RSRP of 32 beams with 20 pre-configured patterns + Rx beam information

- L1-RSRP of 32 beams with 20 pre-configured patterns + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams L1-RSRP in Set A

- Table 4.1.3.2-4: evaluation results for 5 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 5 pre-configured patterns | 67.4 | 77.4 | Top2/1: 83.8 Top4/1: 93.3 Top6/1: 96.2 | Top1/1: 1.00 Top2/1: 0.43 Top4/1: 0.17 Top6/1: 0.09 | 1.85 |
| 5 pre-configured patterns + Tx beam information | 68.5 | 77.7 | Top2/1: 84.6 Top4/1: 93.9 Top6/1: 96.6 | Top1/1: 0.87 Top2/1: 0.36 Top4/1: 0.13 Top6/1: 0.07 | 1.70 |
| 5 pre-configured patterns + Rx beam information | 68.7 | 79.6 | Top2/1: 84.7 Top4/1: 94.0 Top6/1: 96.6 | Top1/1: 0.87 Top2/1: 0.36 Top4/1: 0.13 Top6/1: 0.07 | 1.67 |
| 5 pre-configured patterns + Tx beam information + Rx beam information | 68.5 | 78.7 | Top2/1: 84.7 Top4/1: 93.9 Top6/1: 96.6 | Top1/1: 0.88 Top2/1: 0.36 Top4/1: 0.13 Top6/1: 0.07 | 1.70 |

For beam pair prediction with Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/93%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.8/0.3/0.1/0.07 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.2-5: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 10 pre-configured patterns | 62.6 | 73 | Top2/1: 80.1 Top4/1: 91.1 Top6/1: 94.9 | Top1/1: 1.33 Top2/1: 0.61 Top4/1: 0.26 Top6/1: 0.15 | 2.17 |
| 10 pre-configured patterns + Tx beam information | 65.2 | 75.6 | Top2/1: 82.1 Top4/1: 92.6 Top6/1: 95.9 | Top1/1: 1.04 Top2/1: 0.46 Top4/1: 0.17 Top6/1: 0.09 | 1.82 |
| 10 pre-configured patterns + Rx beam information | 65.2 | 76.1 | Top2/1: 82.6 Top4/1: 92.8 Top6/1: 96.1 | Top1/1: 1.05 Top2/1: 0.45 Top4/1: 0.17 Top6/1: 0.09 | 1.89 |
| 10 pre-configured patterns + Tx beam information + Rx beam information | 64.8 | 75.9 | Top2/1: 82.0 Top4/1: 92.5 Top6/1: 95.9 | Top1/1: 1.07 Top2/1: 0.46 Top4/1: 0.17 Top6/1: 0.09 | 1.91 |

For beam pair prediction with Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 65% beam prediction accuracy of Top-1 DL Tx beam

- 76% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/92%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.0/0.4/0.1/0.09 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.3.2-6: evaluation results for 20 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 20 pre-configured patterns | 57.7 | 65.9 | Top2/1: 76.2 Top4/1: 88.5 Top6/1: 93.0 | Top1/1: 1.80 Top2/1: 0.88 Top4/1: 0.38 Top6/1: 0.23 | 2.71 |
| 20 pre-configured patterns + Tx beam information | 63.4 | 74.0 | Top2/1: 80.8 Top4/1: 91.7 Top6/1: 95.4 | Top1/1: 1.18 Top2/1: 0.51 Top4/1: 0.19 Top6/1: 0.10 | 2.03 |
| 20 pre-configured patterns + Rx beam information | 63.1 | 72.1 | Top2/1: 80.8 Top4/1: 91.6 Top6/1: 95.3 | Top1/1: 1.18 Top2/1: 0.51 Top4/1: 0.19 Top6/1: 0.11 | 2.09 |
| 20 pre-configured patterns + Tx beam information + Rx beam information | 62.7 | 72.6 | Top2/1: 80.6 Top4/1: 91.6 Top6/1: 95.3 | Top1/1: 1.23 Top2/1: 0.53 Top4/1: 0.20 Top6/1: 0.10 | 2.13 |

For beam pair prediction with Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 63% beam prediction accuracy of Top-1 DL Tx beam

- 74% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.2/0.5/0.2/0.1 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.1 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/8 Set A, we have,

For beam pair prediction with Set B = 1/8 Set A, the performance improvement from assistance information becomes larger with the number of pre-configured patterns in Set B increases.

For beam pair prediction with Set B = 1/8 Set A, closed performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx/Rx beam information as assistance information.

### Performance with random patterns in Set C

      - DL Tx beam prediction

Evaluation cases for Set C with 5 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 5 beams in Set C

- L1-RSRP of 4 beams with 5 beams in Set C + Tx beam information

- L1-RSRP of 4 beams with 5 beams in Set C + Rx beam information

- L1-RSRP of 4 beams with 5 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 6 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 6 beams in Set C

- L1-RSRP of 4 beams with 6 beams in Set C + Tx beam information

- L1-RSRP of 4 beams with 6 beams in Set C + Rx beam information

- L1-RSRP of 4 beams with 6 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 7 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 7 beams in Set C

- L1-RSRP of 4 beams with 7 beams in Set C + Tx beam information

- L1-RSRP of 4 beams with 7 beams in Set C + Rx beam information

- L1-RSRP of 4 beams with 7 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 8 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 4 beams with 8 beams in Set C

- L1-RSRP of 4 beams with 8 beams in Set C + Tx beam information

- L1-RSRP of 4 beams with 8 beams in Set C + Rx beam information

- L1-RSRP of 4 beams with 8 beams in Set C + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.4.1-1: evaluation results for 5 beams in Set C and Set B of 4 beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 5 beams in Set C | 67.9 | 72.9 | Top2/1: 81.8 Top4/1: 89.8 Top6/1: 92.8 | Top1/1: 3.57 Top2/1: 2.02 Top4/1: 1.09 Top6/1: 0.73 | 1.18 |
| 5 beams in Set C + Tx beam information | 70.1 | 75.8 | Top2/1: 84.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.19 Top2/1: 1.71 Top4/1: 0.84 Top6/1: 0.54 | 0.86 |
| 5 beams in Set C + Rx beam information | 67.4 | 73.4 | Top2/1: 81.3 Top4/1: 89.6 Top6/1: 92.8 | Top1/1: 3.62 Top2/1: 2.07 Top4/1: 1.10 Top6/1: 0.73 | 1.3 |
| 5 beams in Set C + Tx beam information + Rx beam information | 70.5 | 76.6 | Top2/1: 84.2 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.14 Top2/1: 1.69 Top4/1: 0.85 Top6/1: 0.55 | 1.06 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 5 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 5 beams with Tx beam information as assistance information achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 76% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-2: evaluation results for 6 beams in Set C and Set B of 4 beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 6 beams in Set C | 66.8 | 73.1 | Top2/1: 81.2 Top4/1: 90.0 Top6/1: 93.3 | Top1/1: 3.68 Top2/1: 2.08 Top4/1: 1.06 Top6/1: 0.67 | 1.51 |
| 6 beams in Set C + Tx beam information | 70.5 | 75.3 | Top2/1: 84.0 Top4/1: 91.5 Top6/1: 94.3 | Top1/1: 3.15 Top2/1: 1.71 Top4/1: 0.87 Top6/1: 0.56 | 0.86 |
| 6 beams in Set C + Rx beam information | 66.7 | 72.1 | Top2/1: 80.9 Top4/1: 89.7 Top6/1: 92.9 | Top1/1: 3.68 Top2/1: 2.11 Top4/1: 1.10 Top6/1: 0.72 | 1.49 |
| 6 beams in Set C + Tx beam information + Rx beam information | 70.4 | 75.3 | Top2/1: 83.9 Top4/1: 91.2 Top6/1: 94.1 | Top1/1: 3.17 Top2/1: 1.71 Top4/1: 0.90 Top6/1: 0.59 | 0.77 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 6 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 6 beams with Tx beam information as assistance information achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-3: evaluation results for 7 beams in Set C and Set B of 4 beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 7 beams in Set C | 67.1 | 71.8 | Top2/1: 81.4 Top4/1: 90.1 Top6/1: 93.1 | Top1/1: 3.64 Top2/1: 2.03 Top4/1: 1.02 Top6/1: 0.69 | 1.55 |
| 7 beams in Set C + Tx beam information | 71.8 | 75.2 | Top2/1: 85.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.10 Top2/1: 1.61 Top4/1: 0.84 Top6/1: 0.55 | 0.72 |
| 7 beams in Set C + Rx beam information | 67.1 | 71.8 | Top2/1: 81.5 Top4/1: 89.9 Top6/1: 92.9 | Top1/1: 3.65 Top2/1: 2.01 Top4/1: 1.04 Top6/1: 0.71 | 1.51 |
| 7 beams in Set C + Tx beam information + Rx beam information | 71.6 | 77.5 | Top2/1: 84.8 Top4/1: 91.9 Top6/1: 94.6 | Top1/1: 3.13 Top2/1: 1.65 Top4/1: 0.84 Top6/1: 0.54 | 0.8 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 7 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 7 beams with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 85%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.6/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-4: evaluation results for 8 beams in Set C and Set B of 4 beams that of 1/8 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 8 beams in Set C | 65.8 | 71.5 | Top2/1: 80.6 Top4/1: 89.5 Top6/1: 92.6 | Top1/1: 3.90 Top2/1: 2.17 Top4/1: 1.09 Top6/1: 0.72 | 1.84 |
| 8 beams in Set C + Tx beam information | 72.5 | 77.3 | Top2/1: 84.2 Top4/1: 92.0 Top6/1: 94.6 | Top1/1: 3.05 Top2/1: 1.72 Top4/1: 0.82 Top6/1: 0.54 | 0.6 |
| 8 beams in Set C + Rx beam information | 66.2 | 71.5 | Top2/1: 80.8 Top4/1: 90.3 Top6/1: 93.3 | Top1/1: 3.88 Top2/1: 2.14 Top4/1: 1.01 Top6/1: 0.65 | 1.95 |
| 8 beams in Set C + Tx beam information + Rx beam information | 72.8 | 76.8 | Top2/1: 84.1 Top4/1: 91.7 Top6/1: 94.2 | Top1/1: 3.02 Top2/1: 1.72 Top4/1: 0.85 Top6/1: 0.58 | 0.53 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 8 beams with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 8 beams with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 77% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/92%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/8 Set A, we have,

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, the performance improvement from assistance information becomes more apparent with the number of beams in Set C increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, marginal performance improvement can be achieved among different number of beams in Set C with Tx beam information as assistance information.

Evaluation cases for Set C with 9 beams and Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 9 beams in Set C

- L1-RSRP of 8 beams with 9 beams in Set C + Tx beam information

- L1-RSRP of 8 beams with 9 beams in Set C + Rx beam information

- L1-RSRP of 8 beams with 9 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 10 beams and Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 10 beams in Set C

- L1-RSRP of 8 beams with 10 beams in Set C + Tx beam information

- L1-RSRP of 8 beams with 10 beams in Set C + Rx beam information

- L1-RSRP of 8 beams with 10 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 11 beams and Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 11 beams in Set C

- L1-RSRP of 8 beams with 11 beams in Set C + Tx beam information

- L1-RSRP of 8 beams with 11 beams in Set C + Rx beam information

- L1-RSRP of 8 beams with 11 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 12 beams and Set B of beams that of 1/4 of Set A of beams with/without assistance information:

- L1-RSRP of 8 beams with 12 beams in Set C

- L1-RSRP of 8 beams with 12 beams in Set C + Tx beam information

- L1-RSRP of 8 beams with 12 beams in Set C + Rx beam information

- L1-RSRP of 8 beams with 12 beams in Set C + Tx beam information + Rx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.4.1-5: evaluation results for 9 beams in Set C and Set B of 8 beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 9 beams in Set C | 73.3 | 78.9 | Top2/1: 85.9 Top4/1: 92.7 Top6/1: 95.3 | Top1/1: 2.89 Top2/1: 1.53 Top4/1: 0.76 Top6/1: 0.47 | 0.69 |
| 9 beams in Set C + Tx beam information | 73.1 | 78.4 | Top2/1: 85.5 Top4/1: 92.2 Top6/1: 94.9 | Top1/1: 2.91 Top2/1: 1.58 Top4/1: 0.83 Top6/1: 0.52 | 0.53 |
| 9 beams in Set C + Rx beam information | 73.2 | 77.6 | Top2/1: 85.6 Top4/1: 92.4 Top6/1: 95.0 | Top1/1: 2.90 Top2/1: 1.56 Top4/1: 0.80 Top6/1: 0.51 | 0.55 |
| 9 beams in Set C + Tx beam information + Rx beam information | 73.5 | 78 | Top2/1: 85.9 Top4/1: 92.5 Top6/1: 95.2 | Top1/1: 2.87 Top2/1: 1.55 Top4/1: 0.90 Top6/1: 0.50 | 0.42 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 9 beams without or with different assistance information provides similar performance.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 9 beams with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 85%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.9/1.5/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-6: evaluation results for 10 beams in Set C and Set B of 8 beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 10 beams in Set C | 73.3 | 77.5 | Top2/1: 85.9 Top4/1: 92.6 Top6/1: 95.0 | Top1/1: 2.86 Top2/1: 1.51 Top4/1: 0.76 Top6/1: 0.50 | 0.65 |
| 10 beams in Set C + Tx beam information | 74.8 | 80.1 | Top2/1: 86.6 Top4/1: 92.8 Top6/1: 95.4 | Top1/1: 2.74 Top2/1: 1.47 Top4/1: 0.77 Top6/1: 0.48 | 0.54 |
| 10 beams in Set C + Rx beam information | 73.4 | 76.7 | Top2/1: 85.7 Top4/1: 92.3 Top6/1: 95.0 | Top1/1: 2.88 Top2/1: 1.55 Top4/1: 0.81 Top6/1: 0.52 | 0.75 |
| 10 beams in Set C + Tx beam information + Rx beam information | 74.5 | 78.8 | Top2/1: 86.5 Top4/1: 92.9 Top6/1: 95.3 | Top1/1: 2.77 Top2/1: 1.47 Top4/1: 0.75 Top6/1: 0.49 | 0.44 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 10 beams with Tx beam information brings marginal performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 10 beams with Tx beam information as assistance information achieves approximately,

- 75% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.7/1.4/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-7: evaluation results for 11 beams in Set C and Set B of 8 beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 11 beams in Set C | 73.4 | 79.2 | Top2/1: 85.9 Top4/1: 92.4 Top6/1: 94.9 | Top1/1: 2.86 Top2/1: 1.51 Top4/1: 0.79 Top6/1: 0.52 | 0.76 |
| 11 beams in Set C + Tx beam information | 74.9 | 79.3 | Top2/1: 86.6 Top4/1: 92.8 Top6/1: 95.2 | Top1/1: 2.72 Top2/1: 1.46 Top4/1: 0.76 Top6/1: 0.50 | 0.38 |
| 11 beams in Set C + Rx beam information | 73.4 | 78.4 | Top2/1: 85.9 Top4/1: 92.6 Top6/1: 95.1 | Top1/1: 2.86 Top2/1: 1.53 Top4/1: 0.78 Top6/1: 0.50 | 0.76 |
| 11 beams in Set C + Tx beam information + Rx beam information | 74.9 | 79.9 | Top2/1: 86.6 Top4/1: 93.0 Top6/1: 95.5 | Top1/1: 2.71 Top2/1: 1.46 Top4/1: 0.74 Top6/1: 0.47 | 0.54 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 11 beams with Tx beam information brings marginal performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 11 beams with Tx beam information as assistance information achieves approximately,

- 75% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.7/1.4/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.1-8: evaluation results for 12 beams in Set C and Set B of 8 beams that of 1/4 of Set A of beams

| Cases: DL Tx beam prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 12 beams in Set C | 73.9 | 77.6 | Top2/1: 86.5 Top4/1: 92.9 Top6/1: 95.2 | Top1/1: 2.76 Top2/1: 1.44 Top4/1: 0.73 Top6/1: 0.48 | 0.79 |
| 12 beams in Set C + Tx beam information | 75.8 | 80.7 | Top2/1: 87.5 Top4/1: 93.5 Top6/1: 95.8 | Top1/1: 2.61 Top2/1: 1.36 Top4/1: 0.69 Top6/1: 0.45 | 0.44 |
| 12 beams in Set C + Rx beam information | 73.9 | 77.3 | Top2/1: 86.0 Top4/1: 92.7 Top6/1: 95.1 | Top1/1: 2.77 Top2/1: 1.50 Top4/1: 0.75 Top6/1: 0.49 | 0.89 |
| 12 beams in Set C + Tx beam information + Rx beam information | 75.6 | 80.1 | Top2/1: 87.4 Top4/1: 93.5 Top6/1: 95.7 | Top1/1: 2.63 Top2/1: 1.36 Top4/1: 0.69 Top6/1: 0.45 | 0.43 |

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 12 beams with Tx beam information brings some performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 12 beams with Tx beam information as assistance information achieves approximately,

- 76% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/93%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.6/1.3/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/4 Set A, we have,

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, marginal performance improvement can be achieved among different number of beams in Set C with Tx beam information as assistance information.

      - Beam pair prediction

Evaluation cases for Set C with 17 beams and Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 17 beams in Set C

- L1-RSRP of 16 beams with 17 beams in Set C + Tx beam information

- L1-RSRP of 16 beams with 17 beams in Set C + Rx beam information

- L1-RSRP of 16 beams with 17 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 18 beams and Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 18 beams in Set C

- L1-RSRP of 16 beams with 18 beams in Set C + Tx beam information

- L1-RSRP of 16 beams with 18 beams in Set C + Rx beam information

- L1-RSRP of 16 beams with 18 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 19 beams and Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 19 beams in Set C

- L1-RSRP of 16 beams with 19 beams in Set C + Tx beam information

- L1-RSRP of 16 beams with 19 beams in Set C + Rx beam information

- L1-RSRP of 16 beams with 19 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 20 beams and Set B of beams that of 1/16 of Set A of beams with/without assistance information:

- L1-RSRP of 16 beams with 20 beams in Set C

- L1-RSRP of 16 beams with 20 beams in Set C + Tx beam information

- L1-RSRP of 16 beams with 20 beams in Set C + Rx beam information

- L1-RSRP of 16 beams with 20 beams in Set C + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams L1-RSRP in Set A

- Table 4.1.4.2-1: evaluation results for 17 beams in Set C and Set B of 16 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 17 beams in Set C | 52.1 | 61.5 | Top2/1: 69.4 Top4/1: 81.9 Top6/1: 87.0 | Top1/1: 2.54 Top2/1: 1.44 Top4/1: 0.76 Top6/1: 0.51 | 2.96 |
| 17 beams in Set C + Tx beam information | 55.2 | 64.7 | Top2/1: 72.9 Top4/1: 85.0 Top6/1: 90.0 | Top1/1: 2.13 Top2/1: 1.11 Top4/1: 0.54 Top6/1: 0.34 | 2.79 |
| 17 beams in Set C + Rx beam information | 55.6 | 65.5 | Top2/1: 73.1 Top4/1: 85.3 Top6/1: 90.1 | Top1/1: 2.14 Top2/1: 1.11 Top4/1: 0.53 Top6/1: 0.35 | 2.79 |
| 17 beams in Set C + Tx beam information + Rx beam information | 55.1 | 63.2 | Top2/1: 72.7 Top4/1: 84.8 Top6/1: 89.8 | Top1/1: 2.13 Top2/1: 1.12 Top4/1: 0.55 Top6/1: 0.35 | 2.86 |

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 17 beams with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 17 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.1/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-2: evaluation results for 18 beams in Set C and Set B of 16 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 18 beams in Set C | 49.9 | 59.2 | Top2/1: 66.8 Top4/1: 79.3 Top6/1: 84.8 | Top1/1: 2.93 Top2/1: 1.75 Top4/1: 0.97 Top6/1: 0.68 | 3.19 |
| 18 beams in Set C + Tx beam information | 54.9 | 64.2 | Top2/1: 72.5 Top4/1: 84.7 Top6/1: 89.7 | Top1/1: 2.16 Top2/1: 1.14 Top4/1: 0.55 Top6/1: 0.36 | 2.78 |
| 18 beams in Set C + Rx beam information | 54.8 | 63.6 | Top2/1: 72.3 Top4/1: 84.6 Top6/1: 89.4 | Top1/1: 2.15 Top2/1: 1.14 Top4/1: 0.56 Top6/1: 0.37 | 2.78 |
| 18 beams in Set C + Tx beam information + Rx beam information | 54.9 | 63.7 | Top2/1: 72.5 Top4/1: 84.9 Top6/1: 89.8 | Top1/1: 2.16 Top2/1: 1.13 Top4/1: 0.55 Top6/1: 0.35 | 2.86 |

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 18 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 18 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 72%/85%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.1/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-3: evaluation results for 19 beams in Set C and Set B of 16 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 19 beams in Set C | 47.7 | 55.4 | Top2/1: 64.5 Top4/1: 77.4 Top6/1: 83.5 | Top1/1: 3.31 Top2/1: 2.01 Top4/1: 1.14 Top6/1: 0.77 | 3.51 |
| 19 beams in Set C + Tx beam information | 55.5 | 65.4 | Top2/1: 73.1 Top4/1: 85.3 Top6/1: 90.1 | Top1/1: 2.04 Top2/1: 1.06 Top4/1: 0.51 Top6/1: 0.33 | 2.66 |
| 19 beams in Set C + Rx beam information | 54.8 | 63.8 | Top2/1: 72.4 Top4/1: 84.8 Top6/1: 89.7 | Top1/1: 2.09 Top2/1: 1.11 Top4/1: 0.53 Top6/1: 0.34 | 2.68 |
| 19 beams in Set C + Tx beam information + Rx beam information | 55.5 | 66.3 | Top2/1: 73.1 Top4/1: 85.4 Top6/1: 90.2 | Top1/1: 2.05 Top2/1: 1.07 Top4/1: 0.51 Top6/1: 0.32 | 2.78 |

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 19 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 19 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 65% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.0/1.0/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-4: evaluation results for 20 beams in Set C and Set B of 16 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 20 beams in Set C | 47.1 | 56.1 | Top2/1: 64.4 Top4/1: 77.7 Top6/1: 83.4 | Top1/1: 3.39 Top2/1: 2.05 Top4/1: 1.16 Top6/1: 0.80 | 3.45 |
| 20 beams in Set C + Tx beam information | 56.0 | 66.4 | Top2/1: 74.2 Top4/1: 86.4 Top6/1: 91.0 | Top1/1: 1.93 Top2/1: 0.96 Top4/1: 0.44 Top6/1: 0.28 | 2.58 |
| 20 beams in Set C + Rx beam information | 55.5 | 66.0 | Top2/1: 73.8 Top4/1: 86.1 Top6/1: 90.9 | Top1/1: 1.95 Top2/1: 0.99 Top4/1: 0.46 Top6/1: 0.29 | 2.62 |
| 20 beams in Set C + Tx beam information + Rx beam information | 56.5 | 66.9 | Top2/1: 74.3 Top4/1: 86.5 Top6/1: 91.0 | Top1/1: 1.90 Top2/1: 0.96 Top4/1: 0.44 Top6/1: 0.28 | 2.60 |

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 20 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 20 beams with Tx/Rx beam information as assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL Tx beam

- 66% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 74%/86%/91% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.9/0.9/0.4/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/16 Set A, we have,

For beam pair prediction with Set B = 1/16 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For beam pair prediction with Set B = 1/16 Set A, closed performance can be achieved among different number of beams in Set C with Tx/Rx beam information as assistance information.

Evaluation cases for Set C with 33 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 33 beams in Set C

- L1-RSRP of 32 beams with 33 beams in Set C + Tx beam information

- L1-RSRP of 32 beams with 33 beams in Set C + Rx beam information

- L1-RSRP of 32 beams with 33 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 34 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 34 beams in Set C

- L1-RSRP of 32 beams with 34 beams in Set C + Tx beam information

- L1-RSRP of 32 beams with 34 beams in Set C + Rx beam information

- L1-RSRP of 32 beams with 34 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 35 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 35 beams in Set C

- L1-RSRP of 32 beams with 35 beams in Set C + Tx beam information

- L1-RSRP of 32 beams with 35 beams in Set C + Rx beam information

- L1-RSRP of 32 beams with 35 beams in Set C + Tx beam information + Rx beam information

Evaluation cases for Set C with 36 beams and Set B of beams that of 1/8 of Set A of beams with/without assistance information:

- L1-RSRP of 32 beams with 36 beams in Set C

- L1-RSRP of 32 beams with 36 beams in Set C + Tx beam information

- L1-RSRP of 32 beams with 36 beams in Set C + Rx beam information

- L1-RSRP of 32 beams with 36 beams in Set C + Tx beam information + Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams L1-RSRP in Set A

- Table 4.1.4.2-5: evaluation results for 33 beams in Set C and Set B of 32 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 33 beams in Set C | 70.6 | 81.1 | Top2/1: 86.1 Top4/1: 94.0 Top6/1: 96.5 | Top1/1: 0.79 Top2/1: 0.33 Top4/1: 0.14 Top6/1: 0.08 | 1.51 |
| 33 beams in Set C + Tx beam information | 71.7 | 82.8 | Top2/1: 87.1 Top4/1: 94.6 Top6/1: 97.0 | Top1/1: 0.71 Top2/1: 0.29 Top4/1: 0.12 Top6/1: 0.07 | 1.43 |
| 33 beams in Set C + Rx beam information | 72.2 | 83.2 | Top2/1: 87.1 Top4/1: 94.8 Top6/1: 96.9 | Top1/1: 0.70 Top2/1: 0.29 Top4/1: 0.11 Top6/1: 0.07 | 1.41 |
| 33 beams in Set C + Tx beam information + Rx beam information | 71.1 | 80.3 | Top2/1: 86.0 Top4/1: 94.1 Top6/1: 96.6 | Top1/1: 0.72 Top2/1: 0.32 Top4/1: 0.14 Top6/1: 0.08 | 1.44 |

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 33 beams with Tx beam information and/or Rx beam information brings similar performance improvement compared to the same Set C without assistance information, as well as similar performance using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 33 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/94%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.08 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-6: evaluation results for 34 beams in Set C and Set B of 32 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 34 beams in Set C | 70.1 | 80.1 | Top2/1: 85.6 Top4/1: 93.8 Top6/1: 96.3 | Top1/1: 0.84 Top2/1: 0.36 Top4/1: 0.15 Top6/1: 0.09 | 1.56 |
| 34 beams in Set C + Tx beam information | 71.7 | 82.3 | Top2/1: 87.1 Top4/1: 95.1 Top6/1: 97.2 | Top1/1: 0.69 Top2/1: 0.27 Top4/1: 0.10 Top6/1: 0.06 | 1.49 |
| 34 beams in Set C + Rx beam information | 71.4 | 81.3 | Top2/1: 86.6 Top4/1: 94.8 Top6/1: 97.1 | Top1/1: 0.70 Top2/1: 0.29 Top4/1: 0.10 Top6/1: 0.06 | 1.44 |
| 34 beams in Set C + Tx beam information + Rx beam information | 71.8 | 81.9 | Top2/1: 86.8 Top4/1: 94.8 Top6/1: 97.2 | Top1/1: 0.69 Top2/1: 0.28 Top4/1: 0.11 Top6/1: 0.06 | 1.42 |

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 34 beams with Tx beam information and/or Rx beam information brings similar performance improvement compared to the same Set C without assistance information, as well as similar performance using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 34 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-7: evaluation results for 35 beams in Set C and Set B of 32 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 35 beams in Set C | 68.6 | 79.1 | Top2/1: 84.2 Top4/1: 93.0 Top6/1: 95.6 | Top1/1: 0.97 Top2/1: 0.43 Top4/1: 0.18 Top6/1: 0.11 | 1.69 |
| 35 beams in Set C + Tx beam information | 71.5 | 82.6 | Top2/1: 87.1 Top4/1: 95.0 Top6/1: 97.2 | Top1/1: 0.71 Top2/1: 0.28 Top4/1: 0.10 Top6/1: 0.06 | 1.52 |
| 35 beams in Set C + Rx beam information | 71.1 | 81.7 | Top2/1: 86.6 Top4/1: 94.7 Top6/1: 97.1 | Top1/1: 0.72 Top2/1: 0.29 Top4/1: 0.11 Top6/1: 0.06 | 1.46 |
| 35 beams in Set C + Tx beam information + Rx beam information | 70.3 | 81.4 | Top2/1: 86.3 Top4/1: 94.7 Top6/1: 96.9 | Top1/1: 0.75 Top2/1: 0.30 Top4/1: 0.11 Top6/1: 0.06 | 1.58 |

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 35 beams with Tx beam information and/or Rx beam information brings marginal performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 35 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.4.2-8: evaluation results for 36 beams in Set C and Set B of 32 beams that of 1/16 of Set A of beams

| Cases: Beam pair prediction w/wo assistance info. | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| 36 beams in Set C | 66.8 | 77.2 | Top2/1: 82.9 Top4/1: 92.0 Top6/1: 95.0 | Top1/1: 1.10 Top2/1: 0.52 Top4/1: 0.24 Top6/1: 0.14 | 1.81 |
| 36 beams in Set C + Tx beam information | 71.6 | 82.6 | Top2/1: 87.0 Top4/1: 94.8 Top6/1: 97.2 | Top1/1: 0.72 Top2/1: 0.29 Top4/1: 0.11 Top6/1: 0.06 | 1.58 |
| 36 beams in Set C + Rx beam information | 70.8 | 81.6 | Top2/1: 86.5 Top4/1: 94.6 Top6/1: 97.0 | Top1/1: 0.76 Top2/1: 0.31 Top4/1: 0.11 Top6/1: 0.06 | 1.51 |
| 36 beams in Set C + Tx beam information + Rx beam information | 70.7 | 81.6 | Top2/1: 86.6 Top4/1: 94.7 Top6/1: 97.2 | Top1/1: 0.74 Top2/1: 0.29 Top4/1: 0.10 Top6/1: 0.06 | 1.55 |

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 36 beams with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 36 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Compared above tables for Set B = 1/8 Set A, we have,

For beam pair prediction with Set B = 1/8 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For beam pair prediction with Set B = 1/8 Set A, closed performance can be achieved among different number of beams in Set C with Tx/Rx beam information as assistance information.

### Performance with different Rx beam assumption for Set B in DL Tx beam prediction

In RAN1#112bis meeting, following agreement was approved for DL Rx beam assumption for Set B,

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

  - Companies report how to select the “best” Rx beam(s)

- Option 2: Measurements of specific Rx beam(s)

  - Companies report how to select specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

Other options are not precluded and can be reported by companies.

Three alternatives can be defined in Option 1, while three feasible Rx beam solutions are included in Option 2. Thus, we have,

Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- The best Rx beam searched from the best Tx beam within Set A

- The best Rx beam searched from the best Tx beam within Set B

- The best Rx beam for each Tx beam within Set B

Option 2: Measurements of specific Rx beam(s)

- The best Rx beam searched from one specific Tx beam within Set A

- The best Rx beam searched from one specific Tx beam within Set B

- One fixed Rx beam per model input sample

Option 3: Measurements of random Rx beam(s) per model input sample

- One random Rx beam per model input sample

      - Fixed pattern in Set B

Evaluation cases for fixed pattern in Set B of beams that of 1/8 of Set A of beams:

- L1-RSRP of 4 beams with fixed pattern

Evaluation cases for fixed pattern in Set B of beams that of 1/4 of Set A of beams:

- L1-RSRP of 8 beams with fixed pattern

Rx beam assumption for Set B:

- Option 1: The best Rx beam searched from the best Tx beam within Set A

- Option 1: The best Rx beam searched from the best Tx beam within Set B

- Option 1: The best Rx beam for each Tx beam within Set B

- Option 2: The best Rx beam searched from Tx beam-1 within Set A

- Option 2: The best Rx beam searched from Tx beam-16 within Set A

- Option 2: The best Rx beam searched from 1st Tx beam within Set B

- Option 2: The best Rx beam searched from 2nd Tx beam within Set B

- Option 2: Rx beam-1 per model input sample

- Option 2: Rx beam-4 per model input sample

- Option 3: one random Rx beam per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.5.1-1: evaluation results for fixed pattern in Set B of beams that of 1/8 of Set A of beams with option 1

| Cases: DL Tx beam prediction Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 89.0 | 91.1 | Top2/1: 94.9 Top4/1: 97.1 Top6/1: 98.1 | Top1/1: 0.81 Top2/1: 0.45 Top4/1: 0.24 Top6/1: 0.15 | 1.23 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 67.6 | 72.6 | Top2/1: 82.6 Top4/1: 91.0 Top6/1: 94.2 | Top1/1: 3.47 Top2/1: 1.87 Top4/1: 0.92 Top6/1: 0.57 | 0.76 |
| Option 1: The best Rx beam for each Tx beam within Set B | 70.8 | 75.1 | Top2/1: 85.3 Top4/1: 90.5 Top6/1: 93.1 | Top1/1: 3.42 Top2/1: 1.80 Top4/1: 1.08 Top6/1: 0.74 | 3.65 |

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15%-30% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

- Table 4.1.5.1-2: evaluation results for fixed pattern in Set B of beams that of 1/8 of Set A of beams with option 2 and option 3

| Cases: DL Tx beam prediction Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 67.1 | 71.0 | Top2/1: 80.6 Top4/1: 87.7 Top6/1: 91.1 | Top1/1: 3.80 Top2/1: 2.24 Top4/1: 1.37 Top6/1: 0.96 | 1.36 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 66.4 | 70.1 | Top2/1: 81.1 Top4/1: 87.8 Top6/1: 91.3 | Top1/1: 4.01 Top2/1: 2.26 Top4/1: 1.39 Top6/1: 0.96 | 1.33 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 67.2 | 72.0 | Top2/1: 81.2 Top4/1: 89.5 Top6/1: 93.2 | Top1/1: 3.72 Top2/1: 2.15 Top4/1: 1.15 Top6/1: 0.71 | 0.98 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 65 | 69.1 | Top2/1: 80.0 Top4/1: 87.0 Top6/1: 90.9 | Top1/1: 4.17 Top2/1: 2.37 Top4/1: 1.49 Top6/1: 1.01 | 1.35 |
| Option 2: Rx beam-1 | 55.3 | 56.4 | Top2/1: 75.0 Top4/1: 85.6 Top6/1: 90.0 | Top1/1: 5.28 Top2/1: 2.82 Top4/1: 1.54 Top6/1: 1.02 | 1.27 |
| Option 2: Rx beam-4 | 55.6 | 56.9 | Top2/1: 74.6 Top4/1: 85.6 Top6/1: 90.3 | Top1/1: 5.26 Top2/1: 2.87 Top4/1: 1.53 Top6/1: 0.99 | 1.24 |
| Option 3: One random Rx beam per model input sample | 56.9 | 58.0 | Top2/1: 75.8 Top4/1: 86.6 Top6/1: 91.1 | Top1/1: 5.09 Top2/1: 2.72 Top4/1: 1.43 Top6/1: 0.91 | 1.23 |

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 67% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 81%/89%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.1/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.5.1-3: evaluation results for fixed pattern in Set B of beams that of 1/4 of Set A of beams with option 1

| Cases: DL Tx beam prediction Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 97.3 | 98.5 | Top2/1: 99.3 Top4/1: 99.7 Top6/1: 99.8 | Top1/1: 0.08 Top2/1: 0.03 Top4/1: 0.01 Top6/1: 0.01 | 0.4 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 76.4 | 80.6 | Top2/1: 86.5 Top4/1: 93.0 Top6/1: 95.3 | Top1/1: 2.69 Top2/1: 1.53 Top4/1: 0.78 Top6/1: 0.51 | 0.42 |
| Option 1: The best Rx beam for each Tx beam within Set B | 81.1 | 84.1 | Top2/1: 91.4 Top4/1: 95.3 Top6/1: 96.8 | Top1/1: 2.04 Top2/1: 0.92 Top4/1: 0.46 Top6/1: 0.28 | 2.82 |

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15%-30% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, closed performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

- Table 4.1.5.1-4: evaluation results for fixed pattern in Set B of beams that of 1/4 of Set A of beams with option 2

| Cases: DL Tx beam prediction Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 71.6 | 76.1 | Top2/1: 83.0 Top4/1: 90.0 Top6/1: 93.0 | Top1/1: 3.50 Top2/1: 2.07 Top4/1: 1.18 Top6/1: 0.81 | 0.53 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 70.5 | 73.3 | Top2/1: 82.8 Top4/1: 89.9 Top6/1: 93.1 | Top1/1: 3.71 Top2/1: 2.13 Top4/1: 1.23 Top6/1: 0.80 | 0.6 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 70 | 73.3 | Top2/1: 81.9 Top4/1: 88.9 Top6/1: 92.2 | Top1/1: 3.71 Top2/1: 2.21 Top4/1: 1.32 Top6/1: 0.91 | 0.5 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 70 | 73.4 | Top2/1: 82.2 Top4/1: 89.4 Top6/1: 92.5 | Top1/1: 3.76 Top2/1: 2.20 Top4/1: 1.28 Top6/1: 0.88 | 0.47 |
| Option 2: Rx beam-1 | 56.5 | 57.5 | Top2/1: 74.8 Top4/1: 87.1 Top6/1: 91.5 | Top1/1: 5.38 Top2/1: 2.95 Top4/1: 1.42 Top6/1: 0.89 | 0.73 |
| Option 2: Rx beam-4 | 56.9 | 57.8 | Top2/1: 74.9 Top4/1: 87.3 Top6/1: 91.7 | Top1/1: 5.36 Top2/1: 2.95 Top4/1: 1.42 Top6/1: 0.88 | 0.86 |
| Option 3: One random Rx beam per model input sample | 58.5 | 60 | Top2/1: 75.6 Top4/1: 87.7 Top6/1: 92.3 | Top1/1: 5.15 Top2/1: 2.85 Top4/1: 1.35 Top6/1: 0.81 | 0.83 |

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 14% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.1/1.3/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

      - Random pattern in Set B

Evaluation cases for random pattern in Set B of beams that of 1/8 of Set A of beams:

- L1-RSRP of 4 beams with random pattern + Tx beam information

Evaluation cases for random pattern in Set B of beams that of 1/4 of Set A of beams:

- L1-RSRP of 8 beams with random pattern + Tx beam information

Rx beam assumption for Set B:

- Option 1: The best Rx beam searched from the best Tx beam within Set A

- Option 1: The best Rx beam searched from the best Tx beam within Set B

- Option 1: The best Rx beam for each Tx beam within Set B

- Option 2: The best Rx beam searched from Tx beam-1 within Set A

- Option 2: The best Rx beam searched from Tx beam-16 within Set A

- Option 2: The best Rx beam searched from 1st Tx beam within Set B

- Option 2: The best Rx beam searched from 2nd Tx beam within Set B

- Option 2: Rx beam-1 per model input sample

- Option 2: Rx beam-4 per model input sample

- Option 3: one random Rx beam per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.5.2-1: evaluation results for random pattern in Set B of beams that of 1/8 of Set A of beams with option 1

| Cases: DL Tx beam prediction Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 68.9 | 72.2 | Top2/1: 85.3 Top4/1: 93.1 Top6/1: 96.0 | Top1/1: 3.30 Top2/1: 1.45 Top4/1: 0.59 Top6/1: 0.31 | 4.23 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 55.9 | 59.9 | Top2/1: 74.7 Top4/1: 85.2 Top6/1: 89.9 | Top1/1: 5.12 Top2/1: 2.84 Top4/1: 1.58 Top6/1: 1.02 | 3.68 |
| Option 1: The best Rx beam for each Tx beam within Set B | 57.1 | 62.1 | Top2/1: 79.3 Top4/1: 90.8 Top6/1: 94.7 | Top1/1: 4.89 Top2/1: 2.21 Top4/1: 0.89 Top6/1: 0.47 | 5.37 |

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

- Table 4.1.5.2-2: evaluation results for random pattern in Set B of beams that of 1/8 of Set A of beams with option 2 and option 3

| Cases: DL Tx beam prediction Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 54.9 | 58.8 | Top2/1: 74.7 Top4/1: 85.5 Top6/1: 89.7 | Top1/1: 5.39 Top2/1: 2.85 Top4/1: 1.54 Top6/1: 1.05 | 4.33 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 54.7 | 58.5 | Top2/1: 73.9 Top4/1: 83.9 Top6/1: 88.8 | Top1/1: 5.50 Top2/1: 3.00 Top4/1: 1.74 Top6/1: 1.17 | 4.08 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 53.3 | 57.8 | Top2/1: 72.9 Top4/1: 84.7 Top6/1: 89.8 | Top1/1: 5.65 Top2/1: 3.12 Top4/1: 1.65 Top6/1: 1.05 | 4.28 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 53.5 | 57.7 | Top2/1: 73.4 Top4/1: 85.0 Top6/1: 89.9 | Top1/1: 5.63 Top2/1: 3.08 Top4/1: 1.65 Top6/1: 1.06 | 4.01 |
| Option 2: Rx beam-1 | 45.7 | 46.7 | Top2/1: 68.3 Top4/1: 82.9 Top6/1: 88.8 | Top1/1: 6.73 Top2/1: 3.68 Top4/1: 1.84 Top6/1: 1.14 | 4.39 |
| Option 2: Rx beam-4 | 45.6 | 46.4 | Top2/1: 68.4 Top4/1: 83.6 Top6/1: 89.7 | Top1/1: 6.75 Top2/1: 3.68 Top4/1: 1.77 Top6/1: 1.06 | 4.36 |
| Option 3: One random Rx beam per model input sample | 46.0 | 47.1 | Top2/1: 67.8 Top4/1: 82.4 Top6/1: 88.9 | Top1/1: 6.72 Top2/1: 3.78 Top4/1: 1.90 Top6/1: 1.14 | 4.32 |

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 53% beam prediction accuracy of Top-1 DL Tx beam

- 57% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 5.6/3.0/1.7/1.1 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 4.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.5.2-3: evaluation results for random pattern in Set B of beams that of 1/4 of Set A of beams with option 1

| Cases: DL Tx beam prediction Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 84.0 | 86.8 | Top2/1: 94.2 Top4/1: 97.5 Top6/1: 98.6 | Top1/1: 1.24 Top2/1: 0.45 Top4/1: 0.17 Top6/1: 0.08 | 2.76 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 66.7 | 71.4 | Top2/1: 81.6 Top4/1: 89.3 Top6/1: 92.7 | Top1/1: 3.51 Top2/1: 1.98 Top4/1: 1.12 Top6/1: 0.74 | 2.51 |
| Option 1: The best Rx beam for each Tx beam within Set B | 66.3 | 70.6 | Top2/1: 85.0 Top4/1: 93.1 Top6/1: 95.9 ' | Top1/1: 3.47 Top2/1: 1.46 Top4/1: 0.63 Top6/1: 0.34 | 4.68 |

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 12%~18% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

- Table 4.1.5.2-4: evaluation results for random pattern in Set B of beams that of 1/4 of Set A of beams with option 2 and option 3

| Cases: DL Tx beam prediction Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 63.4 | 67.2 | Top2/1: 79.0 Top4/1: 87.2 Top6/1: 91.0 | Top1/1: 4.14 Top2/1: 2.37 Top4/1: 1.41 Top6/1: 0.97 | 2.91 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 62.2 | 66.2 | Top2/1: 78.8 Top4/1: 86.8 Top6/1: 90.8 | Top1/1: 4.38 Top2/1: 2.45 Top4/1: 1.51 Top6/1: 1.03 | 2.79 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 62.6 | 67.3 | Top2/1: 77.9 Top4/1: 86.6 Top6/1: 90.7 | Top1/1: 4.28 Top2/1: 2.54 Top4/1: 1.49 Top6/1: 1.00 | 3.06 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 62.0 | 65.4 | Top2/1: 77.6 Top4/1: 86.7 Top6/1: 90.9 | Top1/1: 4.37 Top2/1: 2.58 Top4/1: 1.48 Top6/1: 0.98 | 2.92 |
| Option 2: Rx beam-1 | 51.3 | 52.9 | Top2/1: 72.1 Top4/1: 84.7 Top6/1: 90.1 | Top1/1: 5.82 Top2/1: 3.20 Top4/1: 1.66 Top6/1: 1.02 | 3.11 |
| Option 2: Rx beam-4 | 51.1 | 52.6 | Top2/1: 71.5 Top4/1: 84.3 Top6/1: 89.5 | Top1/1: 5.89 Top2/1: 3.27 Top4/1: 1.70 Top6/1: 1.10 | 3.05 |
| Option 3: One random Rx beam per model input sample | 52.6 | 53.7 | Top2/1: 72.1 Top4/1: 84.5 Top6/1: 90.1 | Top1/1: 5.67 Top2/1: 3.19 Top4/1: 1.67 Top6/1: 1.03 | 3.00 |

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 63% beam prediction accuracy of Top-1 DL Tx beam

- 67% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 77%/86%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 4.3/2.5/1.5/1.0 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

      - Pre-configured patterns in Set B

Evaluation cases for pre-configured patterns in Set B of beams that of 1/8 of Set A of beams:

- L1-RSRP of 4 beams with 10 pre-configured patterns + Tx beam information

Evaluation cases for pre-configured patterns in Set B of beams that of 1/4 of Set A of beams:

- L1-RSRP of 8 beams with 10 pre-configured patterns + Tx beam information

Rx beam assumption for Set B:

- Option 1: The best Rx beam searched from the best Tx beam within Set A

- Option 1: The best Rx beam searched from the best Tx beam within Set B

- Option 1: The best Rx beam for each Tx beam within Set B

- Option 2: The best Rx beam searched from Tx beam-1 within Set A

- Option 2: The best Rx beam searched from Tx beam-16 within Set A

- Option 2: The best Rx beam searched from 1st Tx beam within Set B

- Option 2: The best Rx beam searched from 2nd Tx beam within Set B

- Option 2: Rx beam-1 per model input sample

- Option 2: Rx beam-4 per model input sample

- Option 3: one random Rx beam per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams L1-RSRP in Set A

- Table 4.1.5.3-1: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with option 1

| Cases: DL Tx beam prediction 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 88.7 | 91.0 | Top2/1: 95.7 Top4/1: 97.7 Top6/1: 98.5 | Top1/1: 0.80 Top2/1: 0.36 Top4/1: 0.18 Top6/1: 0.11 | 1.74 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 68.4 | 73.1 | Top2/1: 82.2 Top4/1: 89.6 Top6/1: 92.9 | Top1/1: 3.47 Top2/1: 1.98 Top4/1: 1.10 Top6/1: 0.72 | 1.34 |
| Option 1: The best Rx beam for each Tx beam within Set B | 76.8 | 81.7 | Top2/1: 90.7 Top4/1: 95.3 Top6/1: 97.0 | Top1/1: 2.26 Top2/1: 0.90 Top4/1: 0.43 Top6/1: 0.27 | 2.99 |

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides the best performance compared to Option 1 with other Rx beam searching methods.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, similar performance of predicted L1-RSRP difference and approximately 20% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam searched from the best Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, approximately 10% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam for each Tx beam within Set B if compared to Option 1 with the best Rx beam for each Tx beam within Set B, while Option 1 with the best Rx beam for each Tx beam within Set B takes significant performance degradation in predicted L1-RSRP difference.

- Table 4.1.5.3-2: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/8 of Set A of beams with option 2 and option 3

| Cases: DL Tx beam prediction 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 66.5 | 69.8 | Top2/1: 81.0 Top4/1: 88.3 Top6/1: 91.5 | Top1/1: 3.90 Top2/1: 2.22 Top4/1: 1.32 Top6/1: 0.92 | 2.05 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 65.5 | 70.1 | Top2/1: 80.9 Top4/1: 87.5 Top6/1: 90.9 | Top1/1: 4.10 Top2/1: 2.29 Top4/1: 1.45 Top6/1: 1.02 | 1.86 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 63.6 | 68.2 | Top2/1: 79.0 Top4/1: 88.5 Top6/1: 92.5 | Top1/1: 4.35 Top2/1: 2.51 Top4/1: 1.25 Top6/1: 0.79 | 1.87 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 65.4 | 69.3 | Top2/1: 80.8 Top4/1: 88.1 Top6/1: 91.5 | Top1/1: 4.14 Top2/1: 2.27 Top4/1: 1.37 Top6/1: 0.94 | 1.81 |
| Option 2: Rx beam-1 | 53.6 | 54.6 | Top2/1: 73.6 Top4/1: 85.1 Top6/1: 89.9 | Top1/1: 5.57 Top2/1: 3.05 Top4/1: 1.61 Top6/1: 1.05 | 2.04 |
| Option 2: Rx beam-4 | 53.7 | 54.9 | Top2/1: 73.9 Top4/1: 85.5 Top6/1: 90.1 | Top1/1: 5.56 Top2/1: 3.00 Top4/1: 1.55 Top6/1: 1.01 | 2.02 |
| Option 3: One random Rx beam per model input sample | 55.1 | 56.6 | Top2/1: 74.6 Top4/1: 85.8 Top6/1: 90.2 | Top1/1: 5.34 Top2/1: 2.93 Top4/1: 1.54 Top6/1: 1.02 | 2.09 |

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 65% beam prediction accuracy of Top-1 DL Tx beam

- 69% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/88%/91% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 4.1/2.2/1.4/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

- Table 4.1.5.3-3: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams with option 1

| Cases: DL Tx beam prediction 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1: The best Rx beam searched from the best Tx beam within Set A | 94.9 | 96.7 | Top2/1: 98.5 Top4/1: 99.4 Top6/1: 99.6 | Top1/1: 0.23 Top2/1: 0.08 Top4/1: 0.03 Top6/1: 0.02 | 0.92 |
| Option 1: The best Rx beam searched from the best Tx beam within Set B | 72.9 | 77.8 | Top2/1: 83.9 Top4/1: 91.5 Top6/1: 94.2 | Top1/1: 3.04 Top2/1: 1.81 Top4/1: 0.92 Top6/1: 0.61 | 0.8 |
| Option 1: The best Rx beam for each Tx beam within Set B | 80.4 | 84.1 | Top2/1: 92.4 Top4/1: 96.7 Top6/1: 98.0 | Top1/1: 1.76 Top2/1: 0.68 Top4/1: 0.28 Top6/1: 0.16 | 2.92 |

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides the best performance compared to Option 1 with other Rx beam searching methods.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, similar performance of predicted L1-RSRP difference and approximately 20% performance degradation can be observed from Option 1 with the best Rx beam searched from the best Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, approximately 14% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam for each Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A, while Option 1 with the best Rx beam for each Tx beam within Set B takes significant performance degradation in predicted L1-RSRP difference.

- Table 4.1.5.3-4: evaluation results for 10 pre-configured patterns in Set B of beams that of 1/4 of Set A of beams with option 2 and option 3

| Cases: DL Tx beam prediction 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 2: The best Rx beam searched from Tx beam-1 within Set A | 70.2 | 74.4 | Top2/1: 82.1 Top4/1: 88.9 Top6/1: 92.0 | Top1/1: 3.62 Top2/1: 2.16 Top4/1: 1.31 Top6/1: 0.91 | 0.92 |
| Option 2: The best Rx beam searched from Tx beam-16 within Set A | 68.6 | 72.2 | Top2/1: 81.7 Top4/1: 88.8 Top6/1: 92.2 | Top1/1: 3.88 Top2/1: 2.27 Top4/1: 1.35 Top6/1: 0.91 | 1.06 |
| Option 2: The best Rx beam searched from 1st Tx beam within Set B | 70.8 | 75.3 | Top2/1: 82.7 Top4/1: 90.0 Top6/1: 93.1 | Top1/1: 3.41 Top2/1: 2.01 Top4/1: 1.13 Top6/1: 0.77 | 1.00 |
| Option 2: The best Rx beam searched from 2nd Tx beam within Set B | 68.8 | 72.6 | Top2/1: 80.7 Top4/1: 89.0 Top6/1: 92.5 | Top1/1: 3.81 Top2/1: 2.36 Top4/1: 1.27 Top6/1: 0.84 | 1.01 |
| Option 2: Rx beam-1 | 54.8 | 56 | Top2/1: 73.1 Top4/1: 85.6 Top6/1: 90.4 | Top1/1: 5.58 Top2/1: 3.15 Top4/1: 1.58 Top6/1: 1.01 | 1.33 |
| Option 2: Rx beam-4 | 55.4 | 56.3 | Top2/1: 73.1 Top4/1: 85.4 Top6/1: 90.2 | Top1/1: 5.54 Top2/1: 3.16 Top4/1: 1.62 Top6/1: 1.04 | 1.42 |
| Option 3: One random Rx beam per model input sample | 56.6 | 57.8 | Top2/1: 73.9 Top4/1: 86.0 Top6/1: 91.1 | Top1/1: 5.38 Top2/1: 3.05 Top4/1: 1.54 Top6/1: 0.94 | 1.36 |

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 15% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.2/1.1/0.8 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

### Performance impact with quantization

In this subsection, we will focus on studying the quantization steps with and without considering the quantization range for both the best beam and other differential beams among different set B scheme.

      - DL Tx beam prediction

Evaluation cases without considering quantization range for fixed pattern in Set B:

- L1-RSRP of 8 beams without quantization

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam

Evaluation cases without considering quantization range for random pattern/pre-configured pattern Set B:

- L1-RSRP of 8 beams without quantization + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam + Tx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams without quantization

- Table 4.1.6.1-1: performance comparison among different quantization steps for fixed pattern in Set B without considering quantization range

| Cases: Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 76.4 | 80.2 | Top2/1: 86.2 Top4/1: 93.0 Top6/1: 95.2 | Top1/1: 2.70 Top2/1: 1.53 Top4/1: 0.77 Top6/1: 0.51 | 0.34 |
| 1 dB for the best beam, 1 dB for others | 72.3 | 76.4 | Top2/1: 81.7 Top4/1: 87.9 Top6/1: 90.4 | Top1/1: 3.04 Top2/1: 1.89 Top4/1: 1.14 Top6/1: 0.89 | 0.47 |
| 1 dB for the best beam, 2 dB for others | 71.8 | 75.5 | Top2/1: 81.6 Top4/1: 87.7 Top6/1: 90.2 | Top1/1: 3.05 Top2/1: 1.90 Top4/1: 1.16 Top6/1: 0.91 | 0.55 |
| 1 dB for the best beam, 3 dB for others | 71.1 | 75.7 | Top2/1: 81.4 Top4/1: 87.6 Top6/1: 90.0 | Top1/1: 3.08 Top2/1: 1.90 Top4/1: 1.15 Top6/1: 0.90 | 0.59 |
| 1 dB for the best beam, 4 dB for others | 70.6 | 76.1 | Top2/1: 81.3 Top4/1: 87.4 Top6/1: 89.8 | Top1/1: 3.11 Top2/1: 1.91 Top4/1: 1.18 Top6/1: 0.92 | 0.66 |
| 1 dB for the best beam, 5 dB for others | 70.0 | 76.3 | Top2/1: 81.0 Top4/1: 87.3 Top6/1: 89.9 | Top1/1: 3.14 Top2/1: 1.94 Top4/1: 1.19 Top6/1: 0.92 | 0.73 |

- Table 4.1.6.1-2: performance comparison among different quantization steps for random patterns in Set B without considering quantization range

| Cases: Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 66.6 | 70.2 | Top2/1: 81.3 Top4/1: 89.1 Top6/1: 92.6 | Top1/1: 3.52 Top2/1: 2.01 Top4/1: 1.14 Top6/1: 0.75 | 2.6 |
| 1 dB for the best beam, 1 dB for others | 62.7 | 66.9 | Top2/1: 76.7 Top4/1: 83.9 Top6/1: 87.5 | Top1/1: 3.91 Top2/1: 2.41 Top4/1: 1.55 Top6/1: 1.15 | 2.63 |
| 1 dB for the best beam, 2 dB for others | 62.5 | 67.1 | Top2/1: 76.7 Top4/1: 84.3 Top6/1: 87.9 | Top1/1: 3.93 Top2/1: 2.39 Top4/1: 1.51 Top6/1: 1.11 | 2.63 |
| 1 dB for the best beam, 3 dB for others | 62.3 | 66.4 | Top2/1: 76.6 Top4/1: 84.0 Top6/1: 87.5 | Top1/1: 3.95 Top2/1: 2.41 Top4/1: 1.54 Top6/1: 1.15 | 2.72 |
| 1 dB for the best beam, 4 dB for others | 62.3 | 66.3 | Top2/1: 76.5 Top4/1: 83.9 Top6/1: 87.5 | Top1/1: 3.95 Top2/1: 2.42 Top4/1: 1.55 Top6/1: 1.15 | 2.83 |
| 1 dB for the best beam, 5 dB for others | 61.1 | 65.8 | Top2/1: 75.9 Top4/1: 83.8 Top6/1: 87.5 | Top1/1: 4.09 Top2/1: 2.47 Top4/1: 1.56 Top6/1: 1.16 | 2.88 |

- Table 4.1.6.1-3: performance comparison among different quantization steps for 10 pre-configured patterns in Set B without considering quantization range

| Cases: 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 72.8 | 78.4 | Top2/1: 84.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.04 Top2/1: 1.81 Top4/1: 0.89 Top6/1: 0.57 | 0.89 |
| 1 dB for the best beam, 1 dB for others | 68.2 | 73.3 | Top2/1: 78.5 Top4/1: 86.2 Top6/1: 89.3 | Top1/1: 3.47 Top2/1: 2.26 Top4/1: 1.32 Top6/1: 0.99 | 0.91 |
| 1 dB for the best beam, 2 dB for others | 67.8 | 72.2 | Top2/1: 78.4 Top4/1: 85.8 Top6/1: 89.0 | Top1/1: 3.51 Top2/1: 2.28 Top4/1: 1.36 Top6/1: 1.01 | 0.97 |
| 1 dB for the best beam, 3 dB for others | 67.8 | 72.3 | Top2/1: 78.5 Top4/1: 86.1 Top6/1: 89.3 | Top1/1: 3.49 Top2/1: 2.25 Top4/1: 1.33 Top6/1: 0.99 | 1.05 |
| 1 dB for the best beam, 4 dB for others | 66.9 | 71.2 | Top2/1: 78.0 Top4/1: 85.6 Top6/1: 88.9 | Top1/1: 3.58 Top2/1: 2.32 Top4/1: 1.39 Top6/1: 1.03 | 1.22 |
| 1 dB for the best beam, 5 dB for others | 66.4 | 72.4 | Top2/1: 78.0 Top4/1: 85.6 Top6/1: 88.7 | Top1/1: 3.59 Top2/1: 2.31 Top4/1: 1.38 Top6/1: 1.04 | 1.37 |

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 4~-5% performance loss in beam prediction accuracy compared to non-quantitative case, as well as negligible performance loss in predicted L1-RSRP difference.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

Evaluation cases with considering quantization range for fixed pattern in Set B:

- L1-RSRP of 8 beams without quantization

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam

Evaluation cases with considering quantization range for random pattern/pre-configured pattern Set B:

- L1-RSRP of 8 beams without quantization + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 8 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam + Tx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- L1-RSRP of 32 beams without quantization

- Table 4.1.6.1-4: performance comparison among different quantization steps for fixed pattern in Set B with considering quantization range

| Cases: Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 76.4 | 80.2 | top2/1: 86.2 top4/1: 93.0 top6/1: 95.2 | top1/1: 2.70 top2/1: 1.53 top4/1: 0.77 top6/1: 0.51 | 0.34 |
| 1 dB for the best beam, 1 dB for others | 67.4 | 71.4 | top2/1: 78.5 top4/1: 86.1 top6/1: 88.4 | top1/1: 4.07 top2/1: 2.42 top4/1: 1.30 top6/1: 0.98 | 1.29 |
| 1 dB for the best beam, 2 dB for others | 69.4 | 73.7 | top2/1: 80.2 top4/1: 86.9 top6/1: 88.9 | top1/1: 3.69 top2/1: 2.23 top4/1: 1.22 top6/1: 0.95 | 1.60 |
| 1 dB for the best beam, 3 dB for others | 66.6 | 71.6 | top2/1: 79.0 top4/1: 86.4 top6/1: 88.6 | top1/1: 4.05 top2/1: 2.35 top4/1: 1.27 top6/1: 0.96 | 1.40 |
| 1 dB for the best beam, 4 dB for others | 66.5 | 72.1 | top2/1: 79.2 top4/1: 86.5 top6/1: 88.8 | top1/1: 4.06 top2/1: 2.33 top4/1: 1.26 top6/1: 0.95 | 1.51 |
| 1 dB for the best beam, 5 dB for others | 65.6 | 71.0 | top2/1: 78.1 top4/1: 85.5 top6/1: 88.0 | top1/1: 4.14 top2/1: 2.46 top4/1: 1.37 top6/1: 1.03 | 1.49 |

- Table 4.1.6.1-5: performance comparison among different quantization steps for random patterns in Set B with considering quantization range

| Cases: Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 66.6 | 70.2 | Top2/1: 81.3 Top4/1: 89.1 Top6/1: 92.6 | Top1/1: 3.52 Top2/1: 2.01 Top4/1: 1.14 Top6/1: 0.75 | 2.6 |
| 1 dB for the best beam, 1 dB for others | 56.4 | 59.7 | Top2/1: 72.5 Top4/1: 82.3 Top6/1: 86.3 | Top1/1: 5.25 Top2/1: 3.08 Top4/1: 1.72 Top6/1: 1.21 | 4.07 |
| 1 dB for the best beam, 2 dB for others | 56.4 | 60.4 | Top2/1: 73.0 Top4/1: 83.1 Top6/1: 87.1 | Top1/1: 5.25 Top2/1: 3.08 Top4/1: 1.63 Top6/1: 1.10 | 4.0 |
| 1 dB for the best beam, 3 dB for others | 56.8 | 60.6 | Top2/1: 73.1 Top4/1: 83.0 Top6/1: 87.0 | Top1/1: 5.21 Top2/1: 3.05 Top4/1: 1.64 Top6/1: 1.11 | 4.15 |
| 1 dB for the best beam, 4 dB for others | 56.4 | 60.1 | Top2/1: 73.0 Top4/1: 82.6 Top6/1: 86.6 | Top1/1: 5.23 Top2/1: 3.04 Top4/1: 1.70 Top6/1: 1.18 | 4.21 |
| 1 dB for the best beam, 5 dB for others | 56.1 | 60.2 | Top2/1: 72.8 Top4/1: 82.5 Top6/1: 86.5 | Top1/1: 5.26 Top2/1: 3.06 Top4/1: 1.70 Top6/1: 1.19 | 4.18 |

- Table 4.1.6.1-6: performance comparison among different quantization steps for 10 pre-configured patterns in Set B with considering quantization range

| Cases: 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 72.8 | 78.4 | Top2/1: 84.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.04 Top2/1: 1.81 Top4/1: 0.89 Top6/1: 0.57 | 0.89 |
| 1 dB for the best beam, 1 dB for others | 61.7 | 65.1 | Top2/1: 74.5 Top4/1: 84.6 Top6/1: 88.1 | Top1/1: 4.75 Top2/1: 2.97 Top4/1: 1.44 Top6/1: 1.01 | 2.55 |
| 1 dB for the best beam, 2 dB for others | 62.6 | 66.3 | Top2/1: 75.8 Top4/1: 85.3 Top6/1: 88.2 | Top1/1: 4.52 Top2/1: 2.77 Top4/1: 1.39 Top6/1: 1.01 | 2.5 |
| 1 dB for the best beam, 3 dB for others | 62.5 | 66.6 | Top2/1: 76.2 Top4/1: 85.4 Top6/1: 88.2 | Top1/1: 4.51 Top2/1: 2.67 Top4/1: 1.35 Top6/1: 1.00 | 2.52 |
| 1 dB for the best beam, 4 dB for others | 62.3 | 66.3 | Top2/1: 75.7 Top4/1: 84.7 Top6/1: 87.6 | Top1/1: 4.53 Top2/1: 2.70 Top4/1: 1.42 Top6/1: 1.04 | 2.62 |
| 1 dB for the best beam, 5 dB for others | 61.6 | 66.3 | Top2/1: 75.6 Top4/1: 84.6 Top6/1: 87.8 | Top1/1: 4.55 Top2/1: 2.74 Top4/1: 1.44 Top6/1: 1.04 | 2.69 |

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 7~10% performance loss in beam prediction accuracy and 1.5dB degradation in predicted L1-RSRP difference compared to non-quantitative case.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

      - Beam pair prediction

Evaluation cases without considering quantization range for fixed pattern in Set B:

- L1-RSRP of 16 beams without quantization

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam

Evaluation cases without considering quantization range for random pattern/pre-configured pattern Set B:

- L1-RSRP of 16 beams without quantization + Tx/Rx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam + Tx/Rx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam + Tx/Rx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam + Tx/Rx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam + Tx/Rx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam + Tx/Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams without quantization

- Table 4.1.6.2-1: performance comparison among different quantization steps for fixed pattern in Set B without considering quantization range

| Cases: Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 56 | 65.2 | Top2/1: 72.9 Top4/1: 84.7 Top6/1: 89.7 | Top1/1: 2.01 Top2/1: 1.06 Top4/1: 0.52 Top6/1: 0.33 | 2.48 |
| 1 dB for the best beam, 1 dB for others | 55.4 | 64.1 | Top2/1: 71.9 Top4/1: 83.9 Top6/1: 89.3 | Top1/1: 2.02 Top2/1: 1.10 Top4/1: 0.55 Top6/1: 0.35 | 2.48 |
| 1 dB for the best beam, 2 dB for others | 55 | 65.6 | Top2/1: 71.8 Top4/1: 84.1 Top6/1: 89.1 | Top1/1: 2.11 Top2/1: 1.14 Top4/1: 0.55 Top6/1: 0.36 | 2.61 |
| 1 dB for the best beam, 3 dB for others | 53.7 | 64.7 | Top2/1: 70.8 Top4/1: 83.5 Top6/1: 88.7 | Top1/1: 2.19 Top2/1: 1.18 Top4/1: 0.58 Top6/1: 0.38 | 2.69 |
| 1 dB for the best beam, 4 dB for others | 52.3 | 62.5 | Top2/1: 69.8 Top4/1: 82.8 Top6/1: 88.2 | Top1/1: 2.33 Top2/1: 1.25 Top4/1: 0.60 Top6/1: 0.39 | 2.8 |
| 1 dB for the best beam, 5 dB for others | 50.8 | 60.1 | Top2/1: 68.0 Top4/1: 81.3 Top6/1: 87.1 | Top1/1: 2.47 Top2/1: 1.37 Top4/1: 0.69 Top6/1: 0.45 | 2.88 |

- Table 4.1.6.2-2: performance comparison among different quantization steps for random patterns in Set B without considering quantization range

| Cases: Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 21.2 | 27.7 | Top2/1: 36.8 Top4/1: 55.5 Top6/1: 66.7 | Top1/1: 7.55 Top2/1: 4.98 Top4/1: 2.79 Top6/1: 1.83 | 5.84 |
| 1 dB for the best beam, 1 dB for others | 21.1 | 28 | Top2/1: 36.3 Top4/1: 54.9 Top6/1: 66.2 | Top1/1: 7.58 Top2/1: 5.06 Top4/1: 2.84 Top6/1: 1.88 | 5.99 |
| 1 dB for the best beam, 2 dB for others | 21.1 | 28.2 | Top2/1: 36.5 Top4/1: 55.4 Top6/1: 66.5 | Top1/1: 7.63 Top2/1: 5.07 Top4/1: 2.84 Top6/1: 1.86 | 5.96 |
| 1 dB for the best beam, 3 dB for others | 20.6 | 26.1 | Top2/1: 35.9 Top4/1: 54.9 Top6/1: 66.2 | Top1/1: 7.67 Top2/1: 5.12 Top4/1: 2.85 Top6/1: 1.88 | 5.86 |
| 1 dB for the best beam, 4 dB for others | 20.8 | 25.9 | Top2/1: 36.0 Top4/1: 54.2 Top6/1: 65.8 | Top1/1: 7.69 Top2/1: 5.11 Top4/1: 2.91 Top6/1: 1.89 | 5.78 |
| 1 dB for the best beam, 5 dB for others | 19.2 | 26.3 | Top2/1: 33.9 Top4/1: 52.7 Top6/1: 64.1 | Top1/1: 8.08 Top2/1: 5.48 Top4/1: 3.10 Top6/1: 2.03 | 6.16 |

- Table 4.1.6.2-3: performance comparison among different quantization steps for 10 pre-configured patterns in Set B without considering quantization range

| Cases: 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 51.9 | 62.1 | Top2/1: 70.2 Top4/1: 83.8 Top6/1: 89.4 | Top1/1: 2.31 Top2/1: 1.20 Top4/1: 0.56 Top6/1: 0.36 | 2.92 |
| 1 dB for the best beam, 1 dB for others | 51.8 | 62.1 | Top2/1: 70.0 Top4/1: 83.8 Top6/1: 89.4 | Top1/1: 2.34 Top2/1: 1.21 Top4/1: 0.56 Top6/1: 0.35 | 2.94 |
| 1 dB for the best beam, 2 dB for others | 51.3 | 61.5 | Top2/1: 69.8 Top4/1: 83.7 Top6/1: 89.2 | Top1/1: 2.38 Top2/1: 1.22 Top4/1: 0.57 Top6/1: 0.36 | 2.97 |
| 1 dB for the best beam, 3 dB for others | 50.4 | 60.2 | Top2/1: 69.3 Top4/1: 83.4 Top6/1: 89.3 | Top1/1: 2.46 Top2/1: 1.27 Top4/1: 0.59 Top6/1: 0.35 | 3.07 |
| 1 dB for the best beam, 4 dB for others | 49.1 | 58.4 | Top2/1: 68.4 Top4/1: 82.5 Top6/1: 88.7 | Top1/1: 2.59 Top2/1: 1.33 Top4/1: 0.62 Top6/1: 0.37 | 3.21 |
| 1 dB for the best beam, 5 dB for others | 48.1 | 58 | Top2/1: 67.1 Top4/1: 81.8 Top6/1: 88.1 | Top1/1: 2.69 Top2/1: 1.41 Top4/1: 0.66 Top6/1: 0.40 | 3.25 |

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 0.1~1% performance loss in beam prediction accuracy compared to non-quantitative case, as well as negligible performance loss in predicted L1-RSRP difference.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

Evaluation cases with considering quantization range for fixed pattern in Set B:

- L1-RSRP of 16 beams without quantization

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam

Evaluation cases with considering quantization range for random pattern/pre-configured pattern in Set B:

- L1-RSRP of 16 beams without quantization + Tx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 1 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 2 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 3 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 4 dB quantization step for the difference to the best beam + Tx beam information

- L1-RSRP of 16 beams with 1 dB quantization step for the best beam, 5 dB quantization step for the difference to the best beam + Tx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams without quantization

- Table 4.1.6.2-4: performance comparison among different quantization steps for fixed pattern in Set B with considering quantization range

| Cases: Fixed pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 56 | 65.2 | Top2/1: 72.9 Top4/1: 84.7 Top6/1: 89.7 | Top1/1: 2.01 Top2/1: 1.06 Top4/1: 0.52 Top6/1: 0.33 | 2.48 |
| 1 dB for the best beam, 1 dB for others | 37.7 | 44.2 | Top2/1: 52.1 Top4/1: 64.7 Top6/1: 71.5 | Top1/1: 6.38 Top2/1: 4.40 Top4/1: 3.12 Top6/1: 2.49 | 4.20 |
| 1 dB for the best beam, 2 dB for others | 37.8 | 44.6 | Top2/1: 51.9 Top4/1: 64.9 Top6/1: 72.0 | Top1/1: 6.18 Top2/1: 4.24 Top4/1: 2.66 Top6/1: 2.06 | 4.22 |
| 1 dB for the best beam, 3 dB for others | 37.5 | 47.2 | Top2/1: 52.1 Top4/1: 65.0 Top6/1: 71.7 | Top1/1: 6.06 Top2/1: 4.35 Top4/1: 3.02 Top6/1: 2.13 | 4.81 |
| 1 dB for the best beam, 4 dB for others | 37.1 | 44.2 | Top2/1: 51.8 Top4/1: 64.7 Top6/1: 71.2 | Top1/1: 6.03 Top2/1: 4.03 Top4/1: 2.58 Top6/1: 1.99 | 4.34 |
| 1 dB for the best beam, 5 dB for others | 35.9 | 43.7 | Top2/1: 50.8 Top4/1: 64.0 Top6/1: 71.4 | Top1/1: 6.16 Top2/1: 4.42 Top4/1: 2.71 Top6/1: 2.05 | 4.44 |

- Table 4.1.6.2-5: performance comparison among different quantization steps for random patterns in Set B with considering quantization range

| Cases: Random pattern | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 21.2 | 27.7 | Top2/1: 36.8 Top4/1: 55.5 Top6/1: 66.7 | Top1/1: 7.55 Top2/1: 4.98 Top4/1: 2.79 Top6/1: 1.83 | 5.84 |
| 1 dB for the best beam, 1 dB for others | 11.2 | 13.6 | Top2/1: 21.0 Top4/1: 35.7 Top6/1: 46.0 | Top1/1: 12.7 Top2/1: 9.79 Top4/1: 6.65 Top6/1: 5.02 | 8.79 |
| 1 dB for the best beam, 2 dB for others | 11.3 | 16.8 | Top2/1: 21.1 Top4/1: 35.7 Top6/1: 45.9 | Top1/1: 12.5 Top2/1: 9.60 Top4/1: 6.56 Top6/1: 4.94 | 9.43 |
| 1 dB for the best beam, 3 dB for others | 11.7 | 17.3 | Top2/1: 21.7 Top4/1: 36.4 Top6/1: 46.6 | Top1/1: 12.4 Top2/1: 9.53 Top4/1: 6.48 Top6/1: 4.87 | 9.00 |
| 1 dB for the best beam, 4 dB for others | 11.5 | 17.8 | Top2/1: 21.3 Top4/1: 35.7 Top6/1: 45.8 | Top1/1: 12.6 Top2/1: 9.77 Top4/1: 6.71 Top6/1: 5.11 | 9.63 |
| 1 dB for the best beam, 5 dB for others | 11.6 | 18.5 | Top2/1: 21.6 Top4/1: 36.7 Top6/1: 47.3 | Top1/1: 12.8 Top2/1: 9.56 Top4/1: 6.44 Top6/1: 4.79 | 10.15 |

- Table 4.1.6.2-6: performance comparison among different quantization steps for 10 pre-configured patterns in Set B with considering quantization range

| Cases: 10 pre-configured patterns | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Baseline: without quantization | 51.9 | 62.1 | Top2/1: 70.2 Top4/1: 83.8 Top6/1: 89.4 | Top1/1: 2.31 Top2/1: 1.20 Top4/1: 0.56 Top6/1: 0.36 | 2.92 |
| 1 dB for the best beam, 1 dB for others | 34.7 | 41.8 | Top2/1: 50.3 Top4/1: 64.1 Top6/1: 71.3 | Top1/1: 6.45 Top2/1: 4.70 Top4/1: 3.20 Top6/1: 2.49 | 5.12 |
| 1 dB for the best beam, 2 dB for others | 35.3 | 43.3 | Top2/1: 50.9 Top4/1: 64.8 Top6/1: 72.1 | Top1/1: 6.25 Top2/1: 4.30 Top4/1: 2.83 Top6/1: 2.12 | 4.88 |
| 1 dB for the best beam, 3 dB for others | 34.8 | 42.9 | Top2/1: 50.8 Top4/1: 64.6 Top6/1: 71.8 | Top1/1: 6.39 Top2/1: 4.46 Top4/1: 2.92 Top6/1: 2.12 | 5.16 |
| 1 dB for the best beam, 4 dB for others | 35.2 | 43.3 | Top2/1: 51.0 Top4/1: 65.1 Top6/1: 72.3 | Top1/1: 6.30 Top2/1: 4.38 Top4/1: 2.81 Top6/1: 2.10 | 4.85 |
| 1 dB for the best beam, 5 dB for others | 34.2 | 42.1 | Top2/1: 50.4 Top4/1: 64.7 Top6/1: 72.2 | Top1/1: 6.35 Top2/1: 4.58 Top4/1: 3.14 Top6/1: 2.38 | 5.24 |

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 15~20% performance loss in beam prediction accuracy and 2~3.5dB degradation in predicted L1-RSRP difference compared to non-quantitative case.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

### Performance with different label

In RAN1#112bis meeting, 5 options were agreed to further study performance with different types of label,

- Option 1a: Top-1 beam(pair) in Set A

- Option 1b: Top-K beam (pair)s in Set A

- Option 2a: L1-RSRPs per beam of all the beams(pairs) in Set A

- Option 2b: Top-K beam(pair)s in Set A and the corresponding L1-RSRPs

- Option 2c: Top-1 beam(pair) in Set A and the corresponding L1-RSRP

Option 1a and Option 2a are evaluated as preliminary results in the following subsections.

      - DL Tx beam prediction

Evaluation case for fixed pattern in Set B:

- L1-RSRP of 8 beams

Evaluation case for various Set B:

- L1-RSRP of 8 beams + Tx beam information

Rx beam assumption for Set B:

- The best Rx beam searched from the best Tx beam within Set B per model input sample

Evaluation assumption for label:

- Option 1a: Top-1 beam in Set A

- Option 2a: L1-RSRP of 32beams in Set A

- Table 4.1.7.1-1: evaluation results compassion for option 1a/2a with different Set B schemes

| Cases: DL Tx beam prediction | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1a for fixed pattern | 76.9 | N/A | Top2/1: 90.0 Top4/1: 98.2 Top6/1: 99.5 | Top1/1: 2.64 Top2/1: 1.16 Top4/1: 0.19 Top6/1: 0.04 | N/A |
| Option 2a for fixed pattern | 76.4 | 80.2 | Top2/1: 86.2 Top4/1: 93.0 Top6/1: 95.2 | Top1/1: 2.70 Top2/1: 1.53 Top4/1: 0.77 Top6/1: 0.51 | 0.34 |
| Option 1a for random pattern | 62.5 | N/A | Top2/1: 82.7 Top4/1: 94.7 Top6/1: 98.5 | Top1/1: 4.14 Top2/1: 1.82 Top4/1: 0.48 Top6/1: 0.11 | N/A |
| Option 2a for random pattern | 66.6 | 70.2 | Top2/1: 81.3 Top4/1: 89.1 Top6/1: 92.6 | Top1/1: 3.52 Top2/1: 2.01 Top4/1: 1.14 Top6/1: 0.75 | 2.6 |
| Option 1a for 10 pre-configured patterns | 72 | N/A | Top2/1: 86.1 Top4/1: 96.0 Top6/1: 99.0 | Top1/1: 3.12 Top2/1: 1.54 Top4/1: 0.40 Top6/1: 0.08 | N/A |
| Option 2a for 10 pre-configured patterns | 72.8 | 78.4 | Top2/1: 84.0 Top4/1: 91.7 Top6/1: 94.5 | Top1/1: 3.04 Top2/1: 1.81 Top4/1: 0.89 Top6/1: 0.57 | 0.89 |
| Option 1a for Set C with 12 beams | 75.5 | N/A | Top2/1: 83.9 Top4/1: 94.0 Top6/1: 98.5 | Top1/1: 2.63 Top2/1: 1.76 Top4/1: 0.60 Top6/1: 0.14 | N/A |
| Option 2a for Set C with 12 beams | 75.8 | 80.7 | Top2/1: 87.5 Top4/1: 93.5 Top6/1: 95.8 | Top1/1: 2.61 Top2/1: 1.36 Top4/1: 0.69 Top6/1: 0.45 | 0.44 |

For DL Tx beam prediction, Option 1a and Option 2a provide identical performance of beam prediction accuracy with different Set B schemes, i.e. fixed pattern, random pattern, pre-configured pattern and Set C, while option 2a can provide additional predicted beam quality information.

      - Beam pair prediction

Evaluation case for fixed pattern in Set B:

- L1-RSRP of 16 beams

Evaluation case for various Set B:

- L1-RSRP of 16 beams + Tx beam information

Evaluation assumption for label:

- Option 1a: Top-1 beam in Set A

- Option 2a: L1-RSRP of 256 beams in Set A

- Table 4.1.7.2-1: evaluation results compassion for option 1a/2a with different Set B schemes

| Cases: Beam pair prediction | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Option 1a for fixed pattern | 53.9 | N/A | Top2/1: 73.0 Top4/1: 87.3 Top6/1: 92.8 | Top1/1: 2,90 Top2/1: 1.20 Top4/1: 0.40 Top6/1: 0.19 | N/A |
| Option 2a for fixed pattern | 55.9 | 65.6 | Top2/1: 72.5 Top4/1: 84.5 Top6/1: 89.4 | Top1/1: 2.01 Top2/1: 1.08 Top4/1: 0.53 Top6/1: 0.34 | 2.45 |
| Option 1a for random pattern | 11.5 | N/A | Top2/1: 21.6 Top4/1: 37.8 Top6/1: 49.9 | Top1/1: 11.6 Top2/1: 7.73 Top4/1: 4.72 Top6/1: 3.33 | N/A |
| Option 2a for random pattern | 21.2 | 27.7 | Top2/1: 36.8 Top4/1: 55.5 Top6/1: 66.7 | Top1/1: 7.55 Top2/1: 4.98 Top4/1: 2.79 Top6/1: 1.83 | 5.84 |
| Option 1a for 10 pre-configured patterns | 46.2 | N/A | Top2/1: 66.4 Top4/1: 82.8 Top6/1: 89.7 | Top1/1: 3.59 Top2/1: 1.60 Top4/1: 0.60 Top6/1: 0.31 | N/A |
| Option 2a for 10 pre-configured patterns | 51.9 | 62.1 | Top2/1: 70.2 Top4/1: 83.8 Top6/1: 89.4 | Top1/1: 2.31 Top2/1: 1.20 Top4/1: 0.56 Top6/1: 0.36 | 2.92 |
| Option 1a for Set C with 12 beams | 42.4 | N/A | Top2/1: 60.4 Top4/1: 76.6 Top6/1: 84.2 | Top1/1: 4.43 Top2/1: 2.16 Top4/1: 0.96 Top6/1: 0.57 | N/A |
| Option 2a for Set C with 12 beams | 56.5 | 66.9 | Top2/1: 74.3 Top4/1: 86.5 Top6/1: 91.0 | Top1/1: 1.90 Top2/1: 0.96 Top4/1: 0.44 Top6/1: 0.28 | 2.6 |

For beam pair prediction, Option 2a provides a considerable performance improvement in beam prediction accuracy compared to Option 1a among different Set B schemes, i.e. fixed pattern, random pattern, pre-configured pattern and Set C, and additionally yields predicted beam quality information.

### Performance with proprietary processed assistance information

      - Beam pair prediction

From theoretical analysis, 4 types of AI input combination can be obtained for beam pair prediction with pre-configured pattern selection as below. Besides, in the following performance evaluation, a value-based mathematical function is used to map real beam information to a virtual beam information, for example, beam pointing angle of 70 degree may be mapped to 1.8243 after proprietary processing module. Such mapping function is same for training and inference.

Evaluation cases for 50 pre-configured patterns in Set B:

- L1-RSRP of 16 beams + Tx beam information + Rx beam information

- L1-RSRP of 16 beams + proprietary processed Tx beam information + Rx beam information

- L1-RSRP of 16 beams + Tx beam information + proprietary processed Rx beam information

- L1-RSRP of 16 beams + proprietary processed Tx beam information + proprietary processed Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams in Set A

Table 4.1.8.1-1: performance evaluation results for proprietary protection mechanism with a value-based mathematical function

| Cases: Beam pair prediction | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Pre-configured 50 patterns + Tx beam information + Rx beam information | 46.7 | 56.4 | Top-4/1: 80.7 | Top-1/1: 2.82 | N/A |
| Pre-configured 50 patterns + proprietary processed Tx beam information + Rx beam information | 46.9 | 56.4 | Top-4/1: 81.1 | Top-1/1: 2.82 | N/A |
| Pre-configured 50 patterns + Tx beam information + proprietary processed Rx beam information | 47.5 | 56.7 | Top-4/1: 81.0 | Top-1/1: 2.79 | N/A |
| Pre-configured 50 patterns + proprietary processed Tx beam information + proprietary processed Rx beam information | 47.5 | 56.0 | Top-4/1: 81.0 | Top-1/1: 2.79 | N/A |

No performance loss can be observed from proprietary protection with mathematical function processing compared to beam prediction using beam angle directly, if a same processing function is maintained for training and inference.

### Performance of beam prediction with expected beam information

      - Beam pair prediction

According to above discussions on expected information schemes and simulation assumptions, we have following evaluation cases:

Evaluation cases for 50 pre-configured patterns in Set B:

- L1-RSRP of 16 beams + Tx /Rx beam information

- L1-RSRP of 16 beams + Tx /Rx beam information + 1 expected Rx beam information

- L1-RSRP of 16 beams + Tx /Rx beam information + 1 expected Tx beam information

- L1-RSRP of 16 beams + Tx /Rx beam information + 1 expected Tx beam information + 1 expected Rx beam information

Evaluation assumption for label:

- L1-RSRP of 256 beams in Set A

- The output of AI mode in Pre-configured 50 patterns + Tx/Rx beam information as baseline beam pair prediction solution is associated with the total number of Tx beams and the total number of Rx beams, while Pre-configured 50 patterns + Tx/Rx beam information with expected beam information in AI input can be used as enhanced beam pair prediction solution.

- Table 4.1.9.1-1: performance comparison for expected beam information

| Cases: Beam pair prediction | Beam prediction accuracy [%] |  |  | Average L1-RSRP diff.[dB] | Predicted L1-RSRP [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-1 1dB margin | Top-2/1, Top-4/1, Top-6/1 | Top-1/1, Top-2/1, Top-4/1, Top-6/1 | Top-1/1 |
| Pre-configured 50 patterns + Tx/Rx beam information | 46.7 | 56.4 | Top-4/1: 80.7 | Top-1/1: 2.82 | N/A |
| Pre-configured 50 patterns + Tx/Rx beam information + 1 expected Rx beam information | 46.8 | 55.6 | Top-4/1: 81.0 | Top-1/1: 3.09 | N/A |
| Pre-configured 50 patterns + Tx/Rx beam information + 1 expected Tx beam information | 46.5 | 54.4 | Top-4/1: 79.9 | Top-1/1: 2.91 | N/A |
| Pre-configured 50 patterns + Tx/Rx beam information + 1 expected Tx beam information + 1 expected Rx beam information | 43.6 | 49.9 | Top-4/1: 78.5 | Top-1/1: 3.27 | N/A |

Based on the above simulation results, almost same beam prediction accuracy and marginal performance loss of average RSRP difference can be obtained by Pre-configured 50 patterns + Tx/Rx beam information with expected Rx beam scheme or expected Tx beam scheme in comparison with Pre-configured 50 patterns + Tx/Rx beam information, while Pre-configured 50 patterns + Tx/Rx beam information with additional expected Tx beam + expected Rx beam brings a small performance deterioration. Thus, we have following observation and proposals,

More flexible AI model deployment for different number of Tx/Rx beams can be achieved through using expected Tx/Rx beam information method with only marginal performance loss.

Evaluation results for BM-Case2

### Performance with different pattern assumptions in Set B

      - DL Tx beam prediction

In this subsection, different Set B assumptions for BM Case 2 are evaluated. In BM Case 2, there are more than one time instance, and the Set B patterns used in each time instance can be different or same. Here, we assume the following options as in Figure 4.2.1.1-1.

- Option 1: different pattern is assumed for different measurement occasion within T1,

  - Option 1a: size of Set B is number of unique beams in all occasions.

  - Option 1b: size of Set B is number of unique beams per occasion.

- Option 2: same pattern is assumed for each measurement occasion within T1, and size of Set B is number of unique beams in all occasions/per occasion.

For evaluation to compare Option 1 and Option 2, we simulate two configurations for Option 2 with different RS overhead.

  - Option 2 – Configuration a: same pattern in each time instance of T1 to traverse all patterns in Option 1. Hence the RS overhead is higher than Option 1

  - Option 2 – Configuration b: same pattern in each time instance of T1, where RS overhead is same as Option 1

Option 2: same pattern is assumed for each measurement occasion within T1 and size of Set B is number of unique beams in all occasions/per occasion.

- Configuration a: same pattern in each time instance of T1 to traverse all patterns in Option 1. Hence the RS overhead is higher than Option 1. (size of Set B =16, required RS resources = 32)

- Configuration b: same pattern in each time instance of T1, where RS overhead is same as Option 1. (size of Set B =8, required RS resources = 16)

Option 1: different pattern is assumed for different measurement occasion within T1

- Option 1a: size of Set B is number of unique beams in all occasion. (size of Set B = 16, required RS resources = 16)

- Option 1b: size of Set B is number of unique beams per occasions. (size of Set B = 8, required RS resources = 16)

Figure 4.2.1.1-1: Options for Set B pattern in each time instance in BM Case2

Assume that T1 = 80ms, M = 2, and T2= 80ms, P =1/2/4. Set A has 32 Tx beams. UE speed is 30km/h. Rx beam in T1 is the Rx beam of best beam pair within Set A and Rx beam in T2 is the same as that of T1. Different Set B assumptions are compared in Table 4.2.1.1-1.

Table 4.2.1.1-1: performance comparison of different Set B assumptions

| Set B assumptions | Measured beams per time instance | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] based on all beam pairs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  |  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Option 1 | 8 | 93.4 | 92.4 | 90.5 | 96.1 | 95.3 | 93.7 | 0.30 | 0.40 | 0.60 |
| Option 2a | 16 | 94.5 | 93.7 | 92.1 | 96.3 | 95.4 | 93.9 | 0.27 | 0.37 | 0.56 |
| Option 2b | 8 | 91.3 | 90.6 | 89.1 | 95.3 | 94.5 | 93.0 | 0.33 | 0.43 | 0.62 |

Option 1, i.e., different pattern in each time instance of T1, achieves similar performance to Option 2a (same pattern in each time instance of T1 to traverse all patterns in Option 1), while requiring half the measurement resource overhead.

Option 1, i.e., different pattern in each time instance of T1, performs better than Option 2b (same pattern in each time instance of T1, where number of beams measured in each time instance is the same as that of Option 1), and uses the same measurement resource overhead.

### Performance with different Rx assumption

      - DL Tx beam prediction

In the evaluation of subsection 4.3.2.1, the Rx beam used for model input in T1 is a Rx beam of a beam pair whose L1-RSRP with a 1 dB difference from L1-RSRP of the optimal beam pair. The Rx beam used in T2 is based on another Rx beam sweeping after DL Tx prediction. In this subsection, we will further analyze different options of Rx assumptions for both T1 and T2.

In FL’s summary of RAN1#112bis meeting, 3 options are agreed for Rx beam assumption for providing input to AI/ML model for training and/or inference.

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

  - Companies report how to select the “best” Rx beam(s)

- Option 2: Measurements of specific Rx beam(s)

  - Companies report how to select specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

Further, in FL’s summary of RAN1#112 meeting, 5 cases are considered for Rx beam assumption for providing input to AI/ML model for training and/or inference, which reveal more details to obtain the “best” Rx beam or “specific” Rx beam.

- Case 0: the best Rx beam for the best Tx beam within Set B

- Case 1: the best Rx beam searched for one Tx beam within Set B

- Case 2: the best Rx beam for each Tx beam within Set B

- Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

- Case 4: the best Rx beam among specific Rx beams for the best Tx beam within Set B

- Case 5: the best Rx beam for the best Tx beam with in Set A

For BM-Case2, set B is not measured in one measurement occasion as in BM-Case1, but is measured over multiple measurement occasions of T1. Therefore, there are two ways for case 0~5 to be extended to BM-case2. The first way is to do the measurements for Rx determination before the first measurement occasion of T1, determine the Rx beam(s) and use the Rx beam(s) for T1. The other way is to decide Rx beam(s) before each measurement occasion of T1, and each occasion use its own Rx beam(s). For example, the number of Tx beams in set B is 16, T1 contains 4 measurement occasions, and 4 Tx beams are measured in each occasion.

- For case 0/1/3/4/5, one Rx beam is used to measure Tx beams in set B for T1. In this case, only the first way is valid as the second way would require to use multiple Rx beams for set B. Therefore, one Tx beam is selected from set B, the best Rx beam is selected before 1st measurement occasion, and this Rx beam is also used for Tx beam measurements at 2nd ~4th occasions.

- For case 2, either the 1st way or the 2nd way can be used, though the 2nd way may have better measurement performance as the latest best Rx beam is used for multiple Tx beams in set B.

- Study the Rx beam assumption cases in T1 for BM-case 2 based on the following principles,

- For Case 0/1/3/4/5, where one Rx beam is used for set B, the measurement for Rx beam determination is performed before T1

- For Case 2, where each Tx beam in set B uses its own best Rx beam, the measurement of Rx beam determination can be performed before T1 starts, or before each measurement occasion in T1

In this subsection, we consider two ways for Option 1. One way (Case 5) is to use the Rx beam of best beam pair according to exhaustive beam sweeping in set A for both T1 and T2, the other way (Case 2’) is to use best Rx beam for each Tx beam within Set B in T1 and best Rx beam for each Tx beam within Set A in T2.

In FL’s summary of RAN1#112bis meeting, quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the “best” Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping is agreed to study. Option 2 (Measurements of specific Rx beam(s)) can use a quasi-optimal Rx beam. Here, we use the best Rx beam searched for one Tx beam within Set B (Case 1) to evaluate option 2.

Assume that Rx beam(s) in T2 is the same as that of T1, Rx Option 1(Case 2’ and Case 5), Option 2 and Option 3 are compared in Table 4.2.2.1-1. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8, where P is the number of time instances in prediction window T2. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. For Rx Option 1(Case5), Rx beam in T2 for KPI calculation is the same as that of T1, i.e., the Rx beam of best beam pair with exhaustive beam sweeping in set A. For Rx Option 1 (Case2), Rx beams in T2 are the best Rx beam for each Tx beam within Set A. For Rx Option 2, Rx beam in T2 is the same as that of T1, i.e., the best Rx beam for one Tx beam within Set B measured before T1. For Rx Option 3, Rx beam in T2 is the same as that of T1, i.e., a random Rx beam.

Table 4.2.2.1-1: Different Rx beam assumption options for T1(fixed Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Option 1 (Case5): the Rx beam of best beam pair within Set A | 94.0 | 92.6 | 90.7 | 96.5 | 95.2 | 93.6 | 0.25 | 0.40 | 0.60 |
| Option 1(Case2’): the best Rx beam for each Tx beam within Set A | 92.7 | 91.4 | 89.5 | 96.9 | 95.8 | 94.4 | 0.26 | 0.41 | 0.61 |
| Option 2(Case1): the best Rx beam searched for one Tx beam within Set B | 79.1 | 78.3 | 77.2 | 80.6 | 79.9 | 79.0 | 1.24 | 1.39 | 1.59 |
| Option 3: random Rx beam | 12.2 | 12.1 | 12.0 | 12.5 | 12.5 | 12.4 | 15.01 | 15.13 | 15.29 |

Assume that Rx beam(s) in T2 is the same as that of T1, Rx Option 1(Case 2’ and Case 5), Option 2 and Option 3 are compared in Table 4.2.2.1-2. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7, 9,11,13,15], [17,19,21,23, 25,27,29,31]. UE speed is 30km/h. For Rx Option 1(Case5), Rx beam in T2 is the same as that of T1, i.e., the Rx beam of best beam pair with exhaustive beam sweeping in set A. For Rx Option 1 (Case2), Rx beams in T2 are the best Rx beam for each Tx beam within Set A. For Option 2, Rx beam in T2 is the same as that of T1, i.e., the best Rx beam for one Tx beam within Set B measured before T1. For Rx Option 3, Rx beam in T2 is the same as that of T1, i.e., a random Rx beam.

Table 4.2.2.1-2: Different Rx beam assumption options for T1(fixed Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Option 1 (Case5): the Rx beam of best beam pair within Set A | 93.4 | 92.4 | 90.5 | 96.4 | 95.6 | 94.0 | 0.30 | 0.40 | 0.60 |
| Option 1(Case2’): the best Rx beam for each Tx beam within Set A | 92.4 | 91.4 | 89.3 | 96.9 | 96.2 | 94.8 | 0.31 | 0.41 | 0.62 |
| Option 2(Case1): the best Rx beam searched for one Tx beam within Set B | 78.7 | 78.2 | 77.0 | 80.5 | 80.0 | 79.1 | 1.31 | 1.41 | 1.61 |
| Option 3: random Rx beam | 12.0 | 11.9 | 11.8 | 12.4 | 12.4 | 12.3 | 15.07 | 15.15 | 15.31 |

Assume that Rx beam(s) in T2 is the same as that of T1, Rx Option 1(Case 0 and Case 2) and Option 2(Case 1) are compared in Table 4.2.2.1-3. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 random Tx beams are measured. UE speed is 30km/h. For Option 1(Case 0), Rx beam is the best Rx beam for the best Tx beam within Set B measured before T1. For Option 2(Case 1), Rx beam is the best Rx beam for one Tx beam within Set B measured before T1. For Option 1(Case 2), for Tx beams in set B, the Rx beams is the same as that of T1, i.e., the best Rx beam for each Tx beam, and for the Tx beams in set A but not in set B, the best Rx beam is decided based on the measurements in T1.

Table 4.2.2.1-3: Different Rx beam assumptions for T1(random Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2 =40ms |  |  | T2 =40ms |  |  | T2 =40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Case 0: the best Rx beam for the best Tx beam within Set B | 87.7 | 86.3 | 84.1 | 92.6 | 91.6 | 90.3 | 0.45 | 0.61 | 0.84 |
| Case 1: the best Rx beam searched for one Tx beam within Set B | 75.3 | 74.2 | 72.5 | 79.3 | 78.6 | 77.7 | 1.68 | 1.84 | 2.07 |
| Case 2: the best Rx beam for each Tx beam within Set B | 86.5 | 84.9 | 82.7 | 92.6 | 91.7 | 90.4 | 0.55 | 0.72 | 0.96 |

Assume that after model inference, Rx beams are swept, and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. Option 1(Case 2’ and Case5), Option 2 and Option 3 are compared in Table 4.2.2.1-4. For Option 1(Case2’), Rx beams are swept and the best Rx beams for each predicted top-K beams are used for corresponding Tx beams in T2.

Table 4.2.2.1-4: Different Rx beam assumption options for T1(fixed Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Option 1(Case5): the Rx beam of best beam pair within Set A | 96.1 | 95.2 | 94.1 | 99.2 | 98.8 | 98.2 | 0.11 | 0.22 | 0.36 |
| Option 1(Case2’): the best Rx beam for each Tx beam within Set A | 94.6 | 93.9 | 92.7 | 99.2 | 98.8 | 98.3 | 0.15 | 0.25 | 0.39 |
| Option 2(Case1): the best Rx beam searched for one Tx beam within Set B | 89.9 | 89.4 | 88.5 | 96.8 | 96.4 | 95.9 | 0.51 | 0.61 | 0.75 |
| Option 3: random Rx beam | 81.8 | 81.4 | 80.6 | 93.4 | 93.0 | 92.5 | 1.65 | 1.75 | 1.89 |

Assume that after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7,9,11,13,15], [17,19,21,23,25,27,29,31]. UE speed is 30km/h. Option 1(Case 2’ and Case5), Option 2 and Option 3 are compared in Table 4.2.2.1-5. For Option 1(Case2’), Rx beams are swept and the best Rx beams for each predicted top-K beams are used for corresponding Tx beams in T2.

Table 4.2.2.1-5: Different Rx beam assumption options for T1(fixed Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Option 1(Case5): the Rx beam of best beam pair within Set A | 95.4 | 94.8 | 93.5 | 99.0 | 98.7 | 98.1 | 0.18 | 0.24 | 0.38 |
| Option 1(Case2’): the best Rx beam for each Tx beam within Set A | 94.3 | 93.6 | 92.2 | 99.1 | 98.8 | 98.3 | 0.19 | 0.27 | 0.41 |
| Option 2(Case1): the best Rx beam searched for one Tx beam within Set B | 89.5 | 89.1 | 88.2 | 96.6 | 96.3 | 95.8 | 0.55 | 0.62 | 0.76 |
| Option 3: random Rx beam | 81.4 | 81.1 | 80.2 | 93.0 | 92.8 | 92.3 | 1.74 | 1.80 | 1.93 |

Assume that after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 random Tx beams are measured. UE speed is 30km/h. Option 1(Case 0 and Case2) and Option 2(Case1) are compared in Table 4.2.2.1-6. For Option 1(Case2), Rx beams are swept and the best Rx beams for each predicted top-K beam are used for corresponding Tx beams in T2.

Table 4.2.2.1-6: Different Rx beam assumptions for T1(random Set B)

| Rx assumption in T1 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2 =40ms |  |  | T2 =40ms |  |  | T2 =40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Case 0: the best Rx beam for the best Tx beam within Set B | 90.7 | 89.8 | 88.2 | 97.7 | 97.3 | 96.7 | 0.29 | 0.40 | 0.57 |
| Case 1: the best Rx beam searched for one Tx beam within Set B | 86.0 | 85.1 | 83.7 | 95.3 | 94.9 | 94.3 | 0.79 | 0.90 | 1.07 |
| Case 2: the best Rx beam for each Tx beam within Set B | 90.8 | 89.8 | 88.1 | 98.6 | 98.2 | 97.7 | 0.27 | 0.39 | 0.57 |

For Rx assumption in T1, Option 1(Case 2, the best Rx beam for each Tx beam within Set B) offers a similar improvement in the terms of prediction accuracy and average RSRP difference results when compared to that of Option 1(Case 0, the best Rx beam for the best Tx beam within Set B), but a degradation in this aspect can be observed in Option 2 (Case1, the best Rx beam searched for one Tx beam within Set B).

For Rx assumption in T1, Option 1(Case5), i.e., the Rx beam of best beam pair within Set A, offers a similar improvement in the terms of prediction accuracy and average RSRP difference results when compared to that of Option 1 (Case2’, the best Rx beam for each Tx beam within Set A), but a degradation in this aspect can be observed in Option 2 (Case1, the best Rx beam searched for one Tx beam within Set B) and Option 3 (random Rx beam).

### Performance with different scalable T1/T2 assumptions

      - DL Tx beam prediction

In the subsection, different observation/prediction windows or periodicity for T1/T2 time instances are studied.

Assume that T1 = 40ms, M = 4, and T2= 40ms, different P (which is the number of time instances for prediction) values are compared in Table 4.2.3.1-1. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. Rx beam in T1 is the Rx beam of best beam pair within Set A, and Rx beam in T2 is same as that of T1.

Table 4.2.3.1-1: performance comparison of different P

| T1 | M | Measured beams per each time instance | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] based on all beam pairs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  |  |  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| 40ms | 4 | 4 | 94.0 | 92.6 | 90.7 | 96.5 | 95.2 | 93.6 | 0.25 | 0.40 | 0.60 |

Assume that T1 = 80ms, M = 2, and T2= 80ms, different P values are compared in Table 4.2.3.1-2. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7, 9,11,13,15], [17,19,21,23, 25,27,29,31]. UE speed is 30km/h. Rx beam in T1 is the Rx beam of best beam pair within Set A. and Rx beam in T2 is same as that of T1.

Table 4.2.3.1-2: performance comparison of different P

| T1 | M | Measured beams per each time instance | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  |  | BM loss for top1 [dB] based on all beam pairs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T2=80ms |  |  | T2=80ms |  |  |  | T2=80ms |  |  |
|  |  |  | P=1 | P=2 | P=4 | P=1 |  | P=2 | P=4 | P=1 | P=2 | P=4 |
| 80ms | 2 | 8 | 93.4 | 92.4 | 90.5 |  | 96.4 | 95.6 | 94.0 | 0.30 | 0.40 | 0.60 |

When number of time instance P increases, performance decreases in terms of prediction accuracy and average RSRP.

Assume that T2*P= 160/320ms, different T1 and M are compared in Table 4.2.3.1-3. Set A has 32 Tx beams, and set B is a subset of set A. UE speed is 30km/h. Rx beam in T1 is the Rx beam of best beam pair within Set A, and Rx beam in T2 is same as that of T1.

Table 4.2.3.1-3: performance comparison of different T1 and M

| T1 | M | Measured beams per each time instance | Accuracy of Top1 [%] (based on all beam pairs) |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  | BM loss for top1 [dB] based on all beam pairs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T2*P=160ms | T2*P=320ms | T2*P=160ms | T2*P=320ms | T2*P=160ms | T2*P=320ms |
| 40ms | 4 | 4 | 92.6 | 90.7 | 95.2 | 93.6 | 0.40 | 0.60 |
| 80ms | 2 | 8 | 92.4 | 90.5 | 95.6 | 94.0 | 0.40 | 0.60 |

When Set B is the same and T1*M is the same, different T1 and M shows similar prediction accuracy and average RSRP.

      - Beam pair prediction

Figure 4.3.3.2-1: beam pair prediction in temporal domain (P1)

For beam pair prediction scheme, 8 beam pairs from 256 beam pairs are selected with random beam set-B selection scheme and measured at each time instant within a time duration T1, and different beam pairs are selected for measurement among time instants within T1. As a consequence, total of 64 difference beams are used in AI input to predict L1-RSRP of 256 beam pairs at each time instant within T2. In Figure 4.3.3.2-1, input of AI model includes measured L1-RSRP of beam pair, corresponding Tx beam ID and Rx beam ID, and output of AI model is L1-RSRP of all beam pairs in future time instants.

For non-AI scheme, beam pair measurement in time duration T1 is the same as beam pair prediction scheme, and the best beam pair is decided based on measurement of T1 and regarded as best beam pair for time instants within T2.

BM evaluation metrics are calculated based on difference between decided/predicted best beam pair and real best beam pair in T2. Time duration T1 is fixed to 8*40ms, and time duration T2 is equal to 1*40ms, 4*40ms, or 8*40ms respectively.

Table 4.2.3.2-1: performance comparison between non-AI and beam pair prediction in temporal domain (T2=1*40ms)

| Scheme | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- |
| Non-AI | 6.62 | 21.42 | 27.82 | 21.92 |
| Beam pair prediction | 0.67 | 78.11 | 87.61 | 94.51 |

Table 4.2.3.2-2: performance comparison between non-AI and beam pair prediction in temporal domain (T2=4*40ms)

| Scheme | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- |
| Non-AI | 6.79 | 21.24 | 27.58 | 21.84 |
| Beam pair prediction | 0.85 | 77.04 | 86.58 | 94.00 |

Table 4.2.3.2-3: performance comparison between non-AI and beam pair prediction in temporal domain (T2=8*40ms)

| Scheme | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- |
| Non-AI | 7.04 | 20.93 | 27.20 | 21.72 |
| Beam pair prediction | 1.11 | 75.47 | 84.92 | 93.27 |

For BM-Case2, compared with non-AI scheme, beam pair prediction scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly.

- Further study beam pair prediction scheme with expected information as AI input for improving generalization performance in BM-Case2.

### Rx beam selection for the idea L1-RSRP of Top-1 predication beam

      - DL Tx beam prediction

In the evaluation of subsection 4.3.2.1, the Rx beam used for model input in T1 is a Rx beam of a beam pair whose L1-RSRP with a 1 dB difference from L1-RSRP of the optimal beam pair. The Rx beam used in T2 is based on another Rx beam sweeping after DL Tx prediction. In this subsection, we will further analyze different options of Rx assumptions for both T1 and T2.

In FL’s summary of RAN1#112bis meeting, 3 options are agreed for Rx beam assumption for providing input to AI/ML model for training and/or inference.

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

  - Companies report how to select the “best” Rx beam(s)

- Option 2: Measurements of specific Rx beam(s)

  - Companies report how to select specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

In FL’s summary of RAN1#112 meeting, 5 cases are considered for Rx beam assumption for providing input to AI/ML model for training and/or inference.

- Case 0: the best Rx beam for the best Tx beam within Set B

- Case 1: the best Rx beam searched for one Tx beam within Set B

- Case 2: the best Rx beam for each Tx beam within Set B

- Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

- Case 4: the best Rx beam among specific Rx beams for the best Tx beam within Set B

- Case 5: the best Rx beam for the best Tx beam with in Set A

For BM-case2, the above Rx assumptions are considered for model input, that is the Rx assumption used in T1. However, how to determine Rx beam in T2 is another issue. Rx beam used in T2 impacts KPI calculation and final performance. Hence it is better to discuss this in RAN1. This issue is similar for BM Case 1 and BM Case 2, but more serious for BM Case 2. Due to high UE moving speed in BM Case 2, when to measure and acquire the best Rx beams is especially critical. Further, if set B is a subset of Set A, some of the predicted beams are not measured in Set B. Figure 4.2.4.1-1 shows the following three potential options for Rx assumptions for predicted beams and KPI calculation in T2.

- Alt A: T2 uses the same Rx beam(s) as that of T1

- Alt B: T2 uses the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement before T1.

  - Alt B-1: Extra measurement includes Rx beam sweeping for Tx beams in Set B, where the Tx beams are not used for Rx beam determination in T1, and the extra measurement can be before T1 (This option can be used for both training and inference).

  - Alt B-2: Extra measurement includes Rx sweeping for Tx beams in Set A, where the Tx beams are not used for Rx beam determination in T1, and the extra measurement can be before T1 (This option can be used for both training and inference)

- Alt C: T2 uses the best Rx beam(s) based on the predicted top-K Tx beams determined between T1 and T2 (This option can only be used for inference)

Alt C: T2 uses the best Rx beam(s) based on the predicted top-K Tx beams determined between T1 and T2 (This option can only be used for inference)

Alt A: T2 uses the same Rx beam(s) as that of T1

Alt B: T2 uses the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement before T1

Figure 4.2.4.1-1: Options for Rx assumptions for predicted beams and KPI calculation in T2

Any of the above options can be applied on multiple T1 Rx Cases. The general principle here should be the Rx beam used for T2 isn’t worse than the Rx beam for T1.  For example, if Alt A is applied on Rx Option 1(Case 0), the Rx beam used for T2 is the same as that of T1, i.e., the best Rx beam for the best Tx beam within Set B measured before T1. We can further apply Alt A to Rx Option 2(Case 1), where, the Rx beam used for T2 is the best Rx beam searched for one Tx beam within Set B, same as the Rx beam used in T1. If Alt A is applied on Rx Option 1(Case 2) and set B is a subset of set A, for Tx beams in set B, the Rx beams is the same as that of T1, and for the Tx beams in set A but not in set B, the best Rx beam is decided based on the measurements in T1, e.g., using the best Rx beam measured from T1.

For Alt B-1, it can be applied on Rx Option 2(Case 1), where Rx sweeping is done only for one Tx beam in Set B during Rx beam determination for model input, and extra measurements, i.e. Rx sweeping for the rest Tx beams in Set B is together used to decide the Rx beam used for T2. As a result, the Rx beam is the best Rx beam for the best Tx beams within Set B measured before T1

For Alt B-2, the Rx beam used for T2 is the best Rx beam(s) for the best Tx beam(s) within Set A measured before T1. For Rx Option 1(Case 0) and Option 2(Case 1), one best Rx beam is used for T2, and for Rx Option 1(Case 2), each Tx beam within Set A has its own corresponding best Rx beam, that is Rx Option 1(Case 2’).

For Alt C, after model inference, Rx beams are swept and the best Rx beam for each or all predicted top-K beams are found. These Rx beam(s) are used for all the Tx beams or for corresponding Tx beams in T2.

For Rx Option 1(Case 0), T2 Alt A, Alt B-2 and Alt C are compared in Table 4.2.4.1-1. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 random Tx beams are measured. UE speed is 30km/h. For T2 Alt A, Rx beam in T2 is the same as that of T1, i.e., the best Rx beam for the best Tx beam within Set B measured before T1. Alt B-2 is to determine Rx beam in T2 based on measurement for Rx beams for T1 and extra measurement, where the extra measurement includes Rx sweeping for Tx beam(s) in Set A but not used in T1. This can also be regarded as measurement of best Rx for Tx beams in Set A.

Table 4.2.4.1-1: Rx Option 1(Case 0) among different Rx beam assumptions for T2 (random Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] based on all beam pairs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2 =40ms |  |  | T2 =40ms |  |  | T2 =40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 87.7 | 86.3 | 84.1 | 92.6 | 91.6 | 90.3 | 0.45 | 0.61 | 0.84 |
| Alt B-2: the best Rx beam for the best Tx beams within Set A measured before T1 | 88.5 | 87.1 | 84.8 | 95.0 | 93.8 | 92.2 | 0.55 | 0.71 | 0.94 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 90.7 | 89.8 | 88.2 | 97.7 | 97.3 | 96.7 | 0.29 | 0.40 | 0.57 |

For Rx Option 2 (Case 1), T2 Alt A, Alt B-1, Alt B-2 and Alt C are compared in Table 4.2.4.1-2. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 random Tx beams are measured. UE speed is 30km/h. Alt A is the same as that of T1, i.e., the best Rx beam for one Tx beam within Set B measured before T1. Alt B-1 is actually to decide Rx beam in T2 based on measurements of all Rx beams of all Tx beams in Set B, and Alt B-2 can be considered as deciding Rx beam in T2 based on measurements of all Rx beams of all Tx beams in Set A.

Table 4.2.4.1-2: Rx Option 2(Case 1) among different Rx beam assumptions for T2 (random Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] based on all beam pairs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2 =40ms |  |  | T2 =40ms |  |  | T2 =40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 75.3 | 74.2 | 72.5 | 79.3 | 78.6 | 77.7 | 1.68 | 1.84 | 2.07 |
| Alt B-1: the best Rx beam for the best Tx beams within Set B measured before T1 | 81.9 | 80.5 | 78.5 | 89.7 | 88.7 | 87.4 | 1.31 | 1.47 | 1.70 |
| Alt B-2: the best Rx beam for the best Tx beams within Set A measured before T1 | 84.0 | 82.5 | 80.3 | 92.7 | 91.5 | 89.9 | 1.27 | 1.42 | 1.65 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 86.0 | 85.1 | 83.7 | 95.3 | 94.9 | 94.3 | 0.79 | 0.90 | 1.07 |

For Rx Option 1(Case 2), Alt A, Alt B-2 and Alt C are compared in Table 4.2.4.1-3. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 random Tx beams are measured. UE speed is 30km/h. For Alt A, for Tx beams in set B, the Rx beams is the same as that of T1, and for the Tx beams in set A but not in set B, the best Rx beam is decided based on the measurements in T1. For Alt B-2, it can be considered as deciding Rx beam in T2 based on measurements of all Rx beams of all Tx beams in Set A.

Table 4.2.4.1-3: Rx Option1 (Case 2) among different Rx beam assumptions for T2 (random Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] |  |  | Accuracy of Top4/1 [%] |  |  | BM loss for top1 [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (based on all beam pairs) |  |  | (based on all beam pairs) |  |  | based on all beam pairs |  |  |
|  | T2 (*40ms) |  |  | T2 (*40ms) |  |  | T2 (*40ms) |  |  |
|  | 1 | 4 | 8 | 1 | 4 | 8 | 1 | 4 | 8 |
| Alt A: use the same Rx as T1 | 86.5 | 84.9 | 82.7 | 92.6 | 91.7 | 90.4 | 0.55 | 0.72 | 0.96 |
| Alt B-2: the best Rx beam for the best Tx beams within Set A measured before T1 | 89.0 | 87.4 | 85.0 | 96.4 | 95.3 | 93.9 | 0.40 | 0.57 | 0.89 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 90.8 | 89.8 | 88.1 | 98.6 | 98.2 | 97.7 | 0.27 | 0.39 | 0.57 |

For Rx Option 1(Case 5), T2 Alt A and Alt C are compared in table 4.2.4.1-4. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the Rx beam of best beam pair with exhaustive beam sweeping in set A measured before T1. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-4: Rx Option 1(Case 5) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 94.0 | 92.6 | 90.7 | 96.5 | 95.2 | 93.6 | 0.25 | 0.40 | 0.60 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 96.1 | 95.2 | 94.1 | 99.2 | 98.8 | 98.2 | 0.11 | 0.22 | 0.36 |

For Rx Option 1(Case 5), T2 Alt A and Alt C are compared in table 4.2.4.1-5. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7,9,11,13,15], [17,19,21,23,25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the Rx beam of best beam pair with exhaustive beam sweeping in set A measured before T1. For Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-5: Rx Option 1(Case 5) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Alt A: use the same Rx as T1 | 93.4 | 92.4 | 90.5 | 96.4 | 95.6 | 94.0 | 0.30 | 0.40 | 0.60 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 95.4 | 94.8 | 93.5 | 99.0 | 98.7 | 98.1 | 0.18 | 0.24 | 0.38 |

For Rx Option 1(Case 2’), T2 Alt A and Alt C are compared in table 4.2.4.1-6. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the best Rx beam for each Tx beam within Set A measured before T1. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-6: Rx Option 1(Case 2’) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 92.7 | 91.4 | 89.5 | 96.9 | 95.8 | 94.4 | 0.26 | 0.41 | 0.61 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 94.6 | 93.9 | 92.7 | 99.2 | 98.8 | 98.3 | 0.15 | 0.25 | 0.39 |

For Rx Option 1(Case 2’), T2 Alt A and Alt C are compared in table 4.2.4.1-7. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7,9,11,13,15], [17,19,21,23,25,27,29,31].  UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the best Rx beam for each Tx beam within Set A measured before T1. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-7: Rx Option 1(Case 2’) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Alt A: use the same Rx as T1 | 92.4 | 91.4 | 89.3 | 96.9 | 96.2 | 94.8 | 0.31 | 0.41 | 0.62 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 94.3 | 93.6 | 92.2 | 99.1 | 98.8 | 98.3 | 0.19 | 0.27 | 0.41 |

For Rx Option 2(Case 1), T2 Alt A and Alt C are compared in table 4.2.4.1-8. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the best Rx beam for the best Tx beam within Set B measured before T1. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-8: Rx Option 2(Case 1) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 79.1 | 78.3 | 77.2 | 80.6 | 79.9 | 79.0 | 1.24 | 1.39 | 1.59 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 89.9 | 89.4 | 88.5 | 96.8 | 96.4 | 95.9 | 0.51 | 0.61 | 0.75 |

For Rx Option 2(Case 1), T2 Alt A and Alt C are compared in table 4.2.4.1-9. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7,9,11,13,15], [17,19,21,23,25,27,29,31].  UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., the best Rx beam for the best Tx beam within Set B measured before T1. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-9: Rx Option 2(Case 1) among different Rx beam assumptions for T2 (fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Alt A: use the same Rx as T1 | 78.7 | 78.2 | 77.0 | 80.5 | 80.0 | 79.1 | 1.31 | 1.41 | 1.61 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 89.5 | 89.1 | 88.2 | 96.6 | 96.3 | 95.8 | 0.55 | 0.62 | 0.76 |

For Rx Option 3, T2 Alt A and Alt C are compared in table 4.2.4.1-10. Assume that T1 = 40ms, M = 4, T2= 40ms and P = 1/4/8. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 4 beams are measured. The measured beam pattern is [1,3,5,7], [9,11,13,15], [17,19,21,23], [25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., a random Rx beam. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-10: Rx Option 3 among different Rx beam assumptions for T2(fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=40ms |  |  | T2=40ms |  |  | T2=40ms |  |  |
|  | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 | P=1 | P=4 | P=8 |
| Alt A: use the same Rx as T1 | 12.2 | 12.1 | 12.0 | 12.5 | 12.5 | 12.4 | 15.01 | 15.13 | 15.29 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 81.8 | 81.4 | 80.6 | 93.4 | 93.0 | 92.5 | 1.65 | 1.75 | 1.89 |

For Rx Option 3, T2 Alt A and Alt C are compared in table 4.2.4.1-11. Assume that T1 = 80ms, M = 2, T2= 80ms and P = 1/2/4. Set A has 32 Tx beams, and set B is a subset of set A. For each time instance, 8 beams are measured. The measured beam pattern is [1,3,5,7,9,11,13,15], [17,19,21,23,25,27,29,31]. UE speed is 30km/h. T2 Alt A is to use the same Rx beam as that of T1, i.e., a random Rx beam. For T2 Alt C, after model inference, Rx beams are swept and the best Rx beam for all predicted top-K beams are used for all the Tx beams in T2.

Table 4.2.4.1-11: Rx Option 3 among different Rx beam assumptions for T2(fixed Set B)

| Rx assumption in T2 | Accuracy of Top1 [%] (based on all beam pairs) |  |  | Accuracy of Top4/1 [%] (based on all beam pairs) |  |  | BM loss for top1 [dB] (based on all beam pairs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2=80ms |  |  | T2=80ms |  |  | T2=80ms |  |  |
|  | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 | P=1 | P=2 | P=4 |
| Alt A: use the same Rx as T1 | 12.0 | 11.9 | 11.8 | 12.4 | 12.4 | 12.3 | 15.07 | 15.15 | 15.31 |
| Alt C: best Rx beam based on the predicted top-K Tx beams | 81.4 | 81.1 | 80.2 | 93.0 | 92.8 | 92.3 | 1.74 | 1.80 | 1.93 |

For Rx assumption in T2, Alt C, i.e., best Rx beam based on the predicted top-K Tx beams achieves best performance in terms of prediction accuracy and average RSRP difference results, Alt B (the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement) gets the second-best performance and Alt A (same Rx(s) as that of T1) performs worst.

For Rx Option 2(Case 1), performance gain introduced from extra measures before T1 (Alt B) or Rx beam sweeping after DL Tx beam prediction (Alt C) is much bigger than that of Rx Option 1(Case 0/Case2/Case 2’/Case5).

For Rx Option 3, performance gain introduced from Rx beam sweeping after DL Tx beam prediction (Alt C) is much bigger than that of Rx Option 2(Case 1) and Rx Option 1(Case 0/Case2/Case 2’/Case5).

For Case 1, there are two options to improve the performance, one is to use extra measurements before T1, e.g., measure all Rx beams of Tx beams in set B or set A. But it leads to a large number of measurements, especially for all beam pair measurements of set A. Another option is to sweep Rx beams of predicted Top-K Tx beams, which introduce fewer measurements compared to option 1, and achieves better performance.

- Study the issue of Rx assumption options for predicted beams and KPI calculation in T2 in BM Case 2. The following options can be considered as a start point

- Alt A: T2 uses the same Rx beam(s) as that of T1

- Alt B: T2 uses the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement before T1

- Alt C: T2 uses the best Rx beam(s) based on the predicted top-K Tx beams determined between T1 and T2

AI/ML model generalization

Generalization for BM-Case1

### Different pattern of Set B for beam pair prediction

From AI model applicability aspect, the AI model trained with one fixed pattern can be applied the dataset generated by other fixed patterns for generalization performance study. Performance loss is expected. The following results show more details.

Table 5.1.1-1: performance evaluation results for one fixed pattern in Set B with 16 beams

| Training dataset | Validation dataset | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1 [%] |
| --- | --- | --- | --- | --- | --- |
| Fixed pattern-2 | Fixed pattern-1 | 17.6 | 3.20 | 4.89 | 9.49 |
| Fixed pattern-3 | Fixed pattern-2 | 21.0 | 3.72 | 13.2 | 12.0 |
| Fixed pattern-4 | Fixed pattern-2 | 20.6 | 2.74 | 12.0 | 9.29 |
| Fixed pattern-4 | Fixed pattern-3 | 14.4 | 9.94 | 19.7 | 27.5 |

The performance with different training and validation fixed patterns is quite poor and not acceptable, i.e., fixed set B selection scheme suffers serious generalization issue.

### Different gNB antenna setting for beam pair prediction

In this section, we will focus on the influence of another generalization aspect, i.e. different gNB/UE antenna configurations, which bring various beam shape patterns. To simplify this issue, only antenna configurations at gNB with corresponding Tx/Rx pointing angles changed among measurement patterns are evaluated for the following generalization performance study. The number of UE side Rx beams is kept as 8. A universal AI model using random beam pair pattern Set-1 is generated from Table 5.1.2-1, where each input sample includes 16 L1-RSRPs and its corresponding Tx/Rx beam pointing angles, and relative AI output is the RSRPs of total 256 beam pairs. However, random beam pair pattern Set-2 and random beam pair pattern Set-3, which are generated by table 5.1.2-2 and table 5.1.2-3 respectively, have total 128 beam pairs and 64 beam pairs. As a consequence, except random beam pair pattern Set-1, available predicted beams should be selected after model inference for other sets, and the KPI evaluation should also be calculated based on available predicted beams and its total beam pairs.

Table 5.1.2-1: beam shape pattern 1

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 4 8 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 32 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 67.5 112.5 157.5] |

Table 5.1.2-2: beam shape pattern 2

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 2 8 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 16 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 112.5] |

Table 5.1.2-3: beam shape pattern 3

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 2 4 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 8 Tx beams Horizontal angle = [-56.25 -11.25 11.25 56.25] Vertical angle = [22.5 112.5] |

- Random beam pair Set-1 with Tx beam shape 1: Random beam pair selection scheme with 32 Tx beams and 8 Rx beams for each model input sample from Table 19 + Tx/Rx beam pointing angle

- Random beam pair Set-2 with Tx beam shape 2: Random beam pair selection scheme with 16 Tx beams and 8 Rx beams for each model input sample from Table 20 + Tx/Rx beam pointing angle

- Random beam pair Set-3 with Tx beam shape 3: Random beam pair selection scheme with 8 Tx beams and 8 Rx beams for each model input sample from Table 21+ Tx/Rx beam pointing angle

To get a fair comparison result in generalization aspect, same dataset used for training and validation can be considered as an upper bound performance, while an AI model trained by random beam pair pattern Set-1 can be used for inference with random beam pair pattern Set-2 and random beam pair pattern Set-3.

Table 5.1.2-4: performance evaluation results for different beam shape patterns

| Training dataset | Validation dataset | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1 [%] |
| --- | --- | --- | --- | --- | --- |
| Random beam pair Set-2 with Tx beam shape 2 |  | 4.08 | 36.5 | 43.6 | 71.5 |
| Random beam pair Set-1 with Tx beam shape 1 | Random beam pair Set-2 with Tx beam shape 2 | 5.10 (+25%) | 31.92 (-12%) | 39.9 (-8%) | 67.0 (-6%) |
| Random beam pair Set-3 with Tx beam shape 3 |  | 1.89 | 51.1 | 62.9 | 85.7 |
| Random beam pair Set-1 with Tx beam shape 1 | Random beam pair Set-3 with Tx beam shape 3 | 2.95 (+56%) | 41.0 (-20%) | 52.5 (-16%) | 78.4 (-9%) |

Compared to upper bound performance of random beam pair Set-2, approximately 1 dB performance deterioration of average RSRP difference and 5 points of beam prediction accuracy loss can be observed by pre-defined AI model trained by random beam pair Set-1 and inferenced by random beam pair Set-2. With distinction of beam shape pattern between training subset and validation subset increasing, 10 points of beam prediction accuracy loss is obtained for validation random beam pair Set-3 as well as 1.1 dB average RSRP difference loss.

As the difference of beam shape pattern increases, the performance loss of both average RSRP difference and beam prediction accuracy increases along with the difference of the antenna configurations between training subset and validation subset.

Generalization for BM-Case2

### Different gNB antenna setting

      - Beam pair prediction

In this section, we will focus on the influence of a generalization aspect, i.e. different gNB/UE antenna configurations, in temporal domain beam prediction, which brings various beam shape patterns. In the above simulations, including both spatial domain beam prediction and temporal domain beam prediction, beam pointing angle is used as AI input for performance improvement with random or semi-random beam subset. As a same mapping from beam angle to beam ID, which can be called as global beam ID or beam pointing angle, is used for evaluation, it implies that there has another option which uses local beam ID. For local beam ID, different mapping from beam angle to beam ID exists between datasets generated from different antenna configurations. Thus, for generalization study, we evaluate the generalization performance applying a trained AI model learned from a certain set of beams pointing angles/global beam IDs or local beam IDs based on a certain number of antennas for unlearned beam shape.

Figure 5.2.1.1-1: different mapping methods: local beam ID vs beam pointing angle

For the case using local beam ID as model input, the training dataset 32x8 has 32 Tx local beam ID, e.g. Tx beam ID = 0 ~ 31, and for validation dataset 16x8 and 8x8, the range of Tx local beam ID is 0~15 and 0~7 respectively. Further, the beam ID used in model input of validation dataset is 0~15 and 0~7 respectively. For the case using beam angle as model input, the training dataset 32x8 has 32 Tx beams with different pointing angles, denoted as beam 0~31, and for validation dataset 16x8 and 8x8, the beam ID used in model input of validation dataset is mapped according to beam pointing angle. The difference between mapping based on local beam ID and beam angle (global beam ID) is displayed in Figure 5.2.1.1-1.

Table 5.2.1.1-1: beam shape pattern 1

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 4 8 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 32 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 67.5 112.5 157.5] |

Table 5.2.1.1-2: beam shape pattern 2

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 2 8 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 16 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 112.5] |

Table 5.2.1.1-3: beam shape pattern 3

| Antenna configuration at BS | [Mg Ng M N P] = [1 1 2 4 2], [dV, dH] = [0.5,0.5] λ |
| --- | --- |
| BS TX beam pattern | 8 Tx beams Horizontal angle = [-56.25 -11.25 11.25 56.25] Vertical angle = [22.5 112.5] |

To simplify this issue, only antenna configuration at gNB with corresponding Tx/Rx beam pointing angles is changed among different validation datasets, while the number of UE side Rx beams is kept as 8. For training dataset, beam shape pattern is generated from Table 5.2.1.1-1, and for validation dataset, beam shape patterns are generated from Table 5.2.1.1-1, 5.2.1.1-2 and 5.2.1.1-3 respectively.

For AI scheme, beam pair prediction is considered, where 8 beam pairs from 256 beam pairs are selected with random beam set-B selection scheme and measured at each time instant within a time duration T1=8*40ms, and different beam pairs are selected for measurement among time instants within T1. Consequently, total of 64 different beams are used in AI input to predict L1-RSRP of 256, 128 and 64 beam pairs at each time instant within T2 for datasets generated from Table 5.2.1.1-1, Table 5.2.1.1-2 and Table 5.2.1.1-3 respectively. Input of AI model includes measured L1-RSRPs of beam pairs, corresponding Tx beam angles/IDs and Rx beam angles/IDs, and output of AI model is L1-RSRPs of all beam pairs in future time instants.

For comparison in generalization aspect, same dataset used for training and validation can be considered as an upper bound performance, and different AI input are evaluated and compared in Table 5.2.1.1-4~6.

Table 5.2.1.1-4: performance comparison for different beam shape pattern(T2=1*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| AI (baseline) | 16 x 8 | 16 x 8 | 0.57 | 79.11 | 95.05 |
| AI w beam angle | 32 x 8 | 16 x 8 | 0.80 (+40%) | 74.01 | 92.96 |
| AI w local beam ID | 32 x 8 | 16 x 8 | 12.58 | 9.17 | 45.17 |
| AI (baseline) | 8 x 8 | 8 x 8 | 0.33 | 84.78 | 97.45 |
| AI w beam angle | 32 x 8 | 8 x 8 | 0.64 (+93%) | 74.37 | 95.28 |
| AI w local beam ID | 32 x 8 | 8 x 8 | 8.06 | 20.90 | 67.33 |

Table 5.2.1.1-5: performance comparison for different beam shape pattern (T2=4*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| AI (baseline) | 16 x 8 | 16 x 8 | 0.71 | 78.06 | 94.53 |
| AI w beam angle | 32 x 8 | 16 x 8 | 0.95 (+33%) | 73.15 | 92.43 |
| AI w local beam ID | 32 x 8 | 16 x 8 | 12.78 | 8.93 | 44.28 |
| AI (baseline) | 8 x 8 | 8 x 8 | 0.45 | 83.64 | 96.98 |
| AI w beam angle | 32 x 8 | 8 x 8 | 0.75 (+66%) | 73.70 | 94.88 |
| AI w local beam ID | 32 x 8 | 8 x 8 | 8.30 | 20.33 | 66.76 |

Table 5.2.1.1-6: performance comparison for different beam shape pattern (T2=8*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| AI (baseline) | 16 x 8 | 16 x 8 | 0.91 | 76.33 | 93.70 |
| AI w beam angle | 32 x 8 | 16 x 8 | 1.17 (+28%) | 71.64 | 91.63 |
| AI w local beam ID | 32 x 8 | 16 x 8 | 13.04 | 8.57 | 42.99 |
| AI (baseline) | 8 x 8 | 8 x 8 | 0.61 | 81.94 | 96.32 |
| AI w beam angle | 32 x 8 | 8 x 8 | 0.91 (+49%) | 72.63 | 94.22 |
| AI w local beam ID | 32 x 8 | 8 x 8 | 8.64 | 19.48 | 65.72 |

From above tables for beam shapes pattern study, performance loss can be observed in AI method of inference with difference dataset which represents validation dataset has different beam shape pattern compared to training dataset. As assumption of 30km/h is used for dataset generation, beam prediction performance of AI based scheme without considering various beam shape pattern seems sufficient. Thus, the absolute performance deterioration is limited directly compared to the baseline, for example approximately 0.3 dB loss in L1-RSRP difference KPI between validation dataset with 8 Tx antennas in table 5.2.1.1-6, but with large relative performance loss, e.g. up to 93% degradation at same situation.

Besides, it can be observed that local beam ID used in AI input brings significant performance deterioration as the beam ID has different understanding between model training and model inference. Thus, we have following observation and proposal.

Performance loss can be observed if there is difference in beam shape patterns for training and validation in BM-Case2.

For the case using local beam ID as model input, beam loss and accuracy degenerate significantly compared to the performance of AI model training and inference with beam pointing angle.

- Further study generalization performance for different antenna configurations and different beam shapes in BM-Case2.

- Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case2.

- Suggest to use beam pointing angle or other physical IDs reflecting beam pointing angle information as assistance information for AI model input.

### Performance with proprietary processed assistance information

      - Beam pair prediction

From theoretical analysis, 4 types of AI input combination can be obtained for beam pair prediction with pre-configured pattern selection as below. Besides, in the following performance evaluation, a value-based mathematical function is used to map real beam information to a virtual beam information, for example, beam pointing angle of 70 degree may be mapped to 1.8243 after proprietary processing module. Such mapping function is same for training and inference.

Other simulation assumptions can be found in section 5.2.1.1. For comparison in generalization aspect, same dataset used for training and validation can be considered as an upper bound performance, different dataset used for training and validation with local beam ID is regarded as a lower bound performance, and different AI input schemes are evaluated and compared in the following tables.

Table 5.2.2.1-1: performance evaluation results for proprietary protection mechanism with a mapping order based mathematical function (T2=1*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| Local beam ID (baseline1) | 16 x 8 | 16 x 8 | 0.57 | 79.1 | 95.0 |
| Local beam ID (baseline2) | 32 x 8 | 16 x 8 | 12.5 | 9.17 | 45.1 |
| Beam angle | 32 x 8 | 16 x 8 | 0.80 | 74.0 | 92.9 |
| proprietary processed beam angle | 32 x 8 | 16 x 8 | 0.95 | 71.6 | 91.7 |

Table 5.2.2.1-2: performance evaluation results for proprietary protection mechanism with a mapping order based mathematical function (T2=4*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| Local beam ID (baseline1) | 16 x 8 | 16 x 8 | 0.71 | 78.0 | 94.5 |
| Local beam ID (baseline2) | 32 x 8 | 16 x 8 | 12.7 | 8.93 | 44.2 |
| Beam angle | 32 x 8 | 16 x 8 | 0.95 | 73.1 | 92.4 |
| proprietary processed beam angle | 32 x 8 | 16 x 8 | 1.10 | 70.6 | 91.2 |

Table 5.2.2.1-3: performance evaluation results for proprietary protection mechanism with a mapping order based mathematical function (T2=8*40ms)

| Method and input | Tx/Rx beam config. |  | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- | --- | --- |
|  | Training Data | Validation Data |  |  |  |
| Local beam ID (baseline1) | 16 x 8 | 16 x 8 | 0.91 | 76.3 | 93.7 |
| Local beam ID (baseline2) | 32 x 8 | 16 x 8 | 13.0 | 8.57 | 42.9 |
| Beam angle | 32 x 8 | 16 x 8 | 1.17 | 71.6 | 91.6 |
| proprietary processed beam angle | 32 x 8 | 16 x 8 | 1.33 | 69.0 | 90.3 |

For the case using proprietary processed beam angle, beam loss and accuracy degenerate slightly compared to the performance of the case using beam angle directly.

Conclusions

In this contribution, we discuss some issues on AL/ML for beam management and have the following observations:

- As classification model, i.e. option 1a/1b, is mainly used to obtain beam prediction accuracy while regression model, i.e. option 2a/2b/2c, can extra provide beam quality information, conducting a direct performance comparison between classification and regression models would be unfair and unreasonable.

- For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample, the performance differences brought about by different fixed patterns in Set B are significant in the KPI of beam prediction accuracy and L1-RSRP difference for both Set B = 1/8 Set A and Set B = 1/4 Set A.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 83%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/1.9/1.0/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 76% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.6/1.5/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction, the performance differences brought about by different fixed patterns in Set B are significant in the KPI of beam prediction accuracy and L1-RSRP difference for both Set B = 1/16 Set A and Set B = 1/8 Set A.

For beam pair prediction with Set B = 1/16 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For beam pair prediction with Set B = 1/16 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL beam pair

- 66% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 72%/84%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 2.0/1.0/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 2.5 dB predicted L1-RSRP difference for Top-1 DL beam pair

For beam pair prediction with Set B = 1/8 Set A, similar performance can be achieved for a fixed pattern in set B with or without assistance information.

For beam pair prediction with Set B = 1/8 Set A, a fixed pattern in Set B without explicitly assistance information achieves approximately,

- 74% beam prediction accuracy of Top-1 DL beam pair

- 84% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 89%/96%/98% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 0.5/0.2/0.07/0.04 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 1.2 dB predicted L1-RSRP difference for Top-1 DL beam pair

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, random pattern in Set B with Tx beam information brings significant performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, random pattern in Set B with Tx beam information as assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL Tx beam

- 61% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 75%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 5.0/2.7/1.5/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, random pattern in Set B with Tx beam information brings significant performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, random pattern in Set B with Tx beam information as assistance information achieves approximately,

- 66% beam prediction accuracy of Top-1 DL Tx beam

- 70% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 81%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/2.0/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, random pattern in Set B with both Tx beam information and Rx beam information brings considerable gain compared to the same Set B without assistance information.

For beam pair prediction with Set B = 1/16 Set A, random pattern in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 21% beam prediction accuracy of Top-1 DL beam pair

- 27% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 36%/55%/66% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 7.5/4.9/2.7/1.8 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 5.8 dB predicted L1-RSRP difference for Top-1 DL beam pair

For beam pair prediction with Set B = 1/8 Set A, random pattern in Set B with both Tx beam information and Rx beam information brings considerable gain compared to the same Set B without assistance information.

For beam pair prediction with Set B = 1/8 Set A, random pattern in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 29% beam prediction accuracy of Top-1 DL beam pair

- 36% beam prediction accuracy for Top-1 DL beam pair prediction with 1dB margin

- 47%/66%/77% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL beam pair

- 5.5/3.3/1.6/1.0 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL beam pair

- 4.9 dB predicted L1-RSRP difference for Top-1 DL beam pair

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 69% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.3/1.9/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/1.9/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.5/2.0/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, the performance improvement from assistance information becomes more apparent with the number of pre-configured patterns in Set B increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, same performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx beam information as assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 5 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 5 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 83%/90%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.9/1.0/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 10 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 10 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.8/0.9/0.6 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 20 pre-configured patterns in Set B with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, 20 pre-configured patterns in Set B with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 77% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/92%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.9/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, the performance improvement from assistance information becomes more apparent with the number of pre-configured patterns in Set B increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, same performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx beam information as assistance information.

For beam pair prediction with Set B = 1/16 Set A, 5 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 5 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 53% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 71%/84%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.2/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, 10 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 10 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 52% beam prediction accuracy of Top-1 DL Tx beam

- 62% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 70%/84%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.3/1.2/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, 20 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, 20 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 50% beam prediction accuracy of Top-1 DL Tx beam

- 60% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 68%/82%/88% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.5/1.3/0.6/0.4 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, the performance improvement from assistance information becomes larger with the number of pre-configured patterns in Set B increases.

For beam pair prediction with Set B = 1/16 Set A, closed performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx/Rx beam information as assistance information.

For beam pair prediction with Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 5 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 68% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/93%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.8/0.3/0.1/0.07 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 10 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 65% beam prediction accuracy of Top-1 DL Tx beam

- 76% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/92%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.0/0.4/0.1/0.09 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.9 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set B without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, 20 pre-configured patterns in Set B with Tx/Rx beam information as assistance information achieves approximately,

- 63% beam prediction accuracy of Top-1 DL Tx beam

- 74% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.2/0.5/0.2/0.1 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.1 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, the performance improvement from assistance information becomes larger with the number of pre-configured patterns in Set B increases.

For beam pair prediction with Set B = 1/8 Set A, closed performance can be achieved among different number of pre-configured patterns in Set B, i.e. 5/10/20 pre-configured patterns, with Tx/Rx beam information as assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 5 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 5 beams with Tx beam information as assistance information achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 76% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 6 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 6 beams with Tx beam information as assistance information achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 7 beams with Tx beam information brings some performance improvement compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 7 beams with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 85%/91%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.1/1.6/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 8 beams with Tx beam information brings considerable gain compared to the same Set B without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, Set B of 4 beams selected from Set C of 8 beams with Tx beam information as assistance information achieves approximately,

- 72% beam prediction accuracy of Top-1 DL Tx beam

- 77% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 84%/92%/94% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.0/1.7/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, the performance improvement from assistance information becomes more apparent with the number of beams in Set C increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/8 Set A, marginal performance improvement can be achieved among different number of beams in Set C with Tx beam information as assistance information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 9 beams without or with different assistance information provides similar performance.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 9 beams with Tx beam information as assistance information achieves approximately,

- 73% beam prediction accuracy of Top-1 DL Tx beam

- 78% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 85%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.9/1.5/0.8/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 10 beams with Tx beam information brings marginal performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 10 beams with Tx beam information as assistance information achieves approximately,

- 75% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.7/1.4/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 11 beams with Tx beam information brings marginal performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 11 beams with Tx beam information as assistance information achieves approximately,

- 75% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/92%/95% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.7/1.4/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 12 beams with Tx beam information brings some performance improvement compared to the same Set C without assistance information, while no performance improvement is observed from Rx beam information.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, Set B of 8 beams selected from Set C of 12 beams with Tx beam information as assistance information achieves approximately,

- 76% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/93%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.6/1.3/0.7/0.5 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For DL Tx beam prediction with the best Rx beam assumption searched from the best Tx beam within Set B per model input sample and Set B = 1/4 Set A, marginal performance improvement can be achieved among different number of beams in Set C with Tx beam information as assistance information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 17 beams with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 17 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.1/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 18 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 18 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 64% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 72%/85%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.1/1.1/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 19 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 19 beams with Tx/Rx beam information as assistance information achieves approximately,

- 55% beam prediction accuracy of Top-1 DL Tx beam

- 65% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 2.0/1.0/0.5/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.7 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 20 beams with Tx beam information and/or Rx beam information brings considerable gain compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/16 Set A, Set B of 16 beams selected from Set C of 20 beams with Tx/Rx beam information as assistance information achieves approximately,

- 56% beam prediction accuracy of Top-1 DL Tx beam

- 66% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 74%/86%/91% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 1.9/0.9/0.4/0.3 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 2.6 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/16 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For beam pair prediction with Set B = 1/16 Set A, closed performance can be achieved among different number of beams in Set C with Tx/Rx beam information as assistance information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 33 beams with Tx beam information and/or Rx beam information brings similar performance improvement compared to the same Set C without assistance information, as well as similar performance using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 33 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 80% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 86%/94%/96% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.08 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 34 beams with Tx beam information and/or Rx beam information brings similar performance improvement compared to the same Set C without assistance information, as well as similar performance using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 34 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.4 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 35 beams with Tx beam information and/or Rx beam information brings marginal performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 35 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 36 beams with Tx beam information and/or Rx beam information brings some performance improvement compared to the same Set C without assistance information, while similar performance can be observed using both or either one of Tx beam and Rx beam information.

For beam pair prediction with Set B = 1/8 Set A, Set B of 32 beams selected from Set C of 36 beams with Tx/Rx beam information as assistance information achieves approximately,

- 71% beam prediction accuracy of Top-1 DL Tx beam

- 82% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 87%/95%/97% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 0.7/0.3/0.1/0.06 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For beam pair prediction with Set B = 1/8 Set A, the performance improvement from assistance information becomes larger with the number of beams in Set C increases.

For beam pair prediction with Set B = 1/8 Set A, closed performance can be achieved among different number of beams in Set C with Tx/Rx beam information as assistance information.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15%-30% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 67% beam prediction accuracy of Top-1 DL Tx beam

- 72% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 81%/89%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.1/1.1/0.7 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15%-30% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, closed performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 14% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with fixed pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 73% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 82%/89%/93% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.1/1.3/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 0.5 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 15% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 53% beam prediction accuracy of Top-1 DL Tx beam

- 57% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 73%/85%/89% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 5.6/3.0/1.7/1.1 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 4.3 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides obvious performance gain about 12%~18% in beam prediction accuracy related KPI compared to Option 1 with the best Rx beam searched from the best Tx beam within Set B and Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, similar performance can be provided from Option 1 with the best Rx beam searched from the best Tx beam within Set A and Option 1 with the best Rx beam searched from the best Tx beam within Set B in predicted L1-RSRP difference, while significant degradation of predicted L1-RSRP difference can be observed for Option 1 with the best Rx beam for each Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with random pattern and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 63% beam prediction accuracy of Top-1 DL Tx beam

- 67% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 77%/86%/90% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 4.3/2.5/1.5/1.0 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 3.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides the best performance compared to Option 1 with other Rx beam searching methods.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, similar performance of predicted L1-RSRP difference and approximately 20% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam searched from the best Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/8 Set A, approximately 10% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam for each Tx beam within Set B if compared to Option 1 with the best Rx beam for each Tx beam within Set B, while Option 1 with the best Rx beam for each Tx beam within Set B takes significant performance degradation in predicted L1-RSRP difference.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 10% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/8 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 65% beam prediction accuracy of Top-1 DL Tx beam

- 69% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/88%/91% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 4.1/2.2/1.4/0.9 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.8 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, Option 1 with the best Rx beam searched from the best Tx beam within Set A provides the best performance compared to Option 1 with other Rx beam searching methods.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, similar performance of predicted L1-RSRP difference and approximately 20% performance degradation can be observed from Option 1 with the best Rx beam searched from the best Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A.

For DL Tx beam prediction with 10-pre-configured patterns and Set B = 1/4 Set A, approximately 14% beam prediction accuracy degradation can be observed from Option 1 with the best Rx beam for each Tx beam within Set B if compared to Option 1 with the best Rx beam searched from the best Tx beam within Set A, while Option 1 with the best Rx beam for each Tx beam within Set B takes significant performance degradation in predicted L1-RSRP difference.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set A and Option 2 with the best Rx beam searched from one specific Tx beam within Set B provides similar performance, which is close to the performance of Option 1 with the best Rx beam searched from the best Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with one fixed Rx beam and Option 3 with one random Rx beam provides similar performance, while approximately 15% performance loss can be observed in beam prediction accuracy related KPI in comparison with Option 2 with the best Rx beam searched from one specific Tx beam within Set B.

For DL Tx beam prediction with 10 pre-configured patterns and Set B = 1/4 Set A, Option 2 with the best Rx beam searched from one specific Tx beam within Set B achieves approximately,

- 70% beam prediction accuracy of Top-1 DL Tx beam

- 75% beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin

- 80%/89%/92% beam prediction accuracy for Top-2/1, Top-4/1 and Top-6/1 DL Tx beam

- 3.7/2.2/1.1/0.8 dB average L1-RSRP difference for Top-1, Top2/1, Top4/1 and Top6/1 DL Tx beam

- 1.0 dB predicted L1-RSRP difference for Top-1 DL Tx beam

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 4~-5% performance loss in beam prediction accuracy compared to non-quantitative case, as well as negligible performance loss in predicted L1-RSRP difference.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 7~10% performance loss in beam prediction accuracy and 1.5dB degradation in predicted L1-RSRP difference compared to non-quantitative case.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For DL Tx beam prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 0.1~1% performance loss in beam prediction accuracy compared to non-quantitative case, as well as negligible performance loss in predicted L1-RSRP difference.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B without considering quantization range, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the quantization approach that aligns with existing specification, i.e. 1dB quantization step for the best beam and 2 dB quantization step for the difference to the best beam, results in approximately 15~20% performance loss in beam prediction accuracy and 2~3.5dB degradation in predicted L1-RSRP difference compared to non-quantitative case.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance improvement of enhanced quantization method with 1dB quantization step for the best beam and 1 dB quantization step for the difference to the best beam is extremely limited in comparison with existing quantization method.

For beam pair prediction with fixed pattern, random pattern and 10 pre-configured patterns in Set B with considering quantization range from -140dBm to -44dBm, the performance degradation of low-precision quantization methods, including differential quantization step from 3dB to 5dB, are not obvious compared to existing quantization method.

For DL Tx beam prediction, Option 1a and Option 2a provide identical performance of beam prediction accuracy with different Set B schemes, i.e. fixed pattern, random pattern, pre-configured pattern and Set C, while option 2a can provide additional predicted beam quality information.

For beam pair prediction, Option 2a provides a considerable performance improvement in beam prediction accuracy compared to Option 1a among different Set B schemes, i.e. fixed pattern, random pattern, pre-configured pattern and Set C, and additionally yields predicted beam quality information.

No performance loss can be observed from proprietary protection with mathematical function processing compared to beam prediction using beam angle directly, if a same processing function is maintained for training and inference.

More flexible AI model deployment for different number of Tx/Rx beams can be achieved through using expected Tx/Rx beam information method with only marginal performance loss.

Option 1, i.e., different pattern in each time instance of T1, achieves similar performance to Option 2a (same pattern in each time instance of T1 to traverse all patterns in Option 1), while requiring half the measurement resource overhead.

Option 1, i.e., different pattern in each time instance of T1, performs better than Option 2b (same pattern in each time instance of T1, where number of beams measured in each time instance is the same as that of Option 1), and uses the same measurement resource overhead.

For Rx assumption in T1, Option 1(Case 2, the best Rx beam for each Tx beam within Set B) offers a similar improvement in the terms of prediction accuracy and average RSRP difference results when compared to that of Option 1(Case 0, the best Rx beam for the best Tx beam within Set B), but a degradation in this aspect can be observed in Option 2 (Case1, the best Rx beam searched for one Tx beam within Set B).

For Rx assumption in T1, Option 1(Case5), i.e., the Rx beam of best beam pair within Set A, offers a similar improvement in the terms of prediction accuracy and average RSRP difference results when compared to that of Option 1 (Case2’, the best Rx beam for each Tx beam within Set A), but a degradation in this aspect can be observed in Option 2 (Case1, the best Rx beam searched for one Tx beam within Set B) and Option 3 (random Rx beam).

When number of time instance P increases, performance decreases in terms of prediction accuracy and average RSRP.

When Set B is the same and T1*M is the same, different T1 and M shows similar prediction accuracy and average RSRP.

For BM-Case2, compared with non-AI scheme, beam pair prediction scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly.

For Rx assumption in T2, Alt C, i.e., best Rx beam based on the predicted top-K Tx beams achieves best performance in terms of prediction accuracy and average RSRP difference results, Alt B (the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement) gets the second-best performance and Alt A (same Rx(s) as that of T1) performs worst.

For Rx Option 2(Case 1), performance gain introduced from extra measures before T1 (Alt B) or Rx beam sweeping after DL Tx beam prediction (Alt C) is much bigger than that of Rx Option 1(Case 0/Case2/Case 2’/Case5).

For Rx Option 3, performance gain introduced from Rx beam sweeping after DL Tx beam prediction (Alt C) is much bigger than that of Rx Option 2(Case 1) and Rx Option 1(Case 0/Case2/Case 2’/Case5).

The performance with different training and validation fixed patterns is quite poor and not acceptable, i.e., fixed set B selection scheme suffers serious generalization issue.

As the difference of beam shape pattern increases, the performance loss of both average RSRP difference and beam prediction accuracy increases along with the difference of the antenna configurations between training subset and validation subset.

Performance loss can be observed if there is difference in beam shape patterns for training and validation in BM-Case2.

For the case using local beam ID as model input, beam loss and accuracy degenerate significantly compared to the performance of AI model training and inference with beam pointing angle.

For the case using proprietary processed beam angle, beam loss and accuracy degenerate slightly compared to the performance of the case using beam angle directly.

- and proposals:

- For DL Tx beam prediction, the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1[/K] predicted Tx beam and the ideal L1-RSRP of the same Tx beam with the best Rx beam that results in the largest L1-RSRP over all Tx and Rx beams.

- Rx/Set B pattern assumption for beam pair prediction should be implementation of AI/ML model owner, and no need to be discussed. Companies can report the used pattern assumption for EVM.

- Prefer to compare results of different labels under same model type.

- Further study on minimizing quantization overhead with similar beam prediction performance as legacy NR beam report quantization, e.g., increasing quantization step of differential beams.

- Prefer Alt.1 and Alt.4, i.e. beam prediction accuracy related KPIs and predicted L1-RSRP difference, as performance metrics of AI/ML model monitoring for BM-case1 and BM-case2.

- Further clarify regarding Option D for various Set B based on one of the following two.

- Option D1: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C, and the number of beams in Set B is equal to the number of beams used at model input.

- Option D2: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C, and the number of beams in Set C is equal to the number of beams used at model input.

- Deprioritizes Option B in DL Tx beam prediction with quasi-optimal Rx beams.

- For EVM of BM Case 2, companies to report one of the following options for size of Set B and pattern assumption over multiple measurement occasions in T1,

- Option 1: different pattern is assumed for different measurement occasion within T1,

        - Option 1a: size of Set B is number of unique beams in all occasions.

        - Option 1b: size of Set B is number of unique beams per occasion.

- Option 2: same pattern is assumed for each measurement occasion within T1, and size of Set B is number of unique beams in all occasions/per occasion.

- For the evaluation of AI/ML in beam management, considering the following beam management procedures:

- Option 1: AI/ML can be used to predict Top-K Tx beams with corresponding RSRPs based on Set B. The output results can be directly used following same Rx beam assumption as the set B measurement.

- Option 2: AI/ML can be used to predict only Top-K Tx beams based on Set B. A mandatory extra P2 procedure to obtain RSRP of predicted beams is needed and a P3 procedure may also be configured for performance improvement.

- Option 3: AI/ML can be used to predict Top-K Tx-Rx beam pairs with corresponding RSRPs based on Set B. The output results can be directly used.

- Option 4: AI/ML can be used to predict only Top-K Tx-Rx beam pairs. An extra P2+P3 procedure should be configured to obtain corresponding RSRPs.

- Other options are not precluded and can be reported by companies.

- Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM Case 1. Detailed proprietary protection mechanism can be FFS.

- Support to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case1, where a same mapping function is maintained for training and inference.

- Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM-Case 2. Detailed proprietary protection mechanism can be FFS.

- Suggest to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case2, where a same mapping function is maintained for training and inference.

Study beam pair prediction with expected Tx/Rx beam information as the AI input as one of the solutions for generalization to different number of Tx/Rx beams in BM-Case1.

Further study expected information method in BM-Case2.

Further study multiple expected beam information simultaneously used in AI input.

- Study the Rx beam assumption cases in T1 for BM-case 2 based on the following principles,

- For Case 0/1/3/4/5, where one Rx beam is used for set B, the measurement for Rx beam determination is performed before T1

- For Case 2, where each Tx beam in set B uses its own best Rx beam, the measurement of Rx beam determination can be performed before T1 starts, or before each measurement occasion in T1

- Further study beam pair prediction scheme with expected information as AI input for improving generalization performance in BM-Case2.

- Study the issue of Rx assumption options for predicted beams and KPI calculation in T2 in BM Case 2. The following options can be considered as a start point

- Alt A: T2 uses the same Rx beam(s) as that of T1

- Alt B: T2 uses the best Rx beam(s) based on measurement for Rx determination for T1 and extra measurement before T1

- Alt C: T2 uses the best Rx beam(s) based on the predicted top-K Tx beams determined between T1 and T2

- Further study generalization performance for different antenna configurations and different beam shapes in BM-Case2.

- Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case2.

- Suggest to use beam pointing angle or other physical IDs reflecting beam pointing angle information as assistance information for AI model input.

References

Chairman’s notes for 3GPP TSG RAN WG #94-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 Meeting 110bis-e Meeting, October 10th – 19th, 2022

Chairman’s notes for 3GPP TSG RAN WG1 Meeting 111 Meeting, November 14th – 18th, 2022

Chairman’s notes for 3GPP TSG RAN WG1 Meeting 112 Meeting, February 27th – March 3rd, 2023

Chairman’s notes for 3GPP TSG RAN WG1 Meeting 112bis-e Meeting, April 17th – April 26th, 2023.

Appendix A: SLS simulation assumptions for spatial domain beam prediction

| Parameter | Value |
| --- | --- |
| Scenario | Uma with Dense Urban 38.901,7 sites, 3 cells per site |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| System BW | 80 MHz |
| BS and RRH Tx power | 40dBm |
| UE receiver NF | 10 |
| ISD | 200m |
| o2i | 0.5 |
| Antenna configuration at BS | [Mg Ng M N P] = [1 1 4 8 2], [dV, dH] = [0.5,0.5] λ |
| Antenna configuration at UE | [Mg Ng M N P] = [1 2 1 4 2], [dV, dH] = [0.5,0.5] λ |
| BS TX beam pattern | 32 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 67.5 112.5 157.5] |
| UE RX beam pattern | 4 Rx beams per panel Horizontal angle = [-67.5 -22.5 22.5 67.5] Vertical angle = [/] |
| Indoor UE fraction | 80% |
| UE speed | 3 km/s |
| Spatial consistency | False |
| Rotation | False |

Appendix B: SLS simulation assumptions for temporal domain beam prediction

| Parameter | Value |
| --- | --- |
| Scenario | Uma with Dense Urban 38.901,7 sites, 3 cells per site |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| System BW | 80 MHz |
| BS and RRH Tx power | 40 dBm |
| UE receiver NF | 10 |
| ISD | 200m |
| o2i | 0.5 |
| Antenna configuration at BS | [Mg Ng M N P] = [1 1 4 8 2], [dV, dH] = [0.5,0.5] λ |
| Antenna configuration at UE | [Mg Ng M N P] = [1 2 1 4 2], [dV, dH] = [0.5,0.5] λ |
| BS TX beam pattern | 32 Tx beams Horizontal angle = [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Vertical angle = [22.5 67.5 112.5 157.5] |
| UE RX beam pattern | 4 Rx beams per panel Horizontal angle = [-67.5 -22.5 22.5 67.5] Vertical angle = [/] |
| Indoor UE fraction | 0% |
| UE speed | 30km/h (baseline), 60km/h (optional) |
| Spatial consistency | True, Spatial consistency procedure A |
| Rotation | False |
| UE trajectory model | Option #4, Random direction straight-line trajectories, including direction change at the end of time interval |
| Orientation model | Option 1b, Randomly per-UE chosen for UE orientation initially, and UE orientation is fixed during SLS. |