3GPP TSG RAN WG1 #113		R1-2304481

Incheon, Korea, May 22nd – May 26th, 2023

Source:	vivo

Title:	Physical channel design framework for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

- Introduction

The WID for NR sidelink evolution has been approved for Rel-18 in RAN#94e [1]. The channel access mechanisms and the physical channel design framework for sidelink on unlicensed spectrum were agreed for further investigation:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we will discuss the physical channel design framework for sidelink (SL) on unlicensed spectrum.

- Physical Channel design Framework

Operating Scenario

In NR-U, the communication is established between one gNB and one UE, with the gNB having complete control over the operation bandwidth (i.e., one or multiple RB sets) of the UE for communication. Although different bandwidths are supported across different UEs, the BWP size, BWP positions, DCI contents, are not needed to be aligned among UEs.

However, this situation is different in the SL case. Sidelink UEs cannot control the operation bandwidth of other UEs, and it is not practical to force all Sidelink UEs to have the same capabilities. Therefore, it is necessary to support the direct communication between different UEs supporting different bandwidths in the same resource pool. Specifically, there are several different scenarios as shown in Figure 1. Scenario 1 is the communication in fully overlapped RB set(s) between two SL UEs as shown in Figure 1(a). The second scenario (Scenario 2) is the communication in partially overlapped between two SL UEs as shown in Figure 1(b). And the third scenario (Scenario 3) is the communication between a BW-limited UE (i.e., supporting only one RB set) and another advanced UE (WB UE, i.e., supporting more than one RB set) as shown in Figure 1(c). In this scenario, different BW-limited UEs may work in different RB sets.

Figure 1：Scenarios in SL-U

Proposal 1: The Rel-18 SL-U design should support SL communication between different UEs capable of different operating bandwidths (e.g., supporting different RB sets) in the resource pool.

Communication between different UEs capable of different operating bandwidths

In Scenario 2 and Scenario 3, when WB UE indicates a wideband transmission with the SCI transmitted in the lowest RB set of PSSCH as Rel-16, the BW-limited UE cannot receive the transmission if it locates in an RB set other than the RB set where the SCI is sent. It may cause the resource collision problem because the BW-limited UE cannot identify the resources reserved by the SCI in the other RB sets, as shown in Figure 2. To resolve this issue, the WB UE should transmit SCI in every RB set where the allocated PSSCH spans, and avoid reserving resources through a SCI in RB set other than the RB sets of the receiver.

Observation 1: When a UE transmits a PSSCH spanning multiple RB sets, if the corresponding SCI is sent only in the sub-channel of one RB set (e.g., lowest RB set), the other UEs operating in different RB sets (e.g., due to bandwidth limited) cannot receive the transmission.

Proposal 2: When a UE transmits a PSSCH spanning multiple RB sets, the resource indication (e.g., sub-channel indexes, RB set indication, etc.) should be sent in every RB set where the allocated PSSCH spans, so that the BW-limited UE in every RB set can obtain the resource reservation and avoid any potential collision.

Nevertheless, still, there is another issue in Scenario 2 and Scenario 3. When the resource pool of one UE might only contain a subset of RB sets of the resource pool of another UE, the sub-channel index within RB set 1 from different UEs’ perspectives would be different. This leads to different sub-channel indexing in two pools, as well as starting sub-channel, FRIV, etc. Consequently, the frequency resource indication would be misaligned between UEs.

It is worth noting that, this issue not only happens between a pair of UEs performing unicast transmissions, in which case the issue may be somehow mitigated by exchanging the UE capability. It also affects the case of groupcast and broadcast transmissions, and the other UEs performing sensing but not intending to receive the packets, in which cases no UE capability reporting is possible.

Observation 2：The frequency resource indication might be misaligned between UEs supporting different BWs.

For example, a BW-limited UE reserved a blue color resource in Figure 3. The BW-limited UE only supports one RB set bandwidth, so it indicates this resource in SCI with sub-channel index 2 ~ sub-channel index 5 of RB set 0. However, sub-channel index 2 ~ sub-channel index 5 of RB set 0 means the orange color resource to another WB UE performing SL sensing operation. Consequently, the WB UE may exclude the wrong resources, or may select improper resources, due to the incorrect sensing results.

| Figure 2: Resource collision triggered by SCI misdetection | Figure 3: Resource collision triggered by FRIV misunderstanding |
| --- | --- |

To solve this problem, one solution is that all the UEs maintain a same RB set indexing, starting RB set and max number of RB sets. This can be achieved by establishing a common ‘virtual’ resource pool based on the max number RB sets, which may be larger than the BWPs or actual resource pools configured for the UEs. This common resource pool is used for indicating SL resources allocated in the actual resource pool. In this case, the resource indications, even from different resource pools in different RB sets, can be aligned. What’s more, there are only two parameters that need to be added compared to legacy spec, i.e., the starting RB set index and the max number of RB sets, which need little spec effort. This mechanism is somewhat like the common RB grid introduced in NR R15, or the common interlace grid introduced in NR-U.

Figure 4: Common ‘virtual’ resource pool

For example, the UEs can be configured with a resource pool spanning RB set 2 and 3, and a common ‘virtual’ resource pool spanning all the RB sets (0, 1, 2, and 3). Then the UE can be additionally indicated with a starting sub-channel index for the actual resource pool as Figure 4. Hence, the starting sub-channel of BW-limited UE in its supporting bandwidth is not index 0 but the indicated sub-channel index. Thus, the FRIV in Rel-16 can be reused, but based on the common ‘virtual’ resource pool instead of the actual resource pool. Note that, the max number of sub-channels used for deriving the FRIV in SCI, etc., is given by the common ‘virtual’ resource pool.

Proposal 3: When UEs supporting different BWs coexist in the resource pool, the BW-limited UE should be configured with additional frequency domain parameters of the resource pool of WB UE, i.e., the RB set indexing, starting RB set and max number of RB sets, and uses these parameters for deriving the SCI (e.g., FRIV, etc.), so that the resource reservations can be correctly understood between different UEs.

With the abovementioned enhancements, at least the coexistence of UEs supporting different BWs in the same resource pool is feasible. Only minor spec changes are expected.

Observation 3: Only little spec changes are needed to support the coexistence of UEs supporting different BWs in the same resource pool.

Nevertheless, in order to support the communication between the UEs supporting different BWs in the resource pool, some additional (minor) enhancements are needed. Firstly, when a WB UE intends to transmit a TB to the BW-limited UE, the resource selection of candidate resources should be limited to the RB set where the BW-limited UE is operating. Noted that it is similar to the case that some RB sets are not available (e.g., due to consistent LBT failure), thus the design can be reused. Secondly, the PSFCH from the WB UE to the BW-limited UE should also be limited to the PRB set that the BW-limited UE camps. It can be seen that the spec effort is acceptable to support the communication between different UEs capable of different operating bandwidths.

Observation 4：The required spec efforts (i.e., ensuring the PSSCH and PSFCH are transmitted in the RB set that BW-limited UE is operating in) are acceptable to support the communication between different UEs capable of different operating bandwidths.

Resource pool determination

For SL resource pool in unlicensed band, TDD UL/DL configuration is needed to determine the time slots belonging to the resource pool. However, in unlicensed band, due to the uncertainty of the channel availability, TDD configuration with flexible symbols/slots is much more flexible, which is desirable for NRU deployment. Thus, configuring all the slot as flexible is the common understanding in NRU. Consequently, there will be very limited UL slots or even no UL slots. In this case, SL UE may have a problem on the determination of the time slots for resource pool. To solve this problem, a straightforward way is to mandatorily configure enough UL slots in the TDD UL/DL configuration. However, this will reduce the flexibility of the transmissions in the unlicensed band, and consequently degrade the Uu performance. The other alternatives can be allowing SL transmission in flexible slots or assuming all the slots are valid for SL transmission or configuring separate TDD UL/DL configuration for SL transmission. The separate TDD UL/DL configuration can be carried in RRC signaling other than SIB1, and adequate UL slots can be provided for SL transmission.

Proposal 4: The issue of no UL slot being available for deriving SL resources in the unlicensed band due to the deployment of NRU which configures all the slots in the unlicensed band as flexible should be addressed.

Proposal 5: Consider the following options in the determination of SL resource when there is no UL slot in the TDD configuration for unlicensed band:

Ｏption1: The flexible slots can be used to be configured as SL resources.

Option 2: A separate TDD UL/DL configuration (other than SIB1) is used to derive the SL resources.

Option 3: All the slots are assumed as UL slots to derive the SL resources, and the SIB1 TDD UL/DL configuration is ignored.

Another open issue on the resource pool configuration is whether/how to support one SL resource pool that includes a sub-set of PRBs of one RB set. The purpose of supporting a partial RB set resource pool is to support multiple FDMed resource pools in one RB set. However, the justification for multiple FDMed pools in an RB set is not clear. If it is intended to support FDMed transmissions from different UEs in a single RB set, this can also be achieved in a single pool via the existing resource allocation mechanism. Furthermore, the feasibility of FDMed transmissions in FDMed pools in an RB set is questionable. Since the unit of LBT is one RB set, if the channel access fails, UE is not allowed to transmit in any of the FDMed pools. Even when channel access succeeded, transmission from a UE on one pool may block the other UEs on other pools if their CPEs are not aligned. Thus, the benefit of FDMed resource pool configuration is unclear. In addition, the sub-set of RB set configuration would restrict the utilization of RB set. For example, if the TB size is large, this type of RB set cannot be used to transmit even if the LBT succeeds. Therefore, it’s not necessary to support one SL resource pool that can include sub-set of PRBs of one RB set.

Proposal 6: It’s not necessary to support one SL resource pool that can include sub-set of PRBs of one RB set.

SL sub-channel and interlace

Interlace RB-based Sub-channel

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets. - In a resource pool, support the following - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0 - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on - At least support that the above K interlace(s) are contiguous - FFS: whether/how to support the above K interlace(s) are non-contiguous - FFS: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace for this case will be further discussed - Interlace is indexed as per NR-U Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s) - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc. - FFS others E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

In the last meeting, it has been agreed to support explicitly indicating the used sub-channel index(s) and RB set index(s) in the frequency domain resource indication for interlace RB-based PSSCH transmission, and the R16 NR SL FRIV is agreed to be reused as the baseline. In R16 NR SL FRIV, the number of sub-channels of all the initial transmission and reservation of the resource(s) for retransmission(s) is the same. Thus the frequency domain resource indication of initial transmission and retransmissions can be coded jointly.  An open issue here is whether to support different number of RB set(s) while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB. The reason behind changing the number of RB set(s) of retransmission lies in the uncertainty of the probability of channel access.. However, how much gain can be achieved by supporting different number of RB set(s) for initial transmission and retransmissions is questionable as the chance of channel access probability is not predictable. Even if the probability of successful channel access may change heavily, the MAC layer could trigger resource re-selection to adjust the number of allocated RB sets. It is important to note that in most cases, UE should try to select the minimum RB sets for both initial transmission and retransmission, as long as the performance can be ensured, considering that the transmission would be blocked if any one of the allocated RB sets fails in the channel access procedure. In addition, to support different number of RB set(s) while keeping total number of sub-channels unchanged, the number of subchannels used in one RB set can be different among the initial transmission and each retransmission. In this case, the frequency domain resource indication of each transmission should be indicated separately, resulting in a significantly more complex and bit-intensive signalling design. Therefore, it’s not preferred to support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB.

Proposal 7: It’s not preferred to additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB.

Considering one PSSCH transmission may occupy one or multiple RB sets, how to select the number of RB sets should be determined. Since the transmission cannot be performed as long as there is one unsuccessful LBT detection in all the channels, the number of the allocated RB sets should be limited as much as possible, especially when the TB size is small. In addition to TB size, there is some other information, e.g. priority and CBR, should be considered to determine the number of RB sets. For example, if the priority is higher, the reserved resources would more likely be maintained (as the lower priority transmission shall not preempt them). In this case, more RB sets may be allocated as the success rate of access may be higher. In addition, when the CBR measurement is high (e.g., higher than a threshold), it means there are a lot of transmission in the resource pool or in the RB set. So, the transmission is more likely to be blocked and the channel access possibility of multi-RB sets transmission would be affected heavily. Thus, the number of allocated RB sets should be limited in this case.

Proposal 8: The resource allocation procedure in the case of multiple RB sets should consider the parameter of the transmission, e.g. TB size, priority and CBR, to determine the number of RB sets for the TB.

In addition, such determination (i.e., the number of selected RB sets to be used for PSSCH transmission) should be informed to the PHY layer. Otherwise, it may cause confusion of the size of candidate resource(s). For example, if MAC layer indicates the number of sub-channels to be used for PSSCH transmission is 2, whether it means the candidate resources includes 2 sub-channels of every RB set in the resource pool or just part of the RB sets is not clear. With the number of RB sets, the accurate candidate slot size can be determined. Alternatively, MAC layer may determine the number of RB set after the report of PHY layer. In this case, the size of candidate resource informed by MAC layer is the total number of sub-channels across all the allocated RB sets. This means the number of RB sets of candidate resources is not fixed. Consequently, the candidate resource set in this option is more complicated.

Proposal 9: To indicate the size of the candidate resource accurately, the number of RB sets to be used for the PSSCH transmission should also be indicated from the MAC layer.

Contiguous RB-based Sub-channel

Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113: - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission - FFS details, e.g., conditions to apply the above Option(s) - FFS impacts on definition of candidate resource, and resource selection

Regarding the mapping between sub-channel and PRBs in contiguous RB-based PSCCH/PSSCH transmission, it has been agreed that the mapping of sub-channels starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size.

Figure 5: The mapping between sub-channels and PRBs

Thus, the intra-cell guard band could likely be contained within one or two sub-channels. As the guard band PRBs can be used only when the both of two adjacent RB sets are accessed successfully and utilized for transmissions, the allocation of the sub-channels containing intra-cell guard band PRBs needs to be discussed.

Firstly, it should be discussed whether to allow the part of the sub-channel excluding intra-cell guard band to be allocated as the other normal sub-channels. If these sub-channels can be utilized only when the intra-cell guard band PRBs are used, the resource allocation efficiency would be decreased, and the OCB requirement may not be satisfied in some cases (depending on the sub-channel configurations). For example, as shown in Figure 5, if UE only allocates RB set #0, only sub-channel #0 and sub-channel #1 could be selected. This results in a low resource allocation efficiency. Moreover, the OCB requirement (80%) always couldn’t be met without allocating sub-channel #2 if the sub-channel size is too large. However, these issues can be avoided through reasonable sub-channel configurations.

On the contrary, if the partial sub-channel transmission is allowed, PSCCH and PSSCH can locate in the part of sub-channel rather than intra-cell guard band. Thus, the resource efficiency is better, and the OCB requirement can be met. However, this is different from the legacy mechanism in that the sub-channel is the basic unit of resource allocation, and would affect many other aspects, such as the TBS determination.

Proposal 10: Partial sub-channel transmission is not supported, i.e. the part of the sub-channel excluding the intra-cell guard band cannot be allocated as a standalone sub-channel.

As a result, the sub-channels including intra-cell guard band PRBs would not be allocated for PSCCH/PSSCH transmission if only one RB set is allocated. When there is more than one RB set allocated, if such sub-channels can be used for PSCCH transmissions, the size of the sub-channel other than the intra-cell guard band part must be larger than 10 PRBs. Otherwise these sub-channels still cannot be used for PSCCH transmission. Consequently, from the perspective of resource allocation, the complexity of Option 2 is higher than Option 3. Meanwhile, compared to Option 3, the increasement of resource allocation flexibility brought by Option 2 is not obvious, as the resource allocation in Option 2 only allows a few more cases that the starting sub-channel of candidate resources is the last sub-channel in one RB set. For example, when the resource pool is configured with two RB sets and one RB set consists of N sub-channels, there are at most N more resource allocation possibilities of Option 2. The gain seems to be quite minor and cannot justify the additional design efforts and implementation complexity. Therefore, Option 3 is preferred.

Proposal 11: Option 3 (Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission) is preferred.

TBS determination of unequal sub-channel size

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings: - Option A: - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113: - Option A1: N_ref is a fixed value, e.g., 10, 11 - Option A2: N_ref is (pre-)defined - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces. - Option A3: N_ref is (pre-)configured - Option A4: N_ref is dynamically indicated by Tx UE - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration. FFS: for TBS determination, whether/how to handle the impact of additional available PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

For interlace RB-based transmission in SLU, it has been agreed that the TBS is determined based on a reference number of PRBs of on interlace within 1 RB set (denoted as N_ref). In our views, N_ref should be (pre-)configured. Firstly, regarding the division of the interlace in a carrier, i.e., each interlace consists of {m, M+m, 2M+m, 3M+m, …} PRBs, where M is the number of interlaces, the PRBs of interlace are approximately evenly distributed across one or more RB set. For example, when the nominal intra-cell guard bands and the corresponding sizes of the RB sets are configured for wideband operation, as shown in Table 1, the number of PRBs of an interlace within one RB set is either 10 or 11 for 40MHz channel bandwidth with 15kHz SCS and is 10 for 40/60/80MHz channel bandwidth with 30kHz SCS. Even if the actual intra-cell guards and corresponding sizes of the RB sets can be configured by the network, the configurations of intra-cell guards and RB sets may only affect the PRB numbers of interlace but the difference between the PRB numbers of interlace is small. Moreover, thanks to the quantization step of TBS determination, the slight difference between the number of PRBs of allocated subchannels and the number of actual PRBs may have minor impact on TBS determination. Last but not the least, the TBS does not necessarily to be determined precisely based on the actual PRBs. Since Rel-16, a (pre-)configured overhead of reference signal such as DMRS and CSI-RS instead of the actual number of REs used for RS is used in TBS determination. Thus, N_ref is possible to be (pre-)configured with rate-matching/puncturing, instead of dynamically indicated by TX UE due to the existence of the inter-cell guard band.

| SCS (kHz) | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- |
| 15 | 105-6-105 (216) | N/A | N/A |
| 30 | 50-6-50 (106) | 50-6-50-6-50 (162) | 50-6-50-5-50-6-50 (217) |
| 60 | 23-5-23 (51) | 23-5-23-5-23 (79) | 23-5-23-5-23-5-23 (107) |
| NOTE 1: The intra-cell guard band is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the carrier as indicated by offsetToCarrier. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between parentheses. |  |  |  |

Table 1 Nominal intra-cell guard bands for wideband operation

For contiguous RB-based transmission, since the mapping between the subchannel and PRBs is the same as in legacy NR SL, the subchannel size can be (pre-)configured per resource pool and each subchannel in a resource pool has the same number of PRBs regardless of the existence of inter-cell guard band. If it is agreed that the sub-channel containing intra-cell guard band PRB cannot be used for PSSCH, there is no unequal sub-channel size issue for contiguous RB-based transmission on TBS determination.

Proposal 12: For interlace RB-based PSCCH/PSSCH transmission, N_ref can be (pre-)configured on TBS determination.

PSCCH on unlicensed spectrum

The additional starting symbols within a slot has been discussed with the following agreement.

RAN1#109 [3] Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission RAN1#110bis [4] Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details

It has been agreed that PSCCH is transmitted within 1 sub-channel. The number of PRBs for PSCCH can be {10, 12, 15, 20, 25}. Considering the number of PRBs in each interlace will be 10 or 11 for 20MHz bandwidth, the number of PRBs for PSCCH in an RB set in SLU can be 10 or 20 PRBs depending on the number of interlaces configured to a sub-channel.

Proposal 13: The number of PRBs for PSCCH in an RB set can be 10 or 20 for interlaced transmission depending on the number of interlaces configured to a sub-channel.

One remaining issue is when the transmitted PSSCH spans multiple RB sets, whether the PSCCH is transmitted in one of the RB sets or in every RB set that the PSSCH is spanning. As discussed in section 2.1, it is important to configure PSCCH in each RB set and transmit PSCCH in all the RB sets of the corresponding PSSCH, in order to support UEs with different operating bandwidths (e.g., supporting different numbers of RB sets) to co-locate in the same resource pool.  Thus, the option 2 from the agreement should be supported. In each RB set, the SCI in the PSCCH can indicate either the allocated PSSCH within the RB set, or across all the RB sets.

Proposal 14: PSCCH locates in every RB set of corresponding PSSCH. The SCI in the PSCCH can indicate either the allocated PSSCH within the RB set, or across all the RB sets.

For slots with two candidate starting symbols for a PSCCH transmission, there are some issues to be considered. For example, the mini-slot based SL transmission requires multiple PSCCH transmission occasions within a slot. As we discussed before, when wideband operation is supported, PSCCH transmission occasion within each RB set may be necessary. Therefore, if mini-slot based SL transmission is performed over a wide bandwidth larger than 20MHz, UE may need to perform PSCCH detection at the beginning of each mini-slot and in each RB set, which may far beyond the UE capability. Therefore, some mechanisms to reduce the blind detection complexity of PSCCH for UE should be supported.

One potential way is to introduce preamble detection before the PSCCH detection. If the UE detects the preamble, it can continue performing PSCCH detection, otherwise, it can skip the PSCCH detection. The AGC symbol, or the second half of the AGC symbol, can be used to transmit preamble. The preamble can be m-sequence, Gold-sequence, etc. The starting symbol can be identified by a sequence with a pre-defined/pre-configured initial value, or root sequence. The starting symbol can also be identified by a sequence with an initial value that is mapped to the S-PSS/S-SSS ID or the CRC for PSBCH. This approach requires to design a new physical layer signal. Moreover, the decoding complexity of preamble must be significantly lower than decoding SCI, otherwise, the benefits of reduced decoding burden saved by SCI BD will be lost.

Alternatively, the PSCCH detection procedure can be enhanced. For example, the decoding number of PSCCH can be configured for mini-slot transmission, e.g., only a subset of the PSCCH/sub-channel should be blindly decoded. The subset can be pre-configured based on different granularity, e.g., per-resource pool, per-RB set, etc. The UE can only decode PSCCH on the configured subset, etc., and ignore other candidate PSCCHs. One example is that, when PSCCH locates in every RB set of the corresponding PSSCH, gNB can configure the UE to detect PSCCH in the RB set with only even index or odd index. More generally, the UE can detect the PSCCH in the RB set with index mod (N, M), where N is the total number of RB sets, and M is a positive integer configured by gNB.

Proposal 15: The PSCCH detection complexity should be reduced when more than one starting symbol is configured, for example, by introducing preamble before the transmitted PSCCH, or enhancing the PSCCH detection procedure by decoding only a subset of PSCCH candidates.

PSSCH on unlicensed spectrum

The additional starting symbols within a slot has been discussed with the following agreement.

RAN1#109 [3] Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission RAN1#110bis [4] Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details RAN1#111 [5] Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot RAN1#112 [6] Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 RAN1#112bis [6] Agreement Regarding Tx UE behavior, at least when it initiates a COT: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS applicable scenarios - e.g., at least for MCSt with no greater than 16us gap - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE - FFS: Rx UE behavior FFS: COT sharing case Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference number of symbols (denoted as L_ref) - Support the followings - Alt 1: Support Option 4 only - Note: the options are as below - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, If PSCCH/PSSCH transmission starts from 1st starting symbol, it is preferred that the PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions (i.e., Option 3). If the second AGC symbol is always transmitted (i.e., Option 1), it leads to an additional overhead of 1/12 for the PSSCH transmission in the slot without PSFCH. One example of the condition is that, when Tx UE reserves all the sub-channels of the resource pool in a slot, there is very low probability that another UE will occupy the second half of the slot for transmission due to the channel reservation information and the LBT procedure. Therefore, the Tx UE can transmit AGC only in the first starting symbol, and the second AGC symbol can be used to transmit PSSCH. When the Tx UE reserves a sub-set of the sub-channels of the resource pool in a slot, there is relatively high probability that another UE may occupy the other sub-channels in the second half of the slot for mini-slot based transmission. Therefore, the Tx UE should transmit AGC in both of the first and second starting symbols. For Rx UE, it can also determine the AGC detection based on the Tx UE’s channel reservation information.

The performance of Option 1 and Option 3 are evaluated. The following figure shows the system simulation results, where alt-1 is fixed to transmit 2 symbols AGC per slot (i.e., Option 1) and alt-2 is to transmit the second AGC conditionally (i.e, Option 3). It can be observed that 9.9% UPT performance gain can be achieved by Option 3.

Figure 6 Simulation results of UE averaged UPT

Proposal 16: If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, e.g., the 2nd AGC symbol is absent if all the sub-channels are allocated by the UE.

Regarding the transmission starting positions within the COT, it has been agreed that for the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. However, in the remaining COT, whether a Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission is still under discussion. In our view, at least for the COT initiating UE, the 1st starting symbol should be chosen for PSCCH/PSSCH transmission to maintain the COT, especially if it does not share the COT to others. Considering the responding UE, it should also be able to access the channel with more than one opportunity if the Type 2A channel access procedure is used. That is to say, a responding UE as Tx UE in the remaining COT can choose the 2nd starting symbol if it fails to transmit at the 1st starting symbol. When a UE operates as a receiving UE, since different Tx UEs may transmit to the Rx UE from different starting positions, the Rx UE has to detect at both the 1st or the 2nd starting symbol within a slot.

Proposal 17: The COT initiating UE should transmit only at the 1st starting symbol in the remaining COT.

Proposal 18: The responding UE can transmit at either the 1st or the 2nd starting symbol in the remaining COT.

Proposal 19: The Rx UE should always detect at both the 1st and the 2nd starting symbol within a slot.

Another issue is that, when there are 2 AGC symbols in a slot, how does the UE (e.g., transmitting slot based PSSCH) map the RE on the 2nd AGC symbol. The UE should perform rate matching or puncture around the second AGC symbol. If AGC symbol conflicts with DMRS, the UE can transmit DMRS in the symbol after AGC symbol to maintain the channel estimation performance, or choose another DMRS pattern if possible (probably smaller number of DMRS symbols) that is not conflict with the AGC symbol if the performance degradation is acceptable. When the second AGC symbol overlaps with PSSCH, it should be excluded from the PSSCH reference resources when determining the TBS.

Proposal 20: When there are two AGC symbols in a slot, the UE should perform rate-matching or puncturing around the second AGC symbol for the slot-based PSSCH transmission.

Proposal 21: When there are two AGC symbols in a slot, the UE should shift the DMRS symbol overlapped with the second AGC symbol for the slot-based PSSCH transmission.

Proposal 22: When there are two AGC symbols in a slot, the UE should exclude the second AGC symbol for determining the TBS of the slot-based PSSCH transmission.

Moreover, it has been agreed that the reference symbol length L_ref should be (pre-)configured on TBS. For PSSCH transmission, the number of REs allocated for PSSCH within a PRB is determined by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, and the number of REs that can be used for 2nd stage SCI is determined by  QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ, where NsymbolPSSCH=Nsymbsh-NsymbPSFCHand NsymbshNsymbslotNsymbslot = sl-LengthSymbols -2. If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the TBS can be determined by replacing sl-LengthSymbols with L_ref to have less spec impact, where L_ref can be the reference number of sidelink symbols within the slot provided by higher layers. And NsymbPSFCH can be determined in the same way as in legacy NR  SL.

Proposal 23: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the TBS should be determined by replacing sl-LengthSymbols with L_ref and  NsymbPSFCH should be determined in the same way as in legacy NR  SL.

PSFCH on unlicensed spectrum

In RAN1#112b-e meeting [5], the design of the PSFCH transmission to meet OCB and PSD, the location of the PSFCH resources, etc., have not been decided yet.

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS: - RAN1 down-select one of followings in RAN1#113: - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured, FFS value range - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB - FFS: whether any impact on meeting OCB requirement - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - K2=2 - K4 is (pre-)configured, FFS value range - FFS: how to meet PSD limitation - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace - R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details - Note: companies are encouraged to give more details and analyze the specification impact - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc. - E.g., whether introducing more than 6 CS pairs is needed - E.g., for group cast option 2, what’s the maximum group size that can be supported - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH” - FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR - FFS: whether IBE issue exists and whether/how to address it - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB Agreement Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings: - One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration - FFS value range of N - FFS detailed design of such N associated candidate PSFCH occasion(s) - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc. - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s) - FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios. - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them - FFS other details - E.g., how to meet the HARQ RTT restriction

Enhancements for LBT failure

To support one PSCCH/PSSCH transmission that has N associated candidate PSFCH occasion(s) via (pre-) configuration, one remaining issue is the detailed design of such N associated candidate PSFCH occasion(s). As discussed in section 2.5.4, it is important to support the PSSCH transmission and its associated PSFCH resource in the same RB set(s) for communication between UEs with different operating bandwidths in the resource pool. Therefore, the associated candidate PSFCH occasion(s) of one PSSCH/PSSCH transmission can only be in different slots of the same RB set(s) to increase the possibility of channel access.

Proposal 24: The associated candidate PSFCH occasion(s) of one PSCCH/PSSCH transmission should be in different slots of the same RB set(s).

Another remaining issue is whether to support dynamic indicating the availability of the (pre-)configured PSFCH occasion. In our view, it is beneficial for the COT initiating UE to reduce the possibility of losing the COT due to the absence of PSFCH transmission within the COT. Specifically, if a UE initiates a COT with two (pre-)configured PSFCH occasions, the UE can indicate to disable the two PSFCH occasions such that the other UE cannot share the COT to transmit PSFCH, and the two disabled PSFCH occasions can be used for PSSCH transmission to maintain the COT. Thus, the COT initiating UE can dynamically enable/disable the (pre-)configured PSFCH occasion within its COT.

Proposal 25: The COT initiating UE can dynamically indicate the availability of the (pre-)configured PSFCH occasion within its COT.

In addition, the RX UE transmits the PSFCH in a first slot (i.e., the legacy PSFCH occasion) that includes PSFCH resources and is at least 2 or 3 slots after the last slot of the PSSCH reception in R16 NR SL. When supporting N associated candidate PSFCH occasions for a PSCCH/PSSCH transmission, the period of PSFCH occasion is (pre-) configured per resource pool and the RX UE can attempt to transmit the PSFCH in the legacy PSFCH occasion and the subsequent N-1 PSFCH occasion(s), as shown in Figure 7. The RX UE only needs to transmit one shot PSFCH among the N associated candidate PSFCH occasion. If the RX UE fails to access the channel and cannot transmit PSFCH in one PSFCH occasion, it can attempt to transmit the PSFCH in the next PSFCH occasion.

Figure 7 One PSSCH and its corresponding three PSFCH occasions

Proposal 26:  The N associated PSFCH occasion(s) of one PSCCH/PSSCH should consist of  the first PSFCH occasion that is at least 2 or 3 slots after the last slot of this PSCCH/PSSCH and the subsequent N-1 PSFCH occasion(s).

Capacity enhancements for PSFCH

Another remaining issue is whether additional capacity enhancement for PSFCH (e.g., OCC, PRB-level cyclic shifts) is needed. Such kind of enhancement is to increase the PSFCH capacity assuming that introducing the interlace for PSFCH may reduce the PSFCH capability. For example, if one interlace occupies at least 10 PRBs and different UEs use different interlaces, the PSFCH resource capacity may be reduced by 10 times compared with Rel-16. However, since each PSSCH transmission requires to perform LBT, the actual number of required PSFCHs at each occasion may not be a problem.

The system level simulations based on the agreed simulation assumptions are performed to evaluated this problem. The number of actual PSFCH transmissions in each PSFCH occasion for the “single PSFCH occasion” case and the “multiple PSFCH occasions” case in unicast and groupcast option2 are shown in Figure 8 and Figure 9. The simulation result shows that the number of PSFCHs in a PSFCH occasion in unicast is less than 4 at 95th percentile, and less than 12 in groupcast option2. The difference between “single PSFCH occasion” case and “multiple PSFCH occasions” case is negligible. Overall the maximum number of total PSFCHs in a PSFCH occasion is 24 (occurred in groupcast). Noted that if one PSFCH occupy one interlace and 6 pair of cyclic shifts are used, there are totally 60 candidate PSFCH resources in one PSFCH occasion. Therefore, the results show that there is no PSFCH capability issue when interlace-based PSFCH transmission is applied no matter in unicast or groupcast option2.

| Figure 8 Total number of PSFCHs in a PSFCH occasion for unicast | Figure 9 Total number of PSFCHs in a PSFCH occasion for groupcast option2 |
| --- | --- |

Observation 5: According to the simulation results, there is no PSFCH capability issue when interlace-based PSFCH transmission is applied no matter in unicast or groupcast option2.

Proposal 27: Further capacity enhancement for interlace based PSFCH is not needed.

PSFCH resource mapping

Regarding the mapping rule between PSSCH and PSFCH, it has been agreed that R16 NR SL PSSCH-PSFCH mapping is reused as the baseline. Thus, when each PSFCH transmission occupies 1 dedicated interlace, UE could first determine the location of PSFCH PRB according to the legacy mapping rule and then map the PSFCH PRB to a corresponding interlace as shown in Figure 10.

Proposal 28: UE first determine the location of PSFCH PRB according to the PSFCH mapping rule and then map the PSFCH PRB to a corresponding interlace

| Figure 10: How to map from PRB to interlace | Figure 11: The PSFCH mapping rule of interlace |
| --- | --- |

Given that the number of interlaces in one PSFCH occasion slot is limited, different PSFCH PRBs may be mapped to the same interlace. This means PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources. To distinguish these PSSCH transmissions, 6 CS pairs could be applied. Thus, there could be up to 60 (10*6) orthogonal PSFCH resources in one PSFCH occasion within one RB set in 15KHz, while the number of the corresponding PSSCH transmissions on non-overlapped resources in one RB set is up to 40 when the PSFCH period is configured with the maximum value of 4. Similarly, the number of orthogonal PSFCH resources (30>20) in 30KHz is also enough. As shown in Figure 11, when the PSSCH transmissions on slot index #0 and slot index #1 allocate the sub-channel with the same frequency, the corresponding PSFCH PRB is the same one. But they can be distinguished from the code domain by using different CS pairs.

Proposal 29: The CS pairs could be applied to distinguish the PSSCH transmissions on non-overlapped resources if the PSFCH transmission is based on dedicated interlace.

When there are more than one PSFCH occasion per PSCCH/PSSCH transmission, the PSFCH corresponding to PSSCH transmissions associated with multiple PSFCH periods would be mapped into one PSFCH occasion. Then how to update the mapping rule based on R16 NR SL PSSCH-PSFCH mapping should be discussed.

One option (Option 1) is that the set of PSFCH resources for one PSFCH period continues after the allocation of another set of PSFCH resources within one PSFCH occasion. To be specific, the configured PSFCH PRBs for PSFCH transmission can be divided into multiple sub-sets and thus the allocation in each PSFCH period associated with one PSFCH occasion can reuse the R16 NR SL PSSCH-PSFCH mapping. For example, as shown in Figure 12, when assuming the PSFCH period is 2, and there are 2 PSFCH occasions per PSCCH/PSSCH transmission, the PSFCH PRB allocation in slot index 5 corresponding to slot index #2 and slot index #3 in the second PSFCH period continues after the allocation corresponding to slot index #0 and slot index #1 in the first PSFCH period.

Another option (Option 2) is to refine the mapping of the PSSCH slots associated with a PSFCH slot. In R16 NR SL PSSCH-PSFCH mapping, the PSSCH slots associated with a PSFCH slot are restricted as the number of slots in one PSFCH period. Hence, when there are more than one PSFCH occasion (e.g., K) per PSCCH/PSSCH transmission, it just needs to extend the mapping of the PSSCH slots to all the PSSCH slots of the K PSFCH periods. As shown in the Figure 13, the allocation would start in an ascending order of all the PSSCH slots, i.e. sub-channel 0 in slot index #0, slot index #1, then slot index #2 and slot index #3, associated with the PSFCH occasion in slot index #5, and continues in an ascending order of the sub-channel.

By contrast, Option 2 need less spec effort thus Option 2 is preferred.

Proposal 30: Regarding the PSFCH mapping rule when there are more than one PSFCH occasion per PSCCH/PSSCH transmission, the Option 2 that redefining the definition of the PSSCH slots associated with a PSFCH slot is preferred.

| Figure 12：The PSFCH mapping rule with multiple PSFCH occasions | Figure 13: The PSFCH mapping rule with multiple PSFCH occasions |
| --- | --- |

In addition, if there are more than one PSFCH occasion per PSCCH/PSSCH transmission, the maximum number of PSSCH transmissions on non-overlapped resources would increase. However, as discussed in previous section, in the unlicensed band, the capacity is not an issue as the number of simultaneous PSSCH transmissions in one RB set is limited under the restriction of channel access. This can also be seen from the above simulation results. Nevertheless, given the limited orthogonal PSFCH resources, it is possible to properly define the PSSCH-PSFCH mapping rule to reduce the PSFCH collision rate. Considering that, due to the LBT requirement, the number of simultaneous PSSCH transmissions in one RB set is limited as discussed before, which means that the probability of FDMed transmission in one RB set is low. What’s more, as the frequency allocation is in an ad-hoc and distributed manner, the probability that the frequency allocations of the FDMed transmissions are adjacent is further decreased. Hence, the PSFCH resource corresponding to some adjacent sub-channels in one RB set could be allowed to be the same. For example, the corresponding PSFCH of more than one sub-channel in one RB set could be mapped to the same PSFCH PRB. As shown in Figure 14, both of the corresponding PSFCH of the PSSCH on sub-channel index #0 and sub-channel index #1 are mapped to the second PSFCH PRB in slot index #1 when the PSFCH period is 1 slot. It’s expected that few SL UE could successfully access the RB set simultaneously and allocates the adjacent subchannel. Even if this situation happens, they can be identified in the code domain by different CS pairs. What’s more, the more interlaces are included in one sub-channel, the fewer orthogonal PSSCH resources exist in the resource pool. Thus, with proper PSSCH-PSFCH mapping rule between the sub-channel and interlace, the PSFCH collision rate can be limited.

Proposal 31: Regarding the PSSCH-PSFCH mapping rule, the corresponding PSFCH of more than one sub-channel in one RB set could be mapped to the same PSFCH PRB to further reduce the PSFCH collision possibility.

Figure 14:The PSFCH mapping rule with multiple PSFCH occasions

Finally, if the PSFCH capability is really a concern, the PRB-level cyclic shift hopping designed in NRU could be applied. The different cyclic shifts of PRB in the interlace can be used to distinguish the PSSCH transmissions on non-overlapped resources. Thus, the PSFCH capability can be extended obviously.

PSFCH transmission in multiple RB sets

In the case of PSFCH transmission in multiple RB sets, there are several issues to be resolved. Firstly, it should be determined whether the PSFCH resource is located in one RB set or in multiple RB set. As discussed in section 2.1, the communication between different UEs capable of different operating bandwidths in the resource pool should be supported. If the PSFCH resource is configured within a single RB set, according to the existing PSCCH/PSFCH mapping rule, a PSFCH resource in one RB set may be associated with a PSSCH in another RB set, as shown in Figure 15. This is unacceptable for BW-limited UE since it cannot receive its PSFCH in other RB set.

Another way is to configure the PSFCH resource set in a common RB set where BW-limited UE can receive, as shown in Figure 15. But this design cannot maximize the utilization of Wideband RB set(s), and may also lead to PSFCH capacity issue. Moreover, the common RB set would become congestion, because all the SL UE (especially for BW-limited UE) should transmit in the RB set.

A better approach is to configure the PSFCH resource set in each RB set, then a PSFCH resource is associated with a PSSCH in the same RB set. BW-limited UE can receive the PSFCH in the corresponding PSCCH RB set, as shown in Figure 15. Moreover, it can provide more PSFCH opportunities to PSSCH in frequency domain, e.g., if there is a PSSCH spanning three RB sets, there are three candidate PSFCH resources can be used to feedback the PSSCH reception. It can also resolve the PSFCH collision issue for the case when the sub-channel is indexed within each RB set as discussed in 2.2, i.e., the sub-channel index cannot uniquely identify the actual used sub-channel of PSSCH. Given that PSFCH is configured in each RB set, the Rel-16 mapping rule can still be used within each RB set.

Figure 15 PSFCH mapping rule

Proposal 32: PSFCH resource set should be configured in each RB set in the resource pool in SL-U.

S-SSB on unlicensed spectrum

- candidate S-SSB positions in the frequency domain

Agreement For S-SSB transmission within 1 RB set, for 15 kHz and 30 kHz SCS, Alt6 is supported: - Alt 6: Support both Option 3-1(revised) and Option B, and enable one of them by (pre-)configuration Note: the Options are as below - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), whether/how to reduce PAPR. - FFS gap of 0 - Option A: Legacy S-SSB - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption. - Option B: Legacy S-SSB - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption. Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

In the last meeting, it was agreed to support multiple S-SSB transmissions in the frequency domain to meet OCB requirements. The number of S-SSB transmissions in the frequency domain and the gap between them should be pre-configured rather than pre-defined to meet the deployment requirements of different OEMs and NW.

The below table below summarizes the gap between two S-SSB transmissions required to meet OCB when two S-SSBs are configured in FDM manner in an LBT channel. To support better S-SSB coverage, more than two S-SSB transmissions can be configured if needed.

Figure 16 option 3-1

Table 2. Gap between S-SSB repetitions in an LBT channel

| SCS | S-SSB BW 11 RB | Single LBT channel(20MHz) | 80% of LBT channel | Gap between S-SSB repetition if two repetitions are configured |
| --- | --- | --- | --- | --- |
| 15kHz | 1.98MHz | 106 RB | 88.8 RB | e.g. 67~84 RB |
| 30kHz | 3.96MHz | 51 RB | 44.4 RB | e.g. 23~29 RB |
| 60kHz if supported | 7.92MHz | 24 RB | 22.2 RB | e.g. 1~2 RB |

Regarding the sequence of these S-SSB transmissions, there is no need to restrict all S-SSB transmissions to share a same sync sequence. Sending S-SSBs with different sync sequences can achieve the benefit of reducing PAPR. One possible design is that the (pre-)configuration provides an NR-ARFCN number (e.g., sl-AbsoluteFrequencySSB-r18) and synchronization ID (e.g., sl-SSID-r18) corresponding to a S-SSB in a RB set, as well as the gap between S-SSBs. Based on these parameters, the frequency-domain position of each S-SSB, and sync sequence can be derived.

sl-AbsoluteFrequencySSB-r18      ARFCN-ValueNR                                                       OPTIONAL, -- Need R

UE performs synchronization procedure at least based on the S-SSB position corresponding to the NR-ARFCN. Additionally, different IDs for other S-SSBs can be separately provided. If UE obtains additional IDs (e.g. indicated by addtionalSL-SSIDConfigList-r18) for other S-SSBs, it can use the IDs to generate sync sequences; otherwise, it uses the same ID for all S-SSB repetitions in the frequency domain.

sl-SSID-r18                        INTEGER (0..671)                                                        OPTIONAL,    -- Need R

SL-SSB-FreqAllocation-r16 ::=          SEQUENCE {

sl-numOfSSBFrequencyInRBset      INTEGER (1..4),

gapBetweenSSBFrequencyInRBset    INTEGER (1..106),

}

addtionalSL-SSIDConfigList-r18 ::=          SEQUENCE (SIZE (1..maxAddtionalSL-SLSSIDConfig-r18)) OF addtionalSL-SLSSIDConfig-r18    OPTIONAL, -- Need R

addtionalSL-SLSSIDConfig-r18                INTEGER (0..671)                                                        OPTIONAL,    -- Need R

Proposal 33.The number of S-SSB transmissions in a RB set and the length of gap between S-SSB transmissions in a RB set are (pre-)configured. The number of S-SSB transmissions and the length of gap between S-SSB transmissions for different RB sets in a SL BWP are the same.

Proposal 34. For S-SSB transmission(s) in a RB set, one NR-ARFCN and one SLSSID are (pre-)configured to determine the frequency position of one of the S-SSB transmissions. At least the S-SSB is used for synchronization purpose, it is up to UE to use other S-SSB transmission(s) for synchronization.

Proposal 35. SLSS ID(s) for other S-SSB transmission(s) in the frequency domain other than the S-SSB transmission corresponding to the provided NR-ARFCN can be separately (pre-)configured. If not (pre-)configured, all S-SSB transmission(s) in the RB set use the same SLSS ID for sync sequence generation.

Another remaining issue is whether to support S-SSBs on multiple RB sets. For the following two reasons, S-SSBs on multiple RB sets should be supported:

- Maintaining COT: due to the restriction of half-duplex, S-SSBs cannot be FDMed with another channel. If sending S-SSBs on multiple RB sets is not allowed, the UE may lose the COT.

- Coexistence with narrowband UE: as discussed in an earlier section 2.1, it is crucial to ensure that UE with different capabilities can communicate with each other in the same system for SL-U. Some low-capability UEs may support fewer RB sets (e.g., a Narrow-band UE may only support one RB set), and different low-capability UEs may choose different RB sets for communication. To ensure that these low-capability UEs can maintain synchronization with high-capability UEs, both Narrow-band UEs and Wide-band UEs should transmit S-SSB on all the RB sets they access. S-SSBs transmission(s) on the additional RB sets can be a duplication of that of the single RB set.

Proposal 36. Support S-SSB transmission(s) on multi-RB set to maintain COT and for coexistence among UEs with different capabilities.

- Additional candidate S-SSB occasions in the time domain

S-SSB candidate occasions in Rel-16/17 can be equally distributed, and each S-SSB candidate occasion can be considered as a S-SSB burst. Once the LBT launched for an S-SSB burst fails, UE must wait until the next S-SSB burst. Even if LBT to an S-SSB candidate succeeds, a new LBT should be launched for the next S-SSB candidate occasion due to the gap between the bursts, which also increases the latency and complexity of the sync procedure. To improve the number of access opportunities and reduce the impact of the gap, more S-SSB bursts are needed. For example, each set of the additional S-SSB candidate occasions corresponding to Rel-16/17 S-SSB candidate occasions can form a S-SSB burst. Additionally, the additional S-SSB candidate occasions can be consecutive or non-consecutive, as illustrated in case1/case2 in Figure 17. The following options were proposed to address the (pre-)configuration of additional candidate S-SSB occasions:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

- Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

- Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

- Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

Option1 reuses the legacy signaling but is less flexible compared with option2/option3. It cannot achieve the S-SSB pattern in Figure 17 while option2 and option3 can.

In option2, the interval between Rel-16/17 NR SL candidate S-SSB occasion in slot n and the additional candidate S-SSB occasion can be expressed in the unit of physical slot. For example, if the interval is m physical slots, then physical slot n+m is the slot where the additional candidate S-SSB occasion is located.

If option3 is used, the additional candidate S-SSB occasion can be (pre-)configured by using similar signaling as Rel-16/17 S-SSB. For example, with the following parameters, the additional candidate S-SSB occasions either occupy a set of consecutive slots or are equally spaced over a period of 160 ms.

AdditionalSL-SSB-TimeAllocation-r18 ::=          SEQUENCE {

Additionalsl-NumSSB-WithinPeriod-r16             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                          OPTIONAL,    -- Need R

Additionalsl-TimeOffsetSSB-r18                   INTEGER (0..1279)                                                   OPTIONAL,    -- Need R

Additionalsl-TimeInterval-r18                    INTEGER (0..639)                                                    OPTIONAL     -- Need R

}

Option4 and option5 can be covered by option2 and option3, thus option2/3 are preferred.

Figure 17 S-SSB bursts

Proposal 37. Regarding the number and location(s) of additional candidate S-SSB occasion(s) in time domain, Option2/3 can be supported.

Regarding whether to attempt to access additional candidate S-SSB occasions, the following conclusion has been reached. However, as a result of this conclusion, the UE may choose neither to send the R16 S-SSB, nor to send on any additional candidate S-SSB occasions. In Rel-16, S-SSB reception and transmission are basic capabilities that UE must support because it is crucial to maintain the same timing in a cluster of SL UEs. Regarding Rel-18, at least when the UE fails on all Rel-16/17 occasions, the UE shall perform LBT on additional candidate S-SSB occasions. For other cases (e.g., when UE transmitted S-SSB on at least on Rel-16/17 candidate S-SSB occasion), it can be left up to the UE to decide whether to perform LBT or not.

Conclusion Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

Proposal 38. If UE failed to transmit S-SSB on all the Rel-16/17 S-SSB occasion(s) in a S-SSB period, UE shall perform LBT for the corresponding Rel-18 additional S-SSB occasion(s).

Power control on unlicensed band

Agreement Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

Operation in unlicensed spectrum is subject to different regulatory requirements in some regions and frequency bands. To constrain the inter-RAT (Wi-Fi, WiGig radar) and intra-RAT interference levels, any technology attempted accessing the unlicensed spectrum (like SL-U) should comply with the transmit power requirement, in terms of EIRP and PSD. For instance, according to the ETSI regulation [2], the maximum mean EIRP and PSD in the 5150-5350 MHz band are limited to 23 dBm and 10 dBm/MHz, respectively, while in the 5470-5725 MHz band are limited to 30 dBm and 17 dBm/MHz, respectively.

Open-loop power control is adopted in NR Sidelink to determine the transmit power. The existing power control mechanism only restricts the maximum transmit power (e.g., 23 dBm), but does not consider the PSD. For example, considering the transmit power requirement for SL-U UEs to operate in 5150-5350 MHz band, the Peak PSD may be achieved if the SL-U UEs transmit with the maximum transmit power spreading over the 20MHz channels (23 dBm - 10×log10 (20 MHz) =10 dBm/MHz, noted that the PSD limitation is per region based). If the SL transmission with the maximum transmit power occupies a smaller bandwidth (i.e., less than 20MHz), the PSD would exceed the PSD limit. This issue would be critical especially for PSFCH and S-SSB transmission, because this kind of transmission is very unlikely to occupy the full 20MHz channel bandwidth. Thus, SL-U power control enhancement is necessary to comply with the PSD limit.

Proposal 39: SL-U power control enhancement should be introduced considering the PSD limit, especially for some SL channels such as PSFCH or S-SSB.

For a given PSD constraint of 10 dBm/MHz, the Peak PSD conducted from the SL-U transmission shall not be greater than the PSD limit, e.g., 10 dBm in every 1MHz band. To make full use of the maximum transmit power, SL-U power control under the PSD constraint should depend on the frequency domain distribution of the SL-U transmission. For example, according to the Rel-16 power control mechanism, the transmit power is equally distributed over multiple PSFCHs to be transmitted in the same PSFCH occasion, while the specific frequency domain location among the PSFCH are irrelevant to each other. However, on the unlicensed band, a (unevenly distributed) frequency location may lead to PSD exceeding the corresponding limit. As shown in Figure 18, when transmitting the same number of PSFCHs with transmit power of 5 dBm for each PSFCH, the mapping of PSFCH occasion 1 (in the left) has no problem, but the mapping of PSFCH occasion 2 (in the right) exceeds the peak PSD limit (10 dBm/MHz). Therefore, in order to comply with the PSD requirement, the power control mechanism should consider the frequency domain distribution of the PSFCH transmission.

Figure 18 Frequency domain distribution of PSFCH

Proposal 40: SL-U power control under the PSD constraint should consider the frequency domain distribution of the SL-U transmission.

There are some potential solutions to this issue.

- Option-1: In each PSFCH occasion, after the frequency domain locations of all the PSFCHs are generated, the UE determines the Tx power of a PSFCH according to not only the maximum Tx power, but also the PSD limitation.

- Option-2: In each PSFCH occasion, the UE first performs the existing PSFCH power control procedure, and then further drops some of the PSFCHs iteratively (similar to the existing PSFCH prioritization rule) until the PSD limitation is met.

In Option-1, when a UE is scheduled to multiple PSFCH transmissions, the frequency domain locations of all the PSFCHs are generated. The UE first determines the maximum PSD power according to the frequency domain locations of all the scheduled PSFCHs so that the peak PSD does not exceed the PSD limitation. For example, as shown in Figure 18, the maximum PSD power in PSFCH occasion 1 is 12.22dBm (i.e., 10dBm -10log3+10log5=12.22dBm) and the maximum PSD power in PSFCH occasion 2 is 10.97dBm (i.e., 10dBm -10log4+10log5=10.97dBm). Then the UE determines the TX power and number of PSFCH transmission (similar to the existing PSFCH power control procedure) so that the total PSFCH transmit power is no more than the minimum value between the maximum TX power and the PSD limit. Nonetheless, this approach would be a bit complicated.

In Option-2, the UE first performs PSFCH power control based on the existing procedure, i.e., the total PSFCH transmit power is subject to the maximum TX power. Then, the UE should further check if the determined PSFCHs comply with the PSD limitation. If the PSD requirement cannot be met, the UE drops some PSFCHs or reduces the PSFCH Tx power until the PSD limitation is met. For example, as shown in Figure 18, if the determined PSFCH is distributed as in PSFCH occasion 2 and the Tx power of a PSFCH is 5dBm, the peak PSD is 11.02 dBm/MHz ((i.e., 5dBm +10log5=11.02dBm) which exceeds the PSD limitation. In this case, UE start to drop some PSFCHs according to the priorities until the PSD requirement is met among all the PSFCHs (or alternatively, among the contiguous 4 PSFCHs in 1MHz). This solution may have lower complexity of UE implementation but may degrade the PSFCH performance.

Proposal 41: For PSFCH power control mechanism, the following enhancements can be considered:
Option1: In each PSFCH occasion, after the frequency domain locations of all the PSFCHs are generated, the UE determines the Tx power of a PSFCH according to not only the maximum Tx power, but also the PSD limitation.
Option2: In each PSFCH occasion, the UE first performs the existing PSFCH power control procedure, and then further drops some of the PSFCHs iteratively (similar to the existing PSFCH prioritization rule) until the PSD limitation is met.

Regarding the PSSCH transmission, the power of PSSCH transmission is determined by the number of resource blocks of the actual transmission. Therefore, it is straightforward to determine a maximum PSD transmission power limit based on the number of resource blocks for the actual PSSCH transmission, and then to apply to the power control mechanism. For example, if a contiguous PSSCH is scheduled to occupy 18MHz bandwidth, the maximum PSD transmission power is 22.55 dBm (i.e., 10dBm/MHz+10log18MHz=22.55dBm), so that the power of this PSSCH transmission should no more than 22.55dBm (i.e., the minimum value between maximum PSD transmission power (22.55dBm) and maximum UE transmission power (23dBm)). Similar update on the power control mechanism can be applied to S-SSB transmission.

Proposal 42: For PSSCH and S-SSB power control mechanism, the actual Tx power should be further restricted by the PSD limitation in addition to the existing maximum Tx power limit.

- Conclusion

In this contribution, we focus on the physical channel design framework for SL on unlicensed spectrum, and have the following observations and proposals:

Observation 1: When a UE transmits a PSSCH spanning multiple RB sets, if the corresponding SCI is sent only in the sub-channel of one RB set (e.g., lowest RB set), the other UEs operating in different RB sets (e.g., due to bandwidth limited) cannot receive the transmission.

Observation 2：The frequency resource indication might be misaligned between UEs supporting different BWs.

Observation 3: Only little spec changes are needed to support the coexistence of UEs supporting different BWs in the same resource pool.

Observation 4：The required spec efforts (i.e., ensuring the PSSCH and PSFCH are transmitted in the RB set that BW-limited UE is operating in) are acceptable to support the communication between different UEs capable of different operating bandwidths.

Observation 5: According to the simulation results, there is no PSFCH capability issue when interlace-based PSFCH transmission is applied no matter in unicast or groupcast option2.

Proposal 1: The Rel-18 SL-U design should support SL communication between different UEs capable of different operating bandwidths (e.g., supporting different RB sets) in the resource pool.

Proposal 2: When a UE transmits a PSSCH spanning multiple RB sets, the resource indication (e.g., sub-channel indexes, RB set indication, etc.) should be sent in every RB set where the allocated PSSCH spans, so that the BW-limited UE in every RB set can obtain the resource reservation and avoid any potential collision.

Proposal 3: When UEs supporting different BWs coexist in the resource pool, the BW-limited UE should be configured with additional frequency domain parameters of the resource pool of WB UE, i.e., the RB set indexing, starting RB set and max number of RB sets, and uses these parameters for deriving the SCI (e.g., FRIV, etc.), so that the resource reservations can be correctly understood between different UEs.

Proposal 4: The issue of no UL slot being available for deriving SL resources in the unlicensed band due to the deployment of NRU which configures all the slots in the unlicensed band as flexible should be addressed.

Proposal 5: Consider the following options in the determination of SL resource when there is no UL slot in the TDD configuration for unlicensed band:

Ｏption1: The flexible slots can be used to be configured as SL resources.

Option 2: A separate TDD UL/DL configuration (other than SIB1) is used to derive the SL resources.

Option 3: All the slots are assumed as UL slots to derive the SL resources, and the SIB1 TDD UL/DL configuration is ignored.

Proposal 6: It’s not necessary to support one SL resource pool that can include sub-set of PRBs of one RB set.

Proposal 7: It’s not preferred to additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB.

Proposal 8: The resource allocation procedure in the case of multiple RB sets should consider the parameter of the transmission, e.g. TB size, priority and CBR, to determine the number of RB sets for the TB.

Proposal 9: To indicate the size of the candidate resource accurately, the number of RB sets to be used for the PSSCH transmission should also be indicated from the MAC layer.

Proposal 10: Partial sub-channel transmission is not supported, i.e. the part of the sub-channel excluding the intra-cell guard band cannot be allocated as a standalone sub-channel.

Proposal 11: Option 3 (Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission) is preferred.

Proposal 12: For interlace RB-based PSCCH/PSSCH transmission, N_ref can be (pre-)configured on TBS determination.

Proposal 13: The number of PRBs for PSCCH in an RB set can be 10 or 20 for interlaced transmission depending on the number of interlaces configured to a sub-channel.

Proposal 14: PSCCH locates in every RB set of corresponding PSSCH. The SCI in the PSCCH can indicate either the allocated PSSCH within the RB set, or across all the RB sets.

Proposal 15: The PSCCH detection complexity should be reduced when more than one starting symbol is configured, for example, by introducing preamble before the transmitted PSCCH, or enhancing the PSCCH detection procedure by decoding only a subset of PSCCH candidates.

Proposal 16: If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, e.g., the 2nd AGC symbol is absent if all the sub-channels are allocated by the UE.

Proposal 17: The COT initiating UE should transmit only at the 1st starting symbol in the remaining COT.

Proposal 18: The responding UE can transmit at either the 1st or the 2nd starting symbol in the remaining COT.

Proposal 19: The Rx UE should always detect at both the 1st and the 2nd starting symbol within a slot.

Proposal 20: When there are two AGC symbols in a slot, the UE should perform rate-matching or puncturing around the second AGC symbol for the slot-based PSSCH transmission.

Proposal 21: When there are two AGC symbols in a slot, the UE should shift the DMRS symbol overlapped with the second AGC symbol for the slot-based PSSCH transmission.

Proposal 22: When there are two AGC symbols in a slot, the UE should exclude the second AGC symbol for determining the TBS of the slot-based PSSCH transmission.

Proposal 23: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the TBS should be determined by replacing sl-LengthSymbols with L_ref and  NsymbPSFCH should be determined in the same way as in legacy NR  SL.

Proposal 24: The associated candidate PSFCH occasion(s) of one PSCCH/PSSCH transmission should be in different slots of the same RB set(s).

Proposal 25: The COT initiating UE can dynamically indicate the availability of the (pre-)configured PSFCH occasion within its COT.

Proposal 26:  The N associated PSFCH occasion(s) of one PSCCH/PSSCH should consist of  the first PSFCH occasion that is at least 2 or 3 slots after the last slot of this PSCCH/PSSCH and the subsequent N-1 PSFCH occasion(s).

Proposal 27: Further capacity enhancement for interlace based PSFCH is not needed.

Proposal 28: UE first determine the location of PSFCH PRB according to the PSFCH mapping rule and then map the PSFCH PRB to a corresponding interlace

Proposal 29: The CS pairs could be applied to distinguish the PSSCH transmissions on non-overlapped resources if the PSFCH transmission is based on dedicated interlace.

Proposal 30: Regarding the PSFCH mapping rule when there are more than one PSFCH occasion per PSCCH/PSSCH transmission, the Option 2 that redefining the definition of the PSSCH slots associated with a PSFCH slot is preferred.

Proposal 31: Regarding the PSSCH-PSFCH mapping rule, the corresponding PSFCH of more than one sub-channel in one RB set could be mapped to the same PSFCH PRB to further reduce the PSFCH collision possibility.

Proposal 32: PSFCH resource set should be configured in each RB set in the resource pool in SL-U.

Proposal 33.The number of S-SSB transmissions in a RB set and the length of gap between S-SSB transmissions in a RB set are (pre-)configured. The number of S-SSB transmissions and the length of gap between S-SSB transmissions for different RB sets in a SL BWP are the same.

Proposal 34. For S-SSB transmission(s) in a RB set, one NR-ARFCN and one SLSSID are (pre-)configured to determine the frequency position of one of the S-SSB transmissions. At least the S-SSB is used for synchronization purpose, it is up to UE to use other S-SSB transmission(s) for synchronization.

Proposal 35. SLSS ID(s) for other S-SSB transmission(s) in the frequency domain other than the S-SSB transmission corresponding to the provided NR-ARFCN can be separately (pre-)configured. If not (pre-)configured, all S-SSB transmission(s) in the RB set use the same SLSS ID for sync sequence generation.

Proposal 36. Support S-SSB transmission(s) on multi-RB set to maintain COT and for coexistence among UEs with different capabilities.

Proposal 37. Regarding the number and location(s) of additional candidate S-SSB occasion(s) in time domain, Option2/3 can be supported.

Proposal 38. If UE failed to transmit S-SSB on all the Rel-16/17 S-SSB occasion(s) in a S-SSB period, UE shall perform LBT for the corresponding Rel-18 additional S-SSB occasion(s).

Proposal 39: SL-U power control enhancement should be introduced considering the PSD limit, especially for some SL channels such as PSFCH or S-SSB.

Proposal 40: SL-U power control under the PSD constraint should consider the frequency domain distribution of the SL-U transmission.

Proposal 41: For PSFCH power control mechanism, the following enhancements can be considered:
Option1: In each PSFCH occasion, after the frequency domain locations of all the PSFCHs are generated, the UE determines the Tx power of a PSFCH according to not only the maximum Tx power, but also the PSD limitation.
Option2: In each PSFCH occasion, the UE first performs the existing PSFCH power control procedure, and then further drops some of the PSFCHs iteratively (similar to the existing PSFCH prioritization rule) until the PSD limitation is met.

Proposal 42: For PSSCH and S-SSB power control mechanism, the actual Tx power should be further restricted by the PSD limitation in addition to the existing maximum Tx power limit.

- References

- RP-213678, WID on NR sidelink evolution, RAN #94e, Electronic Meeting, Dec. 6-17, 2021.

- ETSI EN 301 893, Harmonised Standard covering the essential requirements Harmonised Standard covering the essential requirements, v2.1.1, 2017-05.

- 3GPP RAN1#109-e, Chairman’s notes.

- 3GPP RAN1#110bis, Chairman’s notes.

- 3GPP RAN1#111, Chairman’s notes

- 3GPP RAN1#112, Chairman’s notes

- 3GPP RAN1#112bis, Chairman’s notes

- R1-2302486, Channel access mechanism for sidelink on unlicensed spectrum, vivo

- Annex I

Table 3 System level simulation assumption

| Parameter | Value |
| --- | --- |
| Deployment | Indoor |
| Communication type | Unicast |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15KHz |
| Channel model | P2P link defined in TR37.885 |
| Link adaptive | Enable |
| Traffic model for SL | R17 sidelink commercial traffic model with periodic model 3 reduced by a factor of 5 |
| Channel access | LBE |
| Tx Power | 23 dBm |
| Antenna number | [M N P Mg Ng Mp Np]=[1 1 2 1 1 1 1] for SL-UE [M N P Mg Ng Mp Np]=[1 2 2 1 1 1 1] for AP |
| UE deploy | 10 pairs of UEs per 20MH, uniformly at random in the area |
| Max multiple PSFCH number | 3 |
| CAPC | 2 |