# 3GPP TSG RAN WG1 #113    	                                             R1-2304538

# Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.2.4.1

Title:	Evaluation on AI positioning enhancement

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN Meeting #94e meeting, a new SID was approved to study AI/ML technologies over air interface [1].  In addition, RAN1#110 confirmed that both direct AI/ML positioning and AI/ML assisted positioning should be studied. In this contribution, we provide our further views on evaluation assumptions and evaluation results on AI/ML for positioning.

Common scenario parameters, InF specific parameters and assumptions for data generation are elaborated in Appendix A, Appendix B and Appendix C respectively. In addition, the evaluation methodologies for AI model input/output, AI model complexity and inference performance are detailed in excel templates for various scenarios/configurations.

Direct AI/ML positioning

Agreement in RAN1#109: For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area. Agreement in RAN1#110b: For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. - Note: CIR and PDP may have different dimensions. - Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples. Agreement in RAN1#111: For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to the first Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Agreement in RAN1#111: Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation. Observation in RAN1#111: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the direct AI/ML positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. Agreement in RAN1#112: For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

According to above agreements.  evaluation results of direct positioning AI/ML are conducted in various scenarios, configurations and hardware imperfections.

PDP based AI/ML positioning

Figure.1 AI model input and output for PDP based AI/ML positioning

As shown in Figure 1, the AI model input is a truncated time delay profile from NTRP(e.g.,18) TRPs. For the cases when the number of paths fed into the AI model (denoted by Nt') is smaller than Nt(e.g.,256), the remaining entries in the input data are set to zero. That is, the input data size keeps the same Nt for all cases even with different number of Nt'. The simulation results are attached in [2] (Excel 1),

- Various values of Nt' are evaluated to investigate the relationship between signaling overhead and model performance

- Different user density of training dataset is constructed to AI/ML positioning accuracy is affected by user density (grid distribution with the width of square grid 1.0 m and 0.5 m)

According to evaluation results for PDP based AI/ML positioning in [2], we have following observations and proposals:

Direct AI/ML positioning has excellent performances even in heavy NLOS conditions (e.g., InF-DH, Clutter parameters {60%, 6m,2m}) when the model input is PDP.

For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation(InF-DH, Clutter parameters {60%, 6m,2m}), the Nt' value should be larger than 32 in order to have sub-meter level positioning accuracy at 90% UEs.

For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}), the Nt' value should be larger than 8 in order to have sub-meter level positioning accuracy at 90% UEs.

With the decrease in Nt' value, positioning performances degrade significantly. In the following exemplary results, the positioning accuracy at 90% UEs is even smaller than the width of square grid in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}):

Nt'=8, 16, 32, 64 when the width of square grid is 1.0 m;

Nt'=8 when the width of square grid is 0.5 m.

AI/ML positioning accuracy is affected by user density/size of the training dataset (InF-DH, Clutter parameters {60%, 6m,2m}). When the training dataset generation follows the grid distribution, the positioning performance is increased in a smaller grid width. Exemplary results are:

The sub-meter level positioning accuracy (i.e., 0.98 m) can be reached at Nt'=16 when the width of square grid is 0.5 m. However, the positioning accuracy is 1.40m in the same settings when the width of square grid is 1.0 m.

CIR based AI/ML positioning

For CIR based AI/ML positioning, evaluation assumptions are similar to PDP based AI/ML positioning except for the input data. Input data size for CIR based positioning is 2×18×256, where the value of the first dimension means the imaginary part and real part of a path respectively. As shown in [2] (Excel 1), with per-path phase information included in the AI model input, the positioning performance is improved obviously in comparison with AI model input without path phase.

According to evaluation results for PDP based AI/ML positioning in [2], we have following observations and proposals:

The positioning performance of CIR based model input is better than PDP based model input, which shows that path phase information is beneficial at least for direct AI/ML positioning. Exemplary results are:

When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of PDP based positioning is about 1.20 times that of CIR based positioning at Nt'=64;

When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of PDP based positioning is about 1.60 times that of CIR base positioning at Nt'=128;

For CIR based AI/ML positioning, if the model is implemented at LMF, gNB/UE should report path power (i.e., RSRPP) and path phase to LMF.

Multi-port PRS based AI/ML positioning

For multi-port AI/ML positioning, evaluation assumptions are similar to PDP based AI/ML positioning except for the following differences:

- When the input data size is 1×18×256, where the first dimension means the model input is PDP from a single port PRS;

- When the input data size is 2×18×256, where the first dimension means the model input is CIR from a single port PRS;

- When the input data size is 4×18×256, where the first dimension means the model input is CIR from a two- port PRS.

Figure. 2 CDFs of positioning errors for multi-port AI/ML positioning

As shown in Figure 2 and [2] (Excel 1), with the same samples for model training, positioning performances increase obviously when enriched channel information from measurement based on multi-port PRS is used for model input.

When the model input includes channel observation from multi-port PRS, better positioning performance can be observed when compared to single port PRS. Exemplary results are:

When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.27 times that of multi-port PRS at Nt'=256 for PDP based positioning;

When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.58 times that of multi-port PRS at Nt'=256 for CIR based positioning;

Due to the enriched channel observations from multi-port PRS, positioning performance is improved under a given number of data samples for model training, which can relief the efforts to collect training data in reality.

Study and identify the benefits of AI/ML based positioning when the model input includes channel observations from multi-port PRS.

When multi-port PRS is evaluated for AI/ML based positioning, companies should report its assumption on the number of ports for a single data sample used in model input.

Model generalization

Agreement in RAN1#110: To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1, …, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1, …, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time. Agreement in RAN1#110: To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: - UE/gNB RX and TX timing error. - The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG). Observation in RAN1#112: Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

According to the above agreement, we evaluate the model generalization and model update (or fine-tuning) for direct AI/ML positioning in various simulation drops, clutter settings, network synchronization errors and UE RX timing errors.

Various simulation drops:

According to the attached Excel ([3], Excel 2), we have the following observations:

For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, direct AI/ML positioning based on CIR cannot generalize its performance in the test dataset.

For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training,

Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same simulation drop as the dataset for model update;

Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update.

For model generalization evaluation in various simulation drops, when training dataset includes mixed data from two simulation drops, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed simulation drops.

Various clutter settings:

According to the attached Excel ([3], Excel 2), we have the following observations:

For model generalization evaluation in various clutter settings, when datasets for training and test are from different clutter settings, direct AI/ML positioning based on CIR cannot achieve cannot generalize its performance in the test dataset.

For model generalization evaluation in various clutter settings, when model update is performed based on dataset in a clutter setting that is different from the clutter setting used for model training,

Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same clutter setting as the dataset for model update;

Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update.

For model generalization evaluation in various clutter settings, when a dataset for training includes data from mixed clutter settings, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed clutter settings.

Various network synchronization errors:

According to the attached Excel ([3], Excel 2), we have the following observations:

For model generalization evaluation in various network synchronization errors, when a dataset for training has ideal network synchronization errors and a dataset for test has network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), the positioning performance degrades seriously compared to ideal network synchronization.

For model generalization evaluation in various network synchronization errors, when both training dataset and test dataset have network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to ideal network synchronization. This may be explained by:

CNN model is translation-invariant to the time shift of channel information due to network synchronization error;

Add random network synchronization errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various network synchronization errors.

Various UE RX timing errors:

According to the attached Excel ([3], Excel 2), we have the following observations:

For model generalization evaluation in various UE Rx timing errors, the positioning performance in a test dataset with UE Rx timing errors degrades slightly compared to the dateset without UE Rx timing errors.

For model generalization evaluation in various UE Rx timing errors, when both training dataset and test dataset have UE Rx timing errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to the case without UE Rx timing error. This may be explained by:

CNN model is translation-invariant to the time shift of channel information due to UE Rx timing error;

Add random UE Rx timing errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various UE Rx timing errors.

Semi-supervised learning

Agreement in RAN1#110: For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

As agreed in RAN1#110, in some cases, the availability of ground-truth labels is not always possible. In reality, it is expensive and time-consuming for manual labeling. Therefore, unsupervised/self-supervised learning has been studied, which has some progress in NLP (natural language processing) domain and computer vision domain. Generally, unsupervised/self-supervised learning doesn’t rely on ground-truth labels, it trains a model based on some proxy tasks to use input data itself as supervision and benefits almost all types of downstream tasks [10]. In our evaluation, semi-supervised learning for direct AI/ML positioning is conducted, which uses a large number of unlabeled samples to pretrain a model and the model is further fine-tuned by a small number of labeled samples. To be detailed, we use MoCo (Momentum Contrast) method, proposed by Kaiming He [10], to learn important features from CIR by contrastive learning. Then, the model trained by self-supervised learning can further be used as the pretraining to predict UE locations via a small number of labeled samples.

According to the evaluation results in [4] (Excel 3), we have following observations:

Semi-supervised learning (i.e., using a large number of unlabeled samples to pretrain a model and the model is further fine-tuned by a small number of labeled samples) outperforms supervised learning (e.g., training from scratch) under the same am of labeled samples for model training. Exemplary results are:

The positioning error of supervised learning based on 2K labeled samples is about 2.29 times that of semi-supervised learning based on 100K unlabeled samples and 2K labeled samples (i.e., 98% data samples are unlabeled);

The positioning error of supervised learning based on 4K labeled samples is about 1.96 times that of semi-supervised learning based on 100K unlabeled samples and 4K labeled samples (i.e., 96% data samples are unlabeled);

The positioning error of supervised learning based on 8K labeled samples is about 1.37 times that of semi-supervised learning based on 100K unlabeled samples and 8K labeled samples (i.e., 93% data samples are unlabeled);

As the increase of labeled samples used for model training, semi-supervised learning has a similar performance as supervised learning. Exemplary results are:

The positioning error of supervised learning based on 16K labeled samples is similar as that of semi-supervised learning based on 100K unlabeled samples and 16K labeled samples (i.e., 86% data samples are unlabeled).

Study and identify the benefits of semi-supervised learning for AI/ML based positioning.

AI/ML assisted positioning

AI/ML assisted RSTD estimation

Agreement in RAN1#110b: For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.) Observation in RAN1#111: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {40%, 2m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <0.4m at CDF=90%, as compared to >9m for conventional positioning method. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method.

As an example, shown in Figure 3, AI/ML assisted positioning generally has two stages to get UE positions. The first stage is to estimate intermediate results (e.g., DL RSTD values) by using AI/ML network. In our simulation, the AI model outputs are 18 DL RSTD values that are relative timing differences to a reference TRP. Finally, the estimated DL RSTD values are used by a non-AI based model (a classical algorithm, e.g., Chan’s algorithm) to get a 2-D UE position.

Figure.3 AI/ML assisted RSTD estimation

According to the evaluation results attached in Excel 1([2]), we have the following observations and proposals:

The AI/ML assisted positioning has an excellent accuracy on the estimation of DL PRS RSTD values even in heavy NLOS conditions (e.g., InF-DH {60%, 6m, 2m}). Exemplary results are:

When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of CIR based positioning is about 0.28 m at Nt'=256;

AI/ML assisted positioning achieves similar positioning performance as direct AI/ML positioning when the model output is DL RSTD.

In order to collect evaluation results for AI/ML assisted positioning, the table for evaluation result collection should add one more column to show the intermediate results.

Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes DL PRS RSTD values.

AI/ML assisted LOS/NLOS identification

In order to investigate the effectiveness of an AI model being used for LOS/NLOS identification, InF-DH channel should at least have some moderate LOS conditions, e.g., the clutter settings can be {40%, 2m, 2m} corresponding to 43.97% LOS probability. In addition, AI/ML assisted LOS/NLOS identification doesn’t rely on spatial consistency among data samples, there is not necessary to generate training dataset within a square grid.

Figure.4 AI model input and output for AI/ML assisted LOS/NLOS identification

According to the evaluation results attached in Excel 1([2]), we have the following observation and proposal:

The AI/ML assisted positioning method can achieve a good accuracy rate in LOS/NLOS identification. Exemplary results are:

When the training samples are 60K (InF-DH, Clutter parameters: {40%, 2m, 2m}), Los/NLos accuracy rate by using CIR from single TRP as model input is about 94.7%.

Study and evaluate the performance of AI/ML assisted positioning where the model output includes confidence level of LOS/NLOS identification.

Model generalization

AI/ML assisted RSTD estimation

According to the evaluation results attached in Excel 2([3]), we have the following observations:

For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, AI/ML assisted positioning using DL RSTD value as model output cannot generalize its performance in the test dataset.

For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training,

Positioning performance of AI/ML assisted positioning using DL RSTD value as model output can be improved in a test dataset that is in the same simulation drop as the dataset for model update;

Positioning performance of AI/ML assisted positioning using DL RSTD value as model output is increased along with the number of data samples used for model update.

AI/ML assisted LOS/NLOS identification

According to the evaluation results attached in Excel 2([3]), we have the following observations and proposals:

For model generalization evaluation on AI/ML assisted LOS/NLOS identification,

The model performance can generalize well to a test dataset that is in a different simulation drop of training dataset but in the same InF-DH scenario (clutter parameters {40%, 2m, 2m});

The model performance degrades slightly when the training data is in InF-DH scenario (clutter parameters {40%, 2m, 2m}) and test data is in InF-SH scenario (clutter parameters {30%, 2m, 2m}).

To evaluate model performance on AI/ML assisted LOS/NLOS identification, the evaluation assumptions for dataset generation should have a moderate LOS probability (e.g., Clutter parameters {40%, 2m, 2m} for InF-DH scenario and clutter parameters {30%, 2m, 2m} for InF-SH scenario)

For model generalization evaluation on AI/ML assisted LOS/NLOS identification, the model performance should be tested in an InF scenario that is different from the InF scenario for training dataset generation.

Conclusion

In this contribution, we provide our further views on evaluation assumptions and evaluation results on AI/ML for positioning. We have following observations and proposals:

Observation 1: Direct AI/ML positioning has excellent performances even in heavy NLOS conditions (e.g., InF-DH, Clutter parameters {60%, 6m,2m}) when the model input is PDP.

Observation 2: For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation(InF-DH, Clutter parameters {60%, 6m,2m}), the Nt' value should be larger than 32 in order to have sub-meter level positioning accuracy at 90% UEs.

Observation 3: For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}), the Nt' value should be larger than 8 in order to have sub-meter level positioning accuracy at 90% UEs.

Observation 4: With the decrease in Nt' value, positioning performances degrade significantly. In the following exemplary results, the positioning accuracy at 90% UEs is even smaller than the width of square grid in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}):

• Nt'=8, 16, 32, 64 when the width of square grid is 1.0 m;

• Nt'=8 when the width of square grid is 0.5 m.

Observation 5: AI/ML positioning accuracy is affected by user density/size of the training dataset (InF-DH, Clutter parameters {60%, 6m,2m}). When the training dataset generation follows the grid distribution, the positioning performance is increased in a smaller grid width. Exemplary results are:

• The sub-meter level positioning accuracy (i.e., 0.98 m) can be reached at Nt'=16 when the width of square grid is 0.5 m. However, the positioning accuracy is 1.40m in the same settings when the width of square grid is 1.0 m.

Observation 6: The positioning performance of CIR based model input is better than PDP based model input, which shows that path phase information is beneficial at least for direct AI/ML positioning. Exemplary results are:

• When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of PDP based positioning is about 1.20 times that of CIR based positioning at Nt'=64;

• When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of PDP based positioning is about 1.60 times that of CIR base positioning at Nt'=128;

Observation 7: When the model input includes channel observation from multi-port PRS, better positioning performance can be observed when compared to single port PRS. Exemplary results are:

• When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.27 times that of multi-port PRS at Nt'=256 for PDP based positioning;

• When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.58 times that of multi-port PRS at Nt'=256 for CIR based positioning;

Observation 8: Due to the enriched channel observations from multi-port PRS, positioning performance is improved under a given number of data samples for model training, which can relief the efforts to collect training data in reality.

Observation 9: For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, direct AI/ML positioning based on CIR cannot generalize its performance in the test dataset.

Observation 10: For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training,

• Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same simulation drop as the dataset for model update;

• Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update.

Observation 11: For model generalization evaluation in various simulation drops, when training dataset includes mixed data from two simulation drops, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed simulation drops.

Observation 12: For model generalization evaluation in various clutter settings, when datasets for training and test are from different clutter settings, direct AI/ML positioning based on CIR cannot achieve cannot generalize its performance in the test dataset.

Observation 13: For model generalization evaluation in various clutter settings, when model update is performed based on dataset in a clutter setting that is different from the clutter setting used for model training,

• Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same clutter setting as the dataset for model update;

• Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update.

Observation 14: For model generalization evaluation in various clutter settings, when a dataset for training includes data from mixed clutter settings, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed clutter settings.

Observation 15: For model generalization evaluation in various network synchronization errors, when a dataset for training has ideal network synchronization errors and a dataset for test has network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), the positioning performance degrades seriously compared to ideal network synchronization.

Observation 16: For model generalization evaluation in various network synchronization errors, when both training dataset and test dataset have network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to ideal network synchronization. This may be explained by:

• CNN model is translation-invariant to the time shift of channel information due to network synchronization error;

• Add random network synchronization errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various network synchronization errors.

Observation 17: For model generalization evaluation in various UE Rx timing errors, the positioning performance in a test dataset with UE Rx timing errors degrades slightly compared to the dateset without UE Rx timing errors.

Observation 18: For model generalization evaluation in various UE Rx timing errors, when both training dataset and test dataset have UE Rx timing errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to the case without UE Rx timing error. This may be explained by:

• CNN model is translation-invariant to the time shift of channel information due to UE Rx timing error;

• Add random UE Rx timing errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various UE Rx timing errors.

Observation 19: Semi-supervised learning (i.e., using a large number of unlabeled samples to pretrain a model and the model is further fine-tuned by a small number of labeled samples) outperforms supervised learning (e.g., training from scratch) under the same am of labeled samples for model training. Exemplary results are:

• The positioning error of supervised learning based on 2K labeled samples is about 2.29 times that of semi-supervised learning based on 100K unlabeled samples and 2K labeled samples (i.e., 98% data samples are unlabeled);

• The positioning error of supervised learning based on 4K labeled samples is about 1.96 times that of semi-supervised learning based on 100K unlabeled samples and 4K labeled samples (i.e., 96% data samples are unlabeled);

• The positioning error of supervised learning based on 8K labeled samples is about 1.37 times that of semi-supervised learning based on 100K unlabeled samples and 8K labeled samples (i.e., 93% data samples are unlabeled);

Observation 20: As the increase of labeled samples used for model training, semi-supervised learning has a similar performance as supervised learning. Exemplary results are:

• The positioning error of supervised learning based on 16K labeled samples is similar as that of semi-supervised learning based on 100K unlabeled samples and 16K labeled samples (i.e., 86% data samples are unlabeled).

Observation 21: The AI/ML assisted positioning has an excellent accuracy on the estimation of DL PRS RSTD values even in heavy NLOS conditions (e.g., InF-DH {60%, 6m, 2m}). Exemplary results are:

• When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of CIR based positioning is about 0.28 m at Nt'=256;

Observation 22: AI/ML assisted positioning achieves similar positioning performance as direct AI/ML positioning when the model output is DL RSTD.

Observation 23: The AI/ML assisted positioning method can achieve a good accuracy rate in LOS/NLOS identification. Exemplary results are:

• When the training samples are 60K (InF-DH, Clutter parameters: {40%, 2m, 2m}), Los/NLos accuracy rate by using CIR from single TRP as model input is about 94.7%.

Observation 24: For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, AI/ML assisted positioning using DL RSTD value as model output cannot generalize its performance in the test dataset.

Observation 25: For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training,

• Positioning performance of AI/ML assisted positioning using DL RSTD value as model output can be improved in a test dataset that is in the same simulation drop as the dataset for model update;

• Positioning performance of AI/ML assisted positioning using DL RSTD value as model output is increased along with the number of data samples used for model update.

Observation 26: For model generalization evaluation on AI/ML assisted LOS/NLOS identification,

• The model performance can generalize well to a test dataset that is in a different simulation drop of training dataset but in the same InF-DH scenario (clutter parameters {40%, 2m, 2m});

• The model performance degrades slightly when the training data is in InF-DH scenario (clutter parameters {40%, 2m, 2m}) and test data is in InF-SH scenario (clutter parameters {30%, 2m, 2m}).

Proposal 1: For CIR based AI/ML positioning, if the model is implemented at LMF, gNB/UE should report path power (i.e., RSRPP) and path phase to LMF.

Proposal 2: Study and identify the benefits of AI/ML based positioning when the model input includes channel observations from multi-port PRS.

Proposal 3: When multi-port PRS is evaluated for AI/ML based positioning, companies should report its assumption on the number of ports for a single data sample used in model input.

Proposal 4: Study and identify the benefits of semi-supervised learning for AI/ML based positioning.

Proposal 5: In order to collect evaluation results for AI/ML assisted positioning, the table for evaluation result collection should add one more column to show the intermediate results.

Proposal 6: Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes DL PRS RSTD values.

Proposal 7: Study and evaluate the performance of AI/ML assisted positioning where the model output includes confidence level of LOS/NLOS identification.

Proposal 8: To evaluate model performance on AI/ML assisted LOS/NLOS identification, the evaluation assumptions for dataset generation should have a moderate LOS probability (e.g., Clutter parameters {40%, 2m, 2m} for InF-DH scenario and clutter parameters {30%, 2m, 2m} for InF-SH scenario)

Proposal 9: For model generalization evaluation on AI/ML assisted LOS/NLOS identification, the model performance should be tested in an InF scenario that is different from the InF scenario for training dataset generation.

References

RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

Excel 1: Pos-Supervised-NoLabelErr-Train-Test-Same-Setting

Excel 2: Pos-Supervised-NoLabelErr-Train-Test-Different-Setting

Excel 3: Pos-Semisupervised-Train-Test-Same-Setting

Draft Report of 3GPP TSG RAN WG1 #110bis-e

Draft Report of 3GPP TSG RAN WG1 #111

Draft Report of 3GPP TSG RAN WG1 #112

Draft Report of 3GPP TSG RAN WG1 #112bis-e

Identity Mappings in Deep Residual Networks, Kaiming He, Xiangyu Zhang, Shaoqing Ren, and Jian Sun, Microsoft Research

Self-supervised Learning: Generative or Contrastive, Xiao Liu, Fanjin Zhang, Zhenyu Hou, Li Mian, Zhaoyu Wang, Jing Zhang, Jie Tang, IEEE Fellow

Momentum Contrast for Unsupervised Visual Representation Learning, Kaiming H, Haoqi Fan, Yuxin Wu, Saining Xie, Ross Girshick, Facebook AI Research (FAIR)

- Appendix A Common scenario parameters applicable for all scenarios

Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: 4-panels UE: - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

- Appendix B Parameters common to InF scenarios

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-DH |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - the convex hull of the horizontal BS deployment. - the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. FFS: which of the above should be baseline. FFS: if an optional evaluation area is needed |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) FFS: if the optional UE antenna height is needed |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4, hc), 8}. FFS: if the optional gNB antenna height is needed |  |  |
| Clutter parameters: {density r, height hc,size dclutter} |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |

- Appendix C Evaluation assumptions on dataset generation

| Frequency range | FR1 |
| --- | --- |
| Bandwidth | 100 MHz |
| Sub-carrier spacing | 30 KHz |
| InF channel | InF-DH |
| Clutter setting | {60%, 6m, 2m} (heavy NLOS conditions) or {40%, 4m, 2m} (moderate NLOS conditions) |
| Spatial consistency modeling | Large scale parameters, small scale parameters and absolute time of arrival, where - the large-scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = dclutter/2 for InF (Section 7.6.3.1 of TR 38.901) - the small-scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 |
| UE distribution for training dataset | Grid distribution, i.e., one training data is collected at the center of one small square grid, the width of the square grid is 0.5/1.0 m. |