3GPP TSG RAN WG1 #113	R1-2304595

Incheon, Korea, May 22nd – May 26th, 2023

Title: 	SBFD Prototype and Preliminary Simulation Results

Source: 	ZTE

Agenda item:	9.3.1

Document for:	Discussion/Decision

Introduction

In RAN#95 meeting, a revised SID on NR duplex evolution has been endorsed [1]. In this contribution, SBFD prototype info and preliminary simulation results are provided. Compared with RAN1#112bis-e meeting, more simulation results are provided and some simulation results are updated.

SBFD prototype

Two SBFD BS prototypes are researched and verified by ZTE and CMCC collaboratively in CMCC’s 5G-A Innovation Lab.

The 1st SBFD prototype is researched to verify the feasibility and evaluate the actual performance of SBFD operation with test UE (TUE). The subband configuration and other the detailed setting about this SBFD prototype is shown in Table below. The first slot and last slot are DL only slot and UL only slot with 100MHz transmission bandwidth, respectively. The DL subband is set as 10MHz transmission bandwidth and the UL subband is set as 90MHz reception bandwidth. The TUE instead of the commercial UE is used for communication with the SBFD base station prototype.

| Item | Setting |
| --- | --- |
| Frequency | 4.9GHz |
| BS antenna configuration | 4T4R |
| UE antenna configuration | 4T4R |
| Power | 4*250mw, 30dBm in total |
| SBFD configuration | DXXXU non-SBFD slot: 100MHz DL subband: 10MHz UL subband: 90MHz |
| Antenna isolation | 21.5cm |
| Self-interference capability | more than 130dB |

Some experiments were carried out to verify the self-interference suppression capability and receiver sensitivity degradation in the SBFD base station. The self-interference capability is more than 130 dB, which includes 55 dB antenna isolation, 45 dB Adjacent Channel Leakage Ratio (ACLR or ACPR) and more than 30 dB sub-band filtering and digital cancellation. The measurement results of receiver sensitivity degradation are captured in the figure 1 below, compared with the UL receiver without DL interference, the receiver sensitivity in SBFD base station is decreased by 0.8dB, 0.7dB and 1dB for QPSK, 64QAM and 256QAM, respectively.

Observation 1: The 1st SBFD prototype achieves more than 130dB self-interference suppression capability (55 dB antenna isolation, 45 dB ACLR and more than 30 dB sub-band filtering and digital cancellation) and less than 1dB receiver sensitivity degradation at the gNB side.

Figure 1: Receiver sensitivity degradation of SBFD gNB

The real-time end-to-end (E2E) round trip latency and the peak data rate obtained from the TUEs using the developed SBFD prototype are verified. The E2E latency is always below 4 ms, with an average value of 3.9 ms. This is mainly achieved by changing the frame structure dynamically for SBFD slots to accommodate the variation of arrival time for each packet. The UL peak data rate is higher than 1.4 Gbps. This is not only due to more UL resources allocated and the advanced interference management, but also some scheduling enhancement by implementation. For instance, the DL sub-band in a SBFD slot can schedule an UL transmission in an UL sub-band in another SBFD slot. Overall, the results of the SBFD prototype not only prove the feasibility of the SBFD for BS but also outline the reliable performance it can offer [2].

The 2nd SBFD prototype is researched to verify the compatibility with legacy commercial UEs and also test the performance. The commercial 5G UEs used in our tests are ZTE Axon 20 and Axon 30 with 2T4R. The same SBFD pattern is applied by configuring flexible TDD slot formats for two commercial UEs, i.e., DXXXU. This requires commercial UEs to support configuring with and transmit/receive in flexible symbols. During the test, the first UE and second UE are configured with 10MHz bandwidth and 90MHz bandwidth, respectively. The commercial UEs can attach to this base station and perform UL transmission successfully. The peak UL data rate is higher than 700Mbps per UE and the achieved E2E latency is around 4ms.

| Prototype | Figures | Performance |
| --- | --- | --- |
| The 1st SBFD prototype with TUE (4T4R) |  | Peak UL data rate: 1.4Gbps E2E latency: less than 4ms |
| The 2nd SBFD prototype with commercial UE (2T4R) |  | Peak UL data rate: 700Mbps E2E latency: around 4ms |

Observation 2:

- The first prototype verifies the SBFD feasibility and achieves 3.9ms E2E round trip latency on average and up to 1.4Gbps peak UL data rate with 4T4R TUE.

- The second prototype verifies that legacy commercial UEs supporting flexible symbols are compatible to the SBFD base station. The peak UL data rate is higher than 700Mbps and the E2E round trip latency is around 4ms for commercial UE with 2T4R.

Proposal 1: Capture the SBFD prototype info in section 2 of R1-2304595 into TR38.858.

Preliminary SLS results for SBFD

In this section, the preliminary simulation results and some observations for SBFD based on the simulation results are captured.

Simulation assumptions

Some key simulation assumptions are listed here and the detailed simulation assumptions can be found in the Appendix.

- In our simulation, the self-interference suppression capability is determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. The self-interference suppression capability can be achieved by antenna separation (50-60dB), ACLR (45dB), subband filter (10-20dB) and digital cancellation (10-20dB).

Simulation results: FR1 Indoor office

Packet size 0.5Mbytes/0.125Mbytes

SBFD Alt.2 subband pattern

| Tdoc/Source | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean | 459.83 | 349.44 | -24.01% | 410.65 | 301.54 | -26.57% | 368.92 | 252.89 | -31.45% |
|  |  | 5% | 166.69 | 123.72 | -25.78% | 127.78 | 89.82 | -29.71% | 64.03 | 10.82 | -83.10% |
|  |  | 50% | 563.97 | 420.84 | -25.38% | 487.60 | 349.48 | -28.33% | 421.94 | 269.54 | -36.12% |
|  |  | 95% | 641.83 | 488.23 | -23.93% | 615.00 | 471.62 | -23.31% | 584.82 | 434.57 | -25.69% |
|  | DL Tail-UPT CDF (Mbps) | Mean | 381.36 | 284.68 | -25.35% | 188.61 | 140.54 | -25.49% | 131.60 | 65.10 | -50.53% |
|  |  | 5% | 114.93 | 84.18 | -26.76% | 30.99 | 19.40 | -37.40% | 9.78 | 4.16 | -57.46% |
|  |  | 50% | 344.09 | 254.84 | -25.94% | 126.63 | 77.54 | -38.77% | 112.28 | 34.41 | -69.35% |
|  |  | 95% | 630.99 | 485.90 | -22.99% | 219.39 | 171.65 | -21.76% | 323.23 | 134.70 | -58.33% |
|  | DL Median-UPT CDF (Mbps) | Mean | 440.27 | 338.48 | -23.12% | 411.85 | 303.61 | -26.28% | 365.12 | 248.38 | -31.97% |
|  |  | 5% | 126.91 | 100.31 | -20.96% | 101.22 | 60.95 | -39.78% | 36.81 | 6.13 | -83.35% |
|  |  | 50% | 572.89 | 448.90 | -21.64% | 429.94 | 307.27 | -28.53% | 365.42 | 244.27 | -33.15% |
|  |  | 95% | 641.83 | 490.15 | -23.63% | 627.45 | 483.80 | -22.89% | 613.70 | 475.58 | -22.51% |
|  | UL Average-UPT CDF (Mbps) | Mean | 143.85 | 191.48 | 33.11% | 130.01 | 182.97 | 40.74% | 114.77 | 170.18 | 48.28% |
|  |  | 5% | 98.69 | 104.51 | 5.90% | 100.14 | 120.81 | 20.64% | 51.43 | 91.93 | 78.75% |
|  |  | 50% | 148.95 | 155.66 | 4.50% | 132.75 | 144.84 | 9.11% | 119.33 | 139.83 | 17.18% |
|  |  | 95% | 161.37 | 283.36 | 75.60% | 148.74 | 273.71 | 84.02% | 143.81 | 263.58 | 83.28% |
|  | UL Tail-UPT CDF (Mbps) | Mean | 110.66 | 149.41 | 35.02% | 50.49 | 91.60 | 81.42% | 37.22 | 67.21 | 80.57% |
|  |  | 5% | 37.23 | 33.51 | -9.99% | 15.71 | 29.03 | 84.79% | 3.56 | 13.53 | 280.06% |
|  |  | 50% | 126.98 | 133.65 | 5.25% | 53.28 | 77.46 | 45.38% | 31.34 | 57.55 | 83.63% |
|  |  | 95% | 145.45 | 258.06 | 77.42% | 82.96 | 160.46 | 93.42% | 78.76 | 138.96 | 76.43% |
|  | UL Median-UPT CDF (Mbps) | Mean | 136.16 | 183.36 | 34.67% | 132.83 | 187.91 | 41.47% | 116.87 | 177.47 | 51.85% |
|  |  | 5% | 35.87 | 30.42 | -15.19% | 91.35 | 126.98 | 39.00% | 34.98 | 78.98 | 125.79% |
|  |  | 50% | 142.49 | 151.76 | 6.51% | 136.92 | 145.45 | 6.23% | 131.76 | 140.35 | 6.52% |
|  |  | 95% | 159.54 | 278.61 | 74.63% | 150.13 | 278.61 | 85.58% | 144.70 | 270.53 | 86.96% |
|  | DL Packet-Latency CDF (ms) | Mean | 12.08 | 16.06 | 32.95% | 16.32 | 25.5 | 56.25% | 32.67 | 55.27 | 69.18% |
|  |  | 5% | 6.02 | 8.02 | 33.22% | 6.02 | 8.04 | 33.55% | 6.05 | 8.09 | 33.72% |
|  |  | 50% | 6.88 | 8.91 | 29.51% | 10.27 | 15.02 | 46.25% | 12.41 | 17.48 | 40.85% |
|  |  | 95% | 27.84 | 38.88 | 39.66% | 45.13 | 75.23 | 66.70% | 112.05 | 205.23 | 83.16% |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean | 9.44 | 7.93 | -16.00% | 9.82 | 6.75 | -31.26% | 18.85 | 8.63 | -54.22% |
|  |  | 5% | 5.59 | 3.27 | -41.50% | 5.63 | 3.38 | -39.96% | 5.63 | 3.41 | -39.43% |
|  |  | 50% | 6.77 | 5.95 | -12.11% | 7.20 | 6.23 | -13.47% | 7.63 | 6.55 | -14.15% |
|  |  | 95% | 11.88 | 10.80 | -9.09% | 19.80 | 12.70 | -35.86% | 49.09 | 21.48 | -56.24% |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL | 6.01% | 5.79% | -3.66% | 20.51% | 19.96% | -2.68% | 33.82% | 31.73% | -6.18% |
|  |  | UL | 2.39% | 2.45% | 2.51% | 6.23% | 6.07% | -2.57% | 10.32% | 10.41% | 0.87% |
|  | Type-2 RU (%) | DL | 7.52% | 9.53% | 26.73% | 25.63% | 32.87% | 28.25% | 42.27% | 52.25% | 23.61% |
|  |  | UL | 11.93% | 6.81% | -42.92% | 31.16% | 16.82% | -46.02% | 51.61% | 28.87% | -44.06% |
|  | Unfinished/dropped Packet Rate (%) | DL | 0.45% | 0.45% |  | 0.41% | 0.34% |  | 0.81% | 5.72% |  |
|  |  | UL | 0.11% | 0.66% |  | 0.26% | 0.20% |  | 0.56% | 0.16% |  |
|  | Additional comments: e.g., |  |  |  |  |  |  |  |  |  |  |

Compared with the baseline TDD pattern, the UL resource of the SBFD subband pattern Alt.2 has been increased by around 80% and the DL resource has been decreased by around 20%. Overall, we have the following observation.

Observation 3: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 24% - 31% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 32%-69% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the larger DL Packet-Latency (mean) of SBFD due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 33% - 48% due to the increased UL resource. The gain of UL average UPT (mean) of SBFD is increased as the increase of traffic load because the UL average UPT (mean) of baseline TDD is decreased as the increase of traffic load due to the limited UL resource in the baseline TDD.

- The UL Packet-Latency (mean) of SBFD is decreased by around 16% - 54% due to the increased UL resource. The gain of UL Packet-Latency (mean) of SBFD is increased as the increase of traffic load because the UL Packet-Latency (mean) of baseline TDD is increased as the increase of traffic load due to the limited UL resource in the baseline TDD.

SBFD Alt.4 subband pattern

| Tdoc/Source | Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean | 459.83 | 438.63 | -4.61% | 410.65 | 386.33 | -5.92% | 368.92 | 328.67 | -10.91% |
|  |  | 5% | 166.69 | 168.74 | 1.23% | 127.78 | 123.01 | -3.73% | 64.03 | 34.89 | -45.51% |
|  |  | 50% | 563.97 | 527.81 | -6.41% | 487.6 | 452.32 | -7.24% | 421.94 | 379.14 | -10.14% |
|  |  | 95% | 641.83 | 608.52 | -5.19% | 615 | 577.61 | -6.08% | 584.82 | 531.36 | -9.14% |
|  | DL Tail-UPT CDF (Mbps) | Mean | 381.36 | 360.34 | -5.51% | 188.61 | 190.00 | 0.74% | 131.60 | 93.18 | -29.19% |
|  |  | 5% | 114.93 | 107.95 | -6.07% | 30.99 | 29.61 | -4.45% | 9.78 | 5.32 | -45.60% |
|  |  | 50% | 344.09 | 319.54 | -7.13% | 126.63 | 143.31 | 13.17% | 112.28 | 79.18 | -29.48% |
|  |  | 95% | 630.99 | 600.54 | -4.83% | 219.39 | 212.73 | -3.04% | 323.23 | 229.74 | -28.92% |
|  | DL Median-UPT CDF (Mbps) | Mean | 440.27 | 429.14 | -2.53% | 411.85 | 389.78 | -5.36% | 365.12 | 324.1 | -11.23% |
|  |  | 5% | 126.91 | 124.93 | -1.56% | 101.22 | 96.51 | -4.65% | 36.81 | 10.44 | -71.64% |
|  |  | 50% | 572.89 | 578.81 | 1.03% | 429.94 | 418.69 | -2.62% | 365.42 | 320.46 | -12.30% |
|  |  | 95% | 641.83 | 607.05 | -5.42% | 627.45 | 597.33 | -4.80% | 613.7 | 587.93 | -4.20% |
|  | UL Average-UPT CDF (Mbps) | Mean | 143.85 | 163.28 | 13.51% | 130.01 | 149.45 | 14.95% | 114.77 | 125.33 | 9.20% |
|  |  | 5% | 98.69 | 94.75 | -3.99% | 100.14 | 114.6 | 14.44% | 51.43 | 44.52 | -13.44% |
|  |  | 50% | 148.95 | 173.63 | 16.57% | 132.75 | 152.41 | 14.81% | 119.33 | 133.03 | 11.48% |
|  |  | 95% | 161.37 | 178.27 | 10.47% | 148.74 | 171.33 | 15.19% | 143.81 | 163.08 | 13.40% |
|  | UL Tail-UPT CDF (Mbps) | Mean | 110.66 | 130.21 | 17.67% | 50.49 | 52.01 | 3.01% | 37.22 | 31.25 | -16.04% |
|  |  | 5% | 21.23 | 18.15 | -14.51% | 15.71 | 11.18 | -28.84% | 3.56 | 1.34 | -62.36% |
|  |  | 50% | 126.98 | 169.18 | 33.23% | 53.28 | 47.10 | -11.60% | 31.34 | 27.46 | -12.38% |
|  |  | 95% | 145.45 | 173.37 | 19.20% | 82.96 | 95.73 | 15.39% | 78.76 | 67.07 | -14.84% |
|  | UL Median-UPT CDF (Mbps) | Mean | 136.16 | 161.97 | 18.96% | 132.83 | 164.4 | 23.77% | 116.87 | 135.43 | 15.88% |
|  |  | 5% | 35.87 | 36.87 | 2.79% | 91.35 | 106.67 | 16.77% | 34.98 | 8.52 | -75.64% |
|  |  | 50% | 142.49 | 173.37 | 21.67% | 136.92 | 172.31 | 25.85% | 131.76 | 169.18 | 28.40% |
|  |  | 95% | 159.54 | 177.78 | 11.43% | 150.13 | 175.55 | 16.93% | 144.7 | 174.45 | 20.56% |
|  | DL Packet-Latency CDF (ms) | Mean | 12.08 | 12.63 | 4.55% | 16.32 | 17.69 | 8.39% | 32.67 | 50.38 | 54.21% |
|  |  | 5% | 6.02 | 6.48 | 7.64% | 6.02 | 6.48 | 7.64% | 6.05 | 6.52 | 7.77% |
|  |  | 50% | 6.88 | 6.91 | 0.44% | 10.27 | 11.52 | 12.17% | 12.41 | 13.63 | 9.83% |
|  |  | 95% | 27.84 | 30.16 | 8.33% | 45.13 | 48.63 | 7.76% | 112.05 | 228.84 | 104.23% |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean | 9.44 | 7.85 | -16.84% | 9.82 | 8.73 | -11.10% | 18.85 | 27.05 | 43.50% |
|  |  | 5% | 5.59 | 5.45 | -2.50% | 5.63 | 5.48 | -2.66% | 5.63 | 5.48 | -2.66% |
|  |  | 50% | 6.77 | 5.73 | -15.36% | 7.2 | 5.8 | -19.44% | 7.63 | 5.91 | -22.54% |
|  |  | 95% | 11.88 | 14.41 | 21.30% | 19.8 | 20.66 | 4.34% | 49.09 | 121.34 | 147.18% |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL | 6.01% | 5.76% | -4.16% | 20.51% | 19.65% | -4.19% | 33.82% | 32.14% | -4.97% |
|  |  | UL | 2.39% | 2.23% | -6.69% | 6.23% | 6.01% | -3.53% | 10.32% | 10.42% | 0.97% |
|  | Type-2 RU (%) | DL | 7.52% | 7.59% | 0.93% | 25.63% | 25.88% | 0.98% | 42.27% | 42.33% | 0.14% |
|  |  | UL | 11.93% | 11.09% | -7.04% | 31.16% | 29.96% | -3.85% | 51.61% | 51.90% | 0.56% |
|  | Unfinished/dropped Packet Rate (%) | DL | 0.45% | 0.45% |  | 0.41% | 0.34% |  | 0.81% | 1.71% |  |
|  |  | UL | 0.11% | 0.33% |  | 0.26% | 0.13% |  | 0.56% | 0.78% |  |
|  | Additional comments: e.g., |  |  |  |  |  |  |  |  |  |  |

For SBFD subband pattern Alt.4, its UL resource ratio and DL resource ratio are almost the same as baseline TDD. Overall, we have the following observation.

Observation 4: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.4 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 4% - 10% due to the UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 4%-8% in case of low and medium traffic load due to UE-UE CLI. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 54% due to the much more serious UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 9% - 15% due to more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 11% - 17% in case of low and medium traffic load. In case of high traffic load, the UL Packet-Latency (mean) of SBFD is increased by around 45% due to the much more serious gNB CLI.

Packet size 5Kbytes/1Kbytes

SBFD Alt.2 subband pattern

| Tdoc/Source | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean | 43.62 | 43.9 | 0.64% | 42.87 | 43.02 | 0.35% | 40.8 | 39.88 | -2.25% |
|  |  | 5% | 42.29 | 42.43 | 0.33% | 40.54 | 40.36 | -0.44% | 36.72 | 28.02 | -23.69% |
|  |  | 50% | 43.75 | 44.04 | 0.66% | 43.05 | 43.33 | 0.65% | 41.36 | 41.55 | 0.46% |
|  |  | 95% | 44.83 | 45.01 | 0.40% | 43.96 | 44.18 | 0.50% | 42.34 | 42.65 | 0.73% |
|  | DL Tail-UPT CDF (Mbps) | Mean | 21.95 | 22.92 | 4.42% | 21.2 | 21.91 | 3.35% | 19.38 | 18.3 | -5.57% |
|  |  | 5% | 12.53 | 12.36 | -1.36% | 11.56 | 11.41 | -1.30% | 9.48 | 5.26 | -44.51% |
|  |  | 50% | 22.68 | 23.89 | 5.34% | 22.12 | 23.27 | 5.20% | 21.59 | 21.08 | -2.36% |
|  |  | 95% | 23.27 | 24.55 | 5.50% | 22.68 | 23.89 | 5.34% | 22.12 | 22.68 | 2.53% |
|  | DL Median-UPT CDF (Mbps) | Mean | 42.18 | 42.01 | -0.40% | 41.5 | 41.33 | -0.41% | 39.4 | 38.1 | -3.30% |
|  |  | 5% | 41.67 | 39.82 | -4.44% | 39.82 | 39.82 | 0.00% | 36.57 | 24.55 | -32.87% |
|  |  | 50% | 41.67 | 41.67 | 0.00% | 41.67 | 41.67 | 0.00% | 39.82 | 39.82 | 0.00% |
|  |  | 95% | 43.71 | 43.71 | 0.00% | 41.67 | 41.67 | 0.00% | 41.67 | 41.67 | 0.00% |
|  | UL Average-UPT CDF (Mbps) | Mean | 5.76 | 8.25 | 43.23% | 5.36 | 7.88 | 47.01% | 4.8 | 7.31 | 52.29% |
|  |  | 5% | 5.38 | 5.4 | 0.37% | 5.08 | 5.1 | 0.39% | 4.5 | 4.57 | 1.56% |
|  |  | 50% | 5.75 | 6.03 | 4.87% | 5.35 | 5.52 | 3.18% | 4.78 | 5.03 | 5.23% |
|  |  | 95% | 6.16 | 12.26 | 99.03% | 5.64 | 11.87 | 110.46% | 5.1 | 11.17 | 119.02% |
|  | UL Tail-UPT CDF (Mbps) | Mean | 2.36 | 4.44 | 88.14% | 1.83 | 3.67 | 100.55% | 1.44 | 3.01 | 109.03% |
|  |  | 5% | 1.99 | 2.03 | 2.01% | 1.64 | 1.68 | 2.44% | 1.29 | 1.31 | 1.55% |
|  |  | 50% | 2.32 | 2.56 | 10.34% | 1.83 | 1.94 | 6.01% | 1.43 | 1.54 | 7.69% |
|  |  | 95% | 2.75 | 8.45 | 207.27% | 2.01 | 6.69 | 232.84% | 1.62 | 5.82 | 259.26% |
|  | UL Median-UPT CDF (Mbps) | Mean | 4.42 | 7.24 | 63.80% | 4.07 | 6.92 | 70.02% | 3.62 | 6.45 | 78.18% |
|  |  | 5% | 4.11 | 4.49 | 9.25% | 3.83 | 3.9 | 1.83% | 3.42 | 3.47 | 1.46% |
|  |  | 50% | 4.35 | 4.62 | 6.21% | 4.04 | 4.19 | 3.71% | 3.58 | 3.76 | 5.03% |
|  |  | 95% | 4.82 | 12.11 | 151.24% | 4.35 | 11.49 | 164.14% | 3.83 | 10.93 | 185.38% |
|  | DL Packet-Latency CDF (ms) | Mean | 0.84 | 0.82 | -2.38% | 0.86 | 0.85 | -1.16% | 0.93 | 1.07 | 15.05% |
|  |  | 5% | 0.45 | 0.45 | 0.00% | 0.45 | 0.48 | 6.67% | 0.48 | 0.48 | 0.00% |
|  |  | 50% | 0.77 | 0.77 | 0.00% | 0.77 | 0.77 | 0.00% | 0.8 | 0.84 | 5.00% |
|  |  | 95% | 1.41 | 1.38 | -2.13% | 1.45 | 1.41 | -2.76% | 1.66 | 2.16 | 30.12% |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean | 1.86 | 1.4 | -24.73% | 2.09 | 1.56 | -25.36% | 2.48 | 1.81 | -27.02% |
|  |  | 5% | 0.59 | 0.48 | -18.64% | 0.59 | 0.48 | -18.64% | 0.63 | 0.48 | -23.81% |
|  |  | 50% | 1.8 | 1.02 | -43.33% | 1.98 | 1.13 | -42.93% | 2.2 | 1.27 | -42.27% |
|  |  | 95% | 3.34 | 2.91 | -12.87% | 4.34 | 3.77 | -13.13% | 5.55 | 4.77 | -14.05% |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL | 8.21% | 8.04% | -2.07% | 14.33% | 14.01% | -2.23% | 26.48% | 25.82% | -2.49% |
|  |  | UL | 1.56% | 1.53% | -1.92% | 3.03% | 3.01% | -0.66% | 4.77% | 4.76% | -0.21% |
|  | Type-2 RU (%) | DL | 10.27% | 13.24% | 28.92% | 17.92% | 23.06% | 28.68% | 33.10% | 42.51% | 28.43% |
|  |  | UL | 7.78% | 4.25% | -45.37% | 15.13% | 8.34% | -44.88% | 23.87% | 13.20% | -44.70% |
|  | Unfinished/dropped Packet Rate (%) | DL | 0.02% | 0.01% |  | 0.02% | 0.02% |  | 0.02% | 0.03% |  |
|  |  | UL | 0.00% | 0.00% |  | 0.01% | 0.01% |  | 0.02% | 0.02% |  |
|  | Additional comments: e.g., |  |  |  |  |  |  |  |  |  |  |

Compared with the baseline TDD pattern, the UL resource of the SBFD subband pattern Alt.2 has been increased by around 80% and the DL resource has been decreased by around 20%. Overall, we have the following observation.

Observation 5: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium traffic load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 15% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 52% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 24% - 27% due to the increased UL resource.

SBFD Alt.4 subband pattern

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 43.62 | 48.52 | 11.23% | 42.87 | 47.95 | 11.85% | 40.80 | 46.11 | 13.01% |
|  | 5% | 42.29 | 47.40 | 12.08% | 40.54 | 45.98 | 13.42% | 36.72 | 40.74 | 10.95% |
|  | 50% | 43.75 | 48.58 | 11.04% | 43.05 | 48.14 | 11.82% | 41.36 | 46.81 | 13.18% |
|  | 95% | 44.83 | 49.37 | 10.13% | 43.96 | 48.75 | 10.90% | 42.34 | 47.68 | 12.61% |
| DL Tail-UPT CDF (Mbps) | Mean | 21.95 | 34.15 | 55.58% | 21.20 | 32.52 | 53.40% | 19.38 | 26.55 | 37.00% |
|  | 5% | 12.53 | 29.38 | 134.48% | 11.56 | 20.13 | 74.13% | 9.48 | 12.03 | 26.90% |
|  | 50% | 22.68 | 35.14 | 54.94% | 22.12 | 33.81 | 52.85% | 21.59 | 28.44 | 31.73% |
|  | 95% | 23.27 | 35.14 | 51.01% | 22.68 | 35.14 | 54.94% | 22.12 | 31.44 | 42.13% |
| DL Median-UPT CDF (Mbps) | Mean | 42.18 | 46.16 | 9.44% | 41.50 | 45.74 | 10.22% | 39.40 | 44.11 | 11.95% |
|  | 5% | 41.67 | 45.95 | 10.27% | 39.82 | 43.71 | 9.77% | 36.57 | 39.82 | 8.89% |
|  | 50% | 41.67 | 45.95 | 10.27% | 41.67 | 45.95 | 10.27% | 39.82 | 43.71 | 9.77% |
|  | 95% | 43.71 | 48.43 | 10.80% | 41.67 | 45.95 | 10.27% | 41.67 | 45.95 | 10.27% |
| UL Average-UPT CDF (Mbps) | Mean | 5.76 | 11.97 | 107.81% | 5.36 | 11.66 | 117.54% | 4.80 | 10.98 | 128.75% |
|  | 5% | 5.38 | 11.63 | 116.17% | 5.08 | 11.33 | 123.03% | 4.50 | 10.49 | 133.11% |
|  | 50% | 5.75 | 11.96 | 108.00% | 5.35 | 11.67 | 118.13% | 4.78 | 11.01 | 130.33% |
|  | 95% | 6.16 | 12.33 | 100.16% | 5.64 | 11.92 | 111.35% | 5.10 | 11.30 | 121.57% |
| UL Tail-UPT CDF (Mbps) | Mean | 2.36 | 7.59 | 221.61% | 1.83 | 6.35 | 246.99% | 1.44 | 5.10 | 254.17% |
|  | 5% | 1.99 | 6.31 | 217.09% | 1.64 | 5.82 | 254.88% | 1.29 | 3.53 | 173.64% |
|  | 50% | 2.32 | 7.59 | 227.16% | 1.83 | 6.31 | 244.81% | 1.43 | 5.27 | 268.53% |
|  | 95% | 2.75 | 8.78 | 219.27% | 2.01 | 6.89 | 242.79% | 1.62 | 5.67 | 250.00% |
| UL Median-UPT CDF (Mbps) | Mean | 4.42 | 11.47 | 159.50% | 4.07 | 11.20 | 175.18% | 3.62 | 10.64 | 193.92% |
|  | 5% | 4.11 | 10.93 | 165.94% | 3.83 | 10.93 | 185.38% | 3.42 | 10.42 | 204.68% |
|  | 50% | 4.35 | 11.49 | 164.14% | 4.04 | 10.93 | 170.54% | 3.58 | 10.42 | 191.06% |
|  | 95% | 4.82 | 12.11 | 151.24% | 4.35 | 11.49 | 164.14% | 3.83 | 10.93 | 185.38% |
| DL Packet-Latency CDF (ms) | Mean | 0.84 | 0.70 | -16.67% | 0.86 | 0.72 | -16.28% | 0.93 | 0.77 | -17.20% |
|  | 5% | 0.45 | 0.45 | 0.00% | 0.45 | 0.45 | 0.00% | 0.48 | 0.56 | 16.67% |
|  | 50% | 0.77 | 0.70 | -9.09% | 0.77 | 0.67 | -12.99% | 0.80 | 0.73 | -8.75% |
|  | 95% | 1.41 | 0.91 | -35.46% | 1.45 | 0.95 | -34.48% | 1.66 | 1.23 | -25.90% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.86 | 0.72 | -61.29% | 2.09 | 0.75 | -64.11% | 2.48 | 0.84 | -66.13% |
|  | 5% | 0.59 | 0.45 | -23.73% | 0.59 | 0.45 | -23.73% | 0.63 | 0.45 | -28.57% |
|  | 50% | 1.80 | 0.70 | -61.11% | 1.98 | 0.70 | -64.65% | 2.20 | 0.77 | -65.00% |
|  | 95% | 3.34 | 1.05 | -68.56% | 4.34 | 1.23 | -71.66% | 5.55 | 1.59 | -71.35% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 8.21% | 8.02% | -2.31% | 14.33% | 13.98% | -2.44% | 26.48% | 25.75% | -2.76% |
|  | UL | 1.56% | 1.45% | -7.05% | 3.03% | 2.88% | -4.95% | 4.77% | 4.61% | -3.35% |
| Type-2 RU (%) | DL | 10.27% | 10.57% | 2.92% | 17.92% | 18.42% | 2.79% | 33.10% | 33.92% | 2.48% |
|  | UL | 7.78% | 7.22% | -7.20% | 15.13% | 14.37% | -5.02% | 23.87% | 22.96% | -3.81% |
| Unfinished/dropped Packet Rate (%) | DL | 0.02% | 0.01% | -50.00% | 0.02% | 0.01% | -50.00% | 0.02% | 0.01% | -50.00% |
|  | UL | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 0.00% | 0.02% | 0.01% | -50.00% |

Observation 6: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.4 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is increased by around 11% - 13% due to increased transmission occasion in the last slot of each TDD period.

- The DL Packet-Latency (mean) of SBFD is decreased by around 16%-17% due to increased transmission occasion in the last slot of each TDD period.

- The UL average UPT (mean) of SBFD is increased by around 107% - 129% due to increased transmission occasion in the first four slots of each TDD period.

- The UL Packet-Latency (mean) of SBFD is decreased by around 61% - 66% due to increased transmission occasion in the first four slots of each TDD period.

Summary

If we compare the performance results between larger packet size (0.5Mbytes/0.125Mbytes) and smaller packet size (5Kbytes/1Kbytes) we can draw the following observation.

Observation 7: Regarding SBFD deployment case1, FR1 Indoor office,

- In case of larger packet size, obvious DL UPT loss is observed due to the reduced DL resource; in case of smaller packet size, small or no DL UPT loss is observed.

- The UL UPT gain in case of smaller packet size is larger than that of larger packet size.

Simulation results: FR1 Urban Macro

Packet size 0.5Mbytes/0.125Mbytes

SBFD Alt.2 subband pattern

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 429.54 | 347.96 | -18.99% | 340.83 | 253.45 | -25.64% | 293.54 | 207.75 | -29.23% |
|  | 5% | 115.54 | 74.20 | -35.78% | 68.67 | 38.55 | -43.86% | 36.69 | 16.63 | -54.67% |
|  | 50% | 581.82 | 461.86 | -20.62% | 350.18 | 248.53 | -29.03% | 263.26 | 200.61 | -23.80% |
|  | 95% | 660.77 | 537.17 | -18.71% | 631.09 | 518.76 | -17.80% | 621.83 | 460.78 | -25.90% |
| DL Tail-UPT CDF (Mbps) | Mean | 364.89 | 302.37 | -17.13% | 258.75 | 186.57 | -27.90% | 173.24 | 114.69 | -33.80% |
|  | 5% | 71.77 | 54.83 | -23.60% | 39.85 | 20.45 | -48.68% | 15.29 | 0.94 | -93.85% |
|  | 50% | 330.87 | 311.54 | -5.84% | 213.54 | 144.24 | -32.45% | 123.69 | 72.10 | -41.71% |
|  | 95% | 626.89 | 532.07 | -15.13% | 603.77 | 507.94 | -15.87% | 465.70 | 347.29 | -25.43% |
| DL Median-UPT CDF (Mbps) | Mean | 420.18 | 354.80 | -15.56% | 300.93 | 220.65 | -26.68% | 227.56 | 189.25 | -16.84% |
|  | 5% | 72.00 | 56.52 | -21.50% | 52.72 | 26.95 | -48.88% | 28.49 | 8.49 | -70.20% |
|  | 50% | 578.81 | 481.72 | -16.77% | 257.18 | 192.61 | -25.11% | 192.61 | 167.79 | -12.89% |
|  | 95% | 638.18 | 527.06 | -17.41% | 623.96 | 514.94 | -17.47% | 524.59 | 479.66 | -8.56% |
| UL Average-UPT CDF (Mbps) | Mean | 85.03 | 138.67 | 63.08% | 81.71 | 130.74 | 60.00% | 81.34 | 116.74 | 43.52% |
|  | 5% | 9.81 | 11.84 | 20.69% | 8.40 | 8.55 | 1.79% | 3.07 | 3.23 | 5.21% |
|  | 50% | 84.77 | 121.48 | 43.31% | 81.33 | 112.23 | 37.99% | 81.17 | 96.21 | 18.53% |
|  | 95% | 162.03 | 287.52 | 77.45% | 157.25 | 284.39 | 80.85% | 159.54 | 272.69 | 70.92% |
| UL Tail-UPT CDF (Mbps) | Mean | 73.75 | 126.05 | 70.92% | 50.10 | 104.78 | 109.14% | 34.24 | 73.13 | 113.58% |
|  | 5% | 9.65 | 10.32 | 6.94% | 6.78 | 8.25 | 21.68% | 0.75 | 2.00 | 166.67% |
|  | 50% | 65.65 | 107.07 | 63.09% | 45.79 | 81.28 | 77.51% | 21.60 | 42.14 | 95.09% |
|  | 95% | 158.64 | 278.61 | 75.62% | 140.35 | 262.91 | 87.32% | 101.27 | 244.54 | 141.47% |
| UL Median-UPT CDF (Mbps) | Mean | 78.59 | 131.83 | 67.74% | 80.45 | 132.73 | 64.98% | 62.87 | 120.58 | 91.79% |
|  | 5% | 14.30 | 11.56 | -19.16% | 9.16 | 11.27 | 23.03% | 3.71 | 4.40 | 18.60% |
|  | 50% | 78.98 | 117.40 | 48.65% | 81.51 | 118.90 | 45.87% | 57.20 | 101.63 | 77.67% |
|  | 95% | 150.94 | 273.17 | 80.98% | 147.76 | 275.86 | 86.69% | 141.06 | 270.53 | 91.78% |
| DL Packet-Latency CDF (ms) | Mean | 15.52 | 20.03 | 29.06% | 23.46 | 39.08 | 66.58% | 31.34 | 76.90 | 145.37% |
|  | 5% | 5.98 | 7.45 | 24.58% | 6.05 | 7.48 | 23.64% | 6.16 | 7.63 | 23.86% |
|  | 50% | 6.84 | 8.30 | 21.35% | 12.41 | 17.73 | 42.87% | 15.05 | 21.98 | 46.05% |
|  | 95% | 33.55 | 47.95 | 42.92% | 71.20 | 136.02 | 91.04% | 104.55 | 344.05 | 229.08% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 22.14 | 19.50 | -11.92% | 23.30 | 18.85 | -19.10% | 20.99 | 20.24 | -3.57% |
|  | 5% | 5.91 | 3.41 | -42.30% | 5.80 | 3.41 | -41.21% | 6.13 | 3.45 | -43.72% |
|  | 50% | 11.91 | 7.95 | -33.25% | 11.73 | 7.73 | -34.10% | 13.34 | 8.52 | -36.13% |
|  | 95% | 66.23 | 70.91 | 7.07% | 74.52 | 73.02 | -2.01% | 54.95 | 50.16 | -8.72% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 2.37% | 2.29% | -3.38% | 21.84% | 23.31% | 6.73% | 40.08% | 43.50% | 8.53% |
|  | UL | 3.69% | 3.74% | 1.36% | 6.89% | 6.74% | -2.18% | 10.06% | 14.02% | 39.36% |
| Type-2 RU (%) | DL | 2.96% | 3.80% | 28.38% | 27.30% | 38.67% | 41.65% | 50.10% | 72.16% | 44.03% |
|  | UL | 18.44% | 10.41% | -43.55% | 34.43% | 18.74% | -45.57% | 50.29% | 39.00% | -22.45% |
| Unfinished/dropped Packet Rate (%) | DL | 0.95% | 0.95% |  | 0.56% | 1.17% |  | 5.92% | 4.82% |  |
|  | UL | 24.47% | 25.67% |  | 28.06% | 29.18% |  | 32.25% | 29.37% |  |

Compared with the baseline TDD pattern, the UL resource of the SBFD subband pattern Alt.2 has been increased by around 80% and the DL resource has been decreased by around 20%. Overall, we have the following observation.

Observation 8: Regarding SBFD deployment case1, FR1 Urban Macro, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 19% - 29% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 29%-145% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the larger DL Packet-Latency (mean) of SBFD due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 63% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

- The UL Packet-Latency (mean) of SBFD is decreased by around 3% - 19% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

Packet size 5Kbytes/1Kbytes

SBFD Alt.2 subband pattern

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 44.50 | 44.69 | 0.43% | 44.20 | 43.76 | -1.00% | 43.74 | 42.42 | -3.02% |
|  | 5% | 41.79 | 41.65 | -0.34% | 42.16 | 40.36 | -4.27% | 41.82 | 35.09 | -16.09% |
|  | 50% | 44.46 | 44.79 | 0.74% | 44.28 | 44.19 | -0.20% | 43.86 | 42.62 | -2.83% |
|  | 95% | 47.08 | 47.26 | 0.38% | 46.19 | 46.11 | -0.17% | 45.28 | 44.45 | -1.83% |
| DL Tail-UPT CDF (Mbps) | Mean | 22.56 | 23.12 | 2.48% | 22.53 | 21.96 | -2.53% | 22.13 | 19.98 | -9.72% |
|  | 5% | 21.59 | 16.14 | -25.24% | 21.59 | 10.36 | -52.01% | 18.86 | 5.48 | -70.94% |
|  | 50% | 22.68 | 23.89 | 5.34% | 22.68 | 23.27 | 2.60% | 22.68 | 22.27 | -1.81% |
|  | 95% | 25.24 | 25.97 | 2.89% | 24.55 | 25.24 | 2.81% | 23.89 | 23.55 | -1.42% |
| DL Median-UPT CDF (Mbps) | Mean | 42.78 | 42.71 | -0.16% | 42.66 | 42.10 | -1.31% | 42.21 | 40.80 | -3.34% |
|  | 5% | 39.82 | 39.82 | 0.00% | 39.82 | 39.82 | 0.00% | 39.82 | 35.14 | -11.75% |
|  | 50% | 43.71 | 43.71 | 0.00% | 43.71 | 41.67 | -4.67% | 41.67 | 41.67 | 0.00% |
|  | 95% | 45.95 | 45.95 | 0.00% | 45.95 | 45.95 | 0.00% | 43.71 | 43.71 | 0.00% |
| UL Average-UPT CDF (Mbps) | Mean | 5.63 | 8.08 | 43.52% | 5.34 | 7.86 | 47.19% | 4.40 | 7.50 | 70.45% |
|  | 5% | 2.33 | 2.32 | -0.43% | 1.69 | 1.64 | -2.96% | 0.46 | 0.70 | 52.17% |
|  | 50% | 5.84 | 6.16 | 5.48% | 5.64 | 5.84 | 3.55% | 5.09 | 5.45 | 7.07% |
|  | 95% | 6.44 | 12.49 | 93.94% | 6.02 | 12.35 | 105.15% | 5.57 | 12.07 | 116.70% |
| UL Tail-UPT CDF (Mbps) | Mean | 2.48 | 4.87 | 96.37% | 2.02 | 4.50 | 122.77% | 1.21 | 3.77 | 211.57% |
|  | 5% | 0.92 | 1.07 | 16.30% | 0.38 | 0.45 | 18.42% | 0 | 0.03 |  |
|  | 50% | 2.75 | 2.78 | 1.09% | 2.14 | 2.37 | 10.75% | 1.62 | 1.89 | 16.67% |
|  | 95% | 2.82 | 8.78 | 211.35% | 2.53 | 8.78 | 247.04% | 1.96 | 8.45 | 331.12% |
| UL Median-UPT CDF (Mbps) | Mean | 4.33 | 7.11 | 64.20% | 4.08 | 6.89 | 68.87% | 3.23 | 6.60 | 104.33% |
|  | 5% | 1.76 | 1.78 | 1.14% | 1.34 | 1.19 | -11.19% | 0.02 | 0.35 | 1650.00% |
|  | 50% | 4.44 | 4.72 | 6.31% | 4.27 | 4.53 | 6.09% | 3.83 | 4.11 | 7.31% |
|  | 95% | 5.03 | 12.11 | 140.76% | 4.62 | 12.11 | 162.12% | 4.27 | 11.49 | 169.09% |
| DL Packet-Latency CDF (ms) | Mean | 0.81 | 0.81 | 0.00% | 0.82 | 1.76 | 114.63% | 0.83 | 2.33 | 180.72% |
|  | 5% | 0.45 | 0.45 | 0.00% | 0.45 | 0.45 | 0.00% | 0.45 | 0.45 | 0.00% |
|  | 50% | 0.73 | 0.73 | 0.00% | 0.73 | 0.77 | 5.48% | 0.77 | 0.77 | 0.00% |
|  | 95% | 1.38 | 1.34 | -2.90% | 1.41 | 1.38 | -2.13% | 1.43 | 1.70 | 18.88% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 2.07 | 1.58 | -23.67% | 5.04 | 4.63 | -8.13% | 21.08 | 8.08 | -61.67% |
|  | 5% | 0.55 | 0.48 | -12.73% | 0.59 | 0.48 | -18.64% | 0.63 | 0.48 | -23.81% |
|  | 50% | 1.80 | 0.95 | -47.22% | 1.91 | 1.05 | -45.03% | 2.27 | 1.16 | -48.90% |
|  | 95% | 3.98 | 3.59 | -9.80% | 4.95 | 4.52 | -8.69% | 66.84 | 5.52 | -91.74% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 3.19% | 3.26% | 2.19% | 12.98% | 13.11% | 1.00% | 18.08% | 20.55% | 13.66% |
|  | UL | 7.06% | 7.34% | 3.97% | 9.97% | 12.51% | 25.48% | 16.41% | 17.57% | 7.07% |
| Type-2 RU (%) | DL | 3.98% | 5.41% | 35.93% | 16.27% | 21.76% | 33.74% | 22.60% | 34.10% | 50.88% |
|  | UL | 35.3% | 20.42% | -42.15% | 49.84% | 34.79% | -30.20% | 82.03% | 48.88% | -40.41% |
| Unfinished/dropped Packet Rate (%) | DL | 0.21% | 0.52% |  | 0.39% | 1.12% |  | 0.57% | 1.81% |  |
|  | UL | 1.87% | 2.09% |  | 1.66% | 1.77% |  | 3.90% | 2.32% |  |

Observation 9: Regarding SBFD deployment case1, FR1 Urban Macro, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low load. In case of medium and high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 114%-180% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 70% due to the increased UL resource. The gain of UL average UPT (mean) of SBFD is increased as the increase of traffic load because the UL average UPT (mean) of baseline TDD is decreased as the increase of traffic load due to the limited UL resource in the baseline TDD.

- The UL Packet-Latency (mean) of SBFD is decreased by around 8% - 62% due to the increased UL resource.

Simulation results: FR1 Dense Urban Macro Layer

Packet size 0.5Mbytes/0.125Mbytes

SBFD Alt.2 subband pattern

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 530.09 | 432.84 | -18.35% | 429.76 | 335.56 | -21.92% | 338.13 | 242.22 | -28.36% |
|  | 5% | 210.28 | 160.58 | -23.64% | 141.40 | 95.66 | -32.35% | 78.02 | 38.17 | -51.08% |
|  | 50% | 583.23 | 477.75 | -18.09% | 463.38 | 351.22 | -24.20% | 356.59 | 242.89 | -31.89% |
|  | 95% | 656.98 | 540.70 | -17.70% | 605.58 | 521.67 | -13.86% | 576.78 | 471.73 | -18.21% |
| DL Tail-UPT CDF (Mbps) | Mean | 456.92 | 368.85 | -19.27% | 230.18 | 167.88 | -27.07% | 136.38 | 92.01 | -32.53% |
|  | 5% | 172.97 | 120.89 | -30.11% | 64.10 | 42.79 | -33.24% | 17.03 | 8.10 | -52.44% |
|  | 50% | 507.94 | 388.21 | -23.57% | 199.82 | 137.00 | -31.44% | 110.84 | 61.52 | -44.50% |
|  | 95% | 649.28 | 522.07 | -19.59% | 501.12 | 393.67 | -21.44% | 347.29 | 270.21 | -22.19% |
| DL Median-UPT CDF (Mbps) | Mean | 493.16 | 398.04 | -19.29% | 407.71 | 279.56 | -31.43% | 318.47 | 224.94 | -29.37% |
|  | 5% | 176.52 | 132.62 | -24.87% | 114.46 | 73.42 | -35.86% | 54.02 | 22.72 | -57.94% |
|  | 50% | 581.82 | 471.58 | -18.95% | 418.69 | 260.77 | -37.72% | 313.29 | 200.18 | -36.10% |
|  | 95% | 645.53 | 527.06 | -18.35% | 620.50 | 512.58 | -17.39% | 603.77 | 498.89 | -17.37% |
| UL Average-UPT CDF (Mbps) | Mean | 117.37 | 145.51 | 23.98% | 109.04 | 140.83 | 29.15% | 104.20 | 133.90 | 28.50% |
|  | 5% | 36.01 | 36.25 | 0.67% | 27.09 | 29.76 | 9.86% | 20.94 | 22.36 | 6.78% |
|  | 50% | 137.51 | 143.22 | 4.15% | 130.59 | 137.59 | 5.36% | 123.94 | 135.17 | 9.06% |
|  | 95% | 168.17 | 290.02 | 72.46% | 156.09 | 284.91 | 82.53% | 153.50 | 274.39 | 78.76% |
| UL Tail-UPT CDF (Mbps) | Mean | 103.98 | 131.47 | 26.44% | 71.60 | 99.24 | 38.60% | 54.57 | 79.19 | 45.12% |
|  | 5% | 24.51 | 27.46 | 12.04% | 9.82 | 11.85 | 20.67% | 6.35 | 7.10 | 11.81% |
|  | 50% | 126.41 | 128.74 | 1.84% | 73.78 | 83.21 | 12.78% | 49.17 | 67.07 | 36.40% |
|  | 95% | 165.19 | 273.17 | 65.37% | 136.92 | 258.06 | 88.48% | 129.93 | 238.30 | 83.41% |
| UL Median-UPT CDF (Mbps) | Mean | 106.65 | 132.80 | 24.52% | 105.24 | 128.62 | 22.22% | 102.60 | 133.86 | 30.47% |
|  | 5% | 24.99 | 27.01 | 8.08% | 20.34 | 22.39 | 10.08% | 16.83 | 19.29 | 14.62% |
|  | 50% | 127.56 | 129.93 | 1.86% | 129.93 | 132.33 | 1.85% | 128.15 | 134.29 | 4.79% |
|  | 95% | 158.64 | 275.86 | 73.89% | 154.27 | 273.17 | 77.07% | 154.27 | 275.86 | 78.82% |
| DL Packet-Latency CDF (ms) | Mean | 8.85 | 11.19 | 26.44% | 13.30 | 18.16 | 36.54% | 24.23 | 45.51 | 87.83% |
|  | 5% | 5.98 | 7.16 | 19.73% | 6.01 | 7.34 | 22.13% | 6.13 | 7.59 | 23.82% |
|  | 50% | 6.66 | 7.88 | 18.32% | 8.98 | 11.77 | 31.07% | 12.63 | 18.95 | 50.04% |
|  | 95% | 17.91 | 23.63 | 31.94% | 35.13 | 50.59 | 44.01% | 81.38 | 176.77 | 117.22% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 12.33 | 11.48 | -6.89% | 16.03 | 14.22 | -11.29% | 18.90 | 16.19 | -14.34% |
|  | 5% | 5.63 | 3.56 | -36.77% | 5.66 | 3.45 | -39.05% | 5.66 | 3.48 | -38.52% |
|  | 50% | 7.52 | 7.20 | -4.26% | 7.84 | 7.38 | -5.87% | 8.30 | 7.55 | -9.04% |
|  | 95% | 31.41 | 30.59 | -2.61% | 48.45 | 41.91 | -13.50% | 67.27 | 59.16 | -12.06% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 7.24% | 6.96% | -3.87% | 26.20% | 24.82% | -5.27% | 51.10% | 48.35% | -5.38% |
|  | UL | 2.21% | 2.29% | 3.62% | 6.95% | 6.62% | -4.75% | 9.64% | 9.54% | -1.04% |
| Type-2 RU (%) | DL | 9.05% | 11.54% | 27.51% | 32.75% | 41.17% | 25.71% | 63.88% | 80.19% | 25.53% |
|  | UL | 11.03% | 6.36% | -42.34% | 34.76% | 18.41% | -47.04% | 48.19% | 26.54% | -44.93% |
| Unfinished/dropped Packet Rate (%) | DL | 0.64% | 1.04% |  | 2.01% | 3.03% |  | 4.83% | 6.37% |  |
|  | UL | 1.95% | 1.20% |  | 4.77% | 1.73% |  | 4.51% | 2.86% |  |

Compared with the baseline TDD pattern, the UL resource of the SBFD subband pattern Alt.2 has been increased by around 80% and the DL resource has been decreased by around 20%. Overall, we have the following observation.

Observation 10: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes,

- The DL average UPT (mean) of SBFD is decreased by around 18% - 28% due to the decreased DL resource and UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 26%-88% due to the decreased DL resource and UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 23%-28% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 6% - 14% due to the increased UL resource.

SBFD Alt.4 subband pattern

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 530.09 | 536.36 | 1.18% | 429.76 | 422.95 | -1.58% | 338.13 | 329.33 | -2.60% |
|  | 5% | 210.28 | 208.81 | -0.70% | 141.40 | 137.31 | -2.89% | 78.02 | 77.02 | -1.28% |
|  | 50% | 583.23 | 592.24 | 1.54% | 463.38 | 449.96 | -2.90% | 356.59 | 344.77 | -3.31% |
|  | 95% | 656.98 | 658.97 | 0.30% | 605.58 | 647.45 | 6.91% | 576.78 | 578.91 | 0.37% |
| DL Tail-UPT CDF (Mbps) | Mean | 456.92 | 464.29 | 1.61% | 230.18 | 218.94 | -4.88% | 136.38 | 131.20 | -3.80% |
|  | 5% | 172.97 | 161.27 | -6.76% | 64.10 | 57.57 | -10.19% | 17.03 | 16.27 | -4.46% |
|  | 50% | 507.94 | 498.89 | -1.78% | 199.82 | 192.61 | -3.61% | 110.84 | 97.60 | -11.95% |
|  | 95% | 649.28 | 653.06 | 0.58% | 501.12 | 485.90 | -3.04% | 347.29 | 340.94 | -1.83% |
| DL Median-UPT CDF (Mbps) | Mean | 493.16 | 503.47 | 2.09% | 407.71 | 363.05 | -10.95% | 318.47 | 307.19 | -3.54% |
|  | 5% | 176.52 | 169.31 | -4.08% | 114.46 | 98.12 | -14.28% | 54.02 | 53.65 | -0.68% |
|  | 50% | 581.82 | 594.16 | 2.12% | 418.69 | 346.21 | -17.31% | 313.29 | 289.03 | -7.74% |
|  | 95% | 645.53 | 653.06 | 1.17% | 620.50 | 638.18 | 2.85% | 603.77 | 627.45 | 3.92% |
| UL Average-UPT CDF (Mbps) | Mean | 117.37 | 141.48 | 20.54% | 109.04 | 132.51 | 21.52% | 104.20 | 122.25 | 17.32% |
|  | 5% | 36.01 | 44.20 | 22.74% | 27.09 | 35.48 | 30.97% | 20.94 | 26.38 | 25.98% |
|  | 50% | 137.51 | 172.74 | 25.62% | 130.59 | 157.07 | 20.28% | 123.94 | 145.13 | 17.10% |
|  | 95% | 168.17 | 177.78 | 5.71% | 156.09 | 180.66 | 15.74% | 153.50 | 175.42 | 14.28% |
| UL Tail-UPT CDF (Mbps) | Mean | 103.98 | 130.31 | 25.32% | 71.60 | 89.82 | 25.45% | 54.57 | 61.00 | 11.78% |
|  | 5% | 24.51 | 27.39 | 11.75% | 9.82 | 12.70 | 29.33% | 6.35 | 6.07 | -4.41% |
|  | 50% | 126.41 | 169.18 | 33.83% | 73.78 | 91.65 | 24.22% | 49.17 | 53.79 | 9.40% |
|  | 95% | 165.19 | 180.06 | 9.00% | 136.92 | 171.25 | 25.07% | 129.93 | 169.18 | 30.21% |
| UL Median-UPT CDF (Mbps) | Mean | 106.65 | 134.90 | 26.49% | 105.24 | 121.01 | 14.98% | 102.60 | 126.19 | 22.99% |
|  | 5% | 24.99 | 39.46 | 57.90% | 20.34 | 25.35 | 24.63% | 16.83 | 17.38 | 3.27% |
|  | 50% | 127.56 | 170.21 | 33.44% | 129.93 | 143.22 | 10.23% | 128.15 | 169.18 | 32.02% |
|  | 95% | 158.64 | 177.78 | 12.07% | 154.27 | 177.78 | 15.24% | 154.27 | 176.66 | 14.51% |
| DL Packet-Latency CDF (ms) | Mean | 8.85 | 8.86 | 0.11% | 13.30 | 13.81 | 3.83% | 24.23 | 26.80 | 10.61% |
|  | 5% | 5.98 | 5.98 | 0.00% | 6.01 | 6.02 | 0.17% | 6.13 | 6.13 | 0.00% |
|  | 50% | 6.66 | 6.38 | -4.20% | 8.98 | 8.91 | -0.78% | 12.63 | 13.16 | 4.20% |
|  | 95% | 17.91 | 18.41 | 2.79% | 35.13 | 37.09 | 5.58% | 81.38 | 90.55 | 11.27% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 12.33 | 9.98 | -19.06% | 16.03 | 12.13 | -24.33% | 18.90 | 17.75 | -6.08% |
|  | 5% | 5.63 | 5.48 | -2.66% | 5.66 | 5.48 | -3.18% | 5.66 | 5.48 | -3.18% |
|  | 50% | 7.52 | 5.80 | -22.87% | 7.84 | 5.88 | -25.00% | 8.30 | 6.16 | -25.78% |
|  | 95% | 31.41 | 24.95 | -20.57% | 48.45 | 35.52 | -26.69% | 67.27 | 66.98 | -0.43% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 7.24% | 6.97% | -3.73% | 26.20% | 25.23% | -3.70% | 51.10% | 49.76% | -2.62% |
|  | UL | 2.21% | 1.98% | -10.41% | 6.95% | 5.83% | -16.12% | 9.64% | 8.49% | -11.93% |
| Type-2 RU (%) | DL | 9.05% | 9.25% | 2.21% | 32.75% | 33.47% | 2.20% | 63.88% | 66.02% | 3.35% |
|  | UL | 11.03% | 9.94% | -9.88% | 34.76% | 29.28% | -15.77% | 48.19% | 42.61% | -11.58% |
| Unfinished/dropped Packet Rate (%) | DL | 0.64% | 0.64% |  | 2.01% | 3.03% |  | 4.83% | 5.35% |  |
|  | UL | 1.95% | 1.93% |  | 4.77% | 1.42% |  | 4.51% | 2.16% |  |

For SBFD subband pattern Alt.4, its UL resource ratio and DL resource ratio are almost the same as baseline TDD. Overall, we have the following observation.

Observation 11: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.4 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is almost the same as legacy TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as legacy TDD.

- The UL average UPT (mean) of SBFD is increased by around 17% - 20% due to more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 19% - 24% in case of low and medium traffic load due to more transmission occasions in time domain. In case of high traffic load, the UL Packet-Latency (mean) of SBFD is decreased by around 6%.

Packet size 5Kbytes/1Kbytes

SBFD Alt.2 subband pattern

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 42.80 | 43.21 | 0.96% | 41.98 | 42.03 | 0.12% | 40.56 | 39.07 | -3.67% |
|  | 5% | 38.62 | 38.15 | -1.22% | 37.34 | 36.90 | -1.18% | 35.58 | 27.62 | -22.37% |
|  | 50% | 43.32 | 43.69 | 0.85% | 42.86 | 42.94 | 0.19% | 41.57 | 40.67 | -2.17% |
|  | 95% | 45.57 | 46.20 | 1.38% | 44.65 | 45.07 | 0.94% | 43.63 | 44.25 | 1.42% |
| DL Tail-UPT CDF (Mbps) | Mean | 18.10 | 18.85 | 4.14% | 17.14 | 17.52 | 2.22% | 15.84 | 15.37 | -2.97% |
|  | 5% | 8.04 | 8.57 | 6.59% | 7.96 | 8.49 | 6.66% | 8.04 | 5.88 | -26.87% |
|  | 50% | 22.23 | 23.27 | 4.68% | 22.12 | 22.68 | 2.53% | 18.47 | 12.36 | -33.08% |
|  | 95% | 23.89 | 25.24 | 5.65% | 23.27 | 24.55 | 5.50% | 22.68 | 23.89 | 5.34% |
| DL Median-UPT CDF (Mbps) | Mean | 41.85 | 41.89 | 0.10% | 41.25 | 40.99 | -0.63% | 39.88 | 37.76 | -5.32% |
|  | 5% | 39.82 | 38.13 | -4.24% | 38.13 | 38.13 | 0.00% | 36.57 | 23.27 | -36.37% |
|  | 50% | 41.67 | 41.67 | 0.00% | 41.67 | 41.67 | 0.00% | 39.82 | 39.82 | 0.00% |
|  | 95% | 43.71 | 43.71 | 0.00% | 43.71 | 43.71 | 0.00% | 41.67 | 41.67 | 0.00% |
| UL Average-UPT CDF (Mbps) | Mean | 6.00 | 7.37 | 22.83% | 5.76 | 7.13 | 23.78% | 5.31 | 6.73 | 26.74% |
|  | 5% | 5.40 | 5.26 | -2.59% | 5.37 | 5.31 | -1.12% | 4.96 | 4.98 | 0.40% |
|  | 50% | 5.98 | 6.17 | 3.18% | 5.76 | 5.86 | 1.74% | 5.32 | 5.42 | 1.88% |
|  | 95% | 6.64 | 12.58 | 89.46% | 6.16 | 12.08 | 96.10% | 5.65 | 11.80 | 108.85% |
| UL Tail-UPT CDF (Mbps) | Mean | 2.70 | 3.76 | 39.26% | 2.37 | 3.50 | 47.68% | 1.78 | 2.80 | 57.30% |
|  | 5% | 2.16 | 1.91 | -11.57% | 1.99 | 1.97 | -1.01% | 1.46 | 1.55 | 6.16% |
|  | 50% | 2.78 | 2.78 | 0.00% | 2.37 | 2.48 | 4.64% | 1.80 | 1.87 | 3.89% |
|  | 95% | 2.89 | 8.78 | 203.81% | 2.75 | 7.86 | 185.82% | 2.03 | 6.69 | 229.56% |
| UL Median-UPT CDF (Mbps) | Mean | 4.60 | 6.12 | 33.04% | 4.42 | 5.96 | 34.84% | 4.04 | 5.62 | 39.11% |
|  | 5% | 3.96 | 3.96 | 0.00% | 4.11 | 4.04 | -1.70% | 3.76 | 3.76 | 0.00% |
|  | 50% | 4.53 | 4.72 | 4.19% | 4.44 | 4.53 | 2.03% | 4.04 | 4.11 | 1.73% |
|  | 95% | 5.40 | 12.11 | 124.26% | 4.82 | 11.49 | 138.38% | 4.27 | 11.49 | 169.09% |
| DL Packet-Latency CDF (ms) | Mean | 0.92 | 0.90 | -2.17% | 0.97 | 0.96 | -1.03% | 1.02 | 1.14 | 11.76% |
|  | 5% | 0.45 | 0.45 | 0.00% | 0.45 | 0.45 | 0.00% | 0.48 | 0.48 | 0.00% |
|  | 50% | 0.77 | 0.77 | 0.00% | 0.77 | 0.77 | 0.00% | 0.80 | 0.84 | 5.00% |
|  | 95% | 1.48 | 1.48 | 0.00% | 2.55 | 2.66 | 4.31% | 2.80 | 3.16 | 12.86% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.75 | 1.51 | -13.71% | 1.85 | 1.59 | -14.05% | 2.13 | 1.81 | -15.02% |
|  | 5% | 0.55 | 0.48 | -12.73% | 0.59 | 0.52 | -11.86% | 0.59 | 0.52 | -11.86% |
|  | 50% | 1.73 | 1.38 | -20.23% | 1.80 | 1.41 | -21.67% | 1.98 | 1.59 | -19.70% |
|  | 95% | 2.88 | 2.84 | -1.39% | 3.30 | 3.02 | -8.48% | 4.48 | 4.13 | -7.81% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.91% | 6.83% | -1.16% | 14.39% | 14.66% | 1.88% | 29.35% | 31.51% | 7.36% |
|  | UL | 2.32% | 2.31% | -0.43% | 5.82% | 5.80% | -0.34% | 11.56% | 11.58% | 0.17% |
| Type-2 RU (%) | DL | 8.64% | 11.32% | 31.02% | 17.99% | 24.31% | 35.13% | 36.68% | 52.27% | 42.50% |
|  | UL | 11.59% | 6.44% | -44.43% | 29.08% | 16.13% | -44.53% | 57.81% | 32.21% | -44.28% |
| Unfinished/dropped Packet Rate (%) | DL | 0.30% | 0.24% |  | 0.66% | 0.70% |  | 0.98% | 1.29% |  |
|  | UL | 0.01% | 0.01% |  | 0.01% | 0.01% |  | 0.01% | 0.02% |  |

Observation 12: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes,

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 12% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 22% - 27% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 13% - 15% due to the increased UL resource.

SBFD Alt.4 subband pattern

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 42.80 | 43.16 | 0.84% | 41.98 | 42.00 | 0.05% | 40.56 | 39.18 | -3.40% |
|  | 5% | 38.74 | 38.53 | -0.54% | 37.34 | 36.85 | -1.31% | 35.58 | 28.54 | -19.79% |
|  | 50% | 43.37 | 43.49 | 0.28% | 42.86 | 42.86 | 0.00% | 41.57 | 40.71 | -2.07% |
|  | 95% | 45.66 | 46.04 | 0.83% | 44.65 | 45.18 | 1.19% | 43.63 | 44.44 | 1.86% |
| DL Tail-UPT CDF (Mbps) | Mean | 18.17 | 18.86 | 3.80% | 17.14 | 17.43 | 1.69% | 15.84 | 15.40 | -2.78% |
|  | 5% | 8.04 | 8.66 | 7.71% | 7.96 | 8.33 | 4.65% | 8.04 | 5.84 | -27.36% |
|  | 50% | 22.32 | 23.27 | 4.26% | 22.12 | 22.12 | 0.00% | 18.47 | 12.53 | -32.16% |
|  | 95% | 23.89 | 25.24 | 5.65% | 23.27 | 24.55 | 5.50% | 22.68 | 23.89 | 5.34% |
| DL Median-UPT CDF (Mbps) | Mean | 41.85 | 41.90 | 0.12% | 41.25 | 40.99 | -0.63% | 39.88 | 37.87 | -5.04% |
|  | 5% | 39.82 | 38.13 | -4.24% | 38.13 | 38.13 | 0.00% | 36.57 | 23.89 | -34.67% |
|  | 50% | 41.67 | 41.67 | 0.00% | 41.67 | 41.67 | 0.00% | 39.82 | 39.82 | 0.00% |
|  | 95% | 43.71 | 43.71 | 0.00% | 43.71 | 43.71 | 0.00% | 41.67 | 41.67 | 0.00% |
| UL Average-UPT CDF (Mbps) | Mean | 6.00 | 7.37 | 22.83% | 5.76 | 7.12 | 23.61% | 5.31 | 6.73 | 26.74% |
|  | 5% | 5.45 | 5.38 | -1.28% | 5.37 | 5.29 | -1.49% | 4.96 | 4.93 | -0.60% |
|  | 50% | 6.01 | 6.17 | 2.66% | 5.76 | 5.88 | 2.08% | 5.32 | 5.43 | 2.07% |
|  | 95% | 6.64 | 12.42 | 87.05% | 6.16 | 12.11 | 96.59% | 5.65 | 11.83 | 109.38% |
| UL Tail-UPT CDF (Mbps) | Mean | 2.75 | 3.88 | 41.09% | 2.37 | 3.49 | 47.26% | 1.78 | 2.79 | 56.74% |
|  | 5% | 2.42 | 2.12 | -12.40% | 1.99 | 1.92 | -3.52% | 1.46 | 1.46 | 0.00% |
|  | 50% | 2.78 | 2.78 | 0.00% | 2.37 | 2.45 | 3.38% | 1.80 | 1.87 | 3.89% |
|  | 95% | 2.89 | 8.78 | 203.81% | 2.75 | 8.15 | 196.36% | 2.03 | 6.69 | 229.56% |
| UL Median-UPT CDF (Mbps) | Mean | 4.62 | 6.12 | 32.47% | 4.42 | 5.96 | 34.84% | 4.04 | 5.63 | 39.36% |
|  | 5% | 4.11 | 4.04 | -1.70% | 4.11 | 4.04 | -1.70% | 3.76 | 3.76 | 0.00% |
|  | 50% | 5.62 | 5.71 | 1.60% | 4.44 | 5.53 | 24.55% | 4.04 | 4.11 | 1.73% |
|  | 95% | 5.47 | 12.11 | 121.39% | 4.82 | 11.49 | 138.38% | 4.27 | 11.49 | 169.09% |
| DL Packet-Latency CDF (ms) | Mean | 0.92 | 0.90 | -2.17% | 0.97 | 0.96 | -1.03% | 1.02 | 1.13 | 10.78% |
|  | 5% | 0.45 | 0.45 | 0.00% | 0.45 | 0.45 | 0.00% | 0.48 | 0.48 | 0.00% |
|  | 50% | 0.77 | 0.77 | 0.00% | 0.77 | 0.77 | 0.00% | 0.80 | 0.84 | 5.00% |
|  | 95% | 1.48 | 1.45 | -2.03% | 2.55 | 2.66 | 4.31% | 2.80 | 3.13 | 11.79% |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.74 | 1.51 | -13.22% | 1.85 | 1.60 | -13.51% | 2.13 | 1.81 | -15.02% |
|  | 5% | 0.55 | 0.48 | -12.73% | 0.59 | 0.52 | -11.86% | 0.59 | 0.82 | 38.98% |
|  | 50% | 1.73 | 1.38 | -20.23% | 1.80 | 1.41 | -21.67% | 1.98 | 1.59 | -19.70% |
|  | 95% | 2.88 | 2.84 | -1.39% | 3.30 | 3.02 | -8.48% | 4.48 | 4.13 | -7.81% |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.91% | 6.84% | -1.01% | 14.39% | 14.60% | 1.46% | 29.35% | 30.77% | 4.84% |
|  | UL | 2.32% | 2.32% | 0.00% | 5.82% | 5.81% | -0.17% | 11.56% | 11.57% | 0.09% |
| Type-2 RU (%) | DL | 8.64% | 11.34% | 31.25% | 17.99% | 24.21% | 34.57% | 36.68% | 51.04% | 39.15% |
|  | UL | 11.60% | 6.47% | -44.22% | 29.08% | 16.17% | -44.39% | 57.81% | 32.20% | -44.30% |
| Unfinished/dropped Packet Rate (%) | DL | 0.29% | 0.25% |  | 0.66% | 0.70% |  | 0.98% | 1.39% |  |
|  | UL | 0.01% | 0.02% |  | 0.01% | 0.02% |  | 0.01% | 0.04% |  |

Observation 13: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.4 subband pattern, Packet size 5Kbytes/1Kbytes,

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 11% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 22% - 27% due to the more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 13% - 15% due to the more transmission occasions in time domain.

Simulation results: FR1 Dense Urban (2 layers)

Packet size 0.5Mbytes/0.125Mbytes

SBFD Alt.2 subband pattern

| Simple description for the sub-case (e.g., 100dB co-site inter-sector co-channel inter-subband isolation, SBFD Alt2, 49dBm gNB Tx power, Twice area&same TxRUs, DL: 4Kbytes, UL: 1Kbyte, UE clustering,…) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT (Mbps) | MacroUE Mean | 562.11 | 391.26 | -30.39% | 510.78 | 345.86 | -32.29% | 417.21 | 254.78 | -38.93% |
|  | MacroUE 5% | 475.66 | 292.07 | -38.60% | 339.47 | 212.42 | -37.43% | 194.02 | 84.44 | -56.48% |
|  | MacroUE 50% | 574.38 | 399.19 | -30.50% | 528.71 | 356.24 | -32.62% | 435.01 | 262.53 | -39.65% |
|  | MacroUE 95% | 629.32 | 455.02 | -27.70% | 603.99 | 434.34 | -28.09% | 567.30 | 397.77 | -29.88% |
|  | LPNUE Mean | 554.47 | 388.87 | -29.87% | 486.67 | 336.66 | -30.82% | 381.81 | 238.04 | -37.65% |
|  | LPNUE 5% | 434.90 | 288.77 | -33.60% | 278.84 | 194.66 | -30.19% | 122.91 | 58.90 | -52.08% |
|  | LPNUE 50% | 563.12 | 400.09 | -28.95% | 503.61 | 346.93 | -31.11% | 400.18 | 249.89 | -37.56% |
|  | LPNUE 95% | 626.72 | 451.38 | -27.98% | 599.36 | 434.98 | -27.43% | 564.56 | 385.85 | -31.65% |
| UL Average-UPT (Mbps) | MacroUE Mean | 140.35 | 154.98 | 10.42% | 134.56 | 149.01 | 10.74% | 122.43 | 135.81 | 10.93% |
|  | MacroUE 5% | 99.35 | 117.32 | 18.09% | 105.39 | 110.91 | 5.24% | 88.03 | 93.61 | 6.34% |
|  | MacroUE 50% | 145.44 | 147.20 | 1.21% | 137.58 | 144.19 | 4.80% | 125.63 | 130.89 | 4.19% |
|  | MacroUE 95% | 155.96 | 296.12 | 89.87% | 151.39 | 282.15 | 86.37% | 142.74 | 261.74 | 83.37% |
|  | LPNUE Mean | 143.97 | 164.91 | 14.54% | 139.26 | 157.71 | 13.25% | 129.99 | 147.44 | 13.42% |
|  | LPNUE 5% | 107.09 | 132.38 | 23.62% | 113.34 | 125.24 | 10.50% | 102.73 | 106.04 | 3.22% |
|  | LPNUE 50% | 146.87 | 150.20 | 2.27% | 141.66 | 144.77 | 2.20% | 131.14 | 136.62 | 4.18% |
|  | LPNUE 95% | 158.11 | 311.23 | 96.84% | 155.05 | 290.64 | 87.45% | 148.82 | 271.29 | 82.29% |
| DL Packet-Latency CDF (ms) | MacroUE Mean | 9.00 | 13.38 | 48.67% | 11.69 | 19.62 | 67.84% | 18.89 | 42.81 | 126.63% |
|  | MacroUE 5% | 5.98 | 8.52 | 42.47% | 6.02 | 8.59 | 42.69% | 6.13 | 8.80 | 43.56% |
|  | MacroUE 50% | 6.73 | 9.38 | 39.38% | 7.84 | 13.09 | 66.96% | 12.48 | 21.84 | 75.00% |
|  | MacroUE 95% | 18.84 | 25.63 | 36.04% | 27.84 | 44.30 | 59.12% | 53.66 | 147.55 | 174.97% |
|  | LPNUE Mean | 9.95 | 15.62 | 56.98% | 13.44 | 20.63 | 53.50% | 23.07 | 44.41 | 92.50% |
|  | LPNUE 5% | 5.98 | 8.52 | 42.47% | 6.05 | 8.63 | 42.64% | 6.20 | 8.91 | 43.71% |
|  | LPNUE 50% | 6.88 | 10.09 | 46.66% | 9.45 | 14.84 | 57.04% | 14.45 | 24.02 | 66.23% |
|  | LPNUE 95% | 22.09 | 29.84 | 35.08% | 32.88 | 51.45 | 56.48% | 66.20 | 143.48 | 116.74% |
| UL Packet-Latency CDF (ms) | MacroUE Mean | 16.93 | 9.62 | -43.18% | 13.62 | 11.23 | -17.55% | 11.68 | 9.80 | -16.10% |
|  | MacroUE 5% | 5.59 | 3.30 | -40.97% | 5.59 | 3.34 | -40.25% | 5.63 | 3.38 | -39.96% |
|  | MacroUE 50% | 6.95 | 6.77 | -2.59% | 7.09 | 6.91 | -2.54% | 7.48 | 7.27 | -2.81% |
|  | MacroUE 95% | 12.91 | 11.91 | -7.75% | 17.77 | 14.20 | -20.09% | 26.45 | 22.70 | -14.18% |
|  | LPNUE Mean | 14.67 | 9.98 | -31.97% | 11.46 | 8.26 | -27.92% | 9.77 | 8.37 | -14.33% |
|  | LPNUE 5% | 5.55 | 3.16 | -43.06% | 5.59 | 3.20 | -42.75% | 5.63 | 3.27 | -41.92% |
|  | LPNUE 50% | 6.77 | 6.55 | -3.25% | 6.95 | 6.73 | -3.17% | 7.23 | 7.02 | -2.90% |
|  | LPNUE 95% | 12.02 | 10.48 | -12.81% | 14.52 | 12.45 | -14.26% | 19.63 | 16.38 | -16.56% |
| DL RU (%) | Macro Type-1 | 9.10% | 8.58% | -5.71% | 17.08% | 16.55% | -3.10% | 31.06% | 31.08% | 0.06% |
|  | LPN Type-1 | 7.56% | 7.09% | -6.22% | 14.49% | 13.78% | -4.90% | 28.45% | 27.74% | -2.50% |
|  | Macro Type-2 | 11.38% | 14.19% | 24.69% | 21.35% | 27.35% | 28.10% | 38.82% | 51.36% | 32.30% |
|  | LPN Type-2 | 9.45% | 11.71% | 23.92% | 18.11% | 22.77% | 25.73% | 35.56% | 45.84% | 28.91% |
| UL RU (%) | Macro Type-1 | 3.70% | 2.38% | -35.68% | 5.20% | 4.20% | -19.23% | 7.84% | 7.42% | -5.36% |
|  | LPN Type-1 | 18.50% | 1.59% | -91.41% | 3.50% | 3.09% | -11.71% | 5.74% | 5.58% | -2.79% |
|  | Macro Type-2 | 2.22% | 6.60% | 197.30% | 26.02% | 11.67% | -55.15% | 39.18% | 20.62% | -47.37% |
|  | LPN Type-2 | 11.10% | 4.42% | -60.18% | 17.49% | 8.57% | -51.00% | 28.69% | 15.49% | -46.01% |
| Unfinished/dropped Packet Rate (%) | DL | 0.16% | 0.55% |  | 0.22% | 1.21% |  | 0.66% | 3.24% |  |
|  | UL | 0.93% | 0.55% |  | 0.16% | 0.38% |  | 0.19% | 0.23% |  |
| Note: |  |  |  |  |  |  |  |  |  |  |

Compared with the baseline TDD pattern, the UL resource of the SBFD subband pattern Alt.2 has been increased by around 80% and the DL resource has been decreased by around 20%. Overall, we have the following observation.

Observation 14: Regarding SBFD deployment case1, FR1 Dense Urban (2-layer scenario), SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- For both Macro layer and Micro layer, the DL average UPT (mean) of SBFD is decreased by around 30% - 39% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI.

- For Macro layer, the DL Packet-Latency (mean) of SBFD is increased by around 48%-127% due to the decreased DL resource and UE-UE CLI; for Micro layer, the DL Packet-Latency (mean) of SBFD is increased by around 57%-93% due to the decreased DL resource and UE-UE CLI.

- For Macro layer, the UL average UPT (mean) of SBFD is increased by around 10% due to the increased UL resource; for Micro layer, the UL average UPT (mean) of SBFD is increased by around 14% due to the increased UL resource.

- For Macro layer, the UL Packet-Latency (mean) of SBFD is decreased by around 16% - 43% due to the increased UL resource; for Micro layer, the UL Packet-Latency (mean) of SBFD is decreased by around 14% - 32% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

Preliminary SLS results for dynamic TDD

In this section, the preliminary simulation results and some observations for dynamic TDD based on the simulation results are captured.

Simulation assumptions

The detailed simulation assumptions can be found in the Appendix.

Simulation results: HetNet (Macro and Indoor office)

| Simple description for the sub-case (e.g., 100dB co-site inter-sector co-channel inter-subband isolation, SBFD Alt2, 49dBm gNB Tx power, Twice area&same TxRUs, DL: 4Kbytes, UL: 1Kbyte, UE clustering,…) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | DTDD | Gain (%) | TDD | DTDD | Gain (%) | TDD | DTDD | Gain (%) |
| DL Average-UPT (Mbps) | MacroUE Mean | 486.30 | 474.47 | -2.43% | 448.64 | 421.00 | -6.16% | 346.75 | 312.43 | -9.90% |
|  | MacroUE 5% | 197.16 | 189.85 | -3.71% | 175.05 | 153.41 | -12.36% | 95.23 | 72.06 | -24.33% |
|  | MacroUE 50% | 600.24 | 574.25 | -4.33% | 527.02 | 491.22 | -6.79% | 383.21 | 320.82 | -16.28% |
|  | MacroUE 95% | 657.36 | 656.89 | -0.07% | 640.42 | 633.14 | -1.14% | 591.49 | 567.26 | -4.10% |
|  | LPNUE Mean | 428.93 | 96.62 | -77.47% | 384.31 | 56.56 | -85.28% | 250.49 | 31.38 | -87.47% |
|  | LPNUE 5% | 200.68 | 36.00 | -82.06% | 161.66 | 8.63 | -94.66% | 62.85 | 4.38 | -93.03% |
|  | LPNUE 50% | 498.57 | 98.82 | -80.18% | 443.71 | 52.69 | -88.13% | 222.36 | 18.43 | -91.71% |
|  | LPNUE 95% | 649.28 | 165.28 | -74.54% | 626.80 | 126.19 | -79.87% | 502.25 | 78.32 | -84.41% |
| UL Average-UPT (Mbps) | MacroUE Mean | 139.38 | 139.85 | 0.34% | 125.81 | 126.02 | 0.17% | 101.70 | 102.32 | 0.61% |
|  | MacroUE 5% | 100.13 | 100.13 | 0.00% | 84.22 | 85.71 | 1.77% | 20.63 | 23.33 | 13.09% |
|  | MacroUE 50% | 142.93 | 142.76 | -0.12% | 129.79 | 130.23 | 0.34% | 110.12 | 110.78 | 0.60% |
|  | MacroUE 95% | 158.81 | 159.93 | 0.71% | 147.67 | 147.79 | 0.08% | 142.56 | 142.03 | -0.37% |
|  | LPNUE Mean | 142.87 | 413.91 | 189.71% | 132.91 | 417.99 | 214.49% | 113.16 | 400.27 | 253.72% |
|  | LPNUE 5% | 123.09 | 246.14 | 99.97% | 108.53 | 369.20 | 240.18% | 64.04 | 353.53 | 452.05% |
|  | LPNUE 50% | 144.14 | 429.07 | 197.68% | 134.87 | 420.77 | 211.98% | 116.84 | 400.38 | 242.67% |
|  | LPNUE 95% | 158.51 | 498.95 | 214.78% | 147.86 | 455.84 | 208.29% | 141.89 | 441.14 | 210.90% |
| DL Packet-Latency CDF (ms) | MacroUE Mean | 10.67 | 10.99 | 3.00% | 12.58 | 13.86 | 10.17% | 22.51 | 26.50 | 17.73% |
|  | MacroUE 5% | 5.98 | 6.02 | 0.67% | 5.98 | 6.02 | 0.67% | 6.05 | 6.09 | 0.66% |
|  | MacroUE 50% | 6.70 | 6.80 | 1.49% | 6.95 | 9.09 | 30.79% | 13.27 | 14.38 | 8.36% |
|  | MacroUE 95% | 19.77 | 22.98 | 16.24% | 29.38 | 34.98 | 19.06% | 71.20 | 93.52 | 31.35% |
|  | LPNUE Mean | 12.12 | 61.19 | 404.87% | 14.91 | 158.30 | 961.70% | 31.36 | 318.66 | 916.14% |
|  | LPNUE 5% | 6.02 | 23.63 | 292.52% | 6.02 | 24.48 | 306.64% | 6.30 | 25.98 | 312.38% |
|  | LPNUE 50% | 7.20 | 51.91 | 620.97% | 12.38 | 79.41 | 541.44% | 19.30 | 206.55 | 970.21% |
|  | LPNUE 95% | 19.84 | 163.45 | 723.84% | 34.05 | 587.52 | 1625.46% | 99.91 | 902.34 | 803.15% |
| UL Packet-Latency CDF (ms) | MacroUE Mean | 8.03 | 7.88 | -1.87% | 10.32 | 10.09 | -2.23% | 33.24 | 32.18 | -3.19% |
|  | MacroUE 5% | 5.59 | 5.59 | 0.00% | 5.63 | 5.63 | 0.00% | 5.70 | 5.70 | 0.00% |
|  | MacroUE 50% | 6.98 | 6.98 | 0.00% | 7.34 | 7.34 | 0.00% | 9.41 | 9.45 | 0.43% |
|  | MacroUE 95% | 12.80 | 12.70 | -0.78% | 23.73 | 23.23 | -2.11% | 146.23 | 134.59 | -7.96% |
|  | LPNUE Mean | 8.91 | 3.17 | -64.42% | 9.82 | 4.33 | -55.91% | 13.49 | 2.79 | -79.32% |
|  | LPNUE 5% | 5.55 | 1.55 | -72.07% | 5.63 | 1.55 | -72.47% | 5.66 | 1.59 | -71.91% |
|  | LPNUE 50% | 6.80 | 2.55 | -62.50% | 7.13 | 2.55 | -64.24% | 7.84 | 2.59 | -66.96% |
|  | LPNUE 95% | 12.13 | 4.34 | -64.22% | 16.66 | 4.59 | -72.45% | 38.13 | 4.77 | -87.49% |
| DL RU (%) | Macro Type-1 | 3.79% | 3.89% | 2.64% | 12.35% | 13.27% | 7.45% | 36.16% | 39.55% | 9.38% |
|  | LPN Type-1 | 4.50% | 4.58% | 1.78% | 14.02% | 14.46% | 3.14% | 44.23% | 19.61% | -55.66% |
|  | Macro Type-2 | 4.74% | 5.94% | 25.32% | 15.44% | 20.27% | 31.28% | 45.20% | 60.43% | 33.69% |
|  | LPN Type-2 | 5.62% | 7.00% | 24.56% | 17.53% | 20.09% | 14.60% | 55.28% | 29.96% | -45.80% |
| UL RU (%) | Macro Type-1 | 2.64% | 2.57% | -2.65% | 6.68% | 6.53% | -2.25% | 11.98% | 11.69% | -2.42% |
|  | LPN Type-1 | 2.64% | 3.90% | 47.73% | 5.99% | 6.76% | 12.85% | 10.57% | 10.16% | -3.88% |
|  | Macro Type-2 | 13.22% | 7.45% | -43.65% | 33.38% | 18.89% | -43.41% | 59.89% | 33.83% | -43.51% |
|  | LPN Type-2 | 13.19% | 11.29% | -14.40% | 29.93% | 19.58% | -34.58% | 52.86% | 29.42% | -44.34% |
| Unfinished/dropped Packet Rate (%) | DL | 0.24% | 0.84% |  | 0.73% | 6.14% |  | 5.01% | 29.72% |  |
|  | UL | 1.67% | 0.85% |  | 0.74% | 0.28% |  | 1.63% | 1.29% |  |
| Note: |  |  |  |  |  |  |  |  |  |  |

Compared with the baseline TDD pattern, the TDD pattern for dynamic TDD Macro is the same, while the TDD pattern for dynamic TDD Indoor is updated to DSUUU. In this case, the UL resource for the Indoor office is increased around 300% and the DL resource for Indoor office is decreased around 50%. Overall, we have the following observation.

Observation 15: Regarding dynamic TDD with HetNet, Packet size 0.5Mbytes/0.125Mbytes

- For Macro layer, the DL average UPT (mean) is decreased by around 2% - 10% due to the UE-UE CLI; for indoor office, the DL average UPT (mean) is decreased by around 77% - 87% mainly due to the decreased DL resource.

- For Macro layer, the DL Packet-Latency (mean) of SBFD is increased by around 3%-18% due to the UE-UE CLI; for indoor office, the DL Packet-Latency (mean) of SBFD is increased by around 404%-916% due to the decreased DL resource.

- For Macro layer, the UL average UPT (mean) of SBFD is almost the same; for indoor office, the UL average UPT (mean) of SBFD is increased by around 189%-254% due to the increased UL resource.

- For Macro layer, the UL Packet-Latency (mean) of SBFD is almost the same; for indoor office, the UL Packet-Latency (mean) of SBFD is decreased by around 56% - 79% due to the increased UL resource.

Simulation results: Indoor

| Reported Parameters |  | 1-layer scenario for dynamic/flexible TDD: {DDDSU} vs. {FFFFF} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |
|  |  | DL: Medium, UL: Low |  |  | DL: High, UL: Medium |  |  |
|  |  | TDD | DTDD | Gain (%) | TDD | DTDD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 303.1859 | 370.6852 | 22.26 | 179.5474 | 193.0344 | 7.51 |
|  | 5% | 72.9828 | 98.3811 | 34.80 | 9.771 | 17.9638 | 83.85 |
|  | 50% | 141.8192 | 184.9711 | 30.43 | 92.4454 | 112.5536 | 21.75 |
|  | 95% | 628.0702 | 848.8381 | 35.15 | 492.09 | 568.3672 | 15.50 |
| UL Average-UPT CDF (Mbps) | Mean | 143.4609 | 485.1149 | 238.15 | 138.2368 | 239.9879 | 73.61 |
|  | 5% | 87.7865 | 146.9816 | 67.43 | 111.4242 | 30.7378 | -72.41 |
|  | 50% | 147.7394 | 526.6874 | 256.50 | 140.3251 | 229.7772 | 63.75 |
|  | 95% | 168.0976 | 651.2508 | 287.42 | 160.4459 | 490.3415 | 205.61 |
| DL Packet-Latency CDF (ms) | Mean | 23.573 | 19.7574 | -16.19 | 101.4172 | 77.6672 | -23.42 |
|  | 5% | 6.0179 | 4.5179 | -24.93 | 6.8393 | 6.4464 | -5.74 |
|  | 50% | 15.2679 | 12.875 | -15.67 | 31.8393 | 31.8036 | -0.11 |
|  | 95% | 59.0893 | 47.7321 | -19.22 | 522.7321 | 347.8036 | -33.46 |
| UL Packet-Latency CDF (ms) | Mean | 8.4442 | 7.8981 | -6.47 | 9.0367 | 68.7257 | 660.52 |
|  | 5% | 5.5893 | 1.4464 | -74.12 | 5.5893 | 1.5179 | -72.84 |
|  | 50% | 6.8393 | 1.7321 | -74.67 | 7.0179 | 13.6607 | 94.66 |
|  | 95% | 11.8393 | 26.3393 | 122.47 | 14.875 | 342.7679 | 2204 |
| Type-1 RU (%) | DL | 15.29 | 16.08 | - | 63.27 | 70.06 | - |
|  | UL | 1.32 | 1.68 | - | 3.84 | 3.59 | - |
| Type-2 RU (%) | DL | 19.11 | - | - | 79.09 | - | - |
|  | UL | 6.6 | - | - | 19.2 | - | - |
| Unfinished/dropped Packet Rate (%) | DL | 2.8986 | 3.1884 | - | 9.2958 | 10.5066 | - |
|  | UL | 0.3049 | 0.6098 | - | 0.4864 | 4.0856 | - |

Observation 16: Regarding dynamic TDD with Indoor, Packet size 0.5Mbytes/0.125Mbytes

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, DL average UPT (mean) is increased by around 22% and 7%, respectively.

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, DL latency (mean) is decreased by 16% and 23%, respectively;

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, UL average UPT (mean) is increased by 238% and 73%, respectively;

- In case of medium UL RU and high DL RU, UL latency (mean) is increased by around 660%;

Preliminary LLS results for SBFD

In RAN1#112bis-e meeting, the following agreements were reached. In our simulation, Option-1 is applied. And the example-2 is applied to obtain the  NTDD and NSBFD.

Agreement For LLS coverage evaluation, RAN1 should consider self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference in TDD system and SBFD system. - Option-1 - The modelling method is as below: - For TDD UL slot, additive white Gaussian noise with variance of NTDD is generated, where NTDD=IUE-gNB+N0 - IUE-gNB is UE-gNB interference and N0 is noise (in linear scale). - For SBFD slot, additive white Gaussian noise with variance of NSBFD is generated, where NSBFD=ISI+Ico-site+IgNB-gNB+IUE-gNB+N0 - ISI, Ico-site, IgNB-gNB, IUE-gNB are self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference (in linear scale), respectively - Companies to report the details of deriving NTDD and NSBFD. Some examples are as below: - Example-1: IUE-gNB and IgNB-gNB are derived based on a certain assumption of the topology of gNBs and UEs (ISI is derived based on 1dB desense and Ico-site is derived based on αco-site as agreed in last meeting). In this example, the interference is pre-receiver interference. - Note: link budget analysis can be applied in this example - Example-2: ∆=10*log10NSBFD-10*log10NTDD is derived based on statistic in SLS, and then ∆ is used in LLS to increase the Gaussian noise power in SBFD symbol compared to TDD UL symbol. In this example, the interference is post-receiver interference. - Example-3: NTDD and NSBFD can be derived based on statistic in SLS. In this example, the interference is post-receiver interference. - Companies to report the RU assumption for the interference. - Note: For simplicity, the interference is independently updated/generated in each slot. - Note: Companies are encouraged to report whether and how channel estimation and interference estimation will be impacted by IUE-gNB and IgNB-gNB. - Based on the modelling method, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 1: For legacy TDD system, assume the SNR in UL only slot is SNRTDD=SNTDD, perform LLS to get the required SNR (SNRTDD) with which UE can achieve a certain bit rate in UL - Step 2: For SBFD system with frame structure XXXXU, assume the SNR in UL only slot is SNRSBFD-UL=SNTDD and the SNR in SBFD slot is SNRSBFD-X=SNSBFD. Perform LLS to get the required SNR (SNRSBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD. - For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL. - For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL. - Option-2 - The UE-gNB interference and inter-site gNB-gNB co-channel inter-subband CLI in LLS coverage evaluation are explicitly modelled based on a given topology of aggressor UEs and gNBs. The UE-gNB and gNB-gNB fast fading channels are explicitly modelled in LLS. The signal model is as follows - Y=HkSk +n≠kHnSn +HgSg+HCLI,mWmSm+Iself+N - Y is the received signal vector at the victim gNB - Hk is the channel matrix from target UE to gNB, Sk is the transmitted signal of the target user - Hn, Sn, n≠k, are the channel matrix and transmitted signal of the UE in the same cell as the target user - Hg and Sg are the channel matrix and transmitted signal of the UEs in the adjacent cell - HCLI,m, Wm and Sm are the channel matrix, the precoding matrix, and leakage CLI signal from aggressor gNB m to the victim gNB. - The power of the signal and interference is included in the channel marix respectively - Iself and N are the self-interference vector of the co-site sectors and the thermal noise signal vector on the receiving antennas - Companies to report the topology of gNBs and UEs to derive the detailed signals and interferences above. One example is as below - Based on the above modelling, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 1: For legacy TDD system, perform LLS to get the required SNR (SNRTDD=SN0) with which UE can achieve a certain bit rate in UL - Step 2: For SBFD system with frame structure XXXXU, perform LLS to get the required SNR (SNRSBFD-UL=SN0) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD. - For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL. - For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL.

Based on example-2 below, the ∆=10*log10NSBFD-10*log10NTDD in our simulation in different traffic load is observed as following.

|  | Low Road | Medium Load | High Load |
| --- | --- | --- | --- |
| Post MMSE ∆ (dB) | 0.14 | 0.48 | 2.19 |

After performing the LLS, the LLS curve and target SNR (BLER=0.1) are summarized as following.

| Case | Target SNR (BLER = 0.1) |
| --- | --- |
| Legacy TDD | -4 |
| Repetition (Low load) | -9.7 |
| Repetition (Medium load) | -9.45 |
| Repetition (High load) | -7.94 |
| Repetition with JCE (Low load) | -10.9 |
| Repetition with JCE (Medium load) | -10.53 |
| Repetition with JCE (High load) | -8.55 |

The detailed simulation assumptions can be found in the Appendix.  The MCL/MIL/MPL results are as following.

Observation 17: The LLS result for PUSCH-FR1-Urban Macro coverage is as following.

| PUSCH-FR1-Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company name | TDD/SBFD | Required SNR | MCL | MIL | MPL | Key assumptions |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Low load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -9.70 | 136.44 | 145.21 | 114.48 |  |
|  | Gain | 5.70 | 5.20 | 5.20 | 5.20 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Medium load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -9.45 | 136.19 | 144.96 | 114.23 |  |
|  | Gain | 5.45 | 4.95 | 4.95 | 4.95 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | High load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -7.94 | 134.68 | 143.45 | 112.72 |  |
|  | Gain | 3.94 | 3.44 | 3.44 | 3.44 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Low load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -10.90 | 137.64 | 146.41 | 115.68 |  |
|  | Gain | 6.90 | 6.40 | 6.40 | 6.40 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Medium load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -10.53 | 137.27 | 146.04 | 115.31 |  |
|  | Gain | 6.53 | 6.03 | 6.03 | 6.03 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | High load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -8.55 | 135.29 | 144.06 | 113.33 |  |
|  | Gain | 4.55 | 4.05 | 4.05 | 4.05 |  |

Conclusion

## SBFD prototype

Observation 1: The 1st SBFD prototype achieves more than 130dB self-interference suppression capability (55 dB antenna isolation, 45 dB ACLR and more than 30 dB sub-band filtering and digital cancellation) and less than 1dB receiver sensitivity degradation at the gNB side.

Observation 2:

- The first prototype verifies the SBFD feasibility and achieves 3.9ms E2E round trip latency on average and up to 1.4Gbps peak UL data rate with 4T4R TUE.

- The second prototype verifies that legacy commercial UEs supporting flexible symbols are compatible to the SBFD base station. The peak UL data rate is higher than 700Mbps and the E2E round trip latency is around 4ms for commercial UE with 2T4R.

Proposal 1: Capture the SBFD prototype info in section 2 of R1-2304595 into TR38.858.

## Preliminary SLS results for SBFD

Observation 3: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 24% - 31% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 32%-69% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the larger DL Packet-Latency (mean) of SBFD due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 33% - 48% due to the increased UL resource. The gain of UL average UPT (mean) of SBFD is increased as the increase of traffic load because the UL average UPT (mean) of baseline TDD is decreased as the increase of traffic load due to the limited UL resource in the baseline TDD.

- The UL Packet-Latency (mean) of SBFD is decreased by around 16% - 54% due to the increased UL resource. The gain of UL Packet-Latency (mean) of SBFD is increased as the increase of traffic load because the UL Packet-Latency (mean) of baseline TDD is increased as the increase of traffic load due to the limited UL resource in the baseline TDD.

Observation 4: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.4 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 4% - 10% due to the UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 4%-8% in case of low and medium traffic load due to UE-UE CLI. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 54% due to the much more serious UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 9% - 15% due to more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 11% - 17% in case of low and medium traffic load. In case of high traffic load, the UL Packet-Latency (mean) of SBFD is increased by around 45% due to the much more serious gNB CLI.

Observation 5: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium traffic load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 15% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 52% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 24% - 27% due to the increased UL resource.

Observation 6: Regarding SBFD deployment case1, FR1 Indoor office, SBFD Alt.4 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is increased by around 11% - 13% due to increased transmission occasion in the last slot of each TDD period.

- The DL Packet-Latency (mean) of SBFD is decreased by around 16%-17% due to increased transmission occasion in the last slot of each TDD period.

- The UL average UPT (mean) of SBFD is increased by around 107% - 129% due to increased transmission occasion in the first four slots of each TDD period.

- The UL Packet-Latency (mean) of SBFD is decreased by around 61% - 66% due to increased transmission occasion in the first four slots of each TDD period.

Observation 7: Regarding SBFD deployment case1, FR1 Indoor office,

- In case of larger packet size, obvious DL UPT loss is observed due to the reduced DL resource; in case of smaller packet size, small or no DL UPT loss is observed.

- The UL UPT gain in case of smaller packet size is larger than that of larger packet size.

Observation 8: Regarding SBFD deployment case1, FR1 Urban Macro, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is decreased by around 19% - 29% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI. The loss of DL average UPT (5%) SBFD is much higher than that of DL average UPT (mean) since UE with poor coverage (e.g., cell edge UE) experiences more serious UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 29%-145% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the larger DL Packet-Latency (mean) of SBFD due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 63% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

- The UL Packet-Latency (mean) of SBFD is decreased by around 3% - 19% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

Observation 9: Regarding SBFD deployment case1, FR1 Urban Macro, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low load. In case of medium and high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 114%-180% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 43% - 70% due to the increased UL resource. The gain of UL average UPT (mean) of SBFD is increased as the increase of traffic load because the UL average UPT (mean) of baseline TDD is decreased as the increase of traffic load due to the limited UL resource in the baseline TDD.

- The UL Packet-Latency (mean) of SBFD is decreased by around 8% - 62% due to the increased UL resource.

Observation 10: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes,

- The DL average UPT (mean) of SBFD is decreased by around 18% - 28% due to the decreased DL resource and UE-UE CLI.

- The DL Packet-Latency (mean) of SBFD is increased by around 26%-88% due to the decreased DL resource and UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 23%-28% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 6% - 14% due to the increased UL resource.

Observation 11: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.4 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- The DL average UPT (mean) of SBFD is almost the same as legacy TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as legacy TDD.

- The UL average UPT (mean) of SBFD is increased by around 17% - 20% due to more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 19% - 24% in case of low and medium traffic load due to more transmission occasions in time domain. In case of high traffic load, the UL Packet-Latency (mean) of SBFD is decreased by around 6%.

Observation 12: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.2 subband pattern, Packet size 5Kbytes/1Kbytes,

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 12% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 22% - 27% due to the increased UL resource.

- The UL Packet-Latency (mean) of SBFD is decreased by around 13% - 15% due to the increased UL resource.

Observation 13: Regarding SBFD deployment case1, FR1 Dense Urban Macro, SBFD Alt.4 subband pattern, Packet size 5Kbytes/1Kbytes,

- The DL average UPT (mean) of SBFD is almost the same as baseline TDD.

- The DL Packet-Latency (mean) of SBFD is almost the same as baseline TDD in case of low and medium load. In case of high traffic load, the DL Packet-Latency (mean) of SBFD is increased by around 11% due to the UE-UE CLI.

- The UL average UPT (mean) of SBFD is increased by around 22% - 27% due to the more transmission occasions in time domain.

- The UL Packet-Latency (mean) of SBFD is decreased by around 13% - 15% due to the more transmission occasions in time domain.

Observation 14: Regarding SBFD deployment case1, FR1 Dense Urban (2-layer scenario), SBFD Alt.2 subband pattern, Packet size 0.5Mbytes/0.125Mbytes

- For both Macro layer and Micro layer, the DL average UPT (mean) of SBFD is decreased by around 30% - 39% due to the decreased DL resource and UE-UE CLI. The higher traffic load, the higher loss of DL average UPT (mean) of SBFD due to the UE-UE CLI.

- For Macro layer, the DL Packet-Latency (mean) of SBFD is increased by around 48%-127% due to the decreased DL resource and UE-UE CLI; for Micro layer, the DL Packet-Latency (mean) of SBFD is increased by around 57%-93% due to the decreased DL resource and UE-UE CLI.

- For Macro layer, the UL average UPT (mean) of SBFD is increased by around 10% due to the increased UL resource; for Micro layer, the UL average UPT (mean) of SBFD is increased by around 14% due to the increased UL resource.

- For Macro layer, the UL Packet-Latency (mean) of SBFD is decreased by around 16% - 43% due to the increased UL resource; for Micro layer, the UL Packet-Latency (mean) of SBFD is decreased by around 14% - 32% due to the increased UL resource. The gain is smaller in case of high traffic load due to the gNB CLI.

## Preliminary SLS results for dynamic TDD

Observation 15: Regarding dynamic TDD with HetNet, Packet size 0.5Mbytes/0.125Mbytes

- For Macro layer, the DL average UPT (mean) is decreased by around 2% - 10% due to the UE-UE CLI; for indoor office, the DL average UPT (mean) is decreased by around 77% - 87% mainly due to the decreased DL resource.

- For Macro layer, the DL Packet-Latency (mean) of SBFD is increased by around 3%-18% due to the UE-UE CLI; for indoor office, the DL Packet-Latency (mean) of SBFD is increased by around 404%-916% due to the decreased DL resource.

- For Macro layer, the UL average UPT (mean) of SBFD is almost the same; for indoor office, the UL average UPT (mean) of SBFD is increased by around 189%-254% due to the increased UL resource.

- For Macro layer, the UL Packet-Latency (mean) of SBFD is almost the same; for indoor office, the UL Packet-Latency (mean) of SBFD is decreased by around 56% - 79% due to the increased UL resource.

Observation 16: Regarding dynamic TDD with Indoor, Packet size 0.5Mbytes/0.125Mbytes

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, DL average UPT (mean) is increased by around 22% and 7%, respectively.

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, DL latency (mean) is decreased by 16% and 23%, respectively;

- In case of low UL RU and medium DL RU and medium UL RU and high DL RU, UL average UPT (mean) is increased by 238% and 73%, respectively;

- In case of medium UL RU and high DL RU, UL latency (mean) is increased by around 660%;

## Preliminary LLS results for SBFD

Observation 17: The LLS result for PUSCH-FR1-Urban Macro coverage is as following.

| PUSCH-FR1-Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company name | TDD/SBFD | Required SNR | MCL | MIL | MPL | Key assumptions |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Low load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -9.70 | 136.44 | 145.21 | 114.48 |  |
|  | Gain | 5.70 | 5.20 | 5.20 | 5.20 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Medium load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -9.45 | 136.19 | 144.96 | 114.23 |  |
|  | Gain | 5.45 | 4.95 | 4.95 | 4.95 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | High load 30RB, MCS=4, Repetition#=5, 192 antenna elements. |
|  | SBFD | -7.94 | 134.68 | 143.45 | 112.72 |  |
|  | Gain | 3.94 | 3.44 | 3.44 | 3.44 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Low load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -10.90 | 137.64 | 146.41 | 115.68 |  |
|  | Gain | 6.90 | 6.40 | 6.40 | 6.40 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | Medium load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -10.53 | 137.27 | 146.04 | 115.31 |  |
|  | Gain | 6.53 | 6.03 | 6.03 | 6.03 |  |
| ZTE | TDD | -4.00 | 131.24 | 140.01 | 109.28 | High load 30RB, MCS=4, Repetition#=5, JCE Option 1 192 antenna elements. |
|  | SBFD | -8.55 | 135.29 | 144.06 | 113.33 |  |
|  | Gain | 4.55 | 4.05 | 4.05 | 4.05 |  |

Reference

- RP-220633, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN#95-e meeting.

- Interference Mitigation for Non-Overlapping Sub-Band Full Duplex for 5G-Advanced Wireless Networks, IEEE Access, https://ieeexplore.ieee.org/document/9992227

Appendix

Simulation assumptions for SLS SBFD

Table.7-1: Simulation parameters for SBFD

| Parameters |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario |  | Indoor office | Urban Macro | Dense Urban Macro layer | Dense Urban 2 layer |
| Layout |  | Indoor office size 120x50 m Single layer: 12BSs per 120m x 50m | Macro layer: Hex. Grid As a layout of macro cell, 7 macro sites, 3 sectors per site model with wrap around | Macro layer: Hex. Grid As a layout of macro cell, 7 macro sites, 3 sectors per site model with wrap around | Macro Layer: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around Micro layer: Micro TRP number 3 within one macro cell geographical area |
| Numerology |  | 14 OFDM symbol slot SCS = 30kHz |  |  |  |
| Inter-BS distance |  | 20m | 500m | 200m | Macro-to-Macro: 200m Minimum Macro-to-micro-center distance: 42m Minimum Micro-center-to-micro-center distance: 40m |
| Carrier frequency |  | 4GHz |  |  |  |
| Simulation bandwidth |  | 100 MHz |  |  |  |
| BS Tx power |  | 24 dBm per 100MHz | 53dBm per 100MHz | 44dBm per 100MHz | Macro layer: 44dBm per 100MHz Micro layer: 30 dBm for 100MHz |
| UE Tx power |  | 23dBm |  |  |  |
| Frame structure |  | Legacy TDD DDDSU, S=[12D:2G:0U] SBFD: XXXXU and XXXXX S = [(subband-1:D; subband-2:U; subbband-3:D] , 273RBs, 104:55:104(DUD),- Guard RB: 5RBs in each side 1symbol gap between S and U symbol. |  |  |  |
| TxRU mapping |  | Per panel, reuse models in TR 36.897. Option 1: a single TXRU is mapped per panel per polarization. |  |  |  |
| BS antenna configuration |  | Legacy TDD: - = (4,4,2,1,1; 4,4) - = (0.5,0.5)λ, +45°/-45° polarization; SBFD: SBFD antenna configuration option-2 (Method2- 1) - Two panel groups - For each panel group: = (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | Legacy TDD: - =(8,8,2,1,1;2,8) - = (0.5, 0.8)λ, +45°/-45° polarization; SBFD: SBFD antenna configuration option-2 (Method2- 1) - Two panel groups - For each panel group: = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |  | Legacy TDD: - =(8,8,2,1,1;2,8) - = (0.5, 0.8)λ, +45°/-45° polarization; SBFD: SBFD antenna configuration option-2 (Method2- 1) - Two panel groups - For each panel group: = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna height |  | 3m | 25m |  | 25m |
| BS antenna radiation pattern |  | Reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (same as Wall-mount model in Table A.2.1-7 in TR 38.802) | Reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) |  | Reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) |
| BS receiver noise figure |  | 5dB for 4GHz |  |  |  |
| UE antenna configuration |  | For 4GHz: 2 Tx/4 Rx antenna ports 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |  |  |
| UE antenna radiation pattern |  | Omni-directional with 0 dBi element gain |  |  |  |
| UE receiver noise figure |  | 9 dB for 4GHz |  |  |  |
| UE power control |  | P0= -60; alpha = 0.6 | P0 = -86, alpha =0.8 |  |  |
| Minimum BS-UE (2D) distance |  | 0m | 35m |  | Macro-to-UE: 35m Micro-to-UE: 10m |
| Minimum UE-UE (2D) distance |  | 1m |  |  |  |
| UE cluster number per macro cell (X) |  | - | 2 | 2 | 3 |
| UE outdoor/indoor proportion |  | 100% indoor in houses: 3km/h | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |
| Indoor UE height |  | 1.5m | 1.5m | 1.5m | Outdoor UE : 1.5 m Indoor UE : 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
| Radius of cluster (R) |  | - | 25m | 20m | 20m |
| Minimum distance between macro TRP to UE cluster center |  | - | 60m | 55m | 35m |
| Minimum distance between two UE cluster centers |  | - | 50m | - | 40m |
| UE density |  | 10 UEs per TRxP | 20 UEs per TRxP | 20 UEs per TRxP | 20UEs per Macro area |
| UE receiver |  | MMSE-IRC |  |  |  |
| Feedback assumption |  | Realistic |  |  |  |
| Channel estimation |  | Ideal |  |  |  |
| UE processing capability |  | UE processing capability 2 |  |  |  |
| UE attachment |  | Based on RSRP from port 0 |  |  |  |
| Polarized antenna model |  | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |
| DL/UL Modulation |  | Up to 64QAM |  |  |  |
| Handover margin |  | 3 dB |  |  |  |
| Transmission scheme and Scheduling |  | SU-MIMO with PF |  |  |  |
| gNB-gNB co-channel inter-subband | Co-site inter-sector | The same assumption as self-interference suppression. |  |  |  |
|  | Inter-site | TX leakage: gNB ACLR = 45dB Receiver impairment: gNB ACS=46dB Tx and Rx isolation: path loss between the aggressor gNB and victim gNB |  |  |  |
| UE-UE co-channel inter-subband |  | IBE model is applied. TX leakage: UE ACLR = 30dB Receiver impairment: UE ACS= 33dB |  |  |  |

Simulation assumptions for dynamic SLS TDD

Table.7-2: Simulation parameters for dynamic TDD

| Parameters | Value | Value |
| --- | --- | --- |
| Scenario | 2 Layer Scenario B: Macro and Indoor office | 1 Layer Scenario: Indoor office |
| Layout | Layer 1: Urban Macro - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m Layer 2: Indoor office (baseline) - Only one building randomly dropped in the whole network as in the figure below. The building has to be confined within one macro cell area. - 12 TRPs per 120m x 50m x 3m - the distance between two indoor TRPs: 20m for 12 TRPs | Indoor office size 120x50 m Single layer: 12BSs per 120m x 50m |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 30kHz |
| Inter-BS distance | Macro-to-macro: 500m Minimum Macro-to-indoor center distance: 100m Minimum Indoor-to-indoor distance: 20m | 20m |
| Carrier frequency | 4GHz | 4GHz |
| Simulation bandwidth | 100 MHz | 100 MHz |
| BS Tx power | 53dBm for Macro 24dBm for indoor | 24dBm for indoor |
| UE Tx power | 23dBm | 23dBm |
| Frame structure | Baseline operation for comparison: legacy static TDD {DDDSU} based on Rel-17 specifications Target flexible TDD operation:legacy static TDD {DSUUU} based on potential enhancements | Baseline operation for comparison: legacy static TDD {DDDSU} based on Rel-17 specifications Dynamic TDD operation:{FFFFF} |
| TxRU mapping | Per panel, reuse models in TR 36.897. Option 1: a single TXRU is mapped per panel per polarization. | Per panel, reuse models in TR 36.897. Option 1: a single TXRU is mapped per panel per polarization. |
| BS antenna configuration | Layer 1: - =(8,8,2,1,1;2,8) - = (0.5, 0.8)λ, +45°/-45° polarization; Layer 2: - = (4,4,2,1,1; 4,4) - = (0.5,0.5)λ, +45°/-45° polarization; | - M,N,P,Mg,Ng;Mp,Np= (4,4,2,1,1; 4,4) dH,dV= (0.5,0.5)λ, +45°/-45° polarization; |
| BS antenna height | Macro: 25m indroor: 3m | 3m |
| BS antenna radiation pattern | Layer 1: Reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) Layer 2: Reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (same as Wall-mount model in Table A.2.1-7 in TR 38.802) | Reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (same as Wall-mount model in Table A.2.1-7 in TR 38.802) |
| BS receiver noise figure | 5dB for 4GHz | 5dB for 4GHz |
| UE antenna configuration | For 4GHz: 2 Tx/2 Rx antenna ports Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) | For 4GHz: 2 Tx/2 Rx antenna ports Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | Omni-directional with 0 dBi element gain |
| UE receiver noise figure | 9 dB for 4GHz | 9 dB for 4GHz |
| UE power control | P0 = -86, alpha =0.8 for Macro P0 = -60, alpha =0.6 for Indoor office | P0 = -60, alpha =0.6 for Indoor office |
| Minimum BS-UE (2D) distance | Macro-to-UE: 35m Indoor-to-UE: 0m | 0m |
| Minimum UE-UE (2D) distance | 1m (TR38.828) | 1m (TR38.828) |
| UE density | Layer 1: Urban Macro - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell outside the Indoor office - Indoor/outdoor proportion: - Option 1 (baseline): 100% outdoor without car penetration loss, 3km/h, UE height is 1.5m - Layer 2: Indoor office (baseline) - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m | - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. UE speed is 3km/h, UE height is 1.5m |
| UE receiver | MMSE-IRC | MMSE-IRC |
| Feedback assumption | Realistic | Realistic |
| Channel estimation | Ideal | Ideal |
| UE processing capability | UE processing capability 2 | UE processing capability 2 |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0 |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 | Model-1 in clause 7.3.2 in TR 38.901 |
| DL/UL Modulation | Up to 64QAM | Up to 64QAM |
| Handover margin | 3dB | 3dB |
| Transmission scheme and Scheduling | SU-MIMO with PF | SU-MIMO with PF |
| gNB2gNB Large-scale channel parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP | FR1: - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). Penetration loss is not modelled. |
| gNB2gNB Fast fading parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP | FR1: - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. |
| UE2UE Large-scale channel parameters | FR1: - Outdoor UE to Outdoor UE: - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 | FR1: - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). Penetration loss is not modelled. |
| UE2UE Fast fading parameters | FR1: - Outdoor UE to Outdoor UE: - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| gNB2UE Large-scale channel parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Outdoor UE: - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. | FR1: - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 Penetration loss is not modelled. |
| gNB2UE Fast fading parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 | FR1: - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building For Indoor office layer: InH-Office in TR 38.901 |

Simulation assumptions for LLS SBFD

| Parameters | Value |
| --- | --- |
| Frame structure | DDDSU |
| Fc | 4GHz |
| SCS | 30kHz |
| BW | 100MHz |
| Channel | TDL-C |
| Delay spread | 300ns |
| UE speed | 3km/h |
| UE Tx anntenna | 1 |
| gNB Rx antenna | 4 |
| Waveform | DFT-s-OFDM |
| PUSCH duration | 14 OS |
| DMRS | 2OS |
| Frequency hopping | disable |
| RB Num | 30 |
| MCS | 4 |
| TBS | 2600 |
| Actual data rate | 1.04 Mbps |
| HARQ configuration | disable |
| Target iBLER | 10% |