3GPP TSG RAN WG1 #113			R1-2304702

Incheon, Korea, 22nd – 26th May, 2023

# Source:             CATT

Title:    	Discussion on RAN2 LS on L1 signaling for cell DTX/DRX activation/deactivation

Agenda Item:    5

Document for:  Discussion and Decision

# Introduction

RAN2 had sent an LS in R1-2304334 (R2-2306618) on the L1 signaling for the activation/deactivation of cell DTX/DRX [1] . RAN2 had agreed on the framework of RF evaluation of LP-WUS.  RAN2 agreed to have cell DTX/DRX configuration indication to the UE by RRC signaling in RAN2#121bis-e.  The cell DTX/DRX is activated/deactivated at least by RRC signaling after the configuration of cell DTX/DRX.   RAN2 asked RAN1 to provide information regarding feasibility and reliability of using dedicated and group common L1 signalling for Cell DTX/DRX activation and deactivation.   In this paper, we discuss the benefit of L1 signaling for Cell DTX/DRX activation/deactivation.

# Discussion of L1 Signaling for Cell DTX/DRX Activation/Deactivation

The framework of cell DTX/DRX configuration and activation/deactivation by UE specific RRC signaling were agreed in RAN2#121bis-e included in the LS as follows,

Agreement in RAN2#121bis-e

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

In RAN2 LS [1],  majority of companies from RAN2 see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation with the benefits as follows,

- Reduced signaling overhead caused by multiple dedicated RRC messages (group common signaling)

- More dynamic changing than RRC signaling (however, RAN2 did not evaluate the network energy saving gain by reducing the latency of activation/deactivation with L1 signaling for more dynamic changing)

In RAN2 LS [1],  RAN2 asks RAN1 to provide information regarding feasibility and reliability of using dedicated and group common L1 signalling for Cell DTX/DRX activation and deactivation for a single configuration with the design details.

In RAN1#112bis-e, RAN1 had discussed the L1 signaling for activation/deactivation of cell DTX/DRX with the working assumption being made than the L1 signaling is considered feasible from RAN1 perspective as follows,

Working Assumption

Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

- This does not imply that L1 activation/deactivation is supported in Rel-18\

- Note: Reliability, overhead, and benefits are FFS

While the cell DTX/DRX is activated or deactivated by UE specific RRC signaling, the cell DTX/DRX can be activated immediately once configured by RRC message and deactivated by RRC released messages. Furthermore, there may be additional indication for activation or deactivation of cell DTX/DRX in RRC message. Since the activation or deactivation of cell DTX/DRX is not so dynamic procedure and cell DTX/DRX is applied in the scenario of low system load. The latency of RRC configuration or reconfiguration and the corresponding RRC signaling overhead are acceptable. The gNB can transmit reliable RRC signaling for activation or deactivation of cell DTX/DRX when the gNB wants to perform or stop network energy saving mode.

The configuration of DTX/DRX is provided and activated/deactivated as default by RRC signaling.   The L1 signaling of the cell DTX/DRX would provide the additional triggering mechanism of cell DTX/DRX activation/deactivation, which is similar to the procedure of SPS and Type 2 CG. The advantage of L1-based cell DTX/DRX activation/deactivation is the dynamic indication and shorter response time of the activating/deactivating cell DTX/DRX for the robust traffic arrival. The L1 signaling of cell activation/deactivation can be UE-specific DCI or group common DCI.

The cell DTX/DRX is applied to all UEs in a cell. Group common L1 signaling can be used to expedite the transition and reduce the overhead of both L1 signaling and RRC signaling to efficiently and dynamically indicate the cell DTX/DRX activation/deactivation to the UE when the cell DTX/DRX had been configured to the UE. However, UEs need to frequently monitor the common search space of PDCCH at the configured PDCCH monitoring occasions to determine whether the cell TX/DRX is activated or deactivated. A dedicated search space of group common signaling for cell DTX/DRX configuration could be configured for UEs to minimize the number of UE PDCCH monitoring occasions and to optimize the system performance in dynamic activation/deactivation based on the estimated traffic arrival pattern at any instance.

- Proposal 1: The L1 signaling for cell DTX/DRX could use the group common L1 signaling for the indication to all UEs within the cell.    A dedicated search space of group common signaling for cell DTX/DRX configuration could be configured for UEs to minimize the number of UE PDCCH monitoring occasions and to optimize the system performance in dynamic activation/deactivation based on the estimated traffic arrival pattern at any instance.

The UE-specific L1 signaling could be used for the indication of cell DTX/DRX activation/deactivation when the traffic is delay insensitive and traffic arrival is very sparse in time.    The UE-specific L1 signaling could be used to indicate the change of cell DTX/DRX from activation to deactivation to the specific UEs with traffic arrival within the cells. Those UEs with data to be scheduled being indicated to the deactivation of cell DTX/DRX would resume the normal PDCCH monitoring and measurements.   Since the deactivation of cell DTX/DRX would enable UE continuous network access and measurement, other UEs without receiving UE-specific L1 signaling of cell DTX/DRX activation/deactivation would continue to assume the cell DTX/DRX being activated regardless all DL signals/channels are transmitted or UL signals/channel are received during the inactive period of cell DTX/DRX.    The UE-specific L1 signaling would be used to re-activate the cell DTX/DRX after each UE completes its data reception/transmission.    The UE-specific L1 signaling for cell DTX/DRX activation/deactivation is used to indicate to the individual UE(s) within the cell in receiving or transmitting data without changing the cell DTX/DRX status.

- Proposal 2: The UE-specific L1 signaling for cell DTX/DRX activation/deactivation is used in the sparse traffic arrival scenario to indicate to the individual UE(s) within the cell in receiving or transmitting data without changing the cell DTX/DRX status.

# Conclusion

We have discussed the advantage and design detail of L1 signaling for cell DTX/DRX activation/deactivation.  The advantages of group common and UE-specific L1 signaling for cell DTX/DRX activation/deactivation had been illustrated for different deployment scenarios.  The following proposals are suggested to be included in the reply to RAN2 LS for the feasibility and overhead of L1 signaling for cell DTX/DRX activation/deactivation,

- Proposal 1: The L1 signaling for cell DTX/DRX could use the group common L1 signaling for the indication to all UEs within the cell.    A dedicated search space of group common signaling for cell DTX/DRX configuration could be configured for UEs to minimize the number of UE PDCCH monitoring occasions and to optimize the system performance in dynamic activation/deactivation based on the estimated traffic arrival pattern at any instance.

- Proposal 2: The UE-specific L1 signaling for cell DTX/DRX activation/deactivation is used in the sparse traffic arrival scenario to indicate to the individual UE(s) within the cell in receiving or transmitting data without changing the cell DTX/DRX status.

# Reference

- R1-2304334, “LS on Cell DTX/DRX activation/deactivation”, RAN2, Huawei

- R1-2302717, ”DTX/DRX for network Energy Saving”, CATT