3GPP TSG RAN WG1 #113                                                                                            R1- 2304714

Incheon, Korea, May 22nd – May 26th, 2023

Source:                 CATT

Title:	Remaining issues of deployment scenarios and evaluation methodologies and preliminary performance results of LP-WUR

Agenda Item:	9.11.1

Document for:	Discussion and Decision

Introduction

In RAN1#112bis-e [1], the evaluation performance, traffic model and power model were discussed for low-power wakeup receiver (LP-WUR) and signal (LP-WUS). Some agreements were made as follows:

Agreement Update as followings for the e-DRX paging probability Note: - For i-DRX with cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW - L=4 Agreement Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model. - Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 Working Assumption For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), - Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal R1-2304150 FL summary #2 of evaluation methodologies on LP-WUS/WUR Moderator (vivo) Agreement The period of synchronization signal that LP-WUR used for at least power evaluation can be - Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR - For evaluations assuming LP-SS - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. Working Assumption For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design. Agreement RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Option #1: PDCCH for paging - Option #2: PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels, e.g., the message size for MSG3 and etc. Agreement Confirm Alt 2 in the following agreement and update as follows Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

This contribution discusses remaining issues of evaluation methodologies and the preliminary evaluation results of UE power saving and coverage performance for LP-WUR/LP-WUS.

Remaining issues of evaluation methodologies of LP-WUS/R

For the evaluation methodologies, several issues are remained after RAN1#112bis [1], such as the procedure of LP-WUR/LP-WUS for RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode UEs, analysis for group paging rate formula, power model for LP-WUR and re-sync duration for main radio (MR), which would be discussed in this section.

- 2.1 The procedure of LP-WUS/R

- 2.1.1 RRC_IDLE/INACTIVE mode UEs

In Rel-17, the paging early indication (PEI) by DCI format 2_7 was introduced to indicate whether RRC_IDLE /INACTIVE mode UE should decode the paging DCI and subsequent paging message at the configured paging occasion. However, the preparation and the coherent detection/decoding of the DCI format 2_7 on PDCCH for PEI periodically before the PO (Paging Occasion) still requires large UE power consumption. The LP-WUS is considered as the candidate to further reduce the UE power consumption in preparation and detection of the wakeup signals. Two schemes of LP-WUS designed with different functions are discussed as follows:

Scheme 1: LP-WUS in place of PEI and triggering the detection of PO

Figure 1 shows that the LP-WUS is transmitted for waking up the target UE as the PEI when data arrival at gNB. Once the LP-WUS is detected by the LP-WUR, the MR is triggered to transition from the ultra-deep sleep state into the active state. The MR needs to acquire time and frequency synchronization from SSB/TRS and calibrate the drift of local clock and oscillator caused by long sleep time. After accomplishing the time and frequency synchronization, the MR can perform the coherent detection of the paging DCI in the time of first PO where UE is configured to monitor. The LP-WUS in place of DCI format 2_7 can save significant UE power, since the LP-WUS only consume 1/10000 power of DCI format 2_7 (DCI-based PEI) for UEs in RRC_IDLE/INACTIVE mode.

Figure 1: The procedure of LP-WUS replacing PEI

Proposal 1:  The LP-WUS used as the paging early indication (PEI) only consume 1/10000 power of DCI format 2_7 (DCI-based PEI) for UEs in RRC_IDLE/INACTIVE mode.

Scheme 2: LP-WUS in place of PO indication

Figure 2 shows another design alternative of LP-WUS procedure for RRC_IDLE/INACTIVE UEs, which the LP-WUS replaces the function of both PEI and Paging as the direct paging indication to the UE. When the LP-WUR receives the LP-WUS for the specific UE, it is a paging indication without any paging information and the MR would be activated from ultra-deep sleep state. The MR would perform the RACH procedure for network access after acquiring time and frequency synchronization. In this scheme, the MR would use the LP-WUS as the paging indication and skip both PEI occasion and decoding of paging information at the paging occasion.

If the LP-WUS is used in place of paging indication, the LP-WUS needs carry additional paging information such as small message, SI change indication, ETWS and UE-specific information, in addition to the wakeup indication, not only in the camping cell but also in the registration area. The LP-WUS design needs to be able to support the contents of one-stage paging indication and paging information. In addition, the LP-WUS design needs to consider the length of LP-WUS, the interference from other Cell and UE (group), and the detection performance. These aspects would bring tremendous challenges for LP-WUS design and the power saving at the LP-WUR.

Figure 2: The procedure of LP-WUS replacing PO

In conclusion, the comparison between two schemes of LP-WUS procedure under power saving effect, information carried by the LP-WUS, payload of LP-WUS and specification impact are summarized in the Table 1.

Table 1: The comparison between Scheme 1 and Scheme 2

|  | Scheme 1: LP-WUS in place of PEI and triggering the detection of PO | Scheme 2: LP-WUS in place of PO indication |
| --- | --- | --- |
| Power Saving Effect | Power saving of PEI detection with LP-WUR in place of DCI format 2_7. | Power Saving without additional detecting PEI, Paging PDCCH and associated Paging information on PDSCH. |
| Information carried by the LP-WUS | Wakeup information | Wakeup information SI change ETSW UE-specific information |
| Specification Impact | PEI | PEI Paging PDCCH Paging information on PDSCH |

Proposal 2:  LP-WUS is used in place of the PO indication is not appropriate because it requires UE-specific LP-WUS to carry the paging indication for the group of UEs and associated paging information with unique ID in the registration area.

- 2.1.2 RRC_CONNECTED mode UEs

In Rel-16/17, the wakeup signals/channels had been designed for C-DRX adaptation to further reduce UE power consumption by sending a wakeup signal (WUS) before the DRX ON duration to indicate whether UE needs to wake up at the configured DRX ON cycle.

The DRX adaptation with UE wakeup by DCI format 2_6 for RRC_CONNECTED mode UEs was introduced in Rel-16 UE power saving by indication of UE wakeup only there is a DL traffic arrival at each DRX cycle to achieve UE power saving. The DCI format 2_6 is used to indicate whether UE needs to wake up at each DRX cycle to minimize the unnecessary PDCCH monitoring during DRX ON when there is no data for the given RRC_CONNECTED mode UEs. The UE wakeup mechanism by DCI format 2_6 for RRC_CONNECTED mode UEs provides a periodic indication to the UE wakeup or not to adapt at the next PDCCH monitoring occasion at DRX ON interval occasion to achieve UE power saving. However, the preparation and the coherent detection/decoding of the DCI format on PDCCH for wakeup indication periodically before the DRX still require large UE power consumption.

Figure 3 shows LP-WUS in place of the DCP in indicating the UE wakeup to the active state.  Due to the ultra-low power consumption of detecting LP-WUS compared to the preparation and detection of DCP, the LP-WUS as the wakeup indication in place of DCI format 2_7 can save significant UE power for RRC_CONNECTED mode UEs. Moreover, it can also be applied to the UEs without DRX configuration as the application of DCI format 2_7 would be restricted by DRX.

Figure 3: Rel-18 LP-WUS replace DCP

Proposal 3:  The LP-WUS as the wakeup indication could reduce detection power consumption of DCI format 2_6 and SSB. Moreover, it can also be applied to the UEs without DRX configuration as the application of DCI format 2_7 restricted by DRX.

- 2.3 Analysis of group paging formula

Considering the limited capacity carried by LP-WUS, the LP-WUS is better to design to carry the UE-group information rather than UE-specific information. Based on the updated RRC_IDLE/INACTIVE traffic model agreed in RAN#112bis [1], the paging group rate is derived based on the number of UE in the group and per UE paging rate. In this section, we discuss group paging rate for UE group for i-DRX and e-DRX, respectively.

The wakeup indication for a UE group can relax the payload size of the LP-WUS compared with the UE-specific indication in the LP-WUS design. However, the paging group indication would increase the probability of false alarm by falsely wake-up some UEs within the group without actually being paged, which will increase the unnecessary UE power consumption. The more UEs grouped with the same wakeup indication, the higher paging group rate with higher false alarm would be occurred caused by the grouping method. Thus, there is a trade-off between the unnecessary wakeup for UE grouping method and information payload of LP-WUS. The paging group rate with different number of UEs under constant paging UE rate are analysed as following.

- Analysis of paging group rate for i-DRX

Based on the agreement of evaluation assumption for i-DRX and the paging group rate formula in previous meeting, the paging group rate with different numbers of UE per group and different paging rate can be obtained as shown in Table 2.

Table 2: The paging group rate for 4~32 number of UE groups for i-DRX

|  | 1% paging rate | 0.1% paging rate | 0.01% paging rate | 0.001% paging rate |
| --- | --- | --- | --- | --- |
| 4 UEs per Group | 3.9% | 0.4% | 0.04% | 0.004% |
| 8 UEs per Group | 7.7% | 0.8% | 0.08% | 0.008% |
| 16 UEs per Group | 14.8% | 1.6% | 0.16% | 0.016% |
| 32 UEs per Group | 27.5% | 3.2% | 0.32% | 0.032% |

From the above Table 2, it can be observed that the larger number of UE grouped in one group, the higher paging group rate would be. Considering 0.1%, 1% of false alarm rate (FAR) are suggested in the agreement in RAN1#112 [2], it can be obtained that:

- The wakeup indication for a UE group cannot satisfy the FAR target   under 1% paging rate per UE.

- With 1% FAR target, the number of UE in one group could be no more than 16 for 1.6% paging group rate under 0.1% paging rate per UE.

- With 0.1% FAR target, the number of UE in one group could be no more than 16 for 0.16% paging group rate under 0.01% paging rate per UE.

- The number of UE in one group can be more than 32 for satisfying 0.1% FAR target under 0.001% paging rate per UE.

- Analysis of paging group rate  for e-DRX

For e-DRX configuration, the paging group rate with different number of UE per group and different paging rate can be derived in the agreed formula. The paging rate per UE for the first i-DRX cycle and each of the remaining L-1 i-DRX cycles within the PTW can be derived by RE, REF with the formula, respectively. The paging rate per UE for e-DRX with 20.48s cycle duration is derived in the following Table 3.

Table 3: The paging rate per UE for e-DRX with 20.48s cycle duration

|  | RE, REF =1% | RE, REF =0.1% | RE, REF =0.01% | RE, REF =0.001% |
| --- | --- | --- | --- | --- |
| Paging rate for the first i-DRX cycle within the PTW | 12.2% | 1.3% | 0.13% | 0.013% |
| Paging rate for each of the remaining L-1 i-DRX cycles within the PTW | 1.0% | 0.1% | 0.01% | 0.001% |

Furthermore, the paging group rate can be obtained by RG = 1 – (1 – RE)N. The paging rate for each of the remaining L-1 i-DRX cycles within the PTW is same as that in e-DRX, the paging group rate with different UEs number can be referred to the Table 3. The paging group rate for the first i-DRX cycle within the PTW is derived in the following Table 4.

Table 4: The paging group rate for the first i-DRX cycle within the PTW

|  | RE, REF =1% | RE, REF =0.1% | RE, REF =0.01% | RE, REF =0.001% |
| --- | --- | --- | --- | --- |
| 4 UEs per Group | 40.70% | 5.07% | 0.52% | 0.0520% |
| 8 UEs per Group | 64.84% | 9.88% | 1.03% | 0.1039% |
| 16 UEs per Group | 87.64% | 18.79% | 2.06% | 0.2078% |
| 32 UEs per Group | 98.47% | 34.05% | 4.07% | 0.4151% |

From the Table 3 and Table 4,  it can be observed that the paging rate is more sensitive to the UE paging grouping method with rapidly increasing in group paging rate as number of increases under RE, REF =1%. The number of UE in one group should not be more than 8 under the condition RE, REF below 0.1%.

Proposal 4: The number of UE in the same group should not be more than 8 for i-DRX and e-DRX with RE, REF below 0.1% for LP-WUS as the PEI.

- 2.4 Power model for LP-WUS

The power consumption of the LP-WUR would have different levels depending on the receiver architecture and its receiver sensitivity. The power consumption would depend on the use of the voltage of power supply, the external component, signal processing, and the oscillator used for LP-WUR. The power consumption of the LP-WUR would increase if the signal processing would require some signals or control processing from the UE receiver, such as the main NR oscillator for slave-drive the LP-WUR oscillator for generating the control signaling associated with the DRX or paging cycle. For LP-WUR ON state, the power consumption of LP-WUR depends on the architecture and requirements. The LP-WUR is characterized by simplified RF and signal processing with low power consumption components. The power consumption level of LP-WUR should be much lower than that of NR main receiver at deep sleep state. Thus, two levels of the power consumptions and ramp-up time of LP-WUR are defined for the receiver association/disassociation with NR configuration with the power consumption level well below the deep sleep state of NR main receiver as shown in Table 5.

Table 5: Power model for LP-WUR

| Power State | Characteristics | Relative Power | Ramp-up time |
| --- | --- | --- | --- |
| Periodic low power WUS “ON” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. | 0.1 | 1ms |
| Periodic low power WUS “OFF” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. Otherwise, the wakeup receiver is shut down. | 0.001 |  |
| Continuous low-power WUS monitoring | Front end wakeup receiver with free-running clock in the active device or passive device monitoring of wakeup signals continuously. | 0.01 |  |

Proposal 5: The suggested power model for LP-WUR is as follows:

| Power State | Characteristics | Relative Power | Ramp-up time |
| --- | --- | --- | --- |
| Periodic low power WUS “ON” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. | 0.1 | 1ms |
| Periodic low power WUS “OFF” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. Otherwise, the wakeup receiver is shut down. | 0.001 |  |
| Continuous low-power WUS monitoring | Front end wakeup receiver with free-running clock in the active device or passive device monitoring of wakeup signals continuously. | 0.01 |  |

Furthermore, the OFDM-based LP-WUR architecture requires FFT demodulation for OFDM signal are proposed in [3]~[5]. Since the main power consumption of the NR is the RF and FFT engine, the power model of sequence detection (e.g., CSI-RS with value 100) in Rel-16 TR38.840 [6] should be reused for the OFDM-based LP-WUR receiver. Similarly, the power model of the OFDM receiver OFF state should reuse the value “1” of deep sleep and the transition energy from deep sleep state to micro sleep. Considering the frequency scaling for smaller BW with 5 MHz (5 MHz is similar to 20 MHz) and antenna scaling to 1 Rx antenna (0.7 scaling factor), the power value of 30 is suggested for OFDM-based LP-WUR using frequency scaling and antenna scaling. The power model for OFDM-based LP-WUR is shown in Table 6.

Table 6: Power model for OFDM-based LP-WUR

| Power State | Relative Power | Total transition time | Additional transition energy |
| --- | --- | --- | --- |
| ON | 30 | 20ms | 290Note |
| OFF | 1 |  |  |
| Note: Similar definition as the additional transition energy in TR38.840. |  |  |  |

Proposal 6: The suggested power model for OFDM-based LP-WUR is as follows:

| Power State | Relative Power | Total transition time | Additional transition energy |
| --- | --- | --- | --- |
| ON | 30 | 20ms | 290Note |
| OFF | 1 |  |  |
| Note: Similar definition as the additional transition energy in TR38.840. |  |  |  |

- 2.5 Synchronization for main radio

It had been discussed in Rel-16 UE power saving that most components of device are turned off during the deep sleep state [6]. The additional components to turn off and associated transition time from ultra-deep sleep state is not much. Based on the discussion in Rel-17 UE power saving enhancement [7], at least 3 SSBs for MR re-sync is need for acquiring Time/Frequency synchronization after wakeup triggered by LP-WUR. MR would perform the following steps for acquiring the Time/Frequency synchronization:

Step1: Timing acquisition from SSB and coarse synchronization – UE is out-of-sync from the network after long ultra-deep sleep. The local oscillator at the MR is in free running mode without the calibration of frequency stability from DL received signals. MR uses the timing after waking up from ultra-deep sleep state for cell search and timing acquisition.

Step2: Frequency and time tracking – After coarse synchronization, MR starts the front-end processing with algorithms of frequency and time offset estimation.

Step3: Calibration of local oscillator – The reference timing clock from local oscillator needs to be calibrated by the received signals with time and frequency offset compensation to correct the frequency drift of the local oscillator in achieving 0.1 ppm frequency stability requirements.

Step4: Time and frequency offset compensation of receiving signals.

Proposal 7:  For MR in out-of-sync, at least 3 SSBs are needed for acquiring Time/Frequency synchronization after wakeup triggered by LP-WUR.

Moreover, the LP-WUR timing could help to reduce the number of SSB for MR synchronization had been discussed.  The LP-WUR timing is quite inaccurate and not feasible to be used as the reference timing to assist the NR synchronization with the reasons shown in the following:

- The local oscillator used by LP-WUR would have large frequency error (e.g., 50-200 ppm).  There is no algorithm of LP-WUR oscillator calibration can meet the 0.1 ppm requirements, since the precise calibration is complicated, costly with high power consumption.  For the LP-WUR can perform the oscillator calibration, the power consumption of LP-WUR would go up to the similar level of NR receiver power consumption.

- The LP-WUR timing could be slaved-driven and calibrated by NR local oscillator before NR receiver transitioned to ultra-deep sleep state (assuming the NR oscillator turned off).  The LP-WUR timing would start with accurate time in sync with NR receiver timing when the NR receiver turn off and transition to ultra-deep sleep state. The inaccurate LP-WUR local oscillator would have large wander and time drift after NR receiver turns off.

From the above analysis, the LP-WUR timing could not be used as the reliable reference timing for MR receiver to reduce the number of SSB required for the synchronization when MR receiver is out-of-sync with the NR network.

Proposal 8：The LP-WUR timing could not be used as the reliable reference timing for MR receiver to reduce the number of SSB required for the synchronization when MR receiver is out-of-sync with the network.

Preliminary evaluation results with LP-WUR

This section discusses the evaluation assumptions and preliminary evaluation results of LP-WUR/LP-WUS for power saving and coverage performance.

- The baseline system configurations and power saving techniques for the UE power saving evaluation of LP-WUR should be based on the latest power saving techniques for RRC_CONNECTED and RRC_IDLE/INACTIVE mode UEs.

- The coverage performance are compared between LP-WUS and two types of NR signals including: PUSCH for msg3 and PDCCH for paging for both normal UE and Redcap UE, respectively.

- 3.1 RRC_IDLE/INACTIVE mode

For RRC_IDLE/INACTIVE mode UEs, the early indication from DCI format 2_7 indicates whether UE should decode the paging DCI and paging message at the subsequent paging occasion for baseline case.

The evaluation assumptions for RRC_IDLE/INACTIVE mode UEs are summarized in the following Table 7.

Table 7:  Evaluation assumptions for RRC_IDLE/INACTIVE mode UEs

| Parameters | Value |
| --- | --- |
| i-DRX cycle length | 1.28s |
| Number of POs in Paging Frame | 1 |
| Sync/re-sync for main radio after ultra-deep sleep | 3 SSBs |
| LP-WUS monitoring | Continuously monitoring |
| Traffic | For RRC_IDLE/INACTIVE state,: - The traffic arrival is modelled as a Poisson Arrival Process where inter-arrival time are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group |

Based on the agreed evaluation methodologies for RRC_IDLE/INACTIVE mode provided in previous section, we provide the evaluation results of power saving performance for LP-WUS compared with Rel-17 PEI. The detail procedure of Rel-17 PEI scheme and LP-WUS scheme are introduced in Table 8.

Table 8: The detail procedure of PEI scheme and LP-WUS scheme

| Scheme | Procedure |
| --- | --- |
| Baseline Scheme: Rel-17 PEI trigger the detection of PO | - UE wakes up before the paging occasion to acquire the SSB for T/F synchronization and detect PEI every DRX cycle. - The PEI indicate whether RRC_IDLE/INACTIVE mode UE should decode the paging DCI in the paging occasion. - If the PEI indicates no paging in the subsequent paging occasion, UE would turn into the sleeping state until the next PEI occasion. Otherwise, UE would receive the paging information in the configured paging occasion. |
| LP-WUS Scheme: LP-WUS in place of PEI and triggering the detection of PO | - The MR can stay in ultra-deep sleep state until activated by LP-WUR. Meanwhile, the LP-WUR would continuously receive LP-WUS. - When the paging information arrived at gNB or the change of system information, the LP-WUS would be transmitted to the target LP-WUR to activate the MR for receiving the paging message. - Moreover, the MR would acquire Time/Frequency synchronization from SSB before monitoring the paging information. |

The power saving performance of LP-WUS scheme and PEI scheme are evaluated in our contribution. Table 9 shows the power saving gain of the LP-WUS scheme compared with Rel-17 DCI format 2_7 as the PEI scheme for 1%, 0.1%, 0.001% paging rate, respectively.

Table 9: Evaluation results for LP-WUS scheme and PEI scheme

|  | 1% paging rate | 0.1% paging rate | 0.01% paging rate |
| --- | --- | --- | --- |
| 3 SSBs synchronization duration for MR | 96.4% | 99.1% | 99.4% |

The results in Table 9 shows that LP-WUS scheme can achieve 96.4%, 99.1% and 99.4% power saving gain comparing to those of Rel-17 PEI with the assumption that LP-WUR having the same receiver sensitivity as that of NR receiver. Since the MR can stay in ultra-deep sleep state for no need to wake up for acquiring SSB and PEI when it is not paged. Moreover, LP-WUR would obtain higher power saving gain with lower paging rate for longer time in ultra-deep sleep state.

Observation 1: Comparing to i-DRX with DCI format 2_7 as the PEI triggering the paging DCI monitoring, LP-WUR/WUS used as the PEI can achieve 96.4%, 99.1% and 99.4% power saving gain, with the assumption that LP-WUR having the same receiver sensitivity as that of NR receiver under 1%, 0.1%, 0.001% paging rate, respectively.

Observation 2: Moreover, LP-WUR would obtain higher power saving gain with lower paging rate for longer time in ultra-deep sleep state.

- 3.2 RRC_CONNECTED mode

For RRC_CONNECTED mode UEs, the DCP is used to indicate whether UE should wake up to the active mode in the subsequent DRX ON for baseline case. The evaluation assumptions for RRC_CONNECTED mode UEs are summarized in the following Table 10.

Table 10:  Evaluation assumptions for RRC_CONNECTED mode UEs

| Parameters | Value |
| --- | --- |
| Scenario and frequency | Dense Urban: 4GHz (TDD) |
| Number of RX chains at the UE’s MR | 2Rx |
| Number of RX chains for LP-WUR | 1Rx |
| BW of gNB | 100M(30kHz) |
| Number of TxRUs for BS | 64TxRUs |
| Number of antenna elements for BS | 192 antenna elements (M,N,P,Mg,Ng) = (12,8,2,1,1) |
| Sync/re-sync for main radio after ultra-deep sleep | 3 SSBs |
| LP-WUS monitoring | Continuously monitoring |
| Traffic Model | FTP 3 traffic model: 200ms/400ms/800ms/1.6s/5s/10s inter-arrival time, 0.5Mbytes packet size. |
| Sensitivity of LP-WUR | -90dBm~-80dBm |

The evaluation results of power saving performance for LP-WUS as wakeup signal are provided comparing to DCP with different DRX configuration and FTP-3 inter-arrival time. The detailed procedure of baseline (DCP scheme) and LP-WUS scheme are shown in the following：

Baseline (DCP scheme): UE periodically detects SSB and DCP on duration for wakeup indication in subsequent DRX ON.

LP-WUS scheme: LP-WUR monitors LP-WUS continuously and indicates to the NR main radio from sleep state to active state in subsequent DRX ON.

Figure 5: The average PSG of LP-WUS scheme

Figure 5 shows the average power saving gain of LP-WUS scheme compared to DCP scheme under different DRX cycle and different FTP3 inter-arrival time. From the results in Figure 5, we can observe that the LP-WUS used as the wakeup indication can achieve 26.7%~88.1%, 25.7%~78.3%, 24.2%~63.4% power saving gain compared to DCP for DRX adaptation with different DRX cycle configurations of 160ms, 320ms, 640ms, respectively. With shorter DRX cycle, more PSG could be obtained by LP-WUS because the power consumption of DCP and SSB detection are the primary factors of the UE power consumption. For a given DRX configuration, large FTP3 inter-arrival time can have higher power saving gain since MR can stay longer time in deep sleep.

Observation 3: With the shorter DRX cycle, the more PSG would be achieved by LP-WUS.

Observation 4: For a given DRX configuration, large FTP3 inter-arrival time can provide high power saving gain since MR can stay longer time in deep sleep.

- LP-WUR sensitivity =-80dBm                                        (b) LP-WUR sensitivity =-85dBm

(c) LP-WUR sensitivity =-90dBm

Figure 6: The average system PSG for LP-WUS scheme for different LP-WUR sensitivity

The LP-WUR sensitivity is the minimum requirement of the receiving power of the LP-WUR in the detection of LP-WUS. The maximum coupling loss and the associated coverage area is derived from the receiver sensitivity of the devices with LP-WUR. The fraction of devices with LP-WUR can meet maximum coupling loss with the LP-WUR sensitivity in system are 32%, 43% and 56% for -80dBm, -85dBm, -90dBm, respectively.

Figure 6 (a) ~ (c) show the average system power saving gain of LP-WUS scheme for different FTP3 inter-arrival time with the LP-WUR sensitivity in a range of -90dB~-80dB, respectively. LP-WUR/LP-WUS can obtain average system power saving gain in a range of 7.7%~28.2%, 10.4%~37.9%, 13.5%~49.3% for 32%, 43%, 56% LP-WUR device ratio with different DRX configurations of 160ms, 320ms, 640ms, respectively. With the increasing of LP-WUR sensitivity, the PSG increases. It means that the larger number of devices with LP-WUS within the coverage performance is, the more power saving can be obtained by LP-WUR. In addition, larger FTP3 inter-arrival time brings higher PSG as shown in Figure 6 (a) ~ (c) due to MR can stay in a long time deep sleep.

Observation 5: The power saving gain increases as the increase of the number of devices with LP-WUR in the coverage area. The increase of the number of devices with LP-WUR would require the increase of the receiver sensitivity of LP-WUR.

- 3.3 Coverage evaluation

The coverage comparison between PUSCH/PDCCH and LP-WUS is shown in the Table 11~Table 14, in which OOK-1 with 1/2 Manchester coding and simple RF envelope detection are applied, respectively. From [8], the OOK waveform with Manchester encoding can significantly improve the detection performance of OOK modulation with at least 8dB compared to that of BPSK modulation. Thus, we evaluate the OOK-1 waveform with 1/2 Manchester coding and simple RF envelope detection for coverage evaluation.

The detail simulation parameters for LP-WUS are shown in Appendix. It can be observed that:

- For normal UE, the LP-WUS with Manchester coding can achieve similar MIL performance with PUSCH for msg3 under Urban 2.6GHz and Urban 4GHz. However, the LP-WUS with Manchester coding cannot reach the MIL level of PDCCH AL16 for normal UE, with 14.4 dB and 22.4 dB MIL performance gap under Urban 2.6GHz and Urban 4GHz, respectively.

- For Redcap UE, the LP-WUS with Manchester coding can obtain up to 6.8dB ,4.4dB, 0.5 dB MIL outperform PDCCH AL8/16 for paging and PUSCH for msg3, respectively.

- For both normal UE and Redcap UE, the coverage of LP-WUS with simple RF envelope detection would be much worse than the PDCCH for paging and PUSCH for msg3 for the poor receiver sensitivity performance of LP-WUR.

Table 11: Coverage comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for normal UE

|  | PDCCH for paging (AL=16) | PUSCH for msg3 | LP-WUS (OOK-1)with Manchester coding | LP-WUS (OOK-1)without Manchester coding |
| --- | --- | --- | --- | --- |
| MIL(dB) | 162.7 | 147.8 | 148.3 | 132.9 |
| Max coverage Range(m) | 549.9 | 273.6 | 234.2 | 95.1 |

Table 12: Coverage comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for Redcap UE

|  | PDCCH for paging (AL=8) | PDCCH for paging (AL=16) | PUSCH for msg3 | LP-WUS (OOK-1)with Manchester coding | LP-WUS (OOK-1)without Manchester coding |
| --- | --- | --- | --- | --- | --- |
| MIL(dB) | 142.9 | 144.6 | 147.8 | 148.3 | 132.9 |
| Max coverage Range(m) | 170.7 | 188.7 | 273.6 | 234.2 | 95.1 |

Table 13: Coverage comparison between PUSCH/PDCCH and LP-WUS for Urban 4GHz for normal UE

|  | PDCCH for paging (AL=16) | PUSCH for msg3 | LP-WUS (OOK-1)with Manchester coding | LP-WUS (OOK-1)without Manchester coding |
| --- | --- | --- | --- | --- |
| MIL(dB) | 166.7 | 147.8 | 144.3 | 128.9 |
| Max coverage Range(m) | 558.3 | 219.5 | 148.4 | 60.3 |

Table 14: Coverage comparison between PUSCH/PDCCH and LP-WUS for Urban 4GHz for Redcap UE

|  | PDCCH for paging (AL=8) | PDCCH for paging (AL=16) | PUSCH for msg3 | LP-WUS (OOK-1)with Manchester coding | LP-WUS (OOK-1)without Manchester coding |
| --- | --- | --- | --- | --- | --- |
| MIL(dB) | 141.5 | 143.9 | 147.83 | 148.3 | 132.9 |
| Max coverage Range(m) | 126.1 | 145.2 | 219.5 | 187.9 | 76.3 |

Observation 6: For normal UE, the LP-WUS with Manchester coding can achieve comparable MIL performance with PUSCH for msg3 under Urban 2.6GHz and Urban 4GHz.

Observation 7: For Redcap UE, the LP-WUS with Manchester coding can obtain up to 6.8dB ,4.4dB, 0.5 dB MIL outperform PDCCH AL8/16 for paging and PUSCH for msg3, respectively.

Observation 8: For both normal UE and Redcap UE, the coverage of LP-WUS with simple RF envelope detection would be much worse than that of PDCCH for paging and PUSCH for msg3 for the poor receiver sensitivity performance of LP-WUR.

- 4 Conclusion

In this contribution, we discuss remaining issues of evaluation methodologies and the preliminary evaluation results of low-power wakeup mechanism. We have the following observations and proposals:

Observation 1: Comparing to i-DRX with DCI format 2_7 as the PEI triggering the paging DCI monitoring, LP-WUR/WUS used as the PEI can achieve 96.4%, 99.1% and 99.4% power saving gain, with the assumption that LP-WUR having the same receiver sensitivity as that of NR receiver under 1%, 0.1%, 0.001% paging rate, respectively.

Observation 2: Moreover, LP-WUR would obtain higher power saving gain with lower paging rate for longer time in ultra-deep sleep state.

Observation 3: With the shorter DRX cycle, the more PSG would be achieved by LP-WUS.

Observation 4: For a given DRX configuration, large FTP3 inter-arrival time can provide high power saving gain since MR can stay longer time in deep sleep.

Observation 5: The power saving gain increases as the increase of the number of devices with LP-WUR in the coverage area. The increase of the number of devices with LP-WUR would require the increase of the receiver sensitivity of LP-WUR.

Observation 6: For normal UE, the LP-WUS with Manchester coding can achieve comparable MIL performance with PUSCH for msg3  under Urban 2.6GHz and Urban 4GHz.

Observation 7: For Redcap UE, the LP-WUS with Manchester coding can obtain up to 6.8dB ,4.4dB, 0.5 dB MIL outperform PDCCH AlL8/16 for paging and PUSCH for msg3, respectively.

Observation 8: For both normal UE and Redcap UE, the coverage of LP-WUS with simple RF envelope detection would be much worse than that of PDCCH for paging and PUSCH for msg3 for the poor receiver sensitivity performance of LP-WUR.

Proposal 1:  The LP-WUS used as the paging early indication (PEI) only consume 1/10000 power of DCI format 2_7 (DCI-based PEI) for UEs in RRC_IDLE/INACTIVE mode.

Proposal 2:  LP-WUS is used in place of the PO indication is not appropriate because it requires UE-specific LP-WUS to carry the paging indication for the group of UEs and associated paging information with unique ID in the registration area.

Proposal 3:  The LP-WUS as the wakeup indication could reduce detection power consumption of DCI format 2_6 and SSB. Moreover, it can also be applied to the UEs without DRX configuration as the application of DCI format 2_7 restricted by DRX.

Proposal 4: The number of UE in the same group should not be more than 8 for i-DRX and e-DRX with RE, REF below 0.1% for LP-WUS as the PEI.

Proposal 5: The suggested power model for LP-WUR is as follows:

| Power State | Characteristics | Relative Power | Ramp-up time |
| --- | --- | --- | --- |
| Periodic low power WUS “ON” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. | 0.1 | 1ms |
| Periodic low power WUS “OFF” state | Front end wakeup receiver is configured to detect the wakeup signals periodically associated with C-DRX or PO. Otherwise, the wakeup receiver is shut down. | 0.001 |  |
| Continuous low-power WUS monitoring | Front end wakeup receiver with free-running clock in the active device or passive device monitoring of wakeup signals continuously. | 0.01 |  |

Proposal 6: The suggested power model for OFDM-based LP-WUR is as follows:

| Power State | Relative Power | Total transition time | Additional transition energy |
| --- | --- | --- | --- |
| ON | 30 | 20ms | 290Note |
| OFF | 1 |  |  |
| Note: Similar definition as the additional transition energy in TR38.840. |  |  |  |

Proposal 7:  For MR in out-of-sync, at least 3 SSBs are needed for acquiring Time/Frequency synchronization after wakeup triggered by LP-WUR.

Proposal 8：The LP-WUR timing could not be used as the reliable reference timing for MR receiver to reduce the number of SSB required for the synchronization when MR receiver is out-of-sync with the network.

References

- RAN1 chairman notes, 3GPP TSG RAN WG1 #112bis-e, April 17th – April 26th, 2023.

- RAN1 chairman notes, 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2303897, Evaluation methodologies for R18 LP-WUS/WUR, vivo, RAN1#112bis-e, April 17th – April 26th, 2023.

- R1-2303612, Evaluation methodology for LP-WUS, Qualcomm Incorporated, RAN1#112bis-e, April 17th – April 26th, 2023.

- R1-2303332, Evaluation on low power WUS, MediaTek Inc, RAN1#112bis-e, April 17th – April 26th, 2023.

- TR38.840, “Study on UE Power Saving”, v16.0.0.

- R1-2007867, Paging enhancement for UE power saving, CATT, RAN1#103-e, October 26th – November 13th, 2020.

- TANG Qiuju, WANG Zhaoying, LIU Ying．The Method of Anti-Doppler Modulation and Demodulation Based on Manchester Code[J]．Radio Engineering,2017,47( 5) : 37－39,105．

Appendix

- 5.1 Coverage evaluation assumption for LP-WUS

Table 15: Simulation assumptions for LP-WUS

| Parameter | Configuration |
| --- | --- |
| Scenario | 2.6GHz Dense Urban 4GHz Dense Urban |
| Number of RX antennas | 1 |
| SCS | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3 km/h |
| Payload size | 8bit |
| Data rate | 16 kbps(OOK-1 with 1/2 rate Manchester) 8 kbps(OOK-1 with RF envelope detection) |
| Time/frequency synchronization | Ideal |
| ADC | Ideal |
| Sampling Rate | - 15.36 MHz |

- 5.2 Power Model

TR 38.840 is reused to assess the UE power consumption evaluation for LP-WUS performance evaluation. The power consumption model of FR1 for evaluation is shown in Table 16 and Table 17.

Table 16: UE power consumption model

| Power state | Relative Power |
| --- | --- |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| Ultra-deep sleep | 0.015 |
| PDCCH only | 100 |
| PDCCH+PDSCH | 300 |

Table 17: UE power consumption during the state transition

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| Ultra-deep sleep | 15000 | 800 ms |
| *Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |