3GPP TSG RAN WG1 #113		R1-2304726

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	CATT

Title:	Evaluation on AI/ML for positioning enhancement

Agenda Item:     9.2.4.1

Document for:    Discussion and Decision

- Introduction

In RAN1 #112bis-e meeting, the following agreements were reached [1].

Agreement For evaluation of both the direct AI/ML positioning and AI/ML assisted positioning, company optionally adopt delay profile (DP) as a type of information for model input. - DP is a degenerated version of PDP, where the path power is not provided. Agreement For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs includes the following approaches. Proponent of each approach provides analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity. - Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0. - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically. - Note: for Approach 1, one model is provided to cover the entire evaluation area. - Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed. - For Approach 2: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18 N’TRP) TRPs. - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models. Agreement In the evaluation of AI/ML based positioning, if N’TRP<18, the set of N’TRP TRPs that provide measurements to model input of an AI/ML model are reported using the TRP indices shown below. Agreement For AI/ML assisted positioning with TOA as model output, study the impact of labelling error to TOA accuracy and/or positioning accuracy. - The ground truth label error of TOA is calculated based on location error. The location error in each dimension of x-axis and y-axis can be modelled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models of labelling error are not precluded - Other timing information, e.g., RSTD, as model output is not precluded. Agreement For AI/ML assisted positioning with LOS/NLOS indicator as model output, study the impact of labelling error to LOS/NLOS indicator accuracy and/or positioning accuracy. - The ground truth label error of LOS/NLOS indicator can be modelled as m% LOS label error and n% NLOS label error. - Value m and n are up to sources. - Companies consider at least hard-value LOS/NLOS indicator as model output. Agreement For the evaluation of AI/ML based positioning method, the measurement size and signaling overhead for the model input is reported. Observation For AI/ML based positioning method, companies have submitted evaluation results to show that for their evaluated cases, for a given company’s model design, a lower complexity (model complexity and computational complexity) model can still achieve acceptable positioning accuracy (e.g., <1m), albeit degraded, when compared to a higher complexity model. Note: For easy reference, sources include CMCC (R1-2303228), InterDigital (R1-2303450), Ericsson (R1-2302335), Huawei/HiSilicon (R1-2302362), CATT (R1-2302699), Nokia (R1-2302632). Observation For direct AI/ML positioning, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian Distribution of the ground truth label error. Observation For AI/ML assisted positioning, evaluation results have been provided by sources for label-based model monitoring methods. With TOA and/or LOS/NLOS indicator as model output, the estimated ground truth label (i.e., TOA and/or LOS/NLOS indicator) is provided by the location estimation from the associated conventional positioning method. The associated conventional positioning method refers to the method which utilizes the AI/ML model output to determine target UE location. Note: Sources include vivo (R1-2302481), MediaTek (R1-2303340), Ericsson (R1-2302335) Observation For both direct AI/ML and AI/ML assisted positioning, evaluation results have been provided by sources to demonstrate the feasibility of label-free model monitoring methods. Note: Sources include vivo (R1-2302481), CATT (R1-2302699), MediaTek (R1-2303340), Ericsson (R1-2302335), Nokia (R1-2302632). Observation For both direct AI/ML and AI/ML assisted positioning, evaluation results submitted to RAN1#112bis show that with CIR model input for a trained model, - For two SNR/SINR values S1 (dB) and S2 (dB), S1>=S2 + 15 dB, positioning error of a model trained with data of S1 (dB) and tested with data of S2 (dB) is more than 5.75 times that of the model trained and tested with data of S1 (dB). - For two SNR/SINR values S1 (dB) and S2 (dB), S1<=S2 – 10 dB, the generalization performance of a model trained with data of S1 (dB) and tested with data of S2 (dB) is better than the performance of a model trained with data of S2 (dB) and tested with data of S1 (dB). Positioning error of a model trained with data of S2 (dB) and tested with data of S1 (dB) is more than 2.97 times that of the model trained with data of S1 (dB) and tested with data of S2 (dB). Note: here the positioning error is the horizontal positioning error (meters) at CDF=90%. Observation For direct AI/ML positioning, based on evaluation results of timing error in the range of 0-50 ns, when the model is trained by a dataset with UE/gNB RX and TX timing error t1 (ns) and tested in a deployment scenario with UE/gNB RX and TX timing error t2 (ns), for a given t1, - For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example, - For the case of (t1, t2)=(50ns, 30ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(50ns, 30ns) is 0.82~0.86 times that of (t1, t2)=(50ns, 50ns). - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(50ns, 0ns) is 0.80~0.82 times that of (t1, t2)=(50ns, 50ns). - For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example, - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(0ns, 10ns) is 1.25~18.7 times that of (t1, t2)=(0ns, 0ns). - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(0ns, 50ns) is 3.5~18.3 times that of (t1, t2)=(0ns, 0ns). Note: here the positioning error is the horizontal positioning error (meters) at CDF=90%. Observation For direct AI/ML positioning, based on evaluation results of network synchronization error in the range of 0-50 ns, when the model is trained by a dataset with network synchronization error t1 (ns) and tested in a deployment scenario with network synchronization error t2 (ns), for a given t1, - For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example, - For the case of (t1, t2)=(50ns, 10ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(50ns, 10ns) is 0.74~0.83 times that of (t1, t2)=(50ns, 50ns). - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#112bis show the positioning error of (t1, t2)=(50ns, 0ns) is 0.73~0.82 times that of (t1, t2)=(50ns, 50ns). - For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example, - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#112bis show the positioning error of (0ns, 10ns) is 1.17~9.5 times that of (0ns, 0ns). - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#112bis show the positioning error of (0ns, 50ns) is 10~40 times that of (0ns, 0ns). Note: here the positioning error is the horizontal positioning error (meters) at CDF=90%.

In this document, we provide our evaluation results on AI/ML-based positioning.

- Simulation assumptions

In RAN1#109-e meeting, the common scenario parameters defined in Table 6-1 of TR 38.857 and some parameters of InF-DH scenario were agreed for evaluation of AI/ML-based positioning. In our simulations, the simulation assumptions are shown in Annex, and the dataset is generated by a system level simulator based on 3GPP simulation methodology. Both cases of perfect network synchronization and network synchronization with a truncated Gaussian distribution of 50 ns are investigated in our simulations. If network synchronization error is assumed, it is defined as a TRP-specific error value between the TRP and a timing reference source which is assumed to have perfect timing. For the generalization evaluation, we also simulate the generalization performance between InF-DH and InF-SH scenarios.

In RAN1#110bis e-meeting, the evaluation methodology of the AI/ML complexity (model complexity, computation complexity) for N TRPs was determined. In our simulations, complexity of Multi-TRP (i.e., one model for N TRPs) and Single-TRP (i.e., same model for N TRPs) are the assessed. The sampling interval of all simulations is 4 ns. In our model, the parameters are in real value.

In our evaluations, we use the same area of 120m*60m for training dataset and testing dataset.

In most of our simulations, for the evaluation of AI/ML based positioning method, CIR of all (NTRP = 18) TRPs with all (NT = 256) taps are used as the model input, and single port (Nport = 1) is assumed. Each sample is represented by double float type (8 Byte, i.e. 64 bits). Thus, the measurement size is NTRP*Nport*NT*2*64 = 294912bit per UE, unless otherwise stated, e.g. in Section 3.1.6, 3.2.3 and 3.2.4. We do not optimize the signaling overhead for AI/ML model at LMF side, and thus it can be viewed as the same with the measurement size.

- Simulation results

For the simulation results, most of them are captured in the excel sheet which are zipped together with this contribution. Those captured in the excel sheets are not duplicated here. Additionally, the LOS/NLOS identification results and model monitoring results are still provided in this contribution.

  - Direct AI/ML positioning

For direct AI/ML positioning, AI/ML model is used to estimate UE’s position directly without intermedium ToA estimation. We provide some simulation results of direct positioning based on AI/ML model in this section, including generalization, fine-tuning, mix-train, label error and different sizes of model input. The AI/ML model for estimating UE’s position is a classical ResNet18 model. The input of this AI/ML model is CIR, and the size of CIR is 18*1*256*2. The output of this AI/ML model is UE’s horizontal positon (x, y). We chose MSE loss as the loss function. The training epoch and learn rate is set to 100 and 1*10-4.

    - AI/ML model performance of baseline and generalization

      - AI/ML model performance with perfect network synchronization

For the simulations in this section, perfect network synchronization is assumed. This AI/ML model inference may be performed at UE side or LMF side. When inference is performed at UE side, the UE may utilize the CIR estimated from DL PRS and an AI/ML model to estimate the UE’s position directly. When this model is deployed at LMF side, the UE needs to transfer the CIR to LMF for AI/ML model inference.

The details of dataset, AI/ML model and evaluation results are given in Table 1 and also the excel sheet submitted together. For perfect network synchronization, the horizontal accuracy is 0.58m@90% of CDF percentile.

Table 1: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type: UE’s position; Size: 1*2 | UE’s position with 100% ground truth label | clutter param: {60%, 6m, 2m} | clutter param: {60%, 6m, 2m} | Training: 19440; Validation: 1080; | 1080 | 11.2M | 2.78G FLOPs | 0.58m |

Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal positioning accuracy is 0.58m@90% of CDF percentile.

      - AI/ML model performance with network synchronization error

In Rel-16, various positioning technologies were specified to support regulatory as well as commercial use cases. In Rel-17, some positioning enhancements had been introduced. However, in current IIoT scenario, it is still worth of thinking about overcoming non-ideal factors, e.g. synchronization errors between multiple TRPs. Due to the non-ideal factors, positioning accuracy traditional positioning algorithms will be degraded. In this section, we investigate whether AI/ML model can overcome the influence of network synchronization error. The network synchronization error with a truncated Gaussian distribution of 50 ns is assumed, and the network synchronization error is defined as a TRP-specific error.

The AI/ML model for estimating UE’s position is a classical ResNet model. The details of dataset, AI/ML model and evaluation results are given in Table 2 and also the excel sheet submitted together. For network synchronization with a truncated Gaussian distribution of 50 ns, the horizontal positioning accuracy is 0.84m@90% of CDF percentile. Compared to the simulation results in 3.1.1.1, the positioning accuracy with network synchronization error is similar to the positioning accuracy of the case with perfect network synchronization. When AI/ML-based approach is applied in positioning, the network synchronization error is included in the corresponding training data set and test data set. If the network synchronization error is captured in the training set, AI/ML model would be able to learn how to overcome network synchronization error to improve positioning accuracy, since AI/ML-based positioning has data-driven characteristics. Based on our simulation results, AI/ML model is able to overcome the impact of network synchronization error through learning the internal relationship between the input training data with network synchronization error and the corresponding label.

Table 2: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type: UE’s position; Size: 1*2 | UE’s position with 100% ground truth label | with network synchronization error | with network synchronization error | Training: 19440; Validation: 1080; | 1080 | 11.2M | 2.78G FLOPs | 0.84m |

Observation 2: For directly estimating UE’s positioning with network synchronization error (truncated Gaussian distribution of 50 ns), the horizontal positioning accuracy is 0.84m@90% of CDF percentile.

    - AI/ML model generalization performance with different assumptions

We design three cases to investigate the generalization capability of AI/ML model. The configurations of 3 test cases are shown in Table 3.

Table 3: Configurations of three test cases for investigating generalization capability of AI/ML model

|  | Scenario | Clutter parameters | Synchronization error |
| --- | --- | --- | --- |
| Case1 | InF-DH | {40%, 2m, 2m} | w/o |
| Case2 | InF-DH | {60%, 6m, 2m} | w |
| Case3 | InF-SH | {20%, 2m, 10m} | w/o |

The AI/ML model described in Section 3.1.1.1 is trained on a dataset with parameters of {60%, 6m, 2m} from InF-DH scenario, with perfect network synchronization. This model is tested on dataset corresponding to case 1 and case 3. For case 2, the time sampling windows are shifted forward by 50ns, resulting in exclusion of last few sampling points due to the excessive time delay. However, the discarded CIR has minimal impact on the previous results. For more details on the dataset, AI/ML model and evaluation results are given in Table 4 and also the accompanying excel sheet.

Table 4: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type: UE’s position; Size: 1*2 | UE’s position with 100% ground truth label | clutter param: {60%, 6m, 2m} | clutter param: {40%, 2m, 2m} | Training: 19440; Validation: 1080; | 1080 | 11.2M | 2.78G FLOPs | 2.64m |
| Type: CIR; Size: 18*1*256 | Type: UE’s position; Size: 1*2 | UE’s position with 100% ground truth label | w/o network synchronization | w/ network synchronization | Training: 19440; Validation: 1080; | 1080 | 11.2M | 2.78G FLOPs | 12.6m |
| Type:CIR; Size:18*1*256 | Type: UE’s position; Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,2m,10m} | Training: 19440; Validation: 1080; | 1080 | 11.2M | 2.78G FLOPs | 6.48m |

Observation 3: When the direct AI/ML positioning is applied to different scenarios or conditions, the generalization performance can be significantly affected, leading to a significant degradation in positioning accuracy.

    - AI/ML model generalization performance with mix-train

Section 3.1.2 presents our simulation results of generalization performance of the AI/ML model. In Section 3.1.3, we investigate the mix-train method as a means to enhance the model’s generalization capability. This involves mixing the datasets from different positioning scenarios/assumptions and training the model on the mixed training dataset. The AI/ML model is then tested on separate test datasets. Our simulations feature three different cases, which are detailed in the accompanying excel sheet.

      - Mix-train with different clutter parameters

In Section 3.1.3.1, we employ the mix-train method by combining 19440 data samples with a clutter parameter of {60%, 6m, 2m} and 1080 data samples with a clutter parameter of {40%, 2m, 2m} for model training. For testing, we used 1080 data samples with a clutter parameter of {40%, 2m, 2m} and another 1080 data samples with a clutter parameter of {60%, 6m, 2m} separately. The details of dataset and the simulation results for are provided in Table 5 and also the accompanying excel sheet submitted together. The horizontal position accuracy is 1.77m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improves positioning accuracy from 2.64m to 1.77m. However, when the AI/ML model is tested on a small dataset with the assumption of a clutter parameter of {60%, 6m, 2m}, the positioning accuracy degrades from 0.58m to 1.31m compared to the case when using only the InF-DH {60%, 6m, 2m} dataset for AI/ML model training and testing.

Table 5: Evaluation results for AI/ML model mix-trained in different clutter parameters deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-DH{40%,2m, 2m} | InF-DH{40%,2m, 2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.77m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-DH{40%,2m, 2m} | InF-DH {60%,6m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.31m |

Observation 4: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.77m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 2.64m.

      - Mix-train with different network synchronization assumptions

In this section, we employ the mix-train method by combining 19440 data samples with perfect synchronization and 1080 data samples with 50ns synchronization error for model training. For testing, we used 1080 data samples with perfect synchronization and another 1080 data samples with 50ns synchronization error separately. The accompanying excel sheet and also Table 6 provide details on the dataset and simulation results for the AI/ML model trained using this method, which achieved a horizontal positioning accuracy of 3.04m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improved the accuracy from 12.6m to 3.04m. However, when the AI/ML model is tested on a small dataset with the assumption of perfect network synchronization, the positioning accuracy degrades from 0.58m to 1.37m when compared to using only the dataset with perfect network synchronization for AI/ML model training and testing.

Table 6: Evaluation results for AI/ML model mix-trained in different network synchronization error deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | Ideal synchronization and network synchronization error 50ns | network synchronization error 50ns | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 3.04m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | Ideal synchronization and network synchronization error 50ns | Ideal synchronization | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.37m |

Observation 5: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 3.04m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.6m.

      - Mix-train with different scenarios

In this section, we employ the mix-train method by combining 19440 data samples with InF-DH {60%, 6m, 2m} and 1080 data with InF-SH {20%, 2m, 10m} for model training. For testing, we used 1080 data samples with InF-SH {20%, 2m, 10m} and another 1080 data samples with InF-DH {60%, 6m, 2m} separately. The accompanying excel sheet and also Table 7 provide details on the dataset and simulation results for the AI/ML model trained using this method, which achieved a horizontal positioning accuracy of 1.81m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improved the accuracy from 6.48m to 1.81m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of InF-DH{60%, 6m, 2m}, the position accuracy is degraded from 0.58m to 1.37m compared to using only the dataset with InF-DH{60%, 6m, 2m} for AI/ML model training and testing.

Table 7: Evaluation results for AI/ML model mix-trained in different scenarios deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.81m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-SH {20%,10m,2m} | InF-DH {60%,6m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.37m |

Observation 6: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.48m.

    - AI/ML model generalization performance with fine-tuning

In Section 3.1.3, we investigate the generalization capability of the AI/ML model and observe that the positioning accuracy will degrade when the training and testing datasets have different clutter parameters or network synchronization assumptions. This aspect is important, as it can be computationally and storage-intensive to train an AI/ML model for every scenarios or assumptions. To enhance the accuracy of the AI/ML model when training and testing datasets have different assumptions, we explore the potential of fine-tuning the model's performance.

We also investigate the impact of the quantity of fine-tuning data on the positioning accuracy. In our simulations, we vary the size of the fine-tuning dataset, which includes: 540, 1080, 1620 and 3600 labels. The simulation results are presented in the subsequent sections.

      - Fine-tuning with different clutter parameters

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of clutter parameter {60%, 6m, 2m}. This AI/ML model serves as the basic AI/ML model, which is then fine-tuned using a small dataset that assumes a clutter parameter {40%, 2m, 2m}. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620 with the aforementioned clutter parameter of {40%, 2m, 2m} is provided in Table 8 and also the accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.1.2, the performance is significantly improved by the fine-tuning of the AI/ML model using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}.

Table 8: Evaluation results for AI/ML model different clutter parameters deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 540 | 1080 | 11.2M | 2.78G FLOPs | 1.01m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 1080 | 1080 | 11.2M | 2.78G FLOPs | 0.95m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 1620 | 1080 | 11.2M | 2.78G FLOPs | 0.91m |

Observation 7: For direct AI/ML positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

      - Fine-tuning with different network synchronization assumptions

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of perfect network synchronization. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620, 3600 with network synchronization error is provided in Table 9 and also accompanying excel sheet. Compared to the positioning accuracy in Section 3.1.2, the performance is significantly improved from 12.6m to 1.43m by the fine-tuning of the AI/ML model with a small dataset with size of 3600 with the assumption of network synchronization error.

Table 9: Evaluation results for AI/ML model for different network synchronization assumptions deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | w/o network synchronization | w/ network synchronization | w/ network synchronization | 19440 | 540 | 1080 | 11.2M | 2.78G FLOPs | 2.69m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | w/o network synchronization | w/ network synchronization | w/ network synchronization | 19440 | 1080 | 1080 | 11.2M | 2.78G FLOPs | 2.23m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | w/o network synchronization | w/ network synchronization | w/ network synchronization | 19440 | 1620 | 1080 | 11.2M | 2.78G FLOPs | 2.00m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | w/o network synchronization | w/ network synchronization | w/ network synchronization | 19440 | 3600 | 1080 | 11.2M | 2.78G FLOPs | 1.43m |

Observation 8: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

      - Fine-tuning with different scenarios

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of InF-DH. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620 and 3600 with InF-SH are provided in accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.1.2, the performance is significantly improved from 6.48m to 1.90m by the fine-tuning of the AI/ML model with a small dataset with size of 3600 with the assumption of different scenarios.

Table 10: Evaluation results for AI/ML model for different scenarios deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 540 | 1080 | 11.2M | 2.78G FLOPs | 3.69m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 1080 | 1080 | 11.2M | 2.78G FLOPs | 2.98m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 1620 | 1080 | 11.2M | 2.78G FLOPs | 2.66m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 19440 | 3600 | 1080 | 11.2M | 2.78G FLOPs | 1.90m |

Observation 9: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

    - AI/ML model performance with labeling error

It was concluded in RAN1#112 meeting to study the impact of labelling error on positioning accuracy. To achieve this, we introduce errors in the x-axis and y-axis of the labels, resulting in inaccurate labels. The error in each dimension of x-axis and y-axis is modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. We then set the value of L as 1m, 3m and 5m. The simulation result is provided in Table 11 and also the accompanying excel sheet.

Specifically:

- When the value of L is 1m, the horizontal position accuracy is 1.47m@90% of CDF percentile.

- When the value of L is 3m, the horizontal position accuracy is 2.95m@90% of CDF percentile.

- When the value of L is 5m, the horizontal position accuracy is 4.41m@90% of CDF percentile.

Table 11: Evaluation results for direct AI/ML positioning model for label error deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label Error | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | L=1m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 1.47m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | L=3m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 2.95m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | L=5m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 and 1080 | 1080 | 11.2M | 2.78G FLOPs | 4.41m |

Observation 10: When AI/ML model is trained with different value of label error, as value of L increases, direct AI/ML positioning error increases approximately in proportion to L.

    - AI/ML model performance with different model input

For direct AI/ML positioning, the current model’s input dimensions may cause an increase in the model complexity and computational complexity. As a result, it is attractive to reduce the input dimensions of the model.

      - Different number of TRP as model input

For the simulation of reducing the number of TRP, we evaluate two cases for direct AI/ML positioning.

The first case is to select 9 fixed TRPs and train a model to cover the entire simulation area. In inference phase, this model is used to cover the entire evaluation area.

The second case is to divide the original area into two regions with equal size, and two models are trained based on the region-specific dataset separately. In inference phase, each model is used to cover the corresponding region, and the whole evaluation area can be covered by these two models.

In this simulation, since NTRP = 9, the measurement size is NTRP*Nport*NT*2*64 = 147456bit per UE.

        - One model covers entire area

In our simulation, we select 9 TRPs out of 18 TRPs for model input. The selected TRPs are with the ID of {3, 4, 5, 9, 10, 11, 15, 16, 17}. The TRP are numbered according to the latest agreement regarding the TRP indices in RAN1#112bis-e. Specifically, two designs are considered for this case.

In our first design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP) that provides measurements as model input. The horizontal position accuracy is slightly degraded from 0.58m@90% to 0.71m@90% of CDF percentile. The computational complexity is reduced from 2.78G FLOPs to 1.62G FLOPs. The details of dataset and the simulation results are provided in the first row of Table 12 and also the accompanying excel sheet.

In our second design, Approach 1-A is applied, i.e. 9 fixed TRPs provides the effective input value to the model and the input of the other 9 TRPs are zero-padded. The horizontal position accuracy is slightly degraded from 0.58m@90% to 0.59m@90% of CDF percentile. The details of dataset and the simulation results are provided in the second row of Table 12 and also the accompanying excel sheet.

We also observe that if the model input dimension is reduced when only part of TRPs are used as input, the FLOPs is reduced compared to the case when all TRPs are used. Additionally, the model overhead is reduced while maintaining similar horizontal positioning accuracy.

Table 12: Evaluation results for AI/ML model for fixed 9 TRPs of one model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 9*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 | 1080 | 11.2M | 1.62G FLOPs | 0.71m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 | 1080 | 11.2M | 2.78G FLOPs | 0.59m |

Observation 11: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 12: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

        - Two models, each covers half the area

In this case, we divide the entire simulation area into two regions, named as region A and region B, by equal proportion. The TRP ID of region A is {0, 1, 2, 3, 4, 5, 6, 7, 8}, and the TRP ID of region B is {9, 10, 11, 12, 13, 14, 15, 16, 17}. The TRP are numbered according to the latest agreement regarding the TRP indices in RAN1#112bis-e. Similarly, two designs are considered for this case.

In our design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP=9) that provides measurements as model input. The horizontal position accuracy is degraded from 0.58m@90% to 1.14m@90% of CDF percentile. The details of dataset and the simulation result are provided in Table 13 and also the accompanying excel sheet.

Table 13: Evaluation results for AI/ML model for fixed 9 TRPs of two models deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 9*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 9720*2 | A:611 B:469 | 11.2M*2 | 1.62G FLOPs | 1.14m |

Compared to the results of using single model to cover the entire area in previous sub-section, the performance of using two models to cover the entire area is slightly degraded. The possible reason is that the training dataset of each model in this case is reduced by half, leading to the insufficient training.

Observation 13: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area.

      - Different CIR taps number as model input

In this section, we evaluate the positioning performance and the computation complexity of the AI/ML model by reducing the number of CIR taps. In the simulation, only the first 128, 64, and 32 CIR taps from the 256 CIR taps in time domain are used as the model input respectively. So the values of measurement size are 147456bits per UE (NT=128), 73728bits per UE (NT=64), 36864bits per UE (NT=32).

The details of dataset and the simulation results are provided in Table 14 and also the accompanying excel sheet.

Table 14: Evaluation results for AI/ML model for different CIR taps number as model input deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*128 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 | 1080 | 11.2M | 1.39G FLOPs | 0.57m |
| Type: CIR; Size: 18*1*64 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 | 1080 | 11.2M | 0.7G FLOPs | 0.58m |
| Type: CIR; Size: 18*1*32 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 19440 | 1080 | 11.2M | 0.35G FLOPs | 0.62m |

Observation 14: For direct AI/ML positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

  - AI/ML-assisted positioning

For AI/ML assisted positioning, AI/ML model is used to estimate timing and/or angle of measurement or LOS/NLOS. We provide some simulation results of AI/ML assisted positioning in this section. The AI/ML model is used to estimate ToA, and then the estimated ToA is used to calculate UE’s position using the traditional positioning calculation algorithms. In Section 3.2.3.2, The AI/ML model is used to identify LOS/NLOS. The LOS/NLOS identification can assist the non-AI/ML positioning algorithm.

    - AI/ML model baseline and generalization performance

      - AI/ML model performance with perfect network synchronization

For the simulations in this section, perfect network synchronization is assumed. The AI/ML model for estimating ToA is a classical ResNet18 model. The input of this AI/ML model is CIR, and the size of CIR is 18*1*256*2. The output of this AI/ML model is 18 ToAs from 18 TRPs to target UE, respectively, i.e. the size of output is 1*18. This AI/ML model inference may be performed at UE side or LMF side. When inference is performed at UE side, the UE may utilize the DL-PRS CIR and an AI/ML model to estimate ToA. When this model is deployed at LMF side, the UE needs to transfer the DL-PRS CIR to LMF for ToA estimating based on AI/ML model.

The details of dataset, AI/ML model and evaluation results are given in Table 15 and also the excel sheet submitted together. For perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

Table 15: Evaluation results for AI/ML-assisted positioning deployed on UE/LMF-side, without model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type: ToA; Size: 1*18 | ToA with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 0.655m |

Observation 15: For AI/ML-assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

      - AI/ML model performance with network synchronization error

In this section, we conduct a simulation with network synchronization error assumption. The network synchronization error with a truncated Gaussian distribution of 50 ns is assumed, and the network synchronization error is defined as a TRP-specific error.

The AI/ML model for estimating UE’s position is a classical ResNet model. The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together. The intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy. Compared to the simulation result in Table 16 and also the excel sheet submitted together, the positioning accuracy of AI/ML assisted positioning with network synchronization error is similar to the positioning accuracy of AI/ML assisted positioning with perfect network synchronization. Based on our simulation results, the positioning accuracy of AI/ML model with network synchronization error is not deteriorated, which means the AI/ML model can overcome the influence of network synchronization error on the test data set through learning the internal relationship between the training data set with network synchronization error and the corresponding labels.

Table 16: Evaluation results for AI/ML-assisted positioning with network synchronization error deployed on UE/LMF-side, without model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type: ToA; Size: 1*18 | ToA with 100% ground truth label | with network synchronization error | with network synchronization error | 20000 | 1600 | 0.7M | 60.8M FLOPs | 0.7m |

Observation 16: For AI/ML-assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

      - AI/ML model generalization performance with different assumptions

For AI/ML assisted positioning, to investigate the AI/ML model generalization capability, we further evaluate the AI/ML model generalization performance with different assumptions between training dataset and test dataset, e.g. different clutter parameters assumptions and different network synchronization assumptions.

The AI/ML model is trained with the assumption of clutter parameter InF-DH {60%, 6m, 2m} with perfect network synchronization, tested dataset with three cases. The test cases are presented in Table 3.The details of dataset, AI/ML model and evaluation results are given in Table 17 and also the excel sheet submitted together.

Table 17: Evaluation results for AI/ML-assisted positioning deployed on UE/LMF-side, with model generalization, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | InF-DH {60%,6m,2m} | InF-DH {40%,2m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 3.11m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | w/o network synchronization error | w/ network synchronization error | 20000 | 1600 | 0.7M | 60.8M FLOPs | 12.8m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | InF-DH{60%,6m,2m} | InF-SH{20%,10m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 6.894m |

Observation 17: For generalization performance with different assumptions on AI/ML assisted positioning, the positioning accuracy is seriously degraded.

      - AI/ML model generalization performance with mix-train

In Section 3.2.1.3, the simulation results of AI/ML assisted positioning model generalization is seriously degraded. We investigate the mix-train method to solve this problem, which is to mix the dataset from different assumptions for model training. In our simulations, the AI/ML model is trained with the mix dataset and tested separately. We keep the size of mix-training dataset is same as the size of fine-tuning dataset to comparing the advantages of two schemes.

        - Mix-train with different clutter parameters

In this section, we employ the mix-train method by combining 20000 data samples with a clutter parameter of {60%, 6m, 2m} and 1600 data samples with a clutter parameter of {40%, 2m, 2m} to for model training. For testing, we used 1600 data samples with a clutter parameter of {40%, 2m, 2m} and 1600 data samples with a clutter parameter of {60%, 6m, 2m}. The details of dataset and the simulation results for an AI/ML model mix-train are provided in Table 18 and also the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.51m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 3.11m to 1.51m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of clutter parameter {60%, 6m, 2m}, the positioning accuracy is degraded from 0.655m to 1.35m compared with AI/ML model training and testing only with InF-DH {60%, 6m, 2m}.

Table 18: Evaluation results for AI/ML model mix-trained in different clutter parameters deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | InF-DH {60%,6m,2m} and InF-DH{40%,2m, 2m} | InF-DH{40%,2m, 2m} | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.51m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | InF-DH {60%,6m,2m} and InF-DH{40%,2m, 2m} | InF-DH {60%,6m,2m} | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.35m |

Observation 18: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.51m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 3.11m.

        - Mix-train with different network synchronization assumptions

In this section, we employ the mix-train method by combining 20000 data samples with perfect synchronization error and 1600 data samples with 50ns synchronization error for model training. For testing, we used 1600 data samples with perfect synchronization error and 1600 data samples with 50ns synchronization error. The details of dataset and the simulation results for an AI/ML model mix-train are provided in Table 19 and also the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.81m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 12.8m to 1.81m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of perfect network synchronization, the positioning accuracy is degraded from 0.655m to 1.32m compared with AI/ML model training and testing only with perfect network synchronization.

Table 19: Evaluation results for AI/ML model mix-trained in different synchronization error deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | Ideal synchronization and network synchronization error 50ns | network synchronization error 50ns | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.81m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | ToA | Ideal synchronization and network synchronization error 50ns | Ideal synchronization | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.32m |

Observation 19: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.8m.

        - Mix-train with different scenarios

In this section we employ the mix-train method by combining 20000 data samples with InF-DH {60%, 6m, 2m} and 1600 data samples with InF-SH {20%, 2m, 10m} for model training. For testing, we used 1600 data samples with InF-SH {20%, 2m, 10m} and 1600 data samples with InF-DH {60%, 6m, 2m}. The details of dataset and the simulation results for an AI/ML model mix-train are provided in Table 20 and also the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.467m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 6.894m to 1.467m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of InF-DH {60%, 6m, 2m}, the position accuracy is degraded from 0.655m to 1.342m compared with AI/ML model training and testing only with InF-DH {60%, 6m, 2m}.

Table 20: Evaluation results for AI/ML model mix-trained in different scenarios deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-SH {20%,10m,2m} | InF-SH {20%,10m,2m} | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.467m |
| Type: CIR; Size: 18*1*256 | Type:position Size:1*2 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} and InF-SH {20%,10m,2m} | InF-DH {60%,6m,2m} | 20000 and 1600 | 1600 | 0.7M | 60.8M FLOPs | 1.342m |

Observation 20: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.467m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.894m.

      - AI/ML model generalization performance with fine-tuning

In Section 3.2.1.3, we investigate the generalization capability of the AI/ML model and observe that the positioning accuracy will degrade when the training and testing datasets have different clutter parameters or network synchronization assumptions. This aspect is important, as it can be computationally and storage-intensive to train an AI/ML model for every scenarios or assumptions. To enhance the accuracy of the AI/ML model when training and testing datasets have different assumptions, we explore the potential of fine-tuning the model's performance

We also investigate the impact of the quantity of fine-tuning data on the positioning accuracy. In our simulations, we vary the size of the fine-tuning dataset size, which includes: 800, 1600, 2400 and 3200 labels. The simulation results are presented in the subsequent sections.

        - Fine-tuning with different clutter parameters

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of clutter parameter {60%, 6m, 2m}. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset that assumes clutter parameter {40%, 2m, 2m}. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 800, 1600, 2400, 3200 with the aforementioned clutter parameter of {40%, 2m, 2m} is provided in Table 21 and also the accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.2.1.3, the performance is significantly improved by the fine-tuning of the AI/ML model using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}.

Table 21: Evaluation results for AI/ML-assisted model different clutter parameters deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type:Cir Size:18*1*256 | Type:ToA Size1*18 | ToA with 100% ground truth label | clutter param: {60%,6m,2m} | clutter param:{40%,2m,2m} | clutter param:{40%,2m,2m} | 20000 | 800 | 400 | 0.7M | 60.8M FLOPs | 1.11m |
| Type:Cir Size:18*1*256 | Type:ToA Size1*18 | ToA with 100% ground truth label | clutter param: {60%,6m,2m} | clutter param:{40%,2m,2m} | clutter param:{40%,2m,2m} | 20000 | 1600 | 400 | 0.7M | 60.8M FLOPs | 0.97m |
| Type:Cir Size:18*1*256 | Type:ToA Size1*18 | ToA with 100% ground truth label | clutter param: {60%,6m,2m} | clutter param:{40%,2m,2m} | clutter param:{40%,2m,2m} | 20000 | 2400 | 400 | 0.7M | 60.8M FLOPs | 0.92m |
| Type:Cir Size:18*1*256 | Type:ToA Size1*18 | ToA with 100% ground truth label | clutter param: {60%,6m,2m} | clutter param:{40%,2m,2m} | clutter param:{40%,2m,2m} | 20000 | 3200 | 400 | 0.7M | 60.8M FLOPs | 0.87m |

Observation 21: For AI/ML-assisted positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

        - Fine-tuning with different network synchronization assumptions

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of perfect network synchronization. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset, AI/ML model and evaluation results are given in the accompanying excel sheet. The simulation results for an AI/ML model fine-tuned by 800, 1600, 2400, 3200 dataset size with network synchronization error is provided in Table 22 and also the excel sheet submitted together. Compared to the positioning accuracy in Section 3.2.1.3, the positioning accuracy is significantly improved from 12.8m to 1.87m by the fine-tuning of the AI/ML model with a small dataset with size of 3200 with network synchronization error.

Table 22: Evaluation results for AI/ML-assisted model for different network synchronization assumptions deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | w/o network synchronization error | w/ network synchronization error | w/ network synchronization error | 20000 | 800 | 400 | 0.7M | 60.8M FLOPs | 2.99m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | w/o network synchronization error | w/ network synchronization error | w/ network synchronization error | 20000 | 1600 | 400 | 0.7M | 60.8M FLOPs | 2.47m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | w/o network synchronization error | w/ network synchronization error | w/ network synchronization error | 20000 | 2400 | 400 | 0.7M | 60.8M FLOPs | 2.19m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | w/o network synchronization error | w/ network synchronization error | w/ network synchronization error | 20000 | 3200 | 400 | 0.7M | 60.8M FLOPs | 1.87m |

Observation 22: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

        - Fine-tuning with different scenarios

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of InF-DH. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with InF-SH. The details of the dataset, AI/ML model simulation results for an AI/ML model fine-tuned by 800, 1600, 2400, 3200 dataset size with InF-SH is provided in Table 23 and also the accompanying excel sheet. Compared to the positioning accuracy in Section 3.2.1.3, the positioning accuracy is significantly improved from 6.894m to 2.41m by the fine-tuning of the AI/ML model with a small dataset with size of 3200 with the assumption of different scenarios.

Table 23: Evaluation results for AI/ML-assisted model for different scenarios deployed on UE/LMF-side, with model generalization and fine-tuning, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | InF-DH{60%,6m2m} | InF-SH{20%,10m,2m} | InF-SH{20%,10m,2m} | 20000 | 800 | 400 | 0.7M | 60.8M FLOPs | 3.226m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | InF-DH{60%,6m2m} | InF-SH{20%,10m,2m} | InF-SH{20%,10m,2m} | 20000 | 1600 | 400 | 0.7M | 60.8M FLOPs | 2.82m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | InF-DH{60%,6m2m} | InF-SH{20%,10m,2m} | InF-SH{20%,10m,2m} | 20000 | 2400 | 400 | 0.7M | 60.8M FLOPs | 2.547m |
| Type:CIR Size:18*1*256 | Type:ToA Size:1*18 | ToA with 100% ground truth label | InF-DH{60%,6m2m} | InF-SH{20%,10m,2m} | InF-SH{20%,10m,2m} | 20000 | 3200 | 400 | 0.7M | 60.8M FLOPs | 2.41m |

Observation 23: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

    - AI/ML model performance with labeling error

It was concluded in RAN1#112 meeting to study the impact of labelling error on positioning accuracy. To achieve this, we introduce errors in the x-axis and y-axis of the labels, resulting in inaccurate labels. We then set the value of L as 1m, 3m and 5m. The error-added UE coordinate POS' is used to generate TOA' with the base station coordinates (no error). TOA' was then used as a label for AI-assisted positioning to train the model. The simulation result is provided in Table 24 and also the accompanying excel sheet.

Table 24: Evaluation results for direct AI/ML positioning model for label error deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label Error | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | L=1m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 1.901m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | L=3m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 4.392m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | L=5m | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 6.753m |

Observation 24: When AI/ML model is trained with different value of label error, as value of L increases, AI/ML–assisted positioning error increases approximately in proportion to L.

    - AI/ML model performance with different model input

According to the conclusion of the RAN1#112 meeting, study the model input, considering the tradeoff among model performance, model complexity and computational complexity.

      - Different number of TRP as model input

For the simulation of reducing the number of TRP, we evaluate two cases for AI/ML-assisted positioning.

The first case is to select 9 fixed TRPs and train a model to cover the entire simulation area. In inference phase, this model is used to cover the entire evaluation area.

The second case is to divide the original area into two regions with equal size, and two models are trained based on the region-specific dataset separately. In inference phase, each model is used to cover the corresponding region, and the whole evaluation area can be covered by these two models.

In this simulation, since NTRP = 9, the measurement size is NTRP*Nport*NT*2*64 = 147456bit per UE.

        - One model covers entire area

In our simulation, we select 9 TRPs out of 18 TRPs for model input. The selected TRPs are with the ID of {3, 4, 5, 9, 10, 11, 15, 16, 17}. The TRP are numbered according to the latest agreement regarding the TRP indices in RAN1#112bis-e. Specifically, two designs are considered for this case.

In our first design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP) that provides measurements as model input. The horizontal position accuracy is slightly degraded from 0.655m@90% to 0.97m@90% of CDF percentile. The computational complexity is reduced from 60.8M FLOPs to 38.46M FLOPs. The details of dataset and the simulation results are provided in the first row of Table 25 and also the accompanying excel sheet.

In our second design, Approach 1-A is applied, i.e. 9 fixed TRPs provides the effective input value to the model and the input of the other 9 TRPs are zero-padded. The horizontal position accuracy is slightly degraded from 0.655m@90% to 0.6577m@90% of CDF percentile. The details of dataset and the simulation results are provided in the second row of Table 25 and also the accompanying excel sheet.

We also observe that if the model input dimension is reduced when only part of TRPs are used as input, the FLOPs is reduced compared to the case when all TRPs are used. Additionally, the model overhead is reduced while maintaining similar horizontal positioning accuracy.

Table 25: Evaluation results for AI/ML model for fixed 9 TRPs of one model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 9*1*256 | Type:ToA Size:1*9 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 38.64M FLOPs | 0.97m |
| Type: CIR; Size: 18*1*256 | Type:ToA Size:1*18 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 60.8M FLOPs | 0.6577m |

Observation 25: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 26: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

        - Two models each covers half the area

In this case, we divide the entire simulation area into two regions, named as region A and region B, by equal proportion. The TRP ID of region A is {0, 1, 2, 3, 4, 5, 6, 7, 8}, and the TRP ID of region B is {9, 10, 11, 12, 13, 14, 15, 16, 17}. The TRP are numbered according to the latest agreement regarding the TRP indices in RAN1#112bis-e. Similarly, two designs are considered for this case.

In our first design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP=9) that provides measurements as model input. The horizontal position accuracy is degraded from 0.655m@90% to 1.153m@90% of CDF percentile. The details of dataset and the simulation result are provided in Table 26 and also the accompanying excel sheet.

Table 26: Evaluation results for AI/ML model for fixed 9 TRPs of two models deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 9*1*256 | Type:ToA Size:1*9 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 10000*2 | A:942 B:658 | 0.7M*2 | 38.64M FLOPs | 1.153m |

Compared to the results of using single model to cover the entire area in previous sub-section, the performance of using two models to cover the entire area is slightly degraded. The possible reason is that the training dataset of each model in this case is reduced by half, leading to the insufficient training.

Observation 27: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area.

      - Different CIR tap as model input

In this section, we evaluate the positioning performance and the computation complexity of the AI/ML model by reducing the number of CIR taps. In the simulation, only the first 128, 64, and 32 CIR taps from the 256 CIR taps in time domain are used as the model input respectively.

So the values of measurement size are 147456bits per UE (NT=128), 73728bits per UE (NT=64), 36864bits per UE (NT=32).

The details of dataset and the simulation results are provided in 27 and also the accompanying excel sheet.

Table 27: Evaluation results for AI/ML model for different CIR taps number as model input deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 18*1*128 | Type:ToA Size:1*18 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 30.39M FLOPs | 0.8742m |
| Type: CIR; Size: 18*1*64 | Type:ToA Size:1*18 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 15.198M FLOPs | 0.70m |
| Type: CIR; Size: 18*1*32 | Type:ToA Size:1*18 | UE’s position with 100% ground truth label | InF-DH {60%,6m,2m} | InF-DH {60%,6m,2m} | 20000 | 1600 | 0.7M | 7.60M FLOPs | 1.104m |

Observation 28: For AI/ML-assisted positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

    - AI/ML model for LOS/NLOS identification

LOS/NLOS identification is to assist the non-AI/ML positioning algorithm. The following is our simulation content and results for different input and model generalization performance. In our simulation, we select InF-DH {40%, 2m, 2m} as baseline. Because scenario in InF-DH {40%, 2m, 2m} probability of LOS/NLOS is approximately 50%, which is suitable for the AI/ML model training.

      - AI/ML model for LOS/NLOS identification performance with different model inputs

The CIR data format is 1*256*2. The AI/ML model is trained with the CIR of InF-DH {40%, 2m, 2m}. The details of dataset and the simulation results for an AI/ML model test by 6480 dataset size with clutter parameter of InF-DH {40%, 2m, 2m} is provided in Table 28.

The AI/ML model is trained with the PDP of InF-DH {40%, 2m, 2m}. The details of dataset and the simulation results for an AI/ML model test by 6480 dataset size with clutter parameter of InF-DH {40%, 2m, 2m} is provided in Table 28.

In this simulation, when PDP is used as model input, the measurement size is NTRP*Nport*NT*1*64 = 147456bit per UE.

Table 28: Evaluation results for AI/ML-based LOS/NLOS identification model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | LOS or NLOS identification accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 1*256*2 | LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | {40%, 2m, 2m} | {40%, 2m, 2m} | 58320 | 6480 | 0.7M | 18*8M FLOPs | 94.2% |
| Type:PDP; Size: 1*256*1 | LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | InF-DH{40%, 2m, 2m} | InF-DH{40%, 2m, 2m} | 58320 | 6480 | 0.7M | 18*8M FLOPs | 94.89% |

Observation 29: For AI/ML-assisted positioning with LOS/NLOS identification, the performance is similar with model inputs between PDP and CIR.

      - AI/ML model for LOS/NLOS identification generalization performance

In this section, we investigate the generalization performance of the AI/ML-based LOS/NLOS identification. Specifically, the AI/ML model is trained with PDP as input and dataset from InF-DH {40%, 2m, 2m}, and tested on a dataset of size 6480 with a clutter parameter of InF-DH {60%, 6m, 2m}. The results show that the LOS or NLOS identification accuracy reaches 96.23%, which is even slightly better than the original result. The reason is that the InF-DH {60%, 6m, 2m} has a much higher probability of NLOS which leads to imbalanced study. Additionally, the AI/ML model is trained with PDP as input from dataset of InF-DH {40%, 2m, 2m}, and tested on a dataset of size 6480 with clutter parameter of InF-SH {20%, 10m, 2m}, The LOS/NLOS identification accuracy can be 91%. The details of dataset and the simulation results are provided in Table 29. Given the good generalization performance, mix-training and fine-tuning methods are not necessary.

Table 29: Evaluation generation performance results for AI/ML-based LOS/NLOS identification model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | LOS or NLOS identification accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type:PDP; Size: 1*256*1 | Probability of LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-DH {40%, 2m, 2m} | 58320 | 6480 | 0.7M | 8M | 94.89% |
| Type:PDP; Size: 1*256*1 | Probability of LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-DH {60%,6m,2m} | 58320 | 6480 | 0.7M | 8M | 96.23% |
| Type:PDP; Size: 1*256*1 | Probability of LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-SH {20%,10m,2m} | 58320 | 6480 | 0.7M | 8M | 91% |

Observation 30: For the AI/ML-assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

  - Evaluation of model monitoring

For AI/ML-based positioning, a PRU with known location can generate a ground truth label for monitoring the AI/ML model. However, collecting other ground truth labels generated by a normal UE may be challenging. Therefore, AI/ML model monitoring without ground truth label should be studied.

The label-based method allows for direct evaluation of AI/ML model performance through the use of coordinates. The statistical prediction error of the CDF is already sufficient to evaluate the AI/ML model performance.

For the label-free method, we provide the method of relative displacement between time T1 and time T2 by the motion sensor to monitor the AI/ML model performance. UE using one or more motion sensors obtains the movement information, and the movement information comprises displacement results estimated as an ordered series of points. The L1 vector was obtained by comparing the movement information at time T1 and time T2 of the kinematic apparatus. The L2 vector was obtained by comparing the predicted values of the AI/ML model at time T1 and time T2. The correlation of Vector L1 and vector L2 are evaluated. More details on this monitoring method can be found in our companion paper [2].

In this section, we provide the results of model monitoring method. We used the simulation assumptions with a poor generalization performance to evaluate our monitoring scheme.

    - Direct AI/ML positioning

For direct AI/ML positioning, we used the AI/ML model in Section 3.1.1.1 trained with the assumption of InF-DH with clutter parameter {60%, 6m, 2m}, and the AI/ML model are monitored with InF-DH {60%, 6m, 2m}, InF-DH {40%, 2m, 2m} and InF-SH {20%, 2m, 10m} respectively. The CDF plots of the |L1-L2| vectors in different test scenarios are shown below.

Figure 1: CDF of relative displacement method for direct AI/ML positioning.

As can be observed from the above figure, the error of |L1-L2| is small when the training dataset and testing dataset are from the same scenario. The error of |L1-L2| significantly increases when the training dataset and testing dataset are from different scenarios, especially in InF-SH with clutter parameter of {20%, 2m, 10m}. Such result validates the possibility of using relative displacement method to monitor the model for direct AI/ML positioning.

Observation 31: For direct AI/ML positioning, the relative displacement method can monitor the model performance.

    - AI/ML-assisted positioning

For AI/ML-assisted positioning, we used the AI/ML model in Section 3.2.1.1 trained with the assumption of InF-DH with clutter parameter {60%, 6m, 2m}, and the AI/ML model are monitored with InF-DH {60%, 6m, 2m}, InF-DH {40%, 2m, 2m} and InF-SH {20%, 2m, 10m} respectively. The CDF plots of the |L1-L2| vectors in different test scenarios are shown below.

Figure 2：CDF of relative displacement method for AI/ML-assisted positioning.

As can be observed from the above figure, the error of |L1-L2| is small when the training dataset and testing dataset are from the same scenario. The error of |L1-L2| significantly increases when the training dataset and testing dataset are from different scenarios, especially in InF-SH with clutter parameter of {20%, 2m, 10m}. Such result validates the possibility of using relative displacement method to monitor the model for AI/ML-assisted positioning.

Observation 32: For AI/ML-assisted positioning, the relative displacement method can monitor the model performance.

- Conclusions

In this contribution, we provide our evaluation results on AI/ML for positioning. The observations are summarized as follows:

Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal positioning accuracy is 0.58m@90% of CDF percentile.

Observation 2: For directly estimating UE’s positioning with network synchronization error (truncated Gaussian distribution of 50 ns), the horizontal positioning accuracy is 0.84m@90% of CDF percentile.

Observation 3: When the direct AI/ML positioning is applied to different scenarios or conditions, the generalization performance can be significantly affected, leading to a significant degradation in positioning accuracy.

Observation 4: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.77m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 2.64m.

Observation 5: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 3.04m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.6m.

Observation 6: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.48m.

Observation 7: For direct AI/ML positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 8: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 9: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 10: When AI/ML model is trained with different value of label error, as value of L increases, direct AI/ML positioning error increases approximately in proportion to L.

Observation 11: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 12: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

Observation 13: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area.

Observation 14: For direct AI/ML positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

Observation 15: For AI/ML-assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

Observation 16: For AI/ML-assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

Observation 17: For generalization performance with different assumptions on AI/ML assisted positioning, the positioning accuracy is seriously degraded.

Observation 18: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.51m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 3.11m.

Observation 19: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.8m.

Observation 20: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.467m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.894m.

Observation 21: For AI/ML-assisted positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 22: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 23: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 24: When AI/ML model is trained with different value of label error, as value of L increases, AI/ML–assisted positioning error increases approximately in proportion to L.

Observation 25: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 26: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

Observation 27: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area.

Observation 28: For AI/ML-assisted positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

Observation 29: For AI/ML-assisted positioning with LOS/NLOS identification, the performance is similar with model inputs between PDP and CIR.

Observation 30: For the AI/ML-assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

Observation 31: For direct AI/ML positioning, the relative displacement method can monitor the model performance.

Observation 32: For AI/ML-assisted positioning, the relative displacement method can monitor the model performance.

- References

- Chairman’s Notes RAN1#112bis-e, e-Meeting, April 17th – April 26th, 2023.

- R1-2304727, Discussion on AI/ML for positioning enhancement, CATT, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023.

- Annex

Table 30: Simulation assumptions in our simulation

| Parameters | Values |
| --- | --- |
| Scenario | 1nF-DH |
| Hall size | 120x60 m |
| Total gNB TX power, dBm | 24 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| Penetration loss | 0 dB |
| Number of floors | 1 |
| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area is selected from the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. |
| UE antenna height | Baseline: 1.5 m |
| Min gNB-UE distance(2D), m | 0 m |
| gNB antenna height | Baseline: 8 m |
| Clutter parameters: {density , height ,size } | High clutter density: - {60%, 6m, 2m} - {40%, 2m, 2m} |
| Carrier frequency, GHz | 3.5 GHz |
| Bandwidth, MHz | 100 MHz |
| Subcarrier spacing, kHz | 30 kHz for 100 MHz |
| Distribution of UE location | Uniform distribution |
| Network synchronization | (1) Perfect network synchronization (2) Network synchronization with a truncated Gaussian distribution of 50 ns |