3GPP TSG RAN WG1 #113	                                            R1-2304752

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.9.2

Source:	CATT

Title:	Further discussion on Network verified UE location for NR NTN

Document for:	Discussion and Decision

Introduction

As described in 3GPP TSG RAN WG1#112b e-meeting, the multi-RTT method in NTN was discussed elaborately and the following agreement had been concluded [1].

Agreement For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements: - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted - Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS - Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4 FFS: One or multiple SRS can be used in determining the arrival time FFS: Additional enhancement including additional information to be reported, if justified Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

And a proposed working assumption was also listed as following.

Proposed working assumption In NTN, for the definition of the gNB Rx – Tx time difference reported to the LMF, further discuss the following two options for the reference point of the definition: Option 1: The uplink time synchronization reference point Option 2: The reference point is on the gNB Note: if option 1 and option 2 are both not feasible, RAN1 can consider defining the reference point on the satellite.

In this document, following the discussion in last RAN1#112b e-meeting, the continuous analysis and proposals will be further studied for the topic of Multi-RTT applied in UE location verification in Non-terrestrial Networks.

Discussion of measuring reference point for gNB Rx-Tx

The issue of reference point was still discussed in the last meeting, and there was no agreement to achieve. Two options were listed, one is the uplink time synchronization reference point, and another is on gNB. The note mentioned that if both options are not feasible, RAN1 can consider defining the reference point on the satellite. In NR, the reference point is treated as the antenna of TRP which is defined as transmission and reception point (TRP) [2]. The TRP is the real point to participate the positioning measurements in terrestrial network. But in the NTN, the satellite with transparent load is instead of TRP. So it is reasonable to treat satellite as the reference point for NTN networks even if the transparent satellite could not actually measure the RTT value.

For fixing the position of UE ultimately, the coordinates of satellite and the RTTs between satellite and UE are involved in LMF, not gNB or uplink time synchronization point. Thus, the satellite plays a critical role in Multi-RTT for NTN no matter the reference point at gNB or uplink time synchronization point. In order to distinguish the actual meaning of reference point, the satellite may be treating as calculating point instead. And a parameter that actually participate the calculation of positioning can be defined as Sat Rx-Tx time difference which can be expressed as TSat-RX – TSat-TX.

Where:

- TSat-RX is the satellite received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time.

- TSat-TX is the satellite transponding timing of downlink subframe #j with PRS from gNB to UE.

- Multiple SRS resources can be used to determine the start of one subframe containing SRS.

- The reference point for TSat-RX shall be:

- the Rx antenna of satellite (i.e. the centre location of the radiating region of the Rx antenna).

- The reference point for TSat-TX shall be:

- the Tx antenna of satellite (i.e. the centre location of the radiating region of the Tx antenna).

Proposal 1: No matter which option of reference point is chosen, the coordinates of satellite and service link RTTs are essential factors that should not be ignored in NTN position solving. And Sat Rx-Tx should be involved to derive the actual RTT between satellite and UE.

Analysis of time difference measurements of Multi-RTT in NTN

As discussed in last e-meeting, the reporting details of UE and gNB receive-transmit time difference measurements were still under controversy. There are 3 Alts listed for an agreement in last e-meeting. In this section, the 3 Alts will be analyzed in detail.

  - Analysis of Alt-1

As concluded in last meeting, the Alt-1 was expressed as following.

- Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

The option 3 of UE Rx-Tx time difference can be described as following.

Option 3: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options: - Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i - Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset - Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity.

And according to the TS 38.215, the definition of gNB Rx-Tx time difference is shown in following.

The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104: the Tx Transceiver Array Boundary connector.

In NTN, there are several parts to consider in an entire Multi-RTT measuring procession, such as gNB, uplink time synchronization reference point (ULTSRP), satellite and UE. The timing sequence of Multi-RTT can be shown as following figure.

Figure 1 illustration of Multi-RTT method in NTN for Alt-1

From the figure above, gNB sent DL subframe#i with PRS at t1. And UE received the DL subframe #i at t3. UE sent UL subframe#j with SRS at t2, and gNB received the UL subframe#j at t4. According to the Alt-1, UE Rx-Tx time difference can be express as option3. Therefore, the additional offset with legacy time difference is expressed the UE Rx-Tx in NTN. There are 3 sub-options in option3.

For option 3-1, the offset is reported by integer value of milliseconds between transmit timing of UL subframe#i and receive timing of DL subframe#i by UE. If the UE triggered the autonomous TA adjustment between transmit timing UL subframe#i and receive timing of DL subframe#i, then the TA adjustment should be considered in RTT calculation. And in this option, it cannot avoid autonomous TA adjustment because UE may not know the PRS transmission with subframe#i.

For option 3-2, UE report the index of the subframe#j that is closet in time to the subframe#i received from the TP. According to specification, one frame contains 10 subframes. If the delay is exceeding the length of one entire frame and across multi frames, then only reported index of subframe may involve the ambiguity, and the index of frame should also be reported at the same time.

For option 3-3, TA report may be malicious, and it should be considered cautiously. By the way, this method cannot avoid autonomous TA adjustment happening between received timing of DL subframe#i and transmit timing of UL subframe#i.

The gNB Rx-Tx time difference reuses the definition of TS 38.215 in Alt-1. According to definition in TS 38.215, the RX-Tx time difference of gNB can illustrate in figure above. The TgNB-RX is the receive timing of UL subframe#j with SRS. The TgNB-TX is the transmit time of DL subframe#j+m because it is the closet DL subframe to UL subframe#j. So the gNB Rx-Tx time difference is equal to TgNB-RX - TgNB-TX which is the same as the legacy gNB Rx-Tx time difference. The offset at the gNB side will not be presented. As shown in the figure above, the offset of gNB can be derived by the index of UL subframe#j and the corresponding DL subframe#j at gNB side. However, if the delay is longer than frame length or across multi frames, there will has ambiguity involved when only index of subframe considered. And the actual offset of gNB cannot derive correctly just based on index of subframe.

Proposal 2: The time range of gNB Rx-Tx across multi frames should be considered seriously when reused the definition of gNB Rx-Tx time difference in NTN.

  - Analysis of Alt-2

As concluded in last meeting, the Alt-2 was expressed as following.

- Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS

The option 2 of UE Rx-Tx time difference can be described as following.

Option 2: - For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. - FFS: details of report and the definition of UE Rx-Tx time difference

The illustration of Multi-RTT procession in NTN for Alt-2 can be described in following figure.

Figure 2 illustration of Multi-RTT method in NTN for Alt-2

According to the option2 of UE Rx-Tx time difference, the time difference between t2 and t3 can be indicated as the figure. There are various ways to indicate the time difference of t3-t2. One simplest solution is indicating it as the legacy method based on granularity of 1 Tc. However, if the time range is larger, such as RTT of GSO, more bits will be required to indicate the time difference. And the legacy parameter should be expanded to adapt the NTN scenario. A large number of bits to indicate the time difference are inefficient, when the positioning is just applied in NR network. So it is necessary to involve a NTN specific parameter to indicate the UE Rx-Tx time difference. According to Note2, it is suggested that the LMF can calculate the RX-Tx time difference based on time stamps of PRS and SRS. Then, the stamps of reception PRS and transmission SRS in UE side should be reported to LMF. In order to achieve high precise, the granularity of time stamps should be tiny, such as 1 Tc. Thus, there are two elaborate time stamps additionally to deliver to LMF. The overhead of this method should be considered seriously.

Considering the long range of UE Rx-Tx in NTN and the reusing of legacy parameter in NR, a specific NTN parameter will be designed to cooperate with legacy parameter in NR to express the UE Rx-Tx time difference. The detail method is proposed to indicate the UE Rx-Tx time difference that is shown in following.

Figure 3 Detail indication of UE Rx-Tx time difference in NTN

As shown in the figure above, the count of frame and sub-frame is involved to expand the range of UE Rx-Tx time difference. The TUE-Rx-TUE-Tx can be express as: count of frame + count of subframe + legacy Rx-Tx parameter. If the TUE-Tx is ahead of TUE-Rx at the gNB side, so the TUE-Rx-TUE-Tx is positive. In some situation, the TUE-Tx is behind of TUE-Rx, the TUE-Rx-TUE-Tx is negative. In order to indicate the polarity of the Rx-Tx time difference, 1 bit for polarity indicated is necessary. Finally, the new style of UE Rx-Tx time difference can be shown as following.

Figure 4 Detail reporting of UE Rx-Tx time difference in NTN

According to the detail indication of UE Rx-Tx time difference, the entire parameter contents two major aspects, one is legacy Rx-Tx parameter and another is additional information for NTN. If the positioning method is only applied in NR networks, the legacy Rx-Tx parameter is used with no additional information for NTN. There is no extra alternation to current specification. If the positioning method is applied in NTN networks, the additional information for NTN is needed and joint with legacy Rx-Tx parameter to indicate the real UE Rx-Tx time difference.

As analysis in last section, if reusing legacy definition of gNB Rx-Tx in TS 38.215, the offset of gNB cannot be correctly expressed when the time range exceeding one frame length.

  - Analysis of Alt-3

As concluded in last meeting, the Alt-3 was expressed as following.

- Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4

The option 2 of UE Rx-Tx time difference can be described as following.

Option 2: - For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. - FFS: details of report and the definition of UE Rx-Tx time difference

The option 4 of gNB Rx-Tx time difference can be described as following.

Option4: - For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS.

The illustration of Multi-RTT procession in NTN for Alt-3 can be described in following figure.

Figure 5 illustration of Multi-RTT method in NTN for Alt-3

The Alt-3 is more clearly in definition of Rx-Tx in both UE and gNB side. The indication of time difference is approaching the real essence of Rx-Tx meaning. From the figure, according to the Alt-3, the UE Rx-Tx time difference is equal to t3-t2, and the gNB Rx-Tx time difference is equal to t4-t1. The RTT between gNB and UE is (t4-t1)-(t2-t3).

According to the same reason, the new indication of UE Rx-Tx time difference can be reused in gNB Rx-Tx time difference. The illustration of gNB Rx-Tx time difference can be depicted as following figure.

Figure 6 illustration of the new method to express the gNB Rx-Tx time difference

The additional information for gNB Rx-Tx time difference can reuse the content of figure4.

Proposal 3: Alt-3 on Rx-Tx time difference is more proper for NTN. And NTN specific signaling for UE RX-TX signaling indication is introduced on top of legacy RX-TX 21 bits as following:

  - 1bit for polarity

  - 8bit for count of frame.

  - 4bit for count of sub-frame

Additional information for LMF

Except for gNB and UE Rx-Tx time difference reporting to LMF, the additional information is needed to calculate the position for LMF in NTN, such as coordinates of satellite. The RTTs between satellite and UE and the coordinates of satellite are essential issues for LMF calculating UE position.

For RTTs calculating, compared with the terrestrial network, satellite involved in NTN network brings the long propagating delay and motivation in Multi-RTT positioning. In order to adapt the positioning calculation mechanism, the Sat Rx-Tx time difference was defined to simplify the calculation. So the RTT between satellite and UE can be calculated by Sat Rx-Tx and UE Rx-Tx. There are two options to calculate the Sat Rx-Tx time difference. One option is that the gNB deduces the Sat Rx-Tx time difference and reports to LMF. Another option is that the LMF compute the Sat Rx-Tx time difference based on reported information. In order to illustrate how to calculate the Sat Rx-Tx time difference, the case of figure 5 is taken for instance. In this case, the TgNB-Tx is ahead of TgNB-Rx. As the definition, Sat Rx-Tx time difference is equal to TSat-Rx-TSat-Tx. In figure 5, the RTT of feedlink which can be indicated by RTTgNB-Sat should be considered in calculating TSat-Rx-TSat-Tx. In order to calculate the feedlink RTT, the TAcommon, TAcommonDrift, TAcommonDriftVariation, K-Mac and epoch time were indicated by network in 3GPP. So the RTTgNB-Sat can be approximated by the following equation.

RTTgNB-Sat=K-Mac+TAcommon+TAcommonDrift×TgNB-RX+TgNB-TX-2t02+TAcommonDriftVariation×TgNB-RX-t02+TgNB-TX-t02/2

where t0 is the epoch time. And it is valid during the procedure of Multi-RTT measuring. Then, then Sat Rx-Tx time difference can be calculated as following.

TSat-RX-TSat-TX=TgNB-RX-TgNB-TX-RTTgNB-Sat

When the Sat Rx-Tx time difference is derived, the RTT of the satellite to UE can be calculated by Sat Rx-Tx and UE Rx-Tx according to method of Multi-RTT solution.

For coordinates of satellite, these can be derived by the ephemeris of satellite and the timing of RTT measuring. There are also two options to get the coordinates of satellite. One way is that gNB deduces the coordinates and sends to LMF. Another way is that LMF calculates the coordinates of satellite based on the ephemeris and so on.

Proposal 4: The RTTs (between satellite and UE) and coordinates of satellite which are essential for LMF to fix position have two options to obtain.

- Option 1: gNB deduces the Sat Rx-Tx time difference and coordinates of satellite and sends to LMF, and then LMF calculates RTTs (between satellite and UE) and fixes the UE position.

- Option 2: The gNB sends the assistant information to LMF which includes gNB Rx-Tx time difference measurements, TAcommon, TAcommonDrift, TAcommonDriftVariation, epoch time, ephemeris and so on. The LMF calculates the RTTs (between satellite and UE) and deduces the coordinates of satellite based on the collected information to fix the UE position.

Discussion of UE autonomous adjustment of TA

In NTN, the UE autonomous adjusting TA is supported. When UE acts autonomous adjusting TA, the adjustment of TA cannot be pre-known by network. According to TS 38.213, for 15kHz subcarriers, the minimum adjustment of TA is 16Ts, almost 0.52ms. It will be involved a large error if the UE autonomous adjusting TA happens between the reception PRS and transmission SRS at UE side. Therefore, the impact of UE autonomous adjusting TA cannot be ignored in NTN.

According figure 1, for Alt-1, if the UE autonomous adjusting TA happens between UL subframe#i and t3, the impact of autonomous adjusting TA cannot be ignored. Meanwhile, for Alt-2 and Alt-3, if the UE autonomous adjusting TA happens between t2 and t3, the impact of autonomous adjusting TA should be considered seriously. Nevertheless, if the UE autonomous adjusting TA doesn’t happen in the intervals mentioned above, it has no effect on RTT measurements.

Observation 1: The impact of UE autonomous adjusting TA cannot be ignored if the autonomous TA adjusting happens during procession of UE Rx-Tx measuring.

In order to eliminate the impact of UE autonomous adjusting TA, there is an easy way that UE should deduct the autonomous adjustment of TA from the UE Rx-Tx time difference when the autonomous TA adjusting happens in the procession of UE Rx-Tx measuring.

Proposal 5: Support UE dealing with the impact of autonomous adjusting TA on the UE Rx-Tx time difference.

Discussion of mirror position problem

In the single LEO satellite scenario, the timing measurements might be collected separately at different time points. Due to the geometry symmetry of the LEO orbit plane, a mirror position will be appeared when solved the positioning equations based on these timing measurements. Symmetry with the LEO orbit plane, there is a mirror position that has the same timing based geometry characteristic to the real UE position. According to the time based positioning methods, it cannot distinguish the real position or mirror position without additional information. Especially for the location underneath the orbit plane, the beam, or cell, or RSPR measurements all cannot distinguish the mirror position and the real position. There is a narrow belt underneath the orbit plane of LEO satellite that the mirror ambiguity is existed. This mirror ambiguity belt may be wider than 10km, and the traditional solutions mentioned before cannot solve smoothly.

Observation2: Take attention to problem of mirror ambiguity of the underneath area of LEO satellite orbit plane.

Conclusion

In this contribution, the Rx-Tx time difference of Multi-RTT method is discussed further, and a few of observations and proposals are made as follows:

Observation 1: The impact of UE autonomous adjusting TA cannot be ignored if the autonomous TA adjusting happens during procession of UE Rx-Tx measuring.

Observation2: Take attention to problem of mirror ambiguity of the underneath areas of LEO satellite orbit plane.

Proposal 1: No matter which option of reference point is chosen, the coordinates of satellite and service link RTTs are essential factors that should not be ignored in NTN position solving. And Sat Rx-Tx should be involved to derive the actual RTT between satellite and UE.

Proposal 2: The time range of gNB Rx-Tx across multi frames should be considered seriously when reused the definition of gNB Rx-Tx time difference in NTN.

Proposal 3: Alt-3 on Rx-Tx time difference is more proper for NTN. And NTN specific signaling for UE RX-TX signaling indication is introduced on top of legacy RX-TX 21 bits as following:

  - 1bit for polarity

  - 8bit for count of frame.

  - 4bit for count of sub-frame

Proposal 4: The RTTs (between satellite and UE) and coordinates of satellite which are essential for LMF to fix position have two options to obtain.

- Option 1: gNB deduces the Sat Rx-Tx time difference and coordinates of satellite and sends to LMF, and then LMF calculates RTTs (between satellite and UE) and fixes the UE position.

- Option 2: The gNB sends the assistant information to LMF which includes gNB Rx-Tx time difference measurements, TAcommon, TAcommonDrift, TAcommonDriftVariation, epoch time, ephemeris and so on. The LMF calculates the RTTs (between satellite and UE) and deduces the coordinates of satellite based on the collected information to fix the UE position.

Proposal 5: Support UE dealing with the impact of autonomous adjusting TA on the UE Rx-Tx time difference.

References

- RP-2302405, FL Summary #4: Network verified UE location for NR NTN, Thales

- TS 38.215, Physical layer measurements (Release17), 3GPP

- TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17), 3GPP

- TS 38.133, Requirements for support of radio resource management (Release 18), 3GPP