3GPP TSG-RAN WG1 Meeting #113	Tdoc R1-2304785

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.10.1

Source:	Ericsson

Title:	L1 enhancements to inter-cell beam management

Document for:	Discussion

# Introduction

L1/L2 mobility (LTM) is the main part of the Rel-18 work item [1].

There was good progress on LTM in RAN1#112bis-e, both regarding beam indication and reporting. The next level of details to be agreed for beam indication and LTM reporting are:

Next level of detail for the beam indication: how will the UE translate the TCI state index to something the UE can use. The procedure for TCI state pre-activation should also be discussed, and potentially agreed. RAN1 could also conclude there is no consensus to support Scenario 1 or 3: beam indication before/after cell-switch.

The applicable measurement RSs. So far, SSB has been agreed. It would be good to finalize which measurement RSs are supported – alternatively conclude there is no consensus to support additional measurement RSs.

Next level of detail of the reporting formats. RAN1 agreed the UE can report RSRP for L×M beams, i.e., the UE would report for L cells, and for each cell, the UE would report RSRP for M beams.

In this contribution, we discuss LTM, with focus on the above issues.

# 2	Discussion

Our understanding of the overall LTM procedure is illustrated in Figure 1.

Figure 1: Illustration of the overall LTM procedure, with a simplified illustration of the intra-RAN signaling. PDCCH-ordered RACH without RAR is assumed. Note that since RAN1 agreed that the reporting is conveyed over UCI, the serving cell configuration needs to be updated in step2.

The steps of the LTM procedure are described below:

Step1: The UE sends a L3 measurement report to the network.

Step 2: The network configures the UE with a set of candidates, and associated L1 measurement configurations. Since the subsequent lower-layer measurements are conveyed over UCI, the serving cell configuration needs to be updated.

Step 3: The UE measures and reports a set of candidates. At some point in time, the network decides to perform a cell-switch to one of the candidates.

Step 4: The network sends a PDCCH order to the UE to send a PRACH preamble towards a selected target.

Step 5: The UE transmits the PRACH towards the selected target cell.

Step 6: The target estimates the TA and forwards it to the serving cell.

Step 7: The serving cell sends the cell-switch command to the UE. The cell-switch command includes the TA to use in the new cell.

LTM shall work for intra-DU and inter-DU. The internal RAN architecture was developed to allow the DUs to operate independently, and one DU knows the configuration of its own cells, but not the configuration of cells controlled by other DUs. During a legacy handover between two cells, the handover command (including the configuration of the target) is prepared by the target cell, and transmitted to the UE by the source cell, without the source cell being aware of the configuration. This effect of the RAN architecture must be considered also during the RAN1 work for LTM. This was highlighted in the two reply LSs [9] and [10], by RAN2 and RAN3 respectively. In their response, RAN3 states that

Regarding Q3 about L1 measurement and TCI state configurations in the LS, RAN3 agreed that based on the current specification, the serving DU cannot know the measurement RS configuration and TCI state configuration of cells served by another DU.

Any possible consideration for Rel-18 on the coordination over F1 among serving DU, target DU, and CU would need clearly identified requirements from other groups.

In their response, RAN2 states that

RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e., measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam management (to the extent it is supported) may be made by the source cell. RAN2 understands that this may require cooperation among source DU, CU, target DU, and/or OAM. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

RAN3 specifies the intra-RAN signalling, which ensures that the correct information is available at the correct RAN node at the correct point in time. Note that additional signalling is required to ensure that information that is available at the UE is also available at the DU: it is not necessarily so that all the information that is available at the UE is available also at the serving DU:

RAN1 should not assume that any information that is available at the UE is also available in the serving DU.

## 2.1	Beam indication

The term “beam indication” is used in beam management and represents the signaling where the UE obtains a new QCL indication for reception of DL signals, most notably the PDCCH/PDSCH. In the context of LTM, once the new QCL indication takes effect, i.e., after the beam application time, the UE can receive the DL from the new TRP.

In RAN1#110bis-e and RAN1#111, the following agreements were reached:

Agreement (RAN1#110bis-e)

- From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart

  - Scenario 1: Beam indication before cell switch command

  - Scenario 2: Beam indication together with cell switch command

  - Scenario 3: Beam indication after cell switch command

Agreement (RAN1#111)

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

In our contribution [2], we provided arguments to support LTM also for scenario 1 and 3. However, with only two meetings left, specifying also scenario 1 and 3 is not realistic. Therefore, we propose

Do not enhance LTM to support scenario 1 or 3.

In RAN1#112bis-e, RAN1 made the following agreements:

Agreement

- Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) ,

  - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE

  - Alt. 2: By indicating Rel-17 TCI state index

Agreement

From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - TA related information (details up to the discussion in A.I. 9.10.2)

  - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell

    - Note: discussion on target SpCell is not precluded

  - Active DL and UL BWPs for the target cell

  - FFS: Triggering of aperiodic TRS transmitted from the target cell

  - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell

  - FFS: Triggering of aperiodic SRS transmission to the target cell

  - FFS: C-RNTI

- FFS: the presence of each field (i.e. always present or configurable)

Based on these agreements, the beam indication will contain a TCI state index and a BWP index, with the target to identify a TCI state, which the UE would use to prepare for the reception of PDCCH/PDSCH. We note that in general, a TCI state is configured per BWP and serving cell, and to uniquely identify a certain TCI state, the UE must also know in which serving cell the TCI state is configured: only a TCI state index and BWP index is not enough, since the candidate configuration includes multiple serving cells. One option is to also include a serving cell index in the cell switch command, but we do not see why this would be necessary: the beam indication could just as well refer to the SpCell:

The beam indication, i.e., the TCI state index and BWP index, refers to the SpCell of the indicated candidate.

Note that the UE knows the identity of the SpCell from the candidate cell configuration.

Based on the TCI state index and BWP index for the SpCell, the UE can determine TCI states for other cells using legacy functionality: the RRC parameters simultaneousU-TCI-UpdateList can be configured in the candidate, and when the UE derives the TCI state for the target SpCell, it can also apply new TCI states according to simultaneousU-TCI-UpdateList.

The reasonable interpretation of the above agreement is that the UE would use the TCI state to receive PDCCH/PDSCH. The reference signals in the TCI state should thus be appropriate for the PDCCH/PDSCH reception. From 38.214, we learn that

For the DM-RS of PDSCH, if the UE is not configured with dl-OrJointTCI-StateList, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

Hence, the allowed reference signals in the TCI state are different types of CSI-RSs. The RRC IE TCI-State is shown in Figure 2:

Figure 2: The RRC IEs TCI-State and QCL-Info (from 38.331).

Interestingly enough, a TCI state that can be used for PDCCH/PDSCH reception only contains pointers to CSI-RS resources. These CSI-RS resources are defined in other parts of the serving cell configuration. Nor does the TCI state contain the PCI or the ARFCN of the target: these are provided in the ServingCellConfigCommon. We thus realize that to determine which reference signal to use to aid the reception of PDCCH/PDSCH DMRS, the UE needs more information than what is conveyed by a TCI state:

The TCI state alone does not include sufficient information for the UE to determine which reference signal to aid the reception of PDCCH/PDSCH.

We note that the candidate cell configuration of the target contains all the information the UE needs. The straightforward option would thus be that the UE derives the TCI state and the related information from the target candidate cell:

The UE can use the target candidate configuration to determine the TCI state corresponding to the beam indication, and to determine the reference signals used to assist the reception of PDCCH/PDSCH.

We note that this would not be in line with the RAN2 assumption from RAN2#121bis-e:

RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration).

RAN2 assumed that the handling of the measurement configuration and the TCI states would be similar. However, there is an important difference: the TCI states in the candidate cell configurations would anyway be configured and used by the UE after the cell switch. The UE would assume that after the cell switch, the TCI state index in the beam indication refers to the TCI state in the serving cell configuration, which was previously the target candidate configuration:

After the RRC reconfiguration trigger by the cell-switch command, the UE assumes the TCI state index in the beam indication refers to the configuration of the SpCell.

As noted in Observation 2, the TCI states alone do not provide enough information for the UE to find a reference signal it can use to assist the demodulation of the PDCCH/PDSCH DMRS. Simply stating that the TCI states are provided separately from the candidate cell configuration, like RAN2 assumes, does not solve anything. Instead, RAN1 must describe in more detail what information the UE must be provided with.

In our understanding, the UE would need to be provided with the following information to be prepared to receive the PDCCH/PDSCH from the target:

The DL carrier frequency and the SSB SCS of the target

The SSB index of the target

The PCI of the target

This is what the UE uses to receive PDCCH/PDSCH before entering connected mode. Note that RAN1 has not agreed that the UE performs TRS tracking before the cell switch command.

With this information, the UE would

Adjust the DL carrier frequency to the DL carrier frequency of the target

Find the SSB timing from the SSB index

Correlate with the PSS/SSS generated from the PCI

Derive the QCL-TypeA properties from the SSB,

The simplest way to provide the mapping is to simply signal a mapping between a TCI state index, and {DL carrier frequency, SSB SCS, PCI, SSB index}:

The UE is provided with an explicit mapping between a TCI state index and {DL carrier frequency, SSB SCS, PCI, SSB index}.

In our understanding, Proposal 3 provides the exact information the UE needs. In legacy, these quantities are conveyed in different parts of the RRC configuration: the DL carrier frequency and the PCI are in the ServingCellConfigCommon, the TCI states and the CSI-RS configurations are in the ServingCellConfig. As an alternative to Proposal 3, larger RRC IEs could be used, but we fail to see the benefit of that. In the end, how this information is provided to the UE is up to RAN2:

How to provide the mapping between a TCI state index and {DL carrier frequency, SSB SCS, PCI, SSB index} is up to RAN2.

- As a general remark, we should avoid duplicating RRC configurations: the UE already has the information it needs, and the benefits of duplicating the information are unclear. If we duplicate the information, we unnecessarily introduce a risk for misconfiguration: the information that is provided outside the candidate cell configuration must match that inside the candidate cell configuration.

- In the agreement about the content of the cell-switch command, four FFS were included, with potential additional content:

Triggering of aperiodic TRS transmitted from the target cell. Triggering aperiodic TRS in the cell-switch command would require tight coordination between source and target, which could be challenging to achieve, especially in the inter-DU case. The target could trigger the TRS once the cell-switch is complete, and since aperiodic TRS is triggered by DCI, the performance benefit would be small.

Triggering the CSI acquisition of the target cell and reporting to the target cell. As for aperiodic TRS triggering, this would require tight coordination with small benefits. Additionally, the UL resources are controlled by the target, and the source could not freely use those.

Triggering of aperiodic SRS transmission to the target cell. As for the previous bullets, the high level of required coordination and the small benefits make this unattractive.

C-RNTI. The selection of C-RNTI is performed by the target. The natural way to signal a new C-RNTI is to make it part of the RRC configuration. There is no motivation why this should be signaled outside the RRC configuration.

- Hence, we propose

Triggering of aperiodic TRS, SRS or CSI acquisition is not included in the cell-switch command.

The content of the LTM mobility trigger information command is summarized in Table 1.

| Information | Number of bits |
| --- | --- |
| RAN2 content | 5? |
| TCI state index | 7 |
| TA | 12 |
| BWP IDs | 2+2 |

Table 1: Contents of the LTM cell-switch command. Note that we have also included a placeholder for the content specified by RAN2. This would include at least a candidate configuration index.

### 2.1.1	Support for separate TCI state index in the cell-switch command

In RAN1#112bis-e, the following working assumption was made:

Working Assumption

On the presence of beam indication within cell switch command, at least for scenario 2, following is supported:

- A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command.

Note: If scenarios 1 and 3 are agreed to be supported in R18 LTM other solutions may be considered.

One of the open issues is if the beam indication could contain two TCI state indices, or equivalently if we should support indication of separate DL/UL TCI states in the beam switch command. The unified TCI state framework can be RRC configured in one out two modes of operation “Joint DL/UL TCI state” or “Separate DL/UL TCI states”. For “Joint DL/UL TCI states” one common Joint/DL TCI state is used for both DL and UL signals/channels, while for “Separate DL/UL TCI states”, one common joint/DL TCI state is used for DL channels/signals and one common UL TCI state is used for UL signals/channels, as illustrated in Figure 3

Figure 3:  Example of “Joint DL/UL TCI” operation with a single beam pair link used for both DL and UL signals/channels (left) and “Separate DL/UL TCI” operation where one beam pair link is used for DL signals/channels and where another beam pair link is used for UL signals/channels.

Support of separate DL/UL TCI states is an optional UE capability: a UE that supports the Rel-17 TCI framework may choose not to support separate DL/UL TCI states. We should thus first clarify that joint is supported

The beam indication may contain one TCI state index, indicating a joint TCI state.

In almost all cases, the UE would use the same TCI state in UL and DL, and operation using joint TCI states would be appropriate: using the beam with the lowest pathloss is reasonable, both in UL and DL:

Using the TCI state corresponding to the lowest pathloss is reasonable and would result in the same UL and DL TCI state.

During the development of the Rel-17 TCI framework, one situation was identified where it could be beneficial to use different TCI states in UL and DL: due to regulatory requirements on MPE (maximum permissible emission), the UE may be forced to reduce the transmit power on some of its panels. This could mean that it could be better for the UE to use an UL TCI state associate with a higher pathloss if the UE could transmit with full power with that TCI state. For the DL, it would still be better to use the TCI state/panel with the lowest pathloss. Hence, in this situation, it could be envisioned that separate DL/UL TCI states could improve performance:

In cases when the UE must reduce its transmit power due to MPE regulations, separate DL/UL TCI states may be useful.

To configure the UE with different DL/UL states, the UE must inform the NW about the reduction in transmit power. For this purpose, a special report was introduced, the enhanced power headroom report. In this report, the UE may include additional reference signals, which the NW could use to select TCI states. This report is re-active: it is only triggered when the PHR of the serving cell changes. Hence, the report is inappropriate for mobility decisions, and we observe

There is no method to provide the NW with information to choose different DL and TCI states before the cell-switch.

Since there is no information available for the NW to choose different DL/UL TCI states before the cell-switch, there is no use case for separate DL/UL TCI states in the cell-switch command. This leads to the natural proposal

Do not support indication of separate UL/DL TCI states in the beam indication.

After the cell-switch, the NW can rely on the enhanced power headroom report to use separate DL/UL TCI states.

### 2.1.2	TCI state activation at the cell-switch command

In RAN1#112bis-e, the following was agreed:

Agreement

For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported:

- Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell,

- Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FFS: signalling details for TCI state activation

FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed

Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

The target of LTM is to reduce the interrupt. Avoiding the TCI state activation delay before the cell-switch is important, and Alt1 describes this.

We note that with Alt1, the UE would have to support at least two activated TCI state: one for the serving cell, and one for the candidate cell. If the UE would only support a single activated TCI state, the communication with the serving cell would be stopped once a TCI state is activated for the candidate, and it would then be impossible for the UE to receive the cell-switch command!

Alt1 requires that the UE supports two active TCI states.

Although the preferred use case for LTM is that the UE activates the TCI state of a candidate before receiving the cell-switch command, it could be so that the NW may have to change the target cell just before the cell-switch command is sent:

The NW may have to change the target just before the cell-switch command is sent.

This may be seen as a fallback mode of operation for LTM. We note that this may also require that the UE initiates a RACH procedure after receiving the cell-switch command.

The Rel-17 TCI framework already has the option to operate without the DCI indication. If the activation command maps the TCI state to only one TCI codepoint, the UE shall apply the indicated TCI state once the indicated mapping for the one single TCI codepoint is applied as described in 38.133. In fact, this is what is supported by default in the Rel-17 TCI framework. Since the cell-switch command only contains a single TCI state index, this would correspond to the Rel-17 mode of operation where a single TCI state is activated.

We thus note that Alt2 would make it possible to operate LTM with only a single activated TCI state, and that it would provide a fallback mode for LTM. Since the behaviour of simultaneous activation and indication is already supported for the Rel-17 TCI framework, we propose

Support also Alt2: the TCI state activation is performed at the reception of the beam indication for the candidate cell.

Finally, we note that it would be unclear what it means if Alt2 is not supported: would that mean that the UE ignores the cell-switch command? This is quite unorthodox: the UE should follow NW signalling, including signalling in MAC CEs.

### 2.1.3	Beam application time

In RAN1#112bis-e, the following was agreed:

Agreement

- Companies are encouraged to study the beam application time for Rel-18 LTM, which may be different from that without serving cell change

  - Definition of the beam application time

  - The exact value(s), condition and UE capability

  - Consider the interaction with the application of the candidate RRC configuration.

During Rel-15, the application time for MAC CEs related to TCI states were defined by RAN1, and rules regarding requirements of time between DCI and PDSCH when TCI is included in DCI. The MAC CE rules defined by RAN1 were subsequently updated by RAN4. In Rel-17, there were also discussions on how to define the beam application time.

In LTM, the beam indication is sent in the same MAC CE that will change the RRC configuration of the UE. The new beam indication and the new RRC configuration must be applied at the same time, since it is unlikely that the new beam indication can be used with the old RRC configuration, and vice versa. Hence, the new beam and the new RRC configuration should be applied at the same time:

The new beam and the new RRC configuration should be applied at the same time.

The application time of the new RRC configuration will be determined by RAN4. In our understanding, the RRC application time should be similar to the application time for a new BWP – at least in some cases. But  this is up to RAN4.

Since the beam application time should coincide with the RRC application, and will specify the RRC application time, RAN1 cannot determine the beam application time. RAN1 could provide some input on the determination, but the final decision would be up to RAN4.

The beam application time for LTM is defined by RAN4.

Already in Rel-15, RAN1 decided that the MAC CE application time was counted from the time when the HARQ ACK of the corresponding PDSCH was sent. This definition should be reused:

The reference for the beam application time for LTM is the time when the HARQ ACK for the corresponding PDSCH is sent.

### 2.1.4	Which channels follow the TCI state in the beam indication?

The Rel-17 TCI framework was designed so that most channels follow the indicated TCI state. However, it is also possible to deviate from this behaviour. This is controlled by RRC: PDCCH is configured to follow the indicated TCI state by a flag in the CORESET configuration, TCI states can be configured in the CSI-RS configuration, and for SRS, a flag in the SRS resource set determines if the indicated TCI should be used or not.

For LTM, it is tempting to state that after the cell switch, the new RRC configuration applies. For most channels, this is fine: channels/signals that are configured to follow the indicated TCI follow the TCI state provided in the cell switch command, and channels/signals that are configured not to follow the indicated TCI state do not. Unfortunately, some channels/signals that are configured not to follow the indicated TCI state rely on a fallback to an SSB identified during a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure. This fallback will not work after an LTM execution. However, this is a small exception. Therefore, we propose

After execution of an LTM cell switch, for CORESETs associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided and followUnifiedTCI-state is not provided, the UE follows the TCI state in the LTM cell switch command until a new TCI state is activated for the CORESETs.

We note that a similar statement must be made related to monitoring occasions for Type0/0A/2-PDCCH CSS sets:

After execution of an LTM cell switch, the UE determines the monitoring occasions for Type 0A/1/2-PDCCH CSS from the TCI state in the LTM cell switch command until a new TCI state is activated for the CORESETs.

## 2.2	LTM measurements and reporting

In RAN1#112bis-e, RAN1 made the following agreements:

Agreement

For the beam selection for SSB based L1-RSRP measurement report,

- Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells

  - FFS: How to select the L cells and M beams per cells is up to UE

- M x L beams are reported in a single report instance

  - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS

    - FFS if UE is allowed to report less than M x L beams

  - The values of M and L are configured to the UE in the reporting configuration

- FFS: The following configurability is introduced in the report configuration

  - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2

  - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency

Agreement

- Periodic and semi-persistent report on PUCCH are also supported for gNB scheduled L1-measurement reporting.

With this agreement, RAN1 decided to follow the UCI reporting paradigm from the Rel-15 beam reporting. We think this is unfortunate, since such UCI reporting has only limited flexibility to deal with varying report content. This would also require additional RRC reconfiguration of the serving cell when candidate cells are configured, as illustrated in Figure 1.

The first agreement above includes a few FFS. For the first FFS, preferably the UE would report the strongest cells, but this is not stated in the specification. We propose to keep this paradigm also for LTM.

How to select the L cells and M beams per cell is up to UE implementation.

We note there is an FFS on special handling of inter-frequency cells: in some cases, it could be so that RSRP measurements from certain frequency bands are always higher, and this may lead to that cells from other frequency bands are excluded from the report, even though these cells could be better targets: the quality could be good enough, and the load in the corresponding cell could be low. Obtaining measurements for such “good-enough-but-not-best” cells can be important in some cases. However, we realize that the network can configure the UE with multiple resource settings to achieve this purpose:

Multiple report settings can be used to ensure that inter-frequency cells are reported.

One of the FFS is related to if the serving cell is always included in the LTM measurement report. The reason to always include measurements from the serving cell is that the LTM measurement report is used to compare the serving cell with the candidates, and a measurement report that only contains measurement for a candidate is not useful. We note that L3 measurement reports always include measurement values related to the serving cell. Based on this, we propose

The UE can be configured to include measurements from one serving cell in every LTM measurement report.

In the next FFS, the issue is if the report should always contain L×M beams, or if the UE under some circumstances could report fewer beams. The motivation is valid: it could be so that the UE is unable to find L cells, and if the report size is fixed, what should the UE report in that case? On the other hand, introducing a varying size of the report will complicate the decoding of the report at the NW, which should be avoided. We note that in legacy, varying report sizes are only used for large reports. Based on this, we propose

One LTM measurement report always contains L cells and M beams per cell according to the configuration.

With Proposal 15, we need to consider the case where the UE does not find all L×M beams. We see two ways the UE could report this:

The UE reports the lowest possible RSRP values, for arbitrarily chosen beam identifier (cell identifier and SSB index).

The UE repeats the measurements from one of the cells it did find.

Either option would work.

The next level of details of the measurement report should also be settled. The first issue is simply to agree on L and M. In legacy beam reporting, up to four beams can be included in one report. For simplicity, we propose to keep that value, even though fewer beams would probably suffice. For the number of reported cells, it is preferable to match the number of candidates. RAN2 has so far not agreed on a maximum number of candidates, but since the functionality is similar to conditional handover (CHO), it is likely that we will end up with a similar number of candidates as for CHO, which is 8. Based on this, we propose

The LTM measurement report supports up to M=4 and L=8.

Note that Proposal 16 describes how we will design the report format: the numbers a UE supports may be lower.

In legacy L1-RSRP reporting, differential encoding is performed if more than one beam is reported to reduce the report size. It would make sense to rely on differential reporting to some extent also here.

One difference compared to the legacy reporting is that the LTM report has a certain structure: the report contains L cells, and for each cell, M beams are included. There is no such structure in the legacy beam report, not even for ICBM: the reported beams are reported individually.

It would be preferable to use the cell/beam structure in the reporting, so that all the measurements for one cell are adjacent in the measurement report. This would simplify interpretation of the report:

The beam measurements are grouped per cell.

The differential encoding specified in Rel-15 was defined to handle measurement values within one cell. Since we are now reporting multiple cells, the differential reporting range may be insufficient: the RSRP for different cells can be quite different. To solve this, we propose to perform the differential encoding per cell: for each cell, we include one absolute RSRP value, and the RSRP values for the remaining beams for that cell are differentially encoded relative to the largest RSRP value of that cell:

Differential encoding of the RSRP values is performed per cell.

So far, RAN1 has not agreed on the identifiers in the report. It is natural that the SSB index is included to identify the beams of a cell. However, to identify the cell, something more is needed. In RAN2#120, RAN2 made the following agreement:

Permanent Identities such as PCI will not be used in L1 L2 signalling, instead L1 L2 signalling will use temporary identities configured by RRC.

Hence, the cell identity in the measurement report cannot be PCI. Since the intention of the measurement reporting is to collect information about candidates, it would make sense that the value range of cell identity in the LTM measurement report matches the number of candidates:

The identifier in the LTM measurement report can be mapped to a candidate configuration.

One possibility is that the identifier in the LTM measurement report is the candidate configuration index. This would make the mapping in the gNB trivial.

One consequence of the RAN1 agreement to rely on UCI over PUSCH for the LTM reporting is that the protection of the LTM measurement needs extra attention. In contrast to reporting over MAC CE, there is no retransmission mechanism available for UCI: an incorrect report can only be dropped. Even worse: for some UCI payloads, there is not even a CRC attached to the UCI. This is highly undesirable for an LTM measurement report: it must be possible to detect that an LTM measurement report was incorrectly decoded. The simplest way to ensure this is that the payload is always larger than 11 bits:

All LTM reports are zero-padded to ensure that the payload is always 12 bits or larger.

With the lower flexibility of the UCI reporting, a higher degree of pre-configuration of measurement targets becomes necessary. In RAN1#112, RAN1 made the following agreement:

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

The agreement leaves many of the design details to RAN2, and hopefully, RAN2 can design the configuration based on these details. Alternatively, RAN1 could clarify a few details:

If the UE is provided a logical ID to identify a measurement target, the UE would have to map that to a PCI, and the mapping between the logical ID and the PCI must also be provided to the UE. In other words, the UE must be provided a PCI identifying the measurement target one way or another. To clarify the agreement, the logical ID could be removed.

As we are designing inter-cell measurements, it would seem appropriate to rely on the same configuration parameters as for L3 measurements. This would mean that we use the SMTC window, and potentially also ssb-ToMeasure and deriveSSB-IndexFromCell. Periodicity and SSB position in burst seem less appropriate.

In RAN1#111, the following agreement was made

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

Hence, RAN1 has so far only agreed reporting of L1-RSRP for SSB, for intra-frequency and inter-frequency.

If additional measurement possibilities are introduced, the configuration overhead grows. For example, the CSI-RS configuration is quite flexible, resulting in a much larger overhead. Based on this, our preference is not to support CSI-RS as a measurement target for LTM In Rel-18:

Do not support CSI-RS as a measurement target for LTM.

Event-driven reporting was discussed during RAN1#111. It is often claimed that event-driven measurements have lower latency than network-triggered measurements. This is incorrect: event-driven reporting does not reduce latency, since the latency is determined by the UL transmission time, which is typically larger for event-driven reporting than for network-initiated reporting:

The latency of event-driven reporting is typically larger than the latency of network-initiated reporting.

Instead, the benefits of event-driven reporting lie in lower reporting overhead. The report is only sent in the UL when certain conditions are fulfilled, i.e., when the event is triggered:

Provided that the event is properly designed, the reporting overhead of event-driven reporting can be lower than network-initiated reporting.

Reducing reporting overhead can be important: if only periodic reporting is used, it is likely that the network would need one report from each UE every 40ms, and it is not clear that the network can handle that load.

Event-driven reporting would require that RAN1 defines events, which RAN1 has never done (apart from the RLM out-of-sync indications). Definitions of events would be further complicated by the fact that RAN1 specifications are state-less: RLM indications are counted in RRC, and beam failure indications in MAC.

The events designed for L3 mobility is quite general, and has required quite some specification effort in RAN2, already for 3G. The solution was then inherited in 4G, and somewhat extended in 5G. Here we would like to highlight one issue that makes the RAN2 event handling different from the events defined in RAN1: the necessity of parallel events.

The mere introduction of events is thus quite complicated. Based on the complexity of defining events, and the need to also specify a MAC CE reporting format, we propose

Event-driven LTM reporting is not specified in Rel-18.

As remarked above, there are benefits of event-driven reporting, and it would be relevant to consider such improvements for Rel-19.

## 2.3	Operations on candidate cells before the cell switch

RAN1 made the following agreement in RAN1#112bis-e:

Agreement

For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported:

- Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell,

- Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FFS: signalling details for TCI state activation

FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed

Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

We note that the agreement is somewhat imprecise: the TCI state activation is an operation that is performed in the UE, and it cannot be “received”.

After activating the TCI state which serves as QCL source for the PDCCH/PDSCH DMRS, the UE is prepared to receive PDCCH/PDSCH. This is described in 38.133:

Excerpt from 38.133, section 8.10.3:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

If the TCI state is in the list of activated TCI states, TOk = 0, indicating that there is no interruption due to the change of TCI state.

Two things are important related to the TCI state activation description in 38.133:

It is critical to ensure that the TCI state is activated before the cell switch: otherwise, there will be an interrupt. The length of the interrupt depends on the SSB occurrences.

Irrespective of which reference signals are in the TCI state, the activation delay depends on the occurrences of the SSB. In particular, the delay does not depend on the TRS periodicity, meaning that a TCI state containing TRS can be activated even before the UE has received that TRS.

In section 2.1, we discussed what information the UE needs to activate a TCI state: the required information is listed in Proposal 3. Not surprisingly, the same information is required also for a TCI activation before the cell-switch:

The information in Proposal 3 is needed also for TCI state activation before the cell-switch command.

The information in Proposal 3 (DL carrier frequency, SSB SCS, PCI, SSB index) would be provided to the UE using RRC signalling. The question is what would trigger the UE to actually perform the TCI state activation, for a TCI state corresponding to a certain TCI state index.

We note that the UE is reporting SSBs to the NW, as part of the reporting. Since the UE is reporting an SSB, it knows the DL carrier frequency, SSB SCS, and the PCI. One option would be that the UE then activates the TCI state based on what the UE reported. For example, the UE could activate the TCI state corresponding to the first SSB in the latest report. This would usually work, since the most common target will be the first SSB the UE reports.

However, the NW may want to select another target than the first SSB in the report. Relying on a UE selection would not work in this case. The network would want to have more control over which TCI state to activate.

As part of the LTM specification, RAN1 is enhancing the TA acquisition. The enhancement involves triggering a PRACH transmission towards a certain candidate cell, using a modified PDCCH order. The PDCCH order will contain a reference to an SSB, which the UE would use to determine the timing for the PRACH transmission. To find the SSB used to determine the DL timing, the UE would have to know the DL carrier frequency, SSB SCS, and the PCI before the PDCCH order is transmitted: these would have to be part of the configuration:

As part of the configuration for the RACH procedure used for TA acquisition, the UE must be configured with the DL carrier frequency, SSB SCS, and the PCI for the target cell.

The network should want to activate the TCI state corresponding to the SSB in the PDCCH order: there is no reason to activate any other TCI state. Hence, we propose

At the reception of a PDCCH order triggering a PRACH transmission with a reference to a certain SSB, the UE activates the TCI state corresponding to that SSB.

The UE uses the mapping between TCI state index and the SSB that is anyway provided to the UE. As already remarked, the UE needs the DL carrier frequency, SSB SCS, and the PCI to transmit PRACH towards a certain candidate.

In a sense, the UL pre-synchronization and (DL) TCI state activation are two sides of the same coin: both operations aim to reduce the delay for the physical layer preparation procedures in the target.

Alternatively, the TCI state could be activated using separate signalling, e.g., using MAC CE. This would simply complicate the LTM procedure.

## 2.4	RRC parameters

There are two main parts of this agenda item: the measurements and the beam indication. Regarding the measurements, RAN1 has already been discussing the resource configuration, and LSs have been exchanged with RAN2. RAN1 has not discussed the reporting configuration though, other than it should be reported over UCI. A proposal is provided in Table 2. Regarding report quantity, we assume that the report quantity from Rel-15 can be reused: ssb-Index-RSRP.

Regarding beam indication, there has not been much discussion in RAN1. However, there will be a need to configure the UE so that a TCI state index can be translated into an ARFCN, PCI and SSB index – potentially without the UE reading the serving cell configuration of the candidate.

Table 2: RRC parameters for LTM.

| RAN2 parent IE | Parameter name in the spec | Description |
| --- | --- | --- |
| CSI-ReportConfig | ltmReporting | Used to configure the LTM reporting. Contains fields - Number of cells (L) - Number of beams per cell (M) - Serving cell measurement always included |
| CSI-ReportConfig | ltmResourceConfigId | Pointer to the LTM measurement resources. Configured outside serving cell configurations |
|  | LTM-MeasurementResourceConfig | Definitions of measurement targets. Contains: - ARFCN - PCI - SMTC window |
|  | LTM-BeamIndicationConfiguration | Signalling of RSs and QCL properties to be used in beam indication and TCI state activation. Contains: - ARFCN - PCI - TCI state index - SSB index |

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 should not assume that any information that is available at the UE is also available in the serving DU.

Observation 2	The TCI state alone does not include sufficient information for the UE to determine which reference signal to aid the reception of PDCCH/PDSCH.

Observation 3	The UE can use the target candidate configuration to determine the TCI state corresponding to the beam indication, and to determine the reference signals used to assist the reception of PDCCH/PDSCH.

Observation 4	After the RRC reconfiguration trigger by the cell-switch command, the UE assumes the TCI state index in the beam indication refers to the configuration of the SpCell.

Observation 5	Using the TCI state corresponding to the lowest pathloss is reasonable and would result in the same UL and DL TCI state.

Observation 6	In cases when the UE must reduce its transmit power due to MPE regulations, separate DL/UL TCI states may be useful.

Observation 7	There is no method to provide the NW with information to choose different DL and TCI states before the cell-switch.

Observation 8	Alt1 requires that the UE supports two active TCI states.

Observation 9	The NW may have to change the target just before the cell-switch command is sent.

Observation 10	The new beam and the new RRC configuration should be applied at the same time.

Observation 11	Multiple report settings can be used to ensure that inter-frequency cells are reported.

Observation 12	The latency of event-driven reporting is typically larger than the latency of network-initiated reporting.

Observation 13	Provided that the event is properly designed, the reporting overhead of event-driven reporting can be lower than network-initiated reporting.

Observation 14	The information in Proposal 3 is needed also for TCI state activation before the cell-switch command.

Observation 15	As part of the configuration for the RACH procedure used for TA acquisition, the UE must be configured with the DL carrier frequency, SSB SCS, and the PCI for the target cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not enhance LTM to support scenario 1 or 3.

Proposal 2	The beam indication, i.e., the TCI state index and BWP index, refers to the SpCell of the indicated candidate.

Proposal 3	The UE is provided with an explicit mapping between a TCI state index and {DL carrier frequency, SSB SCS, PCI, SSB index}.

Proposal 4	How to provide the mapping between a TCI state index and {DL carrier frequency, SSB SCS, PCI, SSB index} is up to RAN2.

Proposal 5	Triggering of aperiodic TRS, SRS or CSI acquisition is not included in the cell-switch command.

Proposal 6	The beam indication may contain one TCI state index, indicating a joint TCI state.

Proposal 7	Do not support indication of separate UL/DL TCI states in the beam indication.

Proposal 8	Support also Alt2: the TCI state activation is performed at the reception of the beam indication for the candidate cell.

Proposal 9	The beam application time for LTM is defined by RAN4.

Proposal 10	The reference for the beam application time for LTM is the time when the HARQ ACK for the corresponding PDSCH is sent.

Proposal 11	After execution of an LTM cell switch, for CORESETs associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided and followUnifiedTCI-state is not provided, the UE follows the TCI state in the LTM cell switch command until a new TCI state is activated for the CORESETs.

Proposal 12	After execution of an LTM cell switch, the UE determines the monitoring occasions for Type 0A/1/2-PDCCH CSS from the TCI state in the LTM cell switch command until a new TCI state is activated for the CORESETs.

Proposal 13	How to select the L cells and M beams per cell is up to UE implementation.

Proposal 14	The UE can be configured to include measurements from one serving cell in every LTM measurement report.

Proposal 15	One LTM measurement report always contains L cells and M beams per cell according to the configuration.

Proposal 16	The LTM measurement report supports up to M=4 and L=8.

Proposal 17	The beam measurements are grouped per cell.

Proposal 18	Differential encoding of the RSRP values is performed per cell.

Proposal 19	The identifier in the LTM measurement report can be mapped to a candidate configuration.

Proposal 20	All LTM reports are zero-padded to ensure that the payload is always 12 bits or larger.

Proposal 21	Do not support CSI-RS as a measurement target for LTM.

Proposal 22	Event-driven LTM reporting is not specified in Rel-18.

Proposal 23	At the reception of a PDCCH order triggering a PRACH transmission with a reference to a certain SSB, the UE activates the TCI state corresponding to that SSB.

# References

RP-213565, New WID on Further NR mobility enhancements, MediaTek, RAN#94e, December 2021

R1-2302413, L1 enhancements to inter-cell beam management, Ericsson, RAN1#112bis-e, April 2023

R1-2208331, LS on L1/L2-based inter-cell mobility, RAN2, RAN2#119-e, August 2022

R1-2302414, Timing advance management to reduce latency, Ericsson, RAN1#112bis-e, April 2023

R2-2211061, LS on RAN2 agreements about L1/L2-triggered mobility (LTM), RAN2, RAN2#119bis-e, October 2022

R1-1716375, Details on TRS design, Ericsson, 3GPP TSG-RAN WG1 Meeting #NR3, September 2017

TS 38.215, NR; Physical layer measurements, RAN#97-e, v 17.2.0, September 2022

TS 38.133, NR; Requirements for support of radio resource management, RAN#96, V 17.6.0, June 2022

R1-2300007, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, RAN3, RAN3#118, August 2022

R1-2300014, Reply LS on L1 measurement and configurations for LTM, RAN2, RAN2#120, August 2022