3GPP TSG RAN WG1 #113	R1-2304797

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.11.1

Source:	Futurewei

Title:	Evaluation of LP-WUS and Performance Results

Document for:	Discussion/Decision

# Introduction

A new study item on low-power wake-up signal and receiver for NR was approved in RAN#94e [1] and revised in RAN#97e [2]. The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, and the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In prior RAN1 meetings, a set of agreements were reached and are available in Appendix C for reference. This contribution includes a discussion on the power consumption model, performance metrics, and evaluation methodology of a low-power (LP) Wake-Up Radio (WUR) capable/equipped UE. Evaluation of power consumption and latency of the LP-WUS is presented under the assumptions of LP-WUR's full coverage support and group addressed LP-WUS. The LP-WUS performance is compared to DRX/eDRX with PEI as baseline power saving schemes while considering the impact of false alarms.

# Evaluation Methodology

Duty-cycled operations in the form of Discontinuous Reception (DRX) and extended Discontinuous Reception (eDRX) are defined for power consumption reduction in NR RRC_IDLE and RRC_INACTIVE states through the reduction of the number of Paging Occasions (POs) monitored by the UE. Further power consumption reduction can be achieved through Paging Early Indication (PEI) in NR RRC_IDLE and RRC_INACTIVE states, which is still subject to the duty-cycled operations. Similar power saving techniques are defined for NR RRC_CONNECTED state in the form of connected mode DRX (C-DRX) and Wake-Up Signal (WUS). Both PEI and WUS can be received by UEs as DCIs over the PDCCH.

The DRX, eDRX, and C-DRX can provide higher power saving gain by increasing the duty cycle duration at the expense of higher latency to be expected by the UE. PEI and WUS can provide more power saving gain without an impact on latency, but the gain is limited by the power consumption required to decode a DCI over PDCCH. A new WUS that can be received with significantly lower power consumption than existing PEI/WUS designs may enable new trade-off regions of Latency versus Power but will require a dedicated Low-Power Wake-Up Radio/Receiver (LP-WUR) with a simple architecture [3].

## DRX and eDRX

For a UE using DRX in RRC_IDLE or RRC_INACTIVE states, it monitors one PEI occasion (PEI-O) and/or one PO per DRX cycle as shown in Figure 1, based on PEI configuration, where a PEI-O/PO consists of a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots [4]. The UE initiates RRC Connection Establishment or RRC Connection Resume procedures upon reception of a CN initiated or RAN initiated paging, respectively. If PEI is configured, the UE monitors an associated PO in a DRX cycle only if the PEI is detected and the UE’s corresponding subgroup is indicated in the PEI.

Figure 1: Processing Timeline: DRX Configuration.

For a UE using eDRX in RRC_IDLE or RRC_INACTIVE states, it monitors one PEI-O and/or one PO per eDRX cycle, based on PEI configuration, as shown in Figure 1 if the configured eDRX cycle is no longer than 1024 radio frames [4]. Otherwise, the UE monitors one PEI-O and/or one PO per eDRX cycle, based on PEI configuration, according to a configured DRX cycle during a UE-specific and periodic Paging Time Window (PTW), where the PTW period is determined by the eDRX cycle and the length is configured by upper layers, as shown in Figure 2. The UE initiates RRC Connection Establishment or RRC Connection Resume procedures upon reception of a CN initiated or RAN initiated paging, respectively. If PEI is configured, the UE monitors an associated PO in a DRX/eDRX cycle only if the PEI is detected and the UE’s corresponding subgroup is indicated in the PEI.

Figure 2: Processing Timeline: eDRX (TeDRX>1024 frame) Configuration.

## LP-WUS

In the evaluation, the performance of a UE considering the LP-WUS is compared to the performance of DRX/eDRX with/without PEI as baseline power saving schemes. For a UE using a LP-WUR in RRC_IDLE or RRC_INACTIVE states, it may either continuously or periodically monitor for a LP-WUS, as shown in Figure 3 for the continuous monitoring case when LP-WUS carries a UE ID. Other UE behavior details are discussed herein, performance metrics and models including impact of false alarms are discussed in Section 2.2.1, and additional receiver processing details are discussed in Section 2.3.

Throughout the rest of the document, the following terminology, as agreed in RAN1#110bis-e, is used

- Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up

- LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

Further, the following terminology for LP-WUS monitoring options can be used interchangeably

- Option 1: “Continuous” and “Always-on” monitoring

- Option 2: “Discontinuous”, “Periodic”, and “Duty-Cycled” monitoring

There are three different options for the behavior of a UE (that are applicable to both LP-WUS monitoring “Option 1” and “Option 2”) in response to the reception of a LP-WUS depending on the content of the LP-WUS and network configuration. The three UE behavior options are

- UE_Behavior (1): LP-WUS carries a unique UE ID and MR is not required to monitor POs.

- UE_Behavior (2): LP-WUS carries a UE group ID and MR is required to monitor legacy POs/PFs.

- UE_Behavior (3): LP-WUS carries a UE group ID and MR is required to monitor newly defined POs/PFs.

UE_Behavior (1), as shown in Figure 3, may result in the best experienced latency under LP-WUS power saving scheme, especially when continuous monitoring mode (Option 1) is used. This is due to the fact that the UE may wake-up the main radio to directly initiate RRC Connection Establishment or RRC Connection Resume procedures upon reception of a CN initiated or RAN initiated paging, respectively, as indicated by the LP-WUS. This UE behavior also eliminates the need to align the LR and MR duty cycles when periodic LP-WUS monitoring is considered. However, this comes at the cost of a large LP-WUS payload size and subsequently a potentially high resource overhead requirement.

Figure 3: An Example Protocol Flow/Timeline based on LP-WUS Configuration with UE Addressing.

UE_Behavior (2), as shown in Figure 4, will result in a LP-WUS latency performance that is limited by the legacy DRX cycle, i.e., 0.32, 0.64, 1.28, 2.56 seconds, and will always underperform the DRX power saving scheme in terms of latency. This is due to the fact that the UE will still have to monitor POs using the MR upon wake up in response to the detection of a LP-WUS. However, power saving gain is still expected compared to DRX, i.e., depending on the UE group size, and managed LP-WUS resource overhead is possible due to use of UE group IDs instead of UE unique IDs. Compared to UE_Behavior (1) and based on the UE group size, there will be a power consumption penalty that may limit any power saving gain considering the expected high transition energy from ‘Ultra-deep sleep’ power state. Further, considering ‘always-on’ monitoring of the LP-WUS under UE_Behavior (2) when the LP-WUS is carrying UE group ID(s) may not result in any latency reduction benefit compared to DRX power saving scheme since the MR will still have to monitor POs according to any of the legacy DRX cycles. However, ‘always-on’ monitoring mode may alleviate the need for the LP-WUR to periodically synchronize with the transmitting entities.

Figure 4: An Example Protocol Flow/Timeline based on LP-WUS Configuration with UE Group Addressing.

UE_Behavior (3) may correspond to the definition of shorter RRC IDLE/INACTIVE state DRX cycles, i.e., <320ms, which may result in a better LP-WUS latency performance compared to UE_Behavior (2) without any impact on power consumption due to the use of LP-WUR and at a managed LP-WUS resource overhead due to the use of UE group IDs.

Both UE_Behavior (2) and (3) may also apply for the case when the LP-WUS carries a unique UE ID but the MR is still required to monitor POs. However, for a LP-WUS with a low false alarm rate (FAR), it might be unreasonable to mandate MR monitoring of POs after detection of LP-WUS carrying a unique UE ID. That is because PO monitoring by the MR will add to the power consumption without providing any additional information to the UE.

Observation 1: For group-addressed LP-WUS, ‘always-on’ monitoring mode may not result in any latency reduction benefits due to the necessity of MR’s monitoring of legacy POs after LP-WUS detection but may alleviate the requirement for frequent LP-WUR synchronization.

### Performance Metrics

In prior RAN1 meetings, agreements (see Appendix C: RAN1 Agreements) on the performance metrics to be considered for the evaluation of the LP-WUS are reached.

Power consumption is one of the metrics that are going to be used in the evaluation of the LP-WUR. During the evaluation, it can be captured as the average relative power consumption per transaction cycle, where the transaction cycle is the average time between two consecutive network events (traffic arrivals). The power consumption of the LP-WUR should be considerably lower than the main radio, e.g., around 100μW or less [3], to achieve a target battery lifetime close to or exceed that may be achieved by eDRX, e.g., >10 years for small size and low frequency messages [5], with considerably better UE reachability/latency.

Latency is another metric which can be captured/measured using one or more of the following definitions depending on the UE behavior options discussed in Section 2.2

- Latency_Definition (1): the average time between the arrival of a network event, i.e., data, and completion of MR synchronization after the detection of a corresponding LP-WUS.

- Latency_Definition (2): the average time between the arrival of a network event, i.e., data, and the reception of a corresponding paging message.

- Latency_Definition (3): the average time between the arrival of a network event, i.e., data, and the transmission of a corresponding PRACH.

The first latency definition applies to UE_Behavior (1). The second latency definition applies to UE_Behavior (2) and UE_Behavior (3) only when the UE is required to monitor a PO after LP-WUS detection. On the other hand, the third latency definition can be applied to any of the UE behaviors (1-3). For any of the latency definitions, system information update, if needed, may be accounted for, where system information update may be considered as part of the MR synchronization for the first definition. However, the contribution of system information update to latency may be ignored since it is expected to only occur infrequently as part of a paging procedure. The MR synchronization is described in Figure 8 of Section 2.3.

For any given DRX configuration, the latency resulting from the consideration of the LP-WUR should be comparable to or better than the DRX scheme at a reasonably small fraction of the power consumption. Other metrics that can be considered for the evaluation of the LP-WUR and associated LP-WUS include network coverage and network resource overhead.

Proposal 1: For a LP-WUS carrying a UE unique ID, the latency is defined as the average time between the arrival of data at gNB and the UE’s completion of MR synchronization upon detection of a corresponding LP-WUS.

Proposal 2: Consider ignoring the impact of system information update on IDLE/INACTIVE state latency due to its infrequent occurrence as part of paging procedure.

#### Power Consumption

To evaluate the power consumption of the DRX/eDRX as baseline schemes and the Low-Power Wake-Up Signal (LP-WUS) as a new proposed scheme, the power consumption model specified in Table 1-Table 3 may be considered. The values in Table 1 and Table 2 (except the ‘Ultra-deep sleep’ power/sleep state) are obtained/derived (according to the latest agreement in RAN1#110-bis-e, see Appendix C: RAN1 Agreements) based on [6] (for eMBB and other cases than IoT and wearables) and [8] (for IoT and wearables) where the scaling down to 20 MHz system bandwidth (BW) in Table 1 assumes a scaling factor of 0.4 and a BWP transition power of 50 units.

The values considered for the ‘Ultra-deep sleep’ power/sleep state and the power consumption values in Table 3 are based on the latest agreements in RAN1#111 and RAN1#112 (see Appendix C: RAN1 Agreements). The overall power consumption of the UE should be evaluated as the combination of the power consumption associated with the main radio and the LP-WUR.

Table 1: UE Power Consumption Model for Main Radio.

| Power State | Characteristics | Relative Power (eMBB) [20 MHz BW] | Relative Power (RedCap) [20 MHz BW] |
| --- | --- | --- | --- |
| Ultra-deep sleep | Power consumption of the main radio when LP-WUR is used for LP-WUS monitoring | [0.015] | [0.015] |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) | 0.8 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 | 18 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 | 31 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 50 | 50 |
| SSB or CSI-RS processing | SSB can be used for fine time-frequency sync. And RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. | 50 | 50 |
| PDCCH + PDSCH | PDCCH + PDSCH. | 120 | 120 |
| Intra-frequency RRM measurement* | Power consumption of the main radio during intra-frequency neighbor cell measurement | 60†/80‡ | 60†/80‡ |
| Inter-frequency RRM measurement* | Power consumption of the main radio during inter-frequency neighbor cell measurement | 60†/150‡ | 60†/150‡ |
| * Synchronous SSB transmissions from cells and number of cells for intra-frequency measurement of N=8 are assumed. † Relative power accounting for measurements only ‡ Relative power accounting for both measurement and neighbor cell search Note1: 2 RX chains are assumed for the relative power values in this table, a scaling factor of ‘0.7’ can be used for non-sleep power states in case of 1 RX chain. Note2: a cell search rate of (1/4) can be considered to determine the effective (combined) relative power for intra/inter-frequency measurement and search. |  |  |  |

Table 2: UE Power Consumption for Main Radio during the State Transition.

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Ultra-deep sleep | Alt 1: 15000** Alt 2: [40000]** | Alt 1: 400 ms† Alt 2: 800 ms† |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Additional transition energy for ‘Ultra-deep sleep’ power state accounts for both ramp-up and ramp-down, it does not account for energy required for (re-)synchronization. † Total transition time for ‘Ultra-deep sleep’ power state accounts for ramp-up time only, assumptions for (re-) synchronization time are discussed in Section 2.3 |  |  |

Table 3: UE Power Consumption Model for LP-WUR.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Off | WUR power consumption when turned off | POFF ∈ (0.001)* |
| On | WUR power consumption when turned on | PON ∈ {0.01/0.05/0.1/0.5/1/2/4}* |
| * Values are dependent on the studied architectures in AI 9.11.2 Note: transition energy is determined as [TLR, ramp-up *(PON-POFF)/2] where TLR, ramp-up=10 ms is the WUR ramp-up time in ms. |  |  |

#### Latency

Latency of any of the baseline schemes or the LP-WUS scheme may be evaluated using system level simulations or analytically depending on the use case and corresponding traffic model. For example, the Poisson arrival process may be considered for the IoT/wearables use case where the number of network events, e.g., paging arrivals, within a simulated/evaluation window follow a Poisson distribution and inter-arrival times between the events follow an exponential distribution. To simplify the evaluation for the Poisson arrival process, analytical expressions can be derived for latency (according to Latency_Definition (2) in Section 2.2.1) where a network event, e.g., data arrival, is assumed to arrive uniformly in any transaction cycle, i.e., mean inter-arrival time. The latency of the DRX LDRX and the eDRX LeDRX schemes can then be derived, respectively, as [see appendix A]

LDRX=A2 ,

LeDRX=PTWBA2+1-PTWBB -PTW2 ,

whereas the latency of the LP-WUR LWUR can be derived, assuming that the main radio is still restricted to a DRX cycle configuration, as [see appendix B]

LWUR=3A2-A-D2A,

where A, B, D, and PTW represent the DRX cycle duration, the eDRX cycle duration, the time required to process the LP-WUS and wake-up the main radio (including main radio synchronization), and the Paging Time Window duration, respectively. Alternatively, the latency of the LP-WUR (according to Latency_Definition (3) in Section 2.2.1) assuming that the main radio may wake-up on demand without being restricted to any DRX cycle configuration can be expressed as

LWUR=D+C2,

where C represents the RACH occasions (ROs) periodicity which can be as frequent as every frame, i.e., 10ms, according to [3GPP TS 38.211 Section 6.3.3.2]. The second term may be ignored when D≫C and the latency may simply be considered as LWUR=D which is the expected LP-WUS latency according to Latency_Definition (1) when the LP-WUS carries a UE ID.

The impact of Latency_Definition (3) on the latency evaluation (e.g., based on system level simulation or derived analytic formulas) of DRX and eDRX power saving schemes depends on the time difference between POs and ROs. For simplicity, we can assume that the time difference is fixed and corresponds to one half the RO periodicity.

#### Coverage and Resource Overhead

The wide range of relative power unit values for LP-WUR ‘On’ state in Table 3 will correspond to different LP-WUR architectures with high variability in sensitivity and subsequently coverage. Coverage mismatch between LP-WUS and PDCCH will have an impact on power saving gain expected from the LP-WUR and should be taken into account during the LP-WUS evaluation and comparison between the different LP-WUR architectures.

Observation 2: Mapping between relative power unit values and LP-WUR coverage ratio/probability, and coverage mismatch, i.e., between LP-WUS and PDCCH, handling needs to be defined for proper evaluation of power saving gains/losses.

If LP-WUR’s full coverage is decided to be a target design, the power saving gain from the LP-WUR is expected to be low with potential restriction on the LP-WUR’s ‘always-on’ monitoring mode due to the expected high LP-WUR’s power consumption during the ‘on’ power state, i.e., considering a single LP-WUR’s receiver (RX) mode of operation. On the other hand, if LP-WUR’s full coverage is decided not to be a design target, the power saving gain during the LP-WUR’s operation is expected to be high even with the ‘always-on’ monitoring mode. However, when the LP-WUR is out-of-coverage, the UE might have to switch to the MR and forgo any power saving gain. The overall power saving gain, in that case, will depend on the UE’s mobility pattern with worst case scenario being for UEs that are almost stationary and reside at/close to the cell edge.

Alternatively, the UE might balance between power saving gain and coverage by considering two or more RX modes of operation for the LP-WUR where, e.g., an ultra-low power RX mode is used in good coverage scenarios to achieve the maximum power saving gain and a low-power RX mode is used in bad coverage scenarios to achieve a reasonable power saving gain without sacrificing coverage. The two or more RX modes of operation may require the same or different signal design. If the same signal design is considered for the LP-WUR’s RX modes of operation, switching between the RX modes of operation can be transparent to the network. However, if different signal design is considered, switching cannot be transparent to the network and may need to be reported. Otherwise, there will be impact on both latency and resource overhead.

The different schemes for handling LP-WUR coverage can then be summarized as follows

- Coverage_Scheme (1): LP-WUR full coverage without RX mode of operation switching.

- Coverage_Scheme (2): LP-WUR limited coverage with switching to MR during out-of-coverage.

- Coverage_Scheme (3): LP-WUR full coverage with RX mode (e.g., two or more) switching.

  - Coverage_Scheme (3-A): RX modes of operation consider the same signal design.

  - Coverage_Scheme (3-B): RX modes of operation consider different signal designs.

Similarly, network resource overhead required to support LP-WUS/LP-WUR may depend on the LP-WUR architecture and its in-band interference rejection/selectivity capability, e.g., ability of the LP-WUR to reliably detect a LP-WUS that is multiplexed in-band with other LP-WUSs or NR signals. Network resource overhead will also depend on the LP-WUR’s supported data rate, LP-WUS design/structure, e.g., number of information bits and coding, and LP-WUS functions, e.g., group-based paging. The LP-WUS functions will determine the frequency of transmissions. For example, UE-based paging might imply more frequent transmissions than group-based paging (depending on the transmission criteria). Further, LP-WUS transmission in support of measurements maybe periodic, e.g., periodic beacon transmissions.

Proposal 3: Coverage mismatch between LP-WUS and PDCCH should be considered when power savings gain/network resource overhead are evaluated and the following 3 options should be adopted

- LP-WUR full coverage without RX mode of operation switching.

- LP-WUR limited coverage with switching to MR during out-of-coverage.

- LP-WUR full coverage with RX mode (e.g., two or more) switching.

Observation 3: LP-WUS design options/functions and mapping to time/frequency resource requirements need to be defined for proper evaluation of the network resource overhead in support of LP-WUS/WUR.

#### LP-WUR False Alarms

In RAN1#110bis-e and RAN1#111, the following agreements on the evaluation of the impact of false alarm rate (FAR) on the performance of LP-WUS are achieved.

Agreement For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency. In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included.. UPT; The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. - Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI - Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact - Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement For the performance evaluations of LP-WUS candidate designs, it is assumed that - The miss-detection rate (MDR) of LP-WUS [1%], - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying reported by companies - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead - FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details.

For proper evaluation of the impact of FAR, identification of the potential sources of false alarms is required. In general, there are two sources of potential false alarms (false wake-up indications) to the LP-WUR, which can be classified into:

- Grouping: under this category/class, the LP-WUR of a UE is falsely alarmed for wake-up due to the correct detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group.

- Noise/Interference: under this category/class, the LP-WUR of a UE is falsely alarmed for wake-up due to the false detection of a LP-WUS due to presence of noise and/or interference where interference may be another LP-WUS intended for other UEs/UE groups or any other signal.

Proper design of the LP-WUS can result in a much lower contribution to false alarms from the second source (Noise/Interference) than the first source (Grouping). Further, it can be noted that the first source (Grouping) may not have a significant impact on the latency experienced by a UE but will have an impact on the UE’s power consumption and expected power saving gain due to employment of LP-WUR. During the evaluation of power saving gain, the first source of false alarms can be captured as part of the traffic model. On the other hand, the second source (Noise/Interference) of false alarms may have an impact on both UE’s experienced latency and power saving gain depending on the UE’s configuration for LP-WUS monitoring and response to LP-WUS detection, i.e., either true detection or false detection. Additionally, similar to impact of miss-detection rate (MDR), the second source of false alarms may result in additional resource overhead associated with the need for LP-WUS re-transmissions.

An example UE behavior is shown in Figure 5 where the LP-WUR is configured with a duty-cycle shorter than the time required to wake-up the MR and the LP-WUR is not required to continue monitoring LP-WUS after sending a wake-up indication to the MR. The figure shows that for this example, the UE will miss the detection of a true LP-WUS due to the (Noise/Interference) false alarm leading to potential increase in latency and resource overhead.

Figure 5: Main Radio and LP-WUR Processing Timeline in Presence of False Alarms due to Noise/Interference.

Observation 4: A UE may miss the detection of a true LP-WUS due to a  false alarm, generated by Noise/Interference, leading to a potential increase in latency and network resource overhead for LP-WURs configured with ‘always-on’ or short ‘duty-cycled’ monitoring.

As shown in Figure 5, due to the dependency between LP-WUR and MR operations, the number of false alarms in any duration T cannot simply be determined as the product of FAR and number of LP-WUS monitoring occasions in that duration T. In RAN1#112, two alternatives for the definition of the FAR target are agreed for further discussion as follows

Agreement - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials

For link level simulation and LP-WUS design, the first alternative (Alt 1) can be more relevant/appropriate. However, for power saving gain evaluation, system level simulation may be required if ‘always-on’ or short ‘duty-cycled’ monitoring modes are considered for the LP-WUR. This would be due to that potential dependency between monitoring occasions, e.g., WUS decoding attempts, when the time between two such occasions is shorter than the time required for the main radio to wake-up, determine a false alarm, and go back to sleep. Therefore, the second alternative (Alt 2) can be used to simplify the evaluation of LP-WUS power saving gain and the DRX cycle of the MR, i.e., when the MR is required to monitor a PO after being woken up by a LP-WUS, can be considered as the reference time duration. Further, to avoid any dependency between consecutive reference time durations, the MR DRX cycle can be limited to values greater than the MR’s ramp-up and synchronization time. The second alternative can also be used when relatively long ‘duty-cycled’ monitoring mode is considered for the LP-WUR and the LP-WUR duty cycle can be considered as the reference time duration.

It is important to note that the LP-WUS design will be different for ‘duty-cycled’ and ‘always-on’ monitoring modes when compared/evaluated at the same FAR when considering FAR Alt 2. This is due to the fact that the number of LP-WUS decoding attempts/trials for ‘always-on’ monitoring is expected to be larger than that for ‘duty-cycled’ monitoring at the same reference time duration and subsequently the FAR target per single LP-WUS decoding attempt/trial is going to be different for the two monitoring modes. For example, this may require the LP-WUS design under ‘always-on’ monitoring to have a longer sequence and/or a longer CRC compared to the LP-WUS design under ‘duty-cycled’ monitoring.

In RAN1#112bis-e, the following working assumption on the FAR target is agreed

Working Assumption For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design.

Observation 5: For the same FAR and reference time duration in FAR Alt 2, LP-WUS design under ‘always-on’ monitoring may require a longer sequence and/or a longer CRC compared to the LP-WUS design under ‘duty-cycled’ monitoring.

Proposal 4: Confirm the working assumption on FAR target and consider MR’s DRX cycle or LP-WUR’s duty cycle as the reference time duration for ‘always-on’ and ‘duty-cycled’ monitoring modes, respectively.

## Receiver Processing Details

The fundamental receiver processing timelines in RRC_IDLE and RRC_INACTIVE states for the DRX, eDRX, LP-WUS power saving schemes are shown in Figure 1, Figure 2, and Figure 3/Figure 4, respectively.

Additional details on the assumptions on the receiver processing timeline for paging reception during a DRX cycle (i.e., for DRX power saving scheme or during DRX cycle in PTW for the eDRX power saving scheme) are shown in Figure 6 and Figure 7 without PEI and with PEI, respectively. In Figure 6, the number of SSB bursts processed, PO monitoring duration, SSB burst processing duration for inter-frequency measurements, and intermediate sleep states are shown for three different SNR cases (low, medium, and high) when PEI is not configured/supported. Further, depending on the group paging rate/probability, either PDCCH only or PDCCH/PDSCH decoding is considered during paging reception. Figure 7 show similar information in addition to the PEI-O monitoring duration and location when PEI is configured/supported.

Figure 6: Assumptions on Receiver Processing for Paging Reception Per DRX (eDRX if TeDRX<1024 frames) Cycle without PEI Configuration.

Figure 7: Assumptions on Receiver Processing for Paging Reception Per DRX (eDRX if TeDRX<1024 frames) Cycle with PEI Configuration.

The LP-WUR processing timeline depends on multiple factors including the LP-WUS content and UE behavior in response to reception of the LP-WUS as discussed in Section 2.2 (i.e., whether LP-WUS is addressed to a UE or a UE group, and whether the MR is required to monitor POs or not), the LP-WUR’s mode of monitoring (i.e., ‘duty-cycled’ or ‘always-on’ monitoring), the criteria for the transmission of the LP-WUS when it is addressed to a UE group under LP-WUR’s ‘always-on’ monitoring mode, and the RRM measurement assumptions.

The criteria for LP-WUS transmission when it is addressing a UE group under ‘always-on’ monitoring mode will result in a trade-off between power saving gain and the latency experienced by each UE within the group. For the purpose of power saving gain evaluation, we may consider an average number of LP-WUS transmissions in a transaction cycle as the ratio between the group paging rate and the UE paging rate, which is approximately equivalent to the number of UEs per paging group for low enough UE/group paging rates, subsequently resulting in no impact on experienced latency compared to uniquely UE addressed LP-WUSs.

Observation 6: For LP-WUS addressing a UE group, there is a trade-off between power saving gain and latency that is dependent on the criteria for LP-WUS transmission when ‘always-on’ (or short ‘duty-cycled’) monitoring mode is considered.

For RRM measurements, the MR, the LP-WUR, or a combination of both may be considered for measurements while the LP-WUR is used to monitor LP-WUSs. Therefore, we may consider any of the following four different options for power saving gain evaluation

- Meas_Option (1): MR performs serving cell and intra/inter-frequency measurements.

- Meas_Option (2): LP-WUR performs serving cell measurements and MR performs intra/inter-frequency measurements.

- Meas_Option (3): LP-WUR performs serving cell and intra-frequency measurements and MR performs inter-frequency measurements.

- Meas_Option (4): LP-WUR performs serving and intra/inter-frequency measurements.

When the MR is being considered for RRM measurements, i.e., Meas_Option (1-3), we can assume that the MR is configured, e.g., for measurement purposes only, with eDRX. However, to avoid any impact on the experienced latency by the UE, the LP-WUR should keep track of at least a tracking/notification area level beacon that can be transmitted with higher periodicity, e.g., a DRX cycle, which may be selected based on a target resource overhead. For a maximum LP-WUR’s power saving gain, the eDRX cycle may be selected as the largest value, i.e., eDRX cycle of 10485.76 seconds, and measurements may be performed for 4 DRX cycles within the PTW. Other values may also be considered depending on the use case. Further, we may assume that the MR can skip measurements in an eDRX cycle if it has recently woken up by a LP-WUS, either uniquely or group addressed based on the (re-)synchronization timeline assumptions discussed below for a MR transitioning from the ‘Ultra-deep sleep’ power state.

During the evaluation of eDRX and LP-WUS power saving schemes, the (re-)synchronization timeline shown in Figure 8 can be considered as a first alternative (Alt 1) following similar assumptions as proposed in [9] where the MR performs PSS/SSS search upon transitioning from ‘Ultra-deep sleep’ state for a duration of [40, 80, 120] ms at a relative power consumption corresponding to inter-frequency measurement and search, e.g., 150 units, followed by PBCH DMRS acquisition for [1, 2, 3] SSB periods, depending on the SNR case, i.e., [high, medium, low].

Figure 8: Assumptions on MR Processing during Ramp-up and (Re-)Synchronization after Waking-up from ‘Ultra-deep sleep’ Power State.

Noting that MR’s wake-up due to the detection of a LP-WUS may already imply some timing and serving cell information, especially for duty-cycled LP-WUR, a second alternative (Alt 2) can be considered where the performance of PSS/SSS search upon transitioning from ‘Ultra-deep sleep’ state for a duration of [40, 80, 120] ms may not be required.

Observation 7: For MR’s (re-)synchronization after LP-WUS detection, LP-WUS may already imply/ convey some timing and serving cell information to the MR.

Proposal 5: Further study the following two alternatives for MR’s (re-)synchronization after LP-WUS detection.
MR Sync Alt 1: PSS/SSS search for, e.g., [40, 80, 120]ms, with no LP-WUS assistance.
MR Sync Alt 2: No PSS/SSS search with LP-WUS assistance.

# Evaluation Results

For the set of results in this section, we consider the list of parameters, schemes, and corresponding values/options defined in Table 4, FAR Alt 2, and Alt 1 for main radio’s “ultra-deep sleep” additional transition energy and ramp-up time of (15000, 400ms).

Table 4: List of Evaluation Parameters, Schemes, and Corresponding Values/Options

| Parameter/Identifier | Definition / Description | Value/Option |
| --- | --- | --- |
| A (baseline) | DRX Cycle of the MR during evaluation of baseline schemes. | Config_1: 5.12 (s) Config_2: 2.56 (s) |
| B (baseline) | eDRX Cycle of the MR during evaluation of baseline schemes. | {61.44s} (s) |
| PTW | Paging Time Window – on period/duration of eDRX cycle when eDRX cycle is >1024 radio frames. | Config_1: 20.48 (s) Config_2: 10.24 (s) |
| L_wus | WUS transmission duration | 1 (ms) |
| X | Total time required to wake-up the main radio (i.e., from ‘ultra-deep sleep’ power state). | MR Sync Alt 1: 580 (ms) MR Sync Alt 2: 460 (ms) |
| N | Number of UEs per paging group | {10} |
| PC_model | MR power consumption model (“eMBB” or “RedCap”) based on Table 1 and Table 2 | “RedCap” |
| PLR, on | Relative power of the LP-WUR ‘on’ power state based on values in Table 3. | [0.01/0.05/0.1/0.5/1/2/4] |
| LP-WUS_Content | The content of the LP-WUS can either be “UE ID” (corresponding to UE_Behavior (1)) or “UE Group ID” (corresponding to UE_Behavior (2)) | “UE Group ID” |
| Group_Paging_Criteria | The criteria for UE group addressed LP-WUS transmission when ‘always-on’ (or short ‘duty-cycled’) monitoring is considered. | “at every UE paging arrival” |
| Meas_Option | The RRM measurement options as defined in Section 2.3, e.g., “Option1” refers to MR performing serving cell & intra/inter-frequency measurements. | “Option1” |
| MR_Meas_Config | eDRX cycle (B_meas) and number of DRX cycles (M) per PTW [B_meas, M] considered by the MR for RRM measurement purposes only when LP-WUR is monitoring for LP-WUS. | [m×A (s), 1 cycles], m=10 |
| Y | i-DRX cycle duration in seconds. | 1.28 |

Two sets of configurations are considered in Table 5 for the MR/LP-WUR during the evaluation of the LP-WUS power consumption such that the latencies achieved by LP-WUS and the considered baseline scheme are approximately matched. Configuration “Config_1” is considered when the performance of LP-WUS is compared against DRX power saving scheme as a baseline, whereas configuration “Config_2” is considered when the performance of LP-WUS is compared against eDRX power saving scheme as a baseline.

Table 5: MR/LP-WUR Configurations to Match Baseline Schemes’ Latency

| Parameter | Definition / Description | Config_1A (DRX as baseline1) | Config_1B (eDRX as baseline1) | Config_2A (DRX as baseline1) | Config_2B (eDRX as baseline1) |
| --- | --- | --- | --- | --- | --- |
| A | DRX Cycle of the MR. | 2.56 (s) | 0.64 (s) | 1.28/0.64 (s) | 2.56 (s) |
| Monitoring_Mode | The monitoring mode of the LP-WUR which can either be “duty-cycled” or “always-on” | Scenario 1: “always-on” Scenario 2: “duty-cycled” | “duty-cycled” | Scenario 1: “always-on” Scenario 2: “duty-cycled” | “duty-cycled” |
| Awus | Duty Cycle of the LP-WUR. | 1.5×2.56 (s) | 46×0.64 (s) | 2×0.64 (s) | 32×2.56 (s) |
| Latency_Definition | Definition of latency considered where “Option2” [“Option1”] is used when MR is [not] required to monitor POs after LP-WUS detection. | “Option2” | “Option2” | “Option2” | “Option2” |
| Note1: PEI configuration does not impact latency. |  |  |  |  |  |

The resulting latency from LP-WUS and each of the baseline schemes is shown in Table 6 where it is noted that the two values for LP-WUS latencies correspond to the two alternatives of MR re-synchronization, as captured by X in Table 4, during its transition from “Ultra-deep sleep” power state.

Table 6: Latency % Reduction/Increase of LP-WUS Compared to DRX/eDRX schemes with Config_1

| Power Saving Scheme |  | Latency [s] |  | Reduction (+) / Increase (-) [%] |
| --- | --- | --- | --- | --- |
| LP-WUS | - | 115.18-15.30 | 22.12-2.5 | - |
| DRX | w/o PEI | 2.56 |  | 2[2.34-17.19] |
|  | w/ PEI | 2.56 |  | 2[2.34-17.19] |
| eDRX | w/o PEI | 15.36 |  | 1[0.39-1.17] |
|  | w/ PEI | 15.36 |  | 1[0.39-1.17] |
| Note1: main radio/LP-WUR is configured according to Config_1B in Table 5. Note2: main radio/LP-WUR is configured according to Config_1A in Table 5. Note3: PEI configuration does not impact latency. |  |  |  |  |

Table 7: Latency % Reduction/Increase of LP-WUS Compared to DRX/eDRX schemes with Config_2

| Power Saving Scheme |  | Latency [s] |  | Reduction (+) / Increase (-) [%] |
| --- | --- | --- | --- | --- |
| LP-WUS | - | 120.94-21.06 | 21.1-1.54 | - |
| DRX | w/o PEI | 1.28 |  | 2[-20.3-14.06] |
|  | w/ PEI | 1.28 |  | 2[-20.3-14.06] |
| eDRX | w/o PEI | 21.76 |  | 1[3.22-3.77] |
|  | w/ PEI | 21.76 |  | 1[3.22-3.77] |
| Note1: main radio/LP-WUR is configured according to Config_2B in Table 5. Note2: main radio/LP-WUR is configured according to Config_2A in Table 5. Note3: PEI configuration does not impact latency. |  |  |  |  |

## Power Saving Gain for ~2.5,15s Target Latency in Table 6

In this section, power saving gains of LP-WUS compared to DRX/eDRX as a baseline schemes are presented for FAR values ∈0.1%,1%, number of UEs per paging group N=10, measurement relaxation factor m∈6,10×5.12s, and LP-WUR and MR are configured according to Config_1/1A/1B in Table 4 and Table 5 for both baseline and LP-WUS power saving schemes. The LP-WUR is configured to perform Scenario 1: ‘always-on’ or Scenario 2: ‘duty-cycled’ monitoring and PEI is configured for the DRX baseline power saving scheme. When LP-WUR is configured to perform ‘duty-cycled’ monitoring, the minimum between 1% duty-cycle and 100 ms is considered for the ON duration.

### DRX as baseline and ‘always-on’ monitoring

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 1% is shown in Figure 9 for LP-WUR ON state relative power Pon≤0.1 units and in Figure 10 for 1≥Pon>0.1. Results show that at a FAR target of 1% and using ‘always-on’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >30% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 9: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and Pon≤0.1 units.

Figure 10: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and 1≥Pon>0.1 units.

Observation 8: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈-7%,64% with mean >30% at PON ≤0.1, 1% FAR target, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 0.1% is shown in Figure 11 for LP-WUR ON state relative power Pon≤0.1 units and in Figure 12 for 1≥Pon>0.1. Results show that at a FAR target of 0.1% and using ‘always-on’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >40% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 11: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and Pon≤0.1 units.

Figure 12: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and 1≥Pon>0.1 units.

Observation 9: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈2%,69% with mean >40% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 0.1% is shown in Figure 13 for LP-WUR ON state relative power Pon≤0.1 units when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘always-on’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >50% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 13: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target, Pon≤0.1 units, and (re-) synchronization energy <2000 unit×ms.

Observation 10: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈32%,69% with mean >50% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

### DRX as baseline and ‘duty-cycled’ monitoring

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 14 for FAR target of 1% and in Figure 15 for FAR target of 0.1%. Results show that at a FAR target ≤1% and using ‘duty-cycled’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >40% compared to I-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 14: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and Pon≤4 units.

Figure 15: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and Pon≤4 units.

Observation 11: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈1%,70% with mean >40% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 16 for FAR target of 0.1% when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘duty-cycled’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >50% compared to I-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 16: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target, Pon≤4 units, and (re-) synchronization energy <2000 unit×ms.

Observation 12: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈37%,70% with mean >50% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

### eDRX as baseline and ‘duty-cycled’ monitoring

Power saving gain (PSG) for LP-WUS compared to e-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 17 for FAR target of 1% and in Figure 18 for FAR target of 0.1%. Results show that at a FAR target ≤1% and using ‘duty-cycled’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >15% compared to e-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 17: PSG of LP-WUS Compared to e-DRX with PEI at 1% FAR target and Pon≤4 units.

Figure 18: PSG of LP-WUS Compared to e-DRX with PEI at 0.1% FAR target and Pon≤4 units.

Observation 13: Compared to e-DRX with PEI at target latency (~15s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈-18%,48% with mean >15% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to e-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 19 for FAR target of 0.1% when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘duty-cycled’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >25% compared to e-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 19: PSG of LP-WUS Compared to e-DRX with PEI at 0.1% FAR target, Pon≤4 units, and (re-) synchronization energy <2000 unit×ms.

Observation 14: Compared to e-DRX with PEI at target latency (~15s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈4%,48% with mean >25% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

## Power Saving Gain for ~1.2,21s Target Latency in Table 7

In this section, power saving gains of LP-WUS compared to DRX/eDRX as a baseline schemes are presented for FAR values ∈0.1%,1%, number of UEs per paging group N=10, measurement relaxation factor m∈20,40×2.56s, and LP-WUR and MR are configured according to Config_2/2A/2B in Table 4 and Table 5 for both baseline and LP-WUS power saving schemes. The LP-WUR is configured to perform Scenario 1: ‘always-on’ or Scenario 2: ‘duty-cycled’ monitoring and PEI is configured for the DRX baseline power saving scheme. When LP-WUR is configured to perform ‘duty-cycled’ monitoring, the minimum between 1% duty-cycle and 100 ms is considered for the ON duration.

### DRX as baseline and ‘always-on’ monitoring

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 1% is shown in Figure 20 for LP-WUR ON state relative power Pon≤0.1 units and in Figure 21 for 1≥Pon>0.1. Results show that at a FAR target of 1% and using ‘always-on’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >55% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 20: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and Pon≤0.1 units.

Figure 21: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and 1≥Pon>0.1 units.

Observation 15: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈33%,78% with mean >55% at PON ≤0.1, 1% FAR target, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 0.1% is shown in Figure 22 for LP-WUR ON state relative power Pon≤0.1 units and in Figure 23 for 1≥Pon>0.1. Results show that at a FAR target of 0.1% and using ‘always-on’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >65% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 22: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and Pon≤0.1 units.

Figure 23: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and 1≥Pon>0.1 units.

Observation 16: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈47%,86% with mean >65% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at FAR target of 0.1% is shown in Figure 24 for LP-WUR ON state relative power Pon≤0.1 units when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘always-on’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >70% compared to I-DRX with PEI at Pon≤0.1 units and RE,REF<1%.

Figure 24: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target, Pon≤0.1 units, and (re-) synchronization energy <2000 unit×ms.

Observation 17: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈57%,86% with mean >70% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

### DRX as baseline and ‘duty-cycled’ monitoring

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 25 for FAR target of 1% and in Figure 26 for FAR target of 0.1%. Results show that at a FAR target ≤1% and using ‘duty-cycled’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >60% compared to I-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 25: PSG of LP-WUS Compared to I-DRX with PEI at 1% FAR target and Pon≤4 units.

Figure 26: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target and Pon≤4 units.

Observation 18: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈36%,87% with mean >60% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to I-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 27 for FAR target of 0.1% when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘duty-cycled’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >75% compared to I-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 27: PSG of LP-WUS Compared to I-DRX with PEI at 0.1% FAR target, Pon≤4 units, and (re-) synchronization energy <2000 unit×ms.

Observation 19: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈61%,87% with mean >75% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

### eDRX as baseline and ‘duty-cycled’ monitoring

Power saving gain (PSG) for LP-WUS compared to e-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 28 for FAR target of 1% and in Figure 29 for FAR target of 0.1%. Results show that at a FAR target ≤1% and using ‘duty-cycled’ monitoring mode for the LP-WUR and “Ultra-deep sleep” power state for the MR, LP-WUS can provide an average PSG >40% compared to e-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 28: PSG of LP-WUS Compared to e-DRX with PEI at 1% FAR target and Pon≤4 units.

Figure 29: PSG of LP-WUS Compared to e-DRX with PEI at 0.1% FAR target and Pon≤4 units.

Observation 20: Compared to e-DRX with PEI at target latency (~21s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈13%,65% with mean >40% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Power saving gain (PSG) for LP-WUS compared to e-DRX power saving scheme with PEI at LP-WUR ON state relative power Pon≤4 units  and LP-WUR duty cycle ratio <=1% is shown in Figure 30 for FAR target of 0.1% when (re-) synchronization energy is limited to <2000 unit×ms, e.g., assuming LP-WUS assistance for MR re-synchronization. Results show that at a FAR target of 0.1% and using ‘duty-cycled’ monitoring mode for the LP-WUR, “Ultra-deep sleep” power state for the MR, and LP-WUS for MR’s (re)synchronization, LP-WUS can provide an average PSG >40% compared to e-DRX with PEI at Pon≤4 units and RE,REF<1%.

Figure 30: PSG of LP-WUS Compared to e-DRX with PEI at 0.1% FAR target, Pon≤4 units, and (re-) synchronization energy <2000 unit×ms.

Observation 21: Compared to e-DRX with PEI at target latency (~21s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈22%,65% with mean ≥45% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

# Conclusion

The contribution discussed performance evaluation metrics, evaluation assumptions, and presented results for the comparison between LP-WUS/WUR and DRX/eDRX power saving schemes. The contribution presented the following proposals and observations.

LP-WUS Evaluation Methodology

Observation 1: For group-addressed LP-WUS, ‘always-on’ monitoring mode may not result in any latency reduction benefits due to the necessity of MR’s monitoring of legacy POs after LP-WUS detection but may alleviate the requirement for frequent LP-WUR synchronization.

Proposal 1: For a LP-WUS carrying a UE unique ID, the latency is defined as the average time between the arrival of data at gNB and the UE’s completion of MR synchronization upon detection of a corresponding LP-WUS.

Proposal 2: Consider ignoring the impact of system information update on IDLE/INACTIVE state latency due to its infrequent occurrence as part of paging procedure.

Observation 2: Mapping between relative power unit values and LP-WUR coverage ratio/probability, and coverage mismatch, i.e., between LP-WUS and PDCCH, handling needs to be defined for proper evaluation of power saving gains/losses.

Proposal 3: Coverage mismatch between LP-WUS and PDCCH should be considered when power savings gain/network resource overhead are evaluated and the following 3 options should be adopted

- LP-WUR full coverage without RX mode of operation switching.

- LP-WUR limited coverage with switching to MR during out-of-coverage.

- LP-WUR full coverage with RX mode (e.g., two or more) switching.

Observation 3: LP-WUS design options/functions and mapping to time/frequency resource requirements need to be defined for proper evaluation of the network resource overhead in support of LP-WUS/WUR.

Observation 4: A UE may miss the detection of a true LP-WUS due to a  false alarm, generated by Noise/Interference, leading to a potential increase in latency and network resource overhead for LP-WURs configured with ‘always-on’ or short ‘duty-cycled’ monitoring.

Observation 5: For the same FAR and reference time duration in FAR Alt 2, LP-WUS design under ‘always-on’ monitoring may require a longer sequence and/or a longer CRC compared to the LP-WUS design under ‘duty-cycled’ monitoring.

Proposal 4: Confirm the working assumption on FAR target and consider MR’s DRX cycle or LP-WUR’s duty cycle as the reference time duration for ‘always-on’ and ‘duty-cycled’ monitoring modes, respectively.

Observation 6: For LP-WUS addressing a UE group, there is a trade-off between power saving gain and latency that is dependent on the criteria for LP-WUS transmission when ‘always-on’ (or short ‘duty-cycled’) monitoring mode is considered.

Observation 7: For MR’s (re-)synchronization after LP-WUS detection, LP-WUS may already imply/ convey some timing and serving cell information to the MR.

Proposal 5: Further study the following two alternatives for MR’s (re-)synchronization after LP-WUS detection.
MR Sync Alt 1: PSS/SSS search for, e.g., [40, 80, 120]ms, with no LP-WUS assistance.
MR Sync Alt 2: No PSS/SSS search with LP-WUS assistance.

Evaluation Results for Power Consumption and Latency

Observation 8: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈-7%,64% with mean >30% at PON ≤0.1, 1% FAR target, and RE,REF<1%.

Observation 9: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈2%,69% with mean >40% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Observation 10: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘always-on’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈32%,69% with mean >50% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Observation 11: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈1%,70% with mean >40% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Observation 12: Compared to i-DRX with PEI at target latency (~2.5s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈37%,70% with mean >50% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

Observation 13: Compared to e-DRX with PEI at target latency (~15s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈-18%,48% with mean >15% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Observation 14: Compared to e-DRX with PEI at target latency (~15s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈4%,48% with mean >25% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

Observation 15: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈33%,78% with mean >55% at PON ≤0.1, 1% FAR target, and RE,REF<1%.

Observation 16: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring can provide power saving gain ∈47%,86% with mean >65% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Observation 17: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘always-on’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈57%,86% with mean >70% at PON ≤0.1, 0.1% FAR target, and RE,REF<1%.

Observation 18: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈36%,87% with mean >60% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Observation 19: Compared to i-DRX with PEI at target latency (~1.25s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈61%,87% with mean >75% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

Observation 20: Compared to e-DRX with PEI at target latency (~21s), LP-WUS with ‘duty-cycled’ monitoring can provide power saving gain ∈13%,65% with mean >40% at PON ≤4, FAR target ≤1%, and RE,REF<1%.

Observation 21: Compared to e-DRX with PEI at target latency (~21s), LP-WUS with ‘duty-cycled’ monitoring and assistance for MR re-synchronization can provide power saving gain ∈22%,65% with mean ≥45% at PON ≤4, 0.1% FAR target, and RE,REF<1%.

# References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#94e, Dec. 6-17, 2021.

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#97e, Sept. 12-16, 2022.

R1-2208379, “Low Power WUS Receiver Architectures Considerations and Modeling”, FutureWei, RAN1#110-bis-e, October 10-19, 2022.

3GPP TS 38.304, “NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 17)” V17.1.0 (2022-06).

3GPP TS 22.261 “Service requirements for the 5G system; Stage 1 (Release 18)” Appendix G.1 and G.2 for asset tracking, V18.6.1 (2022-06).

3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)”, V16.0.0 (2019-06).

3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 18)”, V18.3.0 (2021-12).

3GPP TR 38.875, “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).

R1-2208698, “Low power WUS Evaluation Methodology”, Nokia, Nokia Shanghai Bell, RAN1#110bis-e, October 10-19, 2022.

# Appendices

## Appendix A: Latency of DRX and eDRX Schemes

Given the assumption of uniform arrival of network events within a transaction cycle, the average latency of the DRX scheme can be derived as

LDRX=0AA-xfxdx=0AA-xAdx

=0A1-xAdx=x​0A-x22A​0A
=A-A2=A2 ,

whereas the average latency of the eDRX scheme can be derived as

LeDRX=PTWB0AA-xfxdx+1-PTWB0B-PTWB-PTW-yfydy

=PTWBA2+1-PTWBB-PTW2 ,

where network events arrival is uniform in 0, A, i.e., fx=1/A, with probability PTW/B and uniform in 0, B-PTW, i.e., fy=1/(B-PTW), with probability 1-PTW/B.

## Appendix B: Latency of LP-WUS

Considering the uniform arrival of network events within a transaction cycle, the time required to process the LP-WUS and wake-up the main radio D, and the restriction of the main radio to the DRX configuration, the average latency of the LP-WUS can be derived as

LWUR=0A-DA-xfxdx+A-DAx+Afxdx 
=0A-D1-xAdx +A-DAxA+1dx 
=x​0A-D-x22A ​0A-D+x22A ​A-DA+x ​A-DA 
=3A2-A-D2A

where D<A and fx=1/A.

## Appendix C: RAN1 Agreements

### RAN1#110-bis-e

Agreement For evaluation, 1 Rx chain for LP-WUS receiver is baseline. Agreement Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI. - FFS: Further prioritization if needed during the study item. Agreement Take the following power model for main radio for evaluation in LP-WUS/WUR SI, - For IoT and wearable cases, reuse TR38.875 power model as baseline. - For eMBB and other cases, reuse TR38.840 power model as baseline. - Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver - FFS: The details of ‘Ultra-deep sleep’ power state Agreement - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.], - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.], - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS Agreement - The following power model for LP-WUR/WUS evaluation is considered, - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: - [0.001] - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] - Other values are not precluded to be evaluated. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed. Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) FFS: LP-WUR power consumption values for FR2. Agreement For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered, - XR traffic model with evaluation methodologies and assumptions captured in TR 38.838. - eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840 - Heartbeat traffic models in 3GPP TR 38.875. - Other models are not precluded. Company to further provide the followings, - Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.) - How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring - Other details if any Agreement - For LP-WUS coverage evaluation, the noise figure of LP-WUR is - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies - FFS: how to determine the NF option. - The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion. Agreement For the performance evaluations of LP-WUS candidate designs, it is assumed that - The miss-detection rate (MDR) of LP-WUS [1%], - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying reported by companies - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead - FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details. Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. FFS: Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS]. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD. FFS: UPT; FFS Note: it is for connected mode purpose.. - Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI - Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact - Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement The following is assumed for RRC IDLE/INACTIVE evaluation, Parameters; Value. i-DRX cycle length; 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI. e-DRX cycle length; 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE. Number of POs in Paging Frame; 1. Number of DRXs per PTW; 4. Number of SSB before PO / PEI; 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’. Sync/re-sync after ultra-deep sleep; companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync. RRM Measurement; Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not.. LP-WUS monitoring; Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle.. Traffic; Option 1 (baseline): Per UE paging rate (R_E)= [1%] or [0.1%] or [0.01%] or [0.001%] within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [3000]. Other options are not precluded can be reported by companies.. Others; Reported by companies. Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. - Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss. - Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

### RAN1#111

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency. In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included.. UPT; The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. - Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI - Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact - Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW - L=4 (as agreed in RAN1#110bis) Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112 Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.01/0.05/0.1/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

### RAN1#112

Conclusion: The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any. Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases. Agreement For coverage evaluation, the following is used, Number of RX chains at the UE’s MR; Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS.. Number of RX chains for LP-WUR; 1 Rx Note: agreed in RAN1#110bis. Scenario and frequency; Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 700MHz (FDD). Reference data rates for MR; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps. Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Pathloss model (select from LoS or NLoS); Urban: NloS Rural: NloS. Bandwidth; 100MHz for 4GHz and 2.6GHz. 20MHz (optional for 10MHz) for 700MHz. (FDD). Channel model for link-level simulation; TDL-C for NLOS. Delay spread; Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns. UE velocity; Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor. Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 700 MHz - 64TxRUs for 2.6 and 4 GHz.. Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis. Agreement For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS Attributes; Assumptions. Carrier Frequency; 2.6GHz/4GHz/700MHz. Waveform; OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used. Channel structure; - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence).. SCS of OFDM generator for NR signal; 30kHz/15KHz. Configuration for LP-WUS signal; For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report. WUS duration; Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols. MDR/FAR assumption; - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials. Code scheme; Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….). gNB Channel BW; 20MHz, FFS other values. LP-WUS BW; Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS. Filter; X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed. Adjacent subcarrier interference; - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional. Sampling Rate; - Companies to report.. ADC bit width; 1-bit, 4-bit, 8-bit, ideal and other options are not precluded. Channel Model; See link coverage assumption table (will copy and paste here). Impairment modelling; - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report. Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4, - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option 3/4 is not applicable - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than [TBD] power unit, - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR. - assumptions important for achieving performance by using MR clock for LR should be declared - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm). - Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr - Model 2: random time drifting, FFS details - FFS: Phase noise model

### RAN1#112bis-e

Agreement Update as followings for the e-DRX paging probability Note: - For i-DRX with cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW - L=4 Agreement Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model. - Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 Working Assumption For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm), - Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal Agreement The period of synchronization signal that LP-WUR used for at least power evaluation can be - Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR - For evaluations assuming LP-SS - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement. Working Assumption For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design. Agreement RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Option #1: PDCCH for paging - Option #2: PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels, e.g., the message size for MSG3 and etc. Agreement Confirm Alt 2 in the following agreement and update as follows Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases. Agreement Confirm the WA from RAN1#112 and update as followings Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm], FFS: RTC frequency drift [ppm/s]; - FFS:[0.1]). - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4, - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option 3/4 is not applicable - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than [TBD] power unit, - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR. - assumptions important for achieving performance by using MR clock for LR should be declared - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm). - Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr - Model 2: random time drifting, FFS details - FFS: Phase noise model Working Assumption The following for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. TBD: value(s); For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. [Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power] FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1 Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.