3GPP TSG RAN WG1 #113		R1-2304837

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.1.3.1

Source:	Google

Title:	On DMRS Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #112b, the following agreements on DMRS enhancement have been achieved.

Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP, - Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 68 in Table 7.3.1.2.2-2A-X. - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. …; …; …; …. 68; 2; 0,2,3; 1. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 60 in Table 7.3.1.2.2-3A-X. - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 60; 2; 0,2,3. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 128 in Table 7.3.1.2.2-4A-X. - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. …; …; …; …. 128; 2; 0,2,3; 1. Agreement Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH: - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. - Note: The above Working Assumption for CB based PUSCH may be confirmed later. Agreement For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles. - Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - FFS: The factor () for partial coherent TPMIs Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, - For 2 CWs, - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red). - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11. [4]; [2]; [0,1,2,3,10]. [5]; [2]; [0,1,8,2,3,10]. [6]; [2]; [0,1,8,2,3,10,11]. [7]; [2]; [0,1,8,9,2,3,10,11]. [8]; [2]; [0,2,3,8,9]. [9]; [2]; [0,1,2,3,8,9]. FFS: Additional rows (rows 4~9) if there is technical justification. Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, - For 1 CW, - Do not support row 21-22 - FFS: Whether to support row 23 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 21; [2]; [8-10]. 22; [2]; [8-11]. Working Assumption - Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH. - FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH. Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0-4; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6,7; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-4. 1-15; Reserved; Reserved. Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-5. 1-15; Reserved; Reserved. Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-4; 1. 1; 2; 0,1,2,3,6; 2. 12-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-5; 1. 1; 2; 0,1,2,3,6,8; 2. 2-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8; 2. 1-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8,9; 2. 1-31; Reserved; Reserved; Reserved. Agreement For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X. - FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. - FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64. Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 2; 0-4; 2]. 1; 1; 1; 1; [1; 2; 0,1,2,3,4,6; 2]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,4,5,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,4,5,6,7; 2]. 4; 2; 1; 1; 4; 2; 0,1,2,3,8; 1. 5; 2; 2; 1; 5; 2; 0,1,2,3,8,10; 1. 6; 2; 3; 1; 6; 2; 0,1,2,3,8,9,10; 1. 7; 2; 0,1; 1; 7; 2; 0,1,2,3,8,9,10,11; 1. [8; 2; 2,3; 1]; [8; 1; 0,1,4,5,8; 2]. [9; 2; 0-2; 1]; [9; 1; 0,1,4,5,8,12; 2]. [10; 2; 0-3; 1]; [10; 1; 0,1,4,5,8,9,12; 2]. [11; 2; 0,2; 1]; [11; 1; 0,1,4,5,8,9,12,13; 2]. 12; 2; 0; 2; [12; 2; 0,1,4,5,8; 2]. 13; 2; 1; 2; [13; 2; 0,1,4,5,8,12; 2]. 14; 2; 2; 2; [14; 2; 0,1,4,5,8,9,12; 2]. 15; 2; 3; 2; [15; 2; 0,1,4,5,8,9,12,13; 2]. 16; 2; 4; 2; [16; 2; 2,3,6,7,10; 2]. 17; 2; 5; 2; [17; 2; 2,3,6,7,10,14; 2]. 18; 2; 6; 2; [18; 2; 2,3,6,7,10,11,14; 2]. 19; 2; 7; 2; [19; 2; 2,3,6,7,10,11,14,15; 2]. 20; 2; 0,1; 2; [20; 2; 0,1, 2,3,10; 1]. 21; 2; 2,3; 2; [21; 2; 0,1,8,2,3,10; 1]. 22; 2; 4,5; 2; [22; 2; 0,1,8, 2,3,10,11; 1]. 23; 2; 6,7; 2; [23; 2; 0,1,8,9,2,3,10,11; 1]. [24; 2; 0,4; 2]; [24; 1; 0,1,4,5,12; 2]. [25; 2; 2,6; 2]; [25; 1; 0,1,8,4,5,12; 2]. [26; 2; 0,1,4; 2]; [26; 1; 0,1,8,4,5,12,13; 2]. [27; 2; 2,3,6; 2]; [27; 1; 0,1,8,9,4,5,12,13; 2]. [28; 2; 0,1,4,5; 2]; [28; 2; 0,1,4,5,12; 2]. [29; 2; 2,3,6,7; 2]; [29; 2; 0,1,8,4,5,12; 2]. [30; 2; 0,2,4,6; 2]; [30; 2; 0,1,8,4,5,12,13; 2]. 31; 1; 8; 1; [31; 2; 0,1,8,9,4,5,12,13; 2]. 32; 1; 9; 1; [32; 2; 2,3,6,7,14; 2]. 33; 1; 8,9; 1; [33; 2; 2,3,10,6,7,14; 2]. 34; 2; 8; 1; [34; 2; 2,3,10,6,7,14,15; 2]. 35; 2; 9; 1; [35; 2; 2,3,10,11,6,7,14,15; 2]. 36; 2; 10; 1; [36; 2; 0,2,3,8,9; 1]. 37; 2; 11; 1; [37; 2; 0,1,2,3,8,9; 1]. 38; 2; 8,9; 1. 39; 2; 10,11; 1. [40; 2; 8-10; 1]. [41; 2; 8-11; 1]. [42; 2; 8,10; 1]. 43; 2; 8; 2. 44; 2; 9; 2. 45; 2; 10; 2. 46; 2; 11; 2. 47; 2; 12; 2. 48; 2; 13; 2. 49; 2; 14; 2. 50; 2; 15; 2. 51; 2; 8,9; 2. 52; 2; 10,11; 2. 53; 2; 12,13; 2. 54; 2; 14,15; 2. [55; 2; 8,12; 2]. [56; 2; 10,14; 2]. [57; 2; 8,9,12; 2]. [58; 2; 10,11,14; 2]. [59; 2; 8,9,12,13; 2]. [60; 2; 10,11,14,15; 2]. 61; 2; 8,10,12,14; 2. 62; 1; 0,1,8; 1. 63; 1; 0,1,8,9; 1. 64; 2; 0,1,8; 1. 65; 2; 0,1,8,9; 1. 66; 2; 2,3,10; 1. 67; 2; 2,3,10,11; 1. [69; 1; 0,1,8; 2]. [70; 1; 0,1,8,9; 2]. [71; 1; 4,5,12; 2]. [72; 1; 4,5,12,13; 2]. [73; 2; 0,1,8; 2]. [74; 2; 0,1,8,9; 2]. [75; 2; 4,5,12; 2]. [76; 2; 4,5,12,13; 2]. [77; 2; 2,3,10; 2]. [78; 2; 2,3,10,11; 2]. [79; 2; 6,7,14; 2]. [80; 2; 6,7,14,15; 2]. [81; 2; 5,8,9; 2]. [82; 2; 7,10,11; 2]. [83; 2; 7,12,13; 2]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X. - FFS: For rows 9, 10, 20-23, 33, 34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0; 0; 3; 0-4. 1; 1; 1; 1; 3; 0-5. 2; 1; 0,1; [2; 3; 12-16]. 3; 2; 0; [3; 3; 12-17]. 4; 2; 1; 4; 2; 0,1,2,3,12. 5; 2; 2; 5; 2; 0,1,2,3,12,14. 6; 2; 3; 6; 2; 0-3,12-14. 7; 2; 0,1; 7; 2; 0-3,12-15. 8; 2; 2,3; [8; 3; 0,1,2,3,12]. [9; 2; 0-2]; [9; 3; 0,1,2,3,12,14]. [10; 2; 0-3]; [10; 3; 0-3,12-14]. 11; 3; 0; [11; 3; 0-3,12-15]. 12; 3; 1; [12; 2; 0,2,3,12,13]. 13; 3; 2; [13; 2; 0,1,2,3,14]. 14; 3; 3; [14; 2; 0,1,12,2,3,14]. 15; 3; 4; [15; 2; 0,1,12,2,3,14,15]. 16; 3; 5; [16; 2; 0,1,12,13,2,3,14,15]. 17; 3; 0,1; [17; 3; 0,1,2,3,14]. 18; 3; 2,3; [18; 3; 0,1,12,2,3,14]. 19; 3; 4,5; [19; 3; 0,1,12,2,3,14,15]. [20; 3; 0-2]; [20; 3; 0,1,12,13,2,3,14,15]. [21; 3; 3-5]. [22; 3; 0-3]. [23; 2; 0,2]. 24; 1; 12. 25; 1; 13. 26; 1; 12,13. 27; 2; 12. 28; 2; 13. 29; 2; 14. 30; 2; 15. 31; 2; 12,13. 32; 2; 14,15. [33; 2; 12-14]. [34; 2; 12-15]. 35; 3; 12. 36; 3; 13. 37; 3; 14. 38; 3; 15. 39; 3; 16. 40; 3; 17. 41; 3; 12,13. 42; 3; 14,15. 43; 3; 16,17. [44; 3; 12-14]. [45; 3; 15-17]. [46; 3; 12-15]. [47; 2; 12,14]. 48; 1; 0,1,12. 49; 1; 0,1,12,13. 50; 2; 0,1,12. 51; 2; 0,1,12,13. 52; 2; 2,3,14. 53; 2; 2,3,14,15. 54; 3; 0,1,12. 55; 3; 0,1,12,13. 56; 3; 2,3,14. 57; 3; 2,3,14,15. 58; 3; 4,5,16. 59; 3; 4,5,16,17. [60; 3; 13,15,17]. [61; 3; 13,15]. [62; 2; 13,15]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X. - FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 3; 0-4; 1]. 1; 1; 1; 1; [1; 3; 0-5; 1]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,6,8; 2]. 4; 2; 1; 1; [4; 2; 0,1,2,3,6,7,8; 2]. 5; 2; 2; 1; [5; 2; 0,1,2,3,6,7,8,9; 2]. 6; 2; 3; 1; 6; 2; 0,1,2,3,12; 1. 7; 2; 0,1; 1; 7; 2; 0-3,12,14; 1. 8; 2; 2,3; 1; 8; 2; 0-3,12-14; 1. [9; 2; 0-2; 1]; 9; 2; 0-3,12-15; 1. [10; 2; 0-3; 1]; [10; 3; 0,1,2,3,12; 1]. 11; 3; 0; 1; [11; 3; 0-3,12,14; 1]. 12; 3; 1; 1; [12; 3; 0-3,12-14; 1]. 13; 3; 2; 1; [13; 3; 0-3,12-15; 1]. 14; 3; 3; 1; [14; 1; 0,1,6,7,12; 2]. 15; 3; 4; 1; [15; 1; 0,1,6,7,12,18; 2]. 16; 3; 5; 1; [16; 1; 0,1,6,7,12,13,18; 2]. 17; 3; 0,1; 1; [17; 1; 0,1,6,7,12,13,18,19; 2]. 18; 3; 2,3; 1; [18; 2; 0,1,6,7,12; 2]. 19; 3; 4,5; 1; [19; 2; 0,1,6,7,12,18; 2]. [20; 3; 0-2; 1]; [20; 2; 0,1,6,7,12,13,18; 2]. [21; 3; 3-5; 1]; [21; 2; 0,1,6,7,12,13,18,19; 2]. [22; 3; 0-3; 1]; [22; 2; 2,3,8,9,14; 2]. [23; 2; 0,2; 1]; [23; 2; 2,3,8,9,14,20; 2]. 24; 3; 0; 2; [24; 2; 2,3,8,9,14,15,20; 2]. 25; 3; 1; 2; [25; 2; 2,3,8,9,14,15,20,21; 2]. 26; 3; 2; 2; [26; 3; 0,1,6,7,12; 2]. 27; 3; 3; 2; [27; 3; 0,1,6,7,12,18; 2]. 28; 3; 4; 2; [28; 3; 0,1,6,7,12,13,18; 2]. 29; 3; 5; 2; [29; 3; 0,1,6,7,12,13,18,19; 2]. 30; 3; 6; 2; [30; 3; 2,3,8,9,14; 2]. 31; 3; 7; 2; [31; 3; 2,3,8,9,14,20; 2]. 32; 3; 8; 2; [32; 3; 2,3,8,9,14,15,20; 2]. 33; 3; 9; 2; [33; 3; 2,3,8,9,14,15,20,21; 2]. 34; 3; 10; 2; [34; 3; 4,5,10,11,16; 2]. 35; 3; 11; 2; [35; 3; 4,5,10,11,16,22; 2]. 36; 3; 0,1; 2; [36; 3; 4,5,10,11,16,17,22; 2]. 37; 3; 2,3; 2; [37; 3; 4,5,10,11,16,17,22,23; 2]. 38; 3; 4,5; 2; [38; 2; 0,1,2,3,14; 1]. 39; 3; 6,7; 2; [39; 2; 0,1,12,2,3,14; 1]. 40; 3; 8,9; 2; [40; 2; 0,1,12,2,3,14,15; 1]. 41; 3; 10,11; 2; [41; 2; 0,1,12,13,2,3,14,15; 1]. [42; 3; 0,1,6; 2]; [42; 3; 0,1,2,3,14; 1]. [43; 3; 2,3,8; 2]; [43; 3; 0,1,12,2,3,14; 1]. [44; 3; 4,5,10; 2]; [44; 3; 0,1,12,2,3,14,15; 1]. [45; 3; 0,1,6,7; 2]; [45; 3; 0,1,12,13,2,3,14,15; 1]. [46; 3; 2,3,8,9; 2]; [46; 1; 0,1,6,7,18; 2]. [47; 3; 4,5,10,11; 2]; [47; 1; 0,1,12,6,7,18; 2]. 48; 1; 0; 2; [48; 1; 0,1,12,6,7,18,19; 2]. 49; 1; 1; 2; [49; 1; 0,1,12,13,6,7,18,19; 2]. 50; 1; 6; 2; [50; 2; 0,1,6,7,18; 2]. 51; 1; 7; 2; [51; 2; 0,1,12,6,7,18; 2]. 52; 1; 0,1; 2; [52; 2; 0,1,12,6,7,18,19; 2]. 53; 1; 6,7; 2; [53; 2; 0,1,12,13,6,7,18,19; 2]. 54; 2; 0,1; 2; [54; 2; 2,3,8,9,20; 2]. 55; 2; 2,3; 2; [55; 2; 2,3,14,8,9,20; 2]. 56; 2; 6,7; 2; [56; 2; 2,3,14,8,9,20,21; 2]. 57; 2; 8,9; 2; [57; 2; 2,3,14,15,8,9,20,21; 2]. 58; 1; 12; 1; [58; 3; 0,1,6,7,18; 2]. 59; 1; 13; 1; [59; 3; 0,1,12,6,7,18; 2]. 60; 1; 12,13; 1; [60; 3; 0,1,12,6,7,18,19; 2]. 61; 2; 12; 1; [61; 3; 0,1,12,13,6,7,18,19; 2]. 62; 2; 13; 1; [62; 3; 2,3,8,9,20; 2]. 63; 2; 14; 1; [63; 3; 2,3,14,8,9,20; 2]. 64; 2; 15; 1; [64; 3; 2,3,14,8,9,20,21; 2]. 65; 2; 12,13; 1; [65; 3; 2,3,14,15,8,9,20,21; 2]. 66; 2; 14,15; 1; [66; 3; 4,5,10,11,22; 2]. [67; 2; 12-14; 1]; [67; 3; 4,5,16,10,11,22; 2]. [68; 2; 12-15; 1]; [68; 3; 4,5,16,10,11,22,23; 2]. 69; 3; 12; 1; [69; 3; 4,5,16,17,10,11,22,23; 2]. 70; 3; 13; 1. 71; 3; 14; 1. 72; 3; 15; 1. 73; 3; 16; 1. 74; 3; 17; 1. 75; 3; 12,13; 1. 76; 3; 14,15; 1. 77; 3; 16,17; 1. [78; 3; 12-14; 1]. [79; 3; 15-17; 1]. [80; 3; 12-15; 1]. [81; 2; 12,14; 1]. 82; 3; 12; 2. 83; 3; 13; 2. 84; 3; 14; 2. 85; 3; 15; 2. 86; 3; 16; 2. 87; 3; 17; 2. 88; 3; 18; 2. 89; 3; 19; 2. 90; 3; 20; 2. 91; 3; 21; 2. 92; 3; 22; 2. 93; 3; 23; 2. 94; 3; 12,13; 2. 95; 3; 14,15; 2. 96; 3; 16,17; 2. 97; 3; 18,19; 2. 98; 3; 20,21; 2. 99; 3; 22,23; 2. [100; 3; 12,13,18; 2]. [101; 3; 14,15,20; 2]. [102; 3; 16,17,22; 2]. [103; 3; 12,13,18,19; 2]. [104; 3; 14,15,20,21; 2]. [105; 3; 16,17,22,23; 2]. 106; 1; 12; 2. 107; 1; 13; 2. 108; 1; 18; 2. 109; 1; 19; 2. 110; 1; 12,13; 2. 111; 1; 18,19; 2. 112; 2; 12,13; 2. 113; 2; 14,15; 2. 114; 2; 18,19; 2. 115; 2; 20,21; 2. 116; 1; 0,1,12; 1. 117; 1; 0,1,12,13; 1. 118; 2; 0,1,12; 1. 119; 2; 0,1,12,13; 1. 120; 2; 2,3,14; 1. 121; 2; 2,3,14,15; 1. 122; 3; 0,1,12; 1. 123; 3; 0,1,12,13; 1. 124; 3; 2,3,14; 1. 125; 3; 2,3,14,15; 1. 126; 3; 4,5,16; 1. 127; 3; 4,5,16,17; 1. [129; 1; 0,1,12; 2]. [130; 1; 0,1,12,13; 2]. [131; 1; 6,7,18; 2]. [132; 1; 6,7,18,19; 2]. [133; 2; 0,1,12; 2]. [134; 2; 0,1,12,13; 2]. [135; 2; 6,7,18; 2]. [136; 2; 6,7,18,19; 2]. [137; 2; 2,3,14; 2]. [138; 2; 2,3,14,15; 2]. [139; 2; 8,9,20; 2]. [140; 2; 8,9,20,21; 2]. [141; 3; 0,1,12; 2]. [142; 3; 0,1,12,13; 2]. [143; 3; 6,7,18; 2]. [144; 3; 6,7,18,19; 2]. [145; 3; 2,3,14; 2]. [146; 3; 2,3,14,15; 2]. [147; 3; 8,9,20; 2]. [148; 3; 8,9,20,21; 2]. [149; 3; 4,5,16; 2]. [150; 3; 4,5,16,17; 2]. [151; 3; 10,11,22; 2]. [152; 3; 10,11,22,23; 2]. [153; 3; 7,12,13; 2]. [154; 3; 9,14,15; 2]. [155; 3; 11,16,17; 2]. [156; 3; 9,18,19; 2]. [157; 3; 18,19,20; 2]. [158; 3; 21,22,23; 2]. Conclusion No consensus to support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH Agreement For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17. - Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity. Agreement For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH. - Support row 7 for rank2, row1 for rank3, row 1 for rank4. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 7; 2; 9,11. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 1; 2; 8-10. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 1; 2; 8-11. Agreement For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following. - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. - Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. Conclusion For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - For PUSCH, there is no restriction. Agreement For partial/non-coherent PUSCH with rank=5-8 transmission (i.e. non of the CWs is disabled) with one PTRS port, PTRS-DMRS association for PUSCH is the following. - The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW. 1; 2nd scheduled DMRS port with the CW. 2; 3rd scheduled DMRS port with the CW. 3; 4th scheduled DMRS port with the CW. Conclusion For “The CW with the higher MCS” in RAN1#112 agreement of PTRS-DMRS association field for full-coherent PUSCH with rank=5~8 PUSCH with one port PTRS, following is clarified. - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

In this contribution, we provide some discussion on the remaining issues for DMRS enhancement.

# Discussion

## Number of co-scheduled ports for DL DMRS

For eType1/eType2 DMRS, FD-OCC-4 is introduced to support more orthogonal ports. However, sometimes, there may not be many co-scheduled UEs. For the case without number of co-scheduled UEs, the UE may use a smaller FD-OCC length to decode the eType1/eType2 DMRS. When more than 2 ports in a CDM group are allocated for a UE, the UE has to apply FD-OCC-4 based despreading. When only 1 port in a CDM group is allocated for a UE, the UE may apply FD-OCC-1, FD-OCC-2 or FD-OCC-4 based despreading, which depends on the status for the co-scheduled UE(s). Thus, it is beneficial for the gNB to indicate the number of co-scheduled DMRS ports for a CDM group.

Proposal 1: Support to dynamically indicate the status of number of co-scheduled DL DMRS ports for each indicated CDM group to facilitate the FD-OCC length selection in UE side.

## Orphan RE handling

In RAN1 #110b, the following agreement on orphan RE handling was achieved.

Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - 1) The number of consecutively scheduled PRBs for PDSCH is even. - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded.

One open issue is whether to consider some scheduling restriction on orphan REs in a PRG. If the number of RBs in a PRG is odd, it is possible to create some orphan REs. However, this could depend on the UE’s receiver scheme – whether the UE supports cross-PRG channel estimation. If the UE does not support cross-PRG channel estimation, further restriction on orphan REs in a PRG should be considered. On the other hand, if the UE supports cross-PRG channel estimation, it is unnecessary to introduce such restriction. With regard to different UE receiver schemes, a UE capability should be introduced to report whether the UE support number of add RBs in a PRG for eType1/eType2 DMRS.

Proposal 2: Support to introduce a UE capability to indicate whether the UE supports odd number of consecutive RBs in a PRG.

## UL PT-RS power boosting

The PT-RS power boosting has been supported since Rel-15. The PT-RS power boosting could be based on the constraint of transmission power per port and the constraint of total transmission power across all the ports. The UE may transmit the PT-RS in the time domain density of every other symbol or every 4 symbols. But the UE cannot change the transmission power too frequently. Further, with regard to ICI, an upper limit for PT-RS power boosting should be defined. RAN1 can send an LS to RAN4 asking for their advice on the upper limit for the EPRE ratio between PT-RS and PUSCH. Therefore, the PT-RS power boosting could consider the following principles:

- Principle 1: The transmission power for each RF chain is consistent, as shown in Figure 1

- Principle 2: The total transmission power across all the RF chains is consistent, as shown in Figure 2

- Principle 3: The EPRE ratio between PT-RS and PUSCH should not exceed a threshold with regard to the ICI

Figure 1: Consistent Tx power per RF chain (Principle 1)

Figure 2: Consistent Tx power across all the RF chains (Principle 2)

In RAN1 #112b, the following on PT-RS power boosting has been achieved.

Agreement For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles. - Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - FFS: The factor () for partial coherent TPMIs

For the coherent and non-coherent TPMIs, the agreement above follows the above 3 principles well, and the power boosting upper bound is 9dB. For partial-coherent TPMIs, the same principle should be maintained.

Proposal 3: Support to define the PT-RS power boosting for partial-coherent TPMIs based on the following principles:

- Principle 1: The transmission power for each RF chain is consistent

- Principle 2: The total transmission power across all the RF chains is consistent

- Principle 3: The EPRE ratio between PT-RS and PUSCH should not exceed 9dB

## UL PT-RS time-domain density selection

Currently, the time-domain density for UL PT-RS is determined based on the scheduled MCS and configured MCS thresholds. For 8Tx transmission, the scheduled MCS for the two codewords can be different. It has been agreed that the PT-RS is associated with a DMRS port. Therefore, the scheduled MCS for the CW associated with the DMRS port should be applied for the time-domain density selection for the corresponding UL PT-RS port.

Proposal 4: For 2 codeword operation, the time-domain density for an UL PT-RS port is determined based on the MCS for the associated CW.

## UL PT-RS port and DMRS port association

In RAN1 112b, the following alternatives were agreed for UL PT-RS port indication when 2 PT-RS ports are enabled.

Agreement For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following. - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. - Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1.

Currently the STxMP only supports up to 4 layers. The use case for 2 port PT-RS for 8Tx is unclear. It is unnecessary to optimize the PT-RS to DMRS port indication at current stage. Therefore, the Alt4 should be supported. If the optimization is needed, Alt3 can be considered, since it does not require additional overhead but it can support all types of indication for the associated DMRS ports. But current table in Alt3 seems misleading. The table should be reformulated as follows:

Table 1 (Modified for Alt3): PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port from the first or second half of DMRS ports which shares PTRS port 0 | 0 | 1st DMRS port from the first or second half of DMRS ports which shares PTRS port 1 |
| 1 | 2nd DMRS port from the first or second half of DMRS ports which shares PTRS port 0 | 1 | 2nd DMRS port from the first or second half of DMRS ports which shares PTRS port 1 |

- Note: Whether the DMRS port is selected from the first/second half DMRS ports for each codeword is indicated by reserved filed of DMRS ports indication

Proposal 5: For two PTRS ports, support Alt4 for PTRS-DMRS association indication.

# Conclusion

In this contribution, we provided discussion on remaining issues for DMRS enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Support to dynamically indicate the status of number of co-scheduled DL DMRS ports for each indicated CDM group to facilitate the FD-OCC length selection in UE side.

Proposal 2: Support to introduce a UE capability to indicate whether the UE supports odd number of consecutive RBs in a PRG.

Proposal 3: Support to define the PT-RS power boosting for partial-coherent TPMIs based on the following principles:

- Principle 1: The transmission power for each RF chain is consistent

- Principle 2: The total transmission power across all the RF chains is consistent

- Principle 3: The EPRE ratio between PT-RS and PUSCH should not exceed 9dB

Proposal 4: For 2 codeword operation, the time-domain density for an UL PT-RS port is determined based on the MCS for the associated CW.

Proposal 5: For two PTRS ports, support Alt4 for PTRS-DMRS association indication.