3GPP TSG RAN WG1 #113			R1-2304967

Incheon, Korea, 22nd – 26th May 2023

Source:	Panasonic

Title: 	Discussion on Sidelink Positioning Reference Signal

Agenda Item:		9.5.1.1

Document for:	Discussion, Decision

# Introduction

This contribution discusses SL PRS for support of sidelink positioning. With using existing sequence DL-PRS as a starting point, this paper covers PRS operation scenarios, PRS sequence and other PRS parameters.

# Discussion

## PRS operation scenarios

Since the SL-PRS has two schemes, namely Scheme 1 (network centric) and Scheme 2 (UE autonomous) to be applied with or without network coverage, depending on how PRS related parameters are obtained and periodic/aperiodic transmission of PRS, we think the operation scenarios can be categorized as the following table. Whether all cases need to be supported or not may need some discussion.

Table 1. Different PRS Operation scenarios

| Case | PRS related parameters | Transmission of PRS |
| --- | --- | --- |
| Case 1 | Fully (Pre-)configured | Periodic transmission |
| Case 2 | Fully (Pre-)configured | Aperiodic PRS transmission |
| Case 3 | Scheme 1) obtained from gNB | Periodic transmission |
| Case 4 | Scheme 1) obtained from gNB | Aperiodic PRS transmission |
| Case 5 | Scheme 2) obtained from other UE | Periodic transmission |
| Case 6 | Scheme 2) obtained from other UE | Aperiodic PRS transmission |

Proposal 1: The operation scenarios can be categorized depending on "how PRS related parameters are obtained" and "periodic/aperiodic PRS transmission".

The WID requires to specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. We think a singular design regarding to the 100MHz is enough. For spectrum/BWP less than 100MHz, a truncated PRS resource set configuration would be sufficient.

Proposal 2: The design for SL-PRS should be unified and a truncated configuration would be used for spectrum less than 100MHz.

## PRS Sequence

The following were agreed in last RAN1#112bis meeting.

Agreement For SL PRS sequence generation, no additional parameters other than the following input parameters are used: slot number, symbol number, and the parameter nID,seqSL-PRSnID,seqSL-PRS. Agreement For SL PRS sequence generation, one of the following options is down-selected to define the parameter nID,seqSL-PRS nID,seqSL-PRS: - Option 1:nID,seqSL-PRS nID,seqSL-PRS is a higher layer parameter. - FFS: How the higher layer parameter is obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. - Option 2:nID,seqSL-PRS nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS. - Option 3: based on a combination of higher layer parameter from a configured ID list and 12 LSB bits of CRC of PSCCH associated with the SL PRS. - FFS: How the higher layer parameter/ID list is determined/obtained, e.g., (pre-)configuration or via LPP/SLPP, etc.

For the SL-PRS sequence ID nID,seqSL-PRS, 3 options have been further listed for down-selection. In our understanding, The SL-PRS sequence ID should be different for SL UEs with Scheme 1 and Scheme 2 with separated range of values allocated. Among the listed options, we prefer either option 1 or 3 with higher layer configuration. Then by proper higher layer parameter configurations, different sequence ID values can be allocated for Scheme 1 and Scheme 2 at the physical layer.

Proposal 3: The SL-PRS sequence ID needs to be separately allocated for Scheme 1 and Scheme 2

Proposal 4: nID,seqSL-PRS is either a higher layer configured parameter or based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission.

## Other PRS Parameters

The following were discussed in last RAN1#112bis meeting.

### [Medium] FL4 Proposal 2.4-4 - On comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values within a slot in a dedicated resource pool (to be down-selected from): - Alt A1: Supported without any restrictions. - Alt A2: Supported for same M values and different N values. For different M values, only when the different (M, N) pairs are multiplexed via TDM within a slot. - FFS: Potential restrictions on possible N values for a given M. - Alt A3: Supported if the start timings of SL PRSs with different (M,N) are aligned, and the portion of RE overlap between SL PRSs is below a threshold. - Alt B: Not supported.

In our understanding, although having multiple (M, N) values might be complex but only same M value would be too restrictive. We would like to support the Alt A2

Proposal 5: On comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values within a slot in a dedicated resource pool, support for same M values and different N values.

Starting from referencing the DL-PRS, we think most of the parameters could re-use the legacy SL/DL designs. The following are some of the parameters may need to be further discussed for SL-PRS because of difference Scheme 1 and Scheme 2.

The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB (for scheme 1) and generated by UE itself (scheme 2) with pre-defined rules. For scheme 2, what kind of rules to be applied need to be further discussed with how the value is assigned to different UEs.

Proposal 6: The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB for scheme 1 and or UE self-generation for scheme 2 according to pre-defined rules.

For SL PRS periodicity, the periodicity could be just same as or multiple times of S-SSB for simplicity.  For SL PRS repetition, the concept can be similar to the LTE blind re-transmission, but with more repetitions. We may either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI (for various time gaps between different transmissions). Periodic cases (PRS Operation scenarios case 1, 3, 5) only need the initial SCI or SCI carry single same value.  Aperiodic cases (PRS Operation scenarios case 2, 4, 6) may need specific designs.

Proposal 7: SL-PRS periodicity could be just same as or multiple times of S-SSB for simplicity.

Proposal 8: SL-PRS repetition (if >2) may be either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI.

In last RAN#112bis meeting, the following agreement was made.

Agreement An AGC symbol preceding a SL PRS resource is not considered as part of the SL PRS resource itself.

We understand that AGC symbol is not a part of PRS. Sidelink needs to receive multiple UEs transmitter. Therefore, alignment for each is not required on AFC. For AGC, the sum of multiple UE's signal situation can be different for each of the transmission. Then the past setting of AGC cannot be reused even if the same Tx of PRS is intended to be received. Then QCL used for DL cannot be used. Similar to other SL channels, AGC symbol is necessary for SL-PRS additional to the SL-PRS resources.

Proposal 9: AGC symbol is necessary to be reserved for SL-PRS.

# Conclusion

In this contribution, followings proposals are made:

Proposal 1: The operation scenarios can be categorized depending on "how PRS related parameters are obtained" and "periodic/aperiodic PRS transmission".

Proposal 2: The design for SL-PRS should be unified and a truncated configuration would be used for spectrum less than 100MHz.

Proposal 3: The SL-PRS sequence ID needs to be separately allocated for Scheme 1 and Scheme 2

Proposal 4: nID,seqSL-PRS is either a higher layer configured parameter or based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

Proposal 5: On comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values within a slot in a dedicated resource pool, support for same M values and different N values.

Proposal 6: The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB for scheme 1 and or UE self-generation for scheme 2 according to pre-defined rules.

Proposal 7: SL-PRS periodicity could be just same as or multiple times of S-SSB for simplicity.

Proposal 8: SL-PRS repetition (if >2) may be either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI.

Proposal 9: AGC symbol is necessary to be reserved for SL-PRS.

# Reference

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, RAN#99

[2] TR38.859: Study on expanded and improved NR positioning

[3] TR38.857: Study on NR positioning enhancements

[4] TR38.845: Study on NR positioning support

[5] Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0, Online meeting, 9th – 20th May 2022

[6] Final Report of 3GPP TSG RAN WG1 #110 v1.0.0, Toulouse, France, 22nd – 26th August 2022

[7] Final Report of 3GPP TSG RAN WG1 #110bis v1.0.0, Online meeting, 10th – 19th October 2022

[8] Final Report of 3GPP TSG RAN WG1 #111 v1.0.0, Toulouse, France, 14th – 18th November 2022

[9] Final Report of 3GPP TSG RAN WG1 #112 v1.0.0, Athens, Greece, 27th Feb – 3rd Mar 2023

[10] Draft Report of 3GPP TSG RAN WG1 #112bis v0.4.0, Online meeting, 17th – 26th  April 2023